| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 386-E |
|  |  |  | 2 July 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| R8 | PLENARY MEETING |
| --- | --- |
| EIGHTH SERIES OF TExts submitted by the editorial committee to the plenary meeting |  |
| The following texts are submitted to the Plenary Meeting for second reading: |  |

article  5

Frequency allocations

Annex: 49 pages

article  5

Frequency allocations

MOD           COM4/272/2            (B5/290/20)            (R8/386/1)

5.56	The stations of services to which the bands 14-19.95 kHz and 20.05-70 kHz and in Region 1 also the bands 72-84 kHz and 86-90 kHz are allocated may transmit standard frequency and time signals. Such stations shall be afforded protection from harmful interference. In Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakhstan, Mongolia, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan and Turkmenistan, the frequencies 25 kHz and 50 kHz will be used for this purpose under the same conditions.     (WRC-03)

MOD           COM5/152/3            (B2/190/3)            (R8/386/2)

148-223 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 148-149.9 FIXED MOBILE except aeronautical mobile (R) MOBILE-SATELLITE (Earth-to-space) 5.209 | 148-149.9 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.209 |  |
| 5.218 5.219 MOD 5.221 | 5.218 5.219 MOD 5.221 |  |

MOD           COM4/272/3            (B5/290/21)            (R8/386/3)

5.68	Alternative allocation:  in Angola, Burundi, the Congo, Malawi, Dem. Rep. of the Congo, Rwanda and South Africa, the band 160-200 kHz is allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM4/272/4            (B5/290/22)            (R8/386/4)

5.70	Alternative allocation:  in Angola, Botswana, Burundi, Cameroon, the Central African Rep., the Congo, Ethiopia, Lesotho, Madagascar, Malawi, Mozambique, Namibia, Nigeria, Oman, Dem. Rep. of the Congo, Rwanda, South Africa, Swaziland, Tanzania, Chad, Zambia and Zimbabwe, the band 200-283.5 kHz is allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/5            (B5/290/23)            (R8/386/5)

5.87	Additional allocation:  in Angola, Botswana, Lesotho, Malawi, Mozambique, Namibia, South Africa, Swaziland and Zimbabwe, the band 526.5-535 kHz is also allocated to the mobile service on a secondary basis.     (WRC-03)

MOD           COM4/272/6            (B5/290/24)            (R8/386/6)

5.96	In Germany, Armenia, Austria, Azerbaijan, Belarus, Denmark, Estonia, Finland, Georgia, Hungary, Iceland, Ireland, Israel, Kazakhstan, Latvia, Liechtenstein, Lithuania, Malta, Moldova, Norway, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., the United Kingdom, the Russian Federation, Sweden, Switzerland, Tajikistan, Turkmenistan and Ukraine, administrations may allocate up to 200 kHz to their amateur service in the bands 1 715-1 800 kHz and 1 850-2 000 kHz. However, when allocating the bands within this range to their amateur service, administrations shall, after prior consultation with administrations of neighbouring countries, take such steps as may be necessary to prevent harmful interference from their amateur service to the fixed and mobile services of other countries. The mean power of any amateur station shall not exceed 10 W.     (WRC-03)

MOD           COM4/272/7            (B5/290/25)            (R8/386/7)

5.98	Alternative allocation:  in Angola, Armenia, Azerbaijan, Belarus, Belgium, Bulgaria, Cameroon, the Congo, Denmark, Egypt, Eritrea, Spain, Ethiopia, Georgia, Greece, Italy, Kazakhstan, Lebanon, Lithuania, Moldova, Syrian Arab Republic, Kyrgyzstan, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Turkey and Ukraine, the band 1 810-1 830 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/8            (B5/290/26)            (R8/386/8)

5.99	Additional allocation:  in Saudi Arabia, Austria, Bosnia and Herzegovina, Iraq, Libyan Arab Jamahiriya, Uzbekistan, Slovakia, Romania, Slovenia, Chad, Togo and Serbia and Montenegro, the band 1 810-1 830 kHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/9            (B5/290/27)            (R8/386/9)

5.107	Additional allocation:  in Saudi Arabia, Eritrea, Ethiopia, Iraq, Lesotho, Libyan Arab Jamahiriya, Somalia and Swaziland, the band 2 160-2 170 kHz is also allocated to the fixed and mobile, except aeronautical mobile (R), services on a primary basis. The mean power of stations in these services shall not exceed 50 W.     (WRC-03)

MOD           COM4/272/10            (B5/290/28)            (R8/386/10)

5.112	Alternative allocation:  in Bosnia and Herzegovina, Denmark, Malta, Sri Lanka and Serbia and Montenegro, the band 2 194-2 300 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/11            (B5/290/29)            (R8/386/11)

5.114	Alternative allocation:  in Bosnia and Herzegovina, Denmark, Iraq, Malta, and Serbia and Montenegro, the band 2 502-2 625 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/12            (B5/290/30)            (R8/386/12)

5.117	Alternative allocation:  in Bosnia and Herzegovina, Côte d’Ivoire, Denmark, Egypt, Liberia, Malta, Sri Lanka, Togo and Serbia and Montenegro, the band 3 155-3 200 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/13            (B5/290/31)            (R8/386/13)

5.118	Additional allocation:  in the United States, Mexico, Peru and Uruguay, the band 3 230-3 400 kHz is also allocated to the radiolocation service on a secondary basis.     (WRC-03)

MOD           COM4/272/14            (B5/290/32)            (R8/386/14)

5.140	Additional allocation:  in Angola, Iraq, Kenya, Rwanda, Somalia and Togo, the band 7 000-7 050 kHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM4/272/15            (B5/290/33)            (R8/386/15)

5.152	Additional allocation:  in Armenia, Azerbaijan, China, Côte d’Ivoire, Georgia, Iran (Islamic Republic of), Kazakhstan, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 14 250-14 350 kHz is also allocated to the fixed service on a primary basis. Stations of the fixed service shall not use a radiated power exceeding 24 dBW.     (WRC-03)

MOD           COM4/272/16            (B5/290/34)            (R8/386/16)

5.154	Additional allocation:  in Armenia, Azerbaijan, Georgia, Kazakhstan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 18 068-18 168 kHz is also allocated to the fixed service on a primary basis for use within their boundaries, with a peak envelope power not exceeding 1 kW.     (WRC-03)

MOD           COM4/272/17            (B5/290/35)            (R8/386/17)

5.155	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakhstan, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 21 850-21 870 kHz is also allocated to the aeronautical mobile (R) services on a primary basis.     (WRC-03)

MOD           COM4/272/18            (B5/290/36)            (R8/386/18)

5.163	Additional allocation:  in Armenia, Azerbaijan, Belarus, Georgia, Hungary, Kazakhstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Rep., Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 47-48.5 MHz and 56.5-58 MHz are also allocated to the fixed and land mobile services on a secondary basis.     (WRC-03)

MOD           COM4/339/2            (B11/347/134)            (R8/386/19)

5.164	Additional allocation:  in Albania, Germany, Austria, Belgium, Bosnia and Herzegovina, Botswana, Bulgaria, Côte d’Ivoire, Denmark, Spain, Estonia, Finland, France, Gabon, Greece, Ireland, Israel, Italy, Jordan, Lebanon, Libyan Arab Jamahiriya, Liechtenstein, Luxembourg, Madagascar, Mali, Malta, Morocco, Mauritania, Monaco, Nigeria, Norway, the Netherlands, Poland, Syrian Arab Republic, the United Kingdom, Slovenia, Sweden, Switzerland, Swaziland, Chad, Togo, Tunisia, Turkey and Serbia and Montenegro the band 47-68 MHz, in Romania the band 47-58 MHz, in South Africa the band 47-50 MHz, in the Czech Rep. the band 66-68 MHz, are also allocated to the land mobile service on a primary basis. However, stations of the land mobile service in the countries mentioned in connection with each band referred to in this footnote shall not cause harmful interference to, or claim protection from, existing or planned broadcasting stations of countries other than those mentioned in connection with the band.     (WRC-03)

MOD           COM4/272/19            (B5/290/37)            (R8/386/20)

5.174	Alternative allocation:  in Bulgaria, Hungary and Romania, the band 68-73 MHz is allocated to the broadcasting service on a primary basis and used in accordance with the decisions in the Final Acts of the Special Regional Conference (Geneva, 1960).     (WRC-03)

MOD           COM4/272/20            (B5/290/38)            (R8/386/21)

5.177	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakhstan, Latvia, Moldova, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 73-74 MHz is also allocated to the broadcasting service on a primary basis, subject to agreement obtained under No. 9.21.     (WRC-03)

MOD           COM4/272/21            (B5/290/39)            (R8/386/22)

5.179	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, China, Georgia, Kazakhstan, Lithuania, Moldova, Mongolia, Kyrgyzstan, Slovakia, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 74.6-74.8 MHz and 75.2-75.4 MHz are also allocated to the aeronautical radionavigation service, on a primary basis, for ground-based transmitters only.     (WRC-03)

MOD           COM4/272/22            (B5/290/40)            (R8/386/23)

5.181	Additional allocation:  in Egypt, Israel and Syrian Arab Republic, the band 74.8-75.2 MHz is also allocated to the mobile service on a secondary basis, subject to agreement obtained under No. 9.21. In order to ensure that harmful interference is not caused to stations of the aeronautical radionavigation service, stations of the mobile service shall not be introduced in the band until it is no longer required for the aeronautical radionavigation service by any administration which may be identified in the application of the procedure invoked under No. 9.21.     (WRC-03)

MOD           COM5/263/1            (B5/290/5)            (R8/386/24)

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 108-117.975 AERONAUTICAL RADIONAVIGATION 5.197 ADD 5.BA03 |  |  |

ADD           COM5/263/2            (B5/290/6)            (R8/386/25)

5.BA03	The band 108-117.975 MHz may also be used by the aeronautical mobile (R) service on a primary basis, limited to systems that transmit navigational information in support of air navigation and surveillance functions in accordance with recognized international aviation standards. Such use shall be in accordance with Resolution [COM5/2] (WRC-03) and shall not cause harmful interference to nor claim protection from stations operating in the aeronautical radionavigation service which operate in accordance with international aeronautical standards.     (WRC-03)

MOD           COM4/272/23            (B5/290/41)            (R8/386/26)

5.203B	Additional allocation:  in Saudi Arabia, United Arab Emirates, Oman and Syrian Arab Republic, the band 136-137 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis until 1 January 2005.     (WRC-03)

MOD           COM4/272/24            (B5/290/42)            (R8/386/27)

5.204	Different category of service:  in Afghanistan, Saudi Arabia, Bahrain, Bangladesh, Bosnia and Herzegovina, Brunei Darussalam, China, Cuba, the United Arab Emirates, India, Indonesia, Iran (Islamic Republic of), Iraq, Malaysia, Oman, Pakistan, Philippines, Qatar, Singapore, Thailand, Yemen and Serbia and Montenegro, the band 137-138 MHz is allocated to the fixed and mobile, except aeronautical mobile (R), services on a primary basis (see No. 5.33).     (WRC-03)

MOD           COM4/272/25            (B5/290/43)            (R8/386/28)

5.210	Additional allocation:  in France, Italy, the Czech Rep. and the United Kingdom, the bands 138-143.6 MHz and 143.65-144 MHz are also allocated to the space research service (space-to-Earth) on a secondary basis.     (WRC-03)

MOD           COM4/272/26            (B5/290/44)            (R8/386/29)

5.212	Alternative allocation:  in Angola, Botswana, Burundi, Cameroon, the Central African Rep., the Congo, Gabon, Gambia, Ghana, Guinea, Iraq, Jordan, Lesotho, Liberia, Libyan Arab Jamahiriya, Malawi, Mozambique, Namibia, Oman, Uganda, Dem. Rep. of the Congo, Rwanda, Sierra Leone, South Africa, Swaziland, Chad, Togo, Zambia and Zimbabwe, the band 138-144 Hz is allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM5/152/4            (B2/190/4)            (R8/386/30)

5.221	Stations of the mobile-satellite service in the band 148-149.9 MHz shall not cause harmful interference to, or claim protection from, stations of the fixed or mobile services operating in accordance with the Table of Frequency Allocations in the following countries: Albania, Algeria, Germany, Saudi Arabia, Australia, Austria, Bahrain, Bangladesh, Barbados, Belarus, Belgium, Benin, Bosnia and Herzegovina, Botswana, Brunei Darussalam, Bulgaria, Cameroon, China, Cyprus, Congo, Korea (Rep. of), Côte d’Ivoire, Croatia, Cuba, Denmark, Egypt, the United Arab Emirates, Eritrea, Spain, Estonia, Ethiopia, Finland, France, Gabon, Ghana, Greece, Guinea, Guinea Bissau, Hungary, India, Iran (Islamic Republic of), Ireland, Iceland, Israel, Italy, Jamaica, Japan, Jordan, Kazakhstan, Kenya, Kuwait, Latvia, The Former Yugoslav Republic of Macedonia, Lebanon, Libyan Arab Jamahiriya, Liechtenstein, Lithuania, Luxembourg, Malaysia, Mali, Malta, Mauritania, Moldova, Mongolia, Mozambique, Namibia, Norway, New Zealand, Oman, Uganda, Uzbekistan, Pakistan, Panama, Papua New Guinea, Paraguay, the Netherlands, the Philippines, Poland, Portugal, Qatar, Syrian Arab Republic, Kyrgyzstan, Slovakia, Romania, the United Kingdom, the Russian Federation, Senegal, Serbia and Montenegro, Sierra Leone, Singapore, Slovenia, Sri Lanka, South Africa, Sweden, Switzerland, Swaziland, Tanzania, Chad, Thailand, Togo, Tonga, Trinidad and Tobago, Tunisia, Turkey, Ukraine, Viet Nam, Yemen, Zambia, and Zimbabwe.     (WRC-03)

MOD           COM4/272/28            (B5/290/46)            (R8/386/31)

5.237	Additional allocation:  in the Congo, Eritrea, Ethiopia, Gambia, Guinea, Libyan Arab Jamahiriya, Malawi, Mali, Sierra Leone, Somali, Chad and Zimbabwe, the band 174-223 MHz is also allocated to the fixed and mobile services on a secondary basis.     (WRC-03)

MOD           COM5/263/4            (B5/290/8)            (R8/386/32)

220-335.4 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 235-267 FIXED MOBILE 5.111 5.199 5.252 MOD 5.254 5.256 ADD 5.BE03 |  |  |

ADD           COM5/263/5            (B5/290/9)            (R8/386/33)

5.BE03	Additional allocation:  in China, Russian Federation, Kazakhstan and Ukraine, the band 258-261 MHz is also allocated to the space research service (Earth-to-space) and space operation service (Earth-to-space) on a primary basis. Stations in the space research service (Earth-to-space) and space operation service (Earth-to-space) shall not cause harmful interference to, nor claim protection from, nor constrain the use and development of the mobile service systems and mobile-satellite service systems operating in the band. Stations in space research service (Earth-to-space) and space operation service (Earth-to-space) shall not constrain the future development of fixed service systems of other countries.     (WRC-03)

MOD						           (R8/386/34)

5.254	The bands 235-322 MHz and 335.4-399.9 MHz may be used by the mobile-satellite service, subject to agreement obtained under No. 9.21, on condition that stations in this service do not cause harmful interference to those of other services operating or planned to be operated in accordance with the Table of Frequency Allocations except for the additional allocation made in footnote No. 5.BE03.     (WRC-03)

MOD           COM4/272/29            (B5/290/47)            (R8/386/35)

5.262	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Botswana, Bulgaria, Colombia, Costa Rica, Cuba, Egypt, the United Arab Emirates, Ecuador, Georgia, Hungary, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kazakhstan, Kuwait, Liberia, Malaysia, Moldova, Uzbekistan, Pakistan, the Philippines, Qatar, Syrian Arab Republic, Kyrgyzstan, Romania, the Russian Federation, Singapore, Somalia, Tajikistan, Turkmenistan, Ukraine and Serbia and Montenegro, the band 400.05-401 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM5/263/10            (B5/290/14)            (R8/386/36)

410-470 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 430-432 AMATEUR RADIOLOCATION | 430-432 RADIOLOCATION Amateur |  |
| 5.271 5.272 5.273 5.274 5.275 5.276 5.277 | 5.271 5.276 5.277 5.278 5.279 |  |

MOD           COM5/263/11            (B5/290/15)            (R8/386/37)

| 432-438 AMATEUR RADIOLOCATION Earth exploration-satellite (active) ADD 5.5E03 | 432-438 RADIOLOCATION Amateur Earth exploration-satellite (active) ADD 5.5E03 |
| --- | --- |
| 5.138 5.271 5.272 5.276 5.277 5.280 5.281 5.282 | 5.271 5.276 5.277 5.278 5.279 5.281 5.282 |

ADD           COM5/263/13            (B5/290/17)            (R8/386/38)

5.5E03	The use of this band by sensors in the Earth exploration-satellite service (EESS) (active) shall be in accordance with Recommendation ITU-R SA.1260-1. Additionally, the EESS (active) in the band 432-438 MHz shall not cause harmful interference to the aeronautical radionavigation service in China.

The provisions of this footnote in no way diminish the obligation of the EESS (active) to operate as a secondary service in accordance with Nos. 5.29 and 5.30.     (WRC-03)

MOD           COM5/263/12            (B5/290/16)            (R8/386/39)

| 438-440 AMATEUR RADIOLOCATION | 438-440 RADIOLOCATION Amateur |
| --- | --- |
| 5.271 5.273 5.274 5.275 5.276 5.277 5.283 | 5.271 5.276 5.277 5.278 5.279 |

MOD           COM4/272/30            (B5/290/48)            (R8/386/40)

5.271	Additional allocation:  in Azerbaijan, Belarus, China, India, Latvia, Lithuania, Kyrgyzstan and Turkmenistan, the band 420-460 MHz is also allocated to the aeronautical radionavigation service (radio altimeters) on a secondary basis.     (WRC-03)

MOD           COM4/272/31            (B5/290/49)            (R8/386/41)

5.273	Different category of service:  in Libyan Arab Jamahiriya, the allocation of the bands 430-432 MHz and 438-440 MHz to the radiolocation service is on a secondary basis (see No. 5.32).     (WRC-03)

MOD           COM4/272/32            (B5/290/50)            (R8/386/42)

5.277	Additional allocation:  in Angola, Armenia, Azerbaijan, Belarus, Cameroon, Congo, Djibouti, Georgia, Hungary, Israel, Kazakhstan, Mali, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, the Russian Federation, Rwanda, Tajikistan, Chad, Turkmenistan and Ukraine, the band 430-440 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM4/314/8            (B8/325/20)            (R8/386/43)

5.288	In the territorial waters of the United States and the Philippines, the preferred frequencies for use by on-board communication stations shall be 457.525 MHz, 457.550 MHz, 457.575 MHz and 457.600 MHz paired, respectively, with 467.750 MHz, 467.775 MHz, 467.800 MHz and 467.825 MHz. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174-1.     (WRC-03)

MOD           COM4/272/33            (B5/290/51)            (R8/386/44)

5.294	Additional allocation:  in Burundi, Cameroon, the Congo, Côte d’Ivoire, Ethiopia, Israel, Kenya, Lebanon, Libyan Arab Jamahiriya, Malawi, Sudan, Syrian Arab Republic, Chad and Yemen, the band 470-582 MHz is also allocated to the fixed service on a secondary basis.     (WRC-03)

MOD           COM4/272/34            (B5/290/52)            (R8/386/45)

5.296	Additional allocation:  in Germany, Austria, Belgium, Côte d’Ivoire, Denmark, Spain, Finland, France, Ireland, Israel, Italy, Libyan Arab Jamahiriya, Lithuania, Malta, Morocco, Monaco, Norway, the Netherlands, Portugal, Syrian Arab Republic, the United Kingdom, Sweden, Switzerland, Swaziland and Tunisia, the band 470-790 MHz is also allocated on a secondary basis to the land mobile service, intended for applications ancillary to broadcasting. Stations of the land mobile service in the countries listed in this footnote shall not cause harmful interference to existing or planned stations operating in accordance with the Table in countries other than those listed in this footnote.     (WRC-03)

MOD           COM4/314/9            (B8/325/21)            (R8/386/46)

5.311	Within the frequency band 620-790 MHz, assignments may be made to television stations using frequency modulation in the broadcasting-satellite service subject to agreement between the administrations concerned and those having services, operating in accordance with the Table, which may be affected (see Resolutions 33 (Rev.WRC-03) and 507 (Rev.WRC-03)). Such stations shall not produce a power flux-density in excess of the value –129 dB(W/m2) for angles of arrival less than 20° (see Recommendation 705) within the territories of other countries without the consent of the administrations of those countries. Resolution [COM4/5] (WRC-03) applies.     (WRC-03)

MOD           COM4/272/35            (B5/290/53)            (R8/386/47)

5.312	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Hungary, Kazakhstan, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 645-862 MHz is also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/36            (B5/290/54)            (R8/386/48)

5.316	Additional allocation:  in Germany, Saudi Arabia, Bosnia and Herzegovina, Burkina Faso, Cameroon, Côte d’Ivoire, Croatia, Denmark, Egypt, Finland, Greece, Israel, Jordan, Kenya, The Former Yugoslav Republic of Macedonia, Libyan Arab Jamahiriya, Liechtenstein, Mali, Monaco, Norway, the Netherlands, Portugal, the United Kingdom, Syrian Arab Republic, Sweden, Switzerland and Serbia and Montenegro, the band 790-830 MHz, and in these same countries and in Spain, France, Gabon and Malta, the band 830-862 MHz, are also allocated to the mobile, except aeronautical mobile, service on a primary basis. However, stations of the mobile service in the countries mentioned in connection with each band referred to in this footnote shall not cause harmful interference to, or claim protection from, stations of services operating in accordance with the Table in countries other than those mentioned in connection with the band.     (WRC-03)

MOD           COM4/272/37            (B5/290/55)            (R8/386/49)

5.323	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Hungary, Kazakhstan, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Rep., Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 862-960 MHz is also allocated to the aeronautical radionavigation service on a primary basis. Such use is subject to agreement obtained under No. 9.21 with administrations concerned and limited to ground-based radiobeacons in operation on 27 October 1997 until the end of their lifetime.     (WRC-03)

MOD           COM5/277/2            (B6/306/5)            (R8/386/50)

890-1 300 MHz

1 240-1 300 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) MOD 5.329 5.329A SPACE RESEARCH (active) Amateur ADD 5.282 5.330 MOD 5.331 5.332 5.335 ADD 5.335A

MOD           COM5/277/4            (B6/306/7)            (R8/386/51)

5.329	Use of the radionavigation-satellite service in the band 1 215-1 300 MHz shall be subject to the condition that no harmful interference is caused to, and no protection is claimed from, the radionavigation service authorized under No. 5.331. Furthermore, use of the radionavigation-satellite service in the band 1 215-1 300 MHz shall be subject to the condition that no harmful interference is caused to the radiolocation service and that No. 5.43 shall not apply. Resolution [COM5/5] (WRC-03) shall apply.     (WRC-03)

MOD           COM4/272/38            (B5/290/56)            (R8/386/52)

5.330	Additional allocation:  in Angola, Saudi Arabia, Bahrain, Bangladesh, Cameroon, China, the United Arab Emirates, Eritrea, Ethiopia, Guyana, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Lebanon, Libyan Arab Jamahiriya, Mozambique, Nepal, Pakistan, the Philippines, Qatar, Syrian Arab Republic, Somalia, Sudan, Chad, Togo and Yemen, the band 1 215-1 300 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM5/277/5            (B6/306/8)            (R8/386/53)

5.331	Additional allocation:  in Algeria, Germany, Saudi Arabia, Australia, Austria, Bahrain, Belarus, Belgium, Benin, Bosnia and Herzegovina, Brazil, Burkina Faso, Burundi, Cameroon, China, Korea (Rep. of), Croatia, Denmark, the United Arab Emirates, Estonia, Russian Federation, Finland, France, Greece, Guinea, Equatorial Guinea, Hungary, India, Indonesia, Iran (Islamic Republic of), Iraq, Ireland, Israel, Jordan, Kenya, Kuwait, Lesotho, The Former Yugoslav Republic of Macedonia, Liechtenstein, Lithuania, Luxembourg, Madagascar, Mali, Mauritania, Nigeria, Norway, Oman, the Netherlands, Poland, Portugal, Qatar, Syrian Arab Republic, Slovakia, United Kingdom, Serbia and Montenegro, Slovenia, Somalia, Sudan, Sri Lanka, South Africa, Sweden, Switzerland, Thailand, Turkey and Viet Nam, the band 1 215-1 300 MHz is also allocated to the radionavigation service on a primary basis. In Canada and the United States, the band 1 240-1 300 MHz is also allocated to the radionavigation service, and use of the radionavigation service shall be limited to the aeronautical radionavigation service.     (WRC-03)

MOD           COM5/277/6            (B6/306/9)            (R8/386/54)

5.334	Additional allocation:  in Canada and the United States, the band 1 350-1 370 MHz is also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD           COM5/277/3            (B6/306/6)            (R8/386/55)

1 300-1 350 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |

MOD           COM5/312/1            (B10/345/1)            (R8/386/56)

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 350-1 400 FIXED MOBILE RADIOLOCATION | 1 350-1 400 RADIOLOCATION |  |
| 5.149 5.338 5.339 ADD 5.BB05 | 5.149 5.334 5.339 ADD 5.BB05 |  |

ADD           COM5/312/3            (B10/345/3)            (R8/386/57)

5.BB05	Additional allocation:  the band 1 390-1 392 MHz is also allocated to the fixed-satellite service (Earth-to-space) on a secondary basis and the band 1 430-1 432 MHz is also allocated to the fixed-satellite service (space-to-Earth) on a secondary basis. These allocations are limited to use for feeder links for non-geostationary-satellite networks in the mobile-satellite service with service links below 1 GHz, and Resolution [COM5/14] (WRC-03) applies.     (WRC-03)

MOD           COM5/312/2            (B10/345/2)            (R8/386/58)

| 1 429-1 452 FIXED MOBILE except aeronautical mobile | 1 429-1 452 FIXED MOBILE 5.343 |
| --- | --- |
| 5.341 5.342 ADD 5.BB05 | 5.341 ADD 5.BB05 |

MOD           COM5/305/12            (B7/324/23)            (R8/386/59)

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 492-1 518 FIXED MOBILE except aeronautical mobile | 1 492-1 518 FIXED MOBILE 5.343 | 1 492-1 518 FIXED MOBILE |
| 5.341 5.342 | 5.341 5.344 | 5.341 |
| 1 518-1 525 FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) MOD 5.348 MOD 5.348A ADD 5.BB06 ADD 5.BB12 | 1 518-1 525 FIXED MOBILE 5.343 MOBILE-SATELLITE (space-to-Earth) MOD 5.348 MOD 5.348A ADD 5.BB06 ADD 5.BB12 | 1 518-1 525 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) MOD 5.348 MOD 5.348A ADD 5.BB06 ADD 5.BB12 |
| 5.341 5.342 | 5.341 5.344 | 5.341 |

ADD           COM5/305/14            (B7/324/25)            (R8/386/60)

5.BB12	In the band 1 518-1 525 MHz, stations in the mobile-satellite service shall not claim protection from aeronautical mobile telemetry stations in the mobile service in the territory of the United States (see Nos. 5.343 and 5.344) and in the countries listed in No. 5.342. No. 5.43A does not apply.     (WRC-03)

MOD           COM4/272/40            (B5/290/58)            (R8/386/61)

5.338	In Azerbaijan, Mongolia, Kyrgyzstan, Slovakia, the Czech Rep., Romania and Turkmenistan, existing installations of the radionavigation service may continue to operate in the band 1 350-1 400 MHz.     (WRC-03)

MOD           COM4/272/41            (B5/290/59)            (R8/386/62)

5.340	All emissions are prohibited in the following bands:

1 400-1 427 MHz,

2 690-2 700 MHz,	except those provided for by No. 5.422,

10.68-10.7 GHz,		except those provided for by No. 5.483,

15.35-15.4 GHz,		except those provided for by No. 5.511,

23.6-24 GHz,

31.3-31.5 GHz,

31.5-31.8 GHz,		in Region 2,

48.94-49.04 GHz,		from airborne stations,

50.2-50.4 GHz2,

52.6-54.25 GHz,

86-92 GHz,

100-102 GHz,

109.5-111.8 GHz,

114.25-116 GHz,

148.5-151.5 GHz,

164-167 GHz,

182-185 GHz,

190-191.8 GHz,

200-209 GHz,

226-231.5 GHz,

250-252 GHz.           (WRC-03)

MOD           COM4/272/42            (B5/290/60)            (R8/386/63)

5.347	Different category of service:  in Bangladesh, Bosnia and Herzegovina, Botswana, Bulgaria, Burkina Faso, Cuba, Denmark, Egypt, Greece, Ireland, Italy, Mozambique, Portugal, Sri Lanka, Swaziland, Yemen, Serbia and Montenegro and Zimbabwe, the allocation of the band 1 452-1 492 MHz to the broadcasting-satellite service and the broadcasting service is on a secondary basis until 1 April 2007.     (WRC-03)

MOD           COM5/305/13            (B7/324/24)            (R8/386/64)

5.348	The use of the band 1 518-1 525 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. In the band 1 518-1 525 MHz stations in the mobile-satellite service shall not claim protection from the stations in the fixed service. No. 5.43A does not apply.     (WRC-03)

MOD           COM5/305/15            (B7/324/26)            (R8/386/65)

5.348A	In the band 1 518-1 525 MHz, the coordination threshold in terms of the power flux-density levels at the surface of the Earth in application of No. 9.11A for space stations in the mobile-satellite (space-to-Earth) service, with respect to the land mobile service use for specialized mobile radios or used in conjunction with public switched telecommunication networks (PSTN) operating within the territory of Japan, shall be –150 dB(W/m2) in any 4 kHz band for all angles of arrival, instead of those given in Table 5-2 of Appendix 5. In the band 1 518-1 525 MHz stations in the mobile-satellite service shall not claim protection from stations in the mobile service in the territory of Japan. No. 5.43A does not apply.     (WRC-03)

MOD           COM4/272/43            (B5/290/61)            (R8/386/66)

5.355	Additional allocation:  in Bahrain, Bangladesh, Congo, Egypt, Eritrea, Iraq, Israel, Kuwait, Lebanon, Malta, Qatar, Syrian Arab Republic, Somalia, Sudan, Chad, Togo and Yemen, the bands 1 540-1 559 MHz, 1 610-1 645.5 MHz and 1 646.5-1 660 MHz are also allocated to the fixed service on a secondary basis.     (WRC-03)

MOD           COM4/272/44            (B5/290/62)            (R8/386/67)

5.359	Additional allocation:  in Germany, Saudi Arabia, Armenia, Austria, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Cameroon, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Jordan, Kazakhstan, Kuwait, Lebanon, Libyan Arab Jamahiriya, Lithuania, Mauritania, Moldova, Mongolia, Uganda, Uzbekistan, Pakistan, Poland, Syrian Arab Republic, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Swaziland, Tajikistan, Tanzania, Tunisia, Turkmenistan and Ukraine, the bands 1 550-1 559 MHz, 1 610-1 645.5 MHz and 1 646.5-1 660 MHz are also allocated to the fixed service on a primary basis. Administrations are urged to make all practicable efforts to avoid the implementation of new fixed-service stations in these bands.     (WRC-03)

MOD           COM4/272/45            (B5/290/63)            (R8/386/68)

5.362B	Additional allocation:  The band 1 559-1 610 MHz is also allocated to the fixed service on a primary basis until 1 January 2005 in Germany, Armenia, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Kazakhstan, Lithuania, Moldova, Mongolia, Nigeria, Uganda, Uzbekistan, Pakistan, Poland, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Senegal, Swaziland, Tajikistan, Tanzania, Turkmenistan and Ukraine, and until 1 January 2010 in Saudi Arabia, Cameroon, Jordan, Kuwait, Lebanon, Libyan Arab Jamahiriya, Mali, Mauritania, Syrian Arab Republic and Tunisia. After these dates, the fixed service may continue to operate on a secondary basis until 1 January 2015, at which time this allocation shall no longer be valid. Administrations are urged to take all practicable steps to protect the radionavigation-satellite service and the aeronautical radionavigation service and not authorize new frequency assignments to fixed-service systems in this band.     (WRC-03)

MOD           COM4/272/46            (B5/290/64)            (R8/386/69)

5.369	Different category of service:  in Angola, Australia, Burundi, China, Eritrea, Ethiopia, India, Iran (Islamic Republic of), Israel, Lebanon, Liberia, Libyan Arab Jamahiriya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep. of the Congo, Syrian Arab Republic, Sudan, Swaziland, Togo and Zambia, the allocation of the band 1 610-1 626.5 MHz to the radiodetermination-satellite service (Earth-to-space) is on a primary basis (see No. 5.33), subject to agreement obtained under No. 9.21 from countries not listed in this provision.     (WRC-03)

MOD           COM5/305/1            (B7/324/12)            (R8/386/70)

1 660-1 710 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 660.5-1 668 RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile 5.149 5.341 5.379 5.379A |  |  |

MOD           COM5/305/2            (B7/324/13)            (R8/386/71)

| 1 668-1 668.4 MOBILE-SATELLITE (Earth-to-space) ADD 5.BB06 RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile 5.149 5.341 ADD 5.BB07 5.379 5.379A ADD 5.BB08 ADD 5.BB09 |  |  |
| --- | --- | --- |
| 1 668.4-1 670 METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (Earth-to-space) ADD 5.BB06 RADIO ASTRONOMY 5.149 5.341 ADD 5.BB07 ADD 5.BB08 ADD 5.BB09 ADD 5.BB10 |  |  |
| 1 670-1 675 METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE 5.380 MOBILE-SATELLITE (Earth-to-space) ADD 5.BB06 5.341 ADD 5.BB07 ADD 5.BB08 ADD 5.BB10 ADD 5.BB11 |  |  |
| 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |
| 5.341 | 5.341 | 5.341 |

| 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile | 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) | 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) |
| --- | --- | --- |
| 5.289 5.341 5.382 | 5.289 5.341 5.381 | 5.289 5.341 5.381 |
| 1 700-1 710 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 700-1 710 FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 700-1 710 FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |
| 5.289 5.341 | 5.289 5.341 | 5.289 5.341 5.384 |

ADD           COM5/305/28            (B7/324/39)            (R8/386/72)

5.BB06	For the use of the bands 1 518-1 525 MHz and 1 668-1 675 MHz by the mobile-satellite service, see Resolution 225 (Rev.WRC-03).     (WRC-03)

ADD           COM5/305/3            (B7/324/14)            (R8/386/73)

5.BB07	The use of the band 1 668-1 675 MHz by the mobile-satellite service is subject to coordination under No. 9.11A.     (WRC-03)

ADD           COM5/305/4            (B7/324/15)            (R8/386/74)

5.BB08	For sharing of the band 1 668-1 675 MHz between the mobile-satellite service and the fixed, mobile and space research (passive) services, Resolution [COM5/12] (WRC-03) shall apply.     (WRC-03)

ADD           COM5/305/5            (B7/324/16)            (R8/386/75)

5.BB09	In order to protect the radio astronomy service in the band 1 668-1 670 MHz, the aggregate power flux-density (pfd) values produced by mobile earth stations in a network of the mobile-satellite service operating in this band shall not exceed –181 dB(W/m2) in 10 MHz and 194 dB(W/m2) in any 20 kHz at any radio astronomy station recorded in the Master International Frequency Register, for more than 2% of integration periods of 2 000 s.     (WRC-03)

ADD           COM5/305/6            (B7/324/17)            (R8/386/76)

5.BB10	In the band 1 668.4-1 675 MHz, stations in the mobile-satellite service shall not cause harmful interference to stations in the meteorological aids service in China, Iran (Islamic Republic of), Japan and Uzbekistan. In the band 1 668.4-1 675 MHz, administrations are urged not to implement new systems in the meteorological aids service and are encouraged to migrate existing meteorological aids service operations to other bands as soon as practicable.     (WRC-03)

ADD           COM5/305/7            (B7/324/18)            (R8/386/77)

5.BB11	In the band 1 670-1 675 MHz, stations in the mobile-satellite service shall not cause harmful interference to, nor constrain the development of, existing earth stations in the meteorological-satellite service notified in accordance with Resolution [COM5/13] (WRC-03).     (WRC-03)

SUP           COM5/305/8            (B7/324/19)            (R8/386/78)

5.377

MOD           COM4/272/47            (B5/290/65)            (R8/386/79)

5.381	Additional allocation:  in Afghanistan, Costa Rica, Cuba, India, Iran (Islamic Republic of) and Pakistan, the band 1 690-1 700 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/48            (B5/290/66)            (R8/386/80)

5.382	Different category of service:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Bulgaria, the Congo, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Guinea, Hungary, Iraq, Israel, Jordan, Kazakhstan, Kuwait, the Former Yugoslav Republic of Macedonia, Lebanon, Mauritania, Moldova, Mongolia, Oman, Uzbekistan, Poland, Qatar, Syrian Arab Republic, Kyrgyzstan, Romania, Russian Federation, Somalia, Tajikistan, Tanzania, Turkmenistan, Ukraine, Yemen and Serbia and Montenegro, the allocation of the band 1 690-1 700 MHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. 5.33), and in the Dem. People’s Rep. of Korea, the allocation of the band 1 690-1 700 MHz to the fixed service is on a primary basis (see No. 5.33) and to the mobile, except aeronautical mobile, service on a secondary basis.     (WRC-03)

MOD           COM4/272/49            (B5/290/67)            (R8/386/81)

5.386	Additional allocation:  the band 1 750-1 850 MHz is also allocated to the space operation (Earth-to-space) and space research (Earth-to-space) services in Region 2, in Australia, Guam, India, Indonesia and Japan on a primary basis, subject to agreement obtained under No. 9.21, having particular regard to troposcatter systems.     (WRC-03)

MOD           COM4/272/50            (B5/290/68)            (R8/386/82)

5.387	Additional allocation:  in Azerbaijan, Belarus, Georgia, Kazakhstan, Mongolia, Kyrgyzstan, Slovakia, Romania, Tajikistan and Turkmenistan, the band 1 770-1 790 MHz is also allocated to the meteorological-satellite service on a primary basis, subject to agreement obtained under No. 9.21.     (WRC-03)

MOD           COM4/339/3            (B11/347/135)            (R8/386/83)

5.388A	In Regions 1 and 3, the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz and, in Region 2, the bands 1 885-1 980 MHz and 2 110-2 160 MHz may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications-2000 (IMT-2000), in accordance with Resolution 221 (Rev.WRC-03). Their use by IMT-2000 applications using high altitude platform stations as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations.     (WRC-03)

ADD           COM4/339/4            (B11/347/136)            (R8/386/84)

5.388B	In Algeria, Saudi Arabia, Bahrain, Cameroon, China, Comoros, Côte d’Ivoire, Cuba, Djibouti, Egypt, United Arab Emirates, Eritrea, Gabon, Ghana, India, Iran (Islamic Republic of), Israel, Jordan, Kenya, Kuwait, Libyan Arab Jamahiriya, Mali, Morocco, Mauritania, Nigeria, Oman, Uganda, Qatar, Syrian Arab Republic, Sudan, Tanzania, Chad, Tunisia, Yemen, Zambia and Zimbabwe, for the purpose of protecting fixed and mobile services, including IMT-2000 mobile stations, in their territories from co-channel interference, a HAPS operating as an IMT-2000 base station in neighbouring countries, in the bands referred to in No. 5.388A, shall not exceed a co-channel power flux-density (pfd) of 127 dB (W/(m2 · MHz)) at the Earth’s surface outside a country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of HAPS.      (WRC-03)

SUP           COM4/272/51            (B5/290/69)            (R8/386/85)

5.389D

MOD           COM4/339/5            (B11/347/137)            (R8/386/86)

5.395	In France and Turkey, the use of the band 2 310-2 360 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service.      (WRC-03)

MOD           COM4/272/52            (B5/290/70)            (R8/386/87)

5.400	Different category of service:  in Angola, Australia, Bangladesh, Burundi, China, Eritrea, Ethiopia, India, Iran (Islamic Republic of), Lebanon, Liberia, Libyan Arab Jamahiriya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep. of the Congo, Syrian Arab Republic, Sudan, Swaziland, Togo and Zambia, the allocation of the band 2 483.5-2 500 MHz to the radiodetermination-satellite service (space-to-Earth) is on a primary basis (see No. 5.33), subject to agreement obtained under No. 9.21 from countries not listed in this provision.     (WRC-03)

MOD           COM4/272/53            (B5/290/71)            (R8/386/88)

5.418	Additional allocation:  in Korea (Rep. of), India, Japan, Pakistan and Thailand, the band 2 535-2 655 MHz is also allocated to the broadcasting-satellite service (sound) and complementary terrestrial broadcasting service on a primary basis. Such use is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (Rev.WRC-03). The provisions of No. 5.416 and Table 21-4 of Article 21, do not apply to this additional allocation. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) is subject to Resolution 539 (Rev.WRC-03).     (WRC-03)

SUP           COM4/272/54            (B5/290/72)            (R8/386/89)

5.421

MOD           COM4/272/55            (B5/290/73)            (R8/386/90)

5.422	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Brunei Darussalam, Congo, Côte d’Ivoire, Cuba, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Gabon, Georgia, Guinea, Guinea-Bissau, Iran (Islamic Republic of), Iraq, Israel, Jordan, Lebanon, Mauritania, Moldova, Mongolia, Nigeria, Oman, Uzbekistan, Pakistan, the Philippines, Qatar, Syrian Arab Republic, Kyrgyzstan, the Dem. Rep. of the Congo, Romania, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Ukraine, Yemen and Serbia and Montenegro, the band 2 690-2 700 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985.     (WRC-03)

MOD           COM5/225/1            (B4/240/1)            (R8/386/91)

2 700-4 800 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 900-3 100 RADIONAVIGATION 5.426 RADIOLOCATION ADD 5.BA01 5.425 5.427 |  |  |

ADD           COM5/225/2            (B4/240/2)            (R8/386/92)

5.BA01	In the band 2 900-3 100 MHz, stations in the radiolocation service shall not cause harmful interference to, nor claim protection from, radar systems in the radionavigation service.     (WRC-03)

MOD           COM4/272/56            (B5/290/74)            (R8/386/93)

5.428	Additional allocation:  in Azerbaijan, Cuba, Mongolia, Kyrgyzstan, Romania and Turkmenistan, the band 3 100-3 300 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/57            (B5/290/75)            (R8/386/94)

5.429	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, China, the Congo, Korea (Rep. of), the United Arab Emirates, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kenya, Kuwait, Lebanon, Libyan Arab Jamahiriya, Malaysia, Oman, Pakistan, Qatar, Syrian Arab Republic, Dem. People’s Rep. of Korea and Yemen, the band 3 300-3 400 MHz is also allocated to the fixed and mobile services on a primary basis. The countries bordering the Mediterranean shall not claim protection for their fixed and mobile services from the radiolocation service.     (WRC-03)

MOD           COM4/272/58            (B5/290/76)            (R8/386/95)

5.430	Additional allocation:  in Azerbaijan, Cuba, Mongolia, Kyrgyzstan, Romania and Turkmenistan, the band 3 300-3 400 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/59            (B5/290/77)            (R8/386/96)

5.431	Additional allocation:  in Germany, Israel and the United Kingdom, the band 3 400-3 475 MHz is also allocated to the amateur service on a secondary basis.     (WRC-03)

MOD           COM5/207/1            (B3/239/5)            (R8/386/97)

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (Earth-to-space) 5.367 |  |  |

MOD           COM5/207/2            (B3/239/6)            (R8/386/98)

5 010-5 030 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) MOD 5.443B 5.367

MOD           COM5/207/3            (B3/239/7)            (R8/386/99)

5 030-5 150 AERONAUTICAL RADIONAVIGATION 5.367 5.444 5.444A

MOD           COM5/308/1            (B8/325/1)            (R8/386/100)

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD03 5.446 MOD 5.447 5.447B 5.447C |  |  |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD04 5.448 MOD 5.448A |  |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD04 5.448 MOD 5.448A |  |  |

5 350-5 460 EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) ADD 5.BD08 AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION ADD 5.BD05 MOD 5.448B

5 460-5 470 RADIONAVIGATION 5.449 EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION ADD 5.BD05 MOD 5.448B

5 470-5 570 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION ADD 5.BD06 5.450 5.451 5.452 MOD 5.448B

5 570-5 650 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 RADIOLOCATION ADD 5.BD06 5.450 5.451 5.452

5 650-5 725 RADIOLOCATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 Amateur Space research (deep space) 5.282 5.451 MOD 5.453 5.454 5.455

ADD           COM5/308/2            (B8/325/2)            (R8/386/101)

5.BD02	The use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the stations in the mobile service shall be in accordance with Resolution [COM5/16] (WRC-03).     (WRC-03)

ADD           COM5/308/3            (B8/325/3)            (R8/386/102)

5.BD03	In the band 5 150-5 250 MHz, stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. Number 5.43A does not apply to the mobile service with respect to FSS earth stations.     (WRC-03)

ADD           COM5/308/4            (B8/325/4)            (R8/386/103)

5.BD04	In the band 5 250-5 350 MHz, stations in the mobile service shall not claim protection from the radiolocation service, the Earth exploration-satellite service (active) and the space research service (active). These services shall not impose on the mobile service more stringent protection criteria, based on system characteristics and interference criteria, than those stated in Recommendations ITU-R M.1638 and ITU-R SA.1632.     (WRC-03)

ADD           COM5/308/7            (B8/325/7)            (R8/386/104)

5.BD05	In the frequency band 5 350-5 470 MHz, stations in the radiolocation service shall not cause harmful interference to, nor claim protection from, radar systems in the aeronautical radionavigation service operating in accordance with No. 5.449.     (WRC-03)

ADD           COM5/308/8            (B8/325/8)            (R8/386/105)

5.BD06	In the frequency band 5 470-5 650 MHz, stations in the radiolocation service, except ground-based radars used for meteorological purposes in the band 5 600-5 650 MHz, shall not cause harmful interference to, nor claim protection from, radar systems in the maritime radionavigation service.     (WRC-03)

ADD           COM5/308/9            (B8/325/9)            (R8/386/106)

5.BD07	In the band 5 470-5 725 MHz, stations in the mobile service shall not claim protection from radiodetermination services. Radiodetermination services shall not impose on the mobile service more stringent protection criteria, based on system characteristics and interference criteria, than those stated in Recommendation ITU-R M.1638.     (WRC-03)

MOD           COM5/225/3            (B4/240/3)            (R8/386/107)

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D 5.448 5.448A ADD 5.BD01 |  |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.448 5.448A ADD 5.BD01 |  |  |

ADD           COM5/225/4            (B4/240/4)            (R8/386/108)

5.BD01	Additional allocation:  The band 5 250-5 350 MHz is also allocated to the fixed service on a primary basis in the following countries in Region 3: Australia, [Korea (Rep. of)], India, Indonesia, Iran (Islamic Republic of), Japan, Malaysia, Papua New Guinea, Philippines, Sri Lanka, Thailand and Viet Nam. The use of this band by the fixed service is intended for the implementation of fixed wireless access (FWA) systems and shall comply with Recommendation ITU-R F.1613. In addition, the fixed service shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, but the provisions of No. 5.43A do not apply to the fixed service with respect to the Earth exploration-satellite (active) and space research (active) services. After implementation of FWA systems in the fixed service with protection for the existing radiodetermination systems, no more stringent constraints should be imposed on the FWA systems by future radiodetermination implementations.     (WRC-03)

ADD							(R8/386/109)

5.BD08	The space research service (active) operating in the band 5 350-5 460 MHz shall not cause harmful interference to nor claim protection from other services to which this band is allocated.     (WRC-03)

SUP           COM5/207/4            (B3/239/8)            (R8/386/110)

5.443A

MOD           COM5/207/5            (B3/239/9)            (R8/386/111)

5.443B	In order not to cause harmful interference to the microwave landing system operating above 5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz by all the space stations within any radionavigation-satellite service system (space-to-Earth) operating in the band 5 010-5 030 MHz shall not exceed –124.5 dB(W/m2) in a 150 kHz band. In order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, radionavigation-satellite service systems operating in the band 5 010-5 030 MHz shall comply with the limits in the band 4 990-5 000 MHz defined in Resolution [COM5/1] (WRC-03).     (WRC-03)

MOD           COM5/263/7            (B5/290/11)            (R8/386/112)

5.444	The band 5 030-5 150 MHz is to be used for the operation of the international standard system (microwave landing system) for precision approach and landing. The requirements of this system shall take precedence over other uses of this band. For the use of this band, No. 5.444A and Resolution 114 (Rev.WRC-03) apply.     (WRC-03)

MOD           COM5/263/8            (B5/290/12)            (R8/386/113)

5.444A	Additional allocation:  the band 5 091-5 150 MHz is also allocated to the fixed-satellite service (Earth-to-space) on a primary basis. This allocation is limited to feeder links of non-geostationary mobile-satellite systems in the mobile-satellite service and is subject to coordination under No. 9.11A.

In the band 5 091-5 150 MHz, the following conditions also apply:

–	prior to 1 January 2018, the use of the band 5 091-5 150 MHz by feeder links of non-geostationary-satellite systems in the mobile-satellite service shall be made in accordance with Resolution 114 (Rev.WRC-03);

–	prior to 1 January 2018, the requirements of existing and planned international standard systems for the aeronautical radionavigation service which cannot be met in the 5 000-5 091 MHz band, shall take precedence over other uses of this band;

–	after 1 January 2012, no new assignments shall be made to earth stations providing feeder links of non-geostationary mobile-satellite systems;

–	after 1 January 2018, the fixed-satellite service will become secondary to the aeronautical radionavigation service.     (WRC-03)

MOD           COM5/308/10            (B8/325/10)            (R8/386/114)

5.447	Additional allocation:  in Israel, Lebanon, Pakistan, the Syrian Arab Republic and Tunisia, the band 5 150-5 250 MHz is also allocated to the mobile service, on a primary basis, subject to agreement obtained under No. 9.21. In this case, the provisions of Resolution [COM5/16] (WRC-03) do not apply.     (WRC-03)

MOD           COM4/272/61            (B5/290/79)            (R8/386/115)

5.448	Additional allocation:  in Azerbaijan, Libyan Arab Jamahiriya, Mongolia, Kyrgyzstan, Slovakia, Romania and Turkmenistan, the band 5 250-5 350 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM5/308/5            (B8/325/5)            (R8/386/116)

5.448A	The Earth exploration-satellite (active) and space research (active) services in the frequency band 5 250-5 350 MHz shall not claim protection from the radiolocation service. Number 5.43A does not apply.     (WRC-03)

MOD           COM5/308/6            (B8/325/6)            (R8/386/117)

5.448B	The Earth exploration-satellite service (active) operating in the band 5 350-5 570 MHz and space research service (active) operating in the band 5 460-5 570 MHz shall not cause harmful interference to the aeronautical radionavigation service in the band 5 350-5 460 MHz, the radionavigation service in the band 5 460-5 470 MHz and the maritime radionavigation service in the band 5 470-5 570 MHz.     (WRC-03)

MOD           COM4/272/62            (B5/290/80)            (R8/386/118)

5.450	Additional allocation:  in Austria, Azerbaijan, Iran (Islamic Republic of), Mongolia, Kyrgyzstan, Romania, Turkmenistan and Ukraine, the band 5 470-5 650 MHz is also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-03)

MOD           COM5/308/11            (B8/325/11)            (R8/386/119)

5.453	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Cameroon, China, Congo, Côte d’Ivoire, Korea (Rep. of), Egypt, the United Arab Emirates, Gabon, Guinea, Equatorial Guinea, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kenya, Kuwait, Lebanon, the Libyan Arab Jamahiriya, Madagascar, Malaysia, Nigeria, Oman, Pakistan, the Philippines, Qatar, the Syrian Arab Republic, the Dem. People’s Rep. of Korea, Singapore, Sri Lanka, Swaziland, Tanzania, Chad, Thailand, Viet Nam and Yemen, the band 5 650-5 850 MHz is also allocated to the fixed and mobile services on a primary basis. In this case, the provisions of Resolution [COM5/16] (WRC-03) do not apply.     (WRC-03)

MOD           COM4/272/64            (B5/290/82)            (R8/386/120)

5.454	Different category of service:  in Azerbaijan, Georgia, Mongolia, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan and Turkmenistan, the allocation of the band 5 670-5 725 MHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

MOD           COM4/272/65            (B5/290/83)            (R8/386/121)

5.455	Additional allocation:  in Armenia, Azerbaijan, Belarus, Cuba, Georgia, Hungary, Kazakhstan, Latvia, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 5 670-5 850 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM4/272/66            (B5/290/84)            (R8/386/122)

5.456	Additional allocation:  in Cameroon, the band 5 755-5 850 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM5/209/1            (B3/239/15)            (R8/386/123)

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 7 075-7 145 FIXED MOBILE 5.458 5.459 |  |  |
| 7 145-7 235 FIXED MOBILE SPACE RESEARCH (Earth-to-space) MOD 5.460 5.458 5.459 |  |  |
| 7 235-7 250 FIXED MOBILE 5.458 |  |  |

MOD           COM5/209/2            (B3/239/16)            (R8/386/124)

5.460	The use of the band 7 145-7 190 MHz by the space research service (Earth-to-space) is restricted to deep space; no emissions to deep space shall be effected in the band 7 190-7 235 MHz. Geostationary satellites in the space research service operating in the band 7 190-7 235 MHz shall not claim protection from existing and future stations of the fixed and mobile services and No. 5.43A does not apply.     (WRC-03)

MOD           COM4/272/67            (B5/290/85)            (R8/386/125)

5.466	Different category of service:  in Israel, Singapore and Sri Lanka, the allocation of the band 8 400-8 500 MHz to the space research service is on a secondary basis (see No. 5.32).     (WRC-03)

SUP           COM4/272/68            (B5/290/86)            (R8/386/126)

5.467

MOD           COM4/272/69            (B5/290/87)            (R8/386/127)

5.468	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Burundi, Cameroon, China, the Congo, Costa Rica, Egypt, the United Arab Emirates, Gabon, Guyana, Indonesia, Iran (Islamic Republic of), Iraq, Jamaica, Jordan, Kenya, Kuwait, Lebanon, Libyan Arab Jamahiriya, Malaysia, Mali, Morocco, Mauritania, Nepal, Nigeria, Oman, Pakistan, Qatar, Syrian Arab Republic, Dem. People’s Rep. of Korea, Senegal, Singapore, Somalia, Swaziland, Tanzania, Chad, Togo, Tunisia and Yemen, the band 8 500-8 750 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM4/272/70            (B5/290/88)            (R8/386/128)

5.469	Additional allocation:  in Armenia, Azerbaijan, Belarus, Georgia, Hungary, Lithuania, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, the Czech Rep., Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 8 500-8 750 MHz is also allocated to the land mobile and radionavigation services on a primary basis.     (WRC-03)

MOD           COM4/272/71            (B5/290/89)            (R8/386/129)

5.473	Additional allocation:  in Armenia, Austria, Azerbaijan, Belarus, Bulgaria, Cuba, Georgia, Hungary, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 8 850-9 000 MHz and 9 200-9 300 MHz are also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/72            (B5/290/90)            (R8/386/130)

5.477	Different category of service:  in Algeria, Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Cameroon, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Guyana, India, Indonesia, Iran (Islamic Republic of), Iraq, Jamaica, Japan, Jordan, Kuwait, Lebanon, Liberia, Malaysia, Nigeria, Oman, Pakistan, Qatar, the Dem. People’s Rep. of Korea, Singapore, Somalia, Sudan, Trinidad and Tobago, and Yemen, the allocation of the band 9 800-10 000 MHz to the fixed service is on a primary basis (see No. 5.33).     (WRC-03)

MOD           COM4/272/73            (B5/290/91)            (R8/386/131)

5.478	Additional allocation:  in Azerbaijan, Bulgaria, Mongolia, Kyrgyzstan, Romania, Turkmenistan and Ukraine, the band 9 800-10 000 MHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM4/272/74            (B5/290/92)            (R8/386/132)

5.481	Additional allocation:  in Germany, Angola, Brazil, China, Costa Rica, Côte d’Ivoire, El Salvador, Ecuador, Spain, Guatemala, Hungary, Japan, Kenya, Morocco, Nigeria, Oman, Uzbekistan, Paraguay, Peru, the Dem. People’s Rep. of Korea, Tanzania, Thailand and Uruguay, the band 10.45-10.5 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM4/272/75            (B5/290/93)            (R8/386/133)

5.482	In the band 10.6-10.68 GHz, stations of the fixed and mobile, except aeronautical mobile, services shall be limited to a maximum equivalent isotropically radiated power of 40 dBW and the power delivered to the antenna shall not exceed –3 dBW. These limits may be exceeded subject to agreement obtained under No. 9.21. However, in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Bangladesh, Belarus, China, the United Arab Emirates, Georgia, India, Indonesia, Iran (Islamic Republic of), Iraq, Japan, Kazakhstan, Kuwait, Latvia, Lebanon, Moldova, Nigeria, Pakistan, the Philippines, Qatar, Syrian Arab Republic, Tajikistan and Turkmenistan, the restrictions on the fixed and mobile, except aeronautical mobile, services are not applicable.     (WRC-03)

MOD           COM4/272/76            (B5/290/94)            (R8/386/134)

5.483	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, China, Colombia, Korea (Rep. of), Costa Rica, Egypt, the United Arab Emirates, Georgia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kazakhstan, Kuwait, Lebanon, Mongolia, Uzbekistan, Qatar, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, Tajikistan, Turkmenistan, Yemen and Serbia and Montenegro, the band 10.68-10.7 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985.     (WRC-03)

MOD           COM6/183/2            (B2/190/30)            (R8/386/135)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 11.7-12.5 FIXED BROADCASTING BROADCASTING-SATELLITE MOBILE except aeronautical mobile | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A Mobile except aeronautical mobile 5.485 5.488 | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A |  |
|  | 5.485 5.488 5.489 | 5.487 5.487A 5.492 |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile BROADCASTING |
| 5.487 5.487A 5.492 |  | 5.484A 5.487 SUP |
| 12.5-12.75 | 5.487A 5.488 5.490 5.492 | 12.5-12.75 |
| FIXED-SATELLITE (space-to-Earth) 5.484A (Earth-to-space) 5.494 5.495 5.496 | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |

MOD           COM5/277/10            (B6/306/13)            (R8/386/136)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Standard frequency and time signal-satellite (Earth-to-space) Space research Earth exploration-satellite 5.499 5.500 5.501 MOD 5.502 MOD 5.503 |  |  |

MOD           COM5/164/1            (B2/190/6)            (R8/386/137)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.BB02 Space research 5.505 |  |  |

MOD           COM6/140/1            (B1/155/1)            (R8/386/138)

5.487	In the band 11.7-12.5 GHz in Regions 1 and 3, the fixed, fixed-satellite, mobile, except aeronautical mobile, and broadcasting services, in accordance with their respective allocations, shall not cause harmful interference to, or claim protection from, broadcasting-satellite stations operating in accordance with the Regions 1 and 3 Plan in Appendix 30.     (WRC-03)

MOD           COM4/222/8            (B3/239/35)            (R8/386/139)

5.487A	Additional allocation:  in Region 1, the band 11.7-12.5 GHz, in Region 2, the band 12.2-12.7 GHz and, in Region 3, the band 11.7-12.2 GHz, are also allocated to the fixed-satellite service (space-to-Earth) on a primary basis, limited to non-geostationary systems and subject to application of the provisions of No. 9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite systems in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the broadcasting-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-geostationary-satellite systems in the fixed-satellite service and of the complete coordination or notification information, as appropriate, for the geostationary-satellite networks, and No. 5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated.     (WRC-03)

MOD           COM4/339/6            (B11/347/138)            (R8/386/140)

5.488	The use of the band 11.7-12.2 GHz by geostationary-satellite networks in the fixed-satellite service in Region 2 is subject to application of the provisions of No. 9.14 for coordination with stations of terrestrial services in Regions 1, 2 and 3. For the use of the band 12.2-12.7 GHz by the broadcasting-satellite service in Region 2, see Appendix 30.     (WRC-03)

SUP           COM6/183/1            (B2/190/29)            (R8/386/141)

5.491

MOD           COM4/272/77            (B5/290/95)            (R8/386/142)

5.494	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Cameroon, the Central African Rep., the Congo, Côte d’Ivoire, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Gabon, Ghana, Guinea, Iraq, Israel, Jordan, Kuwait, Lebanon, Libyan Arab Jamahiriya, Madagascar, Mali, Morocco, Mongolia, Nigeria, Qatar, Dem. Rep. of the Congo, Syrian Arab Republic, Somalia, Sudan, Chad, Togo and Yemen, the band 12.5-12.75 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis.     (WRC-03)

MOD           COM4/272/78            (B5/290/96)            (R8/386/143)

5.495	Additional allocation:  in Bosnia and Herzegovina, Croatia, France, Greece, Liechtenstein, Monaco, Uganda, Portugal, Romania, Slovenia, Switzerland, Tanzania, Tunisia and Serbia and Montenegro, the band 12.5-12.75 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis.     (WRC-03)

MOD           COM4/272/79            (B5/290/97)            (R8/386/144)

5.500	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Brunei Darussalam, Cameroon, Egypt, the United Arab Emirates, Gabon, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kuwait, Lebanon, Madagascar, Malaysia, Mali, Malta, Morocco, Mauritania, Nigeria, Pakistan, Qatar, Syrian Arab Republic, Singapore, Sudan, Chad and Tunisia, the band 13.4-14 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM4/272/80            (B5/290/98)            (R8/386/145)

5.501	Additional allocation:  in Azerbaijan, Hungary, Japan, Mongolia, Kyrgyzstan, Romania, the United Kingdom and Turkmenistan, the band 13.4-14 GHz is also allocated to the radionavigation service on a primary basis.     (WRC-03)

MOD           COM5/277/12            (B6/306/15)            (R8/386/146)

5.502	In the band 13.75-14 GHz, an earth station of a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 m and an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition, the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW for elevation angles above 2º and 65 dBW at lower angles. Before an administration brings into use an earth station in a geostationary-satellite network in the fixed-satellite service in this band with an antenna size smaller than 4.5 m, it shall ensure that the power flux-density produced by this earth station does not exceed:

–	–115 dB(W/(m2 · 10 MHz)) for more than 1% of the time produced at 36 m above sea level at the low water mark, as officially recognized by the coastal state;

–	–115 dB(W/(m2 · 10 MHz)) for more than 1% of the time produced 3 m above ground at the border of the territory of an administration deploying or planning to deploy land mobile radars in this band, unless prior agreement has been obtained.

For earth stations within the fixed-satellite service having an antenna diameter greater than or equal to 4.5 m, the e.i.r.p. of any emission should be at least 68 dBW and should not exceed 85 dBW.     (WRC-03)

MOD           COM5/277/13            (B6/306/16)            (R8/386/147)

5.503	In the band 13.75-14 GHz, geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 shall operate on an equal basis with stations in the fixed-satellite service; after that date, new geostationary space stations in the space research service will operate on a secondary basis. Until those geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 cease to operate in this band:

–	in the band 13.770-13.780 GHz, the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in geostationary-satellite orbit shall not exceed:

i)	4.7D + 28 dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) for antenna diameters equal to or greater than 1.2 m and less than 4.5 m;

ii)	49.2 + 20 log(D/4.5) dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) for antenna diameters equal to or greater than 4.5 m and less than 31.9 m;

iii)	66.2 dB(W/40 kHz) for any fixed-satellite service earth station for antenna diameters (m) equal to or greater than 31.9 m;

iv)	56.2 dB(W/4 kHz) for narrow-band (less than 40 kHz of necessary bandwidth) fixed-satellite service earth station emissions from any fixed-satellite service earth station having an antenna diameter of 4.5 m or greater;

–	the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in non-geostationary-satellite orbit shall not exceed 51 dBW in the 6 MHz band from 13.772 to 13.778 GHz.

Automatic power control may be used to increase the e.i.r.p. density in these frequency ranges to compensate for rain attenuation, to the extent that the power flux-density at the fixed-satellite service space station does not exceed the value resulting from use by an earth station of an e.i.r.p. meeting the above limits in clear-sky conditions.     (WRC-03)

SUP           COM5/277/11            (B6/306/14)            (R8/386/148)

5.503A

MOD           COM4/272/81            (B5/290/99)            (R8/386/149)

5.505	Additional allocation:  in Algeria, Angola, Saudi Arabia, Bahrain, Bangladesh, Botswana, Brunei Darussalam, Cameroon, China, Congo, Korea (Rep. of), Egypt, the United Arab Emirates, Gabon, Guatemala, Guinea, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Lesotho, Lebanon, Malaysia, Mali, Morocco, Mauritania, Oman, Pakistan, the Philippines, Qatar, Syrian Arab Republic, the Dem. People’s Rep. of Korea, Singapore, Somalia, Sudan, Swaziland, Tanzania, Chad and Yemen, the band 14-14.3 GHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM5/164/2            (B2/190/7)            (R8/386/150)

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.BB03 Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Radionavigation-satellite |

14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB04 Space research (space-to-Earth)

14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.BB01 ADD 5.BB04 Radio astronomy 5.149

ADD           COM5/164/3            (B2/190/8)            (R8/386/151)

5.BB01	Aircraft earth stations operating in the aeronautical mobile-satellite service in the band 14-14.5 GHz shall comply with the provisions of Annex 1, Part C of Recommendation ITU-R M.1643, with respect to any radio astronomy station performing observations in the 14.47-14.5 GHz band located on the territory of Spain, France, India, Italy, the United Kingdom and South Africa.     (WRC-03)

5.BB02	In the band 14-14.25 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, Côte d’Ivoire, Egypt, Guinea, India, Iran, Kuwait, Lesotho, Nigeria, Oman, Syrian Arab Republic and Tunisia by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.     (WRC-03)

5.BB03	In the band 14.25-14.3 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, China, Côte d’Ivoire, Egypt, France, Guinea, India, Iran, Italy, Kuwait, Lesotho, Nigeria, Oman, Syrian Arab Republic, the United Kingdom and Tunisia by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.     (WRC-03)

5.BB04	In the band 14.3-14.5 GHz, the power flux-density produced on the territory of the countries of Saudi Arabia, Botswana, Cameroon, China, Côte d’Ivoire, Egypt, France, Gabon, Guinea, India, Iran, Italy, Kuwait, Lesotho, Morocco, Nigeria, Oman, Syrian Arab Republic, the United Kingdom, Sri Lanka, Tunisia and Viet Nam by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the limits given in Annex 1, Part B of Recommendation ITU-R M.1643, unless otherwise specifically agreed by the affected administration(s). The provisions of this footnote in no way derogate the obligations of the aeronautical mobile-satellite service to operate as a secondary service in accordance with No. 5.29.     (WRC-03)

MOD           COM4/272/82            (B5/290/100)            (R8/386/152)

5.508	Additional allocation:  in Germany, Bosnia and Herzegovina, France, Italy, The Former Yugoslav Republic of Macedonia, Libyan Arab Jamahiriya, the United Kingdom, Slovenia and Serbia and Montenegro, the band 14.25-14.3 GHz is also allocated to the fixed service on a primary basis.     (WRC-03)

MOD           COM5/312/8            (B10/345/8)            (R8/386/153)

15.63-18.6 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) ADD 5.BC01 ADD 5.BC03 Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| MOD 5.514 | MOD 5.514 5.515 5.517 | MOD 5.514 |

ADD           COM5/312/9            (B10/345/9)            (R8/386/154)

5.BC01	In the band 17.3-17.7 GHz, earth stations of the fixed-satellite service (space-to-Earth) in Region 1 shall not claim protection from the broadcasting-satellite service feeder-link earth stations operating under Appendix 30A, nor put any limitations or restrictions on the locations of the broadcasting-satellite service feeder-link earth stations anywhere within the service area of the feeder link.     (WRC-03)

ADD           COM5/312/6            (B10/345/6)            (R8/386/155)

5.BC03	The following bands are identified for use by high-density applications in the fixed-satellite service (HDFSS):

17.3-17.7 GHz 		(space-to-Earth) in Region 1

18.3-19.3 GHz 		(space-to-Earth) in Region 2

19.7-20.2 GHz 		(space-to-Earth) in all Regions

39.5-40 GHz 		(space-to-Earth) in Region 1

40-40.5 GHz 		(space-to-Earth) in all Regions

40.5-42 GHz 		(space-to-Earth) in Region 2

47.5-47.9 GHz 		(space-to-Earth) in Region 1

48.2-48.54 GHz 		(space-to-Earth) in Region 1

49.44-50.2 GHz 		(space-to-Earth) in Region 1

and

27.5-27.82 GHz 		(Earth-to-space) in Region 1

28.35-28.45 GHz 		(Earth-to-space) in Region 2

28.45-28.94 GHz 		(Earth-to-space) in all Regions

28.94-29.1 GHz 		(Earth-to-space) in Region 2 and 3

29.25-29.46 GHz 		(Earth-to-space) in Region 2

29.46-30 GHz 		(Earth-to-space) in all Regions

48.2-50.2 GHz 		(Earth-to-space) in Region 2

This identification does not preclude the use of these bands by other fixed-satellite service applications or by other services to which these bands are allocated on a co-primary basis and does not establish priority in these Regulations among users of the bands. Administrations should take this into account when considering regulatory provisions in relation to these bands. See Resolution [COM5/6] (WRC-03).     (WRC-03)

MOD           COM5/312/13            (B10/345/13)            (R8/386/156)

15.63-18.6 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 (Earth-to-space) 5.520 MOBILE 5.519 5.521 |  |  |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 MOBILE |  |  |

MOD           COM4/272/83            (B5/290/101)            (R8/386/157)

5.512	Additional allocation:  in Algeria, Angola, Saudi Arabia, Austria, Bahrain, Bangladesh, Bosnia and Herzegovina, Brunei Darussalam, Cameroon, the Congo, Costa Rica, Egypt, El Salvador, the United Arab Emirates, Finland, Guatemala, India, Indonesia, Iran (Islamic Republic of), Jordan, Kenya, Kuwait, Libyan Arab Jamahiriya, Malaysia, Morocco, Mozambique, Nepal, Nicaragua, Oman, Pakistan, Qatar, Singapore, Slovenia, Somalia, Sudan, Swaziland, Tanzania, Chad, Yemen and Serbia and Montenegro, the band 15.7-17.3 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM4/272/84            (B5/290/102)            (R8/386/158)

5.514	Additional allocation:  in Algeria, Angola, Saudi Arabia, Austria, Bahrain, Bangladesh, Bosnia and Herzegovina, Cameroon, Costa Rica, El Salvador, the United Arab Emirates, Finland, Guatemala, Honduras, India, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Libyan Arab Jamahiriya, Nepal, Nicaragua, Oman, Uzbekistan, Pakistan, Qatar, Slovenia, Sudan and Serbia and Montenegro, the band 17.3-17.7 GHz is also allocated to the fixed and mobile services on a secondary basis. The power limits given in Nos. 21.3 and 21.5 shall apply.     (WRC-03)

MOD           COM4/272/85            (B5/290/103)            (R8/386/159)

5.521	Alternative allocation:  in Germany, Denmark, the United Arab Emirates and Greece, the band 18.1-18.4 GHz is allocated to the fixed, fixed-satellite (space-to-Earth) and mobile services on a primary basis (see No. 5.33). The provisions of No. 5.519 also apply.     (WRC-03)

MOD           COM5/312/14            (B10/345/14)            (R8/386/160)

18.6-22.21 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.522B MOBILE except aeronautical mobile Space research (passive) | 18.6-18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.522B ADD 5.BC03 MOBILE except aeronautical mobile SPACE RESEARCH (passive) | 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.522B MOBILE except aeronautical mobile Space research (passive) |
| 5.522A 5.522C | 5.522A | 5.522A |
| 18.8-19.3 FIXED FIXED-SATELLITE (space-to-Earth) 5.523A ADD 5.BC03 MOBILE |  |  |

18.6-22.21 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 Mobile-satellite (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 MOBILE-SATELLITE (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 Mobile-satellite (space-to-Earth) |
| 5.524 | 5.524 5.525 5.526 5.527 5.528 5.529 | 5.524 |
| 20.1-20.2 FIXED-SATELLITE (space-to-Earth) 5.484A ADD 5.BC03 MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 |  |  |

SUP           COM4/272/86            (B5/290/104)            (R8/386/161)

5.534

MOD           COM5/209/5            (B3/239/19)            (R8/386/162)

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 25.5-27 EARTH EXPLORATION-SATELLITE (space-to Earth) MOD 5.536A 5.536B FIXED INTER-SATELLITE 5.536 MOBILE SPACE RESEARCH (space-to-Earth) MOD 5.536A ADD 5.BE01 Standard frequency and time signal-satellite (Earth-to-space) |  |  |

ADD           COM5/209/7            (B3/239/21)            (R8/386/163)

5.BE01	In Algeria, Saudi Arabia, Bahrain, Botswana, Brazil, Cameroon, Comoros, Cuba, Djibouti, Egypt, United Arab Emirates, Estonia, Finland, Iran (Islamic Rep. of), Israel, Jordan, Kenya, Kuwait, Lithuania, Malaysia, Morocco, Nigeria, Oman, Qatar, Syrian Arab Republic, Somalia, Sudan, Tanzania, Tunisia, Uruguay, Zambia and Zimbabwe, earth stations operating in the space research service in the band 25.5-27 GHz shall not claim protection from, or constrain the use and deployment of, stations of the fixed and mobile services.     (WRC-03)

MOD           COM5/312/15            (B10/345/15)            (R8/386/164)

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.539 ADD 5.BC03 MOBILE 5.538 5.540 |  |  |
| 28.5-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.523A 5.539 ADD 5.BC03 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.1-29.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.523C 5.523E 5.535A 5.539 5.541A ADD 5.BC03 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.539 ADD 5.BC03 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.539 ADD 5.BC03 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.539 ADD 5.BC03 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) |
| 5.540 5.542 | 5.525 5.526 5.527 5.529 5.540 5.542 | 5.540 5.542 |

MOD           COM5/209/6            (B3/239/20)            (R8/386/165)

5.536A	Administrations operating earth stations in the Earth exploration-satellite service or the space research service shall not claim protection from stations in the fixed and mobile services operated by other administrations. In addition, earth stations in the Earth exploration-satellite service or in the space research service should be operated taking into account Recommendations ITU-R SA.1278 and ITU-R SA.1625, respectively.     (WRC-03)

MOD           COM5/303/11            (B7/324/7)            (R8/386/166)

5.537A	In Bhutan, Korea (Rep. of), Russian Federation, Indonesia, Iran (Islamic Republic of), Japan, Lesotho, Malaysia, Maldives, Mongolia, Myanmar, Uzbekistan, Pakistan, Philippines, Kyrgyzstan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 27.5-28.35 GHz may also be used by high altitude platform stations (HAPS). The use of HAPS within the band 27.5-28.35 GHz is limited, within the territory of the countries listed above, to a single 300 MHz sub-band. Such use of 300 MHz of the fixed-service allocation by HAPS in the above countries is further limited to operation in the HAPS-to-ground direction and shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems or other co-primary services. Furthermore, the development of these other services shall not be constrained by HAPS. See Resolution [COM5/17] (WRC-03).     (WRC-03)

MOD           COM5/312/16            (B10/345/16)            (R8/386/167)

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 29.9-30 FIXED-SATELLITE (Earth-to-space) 5.484A 5.539 ADD 5.BC03 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.525 5.526 5.527 5.538 5.540 5.542 |  |  |

MOD           COM5/164/5            (B2/190/10)            (R8/386/168)

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 31.8-32 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 5.547B MOD 5.548 |  |  |

MOD           COM5/164/6            (B2/190/11)            (R8/386/169)

32-32.3 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 MOD 5.547C MOD 5.548

MOD           COM5/164/7            (B2/190/12)            (R8/386/170)

32.3-33 FIXED 5.547A INTER-SATELLITE RADIONAVIGATION 5.547 5.547D MOD 5.548

MOD           COM5/303/12            (B7/324/8)            (R8/386/171)

5.543A	In Bhutan, Korea (Rep. of), Russian Federation, Indonesia, Iran (Islamic Republic of), Japan, Lesotho, Malaysia, Maldives, Mongolia, Myanmar, Uzbekistan, Pakistan, Philippines, Kyrgyzstan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 31-31.3 GHz may also be used by systems using high altitude platform stations (HAPS) in the ground-to-HAPS direction. The use of the band 31-31.3 GHz by systems using HAPS is limited to the territory of the countries listed above and shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems, systems in the mobile service and systems operated under No. 5.545. Furthermore, the development of these services shall not be constrained by HAPS. Systems using HAPS in the band 31-31.3 GHz shall not cause harmful interference to the radio astronomy service having a primary allocation in the band 31.3-31.8 GHz, taking into account the protection criterion as given in Recommendation ITU-R RA.769. In order to ensure the protection of satellite passive services, the level of unwanted power density into a HAPS ground station antenna in the band 31.3-31.8 GHz shall be limited to 106 dB(W/MHz) under clear-sky conditions, and may be increased up to –100 dB(W/MHz) under rainy conditions to take account of rain attenuation, provided the effective impact on the passive satellite does not exceed the impact under clear-sky conditions as given above. See Resolution [COM5/17] (WRC-03).     (WRC-03)

MOD           COM4/272/87            (B5/290/105)            (R8/386/172)

5.545	Different category of service:  in Armenia, Azerbaijan, Georgia, Mongolia, Kyrgyzstan, Tajikistan and Turkmenistan, the allocation of the band 31-31.3 GHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

MOD           COM4/272/88            (B5/290/106)            (R8/386/173)

5.546	Different category of service:  in Saudi Arabia, Armenia, Azerbaijan, Belarus, Egypt, the United Arab Emirates, Spain, Estonia, Finland, Georgia, Hungary, Iran (Islamic Republic of), Israel, Jordan, Latvia, Lebanon, Moldova, Mongolia, Uzbekistan, Poland, Syrian Arab Republic, Kyrgyzstan, Romania, the United Kingdom, the Russian Federation, South Africa, Tajikistan, Turkmenistan and Turkey, the allocation of the band 31.5-31.8 GHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. 5.33).     (WRC-03)

MOD           COM5/312/7            (B10/345/7)            (R8/386/174)

5.547	The bands 31.8-33.4 GHz, 37-40 GHz, 40.5-43.5 GHz, 51.4-52.6 GHz, 55.78-59 GHz and 64-66 GHz are available for high-density applications in the fixed service (see Resolutions 75 (WRC-2000) and 79 (WRC-2000)). Administrations should take this into account when considering regulatory provisions in relation to these bands. Because of the potential deployment of high-density applications in the fixed-satellite service in the bands 39.5-40 GHz and 40.5-42 GHz (see No. 5.BC03), administrations should further take into account potential constraints to high-density applications in the fixed service, as appropriate.     (WRC-03)

MOD           COM5/164/8            (B2/190/13)            (R8/386/175)

5.547C	Alternative allocation:  in the United States, the band 32-32.3 GHz is allocated to the radionavigation and space research (deep space) (space-to-Earth) services on a primary basis.     (WRC-03)

MOD           COM5/164/9            (B2/190/14)            (R8/386/176)

5.548	In designing systems for the inter-satellite service in the band 32.3-33 GHz, for the radionavigation service in the band 32-33 GHz, and for the space research service (deep space) in the band 31.8-32.3 GHz, administrations shall take all necessary measures to prevent harmful interference between these services, bearing in mind the safety aspects of the radionavigation service (see Recommendation 707).     (WRC-03)

MOD           COM4/272/89            (B5/290/107)            (R8/386/177)

5.549	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Egypt, the United Arab Emirates, Gabon, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kuwait, Lebanon, Libyan Arab Jamahiriya, Malaysia, Mali, Malta, Morocco, Mauritania, Nepal, Nigeria, Oman, Pakistan, the Philippines, Qatar, Dem. Rep. of the Congo, Syrian Arab Republic, Singapore, Somalia, Sudan, Sri Lanka, Togo, Tunisia and Yemen, the band 33.4-36 GHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

MOD           COM5/225/7            (B4/240/6)            (R8/386/178)

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547

MOD           COM5/209/9            (B3/239/23)            (R8/386/179)

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 35.5-36 METEOROLOGICAL AIDS EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.549 ADD 5.BE02 |  |  |

ADD           COM5/209/11            (B3/239/25)            (R8/386/180)

5.BE02	In the band 35.5-36.0 GHz, the mean power flux-density at the Earth’s surface, generated by any spaceborne sensor in the Earth exploration-satellite service (active) or space research service (active), for any angle greater than 0.8º from the beam centre shall not exceed 73.3 dB(W/m2) in this band.     (WRC-03)

MOD           COM5/312/17            (B10/345/17)            (R8/386/181)

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) 5.551AA ADD 5.BC03 MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) MOD 5.547 |  |  |

MOD           COM5/312/18            (B10/345/18)            (R8/386/182)

40-40.5 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40-40.5 EARTH EXPLORATION-SATELLITE (Earth-to-space) FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.BC03 MOBILE MOBILE-SATELLITE (space-to-Earth) SPACE RESEARCH (Earth-to-space) Earth exploration-satellite (space-to-Earth) |  |  |

MOD           COM4/272/90            (B5/290/108)            (R8/386/183)

5.550	Different category of service:  in Armenia, Azerbaijan, Belarus, Georgia, Mongolia, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan and Turkmenistan, the allocation of the band 34.7-35.2 GHz to the space research service is on a primary basis (see No. 5.33).     (WRC-03)

SUP           COM5/209/10            (B3/239/24)            (R8/386/184)

5.551A

SUP           COM5/225/6            (B4/240/5)            (R8/386/185)

5.551AA

MOD           COM5/312/19            (B10/345/19)            (R8/386/186)

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile MOD 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.BC03 BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) MOD 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile MOD 5.547 |

MOD           COM5/303/1            (B7/324/1)            (R8/386/187)

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F ADD 5.BC04 ADD 5.BC05 |  |  |

ADD           COM5/303/3            (B7/324/3)            (R8/386/188)

5.BC04	The equivalent power flux-density (epfd) produced in the band 42.5-43.5 GHz by all space stations in any non-geostationary-satellite system in the fixed-satellite service (space-to-Earth), or in the broadcasting-satellite service (space-to-Earth) operating in the 42-42.5 GHz band, shall not exceed the following values at a radio astronomy station for more than 2% of the time:

–230 dB(W/m2) in 1 GHz and –246 dB(W/m2) in any 500 kHz of the 42.5-43.5 GHz band at the site of a radio astronomy station registered as a single-dish telescope; and

–209 dB(W/m2) in any 500 kHz of the 42.5-43.5 GHz band at the site of a radio astronomy station registered as a very long baseline interferometry station.

These epfd values shall be evaluated using the methodology given in Recommendation ITU-R S.1586 and the reference antenna pattern and the maximum gain of an antenna in the radio astronomy service given in Recommendation ITU-R RA.1631 and shall apply over the whole sky and for elevation angles higher than the minimum operating angle θmin of the radiotelescope (for which a default value of 5° should be adopted in the absence of notified information).

These values shall apply at any radio astronomy station that either:

–	was in operation prior to the end of WRC-03 and has been notified to ITU before 4 January 2004; or

–	was notified before the date of receipt of the complete Appendix 4 information for coordination or notification, as appropriate, for the space station to which the limits apply.

Other radio astronomy stations notified after these dates may seek an agreement with administrations that have authorized the space stations. In Region 2, Resolution [COM5/7] (WRC-03) shall apply. The limits in this footnote may be exceeded at the site of a radio astronomy station of any country whose administration so agreed.     (WRC-03)

ADD           COM5/303/4            (B7/324/4)            (R8/386/189)

5.BC05	The power flux-density in the band 42.5-43.5 GHz produced by any geostationary space station in the fixed-satellite service (space-to-Earth), or the broadcasting-satellite service (space-to-Earth) operating in the 42-42.5 GHz band, shall not exceed the following values at the site of a radio astronomy station:

–137 dB(W/m2) in 1 GHz and –153 dB(W/m2) in any 500 kHz of the 42.5-43.5 GHz band at the site of a radio astronomy station registered as a single-dish telescope; and

–116 dB(W/m2) in any 500 kHz of the 42.5-43.5 GHz band at the site of a radio astronomy station registered as a very long baseline interferometry station.

These values shall apply at any radio astronomy station that either:

–	was in operation prior to the end of WRC-03 and has been notified to ITU before 4 January 2004; or

–	was notified before the date of receipt of the complete Appendix 4 information for coordination or notification, as appropriate, for the space station to which the limits apply.

Other radio astronomy stations notified after these dates may seek an agreement with administrations that have authorized the space stations. In Region 2, Resolution [COM5/7] (WRC-03) shall apply. The limits in this footnote may be exceeded at the site of a radio astronomy station of any country whose administration so agreed.     (WRC-03)

MOD           COM5/312/20            (B10/345/20)            (R8/386/190)

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47.2-47.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE 5.552A | 47.2-50.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.BC03 MOBILE |  |
| 47.5-47.9 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 (space-to Earth) ADD 5.BC03 ADD 5.BC06 MOBILE |  |  |
| 47.9-48.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE 5.552A |  |  |
| 48.2-48.54 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 (space-to Earth) ADD 5.BC03 ADD 5.BC06 ADD 5.BC07 MOBILE |  |  |
| 48.54-49.44 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE 5.149 5.340 5.555 |  |  |
| 49.44-50.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 (space-to Earth) ADD 5.BC03 ADD 5.BC06 ADD 5.BC07 MOBILE |  |  |
|  | 5.149 5.340 5.552A 5.555 |  |

ADD           COM5/312/21            (B10/345/21)            (R8/386/191)

5.BC06	The use of the bands 47.5-47.9 GHz, 48.2-48.54 GHz and 49.44-50.2 GHz by the fixed-satellite service (space-to-Earth) is limited to geostationary satellites.     (WRC-03)

ADD           COM5/312/22            (B10/345/22)            (R8/386/192)

5.BC07	The power flux-density in the band 48.94-49.04 GHz produced by any geostationary space station in the fixed-satellite service (space-to-Earth) operating in the bands 48.2-48.54 GHz and 49.44-50.2 GHz shall not exceed –151.8 dB(W/m2) in any 500 kHz band at the site of any radio astronomy station.     (WRC-03)

SUP           COM5/303/2            (B7/324/2)            (R8/386/193)

5.551G

SUP           COM4/272/91            (B5/290/109)            (R8/386/194)

5.555A

SUP           COM4/272/92            (B5/290/110)            (R8/386/195)

5.563

______________