| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 389-E |
|  |  |  | 2 July 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| B21 | PLENARY MEETING |
| --- | --- |
| TWENTy first SERIES OF TEXTS SUBMITTED BY THE EDITORIAL COMMITTEE TO THE PLENARY MEETING |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |

| Source | Document | Title |
| --- | --- | --- |
| PLEN | 365 | ARTICLE 5 |
|  |  | – band 5 925-6 700 MHz |
|  |  | – band 14-14.5 GHz |
|  |  | No. 5.AA16 |
|  |  | No. 5.AA17 |
|  |  | No. 5.AA18 |
|  |  | RESOLUTION 82 (WRC-2000) |
|  |  | RESOLUTION [COM4/20] (WRC-03) |
|  |  | RECOMMENDATION [COM4/B] (WRC-03) |

Annex: 9 pages

ARTICLE  5

Frequency allocations

MOD	(B21/389/1)

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 925-6 700 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.AA16 MOBILE 5.149 5.440 5.458 |  |  |

MOD 	(B21/389/2)

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Space research 5.505 |  |  |

MOD	(B21/389/3)

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Radionavigation-satellite | 14.3-14.4 Fixed FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.AA16 ADD 5.AA18 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.AA17 Radio astronomy 5.149 |  |  |

ADD 	(B21/389/4)

5.AA16	In the bands 5 925-6 425 MHz and 14-14.5 GHz, earth stations on board vessels may communicate with space stations of the fixed-satellite service. Such use shall be in accordance with Resolution [COM4/20] (WRC-03).     (WRC-03)

ADD	(B21/389/5)

5.AA17	In the band 14-14.5 GHz, ship earth stations with an e.i.r.p. greater than 21 dBW shall operate under the same conditions as earth stations on board vessels as provided in Resolution [COM4/20] (WRC-03). This footnote shall not apply to ship earth stations for which the complete Appendix 4 information has been received by the Radiocommunication Bureau prior to 5 July 2003.      (WRC-03)

ADD 	(B21/389/6)

5.AA18	Earth stations on board vessels communicating with space stations in the fixed-satellite service may operate in the frequency band 14-14.5 GHz without the need for prior agreement from Cyprus, Greece, Malta, […], within the minimum distance given in Resolution [COM4/20] (WRC-03) from these countries.      (WRC-03)

SUP 	(B21/389/7)

RESOLUTION  82  (WRC-2000)

Provisions relating to earth stations located on board vessels 
which operate in fixed-satellite service networks in the 
bands 3 700-4 200 MHz and 5 925-6 425 MHz

ADD 	(B21/389/8)

RESOLUTION  [COM4/20]  (WRC-03)

Provisions relating to earth stations located on board vessels 
which operate in fixed-satellite service networks in the 
uplink bands 5 925-6 425 MHz and 14-14.5 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that there is a demand for global wideband satellite communication services on vessels;

b) 	that the technology exists that enables earth stations on board vessels (ESVs) to use fixed-satellite service (FSS) networks operating in the uplink bands 5 925-6 425 MHz and 14-14.5 GHz;

c)	that ESVs are currently operating through fixed-satellite service (FSS) networks in the bands 3 700-4 200 MHz, 5 925-6 425 MHz, 10.7-12.75 GHz and 14-14.5 GHz under No. 4.4 of the Radio Regulations;

d)	that ESVs have the potential to cause unacceptable interference to other services in the bands 5 925-6 425 MHz and 14-14.5 GHz;

e)	that, with respect to the bands considered in this Resolution, global coverage is only available in the band 5 925-6 425 MHz and that only a limited number of geostationary FSS systems can provide such global coverage;

f)	that, without special regulatory provisions, ESVs could place a heavy coordination burden on some administrations, especially those in developing countries;

g)	that, in order to ensure the protection and future growth of other services, ESVs need to operate under certain technical and operational limitations;

h)	that, within ITU-R studies, based on agreed technical assumptions, minimum distances from the low-water mark as officially recognized by the coastal State have been calculated, beyond which an ESV will not have the potential to cause unacceptable interference to other services in the bands 5 925-6 425 MHz and 14-14.5 GHz;

i)	that, in order to limit the interference into other networks in the FSS, it is necessary to establish maximum off-axis e.i.r.p. density limits on ESV emissions;

j)	that establishing a minimum antenna diameter for ESVs has an impact on the number of ESVs that will ultimately be deployed, hence it will reduce interference into the fixed service,

noting

a)	that ESVs may be assigned frequencies to operate in FSS networks in the bands 3 700-4 200 MHz, 5 925-6 425 MHz, 10.7-12.75 GHz and 14-14.5 GHz pursuant to No. 4.4 of the Radio Regulations and shall not claim protection from, nor cause interference to, other services having allocations in these bands;

b)	that the regulatory procedures of Article 9 apply for ESVs operating at specified fixed points,

resolves

that ESVs transmitting in the 5 925-6 425 MHz and 14-14.5 GHz bands shall operate under the regulatory and operational provisions contained in Annex 1 and the technical limitations in Annex 2 of this Resolution,

encourages concerned administrations

to cooperate with administrations which license ESVs while seeking agreement under the above-mentioned provisions, and taking into consideration the provisions of Recommendation [COM4/B] (WRC-03),

instructs the Secretary-General

to bring this Resolution to the attention of the Secretary-General of the International Maritime Organization (IMO).

ANNEX 1  TO  RESOLUTION  [COM4/20]  (WRC-03)

Regulatory and operational provisions for ESVs transmitting in the 
5 925-6 425 MHz and 14-14.5 GHz bands

1	The administration that issues the licence for the use of ESVs in these bands (licensing administration) shall ensure that such stations follow the provisions of this Annex and thus do not present any potential to cause unacceptable interference to the services of other concerned administrations.

2	ESV service providers shall comply with the technical limitations listed in Annex 2 and, when operating within the minimum distances as identified in item 4 below, with the additional limitations agreed by the licensing and other concerned administrations.

3	In the 3 700-4 200 MHz band and 10.7-12.75 GHz range, ESVs in motion shall not claim protection from transmissions of terrestrial services operating in accordance with the Radio Regulations.

4	The minimum distances from the low-water mark as officially recognized by the coastal State beyond which ESVs can operate without the prior agreement of any administration are 300 km in the 5 925-6 425 MHz band and 125 km in the 14-14.5 GHz band, taking into account the technical limitations in Annex 2. Any transmissions from ESVs within the minimum distances shall be subject to the prior agreement of the concerned administration(s).

5	The potentially concerned administrations referred to in the previous item 4 are those where fixed or mobile services are allocated on a primary basis in the Table of Frequency Allocations of the Radio Regulations:

| Frequency bands | Potentially concerned administrations |
| --- | --- |
| 5 925-6 425 MHz | All three Regions |
| 14-14.25 GHz | Countries listed in No. 5.505, except those listed in No. 5.AA18 |
| 14.25-14.3 GHz | Countries listed in Nos. 5.505, 5.508 and 5.509, except those listed in No. 5.AA18 |
| 14.3-14.4 GHz | Regions 1 and 3, except countries listed in No. 5.AA18 |
| 14.4-14.5 GHz | All three Regions, except countries listed in No. 5.AA18 |

6	The ESV system shall include means of identification and mechanisms to immediately cease emissions, whenever the station does not operate in compliance with the provisions of items 2 and 4 above.

7	Cessation of emissions as referred to in item 6 above shall be implemented in such a way that the corresponding mechanisms cannot be bypassed on board the vessel, except under the provisions of No. 4.9.

8	ESVs shall be equipped so as to:

–	enable the licensing administration under the provisions of Article 18 to verify earth station performance; and

–	enable the cessation of ESV emissions immediately upon request by an administration whose services may be affected.

9	Each licence-holder shall provide a point of contact to the administration with which agreements have been reached for the purpose of reporting unacceptable interference caused by the ESV.

10	When ESVs operating beyond the territorial sea but within the minimum distance (as referred to in item 4 above) fail to comply with the terms required by the concerned administration pursuant to items 2 and 4, then that administration may:

–	request the ESV to comply with such terms or cease operation immediately; or

–	request the licensing administration to require such compliance or immediate cessation of the operation.

ANNEX  2  TO  RESOLUTION  [COM4/20]  (wrc-03)

Technical limitations applicable to ESVs transmitting in the bands
5 925-6 425 MHz and 14-14.5 GHz

|  | 5 925-6 425 MHz | 14-14.5 GHz |
| --- | --- | --- |
| Minimum diameter of ESV antenna | 2.4 m | 1.2 m* |
| Tracking accuracy of ESV antenna | 0.2 peak | 0.2 peak |
| Maximum ESV e.i.r.p. spectral density toward the horizon | 17 dB(W/MHz) | 12.5 dB(W/MHz) |
| Maximum ESV e.i.r.p. towards the horizon | 20.8 dBW | 16.3 dBW |
| Maximum off-axis e.i.r.p. density** | See below | See below |
| * While operations within the minimum distances are subject to specific agreement with concerned administrations, licensing administrations may authorize the deployment of smaller antenna sizes down to 0.6 m at 14 GHz provided that the interference to the terrestrial services is no greater than that which would be caused with an antenna size of 1.2 m, taking into account Recommendation ITU-R SF.1650. In any case, the use of smaller antenna size shall be in compliance with the tracking accuracy of ESV antenna, maximum ESV e.i.r.p. spectral density toward the horizon, maximum ESV e.i.r.p. towards the horizon and maximum off-axis e.i.r.p. density limits in the Table above and the protection requirements of the FSS intersystem coordination agreements. ** In any case, the e.i.r.p. off-axis limits shall be compliant with the FSS intersystem coordination agreements that may agree to more stringent off-axis e.i.r.p. levels. |  |  |

- Off-axis limits

For earth stations on board vessels operating in the 5 925-6 425 MHz band, at any angle specified below, off the main-lobe axis of an earth-station antenna, the maximum e.i.r.p. in any direction within 3° of the GSO shall not exceed the following values:

5 925-6 425 MHz

| Angle off-axis | Maximum e.i.r.p. per 4 kHz band |
| --- | --- |
| 2.5°    7° | (32 – 25 log ) dB(W/4 kHz) |
| 7° <   9.2° | 11 dB(W/4 kHz) |
| 9.2° <   48° | (35 – 25 log ) dB(W/4 kHz) |
| 48° <  < 180° | –7 dB(W/4 kHz) |

For earth stations on board vessels  operating in the 14.0-14.5 GHz band, at any angle  specified below, off the main-lobe axis of an earth station antenna, the maximum e.i.r.p. in any direction within 3° of the GSO shall not exceed the following values:

14.0-14.5 GHz

| Angle off-axis | Maximum e.i.r.p. in any 40 kHz band |
| --- | --- |
| 2°    7° | 33 – 25 log  dB(W/40 kHz) |
| 7° <   9.2° | 12 dB(W/40 kHz) |
| 9.2° <   48° | 36 – 25 log  dB(W/40 kHz) |
| 48° <   180° | –6 dB(W/40 kHz) |

ADD 	(B21/389/9)

RECOMMENDATION  [COM4/B]  (WRC-03)

Operational procedures for ESV use

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that under the provisions of Resolution [COM4/20] (WRC-03) transmissions from ESVs within the distances defined in item 4 of Annex 1 of  Resolution [COM4/20] (WRC-03) should be based upon prior agreement of concerned administrations;

b)	that it is desirable to provide guidance on activities to achieve such prior agreement with concerned administrations;

c)	that such guidance should include the operational procedures for ESV use,

recommends

that operation of ESVs follow the procedures set forth in Annex 1.

ANNEX  1  to  recommendation  [COM4/B]  (WRC-03)

Operational procedures for ESV use

# A	Initiation of contact

The ESV licensing administration or the licence-holder should contact, in advance of ESV operations within the minimum distances, the concerned administration(s) to obtain agreements that will establish the technical bases for avoiding unacceptable interference to the terrestrial facilities of the concerned administration or administrations.

The minimum distances and concerned administrations are defined in items 4 and 5 of Annex 1 of Resolution [COM4/20] (WRC-03), respectively.

# B	Recommended actions of licensing administrations, licence-holders and concerned administrations

•	The licensing administration or the licence-holder should provide the technical and operational parameters required by the concerned administration, among them, if required, information on the movement of the ship(s) equipped with ESVs within the minimum distances.

•	Concerned administrations that wish to permit the operation of ESVs should determine if they have terrestrial stations that could be affected by ESV operations with a view to identifying possible frequencies for ESV use that would avoid potential interference.

# C	Frequency use arrangements

National practices, as well as applicable Recommendations of ITU-R (such as ITU-R S.1587, SF.1585, SF.1648, SF.1649, SF.1650), may be used in reaching frequency usage arrangements.

# D	Avoidance of unacceptable interference

According to Annex 1 of Resolution [COM4/20] (WRC-03) the ESV licensing administration shall ensure that such stations do not cause unacceptable interference to the services of other concerned administrations. In the event that unacceptable interference occurs, the licence-holder must eliminate the source of any interference from its station immediately upon being advised of such interference. Additionally, the licence-holder shall immediately terminate transmissions at the request of either the concerned administration or the ESV licensing administration if either administration determines that the ESV is causing unacceptable interference or is otherwise not being operated in compliance with the operating agreement.

________________