| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 390-E |
|  |  |  | 2 July 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| B22 | PLENARY MEETING |
| --- | --- |
| TWENTy second SERIES OF TExts submitted by the editorial committee to the plenary meeting |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |

| Source | Document | Title |
| --- | --- | --- |
| PLEN | 359 | ARTICLE 5 |
|  |  | – band 6 765-8 100 kHz |
|  |  | No. 5.AC01 |
|  |  | No. 5.AC02 |
|  |  | No. 5.AC02bis |
|  |  | No. 5.AC03 |
|  |  | No. 5.AC04 |
|  |  | No. 5.AC05 |
|  |  | No. 5.AC05bis |
|  |  | No. 5.AC05ter |
|  |  | No. 5.AC06 |
|  |  | No. 5.AC07 |
|  |  | No. 5.139 |
|  |  | No. 5.142 |

Annex: 3 pages

ARTICLE  5

MOD	(B22/390/1)

6 765-8 100 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 6 765-7 000 FIXED Land mobileMOBILE except aeronautical mobile (R) 5.138 ADD 5.AC01 MOD 5.139 |  |  |
| 7 000-7 100 AMATEUR AMATEUR-SATELLITE 5.140 5.141 [ADD 5.AC02] |  |  |
| 7 100-7 200 AMATEUR [ADD 5.AC02] ADD 5.AC02bis ADD 5.AC03 MOD 5.142 |  |  |
| 7 100200-7 300 BROADCASTING | 7 100200-7 300 AMATEUR MOD 5.142 | 7 100200-7 300 BROADCASTING |
| 7 300-7 350400 BROADCASTING 5.134 5.143 ADD 5.AC05 ADD 5.AC05bis ADD 5.AC05ter ADD 5.AC06 |  |  |
| 7 400-7 450 BROADCASTING ADD 5.AC05bis ADD 5.AC05ter | 7 400-7 450 FIXED MOBILE except aeronautical mobile (R) | 7 400-7 450 BROADCASTING ADD 5.AC05 |
| 7 3450-8 100 FIXED MOBILE except aeronautical mobile (R) 5.144 ADD 5.AC07 |  |  |

ADD	(B22/390/2)

5.AC01	Until 29 March 2009, the band 6 765-7 000 kHz is allocated to the fixed service on a primary basis and to the land mobile service on a secondary basis. After this date, this band is allocated to the fixed and the mobile except aeronautical mobile (R) services on a primary basis.     (WRC-03)

MOD	(B22/390/3)

5.139	Different category of service:  until 29 March 2009, in Armenia, Azerbaijan, Belarus, Georgia, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 6 765-7 000 kHz to the land mobile service is on a primary basis (see No. 5.33).     (WRC-03)

ADD	(B22/390/4)

5.AC02	Additional allocation:  in [Uzbekistan, Kyrgyzstan …], the bands 7 000-7 100 kHz and 7 100-7 200 kHz are also allocated to the fixed and land mobile services on a secondary basis.     (WRC-03)

ADD	(B22/390/5)

5.AC02bis	Additional allocation:  after 29 March 2009, in [Algeria, Saudi Arabia, Australia, Botswana, Brunei Darussalam, China, Korea (Rep. of), Diego Garcia, Egypt, United Arab Emirates, Indonesia, Iran (Islamic Republic of), Japan, New Zealand, Papua New Guinea, Syrian Arab Republic, Singapore, Viet Nam, ...], the band 7 100-7 200 kHz is also allocated to the fixed and the mobile except aeronautical mobile (R) services on a primary basis.     (WRC-03)

ADD	(B22/390/6)

5.AC03	In Regions 1 and 3, the band 7 100-7 200 kHz is allocated to the broadcasting service until [29 March 2009] on a primary basis.     (WRC-03)

MOD	(B22/390/7)

5.142	Until [29 March 2009], the use of the band 7 100-7 300 kHz in Region 2 by the amateur service shall not impose constraints on the broadcasting service intended for use within Region 1 and Region 3. After [29 March 2009] the use of the band 7 200-7 300 kHz in Region 2 by the amateur service shall not impose constraints on the broadcasting service intended for use within Region 1 and Region 3.     (WRC-03)

ADD	(B22/390/8)

5.AC05	In Region 1, the band 7 350-7 450 kHz is allocated, until 29 March 2009, to the fixed service on a primary basis and to the land mobile service on a secondary basis. After 29 March 2009, frequencies in this band may be used by stations in the above-mentioned services, communicating only within the boundary of the country in which they are located, on condition that harmful interference is not caused to the broadcasting service. When using frequencies for these services, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations.     (WRC-03)

ADD	(B22/390/9)

5.AC05bis	In Region 1, the band 7 350-7 450 kHz is allocated, until 29 March 2009, to the fixed service on a primary basis and to the land mobile service on a secondary basis. After 29 March 2009, on condition that harmful interference is not caused to the broadcasting service, frequencies in the band 7 350-7 450 kHz may be used by stations in the fixed and land mobile services communicating only within the boundary of the country in which they are located, each station using a total radiated power of each station shall not exceed 24 dBW.     (WRC-03)

ADD	(B22/390/10)

5.AC05ter	Alternative allocation:  after 29 March 2009 in [Algeria, Saudi Arabia, Egypt, United Arab Emirates, Syrian Arab Republic, ...], the bands 7 350-7 400 kHz and 7 400-7 450 kHz will continue to be used in the fixed service on a primary basis.     (WRC-03)

ADD	(B22/390/11)

5.AC06	In Region 2, the band 7 350-7 400 kHz is allocated, until 29 March 2009, to the fixed service on a primary basis and to the land mobile service on a secondary basis. After 29 March 2009, frequencies in this band may be used by stations in the above-mentioned services, communicating only within the boundary of the country in which they are located, on condition that harmful interference is not caused to the broadcasting service. When using frequencies for these services, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations.     (WRC-03)

ADD	(B22/390/12)

5.AC07	Until 29 March 2009, the band 7 450-8 100 kHz is allocated to the fixed service on a primary basis and to the land mobile service on a secondary basis.     (WRC-03)

_______________