| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 392-E |
|  |  |  | 2 July 2003 |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| B24 | PLENARY MEETING |
| --- | --- |
| twenty-fourth SERIES OF TExts submitted by the editorial committee to the plenary meeting |  |
| The following texts are submitted to the Plenary Meeting for first reading: |  |

| Source | Document | Title |
| --- | --- | --- |
| PLEN | 369 | ARTICLE 5 |
|  |  | – band 2 520-2 655 MHz |
|  |  | – Number 5.418 |
|  |  | – Number 5.418bis |
|  |  | – Number 5.418A |
|  |  | – Number 5.418Abis |
|  |  | – Number 5.418B |
|  |  | – Number 5.418Bbis |
|  |  | – Number 5.418C |
|  |  | – Number 5.418Cbis |

Annex: 4 pages

article  5

Frequency allocations

ADD	(B24/392/1)

5.418bis	In applying provision No. 5.418, in Korea (Rep. of) and Japan, resolves 3 of Resolution 528 (Rev.WRC-03) is relaxed to allow the broadcasting-satellite service (sound) and the complementary terrestrial broadcasting service to additionally operate on a primary basis in the band 2 605-2 630 MHz. This use is limited to systems intended for national coverage. An administration listed in this provision shall not have simultaneously two overlapping frequency assignments, one under this provision and the other under No. 5.416. The provisions of No. 5.416 and Table 21-4 of Article 21 do not apply. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) in the band 2 605-2 630 MHz is subject to the provisions of Resolution 539 (Rev.WRC-03). The power flux-density at the Earth’s surface produced by emissions from a geostationary broadcasting-satellite service (sound) space station operating in the band 2 605-2 630 MHz for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, for all conditions and for all methods of modulation, shall not exceed the following limits:

130     dB(W/(m2 · MHz))	for	0				5

130  0.4 (  5)     dB(W/(m2 · MHz))	for	5				25

122     dB(W/(m2 · MHz))	for	25				90

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees. These limits may be exceeded on the territory of any country whose administration has so agreed. As an exception to the limits above, the pfd value of –122 dB(W/(m2 · MHz)) shall be used as a threshold for coordination under No. 9.11 in an area of 1 000 km around the territory of the administration notifying the BSS (sound) system, for angles of arrival greater than 35 degrees.

ADD	(B24/392/2)

5.418Abis	In Korea (Rep. of) and Japan, use of the band 2 605-2 630 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 4 July 2003, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 5 July 2003.

ADD	(B24/392/3)

5.418Bbis	Use of the band 2 605-2 630 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003, is subject to the application of the provisions of No. 9.12.

ADD	(B24/392/4)

5.418Cbis	Use of the band 2 605-2 630 MHz by geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, has been received after 4 July 2003 is subject to the application of the provisions of No. 9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418bis, and No. 22.2 does not apply.

MOD	(B24/392/5)

2 520-2 700 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 520-2 655 FIXED 5.409 5.410 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 655 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 | 2 520-2 535 FIXED 5.409 5.411 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
|  |  | 5.403 5.415A |
|  |  | 2 535-2 655 FIXED 5.409 5.411 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 |
| 5.339 5.403 5.405 5.412 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.403 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis | 5.339 5.418 ADD 5.418bis 5.418A ADD 5.418Abis 5.418B ADD 5.418Bbis 5.418C ADD 5.418Cbis |

Note to the Editorial Committee – The Editorial Committee is requested to update the text of Resolution 539, as revised by this Conference, to cover the band 2 605-2 655 MHz.

[MOD	(B24/392/6)

5.418	Additional allocation:  in Korea (Rep. of), India, Japan, Pakistan and Thailand, the band 2 535-2 655 MHz is also allocated to the broadcasting-satellite service (sound) and complementary terrestrial broadcasting service on a primary basis. Such use is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92). The provisions of No. 5.416 and Table 21-4 of Article 21, do not apply to this additional allocation. Use of non-geostationary-satellite systems in the broadcasting-satellite service (sound) is subject to Resolution 539 (Rev.WRC-03). Geostationary BSS (sound) systems for which complete Appendix 4 coordination information has been received after 1 June 2005 are limited to systems intended for national coverage. The power flux-density at the Earth’s surface produced by emissions from a geostationary broadcasting-satellite service (sound) space station operating in the band 2 630-2 655 MHz, and for which complete Appendix 4 coordination information has been received after 1 June 2005, shall not exceed the following limits, for all conditions and for all methods of modulation:

130     dB(W/(m2 · MHz))	for	0				5

130  0.4 (  5)     dB(W/(m2 · MHz))	for	5				25

–122     dB(W/(m2 · MHz)) 	for	25				90

where  is the angle of arrival of the incident wave above the horizontal plane, in degrees. These limits may be exceeded on the territory of any country whose administration has so agreed. As an exception to the limits above, the pfd value of –122 dB(W/(m2 · MHz)) shall be used as a threshold for coordination under No. 9.11 in an area of [1 000 km] around the territory of the administration notifying the BSS (sound) system, for angles of arrival greater than [35 degrees].

In addition, an administration listed in this provision shall not have simultaneously two overlapping frequency assignments, one under this provision and the other under No. 5.416 for systems for which complete Appendix 4 coordination information has been received after 1 June 2005.     (WRC-03)]

MOD	(B24/392/7)

5.418A	In certain Region 3 countries listed in No. 5.418, use of the band 2 630-2 655 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound) for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. 9.12A, in respect of geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received after 2 June 2000, and No. 22.2 does not apply. No. 22.2 shall continue to apply with respect to geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, is considered to have been received before 3 June 2000.-     (WRC-03)

MOD	(B24/392/8)

5.418B	Use of the band 2 630-2 655 MHz by non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418, for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. 9.12.-     (WRC-03)

MOD	(B24/392/9)

5.418C	Use of the band 2 630-2 655 MHz by geostationary-satellite networks for which complete Appendix 4 coordination information, or notification information, has been received after 2 June 2000 is subject to the application of the provisions of No. 9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), pursuant to No. 5.418 and No. 22.2 does not apply.     (WRC-03)