| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5D/TEMP/259(Rev.2) | Document IMT-ADV/7-E |
|  | 23 October 2009 |
|  | English only |
| Working Party 5D |  |
| ACKNOWLEDGEMENT OF CANDIDATE SUBMISSION FROM TTA UNDER STEP 3 OF THE IMT-ADVANCED PROCESS (IEEE TECHNOLOGY) |  |

# 1)	Scope

To provide for the further work of WP 5D and the Independent Evaluation Groups, WP 5D has reviewed the candidate technology submission referenced above as received by WP 5D in the October 2009 meeting and provides the WP 5D view on the following:

1)	The completeness of the candidate submission following the guidance of Report ITU-R M.2133.

2)	Details of the required components that were to be provided as defined in Report ITU-R M.2133 and where the information is to be found within the candidate submission materials.

# 2)	Acknowledgement of receipt of submission and assessment of completeness

WP 5D acknowledges the receipt of the candidate technology submission referenced above from TTA.  WP 5D has reviewed this candidate submission under the IMT-Advanced process and has determined that the submission is “complete” per Section 4 of Report ITU-R M.2133.

# 3)	Classification of the candidate submission

RIT, including both TDD and FDD duplexing.

# 4)	Designate the following elements for each candidate RIT, for each candidate RIT within the composite SRIT, and/or for the composite SRIT of the candidate submission (to fulfil Section 4.1 of Report ITU-R M.2133)

To assist WP 5D and the Independent Evaluation Groups in guiding the work, WP 5D has developed this checklist of the information with regard to all the required components of the submission per Report ITU-R M.2133 including specific links to the candidate submission items.

4.1)	Step 2 requirements:  An RIT needs to fulfil the requirements for at least one test environment. Furthermore, an SRIT is defined as a number of RITs each individually fulfilling the minimum requirements for at least one test environment and complementing each other. (Document IMT-ADV/2(Rev.1) Step 2)

Based on Document IMT-ADV/2(Rev.1) Step 2, does the proponent indicate that the condition above has been met?

Yes	⁪ No		⁪Comments: The proposal indicates that it fulfils the requirements in all four test environments (from Document 5D/560).

4.2a)	Templates: The submission of each candidate RIT or SRIT shall consist of completed templates as specified in § 4.2 together with any additional inputs which the proponent may consider relevant to the evaluation.  (M.2133 Section 4.1 § 1)

Description template – characteristics template supplied (M.2133 Section 4.2.3.2)

Yes		⁪ No		⁪ Comments: See Attachment 1 Subclause 6.1

Description template – link budget template supplied (M.2133 Section 4.2.3.3)

Yes		⁪ No		⁪ Comments: See Attachment 1 Subclause 6.2

Compliance template for services supplied (M.2133 Section 4.2.4.1)

Yes		⁪ No		⁪ Comments: See Attachment 1 Subclause 8.1

Compliance template for spectrum supplied (M.2133 Section 4.2.4.2)

Yes		⁪ No		⁪ Comments: See Attachment 1 Subclause 8.2

Compliance template for technical performance supplied (M.2133 Section 4.2.4.3)

Yes		⁪ No		⁪ Comments: See Attachment 1 Subclause 8.3

4.2b)	Version: Each proposal must also indicate the version of the minimum technical requirements and evaluation criteria of the IMT-Advanced currently in force that it is intended for and make reference to the associated requirements.

Summarize the version of the minimum technical requirements and evaluation criteria utilized by the candidate submission.

- The versions used are: Report ITU-R M.2133 Requirements, evaluation criteria and submission templates for the development of IMT-Advanced (Approved 2008-11); Report ITU-R M.2134 Requirements related to technical performance for IMT-Advanced radio interface(s) (Approved 2008-11); Report ITU-R M.2135 Guidelines for evaluation of radio interface technologies for IMT-Advanced  (Approved 2008-11), and Document IMT-ADV/3 Correction of typographical errors and provision of missing texts of IMT-Advanced channel models in Report ITU-R M.2135 (July 2009).

4.3)	Self evaluation: The entity that proposes a candidate RIT or SRIT to the ITU-R (the proponent) shall include with it either an initial self-evaluation or the proponents’ endorsement of an initial evaluation submitted by another entity and based on the compliance templates in § 4.2.4. (M.2133 Section 4.1 § 2).

Self-evaluation supplied    Yes	⁪ No		⁪ Comments: See Attachment 1 Clause 7

Self Evaluation type 	⁪   provided by proponent      
⁪

4.4)	IPR STATEMENT: Proponents and IPR holders should indicate their compliance with the ITU policy on intellectual property rights ((M.2133 Section 4.1 §3)

IPR Statements supplied:  Yes	⁪ No		⁪ Comments (specify): See Attachment 1 Clause 3

# 5)	Use of corrected information in Report ITU-R M.2135

Working Party 5D has identified some aspects of Report ITU-R M.2135 that require an update of the report to address the correction of typographical errors and provision of some missing text.  This information was developed in the 5th meeting of Working Party 5D in Document 5D/TEMP/219(Rev.1).  This was captured in Document IMT-ADV/3.

In particular it is stated that the information presented in Document IMT-ADV/3 should be applied in the use of Report ITU-R M.2135. Technology proponents and evaluation groups are requested to indicate in their inputs to Working Party 5D that they have used both Report ITU-R M.2135 and Document IMT-ADV/3 in their work.

Has the technology proponent confirmed they used both Report ITU-R M.2135 and Document IMT-ADV/3 in their work?  Yes	 ⁪ No		⁪ Comments (specify)

# 6)	Candidate submission

See Attachment 1.

# 7)	Contacts

Dae Jung KIM				E-mail: kdj@tta.or.kr

# 8)	Remarks or other information

Regarding Document 5D/560 from TTA, TTA indicated to the 6th meeting of WP 5D that for the purposes of evaluation, this candidate technology submission is technically the same as the candidate technology submission provided in Document 5D/542.

WP 5D has concluded that because Documents 5D/560 and 5D/542 are technically identical submissions a single evaluation is applicable for both.

Attachment: 	1

Attachment 1

Source: Document 5D/560

Director, Radiocommunication Bureau

SUBMISSION OF A CANDIDATE IMT-ADVANCED RIT
BASED ON IEEE 802.16

In response to the ITU-R Circular Letter 5/LCCE/2 which invites proposals for candidate radio interface technologies for the terrestrial component of IMT-Advanced, TTA is pleased to propose IEEE 802.16 technology for TTA’s submission of a candidate radio interface technology. This proposal presents a complete submission (per § 4.1 of Report ITU-R M.2133) as required in the Step 3 of the IMT-Advanced process in Document IMT-ADV/2(Rev.1).

This document comprises four parts: Part 1 includes the overview of TTA’s proposal, Part 2 includes the indication to the compliance with ITU-R IPR policy and the declaration of version of minimum requirement, Part 3 includes the Description Template which is defined in Report ITU-R M.2133, and Part 4 includes self evaluation report and compliance templates.

TABLE OF CONTENTS

Part 1

1	Introduction and background

2	Overview of TTA’s proposal

Part 2

3	 IPR Policy compliance

4	Declaration of version

References

Abbreviations

Part 3

5	General description of the RIT

6	Description templates

6.1	Description template – Characteristics

6.2	Description template – Link budget

Annex 1 - L1/L2 overhead calculation

Annex 2 - Stage 2 specification: IEEE 802.16m system description Document (SDD)

Part 4

7	Self-evaluation report

7.1	Overview

7.2	Evaluation results

8	Compliance templates

8.1	Compliance template for services

8.2 	Compliance template for spectrum

8.3	Compliance template for technical performance

Annex 3 - Simulation assumptions and configuration parameters for IMT-Advanced test environments

Annex 4 - Details of simulation-related results in the compliance template for technical performance

PART 1

# 1	Introduction and background

The Telecommunications Technology Association (“TTA”) was established under the Telecommunications Act of the Republic of Korea and represents the ICT standardization organization, providing one-stop service for the establishment of ICT standards as well as the test and the certification for ICT products. TTA has contributed to the technology advance and the information and telecommunication service promotion by the effective establishment and the providing of ICT standards in close collaboration with member companies, organizations and groups concerning information and telecommunications such as network operators, service providers, equipment manufacturers, academia, R&D institutes and so on.

TTA has had close relationship with IEEE 802.16 WG since TTA has standardized its WiBro standards which has been harmonized with IEEE 802.16e standard. With this regard, TTA was one of contributor for OFDMA TDD WMAN in the IMT-2000 standardization in ITU-R. In addition, the IMT Wibro Project Group (PG702) under the TTA Technical Committee (TC7) on mobile communication maintains the close cooperation with IEEE 802.16 WG through the liaison relationship.

TTA, one of proponents for IEEE 802.16m technology in the development of IMT-Advanced in ITU-R, intends to propose 802.16m technology as a candidate Radio Interface Technology (RIT) for the terrestrial component of IMT-Advanced technology.

# 2	Overview of TTA’s Proposal

Report ITU-R M.2133 Section 4.1 specifies that three major components which are required for a complete submission. The following information shows where each of three major components are located in this complete submission.

- First component: Completed templates specified in Section 4.2 of Report ITU-R M.2133

  - The completed  description templates can be found at Clause 6 of Part 3 in this document.

  - The completed compliance templates can be found at Clause 8 of  Part 4 in this document.

  - The declaration of the version can be found at Clause 4 of Part 2 in this document.

- Second component: Self-evaluation

  - The self-evaluation can be found at Clause 7 of Part 4 in this document.

- Third component: IPR policy compliance

  - The IPR policy compliance information can be found at Clause 3 of Part 2 in this document.

It is recognized that the self-evaluation results of the proposed technology meet the minimum technical requirements which are defined in the Report ITU-R M.2134. Therefore, it is proposed that TTA’s  proposal should be considered as a candidate RIT in the evaluation process which is under the Step 4 of the development of IMT-Advanced in ITU-R as described in the Document IMT-ADV/2(Rev.1).

TTA also  provides its intention to support the evaluation activities of candidate technolgies for IMT-Advanced as an independent evaluation group during this evaluation process.

PART 2

# 3	IPR Policy compliance

Per Report ITU-R M.2133, “Proponents and IPR holders should indicate their compliance with the ITU policy on intellectual property rights(see Annex 1 of Resolution ITU-R 1), as specified in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC available at http://www.itu.int/ITU-T/dbase/patent/patent-policy.html. ”

TTA does comply with the ITU policy on IPR. The TTA, as a standards developing organization, maintains a patent policy applicable to the development of its standards. The TTA patent policy is described in “Guidelines for the Handling of Intellectual Property Rights in connection with TTA Standards” <http://tta.or.kr/English/TTArules-5.pdf>.

TTA will maintain a list of IPR declaration which has received from patent holders in regards to IEEE 802.16m technology including its amendments. The list of IPR declaration will be available at the following URL:

< http://www.tta.or.kr/English/new/external_relations/IMT-Advanced.jsp >

# 4	Declaration of version

Report ITU-R M.2133 specifies that IMT-Advanced proposal must “indicate the version of the minimum technical requirements and evaluation criteria of the IMT-Advanced currently in force that it is intended for and make reference to the associated requirements.”

This proposal specifies that it addresses IMT-Advanced requirements and evaluation criteria per Reports ITU-R M.2133, ITU-R M.2134 and ITU-R M.2135, subject to the corrections in Document IMT-ADV/3.

# References

{1}	IEEE Std 802.16-2009, IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Broadband Wireless Access Systems, May 2009 <http://ieee802.org/16/pubs/80216-2009.html> (transposition of draft IEEE P802.16Rev2/D9 <http://ties.itu.int/u/itu-r/ede/rsg5/IMT-2000/GCS/GCSrev9/5-6>).

{2}	IEEE 802.16m-07/002, IEEE 802.16m System Requirements Document (SRD) <http://ieee802.org/16/tgm/core.html#07_002>.

{3}	IEEE 802.16m-08/004, IEEE 802.16m Evaluation Methodology Document (EMD) <http://ieee802.org/16/tgm/core.html#08_004>.

{4}	IEEE 802.16m-09/0034, IEEE 802.16m System Description Document (SDD) <http://ieee802.org/16/tgm/core.html#09_0034> (see also Annex 2 in Part 3).

{5}	Report ITU-R M.2133, Requirements, evaluation criteria and submission templates for the development of IMT-Advanced, 2008 <http://www.itu.int/publ/R-REP-M.2133-2008/en>.

{6}	IEEE 802.16m-09/0047, System Evaluation Details for IEEE 802.16 IMT-Advanced Proposal, 23 September 2009 <http://ieee802.org/16/tgm/core.html#09_0047>

{7}	Report ITU-R M.2135, Guidelines for evaluation of radio interface technologies for IMT-Advanced, November 2008 <http://www.itu.int/publ/R-REP-M.2135-2008/en>.

{8}	Report ITU-R M.2134, Requirements related to technical system performance for IMT-Advanced radio interface(s), November 2008 <http://www.itu.int/publ/R-REP-M.2134-2008/en>.

{9}	Recommendation ITU-R M.1822, Framework for Services Supported by IMT, October 2007.

{10}	Andrea Goldsmith, Wireless Communications, Cambridge University Press, New York, 2005.

{11}	WMF-T23-007-R010v02, WiMAX Forum® Mobile System Profile – Release 1 – IMT-2000 Edition, 10 December 2008 <http://wimaxforum.org/imt-2000/9/WMF-T23-007-R010v02_MSP-IMT-2000.pdf> (transposition of WiMAX Forum Mobile System Profile Release 1 - IMT-2000 Edition <http://ties.itu.int/u/itu-r/ede/rsg5/IMT-2000/GCS/GCSrev9/5-6>).

{12}	IEEE C802.16-09/0012, Summary of Simulator Calibrations for IMT-Advanced (input contribution), 24 September 2009 <http://ieee802.org/16/docs/#C09_0012>.

{13}	IEEE C802.16-09/0013, IEEE 802.16m Evaluation Results for PCT for IMT-Advanced (input contribution), 24 September 2009 <http://ieee802.org/16/docs/#C09_0013>.

{14}	IEEE C802.16-09/0014, IEEE 802.16m Evaluation Results for LBT for IMT-Advanced (input contribution), 24 September 2009 <http://ieee802.org/16/docs/#C09_0014>.

# Abbreviations

ABS			Advanced BS

A-MAP			Advanced-MAP

A-A-MAP		Assignment Advanced MAP

AMC			Adaptive Modulation and Coding

AMS			Advanced MS

ARQ			Automatic Repeat Request

ARS			Advanced Relay Station

ASN			Access Service Network

ASN-GW		ASN Gateway

BPSK			Binary Phase Shift Keying

BS			Base Station

BSID			Base Station Identifier

BW-REQ		Bandwidth Request

DL			Downlink

CC			Convolutional Code

CDD			Cyclic Delay Diversity

CDR			Conjugate Data Repetition

CINR			Carrier to Interference and Noise Ratio

CLPC			Closed Loop Power Control

CP			Cyclic Prefix

CPS			Common Part Sublayer

CQI			Channel Quality Indicator

CRC			Cyclic Redundancy Check

CRU			Contiguous Resource Unit

CS			Convergence Sublayer

CSG			Closed Subscriber Group

CSI			Channel State Information

CTC			Convolutional Turbo Code

DHCP			Dynamic Host Configuration Protocol

DRU			Distributed Resource Unit

EAP			Encryption/Authentication Processes

E-MBS			Enhanced MBS

FA			Frequency Assignments

FBCH			Feedback Control Channel

FCC			Federal Communications Commission

FDD			Frequency Division Duplex

FEC			Forward Error Correction

F-FDD			Full Duplex FDD

FFT			Fast Fourier Transform

FID			Flow Identifier

GRA			Group Resource Allocation

HARQ			Hybrid ARQ

H-FBCH		HARQ Feedback Control Channel

H-FDD			Half-Duplex FDD

HF-A-MAP		HARQ Feedback A-MAP

HO			Handover

H-PURDA		Hard Public Use Reservation by Departure Allocation

IE			Information Element

IR			Initial Ranging

IETF			Internet Engineering Task Force

IFFT			Inverse Fast Fourier Transform

InH			Indoor Hotspot scenario

IP			Internet Protocol

LBS			Location Based Services

LRU			Logical Resource Unit

LT-CM			Long-Term Covariance Matrix

MAC			Medium Access Control Layer

MAHO			Mobile Assisted HO

MBS			Multicast Broadcast Services

MBSFN		Multicast-Broadcast Single Frequency Network

MIMO			Multiple Input Multiple Output

ML			Maximum Likelihood

MLRU			A-MAP LRU

MMIB			Mean Mutual Information per Bit

MMSE			Minimum Mean Squared Error

MPDU			MAC PDU

MPEG			Moving Picture Expert Group

MRC			Maximal Ratio Combiner

MSDU			MAC SDU

MU-MIMO		Multi User MIMO

NS/EP			National Security/Emergency Preparedness

OFDMA		Orthogonal Frequency Division Multiple Access

OLPC			Open Loop Power Control

MCS			Modulation and Coding Scheme

MCW			Multi-Codeword

MS			Mobile Station

NRM			Network Reference Model

NUS-A-MAP	Non-User-Specific A-MAP

OSG			Open Subscriber Group

PA			Persistent Allocation

PAPR			Peak to Average Power Ratio

PDU			Program Data Unit

P-FBCH		Primary FBCH

PHY			Physical Layer

PKM			Privacy Key Management

PMI			Precoding Matrix Index

P-SFH			Primary Superframe Header

QAM			Quadrature Amplitude Modulation

QoS			Quality of Service

QPSK			Quadrature Phase Shift Keying

RBIR			Received Bit mutual Information Rate

RIT			Radio Interface Technology

RMa			Rural Macro scenario

RRA			Radio Resource Agent

RRC			Radio Resource Control

RRCM			Radio Resource Control and Management

RRM			Radio Resource Management

RS			Relay Station

RTD			Round Trip Delay

RTG			Receive-to-transmit Transition Gap

RTP			Real-Time Protocol

RTT			Round Trip Time

RU			Resource Unit

RX			Receiver

S-ABS			Serving ABS

SCW			Single Codeword

SDU			Service Data Unit

SFBC			Space-Frequency Block Coding

S-FBCH		Secondary FBCH

SFH			Superframe Header

SID			Silence Insertion Descriptor

SINR			Signal to Interference and Noise Ratio

SON			Self-Organizing Network

S-SFH			Secondary Superframe Header

STID			Station Identifier

STC			Space-Time Coding

SU-MIMO		Single User MIMO

T-ABS			Target ABS

TBCC			Tail-Biting Convolutional Coding

TDD			Time Division Duplex

TDOA			Time Difference of Arrival

TSDID			Temporary STID

TTG			Transmit-to-receive Transition Gap

TTI			Transmission Time Interval

TTR			Time Division Transmit/Receive

TX			Transmitter

UDP			User Datagram Protocol

UL			Uplink

UMa			Urban Macro-cell scenario

UMi			Urban Micro-cell scenario

VAF			Voice Activity factor

VoIP			Voice over IP

PART 3

# 5	General description of the RIT

This clause provides a general description of the Radio Interface Technology (RIT) proposal. The detailed specification of the RIT is under development as an amendment (P802.16m) to IEEE Std 802.16 {1}. Much of the basic functionality of the RIT is inherited from {1}. The advanced functionality of the RIT is described in the Stage 2 Specification IEEE 802.16m-09/0034 (IEEE 802.16m System Description Document (SDD)) {4}. Additional details are provided below.

## 5.1	Network Reference Model (NRM) and Protocol Structure

Figure 5-1 illustrates the non-hierarchical, end-to-end NRM. Further details are provided in {4}. The NRM can be expanded to further include optional relay entities for coverage and performance enhancement in future releases although the RIT includes functionality to support such entities. The RIT specifies Medium Access Control (MAC) and Physical Layer (PHY) protocols for fixed and mobile broadband wireless access systems based on IEEE Std 802.16 {1}. The MAC and PHY functions can be classified into three categories namely data plane, control plane, and management plane. The data plane comprises of functions in the data processing path such as header compression as well as MAC and PHY data packet processing functions. A set of Layer-2 (L2) control functions is needed to support various radio resource configuration, coordination, signalling, and management. This set of functions is collectively referred to as control plane functions. A management plane is also defined for external management and system configuration. Therefore, all management entities fall into the management plane category.

FIGURE 5-1

Network Reference Model (NRM) {4}

The RIT MAC layer is composed of two sublayers: convergence sublayer (CS) and MAC common part sublayer (MAC CPS) {1}. For convenience, we logically classify MAC CPS functions into two groups based on their characteristics as shown in Figure 5-2.

The upper and lower classes are called Radio Resource Control and Management (RRCM) functional group and MAC functional group, respectively. The control plane functions and data plane functions are also separately classified. As shown in Figure 5-2, the RRCM functional group comprises of several functional blocks including:

- Radio resource management: This block adjusts radio network parameters related to the traffic load, and also includes the functions of load control (load balancing), admission control and interference control.

- Mobility management: This block scans neighboring Advanced Base Stations (ABS) and decides whether Advanced Mobile Station (AMS) should perform handover operation.

- Network-entry management: This block controls initialization and access procedures and generates management messages during initialization and access procedures.

- Location management: This block supports Location Based Service (LBS), generates messages including the LBS information.

- Idle mode management: This block controls idle mode operation, and generates the paging advertisement message based on paging message from paging controller in the core network. This block manages location update operation during idle mode.

- Security management: This block performs key management for secure communication. Using managed key, traffic encryption/decryption and authentication are performed.

- System configuration management: this block manages system configuration parameters, and generates broadcast control messages such as Superframe Header (SFH).

- Multicast-Broadcast Service (MBS): This block controls and generates management messages and data associated with MBS.

- Service flow and connection management: This block allocates Station Identifier (STID) and Flow Identifiers (FIDs) during access/handover service flow creation procedures.

Figure 5-2

RIT Protocol Stack {4}

MAC functional group includes functional blocks which are related to PHY and link controls such as:

- PHY control: this block performs PHY signalling such as ranging, channel quality measurement/feedback (Channel Quality Indicator (CQI), and H-ARQ ACK or NACK signalling.

- Control signalling: this block generates resource allocation messages such as Advanced Medium Access Protocol (A-MAP) as well as specific control signalling messages.

- Sleep mode management: this block handles sleep mode operation and generates management messages related to sleep operation and may communicate with the scheduling and resource multiplexing block in order to operate properly according to sleep period.

- Quality-of-Service (QoS): this block handles QoS management based on QoS input parameters from connection management function for each connection.

- Scheduling and resource multiplexing: this block schedules and multiplexes packets based on properties of connections.

The data plane includes functional blocks such as:

- Fragmentation/packing: This block performs fragmentation or packing of MAC Service Data Units (MSDU) based on input from the scheduling and resource multiplexing block.

- Automatic Repeat Request (ARQ): This block performs MAC ARQ function. For ARQ-enabled connections, a logical ARQ block is generated from fragmented or packed MSDUs of the same flow and sequentially numbered.

- MAC Protocol Data Unit (PDU) formation: This block constructs MAC PDU (MPDU) so that ABS/AMS can transmit user traffic or management messages into PHY channels.

The RIT protocol structure is similar to that specified in {1} with some additional functional blocks for new features including the following:

- Self organization and self-optimization functions: This block handles the procedures to request AMS to report measurements for self-configuration and self-optimization.

- Multi-carrier functions: Control and operation of a number of contiguous or non-contiguous RF carriers where the RF carriers can be assigned to unicast and/or multicast and broadcast services. The overlapped guard sub-carriers of contiguous carriers are aligned in order to be used for data transmission. A single MAC instantiation is used to control physical layer spanning over multiple frequency channels.

A generalization of the protocol structure to multi-carrier support using a single MAC instantiation is shown in Figure 5-3. The load balancing functions and RF carrier mapping and control are performed via radio resource control and management functional group.

The carriers utilized in a multi-carrier system, from perspective of an AMS can be divided into two categories:

- A primary RF carrier is the carrier for AMS to complete network entry and is used by the ABS and the AMS to exchange traffic and full PHY/MAC control information.

- A secondary RF carrier is an additional carrier which the ABS may use for only traffic for AMSs capable of multi-carrier support.

Based on the primary and/or secondary usage and target services, the carriers of a multi-carrier system may be configured differently as follows:

- Fully configured carrier: A standalone carrier for which all control channels including synchronization,   broadcast, multicast and unicast control signalling are configured. Fully configured carrier supports both single carrier AMS and multi-carrier AMS.

- Partially configured carrier: A carrier configured for Downlink (DL) only transmission in TDD or a DL carrier without paired Uplink (UL) carrier in FDD mode. This supplementary carrier is used only in conjunction with a fully configured carrier.

Two multi-carrier operation modes are identified as follows:

- Multi-carrier aggregation: The AMS maintains the physical layer connection and monitors the control signalling on the primary carrier while processing data on the secondary carrier.

- Multi-carrier switching: The AMS switches the physical layer connection from the primary carrier to the secondary carrier that is one of partially configured carriers.

Figure 5-3

RIT Multicarrier Protocol Stack and Frame Structure {4}

Multi-radio coexistence functions: Protocols for the multi-radio coexistence where the AMS generates management messages to report the information about its co-located radio activities obtained from inter-radio interface and the ABS responds with the corresponding messages to support multi-radio coexistence operation.

## 5.2	AMS State Diagram

A mobile state diagram (i.e., a set of states and procedures between which the AMS transits when operating in the system to receive and transmit data) for the Reference System (See {4} for definition of the Reference System) based on common understanding of its behavior can be established as follows (see Figure 5-4):

- Initialization state: A state where an AMS without any connection performs cell selection by scanning and synchronizing to an ABS preamble and acquires system configuration information through superframe header.

- Access state: A state where the AMS performs network entry to the selected ABS. The AMS performs the initial ranging process in order to obtain UL synchronization. Then the AMS performs basic capability negotiation with the ABS. The AMS later performs the authentication and authorization procedure. Next, the AMS performs the registration process. The AMS receives specific user identification as part of Access State procedures. The IP address assignment may follow using appropriate procedures.

- Connected state: A state consisting of the following modes: 1) Sleep Mode, 2) Active Mode, and 3) Scanning Mode. During Connected State, the AMS maintains at least one transport connection and two management connections as established during Access State, while the AMS and ABS may establish additional transport connections. In addition, in order to reduce power consumption of the AMS, the AMS or ABS can request a transition to sleep mode. Also, the AMS can scan neighbouring ABSs to reselect a cell which provides more robust and reliable services.

- Idle state: A state comprising two separate modes, Paging Available mode and Paging Unavailable mode. During Idle State, the AMS may attempt power saving by switching between Paging Available mode and Paging Unavailable mode. In the Paging Available mode, the AMS may be paged by the ABS. If the AMS is paged, it transitions to the Access State for its network re-entry. The AMS performs location update procedure during Idle State.

FIGURE 5-4

AMS State Diagram {4}

The AMS state diagram for the RIT is similar to that of the Reference System {4} with the exception of the initialization state that has been simplified to reduce the scan latency and to enable fast cell selection or reselection. The location of the essential system configuration information is fixed so that upon successful DL synchronization, the essential system configuration information can be acquired, this would enable the AMS to make decision for attachment to the ABS without acquiring and decoding MAC management messages and waiting for the acquisition of the system parameters, resulting in power saving in the AMS due to shortening and simplification of the initialization procedure. Although both normal and fast network re-entry processes are shown as transition from the Idle State to the Access State in Figure 5-4, there are differences that differentiate the two processes. The network re-entry is similar to network entry, except it may be shortened by the Target ABS (T-ABS) possession of AMS information obtained from paging controller or other network entity over the backbone.

## 5.3	Overview of RIT Physical Layer

### 5.3.1	Multiple Access Schemes

The proposed RIT employs Orthogonal Frequency Division Multiple Access (OFDMA) as the multiple-access scheme in both DL and UL. It further supports both TDD and FDD duplex schemes including H-FDD operation of the AMSs in FDD networks. Frame structure attributes and baseband processing are common for both duplex schemes (see Table 5-1).

Table 5-1

RIT OFDMA Parameters

| Nominal channel bandwidth (MHz) |  |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling factor |  |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling frequency (MHz) |  |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT size |  |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-carrier spacing (kHz) |  |  | 10.94 | 7.81 | 9.76 | 10.94 | 10.94 |
| Useful symbol time Tu (µs) |  |  | 91.429 | 128 | 102.4 | 91.429 | 91.429 |
| CP Tg=1/8 Tu | Symbol time Ts (µs) |  | 102.857 | 144 | 115.2 | 102.857 | 102.857 |
|  | FDD | Number of OFDM symbols per 5ms frame | 48 | 34 | 43 | 48 | 48 |
|  |  | Idle time (µs) | 62.857 | 104 | 46.40 | 62.857 | 62.857 |
|  | TDD | Number of OFDM symbols per 5ms frame | 47 | 33 | 42 | 47 | 47 |
|  |  | TTG + RTG (µs) | 165.714 | 248 | 161.6 | 165.714 | 165.714 |
| CP Tg=1/16 Tu | Symbol time Ts (µs) |  | 97.143 | 136 | 108.8 | 97.143 | 97.143 |
|  | FDD | Number of OFDM symbols per 5ms frame | 51 | 36 | 45 | 51 | 51 |
|  |  | Idle time (µs) | 45.71 | 104 | 104 | 45.71 | 45.71 |
|  | TDD | Number of OFDM symbols per 5ms frame | 50 | 35 | 44 | 50 | 50 |
|  |  | TTG + RTG (µs) | 142.853 | 240 | 212.8 | 142.853 | 142.853 |
| CP Tg=1/4 Tu | Symbol Time Ts (µs) |  | 114.286 | 160 | 128 | 114.286 | 114.286 |
|  | FDD | Number of OFDM symbols per 5ms frame | 43 | 31 | 39 | 43 | 43 |
|  |  | Idle time (µs) | 85.694 | 40 | 8 | 85.694 | 85.694 |
|  | TDD | Number of OFDM symbols per 5ms frame | 42 | 30 | 37 | 42 | 42 |
|  |  | TTG + RTG (µs) | 199.98 | 200 | 264 | 199.98 | 199.98 |

Tone dropping at both edges of the frequency band based on 10 and 20 MHz systems can be used to support other bandwidths.

### 5.3.2	Frame structure

A superframe is a collection of consecutive equally-sized radio frames whose beginning is marked with a superframe header. The superframe header carries short-term and long-term system configuration information.

In order to decrease the air-link access latency, the radio frames are further divided into a number of subframes where each subframe comprises of an integer number of OFDMA symbols. The transmission time interval is defined as the transmission latency over the air-link and is equal to a multiple of subframe length (default is one subframe). There are four types of subframes: 1) type-1 subframe, which consists of six OFDMA symbols, 2) type-2 subframe, which consists of seven OFDMA symbols, 3) type-3 subframe which consists of five OFDMA symbols, and 4) type-4 subframe, which consists of nine OFDMA symbols.  In the basic frame structure shown in Figure 5-5, superframe length is 20 ms (comprised of four radio frames), radio frame size is 5 ms (comprised of eight subframes), and subframe length is 0.617 ms. The use of subframe concept with the latter parameter set would reduce the one-way air-link access latency to less than 10 ms {4}.

The concept of time zones applies to both TDD and FDD systems. These time zones are time-division multiplexed across time domain in the DL to support both AMSs and R1 MSs. For UL transmissions both time and frequency-division multiplex approaches are supported for multiplexing of legacy and new terminals. The non-backward compatible improvements and features are restricted to the new zones. All backward compatible features and functions are used in the legacy zones. In the absence of any legacy system, the legacy zones will disappear and the entire frame will be allocated to the new zones.

Figure 5-5

Basic Frame Structure {4}

The legacy and new radio frames are offset by a fixed number of subframes to accommodate new features such as preambles, superframe header (system configuration information), and control channels {4}.

Multiple RF carriers can be accommodated with the same frame structure that is used for single carrier operation. All RF carriers are time aligned at the frame, subframe, and symbol level (see Figure 5-5). Alternative frame structures for CP = 1/16 and CP = 1/4 are used that incorporate different number of OFDMA symbols per subframe or different number of subframes per frame {4}.

### 5.3.3	Physical and Logical Resource Units (LRU)

A physical resource unit is the basic physical unit for resource allocation that comprises of Psc contiguous subcarriers by Nsym contiguous OFDMA symbols. Psc is 18 subcarriers and Nsym is 6, 7, 5 and 9 OFDMA symbols for type-1, type-2, type-3 and type-4 subframes. A logical resource unit is the basic logical unit for distributed and localized resource allocations. A logical resource unit comprises of PscNsym subcarriers.

Distributed resource units are used to achieve frequency diversity gain. A Distributed Resource Unit (DRU) contains a group of subcarriers which are distributed within a frequency partition. The size of the distributed resource units is equal to that of physical resource unit. Localized resource units can be used to achieve frequency-selective scheduling gain. A localized resource unit comprises of a group of subcarriers which are contiguous across frequency domain. The size of the localized resource units is equal to that of the physical resource units. To form distributed and localized resource units, the subcarriers over an OFDMA symbol are partitioned into guard and used subcarriers. The DC subcarrier is not used. The used subcarriers are divided into physical resource units. Each physical resource unit contains pilot and data subcarriers. The number of used pilot and data subcarriers depends on MIMO mode, rank and number of multiplexed AMS, as well as the number of symbols within a subframe.

Figure 5-5-1

Illustration of the Downlink/Uplink Subcarrier to Resource Unit Mapping, 
where Pi denotes the i-th Frequency Partition

A multi-cell resource mapping is applied to the physical resource units in the groups of physical resource units. Direct mapping is exclusively applied to localized allocations. The permuted physical resource units are distributed in frequency partitions. Each frequency partition is divided into localized and/or distributed resources. A cell-specific resource mapping is performed where the localized and distributed groups are mapped into logical resources by direct mapping of localized resource units and by subcarrier permutation of distributed resource units. The size of the distributed or localized resources is flexibly configured per sector. Adjacent sectors are not required to have the same configuration of localized and distributed resources.

In the UL, the subframes are divided into a number of frequency partitions, where each partition consists of a set of physical resource units over the available number of OFDMA symbols in the subframe. Each frequency partition can include localized and/or distributed physical resource units. The UL resource partitioning and mapping is similar to that of DL.

The UL distributed units comprise of a group of subcarriers which are distributed within a frequency partition. The size of distributed unit is equal to logical resource units. The minimum unit for constructing a distributed resource unit is a tile. For type-1 subframe, the UL tile sizes are 6 subcarriers by 6 OFDMA symbols. The tile permutation defined for the UL distributed resources spreads the tiles over a frequency partition.

- 5.3.4	Modulation and Coding

Figure 5-6 shows the channel coding and modulation procedures. A Cyclic Redundancy Check (CRC) is appended to a burst (i.e., a physical layer data unit) before it is further processed by burst partition. The 16-bit burst CRC is calculated based on all the bits in the burst. When the burst size including burst CRC exceeds the maximum FEC block size, the burst is partitioned into KFB FEC blocks, each of which is encoded separately. If a burst is partitioned into more than one Forward Error Correction (FEC) blocks, a FEC block CRC is appended to each FEC block before the FEC encoding. The FEC block CRC of an FEC block is calculated based on all the bits in that FEC block. Each partitioned FEC block including 16-bit FEC block CRC has the same length. The maximum FEC block size is 4800 bits. Concatenation rules are based on the number of information bits and do not depend on the structure of the resource allocation (number of logical resource units and their size).

Figure 5-6

Chanel Coding Procedures

The RIT uses Convolutional Turbo Code (CTC) of rate 1/3 defined in the IEEE Std 802.16-2009 for data bursts. The structure of the IEEE Std 802.16-2009 CTC interleaver is maintained. The FEC encoder block depicted in Figure 5-6 includes the sub-block interleavers. The structure of the IEEE Std 802.16-2009 sub-block interleaver is maintained.

Bit selection and repetition are used in the RIT to achieve rate matching. Bit selection adapts the number of coded bits to the size of the resource allocation (in QAM symbols) which may vary depending on the logical resource units and subframe type. The total subcarriers in the allocated logical resource units are segmented to each FEC block. The mother-code bits, the total number of information and parity bits generated by FEC encoder, are considered as a maximum size of circular buffer. In case that the size of the circular buffer Nbuffer is smaller than the number of mother-code bits, the first Nbuffer bits of mother-code bits are considered as selected bits. A repetition is performed when the number of transmitted bits is larger than the number of selected bits. The selection of coded bits is done cyclically over the buffer.

Modulation constellations of QPSK, 16QAM, and 64QAM are supported as defined in IEEE Std 802.16-2009. The mapping of bits to the constellation point depends on the Constellation-Rearrangement (CoRe) version used for HARQ re-transmission as described in Section 11.13 of {4} and depends on the MIMO stream. The QAM symbols are mapped to the input of the MIMO encoder. Only the burst sizes NDB listed in Table 5-2 are supported in the physical layer. The sizes include the addition of CRC (per burst and per FEC block) when application. Other sizes require padding to the next burst size. The code rate and modulation depend on the burst size and the resource allocation.

Table 5-2

Supported Burst Sizes

| index | NDB (byte) | KFB | index | NDB (byte) | KFB | index | NDB (byte) | KFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1 | 23 | 90 | 1 | 45 | 1200 | 2 |
| 2 | 8 | 1 | 24 | 100 | 1 | 46 | 1416 | 3 |
| 3 | 9 | 1 | 25 | 114 | 1 | 47 | 1584 | 3 |
| 4 | 10 | 1 | 26 | 128 | 1 | 48 | 1800 | 3 |
| 5 | 11 | 1 | 27 | 145 | 1 | 49 | 1888 | 4 |
| 6 | 12 | 1 | 28 | 164 | 1 | 50 | 2112 | 4 |
| 7 | 13 | 1 | 29 | 181 | 1 | 51 | 2400 | 4 |
| 8 | 15 | 1 | 30 | 205 | 1 | 52 | 2640 | 5 |
| 9 | 17 | 1 | 31 | 233 | 1 | 53 | 3000 | 5 |
| 10 | 19 | 1 | 32 | 262 | 1 | 54 | 3600 | 6 |
| 11 | 22 | 1 | 33 | 291 | 1 | 55 | 4200 | 7 |
| 12 | 25 | 1 | 34 | 328 | 1 | 56 | 4800 | 8 |
| 13 | 27 | 1 | 35 | 368 | 1 | 57 | 5400 | 9 |
| 14 | 31 | 1 | 36 | 416 | 1 | 58 | 6000 | 10 |
| 15 | 36 | 1 | 37 | 472 | 1 | 59 | 6600 | 11 |
| 16 | 40 | 1 | 38 | 528 | 1 | 60 | 7200 | 12 |
| 17 | 44 | 1 | 39 | 600 | 1 | 61 | 7800 | 13 |
| 18 | 50 | 1 | 40 | 656 | 2 | 62 | 8400 | 14 |
| 19 | 57 | 1 | 41 | 736 | 2 | 63 | 9600 | 16 |
| 20 | 64 | 1 | 42 | 832 | 2 | 64 | 10800 | 18 |
| 21 | 71 | 1 | 43 | 944 | 2 | 65 | 12000 | 20 |
| 22 | 80 | 1 | 44 | 1056 | 2 | 66 | 14400 | 24 |

Incremental Redundancy Hybrid-ARQ (HARQ IR) is used in the RIT by determining the starting position of the bit selection for HARQ retransmissions. Chase Combining is supported and treated as a special case of IR. The 2-bit Sub-Packet Identifier (SPID) is used to indicate the starting position. Constellation Re-arrangement (CoRe) is supported by the RIT.   The CoRe can be expressed by a bit-level interleaver within a tone. Two CoRe versions are supported. The resource allocation and transmission formats in each retransmission in DL can be adaptive according to control signalling. The resource allocation in each retransmission in UL can be fixed or adaptive according to control signalling. In HARQ re-transmissions, the bits or symbols can be transmitted in a different order to exploit the frequency diversity of the channel.

For HARQ retransmission, the mapping of bits or modulated symbols to spatial streams may be applied to exploit spatial diversity with given mapping pattern, depending on the type of HARQ IR. In this case, the predefined set of mapping patterns should be known to both transmitter and receiver. In DL HARQ, the ABS may transmit coded bits exceeding current available soft buffer capacity. The RIT supports a basic ACK/NACK channel to transmit 1-bit feedback.

### 5.3.5	Pilot structure

Transmission of pilot subcarriers in DL is necessary to allow channel estimation, channel quality measurement (e.g., CQI), frequency offset estimation, etc. The RIT supports both common and dedicated pilot structures. The common pilots can be used by all AMSs. Dedicated pilots can be used with both localized and distributed allocations. The dedicated pilots are associated with a specific resource allocation and can be only used by the AMSs assigned to the specific resource allocation; therefore, they can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to eight transmission streams and there is a unified design for common and dedicated pilots in the type-1 subframe (see Figure 5-7). For the type 2 and type 3 subframes, one of OFDMA symbols is deleted or repeated. For the pilot structure of 1 and 2 streams, to overcome the effects of pilot interference among the neighboring sectors or ABSs, an interlaced pilot structure is utilized by cyclically shifting the base pilot pattern such that the pilots of neighboring cells do not overlap.

The UL pilots are dedicated to localized and distributed resource units and are precoded using the same precoding as the data subcarriers, if necessary. The UL pilot structure is defined for up to 4 spatial streams with orthogonal patterns. The PscNsym UL resource units use the same pilot patterns as the DL counterpart for up to 4 spatial streams. For the type-1 subframe, the pilot pattern for 6x6 tile structure is different as shown in Figure 5-7.

FIGURE 5-7

DL/UL Pilot Structures for 1, 2, and 4 Streams in type-1 subframe

### 5.3.6	Control channels

#### 5.3.6.1	DL control channels

The superframe header carries essential system parameters and configuration information. The content of superframe header is divided into two segments; i.e., primary and secondary superframe headers. The information transmitted in secondary superframe header is further divided into different sub-packets. The primary superframe header is transmitted every superframe, whereas the secondary superframe header is transmitted over one or more superframes. The primary and secondary superframe headers are located in the first subframe within a superframe and are time-division-multiplexed with the advanced preamble. The superframe header occupies narrower bandwidth relative to the system bandwidth (i.e., 5 MHz bandwidth). The primary superframe header is transmitted using predetermined modulation and coding scheme. The secondary superframe header is transmitted using predetermined modulation scheme while its repetition coding factor is signaled in the primary superframe header. The primary and secondary superframe headers are transmitted using two spatial streams and space-frequency block coding to improve coverage and reliability. The AMS is not required to know the antenna configuration prior to decoding the primary superframe header {4}. The information transmitted in the secondary superframe header is divided into different sub-packets. The secondary superframe header Sub-Packet 1 (SP1) includes information needed for network re-entry. The secondary superframe header Sub-Packet 2 (SP2) contains information for initial network entry. The secondary superframe header Sub-Packet 3 (SP3) contains remaining system information for maintaining communication with the ABS.

The advanced MAP (i.e., unicast control information) consists of both user-specific and non-user-specific control information. Non-user-specific control information includes information that is not dedicated to a specific user or a specific group of users. It contains information required to decode user-specific control signaling. Non-user-specific control information that is not carried in the superframe header may be included in this category.

User specific control information consists of information intended for one or more users. It includes scheduling assignment, power control information, and HARQ feedback. Resources can be allocated persistently to the AMSs. The periodicity of the allocation is configurable. Group control information is used to allocate resources and/or configure resources to one or multiple AMSs within a user group. Each group is associated with a set of resources. Voice over IP (VoIP) is an example of the class of services that can take advantage of group messages. Within a subframe, control and data channels are frequency-division-multiplexed. Both control and data channels are transmitted on logical resource units that span over all OFDMA symbols within a subframe {4}.

Each DL subframe contains a control region including both non-user-specific and user-specific control information. All advanced MAPs share a physical time-frequency region called A-MAP region. The control regions are located in every subframe. The corresponding UL allocations occurs L subframes later, where L is determined by A-MAP relevance. Coding rate of the control blocks are known to the AMS by group size indication in non-user-specific control information in order to reduce the complexity of blind detection by the AMS.

An Advanced MAP (A-MAP) allocation Information Element (IE) is defined as the basic element of unicast service control.  A unicast control IE may be addressed to one user using a unicast identifier or to multiple users using a multicast/broadcast identifier.  The identifier is masked with CRC in the advanced MAP allocation information element. It may contain information related to resource allocation, HARQ, MIMO transmission mode etc. Each unicast control information element is coded separately. Note that this method is different from the legacy system control mechanism where the information elements of all users are jointly coded. Non-user-specific control information is encoded separately from the user-specific control information. The transmission format of non-user-specific control information is predetermined. In the DL subframes, each frequency partition may contain an A-MAP region. An A-MAP region occupies the first few distributed resource units in a frequency partition. The structure of an A-MAP region is illustrated in Figure 5-8. The resource occupied by each A-MAP physical channel may vary depending on the system configuration and scheduler operation. There are different types of A-MAPs as follows:

- Assignment A-MAP (A-A-MAP) contains resource assignment information which is categorized into multiple types of resource assignment IEs. Each assignment A-MAP IE is coded separately and carries information for one or a group of users. The size of the assignment A-MAP is indicated by non-user-specific A-MAP.

- HARQ Feedback A-MAP (HF-A-MAP) contains HARQ ACK/NACK information for UL data transmission.

- Power Control A-MAP includes fast power control command to AMSs.

There are different A-A-MAP IE types that distinguish between DL/UL, persistent/non-persistent, single user/group resource allocation, basic/extended IE scenarios.

FIGURE 5-8

A-MAP Location and Structure (Example)

- 5.3.6.2	UL Control Channels

The UL control channels carry various types of control information to support air interface procedures. The information carried in the UL control channels is classified as shown in Table 5-3.

TABLE 5-3

UL Control Channels

| UL Control Channel | Description |
| --- | --- |
| MIMO Feedback | MIMO feedback provides wideband and/or narrowband spatial characteristics of the channel that are required for MIMO operation. The MIMO mode, precoder matrix index, rank adaptation information, channel covariance matrix elements, and channel sounding are examples of MIMO feedback information. |
| HARQ Feedback | HARQ feedback (ACK/NACK) is used to acknowledge DL data transmissions. The UL HARQ feedback channel starts at a predetermined offset with respect to the corresponding DL transmission. The HARQ feedback channel is frequency-division-multiplexed with other control and data channels. |
| Bandwidth Request | Bandwidth requests are used to indicate the amount of bandwidth required by an AMS and are transmitted through indicators or messages. Contention or non-contention based random access is used to transmit bandwidth request information. A five-step regular procedure or an optional three-step quick access procedure is utilized. |
| Channel Quality Indicators | Channel quality feedback provides information about channel conditions as seen by the AMS. This information is used by the ABS for link adaptation, resource allocation, power control, etc. There are two types of UL fast feedback control channels: a) primary and b) secondary fast feedback channels. The primary fast feedback channel provides wideband feedback information including channel quality and MIMO feedback and best-1 narrowband CQI and MIMO feedback information. The secondary fast feedback control channel carries narrowband CQI and MIMO feedback information. |
| UL Sounding Channel | The sounding channel is used by a user terminal to transmit sounding reference signals to enable the ABS to measure UL channel conditions. The sounding channel occupies either specific UL sub-bands or the entire bandwidth over an OFDMA symbol. |
| Ranging Channel | The ranging channel is used for UL synchronization. The ranging channel can be further classified into ranging for non-synchronized and synchronized AMSs. A random access procedure, which can be contention or non-contention based is used for ranging. The contention-based random access is used for initial ranging, periodic ranging, and handover. The non-contention based random access is used for handover. The ranging channel for non-synchronized AMSs is frequency-division multiplexed with other UL control and data channels. |
| Power Control | The ABS controls the transmit power per subframe and per user in the DL and UL. The DL advanced MAPs are power-controlled based on the terminal UL channel quality feedback. The per-pilot-subcarrier and per-data-subcarrier power can jointly be adjusted for adaptive DL power control. The UL power control is supported to compensate the path loss, shadowing, fast fading and implementation loss as well as to mitigate inter-cell and intra-cell interference levels. The UL power control includes open-loop and closed-loop power control mechanisms. |

- 5.3.7	Advanced preambles

The RIT utilizes a new hierarchical structure for the DL synchronization where two sets of preambles at superframe and frame intervals are transmitted (Figure 5-9). The first set of preamble sequences mark the beginning of the superframe and are common to a group of sectors or cells. The primary advanced preamble carries information about system bandwidth and carrier configuration. The primary advanced preamble has a fixed bandwidth of 5 MHz and can be used to facilitate location-based services. A frequency reuse of one is applied to the primary advanced preamble in frequency domain. The second set of advanced preamble sequences (secondary advanced preamble) is repeated every frame and spans the entire system bandwidth and carries the cell ID. A frequency reuse of three is used for this set of sequences to mitigate inter-cell interference. The secondary advanced preambles carry 768 distinct cell IDs. Secondary advanced preamble sequences are partitioned and each partition is dedicated to specific ABS type such as macro ABS, femto ABS, etc. The partition information is broadcast in the secondary superframe header.

Figure 5-9

Example structure of advanced preambles

- 5.3.8	Multi-Antenna techniques

- 5.3.8.1	DL MIMO structure

The RIT supports several advanced multi-antenna techniques including single and multi-user MIMO (spatial multiplexing and beamforming) as well as a number of transmit diversity schemes. In single-user MIMO (SU-MIMO) scheme only one user can be scheduled over one resource unit, while in multi-user MIMO (MU-MIMO), multiple users can be scheduled in one resource unit {10}. Vertical encoding (or single codeword) utilizes one encoder block (or layer), whereas horizontal encoding (or multi-codeword) uses multiple encoders (or multiple layers). Each of various SU-MIMO or MU-MIMO open-loop or closed-loop schemes is defined as a MIMO mode.

Figure 5-10

Illustration of downlink MIMO structure {4}

The DL MIMO transmitter structure is shown in Figure 5-10. The encoder block contains the channel encoder, interleaving, rate-matching, and modulating blocks per layer. A layer is defined as an encoding and modulation input path to the MIMO encoder. The resource mapping block maps the complex-valued modulation symbols to the corresponding time-frequency resources. The MIMO encoder block maps the layers onto the streams, which are further processed through the beamforming or the precoder block. The beamforming/precoding block maps the streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode. The OFDMA symbol construction block maps antenna-specific data to the OFDMA symbols. The feedback block contains feedback information such as CQI or Channel State Information (CSI) from the AMS. Table 5-4 contains information on various MIMO modes supported by the RIT.

Table 5-4

DL MIMO Modes

| Mode index | Description | MIMO encoding format | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | Open-loop SU-MIMO | SFBC | non-adaptive |
| Mode 1 | Open-loop SU-MIMO (spatial multiplexing) | Vertical encoding | non-adaptive |
| Mode 2 | Closed-loop SU-MIMO (spatial multiplexing) | Vertical encoding | adaptive |
| Mode 3 | Open-loop MU-MIMO (spatial multiplexing) | Horizontal encoding | non-adaptive |
| Mode 4 | Closed-loop MU-MIMO (spatial multiplexing) | Horizontal encoding | adaptive |
| Mode 5 | Open-loop SU-MIMO (TX diversity) | Conjugate Data Repetition | non-adaptive |

The minimum antenna configuration in the DL and UL is 2x2 and 1x2, respectively. For open-loop spatial multiplexing and closed-loop SU-MIMO, the number of streams is constrained to the minimum of number of transmit or receive antennas. For open-loop transmit diversity modes, the number of streams depends on the Space-Time Coding (STC) schemes that are used by the MIMO encoder. The MU-MIMO can support up to 2 streams with 2 transmit antennas and up to 4 streams for 4 and 8 transmit antennas. Table 5-5 summarized DL MIMO parameters for various MIMO modes.

Table 5-5

DL MIMO Parameters

|  | Number of transmit antennas Nt | STC rate per layer R | Number of streams Mt | Number of subcarriers NF | Number of layers L |
| --- | --- | --- | --- | --- | --- |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |
| MIMO mode 5 | 2 | 1/2 | 1 | 2 | 1 |
|  | 4 | 1/2 | 1 | 2 | 1 |
|  | 8 | 1/2 | 1 | 2 | 1 |

For SU-MIMO, vertical encoding is utilized, whereas for MU-MIMO horizontal encoding is employed at the ABS and only one stream is transmitted to each AMS. The stream to antenna mapping depends on the MIMO scheme that is used. CQI and rank feedback are transmitted to assist the ABS in rank adaptation, mode switching, and rate adaptation. For spatial multiplexing, the rank is defined as the number of streams to be used for each user. In FDD and TDD systems, unitary codebook based precoding is used for closed-loop SU-MIMO. An AMS may feedback some information to the ABS in closed-loop SU-MIMO such as rank, sub-band selection, CQI, Precoding Matrix Index (PMI), and long-term channel state information.

The MU-MIMO transmission with one stream per user is supported. The MU-MIMO schemes include 2 transmit antennas for up to 2 users, and 4 and 8 transmit antennas for up to 4 users. Both unitary and non-unitary MU-MIMO schemes are supported by the RIT. If the columns of the precoding matrix are orthogonal to each other, it is defined as unitary MU-MIMO. Otherwise, it is defined as non-unitary MU-MIMO {10}. Beamforming is enabled with this precoding mechanism.

The RIT has the capability to adapt between SU-MIMO and MU-MIMO in a predefined and flexible manner. Multi-ABS MIMO techniques are also supported for improving sector and cell-edge throughput using multi-ABS collaborative precoding, network coordinated beamforming, or inter-cell interference cancellation. Both open-loop and closed-loop multi-ABS MIMO techniques are under consideration.

- 5.3.8.2	UL MIMO

The block diagram of UL MIMO transmitter is illustrated in Figure 5-11. Note the similarities of MIMO baseband processing in the DL and UL.

Figure 5-11

Illustration UL MIMO Structure {4}

The ABS will schedule users to resource blocks and determines the Modulation and Coding Scheme (MCS) level and MIMO parameters (mode, rank, etc.). The supported antenna configurations include 1, 2, or 4 transmit antennas and more than two receive antennas. In the UL, the AMS measurements of the channel are based on DL reference signals (e.g., common pilots or a mid-amble). UL MIMO modes and parameters are contained in Table 5-6 and Table 5-7.

Table 5-6

UL MIMO Modes

| Mode index | Description | MIMO encoding format | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | Open-loop SU-MIMO | SFBC | non-adaptive |
| Mode 1 | Open-loop SU-MIMO (spatial multiplexing) | Vertical encoding | non-adaptive |
| Mode 2 | Closed-loop SU-MIMO (spatial multiplexing) | Vertical encoding | adaptive |
| Mode 3 | Open-loop Collaborative spatial Multiplexing (MU-MIMO) | Vertical encoding | non-adaptive |
| Mode 4 | Closed-loop Collaborative spatial Multiplexing (MU-MIMO) | Vertical encoding | adaptive |

Table 5-7

UL MIMO Parameters

|  | Number of transmit antennas Nt | STC rate per layer R | Number of streams Mt | Number of subcarriers NF | Number of layers L |
| --- | --- | --- | --- | --- | --- |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 1 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |

A number of antenna configurations and transmission rates are supported in UL open-loop SU-MIMO including 2 and 4 transmit antennas with rate 1 (i.e., transmit diversity mode), 2 and 4 transmit antennas with rates 2, 3, and 4 (i.e., spatial multiplexing). The supported UL transmit diversity modes include 2 and 4 transmit antenna schemes with rate 1 such as Space Frequency Block Coding (SFBC) and rank 1 precoder.

The multiplexing modes supported for open-loop single-user MIMO include 2 and 4 transmit antenna rate 2 schemes with and without precoding, 4 transmit antenna rate 3 schemes with precoding, 4 transmit antenna rate 4 scheme. In FDD and TDD systems, unitary codebook-based precoding is supported. In this mode, an AMS transmits a sounding reference signal in the UL to assist the UL scheduling and precoder selection in the ABS. The ABS signals the resource allocation, MCS, rank, preferred precoder index, and packet size to the AMS.

UL MU-MIMO enables multiple AMSs to be spatially multiplexed on the same radio resources. Both open-loop and closed-loop MU-MIMO are supported. The AMSs with single transmit antenna can operate in open-loop MU-MIMO mode. Unitary codebook-based precoding is supported for both TDD and FDD.

- 5.4	Overview of the RIT MAC Layer

The following sections briefly describe selected MAC features of the RIT.

- 5.4.1		MAC addressing

The RIT defines permanent and temporary addresses for an AMS that identify the user and its connections during operation.  The AMS is identified by a unique 48-bit identifier.  The AMS is further assigned the following temporary identifiers: 1) A station identifier during network entry (or network re-entry) that uniquely identifies the AMS within the cell, and 2) a flow identifier that uniquely identifies the management and transport connections with the AMS.

- 5.4.2 	Network entry

Network entry is the procedure through which an AMS detects a cellular network and establishes a connection with that network. The network entry has the following steps (see Figure 5-4):

- Synchronization with the ABS by acquiring the preambles.

- Acquiring necessary system information such as ABS and network service provider identifiers for initial network entry and cell selection.

- Initial ranging.

- Basic capability negotiation.

- Authentication/authorization and key exchange.

- registration and service flow setup.

Neighbor search is based on the same DL signals as initial network search except some information is provided via neighbor advertisement messages by the Serving-ABS (S-ABS).

- 5.4.3 	Connection management

Connections are identified by the combination of station identifier and flow identifier. Two types of connections (i.e., management and transport connections) are specified. Management connections are used to carry MAC management messages. Transport connections are used to carry user data including upper layer signalling messages and data-plane signalling such as ARQ feedback. Fragmentation and augmentation of the MAC SDUs are supported on transport connections.

Management connection is bidirectional and predefined values of flow identifier are reserved for unicast management connection(s). Management connections are automatically established after station identifier is assigned to an AMS during initial network entry. Transport connection, on the other hand, is unidirectional and is established with unique flow identifier assigned during service flow establishment procedure. Each active service flow is uniquely mapped to a transport connection.

- 5.4.4 	Quality of Service (QoS)

The MAC layer assigns a unidirectional flow of packets with specific QoS requirements with a service flow. A service flow is mapped to a transport connection with a flow identifier. The QoS parameter set is negotiated between the ABS and the AMS during the service flow setup/change procedure. The QoS parameters can be used to schedule traffic and allocate radio resource. The UL traffic may be regulated based on the QoS parameters. The RIT supports adaptation of service flow QoS parameters. The AMS and ABS negotiate possible QoS parameter sets during service flow setup.

- 5.4.5 	MAC management messages

To satisfy the latency requirements for network entry, handover, and state transition, the RIT supports fast and reliable transmission of MAC management connections. The transmission of MAC management messages using HARQ is supported where retransmissions can be triggered by an unsuccessful outcome from the HARQ entity in the transmitter. The MAC management message can be fragmented and only unsuccessful fragments are retransmitted if the HARQ of the message fragment fails. If MAC management message is fragmented into multiple MAC service data units, only unsuccessful fragments are retransmitted.

- 5.4.6 	MAC header

The RIT specifies a number of efficient MAC headers for various applications comprising of fewer fields with shorter size compared to IEEE Std 802.16-2009 generic MAC header. The new generic MAC header consists of Extended Header Indicator, Flow Identifier, and Payload Length fields. Other MAC header types include one-byte compact header that is used for connections with persistent allocation and group allocation, Fragmentation and packing extended header for transport connections, Fragmentation extended header for management connections, and Multiplexing extended header that is used when SDUs or SDU fragments from different connections are included in the same MPDU {4}.

- 5.4.7 	ARQ and HARQ functions

An ARQ block is generated from one or multiple MAC SDUs or MAC SDU fragment(s). ARQ blocks can be variable in size and are sequentially numbered. If the HARQ entity in the transmitter determines that the HARQ process was terminated with an unsuccessful outcome, the HARQ entity in the transmitter informs the ARQ entity in the transmitter about the failure of the HARQ burst. The ARQ entity in the transmitter can then initiate retransmission and re-segmentation of the appropriate ARQ blocks.

The RIT uses adaptive asynchronous and non-adaptive synchronous HARQ schemes in the DL and UL, respectively. The HARQ operation is relying on an N-process (multi-channel) stop-and-wait protocol. In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. A non-adaptive synchronous HARQ scheme is used in the UL where the parameters and the resource allocation for the retransmission are known a priori.

- 5.4.8 	Mobility management and handover

The RIT supports both network-controlled and AMS-controlled handover (HO). The AMS executes the handover as directed by the ABS or cancels the procedure through HO cancellation message. The AMS may also maintain communication with S-ABS while performing network re-entry at T- ABS as directed by S-ABS. Figure 5-12 illustrates the general HO procedure.

FIGURE 5-12

General Handover Procedure {4}

The handover procedure is divided into three stages 1) HO initialization, 2) HO preparation, and 3) HO execution. Upon completion of HO execution, the AMS is ready to perform network re-entry with the T-ABS. In addition, HO cancellation procedure is defined to allow AMS to cancel a HO procedure {4}.

The HO preparation is completed when the S-ABS informs the AMS of its handover decision via a HO control command. The control signalling includes an action time for the AMS to start network re-entry with the T-ABS and an indication whether the AMS should maintain communication with the S-ABS during network re-entry. If communication cannot be maintained between AMS and the S-ABS during network re-entry, the S-ABS stops allocating resources to the AMS for transmission in action time. If directed by S-ABS via HO control command, the AMS performs network re-entry with the T-ABS during action time while continuously communicating with the S-ABS. The AMS cannot exchange data with the T-ABS prior to completion of network re-entry.

- 5.4.9 	Power management

Sleep mode is a state in which an AMS performs pre-negotiated periods of absence from the S-ABS. Using the sleep mode, the AMS is provided with a series of alternative listening and sleep windows. The listening window is the time interval in which the AMS is available for transmit/receive of control signalling and data. The RIT has the capability of dynamically adjusting the duration of sleep and listening windows within a sleep cycle based on changing traffic patterns and HARQ operations. When the AMS is in active mode, sleep parameters are negotiated between the AMS and the ABS. The ABS instructs the AMS to enter sleep mode. MAC management messages can be used for sleep mode request/response {1}. The period of sleep cycle is measured in units of frames or superframes and is the sum of a sleep and listening windows. During the AMS listening window, the ABS may transmit the traffic indication message intended for one or multiple AMSs {4}.

Idle mode allows the AMS to become periodically available for DL broadcast traffic messaging such as paging message without registration with the network. The network assigns AMSs in the idle mode to a paging group during idle mode entry or location update. The AMS monitors the paging message during listening interval. The start of paging listening interval is calculated based on paging cycle and paging offset {4}. The S-ABS transmits the list of paging group identifiers at the predetermined location at the beginning of the paging available interval. The paging message contains identification of the AMSs to be notified of pending traffic or location update.

- 5.4.10 	Security

Security functions provide subscribers with privacy, authentication, and confidentiality across the RIT network. The MAC packet data units are encrypted over the connections between the AMS and the ABS. Figure 5-13 shows the functional blocks of the RIT security architecture.

Figure 5-13

Functional blocks of the RIT security architecture

The security architecture is divided into security management and encryption and integrity logical entities. The security management functions include overall security management and control, EAP encapsulation/de-encapsulation, Privacy Key Management (PKM) control, security association management, and identity/location privacy. The encryption and integrity protection entity functions include user data encryption and authentication, management message authentication, and message confidentiality protection {4}.

# 6	Description templates

## 6.1	Description template – Characteristics

This section provides itemized description of various features and functionalities of the radio access technology as a proposed RIT for IMT-Advanced. To maintain continuity and improve readability of the responses, a brief answer for each question has been provided. References have been made to {4} (which is also attached as Annex 2 in Part 3 for convenience).

| Item | Item to be described |
| --- | --- |
| 4.2.3.2.1 | Test environment(s) |
| 4.2.3.2.1.1 | What test environments (described in Report ITU-R M.2135) does this technology description template address? The proposed RIT addresses all four of the ITU-R M.2135 test environments; i.e., Indoor, Microcellular, Macro-cellular (base coverage urban), and High speed test environments. |
| 4.2.3.2.2 | Radio interface functional aspects |
| 4.2.3.2.2.1 | Multiple access schemes Which access scheme(s) does the proposal use: TDMA, FDMA, CDMA, OFDMA, IDMA, SDMA, hybrid, or another? Describe in detail the multiple access schemes employed with their main parameters. The proposed RIT uses scalable OFDMA for both DL and UL multiple access. It further supports SDMA (alternatively known as Multi-User MIMO) in the DL and UL, where two or more users share the time-frequency radio resources (see item 4.2.3.2.9 for more details). The OFDMA parameters depend on the transmission bandwidth and duplexing scheme and are provided in Table 4 in Section 11.3 of {4}. The OFDMA parameters of the proposed RIT can support various propagation environments including large delay spread and large Doppler spread channels. |
| 4.2.3.2.2.2 | Modulation scheme |
| 4.2.3.2.2.2.1 | What is the baseband modulation scheme? If both data modulation and spreading modulation are required, describe in detail. Describe the modulation scheme employed for data and control information. The data and MAC management messages are modulated using QPSK, 16QAM, or 64QAM. The L1/L2 control signals are modulated using BPSK or QPSK. The proposed RIT consists of the following physical channels in the DL and UL (a physical channel is defined as a set of dedicated physical resources with specific transmission format and physical layer processing): DL - Primary and secondary preambles - Primary and secondary superframe headers - Non-user specific and user-specific A-MAPs for transmission of DL control channels (resource allocation, power control, and HARQ ACK/NACK) - DL shared channel (transmission of user traffic) - Multicast and broadcast channel (transmission of Enhanced MBS (E-MBS) traffic) - Multicast and broadcast control channel (transmission of E-MBS control channels) UL - UL control channels (Primary and secondary CQI, Sounding, Initial and Periodic Ranging, Bandwidth request, HARQ ACK/NACK) - UL shared channel (transmission of user traffic) More details: DL control: See Section 11.7 of {4} UL control: See Section 11.9 of {4} DL/UL Traffic: See Section 11.5 and 11.6 of {4} Summary: TABLE 6-1 Summary of modulation and coding schemes Control/ Traffic Channels; Permissible MCS; Permissible MIMO Schemes; Description. DL; Control; P-SFH: QPSK TBCC 1/4 with repetition 6 S-SFH: QPSK TBCC 1/4 with repetition (Repetition factor determined by P-SFH) A-MAP: Two sets of MCSs can be used: 1) QPSK 1/2 and 1/4 2) QPSK 1/2 and 1/8 with TBCC 1/4 See Section 11.7 of {4} for more details; Open-loop SU-MIMO (Two Stream SFBC) See Section 11.8 of {4} for more details; The control channels in the DL include Primary and Secondary SFH, DL/UL A-MAPs including Assignment, Power Control, and HARQ feedback A-MAPs.. Traffic; See Section 11.5 of {4} for more details; Various Open-loop or Closed-loop SU-MIMO or MU-MIMO with and without precoding using 2x2, 4x2, 4x4, 8x2, 8x8 antenna configurations See Section 11.8 of {4} for more details; DL traffic channels for transmission of user data and MAC management messages. UL; Control; Primary Fast Feedback Channel: BPSK with semi-orthogonal sequences Secondary Fast Feedback Channel: QPSK HARQ feedback: BPSK with orthogonal sequence Ranging: Zadoff-Chu sequence Bandwidth request: Message: QPSK Preamble: BPSK Sounding: Golay sequence; RX diversity or CDD if it is transparent to ABS See Section 11.12 of {4} for more details; The control channels in the UL include Primary and Secondary CQI, HARQ feedback, Ranging and Bandwidth request, Sounding. Traffic; See Section 11.6 of {4} for more details; Various Open-loop or Closed-loop SU-MIMO or MU-MIMO with and without precoding See Section 11.12 of {4} for more details; UL Traffic channels for transmission of user data and MAC management messages. The proposed RIT supports convolutional code and convolutional turbo code as the mandatory forward error correcting schemes. See Section 11.3 of {4} for more details. The spreading modulation does not apply; however, the performance of adaptive modulation generally suffers from the power inefficiencies of multilevel modulation formats. This is due to the variations in bit reliabilities caused by the bit-mapping onto the signal constellation. To overcome this issue, a constellation-rearrangement scheme is utilized where signal constellation of modulation symbols between retransmissions is rearranged; i.e., the mapping of the bits onto the complex-valued symbols between successive HARQ retransmissions is changed, resulting in averaging bit reliabilities over several retransmissions and lower packet error rates. The mapping of bits to the constellation point depends on the constellation-rearrangement type used for HARQ re-transmissions and may also depend on the MIMO scheme. What is the symbol rate after modulation? Useful symbol time is 91.4 µs for 5, 10 and 20 MHz channel bandwidths. Assuming the cyclic prefix (CP) size of 1/16, the symbol rate after modulation can be calculated as 61.8 kbps per subcarrier. |
| 4.2.3.2.2.2.2 | PAPR What is the RF peak to average power ratio after baseband filtering (dB)? Describe the PAPR (peak-to-average power ratio) reduction algorithms if they are used in the proposed RIT. The PAPR is not uniquely derivable from the underlying protocol. A wide range of PAPR reduction algorithms are consistent with full interoperability in accordance with the specifications. Without any such reduction schemes, DL or UL PAPR equals 8.4 dB (99.9%) (note that OFDMA is used in the DL and UL of the proposed RIT.) In the proposed RIT, a hopping/permutation sequence is defined for the power optimized UL resource allocations that spreads the hopping units across frequency that helps reduce the UL PAPR (see Section 11.6.2.3 of {4}). However, any PAPR-reduction algorithm is implementation specific. |
| 4.2.3.2.2.3 | Error control coding scheme and interleaving |
| 4.2.3.2.2.3.1 | Provide details of error control coding scheme for both downlink and uplink? For example, - FEC or other schemes? - Unequal error protection? Explain the decoding mechanism employed. The RIT does not support any unequal error protection schemes. The RIT does not mandate use of any specific decoder. However, conventional CTC and CC decoders with soft decision decoding can be used. FIGURE 6-1 Channel coding procedures Description of Channel Coding in the Proposed RIT: The proposed RIT uses CTC of code rate 1/3 with specified CTC inner interleaver parameters for a variety of FEC block sizes. A burst CRC is appended to a burst before the burst is further processed by burst partition. The burst CRC is calculated based on all the bits in the burst. When the burst size including burst CRC exceeds the maximum FEC block size, the burst is partitioned into a number of smaller blocks, each of which is encoded separately. If a burst is partitioned into more than one FEC blocks, an FEC block CRC is appended to each FEC block before the FEC encoding. The FEC block CRC of an FEC block is calculated based on all the bits in that FEC block. The maximum FEC block size is 4800 bits. Concatenation rules are based on the number of information bits and do not depend on the structure of the resource allocation Bit selection and repetition are used in the proposed RIT to achieve rate matching. Bit selection adapts the number of coded bits to the size of the resource allocation (in QAM symbols) which may vary depending on the resource unit and subframe type. The total subcarriers in the allocated logical resource units are segmented to each FEC block. Mother code bits, the total number of information and parity bits generated by FEC encoder, are considered as a maximum size of circular buffer. In case that the size of the circular buffer is smaller than the number of mother code bits, the first bits (equal to the size of circular buffer) of mother code bits are considered as selected bits. Repetition is performed when the number of transmitted bits is larger than the number of selected bits. The selection of coded bits is done cyclically over the buffer. For traffic channels: Rate 1/3 Convolutional Turbo Coder (CTC), combined with rate matching based on puncturing/repetition to achieve a desired overall code rate. For control channels: Rate-1/4 tail-biting convolutional coding. Special block codes for some UL L1/L2 control signaling. For more details, see Section 11.13 of {4}. Decoding mechanism is implementation specific. |
| 4.2.3.2.2.3.2 | Describe the bit interleaving scheme for both uplink and downlink. Bit interleaving scheme is the same for both UL and DL. CTC inner interleaver and sub-block interleaver are used in CTC encoder. Bit interleaving is performed as part of the encoding/rate-matching process. The details of the convolutional interleaver can be found in Section 11.13 of {4}. |
| 4.2.3.2.3 | Describe channel tracking capabilities (e.g. channel tracking algorithm, pilot symbol configuration, etc.) to accommodate rapidly changing delay spread profile. DL The proposed RIT uses OFDMA for DL/UL access. Pilot subcarriers are used for channel estimation, measurements of channel quality indicators such as the SINR, frequency offset estimation, etc. To optimize the system performance in different propagation environments and applications, the proposed RIT supports both common and dedicated pilot structures. The common pilots can be used by all AMSs. Dedicated pilots can be used with both localized and distributed allocations. Pilot subcarriers that can be used only by a group of AMSs are a special case of common pilots. The dedicated pilots are associated with a specific resource allocation, can be only used by the AMSs allocated to said specific resource allocation, and therefore can be precoded or beam formed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to 8 transmission (TX) streams and there is a unified pilot pattern design for common and dedicated pilots. There is equal pilot density per TX stream, while there is not necessarily equal pilot density per OFDMA symbol of the DL subframe. Further, within the same subframe there is equal number of pilots for each physical resource unit of a data burst assigned to one AMS. UL Pilot subcarriers are used for channel estimation, measurement of channel quality indicators such as SINR, frequency offset and timing offset estimation, etc. The UL pilots are dedicated to localized and distributed resource units and are precoded using the same precoding as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 TX streams with orthogonal patterns. The DL 18x6 pilot patterns defined for the DL are used for UL 18x6 pilots, which include pilots up to 4 TX streams. For 6x6 UL tile, the UL pilot pattern, which supports up to 2 TX streams, is used for distributed resource units. For 4x6 tile the UL pilot pattern is used in legacy support zone for supporting up to 2 TX streams. The RIT also provides for UL channel sounding as a means for the ABS to determine UL channel response for the purpose of UL closed-loop MIMO transmission and UL scheduling. In TDD systems, the ABS can also use the estimated UL channel response to perform DL closed-loop transmission to improve system throughput, coverage and link reliability. In this case ABS can translate the measured UL channel response to an estimated DL channel response when the transmitter and receiver hardware of ABS and AMS are appropriately calibrated. The sounding signal occupies a single OFDMA symbol in the UL sub-frame. See Section 11.5.3 and 11.6.3 of {4] for details. As explained earlier the use of pilot sub-carriers (or alternatively known as reference signals) would allow the RIT to estimate the transmission channel and adapt the transmission parameters to maximize the data rates and to ensure robustness of the link under varying channel conditions. See Section 11.5.3 and 11.6.3 of {4] for more details on pilot structure. |
| 4.2.3.2.4 | Physical channel structure and multiplexing |
| 4.2.3.2.4.1 | What is the physical channel bit rate (Mbit/s) for supported bandwidths? i.e., the product of the modulation symbol rate (in symbols per second), bits per modulation symbol, and the number of streams supported by the antenna system. The physical channel bit rate depends on the modulation scheme and number of streams that are spatially multiplexed on a layer for SU-MIMO. The physical channel bit rate per stream per Resource Unit (RU) can be expressed as follows: Rstream = Nmod x NRU x 185 kbps/RU Where Nmod is the number of bits per modulation symbol for the applied modulation scheme (QPSK: 2, 16QAM: 4, 64QAM: 6) and NRU is the number of resource units in the aggregated frequency domain which depends on the channel bandwidth (e.g. NRU =24 for 5 MHz, NRU =48 for 10 MHz, and NRU =96 for 20 MHz). For channel bandwidths larger than 20 MHz (carrier aggregation), the channel bit rate will scale accordingly. The use of 64QAM modulation, CP size of 1/16, and 4 and 8 transmit antennas with 4 and 8 streams, respectively, results in following physical channel bit rates in the DL for both TDD and FDD: TABLE 6-2 Physical channel bit rates Nominal Channel Bandwidth (MHz); 5; 10; 20. Physical Channel Bit Rate (Mbit/s) for 4 streams; 107; 213; 427. Physical Channel Bit Rate (Mbit/s) For 8 streams; 213; 427; 854. The above physical channel bit rates can be further increased using multi-carrier scheme. In addition, the data transmission over guard subcarriers between the DL RF carriers would result in further higher physical channel bit rates (see Section 11.4 of {4}). |
| 4.2.3.2.4.2 | Layer 1 and Layer 2 overhead estimation. Describe how the RIT accounts for all layer 1 (PHY) and layer 2 (MAC) overhead and provide an accurate estimate that includes static and dynamic overheads. L1/L2 overhead includes: - Reference signals or pilots that are dispersed over each physical resource block that further depend on the MIMO mode and the number of spatial streams - Guard bands or the number of guard subcarriers that depend on the transmission bandwidth - L1/L2 control signals that are frequency division multiplexed with traffic over each subframe - Synchronization signals and superframe headers - The DL/UL or UL/DL switching times in TDD duplex mode and guard time (cyclic prefix) in both FDD and TDD. - MAC header overhead depends on the packet size and is typically small for full-buffer traffic The L1/L2 overhead estimation for the proposed RIT is as follows(for L1/L2 calculation details see Annex 1 of this document): TABLE 6-3 L1/L2 overhead estimation CP=1/8, BW=10 MHz, DL 2x2 MIMO; Minimum; Maximum. L1 overhead; 0.2931; 0.2931. Total overhead (L1/L2); 0.3370; 0.4239. CP=1/16, BW=20 MHz, DL 4x2 MIMO; Minimum; Maximum. L1 overhead; 0.2665; 0.2665. Total overhead (L1/L2); 0.2875; 0.3295. |
| 4.2.3.2.4.3 | Variable bit rate capabilities: Describe how the proposal supports different applications and services with various bit rate requirements. Variable bit rate is supported by the flexible resource allocation to meet the desired bit rates of different applications and service. The RIT supports QoS parameter negotiation during service setup procedure in order to support various application and service efficiently. The information is used to schedule and control traffic by adapting polling and granting mechanism based on predefined rule. For a given combination of modulation and coding scheme and number of spatial-multiplexing layers or MIMO mode, the data rate available to a user can be controlled by the scheduler through assigning different amount of radio resources for transmission of user traffic. In case of multiple services, the available radio resources along with the other transmission attributes are selected to support multiple services with various QoS requirements (including various data rate requirements). See Section 11.3 of {4} for more details on the supported burst sizes in the proposed RIT. |
| 4.2.3.2.4.4 | Variable payload capabilities: Describe how the RIT supports IP-based application layer protocols/services (e.g., VoIP, video-streaming, interactive gaming, etc.) with variable-size payloads. The proposed RIT is designed based on packet-switched protocols. Various payload sizes can be delivered over radio interface by supporting various PHY packet sizes which are controlled by resource assignment, MCS level and MIMO mode and rank. Furthermore, large-size IP packets can be fragmented into small fragments, while small-size IP packets can be packed together to send over the air-interface. The proposed algorithm can support multiple concurrent connections for various IP-based applications with different QoS requirements. |
| 4.2.3.2.4.5 | Signaling transmission scheme: Describe how transmission schemes are different for signaling/control from that of user data. Configuration: Traffic channels transmission parameters are configured using signaling carried on control channels and in some cases (in-band and higher-layer signaling) using signaling carried on traffic channels themselves. The configuration of control channels is by other control channels or is pre-defined in the RIT specification, depending on the hierarchy of the control channels. Resource Allocation: Control channels are categorized based on their contents, functionality and transmission requirements (robustness, transmission periodicity etc). Depending on their position in the control channel hierarchy, the resource allocations for the L1/L2 control channels may be defined by the RIT specification. Transmission Format: Control channels are typically processed with more robust transmission formats to ensure reliability and coverage. For the control channels, QPSK modulation and lower coding rates as well as open-loop SU-MIMO schemes are used. For the traffic channels, higher order modulation and coding (QPSK, 16QAM, 64QAM) as well as open-loop/closed loop SU-MIMO/MU-MIMO schemes with and without precoding are used. In addition, power boosting and link adaptation are used to improve the reliability of the traffic channels. TABLE 6-4 Transmission schemes for DL and UL control channels Control Channel; MCS; MIMO Scheme. Primary SFH; QPSK 1/24 (QPSK+1/4TBCC+6 repetitions); Open-loop TX diversity (Two stream SFBC). Secondary SFH; QPSK variable coding rate (QPSK + 1/4 TBCC + variable repetition factor which is configured by Primary SFH); Open-loop TX diversity (Two stream SFBC). A-MAP; Two sets of MCS can be used 1) QPSK 1/2 and 1/4 2) QPSK 1/2 and 1/8 with 1/4 TBCC; Open-loop TX diversity (Two stream SFBC). HARQ feedback A-MAP; QPSK or BPSK 1/8; Open-loop TX diversity (Two stream SFBC). Power control A-MAP; QPSK or BPSK 1/4; Open-loop TX diversity (Two stream SFBC). Non-user specific A-MAP; QPSK 1/12 (TBCC 1/4 + repetition); Open-loop TX diversity (Two stream SFBC). MIMO Midamble; Golay sequence; N/A. UL Primary CQI; BPSK with Block code (6 info bit coded into 36 tones); RX diversity or Cyclic Delay Diversity (CDD) if it is transparent to ABS.. UL Secondary CQI; QPSK with TBCC 1/5; RX diversity or CDD if it is transparent to ABS.. UL ACK/NACK; BPSK Every 2 bits of ACK/NACK bit is encoded into three 2x2 blocks (i.e. 6 subcarrier for each ACK/NACK bit); RX diversity or CDD if it is transparent to ABS.. UL BW-REQ; Message: QPSK Preamble: BPSK; RX diversity or CDD if it is transparent to ABS.. UL Initial Ranging; Zadoff-Chu sequences; N/A. UL Sounding; Golay sequences; N/A. TABLE 6-5 Transmission schemes for DL and UL data transmission Control Channel; MCS; MIMO Scheme. Unicast Data; See Section 11.7 of {4} for more details; Various Open-loop or Closed-loop SU-MIMO or MU-MIMO with and without precoding using 2x2, 4x2, 4x4, 8x2, 8x8 in the DL and 1x2, 2x4, 1x4, and 4x4 UL antenna configurations See Sections 11.8 and 11.12 of {4} for more details. Multicast Data; See Section 11.5 and 11.7 of {4} for more details; Various Open-loop SU-MIMO or MU-MIMO using 2x2, 4x2, 4x4, 8x2, 8x8 in the DL and 1x2, 2x4, 1x4, and 4x4 UL antenna configurations See Sections 11.8.4 of {4} for more details. |
| 4.2.3.2.5 | Mobility management (Handover) |
| 4.2.3.2.5.1 | Describe the handover mechanisms and procedures which are associated with - Inter-System handover - Intra-System handover - Intra-frequency and Inter-frequency - Within the RIT or between RITs within one SRIT (if applicable) Characterize the type of handover strategy or strategies (for example, MS or BS assisted handover, type of handover measurements). Handovers (HO) can be initiated by either the AMS or the ABS. In the latter case, Mobile-Assisted HO (MAHO) procedures are followed. The AMS acquires the network topology, measurement/reporting trigger conditions through either ABS broadcasts or dedicated signaling messages. The AMS measures HO trigger metrics to identify potential T-ABSs. For inter-system and inter-frequency handovers, neighbor cell measurements are performed during scanning intervals provided by the S-ABS unilaterally or at the request of the AMS. HO execution includes network re-entry procedure with the T-ABS. This process may be optimized by T-ABS possession of AMS context information obtained from S-ABS over the backbone network. If capable, the AMS may also maintain communication with S-ABS while performing network re-entry at T-ABS as directed by S-ABS. The handover procedures provide for seamless and lossless handovers to maintain the QoS of the traffic channels involved in the handover. Section 10.3 of {4} describes the handover procedures in detail. The following diagram illustrates the network reference model used for the proposed RIT. FIGURE 6-2 Network Reference Model (NRM) {4} The inter-Frequency Assignment (inter-FA), intra-FA handover scenarios are analyzed according to above reference model assuming an intra-Access Service Network (intra-ASN) handover scheme (i.e., the S-ABS and T-ABS belong to the same ASN) Assumptions: - Intra-ASN Handovers (the S-ABS and T-ABS belong to the same ASN) - Mobile-assisted Handover (predominant case) - Mobile-initiated Handover (worst-case compared to ABS-initiated) - Optimized Hard Handover (for intra-frequency, AMS is frame-synchronized with S-ABS and T-ABS (T-ABS), AMS contexts including security context, transferred to T-ABS by S-ABS over the backbone network) - All measurements are based on the synchronization channels. FIGURE 6-3 Network re-entry procedure with HO_Reentry_Mode set to 0 Messages depicted with dotted lines are transmitted only in certain HO scenarios. The dashed-line (optional) AAI_RNG-RSP carries time adjustment parameters, etc. {4}. Figure 6.3 refers to the case where the HO_Renentry_Mode=0. If HO_Reentry_Mode is set to 0, the S-ABS stops sending DL data and providing UL allocations to the AMS after expiration of the disconnect time included in the AAI_HO-CMD message or at HO cancellation, whichever occurs first. However, If HO_Reentry_Mode is set to 1, the S-ABS stops sending DL data and providing UL allocations to the AMS upon expiration of the disconnect time or after receiving HO completion confirmation from T-ABS, whichever occurs first. Summary of the Handover procedure: - Initiation - HO triggered at AMS, based on AMS measurements and S-ABS-defined trigger levels - HO request from AMS to S-ABS, containing T-ABS list with preferences and measurement reports - Preparation - S-ABS and T-ABS backbone pre-notification procedures - HO command from S-ABS to AMS with T-ABS details, Disconnect time (may allow communication during network re-entry), S-ABS disconnects at Disconnect time, transfers AMS context to T-ABS over backbone network - Execution - AMS acknowledges S-ABS HO command with selected T-ABS and confirmation/rejection of Disconnect time - Network re-entry - HO Ranging in T-ABS, optional for intra-FA - T-ABS notifies S-ABS of HO completion - Network Re-entry complete with established data path in T-ABS |
| 4.2.3.2.5.2 | What are the handover interruption times for: - Within the RIT (intra- and inter-frequency) Intra-frequency: 0 to 15 ms Inter-frequency: 5 to 35 ms - Between various RITs within a SRIT Not applicable. - Between the RIT and another IMT system. For HO of an R1 MS to/from an ABS from/to an R1 BS, HO interruption time is less than 50 ms. For HO of an AMS to/from an ABS from/to an R1 BS, HO interruption time is less than 50 ms. HO interruption time for HO between the RIT and IMT-2000 OFDMA TDD WMAN follows IMT-2000 OFDMA TDD WMAN HO procedures. Similar procedures can be applied to the FDD case. The HO latency analysis below considers the worst case of HO_Reentry_Mode=0, where the action time equals the disconnect time set by the S-ABS. The HO latency analysis follows the guideline in {8}, where the HO interruption time does not include the process of downlink synchronization and the completion of UL access procedures, if applicable. The following steps capture the HO procedure delay budget (Intra-RIT, Intra-FA): TABLE 6-6 Handover procedure delay budget Step; Procedure; Estimated Latency (ms). 1; The AMS initiates HO by sending an AAI-HO-REQ to the S-ABS.; 4 to 7 frames, 20 to 35ms. 2; The S-ABS processes AAI-HO-REQ and sends HO REQUEST to one or more T-ABS; 1 frames, 5 ms (HO-REQ from S-ABS to T-ABSs). 3; T-ABSs reply S-ABS with HO RESPONSE, which may include HO optimization related MAC pre-update information.; 2 frames, 10 ms (T-ABSs process and reply + HO-RSP from T-ABSs to S-ABS). 4; S-ABS responds to AMS with AAI-HO-CMD containing T-ABS list, Disconnect time; 1 frame, 5 ms. 5; AMS acknowledges S-ABS with AAI-HO-IND containing selected T-ABS and confirmation/rejection of Disconnect time (unsolicited UL grant); 1 frame, 5 ms. 6; At/After Action Time (=Disconnect Time), S-ABS transfer un-acknowledged data and new data, if any, to T-ABS for AMS data continuity at T-ABS; 0 to 2 frames, 0 to 10ms (R8 interface latency, see Section 4 of {4}). 7; AMS switches to T-ABS, acquires DL signal; 0 to 1 frame, 0 to 5 ms. 8; AMS reads UL-MAP for unsolicited UL grant for AMS to send RNG-REQ message and data; 2 frames, 10 ms. 9; AMS sends RNG-REQ to T-ABS; 1 frame, 5 ms. 10; T-ABS responds with RNG-RSP with necessary information for AMS to perform UL synchronization.; 2 frames, 10 ms. 11; AMS processes RNG-RSP; 1 frame, 5 ms. 12; If necessary, repeat steps 8 – 11 K-times Note: The maximum value of K is calculated based upon the number of times that steps 8 to 11 could be repeated before expiration of a timer specified by S-ABS.; K * 5 frames, 0 to 25K ms. 13; T-ABS and AMS continue data communication; 0. HO Interruption time (Using Seamless HO); summation of time required for steps 6 and 7 = 0 to 15 ms. Using seamless HO procedure, HO interruption time includes steps 6 and 7 in Table 6-6. Adding the times required for completion of steps 6 and 7results in a value of 0 to 15 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 10 ms. For intra-RIT/Inter-FA HO an additional step 7.1 is to be inserted between Step-7 and Step-8, which will be counted into HO interruption time. TABLE 6-7 Handover procedure, step 7.1 7.1; The AMS waits for HO ranging opportunity to perform UL synchronization with dedicated ranging code (assigned by TABS during HO preparation.) (after CDMA ranging, UL synchronization procedures are not counted into HO interruption time according to the definition); 1 to 4 frames = 5 to 20 ms (20ms is the worst case when no ranging opportunity is allocated for this HO instance. Most cases, T-ABS has knowledge of AMS capability and how fast it can switch RF, and therefore T-ABS can prepare the ranging opportunity right at the next frame, in which case it will be 5 ms). In this case (inter-FA, intra-RIT) , the HO interruption time is the summation of time required for steps 6, 7, and 7.1, which results in HO interruption time of 5 to 35 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 30 ms. |
| 4.2.3.2.6 | Radio resource management |
| 4.2.3.2.6.1 | Describe the radio resource management, support of, - centralized and/or distributed RRM The proposed RIT supports distributed Radio Resource Management (RRM) schemes (the ABSs manage the radio resources) such as those described below. - dynamic and flexible radio resource management The proposed RIT supports dynamic and flexible RRM schemes such as those described below. - efficient load balancing. The proposed RIT supports efficient load balancing through several means including multi-carrier, ABS switching, etc. The RIT is compatible with the radio resource management specified in Section 4 of {4} (See the network reference model illustrated in item 4.2.3.2.5.2).This RRM is based on a generic architecture. The RRM defines mechanisms and procedures to share radio resource related information between ABS and ASN-Gateway (ASN-GW). The RRM procedures allow different ABSs to communicate with each other or with a centralized RRM entity residing in the same or a different ASN to exchange information related to measurement and management of radio resources. Each ABS performs radio resource measurement locally based on a distributed RRM mechanism. It is also possible to deploy RRM in an ASN using ABSs with RRM function as well as a centralized RRM entity that does not reside in the ABS and collects and updates radio resource indicators such as choice of T-ABS, admission or rejection of service flows, etc., from several ABSs. The RRM procedures facilitate the following functions: - AMS admission control and connection admission control; i.e., whether the required radio resources are available at a candidate T-ABS prior to handover. - Service flow admission control; i.e., creation or modification of existing/additional service flows for an existing AMS in the network, selection of values for admitted and active QoS parameter sets for service flows. - Load balancing by managing and monitoring system load and use of counter-measures to enable the system back to normal loading condition. - Handover preparation and control for improvement/maintenance of overall performance indicators (for example, the RRM may assist in system load balancing by facilitating selection of the most suitable ABS during a handover. The RRM is composed of two functional entities; i.e., Radio Resource Agent (RRA) and Radio Resource Control (RRC). The RRA is a functional entity that resides in the ABS. Each ABS includes an RRA. It maintains a database of collected radio resource indicators. An RRA entity is responsible for assisting local radio resource management as well as communicating to the RRC to collect and measure radio resource indicators from the ABS and from a plurality of mobile terminals served by the ABS using MAC management procedures as specified in {1}. It also communicates RRM control information over the air-interface to the AMS as defined by {1}. An example of such RRM control information is a set of neighbor ABSs and their parameters. It further performs signaling with RRC for radio resource management functions as well as controlling the radio resources of the S-ABS, based on the local measurements and reports received by the ABS and information received from the RRC functional entity. The local resource control includes power control, monitoring the MAC and PHY functions, modifying the contents of the neighbor advertisement message, assisting the local service flow management function and policy management for service flow admission control, making determinations and conducting actions based on radio resource policy, assisting the local handover functions. The RRC functional entity may reside in ABS, in ASN-GW, or as a standalone server in an ASN and is responsible for collection of radio resource indicators from associated RRAs. The RRC can be collocated with RRA in the ABS. The RRC functional entity may communicate with other RRCs in neighboring ABSs which may be in the same or different ASN. The RRC may also reside in the ASN-GW and communicate to other RRAs across R6 reference point. When the RRC is located in the ASN, each RRA is associated with exactly one RRC. The RRC relay functional entity may reside in ASN-GW for the purpose of relaying RRM messages. The RRC relay cannot terminate RRM messages but it only relays these to the final destination RRC. Standard RRM procedures are required between RRA and RRC and between RRCs across network interfaces to ensure interoperability. These procedures are classified into two types; Information Reporting Procedures for delivery of ABS radio resource indicators from RRA to RRC and between RRCs, and Decision Support Procedures from RRC to RRA for communicating suggestions or hints of aggregated RRM status (e.g., in neighboring ABSs) for various purposes. The RRM primitives can be used either to report radio resource indicators (i.e., from RRA to RRC or between RRCs) or to communicate decision support information (i.e., from RRC to RRA). The former type of primitive is called information reporting primitive and the latter is called decision support primitive. The available radio resource information provided by the RRAs to RRC is used by RRC for load balancing. The RRC may interact with the handover controller to ensure load balance. |
| 4.2.3.2.6.2 | Inter-RIT interworking Describe the functional blocks and mechanisms for interworking (such as a network architecture model) between heterogeneous RITs within a SRIT, if supported. The proposed RIT comprises of a single radio interface and therefore this question does not apply. It must be noted that the proposed RIT does support inter-system HO through appropriate interworking functions. For more details, see Section 10.3.4 of {4}. |
| 4.2.3.2.6.3 | Connection/session management The mechanisms for connection/session management over the air-interface should be described. For example: - The support of multiple protocol states with fast and dynamic transitions. - The signaling schemes for allocating and releasing resources. Connection Management Connections are identified by the combination of AMS identifier and flow identifier. Two types of connections are used – management connections and transport connections. Management connections are bi-directional and used to carry MAC management messages. Transport connections are uni-directional and used to carry user data including upper layer signaling messages such as DHCP, etc. and data plane signaling such as ARQ feedback. Session Management A session is defined as the duration of time from the moment that an AMS performs the initial network entry and registers with the network and an exclusive AMS context is generated in the network until the AMS signs off the network and the AMS context is flushed out. During this time, the AMS may transit between different states (initialization, access, connected, and idle states) and may perform a number of network re-entries and re-register with the S-ABS upon transition from idle to connected state. The session/connection management in the proposed RIT can be better understood by reviewing the AMS state transition diagram below (for details, see Section 6 of {4}). There are 4 possible AMS states: Initialization State, Cell search and selection using scanning, DL synchronization, and acquisition of system information. Access State: Network entry, including ranging and UL synchronization, basic capability negotiation, authentication and authorization, registration with the ABS and service flow establishment Connected State: AMS in one of three Connected State modes: Sleep Mode, Active Mode or Scanning Mode. Additional transport connections may be established and Handovers may occur in this state. Idle State: AMS alternates between Idle State Paging Available Mode and Paging Unavailable Mode. - FIGURE 6-4 - MAC state diagram |
| 4.2.3.2.7 | Frame structure |
| 4.2.3.2.7.1 | Describe the frame structure for downlink and uplink by providing sufficient information such as: - frame length, Superframe length = 20 ms; Radio frame length = 5 ms; Subframe length = 0.617 ms (with 6 OFDMA symbols using CP=1/8 Tu with the channel bandwidth of 5, 10, or 20 MHz) The basic frame structure is illustrated in the following Figure 6-5. The number of subframes per frame depends on the channel bandwdth and the CP length. A subframe is assigned for either DL or UL transmission. There are four types of subframes: 1) type-1 subframe consists of six OFDMA symbols, 2) type-2 subframe consists of seven OFDMA symbols, 3) type-3 subframe which consists of five OFDMA symbols, and 4) type-4 subframe which consists of nine OFDMA symbols. This type is only applied to UL subframe for the 8.75MHz channel bandwidth. FIGURE 6-5 Basic frame structure The basic frame structure is applied to FDD and TDD duplexing schemes, including H-FDD AMS operation. - the number of time slots per frame, In basic frame structure, there are 8 subframes per radio frame (if CP=1/8 is used and if the system bandwidth is an integer multiple of 5 MHz). Other bandwidths or CP sizes contain different number of subframes per radio frame. See Section 11.4 of {4} for more information. - the number and position of switch points per frame for TDD There are two switching gaps in each 5ms radio frame for all allowed frame partitions. In each frame, the guard times or switching times (TTG and RTG) are inserted between the DL and UL switching points. For example, In TDD frame structure with D:U ratio of 5:3, corresponding to the nominal channel bandwidths of 5, 10, and 20 MHz with CP = 1/8, the TTG and RTG values are 105.714 μs and 60 μs, respectively (see Table 4 in Section 11.3 of {4} for other configurations). See figures and tables in Section 11.3 of {4} for more information. - guard time or the number of guard bits, The proposed RIT utilizes OFDMA as the multiple access scheme in the DL and UL. The proposed RIT supports three CP sizes of 1/4, 1/8, and 1/16 of the useful OFDM symbol size. See Section 11.3 of {4}. - user payload information per time slot, The user payload is transmitted in units of logical resource blocks (i.e., distributed or localized resource blocks of 18xNsym, where Nsym denotes the number of OFDMA symbols per subframe). The logical resource blocks are frequency division multiplexed with control blocks in the DL and UL. - control channel structure and multiplexing, The control information in the DL consists of resource allocation, HARQ ACK/NACK, and power control. These control information are transmitted in the form of control information elements that are multiplexed with user data as shown in the following figures. For more information see Section 11.5 of {4}. FIGURE 6-6 Example A-map region location in TDD with 4:4 subframe DL:UL ratio - FIGURE 6-7 - Structure of an A-MAP region - TABLE 6-8 - Mapping of information to DL control channels Information; Channel; Location. Synchronization information; Advanced Preamble (A-PREAMBLE): Primary Advanced Preamble (PA-PREAMBLE) and Secondary Advanced Preamble (SA-PREAMBLE); PA-Preamble is located at the first symbol of second frame in a superframe while SA-Preamble is located at the first symbol of remaining three frames. System configuration information; Primary Superframe Header (P-SFH) and Secondary Superframe Header (S-SFH); Inside SFH. Extended system parameters and system configuration information; Additional Broadcast Information on Traffic Channel; Outside SFH. Control and signaling for DL notifications; Additional Broadcast Information on Traffic Channel; Outside SFH. Control and signaling for traffic; Advanced MAP (A-MAP); Outside SFH. - TABLE 6-9 - Mapping of UL control information to UL control channels Information; Channel. Channel quality feedback; UL Fast Feedback Channel. UL Sounding Channel. MIMO feedback; UL Fast Feedback Channel. UL Sounding Channel UL Feedback Extended Header UL Feedback MAC Control Message. HARQ feedback; UL HARQ Feedback Channel. Synchronization; UL Ranging Channel. Bandwidth request; Bandwidth Request Channel. UL In-band Control Signaling. UL Fast Feedback Channel. E-MBS feedback; Optional quick access message. - power control bit rate. No specific power-control rate is specified. The proposed RIT supports (at maximum) one power-control command per subframe and assuming 8 subframes/frame, resulting in 1600 Hz maximum power-control rate. See Section 11.10 of {4} for more details. Details of the TDD and FDD frame structures are specified in Section 11.4 of {4}. |
| 4.2.3.2.8 | Spectrum capabilities and duplex technologies NOTE 1 – Parameters for both downlink and uplink should be described separately, if necessary. |
| 4.2.3.2.8.1 | Spectrum sharing and flexible spectrum use Does the RIT/SRIT support flexible spectrum use and/or spectrum sharing for the bands for IMT? Provide details. Yes, The proposed RIT supports flexible spectrum use and/or spectrum sharing requirements for IMT bands. The proposed RIT supports both FDD and TDD duplex schemes. The H-FDD terminals are supported in FDD networks, as well. The baseband processing is common (to the possible extent) for TDD and FDD duplex schemes. Flexible spectrum use is achieved through use of scalable OFDMA multiple access scheme in the DL and UL, tone dropping techniques in OFDMA, as well as use of one or multiple component RF carriers. Multiple component carriers can be aggregated to achieve up to 100 MHz of transmission bandwidth. The aggregated component carriers can be either contiguous or non-contiguous in the frequency domain. |
| 4.2.3.2.8.2 | Channel bandwidth scalability Describe how the proposal supports channel bandwidth scalability, including the supported bandwidths. The proposed RIT uses a scalable OFDMA multiple access scheme in the DL and UL. The scalable OFDMA can be adapted to different bandwidths (5 to 20 MHz in the proposed RIT) using different FFT/IFFT sizes, resulting in common baseband processing for all bandwidths. The sub-carrier spacing remains the same irrespective of the bandwidth (provided that the same over-sampling factor is used). Therefore, the number of used sub-carriers and the number of available sub-carriers vary with different bandwidth, but the PHY and MAC protocols and processing remain unchanged. Describe whether the proposed RIT supports extensions for scalable bandwidths wider than 40 MHz. Consider, for example: – The scalability of operating bandwidths. – The scalability using single and/or multiple RF carriers. Describe multiple contiguous (or non-contiguous) band aggregation capabilities, if any. Consider for example the aggregation of multiple channels to support higher user bit rates. The proposed RIT supports multi-carrier operation that allows operation in any bandwidth as wide as 100 MHz by aggregating contiguous and/or non-contiguous RF carriers. For each multi-carrier AMS, primary and secondary carriers are designated and configured with the requisite signaling capabilities. See Section 17 of {4} for more details on multi-carrier operation in the proposed RIT. |
| 4.2.3.2.8.3 | What are the frequency bands supported by the RIT? Please list. The proposed RIT supports deployment in all bands identified for IMT in ITU-R Radio Regulations. In addition, proposed RIT supports non-IMT bands below 6 GHz allocated to the Fixed Service and/or Mobile Service. See below for more information on some of the bands where the proposed RIT can be deployed. TABLE 6-10 Some of supported frequency bands Band Class; UL AMS Transmit Frequency (MHz); DL AMS Receive Frequency (MHz); Duplex Mode. 1; 2300-2400; 2300-2400; TDD. 2; 2305-2320, 2345-2360; 2305-2320, 2345-2360; TDD. 2345-2360; 2305-2320; FDD. 3; 2496-2690; 2496-2690; TDD. 2496-2572; 2614-2690; FDD. 4; 3300-3400; 3300-3400; TDD. 5L; 3400-3600; 3400-3600; TDD. 3400-3500; 3500-3600; FDD. 5H; 3600-3800; 3600-3800; TDD. 6; 1710-1770; 2110-2170; FDD. 1920-1980; 2110-2170; FDD. 1710-1755; 2110-2155; FDD. 1710-1785; 1805-1880; FDD. 1850-1910; 1930-1990; FDD. 1710-1785, 1920-1980; 1805-1880, 2110-2170; FDD. 1850-1910, 1710-1770; 1930-1990, 2110-2170; FDD. 7; 698-862; 698-862; TDD. 776-787; 746-757; FDD. 788-793, 793-798; 758-763, 763-768; FDD. 788-798; 758-768; FDD. 698-862; 698-862; TDD/FDD. 824-849; 869-894; FDD. 880-915; 925-960; FDD. 698-716, 776-793; 728-746, 746-763; FDD. 8; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; TDD. 9; 450-470; 450-470; TDD. 450.0-457.5; 462.5-470.0; FDD. |
| 4.2.3.2.8.4 | What is the minimum amount of spectrum required to deploy a contiguous network, including guard-bands (MHz)? The proposed RIT requires a minimum bandwidth of 2x5 MHz for FDD deployment and 5 MHz for TDD deployment. Amount of guard band depends on a variety of factors including deployment scenario and characteristics of systems in adjacent bands. |
| 4.2.3.2.8.5 | What are the minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points? The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component RF carrier) is 5 MHz. - The maximum 99% channel bandwidth (occupied bandwidth of single component RF carrier) is 20 MHz. - Multiple contiguous/non-contiguous component RF carriers can be aggregated to achieve transmission bandwidths in the order of 100 MHz. FIGURE 6-8 Illustration of different bandwidth concepts |
| 4.2.3.2.8.6 | What duplexing scheme(s) is (are) described in this template? (e.g. TDD, FDD or half-duplex FDD). The proposed RIT supports both TDD and FDD duplexing schemes as well as H-FDD terminal operation in FDD networks. Describe details such as: - What is the minimum (up/down) frequency separation in case of full- and half-duplex FDD? The up/down frequency separation depends on various parameters including target operating band plan. The proposed RIT minimum (up/down) frequency separation (as defined by the frequency separation between the centre frequency of UL and centre frequency of DL) is as follows: - 10 MHz for IMT band 450-470 MHz - 30 MHz for IMT band 698-960 MHz - What is the requirement of transmit/receive isolation in case of full- and half-duplex FDD? Does the RIT require a duplexer in either the AMS (MS) or BS? In FDD mode, the RIT supports interoperating H-FDD terminals with Full Duplex FDD (F-FDD) AMSs in FDD network. Duplexers are needed at the ABS for FDD operation; this includes ABSs serving networks including F-FDD as well as H-FDD AMSs interoperating in the network. Duplexers are needed in F-FDD AMSs. Duplexers are not required for H-FDD AMSs. The level of transmit/receive isolation depends on a number of parameters and in general is a function of parameters including interference characteristics of the band, spectrum emission mask, target receive performance requirement and up/down frequency separation. The Transmit-to-Receive Transition Gap (TTG) and Receive-to-Transmit Transition Gap (RTG) are given for TDD duplex mode and different bandwidths in the following table. TABLE 6-11 TTG and RTG gaps for different bandwidths and CP sizes CP= 1/8; TTG (µs); RTG (µs). Bandwidth (MHz). 5/10/20; 105.7; 60. 8.75; 87.2; 74.4. 7; 188; 60. CP 1/16; TTG (µs); RTG (µs). Bandwidth (MHz). 5/10/20; 82.853; 60. 8.75; 138.4; 74.4. 7; 180; 60. CP= 1/4; TTG (µs); RTG (µs). Bandwidth (MHz). 5/10/20; 139.98; 60. 8.75; 189.6; 74.4. 7; 140; 60. - What is the minimum (up/down) time separation in case of TDD? The up/down time separation for TDD mode depends on deployment scenario. The proposed RIT supports configurable up/down time separation as low as 50 μsec plus target Round Trip Delay (RTD). As an example, to accommodate a maximum cell range (RTD/2) of around 5.5 km, an additional 37 μsec is needed to be added to the ABS switching time from transmit to receive mode. This makes the total Transmit to Receive Time (TTG) equal to 87 μsec. This is an example and higher maximum cell ranges are possible. – Whether the DL/UL Ratio variable for TDD? What is the DL/UL ratio supported? If the DL/UL ratio for TDD is variable, what would be the coexistence criteria for adjacent cells? The DL/UL ratio is configurable but is typically fixed for a deployment. The proposed RIT supports the following DL/UL ratios in TDD mode of operation. Note that each radio frame of 5 ms consists of 8 subframes of 0.617 ms length. The permissible DL/UL ratios are as follows where, m, the first element of the (m, n) pairs represent the number of DL subframes and, n, the second element of the pairs represents the number of UL subframes: For 5, 10 and 20 MHz; (8,0), (6,2), (5,3), (4,4), and (3,5) For 8.75 MHz; (7,0), (5,2), and (4,3) For 7 MHz; (6,0), (4,2), and (3,3) As for the coexistence, all cells in a TDD deployment are required to use the same DL/UL ratio to minimize the inter-cell interference effects. Note that the (8,0), (7,0) and (6,0) cases are realization for the broadcast deployment scenario where another carrier or another technologies maybe used to provide for the return link. |
| 4.2.3.2.9 | Support of advanced antenna capabilities |
| 4.2.3.2.9.1 | Fully describe the multi-antenna systems supported in the MS, BS, or both that can be used and/or must be used; characterize their impacts on systems performance; e.g., does the RIT have the capability for the use of: - spatial multiplexing techniques, Yes (see Sections 11.8 and 11.12 of {4} for more information). - space-time coding (STC) techniques, Yes (see Sections 11.8 and 11.12 of {4} for more information). - Beamforming techniques (e.g., adaptive or switched). Yes (see Sections 11.8 and 11.12 of {4} for more information). The proposed RIT supports several open-loop/closed-loop single-user/multi-user MIMO schemes as described below (see Sections 11.8 and 11.12 of {4} for more information). The proposed RIT supports the following MIMO modes in the DL. TABLE 6-12 DL MIMO modes Mode index; Description; MIMO encoding format (MEF); MIMO precoding. Mode 0; Open-loop SU-MIMO; SFBC; non-adaptive. Mode 1; Open-loop SU-MIMO (spatial multiplexing); Vertical encoding; non-adaptive. Mode 2; Closed-loop SU-MIMO (spatial multiplexing); Vertical encoding; adaptive. Mode 3; Open-loop MU-MIMO (spatial multiplexing); Horizontal encoding; non-adaptive. Mode 4; Closed-loop MU-MIMO (spatial multiplexing); Horizontal encoding; adaptive. Mode 5; Open-loop SU-MIMO (TX diversity); Conjugate Data Repetition; non-adaptive. TABLE 6-13 DL MIMO parameters Number of transmit antennas; STC rate per layer; Number of streams; Number of subcarriers; Number of layers. Nt; R; Mt; NF; L. MIMO mode 0; 2; 1; 2; 2; 1. 4; 1; 2; 2; 1. 8; 1; 2; 2; 1. MIMO mode 1 and MIMO mode 2; 2; 1; 1; 1; 1. 2; 2; 2; 1; 1. 4; 1; 1; 1; 1. 4; 2; 2; 1; 1. 4; 3; 3; 1; 1. 4; 4; 4; 1; 1. 8; 1; 1; 1; 1. 8; 2; 2; 1; 1. 8; 3; 3; 1; 1. 8; 4; 4; 1; 1. 8; 5; 5; 1; 1. 8; 6; 6; 1; 1. 8; 7; 7; 1; 1. 8; 8; 8; 1; 1. MIMO mode 3 and MIMO mode 4; 2; 1; 2; 1; 2. 4; 1; 2; 1; 2. 4; 1; 3; 1; 3. 4; 1; 4; 1; 4. 8; 1; 2; 1; 2. 8; 1; 3; 1; 3. 8; 1; 4; 1; 4. MIMO mode 5; 2; 1/2; 1; 2; 1. 4; 1/2; 1; 2; 1. 8; 1/2; 1; 2; 1. The proposed RIT supports the following MIMO modes in the UL. TABLE 6-14 UL MIMO modes Mode index; Description; MIMO encoding format (MEF); MIMO precoding. Mode 0; Open-loop SU-MIMO; SFBC; non-adaptive. Mode 1; Open-loop SU-MIMO (spatial multiplexing); Vertical encoding; non-adaptive. Mode 2; Closed-loop SU-MIMO (spatial multiplexing); Vertical encoding; adaptive. Mode 3; Open-loop Collaborative spatial Multiplexing (MU-MIMO); Vertical encoding; non-adaptive. Mode 4; Closed-loop Collaborative spatial Multiplexing (MU-MIMO); Vertical encoding; adaptive. TABLE 6-15 UL MIMO parameters Number of transmit antennas; STC rate per layer; Number of streams per AMS; Number of subcarriers; Number of layers per AMS. Nt; R; Mt; NF; L. MIMO mode 0; 2; 1; 2; 2; 1. 4; 1; 2; 2; 1. MIMO mode 1 and MIMO mode 2; 2; 1; 1; 1; 1. 2; 2; 2; 1; 1. 4; 1; 1; 1; 1. 4; 2; 2; 1; 1. 4; 3; 3; 1; 1. 4; 4; 4; 1; 1. MIMO mode 3 and MIMO mode 4; 2; 1; 1; 1; 1. 4; 1; 1; 1; 1. 4; 2; 2; 1; 1. 4; 3; 3; 1; 1. |
| 4.2.3.2.9.2 | How many antennas are supported by the BS and MS for transmission and reception? Specify if correlated or uncorrelated antennas in co-polar or cross-polar configurations are used. What is the antenna spacing (in wavelengths)? The proposed RIT supports the following antenna configurations in the DL: The ABS employs a minimum of two and a maximum of 8 transmit antennas. The AMS employs a minimum of two receive antennas. See Tables in item 4.2.3.2.9.1 as well as Section 11.8 of {4}. The proposed RIT supports the following antenna configurations in the UL: The AMS employs a minimum of one and a maximum of 4 transmit antennas. The ABS employs a minimum of two receive antennas. See Tables in item 4.2.3.2.9.1 as well as Section 11.12 of {4}. Correlated/uncorrelated antennas in co-polar/cross-polar configuration is supported but implementation specific. While the question on antenna polarization and spacing is implementation specific and may vary in different deployment scenarios, the proposed RIT has been evaluated based on the antenna spacing and polarization guidelines specified by Report ITU-R M.2135. |
| 4.2.3.2.9.3 | Provide details on the antenna configuration that is used in the self-evaluation. A 4 TX x 2 RX antenna configuration in the DL and a 2 TX x 4 RX antenna configuration in the UL has been used in all four test environments to evaluate the performance of the RIT against the requirements in Report ITU-R M.2134. |
| 4.2.3.2.9.4 | If spatial multiplexing (MIMO) is supported, does the proposal support (provide details if supported) – Single codeword (SCW) and/or multi-codeword (MCW) The proposed RIT supports both SCW (also referred to as vertical coding in the proposed RIT) and MCW (also referred to as horizontal coding in the proposed RIT) schemes. The MCW scheme is used on ABS side in MU-MIMO schemes and the SCW is used on AMS side for both SU-MIMO and MU-MIMO schemes (see Tables in item 4.2.3.2.9.1 as well as Sections 11.8 and 11.12 of {4}). – Open and/or closed loop MIMO Both open-loop and closed-loop SU-MIMO and MU-MIMO are supported by the proposed RIT (see Tables in item 4.2.3.2.9.1 as well as Sections 11.8 and 11.12 of {4}). – Cooperative MIMO The proposed RIT supports cooperative MIMO (MU-MIMO) in the UL (see Tables in item 4.2.3.2.9.1 as well as Sections 18, 11.8 and 11.12 of {4}). – Single-user MIMO and/or multi-user MIMO. Both SU-MIMO and MU-MIMO schemes are supported by the proposed RIT (see Tables in item 4.2.3.2.9.1 as well as Sections 11.8 and 11.12 of {4}). |
| 4.2.3.2.9.5 | Other antenna technologies Does the RIT/SRIT support other antenna technologies, for example: – remote antennas, Yes. The use of remote antennas is not precluded by the RIT. Procedures are s implementation-specific. – distributed antennas. Yes, the proposed RIT supports multi-ABS MIMO scheme (see item 4.2.3.2.9.1 for details). Multi-ABS MIMO techniques are supported for improving sector throughput and cell-edge throughput through multi-ABS collaborative precoding, network coordinated beamforming, or inter-cell interference nulling. Both open-loop and closed-loop multi-ABS MIMO techniques are considered. For closed-loop multi-ABS MIMO, CSI feedback via codebook based feedback or sounding channel will be used. The feedback information may be shared by neighboring ABSs via network interface. Mode adaptation between single-ABS MIMO and multi-ABS MIMO is utilized. Additionally, the RIT does not preclude any implementation-specific distributed antenna schemes. If so, please describe. See Tables in item 4.2.3.2.9.1 as well as Sections 11.8 and 11.12 of {4}. |
| 4.2.3.2.9.6 | Provide the antenna tilt angle used in the self-evaluation. The following values are used in self-evaluation: Indoor – Indoor hotspot: 0 deg. Microcellular – Urban micro-cell: 12 deg. Base coverage urban – Urban macro-cell: 12 deg. High speed – Rural macro-cell: 6 deg. |
| 4.2.3.2.10 | Link adaptation and power control |
| 4.2.3.2.10.1 | Describe link adaptation techniques employed by RIT/SRIT, including: – the supported modulation and coding schemes, – the supporting channel quality measurements, the reporting of these measurements, their frequency and granularity. Provide details of any adaptive modulation and coding schemes, including: Hybrid ARQ or other retransmission mechanisms? - Algorithms for adaptive modulation and coding, which are used in the self-evaluation. - Other schemes? - The following link adaptation techniques are utilized in the proposed RIT: - MCS Adaptation The proposed RIT supports Adaptive Modulation and Coding (AMC) scheme for DL transmissions. For the DL, the S-ABS adapts the modulation and coding scheme (MCS) based on the DL channel quality indicators (CQI) reported by the AMS. DL control channel transmit power is also adapted based on CQI reports by the AMS. There are various MCS levels (combination of modulation, coding rate, repetition factor, and rate matching) that are used depending on channel and mobility conditions to ensure robustness and performance of the link. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure UL channel quality. For the UL, there are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL PFBCH provides wideband channel quality feedback and MIMO feedback. It is used to support robust feedback reports. The UL SFBCH carries narrowband CQI and MIMO feedback information. A set of predefined numbers of bits in this range is supported. The SFBCH can be used to support CQI reporting at higher code rate and thus more CQI information bits. The SFBCH can be allocated in a non-periodic manner based on traffic, channel conditions etc. The number of bits carried in the fast feedback channel can be adaptive. - The S-ABS adapts the modulation and coding scheme based on the UL channel quality estimation and the maximum transmission power by the AMS. UL control channels (excluding initial ranging channel) transmit power is also adapted based on UL power control. HARQ Adaptive, asynchronous HARQ scheme is used in the DL. In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, signaling is used to indicate the resource allocation and transmission format along with other HARQ necessary parameters Non-adaptive synchronous HARQ scheme is used in the UL. Resource (block) allocation for the retransmissions in the UL can be fixed or adaptive through signaling. The default operation mode of HARQ in the UL is non-adaptive, i.e., the parameters and the resource for the retransmission is known a priori. The ABS can by means of signaling enable an adaptive UL HARQ mode. In adaptive HARQ the parameters of the retransmission are signaled explicitly. MIMO Mode and Rank Adaptation (Switching) To support the various radio channels, both MIMO mode and rank adaptation are supported in the proposed RIT. The ABSs and AMSs may adaptively switch between DL MIMO techniques depending on parameters such as antenna configurations and channel conditions. Parameters selected for mode adaptation may have slowly or fast varying dynamics. By switching between DL MIMO techniques and the system can dynamically optimize throughput or coverage for a specific radio environment. The MIMO modes include open-loop MIMO like transmit diversity, spatial multiplexing, and closed-loop MIMO, etc. The adaptation of these modes is related with the system load, the channel information, AMS speed and average CINR. Switching between SU-MIMO and MU-MIMO is also supported. Both dynamic and semi-static adaptation mechanisms are supported in the proposed RIT. For dynamic adaptation, the mode/rank may be changed in a frame by frame basis. For semi-static adaptation, the AMS may request adaptation. The decision of rank and mode adaptation is made by the S-ABS. The adaptation occurs slowly, and involves negligible feedback overhead. Power Control The power control scheme is supported for DL and UL based on the frame structure, DL/UL control structures, and Fractional Frequency Reuse (FFR). The ABS is capable of controlling the transmit power per sub-frame and per user. With DL power control, each user-specific information or control information would be received by the AMS with the controlled power level. DL unicast service control channel is power-controlled based on the AMS UL channel quality feedback. The per pilot tone power and the per data tone power is jointly adjusted for adaptive DL power control. In the case of dedicated pilots this is done on a per user basis and in the case of common pilots this is done jointly for the users sharing the pilots. Power control in DL supports SU-MIMO and MU-MIMO modes. UL power control is supported to compensate the path loss, shadowing, fast fading and implementation loss. UL power control is also used to control inter-cell and intra-cell interference level. UL power control considers optimization of overall system performance and the reduction of battery consumption. UL power control consists of two different modes: Open-Loop Power Control (OLPC) and Closed-Loop Power Control (CLPC). The ABS transmits necessary information through control channel or message to AMSs to support UL power control. The parameters of power control algorithm are optimized on system-wide basis by the ABS, and broadcast periodically or trigged by events. The AMS transmits necessary information through control channel or message to the ABS to support UL power control. The ABS exchanges necessary information with neighbor ABSs through backbone network to support UL power control. In high mobility scenarios, power control scheme may not compensate the fast fading channel effect because of the very dynamic changes of the channel response. As a result, the power control is only used to compensate the distance-dependent path loss, shadowing and implementation loss. UL power control considers the transmission mode depending on the single- or multi-user support in the same allocated resource at the same time. The OLPC compensates the channel variations and implementation loss without frequently interacting with ABS. The AMS determines the transmit power based on the transmission parameters sent by the ABS, UL channel transmission quality (e.g. indicated as ACK or NACK), DL channel state information and interference knowledge obtained from the DL. The AMSs use UL OLPC applying channel and interference knowledge to operate at optimum power settings. OLPC provides a coarse initial power setting of the terminal at the beginning of a connection. As for mitigating inter-cell interference, power control considers S-ABS link target SINR and/or target interference to other cells/sectors. In order to achieve target SINR, the S-ABS path-loss can be fully compensated for a tradeoff between overall system throughput and cell edge performance. When considering target interference to other cells/sectors, AMS TX power is controlled to generate less interference than the target interference levels. The compensation factor for each frequency partition and interference targets for each frequency partition are determined and broadcast by the ABS, with considerations including FFR pattern, cell loading, etc. - The CLPC compensates channel variation with power control commands from the ABS. ABS measures UL channel state information and interference information using UL data and/or control channel transmissions and sends power control commands to AMSs while minimizing signalling overhead. According to the power control command from the ABS, the AMS adjust its UL transmission power. |
| 4.2.3.2.10.2 | Provide details of any power control scheme included in the proposal, for example: In DL, power levels can be set in a per-allocation basis based on the CQI. Details of DL power setting are implementation specific. For UL power control, details are provided below. - Power control step size (dB) 0.5 dB - Power control cycles per second Maximum one power control command per subframe that translates into 1600 Hz. - Power control dynamic range (dB) The power adjustment range is -0.5 dB to 1.0 dB. The minimum power control dynamic range required is 45 dB. - Minimum transmit power level with power control The minimum transmit power of an AMS with power control is -22 dBm, assuming 23 dBm declared maximum transmit power and minimum 45 dB dynamic range. - Associated signaling and control messages. To maintain at the ABS a power density consistent with the modulation and coding rate used by each AMS, the ABS may change the AMS’s TX power through direct power adjustment/control signalling. The UL power control is signaled as part of DL control structure. See the illustrations in item 4.2.3.2.7.1 and more details in Section 11.10 of {4}. |
| 4.2.3.2.11 | Power classes |
| 4.2.3.2.11.1 | Mobile station emitted power Table 6-16 provides the list of AMS power classes. - TABLE 6-16 - AMS Power Classes Class Identifier; Pnom (Nominal Max Output Power) (dBm). Power Class 1; 20. Power Class 2; 23. Power Class 3; 26. - In the Table 6-16, Pnom is the declared maximum conducted transmit power. In power classes of Table 6-16, tolerances are not included. In general, tolerances associated with Pnom depend on various parameters including MCS level, spectrum band span (associated with band edge), etc. Generic and band-specific tolerances associated with these parameters may be added, if applicable. |
| 4.2.3.2.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? The typical conducted output transmit power, measured as the mean power delivered to the antenna port, according to Power Class 2 of item 4.2.3.2.11.1 is less than or equal to 23 dBm. The total radiated antenna power, assuming 0 dBi antenna gain is less than or equal to 23 dBm. |
| 4.2.3.2.11.1.2 | What is the maximum peak power transmitted while in active or busy state? The maximum peak power transmitted while in active or busy state is 23 dBm. |
| 4.2.3.2.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. The average TX power depends on several factors such as the power control parameter settings, the test environment, etc. For example, for the OLPC scheme of the RIT with power control parameters γ and SINRMIN equal to 0.5 and 1 dB (see table A3-3), respectively, the average TX power is 11.4 dBm for the UMi test environment and 11.8 dBm for the UMa test environment. |
| 4.2.3.2.11.2 | Base station emitted power |
| 4.2.3.2.11.2.1 | What is the base station transmit power per RF carrier? ABS’s transmit power, measured as the mean power delivered to the antenna port, depends on various factors including those related to the deployment scenario and possible local regulatory requirements. |
| 4.2.3.2.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? The maximum peak transmitted power per RF carrier radiated from antenna depends on the ABS peak transmit power at the antenna port and the antenna gain. The peak transmitted power per RF carrier radiated from antenna is within a range of values properly considered to cover target deployment scenarios. This value may be subject to local regulatory limitations. |
| 4.2.3.2.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? The average transmitted depends on the network loading factor, traffic type, and test environment. |
| 4.2.3.2.12 | Scheduler, QoS support and management, data services |
| 4.2.3.2.12.1 | QoS support - What QoS classes are supported? The following QoS classes are supported : - Unsolicited Grant Service (UGS) is designed to support real-time service flows that transport fixed-size data packets on a periodic basis, such as T1/E1 and VoIP without silence suppression. - Real-time Polling Service (rtPS) is designed to support real-time service flows that transport variable-size data packets on a periodic basis, such as Moving Pictures Experts Group (MPEG) video. - Extended rtPS (ertPS) is a scheduling mechanism which builds on the efficiency of both UGS and rtPS. The ABS shall provide unicast grants in an unsolicited manner like in UGS, thus saving the latency of a BR. However, whereas UGS allocations are fixed in size, ertPS allocations are dynamic. - Non-real-time polling service (nrtPS) offers unicast polls on a regular basis, which assures that the service flow receives request opportunities even during network congestion. The ABS typically polls nrtPS connections on an interval on the order of one second or less. - Best effort (BE) service provides service for BE traffic. - How QoS classes associated with each service flow can be negotiated. Signalling support is provided using which QoS parameter sets are negotiated between the ABS and the AMS during the service flow setup/change procedure. See further description below. - QoS attributes, for example: The QoS parameters vary depending on the type of service. The following are the typical QoS parameters that are used in conjunction with scheduling services in the proposed RIT. - Traffic priority - Maximum sustained traffic rate - Minimum reserved traffic rate - Maximum latency - Is QoS supported when handing off between radio access networks? Please describe. Yes. The QoS requirements and their satisfaction is one of the criteria for selection of T-ABS during handover procedure. The admission control mechanism in candidate T-ABSs ensure that the required QoS can be satisfied upon handover of the AMS from the S-ABS to the T-ABS. – How users may utilize several applications with differing QoS requirements at the same time. The QoS parameters are defined per service flow and thereby for each MAC connection established between the ABS and AMS. An AMS can have multiple active service flows at each time instant. The proposed RIT MAC associates a unidirectional flow of packets which have a specific QoS requirement with a service flow. A service flow is mapped to one transport connection with one flow identifier. The ABS and AMS provide QoS according to the QoS parameter sets, which are negotiated between the ABS and the AMS during the service flow setup/change procedure. The QoS parameters can be used to schedule traffic and allocate radio resource. In addition, UL traffic may be policed based on the QoS parameters. The proposed RIT supports following additional information field parameters (relative to that of IEEE Std 802.16-2009 {1}): Tolerated packet loss rate: The value of this parameter specifies the maximum packet loss rate for the service flow. Indication of Associated Flows: A parameter that indicates the flow(s) that is associated with the current service flow if any. - Adaptive polling and granting: The proposed RIT supports adaptation of service flow QoS parameters. One or more sets of QoS parameters are defined for one service flow. The AMS and ABS negotiate the supported QoS parameter sets during service flow setup procedure. When QoS requirement/traffic characteristics for traffic changes, the ABS may autonomously switch the service flow QoS parameters such as grant/polling interval or grant size based on predefined rules. In addition, the AMS may request the ABS to switch the Service Flow QoS parameter set with explicit signaling. The ABS then allocates resource according to the new service flow parameter set. - Scheduling Services: Proposed RIT provides a specific scheduling service to support real-time non-periodical applications such as on-line gaming. In addition to the above services, the system also supports: Persistent Allocation (PA): PA is used to reduce resource allocation signaling (MAP) overhead for connections with periodic traffic pattern and with relatively fixed payload size. Group Resource Allocation (GRA): GRA is used to reduce resource allocation signaling (MAP) overhead for multiple connections with a pre-determined and well-known packet size. Instead of allocating resources to single user, the ABS may create one or more groups, each group containing more than one user. |
| 4.2.3.2.12.2 | Scheduling mechanisms - Exemplify scheduling algorithm(s) that may be used for full buffer and VoIP traffic in the technology proposal for evaluation purposes. Describe any measurements and/or reporting required for scheduling. Scheduling algorithms are implementation-specific. The RIT facilitates dynamic scheduling at subframe intervals and fast sub-band/full-band feedback mechanisms for channel conditions and traffic levels. The RIT performance evaluation uses a Proportional-Fair algorithm for full buffer data performance (see details below) and a persistent scheduling for VoIP capacity calculations. In the proposed RIT, dynamic scheduling on a subframe basis is applied to both UL and DL. Typically, the scheduling is based on the instantaneous radio-link quality as seen by the different users, and QoS requirements of individual users in the cell. The former is based on periodic CQI reports from the terminals (DL) or measurements of sounding signals from the terminals (UL). Based on this, the ABS may apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the upper layers. |
| 4.2.3.2.13 | Radio interface architecture and protocol stack |
| 4.2.3.2.13.1 | Describe details of the radio interface architecture and protocol stack such as, Logical channels - Control channels See item 4.2.3.2.7.1 for description of DL and UL control channels, including the mapping of information to the control channels. - Traffic channels Although not explicitly defined for the proposed RIT, the flows and their resource mapping (see items 4.2.3.2.6.3 and 4.2.3.2.12) effectively define the logical traffic channels. Transport channels and/or physical channels. Transport and physical channels are described together in the earlier items (see 4.2.3.2.2.3.1 and 4.2.3.2.4.1) - RIT Protocol Structure The proposed RIT MAC is divided into two sublayers: - Convergence sublayer (CS) - Common Part sublayer (CPS) MAC CPS is further classified into Radio Resource Control and Management (RRCM) functions and MAC functions. The RRCM functions fully reside on the control plane. The MAC functions reside on the control and data planes (see Figure 6-9). The RRCM functions includes several functional blocks that are related with radio resource functions such as: - Radio resource management - Mobility management - Network-entry management - Location management - Idle mode management - Security management - System configuration management - E-MBS - Connection management - Relay functions - Self organization - Multi-carrier The control plane part of MAC includes functional blocks which are related to the physical layer and link controls such as: - PHY Control - Control signaling - Sleep mode management - QoS - Scheduling and resource multiplexing - Multi-radio coexistence - Data forwarding - Interference management - Inter-ABS coordination The data plane includes the following MAC functions: - ARQ - Fragmentation/packing - MAC PDU formation - Encryption FIGURE 6-9 L1 and L2 protocol stack of the proposed RIT |
| 4.2.3.2.13.2 | What is the bit rate required for transmitting feedback information? - Channel quality feedback Channel quality feedback provides information about channel conditions as seen by the AMS. This information is used by the ABS for link adaptation, resource allocation, power control etc. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure UL channel quality. - MIMO feedback MIMO feedback provides wideband and/or narrowband spatial characteristics of the channel that are required for MIMO operation. The MIMO mode, precoder matrix index, rank adaptation information, channel covariance matrix elements, and channel sounding are examples of MIMO feedback information. - HARQ feedback HARQ feedback (ACK/NACK) is used to acknowledge DL transmissions. There are two types of UL fast feedback control channels: Primary Fast Feedback Channel (PFBCH) and Secondary Fast Feedback Channels (SFBCH). The UL PFBCH carries 4 to 6 bits of information. The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the UL SFBCH ranges from 7 to 24. Assuming that CQI is sent periodically every 20 msec, UL PFBCH bit rate can be 200~300 bps, and UL SFBCH bit rate can be 400~1200 bps. See Section 15.3.9 of {6} for details of the UL feedback channels. |
| 4.2.3.2.13.3 | Channel access: Describe in details how RIT/SRIT accomplishes initial channel access, (e.g. contention or non-contention based). The UL ranging channels are used for UL synchronization. The UL ranging channels can be further classified into ranging channel for non-synchronized AMSs and synchronized mobiles stations. A random access procedure, which can be contention based or non-contention based is used for ranging. Contention-based random access is used for initial ranging, while either contention-based on non-contention-based random access is used for periodic ranging and handover. The location of the ranging channels is provided in a DL broadcast control message. The ranging channels are frequency-division multiplexed with other UL control channels and data channels. See Section 11.9 of {4} for details of initial ranging and contention-based access |
| 4.2.3.2.14 | Cell selection |
| 4.2.3.2.14.1 | Describe in detail how the RIT/SRIT accomplishes cell selection to determine the serving cell for the users. The decision for attachment or cell selection is made in the end of the initialization procedures. In the Initialization state, the AMS performs cell selection by scanning and synchronizing to DL preambles, and acquiring the broadcast system configuration information, which includes the network identity and other parameters. Upon successful initialization, the AMS enters the Access State. The S-ABS may unicast the Neighbor Advertisement message to an AMS. The Neighbor Advertisement message may include parameters required for cell selection e.g., cell load and cell type. Upon successful completion of the Initialization state (refer to AMS state transition diagram in item 4.2.3.2.6.3). See Figure 6-10 and Sections 10.8 and 11.9 of {4} for more details on cell selection in the proposed RIT. FIGURE 6-10 Cell search and selection procedures in the proposed RIT |
| 4.2.3.2.15 | Location determination mechanisms |
| 4.2.3.2.15.1 | Describe any location determination mechanisms that may be used, e.g., to support location based services. The RIT supports assisted-GPS and non-GPS methods such as DL Time Difference of Arrival (DL-TDOA) and UL-TDOA, as well as a hybrid of GPS and non-GPS methods. Both network-based and AMS-based methods are possible. See Section 12 of {4} for details of location-based services methods in the proposed RIT. |
| 4.2.3.2.16 | Priority access mechanisms |
| 4.2.3.2.16.1 | Describe techniques employed to support prioritization of access to radio or network resources for specific services or specific users (e.g., to allow access by emergency services). For handling emergency telecommunications service and E-911, emergency service flows will be given priority in admission control over the regular service flows. Default service flow parameters are defined for emergency service flow. The ABS grants resources in response to an emergency service notification from the AMS without going through the complete service flow setup procedure. The AMS can include an emergency service notification in initial ranging or service flow setup requests. If a service provider wants to support National Security/Emergency Preparedness (NS/EP) priority services, the ABS uses its own algorithm as defined by its local country regulation body. For example, in the US the algorithm to support NS/EP is defined by the Federal Communications Commission (FCC) in Hard Public Use Reservation by Departure Allocation (H-PURDA). See Section 10.9.3 of {4} for details on support of emergency services and priority access. |
| 4.2.3.2.17 | Unicast, multicast and broadcast |
| 4.2.3.2.17.1 | Describe how the RIT enables: - broadcast capabilities, - multicast capabilities, - unicast capabilities, using both dedicated carriers and/or shared carriers. Please describe how all three capabilities can exist simultaneously. The basic concepts and procedures in E-MBS are consistent with E-MBS definitions in IEEE Std 802.16-2009 {1}, but the concepts have been adapted to the new MAC and PHY structure. E-MBS refers to a data service offered on multicast connection using specific E-MBS features in MAC and PHY to improve performance and operation in power saving modes. An ABS may allocate simple multicast connections without using E-MBS features. Two types of access to E-MBS may be supported: single-ABS access and multi-ABS access. Single-ABS access is implemented over multicast and broadcast transport connections within one ABS, while multi-ABS access is implemented by transmitting data from service flow(s) over multiple ABSs, using the concept of E-MBS zone. That transmission is supported either in the non-macro diversity mode or macro diversity mode as a wide-area multi-cell Multicast-Broadcast Single Frequency Network (MBSFN). E-MBS service may be delivered via either a dedicated carrier or a mixed unicast-broadcast carrier. See Section 13 of {4} for details on E-MBS support. |
| 4.2.3.2.17.2 | Describe whether the proposal is capable of providing multiple user services simultaneously to any user with appropriate channel capacity assignments? Yes, the proposed RIT can establish several concurrent service flows with different QoS parameters for each user in the network. See Section 10.10 in {4}. |
| 4.2.3.2.17.3 | Provide details of the codec used for VoIP capacity in the self evaluation. Does the RIT support multiple voice and/or video codecs? Provide details. The 12.2 kbps source coder recommended by Report ITU-R M.2135 has been used for VoIP capacity calculations in self-evaluation (Section 7). The proposed RIT is an all-IP air-interface and supports any standard voice/video codec with IETF-specified Real-Time Protocol (RTP) payload. |
| 4.2.3.2.17.4 | If a codec is used that differs from the one specified in Annex 2 of Report ITU-R M.2135, specify the voice quality (e.g., PSQM, PESQ, CCR, E-Model, MOS) for the corresponding VoIP capacity in the self-evaluation. The RIT VoIP performance self-evaluation is based on the 12.2 kbps codec that has been recommended by Report ITU-R M.2135 for VoIP capacity calculations. |
| 4.2.3.2.18 | Privacy, authorization, encryption, authentication and legal intercept schemes |
| 4.2.3.2.18.1 | Any privacy, authorization, encryption, authentication and legal intercept schemes that are enabled in the radio interface technology should be described. Describe whether any synchronization is needed for privacy and encryptions mechanisms used in the RIT. Describe how the RIT may be protected against attacks, for example: − man in the middle, − replay, − denial of service. - Security architecture of the proposed RIT: The security functions provide subscribers with privacy, authentication, and confidentiality across the proposed RIT network. It does this by applying cryptographic transforms to either MPDUs or management messages carried across connections between AMS and ABS. The security architecture of The proposed RIT system consists of the following functional entities; the AMS, the ABS, and the Authenticator. The Figure 6-11 describes the protocol architecture of security services. FIGURE 6-11 Protocol architecture of security services Functional blocks of the proposed RIT security architecture: Within AMS and ABS the security architecture is divided into two logical entities: - Security management entity - Encryption and integrity entity Security management entity functions includes : - Overall security management and control - EAP encapsulation/decapsulation for authentication - Privacy Key Management (PKM) control (e.g. key generation/derivation/distribution, key state management) - Authentication and Security Association (SA) control - Location privacy Encryption and integrity protection entity functions include: - Transport data Encryption/Authentication Processing (EAP) - Management message authentication processing - Management message confidentiality protection Pair-wise mutual authentication of user and device identities takes place between AMS and ABS entities using EAP. Authentication is performed during initial network entry and periodically thereafter. In order to protect the mapping between the STID and the ms MAC Address, a Temporary STID (TSTID) is assigned during initial ranging process, and is used until the STID is allocated using the encrypted registration message. The STID is used for all the remaining transactions. The proposed RIT supports the selective confidentiality/integrity protection over MAC management messages. |
| 4.2.3.2.19 | Frequency planning |
| 4.2.3.2.19.1 | How does the RIT support adding new cells or new RF carriers? Provide details. The proposed RIT support maximum of 512 cell ID’s to support various cell types. The RIT provides features to support self-configuration. RF carriers maybe overlaid or reused in the conventional sense, or supported through multi-carrier support (see item 4.2.3.2.8.2). In multi-carrier, carriers are either primary carrier to exchange traffic and PHY/MAC control information or secondary carrier for traffic only. A secondary carrier may also include control signaling to support multi-carrie operation. Multi-carrier support may be applied to multi-bandwidth; that is, multiple carriers with different channel bandwidth. |
| 4.2.3.2.20 | Interference mitigation within radio interface |
| 4.2.3.2.20.1 | Does the proposal support Interference mitigation? If so, describe the corresponding mechanism. Yes, the proposed RIT supports advanced interference mitigation techniques as described below, in addition to conventional frequency reuse schemes. The RIT supports Fractional Frequency Reuse (FFR) to allow different frequency reuse factors to be applied over different frequency partitions during the designated period for both DL and UL transmissions (Figure 6-12). The operation of FFR is usually integrated with other functions like power control or antenna technologies for adaptive control and joint optimization. The basic concept of FFR is introduced by the example in the following figure. FFR performance can be optimized using AMS measurement reports and inter-ABS co-ordination to allocate resources and power appropriately. FIGURE 6-12 Basic concept of Fractional Frequency Reuse (FFR) Interference mitigation within radio interface Conjugate Data Repetition (CDR) may be supported in the proposed RIT. Multi-antenna beamforming and multi-ABS MIMO techniques are also supported in the RIT. See Section 18 of {4} for more details on interference mitigation techniques used in the proposed RIT. |
| 4.2.3.2.20.2 | What is the signaling, if any, which can be used for inter-cell interference mitigation? Interference mitigation using advanced antenna: In order to support DL PMI coordination to mitigate inter-cell interference, the AMS is capable of measuring the channel from the interfering ABS, calculates the worst or least interfering PMIs, and feedbacks the restricted or recommended PMIs to the S-ABS together with the associated ABS IDs or information assisting in determining the associated ABS IDs. PMI for neighboring cell is reported based on the base codebook. The measurement can be performed over the region implicitly known to AMS or explicitly designated by ABS. The PMIs can then be reported to ABS by UL control channel and/or MAC layer messaging in solicited/unsolicited manner. For UL PMI coordination, the ABS is capable of measuring the channel from the interfering AMS using sounding signals. Neighboring ABS should calculate the PMIs with least interference and forward them to the S-ABS. See Section 18 of {4} for more details on interference mitigation techniques used in the proposed RIT. |
| 4.2.3.2.20.3 | Link level interference mitigation Describe the feature or features used to mitigate inter-symbol interference. The proposed RIT uses OFDMA with various CP sizes to cope with inter-symbol interference effects. The CP length is chosen to absorb the inter-symbol interference due to propagation delay in certain frequency range and cell size. See Section 11.3 OF {4} for more details on different OFDMA parameters. |
| 4.2.3.2.20.4 | Describe the approach taken to cope with multipath propagation effects (e.g. via equalizer, rake receiver, cyclic prefix, etc.). The OFDMA systems rely on CP and simple one-tap equalization methods. Since OFDM systems convert wideband frequency selective channels into several narrowband flat fading channels, the RIT does not require complex equalization schemes such as rake receiver. |
| 4.2.3.2.20.5 | Diversity techniques Describe the diversity techniques supported in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – Time diversity: Repetition, Rake-receiver, etc. – Space diversity: Multiple sectors, etc. – Frequency diversity: Frequency hopping (FH), wideband transmission, etc. – Code diversity: Multiple PN codes, multiple FH code, etc. – Multi-user diversity: Proportional fairness (PF), etc. – Other schemes. Characterize the diversity combining algorithm, for example, switched diversity, maximal ratio combining, equal gain combining. Provide information on the receiver/transmitter RF configurations, for example: - number of RF receivers - number of RF transmitters. Spatial diversity is supported through multiple TX and RX antenna schemes described in item 4.2.3.2.9.4. Frequency diversity is achieved through the use of distributed resource allocations (see item 4.2.3.2.7.1). Time and multi-user diversity are achieved through dynamic scheduling and HARQ (see item 4.2.3.2.12.1). |
| 4.2.3.2.21 | Synchronization requirements |
| 4.2.3.2.21.1 | Describe RIT’s timing requirements, e.g. - Is BS-to-BS synchronization required? Provide precise information, the type of synchronization, i.e., synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy. - Is BS-to-network synchronization required? State short-term frequency and timing accuracy of BS transmit signal. - Network synchronization For TDD and FDD realizations, it is recommended that all ABSs should be time synchronized to a common timing signal. In the event of the loss of the network timing signal, ABSs continue to operate and automatically resynchronize to the network timing signal when it is recovered. The synchronizing references a 1 pps timing pulse and a 10 MHz frequency reference. These signals are typically provided by a GPS receiver but can be derived from any other source which has the required stability and accuracy. For both FDD and TDD realizations, frequency references derived from the timing reference may be used to control the frequency accuracy of ABSs provided that they meet the frequency accuracy requirements. This applies during normal operation and during loss of timing reference. See Section 20 of {4} for more information on inter-ABS synchronization. - DL frame synchronization At the ABS, the transmitted DL radio frame is time-aligned with the 1pps timing pulse with a possible delay shift of n micro-seconds (n being between 0 and 4999). The start of the preamble symbol, excluding the CP duration, is time aligned with 1pps plus the delay of n micro-seconds timing pulse when measured at the antenna port. See Section 20 of {4} for more information on inter-ABS synchronization. |
| 4.2.3.2.21.2 | Describe the synchronization mechanisms used in the proposal, including synchronization between a user terminal and a site. DL synchronization and detection of preambles (unique synchronization sequences) The proposed RIT also includes ranging and UL synchronization to enable proper operation of OFDMA systems. See Section of 11.7 of {4} for details on DL synchronization. |
| 4.2.3.2.22 | Link budget template Proponents should complete the link budget template in § 4.2.3.3 to this description template for the environments supported in the RIT. Please refer to Section 4.2 of this document for link budget template (Tables 6-78 to 6-85). |
| 4.2.3.2.23 | Other items |
| 4.2.3.2.23.1 | Coverage extension schemes Describe the capability to support/ coverage extension schemes, such as relays or repeaters. The proposed RIT supports advanced relaying schemes as described below: Relay models capture the modes of relay operation supported in the proposed RIT based on the frame structure and the access station perspectives. Relaying is performed using a decode–and-forward paradigm. The ABS and Advanced Relay Station (ARS)s deployed within a sector operate using either TDD or FDD of DL and UL transmissions. An ARS operates in Time Division Transmit/Receive (TTR) mode where transmission to subordinate station and reception from the super-ordinate station or transmission to the super-ordinate station and reception from subordinate station is separated in time over the radio frame. The ARSs operates non-transparent mode. Security mode can be either centralized or distributed security. |
| 4.2.3.2.23.2 | Self-organization Describe any self-organizing aspects that are enabled by the RIT/SRIT. - Support for Self-organization Self-Organizing Network (SON) functions are intended for ABSs (e.g. macro, relay, femto) to automate the configuration of ABS parameters and to optimize network performance, coverage and capacity. The scope of SON is limited to the measurement and reporting of air interface performance metrics from AMS/ABS, and the subsequent adjustments of ABS parameters. - Self-configuration Self-configuration is the process of initializing and configuring ABSs automatically with minimum human intervention. The self-configuration may use optimized parameters and provide fast reconfiguration. - Cell initialization Basic MAC and PHY parameters may be decided by core network before ABS operation. If not configured by the core network, OFDM parameters (e.g. CP and OFDM symbol length, DL/UL ratio), channel bandwidth and preamble sequence may be configured or selected through inter-ABS communication, a database, or through measurement by the ABS. An ABS or SON function selects a preamble sequence that precludes any sequence being used by neighbor cells with the same carrier frequency. - Neighbor discovery The initial neighbor list is obtained from core network automatically. Any change of the neighbor environment such as ABSs being added or removed should automatically trigger the ABS to generate an updated neighbor list. The information for updating the neighbor list (e.g. macro ABS, Femto ABS) is collected by ABS/ARS/AMS measurement, core network, inter-ABS network signaling, ABS’s own management. The ABS should direct an AMS to report measurement and use cached and feedback information to reduce the undesirable transmission from the AMS. - Macro ABS self-configuration SON automatically updates the neighbor list whenever there is a change in the neighbor environment. A macro ABS reports the following parameters to initiate automatic neighbor list update: - BSID - Cell site coordinates (longitude, latitude) - Sector bearing (orientation), indicating the direction where the sector is pointing - ABS attributes (e.g. channel bandwidth, FFT Size, CP, etc.) In response, the macro ABS will receive the following parameters to update its neighbor list: - BSID - ABS attributes (e.g. channel bandwidth, FFT Size, CP, etc.) - Self-optimization Self-optimization is the process of analyzing the reported SON measurement from the ABS/AMS and fine-tuning the ABS parameters in order to optimize the network performance which includes QoS, network efficiency, throughput, cell coverage and cell capacity. The reported SON measurements from ABS/AMS may include but not limited to: - Signal quality of S-ABS and neighbor ABSs - Interference level from the neighbor ABSs - Cell information of neighbor ABSs - Status of mobility management (e.g., HO, Idle mode) - Time and location information of AMS at a measurement - Load information of neighbor ABS - Coverage and capacity optimization The coverage and capacity optimization aims to detect and resolve the blind areas for reliable and maximized network coverage and capacity when an AMS cannot receive any strong enough signals from any ABSs. The SON functions process the reported measurement and then determine the location of the blind areas in order for subsequent coverage extension and capacity optimization. - Interference management and optimization Inter-cell interference should be maintained below a certain maximal interference level. Newly deployed ABS may select the carrier frequency, antenna setting, power allocation, and/or channel bandwidth based on the minimum interference level and the available capacity of the backhaul link. This can be achieved by a set of measurements by scanning the surrounding neighbor cells with/without additional information collected from other AMS and ABS. The reassignment/modification due to interference management should take into consideration of the load status and other parameters (e.g. antenna and power setting optimization for femto ABS etc). When a new ABS is deployed, the initialization for interference management is automatically configured by inter-ABS or a SON server. - Load management and balancing Cell reselection and handover procedures of an AMS may be performed at the direction of the ABS to control the unequal traffic load and minimize the number of handover trials and redirections. The load of the cells, modification of neighbor lists, and the selection of alternative carriers should be automatically managed through inter-ABS communication and the SON server. An ABS with unsuitable load status may adjust its cell reselection and handover parameters to control the imbalanced load with the neighbors ABSs. - Self-optimizing FFR Self-optimizing FFR is designed to automatically adjust FFR parameters, frequency partitions and power levels, among ABS sectors in order to optimize system throughput and user experience. |
| 4.2.3.2.23.3 | Describe the frequency reuse schemes (including reuse factor and pattern) for the assessment of cell spectrum efficiency, cell edge user spectral efficiency and VoIP capacity. The RIT performance self-evaluation is based on a frequency reuse factor of 1. |
| 4.2.3.2.23.4 | Is the RIT an evolution of an existing IMT-2000 technology? Provide details. Yes. The proposed RIT is an evolution of IMT-2000 OFDMA TDD WMAN. The proposed RIT provides continuing support and interoperability for R1 MSs and R1 BSs, which are specified in accordance with IMT-2000 OFDMA TDD WMAN (see in particular subclause 5.6.2.1.1 of revision of ITU-R Recommendation M.1457-8). R1 MS is a mobile station compliant with WirelessMAN-OFDMA R1 Reference System. R1 BS is a base station compliant with WirelessMAN-OFDMA R1 Reference System. WirelessMAN-OFDMA R1 Reference System is a system compliant with WirelessMAN-OFDMA capabilities specified in Subclause 12.5 of IEEE Std 802.16-2009. Specifically, the features, functions and protocols enabled in the proposed RIT support the features, functions and protocols employed by IMT-2000 OFDMA TDD WMAN, as follows: - The AMS is able to operate with an R1 BS, at a level of performance equivalent to that of an R1 MS. - Systems based on the proposed RIT and those based on {1} are able to operate on the same RF carrier, with the same channel bandwidth and should be able to operate on the same RF carrier with different channel bandwidths. - The ABS supports a mix of the AMS and R1 MSs when both are operating on the same RF carrier. The system performance with such a mix should improve with the fraction of the AMSs attached to the ABS. - The ABS supports handover of an R1 MS to and from an R1 BS and to and from an ABS, at a level of performance equivalent to handover between two R1 BSs. - The ABS shall be able to support an R1 MS while also supporting AMSs on the same RF carrier, at a level of performance equivalent to that an R1 BS provides to an R1 MS. The proposed RIT, however, provides the ability to disable support for R1 MSs and R1BSs. |
| 4.2.3.2.23.5 | Does the proposal satisfy a specific spectrum mask? Provide details. (This information is not intended to be used for sharing studies.) See Table 6-10 for definition of Band Classes. AMS spectral masks: Unless otherwise specified for specific bands, the spectrum masks of the following three tables (6-17, 6-18, 6-19) are applicable to AMSs. TABLE 6-17 AMS Spectrum Emission Mask for 5 MHz Bandwidth Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port. 1; 2.5 to < 3.5; 50; -13. 2; 3.5 to  12.5; 1000; -13. - TABLE 6-18 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to < 6; 100; -13. 2; 6 to  25; 1000; -13. - TABLE 6-19 - AMS Spectrum Emission Mask for 20 MHz Bandwidth Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to  50; 1000; -13. AMS Band Class 1: TABLE 6-20 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 1 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port.. 1; 2.5 to < 3.5; 50; -13. 2; 3.5 to < 7.5; 1000; -13. 3; 7.5 to < 8; 500; -16. 4; 8 to < 10.4; 1000; -25. 5; 10.4 to ≤ 12.5; 1000; -25. Notes: - f is defined as the frequency offset in MHz from the center frequency of a 5 MHz channel. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-20-1 AMS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1, Ptx ≤23 dBm Segment Number; ffset from channel center (MHz); Integration Bandwidth (KHz); Allowed Emission Level as measured at the antenna port. 1; 4.77 to  9.27; 100; -[26+7×{(Δf¦-4.77 MHz)/4.5 MHz }] dB. 2; 9.27 to 13.23; 100; -[33+4×{(¦Δf¦-9.27 MHz)/3.96 MHz }] dB. 3; 13.23 to  17.73; 100; -[37+2×{(Δf¦-13.23 MHz)/4.5 MHz }] dB. 4; 17.73 to  22.5; 100; -39 dB. Notes: f is defined as the frequency offset in MHz from the center frequency of the 8.75 MHz channel. 2. PTx is the measured power in dBm into the antenna. 3. Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-20-2 AMS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1, Ptx >23 dBm Segment Number; ffset from channel center (MHz); Integration Bandwidth (KHz); Allowed Emission Level as measured at the antenna port. 1; 4.77 to  9.27; 100; -[{(PTx-23)+26}+7×{(Δf-4.77 MHz)/4.5 MHz }] dB. 2; 9.27 to 13.23; 100; -[{(PTx-23)+33}+4×{(¦Δf¦-9.27 MHz)/3.96 MHz }] dB. 3; 13.23 to  17.73; 100; -[{(PTx-23)+37}+2×{(¦Δf¦-13.23 MHz)/4.5 MHz }] dB. 4; 17.73 to  22.5; 100; -[(PTx-23)+39] dB. Notes: f is defined as the frequency offset in MHz from the center frequency of the 8.75 MHz channel. 2. PTx is the measured power in dBm into the antenna. 3. Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-21 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 1 Segment Number; ffset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <6; 100; -13. 2; 6 to <10; 1000; -13. 3; 10 to <11; 1000; -13-12(f -10). 4; 11 to <15; 1000; -25. 5; 15 to <20; 1000; -25. 6; 20 to ≤ 25; 1000; -25. Notes: - f is defined as the frequency offset in MHz from the center frequency of a 10MHz channel. Integration Bandwidth refers to the frequency range over which the emission power is integrated. AMS Band Class 3 – TDD TABLE 6-22 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port.. 1; 2.5 to <3.5; 50; -13. 2; 3.5 to <7.5; 1000; -13. 3; 7.5 to <8; 500; If PTx  +23 and (2547.5  fc  2622.5) then -23-2.28(∆f -7.5) else -16. 4; 8 to <10.4; 1000; -25. 5; 10.4 to  12.5; 1000; If PTx  +23 and (2547.5  fc  2622.5) then -21-1.68(∆f - 8) else -25. TABLE 6-23 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <6; 100; -13. 2; 6 to <10; 1000; -13. 3; 10 to <11; 1000; -13-12(f -10). 4; 11 to <15; 1000; -25. 5; 15 to <20; 1000; If PTx +23 dBm and (2550  fc  2620) then -21 - 32(f –10.5)/19 else -25. 6; 20 to  25; 1000; If PTx  +23 dBm and (2550  fc  2620) then -37 else -25. TABLE 6-24 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to <15; 1000; -13. 3; 15 to <16; 1000; -13-12(f -15). 4; 16 to  50; 1000; -25. AMS Band Class 3 - FDD TABLE 6-25 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port.. 1; 2.5 to <3.5; 50; -13. 2; 3.5 to <7.5; 1000; -13. 3; 7.5 to <8; 500; -16. 4; 8 to <10.4; 1000; -25. 5; 10.4 to  12.5; 1000; -25. TABLE 6-26 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <6; 100; -13. 2; 6 to <10; 1000; -13. 3; 10 to <11; 1000; -13-12(f -10). 4; 11 to <15; 1000; -25. 5; 15 to <20; 1000; -25. 6; 20 to  25; 1000; -25. - TABLE 6-27 - AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to <15; 1000; -13. 3; 15 to <16; 1000; -13-12(f -15). 4; 16 to  50; 1000; -25. AMS Band Class 5 – TDD TABLE 6-28 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 Frequency offset f (MHz); Maximum Emission Level (dBc); Integration Bandwidth. 2.5 to < 3.5; -33.5-15(∆f-2.5); 30 kHz. 3.5 to < 7.5; -33.5-1(∆f-3.5); 1 MHz. 7.5 to < 8.5; -37.5-10(∆f-7.5); 1 MHz. 8.5 to  12.5; -47.5; 1 MHz. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz. - The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth. - Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30 kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively. TABLE 6-29 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 Frequency offset f (MHz); Maximum Emission Level (dBc); Integration Bandwidth. 5.0 to <7.0; -33.5-9(∆f-5.0); 30 kHz. 7.0 to <15.0; -36.5-0.5(∆f-7.0); 1 MHz. 15.0 to <17.0; -40.5-5(∆f-15.0); 1 MHz. 17.0 to 25.0; -50.5; 1 MHz. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz. - The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth. - Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively. AMS Band Class 5 – FDD TABLE 6-30 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (3402.5 fc  3497.5) Frequency offset f (MHz); Maximum Emission Level (dBc); Integration Bandwidth. 2.5 to <3.5; -33.5-15(∆f-2.5); 30 kHz. 3.5 to <7.5; -33.5-1(∆f-3.5); 1 MHz. 7.5 to <8.5; -37.5-10(∆f-7.5); 1 MHz. 8.5 to 12.5; -47.5; 1 MHz. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz. - The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth. - Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30 kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively. TABLE 6-31 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (3405 fc  3495) Frequency offset f (MHz); Maximum Emission Level (dBc); Integration Bandwidth. 5.0 to <7.0; -33.5-9(∆f-5.0); 30 kHz. 7.0 to <15.0; -36.5-0.5(∆f-7.0); 1 MHz. 15.0 to <17.0; -40.5-5(∆f-15.0); 1 MHz. 17.0 to 25.0; -50.5; 1 MHz. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz. - The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth. - Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively. - AMS Band Class 6 – Sub-band 1710-1770/2110-2170 MHz TABLE 6-32 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz) Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port. 1; 2.5 to < 3.5; 50; -13. 2; 3.5 to  12.5; 1000; -13. - TABLE 6-33 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz) Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to < 6; 100; -13. 2; 6 to  25; 1000; -13. TABLE 6-34 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz) Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to 50; 1000; -13. - AMS Band Class 6 – Sub-bands other than 1710-1770/2110-2170 MHz TABLE 6-35 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <3.5; 50; -13. 2; 3.5 to < 7.5; 1000; -10. 3; 7.5 to <8.5; 1000; -13. 4; 8.5 to 12.5; 1000; -25. TABLE 6-36 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <6; 100; -13. 2; 6 to <10; 1000; -10. 3; 10 to <15; 1000; -13. 4; 15 to 25; 1000; -25. TABLE 6-37 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to <15; 1000; -10. 3; 15 to <30; 1000; -13. 4; 30 to 50; 1000; -25. - AMS Band Class 7 – TDD - TABLE 6-38 - AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 ≤fc  795.5) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <2.6; 30; -13. 2; 2.6 to 12.5; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz. TABLE 6-39 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (799.5 fc  859.5) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <7.5; 5; 1.6. 2; 7.5 to 12.5; 2; -10. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz. - TABLE 6-40 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc  793) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5.0 to <5.1; 30; -13. 2; 5.1 to 25.0; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz. TABLE: 6-41 AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (802 fc  857) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <10; 5; 1.6. 2; 10 to 25; 2; -10. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 24 MHz. - TABLE 6-42 - AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (703 fc  793) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10.0 to <10.1; 30; -13. 2; 10.1 to 50.0; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz. TABLE 6-43 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (807 fc  852) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <15; 5; 1.6. 2; 15 to 50; 2; -10. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 49 MHz. - AMS Band Class 7 – FDD - TABLE 6-44 - AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc < 834.5) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <2.6; 30; -13. 2; 2.6 to 12.5; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz. TABLE 6-45 AMS Spectrum Emission for 5 MHz Bandwidth Band Class 7 (834.5 fc  859.5) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <7.5; 5; 1.6. 2; 7.5 to 12.5; 2; -10. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz. - TABLE 6-46 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc < 837) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5.0 to <5.1; 30; -13. 2; 5.1 to 25.0; 100; -13. Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz. - TABLE 6-47 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (837 fc  857) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <10; 5; 1.6. 2; 10 to <15; 2; -10. 3; 15 to 25; 1; -25. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 14 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz. TABLE 6-48 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (708 fc < 842) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10.0 to <10.1; 30; -13. 2; 10.1 to 50.0; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 49.950 MHz. TABLE 6-49 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (842 fc  852) Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (MHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <15; 5; 1.6. 2; 15 to <20; 2; -10. 3; 20 to 50; 1; -25. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 19 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz. - AMS Band Class 8 TABLE 6-50 AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 8 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port. 1; 2.5 to < 3.5; 50; -13. 2; 3.5 to  12.5; 1000; -13. - TABLE 6-51 - AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 8 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to < 6; 100; -13. 2; 6 to  25; 1000; -13. TABLE 6-52 AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 8 Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <11; 200; -13. 2; 11 to <15; 1000; -10. 3; 15 to <30; 1000; -13. 4; 30 to 50; 1000; -25. ABS spectral masks: The spectrum masks in the following three tables (6-53, 6-54, 6-55) are applicable to all bands and all regions unless specific mask for a band or a country/region is specified. TABLE 6-53 ABS Spectrum Emission Mask for 5 MHz Bandwidth Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <7.5; 100; -7-7(∆f-2.55)/5. 2; 7.5 to 12.5; 100; -14. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-54 ABS Spectrum Emission Mask for 10 MHz Bandwidth Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <10; 100; -7-7(∆f-5.05)/5. 2; 10 to <15; 100; -14. 3; 15 to 25; 1000; -13. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 14.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-55 ABS Spectrum Emission Mask for 20 MHz Bandwidth Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <15; 100; -7-7(∆f-10.05)/5. 2; 15 to <20; 100; -14. 3; 20 to 50; 1000; -13. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 19.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. - ABS Band Class 1 TABLE 6-56 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 1-United States Segment Number; Offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to < 3.5; 50; 13. 2; 3.5 to  12.5; 1000; 13. TABLE 6-57 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 1-United States Segment Number; Offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to  6; 100; 13. 2; 6 to  25; 1000; 13. TABLE 6-57-1 ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, Ptx≥40 dBm Segment Number; Offset f from channel center (MHz); Measurement bandwidth (KHz); Allowed emission as measured at the antenna port. 1; 4.77 to  22.5; 100; -56.9 dBc. TABLE 6-57-2 ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, 29≤Ptx<40 dBm Segment Number; Offset f from channel center (MHz); Measurement bandwidth (KHz); Allowed emission as measured at the antenna port. 1; 4.77 to  22.5; 100; -53.9 dBc. TABLE 6-57-3 ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, Ptx≤29 dBm Segment Number; Offset f from channel center (MHz); Measurement bandwidth (KHz); Allowed emission as measured at the antenna port. 1; 4.77 to  22.5; 100; -14.5. - ABS Band Class 3 – TDD TABLE 6-58 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3 Frequency offset from centre; Allowed emission level; Integration Bandwidth. 2.5  f  3.5 MHz; 13 dBm; 50 kHz. 3.5  f  12.5 MHz; 13 dBm; 1 MHz. TABLE 6-59 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class Frequency offset from centre; Allowed emission level; Integration Bandwidth. 5  f  6 MHz; 13 dBm; 100 kHz. 6  f  25 MHz; 13 dBm; 1 MHz. TABLE 6-60 ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3 Frequency offset from centre; Allowed emission level; Integration Bandwidth. 10  f  11 MHz; 13 dBm; 200 kHz. 11  f  50 MHz; 13 dBm; 1 MHz. TABLE 6-61 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-Japan Frequency offset from centre; Allowed emission level; Integration Bandwidth. 7.5 MHz  f < 12.25; 151.4 × (f 7.5) dBm; 1 MHz. 12.25  f  22.5 MHz; 22 dBm; 1 MHz. TABLE 6-62 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-Japan Frequency offset from centre; Allowed emission level; Integration Bandwidth. 15  f  25 MHz; 22 dBm; 1 MHz. - ABS Band Class 3 – FDD TABLE 6-63 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3 - United States Frequency offset from centre; Allowed emission level; Integration Bandwidth. 2.5  f  3.5 MHz; 13 dBm; 50 kHz. 3.5  f  12.5 MHz; 13 dBm; 1 MHz. TABLE 6-64 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3 - United States Frequency offset from centre; Allowed emission level; Integration Bandwidth. 5  f  6 MHz; 13 dBm; 100 kHz. 6  f  25 MHz; 13 dBm; 1 MHz. TABLE 6-65 ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3 - United States Frequency offset from centre; Allowed emission level; Integration Bandwidth. 10  f  11 MHz; 13 dBm; 200 kHz. 11  f  50 MHz; 13 dBm; 1 MHz. - ABS Band Class 5 – TDD The Spectrum Emission Mask for 5, 10 and 20 MHz bandwidth sizes are specified in Tables 6-66 and 6-67. Table 6-66 specifies breakpoints of the underlying piecewise linear power spectral density mask. This mask is a relative mask and conditionally applicable depending on the ABS Pnom power level. Table 6-67 specifies the emission levels of an underlying piecewise step function applicable conditionally only to some of Pnom power levels. TABLE 6-66 ABS Relative Transmit Spectral Power Density Mask Power; Frequency Offset. 0.5*BW; 0.71*BW; 1.06*BW; 2.0*BW; 2.5*BW. 39 dBm  Pnom; -20 dB; -27 dB; -32 dB; -50dB; -50dB. 33 dBm  Pnom ≤39 dBm; -20 dB; -27 dB; -32 dB; -50 dB + (39 dBm - Pnom); Refer to Table 6-67. TABLE 6-67 ABS Absolute Spectral Emission Mask Power; Frequency Offset. 0.50 BW  f  0.71 BW; 0.71 BW  f  1.06 BW; 1.06 BW  f  2.00 BW; 2.00 BW  f  2.50 BW. 33 dBm  Pnom ≤ 39 dBm; Refer to Table 6-66; Refer to Table 6-66; Refer to Table 6-66; -21 + x dBm/MHz. Pnom ≤ 33 dBm; -5.5 dBm/MHz; -5.5 dBm/MHz; -23.5 dBm/MHz; -23.5 dBm/MHz. Notes: In Table 6-67, x = -10 log(BW/10). - ABS Band Class 6 – Sub-band 1710-1755/2110-2155 MHz The Spectrum Emission Mask of Table 6-68, Table 6-69, and Table 6-70 apply to the United States. TABLE 6-68 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz) Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/integration BW) at the antenna port.. 1; 2.5 to < 3.5; 50; -13. 2; 3.5 to  12.5; 1000; -13. TABLE 6-69 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz) Segment Number; Offset from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to < 6; 100; -13. 2; 6 to  25; 1000; -13. TABLE 6-70 ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz) Frequency offset from centre; Allowed emission level; Integration Bandwidth. 10  f  11 MHz; 13 dBm; 200 kHz. 11  f  50 MHz; 13 dBm; 1 MHz. - ABS Band Class 7 The Spectrum Emission Mask of Table 6-71, Table 6-72, and Table 6-73 apply to the United States. - TABLE 6-71 - ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 - United States Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to < 2.6; 30; -13. 2; 2.6 to 12.5; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz. - TABLE 6-72 - ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 - United States Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5.0 to < 5.1; 30; -13. 2; 5.1 to  25.0; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz. - TABLE 6-73 - ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 - United States Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10.0 to <10.1; 30; -13. 2; 10.1 to  50.0; 100; -13. - Notes: - f is the separation between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz. The Spectrum Emission Mask of Table 6-74, Table 6-75, and Table 6-76 apply to Europe. TABLE 6-74 ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 - Europe Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 2.5 to <7.5; 100; -7-7(∆f-2.55)/5. 2; 7.5 to 12.5; 100; -14. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-75 ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 - Europe Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 5 to <10; 100; -7-7(∆f-5.05)/5. 2; 10 to <15; 100; -14. 3; 15 to 25; 100; -13. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 24.95 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. TABLE 6-76 ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 - Europe Segment Number; Frequency offset f from channel center (MHz); Integration Bandwidth (kHz); Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port. 1; 10 to <15; 100; -7-7(∆f-10.05)/5. 2; 15 to <20; 100; -14. 3; 20 to 50; 100; -13. Notes: - f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter. - The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 49.95 MHz. - Integration Bandwidth refers to the frequency range over which the emission power is integrated. |
| 4.2.3.2.23.6 | Describe any MS power saving mechanisms used in the RIT. The proposed RIT provides AMS power management functions including sleep mode and idle mode to reduce AMS battery consumption (see item 4.2.3.2.6.3). Sleep mode is a state in which an AMS conducts pre-negotiated periods of absence from the S-ABS air interface. Per AMS, a single power saving class is managed in order to handle all the active connections of the AMS. Sleep mode may be activated when an AMS is in the connected state. When sleep mode is active, the AMS is provided with a series of alternate listening window and sleep windows. The listening window is the time in which the AMS is available to exchange control signaling as well as data between itself and the ABS. The proposed RIT provides a framework for dynamically adjusting the duration of sleep windows and listening windows within a sleep cycle based on changing traffic patterns and HARQ operations. The length of successive sleep windows may remain constant or may be adaptive based on traffic conditions. Sleep windows and listening windows can be dynamically adjusted for the purpose of data transportation as well as MAC control signaling transmission. AMS can send and receive data and MAC control signaling without deactivating the sleep mode. Idle mode provides efficient power saving for the AMS by allowing the AMS to become periodically available for DL broadcast traffic messaging (e.g. Paging message) without registration at a specific ABS. The network assigns idle mode AMS to a paging group during idle mode entry or location update. The design allows the network to minimize the number of location updates performed by the AMS and the paging signaling overhead caused to the ABSs. The idle mode operation considers user mobility. ABSs and idle mode AMSs may belong to one or multiple paging groups in order to minimize the number of location updates and paging load without increasing average paging delay and without increasing the overhead of transmitting of multiple paging IDs by the ABSs. Idle mode AMSs may be assigned paging groups of different sizes and shapes based on user mobility. The AMS monitors the paging message at AMS’s paging listening interval. The start of the AMS’s paging listening interval is derived based on paging cycle and paging offset. Paging offset and paging cycle are defined in terms of number of superframes. |
| 4.2.3.2.23.7 | Simulation process issues Describe the methodology used in the analytical approach. Proponent should provide information on the width of confidence intervals of user and system performance metrics of corresponding mean values, and evaluation groups are encouraged to provide this information as requested in § 7.1 of Report ITU-R M.2135. In the case of the InH test environment, 200 independent drops are simulated. Since each drop generates results for 20 users (10 users in each of 2 omnidirectional cells), statistics are generated over 200*20 = 4,000 user positions. In the case of the UMi, UMa, and RMa test environments, 10 independent drops are simulated. Since each drop generates results for 570 users (10 users in each of 57 sectors), statistics are generated over 10*570 = 5,700 user positions. Please refer to Section 7, Self-evaluation report for detailed description of simulation methodology. The confidence interval can be calculated for each test environment based on the above data provided that additional assumptions are made on parameters such as the level of confidence, status of a priori knowledge on factors such as mean and standard deviation, etc. |
| 4.2.3.2.24 | Other information Please provide any additional information that the proponent believes may be useful to the evaluation process. The proposed RIT supports femto-cell operations as an overlay in macro-cell deployments. See Section 15 of {4} for more information. A femto ABS is an ABS with low transmit-power, typically installed by a subscriber in home or SOHO to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A femto ABS is connected to the service provider’s network via broadband connection (such as DSL, or cable). Femto ABS typically operates in licensed spectrum and may use the same or different frequency as macro-cells. Their coverage may overlap with macro ABS. A femto ABS may belong to one of the following types. - Closed Subscriber Group (CSG): A CSG femto ABS is accessible only to the AMSs, which are member of the CSG, except for emergency services. AMSs which are not the members of the CSG, should not try to access CSG femto ABSs. The member of the CSG can be modified by the service level agreement between the subscriber and the access provider. - Open Subscriber Group (OSG): An OSG femto ABS is accessible to any AMSs. Femto ABSs and macro ABSs are differentiated using Cell IDs, which is obtained from the preambles. This enables AMSs to quickly identify cells types, avoid too frequent handover attempts into and out of a femto ABS, and avoid performing unnecessary network entry/re-entry. CSG and OSG femto ABSs are differentiated using MAC level identifiers to help an AMS determine its designated femto ABSs vs. other femto ABSs based on which it can apply necessary rules and procedures for network entry and handover in a timely fashion. A femto ABS synchronizes with the network to a common timing and frequency signal. A femto ABSs may use different schemes to achieve synchronization with the network to handle various deployment scenarios. A femto ABSs may synchronize with the overlay ABS’s A-PREAMBLE to automatically adjust its DL synchronization. A femto ABS may maintain synchronization with the overlay ABSs over the air. See Section 15 of {4} for more details on femto ABS support in the proposed RIT. |

- 6.2	Description template – link budget

Guidelines of ITU-R M.2135 {7} have been followed in development of link budget tables in this section.

In addition, the following assumptions have been used:

- Number of transmit/receive antennas

- ABS: 4 transmit antennas and 4 receive antennas

- AMS: 2 transmit antennas and 2 receive antennas

- Target packet error rate

- Data channel: 10% (for initial transmission)

- Control channel: 1%

- Control channel

- DL: A-A-MAP with QPSK 1/8

- UL: 6bit P-FBCH

- MIMO transmission scheme

- DL data: rank-1 wideband beamforming

- DL control: SFBC with non-adaptive precoder

- UL data: rank-1 wideband beamforming

- UL control: single transmit antenna transmission

- Permutation (subchannelization)

- DL/UL data: miniband Contiguous Resource Unit (CRU)

- DL control: DRU

- UL control: P-FBCH

- Pilot boosting: 2dB pilot boosting over data tone for DL and no pilot boosting for UL

- Transmitter array gain is included in required SNR

- No HARQ is assumed for control channel

- 0.5 dB HARQ combining gain for data channel

- Shadowing fade margin

- Shadowing fade margin is determined as a function of the cell edge coverage reliability and the standard deviation of the log-normal shadow fading (including penetration losses, if applicable). Cell edge coverage reliability is determined for the given area coverage reliability as a function of the shadow fading standard deviation and the path loss exponent obtained from the pathloss model. The cell edge reliability can be determined using simulations or using traditional numerical methods.

Table 6-77

Shadowing fade margin information

|  | InH NLoS | UMi NLoS | UMa NLoS (in-car coverage) | RMa NLoS (in-car coverage) |
| --- | --- | --- | --- | --- |
| Shadow fading standard deviation | 4 dB | 4 dB | 7.8 dB | 9.4 dB |
| Pathloss exponent | 4.33 | 3.67 | 3.91 | 3.86 |
| Shadowing fade margin | 2.8 dB | 3.1 dB | 8.1 dB | 10.4 dB |

Below are link budget tables for all test environments; TDD indoor hotspot in Table 6-78, FDD indoor hotspot in Table 6-79, TDD urban micro-cell in Table 6-80, FDD urban micro-cell in Table 6-81, TDD urban macro-cell in Table 6-82, FDD urban macro-cell in Table 6-83, TDD rural macro-cell in Table 6-84 and FDD rural macro-cell in Table 6-85.

TABLE 6-78

Link budget template for indoor test environment 
(indoor hotspot deployment scenario) - TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 3.4 | 3.4 |
| BS antenna heights (m) | 6 | 6 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 20,230,593 | 980,290 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.856 | 0.830 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 18 | 18 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 24 | 21 |
| (4) Transmitter antenna gain (dBi) | 0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 21 | 20 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 20.73 | 20 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | –174 | –174 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -166.21 | -167.81 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 37808640 | 3150720 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.43 | -102.82 |
| (19a) Required SNR for the control channel (dB) | -0.56 | -2.48 |
| (19b) Required SNR for the data channel (dB) | 1.41 | -0.24 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.00 | -103.30 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -87.52 | -101.57 |
| (23a) Hardware link budget for control channel = (9a) + (11) – (22a) dB | 110.00 | 123.30 |
| (23b) Hardware link budget for data channel = (9b) + (11) – (22b) dB | 108.25 | 121.57 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 2.8 | 2.8 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 106.20 | 117.50 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 104.45 | 115.77 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 87.39 | 159.42(4) |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 79.65 | 145.37 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 23993.85 | 79846.00 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 19929.95 | 66391.76 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. (4) InH NLoS pathloss model is only defined for less than 150m. Even though the maximum range for uplink control channel exceeds 150m, same pathloss model is used. |  |  |

TABLE 6-79

Link budget template for indoor test environment 
(indoor hotspot deployment scenario) - FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 3.4 | 3.4 |
| BS antenna heights (m) | 6 | 6 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 16,955,780 | 2,520,745 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.897 | 0.800 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 15 | 18 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 21 | 21 |
| (4) Transmitter antenna gain (dBi) | 0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 18 | 20 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 17.73 | 20 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | –174 | -174 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -166.21 | -167.81 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 3150720 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.44 | -102.82 |
| (19a) Required SNR for the control channel (dB) | -0.56 | -2.48 |
| (19b) Required SNR for the data channel (dB) | 1.23 | -0.50 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.01 | -103.30 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -90.71 | -101.82 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 110.01 | 123.30 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 108.44 | 121.82 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 2.8 | 2.8 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 106.21 | 117.50 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 104.64 | 116.02 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 87.44 | 159.42(4) |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 80.46 | 147.37 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 24020.14 | 79846.00 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 20337.43 | 68224.73 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. (4) InH NLoS pathloss model is only defined for less than 150m. Even though the maximum range for uplink control channel exceeds 150m, same pathloss model is used. |  |  |

TABLE 6-80

Link budget template for microcellular test environment 
(urban micro-cell deployment scenario) - TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.5 | 2.5 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 38 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 44 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 58 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 57.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.57 | -4.10 |
| (19b) Required SNR for the data channel (dB) | -1.05 | -0.96 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.68 | -107.37 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -89.66 | -104.73 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 147.68 | 147.37 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 147.39 | 144.73 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 3.10 | 3.10 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 143.58 | 141.27 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 143.29 | 138.63 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 1030.29 | 891.56 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 1011.89 | 755.50 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 3334807.34 | 2497180.50 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 3216769.97 | 1793159.90 |
| (1) Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin. (2) The spectral efficiency of the chosen modulation scheme. (3) The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135. |  |  |

TABLE 6-81

Link budget template for microcellular test environment 
(urban micro-cell deployment scenario) - FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.5 | 2.5 |
| BS antenna heights (m) | 10 | 10 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 3 | 3 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 35 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 41 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 55 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 54.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.00 | -166.00 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.57 | -4.10 |
| (19b) Required SNR for the data channel (dB) | -1.68 | -1.14 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.69 | -107.37 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -93.30 | -104.91 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 147.69 | 147.37 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 148.03 | 144.91 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 3.10 | 3.10 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 0 | 0 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 143.59 | 141.27 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 143.93 | 138.81 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 1030.96 | 891.56 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 1053.45 | 763.94 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 3339120.21 | 2497180.50 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 3486434.72 | 1833460.41 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

TABLE 6-82

Link budget template for base coverage urban test environment 
(urban macro-cell deployment scenario) - TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2 | 2 |
| BS antenna heights (m) | 25 | 25 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 30 | 30 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 43 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 49 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 63 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 62.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.95 | -3.97 |
| (19b) Required SNR for the data channel (dB) | -0.21 | -0.82 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -90.06 | -107.24 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -88.82 | -104.59 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 153.06 | 147.24 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 151.55 | 144.59 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.8 | 7.8 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 8.1 | 8.1 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 134.96 | 127.14 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 133.45 | 124.49 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 891.93 | 562.90 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 816.07 | 481.56 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 2499264.75 | 995447.89 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 2092190.06 | 728546.98 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

TABLE 6-83

Link budget template for base coverage urban test environment 
(urban macro-cell deployment scenario) - FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 2 | 2 |
| BS antenna heights (m) | 25 | 25 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 30 | 30 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 40 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 46 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 60 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 59.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.95 | -3.97 |
| (19b) Required SNR for the data channel (dB) | -0.24 | -0.59 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -93.07 | -107.24 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -91.86 | -104.36 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 153.07 | 147.24 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 151.59 | 144.36 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.8 | 7.8 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 8.1 | 8.1 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 134.97 | 127.14 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 133.49 | 124.26 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 892.47 | 562.90 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 817.87 | 475.03 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 2502298.50 | 995447.89 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 2101435.73 | 708919.02 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

TABLE 6-84

Link budget template for high speed test environment 
(rural macro-cell deployment scenario) – TDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 0.8 | 0.8 |
| BS antenna heights (m) | 35 | 35 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 7,529,917 | 212,755 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.637 | 0.720 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 120 | 120 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 43 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 49 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 63 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 62.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 18904320 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -90.11 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.19 | -2.42 |
| (19b) Required SNR for the data channel (dB) | -0.70 | 0.37 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -89.30 | -105.69 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -89.31 | -103.40 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 152.30 | 145.69 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 152.04 | 143.40 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 9.4 | 9.4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 10.4 | 10.4 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 131.90 | 123.29 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 131.64 | 121.00 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 2361.39 | 1412.39 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 2323.96 | 1232.23 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 17517973.59 | 6267014.62 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 16967146.48 | 4770131.41 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

TABLE 6-85

Link budget template for high speed test environment 
(rural macro-cell deployment scenario) – FDD

| Item | Downlink | Uplink |
| --- | --- | --- |
| System configuration |  |  |
| Carrier frequency (GHz) | 0.8 | 0.8 |
| BS antenna heights (m) | 35 | 35 |
| UT antenna heights (m) | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 95% | 95% |
| Transmission bit rate for control channel (bit/s) | 89,600 | 1,200 |
| Transmission bit rate for data channel (bit/s) | 5,849,421 | 611,334 |
| Target packet error rate for the required SNR in item (19a) for control channel | 10-2 | 10-2 |
| Target packet error rate for the required SNR in item (19b) for data channel | 10-1 | 10-1 |
| Spectral efficiency(2) (bit/s/Hz) for data | 0.619 | 0.776 |
| Pathloss model(3) (select from LoS or NLoS) | NLoS | NLoS |
| Mobile speed (km/h) | 120 | 120 |
| Feeder loss (dB) | 2 | 2 |
| Transmitter |  |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 4 | 2 |
| (2) Maximal transmit power per antenna (dBm) | 40 | 21 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in Table 6 of Report ITU-R M.2135) | 46 | 24 |
| (4) Transmitter antenna gain (dBi) | 17 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) | 0 | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.2734 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3 | 1 |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm | 60 | 23 |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm | 59.73 | 23 |
| Receiver |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in Table 6 of Report ITU-R M.2135) | 2 | 4 |
| (11) Receiver antenna gain (dBi) | 0 | 17 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | –174 | –174 |
| (15) Receiver interference density (dBm/Hz) | -165 | -166 |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -162.88 | -164.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 9452160 | 787680 |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -93.12 | -105.27 |
| (19a) Required SNR for the control channel (dB) | -1.19 | -2.42 |
| (19b) Required SNR for the data channel (dB) | -1.06 | 1.03 |
| (20) Receiver implementation margin (dB) | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0.5 | 0.5 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -92.31 | -105.69 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -92.68 | -102.74 |
| (23a) Hardware link budget for control channel = (9a) + (11) − (22a) dB | 152.31 | 145.69 |
| (23b) Hardware link budget for data channel = (9b) + (11) − (22b) dB | 152.40 | 142.74 |
| Calculation of available pathloss |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 9.4 | 9.4 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 10.4 | 10.4 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 |
| (27) Penetration margin (dB) | 9 | 9 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB | 131.91 | 123.29 |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB | 132.00 | 120.34 |
| Range/coverage efficiency calculation |  |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) | 2362.84 | 1412.39 |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) | 2375.49 | 1184.79 |
| (31a) Coverage Area for control channel = (π (30a)2) (m2/site) | 17539513.53 | 6267014.62 |
| (31b) Coverage Area for data channel = (π (30b)2) (m2/site) | 17727874.98 | 4409968.99 |

(1) 	Cell area reliability is defined as the percentage of the cell area over which coverage can be guaranteed. It is obtained from the cell edge reliability, shadow fading standard deviation and the path loss exponent. The latter two values are used to calculate a fade margin. Macro diversity gain may be considered explicitly and improve the system margin or implicitly by reducing the fade margin.

(2) 	The spectral efficiency of the chosen modulation scheme.

(3)  The pathloss models are summarized in TABLE A1-2 of Report ITU-R M.2135.

Annex 1

L1/L2 overhead calculation

See attached embedded spread sheet.

Annex 2

IEEE 802.16m system description document

See attached embedded document.

PART 4

# 7	Self-evaluation report

IEEE confirms that the proposed RIT meets all the requirements for IMT-Advanced as specified by the service related minimum capabilities (Section 4.2.4.1), the spectrum capability requirements (Section 4.2.4.2), and all minimum technical requirements (Section 4.2.4.3) of Report ITU-R M.2133 {5}.

## 7.1	Overview

This section provides the details of the proposed RIT’s evaluation results for the completion of the compliance template for technical performance in Report ITU-R M.2133 {5}. The performance evaluation results are presented for the entries of cell spectral efficiency (Section 7.2.1), cell edge user spectral efficiency (Section 7.2.1), number of supported VoIP users (Section 7.2.2), mobility traffic channel link data rates (Section 7.2.3), peak spectral efficiency (Section 7.2.4), control plane latency (Section 7.2.5), user plane latency (Section 7.2.5), intra-frequency hand-over interruption time (Section 7.2.5), inter-frequency handover interruption time within a spectrum band (Section 7.2.5), and inter-frequency handover interruption time between spectrum bands (Section 7.2.5). The evaluation results for other entries in the compliance template for technical performance, i.e. bandwidth, mobility classes, and inter-system handover, are provided in entries of comments in its compliance template in Section 8.3. The evaluation results for the completions of the compliance template for services and compliance template for spectrum in Report ITU-R M.2133 {5} are presented in entries of evaluator’s comments and spectrum capability requirements in each compliance template in Sections 8.1 and 8.2, respectively.

All results for the completion of the compliance template for technical performance were generated based on both Report ITU-R M.2135 {7} and IMT-ADV/3 following ITU-R guidelines. Evaluation assumptions used for evaluating the performance of the RIT through simulations that are in addition to those in {7} are based on {4} and {6} and are outlined in Annex 3. Any additional simulation assumptions not explicitly specified in Annex 3 are presented in each relevant section below. System-level simulation calibration information is contained in {14}.

All final results obtained through evaluations in this section are reflected in the compliance template for technical performance in Section 8.3.

## 7.2	Evaluation results

### 7.2.1	Cell spectral efficiency and cell-edge user spectral efficiency

### 7.2.1.1	Simulation assumptions and parameters for full-buffer data traffic

The simulation assumptions and configuration parameters used in simulations for full-buffer data traffic are described in the tables in Annex 3. Compared to the tables in Annex 3, the following modifications/additions are made for full-buffer data traffic in FDD/TDD operations:

# DL configuration

- Subchannelization for data allocations: In the case of the InH and UMi, Scheme-1 (subband-based CRU subchannelization) was selected in the simulations. In the case of UMa and RMa, Scheme-2 (miniband-based CRU subchannelization) was simulated.

- Multi-antenna Transmission Format for data: In the case of InH and UMi, the 6-bit transformed codebook based MU-MIMO scheme was simulated using the 4 x 2 configuration, i.e., adaptive switching among rank-1, rank-2, rank-3, and rank-4 transmission. In the case of UMa and RMa, MU-MIMO with long term beamforming was simulated using 4 x 2 configuration, i.e., adaptive switching among rank-1, rank-2, rank-3, and rank-4 transmission.

- Receiver Structure: The MMSE receiver was simulated for the AMS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with maximum retransmission delay of four frames was used for the simulations.

- Control signalling: Signalling errors were modelled for A-A-MAP and HF-A-MAP, and sounding estimation errors were modelled for A-MIDAMBLE.

- Control Channel Overhead: Dynamic overhead modelling for A-A-MAP and HF-A-MAP and fixed overhead modelling for non-user specific A-MAP (NUS-A-MAP), A-PREAMBLE, A-MIDAMBLE, and SFH are used in this contribution. Detailed calculation of the DL control channel overhead for each test environment is shown in Section 7.2. 1.2

UL configuration

- Subchannelization for data allocations: In the case of the InH and UMi, Scheme-1 (subband-based CRU subchannelization) was selected in the simulations. In the case of UMa and RMa, Scheme-2 (miniband-based CRU subchannelization) was simulated.

- Multi-antenna transmission format for data: In the case of the InH and UMi, the 3-bit codebook-based MU-MIMO scheme was selected with adaptive switching between a single user and collaborative spatial multiplexing using 2x4 antenna configuration. In the case of UMa and RMa, the MU-MIMO scheme with long-term beamforming and adaptive switching between a single user and collaborative spatial multiplexing was used in the simulations using 2x4 antenna configuration.

- Receiver structure: The MMSE receiver was simulated for the ABS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with retransmission delay of four frames was used for the simulations.

- Control signalling: Signalling errors were modeled for Primary Feedback Control Channel (P-FBCH) (UMa, RMa), Secondary Feedback Control Channel (S-FBCH) (InH, UMi), and HARQ Feedback Control Channel (H-FBCH), and sounding estimation errors were modelled for the Sounding channel.

- Control Channel Overhead: Dynamic overhead modeling for P-FBCH, S-FBCH and H-FBCH and fixed overhead modeling for Long-Term Covariance Matrix (LT-CM), Initial Ranging (IR), and Bandwidth Request (BW-REQ) are adopted. Detailed calculation of the UL control channel overhead for each test environment is shown in Section 7.2.1.2.

# 7.2.1.2	Control channel overhead calculations

In this section, details of the control overhead calculations for DL and UL are presented.

# DL calculations

The following assumptions were made for the DL control channels:

- A-A-MAP: The A-A-MAP control overhead is dynamically calculated based on the scheduler allocations in each simulated frame of both DL and UL in each test environment. The average DL A-A-MAP and UL A-A-MAP overhead is accounted for in the estimation of the cell spectral efficiency and cell-edge user spectral efficiency.

- HF-A-MAP: The overhead calculation for the HF-A-MAP channel is based on the dynamic calculation of the required ACKs/NACKs from the UL system level simulations for each test environment.

- A-PREAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed for both TDD and FDD.

- A-MIDAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed for both TDD and FDD.

- SFH: Fixed overhead of 20 Logical Resource Unit (LRU)s per superframe is assumed for both FDD and TDD.

Table 7-1 contains the summary of the DL control channel overhead calculations for TDD and FDD.

TABLE 7-1

Summary of DL total control channel overhead percentage per mode 
(TDD/FDD) and test environment (InH, UMi, UMa, RMa)

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| TDD | 9.19% | 12.33% | 11.17% | 11.15% |
| FDD | 9.74% | 16.28% | 13.77% | 13.63% |

# UL calculations

The following assumptions were made for the UL control channels:

- P-FBCH: The UL control overhead of the P-FBCH is calculated according to the P-FBCH reporting period of 5 ms for UMa and RMa, and by assuming that all 10 users/sector transmit the PFBCH in each 5 ms frame.

- S-FBCH/UL MAC management: The feedback of CQI/PMI is done either through S-FBCH or UL MAC management messages. In both cases, the reporting period is 5 ms for both TDD and FDD.

- H-FBCH: The overhead of the H-FBCH was based on the dynamic calculation of the required ACKs/NACKs from the DL system level simulations for each test environment.

- LT-CM: Although the feedback of the LT-CM is not assumed to be received through a dedicated UL control channel but through a regular UL MAC management burst, its overhead is included in the calculation of the UL control overhead. Fixed overhead of the LT-CM feedback is assumed assuming a reporting period of 20 ms for both UMa and RMa.

- Sounding: To enable the required sounding capabilities for both FDD and TDD, fixed overhead of 1 symbol per long TTI for both TDD and FDD is assumed.

- IR: Fixed overhead of 4 LRUs per superframe is assumed for both TDD and FDD.

- BW-REQ: Fixed overhead of 4 LRUs per superframe is assumed for both TDD and FDD.

Table 7-2 contains the summary of the UL control channel overhead calculations for TDD and FDD.

TABLE 7-2

Summary of UL total control channel overhead percentage per mode 
(TDD/FDD) and test environment (InH, UMi, UMa, RMa)

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| TDD | 7.85% | 12.60% | 9.23% | 8.34% |
| FDD | 6.02% | 10.58% | 8.01% | 6.51% |

# 7.2.1.3	Cell spectral efficiency and cell edge user spectral efficiency results

In Section 7.2.1.3.1, DL cell spectral efficiency and cell edge user spectral efficiency results obtained from performance evaluation of the RIT are presented for both TDD and FDD. The corresponding numbers for the UL are presented in Section 7.2.1.3.2.

# 7.2.1.3.1	DL results

Table 7-3 and Table 7-4 present the TDD DL cell spectral efficiency and cell-edge user spectral efficiency, respectively, for all test environments. Table 7-5 and Table 7-6 present the corresponding numbers for FDD. In each table, the ITU-R requirement for each test environment is also shown.

TABLE 7-3

DL cell spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.93 | 3.22 | 2.41 | 3.23 |
| ITU-R requirement | 3.0 | 2.6 | 2.2 | 1.1 |

TABLE 7-4

DL cell edge user spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.260 | 0.092 | 0.069 | 0.093 |
| ITU-R requirement | 0.1 | 0.075 | 0.06 | 0.04 |

TABLE 7-5

DL cell spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.87 | 3.27 | 2.41 | 3.15 |
| ITU-R requirement | 3.0 | 2.6 | 2.2 | 1.1 |

TABLE 7-6

DL cell edge user spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.253 | 0.097 | 0.069 | 0.091 |
| ITU-R requirement | 0.1 | 0.075 | 0.06 | 0.04 |

# 7.2.1.3.2	UL results

Table 7-7 and Table 7-8 present the TDD UL cell spectral efficiency and cell edge user spectral efficiency, respectively, for all test environments. Table 7-9 and table 7-10 present the corresponding numbers for FDD. In each table, the ITU-R requirements for each test environment are also shown.

TABLE 7-7

UL cell spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 5.99 | 2.58 | 2.57 | 2.66 |
| ITU-R requirement | 2.25 | 1.8 | 1.4 | 0.7 |

TABLE 7-8

UL cell edge user spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.426 | 0.111 | 0.109 | 0.119 |
| ITU-R requirement | 0.07 | 0.05 | 0.03 | 0.015 |

TABLE 7-9

UL cell spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.23 | 2.72 | 2.69 | 2.77 |
| ITU-R requirement | 2.25 | 1.8 | 1.4 | 0.7 |

TABLE 7-10

UL cell edge user spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.444 | 0.119 | 0.114 | 0.124 |
| ITU-R requirement | 0.07 | 0.05 | 0.03 | 0.015 |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced cell spectral efficiency and cell edge spectral efficiency. See Annex 4 for supporting details on the results presented above.

# 7.2.2	VoIP capacity

# 7.2.2.1	Simulation assumptions and parameters for VoIP traffic

The modeling assumptions and configurations used in the system level simulations for VoIP are as described in tables in Annex 3. Additional details relevant to system level simulations of the RIT for VoIP are listed in this section. In relations to information in Annex 3, the following items are specific to VoIP traffic:

DL configuration

- Subchannelization for VoIP allocations: A mix of DRU and miniband CRU subchannelization schemes is used in the simulations. Since the control channels are always allocated in distributed resources, a fixed partition is used in each subframe between the DRU and miniband CRU resources.

- Multi-antenna transmission format for VoIP: In the case of DRU, SFBC with non-adaptive precoding is used. In the case of miniband CRU, rank-1 tranmission with wideband beamforming is used.

- Receiver structure: The MMSE receiver was simulated for the AMS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with maximum retransmission delay of 4 frames was used.

- Control channel overhead:

  - A-MAP Region: A fixed overhead is assumed for NUS-A-AMAP and HF-A-MAP. The A-A-MAP control overhead is explicitly simulated based on the scheduler allocations in each simulated subframe on both DL and UL in each test environment.

  - A-PREAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed.

  - A-MIDAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed. Midamble estimation errors are included in the presented results.

  - SFH: 20 LRUs are reserved for the SFH in the first subframe of every superframe.

- Control channel signalling errors: Explicitly modelled in the VoIP simulations for both FDD and TDD.

- Persistent scheduling: Persistent scheduling for individual VoIP connections as described in {6} is modelled. The MAC procedure and dynamic control overhead based on the persistent allocation A-MAP IE for initial transmissions is modelled. HARQ retransmissions are not scheduled persistently.

- Control channel overhead: A dynamic overhead model for the A-A-MAP and SFH and a fixed overhead model for the other DL control channels are used.

UL configuration

- Subchannelization for VoIP allocations: DRU-based subchannelization is used.

- Multi-antenna transmission format for VoIP: Open loop SFBC with a 2x4 antenna configuration is used.

- Receiver structure: The MMSE receiver was simulated for the ABS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with retransmission delay of four frames was used for the simulations.

- Control signalling:

  - P-FBCH: The P-FBCH is utilized for feeding back CQI information. Additionally, when wideband beamforming is used, wideband PMI is also fed back. UL control overhead induced by the P-FBCH is calculated in a fixed manner according to the reporting period derived from the number of active users, voice activity factor, and duration of persistence for persistent scheduling.

  - H-FBCH: The overhead from the H-FBCH was based on the maximum number (30) of required ACKs/NACKs in each subframe from the DL system level VoIP simulations for each test environment.

  - Sounding channel: To facilitate sounding for 16 users per frame when the system is operating at the maximum VoIP capacity, a fixed overhead of 2 OFDMA symbols per frame is assumed.

  - IR: Fixed overhead of 1 LRU per frame is used.

  - BW-REQ channels: Fixed overhead of 1 LRU per frame is used.

- Control channel signalling errors: Explicitly modelled in the VoIP simulations for both FDD and TDD.

  - Persistent scheduling: Persistent scheduling for individual VoIP connections as described in {6} is modelled. The MAC procedure and dynamic control overhead based on the persistent allocation A-MAP IE for initial transmissions is modelled. HARQ retransmissions are not scheduled persistently.

  - Control channel overhead: A fixed overhead model for the UL control channels is used.

# 7.2.2.2	VoIP capacity results

Table 7-11 and Table 7-12 summarize the VoIP capacity results for the RIT based on the assumptions of Section 7.2.2.1.

TABLE 7-11

VoIP capacity (users/sector/MHz) for TDD

| Test environment | DL | UL | Minimum {DL, UL} | ITU-R required |
| --- | --- | --- | --- | --- |
| Indoor (InH) | 140 | 165 | 140 | 50 |
| Microcellular (UMi) | 82 | 104 | 82 | 40 |
| Base coverage urban (UMa) | 74 | 95 | 74 | 40 |
| High speed (RMa) | 89 | 103 | 89 | 30 |

TABLE 7-12

VoIP capacity (users/sector/MHz) for FDD

| Test environment | DL | UL | Minimum {DL, UL} | ITU-R required |
| --- | --- | --- | --- | --- |
| Indoor (InH) | 139 | 166 | 139 | 50 |
| Microcellular (UMi) | 77 | 102 | 77 | 40 |
| Base coverage urban (UMa) | 72 | 95 | 72 | 40 |
| High speed (RMa) | 90 | 101 | 90 | 30 |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced VoIP capacity. See Annex 4 for supporting details on the results presented above.

# 7.2.3	Mobility requirements

Following the evaluation methodology for the mobility requirement {7}, in a first step UL system-level simulations were run for each test environment to obtain the UL SINR distribution. From the UL SINR distribution the median (or 50th percentile) value is obtained for each test environment. In a second step, link-level simulations were run to generate the spectral efficiency versus SNR curves for each test environment. The generated spectral efficiency numbers include the impact of control channel overhead by accounting for the average control channel overhead from the UL system-level simulations for each test environment. To assess whether the RIT meets the mobility requirement, the spectral efficiency value from the link-level curves is obtained for the median SINR value from the system-level simulations.

Table 7-13 and Table 7-14 summarize the results for TDD and FDD, respectively. It is noted that the link-level curves were generated for the NLOS channel models of each test environment.

TABLE 7-13

Mobility requirement data for TDD

| Test environment | Median SINR (in dB) | Achieved spectral efficiency (in bit/s/Hz) LoS | Achieved spectral efficiency (in bit/s/Hz) NLoS | ITU-R Required spectral efficiency (in bit/s/Hz) |
| --- | --- | --- | --- | --- |
| InH (10 km/h) | 16.6 | 3.76 | 3.41 | 1.0 |
| UMi (30 km/h) | 5.0 | 1.81 | 1.50 | 0.75 |
| UMa (120 km/h) | 4.3 | 1.72 | 1.30 | 0.55 |
| RMa (350 km/h) | 5.6 | 1.70 | 1.23 | 0.25 |

TABLE 7-14

Mobility requirement data for FDD

| Test environment | Median SINR (in dB) | Achieved spectral efficiency (in bit/s/Hz) LoS | Achieved spectral efficiency (in bit/s/Hz) NLoS | ITU-R Required spectral efficiency (in bit/s/Hz) |
| --- | --- | --- | --- | --- |
| InH (10 km/h) | 16.6 | 3.86 | 3.56 | 1.0 |
| UMi (30 km/h) | 5.0 | 1.72 | 1.51 | 0.75 |
| UMa (120 km/h) | 4.3 | 1.63 | 1.34 | 0.55 |
| RMa (350 km/h) | 5.6 | 1.61 | 1.27 | 0.25 |

Therefore, IEEE confirms that the proposed RIT meets the IMT-Advanced mobility requirements. See Annex 4 for supporting details on the results presented above.

# 7.2.4	Peak spectral efficiency

This section shows the methodology to calculate the peak spectral efficiency and its results for the proposed RIT. The analysis follows the guideline and definition provided in Reports ITU-R M.2134 {8} and Report ITU-R M.2135 {7}. The FDD and TDD modes of the RIT are considered and the case for the channel bandwidth of 20 MHz TDD and 2x20 MHz FDD with the FFT size of 2048 and the size of cyclic prefix (CP) of 1/16 both in DL and UL are addressed to calculate the peak data rate. In addition, 4 and 2 streams are considered in DL and UL, respectively, based on the constraints of antenna configurations specified in {8} and {7}.

One OFDMA symbol with the length of 97.143 µs for preamble exists in each frame with the length of 5 ms and the idle time of 45.71 µs is allocated at the end of each frame. The FDD frame has 5 type-1 subframes, consisting of 6 OFDMA symbols, and 3 type-2 subframes, consisting of 7 OFDMA symbols. The TDD frame has 3 type-1 subframes in DL, consisting of 6 OFDMA symbols, and 1 type-2 subframe in DL, consisting of 7 OFDMA symbols. The same configuration is used in the UL assuming a DL to UL ratio of 1:1. The switching gap has been accounted for in the calculations.

For both TDD and FDD, the number of pilots per resource unit (RU) for both type-1 and type-2 subframes in case of 4 streams for DL is 16, while the numbers of subcarriers per RU for type-1 and type-2 subframes are 108 (= 18x6) and 126 (= 18x7), respectively. Therefore the numbers of available data subcarriers per RU for type-1 and type-2 subframes in case of 4 for DL streams are 92 and 110, respectively. On the other hand, in case of 2 streams for UL, the number of pilots per RU for type-1 and type-2 subframes are 12 and 14, respectively. Therefore the numbers of available data subcarriers per RU for type-1 and type-2 subframes in case of 2 streams for UL are 96 and 112, respectively.

For FDD, the number of RUs for 20 MHz is 96 both in DL and UL. The number of available data subcarriers for 5 type-1 and 3 type-2 subframes with 4 streams for DL are 44160 (= 5x92x96), denoted by NType1Subcarrier4 and 31680 (= 3x110x96), denoted by NType2Subcarrier4, respectively. On the other hand, the number of available data subcarriers for 5 type-1 and 3 type-2 subframes with 2 streams for UL are 46080 (= 5x96x96), denoted by NType1Subcarrier2, and 32256 (= 3x112x96), denoted by NType2Subcarrier2, respectively. Note that in 20 MHz, the 1728 subcarriers per frame is utilized in DL as preamble for physical layer synchronization.

Assuming repetition rate of 1, coding rate of 1, and the maximum MCS level, i.e. the modulation scheme of 64QAM (6 bit/symbol), the peak data rates in DL and UL are defined as follows:

Peak data rate in DL = (NType1Subcarrier4 + NType2Subcarrier4 - 1728) x 6 x 4 / 5 ms = 356 Mbit/s

Peak data rate in UL = (NType1Subcarrier2 + NType2Subcarrier2) x 6 x 2 / 5 ms = 188 Mbit/s

The peak spectral efficiencies in DL and UL can be calculated by dividing the peak data rates by 20 MHz, resulting 17.79 bit/s/Hz and 9.40 bit/s/Hz, for DL and UL respectively.

For TDD, the number of RUs for 20 MHz is 96 both in DL and UL. Assuming and UL:DL ratio of 1:1, the number of available data subcarriers for 3 type-1 and 1 type-2 subframes with 4 streams for DL are 26496 (= 3x92x96), denoted by N’Type1Subcarrier4 and 10560 (= 1x110x96), denoted by N’Type2Subcarrier4, respectively. On the other hand, the number of available data subcarriers for 3 type-1 and 1 type-2 subframes with 2 streams for UL are 27648 (= 3x96x96), denoted by N’Type1Subcarrier2, and 10752 (= 1x112x96), denoted by N’Type2Subcarrier2, respectively. Note that in 20 MHz, the 1728 subcarriers per frame is utilized in DL as preamble for physical layer synchronization.

Assuming repetition rate of 1, coding rate of 1, and the maximum MCS level, i.e. the modulation scheme of 64QAM (6 bit/symbol), the peak data rates in DL and UL are defined as follows:

Peak data rate in DL = (N’Type1Subcarrier4 + N’Type2Subcarrier4 - 1728) x 6 x 4 / 5 ms = 169.57 Mbit/s

Peak data rate in UL = (N’Type1Subcarrier2 + N’Type2Subcarrier2) x 6 x 2 / 5 ms = 92.16 Mbit/s

The peak spectral efficiencies in DL and UL can be calculated by dividing the peak data rates by 20 MHz and adjusting by the DL to UL ratio, resulting 16.96 bit/s/Hz and 9.22 bit/s/Hz, for DL and UL respectively.

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced peak spectral efficiency.

# 7.2.5	User-plane/control-plane latency and handover interruption time

The analyses in this section use the network reference model {4}. In these analyses, it is assumed that each 5 ms radio frame of the proposed RIT consists of eight subframes of 0.617 ms length and the transmission time interval (TTI) is equal to one subframe. The mobile station and base station processing times are assumed to be three subframes (3*TTI).

Figure 7-1

Network reference model {4}

This analysis further assumes an intra-ASN handover mechanism where the serving and target base stations belong to the same ASN entity.

7.2.5.1	User-plane latency

HARQ retransmission probability is estimated based on both 10% and 30% HARQ retransmission probabilities. The HARQ RTT is estimated as 8*TTI as shown below for FDD duplex scheme and a synchronous HARQ scheme. The HARQ RTT for the TDD mode assuming DL:UL ratio of 5:3 and one DL/UL switching point per radio frame is 8*TTI.

Figure 7-2

Example synchronous HARQ in FDD duplex mode

The following reference model is used to estimate the user-plane latency.

Figure 7-3

User-plane latency calculation model

MS

BS

1.85 ms

1.85 ms

HARQ RTT

5 ms

1.85 ms

1.85 ms

TTI + Frame

Alignment

0.92 ms

0.92 ms

The following table summarizes the result of the analysis:

TABLE 7-15

User-plane latencies for 10% and 30% Probability of HARQ Retransmissions

| Step | Description | User-Plane Latency (10% HARQ Retransmission Probability) | User-Plane Latency (30% HARQ Retransmission Probability) |
| --- | --- | --- | --- |
| 0 | AMS wakeup time | Implementation Dependent | Implementation Dependent |
| 1 | AMS Processing Delay | 3 * 0.617 = 1.85 ms | 3 * 0.617 = 1.85 ms |
| 2 | Queuing/Frame Alignment | FDD: 0.31 TDD: 2.5 ms | FDD: 0.31 TDD: 2.5 ms |
| 3 | TTI for UL data packet (Piggy back scheduling information) | 0.617 ms | 0.617 ms |
| 4 | HARQ Retransmission | 0.1 * 5 ms | 0.3 * 5 ms |
| 5 | ABS Processing Delay | 3 * 0.617 = 1.85 ms | 3 * 0.617 = 1.85 ms |
|  | Total one way access delay | FDD: 5.13 ms TDD: 7.32 ms | FDD: 6.13 ms TDD: 8.32 ms |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced user-plane latency.

7.2.5.2	Control-Plane Latency

Table 7-16 contains assumptions and results for control-plane latency analysis for the proposed RIT.

TABLE 7-16

Control-plane Latency for 10% and 30% Probability of HARQ Retransmissions

| Step | Description | Control-Plane Latency (10% HARQ Retransmission Probability) | Control-Plane Latency (30% HARQ Retransmission Probability) |
| --- | --- | --- | --- |
| 0 | AMS wakeup time | Implementation dependent | Implementation dependent |
| 1 | DL scanning and synchronization + Acquisition of the broadcast channel (system configuration information) for initial system entry | < 50 ms Assuming that S-SFH SP2 that contains network re-entry information is transmitted every 50 ms. This could further reduce if SP2 is transmitted more frequently | < 50 ms Assuming that S-SFH SP2 that contains network re-entry information is transmitted every 50 ms. This could further reduce if SP2 is transmitted more frequently |
| 2 | Random access procedure (UL CDMA Code + ABS Processing + DL CDMA_ALLOC_IE) | 5 ms | 5 ms |
| 3 | Initial ranging (RNG-REQ + ABS processing + RNG-RSP) + HARQ retransmission of one message at 10% or 30%, only first-order estimation | HARQ case: 1 frame * 0.9*0.9 + 2 frame * 2*0.1*0.9 + 3 frame * 0.1*0.1 = 1.2 frame = 6 ms The assumption is the message will either succeed in #1 transmission with probability=0.9 or succeed in #2 transmission with probability=0.1 | HARQ case: 1 frame * 0.7*0.7 + 2 frame * 2*0.3*0.7 + 3 frame * 0.3*0.3 = 1.6 frame = 8 ms The assumption is the message will either succeed in #1 transmission with probability=0.7 or succeed in #2 transmission with probability=0.3 |
| 4 | Capability negotiation (SBC-REQ + ABS processing + SBC-RSP) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 5 | Authorization and authentication/key exchange (PKM-REQ + ABS processing + PKM-RSP + …) +HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 6 | Registration (REG-REQ + ABS/ASN-GW processing + REG-RSP) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 7 | RRC connection establishment (DSA-REQ + ABS processing + DSA-RSP + DSA-ACK) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
|  | Total C-plane connection establishment delay | < 31 ms | < 33 ms |
|  | Total IDLE_STATE –> ACTIVE_ACTIVE delay | < 81 ms | < 83 ms |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced control-plane latency.

7.2.5.3	Intra-RIT Handover Interruption Time

In the proposed RIT, the handover (HO) can be initiated by either the AMS or the S-ABS. The HO process is initiated when the AMS issues a HO request to the S-ABS or when the S-ABS issues a HO command to the AMS. The HO request/command is issued following Mobile-Assisted HO (MAHO) procedures. The AMS acquires the network topology through either S-ABS broadcasts or unicast signaling messages. The S-ABS also provides the AMS with the relevant trigger conditions to initiate or cancel neighbor-cell measurements, measurement reporting and HO requests. If required, scanning intervals for neighbor-cell measurements are provided by the S-ABS unilaterally or at the request of the AMS. The HO request/command is an outcome of these measurements and reporting.

The HO procedure is divided into four phases; namely, HO initiation, HO preparation, HO execution, and network reentry. In addition, HO cancellation procedure is defined to allow AMS cancel a HO procedure.

Figure 7-4

Generic HO Procedure in the Proposed RIT

7.2.5.4	Intra-RIT, Intra-frequency handover

The following describes the HO procedure with reference to the message sequence flow shown in Figure 7-4.

HO Initiation

During handover initiation phase, the AMS requests HO based on trigger conditions defined in ABS AAI_NBR-ADV broadcast message {4}. ABS may also initiate HO without AMS request by sending AAI_HO-CMD message to the AMS {4}.

HO Preparation

During HO preparation phase, the T-ABS and AMS obtain relevant information for HO optimization (e.g., AMS identity, AMS security context, T-ABS system information, service flows, etc.) via the S-ABS and the backbone network. For optimized network re-entry, the T-ABS(s) may allocate a dedicated ranging code/opportunity and resource allocations to the AMS via the S-ABS through the AAI-HO-CMD message. The message includes one or more T-ABSs, network re-entry action time and S-ABS link disconnect time. The S-ABS also indicates in the HO command (AAI-HO-CMD) whether the AMS performs CDMA ranging. The HO preparation phase completes with the exchange of AAI-HO-CMD/AAI-HO-IND messages resulting in the selection of a T-ABS and the S-ABS disconnect time.

HO execution

During HO execution, AMS responds with its T-ABS selection in the AAI-HO-IND message if multiple T-ABSs are included in AAI-HO-CMD. The AMS continue data communications with S-ABS, until the S-BS stops sending DL data and providing UL allocations to the AMS after expiration of the action/disconnect time included in the AAI-HO-CMD message. Any AMS bandwidth requests before the disconnect time that cannot be provided by the S-ABS are forwarded to the T-ABS. The S-ABS retains the connections, MAC state machine, and un-transmitted/unacknowledged data associated with the AMS for service continuation in case of HO cancellation until the expiration of resource-related timers.

Network Re-entry

If AMS performs network re-entry without CDMA ranging, the T-ABS and AMS can start data communications after the defined action time. If AMS performs CDMA ranging based network re-entry, the AMS performs network re-entry at the T-ABS using either a dedicated ranging code if allocated through AAI-HO-CMD or a random handover ranging code at the designated ranging opportunity if specified or otherwise the earliest ranging opportunity. Upon reception of the ranging code, the T-ABS allocates UL resources for AMS to send RNG-REQ message and UL data if needed. The T-ABS and AMS finishes AAI-RNG-REQ/RSP transactions by the predefined timer, which serves the purpose of key validation for each other. This validation process does not impact data communication and can happen in parallel as defined in seamless handover procedure in {1}.

HO Cancellation

The handover can be canceled by the AMS at any time during the HO process. The HO cancellation is initiated before the expiration of resource-related timers. The network can advertise HO cancellation trigger conditions.

7.2.5.5	Intra-RIT, Inter-frequency handover

The first three phases are identical with the case of Intra-RIT, Intra-frequency handover. The last phase, network re-entry, differs in that there is no option to skip CDMA ranging in the T-ABS.

For inter-frequency HO, the S-ABS always commands the AMS to perform network re-entry with CDMA ranging process. Depending on the method indicated by the HO command from the S-ABS, the AMS performs network re-entry at the T-ABS using either a dedicated ranging code if allocated through AAI-HO-CMD or a random handover ranging code at the designated ranging opportunity if specified or otherwise the earliest ranging opportunity. Upon reception of the ranging code, the T-ABS responds with AAI-RNG-RSP to instruct the AMS to adjust its UL transmission timing and power, allocate UL resources for AMS to send AAI-RNG-REQ message and UL data if needed. The rest of the procedure is the same as in intra-frequency HO case.

TABLE 7-17

handover interruption time

| Scenario | Interruption Time |
| --- | --- |
| Intra-RIT, Intra-frequency | 5 ms |
| Intra-RIT, Inter-frequency | 15 to 25ms (additional time for AMS to adjust RF and obtain CDMA ranging opportunity) |

Additionally, Inter-RIT, Intra-RIT, Intra-frequency, and Inter-system handover to other IMT systems is supported by the RIT.

The following diagram illustrates the seamless HO service flow in the proposed RIT.

Figure 7-5

Example of a seamless HO service flow in the proposed RIT

7.2.5.6	Summary of the Handover procedure

- Initiation

- HO triggered at AMS, based on AMS measurements and S-ABS-defined trigger levels

- HO request from AMS to S-ABS, containing T-ABS list with preferences and measurement reports

- Preparation

- S-ABS and T-ABS backbone pre-notification procedures

- HO command from S-ABS to AMS with T-ABS details, Disconnect time (may allow communication during network re-entry)

- Execution

- AMS acknowledges S-ABS HO command with selected T-ABS and confirmation/rejection of Disconnect time

- S-ABS disconnects at Disconnect time, transfers AMS context to T-ABS over backbone network

IV.   Network re-entry

- HO Ranging in T-ABS, optional for intra-FA

- T-ABS notifies S-ABS of HO completion

- Network re-entry complete with established data path in T-ABS

Assumptions

- Intra-ASN Handovers (the serving and target base stations belong to the same ASN)

- Mobile-assisted Handover (predominant case)

- Mobile-initiated Handover (worst-case compared to ABS-initiated)

- Optimized Hard Handover (for intra-frequency AMS is frame-synchronized with S-ABS and T-ABS, AMS contexts including security context, transferred to T-ABS by S-ABS over the backbone network)

All measurements are based on the synchronization channels.

The following steps capture the HO procedure delay budget (Intra-RIT, Intra-Frequency):

TABLE 7-18

Intra-RAT Intra-FA Handover interruption time

| Step | Procedure | Estimated Latency (ms) |
| --- | --- | --- |
| 1 | The AMS initiates HO by sending an AAI-HO-REQ to the S-ABS. | 4 to 7 frames, 20 to 35ms |
| 2 | The S-ABS processes AAI-HO-REQ and sends HO REQUEST to one or more T-ABS. | 1 frames, 5 ms (HO-REQ from S-ABS to T-ABSs) |
| 3 | T-ABSs reply S-ABS with HO RESPONSE, which may include HO optimization related MAC pre-update information. | 2 frames, 10ms (T-ABSs process and reply + HO-RSP from T-ABSs to S-ABS) |
| 4 | S-ABS responds to AMS with AAI-HO-CMD containing T-ABS list, Disconnect time | 1 frame, 5 ms |
| 5 | AMS acknowledges S-ABS with AAI-HO-IND containing selected T-ABS and confirmation/rejection of Disconnect time (unsolicited UL grant) | 1 frame, 5 ms |
| 6 | At/After Action Time (=Disconnect Time), S-ABS transfer un-acknowledged data and new data, if any, to T-ABS for AMS data continuity at T-ABS | 0 to 2 frames, 0 to 10ms (R8 interface latency, see Figure 7-1) |
| 7 | AMS switches to T-ABS, acquires DL signal | 1 frame, 5 ms |
| 8 | AMS reads UL-MAP for unsolicited UL grant for AMS to send RNG-REQ message and data | 2 frames, 10 ms |
| 9 | AMS sends RNG-REQ to T-ABS | 1 frame, 5 ms |
| 10 | T-ABS responds with RNG-RSP with necessary information for AMS to perform UL synchronization. | 2 frames, 10 ms |
| 11 | AMS processes RNG-RSP | 1 frame, 5 ms |
| 12 | If necessary, repeat steps 8 – 11 K-times Note: The maximum value of K is calculated based upon the number of times that steps 8 to 11 could be repeated before expiration of a timer specified by S-ABS. | K * 5 frames, 0 to 25K ms |
| 13 | T-ABS and AMS continue data communication | 0 |
| HO Interruption time (Using Seamless HO) |  | summation of time required for steps 6 and 7= 0 to 15 ms |

Using seamless HO procedure, HO interruption time includes steps 6 and 7 in Table 6-6. Adding the times required for completion of steps 6 and 7results in a value of 0 to 15 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 10 ms.

For intra-RIT/Inter-Frequency HO, there will be following two changes:

Step-7 will be longer (implementation dependent) so that the AMS can calibrate its RF for inter-FA operation.

An additional step 7.1 is to be inserted between Step-7 and Step-8, which will be counted into HO interruption time.

TABLE 7-19

Additional step in calculation of inter-FA, intra-RIT handover interruption time

| 7.1 | The AMS waits for HO ranging opportunity to perform UL synchronization with dedicated ranging code (assigned by TABS during HO preparation.) (after CDMA ranging, UL synchronization procedures are not counted into HO interruption time according to the definition) | 1 to 4 frames, 5 to 20 ms (20ms is the worst case when no dedicated ranging opportunity is allocated for this HO instance. Most cases, T-ABS has knowledge of AMS capability and how fast it can switch RF, and therefore T-ABS can prepare the ranging opportunity right at the next frame, in which case it will be 5 ms) |
| --- | --- | --- |

In this case (inter-FA, intra-RIT) , the HO interruption time is the summation of time required for steps 6, 7 and 7.1, which results in HO interruption time of 5 to 35 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 30 ms.

Therefore, IEEE confirms that the proposed RIT meets the IMT-Advanced HO interruption time requirements.

# 8	Compliance templates

## 8.1	Compliance template for services

|  | Service related minimum capabilities within the RIT/SRIT | Evaluator’s comments |
| --- | --- | --- |
| 4.2.4.1.1 | Support of a wide range of services Does the proposal support a wide range of services?: If bullets 4.2.4.1.1.1 - 4.2.4.1.1.3 are marked as "yes" then 4.2.4.1.1 is a "yes". YES / NO | The proposed RIT supports a number of QoS classes (see Section 10.10 of {4} for more details) that are designed to enable a wide range of services and applications. These services include but are not limited to the following: - Multicast and broadcast services (see Section 16 of {4} for more details) would allow support of IP-based multimedia applications such as real-time and non-real-time audio and video streaming, IP-TV, web-casts, etc. - Location based services (see Section 12 of {4} for more details) would allow support of location based applications such as interactive maps and navigation applications, etc. - Low one-way air-link transmission latency of less than 5 ms and short handoff interruption time of less than 20 ms would allow real-time applications such as interactive gaming, on-line collaborations, etc. - High capacity VoIP service is enabled (see Section 10 and 11 of {4} for more details) through efficient DL/UL control channel design, advanced DL/UL MIMO techniques, persistent and group scheduling schemes, low user and control plane latencies. - IP-based data services such as HTTP, e-mail, web-browsing, file transfer are enabled through high spectral efficiency, low user and control plane latencies and flexible QoS classes (see Section 7.2.5 for more details) |
| 4.2.4.1.1.1 | Ability to support basic conversational service class Is the proposal able to support basic conversational service class?: YES / NO | - Given that basic conversational service* is typically characterized by per user throughputs of 20 kbps and latencies of less than 50 ms, using baseline antenna configuration and 10 MHz bandwidth, the proposed RIT with average user throughput of greater than 2.6 Mbps in the DL and 1.3 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The handover interruption time for intra-Frequency Assignment (FA) is 5-10 ms and for inter-FA is 5 to 35 ms which both are significantly less the IMT-Advanced corresponding requirements which would enable a large number of service classes using the proposed RIT. - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. - *See Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |
| 4.2.4.1.1.2 | Support of rich conversational service class Is the proposal able to support rich conversational service class?: YES / NO | - Given that rich conversational service* is typically characterized by per user throughputs of 5 Mbps and latencies of less than 20 ms, using baseline antenna configuration and 20 MHz bandwidth, the proposed RIT with average user throughput of greater than 5.2 Mbps in the DL and 2.6 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. - *See Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |
| 4.2.4.1.1.3 | Support of conversational low delay service class Is the proposal able to support conversational low-delay service class? YES / NO | - Given that conversational low delay service* is typically characterized by per user throughputs of 150 kbps and latencies of less than 10 ms, using baseline antenna configuration and 10 MHz bandwidth, the proposed RIT with average user throughput of greater than 2.6 Mbps in the DL and 1.3 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. - *See Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |

8.2	Compliance template for spectrum

|  | Spectrum capability requirements |
| --- | --- |
| 4.2.4.2.1 | Spectrum bands Is the proposal able to utilize at least one band identified for IMT?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. The proposed RIT supports deployment in all bands identified for IMT in ITU-R Radio Regulations. In addition, proposed RIT supports non-IMT bands below 6 GHz allocated to the Fixed Service and/or Mobile Service. (See below for more details on some of the band classes in which the proposed RIT can be deployed). TABLE 8-1 Some of supported frequency bands Band Class; UL AMS Transmit Frequency (MHz); DL AMS Receive Frequency (MHz); Duplex Mode. 1; 2300-2400; 2300-2400; TDD. 2; 2305-2320, 2345-2360; 2305-2320, 2345-2360; TDD. 2345-2360; 2305-2320; FDD. 3; 2496-2690; 2496-2690; TDD. 2496-2572; 2614-2690; FDD. 4; 3300-3400; 3300-3400; TDD. 5L; 3400-3600; 3400-3600; TDD. 3400-3500; 3500-3600; FDD. 5H; 3600-3800; 3600-3800; TDD. 6; 1710-1770; 2110-2170; FDD. 1920-1980; 2110-2170; FDD. 1710-1755; 2110-2155; FDD. 1710-1785; 1805-1880; FDD. 1850-1910; 1930-1990; FDD. 1710-1785, 1920-1980; 1805-1880, 2110-2170; FDD. 1850-1910, 1710-1770; 1930-1990, 2110-2170; FDD. 7; 698-862; 698-862; TDD. 776-787; 746-757; FDD. 788-793, 793-798; 758-763, 763-768; FDD. 788-798; 758-768; FDD. 698-862; 698-862; TDD/FDD. 824-849; 869-894; FDD. 880-915; 925-960; FDD. 698-716, 776-793; 728-746, 746-763; FDD. 8; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; TDD. 9; 450-470; 450-470; TDD. 450.0-457.5; 462.5-470.0; FDD. |

# 8.3	Compliance template for technical performance

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.1 Cell spectral efficiency (bit/s/Hz/cell) (4.1) | Indoor | Downlink | 3 | TDD: 6.93 FDD: 6.87 |  Yes  No | The proposed RIT exceeds required values in all test environments. See Section 7.2.1 for details. |
|  |  | Uplink | 2.25 | TDD: 5.99 FDD: 6.23 |  Yes  No |  |
|  | Microcellular | Downlink | 2.6 | TDD: 3.22 FDD: 3.27 |  Yes  No |  |
|  |  | Uplink | 1.8 | TDD: 2.58 FDD: 2.72 |  Yes  No |  |
|  | Base coverage urban | Downlink | 2.2 | TDD: 2.41 FDD: 2.41 |  Yes  No |  |
|  |  | Uplink | 1.4 | TDD: 2.57 FDD: 2.69 |  Yes  No |  |
|  | High speed | Downlink | 1.1 | TDD: 3.23 FDD: 3.15 |  Yes  No |  |
|  |  | Uplink | 0.7 | TDD: 2.66 FDD: 2.77 |  Yes  No |  |
| 4.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | Not applicable | Downlink | 15 | TDD: 16.96 FDD: 17.79 |  Yes  No | The proposed RIT exceeds required values. See Section 7.2.4 for details. |
|  |  | Uplink | 6.75 | TDD: 9.22 FDD: 9.40 |  Yes  No |  |
| 4.2.4.3.3 Bandwidth (4.3) | Not applicable | Up to and including (MHz) | 40 | 20 MHz with single carrier; Up to and including 100 MHz with multi-carrier |  Yes  No | The proposed RIT supports multi-carrier operation that allows operation in any bandwidth as wide as 100 MHz by aggregating contiguous and/or non-contiguous RF carriers. |
|  |  | Scalability | Support of at least three band-width values(4) | 5, 7, 8.75, 10, and 20 MHz with single carrier operation, Maximum 100 MHz with multi-carrier operation |  Yes  No | The proposed RIT supports 5, 7, 8.75, 10 and 20 MHz with single carrier and also supports up to 100 MHz with multi-carrier operation |
| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.4 Cell edge user spectral efficiency (bit/s/Hz) (4.4) | Indoor | Downlink | 0.1 | TDD: 0.260 FDD: 0.253 |  Yes  No | The proposed RIT exceeds required values in all test environments. See Section 7.2.1 for details. |
|  |  | Uplink | 0.07 | TDD: 0.426 FDD: 0.444 |  Yes  No |  |
|  | Microcellular | Downlink | 0.075 | TDD: 0.092 FDD: 0.097 |  Yes  No |  |
|  |  | Uplink | 0.05 | TDD: 0.111 FDD: 0.119 |  Yes  No |  |
|  | Base coverage urban | Downlink | 0.06 | TDD: 0.069 FDD: 0.069 |  Yes  No |  |
|  |  | Uplink | 0.03 | TDD: 0.109 FDD: 0.114 |  Yes  No |  |
|  | High speed | Downlink | 0.04 | TDD: 0.093 FDD: 0.091 |  Yes  No |  |
|  |  | Uplink | 0.015 | TDD: 0.119 FDD: 0.124 |  Yes  No |  |
| 4.2.4.3.5 Control plane latency (ms) (4.5.1) | Not applicable | Not applicable | Less than 100 ms | < 81 ms |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.6 User plane latency (ms) (4.5.2) | Not applicable | Not applicable | Less than 10 ms | TDD: 7.32 ms FDD: 5.13 ms |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.7 Mobility classes (4.6) | Indoor | Uplink | Stationary, pedestrian | supported |  Yes  No | Proposed RIT supports required Mobility classes in all test environments. |
|  | Microcellular | Uplink | Stationary, pedestrian, vehicular up to 30 km/h | supported |  Yes  No |  |
|  | Base coverage urban | Uplink | Stationary, pedestrian, vehicular | supported |  Yes  No |  |
|  | High speed | Uplink | High speed vehicular, vehicular | supported |  Yes  No |  |
| 4.2.4.3.8 Mobility Traffic channel link data rates (bit/s/Hz) (4.6) | Indoor | Uplink | 1.0 | LoS TDD: 3.76 NLoS TDD: 3.41 LoS FDD: 3.86 NLoS FDD: 3.56 |  Yes  No | Proposed RIT exceeds required values in all test environments. See Section 7.2.3 for detailed information. |
|  | Microcellular | Uplink | 0.75 | LoS TDD: 1.81 NLoS TDD: 1.50 LoS FDD: 1.72 NLoS FDD: 1.51 |  Yes  No |  |
|  | Base coverage urban | Uplink | 0.55 | LoS TDD: 1.72 NLoS TDD: 1.30 LoS FDD: 1.63 NLoS FDD: 1.34 |  Yes  No |  |
|  | High speed | Uplink | 0.25 | LoS TDD: 1.70 NLoS TDD: 1.23 LoS FDD: 1.61 NLoS FDD: 1.27 |  Yes  No |  |
| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Requirement met? | Comments |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.9 Intra-frequency hand-over interruption time (ms) (4.7) | Not applicable | Not applicable | 27.5 | 5-10 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.10 Inter-frequency handover interruption time within a spectrum band (ms) (4.7) | Not applicable | Not applicable | 40 | 5-35 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.11 Inter-frequency handover interruption time between spectrum bands (ms) (4.7) | Not applicable | Not applicable | 60 | 5-35 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.12 Inter-system handover (4.7) | Not applicable | Not applicable | Not applicable | Not applicable |  Yes  No | The proposed RIT supports inter-RAT handover to and from other IMT-2000 and IMT-Advanced technologies. See Section 10.3 of {4} for detailed information on inter-RAT handover in the proposed RIT. |
| 4.2.4.3.13 Number of supported VoIP users (active users/ sector/MHz) (4.8) | Indoor | As defined in Report ITU-R M.2134 | 50 | TDD: 140 FDD: 139 |  Yes  No | Reported numbers are minimum of UL and DL. Proposed RIT exceeds required values in all test environments. See Section 7.2.2 for detailed information. |
|  | Microcellular | As defined in Report ITU-R M.2134 | 40 | TDD: 82 FDD: 77 |  Yes  No |  |
|  | Base coverage urban | As defined in Report ITU-R M.2134 | 40 | TDD: 74 FDD: 72 |  Yes  No |  |
|  | High speed | As defined in Report ITU-R M.2134 | 30 | TDD: 89 FDD: 90 |  Yes  No |  |
| (1) As defined in Report ITU-R M.2134. (2) According to the evaluation methodology specified in Report ITU-R M.2135. (3) Mandatory when “no” is checked, optional when “yes” is checked. (4) Refer to Report ITU-R M.2135, § 7.4.1. |  |  |  |  |  |  |

Annex 3

Simulation assumptions and configuration parameters for 
IMT-Advanced test environments

Table A3-1

OFDMA and frame parameters

| Parameter | Description | RIT Parameters for Indoor Environment |  | RIT Parameters for Urban Micro-cellular, Urban Macro-cellular, High Speed Environments |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDD | FDD | TDD | FDD |  |
| fc | Carrier frequency | 3.4 GHz |  | Urban Micro-cellular: 2.5 GHz |  |  |
|  |  |  |  | Urban Macro-cellular: 2.0 GHz |  |  |
|  |  |  |  | High Speed: 0.8 GHz |  |  |
| BW | Total bandwidth | 40 MHz for data-only (2x20 MHz) | 2x20 MHz for data-only | 20 MHz for data-only |  | 2x10 MHz for data-only |
|  |  | 10 MHz for VoIP | 5 + 5 MHz for VoIP | 10 MHz for VoIP |  | 5 + 5 MHz for VoIP |
| NFFT | Number of points in full FFT | 2x2048 for data-only | 2048 for data-only | 2048 for data-only |  | 1024 for data-only |
|  |  | 1024 for VoIP | 512 for VoIP | 1024 for VoIP |  | 512 for VoIP |
| FS | Sampling frequency | 44.8 MHz for data-only | 22.4 MHz for data-only | 22.4 MHz for data-only |  | 11.2 MHz for data-only |
|  |  | 11.2 MHz for VoIP | 5.6 MHz for VoIP | 11.2 MHz for VoIP |  | 5.6 MHz for VoIP |
| Δf | Subcarrier spacing | 10.9375 kHz |  |  |  |  |
| To=1Δf | OFDMA symbol duration without cyclic prefix | 91.43 us |  |  |  |  |
| CP | Cyclic prefix length (fraction of To) | 1/16 |  |  |  |  |
| Ts | OFDMA symbol duration with cyclic prefix | 97.143 us |  |  |  |  |
| TF | Frame length | 5 ms |  |  |  |  |
| NF | Number of OFDMA symbols in frame (excluding switching gaps) | 50 | 51 | 50 | 51 |  |
| RDL−UL | Ratio of DL to UL (TDD mode) | 5 DL subframes, 3 UL subframes for data-only | 8 DL subframes for DL and UL | 5 DL subframes, 3 UL subframes for data-only | 8 DL subframes for DL and UL |  |
|  |  | 4 DL subframes, 4 UL subframes for VoIP | 8 DL subframes for DL and UL | 4 DL subframes, 4 UL subframes for VoIP | 8 DL subframes for DL and UL |  |
| Tduplex | Duplex time | TTG+RTG = 142.85 µs | N/A | TTG+RTG = 142.85 µs | N/A |  |

Table A3-2

DL configuration

| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| --- | --- | --- | --- |
| Basic modulation for data | Modulation schemes for data | QPSK, 16QAM, 64QAM |  |
| Basic modulation for control | Modulation schemes for control | QPSK |  |
| Duplexing scheme | TDD or FDD | TDD/FDD |  |
| Subchannelization for data allocations | Contiguous/Distributed Resource Units and associated permutations | - Scheme for InH and UMi: Sub-band CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6};12 equal-size allocations spanning the complete duration of the time resources (DL portion of the TDD frame, DL FDD frame) - Scheme for UMa and RMa: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6}; 6 equal-size allocations spanning the complete duration of the time resources (DL portion of the TDD frame, DL FDD frame) |  |
| Subchannelization for VoIP allocations | Contiguous/Distributed Resource Units and associated permutations | Combination of the following two schemes (selection should be based on the maximization of the VoIP capacity): - Scheme 1: DRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6}; - Scheme 2: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6} |  |
| Subchannelization for A-A-MAP | Contiguous/Distributed Resource Units and associated permutations | Distributed LRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6} |  |
| DL pilot structure | Pilot structure, density etc. | Depends on the number of streams per allocation: 1 or 2 pilot streams from 2 stream interlaced pilot structure as defined in Section 3.3.1.4.1 of {6}with 2 dB pilot power boosting over data tone 3 or 4 pilot streams from 4 stream pilot structure as defined in in Section 3.3.1.4.1 of {6}with 0 dB pilot power boosting over data tone |  |
| Multi-antenna transmission format for data | Multi-antenna configuration and transmission scheme | - Scheme for InH and UMi: 6-bit Transformed Codebook or analog feedback based MU-MIMO using 4x2 configuration; adaptive switching among rank-1 CL-SU-MIMO, two stream CL-MU-MIMO, three stream CL-MU-MIMO and four stream CL-MU-MIMO - Scheme for UMa and RMa: MU-MIMO with long term beamforming using 4x2 configuration (20 ms reporting period for the long-term covariance matrix); adaptive switching among rank-1 CL-SU-MIMO, two stream CL-MU-MIMO, three stream CL-MU-MIMO and four stream CL-MU-MIMO |  |

| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| --- | --- | --- | --- |
| Multi-antenna transmission format for voip | Multi-antenna configuration and transmission scheme | Combination of the following two schemes (selection should be based on the maximization of the VoIP capacity): - Scheme 1: SU-MIMO using 4x2 configuration with SFBC + non-adaptive precoding - Scheme 2: SU-MIMO with wideband beamforming using 4x2 configuration |  |
| Multi-antenna transmission format for A-A-MAP | Multi-antenna configuration and transmission scheme | Open-loop SFBC + non-adaptive precoding (TX diversity) |  |
| Receiver structure | MMSE/ML/MRC, Receiver interference awareness | MMSE for channel estimation and MMSE/MLD data detection |  |
| Data channel coding | Channel coding schemes | Convolutional Turbo Coding (CTC) 1/3 |  |
| Control channel coding for A-A-MAP | Channel coding schemes and block sizes | As described in Section 3.3.2.3.2.2 of {6} with MLRU size equal to 56 tones |  |
| Scheduling | Demonstrate performance / fairness criteria in accordance to traffic mix | Proportional Fair for full buffer data and delay-weighted Proportional Fair with persistent scheduling for VoIP |  |
| Link adaptation | Modulation and Coding Schemes (MCS), CQI feedback delay / error | Choice of 16 MCS schemes inclusive of coding rate and rate matching, see Section 11.13 of {4} |  |
| Link to system mapping | MI based PHY abstraction | RBIR or MMIB by providing the necessary supporting PHY abstraction link curves |  |
| HARQ | Chase combining/ incremental redundancy, synchronous/asynchronous, adaptive/non-adaptive ACK/NACK delay, Maximum number of retransmissions, retransmission delay | Incremental Redundancy Asynchronous, adaptive, 3 subframe ACK/NACK delay, maximum 4 HARQ retransmissions, minimum retransmission delay 3 subframes |  |
| Power allocation | Subcarrier power allocation | Equal power per subcarrier; power boosting should be used only for A-A-MAP allocations (separate power boosting for each A-A-MAP allocation) |  |
| Interference model | Co-channel interference model, fading model for interferers, number of major interferers, threshold | Explicitly modeled Average interference on used subcarriers per CRU (subband or miniband) in PHY abstraction {3} |  |
| Frequency reuse | Frequency reuse pattern | 3 sectors with frequency reuse 1 |  |
| Control signalling | Message/signaling format, overheads | SFH, A-MAP, ACK/NACK, and Power Control overhead/channels as described in Section 3.3.2.2 of {6} Dynamic transmission modeled for A-A-MAP; percentage of signalling error modeled for fixed control overhead, should be derived from corresponding link curve for each test environment |  |
| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| Persistent scheduling | Persistent scheduling for VoIP only | Persistent scheduling for individual VoIP connections as described in Section 3.2.1 of {6} |  |
| Control channel overhead | L1/L2 Overhead | Dynamic overhead model whenever possible |  |
| Antenna tilt angle | Antenna tilt angle | InH: 0 deg. UMi: 12 deg. UMa: 12 deg. RMa: 6 deg. |  |

Table A3-3

UL configuration

| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| --- | --- | --- | --- |
| Basic modulation for data | Modulation schemes for data | QPSK, 16QAM, 64QAM |  |
| Basic modulation for control | Modulation schemes for control | Dependent on feedback channel (FBCH, HARQ,BW-REQ, Ranging, Sounding) as defined in Section 3.3.5 of {6} |  |
| Duplexing scheme | TDD or FDD | TDD/FDD |  |
| Subchannelization for data allocations | Contiguous/Distributed Resource Units and associated permutations | - Scheme for InH and UMi: Sub-band CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.4.1-3.3.4.3 of {6}; 12 equal-size allocations spanning the complete duration of the time resources (UL portion of the TDD frame, UL FDD frame) - Scheme for UMa and RMa: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.4.1-3.3.4.3 of {6}; 6 equal size allocations spanning the complete duration of the time resources (UL portion of the TDD frame, UL FDD frame) |  |
| Subchannelization for VoIP allocations | Contiguous/Distributed Resource Units and associated permutations | DRUs with frequency reuse 1 without FFR as defined in 3.3.4.1-3.3.4.3 of {6} |  |
| UL pilot structure | Pilot structure, density etc. | 1/2 stream pilot structure as defined in 3.3.4.4 of {6} without pilot power boosting |  |
| Multi-antenna transmission format for data | Multi-antenna configuration and transmission scheme | - Scheme for InH and UMi: 3-bit Codebook based MU- MIMO using 2x4 configuration; adaptive switching between single user and collaborative spatial multiplexing - Scheme for UMa and RMa: MU-MIMO with long term beamforming using 2x4 configuration; adaptive switching between single-user and collaborative spatial multiplexing |  |
| Multi-antenna transmission format for VoIP | Multi-antenna configuration and transmission scheme | SU-MIMO using 2x4 configuration with SFBC + non-adaptive precoding |  |
| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| Multi-antenna transmission format for control | Multi-antenna configuration and transmission scheme | 2x4 antenna configuration |  |
| Receiver structure | MMSE/ML/MRC, Receiver interference awareness | MMSE for channel estimation and MMSE/MLD for data detection |  |
| Data channel coding | Channel coding schemes | Convolutional Turbo Coding (CTC) 1/3 |  |
| Control channel coding | Channel coding schemes and block sizes | As described in Section 3.3.5 of {6} for FBCH, HARQ, BW-REQ, Ranging, Sounding |  |
| Scheduling | Demonstrate performance / fairness criteria in accordance to traffic mix | Proportional Fair for full buffer data and delay-weighted Proportional Fair with persistent scheduling for VoIP |  |
| Link adaptation | Modulation and Coding Schemes (MCS), CQI feedback delay / error | Choice of 16 MCS schemes inclusive of coding rate and rate matching, see Section 11.13 of {4} |  |
| Link to system mapping | MI based PHY abstraction | RBIR or MMIB by providing the necessary supporting PHY abstraction link curves |  |
| HARQ | Chase combining/ incremental redundancy, synchronous/asynchronous, adaptive/non-adaptive ACK/NACK delay, Maximum number of retransmissions, retransmission delay | Incremental Redundancy Synchronous, non-adaptive, 3 subframe ACK/NACK delay, maximum 4 HARQ retransmissions, minimum retransmission delay 3 subframes |  |
| Power control | Open loop/closed loop | Open loop power control as described in 3.3.5.4 of {6}; values for γ and SINRMIN should be chosen such that the average IoT meets the IMT-Advanced requirement |  |
| Interference model | Co-channel interference model, fading model for interferers, number of major interferers, threshold | Explicitly modeled Average interference on used subcarriers per CRU (subband or miniband) in PHY abstraction {3} |  |
| Frequency reuse | Frequency reuse pattern | 3 sectors with frequency reuse 1 |  |
| Control signalling | Message/signaling format, overheads | Initial ranging, periodic ranging, handover ranging, bandwidth request fast feedback/CQI channel, sounding overheads modeled as described in 3.3.5 of {6} Percentage of signalling error modeled for fixed control overhead should be derived from corresponding link curve for each test environment |  |
| Persistent scheduling | Persistent scheduling for VoIP only | Persistent scheduling for individual VoIP connections as described in Section 3.2.1 of {6} |  |
| Control channel overhead | L1/L2 Overhead | Dynamic overhead model whenever possible |  |
| Antenna tilt angle | Antenna tilt angle | InH: 0 deg. UMi: 12 deg. UMa: 12 deg. RMa: 6 deg. |  |

Table A3-4

VoIP configuration

| Parameter | Characterization |
| --- | --- |
| Codec | Source rate 12.2 kbit/s |
| Voice Activity Factor (VAF) | 50% (c=0.01, d=0.99) |
| Speech payload | 33 bytes, encoder frame every 20 ms |
| Silence Insertion Descriptor (SID) payload | 7 bytes, SID packet every 160ms during silence |
| Protocol overhead with compressed header | 4 byte MAC Header + Extended Header 4 byte (RTP/UDP/IP) 4 byte (RLC/security) 16 bits (CRC) |
| Packet size | 47 bytes (talk),/ 21 bytes (silence) |

Annex 4

Details of simulation-related results in the compliance 
template for technical performance

This Annex provides the results from all input contributors submitted for filling in the self-evaluation of the proposed RIT. The information below consists of 11 tables which include results for the following cases:

- Full buffer

  - DL and UL control overhead (Tables A4-1 and A4-2)

  - DL and UL cell spectral efficiency and cell edge SE for TDD (Tables A4-3 and A4-4) and FDD (Tables A4-5 and A4-6) systems

- VoIP capacity

  - DL and UL VoIP capacity numbers for TDD (Table A4-7) and FDD (Table A4-8)

- Mobility requirement

  - UL SINR CDF for link simulation (Table A4-9)

  - TDD and FDD systems (Tables A4-10 and A4-11)

In the tables below, reported data is based on {12} and {13} with data attributed to:

Alcatel-Lucent Shanghai Bell, Clearwire, ETRI, Fujitsu, Hitachi, Intel Corporation, ITRI, KDDI R&D Laboratories, LG Electronics, MediaTek Inc., Mitsubishi Electric, Motorola Inc., NEC, Samsung Electronics, Toshiba, and UQ Communications.

In some cases, data from individual sources is reported along with an average value.

Table A4-1

Control overhead (in %) for TDD per test environment and link (downlink/uplink)

| Test Environment | Downlink | Uplink |
| --- | --- | --- |
| Indoor (InH) | 9.19 | 7.85 |
| Mirocellular (UMi) | 12.33 | 12.60 |
| Base coverage urban (UMa) | 11.17 | 9.23 |
| High speed (RMa) | 11.15 | 8.34 |

Table A4-2

Control overhead (in %) for FDD per test environment and link (downlink/uplink)

| Test Environment | Downlink | Uplink |
| --- | --- | --- |
| Indoor (InH) | 9.74 | 6.02 |
| Mirocellular (UMi) | 16.28 | 10.58 |
| Base coverage urban (UMa) | 13.77 | 8.01 |
| High speed (RMa) | 13.63 | 6.51 |

Table A4-3

DL spectral efficiency for TDD

| Test environment | ITU Requirement |  | Source 10 | Source 3 | Source 1 | Source 6 | Source 8 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 3.0 |  |  | 6.75 |  |  | 7.10 | 6.93 |
|  | Cell-edge user | 0.10 |  |  | 0.235 |  |  | 0.286 | 0.260 |
| Microcellular (UMi) | Cell | 2.6 | 3.04 | 3.18 | 3.45 | 3.13 |  | 3.28 | 3.22 |
|  | Cell-edge user | 0.075 | 0.076 | 0.089 | 0.087 | 0.097 |  | 0.112 | 0.092 |
| Base coverage urban (UMa) | Cell | 2.2 | 2.36 | 2.32 | 2.62 | 2.40 | 2.35 | 2.41 | 2.41 |
|  | Cell-edge user | 0.06 | 0.060 | 0.071 | 0.071 | 0.077 | 0.064 | 0.069 | 0.069 |
| High speed (RMa) | Cell | 1.1 |  |  | 3.58 |  |  | 2.88 | 3.23 |
|  | Cell-edge user | 0.04 |  |  | 0.095 |  |  | 0.092 | 0.093 |

Table A4-4

UL spectral efficiency for TDD

| Test environment | ITU Requirement |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 2.25 | 5.20 |  | 6.79 | 5.99 |
|  | Cell-edge user | 0.07 | 0.361 |  | 0.491 | 0.426 |
|  | Average IoT (dB) < 10 dB |  | 9.37 |  | 8.70 |  |
| Microcellular (UMi) | Cell | 1.8 | 2.60 | 2.53 | 2.62 | 2.58 |
|  | Cell-edge user | 0.05 | 0.137 | 0.091 | 0.106 | 0.111 |
|  | Average IoT (dB) < 10 dB |  | 9.00 | 9.30 | 9.80 |  |
| Base coverage urban (UMa) | Cell | 1.4 | 2.38 | 2.49 | 2.86 | 2.57 |
|  | Cell-edge user | 0.03 | 0.113 | 0.091 | 0.123 | 0.109 |
|  | Average IoT (dB) < 10 dB |  | 9.59 | 9.90 | 9.90 |  |
| High speed (RMa) | Cell | 0.7 | 2.45 |  | 2.87 | 2.66 |
|  | Cell-edge user | 0.015 | 0.125 |  | 0.114 | 0.119 |
|  | Average IoT (dB) < 10 dB |  | 8.60 |  | 9.80 |  |

Table A4-5

DL spectral efficiency for FDD

| Test environment | ITU Requirement |  | Source 3 | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 3.0 |  | 6.85 |  | 6.89 | 6.87 |
|  | Cell-edge user | 0.10 |  | 0.239 |  | 0.266 | 0.253 |
| Microcellular (UMi) | Cell | 2.6 | 3.05 | 3.57 | 3.30 | 3.14 | 3.27 |
|  | Cell-edge user | 0.075 | 0.089 | 0.090 | 0.101 | 0.108 | 0.097 |
| Base coverage urban (UMa) | Cell | 2.2 | 2.23 | 2.63 | 2.48 | 2.30 | 2.41 |
|  | Cell-edge user | 0.06 | 0.060 | 0.069 | 0.082 | 0.063 | 0.069 |
| High speed (RMa) | Cell | 1.1 |  | 3.58 |  | 2.72 | 3.15 |
|  | Cell-edge user | 0.04 |  | 0.095 |  | 0.087 | 0.091 |

Table A4-6

UL spectral efficiency for FDD

| Test environment | ITU Requirement |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 2.25 | 5.40 |  | 7.06 | 6.23 |
|  | Cell-edge user | 0.07 | 0.377 |  | 0.511 | 0.444 |
|  | Average IoT (dB) < 10 dB |  | 9.45 |  | 8.70 |  |
| Microcellular (UMi) | Cell | 1.8 | 2.66 | 2.75 | 2.73 | 2.72 |
|  | Cell-edge user | 0.05 | 0.141 | 0.107 | 0.110 | 0.119 |
|  | Average IoT (dB) < 10 dB |  | 9.10 | 9.40 | 9.80 |  |
| Base coverage urban (UMa) | Cell | 1.4 | 2.46 | 2.66 | 2.96 | 2.69 |
|  | Cell-edge user | 0.03 | 0.117 | 0.097 | 0.128 | 0.114 |
|  | Average IoT (dB) < 10 dB |  | 9.39 | 9.80 | 9.90 |  |
| High speed (RMa) | Cell | 0.7 | 2.54 |  | 2.99 | 2.77 |
|  | Cell-edge user | 0.015 | 0.130 |  | 0.118 | 0.124 |
|  | Average IoT (dB) < 10 dB |  | 9.05 |  | 9.80 |  |

Table A4-7

VoIP capacity for TDD

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | UL | 154 |  | 176 |  |
|  | DL | 146 |  | 135 |  |
|  | Min of UL and DL | 146 |  | 135 | 140 |
|  | ITU requirement | 50 | 50 | 50 | 50 |
| Microcellular (UMi) | UL | 99 |  | 110 |  |
|  | DL | 84 | 86 | 77 |  |
|  | Min of UL and DL | 84 | 86 | 77 | 82 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| Base coverage urban (UMa) | UL | 93 |  | 98 |  |
|  | DL | 78 | 78 | 68 |  |
|  | Min of UL and DL | 78 | 78 | 68 | 74 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| High speed (RMa) | UL | 101 |  | 106 |  |
|  | DL | 99 |  | 80 |  |
|  | Min of UL and DL | 99 |  | 80 | 89 |
|  | ITU requirement | 30 | 30 | 30 | 30 |

Table A4-8

VoIP capacity for FDD

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | UL | 156 |  | 176 |  |
|  | DL | 144 |  | 134 |  |
|  | Min of UL and DL | 144 |  | 134 | 139 |
|  | ITU requirement | 50 | 50 | 50 | 50 |
| Microcellular (UMi) | UL | 100 |  | 104 |  |
|  | DL | 80 | 84 | 68 |  |
|  | Min of UL and DL | 80 | 84 | 68 | 77 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| Base coverage urban (UMa) | UL | 94 |  | 96 |  |
|  | DL | 74 | 80 | 64 |  |
|  | Min of UL and DL | 74 | 80 | 64 | 72 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| High speed (RMa) | UL | 102 |  | 100 |  |
|  | DL | 96 |  | 84 |  |
|  | Min of UL and DL | 96 |  | 84 | 90 |
|  | ITU requirement | 30 | 30 | 30 | 30 |

Table A4-9

UL SINR distribution for mobility requirement

Table A4-10

Mobility spectral efficiency (TDD)

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | LOS | 3.51 |  | 4.00 | 3.76 |
|  | NLOS | 3.41 |  |  | 3.41 |
|  | ITU requirement | 1.0 | 1.0 | 1.0 | 1.0 |
| Microcellular (UMi) | LOS | 1.64 |  | 1.98 | 1.81 |
|  | NLOS | 1.33 | 1.66 |  | 1.50 |
|  | ITU requirement | 0.75 | 0.75 | 0.75 | 0.75 |
| Base coverage urban (UMa) | LOS | 1.58 |  | 1.85 | 1.72 |
|  | NLOS | 1.26 | 1.33 |  | 1.30 |
|  | ITU requirement | 0.55 | 0.55 | 0.55 | 0.55 |
| High speed (RMa) | LOS | 1.54 |  | 1.85 | 1.70 |
|  | NLOS | 1.23 |  |  | 1.23 |
|  | ITU requirement | 0.25 | 0.25 | 0.25 | 0.25 |

Table A4-11

Mobility spectral efficiency (FDD)

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | LOS | 3.64 |  | 4.08 | 3.86 |
|  | NLOS | 3.56 |  |  | 3.56 |
|  | ITU requirement | 1.0 | 1.0 | 1.0 | 1.0 |
| Microcellular (UMi) | LOS | 1.70 |  | 1.74 | 1.72 |
|  | NLOS | 1.41 | 1.60 |  | 1.51 |
|  | ITU requirement | 0.75 | 0.75 | 0.75 | 0.75 |
| Base coverage urban (UMa) | LOS | 1.66 |  | 1.60 | 1.63 |
|  | NLOS | 1.30 | 1.38 |  | 1.34 |
|  | ITU requirement | 0.55 | 0.55 | 0.55 | 0.55 |
| High speed (RMa) | LOS | 1.61 |  | 1.60 | 1.61 |
|  | NLOS | 1.27 |  |  | 1.27 |
|  | ITU requirement | 0.25 | 0.25 | 0.25 | 0.25 |

Contacts:	Kyu-Jin Wee
		E-mail: kjwee@kcc.go.kr

Dae-Jung Kim
		E-mail: kdj@tta.or.kr

______________

Document 5D/560 - Submission of a candidate IMT-Advanced RIT based on IEEE 802.16

Document IMT-ADV/2(Rev.1)

See item 4.2.3.2.23.4 in Section 6.1.

In Tables 6-78 to 6-85, reported data is based on {14} with data attributed to:

ETRI, Fujitsu, Intel Corporation, KDDI R&D Laboratories, LG Electronics, MediaTek Inc., Motorola Inc., Samsung Electronics, and UQ Communications.