| Radiocommunication Advisory Group Geneva, 24-26 January 2007 |  |
| --- | --- |
|  | Document RAG07-1/1-E |
|  | 5 September 2006 |
|  | Original: English |

Director, Radiocommunication Bureau

General review of Resolutions and Recommendations of WARC/WRC

1	WRC-03 considered the process of reviewing the Resolutions and the Recommendations of WARCs and WRCs and introduced some modifications to the former practices in this respect (see Resolution 95 (Rev.WRC-03)). With this Resolution, WRC-03 instructed the Director of the Radiocommunication Bureau:

“1	to conduct a general review of the Resolutions and Recommendations of previous conferences and, after consultation with the Radiocommunication Advisory Group and the Chairmen and Vice-Chairmen of the Radiocommunication Study Groups, submit a report to the second session of the Conference Preparatory Meeting in respect of resolves 1 and resolves 2, including an indication of any associated agenda items;

2	to include in the above report, with the cooperation of the Chairmen of the Radiocommunication Study Groups, the progress reports of ITU-R studies on the issues which have been requested by the Resolutions and Recommendations of previous conferences, but which are not placed on the agendas of the forthcoming two conferences.”

2	In response to Resolution 95 (Rev.WRC-03), the Bureau performed an initial study in this respect with consultation as appropriate with the Chairmen and Vice-Chairmen of Study Groups, and the results are submitted to RAG, for advice (see Annex 1 to this document). Following consideration of the matter by RAG, an updated version of this document will be submitted to CPM.

3	The Bureau wishes to emphasize that the indications in the column “Possible follow-up” should not be considered as proposals for the work of the conference, but merely as suggestions concerning the possible course of action to be taken in respect of the concerned Resolution/Recommendation.

4	The Bureau refrained from indicating any possible course of action in respect to those Resolutions/Recommendations that are explicitly on the agenda of WRC-07.

Annex:	1

Annex 1

Review of WARC/WRC Resolutions and Recommendations in response to Resolution 95 (Rev.WRC-03)

| Res. No. | Subject | Remark | Possible follow-up |
| --- | --- | --- | --- |
| 1 | Notification of frequency assignments | Still relevant. | NOC |
| 2 | Equitable use of GSO and frequency bands for space services | Still relevant; text recently updated (at WRC-03). | NOC |
| 4 | Period of validity of GSO space systems | Still relevant; text recently updated (at WRC-03). For consideration by a future WRC. This subject may be considered in the context of Agenda items 1.10 and 1.12. | NOC |
| 5 | Technical cooperation – Propagation in tropical areas | Still relevant; text recently updated (at WRC-03). Supported by studies in SG 3. | NOC |
| 7 | National radio-frequency management | Still relevant; text recently updated (at WRC-03). Supported by BR and studies in SG 1 with respect to spectrum management systems for developing countries; also supported by BR world and regional seminars. | NOC |
| 10 | Wireless communications by the International Red Cross and Red Crescent Movement | Still relevant. | NOC |
| 13 | Formation of call signs | Still relevant, no major problems experienced (Separate BR report to WRC-07). | NOC |
| 15 | Cooperation in space radiocommunications | Still relevant; text recently updated (at WRC-03). Implemented through liaison with ITU-D Study Groups and BR/BDT seminars. | NOC |
| 18 | Identification/non-parties in an armed conflict | Still relevant; may need to be modified if WRC-07 decides to suppress Appendix 13 under Agenda item 1.14. | NOC/MOD |
| 20 | Technical cooperation – Aeronautical service | Still relevant; text recently updated (at WRC-03). | NOC |
| 21 | Transfer of HF-FX in 2007 | Would become obsolete after the completion of the transition period (1 April 2007) This Resolution is referred to in Nos. 5.136, 5.143, 5.146 and 5.151. | SUP |
| 25 | Operation of Global Satellite Systems for personnel communications | Still relevant; text recently updated (at WRC-03). | NOC |
| 26 | Review of footnotes | Still relevant (permanent agenda item at each WRC). Some updates may be required (e.g. in further resolves 3). | NOC/MOD |
| 27 | Incorporation by reference/principles | Still relevant (permanent agenda item at each WRC). Resolution currently under review by SC. | NOC |
| 28 | Revision of references to ITU-R Recommendations | Still relevant; linked with Resolution 27; currently under review by SC. Report by RA-07 to WRC-07 Agenda item 2. | NOC |
| 33 | Procedure for BSS prior to the entry into force of agreements and plans for the BSS | Processing of filings under this Resolution completed. This Resolution is referred to in No. 5.311 (footnote that may be reviewed under Agenda item 1.11). A former version of this Resolution is referred to in No. 5.396. | SUP |
| 34 | Planning the band 12.5-12.75 GHz in R3 | Still relevant. Text may need to be updated in view of WRC-07 decision on Resolution 33. | NOC |
| 42 | Interim systems in R2 (BSS and FSS) in AP30/30A bands | Still relevant; text recently updated (at WRC-03). | NOC |
| 49 | Administrative due diligence | Still relevant. Resolution currently under review by SC. | NOC/MOD |
| 51 | Transitional arrangements concerning coordination and notification | Transitional arrangement implemented by the time of WRC-07. | SUP |
| 55 | Temporary procedures for improving satellite network coordination and notification procedure. | Still relevant. May need to be updated. | MOD |
| 56 | Early application of No. 9.2, as revised by WRC-03 | Obsolete as from 1 January 2005. | SUP |
| 57 | Special arrangements for the networks above 71 GHz | Implemented. May be deleted. Resolution currently under review by SC. | SUP |
| 58 | Transitional measures for coordination in the bands 10.7-12.75 GHz, 17.8-18.6 GHz and 19.7-20.2 GHz | Still relevant. May need to be updated in view of current developments within BR leading to the completion of the “epfd” simulation software package. | MOD |
| 63 | Protection from ISM equipment | Still relevant; text recently updated (at WRC-03). On-going studies in SG 6, in consultation with SG 1. | NOC |
| 72 | Regional preparations | Superfluous. Some actions are completed, others included in other texts (Resolution 80 (PP-02), Resolution 25 (PP-02)). | MOD/SUP |
| 73 | Compatibility BSS-R1/FSS-R3 in 12 GHz | Still relevant. | NOC |
| 74 | Continuing updating of technical bases of Appendix 7 | Still relevant. Permanent agenda item for each WRC; included in Agenda item 7.1 of WRC-07. On-going consideration in SGs 1 and 3. | NOC |
| 75 | Possible update of technical bases of Appendix 7 for determining coordination area of receiving ES in SRS in bands 31.8-32.3 and 37-38 GHz | Still relevant (ongoing studies in SG 7). Closely related to Resolution 74. This Resolution is referred to in No. 5.547. | NOC |
| 76 | Development of calculation methodologies concerning aggregate epfd produced by non-GSO in the bands 10.7-30 GHz | Resolves part still relevant. Invites ITU-R may need to be updated taking account of Recommendation ITU-R S.1588 in force. | NOC/MOD |
| 79 | Criteria for determining coordination distances to protect RA stations from HDFS in band 42.5-43.5 GHz | Still relevant (ongoing studies on methodology provided in Recommendation ITU-R F.1766 (approved 2006). This Resolution is referred to in No. 5.547. | NOC |
| 80 | Principles of the Constitution, to be taken into consideration | For consideration by WRC-07 Agenda item 7.1. BR report to WRC-07. | - |
| 81 | Evaluation of administrative due diligence | Still relevant; may need to be updated. Indirectly considered by SC-WP in reviewing Resolution 49. | NOC/MOD |
| 85 | Protection of GSO systems (FSS and BSS) from NGSO FSS systems | Still relevant. May need to be updated in view of current developments within BR leading to the completion of the “epfd” simulation software package. | MOD |
| 86 | Criteria for implementation of Res. 86 (Rev. PP-02) | For consideration by WRC-07 Agenda item 1.12. BR report to WRC-07. | - |
| 87 | Entry into force of certain provisions relating to non-payment of cost-recovery fees | Implemented. | SUP |
| 88 | Rationalization of Articles 9 and 11 of the RR | For consideration by WRC-07 Agenda item 1.12. Considered by SC-WP. | - |
| 89 | Backlog in satellite filings | Still relevant. Implicitly on WRC-07 Agenda item 1.12. Recommendation ITU-R S.1656 in force; ongoing studies in SG 4, in liaison with SC. | NOC |
| 95 | Review of Resolution/Recommendation | Still relevant (permanent agenda item at each WRC). BR report to RAG-07 and CPM-07. | - |
| 96 | Provisional application of certain provisions of RR revised by WRC-03 and abrogation of certain Res./Rec. | Implemented and could be deleted. | SUP |
| 105 | Improvements in AP30B | Implicitly on the WRC-07 Agenda (item 1.10). Some parts (e.g. concerning 26 dB C/I) in contradiction with WRC-03 decisions. | - |
| 111 | Planning of the FSS in 18/20/30 GHz | Still relevant. | NOC |
| 114 | FSS (feeder links for MSS) in 5 GHz | Still relevant; text recently updated (at WRC-03). Issue on the preliminary agenda for WRC-10, item 3.1, as per RS803. Recommendation ITU-R S.1342 in force. This Resolution is referred to in Nos. 5.444 and 5.444A. | NOC |
| 122 | HAPS in 47/48 GHz | For consideration by WRC-07 Agenda item 1.8. Modifications to Resolution under consideration in WP 4-9S. The former version of this Resolution is referred to in No. 5.552A. | - |
| 124 | Sharing FX/EESS in 8 GHz | Recommendation ITU-R F.1502 contains pfd limits different from those referred to in No. 5.462A. Future WRC to review No. 5.462A. On-going studies in SG 7 on use of the band for EESS. This Resolution is referred to in No. 5.462A. | NOC/ SUP (after the review of No. 5.462A) |
| 125 | Sharing MSS/RA in 1.6 GHz | Ongoing studies, future WRC to review. On-going studies in SG 1 (revision to Recommendation ITU-R SM.1633 or draft new Recommendation). This Resolution contains outdated reference to Recommendation RA.769 (should be 769-2). | NOC |
| 132 | FSS in 18/28 GHz | Implemented. May be considered for deletion. | SUP |
| 136 | Criteria for sharing between GSO FSS and non-GSO FSS in 37.5-50.2 GHz | Still relevant; text recently updated (at WRC-03). Issue on the preliminary agenda for WRC-10, item 2.5. Recommendation ITU-R S.1655 in force. | NOC |
| 139 | Use of FSS for provision of DTH television broadcasting | Still relevant. Studies asked for consideration of WRC-03. | NOC |
| 140 | Equivalent epfd limits in 19.7-20.2 GHz | Still relevant. Recommendation ITU-R S.1715 in force. | NOC |
| 141 | Sharing between NGSO/FSS systems and fixed service in 17.7-19.7 GHz | For consideration by WRC-07 Agenda item 1.18. | - |
| 142 | Transitional arrangements for use of the band 11.7-12.2 GHz by GSO/FSS networks in Region 2 | Resolves 1, 2 and 4 implemented; resolves 3 still relevant. | NOC/MOD |
| 143 | Guidelines for implementation of high-density applications in the FSS in identified frequency bands | Still relevant. On-going studies in SG 4. This Resolution is referred to in No. 5.516B. | NOC |
| 144 | Special requirements for operating earth stations in the FSS in the band 13.75-14 GHz | Parts still relevant (e.g. resolves 2). Recommendation ITU-R S.1712 in force. | NOC/MOD |
| 145 | Potential use of the bands 27.5-28.35 GHz and 31-31.3 GHz by HAPS in the fixed service | For consideration by WRC-07 Agenda item 1.8. Additional details in the BR Report of activities (no system published so far). This Resolution is referred to in Nos. 5.537A and 5.543A. | - |
| 146 | Transitional arrangements for the application of the modified provisions of AP30B | Implemented. Implicitly on the WRC-07 Agenda (item 1.10). | - |
| 205 | Protection of MSS in 406-406.1 MHz | Still relevant (additional details in the BR Report of activities). | NOC |
| 207 | Monitor MMS/AM(R)S | Still relevant; text recently updated (at WRC-03). Monitoring reports regularly posted on the ITU website. Additional details in the BR Report of activities. | NOC |
| 212 | Implementation of IMT-2000 | Many elements implemented. Other issues for consideration by WRC-07 (agenda item 1.4). This Resolution is referred to in Nos. 5.351A and 5.388. | SUP/MOD |
| 215 | Coordination among non-GSO MSS | Some elements are still relevant, ongoing studies in SG 8. | NOC/MOD |
| 217 | Wind profiler radars | Still relevant. On-going studies in SGs 1, 7 and 8. This Resolution is referred to in Nos. 5.162A and 5.291A. | NOC |
| 221 | HAPS for IMT-2000 in the bands around 2 GHz | Still relevant, text recently updated (at WRC-03). This Resolution is referred to in No. 5.388A. | NOC |
| 222 | Use of the bands 1 525-1 559 MHz and 1 626.5-1 660.5 MHz by the MSS | Still relevant (on the preliminary agenda for WRC-10, item 2.3). This Resolution is referred to in Nos. 5.353A and 5.357A. | NOC |
| 223 | Additional bands identified for IMT-2000 | Some elements are still relevant, ongoing studies. The subject matter is under consideration by WRC-07 (Agenda item 1.4). This Resolution is referred to in Nos. 5.384A and 5.388. | NOC/MOD |
| 224 | Frequency bands for the terrestrial component of IMT-2000 below 1 GHz. | Some elements are still relevant, ongoing studies. The subject matter is under consideration by WRC-07 (Agenda item 1.4). This Resolution is referred to in No. 5.317A. | NOC/MOD |
| 225 | Use of additional bands for the satellite component of IMT-2000 | Still relevant. Results of the studies to be reported to a future WRC. The subject matter is under consideration by WRC-07 (Agenda item 1.4). A former version of this Resolution is referred to in No. 5.348C. | NOC/MOD |
| 228 | Further development of IMT-2000 and systems beyond IMT-2000 | For consideration by WRC-07 Agenda item 1.4. | - |
| 229 | Use of bands 5150-5250 MHz, 5250-5350 MHz and 5470-5725 MHz for WAS including RLAN | Still relevant. Recommendations ITU-R M.1739, RS.1166-3 and S.1427-1 in force. Studies ongoing in several SGs. This Resolution is referred to in Nos. 5.446A, 5.447 and 5.453. | NOC/MOD |
| 230 | Mobile allocations for wideband aeronautical telemetry and associated telecommand | For consideration by WRC-07 Agenda item 1.5. | - |
| 331 | Transition arrangements for the GMDSS | For consideration by WRC-07 Agenda item 1.14. A former version of this Resolution is referred to in No. 5.82. | - |
| 339 | Coordination of NAVTEX | Still relevant; text recently updated (at WRC-03). A former version of this Resolution is referred to in No. 5.79A. | NOC/MOD |
| 340 | Additional SAR information | Largely implemented by ITU, although some elements (e.g. in the “invites” part) are still relevant. | MOD |
| 342 | Revision of AP18 | For consideration by WRC-07 Agenda item 1.14. | - |
| 343 | Certificates (vessels using GMDSS equipment on a non-compulsory basis) | Still relevant (to ensure inter-communication between SOLAS and non-SOLAS vessels). | NOC/MOD |
| 344 | Exhaustion of MMSI | For consideration by WRC-07 Agenda item 1.16. BR Report to each WRC. | - |
| 345 | Operation of GMDSS equipment on non-compulsory fitted vessels | Still relevant; ongoing activities. May need some updates in view of the completion of some actions. | NOC/MOD |
| 349 | False alerts in GMDSS | Still relevant; ongoing activities. | NOC/MOD |
| 351 | Review of channel arrangements in the maritime allocations in MF and HF bands | For consideration by WRC-07 Agenda item 1.13. | - |
| 352 | Use of carrier frequencies 12 290 kHz and 16 420 kHz for safety related calling to and from RCC | Still relevant. | NOC |
| 353 | Use of MMSI for equipment other than shipborne mobile equipment | For consideration by WRC-07 Agenda item 1.16. Implemented by BR (details in the BR Report to WRC-07). | - |
| 405 | Frequencies for AM(R) | Still relevant; ongoing activities in ICAO. | NOC |
| 413 | Use of the band 108-117.975 MHz by aeronautical services | Still relevant. (The subject matter is under consideration by WRC-07, Agenda item 1.6). This Resolution is referred to in No. 5.197A. | NOC/MOD |
| 414 | Additional allocations for aeronautical mobile (R) service in the bands between 108 MHz and 6 GHz | For consideration by WRC-07 Agenda item 1.6. | - |
| 415 | Study of current satellite frequency allocations that will support the modernization of civil aviation | For consideration by WRC-07 Agenda item 1.6. | - |
| 506 | GSO only, in BSS bands (12 GHz) | Still relevant, however may need updates to reflect the decisions of WRC-2000 and WRC-03 regarding the adoption of new Regions 1 and 3 Plans and Lists. | MOD |
| 507 | Agreements/Plans for BSS | Still relevant, text recently updated (at WRC-03). This Resolution is referred to in No. 5.311. | NOC |
| 517 | Introduction of digital and SSB modulations in the HFBC | Still relevant; text recently updated (at WRC-03). On-going studies in SG 3. This Resolution is referred to in No. 5.134. | NOC |
| 525 | Introduction of HDTV in 22 GHz | Still relevant; ongoing activities. Some updates might be necessary. A former version of this Resolution is referred to in No. 5.530. | NOC/MOD |
| 526 | Additional provisions for HDTV | Still relevant. For consideration by a future WRC. | NOC |
| 527 | Terrestrial VHF digital sound broadcasting | Some elements are obsolete (in view of the actions taken by RRC-06). Recommendation ITU-R P.1546-2 in force; propagation studies continuing in SG 3. | SUP/MOD |
| 528 | BSS (sound) in 1.5 GHz | Still relevant. For consideration by a future WRC. This Resolution is referred to in Nos. 5.417A and 5.418. A former version of this Resolution is referred to in Nos. 5.345 and 5.393. | NOC |
| 533 | Implementation of certain provisions relating to AP30/30A | Some elements still relevant (e.g. resolves 4.2). | NOC |
| 535 | Application of Article 12 | Still relevant; text recently updated (at WRC-03). | NOC |
| 536 | BSS satellites serving other countries | Obsolete (in view of the decision of WRC-2000). | SUP |
| 539 | Use of the band 2 630-2 655 MHz for non-GSO BSS | Some elements still relevant, text recently updated (at WRC-03). This Resolution is referred to in Nos. 5.417A and 5.418. | NOC |
| 543 | Provisional RF protection ratios for analogue and digital emissions in HFBC | Still relevant. Results of ITU-R studies to be reported to WRC-07. Recommendation ITU-R BS.1615 in force since 2003. Issue on the preliminary agenda for WRC-10, item 2.6. | NOC |
| 544 | Additional spectrum for HFBC | For consideration by WRC-07 Agenda item 1.13. | - |
| 545 | Technical and regulatory procedures for the BSS in the 620-790 MHz | For consideration by WRC-07 Agenda item 1.11. This Resolution is referred to in No. 5.311. | - |
| 546 | Processing of notices under AP30 and AP30A in accordance with the decisions of WRC-03 | Some elements still relevant (e.g. resolves 4.2, 4.3, 5.2 and 5.3). | NOC |
| 547 | Updating of the “Remarks” columns in AP30/30A | For consideration by WRC-03 Agenda item 7.1. BR report to WRC-07. | - |
| 548 | Application of the grouping concept in AP30/30A in Regions 1 and 3 | Still relevant. | NOC |
| 608 | Use of 1215-1300 MHz band by systems in the RNSS (space-to-Earth) | Still relevant; ongoing activities. This Resolution is referred to in No. 5.329. | NOC |
| 609 | Protection of ARNS from the equivalent epfd produced by RNSS networks and systems in the 1 164-1 215 MHz band | Still relevant; ongoing activities. This Resolution is referred to in No. 5.328A. | NOC |
| 610 | Coordination of RNSS networks and systems in the bands 1164-1300 MHz, 1559-1610 MHz and 5010-5030 MHz | Still relevant; ongoing studies in SG 1 (revision to Recommendation ITU-R SM.1633 or draft new Recommendation). This Resolution is referred to in No. 5.328B. | NOC |
| 641 | Use of the band 7 000-7 100 kHz | Still relevant. | NOC |
| 642 | Earth stations in the amateur satellite service | Still relevant. | NOC |
| 644 | Disaster communications | Still relevant (In line with ongoing studies in BR and the Study Groups). Perhaps requires updating to reflect current aspects of the topic. | NOC/MOD |
| 646 | Public protection and disaster relief | Still relevant (In line with ongoing studies in BR and the Study Groups). Perhaps requires updating to reflect current aspects of the topic. | NOC/MOD |
| 670 | Notification and protection of earth stations in the MeteoSat service in the band 1 670-1 675 MHz | Implemented. Recommendation SA.1745 (approved 2006) in force. This Resolution is referred to in No. 5.380A. | SUP |
| 703 | Interference criteria for the shared bands | Although this Resolution was updated recently at WRC-03, its implementation appears difficult and its value questionable. | SUP |
| 705 | Protection of services in 70-130 kHz | Some elements still relevant; for consideration by a future WRC. | NOC/MOD |
| 716 | Use of bands around 2 GHz | Still relevant; ongoing activities. Some updates might be necessary, progress reports to WRCs. Recommendation ITU-R F.382-8 (approved 2006) responds to this Resolution (by providing channelling arrangements). A former version of this Resolution is referred to in Nos. 5.389A, 5.389C and 5.390. | MOD |
| 728 | Non-GSO MSS in 470-862 MHz | Still relevant (in different context); ongoing activities, some updates might be necessary | MOD/SUP |
| 729 | Adaptive systems at MF/HF | For consideration by WRC-07 Agenda item 1.13. | - |
| 731 | Sharing and adjacent-band compatibility between active and passive services above 71 GHz | Still relevant; ongoing studies. Issue on the preliminary agenda for WRC-10, item 2.7. References to Recommendations RA.769 and SA.1029 out-of-date (should be 769-2 and 1029-2). | NOC |
| 732 | Sharing between active services above 71 GHz | Still relevant; ongoing studies. Issue on the preliminary agenda for WRC-10, item 2.7. Several Recommendations in force (approved 2006) concerning EESS and SRS above 71 GHz. | NOC |
| 734 | Use of HAPS in the FX/MO in the bands above 3 GHz allocated exclusively to terrestrial services | Still relevant; ongoing studies (for consideration by a future WRC). Recommendation ITU-R F.1764 (approved 2006) in force. | NOC |
| 738 | Compatibility analyses between EESS (passive) and active services | For consideration by WRC-07 Agenda item 1.20. | - |
| 739 | Compatibility between RA and active space services | For consideration by WRC-07 Agenda item 1.21. This Resolution is referred to in No. 5.347A. | - |
| 740 | Future compatibility analyses between RA and active space services | For consideration by WRC-07 Agenda item 1.21. | - |
| 741 | Protection of RA in the bands 4 990-5 000 MHz | Still relevant. (Reference to Recommendation ITU-R RA.1513 should be 1513-1). This Resolution is referred to in No. 5.443B. | NOC |
| 742 | Use of the band 36-37 GHz | For consideration by WRC-07 Agenda item 1.2. | - |
| 743 | Protection of single-dish RA stations in the band 42.5-43.5 GHz | Still relevant. This Resolution is referred to in Nos. 5.551H and 5.551I. | NOC |
| 744 | Sharing between MSS (Earth-to-space) and other services in the bands 1 668-1 668.4 MHz and 1 668.4-1 675 MHz | For consideration by WRC-07 Agenda item 1.7. This Resolution is referred to in No. 5.379D. | - |
| 745 | Protection of existing services from secondary NGSO/FSS networks around 1.4 GHz | For consideration by WRC-07 Agenda item 1.17. This Resolution is referred to in No. 5.339A. | - |
| 746 | Issues dealing with allocations to science services | For consideration by WRC-07 Agenda item 1.2. | - |
| 747 | Possible upgrade of the radiolocation service allocation in 9 000-9 200 MHz and 9 300-9 500 MHz and possible extension of the EESS (active) and SRS (active) in the band 9 500-9 800 MHz | For consideration by WRC-07 Agenda item 1.3. | - |
| 802 | Agenda for WRC-07 | Obsolete in view of the actions taken by the Council (Resolution 1227). | SUP |
| 803 | Preliminary agenda for WRC-10 | For consideration by WRC-07 Agenda item 7.2. | - |
| 900 | Review of the RoP for No. 9.35 | Parts still relevant. | NOC/MOD |
| 901 | Determination of the orbital arc separation | Parts still relevant. Implicitly under Agenda item 1.12 of WRC-07; ongoing studies in SGs 4, 6 and SC. | NOC/MOD |
| 902 | Provisions related to earth stations located on board vessels, in FSS networks in 5 925-6 425 MHz and 14-14.5 GHz | Still relevant. Recommendation ITU-R S.1587-1 in force and studies continue in SG 4. This Resolution is referred to in Nos. 5.457A, 5.457B, 5.506A and 5.506B. | NOC |
| 950 | Consideration regarding the use of frequencies between 275 and 3 000 GHz | Still relevant; ongoing studies in SGs 1, 3, 4 and 7. Issue on the preliminary agenda for WRC-10, item 2.7. | NOC |
| 951 | Options to improve the international spectrum regulatory framework | Still relevant. Progress report in the Director’s report to WRC-07, Agenda item 7.1. | NOC |
| 952 | Studies regarding devices using UWB technology | Still relevant; “Instructs the Director” implemented. Four Recommendations (approved 2006) on UWB in force and further studies foreseen. | NOC |

| Rec. No. | Subject | Remark | Possible follow-up |
| --- | --- | --- | --- |
| 7 | Standard forms for licenses | Still relevant. | NOC |
| 8 | Automatic identification | Still relevant (in the new context), ongoing studies. | MOD/NOC |
| 9 | Operation of BC stations on board ships/aircraft | Still relevant. | NOC |
| 14 | Identification of special vessels | Some aspects are obsolete; ongoing studies (e.g. Recommendation M.1371) suggest other alternatives. | SUP |
| 34 | Principles for allocation of frequency bands | Still relevant; ongoing studies. | NOC |
| 36 | International monitoring of emissions from space stations | Still relevant; ongoing studies in SG 1. | NOC |
| 37 | Operational procedures for ESV | Still relevant. Recommendation ITU-R S.1587-1 in force and studies continue in SG 4. | NOC |
| 63 | Calculation of necessary bandwidth | Still relevant (in the new context). Recommendation ITU-R SM.328-11 (approved 2006) in force; studies continue. | MOD |
| 71 | Type approval | Still relevant. | NOC |
| 75 | Study of boundary between out-of-band and spurious domains of primary radars using magnetrons | Still relevant; ongoing studies. Recommendation ITU-R SM.1541-2 (approved 2006) in force. | NOC |
| 100 | Bands for troposcatter | Still relevant; (updated at WRC-03). | NOC |
| 104 | pfd and e.i.r.p. limits | Still relevant; ongoing studies. Some elements are obsolete. | MOD |
| 316 | Use of SES within harbours | Some aspects still relevant. | NOC |
| 318 | Improved AP18 | Still relevant; ongoing studies with a view to present results to a future WRC (implicitly on the WRC-07 agenda, item 1.14) | NOC/MOD |
| 401 | Use of worldwide frequencies in AP27 | Although this Recommendation contains useful suggestions to administrations, it is observed only by a few administrations. | NOC/MOD |
| 503 | HFBC | Still relevant. | NOC |
| 506 | Harmonics in BSS | Still relevant. | NOC |
| 517 | RF PR for SSB emissions in HFBC | Implemented. | SUP |
| 520 | Elimination of out-of-band HFBC emissions | Still relevant. | NOC |
| 522 | Coordination of HFBC schedules | Still relevant. | NOC |
| 604 | Characteristics of EPIRBs | Some elements are not relevant any longer (e.g. phasing out of some types of EPIRBs). | MOD/SUP |
| 605 | Shipborne transponders | Some aspects are obsolete; ongoing studies (e.g. Recommendation M.1371) suggest other alternatives. | SUP |
| 606 | Radionav. in 4 200-4 400 MHz | Still relevant, ongoing studies with a view to present results to a future WRC. | NOC |
| 608 | Guidelines for consultation meetings established by RS609 | Still relevant | NOC |
| 622 | Sharing of bands 2 025-2 110 MHz and 2 200-2 290 MHz | Still relevant. | NOC |
| 705 | Sharing BC/BSS in 700 MHz | Still relevant, ongoing studies (some elements are obsolete). This Recommendation is referred to in No. 5.311. | MOD |
| 707 | Sharing in 32-33 GHz | Still relevant, ongoing studies with a view to present results to a future WRC. Recommendation ITU-R S.1151 in force. This Recommendation is referred to in No. 5.548. | NOC |
| 722 | Review of technical, operational and frequency issues for terrestrial wireless interactive multimedia applications | Ongoing studies. Issue for consideration by WRC-10 (agenda item 2.8) as per Resolution 803. | NOC |
| 723 | Spectrum usage and operational characteristics for ENG systems | Ongoing studies. Progress report in the Director’s report to WRC-07, Agenda item 7.1. | NOC |
| 800 | Principles for establishing agendas for WRCs | Still relevant. | NOC |

______________