| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 4A/TEMP/205 Reference: Documents 4A/278 (Annex 10) and 349 Subject: WRC-12 Agenda item 1.13, Recommendation ITU-R BO.1785 | Annex 4 to Document 4A/368-E |
|  | 31 March 2010 |
|  | English only |
| Annex 4 to Working Party 4A Chairman’s Report |  |
| PRELIMINARY DRAFT Revision of RECOMMENDATION ITU-R BO.1785 |  |
| Intra-service sharing criteria for GSO BSS systems in the band 21.4-22.0 GHz in Regions 1 and 3 |  |

Working Party 4A discussed at its meeting held 24 March – 1 April 2010 one contribution (Document 4A/349) proposing revisions to Recommendation ITU-R BO.1785 “Intra-service sharing criteria for GSO BSS systems in the band 21.4-22.0 GHz in Regions 1 and 3”.

Document 4A/349 proposed to slightly improve the wording of this recommendation and proposed to upgrade this document to a preliminary draft revision of Recommendation (PDRR).

This Recommendation was developed in 2007 with the intent to be use for intra-service sharing for GSO BSS systems in the band 21.4-22.0 GHz in Regions 1 and 3 within the coordination arc. Some administrations expressed their concerns about the necessity to develop a pfd value to be used as a hard limit outside the coordination arc.

Members of Working Party 4A are invited to study the current version of Recommendation ITU-R BO.1785, as attached, and to submit contributions to the next meeting especially on the hard limit value outside the coordination arc. In this connection, attention is drawn to the fact that there is a need to establish a pfd limit in order to protect the geostationary-satellite orbit outside the coordination arc (similar to RR Appendices 30 and 30B).

# Summary of the proposed revision

In the initial version of Recommendation ITU-R BO.1785, the pfd mask indicated in recommends 1 was derived using a reference pfd value equal to -105 (dB(W/m2 . 1 MHz)) according to the Recommendation ITU-R BO.1776 in force. It is proposed to update the pfd mask in order to consider alternative pfd values for areas not subject to high total link attenuation, as proposed in the revision of Recommendation ITU-R BO.1776.

Attachment: 1

Attachment

Preliminary DRAFT Revision of RECOMMENDATION ITU-R BO.1785

Intra-service sharing criteria for GSO BSS systems
in the band 21.4-22.0 GHz in Regions 1 and 3

(Questions ITU-R 22-1/6 and ITU-R 104/6, WRC-07 Agenda items 6 and 7.1)

(2007-2010)

Scope

This Recommendation addresses the intra-service sharing for GSO BSS systems in the band 21.4-22.0 GHz in Regions 1 and 3 that may be applied for systems operating in accordance with Resolution 525 (Rev.WRC-07).

The ITU Radiocommunication Assembly,

considering

a)	that the allocation to the broadcasting-satellite service (BSS) in the band 21.4-22.0 GHz in Regions 1 and 3 came into effect on 1 April 2007;

b)

c)	that the BSS systems in the 21.4-22.0 GHz band have the possibility to deliver wideband RF (radio-frequency) signals;

d)	that high e.i.r.p. (equivalent isotropically radiated power) may be needed to compensate for high rain attenuation;

e)	that in order to operate BSS systems in this band, new intra-service sharing criteria could be  necessary;

f)	that such sharing criteria can be expressed as a power flux-density (pfd) mask;

g)	that to generate a pfd mask a reference co-polar antenna pattern is required;

h)	that the wanted pfd at the Earth’s surface of a BSS system depends, inter-alia, on the required annual service availability and the modulation scheme;

j)	that there is no established reference receiving earth station antenna pattern in the band 21.4-22.0 GHz;

k)	that in the absence of an established reference pattern for this band, Recommendation ITU-R BO.1213 could be used,

recommends

1	that the following pfd levels should be used, under assumed free-space propagation conditions, for reference for sharing among GSO BSS systems in the band 21.4-22.0 GHz-:

pfdwanted –38.2    dB(W/(m2 ·1 MHz))	for  0°	   0.27°

pfdwanted –26.8 + 20 log     dB(W/(m2 ·1 MHz))	for  0.27°	   °

pfdwanted –29.7 + 3.12 2    dB(W/(m2 ·1 MHz))	for  1.18°	   °

pfdwanted –19.5 + 25 log     dB(W/(m2 ·1 MHz))	for  2.59°	   °

pfdwanted    dB(W/(m2 ·1 MHz))	for  6.03°	 

where pfdwanted is the pfd at the Earth’s surface of the wanted BSS system, as indicated in Recommendation BO.1776, taking into account the required annual service availability and the modulation scheme;

where  is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective east-west station-keeping accuracies;

NOTE 1 – Appendix 1 gives the derivation of the pfd values given in recommends 1 above.

NOTE 2 – The pfd values in recommends 1 were derived using the Recommendation ITU-R BO.1213 co-polar antenna pattern.

Appendix 1

Derivation of the pfd values

The pfd values for sharing within the BSS in the band 21.4-22.0 GHz are derived by the C/I approach.

The protection of the antennas ranging from 45 cm to 120 cm is based on the assumption that the wanted pfd is reduced dB per dB with increasing receive antenna gain, in such a way that the performance of the link is maintained constant. The protection requirement is set to be equal to the wanted pfd of the BSS system (i.e.  according to recommends 1 or lower value according to recommends 2 at the orbital separation of 6 degrees; hence the protection criterion becomes a C/I single entry of 29.7 dB. The permissible interfering pfd limit is calculated with the same formula in all cases:

Interfering pfd limit (= pfdwanted – 29.7 + G (

where G ( is the off-axis angular discrimination for the corresponding topocentric angle ( = 1.1 ) for the antenna size considered. This pfd mask is shown in Fig. 1. This pfd mask is the lower bound of the group of curves that are shown in Fig. 2.

FIGURE 1

Example of threshold pfd for the BSS in the band 21.4-22.0 GHz
(C/I requirement = 29.7 dB, pfdwanted = –105 dB(W/(m2 · MHz)))

FIGURE 2

Example of protection requirements for 45 cm to 120 cm antennas
(C/I requirement = 29.7 dB, pfdwanted = –105 dB(W/(m2  MHz)))

_____________