| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/195 | Annex 10 to Document 5A/421-E |
|  | 2 December 2013 |
|  | English only |
| Annex 10 to Working Party 5A Chairman’s Report |  |
| Working document towardS a preliminary draft new Report ITU-R М.[5 MHz COMPAT] |  |
| Compatibility analysis of possible amateur systems with fixed, land mobile, maritime mobile and radiolocation services in the frequency band 5 250-5 450 kHz and the aeronautical mobile service in an adjacent band |  |

Editor’s Note:  In reviewing the compatibility studies attached to this working document, it was discussed that the results need to be reviewed in order to correctly portray the potential impact on Fixed Service use.  Three options were discussed as a potential solution and one of these methods should be applied by the study authors in revisions to the next meeting of WP 5A.

Option 1:  Calculate SNR of wanted link (must meet S/N requirements discussed below to indicate a successful link), subtract value of SNR for interfering amateur link (must also meet requirement to be a successful link), compare resulting S/I against the required link margin for the fixed service as shown in Recommendations ITU-R F.1761 (Annex 1, Table 4, required S/N), ITU-R F.1762 (Annex 1, Table 1, required S/N), and ITU-R F.1821 (Annex 1, Table 1, required S/N).  Although the protection criteria in these tables are an S/N ratio, it also represents the required C/I to successfully close a link at the required reliability levels, taking into account the overall system S/N protection criteria found in Recommendation ITU-R F.339.

Option 2:  Calculate SNR of wanted link, subtract SNR of interfering amateur link, convert resulting S/I into S/N ratio and compare to S/N protection criteria of Recommendation ITU-R F.339.  Availability objectives for HF systems would also have to be compared to determine impact to long-term and short-term objectives.

Option 3:  Calculate SNR of wanted link, subtract SNR of interfering amateur link, convert resulting S/I into Signal-to-Interference + Noise (SINR) taking into account the necessary bandwidth.  Calculate resulting SINR from S/N protection criteria in Recommendation  ITU-R F.339 and compare to determine potential impact on fixed/mobile service links, also taking into account availability objectives.

Guidance for conducting compatibility studies has been provided by Working Party 5C in Document 5C/TEMP/133 expressing a preference for Option 1.

# 1	Introduction

The frequency band 5 250-5 450 kHz is allocated to the fixed and mobile services excluding aeronautical mobile), and in the band 5 250-5 275 kHz to the radiolocation service for oceanographic radar purposes on a non-interference basis to the allocated services in accordance with Resolution 612 (Rev.WRC-12).

Resolution 807 (WRC-12) includes, as agenda item 1.4 for WRC-15 “1.4 to consider possible new allocation to the amateur service on a secondary basis within the band 5 250-5 450 kHz in accordance with Resolution 649 (WRC-12)”.

The high frequency (HF) band supports propagation of radio signals via a reflected path incident on the ionosphere. This is called ionospheric or sky-wave propagation. Because of this unique characteristic, an important feature of the HF spectrum is its ability to support long range communications via sky-wave propagation. However, one disadvantage of sky-wave propagation is the probability that noise and interference from distant sources may affect a desired communication. Therefore, the usability of HF sky-wave communication channels depends on both signal propagation and the absence of excess noise and interference.

This Report examines the potential for interference from stations of the amateur service operating in the frequency range 5 250-5 450 kHz to operation of the systems of fixed, radiolocation, and mobile (except aeronautical mobile) services. This Report also considers potential mitigation techniques, while evaluating the compatibility of these services.

This Report discusses compatibility aspects for frequency use in this band.

# 2	Related ITU-R Recommendations and Reports

Editor’s note: Extracted from Doc. 5A/254

Recommendation ITU-R F.339-8 – Bandwidths, signal-to-noise ratios and fading allowances in HF fixed and land mobile radiocommunication systems

Recommendation ITU-R P.368-9 – Ground-wave propagation curves for frequencies between 10 kHz and 30 MHz

Recommendation ITU-R P.372-10 – Radio noise

Recommendation ITU-R P.525-2 – Calculation of free-space attenuation

Recommendation ITU-R P.533-11 – Method for the prediction of the performance of HF circuits

Recommendation ITU-R P.832-3 – World Atlas of Ground Conductivities

Recommendation ITU-R F.1610 – Planning, design and implementation of HF fixed service radio systems

Recommendation ITU-R M.1732-1 − Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies

Recommendation ITU-R F.1761 – Characteristics of HF fixed radiocommunication systems

Recommendation ITU-R F.240-7 – Signal to interference protection ratios for various classes of emissions in the fixed service below about 30 MHz

Recommendation ITU-R F.162-3 – Use of directional transmitting antennas in the fixed service operating in bands below about 30 MHz

Recommendation ITU-R BS.705-1 – HF transmitting and receiving antennas characteristics and diagrams

Recommendation ITU-R M.1874 – Technical and operational characteristics of oceanographic radars operating in sub-bands within the frequency range 3-50 MHz

Recommendation ITU-R SM.1541-5 –Unwanted emissions in the out-of-band domain

Recommendation ITU-R SM.329-12 –Unwanted emissions in the spurious domain

Report ITU-R M.2234 – The feasibility of sharing sub-bands between oceanographic radars operating in the radiolocation service and fixed and mobile services within the frequency band 3-50 MHz

[Preliminary] draft new Report ITU-R M.[5 MHz CHAR] –Characteristics of amateur radio stations in the range 5 250-5 450 kHz for sharing studies

Report ITU-R M.2080 – Consideration of sharing conditions and usage in the 4-10 MHz band

ITU-R Handbook – Ionosphere and its Effects on Radiowave Propagation Handbook, Edition 1998

# Editor’s note: Any other documents that should be added?3	Abbreviations

Editor’s note: Extracted from Doc. 5A/254

AS	Amateur service

BW	Bandwidth

FS	Fixed service

LBT	“Listen-Before-Transmitting” protocol used by amateur operators

LUF	Lowest Usable Frequency

MUF	Maximum Usable Frequency

NVIS	Near-Vertical Incidence Skywave

OOB	Out Of Band

SNR	S/N, Signal-to-Noise Ratio

SNIR	S/(N+I), Signal-to-Noise plus Interference Ratio

SSB	Single-Sideband Suppressed Carrier Analogue Voice Transmission

SSN	Smoothed Sunspot Number

UTC	Coordinated Universal Time

# 4	Background

Editor’s note: Extracted from Doc. 5A/254

The frequency range 5 250-5 450 kHz is allocated to fixed and mobile (except aeronautical mobile) services in all three Regions on a primary basis. Radiolocation services are also allocated in the range 5 250 to 5 275 kHz as a secondary service in Regions 1 and 3 and Primary in Region 2.

The 5 250-5 450 kHz frequency range is actively used by systems in the fixed, land mobile and maritime mobile services. Analysis of the Master International Frequency Register shows that 13 314 frequency assignments in the fixed service, 2 104 frequency assignments to base statins in the land mobile service, 251 frequency assignments to transmitting coast stations in the maritime mobile stations and 14 frequency assignments to receiving stations in the maritime mobile stations are recorded therein. Locations of the stations notified in the frequency range considered are shown in Figure 4.1.

Figure 4.1

Locations of stations operating in the frequency band 5 250-5 450 kHz

Such intensive usage of the frequency band considered stems primarily from the fact that it is often  not feasible to deploy traditional mobile communication networks and satellite communication stations in large sparcely populated, inaccessible and distant areas of the globe including those in the Arctic and Antarctic regions.

Effective (and solitary in some cases) solution of the issue related to providing communication with such areas seems to be based on employing multi-hop communication links established within the fixed and land mobile services. Such links transfer signals based on multiple reflection of propagating electromagnetic waves from the earth surface and its ionosphere.

Frequency assignments notified in the fixed and land mobile services provide for data transfer in voice and telegraph modes. They may be used for different purposes such as permanent communication with sparsely populated, inaccessible and distant areas and establishing communications in areas of natural disasters and recovery operations.

Frequency assignments in the maritime mobile services provide for voice communications and data transfer [and are used for maritime safety.]

Amateur service characteristics are similar to land mobile service with respect to antenna types,  modulation, and transmission bandwidths. An allocation on a secondary basis to the Amateur Radio Service would facilitate providing communications in disaster situations and during relief operations at times and for distances which might not be possible using existing amateur allocations.

This report analyses the practicability of amateur operations on a secondary basis in the range 5 250 to 5 450 kHz with respect to the existing users.

As an example stations in the amateur service operating in frequency spectrum shared with other users might be the  range 10 100-10 150 kHz which is allocated to the fixed service on a primary basis and the amateur service on a secondary basis (but in some countries, the band is allocated to the amateur service on an exclusive basis). No reports of harmful interference to the fixed service are known to have been filed in the thirty years during which the allocation has been available to the amateur service. Noting that the propagation conditions around 5300 kHz are different to that around 10 000 kHz.

It should be noted that there are already a number of administrations who have authorized amateur operations subject to various restrictions in this frequency range under the provisions of Article 4.4, of the Radio Regulations.

Editor’s note: Indication of incumbent usage could go here if desired

# 5	Characteristics of fixed service stations used in the compatibility study

Editor’s note: The following is an extract from liaison statement Document 5A/77

Appropriate fixed service characteristics for sharing studies between fixed service and amateur service stations can be found in Recommendations ITU-R F.1761, ITU-R F.1762 and ITU-R F.1821.

Protection criteria for systems in the fixed service are provided in Recommendation ITU-R F.339-8.

The characteristics of fixed service stations in study below are:

−	Receiver bandwidth: 300 Hz, 3 kHz;

−	Receiver antenna gain: 0, 6, 13.4 dBi.

−	Transmitter  antenna gain: 0, 13.4 dBi.

Editor’s note: Other information to be added if required

# 6	Characteristics of amateur service stations that might be used in the 5 250-5 450 kHz frequency band

Editor’s note: Extracted from Doc. 5A/237

WP 5A has developed a draft Report ITU-R M.[5 MHz CHAR] which contains the list of possible amateur service station parameters in this frequency band. In accordance with this draft Report the majority of commercial amateur equipment has output power of 100-150 W and uses near-omnidirectional antennae.

Therefore in the compatibility study it was assumed that amateur station uses omnidirectional antenna and transmitter with output power of 100 W (20 dBW).

Editor’s note: Other information to be added if required

# 7	Scenarios for the possible impact upon the fixed/[land] mobile service by stations of the amateur service

Editor’s note: For each of the methods the geographic details of the modeled fixed service and potential amateur service stations should go here. As far as possible the links should be representative of real situations.

## 7.1	Scenarios for study method 1

The conducted studies assumed that amateur service transmitter and receiver might be arbitrarily deployed in relation to a receiver in the fixed service. Therefore the study assumed an interference scenario as shown in Figure 7.1.1 below.

Figure 7.1.1

Scenario of interference affecting a FS link

## 7.2 	Scenarios for study method 2

Editor: This section was from section 5 Executive Summary

A hypothetical propagation condition was simulated and analysed following ITU-R recommended procedures, viz., a 1 546 km fixed-link circuit and with potential interference from an amateur service link of comparable length.

Signal-to-Noise (SNR) and Signal-to-Noise-plus-Interference (SNIR) median values were computed for each of the twenty-four hours in each of the twelve months under assumed Smoothed Sunspot Numbers of 10 and 80.

Interference to the fixed link was shown to be very unlikely and this without necessarily taking account of the amateur radio protocol of “listen before transmitting” and the proposed Secondary Service nature of the sought allocation.

## 7.3	Scenarios for study method 3

The objective of this study is to evaluate the sky-wave interference from potential amateur service transmitters to worldwide fixed/land mobile service receivers. Seven amateur service sites were selected in order to provide a distribution of sites that would reflect various geographic regions and climatic zones. The selected sites are shown in Figure 7.3.1.

The fixed service receivers are distributed on the crosspoints of a 180×180 grid equally spaced in the latitude and the longitude around the globe. As for the land mobile service receivers, these crosspoints can be seen as the transient locations when they are moving.

FIGURE 7.3.1

Sites of the amateur stations and grid of fixed/land mobile service receivers

# 8	Methodology of interference impact assessment from amateur service stations on the fixed/land mobile service, aeronautical and oceanographic radar services

## 8.1	Methodology of interference impact assessment proposed by WP 5C

The impact of the considered amateur station transmission links on operation of the fixed service links was determined in accordance with the following methodology:

–	for the amateur station the S/N ratio [dB] as a function of daytime and season was determined for minimum and maximum sunspot numbers;

–	for the fixed service links the S/N ratio [dB] as a function of daytime and season was also determined for minimum and maximum sunspot numbers;

–	for the fixed service links the signal/interference ratio [S/I, dB] as a function of daytime and season was also determined for various levels of solar activity;

–	the obtained dependence of S/I ratio was compared with the required S/N value and the operation modes and time intervals when operation of the fixed service links is feasible were estimated.

## Editors note: Russian & Canadian input

### 8.1.1	Interference impact assessment from amateur service stations on the fixed/land mobile service

Editor’s note: The following is from liaison statement Document 5A/77

Recommendation ITU-R P.533 and the associated software model should be used for performing required sharing and compatibility studies.

Fixed HF systems typically use directional antennas but are not limited to such design.  Therefore studies should take into account a mixture of directional systems using yagi antennas at the maximum gain listed, and omni-directional whip antennas using a gain of 0 dB.

Editor’s note: presumably whip antennas are only used for short range links?

The impact from the amateur station transmission reference link should be calculated by determining the reference link S/N ratio (in dB) for the worst month for the amateur station using a relative sunspot number for portions of the sunspot cycle corresponding to both low sunspot activity and high sunspot activity.

A reference link signal level should also be determined for the fixed service link as an actual S/N level (in dB).  The amateur reference link S/N ratio should be used to reduce the fixed service reference link S/N ratio to determine S/I and compared to the required S/N levels found in Recommendations ITU-R F.1761, ITU-R F.1762 and ITU-R F.1821 for all three transmission types to determine if it can still meet the required S/N level for all three types of service. This will determine the long-term effects of any amateur allocation.

The fixed/mobile service transmissions modes to be studied are:

300 HF1B telegraphy from Rec. ITU-R F.339 using both stable & fading S/N criteria

3k00J3E Single Sideband analogue voice Rec. ITU-R F.339 using both stable & fading S/N criteria

[3k00J2E] Digital voice Recs. ITU-R F.1821 & ITU-R F.339 using both stable & fading S/N criteria

Editorial note: check designation for digital voice

The amateur  service transmissions modes to be used is any mode with a bandwidth up to 2.8 kHz

This would provide results for a number of scenarios as shown in the example table below:

Fixed reference links for evaluating potential interference from amateur allocations at 5 MHz

| Fixed reference link | Antenna | Sunspot number |
| --- | --- | --- |
| <Transmit> to <Receive> | Yagi | <Minimum value> |
| <Transmit> to <Receive> | Yagi | <Maximum value> |
| <Transmit> to <Receive> | Omni-Directional | <Minimum value> |
| <Transmit> to <Receive> | Omni-Directional | <Maximum value> |

Systems from all three characteristic Recommendations should be evaluated.

Editor’s note: The following paragraph is useful for clarity & is from Doc. 5A/257

In accordance with the protection criteria specified in Recommendation ITU-R F.339 the calculation of the indicated values is carried out for the reference bandwidth of 1 Hz.

- Use of omnidirectional antennas is inefficient on longer paths, such as those analysed in the tables.  Use of an antenna with some directionality, such as a horizontal dipole or Yagi, is common practice in both the amateur and fixed services.

- The methodology presented in this analysis provides an overall assessment of the potential impact to the fixed service from potential amateur operations in the 5 250-5 450 kHz frequency range.  However, to determine the impact to a specific link at any given time, the use of optimum frequency for the time of day, desired path, season and solar activity must be taken into account. The statistical probability of interference is based on a number of propagation factors and the impact will be strongly determined by link reliability requirements and required signal-to-noise ratios for type of service.

### 8.1.2	Interference impact assessment on the aeronautical service

### 8.1.3	Interference impact assessment on oceanographic radar systems

Previous ITU-R studies have found amateur service sharing with HF oceanographic radars in the radiolocation service “seems to be difficult . . . .”  For this reason, a secondary allocation to the amateur service within the frequency band 5 250-5 275 kHz should not be established, in consideration of the allocation to radiolocation established at WRC-12.

## 8.2	Methodology of interference impact assessment from amateur service stations on the fixed/land mobile service based on I/N criteria.

This methodology is used in the study 3 and based on I/N criteria, and -6 dB is assumed to be the interference threshold for fixed/land mobile service receivers where N is external noise.

To each fixed/land mobile service receiver on the grid, the received I/Ns at 5 350 kHz on various date and time are calculated. Some of the calculated I/Ns may exceed the specified -6 dB interference threshold.

For each amateur station depicted in figure 1, the distributed area and the percentage of interfered fixed/land mobile service receivers for which the interfered time exceeding 20% will be evaluated.

### 8.2.1	Calculation of co-band interference from an amateur station

The calculation of the interference via the ionospheric sky-wave propagation is based on the NTIA/ITS  HF propagation model ICEAREA, which predicts the expected performance of HF systems for any month, different sunspot activities, hours of the day, and geographic location.

The interference power I received by the fixed/land mobile systems can be calculated as:

I = PT + GT – L + GR

Where:

PT 	is the transmission power from the amateur station, 20 dBW;

GT 	is the transmitter antenna gain of the amateur station, 0 dBi;

L 	is the sky wave propagation loss, dB;

GR 	is the receiver antenna gain of the fixed/land mobile system, 0 dBi.

The sky wave propagation loss L is calculated based on the NTIA/ITS HF propagation model ICEAREA (which is slightly different to Rec. 533) , which predicts the expected performance of HF systems under various conditions of geographic location,  frequency, time, date, smoothed sunspot number (SSN) and geomagnetic index (Qindex). It is assumed that the frequency is 
5 350 kHz, the sunspot number (SSN) is 10, 50 and 100, the Qindex is 1, the time is one of the 
4-hour-time blocks in a day (i.e. 00:00-04:00, 04:00-08:00, 08:00-12:00, 12:00-16:00, 16:00-20:00, 20:00-24:00), and the date is one of the first 10 days of each season (i.e. the total number of days calculated in one year is 40),

Figure 2 displays an example of worldwide propagation loss via ionosphere layer from an amateur service transmitter in Beijing at 5 350 kHz. It is showed that the propagation loss decreases as the distance increases, the minimum value of the propagation loss is 100.9 dB and the maximum value is 672.5 dB.

Figure 2

The propagation loss at 5 350 kHz (February 15, 1800UT, SSN=100, Qindex=1) (dB)

An example of the interference from an amateur service transmitter in Beijing to the fixed/land mobile stations distributed on the grid is presented in Figure 3. It is showed that the maximum interference power from the amateur station is about -70 dBm and occurs at the vicinity of the amateur station.

Figure 3

Example of the calculated interference received by fixed/land mobile stations at frequency 
5 350 kHz (February 15, 1800UT, SSN=100, Qindex=1) (dBm)

### 8.2.2	 Calculation of external noise at an fixed/land mobile station

As stated in Report ITU-R M.2234, the external noise is the dominant limiting factor for HF communications. The calculation of the external noise at the fixed/land mobile stations is based on the software NOISEDAT developed by ITU-R SG 3, which enables one to calculate man-made noise, galactic noise, atmospheric noise, and the overall external noise.

Recommendation ITU-R P.372-10 provides data of radio noise external to the radio receiving system which derives mainly from three causes: man-made noise, galactic noise, and atmospheric noise.

Man-made noise depends on the frequency and the environment. Section 5 in Recommendation ITU-R P.372-10 details how to derive median values of man-made noise Nman  in various environments:

Nman = c-d log f-204 (dBW/Hz)

Where f (MHz) is the operating frequency, and c and d are environment -dependent parameters (Table 1).

TABLE 1

Values of the constants c and d

| Environmental category | C | d |
| --- | --- | --- |
| City | 76.8 | 27.7 |
| Residential | 72.5 | 27.7 |
| Rural | 67.2 | 27.7 |
| Quiet rural | 53.6 | 28.6 |

The “rural” and “quiet rural” environment will be considered in this study.

As the man-made noise, Galactic noise Ngal can also be obtained by the above formula, where c=52.0, d=23.0. The galactic noise component will not be observed at frequencies below the ionospheric critical frequency foF2.

Atmospheric noise Natm depends on frequency, time and date, and its value is given by a series of charts in section 7 of Recommendation ITU-R P.372-10.

Therefore the external noise N can be obtained as:

N=10*log10 (10Ngal/10+10Nman/10+10Natm/10)

Examples of external noise power in 3 kHz bandwidth in rural and quiet rural environment at each fixed/land mobile station is presented in Figure 4 and Figure 5.

Figure 4

External noise at each fixed/land mobile station at 5 350 kHz in rural environment (Winter, 1600-2000UT)

Figure 5

External noise at each fixed/land mobile station at 5 350 kHz in quiet rural environment (Winter, 1600-2000UT)

### 8.2.3	Calculation of adjacent band interference from amateur stations to fixed/land mobile service receivers

This section provides the evaluation methodology of interference from the adjacent band emissions of the amateur stations to the fixed/land mobile service receivers.

The adjacent band interference power I from amateur stations to fixed/land mobile service receivers can be calculated as:

I = PAdj + GT – L + GR

Where PAdj is the adjacent band emission power of amateur stations, other parameters are same as listed in section 8.2.1.

The required Out-of-Band and spurious emission limits of amateur stations below 30 MHz for narrow-band case (necessary band  4 kHz) can be found in Recommendation ITU-R SM.1541-5 and Recommendation ITU-R SM.329-12, as shown in Figure 6. It is assumed that the channel bandwidth is 3 kHz for both amateur and fixed/land mobile service station.

[

FIGURE 6

Adjacent band emission limits for amateur stations operating below 30 MHz in the narrow-band case

According to Figure 6, the adjacent band emission power of the amateur stations in adjacent channels can be evaluated, as shown in Table 2.

Editor note: China will review/recalculate the table below due to concerns about values shown

TABLE 2

Adjacent band emission power of the amateur stations

| Adjacent channel from the operation channel of the amateur stations | Operating channel | 1st | 2nd | 3rd | 4th |
| --- | --- | --- | --- | --- | --- |
| Emission power (dBW) | 20 | 17.0 | 2.6 | -14.0 | -35.2 |

]

Editor note: From section 6 of the Canadian studybut seems to be appropriate for section 8 of final report

# 6	Compatibility of the amateur systems with the fixed systems

The following is a compatibility analysis between stations operating fixed communication links with stations in the amateur radio service.

The received signal strength and Signal-to-Noise Ratios (SNR) from the fixed and amateur links are calculated following the methodology in Recommendation ITU-R P.533-11 as implemented in the software program REC533. Parameters  used are detailed in Appendix 1.

The following analysis assumes operation of both the fixed and the amateur links at 5 300 kHz using Single-Sideband Suppressed-Carrier (SSB) (Emission Designator 2K70J3E) analogue voice employing a nominal bandwidth of 3 kHz.

In accordance with Recommendation ITU-R F.1761, the assumed required signal-to-noise ratio for the fixed link for SSB is taken as 21 dB (from Table 1 “Required Signal-to-Noise Ratios” of 
ITU-R F339-8,the average RF SNR is 48 dB/Hz for J3E telephony single-sideband suppressed carrier, BW of 3 000 Hz, and fading non-diversity). For the amateur link the path is considered “usable” with a SNR value of 14 dB (SNR in 3 000 Hz BW) or more.ITU-R guidance on the choice of operating frequencies is that a HF skywave radio communication link will be operating with best performance between -25% to +10% of the maximum usable frequency (MUF). Operation 10% above the MUF is likely to have unsatisfactory performance and operation at frequencies -25% below MUF is likely to have inadequate signal-to-noise ratios. Thus it might be assumed that a wanted signal would only need to be considered for protection if the 5 300 kHz is within operational range of frequencies at that time. Otherwise it might be assumed that the circuit would actually be operating elsewhere in the HF spectrum. The important parameter is the overall circuit reliability - the ratio of the wanted signal level to the power sum of the combined levels of external radio noise, as given in Recommendation ITU-R P.372, together with the power level of the interference (the S/(I+N) ratio).

For the periods during which the fixed link (Primary Service) is operating, the amateur service would observe SNIR at the receiving end. (Here “I” would be the signal power of the fixed service link). During those time periods when the fixed service is not transmitting the AS would observe only AS SNR.  Both the fixed and the amateur link noise levels (N) are calculated from the received signal-to-noise ratio and signal power at receiver sites in Makkovik.

Taking note of the amateur service signal plus interference values (SNIR - as tabulated in Tables 3d and 4d), if these are such that the 14 dB criterion (in our assumption for 3 kHz SSB) is not attained, then it will be assumed that the secondary service (amateur link) will not transmit. Indeed, these Tables show that – generally - the 14 dB criterion is not attained when the channel is occupied by the fixed link, and the SNIR level is below 0 dB almost all the operating hours for both tables.

Note is also made of the calculated value of the Maximum Usable Frequency (MUF) in each of the time slots, that is, the MUF should fall within a range from 25% below the planned frequency to 10% above. For our study calculations at 5.3 MHz this range corresponds to 4.77 to 6.66 MHz. It is assumed that the fixed-link would be able to adjust accordingly.

Calculations employing Recommendation ITU-R P.533 were completed for assumed Sunspot Numbers of ten (SSN = 10) and eighty (SSN = 80) and for each of twenty-four hours per day for each month of the year.

## 6.1	Study Case: Pangnirtung NU to Makkovik NL

The following section details an analysis of the interference potential to a fixed service link between Pangnirtung, Nunavut (NU) and Makkovik, Newfoundland and Labrador (NL) and an amateur service link from Moosonee, Ontario (ON) to Makkovik – all in Canada. Table 1 shows details of the hypothetical stations. In this analysis both links terminate in the same location (Makkovik) and the noise level is assumed to be identical for both.

Table 1

Geographic details of the transmitters

| Location | Latitude | Longitude | Output Power | Antenna | Path Length |
| --- | --- | --- | --- | --- | --- |
| Pangnirtung NU (Canada) | 66.68° N | 65.68° W | 37 dBW | Log Periodic | 1 546 km |
| Makkovik NL (Canada) | 55.15° N | 59.17° W | 37 dBW | Log Periodic FS Horizontal dipole AS | NA |
| Moosonee ON (Canada) | 51.28° N | 80.10° W | 20 dBW | Horizontal dipole | 1 436 km |

## 6.2	Antenna Assumptions

The fixed link is assumed to be using directional Log periodic antennas having the following characteristics, radiation patterns are shown in Figures 1 and 2.

FIGURE 1

FS log periodic antenna vertical radiation pattern

FIGURE 2

FS log periodic antenna azimuthal radiation pattern

In the study case which follows the Pangnirtung fixed-link antenna is assumed aimed at Makkovik and vice-versa.

The amateur link is assumed to be dipole antennas. The antenna height is 17.6% of the wavelength above ground which makes it act like an NVIS antenna. The radiation pattern is more suitable for medium to short ranges and the radiation patterns are shown in figures 3 and 4.

FIGURE 3

AS dipole antenna vertical radiation pattern

FIGURE 4

AS dipole antenna azimuthal radiation pattern

## 6.3	Study Data Tables

Table 3a details the computed S/N values at Makkovik from Pangnirtung for the fixed service link assuming a Sunspot Number of ten.

Table 3b details the computed SNIR values at Makkovik from Pangnirtung for the fixed service link assuming concurrent operation of the amateur service only when the SNIR observed by the amateur link is 14 dB or greater and with a Sunspot Number of ten. The Interference term (I) in SNIR is the received amateur service signal.

Table 3c details the computed S/N values at Makkovik from Moosonee for the amateur service link assuming a Sunspot Number of ten.

Table 3d details the computed SNIR values at Makkovik from Moosonee for the amateur service link assuming concurrent operation of the fixed service link and a Sunspot Number of ten. The Interference term (I) in SNIR is the received fixed service signal.

Tables 4a, 4b, 4c and 4d repeat the data formats of Tables 3a, 3b, 3c and 3d assuming, however, a Sunspot Number of eighty.

The convention in Tables 1 to 4 is that the hour shown as Hour 1 corresponds to the period 0001 to 0100 UTC, etc. Values shown in the tables are mean values.

Colour coding of the data elements and data cell backgrounds in the following Tables is provided in Table 2.

Table 2

Colour conventions for tables 3 and 4 (a,b,c and d)

| Font colour black: | 5.3 MHz operation frequency falls outside - 25% and +10% of MUF(applies to FS data only) |
| --- | --- |
| Font colour yellow: | 5.3 MHz operation frequency falls inside - 25% and +10% of MUF (applies to FS data only) |
| Cell background green: | Communication link usable (21 dB for fixed-link or 14 dB for amateur-link) |
| Cell background red: | Communication link not usable |

[Editor’s Note: Tables 3a, 3b, 3c, 3d, 4a, 4b, 4c, 4d from the May 2013 version have been deleted and replaced with the following updated tables.]

Table 3a

FS SNR - Pangnirtung > Makkovik for Sunspot Number = 10     (FS >21 dB) MUF (4.77 and 6.66 MHz in yellow )

| UTC | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 30 | 32 | 32 | 31 | 31 | 32 | 24 | 10 | 9 | 13 | 23 | 23 | 25 | 33 | 30 | 30 | 32 | 34 | 37 | 37 | 42 | 40 | 29 | 30 |
| February | 32 | 33 | 31 | 30 | 29 | 25 | 22 | 10 | 9 | 10 | 21 | 14 | 19 | 28 | 25 | 25 | 27 | 30 | 33 | 34 | 38 | 40 | 30 | 30 |
| March | 26 | 20 | 21 | 22 | 20 | 17 | 18 | 15 | 19 | 23 | 7 | 12 | 10 | 21 | 20 | 20 | 25 | 26 | 29 | 30 | 34 | 37 | 35 | 30 |
| April | 27 | 18 | 21 | 16 | 20 | 19 | 22 | 22 | 22 | 16 | 13 | 10 | 9 | 21 | 21 | 21 | 24 | 25 | 27 | 28 | 30 | 33 | 31 | 31 |
| May | 28 | 22 | 18 | 17 | 24 | 23 | 25 | 20 | 20 | 21 | 11 | 9 | 8 | 19 | 18 | 17 | 21 | 22 | 23 | 24 | 27 | 29 | 27 | 28 |
| June | 29 | 29 | 28 | 28 | 25 | 22 | 25 | 31 | 29 | 28 | 24 | 23 | 21 | 21 | 20 | 20 | 21 | 22 | 23 | 23 | 25 | 28 | 27 | 28 |
| July | 30 | 29 | 29 | 23 | 22 | 21 | 24 | 25 | 26 | 27 | 23 | 22 | 21 | 22 | 21 | 21 | 22 | 23 | 24 | 24 | 26 | 28 | 28 | 29 |
| August | 32 | 32 | 25 | 24 | 26 | 25 | 26 | 28 | 26 | 27 | 25 | 22 | 21 | 22 | 22 | 22 | 23 | 24 | 25 | 26 | 28 | 30 | 30 | 32 |
| September | 25 | 18 | 20 | 22 | 25 | 22 | 19 | 14 | 19 | 16 | 11 | 9 | 9 | 20 | 19 | 18 | 22 | 23 | 25 | 26 | 29 | 30 | 28 | 31 |
| October | 24 | 19 | 20 | 22 | 30 | 23 | 19 | 13 | 18 | 23 | 16 | 18 | 15 | 25 | 23 | 23 | 27 | 29 | 31 | 32 | 35 | 37 | 33 | 28 |
| November | 25 | 21 | 21 | 21 | 29 | 25 | 20 | 13 | 13 | 20 | 14 | 20 | 19 | 29 | 27 | 26 | 30 | 32 | 34 | 36 | 39 | 38 | 26 | 25 |
| December | 31 | 33 | 29 | 33 | 32 | 28 | 22 | 10 | 9 | 9 | 26 | 22 | 26 | 34 | 31 | 30 | 32 | 34 | 37 | 38 | 41 | 32 | 30 | 31 |

Table 3b

Conditional FS SNIR - Pangnirtung > Makkovik for Sunspot Number = 10     (AS LBT > 14 dB) MUF (4.77 and 6.66 MHz in yellow)

| UTC | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 30 | 32 | 32 | 31 | 31 | 32 | 24 | 10 | 9 | 13 | 23 | 23 | 25 | 33 | 30 | 30 | 32 | 34 | 37 | 37 | 42 | 40 | 29 | 30 |
| February | 32 | 33 | 31 | 30 | 29 | 25 | 22 | 10 | 9 | 10 | 21 | 14 | 19 | 28 | 25 | 25 | 27 | 30 | 33 | 34 | 38 | 40 | 30 | 30 |
| March | 26 | 20 | 21 | 22 | 20 | 17 | 18 | 15 | 19 | 23 | 7 | 12 | 10 | 21 | 20 | 20 | 25 | 26 | 29 | 30 | 34 | 37 | 35 | 30 |
| April | 27 | 18 | 21 | 16 | 20 | 19 | 22 | 22 | 22 | 16 | 13 | 10 | 9 | 21 | 21 | 21 | 24 | 25 | 27 | 28 | 30 | 33 | 31 | 31 |
| May | 28 | 22 | 18 | 17 | 24 | 23 | 25 | 20 | 20 | 21 | 11 | 9 | 8 | 19 | 18 | 17 | 21 | 22 | 23 | 24 | 27 | 29 | 27 | 28 |
| June | 29 | 29 | 28 | 28 | 25 | 22 | 25 | 31 | 29 | 28 | 24 | 23 | 21 | 21 | 20 | 20 | 21 | 22 | 23 | 23 | 25 | 28 | 27 | 28 |
| July | 30 | 29 | 29 | 23 | 22 | 21 | 24 | 25 | 26 | 27 | 23 | 22 | 21 | 22 | 21 | 21 | 22 | 23 | 24 | 24 | 26 | 28 | 28 | 29 |
| August | 32 | 32 | 25 | 24 | 26 | 25 | 26 | 28 | 26 | 27 | 25 | 22 | 21 | 22 | 22 | 22 | 23 | 24 | 25 | 26 | 28 | 30 | 30 | 32 |
| September | 25 | 18 | 20 | 22 | 25 | 22 | 19 | 14 | 19 | 16 | 11 | 9 | 9 | 20 | 19 | 18 | 22 | 23 | 25 | 26 | 29 | 30 | 28 | 31 |
| October | 24 | 19 | 20 | 22 | 30 | 23 | 19 | 13 | 18 | 23 | 16 | 18 | 15 | 25 | 23 | 23 | 27 | 29 | 31 | 32 | 35 | 37 | 33 | 28 |
| November | 25 | 21 | 21 | 21 | 29 | 25 | 20 | 13 | 13 | 20 | 14 | 20 | 19 | 29 | 27 | 26 | 30 | 32 | 34 | 36 | 39 | 38 | 26 | 25 |
| December | 31 | 33 | 29 | 33 | 32 | 28 | 22 | 10 | 9 | 9 | 26 | 22 | 26 | 34 | 31 | 30 | 32 | 34 | 37 | 38 | 41 | 32 | 30 | 31 |

Table 3c

AS SNR - Moosonee > Makkovik for Sunspot Number = 10     (AS>14 dB)

| ##### UTC | ##### 1 | ##### 2 | ##### 3 | ##### 4 | ##### 5 | ##### 6 | ##### 7 | ##### 8 | ##### 9 | ##### 10 | ##### 11 | ##### 12 | ##### 13 | ##### 14 | ##### 15 | ##### 16 | ##### 17 | ##### 18 | ##### 19 | ##### 20 | ##### 21 | ##### 22 | ##### 23 | ##### 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### January | ##### 12 | 12 | ##### 16 | ##### 16 | ##### 16 | ##### 16 | ##### 16 | ##### 8 | ##### -5 | ##### -5 | ##### 8 | ##### 4 | ##### 5 | ##### -1 | ##### 5 | ##### 3 | ##### 3 | ##### 8 | ##### 12 | ##### 18 | ##### 18 | ##### 23 | ##### 18 | ##### 10 |
| ##### February | ##### 11 | ##### 13 | ##### 10 | ##### 9 | ##### 10 | ##### 10 | ##### 8 | ##### 5 | ##### -6 | ##### -6 | ##### 5 | ##### 2 | ##### 4 | ##### -1 | ##### 4 | ##### 3 | ##### 4 | ##### 8 | ##### 12 | ##### 17 | ##### 17 | ##### 21 | ##### 21 | ##### 11 |
| ##### March | ##### 9 | ##### 7 | ##### 5 | ##### 5 | ##### -4 | ##### 0 | ##### 1 | ##### 2 | ##### -1 | ##### 4 | ##### 7 | ##### 1 | ##### -3 | ##### -7 | ##### -2 | ##### -2 | ##### -2 | ##### 2 | ##### 5 | ##### 9 | ##### 11 | ##### 16 | ##### 20 | ##### 18 |
| ##### April | ##### 15 | ##### 5 | ##### 4 | ##### -4 | ##### -7 | ##### -1 | ##### -1 | ##### 2 | ##### 3 | ##### 7 | ##### 5 | ##### -2 | ##### -5 | ##### -7 | ##### -1 | ##### -2 | ##### -2 | ##### 0 | ##### 2 | ##### 5 | ##### 7 | ##### 11 | ##### 15 | ##### 16 |
| ##### May | ##### 13 | ##### 11 | ##### 3 | ##### 1 | ##### 3 | ##### 12 | ##### 15 | ##### 12 | ##### 7 | ##### 5 | ##### 5 | ##### -3 | ##### -6 | ##### -8 | ##### -2 | ##### -3 | ##### -4 | ##### -1 | ##### 1 | ##### 3 | ##### 4 | ##### 8 | ##### 11 | ##### 11 |
| ##### June | ##### 13 | ##### 14 | ##### 14 | ##### 13 | ##### 5 | ##### 7 | ##### 9 | ##### 11 | ##### 12 | ##### 13 | ##### 11 | ##### 3 | ##### 0 | ##### -3 | ##### -2 | ##### -3 | ##### -3 | ##### -1 | ##### 1 | ##### 4 | ##### 3 | ##### 6 | ##### 9 | ##### 11 |
| ##### July | ##### 14 | ##### 15 | ##### 14 | ##### 6 | ##### 5 | ##### 8 | ##### 10 | ##### 11 | ##### 11 | ##### 5 | ##### 10 | ##### 2 | ##### 0 | ##### -3 | ##### -1 | ##### -2 | ##### -1 | ##### 1 | ##### 3 | ##### 5 | ##### 3 | ##### 7 | ##### 10 | ##### 12 |
| ##### August | ##### 17 | ##### 15 | ##### 8 | ##### 8 | ##### 8 | ##### 6 | ##### 5 | ##### 6 | ##### 15 | ##### 11 | ##### 12 | ##### 4 | ##### 1 | ##### 0 | ##### 1 | ##### 1 | ##### 0 | ##### 2 | ##### 4 | ##### 7 | ##### 6 | ##### 9 | ##### 12 | ##### 15 |
| ##### September | ##### 14 | ##### 6 | ##### 5 | ##### -2 | ##### -7 | ##### 0 | ##### -2 | ##### -4 | ##### -8 | ##### 5 | ##### 3 | ##### -1 | ##### -4 | ##### -8 | ##### -2 | ##### -3 | ##### -3 | ##### 0 | ##### 2 | ##### 4 | ##### 6 | ##### 9 | ##### 13 | ##### 15 |
| ##### October | ##### 8 | ##### 8 | ##### 5 | ##### 5 | ##### 3 | ##### 5 | ##### 1 | ##### -1 | ##### -5 | ##### 5 | ##### 10 | ##### 8 | ##### 3 | ##### -2 | ##### 2 | ##### 0 | ##### 1 | ##### 4 | ##### 6 | ##### 9 | ##### 11 | ##### 15 | ##### 19 | ##### 15 |
| ##### November | ##### 9 | ##### 9 | ##### 0 | ##### 5 | ##### 5 | ##### 14 | ##### 7 | ##### 4 | ##### -3 | ##### -1 | ##### 11 | ##### 9 | ##### 7 | ##### 2 | ##### 5 | ##### 3 | ##### 2 | ##### 6 | ##### 9 | ##### 13 | ##### 15 | ##### 19 | ##### 15 | ##### 9 |
| ##### December | ##### 13 | ##### 4 | ##### 5 | ##### 6 | ##### 9 | ##### 11 | ##### 12 | ##### 11 | ##### -3 | ##### -6 | ##### 1 | ##### 6 | ##### 7 | ##### 1 | ##### 6 | ##### 3 | ##### 3 | ##### 7 | ##### 11 | ##### 17 | ##### 17 | ##### 21 | ##### 13 | ##### 11 |

Table 3d

AS SNIR - Moosonee > Makkovik for Sunspot Number = 10     (AS LBT> 14 dB)

| ##### UTC | ##### 1 | ##### 2 | ##### 3 | ##### 4 | ##### 5 | ##### 6 | ##### 7 | ##### 8 | ##### 9 | ##### 10 | ##### 11 | ##### 12 | ##### 13 | ##### 14 | ##### 15 | ##### 16 | ##### 17 | ##### 18 | ##### 19 | ##### 20 | ##### 21 | ##### 22 | ##### 23 | ##### 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### January | ##### -19 | ##### -20 | ##### -15 | ##### -15 | ##### -14 | ##### -17 | ##### -8 | ##### -4 | ##### -15 | ##### -18 | ##### -14 | ##### -20 | ##### -20 | ##### -33 | ##### -25 | ##### -27 | ##### -29 | ##### -26 | ##### -25 | ##### -20 | ##### -24 | ##### -17 | ##### -11 | ##### -19 |
| ##### February | ##### -20 | ##### -20 | ##### -22 | ##### -21 | ##### -18 | ##### -16 | ##### -14 | ##### -6 | ##### -17 | ##### -17 | ##### -16 | ##### -14 | ##### -15 | ##### -30 | ##### -21 | ##### -23 | ##### -23 | ##### -22 | ##### -21 | ##### -17 | ##### -21 | ##### -19 | ##### -8 | ##### -18 |
| ##### March | ##### -16 | ##### -13 | ##### -17 | ##### -16 | ##### -24 | ##### -17 | ##### -17 | ##### -14 | ##### -19 | ##### -18 | ##### -3 | ##### -12 | ##### -14 | ##### -28 | ##### -21 | ##### -23 | ##### -26 | ##### -24 | ##### -24 | ##### -21 | ##### -23 | ##### -21 | ##### -15 | ##### -12 |
| ##### April | ##### -11 | ##### -12 | ##### -16 | ##### -21 | ##### -28 | ##### -21 | ##### -23 | ##### -20 | ##### -19 | ##### -9 | ##### -8 | ##### -13 | ##### -15 | ##### -29 | ##### -22 | ##### -22 | ##### -26 | ##### -25 | ##### -25 | ##### -23 | ##### -23 | ##### -22 | ##### -17 | ##### -15 |
| ##### May | ##### -16 | ##### -11 | ##### -15 | ##### -15 | ##### -21 | ##### -10 | ##### -9 | ##### -8 | ##### -13 | ##### -16 | ##### -6 | ##### -13 | ##### -16 | ##### -27 | ##### -20 | ##### -21 | ##### -25 | ##### -24 | ##### -22 | ##### -21 | ##### -23 | ##### -21 | ##### -16 | ##### -17 |
| ##### June | ##### -16 | ##### -15 | ##### -14 | ##### -14 | ##### -20 | ##### -15 | ##### -16 | ##### -20 | ##### -18 | ##### -15 | ##### -14 | ##### -20 | ##### -21 | ##### -24 | ##### -22 | ##### -22 | ##### -23 | ##### -22 | ##### -22 | ##### -19 | ##### -23 | ##### -22 | ##### -18 | ##### -18 |
| ##### July | ##### -16 | ##### -14 | ##### -14 | ##### -17 | ##### -17 | ##### -13 | ##### -15 | ##### -14 | ##### -16 | ##### -22 | ##### -13 | ##### -20 | ##### -21 | ##### -24 | ##### -22 | ##### -23 | ##### -24 | ##### -22 | ##### -21 | ##### -19 | ##### -22 | ##### -22 | ##### -18 | ##### -17 |
| ##### August | ##### -15 | ##### -18 | ##### -16 | ##### -15 | ##### -17 | ##### -18 | ##### -21 | ##### -22 | ##### -11 | ##### -16 | ##### -13 | ##### -18 | ##### -20 | ##### -22 | ##### -21 | ##### -22 | ##### -23 | ##### -22 | ##### -21 | ##### -19 | ##### -22 | ##### -21 | ##### -18 | ##### -17 |
| ##### September | ##### -10 | ##### -12 | ##### -16 | ##### -24 | ##### -31 | ##### -22 | ##### -21 | ##### -18 | ##### -26 | ##### -11 | ##### -9 | ##### -12 | ##### -14 | ##### -28 | ##### -20 | ##### -21 | ##### -25 | ##### -23 | ##### -23 | ##### -22 | ##### -22 | ##### -22 | ##### -15 | ##### -16 |
| ##### October | ##### -16 | ##### -12 | ##### -15 | ##### -16 | ##### -27 | ##### -18 | ##### -17 | ##### -15 | ##### -23 | ##### -17 | ##### -6 | ##### -10 | ##### -12 | ##### -27 | ##### -21 | ##### -23 | ##### -26 | ##### -25 | ##### -25 | ##### -22 | ##### -23 | ##### -22 | ##### -15 | ##### -12 |
| ##### November | ##### -15 | ##### -11 | ##### -20 | ##### -15 | ##### -24 | ##### -11 | ##### -13 | ##### -10 | ##### -18 | ##### -21 | ##### -4 | ##### -10 | ##### -12 | ##### -27 | ##### -22 | ##### -23 | ##### -28 | ##### -26 | ##### -25 | ##### -23 | ##### -24 | ##### -19 | ##### -11 | ##### -17 |
| ##### December | ##### -18 | ##### -28 | ##### -24 | ##### -27 | ##### -22 | ##### -17 | ##### -10 | ##### 0 | ##### -14 | ##### -17 | ##### -26 | ##### -16 | ##### -19 | ##### -32 | ##### -25 | ##### -27 | ##### -29 | ##### -27 | ##### -26 | ##### -21 | ##### -24 | ##### -10 | ##### -17 | ##### -21 |

Table 4a

FS SNR - Pangnirtung > Makkovik for Sunspot Number = 80     (FS > 21 dB) MUF (4.77 and 6.66 MHz in yellow)

| UTC | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 36 | 33 | 33 | 37 | 37 | 33 | 32 | 24 | 23 | 24 | 24 | 26 | 22 | 28 | 25 | 24 | 20 | 24 | 30 | 34 | 42 | 41 | 39 | 38 |
| February | 37 | 38 | 38 | 38 | 37 | 37 | 32 | 23 | 23 | 20 | 26 | 21 | 15 | 20 | 16 | 16 | 19 | 23 | 22 | 27 | 35 | 41 | 38 | 38 |
| March | 34 | 26 | 25 | 25 | 33 | 29 | 30 | 26 | 25 | 26 | 14 | 9 | 4 | 13 | 11 | 10 | 16 | 18 | 22 | 25 | 30 | 34 | 36 | 36 |
| April | 33 | 26 | 27 | 26 | 33 | 33 | 32 | 26 | 26 | 24 | 11 | 7 | 3 | 14 | 13 | 12 | 16 | 17 | 19 | 21 | 25 | 29 | 28 | 32 |
| May | 24 | 22 | 25 | 26 | 32 | 30 | 30 | 23 | 22 | 19 | 6 | 2 | 0 | 10 | 9 | 8 | 11 | 13 | 15 | 17 | 20 | 23 | 23 | 26 |
| June | 27 | 26 | 28 | 28 | 29 | 26 | 28 | 29 | 27 | 24 | 19 | 15 | 12 | 12 | 10 | 9 | 10 | 12 | 14 | 16 | 19 | 22 | 22 | 25 |
| July | 27 | 28 | 29 | 30 | 30 | 27 | 28 | 29 | 27 | 24 | 18 | 15 | 12 | 11 | 10 | 10 | 11 | 13 | 15 | 17 | 19 | 22 | 23 | 26 |
| August | 31 | 35 | 34 | 32 | 33 | 29 | 29 | 32 | 30 | 27 | 21 | 17 | 14 | 13 | 12 | 12 | 13 | 14 | 17 | 19 | 22 | 25 | 27 | 29 |
| September | 32 | 28 | 27 | 26 | 31 | 29 | 28 | 23 | 21 | 20 | 9 | 5 | 1 | 11 | 9 | 9 | 13 | 14 | 17 | 19 | 23 | 27 | 27 | 32 |
| October | 33 | 28 | 27 | 27 | 34 | 31 | 29 | 24 | 24 | 29 | 19 | 15 | 11 | 19 | 16 | 15 | 20 | 22 | 25 | 23 | 29 | 34 | 35 | 34 |
| November | 32 | 26 | 26 | 27 | 34 | 34 | 29 | 24 | 24 | 23 | 24 | 20 | 14 | 20 | 16 | 14 | 18 | 20 | 25 | 30 | 36 | 39 | 35 | 34 |
| December | 36 | 37 | 37 | 38 | 37 | 37 | 31 | 21 | 22 | 21 | 26 | 30 | 22 | 25 | 20 | 18 | 19 | 23 | 29 | 34 | 42 | 41 | 39 | 37 |

Table 4b

Conditional FS SNIR - Pangnirtung > Makkovik for Sunspot Number = 80     (FS > 21 dB) MUF (4.77 and 6.66 MHz in yellow)

| UTC | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 36 | 33 | 33 | 37 | 37 | 33 | 32 | 24 | 23 | 24 | 24 | 26 | 22 | 28 | 25 | 24 | 20 | 24 | 30 | 34 | 42 | 41 | 39 | 38 |
| February | 37 | 38 | 38 | 38 | 37 | 37 | 32 | 23 | 23 | 20 | 26 | 21 | 15 | 20 | 16 | 16 | 19 | 23 | 22 | 27 | 35 | 41 | 38 | 38 |
| March | 34 | 26 | 25 | 25 | 33 | 29 | 30 | 26 | 25 | 26 | 14 | 9 | 4 | 13 | 11 | 10 | 16 | 18 | 22 | 25 | 30 | 34 | 36 | 36 |
| April | 33 | 26 | 27 | 26 | 33 | 33 | 32 | 26 | 26 | 24 | 11 | 7 | 3 | 14 | 13 | 12 | 16 | 17 | 19 | 21 | 25 | 29 | 28 | 32 |
| May | 24 | 22 | 25 | 26 | 32 | 30 | 30 | 23 | 22 | 19 | 6 | 2 | 0 | 10 | 9 | 8 | 11 | 13 | 15 | 17 | 20 | 23 | 23 | 26 |
| June | 27 | 26 | 28 | 28 | 29 | 26 | 28 | 29 | 27 | 24 | 19 | 15 | 12 | 12 | 10 | 9 | 10 | 12 | 14 | 16 | 19 | 22 | 22 | 25 |
| July | 27 | 28 | 29 | 30 | 30 | 27 | 28 | 29 | 27 | 24 | 18 | 15 | 12 | 11 | 10 | 10 | 11 | 13 | 15 | 17 | 19 | 22 | 23 | 26 |
| August | 31 | 35 | 34 | 32 | 33 | 29 | 29 | 32 | 30 | 27 | 21 | 17 | 14 | 13 | 12 | 12 | 13 | 14 | 17 | 19 | 22 | 25 | 27 | 29 |
| September | 32 | 28 | 27 | 26 | 31 | 29 | 28 | 23 | 21 | 20 | 9 | 5 | 1 | 11 | 9 | 9 | 13 | 14 | 17 | 19 | 23 | 27 | 27 | 32 |
| October | 33 | 28 | 27 | 27 | 34 | 31 | 29 | 24 | 24 | 29 | 19 | 15 | 11 | 19 | 16 | 15 | 20 | 22 | 25 | 23 | 29 | 34 | 35 | 34 |
| November | 32 | 26 | 26 | 27 | 34 | 34 | 29 | 24 | 24 | 23 | 24 | 20 | 14 | 20 | 16 | 14 | 18 | 20 | 25 | 30 | 36 | 39 | 35 | 34 |
| December | 36 | 37 | 37 | 38 | 37 | 37 | 31 | 21 | 22 | 21 | 26 | 30 | 22 | 25 | 20 | 18 | 19 | 23 | 29 | 34 | 42 | 41 | 39 | 37 |

Table 4c

AS SNR – Moosonee  > Makkovik for Sunspot Number = 80     (AS >14 dB)

| ##### UTC | ##### 1 | ##### 2 | ##### 3 | ##### 4 | ##### 5 | ##### 6 | ##### 7 | ##### 8 | ##### 9 | ##### 10 | ##### 11 | ##### 12 | ##### 13 | ##### 14 | ##### 15 | ##### 16 | ##### 17 | ##### 18 | ##### 19 | ##### 20 | ##### 21 | ##### 22 | ##### 23 | ##### 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### January | ##### 18 | ##### 14 | ##### 14 | ##### 15 | ##### 15 | ##### 15 | ##### 15 | ##### 16 | ##### 8 | ##### -2 | ##### 9 | ##### 9 | ##### 1 | ##### -7 | ##### -3 | ##### -6 | ##### -7 | ##### -10 | ##### -3 | ##### 8 | ##### 14 | ##### 22 | ##### 22 | ##### 18 |
| ##### February | ##### 18 | ##### 17 | ##### 20 | ##### 20 | ##### 20 | ##### 20 | ##### 16 | ##### 16 | ##### 8 | ##### 8 | ##### 5 | ##### 7 | ##### -1 | ##### -10 | ##### -7 | ##### -9 | ##### -8 | ##### -2 | ##### 4 | ##### 5 | ##### 11 | ##### 19 | ##### 22 | ##### 19 |
| ##### March | ##### 16 | ##### 13 | ##### 8 | ##### 8 | ##### 8 | ##### 15 | ##### 15 | ##### 17 | ##### 13 | ##### 12 | ##### 11 | ##### -2 | ##### -10 | ##### -17 | ##### -14 | ##### -15 | ##### -14 | ##### -10 | ##### -5 | ##### 1 | ##### 6 | ##### 11 | ##### 19 | ##### 19 |
| ##### April | ##### 17 | ##### 13 | ##### 9 | ##### 9 | ##### 9 | ##### 16 | ##### 16 | ##### 17 | ##### 11 | ##### 11 | ##### 6 | ##### -6 | ##### -12 | ##### -17 | ##### -12 | ##### -13 | ##### -14 | ##### -11 | ##### -9 | ##### -4 | ##### 0 | ##### 6 | ##### 10 | ##### 14 |
| ##### May | ##### 10 | ##### 12 | ##### 9 | ##### 9 | ##### 9 | ##### 16 | ##### 17 | ##### 13 | ##### 11 | ##### 8 | ##### 2 | ##### -10 | ##### -14 | ##### -18 | ##### -13 | ##### -15 | ##### -16 | ##### -13 | ##### -10 | ##### -6 | ##### -4 | ##### 1 | ##### 6 | ##### 8 |
| ##### June | ##### 10 | ##### 11 | ##### 15 | ##### 15 | ##### 14 | ##### 14 | ##### 16 | ##### 14 | ##### 16 | ##### 11 | ##### 5 | ##### -4 | ##### -9 | ##### -14 | ##### -14 | ##### -16 | ##### -17 | ##### -14 | ##### -10 | ##### -6 | ##### -6 | ##### -1 | ##### 3 | ##### 8 |
| ##### July | ##### 11 | ##### 13 | ##### 15 | ##### 15 | ##### 14 | ##### 14 | ##### 16 | ##### 16 | ##### 16 | ##### 11 | ##### 5 | ##### -5 | ##### -10 | ##### -14 | ##### -14 | ##### -15 | ##### -15 | ##### -12 | ##### -9 | ##### -5 | ##### -5 | ##### 0 | ##### 4 | ##### 9 |
| ##### August | ##### 16 | ##### 16 | ##### 16 | ##### 15 | ##### 13 | ##### 10 | ##### 13 | ##### 15 | ##### 15 | ##### 15 | ##### 9 | ##### -2 | ##### -7 | ##### -11 | ##### -11 | ##### -12 | ##### -12 | ##### -10 | ##### -6 | ##### -2 | ##### -2 | ##### 3 | ##### 8 | ##### 12 |
| ##### September | ##### 15 | ##### 14 | ##### 10 | ##### 9 | ##### 8 | ##### 13 | ##### 14 | ##### 14 | ##### 9 | ##### 7 | ##### 7 | ##### -6 | ##### -13 | ##### -19 | ##### -14 | ##### -16 | ##### -15 | ##### -12 | ##### -9 | ##### -5 | ##### -1 | ##### 5 | ##### 9 | ##### 14 |
| ##### October | ##### 15 | ##### 14 | ##### 10 | ##### 10 | ##### 9 | ##### 18 | ##### 14 | ##### 15 | ##### 12 | ##### 10 | ##### 16 | ##### 5 | ##### -2 | ##### -10 | ##### -8 | ##### -11 | ##### -10 | ##### -6 | ##### -2 | ##### 3 | ##### 2 | ##### 10 | ##### 18 | ##### 17 |
| ##### November | ##### 15 | ##### 13 | ##### 9 | ##### 9 | ##### 9 | ##### 17 | ##### 15 | ##### 14 | ##### 11 | ##### 11 | ##### 13 | ##### 10 | ##### 1 | ##### -10 | ##### -3 | ##### -7 | ##### -16 | -12 | ##### -6 | ##### 2 | ##### 9 | ##### 17 | ##### 20 | ##### 16 |
| ##### December | ##### 18 | ##### 16 | ##### 16 | ##### 18 | ##### 20 | ##### 19 | ##### 20 | ##### 15 | ##### 6 | ##### 7 | ##### 7 | ##### 14 | ##### 4 | ##### -11 | ##### -10 | ##### -15 | ##### -16 | ##### -11 | ##### -4 | ##### 7 | ##### 13 | ##### 22 | ##### 22 | ##### 19 |

Table 4d

AS SNIR – Moosonee  > Makkovik for Sunspot Number = 80     (AS LBT > 14 dB)

| ##### UTC | ##### 1 | ##### 2 | ##### 3 | ##### 4 | ##### 5 | ##### 6 | ##### 7 | ##### 8 | ##### 9 | ##### 10 | ##### 11 | ##### 12 | ##### 13 | ##### 14 | ##### 15 | ##### 16 | ##### 17 | ##### 18 | ##### 19 | ##### 20 | ##### 21 | ##### 22 | ##### 23 | ##### 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### January | ##### -13 | ##### -14 | ##### -13 | ##### -17 | ##### -16 | ##### -12 | ##### -11 | ##### -2 | ##### -9 | ##### -20 | ##### -9 | ##### -12 | ##### -15 | ##### -30 | ##### -23 | ##### -24 | ##### -22 | ##### -29 | ##### -27 | ##### -21 | ##### -23 | ##### -14 | ##### -11 | ##### -14 |
| ##### February | ##### -14 | ##### -15 | ##### -13 | ##### -13 | ##### -12 | ##### -12 | ##### -10 | ##### -1 | ##### -9 | ##### -6 | ##### -15 | ##### -10 | ##### -11 | ##### -25 | ##### -18 | ##### -20 | ##### -22 | ##### -20 | ##### -12 | ##### -16 | ##### -18 | ##### -17 | ##### -11 | ##### -14 |
| ##### March | -12 | ##### -9 | ##### -12 | ##### -12 | ##### -20 | ##### -8 | ##### -9 | ##### -4 | ##### -5 | ##### -9 | ##### 1 | ##### -7 | ##### -13 | ##### -26 | ##### -20 | ##### -21 | ##### -24 | ##### -23 | ##### -22 | ##### -19 | ##### -18 | ##### -18 | ##### -13 | ##### -12 |
| ##### April | ##### -12 | ##### -9 | ##### -13 | ##### -13 | ##### -19 | ##### -11 | ##### -10 | ##### -5 | ##### -10 | ##### -7 | ##### 0 | ##### -10 | ##### -14 | ##### -27 | ##### -21 | ##### -21 | ##### -25 | ##### -23 | ##### -23 | ##### -20 | ##### -20 | ##### -19 | ##### -13 | ##### -12 |
| ##### May | ##### -9 | ##### -5 | ##### -11 | ##### -12 | ##### -19 | ##### -9 | ##### -7 | ##### -5 | ##### -6 | ##### -6 | ##### -2 | ##### -12 | ##### -15 | ##### -24 | ##### -18 | ##### -19 | ##### -23 | ##### -22 | ##### -20 | ##### -18 | ##### -19 | ##### -17 | ##### -11 | ##### -13 |
| ##### June | ##### -11 | ##### -10 | ##### -7 | ##### -9 | ##### -11 | ##### -7 | ##### -7 | ##### -9 | ##### -7 | ##### -9 | ##### -8 | ##### -14 | ##### -17 | ##### -21 | ##### -20 | ##### -21 | ##### -23 | ##### -22 | ##### -20 | ##### -17 | ##### -19 | ##### -18 | ##### -14 | ##### -12 |
| ##### July | ##### -11 | ##### -9 | ##### -9 | ##### -10 | ##### -12 | ##### -8 | ##### -6 | ##### -7 | ##### -7 | ##### -8 | ##### -8 | ##### -15 | ##### -18 | ##### -21 | ##### -20 | ##### -21 | ##### -22 | ##### -21 | ##### -19 | -17 | ##### -19 | ##### -18 | ##### -13 | ##### -11 |
| ##### August | ##### -10 | ##### -14 | ##### -13 | ##### -12 | ##### -15 | ##### -15 | ##### -10 | ##### -12 | ##### -10 | ##### -7 | ##### -7 | ##### -14 | ##### -17 | ##### -20 | ##### -19 | ##### -19 | ##### -21 | ##### -20 | ##### -18 | ##### -16 | ##### -19 | ##### -17 | ##### -13 | ##### -13 |
| ##### September | ##### -12 | ##### -9 | ##### -12 | ##### -12 | ##### -18 | ##### -10 | ##### -9 | ##### -3 | ##### -7 | ##### -8 | ##### 1 | ##### -9 | ##### -14 | ##### -25 | ##### -19 | ##### -21 | ##### -24 | ##### -22 | ##### -21 | ##### -19 | ##### -19 | ##### -18 | ##### -13 | ##### -12 |
| ##### October | ##### -12 | ##### -9 | ##### -13 | ##### -12 | ##### -20 | ##### -8 | ##### -10 | ##### -3 | ##### -7 | ##### -14 | ##### 2 | ##### -5 | ##### -9 | ##### -24 | ##### -19 | ##### -21 | ##### -25 | ##### -23 | ##### -22 | ##### -15 | ##### -22 | ##### -19 | ##### -12 | ##### -12 |
| ##### November | ##### -12 | ##### -9 | ##### -13 | ##### -13 | ##### -20 | ##### -12 | ##### -9 | ##### -4 | ##### -7 | ##### -6 | ##### -6 | ##### -5 | ##### -9 | ##### -25 | ##### -14 | ##### -16 | ##### -28 | ##### -27 | ##### -26 | ##### -23 | ##### -22 | ##### -18 | ##### -10 | ##### -12 |
| ##### December | ##### -13 | ##### -16 | ##### -16 | ##### -14 | ##### -12 | ##### -12 | ##### -6 | ##### 0 | ##### -10 | ##### -8 | ##### -14 | ##### -12 | ##### -13 | ##### -31 | ##### -25 | ##### -28 | ##### -30 | ##### -29 | ##### -28 | ##### -22 | ##### -24 | ##### -13 | ##### -12 | ##### -13 |

# 7	Analysis of the results

Interpretation of the results follows the below sequence:

•	The S/N values for the fixed-link, as detailed in Tables 3a, and 4a, are analysed and those which meet or exceed the minimum usability criterion – 21 dB – are identified in green

•	The S/N values for the amateur-link, as detailed in Tables 3c, and 4c, are analysed and those which meet or exceed the minimum usability criterion – 14 dB – are identified in green

•	The amateur link is assumed only to operate in those time slots where the AS SNIR meets or exceeds 14 dB (The relevant time slots are those coloured green in Tables 3d and 4d) and not to operate during those time slots when the fixed link is active.

•	Fixed service link operating frequency of 5 300 kHz is compared to the MUF and identified if it is within -25% and +10% of the predicted MUF.

•	Notwithstanding the foregoing, in those instances where the same time slot is shown green for both the fixed-link and the amateur-link, a value of SNIR is calculated indicating the degree of interference - if any - which might impact the fixed-link were the amateur-link to operate. The interference (“I”) term in the foregoing is the amateur-service signal as it might be observed at the receive link of the fixed service link.

# 7.1	Circuit Availability

A cursory analysis based upon the foregoing yields the following observations are summarized in Table 5:

Table 5

Summary of radio communication link availabilities for FS and AS

|  | Sunspot Number=10 | Sunspot Number =80 |
| --- | --- | --- |
| Percent of time slots usable by the fixed-link (FS) | 76.0% | 67.0% |
| Percent of time slots usable by the amateur-link (AS) | 13.8% | 37.8.0% |
| Percent of time slots concurrently usable | 0.0% | 0.0% |

For perspective, the following points are re-stated:

•	all data shown in Tables 1a through 4d are mean values theoretically prevailing 50% of the time;

•	the Amateur Radio Service follows a “Listen before Transmit” protocol and would not knowingly initiate communication on an occupied channel. As a practical matter the fixed service signal (I) in the SNIR calculations lowers the amateur service SNIR to levels not practical for any radio communication to be established.

•	The proposed amateur allocation in 5 250 to 5 450 kHz is for a Secondary Allocation.

# 9	Parameters used for compatibility study

Editor’s note 1: metadata is fundamental to a full and proper analysis of the results so this section should contain the parameters/settings used for the REC. ITU-R 533 software. If there is no agreement on the particular details, there will be multiple subsections under this topic.

Editor’s note 2: Need to consider the link reliability figure. While 50% may be suitable for an amateur link, is it suitable for a fixed link?

REC533 parameters

(Version 12.07221)

The REC533 input parameters used in the sample calculations (Specific Example: Pangnirtung > Makkovik and “Moosonee” > Makkovik) are listed in this Annex.

1)	Method: 6

2)	Year: XXXX

3)	Coefficients: CCIR

4)	Time: 0100, 0200, 0300 --------2400 UT

5)	Months: 01, 02, 03---------12

6)	Sunspot numbers: 10, 80

7)	Fixed site locations, same geographic coordinates assumed for hypothetical transmitter and receiver sites as shown in Table 1.

8)	Short or long path: Short

9)	Frequency 5.3, MHz

10)	Noise power at 3 MHz: The man-made noise level at the receiver in –dBW (decibels below 1 Watt) in a 1 Hertz bandwidth at 3 MHz, all sites: –156 dBW

11)	Minimum take-off angle: 3 degrees

12)	Require circuit reliability 50%

13)	Receiver bandwidth 3 000 Hz

14)	Required signal/noise per Hz: high-speed data transmission 56 (dB/Hz)

15)	Transmitter antenna and transmitter power:

Fixed service: log periodic antenna sample type 05 Rec. ITU-R 705 horizontal log periodic array

Amateur service: Type 23 ITSA-1 IONCAP horizontal dipole, 10m above ground

Antenna input power = 5 kW for fixed service and 0.1 kW for amateur service

Transmit antennas with main beam of antenna always directed toward receiver sites.

16)	Receiving antenna:

Same as transmit antennas with main beam of receiving antennas always directed toward transmitter site.

15)	Electrical properties of the ground:

As shown in Figures 1 to 4.

# 10	Result of studies

Editor’s note: Tables etc. showing the results of the various conditions i.e. SSN, antenna type, transmissions modes etc. will go here.

## 10.1	Results obtained for Study 1

The compatibility analysis of the stations in the amateur service with the radio links in the fixed service was carried out assuming single and multihop links of the fixed service. Murmansk (68.98 North 33.08 East) to Moscow (55.9 North, 37.6 East) link was taken as a single hop link of the fixed service of 1 490 km and Petropavlovsk- Kamchatski (53 North, 158 East) to Moscow (55.9 North, 37.6 East) link of 6 800 km was used as a multihop link in the analysis. Depending on the time of the day and season this transmission link can operate in two-hop or in three-hop modes.

Envisioned amateur service transmitters located in Kotka city (60.47 North, 26.95 East) and in Kirkenes (69.72 North, 30.05 East) were assumed as potential interference sources for a single-hop radiolink; transmitters located in Astana city (the interfering path length was 2 277 km) and in the point with coordinates 0 degree North, 37.6 East located in the Equator (the interfering path length was 6 200 km) were assumed as interference sources for the multihop fixed service link. Depending on season and daytime interference signal propagation in Astana-Moscow radiopath the link could operate in single-hop and two-hop modes but Equator-Moscow link could operated only in a multihop mode.

The study analyzed communication links using 300HF1B emission class signals. Protection criteria for the fixed service link operating with 300HF1B signals under steady-state conditions were taken from Recommendation ITU-R F.339. They are presented in Table 10.1 below.

TABLE 10.1

Protection criteria for steady conditions

| 300HF1В Signal |  |  |  |
| --- | --- | --- | --- |
| Bit error rate | Band, Hz | Protection criterion S/N, (dB/Hz) |
| 0,01 | 300 | 26 |
| 0,001 | 300 | 27 |
| 0,0001 | 300 | 29 |

Similar signal is used in Petropavlovsk- Kamchatski-Moscow FS link, registered in the MIFR (ID 081080602, emission class 280HF1В).

Protection criteria for the fixed service link operating with 300HF1B signals under fading conditions were taken from Recommendation ITU-R F.339. They are presented in Table 10.2 below.

TABLE 10.2

Protection criteria for fading conditions

| 300HF1В Signal |  |  |  |
| --- | --- | --- | --- |
| Bit error rate | Band, Hz | Protection criterion S/N, (dB/Hz) |
| 0,01 | 300 | 42 |
| 0,001 | 300 | 49 |
| 0,0001 | 300 | 56 |

It should be noted that interference-to-noise ratios specified in Tables 10.1 and 10.2 were referenced to the bandwidth of 1 Hz (see Rec. ITU-R F.339). The protection criteria for an arbitrary frequency band  can be obtained by the following equation:

The conducted studies assumed that the FS transmitter antenna fed power was 37 dBW. Two antenna types i.e. an isotropic antenna of 0 dB gain and Yagi antenna of 13.4 dB gain were assumed as possible FS antenna.

The study also assumed that amateur stations used omnidirectional antennas with antenna power of 20 dBW.

The compatibility studies were carried out assuming 10 sunspot numbers. The signal-to-interference ratio for the considered links was estimated for each month at 1 hour interval.

### 10.1.1	Study results for multihop links using omnidirectional antennas under steady conditions

The results of estimations for Petropavlovsk-Kamchatsky to Moscow link using omnidirectional antennas are shown in Tables 10.3 – 10.7.

TABLE 10.3

Propagation conditions in Petropavlovsk-Kamchatski to Moscow link, using omnidirectional antennas

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 29 | 24 | 18 | 17 | 14 | 11 | 10 | 15 | 18 | 23 | 28 | 33 | 38 | 44 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 41 | 34 | 29 |
| February | 14 | 7 | -3 | -7 | -9 | -9 | -8 | -8 | -11 | 2 | 13 | 23 | 31 | 37 | 44 | 45 | 45 | 45 | 45 | 44 | 44 | 40 | 33 | 24 | 14 |
| March | 1 | -7 | -10 | -16 | -20 | -21 | -21 | -19 | -18 | -14 | -2 | 9 | 17 | 26 | 33 | 38 | 44 | 44 | 45 | 44 | 39 | 32 | 23 | 15 | 1 |
| April | -8 | -15 | -19 | -24 | -28 | -30 | -31 | -33 | -31 | -26 | -19 | -11 | 1 | 10 | 18 | 26 | 32 | 38 | 39 | 34 | 27 | 21 | 13 | 0 | -8 |
| May | -19 | -26 | -220 | -220 | -41 | -45 | -45 | -42 | -40 | -36 | -31 | -25 | -20 | -13 | -1 | 8 | 14 | 19 | 20 | 15 | 11 | 4 | -5 | -12 | -19 |
| June | -32 | -37 | -41 | -220 | -220 | -52 | -47 | -47 | -44 | -40 | -37 | -31 | -25 | -19 | -15 | -6 | 3 | 7 | 8 | 5 | 0 | -9 | -16 | -27 | -32 |
| July | -25 | -29 | -32 | -34 | -36 | -35 | -39 | -35 | -33 | -34 | -30 | -25 | -15 | -13 | -3 | 4 | 9 | 13 | 13 | 10 | 3 | -3 | -12 | -18 | -25 |
| August | -19 | -24 | -28 | -32 | -35 | -37 | -38 | -36 | -37 | -33 | -28 | -21 | -12 | 0 | 8 | 16 | 22 | 25 | 24 | 20 | 14 | 7 | -4 | -13 | -19 |
| September | -5 | -13 | -18 | -24 | -28 | -31 | -32 | -35 | -33 | -26 | -17 | -2 | 5 | 14 | 23 | 32 | 39 | 44 | 45 | 40 | 33 | 25 | 18 | 9 | -5 |
| October | 16 | 10 | 0 | -6 | -9 | -10 | -9 | -9 | -12 | -3 | 7 | 14 | 23 | 30 | 36 | 43 | 44 | 44 | 45 | 44 | 44 | 40 | 34 | 27 | 16 |
| November | 30 | 25 | 18 | 16 | 11 | 10 | 11 | 11 | 14 | 18 | 25 | 30 | 34 | 40 | 43 | 44 | 44 | 44 | 44 | 44 | 44 | 47 | 42 | 36 | 30 |
| December | 32 | 18 | 13 | 18 | 18 | 17 | 19 | 21 | 23 | 26 | 31 | 32 | 40 | 44 | 43 | 30 | 34 | 45 | 45 | 45 | 44 | 45 | 45 | 38 | 32 |

TABLE 10.4

Propagation conditions in Astana – Moscow interference radiolink

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Astana – Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0.1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 40 | 40 | 40 | 39 | 30 | 23 | 16 | 12 | 11 | 11 | 14 | 20 | 28 | 37 | 43 | 42 | 40 | 40 | 41 | 41 | 41 | 42 | 42 | 42 | 40 |
| February | 40 | 40 | 40 | 32 | 28 | 19 | 15 | 12 | 11 | 12 | 14 | 18 | 25 | 34 | 41 | 43 | 42 | 41 | 41 | 41 | 41 | 41 | 41 | 40 | 40 |
| March | 40 | 41 | 38 | 29 | 22 | 15 | 11 | 9 | 8 | 9 | 11 | 14 | 18 | 25 | 33 | 40 | 42 | 42 | 41 | 39 | 39 | 40 | 40 | 40 | 40 |
| April | 40 | 38 | 31 | 24 | 16 | 12 | 10 | 8 | 7 | 7 | 9 | 11 | 14 | 18 | 25 | 32 | 37 | 42 | 42 | 41 | 41 | 40 | 40 | 40 | 40 |
| May | 38 | 31 | 27 | 19 | 14 | 10 | 8 | 6 | 6 | 6 | 7 | 9 | 11 | 15 | 19 | 26 | 31 | 35 | 41 | 42 | 42 | 43 | 42 | 41 | 38 |
| June | 32 | 29 | 24 | 17 | 12 | 9 | 6 | 5 | 4 | 4 | 5 | 7 | 9 | 13 | 17 | 22 | 28 | 30 | 35 | 40 | 40 | 41 | 39 | 34 | 32 |
| July | 32 | 29 | 25 | 19 | 13 | 9 | 7 | 5 | 4 | 4 | 5 | 8 | 10 | 14 | 19 | 25 | 30 | 33 | 39 | 40 | 40 | 41 | 40 | 37 | 32 |
| August | 38 | 32 | 27 | 21 | 15 | 11 | 8 | 6 | 6 | 6 | 7 | 9 | 11 | 16 | 22 | 28 | 32 | 37 | 41 | 40 | 40 | 40 | 40 | 39 | 38 |
| September | 40 | 40 | 32 | 26 | 18 | 12 | 9 | 7 | 6 | 7 | 8 | 10 | 13 | 18 | 26 | 33 | 42 | 42 | 42 | 41 | 40 | 40 | 40 | 40 | 40 |
| October | 40 | 40 | 42 | 35 | 29 | 21 | 17 | 14 | 13 | 13 | 14 | 17 | 21 | 28 | 34 | 42 | 42 | 42 | 41 | 41 | 40 | 40 | 40 | 40 | 40 |
| November | 40 | 40 | 40 | 41 | 35 | 28 | 21 | 17 | 15 | 16 | 17 | 21 | 27 | 34 | 42 | 42 | 40 | 40 | 39 | 39 | 39 | 40 | 40 | 40 | 40 |
| December | 40 | 40 | 40 | 39 | 37 | 27 | 19 | 13 | 11 | 11 | 13 | 19 | 28 | 39 | 42 | 40 | 40 | 40 | 41 | 41 | 41 | 41 | 42 | 41 | 40 |

TABLE 10.5

Propagation conditions in Petropavlovsk-Kamchatski to Moscow link with interference caused by 
an amateur service transmitter located in Astana

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  |  | 5 kW |  | SSN |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interfering link: Astana – Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0.1 kW |  |  |  | SSN |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 |  | 2 |  | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | -11 |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 | 13 | 10 | 7 | 2 | 3 | 5 | 5 | 4 | 4 | 4 | 3 | -1 | -8 | -11 |
| February |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 | 3 | 3 | 2 | 3 | 4 | 4 | 3 | 3 | -1 | -8 |  |  |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 0 | -2 | 2 | 2 | 4 | 5 | 0 | -8 |  |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -6 | -5 | -4 | -3 | -7 | -14 |  |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -1 | -3 | 2 | 3 | -1 | -7 |  |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 | 1 | 2 | 2 | 4 | 3 | 4 | 0 | -6 | -13 |  |
| November | -10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 | 7 | 6 | 1 | 2 | 4 | 4 | 5 | 5 | 5 | 7 | 2 | -4 | -10 |
| December | -8 |  |  |  |  |  |  |  |  |  |  |  |  | 15 | 18 | 13 | 12 | 5 | 1 | -10 | -6 | 5 | 4 | 4 | 3 | 4 | 3 | -3 | -8 |

TABLE 10.6

Propagation conditions in Equator – Moscow interfering link

| Sgnal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Astana – Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 28 | 28 | 29 | 28 | 14 | -24 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -59 | -28 | 15 | 25 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| February | 29 | 29 | 29 | 29 | 6 | -41 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 11 | 26 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 |
| March | 29 | 29 | 29 | 27 | -5 | -58 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -8 | 23 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| April | 29 | 29 | 29 | 16 | -16 | -62 | -104 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -29 | 8 | 26 | 28 | 28 | 29 | 29 | 29 | 29 | 29 |
| May | 29 | 29 | 24 | 10 | -25 | -66 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -5 | 16 | 28 | 29 | 29 | 29 | 29 | 29 | 29 |
| June | 28 | 28 | 21 | 4 | -29 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -95 | -60 | -221 | -9 | 12 | 24 | 27 | 27 | 27 | 27 | 27 | 28 |
| July | 28 | 28 | 22 | 5 | -33 | -74 | -220 | -220 | -134 | -220 | -220 | -220 | -220 | -93 | -54 | -33 | -3 | 15 | 25 | 27 | 27 | 27 | 27 | 27 | 28 |
| August | 28 | 29 | 28 | 12 | -23 | -72 | -108 | -220 | -220 | -220 | -220 | -220 | -220 | -91 | -220 | -221 | 5 | 22 | 27 | 27 | 27 | 27 | 27 | 27 | 28 |
| September | 29 | 29 | 29 | 24 | -3 | -51 | -97 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -22 | 17 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| October | 29 | 29 | 29 | 29 | 12 | -34 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -16 | 25 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| November | 29 | 29 | 29 | 29 | 19 | -13 | -52 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 2 | 26 | 28 | 28 | 28 | 28 | 28 | 28 | 29 | 29 |
| December | 28 | 28 | 29 | 31 | 21 | -11 | -60 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 11 | 25 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |

TABLE 10.7

Propagation conditions in Petropavlovsk-Kamchatski to Moscow link with interference caused by 
an amateur service transmitter located in Equator

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interfering link: Equator – Moscow, isotropic antenna gain=0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  |  | SSN |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| January | 0 | 1 |  | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| February | 1 |  |  |  |  |  |  |  |  |  |  |  | 248 | 253 | 258 | 103 | 73 | 30 | 20 | 17 | 17 | 17 | 17 | 17 | 13 | 6 | 1 |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 251 | 257 | 264 | 34 | 19 | 16 | 16 | 15 | 15 | 11 | 4 |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 246 | 253 | 46 | 21 | 16 | 17 | 16 | 11 | 4 |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 24 | 12 | 11 | 6 | -2 |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54 | 22 | 16 | 17 | 12 | 5 |  |  |  |  |
| November |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 | 250 | 46 | 5 | 2 | 2 | 2 | 2 | 2 | 1 | -2 |  |
| December | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 250 | 254 | 260 | 263 | 42 | 18 | 16 | 16 | 16 | 16 | 19 | 14 | 7 | 1 |
| January | 4 |  |  |  |  |  |  |  |  |  |  | 246 | 251 | 252 | 260 | 264 | 263 | 19 | 9 | 17 | 17 | 17 | 16 | 17 | 17 | 10 | 4 |

The tables above and below contain red-highlighted cells in which signal-to-noise ratio is less than 26 dB implying infeasibility of FS links operation. Dark green cells denote ones where signal-to-noise ratio is equal or exceeds 29 dB, i.e. bit error rate is 0.0001. Light green cells refer to those ones where signal-to-noise ratio is from 26 dB to 27 dB implying bit error rate of 0.01. The yellow highlighted cells denote ones with signal-to-noise ratio less than 26 dB meaning that interference would make operation of FS station unfeasible.

Table 10.3 presents simulation results of propagation conditions in Petropavlovsk-Kamchatski to Moscow link, using omnidirectional antennae. The analysis of the obtained results shows that when there is no interference caused by the amateur service link the operation of the considered fixed service link is feasible during 8 months per year (excluding May and all summer months). This defect from the interfering link can be eliminated by using the directional antennae in the receiving and transmitting link ends or by increasing transmitter power in the fixed service link.

Table 10.4 contains results of propagation conditions simulation for Astana– Moscow link of 2 277 km.

Table 10.5 shows the impact of Astana located amateur service transmitter on operation of Petropavlovsk-Kamchatski to Moscow link. The analysis of the obtained results shows that the considered amateur service transmitter, located at distance of 2 277 km from the fixed service receiver will cause unacceptable interference to its operation in any daytime (see Table 4).

Table 10.6 presents results of propagation conditions simulation in Equator– Moscow link, using omnidirectional antennae.

Table 10.7 shows the impact of Equator located amateur service transmitter on operation of Petropavlovsk-Kamchatski to Moscow fixed link. The analysis of the obtained results shows that increasing distance between the interfering amateur service transmitter and the fixed service receiver up to 6 200 km would not provide protection for the fixed service receiver since time period available for operation of fixed link is decreased by more than 50% due to interference caused by the amateur service link.

### 10.1.2	Study results for multihop links using directional antennas under steady conditions

The effect from directional antenna on operation of Petropavlovsk-Kamchatski to Moscow link was estimated to define feasibility of improving operation of FS links and mitigating interference impact. Therein it was assumed that FS links used 5-element Yagi antennas of 13.4 dB gain. The results of estimations are shown in Tables 10.8-10.12.

TABLE 10.8

Propagation conditions on Petropavlovsk-Kamchatski to Moscow link, using directional antennae

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna of 13.4 dB gain, receive & transmit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 53 | 47 | 40 | 40 | 38 | 35 | 34 | 38 | 41 | 45 | 52 | 57 | 61 | 66 | 67 | 67 | 67 | 67 | 66 | 66 | 66 | 66 | 62 | 58 | 53 |
| February | 35 | 29 | 19 | 15 | 14 | 14 | 15 | 15 | 12 | 24 | 35 | 46 | 54 | 60 | 66 | 67 | 67 | 67 | 67 | 67 | 66 | 63 | 56 | 47 | 35 |
| March | 22 | 14 | 10 | 4 | 1 | 0 | 0 | 2 | 2 | 7 | 18 | 29 | 38 | 49 | 56 | 61 | 66 | 66 | 67 | 66 | 62 | 55 | 46 | 35 | 22 |
| April | 11 | 4 | -1 | -6 | -10 | -12 | -12 | -15 | -12 | -7 | 1 | 8 | 21 | 29 | 40 | 49 | 56 | 59 | 60 | 57 | 50 | 44 | 33 | 19 | 11 |
| May | -1 | -8 | -220 | -220 | -23 | -26 | -27 | -26 | -24 | -19 | -13 | -7 | 0 | 7 | 20 | 30 | 38 | 43 | 44 | 38 | 31 | 25 | 14 | 6 | -1 |
| June | -17 | -22 | -27 | -220 | -220 | -33 | -30 | -32 | -28 | -24 | -19 | -13 | -7 | 0 | 6 | 16 | 25 | 30 | 31 | 28 | 21 | 10 | 3 | -11 | -17 |
| July | -8 | -13 | -17 | -19 | -21 | -18 | -22 | -20 | -17 | -19 | -15 | -8 | 4 | 6 | 17 | 26 | 33 | 36 | 37 | 32 | 24 | 17 | 6 | -2 | -8 |
| August | -1 | -6 | -11 | -15 | -18 | -20 | -21 | -18 | -20 | -15 | -10 | -2 | 7 | 20 | 29 | 39 | 45 | 49 | 47 | 44 | 35 | 28 | 15 | 6 | -1 |
| September | 15 | 7 | 1 | -5 | -9 | -11 | -12 | -16 | -13 | -6 | 3 | 18 | 26 | 36 | 46 | 55 | 60 | 65 | 66 | 61 | 56 | 47 | 40 | 30 | 15 |
| October | 38 | 32 | 22 | 16 | 13 | 12 | 14 | 14 | 11 | 19 | 29 | 35 | 44 | 53 | 59 | 64 | 66 | 66 | 67 | 66 | 66 | 63 | 58 | 49 | 38 |
| November | 52 | 48 | 41 | 38 | 34 | 33 | 34 | 34 | 37 | 41 | 48 | 53 | 58 | 62 | 65 | 66 | 66 | 66 | 67 | 66 | 66 | 66 | 64 | 59 | 52 |
| December | 56 | 41 | 36 | 41 | 42 | 41 | 42 | 45 | 47 | 50 | 54 | 56 | 61 | 66 | 65 | 52 | 57 | 67 | 67 | 67 | 67 | 67 | 65 | 61 | 56 |

TABLE 10.9

Propagation conditions on Astana– Moscow interfering link

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Astana (isotropic antenna of 0 dB gain) – Moscow (Yagi antenna) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 37 | 38 | 37 | 34 | 29 | 21 | 12 | 8 | 7 | 8 | 11 | 18 | 27 | 35 | 39 | 38 | 36 | 37 | 37 | 37 | 37 | 38 | 38 | 38 | 37 |
| February | 38 | 39 | 38 | 30 | 26 | 15 | 11 | 8 | 7 | 7 | 10 | 15 | 23 | 32 | 39 | 39 | 38 | 36 | 37 | 38 | 38 | 37 | 37 | 37 | 38 |
| March | 39 | 38 | 36 | 27 | 19 | 10 | 7 | 5 | 4 | 5 | 7 | 10 | 14 | 23 | 31 | 38 | 38 | 38 | 37 | 36 | 37 | 38 | 38 | 39 | 39 |
| April | 37 | 36 | 29 | 21 | 12 | 8 | 5 | 3 | 2 | 3 | 4 | 7 | 10 | 14 | 22 | 31 | 36 | 39 | 38 | 37 | 37 | 37 | 38 | 38 | 37 |
| May | 36 | 29 | 24 | 16 | 9 | 6 | 4 | 2 | 1 | 2 | 3 | 5 | 7 | 10 | 15 | 24 | 30 | 34 | 38 | 38 | 38 | 38 | 38 | 37 | 36 |
| June | 30 | 26 | 21 | 12 | 8 | 5 | 2 | 0 | 0 | 0 | 1 | 3 | 5 | 8 | 12 | 20 | 26 | 28 | 34 | 36 | 36 | 37 | 36 | 32 | 30 |
| July | 30 | 27 | 22 | 15 | 8 | 5 | 2 | 0 | 0 | 0 | 1 | 3 | 5 | 9 | 16 | 22 | 28 | 31 | 35 | 37 | 36 | 37 | 36 | 35 | 30 |
| August | 36 | 30 | 25 | 18 | 10 | 7 | 4 | 2 | 1 | 1 | 3 | 5 | 7 | 11 | 19 | 26 | 30 | 36 | 37 | 36 | 36 | 36 | 36 | 35 | 36 |
| September | 39 | 38 | 31 | 24 | 15 | 8 | 5 | 3 | 2 | 2 | 4 | 6 | 9 | 14 | 25 | 31 | 38 | 38 | 38 | 37 | 37 | 37 | 38 | 39 | 39 |
| October | 39 | 38 | 38 | 33 | 27 | 17 | 13 | 10 | 8 | 9 | 10 | 13 | 17 | 26 | 32 | 39 | 38 | 38 | 37 | 37 | 36 | 37 | 37 | 38 | 39 |
| November | 38 | 38 | 37 | 37 | 33 | 26 | 17 | 13 | 11 | 12 | 13 | 17 | 25 | 33 | 38 | 38 | 36 | 36 | 36 | 36 | 36 | 37 | 37 | 37 | 38 |
| December | 37 | 38 | 39 | 36 | 35 | 25 | 17 | 9 | 7 | 7 | 10 | 17 | 27 | 37 | 38 | 37 | 39 | 38 | 38 | 38 | 37 | 38 | 38 | 37 | 37 |

TABLE 10.10

Propagation conditions on Petropavlovsk-Kamchatski to Moscow link with interference 
caused by the amateur service transmitter located in Astana

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna of 13.4 dB gain, receive & transmit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interfering link: Astana (isotropic antenna of 0 dB gain) – Moscow (directional antenna) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0.1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 16 | 9 | 3 | 6 | 9 | 14 | 22 | 30 | 34 | 37 | 41 | 39 | 34 | 31 | 28 | 29 | 31 | 30 | 29 | 29 | 29 | 28 | 24 | 20 | 16 |
| February | -3 | -10 |  |  |  |  |  |  |  |  | 25 | 31 | 31 | 28 | 27 | 28 | 29 | 31 | 30 | 29 | 28 | 26 | 19 | 10 | -3 |
| March |  |  |  |  |  |  |  |  |  |  |  | 19 | 24 | 26 | 25 | 23 | 28 | 28 | 30 | 30 | 25 | 17 | 8 | -4 |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 | 18 | 18 | 20 | 20 | 22 | 20 | 13 | 7 | -5 |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 | 8 | 9 | 6 | 0 | -7 |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | -3 | -8 |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | 5 | 5 | 2 | -5 |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 | 13 | 15 | 13 | 10 | 8 | -1 | -8 |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  | 17 | 22 | 21 | 24 | 22 | 27 | 28 | 24 | 19 | 10 | 2 | -9 |  |
| October | -1 | -6 |  |  |  |  |  |  |  |  | 19 | 22 | 27 | 27 | 27 | 25 | 28 | 28 | 30 | 29 | 30 | 26 | 21 | 11 | -1 |
| November | 14 | 10 | 4 | 1 | 1 | 7 | 17 | 21 | 26 | 29 | 35 | 36 | 33 | 29 | 27 | 28 | 30 | 30 | 31 | 30 | 30 | 29 | 27 | 22 | 14 |
| December | 19 | 3 | -3 | 5 | 7 | 16 | 25 | 36 | 40 | 43 | 44 | 39 | 34 | 29 | 27 | 15 | 18 | 29 | 29 | 29 | 30 | 29 | 27 | 24 | 19 |

TABLE 10.11

Propagation conditions on Equator– Moscow interfering link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Equator (isotropic antenna of 0 dB gain) to Moscow (Yagi directional antenna) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 26 | 27 | 28 | 27 | 15 | -23 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -57 | -27 | 15 | 24 | 25 | 26 | 27 | 27 | 27 | 26 | 26 | 26 |
| February | 26 | 27 | 28 | 27 | 7 | -40 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 12 | 24 | 25 | 26 | 27 | 27 | 27 | 27 | 26 | 26 |
| March | 26 | 27 | 28 | 26 | -4 | -57 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -7 | 23 | 24 | 25 | 26 | 26 | 26 | 26 | 26 | 26 |
| April | 27 | 27 | 27 | 17 | -15 | -61 | -103 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -28 | 9 | 24 | 25 | 26 | 26 | 26 | 26 | 26 | 27 |
| May | 27 | 27 | 25 | 11 | -24 | -65 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -4 | 17 | 25 | 25 | 26 | 26 | 26 | 27 | 27 |
| June | 26 | 26 | 22 | 5 | -28 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -95 | -59 | -221 | -8 | 13 | 22 | 24 | 24 | 24 | 25 | 25 | 26 |
| July | 26 | 27 | 23 | 6 | -31 | -73 | -220 | -220 | -135 | -220 | -220 | -220 | -220 | -92 | -53 | -31 | -2 | 16 | 23 | 24 | 25 | 25 | 25 | 25 | 26 |
| August | 26 | 28 | 27 | 13 | -21 | -71 | -108 | -220 | -220 | -220 | -220 | -220 | -220 | -90 | -220 | -221 | 6 | 21 | 23 | 24 | 25 | 25 | 25 | 25 | 26 |
| September | 27 | 28 | 28 | 24 | -2 | -50 | -96 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -21 | 17 | 24 | 25 | 26 | 26 | 27 | 27 | 27 | 27 |
| October | 27 | 27 | 28 | 26 | 12 | -33 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -15 | 23 | 25 | 25 | 26 | 26 | 27 | 27 | 26 | 27 |
| November | 26 | 27 | 28 | 27 | 20 | -12 | -51 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 3 | 24 | 26 | 26 | 27 | 27 | 27 | 27 | 26 | 26 |
| December | 26 | 27 | 28 | 28 | 21 | -10 | -58 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 12 | 24 | 26 | 26 | 27 | 27 | 27 | 26 | 26 | 26 |

TABLE 10.12

Propagation conditions on Petropavlovsk-Kamchatski – Moscow link with interference 
caused by the amateur service transmitter located in Equator

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna of 13.4 dB gain, receive & transmit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interfering link: Terminator (equator), (with isotropic directional antenna of 0 dB gain) – Moscow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0.1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 27 | 20 | 12 | 13 | 23 | 58 | 254 | 258 | 261 | 265 | 272 | 277 | 281 | 123 | 94 | 52 | 43 | 42 | 40 | 39 | 39 | 39 | 36 | 32 | 27 |
| February | 9 | 2 |  |  |  |  |  |  |  |  | 255 | 266 | 274 | 280 | 286 | 55 | 43 | 42 | 41 | 40 | 39 | 36 | 29 | 21 | 9 |
| March |  |  |  |  |  |  |  |  |  |  |  | 249 | 258 | 269 | 276 | 68 | 43 | 42 | 42 | 40 | 36 | 29 | 20 | 9 |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  | 249 | 260 | 77 | 47 | 35 | 35 | 31 | 24 | 18 | 7 |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 | 42 | 26 | 19 | 13 | 5 |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17 | 9 | 4 |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 57 | 35 | 20 | 14 | 8 |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 249 | 260 | 39 | 28 | 24 | 20 | 10 | 3 |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  | 246 | 256 | 266 | 76 | 43 | 41 | 41 | 35 | 30 | 20 | 13 | 3 |  |
| October | 11 | 5 |  |  |  |  |  |  |  |  | 249 | 255 | 264 | 273 | 279 | 79 | 43 | 41 | 42 | 40 | 40 | 36 | 31 | 23 | 11 |
| November | 26 | 21 | 13 | 11 | 14 | 45 | 85 | 254 | 257 | 261 | 268 | 273 | 278 | 282 | 285 | 63 | 42 | 40 | 41 | 39 | 39 | 39 | 37 | 33 | 26 |
| December | 30 | 14 | 8 | 13 | 21 | 51 | 100 | 265 | 267 | 270 | 274 | 276 | 281 | 286 | 285 | 40 | 33 | 41 | 41 | 40 | 40 | 40 | 39 | 35 | 30 |

Table 10.8 presents assessment results for propagation conditions on Petropavlovsk-Kamchatski to Moscow fixed service link, using transmitting and receiving directional antennae. The analysis of the obtained results shows that application of directional antennae would increase significantly time period when this fixed service link can be used for data transmission. Thus contrary to omnidirectional antennae, usage of Yagi antennae would provide operation of the fixed service link during the whole year.

Table 10.9 contains simulation results for propagation conditions on Astana-Moscow interfering link. Calculations assumed that emission from Astana located amateur service transmitter omnidirectional antennae would affect the fixed service receiver located in Moscow and using directional antennae.

Table 10.10 presents estimates of effect from an amateur service transmitter located in Astana and using omnidirectional antenna on operation of the Petropavlovsk-Kamchatski to Moscow fixed service link receiver, using directional antennae. The analysis of the obtained results shows that in spite of application of directional antennae interference caused by the amateur service transmitter could prevent operation of the fixed service link by more than 50% of the available time period and would significantly increase the bit error rate of the message.

Table 10.11 presents simulation results for propagation conditions of interference caused by transmitter located in the Equator and using omnidirectional antenna.

Table 10.12 presents simulation results of interference impact from the amateur service transmitter located in the Equator on Petropavlovsk-Kamchatski to Moscow fixed service link. The analysis of the obtained results shows that usage of the directional antennae would not allow to avoid the transmitter influence located at the distance of 6 200 km from the fixed service link receiver. It can be concluded that it would quite difficult to use separation distances to ensure compatibility between the amateur service transmitters and the fixed link.

### 10.1.3	Study results for multihop links using omnidirectional antennas under fading conditions

The estimation results for Petropavlovsk-Kamchatski to Moscow multihop link assuming omnidirectional antennas and fading conditions are presented in Tables 10.13-10.17.

Table 10.13

Propagation conditions for Petropavlovsk-Kamchatski to Moscow link using 
omnidirectional antennas under fading conditions

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna of 0 dB gain, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 29 | 24 | 18 | 17 | 14 | 11 | 10 | 15 | 18 | 23 | 28 | 33 | 38 | 44 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 41 | 34 | 29 |
| February | 14 | 7 | -3 | -7 | -9 | -9 | -8 | -8 | -11 | 2 | 13 | 23 | 31 | 37 | 44 | 45 | 45 | 45 | 45 | 44 | 44 | 40 | 33 | 24 | 14 |
| March | 1 | -7 | -10 | -16 | -20 | -21 | -21 | -19 | -18 | -14 | -2 | 9 | 17 | 26 | 33 | 38 | 44 | 44 | 45 | 44 | 39 | 32 | 23 | 15 | 1 |
| April | -8 | -15 | -19 | -24 | -28 | -30 | -31 | -33 | -31 | -26 | -19 | -11 | 1 | 10 | 18 | 26 | 32 | 38 | 39 | 34 | 27 | 21 | 13 | 0 | -8 |
| May | -19 | -26 | -220 | -220 | -41 | -45 | -45 | -42 | -40 | -36 | -31 | -25 | -20 | -13 | -1 | 8 | 14 | 19 | 20 | 15 | 11 | 4 | -5 | -12 | -19 |
| June | -32 | -37 | -41 | -220 | -220 | -52 | -47 | -47 | -44 | -40 | -37 | -31 | -25 | -19 | -15 | -6 | 3 | 7 | 8 | 5 | 0 | -9 | -16 | -27 | -32 |
| July | -25 | -29 | -32 | -34 | -36 | -35 | -39 | -35 | -33 | -34 | -30 | -25 | -15 | -13 | -3 | 4 | 9 | 13 | 13 | 10 | 3 | -3 | -12 | -18 | -25 |
| August | -19 | -24 | -28 | -32 | -35 | -37 | -38 | -36 | -37 | -33 | -28 | -21 | -12 | 0 | 8 | 16 | 22 | 25 | 24 | 20 | 14 | 7 | -4 | -13 | -19 |
| September | -5 | -13 | -18 | -24 | -28 | -31 | -32 | -35 | -33 | -26 | -17 | -2 | 5 | 14 | 23 | 32 | 39 | 44 | 45 | 40 | 33 | 25 | 18 | 9 | -5 |
| October | 16 | 10 | 0 | -6 | -9 | -10 | -9 | -9 | -12 | -3 | 7 | 14 | 23 | 30 | 36 | 43 | 44 | 44 | 45 | 44 | 44 | 40 | 34 | 27 | 16 |
| November | 30 | 25 | 18 | 16 | 11 | 10 | 11 | 11 | 14 | 18 | 25 | 30 | 34 | 40 | 43 | 44 | 44 | 44 | 44 | 44 | 44 | 47 | 42 | 36 | 30 |
| December | 32 | 18 | 13 | 18 | 18 | 17 | 19 | 21 | 23 | 26 | 31 | 32 | 40 | 44 | 43 | 30 | 34 | 45 | 45 | 45 | 44 | 45 | 45 | 38 | 32 |

TABLE 10.14

Propagation conditions in Astana – Moscow interfering link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Astana – Moscow, isotropic antenna of 0 dB gain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 40 | 40 | 40 | 39 | 30 | 23 | 16 | 12 | 11 | 11 | 14 | 20 | 28 | 37 | 43 | 42 | 40 | 40 | 41 | 41 | 41 | 42 | 42 | 42 | 40 |
| February | 40 | 40 | 40 | 32 | 28 | 19 | 15 | 12 | 11 | 12 | 14 | 18 | 25 | 34 | 41 | 43 | 42 | 41 | 41 | 41 | 41 | 41 | 41 | 40 | 40 |
| March | 40 | 41 | 38 | 29 | 22 | 15 | 11 | 9 | 8 | 9 | 11 | 14 | 18 | 25 | 33 | 40 | 42 | 42 | 41 | 39 | 39 | 40 | 40 | 40 | 40 |
| April | 40 | 38 | 31 | 24 | 16 | 12 | 10 | 8 | 7 | 7 | 9 | 11 | 14 | 18 | 25 | 32 | 37 | 42 | 42 | 41 | 41 | 40 | 40 | 40 | 40 |
| May | 38 | 31 | 27 | 19 | 14 | 10 | 8 | 6 | 6 | 6 | 7 | 9 | 11 | 15 | 19 | 26 | 31 | 35 | 41 | 42 | 42 | 43 | 42 | 41 | 38 |
| June | 32 | 29 | 24 | 17 | 12 | 9 | 6 | 5 | 4 | 4 | 5 | 7 | 9 | 13 | 17 | 22 | 28 | 30 | 35 | 40 | 40 | 41 | 39 | 34 | 32 |
| July | 32 | 29 | 25 | 19 | 13 | 9 | 7 | 5 | 4 | 4 | 5 | 8 | 10 | 14 | 19 | 25 | 30 | 33 | 39 | 40 | 40 | 41 | 40 | 37 | 32 |
| August | 38 | 32 | 27 | 21 | 15 | 11 | 8 | 6 | 6 | 6 | 7 | 9 | 11 | 16 | 22 | 28 | 32 | 37 | 41 | 40 | 40 | 40 | 40 | 39 | 38 |
| September | 40 | 40 | 32 | 26 | 18 | 12 | 9 | 7 | 6 | 7 | 8 | 10 | 13 | 18 | 26 | 33 | 42 | 42 | 42 | 41 | 40 | 40 | 40 | 40 | 40 |
| October | 40 | 40 | 42 | 35 | 29 | 21 | 17 | 14 | 13 | 13 | 14 | 17 | 21 | 28 | 34 | 42 | 42 | 42 | 41 | 41 | 40 | 40 | 40 | 40 | 40 |
| November | 40 | 40 | 40 | 41 | 35 | 28 | 21 | 17 | 15 | 16 | 17 | 21 | 27 | 34 | 42 | 42 | 40 | 40 | 39 | 39 | 39 | 40 | 40 | 40 | 40 |
| December | 40 | 40 | 40 | 39 | 37 | 27 | 19 | 13 | 11 | 11 | 13 | 19 | 28 | 39 | 42 | 40 | 40 | 40 | 41 | 41 | 41 | 41 | 42 | 41 | 40 |

TABLE 10.15

Propagation fading conditions on Petropavlovsk-Kamchatski – Moscow link using omnidirectional antennas with interference caused by the amateur service transmitter located in Astana

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna of 0 dB gain, |  |  |  |  |  |  |  |  |  |  | Interfering link: Astana – Moscow, isotropic antenna of 0 dB gain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  | Output power |  |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.0 | 2.0 | 3.0 | 5.0 | 5.0 | 4.0 | 4.0 | 4.0 | 3.0 |  |  |  |
| February |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.0 | 2.0 | 3.0 | 4.0 | 4.0 | 3.0 | 3.0 |  |  |  |  |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 | 2.0 | 4.0 | 5.0 |  |  |  |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -3 | 2.0 | 3.0 |  |  |  |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.0 | 2.0 | 2.0 | 4.0 | 3.0 | 4.0 |  |  |  |  |
| November |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.0 | 2.0 | 4.0 | 4.0 | 5.0 | 5.0 | 5.0 | 7.0 | 2.0 |  |  |
| December |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.0 | 1.0 | -10 | -6 | 5.0 | 4.0 | 4.0 | 3.0 | 4.0 | 3.0 |  |  |

table 10.16

Propagation conditions in Equator – Moscow link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Terminator (equator) – Moscow, isotropic antenna of 0 dB gain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 28 | 28 | 29 | 28 | 14 | -24 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -59 | -28 | 15 | 25 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| February | 29 | 29 | 29 | 29 | 6 | -41 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 11 | 26 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 |
| March | 29 | 29 | 29 | 27 | -5 | -58 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -8 | 23 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| April | 29 | 29 | 29 | 16 | -16 | -62 | -104 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -29 | 8 | 26 | 28 | 28 | 29 | 29 | 29 | 29 | 29 |
| May | 29 | 29 | 24 | 10 | -25 | -66 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -5 | 16 | 28 | 29 | 29 | 29 | 29 | 29 | 29 |
| June | 28 | 28 | 21 | 4 | -29 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -95 | -60 | -221 | -9 | 12 | 24 | 27 | 27 | 27 | 27 | 27 | 28 |
| July | 28 | 28 | 22 | 5 | -33 | -74 | -220 | -220 | -134 | -220 | -220 | -220 | -220 | -93 | -54 | -33 | -3 | 15 | 25 | 27 | 27 | 27 | 27 | 27 | 28 |
| August | 28 | 29 | 28 | 12 | -23 | -72 | -108 | -220 | -220 | -220 | -220 | -220 | -220 | -91 | -220 | -221 | 5 | 22 | 27 | 27 | 27 | 27 | 27 | 27 | 28 |
| September | 29 | 29 | 29 | 24 | -3 | -51 | -97 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -22 | 17 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| October | 29 | 29 | 29 | 29 | 12 | -34 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -16 | 25 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 |
| November | 29 | 29 | 29 | 29 | 19 | -13 | -52 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 2 | 26 | 28 | 28 | 28 | 28 | 28 | 28 | 29 | 29 |
| December | 28 | 28 | 29 | 31 | 21 | -11 | -60 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 11 | 25 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |

table 10.17

Propagation fading conditions in Petropavlovsk-Kamchatski to Moscow link using omnidirectional antennas with interference caused by the amateur service transmitter located in Equator

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, isotropic antenna of 0 dB gain, |  |  |  |  |  |  |  |  |  |  | Interfering link: Equator – Moscow, isotropic antenna of 0 dB gain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  | Output power |  |  | 0.1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January |  |  |  |  |  |  |  |  |  |  |  |  |  | 103 | 73 | 30 | 20 | 17 | 17 | 17 | 17 | 17 |  |  |  |
| February |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 264 | 34 | 19 | 16 | 16 | 15 | 15 |  |  |  |  |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 | 16 | 17 | 16 |  |  |  |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 22 | 16 | 17 |  |  |  |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 59 | 19 | 16 | 17 | 16 | 16 |  |  |  |  |
| November |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 263 | 42 | 18 | 16 | 16 | 16 | 16 | 19 | 14 |  |  |
| December |  |  |  |  |  |  |  |  |  |  |  |  |  | 44 | 263 | 19 | 9 | 17 | 17 | 17 | 16 | 17 | 17 |  |  |

Table 10.13 contains results of simulating radio waves propagation fading conditions in Petropavlovsk-Kamchatski to Moscow links. The obtained results show that operation of FS links under fading propagation conditions operation of FS links would be feasible for a limited periods of time to significantly less than those typical for unfading propagation as shown in Document PTC(13)003. Radio link operation period under fading conditions could be increased significantly with using directional antennas and by transmitter gain increasing or by their combinations.

Table 10.14 presents results of simulating propagation conditions in Astana – Moscow link of 227 km in length.

Table 10.15 reflacts effect from amateur transmitter on FS link under fading conditions. Similar to unfading conditions (see Doc. PTC(13)300) the effect caused by emissions from envisioned amateur transmitter would prevent FS link from operation in any time of day and year. Such conclusions confirm earlier estimates for required protection distances of at least 2 300 km.

Table 10.16 presents results of simulating propagation conditions in Equator – Moscow link.

Table 10.17 reflects of Equator located amateur transmitter on operation of Petropavlovsk-Kamchatski to Moscow FS link under fading conditions. Similar to unfading conditions (see Doc. PTC(13)300) interference from amateur transmitters would reduce potential time of FS link operation by over 50%.

### 10.1.4	Study results for multihop links using directional antennas under fading conditions

To clarify feasibility of sharing between amateur and fixed stations effect of using directional antennas on operation of Petropavlovsk-Kamchatski to Moscow FS link under fading conditions was analyzed/ The analysis assumed usage of directional Yagi antennas (of 13.4 dB gain) 
in transmitting and receiving terminal points of the link. The simulation assumed 10 sunspots. Simulation results are shown in Figures 10.18 – 10.22.

table 10.18

Propagation fading conditions in Petropavlovsk-Kamchatski to Moscow link with directional antennas

| Signal-to-interference ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna gain of 13.4 dB< receive and transmit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 53 | 47 | 40 | 40 | 38 | 35 | 34 | 38 | 41 | 45 | 52 | 57 | 61 | 66 | 67 | 67 | 67 | 67 | 66 | 66 | 66 | 66 | 62 | 58 | 53 |
| February | 35 | 29 | 19 | 15 | 14 | 14 | 15 | 15 | 12 | 24 | 35 | 46 | 54 | 60 | 66 | 67 | 67 | 67 | 67 | 67 | 66 | 63 | 56 | 47 | 35 |
| March | 22 | 14 | 10 | 4 | 1 | 0 | 0 | 2 | 2 | 7 | 18 | 29 | 38 | 49 | 56 | 61 | 66 | 66 | 67 | 66 | 62 | 55 | 46 | 35 | 22 |
| April | 11 | 4 | -1 | -6 | -10 | -12 | -12 | -15 | -12 | -7 | 1 | 8 | 21 | 29 | 40 | 49 | 56 | 59 | 60 | 57 | 50 | 44 | 33 | 19 | 11 |
| May | -1 | -8 | -220 | -220 | -23 | -26 | -27 | -26 | -24 | -19 | -13 | -7 | 0 | 7 | 20 | 30 | 38 | 43 | 44 | 38 | 31 | 25 | 14 | 6 | -1 |
| June | -17 | -22 | -27 | -220 | -220 | -33 | -30 | -32 | -28 | -24 | -19 | -13 | -7 | 0 | 6 | 16 | 25 | 30 | 31 | 28 | 21 | 10 | 3 | -11 | -17 |
| July | -8 | -13 | -17 | -19 | -21 | -18 | -22 | -20 | -17 | -19 | -15 | -8 | 4 | 6 | 17 | 26 | 33 | 36 | 37 | 32 | 24 | 17 | 6 | -2 | -8 |
| August | -1 | -6 | -11 | -15 | -18 | -20 | -21 | -18 | -20 | -15 | -10 | -2 | 7 | 20 | 29 | 39 | 45 | 49 | 47 | 44 | 35 | 28 | 15 | 6 | -1 |
| September | 15 | 7 | 1 | -5 | -9 | -11 | -12 | -16 | -13 | -6 | 3 | 18 | 26 | 36 | 46 | 55 | 60 | 65 | 66 | 61 | 56 | 47 | 40 | 30 | 15 |
| October | 38 | 32 | 22 | 16 | 13 | 12 | 14 | 14 | 11 | 19 | 29 | 35 | 44 | 53 | 59 | 64 | 66 | 66 | 67 | 66 | 66 | 63 | 58 | 49 | 38 |
| November | 52 | 48 | 41 | 38 | 34 | 33 | 34 | 34 | 37 | 41 | 48 | 53 | 58 | 62 | 65 | 66 | 66 | 66 | 67 | 66 | 66 | 66 | 64 | 59 | 52 |
| December | 56 | 41 | 36 | 41 | 42 | 41 | 42 | 45 | 47 | 50 | 54 | 56 | 61 | 66 | 65 | 52 | 57 | 67 | 67 | 67 | 67 | 67 | 65 | 61 | 56 |

table 10.19

Propagation conditions in Astana – Moscow interfering link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Astana (isotropic antenna gain of 0 dB) – Moscow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 37 | 38 | 37 | 34 | 29 | 21 | 12 | 8 | 7 | 8 | 11 | 18 | 27 | 35 | 39 | 38 | 36 | 37 | 37 | 37 | 37 | 38 | 38 | 38 | 37 |
| February | 38 | 39 | 38 | 30 | 26 | 15 | 11 | 8 | 7 | 7 | 10 | 15 | 23 | 32 | 39 | 39 | 38 | 36 | 37 | 38 | 38 | 37 | 37 | 37 | 38 |
| March | 39 | 38 | 36 | 27 | 19 | 10 | 7 | 5 | 4 | 5 | 7 | 10 | 14 | 23 | 31 | 38 | 38 | 38 | 37 | 36 | 37 | 38 | 38 | 39 | 39 |
| April | 37 | 36 | 29 | 21 | 12 | 8 | 5 | 3 | 2 | 3 | 4 | 7 | 10 | 14 | 22 | 31 | 36 | 39 | 38 | 37 | 37 | 37 | 38 | 38 | 37 |
| May | 36 | 29 | 24 | 16 | 9 | 6 | 4 | 2 | 1 | 2 | 3 | 5 | 7 | 10 | 15 | 24 | 30 | 34 | 38 | 38 | 38 | 38 | 38 | 37 | 36 |
| June | 30 | 26 | 21 | 12 | 8 | 5 | 2 | 0 | 0 | 0 | 1 | 3 | 5 | 8 | 12 | 20 | 26 | 28 | 34 | 36 | 36 | 37 | 36 | 32 | 30 |
| July | 30 | 27 | 22 | 15 | 8 | 5 | 2 | 0 | 0 | 0 | 1 | 3 | 5 | 9 | 16 | 22 | 28 | 31 | 35 | 37 | 36 | 37 | 36 | 35 | 30 |
| August | 36 | 30 | 25 | 18 | 10 | 7 | 4 | 2 | 1 | 1 | 3 | 5 | 7 | 11 | 19 | 26 | 30 | 36 | 37 | 36 | 36 | 36 | 36 | 35 | 36 |
| September | 39 | 38 | 31 | 24 | 15 | 8 | 5 | 3 | 2 | 2 | 4 | 6 | 9 | 14 | 25 | 31 | 38 | 38 | 38 | 37 | 37 | 37 | 38 | 39 | 39 |
| October | 39 | 38 | 38 | 33 | 27 | 17 | 13 | 10 | 8 | 9 | 10 | 13 | 17 | 26 | 32 | 39 | 38 | 38 | 37 | 37 | 36 | 37 | 37 | 38 | 39 |
| November | 38 | 38 | 37 | 37 | 33 | 26 | 17 | 13 | 11 | 12 | 13 | 17 | 25 | 33 | 38 | 38 | 36 | 36 | 36 | 36 | 36 | 37 | 37 | 37 | 38 |
| December | 37 | 38 | 39 | 36 | 35 | 25 | 17 | 9 | 7 | 7 | 10 | 17 | 27 | 37 | 38 | 37 | 39 | 38 | 38 | 38 | 37 | 38 | 38 | 37 | 37 |

table 10.20

Propagation fading conditions in Petropavlovsk-Kamchatski – Moscow link using directional antennas 
with interference caused by a transmitter located in Astana

| Signal-to-interference ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna gain of 13.4 dB< receive and transmit |  |  |  |  |  |  |  |  |  |  | Interfering link: Astana – Moscow isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  | Output power |  |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 16 | 9 |  |  |  |  |  |  |  | 37 | 41 | 39 | 34 | 31 | 28 | 29 | 31 | 30 | 29 | 29 | 29 | 28 | 24 | 20 | 16 |
| February |  |  |  |  |  |  |  |  |  |  |  | 31 | 31 | 28 | 27 | 28 | 29 | 31 | 30 | 29 | 28 | 26 | 19 | 10 |  |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 | 25 | 23 | 28 | 28 | 30 | 30 | 25 | 17 | 8 |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18 | 20 | 20 | 22 | 20 | 13 | 7 |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 | 6 | 0 |  |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 | 13 | 10 | 8 |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 | 24 | 22 | 27 | 28 | 24 | 19 | 10 |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  | 27 | 27 | 27 | 25 | 28 | 28 | 30 | 29 | 30 | 26 | 21 | 11 |  |
| November | 14 | 10 |  |  |  |  |  |  |  |  | 35 | 36 | 33 | 29 | 27 | 28 | 30 | 30 | 31 | 30 | 30 | 29 | 27 | 22 | 14 |
| December | 19 |  |  |  | 7 |  | 25 | 36 | 40 | 43 | 44 | 39 | 34 | 29 | 27 | 15 | 18 | 29 | 29 | 29 | 30 | 29 | 27 | 24 | 19 |

TABLe 10.21

Propagation conditions in Equator – Moscow link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Terminator (Equator) – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 26 | 27 | 28 | 27 | 15 | -23 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -57 | -27 | 15 | 24 | 25 | 26 | 27 | 27 | 27 | 26 | 26 | 26 |
| February | 26 | 27 | 28 | 27 | 7 | -40 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 12 | 24 | 25 | 26 | 27 | 27 | 27 | 27 | 26 | 26 |
| March | 26 | 27 | 28 | 26 | -4 | -57 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -7 | 23 | 24 | 25 | 26 | 26 | 26 | 26 | 26 | 26 |
| April | 27 | 27 | 27 | 17 | -15 | -61 | -103 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -28 | 9 | 24 | 25 | 26 | 26 | 26 | 26 | 26 | 27 |
| May | 27 | 27 | 25 | 11 | -24 | -65 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -4 | 17 | 25 | 25 | 26 | 26 | 26 | 27 | 27 |
| June | 26 | 26 | 22 | 5 | -28 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -95 | -59 | -221 | -8 | 13 | 22 | 24 | 24 | 24 | 25 | 25 | 26 |
| July | 26 | 27 | 23 | 6 | -31 | -73 | -220 | -220 | -135 | -220 | -220 | -220 | -220 | -92 | -53 | -31 | -2 | 16 | 23 | 24 | 25 | 25 | 25 | 25 | 26 |
| August | 26 | 28 | 27 | 13 | -21 | -71 | -108 | -220 | -220 | -220 | -220 | -220 | -220 | -90 | -220 | -221 | 6 | 21 | 23 | 24 | 25 | 25 | 25 | 25 | 26 |
| September | 27 | 28 | 28 | 24 | -2 | -50 | -96 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -21 | 17 | 24 | 25 | 26 | 26 | 27 | 27 | 27 | 27 |
| October | 27 | 27 | 28 | 26 | 12 | -33 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -15 | 23 | 25 | 25 | 26 | 26 | 27 | 27 | 26 | 27 |
| November | 26 | 27 | 28 | 27 | 20 | -12 | -51 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 3 | 24 | 26 | 26 | 27 | 27 | 27 | 27 | 26 | 26 |
| December | 26 | 27 | 28 | 28 | 21 | -10 | -58 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | -220 | 12 | 24 | 26 | 26 | 27 | 27 | 27 | 26 | 26 | 26 |

table 10.22

Propagation fading conditions in Petropavlovsk-Kamchatski – Moscow link using directional antennas 
with interference caused by a transmitter located in Equator

| Signal-to interference ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Petropavlovsk-Kamchatski to Moscow, Yagi antenna gain of 13.4 dB |  |  |  |  |  |  |  |  |  |  | Interfering link: Equator – Moscow isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN |  | 10 |  |  |  |  | Output power |  |  | 0.1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 27 | 20 |  |  |  |  |  |  |  | 265 | 272 | 277 | 281 | 123 | 94 | 52 | 43 | 42 | 40 | 39 | 39 | 39 | 36 | 32 | 27 |
| February |  |  |  |  |  |  |  |  |  |  |  | 266 | 274 | 280 | 286 | 55 | 43 | 42 | 41 | 40 | 39 | 36 | 29 | 21 |  |
| March |  |  |  |  |  |  |  |  |  |  |  |  |  | 269 | 276 | 68 | 43 | 42 | 42 | 40 | 36 | 29 | 20 |  |  |
| April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 77 | 47 | 35 | 35 | 31 | 24 | 18 |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 | 19 |  |  |  |  |  |  |
| June |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| July |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| August |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39 | 28 | 24 | 20 |  |  |  |  |  |
| September |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 266 | 76 | 43 | 41 | 41 | 35 | 30 | 20 |  |  |  |
| October |  |  |  |  |  |  |  |  |  |  |  |  | 264 | 273 | 279 | 79 | 43 | 41 | 42 | 40 | 40 | 36 | 31 | 23 |  |
| November | 26 | 21 |  |  |  |  |  |  |  |  | 268 | 273 | 278 | 282 | 285 | 63 | 42 | 40 | 41 | 39 | 39 | 39 | 37 | 33 | 26 |
| December | 30 |  |  |  | 21 |  | 100 | 265 | 267 | 270 | 274 | 276 | 281 | 286 | 285 | 40 | 33 | 41 | 41 | 40 | 40 | 40 | 39 | 35 | 30 |

Table 10.18 contains results of estimating propagation conditions in Petropavlovsk-Kamchatski to Moscow FS link with directional antennas in transmitter and receiver. Analysis of obtained results shows that employment of directional antennas would result in significant increasing for period of time to be used for transferring data in such FS links. Contrary to employment of omnidirectional antennas,  usage of Yagi antennas would provide for operating the FS link for 10 months per year.

Table 10.19 contains simulation results of propagation conditions in Astana– Moscow link. The simulation assumed that emissions from Astana located amateur transmitter omnidirectional antenna would affect Moscow deployed FS receiver with directional antenna.

Table 10.10 presents results of estimating effect from Astana located amateur transmitter with omnidirectional antenna on Petropavlovsk-Kamchatski to Moscow FS link using directional antennas. analysis of obtained results showed that in spite of using directional antennas in FS links interference caused by amateur transmitter emissions could result not only in preventing FS links from operation for over 98% of operational time available but also in significant increasing the probability of bit errors in messages received.

Table 10.21 presents simulation results for interfering emission propagation from Equator deployed amateur transmitter with omnidirectional antenna.

Table 10.22 contains results of estimating the impact of the Equator located amateur service transmitter on operation of Petropavlovsk-Kamchatski – Moscow fixed link. The analysis of the obtained results shows that usage of directional antennas would not result in avoiding effect on FS receiver by transmitter located at a distance of 6 200 km from it. Therefore a conclusion could be drawn that it would be rather difficult to use geographical separation to provide for compatibility of amateur  transmitters and FS links.

### 10.1.5	Study results for single-hop links using directional antennas under steady conditions

Tables 10.23 – 10.27 below reflect results of study in effect from amateur stations on operation of Murmansk – Moscow single-hop FS link with omnidirectional antennas.

TABLE 10.23

Signal non-faded propagation conditions in Murmansk – Moscow link with directional antennae

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Murmansk to Moscow, istropic antenna gain of 0 dB, receive and transmit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN = |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 55.0 | 46.0 | 39.0 | 37.0 | 41.0 | 54.0 | 52.0 | 50.0 | 53.0 | 52.0 | 52.0 | 54.0 | 55.0 | 57.0 | 60.0 | 60.0 | 59.0 | 47.0 | 46.0 | 49.0 | 54.0 | 59.0 | 59.0 | 59.0 | 55.0 |
| February | 45.0 | 40.0 | 35.0 | 37.0 | 50.0 | 52.0 | 49.0 | 46.0 | 50.0 | 49.0 | 49.0 | 51.0 | 52.0 | 54.0 | 57.0 | 61.0 | 59.0 | 59.0 | 52.0 | 47.0 | 46.0 | 47.0 | 50.0 | 50.0 | 45.0 |
| March | 40.0 | 40.0 | 41.0 | 52.0 | 55.0 | 47.0 | 46.0 | 45.0 | 46.0 | 46.0 | 46.0 | 49.0 | 50.0 | 51.0 | 51.0 | 55.0 | 57.0 | 58.0 | 56.0 | 56.0 | 55.0 | 45.0 | 41.0 | 42.0 | 40.0 |
| April | 47.0 | 52.0 | 56.0 | 55.0 | 50.0 | 47.0 | 46.0 | 45.0 | 46.0 | 46.0 | 46.0 | 48.0 | 49.0 | 50.0 | 49.0 | 51.0 | 54.0 | 56.0 | 57.0 | 57.0 | 55.0 | 55.0 | 49.0 | 48.0 | 47.0 |
| May | 58.0 | 57.0 | 53.0 | 52.0 | 49.0 | 45.0 | 44.0 | 43.0 | 45.0 | 44.0 | 44.0 | 47.0 | 47.0 | 48.0 | 47.0 | 48.0 | 49.0 | 50.0 | 53.0 | 54.0 | 55.0 | 55.0 | 55.0 | 58.0 | 58.0 |
| June | 55.0 | 55.0 | 54.0 | 51.0 | 50.0 | 47.0 | 46.0 | 45.0 | 46.0 | 46.0 | 45.0 | 46.0 | 46.0 | 47.0 | 44.0 | 45.0 | 46.0 | 46.0 | 49.0 | 51.0 | 52.0 | 53.0 | 53.0 | 55.0 | 55.0 |
| July | 54.0 | 54.0 | 54.0 | 51.0 | 50.0 | 47.0 | 46.0 | 45.0 | 46.0 | 46.0 | 46.0 | 46.0 | 46.0 | 47.0 | 44.0 | 45.0 | 46.0 | 47.0 | 50.0 | 51.0 | 53.0 | 54.0 | 54.0 | 55.0 | 54.0 |
| August | 56.0 | 56.0 | 55.0 | 53.0 | 51.0 | 48.0 | 47.0 | 46.0 | 46.0 | 46.0 | 46.0 | 46.0 | 47.0 | 48.0 | 45.0 | 46.0 | 47.0 | 50.0 | 51.0 | 53.0 | 56.0 | 56.0 | 55.0 | 56.0 | 56.0 |
| September | 41.0 | 41.0 | 44.0 | 51.0 | 52.0 | 46.0 | 45.0 | 43.0 | 45.0 | 45.0 | 45.0 | 47.0 | 48.0 | 49.0 | 48.0 | 50.0 | 54.0 | 56.0 | 58.0 | 56.0 | 55.0 | 55.0 | 44.0 | 43.0 | 41.0 |
| October | 41.0 | 40.0 | 39.0 | 45.0 | 57.0 | 53.0 | 49.0 | 48.0 | 49.0 | 48.0 | 48.0 | 51.0 | 52.0 | 53.0 | 53.0 | 57.0 | 58.0 | 57.0 | 56.0 | 56.0 | 55.0 | 47.0 | 43.0 | 44.0 | 41.0 |
| November | 43.0 | 39.0 | 36.0 | 38.0 | 44.0 | 54.0 | 54.0 | 50.0 | 51.0 | 50.0 | 50.0 | 53.0 | 54.0 | 57.0 | 57.0 | 58.0 | 57.0 | 56.0 | 45.0 | 41.0 | 40.0 | 42.0 | 45.0 | 47.0 | 43.0 |
| December | 46.0 | 41.0 | 33.0 | 31.0 | 33.0 | 40.0 | 53.0 | 51.0 | 54.0 | 52.0 | 52.0 | 53.0 | 56.0 | 59.0 | 58.0 | 50.0 | 39.0 | 35.0 | 36.0 | 38.0 | 43.0 | 47.0 | 50.0 | 49.0 | 46.0 |

TABLE 10.24

Propagation conditions in Kotka – Moscow interference link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Kotka (isotropic antenna gain of 0 dB) – Moscow, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN = |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 27.0 | 24.0 | 21.0 | 19.0 | 21.0 | 25.0 | 41.0 | 42.0 | 44.0 | 43.0 | 42.0 | 43.0 | 45.0 | 47.0 | 47.0 | 47.0 | 32.0 | 24.0 | 22.0 | 23.0 | 25.0 | 27.0 | 29.0 | 29.0 | 27.0 |
| February | 23.0 | 20.0 | 18.0 | 19.0 | 23.0 | 39.0 | 42.0 | 40.0 | 41.0 | 40.0 | 40.0 | 41.0 | 43.0 | 45.0 | 46.0 | 48.0 | 47.0 | 34.0 | 27.0 | 24.0 | 23.0 | 23.0 | 25.0 | 25.0 | 23.0 |
| March | 19.0 | 19.0 | 19.0 | 22.0 | 41.0 | 41.0 | 40.0 | 38.0 | 38.0 | 37.0 | 37.0 | 39.0 | 40.0 | 41.0 | 42.0 | 44.0 | 46.0 | 46.0 | 45.0 | 33.0 | 25.0 | 22.0 | 20.0 | 21.0 | 19.0 |
| April | 22.0 | 23.0 | 26.0 | 41.0 | 45.0 | 40.0 | 39.0 | 37.0 | 38.0 | 37.0 | 37.0 | 38.0 | 39.0 | 40.0 | 40.0 | 42.0 | 44.0 | 46.0 | 46.0 | 45.0 | 44.0 | 29.0 | 24.0 | 23.0 | 22.0 |
| May | 36.0 | 37.0 | 42.0 | 44.0 | 43.0 | 39.0 | 37.0 | 36.0 | 37.0 | 36.0 | 36.0 | 37.0 | 37.0 | 38.0 | 38.0 | 40.0 | 41.0 | 44.0 | 45.0 | 45.0 | 45.0 | 44.0 | 44.0 | 40.0 | 36.0 |
| June | 43.0 | 44.0 | 45.0 | 44.0 | 42.0 | 40.0 | 39.0 | 37.0 | 36.0 | 36.0 | 36.0 | 36.0 | 36.0 | 37.0 | 36.0 | 37.0 | 38.0 | 40.0 | 41.0 | 42.0 | 43.0 | 43.0 | 43.0 | 43.0 | 43.0 |
| July | 34.0 | 40.0 | 41.0 | 44.0 | 42.0 | 40.0 | 39.0 | 38.0 | 37.0 | 36.0 | 36.0 | 36.0 | 36.0 | 38.0 | 36.0 | 38.0 | 39.0 | 41.0 | 42.0 | 42.0 | 43.0 | 43.0 | 42.0 | 39.0 | 34.0 |
| August | 27.0 | 26.0 | 40.0 | 40.0 | 43.0 | 41.0 | 40.0 | 38.0 | 37.0 | 37.0 | 36.0 | 36.0 | 37.0 | 38.0 | 37.0 | 39.0 | 40.0 | 42.0 | 43.0 | 44.0 | 44.0 | 42.0 | 34.0 | 29.0 | 27.0 |
| September | 19.0 | 18.0 | 19.0 | 26.0 | 39.0 | 40.0 | 38.0 | 37.0 | 37.0 | 36.0 | 36.0 | 37.0 | 38.0 | 39.0 | 40.0 | 42.0 | 44.0 | 45.0 | 46.0 | 45.0 | 35.0 | 27.0 | 21.0 | 20.0 | 19.0 |
| October | 21.0 | 20.0 | 20.0 | 22.0 | 30.0 | 44.0 | 43.0 | 41.0 | 41.0 | 40.0 | 39.0 | 40.0 | 41.0 | 43.0 | 43.0 | 45.0 | 45.0 | 45.0 | 45.0 | 33.0 | 27.0 | 23.0 | 21.0 | 22.0 | 21.0 |
| November | 22.0 | 20.0 | 18.0 | 19.0 | 22.0 | 30.0 | 44.0 | 43.0 | 43.0 | 42.0 | 41.0 | 42.0 | 44.0 | 45.0 | 45.0 | 45.0 | 37.0 | 27.0 | 22.0 | 19.0 | 18.0 | 19.0 | 21.0 | 24.0 | 22.0 |
| December | 25.0 | 22.0 | 19.0 | 16.0 | 18.0 | 22.0 | 32.0 | 43.0 | 44.0 | 43.0 | 42.0 | 43.0 | 44.0 | 46.0 | 46.0 | 31.0 | 21.0 | 16.0 | 15.0 | 16.0 | 20.0 | 23.0 | 25.0 | 26.0 | 25.0 |

TABLE 10.25

Interference effect on signal non-faded propagation in Murmansk – 
Moscow link using omnidirectional antennas

| Signal-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Murmansk – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  | Interfering link: Kotka – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5 kW |  | SSN = |  | 10 |  |  |  |  | Output power |  |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 28.0 | 22.0 | 18.0 | 18.0 | 20.0 | 29.0 | 11.0 | 8.0 | 9.0 | 9.0 | 10.0 | 11.0 | 10.0 | 10.0 | 13.0 | 13.0 | 27.0 | 23.0 | 24.0 | 26.0 | 29.0 | 32.0 | 30.0 | 30.0 | 28.0 |
| February | 22.0 | 20.0 | 17.0 | 18.0 | 27.0 | 13.0 | 7.0 | 6.0 | 9.0 | 9.0 | 9.0 | 10.0 | 9.0 | 9.0 | 11.0 | 13.0 | 12.0 | 25.0 | 25.0 | 23.0 | 23.0 | 24.0 | 25.0 | 25.0 | 22.0 |
| March | 21.0 | 21.0 | 22.0 | 30.0 | 14.0 | 6.0 | 6.0 | 7.0 | 8.0 | 9.0 | 9.0 | 10.0 | 10.0 | 10.0 | 9.0 | 11.0 | 11.0 | 12.0 | 11.0 | 23.0 | 30.0 | 23.0 | 21.0 | 21.0 | 21.0 |
| April | 25.0 | 29.0 | 30.0 | 14.0 | 5.0 | 7.0 | 7.0 | 8.0 | 8.0 | 9.0 | 9.0 | 10.0 | 10.0 | 10.0 | 9.0 | 9.0 | 10.0 | 10.0 | 11.0 | 12.0 | 11.0 | 26.0 | 25.0 | 25.0 | 25.0 |
| May | 22.0 | 20.0 | 11.0 | 8.0 | 6.0 | 6.0 | 7.0 | 7.0 | 8.0 | 8.0 | 8.0 | 10.0 | 10.0 | 10.0 | 9.0 | 8.0 | 8.0 | 6.0 | 8.0 | 9.0 | 10.0 | 11.0 | 11.0 | 18.0 | 22.0 |
| June | 12.0 | 11.0 | 9.0 | 7.0 | 8.0 | 7.0 | 7.0 | 8.0 | 10.0 | 10.0 | 9.0 | 10.0 | 10.0 | 10.0 | 8.0 | 8.0 | 8.0 | 6.0 | 8.0 | 9.0 | 9.0 | 10.0 | 10.0 | 12.0 | 12.0 |
| July | 20.0 | 14.0 | 13.0 | 7.0 | 8.0 | 7.0 | 7.0 | 7.0 | 9.0 | 10.0 | 10.0 | 10.0 | 10.0 | 9.0 | 8.0 | 7.0 | 7.0 | 6.0 | 8.0 | 9.0 | 10.0 | 11.0 | 12.0 | 16.0 | 20.0 |
| August | 29.0 | 30.0 | 15.0 | 13.0 | 8.0 | 7.0 | 7.0 | 8.0 | 9.0 | 9.0 | 10.0 | 10.0 | 10.0 | 10.0 | 8.0 | 7.0 | 7.0 | 8.0 | 8.0 | 9.0 | 12.0 | 14.0 | 21.0 | 27.0 | 29.0 |
| September | 22.0 | 23.0 | 25.0 | 25.0 | 13.0 | 6.0 | 7.0 | 6.0 | 8.0 | 9.0 | 9.0 | 10.0 | 10.0 | 10.0 | 8.0 | 8.0 | 10.0 | 11.0 | 12.0 | 11.0 | 20.0 | 28.0 | 23.0 | 23.0 | 22.0 |
| October | 20.0 | 20.0 | 19.0 | 23.0 | 27.0 | 9.0 | 6.0 | 7.0 | 8.0 | 8.0 | 9.0 | 11.0 | 11.0 | 10.0 | 10.0 | 12.0 | 13.0 | 12.0 | 11.0 | 23.0 | 28.0 | 24.0 | 22.0 | 22.0 | 20.0 |
| November | 21.0 | 19.0 | 18.0 | 19.0 | 22.0 | 24.0 | 10.0 | 7.0 | 8.0 | 8.0 | 9.0 | 11.0 | 10.0 | 12.0 | 12.0 | 13.0 | 20.0 | 29.0 | 23.0 | 22.0 | 22.0 | 23.0 | 24.0 | 23.0 | 21.0 |
| December | 21.0 | 19.0 | 14.0 | 15.0 | 15.0 | 18.0 | 21.0 | 8.0 | 10.0 | 9.0 | 10.0 | 10.0 | 12.0 | 13.0 | 12.0 | 19.0 | 18.0 | 19.0 | 21.0 | 22.0 | 23.0 | 24.0 | 25.0 | 23.0 | 21.0 |

TABLE 10.26

Propagation conditions in Kirkenes – Moscow interference link

| Interference-to-noise ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interfering link: Kirkenes – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 0,1 kW |  | SSN = |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 42.0 | 31.0 | 23.0 | 20.0 | 24.0 | 37.0 | 35.0 | 33.0 | 36.0 | 35.0 | 35.0 | 36.0 | 38.0 | 39.0 | 42.0 | 42.0 | 42.0 | 32.0 | 31.0 | 33.0 | 40.0 | 42.0 | 42.0 | 42.0 | 42.0 |
| February | 29.0 | 24.0 | 19.0 | 21.0 | 33.0 | 35.0 | 30.0 | 28.0 | 32.0 | 32.0 | 32.0 | 33.0 | 34.0 | 36.0 | 39.0 | 43.0 | 42.0 | 41.0 | 41.0 | 32.0 | 31.0 | 31.0 | 34.0 | 34.0 | 29.0 |
| March | 24.0 | 24.0 | 25.0 | 35.0 | 37.0 | 30.0 | 29.0 | 27.0 | 29.0 | 29.0 | 28.0 | 31.0 | 32.0 | 34.0 | 33.0 | 37.0 | 40.0 | 41.0 | 39.0 | 38.0 | 37.0 | 30.0 | 26.0 | 26.0 | 24.0 |
| April | 31.0 | 35.0 | 39.0 | 37.0 | 33.0 | 29.0 | 28.0 | 27.0 | 29.0 | 28.0 | 28.0 | 31.0 | 32.0 | 32.0 | 32.0 | 33.0 | 35.0 | 38.0 | 39.0 | 40.0 | 38.0 | 38.0 | 38.0 | 32.0 | 31.0 |
| May | 40.0 | 39.0 | 36.0 | 34.0 | 32.0 | 27.0 | 26.0 | 25.0 | 27.0 | 27.0 | 27.0 | 29.0 | 30.0 | 30.0 | 29.0 | 30.0 | 32.0 | 32.0 | 35.0 | 36.0 | 37.0 | 38.0 | 38.0 | 41.0 | 40.0 |
| June | 37.0 | 37.0 | 36.0 | 34.0 | 32.0 | 30.0 | 29.0 | 28.0 | 28.0 | 28.0 | 28.0 | 28.0 | 29.0 | 29.0 | 27.0 | 27.0 | 28.0 | 29.0 | 30.0 | 33.0 | 33.0 | 35.0 | 36.0 | 38.0 | 37.0 |
| July | 37.0 | 37.0 | 36.0 | 33.0 | 32.0 | 30.0 | 29.0 | 28.0 | 28.0 | 28.0 | 28.0 | 28.0 | 29.0 | 29.0 | 27.0 | 27.0 | 28.0 | 29.0 | 32.0 | 34.0 | 35.0 | 36.0 | 36.0 | 37.0 | 37.0 |
| August | 38.0 | 38.0 | 38.0 | 36.0 | 33.0 | 31.0 | 29.0 | 29.0 | 29.0 | 28.0 | 28.0 | 29.0 | 29.0 | 30.0 | 27.0 | 28.0 | 29.0 | 32.0 | 34.0 | 36.0 | 39.0 | 39.0 | 38.0 | 39.0 | 38.0 |
| September | 25.0 | 25.0 | 26.0 | 33.0 | 35.0 | 28.0 | 27.0 | 26.0 | 28.0 | 27.0 | 27.0 | 30.0 | 30.0 | 31.0 | 30.0 | 32.0 | 36.0 | 38.0 | 40.0 | 39.0 | 38.0 | 37.0 | 28.0 | 27.0 | 25.0 |
| October | 25.0 | 24.0 | 23.0 | 28.0 | 40.0 | 35.0 | 32.0 | 30.0 | 31.0 | 31.0 | 31.0 | 33.0 | 34.0 | 35.0 | 35.0 | 39.0 | 41.0 | 40.0 | 39.0 | 38.0 | 37.0 | 33.0 | 28.0 | 28.0 | 25.0 |
| November | 28.0 | 23.0 | 20.0 | 21.0 | 27.0 | 37.0 | 37.0 | 33.0 | 34.0 | 33.0 | 33.0 | 35.0 | 36.0 | 38.0 | 39.0 | 41.0 | 40.0 | 39.0 | 29.0 | 25.0 | 24.0 | 26.0 | 29.0 | 32.0 | 28.0 |
| December | 30.0 | 24.0 | 17.0 | 14.0 | 16.0 | 22.0 | 36.0 | 33.0 | 36.0 | 35.0 | 34.0 | 36.0 | 37.0 | 41.0 | 41.0 | 37.0 | 23.0 | 19.0 | 19.0 | 22.0 | 26.0 | 31.0 | 34.0 | 33.0 | 30.0 |

TABLE 10.27

Interference effect on signal non-faded propagation in Murmansk – 
Moscow link using omnidirectional antennas

| Signal-to-interference ratio, dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted link: Murmansk – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  | Interfering link: Kirkenes – Moscow, isotropic antenna gain of 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output power |  | 5kW |  | SSN = |  | 10 |  |  |  |  | Output power |  |  | 0,1 kW |  | SSN |  | 10 |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| January | 13.0 | 15.0 | 16.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 17.0 | 15.0 | 15.0 | 16.0 | 14.0 | 17.0 | 17.0 | 17.0 | 13.0 |
| February | 16.0 | 16.0 | 16.0 | 16.0 | 17.0 | 17.0 | 19.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 11.0 | 15.0 | 15.0 | 16.0 | 16.0 | 16.0 | 16.0 |
| March | 16.0 | 16.0 | 16.0 | 17.0 | 18.0 | 17.0 | 17.0 | 18.0 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 18.0 | 18.0 | 15.0 | 15.0 | 16.0 | 16.0 |
| April | 16.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 19.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 11.0 | 16.0 | 16.0 |
| May | 18.0 | 18.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 18.0 |
| June | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 17.0 | 17.0 | 17.0 | 18.0 | 18.0 | 17.0 | 18.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 19.0 | 18.0 | 19.0 | 18.0 | 17.0 | 17.0 | 18.0 |
| July | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 |
| August | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 17.0 | 17.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 18.0 |
| September | 16.0 | 16.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 18.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 16.0 | 16.0 | 16.0 |
| October | 16.0 | 16.0 | 16.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 17.0 | 17.0 | 17.0 | 18.0 | 18.0 | 14.0 | 15.0 | 16.0 | 16.0 |
| November | 15.0 | 16.0 | 16.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 18.0 | 18.0 | 19.0 | 18.0 | 17.0 | 17.0 | 17.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 15.0 | 15.0 |
| December | 16.0 | 17.0 | 16.0 | 17.0 | 17.0 | 18.0 | 17.0 | 18.0 | 18.0 | 17.0 | 18.0 | 17.0 | 19.0 | 18.0 | 17.0 | 13.0 | 16.0 | 16.0 | 17.0 | 16.0 | 17.0 | 16.0 | 16.0 | 16.0 | 16.0 |

## 10.2 	Result of study 2

## 10.3 	Result of study 3

### 10.3.1	Result of co-band interference

The percentages of time that I/Ns exceed –6 dB at each fixed/land mobile station in the grid for each of the 7 amateur stations are shown in Figure 7 to Figure 20 respectively.

Figure 7

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Beijing, rural, SSN=100

Figure 8

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Beijing, quiet rural, SSN=100

Figure 9

Editors note: Need to note that interference is potential

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Moscow, rural, SSN=100

Figure 10

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Moscow, quiet rural, SSN=100

Figure 11

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - London, rural, SSN=100

Figure 12

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - London, quiet rural, SSN=100

Figure 13

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Chicago, rural, SSN=100

Figure 14

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Chicago, quiet rural, SSN=100

Figure 15

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Lagos, rural, SSN=100

Figure 16

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Lagos, quiet rural, SSN=100

Figure 17

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Canberra, rural, SSN=100

Figure 18

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Canberra, quiet rural, SSN=100

Figure 19

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station – Buenos Aires, rural, SSN=100

Figure 20

Time ratio of I/Ns exceeding -6 dB at each fixed/land mobile station - Buenos Aires, quiet rural, SSN=100

The percentages of areas for which the interfered time exceeding 20% for each amateur station are given in Table 3 below.

TABLE 3

Percentages of interfered area around the globe for different amateur station site

| Amateur Station Site | Rural |  |  | Quiet Rural |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SSN=10 | SSN=50 | SSN=100 | SSN=10 | SSN=50 | SSN=100 |
| Beijing | 11.75% | 11.19% | 10.84% | 24.23% | 24.16% | 24.1% |
| Moscow | 12.73% | 11.17% | 10.04% | 21.31% | 18.11% | 15.32% |
| London | 10.68% | 9.04% | 8.01% | 16.38% | 14.21% | 12.35% |
| Chicago | 10.6% | 8.06% | 6.05% | 9.85% | 8.46% | 7.82% |
| Lagos | 4.07% | 4.07% | 4.03% | 4.86% | 4.79% | 4.67% |
| Canberra | 11.83% | 11.49% | 11.3% | 21.48% | 22.79% | 23.46% |
| Buenos Aires | 8.49% | 8.49% | 8.47% | 17.98% | 17.92% | 16.86% |

### 10.3.2	Result of adjacent band interference

The time ratios of adjacent band interference at each fixed/land mobile station in the grid caused by amateur station located in Beijing with different frequency separations are shown in Figure 21 to Figure 24 in quiet rural environment.

Figure 21

Time ratio of adjacent band interference at each fixed/land mobile station - separated by one 3 kHz channel

Figure 22

Time ratio of adjacent band interference at each fixed/land mobile station - separated by two 3 kHz channels

Figure 23

Time ratio of adjacent band interference at each fixed/land mobile station - separated by three 3 kHz channels

Figure 24

Time ratio of adjacent band interference at each fixed/land mobile station - separated by four 3 kHz channels

# 11	 Analysis of the results

As shown in Table 3, all the percentage figures of interfered areas around the globe for each amateur station at different SSN will exceed 4%, which implies that potential amateur stations operating in 5 250 kHz to 5 450 kHz will cause interference to fixed/land mobile service stations in many areas around the globe.

And from figures above, it is shown that, on a co-frequency basis, the most severely impacted region (that the time ratio of interference exceeding 30%) is up to 2 000 kilometers away from the amateur stations in rural environment and up to 3 000 kilometers in quiet rural environment.

Even when the frequency separation between the amateur station and the fixed/land mobile service stations is three 3 kHz channels, the severely impacted area is approximately up to 1 000 kilometers away from the amateur station in quiet rural environment.

However, when the frequency separation between the amateur station and the fixed/land mobile service stations is beyond four 3 kHz channels, the impact caused by the amateur station will decrease evidently.

## 11.1	Regulatory and operational techniques to mitigate interference and enhance compatibility

Some administrations have accommodated amateur service operations in segments of or in discrete channels within the 5 250-5 450 kHz frequency range, under the provisions of No. 4.4 of the Radio Regulations.  For example, the United States permits amateurs to operate on five discrete channels within this range, with a maximum effective radiated power of 100 watts, and an authorized maximum bandwidth of 2.8 kHz. Operation utilizing modes less than the authorized bandwidth is confined to the center of the channel.  Interference to incumbent services by amateur stations under these conditions generally does not preclude operation in the incumbent services and is generally resolvable on a case by case basis when it does occur.

## 11.2	Analysis of Study 1 results

Analysis of Study 1 results shows that providing for protection of single-hop FS links would require separation distances exceeding 2 000 km. Providing protection for multihop FS links would require separation distances exceeding 6 000 km. Based on the estimates a conclusion could be drawn that it would be extremely difficult to provide for compatibility of potential amateur transmitters with incumbent FS links in the frequency band considered. Editor’s note: Analysis of the results e.g. link availability with & without interference from the amateur service, impacts of antenna types & Smoothed Sunspot Numbers etc. Hopefully with some statistical or probability analysis of likely impact and some indication of uncertainties associated with the propagation models and any other relevant calculations.

# 12	Conclusion

It’s complicated…

Editor’s note: no doubt there will be multiple sections here…

[Due to active global usage of FS links in the frequency band 5 240-5 450 kHz including inter alia for public protection and disaster relief operations and due to extreme difficulty of providing for compatibility of amateur stations with incumbent FS links it would be unfeasible to allocate additional spectrum resource to the amateur service in of the frequency band considered. The amateur service could use the frequency range subject to provisions of RR No. 4.4.

It can be concluded  from the studies above that emissions from possible amateur stations operating in the frequency band 5 250-5 450 kHz will interfere with incumbent fixed/land mobile service stations in many areas around the globe.

However, frequency separations between the amateur stations and the fixed/land mobile service stations will help to reduce the interference.

Therefore, in order to protect incumbent fixed/land mobile service stations, appropriate interference mitigation measures such as frequency separation, power limitation, and transmitting time limitation, etc., should be taken and need further study.

Editors note: Was section 8 in the Canadian study.

An analysis has been performed utilizing Recommendation ITU-R P.533-11 HF propagation prediction method to calculate the signal, noise and interference levels for the incumbent service and proposed amateur service at 5 300 kHz. This report demonstrates that an amateur service link operating under a secondary allocation would present no serious risk of interference to operations by existing Primary Service users.]

## Attachments:

Editor’s note: Each input contribution will be attached or embedded here as annexes to this study:

| 5A/242 (Canada) | 5A/250 (USA) | 5A/254 (Canada) |
| --- | --- | --- |
| 5A/280 (China) | 5A/289 (UK, Netherlands, Norway) | 5A/237 (Russian Federation) |
| 5A/253 (Canada) | Include modified 5A/400, 5A/383, 5A/368. |  |

____________________

Report of the Conference Preparatory Meeting to the 2012 World Radiocommunication Conference § 2/1.15/3.

National Telecommunications and Information Administration, Institute for Telecommunication Sciences, USA.

REC533 software implementation of Recommendation ITU-R P.533 is available for public download from the NTIA Institute for Telecommunication Sciences ITS website at the following address: http://elbert.its.bldrdoc.gov/hf.html.

The SNR value of 14 dB is a conservative level due to the fact the intensity fluctuation factor is not included for this performance level as recommended in ITU-R F.339-8.

The parameters in this Annex are shown in the same format as in the REC533 program.