| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5D/TEMP/728 (Rev.2) | Document IMT-2020/85-E |
|  | 26 October 2022 |
|  | English only |
| Working Party 5D |  |
| SUMMARY OF STEP 4 OF THE IMT-2020 PROCESS FOR EVALUATION OF IMT-2020 CANDIDATE TECHNOLOGY SUBMISSIONS IMT-2020/76 |  |

This document provides composite summary tables of the detailed information based on the final evaluation reports provided to ITU-R under Step 4 (evaluation) of the IMT-2020 process provided in Documents IMT-2020/2(Rev.2), IMT-2020/57 and IMT-2020/58(Rev.1) related to evaluations on the new IMT-2020 candidate technology for ‘after year 2021’ based on Reports ITU-R M.2410, ITU-R M.2411 and ITU-R M.2412. The information in this summary is based upon the documents indicated in Table 1.

Table 1

Index of documents related to IEG Final Evaluation Reports for the new Candidate Technology Submissions 
of IMT-2020 (IMT-2020/76) for ‘after year 2021’ Under Step 4

| IMT-2020/xx | Summary of Step 4 of the IMT-2020 Process for Evaluation of new IMT-2020 Candidate Technology Submission for ‘after year 2021’ |  |  |
| --- | --- | --- | --- |
| Registered Independent Evaluation Group | Summary of IEG Evaluation Results | Based on or References IEG Contributions Document | Evaluation Reports History Documents |
| 5GIF Independent Evaluation Group (5GIF-IEG) | IMT-2020/81 | 5D/1462 | IMT-2020/79 |
| 5GMF IMT-2020 Evaluation Group | IMT-2020/82 | 5D/1412 | IMT-2020/77(Rev.1) |
| Beijing National Research Center for Information Science and Technology (BNRist EG) | IMT-2020/83 | 5D/1529 | IMT-2020/80 |
| Wireless World Research Forum | IMT-2020/84 | 5D/1492 | IMT-2020/78(Rev.1) |

Summary composite tables by candidate technology submission

The tables in this section were compiled to reflect to the extent possible in a uniform manner the information indicated in the evaluation.

Candidate Technology – EUHT-5G RIT (IMT-2020/76)

Sources

| 5GIF | 5GMF | Bnrist | WWRF |
| --- | --- | --- | --- |
| 5D/1462 (#42) | 5D/1251(#41) 5D/1412(#42) | 5D/1529 (#42) | 5D/1283 (#41) 5D/1492 (#42) |

#### 5.2.4.1 	Compliance template for services

TABLE B.1

|  | Service capability requirements | 5GIF | 5GMF | Bnrist | WWRF |
| --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | NO (RIT does not support eMBB, URLLC, mMTC services) | NO (RIT cannot support the usage scenarios of eMBB and URLLC) | Yes | No |

#### 5.2.4.2 	Compliance template for spectrum

TABLE B.2

|  | Spectrum capability requirements | 5GIF | 5GMF | Bnrist | WWRF |
| --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. | Yes | Yes | Yes | Yes |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. | NO (Unable to determine from the EUHT Specification, if both STA and CAP can communicate using the mmWave band) | Yes | Yes | Yes |

#### 5.2.4.3 	Compliance template for technical performance

TABLE B.3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) | Category |  |  | Required value | 5GIF | 5GMF | Bnrist | WWRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | Downlink or uplink |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) | eMBB | Not applicable | Downlink | 20 | No |  | Yes |  |
|  |  |  | Uplink | 10 | No |  | Yes |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | eMBB | Not applicable | Downlink | 30 |  |  | Yes |  |
|  |  |  | Uplink | 15 |  |  | Yes |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) | eMBB | Dense Urban – eMBB | Downlink | 100 |  |  | Yes (Cfg A/B/C) |  |
|  |  |  | Uplink | 50 | No (Cfg A/B) |  | Yes (Cfg A/B/C) |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) | eMBB | Indoor Hotspot – eMBB | Downlink | 0.3 |  | No (Cfg A/B/C) | Yes (Cfg A/B Cfg A/B) |  |
|  |  |  | Uplink | 0.21 | No (Cfg A) | No (Cfg A/B/C) | Yes (Cfg A/B) |  |
|  | eMBB | Dense Urban – eMBB | Downlink | 0.225 |  | No (Cfg A/B) | Yes (Cfg A/B) |  |
|  |  |  | Uplink | 0.15 | No (Cfg A/B) | No (Cfg A/B) | Yes (Cfg A/B) |  |
|  | eMBB | Rural – eMBB | Downlink | 0.12 |  |  | Yes (Cfg B/C) |  |
|  |  |  | Uplink | 0.045 | No (Cfg A/B) |  | Yes (Cfg B/C) |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) | eMBB | Indoor Hotspot – eMBB | Downlink | 9 |  | No (Cfg A/B/C) | Yes (Cfg A/B) |  |
|  |  |  | Uplink | 6.75 | No (Cfg A) | No (Cfg A/B/C) | Yes (Cfg A/B) |  |
|  | eMBB | Dense Urban – eMBB | Downlink | 7.8 |  | No (Cfg A/B) | Yes (Cfg A/B) |  |
|  |  |  | Uplink | 5.4 | No (Cfg A/B) | No (Cfg A/B) | Yes (Cfg A/B) |  |
|  | eMBB | Rural – eMBB | Downlink | 3.3 |  |  | Yes (Cfg B/C) |  |
|  |  |  | Uplink | 1.6 | Yes (Cfg A/B) |  | Yes (Cfg B/C) |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) | eMBB | Indoor-Hotspot – eMBB | Downlink | 10 |  |  | Yes (Cfg A/B) |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) | eMBB | Not applicable | Uplink and Downlink | 4 |  |  | Yes |  |
|  | URLLC | Not applicable | Uplink and Downlink | 1 |  |  | Yes |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) | eMBB | Not applicable | Not applicable | 20 |  |  | Yes |  |
|  | URLLC | Not applicable | Not applicable | 20 |  |  | Yes |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) | mMTC | Urban Macro – mMTC | Uplink | 1 000 000 | No (Cfg A/B) |  | Yes (Cfg A/B) | No (Cfg A) |
| 5.2.4.3.10 Energy efficiency (4.9) | eMBB | Not applicable | Not applicable | Capability to support a high sleep ratio and long sleep duration |  |  | Yes |  |
| 5.2.4.3.11 Reliability (4.10) | URLLC | Urban Macro – URLLC | Uplink or Downlink | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge | No (Cfg A) | No (Cfg A/B) | Yes (Cfg A) | No (Cfg A) |
| 5.2.4.3.12 Mobility classes (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | Stationary, Pedestrian |  |  | Yes |  |
|  | eMBB | Dense Urban – eMBB | Uplink | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  | Yes |  |
|  | eMBB | Rural – eMBB | Uplink | Pedestrian, Vehicular, High speed vehicular |  |  | Yes |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | 1.5 (10 km/h) |  |  | Yes (Cfg A/B) |  |
|  | eMBB | Dense Urban – eMBB | Uplink | 1.12 (30 km/h) | No (Cfg A) |  | Yes (Cfg A/B) |  |
|  | eMBB | Rural – eMBB | Uplink | 0.8 (120 km/h) |  |  | Yes (Cfg A/B) |  |
|  |  |  |  | 0.45 (500 km/h) |  |  | Yes (Cfg A/B) |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) | eMBB and URLLC | Not applicable | Not applicable | 0 |  |  | Yes |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) | Not applicable | Not applicable | Not applicable | At least 100 MHz | No |  | Yes | Yes |
|  |  |  |  | Up to 1 GHz | No |  | Yes | Yes |
|  |  |  |  | Support of multiple different bandwidth values(4) | Yes |  | Yes | Yes |

______________

____________________