| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5D/TEMP/204 | Document IMT-2020/96-E |
|  | 18 February 2025 |
|  | English only |
| Working Party 5D |  |
| SUMMARY OF STEP 4 OF THE IMT-2020 PROCESS FOR EVALUATION OF IMT-2020 CANDIDATE TECHNOLOGY SUBMISSIONS IMT-2020/89 |  |

Introduction

This document provides composite summary tables of the detailed information based on the final evaluation reports provided to ITU-R under Step 4 (evaluation) of the IMT-2020 process provided in Documents IMT-2020/2(Rev.2), IMT-2020/57 and IMT-2020/87(Rev.1) related to evaluations on the new IMT-2020 candidate technology for Revision 3 based on Reports ITU-R M.2410, ITU-R M.2411 and ITU-R M.2412. The information in this summary is based upon the documents indicated in Table 1.

Table 1

Index of documents related to IEG Final Evaluation Reports for the new Candidate Technology Submissions 
of IMT-2020(IMT-2020/89) for Revision 3 under Step 4

| IMT-2020/xx | Summary of Step 4 of the IMT-2020 Process for Evaluation of new IMT-2020 Candidate Technology Submission for Revision 3 |  |  |
| --- | --- | --- | --- |
| Registered Independent Evaluation Group | Summary of IEG Evaluation Results | Based on or References IEG Contributions Docs. 5D/ | Evaluation Reports History Documents |
| ARIB IMT-2020 Evaluation Group (ARIB IEG) | IMT-2020/93 | 5D/457 | IMT-2020/90(Rev.1) |
| 5G India Forum (5GIF) | IMT-2020/94 | 5D/554 | IMT-2020/91(Rev.1) |
| Wireless World Research Forum (WWRF) | IMT-2020/95 | 5D/491 | IMT-2020/92 |

Summary composite tables by candidate technology submission

The tables in this section were compiled to reflect to the extent possible in a uniform manner the information indicated in the evaluation.

Candidate Technology – RIT submission from Nufront (IMT-2020/89)

Sources

| ARIB IEG | 5GIF | WWRF |
| --- | --- | --- |
| 5D/352 (#47) 5D/457 (#48) | 5D/391 (#47) 5D/554 (#48) | 5D/491 (#48) |

#### 5.2.4.1 	Compliance template for services

TABLE B.1

|  | Service capability requirements | ARIB IEG | 5GIF | WWRF |
| --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)? YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support. | NO (RIT cannot support the usage scenarios of eMBB, URLLC and mMTC) | NO (RIT cannot support the usage scenarios of eMBB, URLLC and mMTC) | NO (RIT cannot support the usage scenarios of eMBB and mMTC) |

#### 5.2.4.2 	Compliance template for spectrum

TABLE B.2

|  | Spectrum capability requirements | ARIB IEG | 5GIF | WWRF |
| --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations? YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. | Yes | Yes |  |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz? YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. | Yes | Yes |  |

#### 5.2.4.3 	Compliance template for technical performance

TABLE B.3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference | Category |  |  | Required value | ARIB IEG | 5GIF | WWRF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | Downlink or uplink |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) | eMBB | Not applicable | Downlink | 20 |  |  |  |
|  |  |  | Uplink | 10 |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | eMBB | Not applicable | Downlink | 30 |  |  |  |
|  |  |  | Uplink | 15 |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) | eMBB | Dense Urban – eMBB | Downlink | 100 |  |  |  |
|  |  |  | Uplink | 50 |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) | eMBB | Indoor Hotspot – eMBB | Downlink | 0.3 | No (Cfg A,B,C) |  | No (Cfg A) |
|  |  |  | Uplink | 0.21 | No (Cfg A,B,C) | No (Cfg A) | No (Cfg A) |
|  | eMBB | Dense Urban – eMBB | Downlink | 0.225 | No (Cfg A, B) |  | No (Cfg A) |
|  |  |  | Uplink | 0.15 | No (Cfg A, B) | No (Cfg A,B) | No (Cfg A) |
|  | eMBB | Rural – eMBB | Downlink | 0.12 |  |  |  |
|  |  |  | Uplink | 0.045 |  | No (Cfg A,B) |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) | eMBB | Indoor Hotspot – eMBB | Downlink | 9 | Yes (Cfg A) No (Cfg B,C) |  | No (Cfg A) |
|  |  |  | Uplink | 6.75 | No (Cfg A,B,C) | No (Cfg A) | No (Cfg A) |
|  | eMBB | Dense Urban – eMBB | Downlink | 7.8 | Yes (Cfg A) No (Cfg B) |  | No (Cfg A) |
|  |  |  | Uplink | 5.4 | No (Cfg A,B) | No (Cfg A,B) | No (Cfg A) |
|  | eMBB | Rural – eMBB | Downlink | 3.3 |  |  |  |
|  |  |  | Uplink | 1.6 |  | Yes (Cfg A,B) |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) | eMBB | Indoor-Hotspot – eMBB | Downlink | 10 |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) | eMBB | Not applicable | Uplink and Downlink | 4 |  |  |  |
|  | URLLC | Not applicable | Uplink and Downlink | 1 |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) | eMBB | Not applicable | Not applicable | 20 |  |  |  |
|  | URLLC | Not applicable | Not applicable | 20 |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) | mMTC | Urban Macro – mMTC | Uplink | 1 000 000 | No (Cfg A,B) | No (Cfg A,B) | No (Cfg A) |
| 5.2.4.3.10 Energy efficiency (4.9) | eMBB | Not applicable | Not applicable | Capability to support a high sleep ratio and long sleep duration |  |  |  |
| 5.2.4.3.11 Reliability (4.10) | URLLC | Urban Macro –URLLC | Uplink or Downlink | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge | No (Cfg A,B) | No (Cfg A) |  |
| 5.2.4.3.12 Mobility classes (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | Stationary, Pedestrian |  |  |  |
|  | eMBB | Dense Urban – eMBB | Uplink | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  |  |
|  | eMBB | Rural – eMBB | Uplink | Pedestrian, Vehicular, High speed vehicular |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | 1.5 (10 km/h) |  |  |  |
|  | eMBB | Dense Urban – eMBB | Uplink | 1.12 (30 km/h) |  |  |  |
|  | eMBB | Rural – eMBB | Uplink | 0.8 (120 km/h) |  |  |  |
|  |  |  |  | 0.45 (500 km/h) |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) | eMBB and URLLC | Not applicable | Not applicable | 0 |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) | Not applicable | Not applicable | Not applicable | At least 100 MHz |  |  |  |
|  |  |  |  | Up to 1 GHz |  |  |  |
|  |  |  |  | Support of multiple different bandwidth values |  |  |  |

________________

____________________

If a proponent determines that a specific question does not apply, the proponent should indicate that this is the case and provide a rationale for why it does not apply.