|  | Radiocommunication Study Groups |  |  |
| --- | --- | --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| Source: Document 5A/TEMP/120(Rev.1) |  | Annex 27 to Document 5A/298-E |  |
|  |  | 18 November 2016 |  |
|  |  | English only |  |
| Annex 27 to Working Party 5A Chairman’s Report |  |  |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLAN SHARING] |  |  |  |
| Sharing and compatibility studies of WAS/RLAN in the 5 GHz frequency range |  |  |  |

[Editor’s Note: This document is a compilation of material presented in contributions submitted to May and November, 2016 WP 5A meetings (see Source Document references above and Source Indication below) that the submitting administrations requested to be considered in developing this document. The content of this document has been presented but has not been discussed and is not agreed by WP 5A and needs to be reviewed at next WP 5A meeting in May 2017 with a view to consolidating its content.]

[Editor’s note: Document 5A/210 presents information about the current “U.S. rules” for RLANs in the 5 GHz range for information and futher consideration.

]

[RUS]

# 1	Introduction

This Report includes the sharing and compatibilities studies of WAS/RLAN in the 5 GHz frequency range.

It is intended to represent the response to invites ITU-R c), d), e) and f) of Resolution 239 (WRC-15) under WRC-19 agenda item 1.16.

# 2	Scope of the sharing and compatibility of WAS/RLAN with other services in the 5 GHz range.

The World Radiocommunications Conference 2015 decided on the draft agenda for the upcoming World Radiocommunications Conference scheduled for 2019. Among other items, WRC-19 agenda item 1.16 addresses the need of studies on regulatory actions and additional spectrum allocations to be mobile service, including radio local area networks (WAS/RLAN). Indeed, WRC-19 agenda item 1.16 reads:

1.16	to consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15).

The related Resolution 239 (WRC-15) to the WRC-19 agenda item 1.16 deals with studies concerning Wireless Access Systems including radio local area networks in the frequency bands between 5 150 MHz and 5 925 MHz. The Resolution invites ITU-R to conduct and complete the following in time for WRC-19:

a)	to study WAS/RLAN technical characteristics and operational requirements in the 5 GHz frequency range;

b)	to conduct studies with a view to identify potential WAS/RLAN mitigation techniques to facilitate sharing with incumbent systems in the frequency bands 5 150-5 350 MHz, 5 350-5 470 MHz, 5 725-5 850 MHz and 5 850-5 925 MHz, while ensuring the protection of incumbent services including their current and planned use;

c)	to perform sharing and compatibility studies between WAS/RLAN applications and incumbent services in the frequency band 5 150-5 350 MHz with the possibility of enabling outdoor WAS/RLAN operations including possible associated conditions;

d)	to conduct further sharing and compatibility studies between WAS/RLAN applications and incumbent services addressing:

i)	whether any additional mitigation techniques in the frequency band 5 350-5 470 MHz beyond those analysed in the studies referred to in recognizing a) would provide coexistence between WAS/RLAN systems and EESS (active) and SRS (active) systems;

ii)	whether any mitigation techniques in the frequency band 5 350-5 470 MHz would provide compatibility between WAS/RLAN systems and radio determination systems;

iii)	whether the results of studies under points i) and ii) would enable an allocation of the frequency band 5 350-5 470 MHz to the mobile service with a view to accommodating WAS/RLAN use;

e)	to also conduct detailed sharing and compatibility studies, including mitigation techniques, between WAS/RLAN and incumbent services in the frequency band 5 725-5 850 MHz with a view to enabling a mobile service allocation to accommodate WAS/RLAN use;

f)	to also conduct detailed sharing and compatibility studies, including mitigation techniques, between WAS/RLAN and incumbent services in the frequency band 5 850-5 925 MHz with a view to accommodating WAS/RLAN use under the existing primary mobile service allocation while not imposing any additional constraints on the existing services,

# 3	Overall view of allocations in the 5 GHz range

| Allocation to services |  |  |  |  | Expected studies |  |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |  |  |
| 5 150-5 250 FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile 5.446A 5.446B AERONAUTICAL RADIONAVIGATION 5.446 5.446C 5.447 5.447B 5.447C |  |  |  |  | Coexistence between WAS/RLAN outdoor operations and FSS (feederlinks for non-GSO) and Aeronautical Radionavigation |  |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH 5.447D 5.447E 5.448 5.448A |  |  |  |  | Coexistence between WAS/RLAN outdoor operations and EESS (active), Radiolocation and SRS (active) |  |
| 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH (active) 5.447E 5.448 5.448A |  |  |  |  |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B RADIOLOCATION 5.448D AERONAUTICAL RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448C |  |  |  |  | Further sharing and compatibility studies between WAS/RLAN applications and incumbent services addressing whether additional mitigation techniques would provide coexistence between WAS/RLAN systems and EESS (active), radio determination and SRS (active) systems (see invites ITU-R d) of Res. 239) |
| 5 460-5 470 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION 5.448D RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448B |  |  |  |  |  |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur |  | 5 725-5 830 RADIOLOCATION Amateur |  |  | Coexistence between WAS/RLAN and FSS and Radiolocation |
| 5.150 5.451 5.453 5.455 5.456 |  | 5.150 5.453 5.455 |  |  |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |  |  |
| 5.150 5.451 5.453 5.455 5.456 |  | 5.150 5.453 5.455 |  |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation | Coexistence between WAS/RLAN under the current MS allocation and FS and FSS. |
| 5.150 |  | 5.150 |  | 5.150 |  |

# 4	Assumptions on technical and operational elements for the sharing and compatibility of WAS/RLAN with other services

## 4.1	Technical and operational characteristics of the WAS/RLAN operating in the 5 GHz ranges

Option 1

[RUS]

[Editor’ Note: the text below need to be verified after finalization of the document ITU-R Report M.[RLAN REQ-PAR]]

Technical and operational characteristics of RLANs are presented in Recommendation ITU-R M.1450 «Characteristics of broadband radio local area networks». In accordance with this Recommendation in the territory of USA and Canada e.i.r.p. of RLANs operating in the frequency bands 5 150–5 250 MHz and 5 250–5 350  MHz is 250 mW (-6 dBW) and 1 000 mW (0 dBW) in the frequency band 5 750-5 850 MHz. At the same time RLANs operating in the territory of Europe, and in numerous Region 3 countries including Australia,  are restricted to an e.i.r.p. of 200 mW (-7 dBW) in the frequency bands 5 150-5 250 MHz and 5 250-5 350 MHz and, in the lower 5 150-5 250 MHz segment, to indoor only operation.

At the same time, under preparation for WRC-15 agenda item 1.1,  the issue of the operational feasibility of RLANs in the frequency band 5 350-5 470 MHz was considered. It was assumed that the RLAN transmitter e.i.r.p. would be 200 mW (-7 dBW) in this frequency band.

Therefore two e.i.r.p. values were used for compatibility assessment of RLANs with radiodetermination radars in the frequency bands 5 350-5 460 MHz, 5 460-5 470 MHz and 7 250-7 850 MHz:

–	e.i.r.p. of minus 7 dBW was assumed for RLAN operating in the frequency bands 5 350-5 470 MHz;

–	e.i.r.p. of zero (0) dBW was assumed for RLAN operating in the frequency bands 5 725-5 850 MHz;

e.i.r.p. spectral densities specified in Recommendation ITU-R М.1450 shows that it addresses RLANs having carrier bandwidth of 20 MHz. However taking in account the achievements in RLANs development the considered Report includes analysis of networks having carrier bandwidth of both 20 MHz and 160 MHz.

[UK and ESA]

Option 2

### 4.1.1	Characteristics of RLAN in 5 150-5 250 MHz Band

### 4.1.2	Characteristics of RLAN in 5 250-5 350 MHz Band

### 4.1.3	Characteristics of RLAN in 5 350-5 470 MHz Band

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

4.1.4	Characteristics of RLAN in 5 725-5 850 MHz band

Table 4.1.4-1

Basic RLAN (Wi-Fi) transmitter characteristics in the band 5 725-5 850 MHz

|  | RLAN 1 Omni-Indoor | RLAN 2 Omni Outdoor | RLAN 3 Directional Outdoor |
| --- | --- | --- | --- |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 | 30 | 30 |
| Bandwidth (MHz) | 20/40/80/160 | 20/40/80/160 | 20/40/80/160 |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 | 10/7/4/1 | 10/7/4/1 |
| Typical AP Antenna Type | Omni (azimuth) See Table 4.7.4-8, Table 4.9.4-10 and Table 4.11.4-12 Type 1 and 2 | Omni (azimuth) See Table 4.11.4-12 , Type 1 and 2 | Directional, See Table 4.11.4-12, Type 3 and 4 |
| AP Antenna directivity gain (dBi) | 0-6 | 6-7 | 12/18 |

The figure below provides the spectrum mask for RLAN as function of the nominal channel bandwidth, typically 20, 40, 80 or 160 MHz

Figure 4.2.4-1

Spectrum mask for RLAN

The assumed average channel bandwidths distribution of RLAN devices is given in the following table.

Table 4.3.4-4

RLAN channel bandwidth distribution

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |

The next table provides RLAN receiver parameters for the purpose of compatibility studies with RLAN as a victim.

Table 4.5.4-6

Basic RLAN receiver characteristics in the band 5725-5850 MHz

| System parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Bandwidth (MHz) | 20 | 40 | 80 | 160 |
| kTB dBm / bandwidth | -101 | -98 | -95 | -92 |
| Typical Noise figure dB | 4 |  |  |  |
| Noise Power (dBm / bandwidth) | -97 | -94 | -91 | -88 |
| Typical Sensitivity for MCS0, BPSK (½ coding rate) (dBm) | -92 | -89 | -86 | -83 |
| C/N for MCS0, BPSK (½ coding rate) (dB) | 5 |  |  |  |
| I/N (dB) (Note 1) | -6 |  |  |  |
| C/I (dB) | 11 for I/N -6 dB; 5 for I/N 0 dB |  |  |  |
| Maximum antenna gain at the RLAN Access Point (dBi) | See Table 4.9.4-10 and Table 4.11.4-12 |  |  |  |
| Maximum antenna gain at the RLAN user device (dBi) | See Table 4.7.4-8 |  |  |  |
| Note 1: As per Recommendation ITU-R M.1739, the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB. Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis. |  |  |  |  |

RLAN antenna patterns

The characteristics in Table 4.7.4-8 are representative of an average antenna for all User Equipment within a population of RLAN devices. User Equipment can be defined as mobile or portable devices such as smart phones, tablets, notebooks, wireless scanners etc.

Table 4.7.4-8

RLAN User Equipment antenna (mobile/portable device)

| # | Type | Gain (dBi) | Antenna height above ground (m) |
| --- | --- | --- | --- |
| 1 | Omni-directional Antenna | 1.3 | 1 to 1.5 |
| NOTE: This value is the averaged value obtained from a survey on RLAN UE antennas. For simplicity, this antenna is assumed to be isotropic. |  |  |  |

The antenna pattern in Table 4.9.4-10 is considered as a representative average antenna pattern for indoor access points within the RLAN population. The table specifies the gains available at elevation angles; the antenna pattern is omni-directional in azimuth.

Table 4.9.4-10

Example of Indoor RLAN Access Point Omni-directional (azimuth) Antenna - 
Elevation Pattern

| Elevation angle θ (Degrees) | Gain (dBi) |
| --- | --- |
| 45 θ 90 | -4 |
| 35 θ 45 | 0 |
| 0 θ 35 | 3 |
| –15 θ 0 | -1 |
| –30 θ –15 | -4 |
| –60 θ –30 | -9 |
| –90 θ –60 | -8 |

The elevation angles in Table 4.9.4-10 are defined from the viewpoint of the RLAN Access Point when mounted to the ceiling. Positive elevation angles are towards the ground and negative elevation angles are towards the sky (typically, the RLAN AP is installed for optimal coverage). The pattern is normalised to 3 dBi gain on boresight.

Table 4.11.4-12 sets out the characteristics of RLAN antennas used on fixed indoor or outdoor equipment such as Access Points, Bridges, P2P or P2MP installations. The corresponding antenna patterns are provided in Figure 4.13.4-1 to Figure 4.16.4-4 below.

Table 4.11.4-12

Typical Fixed indoor and outdoor RLAN antenna 
(access points, bridges, P2P and P2MP)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 6 | Indoor & Outdoor | Figure 4.13.4-1 | 6 to 28,5 |
| 2 | Directional Antenna (sector) | 6 | Indoor & Outdoor | Figure 4.14.4-2 |  |
| 3 | Directional Antenna | 12 | Outdoor | Figure 4.15.4-3 |  |
| 4 | Directional Antenna (sector) | 17 | Outdoor | Figure 4.16.4-4 |  |
| NOTE: The (Highly) directional links are often installed on top of buildings |  |  |  |  |  |

Figure 4.13.4-1

RLAN 6 dBi Omni – Elevation (left) and Azimuth (right) Radiation Patterns

Figure 4.14.4-2

RLAN 6 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure 4.15.4-3

RLAN 12 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure 4.16.4-4

RLAN 17 dBi Sector Antenna – Elevation (blue) and Azimuth (red) Radiation Patterns

RLAN power distribution

Table 4.17.4-18

RLAN power distribution

| Tx power e.i.r.p. | 1W (directional) | 1 W (omni) | 200mW (omni) | 80mW (omni) | 50mW (omni) | 25mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

The RLAN power distribution presented here leads to a 5.3 % use of outdoor devices. Sensitivity analysis may be performed with other outdoor use ratio to assess the impact of this parameter on the compatibility studies. Outdoor use ratios from 2-5 % is suggested.

### 4.1.4	Characteristics of RLAN in 5 850-5 925 MHz band

[ESA/EUMETSAT]

RLAN parameters used in the studies with EESS are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)). They are also reiterated in the current working document towards a preliminary draft Report ITU-R M.[RLAN REQ-PAR] and concern:

- Eirp distributions

- Indoor/outdoor ratio

- Channel Bandwidth distribution

- Propagation conditions

- Antenna gain/discrimination

- Number of active RLAN.

## 4.2	Technical and operational characteristics of FSS links used for MSS feeder links in the 5 150-5 250 MHz

## 4.3	Technical and operational characteristics of the Aeronautical Radionavigation service operating in the 5 150-5 250 MHz and 5 350-5 460 MHz

4.3.1	5 150-5 250 MHz band

4.3.2	5 350-5 460 MHz band

[RUS]

Recommendation ITU-R M.1638-1 contains description of one radar of the aeronautical radionavigation service operating in the frequency band 5 350-5 460 MHz (Radar 16). Its technical characteristics and protection criterion are given in Table 4.1 The protection criterion I/N is of minus 10 dB as specified in the Table and it corresponds to Section 4 “Protection criteria” of Recommendation ITU-R М.1638-1.

TABLE 4.1

Technical characteristics and protection criteria of radars in the aeronautical radionavigation service 
operating in the frequency bands 5350-5460 MHz

| Radar | Radar 16 |
| --- | --- |
| Location | Airborne |
| Frequency band, MHz | 5 440 |
| Antenna gain, dB | 34 |
| Noise figure, dB | 5 |
| IF bandwidth, MHz | 1 |
| I/N, dB | -10 |
| Тn, К | 627 |
| Рnoise, add, dBW | -141 |
| Iadd, dBW | -151 |

These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using the following equations:

К	(1)

dBW	(2)

where:

k –	Boltzmann constant;

NF – 	radar receiver noise figure;

–	radar receiver IF operational pass-band.

Maximum permissible noise power at the front end was assessed such as:

, dBW	(3)

The estimated thermal noise level, noise power and permissible interference power are given in Table 4.1.

## 4.4	Technical and operational characteristics of the Earth Exploration Satellite service operating in the frequency ranges 5 250-5 570 MHz

[7C]

The band 5 250-5 570 MHz, allocated to EESS (active) on a primary basis, is currently used by many Administrations operating EESS (active) sensors and is planned to be used by a number of additional sensors. Typically, this band is used by following type of sensors:

–	Synthetic aperture radars (SAR) with operations typically limited to the 5 350-5 470 MHz band.

–	Altimeters with operations typically covering the whole 5 250-5 570 MHz band.

–	Scatterometers with small bandwidths and operations typically within the 5 250-5 350 MHz band or the 5 350-5 470 MHz band.

Studies under WRC-15 AI 1.1 mainly focused on SAR missions, however all types of EESS (active) sensors require relevant protection. Therefore, all mitigation techniques to be studied have to assess protection of all existing and planned SAR, altimeters and scatterometers sensors. Each sensor type has different technical characteristics.  Within each sensor type, however, the characteristics present similar modes of operation.

The table in Annex 1 provides a non-exhaustive listing of existing and planned EESS (active) systems known to date and Annex 2 provides detailed characteristics of some of these EESS (active) systems.

Prior studies included in Annex 35 to Document 4-5-6-7/715 (Chairman’s Report) depicted substantial negative margins for EESS (active) SAR systems, while additional studies are needed for altimeters and scatterometers. However, one can envision similar results as those found with SARs, since aggregate interference from RLANs to EESS (active) is mainly controlled by the sensors antenna gain and the number of active RLANs within the sensor footprint.  Altimeters and scatterometers present lower antenna gain but, by direct effect, larger footprint and hence higher number of active RLAN to be considered in the aggregation calculations.

In case more detailed dynamic studies with altimeters and scatterometers were found necessary, the relevant EESS (active) protection criteria to be used for these sensors (from Recommendation ITU-R RS.1166-4) are given below, together with the SAR interference criteria:

Table 2

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Scatterometer | 8% degradation in measurement of normalized radar backscatter to deduce wind speeds | –5 | 99 | 95 |
| Altimeter | 4% degradation in height noise | –3 | 99 | 95 |
| Synthetic Aperture Radar | 10% degradation of standard deviation of pixel power | -6 | 99 | 95 |

Note : Since interference is most likely to be produced by an aggregation of RLAN interferers, it would be related to population densities and interference would hence more than likely be systematic. The percentage of data availability to be used in the RLAN case is therefore 99%.

## 4.6	Technical and operational characteristics of the Radionavigation service operating in the 5 460-5 470 MHz

[UK]

Editor’s Note: Individual Radar highlighted in the tables below may operate across one of the sub bands in the 5GHz range or across more than one of these sub bands. In addition, some of the radar highlighted in the tables below may already covered by the existing mitigation techniques in Recommendation ITU-R M.1652-1. Future drafts should make the usage situation across the bands clearer and which radar are already covered by the existing mitigation techniques shown in Recommendation ITU-R M.1652-1

[RUS]

Recommendation ITU-R M.1638-1 contains description of two radars of the radionavigation service. The technical characteristics and protection criterion are given in Table 4.2. The protection criterion I/N is of minus 10 dB as specified in the Table and it corresponds to section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.2.

TABLE 4.2

Technical characteristics and protection criteria of radars in the radionavigation service 
operating in the frequency bands 5 460-5 470 MHZ

| Radar | Radar 10 | Radar 10A |
| --- | --- | --- |
| Location | Shipborne, ground | Ground (bistatic) |
| Frequency band, MHz | 5 250-5 875 | 5 250–5 875 |
| Antenna gain, dB | 33 | 33 |
| Noise figure, dB | 3 | 3 |
| IF bandwidth, MHz | 11.0 | 11.0 |
| I/N, dB | -10 | -10 |
| Тn, К | 289 | 289 |
| Рnoise, add, dBW | -134 | -134 |
| Iadd, dBW | -144 | -144 |

## 4.7	Technical and operational characteristics of the Radiolocation service operating in the 5 250-5 470/5 725-5 850 MHz

[UK]

Editor’s Note: Individual Radar highlighted in the tables below may operate across one of the sub bands in the 5GHz range or across more than one of these sub bands. In addition, some of the radar highlighted in the tables below may already covered by the existing mitigation techniques in Recommendation ITU-R M.1652-1. Future drafts should make the usage situation across the bands clearer and which radar are already covered by the existing mitigation techniques shown in Recommendation ITU-R M.1652-1

### 4.7.1	Technical characteristics of Radiolocation systems operating in 5 250-5 350 MHz band

### 4.7.2	Technical characteristics of Radiolocation systems operating in 5 350-5 470 MHz band

[RUS]

Recommendation ITU-R M.1638-1 contains description of 16 radars of the radiolocation service operating in the frequency bands 5 350-5 470/5 725-5 850 MHz. The technical characteristics and protection criterion are given in Table 4.3. The protection criterion I/N is of minus 6 dB as specified in the Table and it corresponds to section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.3.

TABLE 4.3

Technical characteristics and protection criteria of ground based radars in the radiolocation service 
 operating in separate frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

| Radar | Radar 2 | Radar 3 | Radar 4 | Radar 5 | Radar 7 | Radar 9 |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Ground | Ground | Ground | Ground | Ground, shipborne | Airborne |  |
| Frequency band, MHz | 5 350-5 850 | 5 350-6 850 | 5 400-5 900 | 5 400-5 900 | 5 450-5 825 | 5 250-5 725 |  |
| Antenna gain, dB | 54 | 47 | 45.9 | 42 | 30 | 40 |  |
| Noise figure, dB | 5 | 5 | 11 | 5 | 10 | 3.5 |  |
| IF bandwidth, MHz | 0.25 | 1.0 | 2.0 | 8.0 | 1.0 | 1 |  |
| I/N, dB | -6 | -6 | -6 | -6 | -6 | -6 |  |
| Тn, К | 627 | 627 | 3361 | 627 | 2610 | 359 |  |
| Рnoise, add, dBW | -147 | -141 | -130 | -132 | -134 | -143 |  |
| Iadd, dBW | -153 | -147 | -136 | -138 | -140 | -153 |  |
| Radar | Radar 12 | Radar 13 | Radar 15 | Radar 17 | Radar 18 | Radar 19 |
| Location | Ground, shipborne | Ground | Ground | Airborne | Ground | Ground |
| Frequency band, MHz | 5 400-5 900 | 5 450-5 850 | 5 400-5 850 | 5 370 | 5 600-5 650 | 5 300-5 700 |
| Antenna gain, dB | 25 | 43 | 42 | 37.5 | 38.5 | 44.5 |
| Noise figure, dB | 4 | 3 | 2.3 | 6 | 3 | 3 |
| IF bandwidth, MHz | 7.0 | 2.75 | 20 | 0.6 | 4 | 0.75 |
| I/N, dB | -6 | -6 | -6 | -10 | -6 | -6 |
| Тn, К | 438 | 289 | 202 | 865 | 289 | 289 |
| Рnoise, add, dBW | -134 | -140 | -133 | -141 | -138 | -145 |
| Iadd, dBW | -140 | -146 | -139 | -151 | -144 | -151 |
| Radar | Radar 20 | Radar 21 | Radar 22 | Radar 23 |  |  |
| Location | shipborne | Ground | Ground | Ground |  |  |
| Frequency band, MHz | 5 400-5 700 | 5 300-5 750 | 5 400-5 850 | 5 250-5 850 |  |  |
| Antenna gain, dB | 40 | 44.5 | 35 | 31.5 |  |  |
| Noise figure, dB | 2 | 3 | 5 | 13 |  |  |
| IF bandwidth, MHz | 0.5 | 0.8 | 4 | 5 |  |  |
| I/N, dB | -6 | -6 | -6 | -6 |  |  |
| Тn, К | 170 | 289 | 627 | 5496 |  |  |
| Рnoise, add, dBW | -149 | -145 | -135 | -124 |  |  |
| Iadd, dBW | -155 | -151 | -141 | -130 |  |  |

6 meteorological radars operating in the frequency band 5 350-5 470 MHz are given in Recommendation ITU-R М.1849-1. Their technical characteristics and protection criteria are reflected in Table 4.4. The analysis of the presented data shows that one meteorological radar –Radar 8 operates also in the portion of the frequency band 5 725-5 850 MHz (5 725-5 750 MHz). The protection criterion I/N is of minus 10 dB as specified in Table 4.4 and it corresponds to Section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.4.

TABLE 4.4

Technical characteristics and protection criteria of ground based meteorological radars

| Radar | Radar 1 | Radar 4 | Radar 8 | Radar 12 | Radar 13 | Radar 14 |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Ground |  |  |  |  |  |
| Frequency band, MHz | 5 300-5 700 | 5 300-5 700 | 5 250-5 750 | 5 330-5 370 | 5 250-5 370 | 5 430-5 470 |
| Antenna gain, dB | 39 | 40 | 45 | 45 | 50 | 45 |
| Noise figure, dB | 7 | 3 | 3 | 1,9 | 1 | 1,8 |
| IF bandwidth, MHz | 0,5 | 0,6 | 10 | 0,4 | 1,0 | 2,0 |
| I/N, dB | -10 | -10 | -10 | -10 | -10 | -10 |
| Тn, К | 1163 | 289 | 289 | 159 | 75 | 149 |
| Рnoise, add, dBW | -141 | -146 | -134 | -151 | -150 | -144 |
| Iadd, dBW | -151 | -156 | -144 | -161 | -160 | -154 |

### 4.7.2	Technical characteristics of Radiolocation systems operating in 5 725-5 850 MHz band

## 4.8	Technical and operational characteristics of the Fixed service operating in the 5 850-5 925 MHz

[TBD]

## 4.9	Technical and operational characteristics of the Fixed Satellite service operating in the 5 725-5 850 MHz (for Region 1) and 5 850-5 925 MHz

[WP 4A]

Note: The material in the table below is derived from contributions received by WP 4A at its September/October 2016 meeting.

Table 1

FSS Uplink Parameters (Interfered with)

| Frequency range | GHz | 5.725-5.925 | 5.725-5.925 |
| --- | --- | --- | --- |
| CARRIER | Carrier Name | Carrier #11 | Carrier #12 |
| Noise bandwidth | MHz | 4.0-20 | 4.0-20 |
| SPACE STATION |  |  |  |
| Peak receive antenna gain | dBi | 20 | 36.4 |
| Antenna receive gain pattern and beamwidth | – | Section 1.1 of Annex 1 of Rec. ITU-R S.672-4 LS=-25 | Section 1.1 of Annex 1 of Rec. ITU-R S.672-4 LS=-25 |
| System receive noise temperature | K | 400 | 400 |
| INTERFERENCE PROTECTION CRITERIA |  |  |  |
| Interference to Noise Ratio I/N | dB | [-12.2] | [-12.2] |
| Other |  |  |  |
| Additional Notes |  | - |  |

Table 2

FSS Uplink Parameters (Interferer)

| Frequency range | GHz | 5.925-5.925 | 5.925-5.925 |
| --- | --- | --- | --- |
| EARTH STATION CARRIER |  | Carrier #11 | Carrier #12 |
| Antenna diameter | m | 1.8 | 13.2 |
| Peak transmit antenna gain | dBi | 39.9 | 57.2 |
| Peak transmit power spectral density (clear sky) | dBW/Hz | -33 | -33 |
| Antenna gain pattern (ITU Recommendation) | – | Rec. ITU-R 465-6 | Rec. ITU-R 465-6 |
| Minimum elevation angle of transmit earth station | ° | 5 | 5 |
| Other |  |  |  |
| Additional Notes |  |  |  |

[UK]

4.9.1 	Technical characteristics of fixed satellite service in 5 725-5 850 MHz band

In the 125 MHz portion of the band up to 5 850 MHz, this is a Region 1 allocation only. FSS deployments use the whole band 5 725-5 850 MHz and it is used by transmitting earth stations in the Earth-to-space direction operating only to satellites in geostationary orbits.

The following table provides details of the selection of satellites that have been taken as representative of those requiring protection in the visible portion of the geostationary orbit from Europe. In these frequency bands, the satellite beams cover very large areas of the Earth (using global, hemispherical, zonal or regional beams) as can be seen by the satellite footprint coverage plots in Annex 6 of ECC Report 068‎[18].

Table A1-19

Sample Satellite Data for the band 5 725-5 850 MHz

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| D | 3o East | Whole band | 34 | 773 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |

Typical FSS parameters developed by the ITU are provided in Table below.

Table A1-20

Typical FSS parameters in the 6 GHz band

| Parameter | Typical value |
| --- | --- |
| Range of operating frequencies | 5 850-6 700 MHz |
| Antenna diameters (m) | 1.2, 1.8, 2.4, 3.0, 4.5, 8, 16, 32 |
| Antenna reference pattern | Recommendation ITU-R S.465 |
| Range of emission bandwidths | 40 kHz - 72 MHz |
| Receiving space system figure of merit | +5 ↔ -10 dB/K (The database of Recommendation ITU-R S.1328 provides one example with Gsat= 24.8 dBi and Ts= 400 K, corresponding to a G/T of -1.2 dB/K |
| Earth station deployment | All regions, in all locations (rural, semi-urban, urban) |
| Earth station e.i.r.p. density towards the horizon | In accordance with RR No. 21.8 and Recommendation ITU-R S.524-9 |
| Minimum earth station antenna elevation angle, h, (degrees) | 5, 15 and 40 |

4.9.2 	Technical characteristics of Fixed satellite service in 5 850-5 925 MHz band

[LUX]

Already covered in section 4.9 above.

[IARU]

## 4.10	Technical and operational characteristics of the Amateur Radio service operating in 5 650-5 850 MHz in Regions 1 and 3 and in 5 650-5 925 MHz in Region 2

[TBD]

## 4.11	Technical and operational characteristics of the Amateur Satellite service (space-to-Earth) operating in 5 830-5 850 MHz

[TBD]

# 5	Sharing studies per frequency range and per service

## 5.1	Sharing and compatibility of MSS feeder links versus WAS/RLANs in the 5 150-5 250 MHz band

[AUS]

The current, worldwide sharing rules, in the lower 5 150-5 250 MHz band, to protect co-band non-GSO MSS E-to-s feeder links, appear in Resolution 229 (Rev.WRC-12) which, inter alia, requires individual WAS/RLAN transmitters to “be restricted to indoor use with a maximum mean e.i.r.p. of 200 mW and a maximum mean e.i.r.p. density of 10 mW/MHz in any 1 MHz band or equivalently 0.25 mW/25 kHz in any 25 kHz band”.

Further background on the development of Resolution 229 (Rev.WRC-12), together with information on related ITU-R Recommendations, can be found in document 5A/81 from Australia.  Importantly, Resolution 229 (Rev.WRC-12) assumes that only 1% of RLAN deployments would operate outdoors and that the aggregate noise from WAS/RLANs into victim non-GSO MSS E-to-s feeder link satellite receivers would likely come from multiple countries.

## 5.2	Sharing and compatibility of Aeronautical radionavigation versus WAS/RLAN in the 5 150-5 250 MHz and 5 350-5 460 MHz

### 5.2.1	Sharing and compatibility studies without RLAN mitigation techniques

[RUS]

The technical characteristics and protection criterion of one airborne radar (Radar 16) of the aeronautical radionavigation service are reflected in Table 4.1. The interference scenario from ground based RLAN to airborne Radar receiver is given in Fig. 5.1.

FIGURE 5.1

Interference scenario for RLS air-borne radar receivers

The protection distances ensuring interference free operation of this radar in different operation modes of RLAN were defined for estimation of RLAN compatibility with airborne Radar 16. Estimation of the protection distance was defined subject to the propagation model, described in Recommendation ITU-R Р.525 by the following equation:

(4)

where:

,

– 	radar antenna gain, dB;

λ – 	operational wavelength, m;

σ – 	fading in walls, dB.

In the calculations the assumed altitude of aircraft was Н=10 000 m. Interference to onboard radar are caused by outdoor and indoor RLAN transmitters located at height of 14 m, 20 m and 26 m from the Earth’s surface and using the signal with bandwidth of 20 MHz and 160 MHz. To take propagation loss in the walls into account in equation (4) additional propagation loss, σ, equal to 20 dB was considered. Multi-source interference was taken into account using the following equation.

(5)

where:

– 	aggregate interference level at the radar receiver front end;

– 	level of interference produced by the i-th RLAN transmitter at the radar receiver front end;

N – 	a number of interference sources under consideration.

Table 5.1 presents calculation results for separation distance required for protecting the air-borne radars from single indoor and outdoor RLAN transmitter.

TABLE 5.1

Separation distances (km) required for protecting air-borne radar 
from single indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 16 | >RLOS* | 268 | 76 | 27 |
| * RLOS – line-of-sight distance equals 420 km for a typical flight altitude of 10 000 m. |  |  |  |  |

Analysis of the results reflected in Table 5.1 shows that even in case of a single-source interference caused by outdoor RLAN transmitter with signal bandwidth of 20 MHz the required protection distance could exceed line-of-sight distance between an air-borne radar receiver and a RLAN transmitter. For RLAN using the signal bandwidth of 160 MHz the protection distance is several hundreds km. In case of deployment of a RLAN indoor transmitter the required protection distance could be of several tens km.

Table 5.2 presents calculation results for separation distance required for protecting the air-borne radars from three RLAN transmitters simultaneously operating in one building.

TABLE 5.2

Separation distances (km) required for protecting air-borne radars 
from three indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 16 | >RLOS | >RLOS | 130 | 46 |

Analysis of estimation results described in Table 5.2 shows that consideration of multisource interference caused by RLANs transmitters deployed in one building would result in significant increase of protection distances required for operation of air-borne radar without interference. In particular in case of three outdoor RLANs transmitters operation the required protection distance will significantly exceed the line-of-sight distance even for RLAN with bandwidth of 160 MHz.

Further clarification of effect caused by multisource interference, e.g. when, say, 100 outdoor RLANs transmitters operate in buildings of one urban quarter, would result in a protection distances which would significantly exceed line-of-sight distance for RLAN with bandwidth of 20 MHz. The protection distance for RLAN with 160 MHz bandwidth is about 260 km. Based on the above the conclusions may be drawn that compatibility between RLANs and air-borne radiodetermination radars seems to be extremely difficult to provide without using additional mitigation techniques.

### 5.2.2	Sharing and compatibility studies with RLAN mitigation techniques

[TBD]

## 5.3	Sharing and compatibility of Earth exploration satellite versus WAS/RLAN in the band 5 250-5 570 MHz

[ESA/EUMETSAT]

The sharing and compatibility studies between WAS/RLAN and EESS (active) systems concern  different type of sensors operating in different portions of the 5 250-5 570 MHz band:

- Synthetic aperture radars (SAR) in the 5 350-5 470 MHz band.

- Altimeters covering the whole 5 250-5 570 MHz band.

- Scatterometers with small bandwidths in the 5 250-5 350 MHz band or the 5 350-5 470 MHz band.

According to Resolution 239 (WRC-15), the issues to be addressed in relation to EESS (active) are twofold:

- In the 5 250-5 350 MHz band, to study the possibility of enabling outdoor WAS/RLAN operations including possible associated conditions (see invites ITU-R c)). This implies studies with Altimeter and scatterometer sensors.

- In the 5 350-5 470 MHz band, to study whether additional mitigation techniques would provide coexistence between WAS/RLAN systems and EESS (active) (see invites ITU-R d)) This implies studies with Altimeter, SAR and scatterometer sensors.

### 5.3.1	Determination of the number of RLAN overlapping EESS bandwidths and bandwidth factors

As a first step to address sharing between RLAN and EESS (active), it is necessary to calculate the number of RLAN overlapping the EESS bandwidth and bandwidth factors based on the method proposed by the US administration and widely agreed in JTG 4-5-6-7 (see Annex D of Annex 35 to Document 4-5-6-7/704).

For reference, section 5.3.1.1 presents the calculations undertook during previous study period for the EESS (active) SAR sensor with a 100 MHz bandwidth as used in corresponding studies.

Sections 5.3.1.2 and 5.3.1.3 address the EESS (active) altimeter and scatterometers sensors, respectively, and provide consistent calculations to take into account the different sensor bandwidth of 320 MHz and 2 MHz.

#### 5.3.1.1 	SAR sensor with 100 MHz bandwidth

The overlapping of the EESS (active) SAR bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions that were used for sharing studies with SAR sensor:

- Number of RLAN overlapping the EESS (bandwidth) = 11279

- Corresponding RLAN density per inhabitant = 0.0021

- Average bandwidth factor per overlapping RLAN = -1.94 dB

Similarly, for RLAN density options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions that were used for sharing studies with SAR sensor:

- Number of RLAN overlapping the EESS (bandwidth) = 21000 (D2-low) and 210000 (D2-high)

- Corresponding RLAN density per inhabitant = 0.004 (D2-low) and 0.04 (D2-high)

Summary for SAR sensor:

| RLAN density option | Nb of RLAN overlapping the 100 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 100 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 11279 | 0.0021 | -1.94 dB |
| D2-low | 21000 | 0.004 | -1.94 dB |
| D2-high | 210000 | 0.04 | -1.94 dB |

#### 5.3.1.2 	Altimeter sensor with 320 MHz bandwidth

The overlapping of the EESS (active) altimeter bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions for sharing studies:

- Number of RLAN overlapping the EESS (bandwidth) = 25297

- Corresponding RLAN density per inhabitant = 0.0048

- Average bandwidth factor per overlapping RLAN = -0.4 dB

Similarly, for RLAN density option options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions for sharing studies:

- Number of RLAN overlapping the EESS (bandwidth) = 47103 (D2-low) and 471021 (D2-high)

- Corresponding RLAN density per inhabitant = 0.00897 (D2-low) and 0.0897 (D2-high)

Summary for Altimeter sensor:

| RLAN density option | Nb of RLAN overlapping the 320 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 320 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 25297 | 0.0048 | -0.4 dB |
| D2-low | 47103 | 0.00897 | -0.4 dB |
| D2-high | 471021 | 0.0897 | -0.4 dB |

#### 5.3.1.3 	Scatterometer sensor with 2 MHz bandwidth

The overlapping of the EESS (active) scatterometer bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions for sharing studies:

–	Number of RLAN overlapping the EESS (bandwidth) = 5 786

–	Corresponding RLAN density per inhabitant = 0.0011

–	Average bandwidth factor per overlapping RLAN = -16.03 dB

Similarly, for RLAN density option options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions for sharing studies:

–	Number of RLAN overlapping the EESS (bandwidth) = 10 773 (D2-low) and 107 722 (D2-high).

–	Corresponding RLAN density per inhabitant = 0.0021 (D2-low) and 0.021 (D2-high).

Summary for SCA sensor:

| RLAN density option | Nb of RLAN overlapping the 2 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 2 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 5786 | 0.0011 | -16.03 dB |
| D2-low | 10773 | 0.0021 | -16.03 dB |
| D2-high | 107722 | 0.021 | -16.03 dB |

### 5.3.2	Sharing and compatibility studies with EESS (active) in the 5 250-5 350 MHz band

#### 5.3.2.1 	Altimeter case

Detailed sharing studies between WAS/RLAN and EESS (active) altimeters sensors are presented in Annex 3, based on the ESA Sentinel-3 SRAL sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies provide results for “indoor type” WAS/RLAN (limited to WiFi case) deployment and hence representative of the current situation in the 5 250-5 350 MHz band.

Similar studies addressing “outdoor type” WAS/RLAN deployment taking into account both WiFi and LAA/LTE cases are to be performed.

#### 5.3.2.2 	Scatterometer case

Detailed sharing studies between WAS/RLAN and EESS (active) scatterometers sensors are presented in Annex 4, based on the EUMETSAT EPS-SG SCAsensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies provide results for “indoor type” WAS/RLAN (limited to WiFi case) deployment and hence representative of the current situation in the 5 250-5 350 MHz band.

Similar studies addressing “outdoor type” WAS/RLAN deployment taking into account both WiFi and LAA/LTE cases are to be performed.

#### 5.3.2.3	Consideration of potential RLAN mitigation techniques

TBD

### 5.3.3	Sharing and compatibility studies with EESS (active) in the 5 350-5 470 MHz band

#### 5.3.3.1 	Status of sharing studies

Sharing and coexistence between RLAN 5 GHz and EESS (active) in the 5 350-5 470 MHz band were studied during the previous study period (in JTG 4-5-6-7) and led to the following conclusion (see CPM Report to the 2015 World Radiocommunication Conference (Document N° 3 of WRC-15)):

“Results of sharing studies show that with the RLAN parameters described above, sharing between RLAN and the EESS (active) systems in the 5 350-5 470 MHz frequency band would not be feasible. Sharing may only be feasible if additional RLAN mitigation measures are implemented.”

One should also highlight the fact that these findings are duly reproduced in Recognising a) of Resolution 239 (WRC-15).

These conclusions were developed after intensive studies on sharing and coexistence between RLAN 5 GHz and EESS (active) mainly with EESS (active) SAR systems. These conclusions are summarised in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] on “Sharing studies between RLAN and EESS (active) systems in the frequency range 5 350-5 470 MHz” (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)). Taking into account the whole ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) under all scenarios, these studies shows that RLAN deployment in the frequency band 5 350-5 470 MHz would create large interference in the CSAR sensor on board the Sentinel-1 satellite (up to 30.4 dB).

Additional studies have been performed using static and dynamic analysis with altimeter (Sentinel-3 Altimeter sensor (SRAL)) and scatteromter (EPS-SG sensor (SCA)) and are given in Annex 3 and 4, repsectively. These studies take into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies confirms that, under all scenarios, RLAN deployment in the 5 GHz range would create large interference to altimeters (up to 26.6 dB) and scatterometers (up to 20.9 dB), hence in the same order of magnitude than for SAR sensors.

All together, these studies confirm the findings of JTG 4-5-6-7 and CPM-15-2 that sharing between RLAN and EESS (active) in the 5350-5470 MHz band may only be feasible if additional RLAN mitigation measures are implemented.

Consistently with Resolution 239 (WRC-15), the work in this frequency band should hence concentrate on studying whether any additional mitigation techniques beyond those analysed in the previous studies would provide coexistence between WAS/RLAN systems and EESS (active) in the in the 5 350-5 470 MHz band.

#### 5.3.3.2	Consideration of potential RLAN mitigation techniques

TBD

## 5.4	Sharing and compatibility of Radiolocation versus WAS/RLAN in the 5 250-5 470/5 725-5 850 MHz

### 5.4.1	Sharing and compatibility studies without RLAN mitigation techniques

[RUS]

#### 5.4.1.1 	Compatibility of WAS/RLAN with airborne radars of the radiolocation service in the frequency band 5 350-5 470 MHz

Analysis of data given in Table 4.3 shows that airborne Radar 9 and Radar 17 operate in the frequency band 5 350-5 470 MHz. Interference impact assessment from RLAN to these radars was performed subject to the interference scenario given in section 5.2 hereof. Table 5.3 contains the calculation results of the required protection distance for airborne Radars from single outdoor and indoor RLAN transmitters.

TABLE 5.3

Separation distances (km) required for protecting air-borne radars 
from indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 9 | >RLOS | >RLOS | 200 | 71 |
| Radar 17 | >RLOS | 342 | 97 | 34 |

Analysis of the results reflected in Table 5.3 shows that even in case of a single-source interference caused by outdoor RLAN transmitter the required protection distance could far exceed line-of-sight distance between an air-borne radar receiver and a RLAN transmitter. In case of deployment of a RLAN indoor transmitter the required protection distance could range from 34 km to 200 km depending on the RLAN bandwidth.

Estimation of the required protection distances was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building. The estimation results are shown in Table 5.4.

TABLE 5.4

Separation distances (km) required for protecting air-borne radars
 from three indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 9 | >RLOS | >RLOS | 344 | 122 |
| Radar 17 | >RLOS | >RLOS | 166 | 59 |

Analysis of estimation results described in Table 5.4 shows that a partial consideration of multisource interference caused by RLANs transmitters would result in significant increase of protection distances required for operation of air-borne radar without interference.

Further clarification of effect caused by multisource interference, e.g. when, say, 100 outdoor RLANs transmitters operate in buildings of one urban quarter, would result in a protection distance which would significantly exceed line-of-sight distance for airborne radars at the altitude of 10 km.

Consideration of interference caused by deployed in one urban quarter indoor RLANs transmitters with 20 MHz signal bandwidth shows that the required protection distance would exceed the line-of-sight distance for radars 9 and 17. Consideration of RLANs transmitters with 160 MHz signal bandwidth would result in the required protection distance exceeding the line-of-sight distance for radar 9. The protection distance would be 342 km for Radar 17. Based on the above the conclusions may be drawn that compatibility between RLANs and air-borne radars seems to be extremely difficult to provide without using additional mitigation techniques.

#### 5.4.1.2 	Compatibility of WAS/RLAN with ground radars of the radiolocation service in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

Compatibility evaluation of WAS/RLAN with ground-based radars operating in the considered frequency bands was performed in line with the interference scenario given in section 5.2 hereof.

FIGURE 5.2

Interference scenario for RLS ground-based radar receiver

Interference was estimated using a propagation model described in Recommendation ITU-R Р.452. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Propagation loss in walls was considered using the following equation:

, dBW	(6)

where:

–	additional fading, dB.

Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.5 and Table 5.6 presents minimum estimated separation distances required for protection of ground-based radiodetermination radars from a single-source interference caused by outdoor RLAN transmitters in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz accordingly. Estimations were conducted for two operation modes of RLAN with bandwidth of 20 MHz and 160 MHz. Estimation was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building at the same heights of RLAN.

TABLE 5.5

Separation distances (km) required for protecting ground-based radiolocation radars from outdoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 63 | 65 | 68 | 71 | 48 | 54 | 57 | 60 |
| Radar 3 | 54 | 57 | 59 | 62 | 44 | 47 | 50 | 53 |
| Radar 4 | 44 | 48 | 50 | 52 | 36 | 39 | 42 | 44 |
| Radar 5 | 49 | 51 | 54 | 58 | 40 | 43 | 46 | 48 |
| Radar 7 | 32 | 34 | 38 | 39 | 25 | 28 | 32 | 33 |
| Radar 12 | 34 | 38 | 40 | 42 | 27 | 31 | 33 | 35 |
| Radar 13 | 54 | 56 | 58 | 61 | 44 | 47 | 50 | 52 |
| Radar 15 | 54 | 57 | 59 | 62 | 45 | 48 | 50 | 53 |
| Radar 19 | 54 | 57 | 59 | 62 | 45 | 48 | 51 | 53 |
| Radar 20 | 52 | 54 | 57 | 60 | 43 | 46 | 48 | 51 |
| Radar 21 | 54 | 57 | 60 | 63 | 45 | 48 | 51 | 53 |
| Radar 22 | 42 | 44 | 47 | 49 | 33 | 37 | 40 | 42 |
| Radar 23 | 31 | 34 | 37 | 38 | 24 | 28 | 31 | 38 |

TABLE 5.6

Separation distances (km) required for protecting ground-based radiolocation radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 72 | 75 | 77 | 81 | 60 | 63 | 65 | 69 |
| Radar 4 | 52 | 55 | 57 | 60 | 43 | 46 | 48 | 51 |
| Radar 5 | 56 | 59 | 62 | 65 | 47 | 50 | 52 | 55 |
| Radar 7 | 38 | 41 | 43 | 45 | 30 | 34 | 37 | 38 |
| Radar 12 | 40 | 43 | 46 | 49 | 33 | 36 | 38 | 41 |
| Radar 13 | 62 | 65 | 67 | 71 | 51 | 54 | 57 | 60 |
| Radar 15 | 62 | 65 | 68 | 72 | 52 | 55 | 58 | 60 |
| Radar 22 | 49 | 52 | 54 | 57 | 40 | 43 | 46 | 48 |
| Radar 23 | 36 | 39 | 42 | 44 | 29 | 33 | 35 | 37 |

Analysis of the estimation results described in Tables 5.5 and 5.6 shows that the separation distances required for protecting ground radars would be of several dozen km even for RLANs using data transfer channel of 160 MHz. For example, in the frequency band 5 350-5 470 MHz for Radar 2 suffering interference from RLAN transmitter using a data channel of 160 MHz bandwidth and deployed at a height of 26 m the estimated protection distance is 57 km. For the same Radar operating in the frequency band 5 725-5 850 MHz the protection distance is 65 km. Based on the above the conclusions may be drawn that enlarging RLANs bandwidth to reduce spectral density of interference caused for radars may not be considered as one of the interference mitigation techniques in relation to ground-based radiodetermination radars.

Minimum separation distances required for protection of ground radars from a single-source interference caused by indoor RLAN transmitters in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz are presented in Tables 5.7 and 5.8. Estimations were conducted for two operation modes of RLAN. Estimation was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building at height of 14 m, 20 m. and 26 m.

TABLE 5.7

Separation distances (km) required for protecting ground-based radiolocation radars from indoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 3 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 4 | 28 | 31 | 34 | 36 | 22 | 26 | 26 | 29 |
| Radar 5 | 31 | 34 | 37 | 39 | 24 | 28 | 31 | 32 |
| Radar 7 | 17 | 18 | 18 | 24 | 6 | 6 | 6 | 10 |
| Radar 12 | 21 | 23 | 24 | 27 | 8 | 8 | 8 | 15 |
| Radar 13 | 34 | 37 | 40 | 41 | 28 | 31 | 33 | 35 |
| Radar 15 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 19 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 20 | 33 | 37 | 39 | 41 | 27 | 30 | 32 | 34 |
| Radar 21 | 35 | 38 | 41 | 43 | 28 | 32 | 34 | 36 |
| Radar 22 | 26 | 29 | 32 | 33 | 21 | 23 | 24 | 26 |
| Radar 23 | 15 | 15 | 15 | 21 | 5 | 5 | 5 | 8 |

TABLE 5.8

Separation distances (km) required for protecting ground-based radiolocation radars from indoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 48 | 51 | 53 | 56 | 39 | 42 | 45 | 47 |
| Radar 4 | 33 | 36 | 39 | 41 | 27 | 30 | 33 | 34 |
| Radar 5 | 36 | 40 | 43 | 44 | 29 | 33 | 36 | 37 |
| Radar 7 | 23 | 26 | 28 | 30 | 14 | 14 | 14 | 21 |
| Radar 12 | 25 | 28 | 31 | 32 | 19 | 20 | 20 | 26 |
| Radar 13 | 40 | 43 | 46 | 48 | 33 | 36 | 39 | 41 |
| Radar 15 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 22 | 31 | 34 | 37 | 39 | 24 | 28 | 31 | 32 |
| Radar 23 | 23 | 25 | 27 | 29 | 11 | 11 | 11 | 20 |

Analysis of the estimation results described in Tables 5.7 and 5.8 shows that in spite of reducing the level of interference due to fading in the walls the protection distances of several tens km are required for protection of the radar receivers. The results shown in these Tables were gained for the walls with propagation loss of 20 dB. However it is to note that the level of signal fading in walls is overestimated for a significant number of buildings, office ones specifically. Therefore, the required separation distances would exceed those shown in Tables 5.7 and 5.8.

Thus RLAN transmitter signal fading when propagating in walls of buildings may not be considered as an effective interference mitigation technique as well as that of enlarging the channel bandwidth.

#### 5.4.1.3	Compatibility of WAS/RLAN with ground meteorological radars in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

Protection distances for the ground-based meteorological radars operating in the frequency bands considered were also estimated in line with the interference scenario depicted in Figure 5.2. Interference was estimated using a free space propagation model described in Recommendation ITU-R Р.452. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Estimation was also conducted assuming multisource interference caused by three simultaneously operating RLAN transmitters deployed in a single building at height of 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (6). Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.9 and Table 5.10 present minimum estimated separation distances required for protection of ground-based meteorological radars from interference caused by outdoor RLAN transmitters. Estimations were conducted for two modes of RLAN operation.

TABLE 5.9

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 46 | 49 | 51 | 54 | 38 | 41 | 44 | 49 |
| Radar 4 | 54 | 56 | 59 | 62 | 44 | 47 | 50 | 52 |
| Radar 8 | 60 | 62 | 65 | 68 | 50 | 53 | 55 | 59 |
| Radar 12 | 63 | 66 | 69 | 72 | 53 | 55 | 58 | 61 |
| Radar 13 | 75 | 77 | 79 | 83 | 62 | 65 | 68 | 71 |
| Radar 14 | 64 | 66 | 68 | 70 | 53 | 55 | 58 | 61 |

TABLE 5.10

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 8 | 70 | 72 | 74 | 80 | 58 | 60 | 62 | 66 |

Analysis of the estimation results described in Tables 5.9 and 5.10 shows that strengthening the protection criteria for the radars would result in increasing the separation distances ensuring protection of ground-based meteorological radars. For example, in the Radar frequency band 5 350-5 470 MHz the required protection distance would be up to 79 km for Radar 13 assuming a single RLAN transmitter using a data channel of 20 MHz bandwidth deployed at height of 26 m. In case of a RLAN transmitter using a 160 MHz channel bandwidth the separation distance would reduce to 68 km and still would be of several tens km even for the rest considered Radar types. Based on the above the conclusions may be drawn that enlarging RLANs bandwidth to reduce spectral density of interference caused for radars may not be considered as one of the interference mitigation techniques in relation to operation of meteorological radars.

Table 5.11 and Table 5.12 present minimum estimated separation distances required for protection of meteorological radars from interference caused by indoor RLAN transmitters.

TABLE 5.11

separation distances (km) required for protecting ground-based meteorological radars from indoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 29 | 33 | 36 | 37 | 24 | 27 | 29 | 31 |
| Radar 4 | 34 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 8 | 39 | 42 | 45 | 48 | 32 | 35 | 37 | 39 |
| Radar 12 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 13 | 47 | 50 | 53 | 56 | 39 | 42 | 44 | 46 |
| Radar 14 | 44 | 47 | 49 | 51 | 36 | 38 | 41 | 43 |

TABLE 5.12

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 8 | 45 | 49 | 51 | 54 | 38 | 41 | 43 | 45 |

Analysis of the estimation results described in Tables 5.11 and 5.12 shows that in spite of reducing the level of interference to operation of the ground-based meteorological radar receivers the majority of them would require protection distances of several tens km. The results shown in Tables 5.11 and 5.12 were gained for the walls with propagation loss of 20 dB. However it is to note that the level of signal fading in walls is overestimated for a significant number of buildings, office ones specifically. Therefore, the required separation distances would exceed those shown in Tables 5.11 and 5.12.

Thus RLAN transmitter signal fading when propagating in walls of buildings may not be considered as effective interference mitigation technique as well as that of enlarging the channel bandwidth.

### 5.4.2	Sharing and compatibility studies with RLAN mitigation techniques

[SUI]

For sharing between RLAN and meteorological radar the application of a mitigation technique, like DFS is required. Table 21 provides a summary of the DFS requirements for the frequency band 5 350-5 470 MHz. These parameters proved to allow WAS to avoid interfering with the radio determination service in the band 5 600-5 650 MHz.

Table 21

| Parameter | Values for the frequency band 5 350-5 470 MHz |
| --- | --- |
| Minimum pulse width (see detailed test signals in Report ITU-R M.2115) | 0.5 μs |
| PRF (see detailed test signals in Report ITU-R M.2115) | Fixed, Staggered and Interleaved |
| Channel Availability Check (CAC) time | 10 minutes |
| Off-Channel CAC (Note 1) | Yes |
| CAC and Off-Channel CAC detection probability (Note 2) | 99.99% |
| In-service monitoring detection probability | 60% |
| CAC for slave devices with power above 200 mW (after initial detection by In-service) | Yes |
| Detection Threshold | -62 +10 -EIRP Spectral Density (dBm/MHz) + G (dBi), however the DFS threshold level shall not be lower than -64 dBm assuming a 0 dBi receive antenna gain |
| Channel Move time | 10s |
| Channel closing time | 1s |
| Non-occupancy period | 30 minutes |
| Possibility to exclude 5 600-5 650 MHz band from the channel plan or to exclude these channels from the list of usable channels | Yes |
| Requirement that none of the DFS related settings are accessible to the end-user | Yes |

## 5.5	Sharing and compatibility of Radionavigation versus WAS/RLAN in the 5 460-5 470 MHz

### 5.5.1	Sharing and compatibility studies without RLAN mitigation techniques

[RUS]

In accordance with Recommendation ITU-R 1638-1 two types of ground-based navigation radars: Radar 10 and Radar 10A operate in this frequency band. The technical characteristics and protection criteria are given in Table 4.2. For the specified Radars the required protection distances were estimated in line with interference scenario shown in Figure 5.2. With this a propagation model, presented in Recommendation ITU-R Р.452 was used for estimation. The estimation assumed the height of RLAN transmitters to be 14 m, 20m and 26 m. Estimation was also conducted assuming multisource interference caused by three simultaneously operating RLAN transmitters deployed in a single building at height of 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (6). Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.13 presents minimum estimated separation distances required for protection of ground-based radionavigation radars from a single-source interference caused by outdoor RLAN transmitters. Table 5.13 also contains estimation results for a case of multiple interference caused by three simultaneously operating outdoor RLAN transmitters deployed out of a single building. Estimations were conducted for two modes of RLAN operation.

TABLE 5.13

Separation distances (km) required for protecting ground-based radionavigation radars 
from outdoor deployed RLANs in the frequency band 5 460-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 10 | 47 | 50 | 53 | 55 | 38 | 41 | 44 | 46 |
| Radar 10а | 47 | 50 | 53 | 55 | 38 | 41 | 44 | 46 |

Analysis of the estimation results shows that the required protection distances for radionavigation Radars from RLAN with bandwidth of 20 MHz would be of several dozen km. In case of a RLAN transmitter using a 160 MHz channel bandwidth the separation distance would be reduced and still the reduction will not exceed 20% from the protection distance for RLANs using data transfer channel of 20 MHz. Table 5.14 presents estimated separation distances required for protection of radionavigation radars from interference caused by indoor RLAN transmitters.

TABLE 5.14

Separation distances (km) required for protecting ground-based radionavigation radars 
from indoor deployed RLANs in the frequency band 5 460-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 10 | 30 | 33 | 36 | 37 | 23 | 27 | 30 | 31 |
| Radar 10а | 30 | 33 | 36 | 37 | 23 | 27 | 30 | 31 |

Analysis of the estimation results shows that in spite of significant reduction of the  interference level caused by RLAN transmitters the protection distances of several tens km are required for protection of the ground-based radionavigation radar receivers.

### 5.5.2	Sharing and compatibility studies with RLAN mitigation techniques

[TBD]

## 5.6	Sharing and compatibility of Fixed Service versus WAS/RLAN in the 5 850-5 925 MHz

## 5.7	Sharing and compatibility of Fixed Satellite Service versus WAS/RLAN in the 5 725-5 850 MHz (for Region 1) and 5 850-5 925 MHz

### 5.7.1	Sharing and Compatibility in 5 725-5 850 MHz band

### 5.7.1.1	Study 1

[LUX]

Numerous studies have been conducted by interested Administrations (for example, within CEPT SE 24) to estimate the levels of interference which FSS space receivers could absorb without generating a harmful level of interference into FSS space receivers. Extracts of such studies are provided in Doc. 5A/246 [UK] and included here as Study 2. The general conclusions reached indicate that sharing of WAS/RLAN and FSS space receivers would be very difficult and additional techniques should be developed to mitigate the risk of harmful interference created by the aggregate interfering signals originated by transmitting WAS/RLAN stations. Indeed, the agenda item 1.16 called for studies on suitable mitigation techniques (resolves b). At the lack of reliable mitigation techniques which could be implemented at affordable costs for WAS/RLAN networks, assumptions on the RLAN characteristics are based on Annex 24 of the WP 5 A Chairman´s Report from May 2016 meeting.

#### 5.7.1.1.1	Estimation of interference from WAS/RLAN through empirical methods

As a potential way forward to estimate the interference from WAS/RLAN, an innovative contribution was made available to WP 5A (Doc. 5A/91) suggesting the estimation of interference through an empirical method consisting on airborne measurements. A separate report on measurements on interference created by RLAN using airborne is under elaboration.

A detailed analysis on the set of airborne measurements has been made. In general these airborne measurements are potentially interesting to:

- Achieve a better understanding of the study models before the actual RLAN deployment, with a view to determine the actual interference level observed and compare it with calculated results of studies.

- To measure and control, including on the long term, the aggregate interference into FSS space stations when RLAN deployment is developing.

Preliminary conclusions on the airborne measurements do not allow at this stage characterisation of the interference environment and do not allow quantitative conclusions to be drawn. Additional measurements campaigns from airplanes (including in other geographical areas) would be necessary before being able to draw any conclusions from such measurements.

Furthermore, the following issues should be investigated and clarified on possible future airborne measurements:

- Measurements are made locally at low altitude (few kms) whereas FSS space stations cover wide geographic areas and operate from the GSO orbit in space.

- It is not clear how measurements at 2.4 GHz (for which the reference point is not clear, is it less congested than expected?) can help characterise the interference environment at 5 GHz, especially since measurements at 5 GHz are close the noise floor.

- Measurements should be made during busy hours.

- It is not clear how space and time dynamics of an airplane is comparable with the case of GSO satellites (which are seen from the Earth as static in the sky).

- These measurements have considered two elevation angles (30 and 90 degrees). It would seem useful to specifically conduct measurements at elevation and azimuths corresponding to the GSO orbit, since the GSO orbit as seen from the Earth is a particular path in the sky, from 0 degree elevation angle at horizon for the most western and eastern azimuths to about 48 degrees elevation angle at south azimuths, from London.

#### 5.7.1.1.2	Potential mitigation techniques for WAS/RLAN

It can be noted that the characteristics of RLAN systems do not provide sufficient details on potential mitigation techniques, despite the fact that previous studies concluded on the need of such mitigation techniques. It is understood that further specific sharing studies WAS/RLAN and FSS will be conducted as part of the AI 1.16 work plan; presumably, these studies will conclude on the need of adequate mitigation techniques. See, for example, the conclusions reached by JTG at the previous study cycle for similar sharing studies (Example: Doc. 2357).

For protection of the FSS, two possible mitigation techniques can be identified:

  - Limitation of RLAN deployment to indoor only.

  - Placing a limit on the maximum eirp of RLAN APs. The appropriate value can be determined through additional studies for RLAN under WRC-19 agenda item 1.16 (noting a value of 10 to 15 dBm/20 MHz has been determined by the ITU in the context of studies between IMT and the FSS, with an interference configuration similar to the RLAN case, see Report ITU-R S.2367).

These two mitigation techniques seem relatively simple to implement but are sometimes difficult to enforce for the RLAN type of equipment (e.g. indoor only, or non-compliant equipment). The impact of such mitigation techniques on the RLAN deployment and operations (e.g. RLAN eirp distribution and the resulting limitations on data rates possible), should be studied.

#### 5.7.1.1.3	Additional compatibility studies in adjacent bands

Compatibility studies between RLAN operating in any of the sub-bands within 5 725-5 925 MHz and the FSS operating in adjacent sub-bands are necessary.

#### 5.7.1.1.4	Protection of FSS

The protection criterion for FSS is to accept an increase of the equivalent temperature greater than x% due to interference signals on the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.: ΔT/T≤ x%. This criterion is applicable for interference scenarios when the interfering signal is time invariant, which is assumed the case for the aggregate interference levels created by WAS/RLAN in this uplink satellite band, because coverage areas are large (hemispheric or global) and there is no mobility aspect due to the fact that main interference is from RLAN access points which are fixed.

The values of x is 6%, generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment, and x=1%, generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme should be considered where interference from RLANs is limited to half of the ΔT/T= 6% criterion i.e. the ΔT/T objective is reduced to a value of 3%. In addition, geographic apportionment could also be applied depending on the satellite’s coverage and the assumptions for the RLAN deployment.

### 5.7.1.2	Study 2

[UK]

#### 5.7.1.2.1	Background and analysis

This analysis is looking results of some previous sharing studies carried out in ITU-R and CEPT. We use a methodology that makes reference too and analyses the 3 different studies carried out which are-

–	Report ITU-R S.2367 – Sharing and compatibility between International Mobile Telecommunication systems and fixed-satellite service networks in the 5 850-6 425 MHz frequency range.

–	ECC Report 244 – Compatibility studies related to RLANs in the 5 725-5 925 MHz band (see annex1).

–	Annex 25 of the last WP 5A Chairman’s Report – Use of aggregate RLAN measurements from airborne and terrestrial platforms to support studies under WRC-19 agenda item 1.16.

When analysing these three studies in order to arrive at a useful range of results we had to make a choice on which study should be used to form the baseline analysis. The UK decided to use the studies contained in ECC Report 244 as the baseline study. Once choosing the baseline study to work from we took a 3 step approach to the analysis when looking at an assessment of the interference from RLAN into FSS.

–	Step 1: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receivers under consideration (see Table 2) (considering the satellite footprint) whilst satisfying the FSS protection criteria.

–	Step 2: This step delivers a sensitivity analysis of the number of active, on-tune, RLAN transmitters using various different assumptions when looking at possible RLAN deployment models. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing when inputting the different assumptions. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

–	Step 3: This step delivers a comparison of the overall results from the measurement campaigns with the range of various results that can be seen from Step 2 and looks to see if the range of the results that may be valid for further study could be reduced.

#### 5.7.1.2.2	Decisions made regarding assumptions in Step 1

In addition to taking these steps to reduce the amount of range of results from the models further the UK have also made some decisions on the choices in the assumptions made on some of the technical parameters that are considered to be reasonable. From the UK perspective these choices were based on the following principles: evidence based where available, realistic, justifiable and not overly conservative.  Table 1 below shows the list of assumptions, the list of some of the choices for technical parameters in the studies and the which choices the UK consider to be reasonable.

Table 1

UK Views on some technical parameters

| Technical Parameters | Choices for technical parameters to be modelled |  | Choice considered reasonable in the UK | Document 4-5-6-7/566 |
| --- | --- | --- | --- | --- |
| % of outdoor usage | 2% | 5% | 2% | UK submitted a paper in the last study cycle document 4-5-6-7/566 which provided an evidence based analysis of the numbers of outdoor Wi-Fi users in the UK. It concluded even with very conservative assumptions it was very difficult to reach 2% outdoor use. In addition, WRC-03 studies on Wi-Fi vs FSS in 5 150-5 250 MHz bands only took account of studies using 1% outdoor use. It should be noted that ECC Report 244 only presents results for 5.3% outdoor usage. |
| Building penetration losses | 12 dB | 17 dB | 17 dB | The different studies contained in ITU-R Report S.2367 which looks at IMT vs FSS in 5 850-6 425 MHz assume a range of 12-35 dB for building penetration losses, so 17 dB still seems a reasonable assumption based on this agreed ITU-R Report and the choices given in ECC Report 244. |
| Antenna Discrimination/Body loss | 0 dB | 4 dB | 4 dB | In previous ITU-R studies 4 dB was considered realistic in the 5 GHz bands RLAN sharing with EESS and in some IMT studies at lower bands around 2 GHz the body losses from mobile devices was up to 6 dB. |
| Polarisation mismatch | 1.5 dB | 3 dB | 1.5 dB | 3 dB may be overestimating the polarisation losses |
| Clutter losses | Free space model | Free space model + Clutter model based on Rec. ITU-R P.452 in relation to antenna height distribution | Free space model + Clutter model based on Rec. ITU-R P.452 in relation to antenna height distribution | Seems a reasonable assumption to include clutter losses for Urban and Suburban environments. |

In both Report ITU-R S.2367 and ECC Report 244 they used a protection criteria of ΔT/T = 6% coordination criteria as provided in Recommendation ITU-R S.1432 for the sharing analysis. There was also further discussion in the ECC Report 244 around how to take account of both geographic and service apportionment. From the UK perspective as the 5 725-5 850 MHz band is only allocated on a primary basis to FSS for ITU-R Region 1 only and as such we believe there is no need to take account for any geographic apportionment. With regards to appropriate service apportionment for this band we believe that the studies should show the results for the full ΔT/T = 6% for the mobile service as you unlikely to have mobile and fixed services sharing the same channels in a given geographic area due to intra-interference. In addition, the use of the band by fixed services now and into the future looks to be very limited based on current information and projected demand.

#### 5.7.1.2.3	Results from ECC Report 244 for Step 1 and 2

From the studies shown in ECC Report 244 (see attached annex 1 for relevant parts of ECC Report 244) the initial results of the sharing and compatibility analysis carried out for RLAN vs FSS indicate depending on certain assumptions, sharing can be shown to be feasible in a number of cases. Other results show that sharing would be more difficult based on more conservative assumptions for the RLAN parameters considered. The studies in ECC Report 244 contain a sensitivity analysis highlighting 28 cases (Case 1 to Case 28) which show results that range from conservative figures for RLAN numbers/activity put forward by the satellite industry to less conservative proposals put forward by the RLAN industry. See below for a summary of the relevant satellites studied in Table 2 and Table 3 which summarises the range of results from ECC Report 244.

Table 2

Sample Satellite Data for the band 5 725-5 850 MHz

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| D | 3o East | Whole band | 34 | 773 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |

Table 3

Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2 to 2 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4 to 8 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 5 to 15 dB) |

#### 5.7.1.2.4	Additional UK thoughts on step 2

In step 2 there are various input assumptions in the stages we go through to come up with the 28 cases that are presented in ECC Report 244 that are summarised in Table 3 above. If we look into the choices that come up the ranges and make a choice to remove the cases that we think are unrealistic or overly conservative then we can further reduce the cases that may be considered as relevant. If we consider the values chosen to study as a result of Stages 1 – 3 in step 2 to determine the total number of access points accross Europe there is a choice of 300, 400 or 500 million. Looking at the formula the main bulk (91.8% – 92.5%) of these total calculated numbers are from residential access point numbers. In addition the total number of households in Europe used included with 10% increase on current numbers to give 320 million households. Looking at the 500 million figure it looks to have been thought up as a nice number because the calculated numbers were in the range of 300 and 400 million. But when you consider the formula to reach the 500 million number you would have to account for a massive increase 200% in non-residential access points in order to reach the 500 million number. We already some of the figures used in the the formula to be excessive (90% household penetration in EU) considering that the UK maket report for 2015 only had Broadband penetration numbers at 79% of households. Therefore we think that we can eliminate the 500 million number.

The table 4 below gives some of our thoughts on the figures used in stages 4 – 7.

Table 4

UK thoughts on step 2 stage 4 – 6 choices from ECC Report 244

| Stage no and Parameter title | Choices for parameters to be modelled |  |  | Choice considered reasonable in the UK | Reasoning for UK Choice |
| --- | --- | --- | --- | --- | --- |
| Stage 4 Busy Hour Factor | 50% | 62.7% | 70% | 50%, 62.7% | 70% seems a very high figure when you consider the numbers of people who will be working, on holiday, reduction in business use doing other non- WiFi activities out of home and in the home. |
| Stage 5 Apply 5 GHz spectrum factor | 50% | 74% | 97% | 50%, 74% | 97% percent seems extremely high when you consider there are 3 Wi-Fi bands and not all of the home broadband users will need the higher data rates the 5GHz can provide. |
| Stage 6 RF Activity Factor | 3% | 10% | 30% | 3%, 10% | When Wi-FI networks are fully loaded their highest activity factor is around 30% due to the way the protocol works. It is very unlikely that all networks in the busy hour will be fully loaded. |

If we were to input these new assumptions into the ECC Report 244 studies we would end up with 12 different cases for each of the satellites which are summarised below in Table 5.

Table 3

UK choices and new Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1) | 4 | 17 | FSS protection criteria satisfied for all Satellites (margin ranges 1.3 to 12 dB) |
| “Medium” scenario (Case 11) | 4 | 17 | FSS protection criteria satisfied for Satellites B & F (margin ranges 4 to 5 dB). FSS protection criteria exceeded for other Satellites A,D (exceeding ranges 5 to 6 dB) & G (exceeding ranges 2 to 3 dB) |
| “Pessimistic” scenario (Case 23) | 4 | 17 | FSS protection criteria satisfied for Satellites B & F (margin ranges 2 to 7 dB). FSS protection criteria exceeded for other Satellites A,D (exceeding ranges 7 to 8 dB) & G (exceeding ranges 4 to 6 dB) |

#### 5.7.1.2.5	Step 3 – Taking into account Wi-Fi Airborne Measurement Campaigns in 2.4 GHz and 5 GHz

The different factors used in step 2 are subject to some uncertainties because of the difficulties involved when deriving values for these factors and in particular when making predictions for future. Due to the lack of evidence being available, there has been a lot of debate on how aggregate interference from a fully mature rollout of WAS/RLAN would look in the future within the 5GHz band to satellite and airborne platforms. Therefore, it was agreed in CEPT to perform a sensitivity analyses, taking into account ranges of values for some of these factors.

Unfortunately, the introduction of a large range of assumptions into the theoretical models used in steps 1 and 2 has led to a very large range of results that we see in the 27 cases summarized in table 3 above. This is due largely to the number of variables that has been introduced into the models to be studied, this makes it is difficult to come to any conclusions as the large range of assumptions means predicting an acceptable range of results to make conclusions from is very difficult.

The draft working document towards a PDN Report contained in Annex 25 of the last WP 5A chairman’s report introduces new airborne measurement methods and a proposed methodology for comparing RLAN measurement results in both the 2.4 GHz and 5 GHz bands with the theoretical RLAN aggregate interference modelling used in steps 1 and 2 of the ECC Report defined over a defined geographical area.

When taking into account the UK measurements and our analysis of the airborne measurement campaign results contained in Annex 1 and 2 of the draft working document towards a PDN Report contained in Annex 25 of the last WP 5A Chairman’s Report we came to some initial conclusions that the measured figures were somewhere between the "optimistic" scenario (case 1) and "medium" scenario (case 14) that are summarised above in Table 3.

If we assume the measurement results presented in Annex 1 and 2 of The draft working document towards a PDN Report contained in Annex 25 of the last WP 5A Chairman’s Report give a fair reflection of the situation which may appear in a mature 5 GHz market in the future then we can assume even if we take account and reduce the range of results we use for basing our conclusions on then from table 3 above to that for Case 1  =  up to 12 dB below the threshold and the results for Case 14  = up to 8 dB exceeding the margin we would still be leaving an acceptable amount of conservatism in our overall analysis..

Over all when looking at the studies the UK believe that sharing may be possible between RLAN and FSS in the 5 725-550 MHz band especially if further mitigation techniques are investigated further in order to reduce the negative margins shown in the results for case 14 in Table 3. We believe if we take into account all of these factors with some further analysis that we can prove sharing is feasible even when using what we consider to be conservative assumptions in the analysis.

Additional considerations we think that should be investigated further to try to make the studies looking at the potential for RLAN–FSS sharing in this band are in the areas, such as:

–	Studies looking at lower values of outdoor usage (i.e. 1%, 2%);

–	Studies supporting Stage 8 of FSS Step 2 (see section 8.1.3.4);

–	5 GHz Spectrum Factor (Stage 5 of FSS Step 2);

–	Control / monitoring on the long term aggregate effect of RLAN interference into FSS as RLAN deployment increases and investigation of what can be done in a scenario where the interference threshold is reached;

–	Potential mitigation techniques have to be considered and assessment of their impact on the potential for sharing between RLAN and FSS is required. Their feasibility and practicality of being implemented in RLANs and their potential operational effects could be further studied. Example potential mitigation techniques that could be addressed are:

–	RLAN Access Points deployed only indoor (i.e. no fixed outdoor).

–	Additional power limitations for RLAN.

–	Separate rules for fixed outdoor and mobile RLAN use.

–	More evidence gathered around the numbers used in step 2.

–	More measurement campaigns.

#### 5.7.1.2.6	Some thoughts on the possible introduction of LTE LAA or LTE-U into the bands

LAA-LTE is expected to be deployed in the existing 5 GHz bands by operators, mainly in hotspots and enterprise environments. Therefore, in Annex 1 Appendix 2 of this document we have added an analysis using the same methodology used in ECC Report 244 to estimate what the additional deployments of LAA-LTE on top of the existing RLAN deployments would make the results of the sharing studies with FSS look like. From the additional studies that have been made in CEPT for LAA-LTE, when compared to ECC Report 244 the following conclusions became apparent:

–	Compatibility of a mix LAA and WiFi market share with FSS. Results are roughly the same as for the case of WiFi only.

Therefore, we believe that we can assume that any impact of adding LAA-LTE use case in 5 GHz bands appears to have minimal effect on the overall results of compatibility and sharing as shown in ECC Report 244.

Any additional analysis being considered further for future overall RLAN vs FSS studies should also consider the effect any new mitigations is likely to effect the possible use of the RLAN extension bands by LAA-LTE technologies.

5.7.2	Sharing and compatibility in 5 850-5 925 MHz band

[LUX]

Numerous studies have been conducted by interested Administrations (for example, within CEPT SE 24) to estimate the levels of interference which FSS space receivers could absorb without generating a harmful level of interference into FSS space receivers. Extracts of such studies are provided in Doc. 5A/246 [UK] and included here as Study 2. The general conclusions reached indicate that sharing of WAS/RLAN and FSS space receivers would be very difficult and additional techniques should be developed to mitigate the risk of harmful interference created by the aggregate interfering signals originated by transmitting WAS/RLAN stations. Indeed, the agenda item 1.16 called for studies on suitable mitigation techniques (resolves b)). At the lack of reliable mitigation techniques which could be implemented at affordable costs for WAS/RLAN networks, assumptions on the RLAN characteristics are based on Annex 24 of the WP 5A Chairman´s Report from May 2016 meeting.

#### 5.7.2.1	Estimation of interference from WAS/RLAN through empirical methods

As a potential way forward to estimate the interference from WAS/RLAN, an innovative contribution was made available to WP 5A (Doc. 5A/91) suggesting the estimation of interference through an empirical method consisting on airborne measurements. A separate report on measurements on interference created by RLAN using airborne is under elaboration.

A detailed analysis on the set of airborne measurements has been made. In general these airborne measurements are potentially interesting to:

- Achieve a better understanding of the study models before the actual RLAN deployment, with a view to determine the actual interference level observed and compare it with calculated results of studies;

- To measure and control, including on the long term, the aggregate interference into FSS space stations when RLAN deployment is developing.

Preliminary conclusions on the airborne measurements do not allow at this stage characterisation of the interference environment and do not allow quantitative conclusions to be drawn. Additional measurements campaigns from airplanes (including in other geographical areas) would be necessary before being able to draw any conclusions from such measurements.

Furthermore, the following issues should be investigated and clarified on possible future airborne measurements:

- Measurements are made locally at low altitude (few kms) whereas FSS space stations cover wide geographic areas and operate from the GSO orbit in space.

- It is not clear how measurements at 2.4 GHz (for which the reference point is not clear, is it less congested than expected?) can help characterise the interference environment at 5 GHz, especially since measurements at 5 GHz are close the noise floor.

- Measurements should be made during busy hours.

- It is not clear how space and time dynamics of an airplane is comparable with the case of GSO satellites (which are seen from the Earth as static in the sky).

- These measurements have considered two elevation angles (30 and 90 degrees). It would seem useful to specifically conduct measurements at elevation and azimuths corresponding to the GSO orbit, since the GSO orbit as seen from the Earth is a particular path in the sky, from 0 degree elevation angle at horizon for the most western and eastern azimuths to about 48 degrees elevation angle at south azimuths, from London.

#### 5.7.2.2	Potential mitigation techniques for WAS/RLAN

It can be noted that the characteristics of RLAN systems do not provide sufficient details on potential mitigation techniques, despite the fact that previous studies concluded on the need of such mitigation techniques. It is understood that further specific sharing studies WAS/RLAN and FSS will be conducted as part of the AI 1.16 work plan; presumably, these studies will conclude on the need of adequate mitigation techniques. See, for example, the conclusions reached by JTG at the previous study cycle for similar sharing studies (Example: Doc. 2357).

For protection of the FSS, two possible mitigation techniques can be identified:

°	Limitation of RLAN deployment to indoor only.

°	Placing a limit on the maximum eirp of RLAN APs. The appropriate value can be determined through additional studies for RLAN under WRC-19 agenda item 1.16 (noting a value of 10 to 15 dBm/20 MHz has been determined by the ITU in the context of studies between IMT and the FSS, with an interference configuration similar to the RLAN case, see Report ITU-R S.2367).

These two mitigation techniques seem relatively simple to implement but are sometimes difficult to enforce for the RLAN type of equipment (e.g. indoor only, or non-compliant equipment). The impact of such mitigation techniques on the RLAN deployment and operations (e.g. RLAN eirp distribution and the resulting limitations on data rates possible), should be studied.

#### 5.7.2.3	Additional compatibility studies in adjacent bands

Compatibility studies between RLAN operating in any of the sub-bands within 5 725-5 925 MHz and the FSS operating in adjacent sub-bands are necessary.

#### 5.7.2.4	Protection of FSS

The protection criterion for FSS is to accept an increase of the equivalent temperature greater than x% due to interference signals on the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.: ΔT/T≤ x%. This criterion is applicable for interference scenarios when the interfering signal is time invariant, which is assumed the case for the aggregate interference levels created by WAS/RLAN in this uplink satellite band, because coverage areas are large (hemispheric or global) and there is no mobility aspect due to the fact that main interference is from RLAN access points which are fixed.

The values of x is 6%, generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment, and x=1%, generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme should be considered where interference from RLANs is limited to half of the ΔT/T= 6% criterion i.e. the ΔT/T objective is reduced to a value of 3%. In addition, geographic apportionment could also be applied depending on the satellite’s coverage and the assumptions for the RLAN deployment.

## 5.8	Consideration of the cross bands sharing and compatibility issues

# 6	Conclusions of sharing and compatibility studies per frequency range and per service

## 6.1	General considerations

## 6.2	Sharing and compatibility results in the band 5 150-5 350 MHz

## 6.5	Sharing and compatibility results in the band 5 350-5 470 MHz

[RUS]

The compatibility study results of RLAN with airborne Radars of the aeronautical radionavigation and radiolocation services show that the protection distances from several hundreds km up to line-of-sight distance for outdoor RLAN are required. Consideration of potential multi-source interference from RLAN transmitters show that for protection of airborne Radar receivers the required protection distance is equal to the line-of-sight distance in spite of RLAN transmitter bandwidth. It allows to conclude that increase of RLAN bandwidth cannot be considered as the efficient method of reducing the interference to airborne Radars.

The required protection distances range from several tens km to several hundreds km depending on RLAN transmitter bandwidth to ensure protection of airborne radars from single RLAN. Consideration of multi-source interference shows that the protection distances equal to line-of-sight distance are also required for protection of airborne Radars from indoor RLAN.

In case of interference to ground-based radiolocation Radars the protection distances from single-source interference range from several tens km for outdoor RLAN and indoor RLAN as well. Consideration of multi-source interference results in additional increase of the required protection distance subject to the RLAN transmitters’ density and the considered Radar operational characteristics.

Analysis of the estimated results show that compatibility of RLAN with Radars operating in this frequency band will be extremely difficult. Therefore it is required to develop new efficient interference mitigation techniques. Additional fading in propagation in the walls and increase of the RLAN bandwidth cannot be considered as effective interference mitigation techniques.

[ESA/EUMETSAT]

Additional studies performed between WAS/RLAN and EESS (active) confirms that, under all scenarios, RLAN deployment in the 5 GHz range would create large interference to altimeters (up to 26.6 dB) and scatterometers (up to 20.9 dB), hence in the same order of magnitude than for the CSAR sensor (up to 30.4 dB), considered during previous study period.

All together, these studies confirm the previous findings that sharing between RLAN and EESS (active) in the 5350-5470 MHz band may only be feasible if additional RLAN mitigation measures are implemented (see also Recognising a) of Resolution 239 (WRC-15)).

Consistently with Resolution 239 (WRC-15), the work in this frequency band should hence concentrate on studying whether any additional mitigation techniques beyond those analysed in the previous studies would provide coexistence between WAS/RLAN systems and EESS (active) in the in the 5 350-5 470 MHz band.

## 6.6	Sharing and compatibility results in the band 5 725-5 850 MHz

[RUS]

The required protection distances from single-source interference range from several tens km for outdoor RLAN and indoor RLAN as well to ensure protection of ground-based radiolocation radars. Consideration of multi-source interference results in additional increase of the required protection distance subject to the RLAN transmitters’ density and the considered Radar operational characteristics.

Analysis of the estimated results show that compatibility of RLAN with Radars operating in this frequency band will be extremely difficult. Therefore it is required to develop new efficient mitigation techniques of interference caused by RLAN. Additional fading in propagation in the walls and increase of the RLAN bandwidth cannot be considered as effective interference mitigation techniques.

[LUX]

The general conclusions from previous studies conducted by the CEPT (example: Report 57) were that it was not possible at this time to specify any appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the band 5 725-5 925 MHz while ensuring relevant protection of FSS services in this band.

Considering studies conducted by CEPT (and reported in the SE244 Report), provisional conclusions are derived:

−	Sharing between WAS/RLAN and FSS is not possible. Specific mitigation techniques needs be investigated.

−	Empirical methods to estimate the interference levels from aggregate transmissions from WAS/RLAN will require further studies and analysis (including possibly other the scenarios of interference representing closer the cumulative interference from WAS/RLAN into FSS receiver.

−	Before considering the introduction of RLAN in any of the sub-bands within 5 725-5 925 MHz specific compatibility analysis between RLAN and the FSS operating in adjacent sub-bands are necessary.

## 6.7	Sharing and compatibility results in the band 5 850-5 925 MHz

[LUX]

The general conclusions from previous studies conducted by the CEPT (example: Report 57) were that it was not possible at this time to specify any appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the band 5 725-5 925 MHz while ensuring relevant protection of FSS services in this band.

Considering studies conducted by CEPT (and reported in the SE244 Report), provisional conclusions are derived:

−	Sharing between WAS/RLAN and FSS is not possible. Specific mitigation techniques needs be investigated.

−	Empirical methods to estimate the interference levels from aggregate transmissions from WAS/RLAN will require further studies and analysis (including possibly other the scenarios of interference representing closer the cumulative interference from WAS/RLAN into FSS receiver.

−	Before considering the introduction of RLAN in any of the sub-bands within 5 725-5 925 MHz specific compatibility analysis between RLAN and the FSS operating in adjacent sub-bands are necessary.

## 6.8	Cross bands sharing and compatibility issues

[Editor’s note: The following input contributions contain elements relevant to the further development of this working document and are therefore carried forward to the next meeting of WP 5A. These documents will be considered in detail at the next meeting of WP 5A and relevant parts will then be inserted into this working document as appropriate:

[7C]

ANNEX 1

List of current and planned EESS (active) systems in the 5 GHz range (non-exhaustive)

Note: This table has been derived to the best knowledge of the delegates attending the Space Frequency Coordination Group (SFCG) and WP 7C and should hence only be used as informative document since no guarantee can be offered that it fully covers all the existing and planned missions with sensors operating in the 5 GHz range.

| Administration | Satellite | Sensor type | # Sats | Apogee/ Perigee (km/km) | Inclination (°) | Sat. Nb | Date of launch (Month and year) | Mission Duration | RAAN (°) | Argument of perigee (°) | True Anomaly (°) | EPOCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canada | RADARSAT-2C | SAR | 1 | 789 | 98.6 | 1 | 12/2007 | 7 years (design life) Still operational | 173.5 | 90.0 | 38.8 | 15166.23953944 |
|  | RADARSAT-3B | SAR | 6 | 617/586 | 97.7 | 1 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | 120° separation | Not launched |
|  |  |  |  |  |  | 2 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 |  | Not launched |
|  |  |  |  |  |  | 3 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 |  | Not launched |
|  |  |  |  |  |  | 4 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
|  |  |  |  |  |  | 5 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
|  |  |  |  |  |  | 6 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
| China | HY-2 | Altimeter and scatterometer | 1 | 963 | 99.34 | 1 | 08/2011 | 3 years (design life) Still operational | 253.7 | 54.1 | 306 | 17043.685370 |
|  | FY-3-A | Scatterometer | 1 | 854/818 | 98.75 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1 | Planned 11/2018 | no limit in time | 314.17 | 0 | 0 | Not launched |
| France USA | SWOT | Altimeter | 1 | 890.582 (Note 1) | 78 | 1 | Planned 10/2020 | 3 years (design lifetime) | 22.36 | 90.0 | 270.12 (LAN) | Not launched |
| France EUMETSAT USA | JASON2/OSTM /USOCEAN (Note 2) | Altimeter | 1 | 1336 | 66 | 1 | 06/2008 | 6-10 years | 306.05 | 266.33 | 156.34 | 2016 272.15638603 |
|  | JASON3/OSTM /USOCEAN | Altimeter | 1 | 1336 | 66 | 1 | 01/2016 | 6-10 years | 305.6 | 265.85 | 94.16 | 2016 272.53411538 |
| ESA | SENTINEL-1 | SAR | 4 | 701.2/684.5 | 98.183 | 1 | 04/2014 | Up to 12 years | 263.78 | 80.27 | 279.86 | 14/09/2016 04 :33 :48 |
|  |  |  |  |  |  | 2 | 04/2016 | Up to 12 years | 263.30 | 80.37 | 279.77 | 13/09/2016 21 :08 :45 |
|  |  |  |  |  |  | 3 | Planned 2021 | Up to 12 years | 263.XX | 80.XX | TBD | Not launched |
|  |  |  |  |  |  | 4 | Planned 2022 | Up to 12 years | 263.XX | 80.XX | TBD | Not launched |
|  | SENTINEL-3 | Altimeter | 4 | 808.1/791.5 | 98.6 | 1 | 02/2016 | Up to 12 years | 323.58 | 105.7 | 254.43 | 14/09/2016 04 :45 :07 |
|  |  |  |  |  |  | 2 | Planned 2017 | Up to 12 years | 323.XX | 105.XX | Expected 120° from Satellite #1 | Not launched |
|  |  |  |  |  |  | 3 | Planned 2023 | Up to 12 years | 323.XX | 105.XX | TBD | Not launched |
|  |  |  |  |  |  | 4 | Planned 2024 | Up to 12 years | 323.XX | 105.XX | TBD | Not launched |
| EUMETSAT | SENTINEL-6 (Jason-CS) | Altimeter | 2 | 1336 | 66 | 1 | Planned 2020 | 6-10 years | 306 | 90 | 180° separation | Not launched |
|  |  |  |  |  |  | 2 | Planned 2025 | 6-10 years | 306 | 90 |  | Not launched |
|  | ASCAT | Scatterometer | 3 | 832 | 98.7 | 1 | 10/2006 | 10-14 years | 330.19 | 157.1 | 203.07 (mean anomaly) | 2016 272.71848113 |
|  |  |  |  |  |  | 2 | 09/2012 | 10-14 years | 330.65 | 157.03 | 320.85 (mean anomaly) | 2016 272.63852887 |
|  |  |  |  |  |  | 3 | Planned 2018 | 10-14 years | 330.XX | 157.XX | TBD | Not launched |
|  | SCA | Scatterometer | 3 | 832 | 98.7 | 1 | Planned 2021 | 10-14 years | 62.4731+0.98564735 x (Nb Julian days from 1/1/2000) | 90 | 120° separation | Not launched |
|  |  |  |  |  |  | 2 | Planned 2023 | 10-14 years |  | 90 |  | Not launched |
|  |  |  |  |  |  | 3 | Planned 2029 | 10-14 years |  | 90 |  | Not launched |
| India | RISAT | SAR | 1 | 536.38 | 97.6 | 1 | TBD | TBD | TBD | TBD | TBD | TBD |
| Russia |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MINI-RSA (on-board ISS) | SAR | 1 | 400.6/409.6 | 51.64 | 1 | Planned 4Q 2017 | Up to 7 years | 258.95 | 17.78 | 124.40 | 29.09.16 07:56:33 GMT+2 |
|  | GEO-IK-2 (Note 3) | Altimeter | 3 | 1000 | 99.4 | 1 | 06/2016 | no limit in time | TBD | TBD | TBD | TBD |
|  |  |  |  |  |  | 2 | Planned 2017 | no limit in time | TBD | TBD | TBD | TBD |
|  |  |  |  |  |  | 3 | TBD | no limit in time | TBD | TBD | TBD | TBD |

| TOTAL | Altimeters | 13 |
| --- | --- | --- |
|  | Scatterometers | 8 |
|  | SAR | 14 |
|  | TOTAL | 35 |

Note 1: Only 21-day repeat science orbit (890.6 km altitude) is shown; the 1-day repeat “fast-sampling orbit” has a 857.2 km altitude orbit.

Note 2 : starting the week of 3 October 2016 Jason-2 will be moved to an 'interleaved' orbit, ~160° different from Jason-3, to split the ground track and separate them by about 1/2 of the repeat cycle (5 of the ~10 days). The inclination and altitude will be the same, but not the phase.

Note 3 : Satellites of the constellation use  different orbital planes, which are separated by 120°.

[7C]

ANNEX 2

Detailed characteristics of EESS (active) systems

Several types of synthetic aperture radars (SAR), altimeters, and scatterometers are identified as EESS (active) missions in the frequency bands between 5 150 MHz and 5 925 MHz to be studied under agenda item 1.16.  Typical characteristics for EESS (active) sensors are shown in Table 1 for frequency overlaps that should be considered in sharing studies within these bands. The actual mission names as well the generic names which will be used in the final recommendation are provided in the table. It should be noted that the service area for most of these active sensors is global.

Table 22

Typical parameters of EESS (active) sensors in the 5 250-5 570 MHz band

| Mission/Sensor | SAR-D1 (Sentinel- 1 (CSAR)) | SAR-D2 (ASAR) | SAR-D3 (RISAT-1) | SAR-D4 (Radarsat-2) | SAR-D5 (Radarsat-3 (RCM)) | SAR-D6 (Radarsat Next Generation (RNG)) |
| --- | --- | --- | --- | --- | --- | --- |
| Sensor type | SAR | SAR | SAR | SAR | SAR | SAR |
| Type of orbit | Circular SSO | SSO, circular | SSO | Near circular | Near circular | Near circular |
| Altitude, km | 693 | 764 | 536 | 792-813 | 586.9-615.2 | 586.9-615.2 |
| Inclination, deg | 98.18 | 98.6 | 97 | 98.6 | 97.74 | 97.74 |
| Ascending Node LST | 18:00/6:00 | 10:30 | 6:00 | 6:00 | 6:00 | 6:00 |
| Repeat period, days | 12 | 35 | 13 | 24 | 12 | 12 |
| Antenna type | Phase array | Phase array | Planar Phased Array | Planar Phased Array | Planar Phased Array | Planar Phased Array |
| Number of beams | 1 | 1 | 1 | 1 | 1 | 1 |
| Antenna Size/diameter | 12.3 m × 0.8 m | 10 m × 1.3 m | 10 m × 3 m | 15 m × 1.5 m | 6.88 m × .37 m | 6.88 m × 1.37 m |
| Antenna Pk Xmt Gain, dBi | 43.5 to 45.3 | 40 to 45 | 35 | 49 | 453 | 453 |
| Antenna Pk Rcv Gain, dBi | 43.5 to 44.8 | 40 to 45 | 35 | 493 | 453 | 453 |
| Polarization | V, H | H, V | Linear H,V | HH, HV, VH, VV | HH, VV, HV, VH, CH, CV | HH, VV, HV, VH, CH, CV |
| Antenna beam look angle, deg | 20-47 | 15-45 | 10-45 | 9-50 | 16-51 | 16-53 |
| Antenna beam azimuth angle, deg | 90 | 90 | 90 | 0 | 0 | 0 |
| Antenna elev. beamwidth, deg | 6 to 8 | 2.5 | 4.6 | 1.88 (for focused beam) | 2.05 (for focused beam) | 2.05 (for focused beam) |
| Antenna az. beamwidth, deg | 0.3 | 0.3 | 1.4 | 0.19 | 0.42 (for focused beam) | 0.42 (for focused beam) |
| Swath width (km) | 20-410 | 10-405 | 10-225 | 18-500 | 20-500 | 20-500 |
| RF center frequency, MHz | 5 405 | 5 331 | 5 350 | 5 405 | 5 405 | 5 405 |
| RF bandwidth, MHz | 100 | 16 | 18.75-75 | 11.6, 17.3, 30, 50, 100 | 14-100 | 14-300 |
| Transmit Pk pwr, W | 4 140 | 2 500 | 4 000 | 2 400 or 3 700 | 1 490 | 1 990 |
| Transmit Ave. pwr, W | 370 | 200 | 260 | 300 | 180 | 240 |
| Pulsewidth, μsec | 5 to 53 | 16 to 41 | 2 0 | 21,42 | 10 to 50 | 10 to 50 |
| Pulse Repetition Frequency (PRF) Hz | 1 450-2 000 | 1 600-2 100 | 3 250 | 1 000-2 800 | 2 000-7 000 | 2 000-7 000 |
| Chirp rate, MHz/μsec | 0.34-3.75 | 0.39 | 0.937-3.75 | 0.27 to 2.38 | 0.14 to 10 | 0.14 to 10 |
| Transmit duty cycle, % | 0.5-9.0 depending on ops mode | 8.61 | 6.5 | Variable, max 8% | Variable, max 12% | Variable, max 12% |
| e.i.r.p. ave, dBW | 70 (for 9% duty cycle) | 68.0 | 68 | Approx. 73 | 67.67 | 69.0 |
| e.i.r.p. peak, dBW | 80 | 78.0 | 71.0 | 83.5 | 76.7 | 78.0 |
| System Noise figure, dB | 3.2 | 4.5 | 5.8 | 6 | 6 | 6 |

| Mission/Sensor | ALT-D1 (JASON-2/3 SSALT, POSEIDON-3/3B) (Note 3) | ALT-D2 (Sentinel 3 SRAL) (Note 1 & 3) | ALT-D3 (HY-2A) (Note 3) | ALT-D4 (Sentinel-6 POSEIDON-4) (Notes 1, 2 & 3) | ALT-D5 (SWOT) (Note 3) |
| --- | --- | --- | --- | --- | --- |
| Sensor type | Altimeter | Altimeter | Altimeter | Altimeter | Altimeter |
| Type of orbit | NSS | Circular, SSO | SSO | NSS | NSS |
| Altitude, km | 1336 | 814 | 963 | 1 336 | 890 |
| Inclination, deg | 66 | 98.65 | 99.3 | 66 | 78 |
| Ascending Node LST | NSS | 22:00 | 06:00 | NSS | NSS |
| Repeat period, days | 10 | 27 | 14 | 10 | 21 |
| Antenna type | Parabolic reflector | Parabolic reflector | Parabolic reflector | Parabolic reflector | Parabolic reflector |
| Number of beams | 1 | 1 | 1 | 1 | 1 |
| Antenna Size/diameter | 1.2 m | 1.2 m | 1.4 m | 1.2 m | 1.2 m |
| Antenna Pk Xmt Gain, dBi | 32 | 34.5 | 35; 43 | 33.5 | 32 |
| Antenna Pk Rcv Gain, dBi | 32 | 34.5 | 35; 43 | 33.5 | 32 |
| Polarization | linear | linear | linear VV | linear | linear |
| Azimuth scan rate, rpm | 0 | 0 | 0 | 0 | 0 |
| Antenna beam look angle, deg | 0 | 0 | 0 | 0 | 0 |
| Antenna beam azimuth angle, deg | 0 | 0 | 0 | 0 | 0 |
| Antenna elev. beamwidth, deg | 3.4 | 3.4 | 2.3 | 3.4 | 3.4 |
| Antenna az. beamwidth, deg | 3.4 | 3.4 | 2.3 | 3.4 | 3.4 |
| Swath width (km) | 79.4 | 48.4 | 38.7 | 97 | 52.9 |
| RF center frequency, MHz | 5 300 | 5 410 | 5 250 | 5 410 | 5 300 |
| RF bandwidth, MHz | 100, 320 | 320 | 160 | 320 | 100, 320 |
| Transmit Pk pwr, W | 17 | 32 | 20 | 25 | 17 |
| Transmit Ave. pwr, W | 0.51 | 0.4 (LRM), 0.25 (SAR) | 8.2 | <2 | 0.51 |
| Pulsewidth, μsec | 106.0 | 49 | 102.4 | 32 | 106.0 |
| Pulse Repetition Frequency (PRF) Hz | 300 | 275 (LRM), 157 (SAR) | 670 | 2 060-9 280 | 300 |
| Chirp rate, MHz/μsec | 0.9, 3.0 | 6.5 | 1.56 | 9.69 | 0.9, 3.0 |
| Transmit duty cycle, % | 3.1 | 1.5 (LRM), 0.7 (SAR) | 40.96 | 30 | 3.1 |
| e.i.r.p. ave, dBW | 29.5 | 30.8 (LRM), 28.4 (SAR) | 44.1 | 36.51 | 29.5 |
| e.i.r.p. peak, dBW | 44.8 | 49.5 | 48 | 47.47 | 44.8 |
| System Noise figure, dB | 4.45 | 3.8 | 3.5 | 3.5 | 4.45 |

Note 1 − Dual frequency radar altimeter (C/Ku Band) which performs measurements either in low resolution mode (LRM) or synthetic aperture radar mode (Nadir-SAR). LRM mode is the conventional altimeter pulse limited mode with interleaved C/Ku Band pulses, while Nadir-SAR mode is the high along track resolution mode based on SAR processing. The system is a two-satellite constellation.

Note 2 – The Poseidon-4 altimeter of Sentinel-6 is an evolution of the Poseidon-3/3B, SIRAL and SRAL altimeters of the Jason-3, CryoSat-2 and Sentinel-3 satellites, respectively.

Note 3 - It should be noted that EESS (active) altimeters may experience the influence from land backscatter (main lobe or side lobes), oceanic surface having anomalous profiles or other unwanted reflections (mispointing errors for example). It is therefore usually agreed to use the value of 2 times the 3 dB beamwidth as a typical value for the reflected area.

Table 1 (continued)

| Mission/Sensor | SCAT-D1 (Metop-A,B,C ASCAT) | SCAT-D2 (Metop-SG SCA) |
| --- | --- | --- |
| Sensor type | Scatterometer | Scatterometer |
| Type of orbit | SSO | SSO |
| Altitude, km | 832 | 832 |
| Inclination, deg | 98.7 | 98.7 |
| Ascending Node LST | 21:30 | 21:30 |
| Repeat period, days | 29 | 29 |
| Antenna type | Six fan beam-antennas (slotted WG arrays) | Six fan beam-antennas (slotted WG arrays) |
| Number of beams | 6 | 6 |
| Antenna Size/diameter | 2.251 m x 0.337 m (mid), 3.003 m x 0.253 m (side) | 2.757 m x 0.315 m (mid), 3.02 m x 0.315 m (side) |
| Antenna Pk Xmt Gain, dBi | 24-32 | 23-31 |
| Antenna Pk Rcv Gain, dBi | 24-32 | 23-31 |
| Polarization | linear VV for all beams | linear VV for all 6 beams + VH/HV and linear HH for the 2 mid-beams |
| Azimuth scan rate, rpm | 0 | 0 |
| Antenna beam look angle, deg | 22-45.6 (mid beams) 29.5-53.4 (side beams) | 17.5-45.5 (mid beams) 24-54 (side beams) |
| Antenna beam azimuth angle, deg | 45, 90, 135, 225, 270, 315 | 45, 90, 135, 225, 270, 315 |
| Antenna elev. beamwidth, deg | 23.6 (mid beams) 23.9 (side beams) | 28 (mid beams) 30 (side beams) |
| Antenna az. beamwidth, deg | 1.5 (mid beams) 1.2 (side beams) | 1.3 |
| Swath width (km) | 550 on each side of the orbit plane | 665 on each side of the orbit plane |
| RF center frequency, MHz | 5 255 | 5 355 |
| RF bandwidth, MHz | 0.5 | 2 |
| Transmit Pk pwr, W | 120 | 2 512 |
| Transmit Ave. pwr, W | 29 (mid beams) 36.5 (side beams) | 92 |
| Pulsewidth, μsec | 10 000 | 1 000 |
| Pulse Repetition Frequency (PRF) Hz | 28.259 | 32 |
| Chirp rate, MHz/μsec | 0.00002 | 0.00002 |
| Transmit duty cycle, % | 28.29 | 3.68 |
| e.i.r.p. ave, dBW | 39 - 47 | 42-50 |
| e.i.r.p. peak, dBW | 53 | 57-65 |
| System Noise figure, dB | 3.0 | 3.5 |

[ESA]

ANNEX 3

Sharing studies between RLANs and EESS (active) Altimeter sensor

(Sentinel-3 SRAL sensor)

# 1	Introduction/Background

The present Annex addresses sharing between WAS/RLAN and EESS (active) altimeters sensors, based on the ESA Sentinel-3 SRAL sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors and given in Document 4-5-6-7/664 (ESA).

# 2	Technical characteristics

## 2.1	EESS (active)

### 2.1.1	Parameters

The EESS (active) parameters and interference criteria used in the present studies are those provided by WP 7C to WP 5A in their liaison statement in Document 5A/38.

Table 1 gives the technical parameters for the SRAL sensor on board Sentinel-3 satellites being developed by ESA for the Copernicus program of the European Commission.

Table 1

| Parameter | Sentinel-3 SRAL |
| --- | --- |
| Sensor type | ALTIMETER |
| Orbital altitude (km) | 800 |
| Orbital inclination (degrees) | 98.65 |
| RF centre frequency (MHz) | 5 410 |
| Peak radiated power (W) | 32 (at ant input) |
| Polarisation | Linear |
| Antenna type | Parabolic reflector 1.2 m |
| Antenna gain (dBi) | 34.5 |
| Antenna pattern steering capability | No |
| Antenna pattern | Based on F.699 |
| Antenna orientation (degrees from nadir) | Nadir (altimeter) |
| Receiver noise figure (dB) | 3.8 |
| Pulse/Receiver bandwidth (MHz) | 320 |
| Noise power (dBW) | –115 |
| Service area | Global |
| Footprint (km2) | 1 840 |

Concerning the polarisation, it is to be noted that Sentinel-3 SRAL makes use of a dual linearly polarised antenna. Therefore no polarisation discrimination advantage can be taken into account.

The antenna pattern used for SRAL is based on Recommendation ITU-R F.699.

### 2.1.2	Protection criteria

The relevant protection criteria is given in Table 2, taken from Recommendation ITU-R RS.1166-4. Even if RLANs are nomadic/mobile by nature, their very high density implies that the interference will be systematic. The relevant percentage of data availability, corresponding to the percentage of time, is therefore 99% (see Document 5A/38)

Table 2

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Altimeter | 4% degradation in height noise | –3 | 99 | 95 |

For the SRAL instrument, the protection criteria calculated over a 320 MHz bandwidth is 
–118.1 dBW (-88.1 dBm) not to be exceeded more than 1% of the time.

This criteria is applied over data acquisition periods of time when the sensor is operating over the measurement area of interest (as per Recommendation ITU-R RS.1166-4).

## 2.2	Mobile service (WAS/RLAN)

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

–	Bandwidth factor:

The bandwidth factor (BWF) used in this document is derived taking into account the positioning of an EESS (active) 100 MHz bandwidth over the RLAN raster/channel plan, as shown below.

On this basis, the following bandwidth factors are considered (see details in Annex 2):

•	160 MHz (15% of RLANs): 1 channel overlaps fully overlaps the EESS bandwidth and 2 overlap by 80 MHz.

–	BWF = 0 dB for 1/3 of cases

–	BWF = 10 *log(80/160) = - 3 dB for 2/3 of cases

This represent an average BWF = -1.76 dB

•	80 MHz (50% of RLANS): 4 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

•	40 MHz (25% of RLANS): 8 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

•	20 MHz (10% of RLANS): 16 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

Overall, considering all bandwidth, this represent an average BWF = - 0.4 dB

–	Propagation conditions:

•	building attenuation with a Gaussian distribution (17 dB + 7 dB) truncated at 1 dB, as described in the figure below:

Note: when used to calculate aggregate interference from multiple sources (as in the present case), the impact of this distribution is similar to the one leading from a 12 dB average attenuation.

•	Angular clutter loss model from Recommendation ITU-R P.452 associated with RLAN heights distributions and specific parameters for Urban, Suburban and Rural environments. The antenna heights are randomly selected using a uniform probability distribution from the set of floor heights at 3 meter steps. It should be noted that due to the EESS (active) geometry (Nadir) this model leads to no attenuation.

Antenna height

| RLAN deployment region | Antenna height (metres) |
| --- | --- |
| Urban | 1.5 to 28.5 |
| Suburban | 1.5, 4.5 |
| Rural | 1.5, 4.5 |

–	Antenna gain/discrimination (Omnidirectional in azimuth for all scenarios)

•	Option A1: Omnidirectional in elevation with 0 dBi gain.

•	Option A3: An average 4 dB antenna discrimination is applied to the e.i.r.p. level distribution above in the direction of the satellite Omnidirectional in elevation with 0 dBi gain.

Note: since Option A3 is proposing a fixed discrimination of 4 dB, corresponding results can therefore be extrapolated by shifting by 4 dB the results obtained with Option A1 (0 dBi).

–	Number of active RLAN:

•	Option D1: 11 279 active devices per 100 MHz channel per 5.25 million inhabitants (so-called “Sim City” with 5.25 M inhabitants).

•	Option D2: from 0.004 (D2-low) to 0.04 (D2-high) per 100 MHz channel per inhabitant.

Extrapolation of these numbers from a 100 MHz bandwidth to a 320 MHz bandwidth is detailed in Annex 2 and leads to the following figures:

•	Option D1: 25297 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0048 RLAN per inhabitant.

•	Option D2-Low: 47103 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.00897 RLAN per inhabitant.

•	Option D2-High: 471021 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0897 RLAN per inhabitant.

These factors lead to the following number of active RLAN to be considered over the French territory (with a population of 66 M inh.), Dutch territory (with a population of 16.8 M inh.) and the UK territory (with a population of 63.3 M inh.):

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 320 MHz) | Nb of active RLAN over NL (in 320 MHz) | Nb of active RLAN over UK (in 320 MHz) |
| --- | --- | --- | --- | --- |
| Option D1 | 0.0048 per inh. | 318 019 | 80 950 | 305 009 |
| Option D2 | Low (0.00897 per inh.) | 592 142 | 150 727 | 567 918 |
|  | High (0.0897 per inh.) | 5 921 422 | 1 507 271 | 5 679 182 |

Detailed deployment assumptions are described in the analysis sections.

# 3	Analysis

Analyses based on static and dynamic methodologies have been used to address the compatibility between WAS/RLAN and EESS (active) altimeter in the 5 GHz range:

–	Section 3.1: static analyses.

–	Section 3.2: dynamic analyses based on existing population densities.

–	Section 3.3: consideration of some additional parameters proposed in PDN Report ITU-R RS.[EESS RLAN 5 GHz].

## 3.1	Static analyses

### 3.1.1	Single entry static analysis

The following Table 4 provides calculation of the impact of 1 single outdoor RLAN on EESS (active) sensors described in Table 1 above.

Table 4

| Parameter | Sentinel-3 SRAL |
| --- | --- |
| Frequency (MHz) | 5 410 |
| Orbital altitude (km) | 800 |
| Off Nadir Angle (°) | 0 |
| Slant path distance (km) | 800 |
| Free Space losses (dB) | 165.2 |
| EESS antenna gain (dBi) | 34.5 |
| EESS protection criteria (dBm/320 MHz) | –88.1 |
| RLAN EIRP (dBm) | 23 |
| Interference from 1 outdoor RLAN (dBm) | –107.7 |
| Margin for 1 outdoor RLAN (dB) | 19.6 |
| Nb of outdoor RLAN in the EESS footprint to reach the protection criteria | 91 |

This calculation shows that 91 outdoor RLANs within the 1840 km² EESS (active) footprint transmitting with the full e.i.r.p. of 200 mW are sufficient to interfere with an EESS (active) system.

This represents a maximum density of less than 0.05 RLAN / km².

### 3.1.2	Static analysis based on EESS (active) footprint shape

The following analysis provides calculation of the aggregate RLAN impact on EESS (active) considering the so-called ‘Sim City” (circular based) with the footprint shape of the EESS (active) system (with a radius of 24.2 km), as shown on the Figure 1 below.

Figure 1

RLAN deployment and EESS (active) footprint

In this case, the percentage of RLAN within the EESS (active) footprint (as on the right figure) can be calculated as in Table 5 below.

Table 5

|  | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 628 | 100% |
| Rural | 30 | 2 121 | 1133 | 53% |

To consider potential interference from RLAN deployment on EESS (active) sensor, one can therefore use these percentages to determine the total number of RLAN in the EESS (active) footprint, for both density Option D1 (total of 25297 RLAN within the overall city for 5.25 M inhabitants), the low edge of Density Option D2 (47102 RLAN) and the upper edge of Density Option D2 (471022 RLAN) as given in Table 6 below.

Table 6

|  | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 25297 | 47102 | 471022 |
| Urban | 100% | 35.5% | 8978 | 16718 | 167176 |
| Suburban | 100% | 53.3% | 13488 | 25115 | 251148 |
| Rural | 53% | 11.2% | 1512 | 2816 | 28159 |
| Total Nb of RLAN in EESS footprint |  |  | 23979 | 44648 | 446482 |

On this basis, the following Table 7 provides calculation of interference for the “EESS (active) footprint shape scenario” for the Sentinel-3 SRAL.

Table 7

| Parameter | Sentinel-3 SRAL with RLAN Density Option D1 | Sentinel-3 SRAL with RLAN Density Option D2-low | Sentinel-3 SRAL with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5410 | 5410 | 5410 |
| Orbital altitude (km) | 800 | 800 | 800 |
| Off Nadir Angle (°) | 0 | 0 | 0 |
| Slant path distance (km) | 800 | 800 | 800 |
| Free Space losses (dB) | 165.2 | 165.2 | 165.2 |
| EESS antenna gain (dBi) | 34.5 | 34.5 | 34.5 |
| Average RLAN EIRP (dBm) (including average BWF) | 18.6 | 18.6 | 18.6 |
| Interference from 1 RLAN (dBm) | -112.1 | -112.1 | -112.1 |
| Nb of RLAN (see table 6 above) | 23979 | 44648 | 446482 |
| Nb of outdoor RLAN (5%) | 1199 (=30.8 dB) | 2232 (=33.5 dB) | 22324 (=43.5 dB) |
| Nb of indoor RLAN | 22780 (=43.6 dB) | 42416 (=46.3 dB) | 424158 (=56.3 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/320 MHz) | -81.3 | -78.6 | -68.6 |
| Interference from indoor RLAN (dBm/320 MHz) | -80.5 | -77.8 | -67.8 |
| TOTAL INTERFERENCE (dBm/320 MHz) | -77.9 | -75.2 | -65.2 |
| EESS protection criteria (dBm/320 MHz) | -88.1 | -88.1 | -88.1 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 10.2 | 12.9 | 22.9 |

This Table shows that, when considering the “footprint shape” scenario, (with RLAN antenna Option A1), the RLAN deployment exceeds the EESS (active) protection criteria from 10.2 dB (with Density Option D1) to 22.9 dB (with Density Option D2-high).

As a summary, the following Table 8 provides the level of interference in excess considering all different RLAN density scenarios and antenna options.

Table 8

Level of interference in excess (static analysis)

| Scenario | Sentinel-3 SRAL |  |
| --- | --- | --- |
| RLAN Antenna | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) |
| JTG Option D1 | 10.2 dB | 6.2 dB |
| JTG Option D2-low | 12.9 dB | 8.9 dB |
| JTG Option D2-high | 22.9 dB | 18.9 dB |

These calculations therefore show that when considering the “footprint shape” scenario, the RLAN deployment largely exceeds the EESS (active) protection up to 22.9 dB and allow to show there is no compatibility between RLAN and EESS (active) altimeters sensors in the 5 GHz range.

## 3.2	Dynamic analyses

### 3.2.1	RLAN deployment

The dynamic analysis have been considered over France (550 000 km² and 66 M inhabitants) on the one hand and over a more restricted area covering the Paris metropolitan area (a square of 10 000 km² and approximately 10.2 M inhabitants) on the other hand.

Some calculations have also been made considering the Netherlands (41 530 km² and 16.8 M inhabitants).

Other simulations have been considered over the UK (244 000 km² and 63.3 M inhabitants) on the one hand and over a more restricted area covering the London metropolitan area (a square of 10 000 km² and approximately 12.9 M inhabitants) on the other hand.

These simulations are properly reflecting the real scenarios, with real population distributions.

Over these areas, different scenarios related to the number of active RLAN were considered as in Table 9 below:

Table 9

Scenarios considered for the dynamic analysis

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Nb of active RLAN over NL (in 320 MHz) | Nb of active RLAN over UK (in 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Option D1 | 0.0048 per inh. | 318 019 | 49 148 | 80 950 | 305 009 | 62 158 |
| Option D2 | Low (0.00897 per inh.) | 592 142 | 91 513 | 150 727 | 567 918 | 115 737 |
|  | High (0.0897 per inh.) | 5 921 422 | 915 129 | 1 507 271 | 5 679 182 | 1 157 369 |

These active RLAN have been deployed following the population densities, as depicted in Figure 2 below.

An EESS (active) measurement area has been defined around France, with an area of about 1 000 000 km² (blue square on Figure 2A), around Paris with an area of 10 000 km² (blue square on Figure 2B), around the Netherlands with an area of about 120 000 km² (blue square on Figure 2C), around the UK, with an area of about 700 000 km² (blue square on Figure 2D) and around London with an area of 10 000 km².

Figure 2a

RLAN deployment and measurement area over France (592 142 RLANs for D2-low)

Figure 2B

RLAN deployment and measurements area over Paris metropolitan (91 513 RLANs for D2-low)

Figure 2C

RLAN deployment and measurements area over the Netherlands (150 727 RLANs for D2-low)

Figure 2D

RLAN deployment and measurements area over the UK (567 918 RLANs for D2-low)

### 3.2.2	Dynamic analyses conditions

Simulations have been run for the SRAL sensor on board Sentinel-3 with a time step of 1 second and for a period of 30 days.

At each step of the simulation (i.e. corresponding to 1 s dynamic of the EESS satellite), the interference to the EESS (active) sensor from each RLAN in visibility is calculated (taking into account the EESS antenna pattern to determine the relative gain), hence leading to an aggregate interference.

The percentage of time of interference is calculated with reference to the measurement area, which means that only the time steps when the sensor antenna boresight is within the blue area are retained for the calculation of the percentage of time of interference.

Then, compiling the aggregate interference over the whole steps of the simulations allows to deriving the interference distribution that will be compared to the EESS (active) protection criteria.

The SRAL sensor has been considered with an antenna pointing at nadir with a payload active 100% of the time.

### 3.2.3	Dynamic analyses results over France

On this basis, Figure 3 gives the cumulative distribution function of interference for deployment of 592 142 RLANs over France (Option D2-low) and antenna Option A1.

Figure 3

Interference for Sentinel-3 (Over France – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 7.1 dB (–110.9 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 4 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 4

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D2-low)

In addition, Figure 5 gives the cumulative distribution function of interference for deployment of 318 019 RLANs over France (Option D1) and antenna Option A1.

Figure 5

Interference for Sentinel-3 (Over France – options A1 and D1)

It can be seen that the EESS (active) protection criterion is exceeded by 3.4 dB (–114.6 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 10% of the time. The situation is also depicted on Figure 6 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 6

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D1)

It appears obvious that in these situations, the Sentinel-3 sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 10 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 10

Interference in excess (over France)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over France (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 318 019 | 3.4 dB | -0.6 dB |
| JTG Option D2-low | 0.00897 | 592 142 | 7.1 dB | 3.1 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 921 422 | 17.1 dB | 13.1 dB |

### 3.2.4	Dynamic analyses results over Paris metropolitan

Under the same principle, the following Figure 7 gives the cumulative distribution functions of interference corresponding to a deployment of 592 142 RLANs over France, including 91 513 RLANs over Paris metropolitan area (Option D2-low) and antenna Option A1.

Figure 7

Interference for Sentinel-3 (Over Paris Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 14.2 dB 
(–103.8 dBW). (The interference level corresponding to the protection criteria is exceeded for 100 % of the time.)

Similarly, the following Figure 8 gives the cumulative distribution functions of interference corresponding to a deployment of 318 019 RLANs over France, including 49 148 RLANs over Paris metropolitan area (Option D1) and antenna Option A1.

Figure 8

Interference for Sentinel-3 (Over Paris Metro – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 11.3 dB 
(–106.7 dBW). (The interference level corresponding to the protection criteria is exceeded 100 % of the time.)

Finally, Table 11 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 11

Interference in excess (over Paris metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 49 148 | 11.3 dB | 7.3 dB |
| JTG Option D2-low | 0.00897 | 91 513 | 14.2 dB | 10.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 915 129 | 24.2 dB | 20.2 dB |

### 3.2.5	Dynamic analyses results over the Netherlands

Under the same principle, the following Figure 9 gives the cumulative distribution functions of interference corresponding to a deployment of 150 727 RLANs over the Netherlands (Option D2-low) and antenna Option A1.

Figure 9

Interference for Sentinel-3 (Over the Netherlands – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10 dB 
(–-108 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 50% of the time. The situation is also depicted on Figure 10 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 10

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the Netherlands – options A1 and D2-low)

It appears obvious that in this situation (150 727 active RLAN corresponding to 0.00897 active RLAN per inh.), the Sentinel-3 sensor will be totally ineffective over the Netherlands.

Finally, Table 12 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 12

Interference in excess (over the Netherlands)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the Netherlands (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 80 950 | 7 dB | 3 dB |
| JTG Option D2-low | 0.00897 | 150 727 | 10 dB | 6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 507 270 | 20 dB | 16 dB |

### 3.2.6	Dynamic analyses results over the UK

Figure 11 gives the cumulative distribution function of interference for deployment of 567 918 RLANs over the United Kingdom (Option D2-low) and antenna Option A1.

Figure 11

Interference for Sentinel-3 (Over the UK – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.6 dB (–-107.4 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 12 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 12

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the UK – options A1 and D2-low)

In addition, Figure 13 gives the cumulative distribution function of interference for deployment of 305 009 RLANs over the UK (Option D1) and antenna Option A1.

Figure 13

Interference for Sentinel-3 (Over the UK – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 7.8 dB 
(–110.2 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 20% of the time. The situation is also depicted on Figure 14 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 14

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the UK – JTG options A1 and D1)

It appears obvious that in these situations, the Sentinel-3 sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 13 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 13

Interference in excess (over the UK)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the UK (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 305 009 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.00897 | 567 918 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 679 180 | 20.6 dB | 16.6 dB |

### 3.2.7	Dynamic analyses results over London metropolitan

Figure 15 gives the cumulative distribution functions of interference corresponding to a deployment of 567 918 RLANs over the UK, including 115 737 RLANs over London metropolitan area (Option D2-low) and antenna Option A1.

Figure 15

Interference for Sentinel-3 (Over London Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 16.6 dB 
(–104.1 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for 100% of the time.

Finally, Table 14 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 14

Interference in excess (over London metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 62 158 | 13.6 dB | 9.6 dB |
| JTG Option D2-low | 0.00897 | 115 737 | 16.6 dB | 12.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 157 369 | 26.6 dB | 22.6 dB |

### 3.2.8	Dynamic analyses – Summary of results

The results of the dynamic analysis over France are given below (see Table 10 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over France (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 318 019 | 3.4 dB | -0.6 dB |
| JTG Option D2-low | 0.00897 | 592 142 | 7.1 dB | 3.1 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 921 422 | 17.1 dB | 13.1 dB |

The results of the dynamic analysis over Paris Metropolitan are given below (see Table 11 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 49 148 | 11.3 dB | 7.3 dB |
| JTG Option D2-low | 0.00897 | 91 513 | 14.2 dB | 10.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 915 129 | 24.2 dB | 20.2 dB |

The results of the dynamic analysis over the Netherlands are given below (see Table 12 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the Netherlands (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 80 950 | 7 dB | 3 dB |
| JTG Option D2-low | 0.00897 | 150 727 | 10 dB | 6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 507 270 | 20 dB | 16 dB |

The results of the dynamic analysis over the UK are given below (see Table 13 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the UK (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 305 009 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.00897 | 567 918 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 679 180 | 20.6 dB | 16.6 dB |

The results of the dynamic analysis over London Metropolitan are given below (see Table 14 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 62 158 | 13.6 dB | 9.6 dB |
| JTG Option D2-low | 0.00897 | 115 737 | 16.6 dB | 12.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 157 369 | 26.6 dB | 22.6 dB |

Overall, in all cases, the above dynamic analyses confirm the result of static analyses, presenting interference largely exceeding EESS (active) protection criteria and allow to show that there is no compatibility between RLAN and EESS (active) in the 5 GHz range.

## 3.3	consideration of some additional parameters

In addition to the RLAN parameters given in section 2 above, PDN Report ITU-R RS.[EESS RLAN 5 GHz] mentions different parametric assumptions as follows:

–	Outdoor ratio: consider 2% and 10% in addition to the agreed 5%.

–	Indoor/outdoor attenuation: consider a fixed 17 dB in additional to the agreed “Gaussian 17 dB +-7 dB standard deviation”.

As expressed in Document 4-5-6-7/664, and validated by dynamic simulations, the impact of these parametric parameters can be calculated as follows:

Aresult = 10 log(Or + (1-Or)10(-IOa/10)

where:

Aresult =	resulting attenuation

Or =	Outdoor ratio

IOa =	Indoor/outdoor attenuation.

On this basis, the following Table 21 provides the corresponding results:

Table 21

Calculated attenuations due to the parametric parameters

|  | Outdoor ratio |  |  |
| --- | --- | --- | --- |
| Indoor/outdoor attenuation | 2% | 5% | 10% |
| Gaussian 17 dB + -7 dB (resulting in 12 dB) | 10.9 (-1.3) | 9.6 (0) | 8.0 (+1.6) |
| Fixed 17dB | 14.0 (-4.4) | 11.6 (-2) | 9.3 (+0.3) |

Note: The figures in brackets ( x) represent the difference compared to the JTG
agreed scenario (in Red: 5% Outdoor ratio and “Gaussian 17 dB + -7 dB”)

It can therefore be seen that the potential impact of the parametric assumptions mentioned in JTG ranges from a potential decrease of the interference of 4.4 dB up to an increase by 1.5 dB and will therefore not change the overall negative conclusions in previous sections.

# 4	Summary/Conclusions

Under all scenarios and simulation methodologies (static and dynamic), the analyses show that RLAN deployment in the 5 GHz range would create large interference in the SRAL sensor on board the Sentinel-3 satellite.

The static analyses presented above indicate that:

–	the maximum density of outdoor RLANs (transmitting with the full e.i.r.p. of 200 mW) within the 1840 km² EESS (active) is less than 0.05 RLAN / km² (see section 3.1.1)

–	Depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by 6.2 to 22.9 dB (see section 3.1.2)

The dynamic analyses presented above indicate that, depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by (for 0.0048 to 0.0897 active RLAN per inhabitant, respectively):

•	-0.6 to 17.1 dB (case over France) (see section 3.2.3)

•	7.3. to 24.2 dB (case over Paris metropolitan) (see section 3.2.4)

•	3 to 20 dB (case over the Netherlands) (see section 3.2.5)

•	3.8 to 20.6 dB (case over the UK) (see section 3.2.6)

•	9.6 to 26.6 dB (case over London metropolitan) (see section 3.2.7).

It has to be highlighted that these analyses were not considering a number of assumptions that would further increase these negative margins, such as an additional apportionment factor of the protection criteria (since the band is already shared with terrestrial radars). ESA stresses that this apportionment factor has not been introduced in the analysis given the already large negative results obtained under the assumption that no other services could generate interference to EESS (active).

Further, some assumptions related to RLAN remains unclear or unresolved and could also increase the potential interference to EESS (active). This covers in particular the possibilities given to a single RLAN to make use of multiple channels transmission (by means of either orthogonal transmissions or MIMO technique) or to concatenate multiple small channels to provide wider bandwidth with higher power. Such questioning also relates to other applications than RLAN since opening a band to RLAN, low power and unlicensed by nature, will drive the use of different applications such as SRDs, M2M, … (similarly to the current situation in the 2.4 GHz band) or LAA-LTE. Consideration of these additional applications would need to be taken into account.

Overall, this document demonstrates and confirms that RLANs cannot share with EESS (active) in the 5 GHz range, confirming that sharing between RLAN and EESS (active) in the 5 GHz range may only be feasible if additional RLAN mitigation measures are implemented.

[EUMETSAT]

ANNEX 4

Sharing studies between RLANs and EESS (active) scatterometer sensor
(SCA sensor)

# 1	Introduction/Background

The present Annex addresses sharing between WAS/RLAN and EESS (active) scatterometers sensors, based on the EUMETSAT SCA sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors and given in Document 4-5-6-7/664 (ESA).

# 2	Technical characteristics

## 2.1	EESS (active)

### 2.1.1	Parameters

The EESS (active) parameters and interference criteria used in the present studies are those provided by WP 7C to WP 5A in their liaison statement in Document 5A/38.

Table 1 gives the technical parameters for the SCA sensor on board EPS-SG satellites being developed by EUMETSAT.

Table 1

| Parameter | EPS-SG SCA |
| --- | --- |
| Sensor type | SCATTEROMETER |
| Orbital altitude (km) | 832 |
| Orbital inclination (degrees) | 98.7 |
| RF centre frequency (MHz) | 5 355 |
| Peak radiated power (W) | 2512 peak (92 average) |
| Polarisation | VV+VH |
| Antenna type | 6 x fan beam antennas (2 mid-beams at +- 90° azimuths and 4 side-beams at +-45° azimuth and +-135° azimuth) |
| Antenna gain (dBi) | 27-30 for mid-beams 23-31 for side-beams |
| Antenna pattern steering capability | No |
| Antenna pattern | See Below |
| Antenna orientation (degrees from nadir) | 17.5-45.5 (mid beams) 24-54 (side beams) |
| Receiver noise figure (dB) | 3.5 |
| Pulse/Receiver bandwidth (MHz) | 2 |
| Noise power (dBW) | –138 |
| Service area | Global |
| Footprint (km2) | 12 400 km² for mid-beams 21 000 km² for side-beams |

Concerning the polarisation, it is to be noted that EPS-SG SCA makes use of a dual linearly polarised antenna. Thus, no polarisation discrimination advantage can be taken into account.

### 2.1.2	Antenna pattern

The SCA antenna system is composed of 6 fan-beam antennas (with 1° aperture).

–	4 x SIDE-beams at 45°, 135°, 225° and 315° (with an antenna beam look angle from nadir ranging 24° to 54°).

–	2 x MID-beams at 90° and 270°  (with an antenna beam look angle from nadir ranging 17.5° to 45.5°).

Corresponding footprints are synthetised on Figure 1 below.

Figure 1

SCA footprints

The antenna pattern model for sharing analysis is given as follows.

G = max(Gmin ; Gver+Ghor);

with:

Ghor =	10 x log (sinc(coefH.sin(Az))²)

coefH =	40

Az = Azimuth angle taken from the pointing angle of the antenna (in radians)

Gver = linear extrapolation from Table 2 (for MID and SIDE beam) with Elevation angle in degrees

Gmin =	–10

Note: The cardinal sinc function is here used in its form: sincx=sin⁡(πx)πx

Table 2

Specific antenna gain for linear extrapolation

Note : the elevation angles are given with reference (0°) at satellite Nadir

The antenna pattern representation for each beam type (MID and SIDE) are given on Figures 1A, 1B, 2A and 2B for both vertical (at 0° azimuth) and horizontal (at elevation corresponding to the maximum gain, i.e. 45° (MID) and 55° (SIDE).

Figure 1A

Figure 2B

Figure 2A

Figure 2B

### 2.1.3	Protection criteria

The relevant protection criteria is given in Table 3, taken from Recommendation ITU-R RS.1166-4. Even if RLANs are nomadic/mobile by nature, their very high density implies that the interference will be systematic. The relevant percentage of data availability, corresponding to the percentage of time, is therefore 99% (see Document 5A/38)

Table 3

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Scatterometer | 8% degradation in measurement of normalized radar backscatter to deduce wind speeds | –5 | 99 | 95 |

For the SCA instrument, the protection criteria calculated over a 2 MHz bandwidth is –143 dBW
(-113 dBm) not to be exceeded more than 1% of the time.

This criteria is applied over data acquisition periods of time when the sensor is operating over the measurement area of interest (as per Recommendation ITU-R RS.1166-4).

## 2.2	Mobile service (WAS/RLAN)

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

–	Bandwidth factor:

The bandwidth factor (BWF) used in this document is derived taking into account the positioning of an EESS (active) 2 MHz bandwidth over the RLAN raster/channel plan, as shown below.

On this basis, the following bandwidth factors are considered (see details in Annex 2):

•	160 MHz (15% of RLANs): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/160) = -19 dB

•	80 MHz (50% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/80) = -16 dB

•	40 MHz (25% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/40) = -13 dB

•	20 MHz (10% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/20) = -10 dB

Overall, considering all bandwidth, this represent an average BWF = -16 dB

–	Propagation conditions:

•	building attenuation with a Gaussian distribution (17 dB + 7 dB) truncated at 1 dB, as described in the figure below:

Note: when used to calculate aggregate interference from multiple sources (as in the present case), the impact of this distribution is similar to the one leading from a 12 dB average attenuation.

•	Angular clutter loss model from Recommendation ITU-R P.452 associated with RLAN heights distributions and specific parameters for Urban, Suburban and Rural environments. The antenna heights are randomly selected using a uniform probability distribution from the set of floor heights at 3 meter steps. It should be noted that due to the EESS (active) geometry this model leads to no attenuation.

Antenna height

| RLAN deployment region | Antenna height (metres) |
| --- | --- |
| Urban | 1.5 to 28.5 |
| Suburban | 1.5, 4.5 |
| Rural | 1.5, 4.5 |

–	Antenna gain/discrimination (Omnidirectional in azimuth for all scenarios)

•	Option A1: Omnidirectional in elevation with 0 dBi gain

•	Option A3: An average 4 dB antenna discrimination is applied to the e.i.r.p. level distribution above in the direction of the satellite Omnidirectional in elevation with 0 dBi gain

Note: since Option A3 is proposing a fixed discrimination of 4 dB, corresponding results can therefore be extrapolated by shifting by 4 dB the results obtained with Option A1 (0 dBi).

–	Number of active RLAN:

•	Option D1: 11 279 active devices per 100 MHz channel per 5.25 million inhabitants (so-called “Sim City” with 5.25 M inhabitants).

•	Option D2: from 0.004 (D2-low) to 0.04 (D2-high) per 100 MHz channel per inhabitant.

Extrapolation of these numbers from a 100 MHz bandwidth to a 2 MHz bandwidth is detailed in Annex 2 and leads to the following figures:

•	Option D1: 5786 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0011 RLAN per inhabitant.

•	Option D2-Low: 10773 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0021 RLAN per inhabitant.

•	Option D2-High: 107722 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.021 RLAN per inhabitant.

These factors lead to the following number of active RLAN to be considered over the French territory (with a population of 66 M inh.) and the UK territory (with a population of 63.3 M inh.):

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 2 MHz) | Nb of active RLAN over UK (in 2 MHz) |
| --- | --- | --- | --- |
| Option D1 | 0.0011 per inh. | 72 738 | 69 763 |
| Option D2 | Low (0.0021 per inh.) | 135 432 | 129 892 |
|  | High (0.021 per inh.) | 1 354 319 | 1 298 916 |

Detailed deployment assumptions are described in the analysis sections.

# 3	Analysis

Analyses based on static and dynamic methodologies have been used to address the compatibility between WAS/RLAN and EESS (active) altimeter in the 5 GHz range:

–	Section 3.1: static analyses.

–	Section 3.2: dynamic analyses based on existing population densities.

## 3.1	Static analyses

### 3.1.1	Single entry static analysis

The following Table 4 provides calculation of the impact of 1 single outdoor RLAN on EESS (active) SCA sensor, for both MID and SIDE beams described in Table 1 above.

Table 4

| Parameter | SCA SIDE beam | SCA MID beam |
| --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 39 | 31.5 |
| Slant path distance (km) | 1121 | 1001 |
| Free Space losses (dB) | 168.0 | 167.0 |
| EESS antenna gain (dBi)(average over footprint) | 27.1 | 27.5 |
| EESS protection criteria (dBm/2 MHz) | –113 | –113 |
| RLAN EIRP (dBm) | 23 | 23 |
| Bandwidth factor for 20 MHz RLAN (dB) | 10 | 10 |
| Interference from 1 outdoor RLAN (dBm) | -127.9 | -126.5 |
| Margin for 1 outdoor RLAN (dB) | 14.9 | 13.5 |
| Nb of outdoor RLAN in the EESS footprint to reach the protection criteria | 31 | 23 |

This calculations shows that, for the a SIDE beam, 31 outdoor RLANs transmitting with the full e.i.r.p. of 200 mW within the 21000 km² footprint are sufficient to interfere with the SCA sensor.

Similarly, for the a MID beam, 23 outdoor RLANs transmitting with the full e.i.r.p. of 200 mW within the 12 400 km² footprint are sufficient to interfere with the SCA sensor.

This represents a maximum density of 0.0015 and 0.0018 RLAN / km², respectively.

### 3.1.2	Static analysis based on EESS (active) footprint shape

The following analysis provides calculation of the aggregate RLAN impact on EESS (active) considering the so-called ‘Sim City” (circular based) with the footprint shape of the EESS (active) system, as shown on the Figure 3 below.

Figure 3

RLAN deployment and EESS (active) footprint

Considering the size of the EESS (active) scatterometer footprints (with lengths of 660 km (MID beam) and 930 km (SIDE beam)), they largely extend out of the circular based RLAN deployment and limiting calculation to this deployment only would artificially limit the interference. To avoid this, the portion of the EESS (active) scatterometer footprints outside of the circular based “Sim city” have been considered covering rural areas.

In this case, the percentage of RLAN within the EESS (active) footprint (as on the right figure) can be calculated as in Tables 5A and 5B below (considering footprint widths at center footprint of 18.9 km (MID beam) and 22.6 km (SIDE beam)).

Table 5A

| SIDE beam | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 527 | 84% |
| Rural | 30 | 2 121 | 715 | 34% |
| Rural (outside the city) |  |  | 19 680 |  |

Table 5B

| MID beam | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 447 | 71% |
| Rural | 30 | 2 121 | 586 | 28% |
| Rural (outside the city) |  |  | 11 288 |  |

To consider potential interference from RLAN deployment on EESS (active) sensor, one can therefore use these percentages to determine the total number of RLAN in the EESS (active) footprint, for both density Option D1 (total of 25297 RLAN within the overall city for 5.25 M inhabitants), the low edge of Density Option D2 (47102 RLAN) and the upper edge of Density Option D2 (471022 RLAN) as given in Tables 6A and 6B below.

Table 6A

| SIDE beam | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 5785 | 10773 | 107722 |
| Urban | 100% | 35.5% | 2053 | 3824 | 38233 |
| Suburban | 84% | 53.3% | 2589 | 4821 | 48204 |
| Rural | 34% | 11.2% | 218 | 406 | 4061 |
| Rural (outside the city)* |  |  | 6006 | 11185 | 111851 |
| Total Nb of RLAN in EESS footprint |  |  | 10866 | 20236 | 202349 |

* RLAN density of 0.31/km² (D1), 0.57/km² (D2-low) and 5.68/km² (D2-high)

Table 6B

| MID | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 5785 | 10773 | 107722 |
| Urban | 100% | 35.5% | 2053 | 3824 | 38233 |
| Suburban | 71% | 53.3% | 2195 | 4088 | 40877 |
| Rural | 28% | 11.2% | 179 | 333 | 3333 |
| Rural (outside the city)* |  |  | 3445 | 6416 | 64156 |
| Total Nb of RLAN in EESS footprint |  |  | 7873 | 14660 | 146598 |

* RLAN density of 0.31/km² (D1), 0.57/km² (D2-low) and 5.68/km² (D2-high)

On this basis, the following Tables 7A and 7B provides calculation of interference for the “EESS (active) footprint shape scenario” for the EPS-SG SCA instrument for both SIDE and MID beams.

Table 7A (SIDE BEAM)

| Parameter | EPS-SG SCA (SIDE beam) with RLAN Density Option D1 | EPS-SG SCA (SIDE beam) with RLAN Density Option D2-low | EPS-SG SCA (SIDE beam) with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 39 | 39 | 39 |
| Slant path distance (km) (at center beam) | 1121 | 1121 | 1121 |
| Free Space losses (dB) (at center beam) | 168.0 | 168.0 | 168.0 |
| EESS antenna gain (dBi) (average over footprint) | 27.1 | 27.1 | 27.1 |
| Average RLAN EIRP (dBm) (including average BWF) | 3 | 3 | 3 |
| Interference from 1 RLAN (dBm) | -137.9 | -137.9 | -137.9 |
| Nb of RLAN (see table 6A above) | 10866 | 20236 | 202349 |
| Nb of outdoor RLAN (5%) | 543 (=27.2 dB) | 1012 (=30.1 dB) | 10117 (=40.1 dB) |
| Nb of indoor RLAN | 10323 (=40.1 dB) | 19224 (=42.8 dB) | 192231 (=52.8 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/2 MHz) | -110.6 | -107.9 | -97.9 |
| Interference from indoor RLAN (dBm/2 MHz) | -109.8 | -107.1 | -97.1 |
| TOTAL INTERFERENCE (dBm/2 MHz) | -107.1 | -104.4 | -94.4 |
| EESS protection criteria (dBm/2 MHz) | -113.0 | -113.0 | -113.0 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 5.9 | 8.6 | 18.6 |

Table 7B (MID BEAM)

| Parameter | EPS-SG SCA (MID beam) with RLAN Density Option D1 | EPS-SG SCA (MID beam) with RLAN Density Option D2-low | EPS-SG SCA (MID beam) with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 31.5 | 31.5 | 31.5 |
| Slant path distance (km) (at center beam) | 1001 | 1001 | 1001 |
| Free Space losses (dB) (at center beam) | 167.0 | 167.0 | 167.0 |
| EESS antenna gain (dBi) (average over footprint) | 27.5 | 27.5 | 27.5 |
| Average RLAN EIRP (dBm) (including average BWF) | 3 | 3 | 3 |
| Interference from 1 RLAN (dBm) | -136.5 | -136.5 | -136.5 |
| Nb of RLAN (see table 6B above) | 7873 | 14660 | 146598 |
| Nb of outdoor RLAN (5%) | 394 (=26 dB) | 733 (=28.7 dB) | 7330 (=38.7 dB) |
| Nb of indoor RLAN | 7479 (=38.7 dB) | 13927 (=41.4 dB) | 139268 (=51.4 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/2 MHz) | -110.6 | -107.9 | -97.9 |
| Interference from indoor RLAN (dBm/2 MHz) | -109.8 | -107.1 | -97.1 |
| TOTAL INTERFERENCE (dBm/2 MHz) | -107.2 | -104.5 | -94.5 |
| EESS protection criteria (dBm/2 MHz) | -113.0 | -113.0 | -113.0 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 5.8 | 8.5 | 18.5 |

This Tables shows that, when considering the “footprint shape” scenario, (with RLAN antenna Option A1), the RLAN deployment exceeds the EESS (active) protection criteria from 5.8 dB (with Density Option D1) to 18.6 dB (with Density Option D2-high).

As a summary, the following Table 8 provides the level of interference in excess considering all different RLAN density scenarios and antenna options.

Table 8

Level of interference in excess (static analysis)

| Scenario | EPS-SG SCA (SIDE beam) |  | EPS-SG SCA (MID beam) |  |
| --- | --- | --- | --- | --- |
| RLAN Antenna | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) |
| JTG Option D1 | 5.9 dB | 1.9 dB | 5.8 dB | 1.8 dB |
| JTG Option D2-low | 8.6 dB | 4.6 dB | 8.5 dB | 4.5 dB |
| JTG Option D2-high | 18.6 dB | 14.6 dB | 18.5 dB | 14.5 dB |

These calculations therefore show that when considering the “footprint shape” scenario, the RLAN deployment largely exceeds the EESS (active) protection up to 18.6 dB and allow to show there is no compatibility between RLAN and EESS (active) scatterometer sensors in the 5 GHz range.

## 3.2	Dynamic analyses

### 3.2.1	RLAN deployment

The dynamic analysis have been considered over France (550 000 km² and 66 M inhabitants) on the one hand and over a more restricted area covering the Paris metropolitan area (a square of 10 000 km² and approximately 10.2 M inhabitants) on the other hand.

Other simulations have been considered over the UK (244 000 km² and 63.3 M inhabitants) on the one hand and over a more restricted area covering the London metropolitan area (a square of 10 000 km² and approximately 12.9 M inhabitants) on the other hand.

These simulations are properly reflecting the real scenarios, with real population distributions.

Over these areas, different scenarios related to the number of active RLAN were considered as outlined in Table 9 below:

Table 9

Scenarios considered for the dynamic analysis

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Nb of active RLAN over UK (in 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) |
| --- | --- | --- | --- | --- | --- |
| Option D1 | 0.0011 per inh. | 72 738 | 11 241 | 69 763 | 14 217 |
| Option D2 | Low (0.0021 per inh.) | 135 432 | 20 930 | 129 892 | 26 471 |
|  | High (0.021 per inh.) | 1 354 319 | 209 304 | 1 298 916 | 264 708 |

These active RLAN have been deployed following the population densities, as depicted in Figure 2 below.

An EESS (active) measurement area has been defined around France, with an area of about 1 000 000 km² (blue square on Figure 4A), around Paris with an area of 10 000 km² (blue square on Figure 4B), around the Netherlands with an area of about 120 000 km² (blue square on Figure 4C), around the UK, with an area of about 700 000 km² (blue square on Figure 4D) and around London with an area of 10 000 km².

Figure 4a

RLAN deployment and measurement area over France (135 432 RLANs for D2-low)

Figure 4B

RLAN deployment and measurements area over Paris metropolitan (20 930 RLANs for D2-low)

Figure 4C

RLAN deployment and measurements area over the Netherlands (34 474 RLANs for D2-low)

Figure 4D

RLAN deployment and measurements area over the UK (129 892 RLANs for D2-low)

### 3.2.2	Dynamic analyses conditions

Simulations have been run for the EPS-SG SCA sensor (MID beam) with a time step of 1 second and for a period of 30 days.

At each step of the simulation (i.e. corresponding to 1 s dynamic of the EESS satellite), the interference to the EESS (active) sensor from each RLAN in visibility is calculated (taking into account the EESS antenna pattern to determine the relative gain), hence leading to an aggregate interference.

The percentage of time of interference is calculated with reference to the measurement area, which means that only the time steps when the sensor antenna boresight is within the blue area are retained for the calculation of the percentage of time of interference.

Then, compiling the aggregate interference over the whole steps of the simulations allows to deriving the interference distribution that will be compared to the EESS (active) protection criteria.

The SCA sensor has been considered with a payload active 100% of the time.

### 3.2.3	Dynamic analyses results over France

On this basis, Figure 5 gives the cumulative distribution function of interference for deployment of 135 432 RLANs over France (Option D2-low) and antenna Option A1.

Figure 5

Interference for SCA (MID beam) (Over France – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.2 dB (–132.8 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 70% of the time. The situation is also depicted on Figure 6 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 6

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D2-low)

In addition, Figure 7 gives the cumulative distribution function of interference for deployment of 72 738 RLANs over France (Option D1) and antenna Option A1.

Figure 7

Interference for SCA (MID beam) (Over France – options A1 and D1)

It can be seen that the EESS (active) protection criterion is exceeded by 7.7 dB (–135.3 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 40% of the time. The situation is also depicted on Figure 8 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 8

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D1)

It appears obvious that in these situations, the SCA sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 10 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 10

Interference in excess (over France)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over France (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 72 738 | 7.7 dB | 3.7 dB |
| JTG Option D2-low | 0.0021 | 135 432 | 10.2 dB | 6.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 354 319 | 20.2 dB | 16.2 dB |

### 3.2.4	Dynamic analyses results over Paris metropolitan

Under the same principle, the following Figure 9 gives the cumulative distribution functions of interference corresponding to a deployment of 135 432 RLANs over France, including 20 930 RLANs over Paris metropolitan area (Option D2-low) and antenna Option A1.

Figure 9

Interference for SCA (MID beam) (Over Paris Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10.8 dB
(–132.2 dBW). (The interference level corresponding to the protection criteria is exceeded for 100 % of the time.)

Similarly, the following Figure 10 gives the cumulative distribution functions of interference corresponding to a deployment of 72 738 RLANs over France, including 11 241 RLANs over Paris metropolitan area (Option D1) and antenna Option A1.

Figure 10

Interference for SCA (MID beam) (Over Paris Metro – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 8.3 dB 
(–134.7 dBW). (The interference level corresponding to the protection criteria is exceeded 100 % of the time.)

Finally, Table 11 provides the levels of interference in excess for the 3 RLAN density options
(D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 11

Interference in excess (over Paris metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 11 241 | 8.3 dB | 4.3 dB |
| JTG Option D2-low | 0.0021 | 20 930 | 10.8 dB | 6.8 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 209 304 | 20.8 dB | 16.8 dB |

### 3.2.5	Dynamic analyses results over the UK

Figure 11 gives the cumulative distribution function of interference for deployment of 129 892 RLANs over the United Kingdom (Option D2-low) and antenna Option A1.

Figure 11

Interference for SCA (MID beam) (Over the UK – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.6 dB (–-132.4 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 40% of the time. The situation is also depicted on Figure 12 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 12

interfered portion of images (in red) vs non-interfered (in white) (Over the UK – options A1 and D2-low)

In addition, Figure 13 gives the cumulative distribution function of interference for deployment of 69 763 RLANs over the UK (Option D1) and antenna Option A1.

Figure 13

Interference for SCA (MID beam) (Over the UK – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 7.8 dB 
(–135.2 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 14 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 14

Interfered portion of images (in red) vs non-interfered (in white) (Over the UK – JTG options A1 and D1)

It appears obvious that in these situations, the SCA (MID beam) sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 12 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 12

Interference in excess (over the UK)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over the UK (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 69 763 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.0021 | 129 892 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 298 916 | 20.6 dB | 16.6 dB |

### 3.2.6	Dynamic analyses results over London metropolitan

Figure 15 gives the cumulative distribution functions of interference corresponding to a deployment of 129 892 RLANs over the UK, including 26 471 RLANs over London metropolitan area (Option D2-low) and antenna Option A1.

Figure 15

Interference for SCA (MID beam) (Over London Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10.9 dB 
(–132.1 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for 100% of the time.

Finally, Table 13 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 13

Interference in excess (over London metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0011 | 14 217 | 7.9 dB | 3.9 dB |
| JTG Option D2-low | 0.0021 | 26 471 | 10.9 dB | 6.9 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 264 708 | 20.9 dB | 16.9 dB |

### 3.2.7	Dynamic analyses – Summary of results

The results of the dynamic analysis over France are given below (see Table 10 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over France (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 72 738 | 7.7 dB | 3.7 dB |
| JTG Option D2-low | 0.0021 | 135 432 | 10.2 dB | 6.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 354 319 | 20.2 dB | 16.2 dB |

The results of the dynamic analysis over Paris Metropolitan are given below (see Table 11 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 11 241 | 8.3 dB | 4.3 dB |
| JTG Option D2-low | 0.0021 | 20 930 | 10.8 dB | 6.8 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 209 304 | 20.8 dB | 16.8 dB |

The results of the dynamic analysis over the UK are given below (see Table 12 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over the UK (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 69 763 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.0021 | 129 892 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 298 916 | 20.6 dB | 16.6 dB |

The results of the dynamic analysis over London Metropolitan are given below (see Table 13 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0011 | 14 217 | 7.9 dB | 3.9 dB |
| JTG Option D2-low | 0.0021 | 26 471 | 10.9 dB | 6.9 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 264 708 | 20.9 dB | 16.9 dB |

Overall, in all cases, the above dynamic analyses confirm the result of static analyses, presenting interference largely exceeding EESS (active) protection criteria and allow to show that there is no compatibility between RLAN and EESS (active) in the 5 GHz range.

# 4	Summary/Conclusions

Under all scenarios and simulation methodologies (static and dynamic), the analyses show that RLAN deployment in the 5 GHz range would create large interference in the SCA sensor on board the EPS-SG satellites.

The static analyses presented above indicate that:

–	the maximum allowed density of outdoor RLANs (transmitting with the full e.i.r.p. of 200 mW) within the 12400 km² (MID beam) and 21000 km² (SIDE beam) EESS (active) is 0.0015 and 0.0018 RLAN / km², respectively (see section 3.1.1)

–	Depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by 1.8 to 18.6 dB (see section 3.1.2)

The dynamic analyses presented above indicate that, depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by (for 0.0011 to 0.021 active RLAN per inhabitant, respectively):

•	3.7 to 20.2 dB (case over France) (see section 3.3.3)

•	4.3 to 20.8 dB (case over Paris metropolitan) (see section 3.3.4)

•	3.8 to 20.6 dB (case over the UK) (see section 3.3.5)

•	3.9 to 20.9 dB (case over London metropolitan) (see section 3.3.6).

It has to be highlighted that these analyses were not considering a number of assumptions that would further increase these negative margins, such as an additional apportionment factor of the protection criteria (since the band is already shared with terrestrial radars). EUMETSAT stresses that this apportionment factor has not been introduced in the analysis given the already large negative results obtained under the assumption that no other services could generate additional interference to EESS (active).

Further, some assumptions related to RLAN remains unclear or unresolved and could also increase the potential interference to EESS (active). This covers in particular the possibilities given to a single RLAN to make use of multiple channels transmission (by means of either orthogonal transmissions or MIMO technique) or to concatenate multiple small channels to provide wider bandwidth with higher power. Such questioning also relates to other applications than RLAN since opening a band to RLAN, low power and unlicensed by nature, will drive the use of different applications such as SRDs, M2M, … (similarly to the current situation in the 2.4 GHz band) or LAA-LTE. Thus, consideration of these additional applications would need to be taken into account.

Overall, this document demonstrates and confirms again that RLANs cannot share with EESS (active) in the 5 GHz range, confirming that sharing between RLAN and EESS (active) in the 5 GHz range may only be feasible if additional RLAN mitigation measures are implemented.

[UK]

annex 1

FSS study 2 relevant sections from ECC Report 244 showing RLAN vs FSS sharing in the 5 725-5 850 MHz band

Appendix 1

# A1.1	Basic RLAN characteristics

Table A23-1

Basic RLAN (Wi-Fi) transmitter characteristics in the band 5 725-5 850 MHz

|  | RLAN 1 Omni-Indoor | RLAN 2 Omni Outdoor | RLAN 3 Directional Outdoor |
| --- | --- | --- | --- |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 | 30 | 30 |
| Bandwidth (MHz) | 20/40/80/160 | 20/40/80/160 | 20/40/80/160 |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 | 10/7/4/1 | 10/7/4/1 |
| Typical AP Antenna Type | Omni (azimuth) See Table 4.7.4-8, Table 4.9.4-10 and Table 4.11.4-12 Type 1 and 2 | Omni (azimuth) See Table 4.11.4-12, Type 1 and 2 | Directional, See Table 4.11.4-12, Type 3 and 4 |
| AP Antenna directivity gain (dBi) | 0-6 | 6-7 | 12/18 |

The figure below provides the spectrum mask for RLAN as function of the nominal channel bandwidth, typically 20, 40, 80 or 160 MHz

Figure A5-1

Spectrum mask for RLAN

The assumed average channel bandwidths distribution of RLAN devices is given in the following table.

Table A1-24

RLAN channel bandwidth distribution

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |

The next table provides RLAN receiver parameters for the purpose of compatibility studies with RLAN as a victim.

Table A1-25

Basic RLAN receiver characteristics in the band 5 725-5 850 MHz

| System parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Bandwidth (MHz) | 20 | 40 | 80 | 160 |
| kTB dBm / bandwidth | -101 | -98 | -95 | -92 |
| Typical Noise figure dB | 4 |  |  |  |
| Noise Power (dBm / bandwidth) | -97 | -94 | -91 | -88 |
| Typical Sensitivity for MCS0, BPSK (½ coding rate) (dBm) | -92 | -89 | -86 | -83 |
| C/N for MCS0, BPSK (½ coding rate) (dB) | 5 |  |  |  |
| I/N (dB) (note 1) | -6 |  |  |  |
| C/I (dB) | 11 for I/N -6 dB; 5 for I/N 0 dB |  |  |  |
| Maximum antenna gain at the RLAN Access Point (dBi) | See Table 4.9.4-10 and Table 4.11.4-12 |  |  |  |
| Maximum antenna gain at the RLAN user device (dBi) | See Table 4.7.4-8 |  |  |  |
| Note 1: As per Recommendation ITU-R M.1739 ‎[8]‎[8], the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB. Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis. |  |  |  |  |

## A1.1.1	RLAN antenna pattern

The characteristics in Table 4.7.4-8 are representative of an average antenna for all User Equipment within a population of RLAN devices. User Equipment can be defined as mobile or portable devices such as smart phones, tablets, notebooks, wireless scanners etc.

Table A1-26

RLAN User Equipment antenna (mobile/portable device)

| # | Type | Gain (dBi) | Antenna height above ground (m) |
| --- | --- | --- | --- |
| 1 | Omni-directional Antenna | 1.3 | 1 to 1.5 |
| NOTE: This value is the averaged value obtained from a survey on RLAN UE antennas. For simplicity, this antenna is assumed to be isotropic. |  |  |  |

The antenna pattern in Table 4.9.4-10 is considered as a representative average antenna pattern for indoor access points within the RLAN population. The table specifies the gains available at elevation angles; the antenna pattern is omni-directional in azimuth.

Table A1-27

Example of Indoor RLAN Access Point Omni-directional (azimuth) Antenna - 
Elevation Pattern

| Elevation angle θ (Degrees) | Gain (dBi) |
| --- | --- |
| 45 θ 90 | -4 |
| 35 θ 45 | 0 |
| 0 θ 35 | 3 |
| –15 θ 0 | -1 |
| –30 θ –15 | -4 |
| –60 θ –30 | -9 |
| –90 θ –60 | -8 |

The elevation angles in Table 4.9.4-10 are defined from the viewpoint of the RLAN Access Point when mounted to the ceiling. Positive elevation angles are towards the ground and negative elevation angles are towards the sky (typically, the RLAN AP is installed for optimal coverage). The pattern is normalised to 3 dBi gain on boresight.

Table 4.11.4-12 sets out the characteristics of RLAN antennas used on fixed indoor or outdoor equipment such as Access Points, Bridges, P2P or P2MP installations. The corresponding antenna patterns are provided in Figure 4.13.4-1 to Figure 4.16.4-4 below.

Table A1-28

Typical Fixed indoor and outdoor RLAN antenna 
(access points, bridges, P2P and P2MP)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 6 | Indoor & Outdoor | Figure 4.13.4-1 | 6 to 28,5 |
| 2 | Directional Antenna (sector) | 6 | Indoor & Outdoor | Figure 4.14.4-2 |  |
| 3 | Directional Antenna | 12 | Outdoor | Figure 4.15.4-3 |  |
| 4 | Directional Antenna (sector) | 17 | Outdoor | Figure 4.16.4-4 |  |
| NOTE: The (Highly) directional links are often installed on top of buildings |  |  |  |  |  |

Figure A1-6

RLAN 6 dBi Omni – Elevation (left) and Azimuth (right) Radiation Patterns

Figure A1-7

RLAN 6 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure A1-8

RLAN 12 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure A1-9

RLAN 17 dBi Sector Antenna – Elevation (blue) and Azimuth (red) Radiation Patterns

#### A1.1.1.1	RLAN APs antenna pattern measurements

A measurement campaign (see ‎[46]) was carried out to measure 7 different RLAN APs operating in the 5 GHz band (3 consumer and 4 enterprise 802.11ac Aps ‎[27]). This measurement presented the radiation pattern of all access points from 0° to 360° in azimuth and from −90° to +90° in elevation.

RLAN equipment can broadly be categorised in consumer, enterprise, and industrial equipment. The consumer segment is by far the biggest of the three, accounting for more than 85% of unit shipments.

In order to obtain representative results it was decided to characterise state-of-the art IEEE 802.11 ac ‎[27] consumer and enterprise equipment.

The following IEEE 802.11 ac devices were measured:

Table A1-29

Measured RLAN APs

| Consumer | Enterprise |
| --- | --- |
| Linksys EA6500 | Ubiquiti UAP-AC |
| Asus RT-AC66U | Aruba APIN0225 |
| Netgear R6300 | Zyxel NWA 1123-AC |
|  | Cisco Aircap 37021-E-K9 |

Figure A1-10

Consumer and Enterprise Antenna patterns

Consumer Access Points measured showed relatively low directivities and similar average e.i.r.p. with respect to elevation angles. In the majority of cases the maximum e.i.r.p. was observed at elevations between 20° and 75°.

Enterprise APs present higher directivities than the consumer ones. Consequently, the emission pattern depends strongly on how the AP is positioned (i.e. ceiling mounted or desk position).

### A1.1.2	RLAN power distribution

Table A1-30

RLAN power distribution

| Tx power e.i.r.p. | 1W (directional) | 1 W (omni) | 200mW (omni) | 80mW (omni) | 50mW (omni) | 25mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| Outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

The RLAN power distribution presented here leads to a 5.3 % use of outdoor devices. Sensitivity analysis may be performed with other outdoor use ratio to assess the impact of this parameter on the compatibility studies.

### A1.1.3	RLAN deployment and density of active devices

Two options were considered in CEPT Report 57 ‎[42]‎[42] (see section A3.1.7) :

Option A: From 0.0008 to 0.008 active devices per 20 MHz channel per inhabitant (based on 3% to 30% activity factor) applied to any population size”.

Option B: 4837 active devices per 20 MHz channel or 9871 active devices per 100 MHz channel per 5.25 million inhabitants as derived from the deployment figures provided in the table below.

Section ‎0 provides a detailed analysis on RLAN deployment and density of active devices used fort the compatibility studies with FSS.

# A1.2	FSS (Earth to space) in the band 5 725-5 850 MHz

### A1.2.1	FSS technical characteristics and deployments

In the 125 MHz portion of the band up to 5 850 MHz, this is a Region 1 allocation only (i.e. only Europe, Africa, and some of the northernmost countries in Asia). FSS deployments use the whole band 5 725-5 850 MHz and it is used by transmitting earth stations in the Earth-to-space direction operating only to satellites in geostationary orbits.

The following table provides details of the selection of satellites that have been taken as representative of those requiring protection in the visible portion of the geostationary orbit from Europe. In these frequency bands, the satellite beams cover very large areas of the Earth (using global, hemispherical, zonal or regional beams) as can be seen by the satellite footprint coverage plots in Annex 6 of ECC Report 068 ‎[18].

Table A1-31

Sample Satellite Data for the band 5 725-5 850 MHz

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| D | 3o East | Whole band | 34 | 773 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |

Typical FSS parameters developed by the ITU are provided in Table below.

Table A1-32

Typical FSS parameters in the 6 GHz band

| Parameter | Typical value |
| --- | --- |
| Range of operating frequencies | 5 850-6 700 MHz |
| Antenna diameters (m) | 1.2, 1.8, 2.4, 3.0, 4.5, 8, 16, 32 |
| Antenna reference pattern | Recommendation ITU-R S.465 |
| Range of emission bandwidths | 40 kHz - 72 MHz |
| Receiving space system figure of merit | +5 ↔ -10 dB/K (The database of Recommendation ITU-R S.1328 provides one example with Gsat= 24.8 dBi and Ts= 400 K, corresponding to a G/T of -1.2 dB/K |
| Earth station deployment | All regions, in all locations (rural, semi-urban, urban) |
| Earth station e.i.r.p. density towards the horizon | In accordance with RR No. 21.8 and Recommendation ITU-R S.524-9 |
| Minimum earth station antenna elevation angle, h, (degrees) | 5, 15 and 40 |

## A1.2.2	Protection criteria of FSS systems in the bands 3 400-4 200, 4 500-4 800 and 5 850-6 700 MHz

The criterion adopted for the protection of the FSS is that, on any satellite system, the RLAN emissions should not cause an increase of the equivalent temperature greater that x% of the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.  ΔT/T≤ x%.

In the first instance values for x = 6% (generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment) and x=1% (generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS) are utilised.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme has been considered where interference from RLANs is limited to half of the dT/T= 6% criterion i.e. the dT/T objective is reduced to a value of 3%. In addition, geographic apportionment is applied; dependent on the satellite’s coverage, this can have no effect on the dT/T objective or can reduce this to 1.5% or 1%.

# A1.3	Compatibility between RLAN and FSS (Earth to space) in the band 5 725-5 850 MHz

## A1.3.1	Interference from RLAN into FSS (Earth to space) in the band 5 725-5 850 MHz

### A1.3.1.1	Methodology

A methodology similar to the one used in ECC Report 206 ‎[26] is used for the purpose of sharing and compatibility studies between RLAN and the FSS in the range 5 725-5 850 MHz.

The methodology follows a 2-step approach as outlined below:

–	Step 1 is described in section ‎0: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receiver under consideration (see Table A1-19) (considering the satellite footprint) whilst satisfying the FSS protection criteria described in section ‎3.2.2.

•	The criterion adopted for the protection of the FSS is that, on any satellite system, the RLAN emissions should not cause an increase of the equivalent temperature greater that x% of the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.  ΔT/T≤ x%. Some initial calculations and results are provided for x=6% (generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment) and x=1% (generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services into FSS such as FS and MS). Further results are presented, taking account of a service and geographic apportionment scheme applied to the dT/T =6% criterion and further modelling considerations.

•	Interference apportionment between various potential sources of interference into FSS can be addressed through the choice of the protection criterion. For example, for sharing between FSS and at least two other co-primary services such as FS and MS, the portion of interference allowed to the MS can be derived by apportioning the total x%. See section ‎0 for more elements and analysis on geographic and service apportionment.

•	The propagation model is described in section A1.3.1.1.1.3.

–	Step 2 is described in section ‎0: This step delivers the number of active, on-tune, RLAN transmitters using a deployment model. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

#### A1.3.1.1.1	Step 1: Calculations for the maximum number of active RLAN transmitters within the FSS footprint

This section presents the calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver considering a range of protection criteria.

###### A1.3.1.1.1.1	Generic calculations

The following parameters are considered in the calculations:

–	RLAN e.i.r.p. distribution and channel bandwidth distribution as provided below.

Table A1-33

RLAN power distribution

| Tx power e.i.r.p. | 1W (directional) | 1 W (omni) | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

Table A1-34

RLAN channel bandwidth distribution and bandwidth correction values

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |
| Average bandwidth correction ratio | 0.7 | 0.5 | 0.5 | 0.25 |

–	RLAN antenna discrimination towards space: calculations are performed using two values: 0 dBi and 4 dBi.

–	Building (indoor to outdoor) attenuation, calculations are performed using two values: 12 dB and 17 dB.

The basic calculation method is illustrated in the table below. Assuming a free space path loss of 199.8 dB, the number of RLAN devices are increased until ΔT/T = 6 % exactly.

Table A1-35

Example calculation for the maximum number of RLANs

Based on this example calculation, Table A1-36 provides some initial results for the maximum number of RLANs that can be accommodated by the satellite receiver whilst satisfying a dT/T of 6%, taking into account two values of antenna discrimination (0 dB and 4 dB) and two values of building loss (12 dB and 17 dB).

Table A1-36

Initial results for Max number of RLANs for dT/T = 6%

| dT/T = 6% |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 |
| A | 242000 | 606000 | 175000 | 440000 |
| B | 2106000 | 5290000 | 1528000 | 3837000 |
| D | 242000 | 606000 | 175000 | 440000 |
| F | 2106000 | 5290000 | 1528000 | 3837000 |
| G | 375000 | 941000 | 272000 | 683000 |

###### A1.3.1.1.1.2	Service and geographic apportionment

It should be noted that the case of dT/T of 6% (Table A1-36) does not take account of service and geographic apportionment explicitly. The UK do not think there is a need to include any allowances for either geographic or service apportionment for the 5 725-5 850 MHz band but accept that further studies may be required.

###### A1.3.1.1.1.3	Further modelling considerations

There are other well-established factors that should be taken into account when modelling interference on the Earth to space interference paths and in order to achieve a more realistic model, some further modelling has been performed accounting for clutter loss and polarisation mismatch loss.

Clutter loss

Detailed calculations of the impact of clutter loss on the Earth to space interference path are given in ‎[19]. They are based on the method for the calculation of clutter loss on interference paths as set out in Recommendation ITU-R P.452-15 ‎[19].

Depending upon the satellite under consideration, consideration of the clutter loss leads to a percentage increase in the RLAN population in the range 50.34% to 130.16%, as detailed in Table 1-37 below.

Table 1-37

Impact of clutter loss

| Satellite | Increase in the RLAN population when clutter is modelled (%) |
| --- | --- |
| A | 51.43 |
| B | 77.58 |
| D | 51.43 |
| E | 78.08 |
| F | 105.98 |
| G | 118.95 |

Polarisation Mismatch Loss

Rationale for values considered for the polarisation mismatch loss and detailed calculations of its impact are given in ‎[47]. These calculations are based on two values of polarisation mismatch loss, 3 dB and 1.5 dB, applied to those outdoor RLANs that are not exposed to clutter loss.

For all satellites, consideration of polarisation mismatch of 3 dB leads to a percentage increase in the RLAN population of 42% when 12 dB building attenuation is used and 70% when 17 dB building attenuation is used.

When considering a polarisation mismatch loss of 1.5 dB, the increase in the RLAN population is 21% when 12 dB building attenuation is used and 31% when 17 dB building attenuation is used

Maximum number of RLANs after further modelling

Taking into account clutter loss and polarisation mismatch loss, the following Table 1-38 give the maximum number of on-tune, active, RLANs for the bands 5 725-5 850 MHz.

Table 1-38

Maximum number of on-tune RLAN for the 5 725-5 850 MHz band for Step 1 
(apportionment scheme applied)

| Band 5 725-5 850 MHz | With clutter loss and no polarisation mismatch |  |  |  | With clutter loss and polarisation mismatch |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 | 17 | 17 | 12 | 12 |
| A | 91615 | 229416 | 66251 | 166573 | 155746 | 390008 | 94076 | 236534 |
| B | 934959 | 2348496 | 678356 | 1703436 | 1589430 | 3992442 | 963265 | 2418879 |
| D | 91615 | 229416 | 66251 | 166573 | 155746 | 390008 | 94076 | 236534 |
| F | 1084485 | 2724086 | 786844 | 1975863 | 1843624 | 4630945 | 1117318 | 2805726 |
| G | 205266 | 515080 | 148886 | 373857 | 348952 | 875636 | 211418 | 530877 |

#### A1.3.1.1.2	Step 2: RLAN deployment model

Step 1, described in section ‎0, provides calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver based on a range of FSS protection criteria; initial results and results based on more advanced modelling and the apportionment scheme summarised in Table 1-16 are presented. For Step 1, the RLAN deployment is considered over all European countries/areas.

Step 2, presented in this section, aims to develop an RLAN deployment model for Europe with an objective to calculate the number of on-tune RLANs over this area in 2025. In theory, if the value delivered by Step 2 is less than or equal to that obtained in Step 1, this suggests that sharing is feasible.

Although it is obvious that both RLAN Access Points (APs) and terminals present interference potential to incumbent services, this section assumes that APs and terminals are not transmitting simultaneously. The calculation of the number of on-tune RLANs can thus be simplified by calculating the number of on-tune RLAN APs using the following methodology.

This methodology consists of 8 stages:

–	Stages 1 to 3 aim at defining the expected total number of RLAN APs over Europe.

–	Stages 4 to 7 are considering RLAN operational parameters to derive the expected total number of on-tune, active, RLANs over Europe.

–	Stage 8 aims to apply an upper bound on channel re-use.

##### A1.3.1.1.3	Elements related to stages 1 to 3

Stage 1: Define RLAN deployment environments and obtain relevant statistics

The aim is to define RLAN deployment environments and obtain statistics for the set of countries/areas considered (e.g. number of households, number of enterprise establishments).

Stage 2: Assign the statistics obtained in Stage 1 to urban, suburban and rural environments

The aim is to assign the statistics obtained in Stage 1 to urban, suburban and rural environments. This will be more realistic than averaging the RLAN APs over the entire European area. A realistic AP density in urban areas, in particular, allows for a practical investigation of the planning constraints (stage 8).

Stage 3: Apply Market penetration factors in different environments

These factors describe RLAN penetration into the different environments e.g. residential, enterprise and the number of APs per household/enterprise.

Stages 1 to 3 calculations

Detailed statistics and elements related to stages 1 to 3 are provided in ‎[44] and ‎[45] using a deployment model for the projected 2025 RLAN deployment in the whole of Europe and/or the EU-28.

Taking into account these elements and the large number of assumptions, it was agreed that some probable scenarios for the situation in 2025 are those defined in Table 1-39 below, based on the following assumptions from ‎[44] And ‎[45]:

–	Household “high” scenario with 90% penetration for EU-28 and 85% for the whole of Europe and medium projected growth of 10.4%.

–	2 RLAN APs per household >120 sqm (scenario A, corresponding to 1.22 AP average) or 2 RLAN APs per household >100 sqm (scenario B, corresponding to 1.35 AP average).

–	Enterprise “medium” scenario.

–	10 Million non-residential hotspots.

–	5% Add-on to cover all other type of usage, such as transport, industrial, mobile wifi, etc.

Table 1-39

Predictions of number of RLAN APs for 2025

On this basis, it was agreed that taking a value of 400 Million RLAN APs across Europe will provide a representative value to address the following Stages 4 to 7.

However, there is a certain level of uncertainty and, in order to reflect this, it was agreed to consider a range from 300 Million to 500 Million RLAN APs in Europe.

#### A1.3.1.1.4	Elements related to Stages 4 to 7

Stage 4: Apply Busy Hour Factor

This Factor gives the percentage of RLAN APs involved in Busy Hour Activity. A range of values is appropriate.

In Stage 3 an estimate of the actual number of APs is given. Stage 4 applies a factor to the output of Stage 3 to obtain the number and density of APs involved in busy hour in urban, suburban and rural environments.

The deployment model should be worst-case i.e. it should aim to model peak RLAN activity rather than average RLAN activity.

ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz considered an average value (over the population in urban, suburban and rural areas) of 62.7% for the busy hour factor, assumed to be dominated by the corporate usage.

Some consideration was given to the development of a refined Busy Hour model. While the Busy Hour factor is different for enterprise and residential environments no evidence could be found that this is the case for urban, suburban and rural areas. The fact that the satellite footprint covers different time zones has also been considered. Nevertheless, some further work might be required to refine the busy hour factor model, for instance by taking into account the non-uniform distribution of traffic over the RLAN population.

It was agreed to consider figures of 50%, 62.7% and 70% for the busy hour factor.

Stage 5: Apply 5 GHz Spectrum Factor

This Factor gives the percentage of RLAN activity at 5 GHz (rather than at 2.4 GHz or 60 GHz). This factor is applied in order to obtain the number and density of APs operating at 5GHz in urban, suburban and rural environments during Busy Hour.

It should be noted that there will be RLAN activity at 2.4 GHz and 60 GHz and, since the frequency range under consideration is at 5 GHz only, some RLAN activity during peak periods can be discounted.

A figure of 80% was originally proposed by the RLAN industry based on an optimistic model of corporate Busy Hour where 5 GHz dominates.

In addition, ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz considered values for the market factor for different environments:

–	80% for urban;

–	80% for suburban, and

–	50% for rural.

These values represent an average figure (over the population in urban, suburban and rural areas) of 74% for the 5 GHz spectrum factor.

Further considerations have led to values of 50% and 97% being exercised. The rationales for these values are described below:

Rationale for a 5 GHz Spectrum Factor of 97%

RLAN has, at this time, 80 MHz of spectrum available in 2.4 GHz and could have potentially 775 MHz of spectrum including the expansion bands in the 5 GHz frequency range. Therefore, considering these two frequency bands but neglecting 60 GHz, about 90% of the spectrum available would be in the 5 GHz range.

Furthermore, the requirement for additional 5 GHz spectrum is mainly based on the need for high throughputs and hence 80 and 160 MHz channels that are only available at 5 GHz, as shown on the RLAN channel bandwidth distribution currently agreed. This distribution depicts a figure of 65% of use of such 80 MHz and 160 MHz bandwidths that cannot be accommodated in the 2.4 GHz band. In addition, the available number of 20 MHz channels is 3 (not overlapping) in the 2.4 GHz band whereas it is 37 in the extended 5 GHz band showing therefore a share of 8% at 2.4 GHz and 92% at 5 GHz for the small channels (20 or 40 MHz), i.e. 35% of the channel use. Taking into account the fact that the other 65% of the channel use (80 and 160 MHz) will only be accommodated at 5 GHz, this gives an overall share of channel use of 3 % (8% × 35%) at 2.4 GHz and 97% (92% × 35% + 65%) at 5 GHz.

Rationale for a 5 GHz Spectrum Factor of 50%

Estimations for the current market deployment of 2.4/5 GHz spectrum were made based upon single (2.4 GHz only) and dual-band (2.4/5 GHz) Wi-Fi products that were shipped (by the overall connectivity market leader) across multiple segments in 2014. By multiplying the “Percentage of single band versus dual band per segment” by the “Percentage of Total products shipped per segment” the percentage of dual-band devices shipped overall was determined.  Based upon the above the percentage of 5 GHz enabled dual band products shipping across all Wi-Fi segments is approximately 57%. This does not take into account any split in spectrum usage for the dual band devices and the 57% value calculated was therefore an overestimation of the 5 GHz spectrum factor as it assumes that all dual band devices are operating in 5 GHz mode only.

Over the coming years although the RLAN industry expects to see a move to an increase in dual-band devices compared to single band devices it is also expected to see a penetration of up to 50% for 60 GHz devices. It was therefore suggested that a 5 GHz spectrum factor of 50% would be a reasonable estimation for current and longer term.

However, consideration of the 60 GHz band use could have an impact on the total number of APs in Europe as calculated under stage 1 to 3 above and would hence require further study.

It is understood that RLAN usage in the different frequency ranges in 2025 is difficult to predict and it was therefore agreed to consider figures of 50%, 74% and 97% for the 5 GHz spectrum factor.

Stage 6: Apply RF Activity Factor

This Factor gives the percentage of time that a RLAN is transmitting. When applied, this factor provides the number and density of active APs operating at 5 GHz during Busy Hour in urban, suburban and rural environments.

Recommendation ITU-R M.1651 sets out a method and example calculations that can be used as inputs to a calculation for the RF activity factor. For example: in the home environment, an RLAN accessing VHiMM for the entire Busy Hour has an individual Activity Factor (related to this VHiMM service only) of 12%.

It is agreed that a range of 3 to 30% is relevant for this factor. Within this range, it is also proposed to consider an activity factor of 10% corresponding to the figure proposed in JTG by the RLAN industry for rural deployment.

Sources: See section ‎0 and CEPT Report 57 ‎[42] (see section A3.1.7). This range covers JTG options A and B. The values of 3% and 30%.

Stage 7: Calculate the number of on-tune RLAN APs per 40 MHz

This stage provides the number and density of active, on-tune, APs operating at 5 GHz during Busy Hour, incident to a 40 MHz victim receiver bandwidth sourced from all (urban, suburban and rural) environments.

As described in ECC Report 244 ‎[47], this factor depends on the relative positioning of the 40 MHz victim receiver bandwidth on the RLAN channelization scheme, the total number of RLAN channels (for each bandwidth) and the RLAN bandwidth distribution. Stage 7 should be considered together with any bandwidth corrections made during Step 1 (see section ‎8.1.2).

As shown in ECC Report 244 ‎[47], there are 2 options to calculate this factor that differ in their outputs (and in where bandwidth correction factors are applied):

–	Option 1: delivers the total number of on-tune, active RLANs overlapping in frequency with the 40 MHz FSS receiver. Bandwidth correction factors are considered in Step 1;

–	Option 2: delivers the equivalent number of on-tune, active, 40 MHz interferers incident to the 40 MHz FSS receiver. Here the bandwidth correction factor is considered in Step 2.

Outputs from Step 1 are easily adjusted but since the bandwidth correction factor is already considered in Step 1 (see section ‎0), Option 1 is considered the simplest to implement in this study.

This leads to a factor of 12.9% of the total number of RLANs in the 5 GHz range overlapping the 40 MHz FSS receiver bandwidth (see detailed calculations in Error! Reference source not found.).

Stages 4 to 7 calculations

The following table provides the aggregate factor from Stages 4 to 7, for the 27 different cases resulting from the range of figures for Stage 4 (busy hour factor, 3 figures), stage 5 (spectrum factor, 3 figures) and stage 6 (activity factor, 3 figures).

Table A1-40

Aggregate factors 4 to 7

|  | Stage 4 | Stage 5 | Stage 6 | Stage 7 | Aggregate |
| --- | --- | --- | --- | --- | --- |
|  | Busy hour population | 5 GHZ factor | Activity factor | 40 MHz FSS | Stage 4 to 7 |
| Case 1 | 50% | 50% | 3% | 12.9% | 0.0010 |
| Case 2 | 50% | 50% | 10% | 12.9% | 0.0032 |
| Case 3 | 50% | 50% | 30% | 12.9% | 0.0097 |
| Case 4 | 50% | 74% | 3% | 12.9% | 0.0014 |
| Case 5 | 50% | 74% | 10% | 12.9% | 0.0048 |
| Case 6 | 50% | 74% | 30% | 12.9% | 0.0143 |
| Case 7 | 50% | 97% | 3% | 12.9% | 0.0019 |
| Case 8 | 50% | 97% | 10% | 12.9% | 0.0063 |
| Case 9 | 50% | 97% | 30% | 12.9% | 0.0188 |
| Case 10 | 62.70% | 50% | 3% | 12.9% | 0.0012 |
| Case 11 | 62.70% | 50% | 10% | 12.9% | 0.0040 |
| Case 12 | 62.70% | 50% | 30% | 12.9% | 0.0121 |
| Case 13 | 62.70% | 74% | 3% | 12.9% | 0.0018 |
| Case 14 | 62.70% | 74% | 10% | 12.9% | 0.0060 |
| Case 15 | 62.70% | 74% | 30% | 12.9% | 0.0179 |
| Case 16 | 62.70% | 97% | 3% | 12.9% | 0.0024 |
| Case 17 | 62.70% | 97% | 10% | 12.9% | 0.0078 |
| Case 18 | 62.70% | 97% | 30% | 12.9% | 0.0235 |
| Case 19 | 70% | 50% | 3% | 12.9% | 0.0014 |
| Case 20 | 70% | 50% | 10% | 12.9% | 0.0045 |
| Case 21 | 70% | 50% | 30% | 12.9% | 0.0135 |
| Case 22 | 70% | 74% | 3% | 12.9% | 0.0020 |
| Case 23 | 70% | 74% | 10% | 12.9% | 0.0067 |
| Case 24 | 70% | 74% | 30% | 12.9% | 0.0200 |
| Case 25 | 70% | 97% | 3% | 12.9% | 0.0026 |
| Case 26 | 70% | 97% | 10% | 12.9% | 0.0088 |
| Case 27 | 70% | 97% | 30% | 12.9% | 0.0263 |

#### A1.3.1.1.5	Overall application of stages 1 to 7

Taking into account the above elements in sections 8.1.3.1 and 8.1.3.2, final results of calculations in this study are summarised in Table A1-41 below.

Table A1-41

Number of on-tune RLAN APs per 40 MHz

|  | Aggregate Stages 4 to 7 (see section 8.1.3.2) | Total number of on-tune RLAN in Europe (for 300 Million APs) | Total number of on-tune RLAN in Europe (for 400 Million APs) | Total number of on-tune RLAN in Europe (for 500 Million APs) |
| --- | --- | --- | --- | --- |
| Case 1 | 0.0010 | 290118 | 386824 | 483530 |
| Case 2 | 0.0032 | 967061 | 1289414 | 1611768 |
| Case 3 | 0.0097 | 2901182 | 3868243 | 4835304 |
| Case 4 | 0.0014 | 429375 | 572500 | 715625 |
| Case 5 | 0.0048 | 1431250 | 1908333 | 2385417 |
| Case 6 | 0.0143 | 4293750 | 5725000 | 7156250 |
| Case 7 | 0.0019 | 562829 | 750439 | 938049 |
| Case 8 | 0.0063 | 1876098 | 2501464 | 3126830 |
| Case 9 | 0.0188 | 5628294 | 7504392 | 9380490 |
| Case 10 | 0.0012 | 363808 | 485078 | 606347 |
| Case 11 | 0.0040 | 1212694 | 1616926 | 2021157 |
| Case 12 | 0.0121 | 3638083 | 4850777 | 6063471 |
| Case 13 | 0.0018 | 538436 | 717915 | 897394 |
| Case 14 | 0.0060 | 1794788 | 2393050 | 2991313 |
| Case 15 | 0.0179 | 5384363 | 7179150 | 8973938 |
| Case 16 | 0.0024 | 705788 | 941051 | 1176313 |
| Case 17 | 0.0078 | 2352627 | 3136836 | 3921045 |
| Case 18 | 0.0235 | 7057881 | 9410507 | 11763134 |
| Case 19 | 0.0014 | 406166 | 541554 | 676943 |
| Case 20 | 0.0045 | 1353885 | 1805180 | 2256475 |
| Case 21 | 0.0135 | 4061655 | 5415541 | 6769426 |
| Case 22 | 0.0020 | 601125 | 801500 | 1001875 |
| Case 23 | 0.0067 | 2003750 | 2671667 | 3339583 |
| Case 24 | 0.0200 | 6011250 | 8015000 | 10018750 |
| Case 25 | 0.0026 | 787961 | 1050615 | 1313269 |
| Case 26 | 0.0088 | 2626537 | 3502050 | 4377562 |
| Case 27 | 0.0263 | 7879611 | 10506149 | 13132686 |

Comparison of these values with that obtained in Step 1 are provided in ‎[47] for the above 27 cases and all satellites, taking into account 400 Million APs.

Compared to these results for 400 Million APs, the case of 300 Million and 500 Million AP can be extrapolated with a factor -1.25 dB and +0.97 dB, respectively, as shown in ECC Report 244 ‎[47].

In order to summarise these results, it was agreed to present the results for 3 specific scenarios, as described in the Table A1-42 below:

Table A1-42

Scenarios for FSS summary analysis

| Scenario | Number of AP | Busy hour factor | Spectrum factor | RF activity factor | Total number of on-tune RLAN in Europe |
| --- | --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 300 M | 50 % | 50 % | 3 % | 290118 |
| “Medium” scenario (Case 14 above) | 400 M | 62.7 % | 74 % | 10 % | 2393050 |
| “Pessimistic” scenario (Case 27 above) | 500 M | 70 % | 97 % | 30 % | 13132686 |

Also, the different FSS satellites have been split in 5 different groups as in Table A1-43, considering similarities in interference potential, taking into account their main characteristics (e.g. max antenna gain, receiving system noise, orbital position, etc.)

Table A1-43

FSS groups for FSS summary analysis

| FSS Group | Satellites | Main characteritics | Remarks |
| --- | --- | --- | --- |
| FSS Group 1 | B F | Typical 26.5 dBi antenna gain | Global beams |
| FSS Group 2 | G | Typical 28.5 dBi antenna gain | Global beams (Satellite G presents a max antenna gain of 34 dBi but a higher system noise and a far east orbital position) |
| FSS Group 4 | A D | Around 34 dBi antenna gain | Hemispherical beams |

On this basis, the FSS analysis can be summarised as given in Table A1- 44 below, based on results taking into account clutter losses and polarisation mismatch with a 3 dB figure. If a figure of 1.5 dB is considered for the polarisation mismatch, the results would be modified by 0.7 dB (for the cases related to 12 dB building loss) and 1.13 dB (for the cases related to 17 dB building loss). For further details, see sections ‎0 and ECC Report 244 ‎[47]. As an example, the excess interference would be increased by these values.

Table A1- 44

Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 2.6 to 9.9 dB). FSS protection criteria exceeded for other FSS group 4 (exceeding of 0.9 dB) |
|  | 0 | 17 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 8 dB): FSS protection criteria exceeded for other FSS group 4 (exceeding of 2.7 dB) |
|  | 0 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 5.2 to 5.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 1.4 to 4.9 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2.2 to 2.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4.4 to 7.9 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 0 to 0.7 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 6.5 to 10.1 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 1.1 to 11.9 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 3.3 to 14.1 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 4.5 to 15.3 dB) |
|  | 4 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 6.7 to 17.4 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 8.5 to 19.2 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 10.7 to 21.4 dB) |

It is important to understand that the potential for RLAN–FSS sharing (Step 2 in particular) is based on some assumptions that may still need further study and, although providing some relevant results, it is at this stage too early to draw any definite conclusions with regard to the potential for RLAN and FSS to share at 5 GHz.

Stage 8: Apply upper bound on channel re-use

The demographic statistics show that in the European Union (EU28) approximately 50% of the population and households (see ‎[37], ‎[38], ‎[39], ‎[40], 44 and ‎[45]) are situated in less than 2% of the total land mass. This stage focusses on applying an upper bound to the number of co-channel RLANs using a practical planning method (i.e. applying a minimum re-use distance between co-frequency RLANs).

This method uses maximum channel re-use factors for active APs per km2 to derive an upper bound in densely populated areas.

This stage provides an adjustment to the results of Stage 7 by introducing an upper bound on the maximum density per km2 of RLAN APs in highly populated urban areas (e.g. London, UK).

The principle of this Stage 8 is agreed but it is recognised as not being trivial to calculate and quite time consuming. In addition, considering the quite large FSS footprint and since it may only apply to high densely population area, its potential advantage needs to be demonstrated.

At the time where this report was finalised, no studies were provided for Stage 8.

### A1.3.1.1.6	Summary – analysis of results

The studies have focused on the assessment of the interference from RLAN into FSS and follow a two-step approach:

–	Step 1 is described in section ‎0: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receiver under consideration (see Table A1-19) (considering the satellite footprint) whilst satisfying the FSS protection criteria described in section ‎3.2.2.

–	Step 2 is described in section ‎0: This step delivers the number of active, on-tune, RLAN transmitters using a deployment model. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

Concerning step 1, results have been obtained considering 2 different values of building attenuation for indoor use (12 and 17 dB), two values of antenna discrimination (0 and 4 dB), and an approach to service and geographic apportionment of the FSS protection criteria of ΔT/T=6%.

Further modelling takes account of clutter loss and polarisation mismatch loss on the Earth to space interference path.

The different factors used in step 2 are subject to some uncertainties because of the difficulties involved when deriving values for these factors and in particular when making predictions for 2025. Therefore it was agreed to preform sensitivity analyses, taking into account ranges of values for some of these factors.

Calculations and results are presented in this report but, although providing some relevant results, it is at this stage too early to draw definite conclusions.

Conclusions on the potential for RLAN–FSS sharing will be developed in the Part 2 Report, taking into account additional considerations, such as:

–	Antenna discrimination for outdoor RLANs;

–	Further studies on polarisation mismatch;

–	Studies supporting Stage 8 of FSS Step 2 (see section ‎0);

–	5 GHz Spectrum Factor (Stage 5 of FSS Step 2);

–	Control / monitoring on the long term aggregate effect of RLAN interference into FSS as RLAN deployment increases and investigation of what can be done in a scenario where the interference threshold is reached;

–	Further studies on apportionment of the FSS protection criteria.

#### A1.3.1.1.7	Potential mitigation techniques

Some potential mitigation techniques may need to be considered and their impact on the potential sharing between RLAN and FSS should be assessed. Among others, the following potential mitigation techniques could be addressed:

–	RLAN Access Points deployed only indoor;

–	Additional power limitation for RLAN.

There is a need for studies on the feasibility and practicability on the potential mitigation techniques.

aNNEX 1 - Appendix 2

Open Issues from ECC Report 244: LAA-LTE

# A2.1	LAA-LTE Characteristics

## A2.1.1	LAA-LTE RF characteristics

This section includes the LAA-LTE RF characteristics, such as transmission power, channel bandwidth, antenna characteristics, etc. which are relevant for the compatibility studies in the band 5 725-5 925 MHz

### A2.1.1.1	LAA-LTE transmission power characteristics

The first aspects taken into account are related to the maximum transmit power and in-band emission.

Table 45 and Table 46 show the basic characteristics of an LAA-LTE transmitter and a user equipment agreed for standardization ‎[3] ‎[4] . As it can be observed, LAA-LTE will fulfil same requirements as for RLAN characteristics used in ECC Report 244 in terms of conducted and radiated power and power spectral density limits. Also, available channel bandwidths will be aligned with those assumed for RLAN. In LAA-LTE, basic transmission bandwidth per carrier is 20 MHz. Transmissions on bandwidths higher than 20 MHz are done via carrier aggregation.

Table 45

Basic LAA-LTE Base station characteristics in the band 5 725-5 925 MHz

| System Parameters | Indoor |  | Outdoor |  |
| --- | --- | --- | --- | --- |
|  | RLAN | LAA-LTE | RLAN | LAA-LTE |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 |  | 30 |  |
| Bandwidth (MHz) | 20/40/80/160 |  | 20/40/80/160 |  |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 |  | 17/14/11/8 |  |
| Typical AP Antenna Type | Omni (azimuth) | Omni-directional (azimuth) | Omni (azimuth) | Omni-directional (azimuth) |
| AP Antenna gain + cable loses (dBi) | 0-6 | 5 | 6-7 | 5 |
| AP Antenna Height (m) | 6-28.5 | 6 | 6-28.5 | 10 |

Table 46

Basic LAA-LTE UE characteristics in the band 5 725-5 925 MHz

| System parameter | Value |  |
| --- | --- | --- |
|  | RLAN | LAA-LTE |
| Maximum Transmit Power (e.i.r.p. - dBm) |  | 23 |
| Bandwidth (MHz) | 20/40/80/160 |  |
| kTB (dBm / bandwidth) | -101 |  |
| Typical Noise figure (dB) | 4 | 4 |
| Noise Power (dBm / bandwidth) | -97 | -97 |
| I/N (dB) | -6 |  |
| Maximum antenna gain (dBi) | 1.3 | 0 |
| Antenna Height (m) | 1-1.5 | 1.5 |

### A2.1.1.2	LAA-LTE UE power distribution

This section describes the LAA-LTE power distribution. For the case of user equipment (UE), transmit power levels will be allocated following the baseline 3GPP Fractional Power Control (FPC) method which has three components: base-line open loop operating point, dynamic offset part and bandwidth factor. The overall power control equation looks as follows:

Where  is the resulting UE transmit power level,  is the fractional path-loss compensation factor,  is the estimation of actual path-loss between the BS and the UE. The dynamic offset part consists of MCS (Modulation and Coding Scheme) dependent factor,  (TF stands for transmission format) and the explicit power control instruction part. The bandwidth factor is dependent on, which stands for granted bandwidth, in terms of number of granted PRBs.

Figure 11

Statistics of UL power levels used by the terminals in the indoor deployment scenario

Figure 11 shows the UL transmit power distribution for an indoor deployment scenario, which corresponds to the indoor in-building scenario in 3GPP TR 36.889‎[41]. In the indoor scenario, two operators deploy 4 small cells each equally spaced and centered in the single-floor building (50m-by-120m). The distance between two closest nodes from two operators is random. Since the LAA-LTE BS is usually close to the UE in LAA-LTE operations, the terminals transmit in UL using typically low power levels. In this case, the median UE transmit power is less than 0 dBm, for SNRtarget =12 dB.

Figure 12 shows the UE transmit power distribution for an outdoor deployment (as specified in 3GPP TR 36.889‎[41]). In the outdoor scenario, the LAA-LTE cells are in clusters uniformly distributed within the macro geographical area; 4 small cells per operator, uniformly random dropping within the cluster area. As expected, in the outdoor case power levels are higher compared to the indoor case due to higher coupling loss. UL power distribution depends on the specific power control settings, results presented in Figure 11 and Figure 12 represents a reasonable trade-off in terms of noise raise due to intra system interference and achievable mean throughput. In this case, the median UE transmit power is less than 4 dBm, for SNRtarget =9 dB.

Figure 12

Statistics of UL power levels used by the terminals in the outdoor deployment scenario

For the case of LAA-LTE BS, actual transmit power varies according to the cell load variations. Thus, LAA-LTE BS transmits on full power only when the cell is fully loaded according to current specifications (LTE Release 8 BS). It is assumed that on average BS use half of the available power, i.e. an average 50% cell load. Averaged over time BS transmits at half maximum power, i.e. 20 dBm.

Table 3 summarises the LAA-LTE power distribution for both BS and UE, taking into account following assumptions:

–	Bandwidth distribution as shown in Table 4;

–	equal transmission probability between UE and BS (50%/50% share);

–	indoor/outdoor ratio of LAA-LTE UEs assuming that 85% and 15% of the traffic is generated indoors and outdoors, respectively;

–	at least 9 UEs per LAA-LTE BS; thus, out of the LAA-LTE devices, there will be at least 10% LAA-LTE BS and the rest 90% will be LAA-LTE UEs;

–	approximately, one in every six LAA-LTE BS is placed outdoor.

Influence of Wi-Fi deployed nearby LAA-LTE on the power distribution of LAA-LTE remains for further investigation.

Table 47

LAA-LTE power distribution considering both LAA BS and UE

| Tx power e.i.r.p. | 1 W | 200 mW | 140 mW | 100 mW | 50 mW | 13 mW | <=1mW | all |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor, % | 0,00 | 9,55 | 0,96 | 20,58 | 7,96 | 21,50 | 22,95 | 83,50 |
| Outdoor, % | 0,01 | 2,10 | 0,49 | 3,92 | 1,91 | 5,28 | 2,79 | 16,50 |

### A2.1.1.3	LAA-LTE channel bandwidth characteristics

LAA-LTE can support different channel bandwidths through a carrier aggregation mechanism.  It can be observed that IEEE and 3GPP standards are converging in terms of overall spectral efficiency. This is particularly true when the evolution of the 802.11 family with 802.11ax standard is considered. Assuming that LAA-LTE and Wi-Fi will be subject to the same traffic demand by the end users, it is reasonable to expect that very similar channel bandwidth distribution will be required by the two systems.

Based on the above observations, it is expected that Wi-Fi and LAA-LTE will have same channel bandwidth distribution as described in Table 48.

Table 48

LAA-LTE channel bandwidth distribution and bandwidth correction values

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| Device Percentage | 10 % | 25 % | 50 % | 15 % |

### A2.1.1.4	LAA-LTE antenna pattern

The antenna pattern in Figure 13 is considered as representative average antenna pattern for LAA-LTE omnidirectional antenna which can be employed for indoor and outdoor access points. In a typical deployment, LAA-LTE antenna will be mounted to the ceiling and titled down towards the ground which is beneficial in terms of coverage. Table 49 summarize the antenna characteristics for LAA-LTE omni-directional antenna.

Figure 13

LAA-LTE Omnidirectional - Elevation (left) and Azimuth (right) Radiation Patterns

Table 49

Typical Indoor and Outdoor LAA-LTE BS antenna (access point)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Typical Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 5 | Indoor & Outdoor | Figure 3 | 6 (indoor) 10 (outdoor) |

## A2.1.2	Deployment scenarios of LAA-LTE

LAA-LTE is expected to be deployed by operators, mainly in hotspots and enterprise environments. Therefore deployments of possible LAA-LTE in the residential environment (user deployed) have not been considered in the current studies. These limited assumptions will have to be considered either in an expansion of future technical studies or as a possible regulatory issue to be studied.  Operator-deployed means that the actual deployment will be “somewhat planned”, therefore LAA-LTE will only be used in places where the operator coverage of LAA-LTE is available. Notice that the main target for LAA-LTE is mainly indoor small cells deployments, but outdoor deployment is not precluded as specified in 3GPP TR 36.889 Error! Reference source not found.‎[41] which was developed by vendors and operators.

In the sections below, the potential deployment scenario of LAA-LTE is presented.

### A2.1.2.1	Indoor scenario

#### A2.1.2.1.1	Enterprise environment

The enterprise deployment is a scenario where the users are in a business environment. So, in essence, this scenario will be a large office space, sport arena, shopping malls, factories, production facilities, etc. In this scenario, usually the locations are covered by wide area LTE network; however, LAA-LTE provides a capacity booster service. For this scenario, the following needs to be considered:

–	User distribution/density: there will be dense users in these locations; however the users will be mainly active only in certain parts of the day. Also, in some locations, e.g. sports arena, large number of people will be requiring service only periodically and for a short period of time. In case of large office space or production facilities, there are possibilities that LAA-LTE BS can be LoS to each other. This is beneficial in a sense that, hidden node problem from channel access mechanism is largely removed.

–	Usage/activity information: as mentioned earlier, the user density is high at certain times of the day, while there will be no active users in some other times, since the deployment locations for enterprise case only involves activities in certain part of the day.

#### A2.1.2.1.2	Extended indoor coverage

Compared to enterprise deployment, when LAA-LTE is used as capacity booster; for extended indoor coverage, e.g. parking floors or underground places (shops, store houses, etc.), LAA-LTE will be used mainly for providing coverage. In this case, wide area LTE network is sometimes not able to cover the places, thus deployment of LAA-LTE will provide the coverage. In this case, the coverage will be provided when LAA-LTE node will be used in heterogeneous deployment manner.  The current understanding is that, extended indoor coverage may include mainly work/public areas. The residential coverage is out of question at this moment.

### A2.1.2.2	Outdoor Scenario

#### A2.1.2.2.1	Immidiate outdoor scenario

This scenario includes parks, small open area just outside large buildings, enclosed outdoor places inside building areas, etc. This deployment is mainly for boosting the LTE capacity, not necessarily for coverage. Urban city center is one such example. There are few mobile users, however the users can be outdoors, or just inside the buildings. Usually, there are places where there are many people, looking for high data rate services.

#### A2.1.2.2.2	Large open spaces

LAA-LTE will also be deployed in large open spaces with significant number of users, e.g. train stations, or stadium, etc. In this case, the deployment conditions are very similar to the above immediate outdoor scenario; however, the user density at some point of time can be very high.

# A2.2	Compatibility between RLAN/LAA LTE and FSS (Earth to space) in the band 5 725-5 925 MHz

It should be noted that these studies are additional to those presented in ECC Report 244 and take account of both WiFI and LAA-LTE usage in the same band.

### A2.2.1	Step 1: Calculations for the maximum number of active RLAN transmitters within the FSS footprint

This section presents the calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver considering a range of protection criteria‎[2].

The following parameters are considered in the calculations:

–	Mixed RLAN deployment between Wi-Fi and LAA-LTE:

•	For the case of Wi-Fi, power distribution and channel bandwidth distribution as provided in ECC Report 244.

•	For the case of LAA-LTE: power distribution and channel bandwidth distribution according Section 3.1.

–	Antenna discrimination towards space: calculations are performed using two values 0dB and 4 dB for both technologies, i.e. Wi-Fi and LAA-LTE.

–	LAA-LTE will be driven by operators and service provider’s deployment, addressing enterprise and hotspot scenarios. Based on JRC prediction for the number of RLAN devices in 2025, the overall percentage of RLANs deployed in an enterprise and hotspot scenario is around 10% compared to 85% of residential usage. As baseline assumption, LAA-LTE will represent 90% of the enterprise sector and 70% of the hotspots. An additional use of 5% is also envisaged (see Table 52). On this basis, Step 1 calculations are performed with a share between WiFi and LAA of 91.56 % and 8.44 %, respectively.

–	RLAN e.i.r.p. distribution and channel bandwidth distribution as provided in section 1.

–	Building (indoor to outdoor) attenuation, calculations are performed using two values: 12 dB and 17 dB.

Table 50

Initial results for Max number of RLANs for dT/T = 6%

| dT/T (%) | 6% (Table 49) |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 |
| Satellite |  |  |  |  |
| A | 235500 | 588750 | 173600 | 434000 |
| B | 2056000 | 5140000 | 1516000 | 3790000 |
| D | 235500 | 588750 | 173600 | 434000 |
| F | 2056000 | 5140000 | 1516000 | 3790000 |
| G | 365500 | 913750 | 269400 | 673500 |

Maximum number of RLANs after further modelling

Taking into account clutter loss and polarisation mismatch loss and the results given in Tale 28 the following Table 1-38 gives the maximum number of on-tune, active, RLANs for the bands 5 725-5 850 MHz.

Table 51

Maximum number of on-tune RLAN for the 5 725-5 850 MHz band for Step 1 (apportionment scheme applied)

| Band 5 725-5 850 MHz | With clutter loss and no polarisation mismatch |  |  |  | With clutter loss and polarisation mismatch |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 | 17 | 17 | 12 | 12 |
| A | 89344 | 223359 | 65834 | 164585 | 151884 | 379711 | 93485 | 233711 |
| B | 914537 | 2286343 | 673916 | 1684790 | 1554713 | 3886782 | 956961 | 2392402 |
| D | 89344 | 223359 | 65834 | 164585 | 151884 | 379711 | 93485 | 233711 |
| F | 1060797 | 2651993 | 781694 | 1954235 | 1803355 | 4508387 | 1110006 | 2775014 |
| G | 200339 | 500848 | 147244 | 368110 | 340577 | 851442 | 209086 | 522716 |

### A2.2.2	Step 2: RLAN deployment model

This methodology consists of 8 stages:

–	Stages 1 to 3 aim at defining the expected total number of RLAN APs over Europe.

–	Stages 4 to 7 are considering RLAN operational parameters to derive the expected total number of on-tune, active, RLANs over Europe.

Since the predictions made by JRC were technology agnostic and based on predictions on the population growth by 2025, for simplicity sake the overall population of RLAN is assumed to be the reference value in the context of this study (noting that a range is exercised), whatever the market penetration is for the different technologies, although new applications offered by LTE LAA would tend to increase the overall number of access points/Base   stations. Further studies may be required.

The overall percentage of RLAN deployed in an enterprise and hotspot scenario is around 10% compared to 85% deployed in a residential scenario. An additional use of 5% is also envisaged (see Table 52).Three possible scenarios for LAA-LTE and Wi-Fi deployment in 2025:

–	LAA-LTE will represent all of the 10% AP deployed in enterprise and hotspot scenarios.

–	LAA-LTE will be a completely new addition to the number of RLANs determined based on JRC projections.

–	A percentage of RLANs deployed in enterprise and hotspots will be replaced by LAA-LTE.

As baseline assumption, LAA-LTE will represent 90% of Wi-Fi deployed in enterprise and around 70% of the hotspot deployment; however a sensitivity analysis can be derived. Elements related to stages 1 to 3:

Stage 1: 	Define RLAN deployment environments and obtain relevant statistics

Stage 2: 	Assign the statistics obtained in Stage 1 to urban, suburban and rural environments

Stage 3: 	Apply Market penetration factors in different environments

For elements related to stages 1 to 3, refer to section 8.1.3.1 of ECC Report 244.

Results are reminded in the table below:

Table 52

Predictions of number of RLAN APs for 2025

### A2.2.3	Elements related to stages 4 to 7

Stage 4: Apply Busy Hour Factor

The busy hour factor depends on the deployment area and it is not influenced by the RLAN system characteristics. Therefore, the average value of 62.7% considered already in the ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz and, figures of 50% and 70% for the busy hour factor should also be applicable to LAA-LTE system.

Stage 5: Apply 5 GHz Spectrum Factor

In the context of this report, it is reasonable to investigate spectrum factor values similar to those used in ECC Report 244 (refer to section 8.1.3.2 of ECC Report 244). Further studies may be required.

Stage 6: Apply RF Activity Factor

Given the current LAA-LTE functionalities (listen-before-talk, discontinuous transmissions, DFS, carrier selection and TPC) which are following the regulatory requirements for RLANs operating at 5 GHz, it is expected that LAA-LTE device will have same RF activity levels as other RLANs at 5 GHz. Therefore, the agreed range of 3 to 30% for the RF activity factor should also be applicable to LAA-LTE devices.

Stage 7: Calculate the number of on-tune RLAN APs per 40 MHz

Whatever the technology is, the RLAN have access to the same channels in the 5GHz band and have the same system bandwidth available. Therefore, the number of RLAN Access points per 40 MHz as discussed in ECC Report 244 applies to LTE-LAA.

Table 53

Aggregate factors 4 to 7

|  | Stage 4 | Stage 5 | Stage 6 | Stage 7 | Aggregate |
| --- | --- | --- | --- | --- | --- |
|  | Busy hour population | 5 GHZ factor | Activity factor | 40 MHz FSS | Stage 4 to 7 |
| Case 1 | 50% | 50% | 3% | 12.9% | 0.0010 |
| Case 2 | 50% | 50% | 10% | 12.9% | 0.0032 |
| Case 3 | 50% | 50% | 30% | 12.9% | 0.0097 |
| Case 4 | 50% | 74% | 3% | 12.9% | 0.0014 |
| Case 5 | 50% | 74% | 10% | 12.9% | 0.0048 |
| Case 6 | 50% | 74% | 30% | 12.9% | 0.0143 |
| Case 7 | 50% | 97% | 3% | 12.9% | 0.0019 |
| Case 8 | 50% | 97% | 10% | 12.9% | 0.0063 |
| Case 9 | 50% | 97% | 30% | 12.9% | 0.0188 |
| Case 10 | 62.70% | 50% | 3% | 12.9% | 0.0012 |
| Case 11 | 62.70% | 50% | 10% | 12.9% | 0.0040 |
| Case 12 | 62.70% | 50% | 30% | 12.9% | 0.0121 |
| Case 13 | 62.70% | 74% | 3% | 12.9% | 0.0018 |
| Case 14 | 62.70% | 74% | 10% | 12.9% | 0.0060 |
| Case 15 | 62.70% | 74% | 30% | 12.9% | 0.0179 |
| Case 16 | 62.70% | 97% | 3% | 12.9% | 0.0024 |
| Case 17 | 62.70% | 97% | 10% | 12.9% | 0.0078 |
| Case 18 | 62.70% | 97% | 30% | 12.9% | 0.0235 |
| Case 19 | 70% | 50% | 3% | 12.9% | 0.0014 |
| Case 20 | 70% | 50% | 10% | 12.9% | 0.0045 |
| Case 21 | 70% | 50% | 30% | 12.9% | 0.0135 |
| Case 22 | 70% | 74% | 3% | 12.9% | 0.0020 |
| Case 23 | 70% | 74% | 10% | 12.9% | 0.0067 |
| Case 24 | 70% | 74% | 30% | 12.9% | 0.0200 |
| Case 25 | 70% | 97% | 3% | 12.9% | 0.0026 |
| Case 26 | 70% | 97% | 10% | 12.9% | 0.0088 |
| Case 27 | 70% | 97% | 30% | 12.9% | 0.0263 |

Table 54

Total number of on-tune RLAN in Europe for low, medium and high scenarios.

|  | Aggregate Stages 4 to 7 (see section 8.1.3.2) | Total number of on-tune RLAN in Europe (for 300 Million APs) | Total number of on-tune RLAN in Europe (for 400 Million APs) | Total number of on-tune RLAN in Europe (for 500 Million APs) |
| --- | --- | --- | --- | --- |
| Case 1 | 0.0010 | 290118 | 386824 | 483530 |
| Case 2 | 0.0032 | 967061 | 1289414 | 1611768 |
| Case 3 | 0.0097 | 2901182 | 3868243 | 4835304 |
| Case 4 | 0.0014 | 429375 | 572500 | 715625 |
| Case 5 | 0.0048 | 1431250 | 1908333 | 2385417 |
| Case 6 | 0.0143 | 4293750 | 5725000 | 7156250 |
| Case 7 | 0.0019 | 562829 | 750439 | 938049 |
| Case 8 | 0.0063 | 1876098 | 2501464 | 3126830 |
| Case 9 | 0.0188 | 5628294 | 7504392 | 9380490 |
| Case 10 | 0.0012 | 363808 | 485078 | 606347 |
| Case 11 | 0.0040 | 1212694 | 1616926 | 2021157 |
| Case 12 | 0.0121 | 3638083 | 4850777 | 6063471 |
| Case 13 | 0.0018 | 538436 | 717915 | 897394 |
| Case 14 | 0.0060 | 1794788 | 2393050 | 2991313 |
| Case 15 | 0.0179 | 5384363 | 7179150 | 8973938 |
| Case 16 | 0.0024 | 705788 | 941051 | 1176313 |
| Case 17 | 0.0078 | 2352627 | 3136836 | 3921045 |
| Case 18 | 0.0235 | 7057881 | 9410507 | 11763134 |
| Case 19 | 0.0014 | 406166 | 541554 | 676943 |
| Case 20 | 0.0045 | 1353885 | 1805180 | 2256475 |
| Case 21 | 0.0135 | 4061655 | 5415541 | 6769426 |
| Case 22 | 0.0020 | 601125 | 801500 | 1001875 |
| Case 23 | 0.0067 | 2003750 | 2671667 | 3339583 |
| Case 24 | 0.0200 | 6011250 | 8015000 | 10018750 |
| Case 25 | 0.0026 | 787961 | 1050615 | 1313269 |
| Case 26 | 0.0088 | 2626537 | 3502050 | 4377562 |
| Case 27 | 0.0263 | 7879611 | 10506149 | 13132686 |

Table 55

Scenarios for FSS summary analysis

| Scenario | Number of AP | Busy hour factor | Spectrum factor | RF activity factor | Total number of on-tune RLAN in Europe |
| --- | --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 300 M | 50 % | 50 % | 3 % | 290118 |
| “Medium” scenario (Case 14 above) | 400 M | 62.7 % | 74 % | 10 % | 2393050 |
| “Pessimistic” scenario (Case 27 above) | 500 M | 70 % | 97 % | 30 % | 13132686 |

Table 56

Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.2 to 11.9 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 3.3 to 4.0 dB) FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 3.2 to 6.8 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 2.4 to 13.2 dB) |

# A2.3	Conclusions

Three additional studies have been made with LAA-LTE, compared to ECC Report 244:

–	Compatibility of a mix LAA and WiFi market share with FSS. Results are roughly the same as for the case of WiFi only.

Therefore the impact of adding LAA-LTE use case in 5 GHz bands appears to have minimal effect on the overall results of compatibility and sharing as shown in ECC Report 244.

List of References

EC Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for Wireless Access Systems including Radio Local Area Networks in the bands 5 350-5 470 MHz and 5 725-5 925 MHz ('WAS/RLAN extension bands') for the provision of wireless broadband services, September 2013

ECC Decision (04)08: "Harmonised use of 5 GHz for the implementation of WAS/RLANs "

WRC Resolution 229 (WRC-03): "Use of the bands 5 150-5 250, 5 250-5 350 MHz and 5 470-5 725 MHz by the mobile service for the implementation of Wireless Access Systems including Radio Local Area Networks"

ERC Report 67: "Study of the Frequency sharing between HIPERLANs and MSS feeder links in the 5 GHz band"

ERC Report 72: " Compatibility studies related to the possible extension band for HIPERLAN at 5 GHz"

ETSI EN 301 893: " Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonized EN covering the essential requirements of Article 3.2 of the R&TTE Directive"

EC Decision 2005/513/EC complemented by EC Decision 2007/90/EC

Recommendation ITU-R M.1739: "Protection criteria for wireless access systems, including radio local area networks, operating in the mobile service in accordance with Resolution 229 (WRC-03) in the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz"

ERC Report 25: "The European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz"

EC Decision 2008/671/EC: "Commission Decision of 5 August 2008 on the harmonised use of radio spectrum in the 5 875-5 905 MHz frequency band for safety related applications of Intelligent Transport Systems (ITS)"

ECC Decision (08)01: " ECC Decision of 14 March 2008 on the harmonised use of the 5 875-5 925 MHz frequency band for Intelligent Transport Systems (ITS)"

ECC Recommendation (08)01: " Use of the band 5 855-5 875 MHz for Intelligent Transport Systems"

ECC Report 101: "Compatibility studies in the band 5 855-5 925 MHz between Intelligent Transport Systems (ITS) and other systems"

ECC Decision (12)04: " ECC Decision of 2 November 2012 on the withdrawal of ECC Decision (02)01. For national implementation issues related to the withdrawn Decision, check EFIS"

ERC Recommendation 70-03: " Relating to the use of Short Range Devices (SRD)"

ETSI EN 300 674: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band; Part 1: General characteristics and test methods for Road Side Units (RSU) and On-Board Units (OBU)"

CENELEC EN 12253: "" Road transport and traffic telematics - Dedicated short-range communication - Physical layer using microwave at 5.8 GHz"

ECC Report 68: "Compatibility studies in the band 5 725-5 875 MHz between Fixed Wireless Access (FWA) systems and other systems"

ITU-R Recommendation P.452: "Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz"

EC Decision 2006/771/EC: "Commission Decision of 9 November 2006 on the harmonisation of the radio spectrum for use by short-range devices"

IARU Region-1 VHF Managers Handbook

Recommendation ITU-R M.1732-1: "Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies"

ECC Report 210: "Compatibility/sharing studies related to Broadband Direct-Air-to-Ground Communications (DA2GC) in the frequency bands 5 855-5 875 MHz, 2 400-2 483.5 MHz and 3 400-3 600 MHz"

ETSI TR 101 599: "Electromagnetic compatibility and Radio spectrum matters (ERM) System Reference Document (SRDoc); Broadband Direct-Air-to-Ground Communications System employing beamforming antennas, operating in the 2,4 GHz and 5,8 GHz bands"

ETSI TR 103 108: "Electromagnetic compatibility and Radio spectrum Matters (ERM); System Reference document (SRdoc); Broadband Direct-Air-to-Ground Communications System operating in the 5,855 GHz to 5,875 GHz band using 3G technology"

ECC Report 206: "Compatibility studies in the band 5 725-5 875 MHz between SRD equipment for wireless industrial applications and other systems"

IEEE 802.11-2012: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

ETSI EN 302 571 V1.2.2 (2011-10): "Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive"

Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain"

ERC Recommendation 74-01: "Unwanted Emissions in the Spurious Domain"

Recommendation ITU-R F.1336-3: "Reference radiation patterns of omnidirectional, sectoral and other antennas for the fixed and mobile service for use in sharing studies in the frequency range from 400 MHz to about 70 GHz"

ETSI ES 200 674-1 V2.4.1 (2013-05): “Technical characteristics and test methods for High Data Rate (HDR) data transmission equipment operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band”

NTIA 5 GHz report, January 2013, “Evaluation of the 5 350-5 470 MHz and 5 850-5 925 MHz bands pursuant to section 6406(b) of the middle class tax relief and job creation act of 2012

ETSI TR 102 960: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (RTTT DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range; Evaluation of mitigation methods and techniques"

ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range"

http://www.statistiques-mondiales.com/union_europeenne.htm

http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=lfst_hhnhtych&lang=en

https://www.ssb.no/en/befolkning/statistikker/familie/aar/2014-12-12

http://www.bfs.admin.ch/

http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=bd_9ac_l_form_r2&lang=en

3GPP TR 36.889: Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum, http://www.3gpp.org/DynaReport/36889.htm

CEPT Report 57 : "Compatibility and sharing conditions for WAS/RLAN in the bands 5 350-5 470 MHz and 5 725-5 925 MHz"

ECC Report 181 on improving spectrum efficiency in SRD bands

JRC study on RLAN deployment and device densities:  http://www.cept.org/Documents/se-24/25709/SE24(15)070R0_WI52_number-of-RLAN-JRC

JRC study on in-home RLAN coverage and number of RLAN APs per household: http://www.cept.org/Documents/se-24/25805/SE24(15)089R0_WI52_Further-considerations-on-WLAN-indoor-coverage

JRC measurements of 802.11ac RLAN APs operating in the 5 GHz band: http://www.cept.org/Documents/se-24/24051/SE24(15)042R0_WI52_JRC_5GHZ_RLAN-_measurements

ECC Report 244 “Compatibility studies related to RLANs in the 5 725-5 925 MHz band”

______________

____________________

Figure 4.35, "Communications Market Report 2015", Ofcom, 04 August 2016, https://www.ofcom.org.uk/__data/assets/pdf_file/0024/26826/cmr_uk_2016.pdf.

This system is a two-satellites constellation.

Lower gain can be used for the wider beams.

Antenna beam “incident angles”.

Average e.i.r.p. over a pulse repetition interval.

Max e.i.r.p. during pulse transmission.

Antenna gain varies depending on antenna location (mid or side), and incident angle.

2009 status, based on market reports from IMS Research, ABI Research, iSupply, and Plum Consulting.

As per Recommendation ITU-R M.1739, the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB.  Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis