| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/171 | Annex 29 to Document 5A/469-E |
|  | 12 June 2017 |
|  | English only |
| Annex 29 to Working Party 5A Chairman’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLAN SHARING] |  |
| Sharing and compatibility studies of WAS/RLAN in the 5 GHz frequency range |  |

[Editor’s Note: This document is a compilation of material presented in contributions submitted to May and November, 2016 and May 2017 WP 5A meetings (see Source Document references above and Source Indication below) that the submitting administrations requested to be considered in developing this document. The content of this document need to be supported by corresponding sharing studies. The material contained in this document has not been agreed by WP 5A.The material if agreed could be used to satisfy the objective of agenda item 1.16.]

# 1	Introduction

This Report includes the sharing and compatibilities studies of WAS/RLAN in the 5 GHz frequency range.

It is intended to represent the response to invites ITU-R c), d), e) and f) of Resolution 239 (WRC-15) under WRC-19 agenda item 1.16.

# 2	Scope of the sharing and compatibility of WAS/RLAN with other services in the 5 GHz range.

The World Radiocommunications Conference 2015 decided on the draft agenda for the upcoming World Radiocommunications Conference scheduled for 2019. Among other items, WRC-19 agenda item 1.16 addresses the need of studies on regulatory actions and possible additional spectrum allocations to the mobile service, including radio local area networks (WAS/RLAN). Indeed, WRC-19 agenda item 1.16 reads:

1.16	to consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15).

The related Resolution 239 (WRC-15) to the WRC-19 agenda item 1.16 deals with studies concerning Wireless Access Systems including radio local area networks in the frequency bands between 5 150 MHz and 5 925 MHz. The Resolution invites ITU-R to conduct and complete the following in time for WRC-19:

a)	to study WAS/RLAN technical characteristics and operational requirements in the 5 GHz frequency range;

b)	to conduct studies with a view to identify potential WAS/RLAN mitigation techniques to facilitate sharing with incumbent systems in the frequency bands 5 150-5 350 MHz, 5 350-5 470 MHz, 5 725-5 850 MHz and 5 850-5 925 MHz, while ensuring the protection of incumbent services including their current and planned use;

c)	to perform sharing and compatibility studies between WAS/RLAN applications and incumbent services in the frequency band 5 150-5 350 MHz with the possibility of enabling outdoor WAS/RLAN operations including possible associated conditions;

d)	to conduct further sharing and compatibility studies between WAS/RLAN applications and incumbent services addressing:

i)	whether any additional mitigation techniques in the frequency band 5 350-5 470 MHz beyond those analysed in the studies referred to in recognizing a) would provide coexistence between WAS/RLAN systems and EESS (active) and SRS (active) systems;

ii)	whether any mitigation techniques in the frequency band 5 350-5 470 MHz would provide compatibility between WAS/RLAN systems and radio determination systems;

iii)	whether the results of studies under points i) and ii) would enable an allocation of the frequency band 5 350-5 470 MHz to the mobile service with a view to accommodating WAS/RLAN use;

e)	to also conduct detailed sharing and compatibility studies, including mitigation techniques, between WAS/RLAN and incumbent services in the frequency band 5 725-5 850 MHz with a view to enabling a mobile service allocation to accommodate WAS/RLAN use;

f)	to also conduct detailed sharing and compatibility studies, including mitigation techniques, between WAS/RLAN and incumbent services in the frequency band 5 850-5 925 MHz with a view to accommodating WAS/RLAN use under the existing primary mobile service allocation while not imposing any additional constraints on the existing services,

# 3	Overall view of allocations in the 5 GHz range

| Allocation to services |  |  |  |  | Expected studies |  |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |  |  |
| 5 150-5 250 FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile 5.446A 5.446B AERONAUTICAL RADIONAVIGATION 5.446 5.446C 5.447 5.447B 5.447C |  |  |  |  | Coexistence between WAS/RLAN outdoor operations and FSS (feederlinks for non-GSO) and Aeronautical Radionavigation |  |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH 5.447D 5.447E 5.448 5.448A |  |  |  |  | Coexistence between WAS/RLAN outdoor operations and EESS (active), Radiolocation and SRS (active) |  |
| 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH (active) 5.447E 5.448 5.448A |  |  |  |  |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B RADIOLOCATION 5.448D AERONAUTICAL RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448C |  |  |  |  | Further sharing and compatibility studies between WAS/RLAN applications and incumbent services addressing whether additional mitigation techniques would provide coexistence between WAS/RLAN systems and EESS (active), radio determination and SRS (active) systems (see invites ITU-R d) of Res. 239) |
| 5 460-5 470 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION 5.448D RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448B |  |  |  |  |  |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur |  | 5 725-5 830 RADIOLOCATION Amateur |  |  | Coexistence between WAS/RLAN and FSS and Radiolocation |
| 5.150 5.451 5.453 5.455 5.456 |  | 5.150 5.453 5.455 |  |  |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |  |  |
| 5.150 5.451 5.453 5.455 5.456 |  | 5.150 5.453 5.455 |  |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation | Coexistence between WAS/RLAN under the current MS allocation and FS and FSS. |
| 5.150 |  | 5.150 |  | 5.150 |  |

# 4	Assumptions on technical and operational elements for the sharing and compatibility of WAS/RLAN with other services

## 4.1	Technical and operational characteristics of the WAS/RLAN operating in the 5 GHz ranges

Option 1

[RUS 5A/196]

[Editor’ Note: the text below needs to be verified after finalization of the document ITU-R Report M.[RLAN REQ-PAR]]

Technical and operational characteristics of RLANs are presented in Recommendation ITU-R M.1450 «Characteristics of broadband radio local area networks». In accordance with this Recommendation in the territory of USA and Canada e.i.r.p. of RLANs operating in the frequency band 5 250–5 350 MHz is 250 mW conducted (-6 dBW) and 1 000 mW conducted (0 dBW) in the frequency band 5 750-5 850 MHz. In Canada, the e.i.r.p. of RLANs operating in the frequency band 5 150-5 250 MHz is 250 mW conducted (-6 dBW).  In the U.S., the e.i.r.p. of RLANs operating in the frequency band 5 150-5 250 MHz is 1 000 mW conducted (0 dBW), however outdoor operations with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW e.i.r.p., and all WAS/RLAN emissions outside of that band must be below -27 dBm/MHz.  At the same time RLANs operating in the territory of Europe, and in numerous Region 3 countries including Australia, are restricted to an e.i.r.p. of 200 mW (-7 dBW) in the frequency bands 5 150-5 250 MHz and 5 250-5 350 MHz and, in the lower 5 150-5 250 MHz segment, to indoor only operation.

At the same time, under preparation for WRC-15 agenda item 1.1, the issue of the operational feasibility of RLANs in the frequency band 5 350-5 470 MHz was considered. It was assumed that the RLAN transmitter e.i.r.p. would be 200 mW (-7 dBW) in this frequency band.

Therefore two e.i.r.p. values were used for compatibility assessment of RLANs with radiodetermination radars in the frequency bands 5 350-5 460 MHz, 5 460-5 470 MHz and 5 725-
5 850 MHz:

–	e.i.r.p. of minus 7 dBW was assumed for RLAN operating in the frequency bands 5 350-5 470 MHz;

–	e.i.r.p. of zero (0) dBW was assumed for RLAN operating in the frequency bands 5 725-5 850 MHz;

e.i.r.p. spectral densities specified in Recommendation ITU-R М.1450 shows that it addresses RLANs having carrier bandwidth of 20 MHz. However taking in account the achievements in RLANs development such as IEEE standard 802.11ac, the considered Report includes analysis of networks having carrier bandwidth of both 20 MHz and 160 MHz.

[UK and ESA 5A/246, 96]

Option 2

### 4.1.1	Characteristics of RLAN in 5 150-5 250 MHz Band

### 4.1.2	Characteristics of RLAN in 5 250-5 350 MHz Band

### 4.1.3	Characteristics of RLAN in 5 350-5 470 MHz Band

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

4.1.4	Characteristics of RLAN in 5 725-5 850 MHz band

Table 4.1.4-1

Basic RLAN (Wi-Fi) transmitter characteristics in the band 5 725-5 850 MHz

|  | RLAN 1 Omni-Indoor | RLAN 2 Omni Outdoor | RLAN 3 Directional Outdoor |
| --- | --- | --- | --- |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 | 30 | 30 |
| Bandwidth (MHz) | 20/40/80/160 | 20/40/80/160 | 20/40/80/160 |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 | 17/14/11/8 | 17/14/11/8 |
| Typical AP Antenna Type | Omni (azimuth) See Table 4.6.4-7, Table 4.8.4-9 and Table 4.10.4-11 Type 1 and 2 | Omni (azimuth) See Table 4.10.4-11 , Type 1 and 2 | Directional, See Table 4.10.4-11, Type 3 and 4 |
| AP Antenna directivity gain (dBi) | 0-6 | 6-7 | 12/18 |

The figure below provides the spectrum mask for RLAN as function of the nominal channel bandwidth, typically 20, 40, 80 or 160 MHz

Figure 4.1.4-1

Spectrum mask for RLAN in 5 725-5 850 MHz

The assumed average channel bandwidths distribution of RLAN devices is given in the following table.

Table 4.2.4-3

RLAN channel bandwidth distribution in 5 725-5 850 MHz

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |

The next table provides RLAN receiver parameters for the purpose of compatibility studies with RLAN as a victim.

Table 4.4.4-5

Basic RLAN receiver characteristics in the band 5 725-5 850 MHz

| System parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Bandwidth (MHz) | 20 | 40 | 80 | 160 |
| kTB dBm / bandwidth | -101 | -98 | -95 | -92 |
| Typical Noise figure dB | 4 |  |  |  |
| Noise Power (dBm / bandwidth) | -97 | -94 | -91 | -88 |
| Typical Sensitivity for MCS0, BPSK (½ coding rate) (dBm) | -92 | -89 | -86 | -83 |
| C/N for MCS0, BPSK (½ coding rate) (dB) | 5 |  |  |  |
| I/N (dB) (Note 1) | -6 |  |  |  |
| C/I (dB) | 11 for I/N -6 dB; 5 for I/N 0 dB |  |  |  |
| Maximum antenna gain at the RLAN Access Point (dBi) | See Table 4.8.4-9 and Table 4.10.4-11 |  |  |  |
| Maximum antenna gain at the RLAN user device (dBi) | See Table 4.6.4-7 |  |  |  |
| Note 1: As per Recommendation ITU-R M.1739, the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB. Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis. |  |  |  |  |

RLAN antenna patterns

The characteristics in Table 4.6.4-7 are representative of an average antenna for all User Equipment within a population of RLAN devices operating at 5 725-5 850 MHz. User Equipment can be defined as mobile or portable devices such as smart phones, tablets, notebooks, wireless scanners etc.

Table 4.6.4-7

RLAN User Equipment antenna (mobile/portable device)

| # | Type | Gain (dBi) | Antenna height above ground (m) |
| --- | --- | --- | --- |
| 1 | Omni-directional Antenna | 1.3 | 1 to 1.5 |
| NOTE: This value is the averaged value obtained from a survey on RLAN UE antennas. For simplicity, this antenna is assumed to be isotropic. |  |  |  |

The antenna pattern in Table 4.8.4-9 is considered as a representative average antenna pattern for indoor access points within the RLAN population operating at 5 725-5 850 MHz. The table specifies the gains available at elevation angles; the antenna pattern is omni-directional in azimuth.

Table 4.8.4-9

Example of Indoor RLAN Access Point Omni-directional (azimuth) Antenna - 
Elevation Pattern

| Elevation angle θ (Degrees) | Gain (dBi) |
| --- | --- |
| 45 θ 90 | -4 |
| 35 θ 45 | 0 |
| 0 θ 35 | 3 |
| –15 θ 0 | -1 |
| –30 θ –15 | -4 |
| –60 θ –30 | -9 |
| –90 θ –60 | -8 |

The elevation angles in Table 4.8.4-9 are defined from the viewpoint of the RLAN Access Point when mounted to the ceiling. Positive elevation angles are towards the ground and negative elevation angles are towards the sky (typically, the RLAN Access Point is installed for optimal coverage). The pattern is normalised to 3 dBi gain on boresight.

Table 4.10.4-11 sets out the characteristics of RLAN antennas used on fixed indoor or outdoor equipment such as Access Points, Bridges, P2P or P2MP installations. The corresponding antenna patterns are provided in Figure 4.12.4-1 to Figure 4.16.4-4 below.

Table 4.10.4-11

Typical Fixed indoor and outdoor RLAN antenna 
(access points, bridges, P2P and P2MP)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 6 | Indoor & Outdoor | Figure 4.12.4-1 | 6 to 28,5 |
| 2 | Directional Antenna (sector) | 6 | Indoor & Outdoor | Figure 4.13.4-2 |  |
| 3 | Directional Antenna | 12 | Outdoor | Figure 4.14.4-3 |  |
| 4 | Directional Antenna (sector) | 17 | Outdoor | Figure 4.15.4-4 |  |
| NOTE: The (Highly) directional links are often installed on top of buildings |  |  |  |  |  |

Figure 4.12.4-1

RLAN 6 dBi Omni – Elevation (left) and Azimuth (right) Radiation Patterns

Figure 4.13.4-2

RLAN 6 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure 4.14.4-3

RLAN 12 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure 4.15.4-4

RLAN 17 dBi Sector Antenna – Elevation (blue) and Azimuth (red) Radiation Patterns

RLAN power distribution

Table 4.16.4-17

RLAN power distribution

| Tx power e.i.r.p. | 1 W (directional) | 1 W (omni) | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

The RLAN power distribution presented here leads to a 5.3 % use of outdoor devices. Sensitivity analysis may be performed with other outdoor use ratio to assess the impact of this parameter on the compatibility studies. Outdoor use ratios from 2-5 % is suggested.

### 4.1.5	Characteristics of RLAN in 5 850-5 925 MHz band

[ESA/EUMETSAT 5A/96, 97]

RLAN parameters used in the studies with EESS are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)). They are also reiterated in the current working document towards a preliminary draft Report ITU-R M.[RLAN REQ-PAR] and concern:

- E.i.r.p. distributions

- Indoor/outdoor ratio

- Channel Bandwidth distribution

- Propagation conditions

- Antenna gain/discrimination

- Number of active RLAN.

## 4.2	Technical and operational characteristics of FSS links used for MSS feeder links in the 5 150-5 250 MHz

[Editor’s Note: Parameters and antenna pattern below need confirmation from WP 4A]

[Globalstar 5A/395]

The parameters of the feeder uplinks of the HIBLEO-X MSS system are summarized in the Table below.

TABLE 2

MSS Feeder Link Parameters

| Parameter | HIBLEO-X |
| --- | --- |
| Satellite orbit altitude h (km) | 1 414 |
| Satellite Inclination (degrees) | 52 |
| Frequency Range (MHz) | 5 091-5 250 |
| Satellite receiver bandwidth B (MHz) | 1.23 |
| I/N (dB) | -12.2 |
| Satellite receiver noise temperature T (K) | 550 |
| Pnoise, add (dBW) | -140.3 |
| Iadd (dBW) | -152.5 |
| Polarization discrimination Lp (dB) | 1 |

The spacecraft receive antenna is an “iso-flux” antenna and the gain pattern is shown below.

Figure 3

Spacecraft Receive Antenna Pattern

## 4.3	Technical and operational characteristics of the Aeronautical Radionavigation service operating in the 5 150-5 250 MHz and 5 350-5 460 MHz

4.3.1	5 150-5 250 MHz band

[Editor’s Note: Parameters below need confirmation from WP 5B]

[RUS 5A/397]

In accordance with Recommendation ITU-R М.2007, ARNS systems operate in the frequency band 5 150−5 250 MHz all over the world. In compliance with recommends 1 Recommendation ITU-R M. 2007 that the technical and operational characteristics of the radars operating in the ARNS described in Annex 1 should be considered representative of those operating in the frequency band 5 150-5 250 MHz and used in studies of compatibility with systems in other services. Table 1 below provides the technical parameters of airborne sense and avoid systems to be used for consequence assessment of outdoor WAS/RLAN usage.

TABLE 1

Technical parameters of aircraft based sense and avoid radar

| Parameter |  | Radar No. 1 |
| --- | --- | --- |
| Platform height (km) |  | Up to 20 |
| Radar type |  | Air to air traffic collision avoidance system |
| The range of measured ground speed (km/h) |  | Up to 1 500 |
| Frequency tuning range (MHz) |  | 5 150-5 250 |
| Emission type |  | Linear FM (LFM) pulse |
| LFM chirp bandwidth (MHz) |  | 20 |
| Pulse rise and fall times (s) |  | 0.1-0.2 |
| RF emission bandwidth | –3 dB –20 dB (MHz) –40 dB | 18 22 26 |
| Receiver IF –3 dB bandwidth (MHz) |  | 30 |
| Receiver noise figure (dB) |  | 5 |
| Antenna gain (dBi) |  | 33-36 |
| First antenna side lobe (dBi) |  | 18-20 |
| Horizontal beamwidth (degrees) |  | 8 |
| Vertical beamwidth (degrees) |  | 8 |
| Polarization |  | Vertical |
| Vertical antenna scan (degrees) |  | ±45 |
| Horizontal antenna scan (degrees) |  | ±45 |
| Protection criteria (dB) |  | −6 |

4.3.2	5 350-5 460 MHz band

[RUS 5A/196]

Recommendation ITU-R M.1638-1 contains description of one radar of the aeronautical radionavigation service operating in the frequency band 5 350-5 460 MHz (Radar 16). Its technical characteristics and protection criterion are given in Table 4.1. The protection criterion I/N is of minus 10 dB as specified in the Table and it corresponds to Section 4 “Protection criteria” of Recommendation ITU-R М.1638-1.

TABLE 4.1

Technical characteristics and protection criteria of radars in the aeronautical radionavigation service 
operating in the frequency bands 5350-5460 MHz

| Radar | Radar 16 |
| --- | --- |
| Location | Airborne |
| Frequency band, MHz | 5 440 |
| Antenna gain, dB | 34 |
| Noise figure, dB | 5 |
| IF bandwidth, MHz | 1 |
| I/N, dB | -10 |
| Тn, К | 627 |
| Рnoise, add, dBW | -141 |
| Iadd, dBW | -151 |

These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using the following equations:

К	(1)

dBW	(2)

where:

k –	Boltzmann constant;

NF – 	radar receiver noise figure;

–	radar receiver IF operational pass-band.

Maximum permissible noise power at the front end was assessed such as:

, dBW	(3)

The estimated thermal noise level, noise power and permissible interference power are given in Table 4.1.

## 4.4	Technical and operational characteristics of the Earth Exploration Satellite service operating in the frequency ranges 5 250-5 570 MHz

[7C 5A/204]

The band 5 250-5 570 MHz, allocated to EESS (active) on a primary basis, is currently used by many Administrations operating EESS (active) sensors and is planned to be used by a number of additional sensors. Typically, this band is used by the following type of sensors:

–	Synthetic aperture radars (SAR) with operations typically limited to the 5 350-5 470 MHz band.

–	Altimeters with operations typically covering the whole 5 250-5 570 MHz band.

–	Scatterometers with small bandwidths and operations typically within the 5 250-5 350 MHz band or the 5 350-5 470 MHz band.

Studies under WRC-15 AI 1.1 mainly focused on SAR missions, however all types of EESS (active) sensors require relevant protection. Therefore, all mitigation techniques to be studied have to assess protection of all existing and planned SAR, altimeters and scatterometers sensors. Each sensor type has different technical characteristics.  Within each sensor type, however, the characteristics present similar modes of operation.

The table in Annex 1 provides a non-exhaustive listing of existing and planned EESS (active) systems known to date and Annex 2 provides detailed characteristics of some of these EESS (active) systems.

Prior studies included in Annex 35 to Document 4-5-6-7/715 (Chairman’s Report) depicted substantial negative margins for EESS (active) SAR systems, while additional studies are needed for altimeters and scatterometers. However, one can envision similar results as those found with SARs, since aggregate interference from RLANs to EESS (active) is mainly controlled by the sensors antenna gain and the number of active RLANs within the sensor footprint.  Altimeters and scatterometers present lower antenna gain but, by direct effect, larger footprint and hence higher number of active RLAN to be considered in the aggregation calculations.

In case more detailed dynamic studies with altimeters and scatterometers were found necessary, the relevant EESS (active) protection criteria to be used for these sensors (from Recommendation ITU-R RS.1166-4) are given below, together with the SAR interference criteria:

Table 2

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Scatterometer | 8% degradation in measurement of normalized radar backscatter to deduce wind speeds | –5 | 99 | 95 |
| Altimeter | 4% degradation in height noise | –3 | 99 | 95 |
| Synthetic Aperture Radar | 10% degradation of standard deviation of pixel power | -6 | 99 | 95 |

Note : Since interference is most likely to be produced by an aggregation of RLAN interferers, it would be related to population densities and interference would hence more than likely be systematic. The percentage of data availability to be used in the RLAN case is therefore 99%.

## 4.6	Technical and operational characteristics of the Radionavigation service operating in the 5 460-5 470 MHz

[UK 5A/246]

Editor’s Note: Individual Radar highlighted in the tables below may operate across one of the sub bands in the 5GHz range or across more than one of these sub bands. In addition, some of the radar highlighted in the tables below may already covered by the existing mitigation techniques in Recommendation ITU-R M.1652-1. Future drafts should make the usage situation across the bands clearer and which radar are already covered by the existing mitigation techniques shown in Recommendation ITU-R M.1652-1

[RUS 5A/196]

Recommendation ITU-R M.1638-1 contains description of two radars of the radionavigation service. The technical characteristics and protection criterion are given in Table 4.2. The protection criterion I/N is of minus 10 dB as specified in the Table and it corresponds to section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.2.

TABLE 4.2

Technical characteristics and protection criteria of radars in the radionavigation service 
operating in the frequency bands 5 460-5 470 MHZ

| Radar | Radar 10 | Radar 10A |
| --- | --- | --- |
| Location | Shipborne, ground | Ground (bistatic) |
| Frequency band, MHz | 5 250-5 875 | 5 250–5 875 |
| Antenna gain, dB | 33 | 33 |
| Noise figure, dB | 3 | 3 |
| IF bandwidth, MHz | 11.0 | 11.0 |
| I/N, dB | -10 | -10 |
| Тn, К | 289 | 289 |
| Рnoise, add, dBW | -134 | -134 |
| Iadd, dBW | -144 | -144 |

## 4.7	Technical and operational characteristics of the Radiolocation service operating in the 5 250-5 470/5 725-5 850 MHz

[UK 5A/246]

Editor’s Note: Individual Radar highlighted in the tables below may operate across one of the sub bands in the 5GHz range or across more than one of these sub bands. In addition, some of the radar highlighted in the tables below may already covered by the existing mitigation techniques in Recommendation ITU-R M.1652-1. Future drafts should make the usage situation across the bands clearer and which radar are already covered by the existing mitigation techniques shown in Recommendation ITU-R M.1652-1

### 4.7.1	Technical characteristics of Radiolocation systems operating in 5 250-5 350 MHz band

[Editor’s Note: Parameters below need confirmation from WP 5B]

[RUS 5A/398]

The technical characteristics of the radiodetermination radars operating in the frequency band 5 250−5 850  MHz are given in Recommendations ITU-R М.1638-1 “Characteristics of and protection criteria for sharing studies for radiolocation (except ground based meteorological radars) and aeronautical radionavigation radars operating in the frequency bands between 5 250 and 
5 850 MHz” and M.1849-1 “Technical and operational aspects of ground-based meteorological radars”.

Analysis of Recommendation ITU-R M.1638-1 showed that radars operating in separate bands of the considered frequency range are airborne and ground based. Table 1 below provides the technical parameters of airborne radars from this Recommendation used in the compatibility studies.

TABLE 1

Technical parameters and protection criteria of airborne radars operating 
in the frequency band 5 250-5 350 MHz

| Radar |  | Radar No. 8 | Radar No. 9 |
| --- | --- | --- | --- |
| Band, MHz |  | 5300 | 5 250-5 725 |
| Antenna gain, dB |  | 26 | 40 |
| Noise figure, dB |  | 4.9 | 3.5 |
| Intermediate frequency band, MHz |  | 90 | 1 |
| I/N, dB |  | -10 | -10 |
| Тn, К |  | 606 | 359 |
| Рadd noise, dBW |  | -121 | -143 |
| Iadd, dBW |  | -131 | -153 |

The technical characteristics of ground based radars to be used in the compatibility studies are given in Table 2.

TABLE 2

Technical parameters and protection criteria of ground based radars operating 
in the frequency band 5 250-5 350 MHZ

| Radar | Radar 1 | Radar 6 | Radar 10 | Radar 11 | Radar 19 | Radar 21 | Radar 23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Band , MHz | 5 300 | 5 300 | 5 250-5 875 | 5 250-5 350 | 5 300-5 700 | 5 300-5 750 | 5 250-5 850 |
| Antenna gain,dB | 38.3 | 28 | 33 | 16 | 44.5 | 44.5 | 31.5 |
| Noise figure, dB | 6 | 5 | 3 | 10 | 3 | 3 | 13 |
| Intermediate frequency band, MHz | 1.0 | 1.5 | 11.0 | 11.0 | 0.75 | 0.8 | 5 |
| I/N, dB | -6 | -6 | -6 | -6 | -6 | -6 | -6 |
| Тn, К | 865 | 627 | 289 | 2610 | 289 | 289 | 5496 |
| Рnoise, dBW | -139 | -139 | -134 | -124 | -145 | -145 | -124 |
| Iadd, dBW | -145 | -145 | -140 | -130 | -151 | -151 | -130 |

The protection criteria for each considered radar are not given in Recommendation ITU-R М.1638-1. This Recommendation notes that:” An increase of noise power spectral density of about 1 dB for the radiolocation radars except ground based meteorological radar would constitute significant degradation. Such an increase corresponds to an (I + N )/N ratio of 1.26, or an I/N ratio of about −6 dB. For the radionavigation service and meteorological radars considering the safety-of-life function, an increase of about 0.5 dB would constitute significant degradation. Such an increase corresponds to an I /N ratio of about –10 dB.” Therefore the interference-to-noise ratio (I/N) of minus 6 dB used as the protection criteria for the radiolocation radars is given in Tables 1 and 2.

The interference-to-noise ratio (I/N) of minus 10 dB is used as the protection criteria for the meteorological radars described in Table 3.

In accordance with Recommendation ITU -R М.1849-1 6 types of meteorological radars operate in the frequency band 5 250-5 350 MHz. The technical characteristics and protection criteria are given in Table 3.

TABLE 3

Technical parameters and protection criteria of ground based meteorological radars operating 
in the frequency band 5 250-5 350 MHZ

| Radar | Radar 1 | Radar 4 | Radar 8 | Radar 11 | Radar 12 | Radar13 |
| --- | --- | --- | --- | --- | --- | --- |
| Band, MHz | 5 300-5 700 | 5 300-5 700 | 5 250-5 725 | 5 250-5 350 | 5 330-5 370 | 5 250-5 370 |
| Antenna gain,dB | 39 | 40 | 45 | 45 | 45 | 50 |
| Noise figure, dB | 7 | 3 | 3 | 1.2 | 1.9 | 1 |
| Intermediate frequency band, MHz | 0.5 | 0.6 | 10 | 1.6 | 1.4 | 1.4 |
| I/N, dB | -10 | -10 | -10 | -10 | -10 | -10 |
| Тn, К | 1163 | 289 | 289 | 92 | 159 | 75 |
| Рnoise, dBW | -141 | -146 | -134 | -147 | -145 | -148 |
| Iadd, dBW | -151 | -156 | -144 | -157 | -155 | -158 |

### 4.7.2	Technical characteristics of Radiolocation systems operating in 5 350-5 470 MHz band

[RUS 5A/196]

Recommendation ITU-R M.1638-1 contains description of 16 radars of the radiolocation service operating in the frequency bands 5 350-5 470/5 725-5 850 MHz. The technical characteristics and protection criterion are given in Table 4.3. The protection criterion I/N is of minus 6 dB as specified in the Table and it corresponds to section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.3.

TABLE 4.3

Technical characteristics and protection criteria of ground based radars in the radiolocation service 
 operating in separate frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

| Radar | Radar 2 | Radar 3 | Radar 4 | Radar 5 | Radar 7 | Radar 9 |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Ground | Ground | Ground | Ground | Ground, shipborne | Airborne |  |
| Frequency band, MHz | 5 350-5 850 | 5 350-6 850 | 5 400-5 900 | 5 400-5 900 | 5 450-5 825 | 5 250-5 725 |  |
| Antenna gain, dB | 54 | 47 | 45.9 | 42 | 30 | 40 |  |
| Noise figure, dB | 5 | 5 | 11 | 5 | 10 | 3.5 |  |
| IF bandwidth, MHz | 0.25 | 1.0 | 2.0 | 8.0 | 1.0 | 1 |  |
| I/N, dB | -6 | -6 | -6 | -6 | -6 | -6 |  |
| Тn, К | 627 | 627 | 3361 | 627 | 2610 | 359 |  |
| Рnoise, add, dBW | -147 | -141 | -130 | -132 | -134 | -143 |  |
| Iadd, dBW | -153 | -147 | -136 | -138 | -140 | -153 |  |
| Radar | Radar 12 | Radar 13 | Radar 15 | Radar 17 | Radar 18 | Radar 19 |
| Location | Ground, shipborne | Ground | Ground | Airborne | Ground | Ground |
| Frequency band, MHz | 5 400-5 900 | 5 450-5 850 | 5 400-5 850 | 5 370 | 5 600-5 650 | 5 300-5 700 |
| Antenna gain, dB | 25 | 43 | 42 | 37.5 | 38.5 | 44.5 |
| Noise figure, dB | 4 | 3 | 2.3 | 6 | 3 | 3 |
| IF bandwidth, MHz | 7.0 | 2.75 | 20 | 0.6 | 4 | 0.75 |
| I/N, dB | -6 | -6 | -6 | -10 | -6 | -6 |
| Тn, К | 438 | 289 | 202 | 865 | 289 | 289 |
| Рnoise, add, dBW | -134 | -140 | -133 | -141 | -138 | -145 |
| Iadd, dBW | -140 | -146 | -139 | -151 | -144 | -151 |
| Radar | Radar 20 | Radar 21 | Radar 22 | Radar 23 |  |  |
| Location | shipborne | Ground | Ground | Ground |  |  |
| Frequency band, MHz | 5 400-5 700 | 5 300-5 750 | 5 400-5 850 | 5 250-5 850 |  |  |
| Antenna gain, dB | 40 | 44.5 | 35 | 31.5 |  |  |
| Noise figure, dB | 2 | 3 | 5 | 13 |  |  |
| IF bandwidth, MHz | 0.5 | 0.8 | 4 | 5 |  |  |
| I/N, dB | -6 | -6 | -6 | -6 |  |  |
| Тn, К | 170 | 289 | 627 | 5496 |  |  |
| Рnoise, add, dBW | -149 | -145 | -135 | -124 |  |  |
| Iadd, dBW | -155 | -151 | -141 | -130 |  |  |

6 meteorological radars operating in the frequency band 5 350-5 470 MHz are given in Recommendation ITU-R М.1849-1. Their technical characteristics and protection criteria are reflected in Table 4.4. The analysis of the presented data shows that one meteorological radar –Radar 8 operates also in the portion of the frequency band 5 725-5 850 MHz (5 725-5 750 MHz). The protection criterion I/N is of minus 10 dB as specified in Table 4.4 and it corresponds to Section 4 “Protection criteria” of Recommendation ITU-R М.1638-1. These characteristics were used for estimation of thermal noise level, noise power and permissible interference power for the given radars using equations (1)-(3). The estimated results are presented in Table 4.4.

TABLE 4.4

Technical characteristics and protection criteria of ground based meteorological radars

| Radar | Radar 1 | Radar 4 | Radar 8 | Radar 12 | Radar 13 | Radar 14 |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Ground |  |  |  |  |  |
| Frequency band, MHz | 5 300-5 700 | 5 300-5 700 | 5 250-5 750 | 5 330-5 370 | 5 250-5 370 | 5 430-5 470 |
| Antenna gain, dB | 39 | 40 | 45 | 45 | 50 | 45 |
| Noise figure, dB | 7 | 3 | 3 | 1,9 | 1 | 1,8 |
| IF bandwidth, MHz | 0,5 | 0,6 | 10 | 0,4 | 1,0 | 2,0 |
| I/N, dB | -10 | -10 | -10 | -10 | -10 | -10 |
| Тn, К | 1 163 | 289 | 289 | 159 | 75 | 149 |
| Рnoise, add, dBW | -141 | -146 | -134 | -151 | -150 | -144 |
| Iadd, dBW | -151 | -156 | -144 | -161 | -160 | -154 |

#### 4.7.2.1 	Technical characteristics of frequency hopping radars

[Editor’s Note: Text below comes from liaison statement 5A/295 from WP 5B]

Frequency Hopping Radar (FH):

This type of radar typically divides its allocated frequency band into channels. The radar then randomly selects a channel from all available channels for transmission. This random occupation of a channel can occur on a per beam position basis where many pulses on the same channel are transmitted, or on a per pulse basis.

The RLAN device must be agile (flexible) in such a way that the various combinations of frequency hopping and pulse repetition frequencies (PRF) will be taken into account and consequently be detected, even for FH Pulse Doppler radars, with high PRF.

In radars not using a fixed PRF the time between consecutive pulses follows a certain scheme and the radar uses a staggered PRF scheme. Taking into account that different radars implement different schemes to control the PRF, the RLAN DFS mechanism must be agile in the sense that the various staggered modes can be detected.

Note 2: WP 5B notes that the current version of RLAN DFS may not detect that there is power in a certain frequency slot i.e. not detect the radar and transmit in that slot.

Future radar systems:

WP 5B notes that some Members are of the view that in the future, most likely higher Duty Cycles (close to 100%) will be implemented such as CW radars. Todays pulsed system with Duty Cycles between 2.5-10% has a high peak power which is easier to detect compared to a radar presenting more or less continuous emissions with a peak power which approaches the average power. The consequence will be that low power radars will be more difficult to detect.

[FRA,SWZ 5A/435]

Frequency Hopping Radars that operate in the 5 GHz band are capable of hopping across the 5 250-5 850 MHz band. The frequencies will be selected by using a random without replacement algorithm until all frequencies have been used. After the use of all frequencies, the pattern is reset and a new random pattern is generated.

The proposed test signals in the table below are presented to improve DFS specifications if any studies are proposed between radars and RLANs in 5 350-5 470 MHz.

Frequency Hopping DFS test signals

| Frequency hopping radar type (Note 7) | Pulse width (µsec) | Pulse repetition interval (PRI) (µsec) | # Number of pulses per frequency hop | Burst length (ms) (Note 8) | Trial length (ms) | Pulse modulation (Note 9) | Minimum detection probability with 30% channel load (Note 10) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 200 (=5 kHz) | 4 | 0.8 | 480 | none | Pd>80% |
| 1 | 20 | 333 (=3 kHz) | 3 | 1 | 600 | none | Pd>80% |
| 1 | 30 | 500 (=2 kHz) | 2 | 1 | 600 | none | Pd>80% |
| 2 | 3 | 333 (=3 kHz) | 1 to 9 | # | 120 | chirp | Pd>80% |
| 2 | 10 | 500 (=2 kHz) | 1 to 9 | # | 120 | chirp | Pd>80% |
| 2 | 15 | 1000 (=1 kHz) | 1 to 9 | # | 120 | chirp | Pd>80% |
| Note 7: Radar Type 1 : Up to 600 possible frequencies (step 1 MHz) within the range 5 250-5 850 MHz, Radar Type 2: Up to 120 possible frequencies (step 5 MHz) within the range 5 250-5 850 MHz (Note 11), A frequency is selected randomly from a group of 600 (or 120 for radar Type 2) integer frequencies ranging from 5 250-5 850 MHz, using a ‘use without re-use’ scheme. Frequency test signal changes after each burst. Note 8: A burst is randomly composed of 1 to 9 pulses (n), then burst length (or hop length) = n x PRI. Note 9: Modulation used is defined in Note 2, Table D.4 (in reference of ETSI EN301893) Note 10: The proposal includes that a minimum of 30 trials per set be run with a minimum probability of detection calculated by . For ChS=10 MHz, Pd>70%; for ChS = 2 0MHz, Pd>80%. Note 11: Although these frequency hopping radar test signals hop over the entire range from 5 250-5 850 MHz, detection of these signals is only required when operating within the 5 350-5 470 MHz. |  |  |  |  |  |  |  |

#### 4.7.3	Technical characteristics of Radiolocation systems operating in 5 725-5 850 MHz band

#### 4.7.3.1 	Technical characteristics of frequency hopping radars

[Editor’s Note: Text below comes from liaison statement 5A/295 from WP 5B]

Frequency Hopping Radar (FH):

This type of radar typically divides its allocated frequency band into channels. The radar then randomly selects a channel from all available channels for transmission. This random occupation of a channel can occur on a per beam position basis where many pulses on the same channel are transmitted, or on a per pulse basis.

The RLAN device must be agile (flexible) in such a way that the various combinations of frequency hopping and pulse repetition frequencies (PRF) will be taken into account and consequently be detected, even for FH Pulse Doppler radars, with high PRF.

In radars not using a fixed PRF the time between consecutive pulses follows a certain scheme and the radar uses a staggered PRF scheme. Taking into account that different radars implement different schemes to control the PRF, the RLAN DFS mechanism must be agile in the sense that the various staggered modes can be detected.

Note 2: WP 5B notes that the current version of RLAN DFS may not detect that there is power in a certain frequency slot i.e. not detect the radar and transmit in that slot.

Future radar systems:

WP 5B notes that some Members are of the view that in the future, most likely higher Duty Cycles (close to 100%) will be implemented such as CW radars. Todays pulsed system with Duty Cycles between 2.5-10% has a high peak power which is easier to detect compared to a radar presenting more or less continuous emissions with a peak power which approaches the average power. The consequence will be that low power radars will be more difficult to detect.

[Recommendation ITU R M.1638-1 is relevant for the required sharing studies between WAS/RLAN and radiodetermination systems under Resolution 239 (WRC-15).]

## 4.8	Technical and operational characteristics of the Fixed service operating in the 5 850-5 925 MHz

[TBD]

## 4.9	Technical and operational characteristics of the Fixed Satellite service operating in the 5 725-5 850 MHz (for Region 1) and 5 850-5 925 MHz

[WP 4A 5A/181, 462]

Note: The material in the table below is derived from contributions received by WP 4A at its September/October 2016 meeting and May 2017 meeting.

Table 1

FSS Uplink Parameters (Interfered with)

| Frequency range | GHz | 5.725-5.925 | 5.725-5.925 | 5.725-5.925 |
| --- | --- | --- | --- | --- |
| CARRIER | Carrier Name | Carrier #11 | Carrier #12 | Carrier #48 |
| Noise bandwidth | MHz | 4.0-20 | 4.0-20 | 4.0-54 |
| SPACE STATION |  |  |  |  |
| Peak receive antenna gain | dBi | 20 | 36.4 | 41.6 |
| Antenna receive gain pattern and beamwidth | – | Section 1.1 of Annex 1 of Rec. ITU-R S.672-4 LS=-25 | Section 1.1 of Annex 1 of Rec. ITU-R S.672-4 LS=-25 | "Section 1.1 of Annex 1 of Rec. ITU-R S.672-4 LS=-25 Beamwidth:1.5" |
| System receive noise temperature | K | 400 | 400 | 400 -500 |
| INTERFERENCE PROTECTION CRITERIA |  |  |  |  |
| Interference to Noise Ratio I/N | dB | [-12.2] | [-12.2] | [-12.2] |
| Other |  |  |  |  |
| Additional Notes |  | - |  |  |

Table 2

FSS Uplink Parameters (Interferer)

| Frequency range | GHz | 5.725-5.925 | 5.725-5.925 | 5.725-5.925 |
| --- | --- | --- | --- | --- |
| EARTH STATION CARRIER |  | Carrier #11 | Carrier #12 | Carrier #48 |
| Antenna diameter | m | 1.8 | 13.2 | 13.2 |
| Peak transmit antenna gain | dBi | 39.9 | 57.2 | 56.4 |
| Peak transmit power spectral density (clear sky) | dBW/Hz | -33 | -33 | -28 |
| Antenna gain pattern (ITU Recommendation) | – | Rec. ITU-R 465-6 | Rec. ITU-R 465-6 | Rec. ITU-R 465-6 |
| Minimum elevation angle of transmit earth station | ° | 5 | 5 | 5 |
| Other |  |  |  |  |
| Additional Notes |  |  |  |  |

[UK 5A/246]

4.9.1 	Technical characteristics of fixed satellite service in 5 725-5 850 MHz band

In the 125 MHz portion of the band up to 5 850 MHz, this is a Region 1 allocation only. FSS deployments in Region 1 use the whole band 5 725-5 850 MHz and it is used by transmitting earth stations in the Earth-to-space direction operating only to satellites in geostationary orbits.

The following table provides details of the selection of satellites that have been taken as representative of those requiring protection in the visible portion of the geostationary orbit from Europe. In these frequency bands, the satellite beams cover very large areas of the Earth (using global, hemispherical, zonal or regional beams) as can be seen by the satellite footprint coverage plots in Annex 6 of ECC Report 068‎[18].

Table A1-18

Sample Satellite Data for the band 5 725-5 850 MHz in Region 1

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 850 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5° West | Whole band | 34 | 773 |
| B | 14° West | Whole band | 26.5 | 1200 |
| D | 3° East | Whole band | 34 | 773 |
| F | 53° East | Whole band | 26.5 | 1200 |
| G | 59.5° East | Whole band | 34 | 1200 |

Typical FSS parameters developed by the ITU are provided in Table below.

4.9.2 	Technical characteristics of Fixed satellite service in 5 850-5 925 MHz band

[LUX 5A/264]

Table A1-19

Typical FSS parameters in the 6 GHz band

| Parameter | Typical value |
| --- | --- |
| Range of operating frequencies | 5 850-6 700 MHz |
| Antenna diameters (m) | 1.2, 1.8, 2.4, 3.0, 4.5, 8, 16, 32 |
| Antenna reference pattern | Recommendation ITU-R S.465 |
| Range of emission bandwidths | 40 kHz - 72 MHz |
| Receiving space system figure of merit | +5 ↔ -10 dB/K (The database of Recommendation ITU-R S.1328 provides one example with Gsat= 24.8 dBi and Ts= 400 K, corresponding to a G/T of -1.2 dB/K |
| Earth station deployment | All regions, in all locations (rural, semi-urban, urban) |
| Earth station e.i.r.p. density towards the horizon | In accordance with RR No. 21.8 and Recommendation ITU-R S.524-9 |
| Minimum earth station antenna elevation angle, h, (degrees) | 5, 15 and 40 |

[Editor’s Note]

Sharing studies in 5 850-5 925 MHz should take into account the technical characteristics of FSS networks in all three Regions, e.g., coverage footprints.

[IARU 5A/421]

## 4.10	Technical and operational characteristics of the Amateur Radio service operating in 5 650-5 850 MHz in Regions 1 and 3 and in 5 650-5 925 MHz in Region 2

[Editor’s Note: No. 5.453 provides an additional allocation to the fixed and mobile service on a primary basis in several countries.  As the amateur and amateur-satellite service are allocated on a secondary basis further discussions are required to determine whether or not studies are necessary.]

The secondary allocation to the amateur radio service is 5 650 to 5 850 MHz in Regions 1 and 3 and to 5 650 to 5 925 MHz in Region 2. The reference document for amateur signal characteristics for sharing studies is Rec. ITU-R M.1732-2 (01/2017).

Amateur radio service activities in this frequency range and in particular in 5 760 to 5 765 MHz include terrestrial and Earth-Moon-Earth (EME) communications and weak-signal communications. These activities are typically not channelized and are very sensitive to increases in noise and interference.

### 4.10.1	Amateur systems (Morse, analogue voice and data)

| Parameter |
| --- |
| Necessary bandwidth and emission class designator | 150HA1A 150HJ2A 60H0J2B 250HF1B 2K70J3E 11K0F3E 16K0F3E 20K0F3E |
| Transmitter power (dBW) | 3 to 20 |
| Feeder loss (dB) | 1 to 6 |
| Transmitting antenna gain (dBi) | 10 to 42 |
| Typical e.i.r.p. (dBW) | 1 to 45 |
| Antenna polarization | Horizontal, vertical |
| Receiver Noise Figure (dB) | 0.5 to 1 |

While the foregoing parameters principally characterize amateur radio signals in 5 760 to 
5 765 MHz, they may be used anywhere in the allocation.

Receiver bandwidths, as indicated in the emission class designators, range from 150 Hz to 20 kHz

### 4.10.2	Amateur Earth-Moon-Earth (EME) systems

| Parameter |
| --- |
| Necessary bandwidth and emission class designator | 50H0A1A 50H0J2A 1K80F1B 1K50J2D |
| Transmitter power (dBW) | 13 to 20 |
| Feeder loss (dB) | 1 to 4 |
| Transmitting antenna gain (dBi) | 25 to 46 |
| Typical e.i.r.p. (dBW) | 50 to 65 |
| Antenna polarization | Horizontal, vertical, LHCP, RHCP |
| Receiver noise figure (dB) | 1 |

EME systems operating in 5 760 to 5 765 MHz increasingly employ digital “Weak Signal Modes” which are structured for very basic communication with low data rates and narrow bandwidth. The main antenna beam direction can be assumed to be pointing above the horizon; however, the technique is still vulnerable to noise on side lobes.

Receiver bandwidths, as indicated in the emission class designators, range from 50 Hz to 2 kHz.

### 4.10.3	Amateur systems (digital voice, data and multimedia)

Amateur mesh networks, e.g., Broadband HamNet (BBHN) systems, are implemented within the 5 725 to 5 875 MHz range shared with ISM users. However, in 5 760 to 5 765 MHz, narrowband weak signal terrestrial and EME operation is given priority.

| Parameter |
| --- |
| Necessary bandwidth and emission class designator | 2K70G1D 6K00F7D 16K0D1D 150KF1W 10M5G7W |
| Transmitter power (dBW) | 3 to 20 |
| Feeder loss (dB) | 1 to 6 |
| Transmitting antenna gain (dBi) | 10 to 42 |
| Typical e.i.r.p. (dBW) | 1 to 45 |
| Antenna polarization | Horizontal, vertical |
| Receiver noise figure (dB) | 0.5 to 1 |

Receiver bandwidths, as indicated in the emission class designators, range from 2.7 kHz to 10 MHz.

### 4.10.4	Earth-to-space uplinks for amateur satellites

The amateur service allocation in 5 GHz, particularly in 5 760 to 5 765 MHz, is also being considered for uplinks to planned geosynchronous amateur satellites.

| Parameter |
| --- |
| Necessary bandwidth and class of emission (emission designator) | 150HA1A 150HJ2A 2K70J3E 2K70J2E 16K0F3E 44K2F1D 88K3F1D 350KF1D 10M0G7W |
| Transmitter power (dBW) | 3 to 20 |
| Feeder loss (dB) | 1 to 10 |
| Transmitting antenna gain (dBi) | 10 to 42 |
| Typical e.i.r.p. (dBW) | 3 to 45 |
| Antenna polarization | Horizontal, vertical, RHCP, LHCP |
| Satellite receiver noise figure (dB) | 1 to 3 |

The receiver bandwidth of an amateur radio satellite is usually as wide as its uplink frequency band unless the transponder is equipped for demodulation and re-modulation. However, the required signal bandwidths, as indicated in the emission class designators, range from 150 Hz to 10 MHz.

## 4.11	Technical and operational characteristics of the Amateur Satellite service (space-to-Earth) operating in 5 830-5 850 MHz

[Editor’s Note: No. 5.453 provides an additional allocation to the fixed and mobile service on a primary basis in several countries.  As the amateur and amateur-satellite service are allocated on a secondary basis further discussions are required to determine whether or not studies are necessary.]

The secondary allocation to the amateur satellite service is 5 830 to 5 850 MHz. The reference document for amateur signal characteristics for sharing studies is Rec. ITU-R M.1732-2 (01/2017).

### 4.11.1		For Low Earth Orbit (LEO) satellites …

| Parameter |
| --- |
| Necessary bandwidth and emission class designators | 150HA1A 150HJ2A 2K70J3E 2K70J2E 16K0F3E 44K2F1D 88K3F1D 350KF1D 10M0G7W |
| Transmitter power (dBW) | -10 to 10 |
| Transmitting antenna gain (dBi) | 0 to 23 |
| Typical e.i.r.p. (dBW) | 0 to 15 |
| Antenna polarization | Horizontal, vertical, RHCP, LHCP |
| Receiver Noise Figure (dB) | 1 to 7 |

Receiver bandwidths, as indicated in the emission class designators, range from 150 Hz to 10 MHz

### 4.11.2	For High Earth Orbit (HEO) and Geostationary (GEO) satellites …

| Parameter |
| --- |
| Necessary bandwidth and emission class designators | 150HA1A 150HJ2A 2K70J3E 2K70J2E 16K0F3E 44K2F1D 88K3F1D 350KF1D 10M0G7W |
| Transmitter power (dBW) | 0 to 20 |
| Transmitting antenna gain (dBi) | 0 to 20 |
| Typical e.i.r.p. (dBW) | 9 to 30 |
| Antenna polarization | Horizontal, vertical, RHCP, LHCP |
| Receiver Noise Figure (dB) | 1 to 7 |

Receiver bandwidths, as indicated in the emission class designators, range from 150 Hz to 10 MHz.

# 5	Sharing studies per frequency range and per service

## 5.1	Sharing and compatibility of MSS feeder links versus WAS/RLANs in the 5 150-5 250 MHz band

[Editor’s Note: Further discussions and potential future input contributions are invited to improve the text below extracted from documents 5A/381 and 5A/404]

[AUS 5A/81]

The current, worldwide sharing rules, in the lower 5 150-5 250 MHz band, to protect co-band non-GSO MSS E-to-s feeder links, appear in Resolution 229 (Rev.WRC-12) which, inter alia, requires individual WAS/RLAN transmitters to “be restricted to indoor use with a maximum mean e.i.r.p. of 200 mW and a maximum mean e.i.r.p. density of 10 mW/MHz in any 1 MHz band or equivalently 0.25 mW/25 kHz in any 25 kHz band”.

Further background on the development of Resolution 229 (Rev.WRC-12), together with information on related ITU-R Recommendations, can be found in document 5A/81 from Australia.  Importantly, Resolution 229 (Rev.WRC-12) assumes that only 1% of RLAN deployments would operate outdoors and that the aggregate noise from WAS/RLANs into victim non-GSO MSS E-to-s feeder link satellite receivers would likely come from multiple countries.

[USA 5A/381]

The Resolution 229 (WRC-12) indoor use restriction on WAS/RLAN transmitters was based on the assumption that many different MSS companies would share the 5 150-5 250 MHz band, whereas today there is only one satellite operator that operates feeder link stations in the 5 096-5 250 MHz band.

Studies conducted by one administration concluded that the noise floor increase seen by the satellite will be a function of the aggregated energy from WAS/RLAN emissions at elevation angles above 30 degrees. By applying technological measures to operations above this elevation angle, the energy that will be received by the satellite from each individual access point would be sharply reduced, resulting in reduced aggregate noise at the satellite.  As a result, it is far less likely that harmful interference will occur, even with proliferation of access points greater than that originally presumed.

Permitting fixed access point outdoor operations at a conducted power level of up to 1 W (30 dBm), and a PSD of 17 dBm/MHz with an allowance for a 6 dBi antenna gain (i.e. a total 36 dBm e.i.r.p.), and limiting the maximum e.i.r.p. above 30 degrees elevation to 125 mW (21 dBm) e.i.r.p., provides reasonable protection from harmful interference to the MSS system.

Expressing this limit in terms of e.i.r.p. provides flexibility regarding how to design WAS/RLAN equipment, while still achieving the required levels of protection.  WAS/RLAN manufacturers will be able to demonstrate compliance with the e.i.r.p. limit by reducing antenna gain in the upward direction, or by limiting the transmitter power, or a combination of the two, as best suits their particular purpose.  Additionally, the national authority implemented a reporting requirement on any widespread deployments of outdoor access points and required WAS/RLAN operators to take corrective action in the event of any claims of harmful interference, to include reducing power, turning off devices, changing frequency bands, and/or further reducing power in the vertical direction.  To date, none such claims of interference have been made. The power limits above 30 degrees described above for individual devices, combined with the filing requirement for deployments of large numbers of devices will provide a sufficient means for avoiding harmful interference and addressing it if it does occur.  It is important to note that while in-band WAS/RLAN emissions were increased, emissions outside that band were maintained at a level of 
-27 dBm/MHz.

[AUS 5A/404]

In its 5A/404 contribution, Australia (as a major operator of LEO-D, non-GSO MSS feeder uplinks in the 5 150-5 250 MHz band) raised concerns with the USA 5A/210 doc, as has now been now been reflected in the above USA text.  Australia provided the following Tables to compare the USA’s domestic rules for RLANs in this frequency band and the mandatory requirements of Resolution 229 (Rev.WRC-12).

Table 1 provides a comparison of the US and Resolution 229 (Rev.WRC-12) rules for RLAN emission elevation angles less than or equal to 300 and Table 2 for RLAN emission elevation angles greater than 300 elevation.

TABLE 1

RLAN emission elevation angles between 00 and 300

| Parameter | Resolution 229 (Rev.WRC-12) | USA | Difference |
| --- | --- | --- | --- |
| Maximum e.i.r.p. | 200 mW (23 dBm) | 4 W (36 dBm) | 13 dB |
| Location constraint | Yes, indoor only | No, outdoor permitted |  |
| Resultant max. outdoor e.i.r.p. | 6 dBm* | 36 dBm | 30 dB* |

* Assumes building loss of 17 dB

TABLE 2

RLAN emission elevation angles >300

| Parameter | Resolution 229 (Rev.WRC-12) | USA | Difference |
| --- | --- | --- | --- |
| Maximum e.i.r.p. | 200 mW (23 dBm) | 125 mW (21 dBm) | -2 dB |
| Location constraint | Yes, indoor only | No, outdoor permitted |  |
| Resultant max. outdoor e.i.r.p. | 6 dBm* | 21 dBm | +15 dB* |

* Assumes building loss of 17 dB

Australia noted that the USA domestic rules that apply to RLANs in the 5 150-5 250 MHz frequency band potentially result in up to 30 dB (i.e. 1,000 times) more radiated power for RLAN emission elevation angles ≤300 and up to 15 dB more radiated power for elevation angles >300 when compared with that prescribed in Resolution 229 (Rev.WRC-12).

Australia also noted that the USA had not provided any technical, operational, sharing or compatibility studies to support the e.i.r.p. increase or for removing the indoor only requirement. Further, the choice of a 300 elevation angle breakpoint for maximum e.i.r.p. was not supported by reference to any studies and was inconsistent with the operation of the Australian LEO-D feeder uplinks which carry commercial traffic from 100 to the opposite 100 in elevation.

Australia asked the USA to advise WP 5A how the 300 elevation angle was chosen and to provide an analysis of the aggregate noise that would be received by LEO-D as a result of RLANs operating outdoors at the 21 dBm e.i.r.p. level (and also at the 36 dBm e.i.r.p. level).

## 5.2	Sharing and compatibility of Aeronautical radionavigation versus WAS/RLAN in the 5 150-5 250 MHz and 5 350-5 460 MHz

### 5.2.1	Sharing and compatibility studies for 5 150-5 250 MHz

[Editor’s Note: Text below need to be reviewed after propagation models used in the study confirmed by SG 3]

[RUS 5A/397]

The analysis of Recommendation ITU-R М.2007 shows that it contains only description of airborne sense and avoid systems with the maximum permissible interference-to-noise ratio (I/N) as the protection criterion at the receiver input.

Therefore the potential interference scenario given in Figure 1 was used in the studies.

It is proposed to define the maximum permissible interference level at the airborne radar receiver input to estimate the WAS/RLAN system impact to the airborne radar receivers.

Figure 1

Interference scenario for airborne radiodetermination radar receiver

Based on the obtained value the minimum protection distance ensuring compatibility of WAS/RLAN systems with airborne radar receivers can be defined.

The maximum permissible interference power at the receiver input is estimated by the following equation:

, dBW,	(1)

where:	 - required interference/noise ratio at the receiver input;

dBW,	(2)

k – Boltzmann constant,

– noise temperature, degrees К,	(3)

NF – radar noise figure, dB;

- radar operating frequency band, Hz

To estimate compatibility of RLAN with airborne radars (Scenario 1) the required protection distances were determined to ensure operation of the radars without interference in different operation modes of RLANs. The protection distance was determined in accordance with propagation model provided in Recommendation ITU-R Р.525 by the following equation:

,	(4)

where 		,	(5)

- radar antenna gain, dB;

λ – operational wavelength, m;

σ – cross-wall fading, dB.

,	(6)

where:	EIRPeff  1- effective e.i.r.p. of single WAS/RLAN transmitter;

N – number of WAS/RLAN transmitters.

The estimations assumed an aircraft flying at 10 km altitude (Н=10 000 m). Interference to operation of the air-borne receiver of sense and avoid system is caused by indoor and outdoor RLAN transmitters. The free space propagation model described in Recommendation ITU-R Р.525 is used to estimate the interference caused by RLAN transmitters. To take propagation loss in the walls into account in equation (4) additional propagation loss, σ, equal to 20 dB was considered. Multi-source interference was taken into account using equation (6).

The permissible interference power at the airborne sense and avoid receiver input was calculated by the equation (2)-(4) indicated above and is equal to minus 131.9 dBW. It was used to determine the required protection distances providing interference free radar operation in case of indoor and outdoor WAS/RLAN systems operation.

Table 3 presents the calculation results of the protection distances required between the airborne receivers of sense and avoid radar and single outdoor and indoor RLAN transmitters.

table 3

Separation distances required for protection of air-borne radars from indoor and outdoor RLAN, KM

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Protection distance | 51 | 18 | >RLOS | 180 |

*RLOS – line-of-sight distance equals 430 km for a typical flight altitude of 10 000 m and WAS/RLAN transmitter height of 20 m.

The analysis of the results provided in Table 3 shows that in case of outdoor WAS/RLAN systems usage the required protection distance can be increased significantly in comparison with the case of indoor WAS/RLAN deployment. Such usage even of single outdoor WAS/RLAN usage can lead to the case when the required protection distance will exceed the line-of-sight distance for a certain bandwidth of WAS/RLAN system.

In addition the required protection distances were estimated in case when three RLAN transmitters deployed in one building operate simultaneously. The estimation results are presented in Table 4.

TABLE 4

Separation distances required for protection of airborne radars from three indoor and outdoor rlan systems, km

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Protection distance | 88 | 31 | >RLOS | 311 |

While increasing the number of outdoor WAS/RLAN systems falling into the main lobe of the airborne sense and  avoid antenna pattern up to 7 the required protection distance exceeds the line-of-sight distance for both types of the considered WAS/RLAN signals.

The obtained results allow to conclude that the compatibility of outdoor WAS/RLAN systems with the airborne radars will be quite complicated without taking the additional measures for reducing interference.

### 5.2.2	Sharing and compatibility studies for 5 350-5 470 MHz without RLAN mitigation techniques

[RUS 5A/196]

The technical characteristics and protection criterion of one airborne radar (Radar 16) of the aeronautical radionavigation service are reflected in Table 4.1. The interference scenario from ground based RLAN to airborne Radar receiver is given in Fig. 5.1.

FIGURE 5.1

Interference scenario for RLS air-borne radar receivers

The protection distances ensuring interference free operation of this radar in different operation modes of RLAN were defined for estimation of RLAN compatibility with airborne Radar 16. Estimation of the protection distance was defined subject to the propagation model, described in Recommendation ITU-R Р.525 by the following equation:

(4)

where:

,

– 	radar antenna gain, dB;

λ – 	operational wavelength, m;

σ – 	fading in walls, dB.

In the calculations the assumed altitude of aircraft was Н=10 000 m. Interference to onboard radar are caused by outdoor and indoor RLAN transmitters located at height of 14 m, 20 m and 26 m from the Earth’s surface and using the signal with bandwidth of 20 MHz and 160 MHz. To take propagation loss in the walls into account in equation (4) additional propagation loss, σ, equal to 20 dB was considered. Multi-source interference was taken into account using the following equation.

(5)

where:

– 	aggregate interference level at the radar receiver front end;

– 	level of interference produced by the i-th RLAN transmitter at the radar receiver front end;

N – 	a number of interference sources under consideration.

Table 5.1 presents calculation results for separation distance required for protecting the air-borne radars from single indoor and outdoor RLAN transmitter.

TABLE 5.1

Separation distances (km) required for protecting air-borne radar 
from single indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 16 | >RLOS* | 268 | 76 | 27 |
| * RLOS – line-of-sight distance equals 420 km for a typical flight altitude of 10 000 m. |  |  |  |  |

Analysis of the results reflected in Table 5.1 shows that even in case of a single-source interference caused by outdoor RLAN transmitter with signal bandwidth of 20 MHz the required protection distance could exceed line-of-sight distance between an air-borne radar receiver and a RLAN transmitter. For RLAN using the signal bandwidth of 160 MHz the protection distance is several hundreds km. In case of deployment of a RLAN indoor transmitter the required protection distance could be of several tens km.

Table 5.2 presents calculation results for separation distance required for protecting the air-borne radars from three RLAN transmitters simultaneously operating in one building.

TABLE 5.2

Separation distances (km) required for protecting air-borne radars 
from three indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 16 | >RLOS | >RLOS | 130 | 46 |

Analysis of estimation results described in Table 5.2 shows that consideration of multisource interference caused by RLANs transmitters deployed in one building would result in significant increase of protection distances required for operation of air-borne radar without interference. In particular in case of three outdoor RLANs transmitters operation the required protection distance will significantly exceed the line-of-sight distance even for RLAN with bandwidth of 160 MHz.

Further clarification of effect caused by multisource interference, e.g. when, say, 100 outdoor RLANs transmitters operate in buildings of one urban quarter, would result in a protection distances which would significantly exceed line-of-sight distance for RLAN with bandwidth of 20 MHz. The protection distance for RLAN with 160 MHz bandwidth is about 260 km. Based on the above the conclusions may be drawn that compatibility between RLANs and air-borne radiodetermination radars seems to be extremely difficult to provide without using additional mitigation techniques.

### 5.2.3	Sharing and compatibility studies for 5 350-5 470 MHz with RLAN mitigation techniques

[TBD]

## 5.3	Sharing and compatibility of Earth exploration satellite versus WAS/RLAN in the band 5 250-5 570 MHz

[ESA/EUMETSAT 5A/96, 97]

The sharing and compatibility studies between WAS/RLAN and EESS (active) systems concern  different type of sensors operating in different portions of the 5 250-5 570 MHz band:

- Synthetic aperture radars (SAR) in the 5 350-5 470 MHz band.

- Altimeters covering the whole 5 250-5 570 MHz band.

- Scatterometers with small bandwidths in the 5 250-5 350 MHz band or the 5 350-5 470 MHz band.

According to Resolution 239 (WRC-15), the issues to be addressed in relation to EESS (active) are twofold:

[Editor’s Note: Resolution 229 (Rev. WRC-12) states that administrations are requested to take appropriate measures that will result in the predominant number of stations in the mobile service being operated in an indoor environment. Furthermore, stations in the mobile service that are permitted to be used outdoors in 5 250-5 350 MHz shall comply with eirp elevation masks in Resolution 229, however, administrations may exercise some flexibility in adopting other mitigation techniques to achieve an equivalent level of protection to incumbent services.]

- In the 5 250-5 350 MHz band, to study the possibility of enabling expanded outdoor WAS/RLAN operations including possible associated conditions (see invites ITU-R c)). This implies studies with Altimeter and scatterometer sensors.

- In the 5 350-5 470 MHz band, to study whether additional mitigation techniques would provide coexistence between WAS/RLAN systems and EESS (active) (see invites ITU-R d)) This implies studies with Altimeter, SAR and scatterometer sensors.

### 5.3.1	Determination of the number of RLAN overlapping EESS bandwidths and bandwidth factors

As a first step to address sharing between RLAN and EESS (active) at 5 250-5 570 MHz, it is necessary to calculate the number of RLAN overlapping the EESS bandwidth and bandwidth factors based on the method proposed by the U.S. administration and widely agreed in JTG 4-5-6-7 (see Annex D of Annex 35 to Document 4-5-6-7/704).

For reference, section 5.3.1.1 presents the calculations undertook during previous study period for the EESS (active) SAR sensor with a 100 MHz bandwidth as used in corresponding studies.

Sections 5.3.1.2 and 5.3.1.3 address the EESS (active) altimeter and scatterometers sensors, respectively, and provide consistent calculations to take into account the different sensor bandwidth of 320 MHz and 2 MHz.

#### 5.3.1.1 	SAR sensor with 100 MHz bandwidth

The overlapping of the EESS (active) SAR bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions that were used for sharing studies with SAR sensor:

- Number of RLAN overlapping the EESS (bandwidth) = 11279

- Corresponding RLAN density per inhabitant = 0.0021

- Average bandwidth factor per overlapping RLAN = -1.94 dB

Similarly, for RLAN density options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions that were used for sharing studies with SAR sensor:

- Number of RLAN overlapping the EESS (bandwidth) = 21000 (D2-low) and 210000 (D2-high)

- Corresponding RLAN density per inhabitant = 0.004 (D2-low) and 0.04 (D2-high)

Summary for SAR sensor:

| RLAN density option | Nb of RLAN overlapping the 100 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 100 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 11279 | 0.0021 | -1.94 dB |
| D2-low | 21000 | 0.004 | -1.94 dB |
| D2-high | 210000 | 0.04 | -1.94 dB |

#### 5.3.1.2 	Altimeter sensor with 320 MHz bandwidth

The overlapping of the EESS (active) altimeter bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions for sharing studies:

- Number of RLAN overlapping the EESS (bandwidth) = 25297

- Corresponding RLAN density per inhabitant = 0.0048

- Average bandwidth factor per overlapping RLAN = -0.4 dB

Similarly, for RLAN density option options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions for sharing studies:

- Number of RLAN overlapping the EESS (bandwidth) = 47103 (D2-low) and 471021 (D2-high)

- Corresponding RLAN density per inhabitant = 0.00897 (D2-low) and 0.0897 (D2-high)

Summary for Altimeter sensor:

| RLAN density option | Nb of RLAN overlapping the 320 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 320 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 25297 | 0.0048 | -0.4 dB |
| D2-low | 47103 | 0.00897 | -0.4 dB |
| D2-high | 471021 | 0.0897 | -0.4 dB |

#### 5.3.1.3 	Scatterometer sensor with 2 MHz bandwidth

The overlapping of the EESS (active) scatterometer bandwidth on the RLAN channel plan is described below:

On this basis, for the so-called “Sim city” (with 5.25 M inhabitants), the calculation of the number of RLAN overlapping the EESS (active) bandwidth and the bandwidth factor is made as follows (for RLAN density option D1):

These calculation leads to the following basic assumptions for sharing studies:

–	Number of RLAN overlapping the EESS (bandwidth) = 5 786

–	Corresponding RLAN density per inhabitant = 0.0011

–	Average bandwidth factor per overlapping RLAN = -16.03 dB

Similarly, for RLAN density option options D2-low and D2-high, the calculation of the number of RLAN overlapping the EESS (active) bandwidth is made as follows (noting that the bandwidth factor remains similar):

These calculation leads to the following basic assumptions for sharing studies:

–	Number of RLAN overlapping the EESS (bandwidth) = 10 773 (D2-low) and 107 722 (D2-high).

–	Corresponding RLAN density per inhabitant = 0.0021 (D2-low) and 0.021 (D2-high).

Summary for SCA sensor:

| RLAN density option | Nb of RLAN overlapping the 2 MHz EESS bandwidth | Nb of RLAN per inhabitant (density) overlapping the 2 MHz EESS bandwidth | Average bandwidth factor |
| --- | --- | --- | --- |
| D1 | 5786 | 0.0011 | -16.03 dB |
| D2-low | 10773 | 0.0021 | -16.03 dB |
| D2-high | 107722 | 0.021 | -16.03 dB |

### 5.3.2	Sharing and compatibility studies with EESS (active) in the 5 250-5 350 MHz band

#### 5.3.2.1 	Altimeter case

Detailed sharing studies between WAS/RLAN and EESS (active) altimeters sensors are presented in Annex 3, based on the ESA Sentinel-3 SRAL sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies provide results for “indoor type” WAS/RLAN (limited to WiFi case) deployment and hence representative of the current situation in the 5 250-5 350 MHz band.

Similar studies addressing “outdoor type” WAS/RLAN deployment taking into account both WiFi and LAA/LTE cases are to be performed.

#### 5.3.2.2 	Scatterometer case

Detailed sharing studies between WAS/RLAN and EESS (active) scatterometers sensors are presented in Annex 4, based on the EUMETSAT EPS-SG SCAsensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies provide results for “indoor type” WAS/RLAN (limited to WiFi case) deployment and hence representative of the current situation in the 5 250-5 350 MHz band.

Similar studies addressing “outdoor type” WAS/RLAN deployment taking into account both WiFi and LAA/LTE cases are to be performed.

#### 5.3.2.3	Consideration of potential RLAN mitigation techniques

TBD

### 5.3.3	Sharing and compatibility studies with EESS (active) in the 5 350-5 470 MHz band

#### 5.3.3.1 	Status of sharing studies

Sharing and coexistence between RLAN 5 GHz and EESS (active) in the 5 350-5 470 MHz band were studied during the previous study period (in JTG 4-5-6-7) and led to the following conclusion (see CPM Report to the 2015 World Radiocommunication Conference (Document N° 3 of WRC-15)):

“Results of sharing studies show that with the RLAN parameters described above, sharing between RLAN and the EESS (active) systems in the 5 350-5 470 MHz frequency band would not be feasible. Sharing may only be feasible if additional RLAN mitigation measures are implemented.”

One should also highlight the fact that these findings are duly reproduced in Recognising a) of Resolution 239 (WRC-15).

These conclusions were developed after intensive studies on sharing and coexistence between RLAN 5 GHz and EESS (active) mainly with EESS (active) SAR systems. These conclusions are summarised in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] on “Sharing studies between RLAN and EESS (active) systems in the frequency range 5 350-5 470 MHz” (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)). Taking into account the whole ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) under all scenarios, these studies shows that RLAN deployment in the frequency band 5 350-5 470 MHz would create large interference in the CSAR sensor on board the Sentinel-1 satellite (up to 30.4 dB).

Additional studies have been performed using static and dynamic analysis with altimeter (Sentinel-3 Altimeter sensor (SRAL)) and scatteromter (EPS-SG sensor (SCA)) and are given in Annex 3 and 4, repsectively. These studies take into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors during previous study period.

These studies confirms that, under all scenarios, RLAN deployment in the 5 GHz range would create large interference to altimeters (up to 26.6 dB) and scatterometers (up to 20.9 dB), hence in the same order of magnitude than for SAR sensors.

All together, these studies confirm the findings of JTG 4-5-6-7 and CPM-15-2 that sharing between RLAN and EESS (active) in the 5 350-5 470 MHz band may only be feasible if additional RLAN mitigation measures are implemented.

Consistently with Resolution 239 (WRC-15), the work in this frequency band should hence concentrate on studying whether any additional mitigation techniques beyond those analysed in the previous studies would provide coexistence between WAS/RLAN systems and EESS (active) in the in the 5 350-5 470 MHz band.

#### 5.3.3.2	Consideration of potential RLAN mitigation techniques

TBD

## 5.4	Sharing and compatibility of Radiolocation withWAS/RLAN in the 5 250-5 470/5 725-5 850 MHz

### 5.4.1	Sharing and compatibility studies for 5 250-5 350 MHz

[Editor’s Note: Text below need to be reviewed after propagation models used in the study confirmed by SG3]

[Editor’s Note:5B]

[RUS 5A/398]

To assess the impact of outdoor WAS/RLAN systems operation in the frequency band 
5 250-5 350 MHz the required separation distances are defined for protection of radars from indoor and outdoor WAS/RLAN systems emissions and the obtained results are compared.

To estimate compatibility with air-borne radars RLAN transmitter effective e.i.r.p. was defined by the following equation:

(1)

EIRP loss due to propagation in the walls was estimated using the following formula:

, dBW;	(1a)

where	 - additional attenuation, dB.

Then the receiver thermal noise level was estimated for each of the Radars considered using the following equations:

° К,		(2)

dBW,		(3)

where 		k – Boltzmann constant ,

NF – radar receiver noise figure;

- radar receiver IF operational pass-band.

Maximum permissible noise power at the receiver front end was assessed such as:

, dBW.		(4)

A free space propagation model was used to estimate interference to air-borne radars. In that case a separation distance R required for radiodetermination radar protection was estimated in the following way:

,	(5)

where 	 - radar antenna gain, dB;

λ – operational bandwidth, m.

Estimation of interference to ground-based radars used a propagation model, presented in Recommendation ITU-R Р.452.

When considering a case of multi-source interference from several RLAN transmitters the aggregate interference level at the ground-based radar receiver front end was calculated as:

,	(6)

where:	 - aggregate interference level at the radar receiver front end, dBW;

- interference level produced by the i-th RLAN transmitter at the radar receiver front end;

N – number of interference sources under consideration.

#### 5.4.1.1 	Assessment of outdoor WAS/RLAN systems operation consequences for air-borne radiodetermination radars in the frequency band 5 250-5 350 MHz

Interference from RLAN transmitters to operation of air-borne radiolocation stations was estimated using the scenario presented in Figure 1 addressing air-borne radars of type 8 and 9 (Recommendation ITU-R М.1638-1).

FIGuRE 1

Interference scenario for air-borne receivers

The acceptable interference power levels at the receiver front end were calculated for those radars using equations (2) – (4) above. The calculated values are shown in Table 1. They were used for determination of the required protection distances ensuring interference free operation of the Radars in case of indoor and outdoor deployment of single RLAN systems.

The estimations assumed an aircraft flying at 10 km altitude (Н=10 000 m). Interference to operation of the air-borne radar was caused by RLAN transmitters deployed indoor and outdoor at the height of 14 m, 20 m and 26 m. Estimation of interference from RLAN transmitters used a free space propagation model, described in Recommendation ITU-R Р.525. To take propagation loss in the walls into account in equation (4) additional propagation loss, σ, equal to 20 dB was assumed. Multi-source interference was taken into account using equation (6).

Table 5 presents calculation results for separation distance required for protecting the air-borne radars from single indoor and outdoor RLAN transmitters.

table 5

Separation distances (km) required for protecting the air-bore radars from indoor 
and outdoor operating RLANs

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 8 | 14 | 11 | 145 | 109 |
| Radar 9 | 200 | 71 | >RLOS | >RLOS |

* RLOS – line-of-sight distance of 420 km for a typical flight altitude of 12 000 m without refraction consideration

Analysis of the results presented in Table 5 shows that in case of using the outdoor WAS/RLAN systems the required protection distance would increase significantly compared with that for indoor WAS/RLAN systems. Such usage of even single outdoor WAS/RLAN systems could result in the required protection distances exceeding LOS distance for radars of certain types.

Besides, the required protection distances were estimated for a case of three indoor RLAN transmitters simultaneously operating in one building. The results are shown in Table 6.

table 6

Separation distances (km) required for protecting air-borne radars from three indoor and outdoor RLAN

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 8 | 25 | 19 | 249 | 187 |
| Radar 9 | 344 | 122 | >RLOS | >RLOS |

Analysis of the results presented in Table 6 shows that the protection distances required for protecting air-borne radars could increase significantly in case of minor (as compared with the above case) increase in number of outdoor WAS/RLAN systems.

The results so obtained allow drawing the conclusions that it would be extremely difficult to provide compatibility of outdoor WAS/RLAN systems with air-borne radars without employing additional interference mitigation techniques.

#### 5.4.1.2 	Assessment of consequences of outdoor WAS/RLAN systems operation in the frequency band 5 250-5 350 MHz to ground-based radiodetermination radars

Protection distances for the ground-based radiodetermination radars operating in the frequency bands considered were estimated on the basis of assumed interference scenario depicted in Figure 2.

figure 2

Interference scenario for ground-based radar receivers

Interference was estimated using a free space propagation model described in Recommendation ITU-R Р.452-16. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (1a). Attenuation in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (6).

Table 7 presents minimum estimated separation distances required for protection of ground-based radiodetermination radars from a single-source interference caused by indoor RLAN transmitters. In addition the calculations were conducted assuming a case of multi-source interference caused by three simultaneously operating indoor RLAN transmitters deployed in the same building.

TABLE 7

Separation distances (km) required for protecting ground-based radiolocation radars from indoor RLAN

| RLAN spectrum width | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter altitude, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 27 | 30 | 33 | 34 | 21 | 24 | 26 | 28 |
| Radar 6 | 22 | 24 | 26 | 28 | 10 | 10 | 10 | 17 |
| Radar 11 | <5 | <5 | <5 | 6 | <5 | <5 | <5 | <5 |
| Radar 19 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 21 | 35 | 38 | 41 | 43 | 28 | 32 | 34 | 36 |
| Radar 23 | 15 | 15 | 15 | 21 | 5 | 5 | 5 | 8 |

Analysis of estimation results reflected in Table 7 shows that the required protection distance in case of deploying indoor WAS/RLAN systems will not exceed 43 km. This value of protection distance is required only for two types of the radars addressed in case of the aggregate interference from RLAN systems using a signal of 20 MHz in width.

Table 8 presents minimum estimated separation distances required for protection of ground-based radiodetermination radars from a single-source and aggregate interference caused by outdoor RLAN transmitters. Estimations were conducted using equation (2) for every mode of RLAN operation. The estimation assumed the height of RLAN transmitters to be 14 m, 20 m and 26 m.

TABLE 8

Separation distances (km) required for protecting ground-based radiolocation and radionavigation radars 
from outdoor deployed RLANs

| RLAN spectrum width | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter altitude, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 43 | 46 | 49 | 51 | 35 | 38 | 41 | 43 |
| Radar 6 | 35 | 39 | 42 | 44 | 28 | 32 | 34 | 36 |
| Radar 10 | 47 | 50 | 53 | 55 | 38 | 41 | 44 | 46 |
| Radar 11 | 23 | 26 | 28 | 29 | 12 | 14 | 14 | 21 |
| Radar 19 | 54 | 57 | 59 | 62 | 45 | 48 | 51 | 53 |
| Radar 21 | 54 | 57 | 60 | 63 | 45 | 48 | 51 | 53 |
| Radar 23 | 31 | 34 | 37 | 38 | 24 | 28 | 31 | 38 |

Analysis of the estimation results described in Table 8 shows that assuming outdoor RLAN transmitters the separation distances required for protecting relevant radars would be of several dozen km even for RLANs using data transfer channel of 160 MHz. For example, for radar 21 and a RLAN transmitter using a data channel of 20 MHz passband and deployed at a height of 26 m the estimated distance was 60 km. In case of aggregate interference caused from three RLANs the protection distance would increase to 63 km. The obtained results would also provide for drawing a conclusion that compatibility of outdoor WAS/RLAN systems with ground-based radars would be extremely difficult to provide without implementation of additional interference mitigation techniques.

#### 5.4.1.3 	Assessment of consequences of outdoor WAS/RLAN systems operation in the frequency band 5 250-5 350 MHz to ground-based meteorological radars

Protection distances for the ground-based meteorological radars operating in the frequency bands considered were also estimated in line with the interference scenario depicted in Figure 2. The estimation used a free space propagation model described in Recommendation ITU-R Р.452-16. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (1a). Attenuation in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (6).

Table 9 presents minimum estimated separation distances required for protection of ground-based meteorological radars from a single-source interference caused by indoor RLAN transmitters. Table 9 also reflects results of estimation assuming multi-source interference caused by three simultaneously operating indoor RLAN transmitters deployed in the same building.

TABLE 9

Separation distances (km) required for protecting ground-based meteorological radars 
from indoor deployed RLANS

| RLAN spectrum width | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter altitude, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 29 | 33 | 36 | 37 | 24 | 27 | 29 | 31 |
| Radar 4 | 34 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 8 | 39 | 42 | 45 | 48 | 32 | 35 | 37 | 39 |
| Radar 11 | 44 | 47 | 49 | 52 | 36 | 39 | 42 | 44 |
| Radar 12 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 13 | 47 | 50 | 53 | 56 | 39 | 42 | 44 | 46 |

Analysis of the estimation results described in Table 9 shows that providing protection for major number of radars would require separation distances of several dozen kilometers.

Table 10 reflects minimum separation distances required for protecting ground-based radiodetermination radars from single-source interference caused by outdoor RLAN transmitters. Estimations referred to the addressed modes of RLAN operation were performed using expression (2). The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Estimations also addressed a case associated with multi-source interference caused be three simultaneously operating indoor RLAN transmitters deployed in the same building at the height of 14 m, 20 m and 26 m.

TABLE 10

Separation distances (km) required for protecting ground-based meteorological radars 
from outdoor deployed RLANS

| RLAN spectrum width | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter altitude, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 46 | 49 | 51 | 54 | 38 | 41 | 44 | 49 |
| Radar 4 | 54 | 56 | 59 | 62 | 44 | 47 | 50 | 52 |
| Radar 8 | 60 | 62 | 65 | 68 | 50 | 53 | 55 | 59 |
| Radar 11 | 66 | 69 | 71 | 76 | 55 | 58 | 60 | 64 |
| Radar 12 | 63 | 66 | 69 | 72 | 53 | 55 | 58 | 61 |
| Radar 13 | 75 | 77 | 79 | 83 | 62 | 65 | 68 | 71 |

Analysis of the estimation results described in Table 10 shows that absence of additional loss associated with emission propagation in walls would result in significant increase of separation distances required for protecting meteorological radars as compared with the case of indoor WAS/RLAN systems. The obtained results also allow to conclude that it would be extremely difficult to provide compatibility of outdoor WAS/RLAN systems with ground-based radars without additional usage of interference mitigation techniques.

### 5.4.2	Sharing and compatibility studies for 5 350-5 470 MHz without RLAN mitigation techniques

[RUS 5A/196]

#### 5.4.2.1 	Compatibility of WAS/RLAN with airborne radars of the radiolocation service in the frequency band 5 350-5 470 MHz

Analysis of data given in Table 4.3 shows that airborne Radar 9 and Radar 17 operate in the frequency band 5 350-5 470 MHz. Interference impact assessment from RLAN to these radars was performed subject to the interference scenario given in section 5.2 hereof. Table 5.3 contains the calculation results of the required protection distance for airborne Radars from single outdoor and indoor RLAN transmitters.

TABLE 5.3

Separation distances (km) required for protecting air-borne radars 
from indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 9 | >RLOS | >RLOS | 200 | 71 |
| Radar 17 | >RLOS | 342 | 97 | 34 |

Analysis of the results reflected in Table 5.3 shows that even in case of a single-source interference caused by outdoor RLAN transmitter the required protection distance could far exceed line-of-sight distance between an air-borne radar receiver and a RLAN transmitter. In case of deployment of a RLAN indoor transmitter the required protection distance could range from 34 km to 200 km depending on the RLAN bandwidth.

Estimation of the required protection distances was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building. The estimation results are shown in Table 5.4.

TABLE 5.4

Separation distances (km) required for protecting air-borne radars
 from three indoor and outdoor deployed RLANs

|  | EIRPeff=-7 dBW, σ=0 dB |  | EIRPeff=-7 dBW, σ=20 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Radar 9 | >RLOS | >RLOS | 344 | 122 |
| Radar 17 | >RLOS | >RLOS | 166 | 59 |

Analysis of estimation results described in Table 5.4 shows that a partial consideration of multisource interference caused by RLANs transmitters would result in significant increase of protection distances required for operation of air-borne radar without interference.

Further clarification of effect caused by multisource interference, e.g. when, say, 100 outdoor RLANs transmitters operate in buildings of one urban quarter, would result in a protection distance which would significantly exceed line-of-sight distance for airborne radars at the altitude of 10 km.

Consideration of interference caused by deployed in one urban quarter indoor RLANs transmitters with 20 MHz signal bandwidth shows that the required protection distance would exceed the line-of-sight distance for radars 9 and 17. Consideration of RLANs transmitters with 160 MHz signal bandwidth would result in the required protection distance exceeding the line-of-sight distance for radar 9. The protection distance would be 342 km for Radar 17. Based on the above the conclusions may be drawn that compatibility between RLANs and air-borne radars seems to be extremely difficult to provide without using additional mitigation techniques.

#### 5.4.2.2 	Compatibility of WAS/RLAN with ground radars of the radiolocation service in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

Compatibility evaluation of WAS/RLAN with ground-based radars operating in the considered frequency bands was performed in line with the interference scenario given in section 5.2 hereof.

FIGURE 5.2

Interference scenario for RLS ground-based radar receiver

Interference was estimated using a propagation model described in Recommendation ITU-R Р.452. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Propagation loss in walls was considered using the following equation:

, dBW	(6)

where:

–	additional fading, dB.

Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.5 and Table 5.6 presents minimum estimated separation distances required for protection of ground-based radiodetermination radars from a single-source interference caused by outdoor RLAN transmitters in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz accordingly. Estimations were conducted for two operation modes of RLAN with bandwidth of 20 MHz and 160 MHz. Estimation was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building at the same heights of RLAN.

TABLE 5.5

Separation distances (km) required for protecting ground-based radiolocation radars from outdoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 63 | 65 | 68 | 71 | 48 | 54 | 57 | 60 |
| Radar 3 | 54 | 57 | 59 | 62 | 44 | 47 | 50 | 53 |
| Radar 4 | 44 | 48 | 50 | 52 | 36 | 39 | 42 | 44 |
| Radar 5 | 49 | 51 | 54 | 58 | 40 | 43 | 46 | 48 |
| Radar 7 | 32 | 34 | 38 | 39 | 25 | 28 | 32 | 33 |
| Radar 12 | 34 | 38 | 40 | 42 | 27 | 31 | 33 | 35 |
| Radar 13 | 54 | 56 | 58 | 61 | 44 | 47 | 50 | 52 |
| Radar 15 | 54 | 57 | 59 | 62 | 45 | 48 | 50 | 53 |
| Radar 19 | 54 | 57 | 59 | 62 | 45 | 48 | 51 | 53 |
| Radar 20 | 52 | 54 | 57 | 60 | 43 | 46 | 48 | 51 |
| Radar 21 | 54 | 57 | 60 | 63 | 45 | 48 | 51 | 53 |
| Radar 22 | 42 | 44 | 47 | 49 | 33 | 37 | 40 | 42 |
| Radar 23 | 31 | 34 | 37 | 38 | 24 | 28 | 31 | 38 |

TABLE 5.6

Separation distances (km) required for protecting ground-based radiolocation radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 72 | 75 | 77 | 81 | 60 | 63 | 65 | 69 |
| Radar 4 | 52 | 55 | 57 | 60 | 43 | 46 | 48 | 51 |
| Radar 5 | 56 | 59 | 62 | 65 | 47 | 50 | 52 | 55 |
| Radar 7 | 38 | 41 | 43 | 45 | 30 | 34 | 37 | 38 |
| Radar 12 | 40 | 43 | 46 | 49 | 33 | 36 | 38 | 41 |
| Radar 13 | 62 | 65 | 67 | 71 | 51 | 54 | 57 | 60 |
| Radar 15 | 62 | 65 | 68 | 72 | 52 | 55 | 58 | 60 |
| Radar 22 | 49 | 52 | 54 | 57 | 40 | 43 | 46 | 48 |
| Radar 23 | 36 | 39 | 42 | 44 | 29 | 33 | 35 | 37 |

Analysis of the estimation results described in Tables 5.5 and 5.6 shows that the separation distances required for protecting ground radars would be of several dozen km even for RLANs using data transfer channel of 160 MHz. For example, in the frequency band 5 350-5 470 MHz for Radar 2 suffering interference from RLAN transmitter using a data channel of 160 MHz bandwidth and deployed at a height of 26 m the estimated protection distance is 57 km. For the same Radar operating in the frequency band 5 725-5 850 MHz the protection distance is 65 km. Based on the above the conclusions may be drawn that enlarging RLANs bandwidth to reduce spectral density of interference caused for radars may not be considered as one of the interference mitigation techniques in relation to ground-based radiodetermination radars.

Minimum separation distances required for protection of ground radars from a single-source interference caused by indoor RLAN transmitters in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz are presented in Tables 5.7 and 5.8. Estimations were conducted for two operation modes of RLAN. Estimation was also conducted assuming multisource interference caused by three simultaneously operating indoor RLAN transmitters deployed in a single building at height of 14 m, 20 m. and 26 m.

TABLE 5.7

Separation distances (km) required for protecting ground-based radiolocation radars from indoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 3 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 4 | 28 | 31 | 34 | 36 | 22 | 26 | 26 | 29 |
| Radar 5 | 31 | 34 | 37 | 39 | 24 | 28 | 31 | 32 |
| Radar 7 | 17 | 18 | 18 | 24 | 6 | 6 | 6 | 10 |
| Radar 12 | 21 | 23 | 24 | 27 | 8 | 8 | 8 | 15 |
| Radar 13 | 34 | 37 | 40 | 41 | 28 | 31 | 33 | 35 |
| Radar 15 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 19 | 35 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 20 | 33 | 37 | 39 | 41 | 27 | 30 | 32 | 34 |
| Radar 21 | 35 | 38 | 41 | 43 | 28 | 32 | 34 | 36 |
| Radar 22 | 26 | 29 | 32 | 33 | 21 | 23 | 24 | 26 |
| Radar 23 | 15 | 15 | 15 | 21 | 5 | 5 | 5 | 8 |

TABLE 5.8

Separation distances (km) required for protecting ground-based radiolocation radars from indoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 2 | 48 | 51 | 53 | 56 | 39 | 42 | 45 | 47 |
| Radar 4 | 33 | 36 | 39 | 41 | 27 | 30 | 33 | 34 |
| Radar 5 | 36 | 40 | 43 | 44 | 29 | 33 | 36 | 37 |
| Radar 7 | 23 | 26 | 28 | 30 | 14 | 14 | 14 | 21 |
| Radar 12 | 25 | 28 | 31 | 32 | 19 | 20 | 20 | 26 |
| Radar 13 | 40 | 43 | 46 | 48 | 33 | 36 | 39 | 41 |
| Radar 15 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 22 | 31 | 34 | 37 | 39 | 24 | 28 | 31 | 32 |
| Radar 23 | 23 | 25 | 27 | 29 | 11 | 11 | 11 | 20 |

Analysis of the estimation results described in Tables 5.7 and 5.8 shows that in spite of reducing the level of interference due to fading in the walls the protection distances of several tens km are required for protection of the radar receivers. The results shown in these Tables were gained for the walls with propagation loss of 20 dB. However it is to note that the level of signal fading in walls is overestimated for a significant number of buildings, office ones specifically. Therefore, the required separation distances would exceed those shown in Tables 5.7 and 5.8.

Thus RLAN transmitter signal fading when propagating in walls of buildings may not be considered as an effective interference mitigation technique as well as that of enlarging the channel bandwidth.

#### 5.4.2.3	Compatibility of WAS/RLAN with ground meteorological radars in the frequency bands 5 350-5 470 MHz and 5 725-5 850 MHz

Protection distances for the ground-based meteorological radars operating in the frequency bands considered were also estimated in line with the interference scenario depicted in Figure 5.2. Interference was estimated using a free space propagation model described in Recommendation ITU-R Р.452. The assumed height of RLANs transmitters was 14 m, 20 m and 26 m. Estimation was also conducted assuming multisource interference caused by three simultaneously operating RLAN transmitters deployed in a single building at height of 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (6). Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.9 and Table 5.10 present minimum estimated separation distances required for protection of ground-based meteorological radars from interference caused by outdoor RLAN transmitters. Estimations were conducted for two modes of RLAN operation.

TABLE 5.9

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 46 | 49 | 51 | 54 | 38 | 41 | 44 | 49 |
| Radar 4 | 54 | 56 | 59 | 62 | 44 | 47 | 50 | 52 |
| Radar 8 | 60 | 62 | 65 | 68 | 50 | 53 | 55 | 59 |
| Radar 12 | 63 | 66 | 69 | 72 | 53 | 55 | 58 | 61 |
| Radar 13 | 75 | 77 | 79 | 83 | 62 | 65 | 68 | 71 |
| Radar 14 | 64 | 66 | 68 | 70 | 53 | 55 | 58 | 61 |

TABLE 5.10

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 8 | 70 | 72 | 74 | 80 | 58 | 60 | 62 | 66 |

Analysis of the estimation results described in Tables 5.9 and 5.10 shows that strengthening the protection criteria for the radars would result in increasing the separation distances ensuring protection of ground-based meteorological radars. For example, in the Radar frequency band 5 350-5 470 MHz the required protection distance would be up to 79 km for Radar 13 assuming a single RLAN transmitter using a data channel of 20 MHz bandwidth deployed at height of 26 m. In case of a RLAN transmitter using a 160 MHz channel bandwidth the separation distance would reduce to 68 km and still would be of several tens km even for the rest considered Radar types. Based on the above the conclusions may be drawn that enlarging RLANs bandwidth to reduce spectral density of interference caused for radars may not be considered as one of the interference mitigation techniques in relation to operation of meteorological radars.

Table 5.11 and Table 5.12 present minimum estimated separation distances required for protection of meteorological radars from interference caused by indoor RLAN transmitters.

TABLE 5.11

Separation distances (km) required for protecting ground-based meteorological radars from indoor deployed RLANs in the frequency band 5 350-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 1 | 29 | 33 | 36 | 37 | 24 | 27 | 29 | 31 |
| Radar 4 | 34 | 38 | 41 | 43 | 28 | 31 | 34 | 36 |
| Radar 8 | 39 | 42 | 45 | 48 | 32 | 35 | 37 | 39 |
| Radar 12 | 41 | 44 | 47 | 49 | 33 | 36 | 39 | 41 |
| Radar 13 | 47 | 50 | 53 | 56 | 39 | 42 | 44 | 46 |
| Radar 14 | 44 | 47 | 49 | 51 | 36 | 38 | 41 | 43 |

TABLE 5.12

Separation distances (km) required for protecting ground-based meteorological radars from outdoor deployed RLANs in the frequency band 5 725-5 850 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 8 | 45 | 49 | 51 | 54 | 38 | 41 | 43 | 45 |

Analysis of the estimation results described in Tables 5.11 and 5.12 shows that in spite of reducing the level of interference to operation of the ground-based meteorological radar receivers the majority of them would require protection distances of several tens km. The results shown in Tables 5.11 and 5.12 were gained for the walls with propagation loss of 20 dB. However it is to note that the level of signal fading in walls is overestimated for a significant number of buildings, office ones specifically. Therefore, the required separation distances would exceed those shown in Tables 5.11 and 5.12.

Thus RLAN transmitter signal fading when propagating in walls of buildings may not be considered as effective interference mitigation technique as well as that of enlarging the channel bandwidth.

### 5.4.3	Sharing and compatibility studies with RLAN mitigation techniques

[SUI 5A/57]

For sharing between RLAN and meteorological radar the application of a mitigation technique, like DFS is required. Table 19 provides a summary of the DFS requirements for the frequency band 5 350-5 470 MHz. These parameters proved to allow WAS to avoid interfering with the radio determination service in the band 5 600-5 650 MHz.

Table 19

| Parameter | Values for the frequency band 5 350-5 470 MHz |
| --- | --- |
| Minimum pulse width (see detailed test signals in Report ITU-R M.2115) | 0.5 μs |
| PRF (see detailed test signals in Report ITU-R M.2115) | Fixed, Staggered and Interleaved |
| Channel Availability Check (CAC) time | 10 minutes |
| Off-Channel CAC (Note 1) | Yes |
| CAC and Off-Channel CAC detection probability (Note 2) | 99.99% |
| In-service monitoring detection probability | 60% |
| CAC for slave devices with power above 200 mW (after initial detection by In-service) | Yes |
| Detection Threshold | -62 +10 -EIRP Spectral Density (dBm/MHz) + G (dBi), however the DFS threshold level shall not be lower than -64 dBm assuming a 0 dBi receive antenna gain |
| Channel Move time | 10s |
| Channel closing time | 1s |
| Non-occupancy period | 30 minutes |
| Possibility to exclude 5 600-5 650 MHz band from the channel plan or to exclude these channels from the list of usable channels | Yes |
| Requirement that none of the DFS related settings are accessible to the end-user | Yes |

## 5.5	Sharing and compatibility of Radionavigation versus WAS/RLAN in the 5 460-5 470 MHz

### 5.5.1	Sharing and compatibility studies without RLAN mitigation techniques

[RUS 5A/196]

In accordance with Recommendation ITU-R 1638-1 two types of ground-based navigation radars: Radar 10 and Radar 10A operate in this frequency band. The technical characteristics and protection criteria are given in Table 4.2. For the specified Radars the required protection distances were estimated in line with interference scenario shown in Figure 5.2. With this a propagation model, presented in Recommendation ITU-R Р.452 was used for estimation. The estimation assumed the height of RLAN transmitters to be 14 m, 20m and 26 m. Estimation was also conducted assuming multisource interference caused by three simultaneously operating RLAN transmitters deployed in a single building at height of 14 m, 20 m and 26 m. Propagation loss in walls were considered using expression (6). Fading in walls was assumed as 20 dB. The assumed radar antenna height above the ground level was 20 m. Multi-source interference was taken into account using equation (5).

Table 5.13 presents minimum estimated separation distances required for protection of ground-based radionavigation radars from a single-source interference caused by outdoor RLAN transmitters. Table 5.13 also contains estimation results for a case of multiple interference caused by three simultaneously operating outdoor RLAN transmitters deployed out of a single building. Estimations were conducted for two modes of RLAN operation.

TABLE 5.13

Separation distances (km) required for protecting ground-based radionavigation radars 
from outdoor deployed RLANs in the frequency band 5 460-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 10 | 47 | 50 | 53 | 55 | 38 | 41 | 44 | 46 |
| Radar 10а | 47 | 50 | 53 | 55 | 38 | 41 | 44 | 46 |

Analysis of the estimation results shows that the required protection distances for radionavigation Radars from RLAN with bandwidth of 20 MHz would be of several dozen km. In case of a RLAN transmitter using a 160 MHz channel bandwidth the separation distance would be reduced and still the reduction will not exceed 20% from the protection distance for RLANs using data transfer channel of 20 MHz. Table 5.14 presents estimated separation distances required for protection of radionavigation radars from interference caused by indoor RLAN transmitters.

TABLE 5.14

Separation distances (km) required for protecting ground-based radionavigation radars 
from indoor deployed RLANs in the frequency band 5 460-5 470 MHz

| RLAN bandwidth | 20 MHz |  |  |  | 160 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLAN transmitter height, m | 14 | 20 | 26 | Σ | 14 | 20 | 26 | Σ |
| Radar 10 | 30 | 33 | 36 | 37 | 23 | 27 | 30 | 31 |
| Radar 10а | 30 | 33 | 36 | 37 | 23 | 27 | 30 | 31 |

Analysis of the estimation results shows that in spite of significant reduction of the  interference level caused by RLAN transmitters the protection distances of several tens km are required for protection of the ground-based radionavigation radar receivers.

### 5.5.2	Sharing and compatibility studies with RLAN mitigation techniques

[TBD]

## 5.6	Sharing and compatibility of Fixed Service versus WAS/RLAN in the 5 850-5 925 MHz

## [TBD]

## 5.7	Sharing and compatibility of Fixed Satellite Service versus WAS/RLAN in the 5 725-5 850 MHz (for Region 1) and 5 850-5 925 MHz

### 5.7.1	Sharing and Compatibility of FSS in Region 1 in 5 725-5 850 MHz band

### 5.7.1.1	Study 1

[LUX 5A/442]

CEPT Report 57, published in March 2015, covered a significant amount of work that had been carried out by CEPT.

The general conclusion from CEPT Report 57 was that it was not possible at that time to specify any appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the bands 5 350-5 470 MHz and 5 725-5 925 MHz while ensuring relevant protection of incumbent services in these bands. CEPT Report 57 also concluded that these studies (in particular on additional mitigation techniques) that have not been completed and that any further work undertaken by CEPT could be taken into account when reviewing the results of the WRC-15 under Task (3) of the mandate.

This Report reviews and/or reconfirms the compatibility and sharing conditions developed previously and take account of the results of WRC-15.

Since the publication of CEPT Report 57, CEPT has carried out some additional work on compatibility studies related to RLANs in the 5 725-5 925 MHz band; additional studies on further mitigation techniques are still being investigated both within ETSI and CEPT.

Taking account of the studies shown in CEPT Report 57 and ECC Report 244, a summary of the current status of the various sharing and compatibility studies addressing Sharing and compatibility of Fixed Satellite Service versus WAS/RLAN in the 5 725-5 850 MHz (for Region 1) and 
5 850 5 925 MHz is presented hereafter:

Further studies since the publication of CEPT Report 57 have focused on the assessment of the interference from RLAN into FSS using a two-step approach:

Step 1 calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receiver under consideration (considering the satellite footprint) whilst satisfying the FSS protection criteria.

Step 2 delivers the number of active, on-tune, RLAN transmitters using a deployment model. The step 2 outputs can be compared with the step 1 values in order to assess the potential for sharing. In theory, if the step 2 values are less than or equal to the step 1 values, then the results suggest that sharing is possible; else if the step 2 values are greater than the step 1 values, sharing is not possible.

As there were a number of options and associated results studied for both steps 1 and 2, it was agreed to perform sensitivity analyses, taking into account ranges of values for some of these factors. Initial calculations and results were presented in ECC Report 244 but although providing some relevant results, it was too early to draw definite conclusions.

Additional studies on the potential for RLAN-FSS sharing were developed.

The wide range of results available reflects the wide range of inputs to the models considered in the studies. Further work would be required on the modelling including on the range of inputs.

As a result, it has not been possible to arrive at a consensus regarding suitable inputs for the modelling, and further studies would be required. Further mitigation techniques may also need to be investigated and studied for their impact on RLAN operations and results of studies. One possible way forward to address some of the uncertainties currently seen in the range of results is to carry out some airborne measurements to compare actual RLAN use with the predicted results from the model for defined geographical areas. An example of how to compare real measurements with the results of the model has been presented during the course of the studies.

Work is still required on the specification of appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the bands while ensuring relevant protection of the Fixed Satellite Services in these bands.

There has been no study on the potential interference from FSS earth stations into RLAN.

Consequently, numerous studies have been conducted by interested Administrations (for example, within CEPT SE 24 and WG FM) to estimate the levels of interference which FSS space receivers in Region 1 could absorb from RLANs without generating a harmful level of interference into FSS space receivers. The general conclusions reached indicate that sharing of WAS/RLAN and FSS space receivers would be very difficult and additional techniques should be developed to mitigate the risk of harmful interference created by the aggregate interfering signals originated by transmitting WAS/RLAN stations. Indeed, the agenda item 1.16 called for studies on suitable mitigation techniques (resolves b). [Assumptions on the RLAN characteristics are based on Annex 24 of the WP 5 A Chairman´s Report from May 2016 meeting.]

The following table provides a summary of the conclusions of the studies for three selected representative scenarios (out of the 27 scenarios considered).

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz | Band 5 850-5 925 MHz |
| --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) | FSS protection criteria satisfied for all FSS groups 1, 2, 3, 4 and 5 (margin ranges 0.1 to 10.3 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 2.6 to 9.9 dB). FSS protection criteria exceeded for other FSS group 4 (exceeding of 0.9 dB) | FSS protection criteria satisfied for FSS groups 1 2 and 3 (margin ranges 0.6 to 8.1 dB). FSS protection criteria exceeded for other FSS groups 4 and 5 (exceeding ranges 0.2 to 2.1 dB) |
|  | 0 | 17 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 8 dB): FSS protection criteria exceeded for other FSS group 4 (exceeding of 2.7 dB) | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 6.3 dB). FSS protection criteria exceeded for other FSS groups 3, 4 and 5 (exceeding ranges 0 to 3.9 dB) |
|  | 0 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 5.2 to 5.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 1.4 to 4.9 dB) | FSS protection criteria satisfied for FSS groups 1 (margin ranges 0 to 4.1 dB) and Satellite N (28.5 dBi) with a margin of 0 dB. FSS protection criteria exceeded for other FSS groups 2 (except satellite N), 3, 4 and 5 (exceeding ranges 0.2 to 6 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2.2 to 2.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4.4 to 7.9 dB) | FSS protection criteria satisfied for FSS group 1 (margin ranges 0.5 to 1.1 dB). FSS protection criteria exceeded for other FSS groups 2, 3, 4 and 5 (exceeding ranges 3 to 9.1 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 0 to 0.7 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 6.5 to 10.1 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 1.1 to 11.2 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 1.1 to 11.9 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 2.9 to 13.1 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 3.3 to 14.1 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 5.1 to 15.2 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 4.5 to 15.3 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 6.3 to 16.5 dB) |
|  | 4 | 12 | FSS protection criteria exceeded for all FSS groups 2, 3, 4 and 5 (exceeding ranges 6.7 to 17.4 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 8.4 to 18.6 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 3, 4 and 5 (exceeding ranges 8.5 to 19.2 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 10.3 to 20.4 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 10.7 to 21.4 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 12.4 to 22.6 dB) |

The above consideration takes account that all scenarios considered so far by the ECC Report 244 are valid and realistic. Indeed, some previous considerations on the conservativeness of assumptions have not been endorsed by the recent studies of WP 3K/WP 3M and, in particular, these WPs (22-29 March 2017) concluded that the analysis of the clutter loss and building entry loss models provided in the ITU Draft New Recommendations  show that:

–	Working Party 5A should not use the clutter component of P.452

–	The clutter loss model of DNR P. [Clutter], currently applicable from 10 GHz to 
100 GHz, could be extended to the 5 GHz range

–	This clutter model would provide lower values for clutter losses at 5 GHz than those currently assumed in ECC Report 244

–	For the building entry loss model, applicable from 80 MHz to 100 GHz, the average values obtained for the building entry loss at 30 degrees elevation angle is 14 dB at 
5.8 GHz and 13.4 dB at 2.4 GHz, which is only a 0.6 dB difference. Some airborne measurements submitted previously to WP 5 A showed a difference of 6.1 dB (8.4 dB at 2.4 GHz and 14.5 dB at 5 GHz)

Building entry losses. It was concluded that indoor/outdoo attenuation (currently estimated as 12 and 17 dB) should be considered as very similar at 2.4 GHz and 5 GHz and not a difference of 8,4 dB at 2,4 GHz and 14,5 dB at 5 GHz. Instead, the building losses are actually 13.4 dB at 2.4 GHz and 14.5 dB at 5 GHz, almost no significant difference.

#### 5.7.1.1.1	Estimation of interference from WAS/RLAN through empirical methods

As a potential way forward to estimate the interference in Region 1 to FSS from WAS/RLAN, an innovative contribution was made available to WP 5A (Doc. 5A/91) suggesting the estimation of interference through an empirical method consisting on airborne measurements. A separate report on measurements on interference created by RLAN using airborne is under elaboration.

A detailed analysis on the set of airborne measurements has been made. In general these airborne measurements are potentially interesting to:

- Achieve a better understanding of the study models before the actual RLAN deployment, with a view to determine the actual interference level observed and compare it with calculated results of studies.

- To measure and control, including on the long term, the aggregate interference into FSS space stations when RLAN deployment is developing.

Preliminary conclusions on the airborne measurements do not allow at this stage characterisation of the interference environment and do not allow quantitative conclusions to be drawn. Additional measurements campaigns from airplanes (including in other geographical areas) would be necessary before being able to draw any conclusions from such measurements.

Furthermore, the following issues should be investigated and clarified on possible future airborne measurements:

- Measurements are made locally at low altitude (few kms) whereas FSS space stations cover wide geographic areas and operate from the GSO orbit in space.

- It is not clear how measurements at 2.4 GHz (for which the reference point is not clear, is it less congested than expected?) can help characterise the interference environment at 5 GHz, especially since measurements at 5 GHz are close to the noise floor.

- Measurements should be made during busy hours.

- It is not clear how space and time dynamics of an airplane is comparable with the case of GSO satellites (which are seen from the Earth as static in the sky).

- These measurements have considered two elevation angles (30 and 90 degrees). It would seem useful to specifically conduct measurements at elevation and azimuths corresponding to the GSO orbit, since the GSO orbit as seen from the Earth is a particular path in the sky, from 0 degree elevation angle at horizon for the most western and eastern azimuths to about 48 degrees elevation angle at south azimuths, from London.

Noting the difficulties to characterize the overall aggregate interference from RLAN into space station receivers of the FSS in a deterministic approach, the empirical methods should ensure the replicability of the results obtained at one frequency range as applicable in another frequency range. Empirical methods are valid scientific approaches to measure realistic scenarios in a facts finding measurement approach, but it is rather scientifically questionable that an empirical approach is used in one frequency range (2.4 GHz) to derive deterministic measurements to be applied in another frequency range (5.8 GHz).The above consideration has been further recently endorsed by results of the WP 3K/3M making invalid some of the assumptions made regarding clutter/building entry losses.

#### 5.7.1.1.2	Potential mitigation techniques for WAS/RLAN

[LUX 5A/442]

As can be checked through the conclusions obtained by various studies conducted so far by CEPT and other Administrations, the only viable mechanism for reaching acceptable sharing conditions between FSS and RLAN in the 5.8 GHz range is through the development, and demonstration of their reliable implementation, of suitable mitigation techniques to be applied by RLAN systems, both in their technical characteristics and their deployment criteria. The reliability of those mitigation techniques (still to be identified) should take account of the typical deployment scenarios of RLAN services, usually under unlicensed conditions in some countries and, thus, bring additional requirements to enforce compliance against any restriction to the deployment or limitations on the transmitting parameters.

It can be noted that sufficient details on potential mitigation techniques are lacking relative to certain 5 GHz sub-bands, despite the fact that previous studies concluded on the need of such mitigation techniques. It is understood that further specific sharing studies WAS/RLAN and FSS will be conducted as part of the AI 1.16 work plan; presumably, these studies will conclude on the need of adequate mitigation techniques. See, for example, the conclusions reached by JTG at the previous study cycle for similar sharing studies (Example: Doc. 2357).

For protection of the FSS, two possible mitigation techniques can be identified:

  - Limitation of RLAN deployment to indoor only.

  - Placing a limit on the maximum eirp of RLAN APs. The appropriate value can be determined through additional studies for RLAN under WRC-19 agenda item 1.16 (noting a value of 10 to 15 dBm/20 MHz has been determined by the ITU in the context of studies between IMT and the FSS, with an interference configuration similar to the RLAN case, see Report ITU-R S.2367).

  - Elevation angle mask/downward tilt

These two mitigation techniques seem relatively simple to implement but are sometimes difficult to enforce for the RLAN type of equipment (e.g. indoor only, or non-compliant equipment). The impact of such mitigation techniques on the RLAN deployment and operations (e.g. RLAN e.i.r.p. distribution and the resulting limitations on data rates possible), should be studied.

Other RLAN mitigation techniques are required.

#### 5.7.1.1.3	Additional compatibility studies in adjacent bands

Compatibility studies between RLAN operating in any of the sub-bands within 5 725-5 925 MHz in Regions in which FSS is allocated and the FSS operating in adjacent sub-bands are necessary.

#### 5.7.1.1.4	Protection of FSS

The protection criterion for FSS is to accept an increase of the equivalent temperature greater than x% due to interference signals on the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.: ΔT/T≤ x%. This criterion is applicable for interference scenarios when the interfering signal is time invariant, which is assumed the case for the aggregate interference levels created by WAS/RLAN in this uplink satellite band, because coverage areas are large (hemispheric or global) and there is no mobility aspect due to the fact that main interference is from RLAN access points which are fixed.

The values of x is 6%, generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment, and x=1%, generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme should be considered where interference from RLANs is limited to half of the ΔT/T= 6% criterion i.e. the ΔT/T objective is reduced to a value of 3%. In addition, geographic apportionment could also be applied depending on the satellite’s coverage and the assumptions for the RLAN deployment.

### 5.7.1.2	Study 2

[UK 5A/246]

#### 5.7.1.2.1	Background and analysis

This analysis is looking results of some previous sharing studies carried out in ITU-R and CEPT. We use a methodology that makes reference too and analyses the 3 different studies carried out which are-

–	Report ITU-R S.2367 – Sharing and compatibility between International Mobile Telecommunication systems and fixed-satellite service networks in the 5 850-6 425 MHz frequency range.

–	ECC Report 244 – Compatibility studies related to RLANs in the 5 725-5 925 MHz band (see annex1).

–	Annex 25 of the last WP 5A Chairman’s Report – Use of aggregate RLAN measurements from airborne and terrestrial platforms to support studies under WRC-19 agenda item 1.16.

When analysing these three studies in order to arrive at a useful range of results we had to make a choice on which study should be used to form the baseline analysis. The UK decided to use the studies contained in ECC Report 244 as the baseline study. Once choosing the baseline study to work from we took a 3 step approach to the analysis when looking at an assessment of the interference from RLAN into FSS.

–	Step 1: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receivers under consideration (see Table 2) (considering the satellite footprint) whilst satisfying the FSS protection criteria.

–	Step 2: This step delivers a sensitivity analysis of the number of active, on-tune, RLAN transmitters using various different assumptions when looking at possible RLAN deployment models. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing when inputting the different assumptions. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

–	Step 3: This step delivers a comparison of the overall results from the measurement campaigns with the range of various results that can be seen from Step 2 and looks to see if the range of the results that may be valid for further study could be reduced.

#### 5.7.1.2.2	Decisions made regarding assumptions in Step 1

In addition to taking these steps to reduce the amount of range of results from the models further the UK have also made some decisions on the choices in the assumptions made on some of the technical parameters that are considered to be reasonable. From the UK perspective these choices were based on the following principles: evidence based where available, realistic, justifiable and not overly conservative.  Table 1 below shows the list of assumptions, the list of some of the choices for technical parameters in the studies and the which choices the UK consider to be reasonable.

Table 1

UK Views on some technical parameters

| Technical Parameters | Choices for technical parameters to be modelled |  | Choice considered reasonable in the UK | Document 4-5-6-7/566 |
| --- | --- | --- | --- | --- |
| % of outdoor usage | 2% | 5% | 2% | UK submitted a paper in the last study cycle document 4-5-6-7/566 which provided an evidence based analysis of the numbers of outdoor Wi-Fi users in the UK. It concluded even with very conservative assumptions it was very difficult to reach 2% outdoor use. In addition, WRC-03 studies on Wi-Fi vs FSS in 5 150-5 250 MHz bands only took account of studies using 1% outdoor use. It should be noted that ECC Report 244 only presents results for 5.3% outdoor usage. |
| Building penetration losses | 12 dB | 17 dB | 17 dB | The different studies contained in ITU-R Report S.2367 which looks at IMT vs FSS in 5 850-6 425 MHz assume a range of 12-35 dB for building penetration losses, so 17 dB still seems a reasonable assumption based on this agreed ITU-R Report and the choices given in ECC Report 244. |
| Antenna Discrimination/Body loss | 0 dB | 4 dB | 4 dB | In previous ITU-R studies 4 dB was considered realistic in the 5 GHz bands RLAN sharing with EESS and in some IMT studies at lower bands around 2 GHz the body losses from mobile devices was up to 6 dB. |
| Polarisation mismatch | 1.5 dB | 3 dB | 1.5 dB | 3 dB may be overestimating the polarisation losses |
| Clutter losses | Free space model | Free space model + Clutter model based on Rec. ITU-R P.452 in relation to antenna height distribution | Free space model + Clutter model based on Rec. ITU-R P.452 in relation to antenna height distribution | Seems a reasonable assumption to include clutter losses for Urban and Suburban environments. |

In both Report ITU-R S.2367 and ECC Report 244 they used a protection criteria of ΔT/T = 6% coordination criteria as provided in Recommendation ITU-R S.1432 for the sharing analysis. There was also further discussion in the ECC Report 244 around how to take account of both geographic and service apportionment. From the UK perspective as the 5 725-5 850 MHz band is only allocated on a primary basis to FSS for ITU-R Region 1 only and as such we believe there is no need to take account for any geographic apportionment. With regards to appropriate service apportionment for this band we believe that the studies should show the results for the full ΔT/T = 6% for the mobile service as you unlikely to have mobile and fixed services sharing the same channels in a given geographic area due to intra-interference. In addition, the use of the band by fixed services now and into the future looks to be very limited based on current information and projected demand.

#### 5.7.1.2.3	Results from UK studies with reference to ECC Report 244 for Step 1 and 2

From the studies shown in ECC Report 244 (see attached annex 1 for relevant parts of ECC Report 244) the initial results of the sharing and compatibility analysis carried out for RLAN vs FSS indicate depending on certain assumptions, sharing can be shown to be feasible in a number of cases. Other results show that sharing would be more difficult based on more conservative assumptions for the RLAN parameters considered. The studies in ECC Report 244 contain a sensitivity analysis highlighting 28 cases (Case 1 to Case 28) which show results that range from conservative figures for RLAN numbers/activity put forward by the satellite industry to less conservative proposals put forward by the RLAN industry. See below for a summary of the relevant satellites studied in Table 2 and Table 3 which summarises the range of results from ECC Report 244.

Table 2

Sample Satellite Data for the band 5 725-5 850 MHz in Region 1

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| D | 3o East | Whole band | 34 | 773 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |

Table 3

Summary of FSS analysis in Region 1

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2 to 2 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4 to 8 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 5 to 15 dB) |

#### 5.7.1.2.4	Additional UK thoughts on step 2

In step 2 there are various input assumptions in the stages we go through to come up with the 28 cases that are presented in ECC Report 244 that are summarised in Table 3 above. If we look into the choices that come up the ranges and make a choice to remove the cases that we think are unrealistic or overly conservative then we can further reduce the cases that may be considered as relevant. If we consider the values chosen to study as a result of Stages 1 – 3 in step 2 to determine the total number of access points accross Europe there is a choice of 300, 400 or 500 million. Looking at the formula the main bulk (91.8% – 92.5%) of these total calculated numbers are from residential access point numbers. In addition the total number of households in Europe used included with 10% increase on current numbers to give 320 million households. Looking at the 500 million figure it looks to have been thought up as a nice number because the calculated numbers were in the range of 300 and 400 million. But when you consider the formula to reach the 500 million number you would have to account for a massive increase 200% in non-residential access points in order to reach the 500 million number. We already found some of the figures used in the the formula to be excessive (90% household penetration in EU) considering that the UK market report for 2015 only had Broadband penetration numbers at 79% of households. Therefore we think that we can eliminate the 500 million number.

The table 4 below gives some of our thoughts on the figures used in stages 4 – 7.

Table 4

UK thoughts on step 2 stage 4 – 6 choices from ECC Report 244

| Stage no and Parameter title | Choices for parameters to be modelled |  |  | Choice considered reasonable in the UK | Reasoning for UK Choice |
| --- | --- | --- | --- | --- | --- |
| Stage 4 Busy Hour Factor | 50% | 62.7% | 70% | 50%, 62.7% | 70% seems a very high figure when you consider the numbers of people who will be working, on holiday, reduction in business use doing other non- WiFi activities out of home and in the home. |
| Stage 5 Apply 5 GHz spectrum factor | 50% | 74% | 97% | 50%, 74% | 97% percent seems extremely high when you consider there are 3 Wi-Fi bands and not all of the home broadband users will need the higher data rates the 5GHz can provide. |
| Stage 6 RF Activity Factor | 3% | 10% | 30% | 3%, 10% | When Wi-FI networks are fully loaded their highest activity factor is around 30% due to the way the protocol works. It is very unlikely that all networks in the busy hour will be fully loaded. |

If we were to input these new assumptions into the ECC Report 244 studies we would end up with 12 different cases for each of the satellites which are summarised below in Table 5.

Table 3

UK choices and new Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1) | 4 | 17 | FSS protection criteria satisfied for all Satellites (margin ranges 1.3 to 12 dB) |
| “Medium” scenario (Case 11) | 4 | 17 | FSS protection criteria satisfied for Satellites B & F (margin ranges 4 to 5 dB). FSS protection criteria exceeded for other Satellites A,D (exceeding ranges 5 to 6 dB) & G (exceeding ranges 2 to 3 dB) |
| “Pessimistic” scenario (Case 23) | 4 | 17 | FSS protection criteria satisfied for Satellites B & F (margin ranges 2 to 7 dB). FSS protection criteria exceeded for other Satellites A,D (exceeding ranges 7 to 8 dB) & G (exceeding ranges 4 to 6 dB) |

#### 5.7.1.2.5	Step 3 – Taking into account Wi-Fi Airborne Measurement Campaigns in 2.4 GHz and 5 GHz

The different factors used in step 2 are subject to some uncertainties because of the difficulties involved when deriving values for these factors and in particular when making predictions for future. Due to the lack of evidence being available, there has been a lot of debate on how aggregate interference from a fully mature rollout of WAS/RLAN would look in the future within the 5 GHz band to satellite and airborne platforms. Therefore, it was agreed in CEPT to perform a sensitivity analyses, taking into account ranges of values for some of these factors.

Unfortunately, the introduction of a large range of assumptions into the theoretical models used in steps 1 and 2 has led to a very large range of results that we see in the 27 cases summarized in table 3 above. This is due largely to the number of variables that has been introduced into the models to be studied, this makes it is difficult to come to any conclusions as the large range of assumptions means predicting an acceptable range of results to make conclusions from is very difficult.

The draft working document towards a PDN Report contained in Annex 25 of the last WP 5A chairman’s report introduces new airborne measurement methods and a proposed methodology for comparing RLAN measurement results in both the 2.4 GHz and 5 GHz bands with the theoretical RLAN aggregate interference modelling used in steps 1 and 2 of the ECC Report defined over a defined geographical area.

When taking into account the UK measurements and our analysis of the airborne measurement campaign results contained in Annex 1 and 2 of the draft working document towards a PDN Report contained in Annex 25 of the last WP 5A Chairman’s Report we came to some initial conclusions that the measured figures were somewhere between the "optimistic" scenario (case 1) and "medium" scenario (case 14) that are summarised above in Table 3.

If we assume the measurement results presented in Annex 1 and 2 of The draft working document towards a PDN Report contained in Annex 25 of the last WP 5A Chairman’s Report give a fair reflection of the situation which may appear in a mature 5 GHz market in the future then we can assume even if we take account and reduce the range of results we use for basing our conclusions on then from table 3 above to that for Case 1  =  up to 12 dB below the threshold and the results for Case 14  = up to 8 dB exceeding the margin we would still be leaving an acceptable amount of conservatism in our overall analysis..

Over all when looking at the studies the UK believe that sharing may be possible between RLAN and FSS in the 5 725-550 MHz band especially if further mitigation techniques are investigated further in order to reduce the negative margins shown in the results for case 14 in Table 3. We believe if we take into account all of these factors with some further analysis that we can prove sharing is feasible even when using what we consider to be conservative assumptions in the analysis.

Additional considerations we think that should be investigated further to try to make the studies looking at the potential for RLAN–FSS sharing in this band are in the areas, such as:

–	Studies looking at lower values of outdoor usage (i.e. 1%, 2%);

–	Studies supporting Stage 8 of FSS Step 2 (see section 8.1.3.4);

–	5 GHz Spectrum Factor (Stage 5 of FSS Step 2);

–	Control / monitoring on the long term aggregate effect of RLAN interference into FSS as RLAN deployment increases and investigation of what can be done in a scenario where the interference threshold is reached;

–	Potential mitigation techniques have to be considered and assessment of their impact on the potential for sharing between RLAN and FSS is required. Their feasibility and practicality of being implemented in RLANs and their potential operational effects could be further studied. Example potential mitigation techniques that could be addressed are:

–	RLAN Access Points deployed only indoor (i.e. no fixed outdoor).

–	Additional power limitations for RLAN.

–	Separate rules for fixed outdoor and mobile RLAN use.

–	More evidence gathered around the numbers used in step 2.

–	More measurement campaigns.

#### 5.7.1.2.6	Some thoughts on the possible introduction of LTE LAA or LTE-U into the bands

LAA-LTE is expected to be deployed in the existing 5 GHz bands by operators, mainly in hotspots and enterprise environments. Therefore, in Annex 1 Appendix 2 of this document we have added an analysis using the same methodology used in ECC Report 244 to estimate what the additional deployments of LAA-LTE on top of the existing RLAN deployments would make the results of the sharing studies with FSS look like. From the additional studies that have been made in CEPT for LAA-LTE, when compared to ECC Report 244 the following conclusions became apparent:

–	Compatibility of a mix LAA and WiFi market share with FSS. Results are roughly the same as for the case of WiFi only.

Therefore, we believe that we can assume that any impact of adding LAA-LTE use case in 5 GHz bands appears to have minimal effect on the overall results of compatibility and sharing as shown in ECC Report 244.

Any additional analysis being considered further for future overall RLAN vs FSS studies should also consider the effect any new mitigations is likely to effect the possible use of the RLAN extension bands by LAA-LTE technologies.

### 5.7.2	Sharing and compatibility in 5 850-5 925 MHz band

[YAHSAT 5A/449]

[Editor’s Note: Further clarification for the text below is needed in the future meetings]

At worldwide level, there may be geographical areas where population densities are higher than in Europe, such as in Africa and Asia. Therefore, densities for RLAN access points should take these geographical areas as well in the studies.

[LUX 5A/264]

Numerous studies have been conducted by interested Administrations (for example, within CEPT SE 24) to estimate the levels of interference which FSS space receivers could absorb without generating a harmful level of interference into FSS space receivers. Extracts of such studies are provided in Doc. 5A/246 [UK] and included here as Study 2. The general conclusions reached indicate that sharing of WAS/RLAN and FSS space receivers would be very difficult and additional techniques should be developed to mitigate the risk of harmful interference created by the aggregate interfering signals originated by transmitting WAS/RLAN stations. Indeed, the agenda item 1.16 called for studies on suitable mitigation techniques (resolves b)). [At the lack of reliable mitigation techniques which could be implemented at affordable costs for WAS/RLAN networks, assumptions on the RLAN characteristics are based on Annex 24 of the WP 5A Chairman´s Report from May 2016 meeting.]

[Any Mitigation Techniques proposed under Agenda Item 1.16 as a possible measure to facilitate compatibility between RLAN should be accompanied by clear implementation steps so as a) to ensure its efficiency, effectiveness and  its practicality of use. Such technique(s) should be implementable without any technical, logistical and operational burden to the administrations/operators of incumbent services/applications to which the band is allocated. RLAN administration(s)/operator(s) should undertake the due diligence to fully respect the above-mentioned conditions and course of actions.]

[LUX 5A/442]

Without reliable mitigation techniques to be proposed, sharing of FSS and RLAN in this band is very difficult. This conclusion takes account that all scenarios considered so far by the ECC Report 244 are valid and realistic and most of the cases studies will present sharing difficulties. Indeed, some previous considerations on the conservativeness of assumptions adopted by the ECC Report 244 have not been endorsed by the recent studies of WP3K/WP3M and, in particular, these WPs (22-29 March 2017) concluded that the analysis of the clutter loss and building entry loss models provided in the ITU Draft New Recommendations  show that:

–	Working Party 5A should not use the clutter component of P.452

–	The clutter loss model of DNR P. [Clutter], currently applicable from 10 GHz to 
100 GHz, could be extended to the 5G Hz range

–	This clutter model would provide lower values for clutter losses at 5 GHz than those currently assumed in ECC Report 244

–	For the building entry loss model, applicable from 80 MHz to 100 GHz, the average values obtained for the building entry loss at 30 degrees elevation angle is 14 dB at 
5.8 GHz and 13.4 dB at 2.4 GHz, which is only a 0.6 dB difference. Some airborne measurements submitted previously to WP 5 A showed a difference of 6.1 dB (8.4 dB at 2.4 GHz and 14.5 dB at 5 GHz)

Building entry losses. It was concluded that indoor/outdoor attenuation (currently estimated as 12 and 17 dB) should be considered as very similar at 2.4 GHz and 5 GHz and not a difference of 8,4 dB at 2,4 GHz and 14,5 dB at 5 GHz. Instead, the building losses are actually 13.4 dB at 2.4 GHz and 
14.5 dB at 5 GHz, almost no significant difference.

The following table provides a summary of the conclusions of the studies for three selected representative scenarios (out of the 27 scenarios considered).

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz | Band 5 850 5 925-MHz |
| --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) | FSS protection criteria satisfied for all FSS groups 1, 2, 3, 4 and 5 (margin ranges 0.1 to 10.3 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 2.6 to 9.9 dB). FSS protection criteria exceeded for other FSS group 4 (exceeding of 0.9 dB) | FSS protection criteria satisfied for FSS groups 1 2 and 3 (margin ranges 0.6 to 8.1 dB). FSS protection criteria exceeded for other FSS groups 4 and 5 (exceeding ranges 0.2 to 2.1 dB) |
|  | 0 | 17 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 8 dB): FSS protection criteria exceeded for other FSS group 4 (exceeding of 2.7 dB) | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 6.3 dB). FSS protection criteria exceeded for other FSS groups 3, 4 and 5 (exceeding ranges 0 to 3.9 dB) |
|  | 0 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 5.2 to 5.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 1.4 to 4.9 dB) | FSS protection criteria satisfied for FSS groups 1 (margin ranges 0 to 4.1 dB) and Satellite N (28.5 dBi) with a margin of 0 dB. FSS protection criteria exceeded for other FSS groups 2 (except satellite N), 3, 4 and 5 (exceeding ranges 0.2 to 6 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2.2 to 2.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4.4 to 7.9 dB) | FSS protection criteria satisfied for FSS group 1 (margin ranges 0.5 to 1.1 dB). FSS protection criteria exceeded for other FSS groups 2, 3, 4 and 5 (exceeding ranges 3 to 9.1 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 0 to 0.7 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 6.5 to 10.1 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 1.1 to 11.2 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 1.1 to 11.9 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 2.9 to 13.1 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 3.3 to 14.1 dB) | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 5.1 to 15.2 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 4.5 to 15.3 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 6.3 to 16.5 dB) |
|  | 4 | 12 | FSS protection criteria exceeded for all FSS groups 2, 3, 4 and 5 (exceeding ranges 6.7 to 17.4 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 8.4 to 18.6 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 3, 4 and 5 (exceeding ranges 8.5 to 19.2 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 10.3 to 20.4 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 10.7 to 21.4 dB) | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 12.4 to 22.6 dB) |

#### 5.7.2.1	Estimation of interference from WAS/RLAN through empirical methods

As a potential way forward to estimate the interference from WAS/RLAN, an innovative contribution was made available to WP 5A (Doc. 5A/91) suggesting the estimation of interference through an empirical method consisting on airborne measurements. A separate report on measurements on interference created by RLAN using airborne is under elaboration.

A detailed analysis on the set of airborne measurements has been made. In general these airborne measurements are potentially interesting to:

- Achieve a better understanding of the study models before the actual RLAN deployment, with a view to determine the actual interference level observed and compare it with calculated results of studies;

- To measure and control, including on the long term, the aggregate interference into FSS space stations when RLAN deployment is developing.

Preliminary conclusions on the airborne measurements do not allow at this stage characterisation of the interference environment and do not allow quantitative conclusions to be drawn. Additional measurements campaigns from airplanes (including in other geographical areas) would be necessary before being able to draw any conclusions from such measurements.

Furthermore, the following issues should be investigated and clarified on possible future airborne measurements:

- Measurements are made locally at low altitude (few kms) whereas FSS space stations cover wide geographic areas and operate from the GSO orbit in space.

- It is not clear how measurements at 2.4 GHz (for which the reference point is not clear, is it less congested than expected?) can help characterise the interference environment at 5 GHz, especially since measurements at 5 GHz are close the noise floor.

- Measurements should be made during busy hours.

- It is not clear how space and time dynamics of an airplane is comparable with the case of GSO satellites (which are seen from the Earth as static in the sky).

- These measurements have considered two elevation angles (30 and 90 degrees). It would seem useful to specifically conduct measurements at elevation and azimuths corresponding to the GSO orbit, since the GSO orbit as seen from the Earth is a particular path in the sky, from 0 degree elevation angle at horizon for the most western and eastern azimuths to about 48 degrees elevation angle at south azimuths, from London.

#### 5.7.2.2	Potential mitigation techniques for WAS/RLAN

It can be noted  sufficient details on potential mitigation techniques are lacking, despite the fact that previous studies concluded on the need of such mitigation techniques. It is understood that further specific sharing studies WAS/RLAN and FSS will be conducted as part of the AI 1.16 work plan; presumably, these studies will conclude on the need of adequate mitigation techniques. See, for example, the conclusions reached by JTG at the previous study cycle for similar sharing studies (Example: Doc. 2357).

For protection of the FSS, two possible mitigation techniques can be identified:

°	Limitation of RLAN deployment to indoor only.

°	Placing a limit on the maximum eirp of RLAN APs. The appropriate value can be determined through additional studies for RLAN under WRC-19 agenda item 1.16 (noting a value of 10 to 15 dBm/20 MHz has been determined by the ITU in the context of studies between IMT and the FSS, with an interference configuration similar to the RLAN case, see Report ITU-R S.2367).

[downward tilting antenna]

These [two] mitigation techniques seem relatively simple to implement but are sometimes difficult to enforce for the RLAN type of equipment (e.g. indoor only, or non-compliant equipment). The impact of such mitigation techniques on the RLAN deployment and operations (e.g. RLAN eirp distribution and the resulting limitations on data rates possible), should be studied.

#### 5.7.2.3	Additional compatibility studies in adjacent bands

Compatibility studies for Regions in which FSS is allocated on a primary basis in 5 725-5 925 MHz between RLAN operating in any of the sub-bands within 5 725-5 925 MHz and the FSS operating in adjacent sub-bands are necessary.

#### 5.7.2.4	Protection of FSS

The protection criterion for FSS is to accept an increase of the equivalent temperature greater than x% due to interference signals on the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.: ΔT/T≤ x%. This criterion is applicable for interference scenarios when the interfering signal is time invariant, which is assumed the case for the aggregate interference levels created by WAS/RLAN in this uplink satellite band, because coverage areas are large (hemispheric or global) and there is no mobility aspect due to the fact that main interference is from RLAN access points which are fixed.

The values of x is 6%, generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment, and x=1%, generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme should be considered where interference from RLANs is limited to half of the ΔT/T= 6% criterion i.e. the ΔT/T objective is reduced to a value of 3%. In addition, geographic apportionment could also be applied depending on the satellite’s coverage and the assumptions for the RLAN deployment.

## 5.8	Consideration of the cross bands sharing and compatibility issues

# 6	Conclusions of sharing and compatibility studies per frequency range and per service

## 6.1	General considerations

## 6.2	Sharing and compatibility results in the band 5 150-5 350 MHz

### 6.2.1	RLAN Sharing and compatibility studies with Radiolocation in 5 150-5 250 MHz

### [USA 5A/381]

Some administrations have enabled RLAN operations that are beyond restrictions specified in Resolution 229 (WRC-12).  Specifically, these administrations authorized RLAN use of this band in co-existence with MSS operations through e.i.r.p. limitations at higher antenna elevation angles: the authorization generally permits indoor and outdoor RLAN operations in 5 150-5 250 MHz at up to 1 Watt conducted and a power spectral density (PSD) of 17 dBm/MHz with an allowance for a 6 dBi antenna gain (i.e. a total 36 dBm e.i.r.p.).  The outdoor operation of WAS/RLANs devices are permitted in the 5 150-5 250 MHz band at these power levels, except that such operations with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW e.i.r.p. to minimize the likelihood of harmful interference to the operating MSS system.  Expressing a limit in terms of e.i.r.p. provides flexibility regarding how to achieve compatibility with non-GSO MSS feeder uplinks.  It is important to note that one of these administrations is also the notifying administration for the single non-GSO MSS system operating in this band and that, to date, no interference issue have been reported to that administration’s regulator. Also, while in-band e.i.r.p. was increased, unwanted emission levels were retained such that all WAS/RLAN station emissions outside of the 5 150 – 5 350 MHz frequency range shall not exceed an e.i.r.p. of −27 dBm/MHz

[RUS 5A/397]

The results of the conducted studies allow to make the following conclusions:

−	sharing of outdoor WAS/RLAN systems operating in the frequency band 5 150-5 250 MHz having the current characteristics with airborne sense and avoid systems is unfeasible;

−	the effective measures for reducing interference for airborne sense and avoid systems operation are to be developed to enable the usage of outdoor WAS/RLAN in the frequency band 5 150-5 250 MHz. The reduction of e.i.r.p values of WAS/RLAN transmitters approximately by 20 dB while increasing the receiver sensitivity can be considered as the effective method for reducing interference. Such method allows to compensate the absence of additional fading in the walls which provided sharing of WAS/RLAN systems with the ARNS systems operating in the considered frequency band.

Without development and implementation of such measures for reducing the interference the decision of possible outdoor WAS/RLAN systems usage in the considered frequency band cannot be made.

### 6.2.2	RLAN Sharing and compatibility studies with Radiolocation in 5 250-5 350 MHz

### [USA 5A/381]

Some administrations permit RLANs to share with incumbent services in the 5 250- 5 350 MHz band. RLAN devices operating in this band utilizing Dynamic Frequency Selection (DFS), which monitors the spectrum transmissions from incumbent radars and changes channels to avoid them when they are detected.  One administration has improved compliance measurement procedures to improve DFS testing for radar detection and eliminate certain outdated performance tests including:

- DFS Security: Required devices to be designed to prohibit software changes that would allow users to disable the DFS functionality.

- DFS Sensing Bandwidth: Modified rules to require devices to sense for radar signals at 100 percent of their emissions bandwidth in the 5 250-5 350 MHz band.  This requirement is applicable to all DFS and as such provides an additional security layer to protect the Terminal Doppler Weather Radar (TDWR) from any possible harmful interference from co-channel RLANS.

- Sensing Threshold: Revised the DFS sensing procedures by introducing a Power Spectral Density (PSD) limit for devices that meet the requirements for a relaxed sensing threshold.  Modified rules to require that devices operate with both an e.i.r.p. of less than 200 mW (23 dBm), and an e.i.r.p. spectral density of less than 10 dBm/MHz (10 mW/MHz), in order to use the relaxed sensing detection threshold of -62 dBm.  Devices that do not meet the proposed e.i.r.p. and e.i.r.p. spectral density requirements must use the -64 dBm sensing threshold.  This change further enhances protection for radars from co-channel interference by reducing both the range and the in-band spectral density of the RLAN devices that use the relaxed sensing threshold.

- Bin 1 Waveforms: RLAN devices are certified using a testing regime that considers how the equipment responds to sample waveforms that simulate typical parameters that are used by radars that operate in these bands.  The radar parameters are divided up into several “bins,” each representing a different category of radar system.

- Channel Spreading: Modified rules to eliminate the requirement that the DFS process provide a uniform spreading of the loading over all of the available channels and revised Compliance Measurement Procedures to remove the channel spreading requirement.

- Channel Loading: Revised Compliance Measurement Procedures to indicate the general requirement that DFS functionality be tested using a method and level of channel loading that is representative of the data types used by the RLAN device without specifying that the system testing be performed with an MPEG test file that streams full motion video at 30 frames per second for channel loading.

[RUS 5A/398]

Results of the conducted studies allow to make the following conclusions:

sharing of the outdoor WAS/RLAN systems operating in the frequency band 5 250-5 350 MHz having the current characteristics with airborne and terrestrial radars is unfeasible;

–	operation of outdoor WAS/RLAN systems in the frequency band 5 250-5 350 MHz would require development of effective measures of reducing interference caused by them to operation of air-borne and ground-based radars. The reduction of e.i.r.p values of WAS/RLAN transmitters approximately by 20 dB while increasing the receiver sensitivity can be considered as the effective method for reducing interference. Such e.i.r.p reduction allows to compensate increase of interference to radiodetermination radars caused by significant reduction of propagation loss due to the absence of additional fading in the walls.

It would be impossible to make decision on feasibility of employing WAS/RLAN systems in the addressed frequency band without development and implementation of such measures.

## 6.5	Sharing and compatibility results in the band 5 350-5 470 MHz

[RUS 5A/196]

The compatibility study results of RLAN with airborne Radars of the aeronautical radionavigation and radiolocation services show that the protection distances from several hundreds km up to line-of-sight distance for outdoor RLAN are required. Consideration of potential multi-source interference from RLAN transmitters show that for protection of airborne Radar receivers the required protection distance is equal to the line-of-sight distance in spite of RLAN transmitter bandwidth. It allows to conclude that increase of RLAN bandwidth cannot be considered as the efficient method of reducing the interference to airborne Radars.

The required protection distances range from several tens km to several hundreds km depending on RLAN transmitter bandwidth to ensure protection of airborne radars from single RLAN. Consideration of multi-source interference shows that the protection distances equal to line-of-sight distance are also required for protection of airborne Radars from indoor RLAN.

In case of interference to ground-based radiolocation Radars the protection distances from single-source interference range from several tens km for outdoor RLAN and indoor RLAN as well. Consideration of multi-source interference results in additional increase of the required protection distance subject to the RLAN transmitters’ density and the considered Radar operational characteristics.

Analysis of the estimated results show that compatibility of RLAN with Radars operating in this frequency band will be extremely difficult. Therefore it is required to develop new efficient interference mitigation techniques. Additional fading in propagation in the walls and increase of the RLAN bandwidth cannot be considered as effective interference mitigation techniques.

[ESA/EUMETSAT 5A/96, 97]

Additional studies performed between WAS/RLAN and EESS (active) confirms that, under all scenarios, RLAN deployment in the 5 GHz range would create large interference to altimeters (up to 26.6 dB) and scatterometers (up to 20.9 dB), hence in the same order of magnitude than for the CSAR sensor (up to 30.4 dB), considered during previous study period.

All together, these studies confirm the previous findings that sharing between RLAN and EESS (active) in the 5350-5470 MHz band may only be feasible if additional RLAN mitigation measures are implemented (see also Recognising a) of Resolution 239 (WRC-15)).

Consistently with Resolution 239 (WRC-15), the work in this frequency band should hence concentrate on studying whether any additional mitigation techniques beyond those analysed in the previous studies would provide coexistence between WAS/RLAN systems and EESS (active) in the in the 5 350-5 470 MHz band.

## 6.6	Sharing and compatibility results in the 5 725-5 850 MHz band

[USA 5A/381]

WAS including RLANs already operate in many countries within the 5 725-5 850 MHz frequency range. CITEL Recommendation PCC.II/REC. 11(VI-05) also recommends the use of the 5 725-5 825 MHz frequency range by WAS including RLANs. The U.S. administration’s rules for RLAN devices operating at 5 725-5 850 MHz provide for up to 1 Watt conducted output power, with a maximum power spectral density of 30 dBm in any 500 kHz.

It should also be noted that the 5 725-5 875 MHz frequency band is designated for Industrial, Scientific, and Medical (ISM) applications. Per RR No. 5.150, “Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13.”  In addition, No. 5.453 includes over 40 countries which have allocated the 5 650-5 850 MHz frequency range to the fixed and mobile services on a primary basis for which the provisions of Resolution 229 (Rev.WRC-12) do not apply.

It should be noted that the FSS allocation in 5 725-5 850 MHz is limited to Region 1.

[RUS 5A/196]

The required protection distances from single-source interference range from several tens km for outdoor RLAN and indoor RLAN as well to ensure protection of ground-based radiolocation radars. Consideration of multi-source interference results in additional increase of the required protection distance subject to the RLAN transmitters’ density and the considered Radar operational characteristics.

Analysis of the estimated results show that compatibility of RLAN with Radars operating in this frequency band will be extremely difficult for some administrations. Therefore it is required to develop new efficient mitigation techniques of interference caused by RLAN. Additional fading in propagation in the walls and increase of the RLAN bandwidth cannot be considered as effective interference mitigation techniques.

[LUX 5A/264]

The general conclusions from previous studies conducted by the CEPT (example: Report 57) were that it was not possible at this time to specify any appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the band 5 725-5 925 MHz while ensuring relevant protection of FSS services in Region 1 in this band.

Considering studies conducted by CEPT (and reported in the SE244 Report), provisional conclusions are derived:

−	Sharing between WAS/RLAN and FSS is not possible. Specific mitigation techniques needs be investigated.

−	Empirical methods to estimate the interference levels from aggregate transmissions from WAS/RLAN will require further studies and analysis (including possibly other the scenarios of interference representing closer the cumulative interference from WAS/RLAN into FSS receiver.

−	Before considering the introduction of RLAN in any of the sub-bands within 5 725-5 925 MHz specific compatibility analysis between RLAN and the FSS operating in adjacent sub-bands are necessary.

## 6.7	Sharing and compatibility results in the band 5 850-5 925 MHz

[LUX 5A/442]

Sharing of RLAN and FSS is difficult. Development of suitable mitigation techniques to be applied by new RLAN systems are required, noting the nature of the typical deployment of RLAN services under unlicensed conditions.

The general conclusions from previous studies conducted by the CEPT (example: Report 57) were that it was not possible at this time to specify any appropriate mitigation techniques and/or operational compatibility and sharing conditions that would allow WAS/RLANs to be operated in the band 5 725-5 925 MHz while ensuring relevant protection of FSS services in this band.

Considering studies conducted by CEPT (and reported in the SE244 Report), provisional conclusions are derived:

−	Sharing between WAS/RLAN and FSS is not possible. Specific mitigation techniques needs be investigated.

−	Empirical methods to estimate the interference levels from aggregate transmissions from WAS/RLAN will require further studies and analysis (including possibly other the scenarios of interference representing closer the cumulative interference from WAS/RLAN into FSS receiver.

−	Before considering the introduction of RLAN in any of the sub-bands within 5 725-5 925 MHz specific compatibility analysis between RLAN and the FSS operating in adjacent sub-bands are necessary.

## 6.8	Frequency bands Cross cut considerations on sharing and compatibility studies

[JPN 5A/393]

#### 6.8.1	Intelligent Transport System Applications under RR No.5.453 in 5 725-5 850 MHz for some countries in Region 3

Some countries have implemented Intelligent Transport System (ITS) applications under the mobile service for Dedicated Short Range Communication (DSRC). DSRC provides communications to connect vehicles to roads, or vehicles to vehicles. The technical and operational characteristics of DSRC for ITS applications for country in Region 3 are listed in Annex XX.

onnec

ANNEX XX

Table 1

Technical and operational characteristics of DSRC for ITS applications for one Country in Region 3
(Based on Recommendation ITU-R M.1453-2)

| Item | Technical characteristics |
| --- | --- |
| Carrier frequencies | 5.8 GHz band for downlink and uplink |
| RF carrier spacing (channel separation) | 5 MHz |
| Allowable occupied bandwidth | Less than 4.4 MHz |
| Modulation method | ASK, QPSK |
| Data transmission speed (bit rate) | 1 024 kbit/s/ASK, 4 096 kbit/s/QPSK |
| Data coding | Manchester coding/ASK, NRZ/QPSK |
| Duplex separation | 40 MHz in case of FDD |
| Communication type | Transceiver type |
| Maximum e.i.r.p.(1) | +30 dBm (downlink) (For a transmission distance of 10 m or less. Power supplied to antenna 10 dBm) |
|  | +44.7 dBm (downlink) (For a transmission distance of more than 10 m. Power supplied to antenna 24.77 dBm) |
|  | +20 dBm (uplink) (Power supplied to antenna 10 dBm) |
| (1) European Radiocommunications Committee (ERC) Recommendation 70-03 specifies values of 2 W e.i.r.p. for active and 8 W e.i.r.p. for passive systems. |  |

Figure 2

Channel arrangement of DSRC for ITS applications for one Country in Region 3 at 5.8 GHz band

[7C 5A/204]

ANNEX 1

List of current and planned EESS (active) systems in the 5 GHz range (non-exhaustive)

Note: This table has been derived to the best knowledge of the delegates attending the Space Frequency Coordination Group (SFCG) and WP 7C and should hence only be used as informative document since no guarantee can be offered that it fully covers all the existing and planned missions with sensors operating in the 5 GHz range.

| Administration | Satellite | Sensor type | # Sats | Apogee/ Perigee (km/km) | Inclination (°) | Sat. Nb | Date of launch (Month and year) | Mission Duration | RAAN (°) | Argument of perigee (°) | True Anomaly (°) | EPOCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canada | RADARSAT-2C | SAR | 1 | 789 | 98.6 | 1 | 12/2007 | 7 years (design life) Still operational | 173.5 | 90.0 | 38.8 | 15166.23953944 |
|  | RADARSAT-3B | SAR | 6 | 617/586 | 97.7 | 1 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | 120° separation | Not launched |
|  |  |  |  |  |  | 2 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 |  | Not launched |
|  |  |  |  |  |  | 3 | 07/2018 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 |  | Not launched |
|  |  |  |  |  |  | 4 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
|  |  |  |  |  |  | 5 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
|  |  |  |  |  |  | 6 | Planned 2022 | 7 years (design life) Operational system, will be renewed | 250.8 | 90.0 | TBD | Not launched |
| China | HY-2 | Altimeter and scatterometer | 1 | 963 | 99.34 | 1 | 08/2011 | 3 years (design life) Still operational | 253.7 | 54.1 | 306 | 17043.685370 |
|  | FY-3-A | Scatterometer | 1 | 854/818 | 98.75 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1 | Planned 11/2018 | no limit in time | 314.17 | 0 | 0 | Not launched |
| France USA | SWOT | Altimeter | 1 | 890.582 (Note 1) | 78 | 1 | Planned 10/2020 | 3 years (design lifetime) | 22.36 | 90.0 | 270.12 (LAN) | Not launched |
| France EUMETSAT USA | JASON2/OSTM /USOCEAN (Note 2) | Altimeter | 1 | 1336 | 66 | 1 | 06/2008 | 6-10 years | 306.05 | 266.33 | 156.34 | 2016 272.15638603 |
|  | JASON3/OSTM /USOCEAN | Altimeter | 1 | 1336 | 66 | 1 | 01/2016 | 6-10 years | 305.6 | 265.85 | 94.16 | 2016 272.53411538 |
| ESA | SENTINEL-1 | SAR | 4 | 701.2/684.5 | 98.183 | 1 | 04/2014 | Up to 12 years | 263.78 | 80.27 | 279.86 | 14/09/2016 04 :33 :48 |
|  |  |  |  |  |  | 2 | 04/2016 | Up to 12 years | 263.30 | 80.37 | 279.77 | 13/09/2016 21 :08 :45 |
|  |  |  |  |  |  | 3 | Planned 2021 | Up to 12 years | 263.XX | 80.XX | TBD | Not launched |
|  |  |  |  |  |  | 4 | Planned 2022 | Up to 12 years | 263.XX | 80.XX | TBD | Not launched |
|  | SENTINEL-3 | Altimeter | 4 | 808.1/791.5 | 98.6 | 1 | 02/2016 | Up to 12 years | 323.58 | 105.7 | 254.43 | 14/09/2016 04 :45 :07 |
|  |  |  |  |  |  | 2 | Planned 2017 | Up to 12 years | 323.XX | 105.XX | Expected 120° from Satellite #1 | Not launched |
|  |  |  |  |  |  | 3 | Planned 2023 | Up to 12 years | 323.XX | 105.XX | TBD | Not launched |
|  |  |  |  |  |  | 4 | Planned 2024 | Up to 12 years | 323.XX | 105.XX | TBD | Not launched |
| EUMETSAT | SENTINEL-6 (Jason-CS) | Altimeter | 2 | 1336 | 66 | 1 | Planned 2020 | 6-10 years | 306 | 90 | 180° separation | Not launched |
|  |  |  |  |  |  | 2 | Planned 2025 | 6-10 years | 306 | 90 |  | Not launched |
|  | ASCAT | Scatterometer | 3 | 832 | 98.7 | 1 | 10/2006 | 10-14 years | 330.19 | 157.1 | 203.07 (mean anomaly) | 2016 272.71848113 |
|  |  |  |  |  |  | 2 | 09/2012 | 10-14 years | 330.65 | 157.03 | 320.85 (mean anomaly) | 2016 272.63852887 |
|  |  |  |  |  |  | 3 | Planned 2018 | 10-14 years | 330.XX | 157.XX | TBD | Not launched |
|  | SCA | Scatterometer | 3 | 832 | 98.7 | 1 | Planned 2021 | 10-14 years | 62.4731+0.98564735 x (Nb Julian days from 1/1/2000) | 90 | 120° separation | Not launched |
|  |  |  |  |  |  | 2 | Planned 2023 | 10-14 years |  | 90 |  | Not launched |
|  |  |  |  |  |  | 3 | Planned 2029 | 10-14 years |  | 90 |  | Not launched |
| India | RISAT | SAR | 1 | 536.38 | 97.6 | 1 | TBD | TBD | TBD | TBD | TBD | TBD |
| Russia |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MINI-RSA (on-board ISS) | SAR | 1 | 400.6/409.6 | 51.64 | 1 | Planned 4Q 2017 | Up to 7 years | 258.95 | 17.78 | 124.40 | 29.09.16 07:56:33 GMT+2 |
|  | GEO-IK-2 (Note 3) | Altimeter | 3 | 1000 | 99.4 | 1 | 06/2016 | no limit in time | TBD | TBD | TBD | TBD |
|  |  |  |  |  |  | 2 | Planned 2017 | no limit in time | TBD | TBD | TBD | TBD |
|  |  |  |  |  |  | 3 | TBD | no limit in time | TBD | TBD | TBD | TBD |

| TOTAL | Altimeters | 13 |
| --- | --- | --- |
|  | Scatterometers | 8 |
|  | SAR | 14 |
|  | TOTAL | 35 |

Note 1: Only 21-day repeat science orbit (890.6 km altitude) is shown; the 1-day repeat “fast-sampling orbit” has a 857.2 km altitude orbit.

Note 2 : starting the week of 3 October 2016 Jason-2 will be moved to an 'interleaved' orbit, ~160° different from Jason-3, to split the ground track and separate them by about 1/2 of the repeat cycle (5 of the ~10 days). The inclination and altitude will be the same, but not the phase.

Note 3 : Satellites of the constellation use  different orbital planes, which are separated by 120°.

[7C 5A/38]

ANNEX 2

Detailed characteristics of EESS (active) systems

Several types of synthetic aperture radars (SAR), altimeters, and scatterometers are identified as EESS (active) missions in the frequency bands between 5 150 MHz and 5 925 MHz to be studied under agenda item 1.16.  Typical characteristics for EESS (active) sensors are shown in Table 1 for frequency overlaps that should be considered in sharing studies within these bands. The actual mission names as well the generic names which will be used in the final recommendation are provided in the table. It should be noted that the service area for most of these active sensors is global.

Table 20

Typical parameters of EESS (active) sensors in the 5 250-5 570 MHz band

| Mission/Sensor | SAR-D1 (Sentinel- 1 (CSAR)) | SAR-D2 (ASAR) | SAR-D3 (RISAT-1) | SAR-D4 (Radarsat-2) | SAR-D5 (Radarsat-3 (RCM)) | SAR-D6 (Radarsat Next Generation (RNG)) |
| --- | --- | --- | --- | --- | --- | --- |
| Sensor type | SAR | SAR | SAR | SAR | SAR | SAR |
| Type of orbit | Circular SSO | SSO, circular | SSO | Near circular | Near circular | Near circular |
| Altitude, km | 693 | 764 | 536 | 792-813 | 586.9-615.2 | 586.9-615.2 |
| Inclination, deg | 98.18 | 98.6 | 97 | 98.6 | 97.74 | 97.74 |
| Ascending Node LST | 18:00/6:00 | 10:30 | 6:00 | 6:00 | 6:00 | 6:00 |
| Repeat period, days | 12 | 35 | 13 | 24 | 12 | 12 |
| Antenna type | Phase array | Phase array | Planar Phased Array | Planar Phased Array | Planar Phased Array | Planar Phased Array |
| Number of beams | 1 | 1 | 1 | 1 | 1 | 1 |
| Antenna Size/diameter | 12.3 m × 0.8 m | 10 m × 1.3 m | 10 m × 3 m | 15 m × 1.5 m | 6.88 m × .37 m | 6.88 m × 1.37 m |
| Antenna Pk Xmt Gain, dBi | 43.5 to 45.3 | 40 to 45 | 35 | 49 | 453 | 453 |
| Antenna Pk Rcv Gain, dBi | 43.5 to 44.8 | 40 to 45 | 35 | 493 | 453 | 453 |
| Polarization | V, H | H, V | Linear H,V | HH, HV, VH, VV | HH, VV, HV, VH, CH, CV | HH, VV, HV, VH, CH, CV |
| Antenna beam look angle, deg | 20-47 | 15-45 | 10-45 | 9-50 | 16-51 | 16-53 |
| Antenna beam azimuth angle, deg | 90 | 90 | 90 | 0 | 0 | 0 |
| Antenna elev. beamwidth, deg | 6 to 8 | 2.5 | 4.6 | 1.88 (for focused beam) | 2.05 (for focused beam) | 2.05 (for focused beam) |
| Antenna az. beamwidth, deg | 0.3 | 0.3 | 1.4 | 0.19 | 0.42 (for focused beam) | 0.42 (for focused beam) |
| Swath width (km) | 20-410 | 10-405 | 10-225 | 18-500 | 20-500 | 20-500 |
| RF center frequency, MHz | 5 405 | 5 331 | 5 350 | 5 405 | 5 405 | 5 405 |
| RF bandwidth, MHz | 100 | 16 | 18.75-75 | 11.6, 17.3, 30, 50, 100 | 14-100 | 14-300 |
| Transmit Pk pwr, W | 4 140 | 2 500 | 4 000 | 2 400 or 3 700 | 1 490 | 1 990 |
| Transmit Ave. pwr, W | 370 | 200 | 260 | 300 | 180 | 240 |
| Pulsewidth, μsec | 5 to 53 | 16 to 41 | 2 0 | 21,42 | 10 to 50 | 10 to 50 |
| Pulse Repetition Frequency (PRF) Hz | 1 450-2 000 | 1 600-2 100 | 3 250 | 1 000-2 800 | 2 000-7 000 | 2 000-7 000 |
| Chirp rate, MHz/μsec | 0.34-3.75 | 0.39 | 0.937-3.75 | 0.27 to 2.38 | 0.14 to 10 | 0.14 to 10 |
| Transmit duty cycle, % | 0.5-9.0 depending on ops mode | 8.61 | 6.5 | Variable, max 8% | Variable, max 12% | Variable, max 12% |
| e.i.r.p. ave, dBW | 70 (for 9% duty cycle) | 68.0 | 68 | Approx. 73 | 67.67 | 69.0 |
| e.i.r.p. peak, dBW | 80 | 78.0 | 71.0 | 83.5 | 76.7 | 78.0 |
| System Noise figure, dB | 3.2 | 4.5 | 5.8 | 6 | 6 | 6 |

| Mission/Sensor | ALT-D1 (JASON-2/3 SSALT, POSEIDON-3/3B) (Note 3) | ALT-D2 (Sentinel 3 SRAL) (Note 1 & 3) | ALT-D3 (HY-2A) (Note 3) | ALT-D4 (Sentinel-6 POSEIDON-4) (Notes 1, 2 & 3) | ALT-D5 (SWOT) (Note 3) |
| --- | --- | --- | --- | --- | --- |
| Sensor type | Altimeter | Altimeter | Altimeter | Altimeter | Altimeter |
| Type of orbit | NSS | Circular, SSO | SSO | NSS | NSS |
| Altitude, km | 1336 | 814 | 963 | 1 336 | 890 |
| Inclination, deg | 66 | 98.65 | 99.3 | 66 | 78 |
| Ascending Node LST | NSS | 22:00 | 06:00 | NSS | NSS |
| Repeat period, days | 10 | 27 | 14 | 10 | 21 |
| Antenna type | Parabolic reflector | Parabolic reflector | Parabolic reflector | Parabolic reflector | Parabolic reflector |
| Number of beams | 1 | 1 | 1 | 1 | 1 |
| Antenna Size/diameter | 1.2 m | 1.2 m | 1.4 m | 1.2 m | 1.2 m |
| Antenna Pk Xmt Gain, dBi | 32 | 34.5 | 35; 43 | 33.5 | 32 |
| Antenna Pk Rcv Gain, dBi | 32 | 34.5 | 35; 43 | 33.5 | 32 |
| Polarization | linear | linear | linear VV | linear | linear |
| Azimuth scan rate, rpm | 0 | 0 | 0 | 0 | 0 |
| Antenna beam look angle, deg | 0 | 0 | 0 | 0 | 0 |
| Antenna beam azimuth angle, deg | 0 | 0 | 0 | 0 | 0 |
| Antenna elev. beamwidth, deg | 3.4 | 3.4 | 2.3 | 3.4 | 3.4 |
| Antenna az. beamwidth, deg | 3.4 | 3.4 | 2.3 | 3.4 | 3.4 |
| Swath width (km) | 79.4 | 48.4 | 38.7 | 97 | 52.9 |
| RF center frequency, MHz | 5 300 | 5 410 | 5 250 | 5 410 | 5 300 |
| RF bandwidth, MHz | 100, 320 | 320 | 160 | 320 | 100, 320 |
| Transmit Pk pwr, W | 17 | 32 | 20 | 25 | 17 |
| Transmit Ave. pwr, W | 0.51 | 0.4 (LRM), 0.25 (SAR) | 8.2 | <2 | 0.51 |
| Pulsewidth, μsec | 106.0 | 49 | 102.4 | 32 | 106.0 |
| Pulse Repetition Frequency (PRF) Hz | 300 | 275 (LRM), 157 (SAR) | 670 | 2 060-9 280 | 300 |
| Chirp rate, MHz/μsec | 0.9, 3.0 | 6.5 | 1.56 | 9.69 | 0.9, 3.0 |
| Transmit duty cycle, % | 3.1 | 1.5 (LRM), 0.7 (SAR) | 40.96 | 30 | 3.1 |
| e.i.r.p. ave, dBW | 29.5 | 30.8 (LRM), 28.4 (SAR) | 44.1 | 36.51 | 29.5 |
| e.i.r.p. peak, dBW | 44.8 | 49.5 | 48 | 47.47 | 44.8 |
| System Noise figure, dB | 4.45 | 3.8 | 3.5 | 3.5 | 4.45 |

Note 1 − Dual frequency radar altimeter (C/Ku Band) which performs measurements either in low resolution mode (LRM) or synthetic aperture radar mode (Nadir-SAR). LRM mode is the conventional altimeter pulse limited mode with interleaved C/Ku Band pulses, while Nadir-SAR mode is the high along track resolution mode based on SAR processing. The system is a two-satellite constellation.

Note 2 – The Poseidon-4 altimeter of Sentinel-6 is an evolution of the Poseidon-3/3B, SIRAL and SRAL altimeters of the Jason-3, CryoSat-2 and Sentinel-3 satellites, respectively.

Note 3 - It should be noted that EESS (active) altimeters may experience the influence from land backscatter (main lobe or side lobes), oceanic surface having anomalous profiles or other unwanted reflections (mispointing errors for example). It is therefore usually agreed to use the value of 2 times the 3 dB beamwidth as a typical value for the reflected area.

Table 1 (continued)

| Mission/Sensor | SCAT-D1 (Metop-A,B,C ASCAT) | SCAT-D2 (Metop-SG SCA) |
| --- | --- | --- |
| Sensor type | Scatterometer | Scatterometer |
| Type of orbit | SSO | SSO |
| Altitude, km | 832 | 832 |
| Inclination, deg | 98.7 | 98.7 |
| Ascending Node LST | 21:30 | 21:30 |
| Repeat period, days | 29 | 29 |
| Antenna type | Six fan beam-antennas (slotted WG arrays) | Six fan beam-antennas (slotted WG arrays) |
| Number of beams | 6 | 6 |
| Antenna Size/diameter | 2.251 m x 0.337 m (mid), 3.003 m x 0.253 m (side) | 2.757 m x 0.315 m (mid), 3.02 m x 0.315 m (side) |
| Antenna Pk Xmt Gain, dBi | 24-32 | 23-31 |
| Antenna Pk Rcv Gain, dBi | 24-32 | 23-31 |
| Polarization | linear VV for all beams | linear VV for all 6 beams + VH/HV and linear HH for the 2 mid-beams |
| Azimuth scan rate, rpm | 0 | 0 |
| Antenna beam look angle, deg | 22-45.6 (mid beams) 29.5-53.4 (side beams) | 17.5-45.5 (mid beams) 24-54 (side beams) |
| Antenna beam azimuth angle, deg | 45, 90, 135, 225, 270, 315 | 45, 90, 135, 225, 270, 315 |
| Antenna elev. beamwidth, deg | 23.6 (mid beams) 23.9 (side beams) | 28 (mid beams) 30 (side beams) |
| Antenna az. beamwidth, deg | 1.5 (mid beams) 1.2 (side beams) | 1.3 |
| Swath width (km) | 550 on each side of the orbit plane | 665 on each side of the orbit plane |
| RF center frequency, MHz | 5 255 | 5 355 |
| RF bandwidth, MHz | 0.5 | 2 |
| Transmit Pk pwr, W | 120 | 2 512 |
| Transmit Ave. pwr, W | 29 (mid beams) 36.5 (side beams) | 92 |
| Pulsewidth, μsec | 10 000 | 1 000 |
| Pulse Repetition Frequency (PRF) Hz | 28.259 | 32 |
| Chirp rate, MHz/μsec | 0.00002 | 0.00002 |
| Transmit duty cycle, % | 28.29 | 3.68 |
| e.i.r.p. ave, dBW | 39 - 47 | 42-50 |
| e.i.r.p. peak, dBW | 53 | 57-65 |
| System Noise figure, dB | 3.0 | 3.5 |

[ESA 5A/96]

ANNEX 3

Sharing studies between RLANs and EESS (active) Altimeter sensor

(Sentinel-3 SRAL sensor)

# 1	Introduction/Background

The present Annex addresses sharing between WAS/RLAN and EESS (active) altimeters sensors, based on the ESA Sentinel-3 SRAL sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters in 5 250-5 570 MHz (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors and given in Document 4-5-6-7/664 (ESA).

# 2	Technical characteristics

## 2.1	EESS (active)

### 2.1.1	Parameters

The EESS (active) parameters and interference criteria used in the present studies are those provided by WP 7C to WP 5A in their liaison statement in Document 5A/38.

Table 1 gives the technical parameters for the SRAL sensor on board Sentinel-3 satellites being developed by ESA for the Copernicus program of the European Commission.

Table 1

| Parameter | Sentinel-3 SRAL |
| --- | --- |
| Sensor type | ALTIMETER |
| Orbital altitude (km) | 800 |
| Orbital inclination (degrees) | 98.65 |
| RF centre frequency (MHz) | 5 410 |
| Peak radiated power (W) | 32 (at ant input) |
| Polarisation | Linear |
| Antenna type | Parabolic reflector 1.2 m |
| Antenna gain (dBi) | 34.5 |
| Antenna pattern steering capability | No |
| Antenna pattern | Based on F.699 |
| Antenna orientation (degrees from nadir) | Nadir (altimeter) |
| Receiver noise figure (dB) | 3.8 |
| Pulse/Receiver bandwidth (MHz) | 320 |
| Noise power (dBW) | –115 |
| Service area | Global |
| Footprint (km2) | 1 840 |

Concerning the polarisation, it is to be noted that Sentinel-3 SRAL makes use of a dual linearly polarised antenna. Therefore no polarisation discrimination advantage can be taken into account.

The antenna pattern used for SRAL is based on Recommendation ITU-R F.699.

### 2.1.2	Protection criteria

The relevant protection criteria is given in Table 2, taken from Recommendation ITU-R RS.1166-4. Even if RLANs are nomadic/mobile by nature, their very high density implies that the interference will be systematic. The relevant percentage of data availability, corresponding to the percentage of time, is therefore 99% (see Document 5A/38)

Table 2

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Altimeter | 4% degradation in height noise | –3 | 99 | 95 |

For the SRAL instrument, the protection criteria calculated over a 320 MHz bandwidth is 
–118.1 dBW (-88.1 dBm) not to be exceeded more than 1% of the time.

This criteria is applied over data acquisition periods of time when the sensor is operating over the measurement area of interest (as per Recommendation ITU-R RS.1166-4).

## 2.2	Mobile service (WAS/RLAN)

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

–	Bandwidth factor:

The bandwidth factor (BWF) used in this document is derived taking into account the positioning of an EESS (active) 100 MHz bandwidth over the RLAN raster/channel plan, as shown below.

On this basis, the following bandwidth factors are considered (see details in Annex 2):

•	160 MHz (15% of RLANs): 1 channel overlaps fully overlaps the EESS bandwidth and 2 overlap by 80 MHz.

–	BWF = 0 dB for 1/3 of cases

–	BWF = 10 *log(80/160) = - 3 dB for 2/3 of cases

This represent an average BWF = -1.76 dB

•	80 MHz (50% of RLANS): 4 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

•	40 MHz (25% of RLANS): 8 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

•	20 MHz (10% of RLANS): 16 channels fully overlaps the EESS bandwidth. No bandwidth factor is applied.

Overall, considering all bandwidth, this represent an average BWF = - 0.4 dB

–	Propagation conditions:

•	building attenuation with a Gaussian distribution (17 dB + 7 dB) truncated at 1 dB, as described in the figure below:

Note: when used to calculate aggregate interference from multiple sources (as in the present case), the impact of this distribution is similar to the one leading from a 12 dB average attenuation.

•	Angular clutter loss model from Recommendation ITU-R P.452 associated with RLAN heights distributions and specific parameters for Urban, Suburban and Rural environments. The antenna heights are randomly selected using a uniform probability distribution from the set of floor heights at 3 meter steps. It should be noted that due to the EESS (active) geometry (Nadir) this model leads to no attenuation.

Antenna height

| RLAN deployment region | Antenna height (metres) |
| --- | --- |
| Urban | 1.5 to 28.5 |
| Suburban | 1.5, 4.5 |
| Rural | 1.5, 4.5 |

–	Antenna gain/discrimination (Omnidirectional in azimuth for all scenarios)

•	Option A1: Omnidirectional in elevation with 0 dBi gain.

•	Option A3: An average 4 dB antenna discrimination is applied to the e.i.r.p. level distribution above in the direction of the satellite Omnidirectional in elevation with 0 dBi gain.

Note: since Option A3 is proposing a fixed discrimination of 4 dB, corresponding results can therefore be extrapolated by shifting by 4 dB the results obtained with Option A1 (0 dBi).

–	Number of active RLAN:

•	Option D1: 11 279 active devices per 100 MHz channel per 5.25 million inhabitants (so-called “Sim City” with 5.25 M inhabitants).

•	Option D2: from 0.004 (D2-low) to 0.04 (D2-high) per 100 MHz channel per inhabitant.

Extrapolation of these numbers from a 100 MHz bandwidth to a 320 MHz bandwidth is detailed in Annex 2 and leads to the following figures:

•	Option D1: 25297 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0048 RLAN per inhabitant.

•	Option D2-Low: 47103 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.00897 RLAN per inhabitant.

•	Option D2-High: 471021 active devices per 320 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0897 RLAN per inhabitant.

These factors lead to the following number of active RLAN to be considered over the French territory (with a population of 66 M inh.), Dutch territory (with a population of 16.8 M inh.) and the UK territory (with a population of 63.3 M inh.):

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 320 MHz) | Nb of active RLAN over NL (in 320 MHz) | Nb of active RLAN over UK (in 320 MHz) |
| --- | --- | --- | --- | --- |
| Option D1 | 0.0048 per inh. | 318 019 | 80 950 | 305 009 |
| Option D2 | Low (0.00897 per inh.) | 592 142 | 150 727 | 567 918 |
|  | High (0.0897 per inh.) | 5 921 422 | 1 507 271 | 5 679 182 |

Detailed deployment assumptions are described in the analysis sections.

# 3	Analysis

Analyses based on static and dynamic methodologies have been used to address the compatibility between WAS/RLAN and EESS (active) altimeter in the 5 GHz range:

–	Section 3.1: static analyses.

–	Section 3.2: dynamic analyses based on existing population densities.

–	Section 3.3: consideration of some additional parameters proposed in PDN Report ITU-R RS.[EESS RLAN 5 GHz].

## 3.1	Static analyses

### 3.1.1	Single entry static analysis

The following Table 4 provides calculation of the impact of 1 single outdoor RLAN on EESS (active) sensors described in Table 1 above.

Table 4

| Parameter | Sentinel-3 SRAL |
| --- | --- |
| Frequency (MHz) | 5 410 |
| Orbital altitude (km) | 800 |
| Off Nadir Angle (°) | 0 |
| Slant path distance (km) | 800 |
| Free Space losses (dB) | 165.2 |
| EESS antenna gain (dBi) | 34.5 |
| EESS protection criteria (dBm/320 MHz) | –88.1 |
| RLAN EIRP (dBm) | 23 |
| Interference from 1 outdoor RLAN (dBm) | –107.7 |
| Margin for 1 outdoor RLAN (dB) | 19.6 |
| Nb of outdoor RLAN in the EESS footprint to reach the protection criteria | 91 |

This calculation shows that 91 outdoor RLANs within the 1840 km² EESS (active) footprint transmitting with the full e.i.r.p. of 200 mW are sufficient to interfere with an EESS (active) system.

This represents a maximum density of less than 0.05 RLAN / km².

### 3.1.2	Static analysis based on EESS (active) footprint shape

The following analysis provides calculation of the aggregate RLAN impact on EESS (active) considering the so-called ‘Sim City” (circular based) with the footprint shape of the EESS (active) system (with a radius of 24.2 km), as shown on the Figure 1 below.

Figure 1

RLAN deployment and EESS (active) footprint

In this case, the percentage of RLAN within the EESS (active) footprint (as on the right figure) can be calculated as in Table 5 below.

Table 5

|  | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 628 | 100% |
| Rural | 30 | 2 121 | 1133 | 53% |

To consider potential interference from RLAN deployment on EESS (active) sensor, one can therefore use these percentages to determine the total number of RLAN in the EESS (active) footprint, for both density Option D1 (total of 25297 RLAN within the overall city for 5.25 M inhabitants), the low edge of Density Option D2 (47102 RLAN) and the upper edge of Density Option D2 (471022 RLAN) as given in Table 6 below.

Table 6

|  | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 25297 | 47102 | 471022 |
| Urban | 100% | 35.5% | 8978 | 16718 | 167176 |
| Suburban | 100% | 53.3% | 13488 | 25115 | 251148 |
| Rural | 53% | 11.2% | 1512 | 2816 | 28159 |
| Total Nb of RLAN in EESS footprint |  |  | 23979 | 44648 | 446482 |

On this basis, the following Table 7 provides calculation of interference for the “EESS (active) footprint shape scenario” for the Sentinel-3 SRAL.

Table 7

| Parameter | Sentinel-3 SRAL with RLAN Density Option D1 | Sentinel-3 SRAL with RLAN Density Option D2-low | Sentinel-3 SRAL with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5410 | 5410 | 5410 |
| Orbital altitude (km) | 800 | 800 | 800 |
| Off Nadir Angle (°) | 0 | 0 | 0 |
| Slant path distance (km) | 800 | 800 | 800 |
| Free Space losses (dB) | 165.2 | 165.2 | 165.2 |
| EESS antenna gain (dBi) | 34.5 | 34.5 | 34.5 |
| Average RLAN EIRP (dBm) (including average BWF) | 18.6 | 18.6 | 18.6 |
| Interference from 1 RLAN (dBm) | -112.1 | -112.1 | -112.1 |
| Nb of RLAN (see table 6 above) | 23979 | 44648 | 446482 |
| Nb of outdoor RLAN (5%) | 1199 (=30.8 dB) | 2232 (=33.5 dB) | 22324 (=43.5 dB) |
| Nb of indoor RLAN | 22780 (=43.6 dB) | 42416 (=46.3 dB) | 424158 (=56.3 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/320 MHz) | -81.3 | -78.6 | -68.6 |
| Interference from indoor RLAN (dBm/320 MHz) | -80.5 | -77.8 | -67.8 |
| TOTAL INTERFERENCE (dBm/320 MHz) | -77.9 | -75.2 | -65.2 |
| EESS protection criteria (dBm/320 MHz) | -88.1 | -88.1 | -88.1 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 10.2 | 12.9 | 22.9 |

This Table shows that, when considering the “footprint shape” scenario, (with RLAN antenna Option A1), the RLAN deployment exceeds the EESS (active) protection criteria from 10.2 dB (with Density Option D1) to 22.9 dB (with Density Option D2-high).

As a summary, the following Table 8 provides the level of interference in excess considering all different RLAN density scenarios and antenna options.

Table 8

Level of interference in excess (static analysis)

| Scenario | Sentinel-3 SRAL |  |
| --- | --- | --- |
| RLAN Antenna | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) |
| JTG Option D1 | 10.2 dB | 6.2 dB |
| JTG Option D2-low | 12.9 dB | 8.9 dB |
| JTG Option D2-high | 22.9 dB | 18.9 dB |

These calculations therefore show that when considering the “footprint shape” scenario, the RLAN deployment largely exceeds the EESS (active) protection up to 22.9 dB and allow to show there is no compatibility between RLAN and EESS (active) altimeters sensors in the 5 GHz range.

## 3.2	Dynamic analyses

### 3.2.1	RLAN deployment

The dynamic analysis have been considered over France (550 000 km² and 66 M inhabitants) on the one hand and over a more restricted area covering the Paris metropolitan area (a square of 10 000 km² and approximately 10.2 M inhabitants) on the other hand.

Some calculations have also been made considering the Netherlands (41 530 km² and 16.8 M inhabitants).

Other simulations have been considered over the UK (244 000 km² and 63.3 M inhabitants) on the one hand and over a more restricted area covering the London metropolitan area (a square of 10 000 km² and approximately 12.9 M inhabitants) on the other hand.

These simulations are properly reflecting the real scenarios, with real population distributions.

Over these areas, different scenarios related to the number of active RLAN were considered as in Table 9 below:

Table 9

Scenarios considered for the dynamic analysis

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Nb of active RLAN over NL (in 320 MHz) | Nb of active RLAN over UK (in 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Option D1 | 0.0048 per inh. | 318 019 | 49 148 | 80 950 | 305 009 | 62 158 |
| Option D2 | Low (0.00897 per inh.) | 592 142 | 91 513 | 150 727 | 567 918 | 115 737 |
|  | High (0.0897 per inh.) | 5 921 422 | 915 129 | 1 507 271 | 5 679 182 | 1 157 369 |

These active RLAN have been deployed following the population densities, as depicted in Figure 2 below.

An EESS (active) measurement area has been defined around France, with an area of about 1 000 000 km² (blue square on Figure 2A), around Paris with an area of 10 000 km² (blue square on Figure 2B), around the Netherlands with an area of about 120 000 km² (blue square on Figure 2C), around the UK, with an area of about 700 000 km² (blue square on Figure 2D) and around London with an area of 10 000 km².

Figure 2a

RLAN deployment and measurement area over France (592 142 RLANs for D2-low)

Figure 2B

RLAN deployment and measurements area over Paris metropolitan (91 513 RLANs for D2-low)

Figure 2C

RLAN deployment and measurements area over the Netherlands (150 727 RLANs for D2-low)

Figure 2D

RLAN deployment and measurements area over the UK (567 918 RLANs for D2-low)

### 3.2.2	Dynamic analyses conditions

Simulations have been run for the SRAL sensor on board Sentinel-3 with a time step of 1 second and for a period of 30 days.

At each step of the simulation (i.e. corresponding to 1 s dynamic of the EESS satellite), the interference to the EESS (active) sensor from each RLAN in visibility is calculated (taking into account the EESS antenna pattern to determine the relative gain), hence leading to an aggregate interference.

The percentage of time of interference is calculated with reference to the measurement area, which means that only the time steps when the sensor antenna boresight is within the blue area are retained for the calculation of the percentage of time of interference.

Then, compiling the aggregate interference over the whole steps of the simulations allows to deriving the interference distribution that will be compared to the EESS (active) protection criteria.

The SRAL sensor has been considered with an antenna pointing at nadir with a payload active 100% of the time.

### 3.2.3	Dynamic analyses results over France

On this basis, Figure 3 gives the cumulative distribution function of interference for deployment of 592 142 RLANs over France (Option D2-low) and antenna Option A1.

Figure 3

Interference for Sentinel-3 (Over France – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 7.1 dB (–110.9 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 4 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 4

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D2-low)

In addition, Figure 5 gives the cumulative distribution function of interference for deployment of 318 019 RLANs over France (Option D1) and antenna Option A1.

Figure 5

Interference for Sentinel-3 (Over France – options A1 and D1)

It can be seen that the EESS (active) protection criterion is exceeded by 3.4 dB (–114.6 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 10% of the time. The situation is also depicted on Figure 6 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 6

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D1)

It appears obvious that in these situations, the Sentinel-3 sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 10 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 10

Interference in excess (over France)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over France (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 318 019 | 3.4 dB | -0.6 dB |
| JTG Option D2-low | 0.00897 | 592 142 | 7.1 dB | 3.1 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 921 422 | 17.1 dB | 13.1 dB |

### 3.2.4	Dynamic analyses results over Paris metropolitan

Under the same principle, the following Figure 7 gives the cumulative distribution functions of interference corresponding to a deployment of 592 142 RLANs over France, including 91 513 RLANs over Paris metropolitan area (Option D2-low) and antenna Option A1.

Figure 7

Interference for Sentinel-3 (Over Paris Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 14.2 dB 
(–103.8 dBW). (The interference level corresponding to the protection criteria is exceeded for 100 % of the time.)

Similarly, the following Figure 8 gives the cumulative distribution functions of interference corresponding to a deployment of 318 019 RLANs over France, including 49 148 RLANs over Paris metropolitan area (Option D1) and antenna Option A1.

Figure 8

Interference for Sentinel-3 (Over Paris Metro – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 11.3 dB 
(–106.7 dBW). (The interference level corresponding to the protection criteria is exceeded 100 % of the time.)

Finally, Table 11 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 11

Interference in excess (over Paris metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 49 148 | 11.3 dB | 7.3 dB |
| JTG Option D2-low | 0.00897 | 91 513 | 14.2 dB | 10.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 915 129 | 24.2 dB | 20.2 dB |

### 3.2.5	Dynamic analyses results over the Netherlands

Under the same principle, the following Figure 9 gives the cumulative distribution functions of interference corresponding to a deployment of 150 727 RLANs over the Netherlands (Option D2-low) and antenna Option A1.

Figure 9

Interference for Sentinel-3 (Over the Netherlands – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10 dB 
(–-108 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 50% of the time. The situation is also depicted on Figure 10 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 10

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the Netherlands – options A1 and D2-low)

It appears obvious that in this situation (150 727 active RLAN corresponding to 0.00897 active RLAN per inh.), the Sentinel-3 sensor will be totally ineffective over the Netherlands.

Finally, Table 12 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 12

Interference in excess (over the Netherlands)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the Netherlands (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 80 950 | 7 dB | 3 dB |
| JTG Option D2-low | 0.00897 | 150 727 | 10 dB | 6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 507 270 | 20 dB | 16 dB |

### 3.2.6	Dynamic analyses results over the UK

Figure 11 gives the cumulative distribution function of interference for deployment of 567 918 RLANs over the United Kingdom (Option D2-low) and antenna Option A1.

Figure 11

Interference for Sentinel-3 (Over the UK – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.6 dB (–-107.4 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 12 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 12

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the UK – options A1 and D2-low)

In addition, Figure 13 gives the cumulative distribution function of interference for deployment of 305 009 RLANs over the UK (Option D1) and antenna Option A1.

Figure 13

Interference for Sentinel-3 (Over the UK – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 7.8 dB 
(–110.2 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 20% of the time. The situation is also depicted on Figure 14 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 14

Interfered portion of images (in red) vs non-interfered (in white) 
(Over the UK – JTG options A1 and D1)

It appears obvious that in these situations, the Sentinel-3 sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 13 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 13

Interference in excess (over the UK)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the UK (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 305 009 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.00897 | 567 918 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 679 180 | 20.6 dB | 16.6 dB |

### 3.2.7	Dynamic analyses results over London metropolitan

Figure 15 gives the cumulative distribution functions of interference corresponding to a deployment of 567 918 RLANs over the UK, including 115 737 RLANs over London metropolitan area (Option D2-low) and antenna Option A1.

Figure 15

Interference for Sentinel-3 (Over London Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 16.6 dB 
(–104.1 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for 100% of the time.

Finally, Table 14 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 14

Interference in excess (over London metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 62 158 | 13.6 dB | 9.6 dB |
| JTG Option D2-low | 0.00897 | 115 737 | 16.6 dB | 12.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 157 369 | 26.6 dB | 22.6 dB |

### 3.2.8	Dynamic analyses – Summary of results

The results of the dynamic analysis over France are given below (see Table 10 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over France (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 318 019 | 3.4 dB | -0.6 dB |
| JTG Option D2-low | 0.00897 | 592 142 | 7.1 dB | 3.1 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 921 422 | 17.1 dB | 13.1 dB |

The results of the dynamic analysis over Paris Metropolitan are given below (see Table 11 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over Paris Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 49 148 | 11.3 dB | 7.3 dB |
| JTG Option D2-low | 0.00897 | 91 513 | 14.2 dB | 10.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 915 129 | 24.2 dB | 20.2 dB |

The results of the dynamic analysis over the Netherlands are given below (see Table 12 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the Netherlands (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 80 950 | 7 dB | 3 dB |
| JTG Option D2-low | 0.00897 | 150 727 | 10 dB | 6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 507 270 | 20 dB | 16 dB |

The results of the dynamic analysis over the UK are given below (see Table 13 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over the UK (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0048 | 305 009 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.00897 | 567 918 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 5 679 180 | 20.6 dB | 16.6 dB |

The results of the dynamic analysis over London Metropolitan are given below (see Table 14 above):

| Scenario | Active RLAN density (Nb per inhabitant per 320 MHz) | Nb of active RLAN over London Metro (in 320 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0048 | 62 158 | 13.6 dB | 9.6 dB |
| JTG Option D2-low | 0.00897 | 115 737 | 16.6 dB | 12.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.0897 | 1 157 369 | 26.6 dB | 22.6 dB |

Overall, in all cases, the above dynamic analyses confirm the result of static analyses, presenting interference largely exceeding EESS (active) protection criteria and allow to show that there is no compatibility between RLAN and EESS (active) in the 5 GHz range.

## 3.3	consideration of some additional parameters

In addition to the RLAN parameters given in section 2 above, PDN Report ITU-R RS.[EESS RLAN 5 GHz] mentions different parametric assumptions as follows:

–	Outdoor ratio: consider 2% and 10% in addition to the agreed 5%.

–	Indoor/outdoor attenuation: consider a fixed 17 dB in additional to the agreed “Gaussian 17 dB +-7 dB standard deviation”.

As expressed in Document 4-5-6-7/664, and validated by dynamic simulations, the impact of these parametric parameters can be calculated as follows:

Aresult = 10 log(Or + (1-Or)10(-IOa/10)

where:

Aresult =	resulting attenuation

Or =	Outdoor ratio

IOa =	Indoor/outdoor attenuation.

On this basis, the following Table 21 provides the corresponding results:

Table 21

Calculated attenuations due to the parametric parameters

|  | Outdoor ratio |  |  |
| --- | --- | --- | --- |
| Indoor/outdoor attenuation | 2% | 5% | 10% |
| Gaussian 17 dB + -7 dB (resulting in 12 dB) | 10.9 (-1.3) | 9.6 (0) | 8.0 (+1.6) |
| Fixed 17dB | 14.0 (-4.4) | 11.6 (-2) | 9.3 (+0.3) |

Note: The figures in brackets ( x) represent the difference compared to the JTG
agreed scenario (in Red: 5% Outdoor ratio and “Gaussian 17 dB + -7 dB”)

It can therefore be seen that the potential impact of the parametric assumptions mentioned in JTG ranges from a potential decrease of the interference of 4.4 dB up to an increase by 1.5 dB and will therefore not change the overall negative conclusions in previous sections.

# 4	Summary

Under all scenarios and simulation methodologies (static and dynamic), the analyses show that RLAN deployment in 5 250-5 570 MHz within the 5 GHz range would create large interference in the SRAL sensor on board the Sentinel-3 satellite.

The static analyses presented above indicate that:

–	the maximum density of outdoor RLANs (transmitting with the full e.i.r.p. of 200 mW) within the 1840 km² EESS (active) is less than 0.05 RLAN / km² (see section 3.1.1)

–	Depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by 6.2 to 22.9 dB (see section 3.1.2)

The dynamic analyses presented above indicate that, depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by (for 0.0048 to 0.0897 active RLAN per inhabitant, respectively):

•	-0.6 to 17.1 dB (case over France) (see section 3.2.3)

•	7.3. to 24.2 dB (case over Paris metropolitan) (see section 3.2.4)

•	3 to 20 dB (case over the Netherlands) (see section 3.2.5)

•	3.8 to 20.6 dB (case over the UK) (see section 3.2.6)

•	9.6 to 26.6 dB (case over London metropolitan) (see section 3.2.7).

It has to be highlighted that these analyses were not considering a number of assumptions that would further increase these negative margins, such as an additional apportionment factor of the protection criteria (since the band is already shared with terrestrial radars). ESA stresses that this apportionment factor has not been introduced in the analysis given the already large negative results obtained under the assumption that no other services could generate interference to EESS (active).

Further, some assumptions related to RLAN remains unclear or unresolved and could also increase the potential interference to EESS (active). This covers in particular the possibilities given to a single RLAN to make use of multiple channels transmission (by means of either orthogonal transmissions or MIMO technique) or to concatenate multiple small channels to provide wider bandwidth with higher power. Such questioning also relates to other applications than RLAN since opening a band to RLAN, low power and unlicensed by nature, will drive the use of different applications such as SRDs, M2M, … (similarly to the current situation in the 2.4 GHz band) or LAA-LTE. Consideration of these additional applications would need to be taken into account.

Overall, this document demonstrates and confirms that RLANs cannot share with EESS (active) in 5 250-5 570 MHz within the 5 GHz range, confirming that sharing between RLAN and EESS (active) in the 5 GHz range may only be feasible if additional RLAN mitigation measures are implemented.

[EUMETSAT 5A/97]

ANNEX 4

Sharing studies between RLANs and EESS (active) scatterometer sensor
(SCA sensor)

# 1	Introduction/Background

The present Annex addresses sharing between WAS/RLAN and EESS (active) scatterometers sensors, based on the EUMETSAT SCA sensor.

It provides static and dynamic analysis taking into account similar assumptions and ranges of RLAN 5 GHz parameters (e.g. antenna gain discrimination, devices densities, outdoor ratio, building attenuation) than those considered for analysis made for SAR sensors and given in Document 4-5-6-7/664 (ESA).

# 2	Technical characteristics

## 2.1	EESS (active)

### 2.1.1	Parameters

The EESS (active) parameters and interference criteria used in the present studies are those provided by WP 7C to WP 5A in their liaison statement in Document 5A/38.

Table 1 gives the technical parameters for the SCA sensor on board EPS-SG satellites being developed by EUMETSAT.

Table 1

| Parameter | EPS-SG SCA |
| --- | --- |
| Sensor type | SCATTEROMETER |
| Orbital altitude (km) | 832 |
| Orbital inclination (degrees) | 98.7 |
| RF centre frequency (MHz) | 5 355 |
| Peak radiated power (W) | 2512 peak (92 average) |
| Polarisation | VV+VH |
| Antenna type | 6 x fan beam antennas (2 mid-beams at +- 90° azimuths and 4 side-beams at +-45° azimuth and +-135° azimuth) |
| Antenna gain (dBi) | 27-30 for mid-beams 23-31 for side-beams |
| Antenna pattern steering capability | No |
| Antenna pattern | See Below |
| Antenna orientation (degrees from nadir) | 17.5-45.5 (mid beams) 24-54 (side beams) |
| Receiver noise figure (dB) | 3.5 |
| Pulse/Receiver bandwidth (MHz) | 2 |
| Noise power (dBW) | –138 |
| Service area | Global |
| Footprint (km2) | 12 400 km² for mid-beams 21 000 km² for side-beams |

Concerning the polarisation, it is to be noted that EPS-SG SCA makes use of a dual linearly polarised antenna. Thus, no polarisation discrimination advantage can be taken into account.

### 2.1.2	Antenna pattern

The SCA antenna system is composed of 6 fan-beam antennas (with 1° aperture).

–	4 x SIDE-beams at 45°, 135°, 225° and 315° (with an antenna beam look angle from nadir ranging 24° to 54°).

–	2 x MID-beams at 90° and 270°  (with an antenna beam look angle from nadir ranging 17.5° to 45.5°).

Corresponding footprints are synthetised on Figure 1 below.

Figure 1

SCA footprints

The antenna pattern model for sharing analysis is given as follows.

G = max(Gmin ; Gver+Ghor);

with:

Ghor =	10 x log (sinc(coefH.sin(Az))²)

coefH =	40

Az = Azimuth angle taken from the pointing angle of the antenna (in radians)

Gver = linear extrapolation from Table 2 (for MID and SIDE beam) with Elevation angle in degrees

Gmin =	–10

Note: The cardinal sinc function is here used in its form: sincx=sin⁡(πx)πx

Table 2

Specific antenna gain for linear extrapolation

Note : the elevation angles are given with reference (0°) at satellite Nadir

The antenna pattern representation for each beam type (MID and SIDE) are given on Figures 1A, 1B, 2A and 2B for both vertical (at 0° azimuth) and horizontal (at elevation corresponding to the maximum gain, i.e. 45° (MID) and 55° (SIDE).

Figure 1A

Figure 2B

Figure 2A

Figure 2B

### 2.1.3	Protection criteria

The relevant protection criteria is given in Table 3, taken from Recommendation ITU-R RS.1166-4. Even if RLANs are nomadic/mobile by nature, their very high density implies that the interference will be systematic. The relevant percentage of data availability, corresponding to the percentage of time, is therefore 99% (see Document 5A/38)

Table 3

| Sensor type | Interference criteria |  | Data availability criteria (%) |  |
| --- | --- | --- | --- | --- |
|  | Performance degradation | I/N (dB) | Systematic | Random |
| Scatterometer | 8% degradation in measurement of normalized radar backscatter to deduce wind speeds | –5 | 99 | 95 |

For the SCA instrument, the protection criteria calculated over a 2 MHz bandwidth is –143 dBW
(-113 dBm) not to be exceeded more than 1% of the time.

This criteria is applied over data acquisition periods of time when the sensor is operating over the measurement area of interest (as per Recommendation ITU-R RS.1166-4).

## 2.2	Mobile service (WAS/RLAN)

RLAN parameters used in the present studies are those agreed in the previous study period and given in the preliminary draft new Report ITU-R RS.[EESS RLAN 5 GHz] (see Annex 35 to Document 4-5-6-7/715 (Chairman’s Report)):

–	e.i.r.p. distribution:

| RLAN e.i.r.p. Level | 200 mW (Omni-Directional) | 80 mW (Omni-Directional) | 50 mW (Omni-Directional) | 25 mW (Omni-Directional) |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 19% | 27% | 15% | 39% |

Note: Such distribution corresponds to a 19 dBm average e.i.r.p.

–	Indoor/outdoor:

Outdoor ratio: 5% (RLAN devices are assumed to be indoors only, based on the requirement to help facilitate coexistence. For the purposes of sharing studies, 5% of the devices should be modelled without building attenuation)

–	Channel Bandwidth distribution:

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

–	Bandwidth factor:

The bandwidth factor (BWF) used in this document is derived taking into account the positioning of an EESS (active) 2 MHz bandwidth over the RLAN raster/channel plan, as shown below.

On this basis, the following bandwidth factors are considered (see details in Annex 2):

•	160 MHz (15% of RLANs): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/160) = -19 dB

•	80 MHz (50% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/80) = -16 dB

•	40 MHz (25% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/40) = -13 dB

•	20 MHz (10% of RLANS): 1 channel overlaps by 2 MHz.

–	BWF = 10 *log(2/20) = -10 dB

Overall, considering all bandwidth, this represent an average BWF = -16 dB

–	Propagation conditions:

•	building attenuation with a Gaussian distribution (17 dB + 7 dB) truncated at 1 dB, as described in the figure below:

Note: when used to calculate aggregate interference from multiple sources (as in the present case), the impact of this distribution is similar to the one leading from a 12 dB average attenuation.

•	Angular clutter loss model from Recommendation ITU-R P.452 associated with RLAN heights distributions and specific parameters for Urban, Suburban and Rural environments. The antenna heights are randomly selected using a uniform probability distribution from the set of floor heights at 3 meter steps. It should be noted that due to the EESS (active) geometry this model leads to no attenuation.

Antenna height

| RLAN deployment region | Antenna height (metres) |
| --- | --- |
| Urban | 1.5 to 28.5 |
| Suburban | 1.5, 4.5 |
| Rural | 1.5, 4.5 |

–	Antenna gain/discrimination (Omnidirectional in azimuth for all scenarios)

•	Option A1: Omnidirectional in elevation with 0 dBi gain

•	Option A3: An average 4 dB antenna discrimination is applied to the e.i.r.p. level distribution above in the direction of the satellite Omnidirectional in elevation with 0 dBi gain

Note: since Option A3 is proposing a fixed discrimination of 4 dB, corresponding results can therefore be extrapolated by shifting by 4 dB the results obtained with Option A1 (0 dBi).

–	Number of active RLAN:

•	Option D1: 11 279 active devices per 100 MHz channel per 5.25 million inhabitants (so-called “Sim City” with 5.25 M inhabitants).

•	Option D2: from 0.004 (D2-low) to 0.04 (D2-high) per 100 MHz channel per inhabitant.

Extrapolation of these numbers from a 100 MHz bandwidth to a 2 MHz bandwidth is detailed in Annex 2 and leads to the following figures:

•	Option D1: 5786 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0011 RLAN per inhabitant.

•	Option D2-Low: 10773 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.0021 RLAN per inhabitant.

•	Option D2-High: 107722 active devices per 2 MHz channel per 5.25 million inhabitants (Sim City), i.e. a density of 0.021 RLAN per inhabitant.

These factors lead to the following number of active RLAN to be considered over the French territory (with a population of 66 M inh.) and the UK territory (with a population of 63.3 M inh.):

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 2 MHz) | Nb of active RLAN over UK (in 2 MHz) |
| --- | --- | --- | --- |
| Option D1 | 0.0011 per inh. | 72 738 | 69 763 |
| Option D2 | Low (0.0021 per inh.) | 135 432 | 129 892 |
|  | High (0.021 per inh.) | 1 354 319 | 1 298 916 |

Detailed deployment assumptions are described in the analysis sections.

# 3	Analysis

Analyses based on static and dynamic methodologies have been used to address the compatibility between WAS/RLAN and EESS (active) altimeter in the 5 GHz range:

–	Section 3.1: static analyses.

–	Section 3.2: dynamic analyses based on existing population densities.

## 3.1	Static analyses

### 3.1.1	Single entry static analysis

The following Table 4 provides calculation of the impact of 1 single outdoor RLAN on EESS (active) SCA sensor, for both MID and SIDE beams described in Table 1 above.

Table 4

| Parameter | SCA SIDE beam | SCA MID beam |
| --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 39 | 31.5 |
| Slant path distance (km) | 1121 | 1001 |
| Free Space losses (dB) | 168.0 | 167.0 |
| EESS antenna gain (dBi)(average over footprint) | 27.1 | 27.5 |
| EESS protection criteria (dBm/2 MHz) | –113 | –113 |
| RLAN EIRP (dBm) | 23 | 23 |
| Bandwidth factor for 20 MHz RLAN (dB) | 10 | 10 |
| Interference from 1 outdoor RLAN (dBm) | -127.9 | -126.5 |
| Margin for 1 outdoor RLAN (dB) | 14.9 | 13.5 |
| Nb of outdoor RLAN in the EESS footprint to reach the protection criteria | 31 | 23 |

This calculations shows that, for the a SIDE beam, 31 outdoor RLANs transmitting with the full e.i.r.p. of 200 mW within the 21 000 km² footprint are sufficient to interfere with the SCA sensor.

Similarly, for the a MID beam, 23 outdoor RLANs transmitting with the full e.i.r.p. of 200 mW within the 12 400 km² footprint are sufficient to interfere with the SCA sensor.

This represents a maximum density of 0.0015 and 0.0018 RLAN / km², respectively.

### 3.1.2	Static analysis based on EESS (active) footprint shape

The following analysis provides calculation of the aggregate RLAN impact on EESS (active) considering the so-called ‘Sim City” (circular based) with the footprint shape of the EESS (active) system, as shown on the Figure 3 below.

Figure 3

RLAN deployment and EESS (active) footprint

Considering the size of the EESS (active) scatterometer footprints (with lengths of 660 km (MID beam) and 930 km (SIDE beam)), they largely extend out of the circular based RLAN deployment and limiting calculation to this deployment only would artificially limit the interference. To avoid this, the portion of the EESS (active) scatterometer footprints outside of the circular based “Sim city” have been considered covering rural areas.

In this case, the percentage of RLAN within the EESS (active) footprint (as on the right figure) can be calculated as in Tables 5A and 5B below (considering footprint widths at center footprint of 18.9 km (MID beam) and 22.6 km (SIDE beam)).

Table 5A

| SIDE beam | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 527 | 84% |
| Rural | 30 | 2 121 | 715 | 34% |
| Rural (outside the city) |  |  | 19 680 |  |

Table 5B

| MID beam | Distance (km) | Area (km²) | Area enclosed in EESS footprint (km²) | Ratio of RLAN within the EESS footprint |
| --- | --- | --- | --- | --- |
| Urban | 5 | 79 | 79 | 100% |
| Suburban | 15 | 628 | 447 | 71% |
| Rural | 30 | 2 121 | 586 | 28% |
| Rural (outside the city) |  |  | 11 288 |  |

To consider potential interference from RLAN deployment on EESS (active) sensor, one can therefore use these percentages to determine the total number of RLAN in the EESS (active) footprint, for both density Option D1 (total of 25297 RLAN within the overall city for 5.25 M inhabitants), the low edge of Density Option D2 (47102 RLAN) and the upper edge of Density Option D2 (471022 RLAN) as given in Tables 6A and 6B below.

Table 6A

| SIDE beam | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 5785 | 10773 | 107722 |
| Urban | 100% | 35.5% | 2053 | 3824 | 38233 |
| Suburban | 84% | 53.3% | 2589 | 4821 | 48204 |
| Rural | 34% | 11.2% | 218 | 406 | 4061 |
| Rural (outside the city)* |  |  | 6006 | 11185 | 111851 |
| Total Nb of RLAN in EESS footprint |  |  | 10866 | 20236 | 202349 |

* RLAN density of 0.31/km² (D1), 0.57/km² (D2-low) and 5.68/km² (D2-high)

Table 6B

| MID | Ratio of RLAN within the EESS footprint | Ratio of RLAN in each city area (based on model agreed in JTG) | Nb of active RLAN in EESS footprint (Option D1) | Nb of active RLAN in EESS footprint (Option D2-low) | Nb of active RLAN in EESS footprint (D2-high) |
| --- | --- | --- | --- | --- | --- |
| Total Nb of RLAN in city |  | 100% | 5785 | 10773 | 107722 |
| Urban | 100% | 35.5% | 2053 | 3824 | 38233 |
| Suburban | 71% | 53.3% | 2195 | 4088 | 40877 |
| Rural | 28% | 11.2% | 179 | 333 | 3333 |
| Rural (outside the city)* |  |  | 3445 | 6416 | 64156 |
| Total Nb of RLAN in EESS footprint |  |  | 7873 | 14660 | 146598 |

* RLAN density of 0.31/km² (D1), 0.57/km² (D2-low) and 5.68/km² (D2-high)

On this basis, the following Tables 7A and 7B provides calculation of interference for the “EESS (active) footprint shape scenario” for the EPS-SG SCA instrument for both SIDE and MID beams.

Table 7A (SIDE BEAM)

| Parameter | EPS-SG SCA (SIDE beam) with RLAN Density Option D1 | EPS-SG SCA (SIDE beam) with RLAN Density Option D2-low | EPS-SG SCA (SIDE beam) with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 39 | 39 | 39 |
| Slant path distance (km) (at center beam) | 1121 | 1121 | 1121 |
| Free Space losses (dB) (at center beam) | 168.0 | 168.0 | 168.0 |
| EESS antenna gain (dBi) (average over footprint) | 27.1 | 27.1 | 27.1 |
| Average RLAN EIRP (dBm) (including average BWF) | 3 | 3 | 3 |
| Interference from 1 RLAN (dBm) | -137.9 | -137.9 | -137.9 |
| Nb of RLAN (see table 6A above) | 10866 | 20236 | 202349 |
| Nb of outdoor RLAN (5%) | 543 (=27.2 dB) | 1012 (=30.1 dB) | 10117 (=40.1 dB) |
| Nb of indoor RLAN | 10323 (=40.1 dB) | 19224 (=42.8 dB) | 192231 (=52.8 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/2 MHz) | -110.6 | -107.9 | -97.9 |
| Interference from indoor RLAN (dBm/2 MHz) | -109.8 | -107.1 | -97.1 |
| TOTAL INTERFERENCE (dBm/2 MHz) | -107.1 | -104.4 | -94.4 |
| EESS protection criteria (dBm/2 MHz) | -113.0 | -113.0 | -113.0 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 5.9 | 8.6 | 18.6 |

Table 7B (MID BEAM)

| Parameter | EPS-SG SCA (MID beam) with RLAN Density Option D1 | EPS-SG SCA (MID beam) with RLAN Density Option D2-low | EPS-SG SCA (MID beam) with RLAN Density Option D2-high |
| --- | --- | --- | --- |
| Frequency (MHz) | 5355 | 5355 | 5355 |
| Orbital altitude (km) | 832 | 832 | 832 |
| Off Nadir Angle (°) (at center beam) | 31.5 | 31.5 | 31.5 |
| Slant path distance (km) (at center beam) | 1001 | 1001 | 1001 |
| Free Space losses (dB) (at center beam) | 167.0 | 167.0 | 167.0 |
| EESS antenna gain (dBi) (average over footprint) | 27.5 | 27.5 | 27.5 |
| Average RLAN EIRP (dBm) (including average BWF) | 3 | 3 | 3 |
| Interference from 1 RLAN (dBm) | -136.5 | -136.5 | -136.5 |
| Nb of RLAN (see table 6B above) | 7873 | 14660 | 146598 |
| Nb of outdoor RLAN (5%) | 394 (=26 dB) | 733 (=28.7 dB) | 7330 (=38.7 dB) |
| Nb of indoor RLAN | 7479 (=38.7 dB) | 13927 (=41.4 dB) | 139268 (=51.4 dB) |
| Average indoor/outdoor attenuation (dB) | 12 | 12 | 12 |
| Interference from outdoor RLAN (dBm/2 MHz) | -110.6 | -107.9 | -97.9 |
| Interference from indoor RLAN (dBm/2 MHz) | -109.8 | -107.1 | -97.1 |
| TOTAL INTERFERENCE (dBm/2 MHz) | -107.2 | -104.5 | -94.5 |
| EESS protection criteria (dBm/2 MHz) | -113.0 | -113.0 | -113.0 |
| Exceeding = Negative Margin (dB) with antenna Option A1 | 5.8 | 8.5 | 18.5 |

This Tables shows that, when considering the “footprint shape” scenario, (with RLAN antenna Option A1), the RLAN deployment exceeds the EESS (active) protection criteria from 5.8 dB (with Density Option D1) to 18.6 dB (with Density Option D2-high).

As a summary, the following Table 8 provides the level of interference in excess considering all different RLAN density scenarios and antenna options.

Table 8

Level of interference in excess (static analysis)

| Scenario | EPS-SG SCA (SIDE beam) |  | EPS-SG SCA (MID beam) |  |
| --- | --- | --- | --- | --- |
| RLAN Antenna | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) | Antenna Option A1 | Antenna Option A3 (= A1 –4 dB) |
| JTG Option D1 | 5.9 dB | 1.9 dB | 5.8 dB | 1.8 dB |
| JTG Option D2-low | 8.6 dB | 4.6 dB | 8.5 dB | 4.5 dB |
| JTG Option D2-high | 18.6 dB | 14.6 dB | 18.5 dB | 14.5 dB |

These calculations therefore show that when considering the “footprint shape” scenario, the RLAN deployment largely exceeds the EESS (active) protection up to 18.6 dB and allow to show there is no compatibility between RLAN and EESS (active) scatterometer sensors in the 5 GHz range.

## 3.2	Dynamic analyses

### 3.2.1	RLAN deployment

The dynamic analysis have been considered over France (550 000 km² and 66 M inhabitants) on the one hand and over a more restricted area covering the Paris metropolitan area (a square of 10 000 km² and approximately 10.2 M inhabitants) on the other hand.

Other simulations have been considered over the UK (244 000 km² and 63.3 M inhabitants) on the one hand and over a more restricted area covering the London metropolitan area (a square of 10 000 km² and approximately 12.9 M inhabitants) on the other hand.

These simulations are properly reflecting the real scenarios, with real population distributions.

Over these areas, different scenarios related to the number of active RLAN were considered as outlined in Table 9 below:

Table 9

Scenarios considered for the dynamic analysis

|  | Nb of active RLAN per inhabitant | Nb of active RLAN over France (in 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Nb of active RLAN over UK (in 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) |
| --- | --- | --- | --- | --- | --- |
| Option D1 | 0.0011 per inh. | 72 738 | 11 241 | 69 763 | 14 217 |
| Option D2 | Low (0.0021 per inh.) | 135 432 | 20 930 | 129 892 | 26 471 |
|  | High (0.021 per inh.) | 1 354 319 | 209 304 | 1 298 916 | 264 708 |

These active RLAN have been deployed following the population densities, as depicted in Figure 2 below.

An EESS (active) measurement area has been defined around France, with an area of about 1 000 000 km² (blue square on Figure 4A), around Paris with an area of 10 000 km² (blue square on Figure 4B), around the Netherlands with an area of about 120 000 km² (blue square on Figure 4C), around the UK, with an area of about 700 000 km² (blue square on Figure 4D) and around London with an area of 10 000 km².

Figure 4a

RLAN deployment and measurement area over France (135 432 RLANs for D2-low)

Figure 4B

RLAN deployment and measurements area over Paris metropolitan (20 930 RLANs for D2-low)

Figure 4C

RLAN deployment and measurements area over the Netherlands (34 474 RLANs for D2-low)

Figure 4D

RLAN deployment and measurements area over the UK (129 892 RLANs for D2-low)

### 3.2.2	Dynamic analyses conditions

Simulations have been run for the EPS-SG SCA sensor (MID beam) with a time step of 1 second and for a period of 30 days.

At each step of the simulation (i.e. corresponding to 1 s dynamic of the EESS satellite), the interference to the EESS (active) sensor from each RLAN in visibility is calculated (taking into account the EESS antenna pattern to determine the relative gain), hence leading to an aggregate interference.

The percentage of time of interference is calculated with reference to the measurement area, which means that only the time steps when the sensor antenna boresight is within the blue area are retained for the calculation of the percentage of time of interference.

Then, compiling the aggregate interference over the whole steps of the simulations allows to deriving the interference distribution that will be compared to the EESS (active) protection criteria.

The SCA sensor has been considered with a payload active 100% of the time.

### 3.2.3	Dynamic analyses results over France

On this basis, Figure 5 gives the cumulative distribution function of interference for deployment of 135 432 RLANs over France (Option D2-low) and antenna Option A1.

Figure 5

Interference for SCA (MID beam) (Over France – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.2 dB (–132.8 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 70% of the time. The situation is also depicted on Figure 6 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 6

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D2-low)

In addition, Figure 7 gives the cumulative distribution function of interference for deployment of 72 738 RLANs over France (Option D1) and antenna Option A1.

Figure 7

Interference for SCA (MID beam) (Over France – options A1 and D1)

It can be seen that the EESS (active) protection criterion is exceeded by 7.7 dB (–135.3 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 40% of the time. The situation is also depicted on Figure 8 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 8

Interfered portion of images (in red) vs non-interfered (in white) (Over France – options A1 and D1)

It appears obvious that in these situations, the SCA sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 10 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 10

Interference in excess (over France)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over France (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 72 738 | 7.7 dB | 3.7 dB |
| JTG Option D2-low | 0.0021 | 135 432 | 10.2 dB | 6.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 354 319 | 20.2 dB | 16.2 dB |

### 3.2.4	Dynamic analyses results over Paris metropolitan

Under the same principle, the following Figure 9 gives the cumulative distribution functions of interference corresponding to a deployment of 135 432 RLANs over France, including 20 930 RLANs over Paris metropolitan area (Option D2-low) and antenna Option A1.

Figure 9

Interference for SCA (MID beam) (Over Paris Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10.8 dB
(–132.2 dBW). (The interference level corresponding to the protection criteria is exceeded for 100 % of the time.)

Similarly, the following Figure 10 gives the cumulative distribution functions of interference corresponding to a deployment of 72 738 RLANs over France, including 11 241 RLANs over Paris metropolitan area (Option D1) and antenna Option A1.

Figure 10

Interference for SCA (MID beam) (Over Paris Metro – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 8.3 dB 
(–134.7 dBW). (The interference level corresponding to the protection criteria is exceeded 100 % of the time.)

Finally, Table 11 provides the levels of interference in excess for the 3 RLAN density options
(D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 11

Interference in excess (over Paris metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 11 241 | 8.3 dB | 4.3 dB |
| JTG Option D2-low | 0.0021 | 20 930 | 10.8 dB | 6.8 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 209 304 | 20.8 dB | 16.8 dB |

### 3.2.5	Dynamic analyses results over the UK

Figure 11 gives the cumulative distribution function of interference for deployment of 129 892 RLANs over the United Kingdom (Option D2-low) and antenna Option A1.

Figure 11

Interference for SCA (MID beam) (Over the UK – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is exceeded by 10.6 dB (–-132.4 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 40% of the time. The situation is also depicted on Figure 12 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 12

Interfered portion of images (in red) vs non-interfered (in white) (Over the UK – options A1 and D2-low)

In addition, Figure 13 gives the cumulative distribution function of interference for deployment of 69 763 RLANs over the UK (Option D1) and antenna Option A1.

Figure 13

Interference for SCA (MID beam) (Over the UK – options A1 and D1)

It can be seen that the EESS (active) protection criterion is largely exceeded by 7.8 dB 
(–135.2 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for more than 30% of the time. The situation is also depicted on Figure 14 below showing in red the interfered portion of images (vs the non-interfered in white).

Figure 14

Interfered portion of images (in red) vs non-interfered (in white) (Over the UK – JTG options A1 and D1)

It appears obvious that in these situations, the SCA (MID beam) sensor will be totally ineffective over most land and coastal areas, in particular all urban and suburban areas.

Finally, Table 12 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 12

Interference in excess (over the UK)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over the UK (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 69 763 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.0021 | 129 892 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 298 916 | 20.6 dB | 16.6 dB |

### 3.2.6	Dynamic analyses results over London metropolitan

Figure 15 gives the cumulative distribution functions of interference corresponding to a deployment of 129 892 RLANs over the UK, including 26 471 RLANs over London metropolitan area (Option D2-low) and antenna Option A1.

Figure 15

Interference for SCA (MID beam) (Over London Metro – options A1 and D2-low)

It can be seen that the EESS (active) protection criterion is largely exceeded by 10.9 dB 
(–132.1 dBW).

It can also be seen that the interference level corresponding to the protection criteria is exceeded for 100% of the time.

Finally, Table 13 provides the levels of interference in excess for the 3 RLAN density options (D1, D2-low and D2-High) and antenna Options A1 and A3.

Table 13

Interference in excess (over London metropolitan)

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0011 | 14 217 | 7.9 dB | 3.9 dB |
| JTG Option D2-low | 0.0021 | 26 471 | 10.9 dB | 6.9 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 264 708 | 20.9 dB | 16.9 dB |

### 3.2.7	Dynamic analyses – Summary of results

The results of the dynamic analysis over France are given below (see Table 10 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over France (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 72 738 | 7.7 dB | 3.7 dB |
| JTG Option D2-low | 0.0021 | 135 432 | 10.2 dB | 6.2 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 354 319 | 20.2 dB | 16.2 dB |

The results of the dynamic analysis over Paris Metropolitan are given below (see Table 11 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over Paris Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 11 241 | 8.3 dB | 4.3 dB |
| JTG Option D2-low | 0.0021 | 20 930 | 10.8 dB | 6.8 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 209 304 | 20.8 dB | 16.8 dB |

The results of the dynamic analysis over the UK are given below (see Table 12 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over the UK (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 – 4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 | 0.0011 | 69 763 | 7.8 dB | 3.8 dB |
| JTG Option D2-low | 0.0021 | 129 892 | 10.6 dB | 6.6 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 1 298 916 | 20.6 dB | 16.6 dB |

The results of the dynamic analysis over London Metropolitan are given below (see Table 13 above):

| Scenario | Active RLAN density (Nb per inhabitant per 2 MHz) | Nb of active RLAN over London Metro (in 2 MHz) | Interference in excess for antenna Option A1 | Extrapolated for antenna Option A3 (A1 –4 dB) |
| --- | --- | --- | --- | --- |
| JTG Option D1 (D2-Low –3 dB) | 0.0011 | 14 217 | 7.9 dB | 3.9 dB |
| JTG Option D2-low | 0.0021 | 26 471 | 10.9 dB | 6.9 dB |
| JTG Option D2-high (D2-Low +10 dB) | 0.021 | 264 708 | 20.9 dB | 16.9 dB |

Overall, in all cases, the above dynamic analyses confirm the result of static analyses, presenting interference largely exceeding EESS (active) protection criteria and allow to show that there is no compatibility between RLAN and EESS (active) in the 5 GHz range.

# 4	Summary

Under all scenarios and simulation methodologies (static and dynamic), the analyses show that RLAN deployment in 5 250-5 570 MHz within the 5 GHz range would create large interference in the SCA sensor on board the EPS-SG satellites.

The static analyses presented above indicate that:

–	the maximum allowed density of outdoor RLANs (transmitting with the full e.i.r.p. of 200 mW) within the 12 400 km² (MID beam) and 21000 km² (SIDE beam) EESS (active) is 0.0015 and 0.0018 RLAN / km², respectively (see section 3.1.1)

–	Depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by 1.8 to 18.6 dB (see section 3.1.2)

The dynamic analyses presented above indicate that, depending on the scenario (different number of active RLAN (D1 and D2) and RLAN antenna A1 and A3), the interference to the EESS (active) sensor will be in excess of the relevant protection criteria by (for 0.0011 to 0.021 active RLAN per inhabitant, respectively):

•	3.7 to 20.2 dB (case over France) (see section 3.3.3)

•	4.3 to 20.8 dB (case over Paris metropolitan) (see section 3.3.4)

•	3.8 to 20.6 dB (case over the UK) (see section 3.3.5)

•	3.9 to 20.9 dB (case over London metropolitan) (see section 3.3.6).

It has to be highlighted that these analyses were not considering a number of assumptions that would further increase these negative margins, such as an additional apportionment factor of the protection criteria (since the band is already shared with terrestrial radars). EUMETSAT stresses that this apportionment factor has not been introduced in the analysis given the already large negative results obtained under the assumption that no other services could generate additional interference to EESS (active).

Further, some assumptions related to RLAN remains unclear or unresolved and could also increase the potential interference to EESS (active). This covers in particular the possibilities given to a single RLAN to make use of multiple channels transmission (by means of either orthogonal transmissions or MIMO technique) or to concatenate multiple small channels to provide wider bandwidth with higher power. Such questioning also relates to other applications than RLAN since opening a band to RLAN, low power and unlicensed by nature, will drive the use of different applications such as SRDs, M2M, … (similarly to the current situation in the 2.4 GHz band) or LAA-LTE. Thus, consideration of these additional applications would need to be taken into account.

Overall, this document demonstrates and confirms again that RLANs cannot share with EESS (active) in 5 250-5 570 MHz within the 5 GHz range, confirming that sharing between RLAN and EESS (active) in the 5 GHz range may only be feasible if additional RLAN mitigation measures are implemented.

[UK 5A/246]

annex 1

FSS study 2 relevant sections from ECC Report 244 showing RLAN vs FSS sharing in the 5 725-5 850 MHz band

Appendix 1

# A1.1	Basic RLAN characteristics

Table A21-1

Basic RLAN (Wi-Fi) transmitter characteristics in the band 5 725-5 850 MHz

|  | RLAN 1 Omni-Indoor | RLAN 2 Omni Outdoor | RLAN 3 Directional Outdoor |
| --- | --- | --- | --- |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 | 30 | 30 |
| Bandwidth (MHz) | 20/40/80/160 | 20/40/80/160 | 20/40/80/160 |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 | 10/7/4/1 | 10/7/4/1 |
| Typical AP Antenna Type | Omni (azimuth) See Table 4.6.4-7, Table 4.8.4-9 and Table 4.10.4-11 Type 1 and 2 | Omni (azimuth) See Table 4.10.4-11, Type 1 and 2 | Directional, See Table 4.10.4-11, Type 3 and 4 |
| AP Antenna directivity gain (dBi) | 0-6 | 6-7 | 12/18 |

The figure below provides the spectrum mask for RLAN as function of the nominal channel bandwidth, typically 20, 40, 80 or 160 MHz

Figure A5-1

Spectrum mask for RLAN

The assumed average channel bandwidths distribution of RLAN devices is given in the following table.

Table A1-22

RLAN channel bandwidth distribution

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |

The next table provides RLAN receiver parameters for the purpose of compatibility studies with RLAN as a victim.

Table A1-23

Basic RLAN receiver characteristics in the band 5 725-5 850 MHz

| System parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Bandwidth (MHz) | 20 | 40 | 80 | 160 |
| kTB dBm / bandwidth | -101 | -98 | -95 | -92 |
| Typical Noise figure dB | 4 |  |  |  |
| Noise Power (dBm / bandwidth) | -97 | -94 | -91 | -88 |
| Typical Sensitivity for MCS0, BPSK (½ coding rate) (dBm) | -92 | -89 | -86 | -83 |
| C/N for MCS0, BPSK (½ coding rate) (dB) | 5 |  |  |  |
| I/N (dB) (note 1) | -6 |  |  |  |
| C/I (dB) | 11 for I/N -6 dB; 5 for I/N 0 dB |  |  |  |
| Maximum antenna gain at the RLAN Access Point (dBi) | See Table 4.8.4-9 and Table 4.10.4-11 |  |  |  |
| Maximum antenna gain at the RLAN user device (dBi) | See Table 4.6.4-7 |  |  |  |
| Note 1: As per Recommendation ITU-R M.1739 ‎[8]‎[8], the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB. Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis. |  |  |  |  |

## A1.1.1	RLAN antenna pattern

The characteristics in Table 4.6.4-7 are representative of an average antenna for all User Equipment within a population of RLAN devices. User Equipment can be defined as mobile or portable devices such as smart phones, tablets, notebooks, wireless scanners etc.

Table A1-24

RLAN User Equipment antenna (mobile/portable device)

| # | Type | Gain (dBi) | Antenna height above ground (m) |
| --- | --- | --- | --- |
| 1 | Omni-directional Antenna | 1.3 | 1 to 1.5 |
| NOTE: This value is the averaged value obtained from a survey on RLAN UE antennas. For simplicity, this antenna is assumed to be isotropic. |  |  |  |

The antenna pattern in Table 4.8.4-9 is considered as a representative average antenna pattern for indoor access points within the RLAN population. The table specifies the gains available at elevation angles; the antenna pattern is omni-directional in azimuth.

Table A1-25

Example of Indoor RLAN Access Point Omni-directional (azimuth) Antenna - 
Elevation Pattern

| Elevation angle θ (Degrees) | Gain (dBi) |
| --- | --- |
| 45 θ 90 | -4 |
| 35 θ 45 | 0 |
| 0 θ 35 | 3 |
| –15 θ 0 | -1 |
| –30 θ –15 | -4 |
| –60 θ –30 | -9 |
| –90 θ –60 | -8 |

The elevation angles in Table 4.8.4-9 are defined from the viewpoint of the RLAN Access Point when mounted to the ceiling. Positive elevation angles are towards the ground and negative elevation angles are towards the sky (typically, the RLAN AP is installed for optimal coverage). The pattern is normalised to 3 dBi gain on boresight.

Table 4.10.4-11 sets out the characteristics of RLAN antennas used on fixed indoor or outdoor equipment such as Access Points, Bridges, P2P or P2MP installations. The corresponding antenna patterns are provided in Figure 4.12.4-1 to Figure 4.16.4-4 below.

Table A1-26

Typical Fixed indoor and outdoor RLAN antenna 
(access points, bridges, P2P and P2MP)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 6 | Indoor & Outdoor | Figure 4.12.4-1 | 6 to 28,5 |
| 2 | Directional Antenna (sector) | 6 | Indoor & Outdoor | Figure 4.13.4-2 |  |
| 3 | Directional Antenna | 12 | Outdoor | Figure 4.14.4-3 |  |
| 4 | Directional Antenna (sector) | 17 | Outdoor | Figure 4.15.4-4 |  |
| NOTE: The (Highly) directional links are often installed on top of buildings |  |  |  |  |  |

Figure A1-6

RLAN 6 dBi Omni – Elevation (left) and Azimuth (right) Radiation Patterns

Figure A1-7

RLAN 6 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure A1-8

RLAN 12 dBi Directional – Elevation (blue) and Azimuth (red) Radiation Patterns

Figure A1-9

RLAN 17 dBi Sector Antenna – Elevation (blue) and Azimuth (red) Radiation Patterns

#### A1.1.1.1	RLAN APs antenna pattern measurements

A measurement campaign (see ‎[46]) was carried out to measure 7 different RLAN APs operating in the 5 GHz band (3 consumer and 4 enterprise 802.11ac Aps ‎[27]). This measurement presented the radiation pattern of all access points from 0° to 360° in azimuth and from −90° to +90° in elevation.

RLAN equipment can broadly be categorised in consumer, enterprise, and industrial equipment. The consumer segment is by far the biggest of the three, accounting for more than 85% of unit shipments.

In order to obtain representative results it was decided to characterise state-of-the art IEEE 802.11 ac ‎[27] consumer and enterprise equipment.

The following IEEE 802.11 ac devices were measured:

Table A1-27

Measured RLAN APs

| Consumer | Enterprise |
| --- | --- |
| Linksys EA6500 | Ubiquiti UAP-AC |
| Asus RT-AC66U | Aruba APIN0225 |
| Netgear R6300 | Zyxel NWA 1123-AC |
|  | Cisco Aircap 37021-E-K9 |

Figure A1-10

Consumer and Enterprise Antenna patterns

Consumer Access Points measured showed relatively low directivities and similar average e.i.r.p. with respect to elevation angles. In the majority of cases the maximum e.i.r.p. was observed at elevations between 20° and 75°.

Enterprise APs present higher directivities than the consumer ones. Consequently, the emission pattern depends strongly on how the AP is positioned (i.e. ceiling mounted or desk position).

### A1.1.2	RLAN power distribution

Table A1-28

RLAN power distribution

| Tx power e.i.r.p. | 1W (directional) | 1 W (omni) | 200mW (omni) | 80mW (omni) | 50mW (omni) | 25mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| Outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

The RLAN power distribution presented here leads to a 5.3 % use of outdoor devices. Sensitivity analysis may be performed with other outdoor use ratio to assess the impact of this parameter on the compatibility studies.

### A1.1.3	RLAN deployment and density of active devices

Two options were considered in CEPT Report 57 ‎[42]‎[42] (see section A3.1.7) :

Option A: From 0.0008 to 0.008 active devices per 20 MHz channel per inhabitant (based on 3% to 30% activity factor) applied to any population size”.

Option B: 4837 active devices per 20 MHz channel or 9871 active devices per 100 MHz channel per 5.25 million inhabitants as derived from the deployment figures provided in the table below.

Section ‎0 provides a detailed analysis on RLAN deployment and density of active devices used fort the compatibility studies with FSS.

# A1.2	FSS (Earth to space) in the band 5 725-5 850 MHz

## A1.2.1	FSS technical characteristics and deployments

In the 125 MHz portion of the band up to 5 850 MHz, this is a Region 1 allocation only (i.e. only Europe, Africa, and some of the northernmost countries in Asia). FSS deployments use the whole band 5 725-5 850 MHz and it is used by transmitting earth stations in the Earth-to-space direction operating only to satellites in geostationary orbits.

The following table provides details of the selection of satellites that have been taken as representative of those requiring protection in the visible portion of the geostationary orbit from Europe. In these frequency bands, the satellite beams cover very large areas of the Earth (using global, hemispherical, zonal or regional beams) as can be seen by the satellite footprint coverage plots in Annex 6 of ECC Report 068 ‎[18].

Table A1-29

Sample Satellite Data for the band 5 725-5 850 MHz

| Satellite | Sub-satellite longitude | Part of Frequency range 5 725-5 875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| D | 3o East | Whole band | 34 | 773 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |

Typical FSS parameters developed by the ITU are provided in Table below.

Table A1-30

Typical FSS parameters in the 6 GHz band

| Parameter | Typical value |
| --- | --- |
| Range of operating frequencies | 5 850-6 700 MHz |
| Antenna diameters (m) | 1.2, 1.8, 2.4, 3.0, 4.5, 8, 16, 32 |
| Antenna reference pattern | Recommendation ITU-R S.465 |
| Range of emission bandwidths | 40 kHz - 72 MHz |
| Receiving space system figure of merit | +5 ↔ -10 dB/K (The database of Recommendation ITU-R S.1328 provides one example with Gsat= 24.8 dBi and Ts= 400 K, corresponding to a G/T of -1.2 dB/K |
| Earth station deployment | All regions, in all locations (rural, semi-urban, urban) |
| Earth station e.i.r.p. density towards the horizon | In accordance with RR No. 21.8 and Recommendation ITU-R S.524-9 |
| Minimum earth station antenna elevation angle, h, (degrees) | 5, 15 and 40 |

## A1.2.2	Protection criteria of FSS systems in the bands 3 400-4 200, 4 500-4 800 and 5 850-6 700 MHz

The criterion adopted for the protection of the FSS is that, on any satellite system, the RLAN emissions should not cause an increase of the equivalent temperature greater that x% of the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.  ΔT/T≤ x%.

In the first instance values for x = 6% (generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment) and x=1% (generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services, e.g. the MS and FS, into FSS) are utilised.

Apportionment of interference allowance

To apportion these FSS protection criteria among the potential sources of interference, an apportionment scheme has been considered where interference from RLANs is limited to half of the dT/T= 6% criterion i.e. the dT/T objective is reduced to a value of 3%. In addition, geographic apportionment is applied; dependent on the satellite’s coverage, this can have no effect on the dT/T objective or can reduce this to 1.5% or 1%.

# A1.3	Compatibility between RLAN and FSS (Earth to space) in the band 5 725-5 850 MHz

## A1.3.1	Interference from RLAN into FSS (Earth to space) in the band 5 725-5 850 MHz

### A1.3.1.1	Methodology

A methodology similar to the one used in ECC Report 206 ‎[26] is used for the purpose of sharing and compatibility studies between RLAN and the FSS in the range 5 725-5 850 MHz.

The methodology follows a 2-step approach as outlined below:

–	Step 1 is described in section ‎0: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receiver under consideration (see Table A1-18) (considering the satellite footprint) whilst satisfying the FSS protection criteria described in section ‎3.2.2.

•	The criterion adopted for the protection of the FSS is that, on any satellite system, the RLAN emissions should not cause an increase of the equivalent temperature greater that x% of the noise temperature of the satellite receiver in clear sky conditions without interference, i.e.  ΔT/T≤ x%. Some initial calculations and results are provided for x=6% (generally applicable for sharing in the case of two co-primary services, e.g. the FSS and the MS as co-primary services without service apportionment) and x=1% (generally applicable to interference from a non-primary service into FSS, or to interference from several co-primary services into FSS such as FS and MS). Further results are presented, taking account of a service and geographic apportionment scheme applied to the dT/T =6% criterion and further modelling considerations.

•	Interference apportionment between various potential sources of interference into FSS can be addressed through the choice of the protection criterion. For example, for sharing between FSS and at least two other co-primary services such as FS and MS, the portion of interference allowed to the MS can be derived by apportioning the total x%. See section ‎0 for more elements and analysis on geographic and service apportionment.

•	The propagation model is described in section A1.3.1.1.1.3.

–	Step 2 is described in section ‎0: This step delivers the number of active, on-tune, RLAN transmitters using a deployment model. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

#### A1.3.1.1.1	Step 1: Calculations for the maximum number of active RLAN transmitters within the FSS footprint

This section presents the calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver considering a range of protection criteria.

##### A1.3.1.1.1.1	Generic calculations

The following parameters are considered in the calculations:

–	RLAN e.i.r.p. distribution and channel bandwidth distribution as provided below.

Table A1-31

RLAN power distribution

| Tx power e.i.r.p. | 1W (directional) | 1 W (omni) | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | all |
| --- | --- | --- | --- | --- | --- | --- | --- |
| indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |

Table A1-32

RLAN channel bandwidth distribution and bandwidth correction values

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10 % | 25 % | 50 % | 15 % |
| Average bandwidth correction ratio | 0.7 | 0.5 | 0.5 | 0.25 |

–	RLAN antenna discrimination towards space: calculations are performed using two values: 0 dBi and 4 dBi.

–	Building (indoor to outdoor) attenuation, calculations are performed using two values: 12 dB and 17 dB.

The basic calculation method is illustrated in the table below. Assuming a free space path loss of 199.8 dB, the number of RLAN devices are increased until ΔT/T = 6 % exactly.

Table A1-33

Example calculation for the maximum number of RLANs

Based on this example calculation, Table A1-36 provides some initial results for the maximum number of RLANs that can be accommodated by the satellite receiver whilst satisfying a dT/T of 6%, taking into account two values of antenna discrimination (0 dB and 4 dB) and two values of building loss (12 dB and 17 dB).

Table A1-34

Initial results for Max number of RLANs for dT/T = 6%

| dT/T = 6% |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 |
| A | 242000 | 606000 | 175000 | 440000 |
| B | 2106000 | 5290000 | 1528000 | 3837000 |
| D | 242000 | 606000 | 175000 | 440000 |
| F | 2106000 | 5290000 | 1528000 | 3837000 |
| G | 375000 | 941000 | 272000 | 683000 |

##### A1.3.1.1.1.2	Service and geographic apportionment

It should be noted that the case of dT/T of 6% (Table A1-36) does not take account of service and geographic apportionment explicitly. The UK do not think there is a need to include any allowances for either geographic or service apportionment for the 5 725-5 850 MHz band but accept that further studies may be required.

##### A1.3.1.1.1.3	Further modelling considerations

There are other well-established factors that should be taken into account when modelling interference on the Earth to space interference paths and in order to achieve a more realistic model, some further modelling has been performed accounting for clutter loss and polarisation mismatch loss.

Clutter loss

Detailed calculations of the impact of clutter loss on the Earth to space interference path are given in ‎[19]. They are based on the method for the calculation of clutter loss on interference paths as set out in Recommendation ITU-R P.452-15 ‎[19].

Depending upon the satellite under consideration, consideration of the clutter loss leads to a percentage increase in the RLAN population in the range 50.34% to 130.16%, as detailed in Table 1-37 below.

Table 1-35

Impact of clutter loss

| Satellite | Increase in the RLAN population when clutter is modelled (%) |
| --- | --- |
| A | 51.43 |
| B | 77.58 |
| D | 51.43 |
| E | 78.08 |
| F | 105.98 |
| G | 118.95 |

Polarisation Mismatch Loss

Rationale for values considered for the polarisation mismatch loss and detailed calculations of its impact are given in ‎[47]. These calculations are based on two values of polarisation mismatch loss, 3 dB and 1.5 dB, applied to those outdoor RLANs that are not exposed to clutter loss.

For all satellites, consideration of polarisation mismatch of 3 dB leads to a percentage increase in the RLAN population of 42% when 12 dB building attenuation is used and 70% when 17 dB building attenuation is used.

When considering a polarisation mismatch loss of 1.5 dB, the increase in the RLAN population is 21% when 12 dB building attenuation is used and 31% when 17 dB building attenuation is used

Maximum number of RLANs after further modelling

Taking into account clutter loss and polarisation mismatch loss, the following Table 1-36 give the maximum number of on-tune, active, RLANs for the bands 5 725-5 850 MHz.

Table 1-36

Maximum number of on-tune RLAN for the 5 725-5 850 MHz band for Step 1 
(apportionment scheme applied)

| Band 5 725-5 850 MHz | With clutter loss and no polarisation mismatch |  |  |  | With clutter loss and polarisation mismatch |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 | 17 | 17 | 12 | 12 |
| A | 91615 | 229416 | 66251 | 166573 | 155746 | 390008 | 94076 | 236534 |
| B | 934959 | 2348496 | 678356 | 1703436 | 1589430 | 3992442 | 963265 | 2418879 |
| D | 91615 | 229416 | 66251 | 166573 | 155746 | 390008 | 94076 | 236534 |
| F | 1084485 | 2724086 | 786844 | 1975863 | 1843624 | 4630945 | 1117318 | 2805726 |
| G | 205266 | 515080 | 148886 | 373857 | 348952 | 875636 | 211418 | 530877 |

### A1.3.1.1.2	Step 2: RLAN deployment model

Step 1, described in section ‎0, provides calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver based on a range of FSS protection criteria; initial results and results based on more advanced modelling and the apportionment scheme summarised in Table 1-16 are presented. For Step 1, the RLAN deployment is considered over all European countries/areas.

Step 2, presented in this section, aims to develop an RLAN deployment model for Europe with an objective to calculate the number of on-tune RLANs over this area in 2025. In theory, if the value delivered by Step 2 is less than or equal to that obtained in Step 1, this suggests that sharing is feasible.

Although it is obvious that both RLAN Access Points (APs) and terminals present interference potential to incumbent services, this section assumes that APs and terminals are not transmitting simultaneously. The calculation of the number of on-tune RLANs can thus be simplified by calculating the number of on-tune RLAN APs using the following methodology.

This methodology consists of 8 stages:

–	Stages 1 to 3 aim at defining the expected total number of RLAN APs over Europe.

–	Stages 4 to 7 are considering RLAN operational parameters to derive the expected total number of on-tune, active, RLANs over Europe.

–	Stage 8 aims to apply an upper bound on channel re-use.

#### A1.3.1.1.3	Elements related to stages 1 to 3

Stage 1: Define RLAN deployment environments and obtain relevant statistics

The aim is to define RLAN deployment environments and obtain statistics for the set of countries/areas considered (e.g. number of households, number of enterprise establishments).

Stage 2: Assign the statistics obtained in Stage 1 to urban, suburban and rural environments

The aim is to assign the statistics obtained in Stage 1 to urban, suburban and rural environments. This will be more realistic than averaging the RLAN APs over the entire European area. A realistic AP density in urban areas, in particular, allows for a practical investigation of the planning constraints (stage 8).

Stage 3: Apply Market penetration factors in different environments

These factors describe RLAN penetration into the different environments e.g. residential, enterprise and the number of APs per household/enterprise.

Stages 1 to 3 calculations

Detailed statistics and elements related to stages 1 to 3 are provided in ‎[44] and ‎[45] using a deployment model for the projected 2025 RLAN deployment in the whole of Europe and/or the EU-28.

Taking into account these elements and the large number of assumptions, it was agreed that some probable scenarios for the situation in 2025 are those defined in Table 1-37 below, based on the following assumptions from ‎[44] And ‎[45]:

–	Household “high” scenario with 90% penetration for EU-28 and 85% for the whole of Europe and medium projected growth of 10.4%.

–	2 RLAN APs per household >120 sqm (scenario A, corresponding to 1.22 AP average) or 2 RLAN APs per household >100 sqm (scenario B, corresponding to 1.35 AP average).

–	Enterprise “medium” scenario.

–	10 Million non-residential hotspots.

–	5% Add-on to cover all other type of usage, such as transport, industrial, mobile wifi, etc.

Table 1-37

Predictions of number of RLAN APs for 2025

On this basis, it was agreed that taking a value of 400 Million RLAN APs across Europe will provide a representative value to address the following Stages 4 to 7.

However, there is a certain level of uncertainty and, in order to reflect this, it was agreed to consider a range from 300 Million to 500 Million RLAN APs in Europe.

#### A1.3.1.1.4	Elements related to Stages 4 to 7

Stage 4: Apply Busy Hour Factor

This Factor gives the percentage of RLAN APs involved in Busy Hour Activity. A range of values is appropriate.

In Stage 3 an estimate of the actual number of APs is given. Stage 4 applies a factor to the output of Stage 3 to obtain the number and density of APs involved in busy hour in urban, suburban and rural environments.

The deployment model should be worst-case i.e. it should aim to model peak RLAN activity rather than average RLAN activity.

ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz considered an average value (over the population in urban, suburban and rural areas) of 62.7% for the busy hour factor, assumed to be dominated by the corporate usage.

Some consideration was given to the development of a refined Busy Hour model. While the Busy Hour factor is different for enterprise and residential environments no evidence could be found that this is the case for urban, suburban and rural areas. The fact that the satellite footprint covers different time zones has also been considered. Nevertheless, some further work might be required to refine the busy hour factor model, for instance by taking into account the non-uniform distribution of traffic over the RLAN population.

It was agreed to consider figures of 50%, 62.7% and 70% for the busy hour factor.

Stage 5: Apply 5 GHz Spectrum Factor

This Factor gives the percentage of RLAN activity at 5 GHz (rather than at 2.4 GHz or 60 GHz). This factor is applied in order to obtain the number and density of APs operating at 5GHz in urban, suburban and rural environments during Busy Hour.

It should be noted that there will be RLAN activity at 2.4 GHz and 60 GHz and, since the frequency range under consideration is at 5 GHz only, some RLAN activity during peak periods can be discounted.

A figure of 80% was originally proposed by the RLAN industry based on an optimistic model of corporate Busy Hour where 5 GHz dominates.

In addition, ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz considered values for the market factor for different environments:

–	80% for urban;

–	80% for suburban, and

–	50% for rural.

These values represent an average figure (over the population in urban, suburban and rural areas) of 74% for the 5 GHz spectrum factor.

Further considerations have led to values of 50% and 97% being exercised. The rationales for these values are described below:

Rationale for a 5 GHz Spectrum Factor of 97%

RLAN has, at this time, 80 MHz of spectrum available in 2.4 GHz and could have potentially 775 MHz of spectrum including the expansion bands in the 5 GHz frequency range. Therefore, considering these two frequency bands but neglecting 60 GHz, about 90% of the spectrum available would be in the 5 GHz range.

Furthermore, the requirement for additional 5 GHz spectrum is mainly based on the need for high throughputs and hence 80 and 160 MHz channels that are only available at 5 GHz, as shown on the RLAN channel bandwidth distribution currently agreed. This distribution depicts a figure of 65% of use of such 80 MHz and 160 MHz bandwidths that cannot be accommodated in the 2.4 GHz band. In addition, the available number of 20 MHz channels is 3 (not overlapping) in the 2.4 GHz band whereas it is 37 in the extended 5 GHz band showing therefore a share of 8% at 2.4 GHz and 92% at 5 GHz for the small channels (20 or 40 MHz), i.e. 35% of the channel use. Taking into account the fact that the other 65% of the channel use (80 and 160 MHz) will only be accommodated at 5 GHz, this gives an overall share of channel use of 3 % (8% × 35%) at 2.4 GHz and 97% (92% × 35% + 65%) at 5 GHz.

Rationale for a 5 GHz Spectrum Factor of 50%

Estimations for the current market deployment of 2.4/5 GHz spectrum were made based upon single (2.4 GHz only) and dual-band (2.4/5 GHz) Wi-Fi products that were shipped (by the overall connectivity market leader) across multiple segments in 2014. By multiplying the “Percentage of single band versus dual band per segment” by the “Percentage of Total products shipped per segment” the percentage of dual-band devices shipped overall was determined.  Based upon the above the percentage of 5 GHz enabled dual band products shipping across all Wi-Fi segments is approximately 57%. This does not take into account any split in spectrum usage for the dual band devices and the 57% value calculated was therefore an overestimation of the 5 GHz spectrum factor as it assumes that all dual band devices are operating in 5 GHz mode only.

Over the coming years although the RLAN industry expects to see a move to an increase in dual-band devices compared to single band devices it is also expected to see a penetration of up to 50% for 60 GHz devices. It was therefore suggested that a 5 GHz spectrum factor of 50% would be a reasonable estimation for current and longer term.

However, consideration of the 60 GHz band use could have an impact on the total number of APs in Europe as calculated under stage 1 to 3 above and would hence require further study.

It is understood that RLAN usage in the different frequency ranges in 2025 is difficult to predict and it was therefore agreed to consider figures of 50%, 74% and 97% for the 5 GHz spectrum factor.

Stage 6: Apply RF Activity Factor

This Factor gives the percentage of time that a RLAN is transmitting. When applied, this factor provides the number and density of active APs operating at 5 GHz during Busy Hour in urban, suburban and rural environments.

Recommendation ITU-R M.1651 sets out a method and example calculations that can be used as inputs to a calculation for the RF activity factor. For example: in the home environment, an RLAN accessing VHiMM for the entire Busy Hour has an individual Activity Factor (related to this VHiMM service only) of 12%.

It is agreed that a range of 3 to 30% is relevant for this factor. Within this range, it is also proposed to consider an activity factor of 10% corresponding to the figure proposed in JTG by the RLAN industry for rural deployment.

Sources: See section ‎0 and CEPT Report 57 ‎[42] (see section A3.1.7). This range covers JTG options A and B. The values of 3% and 30%.

Stage 7: Calculate the number of on-tune RLAN APs per 40 MHz

This stage provides the number and density of active, on-tune, APs operating at 5 GHz during Busy Hour, incident to a 40 MHz victim receiver bandwidth sourced from all (urban, suburban and rural) environments.

As described in ECC Report 244 ‎[47], this factor depends on the relative positioning of the 40 MHz victim receiver bandwidth on the RLAN channelization scheme, the total number of RLAN channels (for each bandwidth) and the RLAN bandwidth distribution. Stage 7 should be considered together with any bandwidth corrections made during Step 1 (see section ‎8.1.2).

As shown in ECC Report 244 ‎[47], there are 2 options to calculate this factor that differ in their outputs (and in where bandwidth correction factors are applied):

–	Option 1: delivers the total number of on-tune, active RLANs overlapping in frequency with the 40 MHz FSS receiver. Bandwidth correction factors are considered in Step 1;

–	Option 2: delivers the equivalent number of on-tune, active, 40 MHz interferers incident to the 40 MHz FSS receiver. Here the bandwidth correction factor is considered in Step 2.

Outputs from Step 1 are easily adjusted but since the bandwidth correction factor is already considered in Step 1 (see section ‎0), Option 1 is considered the simplest to implement in this study.

This leads to a factor of 12.9% of the total number of RLANs in the 5 GHz range overlapping the 40 MHz FSS receiver bandwidth (see detailed calculations in Error! Reference source not found.).

Stages 4 to 7 calculations

The following table provides the aggregate factor from Stages 4 to 7, for the 27 different cases resulting from the range of figures for Stage 4 (busy hour factor, 3 figures), stage 5 (spectrum factor, 3 figures) and stage 6 (activity factor, 3 figures).

Table A1-38

Aggregate factors 4 to 7

|  | Stage 4 | Stage 5 | Stage 6 | Stage 7 | Aggregate |
| --- | --- | --- | --- | --- | --- |
|  | Busy hour population | 5 GHZ factor | Activity factor | 40 MHz FSS | Stage 4 to 7 |
| Case 1 | 50% | 50% | 3% | 12.9% | 0.0010 |
| Case 2 | 50% | 50% | 10% | 12.9% | 0.0032 |
| Case 3 | 50% | 50% | 30% | 12.9% | 0.0097 |
| Case 4 | 50% | 74% | 3% | 12.9% | 0.0014 |
| Case 5 | 50% | 74% | 10% | 12.9% | 0.0048 |
| Case 6 | 50% | 74% | 30% | 12.9% | 0.0143 |
| Case 7 | 50% | 97% | 3% | 12.9% | 0.0019 |
| Case 8 | 50% | 97% | 10% | 12.9% | 0.0063 |
| Case 9 | 50% | 97% | 30% | 12.9% | 0.0188 |
| Case 10 | 62.70% | 50% | 3% | 12.9% | 0.0012 |
| Case 11 | 62.70% | 50% | 10% | 12.9% | 0.0040 |
| Case 12 | 62.70% | 50% | 30% | 12.9% | 0.0121 |
| Case 13 | 62.70% | 74% | 3% | 12.9% | 0.0018 |
| Case 14 | 62.70% | 74% | 10% | 12.9% | 0.0060 |
| Case 15 | 62.70% | 74% | 30% | 12.9% | 0.0179 |
| Case 16 | 62.70% | 97% | 3% | 12.9% | 0.0024 |
| Case 17 | 62.70% | 97% | 10% | 12.9% | 0.0078 |
| Case 18 | 62.70% | 97% | 30% | 12.9% | 0.0235 |
| Case 19 | 70% | 50% | 3% | 12.9% | 0.0014 |
| Case 20 | 70% | 50% | 10% | 12.9% | 0.0045 |
| Case 21 | 70% | 50% | 30% | 12.9% | 0.0135 |
| Case 22 | 70% | 74% | 3% | 12.9% | 0.0020 |
| Case 23 | 70% | 74% | 10% | 12.9% | 0.0067 |
| Case 24 | 70% | 74% | 30% | 12.9% | 0.0200 |
| Case 25 | 70% | 97% | 3% | 12.9% | 0.0026 |
| Case 26 | 70% | 97% | 10% | 12.9% | 0.0088 |
| Case 27 | 70% | 97% | 30% | 12.9% | 0.0263 |

#### A1.3.1.1.5	Overall application of stages 1 to 7

Taking into account the above elements in sections 8.1.3.1 and 8.1.3.2, final results of calculations in this study are summarised in Table A1-39 below.

Table A1-39

Number of on-tune RLAN APs per 40 MHz

|  | Aggregate Stages 4 to 7 (see section 8.1.3.2) | Total number of on-tune RLAN in Europe (for 300 Million APs) | Total number of on-tune RLAN in Europe (for 400 Million APs) | Total number of on-tune RLAN in Europe (for 500 Million APs) |
| --- | --- | --- | --- | --- |
| Case 1 | 0.0010 | 290118 | 386824 | 483530 |
| Case 2 | 0.0032 | 967061 | 1289414 | 1611768 |
| Case 3 | 0.0097 | 2901182 | 3868243 | 4835304 |
| Case 4 | 0.0014 | 429375 | 572500 | 715625 |
| Case 5 | 0.0048 | 1431250 | 1908333 | 2385417 |
| Case 6 | 0.0143 | 4293750 | 5725000 | 7156250 |
| Case 7 | 0.0019 | 562829 | 750439 | 938049 |
| Case 8 | 0.0063 | 1876098 | 2501464 | 3126830 |
| Case 9 | 0.0188 | 5628294 | 7504392 | 9380490 |
| Case 10 | 0.0012 | 363808 | 485078 | 606347 |
| Case 11 | 0.0040 | 1212694 | 1616926 | 2021157 |
| Case 12 | 0.0121 | 3638083 | 4850777 | 6063471 |
| Case 13 | 0.0018 | 538436 | 717915 | 897394 |
| Case 14 | 0.0060 | 1794788 | 2393050 | 2991313 |
| Case 15 | 0.0179 | 5384363 | 7179150 | 8973938 |
| Case 16 | 0.0024 | 705788 | 941051 | 1176313 |
| Case 17 | 0.0078 | 2352627 | 3136836 | 3921045 |
| Case 18 | 0.0235 | 7057881 | 9410507 | 11763134 |
| Case 19 | 0.0014 | 406166 | 541554 | 676943 |
| Case 20 | 0.0045 | 1353885 | 1805180 | 2256475 |
| Case 21 | 0.0135 | 4061655 | 5415541 | 6769426 |
| Case 22 | 0.0020 | 601125 | 801500 | 1001875 |
| Case 23 | 0.0067 | 2003750 | 2671667 | 3339583 |
| Case 24 | 0.0200 | 6011250 | 8015000 | 10018750 |
| Case 25 | 0.0026 | 787961 | 1050615 | 1313269 |
| Case 26 | 0.0088 | 2626537 | 3502050 | 4377562 |
| Case 27 | 0.0263 | 7879611 | 10506149 | 13132686 |

Comparison of these values with that obtained in Step 1 are provided in ‎[47] for the above 27 cases and all satellites, taking into account 400 Million APs.

Compared to these results for 400 Million APs, the case of 300 Million and 500 Million AP can be extrapolated with a factor -1.25 dB and +0.97 dB, respectively, as shown in ECC Report 244 ‎[47].

In order to summarise these results, it was agreed to present the results for 3 specific scenarios, as described in the Table A1-40 below:

Table A1-40

Scenarios for FSS summary analysis

| Scenario | Number of AP | Busy hour factor | Spectrum factor | RF activity factor | Total number of on-tune RLAN in Europe |
| --- | --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 300 M | 50 % | 50 % | 3 % | 290118 |
| “Medium” scenario (Case 14 above) | 400 M | 62.7 % | 74 % | 10 % | 2393050 |
| “Pessimistic” scenario (Case 27 above) | 500 M | 70 % | 97 % | 30 % | 13132686 |

Also, the different FSS satellites have been split in 5 different groups as in Table A1-41, considering similarities in interference potential, taking into account their main characteristics (e.g. max antenna gain, receiving system noise, orbital position, etc.)

Table A1-41

FSS groups for FSS summary analysis

| FSS Group | Satellites | Main characteritics | Remarks |
| --- | --- | --- | --- |
| FSS Group 1 | B F | Typical 26.5 dBi antenna gain | Global beams |
| FSS Group 2 | G | Typical 28.5 dBi antenna gain | Global beams (Satellite G presents a max antenna gain of 34 dBi but a higher system noise and a far east orbital position) |
| FSS Group 4 | A D | Around 34 dBi antenna gain | Hemispherical beams |

On this basis, the FSS analysis can be summarised as given in Table A1- 42 below, based on results taking into account clutter losses and polarisation mismatch with a 3 dB figure. If a figure of 1.5 dB is considered for the polarisation mismatch, the results would be modified by 0.7 dB (for the cases related to 12 dB building loss) and 1.13 dB (for the cases related to 17 dB building loss). For further details, see sections ‎0 and ECC Report 244 ‎[47]. As an example, the excess interference would be increased by these values.

Table A1- 42

Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.3 to 12 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 2.6 to 9.9 dB). FSS protection criteria exceeded for other FSS group 4 (exceeding of 0.9 dB) |
|  | 0 | 17 | FSS protection criteria satisfied for FSS groups 1 and 2 (margin ranges 0.8 to 8 dB): FSS protection criteria exceeded for other FSS group 4 (exceeding of 2.7 dB) |
|  | 0 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 5.2 to 5.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 1.4 to 4.9 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 2.2 to 2.9 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 4.4 to 7.9 dB) |
|  | 4 | 12 | FSS protection criteria satisfied for FSS group 1 (margin ranges 0 to 0.7 dB). FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 6.5 to 10.1 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 1.1 to 11.9 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2 and 4 (exceeding ranges 3.3 to 14.1 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 4.5 to 15.3 dB) |
|  | 4 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 6.7 to 17.4 dB) |
|  | 0 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 8.5 to 19.2 dB) |
|  | 0 | 12 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 10.7 to 21.4 dB) |

It is important to understand that the potential for RLAN–FSS sharing (Step 2 in particular) is based on some assumptions that may still need further study and, although providing some relevant results, it is at this stage too early to draw any definite conclusions with regard to the potential for RLAN and FSS to share at 5 GHz.

Stage 8: Apply upper bound on channel re-use

The demographic statistics show that in the European Union (EU28) approximately 50% of the population and households (see ‎[37], ‎[38], ‎[39], ‎[40], 44 and ‎[45]) are situated in less than 2% of the total land mass. This stage focusses on applying an upper bound to the number of co-channel RLANs using a practical planning method (i.e. applying a minimum re-use distance between co-frequency RLANs).

This method uses maximum channel re-use factors for active APs per km2 to derive an upper bound in densely populated areas.

This stage provides an adjustment to the results of Stage 7 by introducing an upper bound on the maximum density per km2 of RLAN APs in highly populated urban areas (e.g. London, UK).

The principle of this Stage 8 is agreed but it is recognised as not being trivial to calculate and quite time consuming. In addition, considering the quite large FSS footprint and since it may only apply to high densely population area, its potential advantage needs to be demonstrated.

At the time where this report was finalised, no studies were provided for Stage 8.

#### A1.3.1.1.6	Summary – analysis of results

The studies have focused on the assessment of the interference from RLAN into FSS and follow a two-step approach:

–	Step 1 is described in section ‎0: This step calculates the maximum number of active, on-tune, RLAN transmitters that can be accommodated by the satellite receiver under consideration (see Table A1-18) (considering the satellite footprint) whilst satisfying the FSS protection criteria described in section ‎3.2.2.

–	Step 2 is described in section ‎0: This step delivers the number of active, on-tune, RLAN transmitters using a deployment model. The Step 2 outputs can be compared with the Step 1 values in order to assess the potential for sharing. In theory, if the Step 2 values are less than or equal to the Step 1 values, then the results suggest that sharing is possible; else if the Step 2 values are greater than the Step 1 values, sharing is not possible.

Concerning step 1, results have been obtained considering 2 different values of building attenuation for indoor use (12 and 17 dB), two values of antenna discrimination (0 and 4 dB), and an approach to service and geographic apportionment of the FSS protection criteria of ΔT/T=6%.

Further modelling takes account of clutter loss and polarisation mismatch loss on the Earth to space interference path.

The different factors used in step 2 are subject to some uncertainties because of the difficulties involved when deriving values for these factors and in particular when making predictions for 2025. Therefore it was agreed to preform sensitivity analyses, taking into account ranges of values for some of these factors.

Calculations and results are presented in this report but, although providing some relevant results, it is at this stage too early to draw definite conclusions.

Conclusions on the potential for RLAN–FSS sharing will be developed in the Part 2 Report, taking into account additional considerations, such as:

–	Antenna discrimination for outdoor RLANs;

–	Further studies on polarisation mismatch;

–	Studies supporting Stage 8 of FSS Step 2 (see section ‎0);

–	5 GHz Spectrum Factor (Stage 5 of FSS Step 2);

–	Control / monitoring on the long term aggregate effect of RLAN interference into FSS as RLAN deployment increases and investigation of what can be done in a scenario where the interference threshold is reached;

–	Further studies on apportionment of the FSS protection criteria.

#### A1.3.1.1.7	Potential mitigation techniques

Some potential mitigation techniques may need to be considered and their impact on the potential sharing between RLAN and FSS should be assessed. Among others, the following potential mitigation techniques could be addressed:

–	RLAN Access Points deployed only indoor;

–	Additional power limitation for RLAN.

There is a need for studies on the feasibility and practicability on the potential mitigation techniques.

aNNEX 1 - Appendix 2

Open Issues from ECC Report 244: LAA-LTE

# A2.1	LAA-LTE Characteristics

## A2.1.1	LAA-LTE RF characteristics

This section includes the LAA-LTE RF characteristics, such as transmission power, channel bandwidth, antenna characteristics, etc. which are relevant for the compatibility studies in the band 5 725-5 925 MHz

### A2.1.1.1	LAA-LTE transmission power characteristics

The first aspects taken into account are related to the maximum transmit power and in-band emission.

Table 43 and Table 44 show the basic characteristics of an LAA-LTE transmitter and a user equipment agreed for standardization ‎[3] ‎[4] . As it can be observed, LAA-LTE will fulfil same requirements as for RLAN characteristics used in ECC Report 244 in terms of conducted and radiated power and power spectral density limits. Also, available channel bandwidths will be aligned with those assumed for RLAN. In LAA-LTE, basic transmission bandwidth per carrier is 20 MHz. Transmissions on bandwidths higher than 20 MHz are done via carrier aggregation.

Table 43

Basic LAA-LTE Base station characteristics in the band 5 725-5 925 MHz

| System Parameters | Indoor |  | Outdoor |  |
| --- | --- | --- | --- | --- |
|  | RLAN | LAA-LTE | RLAN | LAA-LTE |
| Maximum Transmit Power (e.i.r.p. - dBm) | 23 |  | 30 |  |
| Bandwidth (MHz) | 20/40/80/160 |  | 20/40/80/160 |  |
| Maximum Transmit Power Density (e.i.r.p. - dBm/MHz) | 10/7/4/1 |  | 17/14/11/8 |  |
| Typical AP Antenna Type | Omni (azimuth) | Omni-directional (azimuth) | Omni (azimuth) | Omni-directional (azimuth) |
| AP Antenna gain + cable loses (dBi) | 0-6 | 5 | 6-7 | 5 |
| AP Antenna Height (m) | 6-28.5 | 6 | 6-28.5 | 10 |

Table 44

Basic LAA-LTE UE characteristics in the band 5 725-5 925 MHz

| System parameter | Value |  |
| --- | --- | --- |
|  | RLAN | LAA-LTE |
| Maximum Transmit Power (e.i.r.p. - dBm) |  | 23 |
| Bandwidth (MHz) | 20/40/80/160 |  |
| kTB (dBm / bandwidth) | -101 |  |
| Typical Noise figure (dB) | 4 | 4 |
| Noise Power (dBm / bandwidth) | -97 | -97 |
| I/N (dB) | -6 |  |
| Maximum antenna gain (dBi) | 1.3 | 0 |
| Antenna Height (m) | 1-1.5 | 1.5 |

### A2.1.1.2	LAA-LTE UE power distribution

This section describes the LAA-LTE power distribution. For the case of user equipment (UE), transmit power levels will be allocated following the baseline 3GPP Fractional Power Control (FPC) method which has three components: base-line open loop operating point, dynamic offset part and bandwidth factor. The overall power control equation looks as follows:

Where  is the resulting UE transmit power level,  is the fractional path-loss compensation factor,  is the estimation of actual path-loss between the BS and the UE. The dynamic offset part consists of MCS (Modulation and Coding Scheme) dependent factor,  (TF stands for transmission format) and the explicit power control instruction part. The bandwidth factor is dependent on, which stands for granted bandwidth, in terms of number of granted PRBs.

Figure 11

Statistics of UL power levels used by the terminals in the indoor deployment scenario

Figure 11 shows the UL transmit power distribution for an indoor deployment scenario, which corresponds to the indoor in-building scenario in 3GPP TR 36.889‎[41]. In the indoor scenario, two operators deploy 4 small cells each equally spaced and centered in the single-floor building (50m-by-120m). The distance between two closest nodes from two operators is random. Since the LAA-LTE BS is usually close to the UE in LAA-LTE operations, the terminals transmit in UL using typically low power levels. In this case, the median UE transmit power is less than 0 dBm, for SNRtarget =12 dB.

Figure 12 shows the UE transmit power distribution for an outdoor deployment (as specified in 3GPP TR 36.889‎[41]). In the outdoor scenario, the LAA-LTE cells are in clusters uniformly distributed within the macro geographical area; 4 small cells per operator, uniformly random dropping within the cluster area. As expected, in the outdoor case power levels are higher compared to the indoor case due to higher coupling loss. UL power distribution depends on the specific power control settings, results presented in Figure 11 and Figure 12 represents a reasonable trade-off in terms of noise raise due to intra system interference and achievable mean throughput. In this case, the median UE transmit power is less than 4 dBm, for SNRtarget =9 dB.

Figure 12

Statistics of UL power levels used by the terminals in the outdoor deployment scenario

For the case of LAA-LTE BS, actual transmit power varies according to the cell load variations. Thus, LAA-LTE BS transmits on full power only when the cell is fully loaded according to current specifications (LTE Release 8 BS). It is assumed that on average BS use half of the available power, i.e. an average 50% cell load. Averaged over time BS transmits at half maximum power, i.e. 20 dBm.

Table 3 summarises the LAA-LTE power distribution for both BS and UE, taking into account following assumptions:

–	Bandwidth distribution as shown in Table 4;

–	equal transmission probability between UE and BS (50%/50% share);

–	indoor/outdoor ratio of LAA-LTE UEs assuming that 85% and 15% of the traffic is generated indoors and outdoors, respectively;

–	at least 9 UEs per LAA-LTE BS; thus, out of the LAA-LTE devices, there will be at least 10% LAA-LTE BS and the rest 90% will be LAA-LTE UEs;

–	approximately, one in every six LAA-LTE BS is placed outdoor.

Influence of Wi-Fi deployed nearby LAA-LTE on the power distribution of LAA-LTE remains for further investigation.

Table 45

LAA-LTE power distribution considering both LAA BS and UE

| Tx power e.i.r.p. | 1 W | 200 mW | 140 mW | 100 mW | 50 mW | 13 mW | <=1mW | all |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor, % | 0,00 | 9,55 | 0,96 | 20,58 | 7,96 | 21,50 | 22,95 | 83,50 |
| Outdoor, % | 0,01 | 2,10 | 0,49 | 3,92 | 1,91 | 5,28 | 2,79 | 16,50 |

### A2.1.1.3	LAA-LTE channel bandwidth characteristics

LAA-LTE can support different channel bandwidths through a carrier aggregation mechanism.  It can be observed that IEEE and 3GPP standards are converging in terms of overall spectral efficiency. This is particularly true when the evolution of the 802.11 family with 802.11ax standard is considered. Assuming that LAA-LTE and Wi-Fi will be subject to the same traffic demand by the end users, it is reasonable to expect that very similar channel bandwidth distribution will be required by the two systems.

Based on the above observations, it is expected that Wi-Fi and LAA-LTE will have same channel bandwidth distribution as described in Table 46.

Table 46

LAA-LTE channel bandwidth distribution and bandwidth correction values

| Channel bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| Device Percentage | 10 % | 25 % | 50 % | 15 % |

### A2.1.1.4	LAA-LTE antenna pattern

The antenna pattern in Figure 13 is considered as representative average antenna pattern for LAA-LTE omnidirectional antenna which can be employed for indoor and outdoor access points. In a typical deployment, LAA-LTE antenna will be mounted to the ceiling and titled down towards the ground which is beneficial in terms of coverage. Table 47 summarize the antenna characteristics for LAA-LTE omni-directional antenna.

Figure 13

LAA-LTE Omnidirectional - Elevation (left) and Azimuth (right) Radiation Patterns

Table 47

Typical Indoor and Outdoor LAA-LTE BS antenna (access point)

| # | Type | Gain (dBi) | Indoor / Outdoor | Antenna pattern | Typical Antenna Height (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Omnidirectional Antenna | 5 | Indoor & Outdoor | Figure 3 | 6 (indoor) 10 (outdoor) |

## A2.1.2	Deployment scenarios of LAA-LTE

LAA-LTE is expected to be deployed by operators, mainly in hotspots and enterprise environments. Therefore deployments of possible LAA-LTE in the residential environment (user deployed) have not been considered in the current studies. These limited assumptions will have to be considered either in an expansion of future technical studies or as a possible regulatory issue to be studied.  Operator-deployed means that the actual deployment will be “somewhat planned”, therefore LAA-LTE will only be used in places where the operator coverage of LAA-LTE is available. Notice that the main target for LAA-LTE is mainly indoor small cells deployments, but outdoor deployment is not precluded as specified in 3GPP TR 36.889 Error! Reference source not found.‎[41] which was developed by vendors and operators.

In the sections below, the potential deployment scenario of LAA-LTE is presented.

### A2.1.2.1	Indoor scenario

#### A2.1.2.1.1	Enterprise environment

The enterprise deployment is a scenario where the users are in a business environment. So, in essence, this scenario will be a large office space, sport arena, shopping malls, factories, production facilities, etc. In this scenario, usually the locations are covered by wide area LTE network; however, LAA-LTE provides a capacity booster service. For this scenario, the following needs to be considered:

–	User distribution/density: there will be dense users in these locations; however the users will be mainly active only in certain parts of the day. Also, in some locations, e.g. sports arena, large number of people will be requiring service only periodically and for a short period of time. In case of large office space or production facilities, there are possibilities that LAA-LTE BS can be LoS to each other. This is beneficial in a sense that, hidden node problem from channel access mechanism is largely removed.

–	Usage/activity information: as mentioned earlier, the user density is high at certain times of the day, while there will be no active users in some other times, since the deployment locations for enterprise case only involves activities in certain part of the day.

#### A2.1.2.1.2	Extended indoor coverage

Compared to enterprise deployment, when LAA-LTE is used as capacity booster; for extended indoor coverage, e.g. parking floors or underground places (shops, store houses, etc.), LAA-LTE will be used mainly for providing coverage. In this case, wide area LTE network is sometimes not able to cover the places, thus deployment of LAA-LTE will provide the coverage. In this case, the coverage will be provided when LAA-LTE node will be used in heterogeneous deployment manner.  The current understanding is that, extended indoor coverage may include mainly work/public areas. The residential coverage is out of question at this moment.

#### A2.1.2.2	Outdoor Scenario

#### A2.1.2.2.1	Immidiate outdoor scenario

This scenario includes parks, small open area just outside large buildings, enclosed outdoor places inside building areas, etc. This deployment is mainly for boosting the LTE capacity, not necessarily for coverage. Urban city center is one such example. There are few mobile users, however the users can be outdoors, or just inside the buildings. Usually, there are places where there are many people, looking for high data rate services.

#### A2.1.2.2.2	Large open spaces

LAA-LTE will also be deployed in large open spaces with significant number of users, e.g. train stations, or stadium, etc. In this case, the deployment conditions are very similar to the above immediate outdoor scenario; however, the user density at some point of time can be very high.

# A2.2	Compatibility between RLAN/LAA LTE and FSS (Earth to space) in the band 5 725-5 925 MHz

It should be noted that these studies are additional to those presented in ECC Report 244 and take account of both WiFI and LAA-LTE usage in the same band.

### A2.2.1	Step 1: Calculations for the maximum number of active RLAN transmitters within the FSS footprint

This section presents the calculations for the maximum number of active on-tune RLANs that can be accommodated by a victim satellite receiver considering a range of protection criteria‎[2].

The following parameters are considered in the calculations:

–	Mixed RLAN deployment between Wi-Fi and LAA-LTE:

•	For the case of Wi-Fi, power distribution and channel bandwidth distribution as provided in ECC Report 244.

•	For the case of LAA-LTE: power distribution and channel bandwidth distribution according Section 3.1.

–	Antenna discrimination towards space: calculations are performed using two values 0dB and 4 dB for both technologies, i.e. Wi-Fi and LAA-LTE.

–	LAA-LTE will be driven by operators and service provider’s deployment, addressing enterprise and hotspot scenarios. Based on JRC prediction for the number of RLAN devices in 2025, the overall percentage of RLANs deployed in an enterprise and hotspot scenario is around 10% compared to 85% of residential usage. As baseline assumption, LAA-LTE will represent 90% of the enterprise sector and 70% of the hotspots. An additional use of 5% is also envisaged (see Table 50). On this basis, Step 1 calculations are performed with a share between WiFi and LAA of 91.56 % and 8.44 %, respectively.

–	RLAN e.i.r.p. distribution and channel bandwidth distribution as provided in section 1.

–	Building (indoor to outdoor) attenuation, calculations are performed using two values: 12 dB and 17 dB.

Table 48

Initial results for Max number of RLANs for dT/T = 6%

| dT/T (%) | 6% (Table 49) |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 |
| Satellite |  |  |  |  |
| A | 235500 | 588750 | 173600 | 434000 |
| B | 2056000 | 5140000 | 1516000 | 3790000 |
| D | 235500 | 588750 | 173600 | 434000 |
| F | 2056000 | 5140000 | 1516000 | 3790000 |
| G | 365500 | 913750 | 269400 | 673500 |

Maximum number of RLANs after further modelling

Taking into account clutter loss and polarisation mismatch loss and the results given in Tale 28 the following Table 1-36 gives the maximum number of on-tune, active, RLANs for the bands 5 725-5 850 MHz.

Table 49

Maximum number of on-tune RLAN for the 5 725-5 850 MHz band for Step 1 (apportionment scheme applied)

| Band 5 725-5 850 MHz | With clutter loss and no polarisation mismatch |  |  |  | With clutter loss and polarisation mismatch |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna discrimination (dB) | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Building loss (dB) | 17 | 17 | 12 | 12 | 17 | 17 | 12 | 12 |
| A | 89344 | 223359 | 65834 | 164585 | 151884 | 379711 | 93485 | 233711 |
| B | 914537 | 2286343 | 673916 | 1684790 | 1554713 | 3886782 | 956961 | 2392402 |
| D | 89344 | 223359 | 65834 | 164585 | 151884 | 379711 | 93485 | 233711 |
| F | 1060797 | 2651993 | 781694 | 1954235 | 1803355 | 4508387 | 1110006 | 2775014 |
| G | 200339 | 500848 | 147244 | 368110 | 340577 | 851442 | 209086 | 522716 |

### A2.2.2	Step 2: RLAN deployment model

This methodology consists of 8 stages:

–	Stages 1 to 3 aim at defining the expected total number of RLAN APs over Europe.

–	Stages 4 to 7 are considering RLAN operational parameters to derive the expected total number of on-tune, active, RLANs over Europe.

Since the predictions made by JRC were technology agnostic and based on predictions on the population growth by 2025, for simplicity sake the overall population of RLAN is assumed to be the reference value in the context of this study (noting that a range is exercised), whatever the market penetration is for the different technologies, although new applications offered by LTE LAA would tend to increase the overall number of access points/Base   stations. Further studies may be required.

The overall percentage of RLAN deployed in an enterprise and hotspot scenario is around 10% compared to 85% deployed in a residential scenario. An additional use of 5% is also envisaged (see Table 50).Three possible scenarios for LAA-LTE and Wi-Fi deployment in 2025:

–	LAA-LTE will represent all of the 10% AP deployed in enterprise and hotspot scenarios.

–	LAA-LTE will be a completely new addition to the number of RLANs determined based on JRC projections.

–	A percentage of RLANs deployed in enterprise and hotspots will be replaced by LAA-LTE.

As baseline assumption, LAA-LTE will represent 90% of Wi-Fi deployed in enterprise and around 70% of the hotspot deployment; however a sensitivity analysis can be derived. Elements related to stages 1 to 3:

Stage 1: 	Define RLAN deployment environments and obtain relevant statistics

Stage 2: 	Assign the statistics obtained in Stage 1 to urban, suburban and rural environments

Stage 3: 	Apply Market penetration factors in different environments

For elements related to stages 1 to 3, refer to section 8.1.3.1 of ECC Report 244.

Results are reminded in the table below:

Table 50

Predictions of number of RLAN APs for 2025

### A2.2.3	Elements related to stages 4 to 7

Stage 4: Apply Busy Hour Factor

The busy hour factor depends on the deployment area and it is not influenced by the RLAN system characteristics. Therefore, the average value of 62.7% considered already in the ITU-R work (JTG 4-5-6-7) on RLAN at 5 GHz and, figures of 50% and 70% for the busy hour factor should also be applicable to LAA-LTE system.

Stage 5: Apply 5 GHz Spectrum Factor

In the context of this report, it is reasonable to investigate spectrum factor values similar to those used in ECC Report 244 (refer to section 8.1.3.2 of ECC Report 244). Further studies may be required.

Stage 6: Apply RF Activity Factor

Given the current LAA-LTE functionalities (listen-before-talk, discontinuous transmissions, DFS, carrier selection and TPC) which are following the regulatory requirements for RLANs operating at 5 GHz, it is expected that LAA-LTE device will have same RF activity levels as other RLANs at 5 GHz. Therefore, the agreed range of 3 to 30% for the RF activity factor should also be applicable to LAA-LTE devices.

Stage 7: Calculate the number of on-tune RLAN APs per 40 MHz

Whatever the technology is, the RLAN have access to the same channels in the 5GHz band and have the same system bandwidth available. Therefore, the number of RLAN Access points per 40 MHz as discussed in ECC Report 244 applies to LTE-LAA.

Table 51

Aggregate factors 4 to 7

|  | Stage 4 | Stage 5 | Stage 6 | Stage 7 | Aggregate |
| --- | --- | --- | --- | --- | --- |
|  | Busy hour population | 5 GHZ factor | Activity factor | 40 MHz FSS | Stage 4 to 7 |
| Case 1 | 50% | 50% | 3% | 12.9% | 0.0010 |
| Case 2 | 50% | 50% | 10% | 12.9% | 0.0032 |
| Case 3 | 50% | 50% | 30% | 12.9% | 0.0097 |
| Case 4 | 50% | 74% | 3% | 12.9% | 0.0014 |
| Case 5 | 50% | 74% | 10% | 12.9% | 0.0048 |
| Case 6 | 50% | 74% | 30% | 12.9% | 0.0143 |
| Case 7 | 50% | 97% | 3% | 12.9% | 0.0019 |
| Case 8 | 50% | 97% | 10% | 12.9% | 0.0063 |
| Case 9 | 50% | 97% | 30% | 12.9% | 0.0188 |
| Case 10 | 62.70% | 50% | 3% | 12.9% | 0.0012 |
| Case 11 | 62.70% | 50% | 10% | 12.9% | 0.0040 |
| Case 12 | 62.70% | 50% | 30% | 12.9% | 0.0121 |
| Case 13 | 62.70% | 74% | 3% | 12.9% | 0.0018 |
| Case 14 | 62.70% | 74% | 10% | 12.9% | 0.0060 |
| Case 15 | 62.70% | 74% | 30% | 12.9% | 0.0179 |
| Case 16 | 62.70% | 97% | 3% | 12.9% | 0.0024 |
| Case 17 | 62.70% | 97% | 10% | 12.9% | 0.0078 |
| Case 18 | 62.70% | 97% | 30% | 12.9% | 0.0235 |
| Case 19 | 70% | 50% | 3% | 12.9% | 0.0014 |
| Case 20 | 70% | 50% | 10% | 12.9% | 0.0045 |
| Case 21 | 70% | 50% | 30% | 12.9% | 0.0135 |
| Case 22 | 70% | 74% | 3% | 12.9% | 0.0020 |
| Case 23 | 70% | 74% | 10% | 12.9% | 0.0067 |
| Case 24 | 70% | 74% | 30% | 12.9% | 0.0200 |
| Case 25 | 70% | 97% | 3% | 12.9% | 0.0026 |
| Case 26 | 70% | 97% | 10% | 12.9% | 0.0088 |
| Case 27 | 70% | 97% | 30% | 12.9% | 0.0263 |

Table 52

Total number of on-tune RLAN in Europe for low, medium and high scenarios.

|  | Aggregate Stages 4 to 7 (see section 8.1.3.2) | Total number of on-tune RLAN in Europe (for 300 Million APs) | Total number of on-tune RLAN in Europe (for 400 Million APs) | Total number of on-tune RLAN in Europe (for 500 Million APs) |
| --- | --- | --- | --- | --- |
| Case 1 | 0.0010 | 290118 | 386824 | 483530 |
| Case 2 | 0.0032 | 967061 | 1289414 | 1611768 |
| Case 3 | 0.0097 | 2901182 | 3868243 | 4835304 |
| Case 4 | 0.0014 | 429375 | 572500 | 715625 |
| Case 5 | 0.0048 | 1431250 | 1908333 | 2385417 |
| Case 6 | 0.0143 | 4293750 | 5725000 | 7156250 |
| Case 7 | 0.0019 | 562829 | 750439 | 938049 |
| Case 8 | 0.0063 | 1876098 | 2501464 | 3126830 |
| Case 9 | 0.0188 | 5628294 | 7504392 | 9380490 |
| Case 10 | 0.0012 | 363808 | 485078 | 606347 |
| Case 11 | 0.0040 | 1212694 | 1616926 | 2021157 |
| Case 12 | 0.0121 | 3638083 | 4850777 | 6063471 |
| Case 13 | 0.0018 | 538436 | 717915 | 897394 |
| Case 14 | 0.0060 | 1794788 | 2393050 | 2991313 |
| Case 15 | 0.0179 | 5384363 | 7179150 | 8973938 |
| Case 16 | 0.0024 | 705788 | 941051 | 1176313 |
| Case 17 | 0.0078 | 2352627 | 3136836 | 3921045 |
| Case 18 | 0.0235 | 7057881 | 9410507 | 11763134 |
| Case 19 | 0.0014 | 406166 | 541554 | 676943 |
| Case 20 | 0.0045 | 1353885 | 1805180 | 2256475 |
| Case 21 | 0.0135 | 4061655 | 5415541 | 6769426 |
| Case 22 | 0.0020 | 601125 | 801500 | 1001875 |
| Case 23 | 0.0067 | 2003750 | 2671667 | 3339583 |
| Case 24 | 0.0200 | 6011250 | 8015000 | 10018750 |
| Case 25 | 0.0026 | 787961 | 1050615 | 1313269 |
| Case 26 | 0.0088 | 2626537 | 3502050 | 4377562 |
| Case 27 | 0.0263 | 7879611 | 10506149 | 13132686 |

Table 53

Scenarios for FSS summary analysis

| Scenario | Number of AP | Busy hour factor | Spectrum factor | RF activity factor | Total number of on-tune RLAN in Europe |
| --- | --- | --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 300 M | 50 % | 50 % | 3 % | 290118 |
| “Medium” scenario (Case 14 above) | 400 M | 62.7 % | 74 % | 10 % | 2393050 |
| “Pessimistic” scenario (Case 27 above) | 500 M | 70 % | 97 % | 30 % | 13132686 |

Table 54

Summary of FSS analysis

| Scenario | Antenna discr. (dB) | Building loss (dB) | Band 5 725-5 850 MHz |
| --- | --- | --- | --- |
| “Optimistic” scenario (Case 1 above) | 4 | 17 | FSS protection criteria satisfied for all FSS groups 1, 2 and 4 (margin ranges 1.2 to 11.9 dB) |
| “Medium” scenario (Case 14 above) | 4 | 17 | FSS protection criteria satisfied for FSS group 1 (margin ranges 3.3 to 4.0 dB) FSS protection criteria exceeded for other FSS groups 2 and 4 (exceeding ranges 3.2 to 6.8 dB) |
| “Pessimistic” scenario (Case 27 above) | 4 | 17 | FSS protection criteria exceeded for all FSS groups 1, 2, 3, 4 and 5 (exceeding ranges 2.4 to 13.2 dB) |

# A2.3	Conclusions

Three additional studies have been made with LAA-LTE, compared to ECC Report 244:

–	Compatibility of a mix LAA and WiFi market share with FSS. Results are roughly the same as for the case of WiFi only.

Therefore the impact of adding LAA-LTE use case in 5 GHz bands appears to have minimal effect on the overall results of compatibility and sharing as shown in ECC Report 244.

List of References

EC Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for Wireless Access Systems including Radio Local Area Networks in the bands 5 350-5 470 MHz and 5 725-5 925 MHz ('WAS/RLAN extension bands') for the provision of wireless broadband services, September 2013

ECC Decision (04)08: "Harmonised use of 5 GHz for the implementation of WAS/RLANs "

WRC Resolution 229 (WRC-03): "Use of the bands 5 150-5 250, 5 250-5 350 MHz and 5 470-5 725 MHz by the mobile service for the implementation of Wireless Access Systems including Radio Local Area Networks"

ERC Report 67: "Study of the Frequency sharing between HIPERLANs and MSS feeder links in the 5 GHz band"

ERC Report 72: " Compatibility studies related to the possible extension band for HIPERLAN at 5 GHz"

ETSI EN 301 893: " Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonized EN covering the essential requirements of Article 3.2 of the R&TTE Directive"

EC Decision 2005/513/EC complemented by EC Decision 2007/90/EC

Recommendation ITU-R M.1739: "Protection criteria for wireless access systems, including radio local area networks, operating in the mobile service in accordance with Resolution 229 (WRC-03) in the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz"

ERC Report 25: "The European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz"

EC Decision 2008/671/EC: "Commission Decision of 5 August 2008 on the harmonised use of radio spectrum in the 5 875-5 905 MHz frequency band for safety related applications of Intelligent Transport Systems (ITS)"

ECC Decision (08)01: " ECC Decision of 14 March 2008 on the harmonised use of the 5 875-5 925 MHz frequency band for Intelligent Transport Systems (ITS)"

ECC Recommendation (08)01: " Use of the band 5 855-5 875 MHz for Intelligent Transport Systems"

ECC Report 101: "Compatibility studies in the band 5 855-5 925 MHz between Intelligent Transport Systems (ITS) and other systems"

ECC Decision (12)04: " ECC Decision of 2 November 2012 on the withdrawal of ECC Decision (02)01. For national implementation issues related to the withdrawn Decision, check EFIS"

ERC Recommendation 70-03: " Relating to the use of Short Range Devices (SRD)"

ETSI EN 300 674: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band; Part 1: General characteristics and test methods for Road Side Units (RSU) and On-Board Units (OBU)"

CENELEC EN 12253: "" Road transport and traffic telematics - Dedicated short-range communication - Physical layer using microwave at 5.8 GHz"

ECC Report 68: "Compatibility studies in the band 5 725-5 875 MHz between Fixed Wireless Access (FWA) systems and other systems"

ITU-R Recommendation P.452: "Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz"

EC Decision 2006/771/EC: "Commission Decision of 9 November 2006 on the harmonisation of the radio spectrum for use by short-range devices"

IARU Region-1 VHF Managers Handbook

Recommendation ITU-R M.1732-1: "Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies"

ECC Report 210: "Compatibility/sharing studies related to Broadband Direct-Air-to-Ground Communications (DA2GC) in the frequency bands 5 855-5 875 MHz, 2 400-2 483.5 MHz and 3 400-3 600 MHz"

ETSI TR 101 599: "Electromagnetic compatibility and Radio spectrum matters (ERM) System Reference Document (SRDoc); Broadband Direct-Air-to-Ground Communications System employing beamforming antennas, operating in the 2,4 GHz and 5,8 GHz bands"

ETSI TR 103 108: "Electromagnetic compatibility and Radio spectrum Matters (ERM); System Reference document (SRdoc); Broadband Direct-Air-to-Ground Communications System operating in the 5,855 GHz to 5,875 GHz band using 3G technology"

ECC Report 206: "Compatibility studies in the band 5 725-5 875 MHz between SRD equipment for wireless industrial applications and other systems"

IEEE 802.11-2012: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

ETSI EN 302 571 V1.2.2 (2011-10): "Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive"

Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain"

ERC Recommendation 74-01: "Unwanted Emissions in the Spurious Domain"

Recommendation ITU-R F.1336-3: "Reference radiation patterns of omnidirectional, sectoral and other antennas for the fixed and mobile service for use in sharing studies in the frequency range from 400 MHz to about 70 GHz"

ETSI ES 200 674-1 V2.4.1 (2013-05): “Technical characteristics and test methods for High Data Rate (HDR) data transmission equipment operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band”

NTIA 5 GHz report, January 2013, “Evaluation of the 5 350-5 470 MHz and 5 850-5 925 MHz bands pursuant to section 6406(b) of the middle class tax relief and job creation act of 2012

ETSI TR 102 960: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (RTTT DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range; Evaluation of mitigation methods and techniques"

ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range"

http://www.statistiques-mondiales.com/union_europeenne.htm

http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=lfst_hhnhtych&lang=en

https://www.ssb.no/en/befolkning/statistikker/familie/aar/2014-12-12

http://www.bfs.admin.ch/

http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=bd_9ac_l_form_r2&lang=en

3GPP TR 36.889: Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum, http://www.3gpp.org/DynaReport/36889.htm

CEPT Report 57 : "Compatibility and sharing conditions for WAS/RLAN in the bands 5 350-5 470 MHz and 5 725-5 925 MHz"

ECC Report 181 on improving spectrum efficiency in SRD bands

JRC study on RLAN deployment and device densities:  http://www.cept.org/Documents/se-24/25709/SE24(15)070R0_WI52_number-of-RLAN-JRC

JRC study on in-home RLAN coverage and number of RLAN APs per household: http://www.cept.org/Documents/se-24/25805/SE24(15)089R0_WI52_Further-considerations-on-WLAN-indoor-coverage

JRC measurements of 802.11ac RLAN APs operating in the 5 GHz band: http://www.cept.org/Documents/se-24/24051/SE24(15)042R0_WI52_JRC_5GHZ_RLAN-_measurements

ECC Report 244 “Compatibility studies related to RLANs in the 5 725-5 925 MHz band”

____________________

The protection criteria for ground-based meteorological radars is found in Recommendation ITU-R M.1849.

Figure 4.35, "Communications Market Report 2015", Ofcom, 04 August 2016, https://www.ofcom.org.uk/__data/assets/pdf_file/0024/26826/cmr_uk_2016.pdf.

This system is a two-satellites constellation.

Lower gain can be used for the wider beams.

Antenna beam “incident angles”.

Average e.i.r.p. over a pulse repetition interval.

Max e.i.r.p. during pulse transmission.

Antenna gain varies depending on antenna location (mid or side), and incident angle.

2009 status, based on market reports from IMS Research, ABI Research, iSupply, and Plum Consulting.

As per Recommendation ITU-R M.1739, the I/N ratio at the WAS/RLAN receiver should not exceed –6 dB, assuring that degradation to a WAS/RLAN receiver’s sensitivity will not exceed approximately 1.0 dB.  Whilst it is designed to address interference from multiple sources, this criterion is also considered in this Report for single-entry analysis