| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/156(Rev.1) Subject: WRC-19 agenda item 1.15 | Annex 31 to Document 5A/469-E |
|  | 12 June 2017 |
|  | English only |
| Annex 31 to Working Party 5A Chairman’s Report |  |
| PRELIMINARY DRAFT NEW REPORT ITU-R M.[300GHZ_MS_CHAR] |  |
| Technical and operational characteristics of the land mobile service applications operating in the frequency range 275-450 GHz |  |

# 1	Introduction

Due to the progress of RF integrated devices and circuits operating in the frequency band above 275 GHz, the contiguous frequency bands become available for land mobile service applications. Some unique applications operating in the frequency band above 275 GHz such as KIOSK downloading, ticket gate downloading and intra-rack and intra-chip communications are introduced and the ultra-high-speed data transmission between terminals whose transmission distance is in the order of centimeters becomes feasible.

RR No. 5.565 identifies the specific frequency bands for the radio astronomy service, the earth exploration satellite service (passive), and the space research service (passive) in the frequency range of 275-1 000 GHz. Although the use of the frequency range 275-1 000 GHz by the passive services does not preclude the use of this range by active services, administrations wishing to make frequencies in the 275-1 000 GHz range available for active service applications are urged to take all practicable steps to protect these passive services from harmful interference.

# 2	Scope

This Report provides the land mobile service applications and their technical and operational characteristics operating in the frequency range 275-450 GHz for sharing and compatibility studies between land mobile service applications and passive services, as well as among active services in the frequency range 275-450 GHz.

# 3	Related Recommendation and Report

| Report ITU-R SM.2352-0: | Technology trends of active services in the frequency range 275-3 000 GHz |
| --- | --- |
| Recommendation ITU-R M.2003: | Multiple Gigabit Wireless Systems in frequencies around 60 GHz |
| Recommendation ITU-R P.676: | Attenuation by atmospheric gases |
| Recommendation ITU-R P.838: | Specific attenuation model for rain for use in prediction methods |
| Recommendation ITU-R P.840: | Attenuation due to clouds and fog |

# 4	List of acronyms and abbreviations

| CPMS | Close proximity mobile system |
| --- | --- |
| THF | Tremendously high frequency |

# 5	Frequency ranges of agenda item 1.15

As the unit of frequency is Hertz (Hz), frequencies shall be expressed in Gigahertz (GHz), above 3 GHz, up to and including 3 000 GHz in accordance with Radio Regulations. However,
the Gigahertz frequency ranges are subdivided into three ranges as shown in Table 1. Because the frequency range of WRC-19 agenda item 1.15 is 275-450 GHz, two frequency bands i.e. EHF and THF must be included in the study of agenda item 1.15.

TABLE 1

Frequency bands above 3 GHz

| Band number | Frequency range (lower limit exclusive, upper limit inclusive) | Corresponding metric subdivision |
| --- | --- | --- |
| 10 | 3 to 30 GHz | Centimetric waves |
| 11 | 30 to 300 GHz | Millimetric waves |
| 12 | 300 to 3000 GHz | Decimillimetric waves |

# 6	Overview of mobile service applications operating in the frequency range 275-450 GHz

## 6.1	Close proximity mobile systems operating in the frequency band on 275–325 GHz and 275–450 GHz

### 6.1.1	KIOSK downloading mobile system

In order to enjoy movies, news, magazines and music by smart phones and tablet terminals, the terminals should have high-data-speed transmission capability and be wirelessly connected to the network to download various contents from the content providers.

Several wireless devices provide wireless broadband connectivity, but the maximum speed of these devices is limited by operational and environmental conditions of the systems and the actual observed transmission rate is sometimes far from the specifications. Kiosk systems, as shown in Figure 1, are introduced to download the heavy contents to the user terminals wirelessly.

Kiosk terminals are connected to the network through wired systems and located in public areas such as train stations, airports, shopping malls and etc. The distance between the user and the Kiosk terminal is typically less than 10 cm and contents are downloaded and/or uploaded to/from user terminals. In order to download two-hour movie whose size is about 900 MB to the user terminal, the required downloading times are 1.6 sec, 1.1 sec and 0.11 sec if effective throughput of devices are 4.6 Gbit/s, 6.9 Gbit/s and 66 Gbit/s, respectively. The data transfer speed in the range
10-100 Gbit/s is achieved applying multi-modulation method and carrier frequencies above millimetre waves. If the large contiguous bandwidth is feasible in the frequency band above 275 GHz, a simple modulation scheme such as BPSK, QPSK can be applied to transmit heavy contents in a short time period.

FIGURE 1

KIOSK downloading mobile system

### 6.1.2	Ticket gate downloading mobile system

The ticket gate downloading devices have two functions, i.e. fare-paying and large-file downloading functions. Figure 2 illustrates the user terminal pays fare and simultaneously downloads video contents such as news, movies, etc. In order to download the contents at the ticket gate, high-speed data transmission capability is required for both mobile terminal and ticket gate station. The transmission range covered by these devices is limited to about 10 cm to avoid interferences between mobile terminals. To meet these requirements, the spectrum above 275 GHz whose features are a broadband bandwidth and short transmission distance can be utilized to this type of application.

FIGURE 2

Ticket gate downloading mobile system

### 6.1.3	Inter-chip communication system

There has been increasing interest in applying wireless links for data centres to replace optical wired connections, because the current device technologies can make it possible to reduce the size of racks of servers/routers in data centre. Figure 3 shows how these devices can be integrated into the compact rack of servers/routers. If the same cabling connections are used in the compact rack, cabling and cooling problems in the rack cannot be avoided. The inter-chip communication between boards in the rack can eliminate cabling and cooling problems in the rack. The frequency band above 275 GHz is suitable for inter-chip communication because the antenna diameter is inversely proportional to the operational frequency.

FIGURE 3

Inter-chip communication system

## 6.2	Intra-device communications

In intra-device communications, one or more communication links are operated within a device. High speed terahertz wireless links could connect two or more PCBs or even chips on the same PCB inside a device. Typically, these devices will be shielded not only preventing emission of THz-radiation but also blocking incoming THz signals.

The terahertz band is huge hence several channels could be used in a small area (within one device). The following figure illustrates the envisaged concept of THz point-to-point communications between boards, where the colour of the beams indicates different frequencies.

FIGURE 4

Intra-devise communication system

## 6.3	Wireless links for data centers

The goal of the introduction of wireless data links in addition to the existing fibres is to provide flexibility by providing reconfigurable routes within a data centre. In the figure some examples are illustrated between or inside the server racks (green colour boxes) for possible line-of-sight (LOS) or multi hop links.

FIGURE 5

Wireless links for data centre

# 7	System characteristics

## 7.1	Close proximity mobile systems (CPMSs) operating in the frequency range
275-450 GHz

The expected range of technical and operational characteristics for close proximity mobile systems planned to operate in the band 275-325 GHz and in the band 275-450 GHz are shown in Table 2.

Two possible radio-frequency channel arrangements for land mobile service applications operating in the frequency range 275-450 GHz are shown in Annex 1

The relationship between transmission data rate and total antenna gain of transmitter and receiver under the spectrum efficiency of 1b/s/Hz is shown in Annex 5 to clearly indicate that the multilevel modulation is indispensable to transmit high-speed data to CPMS devices.

TABLE 2

Expected technical and operational characteristics of a land mobile CPMS applications 
in the frequency range 275-450 GHz

| Parameters | Values |  |
| --- | --- | --- |
|  | CPMS application | Enhanced CPMS application |
| Frequency band (GHz) | 275-325 | 275-450 |
| Deployment density1 | 0.6 devices/km2 | 0.6 devices/km2 |
| Tx output power density (dBm/GHz) | -3.8….6.9 | -10.1…6.7 |
| Max. e.i.r.p. density(dBm/GHz) | 26.2…...36.9 | 19.9…36.7 |
| Duplex Method | FDD/TDD | FDD/TDD |
| Modulation | OOK/BPSK/QPSK/16QAM/64QAM BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM | OOK/BPSK/QPSK/16QAM/64QAM/8PSK/8APSK BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM |
| Average distance between CPMS fixed and mobile devices (m) | 0.1 | 0.1 |
| Maximum distance between CPMS fixed and mobile devices (m) | 1 | 1 |
| Antenna height (m) | 1…2 | - |
| Antenna beamwidth (degree) | 3…10 | 5…90 |
| Antenna elevation (degree) | ±90 | ±90 |
| Frequency reuse | 1 | 1 |
| Antenna type | Horn | Horn |
| Antenna pattern | Gaussian | Gaussian |
| Antenna polarization | Linear | Linear |
| Indoor CPMS fixed device deployment (%) | 100 | 90 |
| Feeder loss (dB) | 2 | 2 |
| Maximum CPMS fixed/mobile device output power (dBm) | 10 | 10 |
| Channel bandwidth (GHz) | 2.16/4.32/8.64/12.96/17.28/ 25.92/51.8 | 2.16/4.32/8.64/12.96/17.28/25.92/51.84/69.12/103.68 |
| Transmitter spectrum mask | see Annex 4 | see Annex 4 |
| Maximum CPMS fixed device antenna gain (dBi) | 30 | 30 |
| Maximum CPMS mobile device antenna gain (dBi) | 15 | 15 |
| Maximum CPMS fixed device output power (e.i.r.p.) (dBm) | 40 | 40 |
| Maximum CPMS mobile device output power (e.i.r.p.) (dBm) | 25 | 25 |
| Average activity factor (%) | 0.76 | 0.2 |
| Average CPMS fixed device power (dBm (e.i.r.p)) | 20 | 20 |
| Receiver noise figure typical (dB) | 15 | 15 |
| 1 Detailed information of deployment density is shown in Annex 3. |  |  |

## 7.2	Intra-device communications

The expected ranges of technical and operational characteristics for wireless THz intra-device links planned to operate in the band 275-450 GHz are shown in Table 3. As an example inside a camera the data rate between the optical sensor and the image processor is 72 Gbit/s for an 8K video with a frame rate of 60 Hz and a resolution of 12 bit for each color [4]. Therefore, a bandwidth of 50 GHz is sufficient to provide such data rate with a simple QPSK modulation. Annex 4 proposes a Transmitter Spectrum Mask.

TABLE 3

Expected technical and operational characteristics of wireless THz intra-device links operating 
in the frequency band 275-450 GHz

| Parameter | Value |
| --- | --- |
| Frequency band (GHz) | 275-450 |
| Deployment density | 0.231 /km2 |
| Maximum device output power (dBm) | 10 |
| Maximum device output power (e.i.r.p.) (dBm) | 30 |
| Maximum Tx output power density (dBm/GHz) | -10.1…6.7 |
| Maximum e.i.r.p. density (dBm/GHz) | 19.9…36.7 |
| Indoor Deployment (%) | 50 |
| Duplex Method | TDD, FDD, SDD |
| Modulation | OOK/BPSK/QPSK/16QAM/64QAM 8PSK/8APSK |
| Maximum distance between devices | <1 m |
| Antenna height (m) | 1…3 |
| Antenna beamwidth (degree) | 15…180 (expected) |
| Frequency reuse | 1 |
| Antenna pattern | Gaussian |
| Antenna polarization | Linear |
| Channel bandwidth (GHz) | 2.16/4.32/8.64/12.96/17.28//25.92/51.84/69.12/103.68 |
| Maximum device antenna gain (dBi) | 20 |
| Typical expected device antenna gain (dBi) | 6 |
| Maximum device activity (%) | 100 |
| Receiver noise figure typical (dB) | 102 |
| 1 The deployment density is estimated as an average based on assuming that every one thousandths citizen in Germany is using such a device. In highly populated cities the density could increase to e.g. 3.95/km2 under the same assumptions. 2 Also systems with a noise figure as low as 8 dB have been reported in publications. This value is a worst case of the published parameters. |  |

## 7.3	Wireless links in data centers

The expected ranges of technical and operational characteristics for wireless links in data centers planned to operate in the band 275-450 GHz are shown in Table 4. A bandwidth of 50 GHz is necessary to achieve a data rate of at least 100 Gbit/s with a simple QPSK modulation and enable compatibility with 100 Gbit/s Ethernet links. Annex 4 proposes a Transmitter Spectrum Mask.

TABLE 4

Expected technical and operational characteristics of wireless links in data centers operating 
in the frequency band 275-450 GHz

| Parameter | Values |
| --- | --- |
| Frequency band (GHz) | 275-450 |
| Deployment density | 0.071 /km2 |
| Maximum device output power (dBm) | 10 |
| Maximum device output power (e.i.r.p.) (dBm) | 40 |
| Tx output power density (dBm/GHz) | -10.1…6.7 |
| e.i.r.p. density (dBm/GHz) | 9.9…26.7 |
| Duplex Method | TDD, FDD, SDD |
| Modulation | OOK/BPSK/QPSK/16QAM/64QAM 8PSK/8APSK |
| Maximum distance between devices | 100 m |
| Antenna beamwidth (degree) | < 25 (expected) |
| Frequency reuse | 1 |
| Antenna pattern | Gaussian |
| Antenna polarization | Linear |
| Indoor deployment (%) | 100 |
| Channel bandwidth (GHz) | 2.16/4.32/8.64/12.96/17.28/ 25.92/51.84/69.12/103.68 |
| Maximum device antenna gain (dBi) | 30 |
| Maximum device activity (%) | 100 |
| Receiver noise figure typical (dB) | 102 |
| 1Based on an evaluation in Germany[3] there are 2170 data centres with more than 100 servers. Assuming that in each 10 links are deployed and taking the area of Germany into account a density of approx. 0.07 links per km2 2 Also systems with a noise figure as low as 8 dB have been reported in publications. This value is a worst case of the published parameters. |  |

# 8	Summary of possible candidate frequency bands for land mobile service applications under WRC-19 agenda item 1.15

Table 5 summarizes the candidate frequency bands for land mobile service applications under 
WRC-19 agenda item 1.15. A total spectrum bandwidth of 50 GHz is sufficient to provide high-data rate transmissions between CPMS devices for KIOSK applications, as well as, intra device applications and wireless links for data centers.

TABLE 5

Possible candidate frequency bands for land mobile service applications

| Applications | Candidate frequency band | Spectrum needs |
| --- | --- | --- |
| CPMS application for KIOSK downloading | 275-325 GHz | 50 GHz (see Annex 5) |
| CPMS application | 275-450 GHz | 50 GHz |
| Intra-device communication | 275-450 GHz | 50 GHz |
| Wireless links for data centers | 275-450 GHz | 50 GHz |

# 9	Bibliographies

[1] 	APT/AWG/REP-66, APT Report on "Short Range Radiocommunication Systems and Application Scenarios Operating in the Frequency Range 275-1 000 GHz".

[2] 	IEEE802.15-15-0109-06-003e, “TG3e Technical Guidance Document (TGD)”.

[3] 	Study of the existing data centres in Germany by the German Ministry for the Environment, Nature Conservation, Building and Nuclear Safety, http://www.umweltbundesamt.de/sites/default/files/medien/461/publikationen/4037.pdf.

[4]	IEEE802.15-14-0304-16-003d, “TG3d Applications Requirements Document　(ARD)”.

Annexes: 	5

Annex 1

Examples of radio-frequency channel arrangement

In Figure A1-1 and A1-2, two examples of channel arrangements are illustrated. The basic channel bandwidth which is widely used for Radio LAN is 2.16 GHz, and the other channels bandwidths are 4.32 GHz, 8.64 GHz, 12.96 GHz, 17.28 GHz, 25.92 and 51.8 GHz. The extra channels are embedded as additional channels in the remaining frequency band. In FIGURE A1-2 the 
Radio-frequency channel arrangement used in the current draft standard IEEE 802.15.3d is provided as an example. The frequency range considered by IEEE begins at approximately 252 GHz though WRC-19 agenda item 1.15 covers only frequencies above 275 GHz.

FIGURE A1-1

Radio-frequency channel arrangement for CPMS application

FIGURE A1-2

Radio-frequency channel arrangement example for CPMS, intra device communications and wireless links in data centres, which is currently used in the draft standard IEEE 802.15.3d.

ANNEX 2

Measurement results of antenna pattern at 300 GHz

FIGURE A2-1

Measured characteristics of 30-dBi and 15-dBi antennas

- 30-dBi Horn antenna			(b) 15-dBi CPMS antenna

ANNEX 3

Deployment density and activity factor of KIOSK downloading systems

KIOSK downloading system will be used in the stations, airport terminals, convenience stores. It is mainly deployed indoor environment. Since the number of stations and airports is much smaller than that of convenience stores, this report summarizes deployment densities of KIOSK terminals equipped at convenience store. The total number of convenience store in Japan is 55,129, but 19,571 convenience stores, i.e. 35% of all stores are distributed in Kanto area whose size is 
32,420 km2. This concludes that deployment density in Kanto is 0.6 stores/km2 and that in Tokyo 3.28 stores/km2 which is the maximum density in Japan.

The average number of customers of major convenience stores in Japan is about 1,000, but the busiest store which is located nearby stations in Tokyo has the peak number of customers nearly 2,000. The following assumption is introduced for estimation of activity factor:

- Average number of customers of convenience store	1,000

- Percentage of customers bringing CPMS devices	20%

- Downloaded 2-hour movies by one customer		2

- CPMS device throughput					6.9 Gb/s (see Table 4)

- Intrinsic time of downloading by one customer		2.2 sec.

- Total time of downloading				440 sec.

- Typical opening hour of convenience store		7 am-11 pm (57,600 sec.)

- Estimated activity factor/store				0.76 %

TABLE A3-1

Numbers of convenience stores and stations in Kanto area

| Metropolitan and Prefecture | Number of convenience store | Size (km2) |
| --- | --- | --- |
| Tokyo | 7,183 | 2,190 |
| Kanagawa | 3,765 | 2,415 |
| Saitama | 2,833 | 3,797 |
| Chiba | 2,637 | 5,157 |
| Ibaraki | 1,315 | 6,096 |
| Gunma | 950 | 6,362 |
| Tochigi | 888 | 6,408 |
| Kanto area1 | 19,571 | 32,425 |
| 1 Kanto is the regional name of Tokyo metropolitan plus the above 6 prefectures. |  |  |

TABLE A3-2

Estimated downloading time of magazine and movie

| Content type | File size (MB) | Download time (sec) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Throughput 4.6 Gb/s | Throughput 6.9 Gb/s | Throughput 66 Gb/s* |
| Magazine | 300 | 0.5 | 0.3 | 0.03 |
| Movie (2hour) H.265 (Hi-definition) | 900 | 1.6 | 1.1 | 0.11 |

Annex 4

Proposal for a Transmitter Spectrum Mask

For the transmitter spectrum mask it is proposed at assume the mask currently proposed in IEEE P802.15.3d as shown in Figure A4-1 and Table A4-1.

FIGURE A4-1

Generic transmit spectral mask

PSD [dBr]

0

-20

-25

-f4

-f3

-f2

-f1

f1

f2

f3

f4

f-fc [GHz]

0

PSD [dBr]

0

-20

-25

-f4

-f3

-f2

-f1

f1

f2

f3

f4

f-fc [GHz]

0

The parameters of the PSD indicated in Figure A4-1 are defined in Table A4-2.

Table A4-1

Transmit spectrum mask parameters

| Channel Bandwidth [GHz] | f1[GHz] | f2[GHz] | f3[GHz] | f4[GHz] |
| --- | --- | --- | --- | --- |
| 2.160 | 0.94 | 1.10 | 1.60 | 2.20 |
| 4.320 | 2.02 | 2.18 | 2.68 | 3.28 |
| 8.640 | 4.18 | 4.34 | 4.84 | 5.44 |
| 12.960 | 6.34 | 6.50 | 7.00 | 7.60 |
| 17.280 | 8.50 | 8.66 | 9.16 | 9.76 |
| 25.920 | 12.82 | 12.98 | 13.48 | 14.08 |
| 51.840 | 25.78 | 25.94 | 26.44 | 27.04 |
| 69.120 | 34.42 | 34.58 | 35.08 | 35.68 |

Annex 5

Information on link budget of KIOSK downloading system

One example of the link budget is shown in Figure A5-1. The transmitting power, carrier frequency and transmission distance are 10 dBm, 300 GHz and 1 m, respectively, as indicated in Table 2. 
The total antenna gain of transmitter and receiver over 45 dBi is required to attain a data rate of 50 Gbit/s by ASK with FEC if BER is less 10-9. Since the spectrum efficiency is 1 b/s/Hz in this case, a bandwidth of 50 GHz is needed to attain 50 Gbit/s. If the maximum bandwidth of 50 GHz is identified for such applications as discussed in section 7.1, the multilevel modulation scheme such as QPSK and 16QAM whose spectrum efficiency over 2 bps/Hz is preferable to increase the transmission data rate.

FIGURE A5-1

Relationship between data rate and total antenna gain under the condition of spectrum efficiency of 1 bit/s/Hz

____________________