| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/250(Rev.1) | Annex 17 to Document 5A/650-E |
|  | 16 November 2017 |
|  | English only |
| Annex 17 to Working Party 5A Chairman’s Report |  |
| WORKING DOCUMENT TOWARD A PRELIMINARY DRAFT NEW REPORT ITU-R M.[RSTT.USAGE] |  |
| Current and future usage of railway radiocommunication systems between train and trackside (RSTT) |  |

# TABLE OF CONTENT

TABLE OF CONTENT	1

1	Scope	3

2	Background	3

3	Related documents	4

4	List of acronyms and abbreviations	4

5	Technical and operational characteristics of currently used RSTT	5

5.1	Radiocommunication systems used for Train radio	7

5.1.1	Analogue Radio	7

5.1.2	Digital Radio	17

5.1.3	Trunked Radio	28

5.1.4	GSM-R	31

5.1.5	LTE based RSTT	36

5.1.6	Emergency System	37

5.1.7	Possible other train radios	40

5.2	Radiocommunication systems used for Train positioning information	52

5.3	Radiocommunication systems used for Train remote systems	59

5.3.1	Shunting System	59

5.4	Radiocommunication systems used for Train surveillance	63

5.4.1	Surveillance System	63

6	Considerations on evolving technologies for RSTT including technical and operational characteristics of future RSTT	65

6.1	Considerations on evolving technologies for RSTT	65

6.2	Radiocommunication systems to be used for Train surveillance	69

7	Spectrum Usage of RSTT	69

7.1	Current spectrum usage of RSTT	69

7.1.1	Radiocommunication systems used for Train radio	69

7.1.2	Radiocommunication systems used for Train positioning information	73

7.1.3	Radiocommunication systems used for Train remote systems	73

7.1.4	Radiocommunication systems used for Train surveillance	74

7.2	Planning spectrum usage of RSTT	74

7.3	Spectrum needs of RSTT	74

7.3.1	Spectrum needs of RSTT with respect to the train radio application	74

[7.3.2	Example of spectrum needs of future 90-GHz RSTT with respect to the train surveillance	75

8	Differing Deployment and Operations Approaches	75

9	Summary of the study	76

A1.1	Introduction	77

A1.2	Summary of the situation of feedback of the questionnaire	77

A1.3	Summary table of answers to each question	78

A1.4	Original questionnaire answers	78

A2.1	Overview	79

A2.1.1	List of the RSTT	79

A2.1.2	Frequency usage	80

A2.1.3	Operational environment	81

A2.2	150 MHz band RSTT	83

A2.2.1	Train Radio System	83

4.2.2	Radiocommunication system for Emergency Cut-Off System	85

A2.2.2.2	Technical parameters	86

A2.2.3	Yard Radio	86

A2.3	300 MHz band RSTT	87

A2.3.1	Train Radio System	87

A2.3.2	Emergency Alarm Radio System	88

A2.3.3	Radiocommunication system for Emergency Cut-Off System	89

A2.3.4	Radiocommunication system for Electronic Blocking System	90

A2.3.5	JRTC Radio	91

A2.3.6	Yard Radio	93

A2.4	400 MHz band RSTT	93

A2.4.1	Train Radio System	93

A2.4.2	Radiocommunication system for High Speed Trains	94

A2.4.3	Yard Radio	96

A2.5	RSTT in the 40 GHz band	96

A2.5.1	40-GHz band video transmission system (MVT)	96

A2.5.2	Train Radio System in the 40 GHz band (TRS-40GHz)	98

A2.6	90-GHz -band RSTT	100

A2.6.1	Network architecture of 90-GHz -band RSTT	100

A2.6.2	Deployment scenario	101

A2.6.3	90-GHz band -band transceivers characteristics	102

A2.6.4	90-GHz band propagation characteristics	104

A2.7	60 GHz RSTT	106

A2.7.1	60-GHz band train platform monitoring system	106

A2.7.2	60-GHz band RSTT	106

A3.1	Overview	107

A3.2	450 MHz -band RSTT	107

A3.3	900 MHz -band RSTT	108

A4.1	Overview	109

A4.2	Train radio systems for train operation and railway traffic control in the HF band	109

A4.2.1	Architecture of train radio system in HF-band	109

A4.2.2	Typical technical characteristics of HF train radio system radio stations	109

A4.3	Train radio systems for train operation and railway traffic control in the VHF band	109

A4.3.1	Architecture of train radio system in VHF-band	109

A4.3.2	Typical technical characteristics of VHF train radio system radio stations	109

A4.4	Train radio systems for train operation and railway traffic control in the UHF band	111

A4.4.1	Architecture of train radio system in UHF-band	111

A4.4.2	Typical technical characteristics of UHF train radio system radio stations	111

A4.5	Train radio systems for train operation and railway traffic control in the SHF band	113

A4.5.1	Architecture of train radio system in SHF-band	113

A4.5.2	Typical technical characteristics of SHF train radio system radio stations	113

A5.1	Overview	114

A5.2	700 MHz band RSTT	114

A5.2.1	Architecture of LTE-R	115

A5.2.2	Technical characteristics	116

A5.2.3	Operational characteristics	116

A5.3	150 MHz band RSTT	117

A5.3.1	Technical characteristics	117

A5.3.2	Operational characteristics	118

A5.4	800 MHz band RSTT	118

A5.4.1	Technical characteristics	119

A5.4.2	Operational characteristics	119

A5.5	400 MHz band RSTT	119

A5.5.1	Technical characteristics	120

A5.6	18 GHz band RSTT	120

A5.6.1	Technical characteristics	120

A6.1	GSM-R train radio	121

A6.1.1	Overview	121

A6.1.2	Available implementation information	121

A6.2	Train positioning information	123

A7.1	Introduction	124

A7.2	The development of RSTT in China	124

A7.3	Study on the Spectrum needs of RSTT	125

A7.3.1	Methodology overview	125

A7.3.2	Assumptions for the study	126

A7.3.3	Calculation flow	126

A7.3.4	Input parameters	126

A7.3.5	Result of the study	130

A7.4	Conclusion	130

A8.1	Introduction	131

A8.2	Evaluation of the Doppler shift for different frequency bands and different train speeds	131

A8.3	Conclusions	135

# 1	Scope

This report addresses the technical and operational characteristics and the spectrum usage of current and planned RSTT as well as the studies on spectrum needs of RSTT, based on the responses to the questionnaire and contributions. Detailed information on RSTT in some administrations is also provided in the Annexes.

# 2	Background

Resolution 236 (WRC-15) recognized that timely studies are required on technologies providing for railway radiocommunication and that international standards and harmonized spectrum would facilitate worldwide deployment of radiocommunication systems between train and trackside. Further, Resolution 236 (WRC-15) invited ITU-R to study the spectrum needs, technical and operational characteristics and implementation of railway radiocommunication systems between train and trackside. It should be noted that, as emphasized in Resolution 236 (WRC-15), the provisions of Nos. 1.59 and 4.10 of the RR do not apply for railway radiocommunication systems.

As the responsible group for the preparation for the agenda item 1.11, ITU-R Working Party 5A realized that it is necessary to summarize the technical and operational characteristics for railway radiocommunication systems between train and trackside for traffic control, passenger safety, and security for train operation. On 3rd June 2016, The Radiocommunication Bureau invited Administrations of Member States of the ITU to provide answers to questions by Administrative Circular 5/LCCE/60.

Based on the answers and contributions on the usage of RSTT, Working Party 5A initiated development of this ITU-R Report, summarizing information on the usage of such systems for facilitating the preparation for WRC-19 agenda item 1.11.

Editor’s Note: Since the questionnaire issue with Administrative Circular 5/LCCE/60 has been concluded, Administrations are encouraged to contribute their information on RSTT directly towards Chapter 5 and Chapter 6 in the coming meetings.

# 3	Related documents

ITU-R Recommendations

Recommendation ITU-R M. [RSTT_FRQ]

Recommendation ITU-R M.1768

ITU-R Report

Report ITU-R M. [RSTT. DESCRIPTION]

# 4	List of acronyms and abbreviations

BS		Base Station

CBTC		Communications Based Train Control

CCTV		Closed-Circuit Television

CTC		Centralized Train Control Centre

CU		Central Unit

CS		Control Station

DPAR		Dispositif Annonce Par Radio (French: Train Announcing Radio System for Maintenance staff on tracksides)

EARS		Emergency Alarm Radio System

EESS		Earth Exploration-Satellite Service

ERTMS		European Railway Traffic Management System

ETSI		European Telecommunications Standards Institute

ETCS		European Train Control System

GSM-R		GSM for Railways

JRTC		Japan Radio Train Control

KVB		Contrôle Voie Balise (French Automatic Train Protection system on conventional lines)

LTE		Long Term Evolution

LCX		Leaky Coaxial Cable

MS		Mobile Station

MVT		Millimetre wave Video Transmission system

NBS		Node Base Station

OCS		Optical Carrier Station

ODC		Operation Direction Center

OFDM		Orthogonal Frequency Division Multiplexing

OFDMA		Orthogonal Frequency Division Multiple Access

QPSK		Quadrature Phase Shift Keying

RAS		Radio Astronomy Service

RBC		Radio Block Centre

REBS		Radiocommunication system for Electronic Blocking System

REMCOS		Radiocommunication system for Emergency Cut Off System

RHST		Radiocommunication system for High Speed Train

RSTT		Railway radiocommunication Systems between Train and Trackside

TDMA		Time Division Multiple Access

TETRA		Terrestrial trunk Radio

TS‑RAUs		Track-Side Radio Access Units

TRS		Trunked Radio System

UIC		Union Internationale des Chemins de fer-(International Union of Railways)

UE		User Equipment

UT		User Terminal

WiFi		Wireless Fidelity

# 5	Technical and operational characteristics of currently used RSTT

[Editor’s note: All Square Brackets in the parameter tables are removed at November 2017 meeting. Administrations are still encouraged to check the parameters in case of errors.]

This chapter presents the technical and operational parameters/characteristics of current RSTT systems provided by Administrations. According to the categorization of four main applications of RSTT, those systems are also categorized to each application accordingly.

In this chapter technical and operational characteristics of currently used RSTT are provided based on information provided by the following administrations [and/or regions.]

37 Administrations:

Angola

Armenia

Australia

Bosnia-Herzegovina

Botswana

Canada

China

Czech Republic

Finland

France

Germany

Hungary

Iraq

Italy

Japan

Republic of Korea

Malawi

Malta

Mozambique

Namibia

Netherlands

Norway

Qatar

Russian Federation

South Africa

Spain

Swaziland,

Sweden

Switzerland

Tanzania

Thailand

United Arab Emirates

United Kingdom

Unites States

Uzbekistan

Viet Nam

Zambia

[1 regional Organization: CEPT]:

[CEPT]

## 5.1	Radiocommunication systems used for Train radio

Tables 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6 and 5.1.7 contains parameters/characteristics of radiocommunication systems relating to Analogue radio systems, Digital radio systems, Trunked radio systems, GSM-R systems, LTE-based systems , Emergency systems and possible other systems used for Train Radio respectively. A brief introduction for each of those technologies has been presented in the Report ITU-R M.[RSTT, DESCRIPTION].The tables also show which applications of train radio are realized by each system and in which deployment scenarios each system was deployed.

### 5.1.1	Analogue Radio

Table 5.1.1

Parameter and usage of Analogue Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | J | J | J | J | RUS |
| Specific name (if have) |  | Analogue TRS | Yard Radio | Analogue TRS | REBS | Yard Radio | Yard Radio |  |
| Frequency Range (MHz) |  | 142-144, 146-149.9 | 142-144,146-149.9, 150.05-156.4875, 156.8375 - 160 | 335.4-360 | 335.4-340 | 335.4-399.9 | 450-470 | 2.124-2.136 2.144-2.156 |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 20 | 20 | 12.5 | 12.5 | 12.5 | 12.5 | 11.8 |
| Antenna gain (dBi) |  | BS: +15 MS: +4.2 | +10.5 | BS: +11 MS: +1 | +1 | TBD | TBD | 0 |
| Polarization |  | vertical | Vertical | vertical | vertical | vertical | vertical | dual |
| Transmitting radiation power (dBm) |  | BS: +47 MS: +40 | +30 to 40 | BS: +36 MS: +30 | +30 | +37 | +37 | 11.5 |
| e.i.r.p. (dBm) |  | BS:+62 MS: +44.2 | +40 to 50 | BS: +47 MS: +31 | +31 | TBD | TBD | 11.5 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <10 | <10 | <10 | <10 | <12 |
| Transmission data rate (kb/s) |  | NA | NA | NA | NA | NA | NA | NA |
| Transmission distance (km) |  | 3-40 | 1 | 1.5 - 3 | max 5 | 1 | TBD | max 15 |
| Modulation |  | FM | FM | FM | FM | FM | FM | FM |
| Multiplexing method |  | FDD | None | FDD | FDMA | None | None | FDMA |
| Reception quality |  | SNR > 40 | SNR > 30 | SNR > 30 | NA | SNR > 30 | SNR > 30 | NA |
| Applications | Voice/Dispatch | X | X | X | X | X |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train Information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  |  |  |  |  |
|  | Railway station, | X | X |  | X | X | X |  |
|  | Shunting yard |  | X |  |  | X | X |  |
|  | Maintenance Base |  | X |  |  | X | X |  |

Table 5.1.1 (continued-1)

Parameter and usage of Analogue Radio

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | D | HNG | E | D | SUI |
| Specific name (if have) |  | Analogue TRS | Analogue RHST | Analog Radio | Analog Radio | Analog Radio (Tren-Tierra) | Digital mobile radio (DMR) | Analog Radio |
| Frequency Range (MHz) |  | 410 - 420 | 412-417.5 / 451.5-462 | 68-87.5 (4 m Band), 146-174 (2 m Band), 443-470 (70 cm Band) | 148.8 - 163.9, 158.100-158.375, 163.100-163.375, 457.400-468.450, 457.38-458.48, 467.38-468.48 | 447.550-448.650 (downlink) / 457.600-458.600(uplink) | 68 - 87.5 (4 m Band) 146 - 174 (2 m Band) | 410 - 430, 450-470 |
| Radiocommunication Standards |  |  |  | UIC Code 751-3 | UIC 751-3, ETSI EN 300 086-2 | UIC 751-3 | ETSI | ETSI EN |
| Channel separation (kHz) |  | 12.5 | UL: 12.5, 288 DL: 700 | 25 | 25 | 50 | 12.5 | 25, 12.5 |
| Antenna gain (dBi) |  | BS: +11 MS: +1 | BS: (LCX) MS: +5 | 1.8-17 | 9 |  |  | 1.8-17 |
| Polarization |  | vertical | vertical | Vpol |  |  |  | Vpol |
| Transmitting radiation power (dBm) |  | +30 | BS: +33 MS: +36 |  |  |  |  | 26.9-54.5 |
| e.i.r.p. (dBm) |  | BS: +41 MS: +31 | BS: (LCX) MS: +41 | 26.9 -54.5 |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | < 10 | app. 4-5 | 20 | SNR>20dB |  | 4-5 |
| Transmission data rate (kb/s) |  | NA | NA |  |  |  | 9.6 |  |
| Transmission distance (km) |  | 1.3 -3 | LCX-MS: 0.001-0.002 (BS interval: 30) | 10-20 |  |  |  | 10-20 |
| Modulation |  | FM | DL: PM UL: FM | FM | FM | FM | 4FSK | FM |
| Multiplexing method |  | none | DL: FDM UL: FDMA/FDD |  |  | TDMA |  | 25, 12.5 |
| Reception quality |  | SNR > 20 | SNR > 30 |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X | X | X | X | X | X |
|  | Maintenance | X | X | X |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X |  | X | X |
|  | Railway station, | X | X | X |  |  | X |  |
|  | Shunting yard | X |  | X |  |  | X |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.1 (continued-2)

Parameter and usage of Analogue Radio

| # |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | F | ARM | UZB | UZB | UZB | THA | S |
| Specific name (if have) |  | Analog RLE (Radio Local Enterprise) | Short wave radio communication system | 46MTsV-MS, 43RTS, 71RTS “Motorola” “Vertex” | Fixed radio station RS-46MTsV.01, RS-46MTsV-02, PS-46-MTsV-41 manufactured in the Russian Federation | Mobile radio station 55R22V-1.2 MK “Transport – RV-1.2 MK” manufactured in the Russian Federation | VHF Land Mobile Radio | Working teams (Railway) |
| Frequency Range (MHz) |  | 150 160-164 165-173 444-445 457-468 | 2.13 | MF band: 2.130, 2.150 VHF band: 150-174 | MF band: 2.130, 2.150, VHF band: 151.725-156.000 MHz | MF band: 2.130, 2.150 VHF band: 151.725-156.000 | 137-174 | 160 |
| Radiocommunication Standards |  | Standard 1: UIC Code 751-3 “Technical regulations for international analogue ground-train radio systems” | domestic standard | Standard: MPT-1327 |  | Effect of climate and mechanical factors as per YaUISH.464421.002 TU | VHF/FM | n/a |
| Channel separation (kHz) |  | 12.5 |  | 12.5 |  | 25 |  |  |
| Antenna gain (dBi) |  | 2,41-7,41 |  | 3.6 dB, 9 dB | ≥7 |  |  |  |
| Polarization |  | vertical |  | vertical | circular |  |  |  |
| Transmitting radiation power (dBm) |  |  | BS: 40.8 |  |  | MF band: up to 41.5, VHF band: up to 40; |  |  |
| e.i.r.p. (dBm) |  | 3,41 - 12,41 |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | NA |  | 1.2 |  |  |  |  |
| Transmission distance (km) |  | 10 km |  | 30 |  |  |  |  |
| Modulation |  | AM | AM | TDMA | angle |  |  |  |
| Multiplexing method |  | FDMA |  |  |  |  |  |  |
| Reception quality |  | NA |  |  |  |  |  |  |
| Applications | Voice/Dispatch | X (voice) |  | X | X | X |  |  |
|  | Maintenance | X |  | X | X | X |  |  |
|  | Train Control |  |  | X | X | X |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  | X |  |  |
| Scenarios | Railway line | X |  |  |  | X |  |  |
|  | Railway station, | X |  | X | X | X |  |  |
|  | Shunting yard | X |  | X | X | X |  |  |
|  | Maintenance Base | X |  | X | X | X |  |  |

Table 5.1.1 (continued-3)

Parameter and usage of Analogue Radio

| # |  | 22 | 23 | 24 | 25 | 26 | 27 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | G | S | S | CHN | KOR | VTN | RUS |
| Specific name (if have) |  | Network Rail RETB System (Radio Electronic Token Block) | Radio link for Radio Block | Radio Block | 450 MHz Band Wireless Train Dispatching System | VHF | Analogue Train radio | VHF |
| Frequency Range (MHz) |  | 180-183/188-191 | 380 | 468 | 457.200-458.650 467.200-468.650 | 153 | 138 – 174 MHz 406.2 – 470 MHz | 151.7125 – 154.0125 154.9875 – 156.0125 |
| Radiocommunication Standards |  | domestic standard | n/a | n/a |  |  |  | GOST-12252-86 (Rus) |
| Channel separation (kHz) |  | 12.5 |  |  | 25 | 25 | 12.5; 25 | 12.5; 25 |
| Antenna gain (dBi) |  |  |  |  | Fixed radio station: 9(Omni-directional), 12(Directional) Locomotive station: 0 | TBD |  | 0 - 17 |
| Polarization |  |  |  |  | NA |  |  | V |
| Transmitting radiation power (dBm) |  |  |  |  | Fixed Radio Station: 34.7-37(simplex), 37-40 (duplex), Locomotive Station: 37(simplex), 40 (duplex), Handset: 34.7 |  | + 47 | 33 … 40 |
| e.i.r.p. (dBm) |  |  |  |  | NA |  |  | 33 …57 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  | NA |  |  | NA |
| Transmission data rate (kb/s) |  |  |  |  | NA |  |  | NA |
| Transmission distance (km) |  |  |  |  | NA |  |  | 10 |
| Modulation |  |  |  |  | FM |  | FM | FM |
| Multiplexing method |  |  |  |  | NA |  |  | NA |
| Reception quality |  |  |  |  | Adjacent-channel Selectivity (receiver) ≥65dB ,Co-channel Rejection≥-8dB |  |  | NA |
| Applications | Voice/Dispatch | X |  |  | X | X | X | X |
|  | Maintenance |  |  |  | X | X | X | X |
|  | Train Control | X |  |  |  |  | X | X |
|  | Emergency |  |  |  |  | X | X | X |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  | X | X | X | X |
|  | Railway station, | X |  |  | X | X | X | X |
|  | Shunting yard |  |  |  | X | X | X | X |
|  | Maintenance Base | X |  |  | X | X | X | X |

Table 5.1.1 (continued-4)

Parameter and usage of Analogue Radio

| # |  | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | USA* | USA | USA | USA | USA | USA | USA |
| Specific name (if have) |  | Dispatch | Train to Train /Mobile | Defect Detector | Locomotive health/fuel management reporting | Crossing health reporting | Switchman communications | Yard operations |
| Frequency Range (MHz) |  | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 7.5 KHz | 7.5 KHz | 7.5 KHz | 7.5 KHz | 7.5 KHz | 7.5 KHz | 7.5 KHz |
| Antenna gain (dBi) |  | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various |
| Polarization |  | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical |
| Transmitting radiation power (dBm) |  | Varies | Varies1 | Varies1 | Varies1 | Varies1 | Varies1 | Varies1 |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | Voice | Voice | Voice | See footnote | See footnote 2 | Voice | Voice |
| Transmission distance (km) |  | Varies | Varies | 10 | Varies | 10 | Varies | Varies |
| Modulation |  | FM | FM | FM | FM | FM | FM | FM |
| Multiplexing method |  | None | None | None | see footnote | see footnote 3 | None | None |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch | x | x |  |  |  | x | x |
|  | Maintenance | x |  | x | x |  | x | x |
|  | Train Control | x | x |  |  | x | x | x |
|  | Emergency | x | x | x |  |  | x | x |
|  | Train Information | x | x | x | x | x | x | x |
| Scenarios | Railway line | x | x | x |  | x |  |  |
|  | Railway station, | x | x | x |  |  | x | x |
|  | Shunting yard | x | x | x |  |  | x | x |
|  | Maintenance Base | x | x | x | x |  | x | x |

* Due to the private ownership of railways in the United States, the parameters and usage can be considered representative of railway communications systems in the United States but not definitive.

Table 5.1.1 (continued-5)

Parameter and usage of Analogue Radio

| # |  | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | USA | USA | USA | USA | USA | USA | USA |
| Specific name (if have) |  | Engineering performance monitoring | Remote Control Switches and Signals | End of Train | Automatic train Control | Microwave | High rail Compliance | Automatic Equipment Identification |
| Frequency Range (MHz) |  | 160.215-161.565 |  | 457.9375 / 452.9375 | 896.888 - 936.988 | 2000-18000 | 403 - 420 | 902 - 928 |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 7.5 KHz |  | 7.5 KHz | 50 |  | 7.5 KHz | 25 KHz |
| Antenna gain (dBi) |  | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various |
| Polarization |  | vertical | Varies1 | vertical | vertical |  | vertical |  |
| Transmitting radiation power (dBm) |  | Varies1 |  | Varies1 | Varies1 |  | Varies1 | Varies1 |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | 4800-96002 |  | 4800-96002 |  |  |  |  |
| Transmission distance (km) |  | 1.3 -3 | Varies | 5 |  |  |  |  |
| Modulation |  | FM | DTMF | FM | FM |  | FM/TDMA/FDMA | FM |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance | x | x |  |  |  | x | x |
|  | Train Control | x | x | x | x | x | x |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  | x | x | x |  | x |
| Scenarios | Railway line | x | x | x | x | x | x | x |
|  | Railway station, | x | x | x | x |  | x | x |
|  | Shunting yard | x | x | x |  |  | x | x |
|  | Maintenance Base | x | x | x |  |  | x | x |

### 5.1.2	Digital Radio

Table 5.1.2

Parameter and usage of Digital Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | J | J | J | CHN | RUS |
| Specific name (if have) |  | Digital TRS (Type 1) | Digital TRS (Type 2) | Digital TRS (Type 3) | Digital TRS | JRTC Radio | Digital Radio | DMR |
| Frequency Range (MHz) |  | 140 - 144, | 146 - 149.9 | 146 - 149.9 | 335.4 - 360 | 335.4 - 360 | 403-423.5 | 151.7125 – 154.0125 154.9875 – 156.0125 |
| Radiocommunication Standards |  |  |  |  |  |  | GB32659-2016 | ETSI 102361-01 |
| Channel separation (kHz) |  | 6.25 | 25 | 6.25 | 6.25 | 6.25 | 12.5 | 12.5 |
| Antenna gain (dBi) |  | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | BS: +11 MS: +1 | NA | 2-17 |
| Polarization |  | Vertical | vertical | Vertical | Vertical | vertical | NA | vertical |
| Transmitting radiation power (dBm) |  | BS: +40 MS: +30 | BS: +37 MS: +30 | BS: +30 MS: +30 | BS: +36 MS: +24.8 | BS: +34.8 MS: +30 | BS:+45 Onboard:+44/+40/+37 MS:+37/+34.7/+30 | 33…46 |
| e.i.r.p. (dBm) |  | BS:+51 MS: +31 | BS:+48 MS: +31 | BS:+41 MS: +31 | BS: +47 MS: +24.8 | BS: +45.8 MS: +31 | BS:+45 Onboard:+44/+40/+37 MS:+37/+34.7/+30 | 35…63 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <10 | <10 | <10 | NA | NA |
| Transmission data rate (kb/s) |  | 9.6 | 32 | 4.8 | 9.6 | 9.6 | NA | NA |
| Transmission distance (km) |  | 1-3 | 1-2 | 1-3 | 1.5-2 | 2-3 | NA | max 20 |
| Modulation |  | π/4QPSK | π/4QPSK | 4FSK | π/4QPSK | π/4QPSK | 4FSK | π/4QPSK |
| Multiplexing method |  | FDMA or SCPC | TDMA | FDMA or SCPC | FDMA | FDD, TDM-TDMA | NA | FDMA |
| Reception quality |  | BER < 10-4 | BER < 10-4 | BER < 10-4 | BER < 10-4 | BER < 10-4 | BER< 10-4 | NA |
| Applications | Voice/Dispatch | X | X | X | X |  | X |  |
|  | Maintenance |  |  |  |  |  | X |  |
|  | Train Control |  |  |  |  | X | X |  |
|  | Emergency | X |  |  |  |  | X |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X | X | X |  |
|  | Railway station, | X | X | X | X | X | X |  |
|  | Shunting yard |  |  |  |  |  | X |  |
|  | Maintenance Base |  |  |  |  |  | X |  |

Table 5.1.2 (continued-1)

Parameter and usage of Digital Radio

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | AUS | AUS | AUS | AUS | CZE |
| Specific name (if have) |  | digital RHST (type1) | digital RHST (type 2) | AUS-Queensland Wayside Radio | AUS-Queensland Automatic Train Protection (ATP) Radio | AUS-Queensland Train Control Radio | AUS-Queensland Maintenance Radio | PMR/PAMR |
| Frequency Range (MHz) |  | 412-417.5 / 451.5-462 | 410 – 420 / 450 - 455 | 403-420 | 403-420 | 403-420 | 403-420 | 148.2-149.05, 152.8-153.65 |
| Radiocommunication Standards |  |  |  | PMR | Wide area PMR | PMR → Tier 3 DMR | PMR → Tier 3 DMR | CEPT Rec. T/R 25-08 |
| Channel separation (kHz) |  | 300 | 240,300,600 | 25, 12.5 | 25 | 25 | 25 | 12.5 |
| Antenna gain (dBi) |  | BS: (LCX) MS: +5 | BS: (LCX) MS: +5 |  |  |  |  |  |
| Polarization |  | NA | NA |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | BS: +33 MS: +36 | BS: +27 MS: +36 |  |  |  |  | 40(max) |
| e.i.r.p. (dBm) |  | BS: NA MS: +41 | BS: NA MS: +41 |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | < 10 | < 10 |  |  |  |  |  |
| Transmission data rate (kb/s) |  | DL: 384+307.2 UL: 384+307.2 | DL: 384+384 UL: 384+307.2 | 9.6 | 9.6 | 9.6 | 9.6 |  |
| Transmission distance (km) |  | LCX 0.001-0.002 (BS interval: 30) | LCX 0.001-0.002 (BS interval: 30) |  |  |  |  |  |
| Modulation |  | π/4QPSK | DL: π/4QPSK UL: π/4QPSK and QPSK |  |  | TDMA | TDMA |  |
| Multiplexing method |  | DL: TDM UL: TDMA FDD | DL: TDM UL: TDMA FDD | TDMA | TDMA |  |  | Frequency Division Multiplexing |
| Reception quality |  | BER < 10-4 | BER < 10-4 |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X |  |  |  |  |  |
|  | Maintenance | X | X | X |  |  | X |  |
|  | Train Control | X |  |  | X | X |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X | X |  |  |
|  | Railway station, | X | X | X | X | X |  |  |
|  | Shunting yard | X | X |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  | X |  |

Table 5.1.2 (continued-2)

Parameter and usage of Digital Radio

| # |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | UZB | UZB | UZB | F | RUS | NOR | VTN |
| Specific name (if have) |  | HYT, manufacturer “Hytera”, People’s Republic of China | “DRM”-standard radiocommunication system, manufacturer “Hytera”, People’s Republic of China | RD-985-type relay | Digital RLE (Radio Local d'Entreprise) Direct Mode | DECT | ERTMS | Digital Train radio |
| Frequency Range (MHz) |  | 150-174 | 136-174 |  | 400 MHz 465.1625 MHz / 455.1625 MHz (UL/DL) | 1880 - 1900 |  | 138–174 MHz 406.2–470 MHz |
| Radiocommunication Standards |  | DMR (Digital Mobile Radio) | Protocol DRM Tier II TDMA; Digital protocol – ETSI-TS102 361-1, -2, -3; Military standard (for MD-785, PD-705, PD-785):(USA) – MIL-STD-810 C/D/E/F/G |  | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 | DECT | 3GPP TS, EIRENE SRS |  |
| Channel separation (kHz) |  | 6.25, 12.5kHz |  |  | 25 | 1728 |  | 6.25; 112.5; 25 |
| Antenna gain (dBi) |  | 3.6 dB, 9 dB | 0 dBi |  | 0 - 1 | 2 … 15 |  |  |
| Polarization |  | vertical |  |  | vertical | V/H |  |  |
| Transmitting radiation power (dBm) |  |  | (for RD.985 and MD-785) 37-47 |  |  | 10 |  | +47 |
| e.i.r.p. (dBm) |  |  |  |  | NA | 12 … 25 |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  | NA |  |  |
| Transmission data rate (kb/s) |  | 2 |  |  | 7,2 kbit/s | 1152 |  |  |
| Transmission distance (km) |  | 25 |  |  | 0,8 - 1,5 | 1 |  |  |
| Modulation |  |  | 4FSK (digital mode) and FM (analogue mode) |  | π/4 DQPSK | GFSK |  | FDMA, TDMA |
| Multiplexing method |  | TDMA |  |  | TDMA | TDMA |  | None |
| Reception quality |  |  |  |  | NA | NA |  |  |
| Applications | Voice/Dispatch | X | X | X | X |  |  | X |
|  | Maintenance | X | X | X | X |  |  | X |
|  | Train Control | X | X | X |  |  | X | X |
|  | Emergency | X | X | X | X |  | X | X |
|  | Train information | X | X | X |  |  |  |  |
| Scenarios | Railway line | X | X | X | X |  | X | X |
|  | Railway station, | X | X | X | X |  | X | X |
|  | Shunting yard | X | X | X | X |  |  | X |
|  | Maintenance Base | X | X | X | X |  |  | X |

Table 5.1.2 (continued-3)

Parameter and usage of Digital Radio

| # |  | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | UZB | UZB | D | SUI | I | I | ARM |
| Specific name (if have) |  | Type MD-785G fixed/mobile radio station | Type PD-705G and PD-785 portable radio station | Radio Remote Control System of Traction Vehicle for Freight Vehicles | Radio Remote Control System of Traction Vehicle for Freight Vehicles | VHF Radio | UHF Radio | Station and repair service of radio communication |
| Frequency Range (MHz) |  |  |  | 419.730, 419.770, 419.790; further Railways: 429.730, 429.770, 429,790 | 410-470 | 164.7 - 164.75 | 440 – 443 | 151.825, 151.875, 153, 154 |
| Radiocommunication Standards |  |  |  | EN 50239:1999, ETSI EN 300 113, ETSI EN 300 220 | EN |  |  | domestic standard |
| Channel separation (kHz) |  |  |  | 12.5/20/25 |  | 12.5 | 12.5 |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  | vertical |  | 360° | 360° | vertical |
| Transmitting radiation power (dBm) |  |  |  | 27(standard), optional up to 33 |  |  |  | 37 |
| e.i.r.p. (dBm) |  |  |  |  |  | 30– 37 | 30 – 37 |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  | 3.1/RF transmission (12.5 kHz) 4.8 /RF transmission (20/25 kHz) |  |  |  |  |
| Transmission distance (km) |  |  |  | < 1 km |  | 1 - 2 | 1 - 2 |  |
| Modulation |  |  |  | F3D |  | FM | FM | FM |
| Multiplexing method |  |  |  | TDMA |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X |  |  | X | X |  |
|  | Maintenance | X | X |  | X | X |  |  |
|  | Train Control | X | X | X |  |  | X |  |
|  | Emergency | X | X |  | X |  |  |  |
|  | Train information | X | X |  |  |  |  |  |
| Scenarios | Railway line | X | X |  |  |  | X |  |
|  | Railway station, | X | X |  | X |  | X |  |
|  | Shunting yard | X | X | X | X | X |  |  |
|  | Maintenance Base | X | X |  |  | X |  |  |

Table 5.1.2 (continued-4)

Parameter and usage of Digital Radio

| # |  | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | CAN | CAN | AUS | CAN | USA | USA | USA |
| Specific name (if have) |  | CTC using voice dispatch | Wayside sensors | AUS-Queensland Linking Radios | Remote Control Locomotives in yards | Dispatch | Train to Train /Mobile | Defect Detector |
| Frequency Range (MHz) |  | 160.17-161.58 | 160.17-161.58 | 403-420, 900, 6000, 8000, 10000 | 450, 900 | 160.215-161.565 MHz | 160.215-161.565 MHz | 160.215-161.565 MHz |
| Radiocommunication Standards |  | domestic standard | domestic standard | PDH, SDH, Ethernet | domestic standard | NXDN | NXDN | NXDN |
| Channel separation (kHz) |  | 12.5 | 12.5 |  |  | 6.25 | 6.25 | 6.25 |
| Antenna gain (dBi) |  |  |  |  |  | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various |
| Polarization |  |  |  |  |  | Varies1 | Varies1 | Varies1 |
| Transmitting radiation power (dBm) |  | Fixed Station:50.4 Mobile:47.8 | Fixed Station:50.4 Mobile:47.8 |  |  |  |  |  |
| e.i.r.p. (dBm) |  | 45.1 | 45.1 |  | −70 dBW/MHz | See footnote 1 | See footnote 1 | See footnote 1 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  | Voice | Voice | Voice |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  | Varies | Varies | 10 |
| Modulation |  |  |  |  |  | FM/4 FSK | FM/4 FSK | FM Editor’s note: since it is digital system, why FM is in use. |
| Multiplexing method |  |  |  |  |  | None | None | None |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch | X |  |  | x | x |  |  |
|  | Maintenance |  | X |  | x |  | x |  |
|  | Train Control | X |  | X | x | x |  |  |
|  | Emergency |  |  |  | x | x | x |  |
|  | Train information |  |  |  | x | x | x |  |
| Scenarios | Railway line | X |  |  | x | x | x |  |
|  | Railway station, | X | X |  | x | x | x |  |
|  | Shunting yard |  | X | X | x | x | x |  |
|  | Maintenance Base |  |  |  | x | x | x |  |

Table 5.1.2 (continued-5)

Parameter and usage of Digital Radio

| # |  | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | USA | USA | USA | USA | ZMB | MWI | TZA |
| Specific name (if have) |  | Locomotive health/fuel management reporting | Crossing health reporting | Switchman communications | Yard operations | PMR/Trunking | PMR | PMR/TRUNKING |
| Frequency Range (MHz) |  | 160.215-161.565 | 160.215-161.565 | 160.215-161.565 | 160.215-161.565 | 340-381/410-450 | 136-174 | 148.4-167/775 Editor’s note: the frequency 775 is needed to be checked. |
| Radiocommunication Standards |  | NXDN | NXDN |  |  |  |  |  |
| Channel separation (kHz) |  | 6.25 | 6.25 | 6.25 | 6.25 | 12.5;25 | 25 | 25 |
| Antenna gain (dBi) |  | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various | BS: +various MS: +various |  |  |  |
| Polarization |  | Varies1 | Varies1 | Varies1 | Varies1 |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  | See footnote 1 | See footnote 1 | See footnote 1 | See footnote 1 |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | See footnote 3 | See footnote 3 | Voice | Voice |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  | Varies | 10 | Varies | Varies |  |  |  |
| Modulation |  | FM | FM | FM/4 FSK | FM/4 FSK | FM | FM | FM |
| Multiplexing method |  | see footnote3 | see footnote3 | NXDN (Location Specific) | NXDN (Location Specific) | FDMA | FDMA | FDMA |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  | x | x |  |  |  |
|  | Maintenance | x |  | x | x |  |  |  |
|  | Train Control |  | x | x | x |  |  |  |
|  | Emergency |  |  | x | x |  |  |  |
|  | Train information | x | x | x | x |  |  |  |
| Scenarios | Railway line |  | x |  |  |  |  |  |
|  | Railway station, |  |  | x | x |  |  |  |
|  | Shunting yard |  |  | x | x |  |  |  |
|  | Maintenance Base | x |  | x | x |  |  |  |

Table 5.1.2 (continued-6)

Parameter and usage of Digital Radio

| # |  | 43 | 44 | 45 | 46 | 47 | 48 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | MOZ | NMB | AGL | BOT | BOT | SWZ |  |
| Specific name (if have) |  | PMR | PMR | PMR | PMR | PMR/Trunking | PMR/PAMR |  |
| Frequency Range (MHz) |  | 401-470 | 138.75-141.45 138.85-138.850// 142.85-143.1750 | 150-170 | 410-430 | 440-450 | 410-450 |  |
| Radiocommunication Standards |  |  |  |  |  |  |  |  |
| Channel separation (kHz) |  | 12.5 | 12.5 | 25 | 25 | 25 | 25 |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  | FM | FM | FM | - | FM | FM |  |
| Multiplexing method |  | FDMA | FDMA | FDMA | TDMA | FDMA | FDMA |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 5.1.3	Trunked Radio

Table 5.1.3

Parameter and usage of Trunked Radio

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | KOR | KOR | FIN | THA | F | QAT | VTN |
| Specific name (if have) |  | ASTRO | TETRA | TETRA | UHF Land Mobile Radio | Digital RLE | Tetra for critical communication | Tetra |
| Frequency Range (MHz) |  | 806-811 851-865 | 806-811 851-865 | 380-385, 390-395, + direct mode channels | 380-510 | 407-470 | 410 – 430 | 410-430 |
| Radiocommunication Standards |  |  |  | ETSI 300 392-2 | UHF/FM, TETRA | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 | ETSI 300 392-1 |  |
| Channel separation (kHz) |  | 12.5 | 4x25 | 25 |  | 25 | 25 | 25 |
| Antenna gain (dBi) |  | 3 | 3 | 5-15 |  | 14 | 0.7 | 7.6 |
| Polarization |  | NA | NA | vPol, xPol |  | vertical |  |  |
| Transmitting radiation power (dBm) |  | BS: 70W Cab: 30W UE: 3W | BS: 25W Cab: 3W UE: 1W | BS: 44 |  | UE :32 Mobile : 35 BTS : 32 - 35 | handset: 30; BTS: 40 | +48.7 |
| e.i.r.p. (dBm) |  | NA | NA | 46-53 |  | 40 |  | +56.3 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | -118 | BS: -125 UE: -112 | C/I 19 |  |  |  |  |
| Transmission data rate (kb/s) |  | 9.6 | 36 | 4,567/7,2 |  | 7,2 kbit/s for one TCH. up to 28,8 kbit/s when allocating up to 4 traffic channels to the same communication. |  |  |
| Transmission distance (km) |  | NA | NA | 5-35 |  | max 4 km |  |  |
| Modulation |  | C4FM | π/4 DQPSK | π/4DQPSK |  | π/4 DQPSK |  | π/4 DQPSK |
| Multiplexing method |  | FDMA | TDMA | TDMA |  | TDMA |  | TDMA |
| Reception quality |  | NA | NA | NA |  | NA |  |  |
| Applications | Voice/Dispatch | X | X | X | X | X | X | x |
|  | Maintenance | X | X | X | X | X |  | X |
|  | Train Control | X | X | X |  |  |  | X |
|  | Emergency | X | X | X | X |  |  | X |
|  | Train information |  |  |  |  |  | X |  |
| Scenarios | Railway line | X | X | X | X | X | X | x |
|  | Railway station, | X | X | X | X | X | X | x |
|  | Shunting yard | X | X | X | X | X |  | X |
|  | Maintenance Base | X | X | X | X | X |  | X |

Table 5.1.3 (continued-1)

Parameter and usage of Trunked Radio

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | D | SUI | RUS | MWI | CHN | AFS |  |
| Specific name (if have) |  | TETRAPOL / TETRA | POLYCOM | TETRA System | TETRA | B-TrunC | TETRA |  |
| Frequency Range (MHz) |  | 410 - 430, 440 - 443, 445 - 448 | 380-400 | 457.4 – 458.45 467.4 – 468.45 | 387-390/397-399.9 | 10MHz in 1785-1805 | 410-450 |  |
| Radiocommunication Standards |  | ETSI | TETRAPOL | ETSI |  | YD/T 2889-2015 |  |  |
| Channel separation (kHz) |  | 12.5 | 12.5 | 25 | 25 |  | 25 |  |
| Antenna gain (dBi) |  |  |  | 3-15 |  | 17.5(2 path) 17.5(4 path) |  |  |
| Polarization |  |  |  | vertical |  |  |  |  |
| Transmitting radiation power (dBm) |  | BS:27,46 |  | 44 |  | Station:43（1 path） Mobile:23 |  |  |
| e.i.r.p. (dBm) |  |  | 44 | 47 - 59 |  | 66.5 |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  | NA |  |  |  |  |
| Transmission data rate (kb/s) |  | 7.2 | 8 | NA |  |  |  |  |
| Transmission distance (km) |  |  | 6-20 | max 12 |  |  |  |  |
| Modulation |  | π/4 DQPSK | GMSK | π/4DQPSK | π/4 DQPSK | QPSK, 16QAM, 64QAM | π/4 DQPSK |  |
| Multiplexing method |  | TDMA | FDMA | TDMA | TDMA | TDD | TDMA |  |
| Reception quality |  |  |  | NA |  |  |  |  |
| Applications | Voice/Dispatch | X | X | X |  | X |  |  |
|  | Maintenance |  |  | X |  | X |  |  |
|  | Train Control |  |  | X |  |  |  |  |
|  | Emergency |  |  | X |  |  |  |  |
|  | Train information | X | X |  |  |  |  |  |
| Scenarios | Railway line | X | X | X |  |  |  |  |
|  | Railway station, | X | X | X |  |  |  |  |
|  | Shunting yard |  |  | X |  | X |  |  |
|  | Maintenance Base |  |  | X |  |  |  |  |

### 5.1.4	GSM-R

Table 5.1.4

Parameter and Usage of GSM-R

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | CHN | CEPT Editor’s note: CEPT Coordinator is requested to clarify whether CEPT is representing all CEPT Administrations | CZE | HNG | HOL | HOL | E |
| Specific name (if have) |  | GSM-R | GSM-R | GSM-R | GSM-R | GSM-R | GSM-R | GSM-R |
| Frequency Range (MHz) |  | 885 - 889 930 - 934 | 876-880 921-925 | 876-880.1, 921-925.1 | 876-880/921-925 | 876-880/921-925 | 876-880/921-925 | 876-880/921-925 |
| Radiocommunication Standards |  |  | ETSI EN 301 502 ETSI EN 301 511 ETSI EN 301 515 ETSI TS 102 281 EIRENE SRS | CEPT ECC Dec (02)05, CEPT Rec. T/R 25-09 | 3GPP TS, EIRENE SRS | 3GPP TS 45.005 | EIRENE 7.4.0/15.4.0 | 3GPP TS, EIRENE SRS |
| Channel separation (kHz) |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| Antenna gain (dBi) |  | MS: ≥0 BS: 17(21) | BS: 17 typical Cab: 0 typical |  | 14 | 18 | Multiple | 9-21 |
| Polarization |  | dual | cross-polar typical |  | dual |  | Multiple | Circular/Vertical/Horizontal |
| Transmitting radiation power (dBm) |  | UE: 33 Cab: 39 BS: 46 | BS: 43 typical Cab: 39 | 55.5(max) | BS: 46 |  | Multiple |  |
| e.i.r.p. (dBm) |  | NA | BS: 57 typical Cab: 39 |  |  | 61 | Multiple | 53.3 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA | BS: 57 typical Cab: 39 |  |  | 8 | Multiple |  |
| Transmission data rate (kb/s) |  | NA | Voice: 13 CSD: 4.8 GPRS/EDGE: 8 to 29.6 |  |  |  | Up to Edge | 2.4-9.6 |
| Transmission distance (km) |  | NA | 7 typical |  |  | 3-4 (urban); 8 (rural) | 3-4 (urban); 8 (rural) cell size | 4 |
| Modulation |  | GMSK | GMSK EDGE: 8PSK | GMSK | GMSK | GMSK | GMSK, 8-PSK | GMSK |
| Multiplexing method |  | TDMA | TDMA | TDMA | TDMA | TDMA | TDMA | TDMA |
| Reception quality |  | SNR MS: ≤-104 SNR BS: ≤ -110 | C/I 12 dB for voice on conventional lines RxQual < 4 at 4m above rail tracks |  | MS:-104 | coverage level ≥-72 dBm, C/I≥12dB (17 dB for ETCS/ERTMS level 2) | coverage level ≥ -72 dBm, C/I ≥ 12dB (17 dB for ETCS/ERTMS level 2) |  |
| Applications | Voice/Dispatch | X | X | X | X | X | X | X |
|  | Maintenance | X | X | X | X | X |  | X |
|  | Train Control | X | X | X | X | X | X | X |
|  | Emergency | X | X | X | X | X | X | X |
|  | Train information | X | X | X | X | X | X | X |
| Scenarios | Railway line | X | X | X | X | X | X | X |
|  | Railway station, | X | X | X | X | X | X | X |
|  | Shunting yard |  | X | X | X | X | X | X |
|  | Maintenance Base | X | X | X | X | X | X | X |

Table 5.1.4 (continued-1)

Parameter and usage of GSM-R

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | UAE | G | NOR | FIN I SUI D | SUI D | S | SUI |
| Specific name (if have) |  | GSM-R | Network Rail GSM-R System | GSM-R including GPRS | GSM-R | GSM-R | Train communications | GSM-P, UMTS |
| Frequency Range (MHz) |  | 876-880/921-925 | 876-880/ 921--925 | 876-880/921-925 | 876-880, 921-925 | 873-876 /918-921 | 925 | 880-915 / 1 920-1 980, 925-960 / 1 805-1 880 |
| Radiocommunication Standards |  | 3GPP TS, EIRENE SRS | 3GPP TS, EIRENE SRS | 3GPP TS, EIRENE SRS | 3GPP TS, EIRENE SRS | 3GPP TS,EIRENESRS | 3GPP TS, EIRENE SRS | 3GPP TS |
| Channel separation (kHz) |  | 200 | 200 | 200/12.5 | 200 | 200 | 200 | 5000 |
| Antenna gain (dBi) |  | 20.8 | BS:14 | 14 | 2-21 | 2-21 | BS:14 | 2-18 |
| Polarization |  | DUAL | DUAL | DUAL | Vpol and Xpol | Vpol and Xpol | DUAL | DUAL |
| Transmitting radiation power (dBm) |  | BS:46 | BS:46 | BS:46 |  |  | BS:46 | 46 |
| e.i.r.p. (dBm) |  |  | 62 | BS:60 | 16-67 | 16 - 67 |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  | 8dB | app. 8 |  | 7 |
| Transmission data rate (kb/s) |  |  |  |  | up to 8 x 22.8 | up to 8 x 22.8 |  | 40000 |
| Transmission distance (km) |  | 5-10 |  |  | typically: ca. 8, max. 35 | typically: ca. 8, max. 35 |  |  |
| Modulation |  | GMSK | GMSK | GMSK | GMSK | GMSK | GMSK | QPSK/16QAM |
| Multiplexing method |  | TDMA | TDMA | TDMA | TDMA | TDMA | TDMA | CDMA |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X | X | X | X | X | X |
|  | Maintenance | X | X | X | X | X | X |  |
|  | Train Control | X | X | X | X | X | X |  |
|  | Emergency | X | X | X | X | X | X |  |
|  | Train information | X | X | X | X | X | X |  |
| Scenarios | Railway line | X | X | X | X | X | X | X |
|  | Railway station, | X | X | X | X | X | X |  |
|  | Shunting yard | X | X | X | X | X | X |  |
|  | Maintenance Base | X | X | X | X | X | X |  |

Table 5.1.4 (continued-2)

Parameter and usage of GSM-R

| # |  | 15 | 16 | 17 | 18 | 19 | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | F | RUS | ZMB | AFS | MOZ | SWZ |  |
| Specific name (if have) |  | GSM-R | GSM-R System | GSM-R |  |  |  |  |
| Frequency Range (MHz) |  | 876-880, 921-925 | 876 – 880 921 - 925 | 876-880 MHz (UL) 921-925 MHz (DL) | 876-880/921-925 | 876-880/921-925 | 876-880/921-925 |  |
| Radiocommunication Standards |  | UIC Standard: Eirene http://www.era.europa.eu/Document-Register/Pages/Set-1-and-2-and-3-EIRENESRS.aspx | GSM-R |  |  |  |  |  |
| Channel separation (kHz) |  | 200 | 200 | 200 | 200 | 200 | 200 |  |
| Antenna gain (dBi) |  | 0 (Mobile) - 18 (Fixed Station) | 2-20 |  |  |  |  |  |
| Polarization |  | vertical, slant +/- 45 degrees | vertical |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | 39 (Mobile) - 43 (Fixed Station) | 33 …46 |  |  |  |  |  |
| e.i.r.p. (dBm) |  | 36 (Mobile) - 57 (Fixed Station) | 35 … 66 |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <8 | NA |  |  |  |  |  |
| Transmission data rate (kb/s) |  | 4,8 | 8x22.8 |  |  |  |  |  |
| Transmission distance (km) |  | max 20km | max 20 |  |  |  |  |  |
| Modulation |  | GMSK | GMSK | GSMK | GSMK | GSMK | GSMK |  |
| Multiplexing method |  | TDMA | TDMA | TDMA | TDMA | TDMA | TDMA |  |
| Reception quality |  | NA | NA |  |  |  |  |  |
| Applications | Voice/Dispatch | X | X |  |  |  |  |  |
|  | Maintenance | X | X |  |  |  |  |  |
|  | Train Control | X | X |  |  |  |  |  |
|  | Emergency | X | X |  |  |  |  |  |
|  | Train information | X |  |  |  |  |  |  |
| Scenarios | Railway line | X | X |  |  |  |  |  |
|  | Railway station, | X | X |  |  |  |  |  |
|  | Shunting yard |  | X |  |  |  |  |  |
|  | Maintenance Base | X | X |  |  |  |  |  |

### 5.1.5	LTE based RSTT

Table 5.1.5

Parameter and usage of LTE based RSTT

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | KOR | AUS-Queensland |  |  |  |  |  |
| Specific name (if have) |  | LTE-R | ETCS Level 2 (proposed) |  |  |  |  |  |
| Frequency Range (MHz) |  | 718 - 728 773 - 783 | 1770 – 1785, 1865 – 1880 |  |  |  |  |  |
| Radiocommunication Standards |  |  | None → LTE |  |  |  |  |  |
| Channel separation (kHz) |  | 10 MHz |  |  |  |  |  |  |
| Antenna gain (dBi) |  | NA |  |  |  |  |  |  |
| Polarization |  | NA |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  | UE: 23 BS: 46 |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  | NA |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | DL: 75000 UL: 37000 |  |  |  |  |  |  |
| Transmission distance (km) |  | NA |  |  |  |  |  |  |
| Modulation |  | DL: OFDMA UL: SC-FDMA |  |  |  |  |  |  |
| Multiplexing method |  | TFD FDD |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch | X |  |  |  |  |  |  |
|  | Maintenance | X |  |  |  |  |  |  |
|  | Train Control | X |  |  |  |  |  |  |
|  | Emergency | X |  |  |  |  |  |  |
|  | Train information | X |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  |  |  |  |  |
|  | Railway station, | X |  |  |  |  |  |  |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base | X |  |  |  |  |  |  |

### 5.1.6	Emergency System

Table 5.1.6

Parameter and usage of Emergency Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | J | J | KOR | S | S | S | S |
| Specific name (if have) |  | Japan REMCOS | Japan EARS | Korea TRPD | Fire alarm on overnight trains | Tunnel Rescue (Analogue NFM, radiating Cable) | Tunnel Rescue (TETRA) | "Typhon" Alarm |
| Frequency Range (MHz) |  | 142-144, 146-149.9, 150.05-156.4875, 156.8375-160 | 370-380 | 443.3125 | 439 | 448 | 378 | 406 |
| Radiocommunication Standards |  |  |  |  | n/a | ETSI tetra | ETSI | n/a |
| Channel separation (kHz) |  | 6.25, 20 | 6.25 | 1x12.5 |  | 25 |  |  |
| Antenna gain (dBi) |  | +1 | +1 | 3 |  |  |  |  |
| Polarization |  | vertical | vertical | vertical |  |  |  |  |
| Transmitting radiation power (dBm) |  | +30 | +30 | 36 |  |  |  |  |
| e.i.r.p. (dBm) |  | +28.5 | <30 | 39 |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | <10 | <10 | <2 |  |  |  |  |
| Transmission data rate (kb/s) |  | 9.6 | 9.6 | 8 |  | 7.2 |  |  |
| Transmission distance (km) |  | 1.3 | <1.0 | 4 |  |  |  |  |
| Modulation |  | π/4-QPSK | π/4-QPSK | GMSK | FSK | π/4-DQPSK |  |  |
| Multiplexing method |  | None | None | Single |  |  |  |  |
| Reception quality |  |  | >10-2 |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  | X |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency | X | X | X | X |  |  |  |
|  | Train information |  |  |  |  | X |  |  |
| Scenarios | Railway line | X | X |  | X | X |  |  |
|  | Railway station, | X | X |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.6 (continued-1)

Parameter and usage of Emergency Systems

| # |  | 8 | 9 | 10 | 11 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | RUS | RUS | RUS | USA |  |  |  |
| Specific name (if have) |  | SIR (Train Separation System) | SUTP (Brake control system) | KUPOL (System for locomotive stoppage) | Positive Train control |  |  |  |
| Frequency Range (MHz) |  | 151.7125 – 156.0125 | 155.075 – 155.275 | 154.9875 – 156.0125 | 217-222 |  |  |  |
| Radiocommunication Standards |  | GOST-12252-86 (RUS) | GOST-12252-86 (RUS) | GOST-12252-86 (RUS) |  |  |  |  |
| Channel separation (kHz) |  | 25 | 25 | 25 | 25 |  |  |  |
| Antenna gain (dBi) |  | 2 …15 | 2 …15 | 2 …15 | BS: +various MS: +various |  |  |  |
| Polarization |  | V | V | V | Vertical |  |  |  |
| Transmitting radiation power (dBm) |  | 40 | 40 | 40 | See footnote 1 |  |  |  |
| e.i.r.p. (dBm) |  | 42 …55 | 42 … 55 | 42 … 55 |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA | NA | NA |  |  |  |  |
| Transmission data rate (kb/s) |  | NA | NA | NA | See footnote |  |  |  |
| Transmission distance (km) |  | 10 | 10 | 10 | Varies |  |  |  |
| Modulation |  | GMSK | FM | FM | 4DQPSK |  |  |  |
| Multiplexing method |  | NA | NA | NA | TDMX |  |  |  |
| Reception quality |  | NA | NA | NA |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control | X | X | X | X |  |  |  |
|  | Emergency |  |  | X |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X |  |  |  |
|  | Railway station, | X | X | X | X |  |  |  |
|  | Shunting yard |  |  |  | X |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

### 5.1.7	Possible other train radios

Table 5.1.7

Parameter and usage of [XXX Systems]

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | D | I | D | FIN | SUI | D | I |
| Specific name (if have) |  | Continuous Automatic Train Running Control | Continuous Automatic Train Running Control | Intermittent Automatic Train Running Control | Intermittent Automatic Train Running Control | ZUB 121/262ct Train Running Control | Intermittent Automatic Train Speed Control | National Automatic Train Control System |
| Frequency Range (MHz) |  | 0.0356-0.0364, 0.0558-0.0562 | 0.00005, 0.000178 | 0.0005, 1, 2 | 27.1 | 0.850 (transmission train to trackside) 0.050 (transmitter circuit) 0.100 (energy supply during communication) | 0.85 | 5750, 5810 |
| Radiocommunication Standards |  | ORE |  |  |  | ERTMS Subset-044 FFFIS | domestic standard |  |
| Channel separation (kHz) |  |  |  |  |  |  | 200 |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | 1.2 ,0.6 |  |  |  | 50 | 50 |  |
| Transmission distance (km) |  | 0.001 |  |  |  | 0.001 | 0.001 |  |
| Modulation |  | FSK |  |  |  | FSK | FSK |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control | X | X | X | X | X | X | X |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X | X | X | X | X | X |
|  | Railway station, |  |  | X | X | X | X | X |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.7 (continued-1)

Parameter and usage of [XXX Systems]

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | FIN | I | E | E | FIN | S | I |
| Specific name (if have) |  | Hot box detector | Hot Box Detector | LZB | ASFA | RFID | RFID for train identification | Automated Track Workers Systems |
| Frequency Range (MHz) |  | Measuring frequency RSR 180: 250 kHz or 307 kHz Oscillating mirror: 2,4 kHz Oscillator on Scanner: 32,786 kHz Measuring frequency RSR 123: 1 MHz Oscillator on Scanner MM2: 60 MHz Oscillator on Scanner MM3: 533 MHz | 0.039, 0.050, 0.038, 0.042 | 0.036 (Track to train) 0.056 (Train to track) | 0.050-0.120 | 865-868 | 860 | 460.4, 460.45, 460.3875, 440.025 |
| Radiocommunication Standards |  | None | EN, FS Railway Standards |  |  | ETSI 302 208 | ETSI | IEEE 802.11ac, IEEE 802.11n |
| Channel separation (kHz) |  |  |  |  |  | 400 | 200 | 12.5 |
| Antenna gain (dBi) |  |  |  |  |  | 6-7,5 |  |  |
| Polarization |  |  |  |  |  | cPol |  | vertical |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  | 30 |
| e.i.r.p. (dBm) |  |  |  |  |  | 31,15 |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  | SNR>15dB/4KHz (Trackside to Train) |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  | 1.2(trackside to train) / 0.6(train to trackside) |  | 40-640 |  | 9.6 |
| Transmission distance (km) |  |  |  |  |  | NA |  | 3-4 |
| Modulation |  |  |  | FSK |  | FM0/Miller |  | 4FSK |
| Multiplexing method |  |  |  |  |  | NA |  | halfplex/time plex |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  | X |
|  | Train Control | X | X |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line | X | X |  |  |  |  | X |
|  | Railway station, | X | X |  |  |  |  |  |
|  | Shunting yard | X | X |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.7 (continued-2)

Parameter and usage of [XXX Systems]

| # |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | I | I | I | F | D | QAT | QAT |
| Specific name (if have) |  | On Train Public Address Systems | Public Address Systems Wi-Fi | Driving Information System data collection at train depot | CBTC (Next E0) | CBTC | CBTC | Wi-Fi BBRS for train to ground data communication |
| Frequency Range (MHz) |  | none | 2400, 5000 | 2400 | 5905-5925 | 2400-2500 ISM band (1785-1805 LTE-M) (5725-5875 ISM band) 5905 - 5925 5935 – 5965 | 2400, 5900 | 5800 |
| Radiocommunication Standards |  |  | IEEE 802.11ac, IEEE 802.11n | IEEE 802.11 WiFi, IEEE 802.11b/g | IEEE 802.11a CSMA | IEEE 802.11 a/b/g/p | EN 301893, IEEE 802.11 | IEEE 802.11 |
| Channel separation (kHz) |  |  | 20000 |  |  |  | 5000 | 5000 |
| Antenna gain (dBi) |  |  |  |  |  |  |  | 9-18 |
| Polarization |  |  | dual band double polarization |  |  |  |  | Vertical/Horizontal |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  | 19 -20 | 22-36 |  | Up to 10 |  | 33 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  | 802.11b:11000, 802.11g:54000 |  |  |  | 144.4 |
| Transmission distance (km) |  |  |  |  |  | up to 1 |  |  |
| Modulation |  |  | OFDM | CCK,DSSS,OFDM |  |  | FHSS | 64QPSK |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  | X |  | X | X | X |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  | X | X | X | X |  |
| Scenarios | Railway line |  | X |  | X | X | X |  |
|  | Railway station, |  |  | X | X | X | X |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.7 (continued-3)

Parameter and usage of [XXX Systems]

| # |  | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | QAT | HOL | VTN | F | I | I | D |
| Specific name (if have) |  | TCS | Marine VHF Radio | Communication-Based Train Control | DPAR | Trains diagnostics data communication | Level Crossing command | Radio-operated Strike-in and Radio Monitoring Signal for Simis LC |
| Frequency Range (MHz) |  | 5900 | VHF channel 18, 20 and 22 | 2400-2483.5 | 445.925 and 446.4625 MHz in a 55km-radius circle around Nançay city 407.7 – 407.9 MHz elsewhere | none | 0.092, 0.098 | 467.425 - 468.300 |
| Radiocommunication Standards |  | EN 301893, IEEE 802.11 |  | IEEE 802.11 | ETSI 300-113- 1 and 2 |  |  |  |
| Channel separation (kHz) |  | 5000 |  |  |  |  |  | 25 |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  | 30 |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  | OFDM |  |  |  |  |  |  |
| Multiplexing method |  | OFDMA |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  | X | X |  |  |  |  |
|  | Maintenance |  |  |  | X |  |  |  |
|  | Train Control |  |  |  |  |  |  | X |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line |  | X | X | X |  |  | X |
|  | Railway station, |  |  | X | X |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.7 (continued-4)

Parameter and usage of [XXX Systems]

| # |  | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | HOL | F | VTN | F | F | F | F |
| Specific name (if have) |  | Microwave | FH | Fixed radio | KVB | Eurobalise | Telepower to Eurobalise | Axle counter |
| Frequency Range (MHz) |  | 18000, 26000, 38000 | 12 000 (7 MHz bandwidth), 13 000 (3.5 MHz bandwidth), 22 000 (3.5 MHz bandwidth), 23 000 (3.5 MHz bandwidth), 26 000 (14 MHz bandwidth), 38 000 (7 MHz bandwidth) | 1427 –1 530 | 4,5MHz (DL) & 27 MHz (UL) | 0,984 – 7,484 MHz | 27.090 – 27.100 MHz |  |
| Radiocommunication Standards |  | ETSI EN 302 217-2-2 |  | ITU-R F.1242 | http://www.securite-ferroviaire.fr/reglementations/systeme-de-signalisation-de-classe-b-controle-de-vitesse-par-balises-kvb-equipemen-0 | Standard 1: ERTMS Subset 36 FIS for Eurobalise (URL): http://www.era.europa.eu/Document-Register/Pages/Set-3-FIS-for-Eurobalise.aspx | Standard 1: ERTMS Subset 36 FIS for Eurobalise (URL): http://www.era.europa.eu/Document-Register/Pages/Set-3-FIS-for-Eurobalise.aspx |  |
| Channel separation (kHz) |  |  |  | 4 000 |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  | H / V |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  | 4 000 |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  |  |  | PSK, QAM |  |  |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance |  |  |  |  |  |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.1.7 (continued-5)

Parameter and usage of [XXX Systems]

| # |  | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | F | F |  | F | F | F | RUS |
| Specific name (if have) |  | Level crossing Remote control | CCTV |  | IRIS | DPAR | FH | ASU VOP-2 (Automatic system for warning) |
| Frequency Range (MHz) |  | 465.1625 MHz / 455.1625 MHz (UL/DL) | 50 MHz |  | 410 – 414.50 MHz | 445.925 and 446.4625 MHz in a 55 km-radius circle around Nançay city 407.7 – 407.9 MHz elsewhere | 12 GHz with a 7 MHz bandwidth 13 GHz with a 3.5 MHz bandwidth 22 GHz with a 3.5 MHz bandwidth 23 GHz with a 3.5 MHz bandwidth 26 GHz with a 14 MHz bandwidth 38 GHz with a 7 MHz bandwidth | 876 – 880 921 - 925 |
| Radiocommunication Standards |  | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 |  |  | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 |  |  | GSM-R |
| Channel separation (kHz) |  | 25 |  |  | 25 |  |  | 200 |
| Antenna gain (dBi) |  | 0 – 14 |  |  | 0 – 14 |  |  | 2 … 20 |
| Polarization |  | vertical |  |  | vertical |  |  | V |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  | 39 … 46 |
| e.i.r.p. (dBm) |  | 2 – 17 |  |  | 2 – 17 |  |  | 41 … 66 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  | NA |
| Transmission data rate (kb/s) |  | 7,2 kbit/s for one TCH. up to 28,8 kbit/s when allocating up to 4 traffic channels to the same communication. |  |  | 7,2 kbit/s for one TCH. up to 28,8 kbit/s when allocating up to 4 traffic channels to the same communication. |  |  | NA |
| Transmission distance (km) |  | max 4 km |  |  | max 4 km |  |  | 5 |
| Modulation |  | π/4 DQPSK |  |  | π/4 DQPSK |  |  | GMSK |
| Multiplexing method |  | TDMA |  |  | TDMA |  |  | TDMA |
| Reception quality |  | NA |  |  | NA |  |  | NA |
| Applications | Voice/Dispatch | X |  |  | X | X |  |  |
|  | Maintenance | X |  |  |  | X |  |  |
|  | Train Control |  |  |  |  |  |  |  |
|  | Emergency | X |  |  | X | X |  |  |
|  | Train information | X |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  | X | X | X |  |
|  | Railway station, |  |  |  | X | X | X |  |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base | X |  |  |  |  |  |  |

Table 5.1.7 (continued-6)

Parameter and usage of [XXX Systems]

| # |  | 43 | 44 | 45 | 46 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | RUS | RUS | RUS | NMB |  |  |  |
| Specific name (if have) |  | SAI Palma (Automatic identification system) | ASU-D (Automatic control system – dispatcher) | ITARUS (Train separation system) | Fixed links |  |  |  |
| Frequency Range (MHz) |  | 865; 867; 869 | 876 – 880 921 - 925 | 876 – 880 921 - 925 | 451.05-452.15 461.05-462.150 |  |  |  |
| Radiocommunication Standards |  | ISO | GSM-R | GSM-R |  |  |  |  |
| Channel separation (kHz) |  | - | 200 | 200 | 25 |  |  |  |
| Antenna gain (dBi) |  | 5 | 0 | 2 … 20 |  |  |  |  |
| Polarization |  | NA | V | V |  |  |  |  |
| Transmitting radiation power (dBm) |  | 33 | 33 | 39 … 46 |  |  |  |  |
| e.i.r.p. (dBm) |  | 38 | 33 | 41 … 66 |  |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  | NA | NA | NA |  |  |  |  |
| Transmission data rate (kb/s) |  | NA | NA | NA |  |  |  |  |
| Transmission distance (km) |  | 0.005 | 10 | 5 |  |  |  |  |
| Modulation |  | NA | GMSK | GMSK |  |  |  |  |
| Multiplexing method |  | NA | TDMA | TDMA | FDMA |  |  |  |
| Reception quality |  | NA | NA | NA |  |  |  |  |
| Applications | Voice/Dispatch |  |  |  |  |  |  |  |
|  | Maintenance | X |  |  |  |  |  |  |
|  | Train Control |  | X | X |  |  |  |  |
|  | Emergency |  |  |  |  |  |  |  |
|  | Train information |  |  |  |  |  |  |  |
| Scenarios | Railway line |  | X | X |  |  |  |  |
|  | Railway station, | X | X | X |  |  |  |  |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base | X |  |  |  |  |  |  |

## 5.2	Radiocommunication systems used for Train positioning information

Table 5.2.1

Parameter and usage of Position Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | D | F | HNG | I | SUI | UAE | FIN |
| Specific name (if have) |  | Eurobalise Transmit Spectrum | Eurobalise | Eurobalise | Eurobalise | Eurobalise | Eurobalise | Eurobalise |
| Frequency Range (MHz) |  | 0.984-7.484 | 0.984-7.484 | 0.984-7.484 | 0.984-7.484 | 0.984 – 7.484 | 27 | 27.1 |
| Channel separation (kHz) |  |  |  |  |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  | horizontal |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  | 564 |  | 564.48 | 50 | 564 |  |  |
| Transmission distance (km) |  | 0.001 |  |  | 0.001 | 0.001 |  |  |
| Modulation |  | FSK | FSK（uplink) / AM（downlink) | ASK | FSK | FSK |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises | X | X | X | X | X | X | X |
|  | Loops/Leaky cable |  |  |  |  |  |  |  |
|  | Annunciators |  |  |  |  |  |  |  |
|  | Radar Scanners |  |  |  |  |  |  |  |
|  | Axle counters |  |  |  |  |  |  |  |
| Scenarios Applications | Railway line | X | X | X | X | X | X | X |
|  | Railway station | X | X | X | X | X | X | X |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.2.1 (continued-1)

Parameter and usage of Position Systems

| # |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | HOL | S | S | D | F | HNG | I |
| Specific name (if have) |  | Eurobalise | ATC Balise and Eurobalise | Automatic Train Control (ATC) On-board | Telepower to Eurobalise | Telepower to Eurobalise | Telepower to Eurobalise | Telepower to Eurobalise |
| Frequency Range (MHz) |  | 27.095, 3.951,4.515 | 4.5 | 27 | 27.09-27.10 | 27.09-27.10 | 27.09 - 27.10 | 27.09 - 27.10 |
| Channel separation (kHz) |  |  |  |  |  |  | 50 | 50 |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |
| Receiving noise figure (dB) |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  | 564 | 564.48 | 564.48 | 564 |
| Transmission distance (km) |  |  |  |  | 0.001 |  |  | 0.001 |
| Modulation |  |  | AFSK | AFSK | FSK |  |  | FSK |
| Multiplexing method |  |  |  |  |  | FSK（uplink)/AM（downlink) |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises | X | X | X | X | X | X | X |
|  | Loops/Leaky cable |  |  |  |  |  |  |  |
|  | Annunciators |  |  |  |  |  |  |  |
|  | Radar Scanners |  |  |  |  |  |  |  |
|  | Axle counters |  |  |  |  |  |  |  |
| Scenarios Applications | Railway line | X | X | X | X | X | X | X |
|  | Railway station | X | X | X |  | X | X | X |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.2.1 (continued-2)

Parameter and usage of Position Systems

| # |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | VTN | D | D | D | I | S | FIN |
| Specific name (if have) |  | EuroBalise-Balise | SOFIS balises | Radio-operated Approach Annunciator | Radar Scanner for Level Crossings | Radar Scanner for Level Crossing | Level crossing detection | Axle Counter |
| Frequency Range (MHz) |  | 984 – 7484 Editor’s note:the frequency 7484 need to be checked | 2450 ISM-Band (2400-2483.5; 75 MHz bandwidth) | 876 - 880, 921 – 925 | 76000-77000, 24125 | 9200 – 9700 | 77 000 | LF |
| Channel separation (kHz) |  |  |  |  |  |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  | 19 and 8.5 | theoretical achievable: 38.5 |  |
| Polarization |  |  | 13- 14 |  |  |  | vertical |  |
| Transmitting radiation power (dBm) |  |  | linear, horizontal |  |  |  | 37,5 dBm |  |
| e.i.r.p. (dBm) |  |  |  |  |  | 26 and 15 | Peak power on the antenna boresight: +38, Mean power: +15 |  |
| Receiving noise figure (dB) |  |  |  |  |  | 15 | 10 dB/ MHz |  |
| Transmission data rate (kb/s) |  |  | 1200 Baud – SOFIS LG S52830-B210-A6 1200 Baud – SOFIS MILG S52830-B210-A10 9600 Baud – SOFIS FZLG S52830-B210-A7 |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  | 0.08 | 0.5 |  |
| Modulation |  | FSK |  |  |  | PM |  |  |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises | X | X |  |  |  |  |  |
|  | Loops/Leaky cable |  |  |  |  |  |  |  |
|  | Annunciators |  |  | X |  |  |  |  |
|  | Radar Scanners |  |  |  | X | X | X |  |
|  | Axle counters |  |  |  |  |  |  | X |
| Scenarios Applications | Railway line | X | X | X | X | X | X | X |
|  | Railway station | X |  |  |  | X | X | X |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.2.1 (continued-3)

Parameter and usage of Position Systems

| # |  | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | F | D | I | CAN |  | VTN | RUS |
| Specific name (if have) |  | Axle Counter | Axle Counter | Axle counter | Proximity Detectors |  | EuroBalise-BTM | Russia “PRISMA-K” |
| Frequency Range (MHz) |  | none | 0.00506, 0.0098 ± 0.001, 0.027 bis 0.032, 0.2-0.045, 0.038-0.042, 0.043 ±0.001, 0.043 ±0.002,0.0745 ±0.0055, 0.325 ±0.02, 0.830 ± 0.04, 0.96 ±0.04, 1.130 ± 0.015, 1.035 ± 0.015,1 ± 0.001, 1.2288± 0.001 | 0.039, 0.050, 0.047, 0.053/0.073, 0.029, 0.031 | 160.17-161.58 |  | 27.09 – 27.10 | 76350 - 76650 |
| Channel separation (kHz) |  |  |  |  | 12.5 |  |  | 500000 |
| Antenna gain (dBi) |  |  |  |  |  |  |  | 50 |
| Polarization |  |  |  |  |  |  |  | Vertical |
| Transmitting radiation power (dBm) |  |  |  |  | Fixed Station:50.4, Mobile:47.8 |  |  | -20 |
| e.i.r.p. (dBm) |  |  |  |  | 45.1 |  |  | 30 |
| Receiving noise figure (dB) |  |  |  |  |  |  |  | <1 |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  | - |
| Transmission distance (km) |  |  |  |  |  |  |  | 0.5 |
| Modulation |  |  |  |  |  |  | CW | LFM |
| Multiplexing method |  |  |  |  |  |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises |  |  |  |  |  | X |  |
|  | Loops/Leaky cable |  |  |  |  |  |  |  |
|  | Annunciators |  |  |  |  |  |  |  |
|  | Radar Scanners |  |  |  |  |  |  | X |
|  | Axle counters | X | X | X |  |  |  |  |
| Scenarios Applications | Railway line | X | X | X |  |  | X |  |
|  | Railway station | X |  | X |  |  | X |  |
|  | Shunting yard |  |  |  |  |  | X |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.2.1 (continued-4)

Parameter and usage of Position Systems

| # |  | 29 | 30 | 31 | 32 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | KOR | CEPT | CEPT | CEPT |  |  |  |
| Specific name (if have) |  | Eurobalise | Eurobalise | Euroloop | Level crossing obstacle detection via radar sensor |  |  |  |
| Frequency Range (MHz) |  | 3.951-4.516 / 27.09-27.10 | Ground-to-train: 0.984-7.484 transmitting only on receipt of a telepowering signal from a train Train-to-ground: 27.09-27.10 | Ground-to-train: 7.3-23.0 transmitting only in presence of a train Train-to-ground: 27.09-27.10 | 76000-77000 |  |  |  |
| Channel separation (kHz) |  |  | ETSI EN 302 608 | ETSI EN 302 609 | ETSI EN 301 091-3 |  |  |  |
| Antenna gain (dBi) |  |  |  |  |  |  |  |  |
| Polarization |  |  |  |  |  |  |  |  |
| Transmitting radiation power (dBm) |  |  |  |  |  |  |  |  |
| e.i.r.p. (dBm) |  |  | Ground-to-train H-field: 9 dBµA/m at 10m Train-to-ground H-field: 42 dBµA/m at 10m | Ground-to-train H-field: -7 dBµA/m at 10m in a bandwidth of 10 kHz, spatially averaged over any 200m length of the loop Train-to-ground H-field: 42 dBµA/m at 10m | 50 dBm average power or 23.5 dBm average power for pulse radar |  |  |  |
| Receiving noise figure (dB) |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  |  |  |  |  |  |  |
| Transmission distance (km) |  |  |  |  |  |  |  |  |
| Modulation |  |  |  |  |  |  |  |  |
| Multiplexing method |  |  | Ground-to-train: FSK Train-to-ground: CW or binary ASK | Ground-to-train: DSSS-BPSK; code length: 472 chips | pulse, CW, FM or FSK |  |  |  |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Balises | X | X |  |  |  |  |  |
|  | Loops/Leaky cable |  |  | X |  |  |  |  |
|  | Annunciators |  |  |  |  |  |  |  |
|  | Radar Scanners |  |  |  | X |  |  |  |
|  | Axle counters |  |  |  |  |  |  |  |
| Scenarios Applications | Railway line | X | X | X | X |  |  |  |
|  | Railway station | X | X | X |  |  |  |  |
|  | Shunting yard |  |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

## 5.3	Radiocommunication systems used for Train remote systems

### 5.3.1	Shunting System

Table 5.3.1

Parameter and usage of Shunting Systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | AUS-Queensland | NOR | S | D | RUS | RUS | RUS |
| Specific name (if have) |  | Shunting Radio | Analogue UHF radio – Shunting radio | Shunting | Data service for automatic train composition | MALS/GALS | MSR-32 | SDU ML |
| Frequency Range (MHz) |  | 403-420 | 36 channels in the 450-470 MHz range | 460 | 167.475, 167.600, 467.900, 468.100, 468.225, 468.250, 458.275 | 151.7125 – 156.0125 457.4 – 458.45 467.4 – 468.45 876 – 880 921 - 925 | 400 - 470 | 151.7125 – 156.0125 |
| Radiocommunication Standards |  | PMR→ Tier 3 DMR |  | n/a | EN 50239:1999, ETSI EN 300 113, ETSI EN 300 220 | GOST-12252-86 RUS | Siemens | NA |
| Channel separation (kHz) |  | 25 | 25 |  | 25 | 25 | 25 | 25 |
| Antenna gain (dBi) |  |  |  |  |  | 2 …15 | 2 …15 | 2 …15 |
| Polarization |  |  |  |  | vertical | V | V | V |
| Transmitting radiation power (dBm) |  |  | 37 |  | 27 (standard), optional up to 33 | 40 … 45 | 40 | 40 |
| e.i.r.p. (dBm) |  |  |  |  |  | 42 …60 | 42 …55 | 42 …55 |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  | NA | NA | NA |
| Transmission data rate (kb/s) |  | 9.6 |  |  | 4.8 | NA | NA | NA |
| Transmission distance (km) |  |  |  |  | < 1 | 10 … 12 | 5 | 10 |
| Modulation |  | 4FSK |  |  | F3D | DRCMSK π/4 DQPSK | MSK | DRCMSK |
| Multiplexing method |  | TDMA |  |  | TDMA | NA | NA | NA |
| Reception quality |  |  |  |  |  |  |  |  |
| Applications | Shunting | X | X | X | X | X | X | X |
| Scenarios | Railway line |  |  |  |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard | X | X | X | X | X | X | X |
|  | Maintenance Base |  |  |  |  |  |  |  |

Table 5.3.1 (continued-1)

Parameter and usage of Shunting Systems

| # |  | 8 | 9 | 10 | 11 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | VTN | F | F | F |  |  |  |
| Specific name (if have) |  | Shunting Radio | Analog RLE (Radio Local Enterprise) | Digital RLE | Digital RLE (Radio Local d'Entreprise) Direct Mode |  |  |  |
| Frequency Range (MHz) |  | 403-450 | 150 160-164 165-173 444-445 457-468 | 407-470 | 400 MHz 465.1625 MHz / 455.1625 MHz (UL/DL) |  |  |  |
| Radiocommunication Standards |  | PMR, TETRA | Standard 1: UIC Code 751-3 “Technical regulations for international analogue ground-train radio systems” | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 | TETRA normes ETSI EN 300 392-1 and EN 300 392-2 |  |  |  |
| Channel separation (kHz) |  | 12.5 / 25 | 12.5 | 25 | 25 |  |  |  |
| Antenna gain (dBi) |  |  | 2,41-7,41 | 0-14 | 0 - 1 |  |  |  |
| Polarization |  |  | vertical | vertical | vertical |  |  |  |
| Transmitting radiation power (dBm) |  |  |  | UE :32 Mobile : 35 BTS : 32 - 35 | TBD |  |  |  |
| e.i.r.p. (dBm) |  |  | 3,41 - 12,41 | 2-1740 | NA |  |  |  |
| Receiving noise figure (dB) Fixed Station/Mobile/Portable |  |  |  |  |  |  |  |  |
| Transmission data rate (kb/s) |  |  | NA | 7,2 kbit/s for one TCH. up to 28,8 kbit/s when allocating up to 4 traffic channels to the same communication. | 7,2 kbit/s |  |  |  |
| Transmission distance (km) |  |  | 10 km | max 4 km | 0,8 - 1,5 |  |  |  |
| Modulation |  | FM / π/4 DQPSK | AM | π/4 DQPSK | π/4 DQPSK |  |  |  |
| Multiplexing method |  | TDMA | FDMA | TDMA | TDMA |  |  |  |
| Reception quality |  |  | NA | NA | NA |  |  |  |
| Applications | Shunting | X |  | X (voice) | X |  |  |  |
| Scenarios | Railway line |  |  | X |  |  |  |  |
|  | Railway station, |  |  |  |  |  |  |  |
|  | Shunting yard | X |  |  | X |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |
|  |  |  |  | X |  |  |  |  |
|  |  |  |  | X |  |  |  |  |
|  |  |  |  | X |  |  |  |  |
|  |  |  |  | X |  |  |  |  |

## 5.4	Radiocommunication systems used for Train surveillance

### 5.4.1	Surveillance System

Table 5.4.1

Parameter and usage of Surveillance systems

| # |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use by which Administration(s) |  | VTN | KOR | J | J | F | D |  |
| Specific name (if have) |  | CCTV |  | MVT/TRS-40GHz | PMS | CCTV | Closed Circuit Television Systems (CCTV) & Public Address Systems (PA) |  |
| Frequency Range (GHz) |  | 12.75–13.25 | 18.86-18.92 19.20-19.26 | 43.5 – 45.5 | 57 - 66 | 0.05 | 5.470 - 5.725, 5.725 - 5.875 |  |
| Radiocommunication Standards |  | ITU-R F.497 |  | NA |  |  | IEEE 802.11 n |  |
| Channel separation (kHz) |  | 28 000 | 6x10 000 | 40 000 | 125 000 |  |  |  |
| Antenna beamwidth (degree) |  | 15 | NA | BS:3 or 9 MS:23 | BS: 3.5 MS: 7 |  |  |  |
| Antenna gain (dBi) |  | 18 | NA | 33typ | BS:31 MS:26 |  |  |  |
| Polarization |  |  | NA | Vertical | Linear |  |  |  |
| Transmitting radiation power (dBm) |  | -3 | 100 | 0 | +10 |  |  |  |
| e.i.r.p. (dBm) |  |  | NA | 33typ | BS:+41 MS:+33 |  | Up to 10 |  |
| Receiving noise figure (dB) |  |  | NA | <20 | <8 |  |  |  |
| Transmission data rate (Mb/s) |  |  | NA | NA | 100 |  |  |  |
| Transmission distance (km) |  |  | 1.5-2.5 | 0.06 | 0.1 |  | Up to 1 |  |
| Modulation |  | FM | OFDM | FM | ASK |  |  |  |
| Multiplexing method |  |  | NA | FDM | FDD |  |  |  |
| Reception quality |  |  | NA | NA |  |  |  |  |
| Applications | CCTV:platform view/crossings | X |  | X | X | X |  |  |
|  | CCTV:coach/compartments | X |  |  |  | X |  |  |
|  | CCTV:device monitoring | X |  |  |  |  |  |  |
| Scenarios | Railway line | X |  |  |  | X |  |  |
|  | Railway station, |  |  | X | X | X |  |  |
|  | Shunting yard | X |  |  |  |  |  |  |
|  | Maintenance Base |  |  |  |  |  |  |  |

# 6	Considerations on evolving technologies for RSTT including technical and operational characteristics of future RSTT

As mentioned by the Resolution 236 (WRC-15), railway transportation systems are evolving and some national and international railway organizations have begun investigations on new technologies for railway radiocommunication systems. Future RSTT in some countries might progressively evolve from analogue to digital or broadband systems introduced in addition to existing technologies in operation.

Some administrations provided information on their planned activities for the future RSTT through responses to the ITU questionnaire and contributions to WP 5A. This information has been compiled as section 6.1. During the study, one administration also provided their detailed parameters of planned RSTT system in 90 GHz band for future super high speed railway systems, which had been placed into section 6.2 of this Chapter.

It could be seen from the tables in section 6.1 and 6.2, that Administrations had made some plans for the migration of RSTT, to meet the requirements of high-speed railway transportations and to adaptive to more complex environment of railway radiocommunicaitons. Generally, the analogue RSTT systems are likely to be replaced by digital systems for RSTT, by using technologies such as TETRA, GSM-R, DMR, LTE-based, and even some millimetric wave technologies. There is also a trend to replace GSM-R with next generation technologies, with the concern that the GSM-R might end-of- service in year of 2030. Some Administrations have been carrying out field tests to verify their technical migrations for RSTT. Meanwhile, the UIC has also been carrying out some projects for the next generation railway radiocommunication which might have deep impact to the railway industry and some administrations.

## 6.1	Considerations on evolving technologies for RSTT

In response to the question 3 of the ITU questionnaire (5/LCCE/60), some administrations provide the following considerations on the evolving technologies for RSTT.

[Editor’s note: Administrations are encouraged to contribute to this section with information on their planned migrations of RSTT.]

| Countries | Answers |
| --- | --- |
| Australia | Train control radio and maintenance radio in South east Queensland currently migrating to Tier 3 DMR. |
| Canada | No. Some Train control technology might be implemented in the future, in addition to the existing systems. |
| China | Yes. China is planning to migrate the system. A field test related to LTE-based next generation railway radiocommunication system is planned to be carried out in 2018 on some high speed railway line to verify system capacity and technical characteristics for RSTT in different typical scenarios. |
| Czech Republic | No |
| Finland | For GSM-R Yes, we are planning to migrate GSM-R voice communication to PPDR's existing TETRA system and partly to public mobile network. For hot box detector, No at the moment but about in five years some 50 hot box detector will be replaced. Other: Not at the moment |
| France | GSM-R to a new technology to be defined by 2022 at EU level Analog radio to be suppressed in 2018 Digital RLE to possibly LTE in 202x Analog RLE to be suppressed in 2018 and beyond CCTV 50MHz to be gradually replaced by TVSE-EAS-NG (10 GHz) Radio band for CBTC to be defined |
| Germany | Continuous Automatic Train Running Control to ETCS until 31.12.2030. Eurobalises until 31.12.2021 (with the same frequency utilisation) |
| Hungary | SEE the NOTE under this table |
| Iraq | Yes |
| Italy | In the forthcoming years RFI (Italian railway infrastructure manager) has a plan to extend the implementation of ERTMS L2 not only to the new High Speed lines, but also to Conventional lines which will be part of European corridors and to the main Urban node lines. |
| Japan | Japan has been facilitating the digitalization of train radio system. And Japan is interested in the RSTT to provide the high-speed data to the train crews and passengers from the train communication networks. So, Japan has begun to study millimetre-wave band radiocommunication systems for railway systems to provide high-speed data to the train crews and passengers to realize more secure and comfortable railway transport services. |
| Republic of Korea | Korea is planning to migrate the system from VHF and TRS to LTE in RSTT |
| The Netherlands | GSM-R:Future spectrum options for GSM-R to consider are:Extension of the UIC band with: UL:873-876 MHz; DL:918-921 MHz; UL:698-703 MHz, DL:753-758 MHz and/or UL:733-736 MHz, DL:788-791 MHz ProRail is planning to replace the GSM-R system when it has reached “end of life” around 2030. The successor of GSM-R is unknown at the moment and is currently defined by the FRMCS (Future Radio Mobile Communications System) program under the UIC. − Microwave :ProRail wants to reduce the number of microwaves in the GSM-R network and replace them with fixed fiber connections. − EuroBalises: The number of Eurobalises will grow significantly the coming year when ERTMS is further deployed in the Netherlands. No replacement foreseen.Marine VHF Radio: The Marine VHF Radio is used for a small number of bridges. No replacement foreseen. |
| Norway | To use GSM-R for shunting radio and phase out the UHF based shunting radio services are currently under consideration. The current license for use of UHF channels expire by 31. December 2016. Jernbaneverket who operates the GSM-R network have not yet adopted a formal position on the future of GSM-R and what technology to deploy after GSM-R. |
| Qatar | Not for the time being as we are green field project. However, migration to LTE will be assessed once technology is fully standardized and matured in rail environment |
| Russia | It is planned to migrate a number of VHF-band analogue radiocommunication networks to work with the DMR, GSM-R and LTE-standard radio interface |
| Spain | − Analog radio (Tren-Tierra) will be migrated to GSM-R. − GSM-R will be migrated in the future to a new broadband radiocommunication system. |
| Sweden | − Radio link – GHz-band; − Tetra – 378 MHz; − GSM R/ERTMS – 925 MHz; − FRMCS – tbd |
| Switzerland | − GSM-R:the successor system of GSM-R (FRMCS) will originally be operated in 7 MHz of the 873-876 / 876-880 MHz and 918-921 / 921-925 MHz band. The services of GSM-P in 2G and 3G for non-critical railway applications will be migrated by 4G/LTE in 800 MHz, 1 800 MHz, 2 100 MHz and 2 600 MHz. − Radio System: FRMCS (working title) |
| Thailand | considering the possibility and appropriateness of the frequency band 800/900 MHz to support the introduction of GSM-R/LTE-R systems. Possible options of frequency ranges include: − 885-895 MHz and 930-940 MHz; − 876-880 MHz and 921-925 MHz ; − Other bands, if the above are not applicable. |
| United States | PTC implementation was originally scheduled to be completed by 2015; however, the completion for installation date has been moved to 31 Dec, 2018. In certain cases, the Federal Railroad Administration FRA has the authority to grant an extension of up to 2 additional years. |
| United Kingdom | − No programme is in place to replace either system, although a proposal to deploy GPRS over GSM-R is under review to support European Rail Traffic Management System (ERTMS) roll-out. − RETB is deployed on two lines of route (Far North and West Highland) and the system supports train control (RETB) and driver to signaler voice communications. For areas other than the Far North and West Highland lines, where GSM-R is fully deployed, Network Rail will review technological approaches which could eventually replace GSM-R. Network Rail brings this thinking to its presence on the UIC FRMCS and Shift2Rail Adaptable Communication System work programmes. |
| Vietnam | Not yet |
| Netherlands (ProRail) | − GSM-R: ProRail is planning to replace the GSM-R system when it has reached “end of life” around 2030. The successor of GSM-R is unknown at the moment and is currently defined by the FRMCS (Future Radio Mobile Communications System) program under the UIC. − Microwave: ProRail wants to reduce the number of microwaves in the GSM-R network and replace them with fixed fiber connections. − EuroBalises: The number of Eurobalises will grow significantly the coming year when ERTMS is further deployed in the Netherlands. No replacement foreseen. − Marine VHF Radio: The Marine VHF Radio is used for a small number of bridges. No replacement foreseen. |

Note: For Hungary：

| Railway radio network migration strategy targets |  |
| --- | --- |
| Old system | New system after the migration |
| 160 MHz and 450 MHz corridor lines which are covered with GSM-R I. phase | GSM-R I. phase |
| 160 MHz main lines | GSM-R I. and II. Phase04 |
| 160 MHz regional lines | UIC 751-3 450 MHz system03 |
| 160 MHz local technology comm.01 | 450 MHz local technology comm.02 |
| 450 MHz main lines | GSM-R I. and II. phase04 |
| 450 MHz regional lines | no planned migration03 |
| 450 MHz local technology comm.01 | no planned migration |
| 01 According to the first trial tests made with GSM-R it’s likely that the local technology communications (shunting) cannot not be migrated to GSM-R, due to the high reaction times ( between channel request and uplink grant or reject messages) and very limited handheld equipment support for this technology. 02 The channel raster of the cab radios (locomotive radios) is limited to 25 kHz in the 450 MHz range due to the fact that every cab radio is developed according to the UIC 751-3 standard. Cab radios used in the railway sector will not going to support 12,5 KHz channel raster. In the public sector this raster is denser (12,5 KHz), which could limit the migration plans from 160 MHz local technology systems and also might limit the usage of the 450 MHz local technology communication systems (no other system which is supported by cab radios is available for this purpose). 03 On regional lines where the installation of GSM-R is not financially feasible, probably we will use cost effective 450 MHz UIC 751-3 train radio systems. 04 On main lines after the completion of the GSM-R I. and II. phase we will be able to migrate from the existing 160 MHz and 450 MHz analog train radio systems to GSM-R. In Hungary we will migrate the rolling stock to dual mode 450 MHz / GSM-R cab radios to be able to handle the above mentioned requirements. |  |

## 6.2	Radiocommunication systems to be used for Train surveillance

Table 6.2-1

Parameter and usage of Surveillance systems

|  |  | 90-GHz band |
| --- | --- | --- |
| Use by which Administration(s) |  | J |
| Specific name (if have) |  | [TBD] |
| Frequency Range (GHz) |  | 92-94. 94.1-100, 102-109.5 |
| Radiocommunication Standards |  | NA |
| Channel separation (MHz) |  | 400 |
| Antenna beamwidth (degree) |  | 1 |
| Antenna gain (dBi) |  | 44 |
| Polarization |  | linear |
| Transmitting radiation power (dBm) |  | 10 |
| e.i.r.p. (dBm) |  | 54 |
| Receiving noise figure (dB) |  | <10 |
| Transmission data rate (Gb/s) |  | 5-10 (Stationary), 1 (Running) |
| Transmission distance (km) |  | 0.5-1 (Open), 3 (Tunnel) |
| Modulation |  | PSK, QPSK, 16QAM, 64QAM |
| Multiplexing method |  | FDD/TDD |
| Reception quality |  | NA |
| Max. operational speed (km/h) |  | 600 |
| Applications | CCTV:platform view/crossings | X |
|  | CCTV:coach/compartments | X |
|  | CCTV:device monitoring | X |
| Scenarios | Railway line | X |
|  | Railway station, | X |
|  | Shunting yard |  |
|  | Maintenance Base | X |

# 7	Spectrum Usage of RSTT

## 7.1	Current spectrum usage of RSTT

### 7.1.1	Radiocommunication systems used for Train radio

#### 7.1.1.1	Overview of current spectrum usage of Radiocommunication systems used for Train radio

The following figure shows current spectrum usage of Radiocommunication systems used for Train radio.

FIGURE 7.1.1.1-1

Overview of current spectrum usage of Radiocommunication systems used for Train radio.

[Editor’s Note: Trunked Radio to be added to this Table.]

[The table below presents a summary of the data provided by the administrations for Train Radio applications. The results show that most Train Radio applications currently use the sub 1 GHz frequency bands, and no Train Radio application uses frequency bands over 40 GHz.]

[Table 7.1.1.1

| Freq Bands of Train Radio Applications | Num of App |
| --- | --- |
| <1 000 MHz | 226 |
| >1 000 & <= 6 000 MHz | 21 |
| >6 000 & <= 20 000 MHz | 5 |
| >20 000 & <= 40 000 MHz | 6 |
| >40 000 & <= 100 000 MHz | 0 |
| Total Counted Num of App | 258 |

]

[Editor’s Note: Request to consider a column including the number of administrations and explanatory text for this Table, Administrations are invited to provide a new proposal for this table.]

[Editor’s Note: Same analysis for the other categories to be included.]

#### 7.1.1.2	Segment view of current spectrum usage of Radiocom.]munication systems used for Train radio

FIGURE 7.1.1.2-1

0~1 000 MHz

FIGURE 7.1.1.2-2

1 GHz~6GHz

FIGURE 7.1.1.2-3

6GHz~20GHz

600~1 000 MHz

(uplink 718-728 MHz, downlink 773-783 MHz)

FIGURE 7.1.1.2-4

[20 GHz~100 GHz]

[Editor’s Note: All tables will be revised according to the RR Article 2.]

### 7.1.2	Radiocommunication systems used for Train positioning information

The following figure shows current spectrum usage of Radiocommunication systems used for Train positioning information.

FIGURE 7.1.2-1

### 7.1.3	Radiocommunication systems used for Train remote systems

The following figure shows current spectrum usage of Radiocommunication systems used for Train remote systems.

FIGURE 7.1.3-1

### 7.1.4	Radiocommunication systems used for Train surveillance

The following figure shows current spectrum usage of Radiocommunication systems used for Train surveillance.

FIGURE 7.1.4-1

## 7.2	Planning spectrum usage of RSTT

| Systems’ name | Spectrum usage in Region 3 (Member States amount: 1 ) |  |  |  |
| --- | --- | --- | --- | --- |
| 90-GHz RBH |  | Freq From (GHz) | Freq To (GHz) | Member State amount Subtotal |
|  | Subtotal | 92 | 109.5 | 1 |
|  | Group 1 | 92 | 94 | 1 |
|  |  | 94.1 | 100 | 1 |
|  |  | 102 | 109.5 | 1 |

## 7.3	Spectrum needs of RSTT

### 7.3.1	Spectrum needs of RSTT with respect to the train radio application

One study on the spectrum needs of RSTT with respect to the train radio application was conducted by China. With the consideration that a future integrated system will realize most of the main applications of RSTT, and that voice, data and messaging are implemented using mobile IP service, this study, implementing the methodology introduced by Recommendation ITU-R M.1768-1, calculated the spectrum needs with respect to train radio of RSTT in China by 2020. During the study, the XIN FENG ZHEN area (3.2 km × 0.45 km) located in Xi’an City of China was considered. Since the area of XIN FENG ZHEN is 1.6 km2 and the shunting yard, line, station and maintenance base are covered in this deployment scenario, so when calculating the spectrum needs of RSTT with respect to the train radio application, the simplified condition (one cell coverage) was considered. The spectrum efficiency parameter were based on a field test and adjusted accordingly for different speeds and environments. For more detailed information, please refer to Annex 7 to this Report.

Under the conditions of this study, spectrum needs of RSTT with respect to Train radio application for the year 2020 at XIN FENG ZHEN area is:

–	Minimum spectrum needs: 11.9 MHz (uplink), 4.7 MHz (downlink);

–	Maximum spectrum needs: 14.04 MHz (uplink), 8.38 MHz (downlink).

Further, when deploy the RSTT network, more factors may need to be taken into consideration and some measures would be introduced to further improve the spectrum usage efficiency, such as multi-cell layout in some of the deployment scenarios of RSTT.

### [7.3.2	Example of spectrum needs of future 90-GHz RSTT with respect to the train surveillance

The number of train vehicles daily operated in Japanese high-speed (super express) train depends on Japan Railway companies, however, the maximum train vehicles of 16 is used for estimation of spectrum needs of 90-GHz band. The following parameters are assumed for calculation of total bandwidth:

Number of train vehicles: 16

Number of video monitoring cameras in a train vehicle: 7

Total number of video monitoring camera in one super express train set: 112

Estimated transmission bit rate of one super express train set: 198 Gb/s (1280×960), 55.7 Gb/s (720×480), 49.5 Gb/s (640×480), 27.8 Gb/s (720×240)

Bit rate per channel: 0.5 Gb/s (QPSK), 1 Gb/s (16QAM), 1.5 Gb/s (64QAM)

Number of channels: 17 (lower) at 92-94 GHz and 94.1-100 GHz, 17 (upper) at 102-109.5 GHz

Spectrum need: 16.4 GHz

The total data rate of 51 Gb/s whose pixels is 640×480 can be transmitted using both lower and upper channels. If the monitoring video can use a low-resolution camera, the additional information except surveillance can be transmitted using the same channel arrangement.]

# 8	Differing Deployment and Operations Approaches

The deployment and operation of railway systems is based on different approaches in some countries, depending on respective national policies and/or local environments. Such alternative approaches (sometimes called ‘operating models’) involve private (or commercial) train operators, private rail track owners, and other non-government entities providing services such as communications, signalling and train control. These may involve such scenarios as private/commercial train operators using/sharing state-owned rail tracks, state-owned trains using privately-built rail tracks (eg. under so-called ‘build-own-operate’ infrastructure financing arrangements), and sub-contracted or out-sourced arrangements with commercial companies for provision of wired/wireless communications systems, signalling systems, and even train-control and scheduling services. These alternative deployment and operations approaches are illustrated in the following diagram.

For convenience, many of these rail operations arrangements use the terms ‘below-the-rail’ and ‘above-the-rail’ to distinguish between: the rail infrastructure segments that are fixed/stationary (such as rail track, stations/sidings, signalling equipment/towers, communications infrastructure, gates/barriers, and other trackside elements); and the locomotives, train carriages/cars/trucks (ie. rolling stock), that travels along the rails; respectively.

Figure XX

Illustration of deployment and operations options

Below-the-rail infrastructure provider(s)

Infrastructure Manager(s)

Train Controller(s)

Application & Information servers

Radiocommunications provider(s)

Above-the-rail train operator(s)

Examples of such alternative approaches can already be seen in urban passenger rail (metro) systems, suburban and regional rail lines, mine-to-port mineral transportation systems, some of the larger trans-continental rail routes, scenic and tourist railways, and other scenarios. Many countries foresee such arrangements continuing, and possibly expanding, in the future, for reasons of relieving national fiscal budgets, improving operational flexibility, and encouraging wider private economic investment.

# 9	Summary of the study

ANNEX 1

Summary of response to the Questionnaire on the usage of Railway Radiocommunication Systems

## A1.1	Introduction

WRC-15 adopted Resolution 236 (WRC-15), which invites ITU-R to study the spectrum needs, technical operational characteristic and implementation of railway radiocommunication systems between train and trackside. Resolution 809 (WRC-15) decided that the agenda item 1.11 of WRC-19 is ‘to take necessary actions, as appropriate, to facilitate global or regional harmonized frequency bands to support railway radiocommunication systems between train and trackside within existing mobile service allocations, in accordance with Resolution 236 (WRC-15)’.

The technical and operational characteristics used by railway radiocommunication systems vary by country. In order to get a clear overview on these systems, it is necessary to summarize the technical and operational characteristics for railway radiocommunication systems between train and trackside for traffic control, passenger safety, and security for train operation. ITU-R Working Party 5A is currently studying the usage of railway radiocommunication systems. Gathering the information on the usage of such systems would facilitate the preparation for WRC-19 agenda item 1.11.

On 3rd June 2016, The Radiocommunication Bureau invited by Administrations of Member States of the ITU to provide answers to questions by Administrative Circular 5/LCCE/60. Response to the Questionnaire from each Administration of Member States is compiled in the document 5A/273R2.

The Radiocommunication Bureau had received responses from 25 Administrations and one ITU regional organization by 31 October 2016. Response to the Questionnaire from each Administration of Member States had compiled in the Document 5A/273(Rev.2). In its MAY 2017 meeting of WP 5A, 3 responses to the questionnaire from Iraq, Russian Federation and United States of America were received (5A/323R2). Additionally, Australia provided more info to the questionnaire through contribution 5A/405. For its NOVEMBER 2017 meeting of WP 5A, 21 African Administrations provided a common statement wherein 9 Administrations (Zambia, Malawi, Tanzania, South Africa, Mozambique, Namibia, Angola, Botswana, Swaziland) provided detailed information (5A/612).

[Editor’s note: the references to input document in this report needs to be deleted since it is allowed to refer to input document numbers in an adopted ITU Report, and further improvements for these paragraphs are needed accordingly.[]

## A1.2	Summary of the situation of feedback of the questionnaire

Radiocommunication Bureau has received responses from 25 Administrations and one ITU regional organization by 31 October 2016, as follows.

Armenia

Australia

Bosnia-Herzegovina

Canada

China

Czech Republic

Finland

France

Germany

Hungary

Italy

Japan

Republic of Korea

Malta

Netherlands

Norway

Qatar

Spain

Sweden

Switzerland

Thailand

United Arab Emirates

United Kingdom

Uzbekistan

Viet Nam

CEPT

In its MAY 2017 meeting of WP 5A, 3 responses to the questionnaire from Iraq, Russian Federation and United State were received (5A/323R2). So, total number of responses from Member States is 28. Additionally, Australia provided more info to the questionnaire through contribution 5A/405.

## A1.3	Summary table of answers to each question

The answers to the questionnaire are compiled and summarized to the excel format table is attached below.

Note:

1	The above Excel sheet is only the combination of received responses provided Administrations and Regional Organization, by May 2017 as well.

2	In its MAY 2017 meeting of WP 5A, some Administrations provided technical and operational parameters/characteristics of currently used RSTT systems into tables within the main body of the report but did not change their original answers to the questionnaire. Therefore, it is important to use the up-to-date information contained in the Chapter 4.

## A1.4	Original questionnaire answers

The original detailed information in 5A/273(Rev.2), 5A/323(Rev.2), 5A/405 and 5A/612is compiled as follows.

ANNEX 2

RSTT in Japan

## A2.1	Overview

The frequency 100 kHz-band, 150 MHz-band, 300 MHz-band and 400 MHz-band have been allocated to the RSTT for safety and stable train operation since 1960s. Regarding the requirement to the conventional RSTT, the broadband transmission capability has become one of important functions to provide high-speed data to the train crews and passengers to realize more secure and comfortable railway transport services. To realize these requests, digital train radio systems have been introduced to the high speed railway. And now many digital radio systems are used for RSTT. Furthermore, Japan has begun to study millimetre-wave. Microwave as well as millimetre-wave frequencies are well known as the spectrum resources supporting the broadband data signal transmission.

### A2.1.1	List of the RSTT

Table A2.1.1-1 is the list of major RSTT used in Japan. This table shows name of system, frequency band, applications, and users of each RSTT.

Table A2.1.1-1

| Name of System | Frequency | Applications and Users of the system |
| --- | --- | --- |
| Train Radio System (TRS) | 150 MHz band 300 MHz band 400 MHz band | Application ･Traffic control information for drivers ･Automatic train control ･Vehicle status monitoring for maintenance crews ･Passenger guidance for conductors Users ･Train traffic controllers ･Train drivers and conductors ･Automatic train control equipment ･Station managers ･Maintenance crews |
| Radiocommunication system for High Speed Train (RHST) | 400 MHz band | Application ･Traffic control ･Automatic train control ･Vehicle status monitoring, Passenger guidance, Cabin monitoring Users ･Train traffic controllers ･Train drivers and conductors ･Automatic train control equipment ･Maintenance crews |
| Emergency Alarm Radio System (EARS) | 300 MHz band | Application ･Emergency signals from train or ground to trains to alert some dangers situations to surrounding drivers by buzzer Users ･Train drivers and conductors ･Train traffic controllers ･Station attendants |
| Radiocommunication system for Emergency Cut Off System (REMCOS) | 150 MHz band | Application ･Emergency signal from train to ground to stop trains by powering Cut Off Users ･Train drivers and conductors ･Train traffic controllers ･Ground maintenance crews |
| Radiocommunication system for Electronic Blocking System (REBS) | 300 MHz band | Application ･Trigger signal transmission from train to ground to control block section Users ･Train drivers ･Ground Interlocking equipment |
| Radiocommunication system for Japan Radio Train Control system (JRTC Radio) | 300 MHz band | Application ･Automatic train control Users ･Ground Train controller equipment ･On-board train controller equipment |
| Yard Radio (YR) | 150 MHz band 300 MHz band 400 MHz band | Application ･Vehicle maintenance ･Shunting Users ･Train drivers ･Ground maintenance crews |
| Millimetre wave Video Transmission system (MVT) | 43 GHz band | Application ･Platform Monitoring Users ･Train drivers and conductors |

### A2.1.2	Frequency usage

Table A2.1.2-1 shows details of frequency usage of RSTT used in Japan. This table also shows frequency usage of future RSTT under study.

Table A2.1.2-1

Frequency usage of RSTT in Japan

| Frequency bands | Systems |
| --- | --- |
| 140-144 MHz, 146-148 MHz, 148-149.9 MHz | Train Radio System, Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 150.05-156.4875 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 156.8375-160 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 335.4-340 MHz | Train Radio System, JRTC radio, Radiocommunication system for Electronic Blocking System, Yard Radio |
| 340-360 MHz | Train Radio System, JRTC radio, Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 360-370 MHz | Radiocommunication system for Emergency Cut-Off system, Yard Radio |
| 370-380 MHz | Emergency Alarm Radio System, Yard Radio |
| 380-387 MHz, 387-390 MHz, 390-399.9 MHz | Yard Radio |
| 410-420 MHz | Radiocommunication system for High Speed Train, |
| 414.4-415.5 MHz | Train Radio System |
| 450-455 MHz | Radiocommunication system for High Speed Train |
| 455-456 MHz, 456-459 MHz, 459-460 MHz, 460-470 MHz | Yard Radio |
| 43.5-47 GHz | Millimetre wave Video Transmission system, Future radio communication system for high speed trains (under study) |
| 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz | Future radio communication system for high speed trains (under study) |

### A2.1.3	Operational environment

Radio propagation characteristics in railway environment depend on the type of track surfaces (e.g. ballast track, concrete slab track, etc.).

Table A2.1.3-1 and Table A2.1.3-2 shows typical railway alignment, size of propagation area and location of antennae for RSTTs. Figure A2.1.3-1 shows an example of cross section view of railway track.

Table A2.1.3-1

Operational environment (deployment scenarios) in inter-city railway lines or urban transport systems

| Parameters | Values |
| --- | --- |
| Antenna height | Base station : [5 m - 15 m] from rail surface level Mobile station : [3.5 m - 4 m] from rail surface level |
| Maximum train speed | 160 km/h |
| Roadbed width | Typ. [10] m |
| Vehicle width | Max. 3.0 m |
| Vehicle height | Max. 4.1 m |
| Minimum Radius of curve | Typ. 2 200 m (Min. 400 m in railway station area) |
| Minimum Vertical radius of curve | Typ. 2 000 m |
| Gradient of track | 35‰ |
| Maximum Superelevation of track | 105 mm @ rail gauge = 1 067 mm |

Table A2.1.3-2

Operational environment (deployment scenarios) in high speed railway line

| Parameters | Values |
| --- | --- |
| Antenna height | Base station(LCX) : [0 m - 3 m] from rail surface level Mobile station : [0.6 m - 1 m] from rail surface level |
| Maximum train speed | 320 km/h |
| Roadbed width | Typ. [12] m |
| Vehicle width | Max. 3.4 m |
| Vehicle height | Max. 4.5 m |
| Minimum Radius of curve | Typ. 4 000 m |
| Minimum Vertical radius of curve | Typ. 10 000 m |
| Maximum Gradient of track | 35‰ |
| Maximum Superelevation of track | 200 mm @ rail gauge = 1 435 mm |

FIGURE A2.1.3-1

An example of cross section view of railway track (viaduct section)

## A2.2	150 MHz band RSTT

### A2.2.1	Train Radio System

#### A2.2.1.1	System architecture

Train Radio System (TRS) is used for inter-city and inner-city train, but not for high speed train. TRS carries traffic control information, train control command, passenger information and vehicle status monitoring data between trains and control centres. In general, a control centre covers several railway lines and TRS accommodate some radio zones, which correspond to each railway line.

Figure A2.2.1.1-1 shows the architecture of TRS. A set of radio frequencies is allocated to each line. There are some base stations in a zone, about 1 km - 40 km each according for propagation scenarios. The Central System connects commanders in the Control Centre and crews on-board. The commanders are able to inform drivers about train control issues. The controllers are also able to inform conductors about passenger guidance. Furthermore, data transmissions for vehicle status monitoring are available.

On-board antennas are on the top of each side of diver’s room. Base station antennas are on the top of poles beside the track and directing the rail along. In some train lines, the system is applied not only for voice and data communications but also for the train control and emergency cut-off system.

FIGURE A2.2.1.1-1

Architecture of Train radio system

#### A2.2.1.2	Technical parameters

Table A2.2.1.2-1 summarizes technical characteristics of TRS operating in 150 MHz band.

Table A2.2.1.2-1

Technical characteristics of Digital Train Radio System (TRS) operating in 150 MHz band

| Parameters | Analog TRS | Digital TRS (Type 1) | Digital TRS (Type 2) | Digital TRS (Type 3) |
| --- | --- | --- | --- | --- |
| Frequency Range | 142MHz-144 MHz, 146 MHz-149.9 MHz | 142MHz-144 MHz | 146 MHz-149.9 MHz | 146 MHz-149.9 MHz |
| Channel separation | 20 kHz | 6.25 kHz | 25 kHz | 6.25 kHz |
| Maximum Antenna gain | Base station : +15 dBi Mobile station : +4.2 dBi | Base station : + 11dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi |
| Polarization | Vertical | Vertical | Vertical | Vertical |
| Maximum Transmission power | Base station : +47 dBm Mobile station : +40 dBm | Base station : +40 dBm Mobile station : +30 dBm | Base station : +37dBm Mobile station : +30 dBm | Base station: +30 dBm Mobile station: +30 dBm |
| E.I.R.P. | Base station :+62 dBm Mobile station: +44.2 dBm | Base station : +51 dBm Mobile station: +31 dBm | Base station : +48 dBm Mobile station : +31 dBm | Base station : +41 dBm Mobile station: +31 dBm |
| Receiving noise figure | < 10 dB | < 10 dB | < 10 dB | < 10 dB |
| Data rate | (n/a) | 9.6 kbps | 32 kbps | 4.8 kbps |
| Reception quality | SNR > 40 dB | BER < 10-4 | BER < 10-4 | BER < 10-4 |
| Transmission distance(km) | 3 - 40 km | 1 - 3 km | 1 - 2 km | 1 - 3 km |
| Modulation | FM | π/4QPSK | π/4QPSK | 4FSK |
| Multiplexing method | FDD | FDMA or SCPC | TDMA | FDMA or SCPC |

### 4.2.2	Radiocommunication system for Emergency Cut-Off System

#### A2.2.2.1	System architecture

Radiocommunication system for Emergency Cut-Off System (REMCOS) is used to avoid accidents. The system is used for sending signal to a railway electrification system on ground and electric power for trains in some emergency aria is cut-off.

Figure A2.2.2.1-1 shows the system architecture of REMCOS. When a train driver confirms some emergency circumstances, the driver operates REMCOS on-board and emergency radio signal is transmitted to Central System in Control Centre via Base Stations. In Control Centre, the operational commander manually cuts off the power for trains near the emergency area or REMCOS automatically sends signal to a railway electrification system to cut off the power.

FIGURE A2.2.2.1-1

Architecture of Radiocommunication system for Emergency Cut Off System

## A2.2.2.2	Technical parameters

Table A2.2.2.2-1 summarizes technical characteristics of REMCOS operating in 150 MHz band.

Table A2.2.2.2-1

Technical characteristics of Radiocommunication system for Emergency Cut-Off System
operating in 150 MHz band

| Parameters | Radiocommunication system for Emergency Cut-Off |
| --- | --- |
| Frequency Range | 142 MHz - 144 MHz, 146 MHz - 149.9 MHz, 150.05 MHz - 156.4875 MHz, 156.8375 MHz - 160 MHz |
| Channel separation | 6.25 kHz, 20 kHz |
| Antenna gain (dBi) | +1 |
| Polarization | Vertical |
| Maximum Transmission power (dBm) | +30 |
| E.I.R.P. | +28.5 |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | 1.3 |
| Modulation | π/4-QPSK |
| Multiplexing method | none |

### A2.2.3	Yard Radio

#### A2.2.3.1	System architecture

Yard radio (YR) is used for voice communication between operator in operation room and drivers on board to switch trains in yards or stations. Figure A2.2.3.1-1 shows the system architecture of YR.

FIGURE A2.2.3.1-1

Architecture of Yard radio

#### A2.2.3.2	Technical parameters

Table A2.2.3.2-1 summarizes technical characteristics of YR operating in 150 MHz band.

Table A2.2.3.2-1

Technical characteristics of Yard radio operating in 150 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range (MHz) | 142MHz-144 MHz 146 MHz-149.9 MHz 150.05 MHz-156.4875 MHz 156.8375 MHz-160 MHz |
| Channel separation | 20 kHz |
| Antenna gain (dBi) | +10 |
| Polarization | Vertical |
| Maximum Transmission power (dBm) | +30 to 40 |
| E.I.R.P. | +40 to 50 |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > [30] dB |
| Transmission distance (km) |  |
| Modulation | FM |
| Multiplexing method | none |

## A2.3	300 MHz band RSTT

### A2.3.1	Train Radio System

#### A2.3.1.1	System architecture

System architecture of the Train radio system (TRS) operating in 300 MHz band is similar to one of 150 MHz band described in A2.2.1.1. But transmission distance from a base station is about 1.5 km – 3 km in this system.

#### A2.3.1.2	Technical parameters

Table A2.3.1.2-1 summarizes technical characteristics of TRS operating in 300 MHz band.

Table A2.3.1.2-1

Technical characteristics of Train radio system operating in 300 MHz band

| Parameters | Analog TRS | Digital TRS |
| --- | --- | --- |
| Frequency Range | 335.4 MHz-360 MHz | 335.4 MHz-360 MHz |
| Channel separation | 12.5 kHz | 6.25 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi | Base station: + 11dBi Mobile station : +1 dBi |
| Polarization | Vertical | Vertical |
| Maximum Transmission power | Base station : +36 dBm Mobile station : +30 dBm | Base station : +36 dBm Mobile station : +24.8 dBm |
| E.I.R.P. | Base station : +47 dBm Mobile station : +31 dBm | Base station : +47 dBm Mobile station: +25.8 dBm |
| Receiving noise figure | < 10 dB | < 10 dB |
| Data rate | (n/a) | 9.6 kbps |
| Reception quality | SNR > 30 dB | BER < 10-4 |
| Transmission distance (km) | 1.5-3 km | 1.5 - 2 km |
| Modulation | FM | π/4QPSK |
| Multiplexing method | FDD | FDMA |

### A2.3.2	Emergency Alarm Radio System

#### A2.3.2.1	System architecture

Emergency Alarm Radio System (EARS) is used to avoid accidents. When a train driver confirms some emergency circumstances on track such as line blocked objects, a train derailment, a fire, etc. the driver is expected to send alarm to approaching train’s drivers by EARS in order to avoid a secondary accident. When EARS is operated, emergency radio signal is directly transmitted to approaching trains as shown in Figure A2.3.2.1-1.

EARS is a very simple system. It consists of only mobile-stations on-board. The mobile-station consists of a radio equipment, a transmission button, and an antenna. When the transmission button is pressed, emergency radio signal is transmitted to approaching trains. When the approaching train’s mobile-station receives the signal, it sounds a warning tone and the driver should take necessary actions such as stopping the train. The emergency radio signal reaches nominally within 1 km radius. If it is difficult to reach the emergency to approaching train according to geographical scenario, such as in tunnels, repeaters are installed on trackside in order to expand the coverage of radio propagation.

EARS is used not only for a train to trains but also for ground to trains. In some stations, “Emergency train stop buttons” are prepared at platforms and anyone can push the button to stop trains around the station in emergency, such as someone falling down from platform. If the button is pushed, emergency radio signal as described above is transmitted from the station to approaching trains. And in some railway lines, EARS is also used to send emergency alarm to stop trains when earthquake occurs.

FIGURE A2.3.2.1-1

Emergency radio signal

Architecture of Emergency Alarm Radio System

#### A2.3.2.2	Technical parameters

Table A2.3.2.2-1 summarizes technical characteristics of EARS operating in 300 MHz band.

Table A2.3.2.2-1

Technical characteristics of Emergency Alarm Radio System operating in 300 MHz band

| Parameters | Emergency Alarm Radio System |
| --- | --- |
| Frequency Range (MHz) | 370 MHz-380 |
| Channel separation | 6.25 kHz |
| Antenna gain (dBi) | + 1 |
| Polarization | Vertical |
| Maximum Transmission power (dBm) | +30 |
| E.I.R.P. (dBm) | < 30 |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | Min. 1 |
| Modulation | π/4-QPSK |
| Multiplexing method | none |

### A2.3.3	Radiocommunication system for Emergency Cut-Off System

#### A2.3.3.1	System architecture

System architecture of the Radiocommunication system for Emergency Cut-Off System (REMCOS) operating in 300 MHz band is similar to one of 150 MHz band REMCOS (see A2.2.2.1).

#### A2.3.3.2	Technical parameters

Table A2.3.3.2-1 summarizes technical characteristics of REMCOS operating in 300 MHz band.

Table A2.3.3.2-1

Technical characteristics of Radiocommunication system for Emergency Cut-Off System
operating in 300 MHz band

| Parameters | Radiocommunication system for Emergency Cut-Off |
| --- | --- |
| Frequency Range (MHz) | 340 MHz-370 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain (dBi) | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power (dBm) | [to be filled out in future] |
| e.i.r.p. (dBm) | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | [to be filled out in future] |
| Multiplexing method | none |

### A2.3.4	Radiocommunication system for Electronic Blocking System

#### A2.3.4.1	System architecture

Radiocommunication system for Electronic Blocking System (REBS) is a radio communication system for Electric Blocking System. The Electric Blocking System is used at single-track railroads in rural areas. Figure A2.3.4.1-1 shows the system architecture. When a train stops at a station and is ready for departure, the diver pushes a button of a radio transmitter on-board. The radio transmitter sends radio signal “departure request” to Station Equipment through Radiative Pair Cable (RPC) antenna and Radio Equipment set up at the machine room of the station. The Station Equipment controls electric switch machines, leaving signals, and home signals then the driver can start the train in safety.

FIGURE A2.3.4.1-1

Architecture of Electronic Blocking System

RPC

Antenna

Radio

Equipment

Station

Equipment

Leaving

signal

Home

signal

#### A2.3.4.2	Technical parameters

Table A2.3.4.2-1 summarizes technical characteristics of REBS operating in 300 MHz band.

Table A2.3.4.2-1

Technical characteristics of Radiocommunication system for Electronic Blocking System
operating in 300 MHz band

| Parameters | Radiocommunication system for Electronic Blocking System |
| --- | --- |
| Frequency Range | 335.4 MHz-340 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | + 1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | +30 dBm |
| e.i.r.p. | +31 dBm |
| Receiving noise figure | < 10 dB |
| Transmission distance (km) | Max. 5 m |
| Modulation | FM |
| Multiplexing method | none |

### A2.3.5	JRTC Radio

#### A2.3.5.1	System architecture

JRTC Radio is a sub-system of Japan Radio Train Control system (JRTC). JRTC is automatic train control system that is based on telecommunications between trains and base stations for train traffic management and railway infrastructure control.

Figure A2.3.5.1-1 shows the system architecture of JRTC. On train, the on-board controller detects its own location information that is consists of its location and speed. The mobile station sends the location information to the Ground Controller though Base Stations. With location information of trains, condition of electric switch machines, and condition of level crossing, the Ground Controller calculates the limit in which the train could run safely and sends the stopping limit to the train. The Ground Controller controls the ground equipment as well, such as electric switch machines, level crossings, etc. On the train, the on-board controller calculates a brake pattern and an upper limit speed curve, by using its own brake performance to stop at the running limit directed by the Ground Controller. The on-board controller directs adequate train-speed to the train-driver and if train-speed exceeds the brake pattern, the on-board controller makes the train slow-down or stop by controlling the brake automatically. Requirements of basic function and system construction have been defined in Japanese Industrial Standards as JIS E 3801. JRTC corresponds to the train control system of ERTMS/ETCS Level 3 in Europe.

FIGURE A2.5.3.1-1

System Architecture of JRTC

Electric switch machine

Figure A2.3.5.1-2 shows the frequency usage of JRTC Radio. Four pairs of frequencies are used repeatedly along railways. Cover area of radio base station is about 3 km.

FIGURE A2.5.3.1-2

Frequency usage of JRTC Radio

#### A2.3.5.2	Technical parameters

Table A2.3.5.2-1 summarizes technical characteristics of JRTC Radio.

Table A2.3.5.1-1

Technical characteristics of JRTC Radio

| Parameters | JRTC Radio |
| --- | --- |
| Frequency Range | 335.4 MHz-360 MHz |
| Channel separation | 6.25 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | Base station : +34.8 dBm Mobile station : +30 dBm |
| e.i.r.p. | Base station : +45.8 dBm Mobile station : +31 dBm |
| Receiving noise figure | < 10 dB |
| Data rate | 9.6 kbps |
| Reception quality | BER < 1x10-4 |
| Transmission distance (km) | 2 - 3 km |
| Modulation | π/4 QPSK |
| Multiplexing method | FDD, TDM-TDMA |

### A2.3.6	Yard Radio

#### A2.3.6.1	System architecture

System architecture of the yard radio (YR) operating in 300 MHz band is similar to one of
150 MHz band yard radio system (see A2.2.3.1).

#### A2.3.6.2	Technical parameters

Table A2.3.6.2-1 summarizes technical characteristics of YR operating in 300 MHz band.

Table A2.3.6.2-1

Technical characteristics of Yard radio operating in 300 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range | 335.4 MHz-399.9 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | +37 dBm |
| e.i.r.p. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > [30] dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | FM |
| Multiplexing method | none |

## A2.4	400 MHz band RSTT

### A2.4.1	Train Radio System

#### A2.4.1.1	System architecture

System architecture of the Train radio system (TRS) operating in 400 MHz band is similar to one of 150 MHz band described in A2.2.1.1. But, base stations are located at railway stations, and transmission distance from a base station is about 1.5 km - 3 km in this 400 MHz band system.

#### A2.4.1.2	Technical parameters

Table A2.4.1.2-1 summarizes technical characteristics of TRS operating in 400 MHz band.

Table A2.4.1.2-1

Technical characteristics of Train Radio System operating in 400 MHz band

| Parameters | Train radio system |
| --- | --- |
| Frequency Range | 410-420 MHz |
| Channel separation | 12.5 kHz |
| Maximum Antenna gain | Base station : +11 dBi Mobile station : +1 dBi |
| Polarization | Vertical |
| Maximum Transmission power | +30 dBm |
| e.i.r.p. | Base station : +41 dBm Mobile station : +31 dBm |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > 20 dB |
| Transmission distance (km) | 1.5-3 km |
| Modulation | FM |
| Multiplexing method | none |

### A2.4.2	Radiocommunication system for High Speed Trains

#### A2.4.2.1	System architecture

The radiocommunication system for high speed trains (RHST) is developed and deployed to support safe and stable operation of Shinkansen trains. The most distinctive feature of this system is to use leaky coaxial cables (LCX) all along the line even at no-tunnel area.

Figure A2.4.2.1-1 shows the system architecture of RHST. LCX as shown at right above is a type of coaxial cable that has holes called “slot”. Through these slots, radio wave gradually leaks outside of the cable. The radio wave is propagated to antennas installed at the “skirt” of the vehicle. LCX method allows the distance between LCX and antennas on board to be so close constantly that the affection of interference or noise can be so smaller and it is possible to maintain stable communication regardless of the location of train, open-site or inside of tunnels. Applying the whole LCX method to train radio systems makes it possible to achieve more than 99.99% connections throughout the entire line even when trains are running at high speed (above 300 km/h).

A Central Unit in Control Centre accommodates Ground Communication Controllers, which are located in the key stations. The Ground Communication Controllers take handover through accommodated Base Stations. Base Stations are located in almost every station and repeaters that compensate for LCX propagation loss, are sided at every 1.3 km intervals along track between Base Stations. Four antennas that are installed at body side of the front vehicle, receive radio waves from LCX.

Because of this stable feature of radio communication, some channels are assigned for automatic train control and the radio based train control system, as described in 8.2.1.4, is in practical use in some high-speed train lines.

FIGURE A2.4.2.1-1

Architecture of Radiocommunication system for High Speed Train

LCX

#### A2.4.2.2	Technical parameters

Table A2.4.2.2-1 summarizes technical characteristics of Radiocommunication system for High Speed Train (RHST) operating in 400 MHz band.

Table A2.4.2.2-1

Technical characteristics of Radiocommunication system for High Speed Train (RHST)

| System | Analog RHST | Digital RHST (Type 1) | Digital RHST (Type 2) |
| --- | --- | --- | --- |
| Frequency Range | 412 MH-417.5 MHz, 451.5 MHz-462 MHz |  |  |
| Channel separation | DL:700kHz, UL:12.5kHz | 240kHz, 300kHz | 240kHz, 300kHz, 600kHz |
| Maximum Antenna gain | Base station : Leaky Coaxial Cable (Coupling loss = 55 dB, 60 dB, 70 dB, 80 dB) Mobile station : Slot array antenna (Gain = [+5] dBi) |  |  |
| Maximum Transmission power | Base station: +33 dBm Mobile station: +36 dBm |  | Base station: +27 dBm Mobile station: +36 dBm |
| Receiving noise figure | < 10 dB |  |  |
| Reception quality | SNR > 30 dB BER < 10-4 | BER < 10-4 |  |
| Transmission distance | 30 km (installation interval of base stations) Radio wave propagation distance between LCX and on-board antenna is about 1 - 2 m. |  |  |
| Modulation | down link : PM up link : FM | π/4 QPSK | down link : π/4 QPSK up link : π/4 QPSK and QPSK |
| Multiplexing method | down link : FDM up link : FDMA FDD | down link : TDM up link : TDMA FDD |  |

### A2.4.3	Yard Radio

#### A2.4.3.1	System architecture

System architecture of the yard radio (YR) operating in 400 MHz band is similar to one of 
150 MHz band yard radio system (see A2.2.3.1).

#### A2.4.3.2	Technical parameters

Table A2.4.3.2-1 summarizes technical characteristics of YR operating in 400 MHz band.

Table A2.4.3.2-1

Technical characteristics of Yard radio operating in 400 MHz band

| Parameters | Yard radio |
| --- | --- |
| Frequency Range (MHz) | 450 MHz-470 MHz |
| Channel separation | 12.5 kHz |
| Antenna gain | [to be filled out in future] |
| Polarization | Vertical |
| Maximum Transmission power | +37 dBm |
| e.i.r.p. | [to be filled out in future] |
| Receiving noise figure | < 10 dB |
| Reception quality | SNR > 30 dB |
| Transmission distance (km) | [to be filled out in future] |
| Modulation | FM |
| Multiplexing method | none |

## A2.5	RSTT in the 40 GHz band

### A2.5.1	40-GHz band video transmission system (MVT)

#### A2.5.1.1	System architecture

In order to keep passengers’ safety, MVT has already been deployed for many railways, in which trains are driving without any conductor. In this case drivers must confirm platform situations by themselves at each station before departure. MVT enables drivers to confirm platform situations by showing these in the driver’s room.

Figure A2.5.1.1-1 shows the architecture of MVT. CCTV cameras are located at several points in every platform. Millimetre waves transmit these cameras’ video streams to the diver’s room through transmitter and receiver. Monitors in the driver’s room show the conditions of the platform from several cameras simultaneously without latency. Therefore, the driver can confirm the situations of platform then start the train safely.

Figure A2.5.1.1-1

Architecture of MVT

#### A2.5.1.2	Technical parameters

Table A2.5.1.2-1 shows technical characteristics of the video transmission system (MVT). Now the modulation of the system is FM because of its low latency feature. But in future, digital modulations will be adopted for digital ITV with low latency video codecs.

Table A2.5.1.2-1

Technical characteristics of MVT

| Parameter | MVT |
| --- | --- |
| Frequency Range (GHz) | 43.5-43.7 |
| Channel separation | 40 MHz |
| Antenna gain (dBi) | 33 typ. |
| Polarization | Vertical |
| Transmitting radiation power (dBm) | 0 |
| e.i.r.p. (dBm) | 33 typ. |
| Receiving noise figure (dB) | <20 |
| Transmission distance (m) | < 60 |
| Modulation | FM |
| Multiplexing method | FDM |

### A2.5.2	Train Radio System in the 40 GHz band (TRS-40GHz)

#### A2.5.2.1	System architecture

Recently there is a growing demand for video transmission from running trains to the control centre. It will be used for real time cabin watching to keep the passenger’s safety, or for immediate outside watching from driver’s room to confirm some emergency circumstances on track.

For video streaming, the broadband transmission capabilities are required and 40 GHz band is the most expected candidate for these applications as the band is already used for video transmission system of platform.

As for TRS-40GHz, some field trials have been continued towards the next generation of TRS.
The architecture of the system is the same as the traditional TRS except for the radio communication between train and track side where LCX is used. The image of communication between train and track side is shown in Figure A2.5.2.1-1. Millimetre-waves are transmitted to the train by narrow-beam width antennas set at the trackside poles.

Figure A2.5.2.1-1

Image of RSTT in the 40 GHz band

#### A2.5.2.2	Deployment scenario

The deployment scenario so called “Linear cell concept” is shown in Figure A2.5.2.2-1. Track-side antennas are linearly distributed along the track and millimetre-waves from these antennas, with the same signal and the same frequency, would compose a linear cell. Optical feeders are used to connect between the trackside antennas and the base stations. The linear cells with frequency 1 and 2 are alternately repeated itself. By using this concept, frequent handovers that cause throughput reducing, are able to be avoidable especially for high-speed trains. Furthermore, the spectral utilization is efficient because if the length of the linear cell is long enough, only two frequencies are needed for inter-cell interference prevention.

Figure A2.5.2.2-1

Linear cell concept

The upper view of track-side antennas setting at track-side poles are shown in Figure A2.5.2.2-2. These antennas are aimed at the on-board antennas which were located at the point of maximum transmission distance. The on-board antennas are equipped around driver’s room and aimed straight forward. The maximum transmission distance is calculated in the situation of heavy rain.

As the antennas are deployed as explained above, the millimetre wave beams are not obstructed by track-side poles or oncoming trains and even in heavy rain. The deployment method promises to achieve a reliable radio transmission that can be used to keep the railway safe.

Figure A2.5.2.2-2

Upper view of track-side antennas

#### A2.5.2.3	Technical parameters

Table A2.5.2.3-2 summarizes technical and operational characteristics of RSTT stations operating in 43.5-45.5 GHz band. The bandwidth of 2.0 GHz can be used for data transmission between on-board radio equipment and related radio infrastructure located along trackside. The transmission distance of these equipment varies according to the railroad line condition.

Table A2.5.2.3-1

Preliminary system parameters of RSTT stations operating in 43.5-47 GHz band

| Frequency Range (GHz) | 43.5-47.0 |
| --- | --- |
| Antenna gain (dBi) | 32 |
| Antenna beamwidth (degree) | 1.0-1.5 |
| Polarization | Circular |
| Transmitting radiation power (dBm) | 10 |
| e.i.r.p. (dBm) | 42 |
| Receiving noise figure (dB) | <10 |
| Transmission data rate (Mb/s) | 100 |
| Transmission distance (km) | 0-3.5 |
| Modulation | 64QAM, OFDM |
| Multiplexing method | FDD |

Table A2.5.2.3-2

Preliminary system parameters of RSTT stations operating in 43.5-45.5 GHz band

| Frequency range (GHz) | 43.5-45.5 |
| --- | --- |
| Channel bandwidth (MHz) | 40 |
| Antenna gain (dBi) | 30 |
| Antenna beamwidth (degree) | ±1.0-1.5 |
| Polarization | Circular or Vertical |
| Transmitting radiation power (dBm) | 15 |
| e.i.r.p. (dBm) | 45 |
| Receiving noise figure (dB) | <10 |
| Transmission data rate (Mb/s) | 100 Mbps(64QAM) x N (carrier parallelization) |
| Transmission distance (km) | < 0.5 (Open site in the heavy rain at BPSK) |
| Modulation | BPSK, QPSK, 64QAM, OFDM |
| Multiplexing method | TDM-TDMA |
| Space diversity | 2x2 |
| Maximum running speed (km/h) | 600 |
| Rainfall attenuation margin (dB) | 24.88 dB/km at rain rate 100 mm/h |
| Wired interface of trackside radio access unit | TBD |
| Propagation model between train and trackside | TBD |

## A2.6	90-GHz -band RSTT

### A2.6.1	Network architecture of 90-GHz -band RSTT

A network topology of a typical broadband TCN with a radio access system is shown in Fig. A2.6.1-1. A central unit (CU) located in an operation direction center (ODC) transmits, receives and manages all the communication signals. The signals are transmitted to an optical carrier station (OCS) via an optical fiber link; the OCS is worked as a repeater. Typical distance between the OCSs will be much longer than 10 km, whose length will be limited by an optical transceiver output power. A node base station (NBS), which is controlled by a control station (CS) directly connected to the OCS, is worked as a radio base station to transport the signal to track-side radio access units (TS-RAUs) located along a railway trackside. The network for connection between the NBS and TS-RAUs is a bus-type wireline network, a passive-single-star-type network, or point-to-point links to each TS-RAU. The TS-RAU is a frontend to transmit and receive radio signals from/to radio transceivers on the train. In general, distance between the TS-RAUs depends on a coverage size of the TS-RAU; for instance, the distance will be much longer than 1 km if microwave-band radio systems are implemented. In this configuration, an optical fiber network will be suitable for configuring network for between CS and NBSs, and between NBS and TS-RAUs for high-speed railway systems.

Figure A2.6.1-1

Conceptual diagram of a broadband TCN using 90-GHz -band RSTT

### A2.6.2	Deployment scenario

Figure A2.6.2-1 shows the schematic concept of seamless wireless connection between on-board equipment and trackside radio access unit. The concept shows that 10 trackside radio access units with two antennas are equipped along the railway line. Two on-board transceivers are equipped with the driver’s room and the conductor’s room which are usually placed at the end of train. Both on-board transceivers are complementally connected to the trackside radio access units to seamlessly maintain link connection through 90-GHz carrier.

Figure A2.6.2-1

Concept of seamless wireless connection between on-board and trackside equipment

### A2.6.3	90-GHz band -band transceivers characteristics

Table A2.6.3-1 shows the frequency bands which are already allocated for use of mobile services and will be planned to use for RSTT. Table A2.6.3-2 summarizes technical and operational characteristics of RSTT stations operating in 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz bands. The total bandwidth of 15.4 GHz can be used for data transmission between on-board radio equipment and trackside radio access units. The transmission distance of these equipment is designed by the railroad line environment.

Table A2.6.3-1

Frequency bands already allocated for mobile servicers

|  | 92-94 |  | 94.1-100 |  | 102-109.5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MS |  | MS |  | MS |  |
|  | BW1=2 GHz |  | BW2=5.9 GHz |  | BW3=7.5 GHz |  |

Table A2.6.3-2

Preliminary system parameters

| Frequency Range (GHz) | 92-94. 94.1-100, 102-109.5 |
| --- | --- |
| Seamless connection mechanism | Backward and forward switching method |
| Channel bandwidth (MHz) | 400 |
| Antenna gain (dBi) | 44 |
| Antenna beamwidth (degree) | 1 |
| Antenna height from rail surface (m) | 4 (Maximum) |
| Polarization | Linear |
| Average transmitting power (dBm) | 10 |
| Average e.i.r.p. (dBm) | 54 |
| Receiving noise figure (dB) | <10 |
| Maximum transmission data rate (Gb/s) | 5-10 (Stationary), 1 (Running) |
| Maximum transmission distance (km) | 0.5-11 (Open), 3 (Tunnel) |
| Modulation | PSK, QPSK, 16QAM, 64QAM |
| Multiplexing method | FDD |
| Space diversity | TBD |
| Maximum running speed (km/h) | 600 |
| Switching time of trackside radio access unit (s) | TBD |
| Rainfall attenuation margin (dB) | TBD |
| Wired interface of trackside radio access unit | [Recommendation ITU-T G.RoF] |
| Propagation model between train and trackside | [Recommendation ITU-R P.1411] |

Figure A2.6.3 -3 shows a block diagram of 90-GHz band track-side radio access unit. The transmitter (Tx unit) consists of a multiplier (MP), amplifiers and an up-converter. The IF signals are up-converted by a broadband up-converter to 90-GHz band signals. The LO signal of the receiver is also supplied from the node base station through an optical fiber. It is also multiplied by a multiplier and then supplied to a mixer to down-convert the received 90-GHz band signals to IF signals (Rx unit). Figure A2.6.3-4 shows the external view of 90-GHz band track-side radio access unit.

Figure A2.6.3-3

Block diagram of 90-GHz band transceiver for track-side radio access unit

Figure A2.6.3-4

External view of 90-GHz band track-side radio access unit

### A2.6.4	90-GHz band propagation characteristics

The viaduct is most commonly used construction for the railway systems. Figure A2.6.4-1 shows the typical structure of viaduct used for high speed railway systems. Figure A2.6.4-2 shows the attenuation characteristics of 90 GHz frequencies guided by viaduct. Table A2.6.4-1 shows the system parameters for propagation measurement. There are attenuation loss differences between wave propagation in the free space and in the guided viaduct. In addition to that, there are also attenuation loss differences between the lower and higher height of the receiver antenna than the height of side walls. This results show that the side wall affects the propagation characteristics of 90 GHz frequencies and decreases the propagation loss.

Figure A.2.6.4-1

Cross sectional view of viaduct.

Figure A2.6.4-2

Attenuation characteristics 90-GHz frequencies guided by viaduct.

TABLE A2.6.4-1

System parameters for measurement

| Centre Frequency | 93.2 GHz |
| --- | --- |
| Transmitter output power | -5 dBm |
| Transmitter antenna type | Horn Antenna |
| Transmitter antenna gain | 25 dBi |
| Transmitter antenna half-value angle | 10 degrees |
| Transmitter antenna height | 0.92 m or 1.92 m |
| Receiver antenna Type | Horn Antenna |
| Receiver antenna gain | 25 dBi |
| Receiver antenna half-value angle | 10 degrees |
| Receiver antenna height | 0.92 m or 1.92 m |

## A2.7	60 GHz RSTT

### A2.7.1	60-GHz band train platform monitoring system

Since passenger safety at the station is a primary concern of railway system, the train platform monitoring system is introduced to monitor passengers on the track line of the station. The video monitors are equipped at the control room in the station room, the train driver’s room and the conductor’s room. Several video cameras are placed to monitor almost entire train platform. The 60-GHz transceivers are connected to those video cameras and monitors to transmit/receive video signals. Due to surveillance capabilities of the monitoring system, serious accident of passengers at the station platform can be prevented. The system configuration is shown in Figure 2.7.1-1.

FIGURE 2.7.1-1

System configuration of 60-GHz train platform monitoring system

60-GHz band train platform monitoring system

### A2.7.2	60-GHz band RSTT

60-GHz band train platform monitoring system is widely introduced for safety operation of railway systems. The train platform monitoring system assists train crews to open/close train doors safely to prevent passengers’ accidents by looking at the monitor equipped in the train driver’s room, as shown in Figure 2.7.1-1. This section provides technical and operational characteristics of 60-GHz band train platform monitoring system which was already developed and is deployed in many stations.

#### A2.7.2.1	Technical and operational characteristics

Table 2.7.2-1 shows technical characteristics of 60-GHz band train platform monitoring system.

Table 2.7.2-1

Technical and operational characteristics of 60-GHz train platform monitoring system

| Parameters | Fixed station | On-board station |
| --- | --- | --- |
| Frequency Range (GHz) | 59-66 | 59-66 |
| Antenna gain (dBi) | 31 | 26 |
| Antenna beam width (degree) | 3.5 | 7 |
| Polarization | Linear | Linear |
| Transmitting radiation power (mW) | 10 | 10 |
| e.i.r.p. (dBm) | 41 | 36 |
| Receiving noise figure (dB) | 8 | 8 |
| Transmission data rate (Mb/s) | 100 | 100 |
| Transmission distance (m) | 100 | 100 |
| Modulation | ASK | ASK |
| Multiplexing method | FDD | FDD |
| Network interface | 100 Base-TX | 100 Base-TX |

ANNEX 3

RSTT in China

## A3.1	Overview

Chinese railway radiocommunication technology has witnessed rapid development since 1950s, especially in 450 MHz wireless train dispatching system and 900 MHz GSM-R system. These systems have been implemented into the whole Chinese railway network, which is of great importance for the safety of railway transportation.

## A3.2	450 MHz -band RSTT

Wireless train dispatching system has been implemented in China since 1950s, which used for voice communication and dispatching order transmission. Up to 2015, 450 MHz system has been deployed over 84,000 kilometers lines in China.

With the development of railway radiocommunication technology, the existing 450 MHz wireless train dispatching system might be gradually replaced by advanced technologies in China, for instance the GSM-R or other next generation railway radiocommunication technologies.

Table A3.2-1

450 MHz band RF characteristics

| Parameter | 450 MHz Band Wireless Train Dispatching System |
| --- | --- |
| Frequency Range (MHz) | 457.200-458.650 467.200-468.650 |
| Channel Spacing (kHz) | 25 |
| Frequency Tolerance | ≤5×10-6 |
| Antenna Gain (dBi) | Fixed radio station: 9(Omni-directional), 12(Directional) Locomotive station: 0 |
| Carrier Power (dBm) | Fixed Radio Station: 34.7-37(simplex), 37-40 (duplex) Locomotive Station: 37(simplex), 40 (duplex) Handset: 34.7 |
| Modulation type | FM |
| Adjacent-channel Selectivity (receiver) (dB) | ≥65 |
| Co-channel Rejection (dB) | ≥-8 |

## A3.3	900 MHz -band RSTT

Since the first deployment of GSM-R system in the Qinghai-Tibet Railway, 900 MHz GSM-R system has been implemented in all new lines from 2006. Up to 2015, GSM-R has been implemented over 37,000 kilometers lines in China, including part of existing regular lines and all high-speed lines. In China, GSM-R system provides voice service and data service for railway transportation.

Voice Service: On-train outgoing voice communication from the driver towards the controller(s) of the train, on-train incoming voice communication from the controller towards a driver, railway emergency communication, trackside maintenance communication and public emergency call, etc.

Data service: Automatic train control communication, Monitoring and control of critical infrastructure, Shunting data communication, etc.

China has set up series of technical specifications for GSM-R. The RF characteristics of GSM-R in China are listed in the following Table.

Table A3.3-1

900 MHz band RF characteristics

| Parameter | 900 MHz Band GSM-R System |
| --- | --- |
| Frequency Range (MHz) | 885~889 (uplink) 930~934 (downlink) |
| Channel separation (kHz) | 200 |
| Antenna gain (dBi) | Mobile station: ≥0 Base station : 65°(Half-power Beam width):17 Or 33°(Half-power Beam width):21 |
| Polarization | Dual-polarized |
| Transmitting radiation power (dBm) | Mobile station (Handset: 33, Locomotive station: 39) Base station: 46 |
| Modulation | GMSK |
| Multiplexing method | TDMA |
| Receiver sensitivity(dBm) | Mobile station: ≤-104 Base station: ≤ -110 |

ANNEX 4

RSTT in Russia

## A4.1	Overview

Radiocommunication systems are widely used in railway transportation in Russia. Those systems provide for fulfilling important tasks including railway traffic management and control, as well as provision of railway transportation safety. The systems also provide train crews, conductors and passengers with services associated with different information exchange. Relevant State Radiofrequency Commission Decisions allotted different HF, VHF and UHF bands for operation of those systems.

## A4.2	Train radio systems for train operation and railway traffic control in the HF band

### A4.2.1	Architecture of train radio system in HF-band

[TBD]

### A4.2.2	Typical technical characteristics of HF train radio system radio stations

Typical technical characteristics for radio stations operating in train radio systems in 2 MHz band are shown in Table A4.2-1.

TABLE A4.2-1

Sample radiostation parameters

| Frequency band (MHz) | 2.124-2.136/ 2.144-2.156 |
| --- | --- |
| Channel passband (kHz) | 11.8 |
| Antenna gain (dBi) | 0 |
| Polarization | Vertical/Horizontal |
| Transmitter output power (dBW) | 11,5 |
| e.i.r.p. (dBW) | 11,5 |
| Receiver noise figure (dB) | <12 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 15 (max) |
| Modulation | FM |
| Multiplexing mode | FDMA |

## A4.3	Train radio systems for train operation and railway traffic control in the VHF band

### A4.3.1	Architecture of train radio system in VHF-band

### A4.3.2	Typical technical characteristics of VHF train radio system radio stations

Typical technical characteristics for analog radio stations operating in train radio systems in 150 MHz band are shown in Table A4.3.2-1.

TABLE A4.3.2-1

Sample radiostation parameters

| Frequency band (MHz) | 146-174 |
| --- | --- |
| Channel passband (kHz) | 12.5, 25 |
| Antenna gain (dBi) | 2 - 5 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 16 |
| e.i.r.p. (dBW) | 18 - 21 |
| Receiver noise figure (dB) | <10 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 20 (max) |
| Modulation | FM |
| Multiplexing mode | FDMA |

Typical technical characteristics of digital radio stations are based on DMR standard for train radio systems, such as shunting automatic locomotive signaling system, systems of interval traffic control, remote control shunting locomotive and the system of forced stop of a locomotive operating in the range of 150 MHz are presented in Table A4.3.2-2.

TABLE A4.3.2-2

Sample radio stations parameters

| Frequency band (MHz) | 151,7125 – 154,0125; 154,9875 – 156,0125 |
| --- | --- |
| Channel passband (kHz) | 12.5 |
| Antenna gain (dBi) | 2-17 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 3 (MS), 10 (locomotive), 16 (BS) |
| e.i.r.p. (dBW) | 4-57 |
| Receiver noise figure (dB) | <10 |
| Data rate (kbps) | 2 |
| Radiocommunication distance (km) | Max. 20 |
| Modulation | 4FSK |
| Multiplexing mode | TDMA |

Typical technical characteristics of a digital radio system of brake control for increased weight and length trains, operating in the range of 150 MHz are presented in Table A4.3.2-3. [Russia]

TABLE A4.3.2-3

Sample radio stations parameters

| Frequency band (MHz) | 155,075 – 155,275 |
| --- | --- |
| Channel passband (kHz) | 25 |
| Antenna gain (dBi) | 2-10 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 10 |
| e.i.r.p. (dBW) | 12-20 |
| Receiver noise figure (dB) | <8 |
| Data rate (kbps) | 2 |
| Radiocommunication distance (km) | Max. 10 |
| Modulation | FM |
| Multiplexing mode | TDMA |

## A4.4	Train radio systems for train operation and railway traffic control in the UHF band

### A4.4.1	Architecture of train radio system in UHF-band

### A4.4.2	Typical technical characteristics of UHF train radio system radio stations

Typical technical characteristics for radio stations operating in train radio systems TETRA standard and shunting automatic locomotive signaling system, in 450 MHz band are shown in Table A4.4.2-1.

table A4.4.2-1

Sample radiostation parameters

| Frequency band (MHz) | 457,40 – 458,45; 467,40 – 468,45 |
| --- | --- |
| Channel passband (kHz) | 25 |
| Antenna gain (dBi) | 3 - 7 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 10-15 |
| e.i.r.p. (dBW) | 13 - 22 |
| Receiver noise figure (dB) | <12 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 10-20 |
| Modulation | π/4-DQPSK, DRCMK |
| Multiplexing mode | TDMA |

Typical technical characteristics for radio stations operating in GSM-R train radio systems, shunting automatic locomotive signaling system, automated control system ACS-D, the system of interval regulation ITARUS and the automated control system of alerting and warning ASWAP-2 in 900 MHz band are shown in Table A4.4.2-2.

TaBLE A4.4.2-2

Sample radiostation characteristics

| Frequency band (MHz) | 876 – 880 921 – 925; |
| --- | --- |
| Channel passband (kHz) | 200 |
| Antenna gain (dBi) | 0 -20 |
| Polarization | Vertical, inclined |
| Transmitter output power (dBW) | -6 … 17 |
| e.i.r.p. (dBW) | -6 …37 |
| Receiver noise figure (dB) | <8 |
| Data rate (kbps) | 8 x 22,8 |
| Radiocommunication distance (km) | Max. 20 |
| Modulation | GMSK, π/4-DQPSK, DRCMK |
| Multiplexing mode | TDMA |

Typical technical characteristics of radio systems for automatic identification of moving vehicles "Palm" operating in the range of 860 MHz is presented in Table A4.4.2-3.

table A4.4.2-3

Sample radio stations parameters

| Frequency band (MHz) | 865, 867, 869 |
| --- | --- |
| Channel passband (kHz) | 200 |
| Antenna gain (dBi) | 5 |
| Polarization | Vertical |
| Transmitter output power (dBW) | 3 |
| e.i.r.p. (dBW) | 8 |
| Receiver noise figure (dB) | <8 |
| Data rate (kbps) | 22,8 |
| Radiocommunication distance (km) | 0,005 |
| Modulation | GMSK |
| Multiplexing mode | - |

Typical technical characteristics for radio stations operating in DECT train radio in 1 900 MHz band are shown in Table A4.4.2-4.

table A4.4.2-4

Sample radio stations parameters

| Frequency band (MHz) | 1880 - 1900 |
| --- | --- |
| Channel passband (MHz) | 1,73 |
| Antenna gain (dBi) | 15 |
| Polarization | Vertical |
| Transmitter output power (dBW) | -20 |
| e.i.r.p. (dBW) | -5 |
| Receiver noise figure (dB) | <5 |
| Data rate (kbps) | 1,152 |
| Radiocommunication distance (km) | 5 |
| Modulation | GMSK |
| Multiplexing mode | TDMA |

## A4.5	Train radio systems for train operation and railway traffic control in the SHF band

### A4.5.1	Architecture of train radio system in SHF-band

Radar operating in the range 76 GHz is installed at the stationary obstacles on the railway track (e.g. track focus stalled on railroad tracks), as shown in Figure A4.5.1-1. This radar measures the motion parameters of the approaching rolling stock (speed, distance) and transmits that data into a Comprehensive system of safety on the dead-end paths, passenger stations for high-speed, passenger, suburban trains and shunting "PRISMA-K". The system "PRISMA-K" defines the threat of a dangerous convergence with an obstacle and sends data and commands to the speed reduction or forced stop the locomotive or the head of an approaching motor car of rolling stock.

FIGURE A4.5.1-1

The deployment of the radar 76 GHz at the track focus stalled on the railroad tracks.

### A4.5.2	Typical technical characteristics of SHF train radio system radio stations

Typical technical characteristics of the radar system measuring the distance and speed of moving objects "Prisma-K", operating in the range 76 GHz are presented in Table A4.5.2-1

table A4.5.2-1

Sample radars parameters

| Frequency band (MHz) | 76,35 – 76,65 |
| --- | --- |
| Channel passband (MHz) | 500 |
| Antenna gain (dBi) | 50 |
| Polarization | Vertical |
| Transmitter output power (dBW) | -50 |
| e.i.r.p. (dBW) | 0 |
| Receiver noise figure (dB) | <1 |
| Data rate (kbps) | - |
| Radiocommunication distance (km) | 0,5 |
| Modulation | LFM |
| Multiplexing mode | - |

ANNEX 5

RSTT in Korea

## A5.1	Overview

Korean railway uses 150 MHz, 400 MHz, 700 MHz, 800 MHz and 18 GHz band for RSTT that carries train control, command, operational information as well as monitoring data between on-board radio equipment and related radio infrastructure located along trackside. Very High Frequency (VHF) system using 150 MHz band supports only voice calls between control centre/base station and a train crew or inter-mobile station radiocommunication. Train Radio Protection Device (TRPD) system is used to alert some dangerous situations to trains within about 4 km range through emergency signals at the 400 MHz band. LTE-R system provides various RSTT functions using LTE based wireless communication operated at the 700 MHz band. Korea is using two TRS (Trunked Radio System) schemes, i.e., TRS-ASTRO and TRS-TETRA. These systems are operated at the 800 MHz band for transferring operational information between train and control centre. The government has a plan to replace these TRS systems to LTE-R systems. In addition, CCTV video systems are used to monitor the platform situations. The system transmits these cameras’ video stream to driver’s room using a wireless communication operated at the 18 GHz band. Train positioning system is based on Balise operating at the 4.234 MHz and the 27.095 MHz.Table A5.1-1 is the list of RSTTs used in Korea.

Table A5.1-1

List of RSTTs used in Korea

| # | Name of the Systems | Frequency bands in use |
| --- | --- | --- |
| 1 | LTE-R | 718-728 MHz (uplink), 773-783 MHz (downlink) |
| 2 | VHF | 153 MHz |
| 3 | TRS | ASTRO: 806-811 MHz (uplink), 851-856 MHz (downlink) TETRA: 806-811 MHz (uplink), 851-856 MHz (downlink) |
| 4 | TRPD | 433.3125 MHz |
| 5 | Video for Platform | 18.86-18.92 GHz, 19.20-19.26 GHz |
| 6 | Balise | 3.951-4.516 MHz, 27.09-27.10 MHz |

## A5.2	700 MHz band RSTT

LTE based 700 MHz band system, LTE based Railway communication (LTE-R), provides voice, data, video and control data radiocommunication services among railway entities including control centre, base station, train crews, drivers, and workers in high-speed train. LTE-R system uses single channel for uplink and downlink.

TTA (Telecommunications Technology Association) has developed standards of LTE based Railway communication system.

–	‘User Requirements for LTE based Railway Communication System’ (TTAK.KO-06.0370, 2014)

–	‘Functional Requirements for LTE based Railway Communication System’ (TTAK.KO-06.0369, 2014)

–	‘System Requirements for LTE based Railway Communication System’ (TTAK.KO-06.0437, 2016)

–	‘LTE based Railway Communication System Architecture’ (TTAK.KO-06.0438, 2016)

### A5.2.1	Architecture of LTE-R

LTE-R system is composed of LTE-R train control, LTE-R mobile network, LTE-R terminal, and track side equipment as shown in Figure A5.2.1-1. LTE-R train control consists of Centralized Train Control Centre (CTC) and Radio Block Centre (RBC). LTE-R mobile network consists of LTE-R core network and LTE-R access network. LTE-R terminal has two types of terminals; hand-held terminal and train-borne equipment. Train-borne equipment is located in the train engine room. Trackside equipment includes devices providing train position and track circuit with communication capability. LTE based railway communication system also shall provide interworking with PS-LTE as well as the legacy railway communication networks such as VHF, ASTRO and TETRA TRS.

Figure A5.2.1-1

LTE based Railway Communication System Architectural Concept

### A5.2.2	Technical characteristics

This system is based on the 3GPP Release 12 basically and Release 13 optionally including D2D function.

Table A5.2.2-2

Technical characteristics of LTE-R system

| Parameter | System 1: (LTE-R) |
| --- | --- |
| Frequency Range (MHz) | Uplink: 718-728 MHz, Downlink: 773-783 MHz |
| Number of Channels | 1 |
| Channel separation (kHz) | 10 MHz |
| Antenna gain (dBi) | - |
| Polarization | - |
| Transmitting radiation power (dBm) | 23 dBm (UE), 46 dBm (BS) |
| e.i.r.p. (dBm) | - |
| Receiving noise figure (dB) | - |
| Transmission data rate (kb/s) | Downlink: Max 75 Mbps, Uplink: Max 37 Mbps |
| Parameter | System 1: (LTE-R) |
| Transmission distance (km) | - |
| Modulation | Downlink: OFDMA, Uplink: SC-FDMA |
| Multiplexing method | Full Duplex FDD |

### A5.2.3	Operational characteristics

LTE system provides voice and data services for control and operation of conventional and high speed railway.

## A5.3	150 MHz band RSTT

150 MHz system provides point-to-point radiocommunication scheme between control centre/base station and a train crew or inter-mobile station radiocommunications in conventional train. VHF system uses 4 channels for exchanging data at the 153 MHz frequency band. Since the radiocommunication is established by voice call depending on propagation range, users must be careful to be in radiocommunication range. Due to point-to-point scheme, various radiocommunication functions such as group radiocommunication, priority radiocommunication are not supported. Furthermore, the main requirement for railway wireless networks, i.e., safety, reliability, and security, are not guaranteed. Table A5.3.1-1 represents frequency band allocation for VHF system.

### A5.3.1	Technical characteristics

Table A5.3.1-1 shows technical characteristics of VHF system.

Table A5.3.1-1

Technical characteristics of VHF system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 153 MHz |
| Number of channels | 4 |
| Channel separation | 25 kHz |
| Antenna gain | 3 dBi |
| Polarization |  |
| Transmitting radiation power | Base statin: 25 W, Train: 25 W, Portable terminal: 3-4.8 W |
| e.i.r.p. |  |
| Receiving noise figure | -113 dB |
| Transmission data rate | 9.6 kbps |
| Transmission distance | - |
| Modulation | FM |
| Multiplexing method |  |

### A5.3.2	Operational characteristics

Table A5.3.2-2 presents channel arrangement for VHF system.

Table A5.3.2-2

Channel arrangement for VHF system

| Item | CH | Tx | Rx |
| --- | --- | --- | --- |
| Portable terminal (worker, driver) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.250 MHz |  |
|  | 3(Work) | 153.280 MHz |  |
|  | 4(Work) | 153.660 MHz |  |
| Portable terminal (shunter) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.340 MHz |  |
|  | 3(Work) | 153.740 MHz |  |
|  | 4(Work) | 153.660 MHz |  |
| Mobile terminal (crew) | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.520 MHz |  |
|  | 3(Work) | 153.590 MHz | 153.110 MHz |
|  | 4(Work) | 153.620 MHz | 153.200 MHz |
| Base station | 1(Normal) | 153.440 MHz | Same as Tx |
|  | 2(Emergency) | 153.520 MHz |  |
|  | 3(Work) | 153.110 MHz | 153.590 MHz |
|  | 4(Work) | 153.200 MHz | 153.620 MHz |

## A5.4	800 MHz band RSTT

Korea is using two TRS (Trunked Radio System) schemes, i.e., TRS-ASTRO and TRS-TETRA. TRS-ASTRO uses FDMA (Frequency Division Multiple Access) which uses one channel per 12.5 kHz. TRS-TETRA uses TDMA (Time Division Multiple Access) and provides 4 channels with 25 kHz bandwidth.

### A5.4.1	Technical characteristics

Table A5.4.1-1 shows technical characteristics of TRS system.

Table A5.4.1-1

Technical characteristics of TRS-ASTRO and TRS-TETRA system

| Technical Parameters | Technical characteristics |  |
| --- | --- | --- |
|  | ASTRO | TETRA |
| Frequency Range | Uplink: 806-811 MHz, Downlink: 851-856 MHz | Uplink: 806-811 MHz, Downlink: 851-856 MHz |
| Channel separation | 12.5 kHz, 25 kHz | 25 kHz |
| Antenna gain | 3 dBi | 3 dBi |
| Polarization | - | - |
| Transmitting radiation power | Base station: 70 W, Train: 30W, Portable terminal: 3W | Base station: 25 W, Train: 3W, Portable terminal: 1W |
| e.i.r.p. | - | - |
| Receiving noise figure | -118 dB | Terminal: -112 dB, Base station: -125 dB |
| Transmission data rare | 9.6 kbps | 36 kbps |
| Transmission distance | - | - |
| Modulation | C4FM (Continuous 4 level FM) | π/4 DQPSK |
| Multiplexing method | FDMA | TDMA |

### A5.4.2	Operational characteristics

TRS provides voice service such as one-to-one call, one-to-many call, group call, emergency call, and direct call as well as data service such as message and packet transmission. TRS-TETRA has versatile availability for railway wireless network compared with TRS-ASTRO and VHF.

## A5.5	400 MHz band RSTT

TRPD (Train Radio Protection Device) in 400 MHz band provides accident information to adjacent trains to avoid additional accidents. This system has a wireless train protection function which is installed on the train for the railway vehicle the event of emergencies such as accidents and dangerous situations. It promptly informs and makes stop or decrease speed of the nearby to prevent from subsequent accidents.

### A5.5.1	Technical characteristics

Table A5.5.1-1

Technical characteristics of TRPD system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 433.3125 MHz |
| Number of channels | 1 |
| Channel separation | 12.5 kHz |
| Antenna gain | 3 dBi |
| Polarization | Vertical |
| Transmitting radiation power | 36 dBm |
| e.i.r.p. | 39 dBm |
| Technical Parameters | Technical characteristics |
| Receiving noise figure | Under 2 |
| Transmission data rare | 8 kbps |
| Transmission distance | 4 km |
| Modulation | GMSK (Gaussian Minimum Shift Keying) |
| Multiplexing method | Single |

## A5.6	18 GHz band RSTT

Platform Video System provides video streams to driver from the camera when the train enters to the platform of a station to monitor the clearance of the trackside.

### A5.6.1	Technical characteristics

Table A5.6.1-1

Technical characteristics of platform video system

| Technical Parameters | Technical characteristics |
| --- | --- |
| Frequency Range | 18.86-18.92 GHz, 19.20-19.26 GHz |
| Number of channels | 6 |
| Channel separation | 10 MHz |
| Antenna gain | - |
| Polarization | - |
| Transmitting radiation power | 100 mW |
| e.i.r.p. | - |
| Receiving noise figure | - |
| Transmission data rate | - |
| Transmission distance | 1.5-2.5 km |
| Modulation | OFDM |
| Multiplexing method | - |

[ANNEX 6

RSTT in Europe

## A6.1	GSM-R train radio

### A6.1.1	Overview

As per the Command-Control and Signalling Technical Specification for Interoperability, Regulation 2016/919/EU, GSM-R is the only radio access technology allowed. It supports the specific railway needs and consists of additional functions and amendments, and represents an enhancement of the commonly used GSM technology.

For the use of GSM-R, Commission Decision 1999/569/EC and ECC Decision (02)05 harmonises the frequency bands 876-880 MHz (train-to-ground) and 921-925 MHz (ground-to-train). The allocation of a dedicated, harmonised frequency band enabled the creation of pan-European radiocommunication network for both passenger and freight trains to travel across EU borders without the need to install any other national radiocommunication systems. This fulfils the requirement of the Interoperability Directive 2016/797/EU. Furthermore, a dedicated frequency band participates to ensure continuous reliable and available access to the network thus supporting mission critical, interoperable and safety-related applications.

This radio network is used to fulfil the operational requirements of railway communications, especially for voice and data communications both supporting safety-related applications. These services are conventional voice services (driver-controller calls, railway emergency calls, group calls, etc.) and data services for ERTMS/ETCS signalling systems and passenger information.

### A6.1.2	Available implementation information

Many GSM-R networks have been rolled out within EU countries but they are still growing and other EU countries are still in the rollout process. The total railway network in EU is 221 025 km, of which 139 859 km of network are planned or in progress, 96 124 km of lines are now in service with some 75 000 terminals integrated in the dashboard of the trains (cab radios) within the EU.

Table A6.1.2-1

Country situation in CEPT (2014)

| Situation | Country |
| --- | --- |
| GSM-R network implemented and in operation, or in test/ trial phase | Austria, Finland, France, Germany, Norway, United Kingdom, Czech Republic, Greece, Italy, Lithuania, Portugal, Russian Federation, Slovak Republic, Spain, Sweden, Switzerland, Turkey, Belarus, Denmark, Hungary, Ireland, FYROM, Poland |
| GSM-R implemented in the national frequency plan but no network implementation yet | Bosnia Herzegovina, Croatia, Estonia, Latvia, Moldova, Montenegro, Serbia, Slovenia |

Figure A6.1.2-1

Country related implementation (2014)

Figure A6.1.2-1 includes the equipped distances of the railway tracks in the respective years with GSM-R technology. Portugal has up to now only 8 BS covering 40 km railways, Slovak Republic has only 17 BS covering 100 km and the Russian Federation is deploying GSM-R systems in the Krasnodar Territory and Kaliningrad Region.

Spain, United Kingdom: estimates from ETSI TR 102 627 (11/2008) are included in Figure A6.1.2-1.

Table A6.1.2-2

Information about GSM-R deployment in 2016

|  | a) What is the total length of rail tracks (length in km) in your country? | b) What amount of rail tracks (length in km) has been covered by GSM-R? | c) How much remains to be covered? | d) How many GSM-R base stations are currently deployed in your country? |
| --- | --- | --- | --- | --- |
| Czech Republic | 9500 km | 1500 km | 1200 km | 320 |
| Finland | 5944 | ≈5000 | None. Railway traffic will move from GSM-R to existing TETRA based network in the 400 MHz band from year 2018 (planned). | - |
| France | 30 000 km | 16 000 km of deployed GSM-R network 8 000 km of operational GSM-R coverage | The roll out of the GSM-R network infrastructure is done. The process to migrate the analogue system to GSM-R system is in progress (end planned in 2017) | 2 500 BTS |
| Germany | ca. 34 000 km | ca. 29 000 km | ca. 1 000 km | ca. 3 800 |
| Greece | about 2 500 km | about 690 km | 17 km | 96 |
| Hungary | 7 938 km | 935 km | approx. 2 394 km | 147 |
| Ireland | Approximately 2 400 km | Approximately 100 km | 2 300 | 42 |
| Italy | 16 726 Km | 11 166 Km | 5 560 Km | 1 825 |
| Liechtenstein | ~10 Km | ~10 Km |  | 1 |
| Lithuania | 1 779 | 1 563 | 216 | 127 |
| Netherlands | ~4 000 km | All | n/a | ~250 |
| Norway | Approximately 3 800 km. | Approximately 3 800 km. | Most of the Norwegian track is already covered. Only some low used side tracks are not covered, and they are not planned to be covered either. | Approximately 540 BTSs. |
| Slovak Republic | 3 625 | 400 | 3 225 | 64 |
| Slovenia | N/A | N/A | N/A | N/A |
| Sweden | 11 000 km | 11 000 km | 0 km | 1 400 base stations and 250 repeaters |
| Switzerland | ~5 380 km (normal gauge) | SBB ~2 050 km SBB including other companies ~2 280 km | SBB ~2 800 km SBB including other companies ~3 100 km | SBB ~650 (planed 840 more) SBB including other companies ~700 (planed 900 more) |
| United Kingdom | 15 500km | 15 108 km | All Network Rail controlled infrastructure has been fitted with GSM-R, with the exception of a few remote lines, where no fitment plans exist | 3 053 cells / 2 427 sites |

## A6.2	Train positioning information

The ERC Recommendation 70-03 Annex 4 for railway applications includes entries for:

–	Balise uplink (ground to train) systems including Eurobalise;

–	Loop uplink (ground to train) systems including Euroloop;

–	Balise telepowering and downlink (train to ground) systems including Eurobalise and activation of the Loop / Euroloop;

–	Obstruction/vehicle detection via radar sensor at railway level crossings.

The applicable national implementation status is also included in the ECO Frequency Information System (EFIS).

The technical requirements for spectrum use for these applications are also harmonised in the Commission Implementing Decision 2017/1483/EU.]

ANNEX 7

Study on spectrum needs of Railway Radiocommunication System between Train and Trackside (RSTT) with respect to the train radio applications

## A7.1	Introduction

With the development of RSTT, it is expected that a future integrated system itself will bear most of the four main applications of RSTT. At present, some organizations and research institutions in China are carrying out studies on future RSTT.

This study focuses on the spectrum needs of RSTT with respect to train radio application towards 2020 in a real deployment scenario, XIN FENG ZHEN area, which is located in Xi’an, China. It is a typical train operating area, which covers the 4 working scenarios defined in this Report, i.e. shunting yard, line, station and maintenance base. This study is based on the spectrum calculation methodology introduced by Recommendation ITU-R M.1768-1.

## A7.2	The development of RSTT in China

In China, GSM-R system (900 MHz) and wireless train dispatching system (450 MHz) provide most of the RSTT functions. GSM-R has been deployed for all high-speed line and some of the common speed lines. By the end of 2016, GSM-R had been operated for about 40,000 km of high-speed lines, which account for 35.8% of total railway mileage (124,000 km). Besides, 450 MHz wireless train dispatching system had been deployed over 80, 000 kilometres common speed lines in China. Figure A7.1-1 shows the development of railway mileage and transportation from 2003 to 2016. Figure A7.1-2 shows the growth of high-speed railway mileage in China from 2013 to 2016.

Figure A7.1-1

Railway mileage and transportation (2003~2016)

Figure A7.1-2

Length of high-speed railway mileage in China (2013~2016)

There is a trend of increasing of the length of railways, the volume of passenger traffic and goods traffic, as well as the length of high-speed railways. China plans to extend the length of railways to 175, 000 kilometres, including 38, 000 kilometres high-speed railways (account for 21.7%) by 2025. According to China railway’s medium and long-term development plan, the railway traffic would strongly increase approximately from 2020. In order to meet the increased railway service requirements, China Railway Corporation (CRC) has proposed the evolution of future RSTT, which is a FDD LTE-based system. Currently, CRC is planning to carry out a field trial of LTE-based RSTT in 2018. The test will be conducted on a high-speed line, from BEIJING to SHENYANG, and the system under test will be operated at 450 MHz frequency band (i.e., LTE band 31).

In order to provide improved railway traffic control, passenger safety and improved security for train operations in future, the functional requirements for future RSTT train radio should be defined to meet the operational needs of the railways, and are applicable for the increased volume of traffic and high-speed railway environment. Taking into account both UIC FRMCS User Requirements Specification and the future requirement of China railway, China proposes typical functional requirements of train radio application for next generation RSTT (please refer to Table A7-3).

## A7.3	Study on the Spectrum needs of RSTT

### A7.3.1	Methodology overview

In considering the evaluation of RSTT would be a LTE-based system. This study adopts ITU-R M.1768-1 Recommendation to estimate the spectrum needs of train radio application for future RSTT. In particular, most of the main applications of future RSTT would be realized by an integrated system in China. This contribution follows the procedures as outlined in ITU-R Recommendation M.1768-1, and a simplified and abbreviated form has been considered. For the voice, data and messaging are implemented using mobile IP services, the traffic distribution to different RATGs and capacity calculation for circuit switched (e.g., blocking probability, Poisson process of call arrival rate) are not necessary. As a result, the steps 5-7 in generic calculation flow recommended by ITU-R Recommendation M.1768 could be omitted in this example, and the spectral efficiency matrix could be assigned for one RATG.

### A7.3.2	Assumptions for the study

–	In this study, the voice, data and messaging are implemented using mobile IP services.

–	The present and future use of RSTT applications have been considered.

–	Aggregated traffic is calculated as a final result in busy hour.

–	Traffic classes related to RSTT applications are defined as four different QoS classes from the user’s perspective: conversational class of service; interactive class of service; streaming class of service and background class of service.

–	The service usage patterns of the users would not siginificantly change by 2020. Historical traffic statistic data of train radio application could be utilized. In this study, the arrival rate per user (sessions/(s • user)), mean service bit rate (bit/s) and mean session duration (s/session) were obtained from the calling record files.

–	When estimating the spectrum needs, only one cell coverage was considered.

–	The frequency range used for spectrum needs estimation of future RSTT train radio is 450 MHz band.

### A7.3.3	Calculation flow

The flow for the spectrum needs calculation of RSTT train radio is shown as follows.

–	Step 1: presents the different definitions used in the methodology, including the geographical area, service category (SC), service environment (SE) and which are given in Tables A7-2 to A7-5.

–	Step 2: lists the spectrum efficiency consideration in different SEs.

–	Step 3: the area spectral efficiency matrix (spectrum efficiency consideration) is assigned in Table A7-6 for one RATG.

–	Step 4: values for the calculation are given in the excel file embedded ( please refer to § 3.5), e.g., session arrival rate per user, mean service bit rate and mean session duration etc., which are obtained from the calling record files from China Railway Corporation.

–	Step 5: calculates aggregated spectrum needs (see the excel file embedded).

### A7.3.4	Input parameters

#### A7.3.4.1	Deployment Scenario

Figure A7.3.4.1-1 illustrates the geographical layout of the XIN FENG ZHEN area.

Figure A7.3.4.1-1

A geographical layout of XIN FENG ZHEN area

The geographical area of XIN FENG ZHEN is determined as

•	Length: 3.2 km Width: 0.45 km

•	Area: 1.6 km2

#### A7.3.4.2	 Service category

Table A7.3.4.2-1

Service categorization of RSTT in XIN FENG ZHEN area

| Traffic class Service type | Conversational | Interactive | Background |
| --- | --- | --- | --- |
| Medium Multimedia | SC1 |  |  |
| Low rate data and low multimedia | SC2 | SC3 | SC4 |

Table A7.3.4.2-2

Service types and their peak bit rates of RSTT in XIN FENG ZHEN area

| Service type | Peak bit rate |
| --- | --- |
| Medium Multimedia | < 2 Mbit/s |
| Low rate data and low multimedia | < 144 kbit/s |

Table A7.3.4.2-3

Typical functional requirements of train radio for future RSTT in China

|  | Service Name |  | Priority | Service categories | Service Characteristic |
| --- | --- | --- | --- | --- | --- |
| Railway operation Related service | Dispatching Communication | Voice Dispatching Communication | 0 | SC2 | voice/dispatch |
|  |  | Multimedia Dispatching Communication | 2 | SC1 | Multimedia/dispatch |
|  | C3/C2+ATO |  | 1 | SC2 | train control |
|  | Train tail information |  | 1 | SC3 | train control |
|  | Earthquake Alarm |  | 1 | SC2 | train control |
|  | Train security Alarm |  | 1 | SC2 | train control |
|  | Dispatching command |  | 2 | SC3 | train information |
|  | Train Radio Number check |  | 2 | SC3 | train control |
|  | Disaster monitoring system |  | 2 | SC3 | train information |
|  | Mobile ticketing |  | 3 | SC3 | train control |
|  | DMS |  | 3 | SC4 | train information |
|  | ZLJ Railway remote system |  | 3 | SC4 | train information |
|  | Train device monitoring |  | 3 | SC4 | train information |
|  | CMD | 6A Monitoring Data | 3 | SC4 | train information |
|  |  | LKJ Monitoring Data | 3 | SC4 | train information |
|  | Power supply monitoring system for HRC(6C) | Train device monitoring | 3 | SC4 | train information |
|  | Coach | TCDS | 3 | SC4 | train information |
|  |  | EMU status monitoring | 3 | SC4 | train information |
|  | Infrastructure | Train Moving Monitoring | 3 | SC4 | train information |
|  |  | Infrastructure health manage system | 3 | SC4 | train information |
|  | Radio communication for marshalling yard | Shunting Radio voice communication | 0 | SC2 | voice/dispatch |
|  |  | Train Number Radio voice communication | 0 | SC2 | voice/dispatch |
|  |  | Goods Radio checking voice communication | 0 | SC2 | voice/dispatch |
|  |  | Train checking radio voice communication | 0 | SC2 | voice/dispatch |
|  |  | Shunting Radio data communication | 1 | SC2 | train information |
|  |  | Goods Radio checking data communication | 2 | SC3 | train information |
|  |  | Train checking radio data communication | 2 | SC3 | train information |
|  |  | Train number radio data communication | 2 | SC3 | train information |
| Operation and Maintenance | Maintenance communication | Operation Maintenance voice communication | 3 | SC4 | voice/maintenance |
|  |  | Work card control system | 4 | SC4 | maintenance data |
|  | Maintenance base | Maintenance voice communication | 5 | SC2 | voice/maintenance |
|  |  | Maintenance work data system | 5 | SC4 | maintenance information |
|  | Railroad goods transport information system | management for goods and device | 4 | SC4 | maintenance data |
|  |  | Railway goods train state information | 4 | SC4 | maintenance data |
| Public safety | Police communication |  | 3 | SC2 | voice/public safety |

Table A7.3.4.2-4

Speed scenarios in XIN FENG ZHEN area

| Scenarios | Common speed ( km/h) | High-speed (km/h) |
| --- | --- | --- |
| Line | <200 | 200~350 |
| Normal station | 0~60 | 0 |
| Shunting yard | 0~30 | 0 |
| Maintenance base | 0 | 0 |

#### A7.3.4.3	Service environments (SEs)

Table A7.3.4.2-5

Identification of service environments in XIN FENG ZHEN area

| Tele density Service usage pattern | Shunting yard | Normal station/ Maintenance base | High-speed Line | Common speed Line |
| --- | --- | --- | --- | --- |
| public area | SE1 (voice, data) | SE2 (voice, data) |  |  |
| train/trackside |  |  | SE3 (voice, data) | SE4 (voice, data) |
| indoor |  |  |  |  |

#### A7.3.4.4	Consideration of Spectrum Efficiency

According to the field test results in high-speed railway (ZTE Corporation 450 MHz LTE system), the spectrum efficiency of uplink and downlink is about 1.2 bit/s/Hz and 1.95 bit/s/Hz at 250 km/h, respectively. Considering spectrum efficiency would vary with the speeds and operation environments, China provides an estimated spectrum efficiency matrix used for this study (see Table A7.3.4.4-1).

Table A7.3.4.4-1

Spectrum efficiency consideration (uplink/downlink) in different SEs

| Tele density usage pattern | Shunting yard | High-speed Line | Common speed Line | Normal station/ Maintenance base |
| --- | --- | --- | --- | --- |
| Uplink | 1.561.95 | 0.961.2 | 1.121.4 | 1.521.9 |
| Downlink | 2.0~2.5 | 1.6~2.0 | 1.76~2.2 | 1.84~2.3 |

### A7.3.5	Result of the study

This subsection gives a brief description of the calculation procedure and provides the relevant calculation table. It is assumed that a wide area network would be deployed below 1 GHz in order to reduce the number of necessary cell sites.

Notes to the EXCEL file above mentioned:

•	All services under the application of Train Radio was mainly categorized into Railway operation Related service, Operation and Maintenance, and Public safety.

•	Detailed services (e.g. dispatching communication, C3/C2+ATO, earthquake alarm etc. voice, data,) were listed and used to calculate accordingly.

•	The user number column was divided according to different SEs.

•	The statistics of progress arrival rate and average duration are estimated from real traffic analysis in 2012, and the analytical data are collected by traffic record files of China Railway Corporation.

•	While calculating, different spectrum efficiency parameters were selected, according to the spectrum efficiency matrix (See Table A7-6), for different SEs.

### A7.4	Conclusion

Under the conditions of this study, spectrum needs of RSTT with respect to Train radio application for the year 2020 at XIN FENG ZHEN area is:

–	Minimum spectrum needs: 11.9 MHz (uplink), 4.7 MHz (downlink)

–	Maximum spectrum needs: 14.04 MHz (uplink), 8.38 MHz (downlink)

Further, when deploy the RSTT network, more factors may need to be taken into consideration and some measures would be introduced to further improve the spectrum usage efficiency such as multi-cell layout in some of the deployment scenarios of RSTT.

ANNEX 8

Consideration of the doppler effect in railway radiocommunication systems between high-speed trains and tracksides

## A8.1	Introduction

WRC-15 (November 2015) adopted Resolution ITU-R СОМ6/12 (WRC-15) which included new item 1.11 in the agenda of the next WRC-19: "To take appropriate measures, as the case may be, facilitating harmonization of frequency bands on global and regional levels, in order to ensure operation of railway radiocommunication systems between train and tracksides within existing allocations to mobile service".

This Resolution "invites ITU-R to undertake a study of spectrum requirements, technical and operational characteristics and implementation issues for railway radiocommunication systems between train and tracksides". In addition, during the 2012-2015 study period, Chinese Administration submitted to the ITU-R WP 3K the Document R12-WP3K-C-0059 "Draft new ITU-R Question for SG 3. Methods for propagation prediction for high-speed train radio services". The document noted the need to study possible impact of the Doppler shift on the operation of railway radiocommunication systems between high-speed trains and tracksides. In the previous study period, the Chinese Administration submitted to ITU-R WP 3K the Document R12-WP3K-C-0093 "Preliminary draft new Recommendation ITU-R P.XXXX-VERSION. Method for propagation prediction for wireless radio systems on high-speed trains. This document also proposes to take into account possible impact of the Doppler shift on signal fading in railway radiocommunication systems between high-speed trains and tracksides.

The Documents R12-WP3K-C-0059 and R12-WP3K-C-0093 of the Chinese Administration did not get further consideration most likely due to the fact that there were no appropriate items in the WRC 15 agenda concerning the development of wireless radiocommunication systems for high-speed trains. Currently, agenda item 1.11 which is directly related to such systems is included in the Agenda of the forthcoming WRC-19.

The purpose of the contribution is the initial analysis of the need to take into account possible impact of the Doppler shift on the propagation prediction process for wireless communication links between high-speed trains and tracksides. This approach is based on approximate evaluation of the Doppler shift for different carrier frequencies and different train speeds. The evaluation of the Doppler shift is based on the known mathematical equation describing the Doppler Effect.

## A8.2	Evaluation of the Doppler shift for different frequency bands and different train speeds

The Doppler shift (Fd) for the case of moving objects (transmitter or receiver) relative each other with a certain speed (V), is calculated using the well-known equation:

Fd = ﴾│V×cos(θ)│×f﴿ /c,  Hz.	(1)

where:

V – 	speed of an object, m/s;

θ – 	angle between direction of an object motion (velocity vector) and the direction towards a stationary object, degrees (see Figure A8.2-1);

f – 	carrier frequency, Hz;

с – 	light velocity in free space, 3×108 m/s.

FIGURE A8.2-1

The Doppler shift measurement layout.

It can be seen from the equation (1) and Figure A8.2-1, that the Doppler shift has a maximum value when the angle θ has a minimum value, i.e. when the distance between two objects is sufficiently large. In general, this condition is mostly met as applied to the geometry of high-speed train motion, and the angle θ could be taken zero.

The Doppler shift was calculated using the equation (1) for train speeds in the range from 300 to 560 km/h and for carrier frequencies corresponding to the central frequency of the frequency bands allocated to the land mobile service in Region 1, i.e. 800 MHz, 1 800 MHz, 1 950 MHz, 2 100 MHz, 2 350 MHz, 2 595 MHz and 3 475 MHz. Calculated Doppler shifts for wireless radio systems between high-speed trains and tracksides are summarized in Table A8.2-1 and shown in Figure A8.2-2. It should be noted that Doppler shifts in Table A8.2-1 are absolute values. That is total Doppler shift in wireless radio systems between high-speed trains and tracksides will be twice the value of Table A8.2-1.

ТАBLE A8.2-1

Calculated Doppler shift in wireless radio systems between high-speed trains and tracksides

| Train speed V, km/h | Carrier frequency f, GHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.8 | 1.8 | 2.1 | 2.235 | 2.595 | 3.475 | 45 | 97 |
|  | Doppler shift Fd , kHz |  |  |  |  |  |  |  |
| 300 | 0.222 | 0.5 | 0.583 | 0.653 | 0.721 | 0.965 | 12.5 | 26.9 |
| 320 | 0.237 | 0.533 | 0.622 | 0.696 | 0.769 | 1.03 | 13.3 | 28.7 |
| 340 | 0.252 | 0.567 | 0.661 | 0.74 | 0.817 | 1.094 | 14.2 | 30.5 |
| 360 | 0.267 | 0.6 | 0.7 | 0.783 | 0.865 | 1.158 | 15.0 | 32,3 |
| 380 | 0.281 | 0.633 | 0.739 | 0.827 | 0.913 | 1.223 | 15.8 | 34.1 |
| 400 | 0.296 | 0.667 | 0.778 | 0.87 | 0.961 | 1.287 | 16.7 | 35.9 |
| 420 | 0.311 | 0.7 | 0.817 | 0.914 | 1.009 | 1.351 | 17.5 | 37.7 |
| 440 | 0.326 | 0.733 | 0.856 | 0.957 | 1.057 | 1.416 | 18.3 | 39.5 |
| 460 | 0.341 | 0.767 | 0.894 | 1.001 | 1.105 | 1.48 | 19.2 | 41.3 |
| 480 | 0.356 | 0.8 | 0.933 | 1.044 | 1.153 | 1.544 | 20.0 | 43.4 |
| 500 | 0.37 | 0.833 | 0.972 | 1.088 | 1.201 | 1.609 | 20.8 | 44.9 |
| 520 | 0.385 | 0.867 | 1.011 | 1.131 | 1.249 | 1.673 | 21.7 | 46.7 |
| 540 | 0.4 | 0.9 | 1.05 | 1.175 | 1.298 | 1.738 | 22.5 | 48.5 |
| 560 | 0.415 | 0.933 | 1.089 | 1.219 | 1.346 | 1.802 | 23.3 | 50.3 |
| 580 |  |  |  |  |  |  |  | 52.1 |
| 600 |  |  |  |  |  |  |  | 53.9 |

FIGURE A8.2-2

The Doppler shift in a wireless radio system between high-speed trains and tracksides 
vs train speed for different carrier frequencies

Let us see Figure A8.2-3 to analyze the impact of the Doppler shift on the operation of land mobile radio systems in a high-speed train.

FIGURE A8.2-3

Analysis of impact of the Doppler shift on the operation of wireless radio systems 
between high-speed trains and tracksides

Let us assume that a user terminal (UT) is located in a high-speed train, and a base station (BS) is fixed. Let us also assume that for the certain moment of time radio communication is established using radio frequency f0n between UT and BS. Due to the Doppler effect, fixed BS will receive UT carrier frequency f0n subject to the Doppler shift Fd, i.e. f0n + Fd. Accordingly, the bandwidth of the wanted UT signal will be also shifted to the right on the frequency scale by the same value Fd, as it is shown in Figure A8.2-3. Since the channel carrier frequencies are located in such a way that their spectra are side-by-side to each other (as it is shown in the upper part of Figure A8.2-3), then with such Doppler shift of n-th channel to the right, a part of the wanted spectrum of UT channel could fall into the bandwidth of the adjacent (n+1)-th BS channel receiver (as it is shown in the lower part of Figure A8.2-3). This share of the wanted signal power of n-th channel in the bandwidth of the adjacent (n+1)-th channel is highlighted by shadowing in the lower part of Figure A8.2-3. It is obvious, that the share of this power is proportional to the shift of the wanted signal spectrum, that is, to the Fd. Therefore, this share of the power (percentage of the total power of the wanted signal spectrum) could be evaluated by the ratio of the Doppler shift Fd to the spectrum of wanted signal Δfs (assuming that the bandwidth of the BS receiver is equal to this value). Table A8.2-2 shows such evaluations for different standards of land mobile communications (and, accordingly, for different central frequencies and parameters of signal) with maximum train speed (500 km/h, or 139 m/s).

TАBLE A8.2-2

Summary of calculated impact of the Doppler shift on wireless radio systems 
between high-speed trains and tracksides

| Land mobile system (center frequency, МHz) | GSM-R (800) | GSM-1800 (1 800) | UMTS- 2100 (2 100) | WiMAX (2 350) | WiMAX (2 595) | WiMAX (3 475) | LTE (1 950) | LTE (2 350) | LTE (2 595) | mmw (45) | mmw (97) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| max(Fd), (kHz) | 0.37 | 0.833 | 0.972 | 1.088 | 1.201 | 1.609 | 0.904 | 1.088 | 1.201 | 20.8 | 44.9 |
| Δfs (MHz) | 0.2 | 0.2 | 3.84 | 9.2 | 9.2 | 9.2 | 9.0 | 9.0 | 9.0 | 35 | 250 |
| max(Fd)/Δfs, (%) | 0.185 | 0.417 | 0.025 | 0.012 | 0.013 | 0.017 | 0.01 | 0.012 | 0.013 | 0.059 | 0.00018 |

## A8.3	Conclusions

Based on data from Table A8.2-1, Table A8.2-2, and Figure A8.2-2 we can make the following conclusions:

–	carrier frequency shift due to the Doppler effect becomes noticeable only for train speeds about 500 km/h and for frequency 2 GHz and higher. Carrier frequency is shifted by ±1.609 kHz from its nominal value (for the WiMAX land mobile system) and by ±20.8 kHz for mmw;

–	although carrier shift at 97-GHz band is larger than that of lower frequency, the Doppler shift is compensated by the carrier tracking method due to larger symbol rate than
100 Mb/s;

–	taking into account the Doppler shift of the carrier frequency and bandwidth of one radio channel in the up-to-date land mobile systems, the share of the UT signal power falling into the bandwidth of receiver of adjacent BS channel does not exceed 0.18% for all considered standards of land mobile systems;

–	when developing propagation models for radio communication scenarios between 
high-speed train and trackside, the effect of the Doppler shift is negligible to be taken into account.

______________

____________________

For FCC Part 90.35 (50), the effective radiated power (erp) must be less than or equal to 44.77 dbm (30 watts). This can be accomplished by adjusting the output power of the radio and selecting an antenna with appropriate gain. For FCC Part 90, the maximum allowable station erp is dependent upon the station's antenna haat and required service area, specific configuration and limits are railroad and site specific.

Depends upon selected channel separation (12.5 kHz narrowband or 6.25 kHz ultra-narrowband).

Multiplexing mode depends upon specific radio and configuration.

Depends upon selected configuration (16 KBPS TO 32 KBPS).

Jernbaneverket have applied for a license for use of UHF channels for 2017. In the application it is indicated that Jernbaneverket are looking into ending their use of UHF band for analogue radio but they will not be able to draw any conclusions until 2017

Jernbaneverket have indicated that:

–	that their preferences are likely to be following a harmonized European approach on use of frequencies for future rail radio systems (Future Railway Mobile Communication System – FRMCS);

–	that they are most likely to adopt a position which is in line with views that has been expressed by other GSM-operators in Europe regarding the successor system for GSM R should consider the opportunity to enlarge the market size and product ecosystem by following the main mobile standards used by commercial operators and base themselves on commercially available carrier sizes;

–	that they have a preference for maintaining their use of the 876-880 / 921-925 MHz band and add bandwith from the band below the current GSM-R band to be able to operate on a 5 MHz carrier basis;

–	that it will be necessary to find a migration solution that are most likely to implicate use of additional spectrum in a transposition period to be able to change technology and at the same time be able to operate trains in Norway.

This new system is under definition in this moment by UIC (Railways International Union) and ERA (European Railway Agency).

Standard 1: 3GPP: TS 36.2xx, TS 36.3xx, TS 36.4xx (t.b.d) Standard 2: European Union Agency for Railways: FRMCS System Requirements Specification (under construction)

http://www.tta.or.kr/eng/new/standardization/eng_ttastddesc.jsp?stdno=TTAK.KO-06.0370

http://www.tta.or.kr/eng/new/standardization/eng_ttastddesc.jsp?stdno=TTAK.KO-06.0369

http://www.tta.or.kr/eng/new/standardization/eng_ttastddesc.jsp?stdno=TTAK.KO-06.0437

http://www.tta.or.kr/eng/new/standardization/eng_ttastddesc.jsp?stdno=TTAK.KO-06.0438

Figures from a data collection performed in 2015.

ITU-R Recommendation M.1768-1 recommended a methodology for the calculation of terrestrial spectrum requirement estimation for International Mobile Telecommunications (IMT), and could be used for other public land mobile radio systems.