| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/236(Rev.1) | Annex 23 to Document 5A/650-E |
|  | 20 November 2017 |
|  | English only |
| Annex 23 to Working Party 5A Chairman’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLAN SHARING 5150-5250 MHz] |  |
| Sharing and compatibility studies of WAS/RLAN in the 5 150-5 250 MHz frequency range |  |

[Editor’s note: This document is a compilation of material presented in contributions submitted to May and November 2016, and the May and November 2017 WP 5A meetings (see Source Indication below) that the submitting administrations requested to be considered in developing this document. The content of this document need to be supported by corresponding sharing studies. The material contained in this document has not been agreed by WP 5A. The material if agreed could be used to satisfy the objective of agenda item 1.16.]

[Editor’s note: Document 5A/554 is embedded below for reference.]

# 1	Introduction

This Report includes the sharing and compatibilities studies of WAS/RLAN in the
5 150-5 250 MHz frequency range.

It is intended to represent the response to a part of invites ITU-R c) “to perform sharing and compatibility studies between WAS/RLAN applications and incumbent services in the frequency band 5 150-5 350 MHz with the possibility of enabling outdoor WAS/RLAN operations including possible associated conditions” of Resolution 239 (WRC-15) under WRC-19 agenda item 1.16.

# 3	Overall view of allocations in the 5 150-5 250 MHz range

| Allocation to services |  |  | Expected studies |
| --- | --- | --- | --- |
| Region 1 | Region 2 | Region 3 |  |
| 5 150-5 250 FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile 5.446A 5.446B AERONAUTICAL RADIONAVIGATION 5.446 5.446C 5.447 5.447B 5.447C |  |  | Coexistence between WAS/RLAN outdoor operations and FSS (feederlinks for non-GSO) and Aeronautical Radionavigation |

# 4	Assumptions on technical and operational elements for the sharing and compatibility of WAS/RLAN with other services

## 4.1	Technical and operational characteristics of the WAS/RLAN operating in the 5 150- 5 250 MHz ranges

[Editor’ note: The text below needs to be modified after finalization of the document Report ITU-R M.[RLAN REQ-PAR]]

[Option 1

[RUS 5A/196]

Technical and operational characteristics of RLANs are presented in Recommendation ITU-R M.1450 «Characteristics of broadband radio local area networks».In Canada, the e.i.r.p. of RLANs operating in the frequency band 5 150-5 250 MHz is 250 mW conducted (-6 dBW).  In the U.S., the e.i.r.p. of RLANs operating in the frequency band 5 150-5 250 MHz is 1 000 mW conducted (0 dBW), however outdoor operations with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW e.i.r.p., and all WAS/RLAN emissions outside of that band must be below -27 dBm/MHz.  At the same time RLANs operating in the territory of Europe, and in numerous Region 3 countries including Australia, are restricted to an e.i.r.p. of 200 mW (-7 dBW) in the frequency bands 5 150-5 250 MHz and indoor only operation.

e.i.r.p. spectral densities specified in Recommendation ITU-R М.1450 shows that it addresses RLANs having carrier bandwidth of 20 MHz. However taking in account the achievements in RLANs development such as IEEE standard 802.11ac, the considered Report includes analysis of networks having carrier bandwidth of both 20 MHz and 160 MHz.

[UK and ESA 5A/246, 96]

Option 2

### 4.1.1	Characteristics of RLAN in 5 150-5 250 MHz Band

## ]4.2	Technical and operational characteristics of FSS links used for MSS feeder links in the 5 150-5 250 MHz

[Globalstar 5A/395]

The parameters of the feeder uplinks of the HIBLEO-X MSS system are summarized in the Table below.

TABLE 2

MSS Feeder Link Parameters

| Parameter | HIBLEO-X |
| --- | --- |
| Satellite orbit altitude h (km) | 1 414 |
| Satellite Inclination (degrees) | 52 |
| Frequency Range (MHz) | 5 091-5 250 |
| Satellite receiver bandwidth B (MHz) | 1.23, 16.5, 19.38 |
| I/N (dB) for time invariant sources of interference | -12.2 |
| I/N (dB) for time variant sources of interference | For further study |
| Satellite receiver noise temperature T (K) | 550 |
| Polarization discrimination Lp (dB) | 1.4 |

The spacecraft receive antenna is an “iso-flux” antenna and the gain pattern is shown below.

Figure 3

Spacecraft Receive Antenna Pattern

## 4.3	Technical and operational characteristics of the Aeronautical Radionavigation service operating in the 5 150-5 250 MHz

[Editor’s Note: Parameters below need confirmation from WP 5B]

[RUS 5A/397]

In accordance with Recommendation ITU-R М.2007, ARNS systems operate in the frequency band 5 150−5 250 MHz all over the world. In compliance with recommends 1 Recommendation ITU-R M. 2007 that the technical and operational characteristics of the radars operating in the ARNS described in Annex 1 should be considered representative of those operating in the frequency band 5 150-5 250 MHz and used in studies of compatibility with systems in other services. Table 1 below provides the technical parameters of airborne sense and avoid systems to be used for consequence assessment of outdoor WAS/RLAN usage.

TABLE 1

Technical parameters of aircraft based sense and avoid radar

| Parameter |  | Radar No. 1 |
| --- | --- | --- |
| Platform height (km) |  | Up to 20 |
| Radar type |  | Air to air traffic collision avoidance system |
| The range of measured ground speed (km/h) |  | Up to 1 500 |
| Frequency tuning range (MHz) |  | 5 150-5 250 |
| Emission type |  | Linear FM (LFM) pulse |
| LFM chirp bandwidth (MHz) |  | 20 |
| Pulse rise and fall times (s) |  | 0.1-0.2 |
| RF emission bandwidth | –3 dB –20 dB (MHz) –40 dB | 18 22 26 |
| Receiver IF –3 dB bandwidth (MHz) |  | 30 |
| Receiver noise figure (dB) |  | 5 |
| Antenna gain (dBi) |  | 33-36 |
| First antenna side lobe (dBi) |  | 18-20 |
| Horizontal beamwidth (degrees) |  | 8 |
| Vertical beamwidth (degrees) |  | 8 |
| Polarization |  | Vertical |
| Vertical antenna scan (degrees) |  | ±45 |
| Horizontal antenna scan (degrees) |  | ±45 |
| Protection criteria (dB) |  | −6 |

## 4.4	Technical and operational characteristics of aeronautical mobile service systems limited to aircraft transmissions of aeronautical mobile telemetry (AMT) for flight testing in the frequency ranges 5 150-5 160 MHz

[Editor’s Note: Parameters below need confirmation from WP 5B]

The following table provides the aeronautical mobile telemetry transmitting and receiving stations characteristics relevant for preforming sharing analysis with other services in co-frequency.

Table 1

Aeronautical mobile telemetry characteristics

| Transmitter (onboard aircraft) |  |  |
| --- | --- | --- |
| Frequency range | MHz | 5 091-5 160 |
| Channel bandwidth | MHz | 8 |
| Modulation |  | Single Carrier-SOQPSK or COFDM-QPSK |
| Maximum transmit power 1 | dBW | 20 |
| Aircraft antennas location |  | One antenna on the bottom of the aircraft and another antenna on the top of the aircraft |
| Tx antenna gain | dBi | 0 |
| Cable loss | dB | 2 |
| Aircraft altitude | m | 0 – 15 000 |
| Aircraft deployment |  | Typical: 3 Aircraft in flight at the same time but not co-frequency (each aircraft use different channels) Maximum: 5 Aircraft in flight at the same time but not co-frequency (each aircraft use different channels) |
| Receiver (on ground) |  |  |
| Antenna pattern |  | Steering Parabolic antenna Recommendation ITU-R S 580-6 |
| Receiver antenna gain | dBi | 40 |
| Receiver noise temperature | K | 310 |
| Receiver altitude from ground level | m | Between 6 and 40 |
| Receiver antenna elevation range | ° | Between -5 and 90 (99% of time the elevation is between -2° and 5°) |
| Protection criteria I/N | dB | -6 |
| 1 The effective power is adjusted to comply with the pfd limits defined in Annex 1 of Resolution 418 (Rev.WRC-15). |  |  |

# 5	Sharing studies per service

## 5.1	Sharing and compatibility of MSS feeder links versus WAS/RLANs in the 5 150-5 250 MHz band

[Editor’s note: Further discussions and potential future input contributions are invited to improve the text below extracted from Documents 5A/381 and 5A/404]

[AUS 5A/81]

The current, worldwide sharing rules, in the lower 5 150-5 250 MHz band, to protect co-band non-GSO MSS E-to-s feeder links, appear in Resolution 229 (Rev.WRC-12) which, inter alia, requires individual WAS/RLAN transmitters to “be restricted to indoor use with a maximum mean e.i.r.p. of 200 mW and a maximum mean e.i.r.p. density of 10 mW/MHz in any 1 MHz band or equivalently 0.25 mW/25 kHz in any 25 kHz band”.

Further background on the development of Resolution 229 (Rev.WRC-12), together with information on related ITU-R Recommendations, can be found in Document 5A/81 from Australia.  Importantly, Resolution 229 (Rev.WRC-12) assumes that only 1% of RLAN deployments would operate outdoors and that the aggregate noise from WAS/RLANs into victim non-GSO MSS E-to-s feeder link satellite receivers would likely come from multiple countries.

[USA 5A/381]

The Resolution 229 (WRC-12) indoor use restriction on WAS/RLAN transmitters was based on the assumption that many different MSS companies would share the 5 150-5 250 MHz band, whereas today there is only one satellite operator that operates feeder link stations in the 5 096-5 250 MHz band.

[Editor’s note: Studies supporting the text below have not been received yet.]

[Studies conducted by one administration concluded that the noise floor increase seen by the satellite will be a function of the aggregated energy from WAS/RLAN emissions at elevation angles above 30 degrees. By applying technological measures to operations above this elevation angle, the energy that will be received by the satellite from each individual access point would be sharply reduced, resulting in reduced aggregate noise at the satellite.  As a result, it is far less likely that harmful interference will occur, even with proliferation of access points greater than that originally presumed.

Permitting fixed access point outdoor operations at a conducted power level of up to 1 W (30 dBm), and a PSD of 17 dBm/MHz with an allowance for a 6 dBi antenna gain (i.e. a total 36 dBm e.i.r.p.), and limiting the maximum e.i.r.p. above 30 degrees elevation to 125 mW (21 dBm) e.i.r.p., provides reasonable protection from harmful interference to the MSS system. ]

Expressing this limit in terms of e.i.r.p. provides flexibility regarding how to design WAS/RLAN equipment, while still achieving the required levels of protection.  WAS/RLAN manufacturers will be able to demonstrate compliance with the e.i.r.p. limit by reducing antenna gain in the upward direction, or by limiting the transmitter power, or a combination of the two, as best suits their particular purpose.  Additionally, the national authority implemented a reporting requirement on any widespread deployments of outdoor access points and required WAS/RLAN operators to take corrective action in the event of any claims of harmful interference, to include reducing power, turning off devices, changing frequency bands, and/or further reducing power in the vertical direction.  To date, none such claims of interference have been made. The power limits above 30 degrees described above for individual devices, combined with the filing requirement for deployments of large numbers of devices will provide a sufficient means for avoiding harmful interference and addressing it if it does occur.  It is important to note that while in-band WAS/RLAN emissions were increased, emissions outside that band were maintained at a level of 
-27 dBm/MHz.

[AUS 5A/404]

In its Document 5A/404 contribution, one administration (as a major operator of LEO-D, non-GSO MSS feeder uplinks in the 5 150-5 250 MHz band) raised concerns with the Doc. 5A/210, as has now been now been reflected in the above text.  This administration provided the following Tables to compare the domestic rules described in Document 5A/210 for RLANs in this frequency band and the mandatory requirements of Resolution 229 (Rev.WRC-12).

Table 1 provides a comparison of the US and Resolution 229 (Rev.WRC-12) rules for RLAN emission elevation angles less than or equal to 300 and Table 2 for RLAN emission elevation angles greater than 300 elevation.

TABLE 1

RLAN emission elevation angles between 00 and 300

| Parameter | Resolution 229 (Rev.WRC-12) | USA | Difference |
| --- | --- | --- | --- |
| Maximum e.i.r.p. | 200 mW (23 dBm) | 4 W (36 dBm) | 13 dB |
| Location constraint | Yes, indoor only | No, outdoor permitted |  |
| Resultant max. outdoor e.i.r.p. | 6 dBm* | 36 dBm | 30 dB* |

* Assumes building loss of 17 dB

TABLE 2

RLAN emission elevation angles >300

| Parameter | Resolution 229 (Rev.WRC-12) | USA | Difference |
| --- | --- | --- | --- |
| Maximum e.i.r.p. | 200 mW (23 dBm) | 125 mW (21 dBm) | -2 dB |
| Location constraint | Yes, indoor only | No, outdoor permitted |  |
| Resultant max. outdoor e.i.r.p. | 6 dBm* | 21 dBm | +15 dB* |

* Assumes building loss of 17 dB

[Editor’s note: These are views from administrations rather than technical material.]

One administration noted that the domestic rules described in Document 5A/210 that apply to RLANs in the 5 150-5 250 MHz frequency band potentially result in up to 30 dB (i.e. 1,000 times) more radiated power for RLAN emission elevation angles ≤300 and up to 15 dB more radiated power for elevation angles >300 when compared with that prescribed in Resolution 229 (Rev.WRC-12).

[One administration also noted that the Document 5A/210 had not provided any technical, operational, sharing or compatibility studies to support the e.i.r.p. increase or for removing the indoor only requirement. Further, the choice of a 300 elevation angle breakpoint for maximum e.i.r.p. was not supported by reference to any studies and was inconsistent with the operation of the Australian LEO-D feeder uplinks which carry commercial traffic from 100 to the opposite 100 in elevation.

This administration asked the contributor of Document 5A/210 to advise WP 5A how the 300 elevation angle was chosen and to provide an analysis of the aggregate noise that would be received by LEO-D as a result of RLANs operating outdoors at the 21 dBm e.i.r.p. level (and also at the 36 dBm e.i.r.p. level). ]

### 5.1.1	Study 1 (USA 5A/534)

#### 5.1.1.2	Sharing Cases

[Editor’s Note: Beamforming was not used in this simulation, but future studies may be available to take account of such technologies.]

Actual real-world system interactions and impacts are often not available when considering new allocations or where conditions of use are concerned. Often, only simplified, assumption-driven analyses outlining worst case interference risks are provided. A better technical understanding of risks is crucial when attempting to expand access for new uses. Consistent with Res. 239, we must also consider further new developments in RLAN technologies, such as beam forming that will allow for lower transmit power and provide higher directivity, improving the interference condition over time.

Consistent with Res. 239, this study attempts to provide as accurate and realistic analysis on the impact of band sharing between RLANs and MSS given our present understanding of all parameters, as described in this document, assuming RLANs  parameters in Table 1A in annex 27 of W P5A chairman’s report (5A/469).

There are two cases of possible interference that could result when RLANs and FSS feeder links operate simultaneously in the 5 150-5 250 MHz band.

##### 5.1.1.2.1	Aggregate RLAN Transmitters-to-FSS Feeder Uplink of MSS Satellite System

In the first case we will undertake a sharing analysis to ascertain the impact of incidental Earth-to-space transmissions of the aggregate of RLAN devices operating within line of sight of the FSS feeder antenna.

##### 5.1.1.2.2	MSS Gateway Transmitter-to- RLAN Receiver

It might also be appropriate to examine interference by MSS gateway transmitters to RLAN receivers. However, while this form of interference may be possible, the number of these gateways is very small. It is possible to establish exclusion zones utilizing standard procedures around the small number of gateways, since they are fixed. This, however, is not the focus of this study and will not be treated further.

#### 5.1.1.3	Technical Characteristics

The following sections summarize the FSS/MSS and RLAN parameters as considered in this study.

##### 5.1.1.3.1	Characteristics of MSS System

This contribution will focus on the study of the MSS system referred to as LEO-D in a number of ITU-R Recommendations and Reports. The system consists of 34 active spacecraft at an altitude of 1 414 kilometers and an inclination angle of the orbits of 52 degrees, with respect to the Equator. The spacecraft antenna has a full Earth-coverage beam with an approximate radius of 2 900 kilometers (0 dBi contour) on the surface of the Earth.

This system can be characterized as a “bent-pipe satellite system” (see Figure 1). In the forward path the feeder uplink uses the C-band between the gateway and the satellites to transmit eight 16.5 MHz channels on each right hand and left hand circular polarizations. The sixteen channels are received at the satellite transponder, converted to S-band and each mapped to one of sixteen downlink service beams. Within each of the beams, there are thirteen 1.23 MHz frequency division multiplexed (FDM) carrier channels. Each 1.23 MHz carrier employs code division multiple access (CDMA) to provide multiple voice or data circuits to user terminals. The return path is provided in a similar manner with L-band employed on the service uplink and C-band on the feeder downlink; however, RLANs operating in the 5 150-5 250 MHz band overlap the C-band frequencies utilized in the forward path and thus are the focus of this analysis.

Additional MSS system characteristics as provided throughout this document are taken from several sources and noted.

Figure 1

Single MSS satellite system with Feeder Links at 5.1-5.2 GHz

Most recent parameter values of the MSS satellite system under study were extracted from an MSS study included in WP 5A Document 5A/395 or other sources, and are summarized in the Table 1 and Figure 2.

Table 2

Feeder Up-link Parameters

For the purposes of this study, the -12.2 dB value was used for comparison reasons only. The I/N protection value for the FSS feeder link applicable to time variant source of interference such as RLANs is still under consideration by WP 4A.

The spacecraft receive antenna is an “iso-flux” antenna, and the gain pattern is shown below.

Figure 2

Spacecraft receive antenna pattern (feeder loss included)

##### 5.1.1.3.2	Characteristics of RLANs

In order to accurately calculate the aggregate power contributed by the multitude of RLANs within the large coverage footprint of the FSS feeder antenna for each MSS satellite, the area is subdivided into grids so that factors such as antenna discriminations, path loss, busy hour, clutter loss, and building entrance loss can be accounted for in a reasonably high resolution manner thereby accurately representing the link interference contribution. To this end the Oak Ridge National Laboratory’s (ORNL) LandScan™ provides 30" X 30" grid resolution (approximately 1 km2) global population distribution data. In order to have a more manageable number of population centers and compute the area associated with a given population sample, granularity of this data was reduced. The grids associated with the new population centers vary in size from 14 km2 to 50 km2 and results in a grid count of 278,248. Characteristic location and population data for each grid is utilized to compute its RLAN interference contribution to the FSS feeder link.

For the purpose of applying representative parameters the analysis categorizes each grid as one of three demographics: urban, suburban, and rural based on population density of the grid. During the JTG one administration proposed a distribution of populations over a defined area for the three demographics. Using this information we define grids in the following manner:

–	Rural, for grids with population density less than or equal to 495 people/km2;

–	Suburban, for grids with population density greater than 495 but less than or equal to 4 177 people/km2;

–	Urban, for grids with population density greater than 4 177 people/km2.

For this analysis the study assumes that interference from WAS/RLAN would be primarily from RLANs and more specifically from access points (AP) in RLAN systems. The RLAN parameters given below are based upon this assumption. As there are potentially millions of RLAN access points in the 5 150-5 250 MHz band, it is impractical to simulate each access point as an individual interferer. Hence, the power from the individual access points has been aggregated as an input to the satellite receiver.

###### 5.1.1.3.2.1	RLAN e.i.r.p level distributions

Table 2 provides a summary of e.i.r.p. level distributions from R15-WP5A-C-0469, Annex 27, M.[RLAN REQ-PAR] Table 1a. The e.i.r.p level distributions were developed for RLANs operating in the 5 725-5 850 MHz band in both indoor and outdoor environments.

Percentage of devices in each environment was calculated to be 94.7% and 5.3% for indoor and outdoor respectively (see Table 2). These are the values applied to grid RLAN device counts to so that the appropriate e.i.r.p. and accompanying losses are applied when calculating aggregate interference to the satellite system.

Table 2

RLAN e.i.r.p level distributions

###### 5.1.1.3.2.2	RLAN antenna discrimination

As has been previously noted, when considering the RLAN antenna discrimination to be used in the sharing studies, the following factors should be considered:

–	For simplification, only AP emissions are considered in the technical studies, but it is recognized that this is based on the fact that either the AP or the client terminal is transmitting. Therefore, in order to accurately represent this condition, a composite of AP and terminal antennas is employed.

–	In addition, when considering the discrimination of the AP antennas, positioning needs to be considered. Indoor antennas may be located on the ceiling facing downward, on the wall facing horizontal or on a desk facing upward. Outdoor antennas may be pole mounted, strand mounted or wall mounted and likewise may face up, down or horizontal. A composite discrimination value in the direction of the satellite is assumed, based on the patterns resulting from the various antenna placements.

–	While client terminal antennas may have some gain greater than zero, it is impossible to determine which direction they are facing during each transmission, so for simplicity’s sake it is assumed that over time they appear omnidirectional with 0 dBi gain. However, terminal antennas are likely to be blocked by the user from the satellite approximately 50% of the time. A proposed value for body loss of 4 dB is provided in Table 4 of Report ITU-R M.2292 and is representative of all different user cases (i.e. speech position, browsing position, etc.).

An analysis was done taking into account the above considerations with the following assumptions:

–	For indoor AP antennas both consumer and enterprise omni-antenna patterns from the United Kingdom, Document 5A/246, were used. All consumer APs were assumed to be desk mount, 80% of enterprise devices were split between ceiling and surface mount and the other 20% were wall mount with a 50/50 split between ±90o rotation.

–	For outdoor AP antennas, omni-directional patterns typical of those used by some U.S. cable operators were used along with the directional antennas, described in Document 5A/469 Annex 29, pointed in the horizontal direction. The outdoor omni devices were split 40% facing up, 40% facing down and 20% directional facing horizontally.

The result was a weighted average loss toward the satellite; i.e., above 0 degree elevation, of 1.8 dB.

###### 5.1.1.3.2.3	RLAN antenna heights

Table 3 provides a summary of the RLAN access point antenna heights. The antenna heights were extracted from R15-WP5A-C-0114 Annex 24, an earlier version of M.[RLAN REQ-PAR]. The antenna heights are employed along with elevation angle to determine clutter loss to apply to the propagation path. In determining clutter loss non-client antenna heights are randomly selected, using a uniform probability, in 3 meter steps distributed across the range shown. For practical reasons the height randomly selected is applied on a grid basis.

Table 3

RLAN antenna heights

###### 5.1.1.3.2.4	Path loss

Path loss is characteristic for each grid to satellite path defined by a number of contributing factors and accounted for in the following formula:

LG  = Lb + LCES + LBEL + LX	(1)

where:

Lb:	Transmission loss for slant distance computed;

LCES:	Earth to space clutter loss;

LBEL:	Building entrance loss;

Lx:	Cross-polarization discrimination.

Each of these factors is discussed below.

A liaison statement from ITU-R Working Party (WP) 3K and 3M to Document 5A/337 provides the following path loss advice for conducting agenda item 1.16 studies:

–	With regard to the propagation model, Recommendation ITU-R P.619 should be used for earth-to-space paths.

–	For building entrance loss Recommendation ITU R P.[BEL] (see Document 3/57(Rev.1)) should be used.

–	For clutter Recommendation ITU R P.[CLUTTER] (see Document 3/51(Rev.1)) should be used. While the lower limit of the frequency does not include 5 GHz at this time, progress is being made to extend this. The current frequency range of applicability of section 3.3 of draft new Recommendation ITU-R P.[CLUTTER] is 10-100 GHz; however, if the deployment scenario is similar to that in section 3.3 of draft new Recommendation ITU R P.[CLUTTER] and in draft new Report ITU-R P.[CLUTTER_REP] (see Document 3/52), the model could reasonably be applied to frequencies as low as 5 GHz but limited to suburban and urban environments and antenna heights up to 6 meters. It is expected that extending draft new Recommendation ITU-R P.[CLUTTER] down to 5 GHz would provide more accurate results than Recommendation ITU-R P.452.

Provided with a note from Chairmen of Study Group 3 and Working Parties 3J, 3K and 3M to Chairman of Task Group 5/1 and various working parties (see Document 5A/357) is an embedded attachment containing an spreadsheet implementation of P.[Clutter] and P.[BEL].

However with respect to the above the following editor’s note was provided in Document 5A/469 Annex 27, M.[RLAN REQ-PAR]:

Editor’s note: Guidance was received from WPs 3K and 3M as shown below, WP 5A is seeking further clarification on applicability of clutter loss in 5 GHz range.

Transmission Loss

Transmission loss is computed for slant distance, and climate and other factors specific to each grid. Recommendation ITU-R P.619 provides the following propagation model for calculating transmission loss and can be expressed as:

Lb = 92.5 + 20 log f + 20 log d + Ag + AD – GS   dB	(2)

where:

ƒ:	frequency (5.2 GHz);

d:	path length (km), slant distance from center of grid to satellite;

Ag:	attenuation due to atmospheric gases (0.6 dB);

AD:	attenuation (dB) due to beam spreading (0 dB);

GS:	“gain” (dB) due to scintillation (0.65 dB).

Note that ITU-R P.619 references a number of other recommendations that provide values for the last three variables:

Ag may be approximated from Figure 6 and equation 28 of Recommendation ITU-R P.676-11, assuming a 45o elevation, as 0.6 dB.

The loss AD due to beam spreading may be calculated from equation 40 of Recommendation ITU-R P.618-12, with the following note: in regular refractive conditions, AD can be ignored at elevation angles above about 3° at latitudes less than 53°. Therefore it is assumed for the purpose of this study that AD is equals 0 dB.

Referring to Figure 12 and employing methods described in Recommendation ITU-R P.531-13, including its Figure 12, Pfluc for 99% of the time was averaged over 6 years, adjusted for frequency and translated to gain/loss. The result is GS = 0.65 dB.

Clutter Loss

The scenario (Earth/ to space) under study is in accord with the deployment scenario defined in section 3.3 of Recommendation ITU-R P.[CLUTTER]（ITU-R P.2108）. Therefore, it is used for the calculation of clutter loss for urban grids where the randomly selected antenna heights are at or below 6 meters and for all suburban grids since the maximum height defined in Table 3 is 4.5 meters.

For a study of this type where interference is aggregated from a large number of devices a 50th percentile clutter loss curve was developed from the P.[Clutter] model. In such a scenario the clutter loss will be greater than that shown for 50% of the grids and at or lower than that shown for 50% of the grids thus any differences are averaged out when aggregated.

Figure 3

P.[Clutter] for suburban and urban grids with antenna heights ≤ 6 meters

Embedded in Document 5A/114 Annex 24, is a spreadsheet implementation of ITU-R P.452. Per the advice from WP 3M and 3K on restrictions for use of P.[Clutter], this study employs P.452 for all rural grids and for urban grids where the antenna height randomly selected is greater than 6 meters. The clutter loss values are calculated for the “sparse houses”, and “urban” clutter (ground-cover) categories were applied to the rural and urban grids respectively. Theta max (°) provides the angle from the RLAN transmitter to the top of the clutter height. Therefore, if the spacecraft is at an elevation angle at or below theta max (°), clutter loss should be added. If the spacecraft is above theta max (°) of the respective clutter category, there is no clutter loss.

For rural grids only the lower 1.5 meter antenna height results in a possible clutter loss of 17.3 dB and only if the elevation angle is < 1.4 deg. For urban grids Figure 4 provides clutter losses on the left vertical axis and maximum elevation angle theta max (°) on the right vertical axis versus the antenna heights displayed on the horizontal axis. When the randomly selected antenna height exceeds 6 meters for urban grids the clutter loss value corresponding to the antenna height is added to the path loss only if the elevation angle is equal to or less than theta max (°).

Figure 4

P.452 clutter loss for urban grids with antenna heights ≥ 6 meters

Building Loss

The spreadsheet implementation of P.[Bel]（ITU-R P.2109） provided by ITU-R Study Group 3 was used to generate a 50th percentile building entrance loss curve for traditional buildings; thermally-efficient buildings were not considered (see Figure 5). As previously noted in the clutter discussion above the actual loss will be greater than that shown for 50% of the grids and at or lower than that shown for 50% of the grids – thus any differences are averaged out when aggregated.

[Editor’s Note: Other distributions of the building loss in Recommendation ITU-R P.2109 may be considered in the further study.]

Figure 5

P.[BEL] for indoor locations

Polarization Discrimination

Polarization mismatch discrimination is the ratio at the receiving point between received power in the expected polarization and received power from a wave transmitted with a different polarization. Since RLAN devices are linearly polarized and the single system’s satellite feeder links utilize circular polarization, some level of discrimination will exist. In the case of an interfering wave in linear polarization (the linear polarization vector can be derived from two circular polarization vectors, right- and left-hand rotation), the discrimination obtained at the victim receive antenna operating in circular polarization is provided in ITU-R S.736-3 as:

Lx = –10 log [1/2 (1 + 10-Dp()/10) dB]	(3)

where:

Dp():	polarization decoupling of the receive antenna (dB).

This would result in 3 dB discrimination in the case of perfect decoupling. Since the operational environment will result in less than perfect decoupling, most studies will assume less than 3 dB. For this study we assumes 1.4 dB per ITU RR Appendix 8 (2.2.3).

###### 5.1.1.3.2.5	Number of RLAN devices simultaneously transmitting in the 5 150-5 250 MHz band

In order to calculate the contribution of RLANs in a grid to the aggregate interference into a 1.23 MHz CDMA carrier channel contained within the sole MSS system’s FSS feeder link the density of RLAN transmissions in the 5 150-5 250 MHz band must first be determined. To calculate this number each grid is first categorized as belonging to one of three demographics, urban, suburban or rural. Then the demographic appropriate factors, shown in Table 4, are applied to the grid population count. As a starting point the factors summarized in the tables in R12-JTG4567-C-0715-Annex 36 were reviewed and adjusted as discussed below. Three additional factors are also included: a time zone factor, a 5 GHz factor, and an overlap factor. These demographic factors are defined as follows with additional discussion following as needed:

–	Busy Hour factor is the percent of the population connected to an AP during the busy hour, but not necessarily transmitting or receiving;

–	Time zone adjustment to account for satellite footprint covering multiple time zones. There are two different adjustments one for daytime and one for nighttime;

–	Market Factor is the percent of population with RLAN devices, i.e., users; this has been updated to reflect latest information concerning Internet usage that might be expected from users within the FSS feeder antenna footprint;

–	System Factor is the ratio of access points to users where an access point defines an RLAN cell. This has been adjusted to reflect inhabitants per household within the FSS feeder antenna footprint;

–	Activity Factor is the percent of RLANs with transmissions;

–	5 GHz Factor is the ratio of RLANs that can operate in the 5 GHz range to the total number of RLANs including 2.4 GHz RLANs;

–	Overlap Factor is the ratio of RLANs operating in the 5 150-5 250 MHz band to those operating across the 5 GHz range;

–	Density refers to the density of RLAN transmissions in a channel in the 5 150-5 250 MHz band per inhabitant. It is simply the product of all other factors for each demographic. The daytime factors are used when the satellite being simulated passes over North America between the hours of 08:00 and 20:00. The nighttime factors are used when the satellite being simulated passes over the North America between the hours of 20:00 and 08:00 the remainder of the time.

Table 4

Assumed demographic factors

Time Zone Adjustment

The footprint of the FSS feeder link satellite receive antenna covers a large geographic area spanning five time zones (See Figure 6). During the time the satellite passes over a location, say Lebanon, Kansas, the geographic midpoint of the United States, other RLANS will also be within the beam footprint but located in other time zones and thus will be operating outside their busy hour by as much as 3 hours. Because of this only a fraction of the area inside the footprint will ever be within its local busy hour at a given time. Therefore, peak traffic is determined by taking the highest average traffic across five consecutive time zones. Also, because time variant interference thresholds are defined in terms of outage per month, we need to consider traffic during the off peak hours defining peak traffic, that is, during the evening and early morning hours. The results of a study of traffic patterns for 50 countries is used as the basis of developing time zone adjustment values.

The results of the study are provided in Figures 7 and 8. The curves show the aggregate of pattern from the 50 countries, each normalized to their specific time zones. Figure 7 provides traffic as a percent of the busy hour during the 08:00 to 20:00 time period and figure 8 during the 20:00 to 08:00 time period.  Taking the highest average across five consecutive time zones busiest results in 91.8% of the busy hour during the daytime hours and 44.4% during the evening/early morning hours. These percentages represent the time zone adjustments and are accounted for in Table 4.

Figure 6

Time zones within the FSS feeder link footprint

Figure 7

Daytime RLAN aggregate traffic pattern for 50 countries

Figure 8

Nighttime RLAN aggregate traffic pattern for 50 countries

Market Factors

Reviewing data, concerning the number of Internet users and populations of Canada, the U.S. and Mexico, it may be shown that the percentage of the population accessing RLANs is 88.5%, 88.4% and 54.1% respectively. We estimate the distribution of population within the FSS feeder antenna footprint centered over the U.S. to be 4% from Canada, 69% from the U.S and 27% from Mexico. From this we compute the weighted average market factor to be 79%.

System Factors

Data, ,  concerning inhabitants per household show numbers for Canada, U.S. and Mexico on the order of 2.4, 2.5, and 3.8 respectively. From this, for households with an operating RLAN we can generally assume that the ratio of access points to users for residences is the inverse or 0.417, 0.4, and 0.263. Applying the population distributions from above we may compute the weighted average ratio for residences to be 0.363. When originally determining system factors for Urban and Suburban areas it was assumed that 50% of the RLANs would be either be corporate or public access based, while the other 50% would be residential based. A review of best practices concerning enterprise and public access design show the average number of clients supported by each AP will range from 24 to 100. As a worst case we assume 24 resulting in a ratio of access points to users for corporate and public access RLANs of 0.042. Applying 50% to 0.042 for corporate and public access RLANs and 50% to 0.363 for residential RLANs then summing the two we get a system factor of 20.2% for both Urban and Suburban demographics. For the rural demographic 100% of the RLANs can be assumed to be residential and hence the system factor is 36.3%.

5 GHz Factor

Since the RLAN density, based on other factors, includes all RLANs that operate in both the 2.4 GHz and 5 GHz bands a median factor of 74% from R15-WP5A-C-0420, table in section 7 is used to adjust the density to just include only those RLANs operating in the 5 GHz band.

Overlap and Bandwidth factors

In order to determine the amount of RLAN interference that will be received by the CDMA carrier channel both overlap and various RLAN channel power spectral densities (PSDs) need to be accounted for. Detailed calculations to account for the overlap and PSDs are provided in the spreadsheet embedded as an attachment to Document 5A/469 Annex 27, M.[RLAN REQ-PAR]. Results applicable to this study are provided utilizing this spreadsheet and are provided in Table 5. It should be noted that for the spreadsheet the “Total number of RLANs in the 5 GHz range” is arbitrary and changing the number does not affect the results.

Overlap Factor

The 5 GHz factor reduces the total number of RLANs operating to just those operating in the 5 GHz band. This number needs to be further reduced to just those operating in the 5 150 to 5 250 MHz band that is shared with the FSS feeder links. Referring to Figure 9 (taken from Figure 1 of Document 5A/469 Annex 27, M.[RLAN REQ-PAR] the 5 GHz channels over which the RLAN traffic load is spread are shown. It should be noted that it should exclude all channels in the 5 350 to 5 470 MHz range. Therefore, the spread sheet shown in Table 5 has been adjusted to only include those channels available or potentially available for carrying RLAN traffic and results in an overlap factor of 14.3% which is included in Table 4 to compute the RLAN transmission density numbers.

Figure 9

RLAN channel overlap in the 5 150-5 250 band

Bandwidth or channelization factor (f)

Because RLANs can operate with different bandwidths and all are larger than the CDMA carrier bandwidth of 1.23 MHz varying adjustments need to be made to account for differences in PSD presented to the victim CDMA carrier channel from RLANs transmitting with different bandwidths. The 16.5 MHz channel bandwidth originally used to calculate the the MSS bandwidth factor represents thirteen FDM 1.23 MHz CDMA carrier channels however the 1.23 MHz channel bandwidth is more appropriate for calculating any interference contribution to the FSS feeder link. Therefore, the spread sheet shown in Table 5 has been changed from 16.5 MHz to 1.23 MHz and results in a bandwidth factor of 18.04 dB which is treated as a loss when calculating the interference contribution coming from a grid.

Table 5

Bandwidth factor calculation

#### 5.1.1.4	Analysis

The analysis is based on a simulation. This simulation involves several large and dynamic data sets, as well as significant computation. Python was used to bring this data together in the simulation and produce results. The simulation tracks the path of 17 of the 34 active satellites in the single MSS system’s constellation as they orbit the Earth. Corresponding to a given satellite position, FSS feeder antenna pattern and population data enables us to calculate how each satellite “sees” the RLANs at any given point in time. With information concerning RLAN deployment and operating characteristics, along with population and demographic distributions, reasonably granular calculations may be made and results aggregated to give an accurate estimation of the interference present at the satellite at any given time.

Using the results of the simulation we calculate I/N ratios present at the satellite.

##### 5.1.1.4.1	Satellite Protection Definition

Recommendation ITU-R S.1432 Annex 1, Section 3, first paragraph states: “There are currently no Recommendations dealing with interference from co-primary allocated mobile systems into FSS systems.” Nevertheless, WP 4A gave some guidance for FSS systems in their liaison statement, Document 5A/462, from which it appears the protection level of -12.2 dB I/N was extracted. However, it is unclear how this applies in the case of interference protection for a NGSO satellite that will experience time variant interference from an aggregation of RLANs when passing overhead.  Never the less this study, will assume a protection benchmark of -12.2 dB as a point of comparison to simulation results.

##### 5.1.1.4.2	Simulation Details

The single MSS satellite system using 5 150-5 250 MHz for feeder links consists of 34 active satellites in 1 414 km low earth orbits. Satellites complete an orbit approximately every 114 minutes. Therefore it takes approximately 15 minutes for a satellite footprint to pass over a point on the Earth. The simulation considered 17 of the 34 satellites. When any point of the U.S., Canada, or Latin America (as far south as Northern part of Brazil) was within line of sight of a satellite under study the aggregate interference was calculated every 30 seconds, with each calculation constituting a sample. The simulation was run for a two day period approximately 25 satellite orbits.

##### 5.1.1.4.3	Calculations

The steps below describe the calculations performed to determine interference levels received, interference protection compliance, and related performance degradation experienced by the satellite MSS system.

Step 1:

At any instant in time the FSS receive antenna geographical footprint defines a number of grids containing some number of RLANs transmitting energy in the direction of the MSS satellite. The positions of the satellite and each grid center are used to determine the slant distance (d) and elevation angle () to be associated with each grid. Using the population count and demographic type, obtained from the LandScan™ database, the number of RLANs simultaneously transmitting in the 5 150-5 250 MHz band is computed as shown in equation 4.

RG = Sp × CD	(4)

where:

RG :	Number of RLANs transmitting in the  (5 150-5 250 MHz band within a given grid;

Sp :	Population within a grid;

CD :	Density corresponding to grid demographic from Table 4.

Step 2:

This step calculates the aggregate power (dBm) from each grid presented to the satellite transponder based on average RLAN e.i.r.p. and all losses characteristic to the link defined by the grid’s slant distance (d) and elevation angle ().

PG = Er – DR + 10log (RG) – LG + Gf – f 	 (5)

where:

PG:	Aggregate power at the input of the FSS feeder transponder from a given grid;

Er: 	Average e.i.r.p of RLANs;

DR:	Average antenna discrimination of RLANs in a grid in the direction of the MSS satellite;

LG:	Total path loss including transmission loss and other propagation losses for grid to satellite slant distance;

Gf:	Receive gain of the satellite feeder antenna in the direction of the grid center point (includes feeder loss);

f: 	Bandwidth factor, average ratio of (e.i.r.p. at the receiver (assuming no losses) to the power that would be present in the CDMA carrier receive channel) in dB.

Step 3:

Every 30 seconds this step aggregates the power from each grid located within the FSS feeder link antenna line of sight and converts the result to dBW:

Iin=10 x logk=1NG10PG/10 - 30	(6)

where:

Iin:	Simulated external interference power into CDMA carrier feeder channel at satellite transponder including building loss for all grid calculations;

NG:	Number of grids within FSS feeder antenna footprint.

Step 4:

The simulation value Iin above includes the building loss for all grids. This step takes into account that only 94.7% of RLANs are indoor and 5.3% are outdoor. For the satellite with the highest interference sample Iin, calculated in step 3, a simulation without BEL is performed to find the interference from outdoor RLANs (Iout).  Then the worst case combination of Iin and Iout is used to determine the effective aggregate interference into the FSS feeder link from both indoor and outdoor RLANs. This may calculated as:

Iagg=10 x log⁡0.947 x 10 Iin10+ 0.053 x 10 (Iout10	(7)

where:

Iout:	Simulated external interference power into CDMA carrier feeder channel at satellite transponder excluding building loss for all grid calculations

Iagg:	Effective aggregate RLAN interference power into a CDMA carrier channel of the FSS FDM feeder channel at satellite, includes impact of both indoor and outdoor RLANs

Step 5:

This step calculates aggregate RLAN interference relative to noise levels at the satellite

I/N = Iagg – Nup	(8)

where:

Nup:	Satellite kTB.

#### 5.1.1.5	Results/Conclusions

Appendix A provides further descriptive information concerning the simulation.

Table 6 provides the calculation of worst case aggregate interference Iagg simulated assuming 5.3% are operating outdoors and 94.7% are operating indoors.

Table 6

Calculation of total aggregate interference Iagg

The maximum value of the aggregate RLAN interference calculated, based on samples collected from both indoor and outdoor RLAN interference simulations was -154.1 dBW and therefore an I/N value of -13.8 dB was never exceeded, safely below the comparison benchmark of -12.2 dB. Considering this outcome, it is evident that allowing RLANs to operate outdoors and at higher powers in the 5 150-5 250 MHz poses no harmful interference to the single operational MSS system, when sharing the band with the system’s FSS feeder uplink.

APPENDIX 1

Simulation Details

The simulation tracked 17 of the 34 active MSS satellites over two days. Figure 10 shows the typical path of a satellite with a 52 degree inclination over one 24 hour period.

Figure 10

24 hour path of satellite (52o inclination)

Figure 11 shows the geo-population data compiled from the Oak Ridge National Laboratory’s (ORNL) LandScan™ database for the study area of interest. Areas in black indicate the presence of population. Whenever the satellite’s feeder receive antenna has line of site to these areas the aggregate power from the corresponding in band RLAN transmissions is calculated. Otherwise a value of -300 dBW is assigned to a sample.

Figure 11

Latitude, longitude considered in the simulation

Table 7 provides statistics for the simulation study area. Of particular interest is that while rural locations comprise 98% of the area they only account for 40% of the population. It also may be observed that suburban areas will constitute the greatest interference contribution.

Table 7

Geo-population statistics of the study area

Figure 12 shows the total aggregate interference power calculated for each pass of satellite M094 over the study area approximately once every 114 minutes. Note time is Greenwich Mean Time (GMT) and 00:00 GMT is 18:00 U.S. mountain time so we see the variation between day and night.

Figure 12

M094 satellite simulated aggregate interference

Figure 13 shows the 0, -3 and -5 dBi feeder receive antenna footprint coverage at one of the sample points in the daytime path of satellite M094. This is the point at which the maximum sample value of aggregate interference was calculated.

Figure 13

M094 satellite at maximum simulated aggregate interference

APPENDIX 2

Nomenclature

:	Expected value of channel activity

B:	Channel bandwidth

Badj:	Band adjustment to from 5 GHz band to FSS feeder link band

Bc:	CDMA carrier channel bandwidth

f:	Bandwidth factor (average ratio of e.i.r.p. at the receiver (assuming no losses) to the power that would be present in the CDMA carrier receive channel) in dB

CD:	Density corresponding to grid demographic from table 4

DR:	Average antenna discrimination of RLANs in a grid in the direction of the MSS satellite

Ed:	Peak e.i.r.p per user from satellite

Er:	Average e.i.r.p. of RLANs

Eu:	Nominal e.i.r.p. of gateway earth station

f:	Neighboring cell interference factor

Gf:	Antenna gain of the satellite feeder antenna in the direction of the grid center point

Gr:	Receive antenna gain user terminal (service link)

Gt:	Satellite transmit antenna gain (service link)

Iagg:	Effective aggregate RLAN interference power into a CDMA carrier channel of the FSS FDM feeder channel at satellite includes impact of both indoor and outdoor RLANs

Iin:	Simulated external interference power into CDMA carrier feeder channel at satellite transponder including building loss for all grid calculations

Iout:	Simulated external interference power into CDMA carrier feeder channel at satellite transponder excluding building loss for all grid calculations

Ie:	Intra-system interference spectral density in service downlink at user terminal

Io:	Intra-system interference spectral density

Is:	Intra-system interference spectral density in feeder up link at satellite

Iup:	Intra-system interference feeder uplink

k:	Boltzmann's constant 1.38 x 10-23W/s/K

Lb:	Transmission loss for slant distance computed

LBEL:	Building entrance loss

LCES:	Earth to space clutter loss

LG:	Total grid path loss including transmission loss and other propagation losses

Lf:	Antenna feed loss

Lfdr:	Path loss computed for feeder uplink

Lscv:	Path loss computed for service downlink

Lx:	Cross-polarization discrimination

m:	Number of times in which the averaged aggregate interference level calculated exceeds the threshold interference level

MS:	System margin

Nb:	Number of simultaneous CDMA user channels a satellite supports in a beam (cell)

Nc:	Number of simultaneous user channels supported in each CDMA carrier channel

Ndn:	KTB noise user terminal

Ne:	User terminal noise spectral density

NG:	Number of grids within FSS feeder antenna footprint

No:	kT noise spectral density

Ns:	Satellite noise spectral density

Nsat:	Number of simultaneous CDMA user channels a satellite supports

Nup:	Satellite kTB

p:	Period of time for which the simulation has effectively run

Pc:	Average RF power available per beam (cell)

Pd:	S-band RF power available per user

PG:	Aggregate power at the input of the FSS feeder transponder from a given grid

PS:	RF –band power available per satellite

Rb:	User channel information data rate (b/s)

RG:	Number of RLANs transmitting in the 5 150-5 250 MHz band within a given grid

Sp:	Population within a grid

t:	Total period (t) of time in which the interference threshold is exceeded

T:	Number of CDMA carriers in beam (cell)

Te:	Noise temperature user terminal

Ts:	Noise temperature satellite

Z:	Number of cells in a satellite's footprint

### 5.1.2	Study 2 (Globalstar 5A/550, Document 5A/554 will be embedded as a separate file into RLAN sharing.)

[Editor’s note: The information presented below from 5A/550 was not reviewed and agreed to by the meeting in November 2017 and appears in square brackets.]

[In 2014, one Administration allowed potentially unlimited outdoor deployment of unlicensed RLAN access points on frequencies included in IEEE 802.11ac channels in the 5 170-5 250 MHz range which overlaps the FSS 5 096-5 250 MHz feeder uplink of the LEO-D Mobile Satellite Service (MSS) system. Since that time, despite the restrictions imposed by this Administration, for the purpose of limiting the emissions from access points, LEO-D has observed a rising noise floor in the feeder uplink as measured by satellites over the territory of that Administration. LEO-D attributes this noise rise to the aggregated interference of outdoor RLAN access points deployed in the frequency band 5 170-5 250 MHz.

A static model of the aggregate interference from outdoor RLANs distributed uniformly within the coverage area of the LEO-D satellite, elaborated in Document 5A/550, shows that the noise rise of 1.8 db in 5 170-5 250 MHz measured by satellites over the territory of that Administration is consistent with the deployment of 1 million outdoor access points that are compliant with the Administration rules of 4 watts EIRP, and operating at a busy hour duty cycle of 10%. The cable TV industry, one group deploying RLANs in the territory of that Administration., reports that 10 million total access points are using 5 170-5 250 MHz as of August of 2017, implying that as many as 10% of access points deployed may be operating outdoors.  The static model also shows the variation in noise rise experienced in the LEO-D uplink as a function of the total number of outdoor RLANS deployed and their average duty cycle. Model results indicate that the same noise rise is also produced by 180 k outdoor access points operating at an average 40% duty cycle. This would imply that a smaller fraction of 1.8% of the total number of RLANs could also produce the same noise rise. This noise level increase is equivalent to an Interference-to-Noise ratio (I/N) of -3 db and  is 9.2 dB higher than the value given in Recommendation ITU-R S.1432.

Document 5A/550 further presents an analysis of the relationship between noise rise in the MSS feeder uplink spectrum and degradation to the LEO-D MSS traffic-handling capacity and satellite RF power amplifier capacity. Based on the cable TV industry statements, the deployment of RLANs is increasing at a rate of 37% per year, and the Cisco Virtual Networking  Index indicates that RLAN deployment is increasing as much as 43% per year in the next 5 years. If outdoor unlicensed RLAN deployments in 5 170-5 250 MHz in the territory of the Administration increase at the projected rates of 35-43% per year, LEO-D will suffer substantial degradation to its MSS traffic capacity and satellite power consumption. Since there is no method for limiting the number of RLAN deployments over geographic regions encompassing a 3901 km radius, the area within which outdoor RLANs in 5 170-5 250 MHz could cause aggregate interference to the MSS feeder uplink,  modification of the current Radio Regulations and Recommendations prohibiting outdoor RLAN operation at 5 150-5 250 MHz could be ill advised..

The model in this report (5A/550) assumes (1) access point characteristics and transmitted power levels that are consistent with the regulations of that Administration, (2) antenna gain characteristics representative of actual equipment in the field, and (3) LEO-D operating characteristics as described in filings with the Administration.  This analysis also presents the effects of urban “clutter,” building shadowing, and different access point duty cycles on aggregate interference to the MSS feeder uplinks and the resulting detrimental impact on LEO-D MSS operations.

This figure shows the predicted rise in the noise floor of the LEO-D system as a result of the outdoor deployment of RLAN access points in the frequency range 5 170-5 250 MHz.]

### 5.1.3 	Study 3 (Globalstar 5A/553)

#### 5.1.3.1	Interference Analysis

The interference situation set out in the previous section was analyzed using a computer simulation that was conducted using the Visualyse simulation tool that is available from Transfinite Systems (www.transfinite.com).

##### 5.1.3.1.1	Simulation Description

The use of the 5 GHz band by WAS/RLAN beyond what is contained in the current Radio Regulations was treated as part of WRC-2015 agenda item 1.1, and studies were conducted by Joint Task Group (JTG) 4-5-6-7 during the period between WRC-12 and WRC-15. The outcome of these studies is summarized in Reports from the JTG.

The technical requirements of WAS/RLANs in the 5 GHz frequency range have been captured in a subsequent document from ITU-R Working Party 5A, “Working document towards a preliminary draft new Report ITU-R M.[RLAN REQ-PAR] – Technical characteristics and operational requirements of WAS/RLAN in the 5 GHz frequency range; (Annex 27 to Document 5A/469). The RLAN parameters used in this study are derived from that document, including RLAN bandwidth and e.i.r.p. distributions and deployment densities.

The assumption was made that interference from WAS/RLAN would be primarily from RLANs rather than from WAS. The WAS/RLAN parameters provided below are based upon this assumption. As there are potentially millions of RLANs in the 5 150-5 250 MHz band, it is impossible to simulate each RLAN as an individual interferer. Hence, the power from the individual RLANs has been aggregated and this aggregation has been used as the output power from a single terrestrial “pseudo-station” (see Section 5.1.3.4.1.1). This technique is consistent with the “reference System” approach that is used to simulate interference involving terrestrial cellular telephone systems.

[Editor’s Note: Further investigation of the effect of pseudo-stations is required.]

[The computer simulation focused on the European region since it has a large population of RLANs and the area easily falls within the footprint of the MSS spacecraft antenna. For the purpose of the simulation it was assumed that Europe consists only of the most populous 45 European countries. A separate terrestrial pseudo-station was established to represent the total number of RLANs deployed in both urban and rural areas. There were, thus, a total of 85 different terrestrial stations, 45 urban and 40 rural, established for the simulation. The coordinates for the pseudo-stations were those of the capital of the country for urban stations, and a rural location for the rural pseudo-stations. A list of these cities, their coordinates, and the respective populations of the countries are shown in Appendix 1.]

###### 5.1.3.1.1.1	Clutter Loss

In this version of the simulation, the effects of clutter and building entry loss were taken into account by combining these losses with the antenna pattern. The clutter loss was based on the method given in Recommendation ITU-R P.2108, noting the recent advice in Document 5A/499 from Working Parties 3K and 3M.

One form of the interference calculation equation in dB is:

I=Ptx+Gtx-Lpath-Lclutter+Grx

In absolute / linear representation, this is (using lower case to represent linear / absolute values):

i=ptxgtxgrxlpathlclutter

Interference aggregation should be done using power summation (i.e., in the linear or absolute domain), and hence the equation for aggregate interference from N transmitters is:

iagg=k=1Nik=k=1Nptx,kgtx,kgrx,klpath,klclutter,k

If the power, gains and path loss are the same for each of the N transmitters then this is simplified to:

iagg=ptxgtxgrxlpathk=1N1lclutter,k

The average of the inverse of the clutter loss is:

1lclutter,avg=1Nk=1N1lclutter,k

Thus:

lclutter,avg=Nk=1N1lclutter,k

This is the average over the sample. As the number of transmitters increases this will tend towards the average over the population. Hence for large numbers it is acceptable to use the average over the population as representative of the average of the sample.

The aggregate interference from N transmitters is then:

iagg=ptxgtxgrxlpath.Nlclutter,avg

In dB this is:

Iagg=Ptx+Gtx-Lpath-Lclutter,avg+Grx+10log10N

A plot of the average clutter loss versus elevation angle from the horizontal is shown in Figure 1.

###### 5.1.3.1.1.2	Building Entry Loss

The building entry loss (BEL) was computed based on the guidance provided in Recommendation ITU-R P.2109. The median building entry loss was used for all indoor RLANs. A plot of the building entry loss (BEL) is shown in Figure 2.

###### 5.1.3.1.1.3	RLAN Transmitter Power

The power output for each pseudo-station that simulates either the urban or rural population of WAS/RLAN devices is determined using the average RLAN power output and the RLAN density per inhabitant (given in the next section) multiplied by the number of inhabitants of each country that live in either the urban or rural portion of that country. Population data was obtained from data on the Internet (see www.worldometers.info/population/countries-in-europe-by-population.

###### 5.1.3.1.2	WAS/RLAN Parameters

The parameters of WAS/RLANs used in the computer simulation were based on Annex 27 to Document 5A/469. As mentioned in the previous section, the parameters chosen for the RLAN transmitters are consistent with those given in Section 3 of Annex 27 of 5A/469.

[Editor’s Note: Further discussion on the text below is needed at next meeting.]

Under the current Radio Regulations, restricting RLAN deployment to indoors, the e.i.r.p. distribution given in Annex 27 yields an average e.i.r.p. of -11 dBW. One Administration has suggested that RLAN transmitters be authorized to use an e.i.r.p. of +6 dBW. Substituting this high power RLAN transmitter for the currently authorized 200 mW (-7 dBW) transmitter changes the average e.i.r.p. Simulations involving outdoor RLAN transmitters assumed that the 200 mW transmitters shown in TABLE 2A of 5A/469 (Annex 27) were replaced by higher power, outdoor deployed transmitters. Since the outdoor deployment percentages were lower than 19%, a smaller percentage of 200 mW transmitters was retained in the distribution of transmitters and the computation of the average e.i.r.p.. This smaller percentage was equal to the difference between 19% and the percentage of outdoor deployed RLAN transmitters.

Two different RLAN antenna patterns were used in the simulation. For indoor RLAN transmitters, an omni-directional pattern with a gain of -2 dBi in all directions was used. This type of antenna is referenced in section 3.5 of Document 5A/469 (Annex 27). For the outdoor RLAN transmitters with 6 dBW e.i.r.p., the antenna pattern shown in Figure 3 below was used. This pattern is derived from commercially available omni-directional antennas with 6 dBi gain.

For the simulation presented here, the pertinent factors are summarized:

| Case under study | Receiver Bandwidth (MHz) | Overlapping Factor | Resulting density (RLAN/inhab.) | Average Bandwidth Factor |
| --- | --- | --- | --- | --- |
| MSS Feeder Links | 19.38 | 14.0% | 0.0038 | 6.06 dB |

Detailed calculations of the overlapping factors and average bandwidth factors are given in the following file.

The parameters given assume that RLANs are distributed over the entire 5 150–5 350 MHz and 5 470-5 925 MHz ranges and would have to be recalculated should this overall range change.

##### 5.1.3.1.3	FSS Feeder link parameters

As mentioned above, the parameters of the feeder uplinks of the LEO-D MSS system were used in the computer simulation. These are summarized in the Table below.

TABLE 2

MSS Feeder Link Parameters

| Parameter | HIBLEO-4 FL |
| --- | --- |
| Satellite orbit altitude h (km) | 1 414 |
| Satellite Inclination (degrees) | 52 |
| Frequency Range (MHz) | 5 150-5 250 |
| Satellite receiver bandwidth B (MHz) | 16.5 |
| I/N (dB) | -12.2 |
| Satellite receiver noise temperature T (K) | 550 |
| Pnoise, add (dBW) | -140.3 |
| Iadd (dBW) | -152.5 |
| Polarization discrimination Lp (dB) | 1 |

The spacecraft receive antenna is an “iso-flux” antenna and the gain pattern is shown below.

Figure 3

Spacecraft Receive Antenna Pattern

#### 5.1.3.2	Computer simulation description

The simulation determined the level of interference that would be experienced by a feeder link carrier that was uplinked from the earth station in Aussaguel, France, near Toulouse. This uplink carrier was switched from one spacecraft to another based on whichever spacecraft was the closest to the earth station. The interference was recorded as an interference-to-noise ratio.

The simulation time step in this version of the simulation was 10 Seconds. A 10 Second interval implied that the area under the footprint changed by 0.139%. The simulation was run for two simulation days or 17 280 time steps. This simulated period approximates the ground track repeat for two successive orbital planes of the MSS system.

The antenna parameters included in the computer simulation include the antenna gain pattern and the clutter and building entry losses, where applicable.

#### 5.1.3.3	Computer simulation results

The results of this simulation are shown in Figure 4.  All of the results pertain to RLANs operating in Europe. RLANs from other nearby regions are not used in this simulation.

The graph shows the probability of interference for 4 different situations as a function of interference-to-noise ratio (I/N).

The leftmost plot in Figure 4 shows the potential interference that would be due to a strict adherence to Resolution 229 and does not include any outdoor RLAN transmitters.

The next plot to the right shows the potential interference expected if the current Resolution 229 is observed with an “accidental” 5% portion of the European RLANs being outdoors.

The next plot to the right shows the potential interference results for RLANs over Europe if 5% of the RLAN transmitters are operated outdoors with omni-directional antennas with 6 dBi gain, pointed at the horizon, and an e.i.r.p. of 6 dBW. RLAN transmitters complying with these characteristics have been proposed by one Administration.

The right most plot shows the potential interference results for RLANs over Europe if 15% of the RLAN transmitters are operated outdoors with omni-directional antennas with 6 dBi gain, pointed at the horizon, and an e.i.r.p. of 6 dBW. This percentage of outdoor operation has been proposed by another Administration.

These simulations assume clutter loss for RLANs operating in urban areas and indoor RLANs include building entry loss (BEL). These losses were explained in Section 5.1.4.1, above. It is assumed that there is no clutter loss in rural areas.

It is worthwhile to note that interference from European RLANs operating at 6 dBW e.i.r.p. will exceed an I/N = -12.2 dB threshold at least 90% of the time.

#### 5.1.3.4	Summary and Assessment of Sharing Feasibility

Computer simulation of the likely effect of outdoor RLAN transmitters has shown that significant interference would be caused to the feeder uplinks of MSS systems. Figure 5 shows the “footprint” of the feeder uplink antenna of the LEO-D system. It is apparent that the spacecraft will “see” all of the RLAN traffic in the European region.

The level of interference to feeder uplinks from co-band 5 GHz RLANs will depend on the total number of RLAN devices that are deployed, their e.i.r.p., and whether they are permitted to operate outdoors.

Figure 5

LEO-D MSS System European Feeder Uplink Coverage

The simulations presented here indicate that there will be unacceptable interference to the feeder uplinks of MSS systems using the 5 150-5 250 MHz band. The projected interference levels from outdoor deployment exceed the industry guideline of -12.2 dB I/N by at least 2 dB, with 50% probability.  Mitigation techniques may be devised and implemented, but it is likely that the proliferation of these devices will, at some time, exceed the capabilities of these techniques and result in unacceptable interference to MSS feeder uplinks. At that time, the only recourse left will be to stop the outdoor deployment of access points.

If no records of the deployment of outdoor access points are maintained by Administrations, there will be no way to determine where the interference to the feeder uplinks is emanating from and no way to limit such interfering operations.

The use of outdoor access points would require Administrations to maintain records of deployment so that when interference occurs, remedial action can be taken by the correct Administration. Without a database of deployments, quantitative limits for outdoor deployed access points will be required at the outset.

Given the results of this study, it appears that the best way of ensuring reasonable ongoing protection of the worldwide non-GSO MSS feeder uplinks is to make no change in the provisions of Resolution 229 (Rev.WRC-12) regarding the 5 150-5 250 MHz band.

APPENDIX 1

Locations of “Pseudo-Stations”

|  | 5 GHz | Stations | for | WP 5A | Simulation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Country | City | Latitude | Longitude | Population |  |
|  | Power |  |  |  |  |  |
|  | =========== | =========== | ======== | ======== | ========= | ======== |
|  | Albania |  |  |  |  |  |
| 1 | U | Tirana | 41.317 | 19.817 | 1872048 |  |
| 2 | R | Lushnje | 40.851 | 19.782 | 1039380 |  |
|  | Andorra |  |  |  |  |  |
| 3 |  | Andorra la V | 42.5 | 1.52 | 68728 |  |
|  | Austria |  |  |  |  |  |
| 4 | U | Vienna | 48.2 | 16.37 | 5670984 |  |
| 5 | R | Wels | 48.05 | 14 | 2921416 |  |
|  | Belarus |  |  |  |  |  |
| 6 | U | Minsk | 53.9 | 27.57 | 7093901 |  |
| 7 | R | Slonim | 53.08 | 25.32 | 2364634 |  |
|  | Belgium |  |  |  |  |  |
| 8 | U | Brussels | 50.85 | 4.35 | 10986077 |  |
| 9 | R | Leopoldsburg | 51.11 | 5.25 | 457753 |  |
|  | Bos&Herz |  |  |  |  |  |
| 10 | U | Sarajevo | 43.866 | 18.417 | 1524689 |  |
| 11 | R | Sanski Most | 44.717 | 16.667 | 2268070 |  |
|  | Bulgaria |  |  |  |  |  |
| 12 | U | Sofia | 41.683 | 23.317 | 5241673 |  |
| 13 | R | Yambol | 49.483 | 26.467 | 1803586 |  |
|  | Crotia |  |  |  |  |  |
| 14 | U | Zagreb | 45.8 | 16 | 2513260 |  |
| 15 | R | Sisak | 45.48 | 16.4 | 1696555 |  |
|  | Cyprus |  |  |  |  |  |
| 16 |  | Nicosia | 35.16 | 33.367 | 794646 |  |
| 17 |  | Galateia | 35.4222 | 34.0722 | 394944 |  |
|  | Czech Rep |  |  |  |  |  |
| 18 | U | Prague | 50.083 | 14.467 | 7895237 |  |
| 19 | R | Brno | 49.1 | 16.55 | 2659893 |  |
|  | Denmark |  |  |  |  |  |
| 20 | U | Copenhagen | 55.717 | 12.567 | 4992146 |  |
| 21 | R | Hovborg | 55.6069 | 8.9411 | 719691 |  |
|  | Estonia |  |  |  |  |  |
| 22 | U | Talinn | 59.417 | 24.75 | 861798 |  |
| 23 | R | Rapla | 58.994 | 24.801 | 443957 |  |
|  | Finland |  |  |  |  |  |
| 24 | U | Helsinki | 60.166 | 24.933 | 4621423 |  |
| 25 | R | Oulu | 60.067 | 25.467 | 919851 |  |
|  | France |  |  |  |  |  |
| 26 | U | Paris | 48.857 | 2.351 | 52080850 |  |
| 27 | R | Guingamp | 48.583 | 3.15 | 12857866 |  |
|  | Germany |  |  |  |  |  |
| 28 | U | Berlin | 52.517 | 13.383 | 62251088 |  |
| 29 | R | Bremervoerde | 53.483 | 9.133 | 18385036 |  |
|  | Greece |  |  |  |  |  |
| 30 | U | Athens | 37.967 | 23.717 | 8714345 |  |
| 31 | R | Sofades | 39.333 | 22.1 | 2178586 |  |
|  | Hungary |  |  |  |  |  |
| 32 | U | Budapest | 47.433 | 19.25 | 7086443 |  |
| 33 | R | Svarvas | 46.864 | 20.557 | 2701462 |  |
|  | Iceland |  |  |  |  |  |
| 34 | U | Reykjavik | 64.133 | 21.933 | 320597 |  |
| 35 | R | Hella | 63.833 | 20.4 | 13706 |  |
|  | Ireland |  |  |  |  |  |
| 36 | U | Dublin | 53.344 | 353.732 | 3034709 |  |
| 37 | R | Mullingar | 53.523 | 352.662 | 1714444 |  |
|  | Italy |  |  |  |  |  |
| 38 | U | Milan | 45.467 | 9.183 | 42277170 |  |
| 39 | R | Volterra | 43.4 | 10.867 | 17520808 |  |
|  | Lativa |  |  |  |  |  |
| 40 | U | Riga | 56.95 | 24.1 | 1361196 |  |
| 41 | R | Pope | 57.4 | 21.852 | 583370 |  |
|  | Liechtenstein |  |  |  |  |  |
| 42 |  | Vaduz | 47.167 | 9.51 | 38022 |  |
|  | Lithuania |  |  |  |  |  |
| 43 | U | Vilnius | 54.683 | 25.317 | 1987069 |  |
| 44 | R | Kretinga | 55.89 | 21.242 | 843513 |  |
|  | Luxembourg |  |  |  |  |  |
| 45 | U | Luxembourg | 49.6 | 6.117 | 497072 |  |
| 46 | R | Betzdorf | 49.6875 | 6.35 | 87031 |  |
|  | Macedonia |  |  |  |  |  |
| 47 | U | Skopje | 42 | 21.433 | 1623115 |  |
| 48 | R | Dolneni | 41.425 | 21.454 | 1180193 |  |
|  | Malta |  |  |  |  |  |
| 49 |  | Valletta | 35.9 | 14.517 | 420521 |  |
|  | Moldova |  |  |  |  |  |
| 50 | U | Chisinau | 47 | 28.917 | 1540763 |  |
| 51 | R | Floresti | 47.893 | 28.301 | 2513877 |  |
|  | Monaco |  |  |  |  |  |
| 52 | U | Monte Carlo | 43.733 | 7.417 | 38010 |  |
|  | Montenegro |  |  |  |  |  |
| 53 | U | Podgorica | 42.783 | 19.467 | 398921 |  |
| 54 | R | Golubuvci | 42.334 | 19.225 | 227329 |  |
|  | Netherlands |  |  |  |  |  |
| 55 | U | Amsterdam | 52.367 | 4.883 | 15363626 |  |
| 56 | R | De Koog | 53.098 | 4.763 | 1669219 |  |
|  | Norway |  |  |  |  |  |
| 57 | U | Oslo | 59.933 | 10.683 | 4195340 |  |
| 58 | R | Froeya | 63.726 | 8.744 | 1135460 |  |
|  | Poland |  |  |  |  |  |
| 59 | U | Warsaw | 52.217 | 21.033 | 23138144 |  |
| 60 | R | Wolsztyn | 52.117 | 16.117 | 15425429 |  |
|  | Portugal |  |  |  |  |  |
| 61 | U | Lisbon | 38.767 | 350.85 | 6795296 |  |
| 62 | R | Coruche | 38.957 | 351.473 | 3469501 |  |
|  | Romania |  |  |  |  |  |
| 63 | U | Bucharest | 44.417 | 26.1 | 11773357 |  |
| 64 | R | Faurei | 45.0842 | 27.2728 | 7641155 |  |
|  | Russian Fed |  |  |  |  |  |
| 65 | U | Moscow | 55.75 | 37.617 | 80520000 |  |
| 66 | R | Kizhi | 62.0667 | 35.2381 | 29480000 |  |
|  | San Marino |  |  |  |  |  |
| 67 | U | San Marino | 45.9417 | 12.4583 | 32104 |  |
|  | Serbia |  |  |  |  |  |
| 68 | U | Belgrade | 44.8 | 20.467 | 5222279 |  |
| 69 | R | Opovo | 45.0519 | 20.4303 | 3554661 |  |
|  | Slovakia |  |  |  |  |  |
| 70 | U | Bratislava | 48.15 | 17.117 | 2922500 |  |
| 71 | R | Surovce | 48.3329 | 17.7174 | 2509657 |  |
|  | Slovenia |  |  |  |  |  |
| 72 | U | Ljubljana | 46.05 | 14.5 | 1033555 |  |
| 73 | R | Brezice | 45.904 | 15.5922 | 1037697 |  |
|  | Spain |  |  |  |  |  |
| 74 | U | Madrid | 40.433 | 356.3 | 37777519 |  |
| 75 | R | Carmona | 37.467 | 5.633 | 8292626 |  |
|  | Sweden |  |  |  |  |  |
| 76 | U | Stockholm | 59.35 | 18.067 | 8392848 |  |
| 77 | R | Torup | 56.958 | 13.081 | 1527776 |  |
|  | Switzerland |  |  |  |  |  |
| 78 | U | Bern | 46.95 | 7.45 | 6154572 |  |
| 79 | R | Gland | 46.42 | 6.27 | 2299511 |  |
|  | Turkey |  |  |  |  |  |
| 80 | U | Ankara | 39.917 | 32.833 | 58247209 |  |
| 81 | R | Yumurtalik | 36.769 | 35.797 | 22316012 |  |
|  | Ukraine |  |  |  |  |  |
| 82 | U | Kiev | 50.45 | 30.5 | 30994725 |  |
| 83 | R | Vysokopillia | 47.488 | 33.532 | 13410330 |  |
|  | U.K. |  |  |  |  |  |
| 84 | U | London | 51.5 | 0 | 53784611 |  |
| 85 | R | Mullaig | 57.004 | 354.173 | 11726486 |  |

Appendix 2

Sample Link for 5% Outdoor Deployment of 6 dBW RLAN Transmitters

| Link Calculation |  |  |  |
| --- | --- | --- | --- |
| Transmit Station |  | Aussaguel |  |
|  | Antenna | Antenna |  |
| Transmit Frequency |  | 5.192-5.208 GHz |  |
|  | Centre | 5.200 GHz |  |
|  | Bandwidth | 16.5 MHz |  |
| Transmit Power |  | -4.7 dBW |  |
|  | EIRP | 42.030 dBW |  |
| Transmit Gain |  | 46.730 dB |  |
|  | Relative Gain | 0.0 dB |  |
|  | Peak Gain | 46.73 dBi |  |
| Path Loss |  | 171.121491 dB |  |
|  | ITU-R Rec. P.525 | 171.070562 dB |  |
|  | ITU-R Rec. P.618 | 0.000331 dB |  |
|  | ITU-R Rec. P.676 (dry) | 0.046746 dB |  |
|  | ITU-R Rec. P.676 (water) | 0.003852 dB |  |
| Receive Station |  | GstarSats-40 |  |
|  | Antenna | Antenna |  |
| Receive Gain |  | 5.645 dB |  |
|  | Relative Gain | 4.134738 dB |  |
|  | Peak Gain | 1.51 dBi |  |
| N (receive noise) |  | -129.021365 dBW |  |
|  | Noise Temperature | 550.0 K |  |
|  | Wanted Bandwidth | 16.5 MHz |  |
|  | Noise/Hz | -201.196205 dBW/Hz |  |
| C (signal strength) |  | -126.346753 dBW |  |
|  | Margin | 23.653247 dBW |  |
|  | Threshold | -150.0 dBW |  |
| C/N |  | 2.674612 dB |  |
|  | Margin | -7.325388 dB |  |
|  | Threshold | 10.0 dB |  |
| Worst Interferer |  |  |  |
| Station |  | 74 Madrid |  |
|  | Antenna | Antenna1 |  |
| Interferer Bandwidth |  | 16.5 MHz |  |
| Interferer Power |  | 32.5 dBW |  |
|  | EIRP | 22.967 dBW |  |
| Interferer Gain |  | -9.533 dB |  |
|  | Relative Gain | 1.756608 dB |  |
|  | Peak Gain | -11.29 dBi |  |
| Path Loss |  | 170.686701 dB |  |
|  | ITU-R Rec. P.525 | 170.639036 dB |  |
|  | ITU-R Rec. P.618 | 0.000103 dB |  |
|  | ITU-R Rec. P.676 (dry) | 0.044865 dB |  |
|  | ITU-R Rec. P.676 (water) | 0.002696 dB |  |
| Wanted Gain |  | 5.131 dB |  |
|  | Relative Gain | 3.62144 dB |  |
|  | Peak Gain | 1.51 dBi |  |
|  | Wanted Feeder Loss | 2.9 dB |  |
| Wanted Bandwidth |  | 16.5 MHz |  |
| Signal Strength |  | -145.488653 dBW |  |
| Bandwidth Advantage |  | 0.0 dB |  |
| Polarisation Advantage |  | 1.46128 dB |  |
| Frequency Adjustments |  | 0.0 dB |  |
| Other Advantages |  | 0.0 dB |  |
| I |  | -146.949933 dBW |  |
| C/I |  | 20.60318 dB |  |
|  | Margin | -4.39682 dB |  |
|  | Threshold | 25.0 dB |  |
| C/(N+I) |  | 2.605197 dB |  |
|  | Margin | -22.394803 dB |  |
|  | Threshold | 25.0 dB |  |
| I/N |  | -17.928568 dB |  |
|  | Margin | 2.728568 dB |  |
|  | Threshold | -20.657135 dB |  |
| Aggregate Interference |  |  |  |
| Number of Interferers |  | 167 |  |
| I |  | -139.683814 dBW |  |
| C/I |  | 13.337061 dB |  |
|  | Margin | -11.662939 dB |  |
|  | Threshold | 25.0 dB |  |
| C/(N+I) |  | 2.316902 dB |  |
|  | Margin | -22.683098 dB |  |
|  | Threshold | 25.0 dB |  |
| I/N |  | -10.662449 dB |  |
|  | Margin | -4.537551 dB |  |
|  | Threshold | -6.124897 dB |  |

### 5.1.4 	Study 4 (CHN 5A/574)

[Editor’s note: The information of COMPASS-MSS system in this study needs confirmation from WP 4A.]

[Editor’s note: Further studies are needed for the percentage of outdoor RLAN.]

This document provides a preliminary analysis of interference from potential outdoor WAS/RLAN applications with the feeder uplinks of the COMPASS-MSS system in the fixed-satellite service (FSS) in the frequency band 5 150-5 250 MHz.

#### 5.1.4.1	Technical characteristics

##### 5.1.4.1.1	Technical and operational characteristics of WAS/RLAN system operating in the 5 150-5 250 MHz frequency band

The parameters of WAS/RLAN used in the analysis are based on “Working document towards a preliminary draft new Report ITU-R M.[RLAN REQ-PAR] – Technical characteristics and operational requirements of WAS/RLAN in the 5 GHz frequency range; (Annex 27 to Document 5A/469). These parameters are summarized in the Table 1 below.

TABLE 1

WAS/RLAN Parameters

| Parameter | Value |
| --- | --- |
| Average RLAN e.i.r.p. (dBm) | 19 |
| Average Antenna Discrimination in Elevation (dB) | 2 |
| Average RLAN Device Density (active devices per inhabitant) | 0.0265 |

Besides the parameters in the Table 1, WAS/RLAN overlapping factors and average bandwidth factors are given in the working document for satellite service, as showed in the Table 2 below.

TABLE 2

WAS/RLAN Overlapping Factors and Average Bandwidth Factors regarding satellite services

| Case under study | Receiver Bandwidth (MHz) | Overlapping factor | Resulting density (RLAN/inhab.) | Average Bandwidth factor |
| --- | --- | --- | --- | --- |
| FSS | 40 | 12.9 % | 0.0034 | 3.59 dB |
| EESS (SAR) | 100 | 22 % | 0.0058 | 1.94 dB |
| EESS (Altimeter) | 320 | 48.9 % | 0.0130 | 0.35 dB |
| EESS (scatterometer) | 2 | 11.0 % | 0.0029 | 15.89 dB |
| MSS Feeder links | 16.5 | 11.0 % | 0.0029 | 6.73 dB |

For the case of COMPASS-MSS system, the satellite receiver bandwidth is 8.2 MHz, which is not included in the table. Following the calculation method in the working document, the overlapping factor and average bandwidth factor of the COMPASS-MSS system can be derived, as showed in the Table 3 below.

TABLE 3

WAS/RLAN Overlapping Factors and Average Bandwidth Factors regarding COMPASS-MSS system

| Case under study | Receiver Bandwidth (MHz) | Overlapping factor | Resulting density (RLAN/inhab.) | Average Bandwidth factor |
| --- | --- | --- | --- | --- |
| COMPASS-MSS system | 8.2 | 11.0 % | 0.0029 | 9.77 dB |

##### 5.1.4.1.2	Technical and operational characteristics of FSS system operating in the 5 150-
5 250 MHz frequency band

China is planning to deploy a MSS system, i.e. next generation COMPASS-MSS system, with the service link of 1.6/2.5 GHz bands. This system will include several IGSO satellites, which plan to utilize inclined-orbit spacecrafts at an altitude of 36 000 kilometers and an inclination angle of the orbits of about 55 degrees. The feeder uplink of the IGSO satellites will operate in 5 091-5 250 MHz frequency band.

The parameters of the feeder uplink of the COMPASS-MSS system are summarized in the Table below.

TABLE 4

COMPASS-MSS Feeder Uplink Parameters

| Parameter | COMPASS-MSS system |
| --- | --- |
| Satellite orbit altitude h (km) | 36 000 |
| Satellite Inclination (degrees) | 55 |
| Frequency Range (MHz) | 5 091-5 250 |
| Satellite receiver bandwidth B (MHz) | 8.2 |
| I/N (dB) | -12.2* |
| Satellite receiver noise temperature T (K) | 500 |
| Pnoise, add (dBW) | -130.5 |
| Iadd (dBW) | -142.7 |
| Polarization discrimination Lp (dB) | 1 |
| *The protection cretiria is being investegated in WP4A. |  |

The spacecraft receive antenna is an “iso-flux” antenna and the gain pattern is shown below.

Figure 1

Spacecraft Receive Antenna Pattern

#### 5.1.4.2	Methodology

##### 5.1.4.2.1	Interference Analysis scenario

Due to the Earth-coverage beam, the satellite can receive emissions from very large numbers of WAS/RLAN transmitters.

The assumption was made that interference from WAS/RLAN would be primarily from RLANs and more specifically from access points in RLAN systems. As there are potentially millions of RLAN access points in the 5 150-5 250 MHz band, it is difficult to simulate each access point as an individual interferer. Hence, the power from the individual access points has been aggregated and this aggregation has been used as the output power from a single terrestrial station.

The area within the red dashed line of Figure 2 shows the “footprint” of the feeder uplink which is uplinked from the earth station located in Hainan, China. It can be found that most counties of South-Eastern Asia and Southern China are within the footprint of the satellite.

Figure 2

Footprint of the Feeder Uplink Antenna of the COMPASS-MSS System

For the simulation it is assumed that there is one terrestrial station (aggregated station) in each country/region of the footprint area. The countries/region and their populations are listed in the Table below (see http://www.worldometers.info/world-population/south-eastern-asia-population).

TABLE 5

Countries/Region in the simulation and their populations

| Country/Region | Population |
| --- | --- |
| Indonesia | 263,991,379 |
| Philippines | 104,918,090 |
| Viet Nam | 95,540,800 |
| Thailand | 69,037,513 |
| Myanmar | 53,370,609 |
| Malaysia | 31,624,264 |
| Cambodia | 16,005,373 |
| Laos | 6,858,160 |
| Singapore | 5,708,844 |
| Brunei Darussalam | 428,697 |
| Southern China | 500,000,000* |
| * This figure is an approximate value based on that this region is one of the most densely populated areas in China. |  |

##### 5.1.4.2.2	Calculations

The interference power for each RLAN terrestrial station (aggregated station) at the COMPASS-MSS system satellite receiver is calculated by equation (1) as below.

(1)

where:

In: 	Interference power from the n-th terrestrial station, dBm;

EIRPave : 	Average e.i.r.p. of RLAN access points, 19dBm;

Nn: 	Number of RLAN access points in the n-th country/region;

LDis: 	Average RLAN Antenna Discrimination in Elevation, 2dB;

BWF: 	Average Bandwidth Factor, 9.77dB;

Lpol: 	Polarization discrimination, 1dB;

Lprop: 	Propagation loss including free space transmission loss, atmospheric loss (Rec. ITU-R P.676) and clutter loss (Rec. ITU-R P.2108), dB;

Gr(θ): 	Antenna gain of the satellite receiver in the direction of the n-th terrestrial station, dBi;

θ:	Off-axis angle, deg.

The number of RLAN access points in the n-th country/region Nn is calculated by equation (2) as below.

Nn = Ds * Pn	(2)

where:

Ds: 	Average RLAN device density, taking into account the overlapping factor, 0.0029 RLAN/inhab;

Pn: 	Population of the n-th country/region, population data is obtained from Table 5.

The aggregate interference is calculated by equation (3) as follows.

(3)

where:

Itotal:	Received aggregate interference from all RLAN terrestrial stations (aggregated stations)  inside footprint size, dBm;

n: 	Index of the RLAN terrestrial station;

In:	Interference power from the n-th terrestrial station, dBm.

#### 5.1.4.3	Interference Analysis

##### 5.1.4.3.1	Simulation Description

The analysis of the interference situation was conducted using the Visualyse simulation tool.

The simulation time step was 300 Seconds. The simulation was run for two simulation days or 576 time steps. This simulated time approximates two orbital periods of the IGSO satellite.

The simulation scenario is shown below.

Figure 3

Simulation scenario

##### 5.1.4.3.2	Simulation results

The results of the simulation are shown in Figure 4 for one and a half simulation periods (days). It can be seen that the interference to the COMPASS-MSS system feeder link from RLAN access points will exceed the I/N = -12.2 dB threshold for over half of a simulation period. That means the COMPASS-MSS system feeder link will suffer interference from RLAN access points for more than 50% of the time.

Figure 4

Results of the preliminary simulation

-12.2 dB

#### 5.1.4.4	Summary

The level of interference to FSS from RLANs in the frequency band 5 150-5 250 MHz will depend on many factors such as the total number of RLAN devices that are deployed, their e.i.r.p. and whether they are permitted to operate outdoors.  This contribution concentrated on interference due to outdoor RLAN applications and has computed the potential interference due to those devices.

The level of the potential interference can be limited by limiting the total number of RLAN devices that are deployed, the e.i.r.p. of the access points above a given elevation angle, the number of outdoor deployed access points, etc.

### 5.1.5 	Study 5 (JPN 5A/586)

Another example of a sharing study with realistic conditions is conducted as follows.

The parameters and protection criteria of the non-GSO MSS feeder links are assumed to be the same as described in section 4.1 (proposed in 5A/395) and additional conditions are assumed to be shown in Table A1:

TABLE A1

Additional conditions for a sharing study with MSS feeder links

| Parameters | Values and conditions |
| --- | --- |
| e.i.r.p. distribution | Based on Table 1A in Report ITU-R M.[RLAN REQ-PAR] (The outdoor usage ratio is 5.3%) |
| Antenna pattern for RLANs | Based on Recommendation ITU-R M.1652 (If the RLAN uses e.i.r.p. of 1 W, the elevation angle mask defined for 5 250-5 350 MHz band in the Resolution 229 (Rev.WRC-12) is applied.) |
| Height of RLAN antenna | 4.5 m |
| Active ratio of RLANs | 4.645% (derived from Report ITU-R M.[RLAN REQ-PAR]) |
| Building entry loss | Based on Recommendation ITU-R P.2109 (Building type: Traditional (The loss will be lower.), probability: p = 0.5) |
| Clutter loss | Based on Recommendation ITU-R P.2108 (percentage of locations: p = 0.5) under the assumption that this recommendation is applied to the 5.2 GHz band |

[Editor’s note: Further clarifications are needed with regard to the probability used in ITU-R P.2109, ITU-R P.2108 and active ratio of RLANs]

Under these conditions, the total amount of interference level is calculated with Monte Carlo simulations and compared with the allowable interference level with random locations of RLAN devices with variable number of RLANs within the coverage of the satellite feeder uplink antenna as shown in Figure A1. The results are the following as shown in Table A2. If the number of RLANs of the 5 150-5 250 MHz band within the coverage of the satellite feeder uplink antenna is less than 113.91 million (6.04 million for outdoor use), which corresponds to 63.85 million (3.38 million for outdoor use) RLANs in the footprint of the satellite, the average of the total amount of interference from RLANs is less than the threshold.

[Therefore, if the number of RLANs within the coverage of the satellite feeder uplink antenna is limited, the sharing with non-GSO MSS feeder links is possible when WAS/RLAN systems are used outdoors.]

FIGURE A1

Interference from RLANs to the MSS feeder links

TABLE A2

The upper limit of the number of RLANs using the 5 150-5 250 MHz band

|  | The number of RLANs using 5 150-5 250 MHz (million). |
| --- | --- |
| Within the coverage of the satellite feeder uplink antenna (For outdoor use (5.3%)) | 113.91 (6.04) |
| In the footprint of the satellite (For outdoor use (5.3%)) | 63.85 (3.38) |

### 5.1.6	Study 6 (FRA 5A/616)

In this study, we consider several cases of indoor/outdoor/ EIRP RLAN distribution.

More than that, since the 5 150-5 250 MHz band is the only band where the DFS is not mandatory, and given the RTTT and radars deployment in the 5.8 GHz, this study would like to trigger the discussions on the possibility of enabling outdoor usage, with a limited power and restricted to in-vehicle usage.

#### 5.1.6.1	Methodology

The methodology used below consists in determining, in a dynamic analysis, the Cumulative Distribution Function (CDF) of interferences arising from the aggregated power of RLAN systems to the MSS receiver (Figure 10). Only the uplink path is studied in this paper.

Figure 10

Aggregate interference into the satellite

The total aggregate interference is the sum of contributions of each RLAN in the visibility of the satellite and can thus be written as follows

(1)

where:

N	is the total number of RLANs in the visibility of the satellite;

Ptn	is the nth RLAN EIRP (dBm);

Gtn 	is the nth RLAN Gain toward the satellite (dBi);

Grn 	is the Relative antenna gain (dBi) of the MSS receiver in the direction of the RLAN of index n;

Lossn 	is the calculated losses between the RLAN of index n and the MSS receiver, this value takes into account: the free space loss, the Building Entry Loss, the Clutter Loss and the polarization mismatch;

BFn 	is the Bandwidth factor correction associated to the nth RLAN, due to the fact that the MSS receiver and the RLAN do not have the same bandwidth, BFndB=10*Log10BWRLANBWMSS.

This interference value is computed for each satellite position and then is used to deduce the I/N (dB) or the ΔTT%. Repeating this operation for several satellite positions allows us to build a Cumulative Distribution Function (CDF) associated to the vector of observation.

#### 5.1.6.2	Scenario

The simulation determined the level of interference that would be experienced by a feeder link carrier that was uplinked from the earth station in Aussaguel, France (Latitude=43.42°, Longitude = 1.49°). This uplink carrier was switched from one spacecraft to another based on whichever spacecraft was the closest to the earth stations.

The simulation time step in our scenario was 10 Seconds, meaning that the satellite positions were recorded each 10 seconds. A quarter day of observation was taken into account (6h of record each 10 seconds), leading to 2 160 snapshots (All depicted in Figure 11).

Figure 11

Set of the satellites positions taken into account during the simulations

At each step, the aggregate RLAN interference into the satellite is computed using equation (1) and recorded into a storage vector. One should note that all the RLAN within the 5degree elevation contour (on ground) from the RLANs to the satellite are taken into account (Mainly from Europe, north of Africa and west of Asia.

An example is depicted in Figure 12, the satellite position (red circle) is Lat=46.8570°, Long=1.1860°. The five degree elevation contour is plotted in the same figure in green, and all the RLAN seeing the satllite with an elevation higher than 5 degrees are plotted in blue (over Europe, Africa and Asia). For information, this correspounds to 1 679 414 simultaneous active RLANs.

Simulations are also carried out, when taking into account the mitigation technique applied in US to minimize RLAN harmful interference to the spacecraft. As presented in Doc. 5A/210, RLAN with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW EIRP.

Figure 12

Example of RLANs taken into account in the interference computation for a satellite at position (Lat=48.326°, Long=10.489°). Green line is the 5° elevation contour, blue points are the RLANs
and Red circle is the Satellite position.

#### 5.1.6.3	RLAN parameters taken into account in the simulations

##### 5.1.6.3.1	RLAN EIRP

The RLAN EIRP taken into account in the simulation are the ones according to Table 1A and Table 3A, from the WDPNR ITU-R M.[RLAN REQ-PAR], Annex 27 to WP 5A Chairman’s Report.

Table 3

RLAN EIRP according to Table 1A

| RLAN e.i.r.p Level | 1 W (directional)* | 1 W (omni) | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 0% | 0% | 18% | 25.6% | 14.2% | 36.9% | 94.7% |
| Outdoor | 0.10% | 0.20% | 0.95% | 1.35% | 0.75% | 1.95% | 5.3% |
| * In the absence of antenna pattern the directional nature is taken into account only through the antenna discrimination toward the satellite, which is also applied to other omnidirectional devices |  |  |  |  |  |  |  |

Table 4

RLAN EIRP according to Table 3A

| RLAN e.i.r.p. Level | 1 W (directional)* | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | All |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor | 0% | 16.15% | 22.95% | 12.75% | 33.15% | 85% |
| Outdoor | 2.85% | 0% | 4.05% | 2.25% | 5.85% | 15% |
| *In the absence of antenna pattern the directional nature is taken into account only through the antenna discrimination toward the satellite, which is also applied to other omnidirectional devices |  |  |  |  |  |  |

Another distribution is proposed in Table 3, this latter takes into account the U.S. license-exempt rules for RLANs operating in the 5 150-5 250 MHz band, as presented in Doc. 5A/210 during the November 2016 WG 5A meeting.

Table 5

Proposed distribution when taking into account the US rules in the band 5 150-5 250 MHz

| RLAN e.i.r.p. Level | 4W (directionnal)* | 1 W (directional)* | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 0,00% | 0,00% | 15,15% | 21,95% | 11,75% | 31,15% | 80% |
| Outdoor | 3,00% | 2,85% | 2,00% | 4,05% | 2,25% | 5,85% | 20% |
| *In the absence of antenna pattern the directional nature is taken into account only through the antenna discrimination toward the satellite, which is also applied to other omnidirectional devices |  |  |  |  |  |  |  |

[Editor’s Note: The figures depicted in the table could be updated according to the results of M.[REQ-PAR].]

Finally, France also proposes to take into account the possibility to allow RLAN outdoor usage exclusively in vehicles in the 5 150-5 250 MHz.  The flowing distribution is proposed for this case. An additional attenuation of 5 dB should be taken into account to reflect the loss due to the vehicle’s body.

Table 6

Other proposed simulation scenario for vehicular usage

| RLAN e.i.r.p Level | 200 mW (omni) | 80 mW (omni) | 50 mW (omni) | 25 mW (omni) | All |
| --- | --- | --- | --- | --- | --- |
| Indoor | 16.15% | 22.95% | 12.75% | 33.15% | 85% |
| Outdoor on a vehicle* | 1.5% | 6% | 4.5% | 3% | 15% |
| * An additional attenuation of 5 dB should be taken into account for the outdoor RLANs in order to reflect the loss due to the vehicle screening. |  |  |  |  |  |

[Editor’s note: The figures depicted in the table could be updated according to the results of M.[REQ-PAR].]

##### 5.1.6.3.2	RLAN Bandwidth

The RLAN EIRP taken into account in the simulation are the ones according to Table 1A and Table 5A, from the WDPNR ITU-R M.[RLAN REQ-PAR], Annex 27 to WP 5A Chairman’s Report.

Table 7

RLAN Bandwidth

| RLAN Transmitter Bandwidth | 20 MHz | 40 MHz | 80 MHz | 160 MHz |
| --- | --- | --- | --- | --- |
| RLAN Device Percentage | 10% | 25% | 50% | 15% |

##### 5.1.6.3.3	RLAN Antenna Gain discrimination towards the satellite

The considered RLAN antenna gain discrimination towards the satellite is as supported by the French contribution dealing with the aggregate RLAN measurement submitted to November 2017 GT 5A meeting, namely Gtn =-4dB.

##### 5.1.6.3.4	RLAN Density and distribution

France considers that the density of simultaneous Active RLAN (AR) per inhabitant should be 0.0022 RLAN/inhabitant as supported by the French contribution dealing with the aggregate RLAN measurement, submitted to November 2017 GT 5A meeting.  This value is based on the statistics and prediction of the Joint Research Center (JRC) of 400 000 000 RLANs available in the market within 2025 (can be consulted at Ref.1 and Ref.2). This value is achieved using the following figures:

Table 8

Methodology to deduce the RLAN density per inhabitant in Europe

| Scenario | Number of AP | Busy hour factor | 5 GHz Spectrum factor | RF activity factor | BW factor | Total number of on-tune RLAN in Europe | Number of Inhabitant | RLAN Density / Inhabitant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Europe | 400 000 000 | 62.7% | 59% | 10% | 11% | 1 602 962 | 728 619 134 | 0,00222582 |

In the absence of any data regarding Africa and Asia, the density of RLAN per inhabitant is divided by 4 for these two continents (namely 5.5000e-04 simultaneous Active RLAN/inhabitant).

Regarding the population density (required to achieve the RLAN distribution), we used the density of population filings provided by the CIESIN (Ref.3), which consists of estimates of human population by 2.5 arc-minute grid cell, the example of Europe is depicted in Figure 13. The population density grids are derived by dividing the population count grids by the land area grid and represent persons per square kilometer (which makes it very precise).

Figure 13

Population density each 2.5 arc-minute over europe (log10 scale on the right)

For each kilometer square, the number of simultaneous active RLANs is deduced and then this RLANs are randomly distributed within this km square, attributing thus a latitude and a longitude to each individual RLAN.

This approach is very interesting, since it allows deducing the elevation by which each RLAN is seeing the satellite (and vice versa) and the distance between the RLAN and the satellite. These two parameters are necessary to compute the Losses (free space, clutter and BEL) and the antennas gains (Satellite or RLAN).

##### 5.1.6.3.5	RLAN heights

Two heights are considered for the RLAN: the first is 1.2m for terminals (70%) and the second one is 2 m for access points (30%).

##### 5.1.6.3.6	Mitigation technique

In our simulation we also applied the technical rules for RLANs operating in the 5 150-5 250 MHz band in U.S, for comparison purposes. As presented in Doc. 5A/210, RLAN with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW EIRP to minimize the likelihood of harmful interference to the operating MSS system. In our simulation, this is applied on the RLANs inside the 30° elevation contour. Statistically, this would have the same effect of attributing random elevations to all the RLANs and then applying it only to the ones with elevation greater than 30°. An example is depicted in Figure 14. In this case 4.2 % of the all observed RLANs are subject to the mitigation technique (70 581 over the 1 679 414 taken into account).

Figure 14

Representation of the outdoor RLANs with antenna elevation greater than 30 degrees
and with EIRP subject to the mitigation technique, in red points.

#### 5.1.6.4	Satellite parameters

##### 5.1.6.4.1	FSS Feeder link parameters

The case of the MSS system identified among ITU-R registrations as HIBLEO-X is studied. This system utilizes 48 spacecraft at an altitude of 1 414 kilometers and an inclination angle of the orbits of 52 degrees, with respect to the equator (see Figure 15).

The parameters of the feeder uplinks are summarized in Table 5, as provided in the GlobalStar contribution during the WG 5A May 2017 meeting (Doc.5A/395).

Table 9

MSS feeder Link Parameters

| Parameter | HIBLEO-4 FL |
| --- | --- |
| Satellite receiver noise temperature T (K) | 550 |
| Satellite receiver bandwidth B (MHz) | 16.5 |
| Protection criterion (dB) | -12.2* |
| * |  |

Figure 15

GlobalStar Constellation (HIBLEO-X)

##### 5.1.6.4.2	Spacecraft receive antenna pattern

The spacecraft antenna pattern is the one depicted in Globalstar’s contribution to WP 5A May 2017 in (Doc. 5A/395).

Figure 16

Spacecraft Receive Antenna Pattern versus off-nadir angle

#### 5.1.6.5	Propagation Loss

Four losses are taken into account in the path from the RLAN to the spacecraft:

•	Free-space loss, according to Rec. ITU-R P.525;

•	Building Entry Loss (BEL), according to the new recommendation being developed within WP-3K, Draft new Recommendation ITU-R P.2109. In the simulations, a mixture of 30% thermally efficient houses and 70% of traditional houses is considered. Regarding the probability  that loss is not exceeded, for each RLAN the probability that loss is not exceeded is picked up in the interval [0,1] according to a uniform law (bear in mind that there is up to 1.6 million RLAN within the satellite coverage);

•	Clutter Loss, according to Recommendation ITU-R P.2108-0 as advised by WP 3K. To do so, the clutter type is required. According to RLAN deployment density statistics in Europe (see Ref.1and Ref.2), 50% of households in the EU-28 area are located in densely populated areas, 23% in intermediate urban areas and 27% in thinly populated areas. These three areas are mapped to the urban, suburban and sparse homes clutter environments (see example over Europe in Figure 17). In the absence of any data for Africa and Asia, the same repartition is applied for these two continents as well. For information all the statistics are mapped in Table 10. Regarding the percentage of locations, the same procedure as for the BEL is used, for each RLAN a percentage of time is picked up in the interval [0,1] according to a uniform law;

•	Polarization mismatch, a value of 3 dB is considered according to what have been supported by France in during TG-5.1 (see Doc TG-5.1/104).

•	Vehicle screening attenuation, in this contribution being conservative, we considered an attenuation of 5 dB due to vehicle screening. It is applied only for the scenario described in Table 4. According to CEPT report 17, in the frequency range 3-6 GHz, cars with metallized windows provide a mean attenuation of about 15 dB and cars without any metallized windows provide a mean attenuation of about 8 dB. Cars with one window being metallized (front window) provide a mean attenuation of 12 dB.

Figure 17

Clutter type over Europe and parts of north Africa and asia, Red (Urban),
Yellow (Suburban) and Blue (Rural).

Table 10

Statistics for the three continents according to the data provided in Ref.3 and to the urban, suburban
and rural apportionment used in our simulations

| Continent | Number of Inhabitant | Simultaneous active RLAN density per Inhabitant | Total number of active RLAN density per Inhabitant | Urban | Suburban | Rural |
| --- | --- | --- | --- | --- | --- | --- |
| Europe | 728 619 134 | 0,0022 | 1 602 962 | 802 239 | 431 159 | 369 564 |
| Africa | 812 404 910 | 0,00055 | 446 822 | 225 199 | 103 455 | 118 168 |
| Asia | 3 676 824 724 | 0,00055 | 2 022 253 | 1 012 769 | 465 666 | 543 818 |

#### 5.1.6.6	Simulation results

In the following the three scenarios presented in Tables 1, 2, 3 and 4 are studied. The complementary cumulative distribution function is plotted based the computed I/N for all the positions of the satellite depicted above.

##### 5.1.6.6.1	Results for Table 1

Figure 18 depicts the simulation results assessed according to Table 1 (Table 1A Doc. [RLAN REQ-PAR]). We can observe, that the protection criterion of I/N=-12.2 dB (namely 6% for ∆T/T), is exceeded 73.5% of time when the mitigation technique is not applied and 56.5% of the time when the mitigation technique is applied. One shall notes that the case described in Table 1, take into account 5% of outdoor usage, which is mainly the accidental outdoor usage. The exceedance in I/N is about 3 dB.

Figure 18

Simulation results with and without the mitigation technique according to Table 1

##### 5.1.6.6.2	Results for Table 2

Figure 19 depicts the simulation results assessed according to Table 3A (Doc. [RLAN REQ-PAR]). We can observe, that the protection criterion of I/N=-12.2 dB is exceeded for all the satellites positions studied in our simulations scenario. The mitigation technique allows to reduce the exceedance of the criterion protection by ~3 dB.

Figure 19

Simulation results with and without the mitigation technique according to Table 2

##### 5.1.6.6.3	Results for Table 3

Figure 20 depicts the simulation results according to Table 3 of this document. Here again, we can observe, that the protection criterion of I/N=-12.2 dB, is exceeded for all the satellites positions studied in our simulations scenario. The mitigation technique allows to reduce the exceedance of the criterion protection by ~6 dB.

Figure 20

Simulation results with and without the mitigation technique according to Table 3

##### 5.1.6.6.4	Results for Table 4

Finally, Figure 12 depicts the results for Table 4. This table proposes an outdoor usage restricted to vehicular application. As stated earlier a 5 dB attenuation is added to take into account the body loss of the vehicle. In this particular case we observe that the protection criterion is exceeded by less that 1 dB only 15,88% of the time. According to those results the solution of vehicular outdoor deployment in this band seems to be the best solution to ensure the protection of the MMS feeder uplink.

Figure 21

Results for Table 4, outdoor vehicle usage, 5 dB additional attenuation to reflect vehicle screening effect.

#### 5.1.6.7	Conclusions

In this paper we studied the possible impact of relaxing the outdoor usage of RLANs in the band 5 150-5 250 MHz, as well as the power rise. According to our simulation, we can drive the following conclusions:

•	Results according to Table 1 show an exceedance of less than 3 dB of the protection criterion is encountered. This scenario corresponds more likely to accidental outdoor usage, and it is very unlikely to be able to control such a percentage of outdoor usage by the administrations.

•	Results according to tables 2 and 3, take into account 15% to 20% of outdoor usage with a power rise to 1W or 4W, in these cases the protection criterion is never respected according to our simulations, and seem to be harmful for MSS. Even with the mitigation technique proposed in Doc 5A/210 the protection criterion could not be reached. More specifically it appears to be very difficult to monitor the antenna elevations deployment in a License exempted regime.

•	Finally, we considered a case of outdoor relaxation limited to in-vehicle usage, taking into account a maximum EIRP of 200 mW and taking into account a 5 dB loss due to the vehicle body, this scenario seems to be the most suitable for outdoor RLAN usage in the 5 150-5 250 MHz band, specially that this band is DFS free.

#### 5.1.6.8	References

[1]	JRC study on RLAN deployment and device densities:  http://www.cept.org/Documents/se-24/25709/SE24(15)070R0_WI52_number-of-RLAN-JRC

[2]	JRC study on in-home RLAN coverage and number of RLAN APs per household: http://www.cept.org/Documents/se-24/25805/SE24(15)089R0_WI52_Further-considerations-on-WLAN-indoor-coverage

[3]	Center for International Earth Science Information Network - CIESIN - Columbia University, and Centro Internacional de Agricultura Tropical - CIAT. 2005. Gridded Population of the World, Version 3 (GPWv3): Population Density Grid. Palisades, NY: NASA Socioeconomic Data and Applications Center (SEDAC). http://dx.doi.org/10.7927/H4XK8CG2, accessed August 2017.

## 5.2	Sharing and compatibility of Aeronautical radionavigation versus WAS/RLAN in the 5 150-5 250 MHz

### 5.2.1	Study 1 (RUS 5A/397)

The analysis of Recommendation ITU-R М.2007 shows that it contains only description of airborne sense and avoid systems with the maximum permissible interference-to-noise ratio (I/N) as the protection criterion at the receiver input.

Therefore the potential interference scenario given in Figure 1 was used in the studies.

It is proposed to define the maximum permissible interference level at the airborne radar receiver input to estimate the WAS/RLAN system impact to the airborne radar receivers.

Figure 1

Interference scenario for airborne radiodetermination radar receiver

Based on the obtained value the minimum protection distance ensuring compatibility of WAS/RLAN systems with airborne radar receivers can be defined.

The maximum permissible interference power at the receiver input is estimated by the following equation:

, dBW,	(1)

where:	 - required interference/noise ratio at the receiver input;

dBW,	(2)

k – Boltzmann constant,

– noise temperature, degrees К,	(3)

NF – radar noise figure, dB;

- radar operating frequency band, Hz

To estimate compatibility of RLAN with airborne radars (Scenario 1) the required protection distances were determined to ensure operation of the radars without interference in different operation modes of RLANs. The protection distance was determined in accordance with propagation model provided in Recommendation ITU-R Р.525 by the following equation:

,	(4)

where 		,	(5)

- radar antenna gain, dB;

λ – operational wavelength, m;

σ – cross-wall fading, dB.

,	(6)

where:	EIRPeff  1- effective e.i.r.p. of single WAS/RLAN transmitter;

N – number of WAS/RLAN transmitters.

The estimations assumed an aircraft flying at 10 km altitude (Н=10 000 m). Interference to operation of the air-borne receiver of sense and avoid system is caused by indoor and outdoor RLAN transmitters. The free space propagation model described in Recommendation ITU-R Р.525 is used to estimate the interference caused by RLAN transmitters. To take propagation loss in the walls into account in equation (4) additional propagation loss, σ, equal to 20 dB was considered. Multi-source interference was taken into account using equation (6).

The permissible interference power at the airborne sense and avoid receiver input was calculated by the equation (2)-(4) indicated above and is equal to minus 131.9 dBW. It was used to determine the required protection distances providing interference free radar operation in case of indoor and outdoor WAS/RLAN systems operation.

Table 3 presents the calculation results of the protection distances required between the airborne receivers of sense and avoid radar and single outdoor and indoor RLAN transmitters.

table 3

Separation distances required for protection of air-borne radars from indoor and outdoor RLAN, KM

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Protection distance | 51 | 18 | >RLOS | 180 |

*RLOS – line-of-sight distance equals 430 km for a typical flight altitude of 10 000 m and WAS/RLAN transmitter height of 20 m.

The analysis of the results provided in Table 3 shows that in case of outdoor WAS/RLAN systems usage the required protection distance can be increased significantly in comparison with the case of indoor WAS/RLAN deployment. Such usage even of single outdoor WAS/RLAN usage can lead to the case when the required protection distance will exceed the line-of-sight distance for a certain bandwidth of WAS/RLAN system.

In addition the required protection distances were estimated in case when three RLAN transmitters deployed in one building operate simultaneously. The estimation results are presented in Table 4.

TABLE 4

Separation distances required for protection of airborne radars from three indoor and outdoor rlan systems, km

|  | EIRPeff=-7 dBW, σ=20 dB |  | EIRPeff=-7 dBW, σ=0 dB |  |
| --- | --- | --- | --- | --- |
| ΔFRLAN, MHz | 20 | 160 | 20 | 160 |
| Protection distance | 88 | 31 | >RLOS | 311 |

While increasing the number of outdoor WAS/RLAN systems falling into the main lobe of the airborne sense and  avoid antenna pattern up to 7 the required protection distance exceeds the line-of-sight distance for both types of the considered WAS/RLAN signals.

The obtained results allow to conclude that the compatibility of outdoor WAS/RLAN systems with the airborne radars will be quite complicated without taking the additional measures for reducing interference.

### 5.2.2 	Study 2 (JPN 5A/586)

[Editor’s note: The text below need to be carefully considered at next WP 5A meeting, especially for the probability.]

Another example of a sharing study with realistic conditions is conducted as follows.

The parameters and protection criteria of the ARNS system are assumed to be almost the same as described in section 4.1 (also described in Recommendation ITU-R M.2007) and the threshold of the total amount of interference from RLANs is -101.9 dBm with the following conditions in Table A3.

TABLE A3

Additional conditions for a sharing study with ARNS systems

| Parameters | Values and conditions |
| --- | --- |
| e.i.r.p. distribution | Based on Table 1A in Report ITU-R M.[RLAN REQ-PAR] (The outdoor usage ratio is 5.3%) |
| Antenna pattern for RLANs | Based on Recommendation ITU-R M.1652 (If the RLAN uses e.i.r.p. of 1 W, the elevation angle mask defined for 5 250-5 350 MHz band in the Resolution 229 (Rev.WRC-12) is applied.) |
| Height of RLAN antenna | 4.5 m |
| Active ratio of RLANs | 4.645% (derived from Report ITU-R M.[RLAN REQ-PAR]) |
| Antenna pattern for ARNS systems | Based on Recommendation ITU-R M.1652 (The maximum antenna gain is 36 dBi as defined in Recommendation ITU-R M.2007.) (*1) |
| Altitude of ARNS systems | 10 km |
| Building entry loss | Based on Recommendation ITU-R P.2109 (Building type: Traditional (The loss will be lower.), probability: p = 0.5) |
| Additional loss | 17 dB (defined for airborne radars in Recommendation ITU-R M.1652). |
| (*1) Recommendation ITU-R M.2007 specifies the maximum antenna gain. However it does not contain antenna patterns. This sharing study adopts antenna patterns used in Annex 6 to Recommendation ITU-R M.1652. |  |

Under these conditions, the total amount and distribution of interference level is calculated based on Monte Carlo simulations and compared with the allowable interference level with random locations of RLAN devices with variable number of RLANs within the line-of-sight range of the ARNS systems as shown in Figure A2.

The minimum distance between RLANs and ARNS systems is assumed to be 0 km or 20 km. The elevation angle of the ARNS antenna is assumed to be -45 degrees that is the lower limit and corresponds to the maximum interference level from a RLAN device near the ARNS system.

The results are shown in Table A4. When no limitation is applied to RLAN locations, if the upper limit of the number of RLANs of the 5 150-5 250 MHz band within the line-of-sight range of the ARNS system is 103.15 million (5.47 million for outdoors), the interference is less than the threshold with a probability of 90%. When the minimum distance between RLANs and ARNS systems is 20 km, the upper limit of the number of RLANs is 210.79 million (11.17 million for outdoor use) in the same way.

Accordingly if the number of RLANs or the separation distance between RLANs and ARNS systems is controlled, the interference level will be less than the threshold with a certain probability. Therefore the sharing with ARNS systems is possible when WAS/RLAN systems are used outdoors.

FIGURE A2

Interference scenario from RLANs to ARNS systems

TABLE A4

The upper limit of the number of RLANs that cause interference levels less than the threshold
with a probability of 90%

|  | Minimum distance between RLANs and ARNS systems: 0 km | Minimum distance between RLANs and ARNS systems: 20 km |
| --- | --- | --- |
| The number of RLANs using 5 150-5 250 MHz within the line-of-sight range of ARNS systems (million) (For outdoor use (5.3%)) | 103.15 (5.47) | 210.79 (11.17) |

## 5.3	Sharing and compatibility of Aeronautical Mobile Service limited to aeronautical mobile telemetry (AMT) for flight testing versus WAS/RLANs in the 5 150-5 250 MHz band

[TBD]

# 6	Conclusions of sharing and compatibility studies per service

## 6.1	General considerations

## 6.2	Sharing and compatibility results in the band 5 150-5 250 MHz

[USA 5A/381]

Some administrations have enabled RLAN operations that are beyond restrictions specified in Resolution 229 (WRC-12).  Specifically, these administrations authorized RLAN use of this band in co-existence with MSS operations through e.i.r.p. limitations at higher antenna elevation angles: the authorization generally permits indoor and outdoor RLAN operations in 5 150-5 250 MHz at up to 1 Watt conducted and a power spectral density (PSD) of 17 dBm/MHz with an allowance for a 6 dBi antenna gain (i.e. a total 36 dBm e.i.r.p.).  The outdoor operation of WAS/RLANs devices are permitted in the 5 150-5 250 MHz band at these power levels, except that such operations with antenna elevation angles in excess of 30 degrees from the horizon must not exceed 125 mW e.i.r.p. to minimize the likelihood of harmful interference to the operating MSS system.  Expressing a limit in terms of e.i.r.p. provides flexibility regarding how to achieve compatibility with non-GSO MSS feeder uplinks.  It is important to note that one of these administrations is also the notifying administration for the single non-GSO MSS system operating in this band and that, to date, no interference issue have been reported to that administration’s regulator. Also, while in-band e.i.r.p. was increased, unwanted emission levels were retained such that all WAS/RLAN station emissions outside of the 5 150 – 5 350 MHz frequency range shall not exceed an e.i.r.p. of −27 dBm/MHz

[RUS 5A/397]

The results of the conducted studies allow to make the following conclusions:

−	sharing of outdoor WAS/RLAN systems operating in the frequency band 5 150-5 250 MHz having the current characteristics with airborne sense and avoid systems is unfeasible;

−	the effective measures for reducing interference for airborne sense and avoid systems operation are to be developed to enable the usage of outdoor WAS/RLAN in the frequency band 5 150-5 250 MHz. The reduction of e.i.r.p values of WAS/RLAN transmitters approximately by 20 dB while increasing the receiver sensitivity can be considered as the effective method for reducing interference. Such method allows to compensate the absence of additional fading in the walls which provided sharing of WAS/RLAN systems with the ARNS systems operating in the considered frequency band.

Without development and implementation of such measures for reducing the interference the decision of possible outdoor WAS/RLAN systems usage in the considered frequency band cannot be made.

[JPN]

Regarding outdoor operations of WAS/RLAN systems in the 5 150-5 250 MHz band, the results of some examples of sharing studies show that WAS/RLAN systems are compatible with non-GSO MSS feeder links and ARNS systems. As an interference mitigation technique, the elevation angle mask defined for the 5 250-5 350 MHz band in Resolution 229 (Rev.WRC-12) is effective. In addition, limitation of the number of RLAN devices, ensuring the minimum distance between RLANs and ARNS systems and the limitation of the locations of RLANs are also effective.

List of References

[1]	EC Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for Wireless Access Systems including Radio Local Area Networks in the bands 5 350-5 470 MHz and 5 725-5 925 MHz ('WAS/RLAN extension bands') for the provision of wireless broadband services, September 2013

[2]	ECC Decision (04)08: "Harmonised use of 5 GHz for the implementation of WAS/RLANs"

[3]	WRC Resolution 229 (WRC-03): "Use of the bands 5 150-5 250, 5 250-5 350 MHz and 5 470-5 725 MHz by the mobile service for the implementation of Wireless Access Systems including Radio Local Area Networks"

[4]	ERC Report 67: "Study of the Frequency sharing between HIPERLANs and MSS feeder links in the 5 GHz band"

[5]	ERC Report 72: "Compatibility studies related to the possible extension band for HIPERLAN at 5 GHz"

[6]	ETSI EN 301 893: "Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonized EN covering the essential requirements of Article 3.2 of the R&TTE Directive"

[7]	EC Decision 2005/513/EC complemented by EC Decision 2007/90/EC

[8]	Recommendation ITU-R M.1739: "Protection criteria for wireless access systems, including radio local area networks, operating in the mobile service in accordance with Resolution 229 (WRC-03) in the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz"

[9]	ERC Report 25: "The European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz"

[10]	EC Decision 2008/671/EC: "Commission Decision of 5 August 2008 on the harmonised use of radio spectrum in the 5 875-5 905 MHz frequency band for safety related applications of Intelligent Transport Systems (ITS)"

[11]	ECC Decision (08)01: " ECC Decision of 14 March 2008 on the harmonised use of the 5 875-5 925 MHz frequency band for Intelligent Transport Systems (ITS)"

[12]	ECC Recommendation (08)01: " Use of the band 5 855-5 875 MHz for Intelligent Transport Systems"

[13]	ECC Report 101: "Compatibility studies in the band 5 855-5 925 MHz between Intelligent Transport Systems (ITS) and other systems"

[14]	ECC Decision (12)04: " ECC Decision of 2 November 2012 on the withdrawal of ECC Decision (02)01. For national implementation issues related to the withdrawn Decision, check EFIS"

[15]	ERC Recommendation 70-03: "Relating to the use of Short Range Devices (SRD)"

[16]	ETSI EN 300 674: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band; Part 1: General characteristics and test methods for Road Side Units (RSU) and On-Board Units (OBU)"

[17]	CENELEC EN 12253: "" Road transport and traffic telematics - Dedicated short-range communication - Physical layer using microwave at 5.8 GHz"

[18]	ECC Report 68: "Compatibility studies in the band 5 725-5 875 MHz between Fixed Wireless Access (FWA) systems and other systems"

[19]	ITU-R Recommendation P.452: "Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz"

[20]	EC Decision 2006/771/EC: "Commission Decision of 9 November 2006 on the harmonisation of the radio spectrum for use by short-range devices"

[21]	IARU Region-1 VHF Managers Handbook

[22]	Recommendation ITU-R M.1732-1: "Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies"

[23]	ECC Report 210: "Compatibility/sharing studies related to Broadband Direct-Air-to-Ground Communications (DA2GC) in the frequency bands 5 855-5 875 MHz, 2 400-2 483.5 MHz and 3 400-3 600 MHz"

[24]	ETSI TR 101 599: "Electromagnetic compatibility and Radio spectrum matters (ERM) System Reference Document (SRDoc); Broadband Direct-Air-to-Ground Communications System employing beamforming antennas, operating in the 2,4 GHz and 5,8 GHz bands"

[25]	ETSI TR 103 108: "Electromagnetic compatibility and Radio spectrum Matters (ERM); System Reference document (SRdoc); Broadband Direct-Air-to-Ground Communications System operating in the 5,855 GHz to 5,875 GHz band using 3G technology"

[26]	ECC Report 206: "Compatibility studies in the band 5 725-5 875 MHz between SRD equipment for wireless industrial applications and other systems"

[27]	IEEE 802.11-2012: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

[28]	ETSI EN 302 571 V1.2.2 (2011-10): "Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive"

[29]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain"

[30]	ERC Recommendation 74-01: "Unwanted Emissions in the Spurious Domain"

[31]	Recommendation ITU-R F.1336-3: "Reference radiation patterns of omnidirectional, sectoral and other antennas for the fixed and mobile service for use in sharing studies in the frequency range from 400 MHz to about 70 GHz"

[32]	ETSI ES 200 674-1 V2.4.1 (2013-05): “Technical characteristics and test methods for High Data Rate (HDR) data transmission equipment operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band”

[33]	NTIA 5 GHz report, January 2013, “Evaluation of the 5 350-5 470 MHz and 5 850-5 925 MHz bands pursuant to section 6406(b) of the middle class tax relief and job creation act of 2012

[34]	ETSI TR 102 960: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (RTTT DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range; Evaluation of mitigation methods and techniques"

[35]	ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range"

[36]	http://www.statistiques-mondiales.com/union_europeenne.htm

[37]	http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=lfst_hhnhtych&lang=en

[38]	https://www.ssb.no/en/befolkning/statistikker/familie/aar/2014-12-12

[39]	http://www.bfs.admin.ch/

[40]	http://appsso.eurostat.ec.europa.eu/nui/show.do?dataset=bd_9ac_l_form_r2&lang=en

[41]	3GPP TR 36.889: Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum, http://www.3gpp.org/DynaReport/36889.htm

[42]	CEPT Report 57 : "Compatibility and sharing conditions for WAS/RLAN in the bands 5 350-5 470 MHz and 5 725-5 925 MHz"

[43]	ECC Report 181 on improving spectrum efficiency in SRD bands

[44]	JRC study on RLAN deployment and device densities:  http://www.cept.org/Documents/se-24/25709/SE24(15)070R0_WI52_number-of-RLAN-JRC

[45]	JRC study on in-home RLAN coverage and number of RLAN APs per household: http://www.cept.org/Documents/se-24/25805/SE24(15)089R0_WI52_Further-considerations-on-WLAN-indoor-coverage

[46]	JRC measurements of 802.11ac RLAN APs operating in the 5 GHz band: http://www.cept.org/Documents/se-24/24051/SE24(15)042R0_WI52_JRC_5GHZ_RLAN-_measurements

[47]	ECC Report 244 “Compatibility studies related to RLANs in the 5 725-5 925 MHz band”

____________________

https://celestrak.com/satcat/search.asp.

Globalstar, L.P., “Description of the Globalstar System”, GS-TR-94-0001Revision E, December 07, 2000.

http://web.ornl.gov/sci/landscan/landscan_documentation.shtml.

See R12 JTG 4567-C-0584.

ITU-R R12-JTG 4567 348.

This was an earlier version M.[RLAN REQ-PAR].

Comparison of User Traffic Characteristics on Mobile-Access versus Fixed-Access Networks, MIT, AKAMAI TECHNOLOGIES (July 2010) (“50 Country Study”).

https://www.statista.com/topics/2237/internet-usage-in-the-united-states/.

http://www.worldometers.info/world-population/population-by-country/.

For this study Mexico is assumed to be a proxy for all of Latin America.

http://www12.statcan.gc.ca/census-recensement/2016/dp-pd/prof/details/Page.cfm?Lang=E&Geo1=PR&Code1=35&Geo2=&Code2=&Data=Count&SearchText=Ontario&SearchType=Begins&SearchPR=01&B1=All&GeoLevel=PR&GeoCode=35.

https://www.statista.com/statistics/183648/average-size-of-households-in-the-us/.

https://www.efe.com/efe/english/life/mexican-households-have-an-average-of-3-8-members-843-in-monthly-income/50000263-2666718.

This assumes a single AP per household.

www.ruckuswireless.com.

This figure comes from the Excel table embedded in section 3.6 of the Working document towards a preliminary draft new Report ITU-R M.[RLAN REQ-PAR] and needs to be further reviewed after the working document has been finalized.