| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/403 | Annex 8 to Document 5A/1065-E |
|  | 9 May 2019 |
|  | English only |
| Annex 8 to Working Party 5A Chairman’s Report |  |
| working document towards a PRELIMINARY DRAFT NEW REPORT ITU-R M.[100-ghz.RSTT.COEXIST] |  |
| Coexistence between high-speed railway radiocommunication system between train and trackside operating in the frequency bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz, and active and passive services |  |

# 1	Introduction

Condition based maintenance (CBM) can provide mean-time-before-failure (MTBF) independent maintenance and safety train control and operation.  However, in order to introduce CBM in high-speed railway systems, contiguous high-capacity wireless links between in-vehicle and ground networks over several Gb/s are required under high-speed movement around 600 km/hour. According to Article 5 to Chapter II to Radio Regulations, the total frequency band of 15.4 GHz is allocated for mobile services (MSs) in the frequency range 92-109.5 GHz. These MS bands have a potential to support high-capacity signal transmission between in-vehicle and ground networks. Other advantages to use such high frequencies around 100 GHz are that transceivers can be made compact and equipped in high-speed trains, and the pencil beams which suppress the sidelobe level can be utilized between transceivers. However, high-speed railway radiocommunication systems between train and trackside should be deployed to protect the incumbent services operating in the frequency bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz, and the adjacent bands. This Report intends to provide sharing and compatibility study results between high-speed railway radiocommunication system between train and trackside and the active and passive services.

# 2	Scope

This Report provides results of sharing and compatibility studies between high-speed railway radiocommunication system between train and trackside operating in the bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz, and active and passive services operating in these or adjacent bands.

# 3	Related Recommendations and Reports

| Recommendation ITU-R M.[RSTT.FRQ] (Annex 4 to Document 5A/1065) | Spectrum harmonization for railway radiocommunication systems between train and trackside (RSTT) |
| --- | --- |
| Recommendation ITU-R P.452 | Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz |
| Recommendation ITU-R P.1411 | Propagation data and prediction methods for the planning of short-range outdoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 100 GHz |
| Recommendation ITU-R RA.769 | Protection criteria used for radio astronomical measurements |
| Recommendation ITU-R RS.1166 | Performance and interference criteria for active spaceborne sensors |
| Recommendation ITU-R RS.1813 | Reference antenna pattern for passive sensors operating in the Earth exploration satellite service (passive) to be used in compatibility analyses in the frequency range 1.4-100 GHz |
| Recommendation ITU-R RS.1861 | Typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz |
| Recommendation ITU-R RS.2017 | Performance and interference criteria for satellite passive remote sensing |
| Recommendation ITU-R RS.2105 | Typical technical and operational characteristics of Earth exploration-satellite service (active) systems using allocations between 432 MHz and 238 GHz |
| Report ITU-R F.2239 | Coexistence between fixed service operating in 71-76 GHz, 81-86 GHz and 92-94 GHz bands and passive services |
| Report ITU-R M.2418 | Description of railway radiocommunication systems between train and trackside (RSTT) |
| Report ITU-R M.2442 | Current and future usage of railway radiocommunication systems between train and trackside (RSTT) |

# 4	List of acronyms and abbreviations

| RAU | Radio access unit |
| --- | --- |
| RSTT | Railway radiocommunication system between train and trackside |

# 5	Summary of coexistence of 100 GHz band RSTT with the passive and active services

Table 1 shows the frequency band which are already allocated for use of mobile services in the frequency range 92-109.5 GHz. In accordance with Article 5 to Chapter II to Radio Regulations (see Annex), in the adjacent bands of those frequencies all emissions are prohibited in the following bands; 86-92 GHz, 100-102 GHz and 109.5-111.8 GHz. To coexist with active and passive services, the same schemes developed by Report ITU-R F.2239, “Coexistence between fixed service operating in 71-76 GHz, 81-86 GHz and 92-94 GHz bands and passive services”, could be used for sharing and compatibility studies of railway radiocommunication systems. The following sharing and compatibility cases should be addressed, as shown in Figure 1:

1)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 92-94 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 86-92 GHz;

2)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the bands 94.1-100 GHz and 102-109.5 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 100-102 GHz;

3)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the band 102-109.5 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations operating in the adjacent band 109.5-111.8 GHz;

4)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz with respect to the protection of radio astronomy service (RAS) stations operating in the band 86-111.8 GHz;

5)	mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the bands 92-94 GHz and 94.1-100 GHz with respect to the protection of Earth exploration-satellite service (EESS) stations (active) operating in the adjacent band 94-94.1 GHz;

6)	Earth exploration-satellite service (EESS) stations (active) operating in the band 94-94.1 GHz with respect to protect mobile service stations such as on-board radio equipment and related radio infrastructure located along trackside operating in the adjacent bands 92-94 GHz and 94.1-100 GHz.

Table 1

Frequency bands already allocated for mobile servicers

|  | 92-94 |  | 94.1-100 |  | 102-109.5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MS |  | MS |  | MS |  |
|  | BW1=2 GHz |  | BW2=5.9 GHz |  | BW3=7.5 GHz |  |

FIGURE 1

Sharing and compatibility schemes for coexistence between mobile services and passive services

# 6	System deployment scenarios

## 6.1	System architecture

Figure 2 shows the schematic concept of 100 GHz wireless connection between on-board equipment and trackside radio access unit. The concept shows that 10 trackside radio access units (RAUs) with two antennas are equipped along the railway line. Two on-board transceivers are equipped with the driver’s room located at the first car of the train and the conductor’s room located at the end car of the train to provide stable communication links between train and trackside. Both on-board transceivers are complementally connected to the trackside radio access units to seamlessly maintain link connection through 100-GHz signals.

FIGURE 2

Concept of 100-GHz wireless connection between on-board and trackside equipment

## 6.2	Linear-cell configuration

Figure 3 illustrates linear-cell configuration of 100-GHz RSTT. The communication zones are formed by the trackside RAUs located along the rail lines. The coverage length of each communication zone is designed in the range 0.5-3 km, depending on the railway line environment such as viaduct, tunnel, cutting, bridge, etc. The same channels are assigned to e.g. communication zones #1 to #N covered by trackside RAUs #1-#N which are connected to node base station through radio over fiber links to transmit/receive high-speed data signals. The communication zones from #1 to #N corresponds to the single frequency network which utilizes frequency channels efficiently. Figure 4 shows the blockdiagram of node base stations-F/B and trackside RAUs-F/B. The node base station-F transmits/receives multi-channels subcarriers to/from the trackside RAU-F in which no digital signal processing units are equipped and the node base station-B to/from the trackside RAU-B. The four transceivers for forward and backward links are equipped at the same trackside RAU location.

FIGURE 3

Illustration of linear-cell configuration of 100-GHz RSTT

FIGURE 4

Blockdiagram of node base station and trackside RAU

## 6.3	Number of on-board transceiver

Since two on-board transceivers are equipped with the driver’s room located at the first train vehicle and the conductor’s room located at the last train vehicle, the total number of on-board transceivers on super express train set in Japan Railway will be four. The number of train set of daily operated super express trains in all Japan Railway companies is shown in Figure 5. The maximum train set of 230 is observed in the evening of 19:00-20:00 hour. Then the total number of on-board transceivers which are operated in 19:00-20:00 hour becomes 920. This number may be used for sharing and compatibility studies taking into account the coverage area of satellite antennas.

FIGURE 5

Total number of super express train set per one day

©Hitachi Kokusai Electric Inc., Data Source: http://ekitan.com/.

## 6.4	Number of trackside radio access unit

Figure 6 shows the currently operated super express service lines in Japan Railway companies. Table 2 summarizes the super express service line length and maximum speed of each Japan Railway company. The distance between on-board transceivers and trackside RAUs are changed in the range of 0.5-3 km in the condition of operational environment. In the estimation of the number of trackside RAUs, the trackside RAU is equipped 1 km interval along the super express service line. Table 2 also estimated the number of trackside RAUs. One trackside RAU has four transceivers which are independently connected to four on-board transceivers equipped on the super express train set, as described in section 6.2.

FIGURE 6

Map of super express service lines in Japan

©2011-2017 Nippon Communications Foundation

Table 2

Estimated number of trackside RAU in each super express service line in Japan

| Name of super express line | Service area | Line length (km) | Maximum speed (km/hour) | Estimated number of transceivers | Total tunnel length (km) |
| --- | --- | --- | --- | --- | --- |
| Tokaido line | Tokyo – Shin Osaka | 562.6 | 285 | 2 248 | 68.663 |
| Sanyo line | Shin Osaka – Hakata | 644.0 | 300 | 2 576 | 280.492 |
| Tohoku line | Tokyo – Shin Aomori | 713.7 | 320 | 2 852 | 236.0 |
| Hokkaido line | Shin Aomori – Shin Hakodatehokuto | 148.8 | 260 | 592 | 94.48 |
| Yamagata line1 | Fukushima – Shinjo | 148.6 | 130 | 592 | – |
| Akita line1 | Morioka – Akita | 127.3 | 130 | 508 | – |
| Jouetsu line | Tokyo – Niigata | 333.9 | 240 | 1 332 | 106.789 |
| Hokuriku line | Tokyo – Kanazawa | 450.5 | 260 | 1 800 | 97.289 |
| Kyushu line | Hakata – Kagoshimachuou | 288.9 | 260 | 1 152 | 125.021 |
| 1 Yamagata and Akita lines are not included in super express service line, but the same train vehicles are operated in those lines. |  |  |  |  |  |

## 6.5	Estimation of RSTT link density

The highest RSTT link density can be expected at the center area of Tokyo because Tokaido line, Tohoku line, Yamagata line, Akita line, Jouetsu line and Hokuriku line are terminated at Tokyo station. RSTT link density is defined as a ratio of the total number of super express trains in operation and the service railway distance between Shin-Yokohama and Omiya Stations. The RSTT link density and the activated transceivers density in urban area can be estimated to 0.372 vehicle/km and 2.98 transceivers/km from Table 3. This number may be used for sharing and compatibility studies as the worst-case scenario.

Table 3

Estimated number of super express trains operating between Shin-Yokohama and Omiya Stations

| Station | Distance from Tokyo (km) | Estimated number of vehicles in operation* | Number of Activated transceivers |
| --- | --- | --- | --- |
| Shin-Yokohama | 28.8 | 7 | 56 |
| Shinagawa | 6.8 | 3 | 24 |
| Tokyo | 0 | 0 | 0 |
| Ueno | 3.6 | 4 | 32 |
| Omiya | 30.3 | 8 | 64 |
| * This number is estimated by adding all vehicles operating in Tokaido line, Tohoku line, Yamagata line, Akita line, Jouetsu line and Hokuriku line at the time around 19:30 when is the busiest time for business trip. |  |  |  |

Figure 7 shows the location of stations and super express service lines in Tokyo area where the highest RSTT link density is estimated. The super express trains are distributed between each station in the hour when the peak train numbers are operated. This area provides the worst-case interference scenario to any active and passive service applications.

FIGURE 7

Super express service lines and stations around Tokyo area

In rural area, the RSTT link density can be estimated in the condition that fifteen super express trains with a maximum speed of 285 km/hour can run on the same service line simultaneously within one hour. The distance between each train set becomes about 20.3 km. The RSTT link density and the activated transceivers density in rural area can be estimated to be 0.05 vehicle/km and 0.39 transceivers/km. In suburban area, the super express train decreases the speed and the average speed is around 180 km/hour. The RSTT link density and the activated transceivers density in suburban area can be estimated to be 0.078 vehicle/km and 0.62 transceivers/km. Table 4 summarizes the estimated RSTT and activated transceiver densities.

Table 4

Estimated RSTT link and activated transceivers density in urban, suburban and rural areas

| Area | RSTT link density (Vehicle/km) | Activated transceivers density (Transceiver/km) |
| --- | --- | --- |
| Urban | 0.372 | 2.98 |
| Suburban | 0.078 | 0.62 |
| Rural | 0.049 | 0.39 |

## 6.6	Antenna elevation of on-board transceivers

Table 5 summarizes the operational environment of super express service lines. Since the maximum gradient of track is 35 ‰, +2-degree of additional antenna elevation of on-board unit and trackside RAU should be taken into account when sharing and compatibility studies are conducted.

Table 5

Operational environment of super express service line

| Parameters | Values |
| --- | --- |
| Roadbed width | Typ. 12 m |
| Vehicle width | Max. 3.4 m |
| Vehicle height | Max. 4.5 m |
| Minimum radius of curve | Typ. 4 000 m |
| Minimum vertical radius of curve | Typ. 10 000 m |
| Maximum gradient of track | 35‰* |
| Maximum superelevation of track | 200 mm @ rail gauge (1 435 mm) |
| * The unit ‰ is pronounced as per mill and 1‰ is 0.001 = 0.1%. |  |

# 7	System characteristics

## 7.1	System characteristics of railway radiocommunication system between train and trackside operating in the bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz

Table 6 summarizes technical and operational characteristics of RSTT stations operating in 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz bands. The total bandwidth of 15.4 GHz can be used for data transmission between on-board radio equipment and trackside radio access units. The transmission distance of these equipment is designed by the railroad line environment.

Table 6

System parameters

| Frequency range (GHz) | 92-94. 94.1-100, 102-109.5 |
| --- | --- |
| Seamless connection mechanism | Backward and forward switching method |
| Channel bandwidth (MHz) | 400 |
| Channelization (MHz) | See Figure 7 and Annex 5 |
| Channel aggregation pattern | See Figure 8 |
| Antenna type | Cassgrain |
| Antenna gain (dBi) | 42 (RAU), 35 (On-board unit) |
| Antenna beamwidth (degree) | 1 |
| Antenna height from rail surface (m) | 4 (Maximum) |
| Polarization | Linear |
| Antenna pattern | See Annex 1 and Recommendation ITU-R F.699 |
| Average transmitting power (dBm) | 10 |
| Average e.i.r.p. (dBm) | 54 |
| Receiving noise figure (dB) | <10 |
| Maximum transmission data rate (Gb/s) | 5-10 (Stationary), 1 (Running) |
| Maximum transmission distance (km) | 0.5-1 (Open), 3 (Tunnel) |
| Modulation | PSK, QPSK, 16QAM, 64QAM |
| Multiplexing method | FDD/TDD |
| Space diversity | TBD |
| Maximum running speed (km/h) | 600 |
| Switching time of trackside radio access unit (s) | TBD |
| Average distance between on-board equipment and trackside radio access unit | TBD |
| Rainfall attenuation margin (dB) | TBD |
| Wired interface of trackside radio access unit | [Recommendation ITU-T G.RoF] |
| Propagation model between train and trackside | Recommendation ITU-R P.1411 |
| I/N (dB) | TBD |

Figure 8 shows the channel arrangement of 100-GHz RSTT which is categorized into three groups. Each group has guard bands at both ends of frequency bands to avoid frequency interferences to the existing radiocommunication services (see Annex 5). Figure 8 shows two channel aggregation pattern which is based on minimum Nyquist bandwidth of 250 MHz and roll-off rate of 0.25. This pattern can ideally transmit 0.5 Gb/s when QPSK modulation technique with a symbol rate of 250 Mb/s is used. Since the maximum data rate per channel is 2 Gb/s when 64 QAM modulation technique is applied, the 8 Gb/s data transmission is feasible by 4-channel bonding pattern.

Figure 9 shows spectrum mask of QPSK modulated 4-channel bonding pattern at RF band. Ch. 6, ch. 7, ch. 8 and ch. 9 whose center frequencies are indicated in Annex 5 are used for 4-channel QPSK modulated signal transmission. The resolution bandwidth of 3 MHz is used for spectrum measurement. The output power per channel Po is estimated using the following equation:

Po    Pm + 10 log (OBW/RBW)	(1)

where:

Pm:	measured transmitting power (dBm);

OBW:	occupied bandwidth (MHz);

RBW:	resolution bandwidth (MHz).

Since the measured transmitting power of each channel is about –10.5 dBm, –12 dBm, –12 dBm and –12.5 dBm, the output power of each channel estimated to be 9.7 dBm, 8.2 dBm, 8.2 dBm and 7.7 dBm which are lower than the specification shown in Table 6 due to excess losses between the transmitter and the antenna. Although no spectrum mask of ch. 4 or ch. 5 is measured, the in-band emission level of ch. 4 and ch. 5 at the band of 94-94.1 GHz could be estimated from the measured data of the bands of 96.7-96.8 GHz and 94.4-94.5 GHz. They are about –38 dBm/3 MHz and 
–41 dBm/3 MHz, respectively. Since the bandwidth of the satellite (CPR-L1) is 0.36 MHz, the in-band emission power is estimated to be –47 dBm/0.36 MHz and –51 dBm/0.36 MHz, respectively. The total in-band emission power could be –45.5 dBm/0.36 MHz.

The out-of-band emission level of 100-GHz RSTT is also estimated from the measured spectrum in Figure 10. It could be about –40 dBm/3 MHz at 92 GHz and then –45 dBm/3 MHz at 89.5 GHz. The level of –45 dBm is the limit value of the measurement system, but this value may provide the worst-case power level of the out-of-band emission. Since the bandwidth of the satellite (L8) is 3 GHz, the total out-of-band emission power in the band of 3 GHz is estimated to be –12.7 dBm.

FIGURE 8

Channel arrangement of 100-GHz RSTT

FIGURE 9

Channel aggregation pattern

FIGURE 10

Spectrum mask of QPSK modulated 4 channels bonding pattern whose channel number is no. 6 (95.1 GHz), 
no. 7 (95.5 GHz), no. 8 (95.9 GHz) and no. 9 (96.3 GHz) (see Table A4-1)

## 7.2	System characteristics of earth exploration-satellite service (passive) operating in the frequency ranges 86-92 GHz, 100-102 GHz and 109.5-111.8 GHz

Technical characteristics of spaceborne radiometers in the frequency band of 86-92 GHz are provided in Recommendation ITU-R RS.1861 – Typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz. The technical characteristics for one spaceborne radiometer system L8 is presented below in Table 7.

Performance requirements for spaceborne passive sensors in the EESS (passive) are provided by Recommendation ITU-R RS.2017 – Performance and interference criteria for satellite passive remote sensing. The performance criteria for the EESS (passive) radiometers specifies a temperature differential, Te of 0.05 K and data availability criteria of at least 99.99%.

TABLE 7

Characteristics of EESS (passive) missions in the 86-92 GHz band

| Parameter | L8 |
| --- | --- |
| Sensor type | Conical scan radiometer |
| Type of orbit | SSO |
| Altitude, km | 700 |
| Inclination, deg | 98.2 |
| Repeat period, days | 16 |
| Antenna type | Parabolic reflector antenna |
| Antenna diameter | 2.0 m |
| Antenna (receive) peak gain, dBi | 62.4 |
| Polarization | Linear H,V |
| Incidence angle at Earth, deg | 55 |
| Azimuth scan rate, rpm | 40 |
| Antenna beam look angle, deg | 47.5 |
| Antenna beam azimuth angle, deg | 0 to 360 |
| Antenna elev. beamwidth, deg | 0.15 |
| Antenna az. beamwidth, deg | 0.15 |
| Beam width, deg | 0.15 |
| RF Center Frequency, GHz | 89.0 |
| RF bandwidth, MHz | 3 000 |
| Horizontal resolution | 2.9 km |
| Vertical resolution | 5.1 km |

## 7.3	System characteristics of earth exploration-satellite service (active) operating in the frequency range 94-94.1 GHz

Technical characteristics of cloud profile radars (CPRs) in the frequency band of 94.0-94.1 GHz are given in Recommendation ITU-R RS.2105 – Typical technical and operational characteristics of Earth exploration-satellite service (active) systems using allocations between 432 MHz and 238 GHz. The technical characteristics for two CPR systems CPR-L1 and CPR-L2 are presented below in Table 8.

Performance requirements for spaceborne active sensors in the EESS (active) are given in Recommendation ITU-R RS.1166-4 – Performance and interference criteria for active spaceborne sensors. The performance criteria for the EESS (active) cloud profile radars requires a measurement of a minimum reflectivity of –30 dBz 10%, I/N no greater than –10 dB and random data availability criteria of no less than 99.8%.

TABLE 8

Characteristics of EESS (active) missions in the 94-94.1 GHz band

| Parameter | CPR-L1 | CPR-L2 |
| --- | --- | --- |
| Sensor type | Cloud Profiling Radar | Cloud Profiling Radar |
| Type of orbit | SSO | SSO |
| Altitude, km | 705 | 393 |
| Inclination, deg | 98.2 | 97 |
| Ascending Node LST | 13:30 | 10:30 |
| Repeat period, days | 16 | 25 |
| Antenna type | Parabolic reflector to Offset cassegrain antenna | Parabolic reflector |
| Antenna diameter | 1.85-2.5 m | 2.5 m |
| Antenna (transmit and receive) peak gain, dBi | 63.1-65.2 | 65.2 |
| Polarization | linear | LHC, RHC |
| Incidence angle at Earth, deg | 0 | 0 |
| Azimuth scan rate, rpm | 0 | 0 |
| Antenna beam look angle, deg | 0 | 0 |
| Antenna beam azimuth angle, deg | 0 | 0 |
| Antenna elev. beamwidth, deg | 0.12 | 0.095 |
| Antenna az. beamwidth, deg | 0.12 | 0.095 |
| Beam width, deg | 0.095-0.108 | 0.095 |
| RF Center Frequency, MHz | 94.050 | 94.050 |
| RF bandwidth, MHz | 0.36 | 7 |
| Transmit Pk pwr, W | 1 000 | 1 430 |
| Transmit Ave. pwr, W | 21.31 | 28.8 |
| Pulsewidth, μsec | 3.33 | 3.3 |
| Pulse Repetition Frequency (PRF), Hz | 4 300 | 6 100-7 500 |
| Chirp rate, MHz/μsec | N/A | 2.1 |
| Transmit duty cycle, % | 1.33 | 2.01 |
| Minimum sensitivity, dBz | –30 to –35 | –30 to –35 |
| Horizontal resolution | 0.7-1.9 km | 800 m |
| Vertical resolution | 250-500 m | 500 m |
| Doppler range | ±10 m/s | –10 ~ +10 m/s |
| Doppler accuracy | 1 m/s | 1 m/s |
| System Noise Figure, dB | 7 | 7 |

## 7.4	System characteristics of radio astronomy service operating in the frequency range 86-111.8 GHz

# 8	Interference scenarios

The four interference scenarios listed in Table 9 and shown in Figure 11 are considered between land mobile service applications (RSTT) and passive services.

TABLE 9

Interference scenarios

| Scenario | Interfering | Interfered with | Propagation model |
| --- | --- | --- | --- |
| A-1 | RSTT on-board terminal | EESS space station | Free space |
| A-2 | RSTT trackside station | EESS space station | Free space |
| B-1 | RSTT on-board terminal | EESS space station | Free space |
| B-1 | EESS space station | RSTT on-board terminal | Free space |
| B-2 | RSTT trackside station | EESS space station | Free space |
| B-2 | EESS space station | RSTT trackside station | Free space |
| C-1 | RSTT on-board terminal | RAS earth station | P.452-16 |
| C-2 | RSTT trackside station | RAS earth station | P.452-16 |

FIGURE 11

Illustration of interference scenario

# 9	Sharing and compatibility studies

## 9.1	Compatibility studies for earth exploration-satellite service (passive)

Interference requirements for spaceborne passive sensors in the EESS (passive) are given in Recommendation ITU-R RS.2017 – Performance and interference criteria for satellite passive remote sensing. The interference threshold criteria for the EESS (passive) radiometer in the 86-92 GHz frequency band is –169 dBW with a percentage of area or time permissible interference level may be exceeded of 0.01%. The study results are included in Annex 3.

## 9.2	 Compatibility studies for earth exploration-satellite service (active)

Interference criteria for spaceborne active sensors in the EESS (active) are provided by Recommendation ITU-R RS.1166-4 – Performance and interference criteria for active spaceborne sensors. The interference threshold criteria for the EESS (active) Cloud Profile Radars in the 94.0-94.1 GHz frequency band is –155 dBW over 300 kHz. The study results are included in Annex 2.

## 9.3	Sharing studies for radio astronomy service

Protection criteria for radio astronomy observations are provided by Recommendation ITU-R RA.769-2. The initial study based on the first geometry shows that the received power level at RAS antenna may exceed the threshold level in the condition of line-of-sight transmission path and its distance. The study results are shown in Annex 4.

# 10	Bibliography

Annex 1

Measurement results of radiation pattern 
in the frequency range 92-100 GHz

This Annex provides various antenna radiation pattern for 100 GHz RSTT.

FIGURE A1-1

Measured radiation pattern of 44-dBi antenna

FIGURE A1-2

Measured radiation pattern of 42-dBi antenna

FIGURE A1-3

Measured radiation pattern of 35-dBi antenna

Figure A1-4

External view of trackside RAU

ANNEX 2

Analysis of potential OOB interference to a spaceborne cloud profile radar in the 94.0-94.1 GHz band from RSTT systems in adjacent bands

# 1	Introduction

The 94.0-94.1 GHz band is allocated to Earth Exploration-Satellite Service (EESS) (active) on a primary basis for EESS (active) spaceborne active sensors, such as cloud profile radars (CPRs). Herein are presented technical characteristics of two typical EESS (active) cloud profile radar systems operating in the frequency band 94.0-94.1 GHz and technical characteristics for typical Railway Radiocommunication Systems between Train and Trackside (RSTT) stations proposed for adjacent bands 92.0-94.0 GHz and 94.1-96.0 GHz. Performance criteria and interference criteria are provided for active spaceborne cloud profile radar operating in the 94.0-94.1 GHz frequency band. Preliminary calculations are performed to determine the amount of attenuation needed to be applied to railway RSTT systems out-of-band (OOB) emission levels in relation to the RSTT in-band emission levels in order for the out-of-band RSTT emissions to not exceed the EESS (active) interference protection criteria levels.

# 2	Interference from railway RSTT systems into EESS (active)

For assessing the potential for interference from the railway RSTT systems into EESS active systems, three different geometrical scenarios are considered. The first is coupling of the antenna main lobe of a nadir-looking EESS (active) satellite with the sidelobes of the railway RSTT system antenna. The second geometrical scenario is coupling of the sidelobes of both the EESS active sensor antenna and the railway RSTT system antenna; and the third geometrical scenario is coupling between the mainbeam of the railway RSTT system antenna and the EESS active sensor antenna sidelobes occurring when the EESS active satellite is on the horizon with respect to the railway RSTT system.

The peak interfering signal power level, I (dBW), received by a spaceborne radar from a terrestrial source is calculated from

I    10 log Pt    Gt    Gr  –  (32.44    20 log ( f R ))  –  La	(1)

where:

Pt:	peak terrestrial source transmitter power (W);

Gt:	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr:	spaceborne radar antenna gain towards terrestrial source (dBi);

f:	frequency (MHz);

R:	slant range between spaceborne sensor and terrestrial source (km);

La:	attenuation due to atmospheric absorption (dB).

Attenuation due to atmospheric absorption, La, is dependent upon the path length to the satellite through the Earth’s atmosphere, and hence upon the elevation angle from the terrestrial source to the satellite. At frequencies around 94 GHz, La decreases rapidly from about 100 dB at 0º elevation angles to 1.5 dB at 90º elevation angles. La is not included in the following tables of calculation of the interference levels and margins.

The interference due to mainlobe reception of the CPR antenna from the RSTT sidelobes allows for the highest values of RFI levels of the three geometrical scenarios. For CPR-L1 and CPR-L2, the CPR-L1 system is impacted the most by interference due to its narrower receiver bandwidth of 300 kHz. For geometric scenario 1 for coupling between the CPR-L1 mainlobe and the railway RSTT sidelobes (elevation angle at 90 º), preliminary calculations are provided in Tables 3 (Case 1) and 4 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system needed to be applied to railway RSTT systems OOB emissions in order for them to meet the EESS (active) interference protection criteria. Considering CPR-L1, two cases were analyzed where the railway RSTT antenna gain in the sidelobes was –10 dBi and 14 dBi, and the density of railway RSTT systems was 10 and 100 in the CPR footprint. The EESS active noise figure used is 7 dB which results in the interference protection criteria threshold of -153.2 dBW. The calculated attenuations in regards to the in-band power emission level of the railway RSTT system needed to meet the EESS (active) interference protection criteria are 9.5 dB in Case 1 to 43.5 dB in Case 2.

For geometric scenario 2 for coupling between the CPR-L1 sidelobes and the railway RSTT sidelobes (elevation angle at 45º), preliminary calculations of two Cases examined are provided in Tables 5 (Case 1) and 6 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system emissions in order that they meet the EESS (active) interference protection criteria for CPR-L1. The two cases were examined with the railway RSTT system antenna gain in the sidelobes at 0 dBi and 14 dBi, and the density of railway RSTT systems set at 10 and 100 in the CPR footprint, respectively. The EESS active system noise figure was 7 dB which resulted in the interference protection criteria threshold of –153.2 dBW. The calculations show that the EESS (active) interference criteria are met with margins of 58.1 dB for Case 1 and 34.1 dB for Case 2.

For geometric scenario 3 for coupling between the CPR-L1 sidelobes and the railway RSTT mainlobe (elevation angle at 0º), preliminary calculations of two Cases examined are provided in Tables 7 (Case 1) and 8 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system emissions in order that they meet the EESS (active) interference protection criteria for CPR-L1. The two cases were examined with the railway RSTT system antenna gain in the sidelobes at 44 dBi, and the density of railway RSTT systems set at 10 and 100 in the CPR footprint, respectively. The EESS active system noise figure was 7 dB which resulted in the interference protection criteria threshold of –153.2 dBW. The calculations show that the EESS (active) interference criteria are met with margins of 24.3 dB for Case 1 and 14.3 dB for Case 2.

Table A2-1

RFI from railway RSTT into EESS (active) at 94 GHz (Case 1)
First Geometric Scenario: CPR M/L to RSTT S/L

| Case 1: Calculation of rcv pwr at 90 deg elev (G_t = -10 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power,W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | –10.00 |
| e.i.r.p., dBW |  | –30.00 |
| Gain_rcv, dBi |  | 65.20 |
| 1/R2, km | 705 | –56.96 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –188.87 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –143.67 |
| k | 1.38E-23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | –0.50 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | –9.50 |

Table A2-2

RFI from railway RSTT into EESS (active) at 94 GHz (Case 2)
First Geometric Scenario: CPR M/L to RSTT S/L

| Case 2: Calculation of rcv pwr at 90 deg elev (G_t = 14 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR–L1 |  |
|  | Value | dB |
| Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 14.00 |
| e.i.r.p., dBW |  | –6.00 |
| Gain_rcv, dBi |  | 65.20 |
| 1/R2, km | 705 | –56.96 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –188.87 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –109.67 |
| k | 1.38E–23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | 33.50 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | –43.50 |

Table A2-3

RFI from railway RSTT into EESS (active) at 94 GHz (Case 1)
Second Geometric Scenario: CPR S/L to RSTT S/L

| Case 1: Calculation of rcv pwr at 45 deg elev (G_t = 0 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 0.00 |
| e.i.r.p., dBW |  | –20.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 952 | –59.57 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –191.48 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –211.28 |
| k | 1.38E-23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | –68.11 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | 58.11 |

Table A2-4

RFI from railway RSTT into EESS (active) at 94 GHz (Case 2)
Second Geometric Scenario: CPR S/L to RSTT S/L

| Case 2: Calculation of rcv pwr at 45 deg elev (G_t=14 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 14.00 |
| e.i.r.p., dBW |  | –6.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 952 | –59.57 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –191.48 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –187.28 |
| k | 1.38E-23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | –44.11 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | 34.11 |

Table A2-5

RFI from railway RSTT into EESS (active) at 94 GHz (Case 1)
Third Geometric Scenario: CPR S/L to RSTT M/L

| Case 1: Calculation of rcv pwr at 0 deg elev (G_t = 44 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 44.00 |
| e.i.r.p., dBW |  | 24.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 3 081 | –69.77 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –201.68 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –177.48 |
| k | 1.38E-23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | –34.31 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | 24.31 |

Table A2-6

RFI from railway RSTT into EESS (active) at 94 GHz (Case 2)
Third Geometric Scenario: CPR S/L to RSTT M/L

| Case 2: Calculation of rcv pwr at 0 deg elev (G_t = 44 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| Transmit power,W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 44.00 |
| e.i.r.p., dBW |  | 24.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 3 081 | –69.77 |
| 1/f2, MHz | 94 050 | –99.47 |
| L-prop, dB |  | –201.68 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –167.48 |
| k | 1.38E-23 | –228.60 |
| Temp, K | 290 | 24.62 |
| BW_EESS, MHz | 0.3 | 54.77 |
| NF_EESS, dB |  | 7.00 |
| Noise pwr, dBW |  | –143.17 |
| I/N, dB |  | –24.31 |
| I/N criteria, dB |  | –10.00 |
| Margin, dB (attenuation) |  | 14.31 |

# 3	Preliminary conclusions

For the three different geometrical interaction scenarios described involving antenna beam coupling between railway RSTT systems and CPRs, the RFI levels at the CPR are highest for the geometrical situation of coupling between the nadir-looking CPR antenna and the sidelobes of the railway RSTT system antenna. With the second geometrical scenario where there is coupling of the sidelobes of both the CPR antenna and the railway RSTT system antenna; the RFI levels are lower than for the first geometric scenario. In the third geometrical scenario where coupling between the mainbeam of the railway RSTT system antenna and the CPR antenna sidelobes with the CPR satellite on the horizon with respect to the railway RSTT systems occurs, the interference was lower than the first geometric scenario as well. The CPR-L1 with the narrower receiver bandwidth of 300 kHz, is more sensitive than is CPR-L2. Two cases of railway RSTT deployment and their interference impact to CPR-L1 were analysed to arrive at a preliminary calculation of attenuation in regards to in-band emission levels of railway RSTT systems required to meet the Rec. ITU-R RS.1166-4 protection criteria for CPR-L1. The two cases consider the railway RSTT antenna gain in the sidelobes at the level in Figure 2 for the appropriate elevation angle and a higher level estimated due to irregularities in the trainside area, and the density of railway RSTT systems at 10 and 100 in the footprint of the CPR-L1 sensor. For the first geometric scenario, in order to meet the EESS (active) Rec. ITU-R RS.1166-4 protection criteria for CPR-L1, railway RSTT system in-band emission levels would have to be attenuated 9.5 dB when considering Case 1 and 43.5 dB when considering Case 2 at the band edges adjoining the EESS (active) band 94.0-94.1 GHz.

ANNEX 3

Analysis of potential OOB interference to a spaceborne radiometer in the 86.0-92.0 GHz band from RSTT systems in adjacent bands

# 1	Introduction

The 86.0-92.0 GHz band is allocated to Earth Exploration-Satellite Service (EESS) (passive) on a primary basis for EESS (passive) spaceborne passive sensors, such as spaceborne radiometers. Herein are presented technical characteristics of one typical EESS (passive) radiometer system operating in the frequency band 86.0-92.0 GHz and technical characteristics for typical Railway Radiocommunication Systems between Train and Trackside (RSTT) stations proposed for adjacent band 92.0-94.0 GHz. Performance criteria and interference criteria are provided for passive spaceborne radiometers operating in the 86.0-92.0 GHz frequency band. Preliminary calculations are performed to determine the amount of attenuation needed to be applied to railway RSTT systems out-of-band (OOB) emission levels in relation to the RSTT in-band emission levels in order for the out-of-band RSTT emissions to not exceed the EESS (passive) interference protection criteria levels.

The compatibility study is performed in response to World Radio Conference (WRC) 2019 agenda item 1.11, Resolution 236 (WRC-15), which addresses the need for harmonized frequency bands for use by railway RSTT.

# 2	Interference from railway RSTT systems into EESS (passive)

For assessing the potential for interference from the railway RSTT systems into EESS passive systems, three different geometrical scenarios are considered. The first is coupling of the antenna main lobe of a conically scanning antenna of an EESS (passive) satellite with the sidelobes of the railway RSTT system antenna. The second geometrical scenario is coupling of the sidelobes of both the EESS passive sensor antenna and the railway RSTT system antenna; and the third geometrical scenario is coupling between the mainbeam of the railway RSTT system antenna and the EESS passive sensor antenna sidelobes occurring when the EESS passive satellite is on the horizon with respect to the railway RSTT system.

The peak interfering signal power level, I (dBW), received by a spaceborne radiometer from a terrestrial source is calculated from

I    10 log Pt    Gt    Gr  –  (32.44    20 log ( f R ))  –  La	(1)

where:

Pt :	peak terrestrial source transmitter power (W);

Gt :	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr :	spaceborne radiometer antenna gain towards terrestrial source (dBi);

f :	frequency (MHz);

R :	slant range between spaceborne sensor and terrestrial source (km);

La :	attenuation due to atmospheric absorption (dB).

Attenuation due to atmospheric absorption, La, is dependent upon the path length to the satellite through the Earth’s atmosphere, and hence upon the elevation angle from the terrestrial source to the satellite. At frequencies around 92 GHz, La decreases rapidly from about 100 dB at 0º elevation angles to 1.5 dB at 90º elevation angles. La is not included in the following tables of calculation of the interference levels and margins.

The interference due to mainlobe reception of the radiometer L-8 antenna from the RSTT sidelobes allows for the highest values of RFI levels of the three geometrical scenarios. For geometric scenario 1 for coupling between the radiometer L-8 mainlobe and the railway RSTT sidelobes (elevation angle at 55 º), preliminary calculations are provided in Tables A3-1 (Case 1) and A3-2 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system needed to be applied to railway RSTT systems OOB emissions in order for them to meet the EESS (passive) interference protection criteria. Considering radiometer L-8, two cases were analyzed where the railway RSTT antenna gain in the sidelobes was 0 dBi and 14 dBi, and the density of railway RSTT systems was 10 and 100 in the CPR footprint. The EESS passive interference protection criteria threshold is –169.0 dBW. The calculated attenuations in regards to the in-band power emission level of the railway RSTT system needed to meet the EESS (passive) interference protection criteria are 28.7 dB in Case 1 to 52.7 dB in Case 2.

For geometric scenario 2 for coupling between the radiometer L-8 sidelobes and the railway RSTT sidelobes (elevation angle at 90º), preliminary calculations of two Cases examined are provided in Tables A3-3 (Case 1) and A3-4 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system emissions in order that they meet the EESS (passive) interference protection criteria for radiometer L-8. The two cases were examined with the railway RSTT system antenna gain in the sidelobes at –10 dBi and 14 dBi, and the density of railway RSTT systems set at 10 and 100 in the CPR footprint, respectively. The EESS passive interference protection criteria threshold is -169.0 dBW. The calculations show that the EESS (passive) interference criteria are met with margins of 49.4 dB for Case 1 and 15.4 dB for Case 2.

For geometric scenario 3 for coupling between the radiometer L-8 sidelobes and the railway RSTT mainlobe (elevation angle at 0º), preliminary calculations of two Cases examined are provided in Tables A3-5 (Case 1) and A3-6 (Case 2) which indicate the amount of attenuation in regards to the in-band power emission level of the railway RSTT system emissions in order that they meet the EESS (passive) interference protection criteria for radiometer L-8. The two cases were examined with the railway RSTT system antenna gain in the sidelobes at 44 dBi, and the density of railway RSTT systems set at 10 and 100 in the CPR footprint, respectively. The EESS passive interference protection criteria threshold is –169.0 dBW. The calculations show that the EESS (passive) interference criteria are met with margin of 8.3 dB for Case 1 but the calculated attenuation in regards to the in-band power emission level of the railway RSTT system needed to meet the EESS (passive) interference protection criteria is 1.7 dB for Case 2.

Table A3-1

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 1)
First Geometric Scenario: L-8 M/L to RSTT S/L

| Case 1: Calculation of rcv pwr at 55 deg elev (G_t = -10 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power,W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 0.00 |
| e.i.r.p., dBW |  | –20.00 |
| Gain_rcv, dBi |  | 62.40 |
| 1/R2, km | 1 114.9 | –60.94 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –192.66 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –140.26 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | –28.74 |

Table A3-2

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 2)
First Geometric Scenario: L-8 M/L to RSTT S/L

| Case 2: Calculation of rcv pwr at 55 deg elev (G_t = 14 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| Transmit power,W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 14.00 |
| e.i.r.p., dBW |  | –6.00 |
| Gain_rcv, dBi |  | 62.40 |
| 1/R2, km | 1 114.9 | –60.94 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –192.66 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –116.26 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | –52.74 |

Table A3-3

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 1)
Second Geometric Scenario: L-8 S/L to RSTT S/L

| Case 1: Calculation of rcv pwr at 90 deg elev (G_t = 0 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power,W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | –10.00 |
| e.i.r.p., dBW |  | –30.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 700 | –56.90 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –188.62 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –218.42 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | 49.42 |

Table A3-4

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 2)
Second Geometric Scenario: L-8 S/L to RSTT S/L

| Case 2: Calculation of rcv pwr at 90 deg elev (G_t = 14 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 14.00 |
| e.i.r.p., dBW |  | –6.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 700 | –56.90 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –188.62 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –184.42 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | 15.42 |

Table A3-5

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 1)
Third Geometric Scenario: L-8 S/L to RSTT M/L

| Case 1: Calculation of rcv pwr at 0 deg elev (G_t = 44 dBi, 10 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| RSTT Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 44.00 |
| e.i.r.p., dBW |  | 24.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 3 069 | –69.74 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –201.46 |
| No. of RSTTs in beam | 10 | 10.00 |
| Interf pwr, dBW |  | –177.26 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | 8.26 |

Table A3-6

RFI from railway RSTT into EESS (passive) at 92 GHz (Case 2)
Third Geometric Scenario: L-8 S/L to RSTT M/L

| Case 2: Calculation of rcv pwr at 0 deg elev (G_t = 44 dBi, 100 railway RSTTs in beam) |  |  |
| --- | --- | --- |
|  | CPR-L1 |  |
|  | Value | dB |
| Transmit power, W | 0.01 | –20.00 |
| Gain_xmit, dBi |  | 44.00 |
| e.i.r.p., dBW |  | 24.00 |
| Gain_rcv, dBi |  | –9.80 |
| 1/R2, km | 3 069 | –69.74 |
| 1/f2, MHz | 92 000 | –99.28 |
| L-prop, dB |  | –201.46 |
| No. of RSTTs in beam | 100 | 20.00 |
| Interf pwr, dBW |  | –167.26 |
| Interf pwr criteria, dBW |  | –169.00 |
| Margin, dB |  | –1.74 |

# 4	Preliminary conclusions

For the three different geometrical interaction scenarios described involving antenna beam coupling between railway RSTT systems and spaceborne radiometers, the RFI levels at the radiometer L-8 are highest for the geometrical situation of coupling between the mainlobe of the radiometer L-8 antenna and the sidelobes of the railway RSTT system antenna. With the second geometrical scenario where there is coupling of the sidelobes of both the radiometer L-8 antenna and the railway RSTT system antenna; the RFI levels are lower than for the first geometric scenario. In the third geometrical scenario where coupling between the mainbeam of the railway RSTT system antenna and the radiometer L-8 antenna sidelobes with the radiometer L-8 satellite on the horizon with respect to the railway RSTT systems occurs, the interference was lower than the first geometric scenario as well. Two cases of railway RSTT deployment and their interference impact to radiometer L-8 were analysed to arrive at a preliminary calculation of attenuation in regards to in-band emission levels of railway RSTT systems required to meet the Rec. ITU-R RS.1166-4 protection criteria for CPR-L1. The two cases consider the railway RSTT antenna gain in the sidelobes at the level in Figure 2 for the appropriate elevation angle and a higher level estimated due to irregularities in the trainside area, and the density of railway RSTT systems at 10 and 100 in the footprint of the CPR-L1 sensor. For the first geometric scenario, in order to meet the EESS (passive) Rec. ITU-R RS.2017 protection criteria for spaceborne radiometers, railway RSTT system in-band emission levels would have to be attenuated 28.7 dB when considering Case 1 and 52.7 dB when considering Case 2 at the band edges adjoining the EESS (passive) band 86.0-92.0 GHz.

ANNEX 4

Sharing studies between RAS and RSTT systems
in the frequency range 92-109.5 GHz

The protection criteria for radio astronomical observations are given in Recommendations ITU-R RA.769 and ITU-R RA.1513. More precisely, an interpolation from the interference threshold levels given for the frequency bands listed in Recommendation ITU-R RA.769 provides the following threshold spectral power flux densities for continuum and spectral line observations in the frequency range 92-109.5 GHz.  According to Recommendation ITU-R RA.1513, interference from any one network should not exceed these thresholds for more than 2% of time.

Table A3-1

Interference threshold level of radio astronomical observations

| Frequency band (GHz) | Threshold interference level – Input power (dBm) |  |
| --- | --- | --- |
|  | Continuum observations (8 GHz bandwidth) | Spectral line observations (1 MHz bandwidth) |
| 92-109.5 | –159 | –179 |

The bands 92-94 GHz, 94.1-100 GHz and 102-109.5 GHz are allocated on an equal primary basis to the mobile service and radio astronomy service including other radiocommunication services in all three Regions. The protection criterion used is derived from Recommendation ITU-R RA.769-2. The received power level at the radiometer is calculated by the following equation:

P769 ＝ Pt + G – Loss – J(ν)

＝ Pt + G – (92.5 + 20 * log(f) + 20 * log(d) + Ag) – J(ν)

where:

Pt:	transmission power of on-board equipment;

G:	antenna gain;

d:	separation distance;

Loss:	propagation loss given by Recommendation ITU-R P.452-16;

J(ν):	knife-edge diffraction loss given by Recommendation ITU-R P.452-16.

Ag = (o+w())d

where:

Ag:	total gaseous absorption (dB);

o+w():	specific attenuation due to dry air and water vapour, respectively, and are found from the equations in Recommendation ITU-R P.676;

:	water vapour density:

 = 7.5 + 2.5     g/m3

:	fraction of the total path over water.

The separation distance which satisfies with the requirement of protection level is calculated from the above equation. The line-of-sight scenario from the on-board equipment to the radio astronomy antenna gives the worst case. Figure A3-1 shows the topographic profile between Nobeyama Radio Observatory and Kokubunji where NICT is located. The distance is 87.8 km, and the altitudes of each location are about 1 350 m and 95 m, respectively. As the first calculation example, the single knife-edge diffraction loss caused by the highest mountain (2,418 m) is calculated and its value of 64.5 dB is obtained. Table A3-2 summarizes the initial result of received power level at Nobeyama Radio Observatory. For the first geometric study, the received power at RAS site is 0.1-dB larger and 64-dB lower than the interference threshold level in line-of-sight and non-line-of-sight propagation paths, respectively. Further studies are required to evaluate the separation distance to protect RAS receivers, considering the current geometric deployment scenario of RSTT systems.

Figure A3-1

Topographic profile between Nobeyama Radio Observatory and Kokubunji

Table A3-2

Received power level at Nobeyama Radio Observatory

| Parameters | Value | Note |
| --- | --- | --- |
| RSTT transmitting power/channel (mW) | 10 |  |
| RSTT antenna gain (dB) | 42 |  |
| Centre frequency (GHz) | 92.5 |  |
| Distance between RSTT transmitter and RAS antenna (km) | 87.8 |  |
| Gaseous absorption (dB | 34.2 |  |
| Knife-edge diffraction loss (dB) | 64.5 | Non-line-of-sight |
| Channel bandwidth (MHz) | 400 |  |
| Received power reduction by 1 MHz | –26 |  |
| Received power level at RAS site (dBm) | –243.4 | Non-line-of-sight |
| Received power level at RAS site (dBm) | –178.9 | Line-of-sight |

ANNEX 5

Frequency arrangement of 100-GHz RSTT

TABLE A4-1

List of center frequencies and bandwidth of each channel, and each guard band frequencies

| Channel number | Center frequency (GHz) | Bandwidth (MHz) | Remark |
| --- | --- | --- | --- |
|  |  |  | Guard band: 92.0-92.2 GHz |
| 1 | 92.4 | 400 |  |
| 2 | 92.8 | 400 |  |
| 3 | 93.2 | 400 |  |
| 4 | 93.6 | 400 |  |
|  |  |  | Guard band: 93.8-94.0 GHz, 94.1-94.5 GHz |
| 5 | 94.7 | 400 |  |
| 6 | 95.1 | 400 |  |
| 7 | 95.5 | 400 |  |
| 8 | 95.9 | 400 |  |
| 9 | 96.3 | 400 |  |
| 10 | 96.7 | 400 |  |
| 11 | 97.1 | 400 |  |
| 12 | 97.5 | 400 |  |
| 13 | 97.9 | 400 |  |
| 14 | 98.3 | 400 |  |
| 15 | 98.7 | 400 |  |
| 16 | 99.1 | 400 |  |
| 17 | 99.5 | 400 |  |
|  |  |  | Guard band: 99.7-100.0 GHz, 102.0-102.3 GHz |
| 18 | 102.5 | 400 |  |
| 19 | 102.9 | 400 |  |
| 20 | 103.3 | 400 |  |
| 21 | 103.7 | 400 |  |
| 22 | 104.1 | 400 |  |
| 23 | 104.5 | 400 |  |
| 24 | 104.9 | 400 |  |
| 25 | 105.3 | 400 |  |
| 26 | 105.7 | 400 |  |
| 27 | 106.1 | 400 |  |
| 28 | 106.5 | 400 |  |
| 29 | 106.9 | 400 |  |
| 30 | 107.3 | 400 |  |
| 31 | 107.7 | 400 |  |
| 32 | 108.1 | 400 |  |
| 33 | 108.5 | 400 |  |
| 34 | 108.9 | 400 |  |
|  |  |  | Guard band: 109.1-109.5 GHz |

____________________

Descending.

The sensor uses an unmodulated pulse.