Part 2-1

Description template of the proposed candidate technology for IMT-2020

5.2.3.1	Description template background

This description template is a template for the description of the characteristics of the proposed RIT. It is to describe our proposal for a radio interface for IMT-2020 to a level of detail that will facilitate a sufficient understanding of the proposed technology in order to enable an independent technical assessment of compliance with the IMT-2020 requirements as specified in the Report ITU-R M.2410.

The proposed RIT refers to the latest 3GPP New Radio Technical Specifications.

5.2.3.2	Description template – characteristics template

| Item | Item to be described |
| --- | --- |
| 5.2.3.2.1 | Test environment(s) |
| 5.2.3.2.1.1 | What test environments (described in Report ITU-R M.2412-0) does this technology description template address? This proposal targets to addresses all the five test environments across the three usage scenarios (eMBB, mMTC, and URLLC) as described in Report ITU-R M.2412-0. |
| 5.2.3.2.2 | Radio interface functional aspects |
| 5.2.3.2.2.1 | Multiple access schemes Which access scheme(s) does the proposal use? Describe in detail the multiple access schemes employed with their main parameters. Multiple access based on a combination of - OFDMA: Transmission to/from different UEs using mutually orthogonal frequency assignments. Multiple OFDM numerologies are supported. Granularity in frequency assignment: One resource block consisting of twelve subcarriers over one OFDM symbol. DFT-spread OFDM is also used in uplink. - TDMA: Transmission to/from different UEs with separation in time. Granularity: 14-symbol duration for slot-based operation, less than slot duration with 2, 4, 7 symbol-duration granularities for non-slot based operation, regardless of OFDM numerology. - CDMA: Inter-cell interference suppressed by processing gain of channel coding and/or spreading allowing for a frequency reuse of one. - SDMA: Possibility to transmit to/from multiple users using the same time/frequency resource (SDMA or multi-user MIMO) as part of the advanced-antenna capabilities. |
| 5.2.3.2.2.2 | Modulation scheme |
| 5.2.3.2.2.2.1 | What is the baseband modulation scheme? If both data modulation and spreading modulation are required, describe in detail. Describe the modulation scheme employed for data and control information. What is the symbol rate after modulation? Data and higher-layer control: π2-BPSK (uplink only when transform precoding enabled), QPSK, 16QAM, 64QAM, 256QAM L1/L2 control: π2-BPSK, BPSK and QPSK for uplink control channel (PUCCH), QPSK for downlink control channel (PDCCH) Symbol rate: 168 and 144 symbols per resource block per slot for normal and extended CP, respectively. Symbol rate per resource block per slot depends on sub-carrier spacing. - For 15 kHz sub-carrier spacing and normal CP: 168 103 symbols per second - For 30 kHz sub-carrier spacing and normal CP: 336 103 symbols per second - For 60 kHz sub-carrier spacing and normal CP: 672 103 symbols per second - For 120 kHz sub-carrier spacing and normal CP: 1344 103 symbols per second - For 240 kHz sub-carrier spacing and normal CP: 2688 103 symbols per second |
| 5.2.3.2.2.2.2 | PAPR What is the RF peak to average power ratio after baseband filtering (dB)? Describe the PAPR (peak-to-average power ratio) reduction algorithms if they are used in the proposed RIT/SRIT. Uplink and downlink when OFDMA is used: PAPR = 8.4 dB (99.9 %) Uplink when DFT-s-OFDM is used: Peak-to-Average Power Ratio (PAPR) and Cubic Metric (CM) depends on modulation scheme and number of component carrier. The CM is a method of predicting the power de-rating from signal modulation characteristics and is found to be more useful than other metrics such as PAPR - π2-BPSK: PAPR = 4.5 dB (99.9 %), CM = 0.3 dB - QPSK: PAPR = 5.8 dB (99.9 %), CM = 1.2 dB - 16QAM: PAPR = 6.5 dB (99.9 %), CM = 2.1 dB - 64QAM: PAPR = 6.6 dB (99.9 %), CM = 2.3 dB - 256QAM: PAPR = 6.7 dB (99.9 %), CM = 2.4 dB Any PAPR-reduction algorithm is transmitter-implementation specific for uplink and downlink. |
| 5.2.3.2.2.3 | Error control coding scheme and interleaving |
| 5.2.3.2.2.3.1 | Provide details of error control coding scheme for both downlink and uplink. For example, – FEC or other schemes? The proponents can provide additional information on the decoding schemes. Error control coding is performed on the transport channels and control information. - For uplink and downlink data except PBCH: LDPC - For downlink PBCH: Polar - For downlink control (DCI): Polar - For uplink control (UCI with payload size 12): Polar - For uplink control (UCI with 3 payload size 11): Reed-Muller - For uplink control (UCI with payload size 2): Simplex code - For uplink control (UCI with payload size 1): Repetition For more details, see TTAT.3G-38.212. Decoding mechanism is implementation specific. |
| 5.2.3.2.2.3.2 | Describe the bit interleaving scheme for both uplink and downlink. According to the section 5.4.2 of TTAT.3G-38.212 (Rate matching for LDPC code), bit interleaving is performed after LDPC encoding for both uplink and downlink. According to the section 5.4.1 of TTAT.3G-38.212 (Rate matching for Polar code), rate matching for Polar code consists of sub-block interleaving, bit collection, and bit interleaving. For more details, see TTAT.3G-38.212 clause 5.4. |
| 5.2.3.2.3 | Describe channel tracking capabilities (e.g. channel tracking algorithm, pilot symbol configuration, etc.) to accommodate rapidly changing delay spread profile. To support channel tracking, different types of reference signals can be transmitted on downlink and uplink respectively. Downlink: - Synchronization signals (SS) consist of primary SS (PSS) and secondary SS (SSS) and can be used for the initial access. PSS and SSS forms a SS block together with physical broadcast channel (PBCH). Each SS block is repeated with a certain periodicity and may correspond to each beam when analog/digital/hybrid beamforming is used. - Channel-state information RS (CSI-RS) can be used for estimation of channel-state information (CSI) to further prepare feedback reporting to base station (CQI for link adaptation, precoder-matrix/vector selection, etc.) to assist beamforming and scheduling. Also, CSI-RS can be used for tracking time/frequency. In this case, UE assumes that CSI-RS burst is quasi co-located with respect to delay spread, average delay, Doppler shift, and Doppler spread with the PDSCH DM-RS. CSI-RS is UE-specifically configurable. CSI-RS can also be used for the measurement for mobility support. For the detailed structure of CSI-RS, see TTAT.3G-38.211 sub-clause 7.4.1.5 - Demodulation RS (DM-RS) can be used for downlink channel estimation for coherent demodulation of Physical Downlink Shared Channel (PDSCH) and PDCCH. At least DM-RS is mapped over 1 or 2 adjacent OFDM symbols which are front-loaded, and the additional DMRS is always present in the prefixed location. DM-RS supports up to 12 ports. For the detailed structure of DM-RS, see TTAT.3G-38.211 sub-clauses 7.4.1.1 and 7.4.1.3. - Phase-tracking RS (PT-RS) is present only in the resource blocks used for the PDSCH, and only if the higher-layer parameter indicates PT-RS being used. For the detailed structure of PT-RS, see TTAT.3G 38.211 sub-clause 7.4.1.2. Uplink: - Sounding RS (SRS) can be used for estimation of uplink CSI to assist uplink scheduling and uplink power control. The transmission of SRS is configurable. For the detailed structure of uplink SRS, see TTAT.3G-38.211 sub-clause 6.4.1.4. - Demodulation RS (DM-RS) can be used for channel estimation for coherent demodulation of the Physical Uplink Shared Channel (PUSCH) and the PUCCH. Uplink DM-RS is mapped to physical resources according to the type given by higher-layer parameter. For the detailed structure of uplink DM-RS for PUSCH and PUCCH, see TTAT.3G-38.211 sub-clauses 6.4.1.1 and 6.4.1.3, respectively. - Phase-tracking RS (PT-RS) is present only in the resource blocks used for the PUSCH, and only if the higher-layer parameter indicates PT-RS being used. For the detailed structure of PT-RS, see TTAT.3G-38.211 sub-clause 6.4.1.2. Channel-tracking/estimation algorithms are receiver-implementation specific. |

| 5.2.3.2.4 | Physical channel structure and multiplexing |
| --- | --- |
| 5.2.3.2.4.1 | What is the physical channel bit rate (M or Gbit/s) for supported bandwidths? i.e., the product of the modulation symbol rate (in symbols per second), bits per modulation symbol, and the number of streams supported by the antenna system. The physical channel bit rate depends on the modulation scheme, subcarrier spacing, number of resource blocks, and number of spatial-multiplexing layers. |
| 5.2.3.2.4.2 | Layer 1 and Layer 2 overhead estimation. Describe how the RIT/SRIT accounts for all layer 1 (PHY) and layer 2 (MAC) overhead and provide an accurate estimate that includes static and dynamic overheads. L1 overhead for downlink considers DMRS, CSI-RS, TRS, PTRS, SSB, and PDCCH. L1 overhead for uplink considers DMRS, SRS, PTRS, RACH, PUCCH, and control signalings on PUSCH. The overhead is set dynamically according to the carrier frequency, subcarrier spacing, bandwidth, and multiplexing, etc. Headers of SADP, PDCP, RLC, and MAC PDUs are also considered as L2 overhead. One MAC PDU may contain one or multiple headers. The sizes of different PDU headers are as follows: - MAC subheader: 2 ~ 3 bytes - RLC subheader: 2 ~ 3 bytes - PDCP subheader: 2 ~ 3 bytes - SDAP subheader: 1 byte L2 overhead would be 7 ~ 10 bytes per transport block assuming one subheader per sublayer. The overhead is increased when the transport block conveys multiple packets or segmented packets. |
| 5.2.3.2.4.3 | Variable bit rate capabilities: Describe how the proposal supports different applications and services with various bit rate requirements. For a given combination of subcarrier spacing, modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled by the scheduler by assigning different number of resource blocks for the transmission. In case of multiple services, the available/assigned resource, and thus the available data rate, is shared between the services. |
| 5.2.3.2.4.4 | Variable payload capabilities: Describe how the RIT/SRIT supports IP-based application layer protocols/services (e.g., VoIP, video-streaming, interactive gaming, etc.) with variable-size payloads. The RIT is designed primarily for IP based services. The variable transport-block size is supported. See TTAT.3G-38.214 sub-clause 5.1.3.2 for details. |
| 5.2.3.2.4.5 | Signalling transmission scheme: Describe how transmission schemes are different for signalling/control from that of user data. Uplink The physical uplink control channel (PUCCH) supports multiple formats as shown in Table 5.2.3.2.4.5-1. Table 5.2.3.2.4.5-1: PUCCH formats PUCCH format; Length in OFDM symbols; Number of bits. 0; 1 – 2; ≤2. 1; 4 – 14; ≤2. 2; 1 – 2; >2. 3; 4 – 14; >2. 4; 4 – 14; >2. Control signalling is limited to π/2-BPSK, BPSK and QPSK modulation. Repetition, simplex, Reed-Muller, and polar coding schemes are supported according to control information. Downlink A control-resource set (CORESET) consists of NRBCORESET resource blocks in the frequency domain, and NsymbolCORESET∈1, 2, 3 symbols in the time domain. A control-channel element (CCE) consists of 6 resource-element groups (REGs) where a REG equals one resource block during one OFDM symbol, which is 12 subcarriers. A physical downlink control channel (PDCCH) consists of one or more CCEs. Control signalling is limited to QPSK modulation and relies on polar coding. Besides PUCCH and PDCCH, higher-layers signalling is carried within transport blocks. |
| 5.2.3.2.4.6 | Small signalling overhead Signalling overhead refers to the radio resource that is required by the signalling divided by the total radio resource which is used to complete a transmission of a packet. The signalling includes necessary messages exchanged in DL and UL directions during a signalling mechanism, and Layer 2 protocol header for the data packet. Describe how the RIT/SRIT supports efficient mechanism to provide small signalling overhead in case of small packet transmissions. In case of small data packet transmission, the L1/L2 control signalling during the connection setup procedure is dominating the uplink and downlink transmissions (e.g. setup of security, setup of SRB1 and DRBs done with different messages). To minimize this overhead NR relies on RRC_INACTIVE state that allows a UE to resume an earlier connection (possibly relying on delta signalling based on stored configurations) that has been suspended. Once a terminal is in RRC connected, dynamically scheduling radio resources consumes DL control resources. To minimize this usage, NR specifies semi-persistent scheduling (SPS) in DL and configured grant (CG) in UL. In both features a terminal is preconfigured with DL (SPS) or UL (CG) data resources and can use them without DL control information scheduling the resources. |
| 5.2.3.2.5 | Mobility management (Handover) |
| 5.2.3.2.5.1 | Describe the handover mechanisms and procedures which are associated with – Inter-System handover including the ability to support mobility between the RIT/SRIT and at least one other IMT system – Intra-System handover 1 Intra-frequency and Inter-frequency 2 Within the RIT or between component RITs within one SRIT (if applicable) Characterize the type of handover strategy or strategies (for example, UE or base station assisted handover, type of handover measurements). What other IMT system (other than IMT-2020) could be supported by the handover mechanism? Network controlled handover mechanism is used for connected mode UEs when performing Inter-System and Intra-System handover. Source base station initiates handover and target base station performs admission control. UE receives a handover command which is provided by source base station and moves to the target base station. Handover mechanism uses intra-frequency, inter-frequency, and inter-RAT measurements which are performed by UEs. UE measures reference signal strength and quality and reports them to base station for handover decision. Time gap for measurements is needed for inter-frequency measurements. For more details, refer to TTAT.3G-38.300 sub-clauses 9.2.3 & 9.3. |
| 5.2.3.2.5.2 | Describe the handover mechanisms and procedures to meet the simultaneous handover requirements of a large number of users in high speed scenarios (up to 500km/h moving speed) with high handover success rate. In NR, the physical layer supports random access channel (RACH) sequences with 15 kHz and 30 kHz subcarrier spacing for frequencies between 410 MHz and 7125 MHz, and 60 kHz and 120 kHz subcarrier spacing for frequency between 24250 MHz and 52600 MHz, which have high tolerance to Doppler effects. The physical layer also supports RACH sequences with 1.25 kHz subcarrier spacing and sequence restriction rules that enable use of RACH sequences in high Doppler scenarios. NR additionally supports multiple RACH resource multiplexing in frequency and time domain that allows large multiplexing of users that enable large number of users to perform handover. Radio resource management (RRM) are designed to work properly in high speed scenarios. Specifically, for the simultaneous handover requirements of a large number of users, the existing handover mechanism in NR can provide sufficient resource (RACH, uplink and downlink data channels) for handover purposes. |
| 5.2.3.2.6 | Radio resource management |
| 5.2.3.2.6.1 | Describe the radio resource management, for example support of: – centralised and/or distributed RRM – dynamic and flexible radio resource management – efficient load balancing. RRM functions are located in base station and perform distributed RRM mechanism. The functions include radio bearer control, radio admission control, and radio resource allocation. RRM performs packet scheduling to dynamically allocate radio resources to UEs. The scheduling is based on QoS and channel quality information, and the periodicity of scheduling is one or multiple slots for slot-based scheduling and less than one slot for non-slot based scheduling. In addition, RRM functions can flexibly configure/modify/release radio bearers and resources to support various service scenarios. RRM provides load balancing between cells with handover, cell reselection mechanisms to efficiently distribute UEs and traffic among cells. |
| 5.2.3.2.6.2 | Inter-RIT interworking Describe the functional blocks and mechanisms for interworking (such as a network architecture model) between component RITs within a SRIT, if supported. The RIT supports interworking with other RAT (e.g. LTE) by providing multi-connectivity and handover mechanisms. Using multi-connectivity, a UE can establish radio connections with multiple base stations. Additionally, handover mechanism supports UE’s mobility among different RATs |
| 5.2.3.2.6.3 | Connection/session management The mechanisms for connection/session management over the air-interface should be described. For example: – The support of multiple protocol states with fast and dynamic transitions. – The signalling schemes for allocating and releasing resources. Connection management is performed from base station and session management is performed from network node (SMF: Session Management Function) by using RRC protocol and NAS protocol. The RRC protocol is composed of three states, RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE. The RRC_INACTIVE state supports fast transition to RRC_CONNECTED state by maintaining connection between UE and base station. RRC messages are used to establish, reconfigure, release of dedicated radio resources between UE and base station. |
| 5.2.3.2.7 | Frame structure |
| 5.2.3.2.7.1 | Describe the frame structure for downlink and uplink by providing sufficient information such as: – frame length, – the number of time slots per frame, – the number and position of switch points per frame for TDD – guard time or the number of guard bits, – user payload information per time slot, – sub-carrier spacing – control channel structure and multiplexing, – power control bit rate. Subcarrier spacings of 15, 30, 60, 120, and 240 kHz are supported. For synchronization signals, 15 and 30 kHz are supported for frequency range 1 (450 MHz ~ 6 GHz), and 120 and 240 kHz are supported for frequency range 2 (24.25 GHz ~ 52.6 GHz). For control and data channels, 15, 30, and 60 kHz are supported for frequency range 1, and 120 kHz are supported for frequency range 2. One radio frame of length 10 ms consists of 10 subframes, each of length 1 ms. Each subframe consists of 1, 2, 4, 8, and 16 slots for subcarrier spacing of 15, 30, 60, 120, and 240 kHz, respectively. Each slot consists of 14 OFDM symbols in case of normal cyclic prefix and 12 OFDM symbols in case of extended cyclic prefix. In TDD, per each slot, up to 2 DL-to-UL switching points are allowed. One or more OFDM symbols serve as a guard time between DL and UL. Downlink control channel region is flexibly allocated in time and frequency. A downlink control region spans 1~3 OFDM symbols in time, and is localized or distributed in frequency. A downlink control channel can be time and/or frequency multiplexed with a downlink data channel on a slot or mini-slot basis. Uplink control channel region is flexibly allocated in time and frequency. An uplink control region spans 4~14 OFDM symbols for long duration formats and 1~2 OFDM symbols for short duration formats, and is localized in frequency. Intra-slot or inter-slot frequency hopping is supported. An uplink control channel can be time and/or frequency multiplexed with an uplink data channel on a slot or mini-slot basis. Uplink control information can be piggy-backed with data, i.e. transmitted with data on PUSCH when UE has uplink data to be transmitted. Up to one or multiple power control commands per slot are supported. The power control rate depends on the slot duration and the maximum allowed number of power control commands per slot. |
| 5.2.3.2.8 | Spectrum capabilities and duplex technologies NOTE 1 – Parameters for both downlink and uplink should be described separately, if necessary. |
| 5.2.3.2.8.1 | Spectrum sharing and flexible spectrum use Does the RIT/SRIT support flexible spectrum use and/or spectrum sharing? Provide the detail. Description such as capability to flexibly allocate the spectrum resources in an adaptive manner for paired and un-paired spectrum to address the uplink and downlink traffic asymmetry. Flexible spectrum use is supported by using one or multiple component carriers. One component carrier supports a scalable bandwidth with a single PRB granularity by means of bandwidth part configuration/adaptation. Multiple component carriers can be aggregated to achieve up to at least several GHz of transmission bandwidth. The aggregated component carriers can be either contiguous or non-contiguous in the same or different frequency band. For unpaired spectrum, resources can be flexibly allocated as UL or DL in a dynamic or semi-static manner. Inter-RAT and inter-operator spectrum sharing is supported for efficient use of spectrum. For example, the RIT supports spectrum sharing with LTE. A carrier of the RIT can share the spectrum with a LTE carrier by FDM (frequency division multiplexing) or TDM (time division multiplexing) manner. To enable efficient multiplexing, OFDM symbols and subcarriers can be aligned in time and frequency, respectively, between the RIT and LTE. In addition, LTE CRS (cell-specific reference signal) can be protected by rate matching the signals and channels of the RIT around the LTE CRS. |
| 5.2.3.2.8.2 | Channel bandwidth scalability Describe how the proposed RIT/SRIT supports channel bandwidth scalability, including the supported bandwidths. Describe whether the proposed RIT/SRIT supports extensions for scalable bandwidths wider than 100 MHz. Describe whether the proposed RIT/SRIT supports extensions for scalable bandwidths wider than 1 GHz, e.g., when operated in higher frequency bands noted in § 5.2.4.2. Consider, for example: – The scalability of operating bandwidths. – The scalability using single and/or multiple RF carriers. Describe multiple contiguous (or non-contiguous) band aggregation capabilities, if any. Consider for example the aggregation of multiple channels to support higher user bit rates. One component carrier supports a scalable bandwidth with a single PRB granularity. The minimum channel bandwidth is 5 or 10 MHz depending on the frequency band for frequency range 1 and 50 MHz for frequency range 2. The maximum channel bandwidth is 100 MHz for frequency range 1 and 400 MHz for frequency range 2. By aggregating up to 16 component carriers, transmission bandwidths up to at least 1.6 GHz and 6.4 GHz are respectively supported for frequency range 1 and frequency range 2 to provide high peak data rates. Component carriers can be either contiguous or non-contiguous in the same or different frequency band. Each component carrier except the primary component carrier can be activated and deactivated in a dynamic manner depending on traffic situation. |
| 5.2.3.2.8.3 | What are the frequency bands supported by the RIT/SRIT? Please list. The RIT will be capable to support IMT identified bands including following frequency bands of 3GPP. Further considerations may be expected in defined bands for 3GPP NR. Introduction of other IMT identified bands are also not precluded in the future. Frequency range 1: NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high; Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high. n77; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz. Frequency range 2: NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high; Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high. n257; 26.5 GHz – 29.5 GHz; 26.5 GHz – 29.5 GHz. n258; 24.25 GHz – 27.5 GHz; 24.25 GHz – 27.5 GHz. |
| 5.2.3.2.8.4 | What is the minimum amount of spectrum required to deploy a contiguous network, including guardbands (MHz)? The minimum amount of paired spectrum is 2 x 5 MHz. The minimum amount of unpaired spectrum is 5 MHz. |
| 5.2.3.2.8.5 | What are the minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points? The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 5 MHz for frequency range 450 – 6000 MHz; - 50 MHz for frequency range 24250 – 52600 MHz - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 100 MHz for frequency range 450 – 6000 MHz; - 400 MHz for frequency range 24250 – 52600 MHz. Multiple component carriers can be aggregated to achieve up to 6.4 GHz of transmission bandwidth. |
| 5.2.3.2.8.6 | What duplexing scheme(s) is (are) described in this template? (e.g. TDD, FDD or half-duplex FDD). Provide the description such as: – What duplexing scheme(s) can be applied to paired spectrum? Provide the details (see below as some examples). – What duplexing scheme(s) can be applied to un-paired spectrum? Provide the details (see below as some examples). Describe details such as: – What is the minimum (up/down) frequency separation in case of full- and half-duplex FDD? – What is the requirement of transmit/receive isolation in case of full- an half-duplex FDD? Does the RIT require a duplexer in either the UE or base station? – What is the minimum (up/down) time separation in case of TDD? – Whether the DL/UL ratio variable for TDD? What is the DL/UL ratio supported? If the DL/UL ratio for TDD is variable, what would be the coexistence criteria for adjacent cells? The RIT supports full-duplex/half-duplex FDD, which operate on a paired spectrum. For the base station, a duplexer is needed for half-duplex/full-duplex FDD. For the terminal, a duplexer is needed for full-duplex FDD only. The RIT also supports TDD which operates on an unpaired spectrum. For both the base station and the terminal, a RF switch is needed for TDD. For TDD, dynamic and semi-static DL/UL ratio adaptation with various symbol-level and slot-level DL/UL ratios is supported. Cross-link interference mitigation schemes are supported to enable different DL/UL configurations among adjacent cells. |

| 5.2.3.2.9 | Support of Advanced antenna capabilities |
| --- | --- |
| 5.2.3.2.9.1 | Fully describe the multi-antenna systems (e.g. massive MIMO) supported in the UE, base station, or both that can be used and/or must be used; characterize their impacts on systems performance; e.g., does the RIT have the capability for the use of: – spatial multiplexing techniques, – spatial transmit diversity techniques, – beam-forming techniques (e.g., analog, digital, hybrid). Two transmission schemes are supported: codebook based transmission and non-codebook based transmission. Semi-open loop, closed loop, and diversity schemes transparent to specification are supported. Hybrid beamforming with analog and digital beamforming is supported. |
| 5.2.3.2.9.2 | How many antenna elements are supported by the base station and UE for transmission and reception? What is the antenna spacing (in wavelengths)? The number of transmit/receive antenna elements is implementation specific and depends on the number of antenna ports. The antenna configuration (correlated/uncorrelated antennas, co-polar/cross-polar configuration, etc.) is implementation specific. Up to 32 antenna ports are supported in the downlink. Up to 4 antenna ports are supported in the uplink. |
| 5.2.3.2.9.3 | Provide details on the antenna configuration that is used in the self-evaluation. Related description is provided with the self-evaluation report. |
| 5.2.3.2.9.4 | If spatial multiplexing (MIMO) is supported, does the proposal support (provide details if supported) – Single-codeword (SCW) and/or multi-codeword (MCW) – Open and/or closed loop MIMO – Cooperative MIMO – Single-user MIMO and/or multi-user MIMO. Multi-codeword (up to two codewords) is supported for DL. Single codeword is supported for UL. Closed loop MIMO is supported. It supports coordinated multipoint transmission/reception. It can mitigate interference between base stations and improve system performance, which could be used to implement different forms of cooperative multi-antenna (MIMO) transmission schemes. Both single-user MIMO and multi-user MIMO are supported. |
| 5.2.3.2.9.5 | Other antenna technologies Does the RIT/SRIT support other antenna technologies, for example: – remote antennas, – distributed antennas. If so, please describe. The use of remote antennas and distributed antennas is supported by NR. |
| 5.2.3.2.9.6 | Provide the antenna tilt angle used in the self-evaluation. Related description is provided with the self-evaluation report. |
| 5.2.3.2.10 | Link adaptation and power control |
| 5.2.3.2.10.1 | Describe link adaptation techniques employed by RIT/SRIT, including: – the supported modulation and coding schemes, – the supporting channel quality measurements, the reporting of these measurements, their frequency and granularity. Provide details of any adaptive modulation and coding schemes, including: – Hybrid ARQ or other retransmission mechanisms? – Algorithms for adaptive modulation and coding, which are used in the self-evaluation. – Other schemes? For data, the RIT supports the modulation schemes π/2-BPSK (for uplink only), QPSK, 16 QAM, 64 QAM and 256 QAM, and distinct coding schemes for either data or control. LDPC coding is applied for both downlink data and uplink data. In both downlink and uplink, link adaptation (selection of modulation scheme and code rate) is controlled by the base station. In the downlink the network selection of modulation-scheme/code-rate combination can e.g. be based on Channel Quality Indicators (CQIs) reported by the terminals. Several different CQI modes exist, including normal resolution and high resolution modes, frequency-selective and wideband modes, periodic, semi-persistent and aperiodic modes. The CQI mode is controlled by the base station. In the uplink, the base station may measure either the traffic channel or sounding reference signals and use this as input to link adaptation. On the MAC layer, hybrid ARQ with soft-combining between transmissions is supported. Different redundancy versions are used for different transmissions. The modulation scheme may be changed for retransmissions. To increase resource efficiency for both downlink and uplink, the retransmission can be based on one transport block or a group of smaller pieces of code blocks depending on the base station’s decision. In order to minimize delay and feedback, a set of parallel stop-and-wait protocols are used. The residual errors can be handled by MAC ARQ on the RLC layer. For more details, refer to TTAT.3G-38.214, 38.321 and 38.322. |
| 5.2.3.2.10.2 | Provide details of any power control scheme included in the proposal, for example: – Power control step size (dB) – Power control cycles per second – Power control dynamic range (dB) – Minimum transmit power level with power control – Associated signalling and control messages. The RIT uplink power control is based on both signal-strength measurements done by the terminal itself (open-loop power control), as well as measurements by the base station. The later measurements are used to generate power-control commands that are subsequently fed back to the terminals as part of the downlink control signalling (closed-loop power control) and can be multiple commands to support the multipoint reception and/or beam-based transmission/reception. Both absolute and relative power-control commands are supported, and each step size is chosen in {-1, 0, 1, 3} for relative control and {-4, -1, 1, 4} for absolute control. The power control command is included in the downlink control message and can be broadcasted to relevant terminals for each slot, i.e., 1 ms or less depending on the numerology. Downlink power control is network-implementation specific and thus outside the scope of the specification. For more details, refer to TTAT.3G-38.213. |

| 5.2.3.2.11 | Power classes |
| --- | --- |
| 5.2.3.2.11.1 | UE emitted power |
| 5.2.3.2.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? The output power is defined separately for different frequency ranges (FRs). For FR1, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The maximum output power is defined by UE power class as following table. <UE maximum output power for FR 1> FR1 band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). n77, n78; 26; +2/-3; 23; +2/-3. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. For FR 2, the maximum output power radiated by the UE for any transmission bandwidth of NR carrier is defined as TRP (Total Radiated Power) and EIRP(Equivalent Isotropically Radiated Power). In each power class in FR2, a UE minimum peak EIRP and UE maximum output power limits (maximum TRP and maximum EIRP) are specified as following table. < UE maximum output power for FR 2> FR2 band; Power class 1; Power class 2; Power class 3; Power class 4. Min Peak EIRP (dBm); Max TRP (dBm); Max EIRP (dBm); Min Peak EIRP (dBm); Max TRP (dBm); Max EIRP (dBm); Min Peak EIRP (dBm); Max TRP (dBm); Max EIRP (dBm); Min Peak EIRP (dBm); Max TRP (dBm); Max EIRP (dBm). n257; 40.0; 35; 55; 29; 23; 43; 22.4; 23; 43; 34; 23; 43. n258; 40.0; 35; 55; 29; 23; 43; 22.4; 23; 43; 34; 23; 43. NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Power class 1 UE is used for fixed wireless access (FWA).. For more details, refer to TTAT.3G-38.101-1 and 38.101-2. |
| 5.2.3.2.11.1.2 | What is the maximum peak power transmitted while in active or busy state? The maximum output power for transmission bandwidth within the channel bandwidth of carrier is defined according to the UE power class and the frequency range (See 5.2.3.2.11.1.1). |
| 5.2.3.2.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. The time averaged power transmitted in active state may be affected by various factors such as UE channel conditions and allocated bandwidth. One method calculating the averaged transmit power is to take median of minimum UE output power and maximum UE output power, where the maximum UE output power is defined in 5.2.3.2.11.1.1 and 5.2.3.2.11.1.2, and the minimum output power of NR UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) as shown in the following tables. For FR2, the minimum output power of power class 1 is specified differently from that of power class 2, 3 and 4. <Minimum UE output power for FR1> Channel bandwidth (MHz); Minimum output power (dBm); Measurement bandwidth (MHz). 5; -40; 4.515. 10; -40; 9.375. 15; -40; 14.235. 20; -40; 19.095. 25; -39; 23.955. 30; -38.2; 28.815. 40; -37; 38.895. 50; -36; 48.615. 60; -35.2; 58.35. 80; -34; 78.15. 90; -33.5; 88.23. 100; -33; 98.31. <Minimum UE output power for FR2> FR2 band; UE power class; Channel bandwidth # (MHz); # Minimum # output power # (dBm); # Measurement # bandwidth # (MHz). # n257, # n258; # Power class 1; # 50; # 4; # 47.52. 100; 4; 95.04. 200; 4; 190.08. 400; 4; 380.16. Power class 2, 3, 4; 50; -13; 47.52. 100; -13; 95.04. 200; -13; 190.08. 400; -13; 380.16. For more details, refer to TTAT.3G-38.101-1 and 38.101-2. |
| 5.2.3.2.11.2 | Base station emitted power |
| 5.2.3.2.11.2.1 | What is the base station transmit power per RF carrier? The base station is declared to support one or more beams. The base station transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction. For more details, refer to TTAT.3G-38.104. |
| 5.2.3.2.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? The base station maximum carrier output power is mean power level measured per carrier at the indicted interface, during the transmitter ON period in a specified reference condition. Despite the general requirements for the BS output power described in TTAT.3G- 38.104, additional regional requirements might be applicable. There may exist regional regulatory requirements which limit the maximum radiated transmit power. For more details, refer to TTAT.3G-38.101-1, 38.101-2 and 38.104. |
| 5.2.3.2.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? The time averaged power transmitted in active state may be affected by various factors such as UE channel conditions and allocated bandwidth. The base station maximum carrier output power is mean power level measured per carrier at the indicted interface, during the transmitter ON period in a specified reference condition. |
| 5.2.3.2.12 | Scheduler, QoS support and management, data services |
| 5.2.3.2.12.1 | QoS support – What QoS classes are supported? – How QoS classes associated with each service flow can be negotiated. – QoS attributes, for example: • data rate (ranging from the lowest supported data rate to maximum data rate supported by the MAC/PHY); • control plane and user plane latency (delivery delay); • packet error ratio (after all corrections provided by the MAC/PHY layers), and delay variation (jitter). – Is QoS supported when handing off between radio access networks? If so, describe the corresponding procedures. – How users may utilize several applications with differing QoS requirements at the same time. QoS flow is the finest granularity of QoS differentiation in a PDU session and it is identified by an QoS Flow ID (QFI) in a packet header. A QoS Flow either be 'Non-GBR (Guaranteed Bit Rate)' or 'GBR' depending on its QoS profile. The QoS profile of a Non-GBR QoS flow contains 5G QoS Identifier (5QI), Allocation and Retention Priority (ARP), and optionally Reflective QoS Attribute (RQA). In case of a GBR QoS flow, the QoS profile contains 5QI, ARP, Guaranteed Flow Bit Rate (GFBR), Maximum Flow Bit Rate (MFBR), and notification control. 7 values of 5QI for GBR and 8 values of 5QI for non-GBR are standardised. The SMF of CN provides the QFI together with the QoS profile to the (R)AN when a QoS flow is established or modified. Data radio bearer is used to convey packets via radio access and QoS flow to DRB association is performed at both NAS (Non-Access Stratum) and AS (Access Stratum). QoS characteristics which are indicated through the 5QI value are resource type (GBR/Non-GBR: Guaranteed Bit Rate), priority level, packet delay budget, packet error rate, averaging window. The QoS profile could be used to determine which QoS flows to prioritize at handover. By using multiple QoS flows having different QoS profile, multiple application flows with different QoS requirements could be accommodated at the same time. Network configures multiple DRBs with different QoS profile to one UE and schedules multiple DRBs at the same time. |
| 5.2.3.2.12.2 | Scheduling mechanisms – Exemplify scheduling algorithm(s) that may be used for full buffer and non-full buffer traffic in the technology proposal for evaluation purposes. Describe any measurements and/or reporting required for scheduling. In NR physical control and shared channels can be separately and dynamically scheduled for both uplink and downlink. A scheduling unit for downlink shared channel may span from 2-14 symbols and for uplink shared channel from 1-14 symbols (14 symbols comprise a “slot”). Sub-carrier spacing for different physical channels may be dynamically changed by switching bandwidth-parts (BWP). Typically, NR scheduling is based on the instantaneous radio-link quality as seen by the different users, and the traffic demand and quality-of-service requirements of individual users and in the cell as a whole. The former is based on CQI reports from the terminals (downlink) or measurements of sounding signals from the terminals (uplink). Based on this the base station may e.g. apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the “higher layers”. For non-full buffer traffic like VOIP (or any traffic having similar characteristics) semi-persistent scheduling in DL can be applied, by which a user can be allocated time-frequency resources in a semi-persistent manner, i.e., fixed resources are allocated at certain intervals without L1/L2 control signaling each time. This is especially useful to reduce the L1/L2 control signaling overhead and to increase VoIP capacity. In addition, with UL Configured Grants, the scheduler can allocate uplink resources to users. When a configured uplink grant is active, if the user cannot find an uplink grant assigned via downlink control channel an uplink transmission according to the configured uplink grant can be made. Otherwise, if the user finds an uplink grant assigned via downlink control channel, this assignment overrides the configured uplink grant. In general for TDD operation a slot may be used for dynamically allocating DL or UL transmissions or both. NR supports slot aggregation in downlink and uplink, by which time-frequency resources can be allocated consecutively to a user for a longer period than a slot by a single L1/L2 control signaling. A larger transport block size or a lower coding rate can be supported by this technique. This is especially useful when the coverage needs to be extended. As another option to extend coverage or improve reliability in addition to slot aggregation, a set of MCS tables supporting very low code rate for both DL and UL can be used. The scheduler may pre-empt an ongoing transmission to one user with a latency-critical transmission to another user. The scheduler can configure users to monitor interrupted transmission indications. If a user receives the interrupted transmission indication, the user may assume that no useful information to that user was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this user. Alternatively, instead of transmitting interruption indication, the scheduler may retransmit only the preempted code blocks to a UE and instruct to do proper transport block decoding with other already received code blocks. For the downlink and the uplink, intercell-interference coordination can be realized by the scheduler that is transparent to the physical layer. |
| 5.2.3.2.13 | Radio interface architecture and protocol stack |
| 5.2.3.2.13.1 | Describe details of the radio interface architecture and protocol stack such as: – Logical channels – Control channels – Traffic channels Transport channels and/or physical channels. The control plane consists of the NAS, RRC, PDCP, RLC, MAC and PHY layers. control plane protocol stack - NAS: Protocol between the UE and CN, and performs e.g., authentication, context activation/ deactivation and location registration management. - RRC: Protocol between the UE and RAN, and performs e.g., system information delivery, connection establishment/release and mobility control. - PDCP: Performs header compression, integrity protection and ciphering. - RLC: Performs segmentation/concatenation of packets and ARQ. - MAC: Performs HARQ and scheduling. The user plane consists of the SDAP, PDCP, RLC, MAC and PHY layers. user plane protocol stack The functions of PDCP, RLC and MAC are the same as for C-plane. - SDAP: SDAP (Service Data Adaptation Protocol) performs mapping between a QoS flow and a data radio bearer. Logical channels are classified to BCCH, PCCH, CCCH, DCCH, and DTCH. Transport channels are composed of BCH, PCH, RACH, DL-SCH, and UL-SCH. Physical channels are composed of PBCH, PRACH, PDCCH, PUCCH, PDSCH, and PUSCH. For more details, refer to: TTAT.3G 38.300, 38.401, 38.201 and 37.340. |
| 5.2.3.2.13.2 | What is the bit rate required for transmitting feedback information? Assuming that an RLC AM Status report is sent once every 50 ms and that comprises on average less than one negative acknowledgement, its size including RLC/MAC header overhead is then on average 3+1=4 octets. Consequently, the overhead this causes is 32/0.05= 640 bit/s. This corresponds to 0.06, 0.006 and 0.0006% overhead for link speeds of 1, 10 and 100 Mbit/s. |
| 5.2.3.2.13.3 | Channel access: Describe in details how RIT/SRIT accomplishes initial channel access, (e.g. contention or non-contention based). The UE gets access to a cell via the contention-based random access procedure at initial connection from idle mode. With this procedure the UE transmits a random access preamble by choosing one from a maximum of 64 possible candidates on a PRACH which are configured by the network and signalled to the UE via broadcast signalling. If the base station detects a random access preamble, the base station sends a response allocating a temporary UE identity and radio resources for the uplink transmission of the initial RRC message. The UE further transmits the initial RRC message to the base station, using the allocated resources. If the base station successfully receives this initial RRC message, the base station echoes the message back to the UE in order to resolve possible contention. The UE could perform multiple attempts until it is successful in accessing the cell or until a timer supervising the procedure has elapsed. Multiple PRACHs can be configured, in order to reduce the probability of contention and to increase the PRACH capacity. For UEs who have gained access to a cell (i.e. has RRC connection), non-contention based channel access request is supported (e.g., during Handover to a new cell) by means of: - Scheduling Request (SR) on PUCCH, in which a dedicated resource on PUCCH for SR is semi-statically assigned by the base station to a UE, and - Dedicated random access resource assignment, in which a dedicated random access preamble on specific PRACH(s) is dynamically assigned by the base station to a UE via dedicated signalling, thereby making it possible to skip the contention resolution procedure. For more details, refer to TTAT.3G-38.300, 38.321 and 38.213. |
| 5.2.3.2.14 | Cell selection |
| 5.2.3.2.14.1 | Describe in detail how the RIT/SRIT accomplishes cell selection to determine the serving cell for the users. The UE performs cell search with aid of synchronisation signals (SSs) which are transmitted using 15, 30 or 60 kHz for frequency range 1, and 120 or 240 kHz for frequency range 2 on the downlink. If multiple SSs are detected, the UE selects the best SS in terms of the received reference-signal power to ensure efficient spectrum usage. If a cell is detected, the UE checks the suitability of the cell, i.e., the UE checks that the received reference-signal power is above the minimum required value and checks the PLMN identity, as well as other parameters restricting camping on the cell (e.g., cell barring status), on system information. If multiple cells are found as suitable, the UE selects the best cell in terms of the received reference-signal power to ensure efficient spectrum usage. For inter-frequency and inter-RAT cell reselection, absolute priorities could be configured so that certain frequency/RAT would be prioritized for camping. The priorities could be signalled by system information broadcast or by dedicated signalling. For more details, refer to TTAT.3G-38.300 and 38.304. |

| 5.2.3.2.15 | Location determination mechanisms |
| --- | --- |
| 5.2.3.2.15.1 | Describe any location determination mechanisms that may be used, e.g., to support location based services. NG RAN provides mechanisms to support or assist the determination of the geographical position of a UE. UE position knowledge can be used for Radio Resource Management, location based services for operators, subscribers, and third party service providers. User plane (U-plane) based solution (SUPL) as well as control plane (C-plane) based techniques are supported and adapted from capabilities already supported for E-UTRAN, UTRAN and GERAN, etc. The standard positioning methods supported for NG-RAN access include: - network-assisted GNSS methods; - observed time difference of arrival (OTDOA) positioning; - enhanced cell ID methods; - barometric pressure sensor positioning; - WLAN positioning; - Bluetooth positioning; - terrestrial beacon system (TBS) positioning. Use of one or more methods from the list above and hybrid positioning using multiple methods is supported using either UE-based, UE-assisted/LMF-based, and NG-RAN node assisted versions. In future releases, the work on NG-RAN RAT-dependent and RAT-independent positioning solutions is expected to continue and further enrich the location determination mechanisms that may be used to support location based services. |
| 5.2.3.2.16 | Priority access mechanisms |
| 5.2.3.2.16.1 | Describe techniques employed to support prioritization of access to radio or network resources for specific services or specific users (e.g., to allow access by emergency services). NR supports overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms. One unified access control framework as specified in 3GPP TS 22.261 section 6.22 is applied for NR. For each access attempt one Access Category and one or more Access Identities are selected. NR broadcasts barring control information associated with Access Categories and Access Identities and the UE determines whether an identified access attempt is authorized or not, based on the broadcasted barring information and the selected Access Category and Access Identities. In the case of multiple core networks sharing the same RAN, the RAN provides broadcasted barring control information for each PLMN individually. The unified access control framework is applicable to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state). For NAS triggered requests, the UE NAS determines one access category and access identity(ies) for the given access attempt and provides them to RRC for access control check. The RRC performs access barring check based on the access control information and the determined access category and access identities. The RRC indicates whether the access attempt is allowed or not to NAS layer. The NAS also performs the mapping of the access category and access identity(ies) associated with the access attempt to establishment cause and provides the establishment cause to RRC for inclusion in connection request to enable the gNB to decide whether to reject the request. For AS triggered request (i.e. RNA update), the RRC determines the resume cause value and the corresponding access category. |
| 5.2.3.2.17 | Unicast, multicast and broadcast |
| 5.2.3.2.17.1 | Describe how the RIT/SRIT enables: – broadcast capabilities, – multicast capabilities, – unicast capabilities, using both dedicated carriers and/or shared carriers. Please describe how all three capabilities can exist simultaneously. The RIT is envisioned to support broadcast, multicast and unicast services. Most of the downlink and uplink transmissions are unicast. Broadcast is supported to transfer some kind of the system information. |
| 5.2.3.2.17.2 | Describe whether the proposal is capable of providing multiple user services simultaneously to any user with appropriate channel capacity assignments? Multiple services per user can be supported by setting up multiple bearers per user. Each bearer is characterized by an individual QoS profile, including the attributes guaranteed bitrate, maximum bitrate, and allocation and retention priority. |
| 5.2.3.2.17.3 | Provide details of the codec used. Does the RIT/SRIT support multiple voice and/or video codecs? Provide the detail. The RIT supports the AMR-WB voice codec and could support various other voice and video codec as desired. The RIT is agnostic of the codec used and is capable of accommodating diverse range of codec that are commonly used today as well as those defined in future. |
| 5.2.3.2.18 | Privacy, authorization, encryption, authentication and legal intercept schemes |
| 5.2.3.2.18.1 | Any privacy, authorization, encryption, authentication and legal intercept schemes that are enabled in the radio interface technology should be described. Describe whether any synchronisation is needed for privacy and encryptions mechanisms used in the RIT/SRIT. Describe how the RIT/SRIT addresses the radio access security, with a particular focus on the following security items: – system signalling integrity and confidentiality, – user equipment identity authentication and confidentiality, – subscriber identity authentication and confidentiality, – user data integrity and confidentiality Describe how the RIT/SRIT may be protected against attacks, for example: – passive, – man in the middle, – replay, – denial of service. NR has made substantial enhancements to subscriber’s privacy compared to earlier generations, see 3GPP TS 33.501. The most important enhancement is the concealment of subscription permanent identifier over-the-air. This feature is mainly aimed against the active attacker. Another enhancement is the guaranteed regular refreshment of subscription temporary identifier. This feature is mainly aimed against the passive attacker. Another enhancement is the decoupling of the permanent identifier from the Paging mechanism, i.e., there is no longer a Paging message with permanent identifier, the Paging timings are no longer based on permanent identifier. This feature mitigates privacy attacks that use side-channel information in the Paging protocol. Another enhancement comes from the best effort protection of information in the initial message, i.e., if security is setup, privacy sensitive information is concealed, otherwise skipped until the security is setup. Yet another effort is description of a device-assisted network-based framework for false base station detection. This feature can be used to thwart denial-of-service kind of attackers. One enhancement is to increase the authentication between the service network and the core network, or the function of the home network control which can be used to prevent false fraud of the service network. One enhancement is mitigation of bidding down attacks, this feature is to avoid the lower security features of the UE or network selection. Another enhancement is the authorization and authentication of the security and network capabilities of the serviced interface. The new features in NR, e.g., multi connectivity, and deploying a single base station as two split units, also help improve resilience of the radio access network. Authentication/authorization in NR builds on strong cryptographic primitives and security characteristics that already existed in LTE-Advanced. On top of this, NR has made great improvement by introduction of the flexible authentication framework for both the 3GPP and non-3GPP network. Even further, NR has significantly reduced the risk of fraud against the subscribers. An enhancement for NR is that a security anchor (SEAF) is introduced in the authentication framework. And the secondary authentication of the external network is increased. NR includes protection against eavesdropping, modification, and replay attacks. The strong and well-proven security algorithms from the LTE-Advanced system are reused and support the transport of 256 bit keys. Signalling traffic is encrypted and integrity protected. User plane traffic is encrypted and can be integrity protected. This integrity protection of user plane traffic is a new enhancement in NR. It is mandatory to integrity protect radio resource control messages that redirect devices. This feature makes it infeasible for attackers to perform rogue redirections. Various timers are specified for different scenarios for devices to wait and retry connecting with the network. This feature mitigates the risk of attackers trying to keep devices locked out from the network. All the enhancements in NR are made while simultaneously complying with regulatory duties. Legal intercept is provided by core network functions. |
| 5.2.3.2.19 | Frequency planning |
| 5.2.3.2.19.1 | How does the RIT/SRIT support adding new cells or new RF carriers? Provide details. 1008 physical cell IDs are supported. Therefore 1008 cell reuse is realized theoretically. New RF carriers can be allocated on desired frequency location with the same or different physical cell IDs. Actual cell and carrier deployment is operation specific. From a user terminal perspective, adding new cells or carriers is supported by carrier aggregation and dual connectivity. Dynamic activation and deactivation of cells or carriers depending on traffic load is supported. |
| 5.2.3.2.20 | Interference mitigation within radio interface |
| 5.2.3.2.20.1 | Does the proposal support Interference mitigation? If so, describe the corresponding mechanism. The RIT has been designed with the aim to minimize the always-on signals to reduce the interference in the system. This is achieved by: - Support longer periodicities for synchronization signals, broadcast channels and periodic reference signals - Use UE-specific demodulation reference signals for control and data that are only transmitted when control and/or data is being transmitted - Control channel resource allocation in the frequency domain is configurable to reduce the interference to control channels in neighbouring cells Coordinated multipoint transmission/reception (CoMP) is another approach supported by the RIT to mitigate interference between cells and improve system performance by dynamic coordination in the scheduling/transmission between/from multiple cell sites. |
| 5.2.3.2.20.2 | What is the signalling, if any, which can be used for intercell interference mitigation? To support handling of Cross Link Interference (CLI) and for Remote Interference Management (RIM), NR will support inter-base station signalling via the Xn interface and the core network. This is further described in 3GPP TR 38.866. |
| 5.2.3.2.20.3 | Link level interference mitigation Describe the feature or features used to mitigate intersymbol interference. The RIT adopts OFDM modulation to mitigate inter-symbol interference for both uplink and downlink. The cyclic prefix effectively eliminates the inter-symbol interference. The OFDMA is also supported, and to align the received timing at uplink, the base station controls the transmission timing of each terminal. The initial transmission timing is known to the terminal via the random access procedure. |
| 5.2.3.2.20.4 | Describe the approach taken to cope with multipath propagation effects (e.g. via equalizer, rake receiver, cyclic prefix, etc.). The use of CP-OFDM with a feasible cyclic prefix provides robustness to time-dispersion/frequency-selectivity on the radio channel. This can apply to both downlink and uplink. According to the configuration for uplink, the DFT-s-OFDM can also be used. Time-dispersion/frequency-selectivity on the radio channel can be handled by receiver-side equalization. The detailed equalization approach is implementation dependent. Examples of equalization approaches include frequency-domain linear equalization and Turbo equalization. The use of cyclic prefix also for the uplink may simplify the equalizer implementation. |
| 5.2.3.2.20.5 | Diversity techniques Describe the diversity techniques supported in the user equipment and at the base station, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – Time diversity: repetition, Rake-receiver, etc. – Space diversity: multiple sectors, etc. – Frequency diversity: frequency hopping (FH), wideband transmission, etc. – Code diversity: multiple PN codes, multiple FH code, etc. – Multi-user diversity: proportional fairness (PF), etc. – Other schemes. Characterize the diversity combining algorithm, for example, switched diversity, maximal ratio combining, equal gain combining. Provide information on the receiver/transmitter RF configurations, for example: – number of RF receivers – number of RF transmitters. The various diversity techniques are supported. For the space domain, the base station cooperation is supported as a macro diversity. The micro diversity is supported implicitly. The antenna selection at a terminal is supported for uplink data transmission. For the frequency domain, the DFT-s-OFDM for uplink transforms among subcarriers to achieve the diversity. The CP-OFDM for both downlink and uplink supports the wideband transmission bandwidth with channel coding, which gains the diversity. For the time diversity, the retransmission based on HARQ-ACK is supported that the retransmission occurs after the coherence time. The multi-user diversity is supported by means of the base station scheduling for both downlink and uplink. The diversity combining algorithm is implementation dependent. The number of RF configurations are implementation dependent and the number of antenna ports can be configured, but it is subject to UE capability. As an example, for single user MIMO operation in a single cell operation, the terminal may have 4 transmitters and 8 receivers. |
| 5.2.3.2.21 | Synchronization requirements |
| 5.2.3.2.21.1 | Describe RIT’s/SRIT’s timing requirements, e.g. – Is base station-to-base station synchronization required? Provide precise information, the type of synchronization, i.e., synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy. – Is base station-to-network synchronization required? State short-term frequency and timing accuracy of base station transmit signal. Tight BS-to-BS synchronization is not required. However, in some scenarios, system performance may have gains from tight BS-to-BS synchronization. As an example, assuming coordinated transmission/reception based on joint processing (joint transmission in the downlink direction, joint reception in the uplink direction) between multiple base stations, tight synchronization and time alignment between the base stations involved in the joint processing would be required. For proper operation, the synchronization should be aligned within the cyclic prefix. Tight BS-to-network synchronization is not required. Frequency accuracy of base station transmit signal is within ±0.05 ppm observed over a period of one slot. Timing accuracy of base station transmit signal is within 65 ns for single carrier (MIMO or TX div), 260 ns for intra-band contiguous carrier aggregation, 3µs for intra-band non-contiguous and inter-band CA. For more information, please refer to TTAT.3G-38.401, 38.133 and 38.104. |
| 5.2.3.2.21.2 | Describe the synchronization mechanisms used in the proposal, including synchronization between a user terminal and a base station. The RIT may support different kind of methods and techniques to satisfy the synchronization requirements. A UE acquires time and frequency synchronization with a cell using the primary synchronization signals (PSS) and secondary synchronization signals (SSS). PSS and SSS together used for cell ID detection. For more details, refer to TTAT.3G-38.213 and 38.211. |
| 5.2.3.2.22 | Link budget template Proponents should complete the link budget template in § 5.2.3.3 to this description template for the environments supported in the RIT. The information is provided with link budget template. |
| 5.2.3.2.23 | Support for wide range of services |
| 5.2.3.2.23.1 | Describe what kind of services/applications can be supported in each usage scenarios in Recommendation ITU-R M.2083 (eMBB, URLLC, and mMTC). Mobile Broadband addresses the human-centric use cases for access to multi-media content, services and data. The demand for mobile broadband will continue to increase, leading to enhanced Mobile Broadband. The enhanced Mobile Broadband usage scenario will come with new application areas and requirements in addition to existing Mobile Broadband applications for improved performance and an increasingly seamless user experience. This usage scenario covers a range of cases, including wide-area coverage and hotspot, which have different requirements. For the hotspot case, i.e. for an area with high user density, very high traffic capacity is needed, while the requirement for mobility is low and user data rate is higher than that of wide area coverage. For the wide area coverage case, seamless coverage and medium to high mobility are desired, with much improved user data rate compared to existing data rates. However, the data rate requirement may be relaxed compared to hotspot. eMBB enables Gigabytes transmission in a second, therefore, facilitates such services as 3D video, UHD screens, work and play in the cloud, and augmented reality. Ultra-reliable and low latency communications have stringent requirements for capabilities such as throughput, latency and availability. Wireless control of industrial manufacturing or production processes, remote medical surgery, distribution automation in a smart grid, transportation safety can be supported with the proposed technology. Massive machine type communications is characterized by a very large number of connected devices typically transmitting a relatively low volume of non-delay-sensitive data. Devices are required to be low cost, and have a very long battery life. Smart city is one of the mMTC applications. For more details, refer to ITU-R M.2083-0. |
| 5.2.3.2.23.2 | Describe any capabilities/features to flexibly deploy a range of services across different usage scenarios (eMBB, URLLC, and mMTC) in an efficient manner, (e.g., a proposed RIT/SRIT is designed to use a single continuous or multiple block(s) of spectrum). NR is capable of deploying a range of services across different usage scenarios. While the specification does not match any physical layer functionality to any service, different components can benefit different services in specific usage scenarios. Specifically, the following low latency structures cater especially to the URLLC services - Front loaded DMRS allows for the channel estimate to be ready before the full data block is received - Frequency-first mapping of data bits to physical resources allows for the channel decoder to operate in a pipelined fashion, starting to decode the data block immediately when the first symbol has been received - Very tight UE processing time budget especially targeted for ultra-low latency device types - Very short scheduling interval achieved with both high subcarrier spacing (short symbol duration) and the possibility to schedule short time intervals only - At least an UL transmission scheme without scheduling grant is supported to reduce UL latency. mMTC services can benefit from the following components - DFT-spreading and Pi/2 BPSK modulation for reduced PAPR and increased average Tx power for better coverage - Slot aggregation for both control and data for better coverage - High-aggregation level downlink control for better coverage - RRC_INACTIVE state for optimized signalling overhead when moving to active state - Extended DRX cycle for RRC active state to improve battery life - Support for narrow-band (low-cost) UEs within a wide-band carrier URLLC, eMBB and mMTC services can coexist within the same spectrum in both time and frequency domain in multiplexed manner. URLLC can pre-empt ongoing eMBB/mMTC transmissions, if necessary, and URLLC services can be mapped to e.g. a shorter allocation duration for lower latency by small number of scheduled symbols, as well as by using higher sub-carrier spacing and thus allocation duration for the same number of scheduled symbols, while eMBB services can be mapped to do the opposite. Different sub-carrier spacings and scheduling interval durations that are appropriate to the desired service type (e.g., different latency and data rate requirements) can coexist in a single carrier with no need for fixed divisions within the carrier, by e.g., using spectral refinement techniques such as filtering, windowing, etc. with the designated waveforms for NR. |
| 5.2.3.2.24 | Global circulation of terminals Describe technical basis for global circulation of terminals not causing harmful interference in any country where they circulate, including a case when terminals have capability of device-to-device direct communication mode. 3GPP defines a set of NR frequency bands with band specific requirements in such a way that each band complies to the regulatory requirements of a given region or regions within the used deployment. The gNB broadcasts the band information on the deployed carriers and possible additional transmit requirements for the UE to comply to. If the UE is not able to comply with the requirements provided by the network, it is not allowed to initiate connection towards the gNB on that band. In more detail, for a given band, a transmission the spectrum mask is specified in terms of a normative (general) spectrum emission mask and an additional spectrum mask in TTAT.3G-38.101-1 and 38.101-2, section 6.5. The additional spectrum emission mask which is signaled by the network to the UE as a normative requirement can be used to address; a specific regional regulatory requirement, a frequency band specific requirement, a roaming requirement and a specific deployment scenario. This additional spectrum emission mask can be used to support the many different sharing requirements in terms of co-existence for a global roaming terminal. 3GPP Release 16 is working to introduce two side link operation modes, where the harmful interference is managed by the network: - Mode 1: The NR gNB schedules the UE’s sidelink, and the UE will only transmit on its sidelink when scheduled by the gNB it is connected to. - Mode 2: The UE’s sidelink is preconfigured and the configuration can be updated whenever the UE has IP connectivity. The sidelink configuration includes a geographical area or areas in which the slidenk transmissions are allowed, and if the UE moves out of the geographical area(s) it is not allowed to transmit on the sidelink. |
| 5.2.3.2.25 | Energy efficiency Describe how the RIT/SRIT supports a high sleep ratio and long sleep duration. Describe other mechanisms of the RIT/SRIT that improve the support of energy efficiency operation for both network and device. Network energy efficiency The fundamental always-on transmission that must take place is the periodic SS/PBCH block. The SS/PBCK block is used for the UE to detect the cell, obtain basic information of it on PBCH, and maintain synchronization to it. The duration, number and frequency of the SS/PBCH block transmission depends on the network setup. For the purposes of blind initial access the UE may assume that there is an SS/PBCH block once every 20 ms. If the network is configured to transmit the SS/PBCH block less frequently, that will improve the network energy efficiency at the cost of increased the initial cell detection time, but after the initial connection has been established, the UE may be informed of the configured SS/PBCH block periodicity in the cell from set of {5, 10, 20, 40, 80, 160} ms. If the cell set up uses analogue beamformer component, it may provide several SS/PBCH blocks multiplexed in time-domain fashion within one SS/PBCH block period. Remaining minimum system information carried over SIB1 needs to be broadcast at least in the cells in which the UEs are expected to be able to set up the connection to the network. There is no specific rate at which the SIB1 needs to be repeated in the cell, and once the UE acquires the SIB1, it does not need to read it again. SIB1 could be time or frequency multiplexed with the SS/PBCH block. In the frequency multiplexing case, there would be no additional on-time for the gNB transmitter. In the time multiplexing case, having a lower rate for SIB1 than for SS/PBCH block would suffice at least for higher SS/PBCH repetition frequencies. The sleep ratio under the above mechanism is evaluated in 3GPP TR37.910. Device energy efficiency Multiple features facilitating device energy efficiency have been specified for NR Rel-15. Discontinuous reception (DRX) in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE When DRX is configured, the UE does not have to continuously monitor PDCCH for scheduling or paging messages, but it can remain sleeping. DRX is characterized by the following: - on-duration: duration that the UE waits for, after waking up, to receive PDCCHs. If the UE successfully decodes a PDCCH, the UE stays awake and starts the inactivity timer; - inactivity-timer: duration that the UE waits to successfully decode a PDCCH, from the last successful decoding of a PDCCH, failing which it can go back to sleep. The UE shall restart the inactivity timer following a single successful decoding of a PDCCH for a first transmission only (i.e. not for retransmissions); - retransmission-timer: duration until a retransmission can be expected; - DRX cycle: specifies the periodic repetition of the on-duration followed by a possible period of inactivity (see figure below). Figure: DRX Cycle Bandwidth part (BWP) adaptation With dynamic bandwidth part adaptation, the UE can fall-back to monitoring the downlink and transmitting the uplink over a narrower bandwidth than the nominal carrier bandwidth used for high data rate transactions. This allows the UEs BB-RF interface to operate with a much lower clock rate and thus reduce energy consumption. Lower data rate exchange can still take place so that there is no need to resume full bandwidth operation just for exchanging network signalling messages or always-on packets of applications. The UE can be moved to the narrow BWP by gNBs transmitting a BWP switch bit on the scheduling DCI on the PDCCH, or based on an inactivity timer. UE can be moved back to the full bandwidth operation at any time by the gNB with the BWP switch bit. RRC_INACTIVE state The introduction of RRC_INACTIVE state to the RRC state machine allows for the UE to maintain RRC connection in an inactive state while having the battery saving characteristics of the Idle mode. This allows for maintaining the RRC connection also when the UE is inactive for longer time durations, and avoid the signalling overhead and related energy consumption needed when the RRC connection is re-established from Idle mode. Figure: NR RRC state machine Pipelining frame structure enabling micro-sleep within slots in which the UE is not scheduled The fact that the typical data transmission employs a control channel in the beginning of the slot, and the absence of the continuous reference signal to receive for channel estimate maintenance allows for the UE to determine early on in the slot whether there is a transmission to it, and if there is no data for it to decode, it may turn off its receiver until the end of the slot. Additional power saving mechanisms for NR are being specified for 3GPP Release 16, including at least the following techniques: - Downlink control channel-based triggering of UE adaptation in RRC_CONNECTED state for improved energy efficiency. - A procedure of cross-slot scheduling power saving allowing the UE to micro-sleep between control channel occasions and only activating the data channel reception when it is scheduled. |
| 5.2.3.2.26 | Other items |
| 5.2.3.2.26.1 | Coverage extension schemes Describe the capability to support/ coverage extension schemes, such as relays or repeaters. NR supports the use of the following mechanisms to improve the coverage - NR can use DFT-spreading and Pi/2 BPSK modulation to reduce PAPR and increase average Tx power for better coverage - NR can use very low coding rate for better coverage. - Slot aggregation for both control and data can be used for better coverage - High-aggregation level (up to 16) downlink control is possible for better coverage - Lower-band supplementary uplink carrier can be used with higher band TDD carrier such that coverage limited users can be allocated on SUL carrier to improve the uplink coverage. - Beam management is used to increase the coverage in case of massive MIMO. - NR also supports the use of different types of repeater (amplify-and-forward) functionality. However, the details of such functionality is outside the scope of the specification as the use of repeaters is transparent to both the UE and the network. |
| 5.2.3.2.26.2 | Self-organisation Describe any self-organizing aspects that are enabled by the RIT/SRIT. The RIT provides self-organisation aspect by self-configuration, ANR (Automatic Neighbour Relation) mechanism. With self-organisation aspect, network automatically configures newly deployed base stations and manages neighbour cell lists in a base station by using UE measurement report. For more detail, refer to TTAT.3G-38.300, 38.413, 38.423 and 38.331. |
| 5.2.3.2.26.3 | Describe the frequency reuse schemes (including reuse factor and pattern) for the assessment of average spectral efficiency and 5th percentile user spectral efficiency. The RIT can access the full frequency dynamically, and no frequency reuse is assumed semi-statically for interference coordination. However, depending on the deployment scenario, an implementation can apply a specific frequency reuse scheme and, in this case, the average/5th user spectral efficiency should be taken care of. |
| 5.2.3.2.26.4 | Is the RIT/component RIT an evolution of an existing IMT technology? Provide the detail. This RIT is new radio developed as 3GPP NR, and will be evolving. |
| 5.2.3.2.26.5 | Does the proposal satisfy a specific spectrum mask? Provide the detail. (This information is not intended to be used for sharing studies.) Yes. For both UE and Base Station side, the spectrum masks are specified in TTAT.3G-38.101-1, 38.101-2 and 38.104, respectively. |
| 5.2.3.2.26.6 | Describe any UE power saving mechanisms used in the RIT/SRIT. DRX in connected, inactive, and idle modes is supported. Inactive state, RF bandwidth adaptation, and pipelining frame structure are newly introduced features to further improve UE power saving. |
| 5.2.3.2.26.7 | Simulation process issues Describe the methodology used in the analytical approach. Proponent should provide information on the width of confidence intervals of user and system performance metrics of corresponding mean values, and evaluation groups are encouraged to provide this information as requested in § 7.1 of Report ITU-R M.2412-0. Related description is provided with the self-evaluation report in accordance with Report ITU-R M.2412-0. |
| 5.2.3.2.26.8 | Operational life time Describe the mechanisms to provide long operational life time for devices without recharge for at least massive machine type communications In order to lengthen battery life, DFT-s-OFDM, uplink power control, DRX in connected mode, configurable bandwidth for transmission/reception are provided. |
| 5.2.3.2.26.9 | Latency for infrequent small packet Describe the mechanisms to reduce the latency for infrequent small packet, which is, in a transfer of infrequent application layer small packets/messages, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the UE to the radio protocol layer 2/3 SDU egress point in the base station, when the UE starts from its most "battery efficient" state. The RIT supports the inactive state as a “battery efficient” state and a UE can maintain connections with base station in the state. So, the UE can transmit infrequent small packets without performing signalling for connection setup. |
| 5.2.3.2.26.10 | Control plane latency Provide additional information whether the RIT/SRIT can support a lower control plane latency (refer to § 4.7.2 in Report ITU-R M.2410-0). As described in the control plane latency evaluation in 3GPP TR37.910, if, in control plane procedure, the latency of step 7 and step 9 can be further reduced, the 10ms target as encouraged by ITU-R can be achieved in some cases. |
| 5.2.3.2.26.11 | Reliability Provide additional information whether the RIT/RSIT can support reliability for larger packet sizes (refer to § 4.10 in Report ITU-R M.2410-0). NR supports 32 Bytes packets transmission with reliability 99.999% within 1ms one-way latency in accordance with ITU-R requirements. NR, in addition, supports: - Reliability higher than 99.999% with packet duplication over two radio links (PDCP duplication). - Reliability of 99.999% within 1 ms one-way latency for larger packets (200 bytes) can be achieved [3GPP TR 38.824, Rel-15 enabled use case]. - Reliability equal or higher than 99.999% for packets larger than 32Bytes within more than 1 ms one-way latency [3GPP TR 38.824]. |
| 5.2.3.2.26.12 | Mobility Provide additional information for the downlink mobility performance of the RIT/SRIT (refer to § 4.11 in Report ITU-R M.2410-0). The intention of DMRS is for channel estimation. In order to support different downlink mobility environments, DM-RS occasions in a slot can be differently configured. In the case of low mobility conditions, front-loaded one DM-RS configuration per slot is beneficial. To combat rapid channel, up to four DM-RS occasions per slot can be configured. Also, TRS and PT-RS are supported for frequency tracking and phase noise compensation, respectively. |
| 5.2.3.2.27 | Other information Please provide any additional information that the proponent believes may be useful to the evaluation process. None |

______________

____________________