| Radiocommunication Study Groups |  |
| --- | --- |
|  | Document 5D/TBD-E |
|  | TECHNOLOGY ASPECTS |

ATIS

3GPP 5G CANDIDATE FOR INCLUSION IN IMT-2020:

SUBMISSION 2 FOR IMT-2020 (RIT)

In response to the ITU-R Circular Letter 5/LCCE/59 which invites proposals for candidate radio interface technologies for the terrestrial component of IMT-2020, the 3GPP PROPONENT joins with the Third Generation Partnership Project (3GPP) in providing a technology submission under Step 3 of the IMT-2020 process.

In this final view, intended to assist WP 5D and the evaluation groups in its preparation work on IMT-2020, the Proponent of the 3GPP submissions (hereafter known as the 3GPP Proponent) is providing at this time all the required components of the complete submissions, including the Description Template, Compliance Templates, and Self Evaluation Report, of its 5G solution:

3GPP 5G CANDIDATE SUBMISSION 2 consists of the following:

    - Submission 2:  RIT

        - NR

Each submission is self-contained and complete unto itself, consisting of Releases 15 & 16.

STATEMENT OF COMPLIANCE WITH THE PROVISIONS IN DOC. IMT-2020/2 REV 1, STEP 2

    - ITU-R Criteria:  An RIT needs to fulfil the minimum requirements for at least three test environments; two test environments under eMBB and one test environment under mMTC or URLLC.

- 3GPP Proponent Statement: The RIT fulfils this ITU-R Criteria as indicated below:

For the RIT (ITU-R Criteria: fufils at least 3 test environments 2 in eMBB, and 1 in either mMTC or URLLC)

TABLE 4

(source ITU-R Report M.2412)

Mapping of test environments and usage scenarios

| Usage Scenarios | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- |
| Test Environments | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |
| RIT | fulfils | Fulfils | fulfils | fulfils | fulfils |

This 3GPP Proponent submission for IMT-2020 is based upon the following currently in force ITU-R Reports, and the minimum technical requirements, evaluation criteria, and other information contained therein:

| ITU-R Report Number & Date | Title | Status |
| --- | --- | --- |
| M.2410-0 (11/2017) | Minimum requirements related to technical performance for IMT-2020 radio interface(s) | In force (Main) |
| M.2411-0 (11/2017) | Requirements, evaluation criteria and submission templates for the development of IMT-2020 | In force (Main) |
| M.2412-0 (11/2017) | Guidelines for evaluation of radio interface technologies for IMT-2020 | In force (Main) |

The 3GPP Submission 2 meets or exceeds the specific requirements, evaluation criteria, and/or other information specified in the ITU-R Reports.

Attachments:

- Part 1: RP-191529: Characteristics Template – RIT

- Part 2: RP-191530: Compliance Template - RIT

- Part 3: RP-191531: Link Budget Template - RIT

- Part 4: RP-191521: TR 37.910 Study on self evaluation towards IMT-2020 submission

This contribution was developed in 3GPP TSG RAN, approved by the Organizational Partners of the 3GPP PCG, and it is being submitted by ATIS on behalf of the 3GPP Proponent. Source: PCG43(19)07; (RP-191533, SP190576).

Developed by 3GPP as 5G, Release 15 and beyond.

The 3GPP Proponent of the 3GPP submission is collectively the 3GPP Organizational Partners (OPs). The Organizational Partners of 3GPP are ARIB, ATIS, CCSA, ETSI, TSDSI, TTA and TTC (http://www.3gpp.org/partners).

Source: RP-191528 for the 3GPP Submission 2 RIT to ITU-R.