| World Radiocommunication Conference (WRC-15) Geneva, 2–27 November 2015 |  |
| --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |
| PLENARY MEETING | Addendum 1 to Document 32(Add.6)-E |
|  | 29 September 2015 |
|  | Original: English |
| Asia-Pacific Telecommunity Common Proposals |  |
| Proposals for the work of the conference |  |
| Agenda item 1.6.1 |  |

1.6	to consider possible additional primary allocations:

1.6.1	to the fixed-satellite service (Earth-to-space and space-to-Earth) of 250 MHz in the range between 10 GHz and 17 GHz in Region 1;

and review the regulatory provisions on the current allocations to the fixed-satellite service within each range, taking into account the results of ITU-R studies, in accordance with Resolutions 151 (WRC-12) and 152 (WRC-12), respectively;

Introduction

APT Members support no change (NOC) method in the frequency bands 10-10.68 GHz, 13.25-13.4 and 14.8-17 GHz due to incompatibility with existing services.

APT Members do not support the additional allocation to the FSS (Earth-to-space) in Region 1 in the frequency band 13.4-13.75 GHz due to incompatibility with existing services.

APT Members do not support the additional allocation to the FSS (space-to-Earth) in Region 1 in the frequency band 14.5-14.8 GHz due to incompatibility with existing services.

APT Members support to make an additional allocation of 250 MHz to the FSS (space-to-Earth) in Region 1 in the frequency band 13.4-13.65 GHz.

Proposals

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

NOC	ASP/32A6A1/1

10-11.7 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 10-10.45 FIXED MOBILE RADIOLOCATION Amateur | 10-10.45 RADIOLOCATION Amateur | 10-10.45 FIXED MOBILE RADIOLOCATION Amateur |
| 5.479 | 5.479 5.480 | 5.479 |
| 10.45-10.5 RADIOLOCATION Amateur Amateur-satellite 5.481 |  |  |
| 10.5-10.55 FIXED MOBILE Radiolocation | 10.5-10.55 FIXED MOBILE RADIOLOCATION |  |
| 10.55-10.6 FIXED MOBILE except aeronautical mobile Radiolocation |  |  |
| 10.6-10.68 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation 5.149 5.482 5.482A |  |  |

Reasons:	No change in the band 10-10.68 GHz due to incompatibility with existing services.

NOC	ASP/32A6A1/2

11.7-14 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 13.25-13.4 EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION 5.497 SPACE RESEARCH (active) 5.498A 5.499 |  |  |

Reasons:	No change in the band 13.25-13.4 GHz due to incompatibility with existing services.

NOC	ASP/32A6A1/3

11.7-14 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 13.4-13.75 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.501A Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |  |

Reasons:	No additional allocation to the FSS (Earth-to-space) is proposed in the band 13.4-13.75 GHz for Region 1 due to incompatibility with existing services.

MOD	ASP/32A6A1/4

11.7-14 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 |  | Region 2 |  | Region 3 |
| 13.4-13.65 EARTH EXPLORATION-SATELLITE (active) FIXED-SATELLITE (space-to-Earth) ADD 5.A161 ADD 5.B161 ADD 5.C161 RADIOLOCATION SPACE RESEARCH ADD 5.D161 Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |  | 13.4-13.65 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH ADD 5.D161 Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |
| 13.65-13.75 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH MOD 5.501A Standard frequency and time signal-satellite (Earth-to-space) 5.499 5.500 5.501 5.501B |  |  |

Reasons:	To allocate the band 13.4-13.65 GHz to the FSS (space-to-Earth) in Region 1.

ADD	ASP/32A6A1/5

5.A161	The use of the band 13.4-13.65 GHz by the fixed-satellite service (space-to-Earth) is limited to geostationary-satellite systems and is subject to agreement obtained under No. 9.21 with respect to satellite systems, operating in the space research service (space-to-space) to relay data from space stations in the geostationary satellite orbit to associated space stations in the non-geostationary satellite orbit, for which information for advance publication has been received by the Bureau prior to 27 November 2015.     (WRC-15)

Reasons:	To limit use of the new FSS allocation (space-to-Earth) in Region 1 to GSO FSS, and to specify the terms and conditions for sharing between newly filed GSO FSS networks and SRS systems already notified to the Bureau, operating on space-to-space link to relay data from GSO space station to non-GSO user space station. There is understanding, that coordination of newly filed GSO FSS networks and already notified to the Bureau SRS (space-to-Earth) systems is subject to RR No. 9.7.

ADD	ASP/32A6A1/6

5.D161	The allocation of the band 13.4-13.65 GHz to the space research service on a primary basis is limited to active spaceborne sensors, as well as satellite systems, operating in the space research service (space-to-Earth and space-to-space) to relay data from space stations in the geostationary-satellite orbit to associated earth stations and space stations in the non-geostationary-satellite orbit, for which information for advance publication has been received by the Bureau prior to 27 November 2015. Satellite systems in the space research service (space-to-Earth and space-to-space) shall not cause harmful interference to nor claim protection from stations in the fixed, mobile, radiolocation and Earth exploration-satellite (active) services. Other uses of the band by the space research service are on a secondary basis.     (WRC-15)

Reasons:	Since only the frequency assignments having allocation of the considered frequency band on an equal basis are taken into account in the coordination under RR Article 9 it is proposed to modify footnote No. 5.501А and to add a new footnote under which the status of the notified frequency assignments of DRS in SRS (s-E and s-s) will be increased up to the primary with regard to FSS. With respect to FSS stations in Region 1 in any case it is required to seek the agreement of other administrations (under RR No. 9.21) operating DRS in SRS (space-to-space) in Region 1, with NGSO user which can be potentially located over the territories of Regions 2 and 3. The direction of the DRS SRS links (space-to-Earth and space-to-space) is defined by the relevant Recommendations. Therefore it is not specified in RR Article 5 footnotes.

ADD	ASP/32A6A1/7

5.B161	Administrations shall not preclude the deployment and operation of transmitting earth stations in the standard frequency and time signal-satellite (Earth-to-space) allocated on a secondary basis in the band 13.4-13.65 GHz, due to the primary allocation to FSS (space-to-Earth).     (WRC-15)

Reasons:	To ensure the deployment of transmitting earth stations for the European ACES system in the band 13.4-13.75 GHz operating under the standard frequency and time signal-satellite.

ADD	ASP/32A6A1/8

5.C161	In the band 13.4-13.65 GHz, geostationary-satellite networks in the fixed-satellite service (space-to-Earth) shall not claim protection from space stations in the Earth exploration-satellite service (active) operating in accordance with these Regulations. Nos. 5.43A and 22.2 do not apply.     (WRC-15)

MOD	ASP/32A6A1/9

5.501A	The allocation of the band 13.65-13.75 GHz to the space research service on a primary basis is limited to active spaceborne sensors. Other uses of the band by the space research service are on a secondary basis.     (WRC-15)

NOC	ASP/32A6A1/10

14-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.5-14.8 FIXED FIXED-SATELLITE (Earth-to-space) 5.510 MOBILE Space research |  |  |

Reasons:	No additional allocation to the FSS (space-to-Earth) is proposed in the band 14.5-14.8 GHz for Region 1 due to incompatibility with existing services.

NOC	ASP/32A6A1/11

14-15.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE Space research 5.339 |  |  |
| 15.35-15.4 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.511 |  |  |

Reasons:	No change in the band 14.8-15.4GHz due to incompatibility with existing services.

NOC	ASP/32A6A1/12

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 RADIOLOCATION 5.511E 5.511F AERONAUTICAL RADIONAVIGATION 5.511D |  |  |
| 15.43-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A RADIOLOCATION 5.511E 5.511F AERONAUTICAL RADIONAVIGATION 5.511C |  |  |
| 15.63-15.7 RADIOLOCATION 5.511E 5.511F AERONAUTICAL RADIONAVIGATION 5.511D |  |  |
| 15.7-16.6 RADIOLOCATION 5.512 5.513 |  |  |
| 16.6-17.1 RADIOLOCATION Space research (deep space) (Earth-to-space) 5.512 5.513 |  |  |

Reasons:	No change in the band 15.4-17 GHz due to incompatibility with existing services.

ARTICLE 21

Terrestrial and space services sharing frequency bands above 1 GHz

Section I − Choice of sites and frequencies

MOD	ASP/32A6A1/13

_______________

1 	21.2.1	For their own protection receiving stations in the fixed or mobile service operating in bands shared with space radiocommunication services (space-to-Earth) should also avoid directing their antennas towards the geostationary-satellite orbit if their sensitivity is sufficiently high that interference from space station transmissions may be significant. In particular, in the bands 13.4-13.65 GHz and 21.4-22 GHz, it is recommended to maintain a minimum separation angle of 1.5 with respect to the direction of the geostationary-satellite orbit.    (WRC-15)

Section V − Limits of power flux-density from space stations

MOD	ASP/32A6A1/14

TABLE  21-4  (continued)     (Rev.WRC-15)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° |  | 5°-25° |  |  | 25°-90° |  |  |
| 12.2-12.75 GHz 7 (Region 3) 12.5-12.75 GHz 7 (Region 1 countries listed in Nos. 5.494 and 5.496) | Fixed-satellite (space-to-Earth) (geostationary-satellite orbit) | −148 |  | −148 + 0.5(δ − 5) |  |  | −138 |  | 4 kHz |
| 13.4-13.65 GHz (Region 1) | Fixed-satellite (space-to-Earth) (geostationary-satellite orbit) | 0°-0.6° | 0.6°-1.25° |  | 1.25°-21.25° | 21.25°-70° |  | 70°-90° | 1 MHz |
|  |  | −137.5 | −136.5 |  | −130.5 | −127.5 |  | [−122*] |  |

_______________

*	The references to services are those services which have allocations in Article 5.

Editor’s Note: A suitable maximum pfd limit value of the FSS downlink maybe needed in order to protect the EESS(active). This value could be decided in the WRC-15.

Reasons:	To insert pfd limits for GSO FSS (space-to-Earth) into RR Article 21 in order to protect allocations to terrestrial services (FS, MS), RLS and EESS (active).

APPENDIX 5 (REV.WRC-12)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	ASP/32A6A1/15

TABLE 5-1     (Rev.WRC-15)

Technical conditions for coordination

(see Article 9)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz 7 025-7 075 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  | With respect to the space services listed in the threshold/condition column in the bands in 1), 2), 3), 4), 5), 6), 7) and 8), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.5 GHz 2bis) 13.4-13.65 GHz (Region 1) | i) Bandwidth overlap, and ii) any network in the FSS or broadcasting-satellite service (BSS), not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 7° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan i) Bandwidth overlap, and ii) any network in the space research service (SRS) or any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 7° of the nominal orbital position of a proposed network in the FSS |  |  |

Reasons:	To specify the order and mechanism of coordination in accordance with provisions of RR No 9.7 between newly notifying networks of the FSS and SRS (space-to-Earth).

MOD	ASP/32A6A1/16

TABLE 5-1 (end)     (Rev.WRC-12)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.21 Terrestrial, GSO, non-GSO/ terrestrial, GSO, non-GSO | A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations referring to No. 9.21 | Band(s) indicated in the relevant footnote except 13.4-13.65 GHz in Region 1 13.4-13.65 GHz in Region 1 | Incompatibility established by the use of Appendices 7, 8, technical Annexes of Appendices 30 or 30A, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU-R Recommendations, as appropriate Any network in the space research service (SRS) within an orbital arc of ±[24]° of the nominal orbital position of a proposed network in the FSS | Methods specified in, or adapted from, Appendices 7, 8, 30, 30A, other technical provisions of the Radio Regulations or ITU-R Recommendations |  |

Reasons:	To define the procedure for coordination under the provisions of RR No. 9.21 between the newly notified FSS networks and SRS networks.

APPENDIX 7 (REV.WRC-12)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

# 3	Horizon antenna gain for a receiving earth station with respect to a transmitting earth station

MOD	ASP/32A6A1/17

TABLE 8c    (Rev.WRC-15)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteorological-satellite 7, 8 | Meteorological-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Fixed- satellite 9 | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 13.4-13.657 |  | 12.5-12.75 12 |  | 15.4-15.7 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Aeronau-tical radio-navigation | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  |  | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | – |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.003 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.0015 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 4 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 |  |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | −18 | −18 | 43 | 43 | 42 | 42 |  | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | −17 5 | −17 5 | −5 | −5 | 10 | 10 |  |  | −10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −60 | −60 | −2 | −2 | −3 | −3 |  | −7 | −5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 |  | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27 × 106 | 27 × 106 |  |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | −151.2 |  |  | −125 | −125 | −154 11 | −142 | −220 | −216 |  |  | −131 | −131 |  |  |  |

Notes to Table 8c:

1	A: analogue modulation; N: digital modulation.

2	E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth.

3	In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area.

4	Digital systems assumed to be non-transhorizon. Therefore Gx = 42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used.

5	These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission.

6	In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station.

7	Geostationary-satellite systems.

8	Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters.

9	Non-geostationary-satellite systems.

10	Space research earth stations in the band 8.4-8.5 GHz operate with non-geostationary satellites.

11	For large earth stations:		Pr(p) = (G − 180) 		dBW

For small earth stations:		Pr(20%) = 2 (G − 26) − 140	dBW 	for  26 < G  29 dBi

Pr(20%) = G − 163		dBW 	for        G  29 dBi

Pr(p)% = G − 163 		dBW 	for        G  26 dBi

12	Applies to the broadcasting-satellite service in unplanned bands in Region 3.

Reasons:	To specify coordination distances for the FSS receiving earth station in order to protect it from interferences produced by terrestrial FS and MS stations, based on the allowable interference criterion I/N = 6%, see Recommendation ITU-R S.1432.

SUP	ASP/32A6A1/18

RESOLUTION 151 (WRC-12)

Additional primary allocations to the fixed-satellite service 
in frequency bands between 10 and 17 GHz in Region 1

______________

_______________