| World Radiocommunication Conference (WRC-19) Sharm el-Sheikh, Egypt, 28 October – 22 November 2019 |  |
| --- | --- |
| PLENARY MEETING | Addendum 13 to Document 16-E |
|  | 4 October 2019 |
|  | Original: English |
| European Common Proposals |  |
| Proposals for the work of the conference |  |
| Agenda item 1.13 |  |

1.13	to consider identification of frequency bands for the future development of International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 238 (WRC-15);

Resolution 238 (WRC-15) – Studies on frequency-related matters for International Mobile Telecommunications identification including possible additional allocations to the mobile services on a primary basis in portion(s) of the frequency range between 24.25 and 86 GHz for the future development of International Mobile Telecommunications for 2020 and beyond

Introduction

WRC-19 agenda item 1.13 considers possible new spectrum allocations suitable for delivery of terrestrial wireless broadband in the 24.25-86 GHz frequency range. This will encompass the following elements, set out in full in Resolution 238 (WRC-15):

–	Spectrum needs for the terrestrial component of IMT

–	Sharing and compatibility studies for the following frequency bands:

–	24.25-27.5 GHz, 37-40.5 GHz, 42.5-43.5 GHz, 45.5-47 GHz, 47.2-50.2 GHz, 50.4-52.6 GHz, 66-76 GHz and 81-86 GHz

–	31.8-33.4 GHz, 40.5-42.5 GHz and 47-47.2 GHz

Summary of European proposals on the frequency bands:

These European proposals support the frequency bands 24.25-27.5 GHz, 40.5-43.5 GHz and 66-71 GHz for allocation to the mobile service on a primary basis in the table of frequency allocation of Article 5 and identification for IMT, as appropriate.

These European proposals do not support the frequency bands 31.8-33.4 GHz, 47.0-47.2 GHz, 47.2-50.2 GHz, 50.4-52.6 GHz, 71-76 GHz and 81-86 GHz for allocation to the mobile service on a primary basis in the table of frequency allocation of Article 5 and identification for IMT, as appropriate.

For the band 37-40.5 GHz, the European proposal in Part 8 provides CEPT’s view on the band including the relevant conditions to ensure protection of incumbent services in the 37-40.5 GHz band and EESS (passive) in the 36-37 GHz band.

The suppression of Resolution 238 (WRC-15) is supported.

The European proposals summarized above are divided into the following addenda:

Addendum 1 	Part 1 – Band 24.25-27.5 GHz

Addendum 2 	Part 2 – Band 31.8-33.4 GHz

Addendum 3	Part 3 – Band 37-40.5 GHz

Addendum 4	Part 4 – Band 40.5-43.5 GHz

Addendum 5	Part 5 – Band 47.0-47.2 GHz

Addendum 6	Part 6 – Band 47.2-50.2 GHz

Addendum 7	Part 7 – Band 50.4-52.6 GHz

Addendum 8	Part 8 – Band 66-71 GHz

Addendum 9 	Part 9 – Band 71-76 GHz

Addendum 10 	Part 10 – Band 81-86 GHz

Addendum 11	Part 11 – Resolution 238 (WRC-15)

______________

_______________

Including studies with respect to services in adjacent bands, as appropriate.

When conducting studies in the band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz.