| World Radiocommunication Conference (WRC-19) Sharm el-Sheikh, Egypt, 28 October – 22 November 2019 |  |
| --- | --- |
| PLENARY MEETING | Addendum 10 to Document 16(Add.13)-E |
|  | 4 October 2019 |
|  | Original: English |
| European Common Proposals |  |
| Proposals for the work of the conference |  |
| Agenda item 1.13 |  |

1.13	to consider identification of frequency bands for the future development of International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 238 (WRC-15);

Part 10 – Frequency band 81-86 GHz

Introduction

This Addendum presents the European Common Proposal for the frequency band 81-86 GHz under WRC-19 agenda item 1.13.

Proposal

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

NOC	EUR/16A13A10/1

81-86 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 81-84 FIXED 5.338A FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) 5.149 5.561A |  |  |
| 84-86 FIXED 5.338A FIXED-SATELLITE (Earth-to-space) 5.561B MOBILE RADIO ASTRONOMY 5.149 |  |  |

Reasons:	The 81-86 GHz frequency band, paired with 71-76 GHz is a fixed link frequency band important for backhauling of 5G. Therefore fixed link usage is expected to increase in the future. Studies have shown that the current IMT-2020 unwanted emissions levels would be insufficient to ensure protection of the EESS (passive) sensors in the 86-92 GHz frequency band and that only a reduction of the IMT-2020 emissions in this band can ensure such protection. Some studies have also shown that the unwanted emissions of both the base station (BS) and user equipment (UE) IMT-2020 would need to be limited to protect automotive radars operating in the 76-81 GHz frequency band. These constraints make the frequency band 81-86 GHz not suitable for IMT.

________________

_______________