| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Documents 5B/TEMP/20, 5B/712 (Annex 13), 5B/62, 5B/79 | Annex 8 to Document 5B/93-E |
|  | 17 August 2020 |
|  | English only |
| Annex 8 to the Working Party 5B Chairman’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[FOD 92-100 GHz] |  |
| Technical and operational characteristics of the foreign object debris detection system operating in the frequency range 92-100 GHz |  |

Scope

This Report provides the technical and operational characteristics of foreign object debris (FOD) detection system operating in the frequency range 92-100 GHz for sharing and compatibility studies between FOD detection system and passive services, as well as among active services.

Keywords

Related ITU Recommendations and Reports

Recommendation ITU-R F.1332 – Radio-frequency signal transport through optical fibres

Report ITU-R F.2239 – Coexistence between fixed service operating in 71-76 GHz, 81-86 GHz and 92-94 GHz bands and passive services

Glossary / abbreviations

dBsm	decibels per square metre

FFT	Fast Fourier transform

EESS	Earth exploration satellite service

FMCW	Frequency modulated continuous wave

FOD	Foreign object debris

IF	Intermediate frequency

PPI	Plan position indicator

SNR	Signal to noise ratio

UTM KL	Universiti Teknologi Malaysia Kuala Lumpur

# 1	Introduction

Foreign object debris (FOD) defined as, “Any object located in an inappropriate location in the airport environment that has the capacity to injure airport or airline personnel and damage aircraft” [1] can cause damage to aircraft as well as injury airport employees. Since FOD directly costs the global aviation industry $1.26 billion annually and indirect costs, such as flight delays, cost the global aviation industry $13.9 billion annually [2], new technologies for FOD have been developed to continuously inspect for and detect FOD. The guidance and specifications for procuring airport FOD detection equipment was published in 2009 in which four types of detection systems are addressed [1]. Millimetre-wave FOD radars operating in the frequency band 78-81 GHz which can provide high detection sensitivity, short detection response time, sufficient coverage of surveillance area and high location accuracy have been well designed and tested and commercialized for safety airport operation.

FOD Hazards can severely injure airport or airline personnel or damage equipment. Types of potential damage include [1]: cutting aircraft tires; being ingested into engines; or becoming lodged in mechanisms affecting flight operations. Personnel injuries can occur when jet blast propels FOD through the airport environment at high velocities.

Dark-coloured items made up nearly 50% of the FOD collected. Common FOD dimensions are 3 cm by 3 cm or smaller. Typical FOD includes the following:

–	aircraft and engine fasteners (nuts, bolts, washers, safety wire, etc.);

–	aircraft parts (fuel caps, landing gear fragments, oil sticks, metal sheets, trapdoors, and tire fragments);

–	mechanics' tools;

–	catering supplies;

–	flight line items (nails, personnel badges, pens, pencils, luggage tags, soda cans, etc.);

–	apron items (paper and plastic debris from catering and freight pallets, luggage parts, and debris from ramp equipment);

–	runway and taxiway materials (concrete and asphalt chunks, rubber joint materials, and paint chips);

–	construction debris (pieces of wood, stones, fasteners and miscellaneous metal objects);

–	plastic and/or polyethylene materials;

–	natural materials (plant fragments and wildlife); and

–	contaminants from winter conditions (snow, ice).

With respect to the frequency allocation of millimetre-wave frequencies, the frequency bands 92-94 GHz, 94-94.1 GHz, 94.1-95 GHz, 95-100 GHz are already allocated to the radiolocation service on a primary basis. The wideband spectrum at these frequency bands can be applied for ground-based aeronautical radar applications, but a few FOD detection systems are demonstrated using a limited bandwidth of 94-GHz band. To facilitate high performance FOD detection systems which utilize such wideband spectrum resource, the technical and operational characteristics of FOD detection systems operating in the frequency range 92-100 GHz are provided in this Report.

# 2	System overview of foreign object debris detection system operating in the frequency range 92-100 GHz

Figure 1 illustrates the concept of the FOD detection system for runways. To balance the radar performance improvement and the system cost reduction, the high-accuracy and high-stability frequency modulated continuous wave (FMCW) radar signals are generated and up-converted to optical signals at the airport operation centre and transmitted through the optical fibre to the multiple FOD radars placed along the runway area.

FIGURE 1

Schematic illustration of foreign object debris detection system in runway at airport

Figure 2 shows the overall configuration of FOD detection system which consists of the airport operation centre and FOD radars. The FOD radars along the runway are connected to the airport operation centre through the optical fibre cables which are already installed in the runway area. The electrical FWCW signals are converted into the optical FMCW signals using the optical source at the airport operation centre.  The optical FWCM signals are optically amplified, demultiplexed by the optical divider and distributed to FOD radars through optical fibre cables. The optical FMCW signals are received by each FOD radar and they are converted into the electrical FMCW signals and radiated into the air. Then the FMCW signals reflected by the obstacles in the runway are received and down converted into IF signals. The IF signals are converted into the digital signals by A/D converter. The airport operation centre receives the digital signals from each FOD radar and these digital signals are processed by fast Fourier transform (FFT) technology. FFT processing signals are displayed and stored in the server.

FIGURE 2

Block diagram of foreign object debris detection system which consists of 
airport operation centre and foreign object detection radars

# 3	System characteristics

## 3.1	Overview of foreign object debris detection system characteristics operating in the frequency range 92-100 GHz

The technical and operational characteristics of FOD detection system is summarized in Table 1. Figure 3 shows the channel plan for FOD radars which utilize the frequency bands allocated for radiolocation services in the range of 92-100 GHz. However, the frequency band 94-94.1 GHz is also allocated for Earth exploration satellite (active) and space research (active) services. The channel plans which exclude the use of the band 94-94.1 GHz are also provided in Figure 3, where a guard band of 10 MHz is inserted in the lower frequency of 94 GHz and in the upper frequency of 94.1 GHz. The single channel operation of FOD radars may cause the frequency interference between FOD radars installed at the different runway. The FOD radar using multichannel plan may be introduced for the airport with several runways to provide the interference free operation. Table 2 summarizes the channel plan, channel number, channel bandwidth, range resolution and their frequency bands. Since the range resolution depends on the operational bandwidth, the airport operator may deploy FOD radars with different performance depending on the requirement with respect to obstacle detection.

TABLE 1

Technical and operational characteristics of foreign object debris 
detection system operating in the frequency range 92-100 GHz

| Parameters | Units | Values |
| --- | --- | --- |
| Frequency range | GHz | 92…..100 |
| Channel plan |  | See Figure 3 |
| Channel bandwidth | GHz | 0.58…..7.98 |
| Output power | mW | 200 |
| Spectrum envelope |  | See Annex 2 |
| Sweep frequency | kHz | 1.250 |
| Antenna type |  | Cassegrain |
| Antenna gain | dBi | 42 |
| Antenna pattern |  | See Figure 4 Recommendation ITU-R M.1851 |
| Antenna height | m | 6….8 |
| Full width at half maximum antenna gain (3 dB beamwidth) | degrees | Elevation: 1.0, Azimuth: 1.0 |
| Antenna rotation speed | rpm | 15 |
| Detection distance | m | 200 |
| Antenna elevation angle | degrees | –1.8 (see Annex 3) |
| Radiated rotation angle in azimuth | degrees | ±60 |
| Radar cross section specification | dBsm* | –20 |
| Range resolution | cm | 3…..50 |
| Emission bandwidth (–3 dB) | MHz | 1 (see Annex 2) |
| Emission bandwidth (–20 dB) | MHz | 3.5 (see Annex 2) |
| Adjacent channel leakage ratio | dBc | < −70 |
| Receiver noise figure | dB | 10 |
| I/N protection criteria | dB | [−6 or −10] |
| * dBsm = decibels per square metre. |  |  |

FIGURE 3

Channel plan for foreign object debris detection system operating in the frequency range 92-100 GHz

TABLE 2

Channel number, channel bandwidth, range resolution 
and frequency bands of each channel plan

| Channel Plan | Channel Number | Channel Bandwidth (GHz) | Range Resolution (cm) | Frequency Band (GHz) |
| --- | --- | --- | --- | --- |
| A | 1 | 7.98 |  | 92.01-99.99 |
| B | 2 | 1.98, 5.98 | 4.5, 12 | B1: 92.01-93.99, B2: 94.11-99.99 |
| C | 4 | 1.98 | 12 | C1: 92.01-93.99, C2: 94.11-95.99, C3: 96.01-97.99, C4: 98.01-99.99 |
| D | 8 | 0.98 | 24 | D1: 92.01-92.99, D2: 93.01-93.99, D3: 94.11-94.99, D4: 95.01-95.99, D5: 96.01-96.99, D6: 97.01-97.99, D7: 98.01-98.99, D8: 99.01-99.99 |
| E | 12 | 0.58 | 50 | E1: 92.01-92.59, E2: 92.71-93.29, E3: 93.41-93.99, E4: 94.11-94.69, E5: 94.71-95.29, E6: 95.31-95.89, E7: 95.91-96.49, E8: 96.51-97.09, E9: 97.11-97.69, E10: 97.71-98.29, E11: 98.51-99.09, E12: 99.11-99.69 |

## 3.2	Performance of foreign object debris transceiver

Figure 4 shows the configuration of FOD radar which is connected through the radio over fibre links to the airport operation centre. To improve the receiving sensitivity, the Bi-Static type configuration is adopted. The optical FMCW signal whose subcarrier frequency is 30 GHz from the airport operation centre is converted to the electrical FMCW signal by photo detector, and the 30-GHz FMCW signal is amplified, tripled, again amplified and then radiated to the air. The radiated FMCW signal from Tx unit is reflected if the target object is on the runway and RX unit finally receives reflection signal. The received signal is down converted to the IF signal using the 90-GHz FMCW signal from the airport operation centre, and the IF signal is converted to digital signal by the A/D converter and transmitted to the airport operation centre and to process information on locations of the target objects. The measured spectrum envelope of FOD radar and CW spectrum and the adjacent channel leakage ratio at an offset frequency of 10 MHz are shown in Annex 2. The emission bandwidth of CW is obtained from Figure A2-2.

FIGURE 4

Block diagram of foreign object debris transceiver

## 3.3	Antenna characteristics at 90-GHz

Figure 5 shows the antenna pattern at 96-GHz. The antenna gain is about 42 dBi. The antenna whose height is about 8 m is mechanically rotated. The measured data at 92 GHz, 94 GHz, 96 GHz, 98 GHz and 100 GHz is provided by the following table.

FIGURE 5

Antenna radiation pattern of 42 dBi at 96-GHz

# 4	Summary

# 5	Bibliographies

[1]	Airport Foreign Object Debris (FOD) Detection Equipment, FAA Advisory Circular (AC) 150/5220-24, “Airport Foreign Object Debris (FOD) Detection Equipment” 30 September 2009.

[2]	Current Airport Inspection Practices Regarding FOD (Foreign Object Debris/Damage), The National Academies PRESS, http://nap.edu/14572.

Annex 1

Field trial of foreign object debris detection system
at Narita Airport in Japan

To demonstrate the fundamental characteristics of FOD detection system at the airport, four FOD units are installed along the runway B and one control unit near the control tower of Narita International Airport, as shown in Figures A1-1. Figure A1-2 shows the location of the four FOD units. The layout plan including the location of existing facilities is decided to reduce the installation cost of FOD detection system. The exiting power supply facilities and optical fibre infrastructure at the airport are utilized to perform the total cost reduction of the system.

Figure A1-1

Field trial at Narita airport

FIGURE A1-2

Foreign object debris unit layout at Narita airport runway B

Figure A1-3 shows the radar plan position indicator (PPI) scope obtained from the four radar units. Reflections from boarder areas between the runway and the grass are measured by FOD units. A maximum distance measured by one FOD unit is about 500 m. By installing multiple FOD units in the runway area, as shown in Figure A1-2, it is possible to expand the detection area. The detection signal at each FOD unit is sent to the control tower and those signals are simultaneously processed to achieve larger-area detection.

Figure A1-3

Radar plan position indicator scope

| (a) Unit A | (b) Unit B |
| --- | --- |
| (c) Unit D | (d) Unit E |

Figure A1-4 shows the measured PPI scope of FOD unit E when there are various objects such as radar cross section of 30 dBsm and 5 dBsm reflectors, that of 0 dBsm and –20 dBsm metallic cylinders and human beings on the runway. The experimental result show that these objects are clearly discriminated.

Figure A1-4

Radar plan position indicator scope of foreign object debris unit E when there are various objects on the runway

Figure A1-5 shows the measured result of a moving −20 dBsm metallic cylinder object on the runway of Narita International Airport. The spectrum at around 426 m from FOD unit is measured by reflection form the runway light. Signal to noise ratio (SNR) above 15 dB is obtained when a 
−20 dBsm metallic cylinder is placed at 442 m which is far from FOD unit. The SNR of FOD unit is not deteriorated in the range of up to 500 m.

Figure A1-5

Measured spectrum of a moving –20 dBsm metallic cylinder object

Figure A1-6

Detection level of −10 dBsm and −20 dBsm foreign objects

Annex 2

Measured spectrum envelope of foreign object debris detection system

The spectrum envelope in the frequency range 89.5-99.5 GHz is shown in Figure A2-1. The centre frequency is 94.5 GHz. CW spectrum of FMCW is also measured in the condition of RBW=1 MHz and VBW=1 MHz, and one example CW spectrum whose frequency is 93.4 GHz is shown in Figure A2-2.  The adjacent channel leakage ratio is −74.6 dBc at an offset frequency of 10 MHz which can be used for interference evaluation to Earth exploration satellite service (EESS) (active), EESS (passive) and radio astronomy service. The emission bandwidths of CW which are 3 dB and 10 dB below from the peak value at the centre frequency of 93.4 GHz are 1 MHz and 3.5 MHz, respectively.

Figure A2-1

Spectrum envelope of frequency modulated continuous wave signal  in the frequency band 92-97 GHz

Figure A2-2

Adjacent channel leakage ratio at an offset frequency of 10 MHz

Annex 3

Relationship between antenna elevation angle and incident beam angle
to foreign object debris on runway

Annex 4

Field trial of foreign object debris detection system
at Universiti Teknologi Malaysia and Kuala Lumpur International Airport

For reliable and secure installation of the radar system, pre-evaluation of the portable radar system has been performed and evaluated under tropical weather conditions. The radar system evaluation has been conducted at Kuala Lumpur International Airport and Universiti Teknologi Malaysia Kuala Lumpur (UTM KL) in Malaysia, as shown in Figure A4-1. Other equipment used in the radar system evaluation are a weather logger, GPS logger, laser range meter and a portable personal computer.

FIGURE A4-1

Radar system evaluation in Malaysia, at Universiti Teknologi Malaysia campus (left) and 
Kuala Lumpur International Airport (right), Malaysia

a.	Experimental work at Universiti Teknologi Malaysia Kuala Lumpur Campus

To evaluate in the real conditions, we tested the radar system in the field at UTM KL as depicted in Figure A4-2. The radar system was set at a courtyard of the campus building with heading to a football field. The obtained range spectrum is also shown in the figure. A broad peak observed at 24-m and 85-m indicators are caused by oscillation noises of an IF amplifier set after the mixer. These stable noise components can be mitigated by post processing. Sharp peaks at 10 m and around 50 m are reflected signal from a sign pole, football goal posts and rugby goal posts in the grass field. Strong peak at 87 m is provided by a barrier wall between the grass field and campus. This barrier wall is configured by mesh wires with a radius less than 1 cm with a mesh pitch of 15 cm×8 cm. Even the radius is just 2–3 times larger than the wavelength of the radar signal (approximately 3 mm), clear reflection is observed. It might be caused by the large cross section area of the mesh barrier.

The PPI image is also shown in Figure A4-3, where the mesh barriers clearly observed along with the grass field. Two-dimensional PPI image helps identifying the objects and their locations. Polarization imaging is also shown in the same figure. In general, the polarizations of the Tx and Rx antennas should be same for increase of the received signal intensity. When the Tx antenna polarization is set perpendicular to the Rx polarization, the differential reflectivity coefficients, which are caused by surface structures such as a direction of the vertical surface of the target, might be obtained. Figure 5 also shows the polarization differential image, as calculated by subtraction between the perpendicular and parallel-polarization PPI images. The barrier walls have strong reflection with perpendicular polarization to the incident radar signals. This polarization rotation could be caused by the configuration of the barrier.

FIGURE A4-2

Location for foreign object debris detection systems measurements conducted at 
Universiti Teknologi Malaysia Kuala Lumpur, Malaysia

FIGURE A4-3

Plan position indicator images under polarization of transmitter and receiver set (top) as parallel and (bottom) as perpendicular. Photo of the barrier wall is also shown. The measurement conducted at Universiti Teknologi Malaysia Kuala Lumpur, Malaysia

150

m

300

m

Barrier wall

Football goal posts

Sign pole

m

150

300

m

There are four parameters that need to be considered when computing the radar detection area, namely the antenna beamwidth, elevation angle (θ), antenna height (h) and distance (R). The elevation angle can be varied accordingly for adjusting the length of the detection area. R corresponds to the distance to the point on the radar detection area that is projected at the elevation angle, where. Using a 300 cylinder 50m away from the RAU, the measured beamwidth of the Cassegrain antenna is 2 degrees, as shown in Figure A4-4. The value of the beamwidth is important for determining the diameter of the radar detection area, where the diameter of the radar detection area will increase as the beamwidth is increased. The range resolution of the tested RAU is approximately between 15 to 20cm for two 5dBsm reflectors that are placed apart. The received power was measured when the two reflectors are separated by various separation distance from the centre of the antenna beamwidth.

FIGURE A4-4

Measured RAU beamwidth

b.	Experimental Work at Kuala Lumpur International Airport

To evaluate this radar system in airport conditions, we tested the radar system at an apron area in the Kuala Lumpur International Airport as shown in Figure A4-5. Since the surface of the apron area is flattened and covered by concrete, as compared to the UTM KL field, the clutter signal from the grass and ground might be significantly smaller. Figure A4-6 shows observed IF spectra of received signals with two FODs: 3” cylinder at a distance of 55 m and a −10-dBsm reflector set at a distance of 105 m. Small fluctuation of noise floor directly reflects the flatness of the airport surface. When a height of the radar head (antenna position) is set 0.8 m, the reflected power from the cylinder is approximately −70 dBm. On the other hand, the signal from the reflector becomes large at the height of 1.4 m although the signal from the cylinder is degraded. This shows the surface has a concave structure with some gradient, which cannot be felt by the eye. The top of the concave blocks the signal to the reflector when the antenna height is short.

The antenna height increases the reflection strength from the reflector; however, the signal from the cylinder decreases because the small beam divergence (approximately 1 degree with 35-dBi antenna gain) may not reach near target. This result indicates that the antenna height in the airport situation is a key to detect small FODs. In airport condition, the maximum range R of the 1mW prototype RAU is more than 200m, as shown in Figure A4-6, for a 300 cylinder. This can further be extended by having a higher gain antenna or reducing the noise floor. 1mW-output system limits the maximum range of the radar, however, optimization of antenna/amplifier will realize the coverage of more than 300m. The prototype RAU was able to detect FOD samples of various sizes, shapes and material, as illustrated in Figure 8, where the reflected power of the various FOD samples are normalized by a 100 cylinder with radar cross section of −20dBsm at 95GHz. Under 20mm/h rainfall, it was observed that some scattering appears in the reading due to rain puddles around the sample FOD, as shown in Figure A4-7.

FIGURE 4-5

Location for foreign object debris detection systems measurement conducted at 
Kuala Lumpur International Airport, Malaysia

FIGURE A4-6

Obtained IF spectra in radar height at (left) 0.8 m and (right)1.4 m. Schematic of targets and radar position in the concave apron structure is also shown in the bottom. The measurement conducted at 
Kuala Lumpur international airport, Malaysia

Figure A4-7

Measured RAU range

Figure A4-8

Rain scattering effect

_________________

____________________