| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Doc. 5C/TEMP/12, Annex 6 to Doc. 5C/648, 5C/27 | Annex 6 to Document 5C/59-E |
|  | 4 August 2020 |
|  | English only |
| Annex 6 to Working Part 5C Chairman’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT REVISION OF REPORT ITU-R F.2416-0 |  |
| Technical and operational characteristics and applications of the point-to-point fixed service applications operating in the frequency band 275-450 GHz |  |

Question ITU-R 257/5

(2017)

Summary of the revision

In this revision, Annex 4 is updated to include measured antenna radiation patterns of offset parabola and Cassegrain antennas together with the results calculated from the most recent version of Recommendations ITU-R F.699 and ITU-R F.1245. Other improvements have also been made to the text of the report, including updating it with the relevant decisions at WRC-19, as outlined in RR No. 5.564. A new Annex 5 is added to address the specific conditions necessary to ensure the protection of Earth exploration-satellite service (passive) applications by the fixed service applications.

# 1	Introduction

Due to progress of RF integrated devices and circuits in the frequency band above 275 GHz, the contiguous frequency bands with large bandwidth may become available for fixed service applications. Some applications in the frequency band above 275 GHz such as the point-to-point backhaul and fronthaul for mobile services maybe  introduced and the ultra-high-speed data transmission between fixed stations may become feasible.

Radio Regulations No. 5.565 identifies the specific frequency bands for applications of the radio astronomy service, the earth exploration satellite service (passive), and the space research service (passive) in the frequency range of 275-1 000 GHz. Although the use of the frequency range 275-1 000 GHz by the passive services does not preclude use of this range by active services, administrations wishing to make frequencies in the 275-1 000 GHz range available for active service applications are urged to take all practicable steps to protect these passive services from harmful interference. Report ITU-R SM.2450 provides results of sharing and compatibility studies between LMS and FS applications planning to operate in the frequency range 275-450 GHz and passive services (radio astronomy service and Earth exploration-satellite service (passive) that have been done pre-WRC-19.

Furthermore, RR No. 5.564A identifies the frequency bands 275-296 GHz, 306-313 GHz, 318-333 GHz and 356-450 GHz for use by administrations for the implementation of land mobile and fixed service applications, where no specific conditions are necessary to protect Earth exploration-satellite service (passive) applications and indicates that the frequency bands 296-306 GHz, 313-318 GHz and 333-356 GHz may only be used by fixed and land mobile service applications when specific conditions to ensure the protection of Earth exploration-satellite service (passive) applications are determined in accordance with Resolution 731 (Rev.WRC-19). Therefore Annex 5 addresses those specific conditions to ensure the protection of Earth exploration-satellite service (passive) applications by the fixed service applications.

This Report intends to provide the planned technical and operational characteristics of the fixed service applications operating in the frequency range 275-450 GHz which will be useful for the sharing and compatibility studies between the fixed service applications and the already identified passive services.

# 2	Scope

This Report provides the fixed service applications and their expected technical and operational characteristics operating in the frequency range 275-450 GHz for sharing and compatibility studies between fixed service applications and passive services, as well as between fixed and other active services in the frequency range 275-450 GHz.

# 3	Related Recommendations and Reports

| Recommendation ITU-R F.699 | Reference radiation patterns for fixed wireless system antennas for use in coordination studies and interference assessment in the frequency range from 100 MHz to 86 GHz |
| --- | --- |
| Recommendation ITU-R F.758 | System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference |
| Recommendation ITU-R F.1245 | Mathematical model of average and related radiation patterns for line-of-sight point-to-point fixed wireless system antennas for use in certain coordination studies and interference assessment in the frequency range from 1 GHz to 86 GHz |
| Recommendation ITU-R M.2083 | IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond |
| Recommendation ITU-R M.2101 | Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies |
| Recommendation ITU-R P.525 | Calculation of free-space attenuation |
| Recommendation ITU-R P.530 | Propagation data and prediction methods required for the design of terrestrial line-of-sight systems |
| Recommendation ITU-R P.676 | Attenuation by atmospheric gases |
| Recommendation ITU-R P.838 | Specific attenuation model for rain for use in prediction methods |
| Recommendation ITU-R P.840 | Attenuation due to clouds and fog |
| Report ITU-R F.2323 | Fixed service use and future trends |
| Report ITU-R M.2376 | Technical feasibility of IMT in bands above 6 GHz |
| Report ITU-R RA.2189 | Sharing between the radio astronomy service and active services in the frequency range 275-3 000 GHz |
| Report ITU-R SM.2352 | Technology trends of active services in the frequency range 275-3 000 GHz |
| Report ITU-R SM.2450 | Sharing and compatibility studies between land-mobile, fixed and passive services in the frequency range 275-450 GHz |

# 4	List of acronyms and abbreviations

| BS | Base Station |
| --- | --- |
| BBU | Base band unit |
| RRH | Remote radio head |
| THF | Tremendously high frequency |

# 5	Frequency ranges under consideration

As the unit of frequency is the hertz (Hz), frequencies shall be expressed in gigahertz (GHz), above 3 GHz, up to and including 3 000 GHz in accordance with Radio Regulations. The gigahertz frequency range is subdivided into three ranges as shown in Table 1.

TABLE 1

Frequency bands above 3 GHz

| Band number | Symbols | Frequency range (lower limit exclusive, upper limit inclusive) | Corresponding metric subdivision |
| --- | --- | --- | --- |
| 10 | SHF | 3 to 30 GHz | Centimetric waves |
| 11 | EHF | 30 to 300 GHz | Millimetric waves |
| 12 | THF | 300 to 3000 GHz | Decimillimetric waves |

# 6	Overview of fixed service applications operating in the frequency range 275-450 GHz

## 6.1	Regulatory information and technology trend above 275 GHz

### 6.1.1	Regulatory information

WRC-12 agenda item 1.6 covered the review of the Radio Regulations to update the spectrum use by the passive services between 275 GHz and 3 000 GHz. The revised footnote RR No. 5.565 highlights that use of the range 275-1 000 GHz by the passive services does not preclude use of this range by active services. It also states that administrations wishing to use the frequency range 275-1 000 GHz for active services are urged to take all practicable steps to protect passive services from harmful interference. Subsequently, WRC-19 agenda item 1.15 adopted RR No. 5.564A to identify candidate frequency bands for use by systems in the land-mobile and fixed services while ensuring protection of the passive services identified in RR No. 5.565:

5.565	The following frequency bands in the range 275–1 000 GHz are identified for use by administrations for passive service applications:

–	radio astronomy service: 275-323 GHz, 327-371 GHz, 388-424 GHz, 426-442 GHz, 
453-510 GHz, 623-711 GHz, 795-909 GHz and 926-945 GHz;

–	Earth exploration-satellite service (passive) and space research service (passive): 275-286 GHz, 296-306 GHz, 313-356 GHz, 361-365 GHz, 369-392 GHz, 397-399 GHz, 409-411 GHz, 416-434 GHz, 439-467 GHz, 477-502 GHz, 523-527 GHz, 538-581 GHz, 611-630 GHz, 634-654 GHz, 657-692 GHz, 713-718 GHz, 729-733 GHz, 750-754 GHz, 771-776 GHz, 823-846 GHz, 850-854 GHz, 857-862 GHz, 866-882 GHz, 905-928 GHz, 951-956 GHz, 968-973 GHz and 985-990 GHz.

The use of the range 275–1 000 GHz by the passive services does not preclude use of this range by active services. Administrations wishing to make frequencies in the 275–1 000 GHz range available for active service applications are urged to take all practicable steps to protect these passive services from harmful interference until the date when the Table of Frequency Allocations is established in the above-mentioned 275–1 000 GHz frequency range.

All frequencies in the range 1 000–3 000 GHz may be used by both active and passive services.    (WRC-12)

5.564A	For the operation of fixed and land mobile service applications in frequency bands in the range 275-450 GHz:

The frequency bands 275-296 GHz, 306-313 GHz, 318-333 GHz and 356-450 GHz are identified for use by administrations for the implementation of land mobile and fixed service applications, where no specific conditions are necessary to protect Earth exploration-satellite service (passive) applications.

The frequency bands 296-306 GHz, 313-318 GHz and 333-356 GHz may only be used by fixed and land mobile service applications when specific conditions to ensure the protection of Earth exploration-satellite service (passive) applications are determined in accordance with Resolution 731 (Rev.WRC-19).

In those portions of the frequency range 275-450 GHz where radio astronomy applications are used, specific conditions (e.g. minimum separation distances and/or avoidance angles) may be necessary to ensure protection of radio astronomy sites from land mobile and/or fixed service applications, on a case-by-case basis in accordance with Resolution 731 (Rev.WRC-19).

The use of the above-mentioned frequency bands by land mobile and fixed service applications does not preclude use by, and does not establish priority over, any other applications of radio services in the range of 275-450 GHz.      (WRC-19)”

### 6.1.2	Technology trends

Progress in semiconductor and photonic devices has enabled handling spectrum above 200 GHz with a simple configuration. Oscillators and amplifiers with operating frequencies from 200 GHz to 400 GHz have been developed by using compound semiconductor technologies, such as Indium Phosphide (InP) and Gallium Arsenide (GaAs) high electron mobility transistors (HEMTs) and heterojunction bipolar transistors (HBTs). According to the International Technology Roadmap for Semiconductors (ITRS), the cut-off frequency of silicon complementary metal–oxide–semiconductors (Si CMOS) will reach 1 THz before 2021.

The spectrum beyond 275 GHz would enable emerging and future innovations that can support bit rate capacities on the road toward 100 Gbit/s. The traditional link configurations, such as FDD, as well as complementary future innovations might better handle potential asymmetric and partly unpaired sub-bands.

### 6.1.3	Propagation aspects

Figure 1 shows the attenuation coefficient in the frequency range 100-1 000 GHz, calculated from Recommendation ITU-R P.676. The attenuation coefficient in THF is generally larger than that in SHF and millimetric wave region. However, in the frequency range of 100-370 GHz, the attenuation coefficient is smaller than in the 60 GHz band.

FIGURE 1

Attenuation coefficient in the frequency range 100-1 000 GHz 
(Dashed line indicates attenuation coefficient highest peak of 60 GHz band)

Figure 2 provides further details about the trends of absorption attenuation due to atmospheric gases (i.e. air) and rain with increasing frequency. The calculation method for these attenuation factors are described in Recommendation ITU-R P.530.

FIGURE 2

Attenuation from atmospheric gases and rain

As can be seen from Figs 1 and 2, even though free space path loss increases with frequency, the overall propagation conditions are only slightly worse in the 275-450 GHz band compared to the frequency ranges 92-114.5 GHz and 130-174.5 GHz, outside the frequency portions having absorption peaks.

The spectrum above 100 GHz consists of a multitude of different sized bands (coloured boxes in Fig. 2 which are allocated to the fixed service. There is some interest in the use of frequencies beyond the frequency range 130-174.5 GHz for fixed service systems within the next decade for applications such as backhaul and fronthaul for IMT-2020 mobile services networks. It should also be noted that the 252-275 GHz frequency range is already allocated to the fixed service. Even though free space path loss increases with frequency, the overall propagation conditions in the range 275-320 GHz band are similar to the frequency range 252-275 GHz, as shown in Fig. 2, thus the range 252-320 GHz would enable 68 GHz for radio systems capable of fulfilling high capacity transmission. Therefore, this frequency range may be used for outdoor point-to-point fixed service applications over a distance of several hundred meters, making it suitable for short distance and very high capacity fixed services as an alternative to wireline backhaul transport applications in dense urban areas.

## 6.2	Point-to-point fronthaul and backhaul

Figure 3 shows an example of the expected network architecture of mobile systems that support high-capacity transmission between a base station and a mobile terminal. The fronthaul is defined as a link connection between the base station’s baseband unit (BBU) and the remote radio head (RRH), while the backhaul is a link between the base station and the higher-level network elements. According to Recommendation ITU-R M.2083 and Report ITU-R M.2376, fronthaul and backhaul are critical challenges to accommodate the increase in data throughput of future mobile traffic. In order to meet the peak data rate 10-20 Gbit/s of the mobile terminals in a small cell, the transmission capacity of fronthaul and backhaul may exceed tens of Gbit/s substantially.

The frequency band above 275 GHz can meet the requirement of transmission capacity of fronthaul and backhaul.

FIGURE 3

Fronthaul and backhaul operation to be used for mobile system network

# 7	System characteristics

## 7.1	Characteristics for systems planned to operate in the bands 275-325 GHz and 380-44 GHz

An example of the technical and operational characteristics of expected fixed point-to-point systems planned to operate in the bands 275-325 GHz and 380-445 GHz is shown in Table 2, provided that sharing analysis can demonstrate that FS can coexist with the passive services.

TABLE 2

Technical and operational characteristics of the fixed service applications planned to operate

| Frequency band (GHz) | 275-325 | 380-445 |
| --- | --- | --- |
| Duplex Method | FDD/TDD | FDD/TDD Editor’s note: Other duplex in schemes are possible |
| Modulation | BPSK/QPSK/8PSK/8APSK/16QAM/32QAM/64QAM BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM | BPSK/QPSK/8PSK/8APSK/ 16QAM/32QAM, 8PSK, 8APSK BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM |
| Channel bandwidth (GHz) | 2…..25 (FDD) 2…..50 (TDD) | 2…..32.5 (FDD) 2…..65 (TDD) |
| Spectrum mask | Annex 2 | Annex 2 |
| Tx output power range (dBm) | 0 ... 20 | −10 ... 10 |
| Tx output power density range (dBm/GHz) | −17 ... 17 | −28 ... 7 |
| Feeder/multiplexer loss range (dB) | 0 … 3 | 0 … 3 |
| Antenna gain range (dBi) | 24 … 50 | 24 … 50 |
| e.i.r.p. range (dBm) | 44 … 70 | 37 … 60 |
| e.i.r.p. density range (dBm/GHz) | 30 … 67 | 19 … 57 |
| Antenna pattern | Recommendation ITU-R F.699 (Single entry) Recommendation ITU-R F.1245 (Aggregate) | Recommendation ITU-R F.699 (Single entry) Recommendation ITU-R F.1245 (Aggregate) |
| Antenna type | Parabolic Reflector | Parabolic Reflector |
| Antenna height (m) | 6-25 | 10-25 |
| Antenna elevation (degree) | ±20 (typical) | ±20 (typical) |
| Receiver noise figure typical (dB) | 15 | 15 |
| Receiver noise power density typical (dBm/GHz) | −69 | −69 |
| Normalized Rx input level for 1×10-6 BER (dBm/GHz) | −61 … −54 | −61 … −54 |
| Link length (m) | 100 … 300 | 100 … 300 |
| Deployment Density | See § 8.1 | See § 8.1 |
| I/N protection criteria | Recommendation ITU-R F.758 | Recommendation ITU-R F.758 |

[Editor’s note: Suggest changing “…”  to  “to” in the entire Table 2 to avoid possible confusion]

# 8	Deployment scenarios of fronthaul/backhaul

## 8.1	Estimation of density of FS links

According to Recommendation ITU-R M.2101, the deployment scenarios of radio access networks for IMT are categorized into four base station locations, i.e. rural, suburban, urban and indoor. Suburban and urban scenarios are further divided into macro and micro locations whose coverage areas are distinguished. The coverage areas of the micro scenario are included in the macro area.

The fixed service applications such as fronthaul and backhaul links are expected to provide a high capacity link between BBU and RRH. The location of BBU may correspond to the macro-cellular base station and that of RRH to the micro-cellular base station, in both urban and suburban areas. However, due to the distance between the BS in suburban areas, the FS links operating in the range 275-450 GHz are assumed to be used only in urban environment whereas other links will be connected through other millimetric wave bands, which are already allocated to the fixed service.

The density of BS in urban areas is estimated to 30 BS/km2 in each of the frequency ranges expected for IMT-2020 (i.e. 24.25-33.4 GHz, 37-43.5 GHz, 45.5-52.6 GHz and 66-86 GHz). The FS link will be used for ultra-high-capacity link for dense urban area only. Although the percentage of dense urban area per 1 km2 is not specifically indicated in any ITU-R publications, a ratio of 7 % of BS is assumed in dense urban areas.

According to this assumption, the total number of BS in Tokyo metropolitan district is calculated by 7% of 120 BS multiplied by 619 km2, i.e. 5 200, as shown in Table 3, for the whole 275-450 GHz band. The other major cities in Japan are also included in Table 3. This calculation shows that a density of 8.4 FS links/km² can be expected in the whole range 275-450 GHz, hence considering a density of 4.2 FS link/km² in each of the 275-325 GHz and 380-445 GHz bands for the evaluation of aggregate effect of emission form FS links.

Although only based on some highly populated cities in Japan, this 4.2 FS link/km² figure is assumed to be globally representatives. Alternatively, another way of addressing the FS links distribution could be to use population densities together with the above ratio of 0.0007 links /inhab (for the whole 275-450 GHz range), i.e. a density of 0.00035 FS link/inhab in each of the 275-325 GHz and 380-445 GHz bands.

TABLE 3

Calculation of FS links in the 275-450 GHz range for some highly populated cities in Japan

| Name of city | Size (km2) | Population (M) | No. of FS links | FS links(1) / km² | FS links / inhab |
| --- | --- | --- | --- | --- | --- |
| Tokyo district | 619 | 9.37 | 5 200 | 8.4 | 0.0006 |
| Yokohama | 437.4 | 3.73 | 3 674 | 8.4 | 0.0010 |
| Osaka | 223 | 2.70 | 1 873 | 8.4 | 0.0007 |
| Nagoya | 326.4 | 2.30 | 2 742 | 8.4 | 0.0012 |
| Total | 1 605.8 | 18.1 | 13 489 | 8.4 | 0.0007 |
| (1) The FS link density is estimated on the condition that all four proposed millimetric waves will be regulated to use for IMT-2020 services |  |  |  |  |  |

## 8.2	Elevation angles of antenna

The antenna heights of the base stations in the urban area are estimated in the range 6-25 m. The elevation angles of the antenna are calculated from the antenna height of FS stations and the distance between FS links. Although the distance between the base stations in the dense urban area is also indicated to be 200 m, the distance range of 100-300 m is assumed to be used for calculation of elevation angle of antenna.

In the metropolitan area of Tokyo, the elevation angle is estimated to be less than ±12 degrees taking into account the above parameters as well as the surface deviation of Tokyo area.

In order to take into account the different urban geographies around the world, it is assumed that a typical elevation would be ±20 degrees.

## [8.3	Spectrum need

According to spectrum need of IMT system in the frequency range between 24.5 GHz and 86 GHz, one study result shows the estimated spectrum need of 18.7 GHz, and another study result was that of 27.4 GHz which includes indoor hotspot system.

Taking into account these study results, the channel bandwidth of 24.5 GHz is sufficient to provide high-capacity link for fronthaul/backhaul for IMT system. If the requirements are similar, the same bandwidth of around 25 GHz may satisfy the initial typical deployment scenarios.]

{Editor’s note: Paragraph above may need improvement.}

# [9	Summary

According to the previous discussion a total long-term spectrum bandwidth of about 50 GHz will sufficiently support the evolution of IMT traffic between BBU and RRH. The possible candidate frequency bands for fronthaul and backhaul applications are 275-325 GHz and 380-445 GHz. The frequency band 330-370 GHz may also be considered in the future, when parameters are available for that range.]

{Editor’s note: The summary above needs to be revisited.}

Annex 1

Attenuation characteristics in the frequency range 275-445 GHz

This Annex gives information on attenuation characteristics calculated from Recommendation ITU-R P.676-10. The attenuation by rain rate and liquid water density in fog is also calculated from Recommendations ITU-R P.838-3 and ITU-R P.840-6. The difference of attenuation by atmospheric gases at 275 GHz and 325 GHz is about 33 dB, but those by rain rate and liquid water density in fog are −0.1 dB and +0.7 dB, respectively. These characteristics should be used for designing channel arrangement of point-to-point fronthaul and backhaul applications.

FIGURE 4

Attenuation characteristics by atmospheric gases

FIGURE 5

Attenuation characteristics by rain rate

FIGURE 6

Attenuation characteristics by liquid water density in fog. The calculation results above 200 GHz 
is extrapolated using Recommendation ITU-R P.840-6

Annex 2

Transmitter spectrum mask

The following spectrum mask is taken from IEEE Std 802.15.3dTM-2017 as shown in Fig. 7 and Table 4.

FIGURE 7

Generic transmit spectral mask

PSD [dBr]

0

-20

-25

-f4

-f3

-f2

-f1

f1

f2

f3

f4

f-fc (GHz)

0

PSD [dBr]

0

-20

-25

-f4

-f3

-f2

-f1

f1

f2

f3

f4

f-fc (GHz)

0

[Editor’s note: The attenuation level associated with f4 should be added to the figure (all the other three levels are labelled).]

The parameters of the PSD indicated in Fig. 7 are defined in Table 4.

TABLE 4

Transmit spectrum mask parameters

| Channel bandwidth [GHz] | f1(GHz) | f2(GHz) | f3(GHz) | f4(GHz) |
| --- | --- | --- | --- | --- |
| 2.160 | 0.94 | 1.10 | 1.60 | 2.20 |
| 4.320 | 2.02 | 2.18 | 2.68 | 3.28 |
| 8.640 | 4.18 | 4.34 | 4.84 | 5.44 |
| 12.960 | 6.34 | 6.50 | 7.00 | 7.60 |
| 17.280 | 8.50 | 8.66 | 9.16 | 9.76 |
| 25.920 | 12.82 | 12.98 | 13.48 | 14.08 |
| 51.840 | 25.78 | 25.94 | 26.44 | 27.04 |
| 69.120 | 34.42 | 34.58 | 35.08 | 35.68 |

Annex 3

C/N evaluation of 300 GHz wireless link

This Annex evaluates the carrier-to-noise ratio of 300 GHz wireless link using the characteristics shown in Table 2. The Tx and Rx antenna gains of 50 dBi, noise figure of 15 dB, rainfall rate of 50 mm/h are used for calculation. Figure 8 shows C/N as a function of transmission bandwidth associated with the  300 GHz wireless link whose output power is 10 dBm. The wider transmission bandwidth is used, the larger attenuation deviation is observed in that bandwidth. In order to achieve C/N over 30 dB in the 50-GHz bandwidth, the output power over 20 dBm is required to transmit the signal up to 300 m. Figure 9 shows C/N as a function of distance for the  300 GHz wireless link and C/N over 40 dB is obtained if output power is over 20 dBm. The sufficient channel capacity in the condition of the output power of 20 dBm, the centre frequency of 300 GHz, the distance of 300 m and the bandwidth of 2.16 GHz is obtained even if the rainfall rate is 100 mm/h, as shown in Fig. 10. This test did not find a big difference of the channel capacity between carrier frequencies from 280 to 320 GHz. In the calculation of C/N and channel capacity, the system availability was assumed to be 100%.

FIGURE 8

C/N versus bandwidth for signal transmission

FIGURE 9

C/N versus distance of 300-GHz wireless link

FIGURE 10

Channel capacity versus rain rate

Annex 4

Measurement results of antenna radiation pattern at 300 GHz

[Editor’s note: More results are sought for this annex. The results for antenna radiation patterns provided in this annex are for now the only measured results available for frequency bands above 275 GHz and they show deviations from Recommendations ITU-R F.699-8 and ITU-R F.1245-3. As a result, membership is invited to provide measured data for the radiation patterns of fixed wireless antennas in order to develop suitable model(s) of average and related radiation patterns for fixed wireless antennas in the frequency range from 275 GHz to 450 GHz. [Recommendations ITU-R F.699-7 and ITU-R F.1245-2 are more conservative patterns relative to the last revisions, providing 10 dB less attenuation above 48 degrees offset from main beam]. Israel proposed to extend in the future Recommendations ITU-R F.699 and ITU-R F.1245 above 86 GHz.]

FIGURE 11

Measured characteristics of offset parabola antenna with a maximum gain of 49 dBi,
where the red line is calculated from Recommendation ITU-R F.699-7

FIGURE 12

Measured characteristics of Cassegrain antenna with a maximum gain of 47 dBi,
where the red line is calculated from Recommendation ITU-R F.699

FIGURE 13

Measured characteristics of offset parabola antenna with a maximum gain of 49 dBi,
where the red line is calculated from preliminary draft revision of Recommendation ITU-R F.1245-2

[Editor’s note: Red pattern to be changed to ITU-R F.1245-3. To verify if Annex 9 of Chair’s Report June 2018, Document 5C/531, is identical to ITU-R F.1245-3.]

FIGURE 14

Measured characteristics of Cassegrain antenna with a maximum gain of 47 dBi,
where the red line is calculated relative to Recommendation ITU-R F.1245-3

Annex 5

Specific conditions to be applied to the fixed service applications in the frequency bands 296-306 GHz, 313-318 GHz and 333-356 GHz, to ensure the protection of Earth exploration-satellite service (passive) applications in accordance with Resolution 731 (Rev. WRC-19)

[Editorial note: The specific conditions should be developed in a separate ITU-R Recommendation/Report or as an Annex to Resolution 731 (Rev. WRC-19)] in WP 5C; contributions are encouraged for the next meeting of WP 5C]

<TBD>

____________________

THF stands for tremendously high frequency, this terminology is used only within this Report.

102-109.5 GHz, 111.8-114.25 GHz, 122.25-123 GHz, 130-134 GHz, 141-148.5 GHz, 151.5-164 GHz, 167-174.8 GHz, 191.8-200 GHz, 209-226 GHz, 231.5-235 GHz, 238-241 GHz, and 252-275 GHz.