|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Addendum 9 to Document 87-E |  |
|  |  | 23 October 2023 |  |
|  |  | Original: English |  |
| African Common Proposals |  |  |  |
| Proposals for the work of the conference |  |  |  |
| Agenda item 1.9 |  |  |  |

1.9	to review Appendix 27 of the Radio Regulations and consider appropriate regulatory actions and updates based on ITU-R studies, in order to accommodate digital technologies for commercial aviation safety-of-life applications in existing HF bands allocated to the aeronautical mobile (R) service and ensure coexistence of current HF systems alongside modernized HF systems, in accordance with Resolution 429 (WRC-19);

Proposals

APPENDIX 27 (REV.WRC-19)*

Frequency allotment Plan for the aeronautical mobile (R)
service and related information

PART  I  –  General provisions

Section II  –  Technical and operational principles used
for the establishment of the Plan of allotment of frequencies
in the aeronautical mobile (R) service

A  –  Channel characteristics and utilization

# 2	Frequencies allotted

ADD	AFCP/87A9/1#1633

27/18A		Individual contiguous or non-contiguous channels complying with the provisions of the Plan3 contained in this Appendix may be aggregated to provide wideband communication without changing the Plan of individual channels.

ADD	AFCP/87A9/2#1634

_______________

3 	27/18A.1	In particular the provisions related to the protection (Part I, Section II B), to power limits (Nos. 27/60 and 27/61), to class of emissions (No. 27/58), to out-of-band spectrum mask (No. 27/74), to assigned frequency (No. 27/75), and to channel spacing (No. 27/11).

C  –  Classes of emission and power

# 1	Classes of emission

MOD	AFCP/87A9/3#1635

27/57	1.1	Telephony – amplitude modulation:

double sideband	A3E*

single sideband, full carrier	H3E*

single sideband, suppressed carrier	J3E, J2E, J7E, J9E

MOD	AFCP/87A9/4#1636

1.2	Telegraphy and data transmission

MOD	AFCP/87A9/5#1637

27/58	1.2.1	Amplitude modulation:

telegraphy without the use of a modulating audio frequency (by on-off keying)	A1A, A1B**

–	telegraphy by the on-off keying of an amplitude modulating audio frequency or audio frequencies or by the on-off keying of the modulated emission and including selective calling, single sideband, full carrier	H2B

–	multichannel voice frequency telegraphy, single sideband, suppressed carrier	J7A

–	telegraphy or data transmissions using any other single sideband, suppressed carrier modulation, under the condition that the reference frequency of the concerned transmission corresponds to the list of carrier (reference) frequencies (No. 27/18) and its occupied bandwidth does not exceed the upper limit of J3E emissions (No. 27/12), i.e. 2 800 Hz for each individual channel	J2B, J2D, J7B, J7D, J9B, J9D

# 2	Power

MOD	AFCP/87A9/6#1638

27/60	2.1	Unless otherwise specified in Part II of this Appendix, the peak envelope powers supplied to the antenna transmission line shall not exceed the maximum values indicated in the Table below; the corresponding peak effective radiated powers being assumed to be equal to two-thirds of these values.

| Class of emission |  | Stations | Maximum peak envelope power |
| --- | --- | --- | --- |
| H2B, J3E, J7A, J2E, J7E, J9E, J2B, J2D, J7B, J7D, J9B, J9D A3E*, H3E* (100% modulation) |  | Aeronautical stations Aircraft stations | 6 kW 400 W |
| Other emissions such as A1A, A1B, F1B |  | Aeronautical stations Aircraft stations | 1.5 kW 100 W |
| * A3E and H3E to be used only on 3 023 kHz and 5 680 kHz. |  |  |

Note: the “(100% modulation)” may require additional clarification.

Reasons:	
1.	The new proposed digital wideband HF systems comply with the existing analogue voice and data communication systems without causing interference or assignment modification unless agreed to by affected member states and operate in accordance with the ICAO international Standards and Recommended Practices and procedures.
2.	Protection of in band and adjacent band services shall be ensured.

SUP	AFCP/87A9/7#1639

RESOLUTION 429 (WRC-19)

Consideration of regulatory provisions for updating Appendix 27 of the Radio Regulations in support of aeronautical HF modernization

__________________

_______________

*	Note by the Secretariat: This edition of Appendix 27 incorporates editorial amendments to the Appendix 27 Aer2 as adopted by the WARC-Aer2.

The references in Appendix 27 now conform to the new numbering scheme of the Radio Regulations. In addition, the text of Appendix 27 contains updated definitions of the relevant aeronautical areas conforming with the new geographical situation reflecting the political changes since 1979. It also contains updated references to the classes of emissions in accordance with Article 2.      (WRC-03)

*	A3E and H3E to be used only on 3 023 kHz and 5 680 kHz.

**	A1A, A1B and F1B are permitted provided they do not cause harmful interference to the classes of emission H2B, J3E, J2E, J7E, J9E, J7A, J2B, J2D, J7B, J7D, J9B, and J9D. In addition, AlA, A1B and FlB emissions shall be in accordance with the provisions in Nos. 27/70 to 27/74 and care should be taken to place these emissions at or near the centre of the channel. However, a modulating audio frequency is permitted with single sideband transmitters, where the carrier is suppressed in accordance with No. 27/69.