|  | World Radiocommunication Conference (WRC-23) Dubai, 20 November - 15 December 2023 |  |  |
| --- | --- | --- | --- |
| PLENARY MEETING |  | Addendum 16 to Document 99-E |  |
|  |  | 27 October 2023 |  |
|  |  | Original: English |  |
| Japan |  |  |  |
| Proposals for the work of the conference |  |  |  |
| Agenda item 1.16 |  |  |  |

1.16	to study and develop technical, operational and regulatory measures, as appropriate, to facilitate the use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) by non-geostationary fixed-satellite service earth stations in motion, while ensuring due protection of existing services in those frequency bands, in accordance with Resolution 173 (WRC-19);

Resolution 173 (WRC-19) – Use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) by earth stations in motion communicating with non-geostationary space stations in the fixed-satellite service

# 1	Introduction

WRC-23 agenda item 1.16 considers the use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz (space-to-Earth), 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) by earth stations in motion communicating with non-geostationary (non-GSO) space stations in the fixed-satellite service (FSS). The studies under this agenda item considered two types of earth stations in motion (ESIMs): aeronautical and maritime only. Studies have been carried out on sharing and compatibility between ESIMs and terrestrial as well as space services allocated in the frequency bands above. For this agenda item, two methods have been identified:

Method A

No changes to the Radio Regulations and suppression of Resolution 173 (WRC-19).

Method B

Add a new footnote in RR Article 5 that refers to a new WRC Resolution with technical, operational and regulatory conditions for the operation of non-GSO maritime and aeronautical ESIMs while ensuring protection of allocated services and consequential suppression of Resolution 173 (WRC-19).

The 6th Meeting of the APT Conference Preparatory Group for WRC-23 (APG23-6) in August 2023, the Asia Pacific Telecommunity (APT) members agreed a view to consider Method A, and also Method B with the APT Common Proposal (ACP) under the condition that all the issues raised in the ACP are successfully resolved an agreed upon at the WRC-23. However, due to time constraints, ACP could only discuss the sections of the draft resolution which were discussed at CPM23-2, and agreed that the rest of the sections had to be considered at the WRC-23.

Taking into account the situation explained above, Japan provides the additional proposals in order to help the satisfaction of the condition above.

# 2	Discussion

Japan provides our view as follows on each discussion point:

–	Responsibilities of affected and/or authorizing administrations

•	Japan has a view that the notifying administration of the non-GSO FSS system with which ESIMs communicate is responsible for resolving the case of unacceptable interference. In addition, Japan also has a view that the authorizing administration also would have a responsibility to some extent for the resolution of unacceptable interference as early as possible in cooperation with the notifying administration.

–	Management system of non-GSO ESIM and unacceptable interference elimination procedure

•	The interference management procedure in case of unacceptable interference caused by non-GSO ESIM is needed to assume and identify that the notifying administration of the non-GSO satellite system with which ESIM communicate is responsible for the operation of non-GSO ESIM.

–	Methodology of pfd verification

•	Invited by Resolution 169 (WRC-19), WP 4A had developed the examination methodology during this WRC-23 study cycle. At the CG of Resolution 169 (WRC-19) held in February 2023 and CPM23-2, Japan raised the issue that the existing calculation algorithm could not protect the terrestrial services, where the exceedance of the pfd limit specified in Resolution 169 (WRC-19) might not be identified in some cases due to an overly optimistic assumption of the antenna gain of aeronautical ESIM towards a terrestrial station.

•	At the WP 4A and SG 4 in July 2023, the issue raised above was considered by interest groups, and WP 4A and SG 4 finalized the examination methodology as Draft New Recommendation ITU-R S.[METHOD] (Doc. 4/93) with respect to the conformity with the pfd limits at the Earth’s surface for aeronautical ESIM communicating with GSO satellites. Japan considers that the modified calculation algorithm which is utilized in the finalized examination methodology is a reasonable solution in order to examine the conformity with the pfd limits specified in Resolution 169 (WRC-19) for the protection of the terrestrial services in the frequency bands since it determines the maximum allowable emission power levels of aeronautical ESIM at each altitude by employing the antenna gain of aeronautical ESIM, which is derived from technical characteristics contained in its RR AP4 submission, towards a terrestrial station.

•	While the finalized examination methodology contained in draft new Recommendation ITU-R S.[METHOD] for Resolution 169 (WRC-19) is applicable to aeronautical ESIMs communicating with GSO satellites, Japan is of the view that it should be taken into account as a basis, for the reason above, when considering a methodology to be applied to aeronautical ESIMs communicating with non-GSO satellites under this agenda item.

•	Japan is also of the view that a certain level of tuning of the methodology is necessary for applying the other parts of the finalized examination methodology for Resolution 169 (WRC-19) than the calculation algorithm, to WRC-23 agenda item 1.16 as well, considering the different characteristics of the GSO and non-GSO aeronautical ESIMs.

–	Conditions to protect terrestrial services and EESS

•	Japan has a view that transmitting non-GSO ESIMs in the frequency band 27.5-29.1 GHz shall not cause unacceptable interference to terrestrial services to which the frequency band is allocated and that operate in accordance with the Radio Regulations, and Annex 1 and Annex 2 to this Resolution shall apply.

–	Treatment of the land ESIM

•	Japan has a view that this Resolution does not establish any technical or regulatory provisions for the operation and use of land ESIMs communicating with non-GSO FSS space stations, and any authorization of land ESIMs remains strictly a national matter, taking also into account the need to avoid cross-border interference.

# 3	Proposal

Japan supports the ACP, however, with the views raised above, Japan also proposes to amend the draft new Resolution [A116] (WRC-23) in the Report of the CPM to WRC-23 to address the protection of terrestrial services from aeronautical ESIM(s) in WRC-23 as indicated in the proposals in the Annex to this document, especially to have amendments on the parts which still have options in the ACP and to have amendments on the parts which were not discussed at CPM23-2 because these sections were not discussed in APG23-6 as well. The proposals of the amendment are highlighted with yellow colour with the reasons in turquoise colour.

Annex – Proposals

Japan supports Method B, and Japanese proposals for WRC-23 agenda item 1.16 are as shown below.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	J/99A16/1#1880

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A ADD 5.A116 (Earth-to-space) 5.516 MOBILE | 17.7-17.8 FIXED FIXED-SATELLITE (space-to-Earth) 5.517 5.517A ADD 5.A116 (Earth-to-space) 5.516 BROADCASTING-SATELLITE Mobile 5.515 | 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A ADD 5.A116 (Earth-to-space) 5.516 MOBILE |
|  | 17.8-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A ADD 5.A116 (Earth-to-space) 5.516 MOBILE 5.519 |  |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A ADD 5.A116 (Earth-to-space) 5.520 MOBILE 5.519 5.521 |  |  |

MOD	J/99A16/2#1881

18.4-22 GHz

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A ADD 5.A116 MOBILE |  |  |  |  |
| … |  |  |  |  |
| 18.8-19.3 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.523A ADD 5.A116 MOBILE |  |  |  |  |
| … |  |  |  |  |
| 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A ADD 5.A116 Mobile-satellite (space-to-Earth) |  | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A ADD 5.A116 MOBILE-SATELLITE (space-to-Earth) |  | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A ADD 5.A116 Mobile-satellite (space-to-Earth) |
| 5.524 |  | 5.524 5.525 5.526 5.527 5.528 5.529 |  | 5.524 |
| 20.1-20.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A ADD 5.A116 MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 |  |  |  |  |

MOD	J/99A16/3#1882

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.539 ADD 5.A116 MOBILE 5.538 5.540 |  |  |
| 28.5-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.523A 5.539 ADD 5.A116 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| … |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 ADD 5.A116 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 ADD 5.A116 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 ADD 5.A116 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) |
| 5.540 5.542 | 5.525 5.526 5.527 5.529 5.540 | 5.540 5.542 |

MOD	J/99A16/4#1883

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 29.9-30 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 ADD 5.A116 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.525 5.526 5.527 5.538 5.540 5.542 |  |  |

ADD	J/99A16/5#1884

5.A116	The operation of earth stations in motion communicating with non-geostationary space stations in the fixed-satellite service in the frequency bands 17.7-18.6 GHz (space-to-Earth), 18.8-19.3 GHz (space-to-Earth) and 19.7-20.2 GHz (space-to-Earth), 27.5-29.1 GHz (Earth-to-space) and 29.5-30 GHz (Earth-to-space) shall be subject to the application of Resolution [A116] (WRC-23).     (WRC-23)

ADD	J/99A16/6#1885

draft new RESOLUTION [A116] (WRC-23)

[Editor’s note: Track changes with yellow highlight are modifications to draft new Resolution [A116] (WRC-23) in the CPM Report (Document 3) in order to benefit from better understandings.]

There are several areas in which there is no consensus either on the text or how to proceed with the implementation of this Resolution. Consequently, the text below is not consistent with resolves 5 of Resolution 173 (WRC-19).

Resolves the ITU Radiocommunication Sector to ensure that the results of ITU-R studies are agreed by Member States by consensus

Use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) by aeronautical and maritime earth stations in motion communicating 
with non-geostationary space stations in the fixed-satellite service

Reasons:	In the ACP, the options in the title are still remained as above. So Japan proposes to support Option 2.

…

resolves

…

1.2.4	the provisions in this Resolution, including Annex 1, set the conditions for the purpose of protecting terrestrial services from unacceptable interference from non-GSO ESIMs in neighbouring countries in accordance with the provisions included in resolves 1.2.2 and 1.2.3 above in the frequency band 27.5-29.1 GHz and in the frequency band 29.5-30.0 GHz with respect to administrations mentioned in No. 5.542; however, the requirement not to cause unacceptable interference to, or claim protection from, terrestrial services to which the frequency bands are allocated and operating in accordance with the Radio Regulations remains valid (see resolves 6);

Reasons:	In the ACP, the options in the resolves 1.2.4 are still remained as above. So Japan proposes to support Option 3.

1.2.5	the Bureau shall examine, in accordance with the provisions included in resolves 1.2.2 and 1.2.3 and with the methodology in Annex 2, the characteristics of aeronautical non-GSO ESIMs with respect to the conformity with the power flux-density (pfd) limits on the Earth’s surface specified in Part 2 of Annex 1 to this Resolution and publish the results of such examination in the BR IFIC;

1.2.5.1	however, the compliance with the technical conditions in Annex 1, does not release the notifying administration of the A-ESIM and M-ESIM with respect to discharging its responsibility that such earth stations shall not cause unacceptable interference and any interrelated receiving part shall not claim protection from the terrestrial stations;

-

Reasons:	The resolves 1.2.5.1 is agreeable from the protection of the terrestrial stations and no claim protection from the terrestrial stations point of view.

1.2.6	if the BR is unable to examine, in accordance with resolves 1.2.5, non-GSO aeronautical ESIMs with respect to conformity with the pfd limits specified in Part 2 of Annex 1, the notifying administration shall send to BR a commitment to ensure that the aeronautical non-GSO ESIMs comply with those limits;

1.2.7	the BR shall formulate a qualified favourable finding under No. 11.31 with respect to the pfd limits contained in Part 2 of Annex 1, otherwise the BR shall formulate an unfavourable finding;

1.2.8	once the methodology to examine the characteristics of aeronautical non-GSO ESIMs with respect to conformity with the pfd limits on the Earth’s surface specified in Part 2 of Annex 1 is available, resolves 1.2.54 shall be applied by the Bureau;

Reasons:	Duplicated resolves 1.2.8 above was deleted.

…

2	that non-GSO ESIMs shall not be used or relied upon for safety-of-life applications;

Reasons:	Moved to new recognizing j), as it refers to an existing provision.

3	that the notifying administrations of those non-GSO FSS systems with which non-GSO ESIMs in the frequency bands in considering a) above are intended to operate shall submit a commitment to the Bureau to immediately act to eliminate or reduce the interference to an acceptable level upon receiving a report of unacceptable interference (see resolves 1.4.2);

…

56	that the application of this Resolution does not provide regulatory status to non-GSO ESIMs different from that derived from the non-GSO FSS satellite system with which they communicate, taking into account the provisions referred to in this Resolution;

Reasons:	The original recognizing b) was deleted.

6	that any course of action taken under this Resolution has no impact on the original date of receipt of the frequency assignments of the non-GSO FSS satellite system with which non-GSO ESIMs communicate or on the coordination requirements of that satellite system;

…

6	that frequency assignments to ESIMs shall be notified by the notifying administration of the non-GSO satellite system in the FSS with which ESIMs communicate;

7	that the notifying administration of the satellite system shall ensure that non-GSO ESIMs operate only in the territory under the jurisdiction of administrations from which an authorization has been obtained, taking into account recognizing further c);

8	that ESIMs shall be designed and operate so as to cease transmission over the territory of any administration/country from which authorization has not been obtained;

9	that, for the implementation of resolves further 2 above, the system shall employ the minimum software and hardware capabilities listed in Annex 4;

10	that, for the implementation of resolves further 1, the notifying administration responsible for the operation of aeronautical and maritime non-GSO ESIMs shall also be responsible for observing and complying with all relevant regulatory and administrative provisions applicable to the operation of the ESIMs as included in this Resolution and those contained in the Radio Regulations;

11	that the authorization to non-GSO ESIM to operate in the territory under the jurisdiction of an administration shall in no way release the notifying administration of the non-GSO satellite system with which the non-GSO ESIM communicates from the obligation to comply with the provisions included in this Resolution and those contained in the Radio Regulations;

instructs the Director of the Radiocommunication Bureau

1	to take all necessary actions to facilitate the implementation of this Resolution, together with providing any assistance for the resolution of interference, when required;

2	to report to future world radiocommunication conferences any difficulties or inconsistencies encountered in the implementation of this Resolution, including whether or not the responsibilities relating to the operation of aeronautical and maritime non-GSO ESIMs have been properly addressed;

3	not to examine, under No. 11.31, the conformity of non-GSO FSS systems with the provisions of resolves 1.1.5 of this Resolution,

-

4	to report to future world radiocommunication conferences any difficulties or inconsistencies encountered in the implementation of Recommendation ITU-R S.1503 for verifying that the non-GSO FSS systems under this Resolution comply with the epfd limits specified in Article 22;

Reasons:	It should remain taking into account the potential risk which will be recognized after the operation.

5	to publish the list of non-GSO satellite systems with which ESIM communicate brought into use with information about its service area and the countries that authorize such use if any; this information shall be updated regularly,

invites administrations

to take into consideration the relevant recommendations to employ Annex 4 procedures when licensing/authorizing the operation of earth stations in motion in their territories,

instructs the Secretary-General

to bring this Resolution to the attention of the Secretary-General of the International Maritime Organization and of the Secretary General of the International Civil Aviation Organization.

Annex 1 to draft new Resolution [A116] (WRC-23)

Provisions for maritime and aeronautical non-GSO ESIMs to protect terrestrial services operating in the frequency band 27.5-29.1 GHz and for the frequency band 29.5-30.0 GHz with respect to/on the territories of/in relation to administrations mentioned in No. 5.542 /as a guidance for administrations when considering to authorize the A-ESIM and M-ESIM in their territories

--

--

The parts below contain provisions to ensure that maritime and aeronautical non-GSO ESIMs do not cause unacceptable interference in neighbouring countries to terrestrial service operations when non-GSO ESIMs operate in frequencies overlapping with those used by terrestrial services at any time to which the frequency band 27.5-29.1 GHz is allocated and that operate in accordance with the Radio Regulations. The provisions in the parts below -also apply in the frequency band 29.5-30 GHz with respect to administrations mentioned in No. 5.542 of the Radio Regulations.

Reasons:	To align with the resolves 1.2.4, the Option 3 is chosen.

--

--

--

Part 1: Maritime non-GSO ESIMs

1	The notifying administration of the non-GSO FSS satellite system with which maritime ESIMs communicates shall ensure compliance of the maritime ESIMs operating within the frequency bands 27.5-29.1 GHz and 29.5-30 GHz, or parts thereof, with both of the following conditions for the protection of terrestrial services to which the frequency bands are allocated within a coastal State:

1.1	The minimum distance from the low-water mark as officially recognized by the coastal State beyond which maritime ESIMs can operate without the prior agreement of any administration is 70 km within the frequency bands 27.5-29.1 GHz and 29.5-30.0 GHz. Any transmissions from maritime ESIMs within the minimum distance shall be subject to the prior agreement of the coastal State(s) concerned.

Reasons:	To align with the provision 1 of Part 1, the frequency bands need to remain.

1.2	The maximum maritime ESIMs e.i.r.p. spectral density towards the territory of any coastal State shall be limited to [12.98/24.44] dBW in a reference bandwidth of [1/14] MHz. Transmissions from maritime ESIMs with higher e.i.r.p. spectral density levels towards the territory of any coastal State shall be subject to the prior agreement of the coastal State(s) concerned.

Reasons:	The values should be discussed in the Conference.

Part 2: Aeronautical non-GSO ESIMs

2	The notifying administration of the non-GSO FSS satellite system with which aeronautical ESIMs communicates shall ensure compliance of the aeronautical ESIMs operating within the frequency bands 27.5-29.1 GHz and 29.5-30 GHz, with all of the following conditions for the protection of the terrestrial services to which the frequency bands are allocated:

2.1	When within line-of-sight of the territory of an administration, and above an altitude of 3 km, the maximum pfd produced at the surface of the Earth on the territory of an administration by emissions from a single aeronautical ESIM shall not exceed:

pfd(θ) = −124.7	(dB(W/(m2 ∙ [14] MHz)))	for	0°	≤ θ ≤ 0.01°

pfd(θ) = −120.9 + 1.9 ∙ logθ	(dB(W/(m2 ∙ 14 MHz)))	for	0.01°	< θ ≤ 0.3°

pfd(θ) = −116.2 + 11 ∙ logθ	(dB(W/(m2 ∙ 14 MHz)))	for	0.3°	< θ ≤ 1°

pfd(θ) = −116.2 + 18 ∙ logθ	(dB(W/(m2 ∙ 14 MHz)))	for	1°	< θ ≤ 2°

pfd(θ) = −117.9 + 23.7 ∙ logθ	(dB(W/(m2 ∙ 14 MHz)))	for	2°	< θ ≤ 8°

pfd(θ) = −96.5	(dB(W/(m2 ∙ 14 MHz)))	for	8°	< θ ≤ 90.0°

where θ is the angle of arrival of the radio-frequency wave (degrees above the horizon).

Reasons:	The bandwidth should be the same as the values in Annex 3 of Resolution 169 for simplification.

2.2	When within line-of-sight of the territory of an administration, and up to an altitude of 3 km, the maximum pfd produced at the surface of the Earth on the territory of an administration by emissions from a single aeronautical ESIM shall not exceed:

pfd(θ) = −136.2	(dB(W/(m2 ∙ 1 MHz)))	for	0°	≤ θ ≤ 0.01°

pfd(θ) = −132.4 + 1.9 ∙ logθ	(dB(W/(m2 ∙ 1 MHz)))	for	0.01°	< θ ≤ 0.3°

pfd(θ) = −127.7 + 11 ∙ logθ	(dB(W/(m2 ∙ 1 MHz)))	for	0.3°	< θ ≤ 1°

pfd(θ) = −127.7 + 18 ∙ logθ	(dB(W/(m2 ∙ 1 MHz)))	for	1°	< θ ≤ 12.4°

pfd(θ) = −108 	(dB(W/(m2 ∙ 1 MHz))) 	for 	12.4°	< θ ≤ 90°

where θ is the angle of arrival of the radio-frequency wave (degrees above the horizon).

-

--

-

-

2.3	The pfd levels provided in §§ 2.1 and 2.2 above relate to the pfd and angles of arrival that shall be obtained using free-space propagation and attenuation due to the aircraft fuselage. Unless there is an available ITU-R Recommendation to calculate attenuation due to the aircraft fuselage in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz, the equations in the table below shall be used for the calculation of attenuation due to the aircraft fuselage in these frequency bands.

Fuselage attenuation model from Report ITU-R M.2221

| Lfuse(γ) = 3.5 + 0.25 · γ | dB | for | 0°≤ γ ≤ 10° |
| --- | --- | --- | --- |
| Lfuse(γ) = −2 + 0.79 · γ | dB | for | 10°< γ ≤ 34° |
| Lfuse(γ) = 3.75 + 0.625 · γ | dB | for | 34°< γ ≤ 50° |
| Lfuse(γ) = 35 | dB | for | 50°< γ ≤ 90° |

Reasons:	Equations is more understandable than the figure.

2.4	An aeronautical ESIM operating in the frequency bands 27.5-29.1 GHz and 29.5-30 GHz, or portions thereof, within the territory of an administration that has authorized fixed-service and/or mobile-service operation in the same frequency bands shall not transmit in these frequency bands without prior agreement of that administration.

Reasons:	The full range of the frequency bands for this agenda item should be reflected in this provision.

2.5	The maximum power in the out-of-band domain should be attenuated below the maximum output power of the aeronautical ESIM transmitter as described in Recommendation ITU-R SM.1541.

Annex 2 to draft new Resolution [A116] (WRC-23)

Methodology with respect to the examination referred to 
in Scenario 1 resolves 1.2.5

Reasons:	Now that the WP 4A and SG 4 were conducted, this NOTE is not necessary.

---

---

-

----

--

--

---

|  |  |  | -δγ |
| --- | --- | --- | --- |
|  |  |  | - |
| - |  |  | - |
|  |  |  | - |

|  |  |  | - |
| --- | --- | --- | --- |
|  |  |  | - |
|  |  |  | - |
|  |  | --- | ----- |
| - |  |  | - |
|  |  |  | - |

-

-

-

-

-

Re

-

-

-

-

-

-

-

--

-

---

-

|  | - |  |  |  |
| --- | --- | --- | --- | --- |
|  | - |  |  |  |

|  |  |  | - |  |
| --- | --- | --- | --- | --- |
|  |  | δ |  |  |

-

-

Re

-

-

-

-

--

-

--

-

-

| - |  |  |  |
| --- | --- | --- | --- |

-

-

| - |  |  |  |
| --- | --- | --- | --- |
|  |  | - |  |
|  |  | - |  |
|  |  | - |  |

--

-

-

-

-

-

-

-

-

# 1	Overview

The methodology below is a functional description to conduct examination of A-ESIM operating with non-GSO satellite systems and their conformity with power flux-density limits specified in Part II of Annex 1 in this Resolution.

# 2	A-ESIM parameters required for the examination

To conduct the relevant examination of A-ESIM and their conformity with respect to the pfd limits, the following parameters are required:

‒	Satellite network name

‒	A-ESIM peak antenna gain

‒	A-ESIM power density and bandwidth as given in Table A2-1

‒	Fuselage attenuation mask expressed as a function of the angle below the horizon of the A-ESIM based on ITU-R reports or recommendations.

# 3	Examination methodology

## 3.1	Introduction

An A-ESIM can operate at different locations defined by latitude, longitude and altitude. This methodology determines the maximum allowable power, Pj, for an A-ESIM transmitter communicating with a non-GSO FSS satellite to ensure compliance with the pre-established pfd limits to protect terrestrial services, at all positions, for a defined set of altitude ranges. The methodology derives the Pj taking into account the relevant loss and attenuation in the geometry considered.

The methodology then compares the computed Pj with the range of notified power for the A-ESIM emission. The minimum and the maximum powers values of the emission Pmin_emission,j and Pmax_emission,j of the A-ESIM are calculated from the data included in the Appendix 4 notification information of the non-GSO satellite system with which the A-ESIM communicates and from the A-ESIM characteristics.

A-ESIM are evaluated over a number of predefined altitude ranges in order to establish a number of Pj levels.

An examination by the Bureau should apply this methodology for the defined altitude range, to determine whether the A-ESIM operating under a given non-GSO satellite system complies with the pre-established pfd limits to protect terrestrial services.

## 3.2	Parameters and geometry

Considering a hypothetical GSO FSS network, Table 1 below provides an example of emissions that are included in one Group associated to the “UO” class of earth station transmitting in the 27.5-29.5 GHz band. Tables A2-2 to A2-4 provide additional assumptions and Figure A2-1 illustrates the geometry involved in the examination.

TABLE A2-1

Example of a group of A-ESIM emissions
(with reference to relevant Appendix 4 data fields)

| Emission No. | C.7.a Designation of emission | BWemission MHz | C.8.c.3 minimum power density dB(W/Hz) | C.8.a.2/C.8.b.2 maximum power density dB(W/Hz) |
| --- | --- | --- | --- | --- |
| 1 | 6M00G7W-- | 6.0 | −69.7 | −66.0 |
| 2 | 6M00G7W-- | 6.0 | −64.7 | −61.0 |
| 3 | 6M00G7W-- | 6.0 | −59.7 | −56.0 |

TABLE A2-2

Additional example assumptions

| ID | Parameter | Notation | Value | Unit |
| --- | --- | --- | --- | --- |
| 1 | Frequency assignment | f | 29.5 | GHz |
| 2 | Reference bandwidth of pfd mask | BWRef | 1.0 | MHz |
| 3 | A-ESIM antenna peak gain | Gmax | 37.5 | dBi |
| 4 | A-ESIM antenna gain pattern | - | As per Rec. ITU-R S.580 (see C.10.d.5.a) |  |

TABLE A2-3

Additional assumptions defined in the methodology

| ID | Parameter | Notation | Value | Unit |
| --- | --- | --- | --- | --- |
| 8 | A-ESIM minimum elevation angle towards GSO satellite | ε | 10 | degrees |
| 9 | Atmospheric attenuation | Latm | Computed with Rec. ITU-R P.676 (see note below) | dB |
| 10 | Angle of arrival of the incident wave on the Earth’s surface | δ | Specified by the pre-established sets of pfd limits, variable from 0° to 90° | deg |
| 11 | Minimum examination altitude | Hmin | 0.01 | km |
| 12 | Maximum examination altitude | Hmax | 15.0 | km |
| 13 | Examination altitude spacing | Hstep | 1.0 | km |
| 14 | Fuselage attenuation | Lf | Computed based on ITU-R Reports or Recommendations (see Table A2-4) | dB |

NOTE: The atmospheric attenuation is computed with Recommendation ITU-R P.676, with the mean annual global reference atmosphere as defined in Recommendation ITU-R P.835.

Figure A2-1

Geometry for the examination of compliance for two different A-ESIM altitudes

TABLE A2-4

Fuselage attenuation model

| Lfuse(γ) = 3.5 + 0.25 · γ | dB | for | 0°≤ γ ≤ 10° |
| --- | --- | --- | --- |
| Lfuse(γ) = −2 + 0.79 · γ | dB | for | 10°< γ ≤ 34° |
| Lfuse(γ) = 3.75 + 0.625 · γ | dB | for | 34°< γ ≤ 50° |
| Lfuse(γ) = 35 | dB | for | 50°< γ ≤ 90° |

Notes:

–	This fuselage attenuation model is based on measurements made at 14.2 GHz (see Fig. 3.6-14 in Rep. ITU-R M.2221-0).

–	Tables A2-5A and A2-5B are taken from Part II of Annex 3 of Resolution 169 (WRC-19). The reference bandwidth for the sets of pfd limits included in Table A2-5A and Table A2-5B are 1 MHz and 14 MHz, respectively.

TABLE A2-5A

Required conformance pfd mask for altitudes up to 3 km

pfd(δ) = −136.2	(dB(W/(m2  1 MHz)))	for	0°	≤ δ ≤ 0.01°

pfd(δ) = −132.4 + 1.9 ∙ log δ	(dB(W/(m2  1 MHz)))	for	0.01°	< δ ≤ 0.3°

pfd(δ) = −127.7 + 11 ∙ log δ	(dB(W/(m2  1 MHz)))	for	0.3°	< δ ≤ 1°

pfd(δ) = −127.7 + 18 ∙ log δ	(dB(W/(m2  1 MHz)))	for	1°	< δ ≤ 12.4°

pfd(δ) = −108 	(dB(W/(m2  1 MHz)))	for 	12.4°	< δ ≤ 90°

TABLE A2-5B

Required conformance pfd mask for altitudes above 3 km

pfd(δ) = −136.2	(dB(W/(m2 ∙ 1 MHz)))	for	0°	≤ δ ≤ 0.01°

pfd(δ) = −132.4 + 1.9 ∙ log δ	(dB(W/(m2 ∙ 1 MHz)))	for	0.01°	< δ ≤ 0.3°

pfd(δ) = −127.7 + 11 ∙ log δ	(dB(W/(m2 ∙ 1 MHz)))	for	0.3°	< δ ≤ 1°

pfd(δ) = −127.7 + 18 ∙ log δ	(dB(W/(m2 ∙ 1 MHz)))	for	1°	< δ ≤ 2°

pfd(δ) = −129.4 + 23.7 ∙ log δ	(dB(W/(m2 ∙ 1 MHz)))	for	2°	< δ ≤ 8°

pfd(δ) = −108	(dB(W/(m2 ∙ 1 MHz)))	for	8°	< δ ≤ 90.0°

## 3.3	Calculation algorithm

This section includes a step-by-step description of how the examination methodology would be implemented.

START

i)	For each A-ESIM altitude, it is necessary to generate as many δn angles (angle of arrival of the incident wave) as required in order to test the full compliance with the applicable set of pfd limits. The N angles δn must be comprised between 0° and 90° and have a resolution compatible with the granularity of the pre-established pfd limits. Each of the angles δn will correspond to as many N points on the ground.

ii)	For each altitude Hj = Hmin, Hmin + Hstep, …, Hmax:

a)	Set the altitude of the A-ESIM to Hj.

b)	Compute the angles below the horizon γj,n as seen from the A-ESIM for each of the N angles δn generated in i) using the following equation:

(2)

where Re is the mean earth radius.

c)	Compute the distance Dj,n, in km, for n = 1, …, N between the A-ESIM and the tested point on the ground:

(3)

d)	Compute the fuselage attenuation Lf j,n (dB) with n = 1, …, N applicable to each of the angles  γj,n computed in b) above.

e)	Compute the gaseous absorption Latm_j,n (dB) with n = 1, …, N applicable to each of the distances Dj,n computed in c) above, using the applicable sections of Recommendation ITU-R P.676.

iii)	a) 	For each altitude Hj = Hmin, Hmin + Hstep, …, Hmax, and each angle below the horizon γj,n, compute the maximum emission power in the reference bandwidth  Pj,n(δn, γj,n) for which the pfd limits are met using the following algorithm:

with Gtx(γj,n + ε) being the transmit antenna gain with the off-axis angle from the boresight, consisting of the summation of both angles  γj,n and minimum elevation angle ε of 10 degrees as defined in Table A2-3.

b)	Compute the minimum Pj across all values calculated at the previous step:

Pj = Min (Pj,n(δn, γj,n))

The output of this step is the maximum power in the reference bandwidth that can be used by the A-ESIM to ensure it complies with the pfd limits indicated in Table A2-5A or Table A2-5B, as applicable, with respect to all angles δn at the altitude Hj, and the elevation indicated in Table A2-3. There will be one Pj for each of the Hj altitudes considered.

The output of step b) is summarized in Table A2-7 below:

TABLE a2-7

Computed Pj values

| Hj (Altitude) | Pj (Maximum power in the reference bandwidth that can be used at minimum elevation) |
| --- | --- |
| (km) | dB(W/BW) |
| 0.01 | TBD |
| 1.0 | TBD |
| 2.0 | TBD |
| 2.99 | TBD |
| 4.0 | TBD |
| 5.0 | TBD |
| 6.0 | TBD |
| 7.0 | TBD |
| 8.0 | TBD |
| 9.0 | TBD |
| 10.0 | TBD |
| 11.0 | TBD |
| 12.0 | TBD |
| 13.0 | TBD |
| 14.0 | TBD |
| 15.0 | TBD |

c) 	For each altitude Hj = Hmin, Hmin + Hstep, …, Hmax, and each of the emission of the groups of emissions under examination, compute the minimum and the maximum powers of the emission in the reference bandwidth:

d)	For each of the emission of the groups of emissions under examination check if there is at least one altitude Hj for which:

The results of this check are illustrated in Table A2-8 below.

TABLE a2-8

Example comparison between Pj and (Pmin_emission,j; Pmax_emission,j)

| Emission No. | C.7.a Designation of emission | BWemission MHz | C.8.c.3 Minimum power density dB(W/Hz) | C.8.a.2/C.8.b.2 Maximum power density dB(W/Hz) | Lowest altitude Hj (km) for which Pmax_emission,j)>Pj > Pmin_emission,j |
| --- | --- | --- | --- | --- | --- |
| 1 | 6M00G7W-- | 6.0 | −69.7 | −66.0 | TBD |
| 2 | 6M00G7W-- | 6.0 | −64.7 | −61.0 | TBD |
| 3 | 6M00G7W-- | 6.0 | −59.7 | −56.0 | TBD |

e)	Based on the test detailed in iii) d) above applied to all emissions of the group under examination, the results of the Bureau’s examination for that group is favourable, after removing emissions that have failed the examination, otherwise it is unfavourable (i.e. all emissions have failed).

iv) 	The output of this methodology should, at a minimum, include:

–	those resulting parameters as contained in Table A2-7;

–	the examination results for each group;

–	for those cases when some emissions successfully pass and some do not, the examination results for resulting new group that includes only those emission(s) which successfully passed the examination.

END

Reasons:	Japan believes that the methodology to verify the pfd mask compliance for GSO A-ESIM agreed in the Resolution 169 (WRC-19) can be applied to non-GSO A-ESIM case as well, however, the aggregate interference effect from multi-A-ESIMs is necessary to be considered for the application of this methodology.

Annex 3 to draft new Resolution [A116] (WRC-23)

Provisions for non-GSO FSS systems1 transmitting to aeronautical and/or maritime ESIMs operating in or over an ocean in the frequency bands 
18.3-18.6 GHz and 18.8-19.1 GHz with respect to EESS (passive) 
operating in the frequency band 18.6-18.8 GHz 
(in accordance with resolves 1.1.6)

Option 1:

Non-GSO fixed-satellite space stations operating with an orbit apogee less than 20 000 km in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz with an aeronautical or maritime ESIM shall not exceed a pfd produced at the surface of the oceans across the 200 MHz of the frequency band 18.6-18.8 GHz, of −123 dB(W/(m² ∙ 200 MHz)). This value can be exceeded provided that the non-GSO fixed-satellite system does not exceed a pfd across the 200 MHz of the frequency band 18.6-18.8 GHz of −137 dB(W/(m² ∙ 200 MHz)) averaged over an area of 10 000 000 km² at the surface of the oceans.

Option 2:

Non-GSO fixed-satellite space stations operating with an orbit apogee less than 20 000 km in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz over oceans with aeronautical or maritime ESIM shall not exceed the following pfd values produced at the surface of the oceans across the 200 MHz of the 18.6-18.8 GHz band:

−123 dB(W/(m² · 200 MHz)) for non-GSO FSS space stations operating at orbital altitudes greater than 2 000 km;

−117 dB(W/(m² · 200 MHz)) for non-GSO FSS space stations operating at orbital altitudes between 1 000 km and 2 000 km;

−104 dB(W/(m² · 200 MHz)) for non-GSO FSS space stations operating at orbital altitudes below 1 000 km.

Option 3:

Any non-GSO fixed satellite space station operating in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz with (i) an orbit apogee less than 20 000 km (ii) communicating with an aeronautical or maritime ESIM over the ocean, and (iii) for which complete notification information has been received by the Radiocommunication Bureau after 1 January 2025, shall not exceed an unwanted emission power flux-density produced at the surface of the ocean in the 18.6-18.8 GHz band, based on the following piecewise equation:

| for N ≥ 10: | pfd = min(−77 − 10 * log(S), −110) | dB(W/(m² · 200 MHz)) |
| --- | --- | --- |
| for N < 10: | pfd = min(−67 − 10 * log(S) − 10 * log(N), −110) | dB(W/(m² · 200 MHz)) |

where S is the non-GSO fixed satellite space station 3 dB beam footprint area on the ground expressed in km² and N is the maximum number of co-frequency beams generated by the non-GSO fixed satellite system within a 10 000 000 km2 square on the Earth.

Option 1:

Annex 4 to draft new Resolution [A116] (WRC-23)

Required/Recommended ESIM capabilities

ESIM shall be designed with the following minimum capabilities:

In order to enable the ESIM to cease transmission when the conditions described are met, it is recommended the ESIM network be designed with appropriate capabilities. Table A4-1 describes applicable capabilities, with a justification for their requirement.

Option 1:

It is also important to note that the NCMC has a database of allowed power spectral density limits per angles (azimuth, elevation and skew), altitude and attitude that are critical to ensure pfd limits are met. The NCMC draws upon this comprehensive and detailed database of allowed levels and continually monitors feedback from the terminal to ensure emissions are fully compliant with regulatory limits.

Option 2:

Option 1:

For each ESIM, the NCMC should have a record of the location, the latitude, longitude and altitude, the transmit frequency, channel bandwidth and non-GSO satellite system with which the non-GSO ESIM communicates. This data can be made available to an administration or authorized agency for the purposes of detecting and resolving interference events.

Option 2:

Option 1:

Table a4-1

Minimum ESIM capabilities and justification

| Capability | Justification |
| --- | --- |
| GNSS (or other geolocation capabilities) | To assess the ESIM’s geographic location so the ESIM is aware when entering an administration’s territory that has not given authorization to cease transmissions accordingly. |
| Monitor loss of frequency lock | To anticipate an error in transmission frequency, which could potentially lead to interference out of assigned transmission band. |
| Monitor loss of LO signal | To anticipate an error in transmission frequency, which could potentially lead to interference out of assigned transmission band. |
| Internal power off/on/reset | For the ESIM to have the ability to self-power down after experiencing a fault condition, then restart or power back on when fault is resolved. |
| Disable/enable transmission and level adjustment | To cease, adjust and re-enable transmissions as necessary to mitigate interference or unauthorized transmissions. |
| Receive and execute commands from NCMC | To receive commands to enable/disable transmission from NCMC or other commands as necessary to mitigate interference or unauthorized transmissions. |

Option 2:

Table a4-1

Minimum ESIM capabilities and justification

| Capability | Justification |
| --- | --- |
| GNSS (or other geolocation capabilities) | Required to assess the ESIM’s geographic location so the ESIM is aware when entering an administration’s territory that has not given authorization and feedback to software to cease transmissions accordingly. |
| Monitor and control of the transmission frequency | Required to anticipate an error in transmission frequency, which could potentially lead to interference out of assigned transmission band. |
| Internal power off/on/reset | Required for the ESIM to have the ability to self-power down after experiencing a fault condition, then restart or power back on when fault is resolved. |
| Disable/enable transmission and level adjustment | Required to cease, adjust and re-enable transmissions as necessary to mitigate interference or unauthorized transmissions. |
| Receive and execute commands from NCMC | Required to receive commands to enable/disable transmission from NCMC or other commands as necessary to mitigate interference or unauthorized transmissions. |

Option 1:

Furthermore, it is recommended the ESIM have the ability to enter the states described in Table A4-2. These states ensure the ESIM is in the correct radio-interface state after some event (such as an initial boot or resuming operations after a fault) and can test system functionality is correct before radiating to avoid any transmission errors.

-

Option 1:

Table A4-2

ESIM states and events

| ESIM state | Radio-interface state | Corresponding event |
| --- | --- | --- |
| Non-valid | Emissions disabled | After power-on, until ESIM can receive commands from NCMC and no-fault conditions are present After any failure/fault During system checks |
| Initial phase | Emissions disabled | When waiting for a transmission enable or disable command from NCMC |
| Transmission enabled | Carrier-off | No carrier transmitted/need for carrier to be transmitted Receive synchronization is lost Pointing threshold is exceeded |
|  | Carrier-on | During transmission and ESIM is correctly pointed |
| Transmission disabled | Emissions disabled | When commanded by NCMC or ESIM automatically enters based on a “Cease Transmission” condition In locations where transmission is not permitted |

Option 2: Suppression of Table A4-2

APPENDIX 4 (REV.WRC-19)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Footnotes to Tables A, B, C and D

MOD	J/99A16/7#1886

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM,
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-23)

Option 1:

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.24 | COMPLIANCE WITH NOTIFICATION OF A NON-GSO SHORT DURATION MISSION |  |  |  |  |  |  |  |  |  | A.24 |  |
| A.24.a | a commitment by the administration that, in the case that unacceptable interference caused by a non-GSO satellite network or system identified as short-duration mission in accordance with Resolution 32 (WRC-19) is not resolved, the administration shall undertake steps to eliminate the interference or reduce it to an acceptable level Required only for notification |  |  |  |  | + |  |  |  |  | A.24.a |  |
| A.25 | COMPLIANCE WITH resolves 1.1.3 OF RESOLUTION 169 (WRC-19) |  |  |  |  |  |  |  |  |  | A.25 |  |
| A.25.a | a commitment that the ESIM operation would be in conformity with the Radio Regulations and draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.25.a |  |
| A.26 | COMPLIANCE WITH resolves 4 OF DRAFT NEW RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.26 |  |
| A.26.a | a commitment that, upon receiving a report of unacceptable interference, the notifying administration for the non-GSO FSS network with which ESIMs communicate shall follow the procedures in resolves 6 of draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.26.a |  |
| A.27 | COMPLIANCE WITH resolves 1.2.4 OF DRAFT NEW RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.27 |  |
| A.27.a | a commitment that aeronautical ESIMs would be in conformity with the pfd limits on the Earth’s surface specified in Part 2 of Annex 1 to draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.27.a |  |

Option 2:

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| A.20 | COMPLIANCE WITH resolves 1.1.4 OF RESOLUTION 169 (WRC-19) |  |  |  |  |  |  |  |  |  | A.20 |  |
| A.20.a | a commitment that the ESIM operation would be in conformity with the Radio Regulations and Resolution 169 (WRC-19) Required only for the notification of earth stations in motion submitted in accordance with Resolution 169 (WRC-19) |  |  |  | + |  |  |  |  |  | A.20.a |  |
| A.21 | COMPLIANCE WITH resolves 1.2.6 OF RESOLUTION 169 (WRC-19) |  |  |  |  |  |  |  |  |  | A.21 |  |
| A.21.a | a commitment that, upon receiving a report of unacceptable interference, the notifying administration for the GSO FSS network with which ESIMs communicate shall follow the procedures in resolves 4 of Resolution 169 (WRC-19) Required only for the notification of earth stations in motion submitted in accordance with Resolution 169 (WRC-19) |  |  |  | + |  |  |  |  |  | A.21.a |  |
| A.22 | COMPLIANCE WITH resolves 7 OF RESOLUTION 169 (WRC-19) |  |  |  |  |  |  |  |  |  | A.22 |  |
| A.22.a | a commitment that aeronautical ESIMs would be in conformity with the pfd limits on the Earth’s surface specified in Part II of Annex 3 to Resolution 169 (WRC-19) Required only for the notification of earth stations in motion submitted in accordance with Resolution 169 (WRC-19) |  |  |  | + |  |  |  |  |  | A.22.a |  |
| A.23 | COMPLIANCE WITH RESOLUTION 35 (WRC-19) |  |  |  |  |  |  |  |  |  | A.23 |  |
| A.23.a | a commitment stating that the characteristics as modified will not cause more interference or require more protection than the characteristics provided in the latest notification information published in Part I-S of the BR IFIC for the frequency assignments to the non-geostationary-satellite system |  |  |  |  | O |  |  |  |  | A.23.a |  |
| A.24 | COMPLIANCE WITH NOTIFICATION OF A NON-GSO SHORT DURATION MISSION |  |  |  |  |  |  |  |  |  | A.24 |  |
| A.24.a | a commitment by the administration that, in the case that unacceptable interference caused by a non-GSO satellite network or system identified as short-duration mission in accordance with Resolution 32 (WRC-19) is not resolved, the administration shall undertake steps to eliminate the interference or reduce it to an acceptable level Required only for notification |  |  |  |  | + |  |  |  |  | A.24a |  |
| A.25 | COMPLIANCE WITH resolves 1.1.1.1 OF RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.25 |  |
| A.25.a | a commitment that the ESIM operation would be in conformity with the Radio Regulations and Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.25.a |  |
| A.26 | COMPLIANCE WITH resolves 1.1.5 OF RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.26 |  |
| A.26.a | a commitment that the ESIM operation would be in conformity with the resolves 1.1.5 of Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.26.a |  |
| A.27 | COMPLIANCE WITH resolves 4 OF RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.27 |  |
| A.27.a | a commitment that, upon receiving a report of unacceptable interference, the notifying administration for the GSO FSS network with which ESIMs communicate shall follow the procedures in resolves 5 of Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.27.a |  |
| A.28 | COMPLIANCE WITH resolves 1.2.2 OF RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.28 |  |
| A.28.a | a commitment that aeronautical ESIMs would be in conformity with the pfd limits on the Earth’s surface specified in Part II of Annex 1 to Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.28.a |  |

Option 3:

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK OR SYSTEM, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network or system subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network or system not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network or system | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.24 | COMPLIANCE WITH NOTIFICATION OF A NON-GSO SHORT DURATION MISSION |  |  |  |  |  |  |  |  |  | A.24 |  |
| A.24.a | a commitment by the administration that, in the case that unacceptable interference caused by a non-GSO satellite network or system identified as short-duration mission in accordance with Resolution 32 (WRC-19) is not resolved, the administration shall undertake steps to eliminate the interference or reduce it to an acceptable level Required only for notification |  |  |  |  | + |  |  |  |  | A.24.a |  |
| A.25 | COMPLIANCE WITH resolves 1.1.3 OF RESOLUTION 169 (WRC-19) |  |  |  |  |  |  |  |  |  | A.25 |  |
| A.25.a | a commitment that the ESIM operation would be in conformity with the Radio Regulations and draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.25.a |  |
| A.26 | COMPLIANCE WITH resolves 4 OF DRAFT NEW RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.26 |  |
| A.26.a | a commitment that, upon receiving a report of unacceptable interference, the notifying administration for the non-GSO FSS network with which ESIMs communicate shall follow the procedures in resolves 6 of draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.26.a |  |
| A.27 | COMPLIANCE WITH resolves 1.2.4 OF DRAFT NEW RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.27 |  |
| A.27.a | a commitment that aeronautical ESIMs would be in conformity with the pfd limits on the Earth’s surface specified in Part 2 of Annex 1 to draft new Resolution [A116] (WRC-23) Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.27.a |  |
| A.28 | COMPLIANCE WITH resolves 1.1.6 OF DRAFT NEW RESOLUTION [A116] (WRC-23) |  |  |  |  |  |  |  |  |  | A.28 |  |
| A.28.a | an indication of whether the LEO system with which the ESIMs communicate employs a frequency reuse scheme with at least three colours. Required only for the notification of earth stations in motion submitted in accordance with draft new Resolution [A116] (WRC-23) |  |  |  |  | + |  |  |  |  | A.28.a |  |

…

SUP	J/99A16/8#1887

RESOLUTION 173 (WRC-19)

Use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space) by 
earth stations in motion communicating with non-geostationary space stations 
in the fixed-satellite service

_______________

The fourth altitude value (H4) computed in accordance with this Hstep is adjusted to 2.99 km to facilitate the examination of compliance with the two sets of predefined pfd values indicated in Table 5A and Table 5B.

1 	These provisions do not apply to non-GSO systems using orbits with an apogee less than 2 000 km that employ a frequency reuse factor of at least three.