# Rules concerning

## APPENDIX  4 to the RR

An. 1

ITEM 3A1

When submitting a notice within the procedure of Article 11, the administrations are required to provide information on the call sign or other identification used, as requested by Nos. 19.7 to 19.9 and 19.29. Bearing in mind the variety of special arrangements concluded between administrations concerning notification of frequency assignments, the Board instructed the Bureau not to perform systematic control of the call signs referred to in No. 19.29 during the validation and examination of the notice. Nevertheless, if non-conformity of the call sign with the international call series is identified, the notifying administration is to be informed thereof.

An. 2

(ADD RRB16/58)

Commitment regarding the implementation or resolves 1.4 of Resolution 156 (WRC-15)

The Board noted that resolves 1.5 of Resolution 156 (WRC-15) requests administrations to submit to the Bureau a commitment for implementation of resolves 1.4 of the same resolution. The Board further noted that this data element is mandatory in the case of submission of notification or coordination of a geostationary satellite network using the frequency bands 19.7-20.2 GHz and 29.5-30.0 GHz for earth stations in motion communicating in the fixed-satellite service.

However this data element is missing in Appendix 4. To correct this inconsistency, the Board decided that the Administrations will be requested to provide, in addition to the relevant characteristics listed in Appendix 4, and in accordance of resolves 1.5 of Resolution 156 (WRC-15) a commitment for implementation of resolves 1.4 of the same Resolution. The Bureau will then subsequently take account of this data element in its completeness check of the submitted data and in the examination under Nos. 9.35 and 11.31 of the Radio Regulations.

A.17.d		(ADD RRB16/58)

WRC-15 modified item A.17.d to submit the mean power flux-density (pfd) produced at the Earth’s surface by any space borne sensor for the frequency band 9 900 – 10 400 MHz for satellite system operating in the Earth exploration-satellite service (active) as defined in Table 21-4. As the limits are dependent on the angles of arrival, the mean pfd has to be provided for each angle of arrival. The formula defining the mean pfd defined in Table 21-4 is given in No. 21.16.8. The Bureau may calculate the mean power flux-density based on the angles of arrival if the information on the necessary bandwidth (item C.7a), which is not currently required for active or passive sensors, is submitted. The necessary bandwidth information is also required for the Bureau to examine the conformity of the submitted frequency assignments with respect to No.5.474A.

In view of the above the Board decided that the Administrations shall provide, in addition to the relevant characteristics listed in Appendix 4, the SAR emission bandwidth information under C.7.a (necessary bandwidth) for active sensors operating in the Earth exploration-satellite service (active) in the frequency band 9 900-10 400 MHz instead of submitting the mean pfd. The Bureau will then subsequently take account of this data element in the examination under No. 11.31 of the Radio Regulations.     (ADD RRB16/58)

A.18 a)

The Board noted that the description of Annex 2 of Appendix 4, § A.18 a) corresponds to the commitment required from an administration in the case of the possible communication of aircraft earth stations in the secondary aeronautical mobile-satellite service with space stations in the fixed-satellite service, in accordance with No. 5.504A. The Board further noted that this data element is mandatory in the case of submission of notification or coordination of a geostationary or a non-geostationary satellite network.

However, this data element is also required to check under No. 11.31 the compliance with respect to No. 5.504A of the notification of an aircraft earth station in the secondary aeronautical mobile-satellite service communicating with a space station in the fixed-satellite service. This requirement was probably omitted by inadvertence at WRC-03.

To correct this inconsistency, the Board decided that the Administrations will be requested to provide, in addition to the relevant characteristics listed in Appendix 4, the data element described in § A.18 a) of Annex 2 of Appendix 4, when submitting the notification information of an aircraft earth station in the secondary aeronautical mobile-satellite service communicating with a space station in the fixed-satellite service in accordance with No. 5.504A. The Bureau will then subsequently take account of this data element § A.18 a) in its completeness check of the submitted data.

B.4 a)

When submitting a notice within the applicable procedures of Articles 9 or 11, in order to better describe the power flux-density pattern on the surface of the Earth resulting from the emission of a space station abroad a non-GSO satellite in circular orbit, the following optional information can be provided together with the other data contained in Appendix 4:

Appendix 4, Annex 2A, § B.4 a) (non-GSO space station antenna characteristics)

1	In addition to the information currently contained in Appendix 4 to be provided under this item, if appropriate, indicate:

1.1	in the case of a transmitting space station aboard a non-GSO satellite in a circular orbit that is intended to communicate with earth stations via a transmitting antenna pointing in a direction that is fixed with respect to the satellite, the maximum isotropic gain (dBi) and the gain contours plotted in a radial projection from the satellite onto a plane perpendicular to the axis from the centre of the Earth to the satellite. The space station antenna gain contours shall be drawn as isolines of the isotropic gain at least for – 2, – 4, – 6, –10, and – 20 dB and at 10 dB intervals thereafter, as necessary, relative to the maximum antenna gain, when any of these contours is located either totally or partially within the limit of visibility of the Earth from the given non-GSO satellite;

1.2	in the case of a space station aboard a non-GSO satellite in a circular orbit where a steerable beam is used, data on the antenna radiation characteristics as follows:

–	if the effective boresight area (see No. 1.175) is identical with the global or nearly global service area, provide only the maximum isotropic antenna gain (dBi) that is then applicable to all points on the surface of the Earth;

–	if the effective boresight area (see No. 1.175) is less than the global or nearly global service area, provide the maximum isotropic gain and the effective gain contours (see No. 1.176) as defined above.

2	The additional information detailed in § 1.1 and 1.2 above is considered as optional. When examining such a case, the Bureau shall use the more detailed information to calculate power flux-density values if it is provided; if it is not the calculation shall be made as at present and be based on the maximum e.i.r.p. transmitted.

____________________

_______________