# Rules concerning

## APPENDIX  30 to the RR

(Rules are arranged by paragraph numbers of Appendix 30)

Art. 3

Execution of the provisions and associated Plans

3.1

For the footnote of § 3.1 see comments made under the Rules of Procedure concerning No. 5.492.

Art. 4

Procedures for modifications to the Region 2 Plan
or for additional uses in Regions 1 and 3

4.1.1 a)
and 4.1.1 b)

1	In determining those administrations of Regions 1 and 3 that may be affected, the proposed new or modified assignment to the List is examined with respect to the Regions 1 and 3 Plan and List as they exist at the date of receipt of the proposed new or modified assignment to the List, including the other proposed new or modified assignments to the List received before that date (whether the procedure of Article 4 is complete or not). The examination consists of ensuring that the limits of Section 1 of Annex 1 of Appendix 30 are not exceeded. Account is also taken of any time-limited new or modified assignments to the List in accordance with § 4.1.13.

2	Following the introduction by the 1983 Conference of the grouping concept for Region 2 (Articles 9 and 10 of Appendices 30A and 30 respectively) and further to the decision of WARC Orb-88 to apply the grouping concept to the Regions 1 and 3 feeder-link Plans (Article 9A of Appendix 30A), the ex-IFRB decided to extend this concept to the 1977 Conference BSS Plan. WRC-2000 endorsed this decision and decided to include the same grouping concept definition in Articles 11 and 9A of Appendices 30 and 30A respectively.

3	The Board’s understanding of the group concept is that in the interference calculation to assignments that are part of the group, only the interference contribution from assignments that are not part of the same group are to be considered. On the other hand, for the interference calculation from assignments belonging to a group into assignments that are not part of the same group, only the worst interference contribution from that group is to be taken into consideration.

4	According to resolves 5 of Resolution 548 (WRC-03)*, in the processing of Regions 1 and 3 Article 4 submissions received after 2 June 2000 for identification of affected administrations, each network in a group shall be examined separately without taking into account the interference contribution from the other networks in the group. This means that the concept of calculating the worst interference contribution from the assignments that are part of a group to assignments that are not part of the same group, as indicated in Article 11 (column 14) of Appendix 30, is not applicable to the grouped networks for the identification of affected administrations in accordance with § 4.1.5 of that Appendix. In applying § 4.1.11, the application of this method to networks received before 3 June 2000 shall not result in additional coordination requirements for those networks.

5	In order to implement this separate examination and calculate the interference effect of an Article 4 network under examination independent of other networks in the group in accordance with resolves 5 of Resolution 548 (WRC-03)*, the Board concluded that the following method should be used.

Identification of affected administrations should be carried out without taking into account the interference contribution from the Plan and the List assignments grouped with assignments of an Article 4 network under examination based on the reference situation established without taking into account the interference contribution from those grouped assignments.

In addition, it is understood that this separate examination is not applicable in the case of other examinations, such as e.g. under Sections 4 and 6 of Annex 1 to Appendix 30, which do not use the grouping concept, for identifying affected administrations under § 4.1.5 of Article 4 of Appendix 30.

6	For the Regions 1 and 3 Plan and List, in accordance with resolves 1 of Resolution 548 (WRC-03)* and the decision of the Plenary of WRC-03, the Board concluded that grouping of networks separated by more than 0.4° in the geostationary arc is not permitted in the List except for application of § 4.1.27. However, grouping of networks separated by more than 0.4 may be used before the inclusion of the assignments in the List to modify the orbital position of a network.

For the Region 2 Plan, with respect to § 4.2.3 c), the Board did not find any regulatory basis to extend the use of groupings involving multiple orbital positions (except for the case of 0.4 orbital separation which was allowed for clusters within the Region 2 Plan and its subsequent modifications).

In the case of a request for replacement of an assignment/entry in the Region 2 Plan of an administration, the implementation of § 2.2 of the Rules of Procedure relating to § 4.2.6 of Appendix 30 requires the processing of the modified assignment requested by that administration under Article 4 of that Appendix based on the following conditions:

–	no interference effect from the initial assignment of the requesting administration is considered in the overall equivalent protection margin calculations of the modified assignment requested by that administration, and vice versa; and

–	no aggregate interference effect from the subject initial assignment and the modified assignment of the requesting administration is considered in the overall equivalent protection margin calculations of other assignments, but only the worst interference effect from the two is to be considered.

The above-mentioned conditions apply only during the time-period afforded for the processing of the modified assignment under Article 4 of that Appendix. After that time-period, either the subject initial assignment or the modified assignment of the requesting administration will remain in the Plan, depending on the successful application or otherwise of the Article 4 procedure for this modified assignment.

4.1.1 c)

In determining those administrations of Region 2 that might be affected, the proposed new or modified assignment to the Regions 1 and 3 List is examined with respect to the Region 2 Plan as it exists at the date of receipt of the proposed new or modified assignment including the proposed modifications of the Region 2 Plan received before that date (whether the procedure of Article 4 is complete or not). The examination will consider only those administrations having assignments whose necessary1 bandwidth overlaps the necessary1 bandwidth of the proposed new or modified assignment to the Regions 1 and 3 List. The Region 2 administration is identified as having services which are considered to be affected when the power flux-density at any test point which lies within the service area of the Region 2 assignment under examination exceeds the limits specified in Section 3 of Annex 1 to Appendix 30.

4.1.1 d)

1	This paragraph is understood by the Board as being intended to protect terrestrial services in any territory or part of a territory in the three Regions where this territory or part of a territory is not covered by a broadcasting-satellite assignment in a given necessary1 bandwidth. Therefore the proposed new or modified assignment to the Regions 1 and 3 List should take account of terrestrial stations in all Regions.

2	For terrestrial stations in all Regions the limit for the power flux-density not to be exceeded by the proposed new or modified assignment to the Regions 1 and 3 List is that specified in Section 4 of Annex 1 to Appendix 30. The agreement of an administration is required when a pfd excess exists over some part of its territory, unless the necessary1 bandwidth of the examined assignment is completely within the necessary1 bandwidth(s) of one or more assignments2 of the potentially affected administration in the Region 2 Plan or in the Regions 1 and 3 Plan or List and the area of pfd excess is inside the service area(s) of those assignments. In the absence of a defined service area contour, the area on the surface of the Earth within the –3 dB contour shall be considered as the service area of those assignments in this examination.

3	The Bureau, in applying Section 4 of Annex 1 shall, where applicable, compare the power flux-density values resulting from the proposed new or modified assignments to the Regions 1 and 3 List with those values in the Regions 1 and 3 Plan or List as appropriate. If it is not possible to do so, the Bureau should use the absolute limit expressed in Section 4 of Annex 1 to that Appendix.

4.1.1 e)

1	This examination shall consider only those administrations having assignments to FSS space stations whose necessary3 bandwidth overlaps the necessary3 bandwidth of the proposed new or modified assignment to the Regions 1 and 3 List.

2	Under this paragraph an assignment recorded in the Master Register shall be understood as defined in § 1 a), 1 b), 1 c) and 1 cbis) of Appendix 5.

3	In the case of inclusion of a new assignment in the Regions 1 and 3 List, different from the frequency assignments in the Plan or List for Regions 1 and 3 as established by WRC-2000, the limit prescribed in § 3 or Note 1 of Section 6 of Annex 1, as appropriate, shall be applied.

4	The Bureau, in applying Section 6 of Annex 1 shall, where applicable, compare the power flux-density values resulting from the proposed new or modified assignments to the Regions 1 and 3 List with those values in the Regions 1 and 3 Plan or List as appropriate. If it is not possible to do so, the Bureau should use the absolute limit expressed in Section 6 of Annex 1 to that Appendix.

4.1.3

1	In the event that the Bureau cancels a frequency assignment in application of § 5.3.2 of Article 5 of this Appendix, the corresponding assignment, which has been submitted either under § 4.2.6 (except in the case of a request for replacement of an assignment in the Region 2 Plan) and entered in the Region 2 Plan, or under § 4.1.3 and entered in the Regions 1 and 3 List, shall also be removed from the Plan or the List according to the case. The Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned cancellation.

2	See also Rules of Procedure concerning Receivability of the Forms of Notice.

4.1.7

Any request by an administration to be included in the list of administrations to be published shall be based only on technical reasons to be verified using Annex 1 as well as other relevant Annexes. If this indicates that the requesting administration should have been included in the list, the Bureau will include it; otherwise the requesting administration will be informed that its name will not be published, it being left to the notifying administration to consider if it is appropriate to take the request into account.

4.1.7bis

The agreement referred to in § 4.1.7bis is the agreement of the administrations identified under § 4.1.1 and of those under § 4.1.7 which have been confirmed by the Bureau using the appropriate criteria.

4.1.8

An administration which has only requested additional information in accordance with § 4.1.8 or § 4.2.12 will not be considered by the Bureau to have submitted comments in accordance with § 4.1.10 or § 4.2.14 respectively.

4.1.11

See also comments under § 4.1.3 and 4.2.6 and Rules relating to the Receivability of the Forms of Notice.

Note:  WRC-15 took the decision related to the RoP on paragraph 4.1.11 of RR Appendices 30 and 30A during the 8th Plenary, Par. 1.39 to 1.42 of Doc. CMR15/505, approval of Doc. CMR15/416 in relation to Section 3.2.6.4 of Doc. 4 (Add2) (Rev1), as follows:

“In Section 3.2.6.2 of Doc. 4 (Add2) (Rev1), the Director described the current practice of the Bureau in examining Part B submissions received under § 4.1.12 of Appendices 30 and 30A:

The Bureau identifies a list of administrations whose assignments are considered as being affected and receiving more interference as a result of the modification than that produced by the initial proposal in accordance with § 4.1.11. The Bureau then requests the notifying administration to modify the submitted characteristics in order to eliminate the above-mentioned identification or to apply again the provisions of § 4.1 of Appendices 30 and 30A.

In reply to the Bureau’s request, some administrations have provided the Bureau with the agreement of the administration identified under § 4.1.11.

As the agreement to accept more interference has been provided and § 4.1.11 does not explicitly prevent this possibility, the Bureau has not rejected such agreements.

WRC-15 endorsed the current BR practice outlined in this section.”           (MOD RRB17/74)

4.1.15

The second part of these paragraphs applies only to those assignments for which the procedure of Article 4 has been successfully applied, i.e., all administrations identified by the Bureau in application of § 4.1.5 or 4.2.8 and § 4.1.7 or 4.2.10 have either given their agreement or failed to comment on the proposed new or modified assignment to the Regions 1 and 3 List or on the proposed modification to the Region 2 Plan.

The Bureau shall update the reference situation of the Regions 1 and 3 Plan and List or of the Region 2 Plan entries and of those networks which are the subject of requests for new or modified assignment to the Regions 1 and 3 List or for Region 2 Plan modifications which are still at the stage of application of Article 4. Nevertheless, the Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned update.

4.1.23

If the assignments in question were deleted from the Regions 1 and 3 List or the Region 2 Plan, the Bureau shall update the reference situation of the assignments in the Regions 1 and 3 Plan and List, or in the Region 2 Plan and of the assignments under Article 4 procedure and inform all administrations of the action taken together with Special Sections published as a result of cancellation of frequency assignments from the Regions 1 and 3 List or the Region 2 Plan. The Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned cancellation.

4.2.1 a)

This paragraph refers to the modification in the sense of a change to “the characteristics of any of its frequency assignments to a space station in the broadcasting-satellite service (BSS) which are shown in the Region 2 Plan”. The Plan as it appears in Article 10 of Appendix 30 contains only eight characteristics, while Annex 2 contains a greater number of characteristics which were used by the RARC-SAT-R2 (Geneva, 1983) Conference to establish the Plan. Among these characteristics only one, the energy dispersal (former Annex 2, § 14 h), now item C.9 b) 8) of Annexes 2A and 2B of Appendix 4), is referred to in the footnote of § 4.2.1. The Board considers that modifications of characteristics other than those listed in Article 10 of Appendix 30 may be considered as modifications to the Plan. These other characteristics are listed in the Rules of Procedure relating to § 5.2.1 b) of Article 5 of Appendix 30.

See also the last paragraph of Rules of Procedure relating to § 4.2.3 d) and 4.2.3 e).

See also Rules of Procedure relating to § 4.2.6.

4.2.1 b)

See Rules of Procedure relating to § 4.2.1 a) above.

See also Rules of Procedure relating to § 4.2.6.

4.2.1 c)

When an administration cancels an assignment from the Region 2 Plan under this paragraph, or when the Bureau, in applying § 4.2.6 deletes an assignment from the Plan, the reference situation of the Plan assignments and those in the process of modification would be updated. The Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned cancellation.

4.2.3 a)
and 4.2.3 b)

In determining the administrations of Region 1 that might be affected, the proposed modification of the Region 2 Plan is examined with respect to the Regions 1 and 3 Plan and List as it exists at the date of receipt of the proposed modification including all proposed new or modified assignments to the Regions 1 and 3 List received before that date (whether the procedure of Article 4 is complete or not). The examination will identify only those administrations having assignments whose necessary4 bandwidth overlaps the necessary4 bandwidth of the proposed modification of the Region 2 Plan. An administration of Region 1 is identified as having services which might be affected when the power flux-density at any test point which lies within the service area of the Region 1 assignment under examination exceeds the limits specified in § 3 of Annex 1 to Appendix 30.

4.2.3 c)

1	In determining those administrations of Region 2 that may be affected, the proposed modification is examined with respect to the Region 2 Plan as it exists at the date of receipt of the request for modification including the proposed modifications received before that date (whether the procedure of Article 4 is complete or not). The examination consists of ensuring that the limits of § 2 of Annex 1 of Appendix 30 are not exceeded. Account is also taken of any time-limited modifications to the Plans in accordance with § 4.2.17.

2	According to Resolution 42 (Rev.WRC-03)*, the Board decided that, when applying this paragraph, the Bureau shall not take account of the interim systems.

3	For considerations related to application of the Group concept see Rules of Procedure related to § 4.1.1 a) and 4.1.1 b).

4.2.3 d)

1	As indicated in the Rules of Procedure relating to § 4.1.1 d), a modification to the Region 2 Plan should take account of terrestrial stations in all Regions.

2	For terrestrial stations in all Regions, the limit for the power flux-density not to be exceeded by the proposed modification to the Region 2 Plan is specified in § 4 of Annex 1 to Appendix 30. The agreement of an administration is required when a power flux-density excess exists over some part of its territory, unless the necessary5 bandwidth of the examined assignment is completely within the necessary5 bandwidth(s) of one or more assignments6 of the potentially affected administration in the Region 2 Plan or in the Regions 1 and 3 Plan or List and the area of power flux-density excess is inside the service area(s) of those assignments. In the absence of a defined service area contour, the area on the surface of the Earth within the –3 dB contour shall be considered as the service area of those assignments in this examination.

3	The Bureau, in applying § 4 of Annex 1 shall, where applicable, compare the power flux-density values resulting from the proposed modification to the Region 2 Plan with those values in the Region 2 Plan. If it is not possible to do so, the Bureau should use the absolute limit expressed in § 4 of Annex 1 to that Appendix.

4.2.3 e)

1	This examination shall consider only those administrations having assignments to FSS space stations whose necessary5 bandwidth overlaps the necessary5 bandwidth of the proposed modification to the Region 2 Plan.

2	Under this paragraph, an assignment recorded in the Master Register shall be understood as defined in § 1 a), 1 b), 1 c) and 1 cbis) of Appendix 5.

3	In the case of inclusion of a new assignment to the Region 2 Plan, different from the frequency assignments in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference, the limits prescribed in § 3 of Section 6 and in the second indent of Section 7 of Annex 1 shall be applied.

4	The Bureau, in applying Sections 6 and 7 of Annex 1 shall, where applicable, compare the power flux-density and T/T values, respectively, resulting from the proposed modification to the Region 2 Plan with those values in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference. If it is not possible to do so, the Bureau should use the absolute limit expressed in § 6 and 7 of Annex 1 to that Appendix.

4.2.3 f )

1	Until there is a Plan for Region 3 for the band 12.5-12.7 GHz, this examination shall consider only those administrations of Region 3 having broadcasting-satellite service assignments whose necessary7 bandwidth overlaps the necessary7 bandwidth of the proposed modification to the Region 2 Plan.

2	Under this paragraph, an assignment recorded in the Master Register shall be understood as defined in § 1 a), 1 b), 1 c) and 1 cbis) of Appendix 5.

4.2.6

See Rules of Procedure relating to § 4.1.3.

4.2.10

See Rules of Procedure relating to § 4.1.7.

4.2.11

The agreement referred to in § 4.2.11 is the agreement of the administrations identified under § 4.2.3 and of those under § 4.2.10 which have been confirmed by the Bureau using the appropriate criteria.

4.2.12

See Rules of Procedure relating to § 4.1.8.

4.2.15

See Rules of Procedure relating to § 4.1.11.

4.2.19

See Rules of Procedure relating to § 4.1.15.

4.2.24

See Rules of Procedure relating to § 4.1.23.

Art. 5

# Notification, examination and recording

5.2.1 b)

1	The Board has considered the question whether the examination with respect to conformity with the Plan8 means only the columns of Articles 10 and 11 of Appendix 30, as updated or whether it also includes an examination with respect to the technical criteria given in Annex 5 to Appendix 30 which were used for the establishment of the Plans. The Board concluded that some of the technical criteria contained in Annex 5 need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Plan is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the Plan concerned as updated (see § 3.1 of Article 3). If the characteristics are different then the examination under § 5.2.1 d) is carried out. For the items below, any characteristics for which the procedure of Article 4 has been successfully applied could be notified;

b)	to ensure that the protection criteria resulting from the Region 2 Plan8, or from the Regions 1 and 3 Plan and List, as appropriate, are not exceeded. To this effect, the following characteristics are examined:

–	beam identification (as indicated in Columns 1 and 2 of Articles 10 and 11, respectively of Appendix 30);

–	nominal orbital position (as indicated in Columns 2 and 3 of Articles 10 and 11, respectively of Appendix 30);

–	channel number/frequency (as indicated in Column 3 of Article 10 and in column 5 of the Table entitled “Minimum equivalent protection margin of assignments in the Regions 1 and 3 Plan (sorted by orbital position)” of Article 11 of Appendix 30);

–	boresight coordinates (as indicated in Column 4 of Articles 10 and 11 of Appendix 30);

–	in the case of an elliptical beam:

–	antenna beamwidth (as indicated in Column 5 of Articles 10 and 11 of Appendix 30),

–	ellipse orientation (as indicated in Columns 6 and 5 of Articles 10 and 11, respectively of Appendix 30),

–	antenna rotational accuracy (same as or better than that of § 3.14 of Annex 5 to Appendix 30);

–	polarization (as indicated in Columns 7 and 10 of Articles 10 and 11, respectively of Appendix 30);

–	power plus co-polar antenna gain (as indicated in Columns 8 and 11 of Articles 10 and 11, respectively of Appendix 30), and in the case of shaped beam the cross-polar antenna gain (as indicated in Column 8 of Article 11 of Appendix 30);

–	service area (test points shall be located within the service area);

–	class of emission and bandwidth (as indicated in Column 12 of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 and 3.8 of Annex 5 to Appendix 30);

–	antenna characteristics (same as or better than those indicated in Columns 6 or 7 as appropriate of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise same as or better than Fig. 9 or 10 as appropriate of Annex 5 to Appendix 30);

–	antenna pointing accuracy (same as or better than that referred to in § 3.14 of Annex 5 to Appendix 30);

–	station keeping tolerance (same as or better than that mentioned in § 3.11 of Annex 5 to Appendix 30);

–	modulation characteristics (same as in Column 12 of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 of Annex 5 to Appendix 30);

–	energy dispersal (same as § 3.18 of Annex 5 to Appendix 30);

–	the power flux-density identified in Note 10 of the Region 2 Plan or in Note 5 of the Regions 1 and 3 Plan, to determine whether the limits are met or whether there is an agreement with the affected administrations.

2	The Board has considered the question whether the examination with respect to conformity with the Regions 1 and 3 List means only the columns of the Tables in Part I of Annex 2 to Resolution 542 (WRC-2000)*, as updated, or whether it also includes an examination with respect to the technical characteristics published by the Bureau for each network of the List in the corresponding Part B Special Section of the Weekly Circular or the IFIC. The Board concluded that all technical characteristics published in the Part B Special Section for a given network need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the List is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network. If the characteristics are different then the examination under § 5.2.1 d) is carried out;

b)	to ensure that the protection criteria resulting from the Regions 1 and 3 Plan and List are not exceeded. To this effect, the characteristics specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network are examined.

5.2.1 d)

1	If an administration notifies any assignment with characteristics different from those listed in the Rules of Procedure related to § 5.2.1 b) of Article 5 of Appendix 30, and those allowed in § 5.2.1 d) of the same Article, a calculation is undertaken by the Bureau to determine if the proposed new characteristics would increase the interference level caused to other assignments in the appropriate Regional Plan, in the Regions 1 and 3 List, in the same service or in another service sharing the same frequency bands.

1.1	With respect to the compatibility of the proposed new characteristics with other assignments of the same Regional Plan and List, as appropriate, the increase of the interference will be checked by comparing the equivalent protection margin/overall equivalent protection margin values of these other assignments, which result from the use of the proposed new characteristics of the network in question on the one hand, and those obtained with the previous9 characteristics of the network in question, on the other hand. These equivalent protection margin/overall equivalent protection margin calculations are performed under the same technical assumptions and conditions, taking into account the orbital separation limit of  9 for assignments in the Regions 1 and 3 Plan and List. A more detailed analysis of the interference situation could also be required by using single entry C/I values in order to identify the assignments of the network in question which are causing the increase of the interference.

In addition, in the case of Regions 1 and 3, the notified assignments with new characteristics for the network in question are examined with respect to their compliance with the power flux-density hard-limit defined in § 1 of Annex 1 to Appendix 30, or, as the case may be, with respect to their compliance with the power flux-density level of the corresponding assignments in the Plan or in the List if those assignments were adopted by WRC-2000 with power flux-density level(s) higher than the above-mentioned power flux-density hard-limit.

1.2	With respect to the compatibility with other inter-regional assignments in the same service or assignments in another service sharing the same frequency bands, as appropriate, the increase of the interference will be checked by calculating the power flux-density value, produced by the proposed new characteristics at any test-point or within the service area of the other assignments or by calculating the T/T value in accordance with the method given in Case II of Appendix 8, and by comparing the resulting power flux-density or T/T values, according to the case, with those obtained with the previous9 characteristics of the subject assignment.

1.3	Should the results of the calculations described in § 1.1 and 1.2 above indicate that the proposed new characteristics increase the interference to other assignments/services, the Bureau would reach an unfavourable finding with respect to § 5.2.1 d) of Article 5 of Appendix 30 and proceed accordingly.

2	With respect to the fifth indent of § 5.2.1 d), in the case of administrations of Region 2 the orbital position shall be examined to ensure compliance with the cluster concept (§ B of Annex 7 to Appendix 30 and § 4.13.1 of Annex 3 to Appendix 30A) as follows:

–	if the orbital position is identical with that shown in the Plan, no further agreements are necessary;

–	however, if the orbital position is different from that contained in the Plan but it is in the same cluster, then only the agreement of administrations having assignments in the same cluster is necessary. The clusters are listed in the Attachment 1 to the present Rules of Procedure concerning Appendix 30. Appendices 30 and 30A do not contain any paragraph indicating the procedure to be followed for this agreement. The task of the Bureau in this respect is to ensure that the agreement of the administrations concerned is indicated in the notice; otherwise it considers the assignment to be not in conformity with the Plan.

3	See comments under No. 5.492.

5.2.2.1

This paragraph implicitly relates to the cases where the Bureau reaches a favourable Finding with respect to § 5.2.1 a) and § 5.2.1 c) and an unfavourable Finding with respect to § 5.2.1 b) but a favourable Finding with respect to § 5.2.1 d). In this event, the frequency assignment shall be recorded in the Master Register.

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.WRC-03)* for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable Finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable Finding with respect to § 5.2.1 b) and 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this Finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

An. 1

Limits for determining whether a service of an administration is affected
by proposed modifications to the Region 2 Plan or by proposed
new or modified assignments to the Regions 1 and 3 List

1

a)	Test points

1	In examining a proposed modification, all test points communicated to the Bureau by administrations are used. These test points are periodically published by the Bureau together with the updated reference situation of the Plan(s) and List(s).

b)	Implementation of the power flux-density limit referred to in the first paragraph of Section 1 of Annex 1 to Appendix 30

The power flux-density limit of –103.6 dB(W/(m2  27 MHz)) which is indicated in the first paragraph of Section 1 of Annex 1 to Appendix 30 was established in order to protect BSS assignments from interference that may be caused by BSS networks located outside an arc of  9 around a wanted BSS network, under worst-case station-keeping conditions. Therefore, this power flux-density limit was intended to be considered as a hard-limit that shall not be exceeded.      (MOD RRB12/60)

c)	Implementation of the power flux-density masks and equivalent protection margin criterion referred to in sub-paragraphs a) and b) of Section 1 of Annex 1 to Appendix 30

1	In accordance with sub-paragraphs a) and b) of Section 1 of Annex 1 to Appendix 30, an administration, which has assignment(s) in the Plan, in the List or assignment(s) for which the procedure of Article 4 of Appendix 30 has already been initiated, is considered as affected by a proposed new or modified assignment in the List if all the following conditions are met:

–	the orbital spacing between both assignments is less than 9°, under worst-case station-keeping conditions; and

–	there is a frequency overlap between the bandwidths assigned to each assignment; and

–	under assumed free-space propagation conditions, the power flux-density value derived from the appropriate power flux-density mask given in § a) of Section 1 of Annex 1 to Appendix 30 is exceeded at least at one of the test-points10 of the wanted assignment; and

–	the reference equivalent protection margin of at least one of the test-points10 of that wanted assignment falls more than 0.45 dB below 0 dB, or if already negative, more than 0.45 dB below that reference equivalent protection margin value.

d)	Reference protection margin11

1	The reference equivalent protection margin values of:

–	the assignments in the downlink or feeder-link Plans;

–	the assignments in the downlink or feeder-link Lists;

–	the assignments for which the procedure of Article 4 of Appendices 30 or 30A has been initiated,

include the potential interference effects of the other assignments of the corresponding Plan and List, as established at WRC-2000, and those of the other assignments entered in the corresponding List after a successful application of the Article 4 procedure.

2	The reference equivalent protection margin used as the basis for comparing the effect of a proposed new or modified assignment is that periodically published by the Bureau and updated once a new or modified assignment is entered in the corresponding List after a successful application of the Article 4 procedure.

An. 4

## Need for coordination of a transmitting space station in the fixed-satellite service
or in the broadcasting-satellite service where this service is not subject to
a Plan: in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or
proposed new or modified assignments in the List for Regions 1 and 3;
in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with
respect to the Plan or proposed modifications to the Plan
in Region 2; in Region 3 (12.2-12.5 GHz) with respect to
the Plan, List or proposed new or modified assignments
in the List for Region 1

(See Article 7)

Clarification on the implementation of Annex 4 to Appendix 30

1	The examination will consider only those administrations having assignments to BSS space stations subject to a Plan whose necessary12 bandwidth overlaps the necessary12 bandwidth of the proposed FSS (or BSS not subject to a Plan) assignment.

2	In the absence of available service area contour of the BSS assignment, the methodology described in Annex 4 to Appendix 30 will be applied but instead of verifying the power flux-density compliance over any portion of the service area, it will be verified at each of the BSS test-points associated with the service area of the corresponding BSS assignment.

An. 5

## Technical data used in establishing the provisions and associated Plans
and the Regions 1 and 3 List, which should be used
for their application

3.5.1
and 3.8		(MOD RRB16/58)

These sections govern the channel spacing between the assigned frequencies of two adjacent channels and the necessary bandwidth values for systems in the Plans for Regions 1, 2 and 3. They also state that “if different bandwidths and/or channel spacing are submitted, they will be treated in accordance with applicable ITU-R Recommendations for protection masks when available. In the absence of such Recommendations, the Bureau will use the worst-case approach.”

Noting that Recommendation ITU-R BO.1293-2 provides a method for calculation of interference only between assignments using different channelling and bandwidth in the case of a digital interferer, the Board therefore decided that, as an interim measure, until the applicable ITU-R Recommendations for protection masks/calculation method are available the calculation methods shown in the Table 1 shall be applied when calculating interference between two assignments in the Plans and/or modifications to Plans.

TABLE  1      (MOD RRB16/58)

| Wanted assignment | Interfering assignment | Method to be applied |
| --- | --- | --- |
| “Standard”1 analogue | “Standard” analogue | As defined in Annex 5 to Appendix 30 |
| “Non-standard” analogue | “Standard” analogue | As described in the Bureau’s MSPACE Manual |
| “Standard” analogue | “Non-standard” analogue | As described in the Bureau’s MSPACE Manual |
| “Non-standard” analogue | “Non-standard” analogue | As described in the Bureau’s MSPACE Manual |
| Digital | “Standard” or “non-standard” analogue | As described in the Bureau’s MSPACE Manual |
| “Standard” or “non-standard” analogue | Digital | As defined in Recommendation ITU-R BO.1293-2 2 |
| Digital | Digital | As defined in Recommendation ITU-R BO.1293-2 2 |
| 1 The standard analogue assignments mentioned in the Table 1 above are those assignments in the Region 2 Plan with 24 MHz bandwidth, 14.58 MHz channel spacing and the assigned frequencies as specified in Article 10 of Appendix 30. 2 Recommendation ITU-R BO.1293-2 (Annexes 1 and 2) is applied which is referred to in § 3.4 of Annex 5 to Appendix 30 and § 3.3 of Annex 3 to Appendix 30A. |  |  |  |

3.11

Section 3.11 of Annex 5 to Appendix 30 describes the space station keeping accuracy under which the space stations operating in the broadcasting-satellite service must be maintained.

In the absence of applicable ITU-R Recommendations describing how these limitations should be implemented in the compatibility analyses performed by the Bureau, the Radio Regulations Board (RRB) decided that the Bureau should develop the appropriate methodology for the application of this section.

ATTACHMENT  1

Clusters for Region 2

| Column No. | Designation |
| --- | --- |
| 1 | Cluster (degrees) |
| 2 | Number of beams in the cluster |
| 3 | Administration names and orbital position |

CLUSTERS  FOR  REGION  2

| 1 | 2 | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| –175.00 | 8 | ALS00003 –175.2 | HWA00003 –175.2 | HWA01003 –175.2 | USAPSA03 –175.2 | ALS00003 –174.8 | HWA00003 –174.8 | USAPSA03 –174.8 | HWA01003 –174.8 |  |
| –166.00 | 8 | ALS00002 –166.2 | HWA00002 –166.2 | HWA01002 –166.2 | USAPSA02 –166.2 | ALS00002 –165.8 | HWA00002 –165.8 | USAPSA02 –165.8 | HWA01002 –165.8 |  |
| –157.00 | 2 | USAWH102 –157.2 | USAWH102 –156.8 |  |  |  |  |  |  |  |
| –148.00 | 2 | USAWH101 –148.2 | USAWH101 –147.8 |  |  |  |  |  |  |  |
| –138.00 | 8 | CAN01101 –138.2 | CAN01201 –138.2 | CAN02101 –138.2 | CAN02201 –138.2 | CAN01101 –137.8 | CAN01201 –137.8 | CAN02101 –137.8 | CAN02201 –137.8 |  |
| –136.00 | 2 | MEX02NTE –136.2 | MEX02NTE –135.8 |  |  |  |  |  |  |  |
| –131.00 | 1 | CTR00201 –130.8 |  |  |  |  |  |  |  |  |
| –129.00 | 12 | CAN01203 –129.2 CAN01403 –128.8 | CAN01303 –129.2 CAN02203 –128.8 | CAN01403 –129.2 CAN02303 –128.8 | CAN02203 –129.2 CAN02403 –128.8 | CAN02303 –129.2 | CAN02403 –129.2 | CAN01203 –128.8 | CAN01303 –128.8 |  |
| –127.00 | 2 | MEX02SUR –127.2 | MEX02SUR –126.8 |  |  |  |  |  |  |  |
| –121.00 | 1 | PNRIFRB2 –121.0 |  |  |  |  |  |  |  |  |
| –119.00 | 2 | USAEH004 –119.2 | USAEH004 –118.8 |  |  |  |  |  |  |  |
| –116.00 | 3 | BLZ00001 –115.8 | CYM00001 –115.8 | TCA00001 –115.8 |  |  |  |  |  |  |
| –115.00 | 6 | BOLAND01 –115.2 | CLMAND01 –115.2 | EQACAND1 –115.2 | EQAGAND1 –115.2 | PRUAND02 –115.2 | VENAND03 –115.2 |  |  |  |
| –110.00 | 4 | PTRVIR02 –110.02 | USAEH003 –110.2 | PTRVIR02 –109.8 | USAEH003 –109.8 |  |  |  |  |  |
| –107.50 | 4 | GTMIFRB2 –107.3 | HNDIFRB2 –107.3 | NCG00003 –107.3 | SLVIFRB2 –107.3 |  |  |  |  |  |
| –106.00 | 5 | CHLCONT5 –106.2 | CHLPAC02 –106.2 | PAQPAC01 –106.2 | CHLCONT4 –105.8 | CHLCONT6 –105.8 |  |  |  |  |
| –104.00 | 2 | VEN02VEN –103.8 | VEN11VEN –103.8 |  |  |  |  |  |  |  |

CLUSTERS  FOR  REGION  2 (continued)

| 1 | 2 | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| –103.00 | 1 | CLM00001 –103.2 |  |  |  |  |  |  |  |  |
| –102.00 | 1 | B SE911 –101.8 |  |  |  |  |  |  |  |  |
| –101.00 | 4 | PTRVIR01 –101.2 | USAEH002 –101.2 | PTRVIR01 –100.8 | USAEH002 –100.8 |  |  |  |  |  |
| –99.00 | 1 | PRG00002 –99.2 |  |  |  |  |  |  |  |  |
| –96.00 | 1 | BERBERMU –96.2 |  |  |  |  |  |  |  |  |
| –95.00 | 2 | EQAC0001 –94.8 | EQAG0001 –94.8 |  |  |  |  |  |  |  |
| –94.00 | 3 | ARGINSU4 –94.2 | ARGSUR04 –94.2 | ARGNORT4 –93.8 |  |  |  |  |  |  |
| –92.50 | 7 | BRB00001 –92.7 | JMC00002 –92.7 | CRBBAH01 –92.3 | CRBBER01 –92.3 | CRBBLZ01 –92.3 | CRBEC001 –92.3 | CRBJMC01 –92.3 |  |  |
| –91.00 | 12 | CAN01304 –91.2 CAN01504 –90.8 | CAN01404 –91.2 CAN02304 –90.8 | CAN01504 –91.2 CAN02404 –90.8 | CAN02304 –91.2 CAN02504 –90.8 | CAN02404 –91.2 | CAN02504 –91.2 | CAN01304 –90.8 | CAN01404 –90.8 |  |
| –89.00 | 1 | CUB00001 –89.2 |  |  |  |  |  |  |  |  |
| –87.00 | 2 | BAHIFRB1 –87.2 | BOL00001 –87.2 |  |  |  |  |  |  |  |
| –86.00 | 1 | PRU00004 –85.8 |  |  |  |  |  |  |  |  |
| –84.50 | 3 | GUY00201 –84.7 | SURINAM2 –84.7 | TRD00001 –84.7 |  |  |  |  |  |  |
| –83.50 | 2 | DOMIFRB2 –83.3 | HTI00002 –83.3 |  |  |  |  |  |  |  |
| –82.00 | 12 | CAN01405 –82.2 CAN01605 –81.8 | CAN01505 –82.2 CAN02405 –81.8 | CAN01605 –82.2 CAN02505 –81.8 | CAN02405 –82.2 CAN02605 –81.8 | CAN02505 –82.2 | CAN02605 –82.2 | CAN01405 –81.8 | CAN01505 –81.8 |  |
| –81.00 | 4 | B SU111 –81.2 | B SU211 –81.2 | B SU111 –80.8 | B SU211 –80.8 |  |  |  |  |  |

CLUSTERS  FOR  REGION  2 (end)

| 1 | 2 | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| –79.50 | 8 | ATGSJN01 –79.7 | MSR00001 –79.7 | SCN00001 –79.7 | VRG00001 –79.7 | DMAIFRB1 –79.3 | GRD00003 –79.3 | LCAIFRB1 –79.3 | VCT00001 –79.3 |
| –78.00 | 2 | MEX01NTE –78.2 | MEX01NTE –77.8 |  |  |  |  |  |  |
| –74.00 | 6 | B N0611 –74.2 | B N0711 –74.2 | B N0811 –74.2 | B N0611 –73.8 | B N0711 –73.8 | B N0811 –73.8 |  |  |
| –72.50 | 4 | CAN01202 –72.7 | CAN02202 –72.7 | CAN01202 –72.3 | CAN02202 –72.3 |  |  |  |  |
| –71.50 | 1 | URG00001 –71.7 |  |  |  |  |  |  |  |
| –70.50 | 4 | CAN01606 –70.7 | CAN02606 –70.7 | CAN01606 –70.3 | CAN02606 –70.3 |  |  |  |  |
| –69.00 | 1 | MEX01SUR –69.2 |  |  |  |  |  |  |  |
| –64.00 | 6 | B CE311 –64.2 | B CE411 – 64.2 | B CE511 –64.2 | B CE311 –63.8 | B CE411 –63.8 | B CE511 –63.8 |  |  |
| –61.50 | 2 | USAEH001 –61.7 | USAEH001 –61.3 |  |  |  |  |  |  |
| –57.00 | 2 | FLKANT01 –57.2 | GRD00059 –57.2 |  |  |  |  |  |  |
| –55.00 | 3 | ARGINSU5 –55.2 | ARGSUR05 –55.2 | ARGNORT5 –54.8 |  |  |  |  |  |
| –53.00 | 4 | GRLDNK01 –53.2 | SPMFRAN3 –53.2 | ATNBEAM1 –52.8 | GUFMGG02 –52.8 |  |  |  |  |
| –45.00 | 8 | B CE312 –45.2 | B CE412 –45.2 | B SU112 –45.2 | B SU212 –45.2 | B CE312 –44.8 | B CE412 –44.8 | B SU112 –44.8 | B SU212 –44.8 |
| –42.00 | 1 | GRD00002 –42.2 |  |  |  |  |  |  |  |
| –34.00 | 2 | GUY00302 –33.8 | JMC00005 –33.8 |  |  |  |  |  |  |
| –31.00 | 2 | BERBER02 –31.0 | FLKFALKS –31.0 |  |  |  |  |  |  |

_______________

* 	Note by the Secretariat: 	This Resolution was revised by WRC-12.

1 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

2 	Assignments to satellite networks of international organizations should not be considered as being national assignments of administrations which notify them on behalf of international satellite organizations.

3 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

4 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

* 	Note by the Secretariat: 	This Resolution was revised by WRC-12.

5 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

6 	Assignments to satellite networks of international organizations should not be considered as being national assignments of administrations which notify them on behalf of international organizations.

7 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

8 	Anytime the “Plan” is referred to, this means the current version of the Plan as updated on the date of the Bureau’s examination in the case of the Region 2 Plan, and, in the case of the Regions 1 and 3 Plan, the current version of the Plan as may be updated pursuant to the possible application of § 4.1.26 or 4.1.27 of Appendix 30.

* 	Note by the Secretariat: 	This Resolution was suppressed by WRC-03.

9 	As appearing in the appropriate Plan or List, according to the case.

* 	Note by the Secretariat: 	This Resolution was revised by WRC-12.

10 	In the case of a wanted assignment in the Plan, the test-points referred to in this paragraph are those defined in that Plan. In the case of a wanted assignment in the List or for which the procedure of Article 4 of Appendices 30/30A has already been initiated, the test-points referred to in this paragraph are those provided under former Annex 2 to Appendices 30/30A or under Appendix 4.

11 	An analysis carried out by the Bureau has shown that the sensitivity to interference, in terms of being identified as affected, by networks received by the Bureau under Article 4 of Appendices 30 and 30A, caused by subsequent proposed modifications to the Plan, decreases when those networks have a very low equivalent protection margin. In those cases where, because of the above phenomenon they are not identified as affected (the equivalent protection margin reduces by at least 0.45 dB) it is up to the administrations concerned to take necessary action, as appropriate.

12 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).