# PART  B

## SECTION  B3

# Rules concerning methodology for calculation of probability
of harmful interference between satellite networks (C/I ratios)

# 1	Introduction

In application of the provisions of No. 11.32A when, as a consequence of continuing disagreement (Nos. 9.63 to 9.65) between two (or a limited number of) administrations, the notifying administration requests the Radiocommunication Bureau, an examination of the probability of harmful interference under No. 11.32A is carried out. For the calculation method and criteria to be used for the interference assessment as well as the findings to be formulated with respect to coordination of their networks under No. 9.7, the Bureau shall proceed as follows.

# 2	Probability of harmful interference

The Bureau, in performing its mandatory tasks relating to the application of the above-mentioned provisions, shall proceed as follows:

2.1	Recommendation ITU-R S.741-2, shall be used to examine the subject assignments with respect to the provisions of No. 11.32A.

2.2	The Bureau shall use the mutually agreed criteria provided by the administrations concerned for accepted interference in the format appearing in Table 2 of Recommendation  ITU-R S.741-2, or, in the absence of such information, the Bureau shall use the single entry limits defined in Table 2 of § 3.2 below, which is derived from Table 2 of Recommendation ITU-R S.741-2, together with the information submitted in accordance with Appendix 4.

2.2.1	In the case where this information is provided by the administrations concerned:

a)	The probability of harmful interference is considered to be negligible if the C/I calculation shows that the applicable criteria for a particular examination between two networks concerned are satisfied. The finding in Column 13A3 shall thus be favourable.

b)	The probability of harmful interference is considered not to be negligible, if the C/I  calculation shows that the applicable criteria for a particular examination between two  networks concerned are not satisfied. The finding in Column 13A3 shall be unfavourable.

2.2.2	In the case where this information is not provided by the administrations concerned:

a)	The probability of harmful interference is considered to be negligible if the interference is less than or equal to the single-entry interference limits indicated in Table 2 of § 3.2 below. The finding in Column 13A3 shall be favourable.

b)	The probability of harmful interference is considered not to be negligible, if the interference is greater than the single-entry interference limits indicated in Table 2 of § 3.2 below. The finding in Column 13A3 shall be unfavourable.

# 3	Methodology

To perform the above-mentioned compatibility analysis the following methodology will be used.

The methodology is based on Recommendation ITU-R S.741-2. A set of carrier-to-interference (C/I ) calculations, using power values submitted by notifying administrations in items C.8.a.1/C.8.b.1 (i.e. the maximum value of the peak envelope power/the total peak envelope power) of Appendix 4 for both wanted and interference carrier levels, are performed following the geometrical considerations of Recommendation ITU-R S.740 and an interference adjustment factor is calculated as shown below to take into consideration the frequency offset situations as well as the difference in the bandwidths between the wanted and the interfering carriers. These C/I values are then compared with the required C/I values derived from the criteria appearing in Table 2 of § 3.2 below which contains a set of single entry interference criteria to protect different types of carriers. In the case of required C/I values agreed by administrations and communicated to the Bureau, the calculated C/I values will be compared with these mutually agreed C/I values.

Thereafter, a set of margins M (C/I calculated – C/I required) are derived. It should be noted that to evaluate the C/I required, a set of carrier-to-noise ratio (C/N ) objectives are used (performance) and a K value, generally of either 12.2 or 14.0 dB, is added in accordance with the above-mentioned Table 2 of § 3.2 below. It should also be noted that these values correspond to a maximum permissible interference of 6% or 4% of the total noise power N of the protected assignments (performance).

In order to identify C/I required to be used for calculations, two scenarios are analyzed:

I.	The assessment of interference caused by incumbent networks into the network submitted for the examination under No. 11.32A:

In this case, to calculate the required C/I of the examined network, the C/N objective of the network (see item C.8.e.1 of Annex 2 of Appendix 4) submitted by the notifying administration for examination under No. 11.32A is used.

II.	The assessment of interference caused by the network submitted for examination under No. 11.32A into incumbent networks:

In this case, to calculate the required C/I of each of the incumbent networks, the lower value between the submitted C/N objective (see item C.8.e.1 of Annex 2 of Appendix 4) and the calculated C/N (using power values submitted by the notifying administration in items C.8.a.1/C.8.b.1 of Appendix 4) of the incumbent network is used.

If no C/N objectives are submitted by notifying administrations (since this was not required in the past), the calculated C/N values are used.

In respect of C/N ratio calculations used to define single entry protection criteria (C/I required), Table 2 of Recommendation ITU-R S.741-2 (see below) defines “C/Ntot ” as a “ratio (dB) of carrier to total noise power which includes all internal system noise and interference from other systems”. Therefore, and to comply with this definition, an additional margin of 0.46 dB for cases involving wanted analogue TV emissions and 1.87 dB for other wanted emissions should be added to the margins calculated on the basis of the internal system noise values provided by the concerned administrations unless the submitted C/N objective already includes a margin to account for inter-system interference. Attachment 2 contains the calculation methodology used for deriving the above-mentioned additional margin.

For the identification of the required C/I with respect to networks received on or after 1 January 2005, whenever the submitted C/N objective is used, no additional margins should be added to the value submitted/provided since, following a revision of Appendix 4 by 
WRC-03, the C/N objective submitted after this date should already include a margin to account for inter-system interference. On the other hand, whenever the calculated C/N is used to identify the required C/I, as it may be the case according to Scenario II above, the relevant additional margin should be added to the value of the calculated C/N.

## 3.1	Interfering cases

Table 1 below presents a summary of the different interfering situations to be dealt with when performing C/I calculations.

TABLE  1

Interference cases

| Desired Interfering |  | Digital | Analogue (TV-FM) | Analogue (other than TV-FM) | Other |
| --- | --- | --- | --- | --- | --- |
| Digital |  | Use C/I plus interference adjustment factor1 (I) | Use C/I plus interference adjustment factor1 (II) | Use C/I plus interference adjustment factor1 (III) | Use C/I plus interference adjustment factor1 (XI) |
| Analogue (TV-FM) |  | Use C/I plus interference adjustment factor2 | Co-frequency: use C/I plus interference adjustment factor1 (X) Non co-frequency: use relative protection ratio mask3 | Use C/I plus interference adjustment factor2 | Use C/I plus interference adjustment factor2 |
|  |  | (IV) | (V) | (VI) | (XII) |
| Analogue (other than TV-FM) |  | Use C/I plus interference adjustment factor2 (VII) | Use C/I plus interference adjustment factor2 (VIII) | Use C/I plus interference adjustment factor2 (IX) | Use C/I plus interference adjustment factor2 (XIII) |
| Other |  | Use C/I plus interference adjustment factor2 (XIV) | Use C/I plus interference adjustment factor2 (XV) | Use C/I plus interference adjustment factor2 (XVI) | Use C/I plus interference adjustment factor2 (XVII) |
| 1 Interference adjustment factor for Cases I, II, III, X and XI is the same (see § 2.1.1 of Attachment 1). 2 Interference adjustment factor for Cases IV, VI to IX and XII to XVII is the same (see § 3.5 below). 3 See § 3.1 of Attachment 1. |  |  |  |  |  |

The selection of an interference case defined in Table 1 above requires the identification of the type of each carrier. Taking into account the information submitted to the Bureau by administrations in accordance with Appendix 4 (i.e. the class of emission as defined in Annex 2 item C.7.a), the Bureau shall use the following carrier type definition:

–	Analogue (TV-FM):

When the Class of Emission (item C.7.a of Annex 2 to Appendix 4) is defined with “F” for the first character and with “F” or “W” for the third character.

–	Analogue (other than TV-FM):

When the first character of the Class of Emission is “F” and the third character is anything other than “F” or “W”.

–	Digital:

When the first character of the Class of Emission is “G”.

–	Other:

When the first character of the Class of Emission is anything other than “F” or “G”.

## 3.2	Margin M, C/I, C/N algorithms

The algorithms described in Attachment 1 shall be used to evaluate compliance with the mutually accepted interference criteria or with the single entry limits established in Table 2.

Table 2 provided below takes into account the information submitted to the Bureau by administrations in accordance with Appendix 4 and the carrier type definition in § 3.1 above and is a simplification of Table 2 of Recommendation ITU-R S.741-2.

TABLE 2

Single entry interference (SEI) protection criteria

| Interfering carrier type Desired carrier type | Analogue (TV-FM) or other | Digital | Analogue (other than TV-FM) |
| --- | --- | --- | --- |
| Analogue (TV-FM) | C/Ntot + 14 (dB) |  |  |
| Digital | If DeNeBd InEqBd then C/Ntot + 9.4 + 3.5 log (δ) – 6 log (i/10) (dB) (i.e. C/Ntot + 5.5 + 3.5 log (DeNeBd (MHz))) Otherwise if DeNeBd > InEqBd then C/Ntot + 12.2 (dB) | C/Ntot + 12.2 (dB) |  |
| Analogue (other than TV-FM) | 13.5 + 2 log (δ) – 3 log (i/10) (dB) (i.e. 11.4 + 2 log (DeNeBd (MHz))) | C/Ntot + 12.2 (dB) |  |
| Other | 13.5 + 2 log (δ) – 3 log (i/10) (dB) (i.e. 11.4 + 2 log (DeNeBd (MHz))) | C/Ntot + 14 (dB) |  |
| where: C/Ntot: ratio (dB) of carrier to total noise power which includes all internal system noise and interference from other systems DeNeBd: necessary bandwidth of desired carrier (Appendix 4, Annex 2, item C.7.a) InEqBd: equivalent bandwidth of interfering carrier (equal to total power to power density ratio (see Appendix 4, Annex 2, items C.8.a.1 and C.8.a.2 respectively)) δ: ratio of desired signal bandwidth to peak-to-peak deviation of the TV carrier caused by the energy dispersal signal (a peak-to-peak deviation of 4 MHz is used in all cases) i: pre-demodulation interference power in the desired signal bandwidth expressed as a percentage of the total pre-demodulation noise power (a value of 20 is used in all cases). |  |  |  |

## 3.3	Single channel per carrier (SCPC) cases

When dealing with composite interference from a number of narrow-band carriers such as a transponder loaded with SCPC carriers the assumption is made, in the absence of more detailed data from administrations, that the interfering satellite has its transponder fully loaded with SCPC carriers and the individual carriers can be replaced with one wideband carrier which has a total power equal to the sum of the powers of the individual SCPC carriers. The protection ratios given in Recommendation ITU-R S.671 are used to protect SCPC transmissions interfered with by analogue television carriers only modulated with energy dispersal signals.

## 3.4	Interference between analogue FDM-FM signals (Case (IX) in Table 1 above)

When dealing with FDM-FM carriers, and to find out the resulting margin, the C/I ratio is calculated and compared with the required C/I. However a C/N  K type protection criteria is developed based on the equations of Recommendation ITU-R SF.766 which are required to calculate the B factor (interference reduction factor). In the absence of detailed information for the calculation of the B factor, the interference adjustment factor described in § 3.5 below shall be used.

## 3.5	Other interference cases

For cases (IV), (VI), (VII), (VIII), IX and (XI) to (XVII) in Table 1 above, the interference adjustment factor mentioned in § 3 above shall be used. In calculating this factor consideration shall be given to the third paragraph of § 3.4 of Annex 1 to Recommendation ITU-R S.741-2.

# ATTACHMENT  1

# Calculation algorithms (M, C/I, C/N)

# 1	Margin algorithm

To compute the margins, it is necessary first to determine the required value, which is a function of the C/N and the K factor:

where:

|  | required C/I value (dB) |
| --- | --- |
|  | ratio (dB) of carrier to total noise power which includes all internal system noise and interference from other systems |
| K : | factor used in computing the required C/I (dB). Generally, this will be either 14.0 or 12.2, depending on the modulation characteristics of the desired signals (see Recommendations ITU-R S.483 and ITU-R S.523). |

The total carrier-to-noise ratio is defined, as follows:

- For receiving frequency assignments of a network received before 1 January 2005:

- Scenario I (as defined in Section 3):

- Scenario II:

- For receiving frequency assignments of a network received on and after 1 January 2005:

- Scenario I:

- Scenario II:

where:

| X : | Additional margin (see Attachment 2, Sections 3 to 5) to comply with the definition of carrier to total noise power, which includes all internal system noise and interference from other systems. Attachment 2 contains the methodology used for deriving the additional margin. |
| --- | --- |
| C/Ni: | Calculated value of carrier-to-noise ratio, based on internal system noise power, defined in Section 3 below. |
| (C/N)obj: | C/N objective of the network (see item C.8.e.1 of Annex 2 of Appendix 4) submitted by the notifying administration for examination under No. 11.32A. |

Since  and  will vary depending on the geographical location within the service area, both values are computed:

–	At the geographical locations of the associated specific earth stations, if any, or,

–	In case of associated typical earth stations, at the test point located within the service area where the  value is minimum in accordance with the method given in Attachment 3.

The margin is the difference between the calculated C/I value and the required C/I value:

M  =

where:

| M : | margin (dB) |
| --- | --- |
|  | adjusted value of C/I, taking into account the interference adjustment factor (dB) |
|  | required C/I value (dB) computed above. |

Therefore, substituting:

M  =   –  K

# 2	The  algorithm for interfering situations

The basic C/I is adjusted as follows:

where:

|  | adjusted value of C/I, taking into account the interference adjustment factor (dB) |
| --- | --- |
|  | basic calculated value of C/I, before taking into account the interference adjustment factor (dB) |
| Ia : | interference adjustment factor (dB). |

## 2.1	Determination of interference adjustment factor

### 2.1.1	Interference from noise-like digital carriers (interference adjustment factor 1)

The current version of Recommendation ITU-R S.741-2 covers the case of co-frequency interference from noise-like digital carriers. For non-co-frequency interference, an interference adjustment factor (or bandwidth advantage factor) resulted from the work of ITU-R study groups concerning the methodology to treat cases of frequency offset carriers. This is reflected in the application of a factor A defined below (mentioned as Ia in § 2 above).

For the case of frequency offset between carriers, the resultant C/I can be determined by the following equation:

C/I  10 log (c/i ) – A

where A  is the bandwidth advantage factor (dB).

The factor A is the ratio of the interfering carrier power contained in the desired signal bandwidth to the total interfering carrier power under the assumption that the interfering carrier has uniform power spectral density across its occupied bandwidth.

### 2.1.2	Interference from noise-like analogue carriers (interference adjustment factor 2)

For these cases, the resultant C/I can be determined by using the equation in § 2.1.1 above where the factor A is the ratio of the interfering carrier power contained in the desired signal bandwidth to the interfering carrier power with the approximation that the power spectral density of the interfering carrier is constant over the bandwidth of the desired carrier and is equal to the maximum value (see the third paragraph of § 3.4 of Annex 1 to Recommendation ITU-R S.741-2).

# 3	The C/N algorithm

The algorithm for C/N requires the computation of the value of N, as follows:

where:

Ni   :	value of internal system noise (dBW)

TR :	receiving system noise temperature (K)

BW :	bandwidth (MHz).

The value of Ni is determined once for the uplink (if there is an uplink) and once for the downlink (if there is a downlink) for the desired system.

Once Ni is determined, C/Ni will be computed at each uplink test point (if there is an uplink) and each downlink test point (if there is a downlink):

where:

C :	carrier (dBW)

Ni  :	internal system noise (dBW) computed above.

## 3.1	Determination of relative protection ratio for Case (V) in Table 1 above (TV-FM) into (TV-FM)

When dealing with a non-co-frequency interfering situation from a TV-FM carrier into another TV-FM carrier, the Radiocommunication Bureau is using the protection ratio masks defined in the Rules of Procedure relating to § 3.5.1 and § 3.8 of Annex 5 to Appendix 30 for the same case of interference. The resulting protection ratio relaxation is applied to the K factor of 14.0 dB established by Recommendation ITU-R S.483.

# ATTACHMENT  2

# Additional margins to be taken into consideration

# 1	Introduction

To finally assess the interfering effect on a given emission, it is necessary to adjust the resulting margins taking into consideration the definition of C/N given by Recommendation  ITU-R S.741-2 which, for most of the cases, is the performance reference necessary to derive the single entry interference criteria levels for FSS carriers (see Table 2 of Recommendation ITU-R S.741-2).

In the above-mentioned Table C/N is defined as: “ratio (dB) of carrier to total noise power which includes all internal system noise and interference from other systems”.

# 2	Calculations performed according to No. 1.174

No. 1.174 defines the equivalent satellite link noise temperature as follows:

“The noise temperature referred to the output of the receiving antenna of the earth station corresponding to the radio frequency noise power which produces the total observed noise at the output of the satellite link excluding the noise due to interference coming from satellite links using other satellites and from terrestrial systems.”

The internal system noise temperature values provided by the administrations to derive the internal system noise, N, i.e., Ts and Te are defined in Appendix 8 as follows:

“Ts :	the receiving system noise temperature of the space station, referred to the output of the receiving antenna of the space station (K)”

“Te :	the receiving system noise temperature of the earth station, referred to the output of the receiving antenna of the earth station (K).”

The above-mentioned values are combined in accordance with Recommendation ITU-R S.738 to derive Tmin, lowest equivalent satellite link noise temperature, as follows:

Tmin  Te  min Ts  Ta

where:

Ta :	other internal noise

min :	minimum transmission gain of a specific satellite link subject to interference.

Calculation of equivalent satellite link was mandatory before WRC-2000. After the decisions of WRC-2000 RR, Appendix 4 strapping information in Appendix 4, which is required to conduct overall link calculations became optional.

Therefore, and for simplicity Ts and Te are used separately to conduct uplink and downlink C/I calculations respectively for all the cases.

# 3	Noise to be calculated in accordance with Recommendation ITU-R S.741-2

To be in accordance with Recommendation ITU-R S.741-2 it seems necessary to add to the values of N calculated by the program on the basis of Te and Ts mentioned above, the maximum permissible level of aggregate interference caused by other satellite systems as appears in Recommendations ITU-R S.466 (for FDM-FM telephony), ITU-R S.483 (for TV analogue) and ITU-R S.523 (for digital emissions) as well as the contribution of terrestrial emissions sharing the same frequency bands as defined in Recommendations ITU-R SF.356-4* (into telephone channels employing frequency modulation), and ITU-R SF.558-2** (into systems employing 8-bit PCM encoded telephony).

# 4	Calculations of additional margins

## 4.1	Telephony FDM-FM

### 4.1.1	Aggregate interference produced by other satellite systems sharing the same frequency band (Recommendation ITU-R S.466)

In accordance with Recommendation ITU-R S.466, in frequency bands in which the network does not practice frequency re-use: the aggregate interference noise power should not exceed 2 500 pW0p, psophometrically weighted one minute mean power for more than 20% of any month. This amount corresponds to the 25% of the allowable noise power of 10 000 pW0p established by Recommendation ITU-R S.353-8*** for the same percentage of time.

### 4.1.2	Maximum allowable values of aggregate interference from radio-relay systems in a telephone channel of a system in the FSS (Recommendation ITU-R SF.356-4*)

In accordance with this Recommendation the interference caused by the aggregate of the transmitters of radio-relay stations should not exceed 1 000 pW0p psophometrically weighted one minute mean power for more than 20% of any month. This amount corresponds to 10% of the allowable noise power of 10 000 pW0p established by Recommendation ITU-R S.353-8*** for the same percentage of time.

### 4.1.3	Calculation of the additional margin

Ntot :	total link noise including all internal noise and interference from other systems

Ni :	link internal noise

X :	noise due to interference from other systems

then:

Ntot    Ni    X

where:

X    (0.25    0.1) Ntot

Therefore:

Ntot    Ni    0.35 Ntot

Ntot (1  –  0.35)    Ni

Ntot    1.53 Ni

Additional margin: 10 * log(1.53)    1.87 dB.

## 4.2	Digital emissions

### 4.2.1	Aggregate interference produced by other satellite systems sharing the same frequency band (Recommendation ITU-R S.523)

In accordance with Recommendation ITU-R S.523, in frequency bands in which the network does not practice frequency re-use: the aggregate interference power level averaged over any 10 min, should not exceed, for more than 20% of any month, 25% of the total noise power level at the input to the demodulator that would give rise to a bit error ratio of 1 in 106 as it is established by Recommendation ITU-R S.522 for the same percentage of time.

### 4.2.2	Maximum allowable values of aggregate interference from radio-relay systems into systems in the FSS. employing 8-bit PCM encoded telephony (Recommendation ITU-R SF.558-2*)

In accordance with this Recommendation the interference caused by the aggregate of the transmitters of radio-relay stations, averaged over any 10 min, should not exceed, for more than 20% of any month, 10% of the total noise power at the input of the demodulator that  would give rise to a bit error ratio of 1 in 106 as it is established by Recommendation ITU-R S.522 for the same percentage of time.

### 4.2.3	Calculation of the additional margin

The same values as in § 4.1.3 above are obtained.

## 4.3	Analogue TV

### 4.3.1	Aggregate interference produced by other satellite systems sharing the same frequency band (Recommendation ITU-R S.483)

In accordance with Recommendation ITU-R S.483, the aggregate interference noise power should not exceed 10% of the permissible video noise in the hypothetical reference circuit for more than 1% of the month.

### 4.3.2	Maximum allowable values of aggregate interference from radio-relay systems into FSS analogue video channel

No recommendations have been arrived at yet for interference from transmitters of the fixed service into FSS analogue video channel.

### 4.3.3	Calculation of the additional margin

Ntot    Ni    0.1 Ntot

Ntot (1  –  0.1)    Ni

Ntot    1.11 Ni

Additional margin: 10 * log(1.11)    0.46 dB.

5	Based on the above a value of 0.46 dB should be added to the margins involving wanted analogue TV emissions and 1.87 dB for other wanted emissions.

# ATTACHMENT 3

# Finding test-points for C/I calculation

# 1	Introduction

The assessment of probability of harmful interference shall be based on:

–	one test-point in downlink within the service area of wanted satellite where the  value is minimum;

–	two test-points in uplink wanted and interfering links producing minimum value 
of .

has minimum value when wanted signal is at minimum and interference signal is at maximum.

# 2	Test-point for the downlink C/I calculation

The position of a wanted receiving earth station for which C/I is calculated is selected using the following criteria:

–	Earth station is located within service area of wanted satellite;

–	Earth station is visible from interfering satellite;

–	The difference between satellite gain of wanted satellite and satellite gain of interfering satellite towards wanted earth station is at minimum.

The minimum gain difference is identified following the procedure below:

–	Generating grid-points within wanted satellite service area ;

–	Finding wanted satellite gain towards each of the grid-point ;

–	Finding interfering satellite gain towards each of the grid-point ;

–	Finding the grid-point where the difference between satellite gains towards each satellite is at minimum, i.e. .

The Bureau developed GIMS gain interpolation library to find the satellite gain for all the grid-points.

Figure A3-1 below gives a graphical example of the identified test-point.

# 3	Test-points for the uplink C/I calculation

For the uplink calculation, it is necessary to identify locations of two earth stations – one transmitting earth station in the wanted link and another transmitting station in the interfering link.

The position of these earth stations are selected using the following criteria:

–	Wanted earth station is located within service area of wanted satellite;

–	Interfering earth station is located within service area of interfering satellite;

–	Interfering earth station is visible from wanted satellite;

–	The difference between wanted satellite gain towards wanted earth station and interfering earth station is at minimum.

The minimum gain difference is identified following the procedure below:

–	Generating grid-points within wanted satellite service area ;

–	Generating grid-points within interfering satellite service area ;

–	Finding wanted satellite gain towards each of the grid-point in ;

–	Selecting test-point  where wanted satellite gain is at minimum ;

–	Finding wanted satellite gain  towards each of the grid-point in ;

–	Selecting test-point  where wanted satellite gain is at maximum .

Figure A3-2 below gives graphical example of the procedure used.

FIGURE A3-1

Finding worst-test point on the downlink

FIGURE A3-2

Finding worst-test points on the uplink

____________________

_______________

*	Note by the Secretariat: This Recommendation was suppressed on 20/02/2014 (CACE/667).

** 	Note by the Secretariat: This Recommendation was suppressed on 15/06/2009 (CACE/482).

***	Note by the Secretariat: This Recommendation was suppressed on 29/09/2014 (CACE/692).

* 	Note by the Secretariat: This Recommendation was suppressed on 15/06/2009 (CACE/482).