# PART  B

## SECTION  B6

# Rules concerning criteria for applying the provisions of No. 9.36 
to a frequency assignment in the terrestrial services whose allocation or identification is governed by Nos. 5.292, 5.293, 5.295, 5.296A, 5.297, 5.308, 5.308A, 5.309, 5.323, 5.325, 5.326, 5.341A, 5.341C, 5.346, 5.346A, 5.429D, 5.429F, 5.430A, 5.431A, 5.431B, 5.432B, 5.4341 and 5.553A

1	The identification of the administrations with which coordination may need to be effected is based on the characteristics of the assignment that is subject to the procedure of No. 9.21 and the worst-case assumptions relating to the propagation characteristics and other technical parameters. These worst-case assumptions were developed on the basis of the information contained in various sources (GE06 Regional Agreement, ITU-R Recommendations and Reports), since the Radiocommunication Bureau has no Technical Standards for application in several frequency bands above 28 MHz.

2	For identification of the administrations whose agreement may need to be obtained, in the context of the provisions of Nos. 5.292, 5.293, 5.295, 5.296A, 5.297, 5.308, 5.308A, 5.309, 5.323, 5.325, 5.326, 5.341A, 5.341C, 5.346, 5.346A, 5.429D, 5.429F, 5.430A, 5.431A, 5.431B, 5.432B, 5.434 and 5.553A, the following criteria are applied:

2.1	the coordination distance concept is applied with respect to the services that are allocated according to Article 5 (these services are indicated in the Table below under the heading “Protected service”);

TABLE 1

Applicability of No. 9.21

| Footnote | Frequency band (MHz) | Allocated service (No. 9.21) | Protected service |
| --- | --- | --- | --- |
| 5.292 1 | 470-512 | FS, MS | BS |
| 5.293 1 | 470-512 and 614-806 | FS, MS | BS |
| 5.295 | 470-512 | LMS (IMT) | BS, FS |
|  | 512-608 | LMS (IMT) | BS |
| 5.296A | 470-698 | LMS (IMT) | BS, FS |
|  | 585-610 | LMS (IMT) | RNS |
| 5.297 | 512-608 | FS, MS | BS |
| 5.308 | 614-698 | MS | BS |
| 5.308A | 614-698 | MS (IMT) | BS |
| 5.309 1 | 614-806 | FS | BS, MS |
| 5.323 | 862-960 | ARNS | FS, MS |
| 5.325 1 | 890-942 | RLS | FS, MS |
| 5.326 1 | 903-905 | LMS,MMS | FS |
| 5.341A2 | 1 429-1 452 1 492-1 518 | LMS (IMT) | AMS |
| 5.341C | 1 429-1 452 1 492-1 518 | LMS (IMT) | AMS |
| 5.3462 | 1 452-1 492 | LMS (IMT) | AMS |
| 5.346A | 1 452-1 492 | LMS (IMT) | AMS |
| 5.429D | 3 300-3 400 | LMS (IMT) | RLS |
| 5.429F | 3 300-3 400 | LMS (IMT) | RLS |
| 5.430A | 3 400-3 600 | LMS, MMS | FS, FSS |
| 5.431A and 5.432B | 3 400-3 500 | LMS, MMS | FS, FSS |
| 5.431B | 3 400-3 600 | LMS (IMT) | FS, FSS |
| 5.434 | 3 600-3 700 | LMS (IMT) | FS, FSS |
| 5.553A | 45 500-47 000 | LMS (IMT) | AMS, RNS |
| 1 Different category of service. 2 For frequency assignments subject to this provision the No. 9.21 procedure does not apply to those administrations whose territories are outside of the distances specified in the corresponding Rules of Procedure on No. 5.341A and No. 5.346. |  |  |  |

2.2	The case-by-case verification is performed for the assignments submitted under the procedure of No. 9.21. This verification consists in determining the distance from the location of a station subject to No. 9.21 to the border of a neighbouring country. In case this distance is shorter than the respective coordination distance, the administration of this neighbouring country is identified as affected.

3	In the calculation of the coordination distances the following approach is used:

3.1	For the protection of the broadcasting (television) service in the frequency band 470-806 MHz, from the radio services indicated in Column 3 of Table 1, in the context of the provisions of Nos. 5.292, 5.293, 5.295, 5.296A, 5.297, 5.308, 5.308A and 5.309, the coordination distances are calculated using propagation curves of Recommendation ITU-R P.1546-5 for 1% of time and 50% of locations with the coordination trigger field strengths produced at a height of 10 m above ground level as provided in the GE06 Agreement and given in Table 2.

TABLE 2

Coordination trigger field strengths for protection of BS

| Service to be protected | Trigger field strength (dB(µV/m)) |  |  |
| --- | --- | --- | --- |
|  | 470-582 MHz | 582-718 MHz | 718-806 MHz |
| BS | 18 | 20 | 22 |

3.2	For the protection of the fixed service in the frequency band 470-698 MHz from IMT, in the context of the provision of Nos. 5.295 and 5.296A, the coordination trigger field strength of 13 dB(µV/m) produced at a height of 10 m above ground level is used.

3.3	For the protection of the radionavigation services in the frequency band 
585-610 MHz from IMT, in the context of the provision of No. 5.296A, the coordination distances are calculated using propagation curves of the Recommendation ITU-R P.1546-5 for 10% of time and 50% of locations with the trigger field strength 13 dB(µV/m), as given in the GE06 Agreement, produced at height of 10 m above ground level.

3.4	For the protection of the fixed and mobile services, from the radionavigation and radiolocation services, in the context of the provisions of Nos. 5.323 and 5.325, propagation curves from Recommendation ITU-R P.528-3 are used in connection with the following data:

Minimum field strength to be protected (FX): 30 dB(µV/m), PR  =  8 dB.

3.5	For the protection of the fixed service in the frequency band 903-905 MHz, from the land mobile and maritime mobile services, in the context of the provisions of No. 5.326, the coordination distances are calculated using propagation curves of the Recommendation ITU-R P.1546-5 for 10% of time and 50% of locations with the coordination trigger of 17 dB (µV/m) produced at the height of 10 m above ground level.

3.6	For protection of ground-based stations in the aeronautical mobile service in the frequency band 1 429-1 518 MHz from IMT, in the context of the provisions of Nos. 5.341A, 5.341C, 5.346 and 5.346A, the coordination distances are calculated using the propagation curves given in Recommendation ITU-R P.1546-5 for 10% of time and 50% of locations with the coordination trigger power flux density of -181 dB(W/m2) within 4 kHz of reference bandwidth produced at the height of 10 m above ground level as given in Recommendation ITU-R M.1459-0.

For protection of stations on-board aircraft in the aeronautical mobile service, the coordination distance of 450 km is used.

3.7	For protection of the radiolocation service in the frequency band 
3 300-3 400 MHz from IMT in the context of the provisions of Nos. 5.429D and 5.429F, the coordination distance is contained in Table 3.

TABLE 3

Coordination distance for protection of the RLS
(from the IMT system, effective antenna height 30 m)
in the frequency band between 3 300-3 400 MHz

| Footnote | Frequency range (MHz) | Allocated service (application) (No. 9.21) | Protected service | Coordination distance (km) |
| --- | --- | --- | --- | --- |
| 5.429D 5.429F | 3 300-3 400 | LMS (IMT) | RLS | 616 |
| NOTE − The coordination distance was calculated using the propagation curves of Recommendation ITU-R P.528-3 for 1% of time and 50% of locations with the interference level of –107 dBm for protection of the airborne radar at the height of 10 000 m derived from Recommendation ITU-R M.1465-3. A reference IMT Advanced station was assumed as having radiated power of 31 dBW (e.i.r.p.) and a bandwidth of 10 MHz as used in Report ITU-R M.2292-0. |  |  |  |  |

3.8	For the protection of the fixed and fixed-satellite services in the frequency bands between 3 400 MHz and 3 700 MHz from the mobile, except aeronautical mobile, service in the context of the provisions of Nos. 5.430A, 5.431A and 5.432B, and from IMT in the context of the provisions of Nos. 5.431B and 5.434, the power flux density of 
–154.5 dB(W/m2·4 kHz)2 produced at the height of 3 m above ground level is used.

Based on the above pfd value the coordination distances are calculated using Recommendation ITU-R P.452-16 for 20% of time with smooth Earth terrain profile.

3.9	For the protection of stations of the aeronautical mobile and radionavigation services in the frequency band 45.5-47 GHz from IMT systems in the context of the provision of No. 5.553A, the coordination distance is contained in Table 4.

TABLE 4

Coordination distance for protection of the AMS and RNS
from the IMT systems in the frequency band between 45.5-47 GHz

| Footnote | Frequency range (GHz) | Allocated service (application) (No. 9.21) | Protected service | Coordination distance (km) |
| --- | --- | --- | --- | --- |
| 5.553A | 45.5-47 | LMS (IMT) | AMS, RNS | 65 |

Note: The coordination distance was calculated using a method based on Recommendation ITU-R P.676-12 for atmospheric attenuation in addition to Recommendation ITU-R P.525-4 for free space loss. The protection criteria (I/N) – 6 dB, receiver antenna gain 27 dBi and noise figure 4 dB were taken from Recommendation ITU-R M.2115-0 for the aeronautical mobile service airborne station in the frequency band 45.5-47 GHz. The maximum e.i.r.p. density value of 25.2 dB(W/200 MHz) was used for the IMT base station. This value was taken from ITU-R studies conducted during preparations for WRC-19 agenda item 1.13.

____________________

_______________

1 	See also Rules of Procedure to Nos. 5.312A, 5.316B, 5.341A and 5.346.

2 	This value was decided by WRC-07 based on the protection of a typical earth station in the fixed-satellite service.