# Rules concerning

## ARTICLE  5 of the RR

5.33

Number 5.152 illustrates this provision. When the transmitting and the receiving stations are both located within one of the countries listed in the footnote, the fixed service has equal rights with the amateur service. This is also the case when one station is located in one country and the other in another country, both countries being listed in No. 5.152. When either station is not within one of the countries listed in the footnote, the assignment is out of band.

5.36

The Radio Regulations contain the procedure defined in No. 9.21 together with a number of footnotes of the Table of Frequency Allocations stipulating that an additional or alternative allocation is made “subject to agreement obtained under the procedure set forth in No. 9.21”. The Board had to indicate to the Bureau under which category of allocation an assignment in the service to which the procedure of No. 9.21 had been successfully applied and where the footnote did not indicate the category of allocation, should be recorded. The following conclusions were reached:

a)	When a footnote allocates a frequency band to a service on a secondary basis or on a non-interference basis, this indication is considered by the Board as a restriction imposed on the allocation.

b)	Number 5.37 stipulates that “If restrictions are imposed on an additional allocation ... this is indicated in the footnote of the Table”.

c)	Therefore, when a footnote does not contain such restrictions, the allocation is necessarily on a primary basis.

5.40

The interpretation given under No. 5.36 for additional allocations when the agreement under No. 9.21 is required applies also in this case to alternative allocations.

5.43

This provision specifies the operation on a non-interference and non-protection basis of a service, or station in a service, in respect to another service, or to another station in the same service. However, this provision does not specify the relation between the respective categories of allocations to which the operation on a non-interference and non-protection basis for a service, in respect to another service, applies. Bearing in mind the scope of application and the complexity of allocations contained in various provisions of Article 5, as well as the circumstances under which the allocations were made, the Board considers that the respective status of each allocation which is subject to the condition of not causing harmful interference to, or not claiming protection from, other service or other station in the same service, is to be derived from the conditions specified in each specific provision.

Bearing in mind the various and complex allocation situations that are described in the provisions of Article 5, as well as the circumstances under which the allocations were made, the Board considers that the attention of a future conference should be drawn to the footnotes specifying the operation on a non-interference and non-protection basis, which involve different categories of service, with the view to specifically establish the relationship between the respective categories of allocation to which the operation on a non-interference and non-protection basis refers.

5.43A

See comments under the Rules of Procedure concerning No. 5.43.

5.49

Several provisions, mainly those relating to allocations to mobile services, restrict allocations to a type of operation or to some specific systems. The Board had no means to examine the extent to which these restrictions are respected. (This fact was noted by WARC Mob-87 where such restrictions were made.) Consequently the Board decided that no symbol relating to these types of restrictions should be included in Column 13C of the Master International Frequency Register (MIFR or Master Register).

5.73

1	This provision represents de facto an additional allocation, by providing the possibility for transmission of supplementary navigational information from any station in the maritime radionavigation service, on condition that no harmful interference is caused to radiobeacon stations operating in the radionavigation service.

2	The meaning of narrow-band: based on the information in Recommendation ITU-R M.476-5, the Board considered that 500 Hz represents a reasonable limit for narrow-band techniques and set this value as a regulatory limit to be checked in the examinations of the conformity of the notified bandwidth in the context of this provision. Therefore, the Bureau shall formulate an unfavourable regulatory finding, in the application of No. 5.73, if that limit is exceeded for notified classes of emission F1B or G1D.

5.132A

This provision limits the application of the radiolocation service to oceanographic radars operating in accordance with Resolution 612 (Rev.WRC-12). The resolves part of this Resolution enters into the category “the other provisions” referred to in Nos. 11.31 and is subject to examinations by the Bureau.

Resolves 6 of Resolution 612 (Rev.WRC-12) specifies the separation distances to be respected for oceanographic radars for “rural” and “quiet rural” areas for the land, sea or mixed propagation paths, unless prior explicit agreements from affected administrations are obtained. Concerning “rural” and “quiet rural” areas, the Bureau has no means to identify whether emissions from oceanographic radars reach a “rural” or “quiet rural” area at the border of another country since the Bureau does not have the relevant topographical data to determine these areas.

As the Bureau has no means for the identification of rural or quiet rural areas, the Board decided that for examination of the notified frequency assignment to a station in the radiolocation service from the view point of its conformity with resolves 6 of Resolution 612 (Rev.WRC-12) the Bureau shall use the separation distances for quiet rural paths listed in Columns 3 and 5, as appropriate, of the Table of resolves 6.

5.145A

The comments and decision made under the Rule of Procedure concerning No. 5.132A apply.

5.149

There is no allocation to radio astronomy in the bands 73-74.6 MHz (Regions 1 and 3), 1 330-1 400 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 6 650-6 675.2 MHz, 22.01-22.21 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 36.43-36.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz (except for KOR), 172.31-172.65 GHz (except for KOR), 173.52-173.85 GHz (except for KOR) and 195.75-196.15 GHz. Notification of frequency assignments to radio astronomy stations in the bands 73-74.6 MHz (Regions 1 and 3), 1 330-1 400 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 6 650-6 675.2 MHz, 22.01-22.21 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 36.43-36.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz (except for KOR), 172.31-172.65 GHz (except for KOR), 173.52-173.85 GHz (except for KOR) and 195.75-196.15 GHz will be considered by the Bureau not to be in conformity with the Table of Frequency Allocations.

5.161A

The comments and decision made under the Rule of Procedure concerning No. 5.132A apply.

5.164

Literal interpretation of this provision for an assignment to a land-mobile station in a country listed in the footnote would require recording:

–	a symbol to indicate that the assignment is primary with respect to the countries listed in this footnote,

–	a symbol to indicate that the assignment is secondary with respect to the broadcasting service for other countries,

–	a symbol to indicate that the assignment is primary with respect to fixed and mobile services in countries listed in Nos. 5.165 and 5.171,

–	a symbol to indicate that the assignment is primary with respect to the amateur service in countries listed in No. 5.169,

–	etc.

The Board decided to have such assignments recorded with Symbol R in Column 13B2 and a reference to the footnote concerned in Column 13B1.

5.172

The French overseas departments and communities in Region 2 are the following geographical areas:

Guadeloupe, Saint Barthélemy, the French part of Saint Martin, French Guyana, Martinique and Saint Pierre and Miquelon.

5.233

1	The band mentioned in this provision is allocated in the body of the Table for Region 3 to three services, i.e. fixed, mobile and broadcasting. The Board interpreted this situation as follows:

a)	The successful application of the No. 9.21 procedure to the space services will give them the same status as the fixed and mobile services, i.e. primary.

b)	In respect of the broadcasting service, irrespective of the result of the application of the procedure of the No. 9.21 procedure, the space services can be operated only under No. 5.43.

2	In accordance with the comments made under No. 5.164, when an assignment is primary with respect to one service (or country) and secondary with respect to another service (or country), it will be recorded with Symbol R in Column 13B2 indicating this situation and a reference to the appropriate footnote in Column 13B1.

5.254 and 5.255			(ADD RRB24/510)

No. 5.254 stipulates that: “The bands 235-322 MHz and 335.4-399.9 MHz may be used by the mobile-satellite service, subject to agreement obtained under No. 9.21, on condition that stations in this service do not cause harmful interference to those of other services operating or planned to be operated in accordance with the Table of Frequency Allocations except for the additional allocation made in footnote No. 5.256A.”, whereas No. 5.255 stipulates that: “The bands 312-315 MHz (Earth-to-space) and 387-390 MHz (space-to-Earth) in the mobile-satellite service may also be used by non-geostationary-satellite systems. Such use is subject to coordination under No. 9.11A.”

Recognizing the difficulty in determining the type of coordination applicable to notified frequency assignments in the mobile-satellite service in the above frequency bands, the Board concluded as follows:

1)	When the Bureau examines frequency assignments of non-GSO MSS systems notified in the frequency bands 312-315 MHz (Earth-to-space) and 387-390 MHz (space-to-Earth) only, the Board, noting the MSS allocations on a secondary basis and the fixed and mobile services allocations on a primary basis in these two frequency bands, instructed the Bureau to only apply the provisions of No. 5.255. As a consequence, only the coordination procedure under No. 9.11A applies.

2)	In cases where frequency assignments submitted in the frequency bands 312-315 MHz (Earth-to-space) or 387-390 MHz (space-to-Earth) overlap with other portions of the frequency bands mentioned in No. 5.254 (235-322 MHz and 335.4-399.9 MHz), both coordination under No. 9.11A and agreement-seeking under No. 9.21 apply and the frequency assignments’ status will be recorded in the MIFR with a reference to No. 5.254 in column 13B1 and “R” in column 13B2, in accordance with § 5.5 of the Rules of Procedures on No. 11.31, footnote 1 of Appendix 5 and § 2.3 of the rules of procedures on No. 9.11A.

In such cases, the notifying administration may also consider suitably modifying the assigned frequency band or to split it before its submission so that a non-GSO MSS frequency assignment in the frequency bands 312-315 MHz (Earth-to-space) or 387-390 MHz (space-to-Earth) is subject to No. 5.255 only.

5.257

1	Space telemetry is limited to measurements made in the spacecraft which may be:

–	either made by a sensor to detect phenomena outside the spacecraft; or

–	related to the functioning of the spacecraft.

The first type normally pertains to services such as the earth exploration-satellite service or the space research service, while the second type pertains to the space operation service. This provision does not indicate the service to which the additional allocation is made. The Board understands it as being limited to space telemetry in the space operation service. Consequently, frequency assignments for telemetry (space-to-Earth) in the space operation service in the band 267-272 MHz may be used on a secondary basis without any condition. They may obtain a primary status within the territory of the notifying administration following the successful application of the procedure of No. 9.21.

2	The qualification “in their countries” can be easily checked when an earth station is concerned, but it is unclear for a space station. The Board considers that this provision will apply to those space stations having a service area mainly limited to the territory of the notifying administration.

5.281

With respect to the French overseas departments and communities in Region 2, see comments under the Rules of Procedure concerning No. 5.172.

5.291

This footnote is similar to No. 5.233; the same Rule applies.

5.312A

1	This provision stipulates through Resolution 760 (Rev.WRC-23) that in Region 1, the use of frequency band 694-790 MHz by the mobile, except aeronautical mobile, service is subject to agreement obtained under No. 9.21 with respect to the aeronautical radionavigation service in countries mentioned in No. 5.312.     (MOD RRB24/510)

2	The criteria for identifying potentially affected administrations under No. 9.21 in this band are given in the Annex to Resolution 760 (Rev.WRC-23) in the form of coordination distances with the most stringent value of a 450 km distance between a base station in the mobile service and a potentially affected station in the aeronautical radionavigation service.     (MOD RRB24/510)

3	Taking into account that No. 5.312 contains only a few countries while a large number of other countries of Region 1 are located at distances that are sufficiently large to exclude a potential for interference to the aeronautical radionavigation service, the Board decided that those administrations whose territories are beyond the distance of 450 km from the countries mentioned in No. 5.312 do not need to apply the No. 9.21 procedure to their mobile service assignments operating under No. 5.312A.

4	Administrations having territories within a distance of 450 km from the countries listed in No. 5.312 are the following: Albania, Armenia, Austria, Azerbaijan, Bosnia and Herzegovina, Belarus, Bulgaria, Czech Rep., Germany, Denmark, Estonia, Finland, Georgia, Greece, Hungary, Croatia, Italy, Iraq, Kazakhstan, Kyrgyzstan, Lithuania, Latvia, Moldova, the Former Yugoslav Rep. of Macedonia, Montenegro, Mongolia, Norway, Poland, Romania, the Russian Federation, Sweden, Serbia, Slovakia, Slovenia, the Syrian Arab Republic, Tajikistan, Turkmenistan, Türkiye, Ukraine and Uzbekistan.     (MOD RRB24/510)

5.312B and 5.314A		(ADD RRB24/510)

1	These provisions stipulate that the use of the frequency bands 694-960 MHz (No. 5.312B) and 698-960 MHz (No. 5.314A) by high-altitude platform stations for International Mobile Telecommunication (IMT) base stations (HIBS) shall be in accordance with Resolution 213 (WRC-23), including the power flux-density (pfd) limits listed in resolves 2, 3, 4.1, 4.2 and 4.3 of that Resolution.

2	Considering that neither these RR provisions nor Resolution 213 (WRC-23) specify the propagation prediction model to be used for the calculation of pfd levels produced by HIBS, the Board decided that Recommendation ITU-R P.528-5 is to be used for the calculation of those pfd levels at 1% of time over a smooth-Earth path, produced at a height of:

–	10 m in application of resolves 2 and 3; and

–	1.5 m in application of resolves 4.1, 4.2 and 4.3.

5.316B

1	This provision stipulates, inter alia, that in Region 1, the allocation to the mobile, except aeronautical mobile, service in the frequency band 790-862 MHz is subject to agreement obtained under No. 9.21 with respect to the aeronautical radionavigation service in countries mentioned in No. 5.312.

2	The criteria for identifying potentially affected administrations under No. 9.21 in this band are given in the Annex to Resolution 749 (Rev.WRC-23) in the form of coordination distances with the most stringent value of a 450 km distance between a base station in the mobile service and a potentially affected station in the aeronautical radionavigation service.     (MOD RRB24/510)

3	Taking into account that No. 5.312 contains only a few countries while a large number of other countries of Region 1 are located at distances that are sufficiently large to exclude a potential for interference to the aeronautical radionavigation service, the Board decided that those administrations whose territories are beyond the distance of 450 km from the countries mentioned in No. 5.312 do not need to apply the No. 9.21 procedure to their mobile service assignments operating under No. 5.316B.

4	Administrations having territories within a distance of 450 km from the countries mentioned in No. 5.312 are the following: Albania, Armenia, Austria, Azerbaijan, Bosnia and Herzegovina, Belarus, Bulgaria, Czech Rep., Germany, Denmark, Estonia, Finland, Georgia, Greece, Hungary, Croatia, Italy, Iraq, Kazakhstan, Kyrgyzstan, Lithuania, Latvia, Moldova, the Former Yugoslav Rep. of Macedonia, Montenegro, Mongolia, Norway, Poland, Romania, the Russian Federation, Sweden, Serbia, Slovakia, Slovenia, the Syrian Arab Republic, Tajikistan, Turkmenistan, Türkiye, Ukraine and Uzbekistan.     (MOD RRB24/510)

5.327A

1	Appendix 4 does not contain data elements which would enable examination as to whether the notified frequency assignment is associated to a system that operates in accordance with recognized international aeronautical standards or to a system that operates under other standards. As the Bureau has no means to make such differentiation, the Board decided that the Bureau shall make no examination of the notified frequency assignment to a station in the aeronautical mobile (R) service (AM(R)S) from the view point of its conformity with this provision.

2	With respect to the requirements contained in resolves 2 and 3 of Resolution 417 (Rev.WRC-15), the Board decided that the Bureau shall make no examination of the notified frequency assignment to a station in the AM(R)S from the view point of its conformity with these provisions since Appendix 4 does not contain data elements which would enable to determine whether the notification is related to a Universal Access Transceiver system or to another system in the AM(R)S.

3	With respect to the power limits contained in resolves 6 of Resolution 417 (Rev.WRC-15), the Board decided that the Bureau shall check the e.i.r.p. limits for the ground based and airborne stations only for the band 960-1 164 MHz since frequency assignments to stations in the AM(R)S notified in the band 960-1 164 MHz do not contain any information concerning out-of-band emissions in the frequency band 1 164-1 215 MHz.

5.328AA

1	Appendix 4 does not contain data elements which would enable examination as to whether a notified frequency assignment in the aeronautical mobile-satellite (R) service (AMS(R)S) is associated to the space station reception of Automatic Dependent Surveillance-Broadcast (ADS-B) emissions from aircraft transmitters that operate in accordance with recognized international aeronautical standards or reception of emissions from aircraft transmitters that operates under other standards. As the Bureau has no means to make such differentiation, the Board decided that the Bureau shall make no examination of the notified frequency assignment in the AMS(R)S from the viewpoint of its conformity with this provision.

2	With respect to the requirements contained in resolves 1, 2 and 3 of Resolution 425 (Rev.WRC-19) and in the absence of relevant data elements in Appendix 4, the Board also decided that the Bureau shall make no examination on the conformity with the above resolves of Resolution 425 (Rev.WRC-19).

5.329

Assignments to stations of the radionavigation-satellite service if recorded need to indicate that they shall not cause harmful interference to assignments to stations of the radionavigation service of the countries listed in No. 5.331 and to stations of the radiolocation service (Symbol R in Column 13B2 and reference to No. 5.329 in Column 13B1).

5.340

The comments made under the Rules of Procedure concerning No. 4.4 apply.

5.341A

1	This provision stipulates, inter alia, that in Region 1 the use of IMT stations in the frequency bands 1 427-1 452 MHz and 1 492-1 518 MHz is subject to agreement obtained under No. 9.21 with respect to the aeronautical mobile service used for aeronautical telemetry in accordance with No. 5.342. Since No. 5.342 applies in the band 1 429-1 535 MHz, the use of IMT stations which operate in the frequency band 1 427-1 429 MHz and do not overlap into the band 1 429-1 535 MHz, used by aeronautical telemetry in the aeronautical mobile service, is not subject to the agreement obtained under No. 9.21.

2	Taking into account that No. 5.342 contains only a few countries while a large number of other countries of Region 1 are located at distances that are sufficiently large to exclude a potential for interference to the aeronautical mobile service, the Board decided that those administrations whose territories are beyond the distance of 670 km from the countries mentioned in No. 5.342 do not need to apply the No. 9.21 procedure to their IMT stations operating under No. 5.341A. For those administrations whose territories are closer than the distance of 670 km Section B6 applies.

3	Administrations having territories within a distance of 670 km from the countries mentioned in No. 5.342 are the following: Albania, Armenia, Austria, Azerbaijan, Bosnia and Herzegovina, Belarus, Bulgaria, Czech Rep., Germany, Denmark, Estonia, Finland, Georgia, Greece, Hungary, Croatia, Iraq, Italy, Kazakhstan, Kyrgyzstan, Lithuania, Latvia, Moldova, the former Yugoslav Republic of Macedonia, Montenegro, Mongolia, Norway, Poland, Romania, the Russian Federation, Sweden, Serbia, Slovakia, Slovenia, the Syrian Arab Republic, Tajikistan, Turkmenistan, Türkiye, Ukraine and Uzbekistan.     (MOD RRB24/510)

5.346

1	This provision stipulates, inter alia, that the implementation of IMT in the frequency band 1 452-1 492 MHz in a number of Region 1 countries, which are listed in this footnote, is subject to agreement obtained under No. 9.21 with respect to the aeronautical mobile service used for aeronautical telemetry in accordance with No. 5.342.

2	Taking into account that No. 5.342 contains only a few countries while a large number of the countries listed in No. 5.346 are located at the distances that are sufficiently large to exclude a potential for interference to the aeronautical mobile service, the Board decided that those administrations whose territories are beyond the distance of 670 km from the countries mentioned in No. 5.342 do not need to apply the No. 9.21 procedure to their IMT stations operating under No. 5.346. For those administrations whose territories are closer than the distance of 670 km Section B6 applies.

3	The administration listed in No. 5.346 having territories within a distance of 670 km from the countries mentioned in No. 5.342 is Iraq.

5.351

1	This provision permits, in derogation of the definitions contained in Nos. 1.70, 1.72, 1.76 and 1.82, the use of the bands allocated to a mobile-satellite service by a station at a specified fixed point (without being a coast, land, base or an aeronautical earth station).

2	The exceptional circumstances referred to in this provision cannot be evaluated by the Bureau.

3	The Board therefore concluded that assignments notified under this provision shall receive a favourable regulatory finding.

5.357

The terrestrial uses authorized by this provision appear to be closely related to the operational conditions within a combined aeronautical system using space and terrestrial radiocommunications. The Bureau has no means to verify such uses and considers this provision an additional allocation to the aeronautical mobile (R) service.

5.364

This provision contains two different types of equivalent isotropically radiated power (e.i.r.p.) density limits for transmitting mobile earth stations in the frequency band 1 610-1 626.5 MHz, namely:

a)	peak e.i.r.p. density limit, and

b)	mean e.i.r.p. density limit.

The peak e.i.r.p. density limit is derived from the maximum power density of the assignment as submitted by the responsible administration.

For the second type, it is not clear whether it is spectral mean, or temporal mean, or spatial mean. The Board decided that, on a provisional basis, and until the relevant ITU-R Recommendation is available, the Bureau use a spectral mean e.i.r.p. density when applying this provision. This spectral mean e.i.r.p. will be derived from the mean power density of an assignment, which is obtained from its total power divided by its necessary bandwidth and multiplied by 4 kHz.

5.366

This provision is considered an additional allocation to the aeronautical radionavigation-satellite service. The comments made under No. 5.49 apply. However, when the Special Section is to be published it shall contain an indication that the assignment is for use on a worldwide basis for “airborne electronic aids to air navigation and any directly associated ground-based or satellite-borne facilities”.

5.376

The comments made under the Rules of Procedure concerning No. 5.357 apply.

5.388A and 5.409A	(ADD RRB24/510)

1	No. 5.388A stipulates that the use of the frequency bands 1 710-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and the frequency bands 1 710-1 980 MHz and 2 110-2 160 MHz in Region 2 by high-altitude platform stations for International Mobile Telecommunication (IMT) base stations (HIBS) shall be in accordance with Resolution 221 (Rev.WRC-23), including the power flux-density (pfd) limits listed in resolves 1.1, 1.2, 1.3 and 1.4 of that Resolution.

2	No. 5.409A stipulates that the use of the frequency band 2 500-2 690 MHz in Regions 1 and 2 and the frequency band 2 500-2 655 MHz in Region 3 by HIBS shall be in accordance with Resolution 218 (WRC-23), including the power flux-density (pfd) limits listed in resolves 1.1, 1.2, 1.3 and 1.4 of that Resolution.

3	Considering that neither these RR provisions nor those Resolutions specify the propagation prediction model to be used for the calculation of pfd levels produced by HIBS, the Board decided that Recommendation ITU-R P.528-5 is to be used for the calculation of those pfd levels at 1% of time at a height of 1.5 m over a smooth-Earth path in application of the resolves parts of Resolutions 218 (WRC-23) and 221 (Rev.WRC-23).

5.399

The Board instructed the Bureau when recording assignments to stations of the radiodetermination-satellite service operating in the frequency band 2 483.5-2 500 MHz to which this footnote applies to place Symbol R in Column 13B2 and a reference to No. 5.399 in Column 13B1.

5.415

1	In this provision, the allocation “is limited to national and regional systems”. The Board concluded that a national system is a system having a service area limited to the territory of the notifying administration. As a consequence of this, the regional system to which reference is made shall be considered to be an aggregate of two or more national systems; they shall be limited to the territories (which are not necessarily restricted to bordering countries) of the administrations concerned and they shall be notified by one of these administrations on behalf of all the administrations concerned. When the allocation is made to more than one Region, a regional system may cover territories in those Regions for which the allocation exists. The Board reached this conclusion keeping in mind No. 5.2.1, relating to the interpretation of the word “regional” without a capital “R”.

2	In accordance with this provision, the fixed-satellite service is limited for use by national or regional systems in the band 2 500-2 690 MHz in Region 2 and in the bands 2 500-2 535 MHz and 2 655-2 690 MHz in Region 3. Only those assignments which satisfy the following conditions shall be considered to be in conformity with the Table of Frequency Allocations:

a)	The service area for a regional system is within the Region concerned, i.e. in Region 2 only in the band 2 535-2 655 MHz or in Regions 2 and 3 in the other bands between 2 500 and 2 690 MHz and:

i)	When an administration submits a coordination request for a service area that covers its national territory and extends beyond it, the responsible administration shall submit at the same time the list of administrations that agreed to form the regional system and the service area shall be formed accordingly. If no agreement is obtained, the service area shall be limited to its national territory;

ii)	When an administration submits a coordination request for a service area that does not include its national territory but only territories of other administrations, it shall submit at the same time the list of administrations that agreed to form the regional system and the service area shall be formed accordingly. If no agreement is obtained, the relevant assignments shall be considered not to be in compliance with the Table of Frequency Allocations and the finding shall be unfavourable.

b)	In the case of a national system, the service area is limited to the territory under the jurisdiction of the notifying administration.

c)	If the satellite network is operated within the framework of an international system to which other countries pertain, the notice must indicate that the use is limited to the Region(s) concerned.

5.416

1)	See comments under the Rules of Procedure concerning No. 5.415 about the use limited to national and regional systems.

2)	In view of the indications in this provision, the Board concluded that the reference to the coordination procedure of No. 9.19 in this provision is a matter for administrations. Consequently, at the stage of examination under No. 11.32, the Bureau will not make any examination of the notified frequency assignment to a transmitting station of a terrestrial service or to a transmitting earth station in the FSS (Earth-to-space) from the viewpoint of its conformity with No. 9.19.

5.418C

1	In accordance with provision No. 5.418C, modified by WRC-03, the use of the band 2 630-2 655 MHz by geostationary-satellite networks is subject to the application of the provisions of No. 9.13 with respect to non-GSO satellite systems in the BSS (sound) pursuant to No. 5.418, as of 3 June 2000.     (MOD RRB21/479)

2	The Board undertook an in-depth examination of the different procedures and provisions that apply to satellite systems in the band 2 630-2 655 MHz and noted the difficulty in linking No. 5.418C reference to “notification information” of GSO BSS systems to the No. 22.2 application referred to in No. 5.418A.

3	In the above context, and taking into account of WRC-03 discussion and decision, the Board understands that No. 9.13 coordination applies as described in the Table below.

| GSO satellite network | Date of receipt of coordination information (No. 9.6) | Date of receipt of notification information (No. 11.2) | No. 9.13 applicability |
| --- | --- | --- | --- |
| BSS (No. 5.418) | < 3.6.2000 | < 3.6.2000 | NO |
|  | < 3.6.2000 | ≥ 3.6.2000 | NO |
|  | ≥ 3.6.2000 | ≥ 3.6.2000 | YES |

Band 2 630-2 655 MHz

1	Provisions Nos. 5.416, 5.418, 5.418A, 5.418B and 5.418C provide information on the different constraints and procedures applying to the broadcasting-satellite service (BSS) and fixed-satellite service (FSS) in the frequency range 2 630-2 655 MHz.

2	The Board undertook an in-depth examination of the different provisions and the applicability of the different coordination procedures (space network-to-space network (Nos. 9.7, 9.12, 9.12A and 9.13)) that apply to satellite systems in the band 2 630-2 655 MHz and noted the possible difficulty in assessing the service (BSS (sound), BSS (television), FSS) and the nature of the satellite network (GSO or non-GSO) to which Nos. 5.418A, 5.418B and 5.418C, should apply, taking due account of the dates of reception of the complete Appendix 4 coordination or notification information, as appropriate. Indeed, in the band 2 630-2 655 MHz, No. 5.418A refers to the application of the provisions of No. 9.12A for non-GSO systems in the BSS (sound) in certain countries listed in No. 5.418, in respect of GSO systems; without further details on the involved services; No. 5.418B refers to the application of the provisions of No. 9.12 for non-GSO systems in the BSS under No. 5.418, in respect of other non-GSO systems; and No. 5.418C refers to the application of No. 9.13 by GSO networks in respect of non-GSO systems in the BSS (sound), allocated under No. 5.418.

3	Taking the above into account and in the light of WRC-03 discussions and decisions, in particular the addition of an explicit reference to No. 5.418 in Nos. 5.418B and 5.418C, the Board understands Nos. 5.418A, 5.418B and 5.418C to only address cases of coordination as follows: non-GSO BSS (sound) (Nos. 5.418) systems vis-à-vis any GSO systems under No. 9.12A, and vis-à-vis any non-GSO systems under No. 9.12, and vice versa, i.e. any GSO systems vis-à-vis non-GSO BSS (sound) (Nos. 5.418) systems under No. 9.13, and any non-GSO systems vis-à-vis non-GSO BSS (sound) (Nos. 5.418) systems under No. 9.12, as described in the Table below. This Table applies to coordination requirements between GSO and non-GSO satellite systems for which the API has been received following 1 January 1999 and complete coordination/notification information was received after 2 June 2000 in the band 2 630-2 655 MHz.

| Coordination request (CR): Column vis-à-vis Row () (2 630-2 655 MHz) | Non-GSO BSS (sound) (5.418) | GSO BSS (5.416, 5.418) or FSS (Region 2) | Non-GSO BSS (5.416) or FSS (Region 2) |
| --- | --- | --- | --- |
| Non-GSO BSS (sound) (5.418) | 9.12 (5.418B) | 9.13 (5.418C) | 9.12 (5.418B) |
| GSO BSS (sound) (5.416, 5.418) or FSS (Region 2) | 9.12A (5.418A) | 9.7 | No CR 22.2 |
| Non-GSO BSS (5.416) or FSS (Region 2) | 9.12 (5.418B) | No CR 22.2 | No CR |

5.441

1	Article 5 defines, in the band 10.7-11.7 GHz, a bidirectional allocation for the fixed-satellite service (FSS) in Region 1. Three provisions (Nos. 5.441, 5.484 and 5.484A) further regulate the usage of the bands. The provisions of No. 5.484 apply to the up-link (Earth-to-space) allocation for BSS feeder links. Nos. 5.441 and 5.484A (covering parts of the band 10.7-11.7 GHz) apply to the down-link. The following problems were noted:

1.1	the Table of Frequency Allocations defines a bidirectional allocation of the whole band 10.7-11.7 GHz for the FSS in Region 1. Number 5.484 defines the up-link allocation for Region 1, while Nos. 5.441 and 5.484A regulate the down-link use for GSO and non-GSO FSS systems. The sub-bands 10.7-10.95 GHz and 11.2-11.45 GHz, for the space-to-Earth direction, are, for GSO applications, covered by the provisions of Appendix 30B. The up- and down-link allocations, for GSO use, are of the same category. Non-GSO uses are under equivalent power flux-density limitations defined by Article 22 and are subject to certain conditions as stipulated in No. 5.484A. The application of No. 22.2 is described in No. 22.5I;

1.2	the applicable Radio Regulatory Procedures for the FSS are as follows:

a)	Earth-to-space (No. 5.484): 10.7-11.7 GHz (Region 1): Articles 9 and 11 apply;

b)	space-to-Earth:

10.7-10.95 GHz and 11.2-11.45 GHz:

–	for GSO use:  Appendix 30B (and Article 11) apply (No. 5.441);

–	for non-GSO:  Articles 9, 11 and 22 apply.

10.95-11.2 GHz and 11.45-11.7 GHz:

–	for GSO:  Articles 9 and 11 apply;

–	for non-GSO:  Articles 9, 11 and 22 apply.

2	The regulatory relationship between GSO FSS uses, namely the up-link (Region 1) and the down-link (Appendix 30B) utilization of the spectrum is not covered by any Radio Regulatory procedure. The Board thus considered this situation as follows. Based on the general principle that the utilization of the spectrum by two internationally recognized applications (coordinated vs. planned use), with the same status, should be mutually taken into account even if the case is not covered by specific procedures and also on the basis of the existing analogies (Article 7 of Appendix 30, Article 7 of Appendix 30A, the Board considering that:

a)	up to now the Bureau has received only one case of the bidirectional use by GSO FSS of the bands 10.7-10.95 GHz and 11.2-11.45 GHz, and

b)	the complexity of the issue does not justify the establishment of a sophisticated methodology to treat this case, and thus decided that the Bureau act as follows:

2.1	Up-link FSS applications in the bands 10.7-10.95 GHz and 11.2-11.45 GHz (Article 9)

The FSS up-link usage (according to No. 5.484) should protect the continuing rights of the Appendix 30B Plan as well as the entries in the Appendix 30B List, as they evolve. To this effect the FSS up-link networks shall apply the coordination (Article 9) and notification (Article 11) procedures not only vis-à-vis other up-link FSS networks of the same direction (Earth-to-space) but also vis-à-vis the Plan and List entries of the opposite direction (space-to-Earth). To take into account the Appendix 30B Plan within the Article 9 procedure, the Plan shall be considered as a coordinated usage of the spectrum. Administrations responsible for the FSS up-link shall obtain coordination agreements from those other administrations whose systems in the Plan or assignments in the List are likely to be affected. The method and criteria for the identification of the administrations to be coordinated with shall be, similar to the case of Appendix 30A (where the same bidirectional problem exists between planned feeder links and other FSS), as follows:

a)	Since in the space-to-space interference scenario a receiving space station of the up-link FSS is subject to receive interference from a transmitting space station of the Appendix 30B FSS Plan, and since currently an agreed method for the assessment of this interference is not available to the Bureau, assignments to receiving space stations operating in the up-link FSS submitted under Articles 9 or 11, shall provisionally not undergo the examination relating to compatibility with Appendix 30B. Therefore a note shall be included in the relevant Special Section to reflect the situation and a symbol shall be inserted in the Master Register to indicate that such assignments shall not claim protection from Appendix 30B.

b)	For the compatibility assessment between earth stations (transmitting ES of the FSS up-links and the receiving ES within the Plan allotment) the method defined in Appendix 7 will be used. The service areas defined in Appendix 30B will be extended by the coordination distance to form an “agreement area” within which a transmitting earth station of the FSS up-link has to be coordinated. For the calculation of the coordination distance the most up-to-date ITU-R Recommendation will be used.

2.2	Down-link FSS applications in the bands 10.7-10.95 GHz and 11.2-11.45 GHz (Appendix 30B, planned usage):

a)	As for the interference which is likely to be caused to FSS up-link from Appendix 30B down-link the same condition referred to in 2.1 a) above applies, i.e., in the examination of Appendix 30B Plan and List entries no account shall be taken of the FSS up-link assignments included in the MIFR with the above-mentioned symbol.

b)	As for the interference which is likely to be caused to Appendix 30B down-link receiving earth stations from FSS up-link transmitting earth stations the same condition referred to in 2.1 b) above applies.

5.441B

This provision stipulates, inter alia, that before an administration brings into use an IMT station in the mobile service in the frequency band 4 800-4 990 MHz, it shall ensure that the power flux-density (pfd) produced by this station does not exceed −155 dB(W/(m2 ‧ 1 MHz)) produced up to 19 km above sea level at 20 km from the coast, defined as the low-water mark, as officially recognized by the coastal State. Resolution 223 (Rev.WRC-23) applies.     (MOD RRB24/510)

Considering that this provision and Resolution 223 (Rev.WRC-23) do not specify the propagation model to be used for the calculation of the pfd produced by IMT stations in the band 4 800 -4 990 MHz, the Board decided that Recommendation ITU-R P.528-5, for 1% of time, is to be used for this calculation.     (MOD RRB24/510)

5.444B

1	This provision limits the use of the band 5 091-5 150 MHz by the aeronautical mobile service to two different applications. However, Appendix 4 does not contain data elements which would enable examination as to whether the notified frequency assignment is associated with any of these specific applications or with other applications in the aeronautical mobile service. As the Bureau has no means to make such differentiation, the Board decided that the Bureau shall make no examination of the notified frequency assignment to a station in the aeronautical mobile service from the view point of its conformity with this provision.

2	With respect to the submissions in the aeronautical mobile (R) service, including the ones referred to in the first indent of this provision, and given the indications in resolves 1 of Resolution 748 (Rev.WRC-19), the recording of any such assignment in the MIFR will be associated with the symbol “R” in column 13B2 (“Finding observation”) and with symbol “RS748” in column 13B1 (“Finding reference”). The Board also considered that the indications in resolves 3 of Resolution 748 (Rev.WRC-19), including the reference to No. 4.10, are intended for administrations and the Bureau shall make no examination of frequency assignments from the view point of their conformity with the conditions set forth in resolves 3 of Resolution 748 (Rev.WRC-19).

3	With respect to the submissions related to aeronautical telemetry transmissions referred to in the second indent of this provision, and in addition to the considerations in § 1 of this Rule of Procedure which are also applicable for aeronautical telemetry applications, the Board considered that the indications in resolves 1 and in resolves 2 of Resolution 418 (Rev.WRC-19) are intended for administrations and the Bureau shall make no examination of the notified frequency assignment to a station in the aeronautical mobile service from the view point of its conformity with the conditions set forth in Annex 1 to Resolution 418 (Rev.WRC-19).

5.446A			(MOD RRB24/510)

1	This provision stipulates that the use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the stations in the mobile, except aeronautical mobile, service 
shall be in accordance with Resolution 229 (Rev.WRC-23). Accordingly, Resolution 229 (Rev.WRC-23) specifies that the use of these bands, by the mobile service, will be for the implementation of wireless access systems (WAS) including radio local area networks (RLAN) (see resolves 1) and, in addition to this, it specifies the maximum e.i.r.p. levels for stations in the mobile service (see resolves 2, 3, 5 and 7).

As far as the band 5 150-5 350 MHz is concerned, the situation is rather simple, given the fact that the provisions of Resolution 229 (Rev.WRC-23) are applicable to all stations in the mobile, except aeronautical mobile, service, with the exception of cases referred to in No. 5.447, which apply to the band 5 150-5 250 MHz and where other (e.g. less stringent) conditions may be established in the context of the application of the procedure of No. 9.21.

On the other hand, the situation in the band 5 470-5 725 MHz is more complex, bearing in mind that other provisions are applicable to stations in the mobile, except aeronautical mobile, service (e.g. those indicated in Nos. 5.451, 5.453 and in Table 21-2 of Article 21), which are stipulating different conditions (e.g. power limits) than the ones indicated in Resolution 229 (Rev.WRC-23). Consequently, administrations referred to in No. 5.453 (for the band 5 650-5 725 MHz) and in No. 5.451 (for the band 5 470-5 725 MHz) may implement other applications in the mobile, except aeronautical mobile, service, which are not necessarily WAS, subject to compliance with the conditions set forth in No. 5.451 and the power limits set forth in Table 21-2 of Article 21.

2	Given the fact that, for the implementation of WAS, high deployment densities are expected, such implementation options could be adequately covered through notifications in the form of typical stations. The notification of terrestrial stations in the mobile, except aeronautical mobile, service in the form of typical stations is normally possible with no restrictions in the bands 5 150-5 350 MHz and 5 470-5 670 MHz in all countries, and in the band 5 670-5 725 MHz in the countries not mentioned in No. 5.453. However, provision No. 11.21A, in conjunction with Table 21-2, does not provide for the possibility of notifying terrestrial stations in the mobile, except aeronautical mobile, service, in the form of typical stations, for the band 5 670-5 725 MHz, for the countries listed in No. 5.453. The strict application of these provisions would mean that the countries listed in No. 5.453 cannot notify their WAS applications in the form of typical stations, even though they conform with the limits of Resolution 229 (Rev.WRC-23). The Board concluded that such a restricted interpretation of all the relevant provisions for the band 5 670-5 725 MHz, for the countries listed in No. 5.453, would result in unnecessary burden for both the administrations listed in No. 5.453 and the Bureau. Consequently, the Board instructed the Bureau to accept notifications for mobile, except aeronautical mobile, stations, in the form of typical stations, from the administrations listed in No. 5.453, provided that the maximum e.i.r.p. does not exceed 1 W, which implies that each typical station notice receivable in the band 5 670-5 725 MHz (with an e.i.r.p. of less than or equal to 1 W) is deemed to be part of a WAS.

5.457D, 5.457E 
and 5.457F 			(ADD RRB24/510)

1	These provisions stipulate that the use of the frequency bands 6 425-7 125 MHz (in Region 1 and some countries in Regions 2 and 3) and 7 025-7 125 MHz (in Region 3) by the terrestrial component of International Mobile Telecommunications (IMT) shall be in accordance with Resolution 220 (WRC-23).

Resolution 220 (WRC-23) specifies the technical conditions for the terrestrial component of IMT within the band 6 425-7 125 MHz. Accordingly, resolves 2 of Resolution 220 (WRC-23) specifies that in order to ensure protection for the FSS (Earth-to-space), the level of expected equivalent isotropically radiated power (e.i.r.p.) spectral density emitted by an IMT base station as a function of the vertical angle above the horizon shall not exceed the values given in resolves 2 of that Resolution. No. 21.5 does not apply.

2	Considering that Appendix 4 does not contain the required data items to notify information on the expected e.i.r.p. spectral density mask specified in resolves 2 of Resolution 220 (WRC-23), the Board decided that when notifying frequency assignments for use by IMT base stations subject to resolves 2 of Resolution 220 (WRC-23), administrations notifying such frequency assignments (i.e. with the nature of service “IM”) in the band 6 425-7 075 MHz shall provide in the “Remarks” field of each notice a commitment that the relevant IMT base station meets the expected e.i.r.p. spectral density mask specified in resolves 2 of Resolution 220 (WRC-23), for example, by the statement “complies with resolves 2 of Res. 220”. When examining compliance with resolves 2 of Resolution 220 (WRC-23), the Bureau shall accept a notice with the commitment statement that it is in compliance with this Resolution. In the absence of such a commitment, the notified frequency assignment will receive an unfavourable regulatory finding under No. 11.31.

5.458

There is no allocation to the Earth exploration-satellite (passive) and space research (passive) services in the frequency bands 6 425-7 075 MHz and 7 075-7 250 MHz. Notification of frequency assignments to Earth exploration-satellite (passive) and space research (passive) services in the band 6 425-7 075 MHz and 7 075-7 250 MHz will be considered by the Bureau not to be in conformity with the Table of Frequency Allocations.

5.461		(ADD RRB24/510)

The Board noted that the World Radiocommunication Conference (Dubai, 2023) (WRC-23) had decided on specific conditions for the application of No. 9.21 for geostationary-satellite orbit (GSO) mobile-satellite service (MSS) networks and non-geostationary-satellite orbit (non-GSO) MSS systems in the frequency bands 7 250-7 375 MHz (space-to-Earth) and 7 900-8 025 MHz (Earth-to-space), i.e. that coordination under No. 9.21 shall not apply to GSO MSS networks for which complete coordination information is received by the Bureau as of 1 January 2025 with respect to non-GSO systems for which complete coordination or notification information, as appropriate, is received by the Bureau as of 1 January 2025.

Also, this provision stipulates that non-GSO systems for which complete coordination or notification information, as appropriate, is received by the Bureau as of 1 January 2025 shall not cause unacceptable interference to, or claim protection from, GSO MSS networks operating in accordance with the Radio Regulations.

The Board concluded that the application of No. 9.21 for satellite networks and systems in the MSS in the frequency bands 7 250-7 375 MHz (space-to-Earth) and 7 900-8 025 MHz (Earth-to-space) is as described in the Table below.

|  | Incoming |  | Existing |  | No. 9.21 applicability (see Preface to the BR IFIC (space services), Table 11A.1) |
| --- | --- | --- | --- | --- | --- |
|  | Network/system | Date of receipt of coordination information (No. 9.6) | Network/system | Date of receipt of coordination (No. 9.6) or first notification information (No. 11.2) |  |
| 7 250-7 375 MHz |  |  |  |  |  |
| GSO vs non-GSO | GSO MSS | < 01.01.2025 | Non-GSO FSS or MSS | < 01.01.2025 | YES (9.21/B) |
|  | GSO MSS | >= 01.01.2025 | Non-GSO FSS or MSS | < 01.01.2025 | YES (9.21/B) |
|  | GSO MSS | >= 01.01.2025 | Non-GSO FSS or MSS | >= 01.01.2025 | NO |
|  | Non-GSO MSS | Any | GSO MSS or FSS | Any | YES (9.21/A) |
| GSO vs GSO | GSO MSS | Any | GSO MSS or FSS | Any | YES (9.21/A) |
| GSO, non-GSO vs terrestrial | GSO MSS Non GSO MSS | Any | Terrestrial | Any | NO1 |
| 7 900-8 025 MHz |  |  |  |  |  |
| GSO vs non-GSO | GSO MSS | < 01.01.2025 | Non-GSO FSS or MSS | < 01.01.2025 | YES (9.21/B) |
|  | GSO MSS | >= 01.01.2025 | Non-GSO FSS or MSS | < 01.01.2025 | YES (9.21/B) |
|  | GSO MSS | >= 01.01.2025 | Non-GSO FSS or MSS | >= 01.01.2025 | NO |
|  | Non-GSO MSS | Any | GSO MSS or FSS | Any | YES (9.21/A) |
| GSO vs GSO | GSO MSS | Any | GSO MSS or FSS | Any | YES (9.21/A) |
| GSO, non-GSO vs terrestrial | GSO MSS Non-GSO MSS | Any | Terrestrial | Any | YES (9.21/C) |

5.461AC		(ADD RRB24/510)

This provision stipulates that, in the frequency band 7 375-7 750 MHz, non-geostationary-satellite orbit (non-GSO) systems operating in the fixed-satellite service (FSS) for which complete coordination or notification information, as appropriate, is received by the Bureau as of 1 January 2025 shall not cause unacceptable interference to, or claim protection from, geostationary-satellite orbit networks in the maritime mobile-satellite service operating in accordance with the Radio Regulations.

Since non-GSO systems in the FSS in the frequency band 7 375-7 750 MHz (space-to-Earth) are not subject to the coordination procedure under Section II of Article 9, the Board concluded that No. 5.461AC applies to non-GSO systems operating in the FSS for which complete notification information is received by the Bureau as of 1 January 2025.

5.474A, 5.475A, 5.478A		(ADD RRB24/510)

1	Pursuant to Nos. 5.474A, 5.475A and 5.478A of the Radio Regulations, the Board noted that the use of active sensors in the space research service (SRS) (active) in the frequency band 9 300-9 900 MHz and in the Earth exploration-satellite service (EESS) (active) in the frequency band 9 200-10 400 MHz requires demonstration of compliance of such use with those footnotes, which means that the different sub-bands may only be used in a specific order based on the increasing requirement of the necessary bandwidth of the frequency assignment under consideration:

1.1	For active sensors in both the SRS (active) and the EESS (active):

–	For any frequency assignment with necessary bandwidth of 300 MHz or less, only the frequency band 9 500-9 800 MHz shall be used.

–	For any frequency assignment with necessary bandwidth greater than 300 MHz but less than or equal to 500 MHz, part or the whole of the frequency band 9 300-9 500 MHz, in addition to the frequency band 9 500-9 800 MHz, shall be used.

–	For any frequency assignment with necessary bandwidth greater than 500 MHz but less than or equal to 600 MHz, part or the whole of the frequency band 9 800-9 900 MHz, in addition to the frequency band 9 300-9 800 MHz, shall be used.

1.2	For the EESS (active) only, in addition to the conditions listed in § 1.1:

–	For any frequency assignment with necessary bandwidth greater than 600 MHz but less than or equal to 1 200 MHz, part or the whole of the frequency bands 9 200-9 300 MHz and/or 9 900-10 400 MHz, in addition to the frequency band 9 200-9 900 MHz, may be used.

2	The Board further noted that frequency assignments to non-geostationary-satellite orbit (non-GSO) systems in the SRS (active) and the EESS (active) in the frequency band 9 300-9 900 MHz are not subject to a coordination procedure and shall therefore be submitted in an advance publication of information in accordance with Section I of Article 9.

3	Since the use of the frequency bands 9 200-9 300 MHz and 9 900-10 400 MHz by active sensors in the EESS (active) is subject to an agreement to be obtained under No. 9.21, a request for coordination shall be submitted under No. 9.30. Furthermore, the Board concluded that the use of the frequency band 9 300-9 900 MHz shall also be submitted, either at the same time or in an earlier submission, under the same satellite name (in the case of a non-GSO system, this should be done through the submission of a notice for advance publication information)1; otherwise, the frequency assignments for the use of the EESS (active) in the frequency bands 9 200-9 300 MHz and/or 9 900-10 400 MHz submitted as part of the request for coordination shall not be considered compliant with the Table of Frequency Allocations.

4	When an administration submits a notification under No. 11.2 containing frequency assignments to a station in the EESS (active) in the frequency band 9 200-10 400 MHz and/or in the SRS (active) in the frequency band 9 300-9 900 MHz, the Board decided that the following rules shall apply:

•	When an administration submits a notification for any use in the frequency band 9 300-9 500 MHz, the use of the frequency band 9 500-9 800 MHz shall also be notified in the same service and under the same satellite name, either at the same time or in an earlier submission, and the necessary bandwidth shall be greater than 300 MHz (see No. 5.475A).

•	When an administration submits a notification for any use in the frequency band 9 800-9 900 MHz, the use of the frequency band 9 300-9 800 MHz shall also be notified in the same service and under the same satellite name, either at the same time or in an earlier submission, and the necessary bandwidth shall be greater than 500 MHz (see No. 5.478A).

•	When an administration submits a notification for any use in the frequency bands 9 200-9 300 MHz and 9 900-10 400 MHz, the use of the frequency band 9 300-9 900 MHz shall be notified in the EESS (active) and under the same satellite name, either at the same time or in an earlier submission, and the necessary bandwidth shall be greater than 600 MHz (see No. 5.474A).

When the above conditions are not met, the relevant frequency assignment shall not be considered compliant with the Table of Frequency Allocations under No. 11.31 of the Radio Regulations and shall be given an unfavourable finding and returned to the notifying administration.

5	Notification submissions with separate assigned frequencies and bandwidths within the frequency bands 9 200-9 300 MHz, 9 300-9 800 MHz, 9 800-9 900 MHz and 9 900-10 400 MHz will receive separate findings based on the relevant allocation status for each of the frequency bands.

6	The Board recalled that notification submissions of a frequency assignment with an assigned frequency bandwidth overlapping the frequency band 9 800-9 900 MHz will receive a single finding based on a secondary allocation status in accordance with § 5.5 of the rules of procedure on No. 11.31.

7	Finally, the Board decided that, in order for the Bureau to be able to examine the above-mentioned submissions under No. 11.31, the information on the necessary bandwidth (item C.8.b.3.c of Annex 2 to Appendix 4) shall be provided for all such submissions, except in the case where only the frequency band 9 500-9 800 MHz is used.

5.480A		(ADD RRB24/510)

1	This provision stipulates that the use of the frequency band 10-10.5 GHz (in some Region 2 countries) by the terrestrial component of International Mobile Telecommunications (IMT) shall be in accordance with Resolution 219 (WRC-23).

2	Appendix 4 does not contain data items providing information that would enable examination of the compliance with the requirements of resolves 3, 4 and 5 of Resolution 219 (WRC-23).

Consequently, the Board decided that when administrations notify frequency assignments for use by IMT base stations subject to resolves 3, 4 and 5 of Resolution 219 (WRC-23) (i.e. with the nature of service “IM”) in the frequency band 10-10.5 GHz, they shall provide in the “Remarks” field of each notice a commitment that the IMT base station meets the levels specified in resolves 3, 4 and 5 of Resolution 219 (WRC-23), for example, by the statement “complies with resolves 3, 4 and 5 of Res. 219”. When examining compliance with resolves 3, 4 and 5 of Resolution 219 (WRC-23), the Bureau shall accept such a notice with the commitment statement that it is in compliance with the Resolution. In the absence of such a commitment, the notified frequency assignment will receive an unfavourable regulatory finding under No. 11.31.

5.484

See comments under the Rules of Procedure concerning No. 5.441.

5.485

1	The wording of this provision raised the following basic question: “Is the band 11.7-12.2 GHz in Region 2 allocated to the broadcasting-satellite service?” The Board considered the following:

a)	that the provision is not titled an “additional allocation”. Some provisions do not have such a title and the Board considered them additional allocations. However, in this case, it is not clear that the intent was to permit an additional allocation;

b)	the provision states that “transponders on space stations in the fixed-satellite service may be used additionally ... in the broadcasting-satellite service”: the use of the word “additionally”, together with the last sentence saying that “this band shall be used principally for the fixed-satellite service”, leads to the understanding that the use by the broadcasting-satellite service is not of the same nature as would be the use of a given band by a service to which the band is allocated;

c)	the provision refers to transponders, which are to be considered transmitting stations. As the procedures of Article 9 apply to each assignment, each transponder shall be considered independently from the others. Consequently the provision may be interpreted in either of the following two ways:     (MOD RRB21/479)

–	a first interpretation consists in considering that some transponders will be used for the FSS and others for the BSS, and this is equivalent to a sharing of the band between two services which raises a question about the word “principally”: how many transponders would be allowed for each of the two services?

–	a second interpretation consists in considering that a given transponder of the FSS may be used in a given period of time for broadcasting (this is not to be confused with the use of the FSS for the transport of a video signal between two fixed points). If in such a case the provision was to be considered an additional allocation, a question arises in relation to the procedure to be applied: Should it be the relevant provisions in Article 9 for the FSS or for the BSS?     (MOD RRB21/479)

2	Keeping in mind the above comments, the Board concluded that the band 11.7-12.2 GHz is not allocated in Region 2 to the broadcasting-satellite service. Those transponders of the fixed-satellite service which are used for broadcasting-satellite purposes will be treated in accordance with the relevant provisions in Article 9 for the FSS (and Appendix 30 if required to define inter-regional sharing). When such a use is indicated in the notice, the Bureau will assume that the coordination of the network was made on the basis that for the period during which a transponder is used for broadcasting, the e.i.r.p. will not exceed the e.i.r.p. notified for the fixed-satellite service. Considering that the fixed-satellite service uses relatively low e.i.r.p., the Bureau will consider the value of 53 dBW to be a limit not to be exceeded.     (MOD RRB21/479)

5.488

Application of power flux-density (pfd) No. 9.14 coordination thresholds (Region 2 GSO FSS in the band 11.7-12.2 GHz) to steerable beams

1	Use of steerable beams is becoming widespread. pfd values produced by assignments in steerable beams often exceed the applicable pfd coordination thresholds for some or all positions of those beams. In these cases, administrations tend to state that pfd coordination thresholds will not be exceeded and sometimes provide appropriate technical description as to how it would be done.

2	For the purpose of transparency and to set an upper limit on the acceptable extent of the pfd control and avoid subjectivity in the evaluation of the pfd control method, the Board concluded that until the time that a relevant ITU-R Recommendation is available, the following Rule will apply on a provisional basis.

3	In cases where frequency assignments in steerable beams of a GSO FSS satellite network operating in the band 11.7-12.2 GHz exceed, for certain positions of these beams, the pfd thresholds that trigger coordination under No. 9.14 in respect to stations of terrestrial services, the Bureau will establish that coordination is not required only if:

a)	there is at least one position of the steerable beam where the applicable pfd coordination thresholds are not exceeded without any reduction of the notified power density; and

b)	the administration states that for the other positions of the steerable beam the applicable pfd coordination thresholds will not be exceeded by applying a method, the description of which should be submitted to the Bureau. One possible example of such a method is described in the Annex to the Rule relating to No. 21.16.

5.492

1	The Board concluded that the frequency bands covered by Appendix 30 are not allocated to the FSS in the Regions where the BSS is subject to the Plan of Appendix 30. Those transponders of the BSS which are also used for FSS purposes will be treated in accordance with Article 5 of Appendix 30.

2	Earth stations receiving FSS transmissions from the BSS transponders will be treated as earth stations of the BSS and are not to be notified as individual earth stations.

5.496

1	The fixed and mobile (except aeronautical mobile) services in the countries listed in this provision:

–	have equal rights with the FSS in the countries of the footnote and in the relations between them, and the coordination under No. 9.17 and No. 9.18 shall be applied;

–	shall be operated under No. 5.43 with respect to the FSS in the other countries of Region 1, and coordination under No. 9.17 cannot be imposed on earth stations. The fixed and mobile stations shall apply coordination under No. 9.18;

–	have equal rights with the services to which the band is allocated in Regions 2 and 3.

2	The comments made under the Rules of Procedure concerning No. 5.164 apply.

5.502

1	As from 5 July 2003, No. 5.502 specifies a minimum antenna diameter of 1.2 and 4.5 m for an earth station of a GSO and non-GSO fixed-satellite service network, respectively, in the frequency band 13.75-14 GHz. Submission of information on antenna diameter became mandatory as from 1 January 2004, with the entry into force of Appendix 4 as modified by WRC-03. To cover the examination of submissions received in the period between these two dates, the Bureau is instructed to use the following maximum earth station antenna gains instead of antenna diameter: maximum antenna gain of 42.3 dBi for D  1.2 m and 53.8 dBi for D  4.5 m (the relation between gain and diameter is derived for the lowest frequency of the band, i.e. f  13.75 GHz, and an antenna efficiency of 57.2%).

2	Power flux-density (pfd) limits of No. 5.502 are applicable as from 5 July 2003. In accordance with Appendix 4 as modified by WRC-03 (data element A.16.b) administrations are required to provide commitment of compliance with these limits when notifying or coordinating a specific earth station with a diameter  1.2 m and < 4.5 m. Where, for submissions received between 5 July 2003 and 1 January 2004 (date of entry into force of modified Appendix 4), administrations have not provided the commitment, the Bureau shall establish favourable finding and request the responsible administrations to submit the commitment after 1 January 2004. If the commitment is not submitted within 30 days of the request, the finding shall be changed to unfavourable.

5.503

1	In No. 5.503, maximum e.i.r.p. density of emissions from a transmitting earth station in the FSS in the frequency band 13.77-13.78 GHz are specified. Non-compliance with these limits results in an unfavourable finding under No. 9.35/11.31. No. 5.503 also permits these limits to be exceeded to compensate for rain attenuation as long as the power flux-density at the FSS space station does not exceed the value resulting from the use by an earth station of an e.i.r.p. meeting the limits in clear-sky conditions. However, No. 5.503 and Appendix 4 do not specify which value for the power-density and total power of an emission (with maximum antenna gain they form on-axis e.i.r.p density and e.i.r.p.) is to be provided:

i)	those prevailing in clear-sky conditions, or

ii)	those prevailing during rainfall.

The Board therefore decided that, for frequency assignments to which No. 5.503 applies, administrations shall provide clear-sky values of maximum power-density (Appendix 4 data item C.8.a.2 or C.8.b.2) and maximum power of emissions (Appendix 4 data item C.8.a.1 or C.8.b.1), when requesting coordination or notifying satellite networks or earth stations. This shall apply to coordination requests and notifications received as from 1 January 2009.

2	In No. 5.503, WRC-03 included e.i.r.p. density limits for earth stations as a function of antenna diameter. These limits are applicable as from 5 July 2003. Antenna diameter not being available before 1 January 2004 (see the Rule relating to No. 5.502), the Bureau shall use the following e.i.r.p. density limits, as a function of maximum earth station antenna gain (or antenna diameter), for examination of submissions received between 5 July 2003 and 1 January 2004.

|  | e.i.r.p. density limits in frequency band 13.77-13.78 GHz for earth stations in the FSS operating with GSO space station |  |
| --- | --- | --- |
| Earth station antenna size (measured by diameter, D (m) or maximum antenna gain, G (dBi)) | For emissions with necessary bandwidth |  |
|  | 40 kHz |  40 kHz |
| 42.3 dBi G < 53.8 dBi | 0.04324  10G/20 + 28 dB(W/40 kHz) |  |
| (1.2 m D < 4.5 m) | (4.7D  28 dB(W/40 kHz)) |  |
| 53.8 dBi G < 70.8 dBi | G – 4.6 dB(W/40 kHz) | 56.2 dB(W/4 kHz) |
| (4.5 m D < 31.9 m) | (49.2  20 log(D/4.5) dB(W/40 kHz)) |  |
| G 70.8 dBi | 66.2 dB(W/40 kHz) |  |
| (D 31.9 m) |  |  |

The conversion of e.i.r.p. limits from a function of antenna diameter to a function of maximum antenna gain is derived for a frequency of 13.75 GHz and an antenna efficiency of 57.2%.

5.504B

With respect to the observance of mandatory power flux-density limits and other conditions of Recommendation ITU-R M.1643-0 (in consequence of the modification of No. 5.504B by WRC-15), applicable under Nos. 5.504B, 5.504C, 5.508A and 5.509A to aircraft earth stations operating in the secondary aeronautical mobile-satellite service, the Board is of the opinion that this is an operational matter. It is therefore for the notifying administration of the satellite network and notifying administrations of aircraft earth stations to ensure the observance of those limits. The Bureau will not perform examination under No. 9.35/11.31 with respect to conformity with those conditions.

5.504C

See Rules of Procedure relating to No. 5.504B.

5.506A

As from 5 July 2003, No. 5.506A requires ship earth stations in the frequency band 14-14.5 GHz with an e.i.r.p. greater than 21 dBW to operate under the same conditions as earth stations located on board vessels, as provided in Resolution 902 (Rev.WRC-23). While Annex 2 of that Resolution specifies a minimum antenna diameter of 1.2 m, Appendix 4 does not include antenna diameter of these ship earth stations as a required data element. The Bureau is instructed to use antenna gain value of 42.5 dBi when checking the compliance with the minimum ship earth station antenna diameter requirement (the relation between gain and diameter is derived for the lowest frequency of the band, i.e. f  14 GHz, and antenna efficiency of 57.2%).     (MOD RRB24/510)

5.508A

See Rules of Procedure relating to No. 5.504B.

5.509A

See Rules of Procedure relating to No. 5.504B.

5.509D and 5.509E

When an administration submits a notification or a request for coordination for a frequency assignment to a space station of a satellite network subject to Resolutions 163 (WRC-15) or 164 (WRC-15), the notice should include a commitment by the administration, as required under § A.16 c) of Annex 2 to Appendix 4, indicating that any earth station associated with the filed satellite network will meet the separation distance as specified in No. 5.509E and the power flux-density limits as specified in No. 5.509D.

The Board decided to instruct the Bureau to use the commitment under § A.16 c) in its examination under No.9.35/11.31 of a frequency assignment of a satellite network with respect to its conformity with No. 5.509D and No. 5.509E.

However, the Bureau’s regulatory examination under No. 11.31 of a frequency assignment to an earth station notified under Article 11 will include checking for conformity with the limits of power-flux density produced by this earth station under No. 5.509D and the distance indicated in No. 5.509E.

For the examination under No.5.509D, the Bureau shall calculate the power flux-density under free-space propagation condition for all altitudes within line-of-sight up to 19 000 m above sea level at 22 km seaward from all coasts based on the IDWM (ITU’s Digitized World Map).

5.523B, 5.523C, 5.523D, 5.523E

Provisions Nos. 5.523B, 5.523C, 5.523D and 5.523E provide information on the different constraints and procedures applying to the FSS in the frequency range 19.3-19.7 GHz. The Board studied the interrelationship between the different FSS usages and also vis-à-vis the terrestrial stations. The Tables relative to 19.3-19.6 GHz and 19.6-19.7 GHz bands below contain the Board’s conclusions on this matter.

Table 19.3-19.6 GHz

| Coordination request (CR): Column vis-à-vis Row () | Non-GSO FSS (MSS feeder-link) (RR No.) | Non-GSO FSS (MSS feeder-link) (RR No.) | Non-GSO FSS (other) (RR No.) | GSO FSS (CR 18.11.95) (RR No.) | GSO FSS (18.11.95 CR) (RR No.) | Terrestrial (RR No.) |
| --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS (MSS feeder-link) | 9.12 (5.523B) | 9.12 (5.523B/5.523D) | No CR (5.523D) | 22.2 (5.523C) | 9.12A (5.523B) | --- (5.523B) |
| Earth station | --- | 9.17A | 9.17A | 9.17A | 9.17A | 9.15 |
| Non-GSO FSS (MSS feeder-link) | 9.12 (5.523B/5.523D) | 9.12 (5.523D) | No CR (5.523D) | 22.2 (5.523D) | 9.12A (5.523D) | --- pfd limits (5.523D) |
| Earth station | 9.17A | --- | --- | --- | --- | 9.15 |
| Non-GSO FSS (other) | No CR (5.523D) | No CR (5.523D) | No CR (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | --- pfd limits (5.523D) |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| GSO FSS (CR 18.11.95) | 22.2 (5.523C) | 22.2 (5.523D) | 22.2 (5.523D) | 9.7 | --- | --- pfd limits |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| GSO FSS (18.11.95 CR) | 9.13 (5.523B) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | --- pfd limits |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| Terrestrial | --- | 9.16 (5.523D) | 9.18 (5.523D) | 9.18 | 9.18 | --- |

Table 19.6-19.7 GHz

| CR request: Column vis-à-vis Row () | Non-GSO FSS (MSS feeder-link) (RR No.) | Non-GSO FSS (other) (RR No.) | Non-GSO FSS (MSS feeder-link) (RR No.) | Non-GSO FSS (other) (RR No.) | GSO FSS (CR 21.11.97) (RR No.) | GSO FSS (CR 21.11.97) (RR No.) | GSO FSS (21.11.97 CR) (RR No.) | GSO FSS (21.11.97 CR) (RR No.) | Terrestrial (RR No.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS (MSS feeder-link) | ## 9.12 | ## 9.12 | 9.12 | No CR | 22.2 (5.523E) | 22.2 (5.523E) | 9.12A (5.523D) | 9.12A (5.523D) | (5.523D) |
| Earth station | ## --- | ## --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.15 |
| Non-GSO FSS (other) | ## 9.12 | ## 9.12 | 9.12 | No CR | 22.2 (5.523D) | 22.2 (5.523D) | 9.12A (5.523D) | 9.12A (5.523D) | (5.523D) |
| Earth station | ## --- | ## --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.15 |
| Non-GSO FSS (MSS feeder-link) | # 9.12 | # 9.12 | 9.12 (5.523D) | No CR (5.523D) | 22.2 (5.523E) | 22.2 (5.523E) | 9.12A (5.523D) | 9.12A (5.523D) | pfd limits (21/Section V) (5.523D) |
| Earth station | ## 9.17A | ## 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.15 |
| Non-GSO FSS (other) | No CR | No CR | No CR (5.523D) | No CR (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | pfd limits (21/Section V) (5.523D) |
| Earth station | ## 9.17A | ## 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.17 |
| GSO FSS (CR 21.11.97) | 22.2 (5.523E) | 22.2 (5.523D) | 22.2 (5.523E) | 22.2 (5.523D) | 9.7 | 9.7 | --- | --- | --- |
| Earth station | ## --- | ## --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.17 |
| GSO FSS (CR 21.11.97) | 22.2 (5.523E) | 22.2 (5.523D) | 22.2 (5.523E) | 22.2 (5.523D) | 9.7 | 9.7 | --- | --- | pfd limits (21/Section V) |
| Earth station | ## 9.17A | ## 9.17A | --- | --- | 9.17A | --- | --- | --- | 9.17 |
| GSO FSS (21.11.97 CR) | 9.13 (5.523E) | 9.13 (5.523D) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | 9.7 | 9.7 | --- |
| Earth station | ## --- | ## --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.17 |
| GSO FSS (21.11.97 CR) | 9.13 (5.523D) | 9.13 (5.523D) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | 9.7 | 9.7 | pfd limits (21/Section V) |
| Earth station | ## 9.17A | ## 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.17 |
| Terrestrial | --- | --- | 9.16 (5.523D) | 9.18 (5.523D) | --- | 9.18 | --- | 9.18 | --- |

5.529A		(ADD RRB24/510)

This provision stipulates that, in the frequency bands 20.2-21.2 GHz and 30-31 GHz, non-geostationary-satellite orbit (non-GSO) systems for which complete coordination or notification information, as appropriate, is received by the Bureau as of 1 January 2025 shall not cause unacceptable interference to, or claim protection from, geostationary-satellite orbit networks in the mobile-satellite service (MSS) operating in accordance with the Radio Regulations.

Since non-GSO systems in the fixed-satellite service (FSS) or MSS in the frequency bands 20.2-21.2 GHz and 30-31 GHz are not subject to the coordination procedure under Section II of Article 9, the Board concluded that No. 5.529A applies to non-GSO systems operating in the FSS or MSS for which complete notification information is received by the Bureau from 1 January 2025.

5.538

For up-link power control beacons, this provision sets an e.i.r.p. limit “in the direction of adjacent satellites on the geostationary-satellite orbit”.

The Board is of the opinion that the intention of this provision is to protect parts of the GSO arc adjacent to the satellite under examination in the direction “laterally tangential to the GSO at the position of the network under examination.”

5.543

The Board concluded that this provision is an additional allocation to the Earth exploration-satellite service for inter-satellite links. The use of the words “telemetry, tracking, and control purposes” leads the Board to understand that the use is limited to space operation.

5.554

This provision does not provide additional allocation for the FSS in the frequency bands specified therein. It authorizes links between land stations at specified fixed points within the MSS or radionavigation-satellite service. A land station in the context of the latter two services means a land earth station, which, in accordance with its definition, is a feeder-link earth station. Therefore, a space or earth station in the FSS (class of station EC or TC) is not authorized in the frequency bands listed in No. 5.554 (except in the band 123-130 GHz where there is an FSS allocation) and links between specific (as opposed to typical) feeder-link earth stations (e.g. class of station VA, or TI, or the like) are authorized within the MSS or radionavigation-satellite service.

5.556

There is no allocation to radio astronomy in the bands listed in this provision. The Board concluded that the words “national arrangements” are referring to arrangements to be made in each country. These arrangements are not required to be communicated to the Bureau. Notifications of frequency assignments to radio astronomy stations in these bands will be considered by the Bureau not to be in conformity with the Table of Frequency Allocations.

____________________

_______________

1 	See also the Annex to the rules of procedure on No. 9.36.

1 	In this context, it is understood that the use of the frequency band 9 300-9 900 MHz by a GSO space station in the EESS (active) has to also be submitted in a coordination request in accordance with No. 9.7.