# Rules concerning

## APPENDIX  4 to the RR

An. 1

ITEM 3A1

When submitting a notice within the procedure of Article 11, the administrations are required to provide information on the call sign or other identification used, as requested by Nos. 19.7 to 19.9 and 19.29. Bearing in mind the variety of special arrangements concluded between administrations concerning notification of frequency assignments, the Board instructed the Bureau not to perform systematic control of the call signs referred to in No. 19.29 during the validation and examination of the notice. Nevertheless, if non-conformity of the call sign with the international call series is identified, the notifying administration is to be informed thereof.

An. 2

A.4.b.7.d.1		(ADD RRB24/510)

The Board noted that the World Radiocommunication Conference (Dubai, 2023) had modified data item A.14.c.4, i.e. the type of mask, among one of the following types: (Earth-based exclusion zone angle, difference in longitude, latitude) or (satellite azimuth, satellite elevation, latitude mean power), to remove the reference to satellite-based exclusion zone angle and difference in longitude, latitude – the so-called X-DeltaLongitude mask. The change was made following the publication of Recommendation ITU-R S.1503-4, which had removed that type of mask.

The Board further noted that Recommendation ITU-R S.1503-4 also limited type of exclusion zone to only Earth-based exclusion zone by removing the satellite-based exclusion zone method; however, no change had been made to the description of item A.4.b.7.d.1 – the type of zone (based on topocentric angle or satellite-based angle for establishing the exclusion zone).

Since only one type of exclusion zone, which shall be Earth-based (i.e. based on topocentric angle), can be used, the Board decided that notifying Administrations are not required to submit data item A.4.b.7.d.1 and that the Bureau shall apply the Earth-based exclusion zone method for all notices received as of 1 January 2025.

A.18 a)

The Board noted that the description of Annex 2 of Appendix 4, § A.18 a) corresponds to the commitment required from an administration in the case of the possible communication of aircraft earth stations in the secondary aeronautical mobile-satellite service with space stations in the fixed-satellite service, in accordance with No. 5.504A. The Board further noted that this data element is mandatory in the case of submission of notification or coordination of a geostationary or a non-geostationary satellite network.

However, this data element is also required to check under No. 11.31 the compliance with respect to No. 5.504A of the notification of an aircraft earth station in the secondary aeronautical mobile-satellite service communicating with a space station in the fixed-satellite service. This requirement was probably omitted by inadvertence at WRC-03.

To correct this inconsistency, the Board decided that the Administrations will be requested to provide, in addition to the relevant characteristics listed in Appendix 4, the data element described in § A.18 a) of Annex 2 of Appendix 4, when submitting the notification information of an aircraft earth station in the secondary aeronautical mobile-satellite service communicating with a space station in the fixed-satellite service in accordance with No. 5.504A. The Bureau will then subsequently take account of this data element § A.18 a) in its completeness check of the submitted data.

A.27.b			(ADD RRB24/510)

The Board noted that item A.27.b under Annex 2 to Appendix 4 is required only for non-geostationary-satellite orbit (non-GSO) space stations submitted in accordance with Resolution 679 (WRC-23).

The description of the item shares similarities with the text of further resolves 2 of Resolution 679 (WRC-23); however:

–	further resolves 2 refers to a firm, objective, actionable, measurable and enforceable commitment; and

–	the commitment under further resolves 2 shall be provided not only by the notifying administration of a non-GSO system but also by the notifying administration of a geostationary-satellite orbit (GSO) network receiving in the frequency band 27.5-30 GHz.

Consequently, the Board concluded that the commitment referred to in item A.27.b shall be provided by the notifying administration of a GSO network or of a non-GSO system receiving in the frequency band 27.5-30 GHz. The Board recalled that notifying administrations providing a commitment under item A.27.b shall ensure that such commitments be firm, objective, actionable, measurable and enforceable.

A.33.a, A.36.c			(ADD RRB24/510)

The Board noted that a “point of contact” is mentioned in Resolutions 121 (WRC-23), 123 (WRC-23), 156 (Rev.WRC-23), 169 (Rev.WRC-23), 679 (WRC-23) and 902 (Rev.WRC-23) for various purposes.

However, only in two instances, i.e. with respect to resolves 10.5 of Resolution 121 (WRC-23) and resolves 7.5 of Resolution 123 (WRC-23), information on the point of contact is included as a requirement in Annex 2 to Appendix 4 (see mandatory items A.33.a and A.36.c). For both cases, it is indicated that the point of contact is for the purpose of tracing any suspected cases of unacceptable interference and that the point of contact is required to immediately respond to such requests.

Similar descriptions are given in Resolutions 169 (Rev.WRC-23) and 679 (WRC-23): a point of contact is required for the purpose of tracing any suspected cases of unacceptable interference and responding immediately to such cases; however, no requirement to provide information on the point of contact is included in Annex 2 to Appendix 4. Noting the similarity of the requirements for the point of contact described in all those resolutions, the Board decided that item A.36.c of Annex 2 to Appendix 4 is also required for submissions under Resolutions 169 (Rev.WRC-23) and 679 (WRC-23).

The information to be provided on the point of contact shall include the name of the person or entity and the e-mail address, contact telephone number and address. The information shall be captured along with other Appendix 4 data items using the Bureau’s capture software. The Board noted that Resolution 121 (WRC-23) mentions that the information should be published in a special section, whereas Resolution 123 (WRC-23) contains no such mention.

However, the Board understands that all information required under Appendix 4 must be published, although not necessarily in a special section. The Board therefore concluded that the Bureau shall include the information in a reference database and make it available on its website and publish it along with other Appendix 4 data in a relevant special section or part of its International Frequency Information Circular (BR IFIC).

B.4 a)

When submitting a notice within the applicable procedures of Articles 9 or 11, in order to better describe the power flux-density pattern on the surface of the Earth resulting from the emission of a space station aboard a non-GSO satellite in circular orbit, the following optional information can be provided together with the other data contained in Appendix 4:

Appendix 4, Annex 2A, § B.4 a) (non-GSO space station antenna characteristics)

1	In addition to the information currently contained in Appendix 4 to be provided under this item, if appropriate, indicate:

1.1	in the case of a transmitting space station aboard a non-GSO satellite in a circular orbit that is intended to communicate with earth stations via a transmitting antenna pointing in a direction that is fixed with respect to the satellite, the maximum isotropic gain (dBi) and the gain contours plotted in a radial projection from the satellite onto a plane perpendicular to the axis from the centre of the Earth to the satellite. The space station antenna gain contours shall be drawn as isolines of the isotropic gain at least for – 2, – 4, – 6, –10, and – 20 dB and at 10 dB intervals thereafter, as necessary, relative to the maximum antenna gain, when any of these contours is located either totally or partially within the limit of visibility of the Earth from the given non-GSO satellite;

1.2	in the case of a space station aboard a non-GSO satellite in a circular orbit where a steerable beam is used, data on the antenna radiation characteristics as follows:

–	if the effective boresight area (see No. 1.175) is identical with the global or nearly global service area, provide only the maximum isotropic antenna gain (dBi) that is then applicable to all points on the surface of the Earth;

–	if the effective boresight area (see No. 1.175) is less than the global or nearly global service area, provide the maximum isotropic gain and the effective gain contours (see No. 1.176) as defined above.

2	The additional information detailed in § 1.1 and 1.2 above is considered as optional. When examining such a case, the Bureau shall use the more detailed information to calculate power flux-density values if it is provided; if it is not the calculation shall be made as at present and be based on the maximum e.i.r.p. transmitted.

C.8.a.2, C.8.b.2, C.8.c.1, C.8.c.3			(ADD RRB24/510)

The Radiocommunication Bureau previously addressed the issue of excessive or unrealistic characteristics in satellite filings in the Reports of the Director to WRC-15 (see § 3.2.3.9 of revision 1 to Addendum 2 to Document CMR15/4) and WRC-19 (see § 3.4.3 of Addendum 2 to Document CMR19/4). Both Conferences expressed general support for raising those issues (see Documents CMR15/505 and CMR19/451) and invited ITU-R to review the parameters discussed in those sections of the Reports.

Although, at that time, the issue had been raised in general, bearing in mind certain specific submissions of geostationary satellite networks, the Bureau observed a sharp increase in the number of submissions of non-GSO satellite systems containing very low maximum power spectral density of emissions (below −100 dBW/Hz).

In view of the above, the Board decided that frequency assignments to GSO satellite networks with power spectral density levels below −100 dBW/Hz were not receivable, and frequency assignments to non-GSO satellite systems or networks with power spectral density levels below −100 dBW/Hz were only receivable if clarifications were provided to the Bureau on the use of very low power spectral density values (e.g. the mode of operation, the use of spread spectrum) as well as example link budget calculations demonstrating that the submitted required C/N ratio objective was met with sufficient interference margin (see Attachment 2 to Section B3 of Part B of the Rules of Procedure).

C.8.b.3.c			(ADD RRB24/510)

The Board noted that the World Radiocommunication Conference (Dubai, 2023) (WRC-23) added item C.8.b.3.c in order for notifying administrations to submit the necessary bandwidth for active sensors. WRC-23 made the submission of that item mandatory only for active sensors operating in the Earth exploration-satellite service (EESS) (active) in the frequency bands 9 200-9 300 MHz and 9 900-10 400 MHz.

However, in order for the Bureau to be able to examine conformity with respect to Nos. 5.475A and 5.478A, the necessary bandwidth information is also required for active sensors operating in the EESS (active) and the space research service (SRS) (active) when the frequency bands 9 300-9 500 MHz and 9 800-9 900 MHz are used.

Therefore, the Board decided that the necessary bandwidth information under item C.8.b.3.c is also required for active sensors operating in the EESS (active) and the SRS (active) using the frequency bands 9 300-9 500 MHz and 9 800-9 900 MHz at the stage of advance publication of information under Section I of Article 9 (for non-geostationary-satellite orbit systems), at the stage of request for coordination (for geostationary-satellite orbit networks) and at the stage of notification under Article 11.

See also the rules of procedure on Nos. 5.474A, 5.475A and 5.478A.

____________________

_______________