# Rules concerning

## APPENDIX  30B to the RR

Art. 4

## Execution of the provisions and associated Plan

4.1

Bidirectional allocation of some bands

1	See comments made under the Rules of Procedure concerning No. 5.441.

Art. 6

## Procedures for the conversion of an allotment into an assignment
for the introduction of an additional system or for
the modification of an assignment in the List

6.3 a)

1	The footnotes attached to provisions § 6.3 a), 6.19 b), 7.5 a) and 8.8 require that the “other provisions” mentioned in those provisions shall be identified and included in the Rules of Procedure.

The regulatory examinations under § 6.3 a), 6.19 b), 7.5 a) and 8.8 include the following:

–	conformity with the Table of Frequency Allocations, including its footnotes and any Resolution or Recommendation which is referred to in such a footnote;

–	all “other” mandatory provisions that are contained in Articles 21 to 22, in Articles 3 and 4 of Appendix 30B to the Radio Regulations and/or in Resolutions that are relevant to the service in the frequency band in which a station of that service operates.

2	The list of “other provisions” that are contained in Articles 21 to 22 with respect to which the notices are examined, is given below:

2.1	conformity with the power limits for earth stations as stipulated in provisions Nos. 21.8 and 21.12, account being taken of provisions Nos. 21.9 and 21.111, and in provisions Nos. 22.26 to 22.29 under the conditions specified in provisions Nos. 22.30, 22.31 and 22.37 where the earth stations are subject to those power limitations;

2.2	conformity with the minimum angle of elevation of earth stations as stipulated in provisions No. 21.142;

2.3	conformity with the limits of power flux-density from space stations produced at the Earth’s surface as indicated in the Table 21-4 (provision No. 21.16), taking into account, as appropriate, the provision Nos. 21.17; however, the Rules of Procedure relating to No. 21.16 concerning the application of power flux-density (pfd) limits to steerable beams do not apply in this case.

2.4	conformity with the limit specified in provisions Nos. 22.8 and 22.19.

2.5	Other provisions of Articles 21 and 22 will not be taken into account in the Regulatory examination under § 6.3 a), 6.19 b), 7.5 a) and 8.8 and the Board understands that these provisions are to be applied between administrations as appropriate.

6.5

1	(Not used)

2	The Board, in reviewing the implementation of the regulatory procedures of Appendix 30B, noted that there is no provision to prohibit the implementation of non-simultaneous transmissions within the context of that Appendix. The Board further noted that this approach is used within the context of Appendices 30 and 30A by means of the grouping concept as defined in Articles 9 and 9A of Appendix 30A, Articles 10 and 11 of Appendix 30 and rules of procedure relating to § 4.1.1 a) and 4.1.1 b) of Appendices 30 and 30A.

3	In view of the above, the Board decided that the same grouping concept can also be applied within the context of §§ 6.5, 6.21 and 6.22. The Board’s understanding of the grouping concept is that in the interference calculation to entries (allotments or assignments) that are part of the group, only the interference contribution from entries that are not part of the same group are to be considered. On the other hand, for the interference calculation from entries belonging to a group into entries that are not part of the same group, only the worst interference contribution from that group is to be taken into consideration.

4	The Board did not find any regulatory basis to extend the use of groupings involving multiple orbital positions. However, grouping of networks in different orbital positions may be used before the inclusion of the assignments in the List to modify the orbital position of a network.

5	Interference between assignments to the “existing systems” as referred to in considering b) and c) of Resolution 148 (Rev.WRC-15) shall not be taken into consideration in single-entry calculation for consistent implementation of instructs the Radiocommunication Bureau 2 of that Resolution.

6	See also Note by the Secretariat relating to the “multi-beam networks” as indicated in column 10 of the tables in Article 10 of Appendix 30B.

6.6

Agreement of an administration whose territory is partially or wholly included in the service area of an assignment

The Board decided that the administrative agreements of the administrations whose territories are partially or wholly included in the intended service area of an assignment under examination are explicitly required and shall be obtained when entering the assignment in the List, irrespective of whether or not their allotments in the Plan or their assignments are identified as affected under § 6.5. If an identified administration does not make comment nor reply to the notifying administration’s request for seeking agreement under § 6.6, it shall be considered that the former administration disagrees to the inclusion of its territory in the intended service area of the assignment.

In the examination of a satellite network submitted under § 6.17, if the Bureau finds that the territory of an administration is wholly or partially included in the service area of the network without having obtained an explicit agreement from that administration prior to the submission under § 6.17, it shall request the notifying administration to exclude the territory and the associated test points from the service area. If the notifying administration insists on keeping the service area unchanged, the finding of the examination under § 6.19 a) shall be unfavourable.

An administration that agreed to include its territory in the service area of an assignment may at any time withdraw its agreement in accordance with § 6.16.

6.16

1	When the Bureau receives an objection to being included in a service area of an assignment from an administration in accordance with § 6.16 of Appendix 30B, the Bureau publishes the modified service area for exclusion of its territory from the service area if the assignment has already been included in the List. If the assignment is at the stage of coordination and not yet included in the List (i.e. published in an AP30B/A6A/-- Special Section only), the Bureau takes that objection into account in the examination under § 6.19 a) when the assignment is submitted by the notifying administration under § 6.17. The final characteristics of the assignment in the List (i.e. those published in an AP30B/A6B/-- Special Section) shall not include the territory and test points that are within the territory of the objecting administration in the service area.

2	However, an administration can object to the inclusion of its territory in the service area of an assignment of the other administrations not yet entered in the List and explicitly request that the objection should be taken into account in the examination of its own network submitted under § 6.17 of Appendix 30B in order to facilitate inclusion of the assignments of its own network to be included in the List. In this case, the objection should be considered definitive. The Bureau shall then, in accordance with § 6.16 of Appendix 30B, exclude the territory and test points that are within the territory of that objecting administration from the service area of the assignment objected and publish the modified service area in a modification to the corresponding AP30B/A6A/-- Special Section. The modification to the service area and the deletion of test points shall then be taken into account in the subsequent examinations including examinations under § 6.21 and 6.22 of Appendix 30B of the network submitted by the objecting administration under § 6.17 of Appendix 30B.

6.19 b)

See Rules of Procedure relating to § 6.3 a).

6.21

See Rules of Procedure relating to § 6.5.

6.25 to 6.29

Note: WRC-15 took the decision related to the provisional entry of converted assignment in RR Appendix 30B List during the 8th Plenary, Par. 1.39 to 1.42 of Doc. CMR15/505, approval of Doc. CMR15/416 in relation to Section 3.2.7.1 of Doc. 4 (Add2) (Rev1), as follows:

“In Section 3.2.7.1 of Doc. 4 (Add2) (Rev1), the Director sought confirmation by the conference of the following course of action:

When an assignment converted from an allotment of Appendix 30B Plan enters in the List provisionally, the initial allotment will not be suppressed from the Plan until the entry in the List of the assignment becomes definitive. When the converted assignment is reinstated, the notifying administration should choose either to keep its initial allotment in the Plan or reinstate with characteristics in the List to replace the initial allotment. In the latter case, the conditions described in § 6.26 to § 6.29 of Article 6 of Appendix 30B shall continue to be applied to the reinstated allotment (i.e. has the same status of the cancelled assignment).

WRC-15 considered and confirmed the course of action presented in this section.”

6.38			(ADD RRB24/510)

The Board understands that the Bureau conducts an examination under §§ 6.5, 6.21 and 6.22 of Article 6 of Appendix 30B to identify potentially affected allotments in the Plan and frequency assignments in the List based on their characteristics in the Plan and in the List. However, in application of § 6.38, the Bureau shall take into account, to the extent possible, the actual operational parameters, as provided by the administration of frequency assignments that have already been brought into use and recorded in the Master Register in its compatibility analysis. Such parameters may be different from the parameters of the corresponding frequency assignments in the List.

Upon receipt of a request for assistance from either the notifying administration applying § 6.37 or an administration identified under § 6.5 of Appendix 30B, the Board decided that the Bureau shall request the notifying administrations of satellite networks which are identified as affected to provide their actual operational parameters within 30 days. If there is no reply within the 30 days, the Bureau shall send a reminder giving an additional 15-day period to reply.

Upon receipt of the requested operational parameters, the Bureau shall perform the compatibility analysis using those parameters instead of the corresponding parameters of the affected satellite network in the List. The compatibility analysis under § 6.38 shall be performed based on the same principles as those established in application of § 6.21, including footnote 7bis to § 6.21 c), and the latest available Appendix 30B master database.2bis The Bureau shall inform both the notifying administration having requested the application of § 6.37 and the notifying administration of the affected satellite network of the results of its compatibility analysis.

The notifying administration of the affected satellite network should also be invited to make modifications to the characteristics of the frequency assignments recorded in the Master Register in order to align them with their actual operational parameters.

If the Bureau does not receive a reply within 15 days following the reminder, the Board concluded that the Bureau shall inform the administrations concerned that it is not in a position to perform the compatibility analysis under § 6.38.

6.39			(ADD RRB24/510)

1	This provision indicates to the Bureau how to generate the uplink satellite antenna gain diagram for a frequency assignment to an additional system not subject to Resolution 170 (Rev.WRC-23) or to a conversion of an allotment into a frequency assignment with modification outside the envelope of the allotment and not subject to Resolution 170 (Rev.WRC-23) during the examination of a submission under § 6.37. The first step to generate the diagram is to create the −10 dB contour of minimum ellipses for all territories inside each service area of the satellite network identified under § 6.5. A question arises as to which space station antenna pattern shall be used in application of § 6.39. The Board instructed the Bureau to use the Appendix 30B space station antenna co-polar pattern for receiving and transmitting antennas for all Regions without fast roll-off for creating the minimum ellipse covering a territory and the −10 dB contour of each individual minimum ellipse, as it is also used for the determination of coordination requirements and interference assessment in the FSS Plan. The co-polar pattern corresponds to the pattern code APSRR_401V01 in the Antenna Pattern Library maintained by the Bureau.

2	To ensure that there are enough test points to generate each minimum ellipse, the individual set of test points per national territory should be those contained in the national allotment plus the originally submitted test points associated with the service area and located inside that territory. If the total number of the test points for any territory in a service area is less than 20, the Bureau shall consult the notifying administration of the identified satellite network to find out whether it wishes to add more test points in that territory.

3	In creating the minimum ellipses, the Board decided that a rotation accuracy of 1.0° and a pointing error of 0.1° should be taken into account.

4	The test points taken from the national allotment or added during the application of § 6.39 are only for the purpose of generating the minimum ellipses and the combined ellipses and will not be used in technical examinations.

6.40			(ADD RRB24/510)

The Board understands that the “latter assignment” mentioned in the provision refers to a frequency assignment identified as potentially affected when examining the submission subject to § 6.37.

With respect to the condition for not updating the reference situation of a frequency assignment which is still identified as affected, it is not clear if “based on its submitted uplink coverage area” refers to the originally submitted coverage area (i.e. the one in the List) or to the coverage area that was submitted as an “actual operational parameter” in application of § 6.38. In addition, this provision does not give clear instruction on whether the reference situation of the “still affected” satellite network should be updated when the administrations concerned reach agreement under § 6.37bis. The Board thus instructed the Bureau, when a frequency assignment subject to § 6.37 is entered in the List, to consult both the notifying administration having requested the application of § 6.37 and the notifying administration of the affected satellite network and not to update the reference situation of the frequency assignments which are still identified as affected, based on the originally submitted coverage area, unless both parties agree to update the reference situation.

Art. 7

## Procedure for the addition of a new allotment to the Plan 
for a new Member State of the Union

7.3

New allotment to the Plan for a new Member State of the Union

1	Provision § 7.3 of Appendix 30B requests the Bureau to identify appropriate technical characteristics and associated orbital locations for a prospective national allotment upon receipt of a request from a new Member State.

The Bureau shall apply the procedures described below to find an appropriate orbital position for an allotment in the Appendix 30B Plan for a new Member State.

2	The Bureau shall ensure that all submitted test-points are located within the national territory of the new Member State. Test-point locations shall be verified using the ITU Digitized World Map. In addition, in the absence of a height above sea level, a value of zero metres shall be assumed by the Bureau.

3	In order to facilitate the implementation of the orbital position selection approach described in § 8 below, the new Member State may provide under § 7.2 c) of Article 7 of Appendix 30B its preferred orbital position(s) and/or its preferred orbital arc(s), bearing in mind that the implementation of these preferences might not be possible due to excesses of interference to or from other allotments or assignments of Appendix 30B.

4	The Bureau shall establish the required minimum elevation angles associated to each test-point in accordance with § 1.3 of Annex 1 to Appendix 30B. The service arc shall then be calculated in order to meet the required minimum elevation angles of all test-points.

5	With regard to the generation of the minimum ellipse to cover the national territory of the new Member State, the Bureau shall use a space station antenna beam pointing error of 0.1 for the generation of elliptical beams under Article 7 of Appendix 30B.

6	With regard to the transmitting and receiving space station antenna maximum gain values, as a function of the major and minor axes of the ellipse, instead of using the definition contained in § 1.7.2 of Annex 1 to Appendix 30B the Bureau shall use the more precise formula defined in § 3.13.1 of Annex 5 and § 3.7.1 of Annex 3 of Appendices 30 and 30A, respectively.

7	With regard to the calculation of the maximum power density values, the Bureau shall assume the worst-case conditions in terms of space station antenna pointing error and rotational accuracy for the calculation of the antenna gain in the direction of each test-point, in order to ensure that the objective C/N ratios defined in § 1.2 of Annex 1 to Appendix 30B are met for all test points, i.e., assume the minimum gain value of the antenna, taking into account a pointing error of 0.1 and a rotational accuracy of  1.0.

8	With regard to the selection of orbital position, the Bureau shall use an automated approach based on an iterative process as follows:

8.1	Once the service arc is calculated, as mentioned in § 4 above, an iterative process is implemented to identify suitable orbital position(s) within that arc for the allotment to the new Member State in question.

8.2	The Bureau shall assume a minimum orbital position step of 0.1 in this process.

8.3	Each new possible orbital position shall be examined by the Bureau as follows:

regenerate the elliptical beam parameters;

recalculate the required power density values to meet the C/N criteria of § 1.2 of Annex 1 to Appendix 30B;

using the methods and criteria contained in Appendices 1 and 2 to Attachment 1 to Resolution 170 (Rev.WRC-23), determine whether the new allotment at that orbital position is compatible with the allotments and the assignments as mentioned in § 7.5 of Article 7.     (MOD RRB24/510)

Note: The World Radiocommunication Conference (Dubai, 2023) (WRC-23) took a decision on the procedure under Article 7 of Appendix 30B at its 13th plenary meeting, see § 13.10 of Document CMR23/528, which reads as follows:

13.10	On issues related to the Article 7 procedure of Appendix 30B, it was proposed that the following text be approved and included in the minutes of the Plenary:

“WRC-23 urges administrations with Appendix 30B Part A submissions received before 12 March 2020 to make all efforts to accommodate Article 7 submissions of other administrations and to take into account the results of the analyses of the Bureau and the measures to avoid further degradation of the C/I level when preparing their Part B submissions.

WRC-23 instructed the Bureau to contact the additional seven countries (Eritrea, Estonia, Latvia, Saint Lucia, Tajikistan, Timor-Leste and Turkmenistan) and the State of Palestine, which still have no allotment in the Appendix 30B Plan, and to identify orbital resources should they wish to initiate the process under Article 7.”      (ADD RRB24/510)

9	The Bureau shall identify the most appropriate orbital position(s) with the aim to minimize the C/I excesses caused or received from other allotment(s), or assignment(s) of Appendix 30B and send this information to the requesting administration in accordance with § 7.3 of Article 7.

7.5 a)

See Rules of Procedure relating to § 6.3 a).

Art. 8

## Procedure for notification and recording in the Master Register 
of assignments in the planned bands for the 
fixed-satellite service

8.8

See Rules of Procedure relating to § 6.3 a).

8.16		(ADD RRB23/498)

§ 6.31bis of Appendix 30B specifies the course of action that shall be taken regarding the submission or updating of the Resolution 49 information when the regulatory time-limit for bringing into use frequency assignments is extended due to launch failure.

However, when the Board decides to grant an extension of the regulatory time-limit for bringing into use frequency assignments in cases of force majeure or co-passenger delay, this also raises the question of whether the deadline for the submission of Resolution 49 (Rev.WRC-19) and notification information should be extended.

Noting that a similar question related to non-planned services is addressed in the rule of procedure concerning Nos. 11.48 and 11.48.1, the Board decided that the rule of procedure concerning Nos. 11.48 and 11.48.1 of the Radio Regulations shall also apply to the extension of bringing into use of frequency assignments subject to Appendix 30B with the understanding that the regulatory period for bringing into use frequency assignments to a satellite network subject to this Appendix is eight years.

An. 3 and 
An. 4

1	WRC-07 revised Appendix 30B and introduced power flux-density limits in Annex 3 of Appendix 30B in order to protect FSS allotments and assignments from interference which may be caused by FSS assignments located outside the orbital arcs defined in Annex 4. Although the reference bandwidth of these limits in Annex 3 is 1 MHz, the maximum power densities which are used for the calculation of power flux-densities are submitted in dB(W/Hz) averaged over the necessary bandwidth (C.8.h) and 4 kHz (C.8.b.2) in accordance with Appendix 4. The discrepancy between the reference bandwidth for the limits and the averaging bandwidth for submission might lead to the overestimation of interference when a few narrow-band carriers are used, e.g. carriers for tracking, telemetry and telecommand. On the other hand, a narrow-band carrier might cause significant interference to other narrow-band carriers if these carriers are accidentally overlapping with each other.

2	In order to avoid the overestimation of interference from narrow-band carriers into wideband carriers caused by integrating the power of narrow-band carriers from 1 Hz to 1 MHz and to provide a mechanism to resolve unexpected interference between narrow-band carriers, the Board decided on the following course of action.

2.1	In the case when:

a)	the maximum power density, in dB(W/Hz), averaged over the worst 1 MHz band, supplied to the input of the antenna taking into account the number of carriers and power level of each carrier to be operated within the averaging bandwidth of 1 MHz;

is lower than;

b)	the maximum power density, in dB(W/Hz), averaged over the necessary bandwidth (C.8.h);

2.2	the power density value as described in 2.1 a) above shall be provided by a notifying administration together with the relevant Appendix 4 information;

2.3	the Bureau shall use the submitted power density value as described in 2.1 a) above for its examination under Annexes 3 and 4 and publish it in the relevant Special Section;

2.4	those operating assignments whose power density value as described in 2.1 b) is higher than that in 2.1 a) shall not cause harmful interference to, or claim protection from, prior assignments recorded in the MIFR.

Note: WRC-19 took the decision related to Annexes 3 and 4 of Appendix 30B, during the 10th Plenary, see items 13.7 to 13.9 of Doc. CMR19/571, approval of Doc. CMR19/510 (see also the Rules of Procedure on Resolution 170 (WRC-19)), as follows:

“Instructions to the Radiocommunication Bureau in application of Annex 3 and Annex 4 of RR Appendix 30B as well as of criteria referred to in Resolution [A7(E)-AP30B] (WRC-19) in its processing, after 22 November 2019, of submissions received under that Appendix

The Radiocommunication Bureau shall continue to calculate and update already accepted single-entry values in both uplink and downlink for all RR Appendix 30B satellite networks in consistency with footnotes X2 and X3 to item 2.1 of the Annex 4 of RR Appendix 30B (Rev.WRC-19), so that this information could be used by administrations during coordination of their respective networks. The Radiocommunication Bureau shall apply:

1	For complete submissions under § 6.1 received by the Bureau before 23 November 2019:

a)	Annex 3 (WRC-07) in its examination under § 6.3 b);

b)	Annex 4 (Rev.WRC-07) in its examination under § 6.5.

Note: Including protection of submissions under Issue E examined before Part A.

2	For complete submissions under § 6.17 received by the Bureau before 23 November 2019:

a)	Annex 3 (WRC-07) in its examination under § 6.19 c);

b)	Annex 4 (Rev.WRC-07) in its examination under § 6.21;

c)	Annex 4 (Rev.WRC-07) in its further examination under the new footnote to § 6.21 c);

d)	Annex 4 (Rev.WRC-07) in its examination under § 6.22.

Note: Including protection of submissions under Issue E examined before Part B.

3	For complete submissions under § 6.17 received by the Bureau after 22 November 2019, related to complete submissions under § 6.1 received by the Bureau before 23 November 2019:

a)	Annex 3 (WRC-07) in its examination under § 6.19 c);

b)	Annex 4 (Rev.WRC-07) in its examination under § 6.21;

c)	Annex 4 (Rev.WRC-07) in its further examination under footnote YY to § 6.21 c) if the remaining affected assignments are recorded in the List before 23 November 2019;

d)	Annex 4 (Rev.WRC-19) in its further examination under footnote YY to § 6.21 c) if the remaining affected assignments are recorded in the List after 22 November 2019;

e)	Annex 4 (Rev.WRC-19) in its examination under § 6.22.

Note: Including protection of submissions under Issue E examined before Parts A and/or B.

4	For complete submissions under § 6.1 received by the Bureau after 22 November 2019:

a)	Annex 3 (Rev.WRC-19) in its examination under § 6.3 b);

b)	Annex 4 (Rev.WRC-19) in its examination under § 6.5.

5	For complete submissions under § 6.17 received by the Bureau after 22 November 2019, related to complete submissions under § 6.1 received by the Bureau after 22 November 2019:

a)	Annex 3 (Rev.WRC-19) in its examination under § 6.19 c);

b)	Annex 4 (Rev.WRC-19) in its examination under § 6.21;

c)	Annex 4 (Rev.WRC-19) in its examination under § 6.22.

6	For complete submissions under § 6.1 in application of Resolution [A7(E)-AP30B] (WRC-19):

a)	Annex 3 (Rev.WRC-19) in its examination under § 6.3 b);

b)	Annex 4 (Rev.WRC-19) and the new criteria referred to in Resolution [A7(E)-AP30B] (WRC-19) in its examination under § 6.5, as appropriate.

Note: Including examination of submissions under Issue E before the examination of the last normal Part A and/or Part B received before 23 November 2019.

7	For complete submissions under § 6.17 in application of Resolution [A7(E)-AP30B] (WRC-19), the Bureau shall apply:

a)	Annex 3 (Rev.WRC-19) in its examination under § 6.19 c);

b)	Annex 4 (Rev.WRC-19) and the new criteria referred to in Resolution [A7(E)-AP30B] (WRC-19) in its examination under § 6.21, as appropriate;

c)	Annex 4 (Rev.WRC-19) and the new criteria referred to in Resolution [A7(E)-AP30B] (WRC-19) in its further examination under footnote YY to § 6.21 c), as appropriate;

d)	Annex 4 (Rev.WRC-19) and the new criteria referred to in Resolution [A7(E)-AP30B] (WRC-19) in its examination under § 6.22, as appropriate.

Application of § 6.16:

–	In excluding the territories of the concerned administration, the Bureau shall apply Annex 4 (Rev.WRC-07) until the last complete submissions under § 6.1 or § 6.17 received by the Bureau before 23 November 2019 has been examined and Annex 4 (Rev.WRC-19) afterward.

–	If § 6.16 request is submitted in order to be taken into account for the examination of a complete submissions under § 6.17, in examining those submissions, the Bureau shall apply appropriate Annex 4 used in the examination under § 6.21 and § 6.22 as indicated above.

Application of § 6.27 in updating criteria:

The Bureau shall apply Annex 4 (Rev.WRC-07) until the last complete submissions under § 6.1 or § 6.17 received by the Bureau before 23 November 2019 has been examined and Annex 4 (Rev.WRC-19) afterward.

Application of § 7.5:

–	For a request under Article 7 received before 23 November 2019, the Bureau shall apply Annex 3 (WRC-07) and Annex 4 (Rev.WRC-07).

–	For a request under Article 7 received after 22 November 2019, the Bureau shall apply Annex 3 (Rev.WRC-19) and Annex 4 (Rev.WRC-19).

In its examination under 6.21 c), the Bureau shall take into account also complete submissions under § 6.1 in application of Resolution [A7(E)-AP30B] (WRC-19) and Article 7 request transferred to Article 6 under § 7.7 that has been examined before the date of receipt of the examined notice submitted under § 6.1.”*

* Note by the Secretariat: The definitive number of Resolution [A7(E)-AP30B] WRC-19 (WRC-19)] is Resolution 170 (WRC-19).  Furthermore, the definitive numbers of footnotes X1, X2 and YY in Appendix 30B are 17bis, 20bis and 7bis respectively.  Finally, “submissions under issue E” referred to submissions under the special procedure described in Attachment 1 to Resolution 170 (WRC-19).

Annex 4

Criteria for determining whether an allotment or
an assignment is considered to be affected

2.1

1	In order to adequately protect the existing networks in their entire downlink service area, an examination based on a single-entry criterion over the downlink service area was introduced under § 2.1 of Annex 4 of Appendix 30B.

2	As indicated in footnote 19 to § 2.1 of Annex 4 of Appendix 30B (Rev.WRC-19), the reference values within the downlink service area are interpolated from the reference values on the corresponding test points. The following interpolation formula and condition shall be used to calculate the interpolated values at grid points4 within the downlink service area:

|  |  | (1) |
| --- | --- | --- |

where:

Th: 	test point number h of the wanted downlink service area;

Eg:	point number g of the grid of examination points on the wanted downlink service area;

Nt:	total number of test points;

dTh:	distance between the test point Th and the grid point Eg;

RTh:	single entry C/I reference value (dB) at the test point Th (i.e. 26.65 dB, or (C/N)d + 11.65 dB, whichever is the lowest);

VEg: 	interpolated single entry C/I reference value (dB) at the grid point Eg.

If the value (RTh –((C/N)d,Th – (C/N)d,Eg)) is lower than  RTh, then (RTh –((C/N) d,Th – (C/N) d,Eg)) shall be used in (1) instead of  RTh,

where：

(C/N)d,Th: 	the downlink C/N value at test point Th;

(C/N)d,Eg:	the downlink C/N value at grid point Eg.

3	If the interpolated value VEg is higher than (C/N)d, Eg +11.65 dB, (C/N)d, Eg +11.65 dB shall be used as the reference value for grid point Eg. Otherwise, the interpolated value is the reference value.

4	Footnote 10 to § 2.1 of Appendix 1 to Attachment 1 to Resolution 170 (WRC-19) refers to the same interpolation method as above. Therefore, when applying § 2.1 of Appendix 1 to Attachment 1 to Resolution 170 (WRC-19), the method contained in §§ 2 and 3 above shall be used to calculate the interpolated values at grid points within the downlink service area with the following modifications:

RTh shall be defined as the single entry C/I reference value (dB) at the test point Th (i.e. 23.65 dB, or (C/N)d + 8.65 dB, or any already accepted value, whichever is the lowest);

a value of (C/N)d, Eg +8.65 dB shall be used instead of (C/N)d, Eg +11.65 dB.

2.2

Note: WRC-19 took the decision related to grid points and test points at sea, during the 8th Plenary, see items 3.11 to 3.15 of Doc. CMR19/569, approval of Doc. CMR19/451, in relation to section 3.2.5.6 of Doc. CMR19/4 (Add.2), as follows:

“In considering section 3.2.5.6 on ‘Grid points at sea in the examination using the methods of Annex 4 of RR Appendix 30B’, WRC-19 decided that only grid points that are located on land and inside the service area should be considered in addition to test-points in application of paragraph 2.2 of Annex 4 to Appendix 30B. In taking this decision WRC-19 acknowledged that, should the use of Appendix 30B expand beyond its current use, it may be necessary to reconsider this decision in the future. WRC-19 also decided that test-points at sea shall not be taken into account by the Radiocommunication Bureau in its technical and regulatory examination of the relevant submissions received by the Bureau.”

Annex 7			(ADD RRB24/510)

Measurements in order to facilitate the addition of a new allotment 
to the Plan for a new Member State of the Union

§ 5 a)

§ 5 states that “should the power density of the proposed new allotment be limited to a single minimum value… meeting the carrier-to-noise (C/N) objectives and an overall aggregate carrier-to-interference value of 21 dB, as indicated in Annex 1 to Appendix 30B,…” additional measures including § 5 a) shall be applied.

The Board noted that, when applying § 7.3 of Article 7 of Appendix 30B for identifying technical characteristics of possible new allotments, the power density values are calculated based on the C/N criteria of § 1.2 of Annex 1 of Appendix 30B without consideration of aggregate C/I values.

However, when the requesting administration selects the characteristics of the new allotment from those proposed by the Bureau, it may request the Bureau to increase the power density values of the selected new allotment if any of its aggregate C/I values are less than 21 dB. The Board instructed the Bureau to then recalculate the power density values for the new allotment in order to meet the 21-dB aggregate C/I target, taking into account any appropriate limitation contained in Articles 21 and 22 and Annex 3 to Appendix 30B.

§ 5 b)

This provision indicates that, in examining a proposed new allotment, §§ 5 a) and 5 d) of Annex 7 shall not be applied to frequency assignments already entered in the List; however, it does not mention which criteria  should be used in that case instead of those under §5 a).

The Board instructed the Bureau that, with respect to frequency assignments entered in the List on or before the date of receipt of the proposed new allotment under examination:

–	a frequency assignment shall be considered as being affected if either one of the single-entry carrier-to-interference ratios ((C/I)d and (C/I)u) or the overall aggregate carrier-to-interference ratio ((C/I)agg), is not equal to or greater than its associated value specified in§ 2.1 of Annex 4 to Appendix 30B; and

–	if the frequency assignment in the List is identified as affected, the proposed new allotment should not be taken into account when updating the reference situation of that frequency assignment, when the proposed new allotment is entered in the List and/or the Plan.

____________________

_______________

1	See Rules of Procedure relating to No. 21.11.

2	See Rules of Procedure relating to No. 21.14.

2bis   The administrations concerned may request the Bureau to use a different master database.

4	The service area is regularly covered by a grid of points located on land and inside the service area.