(ADD RRB24/510)

# Rules concerning

## RESOLUTION  8 (WRC-23)

# Tolerances for certain orbital characteristics of space stations deployed 
as part of non-geostationary-satellite orbit systems in the fixed-satellite,
broadcasting-satellite or mobile-satellite service

1	When the modification of a frequency assignment subject to Section II of Article 9 is submitted in application of resolves 9 of Resolution 8 (WRC-23), it shall be examined under No. 11.43A to determine whether the coordination requirements remained unchanged following the procedure indicated in § 2 of the rules of procedure on No. 11.43A. If, due to the modifications, new coordination requirements are identified for the frequency assignments that are subject to Resolution 35 (Rev.WRC-23) and Resolution 8 (WRC-23)1 and have space stations whose deviations in altitude or inclination were the basis for the modifications, the Board concluded that such frequency assignments shall receive an unfavourable finding and be returned to the notifying administration.

2	When applying resolves 9 and in order to demonstrate a non-increase in interference and subsequent non-increase in coordination requirements following the method contained in § 2 of the rule of procedure on No. 11.43A and in the absence of appropriate criteria or calculation methods, the Board decided that the notifying administration may provide technical justifications based on dynamic interference assessments in the form of a cumulative distribution function of the interference level, expressed as an interference-to-noise (I/N) ratio, for various locations and percentages of time, created in the subsequently filed non-geostationary-satellite orbit (non-GSO) systems or geostationary-satellite orbit (GSO) networks by the existing non-GSO system based on its initial characteristics and modified characteristics, respectively. The Bureau shall thoroughly study the technical justifications provided by the notifying administration in order to make its findings under No. 11.43B.

3	The Board noted that resolves 16 of Resolution 8 (WRC-23) limits the modifications to be submitted under that resolves to any sub-items of Appendix 4 data item A.4.b.4, except data item A.4.b.4.b (i.e. the number of satellites in the orbital plane), and any sub-items of data items A.14, A.4.b.6.a and A.4.b.7. Modifications involving the change of Appendix 4 data item A.4.b.4.b (i.e. a decrease in the number of satellites in the orbital plane) shall be submitted under resolves 11 c) of Resolution 35 (Rev.WRC-23).

However, noting the condition for a favourable finding under No. 11.43B described in resolves 14 c) ii) of Resolution 35 (Rev.WRC-23)2, the Board decided that a modification submitted under resolves 10 of Resolution 8 (WRC-23) involving the change of data item A.4.b.4.b will be treated as complying with the condition in resolves 16 c) ii), provided that the notifying administration indicates that the modification is submitted in simultaneous application of resolves 11 of Resolution 35 (Rev.WRC-23) and resolves 10 of Resolution 8 (WRC-23). Similarly, such a modification may be treated as complying with the condition in resolves 14 c) ii) of Resolution 35 (Rev.WRC-23) if it involves changes to any sub-item of data item A.4.b that is not listed in resolves 14 c) ii) of Resolution 35 (Rev.WRC-23) when such modification is associated with the application of resolves 10 of Resolution 8 (WRC-23).

If, due to modifications, any of the conditions contained in resolves 16 c) i), 16 c) ii) or 16 c) iii) are not met, except when only the condition under resolves 16 c) ii) is not met because the number of satellites in data item A.4.b.4.b is reduced as a result of a simultaneous application of resolves 11 of Resolution 35 (Rev.WRC-23) and resolves 10 of Resolution 8 (WRC-23), the Board further decided that all modified frequency assignments subject to Resolution 35 (Rev.WRC-23) shall receive an unfavourable finding and shall be returned to the notifying administration.

4	The notifying administration will be invited to apply Section II of Article 9 for all frequency assignments receiving unfavourable findings under §§ 1 and 3 above.

____________________

_______________

1 	The frequency assignments subject to Resolution 35 (Rev.WRC-23) are those frequency assignments of non-GSO systems in the frequency bands and in the services listed in the table under resolves 1 of Resolution 35 (Rev.WRC-23).

2 	The modifications are limited to the reduction of the number of orbital planes (Appendix 4 data item A.4.b.2) and modifications to the longitude of the ascending node (Appendix 4 data item A.4.b.4.j) associated with the remaining orbital planes, or reduction of the number of space stations per plane (Appendix 4 data item A.4.b.4.b) and modifications of the initial phase angle of the space stations (Appendix 4 data item A.4.b.4.h) within planes.