# Rules concerning

## RESOLUTION  123 (WRC-23)

# Use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz and
19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and
29.5-30 GHz (Earth-to-space) by aeronautical and
maritime earth stations in motion communicating 
with non-geostationary space stations 
in the fixed-satellite service

1	The Board noted that resolves 2 of Resolution 123 (WRC-23) requires that the characteristics of earth stations in motion (ESIMs) shall remain within the envelope characteristics, including any applicable coordination agreement, of typical earth stations associated with the non-geostationary satellite orbit (non-GSO) system in the fixed-satellite service (FSS) with which ESIMs communicate.

1.1	For applying resolves 2, the Board decided that the Bureau shall identify whether the ESIM characteristics are within the envelope characteristics of typical earth stations associated with the satellite system with which those aeronautical and/or maritime ESIMs communicate by using the method contained in § 2.3 of the Rule of Procedure on No. 9.27. In cases where such examination indicates that coordination requirements of the frequency assignments of aeronautical and/or maritime ESIMs involve any additional satellite network or system, the frequency assignments to the aeronautical and/or maritime ESIMs will be returned to the notifying administration together with an unfavourable finding under No. 11.32. The results of the Bureau’s examination shall be published in its International Frequency Information Circular (BR IFIC).

1.2	In addition to the procedure indicated in § 1.1 above, in cases where ESIMs operate in the frequency bands 27.5-28.6 GHz and 29.5-30 GHz (Earth-to-space), the Board concluded that the minimum elevation angle submitted for ESIMs (see Appendix 4 data item A.36.a) shall be greater than or equal to the minimum elevation angle submitted for the associated group of frequency assignments to the non-GSO FSS system (see Appendix 4 data item A.4.b.7.cbis) in order to ensure that ESIMs comply with epfd limits set out in No. 22.5D.

The Board noted that, for aeronautical ESIMs, the reference minimum elevation angle value of typical earth stations of the associated non-GSO FSS system, when adjusted to an altitude of 15 km, will be greater than the one corresponding to an altitude of 0 km, provided that the same viewing angle is maintained from the non-GSO space station.

2	The Board noted that resolves 3.5 of Resolution 123 (WRC-23) requires that, with respect to the protection of the Earth exploration-satellite (passive) service operating in the frequency band 18.6-18.8 GHz, any non-GSO FSS system with an orbital apogee of less than 20 000 km operating in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz with which aeronautical and/or maritime ESIMs communicate and for which the complete notification information has been received by the Bureau after 1 January 2025 shall comply with the provisions indicated in Annex 3 to the Resolution. Given the fact that Resolution 123 (WRC-23) entered into force on 1 January 2025, the Board concluded that the provision applies to any non-GSO FSS system with an orbital apogee of less than 20 000 km operating in the frequency bands 18.3-18.6 GHz and 18.8-19.1 GHz with which aeronautical and/or maritime ESIMs communicate and for which the notification information has been received by the Bureau as of, rather than only after, 1 January 2025.

3	In addition, the Board concluded that the Bureau shall examine the characteristics of aeronautical ESIMs with respect to conformity with the pfd limits at the Earth’s surface specified in Part II of Annex 1 to Resolution 123 (WRC-23) by using the methodology contained in the rule of procedure on calculation of power-flux density levels produced by aeronautical ESIMs and their validation with the limits in Annex 3 to Resolution 169 (Rev.WRC-23), Annex 2 to Resolution 121 (WRC-23) and Annex 2 to Resolution 123 (WRC-23). The findings shall be in accordance with No. 11.31.

4	With respect to provisions contained in Part 1 of Annex 1 and in Annex 3 to Resolution 123 (WRC-23), the Board further concluded that no examination shall be carried out by the Bureau. The notifying administration of the non-GSO FSS system with which the ESIMs communicate shall ensure compliance with those provisions when providing the commitment required under item A.34.a of Annex 2 to Appendix 4 to operate ESIMS in conformity with the Radio Regulations and Resolution 123 (WRC-23).

____________________

_______________