# Rules concerning

## RESOLUTION 559 (WRC-19)

# Additional temporary regulatory measures following the deletion 
of part of Annex 7 to Appendix 30 (Rev.WRC-15) by WRC-19

Note: WRC-23 took the following decision on issues related to the implementation of Resolution 559 (WRC-19), see item 13.2 of the Minutes of the 13th Plenary meeting, Doc. CMR23/528:

“In considering section 4.2 of the Report “Issues related to the implementation of Resolution 559 (WRC-19)”, WRC-23 considered also Document 87(Add.26)(Add.2). In addition to endorsing all of the additional measures proposed by the Board for implementing Resolution 559 (WRC-19), this document contained additional proposed measures to be endorsed by this WRC to help resolve remaining coordination cases as follows:

1	With respect to the remaining coordination cases under § 4.1.1 b) of RR Appendix 30, WRC-23 approved the following measures:

a)	the notifying administration of an additional use (i.e. assignments in the List and/or pending Article 4 networks) to accept possible interference produced to its test-points located within −3 dB antenna gain contour of the Resolution 559 (WRC-19) submission concerned due to the fact that the ellipse is already the minimum one validated by the Bureau;

b)	the notifying administration of an additional use (i.e. assignments in the List and/or pending Article 4 networks) to accept possible interference produced to its test-points located beyond −20 dB antenna gain contour of the Resolution 559 (WRC-19) submission concerned;

c) 	if the equivalent protection margin (EPM) of a test-point of an additional use network is less than −10 dB at the time of examination by the Bureau of Part A of Resolution 559 (WRC-19) submissions, that test-point should not be considered by the Bureau in reviewing the findings of the Resolution 559 (WRC-19) submission concerned;

d) 	a coordination is deemed to be completed if the nominal orbital separation between a Resolution 559 (WRC-19) submission and an additional use network is equal to or greater than 6 degrees.

2	With respect to the remaining coordination cases under § 4.1.1 e) of RR Appendix 30, WRC-23 approved the following measures:

a)	a coordination is deemed to be completed if the nominal orbital separation between a Resolution 559 (WRC-19) submission and satellite network in non-planned bands concerned is equal to or greater than 6 degrees;

b)	the service area of a satellite network in non-planned bands to be considered shall be on land and located within −3 dB antenna gain contour of that satellite network in non-planned bands instead of the submitted service area which may include the area with very low relative antenna gain contour. It is noted that the satellite network in non-planned bands only protects a Resolution 559 (WRC-19) submission in a service area on land and situated within its −3 dB antenna gain contour;

c)	if an administration agrees not to protect the area, situated inside its national territory, in which the power flux-density (pfd) limit is exceeded, that part of the service area shall not be considered by the Bureau in reviewing the remaining coordination requirements of a Resolution 559 (WRC-19) submission;

d)	the notifying administration of a satellite network in non-planned bands to accept possible interference produced to its service area located beyond −20 dB antenna gains contour of the Resolution 559 (WRC-19) submission concerned.

3	With respect to the remaining coordination cases under No. 4.1.1 b) of RR Appendix 30A, WRC-23 approved that the remaining coordination cases are deemed to be completed due to the fact that:

a)	the Article 4 satellite networks have very large coverage with very high receiving sensitivity over the national territory of the Resolution 559 (WRC-19) administration concerned;

b)	the coverage areas of those Article 4 satellite networks extend far beyond the national territory of the notifying administrations whereas feeder-link earth stations of the Resolution 559 (WRC-19) submission concerned are only located inside the national territory and that cannot be further reduced;

c)	the objective of Resolution 2 (Rev.WRC-03) and Topic F of WRC-23 Agenda Item 7.

4	With respect to the remaining coordination cases under RR No. 4.1.1 a) of Appendices 30 and 30A, WRC-23 approved the following measures:

a)	for multi-beam Plan assignments, if downlink single-entry C/I values are above 21 dB except for one test-point where single-entry C/I is greater than 18 dB, Resolution 559 (WRC-19) submissions and the corresponding Regions 1 and 3 Plan frequency assignments are considered compatible. In order to preserve the same level of protection for such compatible cases of those Regions 1 and 3 Plan frequency assignments from incoming Article 4 submissions, the reference situation of those Regions 1 and 3 Plan frequency assignments shall not be updated when the Resolution 559 (WRC-19) frequency assignments in the List are included in the Plans;

b)	for multi-beam Plan assignments, if feeder-link single-entry C/I values are above 27 dB, Resolution 559 (WRC-19) submissions and the corresponding Regions 1 and 3 Plan frequency assignments are considered compatible. In order to preserve the same level of protection for such compatible cases of those Regions 1 and 3 Plan frequency assignments from incoming Article 4 submissions, the reference situation of those Regions 1 and 3 Plan frequency assignments shall not be updated when the Resolution 559 (WRC-19) frequency assignments in the List are included in the Plans.

5	The Bureau is instructed to:

a)	review the status of all the remaining coordination cases taking into account all the above-mentioned proposals including those of the RRB and BR. In this connection, for the remaining coordination cases under RR No. 4.1.1 b) of Appendix 30, if after taking into account all the above-mentioned proposals, there is only one remaining test-point potentially affected, the coordination is deemed to be completed in respect of affected assignments entered in the List on or after 1 January 2017;

b)	apply all the measures endorsed by WRC-23 to the Resolution 559 (WRC-19) submissions of the Administrations of AFG, GNE, MLT and SEY and to the future applications of § RR Nos. 4.1.26 or 4.1.27 of Article 4 of Appendices 30 and 30A, which have the same nature as Resolution 559 (WRC-19).”

____________________

_______________