# PART  A6

# Rules concerning the Regional Agreement relating to the planning
of VHF/UHF television broadcasting in the African
Broadcasting Area and neighbouring countries
(Geneva, 1989) (GE89)

# 1	Scope of the Agreement

1.1	Following the revision of the GE89 Agreement, carried out in 2006 by the RRC-06-Rev.GE89, and in accordance with the Table of Frequency Allocations contained in Article 5 of the Radio Regulations (RR) (Edition of 2004), the GE89 Agreement governs, as from 17 June 2006, the use of the frequency band 47–68 MHz, by the television broadcasting service and by other primary terrestrial services which have allocations in this band (see also § 4 hereunder), within the planning area of this Agreement (the African Broadcasting Area as defined in Nos. 5.10 to 5.13 of the RR (Edition of 2004) and the following neighbouring countries: Saudi Arabia, Bahrain, United Arab Emirates, Iran (Islamic Republic of), Iraq, Kuwait, Oman, Qatar, Yemen (including those parts of Yemen that are situated outside of the African Broadcasting Area)).

1.2	The Plan annexed to the GE89 Agreement also contains those frequency assignments to television broadcasting stations, in the bands 230-238 MHz and 246-254 MHz, from the Member States listed in No. 5.252 of the RR, for which the procedure No. 9.21 of the RR was successfully completed.

# 2	Receivability of notices

In the application of the Regional Agreement relating to the planning of VHF/UHF television broadcasting in the African Broadcasting Area and neighbouring countries (Geneva, 1989), the Bureau will apply the procedures contained in Articles 4 and 5 of the Agreement and associated technical criteria with respect to the notices from all administrations having territories in the planning area (i.e. all administrations having territories within the African Broadcasting Area as defined in Nos. 5.10 to 5.13 of the RR and those administrations neighbouring the African Broadcasting Area as listed in § 1.8 of Article 1 of the GE89 Agreement), provided that the station concerned is situated within the planning area.

# 3	Execution of the Agreement

When a notice is received for a modification under Article 4 of the Agreement, the relevant coordination distances shall equally be applied to analogue and digital systems. An appropriate symbol shall be used to identify the television standard. Calculations requested in application of § 4.3.8 and 4.3.13 of the Agreement shall, where possible, be made using the most recent ITU-R Recommendation.

# 4	Examination of notices related to the non-planned services in the bands governed by the Regional Agreement GE89

4.1	Section 5.2 of Article 5 of the GE89 Agreement specifies the procedure to be followed for the examination of the notices related to the non-planned primary services in the bands governed by the Agreement. The bands and the services concerned are summarized in the Table below.

TABLE

| Frequency band (MHz) | Services and countries within the planning area |  | Provisions | Notes |
| --- | --- | --- | --- | --- |
| 47-68 | FIXED: | AFS, AGL, BOT, CME, COD, COG, IRN, LSO, MDG, MLI, MOZ, MWI, NGR, NMB, RRW, SOM, SDN, SSD, SWZ, TCD, TZA, ZMB, ZWE | 5.165 5.167 5.171 | 1 |
|  | MOBILE (except AER): | AFS, AGL, BOT, CME, COD, COG, LSO, MDG, MLI, MOZ, MWI, NGR, NMB, RRW, SOM, SDN, SSD, SWZ, TCD, TZA, ZMB, ZWE | 5.165 5.171 | 1 |
|  | MOBILE: | IRN | 5.167 |  |
| 230-238 | FIXED: | from all parties to the Agreement (excepting those referred to in No. 5.252) |  | 2 |
|  | MOBILE: | from all parties to the Agreement (excepting those referred to in No. 5.252) |  | 2 |
|  | AERONAUTICAL RADIONAVIGATION: | ARS, BHR, IRN, OMA, QAT, UAE | 5.247 | 3 |
| 246-254 | FIXED: | from all parties to the Agreement (excepting those referred to in No. 5.252) |  | 2 |
|  | MOBILE: | from all parties to the Agreement (excepting those referred to in No. 5.252) |  |  |
| NOTE 1 – The additional allocations to countries referred to in No. 5.171 are limited to the band 54-68 MHz. NOTE 2 – In the frequency bands 230-238 MHz and 246-254 MHz, in the examinations under § 5.2 of the Agreement, account is taken of only those frequency assignments in the broadcasting service which are entered into the Plan following a successful application of the procedure referred to in No. 9.21, as required by Resolution 1 (GE89) and No. 5.252. NOTE 3 – As the additional allocation to countries referred to in No. 5.247 is limited to the band 223-235 MHz, the procedure of § 5.2 of Article 5 of the GE89 Agreement applies in the band 230-235 MHz only. |  |  |  |  |

4.2	The frequency assignment notices related to the aeronautical radionavigation service of Nigeria, whose allocation is governed by No. 5.251, shall not be subject to the examinations referred to in § 5.2 of Article 5 of the Agreement, since these notices are subject to the application of the procedure of No. 9.21.

4.3	The frequency assignment notices related to land mobile service from countries referred to in No. 5.164 shall not be subject to the examinations requested by § 5.2 of Article 5 of the Agreement, since their allocation is subject to not causing harmful interference to, or claiming protection from, the broadcasting service. Consequently they will be recorded in the Master Register under the conditions of No. 5.43 vis-à-vis the broadcasting service (Symbol R in Column 13B2).

____________________

_______________