# PART  B

## SECTION  B4

# Rules concerning calculation methodology and technical standards
for determining the affected administrations and for assessing
the probability of harmful interference in the bands
between 9 kHz and 28 000 kHz

# Introduction

This Section contains elements of the calculation methodology which is to be used:

–	for identification of administrations whose agreement has to be sought in the application of No. 9.21, in the context of the relevant frequency allocation footnotes, i.e. Nos. 5.61, 5.87A, 5.92, 5.93 and 5.123, in the bands between 9 kHz and 28 000 kHz;

–	for assessing the probability of harmful interference, in the bands between 9 kHz and 28 000 kHz, as may be required in the application of the provisions of No. 7.6, or in any other Radiocommunication Bureau’s study as may be requested.

# 1	Technical Standard A-1:  Signal/interference protection ratio

1.1	The present Technical Standard contains the signal-to-interference protection ratio values (see Table 1) for application in the technical examinations of notices of frequency assignments in the frequency bands between 9 kHz and 28 000 kHz.

1.2	These protection ratio values are based on results of the studies within Radiocommunication Study Groups (see Recommendations ITU-R F.240-7, ITU-R SM.326-6, ITU-R-F.339-7 and former Recommendation ITU-R SM.669-1).

1.3	The signal-to-interference protection ratio (PR) values are expressed in dB, for the main types of transmission (from telegraphy, aural reception to telephony, for connection to the public network) to be protected in the technical examinations in the frequency bands from 9 kHz to 28 000 kHz. These values of protection ratios have been determined from RF steady state protection ratio values by adding allowances for long-term intensity fluctuation and short period fading for a given time percentage corresponding to the performance quality criteria applicable to each type of transmission.

1.4	For the calculation of the signal/interference ratios both the wanted and the interference field strength are considered as median values (exceeded 50% of the time) and on the basis of peak envelope power (p.e.p.; notified power type: PX). Types of power other than p.e.p. (notified as PY or PZ for, respectively, mean or carrier power) are converted to p.e.p. using conversion factors given in Table 2.

TABLE  1

RF signal-to-interference protection ratios (dB)

| Transmission type |  | Frequency band (kHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 9-1 606.5 | 1 606.5-4 000 | 4 000-28 000 |
| Telegraphy, aural reception |  | 8 (3-7) | 11 (5-10) | 15 (7-14) |
| Telegraphy, aural reception; Meteo, Press |  | 9 (3-8) | 13 (5-12) | 17 (7-16) |
| Telegraphy, automatic reception, without error correction |  | 11 (6-10) | 17 (10-16) | 26 (13-25) |
| Telegraphy, automatic reception, with error correction |  | 8 (6-7) | 12 (7-11) | 14 (8-13) |
| Photo telegraphy, facsimile |  | 19 (14-18) | 24 (16-23) | 28 (18-27) |
| Telephony, not for connection | DSB and SSB full carrier | 18 (15-17) | 21 (17-20) | 24 (19-23) |
| to public network (CO) | SSB, reduced or suppressed carrier, ISB | 12 (9-11) | 15 (11-14) | 18 (13-17) |
| Telephony, for connection | DSB and SSB full carrier | 31 (26-30) | 34 (28-33) | 38 (30-37) |
| to public network (CP) | SSB, reduced or suppressed carrier, ISB | 25 (20-24) | 28 (22-27) | 32 (24-31) |
| Broadcasting (except for HFBC in exclusive bands and MFBC in the band 526.5-1 705 kHz) |  | 38 (33-37) | 38 (32-37) | 38 (32-37) |
| Aeronautical mobile service (telegraphy or telephony) |  | 15 | 15 | 15 |
| Radiobeacons |  | 15 | 15 | – |
| Digital transmission, SSB, suppressed carrier (J2D class of emission) |  | 9 | 9 | 9 |

1.5	For each transmission type, two time percentages are used: one (e.g. 99% of the time) which is intended to fully satisfy the required performance quality criteria in the fading signal environment when the wanted signal is at its weakest level at the instant when the interference signal is likely to be at its strongest level (the protection ratio values corresponding to this condition are shown in Table 1 outside the parenthesis), and another one (e.g. 75 %) which ensures protection during a lesser percentage of time (values in brackets in Table 1).

TABLE  2

Conversion factors for different notified power types

| Class of emission |  | Notified power type | Conversion1, 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | mean to p.e.p. | p.e.p. to mean |
| N0N |  | Z | 0 | 0 |
| A1A, A1B, A1C |  | X | – | – 3 |
| A2A, A2B, A2N |  | Y | +4 | – |
| H2A, H2B, H2N, D2A |  | Y | +3 | – |
| R2B, J2B |  | X | – | –3 |
| A3E(BC) |  | Z | +6 | 0 |
| A3E, H3E |  | Y |  4 (3-6) | – |
| R3E, J3E |  | X | – | – 4 (4-10) |
| A3C |  | Y |  4 | – |
| R3C, J3C |  | X | – | 0 |
| A7B, H7B |  | Y |  4 | – |
| R7C, J7C |  | X | – | – 4 (3-6) |
| B7B |  | X | – | – 4 |
| B8E |  | X | – | – 4 (3-13) |
| B8C |  | X | – | 0 |
| AXX |  | Y |  6 | – |
| BXX, JXX |  | X | – | – 4 (3-10) |
| B9W |  | X | – | – 4 |
| F, G/1,2,3,7, X/B,C,D,X |  | Y | 0 | – |
| P,L,M,X/any |  | X | – | 10 log (t /T ) |
| K2B |  | X | – | 10 log (t /T ) – 5 |
| K3E |  | X | – | 10 log (t /T ) – 4 |
| 1 In the case where in, brackets, more than one figure is given, these figures refer to different modulating signal conditions (e.g. smoothly read text instead of sinusoidal modulating signal at 100% carrier modulation) (see Recommendation ITU-R SM.326-6). |  |  |  |  |
| 2 In the case of pulse modulation: |  |  |  |  |

# 2	Technical Standard A-2:  Minimum field strength to be protected

2.1	The present Technical Standard contains values for the minimum field strength to be protected (see Tables 1 to 4 and 5A and 5B) for application in the technical examinations of notices of frequency assignments in the frequency bands between 9 kHz and 28 000 kHz.

2.2	The values contained in this Standard are based on Recommendations ITU-R F.339-7 and ITU-R P.372-8.

2.3	The aim of calculating the minimum field strength to be protected within the technical examinations is to determine the field strength at the receiving point below which the wanted signal is not worth protecting against interfering signals because the wanted signal-to-noise ratio is smaller than that which could satisfy the required performance quality criteria without interference.

2.4	Technical Standard A-2 contains values for the minimum field strength to be protected (dB relative to 1 V/m) for the main types of transmissions (from telegraphy, aural reception, to telephony, for connection to the public network and to digital transmissions) in the frequency bands from 9 kHz to 28 000 kHz. These values of the minimum field strength have been determined from the median values (exceeded 50% of time) of the noise level (atmospheric, man-made or galactic) and the steady state ratio, S/N by adding appropriate allowances for 90% of time to take into account the noise level variation, Du, and the intensity fluctuation of the wanted signal, IF.

2.5	The assessment of the minimum field strength to be protected is based on a uniform reference power type: the peak envelope power (p.e.p. notified as PX). Types of power other than p.e.p. (notified as PY or PZ for, respectively, mean or carrier power) are converted to p.e.p. using conversion factors given in Table 2 of Technical Standard A-1.

2.6	Technical Standard A-2 contains four tables (Tables 1 to 4) giving the noise grades expressed as median of hourly values of the radio noise power in a short vertical antenna relative to the thermal noise, at a frequency of 1 MHz, in terms of latitude and longitude of the receiving point. Separate tables are given for four periods of the year (DC, MR, JN and SE), and in each table the noise grade is given for each of six local time blocks of four consecutive hours (N2, T1, J1, J2, T2, N1). Tables 1 to 4 are superimposed on an outline map of the world.

2.7	Tables 5A and 5B give, for the various types of transmission, the minimum value of the field to be protected (dB relative to 1 V/m) in terms of the noise grades obtained from Tables 1 to 4, the frequency used and the time of transmission. In working with Tables 1 to 5B, interpolations are usually necessary as a result of restricting the size of these Tables to manageable proportions.

| Minimum field strength to be protected (dB relative to 1 V/m) | Type of transmission: Telegraphy, aural reception (B  0.5 kHz) | 5A |
| --- | --- | --- |

| NOISE GRADE | (kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 |  |  | 20 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 500 |  |  |
|  | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 |
| 100 | 72 72 | 72 74 | 74 77 | 70 71 | 72 75 | 81 81 | 72 72 | 72 74 | 74 77 | 70 71 | 72 75 | 81 81 | 72 72 | 72 74 | 74 77 | 70 71 | 72 75 | 81 81 |
| 90 | 69 70 | 69 71 | 72 74 | 67 67 | 69 71 | 77 77 | 69 70 | 69 71 | 72 74 | 67 67 | 69 71 | 77 77 | 69 70 | 69 71 | 72 74 | 67 67 | 69 71 | 77 77 |
| 80 | 66 67 | 66 68 | 69 71 | 63 63 | 65 66 | 73 72 | 66 67 | 66 68 | 69 71 | 63 63 | 65 66 | 73 72 | 66 67 | 66 68 | 69 71 | 63 63 | 65 66 | 73 72 |
| 70 | 64 64 | 63 65 | 66 68 | 60 59 | 61 61 | 68 68 | 64 64 | 63 65 | 66 68 | 60 59 | 61 61 | 68 68 | 64 64 | 63 65 | 66 68 | 60 59 | 61 61 | 68 68 |
| 60 | 61 61 | 60 61 | 64 66 | 57 56 | 57 56 | 64 63 | 61 61 | 60 61 | 64 66 | 57 56 | 57 56 | 64 63 | 61 61 | 60 61 | 64 66 | 57 56 | 57 56 | 64 63 |
| 50 | 58 58 | 57 58 | 61 63 | 53 52 | 53 52 | 60 59 | 58 58 | 57 58 | 61 63 | 53 52 | 53 52 | 60 59 | 58 58 | 57 58 | 61 63 | 53 52 | 53 52 | 60 59 |
| 40 | 55 55 | 55 55 | 58 60 | 49 49 | 50 47 | 56 55 | 38 38 | 39 35 | 46 43 | 26 26 | 28 24 | 35 32 | 14 14 | 16 14 | 22 20 | 7 |  |  |
| 30 | 52 52 | 52 51 | 56 58 | 46 45 | 47 42 | 52 50 | 33 32 | 34 28 | 40 36 | 19 20 | 22 16 | 27 24 | 11 11 | 11 11 | 13 11 | 7 |  |  |
| 20 | 50 49 | 49 48 | 54 55 | 43 40 | 42 37 | 48 46 | 28 27 | 28 20 | 33 30 | 15 15 | 15 15 | 20 15 | 11 11 | 11 11 | 11 11 | 7 |  |  |
| 10 | 48 47 | 46 45 | 51 53 | 40 35 | 39 32 | 44 42 | 22 21 | 23 18 | 28 21 | 15 15 | 15 15 | 15 15 | 11 11 | 11 11 | 11 11 | 7 |  |  |
| 0 | 45 44 | 43 41 | 48 50 | 36 31 | 35 27 | 40 37 | 18 18 | 18 18 | 22 18 | 15 15 | 15 15 | 15 15 | 11 11 | 11 11 | 11 11 | 7 |  |  |

| (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Constants to be added to obtain other types of emissions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 1.5 |  |  | 2 |  |  | 3 |  |  | 4 |  |  |  |  |  |  |  |
| N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 |  | Digital transmissions, J2D |  |  | −8 |
| 52 51 | 54 55 | 52 54 | 47 47 | 50 49 | 41 43 | 44 42 | 47 45 | 34 36 | 38 36 | 42 39 | 23 27 | 34 32 | 38 35 | 16 22 |  | Narrow-band TG (B  0.5 kHz) |  |  | –5 |
| 42 41 | 44 45 | 42 44 | 38 37 | 40 40 | 32 33 | 35 34 | 38 36 | 26 28 | 31 30 | 34 31 | 17 20 | 28 27 | 31 28 | 11 15 |  | Telegraphy aut. (B  0.5 kHz) |  |  | 4 |
| 32 31 | 34 35 | 32 34 | 28 28 | 31 30 | 23 25 | 27 26 | 29 28 | 18 20 | 24 23 | 27 24 | 10 13 | 22 21 | 25 22 | 5 9 |  | Phototelegraphy |  |  | 16 |
| 22 21 | 24 25 | 22 24 | 19 19 | 22 22 | 14 16 | 18 18 | 20 20 | 10 12 | 17 16 | 19 18 | 3 6 | 16 15 | 18 16 | 1 4 |  | T |  | J3E R3E | 14 |
| 12 11 | 14 15 | 12 14 | 10 9 | 12 12 | 6 7 | 10 9 | 12 11 | 2 4 | 10 9 | 12 11 | –1 0 | 10 9 | 12 10 | –1 –1 |  | e l | CO | B8E H3E | 20 |
| 4 4 | 4 5 | 4 4 | 3 3 | 3 3 | 3 3 | 2 2 | 3 3 | 2 2 | 3 2 | 4 3 | –1 –1 | 4 4 | 5 4 | –1 –1 |  | e p |  | A3E | 23 |
| 4 |  |  | 3 |  |  | 2 |  |  | –1 |  |  | –1 |  |  |  | h o |  | J3E R3E B8E | 25 |
| 4 |  |  | 3 |  |  | 2 |  |  | –1 |  |  | –1 |  |  |  | n y | CP | H3E | 31 |
| 4 |  |  | 3 |  |  | 2 |  |  | –1 |  |  | –1 |  |  |  |  |  | A3E | 34 |
| 4 |  |  | 3 |  |  | 2 |  |  | –1 |  |  | –1 |  |  |  | Broad- | LF/MF |  | 49 |
| 4 |  |  | 3 |  |  | 2 |  |  | –1 |  |  | –1 |  |  |  | cast | BC Trop. |  | 46 |

| Minimum field strength to be protected (dB relative to 1 V/m) | Type of transmission: Telegraphy, aural reception (B  0.5 kHz) | 5B |
| --- | --- | --- |

|  | (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOISE GRADE | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  |  | 8 |  |  | 10 |  |
|  | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 |
| 100 | 34 32 | 38 35 | 16 22 | 31 30 | 34 32 | 12 17 | 28 28 | 31 30 | 11 15 | 25 26 | 28 29 | 11 14 | 23 24 | 25 28 | 11 13 | 18 21 | 21 26 |
| 90 | 28 27 | 31 28 | 11 15 | 26 25 | 28 26 | 8 12 | 23 23 | 25 25 | 8 11 | 21 21 | 23 24 | 9 11 | 18 20 | 21 23 | 9 11 | 14 17 | 16 22 |
| 80 | 22 21 | 25 22 | 5 9 | 20 19 | 23 21 | 4 8 | 18 18 | 21 20 | 5 8 | 16 16 | 18 19 | 6 9 | 14 15 | 16 19 | 7 9 | 9 12 | 13 18 |
| 70 | 16 15 | 18 16 | 1 4 | 15 14 | 17 15 | 1 3 | 13 13 | 16 15 | 2 4 | 11 12 | 14 14 | 3 6 | 9 11 | 13 14 | 4 7 | 4 8 | 9 14 |
| 60 | 10 9 | 12 10 | –1 –1 | 9 9 | 12 10 | –3 –1 | 9 9 | 11 10 | –1 1 | 7 8 | 10 10 | 1 3 | 5 7 | 9 10 | 2 4 | 0 3 | 5 10 |
| 50 | 4 4 | 5 4 | –1 –1 | 4 5 | 6 5 | –3 –3 | 4 5 | 6 5 | –3 –2 | 2 4 | 6 5 | –2 –1 | 8 3 | 5 6 | 0 1 | –5 –1 | 1 6 |
| 40 | –1 |  |  | –1 –1 | 0 –1 | –3 –3 | –3 |  |  | –3 –3 | 1 1 | –3 –3 | –3 –3 | 0 2 | –3 –3 | –5 –5 | –3 2 |
| 30 | –1 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –5 –5 | –5 –2 |
| 20 | –1 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –5 |  |
| 10 | –1 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –5 |  |
| 0 | –1 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –3 |  |  | –5 |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Constants to be added to obtain other types of emissions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 |  |  | 15 |  |  | 20 |  |  | 30 |  |  |  |  |  |  |  |
| J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 | N2 N1 | T1 T2 | J1 J2 |  | Digital transmissions, J2D |  |  | −8 |
| 12 14 | 14 19 | 17 25 | 13 15 | 7 14 | 11 22 | 13 15 | –3 7 | 4 17 | 10 14 | –7 –7 | –7 0 | –3 2 |  | Narrow-band TG (B  0.5 kHz) |  |  | –5 |
| 10 12 | 9 14 | 12 21 | 11 13 | 2 9 | 6 18 | 11 13 | –7 0 | –2 11 | 8 12 | –7 |  | –7 –5 |  | Telegraphy aut. (B  0.5 kHz) |  |  | 4 |
| 8 10 | 4 9 | 8 17 | 9 11 | –4 3 | 1 13 | 9 11 | –7 –7 | –7 5 | 5 9 | –7 |  |  |  | Phototelegraphy |  |  | 16 |
| 6 8 | –1 4 | 4 13 | 7 9 | –5 –3 | –3 9 | 7 9 | –7 –7 | –7 –1 | 2 6 | –7 |  |  |  | T |  | J3E R3E | 14 |
| 4 6 | –5 –1 | 0 9 | 5 7 | –5 –5 | –5 4 | 5 7 | –7 –7 | –7 –6 | –2 2 | –7 |  |  |  | e l | CO | B8E H3E | 20 |
| 2 4 | –5 –5 | –4 5 | 3 5 | –5 –5 | –5 0 | 2 5 | –7 |  | –5 –2 | –7 |  |  |  | e p |  | A3E | 23 |
| 0 2 | –5 –5 | –5 1 | 1 3 | –5 |  | 0 2 | –7 |  | –7 –6 | –7 |  |  |  | h o |  | J3E R3E B8E | 25 |
| –2 0 | –5 |  | 0 1 | –5 |  | –2 –1 | –7 |  |  | –7 |  |  |  | n y | CP | H3E | 31 |
| –4 –2 | –5 |  | –2 –1 | –5 |  | –4 –4 | –7 |  |  | –7 |  |  |  |  |  | A3E | 34 |
| –5 –4 | –5 |  | –4 –3 | –5 |  |  | –7 |  |  | –7 |  |  |  | Broad- | LF/MF |  | 49 |
|  | –5 |  |  | –5 |  |  | –7 |  |  | –7 |  |  |  | cast | BC Trop. |  | 46 |

# 3	Technical Standard A-3:  Frequency discrimination

3.1	The present Technical Standard contains values for “receiver discrimination” that are defined as a correction (dB), to be applied to the signal-to-interference ratio and are expressed as a function of the frequency separation between the wanted and unwanted emissions ( f ). The term “receiver discrimination” is equivalent to the definition of “relative RF protection ratio”.

3.2	The values contained in this Technical Standard were determined on the basis of:

–	the selectivity of typical receivers assumed to be used for different classes of emission, and

–	the necessary bandwidth occupied by the interfering stations, together with the energy distribution of the power within and outside the bandwidth.

3.3	Data that are used for the establishment of this Standard were extracted from the Recommendations ITU-R SM.328-8 and ITU-R SM.332-4; they are summarized in Table 1.

3.4	The method of calculation of the values of Technical Standard A-3 consisted in considering the energy accepted by the receiver tuned to a frequency of a given frequency separation ( f ) and comparing this energy with the one that the receiver would accept if tuned to the assigned frequency of the emission ( f   0).

3.5	The value of the discrimination, in cases where the wanted emission bandwidth overlaps with the receiver passband, depends on both the transmitted spectrum and the receiver selectivity curve. However, in cases of higher frequency separation (where there is no overlapping) the discrimination is determined mainly by the slope of out-of-band emission.

3.6	The values of the receiver discrimination are given in Table 2 in terms of the frequency discrimination factor, d. The frequency discrimination factor d represents the difference between the limits of the bandwidths, as indicated in Fig. 1 below.

3.7	In this approach, the likelihood of mutual interference is not considered in cases where the notified bandwidths of the emissions are separated more than 500 Hz (i.e. for d  0.5).

TABLE  1

Assumed characteristics of receivers of wanted emissions and characteristics of interfering emissions

|  | Receiver of wanted emission |  | Interfering emission |  |  |
| --- | --- | --- | --- | --- | --- |
| Class of emission | Assumed passband (kHz) | Attenuation slope (dB/kHz) | Necessary bandwidth (kHz) | Level of components at the edge of the necessary bandwidth (dB) | Slope of out-of-band spectrum |
| A1A (9-1 605 kHz) | 0.3, 0.5, 0.75 or 1 kHz, depending on notified bandwidth of wanted emission | 120 | As notified | –27 at 5 B/2 | –57 dB at 5 B, then 12 dB/oct |
| A1A (1 605-28 000 kHz) (B: up to 200 Bd) | 1 | 120 | As notified | –27 at 5 B/2 | –57 dB at 5 B, then 12 dB/oct |
| F1B (9-1 605 kHz) | 0.3, 0.5, 0.75, 1 or 1.5 kHz, depending on notified bandwidth of wanted emission | 120 | As notified | –15 | 13  1.8 m  20 dB/oct |
| F1B (1 605-28 000 kHz) (2D  200 to 400 Hz, B: up to 200 Bd, m  2 to 6) | 1.5 | 120 | As notified | –15 at 2.6 D  0.55 B | 13  1.8 m  20 dB/oct |
| A2A, A2B (F: up to 1 000 Hz, B: up to 50 Bd) | 2 | 120 | As notified | –24 at (F  5 B/2) | 12 dB/oct |
| A1C, A3C, A7B, AXX, F1C, F2B, F7B | 2, 2.5, 3, 3.5, 4, 4.5, 5, 6, 7, 8, 9, 10 or 12 kHz depending on notified bandwidth of wanted emission | 120 | As notified | –15 | 20 dB/oct in respect to the outer channel |
| A3E | 6 | 20 | As notified | –23 | 12 dB/oct |
| R3E, H3E, J3E | 3 | 100 | As notified | –23 | 12 dB/oct |
| B8E | 6, 9 or 12 kHz depending on notified bandwidth of wanted emission | 100 | As notified | –23 | 12 dB/oct |

B:	telegraphic speed (Bd)
2D:	difference between mark and space frequencies
m:	modulation index 2D/B
F: 	modulation frequency.

TABLE  2

Values of the frequency discrimination (dB) for different interfering emissions

| d | 100HA1A | 500HA1A 500HA1B | 1K00A1B | (A,H)2(A,B,N) | Other telegraphy | Telephony SSB-CP or ISB-CO/CP (with privacy device) | Telephony DSB-CP (with PD) SSB-CO (without PD) | Telephony DSB-CO (without PD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| – 1.0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| – 0.9 | 0 | 0 | 0 | 6 | 0 | 0 | 0 | 0 |
| – 0.8 | 0 | 0 | 0 | 6 | 0 | 0 | 0 | 0 |
| – 0.7 | 0 | 0 | 0 | 6 | 0 | 0 | 0 | 3 |
| – 0.6 | 0 | 0 | 0 | 6 | 0 | 0 | 3 | 9 |
| – 0.5 | 0 | 0 | 0 | 6 | 0 | 3 | 9 | 15 |
| – 0.4 | 0 | 0 | 3 | 6 | 0 | 9 | 15 | 21 |
| – 0.3 | 0 | 0 | 9 | 6 | 0 | 13 | 19 | 25 |
| – 0.2 | 0 | 0 | 14 | 6 | 3 | 17 | 23 | 29 |
| – 0.1 | 0 | 11 | 18 | 6 | 6 | 19 | 25 | 31 |
| 0.0 | 10 | 17 | 22 | 6 | 10 | 21 | 27 | 33 |
| 0.1 | 20 | 26 | 30 | 17 | 20 | 28 | 34 | 38 |
| 0.2 | 30 | 34 | 37 | 28 | 30 | 37 | 40 | 44 |
| 0.3 | 40 | 43 | 45 | 38 | 40 | 44 | 46 | 49 |
| 0.4 | 50 | 51 | 52 | 49 | 50 | 52 | 53 | 55 |
| 0.5 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
|  0.5 |  60 |  60 |  60 |  60 |  60 |  60 |  60 |  60 |
| d: receiver discrimination factor (d   f – 0.5 (Bwa  Bwi)) Bwa: passband of the receiver of wanted emission Bwi: bandwidth of interfering emission  f: frequency separation between assigned frequencies. |  |  |  |  |  |  |  |  |

# 4	Technical Standard A-5:  Propagation and field strength calculations

4.1	The present Technical Standard contains information on the methodology used with respect to the propagation and field strength calculations, for application in the technical examinations of notices of frequency assignments in the frequency bands between 9 kHz and 28 000 kHz.

4.2	Paragraph 4.3 and Tables 1A to 5 of this Technical Standard deal with the field strength calculations in the frequency band between 9 kHz and 3 900 kHz. Paragraph 4.4 deals with the frequency bands between 3 900 kHz and 28 000 kHz.

4.3	Two modes of propagation are considered in the frequency bands between 9 kHz and 3 900 kHz: the ground-wave mode and the sky-wave mode. Values of field strength for these two modes of propagation are contained in Tables 1A and 2 to 5 of this Technical Standard in the form of tables as a function of the distance.

4.3.1	The field strength values contained in Tables 1A and 2 to 5 are expressed as median values (exceeded 50% of the time) (dB relative to 1 V/m). They relate to a radiated power of 1 kW (30 dBW) from a loss-free halfwave dipole isolated in space that produces a field strength of 222 mV/m at a distance of 1 km from the antenna. Table 1B contains antenna efficiency factors (correction factors) to be applied in conjunction with Table 1A to consider differences between radiated and notified power values.

4.3.2	Field strength values for the ground-wave propagation mode are calculated on the basis of Recommendation ITU-R P.368-7 for the following reference values:

–	propagation over sea:     4 S/m, r    80;

–	propagation over land:     10 mS/m, r    4.

4.3.3	In the technical examinations, where the ground-wave propagation mode is involved, only homogeneous paths are considered, with no use of mixed-path methodology.

4.3.4	The tables of field strength values for the sky-wave propagation mode in the frequency bands between 9 kHz and 3 900 kHz contain only the value that corresponds to the strongest mode of propagation. These values have been consolidated from different sources (Recommendations ITU-R P.533-5, ITU-R P.684-1, ITU-R P.1147, ITU-R P.435-7*, etc.).

4.3.5	For the day-time propagation only the ground-wave mode is considered within the same time zone. For the night-time propagation both modes are considered; however, only the greater of the ground-wave and the sky-wave mode is used for subsequent calculations.

4.4	Concerning the calculation of the sky-wave in the bands between 3 900 kHz and 28 000 kHz, the Board noted that the ITU-R recommends the propagation method referred to in Annex 1 to Recommendation ITU-R P.533-5 as it has comparable accuracy to the other more complex methods. The Board also noted that the implementation of that method in the Bureau’s calculation methodology for determining the affected administrations in the application of No. 9.21 may require considerable resources, which may not be justifiable having in mind the expected low volume of application of this methodology. Therefore, the Board decided that the following methodology is to be applied:

4.4.1	The monthly median values of the standard MUF (EJF) are calculated in accordance with Recommendation ITU-R P.434-5* and Report ITU-R P.2011-1, for two reference values (5 and 125) of the relative sunspot number R12 and for two selected months of the year (June and December).

4.4.2	The field-strength values are calculated in accordance with the methodology explained in the NBS Circular No. 462.

TABLE  1A

Field strength (dB relative to 1 V/m)	9-535 kHz

|  | Ground wave : sea |  |  |  |  |  |  |  |  |  |  | Sky wave at night |  |  |  |  |  |  | Ground wave : land |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance | Frequency (kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Distance |
| (km) | 10 | 30 | 60 | 100 | 150 | 200 | 300 | 400 | 500 |  |  | 10-100 | 150 | 200 | 300 | 400 | 500 |  | 10 | 30 | 60 | 100 | 150 | 200 | 300 | 400 | 500 | (km) |
| 10 50 100 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 |  |  |  |  |  | 43 | 43 | 43 |  | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 66 | 87 72 65 | 87 72 64 | 87 71 62 | 10 50 100 |
| 200 300 400 | 60 57 55 | 60 56 54 | 60 56 53 | 60 56 53 | 60 56 52 | 60 55 51 | 60 55 50 | 59 54 50 | 59 54 49 |  |  | 45 | 45 | 44 | 44 44 44 | 44 44 44 | 44 44 43 |  | 60 57 55 | 60 56 54 | 60 56 53 | 60 56 53 | 60 55 52 | 59 54 50 | 57 50 45 | 54 47 40 | 50 42 35 | 200 300 400 |
| 500 600 700 | 52 50 48 | 51 49 46 | 50 48 45 | 50 47 43 | 48 45 42 | 47 44 41 | 47 43 39 | 46 42 38 | 45 41 37 |  |  | 44 43 42 | 44 43 42 | 43 43 42 | 43 42 41 | 43 42 41 | 42 41 40 |  | 52 50 48 | 51 49 46 | 50 48 45 | 50 47 43 | 48 45 42 | 45 42 39 | 40 35 31 | 34 27 22 | 27 21 15 | 500 600 700 |
| 800 900 1 000 | 46 45 43 | 44 42 40 | 43 40 38 | 41 38 36 | 39 37 33 | 38 35 32 | 37 33 30 | 35 31 28 | 33 30 26 |  |  | 41 40 39 | 41 40 39 | 41 39 38 | 40 39 38 | 40 38 37 | 39 38 37 |  | 46 45 43 | 44 42 40 | 43 40 38 | 41 38 36 | 39 36 33 | 35 32 29 | 27 22 18 | 17 12 7 | 7 2 –4 | 800 900 1 000 |
| 1 100 1 200 1 300 | 42 40 39 | 38 37 35 | 36 34 32 | 33 31 29 | 31 29 26 | 29 27 24 | 27 24 21 | 25 21 17 | 22 19 15 |  |  | 38 38 36 | 37 36 35 | 37 36 35 | 36 35 34 | 36 35 33 | 35 34 33 |  | 42 40 39 | 38 37 35 | 36 34 32 | 33 31 29 | 30 27 24 | 25 22 19 | 14 9 6 | 2 –3 –8 | –10 –15 –21 | 1 100 1 200 1 300 |
| 1 400 1 500 1 600 | 38 36 35 | 33 32 29 | 29 27 25 | 26 24 22 | 23 21 18 | 21 18 15 | 17 14 11 | 14 11 7 | 12 8 5 |  |  | 36 35 34 | 34 33 32 | 34 33 32 | 33 32 31 | 32 31 30 | 32 30 29 |  | 38 36 35 | 33 32 29 | 29 27 25 | 27 24 22 | 22 19 16 | 16 13 10 | –2 –2 –7 | –13 –18 –23 |  | 1 400 1 500 1 600 |
| 1 700 1 800 1 900 | 34 32 31 | 2826 24 | 23 20 19 | 19 17 15 | 15 13 10 | 12 10 7 | 8 5 2 | 4 1 –3 | 1 –3 –6 |  |  | 33 32 31 | 32 31 30 | 31 31 30 | 30 29 28 | 29 28 27 | 28 27 26 |  | 34 32 31 | 28 26 24 | 23 20 19 | 20 18 15 | 13 10 8 | 7 4 1 | –10 –14 –18 |  |  | 1 700 1 800 1 900 |
| 2 000 2 200 2 400 | 29 27 25 | 23 19 16 | 17 13 9 | 12 7 4 | 8 3 –2 | 4 –1 –6 | –1 –7 –13 | –5 –12 –18 | –9 –16 –23 |  |  | 31 29 28 | 29 28 27 | 29 27 26 | 27 25 24 | 26 24 23 | 25 23 21 |  | 29 27 25 | 23 19 16 | 17 13 9 | 13 7 4 | 6 0 –5 | –3 –8 –14 | –21 |  |  | 2 000 2 200 2 400 |
| 2 600 2 800 3 000 | 22 20 18 | 13 9 7 | 5 2 –3 | –1 –5 –10 | –7 –11 –17 | –11 16 22 | –18 –24 | –24 |  |  |  | 27 26 25 | 26 24 23 | 25 23 22 | 23 21 20 | 21 20 18 | 20 18 17 |  | 22 20 18 | 13 9 7 | 5 2 –3 | –1 –5 –10 | –10 –15 –21 | –20 |  |  |  | 2 600 2 800 3 000 |
| 3 200 3 400 3 600 | 15 13 10 | 4 0 –3 | –5 –10 –13 | –14 –18 –23 | –21 |  |  |  |  |  |  | 25 24 24 | 23 22 21 | 21 20 20 | 19 18 17 | 17 16 15 | 15 14 13 |  | 15 13 10 | 4 0 –3 | –5 –10 –13 | –14 –18 –23 |  |  |  |  |  | 3 200 3 400 3 600 |
| 3 800 4 000 5 000 | 9 7 –5 | –5 –9 –24 | –16 –20 |  |  |  |  |  |  |  |  | 23 23 23 | 21 20 20 | 19 18 17 | 16 15 14 | 14 13 10 | 12 11 8 |  | 9 7 –5 | –5 –9 –24 | –16 –20 |  |  |  |  |  |  | 3 800 4 000 5 000 |
| 6 000 7 000 | –15 |  |  |  |  |  |  |  |  |  |  | 23 22 | 20 19 | 17 16 | 14 13 | 8 8 | 6 5 |  | –15 |  |  |  |  |  |  |  |  | 6 000 7 000 |

TABLE  1B

Antenna efficiency (correction factor)

| Frequency band (kHz) | Class of station | Correction (dB) |
| --- | --- | --- |
| 9-70 | Fixed, land and radionavigation land stations: – power above 1 kW – power equal to 1 kW – power below 1 kW | – 10 – 12 – 15 |
| 70-150 | Fixed, land and radionavigation land stations: – power above 1 kW – power equal to 1 kW – power below 1 kW | – 7 – 9 – 12 |
| 150-535 | Fixed, land and radionavigation land stations (except non-directional beacons): – power above 1 kW – power equal to 1 kW – power below 1 kW | – 2 – 4 – 7 |

TABLE  2

Field strength (dB relative to 1 V/m)	SEA

|  | 1 605-2 300 kHz |  |  |  |  |  |  | 2 300-2 850 kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 30° | 40° | 50° | 60° |  |  |
| 10 | 86 |  |  |  |  |  |  | 86 |  |  |  |  |  |  | 10 |
| 50 | 72 |  |  |  |  |  |  | 72 |  |  |  |  |  |  | 50 |
| 100 | 65 |  |  |  |  |  |  | 65 |  |  |  |  |  |  | 100 |
| 200 | 57 |  |  |  |  |  |  | 57 |  |  |  |  |  |  | 200 |
| 300 | 50 |  |  |  |  |  |  | 50 |  |  |  |  |  |  | 300 |
| 400 | 43 |  |  |  |  |  | 41 | 43 |  |  |  |  |  | 43 | 400 |
| 500 | 38 |  |  |  |  |  | 41 | 37 |  |  |  |  |  | 43 | 500 |
| 600 | 32 |  |  |  |  |  | 41 | 30 |  |  |  |  |  | 43 | 600 |
| 700 | 27 |  |  |  |  |  | 41 | 24 |  |  |  |  |  | 43 | 700 |
| 800 | 22 |  |  |  |  |  | 41 | 18 |  |  |  |  |  | 43 | 800 |
| 900 | 16 |  |  |  |  |  | 40 | 12 |  |  |  |  | 1 | 42 | 900 |
| 1 000 | 11 |  |  |  |  |  | 39 | 7 |  |  |  | –9 | –1 | 41 | 1 000 |
| 1 100 | 6 |  |  |  | –12 | –7 | 39 | 2 |  |  | –13 | –10 | –3 | 41 | 1 100 |
| 1 200 | 0 |  |  | –15 | –13 | –8 | 38 | –3 | –15 | –15 | –14 | –11 | –4 | 40 | 1 200 |
| 1 300 | –5 | –17 | –17 | –16 | –14 | –9 | 37 | –9 | –16 | –16 | –15 | –12 | –6 | 39 | 1 300 |
| 1 400 | –10 | –18 | –18 | –17 | –15 | –10 | 36 | –16 | –18 | –17 | –16 | –14 | –7 | 38 | 1 400 |
| 1 500 | –15 | –20 | –20 | –19 | –16 | –12 | 35 |  | –19 | –19 | –18 | –15 | –9 | 37 | 1 500 |
| 1 600 |  | –21 | –21 | –20 | –17 | –13 | 34 |  | –20 | –20 | –19 | –16 | –10 | 36 | 1 600 |

TABLE  2

Field strength (dB relative to 1 V/m) (end)	SEA

|  | 1 605-2 300 kHz |  |  |  |  |  |  | 2 300-2 850 kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 30° | 40° | 50° | 60° |  |  |
| 1 700 |  |  |  |  |  |  | 33 |  |  |  |  |  |  | 36 | 1 700 |
| 1 800 |  |  |  |  |  |  | 33 |  |  |  |  |  |  | 35 | 1 800 |
| 1 900 |  |  |  |  |  |  | 32 |  |  |  |  |  |  | 34 | 1 900 |
| 2 000 |  |  |  |  |  |  | 32 |  |  |  |  |  |  | 34 | 2 000 |
| 2 200 |  |  |  |  |  |  | 31 |  |  |  |  |  |  | 33 | 2 200 |
| 2 400 |  |  |  |  |  |  | 30 |  |  |  |  |  |  | 32 | 2 400 |
| 2 600 |  |  |  |  |  |  | 29 |  |  |  |  |  |  | 31 | 2 600 |
| 2 800 |  |  |  |  |  |  | 28 |  |  |  |  |  |  | 30 | 2 800 |
| 3 000 |  |  |  |  |  |  | 27 |  |  |  |  |  |  | 29 | 3 000 |
| 3 200 |  |  |  |  |  |  | 26 |  |  |  |  |  |  | 27 | 3 200 |
| 3 400 |  |  |  |  |  |  | 25 |  |  |  |  |  |  | 26 | 3 400 |
| 3 600 |  |  |  |  |  |  | 23 |  |  |  |  |  |  | 24 | 3 600 |
| 3 800 |  |  |  |  |  |  | 21 |  |  |  |  |  |  | 22 | 3 800 |
| 4 000 |  |  |  |  |  |  | 19 |  |  |  |  |  |  | 20 | 4 000 |
| 4 500 |  |  |  |  |  |  | 15 |  |  |  |  |  |  | 16 | 4 500 |
| 5 000 |  |  |  |  |  |  | 10 |  |  |  |  |  |  | 11 | 5 000 |
| The values for sky-wave field strength have not been included when they are less than the ground-wave, except when they make the interpolation easier. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  3

Field strength (dB relative to 1 V/m)	SEA

|  | 2 850-3 500 kHz |  |  |  |  |  |  | 3 500-3 900 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 10° | 20° | 30° | 40° | 50° | 60° |  |  |
| 10 | 86 |  |  |  |  |  |  | 86 |  |  |  |  |  |  |  |  | 10 |
| 50 | 72 |  |  |  |  |  |  | 72 |  |  |  |  |  |  |  | 47 | 50 |
| 100 | 65 |  |  |  |  |  |  | 65 | 27 | 28 | 29 | 30 | 31 | 35 | 40 | 47 | 100 |
| 200 | 56 |  |  |  |  |  |  | 55 | 24 | 25 | 26 | 27 | 29 | 32 | 37 | 47 | 200 |
| 300 | 48 |  |  |  |  |  | 44 | 47 | 19 | 20 | 21 | 22 | 25 | 29 | 34 | 47 | 300 |
| 400 | 41 |  |  |  |  |  | 44 | 40 | 14 | 15 | 16 | 17 | 21 | 26 | 32 | 47 | 400 |
| 500 | 35 |  |  |  |  |  | 44 | 33 | 5 | 8 | 11 | 14 | 19 | 24 | 30 | 47 | 500 |
| 600 | 29 |  |  |  |  |  | 44 | 26 | 0 | 1 | 6 | 11 | 16 | 21 | 28 | 46 | 600 |
| 700 | 22 |  |  |  |  | 12 | 44 | 19 | –2 | –1 | 1 | 5 | 12 | 17 | 26 | 46 | 700 |
| 800 | 16 |  |  |  | –3 | 10 | 44 | 12 | –5 | –4 | –3 | –2 | 8 | 13 | 24 | 45 | 800 |
| 900 | 10 |  |  |  | –5 | 6 | 44 | 6 | –8 | –6 | –5 | –4 | 4 | 12 | 21 | 44 | 900 |
| 1 000 | 4 |  |  | –10 | –6 | 2 | 43 | –1 | –10 | –8 | –6 | –5 | –2 | 10 | 18 | 43 | 1 000 |
| 1 100 | –3 | –14 | –13 | –12 | –8 | 0 | 43 | –7 |  |  |  | –8 | –3 | 5 | 16 | 42 | 1 100 |
| 1 200 | –8 | –15 | –14 | –13 | –10 | –1 | 42 |  |  |  |  | –9 | –5 | 1 | 14 | 42 | 1 200 |
| 1 300 | –13 | –16 | –15 | –14 | –11 | –3 | 41 |  |  |  |  |  | –7 | –1 | 11 | 41 | 1 300 |
| 1 400 |  | –18 | –17 | –16 | –13 | –4 | 40 |  |  |  |  |  | –9 | –3 | 8 | 40 | 1 400 |
| 1 500 |  | –19 | –19 | –18 | –14 | –6 | 39 |  |  |  |  |  |  | –4 | 7 | 39 | 1 500 |

TABLE  3

Field strength (dB relative to 1 V/m) (end)	SEA

|  | 2 850-3 500 kHz |  |  |  |  |  |  | 3 500-3 900 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 10° | 20° | 30° | 40° | 50° | 60° |  |  |
| 1 600 |  | –20 | –20 | –19 | –15 | –7 | 38 |  |  |  |  |  |  | –5 | 5 | 38 | 1 600 |
| 1 700 |  |  |  |  |  |  | 38 |  |  |  |  |  |  |  |  | 38 | 1 700 |
| 1 800 |  |  |  |  |  |  | 37 |  |  |  |  |  |  |  |  | 37 | 1 800 |
| 1 900 |  |  |  |  |  |  | 36 |  |  |  |  |  |  |  |  | 36 | 1 900 |
| 2 000 |  |  |  |  |  |  | 35 |  |  |  |  |  |  |  |  | 35 | 2 000 |
| 2 200 |  |  |  |  |  |  | 34 |  |  |  |  |  |  |  |  | 34 | 2 200 |
| 2 400 |  |  |  |  |  |  | 33 |  |  |  |  |  |  |  |  | 33 | 2 400 |
| 2 600 |  |  |  |  |  |  | 32 |  |  |  |  |  |  |  |  | 32 | 2 600 |
| 2 800 |  |  |  |  |  |  | 31 |  |  |  |  |  |  |  |  | 31 | 2 800 |
| 3 000 |  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  | 30 | 3 000 |
| 3 200 |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  |  | 28 | 3 200 |
| 3 400 |  |  |  |  |  |  | 27 |  |  |  |  |  |  |  |  | 27 | 3 400 |
| 3 600 |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  |  | 26 | 3 600 |
| 3 800 |  |  |  |  |  |  | 23 |  |  |  |  |  |  |  |  | 25 | 3 800 |
| 4 000 |  |  |  |  |  |  | 21 |  |  |  |  |  |  |  |  | 23 | 4 000 |
| 4 500 |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  | 18 | 4 500 |
| 5 000 |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  | 13 | 5 000 |
| The values for sky-wave field strength have not been included when they are less than the ground-wave, except when they make the interpolation easier. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  4

Field strength (dB relative to 1 V/m)	LAND

|  | 1 605-2 300 kHz |  |  |  |  |  |  | 2 300-2 850 kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 30° | 40° | 50° | 60° |  |  |
| 10 | 78 |  |  |  |  |  |  | 75 |  |  |  |  |  |  | 10 |
| 50 | 48 |  |  |  |  |  | 46 | 45 |  |  |  |  |  | 46 | 50 |
| 100 | 34 |  |  |  |  | 30 | 43 | 21 |  |  | 25 | 27 | 31 | 44 | 100 |
| 200 | 17 |  |  | 10 | 15 | 21 | 41 | 13 | 12 | 13 | 16 | 20 | 25 | 43 | 200 |
| 300 | 5 | 2 | 3 | 4 | 8 | 16 | 41 | –1 | 6 | 7 | 10 | 14 | 22 | 43 | 300 |
| 400 | –8 | –4 | –3 | –2 | 2 | 10 | 41 | –15 | –1 | 0 | 2 | 9 | 16 | 43 | 400 |
| 500 |  | –7 | –6 | –5 | –1 | 6 | 41 |  | –5 | –4 | –2 | 5 | 12 | 43 | 500 |
| 600 |  | –9 | –9 | –8 | –4 | 2 | 41 |  | –8 | –7 | –5 | 0 | 8 | 43 | 600 |
| 700 |  | –11 | –11 | –10 | –6 | –1 | 41 |  | –10 | –9 | –7 | –3 | 6 | 43 | 700 |
| 800 |  | –12 | –12 | –11 | –9 | –3 | 41 |  | –11 | –10 | –9 | –6 | 4 | 43 | 800 |
| 900 |  | –13 | –13 | –12 | –10 | –4 | 40 |  | –12 | –11 | –10 | –8 | 1 | 42 | 900 |
| 1 000 |  | –14 | –14 | –13 | –11 | –5 | 39 |  | –13 | –12 | –11 | –9 | –1 | 41 | 1 000 |
| 1 100 |  | –15 | –15 | –14 | –12 | –7 | 39 |  | –14 | –14 | –13 | –10 | –3 | 41 | 1 100 |
| 1 200 |  | –16 | –16 | –15 | –13 | –8 | 38 |  | –15 | –15 | –14 | –11 | –4 | 40 | 1 200 |
| 1 300 |  | –17 | –17 | –16 | –14 | –9 | 37 |  | –16 | –16 | –15 | –12 | –6 | 39 | 1 300 |
| 1 400 |  | –18 | –18 | –17 | –15 | –10 | 36 |  | –18 | –17 | –16 | –14 | –7 | 38 | 1 400 |
| 1 500 |  | –20 | –20 | –19 | –16 | –12 | 35 |  | –19 | –19 | –18 | –15 | –9 | 37 | 1 500 |
| 1 600 |  | –21 | –21 | –20 | –17 | –13 | 34 |  | –20 | –20 | –19 | –16 | –10 | 36 | 1 600 |

TABLE  4

Field strength (dB relative to 1 V/m) (end)	LAND

|  | 1 605-2 300 kHz |  |  |  |  |  |  | 2 300-2 850 kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 30° | 40° | 50° | 60° |  |  |
| 1 700 |  |  |  |  |  |  | 33 |  |  |  |  |  |  | 36 | 1 700 |
| 1 800 |  |  |  |  |  |  | 33 |  |  |  |  |  |  | 35 | 1 800 |
| 1 900 |  |  |  |  |  |  | 32 |  |  |  |  |  |  | 34 | 1 900 |
| 2 000 |  |  |  |  |  |  | 32 |  |  |  |  |  |  | 34 | 2 000 |
| 2 200 |  |  |  |  |  |  | 31 |  |  |  |  |  |  | 33 | 2 200 |
| 2 400 |  |  |  |  |  |  | 30 |  |  |  |  |  |  | 32 | 2 400 |
| 2 600 |  |  |  |  |  |  | 29 |  |  |  |  |  |  | 31 | 2 600 |
| 2 800 |  |  |  |  |  |  | 28 |  |  |  |  |  |  | 30 | 2 800 |
| 3 000 |  |  |  |  |  |  | 27 |  |  |  |  |  |  | 29 | 3 000 |
| 3 200 |  |  |  |  |  |  | 26 |  |  |  |  |  |  | 27 | 3 200 |
| 3 400 |  |  |  |  |  |  | 25 |  |  |  |  |  |  | 26 | 3 400 |
| 3 600 |  |  |  |  |  |  | 23 |  |  |  |  |  |  | 24 | 3 600 |
| 3 800 |  |  |  |  |  |  | 21 |  |  |  |  |  |  | 22 | 3 800 |
| 4 000 |  |  |  |  |  |  | 19 |  |  |  |  |  |  | 20 | 4 000 |
| 4 500 |  |  |  |  |  |  | 15 |  |  |  |  |  |  | 16 | 4 500 |
| 5 000 |  |  |  |  |  |  | 10 |  |  |  |  |  |  | 11 | 5 000 |
| The values for sky-wave field strength have not been included when they are less than the ground-wave, except when they make the interpolation easier. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  5

Field strength (dB relative to 1 V/m)	LAND

|  | 2 850-3 500 kHz |  |  |  |  |  |  | 3 500-3 900 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 10° | 20° | 30° | 40° | 50° | 60° |  |  |
| 10 | 71 |  |  |  |  |  |  | 67 |  |  |  |  |  |  |  |  | 10 |
| 50 | 41 |  |  |  |  |  | 45 | 37 |  |  |  |  |  |  |  | 47 | 50 |
| 100 | 27 | 26 | 27 | 28 | 28 | 32 | 44 | 23 | 27 | 28 | 29 | 30 | 31 | 35 | 40 | 47 | 100 |
| 200 | 14 | 17 | 18 | 22 | 25 | 28 | 44 | 4 | 24 | 25 | 26 | 27 | 29 | 32 | 37 | 47 | 200 |
| 300 |  | 9 | 10 | 15 | 20 | 27 | 44 |  | 19 | 20 | 21 | 22 | 25 | 29 | 34 | 47 | 300 |
| 400 |  | 2 | 3 | 5 | 15 | 21 | 44 |  | 14 | 15 | 16 | 17 | 21 | 26 | 32 | 47 | 400 |
| 500 |  | –3 | –2 | 1 | 11 | 17 | 44 |  | 5 | 8 | 11 | 14 | 19 | 24 | 30 | 47 | 500 |
| 600 |  | –6 | –5 | –3 | 3 | 14 | 44 |  | 0 | 1 | 6 | 11 | 16 | 21 | 28 | 46 | 600 |
| 700 |  | –8 | –7 | –5 | 0 | 12 | 44 |  | –2 | –1 | 1 | 5 | 12 | 17 | 26 | 46 | 700 |
| 800 |  | –10 | –9 | –8 | –3 | 10 | 44 |  | –5 | –4 | –3 | –2 | 8 | 13 | 24 | 45 | 800 |
| 900 |  | –11 | –10 | –9 | –5 | 6 | 44 |  | –8 | –6 | –5 | –4 | 4 | 12 | 21 | 44 | 900 |
| 1 000 |  | –12 | –11 | –10 | –6 | 2 | 43 |  | –10 | –8 | –6 | –5 | –2 | 10 | 18 | 43 | 1 000 |
| 1 100 |  | –14 | –13 | –12 | –8 | 0 | 43 |  |  |  |  | –8 | –3 | 5 | 16 | 42 | 1 100 |
| 1 200 |  | –15 | –14 | –13 | –10 | –1 | 42 |  |  |  |  | –9 | –5 | 1 | 14 | 42 | 1 200 |
| 1 300 |  | –16 | –15 | –14 | –11 | –3 | 41 |  |  |  |  |  | –7 | –1 | 11 | 41 | 1 300 |
| 1 400 |  | –18 | –17 | –16 | –13 | –4 | 40 |  |  |  |  |  | –9 | –3 | 8 | 40 | 1 400 |
| 1 500 |  | –19 | –19 | –18 | –14 | –6 | 39 |  |  |  |  |  |  | –4 | 7 | 39 | 1 500 |
| 1 600 |  | –20 | –20 | –19 | –15 | –7 | 38 |  |  |  |  |  |  | –5 | 5 | 38 | 1 600 |

TABLE  5

Field strength (dB relative to 1 V/m) (end)	LAND

|  | 2 850-3 500 kHz |  |  |  |  |  |  | 3 500-3 900 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sky wave |  |  |  |  |  |  | Sky wave |  |  |  |  |  |  |  |  |
| Distance | Ground | Noon (low solar activity) |  |  |  |  |  | Ground | Noon (low solar activity) |  |  |  |  |  |  |  | Distance |
| (km) | wave | Latitude |  |  |  |  | Night | wave | Latitude |  |  |  |  |  |  | Night | (km) |
|  |  | 0° | 30° | 40° | 50° | 60° |  |  | 0° | 10° | 20° | 30° | 40° | 50° | 60° |  |  |
| 1 700 |  |  |  |  |  |  | 38 |  |  |  |  |  |  | –7 |  | 38 | 1 700 |
| 1 800 |  |  |  |  |  |  | 37 |  |  |  |  |  |  | –9 |  | 37 | 1 800 |
| 1 900 |  |  |  |  |  |  | 36 |  |  |  |  |  |  |  |  | 36 | 1 900 |
| 2 000 |  |  |  |  |  |  | 35 |  |  |  |  |  |  |  |  | 35 | 2 000 |
| 2 200 |  |  |  |  |  |  | 34 |  |  |  |  |  |  |  |  | 34 | 2 200 |
| 2 400 |  |  |  |  |  |  | 33 |  |  |  |  |  |  |  |  | 33 | 2 400 |
| 2 600 |  |  |  |  |  |  | 32 |  |  |  |  |  |  |  |  | 32 | 2 600 |
| 2 800 |  |  |  |  |  |  | 31 |  |  |  |  |  |  |  |  | 31 | 2 800 |
| 3 000 |  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  | 30 | 3 000 |
| 3 200 |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  |  | 28 | 3 200 |
| 3 400 |  |  |  |  |  |  | 27 |  |  |  |  |  |  |  |  | 27 | 3 400 |
| 3 600 |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  |  | 26 | 3 600 |
| 3 800 |  |  |  |  |  |  | 23 |  |  |  |  |  |  |  |  | 25 | 3 800 |
| 4 000 |  |  |  |  |  |  | 21 |  |  |  |  |  |  |  |  | 23 | 4 000 |
| 4 500 |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  | 18 | 4 500 |
| 5 000 |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  | 13 | 5 000 |
| The values for sky-wave field strength have not been included when they are less than the ground-wave, except when they make the interpolation easier. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

_______________

For cases under No. 5.92 the Rules of Procedure of Section B5 also apply.

This Recommendation replaced ex-CCIR Report 322.

This Recommendation replaced ex-CCIR Report 264-1.

* 	Note by the Secretariat: This Recommendation was suppressed by RA-95.

This Recommendation replaced ex-CCIR Recommendation 434 (New Delhi, 1970).

* 	Note by the Secretariat: This Recommendation was suppressed by RA-97.

This Report replaced ex-CCIR Report 340 (New Delhi, 1970).

This is the Circular of the USA National Bureau of Standards “Ionospheric Radio Propagation”.