# PART  B

## SECTION  B8

# Calculation of power-flux density levels produced by aeronautical 
earth stations in motion (A-ESIM) and their validation with the 
limits contained in Annex 3 to Resolution 169 (Rev.WRC-23), 
Annex 2 to Resolution 121 (WRC-23) and Annex 2 to 
Resolution 123 (WRC-23)

Annex 2 to Resolution 121 (WRC-23) and Annex 2 to Resolution 123 (WRC-23) contain methodologies and procedures to examine power flux-density levels at the Earth’s surface produced by A-ESIM. The corresponding methodology for Resolution 169 (Rev.WRC-23) is included in Recommendation ITU-R S.2158-0.

Reference bandwidth of the pfd limit

The three methodologies contain the same formula to calculate the transmission power from the maximum or minimum power spectral densities of A-ESIM.

Depending on the set of pfd limits that is considered (i.e. for A-ESIM altitudes up to 3 km or for those above 3 km), two different reference bandwidths need to be considered: 1 MHz and 14 MHz, respectively.

The Board noted that Note 2 of Recommendation ITU-R S.2158-0 indicates: “For the operation of emission bandwidth smaller than the reference bandwidth, this methodology is applicable provided that the notifying administration confirms that A-ESIM operates only one emission within the reference bandwidth. If there is no such confirmation, this methodology is not applicable.” Moreover, the remark in Resolution 121 (WRC-23) states that “the methodology assumes that only one emission within the reference bandwidth of 14 MHz is transmitted by A-ESIM”.

As a consequence, the Board understood that the intentions of the World Radiocommunication Conference (Sharm el-Sheikh, 2019) (WRC-19) and the World Radiocommunication Conference (Dubai, 2023) (WRC-23) were to allow only one carrier emission to be in operation within the reference bandwidth of 14 MHz for all three cases addressed in Resolutions 121 (WRC-23), 123 (WRC-23) and 169 (Rev.WRC-23).

The Board therefore concluded that when an Administration submits a frequency assignment to an A-ESIM with an emission bandwidth smaller than a 14 MHz reference bandwidth, it also commits to operate only one single emission with that given emission bandwidth in any 14 MHz bandwidth.

When an Administration wishes to simultaneously operate several transmissions with emission bandwidths smaller than a 14 MHz reference bandwidth, the emission characteristics of the carrier should be suitably modified to indicate that multiple channels per carrier will be operated within a single emission (see Appendix 1 to the Radio Regulations).

Conditions for compliance with the pfd limits

The methodology contained in Annex 2 to Resolution 121 (WRC-23), in Annex 2 to Resolution 123 (WRC-23) or in Recommendation ITU-R S.2158-0 determines the maximum allowable power Pj for an A-ESIM transmitter.

The methodology then compares the computed Pj with the range of notified power levels of the A-ESIM emission. The minimum and maximum power values for emissions from the A-ESIM, Pmin_emission,j and Pmax_emission,j, are calculated from the minimum and maximum power spectral densities of the A-ESIM emission.

An A-ESIM transmission is permitted at a certain altitude j, if the following condition is met:

Considering that the condition will prevent the use of altitude j in cases where the allowable power is high enough to permit the operation of the A-ESIM with its maximum notified power spectral density, the Board concluded that the Bureau should also check the following condition:

Whenever that condition is met, it is understood that the entire range of power levels of an A-ESIM can be used.

______________

_______________