2

Radio 
Regulations

Appendices

Edition of 2016

©  ITU  2016

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

# Note by the Secretariat

This revision of the Radio Regulations, complementing the Constitution and the Convention of the International Telecommunication Union, incorporates the decisions of the World Radiocommunication Conferences of 1995 (WRC-95), 1997 (WRC-97), 2000 (WRC-2000), 2003 (WRC-03), 2007 (WRC-07), 2012 (WRC-12) and 2015 (WRC-15). The majority of the provisions of these Regulations shall enter into force as from 1 January 2017; the remaining provisions shall apply as from the special dates of application indicated in Article 59 of the revised Radio Regulations.

In preparing the Radio Regulations, Edition of 2016, the Secretariat corrected the typographical errors that were drawn to the attention of WRC-15 and which were approved by WRC-15.

This edition uses the same numbering scheme as the 2001 edition of the Radio Regulations, notably:

With respect to Article numbers, this edition follows the standard sequential numbering. The Article numbers are not followed by any abbreviation (such as “(WRC-97)”, “(WRC-2000)”, “(WRC-03)”, “(WRC-07)”, “(WRC-12)” or “(WRC-15)”). Consequently, any reference to an Article, in any of the provisions of these Radio Regulations (e.g. in No. 13.1 of Article 13), in the texts of the Appendices as contained in Volume 2 of this edition (e.g. in § 1 of Appendix 2), in the texts of the Resolutions included in Volume 3 of this edition (e.g. in Resolution 1 (Rev.WRC-97)), and in the texts of the Recommendations included in Volume 3 of this edition (e.g. in Recommendation 8), is considered as a reference to the text of the concerned Article which appears in this edition, unless otherwise specified.

With respect to provision numbers in Articles, this edition continues to use composite numbers indicating the number of the Article and the provision number within that Article (e.g. No. 9.2B means provision No. 2B of Article 9). The abbreviation “(WRC-15)” “(WRC-12)”, “(WRC-07)”, “(WRC-03)”, “(WRC-2000)” or “(WRC-97)” at the end of such a provision means that the relevant provision was modified or added by WRC-15, by WRC-12, by WRC-07, by WRC-03, by WRC-2000 or by WRC-97, as applicable. The absence of an abbreviation at the end of the provision means that the provision is identical with the provision of the simplified Radio Regulations as approved by WRC-95, and whose complete text was contained in Document 2 of WRC-97.

With respect to Appendix numbers, this edition follows the standard sequential numbering, with the addition of the appropriate abbreviation after the Appendix number (such as “(WRC-97)”, “(WRC-2000)”, “(WRC-03)”, “(WRC-07)” “(WRC-12)” or “(WRC-15)”), where applicable. As a rule, any reference to an Appendix, in any of the provisions of these Radio Regulations, in the texts of the Appendices as contained in Volume 2 of this edition, in the texts of the Resolutions and of the Recommendations included in Volume 3 of this edition, is presented in the standard manner (e.g. “Appendix 30 (Rev.WRC-15)”) if not explicitly described in the text (e.g. Appendix 4 as modified by WRC-15). In the texts of Appendices that were partially modified by WRC-15, the provisions that were modified by WRC-15 are indicated with the abbreviation “(WRC-15)” at the end of the concerned text. If an Appendix is referenced without any abbreviation after the Appendix number, in the texts of this edition (e.g., in No. 13.1), or without other description, such reference is considered as a reference to the text of the concerned Appendix which appears in this edition.

Within the text of the Radio Regulations, the symbol, , has been used to represent quantities associated with an uplink. Similarly, the symbol, , has been used to represent quantities associated with a downlink.

Abbreviations have generally been used for the names of world administrative radio conferences and world radiocommunication conferences. These abbreviations are shown below.

| Abbreviation | Conference |
| --- | --- |
| WARC Mar | World Administrative Radio Conference to Deal with Matters Relating to the Maritime Mobile Service (Geneva, 1967) |
| WARC-71 | World Administrative Radio Conference for Space Telecommunications (Geneva, 1971) |
| WMARC-74 | World Maritime Administrative Radio Conference (Geneva, 1974) |
| WARC SAT-77 | World Broadcasting-Satellite Administrative Radio Conference (Geneva, 1977) |
| WARC-Aer2 | World Administrative Radio Conference on the Aeronautical Mobile (R) Service (Geneva, 1978) |
| WARC-79 | World Administrative Radio Conference (Geneva, 1979) |
| WARC Mob-83 | World Administrative Radio Conference for the Mobile Services (Geneva, 1983) |
| WARC HFBC-84 | World Administrative Radio Conference for the Planning of the HF Bands Allocated to the Broadcasting Service (Geneva, 1984) |
| WARC Orb-85 | World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilising It (First Session – Geneva, 1985) |
| WARC HFBC-87 | World Administrative Radio Conference for the Planning of the HF Bands Allocated to the Broadcasting Service (Geneva, 1987) |
| WARC Mob-87 | World Administrative Radio Conference for the Mobile Services (Geneva, 1987) |
| WARC Orb-88 | World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilising It (Second Session – Geneva, 1988) |
| WARC-92 | World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) |
| WRC-95 | World Radiocommunication Conference (Geneva, 1995) |
| WRC-97 | World Radiocommunication Conference (Geneva, 1997) |
| WRC-2000 | World Radiocommunication Conference (Istanbul, 2000) |
| WRC-03 | World Radiocommunication Conference (Geneva, 2003) |
| WRC-07 | World Radiocommunication Conference (Geneva, 2007) |
| WRC-12 | World Radiocommunication Conference (Geneva, 2012) |
| WRC-15 | World Radiocommunication Conference (Geneva, 2015) |
| WRC-19 | World Radiocommunication Conference, 2019 |

VOLUME  2

Appendices

TABLE  OF  CONTENTS

Page

APPENDIX 1	(REV.WRC-12) Classification of emissions and necessary 
bandwidths		3

APPENDIX 2	(REV.WRC-03) Table of transmitter frequency tolerances		9

APPENDIX 3	(REV.WRC-12) Maximum permitted power levels for unwanted emissions in the spurious domain		17

ANNEX 1	Determination of the boundary between the out-of-band and spurious domains		23

APPENDIX 4	(REV.WRC-15) Consolidated list and tables of characteristics for use in the application of the procedures of Chapter III		27

ANNEX 1	Characteristics of stations in the terrestrial 
services		27

ANNEX 2	Characteristics of satellite networks, earth stations or radio astronomy stations		63

APPENDIX 5	(REV.WRC-15) Identification of administrations with which coordination is to be effected or agreement sought under the provisions of Article 9		107

ANNEX 1		125

APPENDIX 7	(REV.WRC-15) Methods for the determination of the coordination area around an earth station in frequency bands between 100 MHz and 105 GHz		135

ANNEX 1	Determination of the required distance for propagation mode (1)		163

ANNEX 2	Determination of the required distance for propagation mode (2)		174

ANNEX 3	Antenna gain towards the horizon for an earth station operating with a geostationary space station		184

Page

ANNEX 4	Antenna gain toward the horizon for an earth station operating with non-geostationary space stations		189

ANNEX 5	Determination of the coordination area for a transmitting earth station with respect to receiving earth stations operating with geostationary space stations in bidirectionally allocated frequency bands		194

ANNEX 6	Supplementary and auxiliary contours		199

ANNEX 7	System parameters and predetermined coordination distances for determination of the coordination area around an earth station		211

APPENDIX 8	(REV.WRC-15) Method of calculation for determining if coordination is required between geostationary-satellite networks sharing the same frequency bands		229

ANNEX 1	Calculation of the topocentric angular separation between two geostationary satellites		236

ANNEX 2	Calculation of the free-space transmission loss		237

ANNEX 3	Radiation patterns for earth station antennas to be used when they are not published		238

ANNEX 4	Example of an application of Appendix 8		239

APPENDIX 9	Report of an irregularity or infringement		243

APPENDIX 10	(REV.WRC-07) Report of harmful interference		247

APPENDIX 11	(REV.WRC-03) System specifications for double-sideband (DSB), single-sideband (SSB) and digitally modulated emissions in the HF broadcasting service		249

APPENDIX 12	Special rules applicable to radiobeacons		255

APPENDIX 14	(REV.WRC-07) Phonetic alphabet and figure code		257

APPENDIX 15	(REV.WRC-15) Frequencies for distress and safety communications for the Global Maritime Distress and Safety System (GMDSS)		259

Page

APPENDIX 16	(REV.WRC-07) Documents with which stations on board ships 
and aircraft shall be provided		263

APPENDIX 17	(REV.WRC-15) Frequencies and channelling arrangements in the high-frequency bands for the maritime mobile service		265

ANNEX 1	Frequencies and channelling arrangements in the high-frequency bands for the maritime mobile service, in force until 31 December 2016		266

ANNEX 2	Frequency and channelling arrangements in the high-frequency bands for the maritime mobile service, which enter into force on 1 January 2017		295

APPENDIX 18	(REV.WRC-15) Table of transmitting frequencies in the VHF maritime mobile band		325

APPENDIX 25	(REV.WRC-03) Provisions and associated frequency allotment Plan for coast radiotelephone stations operating in the exclusive maritime mobile bands between 4 000 kHz and 27 500 kHz		331

APPENDIX 26	(REV.WRC-15) Provisions and associated Frequency Allotment Plan for the aeronautical mobile (OR) service in the bands allocated exclusively to that service between 3 025 kHz and 18 030 kHz		367

APPENDIX 27	(REV.WRC-12) Frequency allotment Plan for the aeronautical mobile (R) service and related information		393

APPENDIX 30	(REV.WRC-15) Provisions for all services and associated Plans and List for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2)		469

ANNEX 1	Limits for determining whether a service of an administration is affected by a proposed modification to the Region 2 Plan or by a proposed new or modified assignment in the Regions 1 and 3 List or when it is necessary under this Appendix to seek the agreement of any other administration		557

ANNEX 2	Basic characteristics to be furnished in notices relating to space stations in the broadcasting-satellite service		562

Page

ANNEX 3	Method for determining the limiting interfering power flux-density at the edge of a broadcasting-satellite service area in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2), and for calculating the power flux-density produced in these bands by a terrestrial station, or by a transmitting earth station in the  fixed-satellite service in the band 12.5-12.7 GHz		562

ANNEX 4	Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Regions 1 and 3; in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with respect to  the Plan or proposed modifications to the Plan in Region 2; in Region 3 (12.2-12.5 GHz) with respect to the Plan, List or proposed new or  modified assignments in the List for Region 1		572

ANNEX 5	Technical data used in establishing the provisions and associated Plans and the Regions 1 and 3 List, which should be used for their application		574

ANNEX 6	Criteria for sharing between services		610

ANNEX 7	Orbital position limitations		616

APPENDIX 30A	(REV.WRC-15) Provisions and associated Plans and List for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3, and 17.3-17.8 GHz in Region 2		619

ANNEX 1	Limits for determining whether a service of an administration is considered to be affected by a proposed modification to the Region 2 feeder-link Plan or by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List or when it is necessary under this Appendix to seek the agreement of any other administration		711

Page

ANNEX 2	Basic characteristics to be furnished in notices relating to feeder-link stations in the fixed-satellite service operating in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz		714

ANNEX 3	Technical data used in establishing the provisions and associated Plans and Regions 1 and 3 feeder-link List, which should be used for their application		714

ANNEX 4	Criteria for sharing between services		754

APPENDIX 30B	(REV.WRC-15) Provisions and associated Plan for the fixed-satellite service in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz, 10.70-10.95 GHz, 11.2-11.45 GHz and 12.75-13.25 GHz		757

ANNEX 1	Parameters used in characterizing the fixed-satellite service allotment Plan		782

ANNEX 2	(SUP - WRC-07)

ANNEX 3	Limits applicable to submissions received under Article 6 or Article 7		786

ANNEX 4	Criteria for determining whether an allotment or an assignment is considered to be affected		787

APPENDIX 1 TO ANNEX 4  Method for determination of the overall single-entry and aggregate carrier-to-interference value averaged over the necessary bandwidth of the modulated carrier		788

APPENDIX 2 TO ANNEX 4  Method for determination of the carrier-to-noise (C/N) values		791

ANNEX 5	(SUP - WRC-07)

ANNEX 6	(SUP - WRC-07)

APPENDIX 42	(REV.WRC-15) Table of allocation of international 
call sign series		793

APPENDICES

APPENDIX 1 (REV.WRC-12)

Classification of emissions and necessary bandwidths

(See Article 2)

§ 1	1)	Emissions shall be designated according to their necessary bandwidth and their classification as explained in this Appendix.

2)	Formulae and examples of emissions designated in accordance with this Appendix are given in Recommendation ITU-R SM.1138-2. Further examples may be provided in other ITU-R Recommendations. These examples may also be published in the Preface to the International Frequency List.     (WRC-12)

Section I – Necessary bandwidth

§ 2	1)	The necessary bandwidth, as defined in No. 1.152 and determined in accordance with the formulae and examples, shall be expressed by three numerals and one letter. The letter occupies the position of the decimal point and represents the unit of bandwidth. The first character shall be neither zero nor K, M or G.

2)	Necessary bandwidths1:

between 0.001 and 999 Hz shall be expressed in Hz (letter H);

between 1.00 and 999 kHz shall be expressed in kHz (letter K);

between 1.00 and 999 MHz shall be expressed in MHz (letter M);

between 1.00 and 999 GHz shall be expressed in GHz (letter G).

3)	For the full designation of an emission, the necessary bandwidth, indicated in four characters, shall be added just before the classification symbols. When used, the necessary bandwidth shall be determined by one of the following methods:

3.1)	use of the formulae and examples of necessary bandwidths and designation of corresponding emissions given in Recommendation ITU-R SM.1138-2;    (WRC-12)

3.2)	computation, in accordance with other ITU-R Recommendations;

3.3)	measurement, in cases not covered by § 3.1) or 3.2) above.

Section II – Classification

§ 3	The class of emission is a set of characteristics conforming to § 4 below.

§ 4	Emissions shall be classified and symbolized according to their basic characteristics as given in Sub-Section IIA and any optional additional characteristics as provided for in Sub-Section IIB.

§ 5	The basic characteristics (see Sub-Section IIA) are:

1)	first symbol – type of modulation of the main carrier;

2)	second symbol – nature of signal(s) modulating the main carrier;

3)	third symbol – type of information to be transmitted.

Modulation used only for short periods and for incidental purposes (such as, in many cases, for identification or calling) may be ignored provided that the necessary bandwidth as indicated is not thereby increased.

Sub-Section IIA – Basic characteristics

§ 6	1)	First symbol – Type of modulation of the main carrier

1.1)	Emission of an unmodulated carrier	N

1.2)	Emission in which the main carrier is amplitude-modulated (including cases
where sub-carriers are angle-modulated)

1.2.1)	Double-sideband	A

1.2.2)	Single-sideband, full carrier	H

1.2.3)	Single-sideband, reduced or variable level carrier	R

1.2.4)	Single-sideband, suppressed carrier	J

1.2.5)	Independent sidebands	B

1.2.6)	Vestigial sideband	C

1.3)	Emission in which the main carrier is angle-modulated

1.3.1)	Frequency modulation	F

1.3.2)	Phase modulation	G

1.4)	Emission in which the main carrier is amplitude-and angle-modulated either 
simultaneously or in a pre-established sequence	D

1.5)	Emission of pulses2

1.5.1)	Sequence of unmodulated pulses	P

1.5.2)	A sequence of pulses

1.5.2.1)	modulated in amplitude	K

1.5.2.2)	modulated in width/duration	L

1.5.2.3)	modulated in position/phase	M

1.5.2.4)	in which the carrier is angle-modulated during the
		angle-period of the pulse	Q

1.5.2.5)	which is a combination of the foregoing or is produced 
		by other means	V

1.6)	Cases not covered above, in which an emission consists of the main carrier
modulated, either simultaneously or in a pre-established sequence, in a
combination of two or more of the following modes: amplitude, angle, pulse	W

1.7)	Cases not otherwise covered	X

2)	Second symbol – Nature of signal(s) modulating the main carrier

2.1)	No modulating signal		0

2.2)	A single channel containing quantized or digital information without the use of a 
modulating sub-carrier3	1

2.3)	A single channel containing quantized or digital information with the use of a 
modulating sub-carrier3	2

2.4)	A single channel containing analogue information	3

2.5)	Two or more channels containing quantized or digital information	7

2.6)	Two or more channels containing analogue information	8

2.7)	Composite system with one or more channels containing quantized or digital 
information, together with one or more channels containing analogue information	9

2.8)	Cases not otherwise covered	X

3)	Third symbol – Type of information to be transmitted4

3.1)	No information transmitted	N

3.2)	Telegraphy – for aural reception	A

3.3)	Telegraphy – for automatic reception	B

3.4)	Facsimile			C

3.5)	Data transmission, telemetry, telecommand	D

3.6)	Telephony (including sound broadcasting)	E

3.7)	Television (video)		F

3.8)	Combination of the above	W

3.9)	Cases not otherwise covered	X

Sub-Section IIB – Optional characteristics for the classification of emissions

§ 7	Two optional characteristics should be added for a more complete description of an emission. These are:

Fourth symbol – Details of signal(s)

Fifth symbol – Nature of multiplexing

Where the fourth or fifth symbol is used it shall be as indicated below.

Where the fourth or the fifth symbol is not used this should be indicated by a dash where each symbol would otherwise appear.

1)	Fourth symbol – Details of signal(s)

1.1)	Two-condition code with elements of differing numbers and/or durations	A

1.2)	Two-condition code with elements of the same number and duration without
error-correction		B

1.3)	Two-condition code with elements of the same number and duration with
error-correction		C

1.4)	Four-condition code in which each condition represents a signal element
(or one or more bits)		D

1.5)	Multi-condition code in which each condition represents a signal element
(of one or more bits)		E

1.6)	Multi-condition code in which each condition or combination of conditions 
represents a character		F

1.7)	Sound of broadcasting quality (monophonic)	G

1.8)	Sound of broadcasting quality (stereophonic or quadraphonic)	H

1.9)	Sound of commercial quality (excluding categories given in § 1.10) and 1.11))	J

1.10)	Sound of commercial quality with the use of frequency inversion or
band-splitting			K

1.11)	Sound of commercial quality with separate frequency-modulated signals to
control the level of demodulated signal	L

1.12)	Monochrome			M

1.13)	Colour				N

1.14)	Combination of the above	W

1.15)	Cases not otherwise covered	X

2)	Fifth symbol – Nature of multiplexing

2.1)	None				N

2.2)	Code-division multiplex5	C

2.3)	Frequency-division multiplex	F

2.4)	Time-division multiplex	T

2.5)	Combination of frequency-division multiplex and time-division multiplex	W

2.6)	Other types of multiplexing	X

APPENDIX 2 (REV.WRC-03)

Table of transmitter frequency tolerances

(See Article 3)

1	Frequency tolerance is defined in Article 1 and is expressed in parts in 106, unless otherwise indicated.

2	The power shown for the various categories of stations is the peak envelope power for single-sideband transmitters and the mean power for all other transmitters, unless otherwise indicated. The term “power of a radio transmitter” is defined in Article 1.

3	For technical and operational reasons, certain categories of stations may need more stringent tolerances than those shown in the table.

| Frequency bands (lower limit exclusive, upper limit inclusive) and categories of stations | Tolerances applicable to transmitters |
| --- | --- |
| Band: 9 kHz to 535 kHz |  |
| 1 Fixed stations: – 9 kHz to 50 kHz – 50 kHz to 535 kHz | 100 50 |
| 2 Land stations: a) Coast stations b) Aeronautical stations | 100 1, 2 100 |
| 3 Mobile stations: a) Ship stations b) Ship’s emergency transmitters c) Survival craft stations d) Aircraft stations | 200 3, 4 500 5 500 100 |
| 4 Radiodetermination stations | 100 |
| 5 Broadcasting stations | 10 Hz |
| Band: 535 kHz to 1 606.5 kHz (1 605 kHz in Region 2) Broadcasting stations | 10 Hz (WRC-03) |
| Band: 1 606.5 kHz (1 605 kHz in Region 2) to 4 000 kHz |  |
| 1 Fixed stations: – power 200 W or less – power above 200 W | 100 7, 8 50 7, 8 |
| 2 Land stations: – power 200 W or less – power above 200 W | 100 1, 2, 7, 9, 10 50 1, 2, 7, 9, 10 |

| Frequency bands (lower limit exclusive, upper limit inclusive) and categories of stations | Tolerances applicable to transmitters |
| --- | --- |
| Band: 1 606.5 kHz (1 605 kHz in Region 2) to 4 000 kHz (cont.) |  |
| 3 Mobile stations: a) Ship stations b) Survival craft stations c) Emergency position-indicating radiobeacons d) Aircraft stations e) Land mobile stations | 40 Hz 3, 4, 12 100 100 100 10 50 13 |
| 4 Radiodetermination stations: – power 200 W or less – power above 200 W | 20 14 10 14 |
| 5 Broadcasting stations | 10 Hz 15 |
| Band: 4 MHz to 29.7 MHz |  |
| 1 Fixed stations: |  |
| a) Single-sideband and independent-sideband emissions: – power 500 W or less – power above 500 W b) Class F1B emissions c) Other classes of emission: – power 500 W or less – power above 500 W | 50 Hz 20 Hz 10 Hz 20 10 |
| 2 Land stations: a) Coast stations | 20 Hz 1, 2, 16 |
| b) Aeronautical stations: – power 500 W or less – power above 500 W c) Base stations | 100 10 50 10 20 7 |
| 3 Mobile stations: a) Ship stations: 1) Class A1A emissions 2) Emissions other than Class A1A b) Survival craft stations c) Aircraft stations d) Land mobile stations | 10 50 Hz 3, 4, 19 50 100 10 40 20 |
| 4 Broadcasting stations | 10 Hz 15, 21 |
| 5 Space stations | 20 |
| 6 Earth stations | 20 |

| Frequency bands (lower limit exclusive, upper limit inclusive) and categories of stations | Tolerances applicable to transmitters |
| --- | --- |
| Band: 29.7 MHz to 100 MHz |  |
| 1 Fixed stations: – power 50 W or less – power above 50 W | 30 20 |
| 2 Land stations | 20 |
| 3 Mobile stations | 20 22 |
| 4 Radiodetermination stations | 50 |
| 5 Broadcasting stations (other than television) | 2 000 Hz 23 |
| 6 Broadcasting stations (television sound and vision) | 500 Hz 24, 25 |
| 7 Space stations | 20 |
| 8 Earth stations | 20 |
| Band: 100 MHz to 470 MHz |  |
| 1 Fixed stations: – power 50 W or less – power above 50 W | 20 26 10 |
| 2 Land stations: a) Coast stations b) Aeronautical stations c) Base stations: – in the band 100-235 MHz – in the band 235-401 MHz – in the band 401-470 MHz | 10 20 28 15 29 7 29 5 29 |
| 3 Mobile stations: a) Ship stations and survival craft stations: – in the band 156-174 MHz – outside the band 156-174 MHz | 10 50 31 |
| b) Aircraft stations c) Land mobile stations: – in the band 100-235 MHz – in the band 235-401 MHz – in the band 401-470 MHz | 30 28 15 29 7 29, 32 5 29, 32 |
| 4 Radiodetermination stations | 50 33 |
| 5 Broadcasting stations (other than television) | 2 000 Hz 23 |
| 6 Broadcasting stations (television sound and vision) | 500 Hz 24, 25 |
| 7 Space stations | 20 |
| 8 Earth stations | 20 |

| Frequency bands (lower limit exclusive, upper limit inclusive) and categories of stations | Tolerances applicable to transmitters |
| --- | --- |
| Band: 470 MHz to 2 450 MHz |  |
| 1 Fixed stations: – power 100 W or less – power above 100 W | 100 50 |
| 2 Land stations | 20 36 |
| 3 Mobile stations | 20 36 |
| 4 Radiodetermination stations | 500 33 |
| 5 Broadcasting stations (other than television) | 100 |
| 6 Broadcasting stations (television sound and vision) in the band 470 MHz to 960 MHz | 500 Hz 24, 25 |
| 7 Space stations | 20 |
| 8 Earth stations | 20 |
| Band: 2 450 MHz to 10 500 MHz |  |
| 1 Fixed stations: – power 100 W or less – power above 100 W | 200 50 |
| 2 Land stations | 100 |
| 3 Mobile stations | 100 |
| 4 Radiodetermination stations | 1 250 33 |
| 5 Space stations | 50 |
| 6 Earth stations | 50 |
| Band: 10.5 GHz to 40 GHz |  |
| 1 Fixed station | 300 |
| 2 Radiodetermination stations | 5 000 33 |
| 3 Broadcasting stations | 100 |
| 4 Space stations | 100 |
| 5 Earth stations | 100 |

Notes in the table of transmitter frequency tolerances

1	For coast station transmitters used for direct-printing telegraphy or for data transmission, the tolerance is:

–	5 Hz for narrow-band phase-shift keying;

–	15 Hz for frequency-shift keying for transmitters in use or installed before 2 January 1992;

–	10 Hz for frequency-shift keying for transmitters installed after 1 January 1992.

2	For coast station transmitters used for digital selective calling, the tolerance is 10 Hz.     (WRC-03)

3	For ship station transmitters used for direct-printing telegraphy or for data transmission, the tolerance is:

–	5 Hz for narrow-band phase-shift keying;

–	40 Hz for frequency-shift keying for transmitters in use or installed before 2 January 1992;

–	10 Hz for frequency-shift keying for transmitters installed after 1 January 1992.

4	For ship station transmitters used for digital selective calling, the tolerance is 10 Hz.     (WRC-03)

5	If the emergency transmitter is used as the reserve transmitter for the main transmitter, the tolerance for ship station transmitters applies.

6	(SUP - WRC-03)

7	For single-sideband radiotelephone transmitters except at coast stations, the tolerance is:

–	50 Hz in the bands 1 606.5 (1 605 Region 2)-4 000 kHz and 4-29.7 MHz, for peak envelope powers of 200 W or less and 500 W or less, respectively;

–	20 Hz in the bands 1 606.5 (1 605 Region 2)-4 000 kHz and 4-29.7 MHz, for peak envelope powers above 200 W and 500 W, respectively.

8	For radiotelegraphy transmitters with frequency-shift keying the tolerance is 10 Hz.

9	For coast station single-sideband radiotelephone transmitters the tolerance is 20 Hz.

10	For single-sideband transmitters operating in the frequency bands 1 606.5 (1 605 Region 2)-4 000 kHz and 4-29.7 MHz which are allocated exclusively to the aeronautical mobile (R) service, the tolerance on the carrier (reference) frequency is:

a)	for all aeronautical stations, 10 Hz;

b)	for all aircraft stations operating on international services, 20 Hz;

c)	for aircraft stations operating exclusively on national services, 50 Hz*.

11	Not used.

12	For A1A emissions the tolerance is 50 × 10−6.

13	For transmitters used for single-sideband radiotelephony or for frequency-shift keying radiotelegraphy the tolerance is 40 Hz.

14	For radiobeacon transmitters in the band 1 606.5 (1 605 Region 2)-1 800 kHz the tolerance is 50 × 10−6.

15	For A3E emissions with carrier power of 10 kW or less the tolerance is 20 × 10−6, 15 × 10−6 and 10 × 10−6 in the bands 1 606.5 (1 605 Region 2)-4 000 kHz, 4-5.95 MHz and 5.95-29.7 MHz respectively.

16	For A1A emissions the tolerance is 10 × 10−6.

17	Not used.

18	Not used.

19	For ship station transmitters in the band 26 175-27 500 kHz, on board small craft, with a carrier power not exceeding 5 W in or near coastal waters and utilizing F3E and G3E emissions, the frequency tolerance is 40 × 10−6.     (WRC-03)

20	The tolerance is 50 Hz for single-sideband radiotelephone transmitters, except for those transmitters operating in the band 26 175-27 500 kHz, and not exceeding a peak envelope power of 15 W, for which the basic tolerance of 40 × 10−6 applies.

21	It is suggested that administrations avoid carrier frequency differences of a few hertz, which cause degradations similar to periodic fading. This could be avoided if the frequency tolerance were 0.1 Hz, a tolerance which would be suitable for single-sideband emissions*.

22	For non-vehicular mounted portable equipment with a transmitter mean power not exceeding 5 W, the tolerance is 40 × 10−6.

23	For transmitters of a mean power of 50 W or less operating at frequencies below 108 MHz a tolerance of 3 000 Hz applies.

24	In the case of television stations of:

–	50 W (vision peak envelope power) or less in the band 29.7-100 MHz;

–	100 W (vision peak envelope power) or less in the band 100-960 MHz;

and which receive their input from other television stations or which serve small isolated communities, it may not, for operational reasons, be possible to maintain this tolerance. For such stations, the tolerance is 2 000 Hz.

For stations of 1 W (vision peak envelope power) or less, this tolerance may be relaxed further to:

–	5 kHz in the band 100-470 MHz;

–	10 kHz in the band 470-960 MHz.

25	For transmitters for system M (NTSC) the tolerance is 1 000 Hz. However, for low power transmitters using this system Note 24 applies.

26	For multi-hop radio-relay systems employing direct frequency conversion the tolerance is 30 × 10−6.

27	Not used.

28	For a channel spacing of 50 kHz the tolerance is 50 × 10−6.

29	These tolerances apply to channel spacings equal to or greater than 20 kHz.

30	Not used.

31	For transmitters used by on-board communication stations a tolerance of 5 × 10−6 shall apply.

32	For non-vehicular mounted portable equipment with a transmitter mean power not exceeding 5 W the tolerance is 15 × 10−6.

33	Where specific frequencies are not assigned to radar stations, the bandwidth occupied by the emissions of such stations shall be maintained wholly within the band allocated to the service and the indicated tolerance does not apply.

34	Not used.

35	Not used.

36	In applying this tolerance administrations should be guided by the latest relevant ITU-R Recommendations.

APPENDIX 3 (REV.WRC-12)

Maximum permitted power levels for unwanted emissions 
in the spurious domain     (WRC-12)

(See Article 3)

1	This Appendix derived indicates the maximum permitted power levels of unwanted emissions in the spurious domain using the values indicated in Table I. The provisions of No. 4.5 apply to unwanted emissions not covered in this Appendix.     (WRC-12)

2	Spurious domain emissions1 from any part of the installation, other than the antenna and its transmission line, shall not have an effect greater than would occur if this antenna system were supplied with the maximum permitted power at the frequency of that emission.     (WRC-12)

3	These levels shall not, however, apply to emergency position-indicating radiobeacon (EPIRB) stations, emergency locator transmitters, ships’ emergency transmitters, lifeboat transmitters, survival craft stations or maritime transmitters when used in emergency situations.

4	For technical or operational reasons, more stringent levels than those specified may be applied to protect specific services in certain frequency bands. The levels applied to protect these services, such as safety and passive services, shall be those agreed upon by the appropriate world radiocommunication conference. More stringent levels may also be fixed by specific agreement between the administrations concerned. Additionally, special consideration of transmitter spurious domain emissions may be required for the protection of safety services, radio astronomy and space services using passive sensors. Information on the levels of interference detrimental to radio astronomy, Earth exploration satellites and meteorological passive sensing is given in the most recent version of Recommendation ITU-R SM.329.     (WRC-12)

5	Spurious domain emission limits for combined radiocommunication and information technology equipment are those for the radiocommunication transmitters.     (WRC-12)

6	The frequency range of the measurement of spurious domain emissions is from 9 kHz to 110 GHz or the second harmonic if higher.     (WRC-03)

7	Except as provided in § 8 and 9 of this Appendix, the spurious domain emission levels are specified in the following reference bandwidths:

–	1 kHz between 9 kHz and 150 kHz

–	10 kHz between 150 kHz and 30 MHz

–	100 kHz between 30 MHz and 1 GHz

–	1 MHz above 1 GHz.     (WRC-03)

8	The reference bandwidth of all space service spurious domain emissions should be 4 kHz.     (WRC-03)

9	For radar systems, the reference bandwidths for specifying spurious domain emission levels should be calculated for each particular system. Thus, for the four general types of radar pulse modulation utilized for radionavigation, radiolocation, acquisition, tracking and other radiodetermination functions, the reference bandwidth values are determined using the following:

–	for a fixed-frequency, non-pulse-coded radar, the reciprocal of the radar pulse length, in seconds (e.g. if the radar pulse length is 1 μs, then the reference bandwidth is 1/(1 μs) = 1 MHz);

–	for a fixed-frequency, phase-coded pulsed radar, the reciprocal of the phase chip length, in seconds (e.g. if the phase-coded chip is 2 μs long, then the reference bandwidth is 1/(2 μs) = 500 kHz);

–	for a frequency modulated (FM) or chirped radar, the square root of the quantity obtained by dividing the chirp bandwidth in MHz by the pulse length, in μs (e.g. if the FM is from 1 250 MHz to 1 280 MHz, i.e. 30 MHz, during the pulse length of 10 μs, then the reference bandwidth is (30 MHz/10 μs)1/2 = 1.73 MHz);

–	for radars operating with multiple waveforms, the reference bandwidth for specifying spurious domain emission levels is determined empirically from observations of the radar emission and is obtained following the guidance given in the most recent version of Recommendation ITU-R M.1177.

In the case of radars, for which the bandwidth, as determined using the method above, is greater than 1 MHz, a reference bandwidth of 1 MHz should be used.     (WRC-03)

10	Guidance regarding the methods of measuring spurious domain emissions is given in the most recent version of Recommendation ITU-R SM.329. The e.i.r.p. method specified in this Recommendation should be used when it is not possible to accurately measure the power supplied to the antenna transmission line, or for specific applications where the antenna is designed to provide significant attenuation in the spurious domain. Additionally, the e.i.r.p. method may need some modification for special cases. Specific guidance regarding the methods of measuring spurious domain emissions from radar systems is given in the most recent version of Recommendation ITU-R M.1177.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth in which spurious domain emissions are measured can be different from the reference bandwidth used for specifying spurious domain emission levels.     (WRC-03)

11	The emission limits of this Appendix apply to all emissions, including harmonic emissions, intermodulation products, frequency conversion products and parasitic emissions, at frequencies in the spurious domain (see Fig. 1). The upper and lower parts of the spurious domain extend outward from a boundary determined using Annex 1.     (WRC-03)

12	For the case of a single satellite operating with more than one transponder in the same service area, and when considering the limits for spurious domain emissions as indicated in § 11 of this Appendix, spurious domain emissions from one transponder may fall on a frequency at which a second, companion transponder is transmitting. In these situations, the level of spurious domain emissions from the first transponder is well exceeded by the fundamental or out-of-band domain emissions of the second transponder. Therefore, the limits of this Appendix should not apply to those emissions of a satellite that fall within either the necessary bandwidth or the out-of-band domain of another transponder on the same satellite, in the same service area (see Fig. 2).     (WRC-03)

Transponders A, B, C and D are operating on the same satellite in the same service area. Transponder A is not required to meet spurious domain emission limits in frequency ranges  and , but is required to meet them in frequency ranges  and .     (WRC-03)

# 13	Examples of applying 43 + 10 log (P) to calculate attenuation requirements

Where specified in relation to mean power, spurious domain emissions are to be at least x dB below the total mean power P, i.e. −x dBc. The power P (W) is to be measured in a bandwidth wide enough to include the total mean power. The spurious domain emissions are to be measured in the reference bandwidths given in the relevant ITU-R Recommendations. The measurement of the spurious domain emission power is independent of the value of necessary bandwidth. Because the absolute emission power limit, derived from 43 + 10 log (P), can become too stringent for high-power transmitters, alternative relative powers are also provided in Table I.

Example 1

A land mobile transmitter, with any value of necessary bandwidth, must meet a spurious domain emission attenuation of 43 + 10 log (P), or 70 dBc, whichever is less stringent. The reference bandwidths used for specifying spurious domain emission levels are provided in § 8 to 10 of this Appendix. Applying this in the frequency range between 30 MHz and 1 GHz gives a reference bandwidth of 100 kHz.

With a measured total mean power of 10 W:

–	Attenuation relative to total mean power = 43 + 10 log (10) = 53 dBc.

–	The 53 dBc value is less stringent than the 70 dBc, so the 53 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 53 dBc in a 100 kHz bandwidth, or converting to an absolute level, they must not exceed 10 dBW − 53 dBc = −43 dBW in a 100 kHz reference bandwidth.

With a measured total mean power of 1 000 W:

–	Attenuation relative to total mean power = 43 + 10 log (1 000) = 73 dBc.

–	The 73 dBc value is more stringent than the 70 dBc limit, so the 70 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 70 dBc in a 100 kHz bandwidth, or converting to an absolute level, they must not exceed 30 dBW − 70 dBc = −40 dBW in a 100 kHz reference bandwidth.     (WRC-03)

Example 2

A space service transmitter with any value of necessary bandwidth must meet a spurious domain emission attenuation of 43 + 10 log (P), or 60 dBc, whichever is less stringent. To measure spurious domain emissions at any frequency, Note 10 to Table I indicates using a reference bandwidth of 4 kHz.

With a measured total mean power of 20 W:

–	Attenuation relative to total mean power = 43 + 10 log (20) = 56 dBc.

–	The 56 dBc value is less stringent than the 60 dBc limit, so the 56 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 56 dBc in a 4 kHz reference bandwidth, or converting to an absolute level, they must not exceed 13 dBW − 56 dBc = −43 dBW in a 4 kHz reference bandwidth.     (WRC-03)

TABLE I     (WRC-12)

Attenuation values used to calculate maximum permitted 
spurious domain emission power levels for 
use with radio equipment

| Service category in accordance with Article 1, or equipment type15 | Attenuation (dB) below the power supplied to the antenna transmission line |
| --- | --- |
| All services except those services quoted below: | 43 + 10 log (P), or 70 dBc, whichever is less stringent |
| Space services (earth stations)10, 16 | 43 + 10 log (P), or 60 dBc, whichever is less stringent |
| Space services (space stations)10, 17 | 43 + 10 log (P), or 60 dBc, whichever is less stringent |
| Radiodetermination14 | 43 + 10 log (PEP), or 60 dB, whichever is less stringent |
| Broadcast television11 | 46 + 10 log (P), or 60 dBc, whichever is less stringent, without exceeding the absolute mean power level of 1 mW for VHF stations or 12 mW for UHF stations. However, greater attenuation may be necessary on a case by case basis |
| Broadcast FM | 46 + 10 log (P), or 70 dBc, whichever is less stringent; the absolute mean power level of 1 mW should not be exceeded |
| Broadcasting at MF/HF | 50 dBc; the absolute mean power level of 50 mW should not be exceeded |
| SSB from mobile stations12 | 43 dB below PEP |
| Amateur services operating below 30 MHz (including those using SSB)16 | 43 + 10 log (PEP), or 50 dB, whichever is less stringent |

Table 1 (end)     (WRC-12)

| Service category in accordance with Article 1, or equipment type15 | Attenuation (dB) below the power supplied to the antenna transmission line |
| --- | --- |
| Services operating below 30 MHz, except space, radiodetermination, broadcast, those using SSB from mobile stations, and amateur12 | 43 + 10 log (X), or 60 dBc, whichever is less stringent, where X = PEP for SSB modulation, and X = P for other modulation |
| Low-power device radio equipment13 | 56 + 10 log (P), or 40 dBc, whichever is less stringent |
| Emergency transmitters18 | No limit |

P: mean power in watts supplied to the antenna transmission line, in accordance with No. 1.158. When burst transmission is used, the mean power P and the mean power of any spurious domain emissions are measured using power averaging over the burst duration. PEP: peak envelope power in watts supplied to the antenna transmission line, in accordance with No. 1.157. dBc: decibels relative to the unmodulated carrier power of the emission. In the cases which do not have a carrier, for example in some digital modulation schemes where the carrier is not accessible for measurement, the reference level equivalent to dBc is decibels relative to the mean power P. 10 Spurious domain emission limits for all space services are stated in a 4 kHz reference bandwidth. 11 For analogue television transmissions, the mean power level is defined with a specified video signal modulation. This video signal has to be chosen in such a way that the maximum mean power level (e.g. at the video signal blanking level for negatively modulated television systems) is supplied to the antenna transmission line. 12 All classes of emission using SSB are included in the category “SSB”. 13 Low-power radio devices having a maximum output power of less than 100 mW and intended for short-range communication or control purposes; such equipment is in general exempt from individual licensing. 14 For radiodetermination systems (radar as defined by No. 1.100), spurious domain emission attenuation (dB) shall be determined for radiated emission levels, and not at the antenna transmission line. The measurement methods for determining the radiated spurious domain emission levels from radar systems should be guided by the most recent version of Recommendation ITU-R M.1177. (WRC-03) 15 In some cases of digital modulation (including digital broadcasting), broadband systems, pulsed modulation and narrow-band high-power transmitters for all categories of services, there may be difficulties in meeting limits close to 250% of the necessary bandwidth. 16 Earth stations in the amateur-satellite service operating below 30 MHz are in the service category “Amateur services operating below 30 MHz (including those using SSB)”. (WRC-2000) 17 Space stations in the space research service intended for operation in deep space as defined by No. 1.177 are exempt from spurious domain emission limits. (WRC-03) 18 Emergency position-indicating radio beacon, emergency locator transmitters, personal location beacons, search and rescue transponders, ship emergency, lifeboat and survival craft transmitters and emergency land, aeronautical or maritime transmitters. (WRC-2000)

ANNEX 1     (WRC-03)

Determination of the boundary between the 
out-of-band and spurious domains

1	Except as provided below, the boundary between the out-of-band and spurious domains occurs at frequencies that are separated from the centre frequency of the emission by the values shown in Table 1. In general, the boundary, on either side of the centre frequency, occurs at a separation of 250% of the necessary bandwidth, or at 2.5 BN, as shown in Table 1. For most systems, the centre frequency of the emission is the centre of the necessary bandwidth. For multichannel or multicarrier transmitters/transponders, where several carriers may be transmitted simultaneously from a final output amplifier or an active antenna, the centre frequency of the emission is taken to be the centre of the −3 dB bandwidth of the transmitter or transponder, and the transmitter or transponder bandwidth is used in place of the necessary bandwidth for determining the boundary. For multicarrier satellite systems, guidance on the boundary between the out-of-band and spurious domains is provided in the most recent version of Recommendation ITU-R SM.1541. Some systems specify unwanted emissions relative to channel bandwidth, or channel spacing. These may be used as a substitute for the necessary bandwidth in Table 1, provided they are found in ITU-R Recommendations.

TABLE 1

Values for frequency separation between the centre frequency 
and the boundary of the spurious domain

| Frequency range | Narrow-band case |  | Normal separation | Wideband case |  |
| --- | --- | --- | --- | --- | --- |
|  | for BN < | Separation |  | for BN > | Separation |
| 9 kHz < fc 150 kHz | 250 Hz | 625 Hz | 2.5 BN | 10 kHz | 1.5 BN + 10 kHz |
| 150 kHz < fc 30 MHz | 4 kHz | 10 kHz | 2.5 BN | 100 kHz | 1.5 BN + 100 kHz |
| 30 MHz < fc 1 GHz | 25 kHz | 62.5 kHz | 2.5 BN | 10 MHz | 1.5 BN + 10 MHz |
| 1 GHz < fc 3 GHz | 100 kHz | 250 kHz | 2.5 BN | 50 MHz | 1.5 BN + 50 MHz |
| 3 GHz < fc 10 GHz | 100 kHz | 250 kHz | 2.5 BN | 100 MHz | 1.5 BN + 100 MHz |
| 10 GHz < fc 15 GHz | 300 kHz | 750 kHz | 2.5 BN | 250 MHz | 1.5 BN + 250 MHz |
| 15 GHz < fc 26 GHz | 500 kHz | 1.25 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| fc > 26 GHz | 1 MHz | 2.5 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| NOTE – In Table 1, fc is the centre frequency of the emission and BN is the necessary bandwidth. If the assigned frequency band of the emissions extends across two frequency ranges, then the values corresponding to the higher frequency range shall be used for determining the boundary. |  |  |  |  |  |

Example 1: The necessary bandwidth of an emission at 26 MHz is 1.8 kHz. Since BN  is less than 4 kHz, the minimum separation of 10 kHz applies. The spurious domain begins 10 kHz each side of the centre of the necessary bandwidth.

Example 2: The necessary bandwidth of an emission at 8 GHz is 200 MHz. Since the wideband case applies for BN  > 100 MHz at that frequency, the spurious domain begins 1.5 × 200 MHz + 100 MHz = 400 MHz each side of the centre of the necessary bandwidth. Using the general separation formula, the out-of-band domain would have extended to 2.5 × 200 MHz = 500 MHz either side of the centre frequency.

2	Tables 2 and 3 show exceptions to Table 1 for narrow-band and wideband cases, respectively, applicable to particular systems or services and frequency bands.

TABLE 2

Narrow-band variations for particular systems or services and frequency bands

| System or service | Frequency range |  | Narrow-band case |  |
| --- | --- | --- | --- | --- |
|  |  |  | for BN < (kHz) | Separation (kHz) |
| Fixed service | 14 kHz-1.5 MHz |  | 20 | 50(1) |
|  | 1.5-30 MHz | PT ≤ 50 W | 30 | 75(2) |
|  |  | PT > 50 W | 80 | 200(2) |
| (1) The separation value is based on an assumption that the maximum value of the necessary bandwidth is about 3 kHz for the frequency range 14 kHz-1.5 MHz. The separation value of 50 kHz is extremely large as compared with the necessary bandwidth. This is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit (70 dBc) at the boundary between the out-of-band and spurious domains. (2) PT is the transmitter power. The separation values are based on an assumption that the maximum value of the necessary bandwidth is about 12 kHz for the frequency range 1.5-30 MHz. The separation value of 200 kHz for PT > 50 W is extremely large as compared with the necessary bandwidth. This is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit, 70 dBc, at the boundary between the out-of-band and spurious domains. Also, if future systems in the fixed service operating in this frequency range require a necessary bandwidth larger than 12 kHz, it may become necessary to review the 200 kHz separation. |  |  |  |  |

TABLE 3

Wideband variations for particular systems or services and frequency bands

| System or service | Frequency range | Wideband case |  |
| --- | --- | --- | --- |
|  |  | For BN > | Separation |
| Fixed service | 14-150 kHz | 20 kHz | 1.5 BN + 20 kHz |
| Fixed-satellite service (FSS) | 3.4-4.2 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 5.725-6.725 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 7.25-7.75 GHz and 7.9-8.4 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 10.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| Broadcasting-satellite service | 11.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 12.75-13.25 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 13.75-14.8 GHz | 500 MHz | 1.5 BN + 500 MHz |

3	For primary radar, the boundary between the out-of-band and spurious domains is the frequency at which the out-of-band domain limits specified in the applicable ITU-R Recommendations are equal to the spurious domain limit defined in Table I of this Appendix. Further guidance on the boundary between the out-of-band and spurious domains for primary radar is provided in the most recent version of Recommendation ITU-R SM.1541.

APPENDIX 4 (REV.WRC-15)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

1	The substance of this Appendix is separated into two parts: one concerning data and their use for terrestrial radiocommunication services and another concerning data and their use for space radiocommunication services or the radio astronomy service.     (WRC-12)

2	Both parts contain a list of characteristics and a table indicating the use of each of the characteristics in specific circumstances.

Annex 1:	Characteristics of stations in the terrestrial services

Annex 2:	Characteristics of satellite networks, earth stations or radio astronomy stations.

ANNEX 1

Characteristics of stations in the terrestrial services1

In application of Appendix 4 there are many cases when the data requirements involve the use of standard symbols in submissions to the Radiocommunication Bureau. These standard symbols may be found in the Preface to the BR International Frequency Information Circular (BR IFIC) (Terrestrial Services). In the Tables, this is referred to simply as “the Preface”. Also additional information may be found in the guidelines published on the Bureau’s website.

Key to the symbols used in Annex 1

| X | Mandatory information |
| --- | --- |
| + | Mandatory under the conditions specified in Column 3 of Table 1 and Column 2 of Table 2 |
| O | Optional information |
| C | Mandatory if used as a basis to effect coordination with another administration |
|  | The data item is not applicable to the corresponding notice |

Reading Appendix 4 Tables 1 and 2

The rules used to link the sign with the text are based on the Table column headings covering specific procedures, services and frequency bands.

1	If any data item has the indication “+”, it shows that the data item is subject to a mandatory requirement under specific conditions. If these conditions are not met, the corresponding item is not applicable unless otherwise specified. These conditions are listed after the data item name and are normally presented as shown below.

2	“Required” without any reference to a column heading is used in the case that the associated condition is valid for every applicable column.

| 1.5.2 | 1B | the reference frequency, as defined in Article 1 |  | + | + |  | 1B |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Required if the modulation envelope is asymmetric |  |  |  |  |  |

“In the case of”, followed by a reference to the column heading is used, as shown below, when the associated conditions are different for individual columns, or if the indication is not the same across all applicable columns.

| 7.1 | 7A | the class of emission | + | X |  | 7A |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | In the case of a VHF/UHF broadcasting station, required for assignments subject to § 5.1.3 of the GE06 Regional Agreement |  |  |  |  |

3	A subheading title limits the range of procedures, services or frequency bands to which the data items grouped under that subheading are applicable. Unless further specific conditions apply, the data items grouped under that subheading have an “X” as the conditional nature is shown in the subheading title.     (WRC-12)

| 1.4.3 |  | For assignments in the bands and services governed by the Geneva 06 Regional Agreement only |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4.3.4 | DAC | the digital broadcasting assignment code | X |  |  | DAC |

Footnotes to Tables 1 and 2

1	The most recent version of Recommendation ITU-R SF.675 should be used to the extent applicable in calculating the maximum power density per Hz.

TABLE 1     (Rev.WRC-15)

Characteristics for terrestrial services

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | GENERAL INFORMATION AND FREQUENCY CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | B | the symbol of the notifying administration (see the Preface) |  |  |  |  | X | X | X | X | X | X | X | B |
| 1.2 | D | the provision code of the Radio Regulations under which the notice has been submitted |  |  |  |  | X | X | X | X | X | X | X | D |
| 1.3 | E | the resubmission indicator In the case of a VHF/UHF broadcasting station, or a typical transmitting station, required for an assignment subject to the GE06 Regional Agreement if the notice is resubmitted in the application of Article 11 In the case of a transmitting station, or a receiving land station, required for an assignment subject to the GE06 Regional Agreement or Nos. 9.16, 9.18 or 9.19 if the notice is resubmitted in the application of Article 11 |  |  |  |  | + |  | + | + | + |  |  | E |
| 1.4 |  | Assignment and allotment identification information |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.4.1 | SYNC | the identification symbols for the synchronized, or single-frequency, network In the case of a VHF/UHF broadcasting station, required for a digital broadcasting assignment in a synchronized or single frequency network subject to the GE06 Regional Agreement In the case of an LF/MF broadcasting station, required for an assignment in a synchronized or single frequency network |  |  |  |  | + | + |  |  |  |  |  | SYNC |
| 1.4.2 | ID1 | the unique identification code given by the administration to the assignment or allotment Required for assignments subject to the GE06 Regional Agreement, and optional for assignments not subject to this Agreement |  |  |  |  | + | O | + | + | + | O |  | ID1 |
| 1.4.3 |  | For assignments in the bands and services governed by the GE06 Regional Agreement only: |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.4.3.1 | ID2 | the unique identification code given by the administration for the associated allotment Required for a digital broadcasting assignment linked to an allotment, or converted from an allotment, within the GE06 Plan |  |  |  |  | + |  |  |  |  |  |  | ID2 |
| 1.4.3.2 | ID3 | the unique identification code given by the administration to the digital broadcasting Plan entry for which § 5.1.3 of the GE06 Agreement is to be applied Required if the notified assignment is to be operated under the mask of a digital broadcasting Plan entry in accordance with § 5.1.3 of the GE06 Regional Agreement |  |  |  |  | + |  | + | + |  |  |  | ID3 |
| 1.4.3.3 | DEC | the digital broadcasting plan entry code that identifies the category of Plan entry to which the assignment belongs |  |  |  |  | X |  |  |  |  |  |  | DEC |
| 1.4.3.4 | DAC | the digital broadcasting assignment code |  |  |  |  | X |  |  |  |  |  |  | DAC |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 |  | Frequency information |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.5.1 | 1A | the assigned frequency, as defined in Article 1 In the case of a transmitting station, required for all services, except adaptive systems in the fixed or mobile service operating in the bands between 300 kHz and 28 MHz (see also Resolution 729 (Rev.WRC-07)) In the case of an HF broadcasting station under Article 12, required if neither the preferred band nor reference frequency is provided |  |  |  |  | X | X | + | X | X |  | + | 1A |
| 1.5.2 | 1B | the reference frequency, as defined in Article 1 Required if the modulation envelope is asymmetric |  |  |  |  |  |  | + | + | + |  | + | 1B |
| 1.5.3 | 1G | the alternative frequency |  |  |  |  |  |  |  |  |  |  | O | 1G |
| 1.5.4 | 1X | the channel number of the proposed or allotted channel Required for submissions in accordance with Nos. 25/1.1.1, 25/1.1.2 or 25/1.25 of Appendix 25 if the assistance of the Bureau is not requested under No. 25/1.3.1 of Appendix 25 |  |  |  |  |  |  |  |  |  | + |  | 1X |
| 1.5.5 | 1Y | the channel number of the alternative proposed channel |  |  |  |  |  |  |  |  |  | O |  | 1Y |
| 1.5.6 | 1Z | the channel number of the channel to be replaced Required if the administration needs to replace its existing allotted channel |  |  |  |  |  |  |  |  |  | + |  | 1Z |
| 1.5.7 | 1AA | the lower limit of the usable frequency range within which the carrier and the bandwidth of the emission will be located Required for adaptive systems in the fixed or mobile service operating in the bands between 300 kHz and 28 MHz (see also Resolution 729 (Rev.WRC-07)) |  |  |  |  |  |  | + |  |  |  |  | 1AA |
| 1.5.8 | 1AB | the upper limit of the usable frequency range within which the carrier and the bandwidth of the emission will be located Required for adaptive systems in the fixed or mobile service operating in the bands between 300 kHz and 28 MHz (see also Resolution 729 (Rev.WRC-07)) |  |  |  |  |  |  | + |  |  |  |  | 1AB |
| 1.5.9 | 1C | the preferred band, in MHz In the case of maritime mobile frequency allotment, required if the assistance of the Bureau is requested under No. 25/1.3.1 of Appendix 25 In the case of an HF broadcasting station under Article 12, required for notices if assistance is requested in accordance with No. 7.6 |  |  |  |  |  |  |  |  |  | + | + | 1C |
| 1.5.10 |  | For digital broadcasting (except assignments subject to § 5.1.3 of the GE06 Regional Agreement): |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.5.10.1 | 1EO | the frequency offset, in kHz Required if the centre frequency of the emission is offset from the assigned frequency |  |  |  |  | + |  |  |  |  |  |  | 1EO |
| 1.5.11 |  | For analogue television broadcasting: |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.5.11.1 | 1E | the vision carrier frequency offset, in multiples of 1/12 of the line frequency of the television system concerned, expressed by a number (positive or negative) Required if the vision carrier frequency offset, in kHz, (1E1) is not provided for assignments subject to the ST61, GE89 or GE06 Regional Agreements |  |  |  |  | + |  |  |  |  |  |  | 1E |
| 1.5.11.2 | 1E1 | the vision carrier frequency offset, in kHz, expressed by a number (positive or negative) Required if the vision carrier frequency offset, in multiples of 1/12 of the line frequency (1E) is not provided for assignments subject to the ST61, GE89 or GE06 Regional Agreements |  |  |  |  | + |  |  |  |  |  |  | 1E1 |
| 1.5.11.3 |  | For the case where the sound carrier frequency offset is different from the vision carrier frequency offset: |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.5.11.3.1 | 1EA | the sound carrier frequency offset, in multiples of 1/12 of the line frequency of the television system concerned, expressed by a number (positive or negative) Required if the sound carrier frequency offset, in kHz, (1E1A) is not provided for assignments subject to the ST61, GE89 or GE06 Regional Agreements |  |  |  |  | + |  |  |  |  |  |  | 1EA |
| 1.5.11.3.2 | 1E1A | the sound carrier frequency offset, in kHz, expressed by a number (positive or negative) Required if the sound carrier frequency offset, in multiples of 1/12 of the line frequency (1EA) is not provided for assignments subject to the ST61, GE89 or GE06 Regional Agreements |  |  |  |  | + |  |  |  |  |  |  | 1E1A |
| 2 |  | DATE OF OPERATION |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 2C | the date (actual or foreseen, as appropriate) of bringing the frequency assignment (new or modified) into use |  |  |  |  | X | X | X | X | X | X |  | 2C |
| 2.2 | 2E | the date for the end of operation of a frequency assignment In the case of a VHF/UHF broadcasting station, required, in the application of Article 11, when the operation of an assignment is limited to a specific period of time under § 4.1.5.4 of the GE06 Regional Agreement In the case of a transmitting station, a receiving land station, or a typical transmitting station, required, in the application of Article 11, when the operation of an assignment is limited to a specific period of time under § 4.2.5.5 of the GE06 Regional Agreement |  |  |  |  | + |  | + | + | + |  |  | 2E |
| 2.3 | 2F | the season of operation code |  |  |  |  |  |  |  |  |  |  | X | 2F |
| 2.4 | 10CA | the start date for the transmission |  |  |  |  |  |  |  |  |  |  | X | 10CA |
| 2.5 | 10CB | the stop date for the transmission |  |  |  |  |  |  |  |  |  |  | X | 10CB |
| 2.6 | 10CC | the days of operation for the transmission during the HFBC schedule |  |  |  |  |  |  |  |  |  |  | X | 10CC |
| 3 |  | CALL SIGN AND STATION IDENTIFICATION |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | 3A1 | the call sign used in accordance with Article 19 In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (Rev.WRC-12)), or standard frequency and time signal service, in application of Article 11, required if the station identification (3A2) is not provided |  |  |  |  | O | O | + |  |  |  | O | 3A1 |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.2 | 3A2 | the station identification used in accordance with Article 19 In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (Rev.WRC-12)), or standard frequency and time signal service, in application of Article 11, required if the call sign (3A1) is not provided |  |  |  |  | O | O | + |  |  |  | O | 3A2 |
| 4 |  | LOCATION OF THE TRANSMITTING ANTENNA(S) |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | 4A | the name of the locality by which the transmitting station is known or in which it is situated |  |  |  |  | X | X | X |  |  |  |  | 4A |
| 4.2 | 4AA | the name of the location of the intended coast station Required for submissions in accordance with No. 25/1.1.1 of Appendix 25 |  |  |  |  |  |  |  |  |  | + |  | 4AA |
| 4.3 | 4B | the code of the geographical area in which the transmitting station is located (see the Preface) |  |  |  |  | X | X | X |  |  |  |  | 4B |
| 4.4 | 4C | the geographical coordinates of the transmitter site Latitude and longitude are provided in degrees, minutes and seconds |  |  |  |  | X | X | X |  |  |  |  | 4C |
| 4.5 | 4CA | the geographical coordinates of the intended coast station Latitude and longitude are provided in degrees, minutes and seconds Required for submissions in accordance with No. 25/1.1.1 of Appendix 25 |  |  |  |  |  |  |  |  |  | + |  | 4CA |
| 4.6 | 4H | HFBC site code Note – The code is assigned by the Bureau prior to commencement of the Article 12 procedure and represents the location of the station, its geographical area and geographical coordinates |  |  |  |  |  |  |  |  |  |  | X | 4H |
| 4.7 |  | For an area in which transmitting stations operate: |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.1 | 4CC | the geographical coordinates of the centre of the circular zone, in which mobile transmitting stations associated with a receiving land station, or a typical transmitting station are operating Latitude and longitude are provided in degrees, minutes and seconds In the case of a receiving land station, required: – for the maritime radionavigation service; and – for other services if the code of a geographical area or standard defined area (4E) is not provided In the case of a typical transmitting station, required if a geographical area or standard defined area (4E) is not provided |  |  |  |  |  |  |  | + | + |  |  | 4CC |
| 4.7.2 | 4D | the nominal radius, in km, of the circular zone, in which mobile transmitting stations associated with a receiving land station, or a typical transmitting station are operating In the case of a receiving land station, required: – for the maritime radionavigation service; and – for other services if the code of a geographical area or standard defined area (4E) is not provided In the case of a typical transmitting station, required if a geographical area or standard defined area (4E) is not provided |  |  |  |  |  |  |  | + | + |  |  | 4D |
| 4.7.3 | 4E | the code of the geographical area or standard defined area (see the Preface) Note – The standard defined area for a receiving land station in the maritime mobile service may be a maritime zone. The standard defined area for a maritime mobile frequency allotment is the allotment area In the case of a receiving land station, for all services, except the maritime radionavigation service, required if a circular zone (4CC and 4D) is not provided In the case of a typical transmitting station, required if a circular zone (4CC and 4D) is not provided |  |  |  |  |  |  |  | + | + | X |  | 4E |
| 4.8 | 4G | the ground conductivity Required for an assignment subject to the GE75 Regional Agreement |  |  |  |  |  | + |  |  |  |  |  | 4G |
| 5 |  | LOCATION OF THE RECEIVING ANTENNA(S) |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | 5A | the name of the locality by which the receiving station is known or in which it is situated In the case of a transmitting station, required for an associated receiving station in the fixed service if the geographical coordinates of a given reception zone (5CA) are not provided |  |  |  |  |  |  | + | X |  |  |  | 5A |
| 5.2 | 5B | the code of the geographical area in which the receiving station(s) is located (see the Preface) In the case of a transmitting station, required for an associated receiving station in the fixed service if the geographical coordinates of a given reception zone (5CA) are not provided |  |  |  |  |  |  | + | X |  |  |  | 5B |
| 5.3 | 5C | the geographical coordinates of the site of the receiving station Latitude and longitude are provided in degrees, minutes and seconds In the case of a transmitting station, required for an associated receiving station in the fixed service if the geographical coordinates of a given reception zone (5CA) are not provided |  |  |  |  |  |  | + | X |  |  |  | 5C |
| 5.4 |  | For an area in which receiving stations operate: |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4.1 | 5CA | the geographical coordinates of a given reception zone A minimum of 3 geographical coordinates are to be provided. All geographical coordinates (latitude and longitude) are provided in degrees, minutes and seconds For an associated receiving station in the fixed service, required if the name of the locality (5A), geographical area (5B) and geographical coordinates (5C) are not provided For all other services, except where the assignment is subject to the GE06 Agreement, required if neither a circular area (5E and 5F) nor a geographical area or standard defined area of reception (5D) is provided |  |  |  |  |  |  | + |  |  |  |  | 5CA |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4.2 | 5D | the code of the geographical area or standard defined area of reception (see the Preface) Note – The standard defined area of a transmitting station may be represented by a maritime zone or aeronautical zone. The standard defined area of a maritime mobile frequency allotment is a maritime zone. The standard defined area of an HF broadcasting station subject to Article 12 is represented by a CIRAF zone In the case of a transmitting station, except transmitting stations in the fixed service, maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement, required if neither a circular receiving area (5E and 5F) nor geographical coordinates of a given reception zone (5CA) is provided |  |  |  |  |  |  | + |  |  | X | X | 5D |
| 5.4.3 | 5E | the geographical coordinates of the centre of the circular receiving area Latitude and longitude are provided in degrees, minutes and seconds Required: – for the maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement; and – for all other services, except the fixed service, if neither a geographical area or standard defined area of reception (5D) nor the geographical coordinates of a given reception zone (5CA) is provided |  |  |  |  |  |  | + |  |  |  |  | 5E |
| 5.4.4 | 5F | the radius, in km, of the circular receiving area Required: – for the maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement; and – for all other services, except the fixed service, if neither the geographical area or standard defined area of reception (5D) nor the geographical coordinates of a given reception zone (5CA) is provided |  |  |  |  |  |  | + |  |  |  |  | 5F |
| 5.5 | 5G | the maximum length of the circuit, in km, for non-circular receiving areas Stations in the HF bands only |  |  |  |  |  |  | O |  |  | O |  | 5G |
| 6 |  | CLASS OF STATION AND NATURE OF SERVICE |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | 6A | the class of station, using the symbols from the Preface |  |  |  |  | X | X | X | X | X | X | X | 6A |
| 6.2 | 6B | the nature of service, using the symbols from the Preface In the case of a transmitting station, required for all services, except the broadcasting service |  |  |  |  |  |  | + | X | X | X |  | 6B |
| 7 |  | CLASS OF EMISSION AND NECESSARY BANDWIDTH (in accordance with Article 2 and Appendix 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | 7A | the class of emission In the case of a VHF/UHF broadcasting station, required for digital broadcasting assignments |  |  |  |  | + | X | X | X | X | X |  | 7A |
| 7.2 | 7AB | the necessary bandwidth In the case of a VHF/UHF broadcasting station, required for analogue sound and digital broadcasting assignments |  |  |  |  | + | X | X | X | X | X | X | 7AB |
| 7.3 |  | System characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.1 | 7A1 | the code describing the frequency stability (RELAXED, NORMAL or PRECISION) Required for analogue television broadcasting |  |  |  |  | + |  |  |  |  |  |  | 7A1 |
| 7.3.2 | 7AA | the code for the type of modulation The type of modulation denotes the use of DSB, SSB or any new modulation techniques recommended by ITU-R |  |  |  |  |  |  |  |  |  |  | X | 7AA |
| 7.3.3 | 7B1 | the adjacent channel protection ratio, in dB Required for the GE75 Regional Agreement |  |  |  |  |  | + |  |  |  |  |  | 7B1 |
| 7.3.4 | 7B2 | the “RJ 81 class” (A, B or C) Required for the RJ81 Regional Agreement |  |  |  |  |  | + |  |  |  |  |  | 7B2 |
| 7.3.5 | 7G | the system code Note – The code identifies the category of system to which the station belongs and hence its protection requirements In the VHF band two codes are required for protection from T-DAB and DVB-T In the UHF band only one code is required for protection from DVB-T Required for an assignment subject to the GE06 Regional Agreement |  |  |  |  |  |  | + | + | + |  |  | 7G |
| 7.3.6 | 7C1 | the code identifying the television system (see the Preface) Required for television broadcasting assignments, except assignments subject to § 5.1.3 of the GE06 Regional Agreement |  |  |  |  | + |  |  |  |  |  |  | 7C1 |
| 7.3.7 | 7C2 | the code corresponding to the colour system (see the Preface) Required for analogue television broadcasting |  |  |  |  | + |  |  |  |  |  |  | 7C2 |
| 7.3.8 | 7D | the code corresponding to the sound broadcasting transmission system (see the Preface) Note – For LF/MF systems, the signal may consist of analogue or digital modulation or data or some combination of them: the latter case is referred to as hybrid modulation In the case of a VHF/UHF broadcasting station, required for sound broadcasting assignments, except assignments subject to the GE06 Regional Agreement In the case of an LF/MF broadcasting station, required for an assignment with digital or hybrid modulation |  |  |  |  | + | + |  |  |  |  |  | 7D |
| 7.3.9 |  | For the GE06 Regional Agreement (except notices subject to § 5.1.3 of the GE06 Regional Agreement): |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.9.1 | 7H | the reference planning configuration (see the Preface) Required for digital sound broadcasting |  |  |  |  | + |  |  |  |  |  |  | 7H |
| 7.3.9.2 | 7J | the type of spectrum mask |  |  |  |  | X |  |  |  |  |  |  | 7J |
| 7.3.9.3 | 7K | the reception mode (see the Preface) Required for digital television broadcasting |  |  |  |  | + |  |  |  |  |  |  | 7K |
| 7.3.10 |  | For the fixed service in the bands shared with space services and any type of modulation as applicable: |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.10.1 | 7E | the peak to peak frequency deviation, in MHz |  |  |  |  |  |  | C |  |  |  |  | 7E |
| 7.3.10.2 | 7F | the sweep frequency, in kHz, of the energy dispersal waveform |  |  |  |  |  |  | C |  |  |  |  | 7F |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | POWER CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | 8 | the symbol (X, Y or Z, as appropriate) describing the type of power (see Article 1) corresponding to the class of emission |  |  |  |  | X | X | X | X | X | X | X | 8 |
| 8.2 | 8A | the power delivered to the antenna transmission line, in kW |  |  |  |  |  | X |  |  |  |  | X | 8A |
| 8.3 | 8AA | the power delivered to the antenna, in dBW In the case of a transmitting station, required for an assignment: – in the bands below 28 MHz, in all services except the radionavigation service; or – in the bands above 28 MHz shared with space services; or – in the bands above 28 MHz not shared with space services: • in the aeronautical mobile service, meteorological aids service; or • in all other services, if the radiated power is not supplied In the case of a receiving land station, required if the associated transmitting station’s radiated power is not supplied In the case of a typical transmitting station, required if the radiated power is not supplied |  |  |  |  |  |  | + | + | + | X |  | 8AA |
| 8.4 | 8AB | the maximum power density1 (dB(W/Hz)) for each carrier type averaged over the worst 4 kHz band for carriers below 15 GHz, or averaged over the worst 1 MHz band for carriers above 15 GHz, supplied to the antenna transmission line For the fixed service in the bands shared with space services |  |  |  |  |  |  | C |  |  |  |  | 8AB |
| 8.5 | 8AC | the maximum power density (dB(W/Hz)) averaged over the worst 4 kHz band, calculated for the maximum effective radiated power Note – For a receiving land station, the maximum power density refers to the associated transmitting station In the case of a VHF/UHF broadcasting station, required for assignments subject to § 5.1.3 of the GE06 Regional Agreement In the case of a transmitting station, a receiving land station, or a typical transmitting station, required for assignments subject to the GE06 Regional Agreement |  |  |  |  | + |  | + | + | + |  |  | 8AC |
| 8.6 | 8B | the radiated power, in dBW, in one of the forms described in Nos. 1.161 to 1.163 Note – Where adaptive systems in the fixed or mobile service operating in the bands between 300 kHz and 28 MHz (see also Resolution 729 (Rev.WRC-07)) use automatic power control, the radiated power includes the level of power control listed under 8BA For assignments in all services and frequency bands, except assignments subject to the GE06 Regional Agreement, required if the power delivered to the antenna (8AA), or the maximum antenna gain (9G), is not provided For an assignment subject to the GE06 Regional Agreement, required if the power delivered to the antenna (8AA) is not provided |  |  |  |  |  |  | + | + | + |  |  | 8B |
| 8.7 | 8BA | the range of power control, in dB Required for adaptive systems in the fixed or mobile service operating in the bands between 300 kHz and 28 MHz (see also Resolution 729 (Rev.WRC-07)), if automatic power control is used |  |  |  |  |  |  | + |  |  |  |  | 8BA |
| 8.8 | 8BH | the maximum effective radiated power, in dBW, of the horizontally polarized component Required for horizontal or mixed polarization |  |  |  |  | + |  |  |  |  |  |  | 8BH |
| 8.9 | 8BV | the maximum effective radiated power, in dBW, of the vertically polarized component Required for vertical or mixed polarization |  |  |  |  | + |  |  |  |  |  |  | 8BV |
| 8.10 | 8BT | the maximum effective radiated power, in dBW, in the plane defined by the beam tilt angle For a digital broadcasting assignment in the UHF band subject to the GE06 Regional Agreement only |  |  |  |  | O |  |  |  |  |  |  | 8BT |
| 8.11 | 8D | the vision-to-sound carrier power ratio, in dB Required for analogue television broadcasting |  |  |  |  | + |  |  |  |  |  |  | 8D |
| 8.12 | 9L | the maximum effective monopole radiated power, in dB(kW) Required for the GE75 Regional Agreement |  |  |  |  |  | + |  |  |  |  |  | 9L |
| 8.13 |  | For the RJ81 and RJ88 Regional Agreements: |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1 | 9I | the r.m.s. value of radiation The product of the r.m.s. characteristic field strength in the horizontal plane and the square root of the power |  |  |  |  |  | X |  |  |  |  |  | 9I |
| 8.13.2 | 9IA | the value of the radiation at the central azimuth of the augmentation, in mV/m at 1 km Required for antenna radiation pattern type “M” (see 9O) |  |  |  |  |  | + |  |  |  |  |  | 9IA |
| 8.13.3 | 9P | the value of the special quadrature factor, in mV/m at 1 km Note – A special quadrature factor may be used with antenna pattern type “M” or “E” to replace the normal expanded quadrature factor when special precautions are taken to ensure pattern stability |  |  |  |  |  | O |  |  |  |  |  | 9P |
| 9 |  | ANTENNA CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 |  | For a transmitting or receiving antenna: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1 | 9 | the indicator showing whether the antenna is directional (D) or non-directional (ND) In the case of a receiving land station, required for an assignment subject to the GE06 Regional Agreement |  |  |  |  | X |  | X | + |  | X | X | 9 |
| 9.1.2 | 9D | the code indicating the type of polarization (see the Preface) In the case of a transmitting station, required for an assignment: – in the fixed service in the bands shared with space services; or – subject to the GE06 Regional Agreement In the case of a receiving land station, required for an assignment subject to the GE06 Regional Agreement |  |  |  |  | X |  | + | + |  |  |  | 9D |
| 9.1.3 | 9E | the height of the antenna above ground level, in metres In the case of a VHF/UHF broadcasting station, required for the ST61, GE84, GE89 or GE06 Regional Agreements, and optional for assignments not subject to these Agreements In the case of a transmitting station, required for an assignment: – in the bands shared with space services; or – subject to the GE06 Regional Agreement In the case of a receiving land station, required for an assignment subject to the GE06 Regional Agreement |  |  |  |  | + |  | + | + |  |  |  | 9E |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2 |  | For a directional transmitting or receiving antenna: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1 | 9C | the total angular width of the radiation main lobe (beamwidth) measured horizontally in a plane containing the direction of maximum radiation, in degrees, within which the power radiated in any direction does not fall more than 3 dB below the power radiated in the direction of maximum radiation In the case of a transmitting station, required for all assignments, except assignments subject to GE06 Regional Agreement where it is optional In the case of a receiving land station, for an assignment subject to the GE06 Regional Agreement only |  |  |  |  |  |  | + | O |  | X |  | 9C |
| 9.2.2 | 9GL | the antenna gain towards the local horizon For an assignment subject to the GE06 Regional Agreement only |  |  |  |  |  |  | O | O |  |  |  | 9GL |
| 9.2.3 | 9K | the lowest total receiving system noise temperature, in kelvins For an associated receiving antenna in the fixed service operating in the bands shared with space services only |  |  |  |  |  |  | C |  |  |  |  | 9K |
| 9.3 |  | For a transmitting antenna: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.1 | 9EA | the altitude of the site above mean sea level, in metres In the case of a VHF/UHF broadcasting station, required for assignments subject to the ST61, GE84, GE89, or GE06 Regional Agreements, and optional for assignments not subject to these Agreements In the case of a transmitting station, required for an assignment: – in the fixed or mobile service in the bands shared with space services; or – subject to the GE06 Regional Agreement |  |  |  |  | + |  | + |  |  |  |  | 9EA |
| 9.3.2 | 9EB | the maximum effective height of the antenna, in metres, above the mean level of the ground between 3 and 15 km from the transmitting antenna In the case of a transmitting station, required for an assignment subject to the GE06 Regional Agreement |  |  |  |  | X |  | + |  |  |  |  | 9EB |
| 9.3.3 | 9EC | the effective height of the antenna, in metres, above the mean level of the ground between 3 and 15 km from the transmitting antenna, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction In the case of a VHF/UHF broadcasting station, required for an assignment subject to the ST61, GE84, GE89 or GE06 Regional Agreements In the case of a transmitting station, required for an assignment subject to the GE06 Regional Agreement |  |  |  |  | + |  | + |  |  |  |  | 9EC |
| 9.3.4 | 9G | the maximum antenna gain (isotropic, relative to a short vertical antenna or relative to a half-wave dipole, as appropriate) of the transmitting antenna (see No. 1.160) For a directional antenna, the gain is in the direction of maximum radiation In the case of a transmitting station, or a typical transmitting station: – for all frequency bands and services, except assignments subject to the GE06 Regional Agreement, required if the antenna is: – directional, including where the antenna beam is rotating or swept; or – non-directional, and the power to the antenna (8AA) or the radiated power (8B) is not provided – for an assignment subject to the GE06 Regional Agreement required if the radiated power (8B) is not provided In the case of a maritime mobile frequency allotment, required if the antenna is directional, including where the antenna beam is rotating or swept |  |  |  |  |  |  | + |  | + | + |  | 9G |
| 9.3.5 | 9M | the transmitting antenna design frequency |  |  |  |  |  |  |  |  |  |  | X | 9M |
| 9.3.6 | 9S | the beam tilt angle, in degrees The beam tilt angle is measured from the horizontal plane towards ground and the sign of the angle is negative Note – In some broadcasting definitions, the angle may have the opposite sign For a digital broadcasting assignment in the UHF band subject to the GE06 Regional Agreement only |  |  |  |  | O |  |  |  |  |  | X | 9S |
| 9.3.7 | 9J | the measured radiation pattern of the antenna, the reference radiation pattern or the symbols in standard references to be used for coordination |  |  |  |  |  |  | O |  |  |  | X | 9J |
| 9.4 |  | For a directional transmitting antenna where the antenna beam is rotating or swept: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.4.1 | 9AB1 | the start azimuth for the range of operational angles for the antenna’s main beam axis, measured in the horizontal plane from True North in a clockwise direction |  |  |  |  |  |  | X |  |  | X |  | 9AB1 |
| 9.4.2 | 9AB2 | the end azimuth for the range of operational angles for the antenna’s main beam axis, measured in the horizontal plane from True North in a clockwise direction |  |  |  |  |  |  | X |  |  | X |  | 9AB2 |
| 9.5 |  | For a directional transmitting antenna where the antenna beam is not rotating or swept: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.5.1 | 9A | the azimuth of maximum radiation of the transmitting antenna, measured in the horizontal plane from True North in a clockwise direction |  |  |  |  |  |  | X |  |  | X | X | 9A |
| 9.5.2 | 9B | the elevation angle of maximum directivity, in degrees Required for an assignment in the bands shared with space services |  |  |  |  |  |  | + |  |  |  |  | 9B |
| 9.5.3 | 9R | the slew angle measured between the azimuth of maximum radiation and the direction of unslewed radiation |  |  |  |  |  |  |  |  |  |  | X | 9R |
| 9.5.4 | 9NH | the value of attenuation of the horizontally polarized component, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction, with respect to the maximum effective radiated power of this component, in dB For all assignments, except digital broadcasting assignments subject to the GE06 Regional Agreement and broadcasting assignments subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is horizontal or mixed |  |  |  |  | + |  |  |  |  |  |  | 9NH |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.5.5 | 9NV | the value of attenuation of the vertically polarized component, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction, with respect to the maximum effective radiated power of this component, in dB For all assignments, except digital broadcasting assignments subject to the GE06 Regional Agreement and broadcasting assignments subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is vertical or mixed |  |  |  |  | + |  |  |  |  |  |  | 9NV |
| 9.5.6 | 9UH | the value of attenuation of the horizontally polarized component in the horizontal plane, normalized to 0 dB, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction, with respect to the maximum radiated power of this component, in dB In the case of a VHF/UHF broadcasting station, for a digital broadcasting assignment subject to the GE06 Regional Agreement and an assignment subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is horizontal or mixed In the case of a transmitting station, for an assignment subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is horizontal or mixed |  |  |  |  | + |  | + |  |  |  |  | 9UH |
| 9.5.7 | 9UV | the value of attenuation of the vertically polarized component in the horizontal plane, normalized to 0 dB, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction, with respect to the maximum radiated power of this component, in dB In the case of a VHF/UHF broadcasting station, for a digital broadcasting assignment subject to the GE06 Regional Agreement and an assignment subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is vertical or mixed In the case of a transmitting station, for an assignment subject to § 5.1.3 of the GE06 Regional Agreement, required if the polarization is vertical or mixed |  |  |  |  | + |  | + |  |  |  |  | 9UV |
| 9.6 | 9Q | the symbol identifying the type of antenna Type A – a simple vertical antenna Type B – a directional or omnidirectional antenna of complex construction |  |  |  |  |  | X |  |  |  |  |  | 9Q |
| 9.7 |  | For a type A antenna (simple vertical antenna): |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.7.1 | 9EP | the transmitting antenna’s physical length in metres Required for the GE75 Regional Agreement |  |  |  |  |  | + |  |  |  |  |  | 9EP |
| 9.7.2 | 9F | the electrical height of the antenna, in degrees Required for the RJ81 or RJ88 Regional Agreements |  |  |  |  |  | + |  |  |  |  |  | 9F |
| 9.8 |  | For a station subject to the GE75 Regional Agreement with a type B antenna (a directional antenna, or omnidirectional antenna of complex construction): |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.8.1 | 9GH | the antenna gain, in dB, in the horizontal plane, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°), measured in the horizontal plane from True North in a clockwise direction |  |  |  |  |  | X |  |  |  |  |  | 9GH |
| 9.8.2 | 9GV | the antenna gain, in dB, in the vertical plane, at 36 different azimuths in 10° intervals (i.e. 0°, 10°, ..., 350°) measured in the horizontal plane from True North in a clockwise direction, and at ten different elevations in 10° intervals (i.e. 0°, 10°, ..., 90°) measured in the vertical plane Note – If administrations have difficulty in providing this information, they can provide a reference to any other information that may be of assistance (e.g. ITU-R Recommendation, antenna pattern) Required for an assignment to be used for night-time operation |  |  |  |  |  | + |  |  |  |  |  | 9GV |
| 9.9 |  | For a station subject to the RJ81 or RJ88 Regional Agreements with a type B antenna (a directional antenna, or omnidirectional antenna of complex construction): |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.9.1 | 9O | the symbol identifying the type of antenna radiation pattern (T, M, or E) |  |  |  |  |  | X |  |  |  |  |  | 9O |
| 9.9.2 |  | For antenna radiation pattern type M: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.9.2.1 | 9NA | the serial number of the augmentation as described by items 9IA, 9AA and 9CA |  |  |  |  |  | X |  |  |  |  |  | 9NA |
| 9.9.2.2 | 9AA | the central azimuth of the augmentation (centre of the span) in degrees |  |  |  |  |  | X |  |  |  |  |  | 9AA |
| 9.9.2.3 | 9CA | the total span of the augmentation, in degrees |  |  |  |  |  | X |  |  |  |  |  | 9CA |
| 9.9.3 |  | For each tower of a type B antenna in the RJ81 or RJ88 Regional Agreements: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.9.3.1 | 9T1 | the serial number of each of the towers whose characteristics are described in items 9T2 to 9T8 |  |  |  |  |  | X |  |  |  |  |  | 9T1 |
| 9.9.3.2 | 9T8 | the symbol corresponding to the tower structure |  |  |  |  |  | X |  |  |  |  |  | 9T8 |
| 9.9.3.3 | 9T7 | the electrical height, in degrees, of the tower under consideration Required if the tower is not top-loaded nor sectionalized (see 9.9.4) |  |  |  |  |  | + |  |  |  |  |  | 9T7 |
| 9.9.3.4 | 9T2 | the ratio of the tower field to the field of the reference tower Required if the antenna consists of two or more towers |  |  |  |  |  | + |  |  |  |  |  | 9T2 |
| 9.9.3.5 | 9T3 | the positive or negative phase difference in the tower field with respect to the field of the reference tower, in degrees Required if the antenna consists of two or more towers |  |  |  |  |  | + |  |  |  |  |  | 9T3 |
| 9.9.3.6 | 9T4 | the electrical spacing of the tower from the reference point, in degrees Required if the antenna consists of two or more towers |  |  |  |  |  | + |  |  |  |  |  | 9T4 |
| 9.9.3.7 | 9T5 | the angular orientation of the tower from the reference point, in degrees (clockwise) from True North Required if the antenna consists of two or more towers |  |  |  |  |  | + |  |  |  |  |  | 9T5 |
| 9.9.4 |  | For each tower of a type B antenna that is top-loaded or sectionalized in accordance with the Regional Administrative MF Broadcasting Conference (Region 2) Rio de Janeiro, 1981 or 1988 Agreements: |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.9.4.1 | 9T9A | the description of a top-loaded or sectionalized tower |  |  |  |  |  | X |  |  |  |  |  | 9T9A |
| 9.9.4.2 | 9T9B | the description of a top-loaded or sectionalized tower Required if tower structure symbol (9T8) is 1, 2, 5, 6, 7, 8 or 9 |  |  |  |  |  | + |  |  |  |  |  | 9T9B |
| 9.9.4.3 | 9T9C | the description of a top-loaded or sectionalized tower Required if the tower structure symbol (9T8) is 2, 5, 7 or 8 |  |  |  |  |  | + |  |  |  |  |  | 9T9C |

| Column No. | Item identifier | Notice related to Description of data items and requirements |  |  |  |  | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.9.4.4 | 9T9D | the description of a top-loaded or sectionalized tower Required if tower structures symbol (9T8) is 2, 5 or 8 |  |  |  |  |  | + |  |  |  |  |  | 9T9D |
| 10 |  | HOURS OF OPERATION |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | 10B | the regular hours of operation (in hours and minutes from ... to ...) of the frequency assignment, in UTC |  |  |  |  | X | O | X | X | X | X | X | 10B |
| 10.2 | 10BA | the local operation period code (see the Preface) |  |  |  |  |  | X |  |  |  |  |  | 10BA |
| 10.3 | 10D | the estimated peak hours of traffic |  |  |  |  |  |  |  |  |  | X |  | 10D |
| 10.4 | 10E | the estimated daily volume of traffic |  |  |  |  |  |  |  |  |  | X |  | 10E |
| 11 |  | COORDINATION AND AGREEMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | 11 | the symbol of each administration with which coordination has been successfully effected Required if coordination is necessary and has been obtained pursuant to the relevant provisions of the Radio Regulations |  |  |  |  | + | O | + | + | O | + |  | 11 |
| 11.2 | 11D | a declaration by the notifying administration that all conditions associated with the remark are fully met for recording the submitted assignment in the Master International Frequency Register Required for a digital broadcasting assignment subject to § 5.1.2 of the GE06 Regional Agreement and for the broadcasting and other primary services assignments notified pursuant to No. 5.1.3 of the Agreement |  |  |  |  | + |  | + | + |  |  |  | 11D |
| 11.3 | 11C | a signed commitment from the notifying administration that the submitted assignment for recording in the Master International Frequency Register shall not cause unacceptable interference and shall not claim protection Required for an assignment subject to § 5.1.8 of the GE06 Regional Agreement |  |  |  |  | + |  |  |  |  |  |  | 11C |
| 11.4 | 11E | a signed commitment from the notifying administration that the submitted assignment for recording in the Master International Frequency Register shall not cause unacceptable interference and shall not claim protection Required for an assignment subject to § 5.2.6 of the GE06 Regional Agreement |  |  |  |  |  |  | + | + | + |  |  | 11E |
| 11.5 | 11F | Recognition by the notifying administration that the registration of assignments in the aeronautical mobile (R) service in the 5 030-5 091 MHz frequency band accords with the purposes of ITU, including No. 7 of Article 1 of the ITU Constitution. Required for an assignment in aeronautical mobile (R) service in the frequency band 5 030-5 091 MHz |  |  |  |  |  |  | + | + | + |  |  | 11F |
| 12 |  | OPERATING ADMINISTRATION OR AGENCY |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | 12A | the symbol for the operating agency |  |  |  |  | O | O | O | O | O |  | O | 12A |
| 12.2 | 12B | the symbol for the address of the administration responsible for the station and to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the circuit (see Article 15, also the Preface) In the case of a VHF/UHF broadcasting station, transmitting station, or a receiving land station, required for application of Article 11 |  |  |  |  | + | X | + | + | X |  | X | 12B |
| 13 |  | REMARKS |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 | 13C | Remarks for assisting the Bureau in processing the notice |  |  |  |  | O | O | O | O | O | O | O | 13C |

TABLE 2

Characteristics for high altitude platform stations (HAPS) frequency assignments
in the terrestrial services

| Item identifier | 1 _ GENERAL CHARACTERISTICS OF THE HAPS | Transmitting station in the bands listed in No. 5.388A for the application of No. 11.2 | Receiving station in the bands listed in No. 5.388A for the application of No. 11.9 | Transmitting station in the bands listed in Nos. 5.537A and 5.552A for the application of No. 11.2 | Receiving station in the bands listed in Nos. 5.543A and 5.552A for the application of No. 11.9 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- |
|  | GENERAL INFORMATION |  |  |  |  |  |
| 1.B | the symbol of the notifying administration (see the Preface) | X | X | X | X | 1.B |
| 1.D | the provision code of the Radio Regulations under which the notice has been submitted | X | X | X | X | 1.D |
| 1.ID1 | the unique identifier given by the administration to the station | X | X | X | X | 1.ID1 |
|  | LOCATION OF THE STATION |  |  |  |  |  |
| 1.4.a | the name by which the station is known | X | X | X | X | 1.4.a |
| 1.4.b | the code of the geographical area, above which the station is located (see the Preface) | X | X | X | X | 1.4.b |
| 1.4.c | the nominal geographical coordinates of the station Latitude and longitude are provided in degrees, minutes and seconds | X | X | X | X | 1.4.c |
| 1.4.h | the nominal altitude of the station above mean sea level, in metres | X | X | X | X | 1.4.h |
| 1.4.t | Station location tolerances: |  |  |  |  | 1.4.t |
| 1.4.t.1.a | the planned latitudinal tolerance northerly limit, using d.m.s units | X | X | X | X | 1.4.t.1.a |
| 1.4.t.1.b | the planned latitudinal tolerance southerly limit, using d.m.s units | X | X | X | X | 1.4.t.1.b |
| 1.4.t.2.a | the planned longitudinal tolerance easterly limit, using d.m.s units | X | X | X | X | 1.4.t.2.a |
| 1.4.t.2.b | the planned longitudinal tolerance westerly limit, using d.m.s units | X | X | X | X | 1.4.t.2.b |
| 1.4.t.3 | the planned altitudinal tolerance, in metres | X | X | X | X | 1.4.t.3 |
|  | COMPLIANCE WITH TECHNICAL OR OPERATIONAL LIMITS |  |  |  |  |  |
| 1.14.b | a commitment that the HAPS does not exceed an out-of-band pfd of −165 dB(W/(m2 · 4 kHz)) at the Earth’s surface in the bands 2 160-2 200 MHz in Region 2 and 2 170-2 200 MHz in Regions 1 and 3 (see Resolution 221 (Rev.WRC-07)) | X |  |  |  | 1.14.b |
| 1.14.c | a commitment that the HAPS does not exceed the out-of-band pfd limits of −165 dB(W/(m2 · MHz)) for angles of arrival (θ) less than 5° above the horizontal plane, −165 + 1.75 (θ − 5) dB(W/(m2 · MHz)) for angles of arrival between 5° and 25° and −130 dB(W/(m2 · MHz)) for angles of arrival between 25° and 90° (see Resolution 221 (Rev.WRC-07)) | X |  |  |  | 1.14.c |
| 1.14.d | a commitment that the unwanted power density into the HAPS ground station antenna in the band 31.3-31.8 GHz shall not exceed −106 dB(W/MHz) under clear-sky conditions and −100 dB(W/MHz) under rainy conditions (see Resolution 145 (Rev.WRC-07)) Required in the band 31-31.3 GHz |  |  |  | + | 1.14.d |
| 1.14.e | a commitment that the maximum power density into an ubiquitous HAPS ground station antenna in the Urban Area Coverage (UAC) shall not exceed 6.4 dB(W/MHz) for elevation angles of ground station antenna greater than 30° and less than or equal to 90° (see Resolution 122 (Rev.WRC-07)) Required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz |  |  |  | + | 1.14.e |
| 1.14.f | a commitment that the maximum power density into an ubiquitous HAPS ground station antenna in the Suburban Area Coverage (SAC) shall not exceed 22.57 dB(W/MHz) for elevation angles of ground station antenna greater than 15° and less than or equal to 30° (see Resolution 122 (Rev.WRC-07)) Required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz |  |  |  | + | 1.14.f |
| 1.14.g | a commitment that the maximum power density into an ubiquitous HAPS ground station antenna in the Rural Area Coverage (RAC) shall not exceed 28 dB(W/MHz) for elevation angles of ground station antenna greater than 5° and less than or equal to 15° (see Resolution 122 (Rev.WRC-07)) Required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz |  |  |  | + | 1.14.g |
| 1.14.h | a commitment that the separation distance between the nadir of the HAPS and a radio astronomy station operating in the band 48.94-49.04 GHz within the territory of another administration shall exceed 50 km (see Resolution 122 (Rev.WRC-07)) Required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz |  |  | + |  | 1.14.h |
|  | COORDINATION AND AGREEMENT |  |  |  |  |  |
| 1.11.a | the symbol of each administration with which coordination has been successfully effected, including where the agreement is to exceed the limits prescribed in the Radio Regulations | + | + | + | + | 1.11.a |
|  | Required if coordination is necessary and has been obtained pursuant to the relevant provisions of the Radio Regulations |  |  |  |  |  |
|  | OPERATING ADMINISTRATION OR AGENCY |  |  |  |  |  |
| 1.12.a | the symbol for the operating agency | O | O | O | O | 1.12.a |
| 1.12.b | the symbol for the address of the administration responsible for the station and to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the circuit (see Article 15) | X | X | X | X | 1.12.b |
|  | REMARKS |  |  |  |  |  |
| 1.13.c | Remarks for assisting the Bureau in processing the notice | O | O | O | O | 1.13.c |

| Item identifier | 2 _ CHARACTERISTICS TO BE PROVIDED FOR EACH INDIVIDUAL OR COMPOSITE HAPS ANTENNA BEAM | Transmitting station in the bands listed in No. 5.388A for the application of No. 11.2 | Receiving station in the bands listed in No. 5.388A for the application of No. 11.9 | Transmitting station in the bands listed in Nos. 5.537A and 5.552A for the application of No. 11.2 | Receiving station in the bands listed in Nos. 5.543A and 5.552A for the application of No. 11.9 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- |
|  | IDENTIFICATION AND DIRECTION OF THE HAPS ANTENNA BEAM |  |  |  |  |  |
| 2.1.a | the designation of the HAPS antenna beam | X | X | X | X | 2.1.a |
| 2.1.b | an indicator showing whether the antenna beam, under 2.1.a, is fixed or whether it is steerable and/or reconfigurable | X | X | X | X | 2.1.b |
| 2.1.c | an indicator showing whether the HAPS antenna tracks the service area | X |  | X |  | 2.1.c |
| 2.1.d | an indicator showing whether the antenna beam is individual or composite beam | X | X | X | X | 2.1.d |
|  | ANTENNA CHARACTERISTICS |  |  |  |  |  |
| 2.9.g | the maximum co-polar isotropic gain | X | X | X | X | 2.9.g |
| 2.9.j | the measured radiation pattern of the antenna, the reference radiation pattern or the symbols in standard references to be used for coordination | X | X |  |  | 2.9.j |
| 2.9.gp | the co-polar antenna gain contours plotted on a map of the Earth’s surface, preferably in a radial projection from the HAPS onto a plane perpendicular to the axis from the centre of the Earth to the HAPS The HAPS antenna gain contours shall be drawn as isolines of the isotropic gain, relative to the maximum antenna gain, when any of these contours is located either totally or partially outside the territory of the notifying administration The antenna gain contours shall include the effects of the planned longitudinal and latitudinal tolerance, planned altitudinal tolerance and the pointing accuracy of the antenna, taking into consideration the movement of the HAPS antenna boresight around the effective boresight area | X | X | X | X | 2.9.gp |

| Item identifier | 3 _ CHARACTERISTICS TO BE PROVIDED FOR EACH FREQUENCY ASSIGNMENT FOR EACH INDIVIDUAL OR COMPOSITE HAPS ANTENNA BEAM | Transmitting station in the bands listed in No. 5.388A for the application of No. 11.2 | Receiving station in the bands listed in No. 5.388A for the application of No. 11.9 | Transmitting station in the bands listed in Nos. 5.537A and 5.552Afor the application of No. 11.2 | Receiving station in the bands listed in Nos. 5.543A and 5.552A for the application of No. 11.9 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- |
|  | ASSIGNED FREQUENCY |  |  |  |  |  |
| 3.1.a | the assigned frequency, as defined in No. 1.148 | X | X | X | X | 3.1.a |
| 3.1.b | the reference frequency, as defined in Article 1 Required if the modulation envelope is asymmetric | + | + | + | + | 3.1.b |
|  | DATE OF OPERATION |  |  |  |  |  |
| 3.2.c | the date (actual or foreseen, as appropriate) of bringing the frequency assignment (new or modified) into use | X | X | X | X | 3.2.c |
|  | LOCATION OF THE ASSOCIATED ANTENNA(S) |  |  |  |  |  |
|  | For an area in which associated transmitting/receiving ground station(s) operate: |  |  |  |  |  |
| 3.5.c.a | the geographical coordinates of a given zone A minimum of six geographical coordinates are required, in degrees, minutes and seconds Note – For the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz the geographical coordinates are provided for each of the UAC, SAC and if applicable RAC (see the most recent version of Recommendation ITU-R F.1500) Required if neither a circular area (3.5.e and 3.5.f) nor a geographical area (3.5.d) are provided | + | + | + | + | 3.5.c.a |
| 3.5.d | the code of the geographical area (see the Preface) Note – For the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz separate geographical areas are provided for each of the UAC, SAC and if applicable RAC (see the most recent version of Recommendation ITU-R F.1500) Required if neither a circular area (3.5.e and 3.5.f) nor the geographical coordinates of a given zone (3.5.c.a) are provided | + | + | + | + | 3.5.d |
| 3.5.e | the geographical coordinates of the centre of the circular area in which the associated ground station(s) are operating The latitude and longitude are provided in degrees, minutes and seconds Note – For the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz different centres of the circular area may be provided for the UAC, SAC and if applicable RAC (see the most recent version of Recommendation ITU-R F.1500) Required if neither a geographical area (3.5.d) or geographical coordinates of a given zone (3.5.c.a) are provided | + | + | + | + | 3.5.e |
| 3.5.f | the radius, in km, of the circular area | + | + | + | + | 3.5.f |
|  | Note – For the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz, a separate radius is provided for each of the UAC, SAC and if applicable RAC (see the most recent version of Recommendation ITU-R F.1500) Required if neither a geographical area (3.5.d) nor geographical coordinates of a given zone (3.5.c.a) are provided |  |  |  |  |  |
|  | CLASS OF STATION AND NATURE OF SERVICE |  |  |  |  |  |
| 3.6.a | the class of station, using the symbols from the Preface | X | X | X | X | 3.6.a |
| 3.6.b | the nature of service, using the symbols from the Preface | X | X | X | X | 3.6.b |
|  | CLASS OF EMISSION AND NECESSARY BANDWIDTH (in accordance with Article 2 and Appendix 1) |  |  |  |  |  |
| 3.7.a | the class of emission | X | X | X | X | 3.7.a |
| 3.7.b | the necessary bandwidth | X | X | X | X | 3.7.b |
|  | POWER CHARACTERISTICS OF THE TRANSMISSION |  |  |  |  |  |
| 3.8 | the symbol (X, Y or Z, as appropriate) describing the type of power (see Article 1) corresponding to the class of emission | X | X | X | X | 3.8. |
| 3.8.aa | the power delivered to the antenna, in dBW, including the level of power control in 3.8.BA Note – For a receiving HAPS, the power delivered to the antenna refers to the associated transmitting ground station(s) | X |  | X | X | 3.8.aa |
| 3.8.AB | the maximum power density1 averaged over the worst 1 MHz band delivered to the antenna | X |  | X |  | 3.8AB |
| 3.8.BA | the range of power control, in dB Note – For a receiving HAPS, the power control refers to its use by the associated transmitting ground station(s) In the case of a receiving HAPS, required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz | X |  |  | + | 3.8.BA |
|  | POLARIZATION AND RECEIVING SYSTEM NOISE TEMPERATURE |  |  |  |  |  |
| 3.9.d | the code indicating the type of polarization (see the Preface) | X | X | X | X | 3.9.d |
| 3.9.j | the reference radiation pattern of the associated ground station(s) Required in the bands 47.2-47.5 GHz and 47.9-48.2 GHz |  |  | + | + | 3.9.j |
| 3.9.k | the lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna |  | X |  | X | 3.9.k |
|  | HOURS OF OPERATION |  |  |  |  |  |
| 3.10.b | the regular hours of operation (in hours and minutes from ... to ...) of the frequency assignment, in UTC | X | X | X | X | 3.10.b |

ANNEX 2

Characteristics of satellite networks, earth stations
or radio astronomy stations2     (Rev.WRC-12)

Information relating to the data listed in the following Tables

In many cases the data requirements involve the use of standard symbols in submissions to the Radiocommunication Bureau. These standard symbols may be found in the “Preface to the BR International Frequency Information Circular”, (BR IFIC) (Space Services), the ITU-R webpage and the Space Radiocommunication Stations on DVD-ROM. (In the Table, this is referred to simply as “the Preface”.) Information relating to the provision of data may also be found in ITU-R Recommendations, for example, information on the mask data can be found in the most recent version of Recommendation ITU-R S.1503, and the most recent version of Recommendation ITU-R SM.1413 provides general information related to submission of data.

Key to the symbols used in Tables A, B, C and D

| X | Mandatory information |
| --- | --- |
| + | Mandatory under the conditions specified in Column 2 |
| O | Optional information |
| C | Mandatory if used as a basis to effect coordination with another administration |
|  | The data item is not applicable to the corresponding notice |

Reading the Appendix 4 Tables

The rules used to link the sign with the text are based on the Table column headings covering specific procedures and specific services.

1	If any data item has a condition attached to it, then it has a “+”.

| A.6.c | if agreement has been reached, the related provision code (see the Preface) |  | + |  | A.6.c |
| --- | --- | --- | --- | --- | --- |

| C.8.f.1 | the space station’s nominal equivalent isotropically radiated power(s) (e.i.r.p.) on the beam axis Required only for a space-to-space link |  | + |  | C.8.f.1 |
| --- | --- | --- | --- | --- | --- |

2	Data items grouped under a common subheading that limits the range of procedures, services or frequency bands have a “X” as the conditional nature is shown in the subheading title.

| A.4.b.5 | For space stations operating in a frequency band subject to the provisions of Nos. 9.11A, 9.12 or 9.12A, the data elements to characterize properly the orbital statistics of the non-geostationary-satellite system: |  |  |  | A.4.b.5 |
| --- | --- | --- | --- | --- | --- |
| A.4.b.5.a | the right ascension of the ascending node (j) for the j-th orbital plane, measured counter-clockwise in the equatorial plane from the direction of the vernal equinox to the point where the satellite makes its South-to-North crossing of the equatorial plane (0° ≤ j < 360°) |  | X |  | A.4.b.5.a |

3	“In the case of”, followed by a reference to the column heading, is used as shown below when the associated conditions are different for individual columns, or if the indication is not the same across all applicable columns.

| A.3.a | the symbol for the operating administration or agency (see the Preface) that is in operational control of the space station, earth station or radio astronomy station In the case of Appendix 30B, required only for notification under Article 8 |  | X | + | A.3.a |
| --- | --- | --- | --- | --- | --- |

Footnotes to Tables A, B, C and D

1	Not required for coordination under No. 9.7A.

2	In calculating the maximum power density per Hz, see the most recent version of Recommendation ITU-R SF.675. For carriers below 15 GHz, the power density is averaged over the worst 4 kHz band. For carriers at or above 15 GHz, the power density is averaged over the worst 1 MHz band.    (WRC-12)

Table of characteristics to be submitted for space and radio astronomy services 
(Rev.WRC-12)

TABLE A

GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, 
EARTH STATION OR RADIO ASTRONOMY STATION     (Rev.WRC-15)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.1 | IDENTITY OF THE SATELLITE NETWORK, EARTH STATION OR RADIOASTRONOMY STATION |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1 |  |
| A.1.a | the identity of the satellite network |  |  |  |  | X | X | X | X | X |  | X | X | X | A.1.a |  |
| A.1.b | the beam identification In the case of Appendix 30 or 30A, required for modification, suppression or notification of Plan assignments In the case of Appendix 30B, required for a network derived from the Allotment Plan |  |  |  |  |  |  |  |  |  |  | + | + | + | A.1.b |  |
| A.1.e | Identity of the earth station or radio astronomy station: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.e |  |
| A.1.e.1 | the type of earth station (specific or typical) |  |  |  |  |  |  |  |  |  | X |  |  |  | A.1.e.1 |  |
| A.1.e.2 | the name of the station |  |  |  |  |  |  |  |  |  | X |  |  |  | A.1.e.2 | X |
| A.1.e.3 | For a specific earth station or radio astronomy station: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.e.3 |  |
| A.1.e.3.a | the country or geographical area in which the station is located, using the symbols from the Preface |  |  |  |  |  |  |  |  |  | X |  |  |  | A.1.e.3.a | X |
| A.1.e.3.b | the geographical coordinates of each transmitting or receiving antenna site constituting the station latitude and longitude in degrees and minutes) For a specific earth station, seconds are to be provided if the coordination area of the earth station overlaps the territory of another administration |  |  |  |  |  |  |  |  |  | X |  |  |  | A.1.e.3.b | X |
| A.1.f | Administration and intergovernmental organization symbol: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.f |  |
| A.1.f.1 | the symbol of the notifying administration (see the Preface) |  |  |  |  | X | X | X | X | X | X | X | X | X | A.1.f.1 | X |
| A.1.f.2 | if the notice is submitted on behalf of a group of administrations, the symbols of each of the administrations in the group, submitting the information on the satellite network (see the Preface) |  |  |  |  | + | + | + | + | + |  | + | + | + | A.1.f.2 |  |
| A.1.f.3 | if the notice is submitted on behalf of an intergovernmental satellite organization, the symbol of that organization (see the Preface) |  |  |  |  | + | + | + | + | + |  | + | + | + | A.1.f.3 |  |
| A.1.g | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.g |  |
| A.1.g.1 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.g.1 |  |
| A.1.g.2 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.1.g.2 |  |
| A.2 | DATE OF BRINGING INTO USE |  |  |  |  |  |  |  |  |  |  |  |  |  | A.2 |  |
| A.2.a | the date (actual or foreseen, as appropriate) of bringing the frequency assignment (new or modified) into use For a frequency assignment to a GSO space station, including frequency assignments in Appendices 30, 30A and 30B, the date of bringing into use is as defined in Nos. 11.44B and 11.44.2 Whenever the assignment is changed in any of its basic characteristics (except in the case of a change under A.1.a, the date to be given shall be that of the latest change (actual or foreseen, as appropriate) Required only for notification |  |  |  |  |  |  |  | + | + | + | + | + | + | A.2.a |  |
| A.2.b | for a space station, the period of validity of the frequency assignments (see Resolution 4 (Rev.WRC-03)) |  |  |  |  |  |  | X | X | X |  |  |  |  | A.2.b |  |
| A.2.c | the date (actual or foreseen, as appropriate) on which reception of the frequency band begins or on which any of the basic characteristics are modified |  |  |  |  |  |  |  |  |  |  |  |  |  | A.2.c | X |
| A.3 | OPERATING ADMINISTRATION OR AGENCY |  |  |  |  |  |  |  |  |  |  |  |  |  | A.3 |  |
| A.3.a | the symbol for the operating administration or agency (see the Preface) that is in operational control of the space station, earth station or radio astronomy station In the case of Appendix 30B, required only for notification under Article 8 |  |  |  |  |  |  | X | X | X | X | X | X | + | A.3.a | X |
| A.3.b | the symbol for the address of the administration (see the Preface) to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the network or station (see Article 15) In the case of Appendix 30B, required only for notification under Article 8 |  |  |  |  |  |  | X | X | X | X | X | X | + | A.3.b | X |
| A.4 | ORBITAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4 |  |
| A.4.a | For a space station onboard a geostationary-satellite: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.a |  |
| A.4.a.1 | the nominal geographical longitude on the geostationary-satellite orbit (GSO) |  |  |  |  | X |  |  | X |  |  | X | X | X | A.4.a.1 |  |
| A.4.a.2 | Orbital tolerances |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.a.2 |  |
| A.4.a.2.a | the planned longitudinal tolerance easterly limit |  |  |  |  |  |  |  | X |  |  | X | X | X | A.4.a.2.a |  |
| A.4.a.2.b | the planned longitudinal tolerance westerly limit |  |  |  |  |  |  |  | X |  |  | X | X | X | A.4.a.2.b |  |
| A.4.a.2.c | the planned inclination excursion |  |  |  |  |  |  |  | X |  |  |  |  | X | A.4.a.2.c |  |
| A.4.a.4 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.a.4 |  |
| A.4.a.4.a | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.a.4.a |  |
| A.4.a.4.b | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.a.4.b |  |

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.4.b | For space station(s) onboard non-geostationary satellite(s): |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b |  |
| A.4.b.1 | the number of orbital planes |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.1 |  |
| A.4.b.2 | the reference body code |  |  |  |  |  | X | X |  | X |  |  |  |  | A.4.b.2 |  |
| A.4.b.3 | For space stations of a non-geostationary fixed-satellite service system operating in the band 3 400-4 200 MHz: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.3 |  |
| A.4.b.3.a | the maximum number of space stations (NN) in a non-geostationary-satellite system simultaneously transmitting on a co-frequency basis in the fixed-satellite service in the Northern Hemisphere |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.3.a |  |
| A.4.b.3.b | the maximum number of space stations (NS) in a non-geostationary-satellite system simultaneously transmitting on a co-frequency basis in the fixed-satellite service in the Southern Hemisphere |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.3.b |  |
| A.4.b.4 | For each orbital plane, where the Earth is the reference body: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.4 |  |
| A.4.b.4.a | the angle of inclination (ij) of the orbital plane with respect to the Earth’s equatorial plane (0° ≤ ij < 180°) |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.a |  |
| A.4.b.4.b | the number of satellites in the orbital plane |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.b |  |
| A.4.b.4.c | the period |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.c |  |
| A.4.b.4.d | the altitude, in kilometres, of the apogee of the space station |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.d |  |
| A.4.b.4.e | the altitude, in kilometres, of the perigee of the space station |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.e |  |
| A.4.b.4.f | the minimum altitude of the space station above the surface of the Earth at which any satellite transmits |  |  |  |  |  |  | X |  | X |  |  |  |  | A.4.b.4.f |  |
| A.4.b.5 | For space stations operating in a frequency band subject to the provisions of Nos. 9.11A, 9.12 or 9.12A, the data elements to characterize properly the orbital statistics of the non-geostationary-satellite system: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.5 |  |
| A.4.b.5.a | the right ascension of the ascending node (Ωj) for the j-th orbital plane, measured counter-clockwise in the equatorial plane from the direction of the vernal equinox to the point where the satellite makes its South-to-North crossing of the equatorial plane (0° ≤ Ωj < 360°) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.5.a |  |
| A.4.b.5.b | the initial phase angle (ωi) of the i-th satellite in its orbital plane at reference time t = 0, measured from the point of the ascending node (0° ≤ ωi < 360°) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.5.b |  |
| A.4.b.5.c | the argument of perigee (ωp), measured in the orbital plane, in the direction of motion, from the ascending node to the perigee (0° ≤ ωp < 360°) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.5.c |  |
| A.4.b.6 | For space stations operating in a frequency band subject to Nos. 22.5C, 22.5D or 22.5F, the data elements to characterize properly the orbital operation of the non-geostationary-satellite system: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.6 |  |
| A.4.b.6.a | For each range of latitudes: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.6.a |  |
| A.4.b.6.a.1 | the maximum number of non-geostationary satellites transmitting with overlapping frequencies to a given location |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.a.1 |  |
| A.4.b.6.a.2 | the associated start of the latitude range |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.a.2 |  |
| A.4.b.6.a.3 | the associated end of the latitude range |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.a.3 |  |
| A.4.b.6.b | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.6.b |  |
| A.4.b.6.c | an indicator showing whether the space station uses station-keeping to maintain a repeating ground track |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.c |  |
| A.4.b.6.d | if the space station uses station-keeping to maintain a repeating ground track, the time in seconds that it takes for the constellation to return to its starting position, i.e. such that all satellites are in the same location with respect to the Earth and each other |  |  |  |  |  |  |  |  | + |  |  |  |  | A.4.b.6.d |  |
| A.4.b.6.e | an indicator showing whether the space station should be modelled with a specific precession rate of the ascending node of the orbit instead of the J2 term |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.e |  |
| A.4.b.6.f | if the space station is to be modelled with a specific precession rate of the ascending node of the orbit instead of the J2 term, the precession rate in degrees/day, measured counter-clockwise in the equatorial plane |  |  |  |  |  |  |  |  | + |  |  |  |  | A.4.b.6.f |  |
| A.4.b.6.g | the longitude of the ascending node (θj) for the j-th orbital plane, measured counter-clockwise in the equatorial plane from the Greenwich meridian to the point where the satellite orbit makes its South-to-North crossing of the equatorial plane (0° ≤ θj < 360°) Note – For the evaluation of epfd a reference to a point on the Earth is used and hence the “longitude of the ascending node” is required. All satellites in the constellation must use the same reference time |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.g |  |
| A.4.b.6.h | the date (day:month:year) at which the satellite is at the location defined by the longitude of the ascending node (θj), (see Note under A.4.b.6.g) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.h |  |
| A.4.b.6.i | the time (hours:minutes) at which the satellite is at the location defined by the longitude of the ascending node (θj), (see Note under A.4.b.6.g) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.i |  |
| A.4.b.6.j | the longitudinal tolerance of the longitude of the ascending node |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.6.j |  |
| A.4.b.7 | For space stations operating in a frequency band subject to Nos. 22.5C, 22.5D or 22.5F, the data elements to characterize properly the performance of the non-geostationary-satellite system: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.7 |  |
| A.4.b.7.a | the maximum number of non-geostationary satellites receiving simultaneously with overlapping frequencies from the associated earth stations within a given cell |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.7.a |  |
| A.4.b.7.b | the average number of associated earth stations with overlapping frequencies per square kilometre within a cell |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.7.b |  |
| A.4.b.7.c | the average distance, in kilometres, between co-frequency cells |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.7.c |  |
| A.4.b.7.d | For the exclusion zone about the geostationary-satellite orbit: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.b.7.d |  |
| A.4.b.7.d.1 | the type of zone (based on topocentric angle, satellite-based angle or other method for establishing the exclusion zone) |  |  |  |  |  |  |  |  | X |  |  |  |  | A.4.b.7.d.1 |  |
| A.4.b.7.d.2 | if the zone is based on a topocentric angle or a satellite-based angle, the width of the zone, in degrees |  |  |  |  |  |  |  |  | + |  |  |  |  | A.4.b.7.d.2 |  |
| A.4.b.7.d.3 | if an alternative method is used for establishing the exclusion zone, a detailed description of the avoidance mechanism |  |  |  |  |  |  |  |  | + |  |  |  |  | A.4.b.7.d.3 |  |
| A.4.c | For an earth station: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.4.c |  |
| A.4.c.1 | the identity of the associated space station(s) with which communication is to be established |  |  |  |  |  |  |  |  |  | X |  |  |  | A.4.c.1 |  |
| A.4.c.2 | if communication is to be established with a geostationary space station, its orbital position |  |  |  |  |  |  |  |  |  | + |  |  |  | A.4.c.2 |  |

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.5 | COORDINATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.5 |  |
| A.5.a.1 | the symbol of any administration (see the Preface) with which coordination has been successfully effected Required only in the case of notification |  |  |  |  |  |  |  | + | + | + 1 |  |  |  | A.5.a.1 |  |
| A.5.a.2 | the symbol of any intergovernmental organization (see the Preface) with which coordination has been successfully effected Required only in the case of notification |  |  |  |  |  |  |  | + | + | + 1 |  |  |  | A.5.a.2 |  |
| A.5.b.1 | the symbol of any administration (see the Preface) with which coordination has been sought but not completed |  |  |  |  |  |  |  | O | O | O |  |  |  | A.5.b.1 |  |
| A.5.b.2 | the symbol of any intergovernmental organization (see the Preface) with which coordination has been sought but not completed |  |  |  |  |  |  |  | O | O |  |  |  |  | A.5.b.2 |  |
| A.5.c | the related provision code (see the Preface) under which coordination has been sought or completed if either A.5.a.1 (and A.5.a.2) or A.5.b.1 (and A.5.b.2) has been supplied |  |  |  |  |  |  |  | + | + | + 1 |  |  |  | A.5.c |  |
| A.6 | AGREEMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.6 |  |
| A.6.a | if appropriate, the symbol of any administration or administration representing a group of administrations (see the Preface) with which agreement has been reached, including where the agreement is to exceed the limits prescribed in these Regulations |  |  |  |  |  |  |  | + | + | + 1 | + | + | + | A.6.a |  |
| A.6.b | if appropriate, the symbol of any intergovernmental organization (see the Preface) with which agreement has been reached, including where the agreement is to exceed the limits prescribed in these Regulations |  |  |  |  |  |  |  | + | + | + 1 | + | + | + | A.6.b |  |
| A.6.c | if agreement has been reached, the related provision code (see the Preface) |  |  |  |  |  |  |  | + | + | + 1 | + | + | + | A.6.c |  |
| A.7 | SPECIFIC EARTH STATION OR RADIO ASTRONOMY STATION SITE CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.7 |  |
| A.7.a.1 | the horizon elevation angle, in degrees, for each azimuth around the earth station |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.a.1 |  |
| A.7.a.2 | the distance, in kilometres, from the earth station to the horizon for each azimuth around the earth station |  |  |  |  |  |  |  |  |  | O |  |  |  | A.7.a.2 |  |
| A.7.b.1 | the planned minimum angle of elevation of the antenna’s main beam axis, in degrees, from the horizontal plane For determining the minimum elevation angle of an earth station, due regard should be given to possible inclined-orbit operation of the associated geostationary space station In the case of an earth station, required for operation to geostationary satellites |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.b.1 | X |
| A.7.b.2 | the planned maximum angle of elevation of the antenna’s main beam axis, in degrees, from the horizontal plane |  |  |  |  |  |  |  |  |  |  |  |  |  | A.7.b.2 | X |
| A.7.c.1 | the start azimuth for the planned range of operating azimuthal angles for the antenna’s main beam axis, in degrees, clockwise from True North For determining the start azimuth of an earth station, due regard should be given to possible inclined-orbit operation of the associated geostationary space station In the case of an earth station, required for operation to geostationary satellites |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.c.1 | X |
| A.7.c.2 | the end azimuth for the planned range of operating azimuthal angles for the antenna’s main beam axis, in degrees, clockwise from True North For determining the end azimuth of an earth station, due regard should be given to possible inclined-orbit operation of the associated geostationary space station In the case of an earth station, required for operation to geostationary satellites |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.c.2 | X |
| A.7.d | the altitude, in metres, of the antenna above mean sea level |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.d |  |
| A.7.e | the minimum angle of elevation of the antenna’s main beam axis, in degrees, from the horizontal plane for each azimuth around the earth station Required for earth stations operating with non-geostationary space stations |  |  |  |  |  |  |  |  |  | + |  |  |  | A.7.e |  |
| A.7.f | the antenna diameter, in metres Required only for fixed-satellite service earth stations operating in the frequency bands 13.75-14 GHz, 14.5-14.75 GHz (in countries listed in Resolution 163 (WRC-15) not for feeder links for the broadcasting-satellite service), 14.5-14.8 GHz (in countries listed in Resolution 164 (WRC-15) not for feeder links for the broadcasting-satellite service), 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | A.7.f |  |
| A.8 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.8 |  |
| A.9 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.9 |  |
| A.10 | EARTH STATION COORDINATION AREA DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.10 |  |
| A.10.a | the diagrams shall be drawn to an appropriate scale, indicating, for both transmission and reception, the location of the earth station and its associated coordination areas, or the coordination area related to the service area in which it is intended to operate the mobile earth station Required only for notification |  |  |  |  |  |  |  |  |  | + |  |  |  | A.10.a |  |
| A.11 | REGULAR HOURS OF OPERATION |  |  |  |  |  |  |  |  |  |  |  |  |  | A.11 |  |
| A.11.a | the start time UTC |  |  |  |  |  |  |  |  |  |  | X | X |  | A.11.a |  |
| A.11.b | the stop time UTC |  |  |  |  |  |  |  |  |  |  | X | X |  | A.11.b |  |
| A.12 | RANGE OF AUTOMATIC GAIN CONTROL, in dB |  |  |  |  |  |  |  |  |  |  |  | X |  | A.12 |  |
| A.13 | REFERENCES TO THE PUBLISHED SPECIAL SECTIONS OF THE BUREAU'S INTERNATIONAL FREQUENCY INFORMATION CIRCULAR (see the Preface) |  |  |  |  |  |  |  |  |  |  |  |  |  | A.13 |  |
| A.13.a | the reference and number of the advance publication information in accordance with No. 9.1 or No. 9.1A |  |  |  |  |  |  |  | X | X | X |  |  |  | A.13.a |  |
| A.13.b | the reference and number of the coordination request in accordance with No. 9.6 In the case of notification of an earth station, the reference to the Special Section of the associated satellite network has to be provided In the case of notification of an earth station coordinated under No. 9.7A, the coordination Special Section number of this earth station has to be provided |  |  |  |  |  |  |  | X | X | X |  |  |  | A.13.b |  |
| A.13.c | the reference and number of the information in accordance with Article 4 of Appendix 30 |  |  |  |  |  |  |  |  |  |  | X |  |  | A.13.c |  |
| A.13.d | the reference and number of the information in accordance with Article 4 of Appendix 30A |  |  |  |  |  |  |  |  |  |  |  | X |  | A.13.d |  |
| A.13.e | the reference and number of the information in accordance with Article 6 of Appendix 30B |  |  |  |  |  |  |  |  |  | X |  |  | X | A.13.e |  |

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.14 | FOR STATIONS OPERATING IN A FREQUENCY BAND SUBJECT TO Nos. 22.5C, 22.5D OR 22.5F: SPECTRUM MASKS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.14 |  |
| A.14.a | For each e.i.r.p. mask used by the non-geostationary space station: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.14.a |  |
| A.14.a.1 | the mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.a.1 |  |
| A.14.a.2 | the lowest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.a.2 |  |
| A.14.a.3 | the highest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.a.3 |  |
| A.14.a.4 | the mask pattern defined in terms of the power in the reference bandwidth for a series of off-axis angles with respect to a specified reference point |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.a.4 |  |
| A.14.b | For each associated earth station e.i.r.p. mask: |  |  |  |  |  |  |  |  |  |  |  |  |  | A.14.b |  |
| A.14.b.1 | the mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.1 |  |
| A.14.b.2 | the lowest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.2 |  |
| A.14.b.3 | the highest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.3 |  |
| A.14.b.4 | the minimum elevation angle at which any associated earth station can transmit to a non-geostationary satellite |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.4 |  |
| A.14.b.5 | the minimum separation angle between the geostationary-satellite orbit arc and the associated earth station main beam-axis at which the associated earth station can transmit towards a non-geostationary satellite |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.5 |  |
| A.14.b.6 | the mask pattern defined in terms of the power in the reference bandwidth for a series of off-axis angles with respect to a specified reference point |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.b.6 |  |
| A.14.c | For each pfd mask used by the non-geostationary space station: Note – The space station pfd mask is defined by the maximum power flux-density generated by any space station in the interfering non-geostationary-satellite system as seen from any point on the surface of the Earth |  |  |  |  |  |  |  |  |  |  |  |  |  | A.14.c |  |
| A.14.c.1 | the mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.c.1 |  |
| A.14.c.2 | the lowest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.c.2 |  |
| A.14.c.3 | the highest frequency for which the mask is valid |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.c.3 |  |
| A.14.c.4 | the type of mask |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.c.4 |  |
| A.14.c.5 | the mask pattern of the power flux-density defined in three dimensions |  |  |  |  |  |  |  |  | X |  |  |  |  | A.14.c.5 |  |
| A.15 | COMMITMENT REGARDING COMPLIANCE WITH ADDITIONAL OPERATIONAL EQUIVALENT POWER FLUX-DENSITY, epfd↓, LIMITS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.15 |  |
| A.15.a | a commitment that the filed for system will meet the additional operational epfd↓ limits that are specified in Table 22-4A1 under No. 22.5I Required only for non-geostationary-satellite systems operating in the fixed-satellite service in the bands 10.7-11.7 GHz (in all Regions), 11.7-12.2 GHz (Region 2), 12.2-12.5 GHz (Region 3), and 12.5-12.75 GHz (Regions 1 and 3) |  |  |  |  |  |  |  |  | + |  |  |  |  | A.15.a |  |
| A.16 | COMMITMENT REGARDING COMPLIANCE WITH OFF-AXIS POWER LIMITATIONS, POWER FLUX-DENSITY (pfd) LIMITS OR SEPARATION DISTANCE |  |  |  |  |  |  |  |  |  |  |  |  |  | A.16 |  |
| A.16.a | a commitment that the associated earth stations operating with a geostationary-satellite network in the fixed-satellite service meet the off-axis power limitations given in Nos. 22.26 to 22.28 or 22.32 (as appropriate) under the conditions specified in Nos. 22.30, 22.31 and 22.34 to 22.39 Required only where the earth stations are subject to those power limitations |  |  |  |  |  |  |  | + |  |  |  |  |  | A.16.a |  |
| A.16.b | a commitment by administrations that the filed system will meet the single entry power flux-density limits that are specified in No. 5.502 Required only for specific earth station antennas less than 4.5 m in diameter operating with geostationary space stations in the fixed-satellite service in the band 13.75-14 GHz |  |  |  |  |  |  |  |  |  | + |  |  |  | A.16.b |  |
| A.16.c | a commitment by administrations that the earth station associated with the filed system will meet the separation distance as specified in No. 5.509E and the power flux-density limits that are specified in No. 5.509D |  |  |  |  |  |  |  | + |  |  |  |  |  | A.16.c |  |
| A.17 | COMPLIANCE WITH POWER FLUX-DENSITY, pfd, LIMITS |  |  |  |  |  |  |  |  |  |  |  |  |  | A.17 |  |
| A.17.a | a commitment of compliance with per-satellite power-flux density level produced at the Earth’s surface of −129 dB(W/(m2 · MHz)) in any 1 MHz band under free space propagation conditions Required only for satellite systems operating in the radionavigation-satellite service in the band 1 164-1 215 MHz |  |  |  |  |  |  |  | + | + |  |  |  |  | A.17.a |  |
| A.17.b.1 | the calculated aggregate power flux-density produced at the Earth’s surface by any geostationary radionavigation-satellite system in the frequency band 4 990-5 000 MHz in a 10 MHz bandwidth, as defined in resolves 1 of Resolution 741 (Rev.WRC-15) Required only for geostationary-satellite systems operating in the radionavigation-satellite service in the frequency band 5 010-5 030 MHz |  |  |  |  |  |  |  | + |  |  |  |  |  | A.17.b.1 |  |
| A.17.b.2 | the calculated aggregate power flux-density produced at the Earth’s surface by all space stations within any radionavigation-satellite service system in the band 5 030-5 150 MHz in a 150 kHz bandwidth, as defined in No. 5.443B Required only for satellite systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz |  |  |  |  |  |  |  | + | + |  |  |  |  | A.17.b.2 |  |
| A.17.b.3 | the equivalent power flux-density produced at the Earth’s surface by all space stations within any non-geostationary radionavigation-satellite service system in the frequency band 4 990-5 000 MHz in a 10 MHz bandwidth, as defined in resolves 2 of Resolution 741 (Rev.WRC-15) Required only for non-geostationary-satellite systems operating in the radionavigation-satellite service in the frequency band 5 010-5 030 MHz |  |  |  |  |  |  |  |  | + |  |  |  |  | A.17.b.3 |  |

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.17.c | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | A.17.c |  |
| A.17.d | the mean power flux-density produced at the Earth’s surface by any spaceborne sensor, as defined in No. 5.549A for the frequency band 35.5-36 GHz or in Table 21-4 for the frequency band 9 900-10 400 MHz Required only for satellite systems operating in • the Earth exploration-satellite service (active) or space research service (active) in the frequency band 35.5-36 GHz • the Earth exploration-satellite service (active) in the frequency band 9 900-10 400 MHz |  |  |  |  |  |  |  | + | + |  |  |  |  | A.17.d |  |
| A.17.e.1 | the calculated equivalent power flux-density produced at the site of a radio astronomy station in the band 42.5-43.5 GHz, as defined in No. 5.551H Required only for non-geostationary-satellite systems operating in the fixed-satellite service and broadcasting-satellite service in the band 42-42.5 GHz |  |  |  |  |  |  |  |  | + |  |  |  |  | A.17.e.1 |  |
| A.17.e.2 | the calculated power flux-density produced at the site of a radio astronomy station in the band 42.5-43.5 GHz, as defined in No. 5.551I Required only for geostationary-satellite systems operating in the fixed-satellite service and broadcasting-satellite service in the band 42-42.5 GHz |  |  |  |  |  |  |  | + |  |  |  |  |  | A.17.e.2 |  |
| A.18 | COMPLIANCE WITH NOTIFICATION OF AIRCRAFT EARTH STATION(S) |  |  |  |  |  |  |  |  |  |  |  |  |  | A.18 |  |
| A.18.a | a commitment that the characteristics of the aircraft earth station (AES) in the aeronautical mobile-satellite service are within the characteristics of the specific and/or typical earth station published by the Bureau for the space station to which the AES is associated Required only for the band 14-14.5 GHz, when an aircraft earth station in the aeronautical mobile-satellite service communicates with a space station in the fixed-satellite service |  |  |  |  |  |  |  | + | + |  |  |  |  | A.18.a |  |
| A.19 | COMPLIANCE WITH § 6.26 OF ARTICLE 6 OF APPENDIX 30B |  |  |  |  |  |  |  |  |  |  |  |  |  | A.19 |  |
| A.19.a | a commitment that the use of the assignment shall not cause unacceptable interference to, nor claim protection from, those assignments for which agreement still needs to be obtained Required if the notice is submitted under § 6.25 of Article 6 of Appendix 30B |  |  |  |  |  |  |  |  |  |  |  |  | + | A.19.a |  |

TABLE B

CHARACTERISTICS TO BE PROVIDED FOR EACH SATELLITE ANTENNA BEAM OR 
EACH EARTH STATION OR RADIO ASTRONOMY ANTENNA    (Rev.WRC-15)

| Items in Appendix | B _ CHARACTERISTICS TO BE PROVIDED FOR EACH SATELLITE ANTENNA BEAM OR EACH EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B.1 | IDENTIFICATION AND DIRECTION OF THE SATELLITE ANTENNA BEAM |  |  |  |  |  |  |  |  |  |  |  |  |  | B.1 |  |
| B.1.a | the designation of the satellite antenna beam For an earth station, the designation of the satellite antenna beam of the associated space station |  |  |  |  |  |  | X | X | X | X | X | X | X | B.1.a |  |
| B.1.b | an indicator showing whether the antenna beam, under B.1.a, is fixed or whether it is steerable and / or reconfigurable |  |  |  |  |  |  | X | X | X |  | X | X | X | B.1.b |  |
| B.2 | TRANSMISSION / RECEPTION INDICATOR FOR THE BEAM OF THE SPACE STATION OR THE ASSOCIATED SPACE STATION |  |  |  |  |  |  | X | X | X | + 1 |  |  | X | B.2 |  |
| B.2bis | CONTINUOUS/NON-CONTINUOUS TRANSMISSION INDICATOR FOR THE BEAM OF THE SPACE STATION |  |  |  |  |  |  |  |  |  |  |  |  |  | B.2bis |  |
| B.2bis.a | an indicator specifying whether the space station only transmits when visible from the notified service area In the case of advance publication, required only for frequency assignments of a non-geostationary satellite transmitting beam In the case of notification or coordination of a non-geostationary-satellite network, required only for frequency assignments of a non-geostationary satellite transmitting beam of a satellite network not subject to Nos. 22.5C, 22.5D or 22.5F |  |  |  |  |  |  | + |  | + |  |  |  |  | B.2bis.a |  |
| B.2bis.b | in case of non-continuous transmission in item B.2bis.a, the minimum elevation angle above which transmissions occur when the space station is visible from the notified service area In the case of notification or coordination of a non-geostationary-satellite network, only for frequency assignments of a non-geostationary satellite transmitting beam of a satellite network not subject to Nos. 22.5C, 22.5D or 22.5F |  |  |  |  |  |  | O |  | O |  |  |  |  | B.2bis.b |  |
| B.3 | SPACE STATION ANTENNA CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3 |  |
| B.3.a | For each space station antenna: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3.a |  |
| B.3.a.1 | the maximum co-polar isotropic gain, in dBi Where a steerable beam (see No. 1.191) is used, if the effective boresight area (see No. 1.175) is identical with the global service area, the maximum antenna gain, in dBi, is applicable to all points on the Earth’s visible surface |  |  |  |  |  |  | X | X | X |  | X | X | X | B.3.a.1 |  |
| B.3.a.2 | if a non-elliptical beam, the maximum cross-polar isotropic antenna gain, in dBi |  |  |  |  |  |  |  |  |  |  | + | + |  | B.3.a.2 |  |
| B.3.b | Antenna gain contours: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3.b |  |
| B.3.b.1 | the co-polar antenna gain contours plotted on a map of the Earth’s surface, preferably in a radial projection from the satellite onto a plane perpendicular to the axis from the centre of the Earth to the satellite The space station antenna gain contours shall be drawn as isolines of the isotropic gain, at least for −2, −4, −6, −10 and −20 dB and at 10 dB intervals thereafter, as necessary, relative to the maximum antenna gain, when any of these contours is located either totally or partially anywhere within the limit of visibility of the Earth from the given geostationary satellite Whenever possible, the gain contours of the space station antenna should also be provided in a numerical format (e.g. equation or table) Where a steerable beam (see No. 1.191) is used, if the effective boresight area (see No. 1.175) is less than the global service area, the contours are the result of moving the boresight of the steerable beam around the limit defined by the effective boresight area and are to be provided as described above but shall also include the 0 dB relative gain isoline. In addition, for a steerable beam, except for the case of Appendix 30B, see also No. 21.16 (and its associated Rules of Procedure) The antenna gain contours shall include the effects of the planned inclination excursion, longitudinal tolerance and the planned pointing accuracy of the antenna Note – Taking due account of applicable technical restrictions and allowing some reasonable degree of flexibility for satellite operations, administrations should, to the extent practicable, align the areas the satellite steerable beams could cover with the service area of their networks with due regard to their service objectives. In the case of Appendix 30, 30A or 30B, required only for non-elliptical beams |  |  |  |  |  |  |  | X |  |  | + | + | + | B.3.b.1 |  |
| B.3.b.2 | if a non-elliptical beam, the cross-polar gain contours shall be provided as defined under B.3.b.1 |  |  |  |  |  |  |  |  |  |  | + | + |  | B.3.b.2 |  |
| B.3.c | Antenna radiation patterns: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3.c |  |
| B.3.c.1 | the co-polar antenna radiation pattern |  |  |  |  |  |  | X | + | X |  | + | + | + | B.3.c.1 |  |
|  | In the case of geostationary space stations required only where the antenna radiation beam is directed towards another satellite In the case of Appendix 30, 30A or 30B, required only for elliptical antenna beams |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B.3.c.2 | if an elliptical beam, the cross-polar antenna radiation pattern |  |  |  |  |  |  |  |  |  |  | + | + |  | B.3.c.2 |  |
| B.3.d | the pointing accuracy of the antenna In the case of Appendix 30, 30A or 30B, required only for elliptical beams |  |  |  |  |  |  |  | X |  |  | + | + | + | B.3.d |  |
| B.3.e | if the space station is operating in a frequency band allocated in the Earth-to-space direction and in the space-to-Earth direction, the gain of the antenna in the direction of those parts of the geostationary-satellite orbit which are not obstructed by the Earth. In the case of Appendix 30, required only in the frequency band 12.5-12.7 GHz |  |  |  |  |  |  |  | + |  |  | + | + |  | B.3.e |  |
| B.3.f | For a space station submitted in accordance with Appendix 30, 30A or 30B: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3.f |  |
| B.3.f.1 | the boresight or aim point of the antenna beam (longitude and latitude) |  |  |  |  |  |  |  |  |  |  | X | X | X | B.3.f.1 |  |
| B.3.f.2 | For each elliptical beam: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.3.f.2 |  |
| B.3.f.2.a | the rotational accuracy, in degrees |  |  |  |  |  |  |  |  |  |  | X | X | X | B.3.f.2.a |  |
| B.3.f.2.b | the major axis orientation, in degrees, anticlockwise from the Equator |  |  |  |  |  |  |  |  |  |  | X | X | X | B.3.f.2.b |  |
| B.3.f.2.c | the major axis, in degrees, at the half-power beamwidth |  |  |  |  |  |  |  |  |  |  | X | X | X | B.3.f.2.c |  |
| B.3.f.2.d | the minor axis, in degrees, at the half-power beamwidth |  |  |  |  |  |  |  |  |  |  | X | X | X | B.3.f.2.d |  |

| Items in Appendix | B _ CHARACTERISTICS TO BE PROVIDED FOR EACH SATELLITE ANTENNA BEAM OR EACH EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B.4 | ADDITIONAL CHARACTERISTICS FOR NON-GEOSTATIONARY SPACE STATION ANTENNA |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4 |  |
| B.4.a.1 | the reference number of each orbital plane in which the space station antenna characteristics are used |  |  |  |  |  |  | X |  | X |  |  |  |  | B.4.a.1 |  |
| B.4.a.2 | if the antenna characteristics of a space station are not common to every satellite in the specified orbital plane, the reference number of each satellite in the specified orbital plane, on which the space station antenna characteristics are used |  |  |  |  |  |  | + |  | + |  |  |  |  | B.4.a.2 |  |
| B.4.a.3 | For a space station submitted in accordance with Nos. 9.11A, 9.12, 9.12A or for active or passive sensors on board a non-geostationary-satellite network not subject to coordination under Section II of Article 9: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.a.3 |  |
| B.4.a.3.a | For the orientation angles of the satellite transmitting and receiving antenna beams: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.a.3.a |  |
| B.4.a.3.a.1 | the orientation angle alpha, in degrees, (see the most recent version of Recommendation ITU-R SM.1413) |  |  |  |  |  |  | X |  | X |  |  |  |  | B.4.a.3.a.1 |  |
| B.4.a.3.a.2 | the orientation angle beta, in degrees, (see the most recent version of Recommendation ITU-R SM.1413) |  |  |  |  |  |  | X |  | X |  |  |  |  | B.4.a.3.a.2 |  |
| B.4.b | For a space station submitted in accordance with Nos. 9.11A, 9.12 or 9.12A: |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.b |  |
| B.4.b.1 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.b.1 |  |
| B.4.b.1.a | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.b.1.a |  |
| B.4.b.1.b | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | B.4.b.1.b |  |
| B.4.b.2 | the satellite antenna gain G(θe) as a function of elevation angle (θe) at a fixed point on the Earth |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.2 |  |
| B.4.b.3 | the spreading loss as a function of elevation angle (to be determined by equations or provided in graphical format) |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.3 |  |
| B.4.b.4 | For each beam: |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.4 |  |
| B.4.b.4.a | the maximum beam peak e.i.r.p./4 kHz |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.4.a |  |
| B.4.b.4.b | the average beam peak e.i.r.p./4 kHz |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.4.b |  |
| B.4.b.4.c | the maximum beam peak e.i.r.p./1 MHz |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.4.c |  |
| B.4.b.4.d | the average beam peak e.i.r.p./1 MHz |  |  |  |  |  |  |  |  | X |  |  |  |  | B.4.b.4.d |  |
| B.4.b.5 | the calculated peak value of power flux-density produced within ±5° inclination of the geostationary-satellite orbit Required only for the fixed-satellite service (space-to-Earth) in the band 6 700-7 075 MHz |  |  |  |  |  |  |  |  | + |  |  |  |  | B.4.b.5 |  |
| B.5 | EARTH STATION ANTENNA CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |  | B.5 |  |
| B.5.a | the isotropic gain, in dBi, of the antenna in the direction of maximum radiation (see No. 1.160) |  |  |  |  |  |  |  |  |  | X |  |  |  | B.5.a |  |
| B.5.b | the half-power beamwidth, in degrees |  |  |  |  |  |  |  |  |  | + 1 |  |  |  | B.5.b |  |
| B.5.c | either the measured radiation pattern of the antenna or the reference radiation pattern to be used for coordination For coordination under No. 9.7A, the reference radiation pattern is to be provided |  |  |  |  |  |  |  |  |  | X |  |  |  | B.5.c |  |
| B.5.d | antenna dimension aligned with the geostationary arc (DGSO), in metres (see the most recent version of Recommendation ITU-R S.1855) except in the case of Appendix 30 or 30A |  |  |  |  |  |  |  |  |  | O |  |  |  | B.5.d |  |
| B.6 | RADIO ASTRONOMY STATION ANTENNA CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |  | B.6 |  |
| B.6.a | the antenna type (see the Preface) |  |  |  |  |  |  |  |  |  |  |  |  |  | B.6.a | X |
| B.6.b | the antenna dimensions (see the Preface) |  |  |  |  |  |  |  |  |  |  |  |  |  | B.6.b | X |
| B.6.c | the effective area of the antenna (see the Preface) |  |  |  |  |  |  |  |  |  |  |  |  |  | B.6.c | X |

TABLE C

CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS 
FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR 
RADIO ASTRONOMY ANTENNA      (Rev.WRC-15)

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.1 | FREQUENCY RANGE |  |  |  |  |  |  |  |  |  |  |  |  |  | C.1 |  |
| C.1.a | the lower limit of the frequency range within which the carriers and the bandwidth of the emission will be located for each Earth-to-space or space-to-Earth service area, or for each space-to-space relay |  |  |  |  | X | X | X |  |  |  |  |  | X | C.1.a |  |
| C.1.b | the upper limit of the frequency range within which the carriers and the bandwidth of the emission will be located for each Earth-to-space or space-to-Earth service area, or for each space-to-space relay |  |  |  |  | X | X | X |  |  |  |  |  | X | C.1.b |  |
| C.2 | ASSIGNED FREQUENCY (FREQUENCIES) |  |  |  |  |  |  |  |  |  |  |  |  |  | C.2 |  |
| C.2.a.1 | the assigned frequency (frequencies), as defined in No. 1.148 – in kHz up to 28 000 kHz inclusive – in MHz above 28 000 kHz to 10 500 MHz inclusive – in GHz above 10 500 MHz If the basic characteristics are identical, with the exception of the assigned frequency, a list of frequency assignments may be provided In the case of advance publication, required only for active sensors In the case of geostationary and non geo-stationary satellite networks, required for all space applications except passive sensors In the case of Appendix 30B, required only for notification under Article 8 |  |  |  |  |  |  | + | + | + | X | X | X | + | C.2.a.1 |  |
| C.2.a.2 | the channel number |  |  |  |  |  |  |  |  |  |  | X | X |  | C.2.a.2 |  |
| C.2.b | the centre of the frequency band observed – in kHz up to 28 000 kHz inclusive – in MHz above 28 000 kHz to 10 500 MHz inclusive – in GHz above 10 500 MHz In the case of satellite networks, required only for passive sensors |  |  |  |  |  |  | + | + | + |  |  |  |  | C.2.b | X |
| C.2.c | if the frequency assignment is to be filed under No. 4.4, an indication to that effect |  |  |  |  |  |  | + | + | + | + |  |  |  | C.2.c | + |
| C.3 | ASSIGNED FREQUENCY BAND |  |  |  |  |  |  |  |  |  |  |  |  |  | C.3 |  |
| C.3.a | the bandwidth of the assigned frequency band, in kHz (see No. 1.147) In the case of advance publication, required only for active sensors In the case of geostationary and non geo-stationary satellite networks, required for all space applications except passive sensors In the case of Appendix 30B, required only for notification under Article 8 |  |  |  |  |  |  | + | + | + | X | X | X | + | C.3.a |  |
| C.3.b | the bandwidth of the frequency band, in kHz, observed by the station In the case of satellite networks, required only for passive sensors |  |  |  |  |  |  | + | + | + |  |  |  |  | C.3.b | X |
| C.4 | CLASS OF STATION AND NATURE OF SERVICE |  |  |  |  |  |  |  |  |  |  |  |  |  | C.4 |  |
| C.4.a | the class of station, using the symbols from the Preface |  |  |  |  |  |  | X | X | X | X | X | X | X | C.4.a | X |
| C.4.b | the nature of service performed, using the symbols from the Preface |  |  |  |  |  |  | X | X | X | X |  |  |  | C.4.b | X |
| C.5 | RECEIVING SYSTEM NOISE TEMPERATURE |  |  |  |  |  |  |  |  |  |  |  |  |  | C.5 |  |
| C.5.a | the lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna of the space station In the case of satellite networks, required for all space applications except for active or passive sensors |  |  |  |  |  |  | + | + | + |  |  | X | X | C.5.a |  |
| C.5.b | the lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna of the earth station under clear-sky conditions This value shall be indicated for the nominal value of the angle of elevation when the associated transmitting station is onboard a geostationary satellite and, in other cases, for the minimum value of the angle of elevation |  |  |  |  |  |  |  |  |  | X |  |  |  | C.5.b |  |
| C.5.c | the overall receiving system noise temperature, in kelvins, referred to the output of the receiving antenna |  |  |  |  |  |  |  |  |  |  |  |  |  | C.5.c | X |
| C.5.d | For active sensors: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.5.d |  |
| C.5.d.1 | the system noise temperature at the output of the signal processor |  |  |  |  |  |  | X | X | X |  |  |  |  | C.5.d.1 |  |
| C.5.d.2 | the receiver noise bandwidth |  |  |  |  |  |  | X | X | X |  |  |  |  | C.5.d.2 |  |
| C.6 | POLARIZATION |  |  |  |  |  |  |  |  |  |  |  |  |  | C.6 |  |
| C.6.a | the type of polarization (see the Preface) In the case of circular polarization, this includes the sense of polarization (see Nos. 1.154 and 1.155) In the case of a space station submitted in accordance with Appendix 30 or 30A, see § 3.2 of Annex 5 to Appendix 30 |  |  |  |  |  |  | X | X | X | + 1 | X | X |  | C.6.a |  |
| C.6.b | if linear polarization is used, the angle, in degrees, measured counter-clockwise in a plane normal to the beam axis from the equatorial plane to the electric vector of the waves as seen from the satellite In the case of a space station submitted in accordance with Appendix 30 or 30A, see § 3.2 of Annex 5 to Appendix 30 |  |  |  |  |  |  | + | + | + | + 1 | + | + |  | C.6.b |  |
| C.7 | NECESSARY BANDWIDTH AND CLASS OF EMISSION (in accordance with Article 2 and Appendix 1) For advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9, changes to this information within the limits specified under C.1 shall not affect consideration of notification under Article 11 Not required for active or passive sensors |  |  |  |  |  |  |  |  |  |  |  |  |  | C.7 |  |
| C.7.a | the necessary bandwidth and the class of emission: for each carrier In the case of Appendix 30B, required only for notification under Article 8 |  |  |  |  |  |  | X | X | X | X | X | X | + | C.7.a |  |
| C.7.b | the carrier frequency or frequencies of the emission(s) |  |  |  |  |  |  | X | C | C | C |  |  |  | C.7.b |  |
| C.8 | POWER CHARACTERISTICS OF THE TRANSMISSION Not required for passive sensors |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8 |  |
| C.8.a | For the case where individual carriers can be identified: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8.a |  |
| C.8.a.1 | the maximum value of the peak envelope power, in dBW, supplied to the input of the antenna for each carrier type Required if neither C.8.b.1 nor C.8.b.3.a is provided |  |  |  |  |  |  | + | + | + | C |  |  |  | C.8.a.1 |  |

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.8.a.2 | the maximum power density, in dB(W/Hz), supplied to the input of the antenna for each carrier type2 In the case of Appendix 30B, required only for notification under Article 8 Required if neither C.8.b.2 nor C.8.b.3.b is provided |  |  |  |  |  |  | + | + | + | O |  |  | + | C.8.a.2 |  |
| C.8.b | For the case where it is not appropriate to identify individual carriers: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8.b |  |
| C.8.b.1 | the total peak envelope power, in dBW, supplied to the input of the antenna For coordination or notification of an Appendix 30A earth station the values shall include the maximum range of power control Required if neither C.8.a.1 nor C.8.b.3.a is provided |  |  |  |  |  |  | + | + | + | + 1 | X | X |  | C.8.b.1 |  |
| C.8.b.2 | the maximum power density, in dB(W/Hz), supplied to the input of the antenna2 For coordination or notification of an Appendix 30A earth station the values shall include the maximum range of power control In the case of Appendix 30B, required only for submission under Article 6 Required if neither C.8.a.2 nor C.8.b.3.b is provided |  |  |  |  |  |  | + | + | + | + 1 | X | X | + | C.8.b.2 |  |
| C.8.b.3 | For the case of active sensors: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8.b.3 |  |
| C.8.b.3.a | the mean peak envelope power, in dBW, supplied to the input of the antenna Required if neither C.8.a.1 nor C.8.b.1 is provided |  |  |  |  |  |  | + | + | + |  |  |  |  | C.8.b.3.a |  |
| C.8.b.3.b | the mean power density, in dB(W/Hz), supplied to the input of the antenna Required if neither C.8.a.2 nor C.8.b.2 is provided |  |  |  |  |  |  | + | + | + |  |  |  |  | C.8.b.3.b |  |
| C.8.c | For all space applications, except active or passive sensors: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8.c |  |
| C.8.c.1 | the minimum value of the peak envelope power, in dBW, supplied to the input of the antenna for each carrier type If not provided, the reason for absence under C.8.c.2 |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.c.1 |  |
| C.8.c.2 | if C.8.c.1 is not provided, the reason for absence of the minimum value of the peak envelope power |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.c.2 |  |
| C.8.c.3 | the minimum power density, in dB(W/Hz), supplied to the input of the antenna for each carrier type2 If not provided, the reason for absence under C.8.c.4 |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.c.3 |  |
| C.8.c.4 | if C.8.c.3 is not provided, the reason for absence of the minimum power density |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.c.4 |  |
| C.8.d.1 | the maximum total peak envelope power, in dBW, supplied to the input of the antenna for each contiguous satellite bandwidth For a satellite transponder, this corresponds to the maximum saturated peak envelope power Required only for a space-to-Earth or space-to-space link |  |  |  |  |  |  | O | + | + |  |  |  |  | C.8.d.1 |  |
| C.8.d.2 | each contiguous satellite bandwidth For the maximum saturated peak envelope power of the satellite transponder, this corresponds to the bandwidth of each transponder Required only for a space-to-Earth or space-to-space link, if different from item C.3.a |  |  |  |  |  |  | O | + | + |  |  |  |  | C.8.d.2 |  |
| C.8.e.1 | for space-to-Earth, Earth-to-space or space-to-space links. for each carrier type, the greater of either the carrier-to-noise ratio, in dB, required to meet the performance of the link under clear-sky conditions or the carrier-to-noise ratio, in dB, required to meet the short-time objectives of the link inclusive of necessary margins If not provided, the reason for absence under C.8.e.2 |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.e.1 |  |
| C.8.e.2 | if C.8.e.1 is not provided, the reason for absence of the carrier-to-noise ratio |  |  |  |  |  |  | + | + | + | + 1 |  |  |  | C.8.e.2 |  |
| C.8.f.1 | the space station’s nominal equivalent isotropically radiated power(s) (e.i.r.p.) on the beam axis Required only for a space-to-space link |  |  |  |  |  |  | + |  |  |  |  |  |  | C.8.f.1 |  |
| C.8.f.2 | the associated space station’s nominal equivalent isotropically radiated power(s) (e.i.r.p.) on the beam axis Required only for a space-to-space link |  |  |  |  |  |  | + |  |  |  |  |  |  | C.8.f.2 |  |
| C.8.g.1 | the maximum aggregate power, in dBW, of all carriers (per transponder, if applicable) supplied to the input of the transmitting antenna of the earth station or the associated earth station Not required for coordination of a specific earth station under Nos. 9.15, 9.17 or 9.17A |  |  |  |  |  |  |  | C | C | C |  |  |  | C.8.g.1 |  |
| C.8.g.2 | the aggregate bandwidth of all carriers (per transponder, if applicable) supplied to the input of the transmitting antenna of the earth station or the associated earth station Not required for coordination of a specific earth station under Nos. 9.15, 9.17 or 9.17A |  |  |  |  |  |  |  | C | C | C |  |  |  | C.8.g.2 |  |
| C.8.g.3 | an indicator showing whether the bandwidth of the transponder corresponds to the aggregate bandwidth of all carriers (per transponder, if applicable) supplied to the input of the transmitting antenna of the earth station or the associated earth station Not required for coordination of a specific earth station under Nos. 9.15, 9.17 or 9.17A |  |  |  |  |  |  |  | C | C | C |  |  |  | C.8.g.3 |  |
| C.8.h | the maximum power density per Hz supplied to the input of the antenna, in dB(W/Hz), averaged over the necessary bandwidth |  |  |  |  |  |  |  |  |  |  | X | X | X | C.8.h |  |
| C.8.i | If power control is used, the maximum range of power control, in dB |  |  |  |  |  |  |  |  |  |  |  | + |  | C.8.i |  |
| C.8.j | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | C.8.j |  |
| C.9 | INFORMATION ON MODULATION CHARACTERISTICS For all space applications except active or passive sensors |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9 |  |
| C.9.a | For each carrier, according to the nature of the signal modulating the carrier: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a |  |
| C.9.a.1 | the type of modulation In the case of a non-geostationary space station required only for Nos. 9.11A, 9.12 or 9.12A |  |  |  |  |  |  | O | C | + |  | X | X |  | C.9.a.1 |  |
| C.9.a.2 | For a carrier frequency modulated by a frequency-division multichannel telephony baseband (FDM/FM) or by a signal that can be represented by a multichannel telephony baseband: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a.2 |  |
| C.9.a.2.a | the lowest frequency of the baseband |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.2.a |  |
| C.9.a.2.b | the highest frequency of the baseband |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.2.b |  |

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.9.a.2.c | the r.m.s. frequency deviation of the pre-emphasized signal for a test tone as a function of baseband frequency |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.2.c |  |
| C.9.a.3 | For a carrier frequency modulated by a television signal: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a.3 |  |
| C.9.a.3.a | the peak-to-peak frequency deviation of the pre-emphasized signal |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.3.a |  |
| C.9.a.3.b | the pre-emphasis characteristic |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.3.b |  |
| C.9.a.3.c | if applicable, the characteristics of the multiplexing of the video signal with the sound signal(s) or other signals |  |  |  |  |  |  | O | C | C |  | + | + |  | C.9.a.3.c |  |
| C.9.a.4 | For a carrier phase-shift modulated by a digital signal: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a.4 |  |
| C.9.a.4.a | the bit rate |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.4.a |  |
| C.9.a.4.b | the number of phases |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.4.b |  |
| C.9.a.5 | For an amplitude modulated carrier (including single sideband): |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a.5 |  |
| C.9.a.5.a | the nature of the modulating signal, as precisely as possible |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.5.a |  |
| C.9.a.5.b | the kind of amplitude modulation used |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.5.b |  |
| C.9.a.6 | For a frequency modulated carrier: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.a.6 |  |
| C.9.a.6.a | the peak-to-peak frequency deviation, in MHz, of the energy dispersal waveform |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.6.a |  |
| C.9.a.6.b | the sweep frequency, in kHz, of the energy dispersal waveform |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.6.b |  |
| C.9.a.6.c | the energy dispersal waveform |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.6.c |  |
| C.9.a.7 | if other forms of modulation than frequency modulation, are being used, the type of energy dispersal |  |  |  |  |  |  | O | C | C |  | + | + |  | C.9.a.7 |  |
| C.9.a.8 | for all other types of modulation, such particulars as may be useful for an interference study |  |  |  |  |  |  | O | C | C |  |  |  |  | C.9.a.8 |  |
| C.9.a.9 | the TV standard |  |  |  |  |  |  | O | C | C |  | X | X |  | C.9.a.9 |  |
| C.9.b | For analogue carriers: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.b |  |
| C.9.b.1 | the sound-broadcasting characteristics |  |  |  |  |  |  |  |  |  |  | X | X |  | C.9.b.1 |  |
| C.9.b.2 | the composition of the baseband |  |  |  |  |  |  |  |  |  |  | X | X |  | C.9.b.2 |  |
| C.9.c | For a non-geostationary space station submitted in accordance with Nos. 9.11A, 9.12 or 9.12A: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.c |  |
| C.9.c.1 | the type of multiple access |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.c.1 |  |
| C.9.c.2 | the spectrum mask |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.c.2 |  |
| C.9.d | For stations operating in a frequency band subject to Nos. 22.5C, 22.5D or 22.5F: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.9.d |  |
| C.9.d.1 | the type of mask |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.d.1 |  |
| C.9.d.2 | the pfd mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.d.2 |  |
| C.9.d.3 | the space station’s e.i.r.p. mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.d.3 |  |
| C.9.d.4 | the associated earth station’s e.i.r.p. mask identification code |  |  |  |  |  |  |  |  | X |  |  |  |  | C.9.d.4 |  |
| C.10 | TYPE AND IDENTITY OF THE ASSOCIATED STATION(S) (the associated station may be another space station, a typical earth station of the network or a specific earth station) For all space applications except active or passive sensors |  |  |  |  |  |  |  |  |  |  |  |  |  | C.10 |  |
| C.10.a | For an associated space station: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.10.a |  |
| C.10.a.1 | the identity of the station |  |  |  |  |  |  | X | X | X |  |  |  |  | C.10.a.1 |  |
| C.10.a.2 | if the associated space station is in the geostationary orbit, its nominal longitude |  |  |  |  |  |  | + | + | + |  |  |  |  | C.10.a.2 |  |
| C.10.b | For an associated earth station: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.10.b |  |
| C.10.b.1 | the name of the station |  |  |  |  |  |  | X | X | X |  |  | X |  | C.10.b.1 |  |
| C.10.b.2 | the type of station (specific or typical) |  |  |  |  |  |  | X | X | X |  |  |  |  | C.10.b.2 |  |
| C.10.c | For a specific associated earth station: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.10.c |  |
| C.10.c.1 | the geographical coordinates of the antenna site |  |  |  |  |  |  | X | X | X |  |  | X |  | C.10.c.1 |  |
| C.10.c.2 | the country or geographical area in which the earth station is located, using the symbols from the Preface |  |  |  |  |  |  | X | X | X |  |  | X |  | C.10.c.2 |  |
| C.10.d | For an associated earth station (whether specific or typical): |  |  |  |  |  |  |  |  |  |  |  |  |  | C.10.d |  |
| C.10.d.1 | the class of station, using the symbols from the Preface |  |  |  |  |  |  | X | X | X |  |  |  |  | C.10.d.1 |  |
| C.10.d.2 | the nature of service performed, using the symbols from the Preface |  |  |  |  |  |  | X | X | X |  |  |  |  | C.10.d.2 |  |
| C.10.d.3 | the isotropic gain, in dBi, of the antenna in the direction of maximum radiation (see No. 1.160) |  |  |  |  |  |  | X | X | X |  | X | X | X | C.10.d.3 |  |
| C.10.d.4 | the beamwidth, in degrees, between the half-power points (described in detail if not symmetrical) |  |  |  |  |  |  | O | X | X |  | X | X | X | C.10.d.4 |  |
| C.10.d.5.a | either the measured co-polar radiation pattern of the antenna or the co-polar reference radiation pattern |  |  |  |  |  |  | X | X | X |  | X | X | X | C.10.d.5.a |  |
| C.10.d.5.b | either the measured cross-polar radiation pattern of the antenna or the cross-polar reference radiation pattern |  |  |  |  |  |  |  |  |  |  | X | X |  | C.10.d.5.b |  |
| C.10.d.6 | if the associated station is a receiving earth station, the lowest total receiving system noise temperature, in kelvins, referred to the output of the receiving antenna of the earth station under clear-sky conditions |  |  |  |  |  |  | + | + | + |  |  |  | + | C.10.d.6 |  |
| C.10.d.7 | the antenna diameter, in metres In cases other than Appendix 30A, required for fixed-satellite service networks operating in the frequency bands 13.75-14 GHz, 14.5-14.75 GHz (in countries listed in Resolution 163 (WRC-15) not for feeder links for the broadcasting-satellite service), 14.5-14.8 GHz (in countries listed in Resolution 164 (WRC-15) not for feeder links for the broadcasting-satellite service), 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) and for maritime mobile-satellite service networks operating in the frequency band 14-14.5 GHz |  |  |  |  |  |  |  | + | + |  |  | X |  | C.10.d.7 |  |
| C.10.d.8 | the equivalent antenna diameter (i.e. the diameter, in metres, of a parabolic antenna with the same off-axis performance as the receiving associated earth station antenna) |  |  |  |  |  |  |  |  |  |  | X |  |  | C.10.d.8 |  |
| C.10.d.9 | antenna dimension aligned with the geostationary arc (DGSO), in metres (see the most recent version of Recommendation ITU-R S.1855) except in the case of Appendix 30 or 30A |  |  |  |  |  |  |  | O |  |  |  |  | O | C.10.d.9 |  |

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.11 | SERVICE AREA (S) For all space applications except active or passive sensors |  |  |  |  |  |  |  |  |  |  |  |  |  | C.11 |  |
| C.11.a | the service area or areas of the satellite beam on the Earth, when the associated transmitting or receiving stations are earth stations For a space station submitted in accordance with Appendix 30, 30A or 30B, the service area identified by a set of a maximum of 100 test points and by a service area contour on the surface of the Earth or defined by a minimum elevation angle Note – When an assignment converted from an allotment is reinstated in the Appendix 30B Plan, the notifying administration may choose a maximum of 20 test points within its national territory for the reinstated allotment |  |  |  |  |  |  | X | X | X |  | X | X | X | C.11.a |  |
| C.11.b | the appropriate information required to calculate the affected region (as defined in Recommendation ITU-R M.1187-1) Required only for a non-geostationary space station in the mobile-satellite service submitted in accordance with No. 9.11A |  |  |  |  |  |  |  |  | + |  |  |  |  | C.11.b |  |
| C.12 | REQUIRED PROTECTION RATIO |  |  |  |  |  |  |  |  |  |  |  |  |  | C.12 |  |
| C.12.a | the minimum acceptable aggregate carrier-to-interference ratio, if less than 21 dB The carrier-to-interference ratio is to be expressed in terms of the power averaged over the necessary bandwidth of the modulated wanted and interfering signals, assuming both the desired carrier and interfering signals have equivalent bandwidths and modulation types |  |  |  |  |  |  |  |  |  |  |  |  | + | C.12.a |  |
| C.13 | CHARACTERISTICS OF OBSERVATIONS FOR RADIO ASTRONOMY STATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  | C.13 |  |
| C.13.a | the class of observations to be taken on the frequency band shown under C.3.b – Class A observations are those in which the sensitivity of the equipment is not a primary factor – Class B observations are those of such a nature that they can be made only with advanced low-noise receivers using the best techniques |  |  |  |  |  |  |  |  |  |  |  |  |  | C.13.a | X |
| C.13.b | the type of radio astronomy station in the frequency band shown under C.3.b – Single-dish, “S”, telescope used for spectral-line or continuum observations using single-dishes or closely connected arrays – Very long baseline interferometry (VLBI), “V”, station used only for VLBI observations |  |  |  |  |  |  |  |  |  |  |  |  |  | C.13.b | X |
| C.13.c | the minimum elevation angle θmin at which the radio astronomy station conducts single-dish or VLBI observations in the frequency band |  |  |  |  |  |  |  |  |  |  |  |  |  | C.13.c | X |
| C.14 | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  | C.14 |  |
| C.15 | DESCRIPTION OF THE GROUP(S) REQUIRED IN THE CASE OF NON-SIMULTANEOUS EMISSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  | C.15 |  |
| C.15.a | if part of an exclusive operation group, the group identification code |  |  |  |  |  |  |  |  |  |  | + | + | + | C.15.a |  |
| C.16 | DESCRIPTION OF ACTIVE AND PASSIVE SENSOR SYSTEMS |  |  |  |  |  |  |  |  |  |  |  |  |  | C.16 |  |
| C.16.a | For active sensors: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.16.a |  |
| C.16.a.1 | the pulse length, in μs |  |  |  |  |  |  | X | X | X |  |  |  |  | C.16.a.1 |  |
| C.16.a.2 | the pulse repetition frequency, in kHz |  |  |  |  |  |  | X | X | X |  |  |  |  | C.16.a.2 |  |
| C.16.b | For passive sensors: |  |  |  |  |  |  |  |  |  |  |  |  |  | C.16.b |  |
| C.16.b.1 | the sensitivity threshold, in kelvins |  |  |  |  |  |  | X | X | X |  |  |  |  | C.16.b.1 |  |

TABLE D

OVERALL LINK CHARACTERISTICS

| Items in Appendix | D _ OVERALL LINK CHARACTERISTICS |  |  |  |  | Advance publication of a geostationary- satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | For non-planned services, this data may be provided by administrations that so desire but only when simple frequency-changing transponders are used on the space station onboard a geostationary satellite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D.1 | CONNECTION BETWEEN EARTH-TO-SPACE AND SPACE-TO-EARTH FREQUENCIES IN THE NETWORK |  |  |  |  |  |  |  |  |  |  |  |  |  | D.1 |  |
| D.1.a | the connection between uplink and downlink frequency assignments for each intended combination of receiving and transmitting beams In the case of Appendix 30 or 30A, required only in Region 2 In the case of Appendix 30B, required except for submission of one link only |  |  |  |  |  |  |  | O |  |  | + | + | + | D.1.a |  |
| D.2 | TRANSMISSION GAINS AND ASSOCIATED EQUIVALENT SATELLITE LINK NOISE TEMPERATURES |  |  |  |  |  |  |  |  |  |  |  |  |  | D.2 |  |
| D.2.a | For each entry under D.1.a: |  |  |  |  |  |  |  |  |  |  |  |  |  | D.2.a |  |
| D.2.a.1 | the lowest equivalent satellite link noise temperature These values shall be indicated for the nominal value of the angle of elevation |  |  |  |  |  |  |  | O |  |  |  |  |  | D.2.a.1 |  |
| D.2.a.2 | the associated transmission gain of the lowest equivalent satellite link noise temperature These values shall be indicated for the nominal value of the angle of elevation The transmission gain is evaluated from the output of the receiving antenna of the space station to the output of the receiving antenna of the earth station |  |  |  |  |  |  |  | O |  |  |  |  |  | D.2.a.2 |  |
| D.2.b.1 | the values of associated equivalent satellite link noise temperature that correspond to the highest ratio of transmission gain to equivalent satellite link noise temperature |  |  |  |  |  |  |  | O |  |  |  |  |  | D.2.b.1 |  |
| D.2.b.2 | the values of transmission gain that correspond to the highest ratio of transmission gain to equivalent satellite link noise temperature |  |  |  |  |  |  |  | O |  |  |  |  |  | D.2.b.2 |  |

APPENDIX 5 (REV.WRC-15)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

1	For the purpose of effecting coordination under Article 9, except in the case under No. 9.21, and for identifying the administrations with which coordination is to be effected, the frequency assignments to be taken into account are those in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the frequency band is allocated with equal rights or a higher category1 of allocation, which might affect or be affected, as appropriate, and which are:      (WRC-15)

a)	in conformity with No. 11.312; and

b)	either recorded in the Master International Frequency Register (Master Register) with a favourable finding with respect to No. 11.32; or

c)	recorded in the Master Register with an unfavourable finding with respect to No. 11.32 and a favourable finding with respect to No. 11.32A or No. 11.33, as appropriate; or

cbis)	recorded in the Master Register under No. 11.41; or     (WRC-03)

d)	coordinated under the provisions of Article 9; or

e)	included in the coordination procedure with effect from the date of receipt3 by the Radiocommunication Bureau, in accordance with No. 9.34, of those characteristics specified in Appendix 4 as mandatory or required, or from the date of dispatch, in accordance with No. 9.29, of the appropriate information listed in Appendix 4; or      (WRC-15)

f)	where appropriate, in conformity with a world or regional allotment or assignment plan and the associated provisions;

g)	for terrestrial radiocommunication stations or earth stations operating in the opposite direction of transmission4 and, in addition, operating in accordance with these Regulations, or to be so operated prior to the date of bringing the earth station assignment into service, or within the next three years from the date of dispatch of coordination data under No. 9.29, whichever is the longer, or from the date of the publication referred to in No. 9.38, as appropriate.     (WRC-2000)

2	For the application of No. 9.21, the agreement of an administration may be required with respect to the frequency assignments in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the band is allocated with equal rights or a higher category of allocation, which may affect or be affected, as appropriate, and:

a)	in cases involving a station in a space radiocommunication service with respect to any other station or involving a terrestrial radiocommunication station with respect to an earth station:

i)	which are in conformity with No. 11.31, and comply with the relevant conditions listed in § 1 b) to 1 g); or

ii)	for which the procedure under No. 9.21 has been initiated, with effect from the date of receipt by the Bureau, in accordance with No. 9.34, of the basic characteristics specified in Appendix 4;

or

b)	for terrestrial radiocommunication stations operating in accordance with these Regulations, or to be so operated prior to the date of bringing the other terrestrial station assignment into service, or within the next three months, whichever is the longer.

3	For each of the frequency assignments to a station of a terrestrial or space radiocommunication service referred to in § 1 and 2 above, the level of interference shall be determined using the method referred to in Table 5-1 which is appropriate to the particular case.

4	The assignment is considered to affect or be affected, as appropriate, and coordination must be sought under the procedure of Article 9, if:

a)	the threshold levels given in Table 5-1 are exceeded; and

b)	the condition specified in Table 5-1 is applicable.

5	Threshold values to determine whether coordination under No. 9.11A is required are given in Table 5-2.

6	No coordination is required:

a)	when the use of a new frequency assignment will not cause or suffer, as appropriate, in respect of any service of another administration, an increase in the level of interference above the threshold calculated in accordance with the method referred to in Tables 5-1 and 5-2; or

b)	when the characteristics of a new or a modified frequency assignment or a new earth station are within the limits of those of a frequency assignment which has previously been coordinated; or

c)	to change the characteristics of an existing assignment in such a way as not to increase the interference to or from, as appropriate, the assignments of other administrations; or

d)	for assignments to stations comprising a satellite network in relation to assignments of other satellite networks:

i)	for a new frequency assignment to a receiving station, when the notifying administration states that it accepts the interference resulting from the frequency assignments referred to in No. 9.27; or

ii)	between earth stations using frequency assignments in the same direction (either Earth-to-space or space-to-Earth); or

e)	for assignments to earth stations in relation to terrestrial stations or earth stations operating in the opposite direction of transmission, when an administration proposes:

i)	to bring into use an earth station the coordination area of which does not include any of the territory of any other country;

ii)	to operate a mobile earth station. However, if the coordination area associated with the operation of such a mobile earth station includes any of the territory of another country, the operation of such a station shall be subject to agreement on coordination between the administrations concerned. This agreement shall apply to the characteristics of the mobile earth station(s), or to the characteristics of a typical mobile earth station, and shall apply to a specified service area. Unless otherwise stipulated in the agreement, it shall apply to any mobile earth stations in the specified service area provided that interference caused by them shall not be greater than that caused by a typical earth station for which the technical characteristics appear in the notice and have been or are being submitted in accordance with Section I of Article 11; or

iii)	to bring into use a new frequency assignment to a receiving earth station and the notifying administration states that it accepts the interference resulting from existing and future terrestrial station assignments or assignments to earth stations operating in the opposite direction of transmission. In such case, administrations responsible for the terrestrial stations or earth stations operating in the opposite direction of transmission are not required to apply the provisions of No. 9.18 or No. 9.17A of Article 9 respectively:

f)	to bring into use an assignment to a terrestrial station or an earth station operating in the opposite direction of transmission which is located, in relation to an earth station, outside the coordination area of that earth station; or

g)	to bring into use an assignment to a terrestrial station or an earth station operating in the opposite direction of transmission within the coordination area of an earth station, provided that the proposed assignment to a terrestrial station or an earth station operating in the opposite direction of transmission is outside any part of a frequency band coordinated for reception by that earth station.

TABLE 5-1     (Rev.WRC-15)

Technical conditions for coordination

(see Article 9)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz 7 025-7 075 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 7° of the nominal orbital position of a proposed network in the FSS |  | With respect to the space services listed in the threshold/condition column in the frequency bands in 1), 2), 2bis), 3), 4), 5), 6), 7) and 8), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.8 GHz | i) Bandwidth overlap, and ii) any network in the FSS or broadcasting-satellite service (BSS), not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 6° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan iii) in the band 14.5-14.8 GHz any network in the space research service (SRS) or FSS not subject to a Plan and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of ±6° of the nominal orbital position of a proposed network in the SRS or FSS not subject to a Plan |  |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 2bis) 13.4-13.65 GHz (Region 1) | i) Bandwidth overlap, and ii) any network in the space research service (SRS) or any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of ±6° of the nominal orbital position of a proposed network in the FSS or SRS |  |  |
|  |  | 3) 17.7-20.2 GHz, (Regions 2 and 3), 17.3-20.2 GHz (Region 1) and 27.5-30 GHz | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |
|  |  | 4) 17.3-17.7 GHz (Regions 1 and 2) | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 5) 17.7-17.8 GHz | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS NOTE – No. 5.517 applies in Region 2. |  |  |
|  |  | 6) 18.0-18.3 GHz (Region 2) 18.1-18.4 GHz (Regions 1 and 3) | i) Bandwidth overlap, and ii) any network in the FSS or meteorological-satellite service and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS or the meteorological-satellite service |  |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 6bis) 21.4-22 GHz (Regions 1 and 3) | i) Bandwidth overlap; and ii) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of ±12° of the nominal orbital position of a proposed network in the BSS (see also Resolutions 554 (WRC-12) and 553 (WRC-12)). |  | No. 9.41 does not apply. |
|  |  | 7) Bands above 17.3 GHz, except those defined in § 3) and 6) | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |
|  |  | 8) Bands above 17.3 GHz except those defined in § 4), 5) and 6bis) | i) Bandwidth overlap, and ii) any network in the FSS or BSS, not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 16° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan, except in the case of a network in the FSS with respect to a network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 9) All frequency bands, other than those in 1), 2), 2bis), 3), 4), 5), 6), 6bis), 7) and 8), allocated to a space service, and the bands in 1), 2), 2bis), 3), 4), 5), 6), 6bis), 7) and 8) where the radio service of the proposed network or affected networks is other than the space services listed in the threshold/ condition column, or in the case of coordination of space stations operating in the opposite direction of transmission | i) Bandwidth overlap, and ii) Value of ∆T/T exceeds 6% | Appendix 8 | In application of Article 2A of Appendix 30 for the space operation functions using the guardbands defined in § 3.9 of Annex 5 of Appendix 30, the threshold/condition specified for the FSS in the bands in 2) applies. In application of Article 2A of Appendix 30A for the space operation functions using the guardbands defined in § 3.1 and 4.1 of Annex 3 of Appendix 30A, the threshold/condition specified for the FSS in the bands in 7) applies |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7A GSO earth station/ non-GSO system | A specific earth station in a GSO satellite network in the FSS in respect of a non-GSO satellite system in the FSS | 10.7-11.7 GHz (space-to-Earth) 11.7-12.2 GHz (space-to-Earth) in Region 2 12.2-12.75 GHz (space-to-Earth) in Region 3 12.5-12.75 GHz (space-to-Earth) in Region 1 17.8-18.6 GHz (space-to-Earth), and 19.7-20.2 GHz (space-to-Earth) | i) Bandwidths overlap; and ii) the GSO satellite network has specific receive earth stations which meet all of the following conditions: a) earth station antenna maximum isotropic gain greater than or equal to 64 dBi for the frequency bands 10.7-12.75 GHz or 68 dBi for the frequency bands 17.8-18.6 GHz and 19.7-20.2 GHz; b) G/T of 44 dB/K or higher; c) emission bandwidth of 250 MHz or higher for the frequency bands below 12.75 GHz or 800 MHz or higher for the frequency bands above 17.8 GHz; and | i) Check by using the assigned frequencies and bandwidths; ii) use the maximum antenna gain (G), the lowest total receiving system noise temperature (T), and the emission bandwidth of the specific receive earth station as given in the Appendix 4 data; and | The threshold/condition for coordination does not apply to typical receive earth stations operating in GSO satellite networks |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7A GSO earth station/ non-GSO system (cont.) |  |  | iii) the equivalent power flux-density, epfd, from the non-GSO satellite system exceeds: a) in the frequency band 10.7-12.75 GHz: −174.5 dB(W/(m2 · 40 kHz)) for any percentage of time for non-GSO satellite systems with all satellites only operating at or below 2 500 km altitude, or −202 dB(W/(m2 · 40 kHz)) for any percentage of the time for non-GSO satellite systems with any satellites operating above 2 500 km altitude; b) in the frequency bands 17.8-18.6 GHz or 19.7-20.2 GHz: −157 dB(W/(m2 · MHz)) for any percentage of time for non-GSO satellite systems with all satellites only operating at or below 2 500 km altitude, or −185 dB(W/(m2 · MHz)) for any percentage of the time for non-GSO satellite systems with any satellites operating above 2 500 km altitude | iii) use the epfd radiated by the non-GSO FSS satellite system into the earth station employing the very large antenna when this antenna is pointed towards the wanted GSO satellite |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7B Non-GSO system/GSO earth station | A non-GSO satellite system in the FSS in respect of a specific earth station in a GSO satellite network in the FSS | 10.7-11.7 GHz (space-to-Earth) 11.7-12.2 GHz (space-to-Earth) in Region 2 12.2-12.75 GHz (space-to-Earth) in Region 3 12.5-12.75 GHz (space-to-Earth) in Region 1 17.8-18.6 GHz (space-to-Earth), and 19.7-20.2 GHz (space-to-Earth) | i) Bandwidths overlap; and ii) the GSO satellite network has specific receive earth stations which meet all of the following conditions: a) earth station antenna maximum isotropic gain greater than or equal to 64 dBi for the frequency bands 10.7-12.75 GHz or 68 dBi for the frequency bands 17.8-18.6 GHz and 19.7-20.2 GHz; b) G/T of 44 dB/K or higher; c) emission bandwidth of 250 MHz or higher for the frequency bands below 12.75 GHz or 800 MHz or higher for the frequency bands above 17.8 GHz; and | i) Check by using the assigned frequencies and bandwidths; ii) use the maximum antenna gain (G), the lowest total receiving system noise temperature (T), and the emission bandwidth of the specific receive earth station as given in the Appendix 4 data; | The threshold/condition for coordination do not apply to typical receive earth stations operating in GSO satellite networks |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7B Non-GSO system/GSO earth station (cont.) |  |  | iii) the epfd from the non-GSO satellite system exceeds: a) in the frequency band 10.7-12.75 GHz: −174.5 dB(W/(m2 · 40 kHz)) for any percentage of time for non-GSO satellite systems with all satellites only operating at or below 2 500 km altitude, or −202 dB(W/(m2 · 40 kHz)) for any percentage of the time for non-GSO satellite systems with any satellites operating above 2 500 km altitude; b) in the frequency bands 17.8-18.6 GHz or 19.7-20.2 GHz: −157 dB(W/(m2 · MHz)) for any percentage of time for non-GSO satellite systems with all satellites only operating at or below 2 500 km altitude, or −185 dB(W/(m2 · MHz)) for any percentage of the time for non-GSO satellite systems with any satellites operating above 2 500 km altitude | iii) use the epfd radiated by the non-GSO FSS satellite system into the earth station employing the very large antenna when this antenna is pointed towards the wanted GSO satellite |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.11 GSO, non-GSO/ terrestrial | A space station in the BSS in any band shared on an equal primary basis with terrestrial services and where the BSS is not subject to a Plan, in respect of terrestrial services | 620-790 MHz (see Resolution 549 (WRC-07)) 1 452-1 492 MHz 2 310-2 360 MHz (No. 5.393) 2 535-2 655 MHz (Nos. 5.417A and 5.418) 17.7-17.8 GHz (Region 2) 74-76 GHz | Bandwidths overlap: The detailed conditions for the application of No. 9.11 in the bands 2 630-2 655 MHz and 2 605-2 630 MHz are provided in Resolution 539 (Rev.WRC-03) for non-GSO BSS (sound) systems pursuant to Nos. 5.417A and 5.418, and in Nos. 5.417A and 5.418 for GSO BSS (sound) networks pursuant to those provisions. | Check by using the assigned frequencies and bandwidths |  |
| No. 9.12 Non-GSO/ non-GSO | A station in a non-GSO satellite network in the frequency bands for which a footnote refers to No. 9.11A or No. 9.12, in respect of any other non-GSO satellite network, with the exception of coordination between earth stations operating in the opposite direction of transmission | Frequency bands for which a footnote refers to No. 9.11A or No. 9.12 | Bandwidths overlap | Check by using the assigned frequencies and bandwidths |  |
| No. 9.12A Non-GSO/ GSO | A station in a non-GSO satellite network in the frequency bands for which a footnote refers to No. 9.11A or No. 9.12A, in respect of any GSO satellite network, with the exception of coordination between earth stations operating in the opposite direction of transmission | Frequency bands for which a footnote refers to No. 9.11A or No. 9.12A | Bandwidths overlap | Check by using the assigned frequencies and bandwidths |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.13 GSO/ non-GSO | A station in a GSO satellite network in the frequency bands for which a footnote refers to No. 9.11A or No. 9.13, in respect of any other non-GSO satellite network, with the exception of coordination between earth stations operating in the opposite direction of transmission | Frequency bands for which a footnote refers to No. 9.11A or No. 9.13 | 1) Bandwidths overlap 2) For the band 1 668-1 668.4 MHz with respect to MSS network coordination with SRS (passive) networks, in addition to bandwidth overlap, the e.i.r.p. spectral density of mobile earth stations in a GSO network of the mobile-satellite service operating in this band exceeds −2.5 dB(W/4 kHz) or the power spectral density delivered to the mobile earth station antenna exceeds −10 dB(W/4 kHz) | 1) Check by using the assigned frequencies and bandwidths 2) Check by using MSS network Appendix 4 data |  |
| No. 9.14 Non-GSO/ terrestrial, GSO/ terrestrial | A space station in a satellite network in the frequency bands for which a footnote refers to No. 9.11A or to No. 9.14, in respect of stations of terrestrial services where threshold(s) is (are) exceeded | 1) Frequency bands for which a footnote refers to No. 9.11A; or 2) 11.7-12.2 GHz (Region 2 GSO FSS) 3) 5 030-5 091 MHz | 1) See § 1 of Annex 1 to this Appendix; In the bands specified in No. 5.414A, the detailed conditions for the application of No. 9.14 are provided in No. 5.414A for MSS networks or 2) In the band 11.7-12.2 GHz (Region 2 GSO FSS): −124 dB(W/(m2 · MHz)) for 0° 5 −124 + 0.5 ( – 5) dB(W/(m2 · MHz)) for 5° < 25 −114 dB(W/(m2 · MHz)) for > 25 where is the angle of arrival of the incident wave above the horizontal plane (degrees) 3) Bandwidth overlap | 1) See § 1 of Annex 1 to this Appendix |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.15 Non-GSO/ terrestrial | A specific earth station or a typical earth station, in respect of terrestrial stations in frequency bands for which a footnote refers to No. 9.11A allocated with equal rights to space and terrestrial services, where the coordination area of the earth station includes the territory of another country | Frequency bands for which a footnote refers to No. 9.11A | The coordination area of the earth station covers the territory of another administration | Appendix 7 |  |
| No. 9.16 Terrestrial/ non-GSO | A transmitting station in a terrestrial service within the coordination area of an earth station in a non-GSO satellite network in frequency bands for which a footnote refers to No. 9.11A | Frequency bands for which a footnote refers to No. 9.11A | Transmitting terrestrial station is situated within the coordination area of a receiving earth station |  | The coordination area of the affected earth station has already been determined using the calculation method of Appendix 7 |
| No. 9.17 GSO, non-GSO/ terrestrial | A specific earth station or a typical mobile earth station in frequency bands above 100 MHz allocated with equal rights to space and terrestrial services, in respect of terrestrial stations, where the coordination area of the earth station includes the territory of another country, with the exception of the coordination under No. 9.15 | Any frequency band allocated to a space service | The coordination area of the earth station covers the territory of another administration | Appendix 7 |  |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.17A GSO, non-GSO/ GSO, non-GSO | A specific earth station in respect of other earth stations operating in the opposite direction of transmission or for any typical mobile earth station in respect of specific earth stations operating in the opposite direction of transmission in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission, where the coordination area of the earth station includes the territory of another country or the earth station is located within the coordination area of a coordinated earth station, with the exception of coordination under No. 9.19 | Any frequency band allocated to a space service | The coordination area of the earth station covers the territory of another administration or the earth station is located within the coordination area of an earth station | Appendix 7 |  |
| No. 9.18 Terrestrial/ GSO, non-GSO | Any transmitting station of a terrestrial service in the bands referred to in No. 9.17 within the coordination area of an earth station, in respect of this earth station, with the exception of the coordination under Nos. 9.16 and 9.19 | Any frequency band allocated to a space service | Transmitting terrestrial station is situated within the coordination area of a receiving earth station | See Remarks column | The coordination area of the affected earth station has already been determined using the calculation method of No. 9.17 |

TABLE 5-1 (continued)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO | Any transmitting station of a terrestrial service or a transmitting earth station in the FSS (Earth-to-space) in a frequency band shared on an equal primary basis with the BSS, with respect to typical earth stations included in the service area of a space station in the BSS | 620-790 MHz (see Resolution 549 (WRC-07)) 1 452-1 492 MHz 2 310-2 360 MHz (terrestrial services in all three Regions in respect of BSS allocation in No. 5.393) 2 520-2 670 MHz (see No. 5.416) 11.7-12.7 GHz (see Article 6 of Appendix 30) 12.5-12.7 GHz (terrestrial services in Nos. 5.494 and 5.496 as well as in Regions 2 and 3, or transmitting earth station in the FSS (Earth-to-space) in Region 1, in respect of BSS allocation in Region 3) 12.7-12.75 GHz (terrestrial services in Nos. 5.494 and 5.496 as well as in Regions 2 and 3, or transmitting earth station in the FSS (Earth-to-space) in Regions 1 and 2, in respect of BSS allocation in Region 3) 17.7-17.8 GHz (terrestrial services in all three Regions in respect of BSS allocation in Region 2) | i) Necessary bandwidths overlap; and ii) the power flux-density (pfd) of the interfering station at the edge of the BSS service area exceeds the permissible level | Check by using the assigned frequencies and bandwidths | See also Article 6 of Appendix 30 |

TABLE 5-1 (end)     (Rev.WRC-15)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO (cont.) |  | 17.3-17.8 GHz (transmitting earth stations in the FSS (Earth-to-space) in respect of BSS allocation in Region 2) (see Article 4 of Appendix 30A) 40.5-42.5 GHz 74-76 GHz |  |  |  |
| No. 9.21 Terrestrial, GSO, non-GSO/ terrestrial, GSO, non-GSO | A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations referring to No. 9.21 | Band(s) indicated in the relevant footnote except 13.4-13.65 GHz in Region 1 | Incompatibility established by the use of Appendices 7, 8, technical Annexes of Appendices 30 or 30A, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU-R Recommendations, as appropriate | Methods specified in, or adapted from, Appendices 7, 8, 30, 30A, other technical provisions of the Radio Regulations or ITU-R Recommendations |  |
|  |  | 13.4-13.65 GHz in Region 1 indicated in No. 5.499A | i) Bandwidth overlap, and ii) any network in the space research service (SRS) with a space station within an orbital arc of ±20° of the nominal orbital position of a proposed network in the FSS |  |  |

ANNEX 1

# 1	Coordination thresholds for sharing between MSS (space-to-Earth) and terrestrial services in the same frequency bands and between non-GSO MSS feeder links (space-to-Earth) and terrestrial services
in the same frequency bands and between RDSS (space-to-Earth) and terrestrial services in the same frequency bands     (WRC-12)

## 1.1	Below 1 GHz*

1.1.1	In the bands 137-138 MHz and 400.15-401 MHz, coordination of a space station of the MSS (space-to-Earth) with respect to terrestrial services (except aeronautical mobile (OR) service networks operated by the administrations listed in Nos. 5.204 and 5.206 as of 1 November 1996) is required only if the pfd produced by this space station exceeds −125 dB(W/(m2 · 4 kHz)) at the Earth’s surface.

1.1.2	In the band 137-138 MHz, coordination of a space station of the MSS (space-to-Earth) with respect to the aeronautical mobile (OR) service is required only if the pfd produced by this space station at the Earth’s surface exceeds:

–	−125 dB(W/(m2 · 4 kHz)) for networks for which complete Appendix 3** coordination information has been received by the Bureau prior to 1 November 1996;

–	−140 dB(W/(m2 · 4 kHz)) for networks for which complete Appendix 4/S4/3** coordination information has been received by the Bureau after 1 November 1996 for the administrations referred to in § 1.1.1 above.

1.1.3	In the band 137-138 MHz, coordination is also required for a space station on a replacement satellite of a MSS network for which complete Appendix 3** coordination information has been received by the Bureau prior to 1 November 1996 and the pfd exceeds −125 dB(W/(m2 · 4 kHz)) at the Earth’s surface for the administrations referred to in § 1.1.1 above.

## 1.2	Between 1 and 3 GHz

### 1.2.1	Objectives

Generally, pfd thresholds were used to determine the need for coordination between space stations of the MSS (space-to-Earth) and terrestrial services and for coordination between space stations of the RDSS (space-to-Earth) and terrestrial services. However, to facilitate sharing between digital fixed service stations and non-GSO MSS space stations, the concept of fractional degradation in performance (FDP) was adopted. This concept involves new methods described in this Annex.

As a consequence of this new concept, the need for coordination between space stations of the MSS (space-to-Earth) and terrestrial services is determined using two methods:

–	simple method: FDP (simple definition of the MSS system and characteristics of reference FS stations are used in inputs) or power flux-density trigger value;

–	more detailed method: system specific methodology (SSM) (specific characteristics of the MSS system and characteristics of reference fixed service stations are used in inputs) as described, for example, in Annex 1 to the most recent version of Recommendation ITU-R M.1143.

If one of the two methods gives a result that does not exceed the criteria relevant to each method, there is no need for coordination.

If only one method is available in an administration, the result of this method must be taken into account.     (WRC-15)

### 1.2.2	General considerations

#### 1.2.2.1	Method for calculating the value of FDP

The FDP is used in cases of sharing between digital fixed service stations with non-GSO MSS stations (space-to-Earth).

To calculate the value of the FDP, the following parameters are needed:

–	technical characteristics of digital fixed service station;

–	technical characteristics of non-GSO MSS constellation.

The FDP is calculated:

–	by simulating the proposed MSS constellation using the information given in § A.4 of Annex 2 to Appendix 4;

–	by positioning the fixed service station at a certain latitude (each station is assumed to operate at an elevation angle of 0°);

–	by calculating for each pointing azimuth (Az) varying between 0° and 360°:

–	at each instant in time of the simulation, the aggregate interference from all visible space stations received at the fixed service station;

–	the FDPAz for the azimuth Az, using the following formula:

–	by the following formula:

FDP  =  max(FDPAz)

(The formula for FDP applies to the 1-3 GHz frequency range only. A different formula may apply at frequencies above 3 GHz.)

where:

Ii:	interference noise power level (W)

fi:	the fractional period of time during which the interference power equals Ii

NT:	station receiving system noise power level = k T B (W)

k:	Boltzmann’s constant = 1.38 × 10−23 (J/K)

T:	FS station receiving system effective noise temperature (T should be calculated by the following formula:

10 log T  =  NF  +  10 log T0

where NF (dB) is the receiver noise figure given in Annex 1 and T0 should be assumed as 290 K)

B:	reference bandwidth = 1 MHz.

NOTE – For the purpose of FDP calculation according to this Annex, it should be assumed that all space stations in the same MSS constellation operate on the same frequencies.

#### 1.2.2.2	Characteristics of reference systems in the fixed service

The following parameters represent the set of reference parameters of the fixed service.

##### 1.2.2.2.1	Characteristics of reference digital point-to-point systems

Three different digital systems are described in the following Table:

–	64 kbit/s capacity used, for example, for outside plant (individual subscriber connection);

–	2 Mbit/s capacity used, for example, for business subscriber connections for the local part of the inside plant;

–	45 Mbit/s capacity used, for example, for trunk networks.

| Capacity | 64 kbit/s | 2 Mbit/s | 45 Mbit/s |
| --- | --- | --- | --- |
| Modulation | 4-PSK | 8-PSK | 64-QAM |
| Antenna gain (dB) | 33 | 33 | 33 |
| Transmit power (dBW) | 7 | 7 | 1 |
| Feeder/multiplexer loss (dB) | 2 | 2 | 2 |
| e.i.r.p. (dBW) | 38 | 38 | 32 |
| Receiver IF bandwidth (MHz) | 0.032 | 0.7 | 10 |
| Receiver noise figure (dB) | 4 | 4.5 | 4 |
| Receiver input level for a BER of 10−3 (dBW) | −137 | −120 | −106 |

Antenna pattern:

G(φ)  =  Gmax  –  2.5  ×  10−3 	for	0	<  φ  <  φm

G(φ)  =  39  –  5 log (D/)  –  2.5 log φ	for	φm	  φ  <  48°

G(φ)  =  – 3  –  5 log (D/)	for	48°	  φ    180°

where:

G(φ):	gain relative to an isotropic antenna (dBi)

φ:	off-axis angle (degrees)

D:	antenna diameter

λ:	wavelength expressed in the same unit as D

G1:	gain of the first side-lobe = 2 + 15 log (D/λ)

(D/λ may be estimated from 20 log (D/λ) ≈ Gmax – 7.7)

Gmax:	main lobe antenna gain (dBi)

φm  = 	20  (λ/D)  ×

It should be noted that the above antenna radiation pattern corresponds to the average side-lobe pattern and it is recognized that individual side-lobes may exceed it by up to 3 dB.

##### 1.2.2.2.2	Characteristics of reference analogue point-to-point systems

| Reference circuit | 12 hops with 50 km distance between stations |
| --- | --- |
| Antenna gain (dBi) | 33 |
| e.i.r.p. (dBW) | 36 |
| Feeder/multiplexer loss (dB) | 3 |
| Receiver noise figure (referred to input of receiver) (dB) | 8 |
| Maximum short- and long-term interference in the reference circuit: – baseband interfering signal power level not to be exceeded for more than 20% of the time – baseband interfering signal power level not to be exceeded for more than 0.01% of the time | 240 pW0p 50 000 pW0p |

Antenna pattern: use antenna pattern of § 1.2.2.2.1.

##### 1.2.2.2.3	Characteristics of reference point-to-multipoint systems

NOTE – In application of the standard computation program, the use of the point-to-multipoint reference fixed service system parameters for the 2 170-2 200 MHz band is not required.

| Parameter | Central station | Outstation |
| --- | --- | --- |
| Antenna type | Omni/sectoral | Dish/horn |
| Antenna gain (dBi) | 10/13 | 20 (analogue) 27 (digital) |
| e.i.r.p. (max) (dBW): – analogue – digital | 12 24 | 21 34 |
| Noise figure (dB) | 3.5 | 3.5 |
| Feeder/multiplexer loss (dB) | 2 | 2 |
| IF bandwidth (MHz) | 3.5 | 3.5 |

Antenna pattern:

For the outstation antenna pattern, the reference pattern described in § 1.2.2.2.1 is to be used.

The reference radiation pattern for omnidirectional or sectoral antennas is the following:

G(θ)  =  G0  –  12 (θ/φ3)2	for	0	≤  θ  <  φ3

G(θ)  =  G0  –  12  –  10 log (θ/φ3)		for	φ3	≤  θ  <  90°

where:

G0:	maximum gain in the horizontal plane (dBi)

θ:	radiation angle above the horizontal plane (degrees)

φ3 (degrees) is given by:

where:

### 1.2.3	Determination of the need for coordination between MSS and RDSS space stations (space-to-Earth) and terrestrial stations    (WRC-12)

#### 1.2.3.1	Method for the determination of the need for coordination between MSS and RDSS space stations (space-to-Earth) and other terrestrial services sharing the same frequency band in the 1 to 3 GHz range

Coordination of assignments for transmitting space stations of the MSS and RDSS with respect to terrestrial services is not required if the pfd produced at the Earth’s surface or the FDP of a station in the fixed service does not exceed the threshold values shown in the following table.    (WRC-12)

TABLE 5-2     (Rev.WRC-12)

| Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| 1 518-1 525 | Analogue FS telephony (NOTE 5) | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 |  |
|  | All other cases FS telephony (NOTES 4 and 8) | −128 dB(W/m2) in 1 MHz | 0.5 | −128 dB(W/m2) in 1 MHz | 0.5 | 25 |

TABLE 5-2 (end)     (Rev.WRC-12)

| Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| 1 525-1 530 | Analogue FS telephony (NOTE 5) | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 |  |
|  | All other cases | −128 dB(W/m2) in 1 MHz | 0.5 | −128 dB(W/m2) in 1 MHz | 0.5 | 25 |
| 2 160-2 200 | Analogue FS telephony (NOTE 5) | −146 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz | 0.5 | −141 dB(W/m2) in 4 kHz and −123 dB (W/m2) in 1 MHz (NOTE 6) | 0.5 |  |
| (NOTE 3) | All other cases | −128 dB(W/m2) in 1 MHz | 0.5 | −123 dB(W/m2) in 1 MHz (NOTE 6) | 0.5 | 25 |
| 2 483.5-2 500 (mobile-satellite service) | All cases | –146 dB(W/m2) in 4 kHz and –128 dB(W/m2) in 1 MHz | 0.5 | −144 dB(W/m2) in 4 kHz and −126 dB(W/m2) in 1 MHz (NOTE 9) | 0.65 |  |
| 2 483.5-2 500 (radiodeterm-ination-satellite service) (NOTE 10) | All cases except the radiolocation service in the countries listed in No. 5.398A | −152 dB(W/m2) in 4 kHz −128 dB(W/m2) in 1 MHz | – | −153 dB(W/m2) in 4 kHz −129 dB(W/m2) in 1 MHz (NOTE 9) |  |  |
| 2 500-2 520 (SUP - WRC-07) |  |  |  |  |  |  |
| 2 520-2 535 (SUP - WRC-07) |  |  |  |  |  |  |
| NOTE 1 – The calculation of FDP is contained in § 1.2.2.1, using the reference FS parameters contained in § 1.2.2.2.1 and 1.2.2.2.3. The use of FDP threshold is limited to the case of digital FS systems. |  |  |  |  |  |  |  |
| NOTE 2 – The following formula should be used for deriving the coordination threshold in terms of pfd: P for 0° ≤  ≤ 5° P + r ( – 5) for 5° <  ≤ 25° P + 20 r for 25° <  ≤ 90° where  is the angle of arrival (degrees). The threshold values are obtained under assumed free-space propagation conditions. |  |  |  |  |  |  |  |

NOTE 3 – The coordination thresholds in the band 2 160-2 170 MHz (Region 2) and 2 170-2 200 MHz (all Regions) to protect other terrestrial services do not apply to International Mobile Telecommunications (IMT) systems, as the satellite and the terrestrial components are not intended to operate in the same area or on common frequencies within these bands. (WRC-12). NOTE 4 – Exceptions for the band 1 518-1 525 MHz are as follows: 4.1 For the land mobile service on the territory of Japan (No. 5.348A): −150 dB(W/m2) in 4 kHz at all angles of arrival is applicable to all satellite space-to-Earth emissions. 4.2 For the aeronautical mobile service for telemetry on the territory of the administrations listed in No. 5.342: −140 dB(W/m2) in 4 kHz at all angles of arrival. 4.3 For the point-to-multipoint systems operating in the fixed service in the territory of New Zealand: −138 dB(W/m2) in 1 MHz for angles of arrival less than or equal to 5° above the horizon and increasing linearly to −125 dB(W/m2) in 1 MHz for angles of arrival equal to 25° or greater above the horizon. (WRC-03). NOTE 5 – In all cases involving sharing with analogue systems for telephony in the FS, further coordination is only required when the pfd values are greater than or equal to the coordination threshold values in both reference bandwidths.. NOTE 6 – The pfd values specified for the band 2 160-2 200 MHz provide full protection for analogue radio-relay systems using the sharing criteria established by the most recent version of Recommendation ITU-R SF.357, for operation with a non-GSO MSS system employing narrow-band time division multiple access/frequency division multiple access techniques.. NOTE 7 – (SUP - WRC-12) NOTE 8 – In the band 1 518-1 520 MHz, for the point-to-multipoint systems operating in the fixed service on the territory of Australia: –138 dB(W/m2) in 1 MHz for angles of arrival less than or equal to 5° above the horizon and increasing linearly to –125 dB(W/m2) in 1 MHz for angles of arrival equal to 25° or greater above the horizon. (WRC-03). NOTE 9 – Instead of the values in the Table, the pfd coordination thresholds of −142.5 dB(W/m2) in 4 kHz and −124.5 dB(W/m2) in 1 MHz for the MSS and −152 dB(W/m2) in 4 kHz and −128 dB(W/m2) in 1 MHz for the RDSS shall apply in Albania, Germany, Andorra, Antigua and Barbuda, Argentina, Australia, Austria, Bahamas, Barbados, Belgium, Belize, Bolivia (Plurinational State of), Bosnia and Herzegovina, Brazil, Bulgaria, Canada, Chile, Cyprus, Vatican, Colombia, Congo (Rep. of the), Costa Rica, Croatia, Denmark, Dominican Rep., Dominica, El Salvador, Ecuador, Spain, Estonia, United States, Finland, France, Greece, Grenada, Guatemala, Guyana, Haiti, Honduras, Hungary, Ireland, Iceland, Israel, Italy, Jamaica, Latvia, The Former Yugoslav Rep. of Macedonia, Liechtenstein, Lithuania, Luxembourg, Malta, Mexico, Monaco, Montenegro, Nicaragua, Nigeria, Norway, Panama, Paraguay, Netherlands, Peru, Poland, Portugal, Slovakia, Czech Rep., Romania, United Kingdom, Saint Lucia, Saint Kitts and Nevis, San Marino, Saint Vincent and the Grenadines, Serbia, Slovenia, Sweden, Switzerland, Suriname, Trinidad and Tobago, Turkey, Uruguay and Venezuela. (WRC-12) NOTE 10 – These pfd values apply only to systems submitted after 17 February 2012 and do not apply to systems for which complete coordination information has been received before 18 February 2012 (see No. 5.401). (WRC-12)

#### 1.2.3.2	A system specific methodology (SSM) to be used in determining the need for detailed coordination of non-GSO MSS (space-to-Earth) systems with fixed service systems

The purpose of the SSM is to allow a detailed assessment of the need to coordinate frequency assignments to non-GSO MSS space stations (space-to-Earth) with frequency assignments to receiving stations in a fixed service network of a potentially affected administration. The SSM takes into account specific characteristics of the non-GSO MSS system and reference fixed service characteristics.

Those administrations planning to establish the need for coordination between non-GSO MSS networks and fixed service systems are encouraged to use the most recent version of Recommendation ITU-R M.1143. While urgent additional development work is being undertaken in the ITU-R to facilitate the use of the methodology described in the most recent version of Recommendation ITU-R M.1143, administrations may be able to effect coordination by applying this SSM.    (WRC-15)

## 1.3	(SUP - WRC-15)

# 2	(SUP - WRC-2000)

# 3	(SUP - WRC-2000)

APPENDIX 7 (REV.WRC-15)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

# 1	Introduction

This Appendix addresses the determination of the coordination area (see No. 1.171) around a transmitting or receiving earth station that is sharing spectrum in frequency bands between 100 MHz and 105 GHz with terrestrial radiocommunication services or with earth stations operating in the opposite direction of transmission.

The coordination area represents the area surrounding an earth station sharing the same frequency band with terrestrial stations, or the area surrounding a transmitting earth station that is sharing the same bidirectionally allocated frequency band with receiving earth stations, within which the permissible level of interference may be exceeded and hence coordination is required. The coordination area is determined on the basis of known characteristics for the coordinating earth station and on conservative assumptions for the propagation path and for the system parameters for the unknown terrestrial stations (see Tables 7 and 8), or the unknown receiving earth stations (see Table 9), that are sharing the same frequency band.

## 1.1	Overview

This Appendix contains procedures and system parameters for calculating an earth station’s coordination area, including predetermined distances.

The procedures allow the determination of a distance in all azimuthal directions around a transmitting or receiving earth station beyond which the predicted path loss would be expected to exceed a specified value for all but a specified percentage of the time. This distance is called the coordination distance (see No. 1.173). When the coordination distance is determined for each azimuth around the coordinating earth station it defines a distance contour, called the coordination contour (see No. 1.172), that encloses the coordination area.

It is important to note that, although the determination of the coordination area is based on technical criteria, it represents a regulatory concept. Its purpose is to identify the area within which detailed evaluations of the interference potential need to be performed in order to determine whether the coordinating earth station or any of the terrestrial stations, or in the case of a bidirectional allocation any of the receiving earth stations that are sharing the same frequency band, will experience unacceptable levels of interference. Hence, the coordination area is not an exclusion zone within which the sharing of frequencies between the earth station and terrestrial stations or other earth stations is prohibited, but a means for determining the area within which more detailed calculations need to be performed. In most cases a more detailed analysis will show that sharing within the coordination area is possible since the procedure for the determination of the coordination area is based on unfavourable assumptions with regard to the interference potential.

For the determination of the coordination area, two separate cases are to be considered:

–	case when the earth station is transmitting and hence capable of interfering with receiving terrestrial stations or earth stations;

–	case when the earth station is receiving and hence may be the subject of interference from transmitting terrestrial stations.

Calculations are performed separately for great circle propagation mechanisms (propagation mode (1)) and, if required by the sharing scenario (see § 1.4), for scattering from hydrometeors (propagation mode (2)). The coordination contour is then determined using the greater of the two distances predicted by the propagation mode (1) and propagation mode (2) calculations for each azimuth around the coordinating earth station. Separate coordination contours are produced for each sharing scenario. Guidance and examples of the construction of coordination contours, and their component propagation mode (1) and propagation mode (2) contours, are provided in § 1.6.

To facilitate bilateral discussion it can be useful to calculate additional contours, defining smaller areas, that are based on less conservative assumptions than those used for the calculation of the coordination contour.

## 1.2	Structure of this Appendix

In this Appendix the general principles are separated from the detailed text on methods. The general principles are contained in the main body of the Appendix, while the methods are contained in a series of Annexes, enabling the user to select only those sections that are relevant for a specific sharing scenario.

Table 1 is provided to help the user to navigate through the Appendix and the Annexes; it also indicates the relevant sections that need to be explored for a specific coordination case.

TABLE 1

Cross-reference between sharing scenarios and calculation methods

|  | Sharing scenarios of § 1.4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Applicable sections and Annexes | § 1.4.1 Earth stations operating with geostationary space stations | § 1.4.2 Earth stations operating with non-geostationary space stations1 | § 1.4.3 Earth stations operating with both geostationary and non-geostationary space stations | § 1.4.4 Earth stations operating in bidirectionally allocated frequency bands | § 1.4.5 Broadcasting-satellite service earth stations | § 1.4.6 Mobile (except aeronautical mobile) earth stations | § 1.4.7 Aeronautical mobile earth stations |
| § 1.3 Basic concepts | X | X | X | X | X | X | X |
| § 1.5 Propagation model concepts | X | X | X | X | See § 1.4.1, 1.4.2, 1.4.3 or 1.4.4 as applicable and § 1.6 | See § 1.4.1, 1.4.2, 1.4.3 or 1.4.4 as applicable and § 1.6 | See § 1.4.1, 1.4.2, 1.4.3 or 1.4.4 as applicable and § 1.6 |
| § 1.6 The coordination contour: concepts and construction | X | X | X | X |  |  |  |
| § 2.1 Earth stations operating with geostationary space stations | X |  | X |  |  |  |  |
| § 2.2 Earth stations operating with non-geostationary space stations |  | X | X |  |  |  |  |
| § 3 Determination of the coordination area between earth stations operating in bidirectionally allocated frequency bands |  |  |  | X |  |  |  |
| § 4 General considerations for the determination of the propagation mode (1) required distance | X | X | X | X |  |  |  |
| § 5 General considerations for the determination of the propagation mode (2) required distance | X |  | X |  |  |  |  |
| Annex 1 Determination of the required distance for propagation mode (1) | X | X | X | X |  |  |  |
| Annex 2 Determination of the required distance for propagation mode (2) | X |  | X |  |  |  |  |
| Annex 3 Antenna gain towards the horizon for an earth station operating with a geostationary space station | X |  | X |  |  |  |  |
| Annex 4 Antenna gain towards the horizon for earth stations operating with non-geostationary space stations |  | X | X | X |  |  |  |
| Annex 5 Determination of the coordination area for a transmitting earth station with respect to receiving earth stations operating with geostationary space stations in bidirectionally allocated frequency bands |  |  |  | X |  |  |  |
| Annex 6 Supplementary and auxiliary contours | X | X | X | X |  |  |  |
| Annex 7 System parameters and predetermined coordination distances for determination of the coordination area around an earth station | X | X | X | X |  |  |  |
| 1 For an earth station using a non-tracking antenna the procedure of § 2.1 is used. For an earth station using a non-directional antenna the procedures of § 2.1.1 are used. |  |  |  |  |  |  |  |

## 1.3	Basic concepts

Determination of the coordination area is based on the concept of the permissible interference power at the antenna terminals of a receiving terrestrial station or earth station. Hence, the attenuation required to limit the level of interference between a transmitting terrestrial station or earth station and a receiving terrestrial station or earth station to the permissible interference power for p% of the time is represented by the “minimum required loss”, which is the loss that needs to be equalled or exceeded by the predicted path loss for all but p% of the time1.

For propagation mode (1) the following equation applies:

Lb(p) = Pt + Gt + Gr – Pr(p)                    dB	(1)

where:

p:	maximum percentage of time for which the permissible interference power may be exceeded

Lb(p ):	propagation mode (1) minimum required loss (dB) for p% of the time; this value must be exceeded by the propagation mode (1) predicted path loss for all but p% of the time

Pt:	maximum available transmitting power level (dBW) in the reference bandwidth at the terminals of the antenna of a transmitting terrestrial station or earth station

Pr(p ):	permissible interference power of an interfering emission (dBW) in the reference bandwidth to be exceeded for no more than p% of the time at the terminals of the antenna of a receiving terrestrial station or earth station that may be subject to interference, where the interfering emission originates from a single source

Gt:	gain (dB relative to isotropic) of the antenna of the transmitting terrestrial station or earth station. For a transmitting earth station, this is the antenna gain towards the physical horizon on a given azimuth; for a transmitting terrestrial station, the maximum main beam axis antenna gain is to be used

Gr:	gain (dB relative to isotropic) of the antenna of the receiving terrestrial or earth station that may be subject to interference. For a receiving earth station, this is the gain towards the physical horizon on a given azimuth; for a receiving terrestrial station, the maximum main beam axis antenna gain is to be used.

In the case of a receiving earth station, the permissible interference power Pr(p) is specified with respect to the actual percentage of time the receiver is in operation, rather than the total elapsed time.

For propagation mode (2), a volume scattering process is involved and a modification of the above approach is necessary. Where the coordinating earth station antenna beam intersects a rain cell, a common volume may be formed with a terrestrial station beam or an earth station beam (operating in the opposite direction of transmission in bidirectionally allocated frequency bands). In the case of a terrestrial station, the assumptions are made that the terrestrial station beamwidth is relatively large in comparison with that of the coordinating earth station (terrestrial station gain values are given in Tables 7 and 8) and that the terrestrial station is some distance from the common volume. The terrestrial station beam is therefore assumed to illuminate the whole rain cell, which is represented by a vertical cylinder filled with hydrometeors that give rise to isotropically scattered signals. This scattering process may give rise to unwanted coupling between the coordinating earth station and terrestrial stations or other earth stations operating in bidirectionally allocated frequency bands, via the common volume.

The earth station antenna gain and its beamwidth are interdependent. The size of the common volume, and the number of scattered signals arising within that volume, increases as the gain of the earth station antenna transmitting or receiving those signals decreases, the one effect compensating for the other. A term which approximates the full integral required to evaluate the volume scattering process within the earth station antenna beam is included in equation (72). Therefore in the procedure for evaluation of interference that may arise from propagation mode (2) mechanisms a simplifying assumption can be made that the path loss is independent of the earth station antenna gain2.

Hence for propagation mode (2), equation (1) reduces to:

Lx(p ) = Pt + Gx – Pr(p )                    dB	(2)

where:

Lx(p ):	minimum loss required for propagation mode (2)

Gx:	maximum antenna gain (dBi) assumed for the terrestrial station. Tables 7 and 8 give values of Gx for the various frequency bands.

To facilitate the calculation of propagation mode (2) auxiliary contours (see Annex 6) the calculation is further modified by placing the terrestrial network antenna gain Gx within the iterative loop for the propagation mode (2) required loss calculations3.

Hence equation (2) further reduces to:

L(p ) = Pt – Pr(p )                    dB	(3)

where:

L(p ):	propagation mode (2) minimum required loss (dB) for p% of the time; this value must be exceeded by the propagation mode (2) predicted path loss for all but p% of the time.

For both modes of propagation, Pt and Pr(p) are defined for the same radio-frequency bandwidth (the reference bandwidth). Further, Lb(p), L(p) and Pr(p) are defined for the same small percentage of the time, and these values are set by the performance criteria of the receiving terrestrial station or receiving earth station that may be subject to interference.

For an earth station operating with a geostationary space station, Annex 3 provides the numerical method for determining the minimum angle between the earth station antenna main beam axis and the physical horizon as a function of azimuth, and the corresponding antenna gain. In the case of a space station in a slightly inclined geostationary orbit, the minimum elevation angle and corresponding horizon gain will depend on the maximum inclination angle to be coordinated.

For an earth station operating with non-geostationary space stations, the antenna gain of the earth station in the direction of the horizon varies as a function of time and Annex 4 provides the numerical methods for its determination.

For an earth station operating in a frequency band with a bidirectional allocation, the antenna gain to be used in determining the propagation mode (1) minimum required loss is calculated using the methods in Annex 3 or Annex 4, as appropriate.

Determination of the coordination area requires the calculation of the predicted path loss and its comparison with the minimum required loss, for every azimuth around the coordinating earth station, where:

–	the predicted path loss is dependent on several factors including the length and general geometry of the interfering path (e.g. antenna pointing, horizon elevation angle), antenna directivity, radio climatic conditions, and the percentage of the time during which the predicted path loss is less than the minimum required loss; and

–	the minimum required loss is based on system and interference model considerations.

The required coordination distance is the distance at which these two losses are considered to be equal for the stated percentage of time.

In determining the coordination area, the pertinent parameters of the coordinating earth station are known, but knowledge of the terrestrial stations or other earth stations sharing that frequency range is limited. Hence it is necessary to rely on assumed system parameters for the unknown terrestrial stations or the unknown receiving earth stations. Furthermore, many aspects of the interference path between the coordinating earth station and the terrestrial stations or other earth stations (e.g. antenna geometry and directivity) are unknown.

The determination of the coordination area is based on unfavourable assumptions regarding system parameter values and interference path geometry. However, in certain circumstances, to assume that all the worst-case values will occur simultaneously is unrealistic, and leads to unnecessarily large values of minimum required loss. This could lead to unnecessarily large coordination areas. For propagation mode (1), detailed analyses, supported by extensive operational experience, have shown that the requirement for the propagation mode (1) minimum required loss can be reduced because of the very small probability that the worst-case assumptions for system parameter values and interference path geometry will exist simultaneously. Therefore, a correction is applied within the calculation for the propagation mode (1) predicted path loss in the appropriate sharing scenario to allow benefit to be derived from these mitigating effects. The application of this correction factor is described in more detail in § 4.4.

This correction applies to cases of coordination with the fixed service. It is frequency, distance and path dependent. It does not apply in the case of the coordination of an earth station with mobile stations, nor with other earth stations operating in the opposite direction of transmission, nor in the case of propagation via hydrometeor scatter (propagation mode (2)).

A number of propagation models are used to cover the propagation mechanisms that exist in the full frequency range. These models predict the path loss as a monotonically increasing function of distance. Therefore, coordination distances are determined by calculating the path loss iteratively for an increasing distance until either the minimum required loss is achieved, or a maximum calculation distance limit is reached (see § 1.5.3).

The iteration method always starts at a defined value of minimum distance, dmin (km), and iteration is performed using a uniform step size, s (km), for increasing the distance. A step size of 1 km is recommended.

## 1.4	Sharing scenarios

The following subsections describe the basic assumptions made for the various earth station sharing scenarios. These subsections need to be read in conjunction with the information contained in Table 1 and § 1.6 which contains guidance on the development of a coordination contour. Except as discussed in § 1.4.5 to 1.4.7, the earth stations around which coordination areas are determined are assumed to be fixed earth stations authorized to operate at a single permanent location. In cases of earth stations that can be operated from a number of fixed locations, the coordination areas are determined for each individual location.4

### 1.4.1	Earth stations operating with geostationary space stations

For an earth station operating with a space station in the geostationary orbit, the space station appears to be stationary with respect to the Earth. However variations in gravitational forces acting on the space station and limitations in positional control mean that a geostationary space station’s orbital parameters are not constant. Movement from the space station’s nominal orbital position in an east/west direction (longitudinal tolerance) is limited under the Radio Regulations (see Nos. 22.6 to 22.18), but movement in the north/south direction (inclination excursion) is not specified.

Relaxation in the north/south station-keeping of a geostationary space station allows its orbit to become inclined, with an inclination that increases gradually with time. Therefore the determination of the coordination area requires consideration of the range of movement of the earth station antenna. Although the direction of pointing of the earth station antenna may in practice vary with time, the earth station antenna may also be pointing in one direction for considerable periods of time. Hence the gain of the earth station antenna in the direction of the horizon is assumed to be constant. For an earth station operating with a space station in an orbit as described above, an assumption of constant horizon gain as the inclination angle increases may lead to a conservative estimation of the coordination area, the degree of conservatism increasing with increasing inclination angle.

For an earth station operating with a geostationary space station the coordination area is determined using the procedures described in § 2.1.

### 1.4.2	Earth stations operating with non-geostationary space stations

Earth stations operating with non-geostationary space stations may use a directional or a non-directional antenna. Furthermore, earth stations using a directional antenna may track the orbital path of a non-geostationary space station.

While an earth station operating with a geostationary space station is assumed to have a constant antenna gain towards the horizon, for an earth station antenna that is tracking the orbital path of a non-geostationary space station, the antenna gain towards the horizon will vary with time. Therefore, it is necessary to estimate the variation of the antenna gain with time towards the horizon for each azimuth in order to determine the coordination area. The procedure is described in § 2.2.

For an earth station operating with a non-geostationary space station, the motion of a relatively high gain tracking antenna reduces the probability of interference due to propagation mode (2) mechanisms and hence the propagation mode (2) required distances will be relatively short. The minimum coordination distance dmin (see § 1.5.3) will provide adequate protection in these cases. The propagation mode (2) contour is therefore taken to be identical to a circle whose radius is the minimum coordination distance. Propagation mode (2) calculations are not required in these circumstances and the coordination area is determined using the propagation mode (1) procedure in § 2.2 only.

For an earth station operating with a non-geostationary space station using a non-directional antenna, a similar situation applies, and the low gain means that propagation mode (2) required distances will be less than the minimum coordination distance. Hence, for the case of a non-directional antenna the propagation mode (2) contour is also coincident with the circle of radius dmin, and the coordination area is determined using the propagation mode (1) procedures described in § 2.1.1 only.

For an earth station operating with a non-geostationary space station using a non-tracking directional antenna, the potential for interference arising from propagation mode (2) is the same as for an earth station operating with a geostationary space station. Hence, for the case of non-tracking directional antenna the coordination area is determined using both the propagation mode (1) and propagation mode (2) procedures described in § 2.1.

### 1.4.3	Earth stations operating with both geostationary and non-geostationary space stations

For earth stations that are sometimes intended to operate with geostationary space stations and at other times with non-geostationary space stations, separate coordination areas are determined for each type of operation. In such cases, the coordination area for the geostationary space station is determined using the procedures described in § 2.1 and the coordination area for the non-geostationary space station is determined using the procedure described in § 2.2. For each case, the percentage of time, p, is specified for all the operational time that the receiving earth station is expected to spend in reception from geostationary space stations or non-geostationary space stations, as appropriate.

### 1.4.4	Earth stations operating in bidirectionally allocated frequency bands

For earth stations operating in some frequency bands there may be allocations with equal rights to space services operating in both the Earth-to-space and space-to-Earth directions. In this case, where two earth stations are operating in opposite directions of transmission it is only necessary to establish the coordination area for the transmitting earth station, as receiving earth stations will automatically be taken into consideration. Hence, a receiving earth station operating in a bidirectionally allocated frequency band will only be involved in coordination with a transmitting earth station if it is located within the transmitting earth station’s coordination area.

For a transmitting earth station operating with either geostationary or non-geostationary satellites in a bidirectionally allocated frequency band, the coordination area is determined using the procedures described in § 3.     (WRC-03)

### 1.4.5	Broadcasting-satellite service earth stations

For earth stations in the broadcasting-satellite service operating in the unplanned bands, the coordination area is determined by extending the periphery of the specified service area within which the earth stations are operating by the coordination distance based on a typical BSS earth station. In calculating the coordination distance, no additional protection can be assumed to be available from the earth station horizon elevation angle, i.e. Ah = 0 dB in Annex 1, for all azimuth angles around the earth station.

### 1.4.6	Mobile (except aeronautical mobile) earth stations

For a mobile (except aeronautical mobile) earth station, the coordination area is determined by extending the periphery of the specified service area, within which the mobile (except aeronautical mobile) earth stations are operating, by the coordination distance. The coordination distance may be represented by a predetermined coordination distance (see Table 10), or it may be calculated. In calculating the coordination distance, no additional protection can be assumed to be available from the earth station horizon elevation angle, i.e. Ah = 0 dB in Annex 1, for all azimuths around the earth station.

### 1.4.7	Aeronautical mobile earth stations

For aeronautical mobile earth stations, the coordination area is determined by extending the periphery of the specified service area within which the aeronautical mobile earth station operates, by an appropriate predetermined coordination (see Table 10) distance for the respective services.

## 1.5	Propagation model concepts

For each mode of propagation, according to the requirements of the specific sharing scenario (see § 1.4) it is necessary to determine the predicted path loss. The determination of this predicted path loss is based on a number of propagation mechanisms.

Interference may arise through a range of propagation mechanisms whose individual dominance depends on climate, radio frequency, time percentage in question, distance and path topography. At any given point in time, one or more mechanisms may be present. The propagation mechanisms that are considered within this Appendix in the determination of the interference potential are as follows:

–	Diffraction: Insofar as it relates to diffraction losses occurring over the earth station’s local physical horizon. This effect is referred to below as “site shielding”. The remainder of the path along each radial is considered to be flat and therefore free of additional diffraction losses.

–	Tropospheric scatter: This mechanism defines the “background” interference level for paths longer than about 100 km, beyond which the diffraction field becomes very weak.

–	Surface ducting: This is the most important short-term interference mechanism over water and in flat coastal land areas, and can give rise to high signal levels over greater distances, sometimes exceeding 500 km. Such signals can exceed the equivalent “free-space” level under certain conditions.

–	Elevated layer reflection and refraction: The treatment of reflection and/or refraction from layers at heights of up to a few hundred metres is an important mechanism that enables signals to by-pass any diffraction losses due to the underlying terrain under favourable path geometry situations. Here again, the impact can be significant over long distances.

–	Hydrometeor scatter: Hydrometeor scatter can be a potential source of interference between terrestrial station transmitters and earth stations because it may act isotropically, and can therefore have an impact irrespective of whether the common volume is on or off the great-circle interference path between the coordinating earth station and terrestrial stations, or other receiving earth stations operating in bidirectionally allocated frequency bands.

In this Appendix, propagation phenomena are classified into two modes as follows:

–	Propagation mode (1): propagation phenomena in clear air (tropospheric scatter, ducting, layer reflection/refraction, gaseous absorption and site shielding). These phenomena are confined to propagation along the great-circle path.

–	Propagation mode (2): hydrometeor scatter.

### 1.5.1	Propagation mode (1)

For the determination of the propagation mode (1) required distances, the applicable frequency range has been divided into three parts:

–	For VHF/UHF frequencies between 100 MHz and 790 MHz and for time percentages from 1% to 50% of an average year.

–	From 790 MHz to 60 GHz and for time percentages from 0.001% to 50% of an average year.

–	From 60 GHz to 105 GHz and for time percentages from 0.001% to 50% of an average year.

The variation in predicted path loss due to the horizon elevation angle around an earth station is calculated by the method described in § 1 of Annex 1, using the horizon elevation angles and distances along different radials from the earth station. For all frequencies between 100 MHz and 105 GHz, the attenuation arising from the horizon characteristics is included in the value of propagation mode (1) predicted path loss, unless its use is specifically prohibited for a particular sharing scenario (see § 1.4.5 and § 1.4.6).

In the determination of the propagation mode (1) required distance, the world is divided into four basic radio-climatic zones. These zones are defined as follows:

–	Zone A1: coastal land, i.e. land adjacent to a Zone B or a Zone C area (see below), up to an altitude of 100 m relative to mean sea or water level, but limited to a maximum distance of 50 km from the nearest Zone B or Zone C area; in the absence of precise information on the 100 m contour, an approximation (e.g. 300 feet) may be used. Large inland areas of at least 7 800 km2 which contain many small lakes, or a river network, comprising more than 50% water, and where more than 90% of the land is less than 100 m above the mean water level may be included in Zone A15.

–	Zone A2: all land, other than coastal land as defined in Zone A1 above.

–	Zone B: “cold” seas, oceans and large bodies of inland water situated at latitudes above 30°, with the exception of the Mediterranean Sea and the Black Sea. A “large” body of inland water is defined, for the administrative purpose of coordination, as one having an area of at least 7 800 km2, but excluding the area of rivers. Islands within such bodies of water are to be included as water within the calculation of this area if they have elevations lower than 100 m above the mean water level for more than 90% of their area. Islands that do not meet these criteria should be classified as land for the purposes of calculating the area of the water.

–	Zone C: “warm” seas, oceans and large bodies of inland water situated at latitudes below 30°, as well as the Mediterranean Sea and the Black Sea.

### 1.5.2	Propagation mode (2)

For the determination of the propagation mode (2) required distance, interference arising from hydrometeor scatter can be ignored at frequencies below 1 000 MHz and above 40.5 GHz outside the minimum coordination distance (see § 1.5.3.1). Below 1 000 MHz, the level of the scattered signal is very low and above 40.5 GHz, although significant scattering occurs, the scattered signal is then highly attenuated along the path from the scatter volume to the receiving terrestrial station or earth station. Site shielding is not relevant to propagation mode (2) mechanisms as the interference path is via the main beam of the coordinating earth station antenna.

### 1.5.3	Distance limits

The effect of interference on terrestrial and space systems often needs to be assessed by considering long- and short-term interference criteria. These criteria are generally represented by a permissible interference power not to be exceeded for more than a specified percentage of time.

The long-term interference criterion (typically associated with percentages of time ≥ 20%) allows the error performance objective (for digital systems) or noise performance objective (for analogue systems) to be met. This criterion will generally represent a low level of interference and hence require a high degree of isolation between the coordinating earth station and terrestrial stations, or other receiving earth stations operating in bidirectionally allocated bands.

The short-term criterion is a higher level of interference, typically associated with time percentages in the range 0.001% to 1% of time, which will either make the interfered-with system unavailable, or cause its specified short-term interference objectives (error rate or noise) to be exceeded.

This Appendix addresses only the protection provided by the short-term criterion. There is therefore an implicit assumption that if the short-term criterion is satisfied, then any associated long-term criteria will also be satisfied. This assumption may not remain valid at short distances because additional propagation effects (diffraction, building/terrain scattering etc.) requiring a more detailed analysis become significant. A minimum coordination distance is therefore needed to avoid this difficulty. This minimum coordination distance is always the lowest value of coordination distance used. At distances equal to or greater than the minimum coordination distance, it can be assumed that interference due to continuous (long-term) propagation effects will not exceed levels permitted by the long-term criteria.

In addition to the minimum coordination distance, it is also necessary to set an upper limit to the calculation distance. Hence the coordination distance, on any azimuth, must lie within the range between the minimum coordination distance and the maximum calculation distance.

#### 1.5.3.1	Minimum coordination distance

For the reasons stated in § 1.5.3, it is necessary to set a lower limit, dmin, for the coordination distance. The iterative calculation of the coordination distance starts at this minimum distance, and this distance varies according to radiometeorological factors and the frequency band (see § 4.2). This same minimum coordination distance applies both to propagation mode (1) and propagation mode (2) calculations.

#### 1.5.3.2	Maximum calculation distance

Maximum calculation distances are required for propagation modes (1) and (2). In the case of mode (1), this distance corresponds to the maximum coordination distance, dmax1, given in § 4.3 for each of the four radioclimatic Zones. The propagation mode (1) maximum calculation distance is therefore dependent on the mixture of radioclimatic Zones in the propagation path, as described in § 4.3.

The maximum calculation distance for propagation mode (2) is given in § 2 of Annex 2.

## 1.6	The coordination contour: concepts and construction

The coordination distance, determined for each azimuth around the coordinating earth station, defines the coordination contour that encloses the coordination area. The coordination distance lies within the range defined by the minimum coordination distance and the maximum calculation distance.

In this Appendix, the procedures determine the distance at which the minimum required loss is equal to the predicted path loss. In addition, some procedures6 require that, for any azimuth, the greater of the distances determined for propagation mode (1) and propagation mode (2) is the distance to be used in determining the coordination contour. In both these cases, the distance at which the minimum required loss is equal to the predicted path loss may or may not be within the range of valid values that define the limits for the coordination distance. Hence, the distance determined from the application of all the procedures is referred to as the required distance.

The coordination area is determined by one of the following methods:

–	calculating, in all directions of azimuth from the earth station, the coordination distances and then drawing to scale on an appropriate map the coordination contour; or

–	extending the service area in all directions by the calculated coordination distance(s); or

–	for some services and frequency bands, extending the service area in all directions by a predetermined coordination distance.

Where a coordination contour includes the potential interference effects arising from both propagation mode (1) and propagation mode (2), the required distance used for any azimuth is the greater of the propagation mode (1) and propagation mode (2) required distances.

The sharing scenarios and the various procedures contained in this Appendix are based on different assumptions. Hence, the coordination area developed for one sharing scenario is likely to be based on different sharing considerations, interference paths and operational constraints than the coordination area developed under a different sharing scenario. Separate coordination areas are therefore required for each sharing scenario described in § 1.4, and each coordination area is specific to the radiocommunication services covered by the sharing scenario under which it was developed. Further, the coordination area developed for one sharing scenario cannot be used to determine the extent of any impact on the radiocommunication services covered by a different sharing scenario. Thus, a coordinating earth station operating in a bidirectionally allocated frequency band that is also allocated to terrestrial services will have two separate coordination areas:

–	one coordination area for determining those administrations with terrestrial services that may be affected by the operation of the coordinating earth station; and

–	one coordination area for determining those administrations with receiving earth stations that may be affected by the operation of the coordinating (transmitting) earth station.

This means that the establishment of the coordination area for an earth station will generally require the determination of several individual coordination areas, each drawn on a separate map. For example, an earth station which transmits to a geostationary space station in the band 10.7-11.7 GHz will need to develop the following coordination areas with respect to:

–	analogue terrestrial services which receive in the same band; this will comprise the potential effects arising from both propagation mode (1) and propagation mode (2) interference paths;

–	an earth station operating with a geostationary space station which receives in the same band; this will comprise the potential effects arising from both propagation mode (1) and propagation mode (2) interference paths;

–	an earth station operating with a non-geostationary space station which receives in the same band; this will comprise the potential effects arising from propagation mode (1) interference paths.

In addition, separate coordination contours are produced if the earth station both transmits and receives in bands shared with terrestrial services. However, for earth stations in bidirectionally allocated frequency bands, the coordination contours with respect to other earth stations are only produced for a transmitting earth station (see § 1.4.4).

Examples of coordination contours for each of the sharing scenarios in § 1.4 is provided in Fig. 1. It will be noticed that for some of the sharing scenarios there is a commonality to the construction of the coordination contour (shown by a solid line) that encompasses each coordination area. For those sharing scenarios where both propagation mode (1) and propagation mode (2) interference paths need to be taken into consideration, the parts of the propagation mode (1) contour and that part of the propagation mode (2) contour located within the overall coordination contour may be drawn using dashed lines.

In addition to the coordination contour, supplementary contours and auxiliary contours (see Annex 6) may be drawn to facilitate more detailed sharing discussions. Supplementary contours are based on the coordinating earth station sharing frequency bands with other radiocommunication services, or other types of radio systems in the same service, that have less onerous sharing criteria than the radio system used for developing the coordination area. These supplementary contours may be developed by the same method used to determine the coordination contour, or by other methods as agreed on a bilateral basis between administrations. For example, the Time Variant Gain method described in § 4 of Annex 6 can be used to generate supplementary contours for earth stations operating with non-geostationary space stations. Auxiliary contours are based on less conservative assumptions, with regard to the interference path and operational constraints, for the unknown terrestrial stations, or earth stations. Auxiliary contours are developed separately for propagation mode (1) and propagation mode (2) interference paths. In this context, the contours from which the coordination contour was developed are called main contours, and the auxiliary contours for propagation mode (1) and propagation mode (2) are referenced to the appropriate main contour. The various assumptions used for developing auxiliary contours to the propagation mode (1) contour, or the propagation mode (2) contour, can also be applied to supplementary contours. Hence, auxiliary contours may be drawn for both a main or a supplementary contour.

Supplementary contours are always drawn on a separate map as they apply to other types of radio system within the same radiocommunication service, or to radio systems in different radiocommunication services. However, as auxiliary contours apply to the various assumptions used in developing the main, or supplementary, contour they are always drawn on the same map that contains the corresponding main, or supplementary, contour.

While the use of supplementary or auxiliary contours allows less conservative assumptions with regard to the interference path and operational constraints to be taken into consideration, earth stations may transmit or receive a variety of classes of emissions. Hence, the earth station parameters to be used in the determination of the coordination contour, and any supplementary or auxiliary contours, are those which lead to the greatest distances for each earth station antenna beam and each allocated frequency band which the coordinating earth station shares with other radiocommunication systems.

# 2	Determination of the earth station coordination area with respect to terrestrial stations

This section contains the procedures for determining the coordination area for the case of earth stations sharing frequency bands with terrestrial stations. These procedures cover the cases for earth stations operating with space stations in the geostationary orbit, or in non-geostationary orbits, and are described in the following subsections.

For earth stations operating with space stations in non-geostationary orbits, consideration has to be given to the potential time-varying nature of the earth station’s antenna gain towards the horizon.

## 2.1	Earth stations operating with geostationary space stations

For an earth station operating with a geostationary space station, the value of Gt and Gr towards the horizon is considered to be constant with time. The percentage of time associated with Lb in equation (1) is the same as the time percentage, p, associated with Pr(p). When determining the coordination area between a coordinating earth station operating with a geostationary space station and terrestrial systems, the coordination distance on any azimuth is the greater of the propagation mode (1) and propagation mode (2) required distances. The required distances for propagation mode (1) and propagation mode (2) are determined using the procedures described in § 2.1.1 and § 2.1.2 respectively, after taking into consideration the following discussion on station-keeping.

When the north/south station-keeping of a geostationary space station is relaxed, the orbit of the space station becomes inclined with an inclination that increases gradually with time. This movement of the space station from its nominal position may require small corresponding adjustments in the elevation angle of the earth station antenna beam. Hence, to avoid considering the time variation in antenna gain in the direction of the horizon, the coordination area of an earth station operating with a space station in a slightly inclined geostationary orbit is determined for the minimum angle of elevation and the associated azimuth at which the space station is visible to the earth station (see Annex 3).

### 2.1.1	Determination of the coordinating earth station’s propagation mode (1) contour

Determination of the propagation mode (1) contour is based on great circle propagation mechanisms and it is assumed, for the interference path, that all the terrestrial stations are pointing directly at the coordinating earth station’s location. The required distance, on each azimuth, for propagation mode (1) is that distance which will result in a value of propagation mode (1) predicted path loss that is equal to the propagation mode (1) minimum required loss, Lb(p) (dB), as defined in § 1.3.

Lb(p ) = Pt + Ge + Gx – Pr(p )                    dB	(4)

where:

Pt and Pr(p):	as defined in § 1.3

Ge:	gain of the coordinating earth station antenna (dBi) towards the horizon at the horizon elevation angle and azimuth under consideration

Gx:	maximum antenna gain (dBi) assumed for the terrestrial station. Tables 7 and 8 give values for Gx for the various frequency bands.

The propagation mode (1) required distance is determined using the procedures described in § 4, and the detailed methods in Annex 1. Specific guidance relevant to the application of the procedures is provided in § 4.4.

### 2.1.2	Determination of the coordinating earth station’s propagation mode (2) contour

The required distance for hydrometeor scatter is that distance that will result in a propagation mode (2) predicted path loss equal to the propagation mode (2) minimum required loss L(p), as defined in equation (3). This propagation mode (2) required distance is determined using the guidance in § 5, and the detailed methods in Annex 2.

For an earth station operating with a geostationary space station having a slightly inclined orbit, the rain-scatter coordination contours for each of the satellite’s two most extreme orbit positions are determined individually, using the relevant elevation angles and their associated azimuths to the satellite. The rain scatter area is the total area contained within the two resulting overlapping coordination contours.

## 2.2	Earth stations operating with non-geostationary space stations

For an earth station that operates with non-geostationary space stations and whose antennas track the space stations, the antenna gain in the direction of the horizon on any azimuth varies with time. The method used to determine the coordination contour is the time invariant gain (TIG) method.

This method uses fixed values of antenna gain based on the maximum assumed variation in horizon antenna gain on each azimuth under consideration. In considering the horizon gain of the antenna for either a transmitting or a receiving earth station, only the horizon antenna gain values during the operational time are to be considered. The horizon antenna gain may be determined using Annex 4. Reference or measured antenna radiation patterns may be used as described in Annex 3. The values of horizon antenna gain defined below are used for each azimuth when applying equation (4) to determine the propagation mode (1) required distances:

Ge = Gmax	for	(Gmax – Gmin) ≤ 20 dB

Ge = Gmin + 20 	for  20 dB  <	(Gmax – Gmin) < 30 dB	(5)

Ge = Gmax – 10	for	(Gmax – Gmin) ≥ 30 dB

where:

Ge:	gain of the coordinating earth station antenna (dBi) towards the horizon at the horizon elevation angle and azimuth under consideration in equation (4)

Gmax, Gmin:	maximum and minimum values of the horizon antenna gain (dBi), respectively, on the azimuth under consideration.

The maximum and minimum values of the horizon antenna gain, on the azimuth under consideration, are derived from the antenna pattern and the maximum and minimum angular separation of the antenna main beam axis from the direction of the physical horizon at the azimuth under consideration.

Where a single value of minimum elevation angle for the main beam axis of the earth station antenna is specified for all azimuths, the minimum and maximum values of the horizon gain can be determined, for each azimuth under consideration, from the antenna pattern and the horizon elevation angle at that azimuth. The plot of the horizon elevation angle against azimuth is called the horizon profile of the earth station.

Additional constraints may be included in the determination of the maximum and minimum values of the horizon antenna gain where an earth station is operating with a constellation of non-geostationary satellites at a latitude for which no satellite is visible at the earth station’s specified minimum elevation angle over a range of azimuths. Over this range of azimuth angles, the minimum elevation angle of the earth station antenna main beam axis is given by the minimum elevation angle at which any satellite of the constellation is visible at that azimuth. The azimuthal dependence of this minimum satellite visibility elevation angle may be determined from consideration of the orbital altitude and inclination of the satellites in the constellation, without recourse to simulation, using the procedure in § 1.1 of Annex 4. In this case, the horizon antenna gain to be used in the method depends on the profile of the composite minimum elevation angle. This minimum composite elevation angle at any azimuth is the greater of the minimum satellite visibility elevation angle, at the azimuth under consideration, and the specified minimum elevation angle for the earth station which is independent of the azimuth.

Thus, at each azimuth under consideration, the maximum horizon antenna gain will be determined from the minimum value of the angular separation between the earth station horizon profile at this azimuth and the profile of the minimum composite elevation angle. Similarly, the minimum horizon antenna gain will be determined from the maximum value of the angular separation from the earth station horizon profile at this azimuth to the profile of the minimum composite elevation angle. The procedure for calculating the minimum and maximum angular separations from the profile of the minimum composite elevation angle is given in § 1.2 of Annex 4.

The propagation mode (1) required distance is then determined using the procedures described in § 4, and the detailed methods in Annex 1. Specific guidance relevant to the application of the propagation calculations is provided in § 4.4.

# 3	Determination of the coordination area between earth stations operating in bidirectionally allocated frequency bands

This section describes the procedures to be used for determination of the coordination area for an earth station transmitting in a frequency band allocated to space services in both Earth-to-space and space-to-Earth directions.

There are various coordination scenarios, involving only non-time-varying antenna gains, or only time-varying antenna gains (both earth stations operate with non-geostationary space stations) or, one time-varying antenna gain and one non-time-varying antenna gain.

The following subsections describe the methods for the determination of coordination area which are specific to each of these bidirectional cases. The procedures applicable to the coordination scenario where both earth stations operate with geostationary space stations are given in § 3.1. The other bidirectional coordination scenarios are considered in § 3.2, where particular attention is given to the approaches for using the horizon antenna gain of the receiving earth station for each of the possible coordination scenarios in the appropriate procedure of § 2.

Table 9 provides the parameters that are to be used in the determination of the coordination area. Table 9 also indicates whether, in each band, the receiving earth stations operate with geostationary or non-geostationary space stations. In some bands, receiving earth stations may operate with both geostationary and non-geostationary space stations. Table 2 indicates the number of coordination contours which need to be drawn for each coordination scenario and the section(s) containing the applicable calculation methods. Once drawn, each coordination contour must be appropriately labelled.

TABLE 2

Coordination contours required for each bidirectional scenario

| Coordinating earth station operating to a space station in the | Unknown receiving earth station operating with a space station in the | Section containing the method to determine Gt and Gr | Contours required |  |
| --- | --- | --- | --- | --- |
|  |  |  | No. | Details |
|  | Geostationary orbit | § 3.1 | 1 | A coordination contour comprising both propagation mode (1) and propagation mode (2) contours |
| Geostationary orbit | Non-geostationary orbit | § 3.2.1 | 1 | A propagation mode (1) coordination contour |
|  | Geostationary or non-geostationary orbits1 | § 3.1.1 and 3.2.1 | 2 | Two separate coordination contours, one for the geostationary orbit (propagation mode (1) and mode (2) contours) and one for the non-geostationary orbit (propagation mode (1) contour) |
|  | Geostationary orbit | § 3.2.2 | 1 | A propagation mode (1) coordination contour |
| Non-geostationary orbit | Non-geostationary orbit | § 3.2.3 | 1 | A propagation mode (1) coordination contour |
|  | Geostationary or non-geostationary orbits1 | § 3.2.2 and 3.2.3 | 2 | Two separate propagation mode (1) coordination contours, one for the geostationary orbit and one for the non-geostationary orbit |
| 1 In this case, the bidirectional frequency band may contain allocations in the Earth-to-space direction for space stations in both the geostationary orbit and non-geostationary orbits. Hence, the coordinating administration will not know whether the unknown receiving earth stations are operating with space stations in the geostationary orbit or non-geostationary orbit. |  |  |  |  |

## 3.1	Coordinating and unknown earth stations operating with geostationary space stations

When both the coordinating and the unknown earth stations operate with space stations in the geostationary orbit, it is necessary to develop a coordination contour comprising both propagation mode (1) and propagation mode (2) contours, using the procedures described in § 3.1.1 and 3.1.2, respectively.

### 3.1.1	Determination of the coordinating earth station’s propagation mode (1) contour

The procedure for the determination of the propagation mode (1) contour in this case differs from that described in § 2.2 in two ways. First, the parameters to be used for the unknown receiving earth station are those in Table 9. Second, and more significantly, the knowledge that both earth stations operate with geostationary satellites can be used to calculate the worst-case value of the horizon antenna gain of the receiving earth station towards the transmitting earth station for each azimuth at the transmitting earth station. The propagation mode (1) required distance is that distance which will result in a value of propagation mode (1) predicted path loss which is equal to the propagation mode (1) minimum required loss, Lb(p ) (dB), as defined in § 1.3, and repeated here for convenience.

Lb(p ) = Pt + Gt + Gr – Pr(p )                    dB	(6)

where:

Pt and Pr(p ):	as defined in § 1.3

Gt:	gain of the coordinating (transmitting) earth station antenna (dBi) towards the horizon at the horizon elevation angle and the azimuth under consideration

Gr:	the horizon antenna gain of the unknown receiving earth station towards the transmitting earth station on the specific azimuth from the coordinating earth station. Values are determined by the procedure in § 2.1 of Annex 5, based on parameters from Table 9.

To facilitate the determination of the values of Gr to be used at an azimuth from the transmitting earth station, several simplifying approximations must be made:

that the horizon elevation of the receiving earth station is zero degrees on all azimuths;

that the receiving earth station operates with a space station that has zero degrees orbital inclination and may be located anywhere on the geostationary orbit that is above the minimum elevation angle, given in Table 9, for the location of the receiving earth station;

that the latitude of the receiving earth station is the same as that of the transmitting earth station;

that plane geometry can be used to interrelate the azimuth angles at the respective earth stations, rather than using the great circle path.

The first three assumptions provide the basis for determining the horizon antenna gain of the receiving earth station on any azimuth. The assumption of 0° horizon elevation angle is conservative since the increase in horizon antenna gain due to a raised horizon would, in practice, be more than offset by any real site shielding7. The last two assumptions in the list simplify the calculation of the sum of Gt and Gr along any azimuth. Since the propagation mode (1) required distances are small, in global

geometric terms these approximations may introduce a small error in the determination of the horizon antenna gain of the receiving earth station antenna that, in any case, will not exceed 2 dB. Because of the assumption of plane geometry, for a given azimuth at the transmitting earth station the appropriate value of the horizon antenna gain of the receiving earth station is the value on the reciprocal (i.e. 180°, see § 2.1 of Annex 5) azimuth at the receiving earth station.

The propagation mode (1) required distance is then determined using the procedures described in § 4, and the detailed methods in Annex 1. Specific guidance relevant to the application of the propagation calculations is provided in § 4.4.

### 3.1.2	Determination of the coordinating earth station’s propagation mode (2) contour

The procedure for the determination of the propagation mode (2) contour for a transmitting earth station operating with a geostationary space station uses the same simplifying approximations as made in § 3.1.1, but it is based on a geometrical construction that avoids the requirement for a complex propagation model (see § 3 of Annex 5). Auxiliary contours cannot be used in this method, as the calculations are not based on the propagation mode (2) required loss.

The propagation mode (2) contour is determined using the elevation angle and the azimuth from the coordinating transmitting earth station to the space station, together with the following two considerations:

–	the minimum coordination distance (see § 4.2), which will be the required distance for some azimuths; and

–	a worst-case required distance determined by the hydrometeor scatter geometry for a receiving earth station located in either of two 6° azimuth sectors. Within these sectors, the receiving earth station is assumed to be operating at the minimum elevation angle to a space station in the geostationary orbit and its main beam intersects the beam for the coordinating transmitting earth station at the point where the latter beam passes through the rain height, hR. Although the scattering can occur anywhere between the coordinating earth station and this point, the intersection of the two beams at this point represents the worst-case interference scenario. Hence, it results in the worst-case distance requirement for receiving earth stations located in the two azimuth sectors.

For an earth station operating with a space station in an inclined orbit, the lowest expected operational antenna elevation angle and its associated azimuth are used in the calculations.

The propagation mode (2) contour is determined using the method in § 3 of Annex 5.

## 3.2	Coordinating or unknown earth stations operating with non-geostationary space stations

To determine the coordination area, the method described in § 2.2 is used. For the cases where a coordinating (transmitting) earth station operates with non-geostationary space stations, the following procedures assume that the earth station antenna is tracking the space station, otherwise see § 1.4.2. Table 9 provides values of horizon antenna gain to be used in the calculations.

One or more of the following three procedures may be needed to determine the required propagation mode (1) coordination contours of Table 2. Propagation mode (2) contours are not required for any of the cases where either of the earth stations operates with space stations in non-geostationary orbits.

### 3.2.1	A coordinating earth station operating with a geostationary space station with respect to unknown earth stations operating with non-geostationary space stations

When the coordinating earth station operates with a space station in the geostationary orbit and the unknown earth stations operate with space stations in non-geostationary orbits, the propagation mode (1) coordination area is determined using the procedures described in § 2.1.1. The only modification needed is to use the horizon antenna gain, Gr, of the unknown receiving earth station in place of the terrestrial station gain, Gx. The appropriate values for this gain and the appropriate system parameters are contained in Table 9.

### 3.2.2	A coordinating earth station operating with non-geostationary space stations with respect to unknown earth stations operating with geostationary space stations

When the coordinating earth station operates to space stations in non-geostationary orbits and the unknown earth stations operate with space stations in the geostationary orbit, the horizon antenna gain, Gr, for the unknown receiving earth station is determined in accordance with the simplifying approximations of § 3.1.1, as elaborated in § 2.1 of Annex 5, and the parameters of Table 9. Determination of the propagation mode (1) coordination area then follows the procedure of § 2.2 by using the appropriate horizon gain of the receiving earth station at each azimuth under consideration and the appropriate system parameters from Table 9.

### 3.2.3	Coordinating and unknown earth stations operating with non-geostationary space stations

When the coordinating earth station operates with space stations in non-geostationary orbits and the unknown earth stations operate with space stations in non-geostationary orbits, the propagation mode (1) coordination area is determined using the procedure described in § 2.2. The only modification is to use the horizon antenna gain, Gr, of the unknown receiving earth station in place of the terrestrial station antenna gain. The appropriate values for this gain and the appropriate system parameters are given in Table 9.

# 4	General considerations for the determination of the propagation mode (1) required distance

For the determination of the propagation mode (1) required distances, the applicable frequency range has been divided into three parts. The propagation calculations for the VHF/UHF frequencies between 100 MHz and 790 MHz are based upon propagation mode (1) predicted path loss curves. From 790 MHz to 60 GHz the propagation modelling uses tropospheric scatter, ducting and layer reflection/refraction models. At higher frequencies up to 105 GHz, the model is based on a free-space loss and a conservative assumption for gaseous absorption. The possible range of time percentages is different in the different propagation models.

After taking site shielding (see § 1 of Annex 1) into consideration, for the coordinating earth station only, the following methods are used to determine the propagation mode (1) required distances:

–	For frequencies between 100 MHz and 790 MHz, the method described in § 2 of Annex 1.

–	For frequencies between 790 MHz and 60 GHz, the method described in § 3 of Annex 1.

–	For frequencies between 60 GHz and 105 GHz, the method described in § 4 of Annex 1.

The three methods referred to above rely on a value of propagation mode (1) minimum required loss, determined according to the appropriate system parameters in Tables 7, 8 and 9.

## 4.1	Radio-climatic information

For the calculation of the propagation mode (1) required distance, the world has been classified in terms of a radio-meteorological parameter representing clear-air anomalous propagation conditions. The percentage of time βe for which these clear-air anomalous propagation conditions exist, is latitude dependent and is given by:

with:

where  is the latitude of the earth station’s location (degrees).

For frequencies between 790 MHz and 60 GHz, the path centre sea level surface refractivity, N0, is used in the propagation mode (1) calculations. This can be calculated using:

(11)

## 4.2	Minimum coordination distance for propagation modes (1) and (2)

The minimum coordination distance can be calculated in two steps. First calculate distance dx using:

km	(12)

where βe is given in § 4.1.

Then calculate the minimum coordination distance at any frequency, f (GHz) in the range 100 MHz to 105 GHz using:

The distance from which all iterative calculations start (for both propagation mode (1) and propagation mode (2)), is the minimum coordination distance, dmin, as given in equations (13) to (18).

## 4.3	Maximum coordination distance for propagation mode (1)

In the iterative calculation described in Annex 1, it is necessary to set an upper limit, dmax1, to the propagation mode (1) coordination distance.

For frequencies less than or equal to 60 GHz and propagation paths entirely within a single Zone, the distance shall not exceed the maximum coordination distance given in Table 3 for that Zone.

For mixed paths, the required distance can comprise one or more contributions from Zones A1, A2, B and C. The aggregate distance for any one zone must not exceed the value given in Table 3. The overall required distance must not exceed the value in Table 3 for the zone in the mixed path having the largest Table 3 value. Thus, a path comprising both Zones A1 and A2 must not exceed 500 km.

TABLE 3

Maximum coordination distances for propagation mode (1) 
for frequencies below 60 GHz

| Zone | dmax1 (km) |
| --- | --- |
| A1 | 500 |
| A2 | 375 |
| B | 900 |
| C | 1 200 |

For frequencies above 60 GHz, the maximum coordination distance, dmax1, is given by:

(19)

where p is defined in § 1.3.

## 4.4	Guidance on application of propagation mode (1) procedures

As explained in § 1.3, for those cases where earth stations are sharing with terrestrial stations, it is appropriate to apply a correction factor, Ci (dB), to the worst-case assumptions on system parameters and interference path geometry. This correction factor takes into account the fact that the assumption that all the worst-case values will occur simultaneously is unrealistic when determining the propagation mode (1) required distances.

The characteristics of terrestrial systems depend on the frequency band, and the value of the correction factor to be applied follows the frequency dependence given in equation (20). At frequencies between 100 MHz and 400 MHz, and between 60 GHz and 105 GHz, sharing between earth stations and terrestrial systems is a recent development and there is little established practical experience, or opportunity to analyse operational systems. Hence, the value of the correction factor is 0 dB in these bands. Between 400 MHz and 790 MHz and between 4.2 GHz and 60 GHz, the value of the correction factor is reduced in proportion to the logarithm of the frequency, as indicated in equation (20).

The value of the nominal correction to be used at any frequency f (GHz) is therefore given by:

(20)

where:

X:	15 dB for a transmitting earth station and 25 dB for a receiving earth station.

In principle, the value of the nominal correction factor, X(f ), is distance and path independent. However, there are a number of issues relating to interference potential at the shorter distances, and it is not appropriate to apply the full nominal correction at these distances. The correction factor Ci is therefore applied proportionally with distance along the azimuth under consideration, starting with 0 dB at dmin, such that the full value of X(f ) is achieved at a nominal distance of 375 km from the earth station.

Hence, the correction is applied using the correction constant Z(f ) (dB/km) where:

dB/km	(21)

The correction factor Ci (dB) is calculated in equations (28b) and (52) from the correction constant Z(f ) (dB/km).

At distances greater than 375 km, the correction factor Ci to be applied is the value of Ci at 375 km distance.

In addition, the correction factor is applied to its highest value only on land paths. The correction factor is 0 dB for wholly sea paths. A proportion of the correction factor is applied on mixed paths. The amount of correction to be applied to a particular path is determined by the path description parameters used for the propagation mode (1) calculation (correction factors Ci and C2i in § 2 and § 3 respectively of Annex 1). As the correction factor is distance dependent, it is applied automatically within the iterative calculation used to determine the propagation mode (1) required distance (see Annex 1).

The correction factor does not apply to the bidirectional case and therefore in the determination of the bidirectional coordination contour:

dB/km

For the determination of propagation mode (1) auxiliary contours, the propagation mode (1) minimum required loss Lb(p ) for p% of time in equation (1) (see § 1.3) is replaced by:

Lbq(p) = Lb(p) + Q                   dB	(22)

where:

Q:	auxiliary contour value (dB).

Note that auxiliary contour values are assumed to be negative (i.e. −5, −10, −15, −20 dB, etc.).

# 5	General considerations for the determination of the propagation mode (2) required distance

The determination of the contour for scattering from hydrometeors (e.g. rain scatter) is predicted on a path geometry that is substantially different from that of the great-circle propagation mechanisms. Hydrometeor scatter can occur where the beams of the earth station and the terrestrial station intersect (partially or completely) at, or below, the rain height hR (see § 3 of Annex 2). It is assumed that at heights above this rain height the effect of scattering will be suppressed by additional attenuation, and it will not, therefore, contribute significantly to the interference potential. For the determination of the propagation mode (2) contour, it is assumed that the main beam of any terrestrial station exactly intersects the main beam of the coordinating earth station. The mitigating effects of partial beam intersections can be determined using propagation mode (2) auxiliary contours.

Since, to a first approximation, microwave energy is scattered isotropically by rain, interference can be considered to propagate equally at all azimuths around the common volume centred at the beam intersection (see § 1.3). Generally, the beam intersection will not lie on the great-circle path between the two stations. A common volume can therefore result from terrestrial stations located anywhere around the earth station, including locations behind the earth station.

The propagation mode (2) contour is a circle with a radius equal to the propagation mode (2) required distance. Unlike the case for propagation mode (1), the propagation mode (2) contour is not centred on the earth station’s physical location, instead it is centred on a point on the earth’s surface immediately below the centre of the common volume.

A common volume can exist, with equal probability, at any point along the earth station beam between the earth station’s location and the point at which the beam reaches the rain height. To provide appropriate protection for/from terrestrial stations8, the centre of the common volume is assumed to be half way between the earth station and the point at which its beam intersects the rain height. The distance between the projection of this point on to the Earth's surface and the location of the earth station is known as Δd (see § 4 of Annex 2). The centre of the propagation mode (2) contour is therefore Δd (km) from the earth station on the azimuth of the earth station’s main beam axis.

## 5.1	The required distance for propagation mode (2)

Propagation mode (2) required distances are measured along a radial originating at the centre of the rain scatter common volume. The calculation requires iteration for distance, starting at the same minimum distance as that defined for propagation mode (1) until either the required propagation mode (2) minimum required loss, or a latitude-dependent propagation mode (2) maximum calculation distance, is achieved. The propagation mode (2) calculations use the method described in Annex 2. The calculations only need to be performed in the frequency range 1 000 MHz to 40.5 GHz. Outside this frequency range, rain scatter interference can be neglected and the propagation mode (2) required distance is set to the minimum coordination distance given by equations (13) to (18).

ANNEX 1

Determination of the required distance for propagation mode (1)

# 1	Adjustments for earth station horizon elevation angle and distance

For propagation mode (1), the required distance depends on the characteristics of the physical horizon around the earth station. The horizon is characterized by the horizon distance dh (see below), and the horizon elevation angle h. The horizon elevation angle is defined here as the angle (degrees), viewed from the centre of the earth station antenna, between the horizontal plane and a ray that grazes the physical horizon in the direction concerned. The value of h is positive when the physical horizon is above the horizontal plane and negative when it is below.

It is necessary to determine horizon elevation angles and distances for all azimuths around an earth station. In practice it will generally suffice to do this in azimuth increments of 5°. However, every attempt should be made to identify, and take into consideration, minimum horizon elevation angles that may occur between those azimuths examined in 5° increments.

For the purposes of the determination of the propagation mode (1) required distance it is useful to separate the propagation effects related to the local horizon around the earth station which, on some or all azimuths, may be determined by nearby hills or mountains, from the propagation effects on the remainder of the path. This is achieved by referencing the propagation model to a 0° horizon elevation angle for the coordinating earth station, and then to include a specific term Ah to deal with the known horizon characteristics of the earth station being coordinated. Where appropriate, Ah modifies the value of the path loss, on each azimuth, from which the propagation mode (1) required distance is derived.

There are two situations in which the level of attenuation for the propagation mode (1) path loss with respect to the reference 0° case can change:

–	The first is where the coordinating earth station has a positive horizon elevation angle (on a particular azimuth). In this case, it will benefit from additional diffraction propagation losses over the horizon (generally referred to as site shielding). As a result, the attenuation Ah is positive and the value of the required path loss is reduced, with respect to the reference 0° horizon elevation angle case (see equations (27a) and (27b)).

–	The second situation is where the coordinating earth station is at a location above the local foreground, and has a negative (downward) horizon elevation angle on a particular azimuth. In this case, a measure of additional protection is necessary because the path angular distance along the radial is reduced and hence the path loss for a given distance will be lower than for the zero degree elevation angle case. It is convenient to deal with this effect as part of the site shielding calculation. As a result, the attenuation Ah will be negative and the value of the required path loss is increased, with respect to the reference 0° horizon elevation angle case.

The contribution made by the attenuation arising from the coordinating earth station’s horizon characteristics to the propagation mode (1) minimum required loss modifies the value of path loss that then needs to be determined in the three propagation mode (1) models. The attenuation Ah is calculated for each azimuth around the coordinating earth station as follows.

The distance of the horizon, dh, from the earth station’s location, is determined by:

The contribution made by the horizon distance, dh, to the total site shielding attenuation is given by Ad (dB) for each azimuth using:

dB	(23)

where f is the frequency (GHz) throughout this Annex.

The total site shielding attenuation along each azimuth from the coordinating earth station is given by:

The value of Ah must be limited to satisfy the conditions:

−10 ≤ Ah  ≤ (30 + εh)	(25)

In equations (23), (24) and (25) the value of h must always be expressed in degrees. The limits defined in equation (25) are specified because protection outside these limits may not be realized in practical situations.

# 2	Frequencies between 100 MHz and 790 MHz

The propagation model given in this section is limited to an average annual time percentage, p, in the range 1% to 50%.

An iterative process is used to determine the propagation mode (1) required distance. First, equation (27) is evaluated. Then, commencing at the minimum coordination distance, dmin, given by the method described in § 1.5.3 of the main body of this Appendix, equations (28) to (31) are iterated for distances di (where i = 0, 1, 2,...) incremented in steps of s (km) as described in § 1.3 of the main body of this Appendix. In each iteration, di is the distance considered. This process is continued until either of the following expressions becomes true:

|  | for the main or supplementary contour for the auxiliary contour | (26a) |
| --- | --- | --- |

or:

|  | for the main or supplementary contour for the auxiliary contour | (26b) |
| --- | --- | --- |

The required distance, d1, or the auxiliary contour distance, dq, are then given by the distance for the last iteration: i.e.

d1 = di	(26c)

or:

dq = di	(26d)

As the eventual mix of zones along a path is unknown, all paths are treated as if they are potential land and sea paths. Parallel calculations are undertaken, the first assuming the path is all land and a second assuming it is all sea. A non-linear interpolation is then performed, the output of which depends upon the current mix of land and sea losses in the distance, di. Where the current mix along the path includes sections of both warm sea and cold sea zones, all the sea along that path is assumed to be warm sea.

For the main or supplementary contour:

L1(p ) = Lb(p ) – Ah	(27a)

For an auxiliary contour:

L1q(p ) = Lbq(p ) – Ah	(27b)

where:

Lb(p ) (dB) and Lbq(p ) (dB):		minimum required loss required for p% of the time for the main or supplementary contour and the auxiliary contour with value Q (dB), respectively (see equation (22)).

Iterative calculations

At the start of each iteration calculate the current distance for i = 0, 1, 2,…:

di = dmin + i · s	(28a)

The correction factor, Ci (dB), (see § 4.4 of the main body of this Appendix) for the distance, di, is given by:

|  | dB dB | for the main or supplementary contour for the auxiliary contour | (28b) |
| --- | --- | --- | --- |

where Z(f) is given by equation (21) in § 4.4 of the main body of this Appendix.

At distances greater than 375 km, the value of the correction factor (Ci in equation (28b)) to be applied is the value of Ci at the 375 km distance.

The loss, Lbl(p ), where it is assumed that the path is wholly land (Zones A1 or A2), is evaluated successively using:

(29)

The loss, Lbs(p ), where it is assumed that the path is wholly cold sea (Zone B) or warm sea (Zone C), is evaluated successively using:

|  | (30a) (30b) |
| --- | --- |

The predicted path loss at the distance considered is then given by:

(31)

where:

dtm (km):	longest continuous land (inland + coastal) distance, i.e. Zone A1 + Zone A2 along the current path.

# 3	Frequencies between 790 MHz and 60 GHz

The propagation model given in this section is limited to an average annual time percentage (p ) in the range 0.001% to 50%.

An iterative process is used to determine the propagation mode (1) required distance. First, equations (33) to (42) are evaluated. Then, commencing at the minimum coordination distance, dmin, equations (43) to (53) are iterated for distances di, where i = 0, 1, 2, ..., incremented in steps of s (km) as described in § 1.3 of the main body of this Appendix. For each iteration, di is the distance considered. This process is continued until either of the following expressions becomes true:

|  | for the main or supplementary contour for the auxiliary contour | (32a) |
| --- | --- | --- |

or:

|  | for the main or supplementary contour for the auxiliary contour | (32b) |
| --- | --- | --- |

The required distance, d1, or the auxiliary contour distance, dq, is then given by the current distance for the last iteration, i.e.:

d1 =di	(32c)

or:

dq =di	(32d)

Specific attenuation due to gaseous absorption

Calculate the specific attenuation (dB/km) due to dry air:

The specific attenuation due to water vapour is given as a function of  (the water vapour density (g/m3)) by the following equation:

(34)

Calculate the specific attenuation (dB/km) due to water vapour for the troposcatter propagation model using a water vapour density of 3.0 g/m3:

wt = w (3.0)	(35a)

Calculate the specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 7.5 g/m3 for paths over land, Zones A1 and A2, using:

wdl = w (7.5)	(35b)

Calculate the specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 10.0 g/m3 for paths over sea, Zones B and C, using:

wds = w (10.0)	(35c)

Note that the value of 10 g/m3 is used for both Zones B and C in view of the lack of data on the variability of water vapour density on a global basis, particularly the minimum values.

Calculate the frequency-dependent ducting specific attenuation (dB/km):

(36)

For the ducting model

Calculate the reduction in attenuation arising from direct coupling into over-sea ducts (dB):

(37)

where dc (km) is the distance from a land based earth station to the coast in the direction being considered.

dc is zero in other circumstances.

Calculate the minimum loss to be achieved within the iterative calculations:

(38)

For the main or supplementary contour:

(39a)

For an auxiliary contour:

(39b)

where:

Lb(p ) (dB) and Lbq(p ) (dB):	minimum required loss required for p% of the time for the main or supplementary contour and the auxiliary contour with value Q (dB) respectively (see equation (22)).

For the tropospheric scatter model

Calculate the frequency-dependent part of the losses (dB):

(40)

Calculate the non-distance-dependent part of the losses (dB):

(41)

where:

h:	earth station horizon elevation angle (degrees)

N0:	path centre sea level surface refractivity (see equation (11), § 4.1 in the main body of this Appendix).

Calculate the minimum required value for the distance dependent losses (dB):

For the main, or supplementary, contour:

L4(p) = Lb(p) – A2	(42a)

For an auxiliary contour:

(42b)

where:

Lb(p ) (dB) and Lbq(p ) (dB):	minimum required loss required for p% of the time for the main or supplementary contour and the auxiliary contour of value Q (dB) respectively (see equation (22)).

Iterative calculations

At the start of each iteration, calculate the distance considered for i = 0, 1, 2, ..:

di = dmin + i · s	(43)

Calculate the specific attenuation due to gaseous absorption (dB/km):

(44)

where:

dt (km):	current aggregate land distance, Zone A1 + Zone A2, along the current path.

Calculate the following zone-dependent parameters:

(45)

where:

dlm (km):	longest continuous inland distance, Zone A2, along the path considered;

(46)

where:

dtm (km):	longest continuous land (i.e. inland + coastal) distance, Zone A1 + Zone A2 along the path considered.

μ1 shall be limited to μ1 ≤ 1.

(47)

shall be limited to   −3.4.

(48)

μ2 shall be limited to μ2 ≤ 1.

|  | for ζr ≤ 70° for ζr ≤ 70° | (49a) (49b) |
| --- | --- | --- |

where ζr is given in equations (9) and (10), § 4.1 in the main body of this Appendix.

Calculate the path-dependent incidence of ducting, , and a related parameter, Γ1 used to calculate the time dependency of the path loss:

β = βe · μ1 · μ2 · μ4	(50)

where e is given in equations (7) and (8), § 4.1 in the main body of this Appendix.

(51)

Calculate the correction factor, C2i (dB) (see § 4.4 in the main body of this Appendix) using:

|  | dB dB | for the main or supplementary contour for the auxiliary contour | (52) |
| --- | --- | --- | --- |

where Z(f) is calculated using equation (21) in § 4.4 in the main body of this Appendix.

At distances greater than 375 km the value of the correction factor C2i in equation (52) to be applied is the value of C2i at the 375 km distance.

Calculate the distance-dependent part of the losses (dB) for ducting:

(53)

and for tropospheric scatter:

(54)

For the determination of distances for auxiliary contours, C2i = 0 dB.

# 4	Frequencies between 60 GHz and 105 GHz

This propagation model is valid for average annual percentage time (p ) in the range from 0.001% to 50%.

An iterative process is used to determine the propagation mode (1) required distance. First, equations (55) to (59) are evaluated. Then commencing at the minimum coordination distance, dmin, equations (60) and (61) are iterated for distances di, where i = 0, 1, 2,..., incremented in steps of s (km) as described in § 1.3 of the main body of this Appendix. For each iteration, di is the distance considered.

This process is continued until either of the following expressions becomes true:

|  | for the main or supplementary contour for the auxiliary contour | (54a) |
| --- | --- | --- |

or:

|  | for the main or supplementary contour for the auxiliary contour | (54b) |
| --- | --- | --- |

The required distance, d1, or the auxiliary contour distance dq are then given by the current distance for the last iteration: i.e.

d1 = di	(54c)

or:

dq = di	(54d)

Calculate the specific attenuation (dB/km) for dry air in the frequency range 60 GHz to 105 GHz using:

|  | (55a) (55b) |
| --- | --- |

Calculate the specific attenuation (dB/km) for an atmospheric water vapour density of 3 g/m3 using:

(56)

Calculate a conservative estimate of the specific attenuation (dB/km) for gaseous absorption using:

gm = om + wm                    dB/km	(57)

For the required frequency and the value of earth station site shielding, Ah (dB), as calculated using the method described in § 1 of this Annex, calculate the minimum loss to be achieved in the iterative calculations:

L7(p )  = 92.5 + 20 log (f) + Ah                    dB	(58)

For the main or supplementary contour:

L8(p ) = Lb(p ) – L7                                       dB	(59a)

For an auxiliary contour:

dB	(59b)

where:

Lb(p ) (dB) and Lbq(p ) (dB):	minimum required loss required for p% of the time for the main or supplementary contour and the auxiliary contour of value Q (dB) respectively (see equation (22)).

Iterative calculations

At the start of each iteration calculate the distance for i = 0, 1, 2, ...:

di = dmin + i · s	(60)

Calculate the distance-dependent losses for the distance:

(61)

For frequencies above 60 GHz, the correction factor (see § 4.4 in the main body of this Appendix) is 0 dB. Therefore, no correction term has been added to equation (61).

ANNEX 2

Determination of the required distance for propagation mode (2)

# 1	Overview

The algorithm given below allows propagation mode (2) path loss, Lr(p ) (dB), to be obtained as a monotonic function of rainfall rate, R(p ) (mm/h), and with the hydrometeor scatter distance, ri (km), as a parameter. The model is valid for average annual time percentage (p ) in the range 0.001% to 10%. The procedure to determine the hydrometeor scatter contour is as follows:

a)	The value of R(p ), is determined for the appropriate rain climatic Zones A to Q.

b)	Values of Lr(p ), are then calculated for incremental values of ri, starting at the minimum coordination distance dmin, in steps of s (km), as described in § 1.3 of the main body of this Appendix. The correct value of ri is that for which the corresponding value of Lr(p ) equals or exceeds the propagation mode (2) minimum required loss L(p ). This value of ri is the propagation mode (2) required distance and is denoted dr.

c)	If the iterative calculation results in ri equalling or exceeding the appropriate maximum calculation distance (dmax2) given in § 2, then the calculation is terminated and dr is assumed to be equal to dmax2. Hence the iteration stops when either of the following expressions becomes true:

Lr(p )  ≥  L(p )	(62a)

or:

ri  ≥  dmax2	(62b)

d)	The contour for propagation mode (2) is a circle of radius dr (km) centred on a point along the azimuth of the earth station antenna main beam at a horizontal distance of Δd (km) from the earth station.

# 2	Maximum calculation distance

As discussed in § 1.5.3 of the main body of this Appendix, it is necessary to set upper limits to the maximum distance used in the iterative calculation of the required distance. The maximum calculation distance to be used for propagation mode (2) (dmax2) is latitude dependent and is given in the following equation:

km

where hR is defined in equations (74) and (75).

# 3	Calculation of the propagation mode (2) contour

Determine R(p ), the rainfall rate (mm/h) exceeded on average for p% of a year. The world has been divided into a number of rain climatic zones (see Figs. 2, 3 and 4) which show different precipitation characteristics.

The curves shown in Fig. 5 represent consolidated rainfall-rate distributions, each applicable to several of these rain climatic zones.

Determine which rain climatic zone is applicable to the location of the earth station:

–	For 0.001% < p < 0.3% and the applicable rain climatic zone:

Determine R(p ) either from Fig. 5 or from equations (63) to (67).

–	For p ≥ 0.3%:

Use equation (68) with values of R(0.3%) and pc obtained from Table 4.

Rain climatic Zones A, B

mm/h	(63)

Rain climatic Zones C, D, E

mm/h	(64)

Rain climatic Zones F, G, H, J, K

mm/h	(65)

Rain climatic Zones L, M

mm/h	(66)

Rain climatic Zones N, P, Q

mm/h	(67)

TABLE 4

Values of R and pc for the different rain climatic zones

| Rain climatic zone | R (0.3%) (mm/h) | pc (%) |
| --- | --- | --- |
| A, B | 1.5 | 2 |
| C, D, E | 3.5 | 3 |
| F, G, H, J, K | 7.0 | 5 |
| L, M | 9.0 | 7.5 |
| N, P, Q | 25.0 | 10 |
| where: pc (%): reference time percentage above which the rainfall rate R(p) can be assumed to be zero. |  |  |

(68)

Determine the specific attenuation (dB/km) due to rain using values of k and α from Table 5 in equation (70). Values of k and α at frequencies other than those in Table 5 can be obtained by interpolation using a logarithmic scale for frequency, a logarithmic scale for k and a linear scale for α.

TABLE 5

Values of k and α for vertical polarization as a function of the frequency

| Frequency (GHz) | k |  |
| --- | --- | --- |
| 1 | 0.0000352 | 0.880 |
| 4 | 0.000591 | 1.075 |
| 6 | 0.00155 | 1.265 |
| 8 | 0.00395 | 1.31 |
| 10 | 0.00887 | 1.264 |
| 12 | 0.0168 | 1.20 |
| 14 | 0.029 | 1.15 |
| 18 | 0.055 | 1.09 |
| 20 | 0.0691 | 1.065 |
| 22.4 | 0.090 | 1.05 |
| 25 | 0.113 | 1.03 |
| 28 | 0.150 | 1.01 |
| 30 | 0.167 | 1.00 |
| 35 | 0.233 | 0.963 |
| 40 | 0.310 | 0.929 |
| 40.5 | 0.318 | 0.926 |

Let:

R = R(p )	(69)

Then the specific attenuation (dB/km) due to rain is given by:

(70)

Calculate the effective diameter of the rain cell:

(71)

Then, calculate the effective scatter transfer function:

(72)

Calculate the additional attenuation outside the common volume:

(73)

Determine the rain height above ground, hR (km):

For North America and Europe west of 60° E longitude:

hR = 3.2 – 0.075 ( – 35)                    for    35 ≤  ≤ 70	(74)

where:

:	latitude of the coordinating earth station.

For all other areas of the world:

|  | (75a) (75b) (75c) (75d) (75e) |
| --- | --- |

Determine the specific attenuation due to water vapour absorption (a water vapour density of 7.5 g/m3 is used):

(76)

## 3.1	Iterative calculations

Evaluate equations (77) to (82) inclusive for increasing values of ri, where ri is the current distance considered (km) between the region of maximum scattering and the possible location of a terrestrial station and i = 0, 1, 2,... Continue this process until either of the conditions given in equations (62a) and (62b) is true. Then the rain-scatter required distance dr is the current value of ri.

ri = dmin + i · s	(77)

Determine the loss above the rain height, Lar (dB), applicable to scatter coupling:

Calculate the additional attenuation for the departure from Rayleigh scattering:

Calculate the effective path length for oxygen absorption:

|  | (80a) (80b) |
| --- | --- |

Calculate the effective path length for water vapour absorption:

|  | (81a) (81b) |
| --- | --- |

Determine the propagation mode (2) path loss, Lr (dB):

(82)

where:

o:	as given in equation (33)

Gx:	terrestrial network antenna gain in Tables 7 or 8.

# 4	Construction of the propagation mode (2) contour

In order to determine the centre of the circular propagation mode (2) contour, it is necessary to calculate the horizontal distance to this point from the earth station, along the azimuth of the earth station antenna main beam axis. The distance, Δd (km), to the centre of the propagation mode (2) contour is given by:

(83)

where:

εs:	earth station antenna main beam axis elevation angle

and

d:	shall be limited to the distance (dr – 50) km.

The propagation mode (2) required distance, dr, must lie within the range between the minimum coordination distance, dmin, and the propagation mode (2) maximum calculation distance, dmax2.

Draw the propagation mode (2) contour as a circle of radius dr (km) around the centre determined above. The propagation mode (2) contour is the locus of points on this circle. However, if any part of the propagation mode (2) contour falls within the contour defined by the minimum coordination distance, this arc of the propagation mode (2) contour is taken to be identical to the contour based on the minimum coordination distance and the propagation mode (2) contour is then no longer circular.

ANNEX 3

Antenna gain towards the horizon for an earth station
operating with a geostationary space station

# 1	General

The gain component of the earth station antenna in the direction of the physical horizon around an earth station is a function of the angular separation between the antenna main beam axis and the horizon in the direction under consideration. When the earth station is used to transmit to a space station in a slightly inclined orbit, all possible pointing directions of the antenna main beam axis need to be considered. For earth station coordination, knowledge of φ(α), the minimum possible value of the angular separation that will occur during the operation of the space station, is required for each azimuth.

When a geostationary space station maintains its location close to its nominal orbital position, the earth station’s main beam axis elevation angle, εs, and the azimuth angle, αs, to the space station from the earth station’s latitude, ζ, are uniquely related. Figure 6 shows the possible location arcs of positions of a space station on the geostationary orbit in a rectangular azimuth/elevation plot. It shows arcs corresponding to a set of earth station latitudes and the intersecting arcs correspond to points on the orbit with a fixed difference in longitude East or West of the earth station. Figure 6 also shows a portion of the horizon profile εh(α). The off-axis angle φ(α) between the horizon profile at an azimuth of 190° and a space station located 28° W of an earth station at 43° N latitude is indicated by the great-circle arc shown dashed on Fig. 6.

When the north/south station-keeping of a geostationary satellite is relaxed, the orbit of the satellite becomes inclined, with an inclination that increases gradually with time. As viewed from the Earth, the position of the satellite traces a figure eight during each 24-hour period. Figure 7 shows the variations in the trajectories of a set of satellites, each with 10° inclination, spaced by 3° along the geostationary orbit from 28° W to 44° E, with respect to an earth station at 43° N latitude. Figure 7 also shows, with a dashed curve, the great-circle arc corresponding to the minimum off-axis angle φ(α) between a point on the trajectory of one of the satellites and the horizon profile at an azimuth of 110°.

For a transmitting earth station operating in a frequency band that is also allocated for bidirectional use by receiving earth stations operating with geostationary space stations, refer to § 2.1 of Annex 5.

# 2	Determination of the angular separation φ()

For the determination of the off-axis angle φ(α), two cases are distinguished. These depend on whether the orbit of the space station has no inclination, or is slightly inclined. The following equations may be used in both of these cases:

ψs(i, δ) = arccos (sin ζ sin i + cos ζ cos i cos δ)	(84)

(85)

(86)

|  | for a space station located east of the earth station (δ ≥ 0) | (87) |
| --- | --- | --- |

|  | for a space station located west of the earth station (δ ≤ 0) | (88) |
| --- | --- | --- |

(89)

where:

ζ:	latitude of the earth station (positive for north; negative for south)

δ:	difference in longitude between the earth station and a space station

i:	latitude of a sub-satellite point (positive for north; negative for south)

ψs(i, δ):	great-circle arc between the earth station and a sub-satellite point

αs(i, δ):	space station azimuth as seen from the earth station

εs(i, δ):	space station elevation angle as seen from the earth station

φλα, i, δ):	angle between the main beam and the horizon direction corresponding to the azimuth, α, under consideration when the main beam is steered towards a space station with a sub-satellite point at latitude, i, and longitude difference, δ

α:	azimuth of the direction under consideration

εh:	elevation angle of the horizon at the azimuth under consideration, α

φ(α):	angle to be used for horizon gain calculation at the azimuth under consideration, α

K:	orbit radius/Earth radius, which for the geostationary orbit is assumed to be 6.62.

All arcs mentioned above are in degrees.

Case 1:	Single space station, no orbital inclination

For a space station operating with no orbital inclination at an orbital position with difference in longitude δ0, equations (84) to (89) may be applied directly using i = 0 to determine φ(α) for each azimuth α. Thus:

φ() = φ, 0, )	(90)

where:

δ0:	difference in longitude between the earth station and the space station.

Case 2:	Single space station, slightly inclined orbit

For a space station operating in a slightly inclined orbit on a portion of the geostationary arc with a nominal longitude difference of δ0, the maximum orbital inclination over its lifetime, is, must be considered. Equations (84) to (89) may be applied to develop the minimum off-axis angle to each of four arcs in azimuth/elevation that bound the trajectory of the space station in angle and elevation. The bounding arcs correspond to the maximum and minimum latitudes of the sub-satellite points and the extremes of the difference in longitude between the earth and space stations when the space station is operating at its maximum inclination.

The determination of the minimum off-axis angles in equations (91) to (95) may be made by taking increments along a bounding contour. The step size in inclination i or longitude δ should be between 0.5° and 1.0° and the end points of the respective ranges should be included in the calculation.

The horizon profile εh(α) used in the determination of φ(α) is specified at increments in azimuth α that do not exceed 5°.

Thus:

(91)

with:

(92)

(93)

(94)

(95)

(96)

where:

is:	maximum operational inclination angle of the satellite orbit

δs:	maximum longitude change from nominal value of the sub-satellite point of a satellite with orbital inclination is.

# 3	Determination of antenna gain

The relationship φ(α) is used to derive a function for the horizon antenna gain (dBi), G(φ) as a function of the azimuth , by using the actual earth station antenna pattern, or a formula giving a good approximation. For example, in cases where the ratio between the antenna diameter and the wavelength is equal to or greater than 35, the following equation is used:

(97)

degrees

Where a better representation of the actual antenna pattern is available, it may be used.

In cases where D/ is not given, it may be estimated from the expression:

where:

Gamax:	main beam axis antenna gain (dBi)

D:	antenna diameter (m)

:	wavelength (m)

G1:	gain of the first side lobe (dBi).

ANNEX 4

Antenna gain toward the horizon for an earth station operating with
non-geostationary space stations

This Annex presents methods which may be used to determine the antenna gain towards the horizon for an earth station operating to non-geostationary satellites using the TIG method described in § 2.2 of the main body of this Appendix.

# 1	Determination of the horizon antenna gain

In its simplest implementation, the TIG method depends on the minimum elevation angle of the beam axis of the earth station antenna (εsys), which is a system parameter that has the same value on all azimuths from the earth station. If the horizon elevation angle at an azimuth under consideration is εh (degrees), the minimum separation angle from the horizon at this azimuth to any possible pointing angle for the main beam axis of the antenna (φmin) is equal to the difference between these two angles (εsys − εh), but it is not less than zero degrees. The maximum separation angle from the horizon at this azimuth to any possible pointing angle for the main beam axis of the antenna (φmax) is equal to the difference between the sum of these two angles and 180° (180 − sys − h). The maximum and minimum values of horizon gain for the azimuth under consideration are obtained from the gain pattern of the earth station antenna at these off-axis angles. Where no pattern is available the pattern of § 3 of Annex 3 may be used.

Additional constraints may be included in the determination of the maximum and minimum values of horizon antenna gain where an earth station operates with a constellation of non-geostationary satellites that are not in near-polar orbit. In this case, depending on the latitude of the earth station, there may be portions of the hemisphere above the horizontal plane at the earth station in which no satellite will appear. To include these visibility limitations within this method, it is first necessary to determine, for a closely spaced set of azimuth angles around the earth station, the minimum elevation angle at which a satellite may be visible. This minimum satellite visibility elevation angle () may be determined from consideration of the visibility of the edge of the shell formed by all possible orbits having the orbital inclination and altitude of the satellites in the constellation.

The lowest elevation angle towards which the main-beam axis of the earth station antenna will point on any azimuth is the minimum composite elevation angle (c), which is equal to the greater of the minimum satellite visibility elevation angle () and the minimum elevation angle of the earth station (sys). After the minimum composite elevation angle has been determined for all azimuths by the procedure of § 1.1 of this Annex, the resulting profile of the minimum composite elevation angles can be used, in the procedure of § 1.2 of this Annex, to determine the maximum and minimum values of horizon gain at any azimuth.

Further information and an example of this method may be found in the most recent version of Recommendation ITU-R SM.1448.     (WRC-15)

## 1.1	Determination of satellite visibility limits

The visibility limits of a constellation of satellites can be determined from the inclination angle of the most inclined satellite and the altitude of the lowest satellite in the constellation. For this determination, six cases may be distinguished, but not all of these may be applicable for a given constellation and a given earth station latitude. The azimuth and the corresponding lower limit on the elevation angle are developed by a parametric method using a set of points on the edge of the orbital shell of the constellation. The approach is to develop this relationship for azimuths to the east of a station in the northern hemisphere. Elevation angles for azimuths to the west of the station and for all azimuths for stations in the southern hemisphere are obtained by symmetry. The following equations, which are applicable to circular orbits only, may be used for the complete determination of the horizon antenna gain in all practical cases:

ψ(δ) = arccos (sin ζe sin is + cos ζe cos is cos δ)	(98)

(99)

(100)

with:

(101)

where:

is:	orbital inclination of the satellites in the constellation assumed to be positive and between 0° and 90°

ζe: 	modulus of the latitude of the earth station

δ:	difference in longitude from the earth station to a point on the edge of the orbital shell of the constellation

ψ(δ):	great-circle arc between the earth station and a point on the surface of the Earth directly below the point on the edge of the orbital shell of the constellation

α(δ):	azimuth from the earth station to a point on the edge of the orbital shell

α0(δ):	principal azimuth, an azimuth between 0° and 180°, from an earth station to a point on the edge of the orbital shell

(δ):	elevation angle from the earth station to a point on the edge of the orbital shell

K1:	orbit radius/Earth radius for the lowest altitude satellite in the constellation (Earth radius = 6 378.14 km)

ψm  =  arccos (1/K1).

All arcs mentioned above are in degrees.

For any latitude on the surface of the Earth, the azimuth for which the minimum elevation angle to a satellite can be greater than zero, and the corresponding elevation angles, may be determined by implementing the calculations under the following case(s). No more than two of these cases will be applicable for any latitude. For situations not specifically addressed in the following cases, no satellite is visible at elevation angles at or below 90° on any azimuth.

Case 1:	For: e ≤ is – ψm

For this case, a satellite may be visible to the horizon for all azimuths about the earth station ( = 0).

Case 2:	For: is – ψm < e ≤ arcsin (sin is cos ψm)

For this case, the azimuth angles and elevation are developed parametrically by choosing a set of values of δuniformly spaced on the interval 0 to δ1, and applying equations (98) to (101). For this purpose the spacing between values is not to exceed 1.0°, and the end points are to be included.

At any principal azimuth (α0(δ)) that is not included in the set, the minimum elevation angle is zero ( = 0), except for azimuths where Case 6 additionally applies.

Case 3:	For: arcsin (sin is cos ψm) < e  is and e  180° − ψm − is

For this case, the azimuth angles and elevation are developed parametrically by choosing a set of values of δuniformly spaced on the interval 0 to δ2, and applying equations (98) to (101). For this purpose the spacing between values is not to exceed 1.0°, and the end points are to be included.

At any principal azimuth (α0(δ)) that is not included in the set, the minimum elevation angle is zero (ε = 0), except for azimuths where Case 6 additionally applies.

Case 4:	For: is ≤ e < is + m and e < 180° – is – m

For this case, the minimum elevation angle is given explicitly in terms of the principal azimuth angle α0, as follows:

where:

Note that a minimum elevation angle of 90° in this formulation indicates that no satellite is visible at elevation angles at or below 90° on these azimuths. Furthermore, within the range of principal azimuths where the minimum elevation angle is zero, Case 6 may additionally apply.

Case 5:	For 180° – is – m  e  90°

For this case, a satellite may be visible to the horizon for all azimuths about the earth station (ε = 0).

Case 6:	For e  m – is

This case may occur additionally with Case 2, Case 3 or Case 4 and a satellite may be visible only above a minimum elevation angle for other principal azimuths.

For this case, the other principal azimuths and the corresponding elevation angles are developed parametrically by choosing a set of values of , uniformly spaced on the interval 0 to 3, and applying equations (98) to (101) with is replaced by −is. For this purpose the spacing between values is not to exceed 1.0° and the end points are to be included.

## 1.2	Determination of minimum and maximum horizon gain from the minimum visible elevation angle profile

The horizon gain of the earth station antenna is determined from the profile of values of the minimum composite elevation angle (εc). At any azimuth, the minimum composite elevation angle is the greater of the minimum satellite visibility elevation angle at that azimuth () and the minimum elevation angle for the earth station (εsys). The following procedure may be used to determine the maximum and minimum values of horizon antenna gain for each azimuth under consideration.

The following equation may be used to determine the angular separation between the horizon profile, at an azimuth angle  and horizon elevation angle h, and a point on the profile of the minimum composite elevation angle, where the minimum composite elevation angle is εc at an azimuth angle of αc:

(102)

where:

α:	azimuth of the direction under consideration

εh (α):	elevation angle of the horizon at the azimuth under consideration, α

εc (αc):	minimum composite elevation angle at the azimuth, αc

αc:	azimuth corresponding to εc.

The minimum value of the separation angle φmin, for the azimuth under consideration, is determined by finding the minimum value of φ(α, αc) for any azimuth αc, and the maximum value, φmax, is determined by finding the maximum value of φ(α, αc) for any azimuth αc. The azimuth angles (α) are usually taken in increments of 5°; however, to accurately determine the minimum separation angle, the values of the minimum composite elevation angle, c, need to be determined for a spacing of 1° or less in the azimuth αc. Where the procedures in § 1.1 of this Annex do not provide a profile of minimum composite elevation angle with a close enough spacing in azimuth angles, linear interpolation may be used to develop the necessary intermediate values. The maximum and minimum horizon antenna gains, Gmax and Gmin, to be used in the equations of § 2.2 of the main body of this Appendix for the azimuth under consideration are obtained by applying the off-axis angles, φmin and φmax, respectively, in the earth station antenna pattern. If the earth station antenna pattern is not known then the antenna pattern in § 3 of Annex 3 is used. In many cases, φmax will be large enough on all azimuths so that Gmin will be equal to the minimum gain of the antenna pattern at all azimuths.

ANNEX 5

Determination of the coordination area for a transmitting earth station 
with respect to receiving earth stations operating with 
geostationary space stations in bidirectionally 
allocated frequency bands

# 1	Introduction

The propagation mode (1) coordination area of a transmitting earth station with respect to unknown receiving earth stations operating with geostationary space stations requires the determination of the horizon gain of the antenna of the receiving earth station at each azimuth of the transmitting earth station. Different methods then need to be applied to determine the coordination area of the coordinating earth station, depending on whether it operates with geostationary or non-geostationary space stations. When both the coordinating earth station and the unknown receiving earth stations operate with geostationary space stations, it is also necessary to determine a propagation mode (2) coordination contour.

The coordination area of a transmitting earth station, with respect to unknown receiving earth stations that operate to non-geostationary space stations, can be determined by minor modifications to the methods applicable to the determination of coordination area of transmitting earth stations with respect to terrestrial stations. (See § 3.2.1 and § 3.2.3 of the main body of the Appendix.)

# 2	Determination of the bidirectional coordination contour for propagation mode (1)

For a transmitting earth station operating in a frequency band that is also allocated for bidirectional use by receiving earth stations operating with geostationary space stations, further development of the procedures in Annex 3 is needed. It is necessary to determine the horizon gain of the unknown receiving earth station, the horizon gain to be used at each azimuth at the coordinating (transmitting) earth station, for the determination of the bidirectional coordination area.

## 2.1	Calculation of horizon gain for unknown receiving earth stations operating with geostationary space stations

The value of Gr, the horizon gain of the receiving earth station, for each azimuth, α, at the transmitting earth station is found by the following steps:

Step 1:	The receiving earth station may be operating with any satellite in the geostationary orbit above a minimum elevation angle, min, contained in Table 9. The maximum difference in longitude (b (degrees)) between the receiving earth station and its associated space station occurs at this minimum elevation angle, min, and is given by:

(103)

where:

: 	latitude of the receiving earth station, which is assumed to be the same as the transmitting earth station

K: 	ratio of the radius of the satellite orbit to the radius of the Earth, equal to 6.62.

Step 2:	For each azimuth, α, at the transmitting earth station:

–	determine the azimuth αr from the receiving earth station to the transmitting earth station:

r    180              for   180

r   – 180              for   180

–	for each azimuth αr, determine the minimum angular separation, (αr), between the receiving earth station main beam axis and the horizon at this azimuth using Case 1 in § 2 of Annex 3. For this evaluation, (αr) is the minimum value of (αr, 0, δ0), where the values of δ0 are between −δb and +δb in steps of 1° or less, making sure to include the end points.

The minimum angular separation, φ(αr), may be used with the gain pattern in § 3 of Annex 3 to determine the horizon gain for this azimuth, α, unless a different gain pattern is referenced in Table 9.

Figure 8 shows plots of the minimum angular separation between the horizon at zero degrees elevation on an azimuth αr and a satellite on the geostationary orbit at an elevation above 3°. Plots are shown for a set of values of the station latitude, ζ, which is assumed to be the same for both transmitting and receiving earth stations. Figure 8 also provides a scale showing the corresponding azimuth, α, of the transmitting earth station.

Further information and an example may be found in the most recent version of Recommendation ITU-R SM.1448.     (WRC-15)

# 3	Determination of the bidirectional rain scatter contour

The procedure for the determination of the bidirectional rain scatter area, as described in § 3.1.2 of the main body of this Appendix, is as follows:

The horizontal distance ds (km) from the coordinating earth station to the point at which the main beam axis attains the rain height hR is calculated by:

km	(104)

where the rain height, hR, can be determined from equations (74) or (75) in Annex 2 and s is the minimum elevation angle of the transmitting earth station.

The maximum calculation distance, demax, to be used in the determination of the propagation mode (2) contour, for the case of a coordinating earth station operating in bidirectionally allocated frequency bands, is dependent on the rain height. It is the greater distance determined from:

km             or

where the minimum coordination distance, dmin, is given in § 4.2 of the main body of this Appendix.

The point, at the distance ds from the earth station, on the azimuth αs of the coordinating earth station’s main beam axis, is the geographic point immediately below the main beam axis intersection with the rain height, and is the reference point from which the maximum calculation distance demax is determined (see Fig. 9).

If the maximum calculation distance, demax, is greater than the minimum coordination distance, dmin, then calculate the maximum latitude at which a receiving earth station may operate with a geostationary satellite with a minimum elevation angle min:

(105)

where:

min:	given in Table 9

K:	ratio of the radius of the satellite orbit to the radius of the Earth, equal to 6.62.

If the coordinating earth station latitude in the northern hemisphere is greater than max, or if the coordinating earth station latitude in the southern hemisphere is less than −max or −71, then the rain scatter contour is a circle of radius dmin, centred on the transmitting earth station.

For all other cases, the coordination area is developed by the following procedure:

Step 1:	The unknown receiving earth station is assumed to be operating with a satellite at the minimum elevation angle min. It is also assumed that the receiving earth station is relatively close to the coordinating earth station in geometric terms and hence a plane geometry approximation can be applied within the coordination area. If the receiving earth station’s main beam axis passes through the intersection of the coordinating earth station’s main beam axis with the rain height, the azimuths from the point on the ground immediately below this intersection to the possible locations of a receiving earth station are given by:

and

αw2 = 360° – αw1

where  is the latitude of the transmitting earth station.

Step 2:	Mark on a map of an appropriate scale the coordinating earth station’s location and draw from this location a line of distance, ds, along the azimuth, αs, to the point below the coordinating earth station’s main beam axis intersection with the rain height.

Step 3:	From the main beam axis intersection point in Step 2, mark on the map the distance, demax, along the two azimuths, αw2 and αw1, and on each azimuth at the distance, demax, draw two equal distance arcs of width 3° clockwise and counter-clockwise. The two arcs, each having a total width of 6°, are the first boundary elements of the bidirectional rain scatter area.

Step 4:	Mark a circle of radius equal to the minimum coordination distance, dmin, around the coordinating earth station’s location, and then draw straight lines from the northern edges of the two arc segments tangential to the northern rim of the circle, and from the southern edges of the two arc segments tangential to the southern rim of the circle.

The area bounded by the two 6° wide arcs, the four straight lines, and the circular sections (of which there is always at least one) between the two northern and the two southern tangent points with the straight lines, constitutes the bidirectional rain scatter area.

Figure 9 illustrates the construction of the bidirectional rain scatter area for a coordinating earth station. (The resulting rain scatter area contains the possible loci of all receiving earth station locations from which a beam path towards the geostationary-satellite orbit will intersect the main beam of the transmitting earth station antenna.)

ANNEX 6

Supplementary and auxiliary contours

# 1	Introduction

The material found in this Annex is intended to assist administrations in bilateral discussions.

# 2	Supplementary contours

The coordination area is determined with respect to the type of terrestrial station (or, in a frequency band with a bidirectional space allocation, an earth station operating in the opposite direction of transmission) that would yield the largest coordination distances. Therefore, in the case of terrestrial services, fixed stations using tropospheric scatter have been assumed to be operating in frequency bands that may typically be used by such radiocommunication systems; and fixed stations operating in line-of-sight configurations and using analogue modulation have been assumed to be operating in other frequency bands. However, other radiocommunication systems (e.g. other terrestrial stations), that typically have lower antenna gains, or otherwise less stringent system parameters, than those on which the coordination area is based, may also operate in the same frequency range. Therefore, it is possible for the administration seeking coordination to identify a supplementary contour using either the methods in § 2 or 3 of the main body of this Appendix, where they are applicable, or other agreed methods. Subject to bilateral agreement between administrations, these supplementary contours can assume the role of the coordination contour for an alternative type of radio system in the same service or another radiocommunication service.

When a supplementary contour is to be developed for other types of systems, for example digital fixed systems, the necessary system parameters may be found in one of the adjacent columns in Tables 7, 8 and 9. If no suitable system parameters are available then the value of the permissible interference power (Pr(p )) may be calculated using equation (127) of § 2 in Annex 7.

In addition, supplementary contours may be prepared by the administration seeking coordination in order to define smaller areas, based on more detailed methods, for consideration when agreed bilaterally between the concerned administrations. These contours can be a useful aid for the rapid exclusion of terrestrial stations or earth stations from further consideration. For earth stations operating with non-geostationary space stations, supplementary contours may be generated using the method in § 4 of this Annex.

Supplementary contours may comprise propagation mode (1) interference paths and, depending on the sharing scenario, propagation mode (2) interference paths. In addition, the propagation mode (1) element of a supplementary contour may, if appropriate for the radiocommunication service, utilize the same level of correction factor (see § 4.4 of the main body of this Appendix) that was applied in the determination of the coordination contour. However, all parts of each supplementary contour must fall on or between the contour defined by the minimum coordination distance and the corresponding propagation mode (1) or propagation mode (2) main contour.

# 3	Auxiliary contours

Practical experience has shown that, in many cases, the separation distance required for the coordinating earth station, on any azimuth, can in fact be substantially less than the coordination distance, since the worst-case assumptions do not apply to every terrestrial station or earth station. There are two main mechanisms that contribute to such a difference between the separation distance and the coordination distance:

–	the terrestrial station antenna gain (or e.i.r.p.), or receiving earth station antenna gain, in the direction of the coordinating earth station is less than that assumed in calculating the coordination contour;

–	appropriate allowance can be made, for example, for the effects of site shielding not included in the coordination distance calculations.

Auxiliary contours must use the same method as that used to determine the corresponding main or supplementary contour. In addition, all parts of each auxiliary contour must fall on or between the contour defined by the minimum coordination distance and the corresponding main or supplementary contour. Auxiliary contours may assist in eliminating from detailed coordination terrestrial stations or earth stations that are located in the coordination area and hence have been identified as potentially affected by the coordinating earth station. Any terrestrial station or earth station that lies outside an auxiliary contour and has an antenna gain towards the coordinating earth station that is less than the gain represented by the relevant auxiliary contour need not be considered further as a significant source, or subject, of interference.

## 3.1	Auxiliary contours for propagation mode (1)

Propagation mode (1) auxiliary contours are calculated with values for the propagation mode (1) minimum required loss in equation (22) in § 4.4 of the main body of this Appendix that are progressively reduced by, for example, 5, 10, 15, 20 dB, etc., below the value derived from the parameters assumed in Tables 7, 8 and 9 for the corresponding main or supplementary propagation mode (1) contour, until the minimum coordination distance is reached. Propagation mode (1) auxiliary contour distances are calculated without the correction factor (see § 4.4 of the main body of this Appendix), and hence could be larger, on any azimuth, than the corresponding main, or supplementary, propagation mode (1) distance. To prevent this, in those cases where a correction factor applies to the main or supplementary contour, the maximum propagation mode (1) auxiliary contour distance on any azimuth is limited to the corresponding main or supplementary propagation mode (1) distance. In effect this means that the correction factor will limit the possible range of auxiliary contour values so that only those auxiliary contours with values greater than the applied correction factor will be shown within the main or supplementary contour (see Fig. 10). For example, if the value of correction factor applicable to the propagation mode (1) main or supplementary contour is 10 dB, then the first auxiliary contour drawn would be for a reduction in minimum required loss of 5 dB and hence the auxiliary contour value would be −15 dB (by convention, auxiliary contours are shown as negative quantities as they represent a reduction in the terrestrial, or receiving earth station, antenna gain, or the terrestrial station e.i.r.p.).

Propagation mode (2) interference effects may still need to be considered even if propagation mode (1) interference effects have been eliminated from detailed coordination, as the propagation models are based on different interference mechanisms.

## 3.2	Auxiliary contours for propagation mode (2)

The propagation mode (2) contour around an earth station is calculated assuming the main beams of the coordinating earth station and the terrestrial station intersect exactly (see § 1.3 of the main body of this Appendix). However, it is unlikely that these antenna main beams will intersect exactly. It is therefore possible to generate propagation mode (2) auxiliary contours that take account of any offset in the pointing of the terrestrial station antenna beam from the direction of the coordinating earth station. This offset would result in partial beam intersections and hence a reduced interference potential. These propagation mode (2) auxiliary contours are calculated according to the method described in § 3.2.1 of this Annex.

Propagation mode (2) auxiliary contours are not generated for different values of antenna gain or e.i.r.p. but for different values of beam avoidance angle. Hence, if there is a need to consider both a lower value of antenna gain, or e.i.r.p., for the terrestrial station and propagation mode (2) auxiliary contours, it is first essential to consider the impact of the reduction in antenna gain, or e.i.r.p., on the propagation mode (2) contour. This is achieved by generating a supplementary contour (see § 2) corresponding to the lower value of antenna gain or e.i.r.p. for the terrestrial station, which is drawn on a separate map. Auxiliary mode (2) contours can then be generated inside this propagation mode (2) supplementary contour for different values of the beam avoidance angle. Hence, propagation mode (2) auxiliary contours may be most frequently applied in conjunction with a supplementary contour rather than with the coordination contour.

The correction factor discussed in § 1.3 of the main body of this Appendix does not apply to propagation mode (2) interference paths and hence is also not applicable to propagation mode (2) auxiliary contours. In addition propagation mode (2) auxiliary contours cannot be developed for the bidirectional case.

Propagation mode (2) auxiliary contours are prepared for appropriate values of terrestrial station main beam avoidance angle (see Fig. 11). When the antenna characteristics of the terrestrial stations are known, the appropriate antenna pattern9 should be used when determining the propagation mode (2) auxiliary contours. If this is not available, the reference antenna pattern given in § 3.2.3 may be used.

### 3.2.1	Determination of auxiliary contours for propagation mode (2)

Propagation mode (2) auxiliary contours allow the azimuthal offset of a terrestrial station antenna beam from the coordinating earth station’s location to be taken into consideration. Figure 12 shows the hydrometeor scatter region projected on to the horizontal plane. In this Figure, the earth station and the terrestrial station are located at the points A and B, respectively, where the terrestrial station is on a radial defined by the angle  from the point C at the centre of the propagation mode (2) main, or supplementary, contour. Point C is also the centre of the auxiliary contour.

The shaded area in Fig. 12 represents the critical region, along the earth station’s main beam axis, between the earth station and the rain height. Within this critical region a common volume can be formed between the earth station beam and the beam of any terrestrial stations within the propagation mode (2) main, or supplementary, contour. This critical region’s length is b and its maximum horizontal extent is at point M. Intersection of this critical region by the terrestrial station main beam axis would result in significant hydrometeor scatter interference via main lobe-to-main lobe coupling.

For a given point within the propagation mode (2) main, or supplementary, contour, the angle subtended by the critical region is termed the critical angle, ψ. The protection angle, , represents the angle of the terrestrial station main beam axis away from the critical region. The beam avoidance angle between the terrestrial station’s main beam axis and the earth station’s location is φ. It is the sum of the two angles ψ and  and it is this quantity that has a fixed value for a specific auxiliary contour. Each auxiliary contour is generated by varying the angle, , and deriving the distance, rb, from point C to the auxiliary contour. As the angle  increases from 0° to 360°, the angles ψ and  change, but their sum remains the same.

The algorithm in § 3.2.2 of this Annex can be used to calculate the auxiliary propagation mode (2) contour for a given value of beam avoidance angle .

The method is based on iteratively decrementing the distance, rb, between terrestrial station and the centre of the common volume, and starting at the main contour distance dr, until either the shortest value of rb is found for which the required minimum loss is achieved, or the minimum coordination distance is reached. For each value of rb, the critical angle  is determined and then the protection angle  is calculated. The terrestrial station antenna gain corresponding to  and the current distance rb are used to obtain the propagation mode (2) path loss in equation (82) in Annex 2.

The above process is repeated for each angle , to generate a complete auxiliary contour for a given value of beam avoidance angle . For some combinations of beam avoidance angle and angle , an auxiliary contour may coincide with the main, or supplementary, propagation mode (2) contour.

### 3.2.2	The step-by-step algorithm

Auxiliary propagation mode (2) contours are constructed by calculating distances along radials from the centre of the circular mode (2) main, or supplementary, contour, which is the point C, at the distance b/2 from the earth station along the azimuth of its main beam axis. The distance b/2 is equal to d, where d is given by equation (83) in Annex 2.

For the selected value of beam avoidance angle, , generate the auxiliary contour for values of angle, , ranging from 0° to 180° in steps of 1°, as follows:

a)	Set rb to the main, or supplementary, mode (2) contour distance dr calculated as described in § 3.1 of Annex 2.

b)	Compute ψ from:

(106)

(107)

ψ = ψ1 + ψ2	(108)

c)	If  >  then the auxiliary mode (2) contour coincides with the main or supplementary mode (2) contour for the current value of , and the calculation for that value of  is completed, and go to step j). Otherwise proceed through the following steps d) to i) until one of the terminating conditions described in step f) and step i) is satisfied.

d)	Decrement rb by subtracting 0.2 km from its value.

e)	Recalculate the critical angle  using equations (106), (107) and (108).

f)	If (0.5 b sin /sin 2) < dmin, the auxiliary mode (2) contour coincides with the minimum coordination distance dmin and the calculation for the current value of  is completed – go to step j). Otherwise, proceed to step g).

g)	Compute the protection angle  = φ − ψ.

h)	Calculate G(), the terrestrial station antenna gain at the angle  relative to the beam axis, using the reference antenna pattern given in this Annex.

i)	In equation (82) in Annex 2, use the gain calculated in step h) in place of Gx and the value considered of rb in place of ri, and calculate the corresponding propagation mode (2) path loss Lr. If Lr < L(p ), then increment rb by adding 0.2 km to its value and take this as the distance for the current radial. Otherwise, repeat from step d).

j)	Once the value of rb has been found for the current value of angle , calculate the angle θd from the location of the earth station, and if appropriate the distance, d, to that contour point using:

d = 0.5 b sin ω / sin ψ2	(109)

θd = ω – ψ2	(110)

An auxiliary propagation mode (2) contour is symmetrical about the earth station main beam axis. Thus, values of d and d corresponding to the values of  from 181° to 359° can be found by noting that results for a given value of  are the same as for (−) or (360 − ).

The step size for incrementing rb used above, 0.2 km, is suitable for most situations. It controls the granularity of the result when viewed as a set of rb values. For low values of earth station beam elevation, the granularity becomes more noticeable in the values of d and θd, and a smaller step size may be used.

### 3.2.3	Reference radiation patterns for line-of-sight radio-relay system antennas

The reference radiation pattern for line-of-sight radio-relay system antennas in this section is used for the unknown terrestrial station antenna in the propagation mode (2) auxiliary contour calculations when the actual antenna pattern is not available.

a)	In cases where the ratio between the antenna diameter and the wavelength is greater than 100, the following equation is used:

for	0	<  φ  <  φm	(111)

for	φm	≤  φ  <  φr	(112)

for	φr	≤  φ  <  48°	(113)

for	48°	≤  φ  ≤  180°	(114)

(115)

(116)

(117)

b)	In cases where the ratio between the antenna diameter and the wavelength is less than or equal to 100, the following equation is used:

for	0	<  φ  <  φm	(118)

for	φm	≤  φ  <  100 	(119)

for	100 	≤  φ  <  48°	(120)

for	48°	≤  φ  ≤  180°	(121)

c)	In cases where only the maximum antenna gain is known, D/λ can be estimated from the following expression:

(122)

where:

Gamax:	main beam axis antenna gain (dBi)

D:	antenna diameter (m)

:	wavelength (m)

G1:	gain of the first side lobe (dBi).

# 4	Determination of a supplementary contour using the time-variant gain (TVG) method

The TVG method requires the cumulative distribution of the time-varying horizon antenna gain of an earth station operating with a non-geostationary space station. In comparison to the TIG method, the TVG method usually produces smaller distances, but requires greater effort in determining the cumulative distribution of the horizon gain of the earth station antenna for each azimuth to be considered.

The TVG method closely approximates the convolution of the distribution of the horizon gain of the earth station antenna and the propagation mode (1) path loss. This method may produce slightly smaller distances than those obtained by an ideal convolution. An ideal convolution cannot be implemented due to the limitations of the current model for propagation mode (1). The propagation mode (1) required distance, at the azimuth under consideration, is taken as the largest distance developed from a set of calculations, each of which is based on equation (4) of the main body of this Appendix. For convenience, in these calculations, this equation may be rewritten for the n-th calculation in the following form:

dB	(123)

with the constraint:

where:

Pt, Pr(p):	as defined in equations in § 1.3 of the main body of this Appendix where p is the percentage of time associated with permissible interference power Pr(p)

Gx:	maximum antenna gain assumed for the terrestrial station (dBi). Tables 7 and 8 give values for Gx for the various frequency bands

Ge(pn):	the horizon gain of the coordinating earth station antenna (dBi) that is exceeded for pn% of the time on the azimuth under consideration

Lb(pv): 	the propagation mode (1) minimum required loss (dB) for pv% of the time; this loss must be exceeded by the propagation mode (1) predicted path loss for all but pv% of the time.

The values of the percentages of time, pn, to be used in equation (123) are determined in the context of the cumulative distribution of the horizon antenna gain. This distribution needs to be developed for a predetermined set of values of horizon antenna gain spanning the range from the minimum to the maximum values for the azimuth under consideration. The notation Ge(pn) denotes the value of horizon antenna gain for which the complement of the cumulative distribution of the horizon antenna gain has the value corresponding to the percentage of time pn. The pn value is the percentage of time that the horizon antenna gain exceeds the n-th horizon antenna gain value. The procedure in § 4.1 may be used to develop this distribution.

For each value of pn, the value of horizon antenna gain for this time percentage, Ge(pn), is used in equation (123) to determine a propagation mode (1) minimum required loss. The propagation mode (1) predicted path loss is to exceed this propagation mode (1) required loss for no more than pv% of the time, as specified by the constraint associated with equation (123). A series of propagation mode (1) distances are then determined using the procedures described in § 4 of the main body of this Appendix.

The propagation mode (1) required distance is then the maximum distance in the series of propagation mode (1) distances that are obtained for any value of pn, subject to the constraint associated with equation (123). A detailed description of the method for using equation (123) to determine the propagation mode (1) required distance is provided in § 4.2.

Further information, including examples, may be found in the most recent version of Recommendation ITU-R SM.1448.    (WRC-15)

## 4.1	Determination of the horizon antenna gain distribution for the TVG method

The TVG method for the determination of an earth station’s supplementary contour requires the determination of the horizon antenna gain statistics for all azimuths (in suitable increments, e.g. 5°) around the earth station. In considering the horizon antenna gain of the antenna for either a transmitting or a receiving earth station, only the horizon antenna gain values during the operational time are to be considered. In developing the cumulative distributions of horizon antenna gain, the percentages of time are percentages of operational time. Thus, there may be periods of time for which no horizon antenna gain is specified.

The determination of the horizon antenna gain distribution requires both earth station and orbital information including whether or not station keeping is used to maintain a single orbital path (repeating/non-repeating ground track system). The cumulative distribution of the time-varying horizon gain of a transmitting or a receiving earth station antenna operating with non-geostationary space stations is calculated as follows:

Step 1:	Simulate the constellation of non-geostationary space stations over a sufficiently long period, with a time step appropriate for orbit altitude, to obtain a valid representation of the antenna gain variations. For repeating ground track constellations, simulate the orbital path for each satellite visible from the earth station over a period of the ground track. For non-repeating ground track constellations, simulate the orbit of each satellite in the constellation over a period long enough to get a stable representation of the distribution.

Step 2:	At each time step, determine the azimuth and elevation angle of each satellite that is both visible at the earth station and above the minimum elevation angle at which the earth station operates. In addition to the minimum elevation angle, other criteria could be used to avoid certain geometric configurations, e.g. geostationary orbit arc avoidance (no transmission between an earth station and a non-geostationary satellite that is within X° from the geostationary orbit arc).

Step 3:	At each step, and for each satellite in communication with the earth station, use the actual earth station antenna pattern, or a formula giving a good approximation of it, to calculate the gain towards the horizon at each azimuth and elevation angle around the earth station.

Step 4:	Choose a gain increment g (dB) and partition the gain range by a number of gain levels between Gmin and Gmax, i.e. G = {Gmin, Gmin + g, Gmin + 2g,..., Gmax}.

These gain levels determine a set of gain intervals so that the n-th gain interval (n = 1, 2, 3, …) includes gain values equal to, or greater than, Gmin + (n – 2)g and less than Gmin + (n – 1)g.

A value of g = 0.1 to 0.5 dB is recommended.

For each azimuth on the horizon around the earth station, accumulate the time that the horizon gain takes a value in each gain interval of width g (dB).

Step 5:	The probability density function (pdf) on each azimuth is determined by dividing the time in each gain interval by the total simulation time.

Step 6:	Determine the cumulative distribution function (cdf) of horizon antenna gain at each azimuth by accumulating the gain density function at that azimuth. The value of the required cdf at any specific gain value is the percentage of time that the gain is less than, or equal to, that gain value.

## 4.2	Determination of the supplementary contour distance using the TVG method

This calculation is based on a cumulative distribution of the horizon gain of the earth station antenna for each azimuth to be considered (in suitable angular increments e.g. 5°). Appropriate distributions for this purpose may be developed by the method in § 4.1. The process for calculating the supplementary contour distance for each azimuth is described in the following procedure.

Step 1:	From the complementary cumulative distribution of the horizon antenna gain, for the azimuth under consideration, determine the percentage of time pn that the horizon gain exceeds the level Gen, where:

Gen = Gmin + (n – 1)g                 (n = 1, 2, 3,...)	(124)

with:

Gmin:	minimum value of horizon gain, and

g:	gain increment.

Step 2:	For each percentage pn that is equal to or greater than 2p%, the percentage of time to be used in determining the propagation mode (1) path loss is pν.

pv = 100 p / pn            %                  for  pn  ≥  2p%	(125)

For each percentage of time, determine the distance, dn (km), for which the propagation mode (1) predicted path loss is equal to the propagation mode (1) minimum required loss, using the propagation model in accordance with § 4 of the main body of this Appendix and the equation:

Lbn (pv) = Pt + Gen + Gx – Pr(p)                    dB	(126)

The values of pν must be within the range of percentage of time of the propagation mode (1) model (see § 1.5.1 of the main body of this Appendix).

Step 3:	The propagation mode (1) required distance for the azimuth under consideration is the largest of the distances, dn (km), calculated in Step 2, except when this largest distance is attained for the smallest value of pn that is equal to or greater than 2p in accordance with equation (125) in Annex 6. In such cases, the propagation mode (1) required distance for the azimuth under consideration is the distance determined from equation (126) in Annex 6 with Gen = Gmax and pv = 50% where Gmax is the maximum value of horizon antenna gain.

Step 4:	The propagation mode (1) supplementary contour distance for the azimuth under consideration is the required distance as determined in Step 3, except that the distance must be between the minimum coordination distance, dmin, and the maximum coordination distance, dmax1. These limits are given in § 4.2 and § 4.3 of the main body of this Appendix, respectively.

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

# 1	Introduction

Tables 7 to 9 contain the system parameter values required by the methods in the main body of this Appendix to determine the coordination area around an earth station when the band is shared with terrestrial radiocommunication services or other earth stations operating in the opposite direction of transmission.

Table 7 is limited to those system parameter values required for the case of a transmitting earth station sharing with terrestrial services; Table 8 is limited to those parameter values required for the case of a receiving earth station sharing with terrestrial services; Table 9 is limited to those parameter values required for the case of a transmitting earth station which is sharing in a bidirectionally allocated band with other earth stations operating in the opposite direction of transmission.

These system parameter tables include primary allocations to the space and terrestrial services in Article 5 in all bands between 100 MHz and 105 GHz. Some of the columns have incomplete information. In some cases, this is because there is no requirement to calculate coordination distances as pre-determined coordination distances apply. In other cases, the service allocations are new and the systems may not be introduced for some years. Hence, the system parameters are the subject of ongoing development within the Radiocommunication Study Groups.

Parameters specific to the earth station, for which coordination is being sought, are provided to the Radiocommunication Bureau in the format specified in Appendix 4 as part of the notification and coordination procedures.

The row in each table entitled “method to be used” directs the user to the appropriate section of the main body of this Appendix which describes the methods to be followed for the determination of the coordination area.

Note that the earth station for which the coordination area is to be determined is identified by the service designation given in the first row of each table.

When a supplementary contour is to be developed, for example for digital fixed systems, the necessary system parameters may be found in one of the adjacent columns in Tables 7, 8 and 9. If no suitable system parameters are available, then the value of the permissible interference power (Pr(p )) may be calculated using equation (127) in § 2.

The predetermined coordination distances specified in Table 10 are used for transmitting and receiving earth stations, in cases defined by the corresponding frequency sharing situation.

# 2	Calculation of the permissible interference power of an interfering emission

Tables 7, 8 and 9 contain values for the parameters which are required for the calculation of the permissible interference power of the interfering emission (dBW), in the reference bandwidth, to be exceeded for no more than p% of the time at the receiving antenna terminal of a station subject to interference, from a single source of interference, using the general formula:

dBW	(127)

where:

k:	Boltzmann's constant (1.38 × 10−23 J/K)

Te:	thermal noise temperature of the receiving system (K), at the terminal of the receiving antenna (see § 2.1 of this Annex)

NL:	link noise contribution (see § 2.2 of this Annex)

B:	reference bandwidth (Hz), i.e. the bandwidth in the receiving station that is subject to the interference and over which the power of the interfering emission can be averaged

p:	percentage of the time during which the interference from one source may exceed the permissible interference power value; since the entries of interference are not likely to occur simultaneously, p = p0/n

p0:	percentage of the time during which the interference from all sources may exceed the threshold value

n:	number of equivalent, equal level, equal probability entries of interference, assumed to be uncorrelated for small percentages of the time

Ms:	link performance margin (dB) (see § 2.3 of this Annex)

W:	a thermal noise equivalence factor (dB) for interfering emissions in the reference bandwidth; it is positive when the interfering emissions would cause more degradation than thermal noise (see § 2.4 of this Annex).

In certain cases, an administration may have reason to believe that, for its receiving earth station, a departure from the values associated with the earth station, as listed in Table 8, may be justified. Attention is drawn to the fact that for specific systems the bandwidths B or, for example in the case of demand assignment systems, the percentages of the time p and p0 may have to be changed from the values given in Table 8.

## 2.1	Calculation of the noise temperature of the receiving system

The noise temperature (K) of the receiving system, referred to the output terminals of the receiving antenna, may be determined (unless specifically given in Table 7) from:

K	(128)

where:

Ta:	noise temperature (K) contributed by the receiving antenna

ℓt1:	numerical loss in the transmission line (e.g. a waveguide) between the antenna terminal and the receiver front end

Tr:	noise temperature (K) of the receiver front end, including all successive stages at the front end input.

For radio-relay receivers and where the waveguide loss of a receiving earth station is not known, a value of ℓt1 = 1.0 is used.

In case of determination of the coordination contours between two earth stations operating in the opposite direction of transmission, the following earth station receiving system noise temperatures should be used if the value is not provided in Table 9. This assumption is necessary because the receiving earth station takes the place of a receiving terrestrial station in the calculations.

TABLE 6

| Frequency range (GHz) | Te (K) |
| --- | --- |
| f < 10 | 75 |
| 10 < f < 17 | 150 |
| f > 17 | 300 |

## 2.2	Determination of the factor NL

The factor NL is the noise contribution to the link. In the case of a satellite transponder, it includes the uplink noise, intermodulation, etc. In the absence of table entries, it is assumed:

NL = 1 dB  	for fixed-satellite links

= 0 dB  for terrestrial links

## 2.3	Determination of the factor Ms

The factor Ms is the factor by which the link noise under clear-sky conditions would have to be raised in order to equal the permissible interference power.

## 2.4	Determination of the factor W

The factor W (dB) is the level of the radio-frequency thermal noise power relative to the received power of an interfering emission which, in the place of the former and contained in the same (reference) bandwidth, would produce the same interference (e.g. an increase in the voice or video channel noise power, or in the bit error ratio). The factor W generally depends on the characteristics of both the wanted and the interfering signals.

When the wanted signal is digital, W is usually equal to or less than 0 dB, regardless of the characteristics of the interfering signal.

# 3	Horizon antenna gain for a receiving earth station with respect to a transmitting earth station

For the determination of the coordination area of a transmitting earth station with respect to a receiving earth station in a bidirectionally allocated band, it is necessary to calculate the horizon antenna gain of the unknown earth station. In cases where the unknown receiving earth stations operate with geostationary satellites, Table 9 provides the necessary receiving earth station parameters for the calculation procedure, which is described in § 2.1 of Annex 5.

In the case where the unknown receiving earth station operates with non-geostationary satellites, the horizon antenna gain to be used for all azimuths is provided in Table 9. The tabulated values were determined by using the method described in § 2.2 of the main body of this Appendix, which uses the maximum and minimum values of horizon antenna gain. For this purpose the maximum horizon antenna gain is the gain of the antenna for an off-axis angle equal to the minimum operating elevation angle. The minimum horizon antenna gain is the gain at large off-axis angles, usually more than 36° or 48°.

In determining the TIG horizon antenna gain entries in Table 9, the difference between the maximum and minimum horizon antenna gain did not exceed 30 dB. Consequently, the TIG horizon antenna gain was taken as the lesser of the maximum horizon antenna gain or 20 dB more than the minimum horizon antenna gain. For the purpose of determining the TIG horizon antenna gain, the reference antenna pattern of § 3 of Annex 3 was used, except in cases noted in the Tables where a different pattern was deemed to be more appropriate.

TABLE 7a     (Rev.WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Mobile-satellite, space operation | Earth exploration-satellite, meteorological satellite |  | Space operation | Space research, space operation | Mobile- satellite | Space operation |  | Mobile- satellite, radio- determination- satellite | Mobile- satellite |  | Space operation, space research |  | Mobile- satellite |  | Space research, space operation, Earth exploration-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  | 148.0-149.9 | 401-403 |  | 433.75-434.25 | 449.75-450.25 | 806-840 | 1 427-1 429 |  | 1 610-1 626.5 | 1 668.4-1 675 |  | 1 750-1 850 |  | 1 980-2 025 |  | 2 025-2 110 2 110-2 120 (Deep space) |  |
| Receiving terrestrial service designations |  | Fixed, mobile | Meteorological aids |  | Amateur, radiolocation fixed, mobile | Fixed, mobile, radio- location | Fixed, mobile broadcasting, aeronautical radionavigation | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  |
| Method to be used |  | § 2.1, § 2.2 | § 2.1, § 2.2 |  | § 2.1, § 2.2 | § 2.1, § 2.2 | § 1.4.6 | § 2.1, § 2.2 |  | § 1.4.6 | § 1.4.6 |  | § 2.1, § 2.2 |  | § 1.4.6 |  | § 2.1, § 2.2 |  |
| Modulation at terrestrial station 1 |  | A | A | N |  | A and N | A and N | A | N |  | A | N | A | N | A | N | A |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 1.0 |  |  |  | 0.01 | 0.01 | 0.01 | 0.01 |  | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |  | 0.01 |  |
|  | N | 1 |  |  |  | 2 | 2 | 2 | 2 |  | 2 | 2 | 2 | 2 | 2 |  | 2 |  |
|  | p (%) | 1.0 |  |  |  | 0.005 | 0.005 | 0.005 | 0.005 |  | 0.005 | 0.005 | 0.005 | 0.005 | 0.005 |  | 0.005 |  |
|  | NL (dB) | – |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |  | 0 |  |
|  | Ms (dB) | – |  |  |  | 20 | 20 | 33 | 33 |  | 33 | 33 | 33 | 33 | 26 2 |  | 26 2 |  |
|  | W (dB) | – |  |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |  | 0 |  |
| Terrestrial station parameters | Gx (dBi) 3 | 8 |  |  |  | 16 | 16 | 33 | 33 |  | 35 | 35 | 35 | 35 | 49 2 |  | 49 2 |  |
|  | Te (K) | – |  |  |  | 750 | 750 | 750 | 750 |  | 750 | 750 | 750 | 750 | 500 2 |  | 500 2 |  |
| Reference bandwidth | B (Hz) | 4 × 103 |  |  |  | 12.5 × 103 | 12.5 × 103 | 4 × 103 | 106 |  | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 |  | 4 × 103 |  |
| Permissible interference power | Pr(p) (dBW) in B | −153 |  |  |  | −139 | −139 | −131 | −107 |  | −131 | −107 | −131 | −107 | −140 |  | −140 |  |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 1 668.4-1 675 MHz may also be used to determine a supplementary contour. (WRC-03) 3 Feeder losses are not included. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 7b    (Rev.WRC-15)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aero-nautical mobile-satellite (R) service | Aero-nautical mobile-satellite (R) service | Fixed- satellite | Fixed- satellite | Fixed- satellite | Fixed- satellite |  | Earth exploration-satellite, space operation, space research |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite | Fixed- satellite 3 | Fixed- satellite | Fixed- satellite 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 | 5.091-5.150 | 5.725-5.850 | 5.725-7.075 |  | 7.100-7.250 5 |  | 7.900-8.400 |  | 10.7-11.7 |  | 12.5-14.8 |  | 13.75-14.3 | 15.43-15.65 | 17.7-18.4 | 19.3-19.7 |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation | Aeronautical mobile (R) | Radiolocation | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Radiolocation radionavigation (land only) | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  | § 2.1 | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 |  | § 2.1 |  | § 2.1, § 2.2 |  | § 2.1 |  | § 2.1, § 2.2 | § 2.2 |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  | A | N | A | N | A | N | A | N | A | N | − |  | N | N |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 | 0.005 | 0.01 |  | 0.005 | 0.005 |
|  | n | 2 |  |  |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 |  | 2 | 2 |
|  | p (%) | 0.005 |  |  |  |  |  | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.005 | 0.0025 | 0.01 |  | 0.0025 | 0.0025 |
|  | NL (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
|  | Ms (dB) | 26 2 |  |  |  |  |  | 33 | 37 | 33 | 37 | 33 | 37 | 33 | 40 | 33 | 40 | 1 |  | 25 | 25 |
|  | W (dB) | 0 |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 | 6 | 6 |  | 46 | 46 | 46 | 46 | 46 | 46 | 50 | 50 | 52 | 52 | 36 |  | 48 | 48 |
|  | Te (K) | 500 2 |  |  |  |  |  | 750 | 750 | 750 | 750 | 750 | 750 | 1 500 | 1 100 | 1 500 | 1 100 | 2 636 |  | 1 100 | 1 100 |
| Reference bandwidth | B (Hz) | 4 × 103 | 150 × 103 | 37.5 × 103 | 150 × 103 | 106 |  | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 4 × 103 | 106 | 107 |  | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | −140 | −160 | −157 | −160 | −143 |  | −131 | −103 | −131 | −103 | −131 | −103 | −128 | −98 | −128 | −98 | −131 |  | −113 | −113 |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 5 725-7 075 MHz may also be used to determine a supplementary contour with the exception that Gx = 37 dBi. 3 Feeder links of non-geostationary satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Actual frequency bands are 7 190-7 250 MHz for the Earth exploration-satellite service, 7 100-7 155 MHz and 7 190-7 235 MHz for the space operation service and 7 145-7 235 MHz for the space research service. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 7c    (Rev.WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | Fixed- satellite 2 | Fixed- satellite 3 | Space research | Earth exploration-satellite, space research | Fixed-satellite, mobile-satellite, radionavigation-satellite | Fixed- satellite 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 | 28.6-29.1 | 29.1-29.5 | 34.2-34.7 | 40.0-40.5 | 42.5-47 47.2-50.2 50.4-51.4 | 47.2-50.2 |  |
| Receiving terrestrial service designations |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile, radiolocation | Fixed, mobile | Fixed, mobile, radionavigation | Fixed, mobile |  |
| Method to be used |  | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 | § 2.1, § 2.2 | § 2.2 |  |
| Modulation at terrestrial station 1 |  | N | N | N |  | N | N | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 | 0.005 | 0.005 |  | 0.005 | 0.005 | 0.001 |  |
|  | n | 1 | 2 | 1 |  | 1 | 1 | 1 |  |
|  | p (%) | 0.005 | 0.0025 | 0.005 |  | 0.005 | 0.005 | 0.001 |  |
|  | NL (dB) | 0 | 0 | 0 |  | 0 | 0 | 0 |  |
|  | Ms (dB) | 25 | 25 | 25 |  | 25 | 25 | 25 |  |
|  | W (dB) | 0 | 0 | 0 |  | 0 | 0 | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 | 50 | 50 |  | 42 | 42 | 46 |  |
|  | Te (K) | 2 000 | 2 000 | 2 000 |  | 2 600 | 2 600 | 2 000 |  |
| Reference bandwidth | B (Hz) | 106 | 106 | 106 |  | 106 | 106 | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | −111 | −111 | −111 |  | −110 | −110 | −111 |  |
| 1 A: analogue modulation; N: digital modulation. 2 Non-geostationary satellites in the fixed-satellite service. 3 Feeder links to non-geostationary-satellite systems in the mobile-satellite service. 4 Feeder losses are not included. |  |  |  |  |  |  |  |  |

TABLE 8a     (Rev.WRC-12)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Space operation, space research | Meteoro-logical- satellite, mobile-satellite |  | Space research | Space research, space operation | Space operation | Mobile-satellite | Meteoro-logical-satellite | Mobile-satellite | Space research | Space operation | Meteoro-logical- satellite | Broad-casting- satellite | Mobile-satellite | Broadcasting- satellite (DAB) | Mobile-satellite, land-mobile satellite, maritime mobile-satellite |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  | 137-138 | 137-138 |  | 143.6-143.65 | 174-184 | 163-167 272-273 5 | 335.4-399.9 | 400.15-401 | 400.15-401 | 400.15-401 | 401-402 | 460-470 | 620-790 | 856-890 | 1 452-1 492 | 1 518-1 530 1 555-1 559 2 160-2 200 1 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile | Fixed, mobile |  | Fixed, mobile, radio-location | Fixed, mobile, broad-casting | Fixed, mobile | Fixed, mobile | Meteoro-logical aids | Meteoro- logical aids | Meteoro-logical aids | Meteoro-logical aids, fixed, mobile | Fixed, mobile | Fixed, mobile, broad-casting | Fixed, mobile, broad casting | Fixed, mobile, broadcasting | Fixed, mobile |
| Method to be used |  |  | § 2.1 | § 2.1 |  | § 2.1 | § 2.1 | § 2.1 | § 1.4.6 | § 1.4.6 | § 1.4.6 | – | § 2.1 | § 2.1 | § 1.4.5 | § 1.4.6 | § 1.4.5 | § 1.4.6 |
| Modulation at earth station 2 |  |  | N |  |  | N |  | N |  |  |  | N | N |  |  |  | N | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.1 |  |  | 0.1 |  | 1.0 |  | 0.012 |  | 0.1 | 0.1 | 0.012 |  |  |  | 10 |
|  | n |  | 2 |  |  | 2 |  | 1 |  | 1 |  | 2 | 2 | 1 |  |  |  | 1 |
|  | p (%) |  | 0.05 |  |  | 0.05 |  | 1.0 |  | 0.012 |  | 0.05 | 0.05 | 0.012 |  |  |  | 10 |
|  | NL (dB) |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  |  |  | 0 |
|  | Ms (dB) |  | 1 |  |  | 1 |  | 1 |  | 4.3 |  | 1 | 1 |  |  |  |  | 1 |
|  | W (dB) |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  |  |  | 0 |
| Terrestrial station parameters | E (dBW) in B 3 | A | – |  |  | – |  | 15 |  |  |  | – | – | 5 |  |  | 38 | 37 4 |
|  |  | N | – |  |  | – |  | 15 |  |  |  | – | – | 5 |  |  | 38 | 37 |
|  | Pt (dBW) in B | A | – |  |  | – |  | –1 |  |  |  | – | – | –11 |  |  | 3 | 0 |
|  |  | N | – |  |  | – |  | –1 |  |  |  | – | – | –11 |  |  | 3 | 0 |
|  | Gx (dBi) |  | – |  |  | – |  | 16 |  |  |  | – | – | 16 |  |  | 35 | 37 |
| Reference bandwidth | B (Hz) |  | 1 |  |  | 1 |  | 103 |  | 177.5 × 103 |  | 1 | 1 | 85 |  |  | 25 × 103 | 4 × 103 |
| Permissible interference power | Pr( p) (dBW) in B |  | −199 |  |  | −199 |  | −173 |  | −148 |  | −208 | −208 | −178 |  |  |  | −176 |
| 1 In the band 2 160-2 200 MHz, the terrestrial station parameters of line-of-sight radio-relay systems have been used. If an administration believes that, in this band transhorizon systems need to be considered, the parameters associated with the frequency band 2 500-2 690 MHz may be used to determine the coordination area. 2 A: analogue modulation; N: digital modulation. 3 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 4 This value is reduced from the nominal value of 50 dBW for the purposes of determination of coordination area, recognizing the low probability of high power emissions falling fully within the relatively narrow bandwidth of the earth station. 5 The fixed-service parameters provided in the column for 163-167 MHz and 272-273 MHz are only applicable to the band 163-167 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 8b    (Rev.WRC-12)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  |  | Space operation (GSO and non-GSO) | Meteoro-logical- satellite (non-GSO) | Meteoro-logical- satellite (GSO) | Space research near-Earth (non-GSO and GSO) |  | Space research deep space (non-GSO) | Space operation (non-GSO and GSO) | Earth exploration- satellite (GSO) | Broadcasting- satellite | Mobile-satellite, radio- determination- satellite | Fixed-satellite, broadcasting satellite |  | Fixed-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Unmanned | Manned |  |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  |  | 1.525-1.535 | 1.670-1.710 | 1.670-1.710 | 1.700-1.710 2.200-2.290 |  | 2.290-2.300 | 2.200-2.290 | 2.200-2.290 | 2.310-2.360 | 2.4835-2.500 6 | 2.500-2.690 |  | 3.400-4.200 |  |
| Transmitting terrestrial service designations |  |  |  | Fixed | Fixed, mobile, meteoro-logical aids | Fixed, mobile, meteoro- logical aids | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile, radiolocation | Fixed, mobile, radiolocation | Fixed, mobile radiolocation |  | Fixed, mobile |  |
| Method to be used |  |  |  | § 2.1, § 2.2 | § 2.2 and 1 | § 2.1 and 1 | § 2.1, § 2.2 |  | § 2.2 | § 2.1, § 2.2 | § 2.1 | § 1.4.5 | § 1.4.6 | § 1.4.5 and § 2.1 |  | § 2.1 |  |
| Modulation at earth station 2 |  |  |  | N | N | N | N |  | N | N | N |  | N | A | N | A | N |
| Earth station interference parameters and criteria |  | p0 (%) |  | 1.0 | 0.006 | 0.011 | 0.1 | 0.001 | 0.001 | 1.0 | 1.0 |  | 10 | 0.03 | 0.003 | 0.03 | 0.005 |
|  |  | n |  | 1 | 3 | 2 | 2 | 1 | 1 | 2 | 2 |  | 1 | 3 | 3 | 3 | 3 |
|  |  | p (%) |  | 1.0 | 0.002 | 0.0055 | 0.05 | 0.001 | 0.001 | 0.5 | 0.5 |  | 10 | 0.01 | 0.001 | 0.01 | 0.0017 |
|  |  | NL (dB) |  | 0 | 0 | 0 | 0 |  | 0 | 0 |  |  | 0 | 1 | 1 | 1 | 1 |
|  |  | Ms (dB) |  | 1 | 2.8 | 0.9 | 1 |  | 0.5 | 1 |  |  | 1 | 7 | 2 | 7 | 2 |
|  |  | W (dB) |  | 0 | 0 | 0 | 0 |  | 0 | 0 |  |  | 0 | 4 | 0 | 4 | 0 |
| Terrestrial station parameters |  | E (dBW) in B 3 | A | 50 | 92 4 | 92 4 | −27 4, 5 |  | −27 5 | 72 | 72 4 |  | 37 | 72 4 | 72 4 | 55 | 55 |
|  |  |  | N | 37 | – | – | –27 |  | −27 | 76 | 76 |  | 37 | 76 | 76 | 42 | 42 |
|  |  | Pt (dBW) in B | A | 13 | 40 4 | 40 4 | −71 4, 5 |  | −71 5 | 28 | 28 4 |  | 0 | 28 4 | 28 4 | 13 | 13 |
|  |  |  | N | 0 | – | – | −71 |  | −71 | 32 | 32 |  | 0 | 32 | 32 | 0 | 0 |
|  |  | Gx (dBi) |  | 37 | 52 | 52 | 44 |  | 44 | 44 | 44 |  | 37 | 44 | 44 | 42 | 42 |
| Reference bandwidth |  | B (Hz) |  | 103 | 106 | 4 × 103 | 1 |  | 1 | 106 | 106 |  | 4 × 103 | 106 | 106 | 106 | 106 |
| Permissible interference power |  | Pr( p) (dBW) in B |  | −184 | −142 | −177 | −216 |  | −222 | −154 | −154 |  | −176 |  |  |  |  |
| 1 See Table 10. 2 A: analogue modulation; N: digital modulation. 3 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 4 In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area, with the exception that E = 50 dBW for analogue terrestrial stations; and Gx = 37 dBi. However, for the space research service only, noting footnote 5 when transhorizon systems are not considered, E = 20 dBW and Pt = −17 dBW for analogue terrestrial stations, E = −23 dBW and Pt = −60 dBW for digital terrestrial stations; and Gx = 37 dBi. 5 These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission. 6 In the band 2.4835-2.5 GHz the terrestrial station parameters of line-of-sight radio-relay systems have been used. If an administration believes that, in this band, transhorizon systems need to be considered, the parameters associated with the frequency band 2 500-2 690 MHz may be used to determine the coordination area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 8c    (Rev.WRC-15)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteoro- logical-satellite 7, 8 | Meteoro- logical-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 13.4-13.65 7 |  | 12.5-12.75 12 |  | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | −18 | −18 | 43 | 43 | 42 | 42 | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | −17 5 | −17 5 | −5 | −5 | 10 | 10 |  | −10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −60 | −60 | −2 | −2 | −3 | −3 | −7 | −5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27 × 106 | 27 × 106 |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | −151.2 |  |  | −125 | −125 | −154 11 | −142 | −220 | −216 |  |  | −131 | −131 |  |  |

Notes to Table 8c:

1	A: analogue modulation; N: digital modulation.

2	E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth.

3	In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area.

4	Digital systems assumed to be non-transhorizon. Therefore Gx = 42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used.

5	These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission.

6	In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station.

7	Geostationary-satellite systems.

8	Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters.

9	Non-geostationary satellite systems.

10	Space research earth stations in the frequency band 8.4-8.5 GHz operate with non-geostationary satellites.

11	For large earth stations:		Pr(p) = (G − 180) 		dBW

For small earth stations:		Pr(20%) = 2 (G − 26) − 140	dBW 	for  26 < G  29 dBi

Pr(20%) = G − 163		dBW 	for          G  29 dBi

Pr(p)% = G − 163 		dBW 	for          G  26 dBi

12	Applies to the broadcasting-satellite service in unplanned bands in Region 3.

TABLE 8d     (Rev.WRC-12)

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Meteoro-logical- satellite | Fixed-satellite | Fixed-satellite 3 | Broad-casting-satellite | Earth exploration-satellite 4 | Earth exploration-satellite 5 | Space research (deep space) | Space research |  | Fixed-satellite 6 | Fixed- satellite 5 | Mobile-satellite | Broadcasting-satellite, fixed-satellite | Mobile-satellite | Radio-navigation-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Unmanned | Manned |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 18.0-18.4 | 18.8-19.3 | 19.3-19.7 | 21.4-22.0 | 25.5-27.0 | 25.5-27.0 | 31.8-32.3 | 37.0-38.0 |  | 37.5-40.5 | 37.5-40.5 | 39.5-40.5 | 40.5-42.5 | 43.5-47.0 | 43.5-47.0 |  |
| Transmitting terrestrial service designations |  |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, radio- navigation | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Broadcasting, fixed | Mobile | Mobile |  |
| Method to be used |  |  | § 2.1 | § 2.1, § 2.2 | § 2.2 | § 1.4.5 | § 2.2 | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  | § 2.2 | § 2.1 | § 1.4.6 | § 1.4.5, § 2.1 | § 1.4.6 | – |  |
| Modulation at earth station 1 |  |  | N | N | N |  | N | N | N | N |  | N | N | N | – | N |  |  |
| Earth station interference parameters and criteria | p0 (%) |  | 0.05 | 0.003 | 0.01 |  | 0.25 | 0.25 | 0.001 | 0.1 | 0.001 | 0.02 | 0.003 |  |  |  |  |  |
|  | n |  | 2 | 2 | 1 |  | 2 | 2 | 1 | 1 | 1 |  | 2 |  |  |  |  |  |
|  | p (%) |  | 0.025 | 0.0015 | 0.01 |  | 0.125 | 0.125 | 0.001 | 0.1 | 0.001 |  | 0.0015 |  |  |  |  |  |
|  | NL (dB) |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |  | 1 | 1 |  |  |  |  |  |
|  | Ms (dB) |  | 18.8 | 5 | 5 |  | 11.4 | 14 | 1 | 1 |  | 6.8 | 6 |  |  |  |  |  |
|  | W (dB) |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |  | 0 | 0 |  |  |  |  |  |
| Terrestrial station parameters | E (dBW) in B 2 | A |  | – | – |  | – | – | – | – |  | – | – | – | – |  |  |  |
|  |  | N | 40 | 40 | 40 | 40 | 42 | 42 | −28 | −28 |  | 35 | 35 | 35 | 44 | 40 | 40 |  |
|  | Pt (dBW) in B | A |  | – | – |  | – | – | – | – |  | – | – | – | – |  |  |  |
|  |  | N | −7 | −7 | −7 | −7 | −3 | −3 | −81 | −73 |  | −10 | −10 | −10 | −1 | −7 | −7 |  |
|  | Gx (dBi) |  | 47 | 47 | 47 | 47 | 45 | 45 | 53 | 45 |  | 45 | 45 | 45 | 45 | 47 | 47 |  |
| Reference bandwidth 6 | B (Hz) |  | 107 | 106 | 106 |  | 107 | 107 | 1 | 1 |  | 106 | 106 | 106 | 106 |  |  |  |
| Permissible interference power | Pr ( p) (dBW) in B |  | −115 | −140 | −137 |  | −120 | −116 | −216 | −217 |  | −140 |  |  |  |  |  |  |
| 1 A: analogue modulation; N: digital modulation. 2 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 3 Non-geostationary mobile-satellite service feeder links. 4 Non-geostationary-satellite systems. 5 Geostationary-satellite systems. 6 Non-geostationary fixed-satellite service systems. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 9a    (Rev.WRC-15)

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | Mobile-satellite | Earth exploration-satellite, meteorological-satellite | Mobile-satellite |  | Fixed-satellite, mobile-satellite | Aeronautical mobile-satellite (R) service |  | Fixed- satellite 3 |  | Fixed-satellite | Fixed-satellite, meteorological-satellite | Fixed-satellite |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 0.272-0.273 | 0.401-0.402 | 1.670-1.675 |  | 2.655-2.690 | 5.030-5.091 |  | 5.150-5.216 |  | 6.700-7.075 | 8.025-8.400 | 8.025-8.400 |
| Space service designation in which the receiving earth station operates |  | Space operation | Space operation | Meteorological-satellite |  | Fixed-satellite, broadcasting-satellite | Aeronautical mobile-satellite (R) service |  | Fixed-satellite | Radiodetermination-satellite | Fixed-satellite | Earth exploration- satellite | Earth exploration- satellite |
| Orbit 6 |  | Non-GSO | Non-GSO | Non-GSO | GSO |  | Non-GSO | GSO | Non-GSO |  | Non-GSO | Non-GSO | GSO |
| Modulation at receiving earth station 1 |  | N | N | N | N |  |  |  |  |  | N | N | N |
| Receiving earth station interference parameters and criteria | p0 (%) | 1.0 | 0.1 | 0.006 | 0.011 |  |  |  |  |  | 0.005 | 0.011 | 0.083 |
|  | n | 1 | 2 | 3 | 2 |  |  |  |  |  | 3 | 2 | 2 |
|  | p (%) | 1.0 | 0.05 | 0.002 | 0.0055 |  |  |  |  |  | 0.0017 | 0.0055 | 0.0415 |
|  | NL (dB) | 0 | 0 | 0 | 0 |  |  |  |  |  | 1 | 0 | 1 |
|  | Ms (dB) | 1 | 1 | 2.8 | 0.9 | 2 |  |  | 2 | 2 | 2 | 4.7 | 2 |
|  | W (dB) | 0 | 0 | 0 | 0 |  |  |  |  |  | 0 | 0 | 0 |
| Receiving earth station parameters | Gm (dBi) 2 | 20 | 20 | 30 | 45 |  | 45 | 45 | 48.5 |  | 50.7 |  |  |
|  | Gr (dBi) 4 | 19 | 19 | 19 9 | 8 |  | 8 | 8 | 10 |  | 10 | 10 | 8 |
|  | εmin 5 | 10° | 10° | 5° | 3° | 3° | 10° | 10° | 3° | 3° | 3° | 5° | 3° |
|  | Te (K) 7 | 500 | 500 | 370 | 118 | 75 | 340 | 340 | 75 | 75 | 75 |  |  |
| Reference bandwidth | B (Hz) | 103 | 1 | 106 | 4 × 103 |  | 37.5 × 103 | 37.5 × 103 |  |  | 106 | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | −177 | −208 | −145 | −178 |  | −163.5 | −163.5 |  |  | −151 | −142 | −154 |

Notes to Table 9a:

1	A: analogue modulation; N: digital modulation.

2	On-axis gain of the receive earth station antenna.

3	Feeder links of non-geostationary-satellite systems in the mobile-satellite service.

4	Horizon antenna gain for the receive earth station (refer to § 3 of the main body of this Appendix).

5	Minimum elevation angle of operation in degrees (non-geostationary or geostationary).

6	Orbit of the space service in which the receiving earth station operates (non-geostationary or geostationary).

7	The thermal noise temperature of the receiving system at the terminal of the receiving antenna (under clear-sky conditions). Refer to § 2.1 of this Annex for missing values.

8	Horizon antenna gain is calculated using the procedure of Annex 5. Where no value of Gm is specified, a value of 42 dBi is to be used.

9	Non-geostationary horizon antenna gain, Ge = Gmin + 20 dB (see § 2.2), with Gmin = 10 – 10 log (D/), D/ = 13 (refer to Annex 3 for definition of symbols).

10	Unmanned space research is not a separate radiocommunication service and the system parameters are only to be used for the generation of supplementary contours.

TABLE 9b    (Rev.WRC-15)

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | Fixed-satellite |  |  | Fixed-satellite |  |  | Fixed-satellite | Fixed-satellite | Fixed- satellite 3 | Fixed- satellite 3 | Earth exploration- satellite, space research |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 10.7-11.7 |  |  | 12.5-12.75 |  |  | 17.3-17.8 | 17.7-18.4 | 19.3-19.6 | 19.3-19.6 | 40.0-40.5 |  |
| Space service designation in which the receiving earth station operates |  | Fixed-satellite |  |  | Fixed-satellite |  |  | Broadcasting-satellite | Fixed-satellite, meteorological- satellite | Fixed-satellite 3 | Fixed-satellite 4 | Fixed-satellite, mobile-satellite |  |
| Orbit 7 |  | GSO |  | Non-GSO | GSO |  | Non-GSO |  | GSO | Non-GSO | GSO | GSO | Non-GSO |
| Modulation at receiving earth station 1 |  | A | N | N | A | N |  |  | N | N |  |  |  |
| Receiving earth station interference parameters and criteria | p0 (%) | 0.03 | 0.003 |  | 0.03 | 0.003 |  |  | 0.003 | 0.01 | 0.003 | 0.003 |  |
|  | N | 2 | 2 |  | 2 | 2 |  |  | 2 | 1 | 2 | 2 |  |
|  | p (%) | 0.015 | 0.0015 |  | 0.015 | 0.0015 |  |  | 0.0015 | 0.01 | 0.0015 | 0.0015 |  |
|  | NL (dB) | 1 | 1 |  | 1 | 1 |  |  | 1 | 0 | 1 | 1 |  |
|  | Ms (dB) | 7 | 4 |  | 7 | 4 |  |  | 6 | 5 | 6 | 6 |  |
|  | W (dB) | 4 | 0 |  | 4 | 0 |  |  | 0 | 0 | 0 | 0 |  |
| Receiving earth station parameters | Gm (dBi) 2 |  |  | 51.9 |  |  | 31.2 |  | 58.6 | 53.2 | 49.5 | 50.8 | 54.4 |
|  | Gr 5 | 9 | 9 | 10 | 9 | 9 | 11 11 |  | 9 | 10 | 10 | 9 | 7 12 |
|  | εmin 6 | 5° | 5° | 6° | 5° | 5° | 10° |  | 5° | 5° | 10° | 10° | 10° |
|  | Te (K) 8 | 150 | 150 |  | 150 | 150 |  |  | 300 | 300 | 300 | 300 |  |
| Reference bandwidth | B (Hz) | 106 | 106 |  | 106 | 106 |  |  | 106 | 106 |  |  |  |
| Permissible interference power | Pr( p) (dBW) in B | −144 | −144 | −144 | −144 | −144 | −144 |  | −138 | −141 |  |  |  |

Notes to Table 9b:

1	A: analogue modulation; N: digital modulation.

2	On-axis gain of the receive earth station antenna.

3	Feeder links of non-geostationary satellite systems in the mobile-satellite service.

4	Geostationary-satellite systems.

5	Horizon antenna gain for the receive earth station (refer to § 3 of the main body of the Appendix).

6	Minimum elevation angle of operation in degrees (non-GSO or GSO).

7	Orbit of the space service in which the receiving earth station operates (GSO or non-GSO).

8	The thermal noise temperature of the receiving system at the terminal of the receiving antenna (under clear-sky conditions). Refer to § 2.1 of this Annex for missing values.

9	Horizon antenna gain is calculated using the procedure of Annex 5. Where no value of Gm is specified, a value of 42 dBi is to be used.

10	Horizon antenna gain is calculated using the procedure of Annex 5, except that the following antenna pattern may be used in place of that given in § 3 of Annex 3: G = 32 − 25 log φ for 1° ≤ φ < 48°; and G = −10 for 48° ≤ φ < 180° (refer to Annex 3 for definition of symbols).

11	Non-geostationary horizon antenna gain. Ge = Gmax (see § 2.2 of the main body of this Appendix) for G = 36 − 25 log (φ) > −6 (refer to Annex 3 for definition of symbols).

12	Non-geostationary horizon antenna gain. Ge = Gmax (see § 2.2 of the main body of this Appendix) for G = 32 − 25 log (φ) > −10 (refer to Annex 3 for definition of symbols).

TABLE 10     (Rev.WRC-15)

Predetermined coordination distances

| Frequency sharing situation |  |  | Coordination distance (in sharing situations involving services allocated with equal rights) (km) |
| --- | --- | --- | --- |
| Type of earth station |  | Type of terrestrial station |  |
| Ground-based in the bands below 1 GHz to which No. 9.11A applies. Ground-based mobile in the bands within the range 1-3 GHz to which No. 9.11A applies |  | Mobile (aircraft) | 500 |
| Aircraft (mobile) (all bands) |  | Ground-based | 500 |
| Aircraft (mobile) (all bands) |  | Mobile (aircraft) | 1 000 |
| Ground-based in the bands: 400.15-401 MHz 1 668.4-1 675 MHz |  | Station in the meteorological aids service (radiosonde) | 580 |
| Aircraft (mobile) in the bands: 400.15-401 MHz 1 668.4-1 675 MHz |  | Station in the meteorological aids service (radiosonde) | 1 080 |
| Ground-based in the radiodetermination-satellite service (RDSS) in the bands: 1 610-1 626.5 MHz 2 483.5-2 500 MHz 2 500-2 516.5 MHz |  | Ground-based | 100 |
| Airborne earth station in the radiodetermination-satellite service (RDSS) in the bands: 1 610-1 626.5 MHz 2 483.5-2 500 MHz 2 500-2 516.5 MHz |  | Ground-based | 400 |
| Receiving earth stations in the meteorological-satellite service |  | Station in the meteorological aids service | The coordination distance is considered to be the visibility distance as a function of the earth station horizon elevation angle for a radiosonde at an altitude of 20 km above mean sea level, assuming 4/3 Earth radius (see Note 1) |
| Non-GSO MSS feeder-link earth stations (all bands) |  | Mobile (aircraft) | 500 |
| Non-GSO MSS feeder-link earth stations in the band 5 091-5 150 MHz | Station in the aeronautical radionavigation service | Note 2 |  |
| Receiving earth stations in the space research service in the band: 2 200-2 290 MHz | Mobile (aircraft) | 880 |  |
| Ground-based in the bands in which the frequency sharing situation is not covered in the rows above | Mobile (aircraft) | 500 |  |

Notes to Table 10: NOTE 1 – The coordination distance, d (km), for fixed earth stations in the meteorological-satellite service vis-à-vis stations in the meteorological aids service assumes a radiosonde altitude of 20 km and is determined as a function of the physical horizon elevation angle h (degrees) for each azimuth, as follows: d = 100 for h ≥ 11° for 0° < h < 11° d = 582 for h ≤ 0° The minimum and maximum coordination distances are 100 km and 582 km, and correspond to physical horizon angles greater than 11° and less than 0°. (WRC-2000) NOTE 2 – For the coordination distance in the frequency band 5 091-5 150 MHz vis-à-vis stations in the aeronautical radionavigation service, see No. 5.444A. (WRC-15)

APPENDIX 8 (REV.WRC-15)

Method of calculation for determining if coordination is required between geostationary-satellite networks sharing the same frequency bands

# 1	Introduction

The method of calculation for determining if coordination is required under provision No. 9.7 is based on the concept that the noise temperature of a system subject to interference increases as the level of the interfering emission increases. It can, therefore, be applied irrespective of the modulation characteristics of these satellite networks, and of the precise frequencies used.

In this method, the apparent increase in the equivalent satellite link noise temperature resulting from an interfering emission of a given system is calculated (see § 2 below) and the ratio of this increase to the equivalent satellite link noise temperature, expressed as a percentage, is compared to a threshold value (see § 3 below).

# 2	Calculation of the apparent increase in equivalent noise temperature of the satellite link subject to an interfering emission

Two possible cases are considered:

Case I:	wanted and interfering networks share one or more frequency bands, each in the same direction of transmission;

Case II:	wanted and interfering networks share one or more frequency bands, each in opposite directions of transmission (bidirectional use).

These two cases cover all relative satellite positions from closely-spaced to near-antipodal positions.

## 2.1	Parameters

Let A be a satellite link of network R associated with satellite S and A′ be a satellite link of network R′ associated with satellite S′. The symbols relating to satellite link A′ bear primes, those relating to satellite link A do not bear primes.

The parameters are defined as follows (for satellite link A):

T :	the equivalent satellite link noise temperature, referred to the output of the receiving antenna of the earth station (K);

Ts :	the receiving system noise temperature of the space station, referred to the output of the receiving antenna of the space station (K);

Te :	the receiving system noise temperature of the earth station, referred to the output of the receiving antenna of the earth station (K);

ΔTs :	apparent increase in the receiving system noise temperature of the satellite S, caused by an interfering emission, referred to the output of the receiving antenna of this satellite (K);

ΔTe :	apparent increase in the receiving system noise temperature of the earth station eR, caused by an interfering emission, referred to the output of the receiving antenna of this station (K);

ps :	maximum power density per Hz delivered to the antenna of satellite S (averaged over the worst 4 kHz band for a carrier frequency below 15 GHz or over the worst 1 MHz band above 15 GHz) (W/Hz);

g3(η) : 	transmitting antenna gain of satellite S in the direction η (numerical power ratio);

ηA :	direction, from satellite S, of the receiving earth station eR of satellite link A;

ηe′ :	direction, from satellite S, of the receiving earth station e′R of satellite link A′;

NOTE – The product ps g3 (ηe′) is the maximum e.i.r.p. per Hz of satellite S in the direction of the receiving earth station e′R of satellite link A′.

ηs′ :	direction, from satellite S, of satellite S′;

pe :	maximum power density per Hz delivered to the antenna of the transmitting earth station eT (averaged over the worst 4 kHz band for a carrier frequency below 15 GHz or over the worst 1 MHz band above 15 GHz) (W/Hz);

g2(δ) : 	receiving antenna gain of satellite S in the direction δ (numerical power ratio);

δA :	direction, from satellite S, of the transmitting earth station eT of satellite link A;

δe′ :	direction, from satellite S, of the transmitting earth station e′T of satellite link A′;

δs′ :	direction, from satellite S, of satellite S′;

θt :	topocentric angular separation in degrees between the two satellites1, taking the longitudinal station-keeping tolerances into account;

NOTE – Only the topocentric angle θt should be used in dealing with Case I.

θg :	geocentric angular separation in degrees between the two satellites, taking the longitudinal station-keeping tolerances into account;

NOTE – Only the geocentric angle θg should be used in dealing with Case II.

g1(θt) : 	transmitting antenna gain of the earth station eT in the direction of satellite S′ (numerical power ratio);

g4(θt): 	receiving antenna gain of the earth station eR in the direction of satellite S′ (numerical power ratio);

k :	Boltzmann’s constant (1.38 × 10−23 J/K);

ld :	free-space transmission loss2 on the downlink (numerical power ratio), evaluated from satellite S to the receiving earth station eR for satellite link A;

NOTE – The free-space transmission loss on any downlink evaluated from the satellites S or S′ to the receiving earth stations eR or e′R is considered to be equal to ld.

lu :	free-space transmission loss2 on the uplink (numerical power ratio), evaluated from the earth station eT, to satellite S for satellite link A;

NOTE – The free-space loss on any uplink evaluated from the earth stations eT or e′T to the satellite S or S′ is considered to be equal to lu.

ls :	free-space transmission loss2 on the inter-satellite link (numerical power ratio), evaluated from satellite S′ to satellite S;

γ :	transmission gain of a specific satellite link subject to interference evaluated from the output of the receiving antenna of satellite S to the output of the receiving antenna of the earth station eR (numerical power ratio, usually less than 1).

## 2.2	General method

In the following equations, the frequency to be used for the calculation of ld, lu, and ls is the average frequency of the band common to both networks in the direction considered. If, in a given direction, there is no overlap of the assigned frequency bands of the two networks, the corresponding value (ΔTs or ΔTe) is taken to be equal to zero. For cases where the Appendix 4 data have not been published, the assigned frequency band for that network shall be considered as being the frequency range as provided for in Appendix 4.

### 2.2.1	Case I – Wanted and interfering networks sharing the same frequency band in the same direction of transmission

The gains g1(θt) and g4(θt) are those of the earth stations concerned. When neither measured data nor a relevant ITU-R Recommendation accepted by the administrations concerned are available the radiation patterns set out in Annex III should be used.

#### 2.2.1.1	Simple frequency-changing transponder on board the satellite

The parameters ΔTs and ΔTe are given by the following equations:

(1)

(2)

The symbol ΔT will be used to denote the apparent increase in the equivalent noise temperature for the entire satellite link referred to the output of the receiving antenna of the receiving earth station eR due to the interfering emission from link A′.

This increase is the result of the interfering emissions entering at both the satellite and the earth station receiver of link A and can accordingly be expressed as:

ΔT = γΔTs + ΔTe 	(3)

Hence,

(4)

An example calculation for the application of the method of this Appendix in Case I is given in Annex IV.

In the same way, the increase ΔT′ in the equivalent noise temperature for the entire satellite link, referred to the output of the receiving antenna of the receiving earth station e′R, under the effect of the interference caused by satellite link A, is given by the following equations:

(5)

(6)

(7)

#### 2.2.1.2	Cases requiring independent treatment of the uplink and the downlink

If there is a change of modulation in the satellite, if the transmission gain for the satellite network being considered has not been supplied, or if the transmission originates on board the satellite, then the apparent increase in the noise temperature must be related to the total receiving system noise temperature of the specific link being examined (the space station or the earth station, whichever is applicable). In this case, the equivalent noise temperature of the entire satellite link and the transmission gain are not used and equations (1) and (2) above are used separately as required (see § 3.2).     (WRC-03)

### 2.2.2	Case II – Wanted and interfering networks sharing the same frequency band in opposite directions of transmission (bidirectional use)

The calculation method below only applies to interfering emissions between satellites.

Interference between earth stations using the same frequency band in opposite directions of transmission (bidirectional use) is to be dealt with by coordination procedures analogous to those used for coordination between earth and terrestrial stations.

All the equations relating to Case II shall use the geocentric angle θg.

#### 2.2.2.1	Simple frequency-changing transponder on board the satellite

The noise temperature increase ΔTs referred to the output of the receiving antenna of the satellite of link A is given by:

(8)

The apparent increase in equivalent link noise temperature is then given by:

ΔT = γΔTs	(9)

The increase ΔT′ in the equivalent noise temperature of the link A′ caused by interfering emissions from the satellite associated with the link A is given by:

(10)

#### 2.2.2.2	Cases requiring independent treatment of the uplink and downlink

In this case equation (8) is used directly with Ts to obtain the percentage increase. The increase ΔT′s in the noise temperature of link A′ caused by interfering emissions from the satellite associated with link A is obtained in a similar manner.

### 2.2.3	Consideration of polarization isolation

The polarization isolation factor described in this paragraph shall be considered only if the administration responsible for each network has consented to such a course and has notified its polarization or published it for coordination under No. 9.7. In this case, the apparent increase in the equivalent satellite link noise temperature shall be determined by the following expressions:

Case I

Case II

where the values of ΔTs and ΔTe are those given in § 2.2.1 and § 2.2.2 and the values of the factors of polarization isolation Yu, Yd and Yss are those given in the Table below.

| Polarization |  | Factor of polarization isolation (numerical ratio) Y |
| --- | --- | --- |
| Network R | Network R′ |  |
| LHC LHC RHC LHC RHC L | RHC L L LHC RHC L | 4 1.4 1.4 1 1 1 |
| Where: LHC: left-hand circular (anti-clockwise) RHC: right-hand circular (clockwise) L: linear |  |  |

## 2.3	Determination of the satellite links to be considered in calculating the increase in equivalent satellite link noise temperature (Case I only)

The greatest increase in equivalent satellite link noise temperature caused to any link of another satellite network, existing or planned, by interfering emissions of the proposed satellite network must be determined.

The most unfavourably sited transmitting earth station of the interfering satellite network should be determined for each satellite receiving antenna of the network subject to interference by superimposing the “Earth-to-space” service areas of the interfering network on the space station receiving antenna gain contours plotted on a map of the Earth’s surface. The most unfavourably sited transmitting earth station is the one in the direction of which the satellite receiving antenna gain of the network subject to interference is the greatest.

The most unfavourably sited receiving earth station of the network subject to interference should be determined in an analogous manner for each “space-to-Earth” service area of that network. The most unfavourably sited receiving earth station is the one in the direction of which the satellite transmitting antenna gain of the interfering network is the greatest.

## 2.4	Use of information furnished under Appendix 4

When an administration elects to use information furnished under Appendix 4 with the calculation procedures of § 2.2.1.1 and § 2.2.2.1 in order to formulate comments, the calculations need to be made for both sets of values of γ and T furnished. The greater of the two values of ΔT/T resulting from these calculations is the one to be used.     (WRC-15)

# 3	Comparison between calculated percentage increase in noise temperature and the threshold value

## 3.1	Simple frequency-changing transponder on board the satellite

The calculated values of the ΔT/T and ΔT′/T′, expressed as percentages, shall be compared with the threshold value of 6%3.

–	If the calculated value of ΔT/T, expressed as a percentage, due to any interfering emission from satellite link A′ to satellite link A, is no greater than the threshold value, coordination is not required with respect to interference from link A′ to link A.

–	If the calculated value of ΔT/T, expressed as a percentage, is greater than the threshold value, coordination is required.

The comparison of ΔT′/T′, with the threshold value, expressed as a percentage, shall be carried out in a similar manner.

## 3.2	Cases requiring independent treatment of the uplink and the downlink

a)	In the case of interference into only one link, the uplink or the downlink, the value ΔTe/Te or ΔTs/Ts, expressed as a percentage, shall be compared with the threshold value of 6%3.

b)	In the case of interference into both the uplink and the downlink, between which there is a change of modulation on board the satellite, or in cases where the optional values for transmission gain and equivalent link noise temperature have not been supplied, the values of ΔTe/Te and ΔTs/Ts, expressed as a percentage, shall each be compared with the threshold value of 6%3.     (WRC-03)

# 4	Consideration of narrow-band and FM-TV carriers

The method of calculation described in this Appendix may underestimate the interference from slow swept TV carriers into certain narrow-band (single channel per carrier (SCPC)) carriers.

In order to facilitate coordination between the satellite systems and to reduce the number of administrations involved in this procedure, the administrations whose SCPC assignments are either recorded in the Master International Frequency Register or are under coordination may inform an administration notifying its new assignment of the radio frequency channels used in their systems for SCPC transmission, so that the notifying administration may be able to avoid using these channels for FM-TV transmissions.

For this special case, administrations are referred to relevant ITU-R texts for guidance in facilitating subsequent coordination.

Conversely, administrations introducing new systems using SCPC transmissions may seek appropriate information from other administrations on their FM-TV transmissions.

ANNEX 1

Calculation of the topocentric angular separation between 
two geostationary satellites

The topocentric angular separation θt between two geostationary satellites from a given earth station can be determined by using the equation:

where d1 and d2 are the distances (km), from the earth station to the two satellites respectively, and evaluated as d by the method described in Annex II, and θg is as defined in § 2.1.

ANNEX 2

Calculation of the free-space transmission loss

The free-space transmission loss L can be determined by using the following equation:

L = 20 (log f + log d) + 32.45          dB

where:

f :	frequency (MHz)

d :	distance (km).

a)	The distance d between an earth station and a geostationary satellite is given by the equation:

km

where:

cos ψ = cos ζ × cos β

where:

ζ :	latitude of the earth station

β :	difference in longitude between the satellite and the earth station.

NOTE – If cos ψ < 0.151, the satellite is below the horizontal plane.

b)	The distance ds between two geostationary satellites is determined as follows:

km

where:

θg:	geocentric angular separation as defined in § 2.1.

ANNEX 3

Radiation patterns for earth station antennas to be used 
when they are not published

When neither measured data nor relevant ITU-R Recommendations accepted by the administrations concerned are available then administrations should use the reference patterns as described below (dBi):

a)	for values of 4 (maximum gain ≥ 48 dBi approximately):

G(φ) = Gmax − 2.5 × 10−3  		for	0	< φ < φm

G(φ) = G1		for 	φm	≤ φ < φr

G(φ) = 32 − 25 log φ		for	φr	≤ φ < 48°

G(φ) = −10		for 	48°	≤ φ <180°

where:

| D: | antenna diameter | expressed in the same unit |
| --- | --- | --- |
| λ : | wavelength |  |

φ: 	off-axis angle of the antenna, in degrees, equal to θt or θg, as applicable

G1: 	gain of the first sidelobe = 2 + 15 log

degrees

degrees

b)	for values of 4 (maximum gain < 48 dBi approximately):

G(φ) = Gmax − 2.5 × 10−3  		for	 0	< φ < φm

G(φ) = G1		for  φm	  ≤ φ <

G(φ) = 52 − 10 log  − 25 log φ	for 	≤ φ < 48°

G(φ) = 10 − 10 log 		for  48°	≤ φ ≤ 180°

The above patterns may be modified as appropriate to achieve a better representation of the actual antenna pattern.

ANNEX 4

Example of an application of Appendix 8

# 1	General

In this example of Case I (see § 2.2.1), two identical satellite networks each with a simple frequency-changing transponder and a global coverage antenna are assumed.

All topocentric angles θt are assumed to be equal to 5°.

For this angular separation and for an earth station antenna with D/λ greater than 100, the reference radiation pattern (32 − 25 log θt) gives a gain of 14.5 dBi in the direction of the satellite of the other network.

The input data are furnished in § 2 below and are expressed in decibels except for the parameters T and θt. In § 3 the calculations are performed in decibels.

It may be noted that since both satellites use global beams there is practically no antenna discrimination between wanted and unwanted signals at the satellite, and that this constitutes a worst case.

# 2	Input data

The values of the network parameters given in the table below are derived from those published in accordance with Appendix 4.

|  | Symbol* | Value | Unit |
| --- | --- | --- | --- |
| Uplink at 6 175 MHz | P′e G′1(θt) G2(δe′) Lu | −37 14.5 15.5 200 | dB(W/Hz) dBi dBi dB |
| Downlink at 3 950 MHz | P′s G′3(ηe) G4(θt) Ld | −57 15.5 14.5 196 | dB(W/Hz) dBi dBi dB |
|  | 10 log γ T θt | −15 105 5 | dB K degrees |
| * All capital symbols, except T, refer to parameters given in logarithmic units. |  |  |  |

# 3	Calculation of

From equation (1)

dBK

Therefore,

ΔTs = 145          K

From equation (2)

dBK

Therefore;

ΔTe = 3.6           K

From equation (3)

K

Thus

%

# 4	Conclusion

In the example shown, the percentage increase in equivalent satellite link noise temperature is 7.8%. Since it exceeds the threshold value of 6%, coordination between the two networks is required.

APPENDIX 9

Report of an irregularity or infringement

(See Article 15, Section V)

Particulars concerning the station infringing the Radio Regulations:

1	Name1 if known (in BLOCK letters)

2	Call sign or other identification (in BLOCK letters)

3	Nationality, if known

4	Frequency used (kHz, MHz, GHz or THz)

5	Class of emission2

6	Class of station and nature of service, if known

7	Location3, 4, 5

Particulars concerning the station, the centralizing office or inspection service reporting 
the irregularity or infringement:

8	Name (in BLOCK letters)

9	Call sign or other identification (in BLOCK letters)

10	Nationality

11	Location3, 4

Particulars of the irregularity or infringement:

12	Name6 of the station (in BLOCK letters) in communication with the station committing the irregularity or infringement

13	Call sign or other identification (in BLOCK letters) of the 
station in communication with the station committing the irregularity or infringement

14	Date and time7

15	Nature of the irregularity or infringement8

16	Extracts from ship log or other information supporting the report

Particulars concerning the transmitting station interfered with9:

17	Name of the station (in BLOCK letters)

18	Call sign or other identification (in BLOCK letters)

19	Frequency assigned (kHz, MHz, GHz or THz)

20	Frequency measured at the time of the interference

21	Class of emission2 and bandwidth (indicate whether measured or estimated, or indicate the necessary bandwidth notified to the Radiocommunication Bureau)

22	Receiving location3, 4 (in BLOCK letters) where the interference was experienced

23	Certificate:

I certify that the foregoing report represents, to the best of my knowledge, a complete and accurate account of what took place.

Signatures10			Date:

Instructions for filling in this form

1	Each report shall refer to only one station (see Note 6). If it is forwarded as a letter, it shall be in duplicate, and whenever practicable should be typewritten. It may also be forwarded as a telegram.

2	The class of emission shall contain the basic characteristics listed in Appendix 1. If any characteristic cannot be determined, indicate the unknown symbol with a dash. However, if a station is not able to identify unambiguously whether the modulation is frequency or phase modulation, indicate frequency modulation (F).

3	In the case of land, fixed, or earth stations, the position shall be expressed in latitude and longitude (Greenwich). If the position cannot be furnished, the area of operation should be indicated.

4	In the case of ship or aircraft stations, the position shall be expressed either in latitude and longitude (Greenwich) or by a true bearing in degrees and distance in nautical miles, or in kilometres, from some well known place. If the position cannot be furnished, the area of operation should be indicated.

5	Where space stations are concerned, information shall be furnished on the orbit.

6	If both communicating stations infringe the Regulations, a separate report shall be made for each of these stations.

7	The time must be expressed as Coordinated Universal Time (UTC) by a group of four figures (0000 to 2359). If the infringement is prolonged or repeated, the dates and times shall be shown.

8	A separate report is required for each irregularity or infringement, unless they are repeated within a short time.

9	This information is to be given only in case of a complaint about interference.

10	This report shall be signed by the operator who has reported the infringement and countersigned by the Master of the ship or person responsible for the aircraft, or the officer in charge of the station in the case of an infringement reported by a station of the mobile service. When the report originates from a centralizing office or from an inspection service, it shall be signed by the head of that office or service and countersigned by an official of the administration sending it.

For the use of the administration only

1	Company controlling the installation of the station against which complaint is made

2	Name of the operator of the station held responsible for the irregularity or infringement of the Regulations

3	Action taken

APPENDIX 10 (REV.WRC-07)

Report of harmful interference

(See Article 15, Section VI)

Particulars concerning the station causing the interference:

a	Name, call sign or other means of identification

b	Frequency measured

Date:

Time (UTC):

c	Class of emission1

d	Bandwidth (indicate whether measured or estimated)

e	Measured field strength or power flux-density2

Date:

Time (UTC):

f	Observed polarization

g	Class of station and nature of service

h	Location/position/area/bearing (QTE3)     (WRC-07)

i	Location of the facility which made the above measurements

Particulars concerning the transmitting station interfered with:

j	Name, call sign or other means of identification

k	Frequency assigned

l	Frequency measured

Date:

Time (UTC):

m	Class of emission4

n	Bandwidth (indicate whether measured or estimated, or indicate the necessary bandwidth notified to the Radiocommunication Bureau)

o	Location/position/area

p	Location of the facility which made the above measurements

Particulars furnished by the receiving station experiencing the interference:

q	Name of station

r	Location/position/area

s	Dates and times (UTC) of occurrence of harmful interference

t	Bearings (QTE5) or other particulars     (WRC-07)

u	Nature of interference

v	Field strength or power flux-density of the wanted emission at the receiving station experiencing the interference6

Date:

Time (UTC):

w	Polarization of the receiving antenna or observed polarization

x	Action requested

NOTE – For convenience and brevity, telegraphic reports shall be in the format above, using the letters in the order listed in lieu of the explanatory titles, but only those letters for which information is provided should be used. However, sufficient information shall be provided to the administration receiving the report, so that an appropriate investigation can be conducted.

APPENDIX 11 (REV.WRC-03)

System specifications for double-sideband (DSB), single-sideband (SSB) and digitally modulated emissions in the HF broadcasting service

PART  A  –  Double-sideband (DSB) system

# 1	System parameters

## 1.1	Channel spacing

The nominal spacing for DSB shall be 10 kHz. However, the interleaved channels with a separation of 5 kHz may be used in accordance with the relative protection criteria, provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

# 2	Emission characteristics

## 2.1	Nominal carrier frequencies

Nominal carrier frequencies shall be integral multiples of 5 kHz.

## 2.2	Audio-frequency band

The upper limit of the audio-frequency band (at −3 dB) of the transmitter shall not exceed 4.5 kHz and the lower limit shall be 150 Hz, with lower frequencies attenuated at a slope of 6 dB per octave.

## 2.3	Modulation processing

If audio-frequency signal processing is used, the dynamic range of the modulating signal shall be not less than 20 dB.

## 2.4	Necessary bandwidth

The necessary bandwidth shall not exceed 9 kHz.

PART  B  –  Single-sideband (SSB) system

# 1	System parameters

## 1.1	Channel spacing

In a mixed DSB, SSB and digital environment (see Resolution 517 (Rev.WRC-03)*), the channel spacing shall be 10 kHz. In the interest of spectrum conservation, it is also permissible to interleave SSB emissions midway between two adjacent DSB channels, i.e., with 5 kHz separation between carrier frequencies, provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

In an all inclusive SSB environment, the channel spacing and carrier frequency separation shall be 5 kHz.     (WRC-03)

## 1.2	Equivalent sideband power

When the carrier reduction relative to peak envelope power is 6 dB, an equivalent SSB emission is one giving the same audio-frequency signal-to-noise ratio at the receiver output as the corresponding DSB emission, when it is received by a DSB receiver with envelope detection. This is achieved when the sideband power of the SSB emission is 3 dB larger than the total sideband power of the DSB emission. (The peak envelope power of the equivalent SSB emission and the carrier power are the same as that of the DSB emission.)

# 2	Emission characteristics

## 2.1	Nominal carrier frequencies

Nominal carrier frequencies shall be integral multiples of 5 kHz.

## 2.2	Frequency tolerance

The frequency tolerance shall be 10 Hz.1

## 2.3	Audio-frequency band

The upper limit of the audio-frequency band (at −3 dB) of the transmitter shall not exceed 4.5 kHz with a further slope of attenuation of 35 dB/kHz and the lower limit shall be 150 Hz with lower frequencies attenuated at a slope of 6 dB per octave.

## 2.4	Modulation processing

If audio-frequency signal processing is used, the dynamic range of the modulating signal shall be not less than 20 dB.

## 2.5	Necessary bandwidth

The necessary bandwidth shall not exceed 4.5 kHz.

## 2.6	Carrier reduction (relative to peak envelope power)

In a mixed DSB, SSB and digital environment, the carrier reduction shall be 6 dB to allow SSB emissions to be received by conventional DSB receivers with envelope detection without significant deterioration of the reception quality.     (WRC-03)

## 2.7	Sideband to be emitted

Only the upper sideband shall be used.

## 2.8	Attenuation of the unwanted sideband

The attenuation of the unwanted sideband (lower sideband) and of intermodulation products in that part of the emission spectrum shall be at least 35 dB relative to the wanted sideband signal level. However, since there is in practice a large difference between signal amplitudes in adjacent channels, a greater attenuation is recommended.

# 3	Characteristics of the reference receiver

The reference receiver has the main characteristics as given below. For more detailed characteristics see the relevant ITU-R Recommendations.

## 3.1	Noise limited sensitivity

The value of the noise limited sensitivity is equal to or less than 40 dB(μV/m).

## 3.2	Demodulator and carrier acquisition

The reference receiver is equipped with a synchronous demodulator, using for the carrier acquisition a device which regenerates a carrier by means of a suitable control loop which locks the receiver to the incoming carrier. The reference receiver should work as well with DSB emissions as with SSB emissions having a carrier reduced to 6 dB below peak envelope power.     (WRC-03)

## 3.3	Overall selectivity

The reference receiver has an overall bandwidth (at −3 dB) of 4 kHz, with a slope of attenuation of 35 dB/kHz.

NOTE – Other combinations of bandwidth and slope of attenuation are possible, as given below, and will provide the same performance at 5 kHz carrier difference.

| Slope of attenuation | Overall bandwidth (−3 dB) |
| --- | --- |
| 25 dB/kHz | 3 300 Hz |
| 15 dB/kHz | 2 700 Hz |

PART C  –  Digital system     (WRC-03)

# 1	System parameters

## 1.1	Channel spacing

The initial spacing for digitally modulated emissions shall be 10 kHz. However, interleaved channels with a separation of 5 kHz may be used in accordance with the appropriate protection criteria appearing in Resolution 543 (WRC-03), provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

## 1.2	Channel utilization

Channels using digitally modulated emissions may share the same spectrum or be interleaved with analogue emissions in the same high frequency broadcasting (HFBC) band, provided the protection afforded to the analogue emissions is at least as great as that which is currently in force for analogue-to-analogue protection. Accomplishing this may require that the digital spectral power density (and total power) be lower by several dB than is currently used for either DSB or SSB emissions.

# 2	Emission characteristics

## 2.1	Bandwidth and centre frequency

A full digitally modulated emission will have a 10 kHz bandwidth with its centre frequency at any of the 5 kHz centre frequency locations in the channel raster currently in use within the HFBC bands.

Among several possible “simulcast” modes are those having a combination of analogue and digital emissions of the same programme in the same channel, that may use a digital emission of 5 kHz or 10 kHz bandwidth, next to either a 5 kHz or 10 kHz analogue emission. In all cases of this type, the 5 kHz interleaved raster used in HFBC shall be adhered to in placing the emission within these bands.

## 2.2	Frequency tolerance

The frequency tolerance shall be 10 Hz1.

## 2.3	Audio-frequency band

The quality of service, using digital source coding within a 10 kHz bandwidth, taking into account the need to adapt the emission coding for various levels of error avoidance, detection and correction, can range from the equivalent of monophonic FM (approximately 15 kHz) to the low-level performance of a speech codec (of the order of 3 kHz). The choice of audio quality is connected to the needs of the broadcaster and listener, and includes the consideration of such characteristics as the propagation conditions expected. There is no single specification, only the upper and lower bounds noted in this paragraph.

## 2.4	Modulation

Quadrature amplitude modulation (QAM) with orthogonal frequency division multiplexing (OFDM) shall be used. 64-QAM is feasible under many propagation conditions; others such as 32-, 16- and 8-QAM are specified for use when needed.

## 2.5	RF protection ratio values

The protection ratio values for analogue and digital emissions for co-channel and adjacent channel conditions shall be in accordance with Resolution 543 (WRC-03) as provisional RF protection ratio values subject to revision or confirmation by a future competent conference.

APPENDIX 12

Special rules applicable to radiobeacons

(See Article 28)

Section I  –  Aeronautical radiobeacons

1)	The assignment of frequencies to aeronautical radiobeacons operating in the bands between 160 kHz and 535 kHz shall be based on a protection ratio against interference of at least 15 dB for each beacon throughout its service area.

2)	The radiated power should be kept to the minimum value necessary to give the desired field strength at the service range.

3)	The daylight service range of radiobeacons referred to in § 1) above shall be based on the following field strengths:

4)	Regions 1 and 2

–	70 μV/m for radiobeacons north of 30° N;

–	120 μV/m for radiobeacons between 30° N and 30° S;

–	70 μV/m for radiobeacons south of 30° S.

5)	Region 3

–	70 μV/m for radiobeacons north of 40° N;

–	120 μV/m for radiobeacons between 40° N and 50° S;

–	70 μV/m for radiobeacons south of 50° S.

Section II  –  Maritime Radiobeacons

1)	The protection ratio required for assignment of frequencies to maritime radiobeacons operating in the bands between 283.5 kHz and 335 kHz shall be based on the effective radiated power being kept to the minimum value necessary to give the desired field strength at the service range and the need to provide adequate geographical separation between radiobeacons operating on the same frequency and at the same time, to avoid harmful interference.

2)	The daylight service range of the radiobeacons referred to in § 1) above shall be based on the following field strengths:

3)	Region 1

–	50 μV/m for radiobeacons north of 43° N;

–	75 μV/m for radiobeacons between 43° N and 30° N;

–	100 μV/m for radiobeacons between 30° N and 30° S;

–	75 μV/m for radiobeacons between 30° S and 43° S;

–	50 μV/m for radiobeacons south of 43° S.

4)	Region 2

–	50 μV/m for radiobeacons north of 40° N;

–	75 μV/m for radiobeacons between 40° N and 31° N;

–	100 μV/m for radiobeacons between 31° N and 30° S;

–	75 μV/m for radiobeacons between 30° S and 43° S;

–	50 μV/m for radiobeacons south of 43° S.

5)	Region 3

–	75 μV/m for radiobeacons north of 40° N;

–	100 μV/m for radiobeacons between 40° N and 50° S;

–	75 μV/m for radiobeacons south of 50° S.

6)	The carrier frequencies of maritime radiobeacons and the separation between channels shall be based on the use of integer multiples of 100 Hz. The separation between adjacent carrier frequencies should be based on relevant ITU-R Recommendations.

APPENDIX 14 (REV.WRC-07)

Phonetic alphabet and figure code

(See Articles 30 and 57)     (WRC-07)

1	When it is necessary to spell out call signs, service abbreviations and words, the following letter spelling table shall be used:

| Letter to be transmitted | Code word to be used | Spoken as1 |
| --- | --- | --- |
| A | Alfa | AL FAH |
| B | Bravo | BRAH VOH |
| C | Charlie | CHAR LEE or SHAR LEE |
| D | Delta | DELL TAH |
| E | Echo | ECK OH |
| F | Foxtrot | FOKS TROT |
| G | Golf | GOLF |
| H | Hotel | HOH TELL |
| I | India | IN DEE AH |
| J | Juliett | JEW LEE ETT |
| K | Kilo | KEY LOH |
| L | Lima | LEE MAH |
| M | Mike | MIKE |
| N | November | NO VEM BER |
| O | Oscar | OSS CAH |
| P | Papa | PAH PAH |
| Q | Quebec | KEH BECK |
| R | Romeo | ROW ME OH |
| S | Sierra | SEE AIR RAH |
| T | Tango | TANG GO |
| U | Uniform | YOU NEE FORM or OO NEE FORM |
| V | Victor | VIK TAH |
| W | Whiskey | WISS KEY |
| X | X-ray | ECKS RAY |
| Y | Yankee | YANG KEY |
| Z | Zulu | ZOO LOO |

2	When it is necessary to spell out figures or marks, the following table shall be used:

| Figure or mark to be transmitted | Code word to be used | Spoken as2 |
| --- | --- | --- |
| 0 | Nadazero | NAH-DAH-ZAY-ROH |
| 1 | Unaone | OO-NAH-WUN |
| 2 | Bissotwo | BEES-SOH-TOO |
| 3 | Terrathree | TAY-RAH-TREE |
| 4 | Kartefour | KAR-TAY-FOWER |
| 5 | Pantafive | PAN-TAH-FIVE |
| 6 | Soxisix | SOK-SEE-SIX |
| 7 | Setteseven | SAY-TAY-SEVEN |
| 8 | Oktoeight | OK-TOH-AIT |
| 9 | Novenine | NO-VAY-NINER |
| Decimal point | Decimal | DAY-SEE-MAL |
| Full stop | Stop | STOP |

3	However, stations of the same country, when communicating between themselves, may use any other table recognized by their administration.

APPENDIX 15 (REV.WRC-15)

Frequencies for distress and safety communications for the Global
Maritime Distress and Safety System (GMDSS)

(See Article 31)

The frequencies for distress and safety communications for the GMDSS are given in Tables 15-1 and 15-2 for frequencies below and above 30 MHz, respectively.

TABLE  15-1     (WRC-07)

Frequencies below 30 MHz

| Frequency (kHz) | Description of usage | Notes |
| --- | --- | --- |
| 490 | MSI | The frequency 490 kHz is used exclusively for maritime safety information (MSI). (WRC-03) |
| 518 | MSI | The frequency 518 kHz is used exclusively by the international NAVTEX system. |
| *2 174.5 | NBDP-COM |  |
| *2 182 | RTP-COM | The frequency 2 182 kHz uses class of emission J3E. See also No. 52.190. |
| *2 187.5 | DSC |  |
| 3 023 | AERO-SAR | The aeronautical carrier (reference) frequencies 3 023 kHz and 5 680 kHz may be used for intercommunication between mobile stations engaged in coordinated search and rescue operations, and for communication between these stations and participating land stations, in accordance with the provisions of Appendix 27 (see Nos. 5.111 and 5.115). |
| *4 125 | RTP-COM | See also No. 52.221. The carrier frequency 4 125 kHz may be used by aircraft stations to communicate with stations of the maritime mobile service for distress and safety purposes, including search and rescue (see No. 30.11). |
| *4 177.5 | NBDP-COM |  |
| *4 207.5 | DSC |  |
| 4 209.5 | MSI | The frequency 4 209.5 kHz is exclusively used for NAVTEX-type transmissions (see Resolution 339 (Rev.WRC-07)). |
| 4 210 | MSI-HF |  |
| 5 680 | AERO-SAR | See note under 3 023 kHz above. |
| *6 215 | RTP-COM | See also No. 52.221. |
| *6 268 | NBDP-COM |  |
| *6 312 | DSC |  |

TABLE  15-1  (end)     (WRC-07)

| Frequency (kHz) | Description of usage | Notes |
| --- | --- | --- |
| 6 314 | MSI-HF |  |
| *8 291 | RTP-COM |  |
| *8 376.5 | NBDP-COM |  |
| *8 414.5 | DSC |  |
| 8 416.5 | MSI-HF |  |
| *12 290 | RTP-COM |  |
| *12 520 | NBDP-COM |  |
| *12 577 | DSC |  |
| 12 579 | MSI-HF |  |
| *16 420 | RTP-COM |  |
| *16 695 | NBDP-COM |  |
| *16 804.5 | DSC |  |
| 16 806.5 | MSI-HF |  |
| 19 680.5 | MSI-HF |  |
| 22 376 | MSI-HF |  |
| 26 100.5 | MSI-HF |  |
| Legend: AERO-SAR These aeronautical carrier (reference) frequencies may be used for distress and safety purposes by mobile stations engaged in coordinated search and rescue operations. DSC These frequencies are used exclusively for distress and safety calls using digital selective calling in accordance with No. 32.5 (see Nos. 33.8 and 33.32). (WRC-07) MSI In the maritime mobile service, these frequencies are used exclusively for the transmission of maritime safety information (MSI) (including meteorological and navigational warnings and urgent information) by coast stations to ships, by means of narrow-band direct-printing telegraphy. MSI-HF In the maritime mobile service, these frequencies are used exclusively for the transmission of high seas MSI by coast stations to ships, by means of narrow-band direct-printing telegraphy. NBDP-COM These frequencies are used exclusively for distress and safety communications (traffic) using narrow-band direct-printing telegraphy. RTP-COM These carrier frequencies are used for distress and safety communications (traffic) by radiotelephony. * Except as provided in these Regulations, any emission capable of causing harmful interference to distress, alarm, urgency or safety communications on the frequencies denoted by an asterisk (*) is prohibited. Any emission causing harmful interference to distress and safety communications on any of the discrete frequencies identified in this Appendix is prohibited. (WRC-07) |  |  |

TABLE  15-2     (WRC-15)

Frequencies above 30 MHz (VHF/UHF)

| Frequency (MHz) | Description of usage | Notes |
| --- | --- | --- |
| *121.5 | AERO-SAR | The aeronautical emergency frequency 121.5 MHz is used for the purposes of distress and urgency for radiotelephony by stations of the aeronautical mobile service using frequencies in the frequency band between 117.975 MHz and 137 MHz. This frequency may also be used for these purposes by survival craft stations. Use of the frequency 121.5 MHz by emergency position-indicating radio beacons shall be in accordance with Recommendation ITU-R M.690-3. Mobile stations of the maritime mobile service may communicate with stations of the aeronautical mobile service on the aeronautical emergency frequency 121.5 MHz for the purposes of distress and urgency only, and on the aeronautical auxiliary frequency 123.1 MHz for coordinated search and rescue operations, using class A3E emissions for both frequencies (see also Nos. 5.111 and 5.200). They shall then comply with any special arrangement between governments concerned by which the aeronautical mobile service is regulated. |
| 123.1 | AERO-SAR | The aeronautical auxiliary frequency 123.1 MHz, which is auxiliary to the aeronautical emergency frequency 121.5 MHz, is for use by stations of the aeronautical mobile service and by other mobile and land stations engaged in coordinated search and rescue operations (see also No. 5.200). Mobile stations of the maritime mobile service may communicate with stations of the aeronautical mobile service on the aeronautical emergency frequency 121.5 MHz for the purposes of distress and urgency only, and on the aeronautical auxiliary frequency 123.1 MHz for coordinated search and rescue operations, using class A3E emissions for both frequencies (see also Nos. 5.111 and 5.200). They shall then comply with any special arrangement between governments concerned by which the aeronautical mobile service is regulated. |
| 156.3 | VHF-CH06 | The frequency 156.3 MHz may be used for communication between ship stations and aircraft stations engaged in coordinated search and rescue operations. It may also be used by aircraft stations to communicate with ship stations for other safety purposes (see also Note f ) in Appendix 18). |
| *156.525 | VHF-CH70 | The frequency 156.525 MHz is used in the maritime mobile service for distress and safety calls using digital selective calling (see also Nos. 4.9, 5.227, 30.2 and 30.3). |
| 156.650 | VHF-CH13 | The frequency 156.650 MHz is used for ship-to-ship communications relating to the safety of navigation in accordance with Note k) in Appendix 18. |
| *156.8 | VHF-CH16 | The frequency 156.8 MHz is used for distress and safety communications by radiotelephony. Additionally, the frequency 156.8 MHz may be used by aircraft stations for safety purposes only. |
| *161.975 | AIS-SART VHF CH AIS 1 | AIS 1 is used for AIS search and rescue transmitters (AIS-SART) for use in search and rescue operations. |
| *162.025 | AIS-SART VHF CH AIS 2 | AIS 2 is used for AIS search and rescue transmitters (AIS-SART) for use in search and rescue operations. |

TABLE  15-2  (end)     (WRC-15)

| Frequency (MHz) | Description of usage | Notes |
| --- | --- | --- |
| *406-406.1 | 406-EPIRB | This frequency band is used exclusively by satellite emergency position-indicating radio beacons in the Earth-to-space direction (see No. 5.266). |
| 1 530-1 544 | SAT-COM | In addition to its availability for routine non-safety purposes, the band 1 530-1 544 MHz is used for distress and safety purposes in the space-to-Earth direction in the maritime mobile-satellite service. GMDSS distress, urgency and safety communications have priority in this band (see No. 5.353A). |
| *1 544-1 545 | D&S-OPS | Use of the band 1 544-1 545 MHz (space-to-Earth) is limited to distress and safety operations (see No. 5.356), including feeder links of satellites needed to relay the emissions of satellite emergency position-indicating radio beacons to earth stations and narrow-band (space-to-Earth) links from space stations to mobile stations. |
| 1 626.5-1 645.5 | SAT-COM | In addition to its availability for routine non-safety purposes, the band 1 626.5-1 645.5 MHz is used for distress and safety purposes in the Earth-to-space direction in the maritime mobile-satellite service. GMDSS distress, urgency and safety communications have priority in this band (see No. 5.353A). |
| *1 645.5-1 646.5 | D&S-OPS | Use of the band 1 645.5-1 646.5 MHz (Earth-to-space) is limited to distress and safety operations (see No. 5.375). |
| 9 200-9 500 | SARTS | This frequency band is used by radar transponders to facilitate search and rescue. |
| Legend: AERO-SAR These aeronautical carrier (reference) frequencies may be used for distress and safety purposes by mobile stations engaged in coordinated search and rescue operations. D&S-OPS The use of these bands is limited to distress and safety operations of satellite emergency position-indicating radio beacons (EPIRBs). SAT-COM These frequency bands are available for distress and safety purposes in the maritime mobile-satellite service (see Notes). VHF-CH# These VHF frequencies are used for distress and safety purposes. The channel number (CH#) refers to the VHF channel as listed in Appendix 18, which should also be consulted. AIS These frequencies are used by automatic identification systems (AIS), which should operate in accordance with the most recent version of Recommendation ITU-R M.1371. (WRC-07) * Except as provided in these Regulations, any emission capable of causing harmful interference to distress, alarm, urgency or safety communications on the frequencies denoted by an asterisk (*) is prohibited. Any emission causing harmful interference to distress and safety communications on any of the discrete frequencies identified in this Appendix is prohibited. (WRC-07) |  |  |

APPENDIX 16 (REV.WRC-07)

Documents with which stations on board ships 
and aircraft shall be provided

(See Articles 42 and 51)

Section I  –  Ship stations for which a Global Maritime Distress and Safety System 
installation is required by international agreement

These stations shall be provided with:

1	the licence prescribed by Article 18;

2	certificates of the operator or operators;

3	a log in which the following are recorded as they occur, together with the time of the occurrence, unless administrations have adopted other arrangements for recording all information which the log should contain:

a)	a summary of communications relating to distress, urgency and safety traffic;

b)	a reference to important service incidents;

4	the List of Ship Stations and Maritime Mobile Service Identity Assignments (see Article 20) in either printed or electronic format;      (WRC-07)

5	the List of Coast Stations and Special Service Stations (see Article 20) in either printed or electronic format;     (WRC-07)

6	the Manual for Use by the Maritime Mobile and Maritime Mobile-Satellite Services (see Article 20) in either printed or electronic format.     (WRC-07)

NOTE – An administration may exempt a ship from the carriage of the documents mentioned in items 5 and 6 above under various circumstances (for example, when that ship carries equivalent information for the ship’s specified trading area).

Section II  –  Other ship stations for which a radio installation is required by 
regional or international agreement     (WRC-07)

These stations shall be provided with:

1	the licence prescribed by Article 18;

2	certificates of the operator or operators;

3	a log or other arrangements which the administration may have adopted for that purpose, in which a summary of communications related to distress, urgency and safety traffic shall be recorded together with the time of their occurrence;

4	the List of Coast Stations and Special Service Stations (see Article 20) in either printed or electronic format;

5	the relevant rules and procedures of radiocommunications, e.g. Manual for Use by the Maritime Mobile and Maritime Mobile-Satellite Services (paper or electronic format) (see Article 20).

NOTE – An administration may exempt a ship from the carriage of the documents mentioned in items 4 and 5 above under various circumstances (for example, when that ship carries equivalent information for the ship’s specified trading area).

Section III  –  Other ship stations     (WRC-07)

These stations shall be provided with:

1	the documents mentioned in items 1 and 2 of Section II;

2	the documents mentioned in items 4 and 5 of Section II, in accordance with the requirements of the administrations concerned.

NOTE – An administration may exempt a ship from the carriage of the documents mentioned in item 2 above under various circumstances (for example, when that ship carries equivalent information for the ship’s specified trading area). Administrations may also, by mutual agreement, exempt ships travelling only between their national jurisdictions from the licensing prescribed by Article 18 and the carriage of the documents mentioned in item 1 above, provided those vessels are otherwise licensed or authorized by regulation.

Section IV  –  Stations on board aircraft     (WRC-07)

These stations shall be provided with:

1	the documents mentioned in items 1 and 2 of Section I;

2	a log, unless administrations have adopted other arrangements for recording all infor-mation which the log should contain;

3	those published documents, in either printed or electronic formats, containing official information relating to stations which the aircraft station may use for the execution of its service.

APPENDIX 17 (REV.WRC-15)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

This Appendix is separated into two annexes:

Annex 1 contains the existing frequency and channelling arrangements in the high-frequency bands for the maritime mobile service, in force until 31 December 2016.

Annex 2 contains the future frequency and channelling arrangements in the high-frequency bands for the maritime mobile service, as revised by WRC-12, which enter into force on 1 January 2017.     (WRC-12)

Annex 1*     (WRC-15)

Frequencies and channelling arrangements in the high-frequency 
bands for the maritime mobile service, in force 
until 31 December 2016     (WRC-12)

PART  A  –  Table of subdivided bands     (WRC-07)

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies (f.) and the spacing in kHz being indicated in italics.

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation a) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| Frequencies assignable to ship stations for wide-band telegraphy, facsimile and special transmission systems | 4 154 to 4 170 5 f. 4 kHz | 6 235 to 6 259 7 f. 4 kHz | 8 302 to 8 338 10 f. 4 kHz | 12 370 to 12 418 13 f. 4 kHz | 16 551 to 16 615 17 f. 4 kHz | 18 848 to 18 868 6 f. 4 kHz | 22 182 to 22 238 15 f. 4 kHz | 25 123 to 25 159 10 f. 4 kHz |
| Limits (kHz) | 4 172 | 6 261 | 8 340 | 12 420 | 16 617 | 18 870 | 22 240 | 25 161.25 |
| Frequencies assignable to ship stations for oceanographic data transmission c) |  | 6 261.3 to 6 262.5 5 f. 0.3 kHz | 8 340.3 to 8 341.5 5 f. 0.3 kHz | 12 420.3 to 12 421.5 5 f. 0.3 kHz | 16 617.3 to 16 618.5 5 f. 0.3 kHz |  | 22 240.3 to 22 241.5 5 f. 0.3 kHz |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) m) p) | 4 172.5 to 4 181.5 18 f. 0.5 kHz | 6 263 to 6 275.5 25 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 181.75 | 6 275.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 280.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) m) p) |  | 6 281 to 6 284.5 8 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 |  | 8 | 12 | 16 | 18/19 |  | 22 |  | 25/26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 186.75 | 6 284.75 |  | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 |  | 22 241.75 |  | 25 161.25 |  |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) p) | 4 187 to 4 202 31 f. 0.5 kHz |  | 6 285 to 6 300 31 f. 0.5 kHz | 8 342 to 8 365.5 48 f. 0.5 kHz | 12 422 to 12 476.5 110 f. 0.5 kHz | 16 619 to 16 683 129 f. 0.5 kHz |  |  | 22 242 to 22 279 75 f. 0.5 kHz |  | 25 161.5 to 25 171 20 f. 0.5 kHz | (WRC-07) |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 365.75 | 12 476.75 | 16 683.25 | 18 870 |  | 22 279.25 |  | 25 171.25 |  |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 370.75 | 12 476.75 | 16 683.25 |  | 18 870 |  | 22 284.25 | 25 172.75 |  |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) p) |  |  |  | 8 371 to 8 376 11 f. 0.5 kHz |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 376.25 | 12 476.75 | 16 683.25 |  | 18 870 |  | 22 284.25 | 25 172.75 |  |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) m) p) |  |  |  | 8 376.5 to 8 396 40 f. 0.5 kHz | 12 477 to 12 549.5 146 f. 0.5 kHz | 16 683.5 to 16 733.5 101 f. 0.5 kHz |  | 18 870.5 to 18 892.5 45 f. 0.5 kHz |  | 22 284.5 to 22 351.5 135 f. 0.5 kHz | 25 173 to 25 192.5 40 f. 0.5 kHz |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 396.25 | 12 549.75 | 16 733.75 |  | 18 892.75 |  | 22 351.75 | 25 192.75 |  |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 396.25 | 12 554.75 | 16 738.75 |  | 18 892.75 |  | 22 351.75 | 25 192.75 |  |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) m) p) |  |  |  |  | 12 555 to 12 559.5 10 f. 0.5 kHz | 16 739 to 16 784.5 92 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 396.25 | 12 559.75 | 16 784.75 |  | 18 892.75 |  | 22 351.75 | 25 192.75 |  |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (non paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK and for A1A or A1B Morse telegraphy (working) b) p) | 4 202.5 to 4 207 10 f. 0.5 kHz | 6 300.5 to 6 311.5 23 f. 0.5 kHz | 8 396.5 to 8 414 36 f. 0.5 kHz | 12 560 to 12 576.5 34 f. 0.5 kHz | 16 785 to 16 804 39 f. 0.5 kHz | 18 893 to 18 898 11 f. 0.5 kHz | 22 352 to 22 374 45 f. 0.5 kHz | 25 193 to 25 208 31 f. 0.5 kHz |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414.25 | 12 576.75 | 16 804.25 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) n) o) p) | 4 209.5 to 4 219 20 f. 0.5 kHz | 6 314 to 6 330.5 34 f. 0.5 kHz | 8 416.5 to 8 436 40 f. 0.5 kHz | 12 579 to 12 656.5 156 f. 0.5 kHz | 16 806.5 to 16 902.5 193 f. 0.5 kHz | 19 680.5 to 19 703 46 f. 0.5 kHz | 22 376 to 22 443.5 136 f. 0.5 kHz | 26 100.5 to 26 120.5 41 f. 0.5 kHz |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz | 22 444 to 22 445 3 f. 0.5 kHz | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for wide-band and A1A or A1B Morse telegraphy, facsimile, special and data transmission systems and direct-printing telegraphy systems |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

a)	See Part B, Section I.

b)	See Part B, Section III.

c)	The frequency bands may also be used by buoy stations for oceanographic data transmission and by stations interrogating these buoys.

d)	See Part B, Section II.

e)	In the frequency bands to be used by ship stations for A1A Morse telegraphy working at speeds not exceeding 40 Bd, administrations may assign additional frequencies interleaved between the assignable frequencies. Any frequencies so assigned shall be multiples of 100 Hz. Administrations shall ensure a uniform distribution of such assignments within the bands.

f)	See Part B, Section V.

g)	See Part B, Section IV.

h)	(SUP - WRC-07)

i)	For the use of the carrier frequencies 4 125 kHz, 6 215 kHz, 8 291 kHz, 12 290 kHz and 16 420 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31.     (WRC-07)

j)	For the use of the frequencies 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by NBDP telegraphy, see Article 31.

k)	For the use of the frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by digital selective calling, see Article 31.

l)	The following paired frequencies (for ship/coast stations) 4 208/4 219.5 kHz, 6 312.5/6 331 kHz, 8 415/ 8 436.5 kHz, 12 577.5/12 657 kHz, 16 805/16 903 kHz, 18 898.5/19 703.5 kHz, 22 374.5/22 444 kHz and 25 208.5/26 121 kHz are the first choice international frequencies for digital selective calling (see Article 54).

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy (working) (see Part B, Section II).

n)	The frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the exclusive international frequencies for the transmission of maritime safety information (MSI) (see Articles 31 and 33).

o)	The frequency 4 209.5 kHz is an exclusive international frequency for the transmission of NAVTEX type information (see Articles 31 and 33).

p)	These sub-bands, except the frequencies referred to in Notes j), n) and o), may be used for the initial testing and the possible future introduction within the maritime mobile service of new digital technologies. Stations using these sub-bands for this purpose shall not cause harmful interference to, and shall not claim protection from, other stations operating in accordance with Article 5.

PART  B  –  Channelling arrangements     (WRC-15)

Section I  –  Radiotelephony

1	Radiotelephone channelling arrangements for the frequencies to be used by coast and ship stations in the bands allocated to the maritime mobile service are indicated in the following Sub-Sections:

Sub-Section A	–	Table of single-sideband transmitting frequencies (kHz) for duplex (two-frequency) operation;

Sub-Section B	–	Table of single-sideband transmitting frequencies (kHz) for simplex (single-frequency) operation and for intership cross-band (two-frequency) operation;

Sub-Section C-1	–	Table of recommended single-sideband transmitting frequencies (kHz) for ship stations in the frequency band 4 000-4 063 kHz shared with the fixed service;

Sub-Section C-2	–	Table of recommended single-sideband transmitting frequencies (kHz) for ship and coast stations in the frequency band 8 100-8 195 kHz shared with the fixed service.

2	The technical characteristics for single-sideband transmitters specified in Recommendation ITU-R M.1173-1 shall be used.     (WRC-15)

3	One or more series of frequencies from Sub-Section A (with the exception of those frequencies mentioned in § 5 below) may be assigned to each coast station, which uses these frequencies associated in pairs (see No. 52.226); each pair consists of a transmitting and a receiving frequency. The series shall be selected with due regard to the areas served and so as to avoid, as far as possible, harmful interference between the services of different coast stations.

4	The frequencies in Sub-Section B are provided for worldwide common use by ships of all categories, according to traffic requirements, for ship transmissions to coast stations and for intership communication. They are also authorized for worldwide common use for transmissions by coast stations (simplex operation) provided the peak envelope power does not exceed 1 kW.

5	The following frequencies in Sub-Section A are allocated for calling purposes:

–	Channel No. 421 in the 4 MHz band;

–	Channel No. 606 in the 6 MHz band;

–	Channel No. 821 in the 8 MHz band;

–	Channel No. 1221 in the 12 MHz band;

–	Channel No. 1621 in the 16 MHz band;

–	Channel No. 1806 in the 18 MHz band;

–	Channel No. 2221 in the 22 MHz band;

–	Channel No. 2510 in the 25 MHz band.

Calling on the carrier frequencies 12 290 kHz and 16 420 kHz shall be permitted only to and from rescue coordination centres (see No. 30.6.1), subject to the safeguards of Resolution 352 (WRC-03) (see Nos. 52.221A and 52.222A).

The remaining frequencies in Sub-Sections A, B, C-1 and C-2 are working frequencies.     (WRC-03)

5A	For the use of the carrier frequencies:

4 125 kHz (Channel No. 421);
  6 215 kHz (Channel No. 606);
  8 291 kHz (Channel No. 833);
12 290 kHz (Channel No. 1221);
16 420 kHz (Channel No. 1621);

in Sub-Section A, by coast and ship stations for distress and safety purposes, see Article 31.    (WRC-07)

6	a)	Maritime radiotelephone stations using single-sideband emissions in the frequency bands between 4 000 kHz and 27 500 kHz exclusively allocated to the maritime mobile service shall operate only on the carrier frequencies shown in the Sub-Sections A and B and, in the case of analogue radiotelephony, shall be in conformity with the technical characteristics specified in Recommendation ITU-R M.1173-1.     (WRC-15)

b)	Ship stations, when using frequencies for single-sideband emissions in the frequency band 4 000-4 063 kHz, and ship and coast stations, when using frequencies for single-sideband emissions in the frequency band 8 100-8 195 kHz, should operate on the carrier frequencies indicated in Sub-Sections C-1 and C-2 respectively. In the case of analogue radiotelephony technical characteristics of the equipment shall be those specified in Recommendation ITU-R M.1173-1.     (WRC-15)

c)	Stations, when employing the single-sideband mode for analogue radiotelephony, shall use only class J3E emissions. For digital communications, class J2D emissions shall be used.     (WRC-03)

7	The channelling plan established in Sub-Section C-2 does not prejudice the rights of administrations to establish, and to notify assignments to stations in the maritime mobile service other than those using radiotelephony in the frequency band 8 100-8 195 kHz, in conformity with the relevant provisions of these Regulations.

8	(SUP - WRC-03)

Sub-Section A

Table of single-sideband transmitting frequencies (kHz) for duplex
(two-frequency) operation

|  | 4 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 401 | 4 357 | 4 358.4 | 4 065 | 4 066.4 |
| 402 | 4 360 | 4 361.4 | 4 068 | 4 069.4 |
| 403 | 4 363 | 4 364.4 | 4 071 | 4 072.4 |
| 404 | 4 366 | 4 367.4 | 4 074 | 4 075.4 |
| 405 | 4 369 | 4 370.4 | 4 077 | 4 078.4 |
| 406 | 4 372 | 4 373.4 | 4 080 | 4 081.4 |
| 407 | 4 375 | 4 376.4 | 4 083 | 4 084.4 |
| 408 | 4 378 | 4 379.4 | 4 086 | 4 087.4 |
| 409 | 4 381 | 4 382.4 | 4 089 | 4 090.4 |
| 410 | 4 384 | 4 385.4 | 4 092 | 4 093.4 |
| 411 | 4 387 | 4 388.4 | 4 095 | 4 096.4 |
| 412 | 4 390 | 4 391.4 | 4 098 | 4 099.4 |
| 413 | 4 393 | 4 394.4 | 4 101 | 4 102.4 |
| 414 | 4 396 | 4 397.4 | 4 104 | 4 105.4 |
| 415 | 4 399 | 4 400.4 | 4 107 | 4 108.4 |
| 416 | 4 402 | 4 403.4 | 4 110 | 4 111.4 |
| 417 | 4 405 | 4 406.4 | 4 113 | 4 114.4 |
| 418 | 4 408 | 4 409.4 | 4 116 | 4 117.4 |
| 419 | 4 411 | 4 412.4 | 4 119 | 4 120.4 |
| 420 | 4 414 | 4 415.4 | 4 122 | 4 123.4 |
| 421 | 4 417 * | 4 418.4 * | 4 125 * 4 | 4 126.4 * |
| 422 | 4 420 | 4 421.4 | 4 128 | 4 129.4 |
| 423 | 4 423 | 4 424.4 | 4 131 | 4 132.4 |
| 424 | 4 426 | 4 427.4 | 4 134 | 4 135.4 |
| 425 | 4 429 | 4 430.4 | 4 137 | 4 138.4 |
| 426 | 4 432 | 4 433.4 | 4 140 | 4 141.4 |
| 427 | 4 435 | 4 436.4 | 4 143 | 4 144.4 |
| 428 1, 3 | 4 351 | 4 352.4 | – | – |
| 429 1, 3 | 4 354 | 4 355.4 | – | – |

|  | 6 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 601 | 6 501 | 6 502.4 | 6 200 | 6 201.4 |
| 602 | 6 504 | 6 505.4 | 6 203 | 6 204.4 |
| 603 | 6 507 | 6 508.4 | 6 206 | 6 207.4 |
| 604 | 6 510 | 6 511.4 | 6 209 | 6 210.4 |
| 605 | 6 513 | 6 514.4 | 6 212 | 6 213.4 |
| 606 | 6 516 * | 6 517.4 * | 6 215 * 5 | 6 216.4 * |
| 607 | 6 519 | 6 520.4 | 6 218 | 6 219.4 |
| 608 | 6 522 | 6 523.4 | 6 221 | 6 222.4 |

|  | 8 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 801 | 8 719 | 8 720.4 | 8 195 | 8 196.4 |
| 802 | 8 722 | 8 723.4 | 8 198 | 8 199.4 |
| 803 | 8 725 | 8 726.4 | 8 201 | 8 202.4 |
| 804 | 8 728 | 8 729.4 | 8 204 | 8 205.4 |
| 805 | 8 731 | 8 732.4 | 8 207 | 8 208.4 |
| 806 | 8 734 | 8 735.4 | 8 210 | 8 211.4 |
| 807 | 8 737 | 8 738.4 | 8 213 | 8 214.4 |
| 808 | 8 740 | 8 741.4 | 8 216 | 8 217.4 |
| 809 | 8 743 | 8 744.4 | 8 219 | 8 220.4 |
| 810 | 8 746 | 8 747.4 | 8 222 | 8 223.4 |
| 811 | 8 749 | 8 750.4 | 8 225 | 8 226.4 |
| 812 | 8 752 | 8 753.4 | 8 228 | 8 229.4 |
| 813 | 8 755 | 8 756.4 | 8 231 | 8 232.4 |
| 814 | 8 758 | 8 759.4 | 8 234 | 8 235.4 |
| 815 | 8 761 | 8 762.4 | 8 237 | 8 238.4 |
| 816 | 8 764 | 8 765.4 | 8 240 | 8 241.4 |
| 817 | 8 767 | 8 768.4 | 8 243 | 8 244.4 |
| 818 | 8 770 | 8 771.4 | 8 246 | 8 247.4 |
| 819 | 8 773 | 8 774.4 | 8 249 | 8 250.4 |
| 820 | 8 776 | 8 777.4 | 8 252 | 8 253.4 |
| 821 | 8 779 * | 8 780.4 * | 8 255 * | 8 256.4 * |
| 822 | 8 782 | 8 783.4 | 8 258 | 8 259.4 |
| 823 | 8 785 | 8 786.4 | 8 261 | 8 262.4 |
| 824 | 8 788 | 8 789.4 | 8 264 | 8 265.4 |
| 825 | 8 791 | 8 792.4 | 8 267 | 8 268.4 |
| 826 | 8 794 | 8 795.4 | 8 270 | 8 271.4 |
| 827 | 8 797 | 8 798.4 | 8 273 | 8 274.4 |
| 828 | 8 800 | 8 801.4 | 8 276 | 8 277.4 |
| 829 | 8 803 | 8 804.4 | 8 279 | 8 280.4 |
| 830 | 8 806 | 8 807.4 | 8 282 | 8 283.4 |
| 831 | 8 809 | 8 810.4 | 8 285 | 8 286.4 |
| 832 | 8 812 | 8 813.4 | 8 288 | 8 289.4 |
| 833 | 8 291 7 | 8 292.4 | 8 291 7 | 8 292.4 |
| 834 3, 6 | 8 707 | 8 708.4 | – | – |
| 835 3, 6 | 8 710 | 8 711.4 | – | – |
| 836 3, 6 | 8 713 | 8 714.4 | – | – |
| 837 3, 6 | 8 716 | 8 717.4 | – | – |

|  | 12 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1201 | 13 077 | 13 078.4 | 12 230 | 12 231.4 |
| 1202 | 13 080 | 13 081.4 | 12 233 | 12 234.4 |
| 1203 | 13 083 | 13 084.4 | 12 236 | 12 237.4 |
| 1204 | 13 086 | 13 087.4 | 12 239 | 12 240.4 |
| 1205 | 13 089 | 13 090.4 | 12 242 | 12 243.4 |

|  | 12 MHz band (end) |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1206 | 13 092 | 13 093.4 | 12 245 | 12 246.4 |
| 1207 | 13 095 | 13 096.4 | 12 248 | 12 249.4 |
| 1208 | 13 098 | 13 099.4 | 12 251 | 12 252.4 |
| 1209 | 13 101 | 13 102.4 | 12 254 | 12 255.4 |
| 1210 | 13 104 | 13 105.4 | 12 257 | 12 258.4 |
| 1211 | 13 107 | 13 108.4 | 12 260 | 12 261.4 |
| 1212 | 13 110 | 13 111.4 | 12 263 | 12 264.4 |
| 1213 | 13 113 | 13 114.4 | 12 266 | 12 267.4 |
| 1214 | 13 116 | 13 117.4 | 12 269 | 12 270.4 |
| 1215 | 13 119 | 13 120.4 | 12 272 | 12 273.4 |
| 1216 | 13 122 | 13 123.4 | 12 275 | 12 276.4 |
| 1217 | 13 125 | 13 126.4 | 12 278 | 12 279.4 |
| 1218 | 13 128 | 13 129.4 | 12 281 | 12 282.4 |
| 1219 | 13 131 | 13 132.4 | 12 284 | 12 285.4 |
| 1220 | 13 134 | 13 135.4 | 12 287 | 12 288.4 |
| 1221 | 13 137 * | 13 138.4 * | 12 290 * 8 | 12 291.4 * |
| 1222 | 13 140 | 13 141.4 | 12 293 | 12 294.4 |
| 1223 | 13 143 | 13 144.4 | 12 296 | 12 297.4 |
| 1224 | 13 146 | 13 147.4 | 12 299 | 12 300.4 |
| 1225 | 13 149 | 13 150.4 | 12 302 | 12 303.4 |
| 1226 | 13 152 | 13 153.4 | 12 305 | 12 306.4 |
| 1227 | 13 155 | 13 156.4 | 12 308 | 12 309.4 |
| 1228 | 13 158 | 13 159.4 | 12 311 | 12 312.4 |
| 1229 | 13 161 | 13 162.4 | 12 314 | 12 315.4 |
| 1230 | 13 164 | 13 165.4 | 12 317 | 12 318.4 |
| 1231 | 13 167 | 13 168.4 | 12 320 | 12 321.4 |
| 1232 | 13 170 | 13 171.4 | 12 323 | 12 324.4 |
| 1233 | 13 173 | 13 174.4 | 12 326 | 12 327.4 |
| 1234 | 13 176 | 13 177.4 | 12 329 | 12 330.4 |
| 1235 | 13 179 | 13 180.4 | 12 332 | 12 333.4 |
| 1236 | 13 182 | 13 183.4 | 12 335 | 12 336.4 |
| 1237 | 13 185 | 13 186.4 | 12 338 | 12 339.4 |
| 1238 | 13 188 | 13 189.4 | 12 341 | 12 342.4 |
| 1239 | 13 191 | 13 192.4 | 12 344 | 12 345.4 |
| 1240 | 13 194 | 13 195.4 | 12 347 | 12 348.4 |
| 1241 | 13 197 | 13 198.4 | 12 350 | 12 351.4 |

|  | 16 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1601 | 17 242 | 17 243.4 | 16 360 | 16 361.4 |
| 1602 | 17 245 | 17 246.4 | 16 363 | 16 364.4 |
| 1603 | 17 248 | 17 249.4 | 16 366 | 16 367.4 |
| 1604 | 17 251 | 17 252.4 | 16 369 | 16 370.4 |
| 1605 | 17 254 | 17 255.4 | 16 372 | 16 373.4 |

|  | 16 MHz band (end ) |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1606 | 17 257 | 17 258.4 | 16 375 | 16 376.4 |
| 1607 | 17 260 | 17 261.4 | 16 378 | 16 379.4 |
| 1608 | 17 263 | 17 264.4 | 16 381 | 16 382.4 |
| 1609 | 17 266 | 17 267.4 | 16 384 | 16 385.4 |
| 1610 | 17 269 | 17 270.4 | 16 387 | 16 388.4 |
| 1611 | 17 272 | 17 273.4 | 16 390 | 16 391.4 |
| 1612 | 17 275 | 17 276.4 | 16 393 | 16 394.4 |
| 1613 | 17 278 | 17 279.4 | 16 396 | 16 397.4 |
| 1614 | 17 281 | 17 282.4 | 16 399 | 16 400.4 |
| 1615 | 17 284 | 17 285.4 | 16 402 | 16 403.4 |
| 1616 | 17 287 | 17 288.4 | 16 405 | 16 406.4 |
| 1617 | 17 290 | 17 291.4 | 16 408 | 16 409.4 |
| 1618 | 17 293 | 17 294.4 | 16 411 | 16 412.4 |
| 1619 | 17 296 | 17 297.4 | 16 414 | 16 415.4 |
| 1620 | 17 299 | 17 300.4 | 16 417 | 16 418.4 |
| 1621 | 17 302 * | 17 303.4 * | 16 420 * 9 | 16 421.4 * |
| 1622 | 17 305 | 17 306.4 | 16 423 | 16 424.4 |
| 1623 | 17 308 | 17 309.4 | 16 426 | 16 427.4 |
| 1624 | 17 311 | 17 312.4 | 16 429 | 16 430.4 |
| 1625 | 17 314 | 17 315.4 | 16 432 | 16 433.4 |
| 1626 | 17 317 | 17 318.4 | 16 435 | 16 436.4 |
| 1627 | 17 320 | 17 321.4 | 16 438 | 16 439.4 |
| 1628 | 17 323 | 17 324.4 | 16 441 | 16 442.4 |
| 1629 | 17 326 | 17 327.4 | 16 444 | 16 445.4 |
| 1630 | 17 329 | 17 330.4 | 16 447 | 16 448.4 |
| 1631 | 17 332 | 17 333.4 | 16 450 | 16 451.4 |
| 1632 | 17 335 | 17 336.4 | 16 453 | 16 454.4 |
| 1633 | 17 338 | 17 339.4 | 16 456 | 16 457.4 |
| 1634 | 17 341 | 17 342.4 | 16 459 | 16 460.4 |
| 1635 | 17 344 | 17 345.4 | 16 462 | 16 463.4 |
| 1636 | 17 347 | 17 348.4 | 16 465 | 16 466.4 |
| 1637 | 17 350 | 17 351.4 | 16 468 | 16 469.4 |
| 1638 | 17 353 | 17 354.4 | 16 471 | 16 472.4 |
| 1639 | 17 356 | 17 357.4 | 16 474 | 16 475.4 |
| 1640 | 17 359 | 17 360.4 | 16 477 | 16 478.4 |
| 1641 | 17 362 | 17 363.4 | 16 480 | 16 481.4 |
| 1642 | 17 365 | 17 366.4 | 16 483 | 16 484.4 |
| 1643 | 17 368 | 17 369.4 | 16 486 | 16 487.4 |
| 1644 | 17 371 | 17 372.4 | 16 489 | 16 490.4 |
| 1645 | 17 374 | 17 375.4 | 16 492 | 16 493.4 |
| 1646 | 17 377 | 17 378.4 | 16 495 | 16 496.4 |
| 1647 | 17 380 | 17 381.4 | 16 498 | 16 499.4 |
| 1648 | 17 383 | 17 384.4 | 16 501 | 16 502.4 |
| 1649 | 17 386 | 17 387.4 | 16 504 | 16 505.4 |
| 1650 | 17 389 | 17 390.4 | 16 507 | 16 508.4 |
| 1651 | 17 392 | 17 393.4 | 16 510 | 16 511.4 |
| 1652 | 17 395 | 17 396.4 | 16 513 | 16 514.4 |
| 1653 | 17 398 | 17 399.4 | 16 516 | 16 517.4 |
| 1654 | 17 401 | 17 402.4 | 16 519 | 16 520.4 |
| 1655 | 17 404 | 17 405.4 | 16 522 | 16 523.4 |
| 1656 | 17 407 | 17 408.4 | 16 525 | 16 526.4 |

|  | 18/19 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1801 | 19 755 | 19 756.4 | 18 780 | 18 781.4 |
| 1802 | 19 758 | 19 759.4 | 18 783 | 18 784.4 |
| 1803 | 19 761 | 19 762.4 | 18 786 | 18 787.4 |
| 1804 | 19 764 | 19 765.4 | 18 789 | 18 790.4 |
| 1805 | 19 767 | 19 768.4 | 18 792 | 18 793.4 |
| 1806 | 19 770 * | 19 771.4 * | 18 795 * | 18 796.4 * |
| 1807 | 19 773 | 19 774.4 | 18 798 | 18 799.4 |
| 1808 | 19 776 | 19 777.4 | 18 801 | 18 802.4 |
| 1809 | 19 779 | 19 780.4 | 18 804 | 18 805.4 |
| 1810 | 19 782 | 19 783.4 | 18 807 | 18 808.4 |
| 1811 | 19 785 | 19 786.4 | 18 810 | 18 811.4 |
| 1812 | 19 788 | 19 789.4 | 18 813 | 18 814.4 |
| 1813 | 19 791 | 19 792.4 | 18 816 | 18 817.4 |
| 1814 | 19 794 | 19 795.4 | 18 819 | 18 820.4 |
| 1815 | 19 797 | 19 798.4 | 18 822 | 18 823.4 |

|  | 22 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2201 | 22 696 | 22 697.4 | 22 000 | 22 001.4 |
| 2202 | 22 699 | 22 700.4 | 22 003 | 22 004.4 |
| 2203 | 22 702 | 22 703.4 | 22 006 | 22 007.4 |
| 2204 | 22 705 | 22 706.4 | 22 009 | 22 010.4 |
| 2205 | 22 708 | 22 709.4 | 22 012 | 22 013.4 |
| 2206 | 22 711 | 22 712.4 | 22 015 | 22 016.4 |
| 2207 | 22 714 | 22 715.4 | 22 018 | 22 019.4 |
| 2208 | 22 717 | 22 718.4 | 22 021 | 22 022.4 |
| 2209 | 22 720 | 22 721.4 | 22 024 | 22 025.4 |
| 2210 | 22 723 | 22 724.4 | 22 027 | 22 028.4 |
| 2211 | 22 726 | 22 727.4 | 22 030 | 22 031.4 |
| 2212 | 22 729 | 22 730.4 | 22 033 | 22 034.4 |
| 2213 | 22 732 | 22 733.4 | 22 036 | 22 037.4 |
| 2214 | 22 735 | 22 736.4 | 22 039 | 22 040.4 |
| 2215 | 22 738 | 22 739.4 | 22 042 | 22 043.4 |
| 2216 | 22 741 | 22 742.4 | 22 045 | 22 046.4 |
| 2217 | 22 744 | 22 745.4 | 22 048 | 22 049.4 |
| 2218 | 22 747 | 22 748.4 | 22 051 | 22 052.4 |
| 2219 | 22 750 | 22 751.4 | 22 054 | 22 055.4 |
| 2220 | 22 753 | 22 754.4 | 22 057 | 22 058.4 |
| 2221 | 22 756 * | 22 757.4 * | 22 060 * | 22 061.4 * |
| 2222 | 22 759 | 22 760.4 | 22 063 | 22 064.4 |
| 2223 | 22 762 | 22 763.4 | 22 066 | 22 067.4 |
| 2224 | 22 765 | 22 766.4 | 22 069 | 22 070.4 |
| 2225 | 22 768 | 22 769.4 | 22 072 | 22 073.4 |

|  | 22 MHz band (end ) |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2226 | 22 771 | 22 772.4 | 22 075 | 22 076.4 |
| 2227 | 22 774 | 22 775.4 | 22 078 | 22 079.4 |
| 2228 | 22 777 | 22 778.4 | 22 081 | 22 082.4 |
| 2229 | 22 780 | 22 781.4 | 22 084 | 22 085.4 |
| 2230 | 22 783 | 22 784.4 | 22 087 | 22 088.4 |
| 2231 | 22 786 | 22 787.4 | 22 090 | 22 091.4 |
| 2232 | 22 789 | 22 790.4 | 22 093 | 22 094.4 |
| 2233 | 22 792 | 22 793.4 | 22 096 | 22 097.4 |
| 2234 | 22 795 | 22 796.4 | 22 099 | 22 100.4 |
| 2235 | 22 798 | 22 799.4 | 22 102 | 22 103.4 |
| 2236 | 22 801 | 22 802.4 | 22 105 | 22 106.4 |
| 2237 | 22 804 | 22 805.4 | 22 108 | 22 109.4 |
| 2238 | 22 807 | 22 808.4 | 22 111 | 22 112.4 |
| 2239 | 22 810 | 22 811.4 | 22 114 | 22 115.4 |
| 2240 | 22 813 | 22 814.4 | 22 117 | 22 118.4 |
| 2241 | 22 816 | 22 817.4 | 22 120 | 22 121.4 |
| 2242 | 22 819 | 22 820.4 | 22 123 | 22 124.4 |
| 2243 | 22 822 | 22 823.4 | 22 126 | 22 127.4 |
| 2244 | 22 825 | 22 826.4 | 22 129 | 22 130.4 |
| 2245 | 22 828 | 22 829.4 | 22 132 | 22 133.4 |
| 2246 | 22 831 | 22 832.4 | 22 135 | 22 136.4 |
| 2247 | 22 834 | 22 835.4 | 22 138 | 22 139.4 |
| 2248 | 22 837 | 22 838.4 | 22 141 | 22 142.4 |
| 2249 | 22 840 | 22 841.4 | 22 144 | 22 145.4 |
| 2250 | 22 843 | 22 844.4 | 22 147 | 22 148.4 |
| 2251 | 22 846 | 22 847.4 | 22 150 | 22 151.4 |
| 2252 | 22 849 | 22 850.4 | 22 153 | 22 154.4 |
| 2253 | 22 852 | 22 853.4 | 22 156 | 22 157.4 |

|  | 25/26 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
| Channel No. | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2501 | 26 145 | 26 146.4 | 25 070 | 25 071.4 |
| 2502 | 26 148 | 26 149.4 | 25 073 | 25 074.4 |
| 2503 | 26 151 | 26 152.4 | 25 076 | 25 077.4 |
| 2504 | 26 154 | 26 155.4 | 25 079 | 25 080.4 |
| 2505 | 26 157 | 26 158.4 | 25 082 | 25 083.4 |
| 2506 | 26 160 | 26 161.4 | 25 085 | 25 086.4 |
| 2507 | 26 163 | 26 164.4 | 25 088 | 25 089.4 |
| 2508 | 26 166 | 26 167.4 | 25 091 | 25 092.4 |
| 2509 | 26 169 | 26 170.4 | 25 094 | 25 095.4 |
| 2510 | 26 172 * | 26 173.4 * | 25 097 * | 25 098.4 * |

1	These coast station frequencies may be paired with a ship station frequency from the Table of simplex frequencies for ship and coast stations (see Sub-Section B) or with a frequency from the band 4 000-4 063 kHz (see Sub-Section C-1) to be selected by the administration concerned.

2	(SUP - WRC-2000)

3	These channels may also be used for simplex (single frequency) operation.

4	For the conditions of use of the carrier frequency 4 125 kHz, see Nos. 52.224 and 52.225, and Appendix 15.

5	For the conditions of use of the carrier frequency 6 215 kHz, see Appendix 15.     (WRC-07)

6	These coast station frequencies may be paired with a ship station frequency from the Table of simplex frequencies for ship and coast stations (see Sub-Section B) or with a frequency from the band 8 100-8 195 kHz (see Sub-Section C-2) to be selected by the administration concerned.

7	For the conditions of use of the carrier frequency 8 291 kHz, see Appendix 15.

8	For the conditions of use of the carrier frequency 12 290 kHz, see Nos. 52.221A and 52.222A and Appendix 15.     (WRC-2000)

9	For the conditions of use of the carrier frequency 16 420 kHz, see Nos. 52.221A and 52.222A and Appendix 15.     (WRC-2000)

*	The frequencies followed by an asterisk are calling frequencies (see Nos. 52.221 and 52.222).

Sub-Section B

Table of single-sideband transmitting frequencies (kHz) for simplex (single-frequency) 
operation and for intership cross-band (two-frequency) operation

(See § 4 of Section I of this Appendix)

| 4 MHz band1 |  | 6 MHz band |  | 8 MHz band2 |  | 12 MHz band3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 4 146 4 149 | 4 147.4 4 150.4 | 6 224 6 227 6 230 | 6 225.4 6 228.4 6 231.4 | 8 294 8 297 | 8 295.4 8 298.4 | 12 353 12 356 12 362 12 365 | 12 354.4 12 357.4 12 363.4 12 366.4 |
| 1 These frequencies may be used for duplex operation with coast stations operating on Channel Nos. 428 and 429 (see Sub-Section A). 2 These frequencies may be used for duplex operation with coast stations operating on Channel Nos. 834 up to and including 837 (see Sub-Section A). 3 For use of frequencies 12 359 kHz and 16 537 kHz, see Nos. 52.221A and 52.222A. (WRC-2000) |  |  |  |  |  |  |  |  |

| 16 MHz band3 |  | 18/19 MHz band |  | 22 MHz band |  | 25/26 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 16 528 16 531 16 534 16 540 16 543 16 546 | 16 529.4 16 532.4 16 535.4 16 541.4 16 544.4 16 547.4 | 18 825 18 828 18 831 18 834 18 837 18 840 18 843 | 18 826.4 18 829.4 18 832.4 18 835.4 18 838.4 18 841.4 18 844.4 | 22 159 22 162 22 165 22 168 22 171 22 174 22 177 | 22 160.4 22 163.4 22 166.4 22 169.4 22 172.4 22 175.4 22 178.4 | 25 100 25 103 25 106 25 109 25 112 25 115 25 118 | 25 101.4 25 104.4 25 107.4 25 110.4 25 113.4 25 116.4 25 119.4 |
| 3 For use of frequencies 12 359 kHz and 16 537 kHz, see Nos. 52.221A and 52.222A. (WRC-2000) |  |  |  |  |  |  |  |  |

Sub-Section C-1

Table of recommended single-sideband transmitting frequencies (kHz) for 
ship stations in the band 4 000-4 063 kHz shared with the fixed service

The frequencies in this Sub-Section may be used:

–	for supplementing ship-to-shore channels for duplex operation in Sub-Section A;

–	for intership simplex (single-frequency) and cross-band operation;

–	for cross-band working with coast stations on channels in Sub-Section C-2;

–	for duplex operation with coast stations working in the band 4 438-4 650 kHz;

–	for duplex operation with Channel Nos. 428 and 429.

| Channel No. | Carrier frequency | Assigned frequency | Channel No. | Carrier frequency | Assigned frequency |
| --- | --- | --- | --- | --- | --- |
| 1 | 4 000* | 4 001.4* | 12 | 4 033 | 4 034.4 |
| 2 | 4 003* | 4 004.4* | 13 | 4 036 | 4 037.4 |
| 3 | 4 006 | 4 007.4 | 14 | 4 039 | 4 040.4 |
| 4 | 4 009 | 4 010.4 | 15 | 4 042 | 4 043.4 |
| 5 | 4 012 | 4 013.4 | 16 | 4 045 | 4 046.4 |
| 6 | 4 015 | 4 016.4 | 17 | 4 048 | 4 049.4 |
| 7 | 4 018 | 4 019.4 | 18 | 4 051 | 4 052.4 |
| 8 | 4 021 | 4 022.4 | 19 | 4 054 | 4 055.4 |
| 9 | 4 024 | 4 025.4 | 20 | 4 057 | 4 058.4 |
| 10 | 4 027 | 4 028.4 | 21 | 4 060 | 4 061.4 |
| 11 | 4 030 | 4 031.4 |  |  |  |
| * Administrations are requested to urge ship stations under their jurisdiction to refrain from using the band 4 000-4 005 kHz when navigating in Region 3 (see also No. 5.126). |  |  |  |  |  |  |

Sub-Section C-2

Table of recommended single-sideband transmitting frequencies (kHz) for ship 
and coast stations in the band 8 100-8 195 kHz shared with the fixed service

(See § 7 of Section I of this Appendix)

The frequencies in this Sub-Section may be used:

–	for supplementing ship-to-shore and shore-to-ship channels for duplex operation in Sub-Section A;

–	for intership simplex (single frequency) and cross-band operation;

–	for cross-band working with ship stations on channels in Sub-Section C-1;

–	for ship-to-shore or shore-to-ship simplex operation;

–	for duplex operation with Channel Nos. 834, 835, 836 and 837.

| Channel No. | Carrier frequency | Assigned frequency | Channel No. | Carrier frequency | Assigned frequency |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 101 | 8 102.4 | 17 | 8 149 | 8 150.4 |
| 2 | 8 104 | 8 105.4 | 18 | 8 152 | 8 153.4 |
| 3 | 8 107 | 8 108.4 | 19 | 8 155 | 8 156.4 |
| 4 | 8 110 | 8 111.4 | 20 | 8 158 | 8 159.4 |
| 5 | 8 113 | 8 114.4 | 21 | 8 161 | 8 162.4 |
| 6 | 8 116 | 8 117.4 | 22 | 8 164 | 8 165.4 |
| 7 | 8 119 | 8 120.4 | 23 | 8 167 | 8 168.4 |
| 8 | 8 122 | 8 123.4 | 24 | 8 170 | 8 171.4 |
| 9 | 8 125 | 8 126.4 | 25 | 8 173 | 8 174.4 |
| 10 | 8 128 | 8 129.4 | 26 | 8 176 | 8 177.4 |
| 11 | 8 131 | 8 132.4 | 27 | 8 179 | 8 180.4 |
| 12 | 8 134 | 8 135.4 | 28 | 8 182 | 8 183.4 |
| 13 | 8 137 | 8 138.4 | 29 | 8 185 | 8 186.4 |
| 14 | 8 140 | 8 141.4 | 30 | 8 188 | 8 189.4 |
| 15 | 8 143 | 8 144.4 | 31 | 8 191 | 8 192.4 |
| 16 | 8 146 | 8 147.4 |  |  |  |

Section II  –  Narrow-band direct-printing telegraphy (paired frequencies)

1	Each coast station which uses paired frequencies is assigned one or more frequency pairs from the following series; each pair consists of a transmitting and a receiving frequency.

2	The speed of the narrow-band direct-printing telegraphy and data systems shall not exceed 100 Bd for FSK and 200 Bd for PSK.

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 4 MHz band 1 |  | 6 MHz band 3 |  | 8 MHz band 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive | Transmit | Receive |  |
| 1 2 3 4 5 | 4 210.5 4 211 4 211.5 4 212 4 212.5 | 4 172.5 4 173 4 173.5 4 174 4 174.5 | 6 314.5 6 315 6 315.5 6 316 6 316.5 | 6 263 6 263.5 6 264 6 264.5 6 265 | 8 376.5 2 8 417 8 417.5 8 418 8 418.5 | 8 376.5 2 8 377 8 377.5 8 378 8 378.5 |  |
| 6 7 8 9 10 | 4 213 4 213.5 4 214 4 214.5 4 215 | 4 175 4 175.5 4 176 4 176.5 4 177 | 6 317 6 317.5 6 318 6 318.5 6 319 | 6 265.5 6 266 6 266.5 6 267 6 267.5 | 8 419 8 419.5 8 420 8 420.5 8 421 | 8 379 8 379.5 8 380 8 380.5 8 381 |  |
| 11 12 13 14 15 | 4 177.5 2 4 215.5 4 216 4 216.5 4 217 | 4 177.5 2 4 178 4 178.5 4 179 4 179.5 | 6 268 2 6 319.5 6 320 6 320.5 6 321 | 6 268 2 6 268.5 6 269 6 269.5 6 270 | 8 421.5 8 422 8 422.5 8 423 8 423.5 | 8 381.5 8 382 8 382.5 8 383 8 383.5 |  |
| 16 17 18 19 20 | 4 217.5 4 218 4 218.5 4 219 | 4 180 4 180.5 4 181 4 181.5 | 6 321.5 6 322 6 322.5 6 323 6 323.5 | 6 270.5 6 271 6 271.5 6 272 6 272.5 | 8 424 8 424.5 8 425 8 425.5 8 426 | 8 384 8 384.5 8 385 8 385.5 8 386 |  |
| 21 22 23 24 25 |  |  | 6 324 6 324.5 6 325 6 325.5 6 326 | 6 273 6 273.5 6 274 6 274.5 6 275 | 8 426.5 8 427 8 427.5 8 428 8 428.5 | 8 386.5 8 387 8 387.5 8 388 8 388.5 |  |
| 26 27 28 29 30 |  |  | 6 326.5 6 327 6 327.5 6 328 6 328.5 | 6 275.5 6 281 6 281.5 6 282 6 282.5 | 8 429 8 429.5 8 430 8 430.5 8 431 | 8 389 8 389.5 8 390 8 390.5 8 391 |  |
| 31 32 33 34 35 |  |  | 6 329 6 329.5 6 330 6 330.5 | 6 283 6 283.5 6 284 6 284.5 | 8 431.5 8 432 8 432.5 8 433 8 433.5 | 8 391.5 8 392 8 392.5 8 393 8 393.5 |  |
| 36 37 38 39 40 |  |  |  |  | 8 434 8 434.5 8 435 8 435.5 8 436 | 8 394 8 394.5 8 395 8 395.5 8 396 |  |
| 1 Ship stations may use the coast station receiving frequencies for transmitting A1A or A1B Morse telegraphy (working), with the exception of channel No. 11 (see Appendix 15). 2 For the conditions of use of this frequency, see Article 31. 3 Ship stations may use the coast station receiving frequencies of channel Nos. 25 up to and including 34 for transmitting A1A or A1B Morse telegraphy (working). 4 Ship stations may use the coast station receiving frequencies of channel Nos. 29 up to and including 40 for transmitting A1A or A1B Morse telegraphy (working). |  |  |  |  |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 12 MHz band 5 |  | 16 MHz band 6 |  | 18/19 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive | Transmit | Receive |
| 1 2 3 4 5 | 12 579.5 12 580 12 580.5 12 581 12 581.5 | 12 477 12 477.5 12 478 12 478.5 12 479 | 16 807 16 807.5 16 808 16 808.5 16 809 | 16 683.5 16 684 16 684.5 16 685 16 685.5 | 19 681 19 681.5 19 682 19 682.5 19 683 | 18 870.5 18 871 18 871.5 18 872 18 872.5 |
| 6 7 8 9 10 | 12 582 12 582.5 12 583 12 583.5 12 584 | 12 479.5 12 480 12 480.5 12 481 12 481.5 | 16 809.5 16 810 16 810.5 16 811 16 811.5 | 16 686 16 686.5 16 687 16 687.5 16 688 | 19 683.5 19 684 19 684.5 19 685 19 685.5 | 18 873 18 873.5 18 874 18 874.5 18 875 |
| 11 12 13 14 15 | 12 584.5 12 585 12 585.5 12 586 12 586.5 | 12 482 12 482.5 12 483 12 483.5 12 484 | 16 812 16 812.5 16 813 16 813.5 16 814 | 16 688.5 16 689 16 689.5 16 690 16 690.5 | 19 686 19 686.5 19 687 19 687.5 19 688 | 18 875.5 18 876 18 876.5 18 877 18 877.5 |
| 16 17 18 19 20 | 12 587 12 587.5 12 588 12 588.5 12 589 | 12 484.5 12 485 12 485.5 12 486 12 486.5 | 16 814.5 16 815 16 815.5 16 816 16 816.5 | 16 691 16 691.5 16 692 16 692.5 16 693 | 19 688.5 19 689 19 689.5 19 690 19 690.5 | 18 878 18 878.5 18 879 18 879.5 18 880 |
| 21 22 23 24 25 | 12 589.5 12 590 12 590.5 12 591 12 591.5 | 12 487 12 487.5 12 488 12 488.5 12 489 | 16 817 16 817.5 16 818 16 695 2 16 818.5 | 16 693.5 16 694 16 694.5 16 695 2 16 695.5 | 19 691 19 691.5 19 692 19 692.5 19 693 | 18 880.5 18 881 18 881.5 18 882 18 882.5 |
| 26 27 28 29 30 | 12 592 12 592.5 12 593 12 593.5 12 594 | 12 489.5 12 490 12 490.5 12 491 12 491.5 | 16 819 16 819.5 16 820 16 820.5 16 821 | 16 696 16 696.5 16 697 16 697.5 16 698 | 19 693.5 19 694 19 694.5 19 695 19 695.5 | 18 883 18 883.5 18 884 18 884.5 18 885 |
| 31 32 33 34 35 | 12 594.5 12 595 12 595.5 12 596 12 596.5 | 12 492 12 492.5 12 493 12 493.5 12 494 | 16 821.5 16 822 16 822.5 16 823 16 823.5 | 16 698.5 16 699 16 699.5 16 700 16 700.5 | 19 696 19 696.5 19 697 19 697.5 19 698 | 18 885.5 18 886 18 886.5 18 887 18 887.5 |
| 36 37 38 39 40 | 12 597 12 597.5 12 598 12 598.5 12 599 | 12 494.5 12 495 12 495.5 12 496 12 496.5 | 16 824 16 824.5 16 825 16 825.5 16 826 | 16 701 16 701.5 16 702 16 702.5 16 703 | 19 698.5 19 699 19 699.5 19 700 19 700.5 | 18 888 18 888.5 18 889 18 889.5 18 890 |
| 41 42 43 44 45 | 12 599.5 12 600 12 600.5 12 601 12 601.5 | 12 497 12 497.5 12 498 12 498.5 12 499 | 16 826.5 16 827 16 827.5 16 828 16 828.5 | 16 703.5 16 704 16 704.5 16 705 16 705.5 | 19 701 19 701.5 19 702 19 702.5 19 703 | 18 890.5 18 891 18 891.5 18 892 18 892.5 |
| 5 Ship stations may use the coast station receiving frequencies of channel Nos. 58 up to and including 156 for transmitting A1A or A1B Morse telegraphy (working), with the exception of channel No. 87 (see Appendix 15). 6 Ship stations may use the coast station receiving frequencies of channel Nos. 71 up to and including 193 for transmitting A1A or A1B Morse telegraphy (working). |  |  |  |  |  |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 12 MHz band 5 (cont.) |  | 16 MHz band 6 (cont.) |  |
| --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |
| 46 47 48 49 50 | 12 602 12 602.5 12 603 12 603.5 12 604 | 12 499.5 12 500 12 500.5 12 501 12 501.5 | 16 829 16 829.5 16 830 16 830.5 16 831 | 16 706 16 706.5 16 707 16 707.5 16 708 |
| 51 52 53 54 55 | 12 604.5 12 605 12 605.5 12 606 12 606.5 | 12 502 12 502.5 12 503 12 503.5 12 504 | 16 831.5 16 832 16 832.5 16 833 16 833.5 | 16 708.5 16 709 16 709.5 16 710 16 710.5 |
| 56 57 58 59 60 | 12 607 12 607.5 12 608 12 608.5 12 609 | 12 504.5 12 505 12 505.5 12 506 12 506.5 | 16 834 16 834.5 16 835 16 835.5 16 836 | 16 711 16 711.5 16 712 16 712.5 16 713 |
| 61 62 63 64 65 | 12 609.5 12 610 12 610.5 12 611 12 611.5 | 12 507 12 507.5 12 508 12 508.5 12 509 | 16 836.5 16 837 16 837.5 16 838 16 838.5 | 16 713.5 16 714 16 714.5 16 715 16 715.5 |
| 66 67 68 69 70 | 12 612 12 612.5 12 613 12 613.5 12 614 | 12 509.5 12 510 12 510.5 12 511 12 511.5 | 16 839 16 839.5 16 840 16 840.5 16 841 | 16 716 16 716.5 16 717 16 717.5 16 718 |
| 71 72 73 74 75 | 12 614.5 12 615 12 615.5 12 616 12 616.5 | 12 512 12 512.5 12 513 12 513.5 12 514 | 16 841.5 16 842 16 842.5 16 843 16 843.5 | 16 718.5 16 719 16 719.5 16 720 16 720.5 |
| 76 77 78 79 80 | 12 617 12 617.5 12 618 12 618.5 12 619 | 12 514.5 12 515 12 515.5 12 516 12 516.5 | 16 844 16 844.5 16 845 16 845.5 16 846 | 16 721 16 721.5 16 722 16 722.5 16 723 |
| 81 82 83 84 85 | 12 619.5 12 620 12 620.5 12 621 12 621.5 | 12 517 12 517.5 12 518 12 518.5 12 519 | 16 846.5 16 847 16 847.5 16 848 16 848.5 | 16 723.5 16 724 16 724.5 16 725 16 725.5 |
| 86 87 88 89 90 | 12 622 12 520 2 12 622.5 12 623 12 623.5 | 12 519.5 12 520 2 12 520.5 12 521 12 521.5 | 16 849 16 849.5 16 850 16 850.5 16 851 | 16 726 16 726.5 16 727 16 727.5 16 728 |
| 91 92 93 94 95 | 12 624 12 624.5 12 625 12 625.5 12 626 | 12 522 12 522.5 12 523 12 523.5 12 524 | 16 851.5 16 852 16 852.5 16 853 16 853.5 | 16 728.5 16 729 16 729.5 16 730 16 730.5 |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 12 MHz band 5 (cont.) |  | 16 MHz band 6 (cont.) |  |
| --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |
| 96 97 98 99 100 | 12 626.5 12 627 12 627.5 12 628 12 628.5 | 12 524.5 12 525 12 525.5 12 526 12 526.5 | 16 854 16 854.5 16 855 16 855.5 16 856 | 16 731 16 731.5 16 732 16 732.5 16 733 |
| 101 102 103 104 105 | 12 629 12 629.5 12 630 12 630.5 12 631 | 12 527 12 527.5 12 528 12 528.5 12 529 | 16 856.5 16 857 16 857.5 16 858 16 858.5 | 16 733.5 16 739 16 739.5 16 740 16 740.5 |
| 106 107 108 109 110 | 12 631.5 12 632 12 632.5 12 633 12 633.5 | 12 529.5 12 530 12 530.5 12 531 12 531.5 | 16 859 16 859.5 16 860 16 860.5 16 861 | 16 741 16 741.5 16 742 16 742.5 16 743 |
| 111 112 113 114 115 | 12 634 12 634.5 12 635 12 635.5 12 636 | 12 532 12 532.5 12 533 12 533.5 12 534 | 16 861.5 16 862 16 862.5 16 863 16 863.5 | 16 743.5 16 744 16 744.5 16 745 16 745.5 |
| 116 117 118 119 120 | 12 636.5 12 637 12 637.5 12 638 12 638.5 | 12 534.5 12 535 12 535.5 12 536 12 536.5 | 16 864 16 864.5 16 865 16 865.5 16 866 | 16 746 16 746.5 16 747 16 747.5 16 748 |
| 121 122 123 124 125 | 12 639 12 639.5 12 640 12 640.5 12 641 | 12 537 12 537.5 12 538 12 538.5 12 539 | 16 866.5 16 867 16 867.5 16 868 16 868.5 | 16 748.5 16 749 16 749.5 16 750 16 750.5 |
| 126 127 128 129 130 | 12 641.5 12 642 12 642.5 12 643 12 643.5 | 12 539.5 12 540 12 540.5 12 541 12 541.5 | 16 869 16 869.5 16 870 16 870.5 16 871 | 16 751 16 751.5 16 752 16 752.5 16 753 |
| 131 132 133 134 135 | 12 644 12 644.5 12 645 12 645.5 12 646 | 12 542 12 542.5 12 543 12 543.5 12 544 | 16 871.5 16 872 16 872.5 16 873 16 873.5 | 16 753.5 16 754 16 754.5 16 755 16 755.5 |
| 136 137 138 139 140 | 12 646.5 12 647 12 647.5 12 648 12 648.5 | 12 544.5 12 545 12 545.5 12 546 12 546.5 | 16 874 16 874.5 16 875 16 875.5 16 876 | 16 756 16 756.5 16 757 16 757.5 16 758 |
| 141 142 143 144 145 | 12 649 12 649.5 12 650 12 650.5 12 651 | 12 547 12 547.5 12 548 12 548.5 12 549 | 16 876.5 16 877 16 877.5 16 878 16 878.5 | 16 758.5 16 759 16 759.5 16 760 16 760.5 |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 12 MHz band 5 (end ) |  | 16 MHz band 6 (end ) |  |
| --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |
| 146 147 148 149 150 | 12 651.5 12 652 12 652.5 12 653 12 653.5 | 12 549.5 12 555 12 555.5 12 556 12 556.5 | 16 879 16 879.5 16 880 16 880.5 16 881 | 16 761 16 761.5 16 762 16 762.5 16 763 |
| 151 152 153 154 155 | 12 654 12 654.5 12 655 12 655.5 12 656 | 12 557 12 557.5 12 558 12 558.5 12 559 | 16 881.5 16 882 16 882.5 16 883 16 883.5 | 16 763.5 16 764 16 764.5 16 765 16 765.5 |
| 156 157 158 159 160 | 12 656.5 | 12 559.5 | 16 884 16 884.5 16 885 16 885.5 16 886 | 16 766 16 766.5 16 767 16 767.5 16 768 |
| 161 162 163 164 165 |  |  | 16 886.5 16 887 16 887.5 16 888 16 888.5 | 16 768.5 16 769 16 769.5 16 770 16 770.5 |
| 166 167 168 169 170 |  |  | 16 889 16 889.5 16 890 16 890.5 16 891 | 16 771 16 771.5 16 772 16 772.5 16 773 |
| 171 172 173 174 175 |  |  | 16 891.5 16 892 16 892.5 16 893 16 893.5 | 16 773.5 16 774 16 774.5 16 775 16 775.5 |
| 176 177 178 179 180 |  |  | 16 894 16 894.5 16 895 16 895.5 16 896 | 16 776 16 776.5 16 777 16 777.5 16 778 |
| 181 182 183 184 185 |  |  | 16 896.5 16 897 16 897.5 16 898 16 898.5 | 16 778.5 16 779 16 779.5 16 780 16 780.5 |
| 186 187 188 189 190 |  |  | 16 899 16 899.5 16 900 16 900.5 16 901 | 16 781 16 781.5 16 782 16 782.5 16 783 |
| 191 192 193 |  |  | 16 901.5 16 902 16 902.5 | 16 783.5 16 784 16 784.5 |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel | 22 MHz band 7 |  | 25/26 MHz band |  |
| --- | --- | --- | --- | --- |
| No. | Transmit | Receive | Transmit | Receive |
| 1 2 3 4 5 | 22 376.5 22 377 22 377.5 22 378 22 378.5 | 22 284.5 22 285 22 285.5 22 286 22 286.5 | 26 101 26 101.5 26 102 26 102.5 26 103 | 25 173 25 173.5 25 174 25 174.5 25 175 |
| 6 7 8 9 10 | 22 379 22 379.5 22 380 22 380.5 22 381 | 22 287 22 287.5 22 288 22 288.5 22 289 | 26 103.5 26 104 26 104.5 26 105 26 105.5 | 25 175.5 25 176 25 176.5 25 177 25 177.5 |
| 11 12 13 14 15 | 22 381.5 22 382 22 382.5 22 383 22 383.5 | 22 289.5 22 290 22 290.5 22 291 22 291.5 | 26 106 26 106.5 26 107 26 107.5 26 108 | 25 178 25 178.5 25 179 25 179.5 25 180 |
| 16 17 18 19 20 | 22 384 22 384.5 22 385 22 385.5 22 386 | 22 292 22 292.5 22 293 22 293.5 22 294 | 26 108.5 26 109 26 109.5 26 110 26 110.5 | 25 180.5 25 181 25 181.5 25 182 25 182.5 |
| 21 22 23 24 25 | 22 386.5 22 387 22 387.5 22 388 22 388.5 | 22 294.5 22 295 22 295.5 22 296 22 296.5 | 26 111 26 111.5 26 112 26 112.5 26 113 | 25 183 25 183.5 25 184 25 184.5 25 185 |
| 26 27 28 29 30 | 22 389 22 389.5 22 390 22 390.5 22 391 | 22 297 22 297.5 22 298 22 298.5 22 299 | 26 113.5 26 114 26 114.5 26 115 26 115.5 | 25 185.5 25 186 25 186.5 25 187 25 187.5 |
| 31 32 33 34 35 | 22 391.5 22 392 22 392.5 22 393 22 393.5 | 22 299.5 22 300 22 300.5 22 301 22 301.5 | 26 116 26 116.5 26 117 26 117.5 26 118 | 25 188 25 188.5 25 189 25 189.5 25 190 |
| 36 37 38 39 40 | 22 394 22 394.5 22 395 22 395.5 22 396 | 22 302 22 302.5 22 303 22 303.5 22 304 | 26 118.5 26 119 26 119.5 26 120 26 120.5 | 25 190.5 25 191 25 191.5 25 192 25 192.5 |
| 41 42 43 44 45 | 22 396.5 22 397 22 397.5 22 398 22 398.5 | 22 304.5 22 305 22 305.5 22 306 22 306.5 |  |  |
| 46 47 48 49 50 | 22 399 22 399.5 22 400 22 400.5 22 401 | 22 307 22 307.5 22 308 22 308.5 22 309 |  |  |
| 7 Ship stations may use the coast station receiving frequencies of channels No. 68 up to and including 135 for transmitting A1A or A1B Morse telegraphy (working). |  |  |  |  |  |  |

Table of frequencies for two-frequency 
operation by coast stations (kHz)

| Channel | 22 MHz band 7 (cont.) |  |
| --- | --- | --- |
| No. | Transmit | Receive |
| 51 52 53 54 55 | 22 401.5 22 402 22 402.5 22 403 22 403.5 | 22 309.5 22 310 22 310.5 22 311 22 311.5 |
| 56 57 58 59 60 | 22 404 22 404.5 22 405 22 405.5 22 406 | 22 312 22 312.5 22 313 22 313.5 22 314 |
| 61 62 63 64 65 | 22 406.5 22 407 22 407.5 22 408 22 408.5 | 22 314.5 22 315 22 315.5 22 316 22 316.5 |
| 66 67 68 69 70 | 22 409 22 409.5 22 410 22 410.5 22 411 | 22 317 22 317.5 22 318 22 318.5 22 319 |
| 71 72 73 74 75 | 22 411.5 22 412 22 412.5 22 413 22 413.5 | 22 319.5 22 320 22 320.5 22 321 22 321.5 |
| 76 77 78 79 80 | 22 414 22 414.5 22 415 22 415.5 22 416 | 22 322 22 322.5 22 323 22 323.5 22 324 |
| 81 82 83 84 85 | 22 416.5 22 417 22 417.5 22 418 22 418.5 | 22 324.5 22 325 22 325.5 22 326 22 326.5 |
| 86 87 88 89 90 | 22 419 22 419.5 22 420 22 420.5 22 421 | 22 327 22 327.5 22 328 22 328.5 22 329 |
| 91 92 93 94 95 | 22 421.5 22 422 22 422.5 22 423 22 423.5 | 22 329.5 22 330 22 330.5 22 331 22 331.5 |
| 96 97 98 99 100 | 22 424 22 424.5 22 425 22 425.5 22 426 | 22 332 22 332.5 22 333 22 333.5 22 334 |
| 101 102 103 104 105 | 22 426.5 22 427 22 427.5 22 428 22 428.5 | 22 334.5 22 335 22 335.5 22 336 22 336.5 |

Table of frequencies for two-frequency 
operation by coast stations (kHz)

| Channel | 22 MHz band 7 (end ) |  |
| --- | --- | --- |
| No. | Transmit | Receive |
| 106 107 108 109 110 | 22 429 22 429.5 22 430 22 430.5 22 431 | 22 337 22 337.5 22 338 22 338.5 22 339 |
| 111 112 113 114 115 | 22 431.5 22 432 22 432.5 22 433 22 433.5 | 22 339.5 22 340 22 340.5 22 341 22 341.5 |
| 116 117 118 119 120 | 22 434 22 434.5 22 435 22 435.5 22 436 | 22 342 22 342.5 22 343 22 343.5 22 344 |
| 121 122 123 124 125 | 22 436.5 22 437 22 437.5 22 438 22 438.5 | 22 344.5 22 345 22 345.5 22 346 22 346.5 |
| 126 127 128 129 130 | 22 439 22 439.5 22 440 22 440.5 22 441 | 22 347 22 347.5 22 348 22 348.5 22 349 |
| 131 132 133 134 135 | 22 441.5 22 442 22 442.5 22 443 22 443.5 | 22 349.5 22 350 22 350.5 22 351 22 351.5 |

Section III  –  Narrow-band direct-printing telegraphy 
(non-paired frequencies)

1	One or more frequencies are assigned to each ship station as transmitting frequencies.

2	All frequencies in this Appendix may also be used by ship stations for transmitting A1A or A1B Morse telegraphy (working).

3	All frequencies appearing in this Appendix may be used for NBDP duplex operation.

The corresponding coast station frequencies should be selected by the administration concerned from the sub-bands for coast station wideband telegraphy, A1A or A1B Morse telegraphy, facsimile, special and data transmission systems and direct-printing telegraphy systems.

4	The speed of the narrow-band direct-printing telegraphy and data systems shall not exceed 100 Bd for FSK and 200 Bd for PSK.

Table of ship station transmitting frequencies (kHz)

| Frequency bands |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | 4 MHz | 6 MHz | 8 MHz | 12 MHz | 16 MHz | 18/19 MHz | 22 MHz | 25/26 MHz |
| 1 2 3 4 5 | 4 202.5 4 203 4 203.5 4 204 4 204.5 | 6 300.5 6 301 6 301.5 6 302 6 302.5 | 8 396.5 8 397 8 397.5 8 398 8 398.5 | 12 560 12 560.5 12 561 12 561.5 12 562 | 16 785 16 785.5 16 786 16 786.5 16 787 | 18 893 18 893.5 18 894 18 894.5 18 895 | 22 352 22 352.5 22 353 22 353.5 22 354 | 25 193 25 193.5 25 194 25 194.5 25 195 |
| 6 7 8 9 10 | 4 205 4 205.5 4 206 4 206.5 4 207 | 6 303 6 303.5 6 304 6 304.5 6 305 | 8 399 8 399.5 8 400 8 400.5 8 401 | 12 562.5 12 563 12 563.5 12 564 12 564.5 | 16 787.5 16 788 16 788.5 16 789 16 789.5 | 18 895.5 18 896 18 896.5 18 897 18 897.5 | 22 354.5 22 355 22 355.5 22 356 22 356.5 | 25 195.5 25 196 25 196.5 25 197 25 197.5 |
| 11 12 13 14 15 |  | 6 305.5 6 306 6 306.5 6 307 6 307.5 | 8 401.5 8 402 8 402.5 8 403 8 403.5 | 12 565 12 565.5 12 566 12 566.5 12 567 | 16 790 16 790.5 16 791 16 791.5 16 792 | 18 898 | 22 357 22 357.5 22 358 22 358.5 22 359 | 25 198 25 198.5 25 199 25 199.5 25 200 |
| 16 17 18 19 20 |  | 6 308 6 308.5 6 309 6 309.5 6 310 | 8 404 8 404.5 8 405 8 405.5 8 406 | 12 567.5 12 568 12 568.5 12 569 12 569.5 | 16 792.5 16 793 16 793.5 16 794 16 794.5 |  | 22 359.5 22 360 22 360.5 22 361 22 361.5 | 25 200.5 25 201 25 201.5 25 202 25 202.5 |
| 21 22 23 24 25 |  | 6 310.5 6 311 6 311.5 | 8 406.5 8 407 8 407.5 8 408 8 408.5 | 12 570 12 570.5 12 571 12 571.5 12 572 | 16 795 16 795.5 16 796 16 796.5 16 797 |  | 22 362 22 362.5 22 363 22 363.5 22 364 | 25 203 25 203.5 25 204 25 204.5 25 205 |
| 26 27 28 29 30 |  |  | 8 409 8 409.5 8 410 8 410.5 8 411 | 12 572.5 12 573 12 573.5 12 574 12 574.5 | 16 797.5 16 798 16 798.5 16 799 16 799.5 |  | 22 364.5 22 365 22 365.5 22 366 22 366.5 | 25 205.5 25 206 25 206.5 25 207 25 207.5 |
| 31 32 33 34 35 |  |  | 8 411.5 8 412 8 412.5 8 413 8 413.5 | 12 575 12 575.5 12 576 12 576.5 | 16 800 16 800.5 16 801 16 801.5 16 802 |  | 22 367 22 367.5 22 368 22 368.5 22 369 | 25 208 |
| 36 37 38 39 40 |  |  | 8 414 |  | 16 802.5 16 803 16 803.5 16 804 |  | 22 369.5 22 370 22 370.5 22 371 22 371.5 |  |
| 41 42 43 44 45 |  |  |  |  |  |  | 22 372 22 372.5 22 373 22 373.5 22 374 |  |

Section IV  –  Morse telegraphy (calling)

Table of calling frequencies assignable to ship stations for A1A or A1B
Morse telegraphy at speeds not exceeding 40 Bd* (kHz)

| Group | Channel series | 4 MHz band | 6 MHz band | 8 MHz band | 12 MHz band | 16 MHz band | 22 MHz band | 25/26 MHz band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | 1 2 | 4 182 4 182.5 | 6 277 6 277.5 | 8 366 8 366.5 | 12 550 12 550.5 | 16 734 16 734.5 | 22 279.5 22 280 | Channel A 25 171.5 Groups I and II |
| Common channel Common channel | 3 4 | 4 184 4 184.5 | 6 276 6 276.5 | 8 368 8 369 | 12 552 12 553.5 | 16 736 16 738 | 22 280.5 22 281 | Common channel C 25 172 |
| II | 5 6 | 4 183 4 183.5 | 6 278 6 278.5 | 8 367 8 367.5 | 12 551 12 551.5 | 16 735 16 735.5 | 22 281.5 22 282 | Channel A 25 171.5 Groups I and II |
| III | 7 8 | 4 185 4 185.5 | 6 279 6 279.5 | 8 368.5 8 369.5 | 12 552.5 12 553 | 16 736.5 16 737 | 22 282.5 22 283 | Channel B 25 172.5 |
| IV | 9 10 | 4 186 4 186.5 | 6 280 6 280.5 | 8 370 8 370.5 | 12 554 12 554.5 | 16 737.5 16 738.5 | 22 283.5 22 284 | Groups III and IV |
| * Channel width in every band: 0.5 kHz. NOTES 1 Only the common channels in the 4, 6, 8, 12 and 16 MHz for A1A Morse telegraphy are harmonically related. 2 Administrations should assign the frequencies as they appear in this Appendix only to ship stations equipped with cristal controlled oscillators. 3 However, administrations may subdivide each appropriate group channel and common channel into specific calling frequencies on every full 100 Hz in the channel and assign these discrete frequencies to ships with synthetized transmitters. |  |  |  |  |  |  |  |  |  |

Examples of subdivision of channels (centre frequencies are underlined)

| 4 181.8 4 181.9 4 182 4 182.1 4 182.2 | 6 276.8 6 276.9 6 277 6 277.1 6 277.2 | 8 365.8 8 365.9 8 366 8 366.1 8 366.2 | 12 549.8 12 549.9 12 550 12 550.1 12 550.2 | 16 733.8 16 733.9 16 734 16 734.1 16 734.2 | 22 279.3 22 279.4 22 279.5 22 279.6 22 279.7 | 25 171.3 25 171.4 25 171.5 25 171.6 25 171.7 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 Administrations should avoid as far as possible, assigning the two frequencies at ± 100 Hz from the harmonically related common channel. 5 In the 22 MHz bands and 25/26 MHz bands the channels are not harmonically related to those in the 4 to 16 MHz bands. However, the principle of subdivision of channels into specific calling frequencies on 100 Hz applies. |  |  |  |  |  |  |  |

Section V  –  Morse telegraphy (working)

Table of working frequencies (kHz) assignable to ship stations
for A1A or A1B Morse telegraphy at speeds
not exceeding 40 Bd

(See also Part A, Note e))

| Frequency bands |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | 4 MHz | 6 MHz | 8 MHz | 12 MHz | 16 MHz | 22 MHz | 25/26 MHz |
| 1 2 3 4 5 | 4 187 4 187.5 4 188 4 188.5 4 189 | 6 285 6 285.5 6 286 6 286.5 6 287 | 8 342 8 342.5 8 343 8 343.5 8 344 | 12 422 12 422.5 12 423 12 423.5 12 424 | 16 619 16 619.5 16 620 16 620.5 16 621 | 22 242 22 242.5 22 243 22 243.5 22 244 | 25 161.5 25 162 25 162.5 25 163 25 163.5 |
| 6 7 8 9 10 | 4 189.5 4 190 4 190.5 4 191 4 191.5 | 6 287.5 6 288 6 288.5 6 289 6 289.5 | 8 344.5 8 345 8 345.5 8 346 8 346.5 | 12 424.5 12 425 12 425.5 12 426 12 426.5 | 16 621.5 16 622 16 622.5 16 623 16 623.5 | 22 244.5 22 245 22 245.5 22 246 22 246.5 | 25 164 25 164.5 25 165 25 165.5 25 166 |
| 11 12 13 14 15 | 4 192 4 192.5 4 193 4 193.5 4 194 | 6 290 6 290.5 6 291 6 291.5 6 292 | 8 347 8 347.5 8 348 8 348.5 8 349 | 12 427 12 427.5 12 428 12 428.5 12 429 | 16 624 16 624.5 16 625 16 625.5 16 626 | 22 247 22 247.5 22 248 22 248.5 22 249 | 25 166.5 25 167 25 167.5 25 168 25 168.5 |
| 16 17 18 19 20 | 4 194.5 4 195 4 195.5 4 196 4 196.5 | 6 292.5 6 293 6 293.5 6 294 6 294.5 | 8 349.5 8 350 8 350.5 8 351 8 351.5 | 12 429.5 12 430 12 430.5 12 431 12 431.5 | 16 626.5 16 627 16 627.5 16 628 16 628.5 | 22 249.5 22 250 22 250.5 22 251 22 251.5 | 25 169 25 169.5 25 170 25 170.5 25 171 |
| 21 22 23 24 25 | 4 197 4 197.5 4 198 4 198.5 4 199 | 6 295 6 295.5 6 296 6 296.5 6 297 | 8 352 8 352.5 8 353 8 353.5 8 354 | 12 432 12 432.5 12 433 12 433.5 12 434 | 16 629 16 629.5 16 630 16 630.5 16 631 | 22 252 22 252.5 22 253 22 253.5 22 254 |  |
| 26 27 28 29 30 | 4 199.5 4 200 4 200.5 4 201 4 201.5 | 6 297.5 6 298 6 298.5 6 299 6 299.5 | 8 354.5 8 355 8 355.5 8 356 8 356.5 | 12 434.5 12 435 12 435.5 12 436 12 436.5 | 16 631.5 16 632 16 632.5 16 633 16 633.5 | 22 254.5 22 255 22 255.5 22 256 22 256.5 |  |
| 31 32 33 34 35 | 4 202 | 6 300 | 8 357 8 357.5 8 358 8 358.5 8 359 | 12 437 12 437.5 12 438 12 438.5 12 439 | 16 634 16 634.5 16 635 16 635.5 16 636 | 22 257 22 257.5 22 258 22 258.5 22 259 |  |
| 36 37 38 39 40 |  |  | 8 359.5 8 360 8 360.5 8 361 8 361.5 | 12 439.5 12 440 12 440.5 12 441 12 441.5 | 16 636.5 16 637 16 637.5 16 638 16 638.5 | 22 259.5 22 260 22 260.5 22 261 22 261.5 |  |
| 41 42 43 44 45 |  |  | 8 362 8 362.5 8 363 8 363.5 8 364 | 12 442 12 442.5 12 443 12 443.5 12 444 | 16 639 16 639.5 16 640 16 640.5 16 641 | 22 262 22 262.5 22 263 22 263.5 22 264 |  |

| Frequency bands (cont.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | 4 MHz | 6 MHz | 8 MHz | 12 MHz | 16 MHz | 22 MHz | 25/26 MHz |
| 46 47 48 49 50 |  |  | 8 364.5 8 365 8 365.5 8 371 8 371.5 | 12 444.5 12 445 12 445.5 12 446 12 446.5 | 16 641.5 16 642 16 642.5 16 643 16 643.5 | 22 264.5 22 265 22 265.5 22 266 22 266.5 |  |
| 51 52 53 54 55 |  |  | 8 372 8 372.5 8 373 8 373.5 8 374 | 12 447 12 447.5 12 448 12 448.5 12 449 | 16 644 16 644.5 16 645 16 645.5 16 646 | 22 267 22 267.5 22 268 22 268.5 22 269 |  |
| 56 57 58 59 60 |  |  | 8 374.5 8 375 8 375.5 8 376 | 12 449.5 12 450 12 450.5 12 451 12 451.5 | 16 646.5 16 647 16 647.5 16 648 16 648.5 | 22 269.5 22 270 22 270.5 22 271 22 271.5 |  |
| 61 62 63 64 65 |  |  |  | 12 452 12 452.5 12 453 12 453.5 12 454 | 16 649 16 649.5 16 650 16 650.5 16 651 | 22 272 22 272.5 22 273 22 273.5 22 274 |  |
| 66 67 68 69 70 |  |  |  | 12 454.5 12 455 12 455.5 12 456 12 456.5 | 16 651.5 16 652 16 652.5 16 653 16 653.5 | 22 274.5 22 275 22 275.5 22 276 22 276.5 |  |
| 71 72 73 74 75 |  |  |  | 12 457 12 457.5 12 458 12 458.5 12 459 | 16 654 16 654.5 16 655 16 655.5 16 656 | 22 277 22 277.5 22 278 22 278.5 22 279 |  |
| 76 77 78 79 80 |  |  |  | 12 459.5 12 460 12 460.5 12 461 12 461.5 | 16 656.5 16 657 16 657.5 16 658 16 658.5 |  |  |
| 81 82 83 84 85 |  |  |  | 12 462 12 462.5 12 463 12 463.5 12 464 | 16 659 16 659.5 16 660 16 660.5 16 661 |  |  |
| 86 87 88 89 90 |  |  |  | 12 464.5 12 465 12 465.5 12 466 12 466.5 | 16 661.5 16 662 16 662.5 16 663 16 663.5 |  |  |
| 91 92 93 94 95 |  |  |  | 12 467 12 467.5 12 468 12 468.5 12 469 | 16 664 16 664.5 16 665 16 665.5 16 666 |  |  |

| Frequency bands (end ) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | 4 MHz | 6 MHz | 8 MHz | 12 MHz | 16 MHz | 22 MHz | 25/26 MHz |
| 96 97 98 99 100 |  |  |  | 12 469.5 12 470 12 470.5 12 471 12 471.5 | 16 666.5 16 667 16 667.5 16 668 16 668.5 |  |  |
| 101 102 103 104 105 |  |  |  | 12 472 12 472.5 12 473 12 473.5 12 474 | 16 669 16 669.5 16 670 16 670.5 16 671 |  |  |
| 106 107 108 109 110 |  |  |  | 12 474.5 12 475 12 475.5 12 476 12 476.5 | 16 671.5 16 672 16 672.5 16 673 16 673.5 |  |  |
| 111 112 113 114 115 |  |  |  |  | 16 674 16 674.5 16 675 16 675.5 16 676 |  |  |
| 116 117 118 119 120 |  |  |  |  | 16 676.5 16 677 16 677.5 16 678 16 678.5 |  |  |
| 121 122 123 124 125 |  |  |  |  | 16 679 16 679.5 16 680 16 680.5 16 681 |  |  |
| 126 127 128 129 |  |  |  |  | 16 681.5 16 682 16 682.5 16 683 |  |  |

Annex 2     (WRC-15)

Frequency and channelling arrangements in the high-frequency 
bands for the maritime mobile service, which 
enter into force on 1 January 2017     (WRC-12)

PART  A  –  Table of subdivided bands     (WRC-12)

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies (f.) and the spacing in kHz being indicated in italics.

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) t) w) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations as well as coast stations for telephony, simplex operation a) u) v) w) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| Frequencies assignable to ship stations for data transmission e) m) p) q) r) u) w) | 4 153.5 to 4 168.5 6 f. 3 kHz | 6 234.5 to 6 258.5 9 f. 3 kHz | 8 301.5 to 8 337.5 13 f. 3 kHz | 12 369.5 to 12 417.5 17 f. 3 kHz | 16 550.5 to 16 613.5 22 f. 3 kHz | 18 847.5 to 18 871.5 9 f. 3 kHz | 22 181.5 to 22 238.5 20 f. 3 kHz | 25 122.5 to 25 176.5 19 f. 3 kHz |
| Limits (kHz) | 4 170 | 6 260 | 8 339 | 12 419 | 16 615 | 18 873 | 22 240 | 25 178 |
| Frequencies assignable to ship as well as coast stations for data transmission e) m) p) q) u) w) |  |  |  |  |  |  |  | 25 179.5 to 25 206.5 10 f. 3 kHz |
| Limits (kHz) | 4 170 | 6 260 | 8 339 | 12 419 | 16 615 | 18 873 | 22 240 | 25 208.25 |
| Frequencies (paired and non-paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK b) d) |  | 6 260.25 to 6 260.75 2 f. 0.5 kHz | 8 339.25 to 8 339.75 2 f. 0.5 kHz | 12 419.25 to 12 419.75 2 f. 0.5 kHz | 16 615.25 to 16 616.75 4 f. 0.5 kHz | 18 873.5 to 18 880 14 f. 0.5 kHz |  |  |
| Limits (kHz) | 4 170 | 6 261 | 8 340 | 12 420 | 16 617 | 18 880.25 | 22 240 | 25 208.25 |
| Frequencies assignable to ship stations for oceanographic data transmission c) |  | 6 261.3 to 6 262.5 5 f. 0.3 kHz | 8 340.3 to 8 341.5 5 f. 0.3 kHz | 12 420.3 to 12 421.5 5 f. 0.3 kHz | 16 617.3 to 16 618.5 5 f. 0.3 kHz |  | 22 240.3 to 22 241.5 5 f. 0.3 kHz |  |
| Limits (kHz) | 4 170 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 880.25 | 22 241.75 | 25 208.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 170 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 880.25 | 22 241.75 | 25 208.25 |
| Frequencies (paired and non-paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK b) d) j) | 4 170.5 to 4 180 20 f. 0.5 kHz | 6 263 to 6 269.5 14 f. 0.5 kHz |  | 12 422 1 f. 0.5 kHz |  |  |  |  |
| Limits (kHz) | 4 180.25 | 6 269.75 | 8 341.75 | 12 422.25 | 16 618.75 | 18 880.25 | 22 241.75 | 25 208.25 |
| Frequencies assignable to ship stations for data transmission e) m) p) q) u) w) | 4 181.75 to 4 187.75 3f. 3 kHz | 6 271.25 to 6 277.25 3 f. 3 kHz | 8 343.25 to 8 358.25 6 f. 3 kHz | 12 423.75 to 12 450.75 10 f. 3 kHz | 16 620.25 to 16 680.25 21 f. 3 kHz | 18 881.75 to 18 893.75 5 f. 3 kHz | 22 243.25 to 22 288.25 16 f. 3 kHz |  |
| Limits (kHz) | 4 189.25 | 6 278.75 | 8 359.75 | 12 452.25 | 16 681.75 | 18 895.25 | 22 289.75 | 25 208.25 |
| Frequencies assignable to ship as well as coast stations for data transmission e) m) p) q) u) w) | 4 190.75 to 4 196.75 3f. 3 kHz | 6 280.25 to 6 310.25 11 f. 3 kHz | 8 361.25 to 8 373.25 5 f. 3 kHz | 12 453.75 to 12 474.75 8 f. 3 kHz |  | 18 896.75 1 f. 3 kHz |  |  |
| Limits (kHz) | 4 198.25 | 6 311.75 | 8 374.75 | 12 476.25 | 16 681.75 | 18 898.25 | 22 289.75 | 25 208.25 |
| Frequencies assignable to coast stations for data transmission e) m) p) q) u) w) | 4 199.75 to 4 205.75 3f. 3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 374.75 | 12 476.25 | 16 681.75 | 18 898.25 | 22 289.75 | 25 208.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 374.75 | 12 476.25 | 16 681.75 | 18 898.25 | 22 289.75 | 25 208.25 |
| Frequencies (paired and non-paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK b) d) j) |  |  | 8 375 to 8 383.5 18 f. 0.5 kHz | 12 476.5 to 12 522.5 93 f. 0.5 kHz | 16 682 to 16 698.5 34 f. 0.5 kHz |  | 22 290 to 22 299 19 f 0.5 kHz |  |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 383.75 | 12 522.75 | 16 698.75 | 18 898.25 | 22 299.25 | 25 208.25 |
| Frequencies assignable to ship as well as coast stations for data transmission e) p) q) u) w) |  |  | 8 385.5 to 8 406.5 8 f. 3 kHz | 12 524.25 to 12 575.25 18 f. 3 kHz | 16 700.5 to 16 802.5 35 f. 3 kHz |  | 22 300.75 to 22 372.75 25 f. 3 kHz |  |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 408 | 12 576.75 | 16 804 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to coast stations for data transmission e ) m) p) q) u) w) |  |  | 8 409.5 to 8 412.5 2 f. 3 kHz |  |  |  |  |  |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414 | 12 576.75 | 16 804 | 18 898.25 | 22 374.25 | 25 208.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414 | 12 576.75 | 16 804 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired and non-paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK b) d) n) o) | 4 209.5 to 4 216 14 f. 0.5 kHz | 6 314 to 6 321.5 16 f. 0.5 kHz | 8 416.5 to 8 423.5 15 f. 0.5 kHz | 12 579 to 12 624.5 92 f. 0.5 kHz | 16 806.5 to 16 821.5 31 f. 0.5 kHz | 19 680.5 1 f. 0.5 kHz | 22 376 1 f. 0.5 kHz | 26 100.5 to 26 102.5 5 f. 0.5 kHz |
| Limits (kHz) | 4 216.25 | 6 321.75 | 8 423.75 | 12 624.75 | 16 821.75 | 19 680.75 | 22 376.25 | 26 102.75 |
| Frequencies assignable to ship stations for data transmission e) m) p) q) u) w) |  |  |  |  |  |  | 22 377.75 to 22 380.75 2 f. 3 kHz |  |
| Limits (kHz) | 4 216.25 | 6 321.75 | 8 423.75 | 12 624.75 | 16 821.75 | 19 680.75 | 22 382.25 | 26 102.75 |
| Frequencies assignable to ship as well as coast stations for data transmission e) m) p) q) u) w) | 4 217.75 1 f. 3 kHz |  |  |  | 16 823.25 to 16 838.25 6 f. 3 kHz |  |  |  |
| Limits (kHz) | 4 219.25 | 6 321.75 | 8 423.75 | 12 624.75 | 16 839.75 | 19 680.75 | 22 382.25 | 26 102.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 219.25 | 6 321.75 | 8 423.75 | 12 624.75 | 16 839.75 | 19 680.75 | 22 382.25 | 26 102.75 |
| Frequencies assignable to coast stations for data transmission e) m) q) u) w) |  | 6 323.25 to 6329.25 3 f. 3 kHz | 8 425.5 to 8 434.5 4 f. 3 kHz | 12 626.25 to 12 653.25 10 f. 3 kHz | 16 841.25 to 16 901.25 21 f. 3 kHz | 19 682.25 1 f. 3 kHz |  | 26 104.25 to 26 119.25 6 f. 3 kHz |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 654.75 | 16 902.75 | 19 683.75 | 22 382.25 | 26 120.75 |
| Frequencies (paired and non-paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK b) d) |  |  |  | 12 655 to 12 656.5 4 f. 0.5 kHz |  | 19 684 to 19 691 15 f. 0.5 kHz | 22 382.5 to 22 389 14 f. 0.5 kHz |  |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 691.25 | 22 389.25 | 26 120.75 |
| Frequencies assignable to coast stations for data transmission e) m) p) q) u) w) |  |  |  |  |  | 19 692.75 to 19 701.75 4 f. 3 kHz | 22 390.75 to 22 441.75 18 f. 3 kHz |  |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.25 | 26 120.75 |
| Frequencies (non-paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK b) |  |  |  |  |  |  | 22 443.5 1 f. 0.5 kHz |  |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz | 22 444 to 22 445 3 f. 0.5 kHz | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable for wide-band systems, facsimile, special and data transmission systems and direct-printing telegraphy systems m) p) s) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) t) w) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

a)	See Part B, Section I.

b)	See Part B, Section III.

c)	The frequency bands may also be used by buoy stations for oceanographic data transmission and by stations interrogating these buoys.

d)	See Part B, Section II.

e)	See Part B, Section IV.

i)	For the use of the carrier frequencies 4 125 kHz, 6 215 kHz, 8 291 kHz, 12 290 kHz and 16 420 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31.

j)	For the use of the assigned frequencies 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by NBDP telegraphy, see Article 31.

k)	For the use of the assigned frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by digital selective calling, see Article 31.

l)	The following paired assigned frequencies (for ship/coast stations) 4 208/4 219.5 kHz, 6 312.5/6 331 kHz, 8 415/8 436.5 kHz, 12 577.5/12 657 kHz, 16 805/16 903 kHz, 18 898.5/19 703.5 kHz, 22 374.5/22 444 kHz and 25 208.5/26 121 kHz are the first choice international frequencies for digital selective calling (see Article 54).

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions. Any frequencies so assigned shall be multiples of 100 Hz. Administrations shall ensure a uniform distribution of such assignments within the bands.

n)	The assigned frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the exclusive international frequencies for the transmission of maritime safety information (MSI) (see Articles 31 and 33).

o)	The frequency 4 209.5 kHz is an exclusive international frequency for the transmission of NAVTEX type information (see Articles 31 and 33).

p)	These sub-bands, except the frequencies referred to in Notes i), j), n) and o), are designated for digitally modulated emissions in the maritime mobile service (e.g. as described in the most recent version of Recommendation ITU-R M.1798). The provisions of No. 15.8 apply.     (WRC-15)

q)	These frequency bands may be used by narrow-band direct-printing applications by administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

r)	Frequencies in these bands may be used for wide-band telegraphy, facsimile and special data transmission on the condition that interference is not caused to and protection is not claimed from stations in the maritime mobile service using digitally modulated emissions.

s)	The frequency bands 4 345-4 351 kHz, 6 495-6 501 kHz, 8 701-8 707 kHz may be used for simplex (single-sideband) telephone operation (regularly spaced by 3 kHz), in accordance with provision No. 52.177, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

t)	The frequency bands 4 065-4 146 kHz, 4 351-4 438 kHz, 6 200-6 224 kHz, 6 501-6 525 kHz, 8 195-8 294 kHz, 8 707-8 815 kHz, 12 230-12 353 kHz, 13 077-13 200 kHz, 16 360-16 528 kHz, 17 242-17 410 kHz, 18 780-18 825 kHz, 19 755-19 800 kHz, 22 000-22 159 kHz, 22 696-22 855 kHz, 25 070-25 100 kHz and 26 145-26 175 kHz may be used, in accordance with the Appendix 25 allotment Plan, for digitally modulated emissions as described in the most recent version of Recommendation ITU-R M.1798 on the condition that it shall not cause harmful interference to, or claim protection from other stations in the maritime mobile service using radiotelephony operations. The digitally modulated emissions may be used provided that their occupied bandwidth does not exceed 2 800 Hz, it is situated wholly within one frequency channel and the peak envelope power of coast stations does not exceed 10 kW and the peak envelope power of ship stations does not exceed 1.5 kW per channel.      (WRC-15)

u)	These frequency bands may be used for wide-band digitally modulated emissions by combining multiple 3 kHz contiguous channels.

v)	The frequency bands 4 146-4 152 kHz, 6 224-6 233 kHz, 8 294-8 300 kHz, 12 353-12 368 kHz, 16 528-16 549 kHz, 18 825-18 846 kHz, 22 159-22 180 and 25 100-25 121 kHz may be used for simplex digitally modulated emissions as described in the most recent version of Recommendation ITU-R M.1798 on condition that it shall not cause harmful interference to, or claim protection from other stations in the maritime mobile service using radiotelephony operations. The digitally modulated emissions may be used provided that their occupied bandwidth does not exceed 2 800 Hz, it is situated wholly within one frequency channel and the peak envelope power of coast stations does not exceed 10 kW and the peak envelope power of ship stations does not exceed 1.5 kW per channel.      (WRC-15)

w)	Administrations that intend to use Annex 2 to introduce data transmissions before 1 January 2017 for stations operating in the maritime mobile service shall not cause harmful interference to nor claim protection from stations in the maritime mobile service operating in accordance with Annex 1 of this Appendix and are encouraged to coordinate bilaterally with affected administrations.

PART  B  –  Channelling arrangements     (WRC-15)

Section I  –  Radiotelephony

1	Radiotelephone channelling arrangements for the frequencies to be used by coast and ship stations in the bands allocated to the maritime mobile service are indicated in the following Sub-Sections:

Sub-Section A	–	Table of single-sideband transmitting frequencies (kHz) for duplex (two-frequency) operation;

Sub-Section B	–	Table of single-sideband transmitting frequencies (kHz) for simplex (single-frequency) operation and for intership cross-band (two-frequency) operation;

Sub-Section C-1	–	Table of recommended single-sideband transmitting frequencies (kHz) for ship stations in the frequency band 4 000-4 063 kHz shared with the fixed service;

Sub-Section C-2	–	Table of recommended single-sideband transmitting frequencies (kHz) for ship and coast stations in the frequency band 8 100-8 195 kHz shared with the fixed service.

2	The technical characteristics for single-sideband transmitters specified in Recommendation ITU-R M.1173-1 shall be used.     (WRC-15)

3	One or more series of frequencies from Sub-Section A (with the exception of those frequencies mentioned in § 5 below) may be assigned to each coast station, which uses these frequencies associated in pairs (see No. 52.226); each pair consists of a transmitting and a receiving frequency. The series shall be selected with due regard to the areas served and so as to avoid, as far as possible, harmful interference between the services of different coast stations.

4	The frequencies in Sub-Section B are provided for worldwide common use by ships of all categories, according to traffic requirements, for ship transmissions to coast stations and for intership communication. They are also authorized for worldwide common use for transmissions by coast stations (simplex operation) provided the peak envelope power does not exceed 1 kW.

5	The following frequencies in Sub-Section A are allocated for calling purposes:

–	Channel No. 421 in the 4 MHz band;

–	Channel No. 606 in the 6 MHz band;

–	Channel No. 821 in the 8 MHz band;

–	Channel No. 1221 in the 12 MHz band;

–	Channel No. 1621 in the 16 MHz band;

–	Channel No. 1806 in the 18 MHz band;

–	Channel No. 2221 in the 22 MHz band;

–	Channel No. 2510 in the 25 MHz band.

Calling on the carrier frequencies 12 290 kHz and 16 420 kHz shall be permitted only to and from rescue coordination centres (see No. 30.6.1), subject to the safeguards of Resolution 352 (WRC-03) (see Nos. 52.221A and 52.222A).

The remaining frequencies in Sub-Sections A, B, C-1 and C-2 are working frequencies.

5A	For the use of the carrier frequencies:

4 125 kHz (Channel No. 421);
  6 215 kHz (Channel No. 606);
  8 291 kHz (Channel No. 833);
12 290 kHz (Channel No. 1221);
16 420 kHz (Channel No. 1621);

in Sub-Section A, by coast and ship stations for distress and safety purposes, see Article 31.

6	a)	Maritime radiotelephone stations using single-sideband emissions in the frequency bands between 4 000 kHz and 27 500 kHz exclusively allocated to the maritime mobile service shall operate only on the carrier frequencies shown in the Sub-Sections A and B and, in the case of analogue radiotelephony, shall be in conformity with the technical characteristics specified in Recommendation ITU-R M.1173-1.      (WRC-15)

b)	Ship stations, when using frequencies for single-sideband emissions in the frequency band 4 000-4 063 kHz, and ship and coast stations, when using frequencies for single-sideband emissions in the frequency band 8 100-8 195 kHz, should operate on the carrier frequencies indicated in Sub-Sections C-1 and C-2 respectively. In the case of analogue radiotelephony technical characteristics of the equipment shall be those specified in Recommendation ITU-R M.1173-1.      (WRC-15)

c)	Stations, when employing the single-sideband mode for analogue radiotelephony, shall use only class J3E emissions. For digital communications, class J2D emissions shall be used.

7	The channelling plan established in Sub-Section C-2 does not prejudice the rights of administrations to establish, and to notify assignments to stations in the maritime mobile service other than those using radiotelephony in the frequency band 8 100-8 195 kHz, in conformity with the relevant provisions of these Regulations.

Sub-Section A

Table of single-sideband transmitting frequencies (kHz) for duplex
(two-frequency) operation

| Channel No. | 4 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 401 | 4 357 | 4 358.4 | 4 065 | 4 066.4 |
| 402 | 4 360 | 4 361.4 | 4 068 | 4 069.4 |
| 403 | 4 363 | 4 364.4 | 4 071 | 4 072.4 |
| 404 | 4 366 | 4 367.4 | 4 074 | 4 075.4 |
| 405 | 4 369 | 4 370.4 | 4 077 | 4 078.4 |
| 406 | 4 372 | 4 373.4 | 4 080 | 4 081.4 |
| 407 | 4 375 | 4 376.4 | 4 083 | 4 084.4 |
| 408 | 4 378 | 4 379.4 | 4 086 | 4 087.4 |
| 409 | 4 381 | 4 382.4 | 4 089 | 4 090.4 |
| 410 | 4 384 | 4 385.4 | 4 092 | 4 093.4 |
| 411 | 4 387 | 4 388.4 | 4 095 | 4 096.4 |
| 412 | 4 390 | 4 391.4 | 4 098 | 4 099.4 |
| 413 | 4 393 | 4 394.4 | 4 101 | 4 102.4 |
| 414 | 4 396 | 4 397.4 | 4 104 | 4 105.4 |
| 415 | 4 399 | 4 400.4 | 4 107 | 4 108.4 |
| 416 | 4 402 | 4 403.4 | 4 110 | 4 111.4 |
| 417 | 4 405 | 4 406.4 | 4 113 | 4 114.4 |
| 418 | 4 408 | 4 409.4 | 4 116 | 4 117.4 |
| 419 | 4 411 | 4 412.4 | 4 119 | 4 120.4 |
| 420 | 4 414 | 4 415.4 | 4 122 | 4 123.4 |
| 421 | 4 417 * | 4 418.4 * | 4 125 * 3 | 4 126.4 * |
| 422 | 4 420 | 4 421.4 | 4 128 | 4 129.4 |
| 423 | 4 423 | 4 424.4 | 4 131 | 4 132.4 |
| 424 | 4 426 | 4 427.4 | 4 134 | 4 135.4 |
| 425 | 4 429 | 4 430.4 | 4 137 | 4 138.4 |
| 426 | 4 432 | 4 433.4 | 4 140 | 4 141.4 |
| 427 | 4 435 | 4 436.4 | 4 143 | 4 144.4 |
| 428 1, 2 | 4 351 | 4 352.4 | – | – |
| 429 1, 2 | 4 354 | 4 355.4 | – | – |

| Channel No. | 6 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 601 | 6 501 | 6 502.4 | 6 200 | 6 201.4 |
| 602 | 6 504 | 6 505.4 | 6 203 | 6 204.4 |
| 603 | 6 507 | 6 508.4 | 6 206 | 6 207.4 |
| 604 | 6 510 | 6 511.4 | 6 209 | 6 210.4 |
| 605 | 6 513 | 6 514.4 | 6 212 | 6 213.4 |
| 606 | 6 516 * | 6 517.4 * | 6 215 * 4 | 6 216.4 * |
| 607 | 6 519 | 6 520.4 | 6 218 | 6 219.4 |
| 608 | 6 522 | 6 523.4 | 6 221 | 6 222.4 |

| Channel No. | 8 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 801 | 8 719 | 8 720.4 | 8 195 | 8 196.4 |
| 802 | 8 722 | 8 723.4 | 8 198 | 8 199.4 |
| 803 | 8 725 | 8 726.4 | 8 201 | 8 202.4 |
| 804 | 8 728 | 8 729.4 | 8 204 | 8 205.4 |
| 805 | 8 731 | 8 732.4 | 8 207 | 8 208.4 |
| 806 | 8 734 | 8 735.4 | 8 210 | 8 211.4 |
| 807 | 8 737 | 8 738.4 | 8 213 | 8 214.4 |
| 808 | 8 740 | 8 741.4 | 8 216 | 8 217.4 |
| 809 | 8 743 | 8 744.4 | 8 219 | 8 220.4 |
| 810 | 8 746 | 8 747.4 | 8 222 | 8 223.4 |
| 811 | 8 749 | 8 750.4 | 8 225 | 8 226.4 |
| 812 | 8 752 | 8 753.4 | 8 228 | 8 229.4 |
| 813 | 8 755 | 8 756.4 | 8 231 | 8 232.4 |
| 814 | 8 758 | 8 759.4 | 8 234 | 8 235.4 |
| 815 | 8 761 | 8 762.4 | 8 237 | 8 238.4 |
| 816 | 8 764 | 8 765.4 | 8 240 | 8 241.4 |
| 817 | 8 767 | 8 768.4 | 8 243 | 8 244.4 |
| 818 | 8 770 | 8 771.4 | 8 246 | 8 247.4 |
| 819 | 8 773 | 8 774.4 | 8 249 | 8 250.4 |
| 820 | 8 776 | 8 777.4 | 8 252 | 8 253.4 |
| 821 | 8 779 * | 8 780.4 * | 8 255 * | 8 256.4 * |
| 822 | 8 782 | 8 783.4 | 8 258 | 8 259.4 |
| 823 | 8 785 | 8 786.4 | 8 261 | 8 262.4 |
| 824 | 8 788 | 8 789.4 | 8 264 | 8 265.4 |
| 825 | 8 791 | 8 792.4 | 8 267 | 8 268.4 |
| 826 | 8 794 | 8 795.4 | 8 270 | 8 271.4 |
| 827 | 8 797 | 8 798.4 | 8 273 | 8 274.4 |
| 828 | 8 800 | 8 801.4 | 8 276 | 8 277.4 |
| 829 | 8 803 | 8 804.4 | 8 279 | 8 280.4 |
| 830 | 8 806 | 8 807.4 | 8 282 | 8 283.4 |
| 831 | 8 809 | 8 810.4 | 8 285 | 8 286.4 |
| 832 | 8 812 | 8 813.4 | 8 288 | 8 289.4 |
| 833 | 8 291 6 | 8 292.4 | 8 291 6 | 8 292.4 |
| 834 2, 5 | 8 707 | 8 708.4 | – | – |
| 835 2, 5 | 8 710 | 8 711.4 | – | – |
| 836 2, 5 | 8 713 | 8 714.4 | – | – |
| 837 2, 5 | 8 716 | 8 717.4 | – | – |

| Channel No. | 12 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1201 | 13 077 | 13 078.4 | 12 230 | 12 231.4 |
| 1202 | 13 080 | 13 081.4 | 12 233 | 12 234.4 |
| 1203 | 13 083 | 13 084.4 | 12 236 | 12 237.4 |
| 1204 | 13 086 | 13 087.4 | 12 239 | 12 240.4 |
| 1205 | 13 089 | 13 090.4 | 12 242 | 12 243.4 |
| 1206 | 13 092 | 13 093.4 | 12 245 | 12 246.4 |
| 1207 | 13 095 | 13 096.4 | 12 248 | 12 249.4 |
| 1208 | 13 098 | 13 099.4 | 12 251 | 12 252.4 |
| 1209 | 13 101 | 13 102.4 | 12 254 | 12 255.4 |
| 1210 | 13 104 | 13 105.4 | 12 257 | 12 258.4 |
| 1211 | 13 107 | 13 108.4 | 12 260 | 12 261.4 |
| 1212 | 13 110 | 13 111.4 | 12 263 | 12 264.4 |
| 1213 | 13 113 | 13 114.4 | 12 266 | 12 267.4 |
| 1214 | 13 116 | 13 117.4 | 12 269 | 12 270.4 |
| 1215 | 13 119 | 13 120.4 | 12 272 | 12 273.4 |
| 1216 | 13 122 | 13 123.4 | 12 275 | 12 276.4 |
| 1217 | 13 125 | 13 126.4 | 12 278 | 12 279.4 |
| 1218 | 13 128 | 13 129.4 | 12 281 | 12 282.4 |
| 1219 | 13 131 | 13 132.4 | 12 284 | 12 285.4 |
| 1220 | 13 134 | 13 135.4 | 12 287 | 12 288.4 |
| 1221 | 13 137 * | 13 138.4 * | 12 290 * 7 | 12 291.4 * |
| 1222 | 13 140 | 13 141.4 | 12 293 | 12 294.4 |
| 1223 | 13 143 | 13 144.4 | 12 296 | 12 297.4 |
| 1224 | 13 146 | 13 147.4 | 12 299 | 12 300.4 |
| 1225 | 13 149 | 13 150.4 | 12 302 | 12 303.4 |
| 1226 | 13 152 | 13 153.4 | 12 305 | 12 306.4 |
| 1227 | 13 155 | 13 156.4 | 12 308 | 12 309.4 |
| 1228 | 13 158 | 13 159.4 | 12 311 | 12 312.4 |
| 1229 | 13 161 | 13 162.4 | 12 314 | 12 315.4 |
| 1230 | 13 164 | 13 165.4 | 12 317 | 12 318.4 |
| 1231 | 13 167 | 13 168.4 | 12 320 | 12 321.4 |
| 1232 | 13 170 | 13 171.4 | 12 323 | 12 324.4 |
| 1233 | 13 173 | 13 174.4 | 12 326 | 12 327.4 |
| 1234 | 13 176 | 13 177.4 | 12 329 | 12 330.4 |
| 1235 | 13 179 | 13 180.4 | 12 332 | 12 333.4 |
| 1236 | 13 182 | 13 183.4 | 12 335 | 12 336.4 |
| 1237 | 13 185 | 13 186.4 | 12 338 | 12 339.4 |
| 1238 | 13 188 | 13 189.4 | 12 341 | 12 342.4 |
| 1239 | 13 191 | 13 192.4 | 12 344 | 12 345.4 |
| 1240 | 13 194 | 13 195.4 | 12 347 | 12 348.4 |
| 1241 | 13 197 | 13 198.4 | 12 350 | 12 351.4 |

| Channel No. | 16 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1601 | 17 242 | 17 243.4 | 16 360 | 16 361.4 |
| 1602 | 17 245 | 17 246.4 | 16 363 | 16 364.4 |
| 1603 | 17 248 | 17 249.4 | 16 366 | 16 367.4 |
| 1604 | 17 251 | 17 252.4 | 16 369 | 16 370.4 |
| 1605 | 17 254 | 17 255.4 | 16 372 | 16 373.4 |
| 1606 | 17 257 | 17 258.4 | 16 375 | 16 376.4 |
| 1607 | 17 260 | 17 261.4 | 16 378 | 16 379.4 |
| 1608 | 17 263 | 17 264.4 | 16 381 | 16 382.4 |
| 1609 | 17 266 | 17 267.4 | 16 384 | 16 385.4 |
| 1610 | 17 269 | 17 270.4 | 16 387 | 16 388.4 |
| 1611 | 17 272 | 17 273.4 | 16 390 | 16 391.4 |
| 1612 | 17 275 | 17 276.4 | 16 393 | 16 394.4 |
| 1613 | 17 278 | 17 279.4 | 16 396 | 16 397.4 |
| 1614 | 17 281 | 17 282.4 | 16 399 | 16 400.4 |
| 1615 | 17 284 | 17 285.4 | 16 402 | 16 403.4 |
| 1616 | 17 287 | 17 288.4 | 16 405 | 16 406.4 |
| 1617 | 17 290 | 17 291.4 | 16 408 | 16 409.4 |
| 1618 | 17 293 | 17 294.4 | 16 411 | 16 412.4 |
| 1619 | 17 296 | 17 297.4 | 16 414 | 16 415.4 |
| 1620 | 17 299 | 17 300.4 | 16 417 | 16 418.4 |
| 1621 | 17 302 * | 17 303.4 * | 16 420 * 8 | 16 421.4 * |
| 1622 | 17 305 | 17 306.4 | 16 423 | 16 424.4 |
| 1623 | 17 308 | 17 309.4 | 16 426 | 16 427.4 |
| 1624 | 17 311 | 17 312.4 | 16 429 | 16 430.4 |
| 1625 | 17 314 | 17 315.4 | 16 432 | 16 433.4 |
| 1626 | 17 317 | 17 318.4 | 16 435 | 16 436.4 |
| 1627 | 17 320 | 17 321.4 | 16 438 | 16 439.4 |
| 1628 | 17 323 | 17 324.4 | 16 441 | 16 442.4 |
| 1629 | 17 326 | 17 327.4 | 16 444 | 16 445.4 |
| 1630 | 17 329 | 17 330.4 | 16 447 | 16 448.4 |
| 1631 | 17 332 | 17 333.4 | 16 450 | 16 451.4 |
| 1632 | 17 335 | 17 336.4 | 16 453 | 16 454.4 |
| 1633 | 17 338 | 17 339.4 | 16 456 | 16 457.4 |
| 1634 | 17 341 | 17 342.4 | 16 459 | 16 460.4 |
| 1635 | 17 344 | 17 345.4 | 16 462 | 16 463.4 |
| 1636 | 17 347 | 17 348.4 | 16 465 | 16 466.4 |
| 1637 | 17 350 | 17 351.4 | 16 468 | 16 469.4 |
| 1638 | 17 353 | 17 354.4 | 16 471 | 16 472.4 |
| 1639 | 17 356 | 17 357.4 | 16 474 | 16 475.4 |
| 1640 | 17 359 | 17 360.4 | 16 477 | 16 478.4 |
| 1641 | 17 362 | 17 363.4 | 16 480 | 16 481.4 |
| 1642 | 17 365 | 17 366.4 | 16 483 | 16 484.4 |
| 1643 | 17 368 | 17 369.4 | 16 486 | 16 487.4 |
| 1644 | 17 371 | 17 372.4 | 16 489 | 16 490.4 |
| 1645 | 17 374 | 17 375.4 | 16 492 | 16 493.4 |
| 1646 | 17 377 | 17 378.4 | 16 495 | 16 496.4 |
| 1647 | 17 380 | 17 381.4 | 16 498 | 16 499.4 |
| 1648 | 17 383 | 17 384.4 | 16 501 | 16 502.4 |
| 1649 | 17 386 | 17 387.4 | 16 504 | 16 505.4 |
| 1650 | 17 389 | 17 390.4 | 16 507 | 16 508.4 |
| 1651 | 17 392 | 17 393.4 | 16 510 | 16 511.4 |
| 1652 | 17 395 | 17 396.4 | 16 513 | 16 514.4 |
| 1653 | 17 398 | 17 399.4 | 16 516 | 16 517.4 |
| 1654 | 17 401 | 17 402.4 | 16 519 | 16 520.4 |
| 1655 | 17 404 | 17 405.4 | 16 522 | 16 523.4 |
| 1656 | 17 407 | 17 408.4 | 16 525 | 16 526.4 |

| Channel No. | 18/19 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 1801 | 19 755 | 19 756.4 | 18 780 | 18 781.4 |
| 1802 | 19 758 | 19 759.4 | 18 783 | 18 784.4 |
| 1803 | 19 761 | 19 762.4 | 18 786 | 18 787.4 |
| 1804 | 19 764 | 19 765.4 | 18 789 | 18 790.4 |
| 1805 | 19 767 | 19 768.4 | 18 792 | 18 793.4 |
| 1806 | 19 770 * | 19 771.4 * | 18 795 * | 18 796.4 * |
| 1807 | 19 773 | 19 774.4 | 18 798 | 18 799.4 |
| 1808 | 19 776 | 19 777.4 | 18 801 | 18 802.4 |
| 1809 | 19 779 | 19 780.4 | 18 804 | 18 805.4 |
| 1810 | 19 782 | 19 783.4 | 18 807 | 18 808.4 |
| 1811 | 19 785 | 19 786.4 | 18 810 | 18 811.4 |
| 1812 | 19 788 | 19 789.4 | 18 813 | 18 814.4 |
| 1813 | 19 791 | 19 792.4 | 18 816 | 18 817.4 |
| 1814 | 19 794 | 19 795.4 | 18 819 | 18 820.4 |
| 1815 | 19 797 | 19 798.4 | 18 822 | 18 823.4 |

| Channel No. | 22 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2201 | 22 696 | 22 697.4 | 22 000 | 22 001.4 |
| 2202 | 22 699 | 22 700.4 | 22 003 | 22 004.4 |
| 2203 | 22 702 | 22 703.4 | 22 006 | 22 007.4 |
| 2204 | 22 705 | 22 706.4 | 22 009 | 22 010.4 |
| 2205 | 22 708 | 22 709.4 | 22 012 | 22 013.4 |
| 2206 | 22 711 | 22 712.4 | 22 015 | 22 016.4 |
| 2207 | 22 714 | 22 715.4 | 22 018 | 22 019.4 |
| 2208 | 22 717 | 22 718.4 | 22 021 | 22 022.4 |
| 2209 | 22 720 | 22 721.4 | 22 024 | 22 025.4 |
| 2210 | 22 723 | 22 724.4 | 22 027 | 22 028.4 |
| 2211 | 22 726 | 22 727.4 | 22 030 | 22 031.4 |
| 2212 | 22 729 | 22 730.4 | 22 033 | 22 034.4 |
| 2213 | 22 732 | 22 733.4 | 22 036 | 22 037.4 |
| 2214 | 22 735 | 22 736.4 | 22 039 | 22 040.4 |
| 2215 | 22 738 | 22 739.4 | 22 042 | 22 043.4 |
| 2216 | 22 741 | 22 742.4 | 22 045 | 22 046.4 |
| 2217 | 22 744 | 22 745.4 | 22 048 | 22 049.4 |
| 2218 | 22 747 | 22 748.4 | 22 051 | 22 052.4 |
| 2219 | 22 750 | 22 751.4 | 22 054 | 22 055.4 |
| 2220 | 22 753 | 22 754.4 | 22 057 | 22 058.4 |
| 2221 | 22 756 * | 22 757.4 * | 22 060 * | 22 061.4 * |
| 2222 | 22 759 | 22 760.4 | 22 063 | 22 064.4 |
| 2223 | 22 762 | 22 763.4 | 22 066 | 22 067.4 |
| 2224 | 22 765 | 22 766.4 | 22 069 | 22 070.4 |
| 2225 | 22 768 | 22 769.4 | 22 072 | 22 073.4 |

| Channel No. | 22 MHz band (end ) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2226 | 22 771 | 22 772.4 | 22 075 | 22 076.4 |
| 2227 | 22 774 | 22 775.4 | 22 078 | 22 079.4 |
| 2228 | 22 777 | 22 778.4 | 22 081 | 22 082.4 |
| 2229 | 22 780 | 22 781.4 | 22 084 | 22 085.4 |
| 2230 | 22 783 | 22 784.4 | 22 087 | 22 088.4 |
| 2231 | 22 786 | 22 787.4 | 22 090 | 22 091.4 |
| 2232 | 22 789 | 22 790.4 | 22 093 | 22 094.4 |
| 2233 | 22 792 | 22 793.4 | 22 096 | 22 097.4 |
| 2234 | 22 795 | 22 796.4 | 22 099 | 22 100.4 |
| 2235 | 22 798 | 22 799.4 | 22 102 | 22 103.4 |
| 2236 | 22 801 | 22 802.4 | 22 105 | 22 106.4 |
| 2237 | 22 804 | 22 805.4 | 22 108 | 22 109.4 |
| 2238 | 22 807 | 22 808.4 | 22 111 | 22 112.4 |
| 2239 | 22 810 | 22 811.4 | 22 114 | 22 115.4 |
| 2240 | 22 813 | 22 814.4 | 22 117 | 22 118.4 |
| 2241 | 22 816 | 22 817.4 | 22 120 | 22 121.4 |
| 2242 | 22 819 | 22 820.4 | 22 123 | 22 124.4 |
| 2243 | 22 822 | 22 823.4 | 22 126 | 22 127.4 |
| 2244 | 22 825 | 22 826.4 | 22 129 | 22 130.4 |
| 2245 | 22 828 | 22 829.4 | 22 132 | 22 133.4 |
| 2246 | 22 831 | 22 832.4 | 22 135 | 22 136.4 |
| 2247 | 22 834 | 22 835.4 | 22 138 | 22 139.4 |
| 2248 | 22 837 | 22 838.4 | 22 141 | 22 142.4 |
| 2249 | 22 840 | 22 841.4 | 22 144 | 22 145.4 |
| 2250 | 22 843 | 22 844.4 | 22 147 | 22 148.4 |
| 2251 | 22 846 | 22 847.4 | 22 150 | 22 151.4 |
| 2252 | 22 849 | 22 850.4 | 22 153 | 22 154.4 |
| 2253 | 22 852 | 22 853.4 | 22 156 | 22 157.4 |

| Channel No. | 25/26 MHz band |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coast stations |  | Ship stations |  |
|  | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 2501 | 26 145 | 26 146.4 | 25 070 | 25 071.4 |
| 2502 | 26 148 | 26 149.4 | 25 073 | 25 074.4 |
| 2503 | 26 151 | 26 152.4 | 25 076 | 25 077.4 |
| 2504 | 26 154 | 26 155.4 | 25 079 | 25 080.4 |
| 2505 | 26 157 | 26 158.4 | 25 082 | 25 083.4 |
| 2506 | 26 160 | 26 161.4 | 25 085 | 25 086.4 |
| 2507 | 26 163 | 26 164.4 | 25 088 | 25 089.4 |
| 2508 | 26 166 | 26 167.4 | 25 091 | 25 092.4 |
| 2509 | 26 169 | 26 170.4 | 25 094 | 25 095.4 |
| 2510 | 26 172 * | 26 173.4 * | 25 097 * | 25 098.4 * |

1	These coast station frequencies may be paired with a ship station frequency from the Table of simplex frequencies for ship and coast stations (see Sub-Section B) or with a frequency from the band 4 000-4 063 kHz (see Sub-Section C-1) to be selected by the administration concerned.

2	These channels may also be used for simplex (single frequency) operation.

3	For the conditions of use of the carrier frequency 4 125 kHz, see Nos. 52.224 and 52.225, and Appendix 15.

4	For the conditions of use of the carrier frequency 6 215 kHz, see Appendix 15.

5	These coast station frequencies may be paired with a ship station frequency from the Table of simplex frequencies for ship and coast stations (see Sub-Section B) or with a frequency from the band 8 100-8 195 kHz (see Sub-Section C-2) to be selected by the administration concerned.

6	For the conditions of use of the carrier frequency 8 291 kHz, see Appendix 15.

7	For the conditions of use of the carrier frequency 12 290 kHz, see Nos. 52.221A and 52.222A and Appendix 15.

8	For the conditions of use of the carrier frequency 16 420 kHz, see Nos. 52.221A and 52.222A and Appendix 15.

*	The frequencies followed by an asterisk are calling frequencies (see Nos. 52.221 and 52.222).

Sub-Section B

Table of single-sideband transmitting frequencies (kHz) for simplex (single-frequency) 
operation and for intership cross-band (two-frequency) operation

(See § 4 of Section I of this Appendix)

| 4 MHz band1 |  | 6 MHz band |  | 8 MHz band2 |  | 12 MHz band3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |  |
| 4 146 4 149 | 4 147.4 4 150.4 | 6 224 6 227 6 230 | 6 225.4 6 228.4 6 231.4 | 8 294 8 297 | 8 295.4 8 298.4 | 12 353 12 356 12 362 12 365 | 12 354.4 12 357.4 12 363.4 12 366.4 |  |
| 1 These frequencies may be used for duplex operation with coast stations operating on Channel Nos. 428 and 429 (see Sub-Section A). 2 These frequencies may be used for duplex operation with coast stations operating on Channel Nos. 834 up to and including 837 (see Sub-Section A). 3 For use of frequencies 12 359 kHz and 16 537 kHz, see Nos. 52.221A and 52.222A. |  |  |  |  |  |  |  |

| 16 MHz band3 |  | 18/19 MHz band |  | 22 MHz band |  | 25/26 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency | Carrier frequency | Assigned frequency |
| 16 528 16 531 16 534 16 540 16 543 16 546 | 16 529.4 16 532.4 16 535.4 16 541.4 16 544.4 16 547.4 | 18 825 18 828 18 831 18 834 18 837 18 840 18 843 | 18 826.4 18 829.4 18 832.4 18 835.4 18 838.4 18 841.4 18 844.4 | 22 159 22 162 22 165 22 168 22 171 22 174 22 177 | 22 160.4 22 163.4 22 166.4 22 169.4 22 172.4 22 175.4 22 178.4 | 25 100 25 103 25 106 25 109 25 112 25 115 25 118 | 25 101.4 25 104.4 25 107.4 25 110.4 25 113.4 25 116.4 25 119.4 |
| 3 For use of frequencies 12 359 kHz and 16 537 kHz, see Nos. 52.221A and 52.222A. |  |  |  |  |  |  |  |  |

Sub-Section C-1

Table of recommended single-sideband transmitting frequencies (kHz) for 
ship stations in the band 4 000-4 063 kHz shared with the fixed service

The frequencies in this Sub-Section may be used:

–	for supplementing ship-to-shore channels for duplex operation in Sub-Section A;

–	for intership simplex (single-frequency) and cross-band operation;

–	for cross-band working with coast stations on channels in Sub-Section C-2;

–	for duplex operation with coast stations working in the band 4 438-4 650 kHz;

–	for duplex operation with Channel Nos. 428 and 429.

| Channel No. | Carrier frequency | Assigned frequency | Channel No. | Carrier frequency | Assigned frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 000* | 4 001.4* | 12 | 4 033 | 4 034.4 |  |
| 2 | 4 003* | 4 004.4* | 13 | 4 036 | 4 037.4 |  |
| 3 | 4 006 | 4 007.4 | 14 | 4 039 | 4 040.4 |  |
| 4 | 4 009 | 4 010.4 | 15 | 4 042 | 4 043.4 |  |
| 5 | 4 012 | 4 013.4 | 16 | 4 045 | 4 046.4 |  |
| 6 | 4 015 | 4 016.4 | 17 | 4 048 | 4 049.4 |  |
| 7 | 4 018 | 4 019.4 | 18 | 4 051 | 4 052.4 |  |
| 8 | 4 021 | 4 022.4 | 19 | 4 054 | 4 055.4 |  |
| 9 | 4 024 | 4 025.4 | 20 | 4 057 | 4 058.4 |  |
| 10 | 4 027 | 4 028.4 | 21 | 4 060 | 4 061.4 |  |
| 11 | 4 030 | 4 031.4 |  |  |  |  |
| * Administrations are requested to urge ship stations under their jurisdiction to refrain from using the band 4 000-4 005 kHz when navigating in Region 3 (see also No. 5.126). |  |  |  |  |  |

Sub-Section C-2

Table of recommended single-sideband transmitting frequencies (kHz) for ship 
and coast stations in the band 8 100-8 195 kHz shared with the fixed service

(See § 7 of Section I of this Appendix)

The frequencies in this Sub-Section may be used:

–	for supplementing ship-to-shore and shore-to-ship channels for duplex operation in Sub-Section A;

–	for intership simplex (single frequency) and cross-band operation;

–	for cross-band working with ship stations on channels in Sub-Section C-1;

–	for ship-to-shore or shore-to-ship simplex operation;

–	for duplex operation with Channel Nos. 834, 835, 836 and 837.

| Channel No. | Carrier frequency | Assigned frequency | Channel No. | Carrier frequency | Assigned frequency |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 101 | 8 102.4 | 17 | 8 149 | 8 150.4 |
| 2 | 8 104 | 8 105.4 | 18 | 8 152 | 8 153.4 |
| 3 | 8 107 | 8 108.4 | 19 | 8 155 | 8 156.4 |
| 4 | 8 110 | 8 111.4 | 20 | 8 158 | 8 159.4 |
| 5 | 8 113 | 8 114.4 | 21 | 8 161 | 8 162.4 |
| 6 | 8 116 | 8 117.4 | 22 | 8 164 | 8 165.4 |
| 7 | 8 119 | 8 120.4 | 23 | 8 167 | 8 168.4 |
| 8 | 8 122 | 8 123.4 | 24 | 8 170 | 8 171.4 |
| 9 | 8 125 | 8 126.4 | 25 | 8 173 | 8 174.4 |
| 10 | 8 128 | 8 129.4 | 26 | 8 176 | 8 177.4 |
| 11 | 8 131 | 8 132.4 | 27 | 8 179 | 8 180.4 |
| 12 | 8 134 | 8 135.4 | 28 | 8 182 | 8 183.4 |
| 13 | 8 137 | 8 138.4 | 29 | 8 185 | 8 186.4 |
| 14 | 8 140 | 8 141.4 | 30 | 8 188 | 8 189.4 |
| 15 | 8 143 | 8 144.4 | 31 | 8 191 | 8 192.4 |
| 16 | 8 146 | 8 147.4 |  |  |  |

Section II  –  Narrow-band direct-printing telegraphy (paired frequencies)

1	Each coast station which uses paired frequencies is assigned one or more frequency pairs from the following series; each pair consists of a transmitting and a receiving frequency.

2	The speed of the narrow-band direct-printing telegraphy and data systems shall not exceed 100 Bd for FSK and 200 Bd for PSK.

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel No. | 4 MHz band |  | 6 MHz band |  | 8 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Transmit | Receive | Transmit | Receive | Transmit | Receive |
| 1 2 3 4 5 | 4 210.5 4 211 4 211.5 4 212 4 212.5 | 4 172.5 4 173 4 173.5 4 174 4 174.5 | 6 314.5 6 315 6 315.5 6 316 6 316.5 | 6 263 6 263.5 6 264 6 264.5 6 265 | 8 376.5 8 417 8 417.5 8 418 8 418.5 | 8 376.5 8 377 8 377.5 8 378 8 378.5 |
| 6 7 8 9 10 | 4 213 4 213.5 4 214 4 214.5 4 215 | 4 175 4 175.5 4 176 4 176.5 4 177 | 6 317 6 317.5 6 318 6 318.5 6 319 | 6 265.5 6 266 6 266.5 6 267 6 267.5 | 8 419 8 419.5 8 420 8 420.5 8 421 | 8 379 8 379.5 8 380 8 380.5 8 381 |
| 11 12 13 14 15 | 4 177.5 4 215.5 4 216 | 4 177.5 4 178 4 178.5 | 6 268 6 319.5 6 320 6 320.5 | 6 268 6 268.5 6 269 6 269.5 | 8 421.5 8 422 8 422.5 8 423 8 423.5 | 8 381.5 8 382 8 382.5 8 383 8 383.5 |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel No. | 12 MHz band |  | 16 MHz band |  | 18/19 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Transmit | Receive | Transmit | Receive | Transmit | Receive |
| 1 2 3 4 5 | 12 579.5 12 580 12 580.5 12 581 12 581.5 | 12 477 12 477.5 12 478 12 478.5 12 479 | 16 807 16 807.5 16 808 16 808.5 16 809 | 16 683.5 16 684 16 684.5 16 685 16 685.5 |  |  |
| 6 7 8 9 10 | 12 582 12 582.5 12 583 12 583.5 12 584 | 12 479.5 12 480 12 480.5 12 481 12 481.5 | 16 809.5 16 810 16 810.5 16 811 16 811.5 | 16 686 16 686.5 16 687 16 687.5 16 688 | 19 684 19 684.5 19 685 19 685.5 | 18 873.5 18 874 18 874.5 18 875 |
| 11 12 13 14 15 | 12 584.5 12 585 12 585.5 12 586 12 586.5 | 12 482 12 482.5 12 483 12 483.5 12 484 | 16 812 16 812.5 16 813 16 813.5 16 814 | 16 688.5 16 689 16 689.5 16 690 16 690.5 | 19 686 19 686.5 19 687 19 687.5 19 688 | 18 875.5 18 876 18 876.5 18 877 18 877.5 |
| 16 17 18 19 20 | 12 587 12 587.5 12 588 12 588.5 12 589 | 12 484.5 12 485 12 485.5 12 486 12 486.5 | 16 814.5 16 815 16 815.5 16 816 16 816.5 | 16 691 16 691.5 16 692 16 692.5 16 693 | 19 688.5 19 689 19 689.5 19 690 19 690.5 | 18 878 18 878.5 18 879 18 879.5 18 880 |
| 21 22 23 24 25 | 12 589.5 12 590 12 590.5 12 591 12 591.5 | 12 487 12 487.5 12 488 12 488.5 12 489 | 16 817 16 817.5 16 818 16 695 16 818.5 | 16 693.5 16 694 16 694.5 16 695 16 695.5 |  |  |
| 26 27 28 29 30 | 12 592 12 592.5 12 593 12 593.5 12 594 | 12 489.5 12 490 12 490.5 12 491 12 491.5 | 16 819 16 819.5 16 820 16 820.5 16 821 | 16 696 16 696.5 16 697 16 697.5 16 698 |  |  |
| 31 32 33 34 35 | 12 594.5 12 595 12 595.5 12 596 12 596.5 | 12 492 12 492.5 12 493 12 493.5 12 494 | 16 821.5 | 16 698.5 |  |  |
| 36 37 38 39 40 | 12 597 12 597.5 12 598 12 598.5 12 599 | 12 494.5 12 495 12 495.5 12 496 12 496.5 |  |  |  |  |
| 41 42 43 44 45 | 12 599.5 12 600 12 600.5 12 601 12 601.5 | 12 497 12 497.5 12 498 12 498.5 12 499 |  |  |  |  |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel No. | 12 MHz band (end) |  |
| --- | --- | --- |
|  | Transmit | Receive |
| 46 47 48 49 50 | 12 602 12 602.5 12 603 12 603.5 12 604 | 12 499.5 12 500 12 500.5 12 501 12 501.5 |
| 51 52 53 54 55 | 12 604.5 12 605 12 605.5 12 606 12 606.5 | 12 502 12 502.5 12 503 12 503.5 12 504 |
| 56 57 58 59 60 | 12 607 12 607.5 12 608 12 608.5 12 609 | 12 504.5 12 505 12 505.5 12 506 12 506.5 |
| 61 62 63 64 65 | 12 609.5 12 610 12 610.5 12 611 12 611.5 | 12 507 12 507.5 12 508 12 508.5 12 509 |
| 66 67 68 69 70 | 12 612 12 612.5 12 613 12 613.5 12 614 | 12 509.5 12 510 12 510.5 12 511 12 511.5 |
| 71 72 73 74 75 | 12 614.5 12 615 12 615.5 12 616 12 616.5 | 12 512 12 512.5 12 513 12 513.5 12 514 |
| 76 77 78 79 80 | 12 617 12 617.5 12 618 12 618.5 12 619 | 12 514.5 12 515 12 515.5 12 516 12 516.5 |
| 81 82 83 84 85 | 12 619.5 12 620 12 620.5 12 621 12 621.5 | 12 517 12 517.5 12 518 12 518.5 12 519 |
| 86 87 88 89 90 | 12 622 12 520 12 622.5 12 623 12 623.5 | 12 519.5 12 520 12 520.5 12 521 12 521.5 |
| 91 92 | 12 624 12 624.5 | 12 522 12 522.5 |

Table of frequencies for two-frequency operation by coast stations (kHz)

| Channel No. | 22 MHz band |  |
| --- | --- | --- |
|  | Transmit | Receive |
| 13 14 15 | 22 382.5 22 383 22 383.5 | 22 290.5 22 291 22 291.5 |
| 16 17 18 19 20 | 22 384 22 384.5 22 385 22 385.5 22 386 | 22 292 22 292.5 22 293 22 293.5 22 294 |
| 21 22 23 24 25 | 22 386.5 22 387 22 387.5 22 388 22 388.5 | 22 294.5 22 295 22 295.5 22 296 22 296.5 |
| 26 | 22 389 | 22 297 |

Section III  –  Narrow-band direct-printing telegraphy 
(non-paired frequencies)

1	One or more frequencies are assigned to each ship station as transmitting frequencies.

2	All frequencies appearing in this Appendix may be used for NBDP duplex operation.

3	The speed of the narrow-band direct-printing telegraphy and data systems shall not exceed 100 Bd for FSK and 200 Bd for PSK.

| Frequency bands |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel No. | 4 MHz | 6 MHz | 8 MHz | 12 MHz | 16 MHz | 18/19 MHz | 22 MHz | 25/26 MHz |
| 1 2 3 4 5 | 4 170.5 4 171 4 171.5 4 172 4 179 | 6 260.25 6 260.75 6 321 6 321.5 | 8 339.25 8 339.75 8 375 8 375.5 8 376 | 12 419.25 12 419.75 12 422 12 476.5 12 655 | 16 615.25 16 615.75 16 616.25 16 616.75 16 682 | 19 691 | 22 290 22 297.5 22 298 22 298.5 22 299 | 26 101 26 101.5 26 102 26 102.5 |
| 6 7 8 | 4 179.5 4 180 |  |  | 12 655.5 12 656 12 656.5 | 16 682.5 16 683 |  | 22 443.5 |  |

Section IV  –  Data transmission

Table of frequencies (kHz) assignable to ship and coast stations
for data transmission (kHz)1

| Channel No. | 4 MHz band |  | 6 MHz band |  | 8 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) |
| 1 2 3 4 5 |  | 4 153.5 3, 4 4 156.5 3, 4 4 159.5 3, 4 4 162.5 3, 4 4 165.5 3, 4 |  | 6 234.5 3, 4 6 237.5 3, 4 6 240.5 3, 4 6 243.5 3, 4 6 246.5 3, 4 |  | 8 301.5 3, 4 8 304.5 3, 4 8 307.5 3, 4 8 310.5 3, 4 8 313.5 3, 4 |
| 6 7 8 9 10 | 4 199.75 4 202.75 4 205.75 4 190.75 2, 3 | 4 168.5 3, 4 4 181.75 4 184.75 4 187.75 4 190.75 2, 3 | 6 323.25 | 6 249.5 3, 4 6 252.5 3, 4 6 255.5 3, 4 6 258.5 3, 4 6 271.25 |  | 8 316.5 3, 4 8 319.5 3, 4 8 322.5 3, 4 8 325.5 3, 4 8 328.5 3, 4 |
| 11 12 13 14 15 | 4 193.75 2, 3 4 196.75 2, 3 4 217.75 2 | 4 193.75 2, 3 4 196.75 2, 3 4 217.75 2 | 6 326.25 6 329.25 6 280.25 2, 3 6 283.25 2, 3 6 286.25 2, 3 | 6 274.25 6 277.25 6 280.25 2, 3 6 283.25 2, 3 6 286.25 2, 3 | 8 409.5 8 412.5 | 8 331.5 3, 4 8 334.5 3, 4 8 337.5 3, 4 8 343.25 8 346.25 |
| 16 17 18 19 20 |  |  | 6 289.25 2, 3 6 292.25 2, 3 6 295.25 2, 3 6 298.25 2, 3 6 301.25 2, 3 | 6 289.25 2, 3 6 292.25 2, 3 6 295.25 2, 3 6 298.25 2, 3 6 301.25 2, 3 | 8 425.5 8 428.5 3 8 431.5 3 8 434.5 3 8 361.25 2, 3 | 8 349.25 8 352.25 3 8 355.25 3 8 358.25 3 8 361.25 2, 3 |
| 21 22 23 24 25 |  |  | 6 304.25 2, 3 6 307.25 2, 3 6 310.25 2, 3 | 6 304.25 2, 3 6 307.25 2, 3 6 310.25 2, 3 | 8 364.25 2, 3 8 367.25 2, 3 8 370.25 2, 3 8 373.25 2, 3 8 385.5 2, 3 | 8 364.25 2, 3 8 367.25 2, 3 8 370.25 2, 3 8 373.25 2, 3 8 385.5 2, 3 |
| 26 27 28 29 30 |  |  |  |  | 8 388.5 2, 3 8 391.5 2, 3 8 394.5 2, 3 8 397.5 2, 3 8 400.5 2, 3 | 8 388.5 2, 3 8 391.5 2, 3 8 394.5 2, 3 8 397.5 2, 3 8 400.5 2, 3 |
| 31 32 |  |  |  |  | 8 403.5 2, 3 8 406.5 2, 3 | 8 403.5 2, 3 8 406.5 2, 3 |

Table of frequencies (kHz) assignable to ship and coast stations
for data transmission (kHz)1

| Channel No. | 12 MHz |  | 16 MHz |  | 18/19 MHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) |
| 1 2 3 4 5 |  | 12 369.5 3, 4 12 372.5 3, 4 12 375.5 3, 4 12 378.5 3, 4 12 381.5 3, 4 |  | 16 550.5 3, 4 16 553.5 3, 4 16 556.5 3, 4 16 559.5 3, 4 16 562.5 3, 4 |  | 18 847.5 3, 4 18 850.5 3, 4 18 853.5 3, 4 18 856.5 3, 4 18 859.5 3, 4 |
| 6 7 8 9 10 |  | 12 384.5 3, 4 12 387.5 3, 4 12 390.5 3, 4 12 393.5 3, 4 12 396.5 3, 4 |  | 16 565.5 3, 4 16 568.5 3, 4 16 571.5 3, 4 16 574.5 3, 4 16 577.5 3, 4 | 19 682.25 | 18 862.5 3, 4 18 865.5 3, 4 18 868.5 3, 4 18 871.5 3, 4 18 881.75 |
| 11 12 13 14 15 |  | 12 399.5 3, 4 12 402.5 3, 4 12 405.5 3, 4 12 408.5 3, 4 12 411.5 3, 4 |  | 16 580.5 3, 4 16 583.5 3, 4 16 586.5 3, 4 16 589.5 3, 4 16 592.5 3, 4 | 19 692.75 19 695.75 3 19 698.75 3 19 701.75 3 18 896.75 2 | 18 884.75 18 887.75 3 18 890.75 3 18 893.75 3 18 896.75 2 |
| 16 17 18 19 20 | 12 626.25 12 629.25 12 632.25 | 12 414.5 3, 4 12 417.5 3, 4 12 423.75 12 426.75 12 429.75 |  | 16 595.5 3, 4 16 598.5 3, 4 16 601.5 3, 4 16 604.5 3, 4 16 607.5 3, 4 |  |  |
| 21 22 23 24 25 | 12 635.25 12 638.25 3 12 641.25 3 12 644.25 3 12 647.25 3 | 12 432.75 12 435.75 3 12 438.75 3 12 441.75 3 12 444.75 3 | 16 841.25 16 844.25 16 847.25 | 16 610.5 3, 4 16 613.5 3, 4 16 620.25 16 623.25 16 626.25 |  |  |
| 26 27 28 29 30 | 12 650.25 3 12 653.25 3 12 453.75 2, 3 12 456.75 2, 3 12 459.75 2, 3 | 12 447.75 3 12 450.75 3 12 453.75 2, 3 12 456.75 2, 3 12 459.75 2, 3 | 16 850.25 16 853.25 16 856.25 16 859.25 16 862.25 | 16 629.25 16 632.25 16 635.25 16 638.25 16 641.25 |  |  |
| 31 32 33 34 35 | 12 462.75 2, 3 12 465.75 2, 3 12 468.75 2, 3 12 471.75 2, 3 12 474.75 2, 3 | 12 462.75 2, 3 12 465.75 2, 3 12 468.75 2, 3 12 471.75 2, 3 12 474.75 2, 3 | 16 865.25 16 868.25 3 16 871.25 3 16 874.25 3 16 877.25 3 | 16 644.25 16 647.25 3 16 650.25 3 16 653.25 3 16 656.25 3 |  |  |
| 36 37 38 39 40 | 12 524.25 2, 3 12 527.25 2, 3 12 530.25 2, 3 12 533.25 2, 3 12 536.25 2, 3 | 12 524.25 2, 3 12 527.25 2, 3 12 530.25 2, 3 12 533.25 2, 3 12 536.25 2, 3 | 16 880.25 3 16 883.25 3 16 886.25 3 16 889.25 3 16 892.25 3 | 16 659.25 3 16 662.25 3 16 665.25 3 16 668.25 3 16 671.25 3 |  |  |
| 41 42 43 44 45 | 12 539.25 2, 3 12 542.25 2, 3 12 545.25 2, 3 12 548.25 2, 3 12 551.25 2, 3 | 12 539.25 2, 3 12 542.25 2, 3 12 545.25 2, 3 12 548.25 2, 3 12 551.25 2, 3 | 16 895.25 3 16 898.25 3 16 901.25 3 16 700.5 2, 3 16 703.5 2, 3 | 16 674.25 3 16 677.25 3 16 680.25 3 16 700.5 2, 3 16 703.5 2, 3 |  |  |

| Channel No. | 12 MHz (end) |  | 16 MHz (end) |  |
| --- | --- | --- | --- | --- |
|  | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) |
| 46 47 48 49 50 | 12 554.25 2, 3 12 557.25 2, 3 12 560.25 2, 3 12 563.25 2, 3 12 566.25 2, 3 | 12 554.25 2, 3 12 557.25 2, 3 12 560.25 2, 3 12 563.25 2, 3 12 566.25 2, 3 | 16 706.5 2, 3 16 709.5 2, 3 16 712.5 2, 3 16 715.5 2, 3 16 718.5 2, 3 | 16 706.5 2, 3 16 709.5 2, 3 16 712.5 2, 3 16 715.5 2, 3 16 718.5 2, 3 |
| 51 52 53 54 55 | 12 569.25 2, 3 12 572.25 2, 3 12 575.25 2, 3 | 12 569.25 2, 3 12 572.25 2, 3 12 575.25 2, 3 | 16 721.5 2, 3 16 724.5 2, 3 16 727.5 2, 3 16 730.5 2, 3 16 733.5 2, 3 | 16 721.5 2, 3 16 724.5 2, 3 16 727.5 2, 3 16 730.5 2, 3 16 733.5 2, 3 |
| 56 57 58 59 60 |  |  | 16 736.5 2, 3 16 739.5 2, 3 16 742.5 2, 3 16 745.5 2, 3 16 748.5 2, 3 | 16 736.5 2, 3 16 739.5 2, 3 16 742.5 2, 3 16 745.5 2, 3 16 748.5 2, 3 |
| 61 62 63 64 65 |  |  | 16 751.5 2, 3 16 754.5 2, 3 16 757.5 2, 3 16 760.5 2, 3 16 763.5 2, 3 | 16 751.5 2, 3 16 754.5 2, 3 16 757.5 2, 3 16 760.5 2, 3 16 763.5 2, 3 |
| 66 67 68 69 70 |  |  | 16 766.5 2, 3 16 769.5 2, 3 16 772.5 2, 3 16 775.5 2, 3 16 778.5 2, 3 | 16 766.5 2, 3 16 769.5 2, 3 16 772.5 2, 3 16 775.5 2, 3 16 778.5 2, 3 |
| 71 72 73 74 75 |  |  | 16 781.5 2, 3 16 784.5 2, 3 16 787.5 2, 3 16 790.5 2, 3 16 793.5 2, 3 | 16 781.5 2, 3 16 784.5 2, 3 16 787.5 2, 3 16 790.5 2, 3 16 793.5 2, 3 |
| 76 77 78 79 80 |  |  | 16 796.5 2, 3 16 799.5 2, 3 16 802.5 2, 3 16 823.25 2, 3 16 826.25 2, 3 | 16 796.5 2, 3 16 799.5 2, 3 16 802.5 2, 3 16 823.25 2, 3 16 826.25 2, 3 |
| 81 82 83 84 |  |  | 16 829.25 2, 3 16 832.25 2, 3 16 835.25 2, 3 16 838.25 2, 3 | 16 829.25 2, 3 16 832.25 2, 3 16 835.25 2, 3 16 838.25 2, 3 |

Table of frequencies (kHz) assignable to ship and coast stations
for data transmission (kHz)1

| Channel No. | 22 MHz |  | 25/26 MHz |  |
| --- | --- | --- | --- | --- |
|  | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) |
| 1 2 3 4 5 |  | 22 181.5 3, 4 22 184.5 3, 4 22 187.5 3, 4 22 190.5 3, 4 22 193.5 3, 4 |  | 25 122.5 3, 4 25 125.5 3, 4 25 128.5 3, 4 25 131.5 3, 4 25 134.5 3, 4 |
| 6 7 8 9 10 |  | 22 196.5 3, 4 22 199.5 3, 4 22 202.5 3, 4 22 205.5 3, 4 22 208.5 3, 4 |  | 25 137.5 3, 4 25 140.5 3, 4 25 143.5 3, 4 25 146.5 3, 4 25 149.5 3, 4 |
| 11 12 13 14 15 |  | 22 211.5 3, 4 22 214.5 3, 4 22 217.5 3, 4 22 220.5 3, 4 22 223.5 3, 4 | 26 104.25 26 107.25 | 25 152.5 3, 4 25 155.5 3, 4 25 158.5 3, 4 25 161.5 25 164.5 |
| 16 17 18 19 20 |  | 22 226.5 3, 4 22 229.5 3, 4 22 232.5 3, 4 22 235.5 3, 4 22 238.5 3, 4 | 26 110.25 26 113.25 3 26 116.25 3 26 119.25 3 25 179.5 2, 3 | 25 167.5 25 170.5 3 25 173.5 3 25 176.5 3 25 179.5 2, 3 |
| 21 22 23 24 25 | 22 390.75 22 393.75 22 396.75 22 399.75 22 402.75 | 22 243.25 22 246.25 22 249.25 22 252.25 22 255.25 | 25 182.5 2, 3 25 185.5 2, 3 25 188.5 2, 3 25 191.5 2, 3 25 194.5 2, 3 | 25 182.5 2, 3 25 185.5 2, 3 25 188.5 2, 3 25 191.5 2, 3 25 194.5 2, 3 |
| 26 27 28 29 30 | 22 405.75 22 408.75 3 22 411.75 3 22 414.75 3 22 417.75 3 | 22 258.25 22 261.25 3 22 264.25 3 22 267.25 3 22 270.25 3 | 25 197.5 2, 3 25 200.5 2, 3 25 203.5 2, 3 25 206.5 2, 3 | 25 197.5 2, 3 25 200.5 2, 3 25 203.5 2, 3 25 206.5 2, 3 |
| 31 32 33 34 35 | 22 420.75 3 22 423.75 3 22 426.75 3 22 429.75 3 22 432.75 3 | 22 273.25 3 22 276.25 3 22 279.25 3 22 282.25 3 22 285.25 3 |  |  |
| 36 37 38 39 40 | 22 435.75 3 22 300.75 2, 3 22 303.75 2, 3 22 306.75 2, 3 22 309.75 2, 3 | 22 288.25 3 22 300.75 2, 3 22 303.75 2, 3 22 306.75 2, 3 22 309.75 2, 3 |  |  |
| 41 42 43 44 45 | 22 312.75 2, 3 22 315.75 2, 3 22 318.75 2, 3 22 321.75 2, 3 22 324.75 2, 3 | 22 312.75 2, 3 22 315.75 2, 3 22 318.75 2, 3 22 321.75 2, 3 22 324.75 2, 3 |  |  |

| Channel No. | 22 MHz (end) |  |
| --- | --- | --- |
|  | Coast Tx (ship Rx) | Ship Tx/Rx (coast Rx) |
| 46 47 48 49 50 | 22 327.75 2, 3 22 330.75 2, 3 22 333.75 2, 3 22 336.75 2, 3 22 339.75 2, 3 | 22 327.75 2, 3 22 330.75 2, 3 22 333.75 2, 3 22 336.75 2, 3 22 339.75 2, 3 |
| 51 52 53 54 55 | 22 342.75 2, 3 22 345.75 2, 3 22 348.75 2, 3 22 351.75 2, 3 22 354.75 2, 3 | 22 342.75 2, 3 22 345.75 2, 3 22 348.75 2, 3 22 351.75 2, 3 22 354.75 2, 3 |
| 56 57 58 59 60 | 22 357.75 2, 3 22 360.75 2, 3 22 363.75 2, 3 22 366.75 2, 3 22 369.75 2, 3 | 22 357.75 2, 3 22 360.75 2, 3 22 363.75 2, 3 22 366.75 2, 3 22 369.75 2, 3 |
| 61 62 63 | 22 372.75 2, 3 22 438.75 22 441.75 | 22 372.75 2, 3 22 377.75 22 380.75 |
| 1 The data transmission should be in accordance with the most recent version of Recommendation ITU-R M.1798. 2 Non-paired (simplex) operations only. 3 Assignable for wide-band operation using multiple 3 kHz contiguous channels. 4 Channels may be paired with wide-band coast station channels in the same band. |  |  |

APPENDIX 18 (REV.WRC-15)

Table of transmitting frequencies in the
VHF maritime mobile band

(See Article 52)

NOTE A – For assistance in understanding the Table, see Notes a) to zz) below.     (WRC-15)

NOTE B – The Table below defines the channel numbering for maritime VHF communications based on 25 kHz channel spacing and use of several duplex channels. The channel numbering and the conversion of two-frequency channels for single-frequency operation shall be in accordance with Recommendation ITU-R M.1084-5 Annex 4, Tables 1 and 3. The Table below also describes the harmonized channels where the digital technologies defined in the most recent version of Recommendation ITU-R M.1842 could be deployed.     (WRC-15)

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 60 | m) | 156.025 | 160.625 |  | x | x | x |
| 01 | m) | 156.050 | 160.650 |  | x | x | x |
| 61 | m) | 156.075 | 160.675 |  | x | x | x |
| 02 | m) | 156.100 | 160.700 |  | x | x | x |
| 62 | m) | 156.125 | 160.725 |  | x | x | x |
| 03 | m) | 156.150 | 160.750 |  | x | x | x |
| 63 | m) | 156.175 | 160.775 |  | x | x | x |
| 04 | m) | 156.200 | 160.800 |  | x | x | x |
| 64 | m) | 156.225 | 160.825 |  | x | x | x |
| 05 | m) | 156.250 | 160.850 |  | x | x | x |
| 65 | m) | 156.275 | 160.875 |  | x | x | x |
| 06 | f) | 156.300 |  | x |  |  |  |
| 2006 | r) | 160.900 | 160.900 |  |  |  |  |
| 66 | m) | 156.325 | 160.925 |  | x | x | x |
| 07 | m) | 156.350 | 160.950 |  | x | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |
| 10 | h), q) | 156.500 | 156.500 | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 | q) | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n), s) | 156.775 | 156.775 |  | x |  |  |
| 16 | f) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n), s) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | m) | 156.925 | 161.525 |  | x | x | x |
| 1078 |  | 156.925 | 156.925 |  | x |  |  |
| 2078 | mm) |  | 161.525 |  | x |  |  |
| 19 | m) | 156.950 | 161.550 |  | x | x | x |
| 1019 |  | 156.950 | 156.950 |  | x |  |  |
| 2019 | mm) |  | 161.550 |  | x |  |  |
| 79 | m) | 156.975 | 161.575 |  | x | x | x |
| 1079 |  | 156.975 | 156.975 |  | x |  |  |
| 2079 | mm) |  | 161.575 |  | x |  |  |
| 20 | m) | 157.000 | 161.600 |  | x | x | x |
| 1020 |  | 157.000 | 157.000 |  | x |  |  |
| 2020 | mm) |  | 161.600 |  | x |  |  |
| 80 | y), wa) | 157.025 | 161.625 |  | x | x | x |
| 21 | y), wa) | 157.050 | 161.650 |  | x | x | x |
| 81 | y), wa) | 157.075 | 161.675 |  | x | x | x |
| 22 | y), wa) | 157.100 | 161.700 |  | x | x | x |
| 82 | x), y), wa) | 157.125 | 161.725 |  | x | x | x |
| 23 | x), y), wa) | 157.150 | 161.750 |  | x | x | x |
| 83 | x), y), wa) | 157.175 | 161.775 |  | x | x | x |
| 24 | w), ww), x), xx) | 157.200 | 161.800 |  | x | x | x |
| 1024 | w), ww), x), xx) | 157.200 |  |  |  |  |  |
| 2024 | w), ww), x), xx) | 161.800 | 161.800 | x (digital only) |  |  |  |
| 84 | w), ww), x), xx) | 157.225 | 161.825 |  | x | x | x |
| 1084 | w), ww), x), xx) | 157.225 |  |  |  |  |  |
| 2084 | w), ww), x), xx) | 161.825 | 161.825 | x (digital only) |  |  |  |
| 25 | w), ww), x), xx) | 157.250 | 161.850 |  | x | x | x |
| 1025 | w), ww), x), xx) | 157.250 |  |  |  |  |  |
| 2025 | w), ww), x), xx) | 161.850 | 161.850 | x (digital only) |  |  |  |
| 85 | w), ww), x), xx) | 157.275 | 161.875 |  | x | x | x |
| 1085 | w), ww), x), xx) | 157.275 |  |  |  |  |  |
| 2085 | w), ww), x), xx) | 161.875 | 161.875 | x (digital only) |  |  |  |
| 26 | w), ww), x) | 157.300 | 161.900 |  | x | x | x |
| 1026 | w), ww), x) | 157.300 |  |  |  |  |  |
| 2026 | w), ww), x) |  | 161.900 |  |  |  |  |
| 86 | w), ww), x) | 157.325 | 161.925 |  | x | x | x |
| 1086 | w), ww), x) | 157.325 |  |  |  |  |  |
| 2086 | w), ww), x) |  | 161.925 |  |  |  |  |
| 27 | z), zx) | 157.350 | 161.950 |  |  | x | x |
| 1027 | z), zz) | 157.350 | 157.350 |  | x |  |  |
| 2027* | z) | 161.950 | 161.950 |  |  |  |  |
| 87 | z), zz) | 157.375 | 157.375 |  | x |  |  |
| 28 | z), zx) | 157.400 | 162.000 |  |  | x | x |
| 1028 | z), zz) | 157.400 | 157.400 |  | x |  |  |
| 2028* | z) | 162.000 | 162.000 |  |  |  |  |
| 88 | z), zz) | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f), l), p) | 162.025 | 162.025 |  |  |  |  |
| * From 1 January 2019, channel 2027 will be designated ASM 1 and channel 2028 will be designated ASM 2. |  |  |  |  |  |  |  |

Notes referring to the Table

General notes

a)	Administrations may designate frequencies in the inter-ship, port operations and ship movement services for use by light aircraft and helicopters to communicate with ships or participating coast stations in predominantly maritime support operations under the conditions specified in Nos. 51.69, 51.73, 51.74, 51.75, 51.76, 51.77 and 51.78. However, the use of the channels which are shared with public correspondence shall be subject to prior agreement between interested and affected administrations.

b)	The channels of the present Appendix, with the exception of channels 06, 13, 15, 16, 17, 70, 75 and 76, may also be used for high-speed data and facsimile transmissions, subject to special arrangement between interested and affected administrations.

c)	The channels of the present Appendix, with the exception of channels 06, 13, 15, 16, 17, 70, 75 and 76, may be used for direct-printing telegraphy and data transmission, subject to special arrangement between interested and affected administrations.     (WRC-12)

d)	The frequencies in this table may also be used for radiocommunications on inland waterways in accordance with the conditions specified in No. 5.226.

e)	Administrations may apply 12.5 kHz channel interleaving on a non-interference basis to 25 kHz channels, in accordance with the most recent version of Recommendation ITU-R M.1084, provided:

–	it shall not affect the 25 kHz channels of the present Appendix maritime mobile distress and safety, automatic identification system (AIS), and data exchange frequencies, especially the channels 06, 13, 15, 16, 17, 70, AIS 1 and AIS 2, nor the technical characteristics set forth in Recommendation ITU-R M.489-2 for those channels;

–	implementation of 12.5 kHz channel interleaving and consequential national requirements shall be subject to coordination with affected administrations.     (WRC-12)

Specific notes

f)	The frequencies 156.300 MHz (channel 06), 156.525 MHz (channel 70), 156.800 MHz (channel 16), 161.975 MHz (AIS 1) and 162.025 MHz (AIS 2) may also be used by aircraft stations for the purpose of search and rescue operations and other safety-related communication.     (WRC-07)

g)	Channels 15 and 17 may also be used for on-board communications provided the effective radiated power does not exceed 1 W, and subject to the national regulations of the administration concerned when these channels are used in its territorial waters.

h)	Within the European Maritime Area and in Canada, these frequencies (channels 10, 67, 73) may also be used, if so required, by the individual administrations concerned, for communication between ship stations, aircraft stations and participating land stations engaged in coordinated search and rescue and anti-pollution operations in local areas, under the conditions specified in Nos. 51.69, 51.73, 51.74, 51.75, 51.76, 51.77 and 51.78.

i)	The preferred first three frequencies for the purpose indicated in Note a) are 156.450 MHz (channel 09), 156.625 MHz (channel 72) and 156.675 MHz (channel 73).

j)	Channel 70 is to be used exclusively for digital selective calling for distress, safety and calling.

k)	Channel 13 is designated for use on a worldwide basis as a navigation safety communication channel, primarily for intership navigation safety communications. It may also be used for the ship movement and port operations service subject to the national regulations of the administrations concerned.

l)	These channels (AIS 1 and AIS 2) are used for an automatic identification system (AIS) capable of providing worldwide operation, unless other frequencies are designated on a regional basis for this purpose. Such use should be in accordance with the most recent version of Recommendation ITU-R M.1371.     (WRC-07)

m)	These channels may be operated as single frequency channels, subject to coordination with affected administrations. The following conditions apply for single frequency usage:

–	The lower frequency portion of these channels may be operated as single frequency channels by ship and coast stations.

–	Transmission using the upper frequency portion of these channels is limited to coast stations.

–	If permitted by administrations and specified by national regulations, the upper frequency portion of these channels may be used by ship stations for transmission. All precautions should be taken to avoid harmful interference to channels AIS 1, AIS 2, 2027* and 2028*.     (WRC-15)

* From 1 January 2019, channel 2027 will be designated ASM 1 and channel 2028 will be designated ASM 2.

mm)	Transmission on these channels is limited to coast stations. If permitted by administrations and specified by national regulations, these channels may be used by ship stations for transmission. All precautions should be taken to avoid harmful interference to channels AIS 1, AIS 2, 2027* and 2028*.     (WRC-15)

* From 1 January 2019, channel 2027 will be designated ASM 1 and channel 2028 will be designated ASM 2.

n)	With the exception of AIS, the use of these channels (75 and 76) should be restricted to navigation-related communications only and all precautions should be taken to avoid harmful interference to channel 16, by limiting the output power to 1 W.     (WRC-12)

o)	(SUP - WRC-12)

p)	Additionally, AIS 1 and AIS 2 may be used by the mobile-satellite service (Earth-to-space) for the reception of AIS transmissions from ships.     (WRC-07)

q)	When using these channels (10 and 11), all precautions should be taken to avoid harmful interference to channel 70.     (WRC-07)

r)	In the maritime mobile service, this frequency is reserved for experimental use for future applications or systems (e.g. new AIS applications, man over board systems, etc.). If authorized by administrations for experimental use, the operation shall not cause harmful interference to, or claim protection from, stations operating in the fixed and mobile services.     (WRC-12)

s)	Channels 75 and 76 are also allocated to the mobile-satellite service (Earth-to-space) for the reception of long-range AIS broadcast messages from ships (Message 27; see the most recent version of Recommendation ITU-R M.1371).     (WRC-12)

t)	(SUP - WRC-15)

u)	(SUP - WRC-15)

v)	(SUP - WRC-15)

w)	In Regions 1 and 3:

Until 1 January 2017, the frequency bands 157.200-157.325 MHz and 161.800-161.925 MHz (corresponding to channels: 24, 84, 25, 85, 26 and 86) may be used for digitally modulated emissions, subject to coordination with affected administrations. Stations using these channels or frequency bands for digitally modulated emissions shall not cause harmful interference to, or claim protection from, other stations operating in accordance with Article 5.

From 1 January 2017, the frequency bands 157.200-157.325 MHz and 161.800-161.925 MHz (corresponding to channels: 24, 84, 25, 85, 26 and 86) are identified for the utilization of the VHF Data Exchange System (VDES) described in the most recent version of Recommendation ITU-R M.2092. These frequency bands may also be used for analogue modulation described in the most recent version of Recommendation ITU-R M.1084 by an administration that wishes to do so, subject to not causing harmful interference to, or claiming protection from other stations in the maritime mobile service using digitally modulated emissions and subject to coordination with affected administrations.     (WRC-15)

wa) 	In Regions 1 and 3:

Until 1 January 2017, the frequency bands 157.025-157.175 MHz and 161.625-161.775 MHz (corresponding to channels: 80, 21, 81, 22, 82, 23 and 83) may be used for digitally modulated emissions, subject to coordination with affected administrations. Stations using these channels or frequency bands for digitally modulated emissions shall not cause harmful interference to, or claim protection from, other stations operating in accordance with Article 5.

From 1 January 2017, the frequency bands 157.025-157.100 MHz and 161.625-161.700 MHz (corresponding to channels: 80, 21, 81 and 22) are identified for utilization of the digital systems described in the most recent version of Recommendation ITU-R M.1842 using multiple 25 kHz contiguous channels.

From 1 January 2017, the frequency bands 157.150-157.175 MHz and 161.750-161.775 MHz (corresponding to channels: 23 and 83) are identified for utilization of the digital systems described in the most recent version of Recommendation ITU-R M.1842 using two 25 kHz contiguous channels. From 1 January 2017, the frequencies 157.125 MHz and 161.725 MHz (corresponding to channel: 82) are identified for the utilization of the digital systems described in the most recent version of Recommendation ITU-R M.1842.

The frequency bands 157.025-157.175 MHz and 161.625-161.775 MHz (corresponding to channels: 80, 21, 81, 22, 82, 23 and 83) can also be used for analogue modulation described in the most recent version of Recommendation ITU-R M.1084 by an administration that wishes to do so, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions and subject to coordination with affected administrations.     (WRC-15)

ww)	In Region 2, the frequency bands 157.200-157.325 and 161.800-161.925 MHz (corresponding to channels: 24, 84, 25, 85, 26 and 86) are designated for digitally modulated emissions in accordance with the most recent version of Recommendation ITU-R M.1842.

In Canada and Barbados, from 1 January 2019 the frequency bands 157.200-157.275 and 161.800-161.875 MHz (corresponding to channels: 24, 84, 25 and 85) may be used for digitally modulated emissions, such as those described in the most recent version of Recommendation ITU-R M.2092, subject to coordination with affected administrations.     (WRC-15)

x)	From 1 January 2017, in Angola, Botswana, Lesotho, Madagascar, Malawi, Mauritius, Mozambique, Namibia, Democratic Republic of the Congo, Seychelles, South Africa, Swaziland, Tanzania, Zambia and Zimbabwe, the frequency bands 157.125-157.325 and 161.725-161.925 MHz (corresponding to channels: 82, 23, 83, 24, 84, 25, 85, 26 and 86) are designated for digitally modulated emissions.

From 1 January 2017, in China, the frequency bands 157.150-157.325 and 161.750-161.925 MHz (corresponding to channels: 23, 83, 24, 84, 25, 85, 26 and 86) are designated for digitally modulated emissions.     (WRC-12)

xx)	From 1 January 2019, the channels 24, 84, 25 and 85 may be merged in order to form a unique duplex channel with a bandwidth of 100 kHz in order to operate the VDES terrestrial component described in the most recent version of Recommendation ITU-R M.2092.     (WRC-15)

y)	These channels may be operated as single or duplex frequency channels, subject to coordination with affected administrations.     (WRC-12)

z)	Until 1 January 2019, these channels may be used for possible testing of future AIS applications without causing harmful interference to, or claiming protection from, existing applications and stations operating in the fixed and mobile services.

From 1 January 2019, these channels are each split into two simplex channels. The channels 2027 and 2028 designated as ASM 1 and ASM 2 are used for application specific messages (ASM) as described in the most recent version of Recommendation ITU-R M.2092.     (WRC-15)

zx)	In the United States, these channels are used for communication between ship stations and coast stations for the purpose of public correspondence.     (WRC-15)

zz)	From 1 January 2019, channels 1027, 1028, 87 and 88 are used as single-frequency analogue channels for port operation and ship movement.     (WRC-15)

APPENDIX 25 (REV.WRC-03)*

Provisions and associated frequency allotment Plan for coast radiotelephone stations operating in the exclusive maritime mobile bands 
between 4 000 kHz and 27 500 kHz

The provisions of this Appendix shall apply to the maritime mobile radiotelephone bands reserved for duplex operation (two-frequency channels) between 4 000 kHz and 27 500 kHz (see Appendix 17). Section I contains the procedure for bringing up to date the frequency allotment Plan for coast stations. The allotment Plan is contained in Section II of this Appendix.

25/1	Section I – Procedure for bringing up to date the frequency allotment plan

25/1.1	1	Before notifying to the Radiocommunication Bureau or bringing into use at any coast radiotelephone station a frequency assignment not covered by an allotment in the Frequency Allotment Plan contained in Section II of this Appendix, an administration which

25/1.1.1	intends to establish a coast radiotelephone station and has no allotment in the Plan, or

25/1.1.2	intends to expand its coast radiotelephone service and requires an additional allotment,

shall send the information listed in Appendix 4 to the Bureau not earlier than two years in the case of No. 25/1.1.1, or not earlier than six months in the case of No. 25/1.1.2, before the projected date of bringing into service of the planned coast radiotelephone service but in any case not later than three months before that date.

25/1.2		The Bureau shall publish the information sent under No. 25/1.1 in a Special Section of the BR International Frequency Information Circular (BR IFIC) together with such apparent incompatibilities between the proposed allotment which is the subject of the publication and any other existing or proposed allotments which the Bureau can identify. The Bureau shall also indicate any information of a technical nature and make such suggestions as it may be able to offer with a view to avoiding these incompatibilities.

25/1.3		If it is requested by any administration, particularly by an administration of a country in need of special assistance, and if the circumstances appear to warrant, the Bureau, using such means at its disposal as are appropriate in the circumstances, shall render the following assistance:

25/1.3.1	indication of a suitable channel or channels for the service projected by the administration before that administration submits the information for publication;

25/1.3.2	carry out the procedure for which provision is made in No. 25/1.4;

25/1.3.3	any other assistance of a technical nature for completion of the procedure in this Section.

25/1.4	2	At the same time as sending the information listed in Appendix 4 to the Bureau for publication, an administration shall seek the agreement of the administrations having an allotment in the same channel as the proposed allotment. A copy of the relevant correspondence shall be sent to the Bureau.

25/1.5		Any administration which, upon examining the information published by the Bureau, considers that its existing services or services planned within the time-limits mentioned in No. 25/1.1 would be affected shall have the right to be brought into the procedure undertaken pursuant to No. 25/1.4.

25/1.6	3	An administration which receives a request under No. 25/1.4 shall acknowledge receipt thereof immediately by telegram. If no acknowledgement is received within thirty days after the date of the BR IFIC containing the information published under No. 25/1.2, the administration seeking agreement shall dispatch a telegram requesting acknowledgement, to which the receiving administration shall reply within a further period of fifteen days.

25/1.7		Upon receipt of the request under No. 25/1.4, an administration shall, having regard to the proposed date of bringing into use of the assignment(s) corresponding to the allotment for which agreement was requested, promptly examine the matter with regard to harmful interference which would be caused to the services rendered by its coast station(s):

25/1.7.1	using a frequency assignment corresponding to an allotment appearing in the Plan; or

25/1.7.2	to be brought into service in conformity with an allotment appearing in the Plan within the time-limit prescribed in No. 25/1.25; or

25/1.7.3	to be brought into service within the time-limit prescribed in No. 25/1.25, in conformity with a proposed allotment for which the information has been submitted to the Bureau under No. 25/1.1 for publication under No. 25/1.2.

25/1.8		Any administration which receives a request under No. 25/1.4 and which considers that the proposed use of a channel will not cause harmful interference to the services rendered by its coast stations as outlined in No. 25/1.7 shall, as soon as possible and not later than two months from the date of the relevant BR IFIC, notify its agreement to the administration seeking agreement.

25/1.9		Any administration which receives a request under No. 25/1.4 and which considers that the proposed use of a channel may cause harmful interference to the services rendered by its coast stations as outlined in No. 25/1.7 shall inform the administration concerned of the reasons for its disagreement as soon as possible and not later than two months from the date of the relevant BR IFIC and shall furnish any information and suggestions with a view to reaching a satisfactory solution of the problem. The administration seeking agreement shall try, as far as possible, to adjust its requirements according to the comments received.

25/1.10		In a case where the administration seeking agreement has no allotment in the band concerned, the administration(s) with which agreement is sought shall, in consultation with the requesting administration, explore all means of meeting the requirement of the requesting administration.

25/1.11	4	An administration seeking agreement may request the Bureau to endeavour to obtain such agreement in those cases where:

25/1.11.1	an administration to which a request has been sent under No. 25/1.4 fails to acknowledge receipt of the request within forty-five days from the date of the BR IFIC containing the pertinent information;

25/1.11.2	an administration has acknowledged receipt under No. 25/1.6 but fails to give a decision within two months from the date of the BR IFIC containing the pertinent information;

25/1.11.3	there is disagreement between the administration seeking agreement and an administration with which agreement is sought as to the sharing possibilities;

25/1.11.4	it is not possible to reach agreement for any other reason.

25/1.12		Either the administration seeking agreement or an administration with which agreement is sought, or the Bureau, may request additional information which it may require in studying any problem relating to this agreement.

25/1.13		Where the Bureau receives a request under No. 25/1.11.1, it shall forthwith send a telegram to the administration concerned requesting immediate acknowledgement.

25/1.14		Where the Bureau receives an acknowledgement following its action under No. 25/1.13, or where the Bureau receives a request under No. 25/1.11.2, it shall forthwith send a telegram to the administration concerned requesting an early decision in the matter.

25/1.15		Where the Bureau receives a request under No. 25/1.11.4, it shall endeavour to obtain agreement to which reference is made in No. 25/1.4. Where the Bureau receives from an administration no acknowledgement to the request it made under the terms of No. 25/1.4 for agreement within the period specified in No. 25/1.6, it shall act, in so far as this administration is concerned, in accordance with No. 25/1.13.

25/1.16		Where an administration fails to reply within fifteen days of the Bureau’s telegram requesting an acknowledgement sent under No. 25/1.13, or fails to give a decision in the matter within thirty days of dispatch of the Bureau’s telegram of request under No. 25/1.14, it shall be deemed that the administration with which agreement was sought has undertaken, once the projected allotment is included in the Plan:

25/1.16.1	that no complaint will be made in respect of any harmful interference which may be caused to the services rendered by its coast radiotelephone stations by the use of assignments in accordance with the allotment for which agreement was requested; and

25/1.16.2	that its existing or projected coast radiotelephone stations will not cause harmful interference to the use of assignments in conformity with the allotment for which agreement was requested.

25/1.17		The Bureau shall examine the proposed allotment with respect to the probability of harmful interference which it may receive from an allotment in the Plan of the administration which failed to reply or which indicated disagreement without supplying the reasons; if the finding is favourable and where the application of the present procedure with respect to the other administrations concerned permits, the Bureau shall enter the proposed allotment in the Plan.

25/1.18		In the event of an unfavourable finding, the Bureau shall inform the administration concerned of the result of the examination; if the administration insists, and where the application of the present procedure with respect to the other administrations concerned permits, the Bureau shall enter the proposed allotment in the Plan.

25/1.19		Where the Bureau receives a request under No. 25/1.11.3, it shall assess the sharing possibilities and it shall inform the administrations concerned of the results obtained.

25/1.20		In the case of continuing disagreement, the Bureau shall examine the proposed allotment from the point of view of harmful interference which may be caused to the services rendered by the stations of the administration having declared its disagreement. In the case where the Bureau’s finding is favourable and where the application of the present procedure with respect to the other administrations concerned permits, it shall enter the proposed allotment in the Plan.

25/1.21		If, after the examination under No. 25/1.20, the Bureau reaches an unfavourable finding, it shall then examine the proposed allotment from the point of view of harmful interference which may be caused to the services on all the various channels in the band. Should the Bureau reach an unfavourable finding in each case, it shall determine the channel which is the least affected and, if so requested by the administration seeking agreement, it shall enter the proposed allotment in this channel in the Plan.

25/1.22	5	An administration seeking agreement for a proposed allotment shall inform the Bureau of the results of its consultations with the administrations concerned. When the Bureau finds that the procedure prescribed in this Section has been applied with respect to each administration concerned, the Bureau shall publish its finding in a Special Section of the BR IFIC and, as the case may be, bring the Plan up to date.

25/1.23	6	Notwithstanding the above provisions and if the circumstances justify, an administration may, in exceptional circumstances, notify to the Bureau for provisional entry in the Master International Frequency Register an assignment which is not covered by an allotment in the Plan. That administration shall, however, begin forthwith the procedure prescribed in this Section.

25/1.24	7	When, within twelve months from the date of the inclusion of the allotment in the Plan, the Bureau does not receive a notice of a first frequency assignment corresponding to this allotment, or where the first notified frequency assignment has not been brought into use within the time-limits prescribed in these Regulations, before proceeding with the deletion of the allotment from the Plan, it shall consult with the administration concerned on the appropriateness of such a deletion and of publishing this information in connection with bringing the Plan up to date. However, in the case where the Bureau, in the light of a request from the administration concerned, finds that exceptional circumstances warrant an extension of this period, the extension shall in no case exceed six months, except in the case of an administration which has no coast station in service in which case the period may be extended to eighteen months.

25/1.25	8	Any administration in whose name an allotment is shown in the Plan, and which has a need to replace this allotment by another allotment in the same frequency band with a view to improving its service, shall apply the procedure described in this Section. When that administration arrives at a positive result in applying this procedure, the Bureau, at its request, shall replace the existing allotment in the Plan by the proposed allotment.

25/1.26	9	The Bureau shall maintain an up-to-date master copy of the Plan resulting from the application of this procedure. It shall prepare in a suitable form, for publication by the Secretary-General, the whole or part of the revised version of the Plan as and when the circumstances justify and in any case once annually.

25/2	Section II – Allotment Plan for coast radiotelephone stations operating in the 
	exclusive maritime mobile bands between 4 000 kHz and 27 500 kHz1

25/2.1		The frequencies in Column 1 are assigned frequencies (see No. 1.148) as listed in Section I of Part B of Appendix 17. Each frequency is followed, in parentheses, by the carrier frequency and the channel number (see Section I of Part B of Appendix 17).

25/2.2		The coast radiotelephone stations operating in the bands allocated exclusively to the maritime mobile service between 4 000 kHz and 27 500 kHz must use the minimum power required to cover their service area. They may in no case use a peak envelope power above 10 kW per channel (see No. 52.219).

25/2.3		The Plan contained in this Appendix will be updated in accordance with the procedure defined in Section I of this Appendix.

25/2.4     (WRC-03)

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Assigned frequency (carrier frequency) (channel number) | Allotment area2 | Observations3 |

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 358.4 | AFS |  |
| (4 357) | AUS | ADD |
|  | CHL |  |
| (401) | CKH |  |
|  | CUB |  |
|  | D1 |  |
|  | DNK |  |
|  | E |  |
|  | GEO |  |
|  | GRC |  |
|  | HRV |  |
|  | INS |  |
|  | J |  |
|  | KOR |  |
|  | LVA |  |
|  | MNE |  |
|  | NIU |  |
|  | PNR | ADD |
|  | PRG |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | S |  |
|  | SMO |  |
|  | SOM |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | URG |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 4 361.4 | ALB |  |
| (4 360) | ALS |  |
|  | ARG |  |
| (402) | AZE |  |
|  | BEN | ADD |
|  | CHN |  |
|  | G |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | IRN |  |
|  | J |  |
|  | KAZ |  |
|  | MDG |  |
|  | PNG |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS NW |  |
|  | THA |  |
|  | TKM |  |
|  | TUN |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 364.4 | AFS |  |
| (4 363) | ARG |  |
|  | CAN CL | ADD |
| (403) | CAN E |  |
|  | CAN NO |  |
|  | CAN W |  |
|  | DNK |  |
|  | E |  |
|  | GRC |  |
|  | HWA |  |
|  | IND E |  |
|  | IRQ |  |
|  | MAC |  |
|  | MCO |  |
|  | NOR |  |
|  | PNR | ADD |
|  | PTR |  |
|  | RUS EO |  |
|  | S |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 367.4 | ALS |  |
| (4 366) | AUS |  |
|  | B |  |
| (404) | CHL SO |  |
|  | CHN |  |
|  | COG |  |
|  | F |  |
|  | IRN |  |
|  | ISR |  |
|  | J |  |
|  | MCO | ADD |
|  | NCL |  |
|  | OMA |  |
|  | PAQ |  |
|  | POL |  |
|  | SEN |  |
|  | THA |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA W |  |
| 4 370.4 | AFS |  |
| (4 369) | ALS |  |
|  | AUS E |  |
| (405) | AZE |  |
|  | B |  |
|  | CHL |  |
|  | CHN |  |
|  | CME |  |
|  | CNR |  |
|  | D2 |  |
|  | F |  |
|  | GMB |  |
|  | GRC |  |
|  | HOL |  |
|  | >> >> |  |
| (405) | << << |  |
|  | HWA |  |
|  | IRN |  |
|  | LTU |  |
|  | LVA |  |
|  | ROU |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | SNG |  |
|  | URG |  |
|  | USA CL | ADD |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 373.4 | ALB |  |
| (4 372) | ALS |  |
|  | ARG |  |
| (406) | ARS |  |
|  | CHN |  |
|  | CLM |  |
|  | COG |  |
|  | CYP |  |
|  | E |  |
|  | FIN |  |
|  | FJI |  |
|  | G |  |
|  | GUM |  |
|  | HWA |  |
|  | ISL |  |
|  | MDG |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | TUN |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 376.4 | AFS |  |
| (4 375) | ALS |  |
|  | ARG |  |
| (407) | AUS | ADD |
|  | BRB |  |
|  | CAN CL |  |
|  | CME |  |
|  | D2 |  |
|  | E |  |
|  | GUM |  |
|  | HOL |  |
|  | HWA |  |
|  | I | ADD |
|  | IND E |  |
|  | IRN |  |
|  | J |  |
|  | NOR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | >> >> |  |
| (407) | << << |  |
|  | RUS NW |  |
|  | SNG |  |
|  | TUR |  |
|  | USA E |  |
|  | USA W |  |
| 4 379.4 | ALS |  |
| (4 378) | ARG |  |
|  | B | ADD |
| (408) | BEL |  |
|  | BES |  |
|  | CAN E |  |
|  | CAN W |  |
|  | CUW |  |
|  | CHN |  |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | I |  |
|  | INS |  |
|  | IRN |  |
|  | J |  |
|  | MDA | ADD |
|  | MLD | ADD |
|  | MOZ |  |
|  | NZL |  |
|  | POL |  |
|  | SMA | ADD |
|  | SUI |  |
|  | SXM |  |
|  | USA E |  |
|  | USA W |  |
| 4 382.4 | ARS |  |
| (4 381) | B |  |
|  | CHN |  |
| (409) | CUB |  |
|  | DNK |  |
|  | GHA | ADD |
|  | I |  |
|  | IND W |  |
|  | NOR |  |
|  | PNG |  |
|  | QAT | ADD |
|  | S |  |
|  | THA |  |
|  | TUR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 385.4 | ALG |  |
| (4 384) | ARG SO |  |
|  | CAN W |  |
| (410) | CHN |  |
|  | CNR |  |
|  | D2 |  |
|  | G |  |
|  | GRC |  |
|  | GUM |  |
|  | >> >> |  |
| (410) | << << |  |
|  | HNG |  |
|  | HOL |  |
|  | IRN |  |
|  | ISR |  |
|  | MLT |  |
|  | MTN |  |
|  | NZL |  |
|  | ROU |  |
|  | SEY |  |
|  | USA E |  |
| 4 388.4 | AMS | ADD |
| (4 387) | ARG NO |  |
|  | BEL |  |
| (411) | E |  |
|  | EQA | ADD |
|  | FLK |  |
|  | HKG |  |
|  | I | ADD |
|  | INS |  |
|  | IRN |  |
|  | J |  |
|  | KIR | ADD |
|  | RUS NW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA W |  |
| 4 391.4 | AUS |  |
| (4 390) | D1 |  |
|  | EST |  |
| (412) | GEO |  |
|  | I |  |
|  | IND W |  |
|  | IRQ |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 4 394.4 | AGL |  |
| (4 393) | ALG |  |
|  | ALS |  |
| (413) | ARG |  |
|  | AZR |  |
|  | BHR |  |
|  | CAN E |  |
|  | CAN W |  |
|  | CPV |  |
|  | D1 |  |
|  | >> >> |  |
| (413) | << << |  |
|  | FIN |  |
|  | GNB |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MCO |  |
|  | MDR |  |
|  | PNR |  |
|  | POR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | TLS |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 397.4 | ALS |  |
| (4 396) | CYP |  |
|  | D1 |  |
| (414) | E |  |
|  | FIN |  |
|  | INS |  |
|  | ISL |  |
|  | J |  |
|  | KEN |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SHN |  |
|  | USA E |  |
|  | USA SO |  |
| 4 400.4 | ALS |  |
| (4 399) | ARG |  |
|  | AUS |  |
| (415) | CHN |  |
|  | DNK |  |
|  | EST |  |
|  | F |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | LTU |  |
|  | LVA |  |
|  | MDG |  |
|  | MLA |  |
|  | PNR |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 403.4 | ALS |  |
| (4 402) | ARG CL |  |
|  | B |  |
| (416) | EST |  |
|  | F |  |
|  | G |  |
|  | GRC |  |
|  | HNG |  |
|  | INS |  |
|  | IRN |  |
|  | ISL |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | MAU |  |
|  | OCE |  |
|  | RUS SW |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA W |  |
| 4 406.4 | ARG |  |
| (4 405) | AUS |  |
|  | BEL |  |
| (417) | CZE |  |
|  | FIN |  |
|  | G |  |
|  | HKG |  |
|  | HRV |  |
|  | IND W |  |
|  | J |  |
|  | MLA |  |
|  | MNE |  |
|  | MRC |  |
|  | PNG |  |
|  | RUS EO |  |
|  | SVK |  |
|  | TUR |  |
|  | TZA | ADD |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 4 409.4 | ARG |  |
| (4 408) | AZE |  |
|  | B | ADD |
| (418) | BUL |  |
|  | CAN E |  |
|  | CAN W |  |
|  | CUB |  |
|  | DJI |  |
|  | DNK |  |
|  | E |  |
|  | EGY |  |
|  | HWA |  |
|  | I | ADD |
|  | INS |  |
|  | ISR |  |
|  | >> >> |  |
| (418) | << << |  |
|  | J |  |
|  | KAZ |  |
|  | MTN |  |
|  | ROU |  |
|  | RUS AS |  |
|  | S |  |
|  | TKM |  |
|  | USA CL |  |
| 4 412.4 | AUS |  |
| (4 411) | B |  |
|  | CHL |  |
| (419) | CHN |  |
|  | CZE |  |
|  | D2 |  |
|  | F |  |
|  | GUM |  |
|  | HOL |  |
|  | HRV |  |
|  | HWA |  |
|  | ISL |  |
|  | J |  |
|  | KOR |  |
|  | LBY |  |
|  | PTR |  |
|  | RUS NW |  |
|  | SVN |  |
|  | TZA | ADD |
|  | USA SO |  |
|  | USA W |  |
| 4 415.4 | ALS |  |
| (4 414) | AZE |  |
|  | BUL |  |
| (420) | CME |  |
|  | DNK |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | IND E |  |
|  | IRN |  |
|  | J |  |
|  | JOR |  |
|  | KAZ |  |
|  | MLA |  |
|  | MRC |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | S |  |
|  | TKM |  |
|  | TUR |  |
|  | USA E |  |
|  | USA W |  |
| 4 421.4 | ALS |  |
| (4 420) | BEL |  |
|  | CAN W |  |
| (422) | CHN |  |
|  | >> >> |  |
| (422) | << << |  |
|  | CNR |  |
|  | CUB |  |
|  | EST |  |
|  | FIN |  |
|  | G |  |
|  | GRC |  |
|  | HNG |  |
|  | INS |  |
|  | IRQ |  |
|  | J |  |
|  | LBY |  |
|  | LTU |  |
|  | LVA |  |
|  | MRC |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SUI | ADD |
|  | USA E |  |
|  | USA W |  |
| 4 424.4 | ALS |  |
| (4 423) | B | ADD |
|  | CHN |  |
| (423) | D1 |  |
|  | I |  |
|  | INS |  |
|  | ISR |  |
|  | J |  |
|  | MLT | ADD |
|  | PNG |  |
|  | PNR |  |
|  | POL |  |
|  | QAT | ADD |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 427.4 | ALG |  |
| (4 426) | ALS |  |
|  | ARG |  |
| (424) | AUS E | ADD |
|  | AUS W |  |
|  | CHN |  |
|  | DNK |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | MRC |  |
|  | PNR | ADD |
|  | PTR |  |
|  | S |  |
|  | SUI |  |
|  | THA |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 430.4 | ALS |  |
| (4 429) | B | ADD |
|  | BEL |  |
| (425) | CHL |  |
|  | DNK |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | HOL |  |
|  | HWA |  |
|  | J |  |
|  | JOR | ADD |
|  | LBY |  |
|  | MLA |  |
|  | NZL |  |
|  | PNR |  |
|  | PTR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 433.4 | AGL |  |
| (4 432) | ALG |  |
|  | ALS |  |
| (426) | ARG |  |
|  | AZR |  |
|  | BUL |  |
|  | CHN |  |
|  | CPV |  |
|  | CYP |  |
|  | DNK |  |
|  | G |  |
|  | GNB |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MDR |  |
|  | MOZ |  |
|  | NOR |  |
|  | PNR |  |
|  | POR |  |
|  | PTR |  |
|  | THA |  |
|  | TLS |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 4 436.4 | ALS |  |
| (4 435) | AUS |  |
|  | BUL |  |
| (427) | CHL |  |
|  | CHN |  |
|  | DGA |  |
|  | E |  |
|  | G |  |
|  | HWA |  |
|  | I |  |
|  | J |  |
|  | >> >> |  |
| (427) | << << |  |
|  | MDG |  |
|  | MNE |  |
|  | PAQ |  |
|  | POR |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | UAE |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
|  | VTN |  |
| 4 352.4 | ARG CL |  |
| (4 351) | ARG SO |  |
|  | AZE |  |
| (428) | BRU | ADD |
|  | CHN |  |
|  | GRC |  |
|  | I |  |
|  | J |  |
|  | OMA |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TKM |  |
|  | UKR |  |
|  | USA SO |  |
| 4 355.4 | ALS |  |
| (4 354) | ARG SO |  |
|  | AUS |  |
| (429) | AZE |  |
|  | B |  |
|  | BLR |  |
|  | CLM |  |
|  | DGA |  |
|  | E |  |
|  | G |  |
|  | GRC |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | I |  |
|  | J |  |
|  | KAZ |  |
|  | MDG |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TKM |  |
|  | >> >> |  |
| (429) | << << |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 6 502.4 | ALG |  |
| (6 501) | ALS |  |
|  | ARG CL |  |
| (601) | ARG SO |  |
|  | ARS |  |
|  | AZE |  |
|  | CAN NO | ADD |
|  | CAN W |  |
|  | CHL CL |  |
|  | CHL NO |  |
|  | CHN |  |
|  | COG |  |
|  | CZE |  |
|  | D1 |  |
|  | EST |  |
|  | GEO |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | HRV |  |
|  | HWA |  |
|  | I | ADD |
|  | INS |  |
|  | IRN |  |
|  | IRQ |  |
|  | ISL |  |
|  | J |  |
|  | KAZ |  |
|  | LBY |  |
|  | LTU |  |
|  | LVA |  |
|  | MLA | ADD |
|  | MLD | ADD |
|  | MNE |  |
|  | NCL |  |
|  | NZL |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS AS |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SVK |  |
|  | TKM |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 6 505.4 | AFS |  |
| (6 504) | ALS |  |
|  | AUS E | ADD |
| (602) | AZE |  |
|  | B | ADD |
|  | BEL |  |
|  | BUL |  |
|  | >> >> |  |
| (602) | << << |  |
|  | CAN E |  |
|  | CAN W |  |
|  | EQA | ADD |
|  | EST |  |
|  | FJI | ADD |
|  | GEO |  |
|  | GHA | ADD |
|  | GUM |  |
|  | HOL |  |
|  | HRV |  |
|  | HWA |  |
|  | I |  |
|  | INS |  |
|  | IRN |  |
|  | KAZ |  |
|  | KOR |  |
|  | LTU |  |
|  | LVA |  |
|  | MCO |  |
|  | MDG |  |
|  | POL |  |
|  | POR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SNG |  |
|  | TKM |  |
|  | TUN |  |
|  | TUR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 6 508.4 | ALB |  |
| (6 507) | ALG |  |
|  | ALS |  |
| (603) | ARG |  |
|  | ARS |  |
|  | AUS | ADD |
|  | CAN NO |  |
|  | CAN W |  |
|  | CYP |  |
|  | DNK |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | HWA |  |
|  | IND E |  |
|  | INS |  |
|  | IRN |  |
|  | IRQ |  |
|  | ISL |  |
|  | ISR |  |
|  | >> >> |  |
| (603) | << << |  |
|  | J |  |
|  | LBY |  |
|  | MLT | ADD |
|  | MTN |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | S |  |
|  | SMO |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VTN | ADD |
| 6 511.4 | ALS |  |
| (6 510) | AUS |  |
|  | B | ADD |
| (604) | BES | ADD |
|  | BUL |  |
|  | CAN W |  |
|  | CHL |  |
|  | CHN |  |
|  | CME |  |
|  | CUW | ADD |
|  | E |  |
|  | GUM |  |
|  | HKG |  |
|  | HRV |  |
|  | HWA |  |
|  | I |  |
|  | INS |  |
|  | IRN |  |
|  | ISR |  |
|  | MDG |  |
|  | MTN |  |
|  | PNG |  |
|  | POL |  |
|  | PTR |  |
|  | RUS EO | ADD |
|  | RUS NW |  |
|  | SXM | ADD |
|  | TUN |  |
|  | TUR |  |
|  | TUV | ADD |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VTN | ADD |
| 6 514.4 | ALG |  |
| (6 513) | ALS |  |
|  | B | ADD |
| (605) | BUL |  |
|  | CAN E |  |
|  | CAN W |  |
|  | CNR |  |
|  | COG |  |
|  | >> >> |  |
| (605) | << << |  |
|  | DNK |  |
|  | EGY |  |
|  | F | ADD |
|  | GUM |  |
|  | HNG |  |
|  | HOL |  |
|  | HRV |  |
|  | HWA |  |
|  | IND W |  |
|  | INS |  |
|  | IRN |  |
|  | IRQ |  |
|  | J |  |
|  | KOR |  |
|  | LBY |  |
|  | MDG |  |
|  | NZL | ADD |
|  | PTR |  |
|  | RUS EO |  |
|  | S |  |
|  | SVN |  |
|  | UKR |  |
|  | URG |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 6 520.4 | ARG |  |
| (6 519) | AUS |  |
|  | CHN |  |
| (607) | CLM |  |
|  | CUB |  |
|  | DGA |  |
|  | F |  |
|  | GRC |  |
|  | HKG |  |
|  | J |  |
|  | MDA | ADD |
|  | MDG |  |
|  | OMA |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | UAE |  |
|  | USA SO |  |
|  | VTN |  |
| 6 523.4 | ALS |  |
| (6 522) | ARG CL |  |
|  | ARG SO |  |
| (608) | AUS |  |
|  | B |  |
|  | BLR |  |
|  | BRU | ADD |
|  | CHN |  |
|  | DGA |  |
|  | E |  |
|  | EST |  |
|  | G |  |
|  | >> >> |  |
| (608) | << << |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KOR |  |
|  | LVA |  |
|  | MDW |  |
|  | MOZ |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 720.4 | AFS |  |
| (8 719) | ALS |  |
|  | BHR |  |
| (801) | CHL |  |
|  | DNK |  |
|  | E |  |
|  | GUM |  |
|  | HWA |  |
|  | ISR |  |
|  | J |  |
|  | MLA |  |
|  | PNR |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS AN |  |
|  | S |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 723.4 | AGL |  |
| (8 722) | ALG |  |
|  | ALS |  |
| (802) | ARG |  |
|  | AUS |  |
|  | AZR |  |
|  | CHN |  |
|  | CLN |  |
|  | CPV |  |
|  | D2 |  |
|  | FIN |  |
|  | G |  |
|  | GNB |  |
|  | GRC |  |
|  | HOL |  |
|  | HWA |  |
|  | IND E |  |
|  | IRQ |  |
|  | MDA | ADD |
|  | >> >> |  |
| (802) | << << |  |
|  | MDR |  |
|  | MOZ |  |
|  | POR |  |
|  | USA E |  |
|  | USA SO |  |
| 8 726.4 | AFS |  |
| (8 725) | BEL |  |
|  | BES |  |
| (803) | CAN E |  |
|  | CUB |  |
|  | CUW |  |
|  | E |  |
|  | KOR |  |
|  | LTU |  |
|  | LVA |  |
|  | PNG |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | S |  |
|  | SEN |  |
|  | SUI | ADD |
|  | SXM |  |
|  | TUR |  |
|  | USA CL |  |
| 8 729.4 | ARG |  |
| (8 728) | E |  |
|  | FIN |  |
| (804) | GRC |  |
|  | IRQ |  |
|  | J |  |
|  | JOR | ADD |
|  | MCO |  |
|  | POL |  |
|  | QAT | ADD |
|  | RUS AS |  |
|  | RUS EO |  |
|  | SNG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 732.4 | AFS |  |
| (8 731) | ALB |  |
|  | BEL |  |
| (805) | E |  |
|  | EQA | ADD |
|  | FIN |  |
|  | HOL |  |
|  | IRN |  |
|  | ISL |  |
|  | ISR |  |
|  | J |  |
|  | LVA |  |
|  | NCL |  |
|  | PNG |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | >> >> |  |
| (805) | << << |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 735.4 | ALS |  |
| (8 734) | ARG |  |
|  | AUS | ADD |
| (806) | BEL |  |
|  | BHR |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HOL |  |
|  | HWA |  |
|  | I |  |
|  | J |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | SMA | ADD |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 8 738.4 | AZE |  |
| (8 737) | CAN W |  |
|  | CHL |  |
| (807) | COG |  |
|  | CUB |  |
|  | CYP |  |
|  | CZE |  |
|  | I | ADD |
|  | ISL |  |
|  | J |  |
|  | MDG |  |
|  | MTN |  |
|  | NZL |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SHN |  |
|  | TKM |  |
|  | USA CL |  |
| 8 741.4 | AFS |  |
| (8 740) | ALS |  |
|  | ARG |  |
| (808) | ARS |  |
|  | DNK |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | I | ADD |
|  | J |  |
|  | ROU |  |
|  | S |  |
|  | USA E |  |
|  | USA W |  |
| 8 744.4 | ALG |  |
| (8 743) | AUS W |  |
|  | CHL |  |
| (809) | CNR |  |
|  | CUB |  |
|  | CZE |  |
|  | D2 |  |
|  | FIN |  |
|  | GRC |  |
|  | ISL |  |
|  | J |  |
|  | MCO |  |
|  | NOR |  |
|  | SVK |  |
|  | THA |  |
|  | USA E |  |
|  | USA W |  |
| 8 747.4 | ARG |  |
| (8 746) | BUL |  |
|  | CAN E |  |
| (810) | CHN |  |
|  | E |  |
|  | FJI |  |
|  | HRV |  |
|  | INS |  |
|  | IRN |  |
|  | J |  |
|  | MOZ |  |
|  | NOR |  |
|  | POL |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 750.4 | ARG |  |
| (8 749) | ARS |  |
|  | AUS |  |
| (811) | BEN | ADD |
|  | DNK |  |
|  | F |  |
|  | HKG |  |
|  | HNG |  |
|  | HRV |  |
|  | J |  |
|  | MNE |  |
|  | NOR |  |
|  | S |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 753.4 | ALS |  |
| (8 752) | ARG SO |  |
|  | BEL |  |
| (812) | CAN NO |  |
|  | CHN |  |
|  | E |  |
|  | GEO |  |
|  | >> >> |  |
| (812) | << << |  |
|  | HWA |  |
|  | I | ADD |
|  | INS |  |
|  | ISR |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | NZL |  |
|  | POL |  |
|  | RUS NW |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 756.4 | AGL |  |
| (8 755) | ALG |  |
|  | ALS |  |
| (813) | AUS |  |
|  | AZR |  |
|  | BEL |  |
|  | CHL NO |  |
|  | CHN |  |
|  | CPV |  |
|  | DNK |  |
|  | GNB |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | HWA |  |
|  | IND W |  |
|  | MDR |  |
|  | MOZ |  |
|  | NOR |  |
|  | PNR |  |
|  | POR |  |
|  | PTR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VTN | ADD |
| 8 759.4 | ALS |  |
| (8 758) | ARG |  |
|  | AZE |  |
| (814) | CAN W |  |
|  | CUB |  |
|  | EST |  |
|  | GEO |  |
|  | GRC |  |
|  | HWA |  |
|  | I |  |
|  | INS |  |
|  | J |  |
|  | KIR | ADD |
|  | LTU |  |
|  | LVA |  |
|  | RUS AN |  |
|  | >> >> |  |
| (814) | << << |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 762.4 | AUS W |  |
| (8 761) | BEL |  |
|  | CHL |  |
| (815) | CHN |  |
|  | D1 |  |
|  | EST |  |
|  | GRC |  |
|  | IRQ |  |
|  | J |  |
|  | JOR | ADD |
|  | MRC |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | SNG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 765.4 | ALS |  |
| (8 764) | ARG |  |
|  | BRB |  |
| (816) | CHN |  |
|  | COG |  |
|  | E |  |
|  | G |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | INS |  |
|  | LTU |  |
|  | LVA |  |
|  | PTR |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TUN |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 768.4 | ALS |  |
| (8 767) | AUS |  |
|  | CAN E |  |
| (817) | CHL |  |
|  | D1 |  |
|  | EGY |  |
|  | F |  |
|  | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | PNR | ADD |
|  | >> >> |  |
| (817) | << << |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | THA |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 8 771.4 | ALS |  |
| (8 770) | ARG |  |
|  | BUL |  |
| (818) | CHN |  |
|  | CME |  |
|  | CYP |  |
|  | DNK |  |
|  | GUM |  |
|  | HWA |  |
|  | LBY |  |
|  | MLA |  |
|  | PNR |  |
|  | PTR |  |
|  | S |  |
|  | SEY |  |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 8 774.4 | ALS |  |
| (8 773) | AZE |  |
|  | B |  |
| (819) | CAN W |  |
|  | EST |  |
|  | G |  |
|  | GEO |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | INS |  |
|  | J |  |
|  | KAZ |  |
|  | LVA |  |
|  | PAQ |  |
|  | PNR | ADD |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | THA |  |
|  | TKM |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 8 777.4 | ALS |  |
| (8 776) | ARG |  |
|  | CYP |  |
| (820) | D1 |  |
|  | D2 | ADD |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | IND E |  |
|  | IRN |  |
|  | J |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS NW |  |
|  | SMO |  |
|  | TZA | ADD |
|  | USA E |  |
|  | USA W |  |
| 8 783.4 | AUS | ADD |
| (8 782) | B |  |
|  | CHN |  |
| (822) | G |  |
|  | HNG |  |
|  | HRV |  |
|  | IRN |  |
|  | KEN |  |
|  | MRC |  |
|  | SUI |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 786.4 | ARG |  |
| (8 785) | CAN E |  |
|  | DNK |  |
| (823) | GRC |  |
|  | I |  |
|  | IND W |  |
|  | IRQ |  |
|  | J |  |
|  | ROU |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | S |  |
|  | TLS |  |
|  | TZA | ADD |
|  | USA W | ADD |
| 8 789.4 | B |  |
| (8 788) | CHN |  |
|  | D1 |  |
| (824) | GRC |  |
|  | IRN |  |
|  | MRC |  |
|  | OMA |  |
|  | POL |  |
|  | RUS NW |  |
|  | SNG |  |
|  | SUI |  |
|  | >> >> |  |
| (824) | << << |  |
|  | TUN |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 792.4 | ALG |  |
| (8 791) | ALS |  |
|  | AMS | ADD |
| (825) | ARG |  |
|  | BRB |  |
|  | CAN CL |  |
|  | CKH |  |
|  | DNK |  |
|  | F |  |
|  | GHA | ADD |
|  | HNG |  |
|  | IND E |  |
|  | IRN |  |
|  | KAZ |  |
|  | KGZ |  |
|  | RUS EO |  |
|  | S | ADD |
|  | TKM |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 795.4 | CAN W |  |
| (8 794) | CAN NO | ADD |
|  | CHN |  |
|  | CLM |  |
| (826) | CME |  |
|  | D2 |  |
|  | G |  |
|  | GUM |  |
|  | HOL |  |
|  | I |  |
|  | INS |  |
|  | J |  |
|  | QAT | ADD |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
| 8 798.4 | ALS |  |
| (8 797) | ARG |  |
|  | DJI |  |
| (827) | DNK |  |
|  | E |  |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | IRN |  |
|  | ISR |  |
|  | KOR |  |
|  | MAC |  |
|  | MNE |  |
|  | NIU |  |
|  | >> >> |  |
| (827) | << << |  |
|  | PNR |  |
|  | PTR |  |
|  | S |  |
|  | SVN |  |
|  | USA E |  |
|  | USA W |  |
| 8 801.4 | ALB |  |
| (8 800) | ALS |  |
|  | B |  |
| (828) | D1 |  |
|  | F |  |
|  | GUM |  |
|  | HNG |  |
|  | HWA |  |
|  | INS |  |
|  | J |  |
|  | MAU |  |
|  | MRC |  |
|  | MTN |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 8 804.4 | AUS |  |
| (8 803) | BEL |  |
|  | BRM | ADD |
| (829) | CHN |  |
|  | CYP |  |
|  | DNK |  |
|  | FIN |  |
|  | GMB |  |
|  | IRN |  |
|  | LBY |  |
|  | MLD | ADD |
|  | NOR |  |
|  | OCE |  |
|  | PRG |  |
|  | S |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 807.4 | AZE |  |
| (8 806) | B |  |
|  | BUL |  |
| (830) | CHN |  |
|  | F |  |
|  | HRV |  |
|  | IND W |  |
|  | INS |  |
|  | IRN |  |
|  | KAZ |  |
|  | MCO | ADD |
|  | PNG |  |
|  | >> >> |  |
| (830) | << << |  |
|  | POL |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | USA SO |  |
|  | YEM |  |
|  | VTN | ADD |
| 8 810.4 | CHN |  |
| (8 809) | COG |  |
|  | D2 |  |
| (831) | FLK |  |
|  | G |  |
|  | I |  |
|  | IRN |  |
|  | ISL |  |
|  | J |  |
|  | MDG |  |
|  | MLA |  |
|  | MRC |  |
|  | PTR |  |
|  | SUI |  |
|  | TUR |  |
|  | USA SO |  |
|  | USA W |  |
| 8 813.4 | ALS |  |
| (8 812) | B |  |
|  | BUL |  |
| (832) | CHN |  |
|  | CLM |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | KOR |  |
|  | MDG |  |
|  | MLT |  |
|  | PTR |  |
|  | QAT |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | TUR |  |
|  | UAE |  |
|  | URG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VTN |  |
| 8 708.4 | AUS |  |
| (8 707) | BHR | ADD |
|  | CHL |  |
| (834) | CHN |  |
|  | CLM |  |
|  | DGA |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KOR |  |
|  | MDW |  |
|  | >> >> |  |
| (834) | << << |  |
|  | POR |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
| 8 711.4 | ALS |  |
| (8 710) | ARG CL |  |
|  | ARG SO |  |
| (835) | AZE |  |
|  | BRU | ADD |
|  | DGA |  |
|  | E |  |
|  | F |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KOR |  |
|  | MDW |  |
|  | MNE |  |
|  | OMA |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | SUI |  |
|  | THA |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 8 714.4 | AUS |  |
| (8 713) | AZE |  |
|  | CHL |  |
| (836) | CHN |  |
|  | E |  |
|  | I |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TKM |  |
|  | UKR |  |
|  | URG |  |
|  | USA SO |  |
| 8 717.4 | ARG CL |  |
| (8 716) | ARG SO |  |
|  | AZE |  |
| (837) | BLR |  |
|  | CHN |  |
|  | CUB |  |
|  | >> >> |  |
| (837) | << << |  |
|  | G |  |
|  | GRC |  |
|  | J |  |
|  | KAZ |  |
|  | MDG |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TKM |  |
|  | UKR |  |
|  | USA SO |  |
| 13 078.4 | ARG |  |
| (13 077) | CAN NO |  |
|  | CHN |  |
| (1201) | CYP |  |
|  | E |  |
|  | G |  |
|  | INS |  |
|  | QAT | ADD |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 081.4 | ARS |  |
| (13 080) | CHL |  |
|  | D2 |  |
| (1202) | FJI |  |
|  | G |  |
|  | GRC |  |
|  | HNG |  |
|  | J |  |
|  | MRC |  |
|  | RUS AN |  |
|  | SUI |  |
|  | TUN |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 084.4 | AGL |  |
| (13 083) | ALS |  |
|  | AUS E |  |
| (1203) | AZR |  |
|  | CHN |  |
|  | CLM |  |
|  | CPV |  |
|  | DNK |  |
|  | GNB |  |
|  | GRC |  |
|  | HWA |  |
|  | IRQ |  |
|  | LBY |  |
|  | >> >> |  |
| (1203) | << << |  |
|  | MDR |  |
|  | MOZ |  |
|  | POR |  |
|  | RUS EO |  |
|  | S |  |
|  | TLS |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 087.4 | ALS |  |
| (13 086) | D2 |  |
|  | F |  |
| (1204) | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | ISR |  |
|  | J |  |
|  | LVA |  |
|  | MAC |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 090.4 | ALS |  |
| (13 089) | ARG |  |
|  | D1 |  |
| (1205) | E |  |
|  | GEO |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | MOZ |  |
|  | NCL |  |
|  | NOR |  |
|  | PTR |  |
|  | TLS |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | YEM |  |
| 13 093.4 | ALB |  |
| (13 092) | AUS W |  |
|  | CHN |  |
| (1206) | D2 |  |
|  | E |  |
|  | FIN |  |
|  | G |  |
|  | >> >> |  |
| (1206) | << << |  |
|  | I |  |
|  | IRN |  |
|  | ISL |  |
|  | J |  |
|  | MDG |  |
|  | MRC |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 096.4 | AGL |  |
| (13 095) | ALG |  |
|  | AZR |  |
| (1207) | BEL |  |
|  | BES |  |
|  | CAN W |  |
|  | CHN |  |
|  | CPV |  |
|  | CUW |  |
|  | EQA | ADD |
|  | GRC |  |
|  | HOL |  |
|  | IRN |  |
|  | ISR |  |
|  | J |  |
|  | MDR |  |
|  | MNE |  |
|  | MOZ |  |
|  | POR |  |
|  | RUS NW |  |
|  | SXM |  |
|  | TLS |  |
| 13 099.4 | ARG |  |
| (13 098) | CHN |  |
|  | CYP |  |
| (1208) | D1 |  |
|  | EST |  |
|  | GRC |  |
|  | HNG |  |
|  | I | ADD |
|  | ISL |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
| 13 102.4 | AFS |  |
| (13 101) | ALS |  |
|  | B |  |
| (1209) | BHR |  |
|  | CAN W |  |
|  | E |  |
|  | EST |  |
|  | >> >> |  |
| (1209) | << << |  |
|  | FIN |  |
|  | I |  |
|  | INS |  |
|  | J |  |
|  | NZL |  |
|  | POL |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 105.4 | CHL |  |
| (13 104) | DJI |  |
|  | DNK |  |
| (1210) | E |  |
|  | GRC |  |
|  | GUM |  |
|  | IND W |  |
|  | INS |  |
|  | ROU |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | S |  |
|  | SUI | ADD |
|  | URG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 108.4 | ALS |  |
| (13 107) | B |  |
|  | CHN |  |
| (1211) | CUB |  |
|  | DNK |  |
|  | E |  |
|  | I |  |
|  | IRQ |  |
|  | J |  |
|  | KAZ |  |
|  | MLA |  |
|  | NOR |  |
|  | PAQ |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | S |  |
|  | TKM |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 111.4 | ALS |  |
| (13 110) | D1 |  |
|  | GRC |  |
| (1212) | HWA |  |
|  | INS |  |
|  | J |  |
|  | MAU |  |
|  | >> >> |  |
| (1212) | << << |  |
|  | MDA | ADD |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
| 13 114.4 | ARG |  |
| (13 113) | BEL |  |
|  | BRB |  |
| (1213) | CAN E |  |
|  | CHN |  |
|  | CNR |  |
|  | FIN |  |
|  | GRC |  |
|  | HOL |  |
|  | I |  |
|  | IND E |  |
|  | IRN |  |
|  | IRQ |  |
|  | ISR |  |
|  | KOR |  |
|  | NOR |  |
|  | RUS AN |  |
|  | SMO |  |
|  | USA W | ADD |
| 13 117.4 | ALS |  |
| (13 116) | AUS |  |
|  | B |  |
| (1214) | CAN W |  |
|  | CUB |  |
|  | DNK |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | IRN |  |
|  | PTR |  |
|  | RUS EO |  |
|  | S |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 120.4 | ALG |  |
| (13 119) | BEL |  |
|  | BHR | ADD |
| (1215) | CME |  |
|  | DNK |  |
|  | E |  |
|  | GRC |  |
|  | HOL |  |
|  | IND W |  |
|  | ISL |  |
|  | ISR |  |
|  | J |  |
|  | PNR |  |
|  | PTR |  |
|  | ROU |  |
|  | >> >> |  |
| (1215) | << << |  |
|  | S |  |
|  | SEY |  |
|  | USA SO |  |
|  | USA W |  |
| 13 123.4 | ALB |  |
| (13 122) | ALS |  |
|  | ARG |  |
| (1216) | CHN |  |
|  | EGY |  |
|  | FIN |  |
|  | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | MRC |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | SNG |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 126.4 | ALG |  |
| (13 125) | AZE |  |
|  | BUL |  |
| (1217) | CUB |  |
|  | DNK |  |
|  | GRC |  |
|  | GUM |  |
|  | IND E |  |
|  | IRQ |  |
|  | J |  |
|  | KAZ |  |
|  | NOR |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | S |  |
|  | SHN |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 129.4 | ALS |  |
| (13 128) | BEL |  |
|  | CHL |  |
| (1218) | CME |  |
|  | CNR |  |
|  | D1 |  |
|  | GUM |  |
|  | HWA |  |
|  | I |  |
|  | IRN |  |
|  | J |  |
|  | NIU |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS SW |  |
|  | >> >> |  |
| (1218) | << << |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 132.4 | ALS |  |
| (13 131) | B |  |
|  | BEL |  |
| (1219) | BUL |  |
|  | DNK |  |
|  | HOL |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | MRC |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | S |  |
|  | SNG |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 135.4 | ALS |  |
| (13 134) | ARG |  |
|  | D2 | ADD |
| (1220) | FJI |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | ISL |  |
|  | J |  |
|  | JOR | ADD |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | TUN |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 141.4 | ALS | ADD |
| (13 140) | ARG |  |
|  | BEN | ADD |
| (1222) | CAN E |  |
|  | CKH |  |
|  | F |  |
|  | HWA |  |
|  | IND W |  |
|  | IRN |  |
|  | J |  |
|  | NOR |  |
|  | ROU |  |
|  | RUS EO |  |
|  | TUR |  |
|  | USA W | ADD |
| 13 144.4 | ARS |  |
| (13 143) | B |  |
|  | CZE |  |
| (1223) | DNK |  |
|  | GRC |  |
|  | GUM |  |
|  | J |  |
|  | MRC |  |
|  | S |  |
|  | SVK |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 147.4 | AFS |  |
| (13 146) | ALS |  |
|  | CHL |  |
| (1224) | D1 |  |
|  | FIN |  |
|  | G |  |
|  | GHA | ADD |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | J |  |
|  | MCO |  |
|  | NZL |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 13 150.4 | CHN |  |
| (13 149) | E |  |
|  | GRC |  |
| (1225) | IRN |  |
|  | JOR | ADD |
|  | MDG |  |
|  | NOR |  |
|  | PNG |  |
|  | ROU |  |
|  | RUS NW |  |
|  | USA E |  |
|  | USA SO |  |
| 13 153.4 | AUS |  |
| (13 152) | CHL |  |
|  | CZE |  |
| (1226) | DNK |  |
|  | F |  |
|  | IRN |  |
|  | J |  |
|  | MCO | ADD |
|  | RUS NW |  |
|  | S | ADD |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 156.4 | ALS |  |
| (13 155) | AUS |  |
|  | E |  |
| (1227) | FIN |  |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | IND E |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | RUS EO |  |
|  | SUI |  |
|  | TZA | ADD |
|  | USA E |  |
|  | USA W |  |
| 13 159.4 | B |  |
| (13 158) | CHL |  |
|  | CHN |  |
| (1228) | CUB |  |
|  | EST |  |
|  | G |  |
|  | GEO |  |
|  | HNG |  |
|  | I | ADD |
|  | LVA |  |
|  | MLD | ADD |
|  | NOR |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA W |  |
|  | VTN | ADD |
| 13 162.4 | ARG |  |
| (13 161) | AUS |  |
|  | AZE |  |
| (1229) | BUL |  |
|  | CAN E |  |
|  | F |  |
|  | HRV |  |
|  | J |  |
|  | KAZ |  |
|  | KGZ |  |
|  | KOR |  |
|  | LTU |  |
|  | LVA |  |
|  | POL |  |
|  | QAT | ADD |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA W |  |
| 13 165.4 | ARG |  |
| (13 164) | CYP |  |
|  | FIN |  |
| (1230) | G |  |
|  | HWA |  |
|  | I |  |
|  | J |  |
|  | MTN |  |
|  | SUI |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 168.4 | ALS |  |
| (13 167) | AUS |  |
|  | F |  |
| (1231) | GRC |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | IRN |  |
|  | LBY |  |
|  | NOR |  |
|  | PNR |  |
|  | POL |  |
|  | PRG |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
|  | VTN | ADD |
| 13 171.4 | ALG |  |
| (13 170) | ALS |  |
|  | ARG |  |
| (1232) | AZE |  |
|  | D2 |  |
|  | G |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KAZ |  |
|  | MTN |  |
|  | PNR | ADD |
|  | SMA | ADD |
|  | TKM |  |
|  | USA E |  |
|  | USA W |  |
| 13 174.4 | AZE |  |
| (13 173) | B |  |
|  | CHN |  |
| (1233) | CLM |  |
|  | E |  |
|  | G |  |
|  | GEO |  |
|  | GRC |  |
|  | J |  |
|  | LVA |  |
|  | MLT |  |
|  | RUS AN |  |
|  | >> >> |  |
| (1233) | << << |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | USA SO |  |
|  | VTN |  |
| 13 177.4 | ALS |  |
| (13 176) | AUS |  |
|  | CHN |  |
| (1234) | CLM |  |
|  | E |  |
|  | HWA |  |
|  | KOR |  |
|  | MDG |  |
|  | OMA |  |
|  | RUS EO |  |
|  | USA SO |  |
|  | USA W |  |
| 13 180.4 | ARG |  |
| (13 179) | CHN |  |
|  | F |  |
| (1235) | G |  |
|  | HOL |  |
|  | J |  |
|  | KOR |  |
|  | LVA |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | THA |  |
|  | TUR |  |
|  | UKR |  |
|  | USA SO |  |
|  | UZB |  |
| 13 183.4 | BRM | ADD |
| (13 182) | CHN |  |
|  | I |  |
| (1236) | RUS EO |  |
|  | UAE |  |
|  | UKR |  |
|  | USA SO |  |
| 13 186.4 | CHN |  |
| (13 185) | F |  |
|  | ISR |  |
| (1237) | J |  |
|  | LVA |  |
|  | PTR |  |
|  | RUS AS |  |
|  | RUS SW |  |
|  | SUI |  |
|  | TUR |  |
|  | UAE |  |
|  | UKR |  |
|  | >> >> |  |
| (1237) | << << |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | VIR |  |
| 13 189.4 | ALS |  |
| (13 188) | B |  |
|  | BLR |  |
| (1238) | CHL |  |
|  | CHN |  |
|  | EST |  |
|  | GUM |  |
|  | HWA |  |
|  | KOR |  |
|  | MCO | ADD |
|  | PAQ |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 192.4 | ALS |  |
| (13 191) | AZE |  |
|  | B |  |
| (1239) | BRU | ADD |
|  | BUL |  |
|  | CAN E |  |
|  | CHN |  |
|  | E |  |
|  | F |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KAZ |  |
|  | MDG |  |
|  | MNE |  |
|  | PTR |  |
|  | QAT |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 195.4 | ARG CL |  |
| (13 194) | ARG SO |  |
|  | AUS |  |
| (1240) | >> >> |  |
| (1240) | << << |  |
|  | CHN |  |
|  | DGA |  |
|  | GRC |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | KGZ |  |
|  | MDW |  |
|  | POR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 13 198.4 | ALS |  |
| (13 197) | CHN |  |
|  | D2 |  |
| (1241) | DGA |  |
|  | GUM |  |
|  | HWA |  |
|  | IND E |  |
|  | IND W |  |
|  | J |  |
|  | MDW |  |
|  | PTR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 17 243.4 | ALS |  |
| (17 242) | ARG |  |
|  | DNK |  |
| (1601) | HWA |  |
|  | J |  |
|  | LTU |  |
|  | NOR |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | S |  |
|  | SEY |  |
|  | TUN |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
| 17 246.4 | ARS |  |
| (17 245) | AUS E |  |
|  | CME |  |
| (1602) | G |  |
|  | GRC |  |
|  | MRC |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | >> >> |  |
| (1602) | << << |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 249.4 | ALS |  |
| (17 248) | ARG NO |  |
|  | CHN |  |
| (1603) | CYP |  |
|  | DNK |  |
|  | HNG |  |
|  | I |  |
|  | MLT | ADD |
|  | NOR |  |
|  | S |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 252.4 | AUS |  |
| (17 251) | BEN | ADD |
|  | CAN E |  |
| (1604) | F |  |
|  | GRC |  |
|  | J |  |
|  | NOR |  |
|  | ROU |  |
| 17 255.4 | DNK |  |
| (17 254) | F |  |
|  | IND W |  |
| (1605) | IRN |  |
|  | J |  |
|  | OCE |  |
|  | RUS SW |  |
|  | S |  |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 17 258.4 | B |  |
| (17 257) | CUB |  |
|  | FIN |  |
| (1606) | G |  |
|  | I |  |
|  | ISL |  |
|  | J |  |
|  | NZL |  |
|  | PTR |  |
|  | RUS SW |  |
|  | TUR |  |
|  | USA SO |  |
|  | USA W |  |
| 17 261.4 | ALS |  |
| (17 260) | BES |  |
|  | CAN E |  |
| (1607) | CUW |  |
|  | GRC |  |
|  | IND E |  |
|  | IRN |  |
|  | MCO |  |
|  | NOR |  |
|  | POL |  |
|  | >> >> |  |
| (1607) | << << |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | SXM |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 264.4 | AFS |  |
| (17 263) | CAN W |  |
|  | CHN |  |
| (1608) | CZE |  |
|  | DNK |  |
|  | EQA | ADD |
|  | I |  |
|  | MTN |  |
|  | S |  |
|  | SVK |  |
|  | TUR |  |
| 17 267.4 | ARS |  |
| (17 266) | BEL |  |
|  | CKH |  |
| (1609) | E |  |
|  | GRC |  |
|  | IND E |  |
|  | ISR |  |
|  | J |  |
|  | RUS NW |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 270.4 | AUS |  |
| (17 269) | CHN |  |
|  | D1 |  |
| (1610) | EGY |  |
|  | INS |  |
|  | IRN |  |
|  | MTN |  |
|  | NOR |  |
|  | RUS NW |  |
|  | TUN |  |
|  | UKR |  |
|  | URG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 273.4 | B |  |
| (17 272) | FIN |  |
|  | G |  |
| (1611) | HRV |  |
|  | J |  |
|  | LBY |  |
|  | MLA |  |
|  | SUI |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 276.4 | ALS |  |
| (17 275) | AUS |  |
|  | CUB |  |
| (1612) | GEO |  |
|  | GUM |  |
|  | HWA |  |
|  | JOR | ADD |
|  | MRC |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 279.4 | ALS |  |
| (17 278) | B |  |
|  | BEL |  |
| (1613) | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | ISR |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | ROU |  |
|  | RUS EO |  |
|  | SNG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 282.4 | CAN W |  |
| (17 281) | CHN |  |
|  | DNK |  |
| (1614) | FIN |  |
|  | I |  |
|  | MLD | ADD |
|  | NIU |  |
|  | RUS AN |  |
|  | S |  |
| 17 285.4 | AGL |  |
| (17 284) | AZR |  |
|  | CPV |  |
| (1615) | FIN |  |
|  | G |  |
|  | GNB |  |
|  | IRN |  |
|  | ISL |  |
|  | MDR |  |
|  | MOZ |  |
|  | POR |  |
|  | RUS EO |  |
|  | SUI |  |
|  | TLS |  |
| 17 288.4 | ALS |  |
| (17 287) | D1 |  |
|  | HWA |  |
| (1616) | I |  |
|  | IRN |  |
|  | J |  |
|  | MRC |  |
|  | RUS NW |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 291.4 | B |  |
| (17 290) | CNR |  |
|  | DNK |  |
| (1617) | F |  |
|  | GRC |  |
|  | HNG |  |
|  | IRN |  |
|  | ISR |  |
|  | RUS EO |  |
|  | S |  |
| 17 294.4 | ARG |  |
| (17 293) | BHR |  |
|  | DNK |  |
| (1618) | G |  |
|  | HRV |  |
|  | IND W |  |
|  | J |  |
|  | MRC |  |
|  | S |  |
|  | TUR |  |
| 17 297.4 | ALS |  |
| (17 296) | D2 |  |
|  | F |  |
| (1619) | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | MAU |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | USA E |  |
|  | USA W |  |
| 17 300.4 | J |  |
| (17 299) | LBY |  |
|  | LTU |  |
| (1620) | LVA |  |
|  | NOR |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
| 17 306.4 | ALS | ADD |
| (17 305) | AUS |  |
|  | DNK |  |
| (1622) | F |  |
|  | GHA | ADD |
|  | GRC |  |
|  | HWA | ADD |
|  | J |  |
|  | PNR | ADD |
|  | ROU |  |
|  | S |  |
|  | SUI | ADD |
| 17 309.4 | ALS |  |
| (17 308) | CHN |  |
|  | E |  |
| (1623) | FIN |  |
|  | G |  |
|  | GUM |  |
|  | HOL |  |
|  | HWA |  |
|  | PNR |  |
|  | PRG |  |
|  | PTR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 312.4 | D1 |  |
| (17 311) | E |  |
|  | I |  |
| (1624) | J |  |
|  | LTU |  |
|  | LVA |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SMO |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 315.4 | ALS |  |
| (17 314) | BEL |  |
|  | GRC |  |
| (1625) | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | ISL |  |
|  | J |  |
|  | POL |  |
|  | PTR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 318.4 | CAN W |  |
| (17 317) | CUB |  |
|  | GRC |  |
| (1626) | HOL |  |
|  | IRQ |  |
|  | J |  |
|  | >> >> |  |
| (1626) | << << |  |
|  | QAT | ADD |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | USA E |  |
| 17 321.4 | ALS | ADD |
| (17 320) | BEL |  |
|  | E |  |
| (1627) | EST |  |
|  | GRC |  |
|  | HNG |  |
|  | HRV |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | NOR |  |
|  | RUS SW |  |
|  | RUS W |  |
| 17 324.4 | CUB |  |
| (17 323) | EQA | ADD |
|  | F |  |
| (1628) | GRC |  |
|  | IRQ |  |
|  | ISR |  |
|  | MCO | ADD |
|  | ROU |  |
|  | RUS EO |  |
|  | RUS NW |  |
| 17 327.4 | ALG |  |
| (17 326) | AUS |  |
|  | BRM | ADD |
| (1629) | CAN E |  |
|  | D2 |  |
|  | GRC |  |
|  | IRN |  |
|  | J |  |
|  | NOR |  |
|  | SEN |  |
| 17 330.4 | ALS |  |
| (17 329) | BEL |  |
|  | E |  |
| (1630) | GEO |  |
|  | GUM |  |
|  | HWA |  |
|  | IND W |  |
|  | ISL |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS SW |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 333.4 | ALG |  |
| (17 332) | BUL |  |
|  | CHL |  |
| (1631) | >> >> |  |
| (1631) | << << |  |
|  | CHN |  |
|  | GRC |  |
|  | IRQ |  |
|  | POL |  |
|  | SUI |  |
|  | USA E |  |
| 17 336.4 | ALS |  |
| (17 335) | ARG |  |
|  | AZR |  |
| (1632) | CYP |  |
|  | G |  |
|  | HNG |  |
|  | J |  |
|  | MDG |  |
|  | MDR |  |
|  | POR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 339.4 | AFS |  |
| (17 338) | ALS |  |
|  | AZE |  |
| (1633) | B |  |
|  | CHN |  |
|  | D2 |  |
|  | F |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | KAZ |  |
|  | KGZ |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | RUS AS |  |
|  | TKM |  |
|  | USA E |  |
|  | USA W |  |
| 17 342.4 | CAN NO |  |
| 17 341 | CHN |  |
|  | D1 |  |
| (1634) | E |  |
|  | GRC |  |
|  | J |  |
|  | KOR |  |
|  | ROU |  |
| 17 345.4 | AGL |  |
| (17 344) | AUS |  |
|  | AZR |  |
| (1635) | BUL |  |
|  | CPV |  |
|  | DNK |  |
|  | GNB |  |
|  | I | ADD |
|  | J |  |
|  | MAC |  |
|  | MDR |  |
|  | MOZ |  |
|  | >> >> |  |
| (1635) | << << |  |
|  | PNR | ADD |
|  | POR |  |
|  | S |  |
|  | TLS |  |
| 17 348.4 | ALG |  |
| (17 347) | ALS |  |
|  | FIN |  |
| (1636) | GRC |  |
|  | GUM |  |
|  | HOL |  |
|  | HWA |  |
|  | IND E |  |
|  | J |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 17 351.4 | AZE |  |
| (17 350) | CHN |  |
|  | E |  |
| (1637) | G |  |
|  | HKG |  |
|  | KAZ |  |
|  | KOR |  |
|  | MDG |  |
|  | NZL |  |
|  | RUS AS |  |
| 17 354.4 | ALS |  |
| (17 353) | BUL |  |
|  | D2 | ADD |
| (1638) | FIN |  |
|  | GUM |  |
|  | HWA |  |
|  | MNE |  |
|  | MRC |  |
|  | POL |  |
|  | SMA | ADD |
|  | USA E |  |
|  | USA W |  |
| 17 357.4 | ALB |  |
| (17 356) | ALS |  |
|  | CHN |  |
| (1639) | D1 |  |
|  | E |  |
|  | GUM |  |
|  | HOL |  |
|  | HWA |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 17 360.4 | BRB |  |
| (17 359) | CHL |  |
|  | D2 |  |
| (1640) | EST |  |
|  | G |  |
|  | GRC |  |
|  | >> >> |  |
| (1640) | << << |  |
|  | J |  |
|  | LVA |  |
|  | PNR | ADD |
| 17 363.4 | ALG |  |
| (17 362) | DNK |  |
|  | IRQ |  |
| (1641) | J |  |
|  | S |  |
|  | SNG |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 366.4 | ALS |  |
| (17 365) | AUS |  |
|  | CLM |  |
| (1642) | F |  |
|  | HWA |  |
|  | J |  |
|  | PTR |  |
|  | RUS EO |  |
|  | UAE |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 17 369.4 | AZE |  |
| (17 368) | CHN |  |
|  | CLM |  |
| (1643) | F |  |
|  | KAZ |  |
|  | QAT |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TKM |  |
|  | UKR |  |
|  | USA SO |  |
| 17 372.4 | ALS |  |
| (17 371) | B |  |
|  | HWA |  |
| (1644) | I |  |
|  | MDA | ADD |
|  | RUS EO |  |
|  | RUS NW |  |
|  | UAE |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| (17 375.4) | ARG |  |
| (17 374) | CHN |  |
|  | ISR |  |
| (1645) | KGZ |  |
|  | KOR |  |
|  | LVA |  |
|  | OMA |  |
|  | >> >> |  |
| (1645) | << << |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TUR |  |
|  | UKR |  |
|  | USA SO |  |
|  | UZB |  |
| 17 378.4 | CHN |  |
| (17 377) | I |  |
|  | RUS EO |  |
| (1646) | RUS SW |  |
|  | RUS W |  |
|  | USA W |  |
|  | VTN | ADD |
| 17 381.4 | ALS |  |
| (17 380) | CAN E |  |
|  | CHN |  |
| (1647) | EST |  |
|  | HWA |  |
|  | KOR |  |
|  | LTU |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 384.4 | ALS |  |
| (17 383) | BLR |  |
|  | CHN |  |
| (1648) | HWA |  |
|  | KOR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA W |  |
|  | VIR |  |
| 17 387.4 | ALS |  |
| (17 386) | B |  |
|  | BUL |  |
| (1649) | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MDG |  |
|  | PTR |  |
|  | RUS AN |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 390.4 | ALS |  |
| (17 389) | ARG CL |  |
|  | ARG SO |  |
| (1650) | AZE |  |
|  | CHN |  |
|  | E |  |
|  | GRC |  |
|  | HKG |  |
|  | HWA |  |
|  | J |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 393.4 | ALS |  |
| (17 392) | BLR |  |
|  | CHN |  |
| (1651) | DGA |  |
|  | E |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 396.4 | CHN |  |
| (17 395) | GUM |  |
|  | HOL |  |
| (1652) | J |  |
|  | MDG |  |
|  | MDW |  |
|  | MNE |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
| 17 399.4 | B |  |
| (17 398) | CHN |  |
|  | E |  |
| (1653) | PTR |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | >> >> |  |
| (1653) | << << |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VTN |  |
| 17 402.4 | CHN |  |
| (17 401) | G |  |
|  | HWA |  |
| (1654) | J |  |
|  | PTR |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 405.4 | ALS |  |
| (17 404) | CHL |  |
|  | CHN |  |
| (1655) | DGA |  |
|  | E |  |
|  | G |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | KGZ |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 17 408.4 | AUS |  |
| (17 407) | CHN |  |
|  | GUM |  |
| (1656) | HWA |  |
|  | LVA |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SUI |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 756.4 | ALS |  |
| (19 755) | AUS |  |
|  | CHN |  |
| (1801) | E |  |
|  | G |  |
|  | HWA |  |
|  | >> >> |  |
| (1801) | << << |  |
|  | J |  |
|  | JOR |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | UAE |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 19 759.4 | CHN |  |
| (19 758) | G |  |
|  | HOL |  |
| (1802) | ISL |  |
|  | J |  |
|  | MOZ |  |
|  | PTR |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | VIR |  |
| 19 762.4 | ALS |  |
| (19 761) | AZE |  |
|  | B |  |
| (1803) | CHN |  |
|  | G |  |
|  | HWA |  |
|  | J |  |
|  | JOR |  |
|  | KOR |  |
|  | LTU |  |
|  | POR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | UAE |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA W |  |
|  | VIR |  |
| 19 765.4 | ALS |  |
| (19 764) | BRU | ADD |
|  | CAN W |  |
| (1804) | CHN |  |
|  | D2 |  |
|  | HWA |  |
|  | J |  |
|  | RUS EO |  |
|  | >> >> |  |
| (1804) | << << |  |
|  | S | ADD |
|  | TUR |  |
|  | USA SO |  |
|  | USA W |  |
| 19 768.4 | ALS |  |
| (19 767) | CHN |  |
|  | HWA |  |
| (1805) | I |  |
|  | J |  |
|  | LVA |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TUR |  |
|  | USA W |  |
| 19 774.4 | ARG CL |  |
| (19 773) | ARG SO |  |
|  | CHN |  |
| (1807) | D2 |  |
|  | GEO |  |
|  | ISL |  |
|  | J |  |
|  | LVA |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | TUR |  |
|  | USA SO |  |
| 19 777.4 | ALS |  |
| (19 776) | BLR |  |
|  | CHN |  |
| (1808) | CUB |  |
|  | HWA |  |
|  | ISR |  |
|  | MCO | ADD |
|  | MDG |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 19 780.4 | ALS |  |
| (19 779) | B |  |
|  | CHN |  |
| (1809) | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | POL | ADD |
|  | >> >> |  |
| (1809) | << << |  |
|  | RUS NW |  |
|  | RUS W |  |
|  | SUI |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 783.4 | ALS |  |
| (19 782) | ARG |  |
|  | BUL |  |
| (1810) | CHN |  |
|  | EST |  |
|  | HKG |  |
|  | HWA |  |
|  | J |  |
|  | LTU |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA W |  |
| 19 786.4 | ALS |  |
| (19 785) | B |  |
|  | CAN E |  |
| (1811) | CHN |  |
|  | DGA |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KOR |  |
|  | MDG |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 789.4 | ALS |  |
| (19 788) | ARG |  |
|  | AZE |  |
| (1812) | CAN E |  |
|  | CHN |  |
|  | HWA |  |
|  | J |  |
|  | MNE |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 792.4 | ALS |  |
| (19 791) | CHN |  |
|  | E |  |
| (1813) | F |  |
|  | HWA |  |
|  | IND E |  |
|  | IND W |  |
|  | J |  |
|  | MDA | ADD |
|  | PTR |  |
|  | S | ADD |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 795.4 | ALS |  |
| (19 794) | AUS |  |
|  | AZE |  |
| (1814) | B |  |
|  | CHN |  |
|  | DGA |  |
|  | E |  |
|  | GUM |  |
|  | HWA |  |
|  | ISL |  |
|  | MDW |  |
|  | MNE |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 19 798.4 | ARG CL |  |
| (19 797) | ARG SO |  |
|  | AZE |  |
| (1815) | BLR |  |
|  | CHN |  |
|  | GUM |  |
|  | J |  |
|  | KAZ |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 697.4 | AUS |  |
| (22 696) | CHN |  |
|  | CME |  |
| (2201) | E |  |
|  | GRC |  |
|  | GUM |  |
|  | >> >> |  |
| (2201) | << << |  |
|  | HNG |  |
|  | RUS NW |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 700.4 | ARG |  |
| (22 699) | BRM | ADD |
|  | CAN E |  |
| (2202) | HNG |  |
|  | I |  |
|  | IRN |  |
|  | MTN |  |
|  | NOR |  |
|  | RUS EO |  |
|  | UKR |  |
| 22 703.4 | AUS E |  |
| (22 702) | BUL |  |
|  | DNK |  |
| (2203) | IRN |  |
|  | J |  |
|  | MRC |  |
|  | PNR | ADD |
|  | S |  |
| 22 706.4 | AFS |  |
| (22 705) | ARG |  |
|  | CAN NO |  |
| (2204) | F |  |
|  | FIN |  |
|  | HRV |  |
|  | ISR |  |
|  | RUS EO |  |
|  | RUS NW |  |
| 22 709.4 | ALG |  |
| (22 708) | AUS |  |
|  | EST |  |
| (2205) | GRC |  |
|  | HOL |  |
|  | IRN |  |
|  | LTU |  |
|  | LVA |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS W |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 712.4 | AFS |  |
| (22 711) | ALS |  |
|  | BHR | ADD |
| (2206) | G |  |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | IND W |  |
|  | J |  |
|  | MRC |  |
|  | POL |  |
|  | PTR |  |
|  | >> >> |  |
| (2206) | << << |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 715.4 | AZR |  |
| (22 714) | CHN |  |
|  | CPV |  |
| (2207) | D1 |  |
|  | ISR |  |
|  | LVA |  |
|  | MDR |  |
|  | POR |  |
|  | RUS SW |  |
|  | TLS |  |
|  | TUN |  |
| 22 718.4 | ARG NO |  |
| (22 717) | BUL |  |
|  | DNK |  |
| (2208) | I | ADD |
|  | IND E |  |
|  | J |  |
|  | MRC |  |
|  | NOR |  |
|  | PNR | ADD |
|  | S |  |
| 22 721.4 | ALS |  |
| (22 720) | BEL |  |
|  | CHN |  |
| (2209) | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | KOR |  |
|  | MRC |  |
|  | PNR |  |
|  | POL |  |
|  | PTR |  |
|  | RUS NW |  |
|  | USA E |  |
|  | USA W |  |
| 22 724.4 | E |  |
| (22 723) | FIN |  |
|  | GRC |  |
| (2210) | HOL |  |
|  | J |  |
|  | UKR |  |
|  | USA E |  |
| 22 727.4 | CHN |  |
| (22 726) | CUB |  |
|  | DNK |  |
| (2211) | I |  |
|  | J |  |
|  | S |  |
|  | UKR |  |
| 22 730.4 | ALS |  |
| (22 729) | AUS |  |
|  | CYP |  |
| (2212) | G |  |
|  | >> >> |  |
| (2212) | << << |  |
|  | GUM |  |
|  | HNG |  |
|  | HWA |  |
|  | MCO | ADD |
|  | PNR |  |
|  | PTR |  |
|  | SNG |  |
|  | USA E |  |
|  | USA W |  |
| 22 733.4 | BUL |  |
| (22 732) | CAN E |  |
|  | DNK |  |
| (2213) | E |  |
|  | GEO |  |
|  | IRQ |  |
|  | LBY |  |
|  | LTU |  |
|  | NZL |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | S |  |
|  | TUR |  |
| 22 736.4 | BEL |  |
| (22 735) | CHN |  |
|  | E |  |
| (2214) | FIN |  |
|  | IRN |  |
|  | RUS NW |  |
|  | SUI |  |
|  | TUR |  |
|  | URG |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 739.4 | CHN |  |
| (22 738) | F |  |
|  | GHA | ADD |
| (2215) | GRC |  |
|  | IRQ |  |
|  | J |  |
|  | NOR |  |
|  | POL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 742.4 | CAN W |  |
| (22 741) | DNK |  |
|  | GRC |  |
| (2216) | GUM |  |
|  | I |  |
|  | J |  |
|  | MTN |  |
|  | USA E |  |
|  | USA SO |  |
| 22 745.4 | ALS |  |
| (22 744) | D1 |  |
|  | E |  |
| (2217) | GRC |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | IRN |  |
|  | ISR |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 22 748.4 | ALS |  |
| (22 747) | CHN |  |
|  | CYP |  |
| (2218) | DNK |  |
|  | F |  |
|  | GUM |  |
|  | HWA |  |
|  | PTR |  |
|  | S |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 751.4 | BEL |  |
| (22 750) | CHN |  |
|  | CUB |  |
| (2219) | GRC |  |
|  | MCO |  |
|  | POL |  |
|  | SMO |  |
| 22 754.4 | CAN W |  |
| (22 753) | CHN |  |
|  | CZE |  |
| (2220) | D2 |  |
|  | G |  |
|  | GRC |  |
|  | SEN |  |
|  | SUI | ADD |
|  | SVK |  |
| 22 760.4 | ARS |  |
| (22 759) | AZR |  |
|  | CPV |  |
| (2222) | D1 |  |
|  | FIN |  |
|  | GRC |  |
|  | KOR |  |
|  | MDR |  |
|  | MLD | ADD |
|  | POR |  |
|  | TLS |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 763.4 | ALS |  |
| (22 762) | AUS |  |
|  | D1 |  |
| (2223) | HWA |  |
|  | I |  |
|  | J |  |
|  | MLT | ADD |
|  | PTR |  |
|  | TUR |  |
|  | USA E |  |
|  | USA W |  |
| 22 766.4 | ALS |  |
| (22 765) | D2 |  |
|  | E |  |
| (2224) | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | IRQ |  |
|  | MAU |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 22 769.4 | ALG |  |
| (22 768) | BEL |  |
|  | CHL |  |
| (2225) | GRC |  |
|  | IND W |  |
|  | ISL |  |
|  | J |  |
| 22 772.4 | ALB |  |
| (22 771) | ALS | ADD |
|  | CHN |  |
| (2226) | D2 |  |
|  | EGY |  |
|  | F |  |
|  | HWA | ADD |
|  | ISL |  |
|  | JOR | ADD |
|  | ROU |  |
|  | USA W | ADD |
| 22 775.4 | ALG |  |
| (22 774) | G |  |
|  | GRC |  |
| (2227) | IND E |  |
|  | J |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 778.4 | AUS |  |
| (22 777) | DNK |  |
|  | GRC |  |
| (2228) | MRC |  |
|  | QAT | ADD |
|  | >> >> |  |
| (2228) | << << |  |
|  | RUS EO |  |
|  | S |  |
|  | USA E |  |
|  | USA W |  |
| 22 781.4 | BEN | ADD |
| (22 780) | BRU | ADD |
|  | CAN E |  |
| (2229) | E |  |
|  | G |  |
|  | IND W |  |
|  | J |  |
|  | UKR |  |
| 22 784.4 | ALS |  |
| (22 783) | AUS |  |
|  | AZE |  |
| (2230) | D2 |  |
|  | E |  |
|  | GUM |  |
|  | HWA |  |
|  | KAZ |  |
|  | KGZ |  |
|  | PNR |  |
|  | PTR |  |
|  | RUS AS |  |
|  | S |  |
|  | TUR |  |
|  | USA E |  |
|  | USA W |  |
| 22 787.4 | ALS |  |
| (22 786) | ARS |  |
|  | CAN W |  |
| (2231) | EST |  |
|  | F |  |
|  | FIN |  |
|  | GRC |  |
|  | J |  |
|  | LVA |  |
|  | MLA |  |
|  | NIU |  |
|  | RUS SW |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 790.4 | CUB |  |
| (22 789) | GEO |  |
|  | GRC |  |
| (2232) | HOL |  |
|  | IRQ |  |
|  | LTU |  |
|  | LVA |  |
|  | POL |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SUI |  |
| 22 793.4 | ALS |  |
| (22 792) | CKH |  |
|  | GRC | ADD |
| (2233) | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | ROU |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 796.4 | ARG |  |
| (22 795) | DNK |  |
|  | INS |  |
| (2234) | J |  |
|  | LBY |  |
|  | NOR |  |
|  | ROU |  |
|  | S |  |
| 22 799.4 | ALS |  |
| (22 798) | F |  |
|  | GRC |  |
| (2235) | GUM |  |
|  | HWA |  |
|  | IRN |  |
|  | J |  |
|  | PTR |  |
|  | QAT | ADD |
|  | RUS NW |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 802.4 | DNK |  |
| (22 801) | E |  |
|  | GRC |  |
| (2236) | IRQ |  |
|  | J |  |
|  | NZL |  |
|  | UKR |  |
|  | USA E |  |
|  | USA W |  |
| 22 805.4 | AZR |  |
| (22 804) | CHN |  |
|  | I |  |
| (2237) | IRN |  |
|  | J |  |
|  | MDR |  |
|  | NOR |  |
|  | POR |  |
|  | ROU |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 808.4 | ALG |  |
| (22 807) | AUS |  |
|  | B |  |
| (2238) | D1 |  |
|  | GRC |  |
|  | HNG |  |
|  | IRQ |  |
|  | J |  |
|  | LTU |  |
|  | LVA |  |
|  | RUS SW |  |
|  | RUS W |  |
| 22 811.4 | ALS |  |
| (22 810) | BEL |  |
|  | CHN |  |
| (2239) | E |  |
|  | GUM |  |
|  | HRV |  |
|  | HWA |  |
|  | IND E |  |
|  | IRN |  |
|  | NOR |  |
|  | PNR |  |
|  | PTR |  |
|  | USA E |  |
|  | USA W |  |
| 22 814.4 | CHL |  |
| (22 813) | GRC |  |
|  | J |  |
| (2240) | MDG |  |
|  | NOR |  |
|  | TUN |  |
| 22 817.4 | ALS |  |
| (22 816) | AZE |  |
|  | CHN |  |
| (2241) | CLM |  |
|  | GEO |  |
|  | HKG |  |
|  | HWA |  |
|  | J |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
|  | VTN |  |
| 22 820.4 | BLR |  |
| (22 819) | CLM |  |
|  | RUS AN |  |
| (2242) | >> >> |  |
| (2242) | << << |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA SO |  |
| 22 823.4 | ALS |  |
| (22 822) | AUS |  |
|  | B |  |
| (2243) | BUL |  |
|  | HWA |  |
|  | J |  |
|  | KOR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS W |  |
|  | UAE |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 22 826.4 | ALS |  |
| (22 825) | HWA |  |
|  | I |  |
| (2244) | J |  |
|  | RUS AN |  |
|  | RUS EO |  |
|  | UKR |  |
|  | USA W |  |
| 22 829.4 | ALS |  |
| (22 828) | ARG CL |  |
|  | ARG SO |  |
| (2245) | CHN |  |
|  | E |  |
|  | HWA |  |
|  | J |  |
|  | RUS EO |  |
|  | UAE |  |
|  | USA SO |  |
|  | USA W |  |
| 22 832.4 | B |  |
| (22 831) | J |  |
|  | KGZ |  |
| (2246) | KOR |  |
|  | LVA |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | SUI |  |
|  | TUR |  |
|  | USA SO |  |
| 22 835.4 | ALS |  |
| (22 834) | CAN E |  |
|  | HWA |  |
| (2247) | J |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 22 838.4 | ALS |  |
| (22 837) | CHN |  |
|  | E |  |
| (2248) | HWA |  |
|  | MDA | ADD |
|  | PTR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 841.4 | ALS |  |
| (22 840) | CHN |  |
|  | HWA |  |
| (2249) | I |  |
|  | J |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS W |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 844.4 | ALS |  |
| (22 843) | AZE |  |
|  | B |  |
| (2250) | DGA |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | KAZ |  |
|  | KOR |  |
|  | MDW |  |
|  | MNE |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | TUR |  |
|  | >> >> |  |
| (2250) | << << |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 847.4 | ALS |  |
| (22 846) | B |  |
|  | BLR |  |
| (2251) | CHN |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MCO | ADD |
|  | MDW |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 850.4 | ALS |  |
| (22 849) | G |  |
|  | GUM |  |
| (2252) | HWA |  |
|  | J |  |
|  | LVA |  |
|  | PTR |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | UAE |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 22 853.4 | ALS |  |
| (22 852) | AUS |  |
|  | AZE |  |
| (2253) | CHN |  |
|  | DGA |  |
|  | E |  |
|  | G |  |
|  | GEO |  |
|  | GRC |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | KAZ |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS NW |  |
|  | >> >> |  |
| (2253) | << << |  |
|  | RUS W |  |
|  | TKM |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 26 146.4 | ALS |  |
| (26 145) | AZE |  |
|  | B |  |
| (2501) | CAN E |  |
|  | CHN |  |
|  | D1 |  |
|  | HNG |  |
|  | HWA |  |
|  | JOR |  |
|  | MNE |  |
|  | RUS EO |  |
|  | TUR |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 26 149.4 | ALS |  |
| (26 148) | AUS |  |
|  | BLR |  |
| (2502) | CHN |  |
|  | G |  |
|  | HWA |  |
|  | J |  |
|  | MOZ |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 26 152.4 | ARG CL |  |
| (26 151) | BUL |  |
|  | CHN |  |
| (2503) | J |  |
|  | RUS EO |  |
|  | SUI |  |
|  | UAE |  |
|  | USA SO |  |
| 26 155.4 | ALS |  |
| (26 154) | ARG SO |  |
|  | B |  |
| (2504) | >> >> |  |
| (2504) | << << |  |
|  | BLR |  |
|  | CHN |  |
|  | HWA |  |
|  | J |  |
|  | PTR |  |
|  | RUS AN |  |
|  | RUS AS |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | TKM |  |
|  | UKR |  |
|  | USA CL |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
|  | VIR |  |
| 26 158.4 | ALS |  |
| (26 157) | B |  |
|  | CHN |  |
| (2505) | E |  |
|  | GUM |  |
|  | HWA |  |
|  | IND E |  |
|  | IND W |  |
|  | ISR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS NW |  |
|  | RUS SW |  |
|  | RUS W |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 26 161.4 | ALS |  |
| (26 160) | ARG |  |
|  | CHN |  |
| (2506) | HWA |  |
|  | I |  |
|  | J |  |
|  | S | ADD |
|  | TUR |  |
|  | USA SO |  |
|  | USA W |  |
| 26 164.4 | ALS |  |
| (26 163) | ARG |  |
|  | AZE |  |
| (2507) | CAN E |  |
|  | CHN |  |
|  | DGA |  |
|  | >> >> |  |
| (2507) | << << |  |
|  | E |  |
|  | GRC |  |
|  | GUM |  |
|  | HKG |  |
|  | HWA |  |
|  | J |  |
|  | KAZ |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS EO |  |
|  | TKM |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |
| 26 167.4 | ALS |  |
| (26 166) | AUS |  |
|  | B |  |
| (2508) | CAN W |  |
|  | CHN |  |
|  | DGA |  |
|  | GRC |  |
|  | GUM |  |
|  | HNG |  |
|  | JOR |  |
|  | MDW |  |
|  | POR |  |
|  | PTR |  |
|  | RUS EO |  |
|  | RUS SW |  |
|  | TUR |  |
|  | UKR |  |
|  | USA E |  |
|  | >> >> |  |
| (2508) | << << |  |
|  | USA SO |  |
|  | USA W |  |
| 26 170.4 | ALS |  |
| (26 169) | ARG CL |  |
|  | ARG SO |  |
| (2509) | CHN |  |
|  | D2 |  |
|  | GUM |  |
|  | HWA |  |
|  | J |  |
|  | MDW |  |
|  | PTR |  |
|  | RUS EO |  |
|  | S | ADD |
|  | TUR |  |
|  | USA E |  |
|  | USA SO |  |
|  | USA W |  |

TABLE OF ALLOTMENTS ADDED TO THE PLAN
adopted by the WMARC-74

Column headings

1	Channel number (the corresponding carrier and assigned frequencies are indicated in Sub-section A of Section I of Part B Appendix 17 and in the present Appendix).

2	Country or area of allotment.

3	Service area description.

3.1	Main service area.

A number between 1 and 22 refers to a Zone defined on the Map of Maritime Zones appearing in the Preface to the BR IFIC.

3.2	Maximum length of circuit in kilometres.

4	Nature of service.

5	Class of emission.

6	Peak envelope power in dBW.

7	Transmitting antenna characteristics.

7.1	In the case of a non-directional antenna, the symbol ND is entered in this column and columns 7.2a), b) and c) are left blank. In the case of a directional antenna, the symbol D is entered in this column and the characteristics are given in columns 7.2a), b) and c).

7.2a)	Azimuth of maximum radiation. The symbol ROT entered in this column means that a rotatable antenna is used.

7.2b)	Angular width of main lobe.

7.2c)	Relative gain of the antenna in dB.

8	Planned scheduled hours of operation in the channel (UTC).

9	Traffic data.

9a)	Estimated peak hours of traffic.

9b)	Estimated daily volume of traffic in minutes.

10	Special section No./Weekly Circular or BR IFIC No./Date (e.g. MAR/10/1305/280278).     (WRC-03)

| 1 | 2 | 3 |  | 4 | 5 | 6 | 7 |  |  |  | 8 | 9 |  | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3.1 | 3.2 |  |  |  | 7.1 | 7.2 a) | 7.2 b) | 7.2 c) |  | 9a) | 9b) |  |
| 401 | AUS | 12 | 800 | CV | J3E | 20.0 | ND |  |  |  | 2200-1000 | 2200-1000 | 30 | MAR/54/1640/021084 |
| 401 | PNR | 9, 18 | 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-1200 |  | 25 | AR16/84/1838/160888 |
| 402 | BEN | 19 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2359 | 2000-0800 | 40 | AP25/133/2520/010604 |
| 403 | CAN CL | 2, 16 | 1 000 | CV | J3E | 30.0 | ND |  |  |  | 0000-2359 | 0800-2000 | 360 | AR16/120/2318/100398 |
| 403 | PNR | 9, 18 | 500 | CP | J3E | 30.0 | ND |  |  |  | 0800-1200 |  | 25 | AR16/84/1838/160888 |
| 404 | MCO | 17 | 300 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1000 1500-1700 | 50 | AP25/125/2379/250599 |
| 405 | USA CL | 16 | 800 | CP | J3E | 30.0 | ND |  |  |  | 1100-2300 | 1200-1800 | 180 | MAR/50/1609/280284 |
|  |  |  |  |  |  | 15.0 |  |  |  |  | 2300-1100 |  |  |  |
| 407 | AUS | 11, 12 | 800 | CO/CP | J3E | 37.0 | ND |  |  |  | 0000-2400 |  |  | MAR/48/1602/100184 |
| 407 | I | 17 | 1 200 | CO | J3E | 31.8 | ND |  |  |  | 0500-2200 | 0700-1100 | 60 | MAR/58/1682/300785 |
| 408 | B | 18, 20 | 800 | CV | J3E | 21.8 | ND |  |  |  | 0000-2400 |  | 120 | MAR/69/1712/040386 |
| 408 | CHN | 5 | 200 | OT | J3E | 26.0 | D | 340 | 60 | 3 | 1100-1900 | 1200-1300 | 190 |  |
| 408 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 408 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 408 | SMA | 8, 12, 13 | 1 000 | CP | J3E | 30.0 | ND |  |  |  | 1800-0400 |  | 30 | MAR/10/1305/280278 |
| 409 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 409 | QAT | 6 | 2 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 |  |  | AR16/89/1886/250789 |
| 411 | AMS | 10 | – | CP | J3E | 24.8 | ND |  |  |  | 0430-0445 0830-0845 1230-1245 |  | 25 | MAR/15/1347/191278 |
| 411 | EQA | 9 | 800 | CP | J3E | 24.0 | ND |  |  |  | 0030-0530 |  | 30 | AR16/90/1895/260989 |
| 411 | I | 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0500-2200 | 0700-1100 | 60 | AR16/75/1747/041186 |
| 411 | KIR | 7, 8 | 500 | CP | J3E | 27.0 | ND |  |  |  | 0800-1800 |  |  | MAR/59/1686/270885 |
| 416 | ARG CL | 14, 20 | 1 000 | CP | J3E | 30.0 | D | 90 | 60 | 2 | 0000-2400 | 1100-1700 | 490 |  |
| 417 | TZA | 6, 10, 19, 21 | 3 200 | CO/CP | J3E | 37.0 | ND |  |  |  | 0700-1800 | 0800-1000 1500-1700 | 240 | MAR/66/1707/280186 |
| 418 | B | 18, 20 | 800 | CV | J3E | 21.8 | ND |  |  |  | 0000-2400 | 0700-1100 | 240 | MAR/69/1712/040386 |
| 418 | I | 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0500-2200 | 0700-1100 | 60 | AR16/75/1747/041186 |
| 419 | TZA | 6, 10, 19, 21 | 3 200 | CO/CP | J3E | 37.0 | ND |  |  |  | 0700-1800 | 0800-1000 1500-1700 | 240 | MAR/57/1680/160785 |
| 422 | SUI | 15, 16, 17, 18, 19 | 4 000 | CP | J2D | 37.0 | D | ROT | 30 | 8 | 0000-2400 | 1900-0200 | 240 | AP25/147/2718/010512 |
| 423 | B | 18, 20 | 800 | CV | J3E | 27.0 | ND |  |  |  | 0000-2400 |  |  | MAR/16/1350/160179 |
| 423 | MLT | 6, 15, 17 | 3 000 | CP | J3E | 31.8 | ND |  |  |  | 1700-0500 | 2000-2100 | 60 | MAR/41/1565/190483 |
| 423 | QAT | 6 6 6 6 | 800 1 500 1 500 1 500 | CP CP CP CP | J3E J3E J3E J3E | 37.0 37.0 37.0 37.0 | ND D D D | 130 200 310 | 60 60 60 | 9 9 9 | 0000-2400 0000-2400 0000-2400 0000-2400 |  | 200 200 200 200 | MAR/23/1412/010480 |
| 424 | AUS E | 12 | 800 | CO/CP | J3E | 30.0 | ND |  |  |  | 0000-2400 |  |  | MAR/48/1602/100184 |
| 424 | PNR | 9, 18 | 500 | CP | J3E | 30.0 | ND |  |  |  | 0800-1200 |  | 25 | AR16/73/1742/300986 |
| 425 | B | 18, 20 | 800 | CV | J3E | 27.0 | ND |  |  |  | 1000-2300 | 1900-2200 | 100 | MAR/16/1350/160179 |
| 425 | JOR | 6, 15, 17 | 5 000 | CP | J3E | 37.0 | ND |  |  |  | 1700-0500 |  |  | MAR/49/1604/240184 |
| 428 | BRU | 7 | 1200 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 601 | CAN NO | 02, 04, 16 | - | CP | J3EJN | 29.2 | ND |  |  |  | 0000-2400 | 0800-2000 | 20 | AP25/149/2731/301012 |
| 601 | I | 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0400-2200 | 0600-1400 | 60 | AR16/75/1747/041186 |
| 601 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 601 | NCL | 7, 8, 12 | 2 500 | CP | J3E | 27.0 | ND |  |  |  | 0000-2400 |  |  | AR16/71/1737/260886 |
| 602 | AUS E | 12 | 1 000 | CV | J3E | 26.0 | ND |  |  |  | 0000-2359 | 1900-0700 |  | AP25/128/2406/301199 |
| 602 | B | 18, 20 | 800 | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 |  |  | MAR/69/1712/040386 |
| 602 | EQA | 9 | 800 | CP | J3E | 24.0 | ND |  |  |  | 0630-1000 |  | 30 | AR16/90/1895/260989 |
| 602 | FJI | 12 | 1 000 | CP | J3E | 30.0 | ND |  |  |  | 1800-0600 | 2000-0500 | 120 | MAR/37/1519/180582 |
| 602 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 603 | AUS | 11, 12 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 | 2100-0900 | 30 | MAR/55/1651/181284 |
| 603 | MLT | 6, 15, 17 | 3 000 | CP | J3E | 31.8 | ND |  |  |  | 0500-1700 | 0900-1100 | 60 | MAR/41/1565/190483 |
| 603 | VTN | 5, 6, 7 | – | CP | J3E | 34.8 | ND |  |  |  | 0000-2400 | 0630-1700 | 230 | AP25/146/2710/100112 |
| 604 | B | 18, 20 | 800 | CP | J3E | 30.0 | ND |  |  |  | 1000-1300 1700-2000 |  |  | MAR/69/1712/040386 |
| 604 | BES | 18 | 1 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-0200 0600-1000 |  | 120 | MAR/35/1495/171181 |
| 604 | CUW | 18 | 1 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-0200 0600-1000 |  | 120 | MAR/35/1495/171181 |
| 604 | RUS EO | 01, 03, 05 | 1500 | CP | J3E | 30.0 | ND |  |  | 2.7 | 0000-2400 | 0000-2400 | 200 | AP25/151/2742/160413 |
| 604 | SXM | 18 | 1 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-0200 0600-1000 |  | 120 | MAR/35/1495/171181 |
| 604 | TUV | 8, 12 | 450 | CP | J3E | 30.0 | ND |  |  |  | 1800-1200 | 2000-0400 | 30 | AR16/91/1897/101089 |
| 604 | VTN | 5, 6, 7, 10, 11 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0100-0400 1000-1800 | 600 | AP25/137/2632/111108 |
| 605 | B | 18, 20 | 800 | CP | J3E | 30.0 | ND |  |  |  | 1000-1300 1700-2000 |  |  | MAR/69/1712/040386 |
| 605 | F | 15, 17 | 2 500 | CP | J3E | 40.0 | ND |  |  |  | 0600-0900 1700-2200 | 1800-2200 | 300 | MAR/56/1679/090785 |
| 605 | NZL | 7, 8, 11, 12, 13 | 6 000 | CP | J3E | 37.0 | ND |  |  |  | 0000-2400 | 0400-0900 | 90 | MAR/63/1695/291085 |
| 607 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 608 | BRU | 7, 11 | 1300 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 802 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 803 | SUI | 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | D | ROT | 30 | 8 | 0600-0200 | 0600-1000 1700-2200 | 50 | MAR/62/1694/221085 |
| 804 | JOR | 6, 15, 17 | 5 000 | CP | J3E | 37.0 | ND |  |  |  | 0500-1700 |  |  | MAR/49/1604/240184 |
| 804 | QAT | 6 6 6 6, 17 | 1 500 2 500 2 500 2 500 | CP CP CP CP | J3E J3E J3E J3E | 37.0 37.0 37.0 37.0 | ND D D D | 130 200 310 | 60 60 60 | 10 10 10 | 0000-2400 0000-2400 0000-2400 0000-2400 |  | 200 200 200 200 | MAR/23/1412/010480 |
| 805 | EQA | 9 | 800 | CP | J3E | 24.0 | ND |  |  |  | 1130-1730 |  | 30 | AR16/90/1895/260989 |
| 806 | AUS | 11 | 2 000 | CP | J3E | 30.0 | ND |  |  |  | 2100-0500 | 2100-0500 | 90 | MAR/52/1631/310784 |
| 806 | SMA | 8, 12, 13 | 3 000 | CP | J3E | 30.0 | ND |  |  |  | 1800-0400 |  | 30 | MAR/11/1310/040478 |
| 807 | I | 15, 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0000-2400 | 0500-1300 | 60 | AR16/75/1747/041186 |
| 808 | I | 15, 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0000-2400 | 1300-2100 | 60 | AR16/75/1747/041186 |
| 811 | BEN | 19 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2359 | 0800-2000 | 40 | AP25/133/2520/010604 |
| 812 | I | 15, 17 | – | CO | J3E | 31.8 | ND |  |  |  | 0000-2400 | 2100-0500 | 60 | AR16/75/1747/041186 |
| 813 | VTN | 5, 6, 7, 10, 11 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0100-0400 1100-1400 | 650 | AP25/137/2632/111108 |
| 814 | KIR | 7, 8 | 500 | CP | J3E | 27.0 | ND |  |  |  | 1800-0800 |  |  | MAR/65/1702/171285 |
| 815 | JOR | 6, 17 | 3 000 | CP | J3E | 37.0 | ND |  |  |  | 0700-2000 | 0800-1200 | 60 | AR16/100/2084/060793 |
| 817 | PNR | 9, 18 | 2 000 | CP | J3E | 30.0 | ND |  |  |  | 1200-2300 |  | 25 | AR16/84/1838/160888 |
| 819 | PNR | 9, 18 | 2 000 | CP | J3E | 30.0 | ND |  |  |  | 1200-2300 |  | 25 | AR16/84/1838/160888 |
| 820 | D2 | 6, 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | ND |  |  |  | 0400-2000 |  | 30 | AR16/82/1827/310588 |
| 820 | TZA | 6, 10, 19, 21 | 3 200 | CO/CP | J3E | 37.0 | ND |  |  |  | 0700-1800 | 0800-1000 1500-1700 | 240 | MAR/66/1707/280186 |
| 822 | AUS | 11, 12 | 3 000 | CP | J3E | 30.0 | ND |  |  |  | 2100-0900 | 2100-0900 | 90 | MAR/64/1696/051185 |
| 823 | TZA | 6, 10, 19, 21 | 3 200 | CO/CP | J3E | 30.0 | ND |  |  |  | 0700-1800 | 0800-1000 1500-1700 | 240 | MAR/66/1707/280186 |
| 823 | USA W | 9 | 1 200 | CO | J3E | 30.0 | ND |  |  |  | 1600-0400 | 1600-1800 0000-0200 | 180 | AR16/92/1910/230190 |
| 825 | AMS | 10 | – | CP | J3E | 24.8 | ND |  |  |  | 0445-0500 0845-0900 1245-1300 |  | 25 | MAR/15/1347/191278 |
| 825 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 825 | S | 5, 15 5, 15 6, 10, 17 6, 10, 17, 19, 21 15, 16, 17, 18, 19, 21 15, 16, 18, 19 15, 16 | – | CP | J3E | 40.0 40.0 40.0 40.0 40.0 40.0 40.0 | D D D D D D D | 10 50 130 170 210 250 310 | 60 60 60 60 60 60 60 | 11 11 11 11 11 11 11 | 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 | 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 | 90 90 90 90 90 90 90 | AR16/70/1730/080786 |
| 826 | CAN NO | 02, 04, 16 | – | CP | J3EJN | 29.2 | ND |  |  |  | 0000-2400 | 0800-2000 | 20 | AP25/148/2720/290512 |
| 826 | QAT | 6 | 2 500 | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 |  |  | AR16/89/1886/250789 |
| 829 | BRM | 5, 6, 7 | 3 300 | CP | J3E | 24.0 | ND |  |  |  | 2330-1130 | 0330-0430 | 30 | AR16/112/2223/160496 |
| 829 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 830 | CHN | 5, 6, 7, 8 | 8 000 | CP | J3E | 38.5 | ND |  |  |  | 0000-2400 | 0000-0800 | 400 |  |
| 830 | MCO | 15, 17 | 800 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1000 1500-1700 | 50 | AP25/125/2379/250599 |
| 830 | VTN | 5, 6, 7 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0800-1800 | 320 | AP25/146/2710/100112 |
| 834 | BHR | 6 | – | CO | J3E | 30.0 | ND |  |  |  | 0001-2400 | 1300-0100 | 300 | AP25/145/2707/151111 |
| 835 | BRU | 7, 11 | 1500 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 1 201 | QAT | 6 | 2 500 | CP | J3E | 30.0 | ND |  |  |  | 0400-0600 1400-1600 |  |  | AR16/89/1886/250789 |
| 1 207 | EQA | 9 | 800 | CP | J3E | 24.0 | ND |  |  |  | 1830-2330 |  | 30 | AR16/90/1895/260989 |
| 1 208 | I | 6, 15, 16, 17, 18 | – | CO | J3E | 31.8 | ND |  |  |  | 0300-2200 | 0600-1100 | 30 | AR16/75/1747/041186 |
| 1 210 | SUI | 6, 10, 15, 16, 17, 18, 19, 20, 21 | 9 000 | CP | J3E | 40.0 | D | ROT | 30 | 8 | 0600-0200 | 0800-1200 1600-2100 | 60 | MAR/62/1694/221085 |
| 1 212 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 1 213 | USA W | 9 | 1 600 | CO | J3E | 30.0 | ND |  |  |  | 1800-2300 | 2100-2200 | 180 | AR16/95/1996/011091 |
| 1 215 | BHR | 6 | – | CO | J3E | 37.0 | ND |  |  |  | 0001-2400 | 1300-0100 | 300 | AP25/145/2707/151111 |
| 1 220 | D2 | 6, 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | ND |  |  |  | 0400-2000 |  | 30 | AR16/82/1827/310588 |
| 1 220 | JOR | 6, 15, 17 | 5 000 | CP | J3E | 37.0 | ND |  |  |  | 0500-1700 |  |  | MAR/49/1604/240184 |
| 1 222 | ALS | 4 | 1 600 | CO | J3E | 30.0 | ND |  |  |  | 2000-0100 | 2300-2400 | 180 | AR16/95/1996/011091 |
| 1 222 | BEN | 19 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2359 | 0800-2000 | 20 | AP25/133/2520/010604 |
| 1 222 | USA W | 9 | 1 600 | CO | J3E | 30.0 | ND |  |  |  | 1800-2300 | 2100-2200 | 180 | AR16/95/1996/011091 |
| 1 224 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 1 225 | JOR | 6, 10 | 5 000 | CP | J3E | 37.0 | D | 144 | 60 | 9 | 0900-1700 | 1300-1500 | 30 | AR16/100/2084/060793 |
| 1 226 | MCO | 01, 02, 06, 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1000 1500-1700 | 50 | AP25/125/2379/250599 |
| 1 226 | S | 5, 15 5, 15 6, 10, 17 6, 10, 17, 19, 21 15, 16, 17, 18, 19, 21 15, 16, 18, 19 15, 16 | – | CP | J3E | 40.0 40.0 40.0 40.0 40.0 40.0 40.0 | D D D D D D D | 10 50 130 170 210 250 310 | 60 60 60 60 60 60 60 | 11 11 11 11 11 11 11 | 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 0000-2400 | 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 0800-1000 | 90 90 90 90 90 90 90 | AR16/70/1730/080786 |
| 1 227 | TZA | 6, 10, 19, 21 | 3 200 | CO/CP | J3E | 37.0 | ND |  |  |  | 0700-1800 | 0800-1000 1500-1700 | 240 | MAR/66/1707/280186 |
| 1 228 | I | 6, 15, 16, 17, 18 | – | CO | J3E | 31.8 | ND |  |  |  | 2200-0500 | 2300-0200 | 30 | AR16/75/1747/041186 |
| 1 228 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 1 228 | VTN | 5, 6, 7, 10, 11 | – | CP | J3E | 37.0 | ND |  |  |  | 0000-2400 | 0100-0400 1000-1800 | 800 | AP25/137/2632/111108 |
| 1 229 | QAT | 6, 17 6 6, 17 6, 17 | 2 000 3 000 3 000 3 000 | CP CP CP CP | J3E J3E J3E J3E | 37.0 37.0 37.0 37.0 | ND D D D | 130 200 310 | 60 60 60 | 11 11 11 | 0400-0600 1400-1600 0400-0600 1400-1600 0400-0600 1400-1600 0400-0600 1400-1600 |  | 200 | MAR/23/1412/010480 |
| 1 231 | VTN | 5, 6, 7 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0000-1500 | 398 | AP25/146/2710/100112 |
| 1 232 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1200-2400 |  | 25 | AR16/84/1838/160888 |
| 1 232 | SMA | 8, 12, 13 | 3 000 | CP | J3E | 30.0 | ND |  |  |  | 1800-0400 |  | 30 | MAR/11/1310/040478 |
| 1 236 | BRM | 5, 6, 7 | 3 300 | CP | J3E | 24.0 | ND |  |  |  | 2330-1130 | 0330-0430 | 30 | AR16/112/2223/160496 |
| 1 238 | MCO | 15, 16, 17 | 5 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1600 | 120 | AP25/129/2445/290501 |
| 1239 | BRU | 5, 7, 11 | 2000 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 1 603 | MLT | 15, 17 | 3 000 | CP | J3E | 31.8 | ND |  |  |  | 0000-1159 |  |  | MAR/21/1379/070879 |
| 1 604 | BEN | 19 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2359 |  | 20 | AP25/133/2520/010604 |
| 1 608 | EQA | 9, 14 | 800 | CP | J3E | 27.0 | ND |  |  |  | 1800-2300 | 2000-2300 | 40 | AR16/111/2221/020496 |
| 1 612 | JOR | 6, 10 | 6 000 | CP | J3E | 37.0 | D | 144 | 60 | 9 | 1000-1600 | 1300-1500 | 20 | AR16/100/2084/060793 |
| 1 614 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 1 622 | ALS | 4 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 2000-0600 | 0200-0300 | 180 | AR16/95/1996/011091 |
| 1 622 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 1 622 | HWA | 8 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 2000-0600 | 0200-0300 | 180 | AR16/95/1996/011091 |
| 1 622 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1200-2400 |  | 25 | AR16/84/1838/160888 |
| 1 622 | SUI | 3, 4, 5, 6, 7, 9, 10, 15, 16, 17, 18, 19, 20, 21 | 10 000 | CP | J3E | 40.0 | D | ROT | 30 | 8 | 0600-0200 | 0800-1700 | 60 | MAR/62/1694/221085 |
| 1 626 | J | 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14 | – | CR | J3E | 37.0 | ND |  |  |  | 0000-2400 | 0800-1000 | 500 |  |
| 1 626 | QAT | 6, 17 6 6, 10, 17 6, 15, 17 | 4 000 6 000 6 000 6 000 | CP CP CP CP | J3E J3E J3E J3E | 40.0 40.0 40.0 40.0 | ND D D D | 130 200 310 | 60 60 60 | 11 11 11 | 0600-0800 1200-1400 0600-0800 1200-1400 0600-0800 1200-1400 0600-0800 1200-1400 |  | 200 | MAR/23/1412/010480 |
| 1 627 | ALS | 4 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 2000-0600 | 0200-0300 | 180 | AR16/95/1996/011091 |
| 1 628 | EQA | 9, 14 | 800 | CP | J3E | 27.0 | ND |  |  |  | 1800-2300 | 2000-2300 | 40 | AR16/111/2221/020496 |
| 1 628 | MCO | 01, 02, 06, 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1000 1400-1600 | 50 | AP25/125/2379/250599 |
| 1 629 | BRM | 5, 6, 7 | 3 300 | CP | J3E | 24.0 | ND |  |  |  | 2330-1130 | 0330-0430 | 30 | AR16/112/2223/160496 |
| 1 630 | J | 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14 | – | CR | J3E | 37.0 | ND |  |  |  | 0000-2400 | 0300-0700 | 650 |  |
| 1 634 | CHN | 8, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 1 635 | I | 5, 6, 7, 9, 10, 14, 15, 16, 18, 20, 21 | – | CO | J3E | 31.8 | ND |  |  |  | 0400-2400 | 0600-1600 | 30 | AR16/75/1747/041186 |
| 1 635 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1500-2400 |  | 25 | AR16/84/1838/160888 |
| 1 637 | CHN | 8, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 1 638 | SMA | 8, 12, 13 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1800-0400 |  | 30 | MAR/10/1305/280278 |
| 1 638 | D2 | 6, 15, 16, 17, 18, 19 | 6 000 | CP | J3E | 40.0 | ND |  |  |  | 0400-2000 |  | 30 | AR16/82/1827/310588 |
| 1 639 | CHN | 5 | 800 | OT | J3E | 31.8 | D | 90 | 60 | 3 | 0000-1200 | 0100-0230 | 300 |  |
| 1 640 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1500-2400 |  | 25 | AR16/84/1838/160888 |
| 1 644 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 1 646 | VTN | 5, 6, 7, 10, 11 | – | CP | J3E | 37.0 | ND |  |  |  | 0000-2400 | 0830-1630 | 750 | AP25/137/2632/111108 |
| 1 804 | BRU | 5, 6, 7, 10, 11, 12 | 5000 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 1 804 | S | 06, 15, 16, 17, 18, 19 | 7 000 | CP | J3E | 38.5 | ND |  |  |  | 0000-2359 | 0600-1900 | 120 | AP25/126/2388/270799 |
| 1 808 | MCO | 15, 16, 17 | 5 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1600 | 120 | AP25/129/2445/290501 |
| 1 809 | POL | 5, 11, 21 | 20 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-2230 | 1730-2230 | 90 | AR16/119/2310/130198 |
| 1 813 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 1 813 | S | 06, 15, 16, 17, 18, 19 | 7 000 | CP | J3E | 38.5 | ND |  |  |  | 0000-2359 | 0600-1900 | 120 | AP25/130/2445/290501 |
| 2 202 | BRM | 5, 6, 7 | 3 300 | CP | J3E | 24.0 | ND |  |  |  | 2330-1130 | 0330-0430 | 30 | AR16/112/2223/160496 |
| 2 203 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1500-2400 |  | 25 | AR16/84/1838/160888 |
| 2 206 | BHR | 6, 10, 15, 17, 19, 21 | – | CP | J3E | 34.8 | ND |  |  |  | 0000-2359 |  |  | AR16/100/2084/060793 |
| 2 208 | I | 5, 6, 7, 9, 10, 14, 15, 16, 18, 20, 21 | – | CO | J3E | 31.8 | ND |  |  |  | 0500-2400 | 0700-2200 | 30 | AR16/75/1747/041186 |
| 2 208 | PNR | 9, 14, 16, 18 | 4 000 | CP | J3E | 30.0 | ND |  |  |  | 1200-2400 |  | 25 | AR16/84/1838/160888 |
| 2 209 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 2 211 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 18 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 240 |  |
| 2 212 | MCO | 01, 02, 06, 10, 15, 16, 17, 18, 19, 20, 21 | 8 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1000 1400-1600 | 50 | AP25/125/2379/250599 |
| 2 215 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 2 215 | GHA | 19 | 500 | CP | J3E | 30.0 | ND D D | 110 330 | 30 30 | 10 10 | 0000-2359 |  |  | AR16/114/2237/230796 |
| 2 218 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 2 220 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 240 |  |
| 2 220 | SUI | 6, 10, 18, 20, 21 | 14 000 | CP | J3E | 40.0 | D | ROT | 70 | 8,5 | 0600-1800 | 0900-1600 | 60 | MAR/27/1431/120880 |
| 2 222 | MLD | 6 | – | CO | J3E | 30.0 | D | 300 | 120 | 5 | 0000-2400 |  |  | AR16/79/1816/150388 |
| 2 223 | MLT | 15, 17 | 3 000 | CP | J3E | 31.8 | ND |  |  |  | 0000-1159 |  |  | MAR/20/1372/190679 |
| 2 226 | ALS | 4 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 2000-0400 | 0100-0200 | 180 | AR16/95/1996/011091 |
| 2 226 | HWA | 8 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 2000-0400 | 0100-0200 | 180 | AR16/95/1996/011091 |
| 2 226 | JOR | 6, 10, 11 | 8 000 | CP | J3E | 37.0 | D | 144 | 60 | 9 | 1100-1400 |  |  | AR16/100/2084/060793 |
| 2 226 | USA W | 9 | 2 400 | CO | J3E | 30.0 | ND |  |  |  | 1800-0200 | 2300-2400 | 180 | AR16/95/1996/011091 |
| 2 228 | QAT | 6, 10, 11 | 2 500 | CP | J3E | 33.0 | D | 140 | 60 | 10 | 0000-1800 | 0400-1100 |  | AR16/96/1997/081091 |
| 2 229 | BEN | 19 | – | CP | J3E | 30.0 | ND |  |  |  | 0000-2359 |  | 20 | AP25/133/2520/010604 |
| 2 229 | BRU | 5, 6, 7, 8, 10, 11, 12 | 6000 | CP | J3E | 21.76 | ND |  |  |  | 2100-1400 | 2200-0100 | 100 | AP25/141/2691/050411 |
| 2 233 | GRC | 17 | 2 600 | CO | J3E | 30.0 | ND |  |  |  | 0500-2200 | 0600, 1000, 2200 | 30 | MAR/51/1621/220584 |
| 2 235 | QAT | 6, 17 6, 10, 11 6, 10, 17, 21 17, 15 | 5 000 8 000 8 000 8 000 | CP CP CP CP | J3E J3E J3E J3E | 40.0 40.0 40.0 40.0 | ND D D D | 130 200 310 | 60 60 60 | 11 11 11 | 0800-1200 0800-1200 0800-1200 0800-1200 |  | 200 200 200 200 | MAR/23/1412/010480 |
| 2 237 | CHN | 4, 9, 10, 11, 12, 13, 14, 15, 17, 19, 20, 21 | 19 000 | CP | J3E | 40.0 | ND |  |  |  | 0000-1000 | 0200-0600 | 200 |  |
| 2 239 | CHN | 7 | 2 700 | CP | J3E | 20.0 | ND |  |  |  | 0100-0930 | 0200-0400 | 280 |  |
| 2 248 | MDA | 17 | – | CO | J3E | 30.0 | ND |  |  |  | 0000-2400 | 0300-2000 | 180 | AP25/142/2692/190411 |
| 2 251 | MCO | 15, 16, 17 | 5 000 | CP | J3E | 40.0 | ND |  |  |  | 0700-2200 | 0800-1600 | 120 | AP25/129/2445/290501 |
| 2 506 | S | 06, 15, 16, 17, 18, 19 | 7 000 | CP | J3E | 38.5 | ND |  |  |  | 0000-2359 | 0600-1900 | 120 | AP25/130/2445/290501 |
| 2 509 | S | 06, 15, 16, 17, 18, 19 | 7 000 | CP | J3E | 38.5 | ND |  |  |  | 0000-2359 | 0600-1900 | 120 | AP25/126/2388/270799 |

APPENDIX 26 (REV.WRC-15)*

Provisions and associated Frequency Allotment Plan for the aeronautical 
mobile (OR) service in the bands allocated exclusively to that 
service between 3 025 kHz and 18 030 kHz

(See Article 43)

PART I  –  General provisions, definitions

26/1	The provisions of this Appendix shall apply to the aeronautical mobile (OR) service in the following frequency bands:

| 3 025-3 155 kHz 3 900-3 950 kHz (Region 1 only) 4 700-4 750 kHz 5 680-5 730 kHz 6 685-6 765 kHz | 8 965- 9 040 kHz 11 175-11 275 kHz 13 200-13 260 kHz 15 010-15 100 kHz 17 970-18 030 kHz. |
| --- | --- |

26/2	For the purpose of this Appendix, the terms used comprise the following:

## 26/2.1	Frequency Allotment Plan

The Plan for the aeronautical mobile (OR) service contained in Part III of this Appendix.

## 26/2.2	Allotment in the aeronautical mobile (OR) service

A frequency allotment in the aeronautical mobile (OR) service which comprises:

–	a frequency channel from the channels appearing in the channelling arrangement in No. 26/3;

–	a bandwidth of up to 2.8 kHz, situated wholly within the frequency channel concerned;

–	a power within the limits laid down in No. 26/4.4 or specified against the allotted frequency channel;

–	an allotment area which is the area in which the aeronautical station can be situated and which coincides with all or part of the territory of the country, or of the geographical area, as indicated against the frequency channel concerned in the Frequency Allotment Plan.

PART  II  –  Technical bases used for the establishment of the Frequency Allotment Plan for the aeronautical mobile (OR) service in the bands allocated exclusively to 
that service between 3 025 kHz and 18 030 kHz

## 26/3	Channelling arrangement

26/3.1	The channelling arrangement for the frequencies to be used by aeronautical stations in the aeronautical mobile (OR) service in the bands allocated exclusively to that service between 3 025 kHz and 18 030 kHz is indicated in Table 1.

TABLE 1

| Frequency band 3 025-3 155 kHz: 43 + 1 channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 023 1 3 053 3 083 3 113 3 143 | 3 026 3 056 3 086 3 116 3 146 | 3 029 3 059 3 089 3 119 3 149 | 3 032 3 062 3 092 3 122 3 152 | 3 035 3 065 3 095 3 125 | 3 038 3 068 3 098 3 128 | 3 041 3 071 3 101 3 131 | 3 044 3 074 3 104 3 134 | 3 047 3 077 3 107 3 137 | 3 050 3 080 3 110 3 140 |
| Frequency band 3 900-3 950 kHz (Region 1 only): 16 channels |  |  |  |  |  |  |  |  |  |
| 3 900 3 930 | 3 903 3 933 | 3 906 3 936 | 3 909 3 939 | 3 912 3 942 | 3 915 3 945 | 3 918 | 3 921 | 3 924 | 3 927 |
| Frequency band 4 700-4 750 kHz: 16 channels |  |  |  |  |  |  |  |  |  |
| 4 700 4 730 | 4 703 4 733 | 4 706 4 736 | 4 709 4 739 | 4 712 4 742 | 4 715 4 745 | 4 718 | 4 721 | 4 724 | 4 727 |
| Frequency band 5 680-5 730 kHz: 15 + 1 channels |  |  |  |  |  |  |  |  |  |
| 5 6801 5 711 | 5 684 5 714 | 5 687 5 717 | 5 690 5 720 | 5 693 5 723 | 5 696 5 726 | 5 699 | 5 702 | 5 705 | 5 708 |
| Frequency band 6 685-6 765 kHz: 26 channels |  |  |  |  |  |  |  |  |  |
| 6 685 6 715 6 745 | 6 688 6 718 6 748 | 6 691 6 721 6 751 | 6 694 6 724 6 754 | 6 697 6 727 6 757 | 6 700 6 730 6 760 | 6 703 6 733 | 6 706 6 736 | 6 709 6 739 | 6 712 6 742 |
| Frequency band 8 965-9 040 kHz: 25 channels |  |  |  |  |  |  |  |  |  |
| 8 965 8 995 9 025 | 8 968 8 998 9 028 | 8 971 9 001 9 031 | 8 974 9 004 9 034 | 8 977 9 007 9 037 | 8 980 9 010 | 8 983 9 013 | 8 986 9 016 | 8 989 9 019 | 8 992 9 022 |
| Frequency band 11 175-11 275 kHz: 33 channels |  |  |  |  |  |  |  |  |  |
| 11 175 11 205 11 235 11 265 | 11 178 11 208 11 238 11 268 | 11 181 11 211 11 241 11 271 | 11 184 11 214 11 244 | 11 187 11 217 11 247 | 11 190 11 220 11 250 | 11 193 11 223 11 253 | 11 196 11 226 11 256 | 11 199 11 229 11 259 | 11 202 11 232 11 262 |
| Frequency band 13 200-13 260 kHz: 20 channels |  |  |  |  |  |  |  |  |  |
| 13 200 13 230 | 13 203 13 233 | 13 206 13 236 | 13 209 13 239 | 13 212 13 242 | 13 215 13 245 | 13 218 13 248 | 13 221 13 251 | 13 224 13 254 | 13 227 13 257 |
| Frequency band 15 010-15 100 kHz: 30 channels |  |  |  |  |  |  |  |  |  |
| 15 010 15 040 15 070 | 15 013 15 043 15 073 | 15 016 15 046 15 076 | 15 019 15 049 15 079 | 15 022 15 052 15 082 | 15 025 15 055 15 085 | 15 028 15 058 15 088 | 15 031 15 061 15 091 | 15 034 15 064 15 094 | 15 037 15 067 15 097 |
| Frequency band 17 970-18 030 kHz: 20 channels |  |  |  |  |  |  |  |  |  |
| 17 970 18 000 | 17 973 18 003 | 17 976 18 006 | 17 979 18 009 | 17 982 18 012 | 17 985 18 015 | 17 988 18 018 | 17 991 18 021 | 17 994 18 024 | 17 997 18 027 |
| 1 For use of the carrier (reference) frequencies 3 023 kHz and 5 680 kHz, see No. 26/3.4. |  |  |  |  |  |  |  |  |  |

26/3.2	The frequencies indicated in No. 26/3.1 are the carrier (reference) frequencies.

26/3.3	With the exception of the carrier (reference) frequencies 3 023 kHz and 5 680 kHz (see No. 26/3.4), one or more frequencies from Table 1 may be assigned to any aeronautical station and/or aircraft station, in accordance with the Frequency Allotment Plan, as contained in Part III of this Appendix.

26/3.4	The carrier (reference) frequencies 3 023 kHz and 5 680 kHz are intended for worldwide common use (see also Appendix 27, Nos. 27/232 to 27/238).

26/3.5	The aeronautical radiotelephone stations shall use only single-sideband emissions (J3E). The upper sideband shall be employed, and the assigned frequency (see No. 1.148) shall be 1 400 Hz higher than the carrier (reference) frequency.

26/3.6	The channelling arrangement specified in No. 26/3.1 does not prejudice the rights of administrations to establish, and to notify assignments to stations in the aeronautical mobile (OR) service other than those using radiotelephony, provided that:

–	the occupied bandwidth does not exceed 2 800 Hz and is situated wholly within one frequency channel;

–	the limits of unwanted emission are met (see Appendix 27, No. 27/74).     (WRC-2000)

## 26/4	Classes of emission and power

26/4.1	In the aeronautical mobile (OR) service, in the bands governed by this Appendix, the use of the emissions listed below is permissible; additionally, the use of other emissions is also permissible, subject to compliance with No. 26/3.6.

## 26/4.2	Telephony

–	J3E (single-sideband, suppressed carrier).

## 26/4.3	Telegraphy (including automatic data transmission)

–	A1A, A1B, F1B;

–	(A,H)2(A,B);

–	(R,J)2(A,B,D);

–	J(7,9)(B,D,X).

26/4.4	Unless otherwise specified in Part III of this Appendix, the following transmitter power limits (i.e. power supplied to the antenna), shall be applied:

| Class of emission | Power limit values (peak envelope power supplied to the antenna) |  |
| --- | --- | --- |
|  | Aeronautical station | Aircraft station |
| J3E | 36 dBW (PX) | 23 dBW (PX) |
| A1A, A1B | 30 dBW (PX) | 17 dBW (PX) |
| F1B | 30 dBW (PX) | 17 dBW (PX) |
| A2A, A2B | 32 dBW (PX) | 19 dBW (PX) |
| H2A, H2B | 33 dBW (PX) | 20 dBW (PX) |
| (R,J)2(A,B,D) | 36 dBW (PX) | 23 dBW (PX) |
| J(7,9)(B,D,X) | 36 dBW (PX) | 23 dBW (PX) |

26/4.5	On the assumption that no antenna gain is involved, the transmitter powers specified in No. 26/4.4 above will result in a mean effective radiated power of 1 kW (for the aeronautical stations) and 50 W (for the aircraft stations), used as the basis for the establishment of the Plan contained in Part III of this Appendix.

PART III  –  Arrangement for the allotment of frequencies for the aeronautical
mobile (OR) service in the exclusive bands between 3 025 and 18 030 kHz

## 26/5.1	Column headings

Column 1:	Carrier (reference) frequency, in kHz.

Column 2:	Allotment area (See Notes a), b) and c) below).

26/5.2	Whenever the allotment area is followed by another administration's code, indicated in parentheses, the notifications are receivable from the latter administration on the basis of an agreement in accordance with Resolution 1 (Rev.WRC-97).

NOTE a): The allotment area is designated by the symbol of the country or the geographical area, the meaning of which is given in the Preface to the IFL. The meaning of the following symbol, which does not appear in the Preface to the IFL, is given below:

CG7	CUB(Guantanamo) (7), as defined in Appendix 26 to the Radio Regulations, Geneva, 1959; (7) means “United States of America stations”

NOTE b): For ease of reference, the allotment arrangement is presented by ITU Region. The symbols REG1, REG2 and REG3 correspond to the definitions of Regions 1, 2 and 3 respectively; the symbol REGY is used for the allotment area ATA (Antarctica), whose parts lie in all three Regions.

NOTE c): The allotment which is followed by an asterisk (*) is subject to coordination with another administration (see Notes on the concluded operational agreements which follow the Arrangement).

| 1 | 2 |  |
| --- | --- | --- |
| 3 026 | REG1 | ARS BEN G KAZ KGZ LIE MCO RUS |
|  | REG2 | ATG DMA GRD JMC KNA LCA VCT |
|  | REG3 | BRU KOR TON |
| 3 029 | REGY | ATA(ARG) |
|  | REG1 | ARS AZR BLR COG E F G I IRQ KAZ MDA NOR POL RUS SEN TUN UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CLM HWA USA |
|  | REG3 | AUS CHN GUM IND J KOR MHL(USA) NZL PNG VTN |
| 3 032 | REGY | ATA(ARG) |
|  | REG1 | ALG AZR BLR COG CTI E EGY F HNG IRQ KAZ MDA MDG MLT MRC NOR OMA POL RUS SEN TUN UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CAN CLM DOM GRL HWA SLV USA |
|  | REG3 | AUS CBG CHN GUM IND J J(USA) LAO MHL(USA) NZL PNG VTN VUT |
| 3 035 | REGY | ATA(ARG) |
|  | REG1 | ARM ARS BFA BHR(USA) BLR COG F G G(USA) GEO HRV I(USA) ISL KAZ KGZ LVA MLT MRC NOR RUS SEN TCD TJK TKM TUN TUR |
|  | REG2 | ALS ARG B BER(USA) BRB(USA) CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CHN GUM IND INS J(USA) NZL PNG |
| 3 038 | REGY | ATA(ARG) |
|  | REG1 | ARM ARS BFA BHR(USA) BLR COG CTI CYP(G) EGY F G G(USA) GEO GRC HRV I(USA) ISL KAZ KGZ LVA MDG MNE MRC MTN* NOR OMA REU RUS SEN SRB SVN TCD TJK TKM TUN |
|  | REG2 | ALS ARG ATG(USA) B BAH(USA) BER(USA) BRB(USA) CAN CG7 GRL HWA MDW MRT NCG PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS CBG CHN GUM IND INS J(USA) LAO MHL(USA) NCL NZL OCE PNG VTN VUT |
| 3 041 | REG1 | ALG G I ISL KWT NMB RUS TJK |
|  | REG3 | HKG IRN KRE PHL TUV |
| 3 044 | REGY | ATA(ARG) |
|  | REG1 | AFS ALG CME COG CZE DJI(F) F G GAB I ISR KAZ LTU MDA MDG MLI* MTN POR ROU RUS SEN* TCD TJK TKM UKR |
|  | REG2 | ARG CAN CLM JON MEX |
|  | REG3 | AUS BGD CHN GUM IRN J NCL NZL OCE PAK PNG |
| 3 047 | REGY | ATA(ARG) |
|  | REG1 | AFS ALG AZE BLR CME COG CTI CZE DJI(F) E F GAB IRL ISL ISR KAZ LTU MDA MDG MLI* MLT MTN NIG POR RUS SEN* TCD TKM TUR UKR |
|  | REG2 | ARG CAN CLM CTR HTI HWA JON MEX |
|  | REG3 | AUS BGD CBG CHN FJI GUM INS J(USA) LAO NCL NZL OCE PNG VTN VUT |
| 3 050 | REGY | ATA(ARG) |
|  | REG1 | AZE AZR BLR CME COG DNK F G GIB I KAZ MDG MLI MLT MRC POR REU RUS SEN* TCD TJK UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CAN CUB HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN DGA(USA) FJI GUM IND IRN J(USA) MHL(USA) NZL PAK PNG |
| 3 053 | REGY | ATA(ARG) |
|  | REG1 | ALB AZR CME COG CTI DNK F G GIB HNG KAZ MDG MLI MRC POR RUS SEN* TCD TJK UKR UZB |
|  | REG2 | ALS ARG B BER(USA) BES CAN CUB CUW GTM HWA MDW PNR PTR SXM USA |
|  | REG3 | AUS CHN FJI GUM IND INS IRN J(USA) MHL(USA) NZL PNG VTN |
| 3 056 | REG1 | BLR COG D EST F G GAB GIB KAZ MDG MLI ROU RUS SEN* TCD TJK UAE UKR UZB |
|  | REG2 | B BES CAN CUW HWA JON MEX MRT SXM USA |
|  | REG3 | AUS GUM IND INS J(USA) KOR PNG |
| 3 059 | REG1 | AZR BLR COG CTI D E F G GAB GRC I KAZ MDG MLI REU ROU RUS SEN* SYR TCD TKM UKR UZB |
|  | REG2 | B CAN CHL HWA JON MEX MRT USA |
|  | REG3 | AUS IND INS J J(USA) KOR NZL PNG VTN |
| 3 062 | REG1 | G GUI I ROU RUS SSD SWZ TKM |
|  | REG3 | IRN J |
| 3 065 | REGY | ATA(ARG) |
|  | REG1 | ARM AZE AZR D F G JOR LVA POR ROU RUS S TJK TKM UKR |
|  | REG2 | ALS ARG B BER(USA) CUB GRL HWA JON PNR USA |
|  | REG3 | AUS GUM IND IRN J MHL(USA) PNG |
| 3 068 | REGY | ATA(ARG) |
|  | REG1 | ARM AZE AZR ERI ETH F G HOL ISL LTU LVA MNE POR RUS S SRB SYR TJK TKM UAE UKR |
|  | REG2 | ALS ARG B BER(USA) CAN CG7 CUB HWA JON PNR PRU USA |
|  | REG3 | AUS CBG GUM INS J(USA) LAO MHL(USA) PNG VTN |
| 3 071 | REGY | ATA(ARG) |
|  | REG1 | AGL AZE BUL DJI(F) F G GRC HOL I ISL KAZ KGZ LTU LVA MOZ POR REU RUS STP TKM TUN UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CLM JON MDW USA |
|  | REG3 | AUS BGD CHN HKG J MHL(USA) PAK PNG |
| 3 074 | REGY | ATA(ARG) |
|  | REG1 | AGL AZE AZR BUL CPV EGY F G GIB GRC HNG I KAZ KGZ LVA MLT MOZ NIG POR RUS S STP TUN UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CAN CLM GRL GTM HTI JON MDW USA |
|  | REG3 | AUS BGD CHN CLN GUM HKG J MHL(USA) MLA PAK PNG SNG* |
| 3 077 | REGY | ATA(ARG) |
|  | REG1 | ARS AZR CYP(G) D F G GRC KGZ LVA MLT POR RUS UKR |
|  | REG2 | ALS ARG B CAN HWA PRG URG USA VEN |
|  | REG3 | AUS CHN HKG J KOR NZL PNG SNG |
| 3 080 | REGY | ATA(ARG) |
|  | REG1 | ARS AZR CYP(G) D EGY F FIN G GIB KEN KGZ LBY LVA MLT POR ROU RUS SOM TUR UKR |
|  | REG2 | ALS ARG B CAN CUB HWA PRG PRU SLV URG USA VEN |
|  | REG3 | AUS CHN CLN FJI GUM HKG IND J J(USA) KOR MLA* NZL PNG SNG |
| 3 083 | REG1 | CYP(G) G GMB GRC I KGZ QAT RUS |
|  | REG3 | HKG J MLD |
| 3 086 | REG1 | AFS BLR CYP(G) D F G GRC KAZ KGZ MDA OMA ROU RUS SVK UKR UZB |
|  | REG2 | ALS B BER(USA) CAN CG7 CHL HWA MDW PNR PTR USA |
|  | REG3 | AUS BRM CHN GUM J(USA) MHL(USA) PNG |
| 3 089 | REGY | ATA(USA) |
|  | REG1 | ALG AZE BLR D EGY G GRC GRC(USA) I I(USA) KAZ MDA MRC POR ROU RUS SEY SUI SVK UAE UKR UZB |
|  | REG2 | ALS B BER(USA) CG7 CHL GRL HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN GUM J(USA) MHL(USA) PNG |
| 3 092 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS AZE AZR DJI(F) F G GEO GIB ISL KAZ POL REU RUS TJK TKM UZB |
|  | REG2 | ALS ARG B BER(USA) CAN CG7 DOM HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BGD CHN GUM J MHL(USA) NZL PNG |
| 3 095 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS CYP(G) E EGY F G GEO GIB GRC(USA) I ISR KAZ KEN LBY MLT POL RUS SOM TJK TKM UZB ZWE |
|  | REG2 | ALS ARG B CAN CG7 CTR DOM HWA MDW MEX PNR PRU PTR USA |
|  | REG3 | AUS BGD CHN CLN FJI GUM HKG J MHL(USA) MLA NZL PNG SNG* |
| 3 098 | REG1 | ALB AZE AZR BHR(USA) BLR CNR E G GEO GIB I I(USA) KAZ NIG RUS TJK UKR |
|  | REG2 | ALS ATG(USA) B BAH(USA) BER(USA) BRB(USA) CHL HWA MDW MRT PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS BGD GUM HKG J MHL(USA) PAK PNG |
| 3 101 | REG1 | AFS ALB AZE AZR BHR(USA) BLR CNR D E EGY ERI ETH G GEO GIB GRC(USA) HNG I I(USA) ISL KAZ LBY MLT RUS SUI TJK TUN UKR |
|  | REG2 | ALS B BER(USA) BRB(USA) CAN CHL GRL HND HWA MDW MRT PNR PTR TRD(USA) USA |
|  | REG3 | AUS BGD CHN CLN GUM HKG J MHL(USA) MLA PAK PNG SNG* |
| 3 104 | REG1 | E GEO GIB I IRL ISL RUS SDN TUN UAE UKR |
|  | REG2 | ALS |
|  | REG3 | J NPL |
| 3 107 | REG1 | CNR D E F G GRC(USA) I KAZ LTU MDA MNG RUS S UKR ZMB |
|  | REG2 | ALS B BER(USA) CG7 CHL HWA MDW PNR PTR USA |
|  | REG3 | AUS BRM CHN GUM IND INS J MHL(USA) PAK PNG |
| 3 110 | REG1 | AFS ALB AZR CNR D E EGY G GRC(USA) I ISL KAZ LTU MDA MNG MRC NIG RUS S TJK TUR UKR UZB |
|  | REG2 | ALS B BER(USA) CAN CG7 CHL GRL HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN DGA(USA) GUM IND INS J(USA) MHL(USA) PAK PNG |
| 3 113 | REG1 | ALB ALG AZE BLR E F G G(USA) GRC ISL KAZ KEN KGZ MDA RUS SVK TJK TKM TUN UKR UZB |
|  | REG2 | B CAN CHL DOM MEX USA VEN |
|  | REG3 | AUS CHN GUM HKG J(USA) PAK PNG SNG |
| 3 116 | REG1 | AFS ALG AZE BLR D EGY G GIB I ISL KAZ KGZ MDA MLT MNG RUS SVK TJK TKM TUN UKR UZB |
|  | REG2 | B CAN CHL CTR DOM EQA MEX USA VEN |
|  | REG3 | AUS CHN CLN HKG IND J J(USA) MLA NZL PAK PNG SNG* |
| 3 119 | REGY | ATA(ARG) |
|  | REG1 | ALB BLR DJI F G GRC(USA) HOL I I(USA) KAZ MRC ROU RUS SVN UKR UZB |
|  | REG2 | ALS ARG B BER(USA) HWA MDW PNR PTR USA |
|  | REG3 | AUS BGD CHN FJI GUM IND INS J KIR MHL(USA) PNG |
| 3 122 | REGY | ATA(ARG) |
|  | REG1 | AZR BLR E EGY F G GEO GRC(USA) HOL I I(USA) KAZ MRC ROU RUS TUR UKR |
|  | REG2 | ALS ARG B BER(USA) BOL CAN GRL HWA MDW PNR PTR USA |
|  | REG3 | AUS BGD CHN FJI GUM INS J KIR MHL(USA) NZL PAK PNG |
| 3 125 | REG1 | BLR CYP(G) G GEO HOL KAZ LBR MLT MNG MWI ROU RUS SMR |
|  | REG2 | BLZ |
|  | REG3 | J PAK SMO |
| 3 128 | REG1 | BEL BLR G GRC HNG HOL I KAZ LVA NIG ROU RUS UKR |
|  | REG2 | ALS BES CAN CUB CUW HWA MDW PNR PTR SXM URG USA |
|  | REG3 | AUS CHN FJI GUM HKG IND INS J MHL(USA) NCL NZL OCE PAK PNG |
| 3 131 | REG1 | BEL EGY G GRC HOL I LSO LVA MNG RUS SRL TKM UKR |
|  | REG2 | ALS BES BOL CAN CHL CUB CUW EQA GTM HWA MDW PNR PTR SUR SXM URG USA |
|  | REG3 | AUS CHN CKH FJI GUM IND INS J MHL(USA) NCL NZL OCE PAK PNG VUT |
| 3 134 | REG1 | ARM ARS(USA) AZE AZR BUL D(USA) E G HOL I KAZ LVA OMA RUS TJK TKM TUR(USA) UKR UZB |
|  | REG2 | ALS B BER(USA) DOM HWA JON PRG USA VEN |
|  | REG3 | AUS CHN GUM IND J MHL(USA) PNG TLS |
| 3 137 | REG1 | ARM ARS(USA) AZE AZR BHR BUL D(USA) E EGY F G G(USA) I ISL KAZ LVA MDA MNG MRC NIG RUS TJK TKM TUR(USA) UKR UZB |
|  | REG2 | ALS B BER(USA) CAN CHL DOM EQA GRL GTM HWA JON PRG SUR USA VEN |
|  | REG3 | AUS CHN GUM IND J(USA) MHL(USA) PHL(USA) PNG TLS |
| 3 140 | REGY | ATA(ARG) |
|  | REG1 | ALG AZE CME COG D F G GAB GEO GRC I KAZ LVA MDA MDG MKD MLI ROU RUS SEN* TCD TJK UKR |
|  | REG2 | ALS ARG B BER(USA) GRL HWA JON PNR USA |
|  | REG3 | AUS CHN GUM J J(USA) MHL(USA) PNG |
| 3 143 | REGY | ATA(ARG) |
|  | REG1 | ALG AZE BIH CME COG CTI CYP(G) D EGY F G GAB GEO GIB GRC HRV KAZ KGZ LVA MDG MKD MLI* MLT MNE MRC ROU RUS SEN SRB SVN TCD TJK TUN UKR |
|  | REG2 | ALS ARG B BER(USA) CAN GRL HWA JON PNR USA |
|  | REG3 | AUS BRM CHN GUM J J(USA) MHL(USA) PNG |
| 3 146 | REG1 | AZE BEL COM CYP G GHA I KGZ MLT MNG RUS |
|  | REG2 | BAH |
|  | REG3 | J NRU PAK |
| 3 149 | REG1 | AGL ALG AZE BLR BUL CME COG D D(F) EST G GAB GHA GRC I KAZ MDG MLI* MLT MTN ROU RUS SEN* TCD TUN UKR |
|  | REG2 | ALS CAN DOM HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BRM CHN GUM INS J PAK PNG WAK |
| 3 152 | REG1 | ALG BLR BUL CME COG CTI D D(F) EGY G GAB KAZ MDG MLI* MRC NIG ROU RUS SEN TCD TUN UAE UKR |
|  | REG2 | ALS ARG B BOL CAN CHL CLM DOM EQA HWA MDW MEX PNR PRG PRU PTR SUR URG USA VEN |
|  | REG3 | AUS CHN GUM INS J NZL PNG WAK |
| 3 900 | REG1 | ALG BIH CME COG CZE D E F G ISL KAZ KGZ LTU MDA MDG MLI* OMA RUS SEN TCD TJK TKM TUN TUR UKR |
| 3 903 | REG1 | AFS ALG CME COG CTI CZE D EGY F G HRV ISL KAZ KGZ LTU MDA MDG MLI MNE MRC REU RUS SEN* SRB SVN TCD TJK TKM TUN TZA UGA UKR |
| 3 906 | REG1 | ALB AZE BEL GMB HOL HRV IRL KAZ MLT NIG RUS TZA UGA UKR YEM |
| 3 909 | REG1 | AZE BLR COG DJI(F) E F G GIB HRV KAZ LVA MDG REU RUS SEN TCD UKR UZB |
| 3 912 | REG1 | BLR COG CTI EGY F G GIB HRV KAZ LVA MDG MNE MRC RUS SEN SRB SVN TCD UKR UZB |
| 3 915 | REG1 | ALB ALG BLR COM CZE F G GRC KAZ LTU LVA MNG ROU RUS SSD SVK TJK TKM UKR UZB YEM |
| 3 918 | REG1 | AFS ALB ALG BLR CZE EGY ERI ETH F G I KAZ LTU LVA MRC NIG ROU RUS SVK TJK TKM UKR UZB |
| 3 921 | REG1 | ALG DJI F G GRC KWT LVA MLT POR ROU RUS UKR UZB ZMB |
| 3 924 | REG1 | AZR BEN CYP(G) D EGY F G GEO GIB GRC LSO LVA MLT POR ROU RUS SEY UAE UKR |
| 3 927 | REG1 | BUL GEO GIB HOL IRL LBR LIE MWI RUS SDN TUR |
| 3 930 | REG1 | AFS ALG BUL CAF CME CYP(G) DJI(F) G GIB GRC HOL LVA MDG MLI MLT ROU RUS SMR SVK TUN UKR |
| 3 933 | REG1 | ALG AUT CAF CME CTI CYP(G) D DJI(F) E F G GIB GRC I KAZ LVA MDG MLI MLT MRC QAT ROU RUS SVK TUN UKR |
| 3 936 | REG1 | AFS AZE BEL CNR E G I KAZ NIG POL RUS TJK TUR UZB YEM |
| 3 939 | REG1 | AFS AZE CNR CYP(G) D E F G GRC I KAZ MLT POL RUS TJK TUN UZB YEM |
| 3 942 | REG1 | CYP CZE F G GIB ISL KAZ LVA NOR POL RUS SRL SWZ UKR UZB YEM |
| 3 945 | REG1 | AFS ALG CZE ERI ETH F G GIB GRC ISL KAZ LVA MRC NOR POL RUS SEN UKR UZB |
| 4 700 | REG1 | ARM ARS AZE BEN BHR(USA) CYP(G) G GIB I KAZ KEN LBY MLT POL RUS SEY SWZ TJK TKM |
|  | REG2 | ALS B CAN DOM HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BGD BRM CHN DGA(USA) FJI GUM HKG IND J(USA) KOR MAC MHL(USA) NZL PAK PNG TLS |
| 4 703 | REG1 | AFS ALG ARM ARS AZE AZR BHR(USA) CYP(G) DNK E EGY F G GEO GIB I KAZ KEN LBY MLT MRC POL RUS SOM TJK TKM TUR |
|  | REG2 | ALS B CAN CHL DOM HWA MDW MEX PNR PTR SUR USA |
|  | REG3 | AUS BGD BRM CHN CLN FJI GUM HKG IND J J(USA) KOR MAC MHL(USA) MLA NZL PAK PNG TLS |
| 4 706 | REGY | ATA(USA) |
|  | REG1 | ALG BLR CYP(G) D F G GEO HRV I I(USA) KAZ KEN KGZ LBY LSO LTU MDA MLT RUS TJK TKM TUR UKR YEM |
|  | REG2 | ALS B BER(USA) CAN CG7 HWA MDW PAQ PNR PRG PTR URG USA |
|  | REG3 | AUS CHN DGA(USA) GUM IND J(USA) MHL(USA) NZL SNG THA |
| 4 709 | REG1 | AFS ALG ARS BLR CYP(G) D F G GRC I I(USA) KAZ KEN KGZ LBR LBY LTU MDA MLT MNE OMA RUS SRB TJK TKM TUR UKR |
|  | REG2 | ALS B BER(USA) CG7 CHL HWA MDW PAQ PNR PRG PTR URG USA |
|  | REG3 | AUS CHN GUM IND INS J MHL(USA) NZL THA |
| 4 712 | REGY | ATA(USA) |
|  | REG1 | AZR BLR CYP(G) EGY F GIB I(USA) IRL ISL KAZ MLT MRC MWI POL ROU RUS SOM SRL SSD UKR YEM |
|  | REG2 | ALS BER(USA) CAN CG7 GRL HWA MDW PNR PRU PTR USA |
|  | REG3 | AUS CBG FJI GUM J(USA) KRE LAO MHL(USA) NPL PHL PNG VTN |
| 4 715 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AGL ALB AZR BHR(USA) BLR CME DJI(F) F G GMB GRC HOL I ISL ISR KAZ LTU MDA MNG MOZ POL POR RUS STP TCD TUN TUR UKR UZB |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BES BRB(USA) CAN CLM CUW HWA MDW PNR PTR SXM TCA(USA) TRD(USA) USA |
|  | REG3 | AUS BGD BRM FJI GUM HKG IND J(USA) MHL(USA) MLA PAK THA |
| 4 718 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AGL ALB ALG AZR BHR(USA) CME CPV DJI(F) F G HOL I ISL ISR KAZ KGZ LTU MDA MDG MLT MOZ POR RUS STP TCD TUN UKR UZB |
|  | REG2 | ALS ARG BER(USA) BES BRB(USA) CAN CLM CUW GRL HWA MDW PNR PRU PTR SXM TRD(USA) USA |
|  | REG3 | AUS BGD BRM CLN FJI GUM HKG IND J(USA) MHL(USA) MLA NZL PAK PNG SNG* THA |
| 4 721 | REGY | ATA(ARG) |
|  | REG1 | AGL ALG BLR CME CNR D D(USA) DJI(F) E F G GEO I KAZ KGZ MLT MOZ POR ROU RUS STP TCD TJK TUR(USA) UKR |
|  | REG2 | ALS ARG BER(USA) CAN CUB GRL HWA JON PNR PRU USA |
|  | REG3 | AUS BGD CHN GUM IND J(USA) MHL(USA) NCL NZL OCE PAK PNG THA TLS |
| 4 724 | REGY | ATA(ARG) |
|  | REG1 | AGL ALG AZR BEL BLR CME CNR CPV D D(USA) DJI(F) E EGY EST F G G(USA) GEO HNG I KAZ MDG MOZ POR REU RUS STP TCD TJK TUR(USA) UAE UKR |
|  | REG2 | ALS ARG BER(USA) CAN CG7 CUB GRL HWA JON PNR USA |
|  | REG3 | AUS BGD CBG CHN GUM IND INS J(USA) LAO MHL(USA) NCL NZL OCE PAK PHL(USA) PNG THA TLS VTN VUT |
| 4 727 | REG1 | AZE BEL BUL COG CYP(G) CZE DJI(F) F G GEO KAZ LVA MDG QAT ROU RUS SEN TCD TJK TUN TUR UKR |
|  | REG2 | ALS BER(USA) CAN CUB FLK GRL HWA JON URG USA |
|  | REG3 | AUS BRM CHN GUM IND J MHL(USA) THA TON |
| 4 730 | REG1 | AFS AZE BUL COG CTI CYP(G) CZE F G GEO I KAZ LVA MDG MNG ROU RUS SEN TJK TUN UKR YEM |
|  | REG2 | ALS ATG BER(USA) CAN CUB DMA EQA FLK GRD GRL HWA JMC JON KNA LCA URG USA VCT |
|  | REG3 | AUS BRM CHN GUM IND INS J(USA) MHL(USA) NZL THA |
| 4 733 | REG1 | ALG BDI BEL COM DJI E G GUI KWT LBN LIE MLT MRC NMB RUS S SDN SMR TKM UAE |
|  | REG2 | BAH HND HWA NCG PRU USA |
|  | REG3 | AUS BTN GUM J MLD NRU SMO VUT |
| 4 736 | REGY | ATA(ARG) |
|  | REG1 | AFS ALB ALG ARS AUT AZE AZR BLR BUL COG D DJI(F) E ERI ETH F GRC I IRL KAZ LBN MDG MLI MRC NOR OMA POR REU RUS SEN* TCD TJK TKM UKR UZB |
|  | REG2 | ALS ARG B BER(USA) CAN CG7 HND HWA JON MDW MEX MRT PNR PTR USA |
|  | REG3 | AUS CHN GUM IND J MHL(USA) NZL THA TUV WAK |
| 4 739 | REGY | ATA(ARG) |
|  | REG1 | ALB ALG ARS AUT AZE AZR BLR COG CTI D EGY F G GIB I ISL KAZ LBN MDG MLI NOR POR ROU RUS SEN* TCD TJK TKM UKR UZB |
|  | REG2 | ALS ARG B BOL CAN CG7 HWA JON MDW MRT PNR PTR USA |
|  | REG3 | AUS CHN FJI GUM IND J MHL(USA) MLA* NZL PAK PNG SNG THA WAK |
| 4 742 | REG1 | ALG CME COG CYP DJI(F) F G GEO GIB I KAZ MDG MKD MLI MNG POL POR REU ROU RUS SEN* TCD TGO TUN UZB YEM |
|  | REG2 | ALS BER(USA) CAN CHL GRL HND HWA JON PRG URG USA VEN |
|  | REG3 | AUS BRU CHN FJI GUM HKG IND IRN J J(USA) KOR MHL(USA) PAK PNG |
| 4 745 | REG1 | AZR BEL CME COG CTI D DJI(F) EGY F G GEO I ISL KAZ MDG MLI* MRC POL POR REU RUS SEN SUI TCD TGO TUN TUR UZB YEM ZMB |
|  | REG2 | ALS BER(USA) CAN CHL GRL HND HWA JON PRG URG USA VEN |
|  | REG3 | AUS CBG CHN FJI GUM IND IRN J(USA) KOR LAO MHL(USA) NZL PNG VTN |
| 5 684 | REGY | ATA(ARG) |
|  | REG1 | AGL ALB AZE AZR BLR CPV CYP D F G GEO I KAZ KWT LVA MOZ POR RUS SRL STP TJK TKM UKR UZB YEM |
|  | REG2 | ARG BES CAN CUW MEX PRG SXM USA |
|  | REG3 | AUS CHN GUM HKG IND J(USA) KOR SMO THA VTN |
| 5 687 | REGY | ATA(ARG) |
|  | REG1 | AFS AGL ALB AZE AZR BLR CPV D E EGY G GEO GIB HRV I KAZ LVA MNE MOZ NIG OMA POR RUS SRB STP SVN TJK TKM UKR UZB |
|  | REG2 | ARG BES CAN CUW EQA MEX PRG SXM USA |
|  | REG3 | AUS CHN GUM IND INS IRN J KOR NZL PNG THA VUT |
| 5 690 | REG1 | BDI DJI E GMB GNE GRC HOL I IRL ROU RUS SWZ TUR UAE |
|  | REG2 | HTI |
|  | REG3 | CHN IRN J TON |
| 5 693 | REGY | ATA(ARG) |
|  | REG1 | AFS ARS AZR CME COG CYP(G) F G GIB I IRQ ISL ISR KAZ LVA MLI MRC ROU RUS SVK TUN TUR UKR YEM |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BRB(USA) CAN CG7 HWA MDW PNR PTR TCA(USA) TRD(USA) USA VEN |
|  | REG3 | AUS BGD BRM GUM HKG J J(USA) MLA NZL PAK PNG THA |
| 5 696 | REGY | ATA(ARG) |
|  | REG1 | ARS BEL CME COG CTI CYP(G) EGY G GIB GRC(USA) IRQ ISL KAZ KEN LBY LVA MCO MDG MLI MLT OMA ROU RUS SOM SVK TUR UKR |
|  | REG2 | ALS ARG BER(USA) BOL BRB(USA) CAN CG7 GRL GTM HWA MDW MEX PNR PTR TRD(USA) USA VEN |
|  | REG3 | AUS BGD BRM CLN FJI GUM J(USA) NZL PAK SNG THA |
| 5 699 | REGY | ATA(ARG) |
|  | REG1 | ALG AZR BFA BLR CME DJI(F) F G GAB KAZ LTU LVA MDA MLI MNE MWI RUS SRB TCD TUR UKR |
|  | REG2 | ALS ARG CAN GRL GTM HWA MEX PTR USA |
|  | REG3 | AUS BRM CHN GUM IND IRN J MAC MHL(USA) NZL PAK THA VTN |
| 5 702 | REGY | ATA(ARG) |
|  | REG1 | ALG AZR BFA BLR CME CTI DJI(F) E EGY ERI ETH F G G(USA) GAB GRC HOL KAZ LSO LTU LVA MDA MDG MLI* MNE MRC MTN OMA POR REU ROU RUS SEN* SRB TCD TJK UKR UZB |
|  | REG2 | ALS ARG BOL CAN CLM GRL MEX USA |
|  | REG3 | AUS BRM CHN FJI IND INS IRN J(USA) MAC NZL PNG THA |
| 5 705 | REG1 | BEN CYP(G) ERI ETH F G GIB GRC HOL KAZ MLT QAT ROU RUS TJK UAE UKR UZB ZMB |
|  | REG2 | ATG B BLZ DMA GRD JMC KNA LCA VCT |
|  | REG3 | BRU HKG J MLD NPL NRU |
| 5 708 | REG1 | AFS AGL COG F GRC HNG IRL IRQ KAZ KGZ LBN MTN* NOR OMA POL ROU RUS SEN SEY SYR TJK TKM TUN TUR YEM |
|  | REG2 | ALS B BER(USA) BOL CAN CHL CLM GRL HWA MDW USA |
|  | REG3 | AUS BRM CHN IND J KOR MHL(USA) NZL PNG SNG THA TLS |
| 5 711 | REG1 | AGL COG CTI F G GIB GRC IRQ ISL KAZ KGZ LBN MDG MRC MTN* NOR POL RUS SEN SYR TJK TKM TUN TUR UAE UKR YEM |
|  | REG2 | ALS B BER(USA) BOL CAN CHL CLM GRL HWA MDW USA |
|  | REG3 | AUS BRM CHN IND J(USA) KOR MHL(USA) MLA NZL PNG THA TLS |
| 5 714 | REGY | ATA(USA) |
|  | REG1 | AFS ARM AUT AZE BLR BOT BUL CME CTI CYP(G) D D(F) DJI(F) F G GIB HRV I KAZ MLI MLT MNG NMB(AFS) REU ROU RUS TCD TGO TJK TKM TUN UKR UZB |
|  | REG2 | ALS B CAN CUB HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN DGA(USA) FJI GUM J(USA) MHL(USA) NZL PAK THA |
| 5 717 | REGY | ATA(USA) |
|  | REG1 | AFS ARM AUT AZE AZR BLR BOT BUL CME CTI CYP(G) D D(F) DJI(F) E EGY EST ERI ETH F G GRC KAZ MDG MLI MLT MRC NMB(AFS) OMA REU ROU RUS SEN* TCD TGO TJK TKM TUN UKR UZB |
|  | REG2 | ALS B BOL CAN CHL CUB GTM HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS CBG CHN DGA(USA) FJI GUM J(USA) LAO MHL(USA) NZL PAK PNG THA VTN |
| 5 720 | REG1 | ALG BEL COM CYP(G) G GIB ISL LBR LIE MLT NMB OMA ROU RUS SMR SSD TKM UAE |
|  | REG2 | BAH BOL GTM |
|  | REG3 | HKG IND J KRE PHL TUV |
| 5 723 | REGY | ATA(USA) |
|  | REG1 | AFS ALG AZE BHR(USA) BLR COG F G GRC(USA) HNG I ISL KAZ LVA MRC MTN NMB(AFS) POR RUS SEN* SOM SVK TKM UAE UKR |
|  | REG2 | ALS ATG(USA) B BER(USA) BRB BRB(USA) CAN CG7 CHL HND HWA MDW PNR PTR TCA(USA) URG USA |
|  | REG3 | AUS CHN GUM IND J J(USA) KOR MHL(USA) NCL OCE PNG THA |
| 5 726 | REGY | ATA(USA) |
|  | REG1 | AFS ALG AZE AZR BHR(USA) BLR COG CTI EGY F G GIB I ISL KAZ LVA MDG MTN NMB(AFS) POR ROU RUS S SEN* SVK TKM UKR YEM |
|  | REG2 | ALS ATG(USA) B BAH(USA) BER(USA) BRB CAN CG7 CHL GRL HND HWA MDW PNR PTR TCA(USA) URG USA |
|  | REG3 | AUS CBG CHN GUM IND J J(USA) KOR LAO MHL(USA) NCL NZL OCE THA VTN VUT |
| 6 685 | REG1 | AFS AGL ALB ARS AZE BHR(USA) CPV D EGY G GEO GNB GRC(USA) I I(USA) ISL KAZ MOZ MNE MRC NIG NOR POR RUS SRB STP SUI SVK TJK TUR UZB |
|  | REG2 | ALS B BER(USA) CAN CG7 DOM EQA HWA MDW MEX PNR PTR URG USA |
|  | REG3 | AUS CBG CHN CLN GUM HKG IND J LAO MHL(USA) MLA PAK PNG SNG* VTN |
| 6 688 | REG1 | ALB ALG AZR EGY F FIN G GRC(USA) HRV I I(USA) ISL MLT MRC RUS SVK TJK TUN YEM ZMB |
|  | REG2 | ALS CG7 DOM HWA MDW NCG PNR PTR USA |
|  | REG3 | AFG AUS BGD FJI GUM J KRE MHL(USA) PAK VUT |
| 6 691 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS AZR BUL CYP(G) CZE E G GHA GIB HNG I I(USA) KAZ KEN LBY MLT ROU RUS TJK TKM UZB |
|  | REG2 | ALS ARG CAN CLM HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BGD BRM CHN GUM HKG IND J J(USA) KOR PAK SLM SNG WAK |
| 6 694 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS AZR BLR BUL CYP(G) CZE EGY ERI ETH G GIB I I(USA) KAZ KEN LBY NIG OMA ROU RUS SOM TKM UZB |
|  | REG2 | ALS ARG CAN HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BRM CHN CLN FJI GUM HKG IND J(USA) KOR MLA NZL PNG SNG* WAK |
| 6 697 | REGY | ATA(ARG) |
|  | REG1 | ARS BDI BHR(USA) BLR CYP(G) D G I I(USA) ISL MLT MRC RUS SMR |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA MDW PNR PTR TRD USA |
|  | REG3 | AUS BGD GUM HKG J(USA) PAK THA |
| 6 700 | REGY | ATA(ARG) |
|  | REG1 | ARS AZR BHR(USA) CYP(G) D EGY F G GIB GRC I I(USA) ISL KEN LBY MLT MRC RUS SOM TUR |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BRB CAN CG7 GRL HWA MDW PNR PTR TCA(USA) TRD USA |
|  | REG3 | AUS BGD CLN GUM HKG J(USA) MHL(USA) MLA NZL PAK PNG SNG* THA |
| 6 703 | REG1 | ALB BEN ERI ETH I IRL ISL LUX NMB QAT RUS SEY SVN UKR |
|  | REG2 | HTI |
|  | REG3 | J MLD NPL PHL SMO |
| 6 706 | REG1 | AFS BLR CYP(G) EGY G GIB GNE GRC KAZ MDA MLT MNE RUS SRB SVK UKR UZB YEM |
|  | REG2 | ALS B CAN CUB HWA MDW PNR PTR USA |
|  | REG3 | AUS BGD CHN DGA(USA) FJI GUM HKG IND INS J KIR MAC MHL(USA) NZL PAK THA |
| 6 709 | REG1 | BEL BIH BLR CYP(G) G GEO HRV KAZ KEN LBY LSO MDA MLT MNE ROU RUS SOM SRB SVN UKR UZB |
|  | REG2 | ALS B CAN CUB HWA MDW PNR PTR SUR USA |
|  | REG3 | AUS BGD CHN CLN FJI GUM HKG IND INS J KIR MAC MHL(USA) NZL PAK PNG THA VTN |
| 6 712 | REG1 | AFS ALG AUT AZE BLR CME COG CYP(G) D D(F) DJI(F) F G GEO ISL ISR KAZ LVA MDG MLI* MLT MTN OMA REU ROU RUS SEN* TCD TGO TJK TKM TUN TUR TUR(USA) UKR UZB |
|  | REG2 | B CAN HWA MEX PNR USA |
|  | REG3 | AUS BRM CHN IND J(USA) KOR PAK THA TLS VTN |
| 6 715 | REG1 | AFS ALG AUT AZE BLR CME COG CTI D D(F) DJI(F) E F G G(USA) HNG ISR KAZ LVA MDG MLI MRC MTN* REU ROU RUS SEN* TCD TGO TJK TKM TUN TUR(USA) UAE UKR UZB |
|  | REG2 | B CAN GRL HWA MEX PNR SUR USA |
|  | REG3 | AUS BRM CHN FJI GUM IND INS J(USA) KOR NZL PAK PHL(USA) PNG THA TLS |
| 6 718 | REG1 | AGL ALG CYP F HOL IRL MLT NIG ROU TUR TZA UZB YEM |
|  | REG2 | BAH |
|  | REG3 | IND NRU PAK |
| 6 721 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AGL ARS AZR BHR(USA) F G GEO GRC(USA) HOL I I(USA) JOR KAZ LTU MDA MRC RUS SRL TJK TZA UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS CHN FJI GUM IND J(USA) MHL(USA) NZL SNG THA |
| 6 724 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AFS ARS BHR(USA) CNR E EGY G GEO GRC GRC(USA) HRV I I(USA) KAZ LBR LTU MDA MNE MRC RUS SRB SVN TJK UKR UZB |
|  | REG2 | ALS ARG BER(USA) CG7 GRL HWA MDW MEX PNR PTR SUR USA |
|  | REG3 | AUS CHN FJI GUM IND J(USA) MHL(USA) MLA* NZL PNG SNG THA |
| 6 727 | REGY | ATA(ARG) |
|  | REG1 | AGL ALG ARS ARS(USA) AZR D(USA) ERI ETH G GRC KAZ LIE MOZ RUS STP TUR(USA) UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN CUB GRL GUY HWA JON MDW PNR USA |
|  | REG3 | AUS CHN GUM IND J MHL(USA) THA |
| 6 730 | REGY | ATA(ARG) |
|  | REG1 | AGL ALG ARM ARS ARS(USA) AZR CPV D D(USA) DNK E ERI ETH F G GNB GRC ISL KAZ MOZ NIG POR ROU RUS STP SYR TUR(USA) UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 CUB GRL GUY HWA JON MDW PNR USA |
|  | REG3 | AUS CHN GUM IND J J(USA) MHL(USA) MLA NZL PAK PNG SNG* THA |
| 6 733 | REG1 | ALG ARM F G GUI I KEN NIG RUS SWZ TUR UAE YEM |
|  | REG2 | B |
|  | REG3 | IND J TUV VTN |
| 6 736 | REG1 | AFS ARM ASC(USA) AZE CYP(G) CZE G GIB GRC I ISL KEN MLT MRC NMB(AFS) OMA ROU RUS SEY(USA) TJK TKM |
|  | REG2 | ALS B BER(USA) CAN CHL CLM GTM HWA PNR PTR URG USA |
|  | REG3 | AUS BRM CHN GUM J KOR MHL(USA) PAK SNG THA VTN |
| 6 739 | REG1 | AFS ARM ASC(USA) AZE CYP(G) CZE EGY F G G(USA) I MLT NMB(AFS) ROU RUS TJK TKM TUR(USA) UKR YEM |
|  | REG2 | ALS BER(USA) CHL CLM GRL GTM HND HWA PNR PTR SUR URG USA |
|  | REG3 | AUS BRM CHN CLN GUM J(USA) KOR MHL(USA) MLA NZL PAK PNG THA VTN VUT |
| 6 742 | REG1 | BFA BLR CAF CME COG CYP(G) DJI(F) F FIN G GIB GRC KAZ LVA MDG MLI* NGR POL REU RUS SEN TCD TGO TUN TUR UKR |
|  | REG2 | ALS BER(USA) CAN CG7 CHL CUB GTM HWA JON MDW PNR PTR USA |
|  | REG3 | AUS CHN GUM HKG IND IRN J MHL(USA) NZL SNG THA VTN WAK |
| 6 745 | REG1 | ALG ASC(USA) BFA BLR CAF CME CNR COG CTI CYP(G) CZE DJI(F) E EGY F FIN G GIB GRC HNG KAZ LVA MDG MLI MLT MRC NGR POL REU RUS SEN* SEY(USA) TCD TGO TUN UKR |
|  | REG2 | ALS BER(USA) BOL CAN CG7 CHL CUB GTM HWA JON MDW PNR PTR USA |
|  | REG3 | AUS BGD CBG CHN FJI GUM HKG IND IRN J LAO MHL(USA) NZL PNG SNG THA VTN WAK |
| 6 748 | REG1 | BEL BUL CYP(G) E G GMB GRC KWT MLT POR REU RUS SDN UAE UKR ZWE |
|  | REG2 | ATG DMA GRD JMC KNA LCA VCT |
|  | REG3 | BGD BRU J TON |
| 6 751 | REG1 | ASC(USA) BFA BUL CME COG COM CTI CYP(G) D DJI E F G HNG KGZ LVA MNE MTN OMA POR RUS SEN* SRB TCD TUN UAE UKR |
|  | REG2 | B CAN CHL HWA JON MEX USA |
|  | REG3 | AUS CHN FJI GUM IND INS J J(USA) MHL(USA) NZL THA VTN |
| 6 754 | REG1 | ALG ASC(USA) BFA COG CTI D EGY ERI ETH F G GRC KGZ LVA MDG MRC NIG RUS SEN TCD TUN UAE UKR |
|  | REG2 | B BOL CAN CHL HWA JON MEX SUR USA |
|  | REG3 | AUS CBG FJI GUM IND INS J LAO MHL(USA) NZL THA VTN VUT |
| 6 757 | REGY | ATA(ARG) |
|  | REG1 | ARS AZE BLR COG F G GIB KAZ KGZ LVA MLT MWI RUS SEN SVK TCD TJK TKM TUN UKR |
|  | REG2 | ARG BER(USA) BES BOL CUW HWA JON SXM USA |
|  | REG3 | AUS BRM CHN GUM IND J MHL(USA) THA TLS |
| 6 760 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS AZE BLR COG CTI F G ISL ISR KAZ KGZ LVA MDG MRC RUS SEN SVK TCD TJK TKM TUN UKR |
|  | REG2 | ALS ARG BER(USA) BES CUW HWA JON SXM USA |
|  | REG3 | AUS BRM CHN GUM IND J J(USA) MHL(USA) MLA NZL PNG SNG* THA TLS |
| 8 965 | REG1 | AFS ASC(USA) CTI CYP(G) D EGY ERI ETH G GIB KEN NMB(AFS) RUS SMR TUR |
|  | REG2 | ALS B CAN GRL HWA MEX PNR USA |
|  | REG3 | AUS BRM FJI HKG J(USA) KRE MHL(USA) NZL PAK PNG |
| 8 968 | REG1 | AFS ARS CYP(G) D G GIB HRV KEN LBY MLT MNE NIG NMB(AFS) OMA RUS SOM SRB SVN |
|  | REG2 | ALS B BOL CAN GRL HWA MEX PNR USA |
|  | REG3 | AUS BRM CLN FJI HKG INS J(USA) MHL(USA) MLA NZL PNG SNG* |
| 8 971 | REGY | ATA(ARG) |
|  | REG1 | ARS AZE AZR BHR(USA) BLR E F G GEO GRC(USA) HOL HRV I I(USA) ISL ISR KAZ KGZ LVA MRC RUS S TJK TKM UKR ZMB |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BES BOL BRB(USA) CG7 CUW DOM HWA MDW PNR PTR SXM TCA(USA) TRD(USA) USA |
|  | REG3 | AUS BRM CHN DGA(USA) GUM J(USA) MHL(USA) PNG VTN |
| 8 974 | REGY | ATA(ARG) |
|  | REG1 | AFS AZE AZR BLR E GEO GNE GRC(USA) HOL I I(USA) IRL ISL ISR KAZ KGZ LVA MRC RUS TJK TKM UKR YEM |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BES BRB(USA) CG7 CUW DOM HWA MDW PNR PTR SXM TCA(USA) USA |
|  | REG3 | AUS BRM CHN GUM J(USA) MHL(USA) NZL PNG VTN |
| 8 977 | REG1 | ALB ARS BHR(USA) G GRC(USA) I ISL MRC MWI OMA RUS UKR |
|  | REG2 | ALS BRB(USA) HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CBG CLN DGA(USA) GUM INS J(USA) LAO |
| 8 980 | REGY | ATA(ARG) |
|  | REG1 | ALB ALG ARS AZR BFA BHR(USA) CME COG CYP(G) D DJI(F) F G I KAZ LBN MDG REU RUS SEN TCD TGO TUN UZB |
|  | REG2 | ALS ARG ATG(USA) BAH(USA) BER(USA) BRB BRB(USA) CG7 HWA MDW PNR PTR TCA(USA) USA |
|  | REG3 | AUS CHN GUM HKG IND INS J(USA) MHL(USA) |
| 8 983 | REGY | ATA(ARG) |
|  | REG1 | ALG BFA BHR(USA) CME COG CYP(G) D DJI(F) F G HNG I KAZ LBN MDG MLT MNG MRC MTN OMA REU RUS SEN* TCD TGO TUN UZB |
|  | REG2 | ALS ARG BER(USA) BRB(USA) CG7 GRL HWA MDW PNR PTR USA |
|  | REG3 | AUS CBG CHN GUM IND J(USA) LAO MHL(USA) NZL PNG VTN |
| 8 986 | REG1 | ALG BHR(USA) CYP(G) F G GRC KGZ MDG MLT ROU RUS TUR UKR YEM |
|  | REG2 | BRB(USA) CG7 |
|  | REG3 | J J(USA) PHL TUV |
| 8 989 | REG1 | AGL BEL BLR G KAZ KGZ LVA MCO MDA MOZ POL POR ROU RUS STP UKR UZB YEM |
|  | REG2 | ALS BER(USA) CAN GRL HWA MEX USA |
|  | REG3 | AUS BRM FJI IND J J(USA) NZL |
| 8 992 | REG1 | AGL ASC(USA) BLR CPV F G GNB GRC ISL KAZ LVA MDA MOZ POL POR RUS S STP SSD UKR UZB |
|  | REG2 | ALS BER(USA) CAN CHL HWA MEX USA |
|  | REG3 | AUS BRM CHN FJI GUM IND J(USA) NZL PNG |
| 8 995 | REG1 | ARS AZR COM CYP(G) G GIB GRC ISL LBR MLT MNG RUS UKR YEM |
|  | REG2 | BLZ |
|  | REG3 | BRU HKG TON |
| 8 998 | REGY | ATA(USA) |
|  | REG1 | AGL AZR BHR(USA) BLR COG F G GRC(USA) HOL ISL LVA MDG MTN NOR SEN* TUN UAE UKR |
|  | REG2 | ALS B BER(USA) CG7 CUB HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CHN GUM IND J(USA) MHL(USA) NZL |
| 9 001 | REGY | ATA(USA) |
|  | REG1 | AGL ALG ARM BHR(USA) BLR COG CTI CYP(G) EGY F G GRC(USA) HOL I(USA) ISL JOR LVA MDG MLT MRC MTN NOR SEN* TUN UKR |
|  | REG2 | ALS B BER(USA) CG7 CUB HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CHN DGA(USA) GUM HKG IND J(USA) MHL(USA) NZL |
| 9 004 | REG1 | ARM BDI BEN BLR CYP(G) IRL ISL KWT LSO LUX MLT ROU |
|  | REG2 | B BAH |
|  | REG3 | HKG IRN J MLD NRU |
| 9 007 | REG1 | AZR BUL CME COG G GIB GRC GRC(USA) I(USA) ISL KAZ MDG MLT MNE REU ROU RUS SEN SRB TCD |
|  | REG2 | ALS B CAN HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS BRM CHN FJI GUM INS IRN J KIR VTN WAK |
| 9 010 | REG1 | ARS AZR BEL BUL CME COG CTI G KAZ LIE MDG REU RUS SEN TCD TUR |
|  | REG2 | ALS ARG B CAN HWA MDW MEX PNR PTR USA VEN |
|  | REG3 | AUS BRM FJI GUM INS IRN J KIR NZL PAK VTN WAK |
| 9 013 | REG1 | AFS ARS ERI ETH G GMB GRC HRV MLT MOZ RUS UKR |
|  | REG2 | ARG ATG DMA GRD GTM JMC KNA LCA VCT |
|  | REG3 | AUS FJI IND J |
| 9 016 | REG1 | AUT COG F G GIB HNG MDG RUS SEN TCD TUN TUR UKR |
|  | REG2 | BER(USA) CHL CUB |
|  | REG3 | AUS CHN FJI HKG IRN J(USA) NZL PAK SNG THA |
| 9 019 | REG1 | ALG AUT CNR COG CTI E F G GIB GRC MDG MLT MRC NIG RUS SEN TCD TUN UKR |
|  | REG2 | ALS BER(USA) BOL CHL CUB HWA |
|  | REG3 | AUS CHN IRN J MLA* NZL PAK PNG SNG THA VUT |
| 9 022 | REGY | ATA(ARG) |
|  | REG1 | AFS ALG ARM AZE AZR COG CYP(G) CZE D(USA) EGY ERI ETH F G GEO KAZ MDG MLT REU RUS SEN SOM TJK TKM UZB |
|  | REG2 | ARG BER(USA) CAN GRL HWA JON PNR PTR USA |
|  | REG3 | AUS CHN GUM HKG IND J MHL(USA) NZL |
| 9 025 | REGY | ATA(ARG) ATA(NZL) |
|  | REG1 | AFS ALG ARM AZE AZR COG CYP(G) CZE D D(USA) E EGY G GEO GIB KAZ MDG MLT REU ROU RUS SEN TJK TKM UZB |
|  | REG2 | ARG BER(USA) CUB HWA JON MEX PNR PTR USA |
|  | REG3 | AUS CHN FJI GUM HKG IND J(USA) MHL(USA) NZL PAK PHL(USA) PNG SNG THA |
| 9 028 | REG1 | COD E G G(USA) GIB GRC MLT MRC QAT ROU RUS UAE UZB |
|  | REG2 | ALS CAN CG7 CUB GRL HWA MEX USA |
|  | REG3 | AUS J MLA SMO |
| 9 031 | REGY | ATA(USA) |
|  | REG1 | CYP(G) G G(USA) GIB GRC(USA) I I(USA) MLT MRC POL RUS SVK SWZ TUR |
|  | REG2 | ALS BER(USA) CAN CHL CLM HWA MDW PNR PTR URG USA |
|  | REG3 | AUS BGD BRM CHN GUM J MHL(USA) MLA NZL PAK TLS WAK |
| 9 034 | REGY | ATA(USA) |
|  | REG1 | AUT DNK G G(USA) GHA GRC(USA) I I(USA) MRC NIG POL RUS SEY TUR YEM |
|  | REG2 | ALS BER(USA) CHL CLM EQA HWA MDW PNR PTR URG USA |
|  | REG3 | BGD BRM CHN GUM INS J MHL(USA) MLA NZL PAK SMO TLS WAK |
| 9 037 | REGY | ATA(USA) |
|  | REG1 | AUT CYP DJI G I I(USA) LTU MRC NMB RUS SRL TUR UAE |
|  | REG2 | ALS CAN HWA MDW PNR PTR USA |
|  | REG3 | AUS DGA(USA) GUM J(USA) MHL(USA) NPL WAK |
| 11 175 | REG1 | ASC(USA) G GRC MLT SDN TUR(USA) UAE |
|  | REG2 | ALS HWA USA |
|  | REG3 | AUS GUM J(USA) |
| 11 178 | REGY | ATA(ARG) |
|  | REG1 | AGL G GRC MOZ NIG NOR POL POR RUS STP TUN TUR(USA) |
|  | REG2 | ALS ARG BES CLM CUW HWA JON SXM USA |
|  | REG3 | AUS CHN GUM IND INS J J(USA) MHL(USA) NZL |
| 11 181 | REGY | ATA(ARG) |
|  | REG1 | AGL AZR CPV E EGY G GNB ISL MOZ NOR POL POR RUS STP TUR TUR(USA) |
|  | REG2 | ALS ARG BES CLM CUW JON SXM USA |
|  | REG3 | AUS CHN GUM IND INS J(USA) MHL(USA) NZL |
| 11 184 | REG1 | CYP(G) E G GNE ISL MKD MLT MNG ROU TUR |
|  | REG2 | BLZ |
|  | REG3 | J MLD TON |
| 11 187 | REGY | ATA(USA) |
|  | REG1 | ALG BEL BHR(USA) BLR CME COG DJI(F) ERI ETH F GEO GRC(USA) ISL ISR KAZ LVA MDG ROU RUS SEN TCD TJK TKM UKR UZB |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) CAN CHL HWA MDW MEX PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS CHN DGA(USA) GUM IRN J(USA) MHL(USA) |
| 11 190 | REGY | ATA(USA) |
|  | REG1 | ALG BHR(USA) BLR CME COG DJI(F) GEO GRC ISR KAZ LVA MDG MRC ROU RUS SEN TCD TJK TKM UKR UZB |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) CAN CHL HWA MDW MEX PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS BRM CHN DGA(USA) GUM INS IRN J(USA) MHL(USA) NZL |
| 11 193 | REG1 | CYP(G) G GRC MNG NIG RUS |
|  | REG2 | MEX URG |
|  | REG3 | IND PHL TUV |
| 11 196 | REG1 | ARS BHR(USA) CYP(G) D G KEN RUS |
|  | REG2 | ALS ATG(USA) B BAH(USA) BER(USA) BRB(USA) CG7 HWA MDW PNR PTR TCA(USA) TRD(USA) URG USA |
|  | REG3 | AUS CHN GUM HKG J(USA) MHL(USA) WAK |
| 11 199 | REG1 | ARS BHR(USA) CYP(G) D EGY G GIB I(USA) KEN LBY MLT MRC OMA RUS SOM |
|  | REG2 | ALS ATG(USA) B BAH(USA) BER(USA) BRB(USA) CG7 HWA MDW PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS CHN CLN GUM HKG IRN J(USA) MLA PNG SNG* WAK |
| 11 202 | REG1 | BHR(USA) CYP IRL SMR TUN YEM |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) CG7 HWA MDW PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS GUM J(USA) WAK |
| 11 205 | REGY | ATA(ARG) |
|  | REG1 | AZR CME COG DJI(F) F G KAZ MDG MNG REU RUS SEN TGO TUN |
|  | REG2 | ALS ARG CAN CUB HWA JON MDW PNR PTR USA |
|  | REG3 | AUS GUM J WAK |
| 11 208 | REGY | ATA(ARG) |
|  | REG1 | ALG AZR CME COG CYP(G) DJI(F) F G GIB GRC(USA) HNG KAZ LBY MDG MRC REU RUS SEN TGO TUN TUR |
|  | REG2 | ALS ARG CAN CUB HWA JON MDW PNR PTR USA |
|  | REG3 | AUS CBG GUM IRN J LAO PNG VTN WAK |
| 11 211 | REG1 | BEL E G OMA RUS SWZ TUN |
|  | REG2 | ALS HWA JON MDW PNR PTR |
|  | REG3 | GUM IRN J MHL(USA) WAK |
| 11 214 | REGY | ATA(ARG) |
|  | REG1 | AUT COG DJI(F) F G GAB GIB ISL MDG MLT REU RUS SEN TCD TUN |
|  | REG2 | ALS ARG BER(USA) CAN HWA MRT USA |
|  | REG3 | AUS BRU NCL NPL OCE |
| 11 217 | REGY | ATA(ARG) |
|  | REG1 | ASC(USA) AUT COG D DJI(F) F G GRC MDG MRC RUS SEN SEY(USA) TCD TUN |
|  | REG2 | ALS ARG BER(USA) CAN GRL HWA MRT USA |
|  | REG3 | AUS CHN NCL NZL OCE |
| 11 220 | REG1 | BDI BEL GMB KWT ROU RUS |
|  | REG2 | CAN USA |
|  | REG3 | AUS CBG CHN J LAO VTN VUT |
| 11 223 | REG1 | BEN G MLT ROU S UKR YEM |
|  | REG2 | ALS ATG CAN DMA GRD JMC KNA LCA VCT |
|  | REG3 | AUS IRN J KRE |
| 11 226 | REG1 | ARS(USA) AZR D D(USA) G MNE RUS SRB SRL TUR(USA) UKR |
|  | REG2 | ALS BER(USA) CHL CUB GRL HWA JON MDW PNR USA |
|  | REG3 | AUS BGD CHN GUM J(USA) MHL(USA) NZL PAK PHL(USA) |
| 11 229 | REG1 | ARS(USA) AZR D D(USA) G MNE MRC RUS SRB TUR(USA) |
|  | REG2 | ALS BER(USA) CAN CG7 CUB GRL HWA JON MDW PNR USA |
|  | REG3 | AUS BGD CHN GUM J MHL(USA) NZL PAK |
| 11 232 | REG1 | HOL IRL LIE NIG QAT RUS UAE YEM |
|  | REG2 | BAH CAN |
|  | REG3 | AUS J SNG |
| 11 235 | REG1 | AFS ARM AZE BLR CYP(G) D F G KAZ KGZ LVA MNG RUS SEN TJK TKM TUN UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN GRL HWA MEX USA |
|  | REG3 | AUS BRM GUM J PNG SNG |
| 11 238 | REG1 | ALG ARM AZE BLR D KAZ KGZ LSO LVA MRC RUS SEN TJK TKM TUN UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN HWA MEX |
|  | REG3 | AUS CHN IRN J J(USA) NZL |
| 11 241 | REG1 | CYP(G) DJI G GIB LBR MLT RUS TUR(USA) |
|  | REG2 | USA |
|  | REG3 | CHN HKG NRU |
| 11 244 | REG1 | ALG COM CYP(G) DNK G G(USA) GIB KAZ MNG RUS TUR(USA) UZB |
|  | REG2 | B BER(USA) CAN USA |
|  | REG3 | AUS FJI IRN J(USA) NZL PNG |
| 11 247 | REG1 | ALG CYP(G) EGY G GIB KAZ LBY MLT RUS UZB ZMB |
|  | REG2 | B BER(USA) CAN HWA MEX |
|  | REG3 | AUS CHN CLN FJI GUM HKG J(USA) MLA NZL |
| 11 250 | REG1 | ALG F G GIB GUI I NIG RUS SEY TUR |
|  | REG2 | CAN |
|  | REG3 | AUS CHN |
| 11 253 | REGY | ATA(USA) |
|  | REG1 | AZE AZR BHR(USA) BLR ERI ETH F G GRC(USA) I I(USA) KAZ MOZ MRC RUS TJK TKM UKR UZB |
|  | REG2 | ALS B BER(USA) BRB(USA) CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | CHN GUM J(USA) MHL(USA) |
| 11 256 | REGY | ATA(USA) |
|  | REG1 | AZE BHR(USA) BLR ERI ETH G GRC(USA) HOL I I(USA) ISL KAZ MRC RUS TJK TKM UKR UZB |
|  | REG2 | ALS B BRB(USA) CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS BRM CHN FJI GUM INS IRN J(USA) |
| 11 259 | REGY | ATA(USA) |
|  | REG1 | AZR BHR(USA) CYP(G) G ISL MLT MWI UAE UKR |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) CG7 HWA MDW PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | GUM J(USA) SMO |
| 11 262 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | CZE D E G GRC(USA) I I(USA) ISL KAZ LTU MDA MRC RUS TUR UKR |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN DGA(USA) GUM IND J(USA) MHL(USA) |
| 11 265 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AZR BEL CZE D EGY GRC(USA) I I(USA) ISL KAZ LTU LVA MDA MNG MRC OMA POR RUS UKR UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA MDW PNR PTR USA |
|  | REG3 | CHN GUM IND J(USA) MHL(USA) |
| 11 268 | REGY | ATA(USA) |
|  | REG1 | ALG ARS BEL COG G ISL KAZ LVA MDG MLT REU RUS SEN SVN UZB |
|  | REG2 | ALS BER(USA) HWA MDW PNR PTR USA |
|  | REG3 | AUS GUM IRN J(USA) MHL(USA) |
| 11 271 | REG1 | ALG ARS AZE BLR BUL COG F G GEO KAZ MDA MDG MLT MRC REU ROU RUS SEN TJK UKR UZB |
|  | REG2 | B CAN MEX |
|  | REG3 | AUS J(USA) |
| 13 200 | REG1 | AFS ALG BEL CYP G GMB RUS UAE YEM |
|  | REG2 | ALS GRL HWA USA |
|  | REG3 | AUS J(USA) KRE NPL |
| 13 203 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS CYP(G) D EGY G GIB KEN NIG ROU RUS SVN TUR TUR(USA) UZB |
|  | REG2 | ALS ARG BES CUW HWA JON MEX SXM USA |
|  | REG3 | AUS HKG IRN J J(USA) PNG |
| 13 206 | REGY | ATA(ARG) |
|  | REG1 | ALG ARS CYP(G) D E G GIB ISL KEN LBY MLT ROU RUS SOM SUI TUR TUR(USA) UZB |
|  | REG2 | ALS ARG BES CUW GRL HWA JON MEX SXM USA |
|  | REG3 | AUS CLN HKG IRN J MLA NZL SNG* |
| 13 209 | REG1 | CYP(G) G GIB LIE LSO MLT MNG RUS SDN |
|  | REG2 | BAH |
|  | REG3 | HKG J MLD SMO |
| 13 212 | REGY | ATA(ARG) |
|  | REG1 | ARS(USA) AZR CAF CME COG CZE D(USA) ERI ETH GRC IRL MDG RUS SEN TUR(USA) |
|  | REG2 | ALS ARG BER(USA) CAN CUB GRL HWA JON PNR PTR USA |
|  | REG3 | AUS BGD CHN GUM J J(USA) MHL(USA) NZL PAK |
| 13 215 | REGY | ATA(ARG) |
|  | REG1 | ARS(USA) AZR CAF CME COG CZE D(USA) E EGY F G MDG MRC OMA RUS SEN TUR(USA) |
|  | REG2 | ALS ARG BER(USA) CAN CG7 CUB GRL HWA JON MEX PNR PTR USA |
|  | REG3 | AUS BGD CHN GUM IRN J(USA) MHL(USA) NZL PAK |
| 13 218 | REG1 | CYP(G) DJI G KAZ LBR MLT MWI RUS SMR |
|  | REG2 | ALS CAN HWA MDW MEX URG USA |
|  | REG3 | AUS HKG J MHL(USA) |
| 13 221 | REG1 | ALG AZE BLR CME COG D DJI(F) GEO GRC(USA) KAZ KGZ LVA MDG MLI REU RUS SEN* TCD TGO TJK TKM TUN UKR UZB |
|  | REG2 | ALS B CAN HWA MDW PNR PTR URG USA |
|  | REG3 | AUS CHN FJI GUM J(USA) KIR MHL(USA) NZL |
| 13 224 | REG1 | ALG ASC(USA) AZE BLR CME COG CTI D DJI(F) F G GEO HNG JOR KAZ KGZ LVA MDG MLI MNG REU RUS S SEN* SEY(USA) TCD TGO TJK TKM TUN UKR UZB |
|  | REG2 | ALS B CAN CUB HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN FJI GUM IRN J(USA) KIR MHL(USA) NZL PNG |
| 13 227 | REG1 | BEL COM GNE IRL KAZ MRC QAT RUS TUR |
|  | REG2 | ALS CAN CUB HWA MDW PNR PTR USA |
|  | REG3 | AUS CBG GUM HKG J(USA) LAO VTN |
| 13 230 | REG1 | G GRC KAZ LTU MLT RUS SRL UAE YEM ZMB |
|  | REG2 | ALS CAN CG7 HWA MDW PNR PTR USA |
|  | REG3 | GUM J(USA) MHL(USA) PHL TON |
| 13 233 | REGY | ATA(ARG) |
|  | REG1 | AUT AZR CME COG D D(F) DJI(F) E F ISL KAZ MDG MLI MNG REU RUS SEN* TCD TGO TJK TKM TUN UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA MDW MRT PNR PTR USA |
|  | REG3 | CHN GUM J(USA) MHL(USA) NCL OCE |
| 13 236 | REGY | ATA(ARG) |
|  | REG1 | AUT AZR CME COG CTI D D(F) DJI(F) F G GRC(USA) I(USA) KAZ MDG MLI MRC NIG REU RUS SEN* TCD TGO TJK TKM TUN UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 GRL HWA MDW MRT PNR PTR USA |
|  | REG3 | AUS CBG CHN GUM J(USA) LAO MHL(USA) NCL NZL OCE VTN VUT |
| 13 239 | REG1 | AZR BEN G HOL KAZ KWT LUX NMB ROU RUS |
|  | REG2 | ATG DMA GRD JMC KNA LCA VCT |
|  | REG3 | BRU IRN J NRU |
| 13 242 | REG1 | ALG ARM AZE BLR CAF CME COG F G G(USA) GEO KAZ MDG POL REU ROU RUS SEN TJK TKM TUN UKR UZB |
|  | REG2 | B BER(USA) HWA JON USA |
|  | REG3 | AUS CHN FJI GUM J(USA) MHL(USA) NZL OCE |
| 13 245 | REG1 | ALG ARM ASC(USA) AZE BLR CAF CME COG E F G GEO GRC ISR KAZ MDG MNG POL REU RUS SEN TJK TKM TUN UKR UZB |
|  | REG2 | B BER(USA) CAN HWA JON USA |
|  | REG3 | AUS BRM CHN FJI GUM J J(USA) MHL(USA) NZL OCE VTN |
| 13 248 | REG1 | ALG BLR COD CYP(G) G G(USA) MLT MNE RUS SRB UKR |
|  | REG2 | USA |
|  | REG3 | AUS HKG J SNG TUV |
| 13 251 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AGL ALB AZR BHR(USA) BLR CYP(G) F GRC(USA) I I(USA) MOZ MRC NOR POR RUS STP UKR |
|  | REG2 | ALS ARG CAN CG7 HWA JON MDW MEX PNR PTR USA |
|  | REG3 | AUS CHN GUM IND IRN J(USA) NZL WAK |
| 13 254 | REGY | ATA(ARG) ATA(USA) |
|  | REG1 | AGL AZR BHR(USA) GRC(USA) HOL I I(USA) MNG MOZ MRC NOR POR RUS STP UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 HWA JON MDW MEX PNR PTR USA |
|  | REG3 | AUS BRM CHN GUM IND J(USA) NZL WAK |
| 13 257 | REGY | ATA(USA) |
|  | REG1 | BEL BHR(USA) CPV G GNB HRV MRC ROU SWZ UZB |
|  | REG2 | CAN CG7 HWA JON MDW PTR USA |
|  | REG3 | AUS GUM INS J(USA) MHL(USA) WAK |
| 15 010 | REG1 | BEL BEN DJI IRL MLT RUS |
|  | REG2 | BLZ CAN HWA |
|  | REG3 | AUS GUM KRE NPL |
| 15 013 | REGY | ATA(ARG) |
|  | REG1 | D(USA) G GRC MLT NIG RUS TUR(USA) UZB |
|  | REG2 | ALS ARG BER(USA) CUB GRL HWA JON PNR USA |
|  | REG3 | GUM J J(USA) MHL(USA) |
| 15 016 | REGY | ATA(ARG) |
|  | REG1 | ASC(USA) CNR D(USA) E G MRC ROU RUS TUR(USA) UZB |
|  | REG2 | ALS ARG BER(USA) CAN CG7 CUB GRL HWA JON PNR PRU USA |
|  | REG3 | AUS CHN GUM IRN J(USA) MHL(USA) NZL PHL(USA) |
| 15 019 | REG1 | ARS F LBR MLT ROU RUS UKR |
|  | REG2 | ALS CAN GRL URG USA |
|  | REG3 | AUS J |
| 15 022 | REGY | ATA(USA) |
|  | REG1 | AGL ALB ARS BHR(USA) BLR GEO ISL KAZ LVA MDA MOZ MRC POR RUS S STP TJK TUR UKR UZB |
|  | REG2 | ALS BRB(USA) CAN HWA MDW PNR PTR TRD(USA) URG USA |
|  | REG3 | AUS CHN DGA(USA) GUM IND IRN J(USA) MAC TLS WAK |
| 15 025 | REGY | ATA(USA) |
|  | REG1 | AGL ARS AZR BHR(USA) BLR CPV G GEO GNB ISL KAZ LVA MDA MLT MOZ MRC OMA POR RUS STP TJK TUR UKR UZB |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) CHL HWA MDW MEX PNR PTR TCA(USA) TRD(USA) USA |
|  | REG3 | AUS FJI GUM IND J(USA) MAC NZL TLS WAK |
| 15 028 | REGY | ATA(USA) |
|  | REG1 | ALG BHR(USA) GRC(USA) ISL MLT RUS TJK |
|  | REG2 | ALS BRB(USA) HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS GUM J(USA) WAK |
| 15 031 | REG1 | ALG COM CYP(G) G MLT RUS TJK |
|  | REG2 | ATG CAN DMA GRD JMC KNA LCA VCT |
|  | REG3 | AUS J J(USA) |
| 15 034 | REG1 | ALG ARS(USA) AZE AZR BLR CME COG D(USA) DJI(F) F G GEO GRC ISR KAZ LTU MDA MDG MLI REU RUS SEN* TCD TJK TKM TUR(USA) UKR UZB |
|  | REG2 | B CAN GRL HWA USA |
|  | REG3 | AUS GUM IRN NZL PHL |
| 15 037 | REG1 | ALG ARS(USA) AZE AZR BLR CME COG CTI D(USA) G GEO KAZ LTU MDA MDG MLI MNE MRC REU RUS SEN* SRB TCD TJK TKM TUR(USA) UKR UZB |
|  | REG2 | ALS B CAN HWA MEX USA |
|  | REG3 | AUS J(USA) |
| 15 040 | REG1 | CYP(G) G GUI LIE QAT RUS |
|  | REG2 | USA |
|  | REG3 | AUS J MLD NRU |
| 15 043 | REGY | ATA(ARG) |
|  | REG1 | CYP(G) DNK ERI ETH G GMB KAZ |
|  | REG2 | ALS ARG CUB |
|  | REG3 | AUS BGD FJI IRN J(USA) PAK |
| 15 046 | REGY | ATA(ARG) |
|  | REG1 | CYP(G) E ERI ETH G ISL KAZ MLT MNE RUS SRB SUI |
|  | REG2 | ALS ARG CUB USA |
|  | REG3 | AUS BGD FJI J NZL PAK PNG |
| 15 049 | REG1 | COD CYP(G) G GIB RUS SMR UAE |
|  | REG2 | USA |
|  | REG3 | AUS HKG J TUV |
| 15 052 | REGY | ATA(ARG) |
|  | REG1 | BHR(USA) G GRC(USA) I I(USA) MRC NOR RUS |
|  | REG2 | ALS ARG BER(USA) HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | CHN GUM IND J(USA) MHL(USA) NZL VTN |
| 15 055 | REGY | ATA(ARG) |
|  | REG1 | AFS ALG ARM BHR(USA) G G(USA) GRC(USA) I I(USA) ISL MRC NOR RUS |
|  | REG2 | ALS ARG BER(USA) HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CHN GUM IND J(USA) MHL(USA) NZL VTN |
| 15 058 | REG1 | ALG ARM BHR(USA) G GRC(USA) I(USA) RUS SWZ |
|  | REG2 | ALS HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS GUM J(USA) MHL(USA) |
| 15 061 | REG1 | ALG CNR E F G GRC LSO RUS UZB |
|  | REG2 | ALS BRB(USA) CG7 HWA MDW PNR PTR USA |
|  | REG3 | AUS GUM J(USA) MHL(USA) |
| 15 064 | REG1 | AZR CME COG DJI(F) F G GRC ISL KAZ KGZ MDG MLI* MTN REU RUS SEN* TCD TGO TJK TKM TUN UZB |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB BRB(USA) CG7 CHL HWA MDW PNR PTR TCA(USA) USA |
|  | REG3 | AUS DGA(USA) GUM J(USA) PNG |
| 15 067 | REG1 | ALG AZR CME COG CTI DJI(F) F KAZ KGZ MDG MLI* MRC REU RUS SEN TCD TGO TJK TKM TUN UZB |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB BRB(USA) CG7 HWA MDW PNR PTR TCA(USA) USA |
|  | REG3 | AUS CBG GUM J(USA) LAO VTN |
| 15 070 | REG1 | BEL BHR(USA) GEO RUS SRL TUR |
|  | REG2 | ALS HWA JON MDW PNR PTR USA |
|  | REG3 | AUS GUM J WAK |
| 15 073 | REGY | ATA(ARG) |
|  | REG1 | BHR(USA) COG D DJI(F) E F GEO GRC(USA) ISL MDG MNG RUS SEN TUN UKR |
|  | REG2 | ALS ARG BER(USA) CAN HWA JON MDW PNR PTR USA |
|  | REG3 | AUS CHN GUM IND J MHL(USA) NCL OCE WAK |
| 15 076 | REGY | ATA(ARG) |
|  | REG1 | AUT BHR(USA) COG CTI D DJI(F) F G MDG MRC RUS SEN TUN UKR |
|  | REG2 | ALS ARG BER(USA) HWA JON MDW PNR PTR USA |
|  | REG3 | AUS CBG CHN GUM IND IRN J LAO MHL(USA) NCL NZL OCE VTN VUT WAK |
| 15 079 | REG1 | BDI E G GRC KWT ROU RUS TKM |
|  | REG2 | PTR USA |
|  | REG3 | BRU J TON |
| 15 082 | REG1 | AZE BHR(USA) BLR CNR E GRC(USA) I I(USA) KAZ KGZ LVA MRC POL ROU RUS TJK TKM UKR |
|  | REG2 | ALS B BER(USA) BRB(USA) HWA MDW MEX PNR PTR USA |
|  | REG3 | AUS FJI GUM J(USA) KIR NZL |
| 15 085 | REG1 | AZE BHR(USA) BLR CNR DNK E G GRC(USA) HOL I I(USA) KAZ KGZ LVA MNG MRC NIG POL RUS TJK TKM UKR |
|  | REG2 | ALS B BER(USA) BRB(USA) HWA MDW MEX PNR PTR TRD(USA) USA |
|  | REG3 | AUS CHN FJI GUM J(USA) KIR MHL(USA) NZL PNG |
| 15 088 | REG1 | BEL BHR(USA) BLR E RUS UAE |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB(USA) HWA MDW PNR PTR TCA(USA) USA |
|  | REG3 | AUS GUM HKG J(USA) |
| 15 091 | REG1 | E G HRV MLT RUS ZMB |
|  | REG2 | B MEX USA |
|  | REG3 | AUS HKG IRN J J(USA) |
| 15 094 | REGY | ATA(ARG) |
|  | REG1 | E HOL MLT MNG MWI RUS TUR |
|  | REG2 | ALS ARG BER(USA) BES CUW GTM HWA SXM USA |
|  | REG3 | AUS CHN GUM J |
| 15 097 | REG1 | CYP IRL RUS SDN TUR |
|  | REG2 | ALS ARG BAH BER(USA) |
|  | REG3 | INS J SMO |
| 17 970 | REG1 | AFS ALG CYP DJI G KWT MCO RUS |
|  | REG2 | ATG DMA GRD JMC KNA LCA VCT |
|  | REG3 | BRU PHL SMO |
| 17 973 | REGY | ATA(ARG) |
|  | REG1 | AGL ALG ARM ARS(USA) AZE AZR BLR CYP(G) D F G I KAZ LTU LVA MDA MNG MOZ NIG POR ROU RUS STP SVN TJK TKM UKR UZB |
|  | REG2 | ALS ARG BER(USA) GRL HWA JON USA |
|  | REG3 | AUS GUM IND IRN J(USA) MAC MHL(USA) TLS |
| 17 976 | REG1 | CPV D G G(USA) I MNE MRC ROU RUS SRB SWZ TUR(USA) UAE UZB |
|  | REG2 | CAN GRL URG USA |
|  | REG3 | AUS J(USA) MLD |
| 17 979 | REG1 | BHR(USA) CYP(G) E G GIB GRC(USA) I I(USA) LSO MRC RUS UZB |
|  | REG2 | ALS B BER(USA) CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS BGD GUM HKG J(USA) NZL PAK |
| 17 982 | REG1 | ARS AZR BHR(USA) CYP(G) EGY G GIB GRC(USA) I I(USA) ISL JOR KEN MLT MRC OMA RUS S UKR |
|  | REG2 | ALS B BER(USA) CAN CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS BGD GUM HKG IRN J(USA) MHL(USA) NZL PAK PNG |
| 17 985 | REG1 | BEN BHR(USA) D G ISL LBY MNG SOM UKR |
|  | REG2 | ALS BER(USA) CG7 HWA MDW PNR PTR TRD(USA) USA |
|  | REG3 | AUS CLN GUM J(USA) MLA SNG |
| 17 988 | REG1 | CYP(G) G GIB LIE MLT NIG RUS TUN |
|  | REG2 | BAH |
|  | REG3 | AUS HKG IND J |
| 17 991 | REGY | ATA(ARG) |
|  | REG1 | AFS CME COG D D(F) DJI(F) F GAB GRC HOL ISL MDG MLI* MTN* REU RUS SEN TCD TGO TUN |
|  | REG2 | ALS ARG BER(USA) GRL HWA JON MRT USA |
|  | REG3 | AUS CHN FJI GUM J NCL NZL OCE |
| 17 994 | REGY | ATA(ARG) |
|  | REG1 | ALG AUT CME COG CTI D D(F) DJI(F) F ISR MDG MLI MNG MRC REU RUS SEN* TCD TGO TKM TUN UKR |
|  | REG2 | ALS ARG CAN GRL HWA JON MRT USA |
|  | REG3 | AUS CBG CHN FJI GUM IRN J LAO NCL NZL OCE VTN VUT |
| 17 997 | REG1 | ALG CYP(G) G GIB LUX MLT MWI RUS TKM UKR |
|  | REG3 | HKG J TON |
| 18 000 | REGY | ATA(ARG) |
|  | REG1 | ALG BLR G GEO GRC KAZ LVA POL RUS TJK TUR UKR UZB ZMB |
|  | REG2 | ARG CAN MEX USA |
|  | REG3 | AUS BGD J J(USA) NZL PAK |
| 18 003 | REGY | ATA(ARG) |
|  | REG1 | ALG BLR COM CYP(G) G GEO KAZ LVA MLT MNG POL RUS TJK TUR UAE UKR UZB |
|  | REG2 | ALS ARG MEX USA |
|  | REG3 | AUS J(USA) NZL PNG |
| 18 006 | REG1 | BEL G HOL LBR MLT RUS SMR |
|  | REG2 | BLZ |
|  | REG3 | AUS IRN J(USA) |
| 18 009 | REGY | ATA(USA) |
|  | REG1 | BHR(USA) CME COG CYP(G) D DJI(F) E F G GRC(USA) I I(USA) ISL MDG MLI MLT MRC REU ROU RUS SEN* TCD TGO TUN |
|  | REG2 | ALS ATG(USA) BAH(USA) BER(USA) BRB BRB(USA) CAN CG7 HWA MDW PNR PTR TCA(USA) USA |
|  | REG3 | AUS CHN FJI GUM J MHL(USA) NZL |
| 18 012 | REGY | ATA(USA) |
|  | REG1 | BHR(USA) CME COG CTI D DJI(F) E F G GRC(USA) I I(USA) MDG MLI* MRC MTN REU ROU RUS SEN* TCD TGO TUN |
|  | REG2 | ALS BER(USA) BRB(USA) CAN CG7 CHL HWA MDW PNR PTR USA |
|  | REG3 | CHN FJI GUM J(USA) MHL(USA) NZL |
| 18 015 | REGY | ATA(USA) |
|  | REG1 | ALG BHR(USA) CNR E F G GRC(USA) I(USA) MNG MRC RUS UKR |
|  | REG2 | ALS BRB(USA) CAN CG7 GRL HWA MDW PNR PTR USA |
|  | REG3 | AUS CHN GUM HKG J(USA) |
| 18 018 | REG1 | ASC(USA) E G G(USA) HRV RUS SRL UKR |
|  | REG2 | CAN |
|  | REG3 | AUS HKG IRN J(USA) |
| 18 021 | REG1 | AZE BEL BLR E G GEO GHA GRC KAZ KGZ LVA OMA RUS TJK TKM UKR |
|  | REG2 | B BER(USA) USA |
|  | REG3 | GUM J TUV |
| 18 024 | REG1 | AZE BLR E G GEO KAZ KGZ LVA MNG MOZ POR RUS S SUI TJK TKM TUR UKR |
|  | REG2 | B BER(USA) CAN GRL USA |
|  | REG3 | AUS FJI INS J(USA) |
| 18 027 | REG1 | BEL G GMB NMB QAT RUS SSD TUR |
|  | REG2 | CAN USA |
|  | REG3 | AUS KRE NPL NRU |

NOTES 
ON THE CONCLUDED OPERATIONAL AGREEMENTS

1	The Administrations of Canada and the United States of America informed the Radiocommmunication Bureau that they had concluded an operational agreement. The agreement constitutes a sharing arrangement between the two countries for use of all shared allotments appearing in the present version of Part III of this Appendix.

2	The Administrations of Mali, Mauritania and Senegal concluded an operational agreement whose terms of reference are given as follows:

2.1	the use of the following allotments by Mali is subject to coordination with the administrations of Mauritania and Senegal: 3 044, 3 047, 3 143, 3 149, 3 152, 3 900, 4 745, 5 702, 6 712, 6 742, 15 064, 15 067, 17 991 and 18 012 kHz;

2.2	the use of the following allotments by Mauritania is subject to coordination with the administrations of Mali and Senegal: 3 038, 5 708, 5 711, 6 715 and 17 991 kHz;

2.3	the use of the following allotments by Senegal is subject to coordination with the administrations of Mali and Mauritania: 3 044, 3 047, 3 050, 3 053, 3 056, 3 059, 3 140, 3 149, 3 903, 4 736, 4 739, 4 742, 5 702, 5 717, 5 723, 5 726, 6 712, 6 715, 6 745, 6 751, 8 983, 8 998, 9 001, 13 221, 13 224, 13 233, 13 236, 15 034, 15 037, 15 064, 17 994, 18 009 and 18 012 kHz.

3	The Administrations of Brunei Darussalam, Malaysia and Singapore concluded an operational agreement whose terms of reference are given as follows:

3.1	the use of the following allotments by Singapore is subject to coordination with the Administration of Malaysia: 3 074, 3 095, 3 101, 3 116, 4 718, 6 685, 6 694, 6 700, 6 730, 6 760, 8 968, 11 199 and 13 206 kHz;

3.2	the use of the following allotments by Malaysia is subject to coordination with the administration of Singapore: 3 080, 4 739, 6 724 and 9 019 kHz.

PART IV  –  Criteria for compatibility assessment

26/6	For assessment of the possibilities of sharing between the allotments contained in Part III of this Appendix, and any new assignment which is not covered by an appropriate allotment, the following criteria shall be used:

26/6.1	A new station, not covered by an allotment, which uses the standardized transmission characteristics (J3E, 36 dBW (PX)) shall be considered compatible with the Plan, if it fulfils the criterion of being separated from any point of any allotment area, indicated in the Plan on the given channel, by the repetition half distance, determined for the given conditions of operation (frequency band used, geographical position of the station, direction of propagation), which are given below:

| Frequency band (kHz) | Repetition half-distance (km) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Northern hemisphere |  | Southern hemisphere |  |
|  | North-South | East-West | North-South | East-West |
| 3 025- 3 155 | 550 | 600 | 550 | 600 |
| 3 900- 3 950 | 650 | 650 | 650 | 650 |
| 4 700- 4 750 | 725 | 775 | 725 | 775 |
| 5 680- 5 730 | 1 175 | 1 325 | 1 150 | 1 300 |
| 6 685- 6 765 | 1 350 | 1 600 | 1 225 | 1 425 |
| 8 965- 9 040 | 2 525 | 3 525 | 2 225 | 3 075 |
| 11 175-11 275 | 3 375 | 5 575 | 2 675 | 3 925 |
| 13 200-13 260 | 4 550 | 6 650 | 3 475 | 5 625 |
| 15 010-15 100 | 5 050 | 7 450 | 4 800 | 7 100 |
| 17 970-18 030 | 5 750 | 8 250 | 5 675 | 7 475 |

26/6.2	The relevant value of the repetition half distance for paths which are situated partly in the northern hemisphere and partly in the southern hemisphere shall be corrected using the linear interpolation procedure. This procedure shall be used to calculate the correction due to the azimuth of the propagation path with respect to true North.

26/6.3	The relevant value of the repetition half distance, obtained in accordance with No. 26/6.2, shall be corrected, where necessary, to take into account the difference in the radiated power of the assignment with respect to the reference radiated power (30 dBW, mean radiated power) on the basis that a variation of 1 dB in the radiated power corresponds to a variation of 4% in the repetition distance.

PART V  –  Procedure for modification and maintenance of Part III

26/7	Part III will be updated by the Bureau in accordance with the following procedure:

26/7.1	a)	when a country which has no allotment in Part III requests an allotment, the Bureau shall select an appropriate allotment on a priority basis and shall enter it in Part III;

26/7.2	b)	when a request is submitted for an additional allotment, the Bureau shall apply the criteria of Part IV, and, where appropriate, enter the corresponding allotment in Part III;

26/7.3	c)	when an administration informs the Bureau that it renounces the use of an allotment, the Bureau shall cancel the allotment concerned from Part III.

26/8	The Bureau shall maintain an up-to-date master copy of Part III, and shall periodically, but no less frequently than once a year, prepare recapitulative documents listing all amendments made to Part III.

26/9	The Secretary-General shall publish an up-to-date version of Part III in an appropriate form at least once every four years.

APPENDIX 27 (REV.WRC-12)*

Frequency allotment Plan for the aeronautical mobile (R)
service and related information

(See Article 43)

TABLE  OF  CONTENTS

PART  I  –  General provisions

Page

Section I	Definitions		3

Section II	Technical and operational principles used for the establishment of the Plan of allotment of frequencies in the aeronautical mobile (R) service

A  –  Channel characteristics and utilization		4

B  –  Interference range contours		7

|  | Major world air route area maps (MWARAs) (Maps 1a, 1b, 4 and 6) |  |
| --- | --- | --- |
|  | Regional and domestic air route area maps (RDARAs) (Maps 2a, 2b, 5 and 7) | Pocket |
|  | VOLMET allotment and reception area maps (Maps 3a, 3b, 8 and 9) |  |
|  | Transparencies used with above Maps |  |

C  –  Classes of emission and power		21

D  –  Limits to the power levels of unwanted emissions		23

E  –  Other technical provisions		24

PART  II  –  Plan for the allotment of frequencies for the aeronautical mobile (R) service 
in the exclusive bands between 2 850 and 22 000 kHz

Page

Section I	Description of the boundaries of the areas and sub-areas

Article 1	Description of the boundaries of the major world air route areas (MWARAs)		25

Article 2	Description of the boundaries of the regional and domestic air route areas (RDARAs)		27

Article 3	Description of the boundaries of the VOLMET allotment areas and VOLMET reception areas		42

Article 4	World-wide allotment areas		44

Section II	Allotment of frequencies in the aeronautical mobile (R) service

Article 1	Frequency allotment Plan by areas		45

Article 2	Frequency allotment Plan (in numerical order of frequencies)		54

Article 3	Frequencies for common use		75

PART  I  –  General provisions

Section I  –  Definitions

27/1	1	Frequency allotment Plan: A Plan which shows the frequencies to be used in particular areas without specifying the stations to which the frequencies are to be assigned.

27/2	2	The terms to express the different methods of frequency distribution as used in this Appendix have the following meanings:

| Frequency distribution to | French | English | Spanish | Arabic | Chinese | Russian |
| --- | --- | --- | --- | --- | --- | --- |
| Services | Attribution (attribuer) | Allocation (to allocate) | Atribución (atribuir) | توزيع (يوزع) | 划分 (划分) | Распределение (распределить) |
| Areas or countries | Allotissement (allotir) | Allotment (to allot) | Adjudicación (adjudicar) | تعيين (يعين) | 分配 (分配) | Выделение (выделить) |
| Stations | Assignation (assigner) | Assignment (to assign) | Asignación (asignar) | تخصيص (يخصص) | 指配 (指配) | Присвоение (присвоить) |

27/3	3	A major world air route is a long-distance route, made up of one or more segments, essentially international in character, extending through more than one country and requiring long-distance communication facilities.

27/4	4	A major world air route area (MWARA) is an area embracing a certain number of major world air routes, which generally follow the same traffic pattern and are so related geographically that the same frequency families may logically be applied.

27/5	5	Regional and Domestic Air Route are all those using the Aeronautical Mobile (R) Service not covered by the definition of a Major World Air Route in No. 27/3.

27/6	6	Regional and Domestic Air Route Area (RDARA) is an area embracing a certain number of the air routes defined in No. 27/5.

27/7	7	A VOLMET Allotment Area is an area encompassing all points where an HF broadcast facility might be required to operate on a family of frequencies common to the area.

27/8	8	A VOLMET Reception Area is an area within which aircraft should be able to receive broadcasts from one or more stations in the associated VOLMET Allotment Area.

27/9	9	A World-Wide Allotment Area is one in which frequencies are allotted to provide long-distance communication between an aeronautical station within that allotment area and aircraft operating anywhere in the world1.

27/10	10	Family of Frequencies in the Aeronautical Mobile (R) Service contains two or more frequencies selected from different aeronautical mobile (R) bands and is intended to permit communication at any time within the authorized area of use (see Nos. 27/213 to 27/231) between aircraft stations and appropriate aeronautical stations.

Section II  –  Technical and operational principles used
for the establishment of the Plan of allotment of frequencies
in the aeronautical mobile (R) service

A  –  Channel characteristics and utilization

# 1	Frequency separation

27/11	1.1	The frequency separation between carrier (reference) frequencies shall be 3 kHz. This is adequate to permit communications using the classes of emission referred to in Nos. 27/56 to 27/59 in the frequency bands between 2 850 kHz and 22 000 kHz allocated exclusively to the aeronautical mobile (R) service. The carrier (reference) frequency of the channels in the Plan shall be an integral multiple of 1 kHz.

27/12	1.2	For radiotelephone emissions the audio frequencies will be limited to between 300 Hz and 2 700 Hz and the occupied bandwidth of other authorized emissions will not exceed the upper limit of J3E emissions. In specifying these limits, however, no restriction in their extension is implied in so far as emissions other than J3E are concerned, provided that the limits of unwanted emissions are met (see Nos. 27/73 and 27/74).

27/13	NOTE  For aircraft and aeronautical station transmitter types first installed before 1 February 1983, the audio frequencies will be limited to 3 000 Hz.

27/14	1.3	On account of the possibility of interference, a given channel should not be used in the same allotment area for radiotelephony and data transmissions.

27/15	1.4	The use of channels derived from the frequencies indicated in No. 27/18 for the various classes of emissions other than J3E and H2B will be subject to special arrangements by the administrations concerned and affected in order to avoid harmful interference which may result from the simultaneous use of the same channel for several classes of emission.

27/16	1.5	To preclude the possibility of interference, adjacent channels in the list of frequencies in No. 27/18 have not as a rule been allotted to the same MWARA, RDARA or VOLMET areas. However, to satisfy particular needs, the administrations concerned may conclude special arrangements for the assignment of adjacent channels derived from the frequencies in the Table.

27/17	1.6	The arrangements contemplated in Nos. 27/15 and 27/16 should be made under the Articles of the Constitution and Convention of the International Telecommunication Union and the Radio Regulations entitled “Special agreements”*.     (WRC-03)

# 2	Frequencies allotted

27/18		The list of carrier (reference) frequencies allotted in the bands allocated exclusively to the aeronautical mobile (R) service, on the basis of the frequency separation provided for under No. 27/11, will be found in the following Table2.

| 2 850-3 025 kHz |  |  |  |
| --- | --- | --- | --- |
| 2 851 |  | 2 938 |  |
| 2 854 |  | 2 941 |  |
| 2 857 |  | 2 944 |  |
| 2 860 |  | 2 947 |  |
| 2 863 |  | 2 950 |  |
| 2 866 |  | 2 953 |  |
| 2 869 |  | 2 956 |  |
| 2 872 |  | 2 959 |  |
| 2 875 |  | 2 962 |  |
| 2 878 |  | 2 965 |  |
| 2 881 |  | 2 968 |  |
| 2 884 |  | 2 971 |  |
| 2 887 |  | 2 974 |  |
| 2 890 |  | 2 977 |  |
| 2 893 |  | 2 980 | 57 |
| 2 896 |  | 2 983 | chan- |
| 2 899 |  | 2 986 | nels |
| 2 902 |  | 2 989 |  |
| 2 905 |  | 2 992 |  |
| 2 908 |  | 2 995 |  |
| 2 911 |  | 2 998 |  |
| 2 914 |  | 3 001 |  |
| 2 917 |  | 3 004 |  |
| 2 920 |  | 3 007 |  |
| 2 923 |  | 3 010 |  |
| 2 926 |  | 3 013 |  |
| 2 929 |  | 3 016 |  |
| 2 932 |  | 3 019 |  |
| 2 935 |  |  |  |
|  |  |  | (R) |
|  |  | 3 023 | and |
|  |  |  | (OR) |
| 3 400-3 500 kHz |  |  |  |
| 3 401 |  | 3 452 |  |
| 3 404 |  | 3 455 |  |
| 3 407 |  | 3 458 |  |
| 3 410 |  | 3 461 |  |
| 3 413 |  | 3 464 |  |
| 3 416 |  | 3 467 |  |
| 3 419 |  | 3 470 |  |
| 3 422 |  | 3 473 | 33 |
| 3 425 |  | 3 476 | chan- |
| 3 428 |  | 3 479 | nels |
| 3 431 |  | 3 482 |  |
| 3 434 |  | 3 485 |  |
| 3 437 |  | 3 488 |  |
| 3 440 |  | 3 491 |  |
| 3 443 |  | 3 494 |  |
| 3 446 |  | 3 497 |  |
| 3 449 |  |  |  |

| 4 650-4 700 kHz |  |  |  |
| --- | --- | --- | --- |
| 4 651 |  | 4 675 |  |
| 4 654 |  | 4 678 |  |
| 4 657 |  | 4 681 | 16 |
| 4 660 |  | 4 684 | chan- |
| 4 663 |  | 4 687 | nels |
| 4 666 |  | 4 690 |  |
| 4 669 |  | 4 693 |  |
| 4 672 |  | 4 696 |  |
| 5 450-5 480 kHz |  |  |  |
| Region2 |  |  |  |
| 5 451 |  | 5 466 |  |
| 5 454 |  | 5 469 | 9 |
| 5 457 |  | 5 472 | chan- |
| 5 460 |  | 5 475 | nels |
| 5 463 |  |  |  |
| 5 480-5 680 kHz |  |  |  |
| 5 481 |  | 5 580 |  |
| 5 484 |  | 5 583 |  |
| 5 487 |  | 5 586 |  |
| 5 490 |  | 5 589 |  |
| 5 493 |  | 5 592 |  |
| 5 496 |  | 5 595 |  |
| 5 499 |  | 5 598 |  |
| 5 502 |  | 5 601 |  |
| 5 505 |  | 5 604 |  |
| 5 508 |  | 5 607 |  |
| 5 511 |  | 5 610 |  |
| 5 514 |  | 5 613 |  |
| 5 517 |  | 5 616 |  |
| 5 520 |  | 5 619 |  |
| 5 523 |  | 5 622 |  |
| 5 526 |  | 5 625 |  |
| 5 529 |  | 5 628 | 66 |
| 5 532 |  | 5 631 | chan- |
| 5 535 |  | 5 634 | nels |
| 5 538 |  | 5 637 |  |
| 5 541 |  | 5 640 |  |
| 5 544 |  | 5 643 |  |
| 5 547 |  | 5 646 |  |
| 5 550 |  | 5 649 |  |
| 5 553 |  | 5 652 |  |
| 5 556 |  | 5 655 |  |
| 5 559 |  | 5 658 |  |
| 5 562 |  | 5 661 |  |
| 5 565 |  | 5 664 |  |
| 5 568 |  | 5 667 |  |
| 5 571 |  | 5 670 |  |
| 5 574 |  | 5 673 |  |
| 5 577 |  | 5 676 |  |
|  |  |  | (R) |
|  |  | 5 680 | and |
|  |  |  | (OR) |

| 6 525-6 685 kHz |  |  |  |
| --- | --- | --- | --- |
| 6 526 |  | 6 607 |  |
| 6 529 |  | 6 610 |  |
| 6 532 |  | 6 613 |  |
| 6 535 |  | 6 616 |  |
| 6 538 |  | 6 619 |  |
| 6 541 |  | 6 622 |  |
| 6 544 |  | 6 625 |  |
| 6 547 |  | 6 628 |  |
| 6 550 |  | 6 631 |  |
| 6 553 |  | 6 634 |  |
| 6 556 |  | 6 637 |  |
| 6 559 |  | 6 640 |  |
| 6 562 |  | 6 643 | 53 |
| 6 565 |  | 6 646 | chan- |
| 6 568 |  | 6 649 | nels |
| 6 571 |  | 6 652 |  |
| 6 574 |  | 6 655 |  |
| 6 577 |  | 6 658 |  |
| 6 580 |  | 6 661 |  |
| 6 583 |  | 6 664 |  |
| 6 586 |  | 6 667 |  |
| 6 589 |  | 6 670 |  |
| 6 592 |  | 6 673 |  |
| 6 595 |  | 6 676 |  |
| 6 598 |  | 6 679 |  |
| 6 601 |  | 6 682 |  |
| 6 604 |  |  |  |
| 8 815 -8 965 kHz |  |  |  |
| 8 816 |  | 8 891 |  |
| 8 819 |  | 8 894 |  |
| 8 822 |  | 8 897 |  |
| 8 825 |  | 8 900 |  |
| 8 828 |  | 8 903 |  |
| 8 831 |  | 8 906 |  |
| 8 834 |  | 8 909 |  |
| 8 837 |  | 8 912 |  |
| 8 840 |  | 8 915 |  |
| 8 843 |  | 8 918 |  |
| 8 846 |  | 8 921 |  |
| 8 849 |  | 8 924 |  |
| 8 852 |  | 8 927 | 49 |
| 8 855 |  | 8 930 | chan- |
| 8 858 |  | 8 933 | nels |
| 8 861 |  | 8 936 |  |
| 8 864 |  | 8 939 |  |
| 8 867 |  | 8 942 |  |
| 8 870 |  | 8 945 |  |
| 8 873 |  | 8 948 |  |
| 8 876 |  | 8 951 |  |
| 8 879 |  | 8 954 |  |
| 8 882 |  | 8 957 |  |
| 8 885 |  | 8 960 |  |
| 8 888 |  |  |  |

| 10 005-10 100 kHz |  |  |  |
| --- | --- | --- | --- |
| 10 006 |  | 10 054 |  |
| 10 009 |  | 10 057 |  |
| 10 012 |  | 10 060 |  |
| 10 015 |  | 10 063 |  |
| 10 018 |  | 10 066 |  |
| 10 021 |  | 10 069 |  |
| 10 024 |  | 10 072 | 31 |
| 10 027 |  | 10 075 | chan- |
| 10 030 |  | 10 078 | nels |
| 10 033 |  | 10 081 |  |
| 10 036 |  | 10 084 |  |
| 10 039 |  | 10 087 |  |
| 10 042 |  | 10 090 |  |
| 10 045 |  | 10 093 |  |
| 10 048 |  | 10 096 |  |
| 10 051 |  |  |  |
| 11 275-11 400 kHz |  |  |  |
| 11 276 |  | 11 339 |  |
| 11 279 |  | 11 342 |  |
| 11 282 |  | 11 345 |  |
| 11 285 |  | 11 348 |  |
| 11 288 |  | 11 351 |  |
| 11 291 |  | 11 354 |  |
| 11 294 |  | 11 357 |  |
| 11 297 |  | 11 360 |  |
| 11 300 |  | 11 363 |  |
| 11 303 |  | 11 366 | 41 |
| 11 306 |  | 11 369 | chan- |
| 11 309 |  | 11 372 | nels |
| 11 312 |  | 11 375 |  |
| 11 315 |  | 11 378 |  |
| 11 318 |  | 11 381 |  |
| 11 321 |  | 11 384 |  |
| 11 324 |  | 11 387 |  |
| 11 327 |  | 11 390 |  |
| 11 330 |  | 11 393 |  |
| 11 333 |  | 11 396 |  |
| 11 336 |  |  |  |

| 13 260-13 360 kHz |  |  |  |
| --- | --- | --- | --- |
| 13 261 |  | 13 312 |  |
| 13 264 |  | 13 315 |  |
| 13 267 |  | 13 318 |  |
| 13 270 |  | 13 321 |  |
| 13 273 |  | 13 324 |  |
| 13 276 |  | 13 327 |  |
| 13 279 |  | 13 330 |  |
| 13 282 |  | 13 333 | 33 |
| 13 285 |  | 13 336 | chan- |
| 13 288 |  | 13 339 | nels |
| 13 291 |  | 13 342 |  |
| 13 294 |  | 13 345 |  |
| 13 297 |  | 13 348 |  |
| 13 300 |  | 13 351 |  |
| 13 303 |  | 13 354 |  |
| 13 306 |  | 13 357 |  |
| 13 309 |  |  |  |
| 17 900-17 970 kHz |  |  |  |
| 17 901 |  | 17 937 |  |
| 17 904 |  | 17 940 |  |
| 17 907 |  | 17 943 |  |
| 17 910 |  | 17 946 |  |
| 17 913 |  | 17 949 | 23 |
| 17 916 |  | 17 952 | chan- |
| 17 919 |  | 17 955 | nels |
| 17 922 |  | 17 958 |  |
| 17 925 |  | 17 961 |  |
| 17 928 |  | 17 964 |  |
| 17 931 |  | 17 967 |  |
| 17 934 |  |  |  |
| 21 924-22 000 kHz |  |  |  |
| 21 925 |  | 21 964 |  |
| 21 928 |  | 21 967 |  |
| 21 931 |  | 21 970 |  |
| 21 934 |  | 21 973 |  |
| 21 937 |  | 21 976 |  |
| 21 940 |  | 21 979 | 25 |
| 21 943 |  | 21 982 | chan- |
| 21 946 |  | 21 985 | nels |
| 21 949 |  | 21 988 |  |
| 21 952 |  | 21 991 |  |
| 21 955 |  | 21 994 |  |
| 21 958 |  | 21 997 |  |
| 21 961 |  |  |  |

27/19	3	The International Civil Aviation Organization (ICAO) coordinates radiocommunications of the aeronautical mobile (R) service with international aeronautical operations and this Organization should be consulted in all appropriate cases in the operational use of the frequencies in the Plan.

# 3	Adaptation of allotment procedure

27/20		It is recognized that not all the sharing possibilities have been exhausted in the allotment Plan contained in this Appendix. Therefore, in order to satisfy particular operational requirements which are not otherwise met by this allotment Plan, Administrations may assign frequencies from the aeronautical mobile (R) bands in areas other than those to which they are allotted in this Plan. However, the use of the frequencies so assigned must not reduce the protection to the same frequencies in the areas where they are allotted by the Plan below that determined by the application of the procedure defined in Part I, Section II B of this Appendix.

27/21	5	When necessary to satisfy the needs of international air operations Administrations may adapt the allotment procedure for the assignment of aeronautical mobile (R) frequencies, which assignments shall then be the subject of prior agreement between Administrations affected.

27/22	6	The coordination described in No. 27/19 shall be effected where appropriate and desirable for the efficient utilization of the frequencies in question, and especially when the procedures of No. 27/21 are unsatisfactory.

B  –  Interference range contours

# 27/23	1	General provisions

## 27/24	1.1	Service range

Due to factors such as the power of the transmitter, propagation loss, noise level, etc., there is a limit to the distance at which reliable communications can be effected between an aeronautical station and an aircraft station. This limiting distance, based on the weakest path, is the service range. The boundary of the air route area is often assumed to be the limiting distance.

## 27/25	1.2	Interference range

This is the minimum distance from the limit of the service range of a wanted station to a potentially interfering station needed to produce a protection ratio of 15 dB. This protection ratio is between the wanted signal at an aircraft station at the limit of the service range and the signal from a potentially interfering aeronautical station operating on the same frequency. The interference range has been calculated for different frequencies indicated on the data Tables contained in Nos. 27/46 to 27/55 for day and night conditions, for median latitudes, for conditions of median sunspot activity and for a mean effective radiated power of 1 kW at the aeronautical station.

## 27/26	1.3	Repetition distance

This is the distance at which a frequency may be successfully shared and is equal to the sum of the service range and the interference range.

27/27	1.4	Figure 1 illustrates the use of the concept of interference range in frequency planning through the determination of repetition distance.

27/28	1.5	The transparencies associated with this Appendix show, for the frequencies stated, the interference range defined in No. 27/25 between an interfering aeronautical station and an aircraft station operating at the limit of its service range. Because of the variability of propagation conditions not only from hour to hour within the daytime and night time periods but also from day to day, with season, with solar activity level and geographic location, the 15 dB protection ratio may be expected to have marked variations and accordingly a greater protection may be available much of the time, especially when the aircraft is not operating at the limit of its service range.

27/29	(SUP - WRC-03)

27/30	1.7	Two types of transparencies are provided for use respectively with the Mercator projection world maps and the Lambert azimuthal equal area of projection maps for the polar areas. The Mercator projection transparencies encompass the area between latitude 60° North and 60° South. The transparencies associated with the Polar area projections encompass the areas north of latitude 30° North and south of latitude 30° South. The Mercator projection overlaps the Polar projection maps between latitudes 30° and 60° North and 30° and 60° South. This overlap is intended to provide continuity between transparencies, of the two projections.

# 2	Type of maps used

27/31		The transparencies mentioned in Nos. 27/28 and 27/30, can be used only on a world or polar map of the projection and scales given on each transparency and will not be suitable for use on any other projection or scale. The world and polar maps associated with this Appendix, depicting MWARA, RDARA and VOLMET areas, are to the correct scale so that the transparencies carrying the interference range contours can be directly used on these maps. The auroral zones are marked on the polar maps.

# 3	Change of scale of projection

27/32	3.1	Should any other scale or projection be desired, then new interference range contours can be drawn to fit the new scales or projections by using the coordinates given in the Tables shown below.

27/33	3.2	When new transparencies are constructed, the intersection of the vertical line of symmetry, i.e., the meridian of longitude and the horizontal line of latitude should be at 00° latitude for the 00° contour, 20° N for the 20° contour, 40° N for 40° contour, etc.

27/34	3.3	The coordinates shown in the Tables under Nos. 27/46 to 27/55 are given with reference to the 180° meridian taken as the axis of symmetry for the construction of the contours.

# 4	Sharing conditions between areas

## 4.1	Frequency bands between 3 and 11.3 MHz

27/35	4.1.1	The transparencies are constructed on the basis of the following sharing conditions:

| Areas | Bands between (MHz) | Sharing conditions |
| --- | --- | --- |
| MWARA or VOLMET area to MWARA or VOLMET area | 3 and 6.6 9 and 11.3 | Night propagation Day propagation NOTE – 6.6 MHz and 5.6 MHz sharing conditions are considered to be the same. |
| MWARA or VOLMET area to RDARA | 3 and 5.6 6.6 and 11.3 | Night propagation Day propagation |
| RDARA to RDARA | 3 and 4.7 5.6 and 11.3 | Night propagation Day propagation |

27/36	4.1.2	The additional “Day” contours included for 3 MHz, 3.5 MHz and 4.7 MHz are for determining daylight sharing possibilities.

## 4.2	Frequency bands between 13 and 22 MHz

27/37	4.2.1	The revised frequency allotment Plan for the 13 MHz, 18 MHz and 22 MHz bands is based on daytime protection only. This results in the following sharing possibilities:

27/38	4.2.2	for the 13 MHz band, the repetition factor is at least 3 whilst for the 18 and 22 MHz bands it is 4. It is to be noted that the longitudinal separation might be decreased to allow for a repetition of 4 (at 13 MHz) and 6 (at 18 and 22 MHz), taking into account operational and local circumstances;

27/39	4.2.3	the sharing takes into account the likely locations of the aeronautical stations rather than the area boundaries.

# 5	Method of use of the transparencies for the bands 3 to 11.3 MHz

27/40	5.1	Take the appropriate MWARA, RDARA or VOLMET area map associated with this Appendix and select the transparency for the frequency order and sharing conditions under consideration.

27/41	5.2	The equal area projections (Lambert) are applicable in the polar areas north of 60° N and south of 60° S; and the Mercator projections are applicable between 60° N and 60° S.

27/42	5.3	Place the centre of the transparency (i.e. the intersection of the axis of symmetry and the latitude line) over the boundary of the area (use the reception area boundary in the case of VOLMET) at the point on the boundary nearest to the potentially interfering transmitter or at the location of the interfering transmitter. Note the latitude of the selected point and use the interference range contour corresponding to this latitude.

27/43	5.4	A transmitter located at any point outside the contour will result, as defined in No. 27/25, in a protection ratio of better than 15 dB.

27/44	5.5	A transmitter located at any point inside the contour will result in a protection ratio of less than 15 dB. However, if the transmitter is located inside the contour but the propagation path traverses an auroral zone, it is assumed that the signal attenuation within this zone will result in a protection ratio of better than 15 dB.

27/45	5.6	For the Northern Hemisphere the Mercator projection transparencies should be used in their natural position as published, but for the Southern Hemisphere the transparencies should be inverted. This point should be carefully observed when following the boundaries of areas which involve the transition of the equator.

# 6	Data for tracing interference contours

27/46	3.0 and 3.5 MHz day

Data for plotting 700 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 6.3 | 180.0 | 16.3 | 180.0 | 26.3 | 180.0 | 36.3 | 180.0 | 46.3 |
|  | 178.9 | 6.2 | 178.9 | 16.2 | 178.8 | 26.2 | 178.6 | 36.2 | 178.4 | 46.2 |
|  | 177.8 | 5.9 | 177.8 | 15.9 | 177.6 | 25.9 | 177.3 | 35.9 | 176.9 | 45.9 |
|  | 176.8 | 5.5 | 176.7 | 15.4 | 176.5 | 25.4 | 176.1 | 35.4 | 175.5 | 45.4 |
|  | 175.9 | 4.8 | 175.8 | 14.8 | 175.5 | 24.8 | 175.1 | 34.7 | 174.3 | 44.7 |
| Coordinates | 175.2 | 4.0 | 175.0 | 14.0 | 174.7 | 24.0 | 174.2 | 33.9 | 173.3 | 43.9 |
| for | 174.5 | 3.1 | 174.4 | 13.1 | 174.1 | 23.0 | 173.5 | 33.0 | 172.5 | 42.9 |
| plotting | 174.1 | 2.2 | 173.9 | 12.1 | 173.6 | 22.0 | 173.0 | 32.0 | 172.0 | 41.9 |
| contours | 173.8 | 1.1 | 173.7 | 11.0 | 173.4 | 21.0 | 172.8 | 30.9 | 171.8 | 40.8 |
|  | 173.7 | 0.0 | 173.6 | 9.9 | 173.3 | 19.9 | 172.7 | 29.8 | 171.8 | 39.7 |
|  | 173.8 | −1.1 | 173.7 | 8.8 | 173.4 | 18.8 | 172.9 | 28.7 | 172.0 | 38.6 |
|  | 174.1 | −2.2 | 174.0 | 7.8 | 173.8 | 17.7 | 173.3 | 27.7 | 172.5 | 37.6 |
|  | 174.5 | −3.1 | 174.5 | 6.8 | 174.3 | 16.8 | 173.9 | 26.7 | 173.2 | 36.6 |
|  | 175.2 | −4.0 | 175.2 | 5.9 | 175.0 | 15.9 | 174.6 | 25.8 | 174.1 | 35.8 |
|  | 175.9 | −4.8 | 175.9 | 5.2 | 175.8 | 25.1 | 175.5 | 25.1 | 175.1 | 35.1 |
|  | 176.8 | −5.5 | 176.8 | 4.5 | 176.8 | 14.5 | 176.5 | 24.5 | 176.2 | 34.5 |
|  | 177.8 | −5.9 | 177.8 | 4.1 | 177.8 | 14.1 | 177.6 | 24.1 | 177.4 | 34.0 |
|  | 178.9 | −6.2 | 178.9 | 3.8 | 178.9 | 13.8 | 178.8 | 23.8 | 178.7 | 33.8 |
|  | 180.0 | −6.3 | 180.0 | 3.7 | 180.0 | 13.7 | 180.0 | 23.7 | 180.0 | 33.7 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 56.3 | 180.0 | 66.3 | 180.0 | 76.3 | 180.0 | 86.3 |  | 83.7 |
|  | 178.0 | 56.2 | 177.3 | 66.2 | 175.4 | 76.2 | 163.9 | 86.1 |  | 83.7 |
|  | 176.2 | 55.9 | 174.7 | 65.8 | 171.2 | 75.8 | 152.2 | 85.4 |  | 83.7 |
|  | 174.5 | 55.3 | 172.5 | 65.3 | 167.7 | 75.1 | 145.2 | 84.5 |  | 83.7 |
|  | 173.0 | 54.6 | 170.6 | 64.5 | 164.9 | 74.3 | 141.9 | 83.4 |  | 83.7 |
| Coordinates | 171.8 | 53.8 | 169.1 | 63.6 | 162.9 | 73.4 | 140.8 | 82.4 | All | 83.7 |
| for | 171.0 | 52.8 | 168.1 | 62.7 | 161.8 | 72.3 | 141.3 | 81.3 | longitudes | 83.7 |
| plotting | 170.4 | 51.8 | 167.5 | 61.6 | 161.3 | 71.2 | 142.8 | 80.2 |  | 83.7 |
| contours | 170.2 | 50.7 | 167.3 | 60.5 | 161.5 | 70.1 | 144.9 | 79.2 |  | 83.7 |
|  | 170.3 | 49.6 | 167.5 | 59.4 | 162.1 | 69.1 | 147.6 | 78.2 |  | 83.7 |
|  | 170.6 | 48.5 | 168.1 | 58.3 | 163.2 | 68.0 | 150.5 | 77.3 |  | 83.7 |
|  | 171.2 | 47.5 | 169.0 | 57.4 | 164.6 | 67.1 | 153.8 | 76.5 |  | 83.7 |
|  | 172.1 | 46.6 | 170.1 | 56.4 | 166.4 | 66.2 | 157.3 | 75.8 |  | 83.7 |
|  | 173.1 | 45.7 | 171.4 | 55.6 | 168.3 | 65.5 | 160.8 | 75.2 |  | 83.7 |
|  | 174.3 | 45.0 | 172.9 | 55.0 | 170.4 | 64.9 | 164.6 | 74.6 |  | 83.7 |
|  | 175.6 | 44.5 | 174.6 | 54.4 | 172.7 | 64.4 | 168.4 | 74.2 |  | 83.7 |
|  | 177.0 | 44.0 | 176.3 | 54.0 | 175.1 | 64.0 | 172.2 | 739 |  | 83.7 |
|  | 178.5 | 43.8 | 178.2 | 53.8 | 177.5 | 63.8 | 176.1 | 73.8 |  | 83.7 |
|  | 180.0 | 43.7 | 180.0 | 53.7 | 180.0 | 63.7 | 180.0 | 73.7 |  | 83.7 |

27/47	3.0 MHz night

Data for plotting 3 500 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 31.5 | 180.0 | 41.5 | 180.0 | 51.5 | 180.0 | 61.5 | 180.0 | 71.5 |
|  | 173.9 | 31.0 | 173.1 | 40.9 | 171.7 | 50.8 | 169.3 | 60.7 | 164.3 | 70.4 |
|  | 168.2 | 29.4 | 166.7 | 39.2 | 164.2 | 48.9 | 160.1 | 58.4 | 152.1 | 67.5 |
|  | 163.0 | 26.9 | 161.1 | 36.4 | 158.0 | 45.8 | 153.0 | 54.9 | 144.2 | 63.5 |
|  | 158.5 | 23.6 | 156.4 | 32.8 | 153.2 | 41.9 | 148.0 | 50.6 | 139.7 | 58.7 |
| Coordinates | 154.9 | 19.6 | 152.9 | 28.6 | 149.8 | 37.4 | 144.9 | 45.8 | 137.5 | 53.6 |
| for | 152.0 | 15.1 | 150.3 | 23.9 | 147.6 | 32.5 | 143.3 | 40.7 | 137.0 | 48.4 |
| plotting | 150.1 | 10.3 | 148.7 | 18.9 | 146.4 | 27.4 | 142.9 | 35.5 | 137.6 | 43.2 |
| contours | 148.9 | 5.2 | 148.0 | 13.7 | 146.3 | 22.1 | 143.4 | 30.3 | 139.1 | 38.1 |
|  | 148.5 | 0.0 | 148.1 | 8.5 | 146.9 | 17.0 | 144.7 | 25.2 | 141.3 | 33.2 |
|  | 148.9 | −5.2 | 149.0 | 3.4 | 148.3 | 11.9 | 146.7 | 20.9 | 144.1 | 28.6 |
|  | 150.1 | −10.3 | 150.6 | −1.6 | 150.3 | 7.1 | 149.3 | 15.8 | 147.4 | 24.3 |
|  | 152.0 | −15.1 | 152.9 | −6.3 | 153.1 | 2.6 | 152.5 | 11.5 | 151.1 | 20.4 |
|  | 154.9 | −19.6 | 156.0 | −10.5 | 156.4 | −1.4 | 156.2 | 7.8 | 155.3 | 16.9 |
|  | 158.5 | −23.6 | 159.7 | −14.2 | 160.3 | −4.8 | 160.3 | 4.6 | 159.8 | 14.0 |
|  | 163.0 | −26.9 | 164.1 | −17.3 | 164.7 | −7.7 | 164.8 | 2.0 | 164.5 | 11.6 |
|  | 168.2 | −29.4 | 169.1 | −19.6 | 169.6 | −9.8 | 169.7 | 0.1 | 169.5 | 9.9 |
|  | 173.9 | −31.0 | 174.4 | −21.0 | 174.7 | −11.1 | 174.8 | −1.1 | 174.7 | 8.9 |
|  | 180.0 | −31.5 | 180.0 | −21.5 | 180.0 | −11.5 | 180.0 | −1.5 | 180.0 | 8.5 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 81.5 | 0 | 88.5 | 0 | 78.5 | 0 | 68.5 |  | 58.5 |
|  | 149.5 | 79.7 | 78.0 | 84.7 | 25.3 | 77.7 | 14.2 | 68.3 |  | 58.5 |
|  | 133.9 | 75.6 | 90.4 | 79.7 | 46.5 | 75.7 | 28.0 | 67.7 |  | 58.5 |
|  | 127.6 | 70.7 | 97.5 | 74.7 | 62.9 | 72.9 | 41.3 | 66.7 |  | 58.5 |
|  | 125.7 | 65.6 | 103.3 | 69.8 | 75.9 | 69.7 | 53.8 | 65.4 |  | 58.5 |
| Coordinates | 126.0 | 60.3 | 108.7 | 65.0 | 86.6 | 66.4 | 65.5 | 63.9 | All | 58.5 |
| for | 127.6 | 55.2 | 113.9 | 60.3 | 95.8 | 62.9 | 76.4 | 62.3 | longitudes | 58.5 |
| plotting | 129.9 | 50.2 | 118.9 | 55.9 | 104.1 | 59.6 | 86.7 | 60.5 |  | 58.5 |
| contours | 132.9 | 45.4 | 124.1 | 51.6 | 111.9 | 56.3 | 96.5 | 58.8 |  | 58.5 |
|  | 136.4 | 40.8 | 129.2 | 47.6 | 119.2 | 53.2 | 105.8 | 57.1 |  | 58.5 |
|  | 140.2 | 36.5 | 134.5 | 43.9 | 126.2 | 50.4 | 114.8 | 55.5 |  | 58.5 |
|  | 144.4 | 32.6 | 139.8 | 40.5 | 133.1 | 47.7 | 123.4 | 54.0 |  | 58.5 |
|  | 148.8 | 29.0 | 145.3 | 37.4 | 139.9 | 45.4 | 131.9 | 52.6 |  | 58.5 |
|  | 153.6 | 25.9 | 150.8 | 34.8 | 146.6 | 43.3 | 140.1 | 51.4 |  | 58.5 |
|  | 158.5 | 23.3 | 156.5 | 32.6 | 153.3 | 41.6 | 148.2 | 50.4 |  | 58.5 |
|  | 163.7 | 21.2 | 162.3 | 30.8 | 160.0 | 40.3 | 156.2 | 49.6 |  | 58.5 |
|  | 169.1 | 19.7 | 168.1 | 29.5 | 166.6 | 39.3 | 164.2 | 49.0 |  | 58.5 |
|  | 174.5 | 18.8 | 174.1 | 28.8 | 173.3 | 38.7 | 172.1 | 48.6 |  | 58.5 |
|  | 180.0 | 18.5 | 180.0 | 28.5 | 180.0 | 38.5 | 180.0 | 48.5 |  | 58.5 |

27/48	3.5 MHz night

Data for plotting 4 000 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 36.0 | 180.0 | 46.0 | 180.0 | 56.0 | 180.0 | 66.0 | 180.0 | 76.0 |
|  | 172.8 | 35.4 | 171.7 | 45.3 | 169.7 | 55.1 | 166.1 | 64.9 | 157.6 | 74.5 |
|  | 166.0 | 33.5 | 164.0 | 43.2 | 160.6 | 52.7 | 154.7 | 62.0 | 142.8 | 70.6 |
|  | 160.0 | 30.6 | 157.5 | 39.9 | 153.4 | 49.0 | 146.6 | 57.7 | 134.9 | 70.6 |
|  | 155.0 | 26.8 | 152.3 | 35.7 | 148.1 | 44.4 | 141.5 | 52.6 | 131.2 | 59.9 |
| Coordinates | 150.9 | 22.2 | 148.4 | 30.8 | 144.5 | 39.2 | 138.7 | 47.0 | 129.9 | 54.0 |
| for | 147.8 | 17.1 | 145.7 | 25.5 | 142.3 | 33.6 | 137.4 | 41.2 | 130.2 | 48.2 |
| plotting | 145.7 | 11.6 | 144.1 | 19.8 | 141.4 | 27.7 | 137.4 | 35.4 | 131.6 | 42.4 |
| contours | 144.4 | 5.9 | 143.4 | 13.9 | 141.4 | 21.9 | 138.3 | 29.5 | 133.8 | 36.7 |
|  | 144.0 | 0.0 | 143.6 | 8.1 | 142.3 | 16.1 | 140.0 | 23.9 | 136.5 | 31.3 |
|  | 144.4 | −5.9 | 144.6 | 2.3 | 143.9 | 10.4 | 142.4 | 18.4 | 139.8 | 26.2 |
|  | 145.7 | −11.6 | 146.4 | −3.3 | 146.3 | 5.0 | 145.4 | 13.3 | 143.6 | 21.5 |
|  | 147.8 | −17.1 | 149.0 | −8.6 | 149.4 | 0.0 | 149.0 | 8.6 | 147.8 | 17.2 |
|  | 150.9 | −22.2 | 152.4 | −13.4 | 153.1 | −4.5 | 153.2 | 4.4 | 152.4 | 13.3 |
|  | 155.0 | −26.8 | 156.6 | −17.6 | 157.5 | −8.4 | 157.8 | 0.8 | 157.4 | 10.1 |
|  | 160.0 | −30.6 | 161.6 | −21.2 | 162.5 | −11.6 | 162.9 | −2.1 | 162.8 | 7.5 |
|  | 166.0 | −33.5 | 167.3 | −23.8 | 168.0 | −14.0 | 168.4 | −4.2 | 168.3 | 5.6 |
|  | 172.8 | −35.4 | 173.5 | −25.4 | 173.9 | −15.5 | 174.1 | −5.6 | 174.1 | 4.4 |
|  | 180.0 | −36.0 | 180.0 | −26.0 | 180.0 | −16.0 | 180.0 | −6.0 | 180.0 | 4.0 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 86.0 | 0 | 84.0 | 0 | 74.0 | 0 | 64.0 |  | 54.0 |
|  | 126.9 | 82.7 | 46.5 | 81.9 | 20.9 | 73.4 | 13.4 | 63.8 |  | 54.0 |
|  | 115.7 | 77.1 | 69.8 | 77.6 | 39.7 | 71.6 | 26.5 | 63.2 |  | 54.0 |
|  | 113.9 | 71.3 | 83.0 | 72.8 | 55.5 | 69.1 | 39.2 | 62.3 |  | 54.0 |
|  | 114.9 | 65.4 | 92.2 | 67.8 | 68.8 | 66.1 | 51.3 | 61.0 |  | 54.0 |
| Coordinates | 117.1 | 59.6 | 99.7 | 62.8 | 80.1 | 62.8 | 62.8 | 59.6 | All | 54.0 |
| for | 120.1 | 54.0 | 106.4 | 57.9 | 90.1 | 59.4 | 73.7 | 58.0 | longitudes | 54.0 |
| plotting | 123.5 | 48.5 | 112.6 | 53.2 | 99.0 | 56.0 | 84.1 | 56.3 |  | 54.0 |
| contours | 127.4 | 43.3 | 118.6 | 48.7 | 107.3 | 52.7 | 93.9 | 54.5 |  | 54.0 |
|  | 131.5 | 38.3 | 124.5 | 44.5 | 115.2 | 49.5 | 103.4 | 52.8 |  | 54.0 |
|  | 135.9 | 33.7 | 130.4 | 40.5 | 122.8 | 46.5 | 112.6 | 51.2 |  | 54.0 |
|  | 140.7 | 29.4 | 136.3 | 36.9 | 130.1 | 43.7 | 121.5 | 49.6 |  | 54.0 |
|  | 145.7 | 25.5 | 142.3 | 33.6 | 137.4 | 41.3 | 130.2 | 48.2 |  | 54.0 |
|  | 150.9 | 22.1 | 148.4 | 30.8 | 144.5 | 39.1 | 138.7 | 47.0 |  | 54.0 |
|  | 156.4 | 19.3 | 154.6 | 28.4 | 151.6 | 37.3 | 147.1 | 45.9 |  | 54.0 |
|  | 162.1 | 17.0 | 160.8 | 26.5 | 158.7 | 35.9 | 155.4 | 45.1 |  | 54.0 |
|  | 168.0 | 15.3 | 167.2 | 25.1 | 165.8 | 34.8 | 163.6 | 44.5 |  | 54.0 |
|  | 174.0 | 14.3 | 173.6 | 24.3 | 172.9 | 34.2 | 171.8 | 44.1 |  | 54.0 |
|  | 180.0 | 14.0 | 180.0 | 24.0 | 180.0 | 34.0 | 180.0 | 44.0 |  | 54.0 |

27/49	4.7 MHz day

Data for plotting 1 200 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 10.8 | 180.0 | 20.8 | 180.0 | 30.8 | 180.0 | 40.8 | 180.0 | 50.8 |
|  | 178.1 | 10.6 | 178.0 | 20.6 | 177.8 | 30.6 | 177.5 | 40.6 | 177.1 | 50.6 |
|  | 176.3 | 10.1 | 176.1 | 20.1 | 175.8 | 30.1 | 175.2 | 40.1 | 174.3 | 50.0 |
|  | 174.6 | 9.3 | 174.3 | 19.3 | 173.8 | 29.2 | 173.1 | 39.2 | 171.8 | 49.1 |
|  | 173.0 | 8.3 | 172.7 | 18.2 | 172.2 | 28.1 | 171.2 | 38.0 | 169.7 | 47.8 |
| Coordinates | 171.7 | 6.9 | 171.4 | 16.8 | 170.3 | 26.7 | 169.7 | 36.5 | 168.0 | 46.4 |
| for | 170.6 | 5.4 | 170.3 | 15.2 | 169.7 | 25.1 | 168.6 | 34.9 | 166.8 | 44.7 |
| plotting | 169.8 | 3.7 | 169.6 | 13.5 | 168.9 | 23.3 | 167.9 | 33.1 | 166.1 | 42.9 |
| contours | 169.4 | 1.9 | 169.1 | 11.7 | 168.6 | 21.5 | 167.5 | 31.3 | 165.8 | 41.0 |
|  | 169.2 | 0.0 | 169.0 | 9.8 | 168.5 | 19.6 | 167.6 | 29.4 | 166.0 | 39.2 |
|  | 169.4 | −1.9 | 169.3 | 8.0 | 168.8 | 17.8 | 168.0 | 27.6 | 166.6 | 37.3 |
|  | 169.8 | −3.7 | 169.8 | 6.2 | 169.4 | 16.0 | 168.7 | 25.8 | 167.5 | 35.6 |
|  | 170.6 | −5.4 | 170.6 | 4.5 | 170.4 | 14.4 | 169.8 | 24.2 | 168.7 | 34.0 |
|  | 171.7 | −6.9 | 171.7 | 3.0 | 171.5 | 12.9 | 171.0 | 22.8 | 170.2 | 32.6 |
|  | 173.0 | −8.3 | 173.1 | 1.7 | 172.9 | 11.6 | 172.6 | 21.5 | 171.9 | 31.4 |
|  | 174.6 | −9.3 | 174.6 | 0.6 | 174.5 | 10.6 | 174.3 | 20.5 | 173.8 | 30.5 |
|  | 176.3 | −10.1 | 176.3 | −0.2 | 176.3 | 9.8 | 176.1 | 19.8 | 175.8 | 29.8 |
|  | 178.1 | −10.6 | 178.1 | −0.6 | 178.1 | 9.4 | 178.0 | 19.3 | 177.9 | 29.3 |
|  | 180.0 | −10.8 | 180.0 | −0.8 | 180.0 | 9.2 | 180.0 | 19.2 | 180.0 | 29.2 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 60.8 | 180.0 | 70.8 | 180.0 | 80.8 | 0 | 89.2 |  | 79.2 |
|  | 176.2 | 60.6 | 174.4 | 70.6 | 168.7 | 80.5 | 71.1 | 88.0 |  | 79.2 |
|  | 172.6 | 60.0 | 169.3 | 69.8 | 159.4 | 79.5 | 87.5 | 86.3 |  | 79.2 |
|  | 169.5 | 59.0 | 165.0 | 68.7 | 152.9 | 78.1 | 96.6 | 84.6 |  | 79.2 |
|  | 167.0 | 57.6 | 161.8 | 67.3 | 149.1 | 76.4 | 103.6 | 82.9 |  | 79.2 |
| Coordinates | 165.1 | 56.1 | 159.6 | 65.6 | 147.2 | 74.6 | 109.9 | 81.2 | All | 79.2 |
| for | 163.8 | 54.4 | 158.4 | 63.8 | 146.8 | 72.8 | 115.8 | 79.6 | longitudes | 79.2 |
| plotting | 163.2 | 52.5 | 158.0 | 62.0 | 147.4 | 70.9 | 121.4 | 78.1 |  | 79.2 |
| contours | 163.1 | 50.7 | 158.3 | 60.1 | 148.9 | 69.1 | 126.9 | 76.7 |  | 79.2 |
|  | 163.5 | 48.8 | 159.1 | 58.3 | 150.8 | 67.4 | 132.3 | 75.3 |  | 79.2 |
|  | 164.3 | 47.0 | 160.4 | 56.6 | 153.3 | 65.8 | 137.7 | 74.1 |  | 79.2 |
|  | 165.5 | 45.3 | 162.1 | 54.9 | 156.0 | 64.3 | 143.0 | 73.0 |  | 79.2 |
|  | 167.0 | 43.8 | 164.2 | 53.5 | 159.1 | 63.0 | 148.3 | 72.0 |  | 79.2 |
|  | 168.3 | 42.5 | 166.4 | 52.2 | 162.3 | 61.9 | 153.6 | 71.2 |  | 79.2 |
|  | 170.3 | 41.3 | 168.9 | 51.2 | 165.7 | 60.9 | 158.9 | 70.5 |  | 79.2 |
|  | 172.9 | 40.4 | 171.6 | 50.3 | 169.1 | 60.2 | 164.2 | 69.9 |  | 79.2 |
|  | 175.8 | 39.7 | 174.3 | 49.7 | 172.7 | 59.6 | 169.4 | 69.5 |  | 79.2 |
|  | 177.6 | 39.3 | 177.1 | 49.3 | 176.3 | 59.3 | 174.7 | 69.3 |  | 79.2 |
|  | 180.0 | 39.2 | 180.0 | 49.2 | 180.0 | 59.2 | 180.0 | 69.2 |  | 79.2 |

27/50	4.7 MHz night and 10.0 MHz day

Data for plotting 5 500 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 49.5 | 180.0 | 59.5 | 180.0 | 69.5 | 180.0 | 79.5 | 178.7 | 89.5 |
|  | 168.5 | 48.5 | 165.5 | 58.2 | 159.6 | 67.8 | 144.9 | 76.7 | 97.0 | 82.4 |
|  | 158.2 | 45.6 | 153.2 | 54.7 | 144.6 | 63.3 | 128.3 | 70.7 | 98.4 | 74.8 |
|  | 149.7 | 41.2 | 144.1 | 49.6 | 135.4 | 57.2 | 121.5 | 63.5 | 101.0 | 67.2 |
|  | 143.0 | 35.6 | 137.8 | 43.3 | 130.1 | 50.3 | 119.0 | 56.0 | 104.1 | 59.7 |
| Coordinates | 138.1 | 29.3 | 133.6 | 36.5 | 127.3 | 43.0 | 118.6 | 48.4 | 107.5 | 52.4 |
| for | 134.6 | 22.3 | 131.1 | 29.2 | 126.1 | 35.4 | 119.5 | 40.8 | 111.0 | 45.1 |
| plotting | 132.3 | 15.1 | 129.8 | 21.6 | 126.1 | 27.8 | 121.2 | 33.4 | 114.8 | 38.1 |
| contours | 130.9 | 7.6 | 129.5 | 14.1 | 127.0 | 20.3 | 123.5 | 26.0 | 118.9 | 31.2 |
|  | 130.5 | 0.0 | 130.1 | 6.5 | 128.7 | 12.8 | 126.5 | 18.9 | 123.2 | 24.7 |
|  | 130.9 | −7.6 | 131.5 | −1.0 | 131.2 | 5.6 | 130.0 | 12.1 | 127.9 | 18.4 |
|  | 132.3 | −15.1 | 133.8 | −8.2 | 134.4 | −1.3 | 134.1 | 5.7 | 132.9 | 12.6 |
|  | 134.6 | −22.3 | 137.0 | −15.2 | 138.3 | −7.8 | 138.8 | −0.3 | 138.4 | 7.3 |
|  | 138.1 | −29.3 | 141.2 | −21.6 | 143.2 | −13.7 | 144.2 | −5.7 | 144.3 | 2.5 |
|  | 143.0 | −35.6 | 146.6 | −27.4 | 148.9 | −19.0 | 150.2 | −10.4 | 150.7 | −1.6 |
|  | 149.7 | −41.2 | 153.2 | −32.4 | 155.5 | −23.4 | 156.9 | −14.2 | 157.6 | −5.0 |
|  | 158.2 | −45.6 | 161.2 | −36.2 | 163.1 | −26.7 | 164.2 | −17.1 | 164.8 | −7.5 |
|  | 168.5 | −48.5 | 170.3 | −38.7 | 171.3 | −28.8 | 172.0 | −18.9 | 172.3 | −9.0 |
|  | 180.0 | −49.5 | 180.0 | −39.5 | 180.0 | −29.5 | 180.0 | −19.5 | 180.0 | −9.5 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 0 | 80.5 | 0 | 70.5 | 0 | 60.5 | 0 | 50.5 |  | 40.5 |
|  | 40.2 | 78.2 | 22.2 | 69.5 | 15.3 | 60.0 | 11.9 | 50.3 |  | 40.5 |
|  | 63.5 | 73.1 | 41.5 | 66.9 | 30.1 | 58.7 | 23.8 | 49.8 |  | 40.5 |
|  | 77.1 | 67.0 | 57.1 | 63.1 | 43.8 | 56.7 | 35.4 | 48.9 |  | 40.5 |
|  | 86.6 | 60.7 | 69.8 | 58.6 | 56.4 | 54.0 | 46.7 | 47.8 |  | 40.5 |
| Coordinates | 94.2 | 54.3 | 80.4 | 53.8 | 67.8 | 51.0 | 57.7 | 46.4 | All | 40.5 |
| for | 100.8 | 47.9 | 89.6 | 48.8 | 78.4 | 47.8 | 68.3 | 44.9 | longitudes | 40.5 |
| plotting | 107.0 | 41.7 | 97.9 | 43.8 | 88.2 | 44.4 | 78.7 | 43.2 |  | 40.5 |
| contours | 112.9 | 35.6 | 105.7 | 38.9 | 97.5 | 41.0 | 88.7 | 41.5 |  | 40.5 |
|  | 118.8 | 29.8 | 113.1 | 34.2 | 106.3 | 37.6 | 98.4 | 39.8 |  | 40.5 |
|  | 124.7 | 24.4 | 120.4 | 29.8 | 114.8 | 34.4 | 108.0 | 38.1 |  | 40.5 |
|  | 130.8 | 19.3 | 127.6 | 25.6 | 123.1 | 31.4 | 117.3 | 36.5 |  | 40.5 |
|  | 137.1 | 14.7 | 134.8 | 21.9 | 131.3 | 28.7 | 126.5 | 35.0 |  | 40.5 |
|  | 143.7 | 10.6 | 142.1 | 18.5 | 139.5 | 26.3 | 135.6 | 33.7 |  | 40.5 |
|  | 150.5 | 7.1 | 149.5 | 15.7 | 147.6 | 24.3 | 144.5 | 32.6 |  | 40.5 |
|  | 157.6 | 4.3 | 157.0 | 13.5 | 155.7 | 22.6 | 153.5 | 31.7 |  | 40.5 |
|  | 164.9 | 2.2 | 164.6 | 11.8 | 163.8 | 21.5 | 162.3 | 31.0 |  | 40.5 |
|  | 172.4 | 0.9 | 172.3 | 10.8 | 171.9 | 20.7 | 171.2 | 30.6 |  | 40.5 |
|  | 180.0 | 0.5 | 180.0 | 10.5 | 180.0 | 20.5 | 180.0 | 30.5 |  | 40.5 |

27/51	5.6 MHz day

Data for plotting 1 500 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 13.5 | 180.0 | 23.5 | 180.0 | 33.5 | 180.0 | 43.5 | 180.0 | 53.5 |
|  | 177.6 | 13.3 | 177.5 | 23.3 | 177.2 | 33.3 | 176.8 | 43.3 | 176.1 | 53.2 |
|  | 175.3 | 12.7 | 175.0 | 22.6 | 174.6 | 32.6 | 173.8 | 42.5 | 172.5 | 52.5 |
|  | 173.2 | 11.7 | 172.8 | 21.6 | 172.1 | 31.5 | 171.0 | 41.4 | 169.3 | 51.3 |
|  | 171.2 | 10.3 | 170.8 | 20.2 | 170.0 | 30.0 | 168.7 | 39.9 | 166.6 | 49.6 |
| Coordinates | 169.6 | 8.6 | 169.1 | 18.5 | 168.3 | 28.3 | 166.9 | 38.0 | 164.6 | 47.7 |
| for | 168.3 | 6.7 | 167.8 | 16.5 | 167.0 | 26.2 | 165.5 | 36.0 | 163.2 | 45.6 |
| plotting | 167.3 | 4.6 | 166.9 | 14.3 | 166.1 | 24.1 | 164.7 | 33.7 | 162.4 | 43.3 |
| contours | 166.7 | 2.3 | 166.4 | 12.1 | 165.7 | 21.8 | 164.4 | 31.4 | 162.3 | 41.0 |
|  | 166.5 | 0.0 | 166.3 | 9.7 | 165.7 | 19.4 | 164.5 | 29.1 | 162.6 | 38.7 |
|  | 166.7 | −2.3 | 166.6 | 7.4 | 166.1 | 17.1 | 165.1 | 26.8 | 163.4 | 36.4 |
|  | 167.3 | −4.6 | 167.3 | 5.2 | 166.9 | 14.9 | 166.0 | 24.6 | 164.6 | 34.3 |
|  | 168.3 | −6.7 | 168.3 | 3.1 | 168.0 | 12.9 | 167.3 | 22.6 | 166.1 | 32.4 |
|  | 169.6 | −8.6 | 169.7 | 1.2 | 169.5 | 11.0 | 169.0 | 20.9 | 168.0 | 30.7 |
|  | 171.2 | −10.3 | 171.4 | −0.4 | 171.2 | 9.5 | 170.8 | 19.3 | 170.1 | 29.2 |
|  | 173.2 | −11.7 | 173.3 | −1.7 | 173.2 | 8.2 | 172.9 | 18.1 | 172.4 | 28.0 |
|  | 175.3 | −12.7 | 175.4 | −2.7 | 175.4 | 7.3 | 175.2 | 17.2 | 174.8 | 27.2 |
|  | 177.6 | −13.3 | 177.7 | −3.3 | 177.7 | 6.7 | 177.6 | 16.7 | 177.4 | 26.7 |
|  | 180.0 | −13.5 | 180.0 | −3.5 | 180.0 | 6.5 | 180.0 | 16.5 | 180.0 | 26.5 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 63.5 | 180.0 | 73.5 | 180.0 | 83.5 | 0 | 86.5 |  | 76.5 |
|  | 174.8 | 63.2 | 172.0 | 73.1 | 160.8 | 82.9 | 35.2 | 86.0 |  | 76.5 |
|  | 170.1 | 62.4 | 164.9 | 72.1 | 147.7 | 81.4 | 59.4 | 84.7 |  | 76.5 |
|  | 166.1 | 61.0 | 159.4 | 70.6 | 140.7 | 79.4 | 75.5 | 83.1 |  | 76.5 |
|  | 162.9 | 59.3 | 155.6 | 68.7 | 137.6 | 77.1 | 87.2 | 81.4 |  | 76.5 |
| Coordinates | 160.7 | 57.3 | 153.3 | 66.5 | 137.0 | 74.8 | 96.7 | 79.6 | All | 76.5 |
| for | 159.3 | 55.1 | 152.3 | 64.2 | 137.8 | 72.5 | 104.9 | 77.9 | longitudes | 76.5 |
| plotting | 158.7 | 52.8 | 152.3 | 61.9 | 139.6 | 70.2 | 112.4 | 76.3 |  | 76.5 |
| contours | 158.8 | 50.4 | 153.0 | 59.6 | 142.0 | 68.1 | 119.3 | 74.7 |  | 76.5 |
|  | 159.5 | 48.1 | 154.4 | 57.4 | 144.9 | 66.0 | 125.9 | 73.3 |  | 76.5 |
|  | 160.7 | 46.0 | 156.2 | 55.3 | 148.2 | 64.1 | 132.2 | 71.9 |  | 76.5 |
|  | 162.3 | 43.9 | 158.4 | 53.3 | 151.7 | 62.4 | 138.4 | 70.7 |  | 76.5 |
|  | 164.2 | 42.1 | 161.0 | 51.6 | 155.4 | 60.9 | 144.5 | 69.6 |  | 76.5 |
|  | 166.4 | 40.4 | 163.8 | 50.1 | 159.3 | 59.6 | 150.5 | 68.7 |  | 76.5 |
|  | 168.9 | 39.0 | 166.8 | 48.8 | 163.3 | 58.5 | 156.5 | 67.9 |  | 76.5 |
|  | 171.5 | 37.9 | 170.0 | 47.8 | 167.4 | 57.6 | 162.4 | 67.3 |  | 76.5 |
|  | 174.3 | 37.1 | 173.3 | 47.1 | 171.6 | 57.0 | 168.3 | 66.9 |  | 76.5 |
|  | 177.1 | 36.7 | 176.6 | 46.6 | 175.8 | 56.6 | 174.1 | 66.6 |  | 76.5 |
|  | 180.0 | 36.5 | 180.0 | 46.5 | 180.0 | 56.5 | 180.0 | 66.5 |  | 76.5 |

27/52	5.6 and 6.6 MHz night

Data for plotting 6 500 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 58.5 | 180.0 | 68.5 | 180.0 | 78.5 | 180.0 | 88.5 | 0 | 81.5 |
|  | 164.2 | 57.1 | 158.1 | 66.6 | 144.0 | 75.4 | 102.4 | 81.3 | 46.7 | 78.3 |
|  | 150.8 | 53.2 | 142.2 | 61.6 | 126.6 | 68.7 | 100.1 | 72.8 | 68.5 | 71.7 |
|  | 140.8 | 47.6 | 132.2 | 54.9 | 119.2 | 60.8 | 101.1 | 64.3 | 80.1 | 64.4 |
|  | 133.6 | 40.8 | 126.2 | 47.2 | 116.0 | 52.4 | 102.9 | 55.8 | 88.0 | 56.7 |
| Coordinates | 128.7 | 33.2 | 122.7 | 39.1 | 114.9 | 43.9 | 105.3 | 47.4 | 94.2 | 49.1 |
| for | 125.3 | 25.2 | 120.8 | 30.7 | 115.1 | 35.4 | 108.0 | 39.1 | 99.7 | 41.5 |
| plotting | 123.1 | 17.0 | 120.1 | 22.2 | 116.0 | 26.9 | 110.9 | 30.9 | 104.9 | 34.0 |
| contours | 121.9 | 8.5 | 120.2 | 13.7 | 117.7 | 18.5 | 114.3 | 22.9 | 110.0 | 26.7 |
|  | 121.5 | 0.0 | 121.1 | 5.2 | 119.9 | 10.3 | 118.0 | 15.1 | 115.1 | 19.6 |
|  | 121.9 | −8.5 | 122.8 | −3.2 | 122.8 | 2.3 | 122.1 | 7.6 | 120.5 | 12.9 |
|  | 123.1 | −17.0 | 125.2 | −11.3 | 126.4 | −5.5 | 126.8 | 0.5 | 126.3 | 6.5 |
|  | 125.3 | −25.2 | 128.6 | −19.2 | 130.8 | −12.8 | 132.0 | −6.2 | 132.4 | 0.5 |
|  | 128.7 | −33.2 | 133.0 | −26.7 | 136.1 | −19.7 | 138.0 | −12.3 | 139.0 | −4.8 |
|  | 133.6 | −40.8 | 138.9 | −33.5 | 142.5 | −25.8 | 144.9 | −17.7 | 146.2 | −9.5 |
|  | 140.8 | −47.6 | 146.4 | −39.5 | 150.2 | −31.0 | 152.6 | −22.2 | 154.0 | −13.3 |
|  | 150.8 | −53.2 | 156.0 | −44.3 | 159.1 | −35.0 | 161.1 | −25.6 | 162.3 | −16.1 |
|  | 164.2 | −57.1 | 167.4 | −47.4 | 169.2 | −37.6 | 170.4 | −27.8 | 171.0 | −17.9 |
|  | 180.0 | −58.5 | 180.0 | −48.5 | 180.0 | −38.5 | 180.0 | −28.5 | 180.0 | −18.5 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 0 | 71.5 | 0 | 61.5 | 0 | 51.5 | 0 | 41.5 |  | 31.5 |
|  | 25.7 | 70.1 | 17.6 | 60.7 | 13.6 | 51.1 | 11.4 | 41.3 |  | 31.5 |
|  | 46.4 | 66.2 | 34.0 | 58.6 | 26.9 | 49.9 | 22.7 | 40.8 |  | 31.5 |
|  | 61.7 | 61.0 | 43.4 | 55.3 | 39.6 | 48.0 | 33.8 | 40.0 |  | 31.5 |
|  | 73.3 | 55.1 | 61.0 | 51.2 | 51.6 | 45.6 | 44.8 | 38.9 |  | 31.5 |
| Coordinates | 82.7 | 48.8 | 71.9 | 46.6 | 62.8 | 42.7 | 55.5 | 37.6 | All | 31.5 |
| for | 90.7 | 42.4 | 81.7 | 41.7 | 73.8 | 39.6 | 66.0 | 36.1 | longitudes | 31.5 |
| plotting | 98.0 | 36.0 | 90.6 | 36.7 | 83.2 | 36.2 | 76.2 | 34.4 |  | 31.5 |
| contours | 104.8 | 29.7 | 99.0 | 31.8 | 92.7 | 32.8 | 86.2 | 32.7 |  | 31.5 |
|  | 111.6 | 23.6 | 107.0 | 26.9 | 101.8 | 29.4 | 96.1 | 31.0 |  | 31.5 |
|  | 115.1 | 17.8 | 114.9 | 22.2 | 110.7 | 26.1 | 105.7 | 29.3 |  | 31.5 |
|  | 124.9 | 12.3 | 122.7 | 17.9 | 119.5 | 23.0 | 115.3 | 27.6 |  | 31.5 |
|  | 131.8 | 7.3 | 130.5 | 13.8 | 128.1 | 20.2 | 124.7 | 26.1 |  | 31.5 |
|  | 139.2 | 2.7 | 138.4 | 10.3 | 136.7 | 17.7 | 134.0 | 24.9 |  | 31.5 |
|  | 146.8 | −1.1 | 146.5 | 7.2 | 145.3 | 15.5 | 143.3 | 23.6 |  | 31.5 |
|  | 154.7 | −4.3 | 154.7 | 4.8 | 154.0 | 13.8 | 152.5 | 22.7 |  | 31.5 |
|  | 162.9 | −6.6 | 163.0 | 3.0 | 162.6 | 12.5 | 161.7 | 22.1 |  | 31.5 |
|  | 171.4 | −8.0 | 171.5 | 1.9 | 171.3 | 11.8 | 170.8 | 21.6 |  | 31.5 |
|  | 180.0 | −8.5 | 180.0 | 1.5 | 180.0 | 11.5 | 180.0 | 21.5 |  | 31.5 |

27/53	6.6 MHz day

Data for plotting 1 900 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 17.1 | 180.0 | 27.1 | 180.0 | 37.1 | 180.0 | 47.1 | 180.0 | 57.1 |
|  | 176.9 | 16.8 | 176.7 | 26.8 | 176.3 | 36.8 | 175.7 | 46.8 | 174.7 | 56.7 |
|  | 174.0 | 16.0 | 173.6 | 26.0 | 172.9 | 35.9 | 171.7 | 45.8 | 169.7 | 55.7 |
|  | 171.3 | 14.8 | 170.7 | 24.6 | 169.7 | 34.5 | 168.1 | 44.3 | 165.5 | 54.0 |
|  | 168.8 | 13.0 | 168.2 | 22.8 | 167.0 | 32.6 | 165.2 | 42.3 | 162.2 | 51.9 |
| Coordinates | 166.7 | 10.9 | 166.1 | 20.6 | 164.9 | 30.3 | 162.9 | 39.9 | 159.8 | 49.4 |
| for | 165.1 | 8.5 | 164.5 | 18.1 | 163.3 | 27.7 | 161.3 | 37.2 | 158.2 | 46.6 |
| plotting | 163.9 | 5.8 | 163.3 | 15.4 | 162.3 | 24.9 | 160.4 | 34.4 | 157.5 | 43.7 |
| contours | 163.1 | 2.9 | 162.7 | 12.5 | 161.8 | 22.0 | 160.2 | 31.5 | 157.5 | 40.8 |
|  | 162.9 | 0.0 | 162.7 | 9.6 | 161.9 | 19.1 | 160.4 | 28.5 | 158.1 | 37.9 |
|  | 163.1 | −2.9 | 163.1 | 6.6 | 162.4 | 16.2 | 161.3 | 25.7 | 159.3 | 35.1 |
|  | 163.9 | −5.8 | 163.9 | 3.8 | 163.5 | 13.4 | 162.5 | 23.0 | 160.9 | 32.5 |
|  | 165.1 | −8.5 | 165.2 | 1.2 | 165.0 | 10.9 | 164.2 | 20.5 | 162.9 | 30.1 |
|  | 166.7 | −10.9 | 167.0 | −1.2 | 166.8 | 8.6 | 166.3 | 18.3 | 165.2 | 28.0 |
|  | 168.8 | −13.0 | 169.1 | −3.2 | 169.0 | 6.6 | 168.6 | 16.4 | 167.8 | 26.2 |
|  | 171.3 | −14.8 | 171.5 | −4.9 | 171.5 | 5.0 | 171.2 | 14.9 | 170.7 | 24.8 |
|  | 174.0 | −16.0 | 174.2 | −6.1 | 174.2 | 3.9 | 174.1 | 13.8 | 173.7 | 23.7 |
|  | 176.9 | −16.8 | 177.1 | −6.8 | 177.1 | 3.1 | 177.0 | 13.1 | 176.8 | 23.1 |
|  | 180.0 | −17.1 | 180.0 | −7.1 | 180.0 | 2.9 | 180.0 | 12.9 | 180.0 | 22.9 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 67.1 | 180.0 | 77.1 | 180.0 | 87.1 | 0 | 82.9 |  | 72.9 |
|  | 172.6 | 66.7 | 167.3 | 76.5 | 137.0 | 85.7 | 23.2 | 82.5 |  | 72.9 |
|  | 166.0 | 65.5 | 157.1 | 75.0 | 123.8 | 83.1 | 43.5 | 81.6 |  | 72.9 |
|  | 160.7 | 63.6 | 150.3 | 72.8 | 120.8 | 80.1 | 60.0 | 80.2 |  | 72.9 |
|  | 156.8 | 61.3 | 146.2 | 70.1 | 121.4 | 77.2 | 73.5 | 78.6 |  | 72.9 |
| Coordinates | 154.4 | 58.6 | 144.4 | 67.3 | 123.5 | 74.3 | 84.9 | 76.9 | All | 72.9 |
| for | 153.1 | 55.8 | 144.0 | 64.3 | 126.5 | 71.5 | 94.8 | 75.2 | longitudes | 72.9 |
| plotting | 152.8 | 52.8 | 144.7 | 61.4 | 130.1 | 68.8 | 103.6 | 73.5 |  | 72.9 |
| contours | 153.3 | 49.9 | 146.3 | 58.6 | 133.9 | 66.3 | 111.8 | 71.8 |  | 72.9 |
|  | 154.4 | 47.1 | 148.4 | 55.9 | 138.0 | 63.9 | 119.4 | 70.3 |  | 72.9 |
|  | 156.1 | 44.4 | 151.0 | 53.3 | 142.3 | 61.7 | 126.8 | 68.8 |  | 72.9 |
|  | 158.2 | 41.9 | 153.9 | 51.0 | 146.7 | 59.7 | 133.8 | 67.5 |  | 72.9 |
|  | 160.7 | 39.6 | 157.2 | 49.0 | 151.3 | 58.0 | 140.7 | 66.3 |  | 72.9 |
|  | 163.5 | 37.6 | 160.7 | 47.2 | 155.9 | 56.5 | 147.4 | 65.3 |  | 72.9 |
|  | 166.5 | 36.0 | 164.3 | 45.7 | 160.7 | 55.2 | 154.0 | 64.4 |  | 72.9 |
|  | 169.7 | 34.6 | 168.1 | 44.5 | 165.4 | 54.2 | 160.6 | 63.8 |  | 72.9 |
|  | 173.1 | 33.7 | 172.0 | 43.6 | 170.3 | 53.5 | 167.1 | 63.3 |  | 72.9 |
|  | 176.5 | 33.1 | 176.0 | 43.1 | 175.1 | 53.0 | 173.5 | 63.0 |  | 72.9 |
|  | 180.0 | 32.9 | 180.0 | 42.9 | 180.0 | 52.9 | 180.0 | 62.9 |  | 72.9 |

27/54	9.0 MHz day

Data for plotting 3 800 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 34.2 | 180.0 | 44.2 | 180.0 | 54.2 | 180.0 | 64.2 | 180.0 | 74.2 |
|  | 173.3 | 33.6 | 172.3 | 43.5 | 170.6 | 53.4 | 167.5 | 63.2 | 160.6 | 72.9 |
|  | 166.9 | 31.9 | 165.1 | 41.6 | 162.1 | 51.2 | 157.0 | 60.6 | 146.8 | 69.4 |
|  | 161.2 | 29.1 | 158.9 | 38.5 | 155.3 | 47.8 | 149.3 | 56.6 | 138.8 | 64.8 |
|  | 156.4 | 25.5 | 154.0 | 34.6 | 150.2 | 43.4 | 144.2 | 51.9 | 134.6 | 59.5 |
| Coordinates | 152.5 | 21.2 | 150.2 | 30.0 | 146.6 | 38.5 | 141.2 | 46.6 | 133.0 | 53.9 |
| for | 149.5 | 16.3 | 147.6 | 24.9 | 144.4 | 33.2 | 139.8 | 41.1 | 132.9 | 48.3 |
| plotting | 147.4 | 11.1 | 145.9 | 19.4 | 143.4 | 27.6 | 139.6 | 35.5 | 134.0 | 42.8 |
| contours | 146.2 | 5.6 | 145.2 | 13.9 | 143.3 | 22.0 | 140.3 | 29.9 | 135.9 | 37.3 |
|  | 145.8 | 0.0 | 145.4 | 8.3 | 144.1 | 16.4 | 141.9 | 24.4 | 138.4 | 32.1 |
|  | 146.2 | −5.6 | 146.3 | 2.7 | 145.7 | 11.0 | 144.1 | 19.2 | 141.5 | 27.2 |
|  | 147.4 | −11.1 | 148.1 | −2.6 | 147.9 | 5.9 | 147.0 | 14.3 | 145.1 | 22.6 |
|  | 149.5 | −16.3 | 150.6 | −7.7 | 150.9 | 1.1 | 150.4 | 9.8 | 149.1 | 18.4 |
|  | 152.5 | −21.2 | 153.9 | −12.3 | 154.5 | −3.2 | 154.4 | 5.8 | 153.6 | 14.8 |
|  | 156.4 | −25.5 | 157.9 | −16.3 | 158.7 | −7.0 | 158.8 | 2.3 | 158.4 | 11.6 |
|  | 161.2 | −29.1 | 162.6 | −19.6 | 163.4 | −10.1 | 163.7 | −0.5 | 163.5 | 9.1 |
|  | 166.9 | −31.9 | 168.0 | −22.1 | 168.7 | −12.3 | 168.9 | −2.5 | 168.8 | 7.3 |
|  | 173.3 | −33.6 | 173.9 | −23.7 | 174.2 | −13.7 | 174.4 | −3.8 | 174.4 | 6.2 |
|  | 180.0 | −34.2 | 180.0 | −24.2 | 180.0 | −14.2 | 180.0 | −4.2 | 180.0 | 5.8 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 84.2 | 0 | 85.8 | 0 | 75.8 | 0 | 65.8 |  | 55.8 |
|  | 137.8 | 81.6 | 56.0 | 83.2 | 22.4 | 75.1 | 13.7 | 65.6 |  | 55.8 |
|  | 123.5 | 76.7 | 77.1 | 78.6 | 42.0 | 73.3 | 27.0 | 65.0 |  | 55.8 |
|  | 119.5 | 71.2 | 88.4 | 73.7 | 58.2 | 70.7 | 39.9 | 64.0 |  | 55.8 |
|  | 119.2 | 65.6 | 96.4 | 68.7 | 71.4 | 67.6 | 52.2 | 62.8 |  | 55.8 |
| Coordinates | 120.6 | 60.0 | 103.2 | 63.8 | 82.5 | 64.3 | 63.8 | 61.3 | All | 55.8 |
| for | 123.0 | 54.5 | 109.3 | 59.0 | 92.2 | 60.8 | 74.7 | 59.7 | longitudes | 55.8 |
| plotting | 126.0 | 49.2 | 115.1 | 54.3 | 101.0 | 57.5 | 85.1 | 58.0 |  | 55.8 |
| contours | 129.5 | 44.1 | 120.7 | 49.9 | 109.1 | 54.2 | 94.9 | 56.2 |  | 55.8 |
|  | 133.4 | 39.3 | 126.3 | 45.7 | 116.7 | 51.0 | 104.3 | 54.5 |  | 55.8 |
|  | 137.6 | 34.8 | 132.0 | 41.9 | 124.1 | 48.1 | 113.4 | 52.9 |  | 55.8 |
|  | 142.1 | 30.7 | 137.7 | 38.3 | 131.3 | 45.4 | 122.2 | 51.4 |  | 55.8 |
|  | 146.9 | 26.9 | 143.5 | 35.2 | 138.3 | 42.9 | 130.8 | 50.0 |  | 55.8 |
|  | 152.0 | 23.7 | 149.3 | 32.4 | 145.3 | 40.8 | 139.2 | 48.7 |  | 55.8 |
|  | 157.2 | 20.9 | 155.3 | 30.1 | 152.3 | 39.0 | 147.5 | 47.7 |  | 55.8 |
|  | 162.7 | 18.7 | 161.4 | 28.2 | 159.2 | 37.6 | 155.7 | 46.9 |  | 55.8 |
|  | 168.4 | 17.1 | 167.6 | 26.9 | 166.1 | 36.6 | 163.8 | 46.3 |  | 55.8 |
|  | 174.2 | 16.1 | 173.3 | 26.1 | 173.1 | 36.0 | 171.9 | 45.9 |  | 55.8 |
|  | 180.0 | 15.8 | 180.0 | 25.8 | 180.0 | 35.8 | 180.0 | 45.8 |  | 55.8 |

27/55	11.3 MHz day

Data for plotting 6 000 km interference contours

| Latitude | 00° |  | 10° |  | 20° |  | 30° |  | 40° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 180.0 | 54.0 | 180.0 | 64.0 | 180.0 | 74.0 | 180.0 | 84.0 | 0 | 86.0 |
|  | 166.6 | 52.8 | 162.3 | 62.5 | 153.3 | 71.8 | 128.2 | 79.7 | 66.2 | 81.2 |
|  | 154.8 | 49.5 | 148.2 | 58.3 | 136.6 | 66.3 | 115.0 | 72.2 | 82.1 | 73.8 |
|  | 145.5 | 44.5 | 138.5 | 52.4 | 127.7 | 59.3 | 111.4 | 64.2 | 90.0 | 66.1 |
|  | 138.5 | 38.3 | 132.2 | 45.4 | 123.2 | 51.6 | 111.0 | 58.2 | 95.7 | 58.5 |
| Coordinates | 133.5 | 31.3 | 128.2 | 37.9 | 121.1 | 43.6 | 111.9 | 48.1 | 100.6 | 50.9 |
| for | 130.0 | 23.9 | 126.0 | 30.0 | 120.6 | 35.5 | 113.6 | 40.1 | 105.2 | 43.4 |
| plotting | 127.7 | 16.1 | 124.9 | 22.0 | 121.1 | 27.5 | 116.0 | 32.2 | 109.7 | 36.1 |
| contours | 126.4 | 8.1 | 124.8 | 13.9 | 122.3 | 19.5 | 118.8 | 24.6 | 114.3 | 29.0 |
|  | 126.0 | 0.0 | 125.6 | 5.9 | 124.3 | 11.6 | 122.2 | 17.1 | 119.1 | 22.2 |
|  | 126.4 | −8.1 | 127.1 | −2.1 | 127.0 | 4.0 | 126.0 | 9.9 | 124.2 | 15.7 |
|  | 127.7 | −16.1 | 129.5 | −9.8 | 130.4 | −3.4 | 130.4 | 3.1 | 129.6 | 9.5 |
|  | 130.0 | −23.9 | 132.8 | −17.2 | 134.6 | −10.3 | 135.4 | −3.2 | 135.4 | 3.9 |
|  | 133.5 | −31.3 | 137.2 | −24.2 | 139.7 | −16.7 | 141.1 | −9.0 | 141.7 | −1.2 |
|  | 138.5 | −38.3 | 142.9 | −30.5 | 145.8 | −22.4 | 147.6 | −14.1 | 148.5 | −5.6 |
|  | 145.5 | −44.5 | 150.0 | −36.0 | 152.9 | −27.2 | 154.8 | −18.2 | 155.6 | −9.1 |
|  | 154.8 | −49.5 | 158.7 | −40.3 | 161.2 | −30.9 | 162.7 | −21.4 | 163.6 | −11.8 |
|  | 166.6 | −52.8 | 163.9 | −43.0 | 170.3 | −33.2 | 171.2 | −23.3 | 171.7 | −13.4 |
|  | 180.0 | −54.0 | 180.0 | −44.0 | 180.0 | −34.0 | 180.0 | −24.0 | 180.0 | −14.0 |

| Latitude | 50° |  | 60° |  | 70° |  | 80° |  | 90° |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. | Long. | Lat. |
|  | 0 | 76.0 | 0 | 66.0 | 0 | 56.0 | 0 | 46.0 |  | 36.0 |
|  | 31.1 | 74.2 | 19.5 | 65.1 | 14.4 | 55.6 | 11.6 | 45.8 |  | 36.0 |
|  | 53.5 | 69.9 | 37.2 | 62.8 | 28.3 | 54.3 | 23.2 | 45.3 |  | 36.0 |
|  | 68.6 | 64.2 | 52.3 | 59.2 | 41.5 | 52.4 | 34.5 | 44.5 |  | 36.0 |
|  | 79.4 | 58.1 | 65.0 | 55.0 | 53.7 | 49.8 | 45.7 | 43.4 |  | 36.0 |
| Coordinates | 88.1 | 51.7 | 75.8 | 50.3 | 65.1 | 46.9 | 56.5 | 42.0 | All | 36.0 |
| for | 95.5 | 45.3 | 85.4 | 45.3 | 75.7 | 43.7 | 67.1 | 40.5 | longitudes | 36.0 |
| plotting | 102.3 | 38.9 | 94.1 | 40.3 | 85.6 | 40.3 | 77.4 | 38.3 |  | 36.0 |
| contours | 108.7 | 32.7 | 102.2 | 35.4 | 95.0 | 36.9 | 87.4 | 37.1 |  | 36.0 |
|  | 115.0 | 26.3 | 110.0 | 30.6 | 104.0 | 33.5 | 97.2 | 35.4 |  | 36.0 |
|  | 121.4 | 21.1 | 117.5 | 26.0 | 112.7 | 30.3 | 106.8 | 33.7 |  | 36.0 |
|  | 127.8 | 15.8 | 125.1 | 21.8 | 121.2 | 27.2 | 116.2 | 32.1 |  | 36.0 |
|  | 134.5 | 11.0 | 132.6 | 17.9 | 129.7 | 24.5 | 125.5 | 30.6 |  | 36.0 |
|  | 141.4 | 6.7 | 140.2 | 14.4 | 138.1 | 22.0 | 134.7 | 29.2 |  | 36.0 |
|  | 148.6 | 3.0 | 148.0 | 11.5 | 146.4 | 19.9 | 143.9 | 28.1 |  | 36.0 |
|  | 156.1 | −0.0 | 155.8 | 9.1 | 154.8 | 18.2 | 152.9 | 27.2 |  | 36.0 |
|  | 163.9 | −2.2 | 163.8 | 7.4 | 163.2 | 17.0 | 162.0 | 26.5 |  | 36.0 |
|  | 171.0 | −3.5 | 171.9 | 6.4 | 171.6 | 16.3 | 171.0 | 26.1 |  | 36.0 |
|  | 180.0 | −4.0 | 180.0 | 6.0 | 180.0 | 16.0 | 180.0 | 26.0 |  | 36.0 |

C  –  Classes of emission and power

# 1	Classes of emission

27/56		In the aeronautical mobile (R) service the use of emissions listed below is permissible subject to compliance with the special provisions applicable to each case and provided that such use does not cause harmful interference to other users of the channel concerned.

## 27/57	1.1	Telephony – amplitude modulation:

double sideband	A3E*

single sideband, full carrier	H3E*

single sideband, suppressed carrier	J3E

## 1.2	Telegraphy (including automatic data transmission)

### 27/58	1.2.1	Amplitude modulation:

telegraphy without the use of a modulating audio frequency (by on-off keying)	A1A, A1B**

–	telegraphy by the on-off keying of an amplitude modulating audio frequency or audio frequencies or by the on-off keying of the modulated emission and including selective calling, single sideband, full carrier	H2B

–	multichannel voice frequency telegraphy, single sideband, suppressed carrier	J7B

–	other transmissions such as automatic data transmission, single sideband, suppressed carrier	JXX

### 27/59	1.2.2	Frequency modulation:

–	telegraphy by frequency shift keying without the use of a modulating audio frequency, one of two frequencies being emitted at any instant	F1B**

# 2	Power

27/60	2.1	Unless otherwise specified in Part II of this Appendix, the peak envelope powers supplied to the antenna transmission line shall not exceed the maximum values indicated in the Table below; the corresponding peak effective radiated powers being assumed to be equal to two-thirds of these values.

| Class of emission |  | Stations | Maximum peak envelope power |
| --- | --- | --- | --- |
| H2B, J3E, J7B, JXX A3E*, H3E* (100% modulation) |  | Aeronautical stations Aircraft stations | 6 kW 400 W |
| Other emissions such as A1A, A1B, F1B |  | Aeronautical stations Aircraft stations | 1.5 kW 100 W |
| * A3E and H3E to be used only on 3 023 kHz and 5 680 kHz. |  |  |

27/61	2.2	It is assumed that the maximum peak envelope powers specified above for aeronautical stations will produce the mean effective radiated power of 1 kW used as a basis for the interference range contours.

27/62	2.3	In order to provide satisfactory communication with aircraft, aeronautical stations serving MWARA, VOLMET and world-wide allotment areas may exceed the power limits specified in No. 27/60, except in the case of 3 023 kHz and 5 680 kHz which are subject to the special provisions of Nos. 27/232 to 27/238. In each such case, the administration having jurisdiction over the aeronautical station shall note No. 15.2 and ensure:

27/63	a)	that when there is any possibility of harmful interference coordination is effected with the administrations concerned;

27/64	b)	that harmful interference is not caused to stations using frequencies in accordance with the applicable provisions of the allotment Plan;

27/65	c)	that in other MWARAs, RDARAs or VOLMET areas allotted the same frequencies, the specified protection ratios within the boundaries of those areas shall be maintained;

27/66	d)	that the directional characteristics of the antenna are such as to minimize radiation in unnecessary directions, particularly towards other MWARAs, RDARAs or VOLMET areas which have been allotted the same frequencies;

27/67	e)	that, in accordance with the Radio Regulations, all details of the assignment(s), including the transmitting antenna characteristics shall be notified to the Radiocommunication Bureau.

27/68	2.4	It is recognized that the power employed by aircraft transmitters may, in practice, exceed the limits specified in No. 27/60. However, the use of such increased power (which normally should not exceed 600 W PX) shall not cause harmful interference to stations using frequencies in accordance with the technical principles on which the allotment Plan is based.

D  –  Limits to the power levels of unwanted emissions

# 1	Technical provisions relating to the use of single-sideband emissions

## 27/69	1.1	Definitions carrier modes:

| Carrier mode | Level N (dB) of the carrier with respect to peak envelope power |
| --- | --- |
| Full carrier (for example H2B) | 0 ≥ N ≥ – 6 |
| Suppressed carrier (for example J3E) | Aircraft stations N < – 26 Aeronautical stations N < – 40 |

# 2	Tolerance for levels of emission outside the necessary bandwidth

27/70	2.1	In a single-sideband transmission, the mean power of any emission supplied to the antenna transmission line of an aeronautical or aircraft station on any discrete frequency, shall be less than the mean power (PY) of the transmitter in accordance with the Table in No. 27/71.

27/71	2.2	For aircraft station transmitter types first installed before 1 February 1983:

| Frequency separation Δ from the assigned frequency (kHz) | Minimum attenuation below mean power (PY) (dB) |
| --- | --- |
| 2 ≤ Δ < 6 | 25 |
| 6 ≤ Δ < 10 | 35 |
| 10 ≤ Δ | Aircraft stations: 40 Aeronautical stations: 43 + 10 log10 (PY) (W) |

27/72	NOTE – All transmitters first placed in operation after 1 February 1983 shall comply with the specifications contained in No. 27/74.

27/73	2.3	In a single-sideband transmission, the peak envelope power (PX) of any emission supplied to the antenna transmission line of an aeronautical or aircraft station on any discrete frequency, shall be less than the peak envelope power (PX) of the transmitter in accordance with the Table in No. 27/74.

27/74	2.4	For aircraft station transmitters first installed after 1 February 1983 and for aeronautical station transmitters in use after 1 February 1983:

| Frequency separation Δ from the assigned frequency (kHz) | Minimum attenuation below peak envelope power (PX) (dB) |
| --- | --- |
| 1.5 ≤ Δ < 4.5 | 30 |
| 4.5 ≤ Δ < 7.5 | 38 |
| 7.5 ≤ Δ | Aircraft stations: 43 Aeronautical stations: * |
| * For transmitter power up to and including 50 W: 43 + 10 log10 (PX) (W). For transmitter powers more than 50 W, the attenuation shall be at least 60 dB. |  |

E  –  Other technical provisions

# 1	Assigned frequencies

27/75	1.1	For single-sideband emissions, except the class of emission H2B, the assigned frequency shall be at a value 1 400 Hz above the carrier (reference) frequency.

27/76	1.2	For aeronautical stations equipped with selective calling systems, the class of emission H2B shall be indicated in the Supplementary Information column of the form of notice (see Appendix 4).

27/77	1.3	For classes of emission A1A, A1B and F1B the assigned frequency shall be chosen in accordance with the provisions of the footnote to Nos. 27/58 and 27/59.

27/78	1.4	The assigned frequency of a station employing double sideband emissions (A3E) shall be at the carrier (reference) frequency.

PART  II  –  Plan for the allotment of frequencies for the aeronautical mobile (R)
service in the exclusive bands between 2 850 and 22 000 kHz

Section I  –  Description of the boundaries of the areas and sub-areas

27/79	1	The boundary descriptions which follow delineate the areas to which frequencies are allotted under the frequency allotment Plan.

27/80	2	These areas are shown graphically on the maps associated with this Appendix. If there is any difference between the areas as shown on the maps and as described, the written description is to be considered correct.

27/81	3	References to the name of a country or of a geographical area in the descriptions or on the maps and the borders shown on the maps do not imply the expression of any opinion whatsoever on the part of the ITU concerning the political status of such a country or geographical area or any official recognition of these borders.

27/82	4	In the description of the Major World Air Route Areas (MWARAs) all lines between points not otherwise specified are defined as great circles.

27/83		In the description of the Regional and Domestic Air Route Areas (RDARAs) and Sub-Areas all lines between points not otherwise specified are defined as straight lines on a Mercator Projection map.

27/84		In the description of the VOLMET areas all lines between points are defined as great circles.

ARTICLE  1

Description of the boundaries of the major world air route areas 
(MWARAs)

27/85	Major World Air Route Area – CARIBBEAN (MWARA-CAR)

From the point 20° N 120° W through the points 35° N 120° W, 35° N 85° W, 43° N 74° W, 40° N 60° W, 00° 48° W, 00° 80° W, to the point 20° N 120° W.

27/86	Major World Air Route Area – CENTRAL EAST PACIFIC (MWARA-CEP)

From the point 50° N 122° W through the points 38° N 120° W, 15° N 110° W, 20° S 145° W, 20° S 152° W, 30° N 165° W, to the point 50° N 122° W.

27/87	Major World Air Route Area – CENTRAL WEST PACIFIC (MWARA-CWP)

From the point 40° N 117° E through the points 25° N 155° W, 17° N 155° W, 00° 165° W, 00° 170° E, 12° S 165° E, 12° S 136° E, 09° N 115° E, 23° N 114° E, to the point 40° N 117° E.

27/88	Major World Air Route Area – EUROPE (MWARA-EUR)

From the point 33° N 12° W through the points 54° N 12° W, 70° N 00°, 74° N 40° E, 74° N 52° E, 60° N 52° E, 40° N 36° E, 29° N 35° 30' E, 32° N 13° E, to the point 33° N 12° W.

27/89	Major World Air Route Area – INDIAN OCEAN (MWARA-INO)

From the South Pole through the points 30° S 26° E, 20° N 35° E, 30° N 60° E, 30° N 90° E, 30° S 120° E, 40° S 160° E to the South Pole.

27/90	Major World Air Route Area – MIDDLE EAST (MWARA-MID)

From the point 51° N 30° E through the points 57° N 37° E, 50° N 80° E, 44° N 94° E, 08° N 76° E, 11° 45' N 42° E, 16° N 42° E, 30° N 30° E, to the point 51° N 30° E.

27/91	Major World Air Route Area – NORTH ATLANTIC (MWARA-NAT)

From the North Pole through the points 60° N 135° W, 49° N 120° W, 49° N 74° W, 39° N 78° W, 18° N 66° W, 05° N 55° W, 16° N 26° W, 32° N 08° W, 44° N 02° E, 60° N 20° E, to the North Pole.

27/92	Major World Air Route Area – NORTH CENTRAL ASIA (MWARA-NCA)

From the North Pole through the points 75° N 10° E, 60° N 25° E, 30° N 25° E, 30° N 73° E, 37° N 73° E, 49° N 85° E, 42° N 97° E, 42° N 110° E, 45° N 113° E, 46° 30' N 120° E, 49° N 116° E, 54° N 123° E, 45° N 133° E, 40° N 124° E, 30° N 124° E, 25° N 135° E, 65° N 170° W, to the North Pole.

27/93	Major World Air Route Area – NORTH PACIFIC (MWARA-NP)

From the North Pole through the points 60° N 135° W, 47° N 118° W, 30° N 165° W, 30° N 115° E, 41° N 116° E, 55° N 135° E to the North Pole.

27/94	Major World Air Route Area – AFRICA (MWARA-AFI)

From the point 40° N 35° W, through the points 37° N 03° W, 37° N 44° E, the border between Iraq and the Islamic Republic of Iran, the points 29° N 48° E, 26° N 56° E, 20° N 62° E, 22° S 60° E, 35° S 30° E, 35° S 16° E, 05° N 03° W, 05° N 35° W, to the point 40° N 35° W.

27/95	Major World Air Route Area – SOUTH ATLANTIC (MWARA-SAT)

From the South Pole through the points 30° S 75° W, 19° S 53° W, 00° 60° W, 20° N 60° W, 25° N 25° W, 41° N 15° W, 41° N 03° W, 15° N 03° W, 20° S 32° E to the South Pole.

27/96	Major World Air Route Area – SOUTH AMERICA (MWARA-SAM)

From the South Pole through the points 15° N 125° W, 15° N 60° W, 10° N 60° W, 05° S 30° W, 36° S 52° W, to the South Pole.

27/97	Major World Air Route Area – SOUTH EAST ASIA (MWARA-SEA)

From the point 26° N 130° E, through the points 00° 130° E, 00° 135° E, 12° S 145° E, 12° S 160° E, 25° S 155° E, 40° S 150° E, 35° S 115° E, 18° N 62° E, 26° N 65° E, to the point 26° N 130° E.

27/98	Major World Air Route Area – SOUTH PACIFIC (MWARA-SP)

From the South Pole through the points 38° S 145° E, 00° 167° E, 00° 175° W, 22° N 158° W, 22° N 156° W, 00° 120° W to the South Pole.

27/99	Major World Air Route Area – EAST ASIA (MWARA-EA)

From the point 55° N 124° E through the points 37° N 145° E, 26° N 130° E, 00° 130° E, 00° 80° E, 18° N 62° E, 37° N 67° E, 55° N 80° E to the point 55° N 124° E.

ARTICLE  2

Description of the boundaries of the regional and domestic air route areas 
(RDARAs)

27/100	Regional and Domestic Air Route Area – 1 (RDARA-1)

From the North Pole along the 15° W meridian to the point 72° N 15° W, then through the points 40° N 50° W, 30° N 39° W, 30° N 10° W, 31° N 10° W, to the point 31° N 10° E. Then along the Libya-Tunisia border to the Mediterranean, thence along the coast of Libya and Egypt to Alexandria. Thence to Cairo, eastward along the Cairo parallel to intersect the 40° E meridian, and north along the 40° E meridian to the intersection with the border between the Syrian Arabic Republic and Iraq and along this border up to the Turkish border. Then along the border between Turkey and the following countries: Iraq, Islamic Republic of Iran, Armenia and Georgia, up to the Black Sea Coast. Thence along the Black Sea Coast of Turkey to intersect the 30° E meridian, then along the 30° E meridian to the border of Romania and Ukraine. Thence along the borders between Romania and Ukraine, Romania and Moldova, Romania and Ukraine. Thence along the border of Ukraine, and the following countries: Hungary, Slovakia and Poland. Thence along the border of Poland and the following countries: Belarus, Lithuania and the Russian Federation. Thence northeastward along the Baltic Sea coast, to the border between Finland and the Russian Federation, and between Norway and the Russian Federation, to the point 70° N 32° E, and along the 32° E meridian to the North Pole.

27/101	Sub-Area 1A

From the point 65° N 26° W, and through the points 40° N 50° W, 40° N 20° W, 60° N 20° W, 60° N 26° W, to the point 65° N 26° W.

27/102	Sub-Area 1B

From the North Pole along the 15° W meridian to the point 72° N 15° W, then through the points 65° N 26° W, 60° N 26° W, 60° N 20° W to the points 50° N 20° W and 50° N 10° W, thence east along the territorial waters between the Channel Islands and the French coastline, reaching the latter at the meridian 03° W. Thence following the French coastline northeastward and the frontier of France with Belgium, Luxembourg and Germany. Thence along the border between Germany and the following countries: Switzerland, Austria, the Czech Rep. and Poland towards the Baltic Sea. Then west along the coastline of Germany to the border between the latter and Denmark. Along this border to the North Sea. Thence along the 55° N parallel to the point 55° N 04° E, then through the points 56° N 03° E, 59° N 02° E, 62° N 01° E. Thence along the 01° E meridian to the North Pole.

27/103	Sub-Area 1C

From the North Pole along the meridian 01° E to the point 62° N 01° E. Thence through the points 59° N 02° E, 56° N 03° E, 55° N 04° E and then east along the 55° N parallel and the border between Denmark and Germany to the Baltic Sea and along the Baltic Sea coast of Germany to the border between Germany and Poland. Along this border and continuing along the western borders of the Czech Rep. and Austria to the borders between Austria and Switzerland, Austria and Liechtenstein and Austria and Switzerland. Thence eastward along the southern borders of Austria and Hungary, thence along the border between Hungary and Romania. Thence, along the border between Ukraine and the following countries: Hungary, Slovakia and Poland. Thence along the border of Poland and the following countries: Belarus, Lithuania and the Russian Federation to the Baltic Sea. Thence northeastward along the Baltic Sea coast, along the borders between Finland and the Russian Federation and between Norway and the Russian Federation to the point 70° N 32° E, then along the 32° E meridian to the North Pole.

27/104	Sub-Area 1D

From the junction of the borders of Ukraine, Hungary and Romania, westward along the southern borders of Hungary and Austria to the border between Switzerland and Italy, and the border between France and Italy to the Mediterranean Sea. Thence to 43° N 10° E to 41° N 10° E to 41° N 07° E, thence along the 07° E meridian to the North African coast. Then along the North African coast including Tunis, Tripoli, Benghazi, to the coastal border between Libya and Egypt. Thence along the coast to Alexandria, then to Cairo, and along the Cairo parallel to the 40° E meridian. North along the 40° E meridian to the intersection with the border between Syrian Arab Republic and Iraq and along this border up to the Turkish border. Then along the border between Turkey and the following countries: Iraq, Islamic Republic of Iran, Armenia and Georgia, up to the Black Sea Coast. Thence along the Black Sea Coast of Turkey to intersect the 30° E meridian. Along the 30° E meridian to the border of Romania and Ukraine, thence along the borders between Romania and Ukraine, Romania and Moldova, Romania and Ukraine to the junction of the borders of Ukraine, Hungary and Romania.

27/105	Sub-Area 1E

From the point 50° N 20° W, through the points 40° N 20° W, 40° N 50° W, 30° N 39° W, 30° N 10° W, 31° N 10° W, to the point 31° N 10° E. Then along the border between Libya and Tunisia to the Mediterranean, thence along the Tunisian coast to intersect the 10° E meridian. Thence along this meridian to the point 43° N 10° E; thence to the borders between Italy and France and between Italy and Switzerland, Austria and Switzerland, Austria and Liechtenstein, Austria and Switzerland, Switzerland and Germany, and between France and Germany, France and Luxembourg, and France and Belgium to the Channel coast. Thence west through the territorial waters between the Channel Islands and the French coast to the points 50° N 10° W and 50° N 20° W.

27/106	Regional and Domestic Air Route Area – 2 (RDARA-2)

From the North Pole along the 32° E meridian to the 70° N parallel. Then along the border between Norway and the Russian Federation and Finland and the Russian Federation to the Baltic coast. Thence southwestward along the Baltic coast to the border between the Russian Federation and Poland. Thence along the border between Poland and the following countries: the Russian Federation, Lithuania, Belarus and Ukraine. Thence along the border between Ukraine and the following countries: Poland, Slovakia, Hungary and Romania, to the junction of the borders of Ukraine, Romania and Moldova. Thence along the borders of Romania and Moldova, Romania and Ukraine, to the Black Sea coast at the intersection of the 30° E meridian. Then along the 30° E meridian to the Black Sea coast of Turkey. Along the Black Sea coast of Turkey to the junction of the borders of Turkey and Georgia. Thence along borders between Turkey and the following countries: Georgia, Armenia and Azerbaijan, to the junction of the borders between the Islamic Republic of Iran and Azerbaijan. Then along the northern border of the Islamic Republic of Iran to Caspian Sea. Then along the Iran Caspian Sea coast to the border of Turkmenistan. Thence eastward along the southern borders of Turkmenistan, Uzbekistan, Tajikistan and Kyrgyzstan, and the eastern border of Kazakhstan, to the junction of the borders of Kazakhstan, the Russian Federation and China. Then along the border between the Russian Federation and China to the intersection of the Mongolia-China-Russian Federation borders at approximately 49° N 88° E. Then along the 88° E meridian to 55° N. Then along the 55° N parallel to 60° E, and along the 60° E meridian to the North Pole.

27/107	Sub-Area 2A

From the North Pole along the 32° E meridian to 70° N. Then along the border between Norway and the Russian Federation, and Finland and the Russian Federation to the Baltic coast, and southwestward along the Baltic coast to the point 55° N 20° E, and thence to Moscow. Then to 55° N 60° E, and along the 60° E meridian to the North Pole.

27/108	Sub-Area 2B

From the point 55° N 88° E and through the point 55° N 60° E to the point 47° N 53° E. Thence along the east coast of the Caspian Sea to the Iranian coast. Then along the Islamic Republic of Iran Caspian Sea coast to the border of Turkmenistan. Thence eastward along the southern borders of Turkmenistan, Uzbekistan, Tajikistan and Kyrgyzstan, and the eastern border of Kazakhstan, to the junction of the borders of Kazakhstan, the Russian Federation and China. Then along the border between the Russian Federation and China to the intersection of the Mongolia-China-Russian Federation borders at approximately 49° N 88° E; thence along the 88° E meridian to 55° N 88° E.

27/109	Sub-Area 2C

From the point 55° N 60° E, to Moscow, to 55° N 20° E. Thence south along the borders between Poland and the following countries: Russian Federation, Lithuania, Belarus and Ukraine. Thence along the border between Ukraine and the following countries: Poland, Slovakia, Hungary and Romania, to junction of the borders of Ukraine, Romania and Moldova. Thence along the borders of Romania and Moldova, Romania and Ukraine to the Black Sea coast at the meridian 30° E. Along the meridian 30° E to the Black Sea coast of Turkey. Along this coastline to the junction of the border between Turkey and Georgia. Thence along the borders between Turkey and the following countries: Georgia, Armenia and Azerbaijan, to the junction of the borders between the Islamic Republic of Iran and Azerbaijan. Then along the northern borders of the Islamic Republic of Iran to the Caspian Sea, then along the south coast of the Caspian Sea and thence north along the East Caspian Sea coast and through the point 47° N 53° E to 55° N 60° E.

27/110	Regional and Domestic Air Route Area – 3 (RDARA-3)

From the North Pole to the point 55° N 60° E, thence along the 55° N parallel to 88° E. Then along the 88° E meridian to the intersection of the Mongolia-China-Russian Federation borders at approximately 49° N 88° E. Then along the borders between Mongolia and China, and the Russian Federation and China, to the coast. Between the territorial waters of the Russian Federation and Japan to the point 43° N 147° E and through the point 50° N 164° E to 65° N 170° W. Then along the 170° W meridian to the North Pole.

27/111	Sub-Area 3A

From the North Pole along the 60° E meridian to 55° N. Then along the 55° N parallel to 88° E. Then through the point 60° N 88° E to 60° N 110° E, and along the 110° E meridian to the North Pole.

27/112	Sub-Area 3B

From the North Pole along the 110° E meridian to 60° N 110° E, and through the points 60° N 147° E, 43° N 147° E, 50° N 164° E, to 65° N 170° W. Then along the 170° W meridian to the North Pole.

27/113	Sub-Area 3C

From the point 60° N 88° E to the intersection of Mongolia-China-the Russian Federation borders at approximately 49° N 88° E. Along the borders between Mongolia and China, and the Russian Federation and China, to the coast. Between the territorial waters of the Russian Federation and Japan to the point 43° N 147° E. Then through the point 60° N 147° E to the point 60° N 88° E.

27/114	Regional arid Domestic Air Route Area – 4 (RDARA-4)

From the point 30° N 39° W, and through the points 10° N 20° W, 05° S 20° W, to the point 05° S 12° E. Thence along the border between the Rep. of the Congo and Angola, then along the northern border of the Dem. Rep. of the Congo, and the borders of the Rep. of the Congo, of the Central African Republic and the Sudan. Thence north along the western border of the Sudan. Along the western border of Egypt, northwards to the Mediterranean and along the Mediterranean and Atlantic coasts of North Africa to the point 30° N 10° W. West along the 30° N parallel to close the area at 30° N 39° W.

27/115	Sub-Area 4A

From the point 30° N 39° W to 21° N 31° W. Thence to Gao and to Zinder. From Zinder, along the northern border of Nigeria, to the junction of the borders of Nigeria, Chad and Cameroon. Then along the border between Chad and Cameroon to a point west of N'Djamena. Then along the parallel to 12° N 22° E. Thence north along the western border of the Sudan, and along the western border of Egypt to the Mediterranean. Along the North African Mediterranean coast and Atlantic coast to a point 30° N 10° W. Thence along the 30° N parallel to close the sub-area at 30° N 39° W.

27/116	Sub-Area 4B

From the point 21° N 31° W, through the points 10° N 20° W, 05° S 20° W to 05° S 12° E. Thence along the southern border of the Rep. of the Congo and the Central African Republic to the junction between the Dem. Rep. of the Congo, the Sudan and the Central African Republic. Along the western border of the Sudan to the point 12° N 22° E. Thence along the N'Djamena parallel to the Nigerian border. Then westward along this border to the point 13° 12' N 10° 45' E, through Zinder and Gao, to the point 21° N 31° W.

27/117	Regional and Domestic Air Route Area – 5 (RDARA-5)

From the point 41° N 40° E to the point 37° N 40° E. Then along the border between Turkey and Syrian Arab Republic to the Mediterranean coast. Thence to the common border of Libya and Egypt on the North African coast excluding Cyprus. Southward along the western border of Egypt, and the Sudan to the border of Kenya. Thence east along the northern border of Kenya, then south along the border between Kenya and Somalia and to the East African coast at 02° S 41° E. Then through the point 02° S 73° E to 37° N 73° E. Then east along the border between Afghanistan and Pakistan, and west along the northern borders of Afghanistan and the Islamic Republic of Iran to the Caspian Sea. Then along the northern border of the Islamic Republic of Iran and Turkey to close the area at 41° N 40° E.

27/118	Sub-Area 5A

From the point 37° N 40° E, along the border between Turkey and the Syrian Arab Republic to the Mediterranean coast. Thence to the Libyan -Egyptian border on the North African coast, excluding Cyprus. Southward, along the western border of Egypt and east along the common border of Egypt and the Sudan to 24° N 37° E. Then through the points 11° 45' N 42° E, 11° 45' N 55° E, 20° N 52° E, to the point 26° N 52° E. Thence along the border between Islamic Republic of Iran and Iraq, and the border between Iraq and Turkey, to the point 37° N 40° E.

27/119	Sub-Area 5B

From the point 41° N 40° E to 37° N 40° E. Thence east along the borders between Turkey and Syrian Arab Republic and Turkey and Iraq, and along the border between Iraq and the Islamic Republic of Iran to the point 30° N 49° E. Thence along the middle of the Gulf through the points 26° N 52° E and 24° N 60° E, to Mumbai. Then to 37° N 73° E. Then east along the border between Afghanistan and Pakistan, then west along the northern borders of Afghanistan and the Islamic Republic of Iran, to the Caspian Sea. Then along the northern border of the Islamic Republic of Iran and Turkey to close the sub-area at 41° N 40° E.

27/120	Sub-Area 5C

From the point 26° N 52° E, and through the points 13° N 52° E, 13° N 54° E, 02° S 54° E, 02° S 73° E, to Mumbai. Then to 24° N 60° E. Then along the middle of the Gulf to 26° N 52° E.

27/121	Sub-Area 5D

From the junction of Egypt, Libya and the Sudan southward along the western border of Sudan to the border of Kenya. Thence along the northern border of Kenya. Then south along the border between Kenya and Somalia to the east African coast, at the point 02° S 42° E. Then through the points 02° S 54° E, 13° N 54° E, 13° N 52° E to the point 12° N 44° E. Thence northwest along the middle of the Red Sea to 24° N 37° E. Thence along the southern border of Egypt to close the sub-area.

27/122	Regional and Domestic Air Route Area – 6 (RDARA-6)

From approximately 49° N 88° E, eastward along the border between China and the following countries: the Russian Federation, Kazakhstan, Kyrgyzstan, Tajikistan and Afghanistan. Then along the border between Afghanistan and Pakistan, and the Islamic Republic of Iran and Pakistan to the point 23° N 61° E. Thence to Mumbai. Then along the 73° E meridian to the point 02° S 73° E, and through the points 02° S 92° E, 10° S 92° E, 10° S 141° E, 00° 141° E, 00° 160° E, 03° 30' N 160° E, 03° 30' N 170° W, 10° N 170° W, 50° N 164° E, to the point 43° N 147° E. Thence west between the territorial waters of Japan and the Russian Federation and along the north-eastern and northern border of China to approximately 49° N 88° E.

27/123	Sub-Area 6A

From the point 37° N 75° E, along the border between Pakistan and Afghanistan, and the Islamic Republic of Iran and Pakistan to the point 23° N 61° E. Thence to Mumbai. From Mumbai to 24° N 80° E. Thence to Calcutta. Thence along the coast of Bangladesh and Myanmar to reach the border between Myanmar and Thailand. North along this border and that between Myanmar and Lao (P.D.R.). Thence along the border between China and Myanmar. Thence westward along the southern border of China to the point 37° N 75° E.

27/124	Sub-Area 6B

From the point 39° 49' 41" N 124° 10' 06" E, through the points 39° 31' 51" N 124° 06' 31" E, 39° N 124° E to the point 32° 30' N 124° E. Between the point 32° 30' N 124° E and the point 25° N 123° E, the limit of this Sub-Area is undefined. From the point 25° N 123° E, through the points 21° N 121° 30' E, 20° N 120° E, 20° N 176° W, 50° N 164° E, 43° N 147° E, thence west between the territorial waters of Japan and the Russian Federation and along the border between the Dem. People’s Rep. of Korea and the Russian Federation, and then the border between China and the Dem. People’s Rep. of Korea, to the point 39° 49' 41" N 124° 10' 06" E.

27/125	Sub-Area 6C

From the point 20° N 130° E through the point 04° N 130° E to 04° N 118° E. Thence along the southern borders of Sabah and Sarawak to the coast and then southward along the west coast of Borneo to the 110° E meridian. Thence along 110° E meridian to the point 10° S 110° E. Thence through the points 10° S 141° E, 00° 141° E, 00° 160° E, 03° 30' N 160° E, 03° 30' N 170° W, 10° N 170° W, 20° N 176° W to 20° N 130° E.

27/126	Sub-Area 6D

From the junction of the borders of China, India and Myanmar, south along the India-Myanmar and Bangladesh-Myanmar borders to the Bay of Bengal. Along the coast of Myanmar to its southernmost point, then to Weh Island (off the north coast of Sumatra). Then to the point 02° S 92° E, and through the point 10° S 92° E to 10° S 110° E. Then eastward to 10° S 141° E extending northward to 00° 141° E and then to 04° N 130° E through the point 20° N 130° E to 20° N 113° E. Thence, south around the Island of Hainan, and along the border between China, Viet Nam, the Lao (P.D.R.) and Myanmar, to close the Sub-Area at the junction of the borders of China, India and Myanmar.

27/127	Sub-Area 6E

From the point 20° N 73° E, and through the points 02° S 73° E, 02° S 92° E, through Weh Island (off the north coast of Sumatra) to 10° N 97° E. Thence along the coasts of Myanmar, Bangladesh and India to Calcutta. Then through the points 24° N 80° E to 20° N 73° E.

27/128	Sub-Area 6F

From the point 25° N 123° E, 21° N 121° 30' E, 20° N 120° E, 20° N 113° E, thence south around the Island of Hainan and along China-Viet Nam, China-Lao (P.D.R.) and China-Myanmar borders to the junction of the borders of China, India and Myanmar, south along the India-Myanmar and Bangladesh-Myanmar borders to the Bay of Bengal. Along the coast of Myanmar to its southernmost point then to Weh Island (off the north coast of Sumatra). Then to the point 02° S 92° E and through the point 10° S 92° E to 10° S 110° E. Then northward along 110° E meridian, thence along the boundary of Sub-Area 6C to the points 20° N 130° E, 43° N 147° E, thence westward between the territorial waters of Japan and the Russian Federation and along the border between the Dem. People’s Rep. of Korea and the Russian Federation, then the border between China and the Dem. People’s Rep. of Korea, to the points 39° 49' 41" N 124° 10' 06" E, 39° 31' 51" N 124° 06' 31" E, 39° N 124° E, then to the point 32° 30' N 124° E.

Between the points 32° 30' N 124° E and 25° N 123° E, the limit of this Sub-Area is undefined.

27/129	Sub-Area 6G

From the point 32° 30' N 124° E northward to 39° N 124° E, 39° 31' 51" N 124° 06' 31" E then to 39° 49' 41" N 124° 10' 06" E on the border between China and the Dem. People’s Rep. of Korea. Then along the border of China to the junction of the border with India and Myanmar. Thence southward along the India-Myanmar and Bangladesh-Myanmar borders to the Bay of Bengal. Along the coast of Myanmar to its southernmost point. Then to Weh Island (off the north coast of Sumatra). Then to the point 02° S 92° E and through the point 10° S 92° E to 10° S 110° E. Then eastward to 10° S 141° E extending northward to 00° 141° E and then to 04° N 130° E through the point 20° N 130° E to 20° N 120° 40' E. Thence northward to the points 21° N 121° 30' E and 25° N 123° E.

Between the points 25° N 123° E and the point 32° 30' N 124° E, the limit of this Sub-Area is undefined.

In the area where Sub-Areas 6D, 6F and 6G are common, the frequencies allotted to Sub-Area 6G shall be used only by the aeronautical stations of China; the frequencies allotted to Sub-Areas 6D and 6F will be used only by the aeronautical stations of the other administrations in the common area. Also in this common area, the operational use by China of the frequencies allotted to Sub-Area 6G shall be within the area defined by a line starting at 21° 32' 52" N 108° E, passing through the points 20° N 108° E, 20° N 107° E, 18° N 107° E, 18° N 108° E, 15° N 110° E, 10° N 110° E, 06° N 108° E, 03° 30' N 112° E, 04° N 113° E, 08° N 116° E, 10° N 118° E, 14° N 119° E, 18° N 119° E to 20° N 120° 40' E and thence along the limit of Sub-Area 6D to 21° 32' 52" N 108° E.

27/130	Regional and Domestic Air Route Area – 7 (RDARA-7)

From the South Pole along the 20° W meridian to 05° S. Then along the 05° S parallel to 12° E. Thence along the border between the Rep. of the Congo and Angola, then along the northern border of the Dem. Rep. of the Congo, along the border between Uganda and Sudan, and the borders between Kenya and Sudan, Ethiopia and Somalia, to the point 02° S 42° E. Then to 02° S 60° E and along the 60° E meridian to 11° S, then through the points 11° S 65° E, 40° S 65° E, 40° S 60° E to the South Pole.

27/131	Sub-Area 7A

From the South Pole along the 20° W meridian to 05° S. Then through the points 05° S 10° E, 40° S 10° E, to 40° S 60° E. Then along the 60° E meridian to the South Pole.

27/132	Sub-Area 7B

From the point 05° S 10° E to 05° S 12° E. Thence along the border between the Rep. of the Congo and Angola, then along the northern border of the Dem. Rep. of the Congo, to the junction of the borders of Uganda, the Dem. Rep. of the Congo and Sudan. Thence along the eastern borders of the Dem. Rep. of the Congo, Rwanda, Burundi, and the Dem. Rep. of the Congo. Thence along the southern borders of the Dem. Rep. of the Congo and Angola to the coast of the South Atlantic. Thence to the point 17° S 10° E, and then to the point 05° S 10° E.

27/133	Sub-Area 7C

From the junction of the borders of Uganda, the Dem. Rep. of the Congo and Sudan along the western borders of Uganda and Tanzania, and then along the southern border of Tanzania to the coast. Thence through the points 11° S 41° E, 11° S 60° E, 02° S 60° E, to 02° S 41° E and thence to the east coast of Africa. Then north along the eastern border of Kenya, then west along the northern borders of Kenya and Uganda to close the sub-area at the junction of the borders of the Dem. Rep. of the Congo, Sudan and Uganda.

27/134	Sub-Area 7D

From the border between Tanzania and Mozambique on Lake Nyasa, south along the west border of Mozambique to the east coast of Africa, then through the points 27° S 33° E, 40° S 33° E, 40° S 65° E, 11° S 65° E to 11° S 41° E. Thence along the northern border of Mozambique to Lake Nyasa.

27/135	Sub-Area 7E

From the point 17° S 10° E, and through the points 40° S 10° E, 40° S 33° E, to 27° S 33° E. Thence along the west border of Mozambique and the part of the western border of Tanzania as far as the northern point of Lake Nyasa. Thence along the borders between Malawi and Tanzania and between Zambia and Tanzania and along the borders between the Dem. Rep. of the Congo and Zambia, Angola and Zambia, and Angola and Namibia to the coast at the point 17° S 10° E.

27/136	Sub-Area 7F

From the point 05° S 10° E to 05° S 12° E, along the border between the Rep. of the Congo and Angola to the junction point of the borders of the Rep. of the Congo, Angola, and the Dem. Rep. of the Congo. Thence along the border between Angola and the Dem. Rep. of the Congo until the coast of the Atlantic, along the coastline until the Zaire River and thence along the northern, eastern and southern border of Angola to the coast of the South Atlantic. Thence to the point 17° S 10° E and then to the point 05° S 10° E.

27/137	Regional and Domestic Air Route Area – 8 (RDARA-8)

From the South Pole along the 60° E meridian to 40° S then through the points 40° S 65° E, 11° S 65° E, 11° S 60° E, 02° S 60° E, 02° S 92° E, 10° S 92° E, to 10° S 110° E. Then along the 110° E meridian to the South Pole.

27/138	Regional and Domestic Air Route Area – 9 (RDARA-9)

From the South Pole along the 160° E meridian to 27° S. Then through the points 19° S 153° E, 10° S 145° E, 10° S 141° E, 00° 141° E, 00° 160° E, 03° 30' N 160° E, 03° 30' N 120° W. Then along the 120° W meridian to the South Pole.

27/139	Sub-Area 9B

From the point 00° 141° E through points 10° S 141° E, 10° S 145° E, 27° S 160° E, 27° S 157° W, 03° 30' N 157° W, 03° 30' N 160° E, 00° 160° E to the point 00° 141° E.

27/140	Sub-Area 9C

From the South Pole along the 170° W meridian to 03° 30' N. Then through the point 03° 30' N 120° W and along the 120° W meridian to the South Pole.

27/141	Sub-Area 9D

From the South Pole along the 160° E meridian to 27° S. Then through the point 27° S 170° W and along the 170° W meridian to the South Pole.

27/142	Regional and Domestic Air Route Area – 10 (RDARA-10)

From the point 50° N 164° E to 66° N 169° W. Then along the 169° W meridian to the North Pole. Then through the points 82° N 30° E, 82° N 00°, 73° N 00°, 73° N 15° W. Then along the 15° W meridian to 72° N. Then through the points 40° N 50° W, 40° N 65° W to 44° 30' N 73° W, 41° N 81° W, 41° N 88° W. 48° N 91° W, 48° N 127° W, 50° N 130° W, then westward to the point 50° N 164° E.

27/143	Sub-Area 10A

From the point 50° N 164° E to 66° N 169° W, along the 169° W meridian to the North Pole, along the 130° W meridian to 50° N, then westward to the point 50° N 164° E.

27/144	Sub-Area 10B

From the point 57° N 140° W, along the 140° W meridian to the North Pole. Then along the 91° W meridian to 48° N. Thence through the points 48° N 127° W, 57° N 139° W, to 57° N 140° W.

27/145	Sub-Area 10C

From the point 57° N 140° W, and through the points 60° N 140° W, 60° N 91° W, 48° N 91° W, 48° N 127° W, 57° N 139° W, to 57° N 140° W.

27/146	Sub-Area 10D

From the point 48° N 98° W, along the 98° W meridian to the North Pole. Then along the 45° W meridian to 69° N. Then through the points 61° N 70° W, 45° N 72° W, 41° N 81° W, 41° N 88° W, 48° N 91° W, to 48° N 98° W.

27/147	Sub-Area 10E

From the point 45° N 74° W, and through the point 61° N 72° W to 69° N 47° W. Then along the 47° W meridian to the North Pole. Then along the 15° W meridian to 72° N. Then through the points 40° N 50° W, 40° N 65° W, to close the sub-area at 45° N 74° W.

27/148	Sub-Area 10F

From the North Pole through the points 82° N 30° E, 82° N 00°, 73° N 00°, 73° N 20° W, 70° N 20° W, 63° 30' N 39° W, 58° 30' N 43° W, 58° 30' N 50° W, 63° 30' N 55° 44' W, 65° 30' N 58° 39' W, 74° N 68° 18' W, 76° N 76° W, 78° N 75° W, 82° N 60° W to the North Pole.

27/149	Regional and Domestic Air Route Area – 11 (RDARA-11)

From the point 29° N 180° through the points 50° N 164° E, 50° N 127° W. Then along the border between the United States of America and Canada to 46° N 67° W, then to 40° N 65° W, 40° N 50° W, 25° N 35° W, 25° N 98° W, 33° N 119° W, 33° N 153° W, 29° N 153° W to the point 29° N 180°.

27/150	Sub-Area 11A

From the point 29° N 180°, through the points 50° N 164° E, 50° N 130° W, 33° N 130° W, 33° N 153° W, 29° N 153° W, to the point 29° N 180°.

27/151	Sub-Area 11B

From the point 50° N 130° W and through the points 33° N 130° W, 33° N 119° W, 25° N 98° W, 25° N 65° W, 40° N 65° W, 46° N 67° W. Then along the border between the United States of America and Canada through 50° N 127° W, to the point 50° N 130° W.

27/152	Sub-Area 11C

From the point 25° N 65° W and through the points 40° N 65° W, 40° N 50° W, 25° N 35° W, to the point 25° N 65° W.

27/153	Regional and Domestic Air Route Area – 12 (RDARA-12)

From the point 03° 30' N 170° W to the point 10° N 170° W, then along the boundary between ITU Regions 2 and 3 to 29° N 180°, and thence to 29° N 153° W, 33° N 153° W, through the points 33° N 120° W, 35° N 120° W, 32° N 104° W, 25° N 91° W, 26° N 91° W, 26° N 79° W, 27° N 79° W, 27° N 76° 30' W, 25° N 70° W, 25° N 35° W and along the boundary between ITU Regions 1 and 2 to 00° 20° W. Thence through the points 00° 44° W, 04° 24' N 50° 39' W. Then along the boundaries between Brazil and the French Guiana, Surinam, Guyana, Venezuela, Colombia to the junction of Brazil, Peru and Colombia then along the boundaries between Peru and Colombia and Peru and Ecuador to the point 04° S 93° W. Then to the point 05° S 93° W and through the points 05° S 120° W, 03° 30' N 120° W to the point 03° 30' N 170° W.

27/154	Sub-Area 12A

From the point 03° 30' N 170° W to the point 10° N 170° W, then along the boundary between ITU Regions 2 and 3 to 29° N 180°, and thence through the points 29° N 153° W, 03° 30' N 153° W to the point 03° 30' N 170° W.

27/155	Sub-Area 12B

From the point 03° 30' N 153° W to 33° N 153° W, through the points 33° N 120° W, 17° N 115° W, 14° N 93° W, 02° N 86° W, 02° N 93° W, 05° S 93° W, 05° S 120° W, 03° 30' N 120° W, to the point 03° 30' N 153° W.

27/156	Sub-Area 12C

From the point 33° N 120° W, through the points 35° N 120° W, 32° N 104° W, 25° N 91° W, 23° N 83° W, 22° N 83° W, 13° N 90° W, 16° N 116° W, to the point 33° N 120° W.

27/157	Sub-Area 12D

From the point 20° N 91° W, through the points 26° N 91° W, 26° N 79° W, 27° N 79° W, 27° N 76° 30' W, 26° N 73° W, 17° N 58° W, to 10° N 58° W. Thence through Panama City, Colon, Swan Island, and Belize City to the point 20° N 91° W.

27/158	Sub-Area 12E

From the point 15° N 95° W and through 23° N 92° W, 23° N 85° W, 19° N 85° W, 09° N 77° W, 02° N 79° W. Thence to 01° N 75° W along the eastern and southern border of Ecuador to the point 04° S 81° W, and from there to 02° N 81° W and 02° N 86° W, 14° N 93° W to close the sub-area at 15° N 95° W.

27/159	Sub-Area 12F

From the point 02° N 79° W to the point 08° N 83° W, then along the border between Panama and Costa Rica, through the points 10° N 83° W, 13° N 83° W, 13° N 70° W, 08° N 70° W, 06° N 67° W and 01° N 66° W. Then along the border between Brazil and Colombia to 04° S 70° W. Thence along the border between Colombia and Peru, continuing along the border between Colombia and Ecuador, to the point 02° N 79° W.

27/160	Sub-Area 12G

From the point 07° N 73° W, through the points 14° N 73° W, 14° N 58° W, 01° 31' N 58° W and along the borders of Brazil with Guyana, Venezuela, Colombia through the points 01° 57' N 68° W, 05° N 69° W, to the point 07° N 73° W.

27/161	Sub-Area 12H

From the point 05° N 70° W, through the points 08° 45' N 60° W, 08° N 58° W, 08° N 49° W, 04° 10' N 51° 36' W, and along the borders of Brazil with French Guiana, Surinam, Guyana, Venezuela and Colombia to the junction of the borders of Brazil, Colombia and Peru, to the point 05° N 70° W.

27/162	Sub-Area 12I

From the point 25° N 70° W, through the point 25° N 35° W and along the boundary between ITU Regions 1 and 2, to 00° 20° W. Thence through the points 00° 44° W, 08° N 54° W, 08° N 58° W, 17° N 58° W, to the point 25° N 70° W.

27/163	Sub-Area 12J

From the point 04° S 93° W, through the points 02° N 93° W, 02° N 79° W. Then along the border between Ecuador and Colombia to the junction with the borders of Colombia, Peru and Ecuador. Thence along the border between Peru and Ecuador to the point 04° S 93° W.

27/164	Regional and Domestic Air Route Area – 13 (RDARA-13)

From the South Pole along the 120° W meridian to 05° S. Then through the points 05° S 93° W, 04° S 82° W, and along the southern border of Ecuador, Colombia, Venezuela, Guyana, Surinam, the French Guiana, to the point 04° 24' N 50° 39' W. Then through the points 04° 24' N 47° W, 00° 32° W to the point 00° 20° W, and along the 20° W meridian to the South Pole.

27/165	Sub-Area 13A

From the point 05° S 120° W through the points 05° S 93° W, 04° S 82° W, 19° S 81° W, 57° S 81° W, to 57° S 90° W. Thence to the South Pole to the point 05° S 120° W.

27/166	Sub-Area 13B

From the point 29° S 111° W, through the points 24° S 111° W, 24° S 104° W, 29° S 104° W, to the point 29° S 111° W.

27/167	Sub-Area 13C

From the point 15° S 47° W, through the points 20° S 44° W, 23° l9' S 42° W, 25° S 45° W, 22° 30' S 50° 39' W, 19° 52' S 58° W, and along the borders of Brazil with Paraguay, Bolivia, Peru, Colombia, Venezuela, Guyana, Surinam and French Guiana to 04° 24' N 50° 39' W, 04° 24' N 47° W, to the point 15° S 47° W.

27/168	Sub-Area 13D

From 11° S 69° 30' W along the border between Bolivia and Brazil and through the point 20° 10' S 58° W, along the border between Bolivia and Paraguay to 22° 30' S 62° 30' W. Then along the border between Bolivia and Argentina and through the point 23° S 67° W along the border between Bolivia and Chile and through the point 16° 30' S 69° 30' W following the border between Bolivia and Peru to the point 11° S 69° 30' W.

27/169	Sub-Area 13M

From the point 19° S 81° W, through the points 04° S 82° W, 03° S 80° W, following the boundaries between Peru and Ecuador, Colombia and Brazil to the point 11° S 69° 30' W, along the border of Peru with Bolivia to 17° 30' S 69° 30' W, then along the border of Peru with Chile to the point 19° S 81° W.

27/170	Sub-Area 13N

From the point 22° 30' S 62° 30' W along the border of Paraguay with Bolivia to 20° 10' S 58° W, along the border of Paraguay with Brazil to 25° 50' S 54° 30' W and thence along the border of Paraguay with Argentina to the point 22° 30' S 62° 30' W.

27/171	Sub-Area 13E

From the point 32° S 81° W through the point 19° S 81° W, up to the intersection of the coast with the border between Chile and Peru, Bolivia and Argentina, to the point of intersection with 32° S and then to the point 32° S 81° W.

27/172	Sub-Area 13F

From the point 57° S 81° W, through the point 32° S 81° W to the intersection of 32° S with the border between Chile and Argentina, through the points 52° S 67° W, 57° S 67° W, 57° S 40° W to the South Pole to the point 57° S 81° W.

27/173	Sub-Area 13G

From the point 36° S 55° W to the intersection of 32° S with the border between Argentina and Chile, then north along the borders of Argentina with Bolivia. Paraguay, Brazil and Uruguay to the point 36° S 55° W.

27/174	Sub-Area 13H

From the point 57° S 90° W and through the point 57° S 70° W to 52° S 70° W. Then along the border between Chile and Argentina to its intersection by 32° S and through the points 36° S 55° W, 57° S 55° W, 57° S 25° W to the South Pole and then to the point 57° S 90° W.

27/175	Sub-Area 13I

From the point 40° S 50° W through the point 36° S 55° W and along the borders of Uruguay with Argentina and Brazil, then through the point 35° S 45° W to the point 40° S 50° W.

27/176	Sub-Area 13J

From the point 15° S 47° W through the points 20° S 44° W, 23° l9' S 42° W, 29° S 40° W, 35° S 45° W, and thence along the borders of Brazil with Uruguay, Argentina, Paraguay and Bolivia to the point 19° 52' S 58° W, then through the point 18° S 57° 37' W to the point 15° S 47° W.

27/177	Sub-Area 13K

From the point 22° 30' S 50° 39' W and through the points 25° S 45° W, 29° S 40° W, 20° S 32° W, 00° 32° W, 04° 24' N 47° W, 04° 24' N 50° 39' W to the point 22° 30' S 50° 39' W.

27/178	Sub-Area 13L

From the point 00° 32° W through the points 00° 20° W, the South Pole, 57° S 55° W, 36° S 55° W, 40° S 50° W, 20° S 32° W, to the point 00° 32° W.

27/179	Regional and Domestic Air Route Area – 14 (RDARA-14)

From the South Pole along the 110° E meridian to 10° S. Then through the points 10° S 145° E, 19° S 153° E, 27° S 160° E. Then along the 160° E meridian to the South Pole.

27/180	Sub-Area 14A

From the South Pole along the 110° E meridian to 19° S. Then through the points 19° S 118° E, 24° S 120° E, 24° S 131° E. Then along the 131° E meridian to the South Pole.

27/181	Sub-Area 14B

From the point 19° S 110° E to the point 10° S 110° E, thence through 10° S 131° E, 24° S 131° E, 24° S 120° E, 19° S 118° E to the point 19° S 110° E.

27/182	Sub-Area 14C

From the point 24° S 131° E to the point 10° S 131° E, thence through 10° S 139° E, 24° S 139° E to the point 24° S 131° E

27/183	Sub-Area 14D

From the South Pole along the 131° E meridian to 24° S, then through the points 24° S 139° E, 27° S 139° E, 27° S 142° E, 34° S 142° E, 34° S 139° E. Then along the 139° E meridian to the South Pole.

27/184	Sub-Area 14E

From the point 24° S 139° E along the 139° E meridian to 10° S, then through the points 10° S 145° E, 19° S 153° E to the point 24° S 139° E.

27/185	Sub-Area 14F

From the point 27° S 139° E along the 139° E meridian to 24° S, then through the points 19° S 153° E, 27° S 160° E to the point 27° S 139° E.

27/186	Sub-Area 14G

From the South Pole along the 139° E meridian to 34° S, then through the points 34° S 142° E, 27° S 142° E, 27° S 160° E. Then along the 160° E meridian to the South Pole.

ARTICLE  3

Description of the boundaries of the VOLMET allotment areas 
and VOLMET reception areas

VOLMET Area – AFRICA-INDIAN OCEAN (AFI-MET)

27/187	The AFI-MET allotment area is defined by a line drawn from the point 29° N 20° W, through the points 37° N 03° W, 37° N 36° E, 30° N 35° E, 10° N 52° E, 22° S 60° E, 35° S 35° E, 35° S 15° E, 08° S 15° W, 12° N 20° W, to the point 29° N 20° W.

27/188	The AFI-MET reception area is defined by a line drawn from the point 37° N 03° W, through the points 37° N 36° E, 30° N 35° E, 10° N 52° E, 10° N 100° E, the South Pole, the points 29° N 40° W, 29° N 20° W, to the point 37° N 03° W.

VOLMET Area – NORTH ATLANTIC (NAT-MET)

27/189	The NAT-MET allotment area is defined by a line drawn from the point 41° N 78° W, through the points 51° N 55° W, 24° N 50° W, 24° N 74° W, to the point 41° N 78° W.

27/190	The NAT-MET reception area is defined by a line drawn from the point 24° N 97° W, through the points 24° N 85° W, 75° N 85° W, 75° N 20° W, 00° 20° W, 00° 95° W, to the point 24° N 97° W.

VOLMET Area – EUROPE (EUR-MET)

27/191	The EUR-MET allotment area is defined by a line drawn from the point 33° N 12° W, through the points 54° N 12° W, 70° N 00°, 74° N 40° E, 40° N 36° E, 29° N 35° 30' E, 32° N 13° E, to the point 33° N 12° W.

27/192	The EUR-MET reception area is defined by a line drawn from the point 15° N 20° W, through the points 40° N 50° W, 75° N 50° W, 75° N 45° E, 15° N 45° E, to the point 15° N 20° W.

VOLMET Area – MIDDLE EAST (MID-MET)

27/193	The MID-MET allotment area is defined by a line drawn from the point 50° N 80° E, through the points 29° N 80° E, 27° N 85° E, 16° N 78° E, 22° N 56° E, 16° N 42° E, 30° N 30° E, 51° N 30° E, 57° N 37° E, to the point 50° N 80° E.

27/194	The MID-MET reception area is defined by a line drawn from the point 50° N 80° E, through the points 50° N 90° E, 35° N 90° E, 27° N 85° E, 16° N 78° E, 22° N 56° E, 16° N 42° E, 30° N 30° E, 51° N 30° E, 57° N 37° E, to the point 50° N 80° E.

VOLMET Area – NORTH CENTRAL ASIA (NCA-MET)

27/195	The NCA-MET allotment area is defined by a line drawn from the point 76° N 32° E, through the points 80° N 90° E, 75° N 168° W, 66° N 168° W, 48° N 160° E, 42° N 135° E, 50° N 130° E, 50° N 90° E, 35° N 70° E, 45° N 30° E, 60° N 20° E, to the point 76° N 32° E.

27/196	The NCA-MET reception area is defined by a line drawn from the North Pole, through the points 40° N 168° W, 30° N 140° E, 35° N 70° E, 30° N 20° E, to the North Pole.

VOLMET Area – PACIFIC (PAC-MET)

27/197	The PAC-MET allotment area is defined by a line drawn from the point 52° N 132° E, through the points 63° N 149° W, 38° N 120° W, 50° S 120° W, 50° S 145° E, 28° S 145° E, 03° S 129° E, 22° N 112° E to the point 52° N 132° E.

27/198	The PAC-MET reception area is defined by a line drawn from the point 60° N 100° E through the points 75° N 160°W, 75° N 110° W, 65° S 110° W, 65° S 145° E, 28° S 145° E, 03° S 129° E, 05° N 80° E, 40° N 80° E, to the point 60° N 100° E.

VOLMET Area – SOUTH EAST ASIA (SEA-MET)

27/199	The SEA-MET allotment area is defined by a line drawn from the point 55° N 75° E, through the points 55° N 135° E, 45° N 135° E, 35° N 130° E, 10° N 130° E, 10° S 155° E, 35° S 155° E, 35° S 116° E, 08° N 75° E, 26° N 65° E, to the point 55° N 75° E.

27/200	The SEA-MET reception area is defined by a line drawn from the point 55° N 50° E, through the points 55° N 180°, 50° S 180°, 50° S 70° E, 08° N 70° E, 08° N 50° E, to the point 55° N 50° E.

VOLMET Area – CARIBBEAN (CAR-MET)

27/201	The CAR-MET allotment area is defined by a line drawn from the point 30° N 110° W, through the points 30° N 75° W, 00° 50° W, following the equator to 00° 80° W to the point 30° N 110° W.

27/202	The CAR-MET reception area is defined by a line drawn from the point 40° N 120° W, through the points 40° N 20° W, 25° S 20° W, 25° S 120° W, to the point 40° N 120° W.

VOLMET Area – SOUTH AMERICA (SAM-MET)

27/203	The SAM-MET allotment area is defined by a line drawn from the point 15° N 83° W, through the points 15° N 60° W, 05° S 35° W, 55° S 60° W, 55° S 83° W, to the point 15° N 83° W.

27/204	The SAM-MET reception area is defined by a line drawn from the point 30° N 120° W through the point 30° N 00°, the South Pole, to the point 30° N 120° W.

ARTICLE  4

World-wide allotment areas

27/205	World-wide Area I

The boundaries of this allotment area comprise those of RDARAs 1, 2 and 3.

27/206	World-wide Area II

The boundaries of this allotment area comprise those of RDARAs 10, 11 12A, 12B, 12C, and 12D.

27/207	World-wide Area III

The boundaries of this allotment area comprise those of RDARAs 6, 8, 9 and 14.

27/208	World-wide Area IV

The boundaries of this allotment area comprise those of RDARAs 12E to 12J inclusive and 13.

27/209	World-wide Area V

The boundaries of this allotment area comprise those of RDARAs 4, 5 and 7.

Section II  –  Allotment of frequencies in the aeronautical mobile (R) service

ARTICLE  1

27/210	Frequency allotment Plan by areas

27/211	NOTE a)	* = For the exact nature of a restriction on the use of the frequency concerned, refer to column 3 of the frequency allotment Plan in numerical order of frequencies (Nos. 27/218 to 27/231).

27/212	NOTE b)	The following list does not include the world-wide common (R) and (OR) frequencies of 3 023 kHz and 5 680 kHz. The allotment of these frequencies is shown in Article 2.

27/213     (WRC-2000)

|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| AFI | 2 851 2 878 | 3 419 3 425 3 467 | 4 657 |  | 5 493 5 652 5 658 | 6 559 6 574 6 673 | 8 894 8 903 |  | 11 300 11 330 | 13 273 13 288 13 294 | 17 961 |
| CAR | 2 887 | 3 455 |  |  | 5 520 5 550 | 6 577 6 586 | 8 846 8 918 |  | 11 387 11 396 | 13 297 | 17 907 |
| CEP | 2 869 | 3 413 | 4 657 |  | 5 547 5 574 | 6 673 | 8 843 | 10 057 | 11 282 | 13 300 | 17 904 |
| CWP | 2 998 | 3 455 | 4 666 |  | 5 652 5 661 | 6 532 6 562 | 8 903 | 10 081 | 11 384 | 13 300 | 17 904 |
| EA | 3 016 | 3 485 3 491 |  |  | 5 655 5 670 | 6 571 | 8 897 | 10 042 | 11 396 | 13 297 13 303 13 309 | 17 907 |
| EUR |  | 3 479 |  |  | 5 661 | 6 598 |  | 10 084 |  | 13 288 | 17 961 |
| INO |  | 3 476 |  |  | 5 634 |  | 8 879 |  |  | 13 306 | 17 961 |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| MID | 2 944 2 992 | 3 467 3 473 | 4 669 |  | 5 658 5 667 | 6 625 6 631 | 8 918 8 951 | 10 018 | 11 375 | 13 288 13 312 | 17 961 |
| NAT | 2 872 2 889 2 962 2 971 3 016 | 3 476 | 4 675 |  | 5 598 5 616 5 649 | 6 622 6 628 | 8 825 8 831 8 864 8 879 8 891 8 906 |  | 11 279 11 309 11 336 | 13 291 13 306 | 17 946 |
| NCA | 3 004 3 019 |  | 4 678 |  | 5 646 5 664 | 6 592 |  | 10 096 |  | 13 303 13 315 | 17 958 |
| NP | 2 932 |  |  |  | 5 628 | 6 655 6 661 |  | 10 048 | 11 330 | 13 300 | 17 904 |
| SAM | 2 944 | 3 479 | 4 669 |  | 5 526 | 6 649 | 8 855 | 10 024 10 096 | 11 360 | 13 297 | 17 907 |
| SAT | 2 854 2 935 | 3 452 |  |  | 5 565 | 6 535 | 8 861 |  | 11 291 | 13 315 13 357 | 17 955 |
| SEA |  | 3 470 3 485 |  |  | 5 649 5 655 | 6 556 | 8 942 | 10 066 | 11 396 | 13 309 13 318 | 17 907 |
| SP |  | 3 467 |  |  | 5 559 5 643 |  | 8 867 | 10 084 | 11 327 | 13 300 | 17 904 |
| 1 |  |  |  |  |  | 6 556 |  | 10 021 | 11 363 |  |  |
| 1B | 2 860* 2 881* 2 890 | 3 458* 3 473* 3 488* |  |  | 5 484 5 568 | 6 550 6 595 |  | 10 066 |  |  |  |
| 1C | 2 977 2 983 | 3 464 3 470 | 4 666 |  | 5 577 5 595 | 6 544 | 8 840 |  | 11 366 |  |  |
| 1D | 2 974 2 980 2 989 | 3 410 3 416 3 446 | 4 651 |  | 5 622 5 628 5 637 | 6 604 6 610 | 8 828 | 10 060 | 11 384 |  |  |
| 1E | 2 965 | 3 491 |  |  | 5 583 | 6 667 |  | 10 036 |  |  |  |
| 2 | 2 938 2 950 |  | 4 696 |  | 5 556 | 6 583 6 601 | 8 846 8 855 8 888 | 10 015 10 045 | 11 297 11 360 11 390 | 13 321 13 357 | 17 964 |
| 2A | 2 851* 2 863 2 869 2 875 2 881 2 887* 2 896 2 917 2 926 2 932 2 941 | 3 416* 3 422 3 434 3 440 3 455 | 4 657* 4 672 4 690 |  | 5 481 5 490 5 496 5 502 5 523 5 547 5 559 5 604 | 6 526 6 532 6 547 6 553 6 559 6 565 6 574 6 673 | 8 822* 8 876 8 909 8 939 | 10 048 10 054 | 11 276 11 285 11 294 |  |  |
| * See No. 27/211. (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 2B | 2 857 2 869 2 875 2 881 2 887* 2 896 2 902 2 908 2 914 2 920 2 929 | 3 401 3 407 3 416* 3 422 3 428 3 449 | 4 660 4 672 4 681 4 690 4 693 |  | 5 490 5 496 5 502 5 508 5 520 5 526 5 550 5 574 5 595 5 607 5 613 5 619 | 6 526 6 532 6 562 6 568 6 577 6 655 6 661 6 667 | 8 819 8 834 8 864 | 10 009 10 024 | 11 279 11 333 11 339 |  |  |
| 2C | 2 857 2 863 2 866 2 884 2 893 2 902 2 908 2 914 2 920 2 926 2 932 | 3 401 3 407 3 428 3 434 3 440 3 449 3 455 | 4 657* 4 660 4 681 4 693 |  | 5 481 5 487 5 508 5 514 5 520 5 526 5 550 5 562 5 574 5 586 5 604 | 6 535 6 541 6 547 6 553 6 562 6 568 6 577 6 586 | 8 819 8 834 8 882 8 939 | 10 009 10 024 10 054 | 11 276 11 333 11372 |  |  |
| 3 | 2 893 2 935 |  | 4 693 |  | 5 556 | 6 583 6 589 | 8 846 8 954 | 10 087 | 11 318 11 336 11 360 | 13 267 13 321 | 17 952 |
| 3A | 2 854 2 860 2 869 2 875 2 881 2 887* 2 896 2 905 2 911* 2 923* 2 959 | 3 404 3 416* 3 422 3 431* 3 443 3 452 | 4 672 4 684 4 690 |  | 5 484 5 490 5 496 5 502 5 511 5 517 5 568 5 580 5 601 5 625 | 6 526 6 532 6 538 6 544 6 550 6 556 6 607 6 613 6 619 6 649 | 8 837 8 861 8 900 8 942 | 10 045 10 057 | 11 309 11 324 11 330 |  |  |
| 3B | 2 851 2 854 2 872 2 878 2 884* 2 902 2 908 2 914 2 968* | 3 401 3 407 3 413 3 419 3 425 3 431* 3 437* 3 443 | 4 657 4 681 |  | 5 493 5 499 5 505 5 514 5 520 5 526 5 550 5 562 5 580 5 601 | 6 529 6 538 6 544 6 559 6 568 6 577 6 595 6 625 6 631 | 8 822 8 852 8 861 8 879 8 957 | 10 024 10 039 | 11 285 11 291 11 327 11 372 |  |  |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 3C | 2 851 2 860 2 866* 2 878 2 905 2 950 2 974 2 980 2 986 | 3 404 3 410 3 419 3 425 3 452 | 4 684 |  | 5 484 5 514 5 562 5 568 5 586 5 637 5 643 | 6 550 6 556 6 595 6 658 6 664 6 670 | 8 837 8 852 8 894 8 915 | 10 039 | 11 291 11 303 11 324 11 378 |  |  |
| 4 |  |  |  |  |  | 6 565 | 8 873 |  |  | 13 300 | 17 904 |
| 4A | 2 926* 2 953 | 3 437 3 491 | 4 672* |  | 5 547 5 559 | 6 526 6 532 6 616 | 8 816 8 837 8 858 | 10 039 10 081 | 11 282 11 318 |  |  |
| 4B | 2 866 2 893 | 3 443 |  |  | 5 481 5 574 5 604 | 6 553 6 577 6 598 |  | 10 063 | 11 324 |  |  |
| 5 |  |  |  |  |  |  | 8 870 8 885 | 10 012 | 11 312 11 327 | 13 354 | 17 949 17 967 |
| 5A | 2 986 | 3 452 |  |  | 5 577 5 583 | 6 544 6 664 | 8 822 8 915 |  | 11 288 |  |  |
| 5B | 2 911 2 968 | 3 431 3 488 |  |  | 5 511 5 568 5 625 | 6 550 6 595 | 8 912 | 10 093 |  |  |  |
| 5C | 2 905 | 3 452 |  |  | 5 583 | 6 544 | 8 822 |  |  |  |  |
| 5D | 2 899 2 971 | 3 482 |  |  | 5 526 5 550 | 6 535 6 547 | 8 843 | 10 048 |  |  |  |
| 6 |  |  |  |  |  |  | 8 840 |  | 11 381 | 13 291 | 17 943 |
| 6A | 2 872 2 923 2 947 3 001 | 3 479 | 4 657* 4 675 |  | 5 484 5 580 5 601 | 6 607 6 613 6 658 | 8 891 8 906 8 948 | 10 006 10 051 10 081* | 11 321 11 357 |  |  |
| 6B | 2 857 2 920 | 3 479 3 488 |  |  | 5 502 5 595 5 625 | 6 607 6 613 6 619 | 8 864 8 885 | 10 021 10 093 | 11 339 11 366 |  | 17 955 |
| 6C | 2 881 2 956 | 3 473 | 4 651 |  | 5 550 5 580 | 6 544 6 631 | 8 834 8 918 | 10 015 |  |  |  |
| 6D | 2 866 2 884 | 3 416 |  |  | 5 490 5 520 5 568 5 574 5 631 | 6 550 6 568 6 577 6 595 | 8 882 8 957 |  | 11 309 11 372 |  |  |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 6E | 2 854 2 872 2 917 3 001 | 3 443 | 4 657* 4 675 |  | 5 514 5 526 5 550 | 6 583 6 655 6 661 | 8 861* 8 906 8 909 | 10 036 10 051 10 084 | 11 357 11 363 |  |  |
| 6F | 2 926 2 941 | 3 434 3 440 |  |  | 5 496 5 508 | 6 526 6 667 | 8 864 8 939 | 10 060 | 11 279 11 366 |  |  |
| 6G | 2 869* 2 875* 2 890 2 896* 2 899 2 902* 2 911* 2 917* 2 938 2 953 2 962 2 968* 2 971 2 977 2 983 2 989 2 995 | 3 413* 3 422* 3 431* 3 437 3 446 3 449* 3 464 3 482 | 4 651* 4 663* 4 669* 4 672* 4 690* 4 696* |  | 5 481 5 487 5 493* 5 499* 5 505* 5 511* 5 517* 5 523 5 547 5 553 5 559 5 565 5 571 5 577 5 583 5 592 5 598 5 604 | 6 529 6 535 6 541 6 547 6 553 6 559 6 565 6 574 6 580 6 586 6 598 6 604 6 610 6 616 6 622 6 628 6 634 6 649 | 8 816 8 825 8 831 8 843 8 858 8 867 8 870* 8 873 8 888* 8 912* 8 960 | 10 018* 10 054* 10 063* | 11 276* 11 282* 11 288 11 294* 11 300* 11 306 11 315 11 369 | 13 270 13 276 | 17 913 |
|  |  |  |  |  | 5 610 5 616 5 622 5 628* 5 634* 5 640* | 6 652 6 673 6 682 |  |  |  |  |  |
| 7 |  |  |  |  | 5 508 | 6 586 | 8 888 |  | 11 285 | 13 354 |  |
| 7B | 2 863 2 965 | 3 455 |  |  | 5 577 5 583 | 6 652 | 8 906 | 10 009 |  |  |  |
| 7C | 2 950 | 3 407 |  |  | 5 592 | 6 568 6 604 | 8 834 | 10 081 | 11 294 |  |  |
| 7D | 2 998 |  |  |  | 5 481 |  |  | 10 096 |  |  |  |
| 7E | 2 887 | 3 485 |  |  | 5 520 | 6 580 6 628 | 8 864 |  | 11 306 |  |  |
| 7F | 2 956 | 3 461 |  |  | 5 547 5 568 | 6 622 | 8 846 8 960 |  |  |  |  |
| 9 |  |  | 4 696 |  | 5 583 | 6 553 | 8 846 8 852 | 10 018 | 11 339 |  |  |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 9B | 2 860 2 905 2 929* | 3 401* 3 419 3 425 3 476* | 4 660 |  | 5 484 5 508 5 523 5 565 | 6 538 6 547 6 598 6 622 | 8 819 8 837 8 861 8 906 | 10 009 10 024 10 039 | 11 393 |  |  |
| 9C | 2 851 | 3 404 3 461 | 4 675 |  | 5 481 | 6 580 | 8 873 | 10 042 | 11 279 11 312 |  |  |
| 9D | 3 016 | 3 404 |  |  | 5 592 | 6 535 | 8 873 |  | 11 312 |  |  |
| 10 |  |  | 4 696 | 5 454 | 5 604 | 6 553 | 8 819 8 834 | 10 006 10 012 | 11 333 11 390 | 13 285 | 17 910 |
| 10A | 2 866 2 875 2 911 2 944 2 956 2 992 | 3 449 3 470 |  | 5 472 5 475 | 5 484 5 490 5 496 5 565 5 631 | 6 535 6 580 6 604 | 8 855 8 876 | 10 066 | 11 357 11 363 11 375 |  |  |
| 10B | 2 854 2 860 | 3 404 3 467 3 488 | 4 651 4 666 4 681 4 690 4 693 | 5 460 5 466 | 5 553 5 568 5 583 | 6 547 6 574 6 598 | 8 837 8 903 8 939 |  |  |  |  |
| 10C | 2 926 2 965 | 3 491 | 4 660 4 669 | 5 457 | 5 481 5 487 5 502 5 562 5 595 | 6 541 6 556 6 568 | 8 867 |  |  |  |  |
| 10D | 2 893 2 935 | 3 419 3 425 3 458 | 4 666 4 669 4 678 | 5 472 5 475 | 5 484 5 490 5 496 5 586 5 625 | 6 535 6 544 6 562 | 8 858 8 900 |  |  |  |  |
| 10E | 2 869 2 944 2 992 | 3 446 3 473 | 4 651 4 666 4 684 | 5 460 | 5 481 5 559 5 577 | 6 547 6 598 | 8 843 8 954 |  | 11 276 |  |  |
| 10F | 2 950 |  | 4 663 | 5 451 | 5 526 | 6 673 | 8 945 | 10 042 |  |  |  |
| 11B | 2 851 2 878 3 004 3 019 | 3 410 3 428 3 434 3 443 | 4 672 | 5 451 5 463 5 469 | 5 508 5 514 5 523 5 571 | 6 538 6 550 6 559 6 565 | 8 822 8 885 8 912 | 10 045 10 093 | 11 288 11 306 | 13 312 | 17 964 |
| 12 |  | 3 440 |  |  | 5 568 |  |  | 10 054 |  |  | 17 901 |
| 12A | 2 950 |  |  |  | 5 604 |  |  |  |  |  |  |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 12C | 2 920 2 980 | 3 401 3 464 | 4 693 | 5 460 | 5 484 5 490 5 496 5 502 5 589 5 613 | 6 535 6 571 6 592 6 622 6 628 | 8 816 8 948 8 957 | 10 021 10 039 | 11 324 |  |  |
| 12D |  | 3 407 |  |  | 5 562 | 6 673 | 8 876 | 10 015 |  |  |  |
| 12E | 2 860 2 956 2 998 | 3 461 3 488 | 4 681 | 5 454 5 475 | 5 481 5 487 5 583 5 595 5 604 | 6 547 6 553 6 598 | 8 852 8 873 | 10 063 10 090 | 11 381 11 393 |  |  |
| 12F | 2 893 2 956 2 965 2 998 | 3 461 3 488 |  | 5 451 5 475 | 5 508 5 556 5 583 5 604 | 6 532 6 553 | 8 873 8 894 | 10 090 | 11 297 |  |  |
| 12G | 2 875 2 956 2 998 | 3 461 3 488 |  |  | 5 484 5 523 5 559 5 646 | 6 526 6 616 |  |  |  |  |  |
| 12H | 2 956 2 998 | 3 461 3 488 |  | 5 451 | 5 583 |  |  |  |  |  |  |
| 12J | 2 860 2 902 2 926 2 965 | 3 419 |  |  | 5 481 5 496 5 619 | 6 535 6 547 | 8 954 |  | 11 381 11 384 |  |  |
| 13 |  |  |  |  |  |  |  |  |  | 13 318 | 17 913 |
| 13A |  |  |  |  |  |  |  | 10 048 |  |  | 17 967 |
| 13B |  |  |  |  |  |  |  | 10 048 |  |  | 17 967 |
| 13C | 2 863 2 869 2 992 | 3 413 3 458 3 473 |  |  | 5 490 5 514 5 553 5 571 5 577 | 6 541 6 556 6 562 6 568 6 580 | 8 819 8 834 8 843 8 939 | 10 042 | 11 327 11 375 | 13 309 |  |
| 13D | 2 914 2 983 | 3 425 3 467 | 4 660 | 5 460 | 5 562 | 6 622 6 628 6 673 | 8 867 8 912 8 957 | 10 084 | 11 318 |  |  |
| 13E | 2 851 | 3 491 | 4 651 4 663 |  | 5 481 5 583 5 604 | 6 553 6 577 | 8 858 |  | 11 303 |  | 17 967 |
| 13F | 2 851 2 956 2 998 | 3 446 3 476 | 4 651 4 663 | 5 454 | 5 481 5 583 5 604 | 6 547 6 553 | 8 831 8 858 8 864 | 10 081 | 11 321 11 330 |  | 17 967 |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| 13G | 2 872 2 971 3 016 | 3 434 3 470 | 4 675* | 5 469 5 475 | 5 574 | 6 586 6 613 | 8 822 8 885 8 900 | 10 006 10 021 10 036 | 11 369 |  |  |
| 13H | 2 899 2 965 | 3 455 3 485 | 4 657 | 5 463 5 472 | 5 484 5 547 | 6 598 | 8 825 8 906 | 10 036 10 045 | 11 282 11 300 | 13 267 |  |
| 13I | 2 860 2 878 2 887 | 3 419 | 4 678 4 693 | 5 451 5 466 | 5 496 5 523 | 6 574 | 8 873 | 10 051 |  |  |  |
| 13J | 2 857 2 863 2 878 2 890 2 920 | 3 410 3 428 3 458 | 4 684 4 696 | 5 451 5 454 | 5 559 5 568 5 577 | 6 550 6 559 6 580 | 8 816 8 843 | 10 012 10 018 10 042 | 11 276 |  |  |
| 13K | 2 863 2 932 3 004 3 019 | 3 401 3 458 3 464 | 4 663 4 672 | 5 463 | 5 481 5 547 5 577 5 604 | 6 547 6 553 6 580 | 8 843 8 849 8 945 | 10 009 10 018 10 042 10 060 | 11 339 11 366 | 13 309 |  |
| 13M | 2 908 2 977 | 3 437 3 449 | 4 660 4 690 | 5 463 | 5 502 | 6 574 6 628 | 8 837 8 867 8 903 | 10 066 | 11 378 |  |  |
| 13N | 2 986 | 3 443 |  | 5 457 | 5 508 | 6 604 | 8 828 | 10 093 |  |  |  |
| 14 | 2 851 2 878 | 3 446 3 461 3 479 |  |  | 5 526 5 604 | 6 580 6 628 | 8 822 8 855 8 870 | 10 045 10 087 | 11 360 | 13 264 | 17 946 |
| 14A | 2 950 | 3 413 | 4 678* |  |  | 6 547 6 553 | 8 816 8 894 |  |  |  |  |
| 14B |  | 3 488 | 4 684* |  |  | 6 535 6 604 6 673 | 8 900 8 954 |  |  |  |  |
| 14C | 2 887 | 3 452 | 4 684* |  |  | 6 541 6 586 | 8 885 8 912 |  |  |  |  |
| 14D | 2 950 | 3 407 | 4 693* |  | 5 481 | 6 559 6 574 | 8 843 8 858 |  |  |  |  |
| 14E |  | 3 413 |  |  |  | 6 565 6 616 | 8 891 8 945 |  |  |  |  |
| 14F |  | 3 488 |  |  |  | 6 526 6 610 | 8 825 8 831 |  |  |  |  |
| 14G | 2 869 2 944 |  | 4 678* |  | 5 481 5 550 5 580 |  | 8 876 8 957 |  |  |  |  |
| VAFI | 2 860 | 3 404 |  |  | 5 499 | 6 538 | 8 852 | 10 057 |  | 13 261 |  |
| VCAR | 2 950 |  |  |  | 5 580 |  |  |  | 11 315 |  |  |
| (See cont.) |  |  |  |  |  |  |  |  |  |  |  |

| (Cont.) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency bands (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| Area | 3 | 3.5 | 4.7 | 5.4 (Reg. 2) | 5.6 | 6.6 | 9 | 10 | 11.3 | 13.3 | 18 | 22 |
|  | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz | kHz |
| VEUR | 2 998 | 3 413 |  |  | 5 640 | 6 580 | 8 957 |  | 11 378 | 13 264 |  |  |
| VMID | 2 956 |  |  |  | 5 589 |  | 8 945 |  |  | 11 393 |  |  |
| VNAT | 2 905 | 3 485 |  |  | 5 592 | 6 604 | 8 870 | 10 051 |  | 13 270 13 276 |  |  |
| VNCA |  | 3 461 | 4 663 |  | 5 676 |  |  | 10 090 |  | 13 279 |  |  |
| VPAC | 2 863 |  |  |  |  | 6 679 | 8 828 |  |  | 13 282 |  |  |
| VSAM | 2 881 |  |  |  | 5 601 |  |  | 10 087 |  | 13 279 |  |  |
| VSEA | 2 965 | 3 458 |  |  | 5 673 | 6 676 | 8 849 |  | 11 387 | 13 285 |  |  |
| W I | 3 010 |  | 4 654 4 687 |  | 5 529 5 532 5 535 5 541 | 6 637 6 643 | 8 921 8 924 8 930 8 936 | 10 027 10 030 10 069 10 072 10 078 | 11 345 11 351 | 13 324 13 327 13 333 13 336 13 342 13 345 13 351 | 17 916 17 922 17 931 | 21 940 21 946 21 952 21 958 21 967 21 973 21 979 21 988 21 997 |
| W II | 3 007 3 013 | 3 494 3 497 | 4 654 4 687 |  | 5 529 5 538 5 544 | 6 637 6 640 6 646 | 8 927 8 933 8 936 | 10 027 10 033 10 075 | 11 342 11 348 11 354 | 13 330 13 339 13 348 | 17 919 17 925 17 934 17 940 | 21 964 21 985 |
| W III | 3 007 |  | 4 687 |  |  | 6 637 | 8 921 8 930 | 10 072 10 078 | 11 342 11 351 | 13 324 13 333 13 342 13 351 | 17 916 17 922 17 928 17 934 17 940 | 21 949 21 970 |
| W IV | 3 010 |  |  |  | 5 535 5 541 | 6 643 | 8 924 | 10 030 10 069 | 11 345 | 13 327 13 336 13345 | 17 919 17 928 17937 | 21 955 21 976 21 991 |
| W V | 3 013 |  |  |  | 5 532 5 538 5 544 | 6 640 6 646 | 8 927 8 933 | 10 033 10 075 | 11 348 11 354 | 13 330 13 339 13 348 | 17 925 17 931 17 937 | 21 943 21 961 21 982 21 994 |

ARTICLE  2

Frequency allotment Plan 
(in numerical order of frequencies)

General Notes:

27/214	1	Class of stations: FD

Classes of emission: see Nos. 27/56 to 27/59.

Power: Unless otherwise indicated in the Plan, the power values for aeronautical and aircraft stations are those shown in Nos. 27/60 to 27/68.

Hours: H24, unless otherwise indicated.

27/215	2	A frequency allotted on a “day-time basis” may be used during the period one hour after sunrise to one hour before sunset

27/216	3	A “common channel” is a channel allotted in common to two or more areas within interference distance of each other and its use is subject to agreement between the administrations concerned.

27/217	4	The world-wide frequency allotments appearing in the Tables at No. 27/213 and Nos. 27/218 to 27/231, except for carrier (reference) frequencies 3 023 kHz and 5 680 kHz, are reserved for assignment by administrations to stations operating under authority granted by the administration concerned, for the purpose of serving one or more aircraft operating agencies. Such assignments are to provide communications between an appropriate aeronautical station and an aircraft station anywhere in the world for exercising control over regularity of flight and for safety of aircraft. World-wide frequencies are not to be assigned by administrations for MWARA, RDARA and VOLMET purposes. Where the operational area of an aircraft lies wholly within a RDARA or Sub-RDARA boundary, frequencies allotted to those RDARAs and Sub-RDARAs shall be used.

| 27/218 Band 2 850-3 025 kHz | 3 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 2 851 | M AFI R 2A 3B 3C 9C 11B 13E 13F 14 |  | CC 3B 3C CC 13E 13F C001/2A |
| 2 854 | M SAT R 3A 3B 6E 10B |  | CC 3A 3B |
| 2 857 | R 2B 2C 6B 13J |  | CC 2B 2C |
| 2 860 | R 1B 3A 3C 9B 10B 12E 12J 13I V VAFI |  | CC 3A 3C CC 12E 12J C001/1B |
| 2 863 | R 2A 2C 7B 13C 13J 13K V VPAC |  | CC 2A 2C CC 13C 13J 13K |
| 2 866 | R 2C 3C 4B 6D 10A |  | C001/3C |
| 2 869 | M CEP R 2A 2B 3A 6G 10E 13C 14G |  | CC 2A 2B 3A C009/6G |
| 2 872 | M NAT R 3B 6A 6E 13G |  | CC 6A 6E |
| 2 875 | R 2A 2B 3A 6G 10A 12G |  | CC 2A 2B 3A C009/6G |
| 2 878 | M AFI R 3B 3C 11B 13I 13J 14 |  | CC 3B3C CC 13I 13J |
| 2 881 | R 1B 2A 2B 3A 6C V VSAM |  | CC 2A 2B 3A C001/lB |
| 2 884 | R 2C 3B 6D |  | C001/3B |
| 2 887 | M CAR R 2A 2B 3A 7E 13I 14C |  | CC 2A2B 3A C001/2A 2B 3A |
| 2 890 | R 1B 6G 13J |  |  |
| 2 893 | R 2C 3 4B 10D 12F |  | CC 2C 3 |
| 2 896 | R 2A 2B 3A 6G |  | CC 2A 2B 3A C009/6G |
| 2 899 | M NAT R 5D 6G 13H |  |  |
| 2 902 | R 2B 2C 3B 6G 12J |  | CC 2B 2C 3B C009/6G |
| 2 905 | R 3A 3C 5C 9B V VNAT |  | CC 3A 3C |
| 2 908 | R 2B 2C 3B 13M |  | CC 2B 2C 3B |
| * See page AP27-75. |  | (See cont.) |  |

| Band 2 850-3 025 kHz | 3 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 2 911 | R 3A 5B 6G 10A |  | C001/3A C010/6G |
| 2 914 | R 2B 2C 3B 13D |  | CC 2B 2C 3B |
| 2 917 | R 2A 6E 6G |  | C010/6G |
| 2 920 | R 2B 2C 6B 12C 13J |  | CC 2B 2C |
| 2 923 | R 3A 6A |  | C001/3A |
| 2 926 | R 2A 2C 4A 6F 10C 12J |  | CC 2A 2C C001/4A |
| 2 929 | R 2B 9B |  | C001/9B |
| 2 932 | M NP R 2A 2C 13K |  | CC 2A 2C |
| 2 935 | M SAT R 3 l0D |  |  |
| 2 938 | R 2 6G |  | C009/6G |
| 2 941 | R 2A 6F |  |  |
| 2 944 | M MID SAM R l0A l0E 14G |  |  |
| 2 947 | R 6A |  |  |
| 2 950 | R 2 3C 7C l0F 12A 14A 14D V VCAR |  | CC 2 3C CC 14A 14D |
| 2 953 | R 4A 6G |  |  |
| 2 956 | R 6C 7F 10A 12E 12F 12G 12H 13F V VMID |  | CC 12E 12F 12G 12H |
| 2 959 | R 3A |  |  |
| 2 962 | M NAT R 6G |  |  |
| 2 965 | R 1E 7B l0C 12F 12J 13H V VSEA |  | CC 12F 12J |
| 2 968 | R 3B 5B 6G |  | C001/3B C009/6G |
| 2 971 | M NAT R 5D 6G 13G |  |  |
| 2 974 | R 1D 3C |  |  |
| 2 977 | R 1C 6G 13M |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 2 850-3 025 kHz | 3 MHz |  |
| --- | --- | --- |
|  |  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 2 980 | R 1D 3C 12C |  |
| 2 983 | R 1C 6G 13D |  |
| 2 986 | R 3C 5A 13N |  |
| 2 989 | R 1D 6G |  |
| 2 992 | M MID R 10A l0E 13C |  |
| 2 995 | R 6G |  |
| 2 998 | M CWP R 7D 12E 12F 12G 12H 13F V VEUR | CC 12E 12F 12G 12H |
| 3 001 | R 6A 6E | CC 6A 6E |
| 3 004 | M NCA R 11B 13K |  |
| 3 007 | W WORLDWIDE | C100/II III |
| 3 010 | W WORLDWIDE | C100/I IV |
| 3 013 | W WORLDWIDE | C100/II V |
| 3 016 | M EA NAT R 9D 13G |  |
| 3 019 | M NCA R 11B 13K |  |

27/219

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 3 023 | W WORLDWIDE (R) and (OR) |  | See Part II, Section II, Article 3 |
| * See page AP27-75. |  |  |  |

| 27/220 Band 3 400-3 500 kHz | 3.5 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 3 401 | R 2B 2C 3B 9B 12C 13K |  | CC 2B 2C 3B C001/9B |
| 3 404 | R 3A 3C 9C 9D 10B V VAFI |  | CC 3A 3C CC 9C 9D |
| 3 407 | R 2B 2C 3B 7C 12D 14D |  | CC 2B 2C 3B |
| 3 410 | R 1D 3C 11B 13J |  |  |
| 3 413 | M CEP R 3B 6G 13C 14A 14E V VEUR |  | CC 14A 14E C009/6G |
| 3 416 | R 1D 2A 2B 3A 6D |  | CC 2A 2B 3A C001/2A 2B 3A |
| 3 419 | M AFI R 3B 3C 9B 10D 12J 13I |  | CC 3B 3C |
| 3 422 | R 2A 2B 3A 6G |  | CC 2A 2B 3A C001/6G C004/6G |
| 3 425 | M AFI R 3B 3C 9B 10D 13D |  | CC 3B 3C |
| 3 428 | R 2B 2C 11B 13J |  | CC 2B 2C |
| 3 431 | R 3A 3B 5B 6G |  | CC 3A 3B C001/3A 3B C009/6G |
| 3 434 | R 2A 2C 6F 11B 13G |  | CC 2A 2C |
| 3 437 | R 3B 4A 6G 13M |  | C001/3B |
| 3 440 | R 2A 2C 6F 12 |  | CC 2A 2C |
| 3 443 | R 3A 3B 4B 6E 11B 13N |  | CC 3A 3B |
| 3 446 | R 1D 6G 10E 13F 14 |  |  |
| 3 449 | R 2B 2C 6G 10A 13M |  | CC 2B 2C C001/6G C004/6G |
| 3 452 | M SAT R 3A 3C 5A 5C 14C |  | CC 3A 3C CC 5A 5C |
| 3 455 | M CAR CWP R 2A 2C 7B 13H |  | CC 2A 2C |
| 3 458 | R 1B 10D 13C 13J 13K V VSEA |  | CC 13C 13J 13K C001/1B |
| 3 461 | R 7F 9C 12E 12F 12G 12H 14 V VNCA |  | CC 12E 12F 12G 12H |
| 3 464 | R 1C 6G 12C 13K |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 3 400-3 500 kHz | 3.5 MHz |  |
| --- | --- | --- |
|  |  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 3 467 | M AFI MID SP R 10B 13D | CC AFI MID |
| 3 470 | M SEA R 1C 10A 13G |  |
| 3 473 | M MID R 1B 6C 10E 13C | C001/1B |
| 3 476 | M INO NAT R 9B 13F | C001/9B |
| 3 479 | M EUR SAM R 6A 6B 14 |  |
| 3 482 | R 5D 6G |  |
| 3 485 | M EA SEA R 7E 13H V VNAT | CC EA SEA |
| 3 488 | R 1B 5B 6B 10B 12E 12F 12G 12H 14B 14F | CC 12E 12F 12G 12H CC 14B 14F C001/lB |
| 3 491 | M EA R 1E 4A 10C 13E | CC 1E 4A |
| 3 494 | W WORLDWIDE | C100/II |
| 3 497 | W WORLDWIDE | C100/II |
| * See page AP27-75. |  |  |

| 27/221 Band 4 650-4 700 kHz | 4.7 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 4 651 | R 1D 6C 6G 10B 10E 13E 13F |  | CC 13E 13F C001/6G |
| 4 654 | W WORLDWIDE |  | C100/I II |
| 4 657 | M AFI CEP R 2A 2C 3B 6A 6E 13H |  | CC 2A 2C C001/2A 2C CC 6A 6E C001/6A 6E |
| 4 660 | R 2B 2C 9B 10C 13D 13M |  | CC 2B 2C CC 13D 13M |
| 4 663 | R 6G 10F 13E 13F 13K V VNCA |  | CC 13E 13F 13K C001/6G |
| 4 666 | M CWP R 1C 10B 10D 10E |  | CC 10B 10D 10E |
| 4 669 | M MID SAM R 6G 10C 10D |  | CC 10C 10D C001/6G |
| 4 672 | R 2A 2B 3A 4A 6G 11B 13K |  | CC 2A 2B 3A C001/4A C001/6G |
| 4 675 | M NAT R 6A 6E 9C 13G |  | CC 6A 6E C001/13G |
| 4 678 | M NCA R 10D 13I 14A 14G |  | CC 14A 14G C001/14A 14G |
| 4 681 | R 2B 2C 3B 10B 12E |  | CC 2B 2C 3B |
| 4 684 | R 3A 3C 10E 13J 14B 14C |  | CC 3A 3C CC 14B 14C C001/14B 14C |
| 4 687 | W WORLDWIDE |  | C100/I II III |
| 4 690 | R 2A 2B 3A 6G 10B 13M |  | CC 2A 2B 3A C001/6G |
| 4 693 | R 2B 2C 3 10B 12C 13I 14D |  | CC 2B 2C 3 C001/14D |
| 4 696 | R 2 6G 9 10 13J |  | C001/6G |
| * See page AP27-75. |  |  |  |

| 27/222 (WRC-2000) Band 5 450-5 480 kHz (Reg. 2) | 5.4 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 5 451 | R l0F 11B 12F 12H 13I 13J | CC 12F 12H CC 13I 13J |
| 5 454 | R 10 12E 13F 13J |  |
| 5 457 | R l0C 13N |  |
| 5 460 | R l0B l0E 12C 13D |  |
| 5 463 | R 11B 13H 13K 13M |  |
| 5 466 | R l0B 13I |  |
| 5 469 | R 11B 13G |  |
| 5 472 | R l0A l0D 13H |  |
| 5 475 | R l0A l0D 12E 12F 13G | CC 12E 12F |

| 27/223 Band 5 480-5 680 kHz | 5.6 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 5 481 | R 2A 2C 4B 6G 7D 9C l0C l0E 12E 12J 13E 13F 13K 14D 14G |  | CC 2A 2C CC 10C 10E CC 12E 12J CC 13E 13F CC 14D 14G |
| 5 484 | R 1B 3A 3C 6A 9B 10A 10D 12C 12G 13H |  | CC 3A 3C |
| 5 487 | R 2C 6G 10C 12E |  |  |
| 5 490 | R 2A 2B 3A 6D 10A 10D 12C 13C |  | CC 2A 2B 3A |
| 5 493 | M AFI R 3B 6G |  | C002/6G |
| 5 496 | R 2A 2B 3A 6F 10A 10D 12C 12J 13I |  | CC 2A 2B 3A |
| 5 499 | R 3B 6G V VAFI |  | C002/6G |
| 5 502 | R 2A 2B 3A 6B 10C 12C 13M |  | CC 2A 2B 3A |
| 5 505 | R 3B 6G |  | C003/6G |
| 5 508 | R 2B 2C 6F 7 9B 11B 12F 13N |  | CC 2B 2C |
| 5 511 | R 3A 5B 6G |  | C002/6G |
| * See page AP27-75. |  | (See cont.) |  |

| Band 5 480-5 680 kHz | 5.6 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 5 514 | R 2C 3B 3C 6E 11B 13C |  | CC 3B 3C |
| 5 517 | R 3A 6G |  | C002/6G |
| 5 520 | M CAR R 2B 2C 3B 6D 7E |  | CC 2B 2C 3B |
| 5 523 | R 2A 6G 9B 11B 12G 13I |  |  |
| 5 526 | M SAM R 2B 2C 3B 5D 6E 10F 14 |  | CC 2B 2C 3B |
| 5 529 | W WORLDWIDE |  | C100/I II |
| 5 532 | W WORLDWIDE |  | C100/I V |
| 5 535 | W WORLDWIDE |  | C100/I IV |
| 5 538 | W WORLDWIDE |  | C100/II V |
| 5 541 | W WORLDWIDE |  | C100/I IV |
| 5 544 | W WORLDWIDE |  | C100/II V |
| 5 547 | M CEP R 2A 4A 6G 7F 13H 13K |  |  |
| 5 550 | M CAR R 2B 2C 3B 5D 6C 6E 14G |  | CC 2B 2C 3B |
| 5 553 | R 6G 10B 13C |  |  |
| 5 556 | R 2 3 12F |  | CC 2 3 |
| 5 559 | M SP R 2A 4A 6G 10E 12G 13J |  |  |
| 5 562 | R 2C 3B 3C 10C 12D 13D |  | CC 3B 3C |
| 5 565 | M SAT R 6G 9B 10A |  |  |
| 5 568 | R 1B 3A 3C 5B 6D 7F 10B 12 13J |  | CC 3A 3C |
| 5 571 | R 6G 11B 13C |  |  |
| 5 574 | M CEP R 2B 2C 4B 6D 13G |  | CC 2B 2C |
| 5 577 | R 1C 5A 6G 7B 10E 13C 13J 13K |  | CC 13C 13J 13K |
| 5 580 | R 3A 3B 6A 6C 14G V VCAR |  | CC 3A 3B |
| 5 583 | R 1E 5A 5C 6G 7B 9 10B 12E 12F 12H 13E 13F |  | CC 5A 5C CC 12E 12F 12H CC 13E 13F |
| 5 586 | R 2C 3C 10D |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 5 480-5 680 kHz | 5.6 MHz |  |
| --- | --- | --- |
|  |  | (Cont.) |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 5 589 | R 12C V VMID |  |  |
| 5 592 | R 6G 7C 9D V VNAT |  |  |
| 5 595 | R 1C 2B 6B 10C 12E |  |  |
| 5 598 | M NAT R 6G |  |  |
| 5 601 | R 3A 3B 6A V VSAM |  | CC 3A 3B |
| 5 604 | R 2A 2C 4B 6G 10 12A 12E 12F 13E 13F 13K 14 |  | CC 2A 2C CC 12E 12F CC 13E 13F |
| 5 607 | R 2B |  |  |
| 5 610 | R 6G |  |  |
| 5 613 | R 2B 12C |  |  |
| 5 616 | M NAT R 6G |  |  |
| 5 619 | R 2B 12J |  |  |
| 5 622 | R 1D 6G |  |  |
| 5 625 | R 3A 5B 6B 10D |  |  |
| 5 628 | M NP R 1D 6G |  | C003/6G |
| 5 631 | R 6D 10A |  |  |
| 5 634 | M INO R 6G |  | C002/6G |
| 5 637 | R 1D 3C |  |  |
| 5 640 | R 6G V VEUR |  | C002/6G |
| 5 643 | M SP R 3C |  |  |
| 5 646 | M NCA R 12G |  |  |
| 5 649 | M NAT SEA |  |  |
| 5 652 | M AFI CWP |  |  |
| 5 655 | M EA SEA |  | CC EA SEA |
| 5 658 | M AFI MID |  | CC AFI MID |
| * See page AP27-75. |  | (See cont.) |  |

| Band 5 480-5 680 kHz | 5.6 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 5 661 | M CWP EUR |  |
| 5 664 | M NCA |  |
| 5 667 | M MID |  |
| 5 670 | M EA |  |
| 5 673 | V VSEA |  |
| 5 676 | V VNCA |  |

27/224

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 5 680 | W WORLDWIDE (R) and (OR) | See Part II, Section II, Article 3 |

| 27/225 Band 6 525-6 685 kHz | 6.6 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 6 526 | R 2A 2B 3A 4A 6F 12G 14F |  | CC 2A 2B 3A |
| 6 529 | R 3B 6G |  |  |
| 6 532 | M CWP R 2A 2B 3A 4A 12F |  | CC 2A 2B 3A |
| 6 535 | M SAT R 2C 5D 6G 9D 10A 10D 12C 12J 14B |  |  |
| 6 538 | R 3A 3B 9B 11B V VAFI |  | CC 3A 3B |
| 6 541 | R 2C 6G 10C 13C 14C |  |  |
| 6 544 | R 1C 3A 3B 5A 5C 6C 10D |  | CC 3A 3B CC 5A 5C |
| * See page AP27-75. |  | (See cont.) |  |

| Band 6 525-6 685 kHz | 6.6 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 6 547 | R 2A 2C 5D 6G 9B 10B 10E 12E 12J 13F 13K 14A |  | CC 2A 2C CC 12E 12J |
| 6 550 | R 1B 3A 3C 5B 6D 11B 13J |  | CC 3A 3C |
| 6 553 | R 2A 2C 4B 6G 9 10 12E 12F 13E 13F 13K 14A |  | CC 2A 2C CC 12E 12F CC 13E 13F |
| 6 556 | M SEA R 1 3A 3C 10C 13C |  | CC 3A 3C |
| 6 559 | M AFI R 2A 3B 6G 11B 13J 14D |  |  |
| 6 562 | M CWP R 2B 2C 10D 13C |  | CC 2B 2C |
| 6 565 | R 2A 4 6G 11B 14E |  |  |
| 6 568 | R 2B 2C 3B 6D 7C 10C 13C |  | CC 2B 2C 3B |
| 6 571 | M EA R 12C |  |  |
| 6 574 | M AFI R 2A 6G 10B 13I 13M 14D |  |  |
| 6 577 | M CAR R 2B 2C 3B 4B 6D 13E |  | CC 2B 2C 3B |
| 6 580 | R 6G 7E 9C 10A 13C 13J 13K 14 V VEUR |  | CC 13C 13J 13K |
| 6 583 | R 2 3 6E |  | CC 2 3 |
| 6 586 | M CAR R 2C 6G 7 13G 14C |  |  |
| 6 589 | R 3 |  |  |
| 6 592 | M NCA R 12C |  |  |
| 6 595 | R 1B 3B 3C 5B 6D |  | CC 3B 3C |
| 6 598 | M EUR R 4B 6G 9B 10B 10E 12E 13H |  |  |
| 6 601 | R 2 |  |  |
| 6 604 | R 1D 6G 7C 10A 13N 14B V VNAT |  |  |
| 6 607 | R 3A 6A 6B |  |  |
| 6 610 | R 1D 6G 14F |  |  |
| 6 613 | R 3A 6A 6B 13G |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 6 525-6 685 kHz | 6.6 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 6 616 | R 4A 6G 12G 14E |  |
| 6 619 | R 3A 6B |  |
| 6 622 | M NAT R 6G 7F 9B 12C 13D |  |
| 6 625 | M MID R 3B |  |
| 6 628 | M NAT R 6G 7E 12C 13D 13M 14 | CC 13D 13M |
| 6 631 | M MID R 3B 6C |  |
| 6 634 | R 6G |  |
| 6 637 | W WORLDWIDE | C100/I II III |
| 6 640 | W WORLDWIDE | C100/II V |
| 6 643 | W WORLDWIDE | C100/I IV |
| 6 646 | W WORLDWIDE | C100/II V |
| 6 649 | M SAM R 3A 6G |  |
| 6 652 | R 6G 7B |  |
| 6 655 | M NP R 2B 6E |  |
| 6 658 | R 3C 6A |  |
| 6 661 | M NP R 2B 6E |  |
| 6 664 | R 3C 5A |  |
| 6 667 | R 1E 2B 6F |  |
| 6 670 | R 3C |  |
| 6 673 | M AFI CEP R 2A 6G 10F 12D 13D 14B |  |
| 6 676 | V VSEA |  |
| 6 679 | V VPAC |  |
| 6 682 | R 6G |  |
| * See page AP27-75. |  |  |

| 27/226 Band 8 815-8 965 kHz | 9 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 8 816 | R 4A 6G 12C 13J 14A |  |  |
| 8 819 | R 2B 2C 9B 10 13C |  | CC 2B 2C |
| 8 822 | R 2A 3B 5A 5C 11B 13G 14 |  | CC 5A 5C C005/2A |
| 8 825 | M NAT R 6G 13H 14F |  |  |
| 8 828 | R 1D 13N V VPAC |  |  |
| 8 831 | M NAT R 6G 13F 14F |  |  |
| 8 834 | R 2B 2C 6C 7C 10 13C |  | CC 2B 2C |
| 8 837 | R 3A 3C 4A 9B 10B 13M |  | CC 3A 3C |
| 8 840 | R 1C 6 |  |  |
| 8 843 | M CEP R 5D 6G 10E 13C 13J 13K 14D |  | CC 13C 13J 13K |
| 8 846 | M CAR R 2 3 7F 9 |  | CC 2 3 |
| 8 849 | R 13K V VSEA |  |  |
| 8 852 | R 3B 3C 9 12E V VAFI |  | CC 3B 3C |
| 8 855 | M SAM R 2 10A 14 |  |  |
| 8 858 | R 4A 6G 10D 13E 13F 14D |  | CC 13E 13F |
| 8 861 | M SAT R 3A 3B 6E 9B |  | CC 3A 3B C011/6E |
| 8 864 | M NAT R 2B 6B 6F 7E 13F |  | CC 6B 6F |
| 8 867 | M SP R 6G 10C 13D 13M |  | CC 13D 13M |
| 8 870 | R 5 6G 14 V VNAT |  | C004/6G |
| 8 873 | R 4 6G 9C 9D 12E 12F 13I |  | CC 9C 9D CC 12E 12F |
| 8 876 | R 2A 10A 12D 14G |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 8 815-8 965 kHz | 9 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 8 879 | M INO NAT R 3B |  |
| 8 882 | R 2C 6D |  |
| 8 885 | R 5 6B 11B 13G 14C |  |
| 8 888 | R 2 6G 7 | C009/6G |
| 8 891 | M NAT R 6A 14E |  |
| 8 894 | M AFI R 3C 12F 14A |  |
| 8 897 | M EA |  |
| 8 900 | R 3A 10D 13G 14B |  |
| 8 903 | M AFI CWP R 10B 13M |  |
| 8 906 | M NAT R 6A 6E 7B 9B 13H | CC 6A 6E |
| 8 909 | R 2A 6E |  |
| 8 912 | R 5B 6G 11B 13D 14C | C004/6G |
| 8 915 | R 3C 5A |  |
| 8 918 | M CAR MID R 6C |  |
| 8 921 | W WORLDWIDE | C100/I III |
| 8 924 | W WORLDWIDE | C100/I IV |
| 8 927 | W WORLDWIDE | C100/II V |
| 8 930 | W WORLDWIDE | C100/I III |
| 8 933 | W WORLDWIDE | C100/II V |
| 8 936 | W WORLDWIDE | C100/I II |
| 8 939 | R 2A 2C 6F 10B 13C | CC 2A 2C |
| 8 942 | M SEA R 3A |  |
| 8 945 | R 10F 13K 14E V VMID |  |
| 8 948 | R 6A 12C |  |
| 8 951 | M MID |  |
| 8 954 | R 3 10E 12J 14B |  |
| 8 957 | R 3B 6D 12C 13D 14G V VEUR |  |
| 8 960 | R 6G 7F |  |
| * See page AP27-75. |  |  |

| 27/227 Band 10 005-10 100 kHz | 10 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 10 006 | R 6A 10 13G |  |  |
| 10 009 | R 2B 2C 7B 9B 13K |  | CC 2B 2C |
| 10 012 | R 5 10 13J |  |  |
| 10 015 | R 2 6C 12D |  |  |
| 10 018 | M MID R 6G 9 13J 13K |  | CC 13J 13K C003/6G |
| 10 021 | R 1 6B 12C 13G |  |  |
| 10 024 | M SAM R 2B 2C 3B 9B |  | CC 2B 2C 3B |
| 10 027 | W WORLDWIDE |  | C100/I II |
| 10 030 | W WORLDWIDE |  | C100/I IV |
| 10 033 | W WORLDWIDE |  | C100/II V |
| 10 036 | R 1E 6E 13G 13H |  | CC 13G 13H |
| 10 039 | R 3B 3C 4A 9B 12C |  | CC 3B 3C |
| 10 042 | M EA R 9C 10F 13C 13J 13K |  | CC 13C 13J 13K |
| 10 045 | R 2 3A 11B 13H 14 |  | CC 2 3A |
| 10 048 | M NP R 2A 5D 13A 13B |  | CC 13A 13B |
| 10 051 | R 6A 6E 13I V VNAT |  | CC 6A 6E |
| 10 054 | R 2A 2C 6G 12 |  | CC 2A 2C C004/6G |
| 10 057 | M CEP R 3A V VAFI |  |  |
| 10 060 | R 1D 6F 13K |  |  |
| 10 063 | R 4B 6G 12E |  | C004/6G |
| 10 066 | M SEA R 1B 10A 13M |  |  |
| 10 069 | W WORLDWIDE |  | C100/I IV |
| 10 072 | W WORLDWIDE |  | C100/I III |
| 10 075 | W WORLDWIDE |  | C100/II V |
| 10 078 | W WORLDWIDE |  | C100/I III |
| 10 081 | M CWP R 4A 6A 7C 13F |  | C006/6A |
| 10 084 | M EUR SP R 6E 13D |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 10 005-10 100 kHz | 10 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 10 087 | R 3 14 V VSAM |  |
| 10 090 | R 12E 12F V VNCA | CC 12E 12F |
| 10 093 | R 5B 6B 11B 13N |  |
| 10 096 | M NCA SAM R 7D |  |

| 27/228 Band 11 275-11 400 kHz | 11.3 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 11 276 | R 2A 2C 6G 10E 13J |  | CC 2A 2C C002/6G |
| 11 279 | M NAT R 2B 6F 9C |  |  |
| 11 282 | M CEP R 4A 6G 13H |  | C003/6G |
| 11 285 | R 2A 3B 7 |  | CC 2A 3B |
| 11 288 | R 5A 6G 11B |  |  |
| 11 291 | M SAT R 3B 3C |  | CC 3B 3C |
| 11 294 | R 2A 6G 7C |  | C002/6G |
| 11 297 | R 2 12F |  |  |
| 11 300 | M AFI R 6G 13H |  | C002/6G |
| 11 303 | R 3C 13E |  |  |
| 11 306 | R 6G 7E 11B |  |  |
| 11 309 | M NAT R 3A 6D |  |  |
| 11 312 | R 5 9C 9D |  | CC 9C 9D |
| 11 315 | R 6G V VCAR |  |  |
| 11 318 | R 3 4A 13D |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 11 275-11 400 kHz | 11.3 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 11 321 | R 6A 13F |  |
| 11 324 | R 3A 3C 4B 12C | CC 3A 3C |
| 11 327 | M SP R 3B 5 13C |  |
| 11 330 | M AFI NP R 3A 13F |  |
| 11 333 | R 2B 2C 10 | CC 2B 2C |
| 11 336 | M NAT R 3 |  |
| 11 339 | R 2B 6B 9 13K |  |
| 11 342 | W WORLDWIDE | C100/II III |
| 11 345 | W WORLDWIDE | C100/I IV |
| 11 348 | W WORLDWIDE | C100/II V |
| 11 351 | W WORLDWIDE | C100/I III |
| 11 354 | W WORLDWIDE | C100/II V |
| 11 357 | R 6A 6E 10A | CC 6A 6E |
| 11 360 | M SAM R 2 3 14 | CC 2 3 |
| 11 363 | R 1 6E 10A |  |
| 11 366 | R 1C 6B 6F 13K | CC 6B 6F |
| 11 369 | R 6G 13G |  |
| 11 372 | R 2C 3B 6D |  |
| 11 375 | M MID R 10A 13C |  |
| 11 378 | R 3C 13M V VEUR |  |
| 11 381 | R 6 12E 12J | CC 12E 12J |
| 11 384 | M CWP R 1D 12J |  |
| 11 387 | M CAR V VSEA |  |
| 11 390 | R 2 10 |  |
| 11 393 | R 9B 12E V VMID |  |
| 11 396 | M CAR EA SEA | CC EA SEA |
| * See page AP27-75. |  |  |

| 27/229 Band 13 260-13 360 kHz | 13.3 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 13 261 | V VAFI |  |  |
| 13 264 | R 14 V VEUR |  |  |
| 13 267 | R 3 13H |  |  |
| 13 270 | R 6G V VNAT |  |  |
| 13 273 | M AFI |  |  |
| 13 276 | R 6G V VNAT |  |  |
| 13 279 | V VNCA VSAM |  |  |
| 13 282 | V VPAC |  |  |
| 13 285 | R 10 V VSEA |  |  |
| 13 288 | M AFI EUR MID |  | CC AFI EUR MID |
| 13 291 | M NAT R 6 |  |  |
| 13 294 | M AFI |  |  |
| 13 297 | M CAR EA SAM |  | CC CAR SAM |
| 13 300 | M CEP CWP NP SP R 4 |  | CC CEP CWP NP SP |
| 13 303 | M EA NCA |  | CC EA NCA |
| 13 306 | M INO NAT |  |  |
| 13 309 | M EA SEA R 13C 13K |  | CC EA SEA CC 13C 13K |
| 13 312 | M MID R 11B |  |  |
| 13 315 | M NCA SAT |  |  |
| 13 318 | M SEA R 13 |  |  |
| 13 321 | R 2 3 |  | CC 2 3 |
| 13 324 | W WORLDWIDE |  | C100/I III |
| 13 327 | W WORLDWIDE |  | C100/I IV |
| 13 330 | W WORLDWIDE |  | C100/II V |
| 13 333 | W WORLDWIDE |  | C100/I III |
| 13 336 | W WORLDWIDE |  | C100/I IV |
| 13 339 | W WORLDWIDE |  | C100/II V |
| 13 342 | W WORLDWIDE |  | C100/I III |
| * See page AP27-75. |  | (See cont.) |  |

| Band 13 260-13 360 kHz | 13.3 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 13 345 | W WORLDWIDE | C100/I IV |
| 13 348 | W WORLDWIDE | C100/II V |
| 13 351 | W WORLDWIDE | C100/I III |
| 13 354 | R 5 7 | CC 5 7 |
| 13 357 | M SAT R 2 |  |

| 27/230 Band 17 900-17 970 kHz | 18 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 17 901 | R 12 |  |  |
| 17 904 | M CEP CWP NP SP R 4 |  | CC CEP CWP NP SP |
| 17 907 | M CAR EA SAM SEA |  | CC CAR SAM CC EA SEA |
| 17 910 | R 10 |  |  |
| 17 913 | R 6G 13 |  |  |
| 17 916 | W WORLDWIDE |  | C100/I III |
| 17 919 | W WORLDWIDE |  | C100/II IV |
| 17 922 | W WORLDWIDE |  | C100/I III |
| 17 925 | W WORLDWIDE |  | C100/II V |
| 17 928 | W WORLDWIDE |  | C100/III IV |
| 17 931 | W WORLDWIDE |  | C100/I V |
| 17 934 | W WORLDWIDE |  | C100/II III |
| 17 937 | W WORLDWIDE |  | C100/IV V |
| 17 940 | W WORLDWIDE |  | C100/II III |
| 17 943 | R 6 |  |  |
| 17 946 | M NAT R 14 |  |  |
| 17 949 | R 5 |  |  |
| * See page AP27-75. |  | (See cont.) |  |

| Band 17 900-17 970 kHz | 18 MHz |
| --- | --- |
|  | (Cont.) |

| Frequency (kHz) | Authorized area of use* | Remarks* |
| --- | --- | --- |
| 1 | 2 | 3 |
| 17 952 | R 3 |  |
| 17 955 | M SAT R 6B |  |
| 17 958 | M NCA |  |
| 17 961 | M AFI EUR INO MID | CC AFI EUR INO MID |
| 17 964 | R 2 11B |  |
| 17 967 | R 5 13A 13B 13E 13F | CC 13A 13B 13E 13F |

| 27/231 Band 21 924-22 000 kHz | 22 MHz |
| --- | --- |

| Frequency (kHz) | Authorized area of use* |  | Remarks* |
| --- | --- | --- | --- |
| 1 | 2 |  | 3 |
| 21 940 | W WORLDWIDE |  | C100/I |
| 21 943 | W WORLDWIDE |  | C100/V |
| 21 946 | W WORLDWIDE |  | C100/I |
| 21 949 | W WORLDWIDE |  | C100/III |
| 21 952 | W WORLDWIDE |  | C100/I |
| 21 955 | W WORLDWIDE |  | C100/IV |
| 21 958 | W WORLDWIDE |  | C100/I |
| 21 961 | W WORLDWIDE |  | C100/V |
| 21 964 | W WORLDWIDE |  | C100/II |
| 21 967 | W WORLDWIDE |  | C100/I |
| 21 970 | W WORLDWIDE |  | C100/III |
| 21 973 | W WORLDWIDE |  | C100/I |
| 21 976 | W WORLDWIDE |  | C100/IV |
| 21 979 | W WORLDWIDE |  | C100/I |
| 21 982 | W WORLDWIDE |  | C100/V |
| 21 985 | W WORLDWIDE |  | C100/II |
| 21 988 | W WORLDWIDE |  | C100/I |
| 21 991 | W WORLDWIDE |  | C100/IV |
| 21 994 | W WORLDWIDE |  | C100/V |
| 21 997 | W WORLDWIDE |  | C100/I |
| * See page AP27-75. |  |  |  |

Explanation of symbols and abbreviations

Column 2 M = MWARA R = RDARA V = VOLMET W = worldwide

Column 3 CC = common channel to

C001/... Restricted to daytime only, in the area indicated after the slant stroke

C002/6G In area 6G, operation is restricted to east of 95° E

C003/6G In area 6G, operation is restricted to west of 95° E

C004/6G Use limited to east of 110° E

C005/2A Use limited to north of 60° N

C006/6A Use limited to east of 75° E

C007 Not used

C008 Not used

C009/6G In area 6G, use limited to east of 110° E and south of 25° N

C010/6G In area 6G, use limited to east of 118° E and north of 40° N

C011/6E In area 6E, use is limited to south of 20° N

C100/... Worldwide Allotment Area is indicated after the symbol. For assignment procedure see No. 27/217.

ARTICLE  3

Frequencies for common use

27/232	1	The carrier (reference) frequencies 3 023 kHz and 5 680 kHz are intended for common use on a world-wide basis.

27/233	2	The use of these frequencies in any part of the world is authorized:

2.1	aboard aircraft for:

a)	communications with approach and aerodrome control;

b)	communication with an aeronautical station when other frequencies of the station are either unavailable or unknown;

2.2	at aeronautical stations for aerodrome and approach control under the following conditions:

a)	with mean power limited to a value of not more than 20 W in the antenna circuit;

b)	special attention must be given in each case to the type of antenna used in order to avoid harmful interference;

c)	the power of aeronautical stations which use these frequencies in accordance with the above conditions may be increased to the extent necessary to meet certain operational requirements subject to coordination between the administrations directly concerned and those whose services may be adversely affected.

27/234	3	Notwithstanding these provisions, the frequency 5 680 kHz may also be used at aeronautical stations for communication with aircraft stations when other frequencies of the aeronautical stations are either unavailable or unknown. However, this use shall be restricted to such areas and conditions that harmful interference cannot be caused to other authorized operations of stations in the aeronautical mobile service.

27/235	4	Additional particulars regarding the use of these channels for the above purposes may be recommended by the meetings of ICAO.

27/236	5	Frequencies 3 023 kHz and 5 680 kHz may also be used by stations of other mobile services participating in coordinated air-surface search and rescue operations, including communications between these stations and participating land stations. Aeronautical stations are authorized to use these frequencies to establish communications with such stations.

27/237	6	These channels may be used for AlA, A1B or A3E emissions, in accordance with special arrangements. Such channels shall not be subdivided.

27/238	7	All stations participating directly in coordinated search and rescue operations and using frequencies 3 023 kHz and 5 680 kHz shall transmit solely on the upper sideband except in the cases provided for in No. 27/57.

APPENDIX 30 (REV.WRC-15)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
         and 12.2-12.7 GHz (in Region 2)    (WRC-03)

(See Articles 9 and 11)     (WRC-03)

TABLE  OF  CONTENTS

Page

Article 1	General definitions		3

Article 2	Frequency bands		4

Article 2A	Use of the guardbands		4

Article 3	Execution of the provisions and associated Plans		5

Article 4	Procedures for modifications to the Region 2 Plan or for additional uses in Regions 1 and 3		6

Article 5	Notification, examination and recording in the Master International Frequency Register of frequency assignments to space stations in the broadcasting-satellite service		18

Article 6	Coordination, notification and recording in the Master International Frequency Register of frequency assignments to terrestrial stations or to earth stations in the fixed-satellite service (Earth-to-space) affecting frequency assignments to broadcasting-satellite stations in the bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2)		23

Article 7	Coordination, notification and recording in the Master International Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in the bands 11.7-12.2 GHz (in Region 2), 12.2-12.7 GHz (in Region 3) and 12.5-12.7 GHz (in Region 1), and to stations in the broadcasting-satellite service in the band 12.5-12.7 GHz (in Region 3) when frequency assignments to broadcasting-satellite stations in the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3 are involved		24

Page

Article 8	Miscellaneous provisions relating to the procedures		25

Article 9	(SUP - WRC-03)

Article 10	The Plan for the broadcasting-satellite service in the frequency band 12.2-12.7 GHz in Region 2		26

Article 11	Plan for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz in Region 3 and 11.7-12.5 GHz in Region 1		65

Article 12	Relationship to Resolution 507 (REV.WRC-03)*		88

Article 13	Interference		88

Article 14	Period of validity of the provisions and associated Plans		88

ANNEXES

Annex 1	Limits for determining whether a service of an administration is affected by a proposed modification to the Region 2 Plan or by a proposed new or modified assignment in the Regions 1 and 3 List or when it is necessary under this Appendix to seek the agreement of any other administration		89

Annex 2	Basic characteristics to be furnished in notices relating to space stations in the broadcasting-satellite service		94

Annex 3	Method for determining the limiting interfering power flux-density at the edge of a broadcasting-satellite service area in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2) and for calculating the power flux-density produced in these bands by a terrestrial station, or by a transmitting earth station in the fixed-satellite service in the band 12.5-12.7 GHz		94

Annex 4	Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Regions 1 and 3; in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with respect to the Plan or proposed modifications to the Plan in Region 2; in Region 3 (12.2-12.5 GHz) with respect to the Plan, List or proposed new or modified assignments in the List for Region 1		104

Annex 5	Technical data used in establishing the provisions and associated Plans and the Regions 1 and 3 List, which should be used for their application		106

Annex 6	Criteria for sharing between services		142

Annex 7	Orbital position limitations		148

ARTICLE  1     (Rev.WRC-03)

General definitions

1	For the purposes of this Appendix, the following terms shall have the meanings defined below:

1.1	1977 Conference: World Administrative Radio Conference for the Planning of the Broadcasting-Satellite Service in the Frequency Bands 11.7-12.2 GHz (in Regions 2 and 3) and 11.7-12.5 GHz (in Region 1), called in short World Broadcasting-Satellite Administrative Radio Conference (Geneva, 1977) (WARC-77).

1.2	1983 Conference: Regional Administrative Radio Conference for the Planning in Region 2 of the Broadcasting-Satellite Service in the Frequency Band 12.2-12.7 GHz and Associated Feeder Links in the Frequency Band 17.3-17.8 GHz, called in short Regional Administrative Conference for the Planning of the Broadcasting-Satellite Service in Region 2 (Geneva, 1983) (RARC Sat-R2).

1.3	1985 Conference: First Session of the World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilizing It (Geneva, 1985), called in short WARC Orb-85.

1.3A	1997 Conference: World Radiocommunication Conference (Geneva, 1997), called in short WRC-97.

1.3B	2000 Conference: World Radiocommunication Conference (Istanbul, 2000), called in short WRC-2000.

1.4	Regions 1 and 3 Plan: The Plan for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz in Region 3 and 11.7-12.5 GHz in Region 1 contained in this Appendix.

1.5	Region 2 Plan: The Plan for the broadcasting-satellite service in the frequency band 12.2-12.7 GHz in Region 2 contained in this Appendix, together with any modifications resulting from the successful application of the procedures of Article 4.

1.6	Frequency assignment in conformity with the Plan:

–	any frequency assignment which appears in the Regions 1 and 3 Plan; or

–	any frequency assignment which appears in the Region 2 Plan or for which the procedure of Article 4 has been successfully applied.

1.7	Additional use in Regions 1 and 3: For the application of the provisions of this Appendix, additional uses in Regions 1 and 3 are:

–	use of assignments with characteristics different from those appearing in the Regions 1 and 3 Plan and which are capable of causing more interference than the corresponding entries in the Plan;

–	use of assignments in addition to those appearing in the Plan.

1.8	Regions 1 and 3 List of additional uses (hereafter called in short the “List”): The List of assignments for additional uses in Regions 1 and 3 as established by WRC-2000 (see Resolution 542 (WRC-2000)*), as updated following the successful application of the procedure of § 4.1 of Article 4.     (WRC-03)

1.9	Frequency assignment in conformity with the List: Any frequency assignment which appears in the List as updated following successful application of § 4.1 of Article 4.     (WRC-03)

1.10	The broadcasting-satellite service subject to one of the Plans: The broadcasting-satellite service subject to one of the Plans referred to in this Appendix is the broadcasting-satellite service in the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3.     (WRC-03)

ARTICLE  2     (WRC-03)

Frequency bands

2.1	The provisions of this Appendix apply to the broadcasting-satellite service in the frequency bands between 11.7 GHz and 12.2 GHz in Region 3, between 11.7 GHz and 12.5 GHz in Region 1 and between 12.2 GHz and 12.7 GHz in Region 2 and to the other services to which these bands are allocated in Regions 1, 2 and 3, insofar as their relationship to the broadcasting-satellite service in these bands is concerned.

2.2	(SUP - WRC-03)

ARTICLE  2A     (REV.WRC-15)

Use of the guardbands

2A.1	The use of the guardbands defined in § 3.9 of Annex 5 to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service (BSS) is not subject to the application of Section I of Article 9.

2A.1.1	Coordination between assignments intended to provide the space operation functions and assignments of the BSS subject to a Plan shall be effected using the provisions of Article 7.

2A.1.2	Coordination among assignments intended to provide the space operation functions and services not subject to a Plan shall be effected using the provisions of Nos. 9.7, 9.17, 9.18 and the associated provisions of Section II of Article 9, or § 4.1.1 d) or 4.2.3 d) of Article 4, as appropriate.

2A.1.3	Coordination of modifications to the Region 2 Plan or assignments to be included in the Regions 1 and 3 List with assignments intended to provide these functions shall be effected using § 4.1.1 e) or 4.2.3 e), as appropriate, of Article 4.

2A.1.4	Requests for the coordination referred to in §2A.1.1, 2A.1.2 and 2A.1.3 shall be sent by the requesting administration to the Bureau, together with the appropriate information listed in Appendix 4.

2A.2	Any assignment intended to provide these functions in support of a geostationary-satellite network in the BSS shall be notified under Article 11 and brought into use within the following time-limits1bis:    (WRC-15)

2A.2.1	a)	for the case where the associated BSS assignments are contained in one of the initial Plans (Region 2 Plans incorporated in the Radio Regulations at WARC Orb-85 and the Regions 1 and 3 Plan adopted at WRC-2000), within the regulatory time-limit referred to in § 4.1.3 or § 4.2.6 of Article 4 from the date of receipt by the Bureau of the complete Appendix 4 data for those assignments intended to provide the space operation functions;

2A.2.2	b)	for the case where the associated BSS assignments have been submitted under § 4.1.3 or § 4.2.6 of Article 4 for entry in the Regions 1 and 3 List or a modification to the Region 2 Plan, within the regulatory time-limit referred to in § 4.1.3 or § 4.2.6 of Article 4 for those associated BSS assignments;

2A.2.3	c)	for the case where the associated BSS assignments have already been brought into use in accordance with the Radio Regulations, within the regulatory time-limit referred to in § 4.1.3 and § 4.2.6 of Article 4 from the date of receipt by the Bureau of the complete Appendix 4 data for those assignments intended to provide the space operation functions.

2A.3	Section II of Article 23 does not apply to assignments in the guardbands intended to provide the above-mentioned functions.

ARTICLE  3     (WRC-2000)

Execution of the provisions and associated Plans

3.1	The Member States in Regions 1, 2 and 3 shall adopt, for their broadcasting-satellite space stations2 operating in the frequency bands referred to in this Appendix, the characteristics specified in the appropriate Regional Plan and the associated provisions.

3.2	The Member States shall not change the characteristics specified in the Regions 1 and 3 Plan or in the Region 2 Plan, or bring into use assignments to broadcasting-satellite space stations or to stations in the other services to which these frequency bands are allocated, except as provided for in the Radio Regulations and the appropriate Articles and Annexes of this Appendix.

3.3	The Regions 1 and 3 Plan is based on national coverage from the geostationary-satellite orbit. The associated procedures contained in this Appendix are intended to promote long-term flexibility of the Plan and to avoid monopolization of the planned bands and orbit by a country or a group of countries.

ARTICLE  4     (Rev.WRC-15)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

## 4.1	Provisions applicable to Regions 1 and 3

4.1.1	An administration proposing to include a new or modified assignment in the List shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations:

a)	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service which is included in the Regions 1 and 3 Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a frequency assignment included in the List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a frequency assignment to a space station in the broadcasting-satellite service which is in conformity with the Region 2 Plan, or in respect of which proposed modifications to that Plan have been received by the Bureau in accordance with the provisions of § 4.2.6 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

d)	having no frequency assignment in the broadcasting-satellite service with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed assignment, or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the proposed assignment exceeds the prescribed limit as a result of the proposed assignment; or

e)	having a frequency assignment in the band 11.7-12.2 GHz in Region 2 or 12.2-12.5 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master International Frequency Register (Master Register) or for which complete coordination information has been received by the Bureau for coordination under No. 9.7, or under § 7.1 of Article 7.

4.1.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.1.3	An administration, or one4 acting on behalf of a group of named administrations, intending to include a new or modified assignment in the List shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the List shall lapse if it is not brought into use within eight years after the date of receipt by the Bureau of the relevant complete information5. A proposed new or modified assignment not included in the List within eight years after the date of receipt by the Bureau of the relevant complete information shall also lapse5.      (WRC-07)

4.1.3bis	The regulatory time-limit for bringing into use of an assignment in the List may be extended once by not more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit6. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 4.1.3:

–	date of launch failure;

–	due diligence information as required in Resolutions 49 (Rev.WRC-15) for the assignment with respect to the satellite that suffered the launch failure, if that information has not already been provided.

If, 11 months after the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information, the Bureau shall promptly send a reminder to the notifying administration. If, within one year of the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information for the new satellite under procurement, the related frequency assignments shall lapse.     (WRC-15)

4.1.4	If the information received by the Bureau under § 4.1.3 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected. The Bureau shall publish7, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send a telegram/fax to the administration proposing the assignment, drawing its attention to the information contained in the relevant BR IFIC.     (WRC-07)

4.1.6	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of the BR IFIC, drawing their attention to the information it contains.     (WRC-07)

4.1.7	An administration which considers that it should have been identified in the publication referred to under § 4.1.5 above shall, within four months of the date of publication of its relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration's request, it shall publish an addendum to the publication under § 4.1.5.

4.1.7bis	Except as provided under § 4.1.18 to 4.1.20, any inclusion of a new or modified frequency assignment in the Regions 1 and 3 List which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.     (WRC-03)

4.1.8	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.1.9	Comments from administrations identified in the publication referred to under § 4.1.5 above shall be sent to the Bureau and to the administration proposing the modification.    (WRC-15)

4.1.10	An administration that has not notified its agreement either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.1.5 shall be deemed to have not agreed to the proposed assignment unless the provisions of § 4.1.10a to 4.1.10d and § 4.1.21 are applied. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months; or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.    (WRC-15)

4.1.10bis	Thirty days prior to the expiry of the same four-month period, the Bureau shall dispatch a reminder telegram or fax to an administration which has not made its comments under § 4.1.10, bringing the matter to its attention.     (WRC-03)

4.1.10ter	(SUP - WRC-15)

4.1.10a	After the same time period as specified in § 4.1.10, the notifying administration may, pursuant to § 4.1.21, request the Bureau to assist in respect of an administration which has not replied within this time period.     (WRC-15)

4.1.10b 	The Bureau, acting under § 4.1.10a, shall send a reminder to the administration which has not replied, together with the results of its previously published compatibility analysis, containing the change in the values referred to in paragraph 1b of Annex 1 to Appendix 30, requesting a decision.     (WRC-15)

4.1.10c	Fifteen days before the expiry of the 30-day period referred to in § 4.1.10d, the Bureau shall send a reminder to the above-mentioned administration drawing its attention to the consequence of no reply.     (WRC-15)

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-15)

4.1.11	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.1 and the subsequent procedure in cases where:

–	the assignments of any other administration received by the Bureau in accordance with § 4.1.3 or § 4.2.6, or § 2A.1.4 of Article 2A, or § 7.1 of Article 7, or No. 9.7 before this modified proposal is received under § 4.1.12;

–	the assignments of any other administration contained in the Plans or the Lists; or

–	the terrestrial services of any other administration,

are considered as being affected and receive more interference as a result of the modifications than that produced by the initial proposal.     (WRC-15)

4.1.12	If agreement has been reached with the administrations identified in the publication referred to under § 4.1.5 above, the administration proposing the new or modified assignment may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.     (WRC-15)

4.1.12bis	In application of § 4.1.12, an administration may indicate the changes to the information communicated to the Bureau under § 4.1.3 and published under § 4.1.5.     (WRC-03)

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the List, the assignment in question shall be maintained in the List until the end of the period referred to in § 4.1.3 above. After that date this assignment shall lapse unless the agreement of the administrations affected is renewed.     (WRC-03)

4.1.14	Where the proposed assignment involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.1.15	The Bureau shall publish8 in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the List.     (WRC-03)

4.1.16	In case of disagreement on the part of an administration whose agreement has been sought, the requesting administration should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.1.17	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by either one of these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

4.1.18	If, in spite of the application of § 4.1.16 and 4.1.17, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Regions 1 and 3 Plan, or in the Region 2 Plan or for which the procedure of § 4.2 has been initiated, and if the notifying administration insists that the proposed assignment be included in the Regions 1 and 3 List, the Bureau shall provisionally enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the List only if the Bureau is informed that the new assignment in the Regions 1 and 3 List has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.     (WRC-03)

4.1.18bis	When requesting the application of § 4.1.18, the notifying administration shall undertake to meet the requirements of § 4.1.20 and provide to the administration in respect of which § 4.1.18 is applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements. Once an assignment is entered in the List provisionally under the provisions of § 4.1.18, the calculation of the equivalent protection margin (EPM)9 of an assignment in the Regions 1 and 3 List or for which the procedure of Article 4 has been initiated and which was the basis for the disagreement shall not take into account the interference produced by the assignment for which the provisions of § 4.1.18 have been applied.     (WRC-03)

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the List shall be reviewed accordingly.

4.1.20	Should harmful interference be caused by an assignment included in the List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

4.1.21	An administration may, at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.1.22	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

4.1.23	When a frequency assignment included in the List is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the List.

4.1.24	No assignment in the List shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.

4.1.25	Where an administration already having included in the List two assignments (not including those systems notified on behalf of a group of named administrations and included in the List by WRC-2000), in the same channel and covering the same service area, proposes to include in the List a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the List in the same channel and which proposes to include in the List a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-15), this administration shall be deemed to have given its agreement to inclusion in the List of the assignment of the latter administration.      (WRC-15)

4.1.26	The procedure of this Article may be applied by the administration of a new ITU Member State in order to include new assignments in the List. Upon completion of the procedure, the next World Radiocommunication Conference may be requested to consider, among the assignments included in the List after the successful completion of this procedure, the inclusion in the Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.     (WRC-03)

4.1.27	When an administration has successfully applied this procedure and received all the agreements10 required to include in the List assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in the Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in the Plan.

4.1.27bis	Should the assignments mentioned in § 4.1.26 and 4.1.27 over the national territory of the administration not be brought into use within the regulatory time-limit mentioned in § 4.1.3, they would be retained in the List until the end of the world radiocommunication conference immediately following the successful completion of the procedure referred to in § 4.1.26 and 4.1.27 respectively, and thereafter they shall be removed from the List.     (WRC-03)

4.1.28	The List, as updated, shall be published periodically by the Bureau.

4.1.29	New or modified assignments in the List shall be limited to digital modulation.

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification11 to the Region 2 Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments to a space station in the broadcasting-satellite service which are shown in the Region 2 Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 Plan a new frequency assignment to a space station in the broadcasting-satellite service; or

c)	to cancel a frequency assignment to a space station in the broadcasting-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5).

4.2.2	The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1.

4.2.3	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations:

a)	of Regions 1 and 3 having a frequency assignment to a space station in the broadcasting-satellite service which is in conformity with the Regions 1 and 3 Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a frequency assignment included in the List or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a frequency assignment in the Region 2 Plan to a space station in the broadcasting-satellite service in the same or adjacent channel which is in conformity with that Plan, or in respect of which proposed modifications to that Plan have been received by the Bureau in accordance with the provisions of § 4.2.6; or

d)	having no frequency assignment in the broadcasting-satellite service in the channel concerned, but in whose territory the power flux-density value exceeds the prescribed limit as a result of the proposed modification, or having an assignment whose associated service area does not cover the whole of the territory of the administration, and in whose territory outside that service area the power flux-density from the broadcasting-satellite space station subject to this modification exceeds the prescribed limit as a result of the proposed modification; or

e)	having a frequency assignment in the band 12.5-12.7 GHz in Region 1 or 12.2-12.7 GHz in Region 3 to a space station in the fixed-satellite service which is recorded in the Master Register, or for which complete coordination information has been received by the Bureau for coordination under No. 9.7 or under § 7.1 of Article 7; or

f )	having a frequency assignment to a space station in the broadcasting-satellite service in the band 12.5-12.7 GHz in Region 3 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment, and:

–	which is recorded in the Master Register; or

–	for which complete coordination information has been received by the Bureau for coordination under No. 9.712 or under § 7.1 of Article 7;

g)	whose services are considered to be affected.

4.2.4	Not used.

4.2.5	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.2.6	An administration, or one13 acting on behalf of a group of named administrations, intending to make a modification to the Region 2 Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use within eight years after the date of receipt by the Bureau of the relevant complete information14. A request for a modification that has not been included in that Plan within eight years after the date of receipt by the Bureau of the relevant complete information shall also lapse14.     (WRC-07)

4.2.6bis	The regulatory time-limit for bringing into use of an assignment in the Region 2 Plan obtained through application of § 4.2 may be extended once by not more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit15. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 4.2.6:

–	date of launch failure;

–	due diligence information as required in Resolution 49 (Rev.WRC-15) for the assignment with respect to the satellite that suffered the launch failure, if that information has not already been provided.

If, 11 months after the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information, the Bureau shall promptly send a reminder to the notifying administration. If, within one year of the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information for the new satellite under procurement, the related frequency assignments shall lapse.     (WRC-15)

4.2.7	If the information received by the Bureau under § 4.2.6 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.3. The Bureau shall publish16, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send a telegram/fax to the administration proposing the modification to the Region 2 Plan, drawing its attention to the information contained in the relevant BR IFIC.     (WRC-07)

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC, drawing their attention to the information it contains.     (WRC-07)

4.2.10	An administration which considers that it should have been included in the publication referred to under § 4.2.8 above shall, within four months of the date of publication in the relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration’s request, it shall publish an addendum to the publication under § 4.2.8.     (WRC-07)

4.2.11	Except as provided under § 4.2.21A to 4.2.21D, any modification to a frequency assignment which is in conformity with the Region 2 Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.     (WRC-03)

4.2.12	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.2.13	Comments from administrations on the information published pursuant to § 4.2.8 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.2.14	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.2.8 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended by up to three months for an administration that has requested additional information under § 4.2.12 or for an administration that has requested the assistance of the Bureau under § 4.2.22. In the latter case, the Bureau shall inform the administrations concerned of this request.

4.2.14bis	Thirty days prior to the expiry of the same four-month period the Bureau shall dispatch a reminder telegram or fax to an administration which has not made its comments under § 4.2.14, bringing the matter to its attention.     (WRC-03)

4.2.14ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations whose agreements are required for completion of the Article 4 procedure.     (WRC-03)

4.2.15	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.2 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.2.16	If no comments have been received on the expiry of the periods specified in § 4.2.14, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.2.16bis	In application of § 4.2.16, an administration may indicate the changes to the information communicated to the Bureau under § 4.2.6 and published under § 4.2.8.     (WRC-03)

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the Plan, the assignment in question shall be maintained in the Plan until the end of the period referred to in § 4.2.6 above. After that date this assignment in the Plan shall lapse unless the agreement of the administrations affected is renewed.     (WRC-03)

4.2.18	When the proposed modification to the Region 2 Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.2.19	The Bureau shall publish17 in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 Plan and will be considered as a frequency assignment in conformity with the Plan.     (WRC-03)

4.2.20	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement on the part of an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.2.21	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

4.2.21A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Region 2 Plan, or in the Regions 1 and 3 Plan or List, or for which the procedure of § 4.1 or 4.2 has been initiated, and if the notifying administration insists that the proposed assignment be included in the Region 2 Plan, the Bureau shall provisionally enter the assignment in the Region 2 Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Region 2 Plan only if the Bureau is informed that the new assignment in the Region 2 Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.     (WRC-03)

4.2.21B	When requesting the application of § 4.2.21A, the notifying administration shall undertake to meet the requirements of § 4.2.21D and provide to the administration in respect of which § 4.2.21A has been applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements.     (WRC-03)

4.2.21C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44, the status of the assignment in the Region 2 Plan shall be reviewed accordingly.     (WRC-03)

4.2.21D	Should harmful interference be caused by an assignment included in the Region 2 Plan under § 4.2.21A to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the Region 2 Plan under § 4.2.21A shall, upon receipt of advice thereof, immediately eliminate this harmful interference.     (WRC-03)

4.2.22	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.2.23	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

### 4.2.24	Cancellation of frequency assignments

When a frequency assignment in conformity with the Region 2 Plan is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the Region 2 Plan.

### 4.2.25	Master copy of the Region 2 Plan

4.2.25.1	The Bureau shall maintain an up-to-date master copy of the Region 2 Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure set out in this Article. This master copy shall contain the overall equivalent protection margins derived from the Plan as established by the 1983 Conference and those derived from all modifications to the Plan as a result of the successful completion of the modification procedure set out in this Article.

4.2.25.2	An up-to-date version of the Region 2 Plan shall be published by the Secretary-General when justified by the circumstances.

ARTICLE  5     (rev.WRC-15)

Notification, examination and recording in the Master International
Frequency Register of frequency assignments to space stations
in the broadcasting-satellite service18     (WRC-07)

## 5.1	Notification

5.1.1	Whenever an administration19 intends to bring into use a frequency assignment to a space station in the broadcasting-satellite service, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions.     (WRC-03)

5.1.2	For any notification under § 5.1.1, an individual notice for each frequency assignment shall be drawn up as prescribed in Appendix 4, the various Sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful.     (WRC-2000)

5.1.2bis	In application of § 5.1.2, an administration may identify the characteristics of assignments in the Plans or the List as notification and send to the Bureau the changes thereto.     (WRC-03)

5.1.3	Each notice must reach the Bureau not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Bureau not later than three months before that date20.     (WRC-2000)

5.1.4	Any frequency assignment the notice of which reaches the Bureau after the applicable period specified in § 5.1.3 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with § 5.1.3.

5.1.5	Any notice made under § 5.1.1 which does not contain the characteristics specified in Appendix 4 shall be returned by the Bureau immediately by airmail to the notifying administration with the relevant reasons.     (WRC-2000)

5.1.6	Upon receipt of a complete notice, the Bureau shall include its particulars, with the date of receipt, in its BR IFIC, which shall contain the particulars of all such notices received since the publication of the previous Circular.     (WRC-2000)

5.1.7	The Circular shall constitute the acknowledgement to the notifying administration of the receipt of a complete notice.

5.1.8	Complete notices shall be considered by the Bureau in order of receipt. The Bureau shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Bureau shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Bureau until it has reached a finding with respect to such earlier notice.

## 5.2	Examination and recording

### 5.2.1	The Bureau shall examine each notice:

a)	with respect to its conformity with the Constitution, the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to § b), c), d) and e) below);

b)	with respect to its conformity with the appropriate Regional Plan or the Regions 1 and 3 List, as appropriate; or

c)	with respect to the coordination requirements specified in the Remarks column of Article 10 or Article 11; or

d)	with respect to its conformity with the appropriate Regional Plan or the Regions 1 and 3 List, however, having characteristics differing from those in the appropriate Regional Plan or in the Regions 1 and 3 List, in one or more of the following aspects:

–	use of a reduced e.i.r.p.,

–	use of a reduced coverage area entirely situated within the coverage area appearing in the appropriate Regional Plan or in the Regions 1 and 3 List,

–	use of other modulating signals in accordance with the provisions of § 3.1.3 of Annex 5,

–	use of the assignment for transmission in the fixed-satellite service in accordance with No. 5.492,

–	in the case of Region 2, use of an orbital position under the conditions specified in § B of Annex 7,

–	in the case of the notification of Plan assignments, use of an e.i.r.p. which produces a pfd that exceeds the limit of −103.6 dB(W/(m2 · 27 MHz)) given in Section 1 of Annex 1 to Appendix 30 on the territory of the notifying administration under the condition that the calculated pfd at test points of any Plan assignment, List assignment or proposed assignment submitted under Article 4 are equal to or below that of the original Plan assignments in the same channel of the administration applying this section; or

e)	with respect to its conformity with the provisions of Resolution 42 (Rev.WRC-03)*.     (WRC-03)

5.2.2	Where the Bureau reaches a favourable finding with respect to § 5.2.1 a), 5.2.1 b) and 5.2.1 c), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments.     (WRC-07)

5.2.2.1	Where the Bureau reaches a favourable finding with respect to § 5.2.1 a), 5.2.1 c) and 5.2.1 d), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations, all frequency assignments brought into use in conformity with the appropriate Regional Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments. When recording these assignments, the Bureau shall indicate by an appropriate symbol the characteristics having a value different from that appearing in the appropriate regional Plan.     (WRC-07)

5.2.2.2	In the case of Region 2, where the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c), but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.WRC-03)*. A frequency assignment for which the provisions of Resolution 42 (Rev.WRC-03)* have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.WRC-03)* and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments.     (WRC-07)

5.2.2.3	In the case of Regions 1 and 3, when the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), the notice shall be returned immediately by airmail to the notifying administration with the Bureau’s reasons for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.     (WRC-2000)

5.2.3	Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Bureau shall be indicated.     (WRC-07)

5.2.4	Where the Bureau reaches an unfavourable finding with respect to:

–	§ 5.2.1 a), or

–	§ 5.2.1 c), or

–	§ 5.2.1 b) and 5.2.1 d) and, where applicable, § 5.2.1 e),

the notice shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.     (WRC-2000)

5.2.5	Where the notifying administration resubmits the notice and the finding of the Bureau becomes favourable with respect to the appropriate parts of § 5.2.1, the notice shall be treated as in § 5.2.2, 5.2.2.1 or 5.2.2.2, as appropriate.

5.2.6	If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Bureau’s finding with respect to § 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with § 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in § 5.2.5 is fulfilled. For Regions 1, 2 and 3, in the event that the Bureau has been informed of agreement to modification of the Plan for a specified period of time in accordance with Article 4, the frequency assignment shall be recorded in the Master Register with a note indicating that the frequency assignment is valid only for the period specified. The notifying administration using the frequency assignment over a specified period shall not subsequently invoke this fact to justify the continued use of the frequency beyond the period specified unless it obtains the agreement of the administration(s) concerned.

5.2.7	If a frequency assignment notified in advance of bringing into use in conformity with § 5.1.3 has received a favourable finding by the Bureau with respect to the provisions of § 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry.

5.2.8	When the Bureau has received confirmation that the frequency assignment has been brought into use, the Bureau shall remove the symbol in the Master Register.

5.2.9	The date of bringing into use notified by the administration concerned shall be recorded in the Master Register.     (WRC-07)

5.2.10	Wherever the use of a frequency assignment to a space station recorded in the Master Register and emanating from the Regions 1 and 3 List is suspended for a period exceeding six months, the notifying administration shall inform the Bureau of the date on which such use was suspended. When the recorded assignment is brought back into use, the notifying administration shall so inform the Bureau, as soon as possible. On receipt of the information sent under this provision, the Bureau shall make that information available on the ITU website as soon as possible and shall publish it in the BR IFIC. The date on which the recorded assignment is brought back into use20bis shall be no later than three years from the date on which the use of the frequency assignment was suspended, provided that the notifying administration informs the Bureau of the suspension within six months from the date on which the use was suspended. If the notifying administration informs the Bureau of the suspension more than six months after the date on which the use of the frequency assignment was suspended, this three-year time period shall be reduced. In this case, the amount by which the three-year period shall be reduced shall be equal to the amount of time that has elapsed between the end of the six-month period and the date that the Bureau is informed of the suspension. If the notifying administration informs the Bureau of the suspension more than 21 months after the date on which the use of the frequency assignment was suspended, the frequency assignment shall be cancelled.     (WRC-15)

5.2.11	If a recorded frequency assignment stemming from the Regions 1 and 3 List is not brought back into use within the suspension period resulting from the application of § 5.2.10 above, the Bureau shall cancel the assignment from the Master Register and the assignment in the List, unless the assignment is one to which § 4.1.26 or § 4.1.27 is being applied.     (WRC-15)

## 5.3	Cancellation of entries in the Master Register

5.3.1	Any notified frequency assignment to which the Article 4 procedures have been applied and which has been provisionally recorded under § 5.2.7 shall be brought into use no later than the end of the period provided under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4. Any other frequency assignment provisionally recorded under § 5.2.7 shall be brought into use by the date specified in the notice. Unless the Bureau has been informed by the notifying administration of the bringing into use of the assignment under § 5.2.8, it shall, no later than 15 days before the notified date of bringing into use or the end of the regulatory period established under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4, as appropriate, send a reminder requesting confirmation that the assignment has been brought into use within the regulatory period. If the Bureau does not receive that confirmation within 30 days following the notified date of bringing into use or the period provided under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4, as the case may be, it shall cancel the entry in the Master Register.     (WRC-15)

5.3.2	If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Bureau within three months, whereupon the entry shall be removed from the Master Register.

ARTICLE  6     (WRC-2000)

Coordination, notification and recording in the Master International
Frequency Register of frequency assignments to terrestrial stations
or to earth stations in the fixed-satellite service (Earth-to-space)
affecting frequency assignments to broadcasting-satellite stations
in the bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz
(in Region 1) and 12.2-12.7 GHz (in Region 2)21

6.1	The provisions of No. 9.19 and the associated provisions under Articles 9 and 11 are applicable in respect of frequency assignments to broadcasting-satellite stations in the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3:

a)	to transmitting terrestrial stations in the band 11.7-12.7 GHz in all Regions;

b)	to transmitting earth stations in the fixed-satellite service in the band 12.5-12.7 GHz (in Region 1).

6.2	In applying the procedures referred to in § 6.1, the provisions of Appendix 5 are replaced by the following:

6.2.1	These procedures are to be applied in respect of administrations whose territory is included within the service area associated with:

a)	assignments in conformity with the appropriate Regional Plan in Appendix 30;

b)	assignments included in the Regions 1 and 3 List;

c)	assignments for which the procedure of Article 4 has been initiated, as from the date of receipt of the complete Appendix 4 information under § 4.1 or 4.2.

6.2.2	The criteria to be applied are those given in Annex 3.

ARTICLE  7     (Rev.WRC-03)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in the bands 11.7-12.2 GHz (in Region 2),
12.2-12.7 GHz (in Region 3) and 12.5-12.7 GHz (in Region 1), and to stations 
in the broadcasting-satellite service in the band 12.5-12.7 GHz (in Region 3) 
when frequency assignments to broadcasting-satellite stations in
the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2
and 11.7-12.2 GHz in Region 3 are involved22

7.1	The provisions of No. 9.723 and the associated provisions under Articles 9 and 11 are applicable in respect of frequency assignments to broadcasting-satellite stations in the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3:

a)	to transmitting space stations in the fixed-satellite service in the bands 11.7-12.2 GHz (in Region 2), 12.2-12.7 GHz (in Region 3) and 12.5-12.7 GHz (in Region 1); and

b)	to transmitting space stations in the broadcasting-satellite service in the band 12.5-12.7 GHz (in Region 3).

7.2	In applying the procedures referred to in § 7.1, the provisions of Appendix 5 are replaced by the following:

7.2.1	The frequency assignments to be taken into account are:

a)	the assignments in conformity with the appropriate Regional Plan in Appendix 30;

b)	the assignments included in the Regions 1 and 3 List;

c)	the assignments for which the procedure of Article 4 has been initiated, as from the date of receipt of the complete Appendix 4 information under § 4.1.3 or 4.2.6.     (WRC-03)

7.2.2	The criteria to be applied are those given in Annex 4.

ARTICLE  8

Miscellaneous provisions relating to the procedures*

8.1	If so requested by any administration, the Board, using such means at its disposal as are appropriate in the circumstances, shall conduct a study of cases of alleged contravention or non-observance of these provisions or of harmful interference.

8.2	The Board shall thereupon prepare and forward to the administration or administrations concerned a report containing its findings and recommendations for the solution of the problem.

8.3	On receiving the Board’s recommendations for the solution of the problem, an administration shall promptly acknowledge their receipt by telegram and shall indicate the action it intends to take. Where the Board’s suggestions or recommendations are unacceptable to the administrations concerned, further efforts should be made by the Board to find an acceptable solution to the problem.

8.4	Where, as a result of a study, the Board submits to one or more administrations suggestions or recommendations for the solution of a problem, and where no reply has been received from one or more of these administrations within a period of three months, the Board shall consider that the suggestions or recommendations concerned are unacceptable to the administrations which did not answer. If it was the requesting administration which failed to answer within this period, the Board shall discontinue the study.

8.5	If so requested by any administration, particularly by an administration of a country in need of special assistance, the Board, using such means at its disposal as are appropriate in the circumstances, shall render the following assistance:

a)	computation necessary in the application of Annexes 1, 3 and 4;

b)	any other assistance of a technical nature for completion of the procedures in this Appendix.

8.6	In making a request to the Board under § 8.5, the administration shall provide the Board with the necessary information.

ARTICLE  9     (SUP - WRC-03)

ARTICLE  10

The Plan for the broadcasting-satellite service in
the frequency band 12.2-12.7 GHz in Region 2

10.1		COLUMN  HEADINGS  OF  THE  PLAN

Col. 1	Beam identification (Column 1 contains the symbol designating the country or the geographical area taken from Table B1 of the Preface to the International Frequency List followed by the symbol designating the service area).

Col. 2	Nominal orbital position, in degrees and hundredths of a degree.

Col. 3	Channel number (see Table 4 showing channel numbers and corresponding assigned frequencies).

Col. 4	Boresight geographical coordinates, in degrees and hundredths of a degree.

Col. 5	Antenna beamwidth. This column contains two figures corresponding to the major axis and the minor axis respectively of the elliptical cross-section half-power beam, in degrees and hundredths of a degree.

Col. 6	Orientation of the ellipse determined as follows: in a plane normal to the beam axis, the direction of a major axis of the ellipse is specified as the angle measured anti-clockwise from a line parallel to the equatorial plane to the major axis of the ellipse to the nearest degree.

Col. 7	Polarization (1 = direct, 2 = indirect)24.

Col. 8	e.i.r.p. in the direction of maximum radiation, in dBW.

Col. 9	Remarks.

10.2	TEXT  FOR  NOTES  IN  REMARKS COLUMN  OF  THE  PLAN

1	Fast roll-off space station transmitting antenna as defined in Annex 5 (item 3.13.3).

2	Television standard with 625 lines using greater video bandwidth and necessary bandwidth of 27 MHz.

3	Not used

4	This assignment may be utilized in the geographical area of Anguilla (AIA) (which is in the beam area).

5	Feeder-link earth stations for this assignment may also be located in the territories of Puerto Rico and the United States Virgin Islands. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

6	Feeder-link earth stations for this assignment may also be located in the States of Alaska and Hawaii. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

7	The feeder-link earth station for this assignment may also be located at the point with geographical coordinates 3° 31′ West, 48° 46′ North. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

8	Feeder-link earth stations for this assignment may also be located at the points with the following geographical coordinates:

| 47° 55′ West | 15° 47′ South | 34° 53′ West | 08° 04′ South |
| --- | --- | --- | --- |
| 43° 13′ West | 22° 55′ South | 60° 02′ West | 03° 06′ South |
| 46° 38′ West | 23° 33′ South | 38° 31′ West | 12° 56′ South |
| 51° 13′ West | 30° 02′ South | 49° 15′ West | 16° 40′ South |

Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

9/GR . . .	This assignment is part of a group, the number of which follows the symbol. The group consists of the beams and has the number of channels assigned to it as indicated in Table 1 below.

a)	The overall equivalent protection margin to be used for the application of Article 4 and Resolution 42 (Rev.WRC-03)* shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

–	for the calculation of interference from assignments belonging to a group to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.     (WRC-03)

b)	If an administration notifies the same frequency in more than one beam of a group for use at the same time, the aggregated C/I produced by all emissions from that group shall not exceed the C/I calculated on the basis of a) above.

10	This assignment shall be brought into use only when the limits given in Table 2 are not exceeded or with the agreement of the affected administration identified in Table 3.

These administrations shall be informed by the notifying administration of changes in characteristics before these beams are brought into use.

TABLE  1

| Group | Beams in the group | Number of channels assigned to the group |
| --- | --- | --- |
| GR1 | ALS00002 HWA00002 USAPSA02 | 32 channels |
| GR2 | ALS00003 HWA00003 USAPSA03 | 32 channels |
| GR3 | ARGINSU4 ARGSUR04 | 16 channels |
| GR4 | ARGINSU5 ARGSUR05 | 12 channels |
| GR5 | BOLAND01 CLMAND01 EQACAND1 EQAGAND1 PRUAND02 VENAND03 | 16 channels |
| GR6 | B SU111 B SU211 | 32 channels |
| GR7 | B CE311 B CE411 B CE511 | 32 channels |
| GR8 | B NO611 B NO711 B NO811 | 32 channels |
| GR9 | B SU112 B SU212 B CE312 B CE412 | 32 channels |
| GR10 | CAN01101 CAN01201 | 32 channels |
| GR11 | Not used |  |
| GR12 | CAN01203 CAN01303 CAN01403 | 32 channels |
| GR13 | CAN01304 CAN01404 CAN01504 | 32 channels |
| GR14 | CAN01405 CAN01505 CAN01605 | 32 channels |
| GR15 | Not used |  |
| GR16 | CHLCONT4 CHLCONT6 | 16 channels |
| GR17 | CHLCONT5 PAQPAC01 CHLPAC02 | 16 channels |
| GR18 | CRBBER01 CRBBLZ01 CRBJMC01 CRBBAH01 CRBECO01 | 16 channels |
| GR19 | EQACOO01 EQAGOO01 | 16 channels |
| GR20 | PTRVIR01 USAEHO02 | 32 channels |
| GR21 | PTRVIR02 USAEHO03 | 32 channels |
| GR22 | VEN02VEN VEN11VEN | 4 channels |

TABLE  2

Applicable criteria

| Symbol | pfd limit criteria |
| --- | --- |
| a | § 3, Annex 1 |
| b | § 5 b), Annex 1 |
| c | § 5 c), Annex 1 |
| d | § 5 d), Annex 1 |
| Note – Section 5 of Annex 1 was merged with Section 4 by WRC-2000. See also the Note to Table 3. (WRC-2000) |  |

11	This assignment shall be brought into use only when the e.i.r.p. in the direction of all points situated within the service area and within the −3 dB contour of the “Metropole” beam (space-to-Earth) in the VIDEOSAT-3 network as described in ex-IFRB Special Section AR11/C/766 to BR IFIC No. 1678 of 2 July 1985 does not exceed the limit 26.8 dBW.

12	This assignment shall be brought into use only when the e.i.r.p. in the direction of all points situated within the service area and within the −3 dB contour of the “Metropole” beam (space-to-Earth) in the VIDEOSAT-3 network as described in ex-IFRB Special Section AR11/C/766 to Weekly Circular No. 1678 of 2 July 1985 does not exceed the limit 26.8 dBW, and when the e.i.r.p. in the direction of all points situated within the service area and also between the −3 dB and −6 dB contours of the same beam does not exceed the limit 29.5 dBW.

TABLE  3

| Beam name | Channels | Limit criteria ref. Table 2 | Countries or geographical areas affected* |
| --- | --- | --- | --- |
| ALS00002 | 1, 4, 5, 6, 9, 10, 11, 14, 15, 16 All channels For channels 20 to 32 | a c d | URS MNG/URS URS |
| ALS00003 | 1, 4, 5, 6, 9, 10, 11, 14, 15, 16 All channels For channels 20 to 32 | a c d | URS URS URS |
| ARGINSU5 | 3, 7, 11, 15, 17, 19 | b | NOR |
| ARGNORT4 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | b | AOE/ASC/AZR/CPV/E/GMB/GNB/GUI/MRC/MTN/POR/SEN |
| ARGNORT5 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | b | AFS/AGL/BOT/NMB/NOR/OCE/PTC/ TKL/COD/ZMB/ZWE |
| ARGSUR04 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | b | ASC |
| ARGSUR05 | 3, 7, 11, 15, 17, 19 | b | NOR |
| B CE311 | For channels 1 to 20 | b | AGL/ALG/CAF/CME/COG/GAB/GNE/ NGR/NIG/NMB/STP/TCD/COD |
| B CE312 | For channels 1 to 20 For channels 1 to 20 All channels | b c c | AFS/BDI/BOT/LSO/RRW/TZA/UGA/ ZMB/ZWE MOZ/MWI/TZA ETH/KEN/SDN |
| B CE411 | For channels 1 to 20 | b | AGL/ALG/CAF/CME/COG/CVA/E/ GAB/GNE/I/LBY/MLT/NGR/NIG/SMR/STP/TCD/TUN/COD |
| B CE412 | For channels 1 to 20 All channels | c c | CYP/TUR ARS/EGY/ISR/SDN/URS |
| B CE511 | For channels 1 to 20 | b | CAF/CME/COG/GAB/GNE/NIG/NMB/ NOR/STP/COD |
| B NO611 | For channels 1 to 20 | b | BEN/GHA/TGO |
| B NO711 | For channels 1 to 20 | b | BEN |
| B SE911 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | b | CPV |

TABLE  3 (continued )

| Beam name | Channels | Limit criteria ref. Table 2 | Countries or geographical areas affected* |
| --- | --- | --- | --- |
| B SU111 | For channels 1 to 20 | b | BFA/CTI/GHA/GUI/LBR/MTN/SHN/TRC |
| B SU211 | For channels 1 to 20 | b | ALG/BFA/CTI/GHA/GUI/LBR/MLI/MRC/MTN/SHN/TRC |
| BERBER02 | 1, 5, 17 5, 9, 13 | a a | CNR/E ISL |
| BOL00001 | 3, 7, 11, 15, 19 | b | ALG/AOE/ASC/E/GMB/GNB/GUI/LBR/ MLI/MRC/MTN/POR/SEN/SRL/TRC |
| CAN01101 | All channels For channels 20 to 32 | c d | URS URS |
| CAN01201 | All channels | c | URS |
| CAN01203 | All channels | c | URS |
| CAN01303 | All channels | c | URS |
| CAN01403 | All channels | c | URS |
| CAN01404 | For channels 1 to 20 | b | ISL/POR |
| CAN01405 | For channels 1 to 20 | b | F/G/IRL/ISL |
| CAN01504 | For channels 1 to 20 | b | AOE/AZR/E/ISL/MRC/MTN/POR |
| CAN01505 | For channels 1 to 20 | b | ALG/E/F/G/IRL/ISL/MRC/POR |
| CAN01605 | For channels 1 to 20 | b | E/F/G/IRL/ISL/MRC/POR |
| CAN01606 | For channels 1 to 20 | b | BEL/F/G/HOL/IRL/ISL/LUX/NOR |
| CLMAND01 | 21, 23, 25, 27, 29, 31 | c | URS |
| CLM00001 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 21, 23, 25, 27, 29, 31 | b c | AZR/CPV URS |
| CRBEC001 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | b | ASC/AZR/GMB/GNB/GUI/ISL/MTN/ SEN/SRL |
| FLKANT01 | 1, 5, 9, 13 | b | NOR |
| GRLDNK01 | 3, 7, 11, 15, 19 | b | D/DNK/G/HOL/ISL/NOR/POL/S/TCH |
| GUFMGG02 | 4, 8, 12, 16, 20 | b | NOR |
| HWA00002 | For channels 1 to 20 All channels | b c | CHN/KRE MNG/URS |
| HWA00003 | For channels 1 to 20 All channels | b c | CHN MNG/URS |
| MEX02NTE | All channels | c | URS |
| MEX01SUR | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | b | KIR |

TABLE  3 (end )

| Beam name | Channels | Limit criteria ref. Table 2 | Countries or geographical areas affected* |
| --- | --- | --- | --- |
| MEX02SUR | All channels | c | URS |
| PRU00004 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | b | ALG/AOE/ASC/BFA/CTI/E/G/GMB/ GUI/ISL/LBR/MLI/MRC/MTN/POR/ SEN/SHN/SRL/TRC |
| SPMFRAN3 | 1, 5, 9, 13, 17 | b | D/DNK/ISL/NOR/S |
| USAEH001 | For channels 1 to 20 | b | ALG/AUT/BEL/CVA/D/DNK/E/F/G/ HOL/I/ISL/LBY/LIE/LUX/MCO/MLT/ NGR/NIG/NOR/OCE/SMR/SUI/TCH/ TUN/YUG |
| USAEH002 | For channels 1 to 20 All channels | b c | AZR/CPV/HWL URS |
| USAEH003 | For channels 1 to 20 All channels | b c | MHL URS |
| USAEH004 | For channels 1 to 20 All channels For channels 20 to 32 | b c d | WAK URS URS |
| USAWH101 | All channels | c | URS |
| USAWH102 | All channels | c | URS |
| VENAND03 | 21, 23, 25, 27, 29, 31 | c | URS |
| VEN11VEN | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 20, 22, 24, 26, 28, 30, 32 | b c | AZR/CPV URS |
| Note – The administrations listed in Table 3 were identified on the basis of the criteria adopted at the Regional Administrative Conference for the Planning of the Broadcasting-satellite Service in Region 2 (Geneva, 1983) (RARC Sat-R2), as shown in Table 2. WRC-2000 and WRC-03 revised the criteria applicable to determine affected administrations. Therefore, the Bureau, when receiving a notification for an assignment in the Region 2 Plan, shall determine which countries are affected on the basis of the revised criteria adopted by WRC-03, which may lead to a different set of affected administration(s) from that currently contained in Table 3. (WRC-07) |  |  |  |

Country symbols

1	For the explanation of symbols designating countries or geographical areas in Region 2, see the Preface to the International Frequency List.

2	One additional symbol, CRB, has been created for the purposes of the 1983 Conference only, to designate to geographical area in the Caribbean Area. The five Caribbean beams are identified as follows:

CRBBAH01, CRBBER01, CRBBLZ01, CRBEC001 and CRBJMC01

and are intended collectively to provide coverage for the following countries or geographical areas: AIA, ATG, BAH, BER, BLZ, BRB, CYM, DMA, GRD, GUY, JMC, LCA, MSR, KNA, SUR, TCA, TRD, VCT and VRG to be so used if approved by them.

TABLE  4

Table showing correspondence between channel
numbers and assigned frequencies

| Channel No. | Assigned frequency (MHz) | Channel No. | Assigned frequency (MHz) |
| --- | --- | --- | --- |
| 1 | 12 224.00 | 17 | 12 457.28 |
| 2 | 12 238.58 | 18 | 12 471.86 |
| 3 | 12 253.16 | 19 | 12 486.44 |
| 4 | 12 267.74 | 20 | 12 501.02 |
| 5 | 12 282.32 | 21 | 12 515.60 |
| 6 | 12 296.90 | 22 | 12 530.18 |
| 7 | 12 311.48 | 23 | 12 544.76 |
| 8 | 12 326.06 | 24 | 12 559.34 |
| 9 | 12 340.64 | 25 | 12 573.92 |
| 10 | 12 355.22 | 26 | 12 588.50 |
| 11 | 12 369.80 | 27 | 12 603.08 |
| 12 | 12 384.38 | 28 | 12 617.66 |
| 13 | 12 398.96 | 29 | 12 632.24 |
| 14 | 12 413.54 | 30 | 12 646.82 |
| 15 | 12 428.12 | 31 | 12 661.40 |
| 16 | 12 442.70 | 32 | 12 675.98 |

|  |  |  |  |  |  |  |  |  |  | 12 224.00 MHz (1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 1 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 1 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 1 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGSUR04 | −94.20 | 1 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| B CE311 | −64.20 | 1 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 1 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 1 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 1 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 1 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.0 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 1 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 1 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 1 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 1 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 1 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.2 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 1 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 1 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BAHIFRB1 | −87.20 | 1 | −76.06 | 24.16 | 1.81 | 0.80 | 142 | 1 | 61.6 |  |  |  |
| BERBERMU | −96.20 | 1 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BERBER02 | −31.00 | 1 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 2 | 10 |
| BOLAND01 | −115.20 | 1 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 1 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 1 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 1 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 1 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 1 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 1 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 1 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 1 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 1 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 1 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 1 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 1 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 1 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 1 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 1 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 1 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 64.9 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 1 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.5 |  | 10 |  |
| EQACAND1 | −115.20 | 1 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.0 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 1 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| FLKANT01 | −57.20 | 1 | −44.54 | −60.13 | 3.54 | 0.80 | 12 | 1 | 59.3 |  | 2 | 10 |
| FLKFALKS | −31.00 | 1 | −59.90 | −51.64 | 0.80 | 0.80 | 90 | 1 | 58.1 |  | 2 |  |
| GRD00002 | −42.20 | 1 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| HWA00002 | −166.20 | 1 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 1 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 1 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 1 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.2 |  | 1 | 10 |
| MEX02NTE | −136.20 | 1 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 1 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.5 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 1 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 1 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 1 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 63.9 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 1 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.5 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 1 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 1 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.4 |  | 2 7 | 10 |
| TRD00001 | −84.70 | 1 | −61.23 | 10.70 | 0.80 | 0.80 | 90 | 1 | 59.4 |  |  |  |
| URG00001 | −71.70 | 1 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 1 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 1 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 1 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.0 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 1 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 1 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 1 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 1 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 1 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 1 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.2 |  | 9/GR5 |  |
| VRG00001 | −79.70 | 1 | −64.37 | 18.48 | 0.80 | 0.80 | 90 | 1 | 58.3 |  | 4 |  |

|  |  |  |  |  |  |  |  |  |  | 12 238.58 MHz (2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 2 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 2 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 2 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.6 |  | 10 |  |
| ARGNORT5 | −54.80 | 2 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| ATNBEAM1 | −52.80 | 2 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.0 |  |  |  |
| B CE311 | −63.80 | 2 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 2 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 2 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 2 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 2 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 2 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 2 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 2 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 2 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 2 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 2 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 2 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 2 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 2 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 2 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 2 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 2 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 2 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 2 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 2 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 2 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 2 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 2 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 2 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 2 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 2 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 2 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 2 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 2 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 2 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.7 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 2 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.6 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 2 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.2 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 2 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 2 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 65.6 |  |  |  |
| EQAC0001 | −94.80 | 2 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 2 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUY00302 | −33.80 | 2 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 63.5 |  |  |  |
| HNDIFRB2 | −107.30 | 2 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 63.4 |  |  |  |
| HTI00002 | −83.30 | 2 | −73.28 | 18.96 | 0.82 | 0.80 | 11 | 2 | 60.9 |  |  |  |
| HWA00002 | −165.80 | 2 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 2 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 2 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 2 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 2 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 2 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.8 |  | 10 |  |
| PTRVIR01 | −100.80 | 2 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 2 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| TCA00001 | −115.80 | 2 | −71.79 | 21.53 | 0.80 | 0.80 | 90 | 2 | 60.4 |  |  |  |
| USAEH001 | −61.30 | 2 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 2 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 2 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 2 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 2 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 2 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 64.9 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 2 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 2 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VCT00001 | −79.30 | 2 | −61.18 | 13.23 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| VEN11VEN | −103.80 | 2 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.1 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 253.16 MHz (3) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 3 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 3 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 3 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 3 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.3 |  | 9/GR4 | 10 |
| ARGSUR04 | −94.20 | 3 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| ARGSUR05 | −55.20 | 3 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.1 |  | 9/GR4 | 10 |
| ATGSJN01 | −79.70 | 3 | −61.79 | 17.07 | 0.80 | 0.80 | 90 | 1 | 58.4 |  |  |  |
| B CE311 | −64.20 | 3 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 3 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 3 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 3 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 3 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 3 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 3 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 3 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 3 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 3 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 3 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 3 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BERBERMU | −96.20 | 3 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BOLAND01 | −115.20 | 3 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 3 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 63.8 |  | 10 |  |
| BRB00001 | −92.70 | 3 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.1 |  |  |  |
| CAN01101 | −138.20 | 3 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 3 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 3 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 3 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 3 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 3 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 3 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 3 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 3 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 3 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 3 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 3 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 3 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 3 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 3 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 3 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.0 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 3 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.6 |  | 10 |  |
| CUB00001 | −89.20 | 3 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.1 |  |  |  |
| EQACAND1 | −115.20 | 3 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.1 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 3 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| GRD00002 | −42.20 | 3 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| GRD00059 | −57.20 | 3 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.5 |  |  |  |
| GRLDNK01 | −53.20 | 3 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.0 |  | 2 | 10 |
| HWA00002 | −166.20 | 3 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 3 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 3 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 3 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.3 |  | 1 | 10 |
| MEX02NTE | −136.20 | 3 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 3 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.6 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 3 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 3 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 3 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.0 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 3 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 3 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SURINAM2 | −84.70 | 3 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.2 |  |  |  |
| URG00001 | −71.70 | 3 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 3 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 3 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 3 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 3 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 3 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 3 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 3 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 3 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 3 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.3 |  | 9/GR5 |  |

|  |  |  |  |  |  |  |  |  |  | 12 267.74 MHz (4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 4 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 4 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 4 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.7 |  | 10 |  |
| ARGNORT5 | −54.80 | 4 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| B CE311 | −63.80 | 4 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 4 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 4 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 4 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.8 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 4 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 4 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 4 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 4 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 4 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 4 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 4 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 4 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 4 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 4 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 4 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 4 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 4 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 4 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 4 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 4 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 4 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 4 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 4 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 4 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.2 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 4 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 4 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 4 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 4 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 4 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 4 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.8 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 4 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.7 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 4 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.3 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 4 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CYM00001 | −115.80 | 4 | −80.58 | 19.57 | 0.80 | 0.80 | 90 | 2 | 59.6 |  |  |  |
| DOMIFRB2 | −83.30 | 4 | −70.51 | 18.79 | 0.98 | 0.80 | 167 | 2 | 61.1 |  |  |  |
| EQAC0001 | −94.80 | 4 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 4 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUFMGG02 | −52.80 | 4 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 62.7 |  | 2 7 | 10 |
| HWA00002 | −165.80 | 4 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 4 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| JMC00005 | −33.80 | 4 | −77.27 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  |  |  |
| LCAIFRB1 | −79.30 | 4 | −61.15 | 13.90 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| MEX01NTE | −77.80 | 4 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 4 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 4 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 4 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.9 |  | 10 |  |
| PTRVIR01 | −100.80 | 4 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 4 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| SLVIFRB2 | −107.30 | 4 | −88.91 | 13.59 | 0.80 | 0.80 | 90 | 1 | 61.7 |  |  |  |
| USAEH001 | −61.30 | 4 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.9 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 4 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 4 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 4 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 4 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 4 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 4 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 4 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VEN11VEN | −103.80 | 4 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.2 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 282.32 MHz (5) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 5 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 5 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 5 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGSUR04 | −94.20 | 5 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| B CE311 | −64.20 | 5 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 5 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 5 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 5 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 5 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.0 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 5 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 5 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 5 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 5 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 5 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.2 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 5 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 5 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BAHIFRB1 | −87.20 | 5 | −76.06 | 24.16 | 1.81 | 0.80 | 142 | 1 | 61.6 |  |  |  |
| BERBERMU | −96.20 | 5 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BERBER02 | −31.00 | 5 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 2 | 10 |
| BOLAND01 | −115.20 | 5 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 5 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 5 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 5 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 5 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 5 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 5 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 5 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 5 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 5 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 5 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 5 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 5 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 5 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 5 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 5 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 5 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 64.9 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 5 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.5 |  | 10 |  |
| EQACAND1 | −115.20 | 5 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.0 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 5 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| FLKANT01 | −57.20 | 5 | −44.54 | −60.13 | 3.54 | 0.80 | 12 | 1 | 59.3 |  | 2 | 10 |
| FLKFALKS | −31.00 | 5 | −59.90 | −51.64 | 0.80 | 0.80 | 90 | 1 | 58.1 |  | 2 |  |
| GRD00002 | −42.20 | 5 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| HWA00002 | −166.20 | 5 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 5 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 5 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 5 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.2 |  | 1 | 10 |
| MEX02NTE | −136.20 | 5 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 5 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.5 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 5 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 5 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 5 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 63.9 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 5 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.5 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 5 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 5 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.4 |  | 2 7 | 10 |
| TRD00001 | −84.70 | 5 | −61.23 | 10.70 | 0.80 | 0.80 | 90 | 1 | 59.4 |  |  |  |
| URG00001 | −71.70 | 5 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 5 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 5 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 5 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.0 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 5 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 5 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 5 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 5 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 5 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 5 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.2 |  | 9/GR5 |  |
| VRG00001 | −79.70 | 5 | −64.37 | 18.48 | 0.80 | 0.80 | 90 | 1 | 58.3 |  | 4 |  |

|  |  |  |  |  |  |  |  |  |  | 12 296.90 MHz (6) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 6 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 6 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 6 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.6 |  | 10 |  |
| ARGNORT5 | −54.80 | 6 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| ATNBEAM1 | −52.80 | 6 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.0 |  |  |  |
| B CE311 | −63.80 | 6 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 6 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 6 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 6 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 6 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 6 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 6 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 6 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 6 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 6 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 6 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 6 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 6 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 6 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 6 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 6 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 6 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 6 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 6 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 6 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 6 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 6 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 6 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 6 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 6 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 6 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 6 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 6 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 6 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 6 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.7 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 6 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.6 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 6 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.2 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 6 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 6 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 65.6 |  |  |  |
| EQAC0001 | −94.80 | 6 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 6 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUY00302 | −33.80 | 6 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 63.5 |  |  |  |
| HNDIFRB2 | −107.30 | 6 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 63.4 |  |  |  |
| HTI00002 | −83.30 | 6 | −73.28 | 18.96 | 0.82 | 0.80 | 11 | 2 | 60.9 |  |  |  |
| HWA00002 | −165.80 | 6 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 6 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 6 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 6 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 6 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 6 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.8 |  | 10 |  |
| PTRVIR01 | −100.80 | 6 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 6 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| TCA00001 | −115.80 | 6 | −71.79 | 21.53 | 0.80 | 0.80 | 90 | 2 | 60.4 |  |  |  |
| USAEH001 | −61.30 | 6 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 6 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 6 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 6 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 6 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 6 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 64.9 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 6 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 6 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VCT00001 | −79.30 | 6 | −61.18 | 13.23 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| VEN11VEN | −103.80 | 6 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.1 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 311.48 MHz (7) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 7 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 7 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 7 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 7 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.3 |  | 9/GR4 | 10 |
| ARGSUR04 | −94.20 | 7 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| ARGSUR05 | −55.20 | 7 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.1 |  | 9/GR4 | 10 |
| ATGSJN01 | −79.70 | 7 | −61.79 | 17.07 | 0.80 | 0.80 | 90 | 1 | 58.4 |  |  |  |
| B CE311 | −64.20 | 7 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 7 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 7 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 7 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 7 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 7 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 7 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 7 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 7 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 7 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 7 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 7 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BERBERMU | −96.20 | 7 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BOLAND01 | −115.20 | 7 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 7 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 63.8 |  | 10 |  |
| BRB00001 | −92.70 | 7 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.1 |  |  |  |
| CAN01101 | −138.20 | 7 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 7 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 7 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 7 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 7 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 7 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 7 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 7 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 7 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 7 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 7 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 7 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 7 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 7 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 7 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 7 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.0 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 7 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.6 |  | 10 |  |
| CUB00001 | −89.20 | 7 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.1 |  |  |  |
| EQACAND1 | −115.20 | 7 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.1 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 7 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| GRD00002 | −42.20 | 7 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| GRD00059 | −57.20 | 7 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.5 |  |  |  |
| GRLDNK01 | −53.20 | 7 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.0 |  | 2 | 10 |
| HWA00002 | −166.20 | 7 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 7 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 7 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 7 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.3 |  | 1 | 10 |
| MEX02NTE | −136.20 | 7 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 7 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.6 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 7 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 7 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 7 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.0 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 7 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 7 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SURINAM2 | −84.70 | 7 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.2 |  |  |  |
| URG00001 | −71.70 | 7 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 7 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 7 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 7 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 7 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 7 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 7 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 7 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 7 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 7 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.3 |  | 9/GR5 |  |

|  |  |  |  |  |  |  |  |  |  | 12 326.06 MHz (8) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 8 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 8 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 8 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.7 |  | 10 |  |
| ARGNORT5 | −54.80 | 8 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| B CE311 | −63.80 | 8 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 8 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 8 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 8 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.8 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 8 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 8 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 8 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 8 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 8 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 8 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 8 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 8 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 8 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 8 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 8 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 8 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 8 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 8 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 8 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 8 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 8 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 8 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 8 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 8 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.2 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 8 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 8 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 8 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 8 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 8 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 8 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.8 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 8 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.7 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 8 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.3 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 8 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CYM00001 | −115.80 | 8 | −80.58 | 19.57 | 0.80 | 0.80 | 90 | 2 | 59.6 |  |  |  |
| DOMIFRB2 | −83.30 | 8 | −70.51 | 18.79 | 0.98 | 0.80 | 167 | 2 | 61.1 |  |  |  |
| EQAC0001 | −94.80 | 8 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 8 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUFMGG02 | −52.80 | 8 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 62.7 |  | 2 7 | 10 |
| HWA00002 | −165.80 | 8 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 8 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| JMC00005 | −33.80 | 8 | −77.27 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  |  |  |
| LCAIFRB1 | −79.30 | 8 | −61.15 | 13.90 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| MEX01NTE | −77.80 | 8 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 8 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 8 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 8 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.9 |  | 10 |  |
| PTRVIR01 | −100.80 | 8 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 8 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| SLVIFRB2 | −107.30 | 8 | −88.91 | 13.59 | 0.80 | 0.80 | 90 | 1 | 61.7 |  |  |  |
| USAEH001 | −61.30 | 8 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.9 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 8 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 8 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 8 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 8 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 8 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 8 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 8 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VEN11VEN | −103.80 | 8 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.2 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 340.64 MHz (9) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 9 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 9 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 9 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGSUR04 | −94.20 | 9 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| B CE311 | −64.20 | 9 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 9 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 9 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 9 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 9 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.0 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 9 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 9 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 9 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 9 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 9 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.2 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 9 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 9 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BAHIFRB1 | −87.20 | 9 | −76.06 | 24.16 | 1.81 | 0.80 | 142 | 1 | 61.6 |  |  |  |
| BERBERMU | −96.20 | 9 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BERBER02 | −31.00 | 9 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 2 | 10 |
| BOLAND01 | −115.20 | 9 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 9 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 9 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 9 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 9 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 9 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 9 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 9 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 9 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 9 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 9 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 9 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 9 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 9 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 9 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 9 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 9 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 64.9 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 9 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.5 |  | 10 |  |
| EQACAND1 | −115.20 | 9 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.0 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 9 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| FLKANT01 | −57.20 | 9 | −44.54 | −60.13 | 3.54 | 0.80 | 12 | 1 | 59.3 |  | 2 | 10 |
| FLKFALKS | −31.00 | 9 | −59.90 | −51.64 | 0.80 | 0.80 | 90 | 1 | 58.1 |  | 2 |  |
| GRD00002 | −42.20 | 9 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| HWA00002 | −166.20 | 9 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 9 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 9 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 9 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.2 |  | 1 | 10 |
| MEX02NTE | −136.20 | 9 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 9 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.5 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 9 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 9 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 9 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 63.9 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 9 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.5 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 9 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 9 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.4 |  | 2 7 | 10 |
| TRD00001 | −84.70 | 9 | −61.23 | 10.70 | 0.80 | 0.80 | 90 | 1 | 59.4 |  |  |  |
| URG00001 | −71.70 | 9 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 9 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 9 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 9 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.0 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 9 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 9 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 9 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 9 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 9 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 9 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.2 |  | 9/GR5 |  |
| VRG00001 | −79.70 | 9 | −64.37 | 18.48 | 0.80 | 0.80 | 90 | 1 | 58.3 |  | 4 |  |

|  |  |  |  |  |  |  |  |  |  | 12 355.22 MHz (10) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 10 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 10 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 10 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.6 |  | 10 |  |
| ARGNORT5 | −54.80 | 10 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| ATNBEAM1 | −52.80 | 10 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.0 |  |  |  |
| B CE311 | −63.80 | 10 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 10 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 10 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 10 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 10 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 10 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 10 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 10 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 10 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 10 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 10 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 10 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 10 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 10 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 10 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 10 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 10 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 10 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 10 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 10 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 10 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 10 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 10 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 10 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 10 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 10 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 10 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 10 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 10 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 10 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.7 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 10 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.6 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 10 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.2 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 10 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 10 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 65.6 |  |  |  |
| EQAC0001 | −94.80 | 10 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 10 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUY00302 | −33.80 | 10 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 63.5 |  |  |  |
| HNDIFRB2 | −107.30 | 10 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 63.4 |  |  |  |
| HTI00002 | −83.30 | 10 | −73.28 | 18.96 | 0.82 | 0.80 | 11 | 2 | 60.9 |  |  |  |
| HWA00002 | −165.80 | 10 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 10 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 10 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 10 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 10 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 10 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.8 |  | 10 |  |
| PTRVIR01 | −100.80 | 10 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 10 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| TCA00001 | −115.80 | 10 | −71.79 | 21.53 | 0.80 | 0.80 | 90 | 2 | 60.4 |  |  |  |
| USAEH001 | −61.30 | 10 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 10 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 10 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 10 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 10 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 10 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 64.9 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 10 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 10 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VCT00001 | −79.30 | 10 | −61.18 | 13.23 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| VEN11VEN | −103.80 | 10 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.1 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 369.80 MHz (11) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 11 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 11 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 11 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 11 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.3 |  | 9/GR4 | 10 |
| ARGSUR04 | −94.20 | 11 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| ARGSUR05 | −55.20 | 11 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.1 |  | 9/GR4 | 10 |
| ATGSJN01 | −79.70 | 11 | −61.79 | 17.07 | 0.80 | 0.80 | 90 | 1 | 58.4 |  |  |  |
| B CE311 | −64.20 | 11 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 11 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 11 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 11 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 11 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 11 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 11 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 11 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 11 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 11 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 11 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 11 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BERBERMU | −96.20 | 11 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BOLAND01 | −115.20 | 11 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 11 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 63.8 |  | 10 |  |
| BRB00001 | −92.70 | 11 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.1 |  |  |  |
| CAN01101 | −138.20 | 11 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 11 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 11 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 11 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 11 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 11 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 11 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 11 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 11 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 11 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 11 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 11 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 11 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 11 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 11 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 11 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.0 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 11 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.6 |  | 10 |  |
| CUB00001 | −89.20 | 11 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.1 |  |  |  |
| EQACAND1 | −115.20 | 11 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.1 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 11 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| GRD00002 | −42.20 | 11 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| GRD00059 | −57.20 | 11 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.5 |  |  |  |
| GRLDNK01 | −53.20 | 11 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.0 |  | 2 | 10 |
| GUY00201 | −84.70 | 11 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.5 |  |  |  |
| HWA00002 | −166.20 | 11 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 11 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 11 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 11 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.3 |  | 1 | 10 |
| MEX02NTE | −136.20 | 11 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 11 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.6 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 11 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 11 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 11 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.0 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 11 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 11 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 11 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 11 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 11 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 11 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 11 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 11 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 11 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 11 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 11 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 11 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.3 |  | 9/GR5 |  |

|  |  |  |  |  |  |  |  |  |  | 12 384.38 MHz (12) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 12 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 12 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 12 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.7 |  | 10 |  |
| ARGNORT5 | −54.80 | 12 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| B CE311 | −63.80 | 12 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 12 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 12 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 12 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.8 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 12 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 12 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 12 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 12 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 12 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 12 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 12 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 12 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 12 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 12 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 12 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 12 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 12 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 12 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 12 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 12 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 12 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 12 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 12 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 12 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.2 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 12 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 12 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 12 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 12 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 12 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 12 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.8 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 12 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.7 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 12 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.3 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 12 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CYM00001 | −115.80 | 12 | −80.58 | 19.57 | 0.80 | 0.80 | 90 | 2 | 59.6 |  |  |  |
| DOMIFRB2 | −83.30 | 12 | −70.51 | 18.79 | 0.98 | 0.80 | 167 | 2 | 61.1 |  |  |  |
| EQAC0001 | −94.80 | 12 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 12 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUFMGG02 | −52.80 | 12 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 62.7 |  | 2 7 | 10 |
| HWA00002 | −165.80 | 12 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 12 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| JMC00005 | −33.80 | 12 | −77.27 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  |  |  |
| LCAIFRB1 | −79.30 | 12 | −61.15 | 13.90 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| MEX01NTE | −77.80 | 12 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 12 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 12 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 12 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.9 |  | 10 |  |
| PTRVIR01 | −100.80 | 12 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 12 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| SLVIFRB2 | −107.30 | 12 | −88.91 | 13.59 | 0.80 | 0.80 | 90 | 1 | 61.7 |  |  |  |
| USAEH001 | −61.30 | 12 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.9 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 12 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 12 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 12 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 12 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 12 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 12 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 12 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VEN11VEN | −103.80 | 12 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.2 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 398.96 MHz (13) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 13 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 13 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 13 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGSUR04 | −94.20 | 13 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| B CE311 | −64.20 | 13 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 13 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 13 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 13 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 13 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.0 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 13 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 13 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 13 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 13 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 13 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.2 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 13 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 13 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BAHIFRB1 | −87.20 | 13 | −76.06 | 24.16 | 1.81 | 0.80 | 142 | 1 | 61.6 |  |  |  |
| BERBERMU | −96.20 | 13 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BERBER02 | −31.00 | 13 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 2 | 10 |
| BOLAND01 | −115.20 | 13 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 13 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 13 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 13 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 13 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 13 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 13 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 13 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 13 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 13 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 13 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 13 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 13 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 13 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 13 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 13 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 13 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 64.9 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 13 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.5 |  | 10 |  |
| EQACAND1 | −115.20 | 13 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.0 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 13 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| FLKANT01 | −57.20 | 13 | −44.54 | −60.13 | 3.54 | 0.80 | 12 | 1 | 59.3 |  | 2 | 10 |
| FLKFALKS | −31.00 | 13 | −59.90 | −51.64 | 0.80 | 0.80 | 90 | 1 | 58.1 |  | 2 |  |
| GRD00002 | −42.20 | 13 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| HWA00002 | −166.20 | 13 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 13 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 13 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 13 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.2 |  | 1 | 10 |
| MEX02NTE | −136.20 | 13 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 13 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.5 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 13 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 13 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 13 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 63.9 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 13 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.5 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 13 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 13 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.4 |  | 2 7 | 10 |
| TRD00001 | −84.70 | 13 | −61.23 | 10.70 | 0.80 | 0.80 | 90 | 1 | 59.4 |  |  |  |
| URG00001 | −71.70 | 13 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 13 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 13 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 13 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.0 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 13 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 13 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 13 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 13 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 13 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 13 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.2 |  | 9/GR5 |  |
| VRG00001 | −79.70 | 13 | −64.37 | 18.48 | 0.80 | 0.80 | 90 | 1 | 58.3 |  | 4 |  |

|  |  |  |  |  |  |  |  |  |  | 12 413.54 MHz (14) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 14 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.7 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 14 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 14 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.6 |  | 10 |  |
| ARGNORT5 | −54.80 | 14 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| ATNBEAM1 | −52.80 | 14 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.0 |  |  |  |
| B CE311 | −63.80 | 14 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 14 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 14 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 14 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 14 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 14 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 14 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 14 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 14 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 14 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.8 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 14 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 14 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 14 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 14 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 14 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 14 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 14 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 14 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.0 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 14 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 14 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 14 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 14 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 14 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 14 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 14 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 14 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 14 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 14 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 14 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 14 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.7 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 14 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.6 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 14 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.2 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 14 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 14 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 65.6 |  |  |  |
| EQAC0001 | −94.80 | 14 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 14 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUY00302 | −33.80 | 14 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 63.5 |  |  |  |
| HNDIFRB2 | −107.30 | 14 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 63.4 |  |  |  |
| HTI00002 | −83.30 | 14 | −73.28 | 18.96 | 0.82 | 0.80 | 11 | 2 | 60.9 |  |  |  |
| HWA00002 | −165.80 | 14 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 14 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 14 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 14 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 14 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 14 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.8 |  | 10 |  |
| PTRVIR01 | −100.80 | 14 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 14 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| TCA00001 | −115.80 | 14 | −71.79 | 21.53 | 0.80 | 0.80 | 90 | 2 | 60.4 |  |  |  |
| USAEH001 | −61.30 | 14 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 14 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 14 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 14 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 14 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.2 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 14 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 64.9 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 14 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 14 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VCT00001 | −79.30 | 14 | −61.18 | 13.23 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| VEN11VEN | −103.80 | 14 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.1 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 428.12 MHz (15) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 15 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 15 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.0 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 15 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 59.9 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 15 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.3 |  | 9/GR4 | 10 |
| ARGSUR04 | −94.20 | 15 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.7 |  | 9/GR3 | 10 |
| ARGSUR05 | −55.20 | 15 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.1 |  | 9/GR4 | 10 |
| ATGSJN01 | −79.70 | 15 | −61.79 | 17.07 | 0.80 | 0.80 | 90 | 1 | 58.4 |  |  |  |
| B CE311 | −64.20 | 15 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 15 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 15 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 15 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 62.7 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 15 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 15 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 15 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 15 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 62.8 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 15 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 15 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 15 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 15 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.3 |  | 8 9/GR9 |  |
| BERBERMU | −96.20 | 15 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 56.8 |  |  |  |
| BOLAND01 | −115.20 | 15 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 67.9 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 15 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 63.8 |  | 10 |  |
| BRB00001 | −92.70 | 15 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.1 |  |  |  |
| CAN01101 | −138.20 | 15 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 15 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 15 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.6 |  |  |  |
| CAN01203 | −129.20 | 15 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 15 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 15 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 15 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 15 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 15 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 15 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 15 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.1 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 15 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 15 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.2 |  | 10 |  |
| CHLCONT5 | −106.20 | 15 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.4 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 15 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.2 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 15 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.0 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 15 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.6 |  | 10 |  |
| CUB00001 | −89.20 | 15 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.1 |  |  |  |
| EQACAND1 | −115.20 | 15 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.1 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 15 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.3 |  | 9/GR5 |  |
| GRD00002 | −42.20 | 15 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.8 |  |  |  |
| GRD00059 | −57.20 | 15 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.5 |  |  |  |
| GRLDNK01 | −53.20 | 15 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.0 |  | 2 | 10 |
| GUY00201 | −84.70 | 15 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.5 |  |  |  |
| HWA00002 | −166.20 | 15 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 15 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.8 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 15 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.5 |  | 1 |  |
| MEX01SUR | −69.20 | 15 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.3 |  | 1 | 10 |
| MEX02NTE | −136.20 | 15 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.2 |  | 1 | 10 |
| MEX02SUR | −127.20 | 15 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.6 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 15 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.2 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 15 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.2 |  |  |  |
| PRUAND02 | −115.20 | 15 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.0 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 15 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 15 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.0 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 15 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.0 |  |  |  |
| USAEH001 | −61.70 | 15 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 61.8 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 15 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 15 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 15 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 15 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 15 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 15 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.1 |  | 10 |  |
| USAWH102 | −157.20 | 15 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.2 |  | 10 |  |
| VENAND03 | −115.20 | 15 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.3 |  | 9/GR5 |  |

|  |  |  |  |  |  |  |  |  |  | 12 442.70 MHz (16) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 16 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.8 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 16 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.0 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 16 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 65.7 |  | 10 |  |
| ARGNORT5 | −54.80 | 16 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.5 |  | 10 |  |
| B CE311 | −63.80 | 16 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.6 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 16 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.0 |  | 8 9/GR9 | 10 |
| B CE411 | −63.80 | 16 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.6 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 16 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 62.8 |  | 8 9/GR9 | 10 |
| B CE511 | −63.80 | 16 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.1 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 16 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 62.9 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 16 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 62.8 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 16 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 62.8 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 16 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.3 |  | 8 | 10 |
| B SU111 | −80.80 | 16 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 62.9 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 16 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.3 |  | 8 9/GR9 |  |
| B SU211 | −80.80 | 16 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.5 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 16 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.3 |  | 8 9/GR9 |  |
| CAN01101 | −137.80 | 16 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.5 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 16 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.6 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 16 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.6 |  |  |  |
| CAN01203 | −128.80 | 16 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.5 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 16 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.1 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 16 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 59.8 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 16 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 61.8 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 16 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 16 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 16 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.2 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 16 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.2 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 16 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.3 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 16 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.2 |  | 10 |  |
| CHLCONT4 | −105.80 | 16 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.1 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 16 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.6 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 16 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.7 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 16 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.8 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 16 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.7 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 16 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.3 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 16 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.1 |  | 9/GR18 |  |
| CYM00001 | −115.80 | 16 | −80.58 | 19.57 | 0.80 | 0.80 | 90 | 2 | 59.6 |  |  |  |
| DOMIFRB2 | −83.30 | 16 | −70.51 | 18.79 | 0.98 | 0.80 | 167 | 2 | 61.1 |  |  |  |
| EQAC0001 | −94.80 | 16 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.0 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 16 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.0 |  | 9/GR19 |  |
| GUFMGG02 | −52.80 | 16 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 62.7 |  | 2 7 | 10 |
| HWA00002 | −165.80 | 16 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 58.8 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 16 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 58.8 |  | 9/GR2 | 10 |
| JMC00005 | −33.80 | 16 | −77.27 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  |  |  |
| LCAIFRB1 | −79.30 | 16 | −61.15 | 13.90 | 0.80 | 0.80 | 90 | 2 | 58.4 |  |  |  |
| MEX01NTE | −77.80 | 16 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.5 |  | 1 |  |
| MEX02NTE | −135.80 | 16 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.2 |  | 1 | 10 |
| MEX02SUR | −126.80 | 16 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.5 |  | 1 | 10 |
| PRU00004 | −85.80 | 16 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 62.9 |  | 10 |  |
| PTRVIR01 | −100.80 | 16 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.6 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 16 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.1 |  | 1 6 9/GR21 |  |
| SLVIFRB2 | −107.30 | 16 | −88.91 | 13.59 | 0.80 | 0.80 | 90 | 1 | 61.7 |  |  |  |
| USAEH001 | −61.30 | 16 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 61.9 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 16 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 61.7 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 16 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.1 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 16 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.6 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 16 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.3 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 16 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.0 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 16 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.1 |  | 10 |  |
| USAWH102 | −156.80 | 16 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.2 |  | 10 |  |
| VEN11VEN | −103.80 | 16 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.2 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 457.28 MHz (17) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |  |
| ALS00002 | −166.20 | 17 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.9 |  | 9/GR1 |  | 10 |
| ALS00003 | −175.20 | 17 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 |  | 10 |
| ARGINSU4 | −94.20 | 17 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |  |
| ARGINSU5 | −55.20 | 17 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  | 10 |
| ARGSUR04 | −94.20 | 17 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  | 10 |
| ARGSUR05 | −55.20 | 17 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.2 |  | 9/GR4 |  | 10 |
| B CE311 | −64.20 | 17 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  | 10 |
| B CE312 | −45.20 | 17 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.2 |  | 8 9/GR9 |  | 10 |
| B CE411 | −64.20 | 17 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  | 10 |
| B CE412 | −45.20 | 17 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.0 |  | 8 9/GR9 |  | 10 |
| B CE511 | −64.20 | 17 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  | 10 |
| B NO611 | −74.20 | 17 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.1 |  | 8 9/GR8 |  | 10 |
| B NO711 | −74.20 | 17 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.1 |  | 8 9/GR8 |  | 10 |
| B NO811 | −74.20 | 17 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |  |
| B SU111 | −81.20 | 17 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  | 10 |
| B SU112 | −45.20 | 17 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.5 |  | 8 9/GR9 |  |  |
| B SU211 | −81.20 | 17 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  | 10 |
| B SU212 | −45.20 | 17 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 |  |  |
| BERBERMU | −96.20 | 17 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |  |
| BERBER02 | −31.00 | 17 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 1 | 57.1 |  | 2 |  | 10 |
| BOLAND01 | −115.20 | 17 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.0 |  | 9/GR5 |  |  |
| CAN01101 | −138.20 | 17 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 |  | 10 |
| CAN01201 | −138.20 | 17 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 |  | 10 |
| CAN01202 | −72.70 | 17 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |  |
| CAN01203 | −129.20 | 17 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 |  | 10 |
| CAN01303 | −129.20 | 17 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.2 |  | 9/GR12 |  | 10 |
| CAN01304 | −91.20 | 17 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.0 |  | 9/GR13 |  |  |
| CAN01403 | −129.20 | 17 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 |  | 10 |
| CAN01404 | −91.20 | 17 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  | 10 |
| CAN01405 | −82.20 | 17 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  | 10 |
| CAN01504 | −91.20 | 17 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  | 10 |
| CAN01505 | −82.20 | 17 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.3 |  | 9/GR14 |  | 10 |
| CAN01605 | −82.20 | 17 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  | 10 |
| CAN01606 | −70.70 | 17 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.4 |  | 10 |  |  |
| CHLCONT5 | −106.20 | 17 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |  |
| CHLPAC02 | −106.20 | 17 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |  |
| CLMAND01 | −115.20 | 17 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.3 |  | 9/GR5 |  |  |
| CLM00001 | −103.20 | 17 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |  |
| EQACAND1 | −115.20 | 17 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |  |
| EQAGAND1 | −115.20 | 17 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.5 |  | 9/GR5 |  |  |
| FLKFALKS | −31.00 | 17 | −59.90 | −51.64 | 0.80 | 0.80 | 90 | 1 | 58.2 |  | 2 |  |  |
| HWA00002 | −166.20 | 17 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 |  | 10 |
| HWA00003 | −175.20 | 17 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.9 |  | 9/GR2 |  | 10 |
| JMC00002 | −92.70 | 17 | −77.30 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.1 |  |  |  |  |
| KNA00001 | −79.70 | 17 | −62.46 | 17.44 | 0.80 | 0.80 | 90 | 1 | 58.6 |  |  |  |  |
| MEX01NTE | −78.20 | 17 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.7 |  | 1 |  |  |
| MEX01SUR | −69.20 | 17 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  | 10 |
| MEX02NTE | −136.20 | 17 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.4 |  | 1 |  | 10 |
| MEX02SUR | −127.20 | 17 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 |  | 10 |
| PAQPAC01 | −106.20 | 17 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |  |
| PRG00002 | −99.20 | 17 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.4 |  |  |  |  |
| PRUAND02 | −115.20 | 17 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |  |
| PTRVIR01 | −101.20 | 17 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |  |
| PTRVIR02 | −110.20 | 17 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |  |
| SPMFRAN3 | −53.20 | 17 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.6 |  | 2 7 |  | 10 |
| SURINAM2 | −84.70 | 17 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.5 |  |  |  |  |
| URG00001 | −71.70 | 17 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |  |
| USAEH001 | −61.70 | 17 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  | 10 |
| USAEH002 | −101.20 | 17 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 |  | 10 |
| USAEH003 | −110.20 | 17 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.3 |  | 1 6 9/GR21 |  | 10 |
| USAEH004 | −119.20 | 17 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 |  | 10 |
| USAPSA02 | −166.20 | 17 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.5 |  | 9/GR1 |  |  |
| USAPSA03 | −175.20 | 17 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.3 |  | 9/GR2 |  |  |
| USAWH101 | −148.20 | 17 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.3 |  | 10 |  |  |
| USAWH102 | −157.20 | 17 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |  |
| VENAND03 | −115.20 | 17 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.6 |  | 9/GR5 |  |  |

|  |  |  |  |  |  |  |  |  |  | 12 471.86 MHz (18) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |  |
| ALS00002 | −165.80 | 18 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 |  | 10 |
| ALS00003 | −174.80 | 18 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 |  | 10 |
| ARGNORT4 | −93.80 | 18 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.0 |  | 10 |  |  |
| ARGNORT5 | −54.80 | 18 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.8 |  | 10 |  |  |
| ATNBEAM1 | −52.80 | 18 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.3 |  |  |  |  |
| B CE311 | −63.80 | 18 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  | 10 |
| B CE312 | −44.80 | 18 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.2 |  | 8 9/GR9 |  | 10 |
| B CE411 | −63.80 | 18 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  | 10 |
| B CE412 | −44.80 | 18 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.0 |  | 8 9/GR9 |  | 10 |
| B CE511 | −63.80 | 18 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  | 10 |
| B NO611 | −73.80 | 18 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.1 |  | 8 9/GR8 |  | 10 |
| B NO711 | −73.80 | 18 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.1 |  | 8 9/GR8 |  | 10 |
| B NO811 | −73.80 | 18 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.1 |  | 8 9/GR8 |  |  |
| B SE911 | −101.80 | 18 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  | 10 |
| B SU111 | −80.80 | 18 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.1 |  | 8 9/GR6 |  | 10 |
| B SU112 | −44.80 | 18 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 |  |  |
| B SU211 | −80.80 | 18 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  | 10 |
| B SU212 | −44.80 | 18 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 |  |  |
| BLZ00001 | −115.80 | 18 | −88.68 | 17.27 | 0.80 | 0.80 | 90 | 2 | 59.2 |  |  |  |  |
| CAN01101 | −137.80 | 18 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 |  | 10 |
| CAN01201 | −137.80 | 18 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 |  | 10 |
| CAN01202 | −72.30 | 18 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |  |
| CAN01203 | −128.80 | 18 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 |  | 10 |
| CAN01303 | −128.80 | 18 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 |  | 10 |
| CAN01304 | −90.80 | 18 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |  |
| CAN01403 | −128.80 | 18 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 |  | 10 |
| CAN01404 | −90.80 | 18 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  | 10 |
| CAN01405 | −81.80 | 18 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  | 10 |
| CAN01504 | −90.80 | 18 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  | 10 |
| CAN01505 | −81.80 | 18 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.3 |  | 9/GR14 |  | 10 |
| CAN01605 | −81.80 | 18 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  | 10 |
| CAN01606 | −70.30 | 18 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.4 |  | 10 |  |  |
| CHLCONT4 | −105.80 | 18 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |  |
| CHLCONT6 | −105.80 | 18 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.7 |  | 9/GR16 |  |  |
| CRBBAH01 | −92.30 | 18 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.9 |  | 9/GR18 |  |  |
| CRBBER01 | −92.30 | 18 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 9/GR18 |  |  |
| CRBBLZ01 | −92.30 | 18 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |  |
| CRBEC001 | −92.30 | 18 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  | 10 |
| CRBJMC01 | −92.30 | 18 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.3 |  | 9/GR18 |  |  |
| CTR00201 | −130.80 | 18 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 66.0 |  |  |  |  |
| DMAIFRB1 | −79.30 | 18 | −61.30 | 15.35 | 0.80 | 0.80 | 90 | 2 | 58.7 |  |  |  |  |
| EQAC0001 | −94.80 | 18 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |  |
| EQAG0001 | −94.80 | 18 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.2 |  | 9/GR19 |  |  |
| HWA00002 | −165.80 | 18 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 |  | 10 |
| HWA00003 | −174.80 | 18 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 |  | 10 |
| MEX01NTE | −77.80 | 18 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.7 |  | 1 |  |  |
| MEX02NTE | −135.80 | 18 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.4 |  | 1 |  | 10 |
| MEX02SUR | −126.80 | 18 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 |  | 10 |
| NCG00003 | −107.30 | 18 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 63.6 |  |  |  |  |
| PRU00004 | −85.80 | 18 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.1 |  | 10 |  |  |
| PTRVIR01 | −100.80 | 18 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.8 |  | 1 6 9/GR20 |  |  |
| PTRVIR02 | −109.80 | 18 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |  |
| USAEH001 | −61.30 | 18 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  | 10 |
| USAEH002 | −100.80 | 18 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 |  | 10 |
| USAEH003 | −109.80 | 18 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.3 |  | 1 6 9/GR21 |  | 10 |
| USAEH004 | −118.80 | 18 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 |  | 10 |
| USAPSA02 | −165.80 | 18 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.5 |  | 9/GR1 |  |  |
| USAPSA03 | −174.80 | 18 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |  |
| USAWH101 | −147.80 | 18 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.3 |  | 10 |  |  |
| USAWH102 | −156.80 | 18 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |  |
| VEN11VEN | −103.80 | 18 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.5 |  | 10 |  |  |

|  |  |  |  |  |  |  |  |  |  | 12 486.44 MHz (19) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 19 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 60.0 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 19 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 19 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 19 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 | 10 |
| ARGSUR04 | −94.20 | 19 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 | 10 |
| ARGSUR05 | −55.20 | 19 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.3 |  | 9/GR4 | 10 |
| B CE311 | −64.20 | 19 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 | 10 |
| B CE312 | −45.20 | 19 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.3 |  | 8 9/GR9 | 10 |
| B CE411 | −64.20 | 19 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 | 10 |
| B CE412 | −45.20 | 19 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.1 |  | 8 9/GR9 | 10 |
| B CE511 | −64.20 | 19 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 | 10 |
| B NO611 | −74.20 | 19 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.2 |  | 8 9/GR8 | 10 |
| B NO711 | −74.20 | 19 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.2 |  | 8 9/GR8 | 10 |
| B NO811 | −74.20 | 19 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 19 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 | 10 |
| B SU112 | −45.20 | 19 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.6 |  | 8 9/GR9 |  |
| B SU211 | −81.20 | 19 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 | 10 |
| B SU212 | −45.20 | 19 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 |  |
| BERBERMU | −96.20 | 19 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 19 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.1 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 19 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 64.2 |  | 10 |  |
| BRB00001 | −92.70 | 19 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.4 |  |  |  |
| CAN01101 | −138.20 | 19 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 19 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 19 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 19 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 19 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 19 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.1 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 19 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 19 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 | 10 |
| CAN01405 | −82.20 | 19 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 | 10 |
| CAN01504 | −91.20 | 19 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 | 10 |
| CAN01505 | −82.20 | 19 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.4 |  | 9/GR14 | 10 |
| CAN01605 | −82.20 | 19 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 | 10 |
| CAN01606 | −70.70 | 19 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.5 |  | 10 |  |
| CHLCONT5 | −106.20 | 19 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 19 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 19 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.4 |  | 9/GR5 |  |
| CLM00001 | −103.20 | 19 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| CUB00001 | −89.20 | 19 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.3 |  |  |  |
| EQACAND1 | −115.20 | 19 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 19 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.6 |  | 9/GR5 |  |
| GRD00059 | −57.20 | 19 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.7 |  |  |  |
| GRLDNK01 | −53.20 | 19 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.2 |  | 2 | 10 |
| GUY00201 | −84.70 | 19 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.8 |  |  |  |
| HWA00002 | −166.20 | 19 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 19 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 19 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.8 |  | 1 |  |
| MEX01SUR | −69.20 | 19 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 | 10 |
| MEX02NTE | −136.20 | 19 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.5 |  | 1 | 10 |
| MEX02SUR | −127.20 | 19 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| MSR00001 | −79.70 | 19 | −61.73 | 16.75 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 4 |  |
| PAQPAC01 | −106.20 | 19 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 19 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.5 |  |  |  |
| PRUAND02 | −115.20 | 19 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 19 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 19 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 19 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 19 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 | 10 |
| USAEH002 | −101.20 | 19 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 19 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 19 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 19 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 19 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.4 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 19 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.4 |  | 10 |  |
| USAWH102 | −157.20 | 19 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 19 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.7 |  | 9/GR5 |  |

|  |  |  |  |  |  |  |  |  |  | 12 501.02 MHz (20) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 20 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 20 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 20 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.1 |  | 10 |  |
| ARGNORT5 | −54.80 | 20 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.9 |  | 10 |  |
| B CE311 | −63.80 | 20 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 | 10 |
| B CE312 | −44.80 | 20 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 20 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 | 10 |
| B CE412 | −44.80 | 20 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 20 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 | 10 |
| B NO611 | −73.80 | 20 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.2 |  | 8 9/GR8 | 10 |
| B NO711 | −73.80 | 20 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.2 |  | 8 9/GR8 | 10 |
| B NO811 | −73.80 | 20 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.2 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 20 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 | 10 |
| B SU111 | −80.80 | 20 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.2 |  | 8 9/GR6 | 10 |
| B SU112 | −44.80 | 20 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 20 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 | 10 |
| B SU212 | −44.80 | 20 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| CAN01101 | −137.80 | 20 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 20 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 20 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 20 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 20 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 20 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 20 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 20 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 | 10 |
| CAN01405 | −81.80 | 20 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 | 10 |
| CAN01504 | −90.80 | 20 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 | 10 |
| CAN01505 | −81.80 | 20 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.4 |  | 9/GR14 | 10 |
| CAN01605 | −81.80 | 20 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 | 10 |
| CAN01606 | −70.30 | 20 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.5 |  | 10 |  |
| CHLCONT4 | −105.80 | 20 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 20 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.8 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 20 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 62.0 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 20 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 57.0 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 20 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 20 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 | 10 |
| CRBJMC01 | −92.30 | 20 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.4 |  | 9/GR18 |  |
| EQAC0001 | −94.80 | 20 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 20 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.3 |  | 9/GR19 |  |
| GRD00003 | −79.30 | 20 | −61.62 | 12.34 | 0.80 | 0.80 | 90 | 2 | 58.9 |  |  |  |
| GTMIFRB2 | −107.30 | 20 | −90.50 | 15.64 | 1.03 | 0.80 | 84 | 1 | 61.4 |  |  |  |
| GUFMGG02 | −52.80 | 20 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 63.0 |  | 2 7 | 10 |
| HWA00002 | −165.80 | 20 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 20 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 20 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.8 |  | 1 |  |
| MEX02NTE | −135.80 | 20 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.5 |  | 1 | 10 |
| MEX02SUR | −126.80 | 20 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| PNRIFRB2 | −121.00 | 20 | −80.15 | 8.46 | 1.01 | 0.80 | 170 | 1 | 65.1 |  |  |  |
| PRU00004 | −85.80 | 20 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.2 |  | 10 |  |
| PTRVIR01 | −100.80 | 20 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.9 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 20 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 20 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 | 10 |
| USAEH002 | −100.80 | 20 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 20 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 20 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 20 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 20 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 20 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.4 |  | 10 |  |
| USAWH102 | −156.80 | 20 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN02VEN | −103.80 | 20 | −63.50 | 15.50 | 0.80 | 0.80 | 90 | 2 | 60.1 |  | 9/GR22 |  |
| VEN11VEN | −103.80 | 20 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.6 |  | 9/GR22 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 515.60 MHz (21) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 21 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 21 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 21 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 21 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 21 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 21 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.2 |  | 9/GR4 |  |
| B CE311 | −64.20 | 21 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 21 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.2 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 21 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 21 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.0 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 21 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 21 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 21 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 21 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 21 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 21 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.5 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 21 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 21 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 21 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 21 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.0 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 21 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 21 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 21 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 21 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 21 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.2 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 21 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 21 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 21 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 21 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 21 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 21 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.3 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 21 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 21 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.4 |  |  |  |
| CHLCONT5 | −106.20 | 21 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 21 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 21 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.3 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 21 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| EQACAND1 | −115.20 | 21 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 21 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.5 |  | 9/GR5 |  |
| HWA00002 | −166.20 | 21 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 21 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.9 |  | 9/GR2 | 10 |
| JMC00002 | −92.70 | 21 | −77.30 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.1 |  |  |  |
| KNA00001 | −79.70 | 21 | −62.46 | 17.44 | 0.80 | 0.80 | 90 | 1 | 58.6 |  |  |  |
| MEX01NTE | −78.20 | 21 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.7 |  | 1 |  |
| MEX01SUR | −69.20 | 21 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 21 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.4 |  | 1 | 10 |
| MEX02SUR | −127.20 | 21 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 21 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 21 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.4 |  |  |  |
| PRUAND02 | −115.20 | 21 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 21 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 21 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 21 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.6 |  | 2 7 |  |
| SURINAM2 | −84.70 | 21 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.5 |  |  |  |
| URG00001 | −71.70 | 21 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 21 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 21 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 21 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.3 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 21 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 21 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.5 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 21 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 21 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.3 |  | 10 |  |
| USAWH102 | −157.20 | 21 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 21 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.6 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 530.18 MHz (22) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 22 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 22 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 22 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.0 |  |  |  |
| ARGNORT5 | −54.80 | 22 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.8 |  |  |  |
| ATNBEAM1 | −52.80 | 22 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.3 |  |  |  |
| B CE311 | −63.80 | 22 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −44.80 | 22 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.2 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 22 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −44.80 | 22 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.0 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 22 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −73.80 | 22 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.1 |  | 8 9/GR8 |  |
| B NO711 | −73.80 | 22 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.1 |  | 8 9/GR8 |  |
| B NO811 | −73.80 | 22 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.1 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 22 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |
| B SU111 | −80.80 | 22 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.1 |  | 8 9/GR6 |  |
| B SU112 | −44.80 | 22 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 22 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −44.80 | 22 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| BLZ00001 | −115.80 | 22 | −88.68 | 17.27 | 0.80 | 0.80 | 90 | 2 | 59.2 |  |  |  |
| CAN01101 | −137.80 | 22 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 22 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 22 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 22 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 22 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 22 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 22 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 22 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −81.80 | 22 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −90.80 | 22 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −81.80 | 22 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.3 |  | 9/GR14 |  |
| CAN01605 | −81.80 | 22 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.30 | 22 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.4 |  |  |  |
| CHLCONT4 | −105.80 | 22 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 22 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.7 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 22 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.9 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 22 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 22 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 22 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |
| CRBJMC01 | −92.30 | 22 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.3 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 22 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 66.0 |  |  |  |
| DMAIFRB1 | −79.30 | 22 | −61.30 | 15.35 | 0.80 | 0.80 | 90 | 2 | 58.7 |  |  |  |
| EQAC0001 | −94.80 | 22 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 22 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.2 |  | 9/GR19 |  |
| HWA00002 | −165.80 | 22 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 22 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 22 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.7 |  | 1 |  |
| MEX02NTE | −135.80 | 22 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.4 |  | 1 | 10 |
| MEX02SUR | −126.80 | 22 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| NCG00003 | −107.30 | 22 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 63.6 |  |  |  |
| PRU00004 | −85.80 | 22 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.1 |  |  |  |
| PTRVIR01 | −100.80 | 22 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 22 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 22 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −100.80 | 22 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 22 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.3 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 22 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 22 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.5 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 22 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 22 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.3 |  | 10 |  |
| USAWH102 | −156.80 | 22 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN11VEN | −103.80 | 22 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.5 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 544.76 MHz (23) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 23 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 60.0 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 23 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 23 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 23 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 23 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 23 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.3 |  | 9/GR4 |  |
| B CE311 | −64.20 | 23 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 23 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 23 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 23 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 23 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 23 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 23 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 23 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 23 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 23 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 23 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 23 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 23 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 23 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.1 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 23 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 64.2 |  |  |  |
| BRB00001 | −92.70 | 23 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.4 |  |  |  |
| CAN01101 | −138.20 | 23 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 23 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 23 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 23 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 23 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 23 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.1 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 23 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 23 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 23 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 23 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 23 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 23 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 23 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.5 |  |  |  |
| CHLCONT5 | −106.20 | 23 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 23 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 23 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.4 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 23 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| CUB00001 | −89.20 | 23 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.3 |  |  |  |
| EQACAND1 | −115.20 | 23 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 23 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.6 |  | 9/GR5 |  |
| GRD00059 | −57.20 | 23 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.7 |  |  |  |
| GRLDNK01 | −53.20 | 23 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.2 |  | 2 |  |
| GUY00201 | −84.70 | 23 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.8 |  |  |  |
| HWA00002 | −166.20 | 23 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 23 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 23 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.8 |  | 1 |  |
| MEX01SUR | −69.20 | 23 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 23 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.5 |  | 1 | 10 |
| MEX02SUR | −127.20 | 23 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| MSR00001 | −79.70 | 23 | −61.73 | 16.75 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 4 |  |
| PAQPAC01 | −106.20 | 23 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 23 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.5 |  |  |  |
| PRUAND02 | −115.20 | 23 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 23 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 23 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 23 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 23 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 23 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 23 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 23 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 23 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 23 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.4 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 23 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.4 |  | 10 |  |
| USAWH102 | −157.20 | 23 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 23 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.7 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 559.34 MHz (24) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 24 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 24 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 24 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.1 |  |  |  |
| ARGNORT5 | −54.80 | 24 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.9 |  |  |  |
| B CE311 | −63.80 | 24 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −44.80 | 24 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 24 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −44.80 | 24 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 24 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −73.80 | 24 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −73.80 | 24 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −73.80 | 24 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.2 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 24 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |
| B SU111 | −80.80 | 24 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −44.80 | 24 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 24 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −44.80 | 24 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| CAN01101 | −137.80 | 24 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 24 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 24 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 24 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 24 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 24 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 24 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 24 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −81.80 | 24 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −90.80 | 24 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −81.80 | 24 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −81.80 | 24 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.30 | 24 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.5 |  |  |  |
| CHLCONT4 | −105.80 | 24 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 24 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.8 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 24 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 62.0 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 24 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 57.0 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 24 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 24 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |
| CRBJMC01 | −92.30 | 24 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.4 |  | 9/GR18 |  |
| EQAC0001 | −94.80 | 24 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 24 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.3 |  | 9/GR19 |  |
| GRD00003 | −79.30 | 24 | −61.62 | 12.34 | 0.80 | 0.80 | 90 | 2 | 58.9 |  |  |  |
| GTMIFRB2 | −107.30 | 24 | −90.50 | 15.64 | 1.03 | 0.80 | 84 | 1 | 61.4 |  |  |  |
| GUFMGG02 | −52.80 | 24 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 63.0 |  | 2 7 |  |
| HWA00002 | −165.80 | 24 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 24 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 24 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.8 |  | 1 |  |
| MEX02NTE | −135.80 | 24 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.5 |  | 1 | 10 |
| MEX02SUR | −126.80 | 24 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| PNRIFRB2 | −121.00 | 24 | −80.15 | 8.46 | 1.01 | 0.80 | 170 | 1 | 65.1 |  |  |  |
| PRU00004 | −85.80 | 24 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.2 |  |  |  |
| PTRVIR01 | −100.80 | 24 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.9 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 24 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 24 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −100.80 | 24 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 24 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 24 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 24 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 24 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 24 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.4 |  | 10 |  |
| USAWH102 | −156.80 | 24 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN02VEN | −103.80 | 24 | −63.50 | 15.50 | 0.80 | 0.80 | 90 | 2 | 60.1 |  | 9/GR22 |  |
| VEN11VEN | −103.80 | 24 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.6 |  | 9/GR22 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 573.92 MHz (25) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 25 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 25 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 25 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 25 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 25 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 25 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.2 |  | 9/GR4 |  |
| B CE311 | −64.20 | 25 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 25 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.2 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 25 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 25 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.0 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 25 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 25 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 25 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 25 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 25 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 25 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.5 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 25 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 25 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 25 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 25 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.0 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 25 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 25 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 25 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 25 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 25 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.2 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 25 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 25 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 25 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 25 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 25 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 25 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.3 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 25 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 25 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.4 |  |  |  |
| CHLCONT5 | −106.20 | 25 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 25 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 25 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.3 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 25 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| EQACAND1 | −115.20 | 25 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 25 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.5 |  | 9/GR5 |  |
| HWA00002 | −166.20 | 25 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 25 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.9 |  | 9/GR2 | 10 |
| JMC00002 | −92.70 | 25 | −77.30 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.1 |  |  |  |
| KNA00001 | −79.70 | 25 | −62.46 | 17.44 | 0.80 | 0.80 | 90 | 1 | 58.6 |  |  |  |
| MEX01NTE | −78.20 | 25 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.7 |  | 1 |  |
| MEX01SUR | −69.20 | 25 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 25 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.4 |  | 1 | 10 |
| MEX02SUR | −127.20 | 25 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 25 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 25 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.4 |  |  |  |
| PRUAND02 | −115.20 | 25 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 25 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 25 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 25 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.6 |  | 2 7 |  |
| SURINAM2 | −84.70 | 25 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.5 |  |  |  |
| URG00001 | −71.70 | 25 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 25 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 25 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 25 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.3 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 25 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 25 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.5 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 25 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 25 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.3 |  | 10 |  |
| USAWH102 | −157.20 | 25 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 25 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.6 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 588.50 MHz (26) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 26 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |  |
| ALS00003 | −174.80 | 26 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |  |
| ARGNORT4 | −93.80 | 26 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.0 |  |  |  |  |
| ARGNORT5 | −54.80 | 26 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.8 |  |  |  |  |
| ATNBEAM1 | −52.80 | 26 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.3 |  |  |  |  |
| B CE311 | −63.80 | 26 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |  |
| B CE312 | −44.80 | 26 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.2 |  | 8 9/GR9 | 10 11 |  |
| B CE411 | −63.80 | 26 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |  |
| B CE412 | −44.80 | 26 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.0 |  | 8 9/GR9 | 10 12 |  |
| B CE511 | −63.80 | 26 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |  |
| B NO611 | −73.80 | 26 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.1 |  | 8 9/GR8 |  |  |
| B NO711 | −73.80 | 26 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.1 |  | 8 9/GR8 |  |  |
| B NO811 | −73.80 | 26 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.1 |  | 8 9/GR8 |  |  |
| B SE911 | −101.80 | 26 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |  |
| B SU111 | −80.80 | 26 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.1 |  | 8 9/GR6 |  |  |
| B SU112 | −44.80 | 26 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |  |
| B SU211 | −80.80 | 26 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |  |
| B SU212 | −44.80 | 26 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |  |
| BLZ00001 | −115.80 | 26 | −88.68 | 17.27 | 0.80 | 0.80 | 90 | 2 | 59.2 |  |  |  |  |
| CAN01101 | −137.80 | 26 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |  |
| CAN01201 | −137.80 | 26 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |  |
| CAN01202 | −72.30 | 26 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |  |
| CAN01203 | −128.80 | 26 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |  |
| CAN01303 | −128.80 | 26 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |  |
| CAN01304 | −90.80 | 26 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |  |
| CAN01403 | −128.80 | 26 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |  |
| CAN01404 | −90.80 | 26 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |  |
| CAN01405 | −81.80 | 26 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |  |
| CAN01504 | −90.80 | 26 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |  |
| CAN01505 | −81.80 | 26 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.3 |  | 9/GR14 |  |  |
| CAN01605 | −81.80 | 26 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |  |
| CAN01606 | −70.30 | 26 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.4 |  |  |  |  |
| CHLCONT4 | −105.80 | 26 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |  |
| CHLCONT6 | −105.80 | 26 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.7 |  | 9/GR16 |  |  |
| CRBBAH01 | −92.30 | 26 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.9 |  | 9/GR18 |  |  |
| CRBBER01 | −92.30 | 26 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 9/GR18 |  |  |
| CRBBLZ01 | −92.30 | 26 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |  |
| CRBEC001 | −92.30 | 26 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |  |
| CRBJMC01 | −92.30 | 26 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.3 |  | 9/GR18 |  |  |
| CTR00201 | −130.80 | 26 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 66.0 |  |  |  |  |
| DMAIFRB1 | −79.30 | 26 | −61.30 | 15.35 | 0.80 | 0.80 | 90 | 2 | 58.7 |  |  |  |  |
| EQAC0001 | −94.80 | 26 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |  |
| EQAG0001 | −94.80 | 26 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.2 |  | 9/GR19 |  |  |
| HWA00002 | −165.80 | 26 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |  |
| HWA00003 | −174.80 | 26 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |  |
| MEX01NTE | −77.80 | 26 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.7 |  | 1 |  |  |
| MEX02NTE | −135.80 | 26 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.4 |  | 1 | 10 |  |
| MEX02SUR | −126.80 | 26 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |  |
| NCG00003 | −107.30 | 26 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 63.6 |  |  |  |  |
| PRU00004 | −85.80 | 26 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.1 |  |  |  |  |
| PTRVIR01 | −100.80 | 26 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.8 |  | 1 6 9/GR20 |  |  |
| PTRVIR02 | −109.80 | 26 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |  |
| USAEH001 | −61.30 | 26 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |  |
| USAEH002 | −100.80 | 26 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |  |
| USAEH003 | −109.80 | 26 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.3 |  | 1 6 9/GR21 | 10 |  |
| USAEH004 | −118.80 | 26 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |  |
| USAPSA02 | −165.80 | 26 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.5 |  | 9/GR1 |  |  |
| USAPSA03 | −174.80 | 26 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |  |
| USAWH101 | −147.80 | 26 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.3 |  | 10 |  |  |
| USAWH102 | −156.80 | 26 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |  |
| VEN11VEN | −103.80 | 26 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.5 |  | 10 |  |  |

|  |  |  |  |  |  |  |  |  |  | 12 603.08 MHz (27) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 27 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 60.0 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 27 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 27 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 27 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 27 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 27 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.3 |  | 9/GR4 |  |
| B CE311 | −64.20 | 27 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 27 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 27 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 27 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 27 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 27 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 27 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 27 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 27 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 27 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 27 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 27 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 27 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 27 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.1 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 27 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 64.2 |  |  |  |
| BRB00001 | −92.70 | 27 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.4 |  |  |  |
| CAN01101 | −138.20 | 27 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 27 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 27 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 27 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 27 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 27 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.1 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 27 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 27 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 27 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 27 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 27 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 27 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 27 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.5 |  |  |  |
| CHLCONT5 | −106.20 | 27 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 27 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 27 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.4 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 27 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| CUB00001 | −89.20 | 27 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.3 |  |  |  |
| EQACAND1 | −115.20 | 27 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 27 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.6 |  | 9/GR5 |  |
| GRD00059 | −57.20 | 27 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.7 |  |  |  |
| GRLDNK01 | −53.20 | 27 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.2 |  | 2 |  |
| GUY00201 | −84.70 | 27 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.8 |  |  |  |
| HWA00002 | −166.20 | 27 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 27 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 27 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.8 |  | 1 |  |
| MEX01SUR | −69.20 | 27 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 27 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.5 |  | 1 | 10 |
| MEX02SUR | −127.20 | 27 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| MSR00001 | −79.70 | 27 | −61.73 | 16.75 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 4 |  |
| PAQPAC01 | −106.20 | 27 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 27 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.5 |  |  |  |
| PRUAND02 | −115.20 | 27 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 27 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 27 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 27 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 27 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 27 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 27 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 27 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 27 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 27 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.4 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 27 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.4 |  | 10 |  |
| USAWH102 | −157.20 | 27 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 27 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.7 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 617.66 MHz (28) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 28 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 28 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 28 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.1 |  |  |  |
| ARGNORT5 | −54.80 | 28 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.9 |  |  |  |
| B CE311 | −63.80 | 28 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −44.80 | 28 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 28 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −44.80 | 28 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 28 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −73.80 | 28 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −73.80 | 28 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −73.80 | 28 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.2 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 28 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |
| B SU111 | −80.80 | 28 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −44.80 | 28 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 28 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −44.80 | 28 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| CAN01101 | −137.80 | 28 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 28 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 28 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 28 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 28 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 28 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 28 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 28 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −81.80 | 28 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −90.80 | 28 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −81.80 | 28 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −81.80 | 28 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.30 | 28 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.5 |  |  |  |
| CHLCONT4 | −105.80 | 28 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 28 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.8 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 28 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 62.0 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 28 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 57.0 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 28 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 28 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |
| CRBJMC01 | −92.30 | 28 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.4 |  | 9/GR18 |  |
| EQAC0001 | −94.80 | 28 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 28 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.3 |  | 9/GR19 |  |
| GRD00003 | −79.30 | 28 | −61.62 | 12.34 | 0.80 | 0.80 | 90 | 2 | 58.9 |  |  |  |
| GTMIFRB2 | −107.30 | 28 | −90.50 | 15.64 | 1.03 | 0.80 | 84 | 1 | 61.4 |  |  |  |
| GUFMGG02 | −52.80 | 28 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 63.0 |  | 2 7 |  |
| HWA00002 | −165.80 | 28 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 28 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 28 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.8 |  | 1 |  |
| MEX02NTE | −135.80 | 28 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.5 |  | 1 | 10 |
| MEX02SUR | −126.80 | 28 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| PNRIFRB2 | −121.00 | 28 | −80.15 | 8.46 | 1.01 | 0.80 | 170 | 1 | 65.1 |  |  |  |
| PRU00004 | −85.80 | 28 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.2 |  |  |  |
| PTRVIR01 | −100.80 | 28 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.9 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 28 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 28 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −100.80 | 28 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 28 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 28 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 28 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 28 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 28 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.4 |  | 10 |  |
| USAWH102 | −156.80 | 28 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN02VEN | −103.80 | 28 | −63.50 | 15.50 | 0.80 | 0.80 | 90 | 2 | 60.1 |  | 9/GR22 |  |
| VEN11VEN | −103.80 | 28 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.6 |  | 9/GR22 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 632.24 MHz (29) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 29 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 29 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 29 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 29 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 29 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 29 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.2 |  | 9/GR4 |  |
| B CE311 | −64.20 | 29 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 29 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.2 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 29 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 29 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.0 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 29 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 29 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 29 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.1 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 29 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 29 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 29 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.5 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 29 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 29 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 29 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 29 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.0 |  | 9/GR5 |  |
| CAN01101 | −138.20 | 29 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 29 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 29 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 29 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 29 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.2 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 29 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 29 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 29 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 29 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 29 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 29 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.3 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 29 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 29 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.4 |  |  |  |
| CHLCONT5 | −106.20 | 29 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 29 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 29 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.3 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 29 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| EQACAND1 | −115.20 | 29 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 29 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.5 |  | 9/GR5 |  |
| HWA00002 | −166.20 | 29 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 29 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 58.9 |  | 9/GR2 | 10 |
| JMC00002 | −92.70 | 29 | −77.30 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.1 |  |  |  |
| KNA00001 | −79.70 | 29 | −62.46 | 17.44 | 0.80 | 0.80 | 90 | 1 | 58.6 |  |  |  |
| MEX01NTE | −78.20 | 29 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.7 |  | 1 |  |
| MEX01SUR | −69.20 | 29 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 29 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.4 |  | 1 | 10 |
| MEX02SUR | −127.20 | 29 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| PAQPAC01 | −106.20 | 29 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 29 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.4 |  |  |  |
| PRUAND02 | −115.20 | 29 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 29 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 29 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| SPMFRAN3 | −53.20 | 29 | −67.24 | 47.51 | 3.16 | 0.80 | 7 | 1 | 60.6 |  | 2 7 |  |
| SURINAM2 | −84.70 | 29 | −55.69 | 4.35 | 1.00 | 0.80 | 86 | 1 | 63.5 |  |  |  |
| URG00001 | −71.70 | 29 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 29 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 29 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 29 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.3 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 29 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 29 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.5 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 29 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 29 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.3 |  | 10 |  |
| USAWH102 | −157.20 | 29 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 29 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.6 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 646.82 MHz (30) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 30 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 30 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 30 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.0 |  |  |  |
| ARGNORT5 | −54.80 | 30 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.8 |  |  |  |
| ATNBEAM1 | −52.80 | 30 | −66.44 | 14.87 | 1.83 | 0.80 | 39 | 2 | 61.3 |  |  |  |
| B CE311 | −63.80 | 30 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −44.80 | 30 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.2 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 30 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −44.80 | 30 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.0 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 30 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −73.80 | 30 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.1 |  | 8 9/GR8 |  |
| B NO711 | −73.80 | 30 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.1 |  | 8 9/GR8 |  |
| B NO811 | −73.80 | 30 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.1 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 30 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |
| B SU111 | −80.80 | 30 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.1 |  | 8 9/GR6 |  |
| B SU112 | −44.80 | 30 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 30 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −44.80 | 30 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| BLZ00001 | −115.80 | 30 | −88.68 | 17.27 | 0.80 | 0.80 | 90 | 2 | 59.2 |  |  |  |
| CAN01101 | −137.80 | 30 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 30 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 30 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 30 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 30 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 30 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 30 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 30 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −81.80 | 30 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −90.80 | 30 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −81.80 | 30 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.3 |  | 9/GR14 |  |
| CAN01605 | −81.80 | 30 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.30 | 30 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.4 |  |  |  |
| CHLCONT4 | −105.80 | 30 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 30 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.7 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 30 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 61.9 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 30 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 56.9 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 30 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 30 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |
| CRBJMC01 | −92.30 | 30 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.3 |  | 9/GR18 |  |
| CTR00201 | −130.80 | 30 | −84.33 | 9.67 | 0.82 | 0.80 | 119 | 2 | 66.0 |  |  |  |
| DMAIFRB1 | −79.30 | 30 | −61.30 | 15.35 | 0.80 | 0.80 | 90 | 2 | 58.7 |  |  |  |
| EQAC0001 | −94.80 | 30 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 30 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.2 |  | 9/GR19 |  |
| HWA00002 | −165.80 | 30 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 30 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 30 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.7 |  | 1 |  |
| MEX02NTE | −135.80 | 30 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.4 |  | 1 | 10 |
| MEX02SUR | −126.80 | 30 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| NCG00003 | −107.30 | 30 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 63.6 |  |  |  |
| PRU00004 | −85.80 | 30 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.1 |  |  |  |
| PTRVIR01 | −100.80 | 30 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 30 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 30 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −100.80 | 30 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 30 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.3 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 30 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 30 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.5 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 30 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 30 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.3 |  | 10 |  |
| USAWH102 | −156.80 | 30 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN11VEN | −103.80 | 30 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.5 |  | 10 |  |

|  |  |  |  |  |  |  |  |  |  | 12 661.40 MHz (31) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −166.20 | 31 | −149.66 | 58.37 | 3.76 | 1.24 | 170 | 1 | 60.0 |  | 9/GR1 | 10 |
| ALS00003 | −175.20 | 31 | −150.98 | 58.53 | 3.77 | 1.11 | 167 | 1 | 60.2 |  | 9/GR2 | 10 |
| ARGINSU4 | −94.20 | 31 | −52.98 | −59.81 | 3.40 | 0.80 | 19 | 1 | 60.1 |  | 9/GR3 |  |
| ARGINSU5 | −55.20 | 31 | −44.17 | −59.91 | 3.77 | 0.80 | 13 | 1 | 59.5 |  | 9/GR4 |  |
| ARGSUR04 | −94.20 | 31 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 60.9 |  | 9/GR3 |  |
| ARGSUR05 | −55.20 | 31 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 60.3 |  | 9/GR4 |  |
| B CE311 | −64.20 | 31 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −45.20 | 31 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −64.20 | 31 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −45.20 | 31 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −64.20 | 31 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −74.20 | 31 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −74.20 | 31 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −74.20 | 31 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 63.1 |  | 8 9/GR8 |  |
| B SU111 | −81.20 | 31 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −45.20 | 31 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −81.20 | 31 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −45.20 | 31 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 61.6 |  | 8 9/GR9 | 12 |
| BERBERMU | −96.20 | 31 | −64.77 | 32.32 | 0.80 | 0.80 | 90 | 2 | 57.0 |  |  |  |
| BOLAND01 | −115.20 | 31 | −65.04 | −16.76 | 2.49 | 1.27 | 76 | 1 | 68.1 |  | 9/GR5 |  |
| BOL00001 | −87.20 | 31 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 64.2 |  |  |  |
| BRB00001 | −92.70 | 31 | −59.85 | 12.93 | 0.80 | 0.80 | 90 | 2 | 59.4 |  |  |  |
| CAN01101 | −138.20 | 31 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −138.20 | 31 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.70 | 31 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 59.8 |  |  |  |
| CAN01203 | −129.20 | 31 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −129.20 | 31 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −91.20 | 31 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 60.1 |  | 9/GR13 |  |
| CAN01403 | −129.20 | 31 | −89.75 | 52.02 | 4.68 | 0.80 | 148 | 1 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −91.20 | 31 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −82.20 | 31 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −91.20 | 31 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −82.20 | 31 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −82.20 | 31 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.70 | 31 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 60.5 |  |  |  |
| CHLCONT5 | −106.20 | 31 | −72.23 | −35.57 | 2.60 | 0.80 | 55 | 1 | 59.6 |  | 9/GR17 |  |
| CHLPAC02 | −106.20 | 31 | −80.06 | −30.06 | 1.36 | 0.80 | 69 | 1 | 59.4 |  | 9/GR17 |  |
| CLMAND01 | −115.20 | 31 | −74.72 | 5.93 | 3.85 | 1.63 | 114 | 1 | 65.4 |  | 9/GR5 | 10 |
| CLM00001 | −103.20 | 31 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 63.9 |  | 10 |  |
| CUB00001 | −89.20 | 31 | −79.81 | 21.62 | 2.24 | 0.80 | 168 | 1 | 61.3 |  |  |  |
| EQACAND1 | −115.20 | 31 | −78.40 | −1.61 | 1.37 | 0.95 | 75 | 1 | 64.4 |  | 9/GR5 |  |
| EQAGAND1 | −115.20 | 31 | −90.34 | −0.62 | 0.90 | 0.81 | 89 | 1 | 61.6 |  | 9/GR5 |  |
| GRD00059 | −57.20 | 31 | −61.58 | 12.29 | 0.80 | 0.80 | 90 | 1 | 58.7 |  |  |  |
| GRLDNK01 | −53.20 | 31 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 60.2 |  | 2 |  |
| GUY00201 | −84.70 | 31 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 63.8 |  |  |  |
| HWA00002 | −166.20 | 31 | −165.79 | 23.42 | 4.20 | 0.80 | 160 | 1 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −175.20 | 31 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 1 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −78.20 | 31 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 60.8 |  | 1 |  |
| MEX01SUR | −69.20 | 31 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 62.5 |  | 1 |  |
| MEX02NTE | −136.20 | 31 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 61.5 |  | 1 | 10 |
| MEX02SUR | −127.20 | 31 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 62.8 |  | 1 | 10 |
| MSR00001 | −79.70 | 31 | −61.73 | 16.75 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 4 |  |
| PAQPAC01 | −106.20 | 31 | −109.18 | −27.53 | 0.80 | 0.80 | 90 | 1 | 56.4 |  | 9/GR17 |  |
| PRG00002 | −99.20 | 31 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 60.5 |  |  |  |
| PRUAND02 | −115.20 | 31 | −74.69 | −8.39 | 3.41 | 1.79 | 95 | 1 | 64.3 |  | 9/GR5 |  |
| PTRVIR01 | −101.20 | 31 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 1 | 60.8 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −110.20 | 31 | −65.86 | 18.12 | 0.80 | 0.80 | 90 | 1 | 61.3 |  | 1 6 9/GR21 |  |
| URG00001 | −71.70 | 31 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 60.2 |  |  |  |
| USAEH001 | −61.70 | 31 | −85.19 | 36.21 | 5.63 | 3.33 | 22 | 1 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −101.20 | 31 | −89.24 | 36.16 | 5.67 | 3.76 | 170 | 1 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −110.20 | 31 | −90.14 | 36.11 | 5.55 | 3.55 | 161 | 1 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −119.20 | 31 | −91.16 | 36.05 | 5.38 | 3.24 | 152 | 1 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −166.20 | 31 | −117.80 | 40.58 | 4.03 | 0.82 | 135 | 1 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −175.20 | 31 | −118.27 | 40.12 | 3.62 | 0.80 | 136 | 1 | 65.4 |  | 9/GR2 |  |
| USAWH101 | −148.20 | 31 | −109.65 | 38.13 | 5.53 | 1.95 | 142 | 1 | 62.4 |  | 10 |  |
| USAWH102 | −157.20 | 31 | −111.41 | 38.57 | 5.51 | 1.54 | 138 | 1 | 63.5 |  | 10 |  |
| VENAND03 | −115.20 | 31 | −67.04 | 6.91 | 2.37 | 1.43 | 111 | 1 | 67.7 |  | 9/GR5 | 10 |

|  |  |  |  |  |  |  |  |  |  | 12 675.98 MHz (32) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 |  | 9 |  |
| ALS00002 | −165.80 | 32 | −149.63 | 58.52 | 3.81 | 1.23 | 171 | 2 | 59.9 |  | 9/GR1 | 10 |
| ALS00003 | −174.80 | 32 | −150.95 | 58.54 | 3.77 | 1.11 | 167 | 2 | 60.2 |  | 9/GR2 | 10 |
| ARGNORT4 | −93.80 | 32 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 66.1 |  |  |  |
| ARGNORT5 | −54.80 | 32 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 63.9 |  |  |  |
| B CE311 | −63.80 | 32 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 61.9 |  | 8 9/GR7 |  |
| B CE312 | −44.80 | 32 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 61.3 |  | 8 9/GR9 | 10 11 |
| B CE411 | −63.80 | 32 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 62.9 |  | 8 9/GR7 |  |
| B CE412 | −44.80 | 32 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 63.1 |  | 8 9/GR9 | 10 12 |
| B CE511 | −63.80 | 32 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 63.4 |  | 8 9/GR7 |  |
| B NO611 | −73.80 | 32 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO711 | −73.80 | 32 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 63.2 |  | 8 9/GR8 |  |
| B NO811 | −73.80 | 32 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 63.2 |  | 8 9/GR8 |  |
| B SE911 | −101.80 | 32 | −45.99 | −19.09 | 2.22 | 0.80 | 62 | 2 | 65.7 |  | 8 |  |
| B SU111 | −80.80 | 32 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 63.2 |  | 8 9/GR6 |  |
| B SU112 | −44.80 | 32 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 62.6 |  | 8 9/GR9 | 11 |
| B SU211 | −80.80 | 32 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 62.8 |  | 8 9/GR6 |  |
| B SU212 | −44.80 | 32 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 61.6 |  | 8 9/GR9 | 12 |
| CAN01101 | −137.80 | 32 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 59.7 |  | 9/GR10 | 10 |
| CAN01201 | −137.80 | 32 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 59.8 |  | 9/GR10 | 10 |
| CAN01202 | −72.30 | 32 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 59.8 |  |  |  |
| CAN01203 | −128.80 | 32 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 59.7 |  | 9/GR12 | 10 |
| CAN01303 | −128.80 | 32 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 60.3 |  | 9/GR12 | 10 |
| CAN01304 | −90.80 | 32 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 60.0 |  | 9/GR13 |  |
| CAN01403 | −128.80 | 32 | −89.70 | 52.02 | 4.67 | 0.80 | 148 | 2 | 62.1 |  | 9/GR12 | 10 |
| CAN01404 | −90.80 | 32 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 60.6 |  | 9/GR13 |  |
| CAN01405 | −81.80 | 32 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01504 | −90.80 | 32 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 60.4 |  | 9/GR13 |  |
| CAN01505 | −81.80 | 32 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 60.4 |  | 9/GR14 |  |
| CAN01605 | −81.80 | 32 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 60.5 |  | 9/GR14 |  |
| CAN01606 | −70.30 | 32 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 60.5 |  |  |  |
| CHLCONT4 | −105.80 | 32 | −69.59 | −23.20 | 2.21 | 0.80 | 68 | 2 | 59.3 |  | 9/GR16 |  |
| CHLCONT6 | −105.80 | 32 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 59.8 |  | 9/GR16 |  |
| CRBBAH01 | −92.30 | 32 | −76.09 | 24.13 | 1.83 | 0.80 | 141 | 1 | 62.0 |  | 9/GR18 |  |
| CRBBER01 | −92.30 | 32 | −64.76 | 32.13 | 0.80 | 0.80 | 90 | 1 | 57.0 |  | 9/GR18 |  |
| CRBBLZ01 | −92.30 | 32 | −88.61 | 17.26 | 0.80 | 0.80 | 90 | 1 | 58.9 |  | 9/GR18 |  |
| CRBEC001 | −92.30 | 32 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 64.6 |  | 9/GR18 |  |
| CRBJMC01 | −92.30 | 32 | −79.45 | 17.97 | 0.99 | 0.80 | 151 | 1 | 61.4 |  | 9/GR18 |  |
| EQAC0001 | −94.80 | 32 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 63.3 |  | 9/GR19 |  |
| EQAG0001 | −94.80 | 32 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 61.3 |  | 9/GR19 |  |
| GRD00003 | −79.30 | 32 | −61.62 | 12.34 | 0.80 | 0.80 | 90 | 2 | 58.9 |  |  |  |
| GTMIFRB2 | −107.30 | 32 | −90.50 | 15.64 | 1.03 | 0.80 | 84 | 1 | 61.4 |  |  |  |
| GUFMGG02 | −52.80 | 32 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 63.0 |  | 2 7 |  |
| HWA00002 | −165.80 | 32 | −165.79 | 23.32 | 4.20 | 0.80 | 160 | 2 | 59.0 |  | 9/GR1 | 10 |
| HWA00003 | −174.80 | 32 | −166.10 | 23.42 | 4.25 | 0.80 | 159 | 2 | 59.0 |  | 9/GR2 | 10 |
| MEX01NTE | −77.80 | 32 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 60.8 |  | 1 |  |
| MEX02NTE | −135.80 | 32 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 61.5 |  | 1 | 10 |
| MEX02SUR | −126.80 | 32 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 62.8 |  | 1 | 10 |
| PNRIFRB2 | −121.00 | 32 | −80.15 | 8.46 | 1.01 | 0.80 | 170 | 1 | 65.1 |  |  |  |
| PRU00004 | −85.80 | 32 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 63.2 |  |  |  |
| PTRVIR01 | −100.80 | 32 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 60.9 |  | 1 6 9/GR20 |  |
| PTRVIR02 | −109.80 | 32 | −65.85 | 18.12 | 0.80 | 0.80 | 90 | 2 | 61.4 |  | 1 6 9/GR21 |  |
| USAEH001 | −61.30 | 32 | −85.16 | 36.21 | 5.63 | 3.32 | 22 | 2 | 62.1 |  | 1 5 6 |  |
| USAEH002 | −100.80 | 32 | −89.28 | 36.16 | 5.65 | 3.78 | 170 | 2 | 62.0 |  | 1 6 9/GR20 | 10 |
| USAEH003 | −109.80 | 32 | −90.12 | 36.11 | 5.55 | 3.56 | 161 | 2 | 62.4 |  | 1 6 9/GR21 | 10 |
| USAEH004 | −118.80 | 32 | −91.16 | 36.05 | 5.38 | 3.24 | 153 | 2 | 62.9 |  | 1 5 6 | 10 |
| USAPSA02 | −165.80 | 32 | −117.79 | 40.58 | 4.04 | 0.82 | 135 | 2 | 63.6 |  | 9/GR1 |  |
| USAPSA03 | −174.80 | 32 | −118.20 | 40.15 | 3.63 | 0.80 | 136 | 2 | 65.3 |  | 9/GR2 |  |
| USAWH101 | −147.80 | 32 | −109.70 | 38.13 | 5.52 | 1.96 | 142 | 2 | 62.4 |  | 10 |  |
| USAWH102 | −156.80 | 32 | −111.40 | 38.57 | 5.51 | 1.55 | 138 | 2 | 63.5 |  | 10 |  |
| VEN02VEN | −103.80 | 32 | −63.50 | 15.50 | 0.80 | 0.80 | 90 | 2 | 60.1 |  | 9/GR22 |  |
| VEN11VEN | −103.80 | 32 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 65.6 |  | 9/GR22 | 10 |

ARTICLE  11     (Rev.WRC-15)

Plan for the broadcasting-satellite service in the frequency bands 
11.7-12.2 GHz in Region 3 and 11.7-12.5 GHz in Region 1

11.1		COLUMN  HEADINGS  OF  THE  PLAN

Col. 1	Notifying administration symbol.

Col. 2	Beam identification (Column 2, normally, contains the symbol designating the administration or the geographical area taken from Table B1 of the Preface to the International Frequency List, followed by the symbol designating the service area).

Col. 3	Nominal orbital position, in degrees and hundredths of a degree from the Greenwich meridian (negative values indicate longitudes which are west of the Greenwich meridian; positive values indicate longitudes which are east of the Greenwich meridian).

Col. 4	Nominal intersection of the beam axis with the Earth (boresight or aim point in the case of a non-elliptical beam), longitude and latitude, in degrees and hundredths of a degree.

Col. 5	Space station transmitting antenna characteristics (elliptical beams). This column contains three numerical values corresponding to the major axis, the minor axis and the major axis orientation respectively of the elliptical cross-section half-power beamwidth, in degrees and hundredths of a degree. Orientation of the ellipse is determined as follows: in a plane normal to the beam axis, the direction of a major axis of the ellipse is specified as the angle measured anticlockwise from a line parallel to the equatorial plane to the major axis of the ellipse, to the nearest degree.

Col. 6	Space station transmitting antenna pattern code.

The codes used for the antenna pattern of the transmitting space station (downlink) antenna are defined as follows:

|  | MOD13FRTSS | Figure 13 in Annex 5 (Recommendation ITU-R BO.1445) |
| --- | --- | --- |
|  | R13TSS | Figure 9 and § 3.13.3 in Annex 5 |
|  | R123FR | Figure 11 and § 3.13.3 in Annex 5 |

In cases where the “Space station transmitting antenna pattern code” field is blank, the necessary antenna pattern data are provided by shaped beam data submitted by the administration. These data are stored in Column 7. A particular shaped beam is identified by the combination of Column 1, Column 7 and Column 13. In such cases the maximum cross-polar gain is given under Column 8 in the “Cross-polar gain” field.

In cases where the “Space station transmitting antenna pattern code” field contains a code which starts with “CB_” characters, it is a composite beam. Any composite beam consists of two or more elliptical beams. Each composite beam is described in the special composite beam file as having the same name plus a GXT extension (e.g. the description of the CB_COMP_BM1 composite beam is stored in the CB_COMP_BM1.GXT file).

Col. 7	Space station transmitting antenna shaped (non-elliptical and non-composite) beam identification.

Col. 8	Maximum space station transmitting antenna co-polar and cross-polar (in the case of shaped beam) isotropic gain (dBi).

Col. 9	Earth station receiving antenna pattern code and maximum antenna co-polar gain (dBi).

The codes used for receiving earth station (downlink) antenna patterns are defined as follows:

|  | R13RES | Figure 7 and § 3.7.2 in Annex 5 |
| --- | --- | --- |
|  | MODRES | Figure 7bis and § 3.7.2 in Annex 5 (Recommendation ITU-R BO.1213) |

Col. 10	Polarization (CL  circular left, CR  circular right, LE  linear referenced to the equatorial plane) and polarization angle in degrees and hundredths of a degree (in the case of linear polarization only).

Col. 11	e.i.r.p. in the direction of maximum radiation (dBW).

Col. 12	Designation of emission.

Col. 13	Identity of the space station.

Col. 14	Group code (an identification code which indicates that all assignments with the same group identification code will be treated as a group).

Group code: if an assignment is part of the group:

a)	The equivalent protection margin to be used for the application of Article 4 shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

–	for the calculation of interference from assignments belonging to a group to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.

b)	If an administration notifies the same frequency in more than one beam of a group for use at the same time, the aggregate carrier-to-interference ratio (C / I) produced by all emissions from that group shall not exceed the C / I ratio calculated on the basis of § a) above.

Col. 15	Assignment status.

The assignment status codes used for beams are defined as follows:

|  | P | Assignment in the Plan which has not been brought into use and/or the date of bringing into use has not been confirmed to the Bureau. For this category of assignments, WRC-2000 protection ratios are applied (21 dB co-channel and 16 dB adjacent channel). |
| --- | --- | --- |
|  | PE | Assignment in the Plan which is in conformity with Appendix 30, has been notified, brought into use and the date of bringing into use has been confirmed to the Bureau before 12 May 2000. For this category of assignments, WRC-97 protection ratios are applied (24 dB co-channel and 16 dB adjacent channel). |

Col. 16	Remarks.

11.2		TEXT  FOR  NOTES  IN  THE  REMARKS  COLUMN
		OF  THE  PLAN     (REV.WRC-15)

1	To be dedicated to the Islamic programme envisaged in WARC SAT-77 documents.

2	Not used.

3	Provisional beam. These assignments have been included in the Plan by WRC-97. These assignments are for exclusive use by Palestine, subject to the Israeli-Palestinian Interim Agreement of 28 September 1995, Resolution 741 of the Council notwithstanding and Resolution 99 (Minneapolis, 1998) of the Plenipotentiary Conference.

4	Assignment intended to ensure coverage of Algeria, Libya, Morocco, Mauritania and Tunisia, with the agreement of the countries concerned. If required, it may be used with the characteristics of the beam TUN15000.

5	This assignment shall be brought into use only when the limits referred to in Table 1 are not exceeded or with the agreement of the administrations identified in Table 2, whose networks/beams listed in this Table may be affected, with respect to (see also the Note to § 11.2):

a)	assignments in the Region 2 Plan on 12 May 2000; or

b)	assignments in the terrestrial services which are recorded in the Master Register with a favourable finding or received by the Bureau prior to 12 May 2000 for recording in the Master Register and which subsequently receive a favourable finding based on the Plan as it existed on 12 May 2000; or

c)	assignments in the fixed-satellite service which:

–	are recorded in the Master Register prior to 12 May 2000 with a favourable finding; or

–	have been coordinated under the provisions of No. 9.7 (or No. 1060) or § 7.2.1 of Article 7 prior to 12 May 2000; or

–	are in process of coordination under the provisions of No. 9.7 (or No. 1060) or § 7.2.1 of Article 7 prior to 31 July 2000 for which complete Appendix 4 data (or Appendix 3 data, as appropriate) have been received by the Bureau under the relevant provisions of Article 9 (or Article 11, as appropriate):

–	filings received by the Bureau prior to 12 May 2000 at 1700 h (Istanbul time) shall be taken into account in the pertinent compatibility analysis by applying the pfd criteria referred to in Table 1; or

–	filings received by the Bureau after 12 May 2000 at 1700 h (Istanbul time), but before 31 July 2000, shall be taken into account by applying the sharing criteria of −138 dB(W/(m2 · 27 MHz)) or the pfd criteria referred to in Table 1, whichever is higher.

6	This assignment shall not claim protection from interference caused by the assignments which pertain to networks/beams identified in Table 3 which are in conformity with the Region 2 Plan on 12 May 2000 (see also the Note to § 11.2).

7	This assignment shall not claim protection from interference caused by assignments in the fixed-satellite service which pertain to networks/beams identified in Table 3 (see also the Note to § 11.2) and:

a)	either are recorded in the Master Register with a favourable finding prior to 12 May 2000;

b)	or for which complete Appendix 4 data (or Appendix 3 data, as appropriate) under the relevant provisions of Article 9 (or No. 1060, or § 7.2.1 of Article 7, as appropriate) have been received prior to 12 May 2000, which have been brought into use prior to 12 May 2000 and for which the complete due diligence information, in accordance with Annex 2 to Resolution 49 (Rev.WRC-15), has been received prior to 12 May 2000.      (WRC-15)

8	This assignment shall not claim protection from the assignments of the administrations for terrestrial services identified in Table 4 which are recorded in the Master Register with a favourable finding, or received by the Bureau prior to 12 May 2000 for recording in the Master Register and which subsequently receive a favourable finding based on the Plan as it existed on 12 May 2000 (see also the Note to § 11.2).

9	(SUP - WRC-12)

TABLE  1

| Symbol | Criteria |
| --- | --- |
| a | § 3 of Annex 11 |
| b | § 4 of Annex 11 |
| c | § 6 of Annex 11 |
| 1 These paragraphs and this Annex are contained in this Appendix as adopted by WRC-03. |  |

NOTE – In cases where assignments from the WRC-97 Plan without remarks were included in the WRC-2000 Regions 1 and 3 Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna characteristics, the coordination status afforded by the WRC-97 Plan shall be preserved.

In cases where assignments from the WRC-97 Plan with remarks were included in the WRC-2000 Regions 1 and 3 Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the compatibility will be reassessed using the revised criteria and methodology of WRC-2000 and the remarks of the WRC-97 Plan assignment will either be maintained or reduced on the basis of the results of this analysis.

In other cases the methodology described in Notes 5 to 8 shall be applied.

TABLE 2    (WRC-15)

Affected administrations and corresponding networks/beams identified based on Note 5 in § 11.2 of Article 11

| Beam name | Channels | Ref. Table 1 | Affected administrations* | Affected networks/beams* |
| --- | --- | --- | --- | --- |
| ARS34000 | 40 | c | CHN, G, J, KOR, MLA, THA, UAE, USA | AM-SAT A4, ASIASAT-AKX, ASIASAT-CKX, ASIASAT-EKX, EMARSAT-1G, JCSAT-3A, JCSAT-3B, KOREASAT-1, MEASAT-1, MEASAT-91.5E, N-SAT-110E, N-SAT-128, SJC-1, THAICOM-A2B, THAICOM-G1K |
| BEL01800 | 26, 28, 30, 32, 34, 36, 38, 40 | c | PAK | PAKSAT-1 |
| BFA10700 | 22, 24 | c | E | HISPASAT-1, HISPASAT-2C3 KU |
| CVA08300 | 1, 3, 5, 7, 9, 11 | c | USA | INTELSAT7 359E, INTELSAT8 359E, INTELSAT10 359E |
| CYP08600 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 359E, INTELSAT8 359E |
| FSM00000 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 157E |
| GMB30200 | 1, 5, 9, 13, 17 | c | USA | USASAT-26A |
| GNB30400 | 22, 24 | c | E | HISPASAT-1, HISPASAT-2C3 KU |
| GRC10500 | 2, 4, 6, 8, 10, 12 | c | USA | INTELSAT7 359E, INTELSAT8 359E, INTELSAT10 359E |
| GUI19200 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | c | USA | USASAT-26A |
| IRL21100 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | c | USA | USASAT-26A |
| ISL04900 | 27 | a | GUY | GUY00302 |
| ISL04900 | 29, 39 | a | JMC | JMC00005 |
| ISL04900 | 31, 33, 35, 37 | a | GUY, JMC | GUY00302, JMC00005 |
| ISL04900 | 23 | c | B, USA | B-SAT I, USASAT-14L |
| KIR__100 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 177E, USASAT-14K |
| KIR__100 | 17 | c | USA | USASAT-14K |
| MLI__100 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 342E, INTELSAT8 342E, INTELSAT8 340E |
| MNG24800 | 31, 35 | c | CHN, THA | APSTAR-4, THAICOM-A2B, THAICOM-G1K |
| MOZ30700 | 2, 6, 10 | c | USA | INTELSAT7 359E, INTELSAT8 359E, INTELSAT10 359E |
| NGR11500 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | c | USA | USASAT-26A |
| NOR12000 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 359E, INTELSAT8 359E, INTELSAT10 359E |
| POR__100 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | c | USA | USASAT-26A |
| RUS-4 | 28, 29, 33, 37 | c | G, KOR | AM-SAT A4, KOREASAT-1, KOREASAT-2 |
| RUS-4 | 31, 35, 39 | c | G | AM-SAT A4 |
| SEN22200 | 23 | c | USA | USASAT-26A |
| SOM31200 | 26, 28, 30, 32, 34, 36, 38, 40 | c | PAK | PAKSAT-1 |
| TGO22600 | 1, 3, 5, 7, 9, 11 | c | USA | INTELSAT8 330.5E |
| TGO22600 | 13 | c | E, USA | HISPASAT-1, HISPASAT-2C3 KU, INTELSAT8 330.5E |
| TGO22600 | 15, 17, 19 | c | E | HISPASAT-1, HISPASAT-2C3 KU |
| TJK06900 | 26, 28, 30, 32, 34, 36, 38, 40 | c | PAK | PAKSAT-1 |
| TKM06800 | 26, 28, 30, 32, 34, 36, 38, 40 | c | UAE | EMARSAT-1G |
| TON21500 | 2, 6, 10, 14, 18 | c | USA | USASAT-14K |
| ZWE13500 | 1, 3, 5, 7, 9, 11, 13 | c | USA | INTELSAT7 359E, INTELSAT8 359E |
| * Administrations and corresponding networks/beams whose assignment(s) may receive interference from the beam shown in the left-hand column. |  |  |  |  |

TABLE 3    (WRC-15)

Affecting administrations and corresponding networks/beams identified based on Notes 6 and 7 in § 11.2 of Article 11

| Beam name | Channels | Note | Affecting administrations* | Affecting networks/beams* |
| --- | --- | --- | --- | --- |
| AND34100 | 2, 6, 10, 12, 14, 16, 18, 20 | 7 | USA | USASAT-26A |
| BFA10700 | 22, 24 | 7 | E | HISPASAT-1, HISPASAT-2C3 KU |
| CVA08300 | 1, 3, 5, 7, 9, 11 | 7 | USA | INTELSAT7 359E |
| CYP08600 | 1, 3, 5, 7, 9, 11, 13 | 7 | USA | INTELSAT7 359E |
| DNK090XR | 29 | 6 | JMC | JMC00005 |
| DNK090XR | 33 | 6 | GUY, JMC | GUY00302, JMC00005 |
| DNK091XR | 31, 35 | 6 | GUY, JMC | GUY00302, JMC00005 |
| FJI19300 | 1, 3, 5, 7, 9, 11, 13 | 7 | HOL | INTELSAT7 183E |
| GMB30200 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | 7 | USA | USASAT-26A |
| GNB30400 | 22, 24 | 7 | E | HISPASAT-1, HISPASAT-2C3 KU |
| GRC10500 | 2, 4, 6, 8, 10, 12 | 7 | USA | INTELSAT7 359E |
| GUI19200 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | 7 | USA | USASAT-26A |
| IRL21100 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | 7 | USA | USASAT-26A |
| ISL04900 | 27 | 6 | GUY | GUY00302 |
| ISL04900 | 29, 39 | 6 | JMC | JMC00005 |
| ISL04900 | 31, 33, 35, 37 | 6 | GUY, JMC | GUY00302, JMC00005 |
| KIR__100 | 1, 3, 5, 7, 9, 11, 13 | 7 | USA | INTELSAT7 177E |
| MNG24800 | 27 | 7 | J | SUPERBIRD-C |
| MNG24800 | 29, 31, 33, 35, 37, 39 | 7 | J, THA | THAICOM-A2B, SUPERBIRD-C |
| MOZ30700 | 2, 6, 10, 12 | 7 | USA | INTELSAT7 359E |
| MTN__100 | 22, 24, 26 | 7 | USA | USASAT-26A |
| NGR11500 | 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 | 7 | USA | USASAT-26A |
| NOR12000 | 1, 3, 5, 7, 9, 11, 13 | 7 | USA | INTELSAT7 359E |
| POR__100 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | 7 | USA | USASAT-26A |
| RUS-4 | 25 | 7 | J | JCSAT-1R, SUPERBIRD-C |
| RUS-4 | 28, 29 | 7 | J, KOR | SUPERBIRD-C, KOREASAT-1, KOREASAT-2 |
| RUS-4 | 31, 33, 35, 37, 39 | 7 | J, KOR | SUPERBIRD-C, KOREASAT-1, KOREASAT-2 |
| SEN22200 | 23, 25 | 7 | USA | USASAT-26A |
| SMO05700 | 1, 3, 5, 7, 9, 11, 13 | 7 | HOL | INTELSAT7 183E |
| SMR31100 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19 | 7 | USA | USASAT-26A |
| SRL25900 | 27 | 6 | GUY | GUY00302 |
| SRL25900 | 29, 39 | 6 | JMC | JMC00005 |
| SRL25900 | 31, 33, 35, 37 | 6 | GUY, JMC | GUY00302, JMC00005 |
| TGO22600 | 13 | 7 | E | HISPASAT-2C3 KU |
| TGO22600 | 15, 17, 19 | 7 | E | HISPASAT-1, HISPASAT-2C3 KU |
| ZWE13500 | 1, 3, 5, 7, 9, 11, 13 | 7 | USA | INTELSAT7 359E |
| * Administrations and corresponding networks/beams whose assignment(s) may cause interference to the beam shown in the left-hand column. |  |  |  |  |

TABLE  4    (WRC-07)

Affecting administrations and corresponding terrestrial stations identified based on Note 8 in § 11.2 of Article 11

| Beam name | Channels | Affecting administrations* | Affecting terrestrial stations* |
| --- | --- | --- | --- |
| EGY02600 | 2 | ISR | HERZILIYA |
| F 09300 | 24, 26 | SUI | GENEVE STUDIO C VOGT |
| I 08200 | 38, 40 | AUT | EHRWALD |
| JOR22400 | 2 | ISR | HERZILIYA, JERUSALEM |
| RUS-4 | 25, 26, 27, 28, 29, 31, 33, 35, 37, 39 | J1 |  |
| * Administrations and corresponding terrestrial stations whose assignment(s) may cause interference to the beam shown in the left-hand column. 1 The identification of this administration is based on its typical terrestrial station assignments as recorded in the Master Register. |  |  |  |

TABLE  5

Table showing correspondence between channel
numbers and assigned frequencies

| Channel No. | Assigned frequency (MHz) | Channel No. | Assigned frequency (MHz) |
| --- | --- | --- | --- |
| 1 | 11 727.48 | 21 | 12 111.08 |
| 2 | 11 746.66 | 22 | 12 130.26 |
| 3 | 11 765.84 | 23 | 12 149.44 |
| 4 | 11 785.02 | 24 | 12 168.62 |
| 5 | 11 804.20 | 25 | 12 187.80 |
| 6 | 11 823.38 | 26 | 12 206.98 |
| 7 | 11 842.56 | 27 | 12 226.16 |
| 8 | 11 861.74 | 28 | 12 245.34 |
| 9 | 11 880.92 | 29 | 12 264.52 |
| 10 | 11 900.10 | 30 | 12 283.70 |
| 11 | 11 919.28 | 31 | 12 302.88 |
| 12 | 11 938.46 | 32 | 12 322.06 |
| 13 | 11 957.64 | 33 | 12 341.24 |
| 14 | 11 976.82 | 34 | 12 360.42 |
| 15 | 11 996.00 | 35 | 12 379.60 |
| 16 | 12 015.18 | 36 | 12 398.78 |
| 17 | 12 034.36 | 37 | 12 417.96 |
| 18 | 12 053.54 | 38 | 12 437.14 |
| 19 | 12 072.72 | 39 | 12 456.32 |
| 20 | 12 091.90 | 40 | 12 475.50 |
| Note – Assigned frequency = 11 708.30 + 19.18 n, where n is the channel number. |  |  |  |

TABLE 6A   (WRC-15)

Basic characteristics of the Regions 1 and 3 Plan (sorted by administration)

| 1 | 2 | 3 | 4 |  | 5 |  |  | 6 | 7 | 8 |  | 9 |  | 10 |  | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Admin. symbol | Beam identification | Orbital position | Boresight |  | Space station antenna characteristics |  |  | Space station antenna code | Shaped beam | Space station antenna gain |  | Earth station antenna |  | Polarization |  | e.i.r.p. | Designation of emission | Identity of the space station | Group code | Status | Remarks |
|  |  |  | Long. | Lat. | Major axis | Minor axis | Orien- tation |  |  | Co-polar | Cross- polar | Code | Gain | Type | Angle |  |  |  |  |  |  |
| AFG | AFG__100 | 50.00 | 65.88 | 33.86 |  |  |  | CB_TSS_AFGA |  | 42.71 |  | MODRES | 35.50 | CL |  | 58.4 | 27M0G7W |  |  | P |  |
| AFS | AFS02100 | 4.80 | 24.50 | −28.00 | 3.13 | 1.68 | 27.00 | R13TSS |  | 37.24 |  | MODRES | 35.50 | CL |  | 59.1 | 27M0G7W |  |  | P |  |
| AGL | AGL29500 | −24.80 | 16.06 | −12.45 | 2.42 | 1.88 | 77.88 | R13TSS |  | 37.87 |  | MODRES | 35.50 | CL |  | 59.1 | 27M0G7W |  |  | P |  |
| ALB | ALB29600 | 62.00 | 20.04 | 41.23 | 0.60 | 0.60 | 61.32 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| ALG | ALG__100 | −24.80 | 1.86 | 27.60 |  |  |  | CB_TSS_ALGA |  | 39.59 |  | MODRES | 35.50 | CL |  | 54.5 | 27M0G7W |  |  | P |  |
| AND | AND34100 | −37.00 | 1.60 | 42.50 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 56.5 | 27M0G7W |  |  | P | 7 |
| ARM | ARM06400 | 22.80 | 44.99 | 39.95 | 0.73 | 0.60 | 148.17 | R13TSS |  | 48.02 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| ARS | ARS__100 | 17.00 | 44.72 | 23.76 |  |  |  | CB_TSS_ARSA |  | 37.81 |  | MODRES | 35.50 | CL |  | 57.7 | 27M0G7W |  | 54 | P |  |
| ARS | ARS34000 | 17.00 | 52.30 | 24.80 | 2.68 | 0.70 | 143.00 | R13TSS |  | 41.71 |  | MODRES | 35.50 | CL |  | 59.2 | 27M0G7W |  | 54 | P | 5 |
| AUS | AUS00400 | 152.00 | 123.00 | −24.20 | 3.06 | 2.17 | 102.00 | R13TSS |  | 36.22 |  | MODRES | 35.50 | CR |  | 58.2 | 27M0G7W |  | 30 | P |  |
| AUS | AUS0040A | 152.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 30 | P |  |
| AUS | AUS0040B | 152.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 30 | P |  |
| AUS | AUS0040C | 152.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 30 | P |  |
| AUS | AUS00500 | 152.00 | 133.90 | −18.40 | 2.82 | 1.74 | 105.00 | R13TSS |  | 37.53 |  | MODRES | 35.50 | CL |  | 59.4 | 27M0G7W |  |  | P |  |
| AUS | AUS00600 | 152.00 | 136.60 | −30.90 | 2.41 | 1.52 | 161.00 | R13TSS |  | 38.80 |  | MODRES | 35.50 | CL |  | 58.4 | 27M0G7W |  |  | P |  |
| AUS | AUS00700 | 164.00 | 145.20 | −38.10 | 2.12 | 1.02 | 147.00 | R13TSS |  | 41.09 |  | MODRES | 35.50 | CR |  | 58.5 | 27M0G7W |  | 31 | P |  |
| AUS | AUS0070A | 164.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 31 | P |  |
| AUS | AUS00800 | 164.00 | 145.90 | −21.70 | 3.62 | 1.63 | 136.00 | R13TSS |  | 36.73 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| AUS | AUS00900 | 164.00 | 147.50 | −32.10 | 2.31 | 1.43 | 187.00 | R13TSS |  | 39.25 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  | 32 | P |  |
| AUS | AUS0090A | 164.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 32 | P |  |
| AUS | AUS0090B | 164.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  | 32 | P |  |
| AUS | AUSA_100 | 152.00 | 132.38 | −38.37 |  |  |  | CB_TSS_AUSA |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| AUS | AUSB_100 | 164.00 | 132.38 | −38.37 |  |  |  | CB_TSS_AUSB |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| AUT | AUT01600 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MOD13FRTSS |  | 42.19 |  | MODRES | 35.50 | CR |  | 59.1 | 27M0G7W |  |  | P |  |
| AZE | AZE06400 | 23.20 | 47.47 | 40.14 | 0.93 | 0.60 | 158.14 | R13TSS |  | 46.98 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| BDI | BDI27000 | 11.00 | 29.90 | −3.10 | 0.71 | 0.60 | 80.00 | R13TSS |  | 48.15 |  | MODRES | 35.50 | CL |  | 58.4 | 27M0G7W |  |  | P |  |
| BEL | BEL01800 | 38.20 | 5.12 | 51.96 | 1.00 | 1.00 | 24.53 | MOD13FRTSS |  | 44.45 |  | MODRES | 35.50 | CL |  | 55.5 | 27M0G7W |  |  | P | 5 |
| BEN | BEN23300 | −19.20 | 2.20 | 9.50 | 1.44 | 0.68 | 97.00 | R13TSS |  | 44.54 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P |  |
| BFA | BFA10700 | −30.00 | −1.50 | 12.20 | 1.45 | 1.14 | 29.00 | R13TSS |  | 42.26 |  | MODRES | 35.50 | CL |  | 57.0 | 27M0G7W |  |  | P | 5, 7 |
| BGD | BGD22000 | 74.00 | 90.30 | 23.60 | 1.46 | 0.84 | 135.00 | R13TSS |  | 43.56 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| BHR | BHR25500 | 34.00 | 50.50 | 26.10 | 0.60 | 0.60 | 0.00 | MOD13FRTSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 54.5 | 27M0G7W |  |  | P |  |
| BIH | BIH14800 | 56.00 | 18.22 | 43.97 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| BLR | BLR06200 | 37.80 | 27.91 | 53.06 | 1.21 | 0.60 | 11.47 | R13TSS |  | 45.83 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| BOT | BOT29700 | −0.80 | 23.30 | −22.20 | 2.13 | 1.50 | 36.00 | R13TSS |  | 39.40 |  | MODRES | 35.50 | CL |  | 58.7 | 27M0G7W |  |  | P |  |
| BRM | BRM29800 | 104.00 | 96.97 | 18.67 | 3.33 | 1.66 | 91.58 | R13TSS |  | 37.04 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| BRU | BRU33000 | 74.00 | 114.70 | 4.40 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 57.5 | 27M0G7W |  |  | P |  |
| BTN | BTN03100 | 86.00 | 90.44 | 27.05 | 0.72 | 0.60 | 175.47 | R13TSS |  | 48.11 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| BUL | BUL02000 | −1.20 | 25.00 | 43.00 | 1.04 | 0.60 | 165.00 | R13TSS |  | 46.50 |  | MODRES | 35.50 | CL |  | 58.6 | 27M0G7W |  |  | P |  |
| CAF | CAF25800 | −13.20 | 21.00 | 6.30 | 2.25 | 1.68 | 31.00 | R13TSS |  | 38.67 |  | MODRES | 35.50 | CL |  | 59.3 | 27M0G7W |  |  | P |  |
| CBG | CBG29900 | 86.00 | 104.82 | 12.34 | 1.04 | 0.86 | 9.45 | R13TSS |  | 44.91 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  |  | P |  |
| CHN | CHN15500 | 62.00 | 88.18 | 31.20 | 3.03 | 1.24 | 163.23 | R13TSS |  | 38.69 |  | MODRES | 35.50 | CL |  | 57.9 | 27M0G7W |  |  | P |  |
| CHN | CHN15800 | 134.00 | 113.29 | 39.70 | 2.80 | 1.55 | 35.44 | R13TSS |  | 38.07 |  | MODRES | 35.50 | CR |  | 57.0 | 27M0G7W |  |  | P |  |
| CHN | CHN19000 | 122.00 | 114.17 | 23.32 | 0.91 | 0.60 | 2.88 | MOD13FRTSS |  | 47.08 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| CHN | CHN20000 | 122.00 | 113.55 | 22.20 | 0.60 | 0.60 | 0.00 | MOD13FRTSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.0 | 27M0G7W |  |  | P |  |
| CHN | CHNA_100 | 62.00 | 90.56 | 39.22 |  |  |  | CB_TSS_CHNA |  | 40.01 |  | MODRES | 35.50 | CR |  | 58.5 | 27M0G7W |  |  | P |  |
| CHN | CHNC_100 | 134.00 | 105.77 | 27.56 |  |  |  | CB_TSS_CHNC |  | 39.51 |  | MODRES | 35.50 | CL |  | 57.1 | 27M0G7W |  |  | P |  |
| CHN | CHNE_100 | 92.20 | 114.96 | 20.16 |  |  |  | CB_TSS_CHNE |  | 44.74 |  | MODRES | 35.50 | CL |  | 59.4 | 27M0G7W |  |  | P |  |
| CHN | CHNF_100 | 92.20 | 123.54 | 45.78 |  |  |  | CB_TSS_CHNF |  | 43.71 |  | MODRES | 35.50 | CR |  | 60.4 | 27M0G7W |  |  | P |  |
| CLN | CLN21900 | 50.00 | 80.60 | 7.70 | 1.18 | 0.60 | 106.00 | R13TSS |  | 45.95 |  | MODRES | 35.50 | CL |  | 56.7 | 27M0G7W |  |  | P |  |
| CME | CME30000 | −13.00 | 12.70 | 6.20 | 2.54 | 1.68 | 87.00 | R13TSS |  | 38.15 |  | MODRES | 35.50 | CR |  | 58.5 | 27M0G7W |  |  | P |  |
| COD | COD__100 | −19.20 | 21.85 | −3.40 |  |  |  | CB_TSS_CODA |  | 38.36 |  | MODRES | 35.50 | CR |  | 59.7 | 27M0G7W |  |  | P |  |
| COG | COG23500 | −13.20 | 14.60 | −0.70 | 2.02 | 1.18 | 59.00 | R13TSS |  | 40.67 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| COM | COM20700 | 29.00 | 44.10 | −12.10 | 0.76 | 0.60 | 149.00 | R13TSS |  | 47.86 |  | MODRES | 35.50 | CR |  | 58.1 | 27M0G7W |  |  | P |  |
| CPV | CPV30100 | −33.50 | −24.12 | 16.09 | 0.77 | 0.63 | 94.46 | R13TSS |  | 47.56 |  | MODRES | 35.50 | CL |  | 57.2 | 27M0G7W |  |  | P |  |
| CTI | CTI23700 | −24.80 | −5.78 | 7.19 | 1.50 | 1.26 | 111.74 | R13TSS |  | 41.67 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| CVA | CVA08300 | −1.20 | 13.02 | 42.09 | 0.75 | 0.66 | 20.53 | R13TSS |  | 47.50 |  | MODRES | 35.50 | CR |  | 60.2 | 27M0G7W |  |  | P | 5, 7 |
| CVA | CVA08500 | −1.20 | 12.59 | 41.09 | 1.72 | 1.31 | 144.13 | MOD13FRTSS |  | 40.92 |  | MODRES | 35.50 | CR |  | 56.5 | 27M0G7W |  |  | P |  |
| CYP | CYP08600 | −1.20 | 33.45 | 35.12 | 0.60 | 0.60 | 0.00 | MOD13FRTSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 56.1 | 27M0G7W |  |  | P | 5, 7 |
| CZE | CZE14401 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| CZE | CZE14402 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| CZE | CZE14403 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  | 37 | P |  |
| D | D 08700 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MOD13FRTSS |  | 42.19 |  | MODRES | 35.50 | CR |  | 59.1 | 27M0G7W |  |  | P |  |
| DJI | DJI09900 | 16.80 | 42.68 | 11.68 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.5 | 27M0G7W |  |  | P |  |
| DNK | DNK__100 | −25.20 | 2.92 | 59.62 |  |  |  | CB_TSS_DNKA |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P |  |
| DNK | DNK090XR | −33.50 | 13.27 | 60.86 | 1.99 | 0.63 | 151.38 | MOD13FRTSS |  | 43.48 |  | MODRES | 35.50 | CR |  | 54.5 | 27M0G7W |  |  | P | 6 |
| DNK | DNK091XR | −33.50 | −15.16 | 63.67 | 1.56 | 0.60 | 170.63 | MOD13FRTSS |  | 44.73 |  | MODRES | 35.50 | CR |  | 58.6 | 27M0G7W |  |  | P | 6 |
| E | E____100 | −30.00 | −9.40 | 34.15 |  |  |  | CB_TSS_E__A |  | 44.79 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  | 01 | P |  |
| E | HISP33D1 | −30.00 | −4.00 | 39.00 |  |  |  |  | COP | 39.80 | 5.50 | MODRES | 35.50 | CL |  | 57.6 | 33M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISP33D2 | −30.00 | −4.00 | 39.00 |  |  |  |  | COP | 39.80 | 5.50 | MODRES | 32.50 | CL |  | 57.6 | 33M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISPA27D | −30.00 | −4.00 | 39.00 |  |  |  |  | COP | 39.80 | 5.50 | MODRES | 38.43 | CL |  | 57.6 | 27M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISPASA4 | −30.00 | −4.00 | 39.00 |  |  |  |  | COP | 39.80 | 5.50 | MODRES | 38.43 | CL |  | 57.6 | 27M0F8W | HISPASAT-1 | 01 | PE |  |
| EGY | EGY02600 | −7.00 | 29.70 | 26.80 | 2.33 | 1.72 | 136.00 | R13TSS |  | 38.42 |  | MODRES | 35.50 | CL |  | 58.1 | 27M0G7W |  | 12 | P | 8 |
| ERI | ERI09200 | 22.80 | 39.41 | 14.98 | 1.67 | 0.95 | 145.48 | R13TSS |  | 42.44 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| EST | EST06100 | 44.50 | 25.06 | 58.60 | 0.77 | 0.60 | 12.27 | R13TSS |  | 47.81 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| ETH | ETH09200 | 36.00 | 40.29 | 8.95 | 2.87 | 2.16 | 174.06 | R13TSS |  | 36.52 |  | MODRES | 35.50 | CL |  | 58.7 | 27M0G7W |  |  | P |  |
| F | F 09300 | −7.00 | 3.52 | 45.41 | 2.22 | 1.15 | 159.34 | R13TSS |  | 40.39 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  | 21 | P | 8 |
| F | F____100 | −7.00 | 50.00 | −15.65 |  |  |  | CB_TSS_F__A |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| F | NCL10000 | 140.00 | 166.00 | −21.00 | 1.14 | 0.72 | 146.00 | R13TSS |  | 45.30 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| F | OCE10100 | −160.00 | −145.00 | −16.30 | 4.34 | 3.54 | 4.00 | R13TSS |  | 32.58 |  | MODRES | 35.50 | CL |  | 58.5 | 27M0G7W |  |  | P |  |
| F | WAL10200 | 140.00 | −176.80 | −14.00 | 0.74 | 0.60 | 29.00 | R13TSS |  | 47.97 |  | MODRES | 35.50 | CR |  | 59.4 | 27M0G7W |  |  | P |  |
| FIN | FIN10300 | 22.80 | 22.50 | 64.50 | 1.38 | 0.76 | 171.00 | MOD13FRTSS |  | 44.24 |  | MODRES | 35.50 | CL |  | 54.5 | 27M0G7W |  | 52 | P |  |
| FIN | FIN10400 | 22.80 | 15.87 | 61.15 | 2.24 | 0.91 | 16.70 | MOD13FRTSS |  | 41.37 |  | MODRES | 35.50 | CL |  | 54.5 | 27M0G7W |  | 52 | P |  |
| FJI | FJI19300 | −178.00 | 179.62 | −17.87 | 1.16 | 0.92 | 155.22 | R13TSS |  | 44.16 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P | 7 |
| FSM | FSM00000 | 158.00 | 151.90 | 5.48 | 5.15 | 1.57 | 167.00 | R13TSS |  | 35.38 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P | 5 |
| G | G 02700 | −33.50 | −3.50 | 53.80 | 1.84 | 0.72 | 142.00 | R13TSS |  | 43.23 |  | MODRES | 35.50 | CR |  | 58.0 | 27M0G7W |  |  | P |  |
| GAB | GAB26000 | −13.20 | 11.80 | −0.60 | 1.43 | 1.12 | 64.00 | R13TSS |  | 42.40 |  | MODRES | 35.50 | CR |  | 58.3 | 27M0G7W |  |  | P |  |
| GEO | GEO06400 | 23.20 | 43.35 | 42.27 | 1.11 | 0.60 | 161.21 | R13TSS |  | 46.23 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| GHA | GHA10800 | −25.00 | −1.20 | 7.90 | 1.48 | 1.06 | 102.00 | R13TSS |  | 42.49 |  | MODRES | 35.50 | CR |  | 58.6 | 27M0G7W |  |  | P |  |
| GMB | GMB30200 | −37.20 | −15.10 | 13.40 | 0.79 | 0.60 | 4.00 | R13TSS |  | 47.69 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P | 5, 7 |
| GNB | GNB30400 | −30.00 | −15.00 | 12.00 | 0.90 | 0.60 | 172.00 | R13TSS |  | 47.12 |  | MODRES | 35.50 | CL |  | 58.1 | 27M0G7W |  |  | P | 5, 7 |
| GNE | GNE30300 | −18.80 | 10.30 | 1.50 | 0.68 | 0.60 | 10.00 | R13TSS |  | 48.34 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| GRC | GRC10500 | −1.20 | 24.51 | 38.08 | 1.70 | 0.95 | 152.97 | MOD13FRTSS |  | 42.40 |  | MODRES | 35.50 | CL |  | 56.3 | 27M0G7W |  |  | P | 5, 7 |
| GUI | GUI19200 | −37.00 | −11.00 | 10.20 | 1.58 | 1.04 | 147.00 | R13TSS |  | 42.29 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P | 5, 7 |
| HNG | HNG10601 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CL |  | 59.3 | 27M0G7W |  |  | P |  |
| HNG | HNG10602 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  |  | P |  |
| HNG | HNG10603 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  | 37 | P |  |
| HOL | HOL21300 | 38.20 | 5.12 | 51.96 | 1.00 | 1.00 | 24.53 | MOD13FRTSS |  | 44.45 |  | MODRES | 35.50 | CL |  | 58.5 | 27M0G7W |  |  | P |  |
| HRV | HRV14801 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| HRV | HRV14802 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| HRV | HRV14803 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  | 37 | P |  |
| I | I 08200 | 9.00 | 12.67 | 40.74 | 1.99 | 1.35 | 144.20 | R13TSS |  | 40.14 |  | MODRES | 35.50 | CR |  | 54.5 | 27M0G7W |  |  | P | 8 |
| IND | IND03700 | 68.00 | 93.00 | 25.50 | 1.46 | 1.13 | 40.00 | R13TSS |  | 42.27 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| IND | IND04700 | 68.00 | 93.30 | 11.10 | 1.92 | 0.60 | 96.00 | R13TSS |  | 43.83 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P |  |
| IND | INDA_100 | 55.80 | 76.16 | 14.72 |  |  |  | CB_TSS_INDA |  | 45.66 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| IND | INDB_100 | 55.80 | 83.43 | 24.22 |  |  |  | CB_TSS_INDB |  | 43.15 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| IND | INDD_100 | 68.00 | 74.37 | 29.16 |  |  |  | CB_TSS_INDD |  | 41.80 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  |  | P |  |
| INS | INSA_100 | 80.20 | 108.82 | −0.73 |  |  |  | CB_TSS_INSA |  | 38.88 |  | MODRES | 35.50 | CR |  | 59.2 | 27M0G7W |  |  | P |  |
| INS | INSB_100 | 104.00 | 129.75 | −3.50 |  |  |  | CB_TSS_INSB |  | 37.53 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P |  |
| IRL | IRL21100 | −37.20 | −8.25 | 53.22 | 0.72 | 0.60 | 157.56 | R13TSS |  | 48.08 |  | MODRES | 35.50 | CL |  | 59.2 | 27M0G7W |  |  | P | 5, 7 |
| IRN | IRN10900 | 34.00 | 54.20 | 32.40 | 3.82 | 1.82 | 149.00 | R13TSS |  | 36.03 |  | MODRES | 35.50 | CL |  | 57.8 | 27M0G7W |  |  | P |  |
| IRQ | IRQ25600 | 50.00 | 43.78 | 33.28 | 1.74 | 1.23 | 156.76 | R13TSS |  | 41.14 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P |  |
| ISL | ISL04900 | −33.50 | −19.00 | 64.90 | 1.00 | 0.60 | 177.00 | R13TSS |  | 46.67 |  | MODRES | 35.50 | CL |  | 60.8 | 27M0G7W |  |  | P | 5, 6 |
| ISL | ISL05000 | −33.50 | −15.35 | 63.25 | 1.58 | 0.60 | 169.00 | R13TSS |  | 44.67 |  | MODRES | 35.50 | CR |  | 57.3 | 27M0G7W |  |  | P |  |
| ISR | ISR11000 | −4.00 | 34.95 | 31.32 | 0.73 | 0.60 | 110.02 | R13TSS |  | 48.01 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| J | 000BS-3N | 109.85 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | R13TSS |  | 33.80 |  | MODRES | 35.50 | CR |  | * | 27M0F8W | BS-3N | 02 | PE |  |
| J | J 10985 | 109.85 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | R13TSS |  | 33.80 |  | MODRES | 35.50 | CR |  | * | 34M5G7W |  | 02 | P |  |
| J | J 11100 | 110.00 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | R13TSS |  | 33.80 |  | MODRES | 35.50 | CR |  | * | 34M5G7W |  | 02 | P |  |
| J | J 1110E | 110.00 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | R13TSS |  | 33.80 |  | MODRES | 35.50 | CR |  | * | 27M0F8W | BS-3M | 02 | PE |  |
| JOR | JOR22400 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MOD13FRTSS |  | 43.19 |  | MODRES | 35.50 | CL |  | 55.5 | 27M0G7W |  |  | P | 8 |
| KAZ | KAZ06600 | 56.40 | 65.73 | 46.40 | 4.58 | 1.76 | 177.45 | R13TSS |  | 35.38 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| KEN | KEN24900 | −0.80 | 37.95 | 0.92 | 2.13 | 1.34 | 98.35 | R13TSS |  | 39.90 |  | MODRES | 35.50 | CL |  | 58.7 | 27M0G7W |  |  | P |  |
| KGZ | KGZ07000 | 50.00 | 73.91 | 41.32 | 1.47 | 0.64 | 5.05 | R13TSS |  | 44.75 |  | MODRES | 35.50 | CR |  | 59.0 | 27M0G7W |  |  | P |  |
| KIR | KIR__100 | 176.00 | −170.31 | −0.56 |  |  |  | CB_TSS_KIRA |  | 42.58 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P | 5, 7 |
| KOR | KO11201D | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | R13TSS |  | 43.40 |  | MODRES | 38.43 | CL |  | ** | 27M0G7W | KOREASAT-1 | 03 | PE |  |
| KOR | KOR11200 | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | R13TSS |  | 43.80 |  | MODRES | 35.50 | CL |  | *** | 27M0G7W |  | 03 | P |  |
| KOR | KOR11201 | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | R13TSS |  | 43.40 |  | MODRES | 38.43 | CL |  | ** | 27M0F8W | KOREASAT-1 | 03 | PE |  |
| KRE | KRE28600 | 140.00 | 128.45 | 40.32 | 1.63 | 0.68 | 18.89 | R13TSS |  | 44.00 |  | MODRES | 35.50 | CL |  | 59.0 | 27M0G7W |  |  | P |  |
| KWT | KWT11300 | 11.00 | 47.48 | 29.12 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.2 | 27M0G7W |  |  | P |  |
| LAO | LAO28400 | 122.20 | 103.71 | 18.17 | 1.87 | 1.03 | 123.99 | MOD13FRTSS |  | 41.60 |  | MODRES | 35.50 | CR |  | 58.8 | 33M0G7W |  |  | P |  |
| LBN | LBN27900 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MOD13FRTSS |  | 43.19 |  | MODRES | 35.50 | CR |  | 55.5 | 27M0G7W |  |  | P |  |
| LBR | LBR24400 | −33.50 | −9.30 | 6.60 | 1.22 | 0.70 | 133.00 | R13TSS |  | 45.13 |  | MODRES | 35.50 | CR |  | 58.2 | 27M0G7W |  |  | P |  |
| LBY | LBY__100 | −24.80 | 17.62 | 26.55 |  |  |  | CB_TSS_LBYA |  | 40.30 |  | MODRES | 35.50 | CL |  | 58.0 | 27M0G7W |  |  | P |  |
| LIE | LIE25300 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MOD13FRTSS |  | 42.19 |  | MODRES | 35.50 | CL |  | 59.1 | 27M0G7W |  |  | P |  |
| LSO | LSO30500 | 4.80 | 27.80 | −29.80 | 0.66 | 0.60 | 36.00 | R13TSS |  | 48.47 |  | MODRES | 35.50 | CR |  | 59.2 | 27M0G7W |  |  | P |  |
| LTU | LTU06100 | 23.20 | 24.51 | 56.09 |  |  |  | CB_TSS_LTUA |  | 48.21 |  | MODRES | 35.50 | CL |  | 56.9 | 27M0G7W |  |  | P |  |
| LUX | LUX11400 | 28.20 | 5.21 | 49.20 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.9 | 27M0G7W |  | 09 | P |  |
| LVA | LVA06100 | 23.20 | 24.51 | 56.09 |  |  |  | CB_TSS_LVAA |  | 48.21 |  | MODRES | 35.50 | CR |  | 56.9 | 27M0G7W |  |  | P |  |
| MAU | MAU__100 | 29.00 | 58.61 | −15.88 |  |  |  | CB_TSS_MAUA |  | 41.42 |  | MODRES | 35.50 | CL |  | 59.0 | 27M0G7W |  |  | P |  |
| MCO | MCO11600 | 34.20 | 7.93 | 43.59 | 1.28 | 0.60 | 21.73 | MOD13FRTSS |  | 45.58 |  | MODRES | 35.50 | CL |  | 58.6 | 27M0G7W |  |  | P |  |
| MDA | MDA06300 | 50.00 | 28.45 | 46.99 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| MDG | MDG23600 | 29.00 | 46.60 | −18.80 | 2.72 | 1.14 | 65.00 | R13TSS |  | 39.53 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P |  |
| MHL | MHL00000 | 146.00 | 167.64 | 9.83 | 2.07 | 0.90 | 157.42 | R13TSS |  | 41.75 |  | MODRES | 35.50 | CR |  | 59.0 | 27M0G7W |  |  | P |  |
| MKD | MKD14800 | 22.80 | 21.61 | 41.56 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| MLA | MLA__100 | 91.50 | 108.05 | 4.00 |  |  |  | CB_TSS_MLAA |  | 43.00 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P |  |
| MLD | MLD30600 | 50.00 | 72.95 | 5.78 | 1.19 | 0.91 | 104.53 | R13TSS |  | 44.09 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| MLI | MLI__100 | −19.20 | −5.35 | 17.11 |  |  |  | CB_TSS_MLIB |  | 41.21 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P | 5 |
| MLT | MLT14700 | 22.80 | 14.40 | 35.90 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 56.0 | 27M0G7W |  |  | P |  |
| MNG | MNG24800 | 74.00 | 102.20 | 46.60 | 3.60 | 1.13 | 169.00 | R13TSS |  | 38.35 |  | MODRES | 35.50 | CR |  | 59.0 | 27M0G7W |  |  | P | 5, 7 |
| MOZ | MOZ30700 | −1.00 | 34.00 | −18.00 | 3.57 | 1.38 | 55.00 | R13TSS |  | 37.52 |  | MODRES | 35.50 | CL |  | 59.2 | 27M0G7W |  |  | P | 5, 7 |
| MRC | MRC20900 | −25.20 | −8.95 | 28.98 | 3.56 | 1.23 | 49.23 | R13TSS |  | 38.02 |  | MODRES | 35.50 | CR |  | 54.9 | 27M0G7W |  |  | P |  |
| MTN | MTN__100 | −36.80 | −10.52 | 19.66 |  |  |  | CB_TSS_MTNA |  | 41.91 |  | MODRES | 35.50 | CR |  | 55.5 | 27M0G7W |  |  | P | 7 |
| MWI | MWI30800 | 4.80 | 33.79 | −13.25 | 1.56 | 0.70 | 92.69 | R13TSS |  | 44.10 |  | MODRES | 35.50 | CR |  | 59.2 | 27M0G7W |  |  | P |  |
| NGR | NGR11500 | −37.20 | 7.63 | 17.01 | 2.20 | 1.80 | 102.40 | R13TSS |  | 38.48 |  | MODRES | 35.50 | CL |  | 59.5 | 27M0G7W |  |  | P | 5, 7 |
| NIG | NIG11900 | −19.20 | 7.80 | 9.40 | 2.16 | 2.02 | 45.00 | R13TSS |  | 38.05 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| NMB | NMB02500 | −18.80 | 17.50 | −21.60 | 2.66 | 1.90 | 48.00 | R13TSS |  | 37.41 |  | MODRES | 35.50 | CL |  | 59.7 | 27M0G7W |  |  | P |  |
| NOR | NOR12000 | −0.80 | 13.42 | 62.76 | 1.43 | 0.60 | 19.61 | MOD13FRTSS |  | 45.10 |  | MODRES | 35.50 | CL |  | 56.2 | 27M0G7W |  | 06 | P | 5, 7 |
| NOR | NOR12100 | −0.80 | 18.00 | 60.23 | 1.67 | 0.83 | 23.85 | R13TSS |  | 43.02 |  | MODRES | 35.50 | CL |  | 57.8 | 27M0G7W |  | 06 | P |  |
| NPL | NPL12200 | 50.00 | 83.70 | 28.30 | 1.72 | 0.60 | 163.00 | R13TSS |  | 44.31 |  | MODRES | 35.50 | CR |  | 59.6 | 27M0G7W |  |  | P |  |
| NRU | NRU30900 | 134.00 | 167.00 | −0.50 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.5 | 27M0G7W |  |  | P |  |
| NZL | NZL__100 | 158.00 | −170.68 | −19.72 |  |  |  | CB_TSS_NZLA |  | 48.88 |  | MODRES | 35.50 | CL |  | 59.6 | 27M0G7W |  |  | P |  |
| OMA | OMA12300 | 17.20 | 55.60 | 21.00 | 1.88 | 1.02 | 100.00 | R13TSS |  | 41.62 |  | MODRES | 35.50 | CR |  | 58.3 | 27M0G7W |  |  | P |  |
| PAK | PAK12700 | 38.20 | 69.60 | 29.50 | 2.30 | 2.16 | 14.00 | R13TSS |  | 37.49 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| PHL | PHL28500 | 98.00 | 121.30 | 11.10 | 3.46 | 1.76 | 99.00 | R13TSS |  | 36.60 |  | MODRES | 35.50 | CL |  | 58.7 | 27M0G7W |  |  | P |  |
| PLW | PLW00000 | 140.00 | 132.98 | 5.51 | 1.30 | 0.60 | 55.41 | R13TSS |  | 45.53 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| PNG | PNG13100 | 134.00 | 148.07 | −6.65 | 3.13 | 2.30 | 168.32 | MOD13FRTSS |  | 35.87 |  | MODRES | 35.50 | CR |  | 54.5 | 27M0G7W |  |  | P |  |
| POL | POL13200 | 50.00 | 20.07 | 51.86 | 1.20 | 0.69 | 17.76 | R13TSS |  | 45.26 |  | MODRES | 35.50 | CL |  | 59.2 | 27M0G7W |  |  | P |  |
| POR | POR__100 | −37.00 | −15.92 | 37.65 |  |  |  | CB_TSS_PORA |  | 47.17 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P | 5, 7 |
| PSE | YYY00000 | −13.20 | 34.99 | 31.86 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P | 3 |
| QAT | QAT24700 | 20.00 | 51.38 | 25.26 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 54.5 | 27M0G7W |  |  | P |  |
| ROU | ROU13600 | 50.00 | 25.12 | 45.75 | 1.17 | 0.73 | 9.52 | R13TSS |  | 45.15 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| RRW | RRW31000 | 11.00 | 30.00 | −2.10 | 0.66 | 0.60 | 42.00 | R13TSS |  | 48.47 |  | MODRES | 35.50 | CL |  | 59.8 | 27M0G7W |  |  | P |  |
| RUS | RSTREA11 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CL |  | 53.0 | 27M0F8W | RST-1 | 05 | PE |  |
| RUS | RSTREA12 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CR |  | 53.0 | 27M0F8W | RST-1 | 05 | PE |  |
| RUS | RSTRED11 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CL |  | 53.0 | 27M0G7W | RST-1 | 05 | PE |  |
| RUS | RSTRED12 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CR |  | 53.0 | 27M0G7W | RST-1 | 05 | PE |  |
| RUS | RSTRSD11 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CL |  | 53.0 | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD12 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CR |  | 53.0 | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD13 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 39.02 | CL |  | 53.0 | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD14 | 36.00 | 38.00 | 53.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 39.02 | CR |  | 53.0 | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD21 | 56.00 | 65.00 | 63.00 | 2.20 | 2.20 | 0.00 | R123FR |  | 37.70 |  | MODRES | 35.50 | CL |  | 55.0 | 27M0G7W | RST-2 | 14 | P |  |
| RUS | RSTRSD22 | 56.00 | 65.00 | 63.00 | 2.20 | 2.20 | 0.00 | R123FR |  | 37.70 |  | MODRES | 35.50 | CR |  | 55.0 | 27M0G7W | RST-2 | 14 | P |  |
| RUS | RSTRSD31 | 86.00 | 97.00 | 62.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CL |  | 55.0 | 27M0G7W | RST-3 | 33 | P |  |
| RUS | RSTRSD32 | 86.00 | 97.00 | 62.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CR |  | 55.0 | 27M0G7W | RST-3 | 33 | P |  |
| RUS | RSTRSD51 | 140.00 | 158.00 | 56.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CL |  | 55.0 | 27M0G7W | RST-5 | 35 | P |  |
| RUS | RSTRSD52 | 140.00 | 158.00 | 56.00 | 2.20 | 2.20 | 0.00 | R13TSS |  | 37.70 |  | MODRES | 35.50 | CR |  | 55.0 | 27M0G7W | RST-5 | 35 | P |  |
| RUS | RUS00401 | 110.00 | 128.73 | 54.30 | 4.25 | 2.02 | 156.81 | R13TSS |  | 35.11 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W | RUS-4 | 34 | P | 5, 7, 8 |
| RUS | RUS00402 | 110.00 | 128.73 | 54.30 | 4.25 | 2.02 | 156.81 | R13TSS |  | 35.11 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W | RUS-4 | 34 | P | 5, 7, 8 |
| S | S 13800 | 5.00 | 16.20 | 61.00 | 1.04 | 0.98 | 14.00 | R13TSS |  | 44.36 |  | MODRES | 35.50 | CL |  | 55.6 | 27M0G7W |  | 04 | P |  |
| S | S 13900 | 5.00 | 17.00 | 61.50 | 2.00 | 1.00 | 10.00 | R13TSS |  | 41.44 |  | MODRES | 35.50 | CL |  | 61.1 | 27M0G7W |  | 04 | P |  |
| SDN | SDN__100 | −7.00 | 30.24 | 13.53 |  |  |  | CB_TSS_SDNA |  | 40.26 |  | MODRES | 35.50 | CR |  | 59.4 | 27M0G7W |  |  | P |  |
| SEN | SEN22200 | −37.00 | −14.40 | 13.80 | 1.46 | 1.04 | 139.00 | R13TSS |  | 42.63 |  | MODRES | 35.50 | CL |  | 58.6 | 27M0G7W |  |  | P | 5, 7 |
| SEY | SEY00000 | 42.50 | 51.86 | −7.23 | 2.43 | 1.04 | 27.51 | R13TSS |  | 40.44 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| SLM | SLM00000 | 128.00 | 159.27 | −8.40 | 1.35 | 1.08 | 118.59 | R13TSS |  | 42.81 |  | MODRES | 35.50 | CL |  | 58.9 | 27M0G7W |  |  | P |  |
| SMO | SMO05700 | −178.00 | −171.70 | −13.87 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.6 | 27M0G7W |  |  | P | 7 |
| SMR | SMR31100 | −36.80 | 12.60 | 43.70 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 57.4 | 27M0G7W |  |  | P | 7 |
| SNG | SNG15100 | 88.00 | 103.86 | 1.42 | 0.92 | 0.72 | 175.12 | R13TSS |  | 46.25 |  | MODRES | 35.50 | CL |  | 58.5 | 27M0G7W |  |  | P |  |
| SOM | SOM31200 | 37.80 | 45.16 | 7.11 | 3.31 | 1.51 | 65.48 | R13TSS |  | 37.46 |  | MODRES | 35.50 | CR |  | 57.4 | 27M0G7W |  |  | P | 5 |
| SRB | SRB14800 | −7.00 | 20.50 | 43.98 | 0.91 | 0.60 | 145.16 | R13TSS |  | 47.07 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| SRL | SRL25900 | −33.50 | −11.80 | 8.60 | 0.78 | 0.68 | 114.00 | R13TSS |  | 47.20 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P | 6 |
| STP | STP24100 | −7.00 | 6.17 | 1.45 | 0.65 | 0.60 | 153.51 | R13TSS |  | 48.56 |  | MODRES | 35.50 | CR |  | 56.4 | 27M0G7W |  |  | P |  |
| SUI | SUI14000 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MOD13FRTSS |  | 42.19 |  | MODRES | 35.50 | CL |  | 59.1 | 27M0G7W |  |  | P |  |
| SVK | SVK14401 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CL |  | 59.3 | 27M0G7W |  |  | P |  |
| SVK | SVK14402 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  |  | P |  |
| SVK | SVK14403 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MOD13FRTSS |  | 42.64 |  | MODRES | 35.50 | CR |  | 59.3 | 27M0G7W |  | 37 | P |  |
| SVN | SVN14800 | 33.80 | 15.01 | 46.18 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| SWZ | SWZ31300 | 4.80 | 31.39 | −26.44 | 0.60 | 0.60 | 90.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.9 | 27M0G7W |  |  | P |  |
| SYR | SYR22900 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MOD13FRTSS |  | 43.19 |  | MODRES | 35.50 | CL |  | 55.5 | 27M0G7W |  | 53 | P |  |
| SYR | SYR33900 | 11.00 | 37.60 | 34.20 | 1.32 | 0.88 | 74.00 | MOD13FRTSS |  | 43.80 |  | MODRES | 35.50 | CL |  | 56.4 | 27M0G7W |  | 53 | P |  |
| TCD | TCD14300 | 17.00 | 18.36 | 15.47 | 3.23 | 2.05 | 82.89 | R13TSS |  | 36.23 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| TGO | TGO22600 | −30.00 | 0.72 | 8.61 | 1.12 | 0.60 | 109.54 | R13TSS |  | 46.19 |  | MODRES | 35.50 | CR |  | 58.5 | 27M0G7W |  |  | P | 5, 7 |
| THA | THA14200 | 98.00 | 100.75 | 12.88 | 2.80 | 1.82 | 93.77 | R13TSS |  | 37.37 |  | MODRES | 35.50 | CL |  | 58.6 | 27M0G7W |  |  | P |  |
| TJK | TJK06900 | 38.00 | 71.14 | 38.41 | 1.21 | 0.73 | 155.31 | R13TSS |  | 45.00 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  |  | P | 5 |
| TKM | TKM06800 | 50.00 | 59.24 | 38.83 | 2.26 | 1.02 | 166.64 | R13TSS |  | 40.81 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P | 5 |
| TLS | TLS00000 | 128.00 | 126.03 | −8.72 | 0.66 | 0.60 | 13.92 | R13TSS |  | 48.50 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| TON | TON21500 | 170.75 | −175.23 | −18.19 | 1.59 | 0.60 | 71.33 | R13TSS |  | 44.64 |  | MODRES | 35.50 | CR |  | 58.3 | 27M0G7W |  |  | P | 5 |
| TUN | TUN15000 | −25.20 | 9.50 | 33.50 | 1.88 | 0.72 | 135.00 | MOD13FRTSS |  | 43.13 |  | MODRES | 35.50 | CR |  | 57.3 | 27M0G7W |  | 55 | P |  |
| TUN | TUN27200 | −25.20 | 2.10 | 31.75 | 3.41 | 1.81 | 179.18 | MOD13FRTSS |  | 36.54 |  | MODRES | 35.50 | CR |  | 55.5 | 27M0G7W |  | 55 | P | 4 |
| TUR | TUR14500 | 42.00 | 34.95 | 39.09 | 3.18 | 0.99 | 0.79 | R13TSS |  | 39.47 |  | MODRES | 35.50 | CL |  | 58.8 | 27M0G7W |  | 36 | P |  |
| TUV | TUV00000 | 176.00 | 177.61 | −7.11 | 0.94 | 0.60 | 137.58 | R13TSS |  | 46.93 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| TZA | TZA22500 | 11.00 | 34.60 | −6.20 | 2.41 | 1.72 | 129.00 | R13TSS |  | 38.27 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| UAE | UAE27400 | 52.50 | 53.85 | 24.34 | 1.19 | 0.85 | 3.72 | R13TSS |  | 44.39 |  | MODRES | 35.50 | CR |  | 58.2 | 27M0G7W |  |  | P |  |
| UGA | UGA05100 | 17.00 | 32.20 | 1.04 | 1.50 | 1.02 | 68.73 | R13TSS |  | 42.62 |  | MODRES | 35.50 | CL |  | 58.2 | 27M0G7W |  |  | P |  |
| UKR | UKR06300 | 38.20 | 31.74 | 48.22 | 2.29 | 0.96 | 177.78 | R13TSS |  | 41.01 |  | MODRES | 35.50 | CR |  | 58.9 | 27M0G7W |  |  | P |  |
| USA | GUM33100 | 122.00 | 144.50 | 13.10 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 58.3 | 27M0G7W |  |  | P |  |
| USA | MRA33200 | 121.80 | 145.90 | 16.90 | 1.20 | 0.60 | 76.00 | R13TSS |  | 45.87 |  | MODRES | 35.50 | CR |  | 58.5 | 27M0G7W |  |  | P |  |
| USA | PLM33200 | 170.00 | −161.40 | 7.00 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CL |  | 57.4 | 27M0G7W |  |  | P |  |
| USA | USAA_100 | 170.00 | −170.51 | −12.72 |  |  |  | CB_TSS_USAA |  | 48.88 |  | MODRES | 35.50 | CL |  | 56.1 | 27M0G7W |  |  | P |  |
| USA | WAK33400 | 140.00 | 166.50 | 19.20 | 0.60 | 0.60 | 0.00 | R13TSS |  | 48.88 |  | MODRES | 35.50 | CR |  | 58.6 | 27M0G7W |  |  | P |  |
| UZB | UZB07100 | 33.80 | 63.80 | 41.21 | 2.56 | 0.89 | 159.91 | R13TSS |  | 40.84 |  | MODRES | 35.50 | CR |  | 58.8 | 27M0G7W |  |  | P |  |
| VTN | VTN32500 | 107.00 | 106.84 | 14.21 | 3.43 | 1.76 | 109.43 | R13TSS |  | 36.65 |  | MODRES | 35.50 | CR |  | 58.4 | 27M0G7W |  |  | P |  |
| VUT | VUT12800 | 140.00 | 168.00 | −16.40 | 1.52 | 0.68 | 87.00 | R13TSS |  | 44.30 |  | MODRES | 35.50 | CL |  | 57.8 | 27M0G7W |  |  | P |  |
| YEM | YEM__100 | 11.00 | 48.05 | 14.64 |  |  |  | CB_TSS_YEMA |  | 47.63 |  | MODRES | 35.50 | CL |  | 54.9 | 27M0G7W |  |  | P |  |
| ZMB | ZMB31400 | −0.80 | 27.50 | −13.10 | 2.38 | 1.48 | 39.00 | R13TSS |  | 38.98 |  | MODRES | 35.50 | CR |  | 58.7 | 27M0G7W |  |  | P |  |
| ZWE | ZWE13500 | −0.80 | 29.60 | −18.80 | 1.46 | 1.36 | 37.00 | R13TSS |  | 41.47 |  | MODRES | 35.50 | CR |  | 59.2 | 27M0G7W |  |  | P | 5, 7 |

COLUMN  HEADINGS  IN  TABLE  6B

Col. 1	Nominal orbital position, in degrees and hundredths of a degree from the Greenwich meridian (negative values indicate longitudes which are west of the Greenwich meridian; positive values indicate longitudes which are east of the Greenwich meridian).

Col. 2	Notifying administration symbol.

Col. 3	Beam identification (Column 2, normally, contains the symbol designating the administration or the geographical area taken from Table B1 of the Preface to the International Frequency List, followed by the symbol designating the service area).

Col. 4	Polarization (CL – circular left, CR – circular right).

Col. 5	Channel number/Indication of minimum equivalent protection margin (EPM) for a given assignment derived from the set of values for all test points belonging to the given beam (dB).

TABLE  6B

Minimum equivalent protection margin of assignments in the Regions 1 and 3 Plan (sorted by orbital position)

| 1 | 2 | 3 | 4 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital Position | Admin. symbol | Beam Identifica- tion | Polar- ization type | Channel number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
|  |  |  |  | Minimum EPM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –178.00 | FJI | FJI19300 | CR | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  | 6.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –178.00 | SMO | SMO05700 | CR | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –160.00 | F | OCE10100 | CL |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.20 | GMB | GMB30200 | CL | 4.6 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.20 | IRL | IRL21100 | CL | 2.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.20 | NGR | NGR11500 | CL |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.00 | AND | AND34100 | CL |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.00 | GUI | GUI19200 | CR |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.00 | POR | POR__100 | CR | 2.4 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –37.00 | SEN | SEN22200 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.3 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  |
| –36.80 | MTN | MTN__100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 3.9 |
| –36.80 | SMR | SMR31100 | CR | 4.4 |  | 3.2 |  | 3.2 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –33.50 | CPV | CPV30100 | CL |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –33.50 | DNK | DNK090XR | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 |  |  |  | 0.9 |  |  |  |  |  |  |  |
| –33.50 | DNK | DNK091XR | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9 |  |  |  | –0.9 |  |  |  |  |  |
| –33.50 | G | G 02700 | CR |  | –0.1 |  | 0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | 0.2 |  | –0.1 |  | 0.1 |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –33.50 | ISL | ISL04900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.8 |  | 7.2 |  | 7.2 |  | 10.1 |  | 14.8 |  | 3.6 |  | 14.8 |  | 3.6 |  | 21.7 |  | 21.7 |  |
| –33.50 | ISL | ISL05000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  | 0.1 |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –33.50 | LBR | LBR24400 | CR | 3.3 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –33.50 | SRL | SRL25900 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 0.9 |  | 1.0 |  | 0.9 |  | 1.0 |  | 1.0 |  |
| –30.00 | BFA | BFA10700 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 2.4 |
| –30.00 | E | E____100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.3 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.6 |  | 5.9 |  | 5.6 |  | 5.9 |  |
| –30.00 | E | HISP33D1 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4 |  |  |  | –0.4 |  |  |  | –0.4 |  |  |  | –0.4 |  |  |  | –0.4 |  |
| –30.00 | E | HISP33D2 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –1.0 |  |  |  | –1.0 |  |  |  | –1.0 |  |  |  | –1.0 |  |  |  | –1.0 |  |
| –30.00 | E | HISPA27D | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |
| –30.00 | E | HISPASA4 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |  |  | 1.3 |  |
| –30.00 | GNB | GNB30400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.7 |
| –30.00 | TGO | TGO22600 | CR | 7.9 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –25.20 | DNK | DNK__100 | CL | –0.9 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  | –1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –25.20 | MRC | MRC20900 | CR | 0.1 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.3 |  | –0.3 |  | –0.4 |  | –0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –25.20 | TUN | TUN15000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –1.0 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  |
| –25.20 | TUN | TUN27200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9 |  |  |  | –0.9 |  |  |  |
| –25.00 | GHA | GHA10800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.1 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 4.7 |  | 5.2 |  | 4.7 |  | 5.2 |  |
| –24.80 | AGL | AGL29500 | CL | 4.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –24.80 | ALG | ALG__100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | 2.2 |
| –24.80 | CTI | CTI23700 | CL | 3.4 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –24.80 | LBY | LBY__100 | CL |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –19.20 | BEN | BEN23300 | CL | 3.1 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –19.20 | COD | COD__100 | CR |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 5.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –19.20 | MLI | MLI__100 | CR | 5.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –19.20 | NIG | NIG11900 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 4.1 |
| –18.80 | AUT | AUT01600 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 |  | –0.1 |  | 0.0 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  |
| –18.80 | D | D 08700 | CR | 1.1 |  | –0.2 |  | –0.1 |  | –0.2 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –18.80 | GNE | GNE30300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  |
| –18.80 | LIE | LIE25300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 2.8 |
| –18.80 | NMB | NMB02500 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  |
| –18.80 | SUI | SUI14000 | CL |  | 0.2 |  | 0.7 |  | 0.2 |  | 0.7 |  | 0.3 |  | 0.7 |  | 0.3 |  | 0.7 |  | 0.2 |  | 0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –13.20 | CAF | CAF25800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.5 |
| –13.20 | COG | COG23500 | CL | 0.8 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –13.20 | GAB | GAB26000 | CR | 0.5 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –13.20 | PSE | YYY00000 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.3 |  | 4.8 |  | 5.1 |  | 4.9 |  | 5.1 |  | 4.9 |  | 5.1 |  | 4.9 |  | 5.1 |  | 4.9 |  |
| –13.00 | CME | CME30000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.5 |  | 0.4 |  | 0.5 |  | 0.4 |  | 0.5 |  | 0.4 |  | 0.5 |
| –12.80 | CZE | CZE14401 | CL | 1.9 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.6 |  |  |  |  |  |  |  | –0.9 |  |  |  |  |  |  |  | –0.8 |  |  |  |  |  |  |  |
| –12.80 | CZE | CZE14402 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9 |  |  |  |  |  |  |  | –0.9 |  |  |  |  |
| –12.80 | CZE | CZE14403 | CR |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9* |  | –0.9* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –12.80 | HNG | HNG10601 | CL |  |  | 1.1 |  |  |  |  |  |  |  | 1.2 |  |  |  |  |  |  |  | 1.1 |  |  |  |  |  |  |  | –0.3 |  |  |  |  |  |  |  | –0.3 |  |  |  |  |  |
| –12.80 | HNG | HNG10602 | CR |  |  |  |  |  | 0.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4 |  |  |  |  |  |  |  | –0.4 |  |  |
| –12.80 | HNG | HNG10603 | CR |  | 0.6* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4* |  | –0.4* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –12.80 | HRV | HRV14801 | CL |  |  |  |  | 0.6 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.3 |  |  |  |  |  |  |  | –0.8 |  |  |  |  |  |  |  | –0.8 |  |  |  |
| –12.80 | HRV | HRV14802 | CR |  |  |  |  |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9 |  |  |  |  |  |  |  | –0.2 |
| –12.80 | HRV | HRV14803 | CR |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.9* |  | –0.9* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –12.80 | SVK | SVK14401 | CL |  |  |  |  |  |  | 1.1 |  |  |  |  |  |  |  | 1.2 |  |  |  |  |  |  |  | –0.4 |  |  |  |  |  |  |  | –0.3 |  |  |  |  |  |  |  | –0.3 |  |
| –12.80 | SVK | SVK14402 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.6 |  |  |  |  |  |  |  | –0.4 |  |  |  |  |  |  |  | –0.4 |  |  |  |  |  |  |
| –12.80 | SVK | SVK14403 | CR |  | 0.6* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4* |  | –0.4* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –7.00 | EGY | EGY02600 | CL |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 10.9 |  | 5.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –7.00 | F | F 09300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.4 |  | 4.3 |  | 5.6 |  | 4.3 |  | 5.6 |  | 4.8 |  | 5.6 |  | 4.8 |  | 5.6 |  | 5.5 |
| –7.00 | F | F____100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.0 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  |
| –7.00 | SRB | SRB14800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –1.2 |  | –1.1 |  | –0.6 |  | –0.9 |  | –0.6 |  | –0.9 |  | –0.6 |  | –0.9 |  | –0.6 |  | –0.5 |
| –7.00 | SDN | SDN__100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.4 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  | 8.0 |  |
| * This assignment shall only be used by the Administrations of Croatia, Czech Republic, Hungary and Slovakia on the basis of equal access subject to mutual agreement between them. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –7.00 | STP | STP24100 | CR |  | 6.1 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 4.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –4.00 | ISR | ISR11000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1 |  | 3.0 |  | 3.1 |  | 3.0 |  | 3.1 |  | 3.0 |  | 3.1 |  | 3.0 |  | 3.1 |  | 5.7 |
| –1.20 | BUL | BUL02000 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.0 |  | –0.4 |  | 1.6 |  | –0.4 |  | 1.6 |  | –0.4 |  | 1.6 |  | –0.4 |  | 1.6 |  | –0.3 |
| –1.20 | CVA | CVA08300 | CR | 2.2 |  | 0.5 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –1.20 | CVA | CVA08500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –1.20 | CYP | CYP08600 | CR | 1.4 |  | 0.0 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 2.2 |  | 2.2 |  | 2.2 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –1.20 | GRC | GRC10500 | CL |  | –0.4 |  | 0.6 |  | –0.1 |  | 0.8 |  | –0.1 |  | 0.9 |  | –0.1 |  | 0.9 |  | –0.3 |  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –1.00 | MOZ | MOZ30700 | CL |  | 2.8 |  | 3.8 |  | 2.8 |  | 3.8 |  | 2.8 |  | 3.8 |  | 2.8 |  | 3.8 |  | 2.8 |  | 3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –0.80 | BOT | BOT29700 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | 0.9 |  | 1.5 |  | 0.9 |  | 1.5 |  | 0.9 |  | 1.5 |  | 0.9 |  | 1.5 |  | 2.2 |
| –0.80 | KEN | KEN24900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.8 |  | 2.8 |  | 3.9 |  | 2.8 |  | 3.9 |  | 2.8 |  | 3.9 |  | 2.8 |  | 3.9 |  | 3.3 |
| –0.80 | NOR | NOR12000 | CL | 4.4 |  | 2.0 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.6 |  | –0.6 |  | –0.6 |  | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –0.80 | NOR | NOR12100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.6 |  |  |  | –0.6 |  |  |  |  |  |  |  |  |  |  |  |  |
| –0.80 | ZMB | ZMB31400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.8 |  | 3.2 |  | 4.3 |  | 3.2 |  | 4.3 |  | 3.2 |  | 4.3 |  | 3.2 |  | 4.3 |  | 3.2 |  |
| –0.80 | ZWE | ZWE13500 | CR | 5.5 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.80 | AFS | AFS02100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.5 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  |
| 4.80 | LSO | LSO30500 | CR | 3.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 2.9 |  | 3.1 |  | 3.1 |  | 3.1 |  | 2.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.80 | MWI | MWI30800 | CR |  | 3.2 |  | 3.8 |  | 3.3 |  | 3.9 |  | 3.3 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.8 |  | 3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.80 | SWZ | SWZ31300 | CL | 4.5 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.00 | S | S 13800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.7 |  | –0.4 |  | 2.3 |  | –0.4 |  | 2.3 |  | 0.6 |  | 4.5 |  | 0.6 |  | 4.5 |  | 0.6 |  |
| 5.00 | S | S 13900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.7 |  |  |  |  |  |  |  |  |  |  |
| 9.00 | I | I 08200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –3.6 |  | –3.8 |  | –3.6 |  | –3.8 |  | –3.8 |  | –3.8 |  | –3.8 |  | –3.8 |  | –3.8 |  | –2.6 |
| 11.00 | BDI | BDI27000 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 5.6 |
| 11.00 | JOR | JOR22400 | CL |  | –1.0 |  | –0.8 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.7 |  | –0.8 |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | KWT | KWT11300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 5.4 |  | 6.3 |
| 11.00 | LBN | LBN27900 | CR | 0.9 |  | –0.6 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | RRW | RRW31000 | CL |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | SYR | SYR22900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –0.5 |
| 11.00 | SYR | SYR33900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.4** |
| 11.00 | TZA | TZA22500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  |
| 11.00 | YEM | YEM__100 | CL | 2.1 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  | –0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.80 | DJI | DJI09900 | CL |  | 8.2 |  | 8.2 |  | 8.1 |  | 8.1 |  | 8.1 |  | 8.1 |  | 8.1 |  | 8.1 |  | 8.1 |  | 6.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.00 | ARS | ARS__100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | 0.0 |
| 17.00 | ARS | ARS34000 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 |
| 17.00 | TCD | TCD14300 | CR | 9.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  | 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.00 | UGA | UGA05100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.6 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  |
| ** See Note 1 of § 11.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.20 | OMA | OMA12300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 0.9 |
| 20.00 | QAT | QAT24700 | CL |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.8 |  | 5.8 |  | 6.2** |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22.80 | ARM | ARM06400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.4 |  | 0.5 |  | 2.7 |
| 22.80 | ERI | ERI09200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.3 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  |
| 22.80 | FIN | FIN10300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.5 |  | 1.2 |  | 1.9 |  | 2.0 |  | 1.8 |  | 2.1 |  | 1.7 |  | 2.2 |  | 1.6 |  | 4.9 |
| 22.80 | FIN | FIN10400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –0.4 |  |  |  | 1.1 |
| 22.80 | MKD | MKD14800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.6 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.2 |  | 3.3 |  | 3.3 |  |
| 22.80 | MLT | MLT14700 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  | 1.8 |  | 1.7 |  | 1.8 |  | 1.7 |  | 1.8 |  | 1.7 |  | 1.7 |  | 1.8 |  | 3.4 |
| 23.20 | AZE | AZE06400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.6 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  |
| 23.20 | GEO | GEO06400 | CR | 5.3 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23.20 | LTU | LTU06100 | CL | 1.4 |  | 0.1 |  | 0.0 |  | 0.0 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.2 |  | –0.2 |  | –0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23.20 | LVA | LVA06100 | CR |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.9 |  | –0.9 |  | –0.9 |  | –0.9 |  | –0.9 |  | –0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28.20 | LUX | LUX11400 | CL | 3.1 |  | 2.9 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  | 2.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29.00 | COM | COM20700 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.2 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.5 |  | 9.6 |  |
| 29.00 | MAU | MAU__100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.1 |  | 11.1 |  | 12.7 |
| 29.00 | MDG | MDG23600 | CL | 14.0 |  | 13.6 |  | 13.5 |  | 13.4 |  | 13.4 |  | 13.3 |  | 13.3 |  | 13.2 |  | 13.2 |  | 13.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33.80 | SVN | SVN14800 | CR |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.4 |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33.80 | UZB | UZB07100 | CR | 3.3 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34.00 | BHR | BHR25500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 |  | 0.9 |  | 8.7 |  | 8.8 |  | 9.2 |  | 8.8 |  | 9.2 |  | 8.8 |  | 9.2 |  | 8.8 |  |
| 34.00 | IRN | IRN10900 | CL | 1.2 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 8.8 |  | 8.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34.20 | MCO | MCO11600 | CL | 0.7 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36.00 | ETH | ETH09200 | CL |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 11.8 |  | 12.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36.00 | RUS | RSTREA11 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –3.6 |  |  |  | –3.6 |  |  |  | –3.6 |  |  |  | –3.6 |  |
| 36.00 | RUS | RSTREA12 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –4.3 |  |  |  | –4.3 |  |  |  | –4.3 |  |  |  | –4.2 |
| 36.00 | RUS | RSTRED11 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –3.6 |  |  |  | –3.6 |  |  |  | –3.6 |  |  |  | –3.6 |  |
| 36.00 | RUS | RSTRED12 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | –4.3 |  |  |  | –4.3 |  |  |  | –4.3 |  |  |  | –4.2 |
| 36.00 | RUS | RSTRSD11 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.5 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  |
| 36.00 | RUS | RSTRSD12 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.4 |
| 36.00 | RUS | RSTRSD13 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.9 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  |
| 36.00 | RUS | RSTRSD14 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.6 |
| 37.80 | BLR | BLR06200 | CL | 3.8 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37.80 | SOM | SOM31200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.3 |  | 3.3 |  | 11.7 |  | 10.2 |  | 11.7 |  | 10.2 |  | 11.7 |  | 10.2 |  | 11.7 |  | 10.4 |
| 38.00 | TJK | TJK06900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.6 |  | 1.2 |  | 7.9 |  | 7.8 |  | 7.9 |  | 7.8 |  | 7.9 |  | 7.8 |  | 7.9 |  | 7.9 |
| 38.20 | BEL | BEL01800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.5 |  | 2.2 |  | 2.0 |  | 1.7 |  | 2.0 |  | 1.7 |  | 2.0 |  | 1.7 |  | 2.0 |  | 3.4 |
| 38.20 | HOL | HOL21300 | CL |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 2.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38.20 | PAK | PAK12700 | CR |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 4.6 |  | 2.0 |  | 2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ** See Note 1 of § 11.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38.20 | UKR | UKR06300 | CR |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42.00 | TUR | TUR14500 | CL |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42.50 | SEY | SEY00000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.3 |  | 13.0 |  | 13.1 |  | 13.1 |  | 13.1 |  | 13.1 |  | 13.1 |  | 13.1 |  | 13.1 |  | 14.4 |
| 44.50 | EST | EST06100 | CR | 6.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | AFG | AFG__100 | CL | –0.3 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | –0.5 |  | 3.0 |  | 2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | CLN | CLN21900 | CL | 0.8 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 1.5 |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | IRQ | IRQ25600 | CL | 4.2 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6 |  | 3.6** |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | KGZ | KGZ07000 | CR |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | MDA | MDA06300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 |  | 0.7 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |
| 50.00 | MLD | MLD30600 | CR | 6.3 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.8 |  | 4.7 |  | 4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | NPL | NPL12200 | CR |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.2 |  | 2.2 |  | 3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | POL | POL13200 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.4 |  | 2.7 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.1 |  | 3.2 |
| 50.00 | ROU | ROU13600 | CR | 4.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | TKM | TKM06800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.0 |  | 2.4 |  | 8.5 |  | 8.5 |  | 8.5 |  | 8.5 |  | 8.5 |  | 8.5 |  | 8.5 |  | 8.7 |
| 52.50 | UAE | UAE27400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.2 |  | 3.2 |  | 14.3 |  | 15.1 |  | 15.1 |  | 15.1 |  | 15.1 |  | 15.1 |  | 15.1 |  | 15.1** |  |
| 55.80 | IND | INDA_100 | CR |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 3.2 |  | 3.9 |  | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55.80 | IND | INDB_100 | CL | 2.7 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 3.2 |  | 3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56.00 | BIH | BIH14800 | CL | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56.00 | RUS | RSTRSD21 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.3 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  |
| 56.00 | RUS | RSTRSD22 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.6 |
| 56.40 | KAZ | KAZ06600 | CR | 3.6 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62.00 | ALB | ALB29600 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.6 |  | 10.9 |  | 33.5 |  | 33.5 |  | 33.5 |  | 33.5 |  | 33.5 |  | 33.5 |  | 33.5 |  | 34.4 |
| 62.00 | CHN | CHN15500 | CL | 2.7 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62.00 | CHN | CHNA_100 | CR |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 2.1 |  | 2.3 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68.00 | IND | IND03700 | CL |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 7.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68.00 | IND | IND04700 | CR | 5.4 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  | 5.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68.00 | IND | INDD_100 | CR | 6.0 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | BGD | BGD22000 | CR | 12.1 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 4.1 |  | 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | BRU | BRU33000 | CR |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 4.8 |  | 4.5 |  | 6.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | MNG | MNG24800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.6 |  | 6.6 |  | 26.4 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  |
| 80.20 | INS | INSA_100 | CR | 12.8 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | BTN | BTN03100 | CR | 11.8 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | CBG | CBG29900 | CR |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 11.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | RUS | RSTRSD31 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.5 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  |
| 86.00 | RUS | RSTRSD32 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |
| 88.00 | SNG | SNG15100 | CL | 2.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ** See Note 1 of § 11.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 91.50 | MLA | MLA__100 | CR |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 3.5** |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 92.20 | CHN | CHNE_100 | CL | 4.4 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 92.20 | CHN | CHNF_100 | CR |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 4.4 |  | 6.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98.00 | PHL | PHL28500 | CL |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.3 |  | 4.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98.00 | THA | THA14200 | CL | 1.8 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  | –0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104.00 | BRM | BRM29800 | CL | 2.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  | –0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104.00 | INS | INSB_100 | CL |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.1 |  | 4.9** |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 107.00 | VTN | VTN32500 | CR |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 109.85 | J | 000BS–3N | CR | 6.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.2 |  | 4.1 |  | 6.5 |  | 12.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 109.85 | J | J 10985 | CR | 6.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.0 |  | 6.6 |  | 13.9 |  | 13.9 |  | 13.9 |  | 13.9 |  | 13.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | J | J 11100 | CR | 5.5 |  | 3.5 |  | 3.5 |  | 3.4 |  | 3.4 |  | 3.3 |  | 5.9 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | J | J 1110E | CR | 5.4 |  | 3.4 |  | 3.4 |  | 3.4 |  | 3.3 |  | 3.1 |  | 5.6 |  | 12.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | RUS | RUS00401 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.2 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  |
| 110.00 | RUS | RUS00402 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 99.9 |  | 99.9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 116.00 | KOR | KO11201D | CL |  | 5.6 |  | 5.7 |  | 5.6 |  | 5.2 |  | 5.0 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 116.00 | KOR | KOR11200 | CL |  | –0.8 |  | –0.8 |  | –0.8 |  | –0.8 |  | –1.2 |  | –1.2 |  | 4.3 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 4.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 116.00 | KOR | KOR11201 | CL |  | 5.6 |  | 5.7 |  | 5.6 |  | 5.2 |  | 5.0 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 121.80 | USA | MRA33200 | CR | 5.3 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.00 | CHN | CHN19000 | CR | 2.9 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.00 | CHN | CHN20000 | CL |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.3 |  | 1.3 |  | 1.4 |  | 0.9 |  | 0.7 |  | 0.7 |  | 0.7 |  | 0.7 |  | 2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.00 | USA | GUM33100 | CL |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 8.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.20 | LAO | LAO28400 | CR |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –1.8 |  | –0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128.00 | SLM | SLM00000 | CL | 13.3 |  | 11.9 |  | 11.9 |  | 11.9 |  | 11.9 |  | 11.9 |  | 11.9 |  | 11.7 |  | 11.7 |  | 11.7 |  | 11.7 |  | 11.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128.00 | TLS | TLS00000 | CR |  | 9.4 |  | 9.3 |  | 9.3 |  | 9.3 |  | 9.3 |  | 9.3 |  | 9.3 |  | 9.2 |  | 9.2 |  | 9.2 |  | 9.2 |  | 10.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | CHN | CHN15800 | CR |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | –1.0 |  | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | CHN | CHNC_100 | CL | 2.1 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  | –0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | NRU | NRU30900 | CL | 8.6 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | PNG | PNG13100 | CR |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | F | NCL10000 | CR |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | F | WAL10200 | CR |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.8 |  | 7.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | KRE | KRE28600 | CL | 15.5 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | PLW | PLW00000 | CR |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | RUS | RSTRSD51 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15.8 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  |
| 140.00 | RUS | RSTRSD52 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |  | 99.9 |
| 140.00 | USA | WAK33400 | CR | 16.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | VUT | VUT12800 | CL | 7.1 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 146.00 | MHL | MHL00000 | CR |  | 25.2 |  | 25.1 |  | 25.2 |  | 25.1 |  | 25.2 |  | 25.4 |  | 25.5 |  | 25.4 |  | 25.5 |  | 25.4 |  | 25.5 |  | 26.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00400 | CR |  |  | 5.7 |  |  |  | 5.7 |  |  |  | 5.7 |  |  |  | 5.7 |  |  |  | 5.7 |  |  |  | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS0040A | CR |  |  | 16.9 |  |  |  | 16.9 |  |  |  | 16.9 |  |  |  | 16.9 |  |  |  | 16.9 |  |  |  | 16.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS0040B | CR |  |  | 16.3 |  |  |  | 16.3 |  |  |  | 16.3 |  |  |  | 16.3 |  |  |  | 16.3 |  |  |  | 16.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ** See Note 1 of § 11.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS0040C | CR |  |  | 17.4 |  |  |  | 17.4 |  |  |  | 17.4 |  |  |  | 17.4 |  |  |  | 17.4 |  |  |  | 17.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00500 | CL |  |  |  | 5.1 |  |  |  | 5.1 |  |  |  | 9.1 |  |  |  | 9.1 |  |  |  | 9.1 |  |  |  | 9.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00600 | CL |  | 4.3 |  |  |  | 4.3 |  |  |  | 4.3 |  |  |  | 8.7 |  |  |  | 8.7 |  |  |  | 8.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA_100 | CR | 9.2 |  |  |  | 7.5 |  |  |  | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 158.00 | FSM | FSM00000 | CR | 14.9 |  | 23.5 |  | 14.6 |  | 23.5 |  | 14.6 |  | 23.5 |  | 23.8 |  | 24.9 |  | 25.3 |  | 24.9 |  | 25.3 |  | 24.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 158.00 | NZL | NZL__100 | CL |  | 12.1 |  | 8.5 |  | 12.1 |  | 8.5 |  | 12.1 |  | 8.6 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 15.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00700 | CR |  |  | 4.8 |  |  |  | 4.8 |  |  |  | 4.8 |  |  |  | 9.4 |  |  |  | 9.4 |  |  |  | 9.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS0070A | CR |  |  | 7.7 |  |  |  | 7.7 |  |  |  | 7.7 |  |  |  | 15.8 |  |  |  | 15.8 |  |  |  | 15.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00800 | CL |  | 5.4 |  |  |  | 5.4 |  |  |  | 5.4 |  |  |  | 5.4 |  |  |  | 5.4 |  |  |  | 5.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00900 | CR | 7.8 |  |  |  | 3.6 |  |  |  | 3.6 |  |  |  | 3.6 |  |  |  | 7.2 |  |  |  | 7.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS0090A | CR | 13.0 |  |  |  | 7.1 |  |  |  | 7.1 |  |  |  | 7.1 |  |  |  | 13.0 |  |  |  | 13.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS0090B | CR | 16.2 |  |  |  | 7.7 |  |  |  | 7.7 |  |  |  | 7.7 |  |  |  | 15.7 |  |  |  | 15.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB_100 | CL |  |  |  | 4.6 |  |  |  | 4.6 |  |  |  | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170.00 | USA | PLM33200 | CL |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 6.5 |  | 9.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170.00 | USA | USAA_100 | CL | 9.9 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  | 7.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170.75 | TON | TON21500 | CR |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.6 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 11.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 176.00 | KIR | KIR__100 | CL | 13.4 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  | 10.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 176.00 | TUV | TUV00000 | CR |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 9.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

11.3	(SUP - WRC-03)

ARTICLE  12

Relationship to Resolution 507 (REV.WRC-03)*

12.1	The provisions and associated Plans for the broadcasting-satellite service in Regions 1 and 3 and in Region 2, of this Appendix, shall be regarded as including a world agreement and associated Plans for Regions 1, 2 and 3 in accordance with resolves 1 of Resolution 507 (Rev.WRC-03)*, which requires the stations in the broadcasting-satellite service to be established and operated in accordance with such agreements and associated Plans.

ARTICLE  13

Interference

13.1	The Member States of the Union shall endeavour to agree on the action required to reduce harmful interference which might be caused by the application of these provisions and the associated Plans.

ARTICLE  14

Period of validity of the provisions and associated Plans

14.1	For Regions 1 and 3, the provisions and associated Plan have been prepared in order to meet the requirements of the broadcasting-satellite service in the bands concerned for a period of at least fifteen years from 1 January 1979.

14.2	For Region 2, the provisions and associated Plan have been prepared in order to meet the requirements of the broadcasting-satellite service in the bands concerned for a period extending until at least 1 January 1994.

14.3	In any event, the provisions and associated Plans shall remain in force until their revision by a competent radiocommunication conference convened in accordance with the relevant provisions of the Constitution and Convention in force.

ANNEX  1     (REV.WRC-15)

Limits for determining whether a service of an administration is affected
by a proposed modification to the Region 2 Plan or by a proposed
new or modified assignment in the Regions 1 and 3 List
or when it is necessary under this Appendix to seek
the agreement of any other administration25

(See Article 4)

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of −103.6 dB(W/(m2 · 27 MHz)).

With respect to § 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 is considered by the Bureau as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.

However, an administration in Region 1 or 3 is considered as not being affected if either of the following two conditions is met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values:26  (WRC-15)

−147    dB(W/(m2  27 MHz))	for  0°      ≤  < 0.23°

−135.7 + 17.74 log     dB(W/(m2  27 MHz))	for  0.23° ≤  < °

−136.7 + 1.66 2    dB(W/(m2  27 MHz))	for  2.0°   ≤  < °

−129.2 + 25 log     dB(W/(m2  27 MHz))	for  3.59° ≤  < °

where  is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin27 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.     (WRC-03)

# 2	Limits to the change in the overall equivalent protection margin for frequency assignments in conformity with the Region 2 Plan

With respect to § 4.2.3 c) of Article 4, an administration in Region 2 is considered as being affected if the overall equivalent protection margin28 corresponding to a test point of its entry in the Region 2 Plan, including the cumulative effect of any previous modification to that Plan or any previous agreement, falls more than 0.25 dB below 0 dB, or, if already negative, more than 0.25 dB below the value resulting from:

–	the Region 2 Plan as established by the 1983 Conference; or

–	a modification of the assignment in accordance with this Appendix; or

–	a new entry in the Region 2 Plan under Article 4; or

–	any agreement reached in accordance with this Appendix.     (WRC-03)

# 3	Limits to the change in the power flux-density to protect the broadcasting-satellite service in Regions 1 and 2 in the band 12.2-12.5 GHz and in Region 3 in the band 12.5-12.7 GHz

With respect to § 4.1.1 c) of Article 4, an administration in Region 2 is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in exceeding the following power flux-density values, at any test point in the service area of its overlapping frequency assignments:

−147    dB(W/(m2  27 MHz))	for  0°        ≤  < 0.23°

−135.7 + 17.74 log     dB(W/(m2  27 MHz))	for  0.23°   ≤  < 1.8°

−134.0 + 0.89 2    dB(W/(m2  27 MHz))	for  1.8°     ≤  < 5.0°

−129.2 + 25 log     dB(W/(m2  27 MHz))	for  5.0°     ≤  < 10.57°

−103.6    dB(W/(m2  27 MHz))	for  10.57° ≤

where  is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

With respect to § 4.2.3 a), 4.2.3 b) or 4.2.3 f) of Article 4, as appropriate, an administration in Region 1 or 3 is considered as being affected if the proposed modification to the Region 2 Plan would result in exceeding the following power flux-density values, at any test point in the service area of its overlapping frequency assignments:

−147    dB(W/(m2 · 27 MHz))	for  0°        ≤  < 0.23°

−135.7 + 17.74 log     dB(W/(m2 · 27 MHz))	for  0.23°   ≤  < 2.0°

−136.7 + 1.66 2    dB(W/(m2 · 27 MHz))	for  2.0°     ≤  < 3.59°

−129.2 + 25 log     dB(W/(m2 · 27 MHz))	for  3.59°   ≤  < 10.57°

−103.6    dB(W/(m2 · 27 MHz))	for  10.57°

where  is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.     (WRC-03)

# 4	Limits to the power flux-density to protect the terrestrial services of other administrations29, 30, 31

With respect to § 4.1.1 d) of Article 4, an administration in Region 1, 2 or 3 is considered as being affected if the consequence of the proposed modified assignment in the Regions 1 and 3 List is to increase the power flux-density arriving on any part of the territory of that administration by more than 0.25 dB over that resulting from that frequency assignment in the Plan or List for Regions 1 and 3 as established by WRC-2000. The same administration is considered as not being affected if the value of the power flux-density anywhere in its territory does not exceed the limits expressed below.

With respect to § 4.2.3 d) of Article 4, an administration in Region 1, 2 or 3 is considered as being affected if the consequence of the proposed modification to an existing assignment in the Region 2 Plan is to increase the power flux-density arriving on any part of the territory of that administration by more than 0.25 dB over that resulting from that frequency assignment in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference. The same administration is considered as not being affected if the value of the power flux-density anywhere in its territory does not exceed the limits expressed below.

With respect to § 4.1.1 d) or § 4.2.3 d) of Article 4, an administration in Region 1, 2 or 3 is considered as being affected if the proposed new assignment in the Regions 1 and 3 List, or if the proposed new frequency assignment in the Region 2 Plan, would result in exceeding a power flux-density, for any angle of arrival, at any point on its territory, of:

−148     dB(W/(m2  4 kHz))	for              5°

−148  +  0.5 ( – 5)     dB(W/(m2  4 kHz)	for  5°   <   25°

−138     dB(W/(m2  4 kHz))	for  25° <   90°

where θrepresents the angle of arrival.     (WRC-03)

# 5	(Not used.)

# 6	Limits to the change in the power flux-density of assignments in the Regions 1 and 3 Plan or List to protect the fixed-satellite service (space-to-Earth) in the frequency band 11.7-12.2 GHz32 in Region 2 or in the frequency band 12.2-12.5 GHz in Region 3, and of assignments in the Region 2 Plan to protect the fixed-satellite service (space-to-Earth) in the frequency band 12.5-12.7 GHz in Region 1 and in the frequency band 12.2-12.7 GHz in Region 3

With respect to § 4.1.1 e) of Article 4, an administration is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 2 or Region 3 of 0.25 dB or more above that resulting from the frequency assignments in the Plan or List for Regions 1 and 3 as established by WRC-2000.

With respect to § 4.2.3 e), an administration is considered as being affected if the proposed modification to the Region 2 Plan would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1 or 3 of 0.25 dB or more above that resulting from the frequency assignments in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference.

With respect to § 4.1.1 e) or 4.2.3 e) of Article 4, with the exception of cases covered by Note 1 below, an administration is considered as not being affected if the proposed new or modified assignment in the Regions 1 and 3 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1, 2 or 3 of less than:

−186.5    dB(W/(m2 · 40 kHz)) 	for  0°        ≤  < 0.054°

−164.0 + 17.74 log     dB(W/(m2 · 40 kHz))	for  0.054° ≤  < 2.0°

−165.0 + 1.66 2    dB(W/(m2 · 40 kHz))	for  2.0°     ≤  < 3.59°

−157.5 + 25 log     dB(W/(m2 · 40 kHz))	for  3.59°   ≤  < 10.57°

−131.9    dB(W/(m2 · 40 kHz))	for  10.57° ≤

where θ is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

NOTE 1 – With respect to § 4.1.1 e) of Article 4, an administration in Region 3 is considered as not being affected if the proposed new or modified assignment in the Regions 1 and 3 List in the orbital arc 105° E-129° E gives a power flux-density anywhere over any portion of the territory of the notifying administration within the service area of its overlapping frequency assignments in the fixed-satellite service in the orbital arc 110° E-124° E of less than:

−186.5    dB(W/(m2 · 40 kHz)) 	for  0°        ≤ θ < 0.054°

−164.0 + 17.74 log θ    dB(W/(m2 · 40 kHz))	for  0.054° ≤ θ < 1.8°

−162.3 + 0.89 θ2    dB(W/(m2 · 40 kHz))	for  1.8°     ≤ θ < 5.0°

−157.5 + 25 log θ    dB(W/(m2 · 40 kHz))	for  5.0°     ≤ θ < 10.57°

−131.9    dB(W/(m2 · 40 kHz))	for  10.57° ≤ θ

where θ is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

The above set of formulas is only applied to networks:

–	for which Appendix 4 information for coordination had been received by the Bureau prior to 30 March 2002; and

–	which had been brought into use prior to 30 March 2002 and for which the date of bringing into use had been confirmed to the Bureau; and

–	for which the complete due diligence information, in accordance with Annex 2 to Resolution 49 (Rev.WRC-15), had been received by the Bureau prior to 30 March 2002.     (WRC-15)

# 7	Limits to the change in equivalent noise temperature to protect the fixed-satellite service (Earth-to-space) in Region 1 from modifications to the Region 2 Plan in the band 12.5-12.7 GHz

With respect to § 4.2.3 e) of Article 4, an administration is considered as being affected if the proposed modification to the Region 2 Plan would result in:

–	the value of ΔT / T of its overlapping frequency assignments in the fixed-satellite service in Region 1 resulting from the proposed modification is greater than the value of ΔT / T resulting from the assignment in the Region 2 Plan as of the date of entry into force of the Final Acts of the 1985 Conference; and

–	the value of ΔT / T of its overlapping frequency assignments in the fixed-satellite service in Region 1 resulting from the proposed modification exceeds 6%,

using the method of Appendix 8 (Case II).     (WRC-07)

ANNEX  2     (Rev.WRC-03)

Basic characteristics to be furnished in notices relating to 
space stations in the broadcasting-satellite service

These data items are listed in Appendix 4.

ANNEX  3     (WRC-03)

Method for determining the limiting interfering power flux-density at
the edge of a broadcasting-satellite service area in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz 
(in Region 2), and for calculating the power flux-density produced in these bands by a terrestrial station, or by a transmitting earth station in the 
fixed-satellite service in the band 12.5-12.7 GHz

# 1	General

1.1	This Annex describes a method of calculating the interference potential from terrestrial transmitters or transmitting earth stations in the fixed-satellite service (FSS) to receiving earth stations in the broadcasting-satellite service (BSS).

1.2	The method is in two parts:

a)	the calculation of the maximum permissible interfering power flux-density at the edge of the BSS area concerned;

b)	the calculation of the likely power flux-density produced at any point on the edge of the service area by the terrestrial transmitter or transmitting earth stations in the FSS of another administration.

1.3	The interference potential of the terrestrial transmitters or the transmitting earth stations in the FSS must be considered case by case; the power flux-density produced by each terrestrial transmitter or each transmitting earth station Fp is compared to the limiting power flux-density F at any point on the edge of the service area of a broadcasting-satellite station of another administration. If, for a given transmitter, the value of the power flux-density produced Fp is lower than the value of the limiting power flux-density F at any point on the edge of the service area, the interference caused to the BSS by this transmitter is considered to be lower than the permissible value and no coordination is required between administrations before the terrestrial service station or the transmitting earth station is brought into use. Where this is not the case, coordination and more precise calculations derived from a mutually agreed basis are necessary.

Section 2 calculates the limit of power flux-density F at the edge of the service area.

Section 3 calculates the power flux-density produced by a terrestrial station or a transmitting earth station, Fp.

1.4	It is emphasized that, should the calculation described in this Annex indicate that the maximum permissible power flux-density is exceeded, it does not necessarily preclude the introduction of the terrestrial or the FSS since the calculations are necessarily based on worst-case assumptions for:

a)	the nature of the terrain of the interference path;

b)	the off-beam discrimination on the broadcasting-satellite receiving installations;

c)	the necessary protection ratios for the BSS;

d)	the type of reception in the BSS, i.e., assuming individual reception, this being more critical than community reception for the angles of elevation concerned;

e)	the value of power flux-density to be protected in the BSS;

f)	the propagation conditions between the terrestrial station or the transmitting earth station in the FSS operating in the opposite direction of transmission, and the BSS area.

# 2	Limit of power flux-density

## 2.1	General

The limiting power flux-density not to be exceeded at the edge of the service area in order to protect the BSS of an administration is given by the formula:

F  =  F0  –  R  +  D  +  P	(1)

where:

F 	the maximum permissible interfering power flux-density (dB(W/m2)) within the necessary bandwidth of the broadcasting-satellite;

F0 	the wanted power flux-density (dB(W/m2)) at the edge of the service area;

R 	the protection ratio (dB) between the wanted and interfering signals;

D 	angular antenna discrimination (dB) provided by the radiation pattern of the broadcasting-satellite receiver antenna;

P 	polarization discrimination (dB) between the wanted and interfering signals.

## 2.2	Wanted power flux-density (F0)

The value of F0 is equal to:

For the Regions 1 and 3 Plan and List, Region 2 Plan and Article 4 submissions under § 4.1.3 and 4.2.6:

a)	−108 dB(W/(m2 · 27 MHz)) for service areas in Regions 1 and 3, and

b)	−115 dB(W/(m2 · 24 MHz)), as well as in dB(W/(m2 · 27 MHz)) with respect to the cases  mentioned in the footnote to § 3.8 of Annex 5 concerning necessary bandwidths in Region 2.

For the analogue BSS assignments in the Region 2 Plan:

−107 dB(W/(m2 · 24 MHz)), as well as in dB(W/(m2 · 27 MHz)) with respect to the cases  mentioned in the footnote to § 3.8 of Annex 5 concerning necessary bandwidths in Region 2.

## 2.3	Protection ratio (R)

2.3.1	For digital BSS assignments, the single entry protection ratio is equal to 30 dB.

2.3.2	For the analogue BSS assignments in the Region 2 Plan and for notified BSS assignments in Regions 1 and 3 Plan and List which are in conformity with the Plans and List of Appendix 30 and which have been brought into use and for which the date of bringing into use has been confirmed to the Bureau before 9 June 2003, the single entry protection ratio against all types of terrestrial transmissions, with the exception of amplitude-modulation multichannel television systems, is 35 dB for carrier frequency differences between the wanted and interfering signals of up to 0 MHz, decreasing linearly from 35 dB to 0 dB for carrier frequency differences between 10 MHz and 35 MHz, and is 0 dB for frequency differences in excess of 35 MHz (see Fig. 1). For amplitude-modulation multichannel television systems which produce high peaks of power flux-density spread over a wide range of their necessary bandwidth, the protection ratio R is 35 dB and is independent of the carrier frequency difference.

2.3.3	The carrier frequency difference should be determined by reference to the frequency assignments in the broadcasting-satellite Plan or, in the case of assignments not contained within a plan, by reference to the characteristics of the proposed or operational system.

2.3.4	A signal from a terrestrial station or a transmitting earth station in the FSS should be considered only if its necessary bandwidth overlaps the necessary bandwidth of the BSS assignment.

## 2.4	Angular antenna discrimination (D)

### 2.4.1	For all Regions (digital)

The value of D to be assumed in equation (1) is derived from the following equations, which are based on Recommendation ITU-R BO.1213 (also found in Annex 5):

D = 0.0025((d/) )2	dB	for  0°  ≤  < m

D = Gmax − (29 − 25 log(r))	dB	for  m ≤  < r	(2)

D = Gmax − (29 − 25 log())	dB	for  r  ≤  ≤ 14.45°

D = Gmax		dB	for            14.45°

where:

:	elevation angle (degrees) for the proposed or operational broadcasting-satellite system for the BSS area concerned

m :	(/d)((Gmax − G1)/(0.0025))0.5 (degrees)

G1 :	29 − 25 log(r) (dB)

r :	95(/d) (degrees)

Gmax :	maximum gain of the antenna (dBi)

d :	diameter of the antenna (m)

:	the wavelength (m).

NOTE 1 – If more than one value of  is specified for a particular service area, the appropriate value of  should be used for each section of the edge of the service area under consideration.

For Regions 1 and 3, Gmax = 35.5 dBi corresponding to a 0.6 m diameter antenna at 11.7 GHz and 65% efficiency. For Region 2, Gmax = 33.3 dBi corresponding to a 0.45 m diameter antenna at 12.2 GHz and 65% efficiency. For a graphical depiction of this antenna discrimination see Fig. 2.

### 2.4.2	For the analogue BSS assignments in the Region 2 Plan

The discrimination D should be derived from the expression (3) below where φ is the elevation angle for the proposed or operational broadcasting-satellite system for the BSS area concerned.

NOTE 1 – If more than one value of φ is specified for a particular service area, the appropriate value of φ should be used for each section of the edge of the service area under consideration.

| D = 0 | dB | for 0° ≤ φ ≤ 0.43° | (3) |
| --- | --- | --- | --- |
| D = 4.15 φ | dB | for 0.43° < φ ≤ 1.92° |  |
| D = 8.24 + 25 log φ | dB | for 1.92° < φ ≤ 25° |  |
| D = 43.2 | dB | for φ  25° |  |

NOTE 2 – For the graphical determination of D see Fig. 3. The unit for φ is degrees.

## 2.5	Polarization discrimination (P)

The value of P is equal to:

a)	3 dB when the interfering service uses linear polarization and the BSS uses circular polarization or vice versa;

b)	0 dB when the interfering service and the BSS both use circular or both use linear polarization.

# 3	Power flux-density produced by a terrestrial station or a transmitting earth station (Fp)

The power flux-density Fp (dB(W/m2)) produced at any point on the edge of the service area by the terrestrial station or the transmitting earth station is determined from the following formula:

Fp  =  E  −  A  +  10 log (4/2)	(4)

where:

E 	equivalent isotropically radiated power (dBW) of the terrestrial station or the transmitting earth station in the direction of the point concerned on the edge of the service area

A 	total path loss (dB)

:	wavelength (m).

## 3.1	Evaluation of path loss A for a terrestrial station or a transmitting earth station at the edge of the service area of the broadcasting satellite

The following propagation model is to be used for determining the minimum path loss between the interfering terrestrial transmitter or transmitting earth station and the edge of the BSS service area.

## 3.2	Propagation model

### 3.2.1	Distance limits

#### 3.2.1.1	Minimum distance limit

The minimum coordination distance is given as:

(5)

where:

f:	frequency (GHz)

βp:	radiometeorological parameter, which reflects the relative incidence of clear-sky anomalous propagation conditions.

The value of βp is latitude dependent. The latitude to be used in determining the correct value for βp is given by:

(6)

where  is the earth station latitude (degrees).

βp is then determined using:

(7)

#### 3.2.1.2	Maximum distance limit

The maximum distance, dmax, for paths comprising a single climatic zone must not exceed the value for that climatic zone given in the Table below. For mixed paths comprising multiple zones the overall maximum distance must not exceed the value in the Table below corresponding to the climatic zone in the mixed path having the largest value (e.g. for a mixed path comprising Zones A1 and A2, dmax is 500 km).

| Climatic Zone1 | Maximum distance, dmax2 |
| --- | --- |
| A1 | 500 |
| A2 | 375 |
| B | 900 |
| C | 1 200 |
| 1 For the definition see Appendix 7, § 1.5.1 and 1.5.3.2. 2 As computed in § 2 of Appendix 7. |  |

### 3.2.2	Ducting model

#### 3.2.2.1	Distance-independent part of the loss (dB) for ducting

For BSS earth stations, no additional protection due to the earth station horizon elevation angle can be assumed, i.e. Ah, the total terrain shielding attenuation, is 0 dB. However, if the detailed information for the transmitting station is known, including any site-shielding-based mitigation techniques that are used, all these factors need to be included in the determination of the coordination distance.

Reduction in attenuation arising from direct coupling into over-sea ducts (dB):

(8)

where dc (km) is the distance from a land-based transmitting station to the coast in the direction being considered. dc is zero in other circumstances.

Distance-independent part of the loss (dB) for ducting:

(9)

#### 3.2.2.2	Distance-dependent part of the loss (dB) for ducting

a)	The specific attenuation (dB/km) due to dry air is given as:

(10)

b)	The specific attenuation due to water vapour is given as a function of , the water vapour density in units of g/m3, by the following equation:

(11)

c)	The specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 7.5 g/m3 for paths over land in Zones A1 and A2 is given as:

wdl  =  w (7.5)	(12)

d)	The specific attenuation (dB/km) due to water vapour for the ducting propagation model using a water vapour density of 10.0 g/m3 for paths over sea in Zones B and C is given as:

wds  =  w (10.0)	(13)

Note that the value of 10.0 g/m3 is used for both Zones B and C in view of the lack of data on the variability of water vapour density on a global basis, particularly the minimum values.

e)	Specific attenuation due to gaseous absorption (dB/km):

(14)

where:

dt (km):	aggregate land distance (Zone A1 + Zone A2) along the path;

di (km):	path length considered, which lies within the range between a minimum calculation distance and a maximum calculation distance.

f )	Values for zone-dependent parameters:

(15)

where:

dlm (km):	longest continuous inland distance (Zone A2) along the path considered.

(16)

where:

dtm (km):	longest continuous land (i.e. inland + coastal) distance (Zone A1 + Zone A2) along the path considered.

1 is limited to 1 ≤ 1.

(17)

is limited to  ≥ –3.4.

(18)

2 is limited to 2 ≤ 1.

|  | for ζr ≤ 70° for ζr ≤ 70° | (19) |
| --- | --- | --- |

g)	Path-dependent incidence of ducting, β, and the related parameter, Γ1, that are used to calculate time dependency of the path loss are given as:

β = βe · μ1 · μ2 · μ4	(20)

(21)

h)	Distance-dependent part of the loss (dB) for ducting:

(22)

where:

p:	the maximum percentage of time for which the permissible interference power may be exceeded; p = 0.3%

γd:	the frequency-dependent ducting specific attenuation (dB/km).

(23)

NOTE 1 – For coordination of terrestrial mobile transmitting stations, fixed stations and transmitting earth stations, the mitigation factor C2i was set equal to zero.

i)	Attenuation due to ducting:

(24)

### 3.2.3	For the tropospheric scatter model

#### 3.2.3.1	Distance-independent part of the loss (dB) for tropospheric scatter

(25)

where:

εh:	earth station horizon elevation angle (degrees)

N0:	path centre sea level surface refractivity given as:

(26)

Lf : 	the frequency-dependent part of the loss (dB), given as:

(27)

#### 3.2.3.2	Distance-dependent part of the loss (dB) for tropospheric scatter

(28)

Total attenuation due to tropospheric scatter:

(29)

#### 3.2.3.3	Minimum path loss

The minimum path loss, Amin, between the site of the interfering transmitter and the edge of the BSS service area is given by:

Amin = min (Aduct, Atrop)	(30)

ANNEX  4     (Rev.WRC-15)

Need for coordination of a transmitting space station in the fixed-satellite 
service or in the broadcasting-satellite service where this service is not subject 
to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Regions 1 and 3; in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with respect to 
the Plan or proposed modifications to the Plan in Region 2; in Region 3 
(12.2-12.5 GHz) with respect to the Plan, List or proposed new or 
modified assignments in the List for Region 1

(See Article 7)

With respect to § 7.1 and 7.2 of Article 7, coordination of a transmitting space station in the fixed-satellite service (FSS) (space-to-Earth) of Region 2 or Region 3 is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the overlapping frequency assignments in the BSS of an administration in Region 1 or Region 3 exceeds the following values:      (WRC-07)

−147    dB(W/(m2  27 MHz))	for  0°        ≤ θ < 0.23°

−135.7  +  17.74 log θ    dB(W/(m2  27 MHz))	for  0.23°   ≤ θ < 2.0°

−136.7  +  1.66 θ    dB(W/(m2  27 MHz))	for  2.0°     ≤ θ < 3.59°

−129.2  +  25 log θ    dB(W/(m2  27 MHz))	for  3.59°   ≤ θ < 10.57°

−103.6    dB(W/(m2  27 MHz))	for  10.57° ≤ θ

where θ is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

In the case of an administration in Region 3 that has notified and brought into use its BSS Plan assignments before 9 June 2003, and whose notified assignments have been recorded in the Master Register with a favourable finding and for which the date of bringing into use has been confirmed to the Bureau, with respect to § 7.2.1 a) of Article 7, the conditions contained above are replaced by the following conditions:

–	under assumed free-space propagation conditions, the power flux-density at any test point within the service area of the overlapping frequency assignments in the Plan does not exceed the following values:33   (WRC-15)

−147    	dB(W/(m2  27 MHz))	for  0°        ≤ θ < 0.23°

−135.7  +  17.74 log θ    dB(W/(m2  27 MHz))	for  0.23°   ≤ θ < 1.8°

−134.0 +  0.89 θ    dB(W/(m2  27 MHz))	for  1.8°     ≤ θ < 5.0°

−129.2  +  25 log θ    dB(W/(m2  27 MHz))	for  5.0°     ≤ θ < 10.57°

−103.6    dB(W/(m2  27 MHz))	for  10.57° ≤ θ

where θ is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

With respect to § 7.1 and 7.2 of Article 7, coordination of a transmitting space station in the FSS (space-to-Earth) in Region 1 or 3 or BSS not subject to a Plan in Region 3 is required when, under assumed free-space propagation conditions, the power flux-density over any portion of the service area of the overlapping frequency assignments in the BSS of an administration in Region 2 exceeds the following values:

−147    	dB(W/(m2  27 MHz))	for  0°        ≤ θ < 0.23°

−135.7  +  17.74 log θ    dB(W/(m2  27 MHz))	for  0.23°   ≤ θ < 1.8°

−134.0  +  0.89 θ    dB(W/(m2  27 MHz))	for  1.8°     ≤ θ < 5.0°

−129.2  +  25 log θ    dB(W/(m2  27 MHz))	for  5.0°     ≤ θ < 10.57°

−103.6    dB(W/(m2  27 MHz))	for  10.57° ≤ θ

where θ is the minimum geocentric orbital separation in degrees between the wanted and interfering space stations, taking into account the respective East-West station-keeping accuracies.

ANNEX  5

Technical data used in establishing the provisions and associated Plans
and the Regions 1 and 3 List, which should be used
              for their application34     (Rev.WRC-03)

# 1	Definitions

## 1.1	Downlink service area

The area on the surface of the Earth in which the administration responsible for the service has the right to demand that the agreed protection conditions be provided.

NOTE – In the definition of service area, it is made clear that within the service area the agreed protection conditions can be demanded. This is the area where there should be at least the wanted power flux-density and protection against interference based on the agreed protection ratio for the agreed percentage of time.

## 1.2	Downlink coverage area

The area on the surface of the Earth delineated by a contour of a constant given value of power flux-density which would permit the wanted quality of reception in the absence of interference.

NOTE 1 – In accordance with the provisions of No. 23.13, the coverage area must be the smallest area which encompasses the service area.

NOTE 2 – The coverage area, which will normally encompass the entire service area, will result from the intersection of the antenna beam (elliptical, circular, or shaped) with the surface of the Earth, and will be defined by a given value of power flux-density. For example, it would be the area delineated by the contour corresponding to the level specified in § 3.16 of this Annex. There will usually be an area outside the service area but within the coverage area in which the power flux-density will be at least equivalent to the minimum specified value; however, protection against interference will not be provided in this area.

NOTE 3 – If coverage is provided by a steerable beam, the contour delineating the coverage area will depend on the pointing capability of the beam and will not necessarily cover the entire service area.

## 1.3	Downlink beam area

The area delineated by the intersection of the half-power beam of the satellite transmitting antenna with the surface of the Earth. The downlink beam area concept was generally used for planning purposes in conjunction with elliptical beams.

NOTE – The beam area is simply that area on the Earth’s surface corresponding to the –3 dB points on the satellite antenna radiation pattern. In many cases the beam area would almost coincide with the coverage area, the discrepancy being accounted for by the permanent difference in path lengths from the satellite throughout the beam area, and also by the permanent variations, if any, in propagation factors across the area. However, for a service area where the maximumdimension as seen from the satellite position is less than 0.6° in Regions 1 and 3, and less than 0.8° in Region 2 (the agreed minimum practicable satellite antenna half-power beamwidths), there could be a significant difference between the beam area and the coverage area.

## 1.4	Nominal orbital position

The longitude of a position in the geostationary-satellite orbit associated with a frequency assignment to a space station in a space radiocommunication service. The position is given in degrees from the Greenwich meridian.

NOTE – Definitions in § 1.6 to 1.11 are applicable to Region 2.     (WRC-2000)

## 1.5	Adjacent channel

The RF channel in the broadcasting-satellite service frequency Plan, or in the associated feeder-link frequency Plan, which is situated immediately higher or lower in frequency with respect to the reference channel.

## 1.6	Second adjacent channel

The RF channel in the broadcasting-satellite service frequency Plan, or in the associated feeder-link frequency Plan, which is situated immediately beyond either of the adjacent channels, with respect to the reference channel.

## 1.7	Overall carrier-to-interference ratio

The overall carrier-to-interference ratio is the ratio of the wanted carrier power to the sum of all interfering RF powers in a given channel including both feeder links and downlinks. The overall carrier-to-interference ratio due to interference from the given channel is calculated as the reciprocal of the sum of the reciprocals of the feeder-link carrier-to-interference ratio and the down-link carrier-to-interference ratio referred to the satellite receiver input and earth station receiver input, respectively35.

## 1.8	Overall co-channel protection margin

The overall co-channel protection margin in a given channel is the difference in decibels between the overall co-channel carrier-to-interference ratio and the co-channel protection ratio.

## 1.9	Overall adjacent channel protection margin

The overall adjacent channel protection margin is the difference in decibels between the overall adjacent channel carrier-to-interference ratio and the adjacent channel protection ratio.

## 1.10	Overall second adjacent channel protection margin

The overall second adjacent channel protection margin is the difference in decibels between the overall second adjacent channel carrier-to-interference ratio and the second adjacent channel protection ratio.

## 1.11	Overall equivalent protection margin36

The overall equivalent protection margin, M, is given in decibels by the expression:

where:

M1 	overall co-channel protection margin (dB) (as defined in § 1.8);

M2, M3 	overall adjacent channel protection margins for the upper and lower adjacent channels, respectively (dB) (as defined in § 1.9);

M4, M5 	overall second adjacent channel protection margins for the upper and lower second adjacent channels, respectively (dB) (as defined in § 1.10)37.

The adjective “equivalent” indicates that the protection margins for all interference sources from the adjacent and second adjacent channels as well as co-channel interference sources have been included.     (WRC-2000)

# 2	Radio propagation factors

In Regions 1 and 3:

2.1	The propagation loss on the space-to-Earth path (used for computing downlink e.i.r.p. and as a guide in choosing orbital locations during the development of the Plan) is equal to the free-space path loss plus the atmospheric absorption and the rain attenuation exceeded for 1% of the worst month. Values of this attenuation can be calculated as a function of angle of elevation for the rain-climatic zones shown in Figs. 1 and 2 from Recommendation ITU-R P.837-1 by applying the method described in Recommendation ITU-R P.618-5.

In Region 2:

2.2	The propagation loss on a space-Earth path is equal to the free-space path loss plus the atmospheric absorption loss plus the rain attenuation exceeded for 1% of the worst month.

### 2.2.1	Atmospheric absorption

The loss due to atmospheric absorption (i.e. clear-sky attenuation) is given by:

dB	for  θ  <  5°

where:

and:

dB	for  θ  ≥  5°

where:

θ 	elevation angle (degrees),

 	surface water vapour concentration (g/m3), being

  =  10 g/m3 for rain climatic zones A to K and

  =  20 g/m3 for rain climatic zones M to P (see Fig. 3).

### 2.2.2	Rain attenuation

The rain attenuation Ap of circularly polarized signals exceeded for 1% of the worst month at 12.5 GHz is given by:

dB	(31)

where:

L:	slant path length through rain

km

r:	rain path length reduction factor

hR:	rain height (km)

km

where:

c  =  0.6				for		|  |  ≤  20°

c  =  0.6 + 0.02 (|  | – 20)	for	20° <	|  |  ≤  40°

c  =  1.0				for		|  |    40°

h0:	height (km) above mean sea level of the earth station;

:	earth station latitude (degrees);

θ:	elevation angle (degrees);

:	specific rain attenuation = 0.0202 R1.198 dB/km;

R:	rain intensity (mm/h) obtained from the table below for the rain climatic zones identified in Fig. 3.

(NOTE – The method is based on R exceeded for 0.01% of an average year.)

Rainfall intensity (R) for the rain climatic zones 
(exceeded for 0.01% of an average year) (see Fig. 3)

| Rain climatic zone | A | B | C | D | E | F | G | K | M | N | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rainfall intensity (mm/h) | 8 | 12 | 15 | 19 | 22 | 28 | 30 | 42 | 63 | 95 | 145 |

Figure 4 presents plots of rain attenuation, as calculated using equation (31), of circularly polarized signals exceeded for 1% of the worst month at 12.5 GHz, as a function of earth station latitude and elevation angle for each of the rain climatic zones shown in Fig. 3.

### 2.2.3	Rain attenuation limit

In the analysis of the Plan for the broadcasting-satellite service in Region 2, a maximum downlink attenuation of 9 dB was agreed in order to limit the inhomogeneity of broadcasting-satellite power flux-density and to facilitate sharing during clear-sky conditions.

### 2.2.4	Procedure for calculating the carrier-to-interference ratio at a test point

The calculation of the down-link carrier-to-interference ratio (exceeded for 99% of the worst month) used to obtain the overall equivalent protection margin at a test point is the minimum value of the carrier-to-interference ratio obtained assuming:

i)	clear-sky conditions (i.e. including atmospheric absorption); or

ii)	rain-faded conditions corresponding to an attenuation value exceeded for 1% of the worst month.

## 2.3	Depolarization

Rain and ice can cause depolarization of radio frequency signals. The level of the co-polar component relative to the depolarized component is given by the cross-polarization discrimination (XPD) ratio. For circularly polarized emissions, the XPD ratio (dB) exceeded for 99% of the worst month is obtained from:

for 5°  ≤  θ  ≤  60°	(32)

where Ap (dB) is the co-polar rain attenuation exceeded for 1% of the worst month (calculated in § 2.2), f is the frequency in GHz and θ is the elevation angle. For angles of θ greater than 60°, use θ = 60° in equation (32).

# 3	Basic technical characteristics

## 3.1	Type of modulation

3.1.1	At WARC-77 and during revision of the Regions 1 and 3 Plan at WRC-97, planning of the broadcasting-satellite service was based on the use of a signal consisting of a video signal with an associated carrier, frequency-modulated by a sound signal, both frequency-modulating a carrier in the 12 GHz band, with a pre-emphasis characteristic in accordance with Fig. 5 (from Recommendation ITU-R F.405-1*). The WRC-2000 Regions 1 and 3 Plan and the List are generally based on digital modulation of sound and television signals.     (WRC-2000)

3.1.2	In Region 2, planning is based on the use of a frequency-modulated composite-coded colour television signal with two sound sub-carriers. However, in recognition of the need to provide for the use of new, enhanced television coding and modulation formats (e.g. time-compressed, multiplexed analogue video component signals and digitally-coded sound and data signals), values of the important technical characteristics have been chosen to take into consideration the implementation of these new formats within the provisions of the Plan.

3.1.3	Nevertheless, other modulating signals having different characteristics (e.g. modulation with sound channels frequency-multiplexed within the bandwidth of a television channel, digital modulation of sound and television signals, or other pre-emphasis characteristics) are not precluded, provided that appropriate protection masks and calculation methods38 are applied or if the use of such characteristics complies with the provisions of § 3.2 of Article 3.

## 3.2	Polarization

3.2.1	For the planning of the broadcasting-satellite service, circular polarization is generally used. However, for implementation of assignments in the Plan, linear polarization may also be used, subject to the successful application of the modification procedure of Article 4.

3.2.2	In Regions 1 and 3, the polarization of different beams intended to serve the same area should, if possible, be the same.

3.2.3	The terms “direct” and “indirect” used in the Plans to indicate the direction of rotation of circularly-polarized waves correspond to right-hand (clockwise) and left-hand (anti-clockwise) polarization respectively according to the following definitions:

Direct polarization (right-hand or clockwise polarization):

An elliptically or circularly-polarized electromagnetic wave, in which the electric field-intensity vector, observed in any fixed plane, normal to the direction of propagation, whilst looking in (i.e., not against) the direction of propagation, rotates with time in a right-hand or clockwise direction.

NOTE – For right-hand circularly-polarized plane waves, the ends of the electric vectors drawn from any points along a straight line normal to the plane of the wave front form, at any instant, a left-hand helix.

Indirect polarization (left-hand or anti-clockwise polarization):

An elliptically or circularly-polarized electromagnetic wave, in which the electric field-intensity vector, observed in any fixed plane, normal to the direction of propagation, whilst looking in (i.e., not against) the direction of propagation, rotates with time in a left-hand or anti-clockwise direction.

NOTE – For left-hand circularly-polarized plane waves, the ends of the electric vectors drawn from any points along a straight line normal to the plane of the wave front form, at any instant, a right-hand helix.

3.2.4	Linear polarization is defined in Recommendation ITU-R BO.1212. This Recommendation should be used when analysing linearly polarized signals.

## 3.3	Carrier-to-noise ratio

For the purpose of planning the broadcasting-satellite service, the carrier-to-noise ratio is equal to or exceeds 14 dB for 99% of the worst month.

In Regions 1 and 3, the reduction in quality in the down-link due to thermal noise in the feeder-link is taken as equivalent to a degradation in the down-link carrier-to-noise ratio not exceeding 0.5 dB for 99% of the worst month. In Region 2, as a guide for planning, the reduction in quality in the down-link due to thermal noise in the feeder link is taken as equivalent to a degradation in the down-link carrier-to-noise ratio of approximately 0.5 dB not exceeded for 99% of the worst month, but the feeder-link and down-link Plans are evaluated on the basis of the overall carrier-to-noise ratio of 14 dB for the combined down-link and feeder-link contributions.

## 3.4	Protection ratio between television signals

For developing the original 1977 broadcasting-satellite service Plan for Regions 1 and 3, the following protection ratios were used39, 40:

–	31 dB for co-channel signals;

–	15 dB for adjacent channel signals.

For revising this Plan at WRC-97, the following aggregate downlink protection ratios were specified in Recommendation ITU-R BO.1297 for the purpose of calculating downlink equivalent protection margins40, 41, 42:

–	24 dB for co-channel signals;

–	16 dB for adjacent channel signals.

In revising the Regions 1 and 3 Plan at WRC-97, the following aggregate overall protection ratio values were used for calculating the overall co-channel and adjacent-channel protection margins as defined in § 1.8 and 1.9:

–	23 dB for co-channel signals;

–	15 dB for adjacent channel signals.

It was also specified that for the revision of the Regions 1 and 3 Plan, no overall co-channel single entry C/I should be lower than 28 dB.

However, for the assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997, the overall equivalent protection margins were calculated using a co-channel overall protection ratio of 30 dB and lower and upper overall adjacent channel protection ratios of 14 dB43.

WRC-2000 adopted, for the protection of digital assignments from digital emissions, the following protection ratio values to be applied for calculation of downlink equivalent protection margins of the WRC-2000 Regions 1 and 3 Plan:

–	21 dB for co-channel signals;

–	16 dB for adjacent channel signals.

During planning at WRC-2000, these values were used for all assignments of the Regions 1 and 3 Plan and List except those for which WRC-2000 adopted different values used in the planning process44.

Revision of the Regions 1 and 3 Plan at WRC-97 and planning at WRC-2000 were generally based on a set of reference parameters such as the average e.i.r.p., the reference earth station receiving antenna, all test points placed within the −3 dB contour, a bandwidth of 27 MHz and the predetermined value of C/N. The Regions 1 and 3 Plan as established by WRC-2000 is generally based on the use of digital modulation.

Protection masks and associated calculation methods for interference into broadcasting satellite systems involving digital emissions shall be in accordance with Recommendation ITU-R BO.1293-2 (Annexes 1 and 245).

In Region 2, the following protection ratios have been adopted for the purpose of calculating the overall equivalent protection margin46:

28 dB for co-channel signals;

13.6 dB for adjacent-channel signals;

−9.9 dB for second adjacent-channel signals.

In Region 2, as a guide for planning, the reduction in the overall C/I ratio due to co-channel interference in the feeder link is taken as equivalent to a degradation in the downlink co-channel C/I ratio of approximately 0.5 dB not exceeded for 99% of the worst month; however, the feeder-link and downlink Plans are evaluated on the basis of the overall equivalent protection margin, which includes the combined downlink and feeder-link contributions.

In Region 2, an overall equivalent protection margin of 0 dB, or greater, indicates that the individual protection ratios have been met for the co-channel, the adjacent channels and the second adjacent channels.     (WRC-03)

### 3.4.1	Adjacent channel protection ratio template for Region 247 (FMTV into FMTV)

The protection ratios for adjacent channels are derived from the template given in Fig. 6. The template is symmetrical and is given in terms of absolute levels for the C/I ratios.

The template is obtained by joining the segment for adjacent channels to the horizontal extension of the co-channel protection ratio value. The adjacent channel protection ratio cannot be adjusted relative to the co-channel value.

The template is given by the following expressions:

| PR = |  | 28 dB for ¦ F0 ¦ ≤ 8.36 MHz −2.762 ¦ F0 ¦ + 51.09 dB for 8.36 < ¦ F0 ¦ ≤ 12.87 MHz −1.154 ¦ F0 ¦ + 30.4 dB for 12.87 < ¦ F0 ¦ ≤ 21.25 MHz −2.00 ¦ F0 ¦ + 48.38 dB for ¦ F0 ¦ > 21.25 MHz |
| --- | --- | --- |

where PR is the protection ratio (dB) and | F0 | is the carrier spacing between the interfering and wanted signals (MHz).

## 3.5	Channel spacing

### 3.5.1	Channel spacing in the Plans

In Regions 1 and 3, the spacing between the assigned frequencies of two adjacent channels is 19.18 MHz.

In Region 2, the spacing between the assigned frequencies of two adjacent channels is 14.58 MHz, which corresponds to 32 channels in the 500 MHz bandwidth allocated to the broadcasting-satellite service.

The Plans give the assigned frequencies for each channel.

However, in the Regions 1 and 3 Plan, for the implementation of assignments different frequency spacing may be used subject to the successful application of the modification procedure of Article 4, ITU-R Recommendations for protection masks should be used if available. In the absence of such Recommendations, the Bureau should apply the worst-case approach as adopted by the Radio Regulations Board.

### 3.5.2	Arrangement of channels in the same beam

Planning in Region 1 at the 1977 Conference was carried out by trying to restrict all the channels radiated within a single antenna beam within a frequency range of 400 MHz, in order to simplify receiver construction. Such a restriction was considered unnecessary for the revision of the Regions 1 and 3 Plan at WRC-97.

### 3.5.3	Spacing between assigned channel frequencies feeding a common antenna

In the 1977 Plan for Regions 1 and 3, owing to technical difficulties in the output circuit of a satellite transmitter, spacing between the assigned frequencies of two channels feeding a common antenna was required to be greater than 40 MHz. This restriction was not imposed in the revision of the Plan.

## 3.6	Figure of merit (G/T) of a receiving station in the broadcasting-satellite service

In planning the broadcasting-satellite service, the value of the figure of merit G/T for clear-sky conditions is:

for Regions 1 and 3:

The original 1977 broadcasting-satellite service Plan used values48 of:

6 dB(K−1) for individual reception

14 dB(K−1) for community reception, and

for Region 2:

10 dB(K−1) for individual reception.

The 1997 revision of the Regions 1 and 3 Plan is based on a uniform value of the figure of merit G/T equal to 11 dB(K−1).

These values were calculated from a formula which allows for pointing error, polarization effects and equipment ageing.

See also Report ITU-R BO.473-3 (Annex 1).

## 3.7	Receiving antennas

### 3.7.1	Half-power beamwidth of receiving antennas

In the development of the original 1977 broadcasting-satellite service Plan for Regions 1 and 3, the minimum receiving antenna diameter was such that the half-power beamwidth was 2° for individual reception and 1° for community reception.

In revising this Plan at WRC-97, the minimum receiving antenna diameter was such that the half-power beamwidth was 2.86°.     (WRC-07)

For planning the broadcasting-satellite service in Region 2, the minimum receiving antenna diameter must be such that the half-power beamwidth φ0 is 1.7°.

### 3.7.2	Receiving antenna reference patterns

The co-polar and cross-polar receiving antenna reference patterns are given in Figs. 7, 7bis and 8.

a)	For Regions 1 and 3, the original 1977 Conference Plan was based on the antenna pattern49 shown in Fig. 7 where the relative antenna gain (dB) is given by the curves for:

–	individual reception, for which use should be made of:

–	Curve A for the co-polar component;

–	Curve B for the cross-polar component;

–	community reception, for which use should be made of:

–	Curve A′ up to the intersection with Curve C, then Curve C, for the co-polar component;

–	Curve B for the cross-polar component.

The WRC-97 revision of the Regions 1 and 3 broadcasting-satellite service Plan was based on the absolute gain (dBi) patterns for a 60 cm antenna given in Recommendation ITU-R BO.1213 as shown in Fig. 7bis.

b)	For Region 2, the relative antenna gain (dB) is given by the curves in Fig. 8 for individual reception, for which use should be made of:

–	Curve A for the co-polar component;

–	Curve B for the cross-polar component.

Curve A:	co-polar component for individual reception without side-lobe suppression (dB relative to main beam gain)

0		for	0		≤	φ  ≤  0.25 φ0

for	0.25 φ0	<	φ  ≤  0.707 φ0

for	0.707 φ0	<	φ  ≤  1.26 φ0

for	1.26 φ0	<	φ  ≤  9.55 φ0

−33		for				φ  <  9.55 φ0

Curve A′:	co-polar component for community reception without side-lobe suppression (dB relative to main beam gain)

0		for	0		≤	φ  ≤  0.25 φ0

for	0.25 φ0	<	φ  ≤  0.86 φ0

|  | for φ  0.86 φ0 up to intersection with Curve C (then Curve C) |
| --- | --- |

Curve B:	cross-polar component for both types of reception (dB relative to main beam gain)

−25		for	0		≤	φ  ≤  0.25 φ0

for	0.25 φ0	<	φ  ≤  0.44 φ0

−20		for	0.44 φ0	<	φ  ≤  1.4 φ0

for	1.4 φ0	<	φ  ≤  2 φ0

−30	until intersection with co-polar component curve; then co-polar component curve.

Curve C:	minus the on-axis gain (Curve C in this figure illustrates the particular case of an antenna with an on-axis gain of 37 dBi).

NOTE – For values of φ0 see § 3.7.1.

Co-polar pattern:

for  0 ≤ φ < φm

where:

for φm ≤ φ < φr

where:

for  φr ≤ φ < φb

where:

Gco (φ)  =  −5 dBi		for  φb	≤ φ < 70°

Gco (φ)  =  0 dBi		for  70°	≤ φ < 180°

Cross-polar pattern:

for  0	≤ φ < 0.25 φ0

where:

=  3 dB beamwidth

for  0.25 φ0	≤ φ < 0.44 φ0

for  0.44 φ0	≤ φ < φ0

for	φ0	≤ φ  < φ1		(WRC-07)

where:

	wavelength corresponding to 12.1 GHz (m)

for	φ1   ≤  φ  <  φ2

where:

Gcross (φ)  =  −5 dBi		for	φ2   ≤  φ  <  70°

Gcross (φ)  =  0 dBi		for	70°  ≤  φ  <  180°

The reference frequency used in calculations for this antenna pattern = 12.1 GHz.

For the 0.60 m antenna pattern, which was used as a reference receiving antenna in replanning the absolute gain of 35.5 dBi was applied.     (WRC-03)

Curve A:	co-polar component without side-lobe suppression (dB relative to main beam gain)

0		for	0	≤  φ  ≤  0.25 φ0

−12 (φ/φ0)2		for	0.25 φ0	<  φ  ≤  1.13 φ0

−{14  +  25 log (φ/φ0)}		for	1.13 φ0	<  φ  ≤  14.7 φ0

−43.2		for	14.7 φ0	<  φ  ≤  35 φ0

−{85.2  −  27.2 log (φ/φ0)}	for	35 φ0	<  φ  ≤  45.1 φ0

−40.2		for	45.1 φ0	<  φ  ≤  70 φ0

−{−55.2  +  51.7 log (φ/φ0)}	for	70 φ0	<  φ  ≤  80 φ0

−43.2		for	80 φ0	<  φ  ≤  180°

Curve B:	cross-polar component (dB relative to main beam gain)

−25		for	0	≤  φ  ≤  0.25 φ0

for	0.25 φ0	<  φ  ≤  0.44 φ0

−20		for	0.44 φ0	<  φ  ≤  1.28 φ0

for	1.28 φ0	<  φ  ≤  3.22 φ0

−30	until intersection with co-polar component curve; then co-polar component curve.

NOTE 1 – For values of φ0 see § 3.7.1.

NOTE 2 – In the angular range between 0.1 φ0 and 1.13 φ0 the co-polar and cross-polar gains must not exceed the reference patterns.

NOTE 3 – At off-axis angles larger than 1.13 φ0 and for 90% of all sidelobe peaks in each of the reference angular windows, the gain must not exceed the reference patterns. The reference angular windows are 1.13 φ0 to 3 φ0, 3 φ0 to 6 φ0, 6 φ0 to 10 φ0, 10 φ0 to 20 φ0, 20 φ0 to 40 φ0, 40 φ0 to 75 φ0 and 75 φ0 to 180°.

## 3.8	Necessary bandwidth

WARC-77 Regions 1 and 3 Plan and the WRC-97 revision of the Regions 1 and 3 Plan used the following:

–	625-line systems in Regions 1 and 3: 27 MHz;

–	525-line systems in Region 3: 27 MHz.     (WRC-2000)

The planning at WRC-2000 was generally based on a necessary bandwidth of 27 MHz.    (WRC-2000)

In Region 2, the Plan is based on a channel bandwidth of 24 MHz50, but different bandwidths may be implemented in accordance with the provisions of this Appendix, provided that applicable ITU-R Recommendations are available. In the absence of such Recommendations, the Bureau will use the worst-case approach.     (WRC-2000)

If different bandwidths and/or channel spacing are submitted, they will be treated in accordance with applicable ITU-R Recommendations for protection masks when available. In the absence of such Recommendations, the Bureau will use the worst-case approach.     (WRC-2000)

## 3.9	Guardbands

3.9.1	A guardband is defined as the portion of the frequency spectrum between the edge of the allocated band and the edge of the necessary bandwidth of the emission in the nearest channel.

3.9.2	For the planning of the broadcasting-satellite service, the guardbands chosen at the 1977 Conference to protect the services in adjacent frequency bands are shown in the Table below.

| Regions | Guardband at the lower edge of the band (MHz) | Guardband at the upper edge of the band (MHz) |
| --- | --- | --- |
| 1 | 14 | 11 |
| 2 | 12 | 12 |
| 3 | 14 | 11 |

For Regions 1 and 3 at WARC-77, the guardbands were derived on the assumption of analogue emissions and a maximum beam centre e.i.r.p. of 67 dBW (value relating to individual reception), and a filter roll-off of 2 dB/MHz. If smaller e.i.r.p. values are assumed, the guardbands can be reduced in width by 0.5 MHz for each decibel decrease in e.i.r.p. The degree of possible reduction also depends on improvements in technology and on the type of modulation.     (WRC-2000)

3.9.3	(SUP - WRC-97)

3.9.4	The guardbands at both the lower and upper edges may be used to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.     (WRC-03)

## 3.10	Orbital spacing

The Plan for Regions 1 and 3 has been based generally on nominal orbital positions spaced uniformly at intervals of 6°. The Plan for Region 2 has been based on a non-uniform spacing.

## 3.11	Satellite station-keeping

Space stations in the broadcasting-satellite service must be maintained in position with an accuracy equal to or better than 0.1° in the E-W directions. For such space stations, the maintenance of the tolerance 0.1° in the N-S direction is recommended but is not a requirement.

## 3.12	Elevation angle of receiving antennas

The Plans have been based on the desirability of a minimum angle of elevation of 20° to minimize the required e.i.r.p. of the satellite and to reduce the effects of shadowing and the possibility of interference from terrestrial services. However, for areas situated in latitudes above about 60°, the angle of elevation is of necessity less than 20°. Attention is also drawn to § 2.1 for the Regions 1 and 3 Plan and to § 2.2.3 for the Region 2 Plan.

For mountainous areas where an elevation angle of 20° may not suffice, an angle of at least 30° has been provided, where possible, to provide an acceptable service. An angle of elevation of at least 40° has been considered for service areas subject to high precipitation, but exceptions were made in some cases in Region 2.

Some dry, non-mountainous areas may be given an acceptable service at angles of elevation less than 20°.

In areas with small elevation angles, the shadowing effect of tall buildings may have to be taken into account.

In choosing a satellite position designed to give the maximum angle of elevation at the ground, the influence of such a position on the eclipse period was taken into account at the 1977 Conference. In the revision of the Regions 1 and 3 Plan at WRC-97, this influence was not considered to be a significant constraint on the choice of orbital position.

## 3.13	Transmitting antennas

### 3.13.1	Cross-section of transmitted beam

Planning in Regions 1, 2 and 3 has been generally based on the use of satellite transmitting antennas with beams of elliptical cross-section.

If the cross-section of the emitted beam is elliptical, the effective beamwidth φ0 is a function of the angle of rotation between the plane containing the satellite and the major axis of the beam cross-section and the plane in which the beamwidth is required.

The relationship between the maximum gain of an antenna and the half-power beamwidth can be derived from the expression:

where:

a and b are the angles (degrees) subtended at the satellite by the major and minor axes of the elliptical cross-section of the beam; an antenna efficiency of 55% was assumed.

However, in implementing their assignments, administrations can choose beams other than elliptical, as described in Annex 2 to this Appendix, subject to successful application of the modification procedure of this Appendix.

### 3.13.2	Minimum beamwidth of transmitting antenna

A minimum value of 0.6° for the half-power beamwidth of a transmitting antenna has been adopted for planning for Regions 1 and 3, and 0.8° for Region 2.

### 3.13.3	Transmitting antenna reference patterns

The reference patterns for the co-polar and cross-polar components of satellite transmitting antennas used in preparing the Plans are given in Fig. 9 for Regions 1 and 3, and in Fig. 10 for Region 2.

Curve A:	co-polar component (dB relative to main beam gain)

for	0	≤	φ  ≤  1.58 φ0

−30			for	1.58 φ0	<	φ  ≤  3.16 φ0

for			φ  >  3.16 φ0

after intersection with Curve C: as Curve C

Curve B:	cross-polar component (dB relative to main beam gain)

for	0	≤	φ  ≤  0.33 φ0

−33			for	0.33 φ0	<	φ  ≤  1.67 φ0

for			φ  >  1.67 φ0

after intersection with Curve C: as Curve C

Curve C:	minus the on-axis gain (Curve C in this figure illustrates the particular case of an antenna with an on-axis gain of 43 dBi).

Curve A:	co-polar component (dB relative to main beam gain)

for               0	≤	(φ/φ0)  ≤  1.45

for				(φ/φ0)  >  1.45

after intersection with Curve C: Curve C

Curve B:	cross-polar component (dB relative to main beam gain)

−30	for               0	≤	(φ/φ0)  ≤  2.51

after intersection with co-polar pattern: co-polar pattern

Curve C:	minus the on-axis gain (Curve C in this figure illustrates the particular case of an antenna with an on-axis gain of 46 dBi).

In Region 2, when it was necessary to reduce interference, the pattern shown in Fig. 11A was used; this use is indicated in the Plan by an appropriate symbol. This pattern is derived from an antenna producing an elliptical beam with fast roll-off in the main lobe assuming a “beamlet” half-power beamwidth of 0.8o. For Regions 1 and 3, the pattern shown in Fig. 11B, based on a “beamlet” beamwidth of 0.6o was used. Curves for three different values of φ0 are shown as examples in Fig. 11A and in Fig. 11B.

Curve A:	co-polar component (dB relative to main beam gain)

for  0  ≤  (φ/φ0)  ≤  0.5

for  0.5  <  (φ/φ0)  ≤

−25.23	for    <  (φ/φ0)  ≤  1.45

for  (φ/φ0)  >  1.45

after intersection with Curve C: Curve C

Curve B:	cross-polar component (dB relative to main beam gain)

−30	for  0  ≤  (φ/φ0)  <  2.51

after intersection with co-polar pattern: co-polar pattern

Curve C:	minus the on-axis gain (Curves A and C represent examples of three antennas having different values of φ0 as labelled in Fig. 11A. The on-axis gains of these antennas are approximately 34, 40 and 46 dBi, respectively).

where:

φ :	off-axis angle (degrees)

φ0 :	dimension of the minimum ellipse fitted around the downlink service area in the direction of interest (degrees)

Bmin = 0.8° for Region 2 and Bmin = 0.6° for Regions 1 and 3

in Region 2

in Regions 1 and 3

The difference in performance between the fast roll-off satellite transmitting antenna and the reference satellite transmitting antenna for Regions 1 and 3 is shown in Fig. 12.

The improved fast roll-off satellite transmitting antenna pattern described in Recommendation ITU-R BO.1445 (see Fig. 13) has been used in the planning at WRC-2000.     (WRC-2000)

Curve A:	co-polar relative gain (dB relative to main beam gain):

where:

for  0  ≤  (φ/φ0)  ≤  0.5

for 0.5  <  (φ/φ0)  ≤ 	(WRC-2000)

for   <  (φ/φ0)  ≤  1.45	(WRC-03)

for  (φφ0)    1.45

after intersection with Curve C

for	0	≤	φ  ≤  1.58 φ0

for	1.58 φ0	<	φ  ≤  3.16 φ0

for			φ    3.16 φ0

after intersection with Curve C

Curve B:	cross-polar relative gain (dB):

for	0	≤	φ  ≤  0.33 φ0

−33			for	0.33 φ0	<	φ  ≤  1.67 φ0

for 			φ    1.67 φ0

after intersection with Curve C

Curve C:	minus the on-axis gain (Curve C in this Figure illustrates the particular case of an antenna with an on-axis gain of 42.8 dBi)

where:

φ:	off-axis angle (degrees)

φ0:	cross-sectional half-power beamwidth in the direction of interest (degrees)

Bmin:	0.6°	for Regions 1 and 3

(WRC-2000)

### 3.13.4	Composite beam

A composite beam represents a single beam (i.e. “simulated shaped beam”) and is formed by combining two or more elliptical beams at a given orbital position. In general, composite beams were used at WRC-2000 for administrations which had more than one beam at a given orbital position in the WRC-97 Regions 1 and 3 Plan.     (WRC-2000)

## 3.14	Satellite antenna pointing accuracy

3.14.1	The deviation of the antenna beam from its nominal pointing direction must not exceed a limit of 0.1° in any direction. Moreover, the angular rotation of a transmitting beam about its axis must not exceed a limit of 1°; the limit on rotation is not necessary for beams of circular cross-section using circular polarization51.

3.14.2	The following factors contribute to the total variation in the area on the surface of the Earth illuminated by the satellite beam:

–	variations in satellite station-keeping;

–	the variations caused by the pointing tolerances, which become more significant for coverage areas with low angles of elevation;

–	the effect of the yaw error, which increases as the beam ellipse lengthens.

3.14.3	The effect of these possible variations should be assessed on a case-by-case basis, since their total effect on the area covered will vary with the geometry of the satellite beam, and it would not be reasonable to indicate a single value of shift in the area covered for all situations.

3.14.4	If linear polarization is used for an emission, yaw error makes a significant contribution to increasing the transmitted cross-polarized component; this increases the interference with other carriers which were originally cross-polarized with the emission in question.

## 3.15	Limitation of output power in the satellite transmitter

The output power of a space station transmitter in the broadcasting-satellite service must not rise by more than 0.25 dB relative to its nominal value throughout the life of the satellite.

## 3.16	Power flux-density at edge of coverage area

The original 1977 broadcasting-satellite service Plan used the following values52 of the power flux-density at the edge of the coverage area exceeded for 99% of the worst month:

−103 dB(W/m2) for individual reception in Regions 1 and 3;

−107 dB(W/m2) for individual reception in Region 2 for 24 MHz, as well as for 27 MHz with respect to the cases mentioned in the footnote to § 3.8.

−111 dB(W/m2) for community reception in Regions 1 and 3.

The 1997 revision of the Regions 1 and 3 Plan was generally based on a uniform value of the power flux-density at the edge of coverage area equal to −108 dB(W/m2). This corresponds to the general reduction in e.i.r.p. of 5 dB referenced to the average e.i.r.p. of 63.9 dBW in the 1977 broadcasting-satellite service Plan.

## 3.17	Difference between the e.i.r.p. directed towards the edge of the coverage area and that on the axis of the beam

For planning, the absolute value of the difference between the e.i.r.p. directed towards the edge of the coverage area and that on the axis of the beam should preferably be 3 dB.

If the beam area is larger than the coverage area, the value will be less than 3 dB.

## 3.18	Use of energy dispersal

For planning, an energy dispersal value has been adopted which reduces by 22 dB the spectral power flux-density measured in a 4 kHz bandwidth in relation to that measured in the entire bandwidth: For frequency-modulated television signals, this reduction corresponds to a peak-to-peak deviation of 600 kHz. Digital modulation can achieve appropriate energy dispersal by proper implementation of digital modulation (e.g. by applying spectrum scrambling and/or interleaving).

## 3.19	Orbital separation limit for interference calculation

WRC-2000 has adopted the use of an orbital separation limit for interference calculation in Regions 1 and 3. Beyond this limit no interference was taken into account.

Initially, the values used for the orbital separation limit were 15° for co-polar and 9° for cross-polar emissions. At a later stage, the unique value of the orbital separation limit of 9° was adopted by WRC-2000.     (WRC-2000)

ANNEX  653     (WRC-03)

Criteria for sharing between services

Part A  –  Technical bases for the criteria for interregional sharing
between space services in Annexes 1 and 4

The revised interregional sharing criteria in the bands governed by Appendix 30 are based nominally on the following assumptions.

# 1	Reference assumptions regarding earth station antenna patterns

1.1	For earth station antennas with diameters between 0.45 m and 2.40 m, the gain of the side lobes given by Recommendation ITU-R BO.1213 were used.

For the patterns of earth station antennas with diameters greater than 2.40 m, the gain of the side lobes given by Recommendation ITU-R S.580-5, with a (29 − 25 log ) side-lobe envelope, complemented by the main-lobe given in Annex 3 to Appendix 8, were used.  is the off-axis angle in degrees.

1.2	For the broadcasting-satellite service and fixed-satellite service earth stations, an antenna efficiency of 65% was used at a frequency of 11.7 GHz.

# 2	Antenna diameters and noise temperatures

The range of antenna diameters and associated noise temperatures considered for the protection of the fixed-satellite service and the broadcasting-satellite service on an interregional basis are given in the following table:

| Receive earth station antenna diameter (m) | 0.45 1 | 0.60 | 0.80 | 1.20 | 2.40 | 5 2 | 8 2 | 11 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receive earth station noise temperature (K) | 110 | 110 | 125 | 150 | 150 | 200 | 250 | 250 |
| Total link noise temperature (K) | 174 | 174 | 198 | 238 | 238 | 317 | 396 | 396 |
| (1 This antenna diameter applies in certain cases (see Annexes 1, 3 and 4). (2 This antenna diameter does not apply for broadcasting-satellite service. |  |  |  |  |  |  |  |  |

The total link noise temperature was calculated from the receive earth station noise temperature (which includes the antenna temperature, the receive amplifier temperature and the noise increase resulting from feeder losses), and adding 2 dB to take account of all other sources of noise (uplink noise, geostationary-satellite orbit interference, cross polarization isolation and frequency reuse interference).

# 3	Protection criteria

The power flux-density masks developed in sections 1, 3 and 6 of Annex 1 and in Annex 4 have been determined by setting at 6% the allowable relative noise increase (T/T), for the earth station antenna characteristics given in the above Table.

The allowable interfering power flux-density was calculated by the following expression:

where:

PFDall (θ):	allowable level of interfering power flux-density for an orbital separation of θ°

T/T:	allowable relative increase in receive link noise = 6%

k:	Boltzmann’s constant (1.38 × 10−23 J/K)

T:	receive link noise temperature (K) (see Table in section 2 above)

brf :	reference bandwidth (27 MHz in Regions 1 and 3; 24 MHz in Region 2)

Gm:	gain for a 1 m2 effective aperture (dBi/m2)

Ga(φ):	receive antenna gain for topocentric angle of φ (dBi)

φ:	topocentric angle (degrees) between the interfering and the wanted satellite, as defined in Annex 1 of Appendix 8.

# 4	Power flux-density levels for fixed-satellite service and broadcasting-satellite service with specific antenna diameters

The table below contains power flux-density levels derived for fixed-satellite service and broadcasting-satellite service earth stations with specific antenna diameters for the characteristics defined in sections 1, 2 and 3 above. These levels were used to develop the power flux-density masks in sections 1, 3 and 6 of Annex 1 and in Annex 4 by taking the envelope of the individual pfd masks for the relevant antenna diameters.

| Orbital separation between wanted and interfering space stations (degrees) | Power flux-density level in dB(W/(m2 · 27 MHz)) corresponding to different antenna diameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.45 m 1 | 0.60 m | 0.80 m | 1.20 m | 2.40 cm | 5 m 2 | 8 m 2 | 11 m 2 |
| 0° | −134.2 | −136.7 | −138.7 | −141.4 | −147.4 | −152.5 | −155.7 | −158.4 |
|  0° | For any value of the orbital separation between the wanted and interfering space stations, the applicable power flux-density should be relaxed from the value corresponding to 0° orbital separation by adding the off-axis antenna discrimination, as calculated under the assumptions in section 1 above |  |  |  |  |  |  |  |
| 1 This antenna diameter applies to certain cases (see Annexes 1, 3 and 4). 2 This antenna diameter does not apply for broadcasting satellite service. |  |  |  |  |  |  |  |  |

Part B  –  Sharing criteria used in establishing the WARC SAT-77 Plan

# 1	Protection requirements for sharing between services in the 12 GHz band

1.1	The establishment of sharing criteria for the different services using the 12 GHz band should be based on the protection requirements listed in the table below.

1.2	The values given as “total acceptable” are those necessary to protect the wanted signal. The “single entry” values are those which should be used as a guide for determining sharing criteria. The total interference from all sources must be calculated, since satisfying the “single entry” criteria for each source may not guarantee that the total interference meets the above protection requirements. A “single entry” is defined as the aggregate of emissions from any one station entering any receiver in the wanted service within the channel to be protected.

1.3	The carrier-to-interference ratio (C/I) refers to the ratio of the wanted-to-interfering power at the affected ground station. The value given shall be exceeded for 80% of the worst month for the fixed-satellite service, and for 99% of the worst month for the broadcasting service and the broadcasting-satellite service.

1.4	The term N refers to the post-demodulation noise power at a point of 0 dBm0 relative test tone level in any voice channel of an FDM/FM telephony system. The value given shall not be exceeded for 80% of the worst month.

1.5	The specified values of protection ratio (i.e., the carrier-to-interference power ratio corresponding to a specified picture quality) are applicable, for planning purposes, to television signals of any of the several television standards.

|  |  |  |  | Protection requirements2 |  |
| --- | --- | --- | --- | --- | --- |
| Wanted service1 | Wanted signal1 | Interfering service1 | Interfering signal1 | Total acceptable3 | Single entry |
| BSS | TV/FM | BSS, FSS, FS, BS | TV/FM | C/I = 30 dB 4, 7 | C/I = 35 dB 4 |
| FSS | FDM/FM | BSS | TV/FM | N = 500 pW0p 8 | N = 300 pW0p |
| FSS | TV/FM | BSS, FSS | TV/FM | C/I = 32 dB 5 | C/I = 37 dB 5 |
| FSS | 4φ-PSK | BSS, FSS | TV/FM | C/I = 30 dB | C/I = 35 dB |
| FSS | FDM/FM | FSS | FDM/FM | N = 1 000 pW0p | N = 400 pW0p |
| FS | FDM/FM | BSS | TV/FM | N = 1 000 pW0p | −125 dB(W/(m2 · 4 kHz)) 6 |
| BS | TV/VSB | BSS | TV/FM | C/I = 50 dB | Not applicable |

1 BSS  broadcasting-satellite service FM  frequency modulation FSS  fixed-satellite service FDM  frequency division multiplex BS  broadcasting service 4φ-PSK  quadraphase shift keying FS  fixed service VSB  vestigial sideband. TV  television 2 These limits include both up-link and down-link contributions. 3 Values (dB) are protection ratios for the sum of interfering signals. Values (pW0p) represent interference noise in the worst telephone channels caused by the sum of interfering signals. 4 For BSS satellites located at the interfaces of the Regions 1 and 3 Plan and the Region 2 Plan, the C/I ratios should be 1 dB higher. 5 See Recommendation ITU-R S.483-3. 6 This value may be suitably modified for tropical regions to take account of rain attenuation. Allowance may also be made for polarization discrimination. 7 C/I  ratio of carrier-to-interfering signal. 8 N  noise power.

1.6	For broadcasting-satellite service systems with FM/TV as the wanted signal, the protection ratios are given for particular reference conditions, the most important of which are:

a)	frequency deviation of the wanted signal (12 MHz peak-to-peak);

b)	quality of the wanted service (grade 4.5)54;

c)	co-channel carriers (no carrier-frequency offset).

1.7	If system design is based on conditions other than those of § a) and b) above, the FM/TV protection ratio is given by:

dB

where:

Dv 	nominal peak-to-peak frequency deviation (MHz);

Q 	the impairment grade, concerning the interference only.

1.8	When carriers are offset in frequency, condition of § c) does not apply and the adjacent channel protection ratios should be adjusted according to the frequency offset as shown in Fig. 1. For example, at a frequency offset of 20 MHz, the total acceptable ratio of protection against interference to an FM/TV signal from another FM/TV signal is 13 dB. The corresponding “single entry” value is 18 dB.

# 2	Reference antenna diameter for a fixed-satellite earth station to be used in calculating interference from space stations in the broadcasting-satellite service

2.1	For antennas larger than 100  (2.5 m) in the fixed-satellite service, the gain of the side-lobes is given by the expression 32 − 25 log θ, where θ is the angle from the boresight (Recommendation ITU-R S.465-5). The side-lobe gain is independent of antenna diameter.

2.2	However, in the case of transmitting earth stations, the level of interference radiated into the up-link of other satellite systems would be inversely proportional to the square of the antenna diameter. In this case, the interference decreases with increasing antenna diameter. Since the 11.7-12.2 GHz band is only assigned in the space-to-Earth direction in the fixed-satellite service, this point is not of direct concern to the broadcasting-satellite service.

2.3	Hence it does not appear appropriate, for antenna diameters greater than 100 , to specify a minimum antenna diameter for receiving earth stations in the fixed-satellite service sharing the band 11.7-12.2 GHz. It may be useful to consider a 4.5 m antenna having an efficiency of 60% and an on-axis gain of 53 dB as typical for the purpose of planning the sharing of this band.

# 3	Use of energy dispersal in the broadcasting-satellite service

3.1	Artificial energy dispersal is useful in promoting sharing between the broadcasting-satellite service and the other services to which the band is also allocated.

3.2	Such energy dispersal is achieved by the addition at baseband of a triangular waveform to the video signal to form a composite baseband which, in turn, is used to frequency-modulate the up-link carrier. The frequency of the triangular waveform is usually synchronized at a sub-multiple of the television frame frequency. Typical frequencies range from 12.5 Hz to 30 Hz.

3.3	The Table below gives the relative reduction in spectral power flux-density in a 4 kHz bandwidth as a function of the peak-to-peak deviation due to the energy dispersal signal. This Table is based on the following equation:

Relative reduction (dB) in a 4 kHz band  =

where:

Δ Fpp 	peak-to-peak deviation due to the energy dispersal signal (kHz);

 frms 	rms deviation due to “natural” energy dispersal (kHz).

In compiling the table below, a value of 40 kHz has been assumed for  frms, on the basis of the value of 10 dB for “natural” dispersion given in Table IV of ex-CCIR Report 631* (Rev.76).

Reduction of spectral power flux-density relative 
to a 4 kHz bandwidth

| Peak-to-peak deviation (kHz) | Relative reduction (dB) |
| --- | --- |
| 0 100 200 300 400 500 600 700 800 900 1 000 | 10 15.44 17.78 19.29 20.41 21.30 22.04 22.67 23.22 23.71 24.15 |

3.4	The value of energy dispersal for the broadcasting-satellite service has been determined such that the spectral power flux-density measured in a 4 kHz bandwidth is reduced by 22 dB relative to that measured in the entire bandwidth; this reduction corresponds to a peak-to-peak deviation of 600 kHz.

ANNEX  7     (Rev.WRC-03)

Orbital position limitations

A	In applying the procedure of Article 4 for proposed modifications to the Region 2 Plan or for proposed new or modified assignments in the Regions 1 and 3 List, administrations should observe the following criteria:

1)	No broadcasting satellite serving an area in Region 1 and using a frequency in the band 11.7-12.2 GHz shall occupy a nominal orbital position further west than 37.2° W or further east than 146° E.

2)	No broadcasting satellite serving an area in Region 2 that involves an orbital position different from that contained in the Region 2 Plan shall occupy a nominal orbital position:

a)	further east than 54° W in the band 12.5-12.7 GHz; or

b)	further east than 44° W in the band 12.2-12.5 GHz; or

c)	further west than 175.2° W in the band 12.2-12.7 GHz.

However, modifications necessary to resolve possible incompatibilities during the incorporation of the Regions 1 and 3 feeder-link Plan into the Radio Regulations shall be permitted.

3)	The purpose of the following orbital position and e.i.r.p. limitations is to preserve access to the geostationary-satellite orbit by the Region 2 fixed-satellite service in the band 11.7-12.2 GHz. Within the orbital arc of the geostationary-satellite orbit between 37.2° W and 10° E, the orbital position associated with any proposed new or modified assignment in the Regions 1 and 3 List of additional uses shall lie within one of the portions of the orbital arc listed in Table 1. The e.i.r.p. of such assignments shall not exceed 56 dBW, except at the positions listed in Table 2.

TABLE  1

Allowable portions of the orbital arc between 37.2°W and 10° E for new or modified 
assignments in the Regions 1 and 3 Plan and List

| Orbital position | 37.2° W to 36° W | 33.5°W to 32.5° W | 30° W to 29° W | 26° W to 24° W | 20° W to 18° W | 14° W to 12° W | 8° W to 6° W | 4° W 1 | 2° W to 0° | 4° E to 6° E | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the power flux-density limit −138 dB(W/(m2 · 27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

TABLE  2

Nominal positions in the orbital arc between 37.2° W and 10° E at which the 
e.i.r.p. may exceed the limit of 56 dBW

| Orbital position | 37° W 0.2° | 33.5° W | 30° W | 25° W 0.2° | 19° W 0.2° | 13° W 0.2° | 7° W 0.2° | 4° W 1 | 1° W 0.2° | 5° E 0.2° | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the power flux-density limit –138 dB(W/(m2 · 27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

B	The Region 2 Plan is based on the grouping of the space stations in nominal orbital positions of ±0.2° from the centre of the cluster of satellites. Administrations may locate those satellites within a cluster at any orbital position within that cluster, provided they obtain the agreement of administrations having assignments to space stations in the same cluster. (See § 4.13.1 of Annex 3 to Appendix 30A.)

APPENDIX 30A (REV.WRC-15)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz
in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands
14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

(See Articles 9 and 11)     (WRC-03)

TABLE  OF  CONTENTS

Page

Article 1	General definitions		3

Article 2	Frequency bands		4

Article 2A	Use of the guardbands		4

Article 3	Execution of the provisions and associated Plans		5

Article 4	Procedures for modifications to the Region 2 feeder-link Plan or for additional uses in Regions 1 and 3		6

Article 5	Coordination, notification, examination and recording in the Master International Frequency Register of frequency assignments to feeder-link transmitting earth stations and receiving space stations in the fixed-satellite service		18

Article 6	Coordination, notification and recording in the Master International Frequency Register of frequency assignments to receiving terrestrial stations in Regions 1 and 3 in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, and in Region 2 in the band 17.7-17.8 GHz, when frequency assignments to feeder-link transmitting earth stations for the broadcasting-satellite service in conformity with the Regions 1 and 3 feeder-link Plan or the Region 2 feeder-link Plan are involved		24

Page

Article 7	Coordination, notification and recording in the Master International Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Region 1 in the band 17.3-18.1 GHz and in Regions 2 and 3 in the band 17.7-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in Region 2 in the band 17.8-18.1 GHz and to stations in the broadcasting-satellite service in Region 2 in the band 17.3-17.8 GHz when frequency assignments to feeder links for broadcasting-satellite stations in the 17.3-18.1 GHz band in Regions 1 and 3 or in the band 17.3-17.8 GHz in Region 2 are involved		25

Article 8	Miscellaneous provisions relating to the procedures		28

Article 9	Plan for feeder links for the broadcasting-satellite service in the fixed-satellite service in the frequency band 17.3-17.8 GHz in Region 2		29

Article 9A	Plan for feeder links for the broadcasting-satellite service in the fixed-satellite service in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3		65

Article 10	Interference		93

Article 11	Period of validity of the provisions and associated Plans		93

ANNEXES

Annex 1	Limits for determining whether a service of an administration is considered to be affected by a proposed modification to the Region 2 feeder-link Plan or by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List or when it is necessary under this Appendix to seek the agreement of any other administration		93

Annex 2	Basic characteristics to be furnished in notices relating to feeder-link stations in the fixed-satellite service operating in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz		96

Annex 3	Technical data used in establishing the provisions and associated Plans and Regions 1 and 3 feeder-link List, which should be used for their application		96

Annex 4	Criteria for sharing between services		136

ARTICLE 1     (Rev.WRC-03)

General definitions

1	For the purposes of this Appendix, the following terms shall have the meanings defined below:

1.1	Regions 1 and 3 feeder-link Plan: The Plan for feeder links in the frequency bands 14.5-14.8 GHz3 and 17.3-18.1 GHz for the broadcasting-satellite service in Regions 1 and 3 contained in this Appendix.

1.2	Region 2 feeder-link Plan: The Plan for feeder links in the frequency band 17.3-17.8 GHz for the broadcasting-satellite service in Region 2 contained in this Appendix, together with any modifications resulting from the successful application of the procedure of Article 4.

1.3	Frequency assignment in conformity with the Plan:

–	any frequency assignment for a receiving space station or transmitting earth station which appears in the Regions 1 and 3 feeder-link Plan; or

–	any frequency assignment for a receiving space station or transmitting earth station which appears in the Region 2 feeder-link Plan or for which the procedure of Article 4 has been successfully applied.

1.4	1983 Conference: Regional Administrative Radio Conference for the Planning in Region 2 of the Broadcasting-Satellite Service in the Frequency Band 12.2-12.7 GHz and Associated Feeder-links in the Frequency Band 17.3-17.8 GHz, called in short Regional Administrative Conference for the Planning of the Broadcasting-Satellite Service in Region 2 (Geneva, 1983) (RARC Sat-R2).

1.5	1985 Conference: First Session of the World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilizing It (Geneva, 1985), called in short WARC Orb-85.

1.6	1988 Conference: Second Session of the World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilizing It (Geneva, 1988), called in short WARC Orb-88.

1.7	1997 Conference: World Radiocommunication Conference (Geneva, 1997), called in short WRC-97.

1.8	2000 Conference: World Radiocommunication Conference (Istanbul, 2000), called in short WRC-2000.

1.9	Additional use in Regions 1 and 3: For the application of the provisions of this Appendix, additional uses in Regions 1 and 3 are:

a)	use of assignments with characteristics different from those appearing in the Regions 1 and 3 feeder-link Plan and which are capable of causing more interference than the corresponding entries in that Plan;

b)	use of assignments in addition to those appearing in the Regions 1 and 3 feeder-link Plan.

1.10	Regions 1 and 3 feeder-link List of additional uses (hereafter called in short the “feeder-link List”): The list of assignments for additional uses in Regions 1 and 3 as established by WRC-2000 (see Resolution 542 (WRC-2000)*), as updated following the successful application of the procedure of § 4.1 of Article 4.     (WRC-03)

1.11	Frequency assignment in conformity with the feeder-link List: Any frequency assignment which appears in the feeder-link List as updated following successful application of § 4.1 of Article 4.     (WRC-03)

1.12	Broadcasting-satellite service (BSS) feeder link subject to one of the Plans: The BSS feeder-link subject to one of the Plans referred to in this Appendix is the BSS feeder link in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3 and 17.3-17.8 GHz in Region 2.     (WRC-03)

ARTICLE 2     (WRC-03)

Frequency bands

2.1	The provisions of this Appendix apply to the feeder-links in the fixed-satellite service (Earth-to-space) in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz for the broadcasting-satellite service in Regions 1 and 3, and 17.3-17.z8 GHz for the broadcasting-satellite service in Region 2 and to other services to which these bands are allocated in Regions 1, 2 and 3 so far as their relationship to the fixed-satellite service (Earth-to-space) in these bands is concerned.

2.2	(SUP - WRC-03)

ARTICLE 2A     (Rev.WRC-15)

Use of the guardbands

2A.1	The use of the guardbands defined in § 3.1 and 4.1 of Annex 3 to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks for the broadcasting-satellite service (BSS) feeder link is not subject to the application of Section I of Article 9.

2A.1.1	Coordination between assignments intended to provide the space operation functions and assignments of the BSS feeder link subject to a Plan shall be effected using the provisions of Article 7.

2A.1.2	Coordination among assignments intended to provide the space operation functions and services not subject to a Plan shall be effected using the provisions of Nos. 9.7, 9.17, 9.17A, 9.18, and the associated provisions of Section II of Article 9, as appropriate.

2A.1.3	Coordination of modifications to the Region 2 feeder-link Plan or assignments to be included in the Regions 1 and 3 feeder-link List, with assignments intended to provide these functions shall be effected using § 4.1.1 d) of Article 4.

2A.1.4	Requests for the coordination referred to in § 2A.1.1, 2A.1.2 and 2A.1.3 shall be sent by the requesting administration to the Bureau, together with the appropriate information listed in Appendix 4.

2A.2	Any assignment intended to provide these functions in support of a geostationary-satellite network for the BSS feeder link shall be notified under Article 11 and brought into use within the following time-limits3bis:      (WRC-15)

2A.2.1	a)	for the case where the associated BSS feeder-link assignments are contained in one of the initial Plans (Region 2 Plans incorporated in the Radio Regulations at WARC Orb-85 and Regions 1 and 3 Plan adopted at WRC-2000), within the regulatory time-limit referred to in § 4.1.3 or 4.2.6 of Article 4 from the date of receipt by the Bureau of the complete Appendix 4 data for those assignments intended to provide the space operation functions;

2A.2.2	b)	for the case where the associated BSS feeder-link assignments have been submitted under § 4.1.3 or § 4.2.6 of Article 4 for entry in the Regions 1 and 3 List or a modification to the Region 2 Plan, within the regulatory time-limit referred to in § 4.1.3 or § 4.2.6 of Article 4 for those associated BSS feeder-link assignments;

2A.2.3	c)	for the case where the associated BSS feeder-link assignments have already been brought into use in accordance with the Radio Regulations, within the regulatory time-limit referred to in § 4.1.3 and § 4.2.6 of Article 4 from the date of receipt by the Bureau of the complete Appendix 4 data for those assignments intended to provide these space operation functions.

ARTICLE 3     (Rev.WRC-03)

Execution of the provisions and associated Plans

3.1	The Member States in Regions 1, 2 and 3 shall adopt, for their feeder-link space and earth stations in the fixed-satellite service  in the frequency bands referred to in this Appendix, the characteristics specified in the appropriate Regional Plan and the associated provisions.

3.2	The Member States shall not change the characteristics specified in the Region 1 and 3 feeder-link Plan or in the Region 2 feeder-link Plan, or bring into use assignments to receiving space stations or transmitting earth stations in the fixed-satellite service or to stations of the other services to which these frequency bands are allocated, except as provided for in the Radio Regulations and the appropriate Articles and Annexes of this Appendix.

3.3	The procedures for the use of interim systems in Region 2 for feeder links in the fixed-satellite service for the bands covered by this Appendix are given in Resolution 42 (Rev.WRC-03)*.     (WRC-03)

3.4	The Regions 1 and 3 feeder-link Plan is based on national coverage from the geostationary-satellite orbit. The associated procedures contained in this Appendix are intended to promote long-term flexibility of the Plan and to avoid monopolization of the planned bands and orbit by a country or a group of countries.

ARTICLE 4     (Rev.WRC-15)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

## 4.1	Provisions applicable to Regions 1 and 3

4.1.1	An administration proposing to include a new or modified assignment in the feeder-link List shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations4, 5:

a)	of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is included in the Regions 1 and 3 feeder-link Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to that Plan have already been received by the Bureau in accordance with the provisions of § 4.2.6 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

d)	having a feeder-link frequency assignment in the frequency band 17.8-18.1 GHz in Region 2 in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service, or a frequency assignment in the frequency band 14.5-14.75 GHz in countries listed in Resolution 163 (WRC-15) and in the frequency band 14.5-14.8 GHz in countries listed in Resolution 164 (WRC-15), in the fixed-satellite service (Earth-to-space) not subject to a Plan, which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 9.7, or under § 7.1 of Article 7, with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment.     (WRC-15)

4.1.2	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.1.3	An administration, or one6 acting on behalf of a group of named administrations, intending to include a new or modified assignment in the feeder-link List shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the feeder-link List shall lapse if it is not brought into use within eight years after the date of receipt by the Bureau of the relevant complete information. A proposed new or modified assignment not included in the List within eight years after the date of receipt by the Bureau of the relevant complete information7 shall also lapse.     (WRC-07)

4.1.3bis	The regulatory time-limit for bringing into use an assignment in the List may be extended once by not more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use; or

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit8. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 4.1.3:

–	date of launch failure;

–	due diligence information as required in Resolution 49 (Rev.WRC-15) for the assignment with respect to the satellite that suffered the launch failure, if that information has not already been provided.

If, 11 months after the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information, the Bureau shall promptly send a reminder to the notifying administration. If, within one year of the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information for the new satellite under procurement, the related frequency assignments shall lapse.     (WRC-15)

4.1.4	If the information received by the Bureau under § 4.1.3 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected. The Bureau shall publish9, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks, and the corresponding feeder-links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send a telegram/fax to the administration proposing the assignment, drawing its attention to the information contained in the relevant BR IFIC.     (WRC-07)

4.1.6	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of the BR IFIC, drawing their attention to the information it contains.     (WRC-07)

4.1.7	An administration which considers that it should have been identified in the publication referred to under § 4.1.5 above shall, within four months of the date of publication of its relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration's request, it shall publish an addendum to the publication under § 4.1.5.

4.1.7bis	Except as provided under § 4.1.18 to 4.1.20, any inclusion of a new or modified frequency assignment in the Regions 1 and 3 List which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.     (WRC-03)

4.1.8	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.1.9	Comments from administrations identified in the publication referred to under § 4.1.5 above shall be sent to the Bureau and to the administration proposing the modification.      (WRC-15)

4.1.10	An administration that has not notified its agreement either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.1.5 shall be deemed to have not agreed to the proposed assignment unless the provisions of § 4.1.10a to 4.1.10d and § 4.1.21 are applied. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months, or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.    (WRC-15)

4.1.10bis	Thirty days prior to the expiry of the same four-month period, the Bureau shall dispatch a reminder telegram or fax to an administration which has not made its comments under § 4.1.10, bringing the matter to its attention.     (WRC-03)

4.1.10ter	(SUP - WRC-15)

4.1.10a	After the same time period as specified in § 4.1.10, the notifying administration may, pursuant to § 4.1.21, request the Bureau to assist in respect of an administration which has not replied within this time period.     (WRC-15)

4.1.10b	The Bureau, acting under § 4.1.10a, shall send a reminder to the administration which has not replied, together with the results of its previously published compatibility analysis, containing the change in the values referred to in paragraph 4 of Annex 1 to Appendix 30A, requesting a decision.     (WRC-15)

4.1.10c	Fifteen days before the expiry of the 30-day period referred to in § 4.1.10d, the Bureau shall send a reminder to the above-mentioned administration drawing its attention to the consequence of no reply.     (WRC-15)

4.1.10d	If no decision is communicated to the Bureau within 30 days after the date of dispatch of the reminder under § 4.1.10b, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.     (WRC-15)

4.1.11	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.1 and the subsequent procedure in cases where:

–	the assignments of any other administration received by the Bureau in accordance with § 4.1.3 or § 4.2.6, or § 2A.1.4 of Article 2A, or § 7.1 of Article 7, or No. 9.7 before this modified proposal is received under § 4.1.12; or

–	the assignments of any other administration contained in the Plans or the Lists,

are considered as being affected and receive more interference as a result of the modifications than that produced by the initial proposal.     (WRC-15)

4.1.12	If agreement has been reached with the administrations identified in the publication referred to under § 4.1.5 above, the administration proposing the new or modified assignment may continue with the appropriate procedure in Article 5 and shall inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.      (WRC-15)

4.1.12bis	In application of § 4.1.12, an administration may indicate the changes to the information communicated to the Bureau under § 4.1.3 and published under § 4.1.5.     (WRC-03)

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the List, the assignment in question shall be maintained in the List until the end of the period referred to in § 4.1.3 above. After that date this assignment shall lapse unless the agreement of the administrations affected is renewed.     (WRC-03)

4.1.14	Where the proposed assignment involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.1.15	The Bureau shall publish10 in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the feeder-link List.     (WRC-03)

4.1.16	In case of disagreement on the part of an administration whose agreement has been sought, the requesting administration should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.1.17	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by either one of these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

4.1.18	If, in spite of the application of § 4.1.16 and 4.1.17, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Regions 1 and 3 Plan, or in the Region 2 Plan or for which the procedure of § 4.2 has been initiated, and if the notifying administration insists that the proposed assignment be included in the Regions 1 and 3 feeder-link List, the Bureau shall provisionally enter the assignment in the Regions 1 and 3 feeder-link List with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the feeder-link List only if the Bureau is informed that the new assignment in the Regions 1 and 3 feeder-link List has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.     (WRC-03)

4.1.18bis	When requesting the application of § 4.1.18, the notifying administration shall undertake to meet the requirements of § 4.1.20 and provide to the administration in respect of which § 4.1.18 is applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements. Once an assignment is entered in the feeder-link List provisionally under the provisions of § 4.1.18, the calculation of the equivalent protection margin (EPM)11 of an assignment in the Regions 1 and 3 feeder-link List or for which the procedure of Article 4 has been initiated and which was the basis for the disagreement shall not take into account interference produced by the assignment for which the provisions of § 4.1.18 have been applied.     (WRC-03)

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the feeder-link List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the feeder-link List shall be reviewed accordingly.     (WRC-03)

4.1.20	Should harmful interference be caused by an assignment included in the feeder-link List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the feeder-link List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.     (WRC-03)

4.1.21	An administration may, at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.1.22	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

4.1.23	When a frequency assignment included in the feeder-link List is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the feeder-link List.     (WRC-03)

4.1.24	No assignment in the feeder-link List shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.     (WRC-03)

4.1.25	Where an administration already having included in the feeder-link List two assignments (not including those systems notified on behalf of a group of named administrations and included in the feeder-link List by WRC-2000) in the same channel and covering the same service area, proposes to include in the feeder-link List a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the feeder-link List in the same channel and which proposes to include in the feeder-link List a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-15), this administration shall be deemed to have given its agreement to inclusion in the feeder-link List of the assignment of the latter administration.     (WRC-15)

4.1.26	The procedure of this Article may be applied by the administration of a new ITU Member State in order to include new assignments in the feeder-link List. Upon completion of the procedure, the next world radiocommunication conference may be requested to consider, among the assignments included in the feeder-link List after the successful completion of this procedure, the inclusion in the Regions 1 and 3 feeder-link Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.     (WRC-03)

4.1.27	When an administration has successfully applied this procedure and received all the agreements12 required to include in the feeder-link List assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Regions 1 and 3 feeder-link Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in this Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in this Plan.     (WRC-03)

4.1.27bis	Should the assignments mentioned in § 4.1.26 and 4.1.27 over the national territory of the administration not be brought into use within the regulatory time-limit mentioned in § 4.1.3, they would be retained in the List until the end of the World Radiocommunication Conference following immediately after the successful completion of procedure referred to in § 4.1.26 and 4.1.27, respectively and thereafter they shall be removed from the List.     (WRC-03)

4.1.28	The feeder-link List, as updated, shall be published periodically by the Bureau.     (WRC-03)

4.1.29	New or modified assignments in the feeder-link List shall be limited to digital modulation.     (WRC-03)

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification to the Region 2 feeder-link Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments in the fixed-satellite service which are shown in the Region 2 feeder-link Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 feeder-link Plan a new frequency assignment in the fixed-satellite service; or

c)	to cancel a frequency assignment in the fixed-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5 and Resolution 42 (Rev.WRC-03)*).     (WRC-03)

4.2.2	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 feeder-link Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations13, 14, 15:

a)	having an assignment for feeder-links in the fixed-satellite service (Earth-to-space) which is in conformity with the Regions 1 and 3 feeder-link Plan with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link List or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Region 2 feeder-link Plan or in respect of which proposed modifications to this Plan have been received by the Bureau in accordance with the provisions of § 4.2.6;

d)	which are considered affected.     (WRC-03)

4.2.3	(Not used.)

4.2.4	The services of an administration are considered to be affected when the limits shown in Annex 1 are exceeded.

4.2.5	The agreement referred to in § 4.2.2 is not required when an administration proposes to bring into use, with characteristics appearing in the Region 2 feeder-link Plan, a fixed feeder-link earth station in the band 17.3-17.8 GHz or a transportable feeder-link earth station in the band 17.3-17.7 GHz. Administrations may communicate to the Bureau the characteristics of such earth stations for inclusion in this Plan.

4.2.6	An administration, or one16 acting on behalf of a group of named administrations, intending to make a modification to the Region 2 feeder-link Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use within eight years after the date of receipt by the Bureau of the relevant complete information17. A request for a modification that has not been included in that Plan within eight years after the date of receipt by the Bureau of the relevant complete information17 shall also lapse.     (WRC-07)

4.2.6bis	The regulatory time-limit for bringing into use of an assignment in the Region 2 Plan obtained through application of § 4.2 may be extended once by no more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use; or

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit18. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 4.2.6:

–	date of launch failure;

–	due diligence information as required in Resolution 49 (Rev.WRC-15) for the assignment with respect to the satellite that suffered the launch failure, if that information has not already been provided.

If, 11 months after the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information, the Bureau shall promptly send a reminder to the notifying administration. If, within one year of the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information for the new satellite under procurement, the related frequency assignments shall lapse.     (WRC-15)

4.2.7	If the information received by the Bureau under § 4.2.6 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided.

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.2. The Bureau shall publish19, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks, and the corresponding feeder links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send a telegram/fax to the administration proposing the modification to the Region 2 feeder-link Plan, drawing its attention to the information contained in the relevant BR IFIC.     (WRC-07)

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC, drawing their attention to the information it contains.     (WRC-07)

4.2.10	An administration which considers that it should have been included in the publication referred to under § 4.2.8 above shall, within four months of the date of publication in the relevant BR IFIC, and giving the technical reasons for so doing, request the Bureau to include its name in the publication. The Bureau shall study this information on the basis of Annex 1 and shall inform both administrations of its conclusions. Should the Bureau agree to the administration’s request, it shall publish an addendum to the publication under § 4.2.8.     (WRC-07)

4.2.11	Except as provided under § 4.2.21A to 4.2.21D, any modification to a frequency assignment which is in conformity with the Region 2 feeder-link Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all affected administrations.     (WRC-03)

4.2.12	The administration seeking agreement or the administration with which agreement is sought may request any additional technical information it considers necessary. The administrations shall inform the Bureau of such requests.

4.2.13	Comments from administrations on the information published pursuant to § 4.2.8 should be sent either directly to the administration proposing the modification or through the Bureau. In any event, the Bureau shall be informed that comments have been made.

4.2.14	An administration which has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of the BR IFIC referred to in § 4.2.8 shall be deemed to have agreed to the proposed modification. This time-limit may be extended by up to three months for an administration which has requested additional information under § 4.2.12 or for an administration which has requested the assistance of the Bureau under § 4.2.22. In the latter case, the Bureau shall inform the administrations concerned of this request.

4.2.14bis	Thirty days prior to the expiry of the same four-month period the Bureau shall dispatch a reminder telegram or fax to an administration which has not made its comments under § 4.2.14, bringing the matter to its attention.     (WRC-03)

4.2.14ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations whose agreements are required for completion of the procedure of Article 4.     (WRC-03)

4.2.15	If, in seeking agreement, an administration modifies its initial proposal, it shall again apply the provisions of § 4.2 and the consequent procedure with respect to any other administration whose services might be affected as a result of modifications to the initial proposal.

4.2.16	If no comments have been received on the expiry of the periods specified in § 4.2.14, or if agreement has been reached with the administrations which have made comments and with which agreement is necessary, the administration proposing the modification may continue with the appropriate procedure in Article 5, and shall so inform the Bureau, indicating the final characteristics of the frequency assignment together with the names of the administrations with which agreement has been reached.

4.2.16bis	In application of § 4.2.16, an administration may indicate the changes to the information communicated to the Bureau under § 4.2.6 and published under § 4.2.8.     (WRC-03)

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the Plan, the assignment in question shall be maintained in the Plan until the end of the period referred to in § 4.2.6 above. After that date this assignment in the Plan shall lapse unless the agreement of the administrations affected is renewed.     (WRC-03)

4.2.18	When the proposed modification to the Region 2 feeder-link Plan involves developing countries, administrations shall seek all practicable solutions conducive to the economical development of the broadcasting-satellite systems of these countries.

4.2.19	The Bureau shall publish20 in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 feeder-link Plan and will be considered as a frequency assignment in conformity with that Plan.     (WRC-03)

4.2.20	When an administration proposing to modify the characteristics of a frequency assignment or to make a new frequency assignment receives notice of disagreement on the part of an administration whose agreement it has sought, it should first endeavour to solve the problem by exploring all possible means of meeting its requirement. If the problem still cannot be solved by such means, the administration whose agreement has been sought should endeavour to overcome the difficulties as far as possible, and shall state the technical reasons for any disagreement if the administration seeking the agreement requests it to do so.

4.2.21	If no agreement is reached between the administrations concerned, the Bureau shall carry out any study that may be requested by these administrations; the Bureau shall inform them of the result of the study and shall make such recommendations as it may be able to offer for the solution of the problem.

4.2.21A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Region 2 feeder-link Plan, or in the Regions 1 and 3 feeder-link Plan or List, or for which the procedure of § 4.1 or 4.2 has been initiated, and if the notifying administration insists that the proposed assignment be included in the Region 2 feeder-link Plan, the Bureau shall provisionally enter the assignment in the Region 2 feeder-link Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Region 2 feeder-link Plan only if the Bureau is informed that the new or modified assignment in the Region 2 feeder-link Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.     (WRC-03)

4.2.21B	When requesting the application of § 4.2.21A, the notifying administration shall undertake to meet the requirements of § 4.2.21D and provide to the administration in respect of which § 4.2.21A has been applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements.     (WRC-03)

4.2.21C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44, the status of the assignment in the Region 2 feeder-link Plan shall be reviewed accordingly.     (WRC-03)

4.2.21D	Should harmful interference be caused by an assignment included in the Region 2 feeder-link Plan under § 4.2.21A to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the Region 2 feeder-link Plan under § 4.2.21A shall, upon receipt of advice thereof, immediately eliminate this harmful interference.     (WRC-03)

4.2.22	An administration may at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.2.23	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

### 4.2.24	Cancellation of frequency assignments

When a frequency assignment in conformity with the Region 2 feeder-link Plan is no longer required, whether or not as a result of a modification, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the Region 2 feeder-link Plan.

### 4.2.25	Master copy of the Region 2 feeder-link Plan

4.2.25.1	The Bureau shall maintain an up-to-date master copy of the Region 2 feeder-link Plan, including the overall equivalent protection margins of each assignment, taking account of the application of the procedure set out in this Article. This master copy shall contain the overall equivalent protection margins derived from that Plan as established by the 1983 Conference and those derived from all modifications to that Plan as a result of the successful completion of the modification procedure set out in this Article.

4.2.25.2	An up-to-date version of the Region 2 feeder-link Plan shall be published by the Secretary-General when justified by the circumstances.

ARTICLE 5     (Rev.WRC-15)

Coordination, notification, examination and recording in the Master
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service21, 22     (WRC-07)

## 5.1	Coordination and notification

5.1.1	When an administration wishes to determine whether it is possible to use, at a given location, an amount of power control which is in excess of that contained in column 12 of the Regions 1 and 3 feeder-link Plan, it shall request the Bureau to determine the amount of permissible power control (not to exceed 10 dB) from that given location using the procedure contained in § 3.11 of Annex 3.

5.1.2	Whenever an administration23 intends to bring into use a frequency assignment to a transmitting earth station or receiving space station in the fixed-satellite service in the bands between 14.5 GHz and 14.8 GHz and between 17.3 GHz and 18.1 GHz in Regions 1 and 3, and between 17.3 GHz and 17.8 GHz in Region 2, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions.     (WRC-03)

5.1.2bis	Frequency assignments relating to a number of earth stations may be notified in the form of the characteristics of a typical earth station and the intended geographical area of operation. Individual notices of frequency assignments are however necessary in the case of earth stations whose coordination area includes all or part of the territory of another administration.     (WRC-03)

5.1.3	Before an administration in Region 1 or 3 notifies to the Bureau or brings into use any frequency assignment to a specific transmitting feeder-link earth station in the bands 14.5-14.8 GHz and 17.7-18.1 GHz with an e.i.r.p. greater than the sum of the values specified in columns 11 and 12 of the Regions 1 and 3 feeder-link Plan, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station using the method detailed in Appendix 7.     (WRC-03)

5.1.4	Before an administration in Region 1 or 3 notifies to the Bureau or brings into use any frequency assignment to a specific transmitting feeder-link earth station in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station, using the method detailed in Appendix 7, in respect of notices concerning stations of the mobile and fixed services in the bands 14.5-14.8 GHz and 17.7-18.1 GHz and of the fixed-satellite service (space-to-Earth) in the band 17.7-18.1 GHz received by the Bureau prior to 3 June 2000 for recording in the International Master Frequency Register (Master Register) and subsequently recorded with a favourable finding24.     (WRC-03)

5.1.5	If an administration with which coordination is sought under § 5.1.4 does not respond within three months, the administration intending to bring into use a frequency assignment to a feeder-link earth station shall notify this frequency assignment in accordance with § 5.1.2 above.

5.1.6	For any notification under § 5.1.2, an individual notice for each frequency assignment shall be drawn up as prescribed in Appendix 4, the various sections of which specify the basic characteristics to be provided as appropriate. It is recommended that the notifying administration should also supply any other data it may consider useful.

5.1.6bis	In application of § 5.1.2, an administration may identify the characteristics of assignments in the Plans or the List as notification and send to the Bureau the changes thereto.     (WRC-03)

5.1.7	Each notice must reach the Bureau not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Bureau not later than three months before that date.

5.1.8	Any frequency assignment the notice of which reaches the Bureau after the applicable period specified in § 5.1.7 shall, where it is to be recorded, bear a remark in the Master Register to indicate that it is not in conformity with § 5.1.7.

5.1.9	Any notice made under § 5.1.2 which does not contain the characteristics specified in Appendix 4 shall be returned by the Bureau immediately by airmail to the notifying administration with the relevant reasons.

5.1.10	Upon receipt of a complete notice, the Bureau shall include its particulars, with the date of receipt, in its BR IFIC which shall contain the particulars of all such notices received since the publication of the previous Circular.

5.1.11	The Circular shall constitute the acknowledgements to the notifying administration of the receipt of a complete notice.

5.1.12	Complete notices shall be considered by the Bureau in order of receipt. The Bureau shall not postpone its finding unless it lacks sufficient data to reach a decision; moreover, the Bureau shall not act upon any notice which has a technical bearing on an earlier notice still under consideration by the Bureau until it has reached a finding with respect to such earlier notice.

## 5.2	Examination and recording

5.2.1	The Bureau shall examine each notice:

a)	with respect to its conformity with the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to § b), c), d), e) and f) below); and

b)	with respect to its conformity with the appropriate Regional feeder-link Plan or the Regions 1 and 3 feeder-link List, as appropriate; or     (WRC-03)

c)	with respect to the coordination requirements specified in the Remarks column of Article 9 or Article 9A; or

d)	with respect to its conformity with the appropriate Regional feeder-link Plan or the Regions 1 and 3 feeder-link List, however, having characteristics differing from those in this Plan or in the Regions 1 and 3 feeder-link List in one or more of the following aspects:

–	use of a reduced e.i.r.p.,

–	use of a reduced coverage area entirely situated within the coverage area appearing in the Plan or in the Regions 1 and 3 feeder-link List,

–	use of other modulating signals in accordance with the provisions of § 3.1.3 to Annex 5 of Appendix 30,

–	in the case of Region 2, use of an orbital position under the conditions specified in § B of Annex 7 to Appendix 30,

–	in the case of Regions 1 and 3, use of the assignment for transmissions in the fixed-satellite service (Earth-to-space) other than for feeder links to the broadcasting-satellite service provided that such transmissions do not cause more interference, or require more protection from interference, than the feeder-link transmissions operating in conformity with the Plan or the List, as appropriate;     (WRC-03)

e)	for Region 2, with respect to its conformity with the provisions of Resolution 42 (Rev.WRC-03)*;     (WRC-03)

f)	for Regions 1 and 3, with respect to its conformity with the provisions of § 5.1.3 and also its conformity with § 5.1.4 or 5.1.5 relating to coordination.

5.2.2	When the Bureau reaches a favourable finding with respect to § 5.2.1 a), 5.2.1 b), 5.2.1 c) and 5.2.1 f), the frequency assignment of an administration shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations all frequency assignments brought into use in conformity with the feeder-link Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments.     (WRC-07)

5.2.2.1	When the Bureau reaches a favourable finding with respect to § 5.2.1 a), 5.2.1 c), 5.2.1 d) and 5.2.1 f), the frequency assignment shall be recorded in the Master Register. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations, all frequency assignments brought into use in conformity with the feeder-link Plan and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments. When recording these assignments, the Bureau shall indicate by an appropriate symbol the characteristics having a value different from that appearing in that Plan.     (WRC-07)

5.2.2.2	In the case of Region 2, when the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.WRC-03)*. A frequency assignment for which the provisions of Resolution 42 (Rev.WRC-03)* have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Bureau shall be entered in the Master Register. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.WRC-03)* and recorded in the Master Register shall be considered to have the same status irrespective of the dates of receipt entered in the Master Register for such frequency assignments. If the finding with respect to § 5.2.1 e), where applicable, is unfavourable, the notice shall be returned immediately by airmail to the notifying administration.     (WRC-07)

5.2.2.3	In the case of Regions 1 and 3, when the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), the notice shall be returned immediately by airmail to the notifying administration with the Bureau’s reasons for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

5.2.2.4	In the case of Regions 1 and 3, when the Bureau reaches a favourable finding with respect to § 5.2.1 a), 5.2.1 b), 5.2.1 c) and 5.2.1 d) but an unfavourable finding with respect to § 5.2.1 f), the notice shall be returned immediately by airmail to the notifying administration with the Bureau’s reasons for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem. If the unfavourable finding under § 5.2.1 f) is due to the coordination under § 5.1.3 only not being effected, the administration shall undertake only to bring this assignment into use with an e.i.r.p. level not greater than the sum of the values specified in columns 11 and 12 of the Regions 1 and 3 feeder-link Plan.

5.2.2.5	When an assignment is recorded as a result of a favourable finding with respect to § 5.2.1 f), a remark shall be included indicating that coordination has been effected.

5.2.3	Whenever a frequency assignment is recorded in the Master Register, the finding reached by the Bureau shall be indicated.     (WRC-07)

5.2.4	When the Bureau reaches an unfavourable finding with respect to:

–	§ 5.2.1 a), or

–	§ 5.2.1 c), or

–	§ 5.2.1 b) and 5.2.1 d) and, where appropriate, § 5.2.1 e),

the notice shall be returned immediately by airmail to the notifying administration with the Bureau’s reasons for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

5.2.5	When the notifying administration resubmits the notice and the finding of the Bureau becomes favourable with respect to the appropriate parts of § 5.2.1, the notice shall be treated as in § 5.2.2, 5.2.2.1 or 5.2.2.2 as appropriate.

5.2.6	If the notifying administration resubmits the notice without modification and insists on its reconsideration, and if the Bureau’s finding with respect to § 5.2.1 remains unfavourable, the notice is returned to the notifying administration in accordance with § 5.2.4. In this case, the notifying administration undertakes not to bring into use the frequency assignment until the condition specified in § 5.2.5 is fulfilled.

5.2.7	If a frequency assignment notified in advance of bringing into use in conformity with § 5.1.3 has received a favourable finding by the Bureau with respect to the provisions of § 5.2.1, it shall be entered provisionally in the Master Register with a special symbol in the Remarks Column indicating the provisional nature of that entry.

5.2.8	When the Bureau has received confirmation that the frequency assignment has been brought into use, the Bureau shall remove the symbol in the Master Register.

5.2.9	The date of bringing into use notified by the administration concerned shall be recorded in the Master Register.     (WRC-07)

5.2.10	Wherever the use of a frequency assignment to a space station recorded in the Master Register and emanating from the Regions 1 and 3 List is suspended for a period exceeding six months, the notifying administration shall inform the Bureau of the date on which such use was suspended. When the recorded assignment is brought back into use, the notifying administration shall so inform the Bureau, as soon as possible. On receipt of the information sent under this provision, the Bureau shall make that information available on the ITU website as soon as possible and shall publish it in the BR IFIC. The date on which the recorded assignment is brought back into use24bis shall be no later than three years from the date on which the use of the frequency assignment was suspended, provided that the notifying administration informs the Bureau of the suspension within six months from the date on which the use was suspended. If the notifying administration informs the Bureau of the suspension more than six months after the date on which the use of the frequency assignment was suspended, this three-year time period shall be reduced. In this case, the amount by which the three-year period shall be reduced shall be equal to the amount of time that has elapsed between the end of the six-month period and the date that the Bureau is informed of the suspension. If the notifying administration informs the Bureau of the suspension more than 21 months after the date on which the use of the frequency assignment was suspended, the frequency assignment shall be cancelled.     (WRC-15)

5.2.11	If a recorded frequency assignment stemming from the Regions 1 and 3 List is not brought back into use within the suspension period resulting from the application of § 5.2.10 above, the Bureau shall cancel the assignment from the Master Register and the assignment in the List, unless the assignment is one to which § 4.1.26 or § 4.1.27 is being applied.    (WRC-15)

## 5.3	Cancellation of entries in the Master Register

5.3.1	Any notified frequency assignment to which the Article 4 procedures have been applied and which has been provisionally recorded under § 5.2.7 shall be brought into use no later than the end of the period provided under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4. Any other frequency assignment provisionally recorded under § 5.2.7 shall be brought into use by the date specified in the notice. Unless the Bureau has been informed by the notifying administration of the bringing into use of the assignment under § 5.2.8, it shall, no later than 15 days before the notified date of bringing into use or the end of the regulatory period established under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4, as appropriate, send a reminder requesting confirmation that the assignment has been brought into use within the regulatory period. If the Bureau does not receive that confirmation within 30 days following the notified date of bringing into use or the period provided under § 4.1.3, 4.1.3bis, 4.2.6 or 4.2.6bis of Article 4, as the case may be, it shall cancel the entry in the Master Register.     (WRC-15)

5.3.2	If the use of any recorded frequency assignment is permanently discontinued, the notifying administration shall so inform the Bureau within three months, whereupon the entry shall be removed from the Master Register.

ARTICLE 6     (Rev.WRC-15)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to receiving terrestrial stations 
in Regions 1 and 3 in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, and in Region 2 in the band 17.7-17.8 GHz, when frequency assignments to feeder-link transmitting earth stations for the broadcasting-satellite service in 
conformity with the Regions 1 and 3 feeder-link Plan or the 
Region 2 feeder-link Plan25 are involved26

6.1	Administrations planning to implement assignments for terrestrial stations in Regions 1 and 3 in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, and in Region 2 in the band 17.7-17.8 GHz should evaluate the level of interference assessed on the basis of coordination contours calculated in accordance with Appendix 727, which might be caused by a feeder-link earth station located on the territory of another administration and included in the service area of an assignment to a broadcasting-satellite service feeder-link space station which is in conformity with the appropriate regional feeder-link Plan. Should the administration planning terrestrial stations find that interference may be caused by such a feeder-link earth station, it may request the administration responsible for the feeder-link earth station to indicate the geographical coordinates, the antenna characteristics and the horizon elevation angle around its existing and planned feeder-link earth stations.

6.2	In the case of Region 2, when the entry in the feeder-link Plan contains information on specific earth stations, this shall be used in the interference calculations referred to in § 6.1 above. Where such information is not contained in the Region 2 feeder-link Plan, an administration which receives a request under § 6.1 shall, within a period of four months, communicate the details of the feeder-link earth stations to the administration planning the terrestrial station, and to the Bureau in order to update this Plan.

6.3	In the case of Regions 1 and 3, an administration which receives a request under § 6.1 shall, within a period of four months, communicate the details of the feeder-link stations to the administration planning the terrestrial station, and to the Bureau for information.

6.4	If, at the end of the period of four months, the administration responsible for the terrestrial station does not receive a reply, it may request the assistance of the Bureau.

6.5	If the administration responsible for the feeder-link earth station does not communicate to the Bureau, within a period of four months, the information requested under § 6.1, this administration shall only implement its feeder-link earth station provided it does not cause harmful interference to the terrestrial station under consideration.

6.6	If, as a result of the application of this Article, an agreement is reached with the administration responsible for the feeder-link earth station or no comments have been received, the administration responsible for the terrestrial station may notify this station under Article 11 for recording in the Master Register. A remark shall be included indicating either that an agreement has been reached or that no comments have been received.

ARTICLE 7     (Rev.WRC-15)

Coordination, notification and recording in the Master International 
Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Region 1 in the frequency band 17.3-18.1 GHz and 
in Regions 2 and 3 in the frequency band 17.7-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in Region 2 in the frequency band 17.8-18.1 GHz, to stations in the fixed-satellite service (Earth-to-space) in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz when frequency assignments to feeder links for broadcasting-satellite stations in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz  in Regions 1 and 3 or 
in the band 17.3-17.8 GHz in Region 2 are involved28

Section I – Coordination of transmitting space or earth stations in the fixed-satellite 
service or transmitting space stations in the broadcasting-satellite service
with assignments to broadcasting-satellite service feeder links

7.1	The provisions of No. 9.729 and the associated provisions under Articles 9 and 11 are applicable to transmitting space stations in the fixed-satellite service in Region 1 in the frequency band 17.3-18.1 GHz, to transmitting space stations in the fixed-satellite service in Regions 2 and 3 in the frequency band 17.7-18.1 GHz, to transmitting earth stations in the fixed-satellite service in Region 2 in the frequency band 17.8-18.1 GHz, to transmitting earth stations in the fixed-satellite service in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz where those stations are not for feeder links for the broadcasting-satellite service, and to transmitting space stations in the broadcasting-satellite service in Region 2 in the frequency band 17.3-17.8 GHz.     (WRC-15)

7.2	In applying the procedures referred to in § 7.1, the provisions of Appendix 5 are replaced by the following:

7.2.1	The frequency assignments to be taken into account are:

a)	the assignments in conformity with the appropriate Regional feeder-link Plan in Appendix 30A;

b)	the assignments included in the Regions 1 and 3 feeder-link List;

c)	the assignments for which the procedure of Article 4 has been initiated as from the date of receipt of the complete Appendix 4 information under § 4.1.3 or 4.2.6.     (WRC-03)

7.2.2	The criteria to be applied are those given in Annex 4.

7.2bis	In applying the procedures referred to in § 7.1 for FSS frequency assignments in countries listed in Resolution 163 (WRC-15) in the frequency band 14.5-14.75 GHz and in countries listed in Resolution 164 (WRC-15) in the frequency band 14.5-14.8 GHz not for feeder links for the broadcasting-satellite service, the provision of No. 11.41 is replaced by the following provision. No. 11.41.2 continues to apply.     (WRC-15)

7.2bis.1	If, after a notice is returned under No. 11.38, the notifying administration resubmits the notice and insists upon its reconsideration, and the assignment which was the basis of the unfavourable finding is neither an assignment in the Regions 1 and 3 Plan, nor an assignment of definitive recording in the Regions 1 and 3 feeder-link List or one for which the complete Appendix 4 information has been received under § 4.1.12 for the definitive recording by the date of submission pursuant to No. 9.30 of this notice, the Bureau shall enter the assignment in the Master Register with an indication of those administrations whose assignments were the basis of the unfavourable finding, provided that such resubmission is accompanied with a formal commitment indicating that, if unacceptable interference is caused to the Regions 1 and 3 recorded assignments which were the basis of the unfavourable finding, the notifying administration of the FSS shall immediately eliminate this unacceptable interference (see also No. 11.42).     (WRC-15)

Section II – Coordination with assignments in conformity with the
appropriate Regional feeder-link Plan in Appendix 30A

7.3	Administrations planning to implement assignments for receiving earth stations in all Regions in the band 17.7-18.1 GHz in the fixed-satellite service (space-to-Earth) or in the band 17.3-17.8 GHz in the broadcasting-satellite service should evaluate the level of interference, assessed on the basis of coordination contours calculated in accordance with Appendix 7, which might be caused by a feeder-link earth station located on the territory of another administration and included in the service area of an assignment to a broadcasting-satellite service feeder-link space station which is in conformity with the appropriate Regional feeder-link Plan. Should the administration planning receiving earth stations find that interference may be caused by such a feeder-link earth station, it may request the administration responsible for the feeder-link earth station to indicate the geographical coordinates, the antenna characteristics and the elevation angle of the horizon around its existing and planned feeder-link earth stations.

7.4	In the case of Region 2, when the entry in the feeder-link Plan contains information on specific earth stations this shall be used in the interference calculations mentioned in § 7.3 above. Where such information is not contained in this Plan an administration which receives a request under § 7.3 shall, within a period of four months, communicate the details of the feeder-link earth stations to the administration planning the receiving earth station, and to the Bureau in order to update this Plan.

7.5	In the case of Regions 1 and 3, an administration which receives a request under § 7.3 shall, within a period of four months, communicate the details of the feeder-link earth stations to the administration planning the receiving earth station, and to the Bureau for information.

7.6	If, at the end of the period of four months, the administration responsible for the fixed-satellite or broadcasting-satellite receiving earth station(s) does not receive a reply, it may request the assistance of the Bureau.

7.7	If the administration responsible for the feeder-link earth stations does not communicate to the Bureau, within a period of four months, the information requested under § 7.3, this administration shall only implement its feeder-link earth station provided it does not cause harmful interference to the fixed-satellite or broadcasting-satellite earth station(s) under consideration.

7.8	If, as a result of the application of this Article, an agreement is reached with the administration responsible for the feeder-link earth station or no comments have been received, and where the station is recorded in the Master Register in accordance with Article 11, the Bureau shall enter a remark indicating either that an agreement has been reached or that no comments have been received.

Section III – Coordination with assignments in the Regions 1 and 3 feeder-link List,
or for which the procedure of Article 4 has been initiated

7.9	The provisions of No. 9.17A and the associated provisions under Articles 9 and 11 and Appendix 5 are applicable to fixed-satellite service and broadcasting-satellite service receiving earth stations, in respect of frequency assignments to transmitting broadcasting-satellite service feeder-link earth stations, in the fixed-satellite service in the bands 17.3-18.1 GHz in Regions 1 and 3 and 17.3-17.8 GHz in Region 2 which correspond to assignments to receiving broadcasting-satellite service feeder-link space stations already included in the Regions 1 and 3 feeder-link List, or for which the procedure of Article 4 has been initiated, as from the date of receipt of the complete Appendix 4 information.     (WRC-03)

ARTICLE 8

Miscellaneous provisions relating to the procedures*

Section I – Studies and Recommendations

8.1.1	If it is requested by any administration, the Board, using such means at its disposal as are appropriate in the circumstances, shall conduct a study of cases of alleged contravention or non-observance of these provisions, or of harmful interference.

8.1.2	The Board shall thereupon prepare and forward to the administrations concerned a report containing its findings and recommendations for the solution of the problem.

8.1.3	On receiving the Board’s recommendations for the solution of the problem, an administration shall promptly acknowledge the receipt by telegram and shall subsequently indicate the action it intends to take. In cases when the Board’s suggestions or recommendations are unacceptable to the administrations concerned, further efforts should be made by the Board to find an acceptable solution to the problem.

8.1.4	In a case where, as a result of a study, the Board submits to one or more administrations suggestions or recommendations for the solution of a problem, and where no answer has been received from one or more of these administrations within a period of four months, the Board shall consider that the suggestions or recommendations concerned are unacceptable to the administrations which did not answer. If it was the requesting administration which failed to answer within this period, the Board shall close the study.

Section II – Miscellaneous provisions

8.2.1	If it is requested by any administration, particularly by an administration of a country in need of special assistance, the Board, using such means at its disposal as are appropriate in the circumstances, shall render the following assistance:

a)	computation necessary in the application of Annexes 1, 3 and 4;

b)	any other assistance of a technical nature for completion of the procedures in this Appendix.

8.2.2	In making a request to the Board under § 8.2.1, the administration shall furnish the Board with the necessary information.

ARTICLE 9     (REV.WRC-12)

Plan for feeder links for the broadcasting-satellite service in
the fixed-satellite service in the frequency band
17.3-17.8 GHz in Region 2

9.1				COLUMN HEADINGS OF THE PLAN

Col. 1	Beam identification (column 1 contains the symbol designating the country or the geographical area taken from Table B1 of the Preface to the International Frequency List followed by the symbol designating the service area).

Col. 2	Nominal orbital position, in degrees and hundredths of a degree.

Col. 3	Channel number (see Table 2 showing channel numbers and corresponding assigned frequencies).

Col. 4	Boresight geographical coordinates, in degrees and hundredths of a degree.

Col. 5	Antenna beamwidth. This column contains two figures corresponding to the major axis and the minor axis respectively of the elliptical cross section half-power beam, in degrees and hundredths of a degree.

Col. 6	Orientation of the ellipse determined as follows: in a plane normal to the beam axis, the direction of a major axis of the ellipse is specified as the angle measured anticlockwise from a line parallel to the equatorial plane to the major axis of the ellipse to the nearest degree.

Col. 7	Polarization (1 = direct, 2 = indirect)30.

Col. 8	Earth station e.i.r.p. in the direction of maximum radiation, in dBW.

Col. 9	Remarks31.

9.2		TEXT FOR NOTES IN REMARKS COLUMN OF THE PLAN

1	Fast roll-off space station receiving antenna as defined in Annex 3 (§ 4.6.3).

2	Television standard with 625 lines using greater video bandwidth and necessary bandwidth of 27 MHz.

3	This assignment may cause interference to feeder-link assignments* of Spain, Guinea-Bissau and Portugal in the Regions 1 and 3 feeder-link Plan adopted at the 1988 Conference and shall only be brought into use if:

a)	the administrations of Spain, Guinea-Bissau and Portugal agree: or

b)	their feeder-link equivalent protection margins, as defined in § 1.7 of Annex 3, are positive.

The affected administrations shall be informed by the notifying administration of the required changes in characteristics before this assignment is brought into use.

4	This assignment may be utilized in the geographical area of Anguilla (AIA) (which is in the beam area).

5	Feeder-link earth stations for this assignment may also be located in the territories of Puerto Rico and the United States Virgin Islands. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

6	Feeder-link earth stations for this assignment may also be located in the States of Alaska and Hawaii. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

7	The feeder-link earth station for this assignment may also be located at the point with geographical coordinates 3° 31′ West, 48° 46′ North. Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

8	Feeder-link earth stations for this assignment may also be located at the points with the following geographical coordinates:

| 47° 55′ West | 15° 47′ South | 34° 53′ West | 08° 04′ South |
| --- | --- | --- | --- |
| 43° 13′ West | 22° 55′ South | 60° 02′ West | 03° 06′ South |
| 46° 38′ West | 23° 33′ South | 38° 31′ West | 12° 56′ South |
| 51° 13′ West | 30° 02′ South | 49° 15′ West | 16° 40′ South |

Such operation shall not cause more interference nor require more protection than the assignment under the Plan.

9/GR . . 	This assignment is part of a group, the number of which follows the symbol. The group consists of the beams and has the number of channels assigned to it as indicated in Table 1.

a)	The overall equivalent protection margin to be used for the application of Article 4 and Resolution 42 (Rev.WRC-03)* shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

–	for the calculation of interference from assignments belonging to a group, to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.     (WRC-03).

b)	If an administration notifies the same frequency in more than one beam of a group for use at the same time, the overall C/I produced by all emissions from that group shall not exceed the C/I calculated on the basis of § a) above.

TABLE 1

| Group | Beams in the group | Number of channels assigned to the group |
| --- | --- | --- |
| GR1 | ALS00002 HWA00002 USAPSA02 | 32 channels |
| GR2 | ALS00003 HWA00003 USAPSA03 | 32 channels |
| GR3 | ARGINSU4 ARGSUR04 | 16 channels |
| GR4 | ARGINSU5 ARGSUR05 | 12 channels |
| GR5 | BOLAND01 CLMAND01 EQACAND1 EQAGAND1 PRUAND02 VENAND03 | 16 channels |
| GR6 | B SU111 B SU211 | 32 channels |
| GR7 | B CE311 B CE411 B CE511 | 32 channels |
| GR8 | B NO611 B NO711 B NO811 | 32 channels |
| GR9 | B SU112 B SU212 B CE312 B CE412 | 32 channels |
| GR10 | CAN01101 CAN01201 | 32 channels |
| GR11 | Not used |  |
| GR12 | CAN01203 CAN01303 CAN01403 | 32 channels |
| GR13 | CAN01304 CAN01404 CAN01504 | 32 channels |
| GR14 | CAN01405 CAN01505 CAN01605 | 32 channels |
| GR15 | Not used |  |
| GR16 | CHLCONT4 CHLCONT6 | 16 channels |
| GR17 | CHLCONT5 PAQPAC01 CHLPAC02 | 16 channels |
| GR18 | CRBBER01 CRBBLZ01 CRBJMC01 CRBBAH01 CRBEC001 | 16 channels |
| GR19 | EQACOO01 EQAGOO01 | 16 channels |
| GR20 | PTRVIR01 USAEHO02 | 32 channels |
| GR21 | PTRVIR02 USAEHO03 | 32 channels |
| GR22 | VEN02VEN VEN11VEN | 4 channels |

Country symbols

1	For the explanation of symbols designating countries or geographical areas in Region 2, see the Preface to the International Frequency List.

2	One additional symbol, CRB, has been created for the purposes of the 1983 Conference only, to designate to geographical area in the Caribbean Area. The five Caribbean beams are identified as follows:

CRBBAH01, CRBBER01, CRBBLZ01, CRBEC001 and CRBJMC01

and are intended collectively to provide coverage for the following countries or geographical areas: AIA, ATG, BAH, BER, BLZ, BRB, CYM, DMA, GRD, GUY, JMC, LCA, MSR, KNA, SUR, TCA, TRD, VCT and VRG to be so used if approved by them.

TABLE 2

Table showing correspondence between channel numbers and assigned frequencies

| Channel No. | Assigned frequency (MHz) | Channel No. | Assigned frequency (MHz) |
| --- | --- | --- | --- |
| 1 | 17 324.00 | 17 | 17 557.28 |
| 2 | 17 338.58 | 18 | 17 571.86 |
| 3 | 17 353.16 | 19 | 17 586.44 |
| 4 | 17 367.74 | 20 | 17 601.02 |
| 5 | 17 382.32 | 21 | 17 615.60 |
| 6 | 17 396.90 | 22 | 17 630.18 |
| 7 | 17 411.48 | 23 | 17 644.76 |
| 8 | 17 426.06 | 24 | 17 659.34 |
| 9 | 17 440.64 | 25 | 17 673.92 |
| 10 | 17 455.22 | 26 | 17 688.50 |
| 11 | 17 469.80 | 27 | 17 703.08 |
| 12 | 17 484.38 | 28 | 17 717.66 |
| 13 | 17 498.96 | 29 | 17 732.24 |
| 14 | 17 513.54 | 30 | 17 746.82 |
| 15 | 17 528.12 | 31 | 17 761.40 |
| 16 | 17 542.70 | 32 | 17 775.98 |

| 17 324.00 MHz (1) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 1 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS000/03 | −175.20 | 1 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 1 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGSUR04 | −94.20 | 1 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| B CE311 | −64.20 | 1 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 1 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 1 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 1 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 1 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 1 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 1 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 1 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 1 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 1 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 1 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 1 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BAHIFRB1 | −87.20 | 1 | −76.06 | 24.16 | 1.81 | 0.70 | 142 | 1 | 87.4 |  |
| BERBERMU | −96.20 | 1 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BERBER02 | −31.00 | 1 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| BOLAND01 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 1 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 1 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 1 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 1 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 1 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 1 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 1 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 1 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 1 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 1 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 1 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 1 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 1 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 1 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 1 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 1 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| FLKANT01 | −57.20 | 1 | −44.54 | −60.13 | 3.54 | 0.68 | 12 | 1 | 87.4 | 2 |
| FLKFALKS | −31.00 | 1 | −59.90 | −51.64 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| GRD00002 | −42.20 | 1 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 1 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 1 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 1 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 1 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 1 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 1 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 1 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 1 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 1 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 1 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 1 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| TRD00001 | −84.70 | 1 | −61.23 | 10.70 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| URG00001 | −71.70 | 1 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 1 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 1 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 1 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 1 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 1 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 1 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 1 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 1 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 1 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| VRG00001 | −79.70 | 1 | −64.37 | 18.48 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |

| 17 338.58 MHz (2) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 2 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 2 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 2 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 2 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 2 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 2 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 2 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 2 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 2 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 2 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 2 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 2 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 2 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 2 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 2 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 2 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 2 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 2 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 2 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 2 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 2 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 2 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 2 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 2 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 2 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 2 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 2 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 2 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 2 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 2 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 2 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 2 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 2 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 2 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 2 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 2 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 2 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 2 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 2 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 2 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 2 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUY00302 | −33.80 | 2 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 87.4 |  |
| HNDIFRB2 | −107.30 | 2 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 87.4 |  |
| HTI00002 | −83.30 | 2 | −73.28 | 18.96 | 0.82 | 0.68 | 11 | 2 | 87.4 |  |
| HWA00002 | −165.80 | 2 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 2 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 2 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 2 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 2 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 2 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 2 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 2 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| TCA00001 | −115.80 | 2 | −71.79 | 21.53 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| USAEH001 | −61.30 | 2 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 2 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 2 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 2 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 2 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 2 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 2 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 2 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VCT00001 | −79.30 | 2 | −61.18 | 13.23 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 2 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 353.16 MHz (3) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 3 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 3 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 3 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 3 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 3 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 3 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| ATGSJN01 | −79.70 | 3 | −61.79 | 17.07 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| B CE311 | −64.20 | 3 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 3 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 3 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 3 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 3 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 3 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 3 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 3 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 3 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 3 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 3 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 3 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 3 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 3 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 3 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 3 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 3 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 3 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 3 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 3 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 3 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 3 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 3 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 3 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 3 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 3 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 3 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 3 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 3 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 3 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 3 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 3 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00002 | −42.20 | 3 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRD00059 | −57.20 | 3 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 3 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| HWA00002 | −166.20 | 3 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 3 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 3 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 3 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 3 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 3 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 3 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 3 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 3 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 3 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SURINAM2 | −84.70 | 3 | −55.69 | 4.35 | 1.00 | 0.69 | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 3 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 3 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 3 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 3 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 3 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 3 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 3 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 3 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 3 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 3 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 367.74 MHz (4) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 4 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 4 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 4 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 4 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 4 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 4 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 4 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 4 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 4 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 4 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 4 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 4 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 4 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 4 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 4 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 4 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 4 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 4 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 4 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 4 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 4 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 4 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 4 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 4 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 4 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 4 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 4 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 4 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 4 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 4 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 4 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 4 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 4 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 4 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 4 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 4 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 4 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CYM00001 | −115.80 | 4 | −80.58 | 19.57 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| DOMIFRB2 | −83.30 | 4 | −70.51 | 18.79 | 0.98 | 0.69 | 167 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 4 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 4 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUFMGG02 | −52.80 | 4 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 4 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 4 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| JMC00005 | −33.80 | 4 | −77.27 | 18.12 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| LCAIFRB1 | −79.30 | 4 | −61.15 | 13.90 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| MEX01NTE | −77.80 | 4 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 4 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 4 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 4 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 4 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 4 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| SLVIFRB2 | −107.30 | 4 | −88.91 | 13.59 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| USAEH001 | −61.30 | 4 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 4 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 4 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 4 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 4 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 4 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 4 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 4 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 4 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 382.32 MHz (5) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 5 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 5 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 5 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGSUR04 | −94.20 | 5 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| B CE311 | −64.20 | 5 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 5 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 5 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 5 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 5 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 5 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 5 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 5 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 5 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 5 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 5 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 5 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BAHIFRB1 | −87.20 | 5 | −76.06 | 24.16 | 1.81 | 0.70 | 142 | 1 | 87.4 |  |
| BERBERMU | −96.20 | 5 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BERBER02 | −31.00 | 5 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| BOLAND01 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 5 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 5 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 5 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 5 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 5 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 5 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 5 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 5 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 5 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 5 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 5 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 5 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 5 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 5 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 5 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 5 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| FLKANT01 | −57.20 | 5 | −44.54 | −60.13 | 3.54 | 0.68 | 12 | 1 | 87.4 | 2 |
| FLKFALKS | −31.00 | 5 | −59.90 | −51.64 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| GRD00002 | −42.20 | 5 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 5 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 5 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 5 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 5 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 5 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 5 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 5 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 5 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 5 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 5 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 5 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| TRD00001 | −84.70 | 5 | −61.23 | 10.70 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| URG00001 | −71.70 | 5 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 5 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 5 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 5 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 5 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 5 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 5 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 5 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 5 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 5 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| VRG00001 | −79.70 | 5 | −64.37 | 18.48 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |

| 17 396.90 MHz (6) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 6 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 6 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 6 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 6 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 6 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 6 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 6 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 6 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 6 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 6 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 6 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 6 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 6 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 6 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 6 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 6 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 6 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 6 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 6 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 6 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 6 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 6 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 6 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 6 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 6 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 6 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 6 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 6 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 6 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 6 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 6 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 6 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 6 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 6 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 6 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 6 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 6 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 6 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 6 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 6 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 6 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUY00302 | −33.80 | 6 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 87.4 |  |
| HNDIFRB2 | −107.30 | 6 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 87.4 |  |
| HTI00002 | −83.30 | 6 | −73.28 | 18.96 | 0.82 | 0.68 | 11 | 2 | 87.4 |  |
| HWA00002 | −165.80 | 6 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 6 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 6 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 6 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 6 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 6 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 6 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 6 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| TCA00001 | −115.80 | 6 | −71.79 | 21.53 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| USAEH001 | −61.30 | 6 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 6 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 6 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 6 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 6 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 6 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 6 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 6 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VCT00001 | −79.30 | 6 | −61.18 | 13.23 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 6 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 411.48 MHz (7) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 7 | −109.94 | 36.86 | 6.04 |  | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 7 | −116.23 | 37.50 | 5.60 |  | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 7 | −52.98 | −59.81 | 3.40 |  | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 7 | −44.17 | −59.91 | 3.77 |  | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 7 | −65.04 | −43.33 | 3.32 |  | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 7 | −63.68 | −43.01 | 2.54 |  | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| ATGSJN01 | −79.70 | 7 | −61.79 | 17.07 | 0.60 |  | 0.60 | 90 | 1 | 87.4 |  |
| B CE311 | −64.20 | 7 | −40.60 | −6.07 | 3.04 |  | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 7 | −40.27 | −6.06 | 3.44 |  | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 7 | −50.97 | −15.27 | 3.86 |  | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 7 | −50.71 | −15.30 | 3.57 |  | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 7 | −53.10 | −2.90 | 2.44 |  | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 7 | −59.60 | −11.62 | 2.85 |  | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 7 | −60.70 | −1.78 | 3.54 |  | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 7 | −68.76 | −4.71 | 2.37 |  | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 7 | −51.12 | −25.63 | 2.76 |  | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 7 | −50.75 | −25.62 | 2.47 |  | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 7 | −44.51 | −16.95 | 3.22 |  | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 7 | −44.00 | −16.87 | 3.20 |  | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 7 | −64.77 | 32.32 | 0.60 |  | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 7 | −71.37 | −4.69 | 6.49 |  | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 7 | −64.61 | −16.71 | 2.52 |  | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 7 | −59.85 | 12.93 | 0.60 |  | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 7 | −114.60 | 51.08 | 7.28 |  | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 7 | −114.60 | 51.08 | 7.28 |  | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 7 | −81.34 | 50.02 | 7.96 |  | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 7 | −113.02 | 51.08 | 7.47 |  | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 7 | −113.02 | 51.08 | 7.47 |  | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 7 | −86.71 | 50.48 | 8.58 |  | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 7 | −113.02 | 51.08 | 7.47 |  | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 7 | −86.71 | 50.48 | 8.58 |  | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 7 | −84.11 | 50.20 | 8.31 |  | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 7 | −86.71 | 50.48 | 8.58 |  | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 7 | −84.11 | 50.20 | 8.31 |  | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 7 | −84.11 | 50.20 | 8.31 |  | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 7 | −80.77 | 50.03 | 7.88 |  | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 7 | −72.23 | −35.57 | 2.60 |  | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 7 | −80.06 | −30.06 | 1.36 |  | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 7 | −71.37 | −4.69 | 6.49 |  | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 7 | −74.50 | 5.87 | 3.98 |  | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 7 | −79.81 | 21.62 | 2.24 |  | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 7 | −71.37 | −4.69 | 6.49 |  | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 7 | −71.37 | −4.69 | 6.49 |  | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00002 | −42.20 | 7 | −61.58 | 12.29 | 0.60 |  | 0.60 | 90 | 1 | 87.4 |  |
| GRD00059 | −57.20 | 7 | −61.58 | 12.29 | 0.60 |  | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 7 | −44.89 | 66.56 | 2.70 |  | 0.82 | 173 | 1 | 87.4 | 2 |
| HWA00002 | −166.20 | 7 | −109.94 | 36.86 | 6.04 |  | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 7 | −116.23 | 37.50 | 5.60 |  | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 7 | −105.81 | 26.01 | 2.89 | 2.08 |  | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 7 | −94.84 | 19.82 | 3.05 | 2.09 |  | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 7 | −107.21 | 26.31 | 3.84 | 1.55 |  | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 7 | −96.39 | 19.88 | 3.18 | 1.87 |  | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 7 | −109.18 | −27.53 | 0.60 | 0.60 |  | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 7 | −58.66 | −23.32 | 1.45 | 1.04 |  | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 7 | −71.37 | −4.69 | 6.49 | 2.57 |  | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 7 | −93.94 | 36.32 | 8.24 | 3.56 |  | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 7 | −95.23 | 36.29 | 8.27 | 3.37 |  | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SURINAM2 | −84.70 | 7 | −55.69 | 4.35 | 1.00 | 0.69 |  | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 7 | −56.22 | −32.52 | 1.02 | 0.89 |  | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 7 | −87.57 | 36.17 | 6.42 | 3.49 |  | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 7 | −93.94 | 36.32 | 8.24 | 3.56 |  | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 7 | −95.23 | 36.29 | 8.27 | 3.37 |  | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 7 | −96.45 | 36.21 | 8.20 | 3.12 |  | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 7 | −109.94 | 36.86 | 6.04 | 1.11 |  | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 7 | −116.23 | 37.50 | 5.60 | 0.75 |  | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 7 | −111.02 | 40.68 | 4.36 | 2.15 |  | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 7 | −113.07 | 40.74 | 3.72 | 1.78 |  | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 7 | −71.37 | −4.69 | 6.49 | 2.57 |  | 87 | 1 | 87.4 | 9/GR5 |

| 17 426.06 MHz (8) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 8 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 8 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 8 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 8 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 8 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 8 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 8 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 8 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 8 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 8 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 8 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 8 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 8 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 8 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 8 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 8 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 8 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 8 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 8 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 8 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 8 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 8 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 8 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 8 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 8 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 8 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 8 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 8 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 8 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 8 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 8 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 8 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 8 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 8 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 8 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 8 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 8 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CYM00001 | −115.80 | 8 | −80.58 | 19.57 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| DOMIFRB2 | −83.30 | 8 | −70.51 | 18.79 | 0.98 | 0.69 | 167 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 8 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 8 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUFMGG02 | −52.80 | 8 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 8 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 8 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| JMC00005 | −33.80 | 8 | −77.27 | 18.12 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| LCAIFRB1 | −79.30 | 8 | −61.15 | 13.90 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| MEX01NTE | −77.80 | 8 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 8 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 8 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 8 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 8 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 8 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| SLVIFRB2 | −107.30 | 8 | −88.91 | 13.59 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| USAEH001 | −61.30 | 8 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 8 | −93.85 | 36.31 | 8.26 | 3.55 | 71 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 8 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 8 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 8 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 8 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 8 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 8 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 8 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 440.64 MHz (9) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 9 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 9 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 9 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGSUR04 | −94.20 | 9 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| B CE311 | −64.20 | 9 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 9 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 9 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 9 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 9 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 9 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 9 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 9 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 9 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 9 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 9 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 9 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BAHIFRB1 | −87.20 | 9 | −76.06 | 24.16 | 1.81 | 0.70 | 142 | 1 | 87.4 |  |
| BERBERMU | −96.20 | 9 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BERBER02 | −31.00 | 9 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| BOLAND01 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 9 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 9 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 9 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 9 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 9 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 9 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 9 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 9 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 9 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 9 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 9 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 9 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 9 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 9 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 9 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 9 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| FLKANT01 | −57.20 | 9 | −44.54 | −60.13 | 3.54 | 0.68 | 12 | 1 | 87.4 | 2 |
| FLKFALKS | −31.00 | 9 | −59.90 | −51.64 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| GRD00002 | −42.20 | 9 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 9 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 9 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 9 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 9 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 9 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 9 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 9 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 9 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 9 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 9 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 9 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| TRD00001 | −84.70 | 9 | −61.23 | 10.70 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| URG00001 | −71.70 | 9 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 9 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 9 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 9 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 9 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 9 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 9 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 9 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 9 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 9 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| VRG00001 | −79.70 | 9 | −64.37 | 18.48 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |

| 17 455.22 MHz (10) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 10 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 10 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 10 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 10 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 10 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 10 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 10 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 10 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 10 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 10 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 10 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 10 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 10 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 10 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 10 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 10 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 10 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 10 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 10 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 10 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 10 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 10 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 10 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 10 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 10 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 10 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 10 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 10 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 10 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 10 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 10 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 10 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 10 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 10 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 10 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 10 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 10 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 10 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 10 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 10 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 10 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUY00302 | −33.80 | 10 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 87.4 |  |
| HNDIFRB2 | −107.30 | 10 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 87.4 |  |
| HTI00002 | −83.30 | 10 | −73.28 | 18.96 | 0.82 | 0.68 | 11 | 2 | 87.4 |  |
| HWA00002 | −165.80 | 10 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 10 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 10 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 10 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 10 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 10 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 10 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 10 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| TCA00001 | −115.80 | 10 | −71.79 | 21.53 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| USAEH001 | −61.30 | 10 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 10 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 10 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 10 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 10 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 10 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 10 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 10 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VCT00001 | −79.30 | 10 | −61.18 | 13.23 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 10 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 469.80 MHz (11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 11 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 11 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 11 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 11 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 11 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 11 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| ATGSJN01 | −79.70 | 11 | −61.79 | 17.07 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| B CE311 | −64.20 | 11 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 11 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 11 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 11 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 11 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 11 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 11 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 11 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 11 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 11 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 11 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 11 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 11 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 11 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 11 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 11 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 11 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 11 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 11 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 11 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 11 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 11 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 11 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 11 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 11 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 11 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 11 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 11 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 11 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 11 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 11 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 11 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00002 | −42.20 | 11 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRD00059 | −57.20 | 11 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 11 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 11 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 11 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 11 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 11 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 11 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 11 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 11 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 11 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 11 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 11 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 11 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 11 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 11 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 11 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 11 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 11 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 11 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 11 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 11 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 11 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 11 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 484.38 MHz (12) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 12 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 12 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 12 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 12 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 12 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 12 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 12 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 12 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 12 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 12 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 12 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 12 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 12 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 12 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 12 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 12 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 12 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 12 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 12 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 12 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 12 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 12 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 12 | −86.57 | 50.48 | 8.58 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 12 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 12 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 12 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 12 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 12 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 12 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 12 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 12 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 12 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 12 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 12 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 12 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 12 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 12 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CYM00001 | −115.80 | 12 | −80.58 | 19.57 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| DOMIFRB2 | −83.30 | 12 | −70.51 | 18.79 | 0.98 | 0.69 | 167 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 12 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 12 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUFMGG02 | −52.80 | 12 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 12 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 12 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| JMC00005 | −33.80 | 12 | −77.27 | 18.12 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| LCAIFRB1 | −79.30 | 12 | −61.15 | 13.90 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| MEX01NTE | −77.80 | 12 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 12 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 12 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 12 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 12 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 12 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| SLVIFRB2 | −107.30 | 12 | −88.91 | 13.59 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| USAEH001 | −61.30 | 12 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 12 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 12 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 12 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 12 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 12 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 12 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 12 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 12 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 498.96 MHz (13) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 13 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 13 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 13 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGSUR04 | −94.20 | 13 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| B CE311 | −64.20 | 13 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 13 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 13 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 13 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 13 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 13 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 13 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 13 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 13 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 13 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 13 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 13 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BAHIFRB1 | −87.20 | 13 | −76.06 | 24.16 | 1.81 | 0.70 | 142 | 1 | 87.4 |  |
| BERBERMU | −96.20 | 13 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BERBER02 | −31.00 | 13 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| BOLAND01 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 13 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 13 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 13 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 13 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 13 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 13 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 13 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 13 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 13 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 13 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 13 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 13 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 13 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 13 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 13 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 13 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| FLKANT01 | −57.20 | 13 | −44.54 | −60.13 | 3.54 | 0.68 | 12 | 1 | 87.4 | 2 |
| FLKFALKS | −31.00 | 13 | −59.90 | −51.64 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| GRD00002 | −42.20 | 13 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 13 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 13 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 13 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 13 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 13 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 13 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 13 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 13 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 13 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 13 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 13 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| TRD00001 | −84.70 | 13 | −61.23 | 10.70 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| URG00001 | −71.70 | 13 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 13 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 13 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 13 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 13 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 13 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 13 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 13 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 13 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 13 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| VRG00001 | −79.70 | 13 | −64.37 | 18.48 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |

| 17 513.54 MHz (14) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 14 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 14 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 14 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 14 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 14 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 14 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 14 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 14 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 14 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 14 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 14 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 14 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 14 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 14 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 14 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 14 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 14 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 14 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 14 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 14 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 14 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 14 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 14 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 14 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 14 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 14 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 14 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 14 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 14 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 14 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 14 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 14 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 14 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 14 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 14 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 14 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 14 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 14 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 14 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 14 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 14 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUY00302 | −33.80 | 14 | −59.07 | 4.77 | 1.43 | 0.85 | 91 | 2 | 87.4 |  |
| HNDIFRB2 | −107.30 | 14 | −86.23 | 15.16 | 1.14 | 0.85 | 8 | 1 | 87.4 |  |
| HTI00002 | −83.30 | 14 | −73.28 | 18.96 | 0.82 | 0.68 | 11 | 2 | 87.4 |  |
| HWA00002 | −165.80 | 14 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 14 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 14 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 14 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 14 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 14 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 14 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 14 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| TCA00001 | −115.80 | 14 | −71.79 | 21.53 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| USAEH001 | −61.30 | 14 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 14 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 14 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 14 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 14 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 14 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 14 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 14 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VCT00001 | −79.30 | 14 | −61.18 | 13.23 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 14 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 528.12 MHz (15) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 15 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 15 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 15 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 15 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 15 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 15 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| ATGSJN01 | −79.70 | 15 | −61.79 | 17.07 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| B CE311 | −64.20 | 15 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 15 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 15 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 15 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 15 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 15 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 15 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 15 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 15 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 15 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 15 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 15 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 15 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 15 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 15 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 15 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 15 | −114.60 | 51.08 | 7.28 | 1.10 | 160 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 15 | −81.34 | 50.02 | 7.96 | 2.55 | 5 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 15 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 15 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 15 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 15 | −113.02 | 51.08 | 7.47 | 1.26 | 162 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 15 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 15 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 15 | −86.71 | 50.48 | 8.58 | 2.54 | 178 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 15 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 15 | −84.11 | 50.20 | 8.31 | 2.58 | 1 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 15 | −80.77 | 50.03 | 7.88 | 2.53 | 6 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 15 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 15 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 15 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 15 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00002 | −42.20 | 15 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRD00059 | −57.20 | 15 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 15 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 15 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 15 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 15 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 15 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 15 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 15 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 15 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 15 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 15 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 15 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 15 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 15 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 15 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 15 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 15 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 15 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 15 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 15 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 15 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 15 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 15 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 542.70 MHz (16) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 16 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 16 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 16 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 16 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 16 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 16 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 16 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 16 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 16 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 16 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 16 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 16 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 16 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 16 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 16 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 16 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 16 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 16 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 16 | −114.10 | 50.92 | 7.22 | 1.11 | 160 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 16 | −81.23 | 50.12 | 7.99 | 2.53 | 5 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 16 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 16 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 16 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 16 | −113.04 | 51.04 | 7.53 | 1.26 | 162 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 16 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 16 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 16 | −86.57 | 50.48 | 8.59 | 2.54 | 178 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 16 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 16 | −83.80 | 50.22 | 8.35 | 2.57 | 2 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 16 | −80.64 | 50.02 | 7.88 | 2.52 | 6 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 16 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 16 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 16 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 16 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 16 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 16 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 16 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CYM00001 | −115.80 | 16 | −80.58 | 19.57 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| DOMIFRB2 | −83.30 | 16 | −70.51 | 18.79 | 0.98 | 0.69 | 167 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 16 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 16 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GUFMGG02 | −52.80 | 16 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 16 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 16 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| JMC00005 | −33.80 | 16 | −77.27 | 18.12 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| LCAIFRB1 | −79.30 | 16 | −61.15 | 13.90 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| MEX01NTE | −77.80 | 16 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 16 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 16 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PRU00004 | −85.80 | 16 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 16 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 16 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| SLVIFRB2 | −107.30 | 16 | −88.91 | 13.59 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| USAEH001 | −61.30 | 16 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 16 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 16 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 16 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 16 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 16 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 16 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 16 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 16 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 557.28 MHz (17) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 17 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 17 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 17 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 17 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 17 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 17 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 17 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 9/GR3 |
| B CE312 | −45.20 | 17 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 9/GR4 |
| B CE411 | −64.20 | 17 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 17 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 17 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 17 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 17 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 17 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 17 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 17 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 17 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 17 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 17 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BERBER02 | −31.00 | 17 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| BOLAND01 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 17 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 17 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 17 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 17 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 17 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 17 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 17 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 17 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 17 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 17 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 17 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 17 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 17 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 17 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 17 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 17 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| FLKFALKS | −31.00 | 17 | −59.90 | −51.64 | 0.60 | 0.60 | 90 | 1 | 87.4 | 2 3 |
| HWA00002 | −166.20 | 17 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 17 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| JMC00002 | −92.70 | 17 | −77.30 | 18.12 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| KNA00001 | −79.70 | 17 | −62.46 | 17.44 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| MEX01NTE | −78.20 | 17 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 17 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 17 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 17 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 17 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 17 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 17 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 17 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 17 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| SURINAM2 | −84.70 | 17 | −55.69 | 4.35 | 1.00 | 0.69 | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 17 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 17 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 17 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 17 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 17 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 17 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 17 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 17 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 17 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 17 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 571.86 MHz (18) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 18 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 18 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 18 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 18 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 18 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 18 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 18 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 18 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 18 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 18 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 18 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 18 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 18 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 18 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 18 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 18 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 18 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 18 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| BLZ00001 | −115.80 | 18 | −88.68 | 17.27 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| CAN01101 | −137.80 | 18 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 18 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 18 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 18 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 18 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 18 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 18 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 18 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 18 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 18 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 18 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 18 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 18 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 18 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 18 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 18 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 18 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 18 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 18 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 18 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 18 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| DMAIFRB1 | −79.30 | 18 | −61.30 | 15.35 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 18 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 18 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| HWA00002 | −165.80 | 18 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 18 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 18 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 18 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 18 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| NCG00003 | −107.30 | 18 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 18 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 18 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 18 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 18 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 18 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 18 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 18 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 18 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 18 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 18 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 18 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 18 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 586.44 MHz (19) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 19 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 19 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 19 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 19 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 19 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 19 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 19 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 19 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 19 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 19 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 19 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 19 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 19 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 19 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 19 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 19 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 19 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 19 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 19 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 19 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 19 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 19 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 19 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 19 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 19 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 19 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 19 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 19 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 19 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 19 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 19 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 19 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 19 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 19 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 19 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 19 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 19 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 19 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00059 | −57.20 | 19 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 19 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 19 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 19 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 19 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 19 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 19 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 19 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 19 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| MSR00001 | −79.70 | 19 | −61.73 | 16.75 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |
| PAQPAC01 | −106.20 | 19 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 19 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 19 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 19 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 19 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 19 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 19 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 19 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 19 | −96.45 | 36.31 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 19 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 19 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 19 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 19 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 19 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 601.02 MHz (20) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 20 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 20 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 20 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 20 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 20 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 20 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 20 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 20 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 20 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 20 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 20 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 20 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 20 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 20 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 20 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 20 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 20 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 20 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 20 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 20 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 20 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 20 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 20 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 20 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 20 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 20 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 20 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 20 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 20 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 20 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 20 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 20 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 20 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 20 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 20 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 20 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 20 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| EQAC0001 | −94.80 | 20 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 20 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GRD00003 | −79.30 | 20 | −61.62 | 12.34 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| GTMIFRB2 | −107.30 | 20 | −90.50 | 15.64 | 1.03 | 0.74 | 84 | 1 | 87.4 |  |
| GUFMGG02 | −52.80 | 20 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 20 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 20 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 20 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 20 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 20 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PNRIFRB2 | −121.00 | 20 | −80.15 | 8.46 | 1.01 | 0.73 | 170 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 20 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 20 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 20 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 20 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 20 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 20 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 20 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 20 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 20 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 20 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 20 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN02VEN | −103.80 | 20 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 | 9/GR22 |
| VEN11VEN | −103.80 | 20 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 | 9/GR22 |

| 17 615.60 MHz (21) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 21 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 21 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 21 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 21 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 21 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 21 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 21 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 21 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 21 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 21 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 21 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 21 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 21 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 21 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 21 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 21 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 21 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 21 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 21 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 21 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 21 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 21 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 21 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 21 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 21 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 21 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 21 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 21 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 21 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 21 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 21 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 21 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 21 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 21 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 21 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| HWA00002 | −166.20 | 21 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 21 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| JMC00002 | −92.70 | 21 | −77.30 | 18.12 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| KNA00001 | −79.70 | 21 | −62.46 | 17.44 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| MEX01NTE | −78.20 | 21 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 21 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 21 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 21 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 21 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 21 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 21 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 21 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 21 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| SURINAM2 | −84.70 | 21 | −55.69 | 4.35 | 1.00 | 0.69 | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 21 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 21 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 21 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 21 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 21 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 21 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 21 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 21 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 21 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 21 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 630.18 MHz (22) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 22 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 22 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 22 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 22 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 22 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 22 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 22 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 22 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 22 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 22 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 22 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 22 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 22 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 22 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 22 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 22 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 22 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 22 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| BLZ00001 | −115.80 | 22 | −88.68 | 17.27 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| CAN01101 | −137.80 | 22 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 22 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 22 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 22 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 22 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 22 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 22 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 22 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 22 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 22 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 22 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 22 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 22 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 22 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 22 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 22 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 22 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 22 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 22 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 22 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 22 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| DMAIFRB1 | −79.30 | 22 | −61.30 | 15.35 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 22 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 22 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| HWA00002 | −165.80 | 22 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 22 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 22 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 22 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 22 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| NCG00003 | −107.30 | 22 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 22 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 22 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 22 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 22 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 22 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 22 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 22 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 22 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 22 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 22 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 22 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 22 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 644.76 MHz (23) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 23 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 23 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 23 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 23 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 23 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 23 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 23 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 23 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 23 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 23 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 23 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 23 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 23 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 23 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 23 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 23 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 23 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 23 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 23 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 23 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 23 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 23 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 23 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 23 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 23 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 23 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 23 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 23 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 23 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 23 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 23 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 23 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 23 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 23 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 23 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 23 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 23 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 23 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00059 | −57.20 | 23 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 23 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 23 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 23 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 23 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 23 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 23 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 23 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 23 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| MSR00001 | −79.70 | 23 | −61.73 | 16.75 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |
| PAQPAC01 | −106.20 | 23 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 23 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 23 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 23 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 23 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 23 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 23 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 23 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 23 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 23 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 23 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 23 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 23 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 23 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 659.34 MHz (24) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 |  | 8 | 9 |
| ALS00002 | −165.80 | 24 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 |  | 9/GR1 |
| ALS00003 | −174.80 | 24 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 |  | 9/GR2 |
| ARGNORT4 | −93.80 | 24 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |  |
| ARGNORT5 | −54.80 | 24 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |  |
| B CE311 | −63.80 | 24 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 |  | 8 9/GR7 |
| B CE312 | −44.80 | 24 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 |  | 8 9/GR9 |
| B CE411 | −63.80 | 24 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 |  | 8 9/GR7 |
| B CE412 | −44.80 | 24 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 |  | 8 9/GR9 |
| B CE511 | −63.80 | 24 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 |  | 8 9/GR7 |
| B NO611 | −73.80 | 24 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 |  | 8 9/GR8 |
| B NO711 | −73.80 | 24 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 |  | 8 9/GR8 |
| B NO811 | −73.80 | 24 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 |  | 8 9/GR8 |
| B SE911 | −101.80 | 24 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 |  | 8 |
| B SU111 | −80.80 | 24 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 |  | 8 9/GR6 |
| B SU112 | −44.80 | 24 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 |  | 8 9/GR9 |
| B SU211 | −80.80 | 24 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 |  | 8 9/GR6 |
| B SU212 | −44.80 | 24 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 |  | 8 9/GR9 |
| CAN01101 | −137.80 | 24 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 |  | 9/GR10 |
| CAN01201 | −137.80 | 24 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 |  | 9/GR10 |
| CAN01202 | −72.30 | 24 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |  |
| CAN01203 | −128.80 | 24 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 |  | 9/GR12 |
| CAN01303 | −128.80 | 24 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 |  | 9/GR12 |
| CAN01304 | −90.80 | 24 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 |  | 9/GR13 |
| CAN01403 | −128.80 | 24 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 |  | 9/GR12 |
| CAN01404 | −90.80 | 24 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 |  | 9/GR13 |
| CAN01405 | −81.80 | 24 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 |  | 9/GR14 |
| CAN01504 | −90.80 | 24 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 |  | 9/GR13 |
| CAN01505 | −81.80 | 24 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 |  | 9/GR14 |
| CAN01605 | −81.80 | 24 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 |  | 9/GR14 |
| CAN01606 | −70.30 | 24 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |  |
| CHLCONT4 | −105.80 | 24 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 |  | 9/GR16 |
| CHLCONT6 | −105.80 | 24 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 |  | 9/GR16 |
| CRBBAH01 | −92.30 | 24 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 |  | 9/GR18 |
| CRBBER01 | −92.30 | 24 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 |  | 9/GR18 |
| CRBBLZ01 | −92.30 | 24 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 |  | 9/GR18 |
| CRBEC001 | −92.30 | 24 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 |  | 9/GR18 |
| CRBJMC01 | −92.30 | 24 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 |  | 9/GR18 |
| EQAC0001 | −94.80 | 24 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 |  | 9/GR19 |
| EQAG0001 | −94.80 | 24 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 |  | 9/GR19 |
| GRD00003 | −79.30 | 24 | −61.62 | 12.34 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |  |
| GTMIFRB2 | −107.30 | 24 | −90.50 | 15.64 | 1.03 | 0.74 | 84 | 1 | 87.4 |  |  |
| GUFMGG02 | −52.80 | 24 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 |  | 2 7 |
| HWA00002 | −165.80 | 24 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 |  | 9/GR1 |
| HWA00003 | −174.80 | 24 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 |  | 9/GR2 |
| MEX01NTE | −77.80 | 24 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 |  | 1 |
| MEX02NTE | −135.80 | 24 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 |  | 1 |
| MEX02SUR | −126.80 | 24 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 |  | 1 |
| PNRIFRB2 | −121.00 | 24 | −80.15 | 8.46 | 1.01 | 0.73 | 170 | 1 | 87.4 |  |  |
| PRU00004 | −85.80 | 24 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |  |
| PTRVIR01 | −100.80 | 24 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 |  | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 24 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 |  | 1 6 9/GR21 |
| USAEH001 | −61.30 | 24 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 |  | 1 5 6 |
| USAEH002 | −100.80 | 24 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 |  | 1 6 9/GR20 |
| USAEH003 | −109.80 | 24 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 |  | 1 6 9/GR21 |
| USAEH004 | −118.80 | 24 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 |  | 1 5 6 |
| USAPSA02 | −165.80 | 24 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 |  | 9/GR1 |
| USAPSA03 | −174.80 | 24 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 |  | 9/GR2 |
| USAWH101 | −147.80 | 24 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |  |
| USAWH102 | −156.80 | 24 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |  |
| VEN02VEN | −103.80 | 24 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  | 9/GR22 |
| VEN11VEN | −103.80 | 24 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  | 9/GR22 |

| 17 673.92 MHz (25) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 25 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 25 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 25 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 25 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 25 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 25 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 25 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 25 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 25 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 25 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 25 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 25 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 25 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 25 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 25 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 25 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 25 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 25 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 25 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 25 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 25 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 25 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 25 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 25 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 25 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 25 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 25 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 25 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 25 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 25 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 25 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 25 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 25 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 25 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 25 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| HWA00002 | −166.20 | 25 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 25 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| JMC00002 | −92.70 | 25 | −77.30 | 18.12 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| KNA00001 | −79.70 | 25 | −62.46 | 17.44 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| MEX01NTE | −78.20 | 25 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 25 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 25 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 25 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 25 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 25 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 25 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 25 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 25 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| SURINAM2 | −84.70 | 25 | −55.69 | 4.35 | 1.00 | 0.69 | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 25 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 25 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 25 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 25 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 25 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 25 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 25 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 25 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 25 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 25 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 688.50 MHz (26) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 26 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 26 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 26 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 26 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 26 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 26 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 26 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 26 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 26 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 26 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 26 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 26 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 26 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 26 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 26 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 26 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 26 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 26 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| BLZ00001 | −115.80 | 26 | −88.68 | 17.27 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| CAN01101 | −137.80 | 26 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 26 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 26 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 26 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 26 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 26 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 26 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 26 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 26 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 26 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 26 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 26 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 26 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 26 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 26 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 26 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 26 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 26 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 26 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 26 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 26 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| DMAIFRB1 | −79.30 | 26 | −61.30 | 15.35 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 26 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 26 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| HWA00002 | −165.80 | 26 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 26 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 26 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 26 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 26 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| NCG00003 | −107.30 | 26 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 26 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 26 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 26 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 26 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 26 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 26 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 26 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 26 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 26 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 26 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 26 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 26 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 703.08 MHz (27) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 27 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 27 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 27 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 27 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 27 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 27 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 27 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 27 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 27 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 27 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 27 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 27 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 27 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 27 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 27 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 27 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 27 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 27 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 27 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 27 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 27 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 27 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 27 | −112.04 | 55..95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 27 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 27 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 27 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 27 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 27 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 27 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 27 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 27 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 27 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 27 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 27 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 27 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 27 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 27 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 27 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00059 | −57.20 | 27 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 27 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 27 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 27 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 27 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 27 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 27 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 27 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 27 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| MSR00001 | −79.70 | 27 | −61.73 | 16.75 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |
| PAQPAC01 | −106.20 | 27 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 27 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 27 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 27 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 27 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 27 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 27 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 27 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 27 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 27 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 27 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 27 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 27 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 27 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 717.66 MHz (28) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 28 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 28 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 28 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 28 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 28 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 28 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 28 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 28 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 28 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 28 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 28 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 28 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 28 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 28 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 28 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 28 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 28 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 28 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 28 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 28 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 28 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 28 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 28 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 28 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 28 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 28 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 28 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 28 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 28 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 28 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 28 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 28 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 28 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 28 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 28 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 28 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 28 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| EQAC0001 | −94.80 | 28 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 28 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GRD00003 | −79.30 | 28 | −61.62 | 12.34 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| GTMIFRB2 | −107.30 | 28 | −90.50 | 15.64 | 1.03 | 0.74 | 84 | 1 | 87.4 |  |
| GUFMGG02 | −52.80 | 28 | −56.42 | 8.47 | 4.16 | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 28 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 28 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 28 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 28 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 28 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| PNRIFRB2 | −121.00 | 28 | −80.15 | 8.46 | 1.01 | 0.73 | 170 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 28 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 28 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 28 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 28 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 28 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 28 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 28 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 28 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 28 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 28 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 28 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN02VEN | −103.80 | 28 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 | 9/GR22 |
| VEN11VEN | −103.80 | 28 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 | 9/GR22 |

| 17 732.24 MHz (29) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 29 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 29 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 29 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 29 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 29 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 29 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 29 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 29 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 29 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 29 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 29 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 29 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 29 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 29 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 29 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 29 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 29 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 29 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 29 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CAN01101 | −138.20 | 29 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 29 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 29 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 29 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 29 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 29 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 29 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 29 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 29 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 29 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 29 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 29 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 29 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 29 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 29 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 29 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| HWA00002 | −166.20 | 29 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 29 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| JMC00002 | −92.70 | 29 | −77.30 | 18.12 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| KNA00001 | −79.70 | 29 | −62.46 | 17.44 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| MEX01NTE | −78.20 | 29 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 29 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 29 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 29 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| PAQPAC01 | −106.20 | 29 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 29 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 29 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 29 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| SPMFRAN3 | −53.20 | 29 | −67.24 | 47.51 | 3.16 | 0.79 | 7 | 1 | 87.4 | 2 7 |
| SURINAM2 | −84.70 | 29 | −55.69 | 4.35 | 1.00 | 0.69 | 86 | 1 | 87.4 |  |
| URG00001 | −71.70 | 29 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 29 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 29 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 29 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 29 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 29 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 29 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 29 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 29 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 29 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 746.82 MHz (30) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 30 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 30 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 30 | −63.96 | −30.01 | 3.86 | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 30 | −62.85 | −29.80 | 3.24 | 2.89 | 47 | 2 | 87.4 |  |
| ATNBEAM1 | −52.80 | 30 | −66.44 | 14.87 | 1.83 | 0.68 | 39 | 2 | 87.4 |  |
| B CE311 | −63.80 | 30 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 30 | −40.26 | −6.06 | 3.44 | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 30 | −50.97 | −15.26 | 3.86 | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 30 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 30 | −53.11 | −2.98 | 2.42 | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 30 | −59.60 | −11.62 | 2.86 | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 30 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 30 | −68.75 | −4.71 | 2.37 | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 30 | −45.99 | −19.09 | 2.22 | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 30 | −51.10 | −25.64 | 2.76 | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 30 | −50.76 | −25.62 | 2.47 | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 30 | −44.51 | −16.94 | 3.22 | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 30 | −43.99 | −16.97 | 3.27 | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| BLZ00001 | −115.80 | 30 | −88.68 | 17.27 | 0.62 | 0.62 | 90 | 2 | 87.4 |  |
| CAN01101 | −137.80 | 30 | −125.60 | 57.24 | 3.45 | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 30 | −111.92 | 55.89 | 3.33 | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 30 | −107.64 | 55.62 | 2.75 | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 30 | −111.43 | 55.56 | 3.07 | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 30 | −102.39 | 57.12 | 3.54 | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 30 | −99.00 | 57.33 | 1.96 | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 30 | −89.70 | 52.02 | 4.67 | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 30 | −84.78 | 52.41 | 3.09 | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 30 | −84.02 | 52.34 | 2.82 | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 30 | −72.68 | 53.78 | 3.57 | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 30 | −71.76 | 53.76 | 3.30 | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 30 | −61.54 | 49.50 | 2.66 | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 30 | −61.32 | 49.51 | 2.41 | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 30 | −69.59 | −23.20 | 2.21 | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 30 | −73.52 | −55.52 | 3.65 | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 30 | −76.09 | 24.13 | 1.83 | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 30 | −64.76 | 32.13 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 30 | −88.61 | 17.26 | 0.64 | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 30 | −60.07 | 8.26 | 4.20 | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 30 | −79.45 | 17.97 | 0.99 | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| CTR00201 | −130.80 | 30 | −84.33 | 9.67 | 0.82 | 0.68 | 119 | 2 | 87.4 |  |
| DMAIFRB1 | −79.30 | 30 | −61.30 | 15.35 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| EQAC0001 | −94.80 | 30 | −78.31 | −1.52 | 1.48 | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 30 | −90.36 | −0.57 | 0.94 | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| HWA00002 | −165.80 | 30 | −165.79 | 23.32 | 4.20 | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 30 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 30 | −105.80 | 25.99 | 2.88 | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 30 | −107.36 | 26.32 | 3.80 | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 30 | −96.39 | 19.88 | 3.19 | 1.87 | 158 | 2 | 87.4 | 1 |
| NCG00003 | −107.30 | 30 | −84.99 | 12.90 | 1.05 | 1.01 | 176 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 30 | −74.19 | −8.39 | 3.74 | 2.45 | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 30 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 30 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 30 | −87.53 | 36.18 | 6.41 | 3.49 | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 30 | −93.85 | 36.31 | 8.26 | 3.55 | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 30 | −95.47 | 36.38 | 8.10 | 3.45 | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 30 | −96.42 | 36.21 | 8.20 | 3.12 | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 30 | −109.83 | 36.82 | 6.03 | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 30 | −116.10 | 37.47 | 5.60 | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 30 | −111.01 | 40.67 | 4.38 | 2.15 | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 30 | −113.01 | 40.71 | 3.74 | 1.79 | 149 | 2 | 87.4 |  |
| VEN11VEN | −103.80 | 30 | −66.79 | 6.90 | 2.50 | 1.77 | 122 | 2 | 87.4 |  |

| 17 761.40 MHz (31) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  | 6 | 7 | 8 | 9 |
| ALS00002 | −166.20 | 31 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| ALS00003 | −175.20 | 31 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| ARGINSU4 | −94.20 | 31 | −52.98 | −59.81 | 3.40 | 0.68 | 19 | 1 | 87.4 | 9/GR3 |
| ARGINSU5 | −55.20 | 31 | −44.17 | −59.91 | 3.77 | 0.70 | 13 | 1 | 87.4 | 9/GR4 |
| ARGSUR04 | −94.20 | 31 | −65.04 | −43.33 | 3.32 | 1.50 | 40 | 1 | 87.4 | 9/GR3 |
| ARGSUR05 | −55.20 | 31 | −63.68 | −43.01 | 2.54 | 2.38 | 152 | 1 | 87.4 | 9/GR4 |
| B CE311 | −64.20 | 31 | −40.60 | −6.07 | 3.04 | 2.06 | 174 | 1 | 87.4 | 8 9/GR7 |
| B CE312 | −45.20 | 31 | −40.27 | −6.06 | 3.44 | 2.09 | 174 | 1 | 87.4 | 8 9/GR9 |
| B CE411 | −64.20 | 31 | −50.97 | −15.27 | 3.86 | 1.38 | 49 | 1 | 87.4 | 8 9/GR7 |
| B CE412 | −45.20 | 31 | −50.71 | −15.30 | 3.57 | 1.56 | 52 | 1 | 87.4 | 8 9/GR9 |
| B CE511 | −64.20 | 31 | −53.10 | −2.90 | 2.44 | 2.13 | 104 | 1 | 87.4 | 8 9/GR7 |
| B NO611 | −74.20 | 31 | −59.60 | −11.62 | 2.85 | 1.69 | 165 | 2 | 87.4 | 8 9/GR8 |
| B NO711 | −74.20 | 31 | −60.70 | −1.78 | 3.54 | 1.78 | 126 | 2 | 87.4 | 8 9/GR8 |
| B NO811 | −74.20 | 31 | −68.76 | −4.71 | 2.37 | 1.65 | 73 | 2 | 87.4 | 8 9/GR8 |
| B SU111 | −81.20 | 31 | −51.12 | −25.63 | 2.76 | 1.05 | 50 | 1 | 87.4 | 8 9/GR6 |
| B SU112 | −45.20 | 31 | −50.75 | −25.62 | 2.47 | 1.48 | 56 | 1 | 87.4 | 8 9/GR9 |
| B SU211 | −81.20 | 31 | −44.51 | −16.95 | 3.22 | 1.36 | 60 | 1 | 87.4 | 8 9/GR6 |
| B SU212 | −45.20 | 31 | −44.00 | −16.87 | 3.20 | 1.96 | 58 | 1 | 87.4 | 8 9/GR9 |
| BERBERMU | −96.20 | 31 | −64.77 | 32.32 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| BOLAND01 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| BOL00001 | −87.20 | 31 | −64.61 | −16.71 | 2.52 | 2.19 | 85 | 1 | 87.4 |  |
| BRB00001 | −92.70 | 31 | −59.85 | 12.93 | 0.60 | 0.60 | 90 | 2 | 87.4 |  |
| CAN01101 | −138.20 | 31 | −125.63 | 57.24 | 3.45 | 1.27 | 157 | 1 | 87.4 | 9/GR10 |
| CAN01201 | −138.20 | 31 | −112.04 | 55.95 | 3.35 | 0.97 | 151 | 1 | 87.4 | 9/GR10 |
| CAN01202 | −72.70 | 31 | −107.70 | 55.63 | 2.74 | 1.12 | 32 | 1 | 87.4 |  |
| CAN01203 | −129.20 | 31 | −111.48 | 55.61 | 3.08 | 1.15 | 151 | 1 | 87.4 | 9/GR12 |
| CAN01303 | −129.20 | 31 | −102.42 | 57.12 | 3.54 | 0.91 | 154 | 1 | 87.4 | 9/GR12 |
| CAN01304 | −91.20 | 31 | −99.12 | 57.36 | 1.98 | 1.72 | 2 | 1 | 87.4 | 9/GR13 |
| CAN01403 | −129.20 | 31 | −89.75 | 52.02 | 4.68 | 0.78 | 148 | 1 | 87.4 | 9/GR12 |
| CAN01404 | −91.20 | 31 | −84.82 | 52.42 | 3.10 | 2.05 | 152 | 1 | 87.4 | 9/GR13 |
| CAN01405 | −82.20 | 31 | −84.00 | 52.39 | 2.84 | 2.29 | 172 | 1 | 87.4 | 9/GR14 |
| CAN01504 | −91.20 | 31 | −72.66 | 53.77 | 3.57 | 1.67 | 156 | 1 | 87.4 | 9/GR13 |
| CAN01505 | −82.20 | 31 | −71.77 | 53.79 | 3.30 | 1.89 | 162 | 1 | 87.4 | 9/GR14 |
| CAN01605 | −82.20 | 31 | −61.50 | 49.55 | 2.65 | 1.40 | 143 | 1 | 87.4 | 9/GR14 |
| CAN01606 | −70.70 | 31 | −61.30 | 49.55 | 2.40 | 1.65 | 148 | 1 | 87.4 |  |
| CHLCONT5 | −106.20 | 31 | −72.23 | −35.57 | 2.60 | 0.68 | 55 | 1 | 87.4 | 9/GR17 |
| CHLPAC02 | −106.20 | 31 | −80.06 | −30.06 | 1.36 | 0.68 | 69 | 1 | 87.4 | 9/GR17 |
| CLMAND01 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| CLM00001 | −103.20 | 31 | −74.50 | 5.87 | 3.98 | 1.96 | 118 | 1 | 87.4 |  |
| CUB00001 | −89.20 | 31 | −79.81 | 21.62 | 2.24 | 0.68 | 168 | 1 | 87.4 |  |
| EQACAND1 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| EQAGAND1 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| GRD00059 | −57.20 | 31 | −61.58 | 12.29 | 0.60 | 0.60 | 90 | 1 | 87.4 |  |
| GRLDNK01 | −53.20 | 31 | −44.89 | 66.56 | 2.70 | 0.82 | 173 | 1 | 87.4 | 2 |
| GUY00201 | −84.70 | 31 | −59.19 | 4.78 | 1.44 | 0.85 | 95 | 1 | 87.4 |  |
| HWA00002 | −166.20 | 31 | −165.79 | 23.42 | 4.20 | 0.68 | 160 | 1 | 87.4 | 9/GR1 |
| HWA00003 | −175.20 | 31 | −166.10 | 23.42 | 4.25 | 0.68 | 159 | 1 | 87.4 | 9/GR2 |
| MEX01NTE | −78.20 | 31 | −105.81 | 26.01 | 2.89 | 2.08 | 155 | 1 | 87.4 | 1 |
| MEX01SUR | −69.20 | 31 | −94.84 | 19.82 | 3.05 | 2.09 | 4 | 1 | 87.4 | 1 |
| MEX02NTE | −136.20 | 31 | −107.21 | 26.31 | 3.84 | 1.55 | 148 | 1 | 87.4 | 1 |
| MEX02SUR | −127.20 | 31 | −96.39 | 19.88 | 3.18 | 1.87 | 157 | 1 | 87.4 | 1 |
| MSR00001 | −79.70 | 31 | −61.73 | 16.75 | 0.60 | 0.60 | 90 | 1 | 87.4 | 4 |
| PAQPAC01 | −106.20 | 31 | −109.18 | −27.53 | 0.60 | 0.60 | 90 | 1 | 87.4 | 9/GR17 |
| PRG00002 | −99.20 | 31 | −58.66 | −23.32 | 1.45 | 1.04 | 76 | 1 | 87.4 |  |
| PRUAND02 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |
| PTRVIR01 | −101.20 | 31 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −110.20 | 31 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| URG00001 | −71.70 | 31 | −56.22 | −32.52 | 1.02 | 0.89 | 11 | 1 | 87.4 |  |
| USAEH001 | −61.70 | 31 | −87.57 | 36.17 | 6.42 | 3.49 | 12 | 1 | 87.4 | 1 5 6 |
| USAEH002 | −101.20 | 31 | −93.94 | 36.32 | 8.24 | 3.56 | 171 | 1 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −110.20 | 31 | −95.23 | 36.29 | 8.27 | 3.37 | 168 | 1 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −119.20 | 31 | −96.45 | 36.21 | 8.20 | 3.12 | 165 | 1 | 87.4 | 1 5 6 |
| USAPSA02 | −166.20 | 31 | −109.94 | 36.86 | 6.04 | 1.11 | 137 | 1 | 87.4 | 9/GR1 |
| USAPSA03 | −175.20 | 31 | −116.23 | 37.50 | 5.60 | 0.75 | 132 | 1 | 87.4 | 9/GR2 |
| USAWH101 | −148.20 | 31 | −111.02 | 40.68 | 4.36 | 2.15 | 162 | 1 | 87.4 |  |
| USAWH102 | −157.20 | 31 | −113.07 | 40.74 | 3.72 | 1.78 | 149 | 1 | 87.4 |  |
| VENAND03 | −115.20 | 31 | −71.37 | −4.69 | 6.49 | 2.57 | 87 | 1 | 87.4 | 9/GR5 |

| 17 775.98 MHz (32) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |  | 5 |  |  | 6 | 7 | 8 | 9 |
| ALS00002 | −165.80 | 32 | −109.83 | 36.82 | 6.03 |  | 1.12 | 137 | 2 | 87.4 | 9/GR1 |
| ALS00003 | −174.80 | 32 | −116.10 | 37.47 | 5.60 |  | 0.76 | 132 | 2 | 87.4 | 9/GR2 |
| ARGNORT4 | −93.80 | 32 | −63.96 | −30.01 | 3.86 |  | 1.99 | 48 | 2 | 87.4 |  |
| ARGNORT5 | −54.80 | 32 | −62.85 | −29.80 | 3.24 |  | 2.89 | 47 | 2 | 87.4 |  |
| B CE311 | −63.80 | 32 | −40.60 | −6.07 | 3.04 |  | 2.06 | 174 | 2 | 87.4 | 8 9/GR7 |
| B CE312 | −44.80 | 32 | −40.26 | −6.06 | 3.44 |  | 2.09 | 174 | 2 | 87.4 | 8 9/GR9 |
| B CE411 | −63.80 | 32 | −50.97 | −15.26 | 3.86 |  | 1.38 | 49 | 2 | 87.4 | 8 9/GR7 |
| B CE412 | −44.80 | 32 | −50.71 | −15.30 | 3.57 |  | 1.56 | 52 | 2 | 87.4 | 8 9/GR9 |
| B CE511 | −63.80 | 32 | −53.11 | −2.98 | 2.42 |  | 2.15 | 107 | 2 | 87.4 | 8 9/GR7 |
| B NO611 | −73.80 | 32 | −59.60 | −11.62 | 2.86 |  | 1.69 | 165 | 1 | 87.4 | 8 9/GR8 |
| B NO711 | −73.80 | 32 | −60.70 | −1.78 | 3.54 |  | 1.78 | 126 | 1 | 87.4 | 8 9/GR8 |
| B NO811 | −73.80 | 32 | −68.75 | −4.71 | 2.37 |  | 1.65 | 73 | 1 | 87.4 | 8 9/GR8 |
| B SE911 | −101.80 | 32 | −45.99 | −19.09 | 2.22 |  | 0.79 | 62 | 2 | 87.4 | 8 |
| B SU111 | −80.80 | 32 | −51.10 | −25.64 | 2.76 |  | 1.06 | 50 | 2 | 87.4 | 8 9/GR6 |
| B SU112 | −44.80 | 32 | −50.76 | −25.62 | 2.47 |  | 1.48 | 56 | 2 | 87.4 | 8 9/GR9 |
| B SU211 | −80.80 | 32 | −44.51 | −16.94 | 3.22 |  | 1.37 | 60 | 2 | 87.4 | 8 9/GR6 |
| B SU212 | −44.80 | 32 | −43.99 | −16.97 | 3.27 |  | 1.92 | 59 | 2 | 87.4 | 8 9/GR9 |
| CAN01101 | −137.80 | 32 | −125.60 | 57.24 | 3.45 |  | 1.27 | 157 | 2 | 87.4 | 9/GR10 |
| CAN01201 | −137.80 | 32 | −111.92 | 55.89 | 3.33 |  | 0.98 | 151 | 2 | 87.4 | 9/GR10 |
| CAN01202 | −72.30 | 32 | −107.64 | 55.62 | 2.75 |  | 1.11 | 32 | 2 | 87.4 |  |
| CAN01203 | −128.80 | 32 | −111.43 | 55.56 | 3.07 |  | 1.15 | 151 | 2 | 87.4 | 9/GR12 |
| CAN01303 | −128.80 | 32 | −102.39 | 57.12 | 3.54 |  | 0.92 | 154 | 2 | 87.4 | 9/GR12 |
| CAN01304 | −90.80 | 32 | −99.00 | 57.33 | 1.96 |  | 1.73 | 1 | 2 | 87.4 | 9/GR13 |
| CAN01403 | −128.80 | 32 | −89.70 | 52.02 | 4.67 |  | 0.79 | 148 | 2 | 87.4 | 9/GR12 |
| CAN01404 | −90.80 | 32 | −84.78 | 52.41 | 3.09 |  | 2.06 | 153 | 2 | 87.4 | 9/GR13 |
| CAN01405 | −81.80 | 32 | −84.02 | 52.34 | 2.82 |  | 2.30 | 172 | 2 | 87.4 | 9/GR14 |
| CAN01504 | −90.80 | 32 | −72.68 | 53.78 | 3.57 |  | 1.67 | 157 | 2 | 87.4 | 9/GR13 |
| CAN01505 | −81.80 | 32 | −71.76 | 53.76 | 3.30 |  | 1.89 | 162 | 2 | 87.4 | 9/GR14 |
| CAN01605 | −81.80 | 32 | −61.54 | 49.50 | 2.66 |  | 1.39 | 144 | 2 | 87.4 | 9/GR14 |
| CAN01606 | −70.30 | 32 | −61.32 | 49.51 | 2.41 |  | 1.65 | 148 | 2 | 87.4 |  |
| CHLCONT4 | −105.80 | 32 | −69.59 | −23.20 | 2.21 |  | 0.69 | 68 | 2 | 87.4 | 9/GR16 |
| CHLCONT6 | −105.80 | 32 | −73.52 | −55.52 | 3.65 |  | 1.31 | 39 | 2 | 87.4 | 9/GR16 |
| CRBBAH01 | −92.30 | 32 | −76.09 | 24.13 | 1.83 |  | 0.68 | 141 | 1 | 87.4 | 9/GR18 |
| CRBBER01 | −92.30 | 32 | −64.76 | 32.13 | 0.60 |  | 0.60 | 90 | 1 | 87.4 | 9/GR18 |
| CRBBLZ01 | −92.30 | 32 | −88.61 | 17.26 | 0.64 |  | 0.64 | 90 | 1 | 87.4 | 9/GR18 |
| CRBEC001 | −92.30 | 32 | −60.07 | 8.26 | 4.20 |  | 0.86 | 115 | 1 | 87.4 | 9/GR18 |
| CRBJMC01 | −92.30 | 32 | −79.45 | 17.97 | 0.99 |  | 0.68 | 151 | 1 | 87.4 | 9/GR18 |
| EQAC0001 | −94.80 | 32 | −78.31 | −1.52 | 1.48 |  | 1.15 | 65 | 1 | 87.4 | 9/GR19 |
| EQAG0001 | −94.80 | 32 | −90.36 | −0.57 | 0.94 |  | 0.89 | 99 | 1 | 87.4 | 9/GR19 |
| GRD00003 | −79.30 | 32 | −61.62 | 12.34 | 0.60 |  | 0.60 | 90 | 2 | 87.4 |  |
| GTMIFRB2 | −107.30 | 32 | −90.50 | 15.64 | 1.03 |  | 0.74 | 84 | 1 | 87.4 |  |
| GUFMGG02 | −52.80 | 32 | −56.42 | 8.47 | 4.16 |  | 0.81 | 123 | 2 | 87.4 | 2 7 |
| HWA00002 | −165.80 | 32 | −165.79 | 23.32 | 4.20 |  | 0.68 | 160 | 2 | 87.4 | 9/GR1 |
| HWA00003 | −174.80 | 32 | −166.10 | 23.42 | 4.25 |  | 0.68 | 159 | 2 | 87.4 | 9/GR2 |
| MEX01NTE | −77.80 | 32 | −105.80 | 25.99 | 2.88 |  | 2.07 | 155 | 2 | 87.4 | 1 |
| MEX02NTE | −135.80 | 32 | −107.36 | 26.32 | 3.80 |  | 1.57 | 149 | 2 | 87.4 | 1 |
| MEX02SUR | −126.80 | 32 | −96.39 | 19.88 | 3.19 |  | 1.87 | 158 | 2 | 87.4 | 1 |
| PNRIFRB2 | −121.00 | 32 | −80.15 | 8.46 | 1.01 |  | 0.73 | 170 | 1 | 87.4 |  |
| PRU00004 | −85.80 | 32 | −74.19 | −8.39 | 3.74 | 2.45 |  | 112 | 2 | 87.4 |  |
| PTRVIR01 | −100.80 | 32 | −93.85 | 36.31 | 8.26 | 3.55 |  | 171 | 2 | 87.4 | 1 6 9/GR20 |
| PTRVIR02 | −109.80 | 32 | −95.47 | 36.38 | 8.10 | 3.45 |  | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH001 | −61.30 | 32 | −87.53 | 36.18 | 6.41 | 3.49 |  | 12 | 2 | 87.4 | 1 5 6 |
| USAEH002 | −100.80 | 32 | −93.85 | 36.31 | 8.26 | 3.55 |  | 171 | 2 | 87.4 | 1 6 9/GR20 |
| USAEH003 | −109.80 | 32 | −95.47 | 36.38 | 8.10 | 3.45 |  | 168 | 2 | 87.4 | 1 6 9/GR21 |
| USAEH004 | −118.80 | 32 | −96.42 | 36.21 | 8.20 | 3.12 |  | 165 | 2 | 87.4 | 1 5 6 |
| USAPSA02 | −165.80 | 32 | −109.83 | 36.82 | 6.03 | 1.12 |  | 137 | 2 | 87.4 | 9/GR1 |
| USAPSA03 | −174.80 | 32 | −116.10 | 37.47 | 5.60 | 0.76 |  | 132 | 2 | 87.4 | 9/GR2 |
| USAWH101 | −147.80 | 32 | −111.01 | 40.67 | 4.38 | 2.15 |  | 162 | 2 | 87.4 |  |
| USAWH102 | −156.80 | 32 | −113.01 | 40.71 | 3.74 | 1.79 |  | 149 | 2 | 87.4 |  |
| VEN02VEN | −103.80 | 32 | −66.79 | 6.90 | 2.50 | 1.77 |  | 122 | 2 | 87.4 | 9/GR22 |
| VEN11VEN | −103.80 | 32 | −66.79 | 6.90 | 2.50 | 1.77 |  | 122 | 2 | 87.4 | 9/GR22 |

ARTICLE 9A     (Rev.WRC-15)

Plan for feeder links for the broadcasting-satellite service in
the fixed-satellite service in the frequency bands
14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3

9A.1	COLUMN HEADINGS OF THE PLAN

Col. 1	Notifying administration symbol.

Col. 2	Beam identification (Column 2, normally, contains the symbol designating the administration or the geographical area taken from Table B1 of the Preface to the International Frequency List, followed by the symbol designating the service area).

Col. 3	Nominal orbital position, in degrees and hundredths of a degree from the Greenwich meridian (negative values indicate longitudes which are west of the Greenwich meridian; positive values indicate longitudes which are east of the Greenwich meridian).

Col. 4	Nominal intersection of the beam axis with the Earth (boresight or aim point in the case of a non-elliptical beam), longitude and latitude, in degrees and hundredths of a degree.

Col. 5	Space station receiving antenna characteristics (elliptical beams). This Column contains three numerical values corresponding to the major axis, the minor axis and the major axis orientation respectively of the elliptical cross-section half-power beam, in degrees and hundredths of a degree. Orientation of the ellipse determined as follows: in a plane normal to the beam axis, the direction of a major axis of the ellipse is specified as the angle measured anticlockwise from a line parallel to the equatorial plane to the major axis of the ellipse, to the nearest degree.

Col. 6	Space station receiving antenna pattern code.

The codes used for the antenna pattern of the receiving space station (feeder link) antenna are defined as follows:

| R13RSS | Figure B (Curves A, B and C) and § 3.7.3 in Annex 3 |
| --- | --- |
| R123FR | Figure C and § 3.7.3 in Annex 3 |
| MODRSS | Figure B (Curves A′, B′ and C) and § 3.7.3 in Annex 3 (Recommendation ITU-R BO.1296) |

In cases where the “Space station receiving antenna pattern code” field is blank, the necessary antenna pattern data are provided by shaped beam data submitted by the administration. These data are stored in Column 7. A particular shaped beam is identified by the combination of Column 1, Column 7 and Column 14. In such cases the maximum cross-polar gain is given in Column 8, Cross-polar gain field.

In cases where the “Space station receiving antenna pattern code” field contains a code which starts with “CB_” characters, it is a composite beam. Any composite beam consists of two or more elliptical beams. Each composite beam is described in the special composite beam file having the same name plus a GXT extension (e.g. the description of the CB_COMP_BM1 composite beam is stored in the CB_COMP_BM1.GXT file).

Col. 7	Space station receiving antenna shaped (non-elliptical, non-composite) beam identification.

Col. 8	Maximum space station receiving antenna co-polar and cross-polar (in the case of shaped beam) isotropic gain (dBi).

Col. 9	Earth station transmitting antenna pattern code and maximum gain (dBi).

The codes used for transmitting earth station (feeder-link) antenna patterns are defined as follows:

| R13TES | Figure A (Curves A and B) and § 3.5.3 in Annex 3 |
| --- | --- |
| MODTES | Figure A (Curves A′ and B′) and § 3.5.3 in Annex 3 (Recommendation ITU-R BO.1295) |

Col. 10	Polarization (CL – circular left, CR – circular right, LE – linear referenced to the equatorial plane) and polarization angle in degrees and hundredths of a degree (in the case of linear polarization only).

Col. 11	e.i.r.p. in the direction of maximum radiation (dBW).

Col. 12	Permitted increase in earth station e.i.r.p. (dB) for the purpose of power control (see § 3.11 of Annex 3).32  (WRC-15)

Col. 13	Designation of emission.

Col. 14	Identity of the space station.

Col. 15	Group code (an identification code which indicates that all assignments with the same group identification code will be treated as a group).

Group code: if an assignment is part of the group:

a)	the equivalent protection margin to be used for the application of Article 4 shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included, and

–	for the calculation of interference from assignments belonging to a group to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.

b)	If an administration notifies the same frequency in more than one beam of a group for use at the same time, the aggregate C/I ratio produced by all emissions from that group shall not exceed the C/I ratio calculated on the basis of § a) above.

Col. 16	Assignment status.

The assignment status codes used for beams are defined as follows:

| P | Assignment in the Regions 1 and 3 feeder-link Plan which has not been brought into use and/or the date of bringing into use has not been confirmed to the Bureau. For this category of assignments, WRC-2000 protection ratios are applied (27 dB co-channel and 22 dB adjacent channel). |
| --- | --- |
| PE | Assignment in the Regions 1 and 3 feeder-link Plan, which is in conformity with Appendix 30A, has been notified, brought into use and the date of bringing into use has been confirmed to the Bureau before 12 May 2000. For this category of assignments, WRC-97 protection ratios are applied (30 dB co-channel and 22 dB adjacent channel). |

Col. 17	Remarks.

9A.2	TEXT FOR NOTES IN THE REMARKS COLUMN OF THE
	REGIONS 1 AND 3 FEEDER-LINK PLAN     (WRC-15)

1	(Not used.)

2	(Not used.)

3	(Not used.)

4	(Not used.)

5	This assignment shall be brought into use only when the limits given in § 5 of Annex 1 are not exceeded, or with the agreement of the administrations identified in Table 1A, whose networks or beams listed in this Table may be affected with respect to assignments which are in conformity with the Region 2 feeder-link Plan on 12 May 2000 (see also Note to § 9A.2).

6	This assignment shall not claim protection from interference caused by the assignments which pertain to networks or beams identified in Table 1B which are in conformity with the Region 2 feeder-link Plan on 12 May 2000 (see also Note to § 9A.2).

7	This assignment shall not claim protection from interference caused by the assignments which pertain to networks or beams identified in Table 1B which are recorded in the Master Register with a favourable finding prior to 12 May 2000 (see also Note to § 9A.2).

The methodology and criteria for this analysis shall be those contained in § 1 of Annex 4, modified to take into consideration the system noise temperature of the received space station to be 600 K and to apply a T/T criterion of 6%.

8	Provisional beam. These assignments have been included in the Regions 1 and 3 feeder-link Plan by WRC-97. These assignments are for exclusive use by Palestine, subject to the Israeli-Palestinian Interim Agreement of 28 September 1995, Resolution 741 of the Council notwithstanding and Resolution 99 (Minneapolis, 1998) of the Plenipotentiary Conference.

9	(Not used.)

10	(SUP - WRC-12)

NOTE – In cases where assignments from the WRC-97 Plans without Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna characteristics, the coordination status afforded by the WRC-97 Plans shall be preserved.

In cases where assignments from the WRC-97 Plans with Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the compatibility will be reassessed using the revised criteria and methodology of WRC-2000 and the Remarks of the WRC-97 Plans assignment will either be maintained or reduced on the basis of the results of this analysis.

In other cases, the methodology described in Notes 5 to 7 shall be applied.

TABLE 1A     (WRC-07)

Affected administrations and corresponding networks/beams identified based on Note 5 in § 9A.2 of Article 9A

| Beam name | Channels | Affected administrations* | Affected networks/beams* |
| --- | --- | --- | --- |
| CPV30100 | 2, 4, 8, 10, 12 | GUY JMC | GUY00302, JMC00005 |
| CPV30100 | 6 | JMC | JMC00005 |
| G 02700 | 2, 4, 8, 10, 12 | GUY JMC | GUY00302, JMC00005 |
| G 02700 | 6 | JMC | JMC00005 |
| LBR24400 | 1 | GUY | GUY00302 |
| LBR24400 | 3, 9, 13 | JMC | JMC00005 |
| LBR24400 | 5, 7, 11 | GUY JMC | GUY00302, JMC00005 |
| * Administrations and corresponding networks or beams whose assignment(s) may receive interference from the beam shown in the left-hand column. |  |  |  |

TABLE 1B     (WRC-15)

Affecting administrations and corresponding networks/beams identified based on Notes 6 and 7 in § 9A.2 of Article 9A

| Beam name | Channels | Note | Affecting administrations* | Affecting networks/beams* |
| --- | --- | --- | --- | --- |
| CPV30100 | 2, 4, 8, 10, 12 | 6 | GUY JMC | GUY00302, JMC00005 |
| CPV30100 | 6 | 6 | JMC | JMC00005 |
| E____100 | 1, 3, 5, 7, 9, 11, 13 | 6 | G | BERBER02 |
| G 02700 | 2, 4, 8, 10, 12 | 6 | GUY JMC | GUY00302, JMC00005 |
| G 02700 | 6 | 6 | JMC | JMC00005 |
| LBR24400 | 1 | 6 | GUY | GUY00302 |
| LBR24400 | 3, 9, 13 | 6 | JMC | JMC00005 |
| LBR24400 | 5, 7, 11 | 6 | GUY JMC | GUY00302, JMC00005 |
| * Administrations and corresponding networks or beams whose assignment(s) may cause interference to the beam shown in the left-hand column. |  |  |  |  |

TABLE 2A

Table showing correspondence between channel numbers and assigned frequencies1
for the feeder links in the frequency band 14.5-14.8 GHz

| Channel No. | Assigned feeder-link frequency (MHz) |
| --- | --- |
| 1 | 14 525.30 |
| 2 | 14 544.48 |
| 3 | 14 563.66 |
| 4 | 14 582.84 |
| 5 | 14 602.02 |
| 6 | 14 621.20 |
| 7 | 14 640.38 |
| 8 | 14 659.56 |
| 9 | 14 678.74 |
| 10 | 14 697.92 |
| 11 | 14 717.10 |
| 12 | 14 736.28 |
| 13 | 14 755.46 |
| 14 | 14 774.64 |
| 1 Assigned frequency = 14 506.12 + 19.18 n, where n is the channel number. |  |

TABLE 2B

Table showing correspondence between channel numbers and assigned frequencies1
for the feeder links in the frequency band 17.3-18.1 GHz

| Channel No. | Assigned feeder-link frequency (MHz) | Channel No. | Assigned feeder-link frequency (MHz) |
| --- | --- | --- | --- |
| 1 | 17 327.48 | 21 | 17 711.08 |
| 2 | 17 346.66 | 22 | 17 730.26 |
| 3 | 17 365.84 | 23 | 17 749.44 |
| 4 | 17 385.02 | 24 | 17 768.62 |
| 5 | 17 404.20 | 25 | 17 787.80 |
| 6 | 17 423.38 | 26 | 17 806.98 |
| 7 | 17 442.56 | 27 | 17 826.16 |
| 8 | 17 461.74 | 28 | 17 845.34 |
| 9 | 17 480.92 | 29 | 17 864.52 |
| 10 | 17 500.10 | 30 | 17 883.70 |
| 11 | 17 519.28 | 31 | 17 902.88 |
| 12 | 17 538.46 | 32 | 17 922.06 |
| 13 | 17 557.64 | 33 | 17 941.24 |
| 14 | 17 576.82 | 34 | 17 960.42 |
| 15 | 17 596.00 | 35 | 17 979.60 |
| 16 | 17 615.18 | 36 | 17 998.78 |
| 17 | 17 634.36 | 37 | 18 017.96 |
| 18 | 17 653.54 | 38 | 18 037.14 |
| 19 | 17 672.72 | 39 | 18 056.32 |
| 20 | 17 691.90 | 40 | 18 075.50 |
| 1 Assigned frequency = 17 308.3 + 19.18 n, where n is the channel number. |  |  |  |

TABLE 3A1

Basic characteristics of the Regions 1 and 3 feeder-link Plan in the frequency band 14.5-14.8 GHz (sorted by administration)

| 1 | 2 | 3 | 4 |  | 5 |  |  | 6 | 7 | 8 |  | 9 |  | 10 |  | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Admin. symbol | Beam identification | Orbital position | Boresight |  | Space station antenna characteristics |  |  | Space station antenna code | Shaped beam | Space station antenna gain |  | Earth station antenna |  | Polarization |  | e.i.r.p. | Power control | Designation of emission | Identity of the space station | Group code | Status | Remarks |
|  |  |  | Long. | Lat. | Major axis | Minor axis | Orien- tation |  |  | Co-polar | Cross-polar | Code | Gain | Type | Angle |  |  |  |  |  |  |  |
| AFS | AFS02101 | 4.80 | 24.50 | –28.00 | 3.13 | 1.68 | 27.00 | MODRSS |  | 37.24 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4L | P |  |
| AFS | AFS02102 | 4.80 | 24.50 | –28.00 | 3.13 | 1.68 | 27.00 | MODRSS |  | 37.24 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4L | P |  |
| CHN | CHN19001 | 122.00 | 114.17 | 23.32 | 0.91 | 0.60 | 2.88 | MODRSS |  | 47.08 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 4C | P |  |
| CHN | CHN19002 | 122.00 | 114.17 | 23.32 | 0.91 | 0.60 | 2.88 | MODRSS |  | 47.08 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 4C | P |  |
| CME | CME30001 | –13.00 | 12.70 | 6.20 | 2.54 | 1.68 | 87.00 | MODRSS |  | 38.15 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 4I | P |  |
| CME | CME30002 | –13.00 | 12.70 | 6.20 | 2.54 | 1.68 | 87.00 | MODRSS |  | 38.15 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 4I | P |  |
| ETH | ETH09201 | 36.00 | 40.49 | 9.20 | 2.83 | 2.26 | 174.44 | MODRSS |  | 36.40 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4P | P |  |
| ETH | ETH09202 | 36.00 | 40.49 | 9.20 | 2.83 | 2.26 | 174.44 | MODRSS |  | 36.40 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4P | P |  |
| GHA | GHA10801 | –25.00 | –1.20 | 7.90 | 1.48 | 1.06 | 102.00 | MODRSS |  | 42.49 |  | MODTES | 57.00 | CR |  | 83.0 |  | 27M0G7W |  | 4F | P |  |
| GHA | GHA10802 | –25.00 | –1.20 | 7.90 | 1.48 | 1.06 | 102.00 | MODRSS |  | 42.49 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  | 4F | P |  |
| IND | INDA_101 | 55.80 | 76.16 | 14.72 |  |  |  | CB_RSS_INDA |  | 45.66 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4U | P |  |
| IND | INDA_102 | 55.80 | 76.16 | 14.72 |  |  |  | CB_RSS_INDA |  | 45.66 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4U | P |  |
| IRN | IRN10901 | 34.00 | 54.20 | 32.40 | 3.82 | 1.82 | 149.00 | MODRSS |  | 36.03 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4S | P |  |
| IRN | IRN10902 | 34.00 | 54.20 | 32.40 | 3.82 | 1.82 | 149.00 | MODRSS |  | 36.03 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4S | P |  |
| IRQ | IRQ25601 | 50.00 | 43.86 | 32.86 | 1.82 | 1.34 | 162.65 | MODRSS |  | 40.58 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4M | P |  |
| IRQ | IRQ25602 | 50.00 | 43.86 | 32.86 | 1.82 | 1.34 | 162.65 | MODRSS |  | 40.58 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4M | P |  |
| KOR | KO11201D | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | R13RSS |  | 43.40 |  | R13TES | 57.30 | CL |  | 82.0 |  | 27M0G7W | KOREASAT-1 | 03 | PE |  |
| KOR | KOR11201 | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | R13RSS |  | 43.40 |  | R13TES | 57.30 | CL |  | 82.0 |  | 27M0F8W | KOREASAT-1 | 03 | PE |  |
| MOZ | MOZ30701 | –1.00 | 34.00 | –18.00 | 3.57 | 1.38 | 55.00 | MODRSS |  | 37.52 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4K | P |  |
| MOZ | MOZ30702 | –1.00 | 34.00 | –18.00 | 3.57 | 1.38 | 55.00 | MODRSS |  | 37.52 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4K | P |  |
| NIG | NIG11901 | –19.20 | 7.80 | 9.40 | 2.16 | 2.02 | 45.00 | MODRSS |  | 38.05 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4G | P |  |
| NIG | NIG11902 | –19.20 | 7.80 | 9.40 | 2.16 | 2.02 | 45.00 | MODRSS |  | 38.05 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4G | P |  |
| NMB | NMB02501 | –18.80 | 17.50 | –21.60 | 2.66 | 1.90 | 48.00 | MODRSS |  | 37.41 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4H | P |  |
| NMB | NMB02502 | –18.80 | 17.50 | –21.60 | 2.66 | 1.90 | 48.00 | MODRSS |  | 37.41 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4H | P |  |
| NPL | NPL12201 | 50.00 | 83.70 | 28.30 | 1.72 | 0.60 | 163.00 | MODRSS |  | 44.31 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4N | P |  |
| NPL | NPL12202 | 50.00 | 83.70 | 28.30 | 1.72 | 0.60 | 163.00 | MODRSS |  | 44.31 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4N | P |  |
| PAK | PAK12701 | 38.20 | 69.60 | 29.50 | 2.30 | 2.16 | 14.00 | MODRSS |  | 37.49 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4R | P |  |
| PAK | PAK12702 | 38.20 | 69.60 | 29.50 | 2.30 | 2.16 | 14.00 | MODRSS |  | 37.49 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4R | P |  |
| PNG | PNG13101 | 134.00 | 148.07 | –6.65 | 3.13 | 2.30 | 168.32 | MODRSS |  | 38.87 |  | MODTES | 57.00 | CR |  | 89.0 |  | 27M0G7W |  | 4B | P |  |
| PNG | PNG13102 | 134.00 | 148.07 | –6.65 | 3.13 | 2.30 | 168.32 | MODRSS |  | 38.87 |  | MODTES | 57.00 | CL |  | 89.0 |  | 27M0G7W |  | 4B | P |  |
| SDN | SDN__101 | –7.00 | 30.13 | 13.52 |  |  |  | CB_RSS_SDNA |  | 37.20 |  | MODTES | 57.00 | CL |  | 86.0 |  | 27M0G7W |  | 4J | P |  |
| SDN | SDN__102 | –7.00 | 30.13 | 13.52 |  |  |  | CB_RSS_SDNA |  | 37.20 |  | MODTES | 57.00 | CR |  | 86.0 |  | 27M0G7W |  | 4J | P |  |
| SEN | SEN22201 | –37.00 | –14.40 | 13.80 | 1.46 | 1.04 | 139.00 | MODRSS |  | 42.63 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4D | P |  |
| SEN | SEN22202 | –37.00 | –14.40 | 13.80 | 1.46 | 1.04 | 139.00 | MODRSS |  | 42.63 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4D | P |  |
| SEY | SEY00001 | 42.50 | 51.86 | –7.23 | 2.43 | 1.04 | 27.51 | MODRSS |  | 40.44 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 4T | P |  |
| SEY | SEY00002 | 42.50 | 51.86 | –7.23 | 2.43 | 1.04 | 27.51 | MODRSS |  | 40.44 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 4T | P |  |
| SOM | SOM31201 | 37.80 | 45.17 | 6.61 | 3.37 | 1.68 | 62.04 | MODRSS |  | 36.92 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  | 4Q | P |  |
| SOM | SOM31202 | 37.80 | 45.17 | 6.61 | 3.37 | 1.68 | 62.04 | MODRSS |  | 36.92 |  | MODTES | 57.00 | CR |  | 83.0 |  | 27M0G7W |  | 4Q | P |  |
| TGO | TGO22601 | –30.00 | 0.68 | 8.57 | 1.13 | 0.60 | 108.43 | MODRSS |  | 46.14 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4E | P |  |
| TGO | TGO22602 | –30.00 | 0.68 | 8.57 | 1.13 | 0.60 | 108.43 | MODRSS |  | 46.14 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4E | P |  |
| USA | USAC_101 | 140.00 | 177.50 | 16.35 |  |  |  | CB_RSS_USAC |  | 44.06 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 4A | P |  |
| USA | USAC_102 | 140.00 | 177.50 | 16.35 |  |  |  | CB_RSS_USAC |  | 44.06 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 4A | P |  |
| YEM | YEM__101 | 11.00 | 48.29 | 14.53 |  |  |  | CB_RSS_YEMA |  | 47.78 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  | 4O | P |  |
| YEM | YEM__102 | 11.00 | 48.29 | 14.53 |  |  |  | CB_RSS_YEMA |  | 47.78 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  | 4O | P |  |

TABLE 3A2    (WRC-15)

Basic characteristics of the Regions 1 and 3 feeder-link Plan in the frequency band 17.3-18.1 GHz (sorted by administration)

| 1 | 2 | 3 | 4 |  | 5 |  |  | 6 | 7 | 8 |  | 9 |  | 10 |  | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Admin. symbol | Beam identification | Orbital position | Boresight |  | Space station antenna characteristics |  |  | Space station antenna code | Shaped beam | Space station antenna gain |  | Earth station antenna |  | Polarization |  | e.i.r.p. | Power control | Designation of emission | Identity of the space station | Group code | Status | Remarks |
|  |  |  | Long. | Lat. | Major axis | Minor axis | Orien- tation |  |  | Co-polar | Cross-polar | Code | Gain | Type | Angle |  |  |  |  |  |  |  |
| AFG | AFG24501 | 50.00 | 67.00 | 34.30 | 1.89 | 1.19 | 18.00 | MODRSS |  | 40.93 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7I | P |  |
| AFG | AFG24502 | 50.00 | 67.00 | 34.30 | 1.89 | 1.19 | 18.00 | MODRSS |  | 40.93 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7I | P |  |
| AGL | AGL29500 | −24.80 | 16.43 | −12.37 | 2.66 | 1.75 | 77.43 | MODRSS |  | 37.77 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| ALB | ALB29600 | 62.00 | 19.50 | 41.37 | 0.60 | 0.60 | 69.35 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 82.6 |  | 27M0G7W |  |  | P |  |
| ALG | ALG25152 | −24.80 | 1.50 | 27.60 | 3.65 | 2.94 | 135.00 | MODRSS |  | 34.14 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| AND | AND34100 | −37.00 | 1.60 | 42.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| ARM | ARM06400 | 22.80 | 44.99 | 39.95 | 0.73 | 0.60 | 148.17 | MODRSS |  | 48.02 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| ARS | ARS00375 | 17.00 | 44.60 | 23.40 | 4.21 | 2.48 | 145.00 | MODRSS |  | 34.26 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 54 | P |  |
| ARS | ARS34000 | 17.00 | 44.60 | 23.40 | 4.21 | 2.48 | 145.00 | MODRSS |  | 34.28 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 54 | P |  |
| AUS | AUS00400 | 152.00 | 135.00 | −24.20 | 7.19 | 5.20 | 140.00 | MODRSS |  | 28.71 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00401 | 152.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00402 | 152.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00403 | 152.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00404 | 152.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00405 | 152.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00406 | 152.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS0040A | 152.00 | 135.36 | −23.95 | 6.89 | 4.83 | 141.15 | R123FR |  | 29.23 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 30 | P |  |
| AUS | AUS00500 | 152.00 | 135.00 | −24.20 | 7.19 | 5.20 | 140.00 | MODRSS |  | 28.71 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00501 | 152.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00502 | 152.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00503 | 152.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00504 | 152.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00505 | 152.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00506 | 152.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 41 | P |  |
| AUS | AUS00600 | 152.00 | 135.50 | −24.20 | 7.19 | 5.20 | 140.00 | MODRSS |  | 28.71 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00601 | 152.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00602 | 152.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00603 | 152.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00604 | 152.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00605 | 152.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00606 | 152.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 42 | P |  |
| AUS | AUS00700 | 164.00 | 136.00 | −23.90 | 7.26 | 4.48 | 132.00 | MODRSS |  | 29.32 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00701 | 164.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00702 | 164.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00703 | 164.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00704 | 164.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00705 | 164.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00706 | 164.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS0070A | 164.00 | 136.62 | −24.16 | 6.82 | 4.20 | 134.19 | R123FR |  | 29.87 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 31 | P |  |
| AUS | AUS00800 | 164.00 | 136.00 | −23.90 | 7.26 | 4.48 | 132.00 | MODRSS |  | 29.32 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00801 | 164.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00802 | 164.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00803 | 164.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00804 | 164.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00805 | 164.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00806 | 164.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 44 | P |  |
| AUS | AUS00900 | 164.00 | 136.00 | −23.90 | 7.26 | 4.48 | 132.00 | MODRSS |  | 29.32 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00901 | 164.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00902 | 164.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00903 | 164.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00904 | 164.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00905 | 164.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS00906 | 164.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUS0090A | 164.00 | 136.62 | −24.16 | 6.82 | 4.20 | 134.19 | R123FR |  | 29.87 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 32 | P |  |
| AUS | AUSA0000 | 152.00 | 135.36 | −23.95 | 6.89 | 4.83 | 141.15 | R123FR |  | 29.23 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0001 | 152.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0002 | 152.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0003 | 152.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0004 | 152.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0005 | 152.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSA0006 | 152.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 40 | P |  |
| AUS | AUSB0000 | 164.00 | 136.62 | −24.16 | 6.82 | 4.20 | 134.19 | R123FR |  | 29.87 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0001 | 164.00 | 96.83 | −12.19 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0002 | 164.00 | 105.69 | −10.45 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0003 | 164.00 | 110.52 | −66.28 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0004 | 164.00 | 158.94 | −54.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0005 | 164.00 | 159.06 | −31.52 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUS | AUSB0006 | 164.00 | 167.93 | −29.02 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 43 | P |  |
| AUT | AUT01600 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MODRSS |  | 42.19 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| AZE | AZE06400 | 23.20 | 47.47 | 40.14 | 0.93 | 0.60 | 158.14 | MODRSS |  | 46.98 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BDI | BDI27000 | 11.00 | 29.90 | −3.10 | 0.71 | 0.60 | 80.00 | MODRSS |  | 48.15 |  | MODTES | 57.00 | CL |  | 81.0 |  | 27M0G7W |  |  | P |  |
| BEL | BEL01800 | 38.20 | 5.12 | 51.96 | 1.00 | 1.00 | 0.00 | MODRSS |  | 44.44 |  | MODTES | 57.00 | CR |  | 85.5 |  | 27M0G7W |  |  | P |  |
| BEN | BEN23300 | −19.20 | 2.20 | 9.50 | 1.44 | 0.68 | 97.00 | MODRSS |  | 44.54 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BFA | BFA10700 | −30.00 | −1.50 | 12.20 | 1.45 | 1.14 | 29.00 | MODRSS |  | 42.26 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BGD | BGD22000 | 74.00 | 90.30 | 23.60 | 1.46 | 0.84 | 135.00 | MODRSS |  | 43.56 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BHR | BHR25500 | 34.00 | 50.50 | 26.10 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 83.0 |  | 27M0G7W |  |  | P |  |
| BIH | BIH14800 | 56.00 | 18.22 | 43.97 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BLR | BLR06200 | 37.80 | 28.04 | 53.18 | 1.17 | 0.60 | 9.68 | MODRSS |  | 45.96 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BOT | BOT29700 | −0.80 | 23.30 | −22.20 | 2.13 | 1.50 | 36.00 | MODRSS |  | 39.40 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BRM | BRM29800 | 104.00 | 96.97 | 18.68 | 3.33 | 1.66 | 91.63 | MODRSS |  | 37.02 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BRU | BRU3300A | 74.00 | 114.70 | 4.40 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BTN | BTN03100 | 86.00 | 90.44 | 27.05 | 0.72 | 0.60 | 175.47 | MODRSS |  | 48.11 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| BUL | BUL02000 | −1.20 | 25.00 | 43.00 | 1.04 | 0.60 | 165.00 | MODRSS |  | 46.50 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| CAF | CAF25800 | −13.20 | 21.00 | 6.30 | 2.25 | 1.68 | 31.00 | MODRSS |  | 38.67 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CBG | CBG29900 | 86.00 | 104.89 | 12.79 | 1.12 | 0.94 | 32.89 | MODRSS |  | 44.22 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CHN | CHN15400 | 62.00 | 101.90 | 33.50 | 5.10 | 2.80 | 143.00 | MODRSS |  | 32.90 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 45 | P |  |
| CHN | CHN15500 | 62.00 | 101.90 | 33.50 | 5.10 | 2.80 | 143.00 | MODRSS |  | 32.90 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 45 | P |  |
| CHN | CHN15800 | 134.00 | 113.21 | 34.27 | 6.40 | 3.16 | 10.74 | MODRSS |  | 31.39 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 46 | P |  |
| CHN | CHN15900 | 134.00 | 113.21 | 34.27 | 6.40 | 3.16 | 10.74 | MODRSS |  | 31.39 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 46 | P |  |
| CHN | CHN16000 | 92.20 | 108.10 | 33.70 | 5.00 | 4.00 | 148.00 | MODRSS |  | 31.44 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 47 | P |  |
| CHN | CHN16100 | 92.20 | 108.10 | 33.70 | 5.00 | 4.00 | 148.00 | MODRSS |  | 31.44 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 47 | P |  |
| CHN | CHN20000 | 122.00 | 113.55 | 22.20 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CLN | CLN21900 | 50.00 | 80.60 | 7.70 | 1.18 | 0.60 | 106.00 | MODRSS |  | 45.95 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| COD | COD__100 | −19.20 | 21.85 | −3.40 |  |  |  | CB_RSS_CODA |  | 38.36 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| COG | COG23500 | −13.20 | 14.60 | −0.70 | 2.02 | 1.18 | 59.00 | MODRSS |  | 40.67 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| COM | COM20700 | 29.00 | 44.10 | −12.10 | 0.76 | 0.60 | 149.00 | MODRSS |  | 47.86 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CPV | CPV30100 | −33.50 | −24.12 | 16.09 | 0.77 | 0.63 | 94.46 | MODRSS |  | 47.56 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P | 5, 6 |
| CTI | CTI23700 | −24.80 | −5.66 | 7.39 | 1.45 | 1.29 | 126.59 | MODRSS |  | 41.73 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CVA | CVA08300 | −1.20 | 13.02 | 42.09 | 0.75 | 0.66 | 20.53 | MODRSS |  | 47.48 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CVA | CVA08500 | −1.20 | 13.02 | 42.09 | 0.75 | 0.66 | 20.53 | MODRSS |  | 47.48 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CYP | CYP08600 | −1.20 | 33.45 | 35.12 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CZE | CZE14401 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CZE | CZE14402 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| CZE | CZE14403 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 37 | P |  |
| D | D 08700 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MODRSS |  | 42.19 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| DJI | DJI09900 | 16.80 | 42.68 | 11.68 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| DNK | DNK__100 | −25.20 | 5.28 | 61.83 |  |  |  | CB_RSS_DNKA |  | 48.88 |  | MODTES | 57.00 | CL |  | 79.5 |  | 27M0G7W |  |  | P |  |
| DNK | DNK09000 | −33.50 | 14.34 | 61.72 | 1.83 | 0.60 | 151.50 | MODRSS |  | 44.05 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| DNK | DNK09100 | −33.50 | −14.94 | 63.79 | 1.52 | 0.60 | 168.57 | MODRSS |  | 44.86 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| E | E____100 | −30.00 | −9.40 | 34.15 |  |  |  | CB_RSS_E__A |  | 44.79 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 01 | P | 6 |
| E | HISP27D4 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | R13TES | 55.00 | CR |  | 82.5 |  | 27M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISP27D6 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | R13TES | 58.50 | CR |  | 83.5 |  | 27M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISP33D4 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | MODTES | 55.00 | CR |  | 82.5 |  | 33M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISP33D6 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | MODTES | 58.50 | CR |  | 83.5 |  | 33M0G7W-- | HISPASAT-1 | 01 | PE |  |
| E | HISPASA4 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | R13TES | 55.00 | CR |  | 82.5 |  | 27M0F8W | HISPASAT-1 | 01 | PE |  |
| E | HISPASA6 | −30.00 | −3.10 | 39.90 |  |  |  |  | ECO | 43.00 | 18.70 | R13TES | 58.50 | CR |  | 83.5 |  | 27M0F8W | HISPASAT-1 | 01 | PE |  |
| EGY | EGY02600 | −7.00 | 29.70 | 26.80 | 2.33 | 1.72 | 136.00 | MODRSS |  | 38.42 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 12 | P |  |
| ERI | ERI09200 | 22.80 | 39.41 | 14.98 | 1.67 | 0.95 | 145.49 | MODRSS |  | 42.44 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| EST | EST06100 | 44.50 | 25.40 | 59.18 | 0.67 | 0.60 | 5.99 | MODRSS |  | 48.42 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| F | F 09300 | −7.00 | 3.30 | 45.37 | 2.18 | 1.20 | 156.36 | MODRSS |  | 40.27 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 21 | P |  |
| F | F____100 | −7.00 | 29.16 | 13.43 |  |  |  | CB_RSS_F__A |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 12 | P |  |
| F | F____200 | 140.00 | 174.50 | −17.30 |  |  |  | CB_RSS_F__B |  | 45.80 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7F | P |  |
| F | F____300 | 140.00 | 174.65 | −17.65 |  |  |  | CB_RSS_F__C |  | 47.97 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7F | P |  |
| F | OCE10100 | −160.00 | −145.00 | −16.30 | 4.34 | 3.54 | 4.00 | MODRSS |  | 32.58 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| FIN | FIN10300 | 22.80 | 17.61 | 61.54 | 2.18 | 0.90 | 11.59 | MODRSS |  | 41.53 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 52 | P |  |
| FIN | FIN10400 | 22.80 | 17.61 | 61.54 | 2.18 | 0.90 | 11.59 | MODRSS |  | 41.53 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 52 | P |  |
| FJI | FJI19300 | −178.00 | 179.62 | −17.87 | 1.16 | 0.92 | 155.22 | MODRSS |  | 44.16 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| FSM | FSM00000 | 158.00 | 151.90 | 5.48 | 5.15 | 1.57 | 167.00 | MODRSS |  | 35.38 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| G | G 02700 | −33.50 | −3.50 | 53.80 | 1.84 | 0.72 | 142.00 | MODRSS |  | 43.23 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P | 5, 6 |
| GAB | GAB26000 | −13.20 | 11.80 | −0.60 | 1.43 | 1.12 | 64.00 | MODRSS |  | 42.40 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| GEO | GEO06400 | 23.20 | 43.35 | 42.27 | 1.11 | 0.60 | 161.21 | MODRSS |  | 46.23 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| GMB | GMB30200 | −37.20 | −15.10 | 13.40 | 0.79 | 0.60 | 4.00 | MODRSS |  | 47.69 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| GNB | GNB30400 | −30.00 | −15.00 | 12.00 | 0.90 | 0.60 | 172.00 | MODRSS |  | 47.12 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| GNE | GNE30300 | −18.80 | 10.30 | 1.50 | 0.68 | 0.60 | 10.00 | MODRSS |  | 48.34 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| GRC | GRC10500 | −1.20 | 24.52 | 38.11 | 1.70 | 0.95 | 152.55 | MODRSS |  | 42.37 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| GUI | GUI19200 | −37.00 | −11.00 | 10.20 | 1.58 | 1.04 | 147.00 | MODRSS |  | 42.29 |  | MODTES | 57.00 | CR |  | 85.0 |  | 27M0G7W |  |  | P |  |
| HNG | HNG10601 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| HNG | HNG10602 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| HNG | HNG10603 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 37 | P |  |
| HOL | HOL21300 | 38.20 | 5.12 | 51.96 | 1.00 | 1.00 | 0.00 | MODRSS |  | 44.44 |  | MODTES | 57.00 | CL |  | 85.5 |  | 27M0G7W |  |  | P |  |
| HRV | HRV14801 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| HRV | HRV14802 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| HRV | HRV14803 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 37 | P |  |
| I | I 08200 | 9.00 | 12.67 | 40.74 | 1.99 | 1.35 | 144.20 | MODRSS |  | 40.14 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| IND | IND03700 | 68.00 | 93.00 | 25.50 | 1.46 | 1.13 | 40.00 | MODRSS |  | 42.27 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| IND | IND04701 | 68.00 | 93.30 | 11.10 | 1.92 | 0.60 | 96.00 | MODRSS |  | 43.83 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7E | P |  |
| IND | IND04702 | 68.00 | 93.30 | 11.10 | 1.92 | 0.60 | 96.00 | MODRSS |  | 43.83 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7E | P |  |
| IND | INDA_101 | 55.80 | 76.16 | 14.72 |  |  |  | CB_RSS_INDA |  | 45.66 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7G | P |  |
| IND | INDA_102 | 55.80 | 76.16 | 14.72 |  |  |  | CB_RSS_INDA |  | 45.66 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7G | P |  |
| IND | INDB_101 | 55.80 | 83.67 | 23.73 |  |  |  | CB_RSS_INDB |  | 43.13 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7H | P |  |
| IND | INDB_102 | 55.80 | 83.67 | 23.73 |  |  |  | CB_RSS_INDB |  | 43.13 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7H | P |  |
| IND | INDD_100 | 68.00 | 74.37 | 29.16 |  |  |  | CB_RSS_INDD |  | 41.79 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| INS | INS02800 | 80.20 | 113.60 | −1.40 | 6.73 | 3.33 | 160.00 | MODRSS |  | 30.94 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| INS | INS03501 | 104.00 | 115.20 | −1.70 | 9.14 | 3.43 | 170.00 | MODRSS |  | 29.48 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7D | P |  |
| INS | INS03502 | 104.00 | 115.20 | −1.70 | 9.14 | 3.43 | 170.00 | MODRSS |  | 29.48 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7D | P |  |
| IRL | IRL21100 | −37.20 | −8.25 | 53.22 | 0.72 | 0.60 | 157.56 | MODRSS |  | 48.08 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| IRN | IRN10900 | 34.00 | 54.20 | 32.40 | 3.82 | 1.82 | 149.00 | MODRSS |  | 36.03 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| ISL | ISL04900 | −33.50 | −19.00 | 64.90 | 1.00 | 0.60 | 177.00 | MODRSS |  | 46.67 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| ISL | ISL05000 | −33.50 | −14.94 | 63.79 | 1.52 | 0.60 | 168.57 | MODRSS |  | 44.86 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| ISR | ISR11000 | −4.00 | 34.95 | 31.32 | 0.73 | 0.60 | 110.02 | MODRSS |  | 48.03 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| J | 000BS−3N | 109.85 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | MODRSS |  | 33.80 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0F8W | BS-3N | 02 | PE |  |
| J | J 10985 | 109.85 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | MODRSS |  | 33.80 |  | MODTES | 57.00 | CR |  | 87.0 |  | 34M5G7W |  | 02 | P |  |
| J | J 11100 | 110.00 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | MODRSS |  | 33.80 |  | MODTES | 57.00 | CR |  | 87.0 |  | 34M5G7W |  | 02 | P |  |
| J | J 1110E | 110.00 | 134.50 | 31.50 | 3.52 | 3.30 | 68.00 | MODRSS |  | 33.80 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0F8W | BS-3M | 02 | PE |  |
| JOR | JOR22400 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MODRSS |  | 43.19 |  | MODTES | 57.00 | CL |  | 85.0 |  | 27M0G7W |  |  | P |  |
| KAZ | KAZ06600 | 56.40 | 65.73 | 46.40 | 4.58 | 1.76 | 177.45 | MODRSS |  | 35.38 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| KEN | KEN24900 | −0.80 | 37.99 | 0.88 | 2.06 | 1.30 | 99.68 | MODRSS |  | 40.17 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| KGZ | KGZ07000 | 50.00 | 73.91 | 41.32 | 1.47 | 0.64 | 5.05 | MODRSS |  | 44.75 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| KIR | KIR__100 | 176.00 | −170.31 | −0.56 |  |  |  | CB_RSS_KIRA |  | 42.60 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| KOR | KOR11201 | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | MODRSS |  | 43.43 |  | MODTES | 57.00 | CL |  | 89.0 |  | 27M0G7W |  | 03 | P |  |
| KOR | KOR11202 | 116.00 | 127.50 | 36.00 | 1.24 | 1.02 | 168.00 | MODRSS |  | 43.43 |  | MODTES | 57.00 | CR |  | 89.0 |  | 27M0G7W |  | 03 | P |  |
| KRE | KRE28600 | 140.00 | 128.45 | 40.32 | 1.63 | 0.68 | 18.89 | MODRSS |  | 44.00 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  |  | P |  |
| KWT | KWT11300 | 11.00 | 47.48 | 29.12 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 83.0 |  | 27M0G7W |  |  | P |  |
| LAO | LAO28400 | 122.20 | 103.71 | 18.17 | 1.87 | 1.03 | 123.99 | MODRSS |  | 42.18 |  | MODTES | 57.00 | CR |  | 84.0 |  | 33M0G7W |  |  | P |  |
| LBN | LBN27900 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MODRSS |  | 43.19 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| LBR | LBR24400 | −33.50 | −9.30 | 6.60 | 1.22 | 0.70 | 133.00 | MODRSS |  | 45.13 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P | 5, 6 |
| LBY | LBY28021 | −24.80 | 17.50 | 26.30 | 3.68 | 1.84 | 130.00 | MODRSS |  | 36.14 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| LIE | LIE25300 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MODRSS |  | 42.19 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| LSO | LSO30500 | 4.80 | 27.80 | −29.80 | 0.66 | 0.60 | 36.00 | MODRSS |  | 48.47 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| LTU | LTU06100 | 23.20 | 24.52 | 56.11 |  |  |  | CB_RSS_LTUA |  | 47.92 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| LUX | LUX11400 | 28.20 | 5.21 | 49.20 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 09 | P |  |
| LVA | LVA06100 | 23.20 | 24.52 | 56.11 |  |  |  | CB_RSS_LVAA |  | 47.92 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MAU | MAU__100 | 29.00 | 58.61 | −15.88 |  |  |  | CB_RSS_MAUA |  | 41.42 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MCO | MCO11600 | 34.20 | 7.40 | 43.70 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 81.0 |  | 27M0G7W |  |  | P |  |
| MDA | MDA06300 | 50.00 | 28.45 | 46.99 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MDG | MDG23600 | 29.00 | 46.20 | −18.60 | 2.57 | 0.80 | 67.00 | MODRSS |  | 41.32 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MHL | MHL00000 | 146.00 | 167.64 | 9.83 | 2.07 | 0.90 | 157.42 | MODRSS |  | 41.75 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MKD | MKD14800 | 22.80 | 21.53 | 41.50 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MLA | MLA__100 | 91.50 | 108.07 | 3.92 |  |  |  | CB_RSS_MLAA |  | 41.75 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MLD | MLD30600 | 50.00 | 73.10 | 6.00 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MLI | MLI__100 | −19.20 | −4.80 | 16.10 |  |  |  | CB_RSS_MLIA |  | 41.11 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  |  | P |  |
| MLT | MLT14700 | 22.80 | 14.40 | 35.90 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| MNG | MNG24800 | 74.00 | 101.95 | 46.79 | 3.32 | 1.04 | 169.27 | MODRSS |  | 39.07 |  | MODTES | 59.92 | CL |  | 86.9 |  | 27M0G7W |  |  | P |  |
| MRC | MRC20900 | −25.20 | −8.90 | 28.90 | 3.96 | 1.55 | 50.00 | MODRSS |  | 36.57 |  | MODTES | 57.00 | CR |  | 80.0 |  | 27M0G7W |  |  | P |  |
| MTN | MTN__100 | −36.80 | −11.24 | 20.91 |  |  |  | CB_RSS_MTNA |  | 37.55 |  | MODTES | 57.00 | CR |  | 86.0 |  | 27M0G7W |  |  | P |  |
| MWI | MWI30800 | 4.80 | 33.79 | −13.25 | 1.56 | 0.70 | 92.69 | MODRSS |  | 44.10 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| NGR | NGR11500 | −37.20 | 7.63 | 16.97 | 2.20 | 1.80 | 100.58 | MODRSS |  | 38.47 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| NOR | NOR12000 | −0.80 | 16.70 | 61.58 | 1.84 | 0.95 | 177.31 | MODRSS |  | 42.02 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 06 | P |  |
| NOR | NOR12100 | −0.80 | 16.70 | 61.58 | 1.84 | 0.95 | 177.31 | MODRSS |  | 42.02 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 06 | P |  |
| NRU | NRU30900 | 134.00 | 167.00 | −0.50 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| NZL | NZL__100 | 158.00 | −174.35 | −24.30 |  |  |  | CB_RSS_NZLA |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| OMA | OMA12300 | 17.20 | 55.60 | 21.00 | 1.88 | 1.02 | 100.00 | MODRSS |  | 41.62 |  | MODTES | 57.00 | CL |  | 85.0 |  | 27M0G7W |  |  | P |  |
| PHL | PHL28500 | 98.00 | 121.30 | 11.10 | 3.46 | 1.76 | 99.00 | MODRSS |  | 36.60 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| PLW | PLW00000 | 140.00 | 132.98 | 5.51 | 1.30 | 0.60 | 55.41 | MODRSS |  | 45.53 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| POL | POL13200 | 50.00 | 19.71 | 52.18 | 1.22 | 0.63 | 16.12 | MODRSS |  | 45.59 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| POR | POR__100 | −37.00 | −15.92 | 37.65 |  |  |  | CB_RSS_PORA |  | 47.17 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| PSE | YYY00001 | −13.20 | 34.99 | 31.86 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 80.5 |  | 27M0G7W |  |  | P | 8 |
| QAT | QAT24700 | 20.00 | 51.59 | 25.35 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| ROU | ROU13600 | 50.00 | 25.12 | 45.75 | 1.17 | 0.73 | 9.52 | MODRSS |  | 45.15 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| RRW | RRW31000 | 11.00 | 30.00 | −2.10 | 0.66 | 0.60 | 42.00 | MODRSS |  | 48.47 |  | MODTES | 57.00 | CR |  | 81.0 |  | 27M0G7W |  |  | P |  |
| RUS | RSTREA11 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0F8W | RST-1 | 05 | PE |  |
| RUS | RSTREA12 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0F8W | RST-1 | 05 | PE |  |
| RUS | RSTRED11 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RST-1 | 05 | PE |  |
| RUS | RSTRED12 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RST-1 | 05 | PE |  |
| RUS | RSTRSD11 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD12 | 36.00 | 38.00 | 53.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RST-1 | 05 | P |  |
| RUS | RSTRSD21 | 56.00 | 65.00 | 63.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RST-2 | 14 | P |  |
| RUS | RSTRSD22 | 56.00 | 65.00 | 63.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RST-2 | 14 | P |  |
| RUS | RSTRSD31 | 86.00 | 97.00 | 62.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RST-3 | 33 | P |  |
| RUS | RSTRSD32 | 86.00 | 97.00 | 62.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RST-3 | 33 | P |  |
| RUS | RSTRSD51 | 140.00 | 158.00 | 56.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RST-5 | 35 | P |  |
| RUS | RSTRSD52 | 140.00 | 158.00 | 56.00 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RST-5 | 35 | P |  |
| RUS | RUS00401 | 110.00 | 118.22 | 51.52 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W | RUS-4 | 34 | P |  |
| RUS | RUS00402 | 110.00 | 118.22 | 51.52 |  |  |  |  | COP | 38.40 | 8.40 | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W | RUS-4 | 34 | P |  |
| S | S 13800 | 5.00 | 17.00 | 61.50 | 2.00 | 1.00 | 10.00 | MODRSS |  | 41.44 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 04 | P |  |
| S | S 13900 | 5.00 | 17.00 | 61.50 | 2.00 | 1.00 | 10.00 | MODRSS |  | 41.44 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 04 | P |  |
| SEY | SEY00000 | 42.50 | 51.86 | −7.23 | 2.43 | 1.04 | 27.51 | MODRSS |  | 40.44 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SLM | SLM00000 | 128.00 | 159.27 | −8.40 | 1.35 | 1.08 | 118.59 | MODRSS |  | 42.81 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SMO | SMO05700 | −178.00 | −171.70 | −13.87 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SMR | SMR31100 | −36.80 | 12.50 | 43.90 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 83.0 |  | 27M0G7W |  |  | P |  |
| SNG | SNG15100 | 88.00 | 103.86 | 1.42 | 0.92 | 0.72 | 175.12 | MODRSS |  | 46.25 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SRB | SRB14800 | −7.00 | 20.50 | 43.98 | 0.91 | 0.60 | 145.16 | MODRSS |  | 47.07 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SRL | SRL25900 | −33.50 | −11.80 | 8.60 | 0.78 | 0.68 | 114.00 | MODRSS |  | 47.20 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| STP | STP24100 | −7.00 | 7.00 | 0.80 | 0.60 | 0.60 | 0.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SUI | SUI14000 | −18.80 | 10.31 | 49.47 | 1.82 | 0.92 | 151.78 | MODRSS |  | 42.19 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SVK | SVK14401 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SVK | SVK14402 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  |  | P |  |
| SVK | SVK14403 | −12.80 | 16.77 | 46.78 | 1.71 | 0.89 | 149.15 | MODRSS |  | 42.64 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 37 | P |  |
| SVN | SVN14800 | 33.80 | 15.01 | 46.18 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  |  | P |  |
| SWZ | SWZ31300 | 4.80 | 31.39 | −26.44 | 0.60 | 0.60 | 90.00 | MODRSS |  | 48.88 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  |  | P |  |
| SYR | SYR22900 | 11.00 | 37.55 | 34.02 | 1.47 | 0.91 | 73.16 | MODRSS |  | 43.19 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 53 | P |  |
| SYR | SYR33900 | 11.00 | 37.60 | 34.20 | 1.32 | 0.88 | 74.00 | MODRSS |  | 43.80 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 53 | P |  |
| TCD | TCD14300 | 17.00 | 18.39 | 15.52 | 3.21 | 2.05 | 83.26 | MODRSS |  | 36.26 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| THA | THA14200 | 98.00 | 100.75 | 12.88 | 2.80 | 1.82 | 93.77 | MODRSS |  | 37.38 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| TJK | TJK06900 | 38.00 | 71.14 | 38.41 | 1.21 | 0.73 | 155.31 | MODRSS |  | 45.00 |  | MODTES | 57.00 | CL |  | 82.0 |  | 27M0G7W |  |  | P |  |
| TKM | TKM06800 | 50.00 | 59.24 | 38.83 | 2.26 | 1.02 | 166.64 | MODRSS |  | 40.81 |  | MODTES | 57.00 | CL |  | 85.7 |  | 27M0G7W |  |  | P |  |
| TLS | TLS00000 | 128.00 | 126.03 | −8.72 | 0.66 | 0.60 | 13.92 | MODRSS |  | 48.50 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| TON | TON21500 | 170.75 | −175.23 | −18.19 | 1.59 | 0.60 | 71.33 | MODRSS |  | 44.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| TUN | TUN15000 | −25.20 | 9.50 | 33.50 | 1.88 | 0.72 | 135.00 | MODRSS |  | 43.13 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 55 | P |  |
| TUN | TUN27200 | −25.20 | 2.50 | 32.00 | 3.59 | 1.75 | 175.00 | MODRSS |  | 36.47 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 55 | P |  |
| TUR | TUR14500 | 42.00 | 35.14 | 38.99 | 3.19 | 1.10 | 0.03 | MODRSS |  | 39.00 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 36 | P |  |
| TUV | TUV00000 | 176.00 | 177.61 | −7.11 | 0.94 | 0.60 | 137.58 | MODRSS |  | 46.93 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| TZA | TZA22500 | 11.00 | 34.60 | −6.20 | 2.41 | 1.72 | 129.00 | MODRSS |  | 38.27 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| UAE | UAE27400 | 52.50 | 53.98 | 24.37 | 1.23 | 0.84 | 6.62 | MODRSS |  | 44.31 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| UGA | UGA05100 | 17.00 | 32.20 | 1.04 | 1.50 | 1.02 | 68.73 | MODRSS |  | 42.62 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| UKR | UKR06300 | 38.20 | 31.82 | 48.19 | 2.32 | 0.95 | 177.32 | MODRSS |  | 41.01 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| USA | GUM33101 | 122.00 | 155.56 | 13.21 |  |  |  | CB_RSS_GUMA |  | 43.61 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 7C | P |  |
| USA | GUM33102 | 122.00 | 155.56 | 13.21 |  |  |  | CB_RSS_GUMA |  | 43.61 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 7C | P |  |
| USA | MRA33200 | 121.80 | 155.56 | 13.21 |  |  |  | CB_RSS_MRAA |  | 43.61 |  | MODTES | 57.00 | CR |  | 91.0 |  | 27M0G7W |  |  | P |  |
| USA | PLM33200 | 170.00 | −145.55 | 19.50 |  |  |  | CB_RSS_PLMA |  | 39.35 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  |  | P |  |
| USA | USAA_101 | 170.00 | −145.55 | 19.50 |  |  |  | CB_RSS_USAA |  | 39.35 |  | MODTES | 57.00 | CR |  | 87.0 |  | 27M0G7W |  | 7A | P |  |
| USA | USAA_102 | 170.00 | −145.55 | 19.50 |  |  |  | CB_RSS_USAA |  | 39.35 |  | MODTES | 57.00 | CL |  | 87.0 |  | 27M0G7W |  | 7A | P |  |
| UZB | UZB07100 | 33.80 | 63.80 | 41.21 | 2.56 | 0.89 | 159.91 | MODRSS |  | 40.84 |  | MODTES | 57.00 | CR |  | 82.0 |  | 27M0G7W |  |  | P |  |
| VTN | VTN32500 | 107.00 | 106.84 | 14.21 | 3.43 | 1.76 | 109.43 | MODRSS |  | 36.64 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| VUT | VUT12801 | 140.00 | 168.00 | −16.40 | 1.52 | 0.68 | 87.00 | MODRSS |  | 44.30 |  | MODTES | 57.00 | CL |  | 84.0 |  | 27M0G7W |  | 7B | P |  |
| VUT | VUT12802 | 140.00 | 168.00 | −16.40 | 1.52 | 0.68 | 87.00 | MODRSS |  | 44.30 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  | 7B | P |  |
| ZMB | ZMB31400 | −0.80 | 27.50 | −13.10 | 2.38 | 1.48 | 39.00 | MODRSS |  | 38.98 |  | MODTES | 57.00 | CR |  | 84.0 |  | 27M0G7W |  |  | P |  |
| ZWE | ZWE13500 | −0.80 | 29.60 | −18.80 | 1.46 | 1.36 | 37.00 | MODRSS |  | 41.47 |  | MODTES | 57.00 | CL |  | 85.0 |  | 27M0G7W |  |  | P |  |

COLUMN HEADINGS OF TABLES 3B1 AND 3B2

Col. 1	Nominal orbital position, in degrees and hundredths of a degree from the Greenwich meridian (negative values indicate longitudes which are west of the Greenwich meridian; positive values indicate longitudes which are east of the Greenwich meridian).

Col. 2	Notifying administration symbol.

Col. 3	Beam identification (Column 2, normally, contains the symbol designating the administration or the geographical area taken from Table B1 of the Preface to the International Frequency List, followed by the symbol designating the service area).

Col. 4	Polarization (CL – circular left, CR – circular right).

Col. 5	Channel number/indication of minimum equivalent protection margin (EPM) for a given assignment derived from the set of values for all test points belonging to the given beam.

TABLE 3B1

Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in
the frequency band 14.5-14.8 GHz (sorted by orbital position)

| 1 | 2 | 3 | 4 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital position | Admin. symbol | Beam Identification | Polarization type | Channel number |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
|  |  |  |  | Minimum equivalent protection margin |  |  |  |  |  |  |  |  |  |  |  |  |
| −37.00 | SEN | SEN22201 | CL |  |  |  | 40.8 |  | 39.6 |  | 39.6 |  | 39.6 |  | 39.6 |  |
| −37.00 | SEN | SEN22202 | CR |  |  |  |  | 39.6 |  | 39.6 |  | 39.6 |  | 39.6 |  | 40.7 |
| −30.00 | TGO | TGO22601 | CL |  |  |  | 15.0 |  | 14.1 |  | 14.1 |  | 14.1 |  | 14.1 |  |
| −30.00 | TGO | TGO22602 | CR |  |  |  |  | 14.1 |  | 14.1 |  | 14.1 |  | 14.1 |  | 15.0 |
| −25.00 | GHA | GHA10801 | CR |  |  |  | 14.9 |  | 14.1 |  | 14.1 |  | 14.1 |  | 14.1 |  |
| −25.00 | GHA | GHA10802 | CL |  |  |  |  | 14.1 |  | 14.1 |  | 14.1 |  | 14.1 |  | 14.9 |
| −19.20 | NIG | NIG11901 | CR |  |  |  | 6.4 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  |
| −19.20 | NIG | NIG11902 | CL |  |  |  |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 6.4 |
| −18.80 | NMB | NMB02501 | CL |  |  |  | 6.9 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  |
| −18.80 | NMB | NMB02502 | CR |  |  |  |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 6.9 |
| −13.00 | CME | CME30001 | CL |  |  |  | 17.2 |  | 16.3 |  | 16.3 |  | 16.3 |  | 16.3 |  |
| −13.00 | CME | CME30002 | CR |  |  |  |  | 16.3 |  | 16.3 |  | 16.3 |  | 16.3 |  | 17.2 |
| −7.00 | SDN | SDN__101 | CL |  |  |  | 27.1 |  | 26.1 |  | 26.1 |  | 26.1 |  | 26.1 |  |
| −7.00 | SDN | SDN__102 | CR |  |  |  |  | 26.1 |  | 26.1 |  | 26.1 |  | 26.1 |  | 27.1 |
| −1.00 | MOZ | MOZ30701 | CL |  |  |  | 16.6 |  | 15.7 |  | 15.7 |  | 15.7 |  | 15.7 |  |
| −1.00 | MOZ | MOZ30702 | CR |  |  |  |  | 15.7 |  | 15.7 |  | 15.7 |  | 15.7 |  | 16.6 |
| 4.80 | AFS | AFS02101 | CL |  |  |  | 11.9 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  |
| 4.80 | AFS | AFS02102 | CR |  |  |  |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.0 |  | 11.9 |
| 11.00 | YEM | YEM__101 | CR |  |  |  | 47.8 |  | 47.3 |  | 47.3 |  | 47.3 |  | 47.3 |  |
| 11.00 | YEM | YEM__102 | CL |  |  |  |  | 47.3 |  | 47.3 |  | 47.3 |  | 47.3 |  | 47.8 |
| 34.00 | IRN | IRN10901 | CR |  | 15.2 |  | 13.9 |  | 13.9 |  | 13.9 |  | 13.9 |  | 13.9 |  |
| 34.00 | IRN | IRN10902 | CL |  |  | 14.3 |  | 13.9 |  | 13.9 |  | 13.9 |  | 13.9 |  | 14.8 |
| 36.00 | ETH | ETH09201 | CL |  |  |  | 2.3 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  |
| 36.00 | ETH | ETH09202 | CR |  |  |  |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.4 |  | 2.3 |
| 37.80 | SOM | SOM31201 | CL |  |  |  | 0.0 |  | −0.3 |  | −0.3 |  | −0.3 |  | −0.3 |  |
| 37.80 | SOM | SOM31202 | CR |  |  |  |  | −0.3 |  | −0.3 |  | −0.3 |  | −0.3 |  | 1.6 |
| 38.20 | PAK | PAK12701 | CR |  | 14.2 |  | 3.2 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  |
| 38.20 | PAK | PAK12702 | CL |  |  | 4.2 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 3.3 |
| 42.50 | SEY | SEY00001 | CL |  |  |  | 36.3 |  | 35.3 |  | 35.3 |  | 35.3 |  | 35.3 |  |
| 42.50 | SEY | SEY00002 | CR |  |  |  |  | 35.3 |  | 35.3 |  | 35.3 |  | 35.3 |  | 36.4 |
| 50.00 | IRQ | IRQ25601 | CL |  |  |  | −0.1 |  | −0.1 |  | −0.1 |  | −0.1 |  | −0.1 |  |
| 50.00 | IRQ | IRQ25602 | CR |  |  |  |  | −0.1 |  | −0.1 |  | −0.1 |  | −0.1 |  | 2.4 |
| 50.00 | NPL | NPL12201 | CR |  | 38.2 |  | 3.9 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  |
| 50.00 | NPL | NPL12202 | CL |  |  | 4.6 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 3.9 |
| 55.80 | IND | INDA_101 | CR |  | 25.7 |  | 24.7 |  | 24.7 |  | 24.7 |  | 24.7 |  | 24.7 |  |
| 55.80 | IND | INDA_102 | CL |  |  | 24.7 |  | 24.7 |  | 24.7 |  | 24.7 |  | 24.7 |  | 25.6 |
| 116.00 | KOR | KO11201D | CL | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  |  |
| 116.00 | KOR | KOR11201 | CL | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  | 7.5 |  |  |
| 122.00 | CHN | CHN19001 | CL |  | 47.7 |  | 47.7 |  | 47.7 |  | 47.7 |  | 47.7 |  | 50.7 |  |
| 122.00 | CHN | CHN19002 | CR |  |  | 42.0 |  | 42.0 |  | 42.0 |  | 42.0 |  | 42.0 |  | 999.9 |
| 134.00 | PNG | PNG13101 | CR |  | 26.1 |  | 25.2 |  | 25.2 |  | 25.2 |  | 25.2 |  | 25.2 |  |
| 134.00 | PNG | PNG13102 | CL |  |  | 25.2 |  | 25.2 |  | 25.2 |  | 25.2 |  | 25.2 |  | 26.1 |
| 140.00 | USA | USAC_101 | CL |  | 19.4 |  | 18.6 |  | 18.6 |  | 18.6 |  | 18.6 |  | 18.6 |  |
| 140.00 | USA | USAC_102 | CR |  |  | 18.6 |  | 18.6 |  | 18.6 |  | 18.6 |  | 18.6 |  | 19.4 |

TABLE 3B2

Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in the frequency band 17.3-18.1 GHz (sorted by orbital position)

| 1 | 2 | 3 | 4 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital Position | Admin. symbol | Beam Identification | Polarization type | Channel number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
|  |  |  |  | Minimum equivalent protection margin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| –178.00 | FJI | FJI19300 | CR | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  | 3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −178.00 | SMO | SMO05700 | CL | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −160.00 | F | OCE10100 | CL |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −37.20 | GMB | GMB30200 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.3 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  | 3.2 |  |
| −37.20 | IRL | IRL21100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.3 |  | 10.2 |  | 10.2 |  | 10.3 |  | 10.3 |  |
| −37.20 | NGR | NGR11500 | CL | 1.8 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −37.00 | AND | AND34100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 13.3 |
| −37.00 | GUI | GUI19200 | CR |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −37.00 | POR | POR__100 | CR | 2.4 |  | 0.0 |  | −0.1 |  | 0.0 |  | −0.1 |  | 0.0 |  | −0.1 |  | 0.0 |  | −0.1 |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −36.80 | MTN | MTN__100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 2.0 |
| −36.80 | SMR | SMR31100 | CL |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 10.6 |  | 12.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −33.50 | CPV | CPV30100 | CL |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 14.3 |  | 11.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −33.50 | DNK | DNK09000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1 |  |  |  |  |  | 5.1 |  |  |  |  |  |  |
| −33.50 | DNK | DNK09100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1 |  |  |  | 1.1 |  |  |  |  |
| −33.50 | G | G 02700 | CR |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 6.4 |  | 4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −33.50 | ISL | ISL04900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | −0.1 |  | −0.1 |  | 1.8 |  | 7.5 |  | 2.8 |  | 1.8 |  | 1.8 |  | 2.8 |  | 14.1 |  |
| −33.50 | ISL | ISL05000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1 |  | 1.1 |  | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −33.50 | LBR | LBR24400 | CR | 10.6 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −33.50 | SRL | SRL25900 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.7 |  | 11.8 |  | 11.8 |  | 11.4 |  | 13.1 |  | 13.7 |  | 11.4 |  | 11.4 |  | 13.7 |  | 17.1 |  |
| −30.00 | BFA | BFA10700 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 12.0 |  | 14.9 |
| −30.00 | E | E____100 | CR | 7.9 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISP27D4 | CR | 10.2 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISP27D6 | CR | 11.2 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISP33D4 | CR | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISP33D6 | CR | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISPASA4 | CR | 10.2 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  | 10.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | E | HISPASA6 | CR | 11.2 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  | 11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −30.00 | GNB | GNB30400 | CL | 15.6 |  | 16.9 |  | 15.2 |  | 16.9 |  | 15.2 |  | 16.9 |  | 15.2 |  | 16.9 |  | 15.2 |  | 16.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −25.20 | DNK | DNK__100 | CL | 1.2 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.6 |  | −0.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −25.20 | MRC | MRC20900 | CR |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | −1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −25.20 | TUN | TUN15000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.2 |
| −25.20 | TUN | TUN27200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −1.2 |  |  |  | −1.2 |  |  |
| −24.80 | AGL | AGL29500 | CR | 9.2 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  | 6.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −24.80 | ALG | ALG25152 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.1 |  | −1.0 |  | −1.1 |  | 0.0 |
| −24.80 | CTI | CTI23700 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.5 |  | 5.5 |  | 5.5 |  | 5.5 |  | 5.5 |  | 5.5 |  | 5.3 |  | 5.2 |  | 5.3 |  | 5.2 |  |
| −24.80 | LBY | LBY28021 | CL |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | 0.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −19.20 | BEN | BEN23300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  | 9.4 |  |
| −19.20 | COD | COD__100 | CL | 4.5 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −19.20 | MLI | MLI__100 | CR |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 4.2 |  | 2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −18.80 | AUT | AUT01600 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.3 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.2 |  |
| −18.80 | D | D 08700 | CR | 2.5 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −18.80 | GNE | GNE30300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 14.7 |
| −18.80 | LIE | LIE25300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 2.5 |
| −18.80 | SUI | SUI14000 | CL |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −13.20 | CAF | CAF25800 | CR |  | 1.1 |  | −0.4 |  | 1.1 |  | −0.4 |  | 1.1 |  | −0.4 |  | 1.1 |  | −0.4 |  | 1.1 |  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −13.20 | COG | COG23500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 9.8 |  | 11.7 |
| −13.20 | GAB | GAB26000 | CL | 4.9 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −13.20 | PSE | YYY00001 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.6 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  | 10.1 |  |
| −12.80 | CZE | CZE14401 | CR | 2.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  |
| −12.80 | CZE | CZE14402 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7 |  |  |  |  |  |  |  | −0.7 |  |  |  |  |
| −12.80 | CZE | CZE14403 | CL |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7* |  | −0.7* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −12.80 | HNG | HNG10601 | CR |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |
| −12.80 | HNG | HNG10602 | CL |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7 |  |  |  |  |  |  |  | −0.7 |  |  |
| −12.80 | HNG | HNG10603 | CL |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7* |  | −0.7* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −12.80 | HRV | HRV14801 | CR |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  | 0.0 |  |  |  |
| * This assignment shall only be used by the administrations of Croatia, Hungary, Slovakia and the Czech Rep. on the basis of equal access subject to mutual agreement between them. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −12.80 | HRV | HRV14802 | CL |  |  |  |  |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7 |  |  |  |  |  |  |  | 2.1 |
| −12.80 | HRV | HRV14803 | CL |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7* |  | −0.7* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −12.80 | SVK | SVK14401 | CR |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  | 0.0 |  |  |  |  |  |  |  | 0.0 |  |
| −12.80 | SVK | SVK14402 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  |  |  |  |  |  |  | −0.7 |  |  |  |  |  |  |  | −0.7 |  |  |  |  |  |  |
| −12.80 | SVK | SVK14403 | CL |  | 0.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7* |  | −0.7* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −7.00 | EGY | EGY02600 | CR |  | 27.4 |  | 28.1 |  | 27.4 |  | 28.1 |  | 27.4 |  | 28.1 |  | 27.4 |  | 28.7 |  | 27.8 |  | 9.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −7.00 | F | F 09300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |  | −0.4 |
| −7.00 | F | F____100 | CL | 17.3 |  | 16.4 |  | 16.4 |  | 16.4 |  | 16.4 |  | 16.4 |  | 16.4 |  | 17.1 |  | 17.9 |  | 17.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −7.00 | SRB | SRB14800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 2.0 |
| −7.00 | STP | STP24100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.0 |  | 14.1 |
| −4.00 | ISR | ISR11000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 18.2 |  | 20.9 |
| −1.20 | BUL | BUL02000 | CL | 3.5 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 3.2 |  | 5.6 |  | 5.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −1.20 | CVA | CVA08300 | CR |  | 1.7 |  | 2.3 |  | 1.7 |  | 2.3 |  | 1.7 |  | 2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −1.20 | CVA | CVA08500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −1.20 | CYP | CYP08600 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.7 |  | 4.6 |  | 4.7 |  | 4.6 |  | 4.7 |  | 4.6 |  | 4.7 |  | 4.6 |  | 4.7 |  | 4.6 |  |
| −1.20 | GRC | GRC10500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  | −0.6 |  | 0.2 |  | −0.6 |  | 0.2 |  | −0.6 |  | 0.2 |  | −0.6 |  | 0.2 |  | 0.6 |
| −0.80 | BOT | BOT29700 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.2 |  | −0.4 |  | 0.8 |  | −0.4 |  | 0.8 |  | −0.4 |  | 0.8 |  | −0.4 |  | 0.8 |  | −0.4 |  |
| −0.80 | KEN | KEN24900 | CR |  | 1.4 |  | 2.4 |  | 1.4 |  | 2.5 |  | 1.4 |  | 2.5 |  | 1.4 |  | 3.3 |  | 1.9 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −0.80 | NOR | NOR12000 | CR | 1.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.9 |  | 0.9 |  | 4.2 |  | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| −0.80 | NOR | NOR12100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.4 |  |  |  | 4.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| −0.80 | ZMB | ZMB31400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  | −1.0 |  | 0.2 |  | −1.0 |  | 0.2 |  | −1.0 |  | 0.2 |  | −1.0 |  | 0.2 |  | 0.7 |
| −0.80 | ZWE | ZWE13500 | CL | 8.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.6 |  | 7.8 |  | 7.9 |  | 7.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.80 | LSO | LSO30500 | CL |  | 6.3 |  | 6.2 |  | 6.3 |  | 6.2 |  | 6.3 |  | 6.2 |  | 6.1 |  | 6.1 |  | 5.8 |  | 8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.80 | MWI | MWI30800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.6 |  | 11.1 |  | 11.1 |  | 11.1 |  | 6.9 |  | 6.9 |  | 6.9 |  | 6.9 |  | 6.9 |  | 5.4 |  |
| 4.80 | SWZ | SWZ31300 | CR | 6.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.9 |  | 3.8 |  | 3.8 |  | 3.8 |  | 3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.00 | S | S 13800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.5 |  | 7.2 |  | 8.2 |  | 7.2 |  | 8.2 |  | 7.2 |  | 8.2 |  | 7.2 |  | 8.2 |  | 7.2 |  |
| 5.00 | S | S 13900 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.8 |  |  |  |  |  |  |  |  |  |  |
| * This assignment shall only be used by the administrations of Croatia, Hungary, Slovakia and the Czech Rep. on the basis of equal access subject to mutual agreement between them. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.00 | I | I 08200 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.2 |  | 12.0 |  | 12.2 |  | 11.6 |  | 11.1 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.2 |  | 11.4 |
| 11.00 | BDI | BDI27000 | CL | 3.2 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | JOR | JOR22400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 5.8 |  | 7.6 |
| 11.00 | KWT | KWT11300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.0 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  | 7.1 |  |
| 11.00 | LBN | LBN27900 | CR | 2.0 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  | −0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | RRW | RRW31000 | CR |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | SYR | SYR22900 | CL |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | SYR | SYR33900 | CL |  | −0.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.00 | TZA | TZA22500 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 0.1 |  | 2.0 |
| 16.80 | DJI | DJI09900 | CL | 8.7 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.00 | ARS | ARS00375 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 4.3 |  | 6.8 |
| 17.00 | ARS | ARS34000 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.8 |
| 17.00 | TCD | TCD14300 | CR | 4.3 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.00 | UGA | UGA05100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.5 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  | 10.7 |  |
| 17.20 | OMA | OMA12300 | CL |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 4.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20.00 | QAT | QAT24700 | CL |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 13.7 |  | 15.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22.80 | ARM | ARM06400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 2.4 |  | 5.1 |
| 22.80 | ERI | ERI09200 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.7 |
| 22.80 | FIN | FIN10300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 1.7 |
| 22.80 | FIN | FIN10400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  |  |  | 1.7 |
| 22.80 | MKD | MKD14800 | CL |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 8.7 |  | 10.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22.80 | MLT | MLT14700 | CR | 9.1 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.7 |  | 7.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23.20 | AZE | AZE06400 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.7 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  | 2.5 |  |
| 23.20 | GEO | GEO06400 | CL | 8.1 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23.20 | LTU | LTU06100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.6 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  |
| 23.20 | LVA | LVA06100 | CR | 6.4 |  | 6.3 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.2 |  | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28.20 | LUX | LUX11400 | CL | 18.5 |  | 18.2 |  | 17.7 |  | 18.2 |  | 17.7 |  | 18.2 |  | 17.7 |  | 18.2 |  | 18.2 |  | 18.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29.00 | COM | COM20700 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  | 9.9 |  |
| 29.00 | MAU | MAU__100 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 2.6 |  | 5.6 |
| 29.00 | MDG | MDG23600 | CL | 28.0 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.5 |  | 27.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33.80 | SVN | SVN14800 | CR |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 5.9 |  | 6.0 |  | 6.0 |  | 6.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33.80 | UZB | UZB07100 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.3 |  | 2.5 |  | 5.8 |  | 4.7 |  | 5.8 |  | 4.7 |  | 5.8 |  | 4.7 |  | 5.8 |  | 5.0 |
| 34.00 | BHR | BHR25500 | CR |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.6 |  | 6.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34.00 | IRN | IRN10900 | CL | 2.4 |  | −0.2 |  | −0.2 |  | −0.2 |  | −0.3 |  | −0.2 |  | −0.3 |  | −0.2 |  | −0.2 |  | −0.2 |  | 0.6 |  | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34.20 | MCO | MCO11600 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.3 |  | 8.7 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.7 |
| 36.00 | RUS | RSTREA11 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.8 |  |  |  | −0.8 |  |  |  | −0.8 |  |  |  | −0.8 |  |
| 36.00 | RUS | RSTREA12 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −1.2 |  |  |  | −1.2 |  |  |  | −1.2 |  |  |  | −0.4 |
| 36.00 | RUS | RSTRED11 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.8 |  |  |  | −0.8 |  |  |  | −0.8 |  |  |  | −0.8 |  |
| 36.00 | RUS | RSTRED12 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −1.2 |  |  |  | −1.2 |  |  |  | −1.2 |  |  |  | −0.4 |
| 36.00 | RUS | RSTRSD11 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.5 |  | −0.5 |  | −0.5 |  | −0.5 |  | −0.5 |  | −0.5 |  | −0.5 |  | −0.5 |  |
| 36.00 | RUS | RSTRSD12 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | −0.7 |  | 0.2 |
| 37.80 | BLR | BLR06200 | CL | 2.1 |  | 0.4 |  | 0.2 |  | 0.4 |  | 0.2 |  | 0.4 |  | 0.2 |  | 0.4 |  | 0.4 |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38.00 | TJK | TJK06900 | CL |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 1.9 |  | 4.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38.20 | BEL | BEL01800 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −1.2 |  | 0.6 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 4.2 |
| 38.20 | HOL | HOL21300 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | −0.1 |  | −0.8 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  |
| 38.20 | UKR | UKR06300 | CR |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.6 |  | 0.6 |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42.00 | TUR | TUR14500 | CL |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  | 7.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42.50 | SEY | SEY00000 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19.9 |  | 20.1 |  | 20.2 |  | 20.2 |  | 20.2 |  | 20.2 |  | 20.2 |  | 20.2 |  | 20.2 |  | 21.0 |
| 44.50 | EST | EST06100 | CR | 14.4 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | AFG | AFG24501 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  |
| 50.00 | AFG | AFG24502 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 1.5 |  | 4.0 |
| 50.00 | CLN | CLN21900 | CL | 1.0 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.9 |  | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  | 4.9 |  | 4.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | KGZ | KGZ07000 | CR |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | MDA | MDA06300 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.2 |  | 0.8 |  | 1.0 |  | 1.0 |  | 0.7 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  |
| 50.00 | MLD | MLD30600 | CR | 5.5 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.6 |  | 5.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | POL | POL13200 | CR | 5.9 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  | 4.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50.00 | ROU | ROU13600 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 |  | 0.5 |  | 0.7 |  | −0.1 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | −0.9 |  | 2.0 |
| 50.00 | TKM | TKM06800 | CL | −0.2 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  | −1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52.50 | UAE | UAE27400 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.1 |  | 28.6 |  | 34.6 |  | 35.0 |  | 26.8 |  | 26.2 |  | 26.2 |  | 26.2 |  | 26.2 |  | 26.2 |  |
| 55.80 | IND | INDA_101 | CR |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  | 4.7 |  | 4.7 |  | 4.7 |  | 15.0 |  | 15.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55.80 | IND | INDA_102 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.3 |  | 9.3 |  | 9.3 |  | 11.7 |  | 17.4 |  | 14.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55.80 | IND | INDB_101 | CR | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55.80 | IND | INDB_102 | CL |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | 7.3 |  | −0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56.00 | BIH | BIH14800 | CR | 13.0 |  | 12.6 |  | 12.6 |  | 12.6 |  | 12.6 |  | 12.6 |  | 12.3 |  | 12.3 |  | 12.3 |  | 12.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56.00 | RUS | RSTRSD21 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.0 |  | 17.6 |  | 17.2 |  | 17.1 |  | 17.1 |  | 17.1 |  | 17.1 |  | 17.1 |  |
| 56.00 | RUS | RSTRSD22 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17.7 |  | 17.6 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 17.2 |  | 18.2 |
| 56.40 | KAZ | KAZ06600 | CL | 3.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 3.1 |  | 6.0 |  | 6.0 |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62.00 | ALB | ALB29600 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.8 |  | 14.1 |  | 46.6 |  | 46.5 |  | 46.3 |  | 46.3 |  | 46.3 |  | 46.3 |  | 46.3 |  | 47.4 |
| 62.00 | CHN | CHN15400 | CR |  | 13.5 |  | 13.5 |  | 13.5 |  | 13.5 |  | 13.5 |  | 13.7 |  | 15.1 |  | 15.1 |  | 15.1 |  | 15.5 |  | 2.6 |  | 2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62.00 | CHN | CHN15500 | CL | 14.5 |  | 13.6 |  | 13.6 |  | 13.6 |  | 13.6 |  | 13.6 |  | 15.0 |  | 15.4 |  | 15.4 |  | 15.4 |  | 2.5 |  | −0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68.00 | IND | IND03700 | CL |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 5.2 |  | 8.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68.00 | IND | IND04701 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40.8 |  | 40.8 |  | 40.8 |  | 40.8 |  | 40.8 |  | 40.8 |  |
| 68.00 | IND | IND04702 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 44.2 |  | 44.2 |  | 44.2 |  | 44.2 |  | 44.2 |  | 47.1 |
| 68.00 | IND | INDD_100 | CR | 6.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  | 3.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | BGD | BGD22000 | CR | 4.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 1.7 |  | 0.9 |  | 0.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | BRU | BRU3300A | CR |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 12.1 |  | 11.7 |  | 11.3 |  | 13.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74.00 | MNG | MNG24800 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.1 |  | 8.1 |  | 15.8 |  | 999.9 |  | 48.6 |  | 48.1 |  | 48.1 |  | 48.1 |  | 48.1 |  | 48.1 |  |
| 80.20 | INS | INS02800 | CR | 16.0 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.6 |  | 14.5 |  | 14.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | BTN | BTN03100 | CR | 8.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  | 5.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | CBG | CBG29900 | CR |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 2.1 |  | 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86.00 | RUS | RSTRSD31 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |
| 86.00 | RUS | RSTRSD32 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |
| 88.00 | SNG | SNG15100 | CL | 15.4 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  | 13.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 91.50 | MLA | MLA__100 | CR |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 11.4 |  | 13.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 92.20 | CHN | CHN16000 | CR | 7.7 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  | 4.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 92.20 | CHN | CHN16100 | CL |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.1 |  | 11.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98.00 | PHL | PHL28500 | CL |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 5.1 |  | 7.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98.00 | THA | THA14200 | CR | 8.1 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104.00 | BRM | BRM29800 | CR | 15.4 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104.00 | INS | INS03501 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 36.4 |  | 37.0 |  | 37.0 |  | 37.0 |  | 37.0 |  | 37.0 |  |
| 104.00 | INS | INS03502 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42.1 |  | 42.1 |  | 42.1 |  | 42.1 |  | 42.1 |  | 45.1 |
| 107.00 | VTN | VTN32500 | CR |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.2 |  | 14.3 |  | 14.4 |  | 17.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 109.85 | J | 000BS−3N | CR | 21.6 |  | 19.4 |  | 19.2 |  | 19.2 |  | 19.2 |  | 19.2 |  | 19.9 |  | 21.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 109.85 | J | J 10985 | CR | 23.9 |  | 21.6 |  | 21.4 |  | 21.4 |  | 21.4 |  | 21.4 |  | 22.1 |  | 23.1 |  | 21.7 |  | 21.4 |  | 21.4 |  | 21.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | J | J 11100 | CR | 23.7 |  | 21.4 |  | 21.2 |  | 21.2 |  | 21.2 |  | 21.2 |  | 22.0 |  | 23.0 |  | 21.6 |  | 21.2 |  | 21.3 |  | 21.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | J | J 1110E | CR | 21.4 |  | 19.2 |  | 19.0 |  | 19.0 |  | 19.0 |  | 19.0 |  | 19.8 |  | 20.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110.00 | RUS | RUS00401 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29.2 |  | 999.9 |  | 14.1 |  | 13.2 |  | 13.2 |  | 13.2 |  | 13.2 |  | 13.2 |  |
| 110.00 | RUS | RUS00402 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 20.8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 116.00 | KOR | KOR11201 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.2 |  | 29.2 |  | 29.2 |  | 29.2 |  | 29.2 |  | 29.2 |  |
| 116.00 | KOR | KOR11202 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 31.8 |  | 31.8 |  | 31.8 |  | 31.8 |  | 31.8 |  | 34.7 |
| 121.80 | USA | MRA33200 | CR | 3.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.00 | CHN | CHN20000 | CL |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.8 |  | 13.9 |  | 13.9 |  | 13.8 |  | 13.8 |  | 13.8 |  | 15.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122.00 | USA | GUM33101 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29.3 |  | 28.6 |  | 28.6 |  | 28.6 |  | 28.6 |  | 28.6 |  |
| 122.00 | USA | GUM33102 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.6 |  | 28.6 |  | 28.6 |  | 28.6 |  | 28.6 |  | 29.3 |
| 122.20 | LAO | LAO28400 | CR |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.4 |  | 0.4 |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128.00 | SLM | SLM00000 | CL | 16.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  | 13.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128.00 | TLS | TLS00000 | CR |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 19.6 |  | 22.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | CHN | CHN15800 | CL |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 0.6 |  | 3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | CHN | CHN15900 | CR | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  | 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134.00 | NRU | NRU30900 | CL | 18.8 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  | 17.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | F | F____200 | CL |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 11.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | F | F____300 | CR | 8.5 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  | 6.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | KRE | KRE28600 | CL | 13.6 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  | 12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | PLW | PLW00000 | CR |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 9.7 |  | 7.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140.00 | RUS | RSTRSD51 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  | 999.9 |  | 3.0 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  |
| 140.00 | RUS | RSTRSD52 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 7.3 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 1.6 |  | 3.0 |
| 140.00 | VUT | VUT12801 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  |
| 140.00 | VUT | VUT12802 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 8.9 |  | 10.4 |
| 146.00 | MHL | MHL00000 | CR |  | 41.3 |  | 40.8 |  | 41.3 |  | 40.8 |  | 41.3 |  | 41.1 |  | 41.6 |  | 41.1 |  | 41.6 |  | 41.1 |  | 41.6 |  | 41.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00400 | CL |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00401 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00402 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00403 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00404 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00405 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00406 | CL |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS0040A | CL |  |  | −0.4 |  |  |  | −0.4 |  |  |  | −0.4 |  |  |  | −0.4 |  |  |  | −0.4 |  |  |  | −0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00500 | CR |  |  |  | −3.3 |  |  |  | −3.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  | −0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00501 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00502 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00503 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00504 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00505 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00506 | CR |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUS00600 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00601 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00602 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00603 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00604 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00605 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUS00606 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.9 |  | 999.9 |  | 999.9 |  | 999.9 |  |  |  | 999.9 |  |  |  | 999.9 |
| 152.00 | AUS | AUSA0000 | CL | 39.7 |  |  |  | −0.4 |  |  |  | −0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0001 | CL | 61.2 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0002 | CL | 60.5 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0003 | CL | 61.5 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0004 | CL | 53.4 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0005 | CL | 55.9 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152.00 | AUS | AUSA0006 | CL | 48.3 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 158.00 | FSM | FSM00000 | CR | 8.8 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 158.00 | NZL | NZL__100 | CL |  | 8.5 |  | 8.4 |  | 8.5 |  | 8.4 |  | 8.5 |  | 8.4 |  | 8.6 |  | 8.6 |  | 8.6 |  | 8.6 |  | 8.6 |  | 11.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00700 | CR |  |  | −2.9 |  |  |  | −2.9 |  |  |  | −2.9 |  |  |  | 0.1 |  |  |  | 0.1 |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00701 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00702 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00703 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00704 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00705 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00706 | CR |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 3.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS0070A | CR |  |  | −3.1 |  |  |  | −3.1 |  |  |  | −3.1 |  |  |  | −0.1 |  |  |  | −0.1 |  |  |  | −0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00800 | CL |  | 0.2 |  |  |  | 0.2 |  |  |  | 0.2 |  |  |  | 0.2 |  |  |  | 0.2 |  |  |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00801 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00802 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00803 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00804 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00805 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00806 | CL |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUS00900 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 31.9 |  | 999.9 |  | 38.1 |  | 36.8 |  |  |  | 36.8 |  |  |  | 36.8 |  |
| 164.00 | AUS | AUS00901 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65.2 |  | 999.9 |  | 63.1 |  | 61.7 |  |  |  | 61.7 |  |  |  | 61.7 |  |
| 164.00 | AUS | AUS00902 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65.4 |  | 999.9 |  | 63.3 |  | 61.9 |  |  |  | 61.9 |  |  |  | 61.9 |  |
| 164.00 | AUS | AUS00903 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60.5 |  | 999.9 |  | 62.1 |  | 60.8 |  |  |  | 60.8 |  |  |  | 60.8 |  |
| 164.00 | AUS | AUS00904 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50.2 |  | 999.9 |  | 60.8 |  | 59.5 |  |  |  | 59.5 |  |  |  | 59.5 |  |
| 164.00 | AUS | AUS00905 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 56.4 |  | 999.9 |  | 59.4 |  | 58.1 |  |  |  | 58.1 |  |  |  | 58.1 |  |
| 164.00 | AUS | AUS00906 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48.1 |  | 999.9 |  | 57.0 |  | 55.8 |  |  |  | 55.8 |  |  |  | 55.8 |  |
| 164.00 | AUS | AUS0090A | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42.0 |  | 999.9 |  | 40.3 |  | 38.9 |  |  |  | 38.9 |  |  |  | 38.9 |  |
| 164.00 | AUS | AUSB0000 | CL |  |  |  | −0.1 |  |  |  | −0.1 |  |  |  | −0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0001 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0002 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0003 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0004 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0005 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164.00 | AUS | AUSB0006 | CL |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170.00 | USA | PLM33200 | CL |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  | 10.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170.00 | USA | USAA_101 | CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45.5 |  | 45.5 |  | 999.9 |  | 45.5 |  | 999.9 |  | 45.5 |  |
| 170.00 | USA | USAA_102 | CL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45.5 |  | 48.5 |  | 48.5 |  | 48.5 |  | 48.5 |  | 48.5 |
| 170.75 | TON | TON21500 | CR |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  | 11.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 176.00 | KIR | KIR__100 | CL | 4.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 176.00 | TUV | TUV00000 | CR |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 4.7 |  | 7.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

ARTICLE 10

Interference

10.1	The Member States shall endeavour to agree on the action required to reduce harmful interference which might be caused by the application of these provisions and the associated Plans.

ARTICLE 11

Period of validity of the provisions and associated Plans

11.1	The provisions and associated Plans have been prepared in order to meet the requirements for feeder-links for the broadcasting-satellite service in the bands concerned for a period extending until at least 1 January 1994.

11.2	In any event, the provisions and associated Plans shall remain in force until their revision by a competent administrative radio conference convened in accordance with the relevant provisions of the Convention in force.

ANNEX 1

Limits for determining whether a service of an administration is considered
to be affected by a proposed modification to the Region 2 feeder-link Plan
or by a proposed new or modified assignment in the Regions 1 and 3
feeder-link List or when it is necessary under this Appendix to seek
the agreement of any other administration     (Rev.WRC-03)

# 1	(SUP - WRC-2000)

# 2	(SUP - WRC-2000)

# 3	Limits to the change in the overall equivalent protection margin with respect to frequency assignments in conformity with the Region 2 feeder-link Plan33     (WRC-2000)

With respect to the modification to the Region 2 feeder-link Plan and when it is necessary under this Appendix to seek the agreement of any other administration of Region 2, except in cases covered by Resolution 42 (Rev.WRC-03)*, an administration is considered as being affected if the overall equivalent protection margin34 corresponding to a test point of its entry in that Plan, including the cumulative effect of any previous modification to that Plan or any previous agreement, falls more than 0.25 dB below 0 dB, or, if already negative, more than 0.25 dB below the value resulting from:

–	the feeder-link Plan as established by the 1983 Conference; or

–	a modification of the assignment in accordance with this Appendix; or

–	a new entry in the feeder-link Plan under Article 4; or

–	any agreement reached in accordance with this Appendix except for Resolution 42 (Rev.WRC-03)*.     (WRC-03)

# 4	Limits to the interference into frequency assignments in conformity with the Regions 1 and 3 feeder-link Plan or with the Regions 1 and 3 feeder-link List or proposed new or modified assignments in the Regions 1 and 3 feeder-link List     (WRC-03)

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the feeder-link List shall not exceed the value of −76 dB(W/(m2 · 27 MHz)) at any point in the geostationary-satellite orbit, and the relative off-axis e.i.r.p. of the associated feeder-link antenna shall be in compliance with Fig. A (WRC-97 curves) of Annex 3.     (WRC-03)

With respect to § 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 is considered by the Bureau as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.     (WRC-03)

However, an administration is not considered as being affected if, under assumed free-space propagation conditions, the effect of the proposed new or modified assignments in the feeder-link List is that the feeder-link equivalent protection margin35 corresponding to a test point of its assignment in the feeder-link Plan or the feeder-link List or for which the procedure of Article 4 has been initiated, including the cumulative effect of any previous modification to the feeder-link List or any previous agreement, does not fall more than 0.45 dB below 0 dB, or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 feeder-link Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the feeder-link List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 feeder-link List as a result of the successful application of Article 4 procedures.     (WRC-03)

For a proposed new or modified assignment to the feeder-link List, in the interference analysis, for each test point, the antenna characteristics described in § 3.5 of Annex 3 shall apply.     (WRC-03)

# 5	Limits applicable to protect a frequency assignment in the bands 
17.3-18.1 GHz (Regions 1 and 3) and 17.3-17.8 GHz (Region 2) to a receiving space station in the fixed-satellite service (Earth-to-space)

An administration in Region 1 or 3 is considered as being affected by a proposed modification in Region 2, with respect to § 4.2.2 a) or 4.2.2 b) of Article 4, or an administration in Region 2 is considered as being affected by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List, with respect to § 4.1.1 c) of Article 4, when the power flux-density arriving at the receiving space station of a broadcasting-satellite feeder-link would cause an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of Δ T / T corresponding to 6%, where Δ T / T is calculated in accordance with the method given in Appendix 8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the necessary bandwidth of the feeder-link carriers.     (WRC-03)

Interim systems of Region 2 in accordance with Resolution 42 (Rev.WRC-03)* shall not be taken into consideration when applying the above paragraph to proposed new or modified assignments in the Regions 1 and 3 feeder-link List. However, the above paragraph shall be applied to Region 2 interim systems with respect to Regions 1 and 3 administrations, referred to in § 5.2 b) of Resolution 42 (Rev.WRC-03)*.     (WRC-03)

# 6	Limits applicable to protect a frequency assignment in the band 
17.8-18.1 GHz (Region 2) to a receiving feeder-link space station 
in the fixed-satellite service (Earth-to-space) or a frequency assignment in the frequency bands 14.5-14.75 GHz (in countries listed in Resolution 163 (WRC-15)) and 14.5-14.8 GHz (in countries listed in Resolution 164 (WRC-15)) to a receiving space station in the fixed-satellite service (Earth-to-space) not subject to a Plan     (WRC-15)

With respect to § 4.1.1 d) of Article 4, an administration is considered affected by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List when the power flux-density arriving at the receiving space station of a broadcasting-satellite feeder link in Region 2 or the receiving space station of the fixed-satellite service uplinks not subject to a Plan in all Regions of that administration would cause an increase in the noise temperature of the receiving uplink space station which exceeds the threshold value of ΔT/T corresponding to 6%, where ΔT/T is calculated in accordance with the method given in Appendix 8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the necessary bandwidth of the feeder-link carriers.     (WRC-15)

ANNEX 2     (REV.WRC-03)

Basic characteristics to be furnished in notices relating to feeder-link
stations in the fixed-satellite service operating in the frequency
bands 14.5-14.8 GHz and 17.3-18.1 GHz

These data are listed in Appendix 4.

ANNEX 3

Technical data used in establishing the provisions and associated
Plans and Regions 1 and 3 feeder-link List, which should
be used for their application36     (Rev.WRC-03)

# 1	Definitions

## 1.1	Feeder link

The term feeder link, as defined in No. 1.115, is further qualified to indicate a fixed-satellite service link in the frequency band 17.3-17.8 GHz in the Region 2 broadcasting-satellite service Plan and in the frequency bands 14.5-14.8 GHz for countries outside Europe, and 17.3-18.1 GHz in the Regions 1 and 3 Plan, from any earth station within the feeder-link service area to the associated space station in the broadcasting-satellite service.

## 1.2	Feeder-link beam area

The area delineated by the intersection of the half-power beam of the satellite receiving antenna with the surface of the Earth.

## 1.3	Feeder-link service area

The area on the surface of the Earth within the feeder-link beam area within which the administration responsible for the service has the right to locate transmitting earth stations for the purpose of providing feeder-links to broadcasting-satellite space stations.

## 1.4	Nominal orbital position

The longitude of a position in the geostationary-satellite orbit associated with a frequency assignment to a space station in a space radiocommunication service. The position is given in degrees from the Greenwich meridian.

## 1.5	Adjacent channel

The RF channel in the broadcasting-satellite service frequency Plan, or in the associated feeder-link frequency Plan, which is situated immediately higher or lower in frequency with respect to the reference channel.

## 1.6	Second adjacent channel

The RF channel in the broadcasting-satellite service frequency Plan, or in the associated feeder-link frequency Plan, which is situated immediately beyond either of the adjacent channels, with respect to the reference channel.

## 1.7	Feeder-link equivalent protection margin for Regions 1 and 337     (WRC-2000)

The feeder-link equivalent protection margin (Mu) is given by the formula:

dB

where:

M1:	is the value in dB of the protection margin for the same channel, i.e.:

M2 and M3:	are the values in dB of the protection margin for the upper and lower adjacent channels, respectively, i.e.:

All powers are evaluated at the receiver input. All protection ratios are given in § 3.3.

## 1.8	Overall carrier-to-interference (C/I) ratio

The overall C/I ratio is the ratio of the wanted carrier power to the sum of all interfering RF powers in a given channel including both feeder-links and downlinks. The overall C/I ratio due to interference from the given channel is calculated as the reciprocal of the sum of the reciprocals of the feeder-link C/I ratio and the downlink C/I ratio referred to the satellite receiver input and earth station receiver input, respectively38.

## 1.9	Overall co-channel protection margin

The overall co-channel protection margin in a given channel is the difference (dB) between the overall co-channel C/I ratio and the co-channel protection ratio.

## 1.10	Overall adjacent channel protection margin

The overall adjacent channel protection margin is the difference (dB) between the overall adjacent channel C/I ratio and the adjacent channel protection ratio.

## 1.11	Overall second adjacent channel protection margin

The overall second adjacent channel protection margin is the difference (dB) between the overall second adjacent channel C/I ratio and the second adjacent channel protection ratio.

## 1.12	Overall equivalent protection margin

The overall equivalent protection margin M is given in dB by the expression39:

where:

n:	is generally equal to 3 for Regions 1 and 3, n is equal to 5 for Region 2

M1:	overall co-channel protection margin (dB) (as defined in § 1.9)

M2, M3:	overall adjacent channel protection margins for the upper and lower adjacent channels, respectively (dB) (as defined in § 1.10)

M4, M5:	overall second adjacent channel protection margins for the upper and lower second adjacent channels, respectively (dB) as defined in § 1.11).40

The adjective “equivalent” indicates that the protection margins for all interference sources from the adjacent and second adjacent as well as co-channel interference sources have been included.

The following alternative formula for overall equivalent protection margin was used at the 1988 Conference (WARC Orb-88) in developing the original feeder-link Plan for Regions 1 and 3. It may be used as a tool to assess the relative contributions of the feeder link and downlink to the overall equivalent protection margin defined above.

where:

Mu:	equivalent protection margin for the feeder link (as defined in § 1.7)

Md:	equivalent protection margin for the downlink (as defined in § 3.4, Annex 5 to Appendix 30

Rcu:	co-channel feeder-link protection ratio

Rcd :	co-channel downlink protection ratio

Rco:	co-channel overall protection ratio.

The values of the protection ratios used for the 1988 feeder-link Plan were as follows:

Rcu  =  40 dB

Rcd  =  31 dB

Rco  =  30 dB

The adjective “equivalent” indicates that the protection margins for all interference sources from the adjacent channels as well as co-channel interference sources have been included.

The corresponding values for analysing the 1997 feeder-link Plan are:

Rcu  =  30 dB

Rcd  =  24 dB

Rco  =  23 dB

However, the latter values are restricted to the case of channels having the standard channel spacing and necessary bandwidth given in § 3.5 and 3.8, respectively, of Annex 5 to Appendix 30.

WRC-2000 generally applied the following protection ratio values for development of the WRC-2000 Regions 1 and 3 feeder-link Plan:

Rcu  =  27 dB

Rcd  =  21 dB										(WRC-2000)

These values were used for all assignments in WRC-2000 planning except those for which WRC-2000 adopted different values (see § 3.3). The planning at WRC-2000 was based on use of the equivalent protection margin criterion.     (WRC-2000)

# 2	Radio propagation factors

The propagation loss on an Earth-to-space path is equal to the free-space path loss plus the atmospheric absorption loss plus the rain attenuation exceeded for 1% of the worst month in Region 2. In Regions 1 and 3, the atmospheric absorption loss is not included.

## 2.1	Atmospheric absorption

For Region 2 (see Fig. 2)

The loss due to atmospheric absorption (i.e. clear-sky attenuation) is given by:

dB	for θ <  5°

where:

and:

dB	for  θ  ≥  5°

where:

θ:	elevation angle (degrees)

ρ:	surface water vapour concentration, g/m3, with

ρ = 10 g/m3 for rain climatic zones A to K and

ρ = 20 g/m3 for rain climatic zones M to P.

For Regions 1 and 3 (see Figs. 1 and 3 taken from Recommendation ITU-R P.837-1)

In the Regions 1 and 3 feeder-link Plan, the atmospheric absorption loss is not included for the calculation of margins.

## 2.2	Rain attenuation

The propagation model for feeder links using circularly polarized signals is based on the value of rain attenuation for 1% of the worst month.

Figures 1, 2 and 3 give the rain climatic zones for Regions 1, 2 and 3.

Figure 4 presents a plot of rain attenuation of circularly polarized signals exceeded for 1% of the worst month at 17.5 GHz as a function of earth station latitude and elevation angle for each of the rain climatic zones in Region 2.

For calculation, the following data are needed:

R0.01:	point rainfall rate for the location exceeded for 0.01% of an average year (mm/h)

h0:	height above mean sea level of the earth station (km)

θ:	elevation angle (degrees)

f:	frequency (GHz)

ζ:	latitude of earth station (degrees).

Mean frequencies will be used for calculations for the frequency bands, i.e. 17.7 GHz and 14.65 GHz for Regions 1 and 3, 17.5 GHz for Region 2.

The calculation procedure used for the Region 2 feeder-link Plan and for the original 1988 Regions 1 and 3 feeder-link Plan consists of the following seven steps:

Step 1:	the mean zero-degree isotherm height hF is:

km

Step 2:	the rain height hR is:

km

where:

C  =  0.6	for	 0°	≤	| ζ |   <   20°

C  =  0.6  +  0.02 ( | ζ |  –  20)	for	20°	≤	| ζ |   <   40°

C  =  1	for			| ζ |    ≥   40°

Step 3:	the slant-path length, Ls, below the rain height is:

km

where Re is the effective radius of the Earth (8 500 km).

Step 4:	the horizontal projection, LG, of the slant-path is:

km

Step 5:	the rain path reduction factor r0.01, for 0.01% of the time is:

Step 6:	the specific attenuation R is determined from:

dB/km

where R0.01 is given in Table 1 for each rain climatic zone. The frequency dependent coefficients k and  are given in Table 2 and the rain climatic zones are given in Figs. 1, 2 and 3 for Regions 1, 2 and 3.

TABLE 1

Rainfall intensity (R) for the rain climatic zones (exceeded for 0.01% of an average year)

| Rain climatic zone | A | B | C | D | E | F | G | H | J | K | L | M | N | P | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rainfall intensity (mm/h) | 8 | 12 | 15 | 19 | 22 | 28 | 30 | 32 | 35 | 42 | 60 | 63 | 95 | 145 | 115 |

TABLE 2

Frequency dependent coefficients

| Frequency (GHz) | k |  |  |
| --- | --- | --- | --- |
| 14.65 | 0.0327 | 1.149 | For Regions 1 and 3 |
| 17.5 | 0.0521 | 1.114 | For Region 2 |
| 17.7 | 0.0531 | 1.110 | For Regions 1 and 3 |

Step 7:	the attenuation exceeded for 1% of the worst month is:

A1%	=	0.223 γR Ls r0.01      dB	for Regions 1 and 3

A1%	=	0.21 γR Ls r0.01        dB	for Region 2.

For calculation of the permissible increase in e.i.r.p. to overcome rain fading (power control, see § 3.11.1) in the Regions 1 and 3 Plan revised by WRC-97, the same calculation procedure is used with the following changes to conform to Recommendation ITU-R P.618-5.

To calculate the rain height hR, Steps 1 and 2 are replaced by:

Steps 3 and 4 remain the same. However, to calculate the rain path reduction factor r0.01, for 0.01% of the time, the equation of Step 5 is replaced by:

where:

L0  =	35 exp (−0.015 R0.01)

and R0.01 is given in Table 1 for each rain climatic zone.

Step 6 remains the same except the frequency dependent coefficients k and shall be obtained from Recommendation ITU-R P.838-3.     (WRC-07)

Step 7 should be replaced as follows:

where:

p (%)  =	0.30 pw (%)1.15        (Recommendation ITU-R P.841)

p is the average annual time percentage of excess corresponding to desired worst-month time percentage of excess pw.

## 2.3	Rain attenuation limit

In the analysis of the Plan for Region 2, a maximum rain attenuation on the feeder link of 13 dB was considered assuming that other means would be used at the implementation stage to compensate for larger rain attenuation on the feeder link.

In the analysis of the Regions 1 and 3 Plan, no rain attenuation is included in the margins.

## 2.4	Depolarization

Rain and ice can cause depolarization of radio frequency signals. The level of the co-polar component relative to the depolarized component is given by the cross-polarization discrimination (XPD) ratio. For the feeder link, the XPD ratio (dB) not exceeded for 1% of the worst month, is given by:

XPD  =  30 log f  –  40 log (cos θ)  –  V log Ap 		for 5°  ≤  θ  ≤  60°

where:

V  =  20							for 14.5-14.8 GHz

and

V  =  23							for 17.3-18.1 GHz

where:

Ap :	co-polar rain attenuation exceeded for 1% of the worst month

f :	frequency (GHz)

θ :	elevation angle (degrees).

To calculate the depolarization value to be used for power control in the Regions 1 and 3 Plan, the following algorithm (Steps 1 to 8), which was obtained from Recommendation ITU-R P.618-5, shall be used.

To calculate long-term statistics of depolarization from rain attenuation statistics the following parameters are needed:

Ap :	rain attenuation (dB) exceeded for the required percentage of time, p, for the path in question, commonly called co-polar attenuation (CPA)

τ :	tilt angle of the linearly-polarized electric field vector with respect to the horizontal (for circular polarization use τ = 45°)

f :	frequency (GHz)

θ :	path elevation angle (degrees).

The method described below to calculate XPD statistics from rain attenuation statistics for the same path is valid for 8 GHz ≤ f ≤ 35 GHz and θ ≤ 60°.

Step 1:	calculate the frequency-dependent term:

Cf  =  30 log f	for 	8 GHz	≤  f  ≤  35 GHz

Step 2:	calculate the rain attenuation dependent term:

CA  =  V ( f ) log Ap

where:

V( f )  =  12.8 f 0.19	for	8 GHz	≤  f  ≤  20 GHz

V( f )  =  22.6	for	20 GHz	<  f  ≤  35 GHz

Step 3:	calculate the polarization improvement factor:

Cτ  =  −10 log [1  −  0.484 (1  +  cos 4τ)]

The improvement factor Cτ = 0 for τ = 45° and reaches a maximum value of 15 dB for τ = 0° or 90°.

Step 4:	calculate the elevation angle dependent term:

Cθ  =  −40 log (cos θ )	for	θ  ≤  60°

Step 5:	calculate the canting angle dependent term:

Cσ  =  0.0052 σ2

σ is the effective standard deviation of the raindrop canting angle distribution, expressed in degrees; σ takes the value 0°, 5°, 10° and 15° for 1%, 0.1%, 0.01% and 0.001% of the time, respectively.

Step 6:	calculate rain XPD not exceeded for p% of the time:

XPDrain  =  Cf  −  CA  +  Cτ  +  Cθ  +  Cσ               dB

Step 7:	calculate the ice crystal dependent term:

Cice  =  XPDrain (0.3  +  0.1 log p)  2               dB

Step 8:	calculate the XPD not exceeded for p% of the time, including the effects of ice:

XPDp  =  XPDrain  −  Cice               dB

For values of θ greater than 60°, use θ = 60° in the above equations.

## 2.5	Procedure for calculating the C/I ratio at a space station receiver input

In Region 2, the calculation of the feeder-link C/I ratio (exceeded for 99% of the worst month) at a space station receiver input used to obtain the overall equivalent protection margin at a test point assumes a rain attenuation value not exceeded for 99% of the worst month on the wanted feeder-link path. For the interfering feeder-link signal path, clear sky propagation (i.e., including atmospheric absorption only) is assumed.

In Regions 1 and 3, the calculation of the feeder-link C/I ratio at a space station receiver input used to obtain the feeder-link equivalent protection margin at a test point assumes free space conditions on the wanted feeder-link path and on the interfering feeder-link path.

# 3	Basic technical characteristics for Regions 1 and 3

## 3.1	Translation frequency and guardbands

a)	17 GHz feeder-links

The feeder-link Plan generally uses a frequency translation of 5.6 GHz between the 17 GHz feeder-link channels and the 12 GHz downlink channels. Other values of the translation frequency may be used, provided that the corresponding channels have been assigned to the space station of the administration concerned.

With the value of frequency translation between the feeder-link frequency band (17.3-18.1 GHz in Regions 1 and 3) and the downlink frequency band (11.7-12.5 GHz in Region 1 and 11.7-12.2 GHz in Region 3), the guardbands specified in § 3.9 of Annex 5 to Appendix 30 for the downlink Plan result in corresponding guardband bandwidths of 11 MHz at the upper and 14 MHz at the lower feeder-link band edges. These feeder-link guardbands may be used to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.     (WRC-03)

b)	14 GHz feeder-links

As the maximum available bandwidth for the feeder-link band 14.5-14.8 GHz is only 300 MHz divided into fourteen 27 MHz channels, against 800 MHz (40 channels) and 500 MHz (24 channels) in the downlink Plan for Regions 1 and 3, respectively, several translation frequencies must be considered to allow any channel in the Plan to be used. Consequently, a particular feeder-link channel has been assigned to several broadcasting-satellite service Plan channels simultaneously.

Generally, the translation frequencies from the feeder-link channels are:

2 797.82 MHz	to downlink broadcasting-satellite service channels 1 to 14;

2 529.30 MHz	to downlink broadcasting-satellite service channels 15 to 28;

2 260.78 MHz	to downlink broadcasting-satellite service channels 29 to 40.

The guardband bandwidths are 11.80 MHz at the lower band edge and 11.86 MHz at the upper band edge.

c)	Frequency translation rules

Specific rules for selecting appropriate frequency translations are given in § 6.2.1.2.2 and 6.2.1.3.3 of the 1985 Conference (WARC Orb-85) Report to the 1988 Conference (WARC Orb-88). These rules permit the derivation of simple-to-use tables that define the channel translations that were avoided in revising the Regions 1 and 3 feeder-link Plan for both the 14 GHz and 17 GHz bands (see Tables 3 and 4).

TABLE 3

14.5-14.8 GHz/11.7-12.5 GHz channel translations that should be avoided (as far as possible)
according to the 1985 Conference frequency translation rules

| 14 GHz feeder-link channel number | Downlink channel numbers to be avoided (as far as possible) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 7 | 8 | 9 | 19 | 20 |
| 2 | 8 | 9 | 10 | 20 | 21 |
| 3 | 9 | 10 | 11 | 21 | 22 |
| 4 | 10 | 11 | 12 | 22 | 23 |
| 5 | 11 | 12 | 13 | 23 | 24 |
| 6 | 12 | 13 | 14 | 24 | 25 |
| 7 | 13 | 14 | 15 | 25 | 26 |
| 8 | 14 | 15 | 16 | 26 | 27 |
| 9 | 15 | 16 | 17 | 27 | 28 |
| 10 | 16 | 17 | 18 | 28 | 29 |
| 11 | 17 | 18 | 19 | 29 | 30 |
| 12 | 18 | 19 | 20 | 30 | 31 |
| 13 | 19 | 20 | 21 | 31 | 32 |
| 14 | 20 | 21 | 22 | 32 | 33 |

TABLE 4

17.3-18.1 GHz/11.7-12.5 GHz channel translations that should be avoided (as far as possible) according to the 1985 Conference frequency translation rules

| 17 GHz feeder-link channel number | Downlink channel numbers to be avoided (as far as possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 1 |  |  | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 1 | 2 |  |  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 1 | 2 | 3 |  |  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | 2 | 3 | 4 |  |  | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | 2 | 3 | 4 | 5 |  |  | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |  |  |  |  |  |  |  |  |  |  |
| 15 | 1 | 2 | 3 | 4 | 5 | 6 |  |  | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 |  |  |  |  |  |  |  |  |  |
| 16 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |  |  |  |  |  |  |  |  |
| 17 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |  |  |  |  |  |  |  |
| 18 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 |  |  |  |  |  |  |
| 19 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 20 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  |  | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 21 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 22 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |  |  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 23 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |  |  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 24 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |  |  | 33 | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 25 |  |  | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |  | 34 | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 26 |  |  |  | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |  |  | 35 | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 27 |  |  |  |  | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |  |  | 36 | 37 | 38 | 394 | 40 |  |  |  |  |  |
| 28 |  |  |  |  |  | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |  |  | 37 | 38 | 39 | 40 |  |  |  |  |  |
| 29 |  |  |  |  |  |  | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |  |  | 38 | 39 | 40 |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |  |  | 39 | 40 |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 |  |  | 40 |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |  |  |  | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |  |  |  |
| 37 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |  |  |
| 38 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |  |
| 39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 31 |

## 3.2	Carrier-to-noise ratio

§ 3.3 of Annex 5 to Appendix 30 provides guidance for planning and the basis for the evaluation of the carrier-to-noise (C/N) ratios of the feeder-link and downlink Plans.

As guidance for planning, the reduction in quality in the downlink due to thermal noise in the feeder-link is taken as equivalent to a degradation in the downlink C/N ratio of approximately 0.5 dB not exceeded for 99% of the worst month.

For downlinks, as indicated in Appendix 30, the 1977 Conference (WARC SAT-77) adopted a C/N value of 14.5 dB for 99% of the worst month at the edge of the service area. The required feeder-link C/N is 24 dB for 99% of the worst month, at the edge of the service area, to produce an overall C/N performance of 14 dB.

## 3.3	Protection ratios

For planning in Regions 1 and 3 at the 1988 Conference (WARC Orb-88), the following protection ratios were applied for the purpose of calculating the feeder-link equivalent protection margins41:

–	co-channel protection ratio = 40 dB;

–	adjacent channel protection ratio = 21 dB.

The method for the calculation of the feeder-link equivalent protection margin is given in § 1.7.

For revising the Regions 1 and 3 feeder-link Plan at WRC-97, the corresponding values of aggregate protection ratio that were used to calculate the feeder-link equivalent protection margins which appear in the alternative formula for overall equivalent protection margin given in § 1.12 are specified in Recommendation ITU-R BO.1297, as follows42, 43:

–	co-channel protection ratio = 30 dB;

–	adjacent channel protection ratio = 22 dB.     (WRC-2000)

However, it should be noted that the revision of the Regions 1 and 3 feeder-link Plan by WRC-97 was based on “simultaneous planning of feeder links and downlinks with calculation of overall equivalent protection margins” (as defined in § 1.11 of Annex 5 to Appendix 30 and in § 1.12) using the following values of aggregate protection ratio:

–	co-channel = 23 dB;

–	adjacent channel = 15 dB.     (WRC-03)

It was also specified that, for the revision of the Regions 1 and 3 feeder-link Plan, no overall co-channel single entry C/I ratio should be lower than 28 dB.     (WRC-03)

Nevertheless, for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997, the overall equivalent protection margins were calculated using a co-channel overall protection ratio of 30 dB and lower and upper overall adjacent channel protection ratios of 14 dB.

Revision of the Regions 1 and 3 feeder-link Plan at WRC-97 and planning at WRC-2000 were generally based on a set of reference parameters such as the average e.i.r.p., the reference earth station transmitting antenna, all test points placed within the −3 dB contour, a bandwidth of 27 MHz and the predetermined value of C/N. The Regions 1 and 3 feeder-link Plan as established by WRC-2000 is generally based on the use of digital modulation.     (WRC-2000)

WRC-2000 adopted for the protection of digital assignments from digital emissions the following protection ratio values to be applied for calculation of feeder-link equivalent protection margins of the WRC-2000 Regions 1 and 3 feeder-link Plan:

–	27 dB for co-channel signals;

–	22 dB for adjacent channel signals.     (WRC-2000)

During planning at WRC-2000, these values were used for all assignments of the Regions 1 and 3 feeder-link Plan and List, except those for which WRC-2000 adopted different values to be used in the planning process44.     (WRC-03)

Protection masks and associated calculation methods for interference into broadcasting-satellite systems involving digital emissions shall be in accordance with Recommendation ITU-R BO.1293-2 (Annexes 1 and 245).     (WRC-03)

## 3.4	Feeder-link e.i.r.p.

The level of e.i.r.p. of each feeder link is specified in Article 9A.

The level of e.i.r.p. specified in the Plan can only be exceeded under certain conditions explained in § 3.11 of this Annex (see also Article 5, § 5.1.1).

## 3.5	Transmitting antenna

### 3.5.1	Antenna diameter

The feeder-link Plan is based on an antenna diameter of 5 m for the band 17.3-18.1 GHz and 6 m for the band 14.5-14.8 GHz.

For all antenna diameters including antennas smaller than 5 m for the 17.3-18.1 GHz band and 6 m for the 14.5-14.8 GHz band, the off-axis e.i.r.p. shall not exceed the limits indicated by Curve A in Fig. A of § 3.5.3 of this Annex for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997 and by the Curve A′ of Fig. A for other assignments.

### 3.5.2	On-axis gain

The on-axis gain for the 5 m antenna at 17.3-18.1 GHz and for the 6 m antenna at 14.5-14.8 GHz is taken as 57 dBi.

### 3.5.3	Off-axis e.i.r.p. of transmitting antennas

The co-polar and cross-polar off-axis e.i.r.p. values used for the original 1988 feeder-link Plan in Regions 1 and 3 are shown by Curves A and B respectively in Fig. A46.

The corresponding off-axis e.i.r.p. values used for planning at WRC-97 are shown by Curves A′ and B′ in Fig. A as specified in Recommendation ITU-R BO.1295.

### 3.5.4	Pointing accuracy

The Plan has been developed to accommodate a loss in gain of 1 dB due to earth station antenna mispointing.

The deviation of the antenna beam from its nominal pointing direction must not exceed a limit of 0.1° in any direction. Moreover, the angular rotation of the receiving beam about its axis must not exceed a limit of 1°; the limit on rotation is not necessary for beams of circular cross section using circular polarization.

Co-polar component (dBW):

| Curve A (WARC Orb-88) |  |  |  |  |  |  |  | Curve A′ (WRC-97) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E | for | 0° | < |  | < | 0.1° |  |  | E | for | 0° |  |  |  | 0.1° |
| E − 21 − 20 log | for | 0.1° | < |  | < | 0.32° |  |  | E − 21 − 20 log | for | 0.1° | < |  |  | 0.32° |
| E − 5.7 − 53.2 2 | for | 0.32° | < |  | < | 0.44° |  |  | E − 5.7 − 53.2 2 | for | 0.32° | < |  |  | 0.54° |
| E − 25 − 25 log | for | 0.44° | < |  | < | 48° |  |  | E − 28 − 25 log | for | 0.54° | < |  |  | 36.31° |
| E − 67 | for | 48° | < |  |  |  |  |  | E − 67 | for | 36.31° | < |  |  |  |

Cross-polar component (dBW):     (WRC-03)

| Curve B (WARC Orb-88) |  |  |  |  |  |  |  | Curve B′ (WRC-97) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E – 30 | for | 0° | < |  | < | 1.6° |  |  | E – 35 | for | 0° |  |  |  | 1.91° |
| E – 25 – 25 log | for | 1.6° | < |  | < | 48° |  |  | E – 28 – 25 log | for | 1.91° | < |  |  | 36.31° |
| E – 67 | for | 48° | < |  |  |  |  |  | E – 67 | for | 36.31° | < |  |  |  |

where:

E :	earth station e.i.r.p. on the antenna axis (dBW);

:	off-axis angle referred to the main lobe axis (degrees).

## 3.6	Transmitter power

The maximum transmitter power delivered to the input of the antenna of the feeder-link earth station per 27 MHz television channel shall be such as to ensure that the e.i.r.p. envelope in § 3.5.3 is not exceeded except under certain conditions specified in § 3.11.

## 3.7	Satellite receiving antenna

### 3.7.1	Cross-section of receiving antenna beam

Planning has generally been based on beams of elliptical or circular cross-section. When the assignments are implemented, or when the Plan is modified, administrations may use non-elliptical (shaped) beams as described in Annex 2.

For planning purposes at WRC-97, an antenna diameter of 5 m for the band 17.3-18.1 GHz and 6 m for the band 14.5-14.8 GHz were assumed.

The on-axis gain for the 5 m antenna at 17.3-18.1 GHz and for the 6 m antenna at 14.5-14.8 GHz is taken as 57 dBi.

If the cross-section of the receiving antenna beam is elliptical, the effective beamwidth φ0 is a function of the angle of rotation q between the plane containing the satellite and the major axis of the beam cross-section and the plane in which the beamwidth is required.

The relationship between the maximum gain of an antenna and the half-power beamwidth can be derived from the expression:

Gm  =  27 843/ab

where:

a and b are the angles (degrees) subtended at the satellite by the major and minor axes of the elliptical cross-section of the beam. An antenna efficiency of 55% is assumed.

### 3.7.2	Minimum beamwidth

A minimum value of 0.6° for the half-power beamwidth of the receiving antenna has been used for planning.

### 3.7.3	Reference patterns

The reference patterns for the co-polar and cross-polar components of the satellite receiving antenna used for planning at the 1988 Conference (WARC Orb-88) are given by Curves A and B respectively in Fig. B47.

The corresponding curves used for replanning at WRC-97 are given by Curves A′ and B′ in Fig. B, as specified in Recommendation ITU-R BO.1296.

Co-polar relative gain (dB):

Curve A (WARC Orb-88) and Curve A′ (WRC-97):

G = −12 (/0)2 for 0 /0 < 1.3 G = −17.5 − 25 log (/0) for 1.3 /0

After intersection with Curve C, as Curve C

Cross-polar relative gain (dB):

| Curve B (WARC Orb-88) | Curve B′ (WRC-97) |
| --- | --- |
| G = −30 − 12 (/0)2 for 0 /0 < 0.5 G = −33 for 0.5 < /0 < 1.67 | G = −35 for 0 φ/φ0 < 1.75 |
| G = −40 − 40 log for 1.67 /0 | G = −40 − 40 log for 1.75 /0 |
| After intersection with Curve C, as Curve C | After intersection with Curve C, as Curve C |

Curve C:	minus the on-axis gain (Curve C in the above Figure illustrates the particular case of an antenna with an on-axis gain of 44.44 dBi)

where:

:	off-axis angle (degrees)

0:	cross-sectional half-power beamwidth in the direction of interest (degrees).

The relationship between the maximum gain of an antenna and the half-power beamwidth can be derived from the expression in § 3.7.1.

In some cases, to reduce co-polar interference, the pattern shown in Fig. C is used; this use is indicated in the Plan by note 1. This pattern is derived from an antenna producing an elliptical beam with fast roll-off in the main lobe assuming a “beamlet” beamwidth of 0.6o. Three curves for different values of φ0 are shown as examples.

Curve A:	co-polar component (dB relative to main beam gain)

−12 (φ/φ0)2	for	0	≤  φ/φ0  ≤  0.5

−33.33 φ02 	for	0.5  <  φ/φ0  ≤    +  x

−25.23	for	  +  x  <  φ/φ0  ≤  1.45

−(22  +  20 log (φ/φ0))	for	φ/φ0    1.45

after intersection with Curve C, as Curve C.

Curve B:	cross-polar component (dB relative to main beam gain)

−30	for	0  ≤  φ/φ0  <  2.51

after intersection with Curve A, as Curve A.

Curve C:	minus the on-axis gain (curves A and C represent examples for three antennas having different values of 0 as labelled in Fig. C. The on-axis gains of these antennas are 37, 43 and 49 dBi, respectively),

where:

φ:	off-axis angle (degrees);

φ0:	dimension of the minimum ellipse fitted around the feeder-link service area in the direction of interest (degrees);

### 3.7.4	Pointing accuracy

The deviation of the receiving antenna beam from its nominal pointing direction must not exceed 0.1o in any direction. Moreover, the angular rotation of the receiving beam about its axis must not exceed 1°; this limit is not necessary for beams of circular cross-section using circular polarization.

### 3.7.5	Composite beam     (WRC-2000)

A composite beam represents a single beam (i.e. “simulated shaped beam”) and is formed by combining two or more elliptical beams at a given orbital position. In general, composite beams were used at WRC-2000 for administrations which had more than one beam at a given orbital position in the WRC-97 Regions 1 and 3 feeder-link Plan.     (WRC-2000)

## 3.8	System noise temperature

The satellite system noise temperature values generally used in the Plan at the 1988 Conference (WARC Orb-88) are 1 800 K for 17 GHz and 1 500 K for 14 GHz48. For revising the Regions 1 and 3 Plan at WRC-97 these values are 900 K for 17 GHz and 750 K for 14 GHz. A value of 600 K was used for the 17 GHz band in the revision of the Regions 1 and 3 Plan at WRC-2000. WRC-2000 did not change the value for the 14 GHz band.     (WRC-03)

## 3.9	Polarization

In Regions 1 and 3, circular polarization was normally used for the purpose of planning the feeder-links.

For the definitions of the terms “direct and indirect polarization”, see § 3.2.3 of Annex 5 to Appendix 30.

For the planning of the broadcasting-satellite service, circular polarization is generally used. However, for implementation of assignments in the Regions 1 and 3 Plan, linear polarization may also be used subject to successful application of the modification procedure of Article 4. Linear polarization is defined in Recommendation ITU-R BO.1212. This Recommendation should be used when analysing linearly polarized signals.

## 3.10	Automatic gain control

The downlink Plan was based on constant satellite output power. However, the feeder-link Plan does not take account of the effect of automatic gain control on board satellites. Up to 15 dB of automatic gain control is permitted, subject to no increase in interference to other satellite systems.

## 3.11	Power control

In Regions 1 and 3, a permitted increase which may be used to overcome rain fading for each assignment is included in the Plan.

In the calculation, in cases where satellites do not use common or adjacent channels cross-polarized to each other, the maximum permissible e.i.r.p. increase, which must not exceed 10 dB, corresponds to the amount of rain attenuation which occurs on the interfering feeder link.

### 3.11.1	Method for determination of the increase in e.i.r.p. during rain attenuation for an assignment over the Plan value

Condition to be observed

The increase in e.i.r.p. of the assignment studied must not entail an impairment of more than 0.5 dB of the feeder-link equivalent protection margin of any other assignment of any other administration.

Calculation method

Step 1:	compile a list of all assignments of other administrations (A, B, C, . . .) in the same orbital position and positions within  6° (or further if no station is found within 6° arc) liable to suffer interference from the assignment studied.

Step 2:	calculate the feeder-link equivalent protection margin of assignment A in free-space conditions, taking account of all interference sources affecting A at the worst test points, namely:

–	for assignment A: the point corresponding to the minimum C/N ratio;

–	for each interference source affecting A: the point corresponding to the maximum interference power affecting A.

Step 3:	introduce for the assignment studied the rain attenuation for 0.1% of the worst month and the corresponding rain depolarization value.

Step 4:	recalculate the feeder-link equivalent protection margin of assignment A at the worst test points, namely:

–	for assignment A: the test point used in Step 2 above;

–	for the assignment studied: the test point corresponding to the maximum interference power affecting A.

At this stage, the e.i.r.p. of the assignment studied is that contained in the Plan.

Step 5:	increase the e.i.r.p. of the assignment studied by 0.1 dB and recalculate the equivalent uplink margin of A as in Step 4 above.

Step 6:	repeat the operation of Step 5 above until the equivalent uplink margin of assignment A is impaired by more than 0.5 dB in relation to the value found under Step 2 above, or until the e.i.r.p. increase exceeds 10 dB or the rain attenuation (see Step 3). Adopt the e.i.r.p. increase in the preceding iteration step.

Step 7:	repeat the operations in Step 2 to Step 6 above, considering the assignments B, C, . . .

Step 8:	adopt the smallest of the increases in e.i.r.p. found under Step 6 above for the various assignments A, B, C, . . .

### 3.11.2	Propagation model

For the calculation of rain attenuation for 0.1% of the worst month, the model described in § 2.2 should be used. It shall be assumed that the 0.1% value is 3.3 times the 1% value (dB).

Rain depolarization shall be calculated on the basis of attenuation, using the method described in § 2.4.

### 3.11.3	Variation of power with rain attenuation

The instantaneous increase in power to overcome rain attenuation must not exceed the bounds given by the characteristics shown in Fig. 5.

### 3.11.4	Procedures

An administration wishing to introduce power control may use a value not exceeding that given in Article 9A or it may request, where this is possible, the use of a higher value for a given earth station location. In this latter case, it shall request the Bureau to calculate the maximum permissible value for that site. The administration shall provide the Bureau with the coordinates of the station, the proposed antenna characteristics, including the off-axis co-polar and cross-polar characteristics, and the rain climatic zone.

The Bureau shall calculate the permissible increase in power using the method described in § 3.11.1.

The Bureau shall communicate the results of the calculations to the requesting administrations as well as to those administrations whose feeder-link equivalent protection margin is reduced.

In any case, the permitted increase in e.i.r.p. above that given in the Plan shall not exceed 10 dB.

In the event of modifications to the Plan, the Bureau shall recalculate the value of power control for the assignment subject to the modification and insert the appropriate value for that assignment in the Plan. A modification to the Plan shall not require the adjustment of the values of permissible power increase of other assignments in the Plan.

## 3.12	(SUP - WRC-97)

## 3.13	Depolarization compensation

The Plan is developed without the use of depolarization compensation. Depolarization compensation is permitted only to the extent that interference to other satellites does not increase by more than 0.5 dB49 relative to that calculated in the feeder-link Plan.

## 3.14	Amplitude-modulation to phase-modulation conversion

The degradation caused by AM to PM conversion was taken into account when calculating the carrier-to-noise ratio of the feeder link. A value of 2.0 dB was allowed.

## 3.15	Orbit positions

The Plan is generally based on the use of a regular spacing of 6°. The orbital positions are those given in the Plan.     (WRC-03)

## 3.16	Satellite station-keeping

Space stations in the broadcasting-satellite service must be maintained in position with an accuracy equal to or better than 0.1° in the E-W direction. For such space stations, the maintenance of the tolerance 0.1° in the N-S direction is recommended but is not a requirement.

## 3.17	Orbital separation limit for interference calculation     (WRC-2000)

WRC-2000 has adopted the use of an orbital separation limit for interference calculation in Regions 1 and 3. Beyond this limit no interference was taken into account.     (WRC-2000)

Initially, the values used for the orbital separation limit were 15° for co-polar and 9° for cross-polar emissions. At a later stage, the unique value of the orbital separation limit of 9° was adopted by WRC-2000.     (WRC-2000)

# 4	Basic technical characteristics for Region 2

## 4.1	Translation frequency and guard bands

The feeder-link Plan is based on the use of a single frequency translation of 5.1 GHz between the 17 GHz feeder-link channels and the 12 GHz downlink channels. Other values of the translation frequency may be used, provided that the corresponding channels have been assigned to the space station of the administration concerned.

With a single value frequency translation between the feeder-link frequency band (17.3-17.8 GHz) and the downlink frequency band (12.2-12.7 GHz), the guard bands present in the downlink Plan result in corresponding bandwidths of 12 MHz at the upper and lower feeder-link band edges. These feeder-link guard bands may be used to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service.     (WRC-03)

## 4.2	Carrier-to-noise ratio

Section 3.3 of Annex 5 to Appendix 30 provides guidance for planning and the basis for the evaluation of the carrier-to-noise ratios of the feeder-link and downlink Plans.

As a guidance for planning, the reduction in quality in the downlink due to thermal noise in the feeder link is taken as equivalent to a degradation in the downlink carrier-to-noise ratio of approximately 0.5 dB not exceeded for 99% of the worst month.

## 4.3	Carrier-to-noise ratio

Section 3.4 of Annex 5 to Appendix 30 provides guidance for planning for the contribution of the feeder-link co-channel interference to the overall co-channel carrier-to-interference ratio. However, the feeder-link and downlink Plans are evaluated on the overall equivalent protection margin which includes the combined downlink and feeder-link contributions. Definitions given in § 1.7, 1.8, 1.9, 1.10 and 1.11 of this Annex and the protection ratios given in Section 3.4 of Annex 5 to Appendix 30 are used in the analysis of the Plans.

For the adjacent channels, the Plan is based on an orbital separation of 0.4° between nominally co-located satellites having cross-polarized adjacent channel assignments.

For the second adjacent channels, the Plan is based on a 10 dB improvement on the feeder-link carrier-to-interference ratio due to the satellite receive filtering.

## 4.4	Transmitting antenna

### 4.4.1	Antenna diameter

The feeder-link Plan is based on an antenna diameter of 5 m.

The minimum antenna diameter permitted in the Plan is 2.5 m. However, the feeder-link carrier-to-noise ratio and carrier-to-interference ratio resulting from the use of antennas with diameters smaller than 5 m would generally be less than those calculated in the Plan.

The use of antennas larger than 5 m, with corresponding values of on-axis e.i.r.p. higher than the planned value (indicated in § 4.4.3) but without augmented off-axis e.i.r.p., is permitted if the orbital separation between the assigned orbital location of the administration and the assigned orbital location of any other administration is greater than 0.5°.

Antennas with diameters larger than 5 m can also be implemented if the above orbital separation is less than 0.5° and if the e.i.r.p. of the desired feeder-link earth station does not exceed the planned value of e.i.r.p.

If the above orbital separation is less than 0.5° and if the e.i.r.p. of the desired feeder-link earth station exceeds the planned value, agreement between administrations is required.

### 4.4.2	Transmitting antenna reference patterns     (WRC-03)

The co-polar and cross-polar reference patterns of transmitting antennas used for planning in Region 2 are given in Fig. 6.

### 4.4.3	Antenna efficiency

The Plan is based on an antenna efficiency of 65%. The corresponding on-axis gain for an antenna having a 5 m diameter is 57.4 dBi at 17.55 GHz, and the corresponding value of e.i.r.p. used for planning purposes is 87.4 dBW.

### 4.4.4	Pointing accuracy

The Plan has been developed to accommodate a loss in gain due to earth station antenna mis-pointing of 1 dB. Under no circumstances shall the Plan allow for a mis-pointing angle greater than 0.1°.

Curve A:	co-polar component (dBi)

Gco = Gmax			for	0°        φ  <  0.1°

Gco = 36 − 20 log φ		for	0.1°     φ  <  0.32°

Gco = 51.3 − 53.2 φ		for	0.32°   φ  <  0.54°

Gco = max (29 – 25 log φ)		for	0.54°   φ    180°

If Gco > Gmax: Gco = Gmax			(WRC-03)

Curve B:	cross-polar component (dBi)

Gcross = Gmax − 30		for	0°             φ  <  (0.6/D)°

Gcross = max (9 − 20 log φ−)		for	(0.6/D)°   φ    180°

If Gcross > Gmax − 30: Gcross = Gmax − 30	(WRC-03)

where:

φ :	off-axis angle referred to the main-lobe axis (degrees)

Gmax :	on-axis co-polar gain of the antenna (dBi)

D :	diameter of the antenna (m) (D ≥ 2.5).

NOTE 1 – In the angular range between 0.1° and 0.54°, the co-polar gain must not exceed the reference pattern.

NOTE 2 – In the angular range between 0° and (0.6/D)°, the cross polar gain must not exceed the reference pattern.

NOTE 3 – At the larger off–axis angles and for 90% of all side-lobe peaks in each of the reference angular windows, the gain must not exceed the reference pattern. The reference angular windows are 0.54° to 1°, 1° to 2°, 2° to 4°, 4° to 7°, 7° to 10°, 10° to 20°, 20° to 40°, 40° to 70°, 70° to 100° and 100° to 180°. The first reference angular window for evaluating the cross-polar component should be (0.6/D)° to 1°.

## 4.5	Transmit power

The maximum transmit power delivered to the input of the antenna of the feeder-link earth station is 1 000 W per 24 MHz television channel. This level of power can only be exceeded under certain conditions specified in § 4.10.

## 4.6	Receiving antenna

### 4.6.1	Cross-section of receiving antenna beam

Planning has been based on beams of elliptical or circular cross-section. When the assignments are implemented, or when the Plan is modified, administrations may use non-elliptical or shaped beams.

If the cross-section of the receiving antenna beam is elliptical, the effective beamwidth φ0 is a function of the angle of rotation q between the plane containing the satellite and the major axis of the beam cross-section and the plane in which the beamwidth is required.

The relationship between the maximum gain of an antenna and the half-power beamwidth can be derived from the expression:

Gm  =  27 843/ab

or

Gm  (dB)  =  44.44  –  10 log a  –  10 log b

where:

a and b are the angles (degrees) subtended at the satellite by the major and minor axes of the elliptical cross-section of the beam.

An antenna efficiency of 55% is assumed.

### 4.6.2	Minimum beamwidth

A minimum value of 0.6° for the half-power beamwidth of the receiving antenna has been agreed on for planning.

### 4.6.3	Receiving antenna reference patterns     (WRC-03)

The reference patterns for the co-polar and cross-polar components of the satellite receiving antenna used in preparing the Plan are given in Fig. 7.

Where it was necessary to reduce interference, the pattern shown in Fig. 8 was used; this use will be indicated in the Plan by an appropriate symbol. This pattern is derived from an antenna producing an elliptical beam with fast roll-off in the main lobe. Three curves for different values of φ0 are shown as examples.

Curve A:	co-polar component (dB relative to main beam gain)

−12 (φ/φ0)2	for	0  ≤	(φ/φ0)	≤  1.45

−(22  +  20 log (φ/φ0))	for		(φ/φ0)	  1.45

after intersection with Curve C, as Curve C.

Curve B:	cross-polar component (dB relative to main beam gain)

−30	for	0  ≤	(φ/φ0)	≤  2.51

after intersection with Curve A, as Curve A.

Curve C:	minus the on-axis gain (Curve C in this Figure illustrates the particular case of an antenna with an on-axis gain of 46 dBi)

Curve A:	co-polar component (dB relative to main beam gain)

−12 (φ/φ0)2	for	0	≤  φ/φ0	≤  0.5

−33.33 φ02 (φ/φ0  –  x)2	for	0.5	<  φ/φ0	≤

−25.23	for		<  φ/φ0 	≤  1.45

−(22  +  20 log (φ/φ0))	for	φ/φ0	  1.45

after intersection with Curve C, as Curve C.	(WRC-03)

Curve B:	cross-polar component (dB relative to main beam gain)

−30	for	0	≤  (φ/φ0)	≤  2.51

after intersection with Curve A, as Curve A.

Curve C:	minus the on-axis gain (Curves A and C represent examples for three antennas having different values of φ as labelled in Fig. 8. The on-axis gains of these antennas are 37, 43 and 49 dBi, respectively).

where:

φ :	off-axis angle (degrees)

φ :	dimension of the minimum ellipse fitted around the feeder-link service area in the direction of interest (degrees)

x  =	0.5

### 4.6.4	Pointing accuracy

The deviation of the receiving antenna beam from its nominal pointing direction must not exceed 0.1° in any direction. Moreover, the angular rotation of the receiving beam about its axis must not exceed 1°; this latter limit is not necessary for beams of circular cross-section using circular polarization.

## 4.7	System noise temperature

The Plan is based on a value of 1 500 K for the satellite system noise temperature. WRC-03 decided that for feeder-link assignments in the Plan which have not been subsequently modified through successful application of Article 4, a value of 600 K (instead of 1 500 K) is used in application of § 5 of Annex 1 and § 1 of Annex 4. For those assignments which have been subsequently modified, the noise temperature value provided in that modification is used.     (WRC-03)

## 4.8	Polarization

4.8.1	In Region 2, for the purpose of planning the feeder links, circular polarization is used.

4.8.2	In the cases where there are polarization constraints, use of polarization other than circular is permitted only upon agreement of administrations that may be affected.

## 4.9	Automatic gain control

4.9.1	The Plan is based on the use of automatic gain control on board satellites to maintain a constant signal level at the satellite transponder output.

4.9.2	The dynamic range of automatic gain control is limited to 15 dB when satellites are located within 0.4° of each other and operate on cross-polarized adjacent channels serving common or adjacent feeder-link service areas.

4.9.3	The 15 dB limit of automatic gain control does not apply to satellites other than those specified in § 4.9.2 above.

## 4.10	Power control

The Plan has been developed without the use of power control.

The use of transmit power levels higher than those given in § 4.5 is permitted only when rain attenuation exceeds 5 dB at 17 GHz. In such cases, the transmit power may be increased by the amount that the instantaneous rain attenuation exceeds 5 dB at 17 GHz up to the limit given in Table 5.

TABLE 5

Transmit radio frequency power (delivered to the input of the feeder-link
earth station antenna) permitted in excess of 1 000 W
as a function of elevation angle

| Elevation angle of feeder-link earth station antenna (degrees) | Transmit power permitted in excess of 1 000 W (dB) |
| --- | --- |
| 0 to 40 40 to 50 50 to 60 60 to 90 | 0 2 3 5 |

## 4.11	Site diversity

Site diversity refers to the alternate use during rain of two or more transmitting earth stations which may be separated by sufficient distance to ensure uncorrelated rainfall conditions.

The use of site diversity is permitted and is considered to be an effective technique for maintaining high carrier-to-noise ratio and carrier-to-interference ratio during periods of moderate to severe rain attenuation. However, the Plan is not based on the use of site diversity.

## 4.12	Depolarization compensation

The Plan is developed without the use of depolarization compensation. Depolarization compensation is permitted only to the extent that interference to other satellites does not increase by more than 0.5 dB relative to that calculated in the feeder-link Plan.

## 4.13	Minimum separation between satellites

Figure 9 illustrates two adjacent clusters of satellites separated by 0.9° between the centres of the clusters. A identifies a satellite of administration . A cluster is formed by two or more satellites separated by 0.4° and located at two nominal orbital positions as specified in the Plan; one position for right-hand polarized channels and the other position for left-hand polarized channels.

### 4.13.1	Satellites of the same cluster

The Plan is based on an orbital separation of 0.4° between satellites having cross-polarized adjacent channels (i.e. satellites located at +0.2° and −0.2° from the centre of the cluster). However, satellites within a cluster may be located at any orbital position within the cluster, requiring only the agreement of the other administrations having satellites sharing the same cluster. Such orbital positioning of satellites within a cluster is illustrated in Fig. 9 by some of the satellites A5, A6 and A7.

The station-keeping tolerance of 0.1° indicated in § 3.11 of Annex 5 to Appendix 30 must be applied to satellites located at any position within the 0.4° wide cluster.

### 4.13.2	Satellites of different clusters

In the Plan, the orbital separation between the centres of adjacent clusters of satellites is at least 0.9°. The value of 0.9° is also the minimum orbital separation to provide flexibility in the implementation of feeder links indicated in § 4.4.1 without the need for an agreement (see § 4.13.1).

ANNEX 4     (Rev.WRC-15)

Criteria for sharing between services

# 1	Threshold values for determining when coordination is required between, on one hand, transmitting space stations in the fixed-satellite service or the broadcasting-satellite service and, on the other hand, a receiving space station in the feeder-link Plan or List or a proposed new or modified receiving space station in the List, in the frequency bands 17.3-18.1 GHz (Regions 1 and 3) and in the feeder-link Plan or a proposed modification to the Plan in the frequency band 17.3-17.8 GHz (Region 2)     (WRC-03)

With respect to § 7.1, Article 7, coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service with a receiving space station in a broadcasting-satellite service feeder link in the Regions 1 and 3 feeder-link Plan or List, or a proposed new or modified receiving space station in the List, or in the Region 2 feeder-link Plan or proposed modification to the Plan is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite service feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of ΔTs / Ts corresponding to 6%. ΔTs / Ts is calculated in accordance with Case II of the method given in Appendix 8.     (WRC-03)

# 2	Threshold values for determining when coordination is required between transmitting feeder-link earth stations in the fixed-satellite service in Region 2 and a receiving space station in the Regions 1 and 3 feeder-link Plan or List or a proposed new or modified receiving space station in the List, in the frequency band 17.8-18.1 GHz     (WRC-03)

With respect to § 7.1, Article 7, coordination of a transmitting feeder-link earth station in the fixed-satellite service with a receiving space station in a broadcasting-satellite feeder link in the Regions 1 and 3 feeder-link Plan or List, or a proposed new or modified receiving space station in the List, is required when the power flux density arriving at the receiving space station of a broadcasting-satellite service feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of ΔT/T corresponding to 6%, where ΔT/T is calculated in accordance with the method given in Appendix 8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the necessary bandwidth of the feeder-link carriers.     (WRC-03)

# 3	Threshold values for determining when coordination is required between transmitting earth stations in the fixed-satellite service in the frequency band 14.5-14.75 GHz (in countries listed in Resolution 163 (WRC-15)) and 14.5-14.8 GHz (in countries listed in Resolution 164 (WRC-15)) not for feeder links for the broadcasting-satellite service and a receiving space station subject to a Plan in the frequency band 14.5-14.8 GHz     (WRC-15)

With respect to § 7.1, Article 7, coordination of a transmitting earth station in the fixed-satellite service with a receiving space station in a broadcasting-satellite feeder link in the Regions 1 and 3 feeder-link Plan or List, or a proposed new or modified receiving space station in the List, is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite service feeder link of another administration exceeds the value of:

−197.0 – GRx dB(W/(m2 · Hz)) with regard to the assignment in the Regions 1 and 3 feeder-link Plan or

−193.9 – GRx dB(W/(m2 · Hz)), with regard to the assignment included in the feeder-link List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3,

where GRx is the relative receive antenna gain of the space station subject to a Plan at the location of the transmitting earth station in the fixed-satellite service not for feeder links for the broadcasting-satellite service.     (WRC-15)

APPENDIX 30B (REV.WRC-15)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

TABLE  OF  CONTENTS

Page

Article 1	Objective of the provisions and associated Plan		2

Article 2	Definitions		2

Article 3	Frequency bands		3

Article 4	Execution of the provisions and associated Plan		3

Article 5	(SUP - WRC-07)

Article 6	Procedures for the conversion of an allotment into an assignment, for the introduction of an additional system or for the modification of an assignment in the List		4

Article 7	Procedure for the addition of a new allotment to the Plan for a new Member State of the Union		10

Article 8	Procedure for notification and recording in the Master Register of assignments in the planned bands for the fixed-satellite service		12

Article 9	General provisions		14

Article 10	Plan for the fixed-satellite service in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz, 10.70-10.95 GHz, 11.20-11.45 GHz and 
12.75-13.25 GHz		15

Article 11	Period of validity of the provisions and associated Plan		26

ANNEXES

Annex 1	Parameters used in characterizing the fixed-satellite service allotment Plan		26

Annex 2	(SUP - WRC-07)

Annex 3	Limits applicable to submissions received under Article 6 or Article 7		30

Annex 4	Criteria for determining whether an allotment or an assignment is considered to be affected		31

Appendix 1 	Method for determination of the overall single-entry and aggregate 
to Annex 4	carrier-to-interference value averaged over the necessary bandwidth of 
	the modulated carrier		32

Appendix 2	Method for determination of the carrier-to-noise (C/N) 
to Annex 4	values		35

_________________

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

ARTICLE 1     (REV.WRC-07)

Objective of the provisions and associated Plan

1.1	The objective of the procedures prescribed in this Appendix is to guarantee in practice, for all countries, equitable access to the geostationary-satellite orbit in the frequency bands of the fixed-satellite service covered by this Appendix.

1.2	The procedures prescribed in this Appendix shall in no way prevent the implementation of assignments in conformity with the national allotments of the Plan.     (WRC-07)

ARTICLE 2     (REV.WRC-07)

Definitions

2.1	Conference: World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and the Planning of Space Services Utilizing It, First Session, Geneva, 1985; Second Session, Geneva, 1988.

2.2	Plan: The Plan for the fixed-satellite service in the frequency bands contained in this Appendix, consisting of national allotments.     (WRC-07)

2.2bis	List of assignments (hereinafter, called the “List”):  The List associated with the Plan containing assignments resulting from the successful application of the provisions of Article 6 of Appendix 30B or the application of Resolution 148 (WRC-07).     (WRC-07)

2.3	Allotment: For the purpose of this Appendix, an allotment comprises:

–	a nominal orbital position;

–	a bandwidth of 800 MHz (up-link and down-link) in the frequency bands listed in Article 3 of this Appendix;

–	a service area for national coverage.     (WRC-07)

2.4	Existing systems: Those satellite systems in the frequency bands covered by this Appendix which are identified in Resolution 148 (WRC-07).     (WRC-07)

2.5	(SUP - WRC-07)

2.6	Additional system: For the application of the provisions of this Appendix, an additional system is a system for which the assignments submitted by an administration are not the result of conversion of an allotment into assignments. When submitting an additional system, the national allotment in the Plan of the submitting administration shall be retained. An additional system may also be submitted on behalf of a group of named administrations, with one administration designated to act as the notifying administration in respect of that additional system.     (WRC-07)

2.6bis	When submitting additional system(s), administrations shall fully comply with the requirements stipulated in Article 44 of the ITU Constitution. In particular, these administrations shall limit the number of orbital positions and associated spectrum so that:

a)	the orbital/spectrum natural resources are used rationally, efficiently and economically; and

b)	the use of multiple orbital locations to cover the same service area is avoided.     (WRC-07)

ARTICLE 3

Frequency bands

3.1	The provisions of this Appendix shall apply to the fixed-satellite service in the frequency bands between:

–	4 500 and 4 800 MHz (space-to-Earth);

–	6 725 and 7 025 MHz (Earth-to-space);

–	10.70 and 10.95 GHz (space-to-Earth);

–	11.20 and 11.45 GHz (space-to-Earth);

–	12.75 and 13.25 GHz (Earth-to-space).

ARTICLE 4

Execution of the provisions and associated Plan

4.1	The Member States of the Union shall adopt, for their fixed-satellite service stations operating in the frequency bands referred to in this Appendix, the characteristics consistent with those specified in the Plan and its associated provisions.

4.2	The Member States of the Union shall not change the characteristics, or bring into use assignments to fixed-satellite service stations, or stations in the other services to which these frequency bands are allocated, except as provided for in the Radio Regulations and the appropriate Articles and Annexes of this Appendix.

ARTICLE 5     (SUP - WRC-07)

ARTICLE 6     (REV.WRC-15)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2     (WRC-15)

6.1	When an administration intends to convert an allotment into an assignment or when an administration, or one acting on behalf of a group of named administrations3, intends to introduce an additional system or modify the characteristics of assignments in the List that have been brought into use, it shall, not earlier than eight years and not later than two years before the planned date of bringing the assignment into use, send to the Bureau the information specified in Appendix 44, 5.

6.2	If the information received by the Bureau under § 6.1 is found to be incomplete, the Bureau shall immediately seek any clarification required and information not provided from the administration concerned.

6.3	Upon receipt of a complete notice under § 6.1, the Bureau shall examine it with respect to its conformity with:

a)	the Table of Frequency Allocations and the other provisions6 of the Radio Regulations, except those provisions relating to conformity with the fixed-satellite service Plan; and

b)	Annex 3 to this Appendix.

6.4	When the examination with respect to § 6.3 leads to an unfavourable finding, the relevant part of the notice shall be returned to the notifying administration with an indication of the appropriate action.

6.5	When the examination of each assignment in a notice received under § 6.1 with respect to § 6.3 leads to a favourable finding, the Bureau shall use the method of Annex 4 to determine administrations whose:

a)	allotments in the Plan; or

b)	assignments which appear in the List; or

c)	assignments which the Bureau has previously examined under this paragraph after receiving complete information in accordance with § 6.1 of this Article,

are considered as being affected by any assignment in that notice.

6.6	The Bureau shall then identify those administrations whose territories have been included in the service area of the assignment under examination. The notifying administration shall seek the agreement of any administration whose territory is partially or wholly included in the intended service area of the assignment.

6.7	The Bureau shall publish, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 6.1 and examined under § 6.5, together with:

a)	the names of the administrations identified under § 6.5 and the corresponding allotments in the Plan, assignments in the List and assignments for which the Bureau has previously received complete information in accordance with § 6.1 and which it has examined under § 6.5 of this Article;

b)	the names of the administrations identified under § 6.6.

6.8	Following the examination under § 6.5 and 6.6, the Bureau shall immediately send a telegram or fax to the administration that has submitted the notice under § 6.1, drawing attention to the requirement to seek and obtain the agreement of those administrations identified in the Special Section of the BR IFIC published under § 6.7.

6.9	The Bureau shall also send a telegram or fax to each administration listed in the Special Section of the BR IFIC published under § 6.7, drawing its attention to the information it contains.

6.10	Comments from administrations identified as affected under § 6.5 in the Special Section of the BR IFIC published under § 6.7 shall be sent to the Bureau and to the administration that has submitted the notice under § 6.1, either directly or through the Bureau, within a period of four months following the date of the publication in the BR IFIC. When an administration has not replied within this four-month period, it is deemed that this administration has not agreed to the proposed assignment, unless the provisions of § 6.13 to 6.15 are applied.

The above-mentioned four-month period shall be extended for an administration that has requested the assistance of the Bureau by up to thirty days following the date on which the Bureau communicated the result of its action.

6.11	Thirty days prior to the expiry of the same four month period, the Bureau shall dispatch a reminder telegram or fax to each administration listed in the Special Section published under § 6.7 which has not made its comments under § 6.10, bringing the matter to its attention.

6.12	An administration which considers that it should have been identified as affected in the publication referred to under § 6.7 above shall, within four months of the date of publication of the relevant BR IFIC, request the Bureau to include its name in the publication while providing the reasons therefor. The Bureau shall study this information on the basis of Annex 4 and shall inform both the affected administration and the administration that submitted the notice of its conclusions. Should the Bureau agree to the administration’s request, it shall publish an addendum to the publication under § 6.7.

6.13	After the same time period as specified in § 6.10, the notifying administration may request the Bureau to assist in respect of an administration which has not replied within this time period.

6.14	The Bureau, acting on a request for assistance under § 6.13, shall send a reminder to the administration which has not replied, together with the results of its previously published compatibility analysis, containing the change in the values referred to in paragraph 2.3 of Annex 4 to Appendix 30B, requesting a decision.      (WRC-15)

6.14bis	Fifteen days before the expiry of the 30-day period referred to in § 6.15, the Bureau shall send a reminder to the above-mentioned administration drawing its attention to the consequence of no reply.

6.15	If no decision is communicated to the Bureau within thirty days after the date of dispatch of the reminder under § 6.14, it shall be deemed that the administration which has not given a decision has agreed to the proposed assignment.

6.16	An administration may at any time during or after the above-mentioned four-month period inform the Bureau about its objection to being included in the service area of any assignment, even if this assignment has been entered in the List. The Bureau shall then inform the administration responsible for the assignment and exclude the territory and test points that are within the territory of the objecting administration from the service area. The Bureau shall update the reference situation without reviewing the previous examinations.

6.17	If agreements have been reached with administrations published in accordance with § 6.7, the administration proposing the new or modified assignment may request the Bureau to have the assignment entered into the List, indicating the final characteristics of the assignment together with the names of the administrations with which agreement has been reached. For this purpose, it shall send to the Bureau the information specified in Appendix 4. In submitting the notice, the administration may request the Bureau to examine the notice under § 6.19, 6.21 and 6.22 (entry into the List) and then the notice submitted separately under Article 8 of this Appendix (notification).      (WRC-15)

6.18	If the information received by the Bureau under § 6.17 is found to be incomplete, the Bureau shall immediately seek any clarification required and information not provided from the administration concerned.

6.19	Upon receipt of a complete notice under § 6.17, the Bureau shall examine each assignment in the notice:

a)	with respect to the requirement for the notifying administration to seek the agreement of those administrations identified in § 6.6;

b)	with respect to its conformity with respect to the Table of Frequency Allocations and the other provisions7 of the Radio Regulations, except those provisions relating to conformity with the fixed-satellite service Plan; and

c)	with respect to its conformity with Annex 3 to this Appendix.

6.20	When the examination with respect to § 6.19 of an assignment received under § 6.17 leads to an unfavourable finding, the notice shall be returned to the notifying administration with an indication that subsequent resubmission under § 6.17 will be considered with a new date of receipt.

6.21	When the examination with respect to § 6.19 of an assignment received under § 6.17 leads to a favourable finding, the Bureau shall use the method of Annex 4 to examine if the affected administrations and the corresponding:

a)	allotments in the Plan;

b)	assignments which appear in the List at the date of receipt of the examined notice submitted under § 6.1;

c)	assignments for which the Bureau has previously received complete information in accordance with § 6.1 and has conducted the examination under § 6.5 of this Article at the date of receipt of the examined notice submitted under § 6.1;

indicated in the Special Section published under § 6.7 and whose agreement has not been provided under § 6.17 are still considered as being affected by that assignment.

6.22	The Bureau shall determine if the final characteristics of an assignment received under § 6.17 cause more interference by checking if they decrease the uplink and/or downlink single-entry C/I value of an allotment in the Plan or an assignment in the List or an assignment for which the Bureau has received complete information in accordance with this Article before the date of receipt of the complete notice under § 6.17. If the final characteristics cause more interference than was produced by the characteristics previously submitted under § 6.1 to an allotment in the Plan or assignment in the List or assignment for which the Bureau has received complete information in accordance with this Article, the Bureau shall use the method of Annex 4 to determine whether that allotment or assignment is considered as being affected by the proposed assignment without the explicit agreement of the identified administrations.

6.23	In the event of a favourable finding under § 6.21 and 6.22, the Bureau shall enter the proposed assignment in the List8 and publish in a Special Section of its BR IFIC the characteristics of the assignment received under § 6.17, together with the names of administrations with which the provisions of this Article have been successfully applied. The administration may then notify the assignment in accordance with Article 8 of this Appendix.

6.24	When the examination under § 6.21 or 6.22 leads to an unfavourable finding, the Bureau shall return the notice received under § 6.17 to the notifying administration together with the names of the administrations with which necessary agreements under § 6.21 or 6.22 have not been provided and with an indication that subsequent resubmission under § 6.17 will be considered with a new date of receipt.

6.25	After a notice is returned under § 6.24, should the notifying administration resubmit the notice and insist upon its reconsideration, the Bureau, on the condition of a favourable finding under § 6.21 and 6.22 with respect to allotments in the Plan, shall enter the assignment provisionally in the List, with an indication of those administrations whose assignments were the basis of the unfavourable finding. The entry in the List shall be changed from provisional to definitive only if the Bureau is informed that all required agreements have been obtained.

6.26	Notices submitted under § 6.25 shall also include a signed commitment by the notifying administration, indicating that use of an assignment recorded in the List under § 6.25 shall not cause unacceptable interference to, nor claim protection from, those assignments for which agreement still needs to be obtained.

6.27	When an assignment is entered provisionally in the List under the provisions of § 6.25, that assignment shall not be taken into account in updating the reference situation of those assignments which were the basis for the unfavourable finding. If the Bureau is informed that an agreement has been reached with respect to a given assignment, the reference situation of this assignment shall be updated.

6.28	Should the assignments that were the basis of the unfavourable finding not be brought into use within the period specified in § 6.1 or within the extension period under § 6.31bis, then the status of the assignment in the List shall be reviewed accordingly.    (WRC-12)

6.29	Should unacceptable interference be caused by an assignment entered in the List under § 6.25 to any assignment in the List which was the basis of the disagreement, the notifying administration of the assignment entered in the List under § 6.25 shall, upon receipt of advice thereof, immediately eliminate this unacceptable interference.

6.30	When an assignment included in the List is no longer required, the notifying administration shall so inform the Bureau.

6.31	The regulatory time-limit for bringing into use an assignment to a space station of a satellite network is no more than eight years from the date of receipt by the Bureau of the complete notice under § 6.1.      (WRC-15)

6.31bis	The regulatory time-limit in § 6.31 for bringing into use of an assignment to a space station of a satellite network may be extended once by not more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 17 February 2012, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 6.31:

–	date of launch failure;

–	due diligence information as required in Resolution 49 (Rev.WRC-15), if this Resolution applies to the satellite network in which the space station is to operate, for the assignments with respect to the satellite that suffered the launch failure, if that information has not already been provided.

If, 11 months after the request for extension, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information, the Bureau shall promptly send a reminder to the notifying administration. If, for a satellite network or satellite system to which Resolution 49 (Rev.WRC-15) applies, the administration has not provided to the Bureau updated Resolution 49 (Rev.WRC-15) information for the new satellite under procurement within one year of the request for extension, the related frequency assignments shall lapse.     (WRC-15)

6.32	Thirty days prior to the date of bringing into use under § 6.31 or § 6.31bis, the Bureau shall dispatch a reminder telegram or fax to the notifying administration which has not brought its assignment into use, bringing the matter to its attention.     (WRC-12)

6.33

When:

i)	an assignment is no longer required; or

ii)	an assignment recorded in the List and brought into use has been suspended for a period exceeding the suspension period resulting from the application of § 8.17 below and ending after the expiry date specified in § 6.31; or     (WRC-15)

iii)	an assignment recorded in the List has not been brought into use within the eight-year period following the receipt by the Bureau of the relevant complete information under § 6.1 (or within the extended period in the event of an extension under § 6.31bis), with the exception of assignments submitted by new Member States where § 6.35 and 7.7 apply,

the Bureau shall:

a)	publish in a Special Section of its BR IFIC the cancellation of the related Special Sections and the assignments recorded in the Appendix 30B List;

b)	if the cancelled assignment is the result of a conversion of an allotment without modification, reinstate the allotment in the Appendix 30B Plan;

c)	if the cancelled assignment is the result of the conversion of an allotment with modifications, reinstate the allotment with the same orbital location and technical parameters of the cancelled assignment except for its service area, which shall be the national territory of the administration whose allotment is being reinstated; and

d)	update the reference situation for the allotments of the Plan and the assignments of the List.     (WRC-15)

6.34	When a proposed new or modified frequency assignment has not fulfilled all the requirements for entering the List, in accordance with § 6.23 or 6.25, by the expiry date specified in § 6.31 or § 6.31bis in the event of an extension under that provision, the Bureau shall publish in a Special Section of the BR IFIC the cancellation of the related Special Sections.    (WRC-12)

6.35	The procedure of this Article may be applied by the administration of a country* which has joined the Union as an ITU Member State and does not have a national allotment in the Plan or an assignment in the List stemming from the conversion of an allotment in order to include new assignments in the List. Upon completion of the procedure, the next world radiocommunication conference may be requested to consider, among the assignments included in the List after the successful completion of this procedure, the inclusion in the Plan of a new allotment over the national territory of the new Member State.

6.36	Should the assignments mentioned in § 6.35 over the national territory of the administration not be brought into use within the eight years following the receipt by the Bureau of the relevant complete information under § 6.1 or within the extension period under § 6.31bis, they would be retained in the List until the end of the World Radiocommunication Conference immediately following the successful completion of the procedure referred to in § 6.35.    (WRC-12)

ARTICLE 7     (REV.WRC-15)

Procedure for the addition of a new allotment to the Plan
for a new Member State of the Union

7.1	The administration of a country** which has joined the Union as a Member State and does not have a national allotment in the Plan9 or an assignment stemming from the conversion of an allotment shall obtain a national allotment by the following procedure.      (WRC-15)

7.2	The administration shall submit its request for an allotment to the Bureau, with the following information:

a)	the geographical coordinates of not more than 20 test points for determining the minimal ellipse to cover its national territory;

b)	the height above sea level of each of its test points;

c)	any special requirement which is to be taken into account to the extent practicable.

7.3	Upon receipt of the complete information (mentioned in § 7.2 above), the Bureau shall expeditiously and ahead of submissions for which the examination under § 6.5 has not yet started, identify appropriate technical characteristics and associated orbital locations for a prospective national allotment. The Bureau shall send this information to the requesting administration.

7.4	Upon receipt of the Bureau’s response under § 7.3, the requesting administration shall, within thirty days, indicate which of the proposed orbital locations with the associated technical parameters as identified by the Bureau it has selected. During this period, the requesting administration may at any time seek the assistance of the Bureau.

7.4bis	If a selection for an allotment under § 7.4 has not been received by the Bureau within the specified time-limit, the Bureau will resume examination of submissions under § 6.5, or subsequent submission under Article 7, as appropriate, and inform the requesting administration that its request will be processed under § 7.5 when the Bureau is informed about the selected orbit location.

7.5	Upon receipt of a request under § 7.4, the Bureau shall process the request ahead of submissions for which the examination under § 6.5 has not yet started and, using Annexes 3 and 4, examine it with respect to its conformity with:

a)	the Table of Frequency Allocations and the other provisions10 of the Radio Regulations, except those provisions relating to conformity with the fixed-satellite service Plan which are the subject of the following subparagraph;

b)	allotments in the Plan;

c)	assignments which appear in the List;

d)	assignments for which the Bureau has previously received complete information and which have been examined, or are at the stage of examination under § 6.5.

7.6	When the examination under § 7.5 leads to a favorable finding, the Bureau shall enter the national allotment of the new Member State of the Union in the Plan and publish the characteristics of the allotment concerned and the result of its examination in a Special Section of the BR IFIC with the updated reference situation.

7.7	In the event that the Bureau’s findings under § 7.5 are unfavourable, the proposed allotment of the Member State shall be treated as a submission under § 6.1 and shall be treated by the Bureau ahead of any other submissions received under Article 6, except for submissions which were already under examination under § 6.5 by the Bureau at the time of completion of the examination of the request of the new Member State under § 7.5.

ARTICLE 8     (REV.WRC-15)

Procedure for notification and recording in the Master Register
of assignments in the planned bands for the
fixed-satellite service11, 12     (WRC-15)

8.1	Any assignment for which the relevant procedure of Article 6 has been successfully applied shall be notified to the Bureau using the relevant characteristics listed in Appendix 4, not earlier than three years before the assignments are brought into use.     (WRC-03)

8.2	If the first notice referred to in § 8.1 has not been received by the Bureau within the eight-year period mentioned in § 6.1 of Article 6, the assignments in the List shall no longer be taken into account by the Bureau and administrations. The Bureau shall then act as if the assignment in the List has not been brought into use in conformity with § 6.1 of Article 6. The Bureau shall inform the notifying administration, three months in advance of the end of the eight-year period, of the actions it intends to take.     (WRC-07)

8.3	Notices not containing those characteristics specified in Appendix 4 as mandatory or required shall be returned with comments to help the notifying administration to complete and resubmit them, unless the information not provided is immediately forthcoming in response to an inquiry by the Bureau.     (WRC-03)

8.4	(SUP - WRC-07)

8.5	Complete notices shall be marked by the Bureau with their date of receipt and shall be examined in the date order of their receipt. Following receipt of a complete notice the Bureau shall, within not more than two months, publish its contents, with any diagrams and maps and the date of receipt, in the BR IFIC, which shall constitute the acknowledgement to the notifying administration of receipt of its notice. When the Bureau is not in a position to comply with the time-limit referred to above, it shall periodically so inform the administrations, giving the reasons thereof.     (WRC-07)

8.6	The Bureau shall not postpone the formulation of a finding on a complete notice unless it lacks sufficient data to reach a conclusion thereon.     (WRC-03)

8.7	Each notice shall be examined:     (WRC-03)

8.8	a)	with respect to its conformity with the Table of Frequency Allocations and the other provisions13 of these Regulations, except those provisions relating to conformity with the fixed-satellite service Plan which are the subject of the following subparagraph;     (WRC-03)

8.9	b)	with respect to its conformity with the fixed-satellite service Plan and the associated provisions14.     (WRC-07)

8.10	When the examination with respect to § 8.8 leads to a favourable finding, the assignment shall be examined further with respect to § 8.9; otherwise the notice shall be returned with an indication of the appropriate action.     (WRC-03)

8.11	When the examination with respect to § 8.9 leads to a favourable finding, the assignment shall be recorded in the Master Register. When the finding is unfavourable, the notice shall be returned to the notifying administration, with an indication of the appropriate action.     (WRC-03)

8.12	In every case when a new assignment is recorded in the Master Register it shall, in accordance with the provisions of Article 8, include an indication of the finding reflecting the status of the assignment. This information shall also be published in the BR IFIC.     (WRC-03)

8.13	A notice of a change in the characteristics of an assignment already recorded, as specified in Appendix 4, shall be examined by the Bureau under § 8.8 and § 8.9, as appropriate. Any changes to the characteristics of an assignment that has been notified and confirmed as having been brought into use shall be brought into use within eight years from the date of the notification of the modification. Any changes to the characteristics of an assignment that has been notified but not yet brought into use shall be brought into use within the period provided for in §§ 6.1, 6.31 or 6.31bis of Article 6.    (WRC-12)

8.14	(SUP - WRC-07)

8.15	In applying the provisions of this Article, any resubmitted notice which is received by the Bureau more than six months after the date on which the original notice was returned by the Bureau shall be considered to be a new notice.     (WRC-03)

8.16	All frequency assignments notified in advance of their being brought into use shall be entered provisionally in the Master Register. Any frequency assignment provisionally recorded under this provision shall be brought into use no later than the end of the period provided for in § 6.1 or § 6.31bis in the event of an extension under that provision. Unless the Bureau has been informed by the notifying administration of the bringing into use of the assignment, it shall, no later than 15 days before the end of the regulatory period established under § 6.1 or § 6.31bis, send a reminder requesting confirmation that the assignment has been brought into use within the regulatory period. If the Bureau does not receive that confirmation within 30 days following the period provided under § 6.1 or §6.31bis in the event of an extension under that provision, it shall cancel the entry in the Master Register. In the event that an extension was requested under § 6.31bis but the Bureau determines that the conditions for an extension under § 6.31bis are not met, the Bureau shall inform the administration of its findings and cancel the entry in the Master Register.    (WRC-12)

8.17	Wherever the use of a recorded frequency assignment to a space station is suspended for a period exceeding six months, the notifying administration shall inform the Bureau of the date on which such use was suspended. When the recorded assignment is brought back into use, the notifying administration shall so inform the Bureau, as soon as possible. On receipt of the information sent under this provision, the Bureau shall make that information available on the ITU website as soon as possible and shall publish it in the BR IFIC. The date on which the assignment is brought back into use14bis shall be no later than three years from the date on which the use of the frequency assignment was suspended, provided that the notifying administration informs the Bureau of the suspension within six months from the date on which the use was suspended. If the notifying administration informs the Bureau of the suspension more than six months after the date on which the use of the frequency assignment was suspended, this three-year time period shall be reduced. In this case, the amount by which the three-year period shall be reduced shall be equal to the amount of time that has elapsed between the end of the six-month period and the date that the Bureau is informed of the suspension. If the notifying administration informs the Bureau of the suspension more than 21 months after the date on which the use of the frequency assignment was suspended, the frequency assignment shall be cancelled from the Master Register and the Bureau shall apply the provisions of § 6.33.     (WRC-15)

8.18	No provision of this Appendix shall be considered as modifying the requirements of Article 9 relating to coordination between earth stations in the fixed-satellite service and stations of terrestrial services sharing the planned bands on an equal primary basis.     (WRC-03)

8.19	Notification of assignments to a specific earth station using assignments included in the List shall be effected applying the provisions of Article 11.     (WRC-03)

ARTICLE 9     (REV.WRC-07)

General provisions

9.1	The Plan is limited to national systems providing a domestic service. Administrations may, however, in accordance with the provisions of Article 6, convert their allotments or propose additional systems to provide national or multinational services.

9.2	(SUP - WRC-07)

ARTICLE 10     (REV.WRC-15)

Plan for the fixed-satellite service in the frequency bands
4 500-4 800 MHz, 6 725-7 025 MHz, 10.70-10.95 GHz,
11.20-11.45 GHz and 12.75-13.25 GHz

A.1		COLUMN HEADINGS OF THE PLAN

Col. 2	Nominal orbital position, in degrees

Col. 3	Longitude of the boresight, in degrees

Col. 4	Latitude of the boresight, in degrees

Col. 5	Major axis of the elliptical cross-section half-power beam, in degrees

Col. 6	Minor axis of the elliptical cross-section half-power beam, in degrees

Col. 7	Orientation of the ellipse determined as follows: in a plane normal to the beam axis, the direction of the major axis of the ellipse is defined by the angle measured anticlockwise from a line parallel to the equatorial plane to the major axis of the ellipse, to the nearest degree

Col. 8	Earth station e.i.r.p. density (dB(W/Hz))

Col. 9	Satellite e.i.r.p. density (dB(W/Hz))

Col. 10	Remarks

1	Assignment converted from allotment.

2	The Administration of Luxembourg (LUX) agreed to operate the LUX-30B-6 satellite network within the characteristics included in the Appendix 30B List, as modified during WRC-07, and to immediately eliminate interference that could be caused by LUX-30B-6 to the national allotment of the Islamic Republic of Iran (IRN00000) (IRN).

3	Allotment converted into assignment with a shaped beam and then reinstated back into the Plan.

4-5	(SUP - WRC-07)

6	Allotment reinstated from assignments which were provisionally entered in the List in accordance with § 6.25. §§ 6.26 to 6.29 apply.     (WRC-15)

Note by the Secretariat (applicable when an asterisk (*) appears in column 10): It is to be noted that this beam is intended to be implemented as part of a multi-beam network, operating from a single orbital location. Within any multi-beam network, the beams are the responsibility of a single administration, hence interference between them has not been taken into account during the Conference. The number which appears in the alphanumeric code that follows the asterisk serves to identify the multi-beam network concerned.

| 4 500-4 800 MHz, 6 725-7 025 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| ABW00000 | −98.20 | −69.10 | 12.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| ADL00000 | 113.00 | 140.00 | −66.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 | */MB1 |
| AFG00000 | 50.00 | 66.40 | 33.90 | 2.20 | 1.60 | 15.00 | −9.6 | −39.4 |  |
| AFS00000 | 71.00 | 27.20 | −30.10 | 5.30 | 1.60 | 128.00 | −7.8 | −38.6 |  |
| AGL00000 | −36.10 | 15.90 | −12.40 | 2.40 | 1.60 | 78.00 | −9.6 | −39.1 |  |
| ALB00000 | 4.13 | 20.00 | 41.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| ALG00000 | −33.50 | 1.60 | 27.80 | 3.30 | 2.20 | 133.00 | −8.6 | −38.9 |  |
| ALS00000 | −159.00 | −158.60 | 57.50 | 6.30 | 1.60 | 1.00 | −7.9 | −38.8 | */MB2 |
| AND00000 | −41.00 | 1.50 | 42.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| ARG00000 | −51.00 | −62.00 | −33.60 | 4.80 | 2.90 | 93.00 | −2.5 | −38.1 | */MB3 |
| ARGINSUL | −51.00 | −60.00 | −57.50 | 3.60 | 1.60 | 154.00 | −9.6 | −38.5 | */MB3 |
| ARM00000 | 71.40 | 45.13 | 40.12 | 1.60 | 1.60 | 90.00 | −9.6 | −40.4 |  |
| ARS00000 | 51.90 | 45.70 | 23.10 | 3.70 | 2.60 | 153.00 | −8.7 | −39.3 |  |
| ASCSTHTC | −37.10 | −11.80 | −19.60 | 5.60 | 1.80 | 77.00 | −8.0 | −39.0 | */MB4 |
| ATG00000 | −77.70 | −61.80 | 17.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| AUS00001 | 144.10 | 134.30 | −24.50 | 6.60 | 5.30 | 146.00 | 1.9 | −38.2 | */MB6 |
| AUS00002 | 144.10 | 163.60 | −30.50 | 1.60 | 1.60 | 90.00 | −9.6 | −39.5 | */MB6 |
| AUS00003 | 144.10 | 101.50 | −11.10 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 | */MB6 |
| AUS00004 | 144.10 | 159.00 | −54.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 | */MB6 |
| AUS00005 | 144.10 | 110.40 | −66.30 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 | */MB6 |
| AUT00000 | −11.40 | 13.20 | 47.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.8 |  |
| AZE00000 | 95.90 | 47.20 | 40.34 | 1.60 | 1.60 | 0.00 | −9.6 | −42.2 |  |
| AZR00000 | −10.60 | −28.00 | 38.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.1 | */MB7 |
| B 00001 | −66.25 | −62.60 | −6.00 | 4.10 | 4.00 | 43.00 | −2.5 | −38.7 |  |
| B 00002 | −63.60 | −45.40 | −6.30 | 4.60 | 4.10 | 152.00 | −1.9 | −38.6 |  |
| B 00003 | −69.45 | −50.00 | −20.90 | 4.30 | 3.00 | 60.00 | −3.4 | −38.5 |  |
| BAH00000 | −74.30 | −75.80 | 24.00 | 1.60 | 1.60 | 133.00 | −9.6 | −39.4 |  |
| BDI00000 | −3.50 | 29.90 | −3.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| BEL00000 | 54.55 | 5.20 | 50.60 | 1.60 | 1.60 | 90.00 | −9.6 | −41.2 |  |
| BEN00000 | −30.60 | 2.30 | 9.30 | 1.60 | 1.60 | 90.00 | −9.6 | −39.9 |  |
| BERCAYS | −37.10 | −68.60 | 22.50 | 3.70 | 2.30 | 41.00 | −5.6 | −38.2 | */MB4 |
| BFA00000 | 10.79 | −1.40 | 12.20 | 1.70 | 1.60 | 24.00 | −9.6 | −39.5 |  |
| BGD00000 | 133.00 | 90.20 | 24.00 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 |  |
| BHR00000 | 13.60 | 50.60 | 26.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 |  |
| BLR00000 | 64.40 | 27.01 | 53.60 | 1.60 | 1.60 | 0.00 | −9.4 | −41.3 |  |
| BLZ00000 | −90.80 | −88.60 | 17.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| BOL00000 | −34.80 | −64.40 | −17.10 | 2.70 | 1.70 | 129.00 | −7.5 | −38.6 |  |
| BOT00000 | 21.20 | 24.00 | −21.80 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| BRB00000 | −29.60 | −59.60 | 13.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| BRM00000 | 111.50 | 97.00 | 18.90 | 3.20 | 1.60 | 88.00 | −7.2 | −38.8 |  |
| BRU00000 | 157.30 | 114.60 | 4.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 |  |
| BTN00000 | 59.10 | 90.40 | 27.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.5 |  |
| BUL00000 | 56.02 | 25.60 | 42.80 | 1.60 | 1.60 | 90.00 | −9.6 | −40.8 |  |
| CAF00000 | 14.40 | 21.50 | 6.50 | 2.70 | 1.70 | 14.00 | −8.4 | −39.1 |  |
| CAN0CENT | −111.10 | −96.10 | 51.40 | 4.30 | 2.00 | 155.00 | −7.6 | −38.4 |  |
| CAN0EAST | −107.30 | −76.60 | 50.10 | 5.00 | 1.70 | 154.00 | −7.0 | −38.3 |  |
| CAN0WEST | −114.90 | −120.10 | 57.40 | 3.10 | 1.90 | 173.00 | −9.6 | −38.7 |  |
| CBG00000 | 96.10 | 105.10 | 12.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.4 |  |
| CHL00000 | −74.90 | −82.60 | −32.80 | 8.10 | 6.10 | 155.00 | −0.7 | −38.4 |  |
| CHN00001 | 101.40 | 103.70 | 35.00 | 8.10 | 4.30 | 2.00 | −0.1 | −38.3 |  |
| CHN00002 | 135.50 | 114.80 | 16.40 | 4.90 | 2.40 | 65.00 | −3.6 | −38.7 |  |
| CLM00000 | −70.90 | −74.00 | 5.70 | 4.00 | 2.30 | 121.00 | −5.1 | −38.9 |  |
| CLN00000 | 121.50 | 80.10 | 7.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.2 |  |
| CME00000 | 7.98 | 12.90 | 6.30 | 2.50 | 1.90 | 84.00 | −8.4 | −39.5 |  |
| CNR00000 | −30.00 | −15.90 | 28.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 | */MB8 |
| COD00000 | 50.95 | 24.40 | −4.60 | 3.90 | 3.50 | 92.00 | −7.4 | −38.5 |  |
| COG00000 | −16.35 | 14.80 | −0.60 | 2.00 | 1.60 | 63.00 | −9.1 | −38.8 |  |
| COM00000 | 94.50 | 44.10 | −12.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.0 |  |
| CPV00000 | −85.70 | −24.10 | 16.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| CTI00000 | −15.76 | −5.90 | 7.80 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| CTR00000 | −96.00 | −85.30 | 8.20 | 1.60 | 1.60 | 90.00 | −9.6 | −40.2 |  |
| CUB00000 | −80.60 | −79.50 | 21.00 | 2.00 | 1.60 | 172.00 | −9.6 | −39.3 |  |
| CVA00000 | 59.00 | 12.50 | 41.90 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| CYP00000 | 0.50 | 33.20 | 35.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| CYPSBA00 | 57.50 | 32.90 | 34.60 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 | */MB9 |
| CZE00000 | −31.90 | 15.68 | 49.81 | 1.60 | 1.60 | 0.00 | −9.6 | −41.3 |  |
| D 00001 | 26.40 | 9.70 | 50.70 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 |  |
| D 00002 | 37.20 | 12.60 | 51.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.8 |  |
| DJI00000 | −17.46 | 42.60 | 11.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| DMA00000 | −70.00 | −61.30 | 15.30 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| DNK00001 | 32.28 | 11.60 | 56.00 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 |  |
| DNK00002 | −49.00 | 12.50 | 56.30 | 1.60 | 1.60 | 90.00 | −9.6 | −40.6 | */MB10 |
| DNK00FAR | −49.00 | −7.20 | 61.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.1 | */MB10 |
| DOM00000 | −85.40 | −70.40 | 18.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 |  |
| E 00002 | −30.00 | −3.00 | 39.90 | 2.10 | 1.60 | 8.00 | −9.6 | −39.5 | */MB8 |
| EGY00000 | 67.11 | 30.30 | 26.20 | 2.30 | 1.60 | 54.00 | −9.6 | −39.2 |  |
| EQA00000 | −104.00 | −83.10 | −1.40 | 3.10 | 1.60 | 174.00 | −7.8 | −38.9 |  |
| ETH00000 | 58.30 | 40.60 | 10.30 | 2.80 | 2.80 | 64.00 | −9.4 | −39.4 |  |
| F 00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| FIN00000 | 46.80 | 23.80 | 64.30 | 1.60 | 1.60 | 90.00 | −9.6 | −39.3 |  |
| FJI00000 | 148.80 | 178.50 | −17.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.5 |  |
| FLKSTGGL | −37.10 | −46.80 | −59.60 | 3.70 | 1.60 | 170.00 | −9.6 | −38.8 | */MB4 |
| G 00000 | −37.10 | −4.10 | 53.90 | 1.60 | 1.60 | 151.00 | −9.6 | −39.0 | */MB4 |
| GAB00000 | 39.00 | 11.70 | −0.70 | 1.60 | 1.60 | 90.00 | −9.6 | −39.8 |  |
| GDL00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| GDL00002 | −115.90 | −61.80 | 16.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 | */MB13 |
| GHA00000 | 15.90 | −1.30 | 7.70 | 1.60 | 1.60 | 90.00 | −9.6 | −39.7 |  |
| GIB00000 | 57.50 | −5.40 | 36.10 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 | */MB9 |
| GMB00000 | −34.00 | −16.40 | 13.40 | 1.60 | 1.60 | 90.00 | −9.6 | −42.1 |  |
| GNB00000 | 40.00 | −15.40 | 12.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| GNE00000 | −32.30 | 10.50 | 1.70 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 |  |
| GRC00000 | 22.05 | 24.70 | 38.30 | 1.70 | 1.60 | 160.00 | −9.6 | −39.3 |  |
| GRD00000 | −32.80 | −61.60 | 12.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| GRL00000 | −49.00 | −42.90 | 68.60 | 2.30 | 1.60 | 174.00 | −9.6 | −38.6 | */MB10 |
| GTM00000 | −135.70 | −90.50 | 15.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 |  |
| GUF00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| GUF00002 | −115.90 | −53.30 | 4.30 | 1.60 | 1.60 | 90.00 | −8.6 | −39.4 | */MB13 |
| GUI00000 | 27.50 | −10.90 | 10.20 | 1.60 | 1.60 | 90.00 | −9.6 | −39.2 |  |
| GUMMRA0 | −159.00 | 145.40 | 16.70 | 1.70 | 1.60 | 79.00 | −9.4 | −38.3 | */MB2 |
| GUY00000 | −23.80 | −59.20 | 4.70 | 1.60 | 1.60 | 90.00 | −9.6 | −39.4 |  |
| HKG00000 | 57.50 | 114.50 | 22.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.6 |  |
| HND00000 | −76.20 | −86.10 | 15.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| HNG00000 | −7.50 | 19.40 | 47.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.0 |  |
| HOL00000 | −5.00 | 5.40 | 52.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 | */MB5 |
| HTI00000 | −92.00 | −73.00 | 18.80 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 |  |
| HWA00000 | −159.00 | −157.60 | 20.70 | 1.60 | 1.60 | 90.00 | −9.6 | −40.2 | */MB2 |
| HWL00000 | −159.00 | −176.60 | 0.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 | */MB2 |
| I 00000 | −23.40 | 11.30 | 40.90 | 2.10 | 1.60 | 141.00 | −9.6 | −38.9 |  |
| IND00000 | 74.00 | 82.70 | 18.90 | 6.20 | 4.90 | 120.00 | 0.3 | −38.5 |  |
| INS00000 | 115.40 | 117.60 | −1.80 | 9.40 | 4.30 | 170.00 | 1.8 | −38.6 |  |
| IRL00000 | −21.80 | −8.20 | 53.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.1 |  |
| IRN00000 | 24.19 | 54.30 | 33.00 | 3.70 | 1.60 | 143.00 | −9.6 | −39.0 |  |
| IRQ00000 | 65.45 | 44.30 | 33.10 | 1.60 | 1.60 | 90.00 | −9.6 | −39.4 |  |
| ISL00000 | −35.20 | −18.20 | 64.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 |  |
| ISR00000 | −4.00 |  |  |  |  |  |  |  | 1 |
| J 00000 | 152.50 | 140.40 | 30.40 | 5.70 | 3.70 | 15.00 | −2.3 | −38.5 |  |
| JAR00000 | −159.00 | −160.00 | −0.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 | */MB2 |
| JMC00000 | −108.60 | −77.60 | 18.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.5 |  |
| JON00000 | −159.00 | −168.50 | 17.00 | 1.60 | 1.60 | 90.00 | −9.6 | −42.2 | */MB2 |
| JOR00000 | 81.76 | 36.70 | 31.30 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 |  |
| KAZ00000 | 58.50 | 66.36 | 46.72 | 4.60 | 1.69 | 176.88 | −9.6 | −41.0 |  |
| KEN00000 | 78.20 | 38.40 | 0.80 | 2.10 | 1.60 | 95.00 | −9.6 | −39.3 |  |
| KER00000 | 113.00 | 69.30 | −43.90 | 1.90 | 1.60 | 169.00 | −9.6 | −38.7 | */MB1 |
| KGZ00000 | 64.60 | 74.54 | 41.15 | 1.60 | 1.60 | 90.00 | −9.6 | −38.8 |  |
| KIR00000 | 150.00 | 173.00 | 1.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| KNA00000 | −88.80 | −62.90 | 17.30 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| KOR00000 | 116.20 | 127.70 | 36.20 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 |  |
| KRE00000 | 145.00 | 127.80 | 39.80 | 1.60 | 1.60 | 90.00 | −9.6 | −39.6 |  |
| KWT00000 | 30.90 | 47.70 | 29.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 |  |
| LAO00000 | 142.00 | 104.10 | 18.10 | 1.60 | 1.60 | 90.00 | −9.6 | −39.1 |  |
| LBN00000 | 97.50 | 35.80 | 33.80 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| LBR00000 | −41.80 | −8.90 | 6.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.4 |  |
| LBY00000 | 28.90 | 19.00 | 25.90 | 3.00 | 2.70 | 165.00 | −6.8 | −39.2 |  |
| LIE00000 | −17.10 | 9.50 | 47.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 |  |
| LSO00000 | −19.30 | 28.40 | −29.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.5 |  |
| LTU00000 | −9.30 | 23.67 | 55.23 | 1.60 | 1.60 | 0.00 | −9.6 | −42.8 |  |
| LUX00000 | 19.20 | 6.20 | 49.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| MAC00000 | 117.00 | 113.60 | 22.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| MAU00000 | 92.20 | 57.50 | −20.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| MCO00000 | 52.00 |  |  |  |  |  | −15.6 | −28.7 | 3, 6 |
| MDG00000 | 16.90 | 46.60 | −18.70 | 2.60 | 1.60 | 66.00 | −7.5 | −38.6 |  |
| MDR00000 | −10.60 | −16.20 | 31.60 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 | */MB7 |
| MDW00000 | −159.00 | −177.40 | 28.20 | 1.60 | 1.60 | 90.00 | −9.6 | −42.0 | */MB2 |
| MEX00000 | −113.00 | −103.60 | 23.30 | 5.80 | 2.40 | 161.00 | −4.7 | −38.8 |  |
| MHL00000 | −159.00 | 175.30 | 8.70 | 2.30 | 1.60 | 94.00 | −8.6 | −38.8 | */MB2 |
| MLA00000 | 78.50 | 108.20 | 4.70 | 3.20 | 1.60 | 0.00 | −6.3 | −38.5 |  |
| MLD00000 | 117.60 | 73.40 | 2.50 | 2.20 | 1.60 | 88.00 | −9.6 | −38.7 |  |
| MLI00000 | −6.00 | −3.90 | 17.60 | 3.30 | 2.50 | 21.00 | −7.6 | −39.2 |  |
| MLT00000 | −3.00 | 14.40 | 35.90 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| MNG00000 | 113.60 | 103.80 | 46.80 | 3.60 | 1.60 | 3.00 | −9.6 | −38.9 |  |
| MOZ00000 | 90.60 | 35.60 | −17.20 | 3.10 | 1.60 | 98.00 | −7.7 | −38.3 |  |
| MRC00000 | 32.86 | −8.90 | 27.90 | 3.40 | 1.60 | 45.00 | −9.6 | −38.8 |  |
| MTN00000 | −21.10 | −10.30 | 19.80 | 2.50 | 2.40 | 76.00 | −9.6 | −39.4 |  |
| MWI00000 | 28.00 | 34.10 | −13.30 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| MYT00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| NCG00000 | −84.40 | −84.90 | 12.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.6 |  |
| NCL00000 | 113.00 | 165.80 | −21.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.6 | */MB1 |
| NGR00000 | −38.50 | 7.50 | 17.20 | 2.10 | 1.70 | 100.00 | −9.6 | −38.9 |  |
| NIG00000 | 41.82 | 8.00 | 9.90 | 2.50 | 1.60 | 47.00 | −7.7 | −38.5 |  |
| NMB00000 | 12.20 | 18.50 | −21.00 | 2.70 | 2.60 | 155.00 | −9.6 | −39.5 |  |
| NOR00000 | −0.80 | 11.70 | 64.60 | 2.00 | 1.60 | 17.00 | −9.6 | −38.7 |  |
| NPL00000 | 123.30 | 84.40 | 28.00 | 1.60 | 1.60 | 90.00 | −9.6 | −40.8 |  |
| NRU00000 | 146.00 | 166.90 | −0.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| NZL00001 | 152.00 | 170.90 | −44.80 | 5.40 | 1.60 | 49.00 | −7.4 | −38.1 | */MB14 |
| NZL00002 | 152.00 | −165.40 | −13.20 | 2.70 | 2.00 | 82.00 | −7.3 | −38.3 | */MB14 |
| OCE00000 | −115.90 | −141.90 | −16.10 | 3.50 | 2.40 | 139.00 | −7.1 | −38.9 | */MB13 |
| OMA00000 | 104.00 | 55.10 | 21.60 | 1.90 | 1.60 | 61.00 | −9.6 | −39.2 |  |
| PAK00000 | 56.50 | 69.90 | 29.80 | 3.00 | 2.00 | 22.00 | −9.3 | −39.0 |  |
| PHL00000 | 161.00 | 122.23 | 11.37 | 3.33 | 1.60 | 79.65 | −6.3 | −38.4 |  |
| PLM00000 | −159.00 | −161.40 | 7.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 | */MB2 |
| PNG00000 | 154.10 | 148.40 | −6.60 | 3.30 | 2.30 | 167.00 | −6.2 | −39.0 |  |
| PNR00000 | −79.20 | −80.20 | 8.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.4 |  |
| POL00000 | 15.20 | 19.30 | 52.00 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| POR00000 | −10.60 | −8.00 | 39.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.2 | */MB7 |
| PRG00000 | −81.50 | −58.70 | −23.10 | 1.60 | 1.60 | 90.00 | −9.6 | −39.1 |  |
| PRU00000 | −89.90 | −74.20 | −8.40 | 3.60 | 2.40 | 111.00 | −5.4 | −38.7 |  |
| PTC00000 | −62.30 | −130.10 | −25.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.2 |  |
| QAT00000 | 0.90 | 51.60 | 25.40 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| REU00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| REU00002 | 113.00 | 55.60 | −21.10 | 1.60 | 1.60 | 90.00 | −9.6 | −40.6 | */MB1 |
| ROU00000 | 30.45 | 25.00 | 46.30 | 1.60 | 1.60 | 90.00 | −9.6 | −39.6 |  |
| RRW00000 | 17.60 | 29.70 | −1.90 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 |  |
| RUS00001 | 61.00 | 51.50 | 52.99 | 5.56 | 2.01 | 10.74 | −7.2 | −38.3 |  |
| RUS00002 | 88.10 | 94.80 | 48.60 | 7.50 | 3.50 | 175.00 | −1.4 | −38.3 |  |
| RUS00003 | 138.50 | 138.14 | 53.83 | 5.86 | 2.09 | 8.41 | −6.7 | −38.2 |  |
| S 00000 | 5.00 | 16.70 | 60.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.2 |  |
| SDN00001 | 23.55 |  |  |  |  |  |  |  | 1 |
| SDN00002 | 23.55 |  |  |  |  |  |  |  | 1 |
| SEN00000 | −48.40 | −14.00 | 14.10 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 |  |
| SEY00000 | 42.25 | 51.50 | −3.20 | 13.80 | 3.80 | 48.50 | −3.0 | −43.8 |  |
| SLM00000 | 147.50 | 159.00 | −9.10 | 1.60 | 1.60 | 90.00 | −9.6 | −39.5 |  |
| SLV00000 | −130.50 | −89.00 | 13.70 | 1.60 | 1.60 | 90.00 | −9.6 | −40.9 |  |
| SMA00000 | −159.00 | −170.70 | −14.20 | 1.60 | 1.60 | 90.00 | −9.6 | −42.2 | */MB2 |
| SMO00000 | −125.50 | −172.10 | −13.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.1 |  |
| SMR00000 | 16.50 | 12.50 | 43.90 | 1.60 | 1.60 | 90.00 | −9.6 | −42.0 |  |
| SNG00000 | 98.10 | 103.90 | 1.30 | 1.60 | 1.60 | 90.00 | −9.6 | −41.6 |  |
| SOM00000 | 98.40 | 46.00 | 6.30 | 3.10 | 1.60 | 72.00 | −9.6 | −38.8 |  |
| SPM00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| SRL00000 | −51.80 | −11.90 | 8.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| STP00000 | 30.25 | 7.00 | 1.00 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 |  |
| SUI00000 | 9.45 | 8.20 | 46.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.3 |  |
| SUR00000 | −77.00 | −55.60 | 3.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.7 |  |
| SVK00000 | −19.82 | 17.30 | 49.60 | 1.60 | 1.60 | 90.00 | −9.6 | −40.0 |  |
| SWZ00000 | 30.10 | 31.30 | −26.40 | 1.60 | 1.60 | 90.00 | −9.6 | −42.0 |  |
| SYR00000 | 18.00 | 38.60 | 35.30 | 1.60 | 1.60 | 90.00 | −9.6 | −40.8 |  |
| TCD00000 | −9.90 | 18.40 | 15.60 | 3.50 | 1.60 | 97.00 | −8.9 | −39.0 |  |
| TGO00000 | −23.15 | 0.80 | 8.60 | 1.60 | 1.60 | 90.00 | −9.6 | −40.4 |  |
| THA00000 | 120.60 | 100.90 | 12.80 | 2.80 | 1.60 | 83.00 | −7.7 | −38.8 |  |
| TON00000 | −128.00 | −175.20 | −21.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.0 |  |
| TRD00000 | −73.40 | −61.10 | 10.80 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| TUN00000 | 5.74 | 9.40 | 33.50 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 |  |
| TUR00000 | 8.50 | 34.10 | 38.90 | 2.80 | 1.60 | 171.00 | −6.4 | −38.6 |  |
| TUV00000 | 158.00 | 179.20 | −8.50 | 1.60 | 1.60 | 90.00 | −9.6 | −41.8 |  |
| TZA00000 | 67.50 | 35.40 | −5.90 | 2.40 | 1.60 | 117.00 | −9.6 | −39.3 |  |
| UAE00000 | 63.50 | 53.80 | 24.90 | 1.60 | 1.60 | 90.00 | −9.6 | −41.1 |  |
| UGA00000 | 31.50 | 32.20 | 0.90 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 |  |
| UKR00001 | 38.20 | 31.73 | 48.22 | 1.98 | 1.60 | 178.15 | −15.1 | −40.7 |  |
| URG00000 | −86.10 | −56.30 | −33.70 | 1.60 | 1.60 | 90.00 | −9.6 | −40.7 |  |
| USA00000 | −101.00 | −93.90 | 36.80 | 8.20 | 3.60 | 172.00 | −0.9 | −38.3 | */MB16 |
| USAVIPRT | −101.00 | −64.50 | 17.80 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 | */MB16 |
| UZB00000 | 110.50 | 65.45 | 41.09 | 1.60 | 1.60 | 0.00 | −9.6 | −40.3 |  |
| VCT00000 | −93.10 | −61.10 | 13.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.5 |  |
| VEN00001 | −82.70 | −66.40 | 6.80 | 2.80 | 2.10 | 142.00 | −7.0 | −38.9 | */MB17 |
| VEN00002 | −82.70 | −63.60 | 15.70 | 1.60 | 1.60 | 90.00 | −9.6 | −41.7 | */MB17 |
| VTN00000 | 107.00 | 16.00 |  |  |  |  | −7.1 | −35.8 | 3 |
| VUT00000 | 150.70 | 168.40 | −17.20 | 1.60 | 1.60 | 90.00 | −9.6 | −40.3 |  |
| WAK00000 | −159.00 | 166.50 | 19.20 | 1.60 | 1.60 | 90.00 | −9.6 | −41.9 | */MB2 |
| WAL00000 | 113.00 | −177.10 | −13.80 | 1.60 | 1.60 | 90.00 | −9.0 | −39.8 | */MB1 |
| XAN00000 | −5.00 | −65.60 | 15.10 | 1.60 | 1.60 | 90.00 | −9.6 | −38.9 | */MB5 |
| XCQ00000 | −159.00 | 173.40 | 4.60 | 10.20 | 2.40 | 175.00 | 4.5 | −35.6 | */MB2 |
| XYU00000 | 43.04 | 18.70 | 44.40 | 1.60 | 1.60 | 90.00 | −9.6 | −40.5 |  |
| YEM00001 | 27.00 | 44.20 | 15.10 | 1.60 | 1.60 | 90.00 | −9.6 | −41.4 |  |
| YEM00002 | 108.00 | 49.90 | 14.80 | 1.60 | 1.60 | 90.00 | −9.6 | −39.7 |  |
| ZMB00000 | 39.55 | 27.90 | −12.80 | 2.40 | 1.60 | 26.00 | −9.6 | −39.6 |  |
| ZWE00000 | 65.60 | 30.00 | −18.90 | 1.60 | 1.60 | 90.00 | −9.6 | −39.9 |  |

| 10.70-10.95 GHz, 11.20-11.45 GHz, 12.75-13.25 GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| ABW00000 | −98.20 | −69.10 | 12.40 | 0.80 | 0.80 | 90.00 | −6.4 | −25.8 |  |
| ADL00000 | 113.00 | 140.00 | −66.70 | 0.80 | 0.80 | 90.00 | −10.2 | −31.9 | */MB1 |
| AFG00000 | 50.00 | 66.40 | 33.90 | 2.20 | 1.30 | 15.00 | −4.1 | −29.2 |  |
| AFS00000 | 71.00 | 27.20 | −30.10 | 5.30 | 1.40 | 128.00 | 3.3 | −26.7 |  |
| AGL00000 | −36.10 | 15.90 | −12.40 | 2.40 | 1.40 | 78.00 | 1.1 | −25.8 |  |
| ALB00000 | 4.13 | 20.00 | 41.10 | 0.80 | 0.80 | 90.00 | −8.6 | −28.2 |  |
| ALG00000 | −33.50 | 1.60 | 27.80 | 3.30 | 2.20 | 133.00 | 3.4 | −26.6 |  |
| ALS00000 | −159.00 | −158.60 | 57.50 | 6.30 | 1.50 | 1.00 | 1.6 | −28.7 | */MB2 |
| AND00000 | −41.00 | 1.50 | 42.50 | 0.80 | 0.80 | 90.00 | −10.2 | −30.0 |  |
| ARG00000 | −51.00 | −62.00 | −33.60 | 4.80 | 2.90 | 93.00 | 9.4 | −21.9 | */MB3 |
| ARGINSUL | −51.00 | −60.00 | −57.50 | 3.60 | 1.30 | 154.00 | −1.4 | −28.6 | */MB3 |
| ARM00000 | 71.40 | 45.13 | 40.12 | 0.80 | 0.80 | 90.00 | −10.2 | −30.1 |  |
| ARS00000 | 51.90 | 45.70 | 23.10 | 3.70 | 2.60 | 153.00 | 0.8 | −29.4 |  |
| ASCSTHTC | −37.10 | −11.80 | −19.60 | 5.60 | 1.80 | 77.00 | 2.1 | −28.6 | */MB4 |
| ATG00000 | −77.70 | −61.80 | 17.00 | 0.80 | 0.80 | 90.00 | −7.2 | −27.1 |  |
| AUS00001 | 144.10 | 134.30 | −24.50 | 6.60 | 5.30 | 146.00 | 13.4 | −22.1 | */MB6 |
| AUS00002 | 144.10 | 163.60 | −30.50 | 1.60 | 1.00 | 15.00 | −2.9 | −26.5 | */MB6 |
| AUS00003 | 144.10 | 101.50 | −11.10 | 1.10 | 1.00 | 15.00 | −6.9 | −28.5 | */MB6 |
| AUS00004 | 144.10 | 159.00 | −54.50 | 0.80 | 0.80 | 90.00 | −10.2 | −32.3 | */MB6 |
| AUS00005 | 144.10 | 110.40 | −66.30 | 0.80 | 0.80 | 90.00 | −10.2 | −31.8 | */MB6 |
| AUT00000 | −11.40 | 13.20 | 47.50 | 0.80 | 0.80 | 90.00 | −8.1 | −27.2 |  |
| AZE00000 | 95.90 | 47.20 | 40.34 | 0.80 | 0.80 | 0.00 | −10.2 | −31.0 |  |
| AZR00000 | −10.60 | −28.00 | 38.70 | 0.80 | 0.80 | 90.00 | −8.7 | −27.9 | */MB7 |
| B 00001 | −66.25 | −62.60 | −6.00 | 4.10 | 4.00 | 43.00 | 9.8 | −22.4 |  |
| B 00002 | −63.60 | −45.40 | −6.30 | 4.60 | 4.10 | 152.00 | 10.4 | −22.4 |  |
| B 00003 | −69.45 | −50.00 | −20.90 | 4.30 | 3.00 | 60.00 | 8.9 | −22.2 |  |
| BAH00000 | −74.30 | −75.80 | 24.00 | 1.60 | 1.00 | 133.00 | −0.8 | −24.5 |  |
| BDI00000 | −3.50 | 29.90 | −3.40 | 0.80 | 0.80 | 90.00 | −10.2 | −29.9 |  |
| BEL00000 | 54.55 | 5.20 | 50.60 | 0.80 | 0.80 | 90.00 | −10.2 | −30.2 |  |
| BEN00000 | −30.60 | 2.30 | 9.30 | 1.20 | 1.00 | 89.00 | −2.1 | −23.0 |  |
| BERCAYS | −37.10 | −68.60 | 22.50 | 3.70 | 2.30 | 41.00 | 7.4 | −21.8 | */MB4 |
| BFA00000 | 10.79 | −1.40 | 12.20 | 1.70 | 1.00 | 24.00 | −0.6 | −25.0 |  |
| BGD00000 | 133.00 | 90.20 | 24.00 | 0.80 | 0.80 | 90.00 | −3.9 | −21.9 |  |
| BHR00000 | 13.60 | 50.60 | 26.10 | 0.80 | 0.80 | 90.00 | −10.2 | −32.2 |  |
| BLR00000 | 64.40 | 27.01 | 53.60 | 1.14 | 0.80 | 25.74 | −3.0 | −30.0 |  |
| BLZ00000 | −90.80 | −88.60 | 17.20 | 0.80 | 0.80 | 90.00 | −6.5 | −26.6 |  |
| BOL00000 | −34.80 | −64.40 | −17.10 | 2.70 | 1.70 | 129.00 | 4.3 | −22.5 |  |
| BOT00000 | 21.20 | 24.00 | −21.80 | 1.50 | 1.50 | 94.00 | −6.0 | −30.0 |  |
| BRB00000 | −29.60 | −59.60 | 13.20 | 0.80 | 0.80 | 90.00 | −7.0 | −26.4 |  |
| BRM00000 | 111.50 | 97.00 | 18.90 | 3.20 | 1.60 | 88.00 | 4.6 | −22.6 |  |
| BRU00000 | 157.30 | 114.60 | 4.50 | 0.80 | 0.80 | 90.00 | −6.9 | −24.9 |  |
| BTN00000 | 59.10 | 90.40 | 27.00 | 0.80 | 0.80 | 90.00 | −10.2 | −29.3 |  |
| BUL00000 | 56.02 | 25.60 | 42.80 | 0.80 | 0.80 | 90.00 | −7.8 | −27.0 |  |
| CAF00000 | 14.40 | 21.50 | 6.50 | 2.70 | 1.70 | 14.00 | 3.8 | −22.8 |  |
| CAN0CENT | −111.10 | −96.10 | 51.40 | 4.30 | 2.00 | 155.00 | 3.9 | −26.7 |  |
| CAN0EAST | −107.30 | −76.60 | 50.10 | 5.00 | 1.70 | 154.00 | 6.2 | −25.0 |  |
| CAN0WEST | −114.90 | −120.10 | 57.40 | 3.10 | 1.90 | 173.00 | −0.6 | −28.7 |  |
| CBG00000 | 96.10 | 105.10 | 12.90 | 1.20 | 1.00 | 35.00 | −2.5 | −23.2 |  |
| CHL00000 | −74.90 | −82.60 | −32.80 | 8.10 | 6.10 | 155.00 | 9.0 | −28.4 |  |
| CHN00001 | 101.40 | 103.70 | 35.00 | 8.10 | 4.30 | 2.00 | 13.6 | −23.2 |  |
| CHN00002 | 135.50 | 114.80 | 16.40 | 4.90 | 2.40 | 65.00 | 8.2 | −22.5 |  |
| CLM00000 | −70.90 | −74.00 | 5.70 | 4.00 | 2.30 | 121.00 | 7.1 | −22.6 |  |
| CLN00000 | 121.50 | 80.10 | 7.70 | 0.80 | 0.80 | 90.00 | −6.5 | −24.8 |  |
| CME00000 | 7.98 | 12.90 | 6.30 | 2.50 | 1.90 | 84.00 | 3.9 | −22.7 |  |
| CNR00000 | −30.00 |  |  |  |  |  |  |  | 1 |
| COD00000 | 50.95 | 24.40 | −4.60 | 3.90 | 3.50 | 92.00 | 6.5 | −24.4 |  |
| COG00000 | −16.35 | 14.80 | −0.60 | 2.00 | 1.10 | 63.00 | 0.7 | −22.7 |  |
| COM00000 | 94.50 | 44.10 | −12.20 | 0.80 | 0.80 | 90.00 | −6.7 | −24.7 |  |
| CPV00000 | −85.70 | −24.10 | 16.00 | 0.80 | 0.80 | 90.00 | −10.2 | −30.4 |  |
| CTI00000 | −15.76 | −5.90 | 7.80 | 1.40 | 1.20 | 66.00 | −0.9 | −23.1 |  |
| CTR00000 | −96.00 | −85.30 | 8.20 | 1.30 | 1.00 | 64.00 | −2.1 | −23.2 |  |
| CUB00000 | −80.60 | −79.50 | 21.00 | 2.00 | 1.00 | 172.00 | 0.1 | −24.6 |  |
| CVA00000 | 59.00 | 12.50 | 41.90 | 0.80 | 0.80 | 90.00 | −9.3 | −28.8 |  |
| CYP00000 | 0.50 | 33.20 | 35.10 | 0.80 | 0.80 | 90.00 | −10.2 | −29.8 |  |
| CYPSBA00 | 57.50 | 32.90 | 34.60 | 0.80 | 0.80 | 90.00 | −10.2 | −30.2 | */MB9 |
| CZE00000 | −31.90 | 15.68 | 49.81 | 0.80 | 0.80 | 0.00 | −8.4 | −30.5 |  |
| D 00001 | 26.40 | 9.70 | 50.70 | 1.10 | 1.00 | 41.00 | −7.7 | −28.7 |  |
| D 00002 | 37.20 | 12.60 | 51.40 | 0.80 | 0.80 | 90.00 | −9.3 | −28.2 |  |
| DJI00000 | −17.46 | 42.60 | 11.70 | 0.80 | 0.80 | 90.00 | −10.2 | −30.1 |  |
| DMA00000 | −70.00 | −61.30 | 15.30 | 0.80 | 0.80 | 90.00 | −7.3 | −27.3 |  |
| DNK00001 | 32.28 | 11.60 | 56.00 | 0.80 | 0.80 | 90.00 | −10.2 | −29.0 |  |
| DNK00002 | −49.00 | 12.50 | 56.30 | 0.80 | 0.80 | 90.00 | −8.2 | −27.7 | */MB10 |
| DNK00FAR | −49.00 | −7.20 | 61.70 | 0.80 | 0.80 | 90.00 | −10.2 | −29.5 | */MB10 |
| DOM00000 | −85.40 | −70.40 | 18.70 | 0.80 | 0.80 | 90.00 | −7.2 | −27.1 |  |
| E 00002 | −30.00 |  |  |  |  |  |  |  | 1 |
| EGY00000 | 67.11 | 30.30 | 26.20 | 2.30 | 1.50 | 54.00 | −2.7 | −28.8 |  |
| EQA00000 | −104.00 | −83.10 | −1.40 | 3.10 | 1.40 | 174.00 | 3.8 | −22.7 |  |
| ETH00000 | 58.30 | 40.60 | 10.30 | 2.80 | 2.80 | 64.00 | 1.1 | −28.6 |  |
| F 00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| FIN00000 | 46.80 | 23.80 | 64.30 | 1.50 | 1.00 | 23.00 | −6.2 | −28.6 |  |
| FJI00000 | 148.80 | 178.50 | −17.20 | 0.80 | 0.80 | 90.00 | −7.0 | −26.2 |  |
| FLKSTGGL | −37.10 | −46.80 | −59.60 | 3.70 | 1.40 | 170.00 | −0.9 | −28.7 | */MB4 |
| G 00000 | −37.10 | −4.10 | 53.90 | 1.60 | 1.00 | 151.00 | −4.7 | −27.8 | */MB4 |
| GAB00000 | 39.00 | 11.70 | −0.70 | 1.40 | 1.10 | 79.00 | −1.5 | −23.0 |  |
| GDL00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| GDL00002 | −115.90 | −61.80 | 16.40 | 0.80 | 0.80 | 90.00 | −4.6 | −22.7 | */MB13 |
| GHA00000 | 15.90 | −1.30 | 7.70 | 1.50 | 1.10 | 90.00 | −1.0 | −23.0 |  |
| GIB00000 | 57.50 | −5.40 | 36.10 | 0.80 | 0.80 | 90.00 | −6.8 | −27.0 | */MB9 |
| GMB00000 | −34.00 | −16.40 | 13.40 | 0.80 | 0.80 | 90.00 | −10.2 | −31.0 |  |
| GNB00000 | 40.00 | −15.40 | 12.00 | 0.80 | 0.80 | 90.00 | −9.2 | −28.8 |  |
| GNE00000 | −32.30 | 10.50 | 1.70 | 0.80 | 0.80 | 90.00 | −6.8 | −24.9 |  |
| GRC00000 | 22.05 | 24.70 | 38.30 | 1.70 | 1.00 | 160.00 | −2.7 | −26.6 |  |
| GRD00000 | −32.80 | −61.60 | 12.00 | 0.80 | 0.80 | 90.00 | −7.1 | −26.5 |  |
| GRL00000 | −49.00 | −42.90 | 68.60 | 2.30 | 1.00 | 174.00 | −3.3 | −27.8 | */MB10 |
| GTM00000 | −135.70 | −90.50 | 15.50 | 0.80 | 0.80 | 90.00 | −4.2 | −22.2 |  |
| GUF00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| GUF00002 | −115.90 | −53.30 | 4.30 | 0.80 | 0.80 | 90.00 | −5.3 | −23.4 | */MB13 |
| GUI00000 | 27.50 | −10.90 | 10.20 | 1.30 | 1.10 | 104.00 | −1.5 | −22.9 |  |
| GUMMRA0 | −159.00 | 145.40 | 16.70 | 1.70 | 1.00 | 79.00 | 0.0 | −22.2 | */MB2 |
| GUY00000 | −23.80 | −59.20 | 4.70 | 1.40 | 1.00 | 94.00 | −1.4 | −22.8 |  |
| HKG00000 | 57.50 | 114.50 | 22.40 | 0.80 | 0.80 | 90.00 | −6.5 | −24.5 |  |
| HND00000 | −76.20 | −86.10 | 15.40 | 1.40 | 1.00 | 26.00 | −1.8 | −23.1 |  |
| HNG00000 | −7.50 | 19.40 | 47.40 | 0.80 | 0.80 | 90.00 | −8.8 | −28.1 |  |
| HOL00000 | −5.00 | 5.40 | 52.40 | 0.80 | 0.80 | 90.00 | −10.2 | −30.8 | */MB5 |
| HTI00000 | −92.00 | −73.00 | 18.80 | 0.80 | 0.80 | 90.00 | −7.1 | −26.9 |  |
| HWA00000 | −159.00 | −157.60 | 20.70 | 1.20 | 1.00 | 157.00 | −2.2 | −23.1 | */MB2 |
| HWL00000 | −159.00 | −176.60 | 0.10 | 0.80 | 0.80 | 90.00 | −7.3 | −27.4 | */MB2 |
| I 00000 | −23.40 | 11.30 | 40.90 | 2.10 | 1.00 | 141.00 | −1.6 | −26.4 |  |
| IND00000 | 74.00 | 82.70 | 18.90 | 6.20 | 4.90 | 120.00 | 12.6 | −22.2 |  |
| INS00000 | 115.40 | 117.60 | −1.80 | 9.40 | 4.30 | 170.00 | 13.7 | −22.4 |  |
| IRL00000 | −21.80 | −8.20 | 53.20 | 0.80 | 0.80 | 90.00 | −10.2 | −29.3 |  |
| IRN00000 | 24.19 | 54.30 | 33.00 | 3.70 | 1.50 | 143.00 | 1.1 | −27.5 | 2 |
| IRQ00000 | 65.45 | 44.30 | 33.10 | 1.60 | 1.30 | 178.00 | −4.0 | −28.0 |  |
| ISL00000 | −35.20 | −18.20 | 64.90 | 0.80 | 0.80 | 90.00 | −8.5 | −27.4 |  |
| ISR00000 | −4.00 |  |  |  |  |  |  |  | 1 |
| J 00000 | 152.50 | 140.40 | 30.40 | 5.70 | 3.70 | 15.00 | 11.1 | −22.8 |  |
| JAR00000 | −159.00 | −160.00 | −0.40 | 0.80 | 0.80 | 90.00 | −7.5 | −27.5 | */MB2 |
| JMC00000 | −108.60 | −77.60 | 18.20 | 0.80 | 0.80 | 90.00 | −6.9 | −25.9 |  |
| JON00000 | −159.00 | −168.50 | 17.00 | 0.80 | 0.80 | 90.00 | −10.2 | −32.5 | */MB2 |
| JOR00000 | 81.76 | 36.70 | 31.30 | 0.80 | 0.80 | 90.00 | −9.7 | −28.5 |  |
| KAZ00000 | 58.50 | 66.36 | 46.72 | 4.60 | 1.69 | 176.88 | −0.6 | −28.0 |  |
| KEN00000 | 78.20 | 38.40 | 0.80 | 2.10 | 1.30 | 95.00 | −2.1 | −27.6 |  |
| KER00000 | 113.00 | 69.30 | −43.90 | 1.90 | 1.60 | 169.00 | −2.2 | −27.8 | */MB1 |
| KGZ00000 | 64.60 | 74.54 | 41.15 | 1.56 | 0.80 | 10.12 | −8.3 | −29.7 |  |
| KIR00000 | 150.00 | 173.00 | 1.00 | 0.80 | 0.80 | 90.00 | −7.2 | −27.1 |  |
| KNA00000 | −88.80 | −62.90 | 17.30 | 0.80 | 0.80 | 90.00 | −7.1 | −26.5 |  |
| KOR00000 | 116.20 | 127.70 | 36.20 | 1.30 | 1.00 | 4.00 | −4.3 | −26.7 |  |
| KRE00000 | 145.00 | 127.80 | 39.80 | 1.40 | 1.00 | 14.00 | −1.2 | −23.3 |  |
| KWT00000 | 30.90 | 47.70 | 29.10 | 0.80 | 0.80 | 90.00 | −10.2 | −31.6 |  |
| LAO00000 | 142.00 | 104.10 | 18.10 | 1.50 | 1.00 | 101.00 | −0.7 | −22.6 |  |
| LBN00000 | 97.50 | 35.80 | 33.80 | 0.80 | 0.80 | 90.00 | −10.2 | −30.5 |  |
| LBR00000 | −41.80 | −8.90 | 6.50 | 0.80 | 0.80 | 90.00 | −4.0 | −22.1 |  |
| LBY00000 | 28.90 | 19.00 | 25.90 | 3.00 | 2.70 | 165.00 | 3.1 | −27.8 |  |
| LIE00000 | −17.10 | 9.50 | 47.20 | 0.80 | 0.80 | 90.00 | −10.2 | −31.2 |  |
| LSO00000 | −19.30 | 28.40 | −29.50 | 0.80 | 0.80 | 90.00 | −10.2 | −31.1 |  |
| LTU00000 | −9.30 | 23.67 | 55.23 | 0.80 | 0.80 | 0.00 | −10.2 | −32.5 |  |
| LUX00000 | 19.20 | 6.20 | 49.70 | 0.80 | 0.80 | 90.00 | −10.2 | −31.6 |  |
| MAC00000 | 117.00 | 113.60 | 22.20 | 0.80 | 0.80 | 90.00 | −7.2 | −27.1 |  |
| MAU00000 | 92.20 | 57.50 | −20.20 | 0.80 | 0.80 | 90.00 | −6.9 | −25.6 |  |
| MCO00000 | 52.00 |  |  |  |  |  |  |  | 1 |
| MDG00000 | 16.90 | 46.60 | −18.70 | 2.60 | 1.00 | 66.00 | 1.6 | −22.5 |  |
| MDR00000 | −10.60 | −16.20 | 31.60 | 0.80 | 0.80 | 90.00 | −10.2 | −30.5 | */MB7 |
| MDW00000 | −159.00 | −177.40 | 28.20 | 0.80 | 0.80 | 90.00 | −10.2 | −32.2 | */MB2 |
| MEX00000 | −113.00 |  |  |  |  |  |  |  | 1 |
| MHL00000 | −159.00 | 175.30 | 8.70 | 2.30 | 1.40 | 94.00 | 2.7 | −22.6 | */MB2 |
| MLA00000 | 78.50 | 108.20 | 4.70 | 3.20 | 1.40 | 0.00 | 4.1 | −22.3 |  |
| MLD00000 | 117.60 | 73.40 | 2.50 | 2.20 | 0.80 | 88.00 | 0.1 | −22.4 |  |
| MLI00000 | −6.00 | −3.90 | 17.60 | 3.30 | 2.50 | 21.00 | 6.3 | −24.8 |  |
| MLT00000 | −3.00 | 14.40 | 35.90 | 0.80 | 0.80 | 90.00 | −10.2 | −30.4 |  |
| MNG00000 | 113.60 | 103.80 | 46.80 | 3.60 | 1.10 | 3.00 | −0.3 | −27.6 |  |
| MOZ00000 | 90.60 | 35.60 | −17.20 | 3.10 | 1.10 | 98.00 | 3.2 | −22.0 |  |
| MRC00000 | 32.86 | −8.90 | 27.90 | 3.40 | 1.00 | 45.00 | −0.5 | −27.0 |  |
| MTN00000 | −21.10 | −10.30 | 19.80 | 2.50 | 2.40 | 76.00 | 0.1 | −28.4 |  |
| MWI00000 | 28.00 | 34.10 | −13.30 | 1.60 | 1.00 | 101.00 | −6.7 | −29.3 |  |
| MYT00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| NCG00000 | −84.40 | −84.90 | 12.90 | 1.10 | 1.00 | 16.00 | −2.8 | −23.1 |  |
| NCL00000 | 113.00 | 165.80 | −21.40 | 0.80 | 0.80 | 90.00 | −5.9 | −23.9 | */MB1 |
| NGR00000 | −38.50 | 7.50 | 17.20 | 2.10 | 1.70 | 100.00 | −0.6 | −27.3 |  |
| NIG00000 | 41.82 | 8.00 | 9.90 | 2.50 | 1.60 | 47.00 | 3.4 | −22.4 |  |
| NMB00000 | 12.20 | 18.50 | −21.00 | 2.70 | 2.60 | 155.00 | −0.7 | −29.6 |  |
| NOR00000 | −0.80 |  |  |  |  |  |  |  | 1 |
| NPL00000 | 123.30 | 84.40 | 28.00 | 0.80 | 0.80 | 90.00 | −7.2 | −26.6 |  |
| NRU00000 | 146.00 | 166.90 | −0.50 | 0.80 | 0.80 | 90.00 | −7.2 | −27.2 |  |
| NZL00001 | 152.00 | 170.90 | −44.80 | 5.40 | 1.00 | 49.00 | 2.0 | −26.5 | */MB14 |
| NZL00002 | 152.00 | −165.40 | −13.20 | 2.70 | 2.00 | 82.00 | 5.4 | −22.0 | */MB14 |
| OCE00000 | −115.90 | −141.90 | −16.10 | 3.50 | 2.40 | 139.00 | 6.8 | −24.2 | */MB13 |
| OMA00000 | 104.00 | 55.10 | 21.60 | 1.90 | 1.00 | 61.00 | −6.0 | −29.3 |  |
| PAK00000 | 56.50 | 69.90 | 29.80 | 3.00 | 2.00 | 22.00 | 3.7 | −25.7 |  |
| PHL00000 | 161.00 | 122.23 | 11.37 | 3.33 | 1.41 | 79.65 | 4.8 | −22.3 |  |
| PLM00000 | −159.00 | −161.40 | 7.00 | 0.80 | 0.80 | 90.00 | −7.6 | −27.6 | */MB2 |
| PNG00000 | 154.10 | 148.40 | −6.60 | 3.30 | 2.30 | 167.00 | 6.0 | −22.7 |  |
| PNR00000 | −79.20 | −80.20 | 8.50 | 1.20 | 1.00 | 177.00 | −2.4 | −23.2 |  |
| POL00000 | 15.20 | 19.30 | 52.00 | 1.30 | 1.00 | 166.00 | −7.0 | −28.7 |  |
| POR00000 | −10.60 | −8.00 | 39.70 | 0.80 | 0.80 | 90.00 | −9.0 | −28.1 | */MB7 |
| PRG00000 | −81.50 | −58.70 | −23.10 | 1.50 | 1.30 | 116.00 | 0.1 | −22.8 |  |
| PRU00000 | −89.90 | −74.20 | −8.40 | 3.60 | 2.40 | 111.00 | 6.9 | −22.5 |  |
| PTC00000 | −62.30 | −130.10 | −25.10 | 0.80 | 0.80 | 90.00 | −10.2 | −27.3 |  |
| QAT00000 | 0.90 | 51.60 | 25.40 | 0.80 | 0.80 | 90.00 | −10.2 | −31.5 |  |
| REU00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| REU00002 | 113.00 | 55.60 | −21.10 | 0.80 | 0.80 | 90.00 | −6.4 | −24.5 | */MB1 |
| ROU00000 | 30.45 | 25.00 | 46.30 | 1.50 | 1.00 | 178.00 | −5.2 | −28.0 |  |
| RRW00000 | 17.60 | 29.70 | −1.90 | 0.80 | 0.80 | 90.00 | −10.2 | −30.8 |  |
| RUS00001 | 61.00 | 51.50 | 52.99 | 5.56 | 2.01 | 10.74 | 3.1 | −28.2 |  |
| RUS00002 | 88.10 |  |  |  |  |  | 5.4 | −26.32 | 3 |
| RUS00003 | 138.50 | 138.14 | 53.83 | 5.86 | 2.09 | 8.41 | 3.3 | −28.4 |  |
| S 00000 | 5.00 |  |  |  |  |  |  |  | 1 |
| SDN00001 | 23.55 |  |  |  |  |  |  |  | 1 |
| SDN00002 | 23.55 |  |  |  |  |  |  |  | 1 |
| SEN00000 | −48.40 | −14.00 | 14.10 | 1.10 | 1.00 | 148.00 | −2.3 | −23.8 |  |
| SEY00000 | 42.25 | 51.50 | −3.20 | 13.80 | 3.80 | 48.50 | −1.3 | −33.8 |  |
| SLM00000 | 147.50 | 159.00 | −9.10 | 1.50 | 1.00 | 147.00 | −1.2 | −23.0 |  |
| SLV00000 | −130.50 | −89.00 | 13.70 | 0.80 | 0.80 | 90.00 | −6.8 | −24.9 |  |
| SMA00000 | −159.00 | −170.70 | −14.20 | 0.80 | 0.80 | 90.00 | −10.2 | −31.1 | */MB2 |
| SMO00000 | −125.50 | −172.10 | −13.70 | 0.80 | 0.80 | 90.00 | −6.6 | −24.6 |  |
| SMR00000 | 16.50 | 12.50 | 43.90 | 0.80 | 0.80 | 90.00 | −10.2 | −30.3 |  |
| SNG00000 | 98.10 | 103.90 | 1.30 | 0.80 | 0.80 | 90.00 | −7.3 | −25.4 |  |
| SOM00000 | 98.40 | 46.00 | 6.30 | 3.10 | 1.00 | 72.00 | −0.8 | −25.5 |  |
| SPM00000 | −8.00 |  |  |  |  |  |  |  | 1 |
| SRL00000 | −51.80 | −11.90 | 8.50 | 0.80 | 0.80 | 90.00 | −6.9 | −25.4 |  |
| STP00000 | 30.25 | 7.00 | 1.00 | 0.80 | 0.80 | 90.00 | −7.1 | −27.0 |  |
| SUI00000 | 9.45 | 8.20 | 46.50 | 0.80 | 0.80 | 90.00 | −10.2 | −29.4 |  |
| SUR00000 | −77.00 | −55.60 | 3.90 | 1.00 | 0.90 | 37.00 | −3.6 | −23.2 |  |
| SVK00000 | −19.82 | 17.30 | 49.60 | 1.30 | 1.00 | 166.00 | −5.1 | −27.4 |  |
| SWZ00000 | 30.10 | 31.30 | −26.40 | 0.80 | 0.80 | 90.00 | −10.2 | −30.9 |  |
| SYR00000 | 18.00 | 38.60 | 35.30 | 1.10 | 1.00 | 32.00 | −7.1 | −28.3 |  |
| TCD00000 | −9.90 | 18.40 | 15.60 | 3.50 | 1.60 | 97.00 | 5.0 | −24.1 |  |
| TGO00000 | −23.15 | 0.80 | 8.60 | 1.10 | 1.00 | 116.00 | −2.7 | −23.2 |  |
| THA00000 | 120.60 | 100.90 | 12.80 | 2.80 | 1.60 | 83.00 | 4.0 | −22.6 |  |
| TON00000 | −128.00 | −175.20 | −21.20 | 0.80 | 0.80 | 90.00 | −6.7 | −24.7 |  |
| TRD00000 | −73.40 | −61.10 | 10.80 | 0.80 | 0.80 | 90.00 | −7.2 | −27.3 |  |
| TUN00000 | 5.74 | 9.40 | 33.50 | 1.30 | 1.00 | 104.00 | −5.9 | −28.2 |  |
| TUR00000 | 8.50 | 34.10 | 38.90 | 2.80 | 1.00 | 171.00 | 0.0 | −26.0 |  |
| TUV00000 | 158.00 | 179.20 | −8.50 | 0.80 | 0.80 | 90.00 | −7.1 | −27.1 |  |
| TZA00000 | 67.50 | 35.40 | −5.90 | 2.40 | 1.40 | 117.00 | −1.3 | −27.8 |  |
| UAE00000 | 63.50 | 53.80 | 24.90 | 1.10 | 1.00 | 12.00 | −9.7 | −30.4 |  |
| UGA00000 | 31.50 | 32.20 | 0.90 | 1.50 | 1.00 | 70.00 | −6.3 | −28.9 |  |
| UKR00001 | 38.20 | 31.73 | 48.22 | 2.21 | 0.97 | 178.15 | −9.1 | −31.0 |  |
| URG00000 | −86.10 | −56.30 | −33.70 | 1.10 | 1.00 | 58.00 | −6.5 | −27.7 |  |
| USA00000 | −101.00 |  |  |  |  |  | 11.2 | −23.9 | 3,*/MB16 |
| USAVIPRT | −101.00 | −64.50 | 17.80 | 0.80 | 0.80 | 90.00 | −6.9 | −25.5 | */MB16 |
| UZB00000 | 110.50 | 65.45 | 41.09 | 1.49 | 1.05 | 10.98 | −10.2 | −31.0 |  |
| VCT00000 | −93.10 | −61.10 | 13.20 | 0.80 | 0.80 | 90.00 | −7.0 | −26.2 |  |
| VEN00001 | −82.70 | −66.40 | 6.80 | 2.80 | 2.10 | 142.00 | 4.9 | −22.8 | */MB17 |
| VEN00002 | −82.70 | −63.60 | 15.70 | 0.80 | 0.80 | 90.00 | −7.1 | −27.0 | */MB17 |
| VTN00000 | 107.00 |  |  |  |  |  | 2.9 | −18.6 | 3 |
| VUT00000 | 150.70 | 168.40 | −17.20 | 1.20 | 1.00 | 122.00 | −2.4 | −23.1 |  |
| WAK00000 | −159.00 | 166.50 | 19.20 | 0.80 | 0.80 | 90.00 | −10.2 | −31.9 | */MB2 |
| WAL00000 | 113.00 | −177.10 | −13.80 | 0.80 | 0.80 | 90.00 | −6.0 | −24.1 | */MB1 |
| XAN00000 | −5.00 | −65.60 | 15.10 | 1.30 | 1.00 | 58.00 | −1.1 | −22.3 | */MB5 |
| XCQ00000 | −159.00 | 173.40 | 4.60 | 10.20 | 2.40 | 175.00 | 16.0 | −16.0 | */MB2 |
| XYU00000 | 43.04 | 18.70 | 44.40 | 1.10 | 1.00 | 161.00 | −5.6 | −27.3 |  |
| YEM00001 | 27.00 | 44.20 | 15.10 | 1.00 | 1.00 | 103.00 | −9.8 | −30.1 |  |
| YEM00002 | 108.00 | 49.90 | 14.80 | 1.40 | 1.00 | 53.00 | −5.7 | −26.9 |  |
| ZMB00000 | 39.55 | 27.90 | −12.80 | 2.40 | 1.60 | 26.00 | −3.0 | −29.2 |  |
| ZWE00000 | 65.60 | 30.00 | −18.90 | 1.50 | 1.10 | 140.00 | −6.0 | −28.9 |  |

ARTICLE 11

Period of validity of the provisions and associated Plan

11.1	These provisions and associated Plan have been prepared in order to guarantee in practice for all countries equitable access to the GSO and the frequency bands contained in Article 3, to meet the requirements of the fixed-satellite service for a period of at least 20 years from the date of entry into force of this Appendix.

11.2	These provisions and associated Plan shall, in any event, remain in force until their revision by a competent world radiocommunication conference, convened in accordance with the relevant provisions of the ITU Constitution and Convention in force.     (WRC-07)

ANNEX 1     (WRC-03)

Parameters used in characterizing the fixed-satellite service
allotment Plan     (WRC-07)

Section A     (SUP - WRC-07)

# 1	Basic technical characteristics

The allotments in the Plan are based on a reference satellite network with the following assumptions:

## 1.1	Type of modulation

The Plan is independent of modulation characteristics and accessing techniques.

## 1.2	Parameters used for calculating the earth station and space station power densities

The carrier-to-noise ratio (C/N) is as follows:

a)	the uplink C/N ratio exceeds 21 dB under rain-faded conditions with a minimum earth station transmitter power density of −60 dB(W/Hz) averaged over the necessary bandwidth of the modulated carrier;

b)	the downlink C/N ratio exceeds 15 dB under rain-faded conditions;

c)	for the 6/4 GHz bands, the above C/Ns are exceeded for 99.95% of the year

(NOTE – The rain attenuation margin is limited to a maximum of 8 dB);

d)	for the 13/10-11 GHz bands, the above C/Ns are exceeded for 99.9% of the year

(NOTE – The rain attenuation margin is limited to a maximum of 8 dB);

e)	the gaseous atmospheric attenuation and rain attenuation models used are those described in Recommendations ITU-R P.676-7 and ITU-R P.618-9.     (WRC-07)

## 1.3	Earth station antenna elevation angle

The minimum elevation angle for each test point included in the service area is based on the following:

10°  for 		Rp ≤ 40 mm/h;

20°  for  40 <	Rp ≤ 70 mm/h;

30°  for  70 <	Rp ≤ 100 mm/h;

40°  for 		Rp > 100 mm/h.

Where Rp is the rainfall rate exceeded for any given percentage p of the average year, calculated in accordance with Recommendation ITU-R P.837-5. Administrations may select lower elevation angles for their service areas. For countries at high latitudes or with dispersed territories, in the absence of such a request, if the above values for minimum elevation angle are unobtainable, then the highest elevation angle leading to a non-zero range of possible orbital positions applies. In mountainous areas, the elevation angles are specified by the administrations concerned.     (WRC-07)

## 1.4	Interference criteria

The Plan has been prepared with a view to assuring for each allotment an overall aggregate carrier-to-interference value under free-space conditions of 21 dB or higher, and an overall single entry carrier-to-interference value under free-space conditions of 25 dB.     (WRC-07)

## 1.5	Polarization

Polarization isolation between satellite networks was not used in the development of the Allotment Plan.

## 1.6	Earth station characteristics

1.6.1	The diameters of the earth station antennas are:

5.5 m for the 6/4 GHz band;

2.7 m for the 13/10-11 GHz band.     (WRC-07)

1.6.2	The earth station receiving system noise temperature referred to the output of the receiving antenna is:

95 K for the 4 GHz band;

125 K for the 10-11 GHz band.     (WRC-07)

1.6.3	The earth station antenna efficiency is 70%.

1.6.3bis	The gains of the earth station antennas for the diameters and the efficiency specified above at the indicated evaluation frequencies are as follows:

50.4 dBi at 6 875 MHz;

47.0 dBi at 4 650 MHz;

49.8 dBi at 13.0 GHz;

48.4 dBi at 11.075 GHz.     (WRC-07)

1.6.4	The applicable earth station reference antenna pattern is shown in Table 1 below.    (WRC-07)

TABLE  1     (WRC-07)

| Gmax = 10 log ((D/)2) dBi |  |  |  |
| --- | --- | --- | --- |
| G(φ) = Gmax − 2.5 × 10–3 for 0 < φ < φm dBi |  |  |  |
| G(φ) = min (G1, 29 – 25 log φ) for φm ≤ φ 19.95° dBi |  |  |  |
| G(φ) = max (min (−3.5, 32 − 25 log φ) – 10) for φ > 19.95° dBi |  |  |  |
| where: |  |  |  |
|  | D : antenna diameter  : wavelength | expressed in the same unit |  |
| φ : off-axis angle of the antenna (degrees) |  |  |  |
|  : antenna efficiency |  |  |  |

## 1.7	Space station characteristics     (WRC-07)

1.7.1	The allotment Plan is based on the use of space station antennas with beams of elliptical cross-section.

1.7.2	The antenna radiation characteristics are as shown in Fig. 1.

FIGURE 1*     (WRC-07)

Reference patterns for satellite antennas
with fast roll-off in the main beam

Gmax  =  44.45  –  10 log (φ01    φ02)          dBi     (WRC-07)

Curve A:	dB relative to main beam gain

−12 (φ/φ0)2	for  0  ≤  (φ/φ0)  ≤  0.5

for  0.5  <  (φ/φ0)  ≤

−25.23	for

−(22  +  20 log (φ/φ0))	for  (φφ0)    1.45

after intersection with Curve B: Curve B.

Curve B:	minus the on-axis gain (Curve B represents examples of four antennas having different values of φ0 as labelled in Fig. 1. The on-axis gains of these antennas are approximately 28.3, 34.3, 40.4 and 46.4 dBi, respectively)     (WRC-07)

where:

φ :	off-axis angle (degrees)

φ0 :	cross-sectional half-power beamwidth in the direction of interest (degrees)

φ01, φ02 :	major and minor axis half-power beamwidth, respectively, of elliptical beam (degrees)     (WRC-07)

where:

1.7.3	The space station receiving system noise temperature referred to the output of the receiving antenna is:

500 K for the 6 GHz band;

550 K for the 13 GHz band.

1.7.4	The minimum beamwidth size, in terms of the half-power beamwidth, is 1.6° for the 6/4 GHz band and 0.8° for the 13/10-11 GHz band.

1.7.5	The space station antenna efficiency is 55%.

1.7.6	The deviation of the space station antenna beam from its nominal pointing direction is limited to 0.1° in any direction. The rotation accuracy of elliptical beams is ±1.0°.

## 1.8	Bandwidth

The allotment Plan is based on the carrier power averaged over the necessary bandwidth of the modulated carrier and referred to a 1 MHz bandwidth.

Section B     (SUP - WRC-07)

ANNEX 2     (SUP - WRC-07)

ANNEX 3     (WRC-07)

Limits applicable to submissions received under Article 6 or Article 715

Under assumed free-space propagation conditions, the power flux-density (space-to-Earth) of a proposed new allotment or assignment produced on any portion of the surface of the Earth shall not exceed:

–	−127.5 dB(W/(m2 · MHz)) in the 4 500-4 800 MHz band; and

–	−114.0 dB(W/(m2 · MHz)) in the 10.70-10.95 GHz and 11.20-11.45 GHz bands.

Under assumed free-space propagation conditions, the power flux-density (Earth-to-space) of a proposed new allotment or assignment shall not exceed:

–	−140.0 dB(W/(m2 · MHz)) towards any location in the geostationary-satellite orbit located more than 10° from the proposed orbital position in the 6 725-7 025 MHz band, and

–	−133.0 dB(W/(m2 · MHz)) towards any location in the geostationary-satellite orbit located more than 9° from the proposed orbital position in the 12.75-13.25 GHz band.

ANNEX 4     (REV.WRC-07)

Criteria for determining whether an allotment or
an assignment is considered to be affected

An allotment or an assignment is considered as being affected by a proposed new allotment or assignment:

1	if the orbital spacing between its orbital position and the orbital position of the proposed new allotment or assignment is equal to or less than:

1.1	10° in the 4 500-4 800 MHz (space-to-Earth) and 6 725-7 025 MHz (Earth-to-space) bands;

1.2	9° in the 10.70-10.95 GHz (space-to-Earth), 11.20-11.45 GHz (space-to-Earth) and 12.75-13.25 GHz (Earth-to-space) bands;

and

2	if at least one of the following three conditions is not satisfied:

2.1	the calculated16 Earth-to-space single-entry carrier-to-interference (C/I)u value at each test point associated with the allotment or assignment under consideration is greater than or equal to a reference value that is 30 dB, or (C/N)u + 9 dB17, or any already accepted Earth-to-space single-entry (C/I)u18, whichever is the lowest;

2.2	the calculated16 space-to-Earth single-entry (C/I)d value everywhere within the service area of the allotment or assignment under consideration is greater than or equal to a reference value19 that is 26.65 dB, or (C/N)d + 11.65 dB20, or any already accepted space-to-Earth single-entry (C/I)d value, whichever is the lowest;

2.3	the calculated16 overall aggregate (C/I)agg value at each test point associated with the allotment or assignment under consideration, is greater than or equal to a reference value that is 21 dB, or (C/N)t + 7 dB21, or any already accepted overall aggregate (C/I)agg value, whichever is the lowest, with a tolerance of 0.25 dB22 in the case of assignments not stemming from the conversion of an allotment into an assignment without modification, or when the modification is within the envelope characteristics of the initial allotment.

APPENDIX 1 TO ANNEX 4     (Rev.WRC-07)

Method for determination of the overall single-entry and aggregate
carrier-to-interference value averaged over the necessary
bandwidth of the modulated carrier

# 1	Single-entry C/I

This section describes the method for calculating the single-entry interference potential.

The method is based on the single-entry carrier-to-interference ratio (C/I) which a given allotment or assignment made in accordance with the provisions of Appendix 30B might experience due to an emission from the proposed new assignment or modification. The single-entry uplink (C/I)u and downlink (C/I)d values due to a single interfering satellite network are given by:

dB

dB

where:

θ, φ, ρ, η, ξ are angles as defined in Fig. 1 above.

In the following, all ratios are numerical power ratios:

p1:	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the desired earth station transmitting antenna (W/Hz)

g1:	maximum gain of the desired transmitting earth station antenna

lsu:	free-space path loss of the desired up-path signal

lsu′ :	free-space path loss of the interfering up-path signal

g2(φ):	gain of the desired space station receiving antenna in the direction of the desired earth station

g2:	maximum gain of the desired space station receiving antenna

p1′ :	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the interfering earth station transmitting antenna (W/Hz)

g1′  (θ):	interfering earth station antenna gain in the direction of the desired satellite

lsd:	free-space path loss of the desired down-path signal

lsd ′ :	free-space path loss of the interfering down-path signal

g2(ρ):	gain of the desired space station receiving antenna in the direction of the interfering earth station

p3:	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the desired space station transmitting antenna (W/Hz)

g3(φ):	desired space station transmitting antenna gain in the direction of the desired earth station

g3:	maximum gain of the desired space station transmitting antenna

g4:	maximum gain of the desired receiving earth station antenna

p3′ :	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the interfering space station transmitting antenna (W/Hz)

g3′  (η):	interfering space station transmitting antenna gain in the direction of the desired earth station

g4(ξ):	desired earth station receiving antenna gain in the direction of the interfering satellite.

The overall single-entry (C/I)t at a given downlink test point due to a single interfering allotment or assignment is given by:

dB

where:

(C/I)umin:	lowest uplink C/I value among all uplink test points

(C/I)d:	downlink C/I value at the test point under consideration.

NOTE – When only one of the uplink or the downlink is implemented in the bands subject to Appendix 30B, only the contribution from the link that is implemented in the bands subject to Appendix 30B shall be considered in calculating (C/I)t.

# 2	Aggregate C/I

The aggregate (C/I)agg at a given downlink test point is given by:

dB

j  =  1, 2, 3 . . . n,

where:

(C/I)tj:	overall carrier-to-interference ratio due to interference from the j-th allotment or assignment calculated using the method for overall single-entry (C/I)t as provided in § 1 of Appendix 1 to this Annex; and

n:	total number of interfering allotments or assignments for which the orbital separation with the desired satellite is less than or equal to 10° in the case of the 6/4 GHz band and less than or equal to 9° in the case of the 13/10-11 GHz band.

APPENDIX 2 TO ANNEX 4     (WRC-07)

Method for determination of the carrier-to-noise (C /N) values

The uplink carrier-to noise value (C/N)u and the downlink carrier-to-noise value (C/N)d are calculated as follows:

dB

dB

where:

In the following, all ratios are numerical power ratios.

p1:	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the earth station transmitting antenna (W/Hz)

g1:	maximum gain of the transmitting earth station antenna

lsu:	free-space path loss of the up-path signal

g2(φ):	gain of the space station receiving antenna in the direction of the earth station

Ts:	space station receiving system noise temperature referred to the output of the receiving antenna

p3:	power density, averaged over the necessary bandwidth of the modulated carrier, fed into the space station transmitting antenna (W/Hz)

g3(φ):	space station transmitting antenna gain in the direction of the earth station

lsd:	free-space path loss of the down-path signal

g4:	maximum gain of the receiving earth station antenna

Te:	earth station receiving system noise temperature, referred to the output of the receiving antenna

k:	Boltzmann’s constant.

The overall carrier-to-noise value (C/N)t is then calculated as follows:

dB

where:

(C/N)umin:	lowest uplink C/N value among all test points

(C/N)d:	downlink C/N value at the test point under consideration.

NOTE – When only one of the uplink or the downlink is implemented in the bands subject to Appendix 30B, only the contribution from the link that is implemented in the bands subject to Appendix 30B shall be considered in calculating (C/N)t.

ANNEX 5     (SUP - WRC-07)

ANNEX 6     (SUP - WRC-07)

APPENDIX 42 (REV.WRC-15)

Table of allocation of international call sign series

(See Article 19)

| Call sign series | Allocated to |
| --- | --- |
| AAA-ALZ | United States of America |
| AMA-AOZ | Spain |
| APA-ASZ | Pakistan (Islamic Republic of) |
| ATA-AWZ | India (Republic of) |
| AXA-AXZ | Australia |
| AYA-AZZ | Argentine Republic |
| A2A-A2Z | Botswana (Republic of) |
| A3A-A3Z | Tonga (Kingdom of) |
| A4A-A4Z | Oman (Sultanate of) |
| A5A-A5Z | Bhutan (Kingdom of) |
| A6A-A6Z | United Arab Emirates |
| A7A-A7Z | Qatar (State of) |
| A8A-A8Z | Liberia (Republic of) |
| A9A-A9Z | Bahrain (Kingdom of) |
| BAA-BZZ | China (People’s Republic of) |
| CAA-CEZ | Chile |
| CFA-CKZ | Canada |
| CLA-CMZ | Cuba |
| CNA-CNZ | Morocco (Kingdom of) |
| COA-COZ | Cuba |
| CPA-CPZ | Bolivia (Republic of) |
| CQA-CUZ | Portugal |
| CVA-CXZ | Uruguay (Eastern Republic of) |
| CYA-CZZ | Canada |
| C2A-C2Z | Nauru (Republic of) |
| C3A-C3Z | Andorra (Principality of) |
| C4A-C4Z | Cyprus (Republic of) |
| C5A-C5Z | Gambia (Republic of the) |
| C6A-C6Z | Bahamas (Commonwealth of the) |
| *C7A-C7Z | World Meteorological Organization |
| C8A-C9Z | Mozambique (Republic of) |
| DAA-DRZ | Germany (Federal Republic of) |
| DSA-DTZ | Korea (Republic of) |
| DUA-DZZ | Philippines (Republic of the) |
| D2A-D3Z | Angola (Republic of) |
| D4A-D4Z | Cape Verde (Republic of) |
| D5A-D5Z | Liberia (Republic of) |
| D6A-D6Z | Comoros (Union of) |
| D7A-D9Z | Korea (Republic of) |

| Call sign series | Allocated to |  |
| --- | --- | --- |
| EAA-EHZ | Spain |  |
| EIA-EJZ | Ireland |  |
| EKA-EKZ | Armenia (Republic of) |  |
| ELA-ELZ | Liberia (Republic of) |  |
| EMA-EOZ | Ukraine |  |
| EPA-EQZ | Iran (Islamic Republic of) |  |
| ERA-ERZ | Moldova (Republic of) |  |
| ESA-ESZ | Estonia (Republic of) |  |
| ETA-ETZ | Ethiopia (Federal Democratic Republic of) |  |
| EUA-EWZ | Belarus (Republic of) |  |
| EXA-EXZ | Kyrgyz Republic |  |
| EYA-EYZ | Tajikistan (Republic of) |  |
| EZA-EZZ | Turkmenistan |  |
| E2A-E2Z | Thailand |  |
| E3A-E3Z | Eritrea |  |
| E4A-E4Z | Palestinian Authority1 |  |
| E5A-E5Z | New Zealand – Cook Islands | (WRC-07) |
| E6A-E6Z | New Zealand – Niue | (WRC-15) |
| E7A-E7Z | Bosnia and Herzegovina | (WRC-07) |
| FAA-FZZ | France |  |
| GAA-GZZ | United Kingdom of Great Britain and Northern Ireland |  |
| HAA-HAZ | Hungary (Republic of) |  |
| HBA-HBZ | Switzerland (Confederation of) |  |
| HCA-HDZ | Ecuador |  |
| HEA-HEZ | Switzerland (Confederation of) |  |
| HFA-HFZ | Poland (Republic of) |  |
| HGA-HGZ | Hungary (Republic of) |  |
| HHA-HHZ | Haiti (Republic of) |  |
| HIA-HIZ | Dominican Republic |  |
| HJA-HKZ | Colombia (Republic of) |  |
| HLA-HLZ | Korea (Republic of) |  |
| HMA-HMZ | Democratic People’s Republic of Korea |  |
| HNA-HNZ | Iraq (Republic of) |  |
| HOA-HPZ | Panama (Republic of) |  |
| HQA-HRZ | Honduras (Republic of) |  |
| HSA-HSZ | Thailand |  |
| HTA-HTZ | Nicaragua |  |
| HUA-HUZ | El Salvador (Republic of) |  |
| HVA-HVZ | Vatican City State |  |
| HWA-HYZ | France |  |
| HZA-HZZ | Saudi Arabia (Kingdom of) |  |
| H2A-H2Z | Cyprus (Republic of) |  |
| H3A-H3Z | Panama (Republic of) |  |
| H4A-H4Z | Solomon Islands |  |
| H6A-H7Z | Nicaragua |  |
| H8A-H9Z | Panama (Republic of) |  |
| IAA-IZZ | Italy |  |
| 1 In response to Resolution 99 (Rev. Busan, 2014) of the Plenipotentiary Conference. (WRC-15) |  |  |

| Call sign series | Allocated to |
| --- | --- |
| JAA-JSZ | Japan |
| JTA-JVZ | Mongolia |
| JWA-JXZ | Norway |
| JYA-JYZ | Jordan (Hashemite Kingdom of) |
| JZA-JZZ | Indonesia (Republic of) |
| J2A-J2Z | Djibouti (Republic of) |
| J3A-J3Z | Grenada |
| J4A-J4Z | Greece |
| J5A-J5Z | Guinea-Bissau (Republic of) |
| J6A-J6Z | Saint Lucia |
| J7A-J7Z | Dominica (Commonwealth of) |
| J8A-J8Z | Saint Vincent and the Grenadines |
| KAA-KZZ | United States of America |
| LAA-LNZ | Norway |
| LOA-LWZ | Argentine Republic |
| LXA-LXZ | Luxembourg |
| LYA-LYZ | Lithuania (Republic of) |
| LZA-LZZ | Bulgaria (Republic of) |
| L2A-L9Z | Argentine Republic |
| MAA-MZZ | United Kingdom of Great Britain and Northern Ireland |
| NAA-NZZ | United States of America |
| OAA-OCZ | Peru |
| ODA-ODZ | Lebanon |
| OEA-OEZ | Austria |
| OFA-OJZ | Finland |
| OKA-OLZ | Czech Republic |
| OMA-OMZ | Slovak Republic |
| ONA-OTZ | Belgium |
| OUA-OZZ | Denmark |
| PAA-PIZ | Netherlands (Kingdom of the) |
| PJA-PJZ | Netherlands (Kingdom of the) – Netherlands Antilles |
| PKA-POZ | Indonesia (Republic of) |
| PPA-PYZ | Brazil (Federative Republic of) |
| PZA-PZZ | Suriname (Republic of) |
| P2A-P2Z | Papua New Guinea |
| P3A-P3Z | Cyprus (Republic of) |
| P4A-P4Z | Netherlands (Kingdom of the) – Aruba |
| P5A-P9Z | Democratic People’s Republic of Korea |
| RAA-RZZ | Russian Federation |

| Call sign series | Allocated to |
| --- | --- |
| SAA-SMZ | Sweden |
| SNA-SRZ | Poland (Republic of) |
| SSA-SSM | Egypt (Arab Republic of) |
| SSN-STZ | Sudan (Republic of the) |
| SUA-SUZ | Egypt (Arab Republic of) |
| SVA-SZZ | Greece |
| S2A-S3Z | Bangladesh (People’s Republic of) |
| S5A-S5Z | Slovenia (Republic of) |
| S6A-S6Z | Singapore (Republic of) |
| S7A-S7Z | Seychelles (Republic of) |
| S8A-S8Z | South Africa (Republic of) |
| S9A-S9Z | Sao Tome and Principe (Democratic Republic of) |
| TAA-TCZ | Turkey |
| TDA-TDZ | Guatemala (Republic of) |
| TEA-TEZ | Costa Rica |
| TFA-TFZ | Iceland |
| TGA-TGZ | Guatemala (Republic of) |
| THA-THZ | France |
| TIA-TIZ | Costa Rica |
| TJA-TJZ | Cameroon (Republic of) |
| TKA-TKZ | France |
| TLA-TLZ | Central African Republic |
| TMA-TMZ | France |
| TNA-TNZ | Congo (Republic of the) |
| TOA-TQZ | France |
| TRA-TRZ | Gabonese Republic |
| TSA-TSZ | Tunisia |
| TTA-TTZ | Chad (Republic of) |
| TUA-TUZ | Côte d'Ivoire (Republic of) |
| TVA-TXZ | France |
| TYA-TYZ | Benin (Republic of) |
| TZA-TZZ | Mali (Republic of) |
| T2A-T2Z | Tuvalu |
| T3A-T3Z | Kiribati (Republic of) |
| T4A-T4Z | Cuba |
| T5A-T5Z | Somali Democratic Republic |
| T6A-T6Z | Afghanistan |
| T7A-T7Z | San Marino (Republic of) |
| T8A-T8Z | Palau (Republic of) |
| UAA-UIZ | Russian Federation |
| UJA-UMZ | Uzbekistan (Republic of) |
| UNA-UQZ | Kazakhstan (Republic of) |
| URA-UZZ | Ukraine |

| Call sign series | Allocated to |  |
| --- | --- | --- |
| VAA-VGZ | Canada |  |
| VHA-VNZ | Australia |  |
| VOA-VOZ | Canada |  |
| VPA-VQZ | United Kingdom of Great Britain and Northern Ireland |  |
| VRA-VRZ | China (People’s Republic of) – Hong Kong |  |
| VSA-VSZ | United Kingdom of Great Britain and Northern Ireland |  |
| VTA-VWZ | India (Republic of) |  |
| VXA-VYZ | Canada |  |
| VZA-VZZ | Australia |  |
| V2A-V2Z | Antigua and Barbuda |  |
| V3A-V3Z | Belize |  |
| V4A-V4Z | Saint Kitts and Nevis (Federation of) |  |
| V5A-V5Z | Namibia (Republic of) |  |
| V6A-V6Z | Micronesia (Federated States of) |  |
| V7A-V7Z | Marshall Islands (Republic of the) |  |
| V8A-V8Z | Brunei Darussalam |  |
| WAA-WZZ | United States of America |  |
| XAA-XIZ | Mexico |  |
| XJA-XOZ | Canada |  |
| XPA-XPZ | Denmark |  |
| XQA-XRZ | Chile |  |
| XSA-XSZ | China (People’s Republic of) |  |
| XTA-XTZ | Burkina Faso |  |
| XUA-XUZ | Cambodia (Kingdom of) |  |
| XVA-XVZ | Viet Nam (Socialist Republic of) |  |
| XWA-XWZ | Lao People’s Democratic Republic |  |
| XXA-XXZ | China (People’s Republic of) – Macao | (WRC-07) |
| XYA-XZZ | Myanmar (Union of) |  |
| YAA-YAZ | Afghanistan |  |
| YBA-YHZ | Indonesia (Republic of) |  |
| YIA-YIZ | Iraq (Republic of) |  |
| YJA-YJZ | Vanuatu (Republic of) |  |
| YKA-YKZ | Syrian Arab Republic |  |
| YLA-YLZ | Latvia (Republic of) |  |
| YMA-YMZ | Turkey |  |
| YNA-YNZ | Nicaragua |  |
| YOA-YRZ | Romania |  |
| YSA-YSZ | El Salvador (Republic of) |  |
| YTA-YUZ | Serbia (Republic of) | (WRC-07) |
| YVA-YYZ | Venezuela (Bolivarian Republic of) |  |
| Y2A-Y9Z | Germany (Federal Republic of) |  |
| ZAA-ZAZ | Albania (Republic of) |  |
| ZBA-ZJZ | United Kingdom of Great Britain and Northern Ireland |  |
| ZKA-ZMZ | New Zealand |  |
| ZNA-ZOZ | United Kingdom of Great Britain and Northern Ireland |  |
| ZPA-ZPZ | Paraguay (Republic of) |  |

| Call sign series | Allocated to |  |
| --- | --- | --- |
| ZQA-ZQZ | United Kingdom of Great Britain and Northern Ireland |  |
| ZRA-ZUZ | South Africa (Republic of) |  |
| ZVA-ZZZ | Brazil (Federative Republic of) |  |
| Z2A-Z2Z | Zimbabwe (Republic of) |  |
| Z3A-Z3Z | The Former Yugoslav Republic of Macedonia |  |
| Z8A-Z8Z | South Sudan (Republic of) | (WRC-15) |
| 2AA-2ZZ | United Kingdom of Great Britain and Northern Ireland |  |
| 3AA-3AZ | Monaco (Principality of) |  |
| 3BA-3BZ | Mauritius (Republic of) |  |
| 3CA-3CZ | Equatorial Guinea (Republic of) |  |
| 3DA-3DM | Swaziland (Kingdom of) |  |
| 3DN-3DZ | Fiji (Republic of) |  |
| 3EA-3FZ | Panama (Republic of) |  |
| 3GA-3GZ | Chile |  |
| 3HA-3UZ | China (People’s Republic of) |  |
| 3VA-3VZ | Tunisia |  |
| 3WA-3WZ | Viet Nam (Socialist Republic of) |  |
| 3XA-3XZ | Guinea (Republic of) |  |
| 3YA-3YZ | Norway |  |
| 3ZA-3ZZ | Poland (Republic of) |  |
| 4AA-4CZ | Mexico |  |
| 4DA-4IZ | Philippines (Republic of the) |  |
| 4JA-4KZ | Azerbaijani Republic |  |
| 4LA-4LZ | Georgia |  |
| 4MA-4MZ | Venezuela (Bolivarian Republic of) |  |
| 4OA-4OZ | Montenegro | (WRC-07) |
| 4PA-4SZ | Sri Lanka (Democratic Socialist Republic of) |  |
| 4TA-4TZ | Peru |  |
| *4UA-4UZ | United Nations |  |
| 4VA-4VZ | Haiti (Republic of) |  |
| 4WA-4WZ | Timor-Leste (Democratic Republic of) | (WRC-03) |
| 4XA-4XZ | Israel (State of) |  |
| *4YA-4YZ | International Civil Aviation Organization |  |
| 4ZA-4ZZ | Israel (State of) |  |
| 5AA-5AZ | Libya |  |
| 5BA-5BZ | Cyprus (Republic of) |  |
| 5CA-5GZ | Morocco (Kingdom of) |  |
| 5HA-5IZ | Tanzania (United Republic of) |  |
| 5JA-5KZ | Colombia (Republic of) |  |
| 5LA-5MZ | Liberia (Republic of) |  |
| 5NA-5OZ | Nigeria (Federal Republic of) |  |
| 5PA-5QZ | Denmark |  |
| 5RA-5SZ | Madagascar (Republic of) |  |
| 5TA-5TZ | Mauritania (Islamic Republic of) |  |
| 5UA-5UZ | Niger (Republic of the) |  |
| 5VA-5VZ | Togolese Republic |  |
| 5WA-5WZ | Samoa (Independent State of) |  |
| 5XA-5XZ | Uganda (Republic of) |  |
| 5YA-5ZZ | Kenya (Republic of) |  |

| Call sign series | Allocated to |
| --- | --- |
| 6AA-6BZ | Egypt (Arab Republic of) |
| 6CA-6CZ | Syrian Arab Republic |
| 6DA-6JZ | Mexico |
| 6KA-6NZ | Korea (Republic of) |
| 6OA-6OZ | Somali Democratic Republic |
| 6PA-6SZ | Pakistan (Islamic Republic of) |
| 6TA-6UZ | Sudan (Republic of the) |
| 6VA-6WZ | Senegal (Republic of) |
| 6XA-6XZ | Madagascar (Republic of) |
| 6YA-6YZ | Jamaica |
| 6ZA-6ZZ | Liberia (Republic of) |
| 7AA-7IZ | Indonesia (Republic of) |
| 7JA-7NZ | Japan |
| 7OA-7OZ | Yemen (Republic of) |
| 7PA-7PZ | Lesotho (Kingdom of) |
| 7QA-7QZ | Malawi |
| 7RA-7RZ | Algeria (People’s Democratic Republic of) |
| 7SA-7SZ | Sweden |
| 7TA-7YZ | Algeria (People’s Democratic Republic of) |
| 7ZA-7ZZ | Saudi Arabia (Kingdom of) |
| 8AA-8IZ | Indonesia (Republic of) |
| 8JA-8NZ | Japan |
| 8OA-8OZ | Botswana (Republic of) |
| 8PA-8PZ | Barbados |
| 8QA-8QZ | Maldives (Republic of) |
| 8RA-8RZ | Guyana |
| 8SA-8SZ | Sweden |
| 8TA-8YZ | India (Republic of) |
| 8ZA-8ZZ | Saudi Arabia (Kingdom of) |
| 9AA-9AZ | Croatia (Republic of) |
| 9BA-9DZ | Iran (Islamic Republic of) |
| 9EA-9FZ | Ethiopia (Federal Democratic Republic of) |
| 9GA-9GZ | Ghana |
| 9HA-9HZ | Malta |
| 9IA-9JZ | Zambia (Republic of) |
| 9KA-9KZ | Kuwait (State of) |
| 9LA-9LZ | Sierra Leone |
| 9MA-9MZ | Malaysia |
| 9NA-9NZ | Nepal (Federal Democratic Republic of) |
| 9OA-9TZ | Democratic Republic of the Congo |
| 9UA-9UZ | Burundi (Republic of) |
| 9VA-9VZ | Singapore (Republic of) |
| 9WA-9WZ | Malaysia |
| 9XA-9XZ | Rwanda (Republic of) |
| 9YA-9ZZ | Trinidad and Tobago |
| * Series allocated to an international organization. |  |

_______________

The date of this conference has not been finalized.

1 	Examples:

| 0.002 | Hz | = | H002 |  | 6 | kHz | = | 6K00 |  | 1.25 | MHz | = | 1M25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | Hz | = | H100 |  | 12.5 | kHz | = | 12K5 |  | 2 | MHz | = | 2M00 |
| 25.3 | Hz | = | 25H3 |  | 180.4 | kHz | = | 180K |  | 10 | MHz | = | 10M0 |
| 400 | Hz | = | 400H |  | 180.5 | kHz | = | 181K |  | 202 | MHz | = | 202M |
| 2.4 | kHz | = | 2K40 |  | 180.7 | kHz | = | 181K |  | 5.65 | GHz | = | 5G65 |

2 	Emissions where the main carrier is directly modulated by a signal which has been coded into quantized form (e.g. pulse code modulation) should be designated under § 1.2) or 1.3).

3 	This excludes time-division multiplex.

4 	In this context the word “information” does not include information of a constant, unvarying nature such as is provided by standard frequency emissions, continuous wave and pulse radars, etc.

5 	This includes bandwidth expansion techniques.

*	NOTE – In order to achieve maximum intelligibility, it is suggested that administrations encourage the reduction of this tolerance to 20 Hz.

*	NOTE – The single-sideband system adopted for the bands exclusively allocated to HF broadcasting does not require a frequency tolerance less than 10 Hz. The above-mentioned degradation occurs when the ratio of wanted-to-interfering signal is well below the required protection ratio. This remark is equally valid for both double- and single-sideband emissions.

1 	Spurious domain emissions are unwanted emissions at frequencies within the spurious domain.

1 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Terrestrial Services).

2 	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).    (WRC-12)

1 	The coordination under Nos. 9.11A to 9.19 applies only to assignments in frequency bands allocated with equal rights.      (WRC-15)

2 	For the purpose of effecting coordination, an assignment for which the process of obtaining agreement under No. 9.21 has been initiated is considered to be in conformity with No. 11.31 with respect to No. 9.21.

3 	See No. 9.1A concerning the date to be considered as the date of receipt by the Bureau of the information relating to the coordination of a satellite network or the notification of a frequency assignment.      (WRC-15)

4 	The associated space network characteristics must have been communicated to the Bureau under No. 9.30 or under § 4.1.3/4.2.6 of Article 4 of Appendix 30 or § 4.1.3/4.2.6 of Article 4 of Appendix 30A.     (WRC-2000)

*	These provisions apply only to the MSS.

**	Note by the Secretariat: Edition of 1990, revised in 1994.

1	When p is a small percentage of the time, in the range 0.001% to 1.0%, the interference is referred to as “short-term”; if p ≥ 20%, it is referred to as “long-term” (see § 1.5.3).

2	If the earth station antenna has a wide beamwidth, the method can still be used to determine the propagation mode (2) contour. However, the fact that the antenna beam may be wider than the rain cell and hence not actually fully filled with hydrometeors will mean that the interference potential may be slightly overestimated.

3	See equation (82).

4	While some fixed satellite systems transmit to fixed earth stations operating at unspecified locations within a service area defined by an administration, methods for determining the coordination areas are specified only for individual sites. To minimize the number of individual earth stations requiring detailed coordination in these cases, administrations may wish to develop bilateral agreements based on distances, calculated in accordance with Recommendation ITU-R SM.1448, extended from the periphery of a service area.

5	These additional areas may be declared as coastal Zone A1 areas by administrations for inclusion in the ITU Digital World Map (IDWM).

6	The same procedures are also used to develop supplementary and auxiliary contours (see Annex 6).

7	While no site shielding can be assumed for the receiving earth station, any site shielding that may exist at the transmitting earth station is considered by taking into account the horizon elevation angle in accordance with § 1 of Annex 1.

8	This procedure does not apply for the case of an earth station sharing a frequency band with other earth stations operating in the opposite direction of transmission, as for that specific case the propagation mode (2) contour is based on a geometric construction.

9	The method requires the antenna pattern to be monotonic in terms of the reduction in gain either side of the main beam axis.

1 	A method for calculation of the topocentric angular separation is given in Annex I.

2 	A method for calculation of the free-space transmission loss is given in Annex II.

3 	Values other than 6% are used in the application of Appendix 30 and Appendix 30A.

4 	In cases where  is not given, it may be estimated from the expression 20 log  ≈ Gmax − 7.7, where Gmax is the main lobe antenna gain (dBi).

1	The class of emission shall contain the basic characteristics listed in Appendix 1. If any characteristic cannot be determined, indicate the unknown symbol with a dash. However, if a station is not able to identify unambiguously whether the modulation is frequency or phase modulation, indicate frequency modulation (F).

2	When measurements are not available, signal strengths according to the QSA scale should be provided.

3	See the most recent version of Recommendation ITU-R M.1172.     (WRC-07)

4	See footnote 1.

5	See footnote 3.

6	See footnote 2.

*	Note by the Secretariat: This Resolution was revised by WRC-07 and WRC-15.

1	See Note 21 of Appendix 2.

1 	See Note 21 of Appendix 2.

1	The syllables to be emphasized are underlined.

2	Each syllable should be equally emphasized.

* 	Note by the Secretariat: Annex 1 contains the entire text of Appendix 17     (REV.WRC-07)

1	Within the non-shaded boxes.

1	Within the non-shaded boxes.

*	This revision contains an up-to-date version of Appendix, reflecting all amendments to Section II and the Table of allotments resulting from the application of the procedure of Section I, up to and including 10 May 2016, as well as those amendments, which resulted from geopolitical changes that occurred up to and including that date.

1	The Plan includes additions, modifications and deletions of allotments in the channels of the Frequency Allotment Plan adopted by the WMARC-74, resulting from the application of the relevant plan updating procedures up to and including 10 May 2016, as well as those amendments, which resulted from geopolitical changes that occurred up to and including that date.

2	The meaning of the symbols is given in Tables “Areas” and “Standard Defined Areas” of the Preface to the BR IFIC.

3	ADD	This allotment has been entered in the Plan as a result of the application of the procedure of Section I of this Appendix.

*	This revision contains an up-to-date version of Part III, reflecting all amendments to Part III resulting from the application of the procedures of Part V, up to and including 10 May 2016, as well as those amendments, which resulted from geopolitical changes that occurred up to and including that date.

*	Note by the Secretariat: This edition of Appendix 27 incorporates editorial amendments to the Appendix 27 Aer2 as adopted by the WARC-Aer2.

The references in Appendix 27 now conform to the new numbering scheme of the Radio Regulations. In addition, the text of Appendix 27 contains updated definitions of the relevant aeronautical areas conforming with the new geographical situation reflecting the political changes since 1979. It also contains updated references to the classes of emissions in accordance with Article 2.      (WRC-03)

1 	27/9.1	The type of communication referred to in 27/9 may be regulated by administrations.

* 	Note by the Secretariat: The relevant Article in the Radio Regulations is now Article 6 entitled “Special Agreements”.

2 	27/18.1	To calculate the assigned frequency from a carrier (reference) frequency given in the table, reference should be made to Nos. 27/75, 27/77 and 27/78.

*	A3E and H3E to be used only on 3 023 kHz and 5 680 kHz.

** 	A1A, A1B and F1B are permitted provided they do not cause harmful interference to the classes of emission H2B, J3E, J7B and JXX. In addition, AlA, A1B and FlB emissions shall be in accordance with the provisions in Nos. 27/70 to 27/74 and care should be taken to place these emissions at or near the centre of the channel. However, a modulating audio frequency is permitted with single sideband transmitters, where the carrier is suppressed in accordance with No. 27/69.

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position. See also Annex 7 for the orbital limitations.     (WRC-2000)

1	The Regions 1 and 3 List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

* 	Note by the Secretariat: This Resolution was revised by WRC-12 and WRC-15.

*	Note by the Secretariat: This Resolution was abrogated by WRC-03.

1bis	The time-limit is established at the time when the request is received under § 2A.1.4.     (WRC-15)

2	Such stations may also be used for transmissions in the fixed-satellite service (space-to-Earth) in accordance with No. 5.492.

3 	The provisions of Resolution 49 (Rev.WRC-15) apply.     (WRC-15)

4	Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.     (WRC-03)

5	The provisions of Resolution 533 (Rev.WRC-2000)* apply.     (WRC-03)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

6 	For a launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as from 5 July 2003.     (WRC-03)

7	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

8 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

9 	For the definition of EPM, see § 3.4 of Annex 5.     (WRC-03)

10 	In such a case, § 4.1.18 does not apply.

11 	For assignments using analogue modulation, the intention not to employ energy dispersal in accordance with § 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article.

12 	Or under Resolution 33 (Rev.WRC-97)* for assignments for which the API or the request for coordination has been received by the Bureau prior to 1 January 1999.

*	Note by the Secretariat: This Resolution was revised by WRC-03 and WRC-15.

13	Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.     (WRC-03)

14	The provisions of Resolution 533 (Rev.WRC-2000)** apply.     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-12.

15 	For a launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as from 5 July 2003.     (WRC-03)

16	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.      (WRC-07)

17 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

18	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.6 and the corresponding entries in the Master Register under § 5.2.2, 5.2.2.1, 5.2.2.2 or 5.2.6, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received. See also Resolution 905 (WRC-07)*.     (WRC-07)

* 	Note by the Secretariat: This Resolution was abrogated by WRC-12.

19	A frequency assignment may be notified by one administration acting on behalf of a group of named administrations. Any further notice (modification or deletion) relating to that assignment shall, in the absence of information to the contrary, be regarded as having been submitted on behalf of the entire group.     (WRC-03)

20	Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned or for including assignments in the Regions 1 and 3 List in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Rev.WRC-03) and § B of Annex 7.     (WRC-03)

* 	Note by the Secretariat: This Resolution was revised by WRC-12 and WRC-15.

* 	Note by the Secretariat: This Resolution was revised by WRC-12 and WRC-15.

20bis The date of bringing back into use of a frequency assignment to a space station in the geostationary-satellite orbit shall be the commencement of the 90-day period defined below. A frequency assignment to a space station in the geostationary-satellite orbit shall be considered as having been brought back into use when a space station in the geostationary-satellite orbit with the capability of transmitting or receiving that frequency assignment has been deployed and maintained at the notified orbital position for a continuous period of 90 days. The notifying administration shall inform the Bureau within 30 days from the end of the 90-day period. Resolution 40 (WRC-15) shall apply.      (WRC-15)

21 	These procedures do not replace the procedures prescribed for terrestrial stations in Articles 9 and 11.

22 	These provisions do not replace the procedures prescribed in Articles 9 and 11 when stations other than those in the broadcasting-satellite service subject to a Plan are involved.     (WRC-03)

23 	The provisions of Resolution 33 (Rev.WRC-97)* are applicable to space stations in the broadcasting-satellite service for which the advance publication information or the request for coordination has been received by the Bureau prior to 1 January 1999.

*	Note by the Secretariat:  This Resolution was revised by WRC-03 and WRC-15.

*	Note by the Secretariat: WRC-97 did not review this Article. The subject matter is also dealt with in Articles 13 and 14, which were reviewed by WRC-97.

24 	See Annex 5 (§ 3.2) of this Appendix.

* 	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

*	Note by the Secretariat: This Table was not modified by WRC-97. As such, the references to ETH, TCH, URS and YUG in this column refer to the countries or geographical areas described by these symbols when the Plan was established.

*	Channel 1: 58.2 dBW, channels 3, 5, 7: 59.2 dBW, channels 9, 11, 13: 59.3 dBW, other channels: 59.4 dBW.

**	Channels 2, 4, 6: 63.6 dBW, channels 8, 10, 12: 63.7 dBW.

***	Channels 2, 4, 6: 59.0 dBW, other channels: 59.1 dBW.

* 	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

25 	With respect to this Annex, except for Section 2, the limits relate to the power flux-density which would be obtained assuming free-space propagation conditions.

With respect to Section 2 of this Annex, the limit specified relates to the overall equivalent protection margin calculated in accordance with § 2.2.4 of Annex 5.

26 	(SUP - WRC-15)

27 	For the definition of the equivalent protection margin, see § 3.4 of Annex 5.

28 	For the definition of the overall equivalent protection margin, see § 1.11 of Annex 5.

29 	See § 3.18 of Annex 5.

30	In the band 12.5-12.7 GHz in Region 1, these limits are applicable only to the territory of administrations mentioned in Nos. 5.494 and 5.496.

31 	See Resolution 34*.

* 	Note by the Secretariat:  This Resolution was revised by WRC-03 and WRC-15.

32 	Including assignments operating under No. 5.485.

33 	(SUP - WRC-15)

34 	In revising this Annex at WRC-97 and at WRC-2000, no changes have been made to the technical data applicable to the Region 2 Plan. However, for all three Regions, it should be noted that some of the parameters of networks proposed as modifications to the Region 2 Plan and the Regions 1 and 3 List may differ from the technical data presented herein.     (WRC-2000)

35 	There are a total of five overall carrier-to-interference ratios used in the analysis of the Plan for the broadcasting-satellite service in Region 2, namely, co-channel, upper and lower adjacent channels, and upper and lower second adjacent channels. In Regions 1 and 3, three ratios are normally used, namely, co-channel and upper and lower adjacent channels. However, see the footnote to the definition of M4 and M5 in § 1.11 of this Annex.

36 	For calculation of overall equivalent protection margin for Regions 1 and 3, as defined at WARC Orb-88, see alternative formula in § 1.12 to Annex 3 of Appendix 30A.

37 	M4 and M5 are applicable only for Region 2.     (WRC-2000)

*	Note by the Secretariat: This Recommendation was suppressed by the Radiocommunication Assembly (Geneva, 2003).

38 	Protection masks for verifying that this provision is met are not yet fully defined in existing ITU-R Recommendations. Recommendations for interference between analogue and digital signals are still under development. In absence of criteria to evaluate interference, the Bureau will use the worst-case approach as adopted by the Radio Regulations Board.

39 	These protection ratio values were used for the assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

40 	The equivalent protection margin M is given in dB by the formula:

M    −10 log (10−M1/10  +  10−M2/10  +  10−M3/10)

where M1 is the value (dB) of the protection margin for the same channel. This is defined in the following expression where the powers are evaluated at the receiver input:

| wanted power sum of the co-channel interfering powers | (dB) – co-channel protection ration (dB) |
| --- | --- |

M2 and M3 are the values (dB) of the upper and lower adjacent-channel protection margins respectively.

The definition of the adjacent-channel protection margin is similar to that for the co-channel case except that the adjacent-channel protection ratio and the sum of the interfering powers due to emissions in the adjacent channel are considered.

41 	These protection ratio values were used for the assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau between 27 October 1997 and 12 May 2000.     (WRC-2000)

42 	These protection ratio values were used for protection of digital and analogue assignments from analogue emissions.     (WRC-2000)

43	The overall protection margin calculation method used is based on the first formula in § 1.12 of Annex 3 to Appendix 30A.

44	For analogue assignments, the protection ratios adopted by WRC-97 were used (24 dB co-channel and 16 dB adjacent channel).     (WRC-2000)

45	Annex 3 of this Recommendation may be applied only in compatibility analysis for bilateral coordination between administrations.     (WRC-03)

46	The definitions in § 1.7, 1.8, 1.9, 1.10 and 1.11 of this Annex apply to these calculations.     (WRC-03)

47	See Annex 6 for the protection ratio template for interference between TV/FM signals in Regions 1 and 3.

48 	These values are still used for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

49 	This antenna pattern is used in the broadcasting-satellite service Plan for Regions 1 and 3 for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

50 	For France, Denmark and some of the United Kingdom requirements which use 625-line standards with greater video bandwidth, the channels shown in the Plan have a necessary bandwidth of 27 MHz. This is indicated by an appropriate symbol in the Plan.

51 	In the original 1977 broadcasting-satellite service Plan for Regions 1 and 3, the angular rotation of a transmitting beam about its axis must not exceed a limit of ±2°. This limit is still applied for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

52	These values are still used for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

53	Sections 1 and 2 are applicable when the services of Regions 1 or 3 are involved. Section 3 is applicable to all Regions.

54 	Impairment grade on a 5-point scale as defined in Recommendation ITU-R BT.500-7.

*	Note by the Secretariat:  See Report ITU-R BO.631.

*	The expression “frequency assignment to a space station”, wherever it appears in this Appendix, shall be understood to refer to a frequency assignment associated with a given orbital position.     (WRC-03)

1	The Regions 1 and 3 feeder-link List of additional uses is annexed to the Master International Frequency Register (see Resolution 542 (WRC-2000)**).     (WRC-03)

**	Note by the Secretariat: This Resolution was abrogated by WRC-03.

2	This use of the band 14.5-14.8 GHz is reserved for countries outside Europe.

Note by the Secretariat: Reference to an Article with the number in roman is referring to an Article in this Appendix.

3	This use of the band 14.5-14.8 GHz is reserved for countries outside Europe.

*	Note by the Secretariat: This Resolution was abrogated by WRC-03.

3bis	The time-limit is established at the time when the request is received under § 2A.1.4.      (WRC-15)

* 	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

4	Agreement with administrations having a frequency assignment in the bands 14.5-14.8 GHz or 17.7-18.1 GHz to a terrestrial station, or having a frequency assignment in the band 17.7-18.1 GHz to an earth station in the fixed-satellite service (space-to-Earth), or having a frequency assignment in the band 17.3-17.8 GHz in the broadcasting-satellite service shall be sought under No. 9.17, No. 9.17A or No. 9.19, respectively.

5	Coordination under Nos. 9.17 or 9.17A is not required for an earth station of an administration on the territory of which this earth station is located and for which the procedures of former § 4.2.1.2 and 4.2.1.3 of Appendix 30A (WRC-97) have been successfully applied by that administration before 3 June 2000 in respect of terrestrial stations or earth stations operating in the opposite direction of transmission.     (WRC-03)

6	Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networds or systems.     (WRC-03)

7	The provisions of Resolution 533 (Rev.WRC-2000)* apply.     (WRC-03)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

8	For a launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as from 5 July 2003.     (WRC-03)

9	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

10	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

11	For the definition of the EPM, see § 1.7 of Annex 3.     (WRC-03)

12	In such a case, § 4.1.18 does not apply.

*	Note by the Secretariat: This Resolution was revised by WRC-12 and WRC-15.

13	Agreement with administrations having a frequency assignment in the bands 17.7-17.8 GHz to a terrestrial station or to an earth station in the fixed-satellite service (space-to-Earth) shall be sought under No. 9.17 or No. 9.17A, respectively.

14	Coordination under No. 9.17 or 9.17A is not required for an earth station of an administration on the territory of which this earth station is located and for which the procedures of former § 4.2.3.2 and 4.2.3.3 of Appendix 30A (WRC-97) have been successfully applied by that administration before 3 June 2000 in respect of terrestrial stations or earth stations operating in the opposite direction of transmission.     (WRC-03)

15	Agreement with administrations having a frequency assignment in the band 17.3-17.8 GHz to an earth station in the broadcasting-satellite service shall be sought under No. 9.19.

16	Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.     (WRC-03)

17	The provisions of Resolution 533 (Rev.WRC-2000)* apply.     (WRC-03)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

18	For a launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as from 5 July 2003.     (WRC-03)

19	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

20	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received.     (WRC-07)

21	Notification of assignments to transmitting feeder-link earth stations included in the Region 2 feeder-link Plan after 2 June 2000, or included in the feeder-link List, following successful application of Article 4, shall be effected applying the provisions of Article 11 following completion of the procedure of Article 9.     (WRC-03)

22	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 5.1.10 and the corresponding entries in the Master Register under § 5.2.2, 5.2.2.1 or 5.2.2.2, as appropriate, and the corresponding entries included in the Plan on and after 3 June 2000 or in the List, as appropriate, after informing the administration concerned. The Bureau shall inform all administrations of such action. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482 unless the payment has already been received. See also Resolution 905 (WRC-07)*.      (WRC-07)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

23	A frequency assignment to a space station or typical earth station in the satellite network may be notified by one administration acting on behalf of a group of named administrations. Any further notice (modification or deletion) relating to that assignment shall, in the absence of information to the contrary, be regarded as having been submitted on behalf of the entire group.     (WRC-03)

24	In cases where assignments from the WRC-97 Plans without Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the coordination status afforded by the WRC-97 Plans shall be preserved.

In cases where assignments from the WRC-97 Plans with Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the compatibility shall be reassessed using the revised criteria and methodology in force and the Remarks of the WRC-97 Plans assignment shall either be maintained or reduced on the basis of the results of this analysis.     (WRC-03)

*	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

24bis  The date of bringing back into use of a frequency assignment to a space station in the geostationary-satellite orbit shall be the commencement of the 90-day period defined below. A frequency assignment to a space station in the geostationary-satellite orbit shall be considered as having been brought back into use when a space station in the geostationary-satellite orbit with the capability of transmitting or receiving that frequency assignment has been deployed and maintained at the notified orbital position for a continuous period of 90 days. The notifying administration shall inform the Bureau within 30 days from the end of the 90-day period. Resolution 40 (WRC-15) shall apply.     (WRC-15)

25	Only assignments included in the Region 2 feeder-link Plan before 3 June 2000 shall be taken into account.     (WRC-03)

26	These procedures do not replace the procedures prescribed for terrestrial stations in Articles 9 and 11.     (WRC-03)

27	In the case of Regions 1 and 3, the feeder-link earth-station power to be taken into account is obtained by adding the values specified in Columns 11 and 12 of the feeder-link Plan.

28	These provisions do not replace the procedures prescribed in Articles 9 and 11 when stations other than those for feeder links in the broadcasting-satellite service subject to a Plan are involved.     (WRC-03)

29	The provisions of Resolution 33 (Rev.WRC-97)* are applicable to space stations in the broadcasting-satellite service for which the advance publication information or the request for coordination has been received by the Bureau prior to 1 January 1999.

*	Note by the Secretariat: This Resolution was revised by WRC-03 and WRC-15.

*	Note by the Secretariat: WRC-97 did not review this Article. The subject matter is also dealt with in Articles 13 and 14, which were reviewed by WRC-97.

30	See Annex 3 (§ 4.8) to this Appendix.

31	The location of earth stations, together with the antenna characteristics and elevation angle of the horizon, are given as an annex to this Plan, and will be published when the Plan is republished in accordance with § 4.2.25.2 of Article 4.

*	Note by the Secretariat: Since the orbital positions of these countries were changed by WRC-97, this paragraph might need to be revised.

*	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

32	(SUP - WRC-15)

33	With respect to § 3 the limit specified relates to the overall equivalent protection margin calculated in accordance with § 1.12 of Annex 3.

*	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

34	For the definition of the overall equivalent protection margin, see § 1.11 of Annex 5 to Appendix 30.

*	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

35	For the definition of the equivalent protection margin, see § 1.7 of Annex 3.

* 	Note by the Secretariat:  This Resolution was revised by WRC-12 and WRC-15.

36	In revising this Annex at WRC-97 and at WRC-2000, no changes were made to the technical data applicable to the Region 2 feeder-link Plan. However, for all three Regions it should be noted that some of the parameters of networks proposed as modifications to the Region 2 feeder-link Plan and the Regions 1 and 3 feeder-link Lists may differ from the technical data presented herein.     (WRC-2000)

37	This quantity is used in the alternative formula for the overall equivalent protection margin given in § 1.12. However, in certain cases (e.g. when the channel spacing and/or bandwidth are different from the values given in § 3.5 and 3.8 of Annex 5 to Appendix 30) the Bureau will use the worst-case approach until a relevant ITU-R Recommendation is incorporated in this Annex by reference.     (WRC-2000)

38	In Region 2, there are a total of five overall C/I ratios used in the analysis of the Plan, namely, co-channel, upper and lower adjacent channels and upper and lower second adjacent channels. In Regions 1 and 3, three ratios are used, namely, co-channel and upper and lower adjacent channels.

39	This formula is also used to calculate the overall equivalent protection margin of the assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

40	M4 and M5 are applicable only for Region 2.     (WRC-2000)

41	These protection ratio values were used for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

42	These protection ratio values were used for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau between 27 October 1997 and 12 May 2000.     (WRC-2000)

43	These protection ratio values were used for protection of digital and analogue assignments from analogue emissions.     (WRC-2000)

44	For analogue assignments, the protection ratios of WRC-97 (30 dB co-channel, 22 dB adjacent channel) were used.     (WRC-2000)

45	Annex 3 of this Recommendation may be applied only in compatibility analysis for bilateral coordination between administrations.     (WRC-03)

46	This antenna pattern is used in the revision of the Regions 1 and 3 Plan for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

47	See footnote 46.

48	These system temperature values are still used for assignments notified, which are in conformity with this Appendix, brought into use, and for which the date of bringing into use has been confirmed to the Bureau before 27 October 1997.

49	This margin has to be shared between power control effects and depolarization compensation effects when both are involved (see § 3.11).

1	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 6.7 and/or 6.23 and the corresponding entries in the List under § 6.23 and/or 6.25, as appropriate, and reinstate any allotments back into the Plan after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482, unless the payment has already been received. See also Resolution 905 (WRC-07)*.

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

2	Resolution 49 (Rev.WRC-15) applies.      (WRC-15)

3	Whenever, under § 6.1, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own allotments or assignments.

4	Submissions may include conversion of the 6/4 GHz or the 13/10-11 GHz portion (both uplink and downlink) of an allotment into an assignment provided that the orbital location of the assignment is the same as the unconverted portion of the allotment.

5	Submissions for additional systems may include use of only space-to-Earth or only Earth-to-space links.

6	The “other provisions” shall be identified and included in the Rules of Procedure.

7	The “other provisions” shall be identified and included in the Rules of Procedure.

8	In the case of a conversion of an allotment into an assignment, the part of the allotment that has been converted shall be removed from the Plan and the reference situation shall be updated.

* 	This procedure may be applied by Palestine to obtain assignments in the Appendix 30B Plan. Such assignments are for exclusive use by Palestine, in accordance with the Israeli-Palestinian Interim Agreement of 28 September 1995, Resolution 741 of the Council notwithstanding, and Resolution 99 (Rev. Antalya, 2006) of the Plenipotentiary Conference. This is without prejudice of future agreements between the State of Israel and Palestine.

** 	This procedure may be applied by Palestine to obtain an allotment in the Appendix 30B Plan. Such allotment is for exclusive use by Palestine, in accordance with the Israeli-Palestinian Interim Agreement of 28 September 1995, Resolution 741 of the Council notwithstanding, and Resolution 99 (Rev. Antalya, 2006) of the Plenipotentiary Conference. This is without prejudice of future agreements between the State of Israel and Palestine.

9	(SUP - WRC-15)

10	The “other provisions” shall be identified and included in the Rules of Procedure.

11	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication specified in § 8.5 and 8.12 and the corresponding entries in the Master Register under § 8.11, after informing the administration concerned. The Bureau shall inform all administrations of such action and that any resubmitted notice shall be considered to be a new notice. The Bureau shall send a reminder to the notifying administration not later than two months prior to the deadline for the payment in accordance with the above-mentioned Council Decision 482, unless the payment has already been received. See also Resolution 905 (WRC-07)*.     (WRC-07)

*	Note by the Secretariat: This Resolution was abrogated by WRC-12.

12	Resolution 49 (Rev.WRC-15) applies.     (WRC-15)

13	The “other provisions” shall be identified and included in the Rules of Procedure.     (WRC-03)

14	When an administration notifies any assignment with characteristics different from those entered in the List through successful application of Article 6 of Appendix 30B, the Bureau shall undertake calculation to determine if the proposed new characteristics increase the interference level caused to other allotments and assignments in the Plan and List. The increase of the interference due to characteristics different from those entered in the List will be checked by comparing the C/I ratios of these other allotments and assignments, which result from the use of the proposed new characteristics of the subject assignment on the one hand, and those obtained with the characteristics of the subject assignment in the List, on the other hand. This C/I calculation is performed under the same technical assumptions and conditions.     (WRC-07)

14bis	The date of bringing back into use of a frequency assignment to a space station in the geostationary-satellite orbit shall be the date of the commencement of the 90-day period defined below. A frequency assignment to a space station in the geostationary-satellite orbit shall be considered as having been brought back into use when a space station in the geostationary-satellite orbit with the capability of transmitting or receiving that frequency assignment has been deployed and maintained at the notified orbital position for a continuous period of 90 days. The notifying administration shall inform the Bureau within 30 days from the end of the 90-day period. Resolution 40 (WRC-15) shall apply.       (WRC-15)

*	Figure 1 represents patterns for some combinations of Bmin and φ0.     (WRC-07)

15	These limits shall not apply to assignments recorded in the List before 17 November 2007.

16	Including a computational precision of 0.05 dB.

17	C/Nu is calculated as in Appendix 2 to this Annex.

18	Excluding values accepted in accordance with § 6.15 of Article 6.

19	The reference values within the service area are interpolated from the reference values on the test points.

20	C/Nd is calculated as in Appendix 2 to this Annex.

21	(C/N)t is calculated as in Appendix 2 of this Annex.

22	Inclusive of the 0.05 dB computational precision.