Report ITU-R P.2345-3

(06/2025)

P Series: Radiowave propagation

Propagation model for IF77

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from https://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Reports (Also available online at https://www.itu.int/publ/R-REP/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| TF | Time signals and frequency standards emissions |

Note: This ITU-R Report was approved in English by the Study Group under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2025

ITU 2025

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

REPORT  ITU-R  P.2345-3

Propagation model for IF77

(2015-2016-2020-2025)

Scope

This Report describes the IF77 propagation model, as described by Gierhart and Johnson [1]. The IF77 model predicts the basic transmission loss in the frequency range 125 MHz – 20 GHz for aeronautical and satellite services. While the propagation model in Recommendation ITU-R P.528-5, et seq. also predicts the basic transmission loss for aeronautical and satellite services, the propagation model in Recommendation ITU-R P.528-5, et seq. is different than IF77; for example, the propagation model in Recommendation ITU-R P.528-5, et seq. assumes a smooth earth and does not consider inter alia precipitation and scintillation fading. The basic underlying methods presented in this Report, however, are informative in understanding Recommendation ITU-R P.528-5, et seq.

TABLE OF CONTENTS

Page

Policy on Intellectual Property Right (IPR)		ii

1	Introduction		3

2	Method description		3

2.1	Transmission loss		4

2.2	Power available		6

2.3	Power density		6

2.4	Desired-to-undesired signal ratio		6

3	Horizon geometry		7

3.1	Effective earth radius		7

4	High antenna height		8

4.1	Smooth earth horizons		8

4.2	Facility radio horizon		10

4.3	Higher antenna (aircraft or satellite) horizon		14

5	Diffraction region		15

5.1	Rounded earth diffraction		15

5.2	Knife-edge diffraction		19

5.3	Diffraction attenuation		20

6	Line-of-sight region		21

6.1	Two-ray path length geometry		21

6.2	Effective reflection coefficient		24

6.3	Line-of-sight transition distance, d0		31

6.4	Line-of-sight attenuation, ALoS		32

7	Scattering region		33

8	Terrain attenuation		36

9	Free space loss		36

10	Atmospheric absorption		37

10.1	Surface absorption rates		39

11	Variability		41

11.1	Variability due to long-term fading		41

11.2	Variation due to multipath		46

11.3	Rain attenuation		49

11.4	Variation due to scintillation		54

11.5	Mixing distributions		55

Attachment 1		56

1	Ray tracing		56

2	Counterpoise or ground plane		58

2.1	Effective reflection coefficient		58

2.2	Counterpoise factors		58

3	Cumulative distribution		60

Attachment 2 – Abbreviations, acronyms and symbols		61

Attachment 3 – Data sets from Figs 13 and 16		70

Bibliography		72

# 1	Introduction

In the present time of frequency spectrum scarcity, sharing frequency bands has become ubiquitous in all aspects of wireless usage. The key to sharing frequency bands is performing good frequency sharing studies and the key to performing good frequency sharing studies is to use the appropriate propagation method.

While aeronautical and satellite links have many similar characteristics to terrestrial communication links, there are some additional characteristics of the aeronautical/satellite wireless channel not covered in propagation prediction methods for terrestrial systems. The IF77 propagation model has been used successfully for several years for propagation prediction of aeronautical and satellite links.

The organization of this Report is as follows. Section 2 covers the method description and includes calculations for the major outputs of the method. Horizon geometry is defined in §§ 3 and 4. Sections 5, 6 and 7 cover the diffraction region, the line-of-sight (LoS) region and the scatter region respectively. The diffraction region appears first since parameters described in that section define region boundaries. The rest of the sections cover terrain attenuation (§ 8), free space loss (§ 9), atmospheric absorption (§ 10) and variability (§ 11). This material is based on three reports [1], [2], [3] produced by the Institute for Telecommunication Sciences, ITS.

# 2	Method description

IF77 calculates propagation parameters for ground-ground, ground-air, ground-satellite, air-air and air-satellite links. Calculations of these links also cover air-ground, satellite-ground, air-air (where the higher antenna is the transmitting antenna) and satellite-air by reciprocity. This method always defines the lower antenna as the transmitting antenna designated as antenna 1 and all the parameters associated with antenna 1 have a designation of 1.The higher antenna with its associated parameters have the designation 2. Several parameters have multiple designations separated by a comma (i.e. h1, 2 or GET.R). This applies to multiple parameters (h1 and h2 or GET and GER). Equations with this type of parameter represent multiple equations, one for each number designation.

This method is a propagation prediction method for aeronautical and satellite links. There are restrictions on this model. It covers the frequency range of 0.1 to 20 GHz for antenna heights greater than 0.5 m. The radio horizon of the higher antenna, antenna 2, has a lower elevation than the higher antenna. It is, also, either common with the horizon of the lower antenna or is a smooth earth horizon with the same elevation as the effective reflective plane of the lower antenna.

Attenuation verses distance calculated for this model for three regions, LoS, diffraction, and scattering includes transition regions where the parameters are blended together. This method includes allowances for:

•	average ray bending;

•	horizon effects;

•	long-term power fading;

•	antenna pattern (elevation only) at each terminal;

•	surface reflection multipath;

•	tropospheric multipath;

•	atmospheric absorption;

•	ionospheric scintillation;

•	rain attenuation;

•	sea state;

•	a divergence factor;

•	very high antennas;

•	antenna tracking options.

IF77 addresses all these elements to provide a tool for frequency planning and sharing studies. The IF77 propagation model uses a combination of theoretical and empirical techniques to calculate the transmission loss of a defined system. The outputs of this method include transmission loss, power availability, power density and a desired-to-undesired ratio.

This section will present the equations to calculate the various outputs. The processes to calculate the parameters in these equations follow in the later sections.

## 2.1	Transmission loss

Transmission loss is the ratio of power radiated to power available at the receiving antenna. This loss does not include circuit losses of the receiving system other than the radiation resistance of the receiving antenna. The calculation for transmission loss, L(q), not exceeded during a fraction of time q is:

Lq= Lb0.5+ Lgp- GET- GER- YΣq       dB	(1)

Lb (0.5) is the median basic transmission loss and Lgp is the path antenna gain loss. Lgp is zero for this method which is valid when (a) the transmitting and receiving antennas have the same polarization which is true by definition in this method and (b) the maximum antenna gain is less than 50 dB. The values GET and GER are the free space antenna gains for the transmitting and receiving antennas respectively and Y∑(q) is the total variability of the channel. The median basic transmission loss calculation is:

Lb0.5= Lbr+ AY+ Aa    dB	(2)

The value Lbr is a reference level for the basic transmission loss and Aa is loss due to atmospheric absorption. AY is a conditional adjustment factor to prevent available signal powers from exceeding expected free space by an unrealistic amount. This condition can occur when the variability, Y(q) is large and the median basic transmission loss is near the free space level.

The reference for basic transmission loss, Lbr, comes from the free space basic transmission loss, Lbf, from (206), the terrain attenuation, AT, from (205), and a variability adjustment term, Ve (0.5), from (231).

Lbr= Lbf+ AT-Ve0.5         dB	(3)

The free space antenna gains, GET,R, are calculated from:

GET,R= GT,R+ GNT,R     dBi	(4)

GT,R are the main beam maximum free space antenna gains for the transmitting and the receiving antennas respectively. GNT,R give the relative gain for a defined elevation angle for the transmitting and the receiving antennas respectively. These parameters are inputs to the model.

The variability term from (1), Y(q), includes long term power fading, surface reflection multipath, tropospheric multipath, rain attenuation and ionospheric scintillation. The discussion for these terms is in § 11. The median value for Y∑ (q = 0.5) is equal to zero since the median value of basic transmission loss, Lb(0.5), contains the adjustment terms for long term fading.

Figure 1 shows the flow diagram for calculating the transmission loss, L(q). Calculations for the horizon parameters and the diffraction region parameters are at the top of the flow since those parameters are needed to define the region boundaries.

FIGURE 1

Computational flow diagram for transmission loss

## 2.2	Power available

Power available is the power available at the terminals of the receiving antenna under matched conditions with losses in the receiving system neglected. Power available levels exceeded for a fraction of time q, Pa (q), is calculated by:

Pa q=PEIRP+ GR+ GNT+ GNR-Lb0.5+ YΣ q      dBW	(5)

where:

PEIRP= PTR+GT     dBW	(6)

PEIRP is the equivalent isotropically radiated power. PTR is the total power radiated by the transmitting antenna referenced to 1 W. GT,R are the maximum gains of the transmitting and receiving antennas respectively as discussed in § 2.1 and GNT,R are the gains from (4). The variability, Y∑ (q), was discussed in § 2.1 and is calculated in (219). The relative gains GNT and GNR allow for directivity when the antennas are not pointed at each other.

## 2.3	Power density

The power density, SR (q), in decibels relative to 1 watt per square metre exceeded for the fraction of time q uses parameters discussed in §§ 2.1 and 2.2 and the effective area, AI, of an isotropic antenna.

SR q=PEIRP+ GNT- Lb 0.5+ YΣ q- AI   dBWsq m	(7)

AI is calculated by:

AI=10 log10λm24π     dB sq m	(8)

where λm is the wavelength in metres for the defined frequency in MHz.

λm= 299.7925f       m	(9)

## 2.4	Desired-to-undesired signal ratio

Desired-to-undesired signal ratios available for a fraction of time q, D/U (q), at the terminals of a lossless receiving antenna are calculated by:

DU q= DU 0.5+ YDU q        dB	(10)

where

DU 0.5= Pa (0.5)Desired-  Pa (0.5)Undesired   dB	(11)

and

YDU q= ± YΣqDesired2+ YΣ1- qUndesired2      dB	(12)

– for q ≥ 0.5

+ otherwise

Use (5) to calculate Pa (0.5) for both desired and undesired signals with parameters for desired and undesired signals respectively and with Y∑ (0.5) = 0.

# 3	Horizon geometry

Horizons for both terminals are calculated from the input parameters used to determine the lower terminal horizon. The higher terminal horizon is either considered to be the same as the lower terminal horizon or as a smooth earth when the lower terminal is shadowed by the smooth earth from the higher terminal.

## 3.1	Effective earth radius

To calculate the effective earth radius, a, adjust the actual earth radius, a0, with the surface refractivity associated with the effective reflecting surface. Starting with the monthly mean surface refractivity, N0, calculate the surface refractivity, Ns, for the reflecting surface height, hr.

Ns=greater of N0exp- 0.1057 hror200           N-units	(13)

Using a0 = 6 370 km, calculate the effective earth radius.

a = a0 1 - 0.04665 exp 0.005577 Ns-1      km	(14)

Figure 2

Sea level surface refractivity from Recommendation ITU-R P.453-10

Both N0 and hr are input parameters. Figure 2 shows the sea level surface refractivity.

# 4	High antenna height

For high antennas (> 3 km), the effective earth radius can overestimate the amount of ray bending. A solution for this is to calculate ray tracing through an exponential atmosphere. The refractivity varies with the height above mean sea level, h, as:

N = Ns exp - logeNsNs+ ∆N h - hr         N-units	(15)

where h and hr are input parameters, Ns is calculated in (13) and

∆N = -7.32 exp 0.005577 Ns    N-units/km	(16)

The ray tracing algorithm calculates the change in the ray through a horizontally stratified atmosphere with layer heights above hr of 0.01, 0.02, 0.05, 0.1, 0.2, 0.305, 0.5, 0.7, 1, 1.524, 2, 3.048, 5, 7, 10, 20, 30.48, 50, 70, 90, 110, 225, 350 and 475 km. Above 475 km, the rays are assumed to be straight relative to the true earth radius a0. The ray tracing algorithm is in Attachment 1.

## 4.1	Smooth earth horizons

Antenna heights input to the method have relationships as shown in Fig. 3. Antenna heights above the effective reflection surface are calculated by:

hr1= hsl+ hsml- hr    km	(17)

h1,2= hr1,2+ hr             km	(18)

Antenna heights h1,2 are the heights above mean sea level for antennas 1 and 2 and hr1,2 are the antenna heights above the reflective surface for antennas 1 and 2 respectively. The height of antenna 1 above a counterpoise or ground plane, if present, is calculated by:

hfc= hsl- hc       km	(19)

Figure 3

Input antenna heights and surface elevations

For those scenarios where the effective earth geometry overestimates ray bending, effective antenna heights, he1,2 become heights that produce the smooth earth horizon distances from each terminal, dLsR1,2, calculated with the ray tracing algorithm. Figure 4 shows that the effective heights are lower than the actual heights. When determining the smooth earth distances, the initial take-off angle is 0° and the surface refractivity, Ns, is calculated from N0 in (13). After determining the smooth earth distances, dLsR1,2, use those values and from (14) to calculate the differences in the actual antenna heights and the effective antenna heights, ∆he1,2, as follows:

θsR1,2= dLsR1,2a    rad	(20)

he1,2=lesser of hr1,2or0.5 dLsR1,22a   if  θsR1,2 ≤0.1 radasecθsR1,2-1           otherwise      km	(21)

∆he1,2= hr1,2- he1,2       km	(22)

FIGURE 4

Effective height geometry

The final smooth earth distances, dLs1,2, are either the values determined by ray tracing, dLsR1,2, with high antennas or the values calculated by effective earth geometry, dLsE1,2:

dLsE1,2= 2a he1,2    km	(23)

dLs1,2= dLsR1,2  if ∆he1,2 >0dLsE1,2       otherwise   km	(24)

dLs= dLs1+ dLs2   km	(25)

The ray elevation angles, θes1,2, become either the ray elevation angles from ray tracing, θesR1,2, for high antennas or the negative of the ray elevation angles calculated with the final smooth earth distances, θs1,2:

θs1,2= dLs1,2a        rad	(26)

θes1.2= θesR1,2   if ∆he1,2 >0-θs1,2        otherwise    rad	(27)

## 4.2	Facility radio horizon

There are four sets of parameters that can produce the horizon for the lower or facility antenna. These parameters are: a) any two horizon parameters (distance, elevation or elevation angle); b) any one horizon parameter and the terrain parameter ∆h; c) ∆h alone; or d) smooth earth parameters. Figure 5 shows the facility horizon geometry.

For case (a) where at least two horizon parameters are specified, the horizon parameters for distance, dLE1, horizon elevation, hLE1, and horizon elevation angle, θeE1, are related by:

dLE1= ± 2ahLE1-h1+ a2 tan2θeE1 -a tanθeE1         km	(28)

hLE1= h1+ dLE122a+ dLE1tanθeE1          km	(29)

θeE1= tan-1hLE1- h1dLE1- dLE12a       rad	(30)

where a is from (14). The choice of the ± in (28) should produce the smallest positive value for hLE1 in (29).

Figure 5

Facility radio horizon geometry

Note to Fig. 5: Effective earth (straight ray) geometry is illustrated for the facility using a dotted ray. The solid ray illustrates that the horizon ray obtained by ray tracing from the facility horizon to the aircraft yields a smaller distance than would be obtained with effective earth geometry.

One can use the terrain parameter ∆h which is an input parameter and dLs1 from (24) to estimate dLE1 and θeE1 when those horizon parameters are not specified. When using he here, use 0.005 km if he falls below that number.

dh= dLs1exp-0.07 ∆hhe         km	(31)

dLE1= 0.1 dLs1 if dh <0.1 dLs13 dLs1        if dh >3 dLs1dh                     otherwise              km	(32)

θeE1=lesser of 0.5dLs1 1.3 ∆h dLs1dLE1-1-4 he1or                                               0.2094                                                      rad	(33)

where dLs1 is from (24).

In the case of high antennas, however, the accurate calculation of more than one horizon parameter is dependent on ray tracing results. Figure 6 shows the full logic flow diagram for calculating the parameters for the lower or facility horizon. The full method for calculating horizon parameters is:

1	Calculate the horizon parameters using equations (28) through (33), but consider the values to be initial values.

2	Use h1 from equation (18) and he1 from equation (21) to test the values from step 1.

he1 >3 km   and   θeE1 >0 and h1 > hLE1orθeE1 ≤0 and h1 < hLE1  	(34)

When these conditions are true, set the parameters as in Fig. 5 for smooth earth. Otherwise keep the parameter values in step 1.

3	When the test conditions in step 2 are false, use the following values:

dL1=  dLE1                    if  ∆he1 =0use ray tracing otherwise           km	(35)

hL1= hLE1      km	(36)

hLr1= hL1- hr      km	(37)

θe1=  θeE1                      if ∆he1=0use ray tracing otherwise               rad	(38)

For ray tracing in (35) and (38), start at the horizon elevation, hLr1, with a take-off angle of –θL.

θL= θeE1+ dLE1a     rad	(39)

Figure 6

Logic flow to calculate facility horizon parameters

Continue through the ray tracing algorithm until the lower antenna height, hr1, is reached. Then the negative of the arrival angle is θe1 and the great circle distance traversed by the ray is dL1.

The great circle distance from hLE1 to the aircraft altitude h2, dLR2, is calculated via ray tracing using a take-off angle of θL at hLE1. The maximum LoS distance, dML, is calculated by:

dML= dLs1+ dLs2                                        for smooth earthacos-1a+ he1 cosθe1a+ he2- θe1              if ∆he2=0dL1+ dLR2                                                        otherwise               km	(40)

## 4.3	Higher antenna (aircraft or satellite) horizon

In the IF77 the higher antenna horizon is defined as either a common horizon with the lower antenna horizon (as in an obstacle horizon) or as a smooth earth horizon. Figure 7 shows the geometry for calculating the great circle distance from the horizon to the higher antenna.

Using the great circle distance between the lower and the higher antennas, d, which is an input parameter, the great circle horizon distance for the higher antenna, dL2, is calculated by:

dsL= 2a hLr1    km	(41)

dLM= dL1+ dsL+dLs2      km	(42)

dL2= d- dL1     if  dML ≤d ≤ dLMdLs2                               otherwise           km	(43)

The value hLr1 is the height of the obstacle horizon for the lower antenna and dsL is the smooth earth horizon distance for the obstacle. The value a, is from (14), dL1 is from (35) and dLs2 is from (24).

The horizon height for the higher antenna, hL2, is the obstacle height of the lower antenna from the reflecting surface or the smooth earth horizon height.

hLr2= hLr1    if dML <d < dLM0                          otherwise       km	(44)

hL2= hLr2+ hr     km	(45)

where hr is the surface reflection height which is an input parameter. The horizon ray elevation angle is relative to the horizontal at the higher antenna where the angle is positive if it is above the horizontal.

θe2= tan-1hLr2- he2dL2- dL22a          rad	(46)

Figure 7

Geometry for the higher antenna radio horizon

# 5	Diffraction region

The IF77 method assumes that diffraction attenuation, Ad, is linear with distance when other parameters are fixed. To establish the linear relationship, the method determines a diffraction value at two distances. The first distance, dML, is the maximum LoS distance and the second distance, dA, is the shortest beyond-the-horizon distance defined by the facility horizon obstacle and the smooth earth horizon for the receiving antenna. Figure 8 shows the geometry of the paths. Values for both smooth earth diffraction and knife-edge diffraction in each case are calculated and then combined. The first two sections discuss rounded earth diffraction and knife-edge diffraction respectively. The third section shows how to combine the two diffraction pieces.

## 5.1	Rounded earth diffraction

The two paths used in the case of rounded earth diffraction are path F-O-ML and path O-A from Fig. 8.

The following set of equations produce straight-line attenuation values versus distance.

hep1= he1  for path F-O-MLhLr1            for path O-A        km 	(47)

where he1 is from (21) and hLr1 is from (37). The height hep2 equals he2 from (21) for both paths.

Figure 8

Paths used to determine diffraction attenuation

dLp1= dL1                      for path F-O-ML dLO1=dsL                  for  path O-A         km	(48)

dLp2=dML- dL1         for path F-O-MLdLO2= dLs2                for path O-A         km	(49)

The parameters for these equations include dL1 from (35), dsL from (41), dML from (40) and dLs2 from (24). The path parameters for rounded earth diffraction are the attenuation line intercept (Ap), the attenuation path slope (Mp) and the height gain function (Gɦp1,2). For the following equations, the values for the F-O-ML path, AF, MF, and GɦF1,2, will equal Ap, Mp and Gɦ1,2 respectively using hep1 from (47), hep2 equal to he2 from (21) and dLp1,2 values for the F-O-ML path as determined in (48) and (49). The values for the O-A path, AA, MA and GɦA1,2 will equal Ap, Mp and Gɦ1,2 respectively using hep1 from (47), hep2 equal to he2 from (21) and dLp1,2 values for the O-A path as determined in (48) and (49).

Constants for the equations are the effective earth radius (a from (14)), the frequency (f in MHz), the conductivity for the type of surface (σ from Table 1) and the dielectric constant (ε from Table 1).

TABLE 1

Surface types and constants [4]

| Type | Conductivity (mhos/m) | Dielectric Constant |
| --- | --- | --- |
| Poor ground | 0.001 | 4 |
| Average ground | 0.005 | 15 |
| Good ground | 0.02 | 25 |
| Sea water | 5* | 81* |
| Fresh water | 0.01* | 81* |
| Concrete | 0.01 | 5 |
| Metal | 107 | 10 |
| * More appropriate values are calculated if surface sea temperature is specified. |  |  |

θ3=0.5 a f-13   rad	(50)

θ4=3θ3        rad	(51)

dLp= dLp1+ dLp2     km	(52)

d3= dLp+aθ3         km	(53)

d4= dLp+aθ4         km	(54)

a1,2= dLp1,222hep1,2       km	(55)

a3,4= d3,4- dLPθ3,4   km	(56)

Kd1,2,3,4=0.36278 fa1,2,3,4-13 ε-12+ 18 000σf2-14  	(57)

K1,2,3,4= Kd1,2,3,4                                                      for horizontal polarization Kd1,2,3,4 ε2+ 18 000σf212            for vertical polarization	(58)

KF1,2=lesser of K1,2 or 0.99999	(59)

B1,2,3,4=416.4 f13 1.607- K1,2,3,4	(60)

X1,2= B1,2 a1,2-23 dLp1,2    km	(61)

X3,4= B3,4 a3,4-23 d3,4- dLp+ X1+ X2    km	(62)

G1,2,3,4=0.05751 X1,2,3,4-10 log10X1,2,3,4	(63)

W1,2=0.0134 X1,2exp-0.005 X1,2	(64)

y1,2=40 log10X1,2-117     dB	(65)

F1,2=y1,2 or-117whichever has the lesser absolute value for 0≤KF1,2≤ 10-5y1,2                                         for 10-5 ≤ KF1,2  and -450log10KF1,2-3 ≤ X1,220 log10KF1,2-15+ 2.5 10-5 X1,22KF1,2                                                              otherwisedB (66)

FX1,2= F1,2                                                         for 0≤ X1,2 ≤200W1,2 y1,2+ 1- W1,2 G1,2   for 200≤ X1,2 ≤2 000G1,2                                                            for 2 000 ≤ X1,2    dB	(67)

A3,4= G3,4- FX1- FX2-20      dB	(68)

The values for Ap and Mp are:

Mp= A4- A3d4- d3            dB/km	(69)

Ap= A4- Mpd4         dB	(70)

Rounded earth diffraction attenuation the path F-O-ML uses MF equal to Mp and AF equal to Ap with the values of (67), (68) associated with the F-O-ML path in (47), (48) and (49).

ArF= AF+ MFd         dB	(71)

The value d is the great circle path distance in km which is an input parameter. Rounded earth diffraction attenuation the path O-A uses AA equal to Ap and MA equal to Mp with the values of (67) and (68) associated with the O-A path in (47), (48) and (49).

ArA= AA+ MAdLO1+ dLO2         dB	(72)

Here are the equations for Gɦp1,2 that equals either GɦF1,2 or GɦO1,2 depending on whether one uses values in (47), (48) and (49) associated with F-O-ML or O-A respectively.

BN1,2=1.607- K1,2	(73)

ɦ1,2=2.235 BN1,22 f2a1,213 hep1,2	(74)

fc1,2=0.3 dLp1,2λm10-3	(75)

where λm is from (9).

GW1,2=  0                                                     if hep1,2 ≥2fc1,21                                                     if hep1,2 ≤ fc1,1 0.5 1+ cosπ hep1,2- fc1,2fc1,2              otherwise	(76)

|  | Gɦ1,2= 0 whenever hep1,2 ≥2fc1,2-6.6-0.013 ɦ1,2-2 log10ɦ1,2 whenever ɦ1,2 ≥2.5or when K1,2 ≤0.011.2-13.5 ɦ1,2+15 log10ɦ1,2 if ɦ1,2 <0.3-6.5-1.67 ɦ1,2+6.8 log10ɦ1,2 if 0.3 ≤ ɦ1,2 <2.5or when 0.01 < K1,2 ≤0.05T-25 T-B 0.05-K1,2whereT= -13.9+24.1 ɦ1,2+3.1 log10ɦ1,2 if ɦ1,2 < 0.25T= -5.9-1.9 ɦ1,2+6.6 log10ɦ1,2 if 0.25 ≤ɦ1,2 <2.5B=1.2-13.5 ɦ1,2+15 log10ɦ1,2 if ɦ1,2 <0.3B= -6.5 -1.67 ɦ1,2+6.8 log10ɦ1,2 if 0.3 ≤ ɦ1,2 <2.5or when K1,2 >0.05T-20 T-B 0.1- K1,2whereT= -13 if ɦ1,2 <0.1T= -4.7-2.5 ɦ1,2+7.6 log10ɦ1,2 if 0.1 ≤ ɦ1,2 <2.5B= -13.9+24.1 ɦ1,2+3.1 log10ɦ1,2 if ɦ1,2 <0.25B= -5.9-1.9 ɦ1,2+6.6 log10ɦ1,2 if 0.25 ≤ ɦ1,2 <2.5 dB | (77) |
| --- | --- | --- |

where the values associated with K1,2 are from curve fitting to gain curves [5]. The height gain functions are calculated as follows:

|  | Gɦp1,2= Gɦ1,2 GW1,2 dB | (78) |
| --- | --- | --- |
|  | GɦF1,2= Gɦp1,2 dB for path F-O-ML | (79) |
|  | GɦA1,2= Gɦp1,2 dB for path O-A | (80) |

## 5.2	Knife-edge diffraction

Knife-edge diffraction attenuation is a combination of the knife-edge diffraction attenuations for paths F-O-ML and O-A as in the rounded earth diffraction. For the knife-edge diffraction attenuation for path F-O-ML, AKML:

|  | AKML=6- GɦF1- GɦF2 dB | (81) |
| --- | --- | --- |

where GɦF1,2 are from (79). The knife-edge diffraction attenuation for the path O-A, AKA, is calculated by the following sequence of equations:

|  | dLsA= dLO1+ dLO2 km | (82) |
| --- | --- | --- |

The values dLO1,2 are from (48) and (49).

|  | θv= θe1+ θe2+ dL1+ dLsAa rad | (83) |
| --- | --- | --- |

The values θe1,2 and dL1 are from Fig. 5.

|  | v=5.1658 sin0.5 θv f dLsA dL1dLsA+ dL1 rad | (84) |
| --- | --- | --- |

The values Cv and Sv are calculated from Fresnel integrals.

|  | Cv= 0vcosπt22 dt | (85) |
| --- | --- | --- |
|  | Sv= 0vsinπt22 dt | (86) |
|  | fv=0.5 1- Cv+ Sv2+ Cv- Sv2 | (87) |
|  | AKA= ArA- GɦF1- GɦA1-20 log10fv dB | (88) |

The value ArA is from (72) and the values GɦF1 and GɦA1 are from (79) and (80) for paths F-O-ML and O-A respectively.

## 5.3	Diffraction attenuation

Rounded earth diffraction attenuation and knife-edge diffraction attenuation combine to form the total diffraction attenuation, Ad. The blending factor, W, is calculated from:

|  | W= 1 when dML ≥ dLs (rounded earth only)0 when dML ≤ 0.9 dLs (knife-edge only)0.5 1+ cosπ dLs- dML0.1 dLs otherwise | (89) |
| --- | --- | --- |

where the maximum LoS distance, dML, is from (40) and the total smooth earth distance, dLs, is from (25). The diffraction attenuation for path F-O-ML, AML, uses the blending factor:

|  | AML= ArML if W >0.999AKML if W <0.0011-WAKML+WArML otherwise dB | (90) |
| --- | --- | --- |

where ArML is from (71) calculated with dML and AKML is from (81). The facility-to-antenna 2, dA, distance is calculated from:

|  | dA= dL1+dLsA km | (91) |
| --- | --- | --- |

where dL1 is from (34) and dLsA is from (80). The diffraction attenuation for path O-A, AA, also, uses the blending factor, W:

|  | AA= ArA if W >0.999AKA if W <0.0011-WAKA+WArA otherwise dB | (92) |
| --- | --- | --- |

where ArA = ArML from (71) with d = dA and AKA is from (88). The values for the diffraction attenuation, Ad, and Md are:

|  | Md= AML- AAdML- dA dB/km | (93) |
| --- | --- | --- |
|  | Ad= AML+ Mdd- dML dB | (94) |

The value d is the great circle path distance in km which is an input parameter. The terrain attenuation, AT, uses Ad in § 7.

# 6	Line-of-sight region

For paths shorter than the maximum LoS range (i.e. d < dML), calculations for the LoS attenuation, ALoS, start with a two ray model involving the phasor sum of the direct and the earth-reflected rays. This scenario, also, includes the short term fading statistics, Yπ, from § 10.2. Diffraction makes a contribution to ALoS for distances less than the maximum LoS distance but greater than the LoS transition distance (i.e. d0 < d < dML). Some of the parameters affected by a counterpoise include the counterpoise component in this section, but the discussion on counterpoise is in Attachment 1. For any parameter in this section that has a “g” subscript, there is a parameter with a “c” subscript that uses parameters related to a counterpoise that will are discussed in this section or in Attachment 1.

## 6.1	Two-ray path length geometry

Figure 9 shows the geometry for the two-ray model used on the LoS region. The value ∆r is the difference between the direct ray, r0, and the length of the reflected ray, r12 = r1 + r2.

|  | ∆r= r12- r0 km | (95) |
| --- | --- | --- |

The following set of equations determines ∆r and path length d for various values of the grazing angle, ψ. This calculation is repeated for different values of ψ until there is sufficient coverage for ∆r and d. the counterpoise parameters are included here, but will be used in Attachment 1.

|  | z= a0a -1 | (96) |
| --- | --- | --- |
|  | ka= 11+z cosψ | (97) |
|  | aa= a0ka km | (98) |
|  | ∆ha1,2= ∆he1,2 aa- a0a- a0 km | (99) |
|  | H1= hr1- ∆ha1 for earthhfc for counterpoise km | (100a) |
|  | H2= hr2- Δha2 for earthhr2- Δha2- hc for counterpoise km | (100b) |

where a0 is the actual earth radius of 6 370 km, a is the effective earth radius from (14), hc is an input parameter and hfc is from (19).

|  | z1,2= aa+ H1,2 km |  | (101) |
| --- | --- | --- | --- |
|  | θ1,2= cos-1aa cosψz1,2- ψ rad |  | (102) |
|  | D1,2= z1,2 sinθ1,2 km |  | (103) |
|  | H'1,2= D1,2 tanψ for ψ <1.56 radH1,2 otherwise km |  | (104) |
|  | α= tan-1H'2- H'1D1+ D2 for ψ <1.56 radψ otherwise km |  | (105) |
|  | r0= D1+ D2cosα for ψ <1.56 radH2- H1 otherwise km |  | (106) |

Figure 9

Spherical earth with two ray path geometry

Note to Fig. 9: Relationships between the various geometric parameters shown here were previously provided in IF-73 [3, section A.4.2].

|  | r12= D1+ D2cosψ for ψ<1.56 radH1+ H2 otherwise km | (107) |
| --- | --- | --- |
|  | ∆r=4 H'1H'2r0+ r12 km | (108) |

There are two types of ∆r, ∆rg refers to earth parameters and equals ∆r from (108) when using the earth parameters from (100a) and (100b) to calculate H’1,2. The value ∆rc  refers to counterpoise parameters and equals ∆r from (108) when using the counterpoise parameters from (100a) and (100b) to calculate H’1,2.

|  | θh1= α- θ1 rad | (109a) |
| --- | --- | --- |
|  | θh2= -α+ θ2 rad | (109b) |
|  | θr1,2= -ψ+ θh1,2 rad | (110) |
|  | θ0= θr1+ θr2 rad | (111) |
|  | d= aaθ0 km | (112) |

This set of equations to calculate the great circle distance d uses an adjusted effective earth radius, aa, and adjusted effective antenna heights, H1,2, that vary with ψ since the values a from (14) and hr1,2 from (17) are not appropriate when cos ψ is not near to a value of 1. The value ∆r is calculated by (108) rather than (95), because r0 and r12 may be very large and almost equal.

Except where there is a specific reference to the counterpoise case, any calculations based on ∆r, including equations (108) through (111), are from the earth reflection case (e.g. ∆r = ∆rg).

## 6.2	Effective reflection coefficient

The magnitude of the effective reflection coefficient, RTg, includes the divergence, Dv, and ray length factor, Fr, from § 6.2.1; the surface roughness factor, Fσh, from § 6.2.2; antenna gain pattern factor, gRg, from § 6.2.3; plane earth reflection coefficient magnitude, Rg, from § 6.2.4; and a counterpoise factor, fg, from Attachment 1. Where there is no counterpoise, fg equals 1.

|  | RTg= DvFrFσhfggRgRg | (113) |
| --- | --- | --- |

### 6.2.1	Divergence and ray length factors

The divergence factor, Dv, accounts for a reduction in the magnitude of the reflection from a curved surface as compared to a reflection from a flat surface. In the case where the reflected ray is much longer than the direct ray, such as two close aircraft antennas, the ray length factor, Fr, accounts for the reduction in the magnitude of the reflected ray.

The equations to calculate Dv and Fr are:

|  | r1,2= H1,2 if ψ=90°D1,2cosψ otherwise km | (114) |
| --- | --- | --- |

where H1,2 are from (100a) and (100b), the grazing angle, ψ, is from Fig. 9, and D1,2 are from (103).

|  | Rr= r1r2r12 km | (115) |
| --- | --- | --- |

where r12 is from (107).

|  | Dv= 1+ 2Rr1+ sinψ2aasinψ + 2Rraa2-12 | (116) |
| --- | --- | --- |

where aa is from (98).

|  | Fr= r0r12 | (117) |
| --- | --- | --- |

where r0 is from (106).

### 6.2.2	Surface roughness factors

There are surface roughness factors for specular, Fσh, and diffuse, Fdσh, reflections. The calculations for these factors are:

|  | ∆hd= ∆hm 1- 0.8 exp-0.02 d m | (118) |
| --- | --- | --- |

Here the ∆hm value is the terrain parameter ∆h in metres. The ∆h value is an input parameter. The great circle distance, d, is from (112).

|  | σh= 0.25 H13 for water0.39 ∆hd for ∆hd ≤4 m0.78 ∆hd exp-0.5 ∆hd14 otherwise m | (119) |
| --- | --- | --- |

where H1/3 is a value for significant wave height based on the sea state for a water reflecting surface. This parameter is chosen from Table 2.

|  | δ= σhλmsinψ m | (120) |
| --- | --- | --- |

where the grazing angle, ψ, from Fig. 9 is a starting parameter for calculating ∆r in § 5.1. The wavelength in metres, λm, is from (9).

|  | Fσh=exp-2πδ | (121) |
| --- | --- | --- |

TABLE 2

Estimates of parameters for sea states [4]

| Sea State Code (a) | Descriptive terms (a) | Average wave height range metres (feet) | H1/3 metres (feet) (b) | σh metres (feet) (c) |
| --- | --- | --- | --- | --- |
| 0 | Calm (glossy) | 0 (0) | 0 (0) | 0 (0) |
| 1 | Calm (rippled) | 0 – 0.1 (0 – 0.33) | 0.09 (0.3) | 0.00 (0.08) |
| 2 | Smooth (wavelets) | 0.1 – 0.5 (0.33 – 1.6) | 0.43 (1.4) | 0.11 (0.35) |

TABLE 2 (end)

| Sea State Code (a) | Descriptive terms (a) | Average wave height range metres (feet) | H1/3 metres (feet) (b) | σh metres (feet) (c) |
| --- | --- | --- | --- | --- |
| 3 | Slight | 0.5 – 1.25 (1.6 – 4.0) | 1 (3.3) | 0.25 (0.82) |
| 4 | Moderate | 1.25 – 2.5 (4 – 8) | 1.9 (6.1) | 0.46 (1.5) |
| 5 | Rough | 2.5 – 4 (8 – 13) | 3 (10) | 0.76 (2.5) |
| 6 | Very rough | 4 – 6 (13 – 20) | 4.6 (15) | 1.2 (3.8) |
| 7 | High | 6 – 9 (20 – 30) | 7.9 (26) | 2 (6.5) |
| 8 | Very high | 9 – 14 (30 – 46) | 12 (40) | 3 (10) |
| 9 | Phenomenal | >14 (>46) | >14 (45) | >3.5 (>11) |
| (a) Based on international meteorological |  |  |  |  |
| (b) Estimates significant wave heights, average of highest one-third, H1/3 |  |  |  |  |
| (c) Estimated using a formulation provided by Moskowitz with H1/3 estimates |  |  |  |  |

|  | Fdσh= 0.01+946 δ2 if δ<0.003256.15 δ if 0.00325 ≤ δ ≤0.07390.45+ 0.000843- δ-0.10262 if 0.0739 ≤ δ ≤0.12370.601-1.06 δ if 0.1237 ≤ δ ≤0.30.01+0.875exp-3.88 δ otherwise | (122) |
| --- | --- | --- |

The determination of Fdσh is curve fitting to measured data [6]. The diffuse reflection coefficient, Rd, uses Fdσh, the plane earth reflection coefficient magnitude, Rg, from § 6.2.4, the ray length factor, Fr, from (117), the counterpoise factor, fg, from (293) and the antenna gain pattern factor, gRg, from § 6.2.3. When there is no counterpoise, fg equals 1.

|  | Rd= FrFdσhfggRgRg | (123) |
| --- | --- | --- |

### 6.2.3	Antenna pattern gain factors

The antenna gain factors gD,R and gRh,v allow for situations where antenna gains effective for the direct ray path are different from the gains effective for the reflected ray path. Figure 10 shows the relationship between the two ray paths and the gains. The gains are relative voltage gains and are measured relative to the boresight of the their respective antennas (i.e. for boresight, gD,R = gR1,2 = 1 V/V). Relative gain values can be determined from antenna patterns (i.e. Fig. 11).

Figure 10

Antenna gain notation and correspondence between ray take-off angles and gains

Note to Fig. 10: This sketch is drawn with flat earth, straight rays, and an exaggerated scale so that the geometry shown is over-simplified.

These gains are complex quantities, but are treated as scalars in this method since, in many practical applications, the relative phase between the direct and reflected rays is expected to be zero or is unknown. The voltage gains are ratios that are dimensionless, but are different from power ratios that are also dimensionless. Decibel gains are related to these gains by:

|  | GR1,2=20 log10gR1,2 dB | (124) |
| --- | --- | --- |

Here GR1,2 ≤ 0 because 0 < gR1,2 ≤ 1. The calculations for gD,R are:

|  | gD= gD1gD2 for linear polarization 0.5 ghD1ghD2+ gvD1gvD2 for elliptical polarization | (125) |
| --- | --- | --- |
|  | gR= 1 for isotropic andorelliptical polarizationgR1gR2 otherwise | (126) |

Isotropic implies that, for the radiation angle of interest, gR1 = gD1 and gR2 = gD2. Elliptical polarization uses both the horizontally polarized (ghD1,2 and ghR1,2) and vertically polarized (gvD1,2 and gvR1,2) components of the antennas. Table 3 shows values for gD1,1.

The gain factor gRv involves the vertical polarization gains, gvR1,2.

|  | gRv= 1 for isotropic antennasgvR1gvR2 otherwise | (127) |
| --- | --- | --- |

The gain factor gRh is the same for horizontal polarization gains.

|  | gRh= 1 for isotropic antennasghR1ghR2 otherwise | (128) |
| --- | --- | --- |

Figure 11

Normalized antenna gain vs. elevation angle [2]

The antenna patterns in Fig. 11 are vertical plane patterns. Adjusting GT,R from (4) can change gain variations in azimuth. The values GNT,R for (4) are:

|  | GNT= 20 log10gD1 if the lower antenna is transmitting20 log10gD2 otherwise | (129) |
| --- | --- | --- |
|  | GNR= 20 log10gD1 if the lower antenna is receiving20 log10gD2 otherwise | (130) |

The elevation angles associated with the gain factors are in Table 3.

TABLE 3

Gain factors for elevation angles [1]

| Factor | Elevation angle |
| --- | --- |
| gD1 | θH1 for LoS, θe1 from Fig. 6 otherwise |
| gD2 | θH2 for LoS, θe2 from (44) otherwise |
| gR1, gvR1, ghR1 | θg1 |
| gR2, gvR2, ghR2 | θg2 |

Calculations for these angles are:

|  | θL1,2= θes1,2+ θs1,2 aa- a0a- a0 rad | (131) |
| --- | --- | --- |

where θes1,2 are from (27), θs1,2 are from (26), aa is from (98) and a is from (14). The value a0 is the actual earth radius at 6 370 km.

|  | θH1,2= θh1,2+ θL1,2 rad | (132) |
| --- | --- | --- |
|  | θg1,2= θr1,2+ θL1,2 rad | (133) |

where θh1,2 is from (109a) and (109b) and θr1,2 is from (110). The angles θL1,2 force an element of ray tracing in the elevation angles at the smooth earth horizon and prorate the values elsewhere.

The values H1,2 in (100a) and (100b) have the option for a value based on a counterpoise. This, in turn affects θh1,2 and θr1,2. The counterpoise case is discussed further in Attachment 1. The parameter gRg from (123) equals gR from (126) associated with the H1,2 value for ground reflection.

### 6.2.4	Plane earth reflection coefficients

The dielectric constant, ε, and conductivity, σ, of the reflective surface determine the plane earth reflection coefficient, R exp(–jφ).

For water:

|  | ε= εs- ε01+ 2πfT2+ ε0 | (134) |
| --- | --- | --- |
|  | σ= f2Tε- ε02 865+ σi mho/m | (135) |

The dielectric constant, ε0, represents electronic and atomic polarizations and equals 4.9. The value εs is the static dielectric constant; the frequency, f, is in MHz; the relaxation time, T, is in μs and σi is the ionic conductivity. The values for εs, T and σi for water are from Table 4.

TABLE 4

Surface dielectric constant and conductivity for water [2]

| For fresh water |  |  |  |
| --- | --- | --- | --- |
|  | 0°C | 10°C | 20°C |
| es | 88 | 84 | 80 |
| T [µs] | 1.87 × 10–5 | 1.36 × 10–5 | 1.01 × 10–5 |
| σi [mho/m] | 0.01 | 0.01 | 0.01 |
| For sea water |  |  |  |
|  | 0°C | 10°C | 20°C |
| es | 75 | 72 | 69 |
| T [µs] | 1.69 × 10–5 | 1.21 × 10–5 | 9.2 × 10–6 |
| σi [mho/m] | 3.0 | 4.1 | 5.2 |

For all other surfaces, the appropriate dielectric constant, ε, and conductivity, σ, are from Table 5.

TABLE 5

Surface types and constants [4]

| Type | Conductivity (mhos/m) | Dielectric constant |
| --- | --- | --- |
| Poor ground | 0.001 | 4 |
| Average ground | 0.005 | 15 |
| Good ground | 0.02 | 25 |
| Sea water | 5* | 81* |
| Fresh water | 0.01* | 81* |
| Concrete | 0.01 | 5 |
| Metal | 107 | 10 |
| * More appropriate values are calculated if surface sea temperature is specified. |  |  |

The formulation for R exp (–jφ) is as follows:

|  | εc= ε-j60 λmσ | (136) |
| --- | --- | --- |

where λm is from (9).

|  | Yc= εc-cos2ψ | (137) |
| --- | --- | --- |

where the grazing angle, ψ, from Fig. 9 is a starting parameter for ∆r in § 5.1.

|  | Rv exp-jπ- cv= εc sinψ - Ycεc sinψ+ Yc gR | (138a) |
| --- | --- | --- |

where gR is from (126).

Rh exp-jπ- ch= sinψ - Yc sinψ+ Yc gR	(138b)

Re exp-jπ- ce=0.5 gRhRh exp-jπ- ch+ gRv Rv exp-jπ- cv	(138c)

where gRv,h are from (127) and (128), and

|  | R exp-jϕ= Rv exp-jπ- cv for vertical polarizationRh exp-jπ- ch for horizontal polarizationRe exp-jπ- ce for elliptical polarization | (139) |
| --- | --- | --- |

For elliptical polarization, both antennas must have the sense and circular polarization is a special case of elliptical polarization.

When the parameters in equations (134) through (139) use reflection parameters related to earth or ground, the reflection coefficient, R exp (–jφ) becomes:

|  | Rgexp-jg=Rexp-j | (140) |
| --- | --- | --- |

For the counterpoise case, the reflection coefficient becomes Rc exp (–jφg), see Attachment 1. This is true where the geometry used to calculated gain factors in § 6.2.3 use the H1,2 values from (100a) and (100b).

The total phase lag between the direct ray and the reflected ray is:

|  | Tg= 2 π Δrgλ+ g+ kg+πvg22 rad | (141) |
| --- | --- | --- |

The value ∆rg equals ∆r from (108). The parameters φkg and νg are not necessary when there is no counterpoise since they are part of the phase lag associated with the knife-edge diffraction over the edge of a counterpoise. The values for φkg and νg are discussed in Attachment 1.

## 6.3	Line-of-sight transition distance, d0

The LoS transition distance, d0, is the largest distance in the LoS region where the diffraction component is negligible. This parameter is determined as follows:

|  | θh5=2 sin-10.55.1658 dMLf dL1dML- dL1 rad | (142) |
| --- | --- | --- |

where dML is from (40), f is frequency in MHz, and dL1 is from Fig. 6.

θ5= θh5- tan-1h1- hL1dL1- dL12a      rad

(143)

where hL1 is from Fig. 6, a is from (14), and h1 is the lower antenna height above mean sea level as shown in Fig. 3.

|  | hem2= h2- Δhe2 km | (144) |
| --- | --- | --- |

where h2 is the higher antenna height above mean sea level as shown in Fig. 3 and ∆he2 is from (22).

|  | dL5= -a tanθ5+ a tanθ52+2ahem2- hL1 km | (145) |
| --- | --- | --- |
|  | d5= dL5+ dL1 km | (146) |
|  | θe5= tan-1hL1- hem2dL5- dL52a rad | (147) |
|  | θ6= θe1+ θe5+ d5a rad | (148) |

The value θe1 is from (38).

|  | v5=5.1658 sin0.5 θ6 f dL1dL5d5 | (149) |
| --- | --- | --- |

The value v5 is the upper limit of the Fresnel integrals:

|  | C5= 0v5cosπt22 dt | (150) |
| --- | --- | --- |
|  | S5= 0v5sinπt22 dt | (151) |
|  | f5=0.5 1- C5+ S52+ C5- S52 | (152) |
|  | AK5= -20 log10f5 dB | (153) |
|  | Ar5= AF+ MF d5 dB | (154) |

where MF and AF equal Mp and Ap from (69, 70) for the F-O-ML path respectively.

|  | A5= Ar5 if W>0.99AK5 if W<0.0011-WAK5+WAr5 otherwise dB | (155) |
| --- | --- | --- |

where W is from (89), and

|  | dd= dML- AMLdML- d5AML- A5 km | (156) |
| --- | --- | --- |

where AML is from (90) and dML is from (40). The parameter dd is too short for the LoS transition distance when both antennas have low heights, therefore:

|  | d0= dL1 when dL1> dddλ/6 when dλ/6> dL1 and dddd otherwise km | (157) |
| --- | --- | --- |

where dL1 is the horizon distance for the lower antenna in Fig. 6. The parameter dλ/6 is the largest distance using the two-ray model from § 6.1 where the reflection coefficient is –1. This is the distance in the set of equations (96) through (108) where ∆r equals λ/6 (λ is the wavelength.).

## 6.4	Line-of-sight attenuation, ALoS

LoS attenuation, ALoS, is calculated as follows:

|  | Ffs= 1 if lobing is used and Δrg<10λ1 if Δrg ≤0.5 λ and gD + RTgexp-jTg< gD0 otherwise | (158) |
| --- | --- | --- |

where ∆rg equals ∆r from (106), λ is from (9), gD is determined in § 5.2.3, RTg is from (113) and φTg is from (141).

|  | WR0= gD+ Ffs RTgexp-jTg+ RTcexp-jTc 2+0.0001 gD | (159) |
| --- | --- | --- |

The term RTc exp (–jφTc) is due to a counterpoise and is calculated in Attachment 1 if a counterpoise is present. When there is no counterpoise, the term is equal to zero.

|  | AR0=10 log10gD2WR0 dB | (160) |
| --- | --- | --- |
|  | ML= AML- A0dML- d0 dB/km | (161) |

where AML is from (90), dML is from (40) and d0 is from (157). The parameter A0 equals AR0 with the gD and WR0 values evaluated for d = d0.

|  | ALOS= AR0 if d< d0A0+ MLd- d0 if d0≤d ≤ dML dB | (162) |
| --- | --- | --- |

where d is from (112). The lobing mentioned in (158) is discussed in Attachment 1.

# 7	Scattering region

This formulation to calculate the tropospheric scattering attenuation, As, contains a ray tracing technique that makes it appropriate for use with high antennas. The equation for tropospheric scattering attenuation is:

|  | As= Se+ Sv+10 log10κθ3ls dB | (163) |
| --- | --- | --- |

Se is a scattering efficiency term and Sv is a scattering volume term. The terms κ, θ and ls are the wave number, the scattering angle and the total ray length through the scattering volume respectively.

The ray tracing technique for calculating the scattering angle, θ, begins as:

|  | θa1,2=hLr1,2- he1,2a+ he1,2+2 sindL1,22a2sindL1,2a rad | (164) |
| --- | --- | --- |

The heights hLr1,2 are from (37) and (49) and he1,2 are from (21) respectively. The value a is from (14) and dL1,2 are from (34) and (43).

|  | ds=d- dL1- dL2 km | (165) |
| --- | --- | --- |

where the great circle distance, d, is an input parameter.

|  | θs= θa1+ θa2+dsa rad | (166) |
| --- | --- | --- |
|  | dz1=ds2a+ θa2 ds+ hL2- hL1θs rad | (167) |
|  | dz2= ds- dz1 km | (168) |

Here is introduced the upper limit of the gradient of the ray tracing, Am = 157 * 10–6 km–1 which relates to the edge of super refractivity. Gradients larger than Am can produce ducting.

|  | dN= Am- a-1 km-1 | (169) |
| --- | --- | --- |
|  | γe= Ns* 10 -6dN km | (170) |

where Ns is from (13).

|  | za1,2= 12a dz1,222+ θa1,2dz1,22+ hLr1,2 km | (171) |
| --- | --- | --- |
|  | zb1,2= 12a dz1,22+ θa1,2 dz1,2+ hLr1,2 km | (172) |
|  | Qo1,2= Am-dNexp-hLr1,2γe km-1 | (173) |
|  | Qa1,2= Am-dNexp-za1,2γe km-1 | (174) |
|  | Qb1,2= Am-dNexp-zb1,2γe km-1 | (175) |
|  | Za1,2= 7Qo1,2+6Qa1,2- Qb1,224 dz1,222+ θa1,2 dz1,22+ hLr1,2 km | (176) |
|  | Zb1,2= Qo1,2+2 Qa1,26 dz1,22+ θa1,2 dz1,2+ hLr1,2 km | (177) |
|  | QA1,2= Am-dNexp-za1,2γe km-1 | (178) |
|  | QB1,2= Am-dNexp-zb1,2γe km-1 | (179) |
|  | θA1,2= Qo1,2+4 QA1,2+ QB1,26 dz1,2+ θa1,2 rad | (180) |
|  | θ= θA1+ θA2 rad | (181) |

The value θ is the scattering angle for the tropospheric scattering attenuation, As, from (163).

|  | ZA1,2= Qo1,2+2 QA1,26 dz1,22+ θa1,2 dz1,2+ hLr1,2 km | (182) |
| --- | --- | --- |
|  | Xa= ZA2- ZA1θ km | (183) |
|  | dZ1= dz1+ Xa km | (184) |
|  | dZ2= dz2- Xa km | (185) |
|  | hv= θA1ZA2+ θA2ZA1θ km | (186) |

The value, hʋ, is the height of the scattering volume.

|  | XA1,2= he1,2- hL1,22+4a+ he1,2a+ hL1,2 sindL1,22a2 km | (187) |
| --- | --- | --- |
|  | l1,2= XA1,212 + dZ1,2 km | (188) |
|  | l= l1+ l2 km | (189) |

The value, l, is the total ray length for the tropospheric scattering attenuation, As, from (163).

|  | ε1=5.67* 10-6 Ns2-2.32* 10-3 Ns+0.031 | (190) |
| --- | --- | --- |
|  | ε2=0.0002 Ns2-0.06 Ns+6.6 | (191) |
|  | ϑ=0.1424 1+ ε1exphv46 km-1 | (192) |
|  | Se=83.1- ε21+0.07716 hv2+20 log100.1424ϑ2 expϑ hv dB | (193) |
|  | s= l1- l2l | (194) |

Se is the scattering efficiency term for the tropospheric scattering attenuation, As, from (163) and s is the modulus of asymmetry.

|  | η= ϑθl2 | (195) |
| --- | --- | --- |
|  | κ= 2πλ km-1 | (196) |

where λ is the wavelength in km. This is equal to λm/1000 where λm is from (9). The value κ is the wave number. The values he1,2 from (21) are used to find ρ1,2.

|  | ρ1,2=2κθhe1,2 | (197) |
| --- | --- | --- |
|  | A= 1- s22 | (198) |
|  | Xv1= 1+s2η | (199) |
|  | Xv2= 1-s2η | (200) |
|  | q1,2= Xv1,22+ ρ1,22 | (201) |
|  | Bs=6+8s2+81+sXv22ρ22q22+8 1-sXv12ρ12q12+21- s21+ 2Xv12q11+ 2Xv22q2 | (202) |
|  | Cs=12ρ1+ 2ρ12ρ2+ 2ρ22ρ1+ ρ2ρ1+ ρ2+22 | (203) |
|  | Sv=10 log10Aη2+ Bsηq1q2ρ12ρ12+ Cs dB | (204) |

Sv is the scattering volume term for the tropospheric scattering attenuation, As, from (163).

# 8	Terrain attenuation

Calculations for the terrain attenuation, AT, change for different regions.

| AT= ALOS for d < dMLAd if Asx≥ AdxAsx+ Asx- AMLdx- dML d- dx otherwise for dML≤d ≤ dxlesser of Ad or Aswhere As has been selected for a shorter distance for dx <d dB | (205) |
| --- | --- |

ALoS is from (162). The value d is a specified parameter except in the LoS region where it is from (112). Ad is from (94). Asx = As from (163) and Adx = Ad from (94) both for d = dx. AML is from (90) and dML is from (40).

The distance, dx, is the shortest distance beyond the radio horizon at which As > 20 dB and Ms ≤ Md. Ms is the slope of the curve for As versus distance determined by successive As calculations ( see § 7). Md is from (93).

# 9	Free space loss

The free space loss calculation is:

|  | Lbf=32.45+20 log10fr | (206) |
| --- | --- | --- |

where f in MHz is an input parameter and r is calculated below.

For LoS paths, calculate rWH.

|  | rWH= h2- h12+4h1+ a0h2+ a0sin0.5 da0212 km | (207) |
| --- | --- | --- |

The heights h1,2 are the heights of the lower and higher antennas, respectively, from mean sea level from Fig. 3. The value a0 is the actual earth radius of 6 370 km and the great circle distance, d, is from (112) for LoS paths.

|  | r=greater of r0 or rWH km | (208) |
| --- | --- | --- |

where r0 is from (106).

For all other paths, calculate the two values, rL1,2:

| rL1,2= h1,2- hL1,22+4h1,2+ a0 hL1,2+ a0sin0.5 dL1,2a0212 km | (209) |
| --- | --- |

Heights hL1,2 are from (36) and (45) respectively. Distances dL1,2 are calculated in § 4.

|  | rBH= rL1+ rL2+ds km | (210) |
| --- | --- | --- |

where ds is from (165).

|  | r=greater of d or rBH km | (211) |
| --- | --- | --- |

where d is an input variable.

# 10	Atmospheric absorption

Estimating the median atmospheric absorption combines the amount of absorption of oxygen and water vapour for a given frequency with the ray length through the identified atmospheric layers. The absorption is calculated from the surface absorption rates and ray lengths as:

|  | Aa= ΥooReo+ ΥowRew dB | (212) |
| --- | --- | --- |

The figures of the geometry of the ray lengths are in Fig. 12.

Figure 12

Geometry associated with atmospheric absorption

## 10.1	Surface absorption rates

Effective ray lengths for oxygen and water vapour combined with the surface absorption rates give an estimate for attenuation due to atmospheric absorption. Curves for the surface absorption rates for oxygen and water vapour, ϒoo and ϒow respectively, for frequency are on Fig. 13. Data from the graph in Fig. 13 are available in table form in Attachment 2.

Figure 13

Surface values of atmospheric absorption by oxygen and water vapour [5]

### 10.1.1	Determining ray length

The geometry for determining ray length through the appropriate layers of the atmosphere is illustrated in Fig. 12. The values Teo and Tew are 3.25 km and 1.36 km respectively.

Calculate At and Hz

|  | At= β+0.5 π | (213) |
| --- | --- | --- |
|  | Hz=lesser of Teo,w+ aΥHγ2+ aγ | (214) |

For Hγ1 < Teo,w

|  | Aq= sin-1HΥ1+ aγ sinAtHz | (215) |
| --- | --- | --- |
|  | Ae= π- At+ Aq | (216) |
|  | Reo,w= Teo,w- HΥ1 if Aq<0.02 radHγ1+ aγ sinAesinAq otherwise km | (217) |

For Teo,w < Hγ1

|  | Hc= HΥ1+ aγsinAt | (218) |
| --- | --- | --- |
|  | Reo,w= 0 if Teo,w+ aΥ ≤ Hc or At ≤ π/22Teo,w+ aγsincos-1HcTeo,w+ aγ otherwise km | (219) |

### 10.1.2	Line-of-sight paths (d ≤ dML)

Atmospheric absorption calculations for LoS paths use the following values:

•	Hϒ1,2 = H1,2 from (100 a and b)

•	aϒ = aa from (14)

•	β = θh1 from (109a)

### 10.1.3	Single horizon paths (dML ≤ d ≤ dL1 + dLo1)

The single horizon paths use two sets of starting parameters. R1eo,w and R2eo,w equal Reo,w in the calculations above for the two sets of initial parameters. Reo,w is the sum of R1eo,w and R2eo,w.

Atmospheric absorption calculations for R1eo,w use the following values:

•	Hϒ1 = lesser of he1 or hLr1

•	Hϒ2 = greater of he1 or hLr1

•	aϒ = a from (14)

•	β = θe1 from (38) if Hϒ1 = he1 and θL from (39) otherwise

Atmospheric absorption calculations for R2eo,w use the following values:

•	Hϒ1 = lesser of he2 or hLr2

•	Hϒ2 = greater of he2 or hLr2

•	aϒ = a from (14)

•	β = θe2 from (38) if Hϒ1 = he2 and –θL from (39) otherwise

### 10.1.4	Two horizon paths

Two-horizon paths also use two sets of starting parameters. Again, R1eo,w and R2eo,w equal Reo,w in the calculations above for the two sets of initial parameters and Reo,w is the sum of R1eo,w and R2eo,w.

Atmospheric absorption calculations for R1eo,w use the following values:

•	Hϒ1 = lesser of he1 or hv

•	Hϒ2 = greater of he1 or hv

•	aϒ = a from (14)

•	β = θe1 from (36) if Hϒ1 = he1 and – tan–1 θA1 (θA1 from (178)) otherwise

Atmospheric absorption calculations for R2eo,w use the following values:

•	Hϒ1 = lesser of he2 or hv

•	Hϒ2 = greater of he2 or hv

•	aϒ = a from (14)

•	β = θe2 from (37) if Hϒ1 = he2 and – tan–1 θA2 (θA2 from (176)) otherwise

# 11	Variability

Total variability, Y(q), for this method includes terms for long-term (hourly median) variability, Ye(q) and short-term variability from multipath, Yπ(q), rain attenuation, Yr(q), and ionospheric scintillation, YI(q). The terms are combined by:

|  | YΣ(q)= ± Ye2q+ Yπ2q+ Yr2q+ YI2q | (220) |
| --- | --- | --- |

+ for q ≤ 0.5

– otherwise

For an hourly median availability, all the short-term variabilities are set to zero. Variability due to rain attenuation is considered equal to zero when q ≤ 0.98 since the assumption is that rain attenuation occurs during 2 percent of the time. Also, variability due to ionospheric scintillation is considered equal to zero except for satellite paths that pass through the ionosphere at about 350 km above sea level.

## 11.1	Variability due to long-term fading

The variable, Ye(q), represents long-term or hourly median fading. This is a signed value such that positive values with q ≤ 0.5 indicate a decrease in transmission loss or an increase in received power. Long-term fading has two components. One component is due to seasonal changes and the other component is due to climatic changes. The long-term fading uses an effective distance, de, calculated as follows:

|  | de= 130 ddq for d ≤ dq130+d- dq otherwise km | (221) |
| --- | --- | --- |

where d is the great circle distance that is a specified parameter or, in the LoS region, is from (112) and dq is calculated by:

|  | dq= dLq+ 65 100f13 km | (222) |
| --- | --- | --- |

where f is frequency in MHz and dLq is the total smooth earth horizon distance determined by ray tracing in Attachment 1 for an effective earth radius of 9 000 km. The equations to calculate the gain factors, g (0.1 or 0.9, f), associated with Ye(q) are:

|  | g0.1, f= 0.21 sin5.22 logf200+1.28 for 60 ≤f ≤1 600 MHz1.05 for f >1 600 MHz | (223) |
| --- | --- | --- |
|  | g0.9, f= 0.18 sin5.22 logf200+1.23 for 60 ≤f ≤1 600 MHz1.05 or f>1 600 MHz | (224) |

The calculations for the reference levels of the hourly Y median variability, Y0(0.1) and Y0(0.9), and for the parameter, V(0.5) start with the equation for f2.

|  | f2= f∞+ fm- f∞exp-c2den2 | (225) |
| --- | --- | --- |

feeds into:

|  | V0.5Y00.1-Y0(0.9)= c1den1- f2exp-c3den3 + f2 dB | (226) |
| --- | --- | --- |

The values c1, c2, c3, n1, n2, n3, fm and f∞ are from Table 6 (all hours all year) for Y0 (0.1), Table 7 (all hours all year) for Y0 (0.9) or Table 8 (Climate 1) for V(0.5). Other options for determining these parameters are using time blocks (§ 11.1.1), climates (§ 11.1.2) or mixed distributions (§ 10.1.3).

|  | Y0.1 or 0.9=g0.1 or 0.9, f Y00.1 or 0.9 dB | (227) |
| --- | --- | --- |

Use Y(0.1) and Y(0.9) to calculate other percentiles for the hourly median received power.

TABLE 6

Constants for calculating Y0(0.1) [5]

| Time block | c1 | de in km c2 | c3 | n1 | n2 | n3 | fm | f∞ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1.22–2 | 9.81–6 | 1.09–8 | 1.36 | 2.00 | 3.58 | 10.8 | 5.5 |
| 5 | 2.58–4 | 3.41–6 | 2.01–11 | 2.05 | 2.25 | 4.78 | 8.0 | 4.0 |
| 6 | 3.84–3 | 4.22–5 | 7.76–9 | 1.57 | 1.76 | 3.66 | 9.6 | 5.2 |
| 7 | 7.95–3 | 3.76–5 | 3.19–8 | 1.47 | 1.76 | 3.40 | 11.2 | 5.5 |
| S* | 4.47–3 | 1.66–5 | 2.06–8 | 1.55 | 1.90 | 3.48 | 9.98 | 5.1 |
| 1 | 1.09–4 | 1.21–6 | 8.29–8 | 2.28 | 2.29 | 3.26 | 9.6 | 2.8 |
| 2 | 1.04–5 | 4.28–8 | 3.51–8 | 2.71 | 2.91 | 3.41 | 9.15 | 2.8 |
| 3 | 2.02–4 | 1.45–6 | 4.27–8 | 2.15 | 2.28 | 3.37 | 9.4 | 2.8 |
| 8 | 1.70–4 | 7.93–7 | 1.29–7 | 2.19 | 2.37 | 3.18 | 9.5 | 3.0 |
| W* | 2.46–4 | 1.74–7 | 1.27–8 | 2.11 | 2.64 | 3.62 | 9.37 | 2.8 |
| A* | 5.25–4 | 1.57–6 | 4.70–7 | 1.97 | 2.31 | 2.90 | 10.0 | 5.4 |
| Time blocks “S”, “W”, and “A” are all hours summer, all hours winter, and all hours all years respectively. Small digits represent the exponent of the number, for example, 2.33−2 = 2.33 × 10−2 |  |  |  |  |  |  |  |  |

TABLE 7

Constants for calculating Y0(0.9) [5]

| Time block | c1 | de in km c2 | c3 | n1 | n2 | n3 | fm | f∞ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1.84–4 | 2.22–6 | 3.65–16 | 2.09 | 2.29 | 6.82 | 8.0 | 4.0 |
| 5 | 3.80–4 | 4.76–6 | 8.39–17 | 1.92 | 2.19 | 7.10 | 6.6 | 3.3 |
| 6 | 1.81–3 | 5.82–6 | 6.37–13 | 1.67 | 2.15 | 5.38 | 8.4 | 4.1 |
| 7 | 3.19–3 | 2.51–6 | 5.03–9 | 1.60 | 2.27 | 3.69 | 10.0 | 4.4 |
| S* | 7.42–4 | 5.55–5 | 4.37–8 | 1.84 | 1.69 | 3.28 | 8.25 | 4.0 |
| 1 | 1.72–4 | 6.39–8 | 2.93–20 | 2.10 | 2.79 | 4.24 | 8.2 | 2.4 |
| 2 | 1.05–5 | 7.00–13 | 7.64–9 | 2.59 | 4.80 | 3.68 | 7.05 | 2.8 |
| 3 | 3.64–5 | 3.74–9 | 3.53–7 | 2.40 | 3.28 | 2.94 | 7.8 | 2.2 |
| 8 | 1.64–6 | 1.43–7 | 3.14-7 | 3.08 | 2.66 | 3.03 | 8.6 | 2.6 |
| W* | 3.45–6 | 1.25–8 | 7.50–7 | 2.87 | 3.07 | 2.82 | 7.92 | 2.45 |
| A* | 2.93–4 | 3.78–8 | 1.02–7 | 2.00 | 2.88 | 3.15 | 8.2 | 3.2 |
| Time blocks “S”, “W”, and “A” are all hours summer, all hours winter, and all hours all years respectively. Small digits represent the exponent of the number, for example, 4.97−4 = 4.97 × 10−4. |  |  |  |  |  |  |  |  |

TABLE 8

Constants for calculating V(0.5) [5]

| Climate | c1 | de in km c2 | c3 | n1 | n2 | n3 | fm | f∞ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Continental Temperate | 1.59–5 | 1.56–11 | 2.77–8 | 2.32 | 4.08 | 3.25 | 3.9 | 0 |
| 2. Maritime Temperate Overland | 1.12–4 | 1.26–20 | 1.17–11 | 1.68 | 7.30 | 4.41 | 1.7 | 0 |
| 3. Maritime Temperate Oversea | 1.18–4 | 3.33–13 | 3.82–9 | 2.06 | 4.60 | 3.75 | 7.0 | 3.2 |
| 4. Maritime Subtropical Overland | 1.09–4 | 5.89–18 | 2.21–7 | 2.06 | 6.81 | 2.97 | 5.8 | 2.2 |
| 5. Maritime Subtropical Oversea | (deleted) |  |  |  |  |  |  |  |
| 6. Desert (Sahara) (Computes – V(0.5)) | 8.85–7 | 2.76–14 | 2.25–12 | 2.80 | 4.82 | 4.78 | 8.4 | 8.2 |
| 7. Equatorial | 3.45–7 | 3.74–12 | 6.97–8 | 2.97 | 4.43 | 3.14 | 1.2 | –8.4 |
| 8. Continental Subtropical | 1.59–5 | 1.56–11 | 2.77–8 | 2.32 | 4.08 | 3.25 | 3.9 | 0 |

|  | Yq= cY0.1 for q<0.50 for q=0.5cY0.9 for q>0.5 dB | (228) |
| --- | --- | --- |

The distribution calculations to obtain values for c are in Attachment 1. The values of c for q < 0.1 are in Table 9 and the values of c for q > 0.9 are in Table 10.

TABLE 9

Factor c for q [2]

| Climate number | q = 0.01 | q = 0.001 | q = 0.001 |
| --- | --- | --- | --- |
| 1, 6 and 8 | 1.95 | 2.73 | 3.33 |
| 2 | 1.79 | 2.27 | 2.66 |
| 3 | 2.20 | 3.30 | 3.70 |
| 4 | 1.82 | 2.41 | 2.90 |
| 5 | ----- | ----- | ----- |
| 7a and 7b | 2.15 | 3.05 | 3.80 |

TABLE 10

Factor c for q [2]

| q | c* |
| --- | --- |
| 0.95 | 1.28 |
| 0.99 | 1.82 |
| 0.995 | 2.01 |
| 0.999 | 2.41 |
| 0.9995 | 2.57 |
| 0.9999 | 2.90 |
| * For q > 0.9 c values follow a log-normal distribution for all climates. |  |

There is a correction factor for the elevation angle of the lower antenna, fθh.

|  | fθh= 0.5- π-1tan-120 log1032 θh1 for LoS paths with θh1> 01 otherwise | (229) |
| --- | --- | --- |

The value θh1 is from (109a).

|  | YeIq= fθhYq dB | (230) |
| --- | --- | --- |
|  | Ve0.5= fθhV0.5 dB | (231) |

When using lobing, calculate YT.

|  | YT= Lb- Lbf- Ve0.5-20 log10RTg+ RTc dB | (232) |
| --- | --- | --- |

Where Lb(0.5) is from (2), Lbf is from (205), and RTg is from (113). The value for RTc equals zero when there is no counterpoise; otherwise it is calculated in Attachment 1.

|  | AYI= Lbf-3- Lbr- YeI(0.1) | (233) |
| --- | --- | --- |

Lbr is from (3). The value AYI is zero when using lobing and the higher antenna is within 10 lobes of its radio horizon.

|  | AY= 0 if AYI ≤010 if AYI ≥10AYI otherwise | (234) |
| --- | --- | --- |

The calculations for the long-term variability, Ye(q) are:

|  | Yeq≥0.1= YeIq dB | (235) |
| --- | --- | --- |
|  | Yeq<0.1= lesser of YeIq or YT for lobinglesser of YeIq or Lbr+ AY-(Lbf- cY) otherwise | (236) |

where cY is 6, 5.8, and 5 dB for q values of 0.0001, 0.001, 0.01 respectively.

### 11.1.1	Time blocks

This method has options to calculate variabilities for blocks of time and seasonal groupings. These time blocks are defined in Table 11 and are associated with a continental temperate climate. To add the effects of time blocks to the long-term variability, one uses values in Tables 6, 7 and 8 that are appropriate to the time block desired and use those values to calculate V(0.5), Y0(0.1) and Y0(0.9) in (225). To combine time blocks use the technique for mixing distributions described in § 11.5.

TABLE 11

Time block ranges [1]

| Number | Months | Hours |
| --- | --- | --- |
| 1 | Nov. – Apr. | 0600 – 1300 |
| 2 | Nov. – Apr. | 1300 – 1800 |
| 3 | Nov. – Apr. | 1800 – 2400 |
| 4 | May – Oct. | 0600 – 1300 |
| 5 | May – Oct. | 1300 – 1800 |
| 6 | May – Oct. | 1800 – 2400 |
| 7 | May – Oct. | 0000 – 0600 |
| 8 | Nov. – Apr. | 0000 – 0600 |
| Summer | May – Oct. | All Hours |
| Winter | Nov. – Apr. | All Hours |

### 11.1.2	Climates

There are options for different climates as defined in (238) and (239). Table 12 provides the values b1, b2, b3, c1, and c2 for the following equations to use in calculating the long-term variability.

|  | V0.5Y00.1-Y0(0.9)= deb121+ deb12 c1+ c21+ de- b2b32 dB | (237) |
| --- | --- | --- |

The value, de, is from (220).

| g0.1,f= 1 for all climates except 2, 4 and 60.18 sin5 log10f200+1.06 for 60 ≤f ≤1 500 MHzclimates 2 and 61 suggested for 60 ≤f ≤200 MHz climate 40.10 sin5 log10f200+1.02 for 200 ≤f ≤1 500 MHz climate 40.93 for f>1 500 MHz climates 2, 4 and 6 |  | (238) |
| --- | --- | --- |
| g0.9, f= 1 for all climates except 60.13 sin5 log10f200+1.04 for 60≤f ≤1 500 MHz in climate 60.92 for f>1 500 Mhz in climate 6 | (239) |  |

where f is the input frequency in MHz. These values are then used in (225) and (229) to calculate Ye (q) as in § 11.1. Section 11.6 describes how to combine the distributions for multiple climates.

TABLE 12

Constants to calculate v(0.5), Y0(0.1), and Y0(0.9) [2]

| Climate | Parameter | b1 | b2 | b3 | c1 | c2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Equatorial | V(0.5) Yo(0.1) Yo(0.9) | 144.9 636.9 762.2 | 190.3 134.8 123.6 | 133.8 95.6 94.5 | –9.67 2.70 –2.73 | 12.7 131.1 –204.4 |
| 2. Continental subtropical | V(0.5) Yo(0.1) Yo(0.9) | 228.9 138.7 100.4 | 205.2 143.7 172.5 | 143.6 98.6 136.4 | –0.62 8.8 –3.41 | 9.19 19.9 –9.83 |
| 3. Maritime subtropical | V(0.5) Yo(0.1) Yo(0.9) | 262.6 165.3 138.2 | 185.2 225.7 242.2 | 99.8 129.7 178.6 | 1.26 12.9 –7.83 | 15.5 12.3 –8.52 |
| 4. Desert | V(0.5) Yo(0.1) Yo(0.9) | 84.1 464.4 139.1 | 101.1 93.1 132.7 | 98.6 94.2 193.5 | –9.21 4.72 –2.54 | 9.05 204.2 –16.8 |
| 5. *Mediterranean | -------- | -------- | -------- | -------- | -------- | -------- |
| 6. Continental temperate | V(0.5) Yo(0.1) Yo(0.9) | 228.9 93.2 93.7 | 205.2 135.9 186.8 | 143.6 113.4 133.5 | –0.62 6.04 –3.43 | 9.19 10.4 –9.17 |
| 7a. Maritime temperate overland | V(0.5) Yo(0.1) Yo(0.9) | 141.7 216.0 187.8 | 315.9 152.0 169.6 | 167.4 122.7 108.9 | –0.39 11.0 –8.79 | 2.86 17.9 –13.3 |
| 7b. Maritime temperate oversea | V(0.5) Yo(0.1) Yo(0.9) | 2222.0 136.2 609.8 | 164.8 188.5 119.9 | 116.3 122.9 106.6 | 3.15 10.8 –10.9 | 857.9 10.5 –217.6 |
| 8. *Polar | Use Climate 6 |  |  |  |  |  |
| * For climate numbers 5 and 8, Mediterranean and Polar, values are not available; a substitute for Polar is suggested for use unless more definite information is available from other sources. |  |  |  |  |  |  |

## 11.2	Variation due to multipath

The variable, Yπ, represents short-term (less than an hour) fading due to multipath. This fading is a result of both surface reflection and tropospheric multipath and generally follows a Nakagami-Rice distribution.

### 11.2.1	Surface reflection multipath

Surface reflection multipath contributes to the short-term variability of LoS paths. This method uses both the specular and diffuse components of the surface multipath mechanism. These components as well as the tropospheric multipath components make up the short-term fading, Yπ(q), component of the total variability for the model.

The model uses the following fading formulas to incorporate the surface reflection multipath elements into Yπ(q):

|  | FAY=1 if AY ≤00.1 if AY ≥90.5 1.1+0.9cosπAY9 otherwise | (240) |
| --- | --- | --- |

where AY is from (232) and

|  | F∆r= 1 if ∆rg ≥λ20.1 if ∆rg ≤λ61.1-0.9cos3π∆rg- λ/6/λ2 otherwise | (241) |
| --- | --- | --- |

where ∆rg equals ∆r from (108) when using ground parameters and λ is the wavelength in km where the wavelength in metres, λm, is from (9).

The reflection contributions are RS2 for the relative contribution of the specular component and Rd2 is the contribution of the diffuse component. These values are calculated by:

|  | Rs= RTg FAYF∆r | (242) |
| --- | --- | --- |

where RTg is from (113) and

|  | Rd= RTg FdσhFσhDv | (243) |
| --- | --- | --- |

where Fdσh is from (121), Fσh is from (122) and Dv is from (116).

The relative power level, WR, from the surface reflection components is calculated by:

|  | WR= Rs2+ Rd2gD2 for LoS (d≤ dML)0 otherwise | (244) |
| --- | --- | --- |

where d is the path length from (112), dML is from (40), and gD is from (125).

### 11.2.2	Tropospheric multipath

Tropospheric multipath is caused by elevated layers in the troposphere and is part of the short-term variability. This mechanism is only used when working with time availability for instantaneous levels exceeded. Tropospheric multipath determines relative power level Wa which the method uses to determine the short-term multipath fading, Yπ(q).

The fading component from tropospheric multipath is calculated as:

|  | F=10 log10 f Rew3-84.26 dB | (245) |
| --- | --- | --- |

when d ≤ dML. The input frequency f is in MHz, Rew is calculated in § 10.1 for different paths, d is from (112) and dML is the maximum LoS distance from (40). The variable F is not calculated for d ≥ dML.

NOTE – The values for KLoS are from Table 13 for column Yπ(0.99). KML is determined for F values with d = dML.

|  | KLOS= 40 dB if F ≤0.14-20 dB if F ≥18.4obtained from Table 13 otherwise | (246) |
| --- | --- | --- |

The values for KLoS are K values from Table 13 for column Yπ(0.99) for –F.

TABLE 13

Characteristics of Nakagami-Rice phase interference fading distribution [5]

|  | Kt= KLOS if d ≤ dML-20 if θs>0.02618 radKML+ - 20- KMLθs 0.02618 otherwise | (247) |
| --- | --- | --- |

where θs is the scattering angle from (166). This angle is negative in the LoS region. Kt determines the relative power, Wa. The value KML is the same as the value KLoS when Rew is evaluated at dML.

|  | Wa= 10-Kt10 | (248) |
| --- | --- | --- |

### 11.2.3	Variation due to multipath

The model uses the relative powers WR and Wa to calculate the ratio between the direct ray component and the Rayleigh fading component, K.

|  | K= -10 log10 WR+ Wa dB | (249) |
| --- | --- | --- |

K is then the value to determine the Nakagami-Rice probability distribution for Yπ(q) from Fig. 14.

Figure 14

Nakagami-Rice probability distribution of instantaneous fading associated with phase interference [5]

## 11.3	Rain attenuation

There are two options for determining rain attenuation, Yr. The first option requires rain zone and storm size input values. There are three storm sizes which are small, medium and large (5, 10, or 20 km, respectively). This method assumes only one storm in any path. There are five steps for calculating Yr using this option.

1	Determine point rain rates (rate at a particular point of observation). Table 14 identifies the point rain rates for a rain zone chosen from Fig. 15 and for a time percentage. Note that for all q ≤ 0.98, the point rain rate equals zero which makes Yr(q) equal zero.

2	Determine path average rain rates. Take the point rain rate value and the identified storm size to find the path average rain rate from Table 15.

3	Determine the attenuation rate, Arr(q). Use the path average rain rate from step 3 and the frequency to find the attenuation rate from Table 16. Use linear interpolation to calculate the rate.

4	Determine the in-storm ray length. The ray length within Tes, Res, is calculated using the following parameters and calculations from the geometry in Fig. 12.

Line of sight paths (d ≤ dML)

Atmospheric absorption calculations for LoS paths use the following values:

•	Hϒ1,2 = H1,2 from (98)

•	aϒ = aa from (14)

•	β = θh1 from (107a)

Single horizon paths (dML ≤ d ≤ dL1 + dL01)

The single horizon paths use one set of starting parameters. Atmospheric absorption calculations for Res use the following values:

•	Hϒ1 = lesser of he1 or hLr1

•	Hϒ2 = greater of he1 or hLr1

•	aϒ = a from (14)

•	β = θe1 from (37) if Hϒ1 = he1 and θL from (38) otherwise

Two-horizon paths (dL1 + dL01 ≤ d)

Two-horizon paths also use one set of starting parameters. Atmospheric absorption calculations for Res use the following values:

•	Hϒ1 = lesser of he1 or hv

•	Hϒ2 = greater of he1 or hv

•	aϒ = a from (14)

•	β = θe1 from (37) if Hϒ1 = he1 and –tan–1 θA1 (θA1 from (176)) otherwise

Calculate At and Hz

|  | At= β+0.5 π | (250) |
| --- | --- | --- |
|  | Hz=lesser of Tes+ aγHγ1+ aγ | (251) |

TABLE 14

Point rain rates (mm/hr) not exceeded for a fraction of time, q [2]

| Rain zone |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| ≤.98 | 0 | 0 | 0 | 0 | 0 | 0 |
| .99 | 0.17 | 0.25 | 0.31 | 0.54 | 0.75 | 1.0 |
| .995 | 0.62 | 0.98 | 1.54 | 2.07 | 2.7 | 3.35 |
| .998 | 1.8 | 3.1 | 4.8 | 6.2 | 7.8 | 9.4 |
| .999 | 3.2 | 5.4 | 8.8 | 11.7 | 14.0 | 17.0 |
| .9995 | 5.1 | 9.6 | 14.5 | 19.0 | 23.5 | 28.5 |
| .9998 | 8.2 | 17.0 | 25.0 | 33.0 | 40.0 | 48.0 |
| .9999 | 11.3 | 22.8 | 34.0 | 44.5 | 54.0 | 67.0 |
| .99995 | 14.6 | 29.5 | 43.0 | 57.0 | 68.0 | 84.0 |
| .99998 | 18.8 | 37.8 | 56.0 | 73.0 | 91.0 | 112.0 |
| .99999 | 24.0 | 44.0 | 64.0 | 86.0 | 110.0` | 160.0 |

For Hγ1 < Tes

|  | Aq= sin-1Hγ1+ aγ sinAtHz | (252) |
| --- | --- | --- |
|  | Ae= π- At+ Aq | (253) |
|  | Res= Tes- Hγ1 if Aq<0.02 radHγ1+ aγ sinAesinAq otherwise km | (254) |

For Tes < Hγ1

|  | Hc= Hγ1+ aγsinAt | (255) |
| --- | --- | --- |
|  | Res= 0 if Tes+ aγ ≤ Hc or At ≤ π/22Tes+ aγsincos-1HcTes+ aγ otherwise km | (256) |

Tes equals the storm size input value. The in-storm length rs is determined by:

|  | rs= Tes if Res ≥ TesRes otherwise | (257) |
| --- | --- | --- |

5	Determine rain attenuations values. The rain attenuation variability, Yr(q), for q > 0.98 is calculated by:

|  | Yrq= -Arrrs | (258) |
| --- | --- | --- |

Figure 15

Rain zones of the world [4]

TABLE 15

Path average-to-point rain rate ratio [2]

| Point rain rate in mm/hr | Storm size estimated |  |  |
| --- | --- | --- | --- |
|  | 5 km | 10 km | 20 km |
| 10 | 1.0 | 1.0 | 1.0 |
| 15 | 0.96 | 0.916 | 0.835 |
| 18 | 0.94 | 0.865 | 0.73 |
| 20 | 0.93 | 0.85 | 0.70 |
| 23 | 0.915 | 0.83 | 0.66 |
| 27 | 0.899 | 0.797 | 0.61 |
| 30 | 0.888 | 0.775 | 0.58 |
| 33 | 0.878 | 0.755 | 0.564 |
| 37 | 0.865 | 0.730 | 0.54 |
| 40 | 0.860 | 0.720 | 0.53 |
| 44 | 0.850 | 0.704 | 0.51 |
| 50 | 0.840 | 0.683 | 0.493 |
| 55 | 0.833 | 0.670 | 0.480 |
| 60 | 0.824 | 0.650 | 0.470 |
| 65 | 0.818 | 0.640 | 0.452 |
| 70 | 0.813 | 0.627 | 0.440 |
| 80 | 0.805 | 0.610 | 0.422 |
| 90 | 0.798 | 0.592 | 0.408 |
| 100 | 0.790 | 0.575 | 0.392 |
| 115 | 0.780 | 0.565 | 0.378 |
| 140 | 0.770 | 0.550 | 0.357 |
| 155 | 0.765 | 0.540 | 0.350 |
| 185 | 0.760 | 0.528 | 0.325 |
| 200 | 0.758 | 0.520 | 0.310 |

TABLE 16

Attenuation in mm/km for various rain rates [2]

| Rainfall Rate |  |  |  |  | Frequency in GHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mm/hr | in/hr | 100 | 60 | 30 | 20 | 15 | 10 | 7.5 | 6 | 5 | 4.3 | 3 | 2 |
| 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 0.01 | 0.25 | 0.16 | 0.03 | 0.01 | 0.006 | 0.002 | 0.0008 | 0.0004 | 0.000 | 0.000 | 0.000 | 0.000 |
| 1.25 | 0.05 | 1.29 | 0.76 | 0.21 | 0.09 | 0.04 | 0.01 | 0.004 | 0.002 | 0.001 | 0.0008 | 0.0004 | 0.000 |
| 2.5 | 0.10 | 2.19 | 1.43 | 0.45 | 0.20 | 0.10 | 0.03 | 0.01 | 0.005 | 0.003 | 0.002 | 0.0007 | 0.0003 |
| 5 | 0.20 | 3.68 | 2.63 | 0.93 | 0.43 | 0.23 | 0.07 | 0.03 | 0.01 | 0.006 | 0.003 | 0.001 | 0.0006 |
| 12.5 | 0.49 | 7.08 | 5.46 | 2.43 | 1.18 | 0.71 | 0.24 | 0.08 | 0.03 | 0.02 | 0.009 | 0.003 | 0.001 |
| 25 | 0.98 | 11.7 | 9.86 | 4.87 | 2.49 | 1.53 | 0.60 | 0.22 | 0.09 | 0.04 | 0.020 | 0.007 | 0.002 |
| 50 | 1.97 | 19.6 | 17.0 | 9.59 | 5.15 | 3.28 | 1.45 | 0.59 | 0.24 | 0.10 | 0.050 | 0.010 | 0.005 |
| 100 | 3.94 | 33.7 | 29.4 | 18.4 | 10.4 | 6.77 | 3.43 | 1.55 | 0.64 | 0.26 | 0.120 | 0.030 | 0.010 |
| 150 | 5.91 | 46.8 | 40.9 | 26.8 | 15.7 | 10.2 | 5.49 | 2.71 | 1.13 | 0.47 | 0.210 | 0.050 | 0.020 |
| 200 | 7.87 | 61.0 | 56.0 | 34.0 | 22.0 | 14.5 | 8.10 | 4.10 | 1.80 | 0.73 | 0.34 | 0.082 | 0.036 |

## 11.4	Variation due to scintillation

Propagation paths that pass through an altitude of about 350 km in the ionosphere include a variability associated with ionospheric scintillation, YI(q). This method uses two types of calculations to determine the variability. One specifies the variability by selecting a scintillation index group. The other uses a weighted mixture of distributions selected for specific cases. This method also allows YI(q) to change with latitude of earth stations of a geostationary satellite.

One can find the variability due to scintillation for time percentage q on Fig. 16. Data values from the graph in Fig. 16 are in table form in Attachment 2. The index group is calculated by:

|  | SI= Ppeak- PminPpeak+ Pmin ×100% | (259) |
| --- | --- | --- |

For a geostationary satellite, YI(q) can change to reflect the latitude of earth facilities along the sub-satellite meridian. Start with the value associated with the latitude and time percentage from Fig. 16. This technique uses data measured at 136 MHz (Y136(q)). To calculate YI(q), find the index n by:

|  | n= 1 for θFL ≤17°1+ θFL-177 for 17° < θFL <24° 2 for 24° < θFL <45°1+ 52°- θFL7 for 45° < θFL <52°1 for θFL ≥52° | (260) |
| --- | --- | --- |

where θFL is the magnitude of the earth facility latitude in degrees. Equation (261) shows the conversion from Y136(q) to YI(q).

|  | YIq= 136fn Y136(q) | (261) |
| --- | --- | --- |

This method has a process for mixing scintillation indices. The process for mixing the distributions is described in § 11.5.

Figure 16

Signal level distribution for ionospheric scintillation index groups

## 11.5	Mixing distributions

This process combines distributions for time blocks (§ 11.1.1) to produce distributions for summer and winter or across boundaries for different climate types. The process can, also, be used for combining distributions for fading associated with ionospheric scintillation index groups (§ 11.4).

1	Select M ( ≥ 10) levels of variability that cover the entire range of the variable (e.g. transmission loss) (V1,…Vi, …, VM).

2	Identify the fraction of time (weighting factor) over which each distribution is applicable (W1, …Wi, …, WN).

3	Determine the time percentage applicable for the variable level is not to be exceeded (q11, …, qij, …, qMN).

4	Calculate the time percentages for each level of variability from step 1.

q1 = q11W1 + … + q1jWj + … + q1NWN

.

.

.

qi = qi1W1 + … + qijWj + … + qiNWN

.

.

.

qM = qM1W1 + … + qMjWj + … + qMNWN

For distributions of long-term variability, the variability functions are from:

|  | VCq= V0.5+Y(q)¦C | (262) |
| --- | --- | --- |

The |C indicates that V(0.5) and V(q) are the appropriate values for the conditions associated with the situation C. After the mixing, Y(q) is calculated by:

|  | Yq=Vq-V(0.5) | (263) |
| --- | --- | --- |

For scintillation index values covering an identified condition, the YI(q) from the mixing is the value for the condition.

Attachment 1

# 1	Ray tracing

In cases where there are high (3 km) antennas, the ray bending technique used in IF77 may overestimate ray bending. In these cases, the IF77 method uses a ray tracing technique which is covered in this section.

This algorithm assumes a horizontally stratified atmosphere defines the layer heights beginning at hr and rising for layers in km of 0.0, 0.01, 0.02, 0.05, 0.1, 0.2, 0.5, 0.7, 1.0, 1.524, 2, 3.048, 5, 7, 10, 20, 30.48, 50, 70, 90, 110, 225, 350 and 475. These values are contained in Hlayers (I). The initial elevation angle can be negative. Any angle that is excessively negative is set to the grazing angle for a smooth earth.

For each element of Hlayers:

|  | Nlayers=exp-Ce Hlayers(I) Ns 10-6 | (264) |
| --- | --- | --- |

where Ce (a scale height coefficient) equals the term loge(Ns/(Ns+ΔN)) from equation (15) and Ns is from equation (13).

|  | NII=1 + EN(I) | (265) |
| --- | --- | --- |
|  | RI=a0+ hr + HlayersI | (266) |
|  | hT1= as+ hr+ hr1 km | (267) |
|  | hT2= as+ hr+ hr2 km | (268) |
|  | N1= Ns 10-6exp-Ce hr1 | (269) |
|  | N2= Ns 10-6exp-Ce hr2 | (270) |
|  | NI1=1+ N1 | (271) |
|  | NI2=1+ N2 | (272) |

where hr, hr1,2 are defined in § 4.1. Subscripts T1 and T2 denote terminals (antennas) one and two respectively throughout this section.

Starting with element 2 of Hlayers (I = 2):

|  | AI= logeNII- logeNII-1logeRI- logeRI-1 | (273) |
| --- | --- | --- |

Use equations (274) to (280) starting at the first Hlayers value above hr1 for each layer indicated by index I until the Hlayers value is greater than hr2:

|  | X = hRT12 R(I) | (274) |
| --- | --- | --- |
|  | Y=2 sin22 θT1 | (275) |

where θT1 is the absolute value of the take-off angle (shown as –θsR1 in Fig. 4).

|  | Z= RI- hT1hT1 | (276) |
| --- | --- | --- |
|  | W= N1- Nlayers(I) cosθT1NI(I) | (277) |
|  | θII=2 sin-1X Y+Z-W | (278) |
|  | XA= -AI(I)AI(I)+1 | (279) |
|  | θall=θlast+ θI(I)- θI(I-1) XA | (280) |
|  | θlast=θall | (281) |
|  | XB=NIlast R(last)hr2 | (282) |

When equations (274) to (282) are done for the value of Hlayers > hr2 or the index, I, equals 25, angle at the higher antenna and the great circle distance between antennas 1 and 2 are:

|  | θF= θI(I) if hr2<475 kmcos-1cosθI(I)XB otherwise | (283) |
| --- | --- | --- |
|  | dT1= as θF+ θall- θT1 | (284) |

# 2	Counterpoise or ground plane

## 2.1	Effective reflection coefficient

The presence of a counterpoise affects the effective reflection coefficient of the system. The effective reflection coefficient related to a counterpoise, RTc, uses a divergence, Dv, ray length factors, Fr, a surface roughness factor, Fσh, counterpoise factor, fc, antenna gain factor for the counterpoise, gc, and the plane earth reflection coefficient magnitude, Rc.

|  | RTc=DvFrFσhfcgRcRc for counterpoise present0 otherwise | (285) |
| --- | --- | --- |

In the counterpoise case, DvFrFσh is assumed equal to unity since the counterpoise would be near the lower antenna and flat. Therefore they are not considered in this section.

## 2.2	Counterpoise factors

The counterpoise factors reduce the amount of reflection for both the counter poise and the ground reflections when a counterpoise is present. The geometry of the counterpoise is shown in Figs 17 and 18.

|  | θce= tan-12 hfcdc | (286) |
| --- | --- | --- |

where hfc is the height of antenna 1 above the counterpoise from equation (20) and dc is the counterpoise diameter which is an input parameter.

Figure 17

Geometry for earth reflection diffraction parameter, vg, with counterpoise shadowing

Figure 18

Geometry for earth reflection diffraction parameter, vc, with limited surface of counterpoise

|  | rc= 0.5 dccosθce | (287) |
| --- | --- | --- |
|  | θkg= θce- θr1 | (288) |

where θr1 is from equation (110).

|  | Yv= 8 rcλ | (289) |
| --- | --- | --- |

where λ is the wavelength in km equal to λm/1 000 which λm is from (9).

|  | θkc= θce- θh1 | (290) |
| --- | --- | --- |

where θh1 is calculated using counterpoise parameters in (100) through (109).

|  | vg= ± Yv sinθkg2 - for θr1< θce+ otherwise | (291) |
| --- | --- | --- |
|  | vc= ± Yv sinθkc2 - for θh1> θce+ otherwise | (292) |
|  | Cg,c= 0vg,ccosπt22 dt | (293) |
|  | Sg,c= 0vg,csinπt22 dt | (294) |

where Cg,c and Sg,c are Fresnel integrals.

|  | fc,g=0.5 1- Cg,c+Sg,c2+ Cg,c- Sg,c2 | (295) |
| --- | --- | --- |
|  | kg,c= tan-1Cg,c- Sg,c1- Cg,c- Sg,c | (296) |

The value fg is used in (113), fc is used for RTc is equation (283) and φkg,c are used in (158) and (159) in § 6.4.

# 3	Cumulative distribution

For long term fading in § 10.1, to calculate percentiles for hourly received power starts with the Y(0.9) and Y(0.1) values calculated in equation (223). The assumption for this distribution is that it is normal or nearly normal. The equation for other percentiles is equation (224). The values for c use the Qi(q/100) values where

|  | c= Qi(q)Qi(0.9) | (297) |
| --- | --- | --- |

for Y(0.9) and a defined q and

|  | c= Qi(q)Qi(0.1) | (298) |
| --- | --- | --- |

for Y(0.1) and a defined q. The value for q = 0.5 here is zero and, so, is not included in the calculations. Qi(q/100) values can be found in Table 17.

TABLE 17

Approximate inverse complementary cumulative normal distribution values 
[Rec. ITU-R P.1546 Table 3]

| q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.327 | 26 | 0.643 | 51 | –0.025 | 76 | –0.706 |
| 2 | 2.054 | 27 | 0.612 | 52 | –0.050 | 77 | –0.739 |
| 3 | 1.881 | 28 | 0.582 | 53 | –0.075 | 78 | –0.772 |
| 4 | 1.751 | 29 | 0.553 | 54 | –0.100 | 79 | –0.806 |
| 5 | 1.645 | 30 | 0.524 | 55 | –0.125 | 80 | –0.841 |
| 6 | 1.555 | 31 | 0.495 | 56 | –0.151 | 81 | –0.878 |
| 7 | 1.476 | 32 | 0.467 | 57 | –0.176 | 82 | –0.915 |
| 8 | 1.405 | 33 | 0.439 | 58 | –0.202 | 83 | –0.954 |
| 9 | 1.341 | 34 | 0.412 | 59 | –0.227 | 84 | –0.994 |
| 10 | 1.282 | 35 | 0.385 | 60 | –0.253 | 85 | –1.036 |

TABLE 17 (end)

| q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 1.227 | 36 | 0.358 | 61 | –0.279 | 86 | –1.080 |
| 12 | 1.175 | 37 | 0.331 | 62 | –0.305 | 87 | –1.126 |
| 13 | 1.126 | 38 | 0.305 | 63 | –0.331 | 88 | –1.175 |
| 14 | 1.080 | 39 | 0.279 | 64 | –0.358 | 89 | –1.227 |
| 15 | 1.036 | 40 | 0.253 | 65 | –0.385 | 90 | –1.282 |
| 16 | 0.994 | 41 | 0.227 | 66 | –0.412 | 91 | –1.341 |
| 17 | 0.954 | 42 | 0.202 | 67 | –0.439 | 92 | –1.405 |
| 18 | 0.915 | 43 | 0.176 | 68 | –0.467 | 93 | –1.476 |
| 19 | 0.878 | 44 | 0.151 | 69 | –0.495 | 94 | –1.555 |
| 20 | 0.841 | 45 | 0.125 | 70 | –0.524 | 95 | –1.645 |
| 21 | 0.806 | 46 | 0.100 | 71 | –0.553 | 96 | –1.751 |
| 22 | 0.772 | 47 | 0.075 | 72 | –0.582 | 97 | –1.881 |
| 23 | 0.739 | 48 | 0.050 | 73 | –0.612 | 98 | –2.054 |
| 24 | 0.706 | 49 | 0.025 | 74 | –0.643 | 99 | –2.327 |
| 25 | 0.674 | 50 | 0.000 | 75 | –0.674 |  |  |

Attachment 2

Abbreviations, acronyms and symbols

This list includes most of the abbreviations, acronyms, and symbols used in this report. The units given for symbols in this list are those required by or resulting from equations as given in this report and are applicable except when other units are specified.

In the following list, the English alphabet precedes the Greek alphabet, letters precede numbers, and lower-case letters precede upper-case letters. Miscellaneous symbols and notations are given after the alphabetical items.

a	effective earth radius calculated from eq. (14) (km)

aa	an effective earth radius associated with ka in the two ray model in the LoS region, eq. (98) (km)

a0	actual earth radius, 6 370 km

a1,2,3,4	effective earth radii from diffraction calculations, eq. (55) (56)

aγ	effective earth radius used in atmospheric absorption from § 10 (km)

A	an intermediate parameter for tropospheric scatter attenuation, eq. (198)

Aa	atmospheric absorption attenuation, eq. (212) (dB)

AA	diffraction attenuation intercept for path O-A, eq. (92) (dB)

Ad	diffraction attenuation for beyond the horizon paths, eq. (94) (dB)

Adx	Ad at dx, eq. (205) (dB)

Ae,q,t	angles from Fig. 12 (rad)

AF	attenuation path intercept for diffraction path F-O-ML, eq. (71) (dB)

AI	effective area of an isotropic antenna, eq. (8) (dB sq. m)

AKA	knife-edge diffraction attenuation for O-A path, eq. (88) (dB)

AKML	knife-edge diffraction attenuation for F-O-ML path, eq. (81) (dB)

AK5	knife-edge diffraction attenuation at d5, eq. (153) (dB)

ALoS	attenuation in the LoS region, eq. (162) (dB)

Am	upper limit of the ray tracing gradient, eq. (169) (km–1)

AML	diffraction attenuation for F-O-ML path, eq. (90) (dB)

A0	AR0 at d0, eq. (161) (dB)

Ap	attenuation path intercept for rounded earth diffraction, eq. (70) (km)

ArA	rounded earth diffraction attenuation for the O-A path, eq. (72) (dB)

ArF	rounded earth diffraction attenuation for the F-O-A path, eq. (71) (dB)

ArML	rounded earth diffraction attenuation for the F-O-ML, eq. (71) path

ARO	attenuation in the LoS region where the diffraction effects associated with terrain are negligible, eq. (160) (dB)

Arr(q)	attenuation rate associated with rain and a fraction of time, q, step 3 in § 11.3 (dB/km)

Ar5	rounded earth diffraction attenuation for the F-O-A path at d5, eq. (154) (dB)

AS	forward scatter attenuation, eq. (163) (dB)

ASX	forward scatter attenuation at dx, eq. (205) (dB)

AT	terrain attenuation, eq. (205) (dB)

AY	conditional adjustment factor used to prevent available signal powers form exceeding levels expected for free space propagation by unrealistic amounts, eq. (234) (dB)

AYI	initial value for conditional adjustment factor, AY, eq. (233) (dB)

A3,4 	intermediate parameters for rounded earth diffraction calculations, eq. (68) (dB)

A5 	combined diffraction attenuation at d5, eq. (155) (dB)

b1,2,3	long term variability parameters for eq. (237) from Table 12

B	intermediate parameter for Gɦ1,2, eq. (77)

BN1,2	parameters used for rounded earth calculations, eq. (73)

BS	intermediate parameter for scattering volume, SV, eq.(202)

B1,2,3,4	intermediate parameters for rounded earth diffraction calculations, eq. (60)

c	cumulative distribution for long term fading calculations, eq. (228) and Attachment 1

ce,h,v	phase of plane earth reflection coefficient relative to π for elliptical, horizontal, and vertical polarisations, eqs. (138) (139)

cY	variability limiting parameter, eq. (236) (dB)

c1,2,3	parameters for long term fading calculations, eqs. (225) (226) (237) from Tables 6, 7, 8

CS	intermediate parameter for tropospheric scatter calculations, eq. (203)

Cν,g,c,5	Fresnel integrals, eqs. (85) (293) (152)

D	great circle distance between lower and higher antennas (may be an input or calculated by eq. (112) (km)

dA	facility to aircraft distance, eq. (91) (km)

dc	counterpoise diameter, eq. (286) (km)

dd	initial estimate of d0, eq. (156) (km)

de	effective distance for long term power fading, eq. (221) (km)

dh	distance used to calculate facility horizon, eq. (31) (km)

dL1,2	horizon distances for the lower (1) and higher (2) antennas § 4.2, § 4.3 (km)

dLE1	horizon distances for the lower (1) and higher (2) antennas based on effective earth radius, eq. (32) (km)

dLM	greatest distance between the lower and higher antennas where there is a common horizon, eq. (42) (km)

dLO1,2	smooth earth horizon distances for lower path O-A, eqs. (48) (49) (km)

dLp	total horizon distance for the path for diffraction, eq. (52) (km)

dLp1,2	radio horizon distances for lower (1) and higher (2) antennas for the path for diffraction, eqs. (48) (49) (km)

dLq	total smooth earth horizon distance determined by ray tracing, equals dT1 for aS = 9 000 km, eq. (284) (km)

dLR2	horizon distance for higher antenna discussed before eq. (40) (km)

dLs	total smooth earth horizon distance between the lower and higher antennas, eq. (25) (km)

dLsA	smooth earth horizon distance for O-A path, eq. (82) (km)

dLs1,2	smooth earth horizon distance for the lower (1) and higher (2) antennas, eq. (24) (km)

dLsE1,2	effective smooth earth horizon distances for the lower (1) and higher (2) antennas, eq. (23) (km)

dLsR1,2	smooth earth horizon distances for the lower (1) and higher (2) antennas § 4.1 Fig. 3 (km)

dLO1,2	smooth earth horizon distances for path O-A in eqs. (48) (49) (km)

dL1,2	total smooth earth horizon distance between the lower and higher antennas, eq. (25) (km)

dL5	intermediate distance in LoS region, eq. (145) (km)

dML	maximum LoS distance from the lower antenna, eq. (40) (km)

dq	intermediate distance in calculating de, eq. (222) (km)

ds	scattering distance, eq. (165) (km)

dsL	smooth earth horizon distance with an obstacle horizon, eq. (41) (km)

dx	distance just beyond the radio horizon discussed after eq. (205) (km)

dz1,2	distances used in tropospheric scatter calculations, eqs. (167) (168) (km)

dZ1,2	distances used in tropospheric scatter calculations, eqs. (184) (185) (km)

D/U (q)	desired to undesired signal ratio, eq. (10) (dB)

Dv	divergence factor, eq. (116)

d0	largest distance in the LoS region where the diffraction component is negligible, eq. (157) (km)

d3,4	distances that are intermediate values in the rounded diffraction calculations, eqs. (53) (54) (km)

d5	intermediate distance in LoS region, eq. (146) (km)

D1,2	distances associated with θ1,2 respectively, eq. (103) (km)

e.i.r.p.	equivalent isotropically radiated power, eq. (6) (dBW)

f	frequency, input parameter (MHz)

fc	counterpoise factor for RTc, eq. (295)

fc1,2	intermediate parameter used to calculate the residual weighted height function, eq. (75)

fg	counterpoise factor for RTg, eq. (295)

fm,∞	factors used in long term variability discussed after eq. (226)

fν,5	knife-edge diffraction factors, eqs. (87) (152)

f2	factor used in long term variability, eq. (225)

fθh	factor used in long term variability, eq. (229)

F	fade margin, eq. (245)

FAY	specular reflection reduction factor, eq. (240)

Fdσh	surface roughness factor for diffuse reflections, eq. (122)

Ffs	LoS attenuation factor, eq. (158)

Fr	ray length factor, eq. (117)

Fx1,2	intermediate parameters for rounded earth diffraction calculations, eq. (67)

F1,2	intermediate parameters for rounded earth diffraction calculations, eq. (66)

FΔr	reflection reduction factor, eq. (241)

Fσh	surface roughness factor for specular reflections, eq. (121)

g(0.1,f)	frequency gain factor, eq. (223)

g(0.9,f)	frequency gain factor, eq. (224)

gD,R	voltage gain factors, eqs. (125) (126)

gD1,2	voltage gain factors Table 3

ghD1,2	voltage gain factors, direct ray, horizontal polarization discussed after eq. (126)

ghR1,2	voltage gain factors, reflected ray, horizontal polarization discussed after eq. (126)

gRh	gain factor, reflected ray, horizontal polarization, eq. (128)

gR1,2	voltage gain relative to boresight discussed in § 6.2.3

gvD1,2	voltage gain factors, direct ray, vertical polarization discussed after eq. (126)

gvR1,2	voltage gain factors, reflected ray, horizontal polarization discussed after eq. (126)

GET,R	free space antenna gains for transmitting (T) and receiving (R) antennas, eq. (4) (dBi)

GNT,R	relative gain of a elevation angle for transmitting (T) and receiving (R) antennas discussed after eq. (4) (dBi)

GR1,2	antenna gains, eq. (124) (dBi)

GT,R	main beam free space antenna gains for transmitting (T) and receiving (R) antennas discussed after eq. (4) (dBi)

GW1,2	weighting factor for height gain function, eq. (76) (dBi)

G1,2,3,4	intermediate parameters for rounded earth diffraction calculations, eq. (63) (dB)

Gɦ1,2	values for the residual height gain function, eq. (77) (dB)

GɦF1,2	residual height gain factor for F-O-ML path, eq. (79) (dB)

GɦA1,2	residual height gain factor for O-A path, eq. (80) (dB)

Gɦp1,2	residual height gain factor for a defined path, eq. (78) (dB)

h	height above mean sea level, eq. (15) (dB)

hc	height of the counterpoise or ground plane above the facility height (hsml), input parameter (km)

he	effective antenna height discussed before eq. (31) (km)

he1,2	effective heights for the lower (1) and higher (2) antennas, eq. (21) (km)

hem2	effective aircraft altitude, eq. (144) (km)

hep1,2	effective heights for lower (1) and higher (2) antennas for paths defined for diffraction, eq. (47) (km)

hfc	height of the lower antenna above the counterpoise or ground plane, eq. (19) (km)

hL1,2	horizon elevations above mean sea level for the lower (1) and higher (2) antennas (36), eq. (45) (km)

hLE1,2	horizon elevation for lower (1) and higher (2) antennas (29) (km)

hLr1,2	horizon elevations above effective reflecting surface for the lower (1) and higher (2) antennas, eqs. (37) (44) (km)

hr	reflecting surface height, input parameter (km)

hr1,2	heights above the effective reflection surface for the lower (1) and higher (2) antennas, eq. (18) (km)

hsl	height of lower antenna above the facility height, input parameter (km)

hsml	height of the facility above mean sea level, input parameter (km)

hv	height of the scattering volume, eq. (186) (km)

h1,2	heights above mean sea level for lower (1) and higher (2) antennas, eq. (18) (km)

ɦ1,2	intermediate values for smooth earth diffraction calculations for lower (1) and higher (2) antennas, eq. (74)

Hc,q,t,z	heights defined for atmospheric absorption calculations in Fig. 12 (km)

H1,2	antenna heights above and perpendicular to the reflective surface, eqs. (100 a&b) (km)

H1/3	values for significant wave height in Table 2 used in eq. (119) (m)

Hϒ1,2	antenna heights above and perpendicular to the surface of the earth in Fig. 12 (km)

H'1,2	antenna heights used in determining the two ray path, eq. (104) (km)

ka	adjusted earth radius factor in the two ray method in LoS region, eq. (97)

K	ratio between steady component of received power and the Rayleigh fading component used to determine correct Nakagami-Rice phase interference fading distribution value for tropospheric multipath calculations from Table 13 (dB)

Kd1,2,3,4	intermediate parameters for rounded earth diffraction calculations, eq. (57)

KF1,2	intermediate parameters for rounded earth diffraction calculations, eq. (59)

KLoS	parameter for tropospheric multipath calculations, eq. (246)

KML	KLoS value at d = dML, eq. (246)

KT	parameter for tropospheric multipath calculations, eq. (246)

K1,2,3,4	intermediate parameters for rounded earth diffraction calculations, eq. (58)

l	total ray length for the tropospheric scattering attenuation, eq. (189) (km)

l1,2	ray lengths for the tropospheric scattering attenuation, eq. (188) (km)

L(q)	transmission loss not exceeded during time period q, eq. (1) (dB)

Lb(0.5)	median basic transmission loss, eq. (2) (dB)

Lbf	free space basic transmission loss, eq. (206) (dB)

Lbr	reference level for the basic transmission loss, eq. (3) (dB)

Lgp	antenna gain path loss, eq. (1) (dB)

LoS	Line of Sight

msl	mean sea level

M	number of transmission loss levels used in mixing distributions, § 11.5

MA	attenuation path slope for diffraction for O-A path defined in § 5.1 (dB/km)

Md	attenuation path slope for combined diffraction of beyond-the-horizon paths, eq. (93) (dB/km)

MF	slope of K value defined in § 5.1 (dB/km)

ML	slope of the diffraction attenuation line just inside the radio horizon, eq. (161) (dB/km)

Mp	attenuation path slope for diffraction, eq. (69) (dB/km)

MS	slope of successive scatter attenuation points discussed after eq. (205) (dB/km)

n	index used in ionospheric scintillation calculations, eq. (260)

n1,2,3	values from Table 6 used in long term fading calculations, eq. (226)

N	refractivity for height h in an exponential atmosphere, eq.(15) (N-units)

N	number of distributions to be mixed, § 11.5

N0	monthly mean surface refractivity, eq. (13) (N-units)

NS	surface refractivity relative to the reflecting surface height, eq. (13) (N-units)

Pa(q)	power available for at least time period q, eq. (5) (dBW)

PTR	total power radiated by the transmitting antenna, eq. (6) (dBW)

q	fraction of time availability

q1,2	intermediate parameters used in tropospheric scattering calculations, eq. (201)

Qa1,2	intermediate parameters used in tropospheric scattering calculations, eq. (174)

QA1,2	intermediate parameters used in tropospheric scattering calculations, eq. (178)

Qb1,2	intermediate parameters used in tropospheric scattering calculations, eq. (175)

QB1,2	intermediate parameters used in tropospheric scattering calculations, eq. (179)

QO1,2	intermediate parameters used in tropospheric scattering calculations, eq. (173)

r	ray length used in the free space loss calculation, eq. (208) (km)

rBH	ray length for beyond the horizon paths, eq. (210) (km)

rc	distance in a counterpoise calculation, eq. (287) (km)

rL1,2	ray length from antenna 1 or antenna 2 to horizon, eq. (209) (km)

r0	the length of the direct ray from antenna 1 to antenna 2 in the two ray model, eq. (106) (km)

rs	in-storm ray length, eq. (257) (km)

rWH	ray length for within the horizon paths, eq. (209) (km)

r1,2	the length of the direct ray from antenna 1or antenna 2 to the ground in the two ray model Fig. 9 (km)

r12	the length of the reflected ray from antenna 1 to antenna 2 in the two ray model, eq. (107) (km)

Rc	counterpoise reflection coefficient magnitude discussed after eq. (140)

Rd	diffuse reflection coefficient, eqs. (123) (243)

Re,h,v	reflection coefficient magnitudes for elliptical, horizontal, or vertical polarisation, eq. (138)

Reo,s,w	partial effective ray lengths inside oxygen layer, rain storm layer, or water vapour layer, eqs. (217) (219) § 11.3 (km)

Rg	plane earth reflection coefficient magnitude, eq. (140)

Rr	reflected ray length ratio, eq. (115)

Rs	specular component of surface reflection multipath, eq. (242)

RTc	effective reflection coefficient associated with counterpoise reflection, eq. (285)

RTg	effective reflection coefficient associated with ground reflection, eq. (113)

S	modulus of symmetry for tropospheric scattering, eq. (194)

Se	scattering efficiency term for the tropospheric scattering attenuation, eq. (193)

SR(q)	power density at receiving antenna for at least time period q, eq. (7) (dBW/sq. m)

Sν,g,c,5	Fresnel integrals, eqs. (86) (294) (153)

SV	scattering volume for tropospheric scatter calculations, eq. (204) (dB)

T	intermediate parameter for Gɦ1,2, eq. (77)

T	relaxation time used in the calculation of water surface constants from Table 4

Teo,s,w	storm height or layer thickness for oxygen layer, rain storm layer, or water vapour layer discussed in § 10.1.1 and § 11.3 (km)

V	variability levels used in mixing distributions, § 11.5

V(0.5)	parameter from eqs. (226) (237)

Vc	variability for specific climate or time block, eq. (262)

Ve(0.5)	variability adjustment term, eq. (231)

VeI(q)	initial value of variability adjustment term, eq. (230)

W	weighting factor between rounded earth and knife-edge diffraction, eq. (89)

W	weighting factors used in mixing distributions, § 11.5

WA	relative power level for the Rayleigh fading component associated with tropospheric multipath, eq. (248)

WR	relative power level for the Rayleigh fading component associated with tropospheric multipath, eq. (244)

WRO	relative power level for the Rayleigh fading component associated with surface reflection multipath, eq. (159)

W1,2	intermediate parameters for rounded earth diffraction calculations, eq. (64)

Xa	parameter used in tropospheric scatter calculations, eq. (183)

XA1,2	parameter used in tropospheric scatter calculations, eq. (187)

Xv1,2	parameter used in tropospheric scatter calculations, eqs. (199) (200)

X1,2,3,4	intermediate parameters for rounded earth diffraction calculations, eqs. (61) (62)

y1,2	intermediate parameters for rounded earth diffraction calculations, eq. (65)

Y(q)	variability of hourly median received power about the median, eq. (228) (dB)

Y(0.1)	reference variability level of hourly received power of the channel used to calculate other percentiles, eq. (227) (dB)

Y(0.9)	reference variability level of hourly received power of the channel used to calculate other percentiles, eq. (227) (dB)

YC	complex parameter used in the calculation of the plane earth reflection coefficient, eq. (262)

YDU(q)	total variability of desired to undesired signal ratios YDU(0.5) = 0, eq. (12)

Ye(q)	effective variability of hourly median received power about the median, eqs.(235) (236) (dB)

YeI(q)	initial value of the effective variability of hourly median received power about the median, eq. (230) (dB)

YI(q)	variability associated with ionospheric scintillation, eq. (261) (dB)

Y0(0.1)	reference variability level used to calculate Y(0.1), eqs. (226) (237) (dB)

Y0(0.9)	reference variability level used to calculate Y(0.9), eq. (226) (237) (dB)

Yr(q)	variability associated with rain attenuation, eq. (258) (dB)

YT(q)	top limiting level in the lobing mode in the LoS region, eq. (232)

Yν	intermediate parameter to calculate νg, eq. (289)

Y136(q)	variability that is associated with ionospheric scintillation at 136 MHz, § 11.4

Yπ(q)	short term variability associated with multipath, Fig. 4 (dB)

Y(q)	total variability, eq. (220) (dB)

z	intermediate parameter for the two ray model in the LoS region, eq. (96)

za1,2	intermediate parameters used in tropospheric scatter calculations, eq. (171) (km)

zb1,2	intermediate parameters used in tropospheric scatter calculations, eq. (172) (km)

z1,2	heights of antennas including the effective earth radius, eq. (101) (km)

Za1,2	intermediate parameters used in tropospheric scatter calculations, eq. (176) (km)

Zb1,2	intermediate parameters used in tropospheric scatter calculations, eq. (177) (km)

ZA1,2	intermediate parameters used in tropospheric scatter calculations, eq. (182) (km)

α	angle from Fig. 9, eq. (109) (rad)

β	angle used in atmospheric absorption calculations defined in § 10.1

γ	index for parameters used in atmospheric absorption calculations in § 10.1

γe	intermediate parameter used in tropospheric scatter calculations, eq. (170)

γoo,w	surface absorption rates for oxygen, water vapor, Fig. 13

δ	parameter used in surface roughness factor calculation, eq. (120)

∆h	terrain height, input parameter (km)

∆ha1,2	effective altitude correction factors of the lower (1) and higher (2) antennas, eq. (99) (km)

∆hd	interdecile range of terrain heights, eq. (118) (m)

∆he1,2	difference between the actual and effective heights of the lower (1) and higher (2) antennas, eq. (22) (km)

∆hm	∆h expressed in metres in eq. (118)

∆N	differential refractivity for ray tracing, eq. (16) (N-units)

∆r	difference between the lengths of the direct ray and the reflected ray in the two ray model, eq. (95) (km)

ε	dielectric constant, input parameter

εc	complex dielectric constant, eq. (136)

εo	dielectric constant representing the sum of electronic and atomic polarisations, § 6.2.4

εs	static dielectric constant for water, Table 4

ε1,2	parameters used in tropospheric scatter calculations, eqs. (190) (191)

η	parameter used in tropospheric scatter calculations, eq. (195)

ϑ	angle in tropospheric scatter calculations, eq. (192) (rad)

λm	wavelength of defined frequency in metres, eq. (9) (m)

θ	scattering angle for the tropospheric scattering attenuation, As, eq. (181) (rad)

θa1,2	angles used in calculating the scattering angle, eq. (164) (rad)

θA1,2	angles used in tropospheric scatter calculations, eq.(180) (rad)

θce	counterpoise angle defined in Fig. 17, eq. (286) (rad)

θe1,2	horizon elevation angles for lower (1) and higher (2) antennas, eqs. (38) (46) (rad)

θeE1	horizon elevation angle for intermediate calculations, eq. (33) (rad)

θesR1,2	ray elevation and central angles for lower (1) and higher (2) antennas from ray tracing, § 4.1 (rad)

θes1,2	effective ray elevation and central angles for lower (1) and higher (2) antennas, eq. (27) (rad)

θe5	angle used in the LoS transition region calculations, eq. (147) (rad)

θg1,2	elevation angles of the ground reflected rays, eq. (133) (rad)

θh1,2	parameters to calculate θH1,2, eq. (109) (rad)

θh5	angle in eq. (142) (rad)

θH1,2	direct ray elevation angles, eq. (132) (rad)

θFL	magnitude of earth facility latitude, input parameter (deg)

θkg,c	angles associated with a counterpoise defined in Figs 17, 18, eqs. (288) (290) (rad)

θL	elevation angle of horizon to higher antenna ray from lower antenna horizon, eq. (39) (rad)

θL1,2	horizon elevation angle adjustment terms, eq. (131) (rad)

θr1,2	angles used in two ray geometry calculations, eq. (110) (rad)

θs	angle used in scattering region calculations, eq. (166) (rad)

θsR1,2	ray elevation angles for lower (1) and higher (2) antennas from Fig. 4, eq. (20) (rad)

θs1,2	smooth earth ray elevation and central angles for lower (1) and higher (2) antennas, eq. (26) (rad)

θν	diffraction angle for the O-A path, eq. (83) (rad)

θ0	central angle used in two ray geometry calculations, eq. (111) (rad)

θ1,2	angles associated with effective earth radius from Fig. 9, eq. (102) (rad)

θ3,4	angles that are intermediate values for rounded earth diffraction calculations, eqs. (50) (51) (rad)

θ5	angle used in the LoS transition region calculations, eq. (143) (rad)

θ6	angle used in the LoS transition region calculations, eq. (148) (rad)

κ	wave number, eq. (196) (km–1)

σ	surface conductivity, input parameter (mho/m)

σh	root mean square deviation of terrain and terrain clutter within the limits of the first Fresnel zone, eq. (119) (m)

σi	ionic conductivity for water from Table 4 (mho/m)

φ	phase advance associated with complex earth reflection coefficient (rad)

φg,c	phase shift of effective reflection coefficient for ground or counterpoise reflection, eqs. (113) (285) (rad)

φkg,c	phase shift for knife-edge diffraction for ground or counterpoise, eq. (296) (rad)

φTg,c	total phase shift of effective reflection coefficient for ground or counterpoise reflection, eq. (141) (rad)

ɦ1,2	intermediate parameters in diffraction calculations, eq. (74)

δ	parameter in Fresnel zone calculations, eq. (120)

ψ	grazing angle from Fig. 9 discussed in § 6.1 (rad)

ν	Fresnel parameter for knife edge diffraction calculations, eq. (84) (rad)

νg,c,5	knife-edge diffraction parameters, eqs. (291) (292) (149)

Attachment 3

Data sets from Figs 13 and 16

Gaseous absorption data from Fig. 13:

| Frequency (GHz) | Oxygen absorption | Water vapour absorption |
| --- | --- | --- |
| 0.1 | 0.00019 | 0 |
| 0.15 | 0.00042 | 0 |
| 0.205 | 0.0007 | 0 |
| 0.3 | 0.00096 | 0 |
| 0.325 | 0.0013 | 0 |
| 0.35 | 0.0015 | 0 |
| 0.4 | 0.0018 | 0 |
| 0.55 | 0.0024 | 0 |
| 0.7 | 0.003 | 0 |
| 1 | 0.0042 | 0 |
| 1.52 | 0.005 | 0 |
| 2 | 0.007 | 0 |
| 3 | 0.0088 | 0 |
| 3.4 | 0.0092 | 0.0001 |
| 4 | 0.01 | 0.00017 |
| 4.9 | 0.011 | 0.00034 |
| 8.3 | 0.014 | 0.0021 |
| 10.2 | 0.015 | 0.009 |
| 15 | 0.017 | 0.025 |
| 17 | 0.018 | 0.045 |
| 20 | 0.02 | 0.1 |
| 22 | 0.021 | 0.22 |
| 23 | 0.022 | 0.2 |
| 25 | 0.024 | 0.16 |
| 26 | 0.027 | 0.15 |
| 30 | 0.03 | 0.11 |
| 32 | 0.032 | 0.14 |
| 33 | 0.035 | 0.1 |
| 35 | 0.04 | 0.099 |
| 37 | 0.044 | 0.098 |
| 38 | 0.05 | 0.0963 |
| 40 | 0.06 | 0.0967 |
| 42 | 0.07 | 0.0981 |
| 43 | 0.09 | 0.0987 |
| 44 | 0.1 | 0.099 |
| 47 | 0.15 | 0.1 |
| 48 | 0.23 | 0.101 |
| 51 | 0.5 | 0.103 |
| 54 | 1.8 | 0.109 |
| 58 | 4 | 0.118 |
| 59 | 7 | 0.12 |
| 60 | 15 | 0.122 |
| 61 | 8 | 0.127 |
| 62 | 5 | 0.13 |
| 63 | 3 | 0.132 |
| 64 | 1.7 | 0.138 |
| 68 | 1.2 | 0.154 |
| 70 | 0.9 | 0.161 |
| 72 | 0.5 | 0.175 |
| 76 | 0.35 | 0.2 |
| 84 | 0.2 | 0.25 |
| 90 | 0.14 | 0.34 |
| 100 | 0.1 | 0.56 |

Scintillation data from Fig. 16:

|  | Variability about median signal level (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scintillation index group | 0 | 1 | 2 | 3 | 4 | 5 |
| Time % |  |  |  |  |  |  |
| 0.001 | 0 | 4.4 | 5.8 | 10.8 | 13.7 | 14.5 |
| 0.002 | 0 | 4.2 | 5.5 | 10.2 | 13 | 13.8 |
| 0.005 | 0 | 3.9 | 5 | 9.2 | 12 | 13 |
| 0.01 | 0 | 3.6 | 4.7 | 8.2 | 11.1 | 12.4 |
| 0.02 | 0 | 3.4 | 4.4 | 7.6 | 10.3 | 11.7 |
| 0.05 | 0 | 3.05 | 3.95 | 6.5 | 9.2 | 10.7 |
| 0.1 | 0 | 2.8 | 3.65 | 5.8 | 8.4 | 10 |
| 0.2 | 0 | 2.55 | 3.35 | 5.2 | 7.5 | 9.5 |
| 0.5 | 0 | 2.15 | 2.9 | 4.35 | 6.35 | 8.2 |
| 1 | 0 | 1.85 | 2.55 | 3.8 | 5.45 | 7.5 |
| 2 | 0 | 1.55 | 2.2 | 3.25 | 4.55 | 6.55 |
| 5 | 0 | 1.1 | 1.65 | 2.5 | 3.4 | 5.2 |
| 10 | 0 | 0.8 | 1.2 | 1.8 | 2.55 | 4 |
| 15 | 0 | 0.6 | 0.95 | 1.4 | 2 | 3.2 |
| 20 | 0 | 0.4 | 0.75 | 1.1 | 1.6 | 2.6 |
| 30 | 0 | 0.2 | 0.45 | 0.7 | 0.98 | 1.6 |
| 40 | 0 | 0.05 | 0.2 | 0.3 | 0.5 | 0.75 |
| 50 | 0 | 0 | 0 | 0 | 0 | 0 |
| 60 | 0 | –0.15 | –0.3 | –0.35 | –0.5 | –0.8 |
| 70 | 0 | –0.35 | –0.55 | –0.7 | –1 | –1.7 |
| 80 | 0 | –0.6 | –0.9 | –1.2 | –1.8 | –2.95 |
| 85 | 0 | –0.7 | –1.1 | –1.5 | –2.4 | –3.8 |
| 90 | 0 | –0.9 | –1.4 | –1.9 | –3.1 | –5 |
| 95 | 0 | –1.3 | –2 | –2.6 | –4.4 | –7.05 |
| 98 | 0 | –1.8 | –2.6 | –3.5 | –6 | –9.8 |
| 99 | 0 | –2.15 | –3.1 | –4.2 | –7.2 | –12.2 |
| 99.5 | 0 | –2.55 | –3.65 | –4.95 | –8.5 | –15 |
| 99.8 | 0 | –3.05 | –4.4 | –6.05 | –10 | –19.1 |
| 99.9 | 0 | –3.5 | –5 | –7 | –11.1 | –22.2 |
| 99.95 | 0 | –4 | –5.6 | –7.9 | –12.2 | –25.8 |
| 99.98 | 0 | –4.6 | –6.5 | –9.1 | –14 | –30.2 |
| 99.99 | 0 | –5 | –7.2 | –10 | –15.4 | –34 |
| 99.995 | 0 | –5.5 | –8 | –11 | –16.7 | –37.2 |
| 99.998 | 0 | –6.2 | –9.2 | –12.4 | –18.4 | –42.5 |
| 99.999 | 0 | –6.6 | –10.1 | –13.4 | –19.5 | –46 |

Bibliography

[1]	Gierhart, G.D. and M.E. Johnson (1983), The Electromagnetic Wave Propagation Model, DOT Report FAA-ES-83-3.

[2]	Gierhart, G.D. and M.E. Johnson (1978), Propagation Model (0.1 to 20 GHz) Extensions for 1977 Computer Programs, DOT Report FAA-RD-77-129.

[3]	Gierhart, G.D. and M.E. Johnson (1973), Computer Programs for Air/Ground Propagation and Interference Analysis, 0.1 to 20 GHz, DOT Report FAA-RD-73-103.

[4]	Johnson, M.E. and G.D. Geirhart (1978), Applications Guide for Propagation and Interference analysis Computer Programs (0.1 to 20 GHz), DOT Report FAA-RD-77-60.

[5]	Rice, P.L., A.G. Longley, K.A. Norton and A.P. Barsis (1967), Transmission Loss Predictions for Tropospheric Communications Circuits, NBS Tech. Note 101, I and II revised.

[6]	Bean, B.R., and G.D. Thayer (1959), CRPL Exponential Reference Atmosphere, NBS Monograph 4.