Report ITU-R S.2362-0 (06/2015)

Methodology to estimate the sensitivity of GSO FSS interference levels to the geographical location of earth stations communicating with GSO satellites in the fixed-satellite service in the 14 GHz and 30 GHz frequency ranges

S Series Fixed satellite service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Reports (Also available online at http://www.itu.int/publ/R-REP/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |

Note: This ITU-R Report was approved in English by the Study Group under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2015

ITU 2015

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

REPORT  ITU-R  S.2362-0

Methodology to estimate the sensitivity of GSO FSS interference levels to the geographical location of earth stations communicating with GSO satellites in the fixed-satellite service in the 14 GHz and 30 GHz frequency ranges

(2015)

TABLE OF CONTENTS

Page

1	Introduction		2

2	Overview of the methodology		2

3	List of symbols		3

4	Model for the interference analysis		4

4.1	Statistical model for the terminal locations and their e.i.r.p. levels		5

4.2	Interference signal using actual locations		5

4.3	Interference estimated using quantized spatial locations		6

4.4	The difference between the interference PSDs		8

4.5	Interference PSD in the presence of antenna pointing errors		8

5	Simulation results		9

5.1	Results at 30 GHz		9

5.2	Results at 14 GHz		12

6	Discussion		15

7	Conclusion		16

Annex – A step-by-step method to estimate the interference PSDs		16

1	Estimating the interference PSD using the actual locations		16

2	Estimating the interference PSD using the grid points		17

# 1	Introduction

A network of earth stations using the multi-frequency time-division multiple access (MF-TDMA) protocol can be used to provide high-data rate services over the geostationary-satellite orbit (GSO) fixed-satellite service (FSS). Methodologies to assess the interference from such networks were addressed in Recommendation ITU-R S.2029.

Interference from an earth station to a victim receiver in an adjacent satellite network depends on the location of the interfering earth station. Therefore, it is useful to investigate the sensitivity of interference to the location of an earth station. This Report investigates the effect of the location accuracy of the earth station for determining the interference levels onto an adjacent GSO FSS satellite network. The document considers a network of small aperture earth stations that operate using a TDMA protocol. Moreover, the earth stations may operate in the presence of antenna pointing errors. Because of the antenna pointing errors, the TDMA protocol, and the different e.i.r.p. spectral density levels of the earth stations, the proposed methodology is based on a statistical analysis. This methodology quantifies the sensitivity of the interference levels to the geographical locations of the interfering earth stations. This methodology is suitable for estimating the interference from a network of earth stations to victim earth terminals in the GSO FSS.

This Report is organized as follows. Section 2 describes the analysis approach and provides an overview of the methodology. Section 3 contains a detailed list of symbols used in this document. Section 4 discusses the statistical model for the analysis of interference; it describes the approach used for approximating the locations of the terminals; and it gives a methodology to obtain the cumulative distribution functions (CDFs) of the interference using the exact location, the approximate location, and next it determines the difference in the interference power spectral density (PSD). Section 5 presents some simulation results for satellite networks operating at the 30 GHz and 14 GHz frequency ranges. Section 6 discusses the applications of the methodology and section 7 provides the conclusions.

# 2	Overview of the methodology

The methodology described in this Report is based on a statistical analysis of the interference PSD that is received at a victim receiver of an adjacent satellite network. It assumes that all link parameters necessary to compute this interference PSD are available. However, the actual locations of the terminals are not available but they are approximated by a set of predetermined grid points as described below. The location “error” of a terminal is given by the difference between the actual location of the terminal and its approximated value. The interference PSD computed using these approximated location data of the terminals could be greater or less than its actual value resulting in, respectively, overestimation or underestimation of the interference PSD. The difference in interference PSD between the actual and the approximated location data is computed and with this information the CDF of the difference in the interference PSD as a function of the location error of the terminals is obtained. Furthermore, the difference in the interference PSD is examined as a function of the distance between adjacent grid points and the e.i.r.p. density of the terminals.

The specific steps used in this analysis are described as follows.

–	Step 1. Compute the interference PSD to a victim receiver in an adjacent satellite network and estimate its CDF. This step makes use of the actual locations of the terminals and the relevant link parameters of the satellite networks.

–	Step 2. Estimate the interference PSD using the approximated locations of the terminals. Quantize the spatial region where the terminals are operating to a finite set of grid points. These grid points may be determined a-priori. The spacing between adjacent points determines the accuracy of the location of the terminal.

–	Step 2 (a). Approximate the location of each terminal to one of the grid points, preferably the nearest. The location error in this case is the difference between the actual location of the earth station and its approximated location to this grid point.

–	Step 2 (b). Estimate the interference PSD and its CDF, assuming that the locations of the terminals are given by these approximated grid points.

Carry out this analysis for different e.i.r.p. density values of the earth station. This is necessary to examine the effect of the e.i.r.p. density level on the estimated interference using the approximated locations.

–	Step 3. Determine the difference between the interference PSDs obtained using the actual locations of the terminals and their approximated locations. Estimate its CDF.

# 3	List of symbols

The symbols listed below are used in this Report:

βi.r :	angle at Satellite Si between directions SiCi and SiTi.

βv,r :	angle at Satellite Sv between directions SvCv and SvTi.

γi :	transmission gain from the output of receive antenna at Satellite Si to output of receive antenna at Rv.

γv :	transmission gain from the output of receive antenna at Satellite Si to output of receive antenna at Rv.

ϕ:	antenna pointing error at the interfering terminal.

ϕϵ, ϕa:	components of the antenna pointing error in the elevation and azimuth directions.

ψ:	off-axis angle, measured from the antenna boresight direction, at Ti.

ψcenter,v:	off-axis angle towards Sv at an interference terminal located at the beam center of Sv.

ψr,v:	angle at Ti between its boresight direction and direction TiSv.

Cv, Ci :	beam centers of Satellites Sv, Si on the Earth.

dx1,x2:	distance metric between points x1 and x2.

En:	boresight e.i.r.p. spectral density of interfering terminal. n=1,2,…nE.

Er:	boresight e.i.r.p. spectral density at Ti located at r.

Erψ:	e.i.r.p. spectral density in an off-axis angle ψ from Ti located at r.

Ered:	reduction in boresight e.i.r.p. spectral density to account for underestimation of the actual interference when quantized locations are used.

Gvs, Gis :	receive antenna gain pattern at Sv, Si.

Icenter:	interference power spectral density at the output of receive antenna of Rv due to a hypothetical interference terminal located at the beam center of Sv and transmitting at the ESD level specified by Recommendations ITU-R S.524-9 or S.728-1.

Ie,n,kr:	difference between the interference power spectral densities when they are computed using the actual and approximated locations and when the transmit terminal is in the Region Rk  with boresight e.i.r.p. spectral density En.

Iir:	interference power spectral density received via Satellite Si at the output of receive antenna of Rv.

Ii,qr:	interference power spectral density received via Satellite Si at the output of receive antenna of Rv when spatial locations are quantized.

Ivr:	interference power spectral density received via Satellite Sv at the output of receive antenna of Rv.

Iv,qr:	interference power spectral density received via Satellite Sv at the output of receive antenna of Rv when spatial locations are quantized.

Lu,r:	uplink loss from  Ti located at r to satellites Sv or Si.

nE :	number of different boresight e.i.r.p. spectral density levels.

pn,Er :	probability that the interfering terminal at r is transmitting at boresight e.i.r.p. spectral density level En.

pn,k,Er :	probability that an interfering terminal located in Region Rk is transmitting at boresight 	e.i.r.p. spectral density level En.

pr :	probability that the interfering terminal at r is transmitting data.

prk:	probability that the transmit terminal is in Region Rk.

r :	spatial location variable of the interfering terminal that is transmitting data.

rΔ:	distance between adjacent grid points.

rk :	quantized values of r. k=1,2,…K.

R :	spatial region where the interfering terminals are distributed.

Rk :	kth quantization region.

Rv :	victim receiver.

Sv, Si :	wanted and interfering satellites.

tk :	total time duration interfering terminals are transmitting from Region Rk.

tn,k :	total time duration interfering terminals are transmitting from Region Rk with boresight 	ESD level En.

Tv, Ti :	wanted and interfering transmit terminals.

# 4	Model for the interference analysis

A graphical representation of the interference model is shown in Fig. 1. The transmit terminals of the interfering network are denoted by Ti. This figure also shows the victim and interfering satellites, Sv and Si, wanted (victim) transmit terminal Tv, and victim receive terminal Rv. The spatial location of the Terminal Ti is denoted by the variable r, which is in general a three-dimensional variable corresponding to the spatial coordinates of that location. The interfering terminals use the multi-frequency (MF) time-division multiple access (TDMA) protocol with only a single terminal transmitting at a particular time instant in a narrow frequency band of interest. For modeling purposes, it is assumed that the Satellites Sv and Si employ the same frequency translation from the uplink to the downlink. If this is not the case, the downlinks from Sv and Si to Rv correspond to terminals from different interfering networks.

Figure 1

Interference paths from Terminal Ti to the victim receiver Rv, via Satellites Si and Sv. Ci and Cv denote 
the beam centers of Si and Sv on the Earth surface; O is the origin from which the distances 
to the terminal locations are measured

Further details of this interference model and methodologies to assess the interference are given in Recommendation ITU-R S.2029. The focus of this Report is on investigating the effect on the interference when the locations of the interfering terminals are approximated by a set of quantized values of r.

## 4.1	Statistical model for the terminal locations and their e.i.r.p. levels

In this analysis the interfering terminals of the earth station network transmit in a random manner, and in a given time instant only a single terminal transmits data. The location of this terminal is represented by the random variable r, and its probability density function (PDF) denoted by pr. When all the interfering terminals are in a Region R, it follows that Rprrdr. Observe that only the interfering terminal that transmits at a particular time instant is of interest and the location of this terminal is represented by r; the interfering terminals in the network that do not transmit at this time instant are not of interest from an interference standpoint.

For small aperture terminals, because of off-axis emission limits, the e.i.r.p. spectral density (ESD) in the boresight direction depends on the aperture size of the terminal’s antenna. Consider a network of terminals consisting of nE antenna aperture sizes that are distributed randomly. Then the distribution of the boresight ESD can be represented by a discrete probability distribution with values from the set E1,E2,…EnE. The conditional PDF of the ESD, conditioned on the specific location r of the terminal, is given by the discrete probabilities p1,Er,p2,Er,…pnE,Er, where pn,Eris the probability that the terminal at r transmits with boresight ESD level En and n=1nEpn,Er=1. Observe that with this formulation there could be multiple terminals at r with different boresight ESD levels; however, only a single terminal can transmit at a given time instant.

## 4.2	Interference signal using actual locations

The signal paths from the interfering terminals to the victim receiver via the interfering and victim satellites are shown Fig. 1. The interference power spectral densities (PSDs) at the victim receiver, via Satellites Sv and Si, due to the interfering terminal Ti at r are denoted by Ivr and Iir, respectively. These can be expressed in terms of the satellite link parameters and γv and γi. Note that the downlink path transmission gains from the satellites are not functions of the specific locations of the interfering terminals. Therefore, the values of γv and γi do not change when the interference is evaluated by assuming that the locations of the transmit terminals are approximated by a set of quantized values of the location variable.

Using the satellite link parameters and notation listed in § 3, the interference power spectral densities received via Satellites Sv and Si are expressed as

Ivr=Erψr,vGvsβv,,rLu,rγvIir=Er0Gisβi,rLu,rγi. 	(1)

In the above it is assumed that the orbital separation between the satellites is close so that the uplink path losses to the satellites are the same. Note that the total interference PSD at the victim receiver is Ivr+Iir and for high-gain receive antennas at the victim receiver Ivr≫Iir because γv≫γi.  The analytical expressions for the CDFs of Ivr and Iir can be determined using the PDFs of the random variables r and Er. In the Annex, a step-by-step Monte-Carlo simulation process is given to calculate the CDFs of these interference PSD terms.

## 4.3	Interference estimated using quantized spatial locations

The locations of the interfering terminals are approximated by a set of grid points obtained by quantizing the location variable r. These quantized points are denoted by rk, k=1,2,…K, and Rk is the kthquantization region, that is, all spatial locations within Rk are represented by the grid point rk. An illustrative quantization scheme for the locations of the interfering terminals is shown in Fig. 2.

Figure 2

Quantization of Region R to Regions Rk, k = 1, 2, . . . , K. Representative centre of Rk is shown as rk. 
The points “o” denote the specific locations of the interfering terminals. En, n=1,2,…nE, 
denote the boresight e.i.r.p. density levels at these terminals

This Figure shows K quantized regions, R1, R2, . . . RK. These quantized regions are disjoint and encompass the entire region of interest R. Note that although this Figure shows a regular pattern for the quantized regions, in general, they may take the shape of some conveniently chosen predetermined pattern. In this figure the actual locations of the interfering terminals are shown as “o” and these terminal locations are quantized to one of the points rk, k=1,2,…K, such that the quantized point is the nearest to the location of the terminal. Specifically, denote the distance metric between the interfering terminal located at r and a quantized point rk as dr,rk. Then this terminal is located in Region Rk  and represented by the grid point rk when the following is satisfied

r∈Rk      if dr,rk≤dr,rk',  rk≠rk'.	(2)

In the simulations presented later the Euclidean distance metric is considered for the function dx1,x2, which is the distance between the two points x1 and x2.

The interference is now computed assuming that the interfering terminals are located at the representative points rk rather than at their actual locations r. Specifically, the interfering terminal at r with boresight ESD Er is relocated to rk. So the resulting interference PSDs corresponding to Ivr and Iir in equation (1) are expressed as

Iv,qrk=Erψrk,vGvsβv,rkLu,rkγvIi,qrk=Er0Gisβi,rkLu,rkγi.	(3)

The link parameters on the right-hand side of the above equation are with respect to the quantized locations rk. The variables,  ψrk,v , Gvsβv,rk, Gisβi,rk  and  Lu,rk can be determined by using their respective values at the grid point, rk. The boresight ESD Er is a random variable and it is the same value as that of the corresponding terminal located at the actual location r.

### 4.3.1	Statistics of rk and Er in quantized regions

Estimating the PDFs of the random variables rk and Er is described next. Denote by 
prk=Probr∈Rk, which is the probability that the transmit terminal is in the Region Rk. This probability can be expressed as:

prk=r∈Rkprrdr 	(4)

Note that k=1Kprk=1. When the PDF pr is known these probabilities can be determined for a given set of quantization regions. On the other hand, these probabilities can also be estimated using the method of relative frequency of occurrence of the terminals in the respective quantization regions. For example, suppose tk is the total time interval the terminals are transmitting from the Region Rk, then

prk=tki=1Kti 	(5)

Next, Er takes values from the set E1,E2,…EnE and the desired probability is 
pn,k,Er=ProbEr=En|r∈Rk, that is the probability that the boresight ESD of the transmit terminal is En when it is transmitting from Region Rk. This probability can be determined analytically when the PDFs pr and pn,Er are known. Alternatively, the following method could be used to estimate this PDF. This method requires tabulating the time duration of occurrence of each ESD level at each Region Rk in the desired observation time interval. Denote by tn,k the time duration for which the ESD En is used in the Region Rk. Then the desired PDF is estimated as

pn,k,Er=tn,ki=1nEti,k 	(6)

The CDFs of the interference terms Iv,qrk and Ii,qrk given in equation (3) can be determined using the above PDFs. A Monte-Carlo simulation method to estimate these CDFs is given in the Annex.

## 4.4	The difference between the interference PSDs

The interference PSDs using the actual locations of the terminals are given by Ivr and Iir in equation (1). The corresponding interference PSDs when the locations are approximated by the grid points rk are given by Iv,qrk and Ii,qrk in equation (3). The difference in the interference PSD is because the actual locations of the terminals are approximated by the grid points. In order to determine this difference, first consider a terminal with a boresight ESD level En at r. The corresponding interference PSDs can be obtained from equation (1) by setting Er=En. Observe that this event occurs with probability pn,Er. The location of this terminal, r, is now quantized to the grid point rk and the corresponding interference PSDs are obtained from equation (3) by replacing Erwith En. The difference between the interference PSDs is the “error” in the interference PSD and occurs with probability pn,Er.

This difference term can be expressed as:

Ie,n,kr=Enψr,,vGvsβv,rLu,r-Enψrk,vGvsβv,rkLu,rkγv+En0Gisβi,rLu,r-En0Gisβi,rkLu,rkγi.	(7)

The CDF of Ie,n,kr can be computed using the PDFs of the underlying random variables r and Er. In the simulations given in § 5, a Monte-Carlo method is used to evaluate the CDF of this difference between the interference PSDs.

Note the following two cases:

Ie,n,kr<0, the interference PSD estimated using the approximated locations is more than the actual interference PSD and it overestimates the actual interference PSD;

Ie,n,kr>0, the interference PSD estimated using the approximated locations is less than the actual interference PSD and it underestimates the actual interference PSD.

Observe that by reducing the boresight ESD of the terminals, that is En in the first and third terms on the right-hand side of equation (7), this underestimation error can be reduced. Section 5 presents simulation results to study this effect.

## 4.5	Interference PSD in the presence of antenna pointing errors

Methodologies to account for the effects of antenna pointing errors in the interference PSD are addressed in Recommendations ITU-R S.1857 and ITU-R S.2029. The antenna pointing error, ϕ, is defined as the angle between the boresight direction of the interfering terminal’s antenna and the desired direction to the satellite. The antenna pointing error is usually measured in terms of its components: the errors in the elevation and azimuth directions, which are denoted by ϕε and ϕa, respectively. Recommendation ITU-R S.1857 describes the method to compute ϕ using the observations ϕε and ϕa. Modifying (1) to account for the antenna pointing errors, the interference PSDs due to a terminal at r with an antenna pointing error of ϕ is

Ivr,ϕ=Erψr,vϕGvsβv,,rLu,rγvIir,ϕ=ErϕGisβi,rLu,rγi  	(8)

where the off-axis angle is shown as a function of ϕ. When r is quantized to the grid point rk the terminal with boresight ESD Er is relocated to rk. The antenna pointing error at this terminal is the same at both r and rk. Modifying equation (3) to account for the antenna pointing errors, the interference PSDs estimated using the grid points is

Iv,qrk,ϕ=Erψrk,vϕGvsβv,rkLu,rkγvIi,qrk,ϕ=ErϕGisβi,rkLu,rkγi.	(9)

Finally, the difference between the interference PSDs can be computed using equations (8) and (9). In the results presented in the next section computer simulation examples are given for these interference PSDs and the difference between the interference PSDs.

# 5	Simulation results

This section presents some illustrative simulation results for satellite networks operating at the 30 GHz and 14 GHz frequency ranges.

## 5.1	Results at 30 GHz

In these simulations the terminals are distributed as shown in Fig. 3; the corresponding link parameters are given in Table 1. The beam pattern at the victim satellite corresponds to a narrow spot beam. This figure show the 6 dB contour of the receive antenna beam. Distribution-A, shown in Fig. 3, is a two-dimensional distribution centered at Washington, D.C.

Figure 3

Distribution-A of Terminals Ti and the 6-dB contour of receive antenna beam of Satellite Sv.
The beam center of Sv is at Harrisburg, Pennsylvania. This shows a set of grid points within Region R

TABLE 1

Satellite link parameters used with terminal Distribution-A in Fig. 3

| Uplink frequency | 29.75 GHz |
| --- | --- |
| Aperture diameters of Ti | (0.2, 0.25, 0.3, 0.35, 0.4) m uniformly distributed |
| Antenna pointing errors at Ti, ϕε and ϕa | independent and identically distributed normal random variables with mean zero and variance 0.2 degrees |
| Locations of Ti, Distribution-A | uniformly distributed within a circular area centered at Washington, D.C. with a radius of 70 km |
| Longitudes at Satellites Sv, Si | 102.80 W and 100.20 W |
| Locations of Cv, Ci: (latitude, longitude) | Harrisburg, Pennsylvania. (41.270 N, 76.880 W) |
| Receive antennas at satellites | 1.4 m diameter aperture with parabolic illumination and efficiency 0.55 |

The grid points for Distribution-A form a square grid within the Region R with distance between adjacent grid points, rΔ=rk-rk-1, set to a constant value. In the simulations the effect of different values of rΔ on the interference is examined.

The boresight ESD of the terminals are determined so that they transmit at the maximum level subject to the ESD level KaMask-Ered dB(W/Hz), where KaMask is the ESD level established in recommends 4 of Recommendation ITU-R S.524-9 and Ered is a small value to reduce the ESD levels to accommodate the underestimation of the interference PSD when quantized locations are used. Note that Ered is used only with the first and third terms on the right-hand side of equation (7).

Also, note that the ESD level KaMask-Ered is satisfied by the terminals in the absence of antenna pointing errors; the boresight ESD was not reduced to account for the antenna pointing errors. These simulations considered a large aperture receive antenna at the victim terminal. Therefore, the terms Iir and Ii,qrk in equations (1) and (3) are ignored.

In order to normalize the interference PSDs consider a hypothetical terminal located at the beam center of Satellite Sv and assume that this terminal is transmitting at its maximum off-axis emission level given in KaMask, which is 19-25log10ψcenter,v dB(W/40 kHz) where 
ψcenter,v 20≤ψcenter,v<70 is the off-axis angle toward Sv in degrees. Denote by Icenter the interference PSD from this terminal. Note that Icenter is the maximum value of the interference PSD for any terminal location and any antenna aperture size.

Figure 4 shows the complementary CDFs of the normalized interference values Ivr/Icenter (without quantization) and Iv,qrk/Icenter for the terminal Distribution-A, shown in Table 1.

For Iv,qrk/Icenter the results are for rΔ=20 and 50 km. It can be seen that when the boresight ESD of the terminals are not reduced, that is when Ered= 0 dB, there is very little difference in the interference PSDs. On the other hand, when Ered= 0.2 dB the interference PSD computed using the actual locations decreases. Because the interference PSD with quantized locations is more than the actual interference this results in overestimation of the actual interference PSD.

Figure 4

Complementary CDFs of the normalized interference PSDs for the Distribution-A shown in Table 1
Ered=0 dB (top figure) and Ered=0.2 dB (bottom figure). Legend shows values of rΔ

The CDF of the normalized difference between the interference PSDs, Ie,n,kr/Icenter, for terminal Distribution-A is shown in Fig. 5. These results are for different values of rΔ and Ered. Unlike the results in Fig. 4, these results show that the error is noticeable even when Ered=0 dB. As expected the error in the interference PSD increases for larger values of rΔ. For example, probability that Ie,n,krIcenter<0.02 is 0.9 when Ered=0 dB and for rΔ=20 km.

For the same probability, this error reduces to  Ie,n,krIcenter<-0.0004 when Ered=0.2 dB. Note that Ie,n,kr>0 corresponds to underestimation of the actual interference PSD and these results show that the underestimation of the actual interference PSD can be substantially reduced by slightly reducing the boresight ESD of the terminals. Fig. 6 shows ProbIe,n,krIcenter<0, which is the probability of overestimating the actual interference PSD, as a function of Ered. As expected it can be seen that this probability increases with increasing values of Ered.

Figure 5

CDF of the difference between the interference PSDs for Distribution-A shown in Table 1
Legend shows rΔ and Ered

Figure 6

ProbIe,n,krIcenter<0 as a function of Ered for Distribution-A shown in Table 1. Legend shows rΔ

## 5.2	Results at 14 GHz

The results given in the preceding subsection are for a narrow spot beam antenna in the 30 GHz frequency range. In this subsection a wide beam receive antenna in a Ku-band satellite is considered. The 6 dB contour of the receive antenna beam of the victim satellite and the distribution of the terminals is shown in Fig. 7. Here the terminals are uniformly distributed (Distribution-B) in a region centered at Atlanta, Georgia and with radius 300 km. Table 2 shows the link parameters used in these simulations. The ESD levels of the terminals in this case comply with 
KuMask-Ered dB(W/Hz), where KuMask is the ESD level established in Recommendation ITU-R S. 728-1. Also, Icenter corresponds to the ESD level KuMask and the beam center of this antenna pattern is shown in Fig. 7. Note that, as in the results for the 30 GHz range, Ered is used only with the first and third terms on the right-hand side of equation (7).

Figure 7

Distribution-B of Terminals  Ti and the 6-dB contour of receive antenna beam of Satellite Sv. 
The beam center of Sv is at Lexington, Kentucky

TABLE 2

Satellite link parameters used with the terminal distribution in Fig. 7

| Uplink frequency | 14.25 GHz |
| --- | --- |
| Aperture diameters of Ti | (0.3, 0.35, 0.4, 0.45, 0.5) m, uniformly distributed |
| Antenna pointing errors at Ti, ϕε and ϕa | independent and identically distributed normal random variables with mean zero and variance 0.20 |
| Locations of Ti, Distribution-B | uniformly distributed within a circular area centered at Atlanta, Georgia with a radius of 300 km |
| Longitudes at Satellites Sv, Si | 102.80 W and 100.20 W |
| Locations of Cv, Ci: (latitude, longitude) | Lexington, Kentucky. (38.030 N, 84.50 W) |
| Receive antennas at satellites | 0.7 m diameter aperture with parabolic illumination and efficiency 0.55 |

Figure 8 shows the complementary CDFs of Ivr/Icenter (without quantization) and Iv,qrk/Icenter for the terminal Distribution-B shown in Fig. 7 and the link parameters in Table 2. As seen from this figure when Ered=0.2 dB there is a noticeable difference between the actual interference and the interference estimated using the approximated locations. Observe that r∆ for these results are 100 km and 150 km. This is because, for a wide beam antenna at the victim satellite, the change in the satellite antenna gain pattern for these distances is very small.

Figure 8

Complementary CDFs of the normalized interference PSDs for the Distribution-B shown in Table 2
Ered=0 dB (top figure) and Ered=0.2 dB (bottom figure). Legend shows values of rΔ

Figures 9 and 10 show the variations of the normalized error in the interference PSD for different values of Ered and r∆. As in the case for the narrow spot beam antenna, it can be seen that the probability of underestimating the actual interference can be reduced by slightly reducing the boresight ESD of the terminals.

Figure 9

CDF of the difference between the interference PSDs for Distribution-B shown in Table 2
Legend shows rΔ and Ered

Figure 10

ProbIe,n,krIcenter<0  as a function of Ered for Distribution-B shown in Table 2. Legend shows rΔ

# 6	Discussion

The methodology presented in this Report allows the evaluation of interference from a network of earth stations operating with GSO FSS networks by using approximate locations, i.e. selected grid points within a grid. The results in Figs 5 and 9 show that the distance between grid points determines the accuracy of the estimation of interference using the approximate locations. It follows from these results that there is an imperceptible difference when the distances are small (no quantization, 100 m or 1 km) and the difference is noticeable only when the separation between grid points becomes larger, for example, 20 km at 30 GHz or 50 km at 14 GHz.

The results also show that by slightly reducing the e.i.r.p. levels of the transmit terminals the underestimation of the actual interference can be significantly reduced. Hence, the e.i.r.p. levels of the terminals can be reduced to guarantee that the actual interference is less than the estimated interference for a given probability level.

Therefore, in situations where it is not possible to provide the actual location of terminals with a high resolution, an alternative approach would be to provide the grid characteristics of the quantized region (e.g. size, grid-point separation, location within the satellite beam, etc.) and the associated distribution of the earth stations at these grids points and their e.i.r.p. distribution.

A step-by-step process included in the Annex shows how to use this information to obtain the interference values with a certain probability.

# 7	Conclusion

This Report presents a statistical methodology to estimate interference levels from an earth station network onto GSO FSS networks, based on approximate location information (latitude, longitude) of the terminals. The sample simulation results show the sensitivity of terminal location information to the resulting interference. Furthermore, the results show that approximated location values, within a certain range, i.e. not the actual locations, can provide reasonably good estimates of the actual interference.

Annex

A step-by-step method to estimate the interference PSDs

This Annex gives a step-by-step Monte-Carlo simulation method to estimate the interference PSDs using the actual locations and their approximated locations. Also, it shows how to estimate the difference in these interference PSDs.

It is assumed that γv ≫γi  so only the interference arriving via Satellite Sv is considered. Also, antenna pointing errors are not considered here.

# 1	Estimating the interference PSD using the actual locations

Input to the estimation process

Nr, number of actual locations of the terminals in the Region R.

Longitudes at Satellites Sv and Si; location of the beam center of Sv, rcenter; uplink frequency; and receive antenna gain pattern of Sv, Gvs.

pr, PDF of the locations of the terminals; and pn,Er, probability that the terminal at r is transmitting at boresight e.i.r.p. spectral density level En.

Estimation process

Step 1: Using the PDF pr generate the 1×Nr location vector ρ=ρ1,ρ2,…ρNr.  This vector represents the actual locations, r, of the interfering terminals.

Step 2: Use the locations of the Satellites Sv and Si and the location vector ρ, to compute the off-axis angle 1×Nr vector to Sv, ψ=ψρ1,v,ψρ2,v,…ψρNr,v.

Step 3: Using ρ and longitude and the beam center of Sv compute the angles βv,ρ1, βv,ρ2,… βv,ρNr. Then the 1×Nr antenna gain pattern of Sv is G= Gvsβv,ρ1, Gvsβv,ρ1,… Gvsβv,ρNr.

Step 4: The path losses from the terminal locations ρ to the Satellite Sv can be computed using longitude at Sv and the uplink frequency. This 1×Nr path loss vector is expressed as

L=Lu,ρ1,Lu,ρ2,… Lu,ρNr.

Step 5: The probability of occurrence of the boresight ESD level En at a given location ρi is given by pn,Eρi. The off-axis ESD vector at all the locations is

Enψ=Enψρ1,vpn,Eρ1,Enψρ2,vpn,Eρ2,…EnψρNr,vpn,EρNr .

Step 6: The interference PSD due to a terminal at r and ESD level En can be computed using (1). The 1×Nr interference PSD vector due to terminals at ρ with ESD level En is

In=Enψ×GLγv,

where the multiplication and division of the vectors is defined such that the ith-element of In,
Ini=Enψi×GiLiγv.

Step 7: Consider the interference PSD for all nE ESD levels. The 1×Nr×nE interference PSD vector is I=I1,I2,…InE.

Step 8: Normalize I with respect to Icenter. Using rcenter determine the angle ψcenter,v. The off-axis ESD level from this hypothetical terminal is EMaskψcenter,v where EMaskψ is the off-axis ESD levels specified in Recommendations ITU-R S.524-9 or ITU-R S.728-1.

Then Icenter=EMaskψcenter,vGvs0Lu,rcenterγv. The normalized interference PSD is IIcenter, which does not contain the term γv.

Step 9. Estimate the CDF of IIcenter.

# 2	Estimating the interference PSD using the grid points

Additional input to the estimation process

1×K vector of grid points rk=r1,r2,…rK.

Estimation process

Step 1: Using rk determine the 1×K vectors Gk= Gvsβv,r1,  Gvsβv,r2,…Gvsβv,rk, 
Lk=Lu,r1,Lu,r2,,… Lu,rk , and ψk=ψr1,v,ψr2,v,…ψrk,v.

Step 2: Quantize each element of ρ to the nearest point in rk.  Construct this 1×Nr quantized vector of ρ, ρq=ρq,1,ρq,2,…ρq,Nr, where ρq,i= rk when ρi∈ Rk .  Note that the elements of ρq are  rk, k=1,2,…K. Denote the mapping of the elements from rk to ρq by Q. That is ρq=Qrk.

Step 3: Apply the mapping Q to the 1×K vectors in Step 1 to rearrange the terms to 1×Nr vectors and denote them by Gq=QGk, Lq=QLk and ψq=Qψk.

Step 4: The 1×Nr off-axis ESD vector is obtained by replacing the off-axis angle, with that corresponding to the quantized locations, in the off-axis ESD level given in Step 5 of the preceding section. The resulting off-axis ESD vector is

Eqnψ=Enψq,1pn,Eρ1,Enψq,2pn,Eρ2,…Enψq,Nrpn,EρNr ,

where ψq,i is the ith- element of ψq.

Step 5: The 1×Nr interference PSD vector due to quantized locations and boresight ESD level En is

Iqn=Eqnψ×GqLqγv.

Step 6: Form the 1×nE×Nr interference PSD vector due to all the En levels Iq=Iq1,Iq2,…IqnE.

Step 7: Estimate the CDF of the normalized vector, IqIcenter.

Step 8: The normalized error in the interference PSDs is I-IqIcenter. Estimate its CDF.

_________________