Chinese Evaluation Group 

Calibration Activities in Chinese Evaluation Group

# 1	Introduction

According to the ITU-R Working Party 5D work plan, the IMT-Advanced candidate RIT/SRITs may be received from different SDO or countries before the deadline at the October 2009. The evaluation reports for these received candidate RIT/SRITs will be submitted before the cut off time in June of 2010.

In preparation for the upcoming evaluation activities of the candidate technologies, Chinese Evaluation Group (ChEG) started the activities of simulation calibration among its members. The purpose of this document is to provide information about ChEG and its relevant activities. The contents of this document are informative.

# 2	Introduction to Chinese Evaluation Group

So far, plenty of Evaluation Groups have registered to ITU-R Working Party 5D. In order to facilitate the evaluation work of ITU-R Working Party 5D efficiently, ChEG, one of the Evaluation Groups, has already started the calibration activities to prepare for the coming IMT-Advanced evaluation activities that require simulation effort.

There are 13 members participating in the ChEG activities:

–	China Academy of Telecommunication Research of MIIT,

–	Southeast University,

–	China Mobile Communications Corporation,

–	China United Network Communications Corporation,

–	China Telecom Corporation Limited,

–	Tsinghua University,

–	Beijing University of Posts and Telecommunications,

–	The State Radio Monitoring Center,

–	54th Research Institute, Electronic Technology Corporation, China,

–	Datang Mobile Communication Equipment Co., Ltd.,

–	Huawei Technologies Co., Ltd.,

–	ZTE Corporation,

–	Alcatel Shanghai Bell Co., Ltd.

# 3	Calibration activities in ChEG

For calibration among its members, a step-by-step approach has been taken by ChEG. Following steps have been defined and preliminary results are included in next section for reference.

## 3.1	Channel model calibration

The intention of this step is to calibrate the implementation of newly defined IMT-Advanced MIMO channel. The output is in form of statistical characteristics of MIMO fast fading, which are expected to match with the theoretical values that can be derived from the definition of the MIMO channel in Report ITU-R M.2135.

## 3.2	Link level calibration

1)	Link performance in AWGN channel

This step is to calibrate the implementation of basic transceiver algorithms among ChEG members. The output is in form of link level curve.

2)	Link performance in MIMO fading channel (with ideal channel estimation)

This step is to calibrate the implementation of MIMO transceiver algorithms among ChEG members. The output is in form of link level curve.

3) 	Link performance in MIMO fading channel (with real channel estimation)

This step is to calibrate the implementation of channel estimation algorithms among ChEG members. The output is in form of link level curve.

4)	Link performance in MIMO fading channel with adaptive scheduling and HARQ

This step is to calibrate the implementation of full link level processes, including channel measurement / feedback, Adaptive Scheduling, AMC, HARQ, etc. The output is in form of link level spectrum efficiency.

## 3.3	System level calibration

Here ChEG referred the methodologies taken by 3GPP in LTE-advanced self evaluation. Those include:

1)	Large scale fading characteristics of system level platform.

This step is to calibrate the basic characteristics of system level platform among ChEG members. Those characteristics include distribution of downlink wideband SINR (also known as geometry) as well as coupling loss.

2)	System performance in simple scenario

This step is to calibrate the system level performance with simple system level simulation assumption, e.g. SIMO, Round Robin scheduling, etc. Output is in form of the distribution of user SINR and throughput.

# 4	Some result of calibration in ChEG

In this section, we provide some calibration results which are the latest outcomes of calibration activities of ChEG.

## 4.1	Channel model calibration

Compared with the theoretical values, calibration results about statistical characteristic of IMT-Advanced MIMO fast fading channel of different scenarios (indoor, UMa, UMi and RMa) are shown in the following embedded documents.

## 4.2	Link level calibration

1)	Link performance in AWGN channel

•	Simulation assumptions

–	Antenna configuration: 1x1;

–	Turbo decoding algorithm: Max-Log-MAP with 8 iterations;

–	Others:

|  | Short code length | Middle code length | Long code length |
| --- | --- | --- | --- |
| CQI | 5 | 9 | 15 |
| Modulation | 2 | 4 | 6 |
| TBS | 392 | 1128 | 3112 |
| rate coding | 0.4385 | 0.601 | 0.9258 |
| SNR | [−1 : 0.5 : 2] | [7.8 : 0.2 : 9] | [19.4 : 0.2 : 20.6] |

•	Result calibration

2)	Link performance in MIMO fading channel (with ideal channel estimation)

•	Simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | UMA NLoS, 30 km/h (CDL) |
| Transmission scheme | 4*2 Precoding vector: [1,0;1,0;0,1;0,1] |
| Channel estimation | Ideal |
| RB/block size | 4RB Pair (1 ms)/(1 128 + 24)*2 codeword)/16QAM |
| simulation time | 10000 packages |
| Antenna configuration in base station | 10 lambda spacing |
| Antenna configuration in UE | 0.5 lambda spacing |
| MIMO receiver | MMSE |

•	Result calibration

3)	Link performance in MIMO fading channel (with real channel estimation)

•	Simulation assumption

| Channel estimation algorithm | 2D Viena filter (CATR, Huawei) LMMSE with linear interpolation (Datang Mobile) |
| --- | --- |

•	Result calibration

4)	Link performance in MIMO fading channel with adaptive scheduling and HARQ

•	Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenarios | UMA NLoS, 30 km/h |
| Channel model | CDL |
| Antenna configuration | 1*4, 10 lambda spacing |
| Channel estimation | Real |
| RB size | 4RB Pair (1 ms) |
| HARQ Timing | Refer to LTE R8 uplink |
| SNR | 4.8 dB |
| Output | Spectrum Efficiency |
| AMC | On, refer to LTE R8 uplink |

•	Result calibration

Output Spectrum Efficiency from members: 1.50~1.56 bps/Hz

## 4.3	System level calibration

1)	Large-scale fading catachrestic of system level platform

ChEG plans to perform system level calibration from June to August 2009, and the results will be provided later to the 6th ITU-R WP 5D meeting in October 2009.

2)	System performance in simple scenario

ChEG plans to perform system level calibration from June to August 2009, and the results will be provided later to the 6th ITU-R WP 5D meeting in October 2009.

Contact information:	please refer to the ITU-R IMT-Advanced submission and evaluation process web page for the contact information of the Chinese Evaluation Group