Annex to Recommendation ITU-R TF.768 
“Standard frequencies and time signals”

Characteristics of standard-frequency and time-signal emissions in allocated bands and characteristics of stations emitting with regular schedules with
stabilized frequencies, outside of allocated bands

This document provides information to supplement Recommendation ITU-R TF.768 “Standard frequencies and time signals”.

The characteristics of standard-frequency and time signal emissions are listed in two tables. Table 1 contains information about emissions in the allocated bands, and table 2 contains information about emissions in additional bands. The characteristics of some navigational aids are given in table 3.

For information concerning changes which may have occurred since the date when this document was last updated, reference may be made to the BIPM Annual Report on time activities or directly to the respective authority for each service as listed at the end of the Annex.

Document last updated: 2008 October 10.

TABLE 1

Characteristics of standard-frequency and time-signal emissions in the allocated bands

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (MHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| BPM(2) | Pucheng, China | 35 00 N 109 31 E | Omni-directional | 10-20 | 2 | 7 | 24(3) | 2.5, 5, 10, 15 | 1, 1 000 | 20/30 (UTC) 4/30 (UT1) | Nil |  10 | Direct emission of UT1 time signal |
| HLA | Taejon, Taedok Science Town, Republic of Korea | 36 23 N 127 22 E | Vertical (conical monopole) | 2 | 1 | 5 (4) | 7 (5) | 5 | 1 | Continuous | Continuous |  10 | ITU-R code by double pulse |
| LOL(6) | Buenos Aires, Argentina | 34 37 N 58 21 E | Horizontal 3-wire folded dipole | 2 | 3 | 7 | 5 | 5, 10, 15 | 1, 440, 1 000 | Continuous | 3/5 |  20 | ITU-R code by lengthening |

TABLE 1 (end)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (MHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| RWM(6) | Moscow | 55 44 N 38 12 E | Horizontal dipole | 5 5 8 | 3 | 7 | 24 | 4.996, 9.996, 14.996 | 1/60, 1, 10 | 40/60 | Nil |  10 | ITU-R code by double pulse, additional information dUT1 (7) |
| WWV(6) | Fort Collins, Colorado, United States | 40 41 N 105 02 W | Vertical /2 dipoles | 2.5-10 | 5 | 7 | 24 | 2.5, 5, 10, 15, 20 | 1, 440, 500, 600 | Continuous (8) | Continuous (9) |  10 | ITU-R code by double pulse, additional information on UT1 corrections |
| WWVH(6) | Kekaha, Kauai, Hawaii, United States | 21 59 N 159 46 W | Vertical /2 dipole arrays | 2.5-10 | 4 | 7 | 24 | 2.5, 5, 10, 15 | 1, 440, 500, 600 | Continuous (8) | Continuous (9) |  10 | ITU-R code by double pulse, additional information on UT1 corrections |
| YVTO | Caracas, Venezuela | 10 30 N 66 55 W |  |  | 1 | 7 | 24 | 5 |  |  |  |  |  |

Notes to Table 1:

(1)	This value applies at the transmitter. To realize the quoted uncertainty at the point of reception it will be necessary to observe the received phase time or frequency over a sufficiently long period in order to eliminate noise and random effects.

(2)	Call sign in Morse and language.

(3)	2.5 MHz: 07:30-01:00 UTC; 15 MHz: 01:00-09:00 UTC; 5 MHz and 10 MHz: continuous.

(4)	Monday to Friday (except national holidays in Korea).

(5)	01:00 to 08:00 UTC. Pulses of 9 cycles of 1800 Hz modulation. 59th and 29th second pulses omitted. Hour identified by 0.8 s long 1500 Hz tone. Beginning of each minute identified by a 0.8 s long 1800 Hz tone. Voice announcement of hours and minutes each minute following 52nd second pulse. BCD time code given on 100 Hz sub-carrier.

(6)	These stations have indicated that they follow the UTC system as specified in Recommendation ITU-R TF.460.

(7)	The additional information about the value of the difference UT1 – UTC is transmitted by code dUT1. It provides more precisely the difference UT1 – UTC in multiples of 0.02 s. The total value of the correction is DUT1  dUT1. Possible values of dUT1 are transmitted by marking of p second pulses between the 21st and 24th seconds of the minute, so that dUT1   0.02 s  p. Negative values of dUT1 are transmitted by marking of q second pulses between the 31st and 34th second of the minute, so that dUT1  –0.02 s  q.

(8)	In addition to other timing signals and time announcements, a modified IRIG-H time code is produced at a 1-pps rate and radiated continuously on a 100 Hz sub-carrier on all frequencies. A complete code frame is 1 min. The 100 Hz sub-carrier is synchronous with the code pulses, so that 10 ms resolution is obtained. The code contains DUT1 values; UTC time expressed in year, day of year, hour and minute; and status indicators relating to impending leap seconds and Daylight Saving Time.

(9)	Except for voice announcement periods and the 5 min semi-silent period each hour.

TABLE 2

Characteristics of standard-frequency and time-signal emissions in additional bands

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| CHU(2) | Ottawa, Canada | 45 18 N 75 45 W | Omni- directional | 3, 10, 3 | 3 | 7 | 24 | 3 330, 7 335, 14 670 | 1 (3) | Continuous | Nil |  5 | ITU-R code by split pulses |
| DCF77(2) | Mainflingen, F.R. of Germany | 50 01 N 09 00 E | Omni- directional | 30 (4) | 1 | 7 | 24 | 77.5 | 1 | Continuous (5) | Nil (6) |  2 (7) | No DUT1 transmission |
|  | Droitwich, United Kingdom | 52 16 N 02 09 W | T | 400 | 1 | 7 | 22 | 198 (8) | Nil | Nil | A3E broadcast continuously |  20 |  |
|  | Westerglen, United Kingdom | 55 58 N 03 50 W | T | 50 | 1 | 7 | 22 | 198 (8) | Nil | Nil | A3E broadcast continuously |  20 |  |
|  | Burghead, United Kingdom | 57 42 N 03 28 W | T | 50 | 1 | 7 | 22 | 198 (8) | Nil | Nil | A3E broadcast continuously |  20 |  |
| EBC | San Fernando, Cadiz, Spain | 36 28 N 06 12 W | Omni-directional | 1 | 1 | 5 | 1 | 15006 4998 | (9) | 25 | J3E |  100 | ITU-R code by double pulse |
| HBG(2) | Prangins, Switzerland | 46 24 N 06 15 E | Omni- directional | 20 | 1 | 7 | 24 | 75 | 1 (10) | Continuous | Nil |  1 | No DUT1 transmission |

| TABLE 2 (continued) Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| JJY | Fukushima, Japan | 37 22 N 140 51 E | Omni-directional | 10 | 1 | 7 | 24 | 40 | 1 | Continuous | Nil |   | No DUTI transmission |
| JJY | Saga, Japan | 33°, 28' N 130° 11' E | Omni-directional | 10 | 1 | 7 | 24 | 60 | 1 | Continuous | Nil |  1 | No DUTI transmission |
| MSF | Anthorn, United Kingdom | 54 55 N 03 15 W | Omni- directional | 15 (5) | 1 | 7 | 24 (11) | 60 | 2 (12) | Continuous | Nil |  2 | ITU-R code by double pulse |
|  | Milano, Italy | 45 20 N 09 12 E | Omni- directional | 600 | 1 | 7 | 24 | 900 | Nil | Nil | A3E broadcast continuously |  2 |  |
| NAA (3) (13) (14) | Cutler, Maine, United States | 44 39 N 67 17 W | Omni- directional | 1 000 (5) | 1 | 7 | 24 (15) | 24 (16) | Nil | Nil | Nil |  10 |  |
| NAU (3) (13) (14) | Aguada, Puerto Rico | 18 23 N 67 11 W | Omni- directional | 100 (17) | 1 | 7 | 24 | 28.5 | Nil | Nil | Nil |  10 |  |
| NTD (3) (13) (14) | Yosami, Japan | 34 58 N 137 01 E | Omni- directional | 50 (5) | 1 | 7 | 24 (18) | 17.4 | Nil | Nil | Nil |  10 |  |
| NLK (3) (13) (14) | Jim Creek, Washington, United States | 48 12 N 121 55 W | Omni- directional | 125 (5) | 1 | 7 | 24 (19) | 24.8 | Nil | Nil | Nil |  10 |  |
| NPM (3) (13) (14) | Lualualei, Hawaii, United States | 21 25 N 158 09 W | Omni- directional | 600 (5) | 1 | 7 | 24 (20) | 23.4 | Nil | Nil | Nil |  10 |  |

TABLE 2 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| NWC (3) (13) (14) | Exmouth, Australia | 21 49 S 114 10 E | Omni- directional | 1 000 (5) | 1 | 7 | 24 (22) | 22.3 | Nil | Nil | Nil |  10 |  |
| RAB-99 | Khabarovsk | 48° 30 N 134° 50 E | Omni- directional | 300 | 1 | 7 (23) | 1.1 | 25.0, 25.1, 25.5, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 11 min 2 times per day (25) | Nil |  5 |  |
| RBU(2) | Moskva | 55° 44 N 38° 12 E | Omni- directional | 10 | 1 | 7 | 24 | 200/3 | 10, 100, 312.5 | Continuous DXXXW (26) | Nil |  2 | ITU-R code by double pulse (27) |
| RJH-63 | Krasnodar | 44° 46 N 39° 34 E | Omni- directional | 300 | 1 | 7 (28) | 0.6 | 25.5, 25.1, 25.0, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 11 min (29) | Nil |  5 |  |
| RJH-69 | Molodechno | 54° 28 N 26° 47 E | Omni- directional | 300 | 1 | 7 (30) | 0.7 | 25.5, 25.1, 25.0, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 15 min (31) | Nil |  5 |  |
| RJH-77 | Arkhangelsk | 64° 22 N 41° 35 E | Omni- directional | 300 | 1 | 7 (32) | 0.7 | 25.5, 25.1, 25.0, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 15 min (33) | Nil |  5 |  |

TABLE 2 (end)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simulta- neous trans-missions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) (1) | Method of DUT1 indication |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Modu-lation (Hz) | Time signal (min) | Audio-modulation (min) |  |  |
| RJH-86 | Beshkeck | 43° 03 N 73° 37 E | Omni- directional | 300 | 1 | 7 (34) | 1.4 | 25.5, 25.1, 25.0, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 15 min twice per day (35) | Nil |  5 |  |
| RJH-90 | Nizhni Novgorod | 56° 11 N 43° 57 E | Omni- directional | 300 | 1 | 7 (36) | 0.7 | 25.0, 25.1, 25.5, 23.0, 20.5 | 1/60, 1/10, 1, 10, 40 (24) | 15 min (37) | Nil |  5 |  |
| RTZ(2) | Irkutsk | 52° 26 N 103° 41 E | Omni- directional | 10 | 1 | 7 | 23 | 50 | 1/60, 1, 10 | 6/60 | Nil |  5 | ITU-R code by double pulse (27) |
| RW-166 | Irkutsk | 52° 26 N 103° 18 E | Omni- directional | 40 | 1 | 7 | 12 | 198 |  | Nil | A3E broadcast continuously |  5 |  |
| SAJ | Stockholm, Sweden | 59 15 N 18 06 E | Omni- directional | 0.02 (e.r.p.) | 1 | 3 (37) | 2 (38) | 150 000 | Nil | 10 (39) |  |  2 |  |
| STFS | Sikandrabad India | 28 28 N 77 13 E | Satellite |  | 1 | 7 | 24 | 2599675 |  | Continuous |  |  10 |  |
| TDF | Allouis, France | 47 10 N 02 12 E | Omni- directional | 1 000 to 2 000 | 1 | 7 | 24 | 162 | 1 (41) | Continuous | A3E broadcast continuously |  2 | No DUT1 transmission |
| WWVB(2) | Fort Collins, Colorado, United States | 40 40 N 105 03 W | Two top-loaded vertical | 50 | 1 | 7 | 24 | 60 | 1 (42) | Continuous | Nil |  10 | Modified ITU-R code |

Notes to Table 2:

- This value applies at the transmitter. To realize the quoted uncertainty at the point of reception it will be necessary to observe the received phase time or frequency over a sufficiently long period in order to eliminate noise and random effects.

- These stations have indicated that they follow the UTC system as specified in Recommendation ITU-R TF.460.

- Pulses of 300 cycles of 1000 Hz tone: the first pulse in each minute is prolonged.

- Figures give the estimated radiated power.

- The time signals are generated by the Physikalisch-Technische Bundesanstalt (PTB) and are in accordance with the legal time of the Federal Republic of Germany which is UTC (PTB)+ 1 h (Central European Time, CET) or UTC (PTB)+ 2 h (Central European Summer Time, CEST). At the beginning of each second (except in the last second of each minute) the carrier amplitude is reduced to 25% for a duration of 0.1 s or 0.2 s corresponding to “binary 0” or “binary 1”, respectively, referred to as second marks 0 to 59 in the following. The number of the minute, hour, day of the month, day of the week, month and year are transmitted in BCD code using second marks 20 to the 58, including overhead. The current zone time, CET or CEST, is indicated by a “binary 1” in the second marks 18 or 17, respectively. Information emitted during minute n is valid for minute n+1. The information transmitted during the second marks 1 to 14 is provided by third parties. Information on that additional service can be obtained from PTB. To achieve a more accurate time transfer and a better use of the frequency spectrum available an additional pseudo-random phase shift keying of the carrier is transmitted during each second.

- Compliant with the Radio Regulations, the call sign DCF77 is not transmitted as the identification of the service is possible due to its unambiguous signal structure.

- The uncertainty refers to one day averaging and 95% coverage factor (k = 2).

- No coherence between carrier frequency and time signals.

- Seconds pulses of a duration of 0.1 s, modulated at 1000 Hz.
Minutes pulses of a duration of 0.5 s, modulated at 1250 Hz.

- At the beginning of each second (except the 59th second), the carrier is interrupted for a duration of 0.1 or 0.2 s corresponding to “binary 0” or “binary 1”, respectively, double pulse each minute. The number of the minute, hour, day of the month, day of the week, month and year are transmitted in BCD code from the 21st to the 58th second. The time signals are generated by the Swiss Federal Office of Metrology and in accordance with the legal time of Switzerland which is UTC (CH)  1 h (Central European Time CET) or UTC (CH) + 2 h (Central European Summer Time CEST). In addition, CET and CEST are indicated by a “binary 1” at the 18th or 17th second, respectively.

- The transmission is interrupted during occasional maintenance periods, typically from 10:00 to 14:00 local time on the first Tuesday of January, May and September.

- The carrier is interrupted for 500 ms during the first second of each minute, and for 100 ms during other seconds followed by two data bits of 100 ms duration. The BCD time code gives year, month, day-of-month, day-of-week, hour and minute, Summer Time in effect, parity checking and DUT1.

- MSK (minimum shift keying) in use: a phase-stable carrier can be recovered after suitable multiplication and mixing in the receiver. It will be recalled that the use of minimum shift keying means that no discrete component exists at the respective carrier frequencies which are given in the table. The MSK signal can be expressed as:

S(t)  cos [2 fc t  an ( t/2T)  n]

where an  i(– 1) for mark (space) and n  0,  (modulo 2).

If the transmission is to be useful as a frequency reference it is necessary to recover a phase coherent carrier free from the /2 increments introduced by the modulation. There are two approaches.

Notes to Table 2 (continued):

The MSK signal is considered as a continuous-phase frequency shift keying (CPFSK) with a modulation index of 0.5. Squaring the signal followed by band-pass filtering at centre frequency 2fc produces a CPFSK signal with spectral components at 2fc  2fb and 2fc – 2fb, corresponding to mark and space, respectively. The components can be extracted by means of two phase-locked loops (PLL) and the reference carrier recovered by multiplication, division and filtering.

The other approach treats the MSK signal as a form of phase-shift keying (PSK), MSK being obtained by transformations from binary PSK (BPSK) or quadrature PSK (QPSK). The carrier recovery techniques available for PSK such as Costas-loop can thus be applied to MSK; such a demodulator has been realized in a single-chip form.

- This station is primarily for communication purposes; while these data are subject to change, the changes are announced in advance to interested users by the US Naval Observatory, Washington, DC, USA.

- From 12:00 to 20:00 UTC each Sunday while NSS is off the air (until 15 July).

- As of 23 January 1984, until further notice.

- Became operational on 14 August 1984, 74 kW.

- 23:00 to 09:00 UTC just first Thursday-Friday, 23:00 to 07:00 UTC all other Thursday-Fridays. Half power 22:00 to 02:00 UTC each Monday and Friday.

- Except from 16:00 to 24:00 UTC each Thursday. During Daylight Saving Time 15:00 to 23:00 UTC each Thursday.

- 2.5 MHz: 00:00-10:00 UTC; 5 MHz: 09:00-01:00 UTC; 10 MHz: continuous; 15 MHz: 01:00-09:00 UTC.

- Off the air until 21:00 UTC on 15 July, except for 14 hours each Sunday to cover the period when NAA is off the air.

- From 00:00 to 08:00 UTC usually each Monday.

- Daily, except the 10th, 20th and 30th days of each month.

- Two types of signal are transmitted during a duty period: 
	a)	A1A signals with carrier frequency 25 kHz, duration 0.0125; 0.025; 0.1; 1 and 10 s with repetition periods of 0.025; 0.1; 1; 10 and 60 s respectively; 
	b)	N0N signals with carrier frequencies 25.0; 25.1; 25.5; 23.0; 20.5 kHz. The phases of these signals are matched with the time markers of the transmitted scale.

- From 02:06 to 02:40 and 06:06 to 06:40 UTC.
From 01:06 to 01:40 and 05:06 to 05:40 UTC during daylight saving time.

- DXXXW signals are made up of carrier sine-wave oscillations with the frequency of 662/3 kHz, which are interrupted for 5 ms every 100 ms; 10 ms after an interruption the carrier oscillations are narrow-band phase-modulated for 80 ms by sine-wave signals with sub-carriers of 100 or 312.5 Hz and a modulation index of 0.698. Amplitude-modulated signals with a repetition frequency of 10 Hz are used to transmit time markers. Signals with a sub-carrier of 312.5 Hz are used to indicate second and minute markers, and also “1s” in the binary coded decimal code for the transmission of time-scale information and in the position code where UT1 – UTC time-scale difference information is being transmitted; signals with a frequency of 100 Hz are used to mark “0s” in the binary coded decimal code for the transmission of time-scale information, as well as to fill in all the other 80 ms intervals in which there is no information transmission of any kind. For the transmission of time-scale information, the first two 80 ms intervals after the second marker are used.

Notes to Table 2 (end):

- The additional information about the value of the difference UT1 – UTC is transmitted by code dUT1. It provides more precisely the difference UT1 – UTC down to multiples of 0.02 s. The total value of the correction is DUT1 + dUT1. The values of DUT1 are coded by marking the second 80 ms intervals from the first to the sixteenth second in ITU-R’s standard position single unit code. Positive values of DUT1 are transmitted in the interval between the first and eighth seconds, so that DUT1 = +0.1s n, with negative values being transmitted between the ninth and sixteenth seconds, so that DUT1 = 0.1s k, where n and k are the number of marked 80 ms intervals. The values of dUT1 are coded by marking the first 80 ms intervals in the intervals between the third and seventh or eleventh and fifteenth seconds, so that dUT1 =  0.02s p, where p is the number of marked 80 ms intervals. dUT1 information is transmitted during those second intervals which do not contain a DUT1 transmission. The dUT1 sign is coded by marking the first 80 ms interval in the seventh or fifteenth second.

- Daily, except the 3rd, 13th and 23rd days of each month.

- From 11:06 to 11:40 UTC.
From 10:06 to 10:40 UTC during daylight saving time.

- Daily, except the 2nd, 12th and 22nd days of each month.

- From 07:06 to 07:47 UTC.
From 06:06 to 06:47 UTC during daylight saving time.

- Daily, except the 4th, 14th, and 24th days of each month.

- From 09:06 to 09:47 UTC.
From 08:06 to 08:47 UTC during daylight saving time.

- Daily, except the 6th, 16th and 26th days of each month.

- From 04:06 to 04:47 and 10:06 to 10:47 UTC.
From 03:06 to 03:47 and 09:06 to 09:47 UTC during daylight saving time.

- Daily, except the 8th, 18th and 28th days of each month.

- From 05:06 to 05:47 UTC.
From 04:06 to 04:47 UTC during daylight saving time.

- Each Monday, Wednesday and Friday.

- From 09:30 to 11:30 UTC. When summer time, add one hour to the times given.

- Second pulses of 8 cycles of 1 kHz modulation during 5 min beginning at 11:00 UTC and 11:25 UTC. When summer time is in effect, add one hour to the instants given.

- Phase modulation of the carrier by  1 and – 1 radian in 0.1 s every second except the 59th second of each minute. This modulation is doubled to indicate “binary 1”. The numbers of the minute, hour, day of the month, day of the week, month and year are transmitted each minute from the 21st to the 58th second, in accordance with the French legal time scale. In addition, a “binary 1” at the 17th second indicates that the local time is 2 h ahead of UTC (summer time), a “binary 1” at the 18th second indicates when the local time is one hour ahead of UTC (winter time); a “binary 1” at the 14th second indicates the current day is a public holiday (Christmas, 14 July, etc.), a “binary 1” at the 13th second indicates that the current day is the eve of a public holiday.

- Time code used which reduces carrier by 10 dB at the beginning of each second. The code contains information on the year, day of year, hour, minute, UT1 value and status indicators for impending leap seconds and Daylight Saving Time.

TABLE 3

Characteristics of some navigational aids

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C(1) (7980-Z, 9960-Y) | Carolina Beach, NC, United States | 34 03.8 N 77 54.8 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 79 800 (3) | Continuous (4) | Nil |  1 |
| Loran-C (7980-Y) | Jupiter, Florida, United States | 27 02.0 N 80 06.9 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 79 800 (3) | Continuous (4) eLoran(11) (LDC) | Nil |  1 |
| Loran-C(1) (5930-Y, 7930-W) | Cape Race, Newfoundland | 46 46.5 N 53 10.5 W | Omni- directional | 1 000 (2) | 1 | 7 | 24 | 100 | 79 300 59 300 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (5930-X, 9960-X) | Nantucket Island, United States | 41 15.2 N 69 58.6 W | Omni- directional | 400 | 1 | 7 | 24 | 100 | 59 300 (3) 99 600 | Continuous (4) | Nil |  1 |
| Loran-C(1) (8970-M, 9960-Z) | Dana, Indiana, United States | 39 51.1 N 87 29.2 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 89 700 (3) 99 600 | Continuous (4) eLoran(11) (LDC)(12) | Nil |  1 |
| Loran-C(1) (7970-M 9980-X) | Ejde, Faeroe Islands, Denmark | 62 18.0 N 7 04.4 W | Omni- directional | 325 (2) | 1 | 7 | 24 | 100 | 79 700 (3) 99 800 | Continuous (4) | Nil |  1 |
| Loran-C (7970-W) | Sylt, F.R. of Germany | 54 48.5 N 8 17.6 E | Omni- directional | 325 (2) | 1 | 7 | 24 | 100 | 79 700 (3) | Continuous (4) | Nil |  1 |
| Loran-C (7970-X) | Bo, Norway | 68 38.1 N 14 27.8 E | Omni- directional | 165 (2) | 1 | 7 | 24 | 100 | 79 700 (3) | Continuous (4) | Nil |  1 |
| Loran-C (7970-Z) | Jan Mayen, Norway | 70 54.9 N 8 44.0 W | Omni- directional | 165 (2) | 1 | 7 | 24 | 100 | 79 700 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (5930-Z, 7930-M) | Fox Harbour, Canada | 52 22.6 N 55 42.5 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 59 300 79 300 | Continuous (4) | Nil |  1 |

TABLE 3 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C (8930-X) | Minami – Torishima, Japan | 24 17.1 N 153 58.9 E | Omni- directional | 1 100 (2) | 1 | 7 | 24 | 100 | 89 300 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (8930-Y, 7950-Y) | Tokatibuto, Japan | 42 44.6 N 143 43.2 E | Omni- directional | 1 000 (2) 600 | 1 | 7 | 24 | 100 | 89 300 (3) 79 500 | Continuous (4) | Nil |  1 |
| Loran-C(1) (8930-W, 9930-X) | Gesashi, Japan | 26 36.4 N 128 08.9 E | Omni- directional | 1 000 (2) 600 | 1 | 7 | 24 | 100 | 89 300 (3) 99 300 | Continuous (4) | Nil |  1 |
| Loran-C (8930-M 9930-Y) | Niijima, Japan | 34 24.2 N 139 16.3 E | Omni- directional | 1 000 (2) | 1 | 7 | 24 | 100 | 89 300 99 300 (3) | Continuous (4) | Nil |  1 |
| Loran-C (9990-M) | St. Paul, Pribiloff Islands, Alaska | 57 09.2 N 170 15.1W | Omni- directional | 325 (2) | 1 | 7 | 24 | 100 | 99 900 (3) | Continuous (4) | Nil |  1 |
| Loran-C (9990-X) | Attu, Alaska | 52 49.7 N 173 10.8 E | Omni- directional | 625 (2) | 1 | 7 | 24 | 100 | 99 900 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (9960-M, 8970-X) | Seneca, NY, United States | 42 42.8 N 76 49.6 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 99 600 (3) 89 700 (3) | Continuous (4) eLoran(11) (LDC)(12) | Nil |  1 |
| Loran-C(1) (9960-W, 5930-M) | Caribou, ME, United States | 46 48.5 N 67 55.6 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 59 300 (3) 99 600 (3) | Continuous (4) | Nil |  1 |

TABLE 3 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C(1) (8970-W, 7980-M) | Malone, FL, United States | 30 59.6 N 85 10.1 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 89 700 (3) 79 800 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (8970-Y 8290-W) | Baudette, MN, United States | 48 36.8 N 94 33.3 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 89 700 (3) 82 900 | Continuous (4) | Nil |  1 |
| Loran-C(1) (7980-W 9610-Z) | Grangeville, LA, United States | 30 43.6 N 90 49.7 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 79 800 (3) 96 100 | Continuous (4) eLoran(11) (LDC)(12) | Nil |  1 |
| Loran-C(1) (7980-X 9610-Y) | Raymondville, TX, United States | 26 31.9 N 97 50.0 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 79 800 (3) 96 100 | Continuous (4) | Nil |  1 |
| Loran-C(1) (9990-Y 7960-Z) | Pt. Clarence, Alaska | 65 14.7 N 166 53.2 W | Omni- directional | 1 000 (2) | 1 | 7 | 24 | 100 | 99 900 (3) 79 600 | Continuous (4) | Nil |  1 |
| Loran-C(1) (9990-Z, 7960-X) | Narrow Cape, Alaska | 57 26.3 N 152 22.2 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 99 900 79 600 (3) | Continuous (4) | Nil |  1 |
| Loran-C (7960-M) | Tok, Alaska | 63 19.7 N 142 48.5 W | Omni- directional | 540 (2) | 1 | 7 | 24 | 100 | 79 600 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (7960-Y, 5990-X) | Shoal Cove, Alaska | 55 26.3 N 131 15.3 W | Omni- directional | 540 (2) | 1 | 7 | 24 | 100 | 79 600 59 900 (3) | Continuous (4) | Nil |  1 |
| Loran-C(1) (5990-M 8290-Y) | Williams Lake, BC, Canada | 51 58.0 N 122 22.0 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 59 900 (3) 82 900 | Continuous (4) | Nil |  1 |

TABLE 3 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C(1) (5990-Y, 9940-W) | George, Washington, United States | 47 03.8 N 119 44.6 W | Omni- directional | 1 600 (2) | 1 | 7 | 24 | 100 | 59 900 99 400 (3) | Continuous (4) eLoran(11) (LDC)(12) | Nil |  1 |
| Loran-C (9940-M) | Fallon, Nevada, United States | 39 33.1 N 118 49.9 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 99 400 (3) | Continuous (4) | Nil |  1 |
| Loran-C (9940-X) | Middletown, California, United States | 38 46.9 N 122 29.7 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 99 400 (3) | Continuous (4) eLoran(11) (LDC) | Nil |  1 |
| Loran-C(1) (9940-Y 9610-W) | Searchlight, Nevada, United States | 35 19.3 N 114 48.3 W | Omni- directional | 540 (2) | 1 | 7 | 24 | 100 | 99 400 (3) 96 100 | Continuous (4) | Nil |  1 |
| Loran-C (5990-Z) | Port Hardy, BC, Canada | 50 36.5 N 127 21.5 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 59 900 (3) | Continuous (4) | Nil |  1 |
| RNS-E(A) | Briansk | 53 08 N 34 55 E | Omni- directional | 650 | 1 | 7 (5) | 10 (6) | 100 | 80 000 (7) | Continuous (4) | Nil |  5 |
| RNS-E(D) | Syzran | 53 18 N 49 07 E | Omni- directional | 700 | 1 | 6 (5) | 10 (6) | 100 | 80 000 (7) | (8) | Nil |  5 |
| RNS-V(A) | Aleksandrovsk, Sakhalinsky | 51 05 N 142 42 E | Omni- directional | 700 | 1 | 7 (9) | 12 (10) | 100 | 89 500 | Continuous | Nil |  5 |
| Loran-C (8290-M) | Havre, ND, United States | 48 44.6 N 109 58.9 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 82 900 | Continuous | Nil |  1 |
| Loran-C(1) (8290-X, 9610-V) | Gillette, WY, United States | 44 00.2 N 105 37.4 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 82 900 96 100 | Continuous eLoran(11) (LDC)(12) | Nil |  1 |

TABLE 3 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C(1) (8970-Z, 9610-M) | Boise City, ID, United States | 36 30.3 N 102 54.0 W | Omni- directional | 800 (2) | 1 | 7 | 24 | 100 | 89 700 96 100 | Continuous | Nil |  1 |
| Loran-C (9610-X) | Las Cruces, NM, United States | 32 04.3 N 106 52.1 W | Omni- directional | 400 (2) | 1 | 7 | 24 | 100 | 96 100 | Continuous eLoran(11) (LDC) | Nil |  1 |
| Loran-C (5970-M) | Pohang, Korea | 36 11.1 N 129 20.5 E | Omni- directional | 35 | 1 | 7 | 24 | 100 | 59 700 | Continuous | Nil |  1 |
| Loran-C (5970-X) | Kwangju, Korea | 35 02.4 N 126 32.5 E | Omni- directional | 35 | 1 | 7 | 24 | 100 | 59 700 | Continuous | Nil |  1 |
| Loran-C (6930-M) | Xindu, China | 23 58.1 N 111 43.1 E | Omni- directional | 1 000 | 1 | 7 | 24 | 100 | 69 300 | Continuous | Nil |  1 |
| Loran-C (6930-1) | Xinhe, China | 22 25.0 N 107 21.0 E | Omni- directional | 1 000 | 1 | 7 | 24 | 100 | 69 300 | Continuous | Nil |  1 |
| Loran-C (6930-2) | Zhangxi, China | 23 43.7 N 116 53.8 E | Omni- directional | 1 000 | 1 | 7 | 24 | 100 | 69 300 | Continuous | Nil |  1 |
| Loran-C (7170-M) | Al Khamasin, Saudi Arabia | 20 28.0 N 44 34.9 E | Omni- directional | 800 | 1 | 7 | 24 | 100 | 71 700 | Continuous | Nil |  1 |
| Loran-C(1) (7170-W, 8990-V) | Salwa, Saudi Arabia | 24 50.0 N 50 34.2 E | Omni- directional | 800 | 1 | 7 | 24 | 100 | 71 700 89 900 | Continuous | Nil |  1 |

TABLE 3 (continued)

| Station |  |  | Type of antenna(s) | Carrier power (kW) | Number of simultaneous transmissions | Period of operation |  | Standard frequencies used |  | Duration of emission |  | Uncertainty of frequency and time intervals (parts in 1012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign | Approximate location | Latitude Longitude |  |  |  | Days/ week | Hours/ day | Carrier (kHz) | Pulse repetition (s) | Time signal | Audio-modulation |  |
| Loran-C(1) (7170-X, 8990-M) | Afif, Saudi Arabia | 23 48.6 N 42 51.3 E | Omni- directional | 800 | 1 | 7 | 24 | 100 | 71 700 89 900 | Continuous | Nil |  1 |
| Loran-C(1) (7170-Y, 8990-Y) | Al Lith, Saudi Arabia | 20 13.9 N 40 12.5 E | Omni- directional | 200 | 1 | 7 | 24 | 100 | 71 700 89 900 | Continuous | Nil |  1 |
| Loran-C(1) (7170-Z, 8990-Z) | Al Muwassam, Saudi Arabia | 16 25.9 N 42 48.1 E | Omni- directional | 800 | 1 | 7 | 24 | 100 | 71 700 89 900 | Continuous | Nil |  1 |
| Loran-C (8990-W) | Ar Ruqi, Saudi Arabia | 29 01.1 N 46 37.4 E | Omni- directional | 200 | 1 | 7 | 24 | 100 | 71 700 | Continuous | Nil |  1 |
| Loran-C (8990-X) | Ash Shaykh Humayd, Saudi Arabia | 28 09.3 N 34 45.9 E | Omni- directional | 400 | 1 | 7 | 24 | 100 | 71 700 | Continuous | Nil |  1 |
| Loran-C | Anthorn, United Kingdom | 54 55 N 03 16 W | Omni- directional |  | 1 | 7 | 24 | 100 |  | Continuous eLoran(11) (Eurofix) | Nil |  1 |

TABLE 3 (end)

(1)	Dual-rated stations.

(2)	Peak radiated power.

(3)	Time pulses appear in groups of 9 for the master station (M) and groups of 8 for the secondary stations (W, X, Y, Z).

(4)	Maintained within  5 s of UTC. Time of Coincidence (TOC) with the UTC second changes with the recurrence of leap-seconds and is designated in TOC Tables issued to interested users by the US Naval Observatory, Washington DC, USA.

(5)	No transmission on 23-26 January, 18-21 February, 23-26 March, 23-26 April, 9-23 May, 23-26 June, 23-26 July, 23-26 August, 15-23 September, 23-26 October, 23-26 November, 23-26 December.

(6)	From 2100 to 0200 h and 0800 to 1500 h UTC.

(7)	The signals of primary stations (A) are marked by the transmission of an additional ninth pulse in each group. Each pulse group coinciding with a UTC second marker is marked by the transmission of an additional (tenth) pulse. In the event of coincidence with the minute marker, the subsequent ten groups are additionally marked, and in the event of coincidence with the 5 min marker after 12 s, the subsequent 11 groups are also marked. The UTC second markers are accompanied by characteristic points situated at the leading edges of the eighth pulses at a level of 0.6 of the maximum signal value.

(8)	Generally operates without a second marker. In individual cases operates with a second marker shifted in relation to UTC.

(9)	No transmission on the 20th or 21st of each month.

(10)	From 2300 to 2400 h and 0000 to 1100 h UTC.

(11)	eLoran: enhanced Loran (eLoran) services are being implemented on a number of Loran-C transmitters. These services are currently considered to be for research purposes and may be subject to change. There are two different formats in use, both providing additional data channels that include (or are intended to include) a UTC time code. The eLoran format implemented at some Loran stations in the USA is designated LDC, for Loran Data Channel. The eLoran format implemented at some Loran stations in Europe is designated Eurofix.

(12)	From 08:00 to 15:00 local time, Monday to Friday.

Authorities responsible for stations appearing in Tables 1 and 2

Station			Authority

BPM	Time and Frequency Division
National Time Service Center, NTSC
Chinese Academy of Sciences
PO Box 18 - Lintong
Shaanxi 710600, China

CHU	National Research Council of Canada
Institute for National Measurement
Standards – Frequency and Time Standards
Bldg M-36, 1200 Montreal Road
Ottawa, Ontario, K1A 0R6, Canada

DCF77	Physikalisch-Technische Bundesanstalt
Time and Frequency Department, WG 4.42
Bundesallee 100
D-38116 Braunschweig
Germany

Droitwich, Westerglen, Burghead	National Grid Wireless
Wireless House, Warwick Technology Park
Heathcote Lane
Warwick, CV34 6DD
UK

EBC	Real Instituto y Observatorio de la Armada
Cecilio Pujazón s/n
11.110 San Fernando
Cádiz, Spain

HBG	METAS
Swiss Federal Office of Metrology
Electricity, Acoustic and Time Section
Lindenweg 50
CH-3003 Bern-Wabern
Switzerland

HLA	Time and Frequency Group
Korea Research Institute of Standards and Science
P.O. Box 102, Yuseong Daejeon 305-340
Republic of Korea

JJY	Space-Time Standards Group
National Institute of Information and Communications Technology
4-2-1, Nukui-kitamachi
Koganei, Tokyo
184-8795 Japan

LOL	Servicio de Hidrografia Naval
Observatorio Naval Buenos Aires
Av. España 2099
C1107AMA - Buenos Aires, Argentina

MSF	National Physical Laboratory
Hampton Road
Teddington, Middlesex, TW11 OLW
United Kingdom
http://www.npl.co.uk/server.php?show=ConWebDoc.998

NAA, NDT, NLK, NPM,	Superintendent
NSS, NWC, NMO, NPN	US Naval Observatory
	Washington, DC 20390
	United States of America

RAT, RCH,	State Committee of Standards of the Russian Federation
RWM	Lenisky Prospect 9
	117049 Moscow, Russia

SAJ	Swedish Telecommunications Administration
Radio Services
S-123 86 Farsta, Sweden

STFS	National Physical Laboratory
Dr. K. S. Krishnan Road
New Delhi – 110012, India

TDF	France Telecom Recherche et Développement
Laboratoire RTA/D2M
Technopole ANTICIPA
2, avenue Pierre Marzin
22307 – Lannion Cedex, France

WWV, WWVB, WWVH	Time and Frequency Division, 847.00
National Institute of Standards and Technology
325 Broadway, Boulder, Colorado 80305
United States of America
WWV: 	  http://tf.nist.gov/stations/wwv.html
WWVB: http://tf.nist.gov/timefreq/stations/wwvb.htm
WWVH: http://tf.nist.gov/timefreq/stations/wwvh.htm

YVTO	Dirección de Hidrografia y Navegación
Observatorio Naval Cagigal
Apartado Postal No 6745 
Caracas, Venezuela