|  | Project Team 1 of the Conference Preparatory Group of the European Radiocommunications Committee |  | CEPT/ERC/CPG2000/PT1(99)LUX-03r2 |  |
| --- | --- | --- | --- | --- |

## DRAFT

References: PT1(99)003

Subject:	Satellite Procedures- Resolution [COM5/17] Minneapolis.

### Origin:	Luxembourg

Date:	22 February, 1999

Attached is a draft submission to the SC and WP4A meetings and it is an updated version of the LUX documents submitted to previous PT1 meetings. As you will recall this subject was discussed at the last PT1 meeting and it was agreed that this document would also be submitted to WP4A. Some of the details of this present document were not in the previous versions and there have been some additions to the document. This is reflected in the opening paragraph of the ITU document.

|  | INTERNATIONAL TELECOMMUNICATION UNION RADIOCOMMUNICATION STUDY GROUPS | Document SC99-1/LUX03-E WP4A/LUX03-E WP4-9/S/LUX03-E 21 February 1999 Original: English |
| --- | --- | --- |

## DRAFT

LUXEMBOURG

###### Satellite Procedures

###### Resolution [COM5/17] Minneapolis

1.	This document has been developed within the framework of CEPT but it is to be noted some details and elements are new to what has been discussed within CEPT.

2.	The PP998 adopted Res. [COM5/17] on the coordination and notification procedures for Satellite Networks. The objective of this Res. is to permit WRC-2000 and subsequent WRCs to review and update the various coordination and notification procedures for Satellite networks to achieve additional simplification of the procedures and cost savings for the BR and for administrations. During the past few years, the complexity of satellite networks has increased with the resulting increase in the complexity in the ITU notifications. This increased complexity together with the increased volume of notices to the ITU has resulted in very large backlogs in the processing of notices. Another important aspect is that the operational use of many satellite networks changes over time and it is probably not practical to expect operators and administrations to keep the ITU data base current. One of the other purposes of Res. [COM5/17] is to correct any problems that arise in the application of the procedures.

3.	Another important change during the past few years has been the increasing role of satellite operators. An example of this is the coordination arrangements that have been made in the MSS in the L band, in which there are general inter-governmental agreements and then the operators meet on an annual basis to modify the operational requirements for the following year. This particular process may not work for other bands/services, but it does reflect the need to use new approaches to coordination.

4.	The increasing complexity and volume of notices has increased the complexity and amount of work for BR and administrations, and thus the backlogs. It is not realistic to expect that BR will be provided with the necessary resources to be able to process the notices within the time period specified in the RR based on the present procedures.    On the other hand, the time to build and launch satellites is decreasing significantly and the long processing delays by BR, are not consistent with these shorter times. As an example, the delay for the processing of a coordination request by BR is longer than some of the contracted satellite construction and launch times.    Consequently, the most logical approach would seem to be to try to find some considerable simplification to the ITU procedures without sacrificing the major objective of the procedures.

5.	It is proposed that the following changes to the ITU procedures be considered and if these changes can be agreed in principle, then the necessary changes to provisions of the RR can be drafted and submitted to the CPM/WRC.

6.	Coordination Procedures

There are a number of related changes to the coordination procedures that would result in considerable simplicity and cost savings for both BR and administrations.

6.1	Coordination Trigger for the FSS

At the present time App. S8 (formerly App. 29) is used to determine the coordination requirements, and with the increasing complexity of the notices these calculations are very complex.    The present procedures Articles S9, S11 and the App. S8 calculations require detailed data    (App. S4-previous App. 3 data) on the satellite networks. One of the alternative approaches that was considered during the review under Res. 18 (Kyoto) was a coordination angle, but this concept was not retained at that time. The major volume of notices is for the FSS service and perhaps a more simplistic approach should be tried for that service first.

A possible approach could be that coordination would only be required with those networks that are within a specified orbital separation of the proposed network. This separation angle could be different for the different frequency bands.

| #### FSS Frequency Band | #### Coordination Trigger Angle |
| --- | --- |
| 4-8 GHz | [X] degrees |
| 8-15 GHz | [Y] degrees |
| >15 GHz | [Z] degrees |

The values of X,Y    and Z and the technical justification for these values are contained in the Annex to this document. This Annex will be submitted as an Addendum to this document.

It was recognized during the previous consideration of this matter that it might be necessary to set very large angles to ensure that coordination can be undertaken in all cases where there is need to do so. In order to permit reasonably small coordination arcs and at the same time ensure that any real need for coordination outside of these arcs is permitted, it would be possible to combine a simple and small coordination arc with the concept that is used for No. S9.11A (ex Res. 46) of self-identification outside of the coordination arc. At the same time it would be possible with the simplified coordination trigger to simplify the coordination data which must be sent to the ITU (see Section 6.2 below).

Taking these points together, one could consider the following scenario for coordination of satellite networks:

- Notifying administration submits simplified data (simplified API- see 6.2 below) to the ITU’;

- BR publishes simplified data and identifies other networks using coordination angle trigger;

- Within a period of [4] months after the publication any administration having a network outside the coordination angle and believing that it has a network which might be affected can request the detailed coordination data (App.S4) from the notifying administration;

- if this administration can, within [2 months] of receiving the detailed coordination data, shows using DT/T calculations of App. S8 that its networks might be affected (different DT/T triggers for up and down paths), then these networks (only for the frequencies/emissions identified) must also be included in the coordination process (copy to BR for information and to include the network in the list a possibly affected networks).

6.2	Coordination Data

As a result of the above simplification of the coordination trigger for the FSS, it would be possible to greatly simplify the coordination data that is necessary to be sent to the ITU for the request for coordination. With this type of coordination trigger, it would be possible to have the data sent to the ITU to be very similar to the simplified API data, perhaps with some slight additions. The detailed coordination information that is now sent to the ITU would be used for exchange between the administrations and operators for use during the coordination discussions. The consequence of this would be that for the FSS there would be a merging of the API and coordination data requirements and phases.

As the Master Register normally records frequency assignments, it is possible to consider two possible options for the recording of the coordinated data in the MIFR:

- recording the information of the simplified API plus a listing of the assigned frequencies. In some cases it may be necessary to include some additional elements to permit the BR to do any required examinations with respect to technical limits of the Radio Regulations;

- recording the existing App. S4 data to reflect what was coordinated with no validation of the data by the BR.

6.3	Separation of Up link and Down link data

At the present time, the data requirements are complicated by the need to provide strapping tables to cover all of the possible combinations of the up and down link frequencies, however, in the end it is necessary to identify separate coordination requirements for the two directions of transmissions. With the simplified coordination triggers it would be possible to treat the up and down links completely separate by looking only at the orbital separation where there is a frequency band overlap and considering the two directions separately. For identifying possible coordination needs outside the coordination arc, there would be considerable simplification in the exchange of data between administrations, if the strapping information is not a part of the data.

Even without the change in the coordination triggers there would be considerable simplification under the present process if the two directions were treated separately.

6.4	Identification of Networks Subject to Coordination

Under the present procedures, App. S8 (ex. App. 29) is used to identify the networks, with which coordination is required, but the procedures require the identification of the administrations affected and this results in some problems. Under the existing provisions in the application of No. S9.7 plus others, the BR is required to identify the administrations with which coordination is required.    The trigger requirements under App. S8 are based on individual networks. The present practice of the BR is to stop the examination for networks of a particular administration, once one network is identified. This identified network may be an insignificant or very significant problem in the coordination process. In the publications of the BR including the Special Sections and the MR, BR only identifies the administration with no identification of the networks involved. The reasons for including an administration in the coordination requirements are not public, as the networks are not listed.    When an administration receives the publication indicating that it is included in the coordination requirements for the network being published, it does not know which of its networks triggered the coordination requirement. It is then necessary for the administrations to agree on which networks will be included. Once the coordination is completed and communicated to the BR, there is no record with BR as to whether a particular network was included in the agreement or not. There are also provisions in App. S5 which state that no coordination is required for a modification if the interference is not increased, but BR has no record as to whether the networks were in fact coordinated in the first place.

If it is agreed that this problem should be fixed, the changes to the RR should be quite straightforward. BR has raised the resource/time implication, but with a good computerized system and some other improvements, the resource implications should not be significant.

This new approach would be equally applicable if a coordination angle is used for the trigger.

6.5	Technical Examination

The present Radio Regulations provide for BR to do a technical examination in those cases where coordination could not be effected.    However, these provisions have only been applied very infrequently. As a consequence of not having detailed data on all satellite networks, there would have to be some consequential changes to these provisions. This could take the form of having both networks recorded with remarks about not causing interference to each other or having BR do a technical examination (for info only) with the data to be supplied by the administrations on a case by case basis.

6.6	Coordination Between Space Stations and Terrestrial networks

Since the first provisions for the sharing of frequencies between GSO Space stations and terrestrial stations, the approach has been to have fixed hard pfd limits (i.e. Art. S21- Table S21.4) which if exceeded result in the new space network frequencies receiving an unfavourable finding with respect to the Table of Frequency Allocation and other provisions (No. S11.31). However, there is a provision (No. S21.17) which permits these limits to be exceeded subject to the agreement of the administration concerned but with no process identified for this agreement procedure. In the RR98 provisions (S9.14), the sharing between NGSO and terrestrial is treated as a coordination process within the general provisions relating to coordination. As the present procedures of hard limits between GSO and terrestrial stations was develop many years ago and the recent conferences has adopted coordination procedures, perhaps it would be appropriate to treat the GSO/terrestrial case in a similar manner. This could be done by moving the hard limits of Art. S21 to App. S5 and adding a provision similar to S9.14 but for the GSO/terrestrial situation.

##### ANNEX

## Coordination Trigger Angles

(to be developed)