| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Documents 3M/TEMP/166, 3M/4, 3M/5, 3J/FAS/9, ITU-R P.452-17, 3M/364N4, 3M/364N5, ITU-R P.838-3, and 3M/392P2 Subject: Recommendation ITU-R P.452 | Document 3M/FAS/12-E |
|  | 14 June 2023 |
|  | English only |
| Working Party 3M |  |
| fascicle 3M/FAS/12 |  |
| Fascicle on hydrometeor scatter interference |  |

Scope

This fascicle provides mathematical foundations and validation of the hydrometeor scatter interference prediction method utilized in Recommendation ITU-R P.452.

Table of contents

Page

1	Introduction		3

2	Parameters controlling hydrometeor scatter interference		3

2.1	Meteorological parameters		3

2.2	Geometrical parameters		6

2.3	Electromagnetic parameters		10

3	Rain bi-static scattering cross section		13

4	Rain specific attenuation		17

5	Hydrometeor scatter transmission loss formulation		18

6	Hydrometeor scatter geometry		19

6.1	Cylindrical integration coordinate		20

6.2	Station 1 characteristics		20

6.3	Station 2 characteristics		21

6.4	Polarization transformation		22

6.5	Integration volume		24

7	References		27

Annex A – Raindrop scattered field representation		29

Annex B – Ricatti-Bessel and Hankel functions in Mie scattering		34

Annex C – Associated Legendre polynomial in Mie scattering		36

Annex D – Bi-static cross section regression coefficients		37

Annex E – Specific attenuation regression coefficients		44

Annex F – Analytic solution for the scatter transfer integral		47

# 1	Introduction

This fascicle provides mathematical foundations and validation of the hydrometeor scatter interference prediction method. The fascicle starts by introducing the parameters impacting hydrometeor scatter (Section 2). There are three types of those parameters: meteorological parameters (Section 2.1), geometrical parameters (Section 2.2), and electromagnetic parameters (Section 2.3). Plane Earth representation and off axis squint angles are the only two geometric parameters addressed in this fascicle. Rain specific attenuation and raindrop bi-static cross section are the two electromagnetic parameters impacting hydrometeor scatter. General formulation for the specific attenuation and the bi-static cross section are considered in Section 2.3, and their polarization representation is given in Annex A. Three types of bi-static cross section for raindrops are considered: one formulation based on the Rayleigh approximation (Section 2.3.1), another formulation based on Mie scattering theorem (Section 2.3.2, Annex B, Annex C), and a new formulation derived in Section 3. Mathematical formulations relating bi-static cross section and specific attenuation are to rainfall rate and frequency are derived in Annex D and Annex E respectively. Those formulations are derived at two different temperatures: 0 and 30° C.

The hydrometeor scatter transmission loss involving the new bi-static cross section formulation as well as the new specific attenuation formulation found in Section 5. The transmission loss formulation encompasses a scatter transfer function which involves three-dimensional integration. Section 6 provides the parameters of the hydrometeor scattering geometry required for constructing the scatter transfer function. The integration of the scatter transfer function is performed numerically in [Document 3M/295 (WP 3M Chairman’s Report 3M/343 (Annex 10) (revision to Rec. ITU-R P.452, Section 5)] using Gauss-Legendre quadrature. An algorithm for calculating the nodes and weights required by Gauss-Legendre quadrature integration is given in Annex F. An analytic solution for the scatter transfer function is reported in Annex G.

# 2	Parameters controlling hydrometeor scatter interference

## 2.1	Meteorological parameters

There are two types of meteorological parameters impacting hydrometeor scatter:

–	One type related to the background atmosphere, and

–	Another type related to the hydrometeor particles.

The meteorological parameters related to the background atmosphere are the atmospheric temperature profile, and water vapour (humidity) profile. These two profiles determine the attenuation due to atmospheric gases attenuating the hydrometeor scatter signals, and can be obtained from Recommendation ITU-R P.676-13.

The meteorological parameters related to the hydrometeor particles are the hydrometeor temperature, and rainfall rate. The hydrometeor temperature, depending on the operating frequencies, determines the hydrometeor complex relative permittivity εrεr= εr'-jεr'' which determines scattering amplitudes of raindrops, which in turn determine the bi-static scattering cross sections and the specific attenuation of raindrops controlling level and spatial distributions of scattered signals by the hydrometeor particles. On the other hand, rainfall rate R is a function of rainfall terminal velocity, and it determines the raindrop size distribution (DSD) and rain cell size which are required for getting raindrop scattering amplitudes, and hence both bi-static scattering cross section and specific attenuation of raindrops. Also at lower frequencies, where the Rayleigh approximation can be used, rainfall rate can be used for getting radar reflectivity used in section 5 of Recommendation ITU-R P.452-16.

### 2.1.1	Rainfall rate

The rainfall rate Rmm/h mathematical formulation is:

R=0.6π DminDmaxD3νD nDdD	(1)

where, ν(D) is the raindrop terminal fall velocity in m/s, D is raindrop diameter in mm, and n(D) is raindrop number concentration (rain size distribution) in mm-1m-3.

### 2.1.2	Drop terminal fall velocity

Three formulas are widely used in representing the drop terminal fall velocity in terms of raindrop diameter:

νD=3.778 D0.67	(2a)

νD= -0.1021+4.932D-0.9551D2+0.07934D3-0.002362D4	(2b)

νD=9.65-10.3 exp-0.6D	(2c)

Numerical calculations showed that terminal fall velocity values based on (2b) and (2c) are in agreement with each other. This implies that terminal fall velocity values based on either (2b) or (2c) are more accurate than the corresponding values based on (2a). Accordingly, either (2b) or (2c) could be used in rain fall formulation (1b). However, instead of so doing, one of them is used in deriving the following fall velocity formulation that enables getting a closed form solution for the integral of (1).

vD=cDβexp-wD  	(3)

c=4.8870	(3a)

β=1.0586	(3b)

w=0.2191	(3c)

The above terminal fall velocity formulation is derived from simulated data through a multi-variant linear regression technique. A comparison between the predictions of (3) for terminal fall velocity, and the corresponding predictions of either (2b) or (2c) showed a good agreement between terminal velocity values based on (3) and the corresponding values based on either (2b) or (2c). Accordingly, (3) is used in rain fall integral (1).

### 2.1.3	Rain drop size distribution

Several types of formulations are usually used in formulating rain drop size distribution DSD. Among those types are negative exponential, gamma, normalized gamma, lognormal, and Weibull distributions. In this fascicle only the negative exponential, the gamma, Weibull, and lognormal distributions are considered. The negative exponential distribution functions could be captured into this formulation:

nD= N0 D exp-ΛD	(4)

N0 mm-1m-3 is a number concentration parameter, and 𝞚 mm-1 is a slope factor. The coefficients N0 and 𝞚 may be constants or functions of rainfall rate R. Quite a number of variants of the negative exponential distribution have appeared in the literature, of which perhaps the most widely used is that derived by Marshall and Palmer in 1948. Table 1 lists values for the coefficients of two of those functions.

Table 1

Coefficients of negative exponential rain size distribution functions

| Reference | Code | N0(mm-1m-3) | 𝞚 (mm-1) |
| --- | --- | --- | --- |
| Marshall and Palmer (1948) | MP | 8 000 | 4.1 R-0.21 |
| Ulbrich (Laws & Parsons) (1983) | ULP | 5 100R-0.03 | 3.8R-0.2 |

The gamma DSD is a generalized function of which the negative exponential is a special case. This DSD function was first introduced in the form:

nD= N0 Dpexp-ΛDq	(5)

This function has been employed by a number of authors, and that which is used in this study is listed in Table 2.

Table 2

Coefficients of gamma rain size distribution functions

| Reference | Code | N0(mm-1-pm-3) |  | 𝜦(mm-q) | p | q |
| --- | --- | --- | --- | --- | --- | --- |
| Ajayi and Olsen (1985) | AO | R≤15 | 260 | 0.97R-0.432 | 1.43 | 2.6 |
|  |  | R>15 | 210 | 0.71R-0.515 | 1.43 | 3.1 |

The Weibull DSD is given by

nD= N0 μξDσμ-1exp-ΛDσμ	(6)

Table 3 gives expressions for the coefficients for Weibull DSD functions used in this fascicle.

Table 3

Coefficients in Weibull DSD function

| Reference | Code | N0 | β | 𝜦 | σ |
| --- | --- | --- | --- | --- | --- |
| Åsen and Gibbins [2002] | AGW1 | 1 180 R0.23 | 1.69 R0.13 | 1 | 0.65 R0.30 |
|  | AGW1 | 421 R0.50 | 2.15 R0.1 | 1 | 0.99 R0.19 |

The lognormal DSD function is given by:

nD= N0σD2π exp-12logeD- μσ2	(7)

Table 4 gives the coefficients of the lognormal DSD used in this fascicle.

Table 4

Coefficients in Weibull DSD function

| Reference | Code | N0 | σ | μ |
| --- | --- | --- | --- | --- |
| Veyrunes | Ve | 136R0.465 | 0.3573 + 0.0051lnR | 0.0679lnR-0.103 |

### 2.1.4	Rain cell structure

This section presents only the rain cell structure because the model deals with scattering from one cell only. The extensive literature on the rain cell topic suggests that the exponential rain cell and the mixed Gaussian-exponential rain cell (Luini and Capsoni, 2011, Owolawi, and Walingo, 2015) represent at best the main integral parameters of real rain cells (e.g. coverage area, mean rain rate, rain accumulation, etc.). In this fascicle, the exponential rain cell is considered. Such a cell has a rotational symmetry within the horizontal cross-section, where the rainfall rate is assumed to vary exponentially and can be expressed as (Owolawi, and Walingo, 2015)

Rρ=Rm exp-ρ/ρ0    mm/hr	(8)

where ρ is the radial distance from the centre of the rain cell, Rm is the peak rain rate at the centre, and ρ0 is the equivalent radius of the cell:

ρ0= 10-1.5 log10RmlogeRm0.4     km,    Rm>0.4 mm/hr	(9)

To reduce computer running time (Owolawi and Walingo, 2015), the model in (9) linking Rm and 0 employs a simplified/approximated version of the corresponding original model of Luini and Capsoni (2011). On the other side, the implementation of the latter is expected to enhance the representativeness of the synthetic rain environment used for hydrometeor scattering calculations, especially by taking into account the local probability of occurrence of each rain cell.

## 2.2	Geometrical parameters

### 2.2.1	Conversion to plane-earth representation

Figure 1 depicts the geometry of two stations on the curved Earth surface, separated by a distance d (km) along Earth surface and by an angle δ at the Earth centre.

δ= dreff  rad,     and   reff=ae= k50 a        km 	(10)

In (10), reff  is the effective (median) radius of the Earth, k50 (as determined in equation (5) of ITU-R P.452-17) is the median effective Earth radius factor, and a is the average Earth radius (as determined in page 8 of ITU-R P.452-17). The local elevation angles of each station antenna are, ε1_locand ε2_loc; and the azimuthal offsets of the antenna main-beam axes (boresights) for each station from the direction of the other station are α1_loc  and α2_loc. The azimuth angles are defined as positive in the clockwise sense. Station 1 coordinate is described by xyz with the horizontal plane as the x-y plane, the x-axis pointing in the direction of Station 2, the y-axis is pointed inside the page, and the z-axis pointing vertically upwards. Station 2 coordinate is described by XYZ with the X-axis is taken to be parallel to the earth surface and pointing toward Station 1. The Y-axis is pointing out of the page, and the Z-axis is along the vertical of Station 2.

Converting a curved Earth to a plane Earth representation can be achieved through the following steps:

–	Choosing Station 1 coordinate xyz as the reference coordinate;

–	Transferring Station 2 coordinate XYZ into the reference coordinate xyz.

FIGURE 1

Geometry of two stations on curved Earth

Transferring Station 2 coordinate XYZ into the reference coordinate xyz can be achieved as follows:

–	Titling the vertical of Station 2 by the angle δ around the Y axis to make the vertical of Station 2 parallel to the vertical of the reference frame; and

–	Rotating the horizontal coordinate XY by angle π to make the X-Y plane parallel to the x-y plane.

FIGURE 2

Rotation of the coordinate XYZ around the Y yielding x'y'z'

z'

Z

δ

x'

X

Tilting the vertical coordinate XYZ by the angle δ yields a coordinate x'y'z'. The unit vectors X, Y, and Z are related to the unit vectors x', y', and z' as follow

X= cosδ x'-sinδz'	(11a)

Y=y'	(11b)

Z= sinδx'+ cosδz'	(11c)

Furthermore, rotating the horizontal coordinate x'y' by the angle π brings the coordinate x'y'z' to the reference coordinate xyz.

x'=-x	(12a)

y'=-y	(12b)

z'=z	(12c)

Introducing (12) into (11) transfers the coordinate XYZ into the reference coordinate xyz

X= -cosδ x-sinδz	(13a)

Y=-y	(13b)

Z= -sinδx+ cosδz	(13c)

In the reference coordinate, Station 1 antenna has a height of null value (h1=0) and Station 2 antenna has a height h2 with:

h2=h2_loc-h1_loc-dδ2                km	(14)

Moreover, the elevation ε1 and azimuth α1angles describing the boresight of Station 1 antenna in the reference frame are equal to the corresponding local angles.

ε1= ε1_loc,    α1= α1_loc                rad	(15)

As for the formulations of the elevation angle ε2 and the azimuth angle α2 describing the boresight axis of Station 2 antenna in the reference frame they can be derived as follows. First the boresight axis of Station 2 antenna V20_loc is represented in the local frame XYZ of Station 2 as in (16).

V20_loc= cosε2_loccosα2_locX+ sinα2_locY+ sinε2_locZ	(16)

Then it is represented by V20 in the reference frame xyz as in (17).

V20= cosε2cosα2x+ sinα2y+ sinε2z	(17)

Introducing (13) into (16) yields (18):

V20= sinε2_locsinδ-cosε2_loccosα2_loccosδx-cosε2_locsinα2_locy

+ sinε2_loccosδ+cosε2_loccosα2_loc sinδz	(18)

Comparing (18) against (17) leads to the following identities:

sinε2=sinε2_loccosδ+cosε2_loc cosα2_locsinδ	(19)

tanα2= cosε2_locsinα2_loccosε2_loccosα2_loccosδ-sinε2_locsinδ	(20)

From (19) the elevation angle of Station 2 in the plane Earth representation is:

ε2=sin-1sinε2_loccosδ+cosα2_loccosα2_loc sinδ	(21)

And from (20), the azimuthal offset of Station 2 from Station 1 is:

α2=tan-1 cosε2_locsinα2_loccosε2_loccosα2_loccosδ-sinε2_locsinδ	(22)

### 2.2.2	Off axis squint angles

The off axis squint angles are required to determine if hydrometeor scatter can cause interference. These angles are formulated for the rain scatter geometry depicted in Figure 3.

FIGURE 3

Schematic of rain scatter geometry for the general case of side scattering

In getting the off axis squint angles, the following vector notations are used:

–	a vector in three-dimensional space is represented by a three-element single-column matrix of the lengths of the projections of the line onto the Cartesian x, y and z axes;

–	a vector is represented by a symbol in bold typeface. Unit vector is represented by the symbol V; and

–	a general vector (i.e. including magnitude) is represented by another, appropriate symbol, for example R.

The interference path in Figure 3 may be from the Station 2 side-lobes into the Station 1 main beam, or vice versa. Moreover, centre of the rain cell is located along the main beam antenna axis of Station 1 at the point of closest approach between the two antenna beams. The geometry is established in vector notation as follows. The vector from Station 1 to Station 2 is defined as:

R12= d0h2    km	(23)

In the above, d is the distance between the two stations along the Earth surface, and h2 is height of Station 2 relative to the height of Station 1. The height h2 accounts for Earth curvature if it is required as described in Section 2.2.1. The unit-length vector V10 in the direction of the Station 1 antenna main beam is:

V10=cosε1cosα1cosε1sinα1sinε1=x10y10z10	(24)

and the unit-length vector V20 in the direction of the Station 2 antenna main beam with taking Earth surface curvature into account (section 2.2.1) is:

V20=sinε2_locsinδ-cosε2_loccosα2_loccosδ- cosε2_locsinα2_locsinε2_loccosδ+cosε2_loccosα2_loc sinδ=cosε2cosα2cosε2sinα2sinε2=x20y20z20	(25)

To determine the off axis squint angles the vectors R12, r2V20, rSVS0 and r1V10 are considered with the vector VS0 perpendicular to both V10 and V20.

VS0= V20 × V10sinφmS=1sinφmSy20z10- z20y10z20x10- x20z10x20y10- y20x10= xS0yS0zS0 	(26)

Since the four vectors R12, r2V20, rSVS0 and r1V10 form a closed three-dimensional polygon, the identity (27) can be written:

(27)

The identity (27) can be converted into a matrix equation that can be solved for the unknown distances r1, r2, rS as in (28).

rsr1r2= xS0x10-x20yS0y10-y20zS0z10-z20-1d0h2	(28)

The operator -1 in equations (29) is the inverse matrix operator.

Getting the unknown distances r1, r2, rS enables calculating the off-axis squint angle ψ1,2 at Station 1 or 2 of the point of closest approach on the other station main beam axis as in (29).

Ψ1,2=tan-1rSr1,2	(29)

## 2.3	Electromagnetic parameters

Raindrop electromagnetic parameters represented by the bi-static cross section σpq per unit volume, and the specific attenuation γq of raindrops are required for hydrometeor scatter study. For a field with qi polarization illuminating an integration element in i direction the specific attenuation γq is:

γq= 4πkDminDmaxnD Im fqq(i, i)dD	(30)

and the bi-static cross section σpq in a scattering direction s for a polarization ps is:

σpq= 4πDminDmaxnD fpqs, i2 dD	(31)

In (31), fpqs, i is the pq element of the scattering amplitude matrix. Details about the scattering amplitude and field polarization representations are given in Annex A. In (30), fppi, i is the scattering amplitude in the forward direction, and Im is the imaginary part operator.

The scattering amplitude fqqs, i of a single raindrop can be obtained through either matching the boundary conditions at the raindrop surface or using the following integrand:

fpq s,i  = k24πεr-1p⋅Eq(r')expjks⋅r' dv'	(32)

In (32), Eq(r') is the field inside the raindrop at a position vector r' when the incident field has q polarization, and εr (εr=εr'-jεr'') is complex relative permittivity of raindrop. Moreover, the integral in (32) is carried over the raindrop volume. The field inside the raindrop can be obtained through either numerical techniques or approximate techniques. The most widely used approximate techniques are the Rayleigh approximation, and the Rayleigh - Gans approximation. The Rayleigh approximation, which is used in section 5 of Recommendation ITU-R P.452-16, is valid only for raindrops with diameters very small compared to the operational wavelength. This is because in the Rayleigh approximation the inner field is estimated by the corresponding static field.

The Rayleigh – Gans approximation, in which the inner field is estimated by the incident field, is applicable only to tenuous particles with εr~ 1. Th kind of particle cannot be used to model raindrop particles. This is because raindrops have complex relative permittivity with values that could reach 80 or 90 depending on frequency. On the other hand, matching the boundary conditions at the raindrop surface yields more rigorous formulation to the scattering amplitude element such as Mie scattering formulation which is reported in section 2.3.2 of this fascicle.

### 2.3.1	Rayleigh approximation

For a spherical raindrop with diameter very small compared to the wavelength, the inner normalized field amplitude Eq(r') with a unit incident electric field |Eq0r'|=1 can be approximated as:

Eqr'= 3 qiεr+2 	(33)

Introducing (33) into (32) yields the following scattering amplitude formulation:

fpqs, i= 3 k2v04π  εr-1εr+2 ps⋅qi	(34)

with v0 is the raindrop volume. Substituting for the volume v0 v0=4π3 D23 and the wavenumber k (k=2π/λ, 𝛌 is wavelength) into (34) yields:

fpqs, i=12πλ2εr-1εr+2D3ps⋅qi  	(35)

Based on (31) and (35), the radar cross section per unit volume under the Rayleigh approximation can be written as:

σpq=10-18 π5λ4 εr-1εr+22Z Mpq 	(36)

Z= DminDmaxD6nDdD           mm6/m3	(37)

Mpq=ps.qi2  	(38)

The factor 10-18 is introduced into (36) to make the units of radar reflectivity Z in mm6/m3. Dropping the last term of (36) and introducing the resultant into (73) of Recommendation ITU-R P.452-16 yields (74) of Recommendation ITU-R P.452-16 except of the high frequency correction term. The radar reflectivity is related to the rainfall rate R through the following relationship:

Z=A Rb	(39)

Table 5 provides some typical values for the coefficients A and b of equation (39).

Table 5

Coefficients for usage of Z-R relationship (39)

| Usage | A | b | Optimum for | Recommended for |
| --- | --- | --- | --- | --- |
| Marshall – Palmer | 200 | 1.6 | General stratiform precipitation |  |
| East – Cool Stratiform | 130 | 2.0 | Winter stratiform precipitation east of continental divide | Orographic rain – East |
| West – Cool Stratiform | 75 | 2.0 | Winter stratiform precipitation west of continental divide | Orographic rain – West |
| WSR-88D Convective | 300 | 1.4 | Summer Deep convection | Other non-tropical convection |
| Rosenfeld Tropical | 250 | 1.2 | Tropical convective systems |  |

It is worth mentioning that Mpq's in (36) and (38) are the polarization loss factors that were not accounted for by ITU-R P.452-16. The explicit expressions of these factors can be written through using formulations reported in Equations (A-3)-(A-5), and the corresponding formulations in the scattered directions are as follows.

Mvv= sinεisinεscosαs-αi+cosεicosεs2	(40a)

Mvh= sinεssinαs-αi2	(40b)

Mhv= sinεisinαs-αi2	(40c)

Mhh= cosαs-αi2	(40d)

In the above, εi and εs are the elevation angles of the incident i and scattered s directions respectively (Figure A-1). The angles αi and αs are the corresponding azimuth angles.

### 2.3.2	Mie scattering theorem

Based on the Mie scattering theorem the bi-static scattering cross section σpq per unit volume due to scattering from a spherical raindrop is:

σpq=DminDmaxnD ξpq dD	(41)

Where ξpq can be written after substituting for the scattering amplitudes in (31) from (A-24) – (A-27) yielding:

ξvv=4πk2 S2φscosαvscosαvi+S1φssinαvssinαvi2	(42a)

ξvh=4πk2 S2φscosαvssinαvi-S1φssinαvscosαvi2	(42b)

ξhv= 4πk2S2φssinαvscosαvi-S1φscosαvssinαvi2	(42c)

ξhh=4πk2 S2φssinαvssinαvi+S1φscosαvscosαvi2	(42d)

In the above, φs is the scattering angle (A-10). In addition, αvi is the angle that rotates from the incident vertical polarization vector vi anticlockwise to the polarization vector 1i (Figure A-2 and A-3). Furthermore, αvs is the angle that rotates from vs scattered vertical polarization anticlockwise to 1s (Figure A-2 and A-3). S1φs and S2φs are Mie’s scattering amplitude in the scattering plane system, which are called local scattering amplitudes:

S1φs= n=1∞2n+1nn+1 anπncosφs+bnτncosφs	(43a)

S2φs=n=1∞2n+1nn+1 anτncosφs+bnπncosφs	(43b)

In (43a) and (43b), an and bn are Mie scattering coefficients.

an= yψny ψn'x-xψnx ψn'yyψny ξn'x-xξnx  ψn'y

bn= yψnx ψn'y-xψny ψn'xyξnx ψn'y-xψny ξn'x	(44)

x=kD/2, and y=εrx. In addition, ψnx and ξnx are the Ricatti Bessel and Hankel functions; and ψn'x and ξn'x are the corresponding first order derivatives with respect to the argument. Details about Ricatti Bessel and Hankel functions; and their first order derivatives are given in Annex B. Furthermore, πn(cosφs) and τn(cosφs) in (43) are the associated Legendre polynomials and their derivatives are given in Annex C.

# 3	Rain bi-static scattering cross section

The proposed rain bi-static scattering cross section σpq per unit volume is taken to be the bi-static cross section reported in (41) based on Mie scattering. These bi-static cross section formulations are used because they yield more rigorous values. Those values can also be used to investigate the validity domains of the corresponding values based on the Rayleigh approximation.

Introducing (42a)-(42d) into (41) and performing the integration yields the following formulations for the rain bi-static cross section per unit volume.

σvv= η2cosαvscosαvi2+η1sinαvssinαvi2+12η12sin2αvisin2αvs

σvh= η2cosαvssinαvi2+η1sinαvscosαvi2-12η12sin2αvisin2αvs

σhv= η2sinαvscosαvi2+η1cosαvssinαvi2-12η12sin2αvisin2αvs

σhh= η2sinαvssinαvi2+η1cosαvscosαvi2+12η12sin2αvisin2αvs	(45)

with η1, η2, and η12 are the local bi-static scattering cross sections per unit volume

η1= 4πk2DminDmaxn(D)S1φs2dD

η2= 4πk2DminDmaxn(D)S2φs2dD

η12= 4πk2DminDmaxn(D)ReS1φsS2*φsdD	(46)

The angles αvs and αviin (45) provide the dependence of bi-static cross sections on the geometric factors and polarizations. Moreover, numerical and analytical analysis showed that for frequencies up to 100 GHz, the following identities hold.

η2=η1cosφs2

η12=η1cosφs 	(47)

Figure 4 and Figure 5 are presented to justify the above identities. In both figures, values of two bi-static cross section ratios are calculated:

–	Like polarized ratio (η2/η1), and

–	Cross polarized ratio (η12/η1)

The values of like polarized ratio and cross polarized ratio are compared against the corresponding values of cosφs2 and cosφs respectively. In calculating Figures 4 and 5, Marshall and Palmer drop size distribution of Table 1 is used for n(D) with Dmin = 0.1 mm, and Dmax = 5 mm. The integration in (46) is carried out using trapezoidal rule with equal diameter steps of 0.4 mm. Moreover, the complex relative permittivity εr of the raindrop is equated to the corresponding complex relative permittivity of pure water given by equations (5) – (13) of ITU-R P. 527-4.

Figure 4

Like (η2/η1) and cross (η12/η1) polarized bi-static cross section at a frequency of 20 GHz (cosine stands for cosφs , and cosine square stands for cosφs2, Rainfall rate = 100 mm/hr)

Figure 5

Like (η2/η1) and cross (η12/η1) polarized bi-static cross section at a frequency of 100 GHz (cosine stands for cosφs , and cosine square stands for cosφs2, Rainfall rate = 100 mm/hr)

The trends of the like and cross polarized ratios represented by (47) and depicted in Figure 4 and Figure 5 are accepted. This is because due to the symmetry of the raindrop at both forward scattering direction (φs=0°) and backscattering direction (φs= 180°), the bi-static cross sections η1 and η2 should have equal values. At scattering angle of 90 degrees, the two polarization vectors associated with η2 ( 2i and 2s in Figure A-3) are perpendicular to each other as it can be inferred from (A-8) and (A-9). Moreover, (47) agrees with Equation (1.2.10), page 11 of Tsang et al (2000).

Based on (47), the bi-static cross section formulation given in (45) can be simplified into

σvv=η1 cosφscosαvscosαvi+sinαvssinαvi2	(48a)

σvh=η1 cosφscosαvssinαvi-sinαvscosαvi2	(48b)

σhv= η1cosφssinαvscosαvi-cosαvssinαvi2	(48c)

σhh=η1 cosφssinαvssinαvi+cosαvscosαvi2	(48d)

From (48a) – (48d), it is obvious that the local bi-static cross section η1 provides the dependence on rainfall rate, rain temperature, and frequency as well as scattering angle. This is illustrated in Figure 6 and Figure 7 where the horizontal local bi-static cross η1is calculated as a function of scattering angle at two different temperatures and two different frequencies. Figures 6 and 7 are calculated using the same parameters used in calculating Figures 4 and 5.

Figure 6

Local bi-static cross section (η1) at a frequency of 20 GHz

Figure 7

Local bi-static cross section (η1) at a frequency of 100 GHz

The explicit dependence of the bi-static scattering section η1 on the scattering angle can be written as.

η1=ηh= expu0+u1sin0.5φs2+u2sin0.5φs4+u3sin0.5φs6	(49)

The above formulation is derived through applying multi-variant linear regression on bi-static scattering cross simulated. The simulated data are generated through performing the integration of the first equation of (47) using different values for the scattering angle.

Equations (50) provide the dependence of the coefficients ui's (i=0,1,2,3) of (49) on rainfall rate.

gi= a0i+ a1iloge(R)+ a2iloge(R)2,               i=0, 1

gi= a0i+ a1iloge(R)+ a2iloge(R)2+a3iloge(R)3,      i=2,3	(50)

The dependence of the regression coefficients aim(m=0,1,2,3, and i=1,2,3) of (50) on frequency, and temperature is provided in Annex D.

In concluding this section it is worth mentioning that the quantities within the curly brackets of (48a) – (48d) can be considered as polarization loss factors in analogy to those reported in (40a) – (40b) for the Rayleigh approximation.

# 4	Rain specific attenuation

The rain specific attenuation prediction model in this recommendation is different from that in Recommendation ITU-R P. 838-3 and should only be used for the purposes as required in § 5 for hydrometeor-scatter interference prediction. The reason for this warning is because the rain specific attenuation prediction model of Recommendation ITU-R P. 838-3 is developed for oblate spheroidal raindrops, while the hydrometeor scatter model is developed for spherical raindrops. This difference has an impact on formulations that include the specific rain attenuation formulations of Equations (82) – (87), the raindrop bistatic cross section formulations of Equations (123a) – (123d). Furthermore, the rain specific attenuation model of Recommendation ITU-R P.838-3 has no temperature dependence, while both the rain specific attenuation and the bistatic cross section in this section have temperature dependence.

Based on Mie scattering theorem, the rain specific attenuation can be obtained through introducing (A-29) into (30) yielding:

γmv=γmh= 2πk2DminDmaxnDn=1∞2n+1 Rean+bndD	(51)

Figure 8

Rain specific attenuation based on (51) as a function of frequency at two different rainfall values and two different temperature values

Based on (51), the specific attenuation due to spherical raindrops is independent on either polarization or the scattering angle. It depends only on rainfall rate, frequency, and temperature as shown in Figure 8 which is calculated using the same parameters used in calculating Figure 4 and Figure 5.

The dependence of rain specific attenuation on rainfall rate is given in (52):

γv=γh= κ Rα	(52)

and explicit dependence of the above regression coefficients κ and α on frequency, and temperature is given in Annex E.

# 5	Hydrometeor scatter transmission loss formulation

To derive the hydrometeor scatter transmission loss between two stations located on the Earth surface equation (73) of Recommendation ITU-R P.452-16 is recalled.

Pr= Pt λ24π3 G1G2r12r22 σpqexp-cKatm+Krainζ(h) δV     W 	(53)

where:

Pt: 	transmitted power (W)

Pr: 	received power (W)

𝛌: 	wavelength (m)

G1,2: 	antenna 1 and antenna 2 gain (linear)

σpq: 	raindrop bi-static cross section per unit volume (m2/m3)

δV: 	differential volume at the scattering point (m3)

r1,2: 	distances from scattering volume to antenna 1 and antenna 2 respectively (m)

Katm: 	attenuation due to atmospheric gases (dB)

Krain: 	attenuation due to rain (dB)

c: 	a constant for converting dB to Nepers (c=0.23026)

ζ(h):	height dependence of the radar reflectivity:

ζ(h)=1                              for h≤hR10-0.65(h-hR)    for h>hR

Equation (53) can be rewritten as:

Prp=10-3Ptqλ24π3 G1G2 σpqR12R22exp-cKatm+Krain ζ(h)dV,                W	(54)

where

R1,2: 	distances in km (R1,2= r1,2/103)

dV: 	scattering volume in km3 (dV=δV/109).

Substituting for the wavelength in terms of the frequency reduces (53) into:

Prp= Ptq4.5291×10-8f2 G1G2 σpqR12R22exp-cKatm+Krainζ(h) dV,                  W	(55)

with f is the frequency in GHz (f λ=0.299792458).

On the other hand, the transmission loss L in dB unit is defined as:

Lpq=10log10PtqPrp	(56)

Accordingly, based on (54)-(55) the scattering loss transmission loss can be written as:

Lpq =73.4399+20log10f-10log10Cpq,         dB	(57)

With Cpq is the scatter transfer function:

Cpq=G1G2σpqR12R22exp-cKatm+Krainζ(h) dV,            m-1 km-1	(58)

Equations (57) and (58) provide the proposed transmission loss formulation for updating section 5 of Recommendation ITU-R P.452-16.

The calculation of the path attenuation Krain relies on the single rain cell illustrated in Section 2.1.4: the prediction of path attenuation could be enhanced by employing full synthetic rain fields resulting from the aggregation of multiple rain cells (Luini and Capsoni, 2011).

# 6	Hydrometeor scatter geometry

In constructing the hydrometeor scatter geometry, the centre of the rain cell may be varied depending on the scenarios that minimize values of transmission losses, and hence maximize the interfering scattered power. Then a cylindrical coordinate system is established to facilitate the integration of the scatter transfer function (58). The characteristics of both station antennas in the cylindrical coordinate are derived along with the integration limits. The station characteristics are represented by position vectors from the station to the integration elements, the polarization vectors associated with each position vector, the polarization transformation, and the off boresight angle between each position vector and the boresight of the corresponding antenna.

## 6.1	Cylindrical integration coordinate

The cylindrical coordinate system (ρ, φ, z) is constructed through selecting a rain cell with its vertical axis intersecting the main beam axis of Station 1 at point 0 (x0, y0, h0) as shown in Figure 9.

x0= r1cosε1cosα1

y0= r1cosε1sinα1	(59)

h0= r1sinε1

The distance r1in (59) can be obtained from (28). By selecting the point 0 (x0, y0, h0) as the centre of the cylindrical coordinate, the Cartesian coordinate of an arbitrary point A (x, y, h) within the cell can be transformed into a cylindrical coordinate (ρ, φ, z) as in (60).

x= x0+ρcosφ

y= y0+ρsinφ

h= h0+z

dV=ρ dρdφdz	(60)

Figure 9

Hydrometeor scatter link geometry

## 6.2	Station 1 characteristics

The vector RA1(RA1=rA1VA1,  rA1  is the magnitude, and VA1is the unit length vector) is the position vector extending from Station 1 to the integration element A (x, y, h). The magnitude rA1 of this vector can be expressed as follows:

rA1= x2+ y2+h2	(61)

Introducing (60) into (61) yields:

rA1= r1DA1,

DA1= 1+ρ2+z2+2ρd1cosα1-φ+2h0z/r12

r1= d12+h02,       d1=x02+y02	(62)

The direction of the vector RA1is given by the unit vector VA1:

VA1=1rA1 xyh=1r1DA1 x0+ρcosφy0+ρsinφh0+z-h1=cosεA1cosαA1cosεA1sinαA1sinεA1	(63)

RA1(ρ,φ,z)=xyz=x0+ρcosφy0+ρsinφh0+z-h1	(64)

with:

εA1= sin-1h0+zr1DA1	(65)

αA1=tan-1VA1⋅yVA1⋅x=tan-1y0+ρsinφx0+ρcosφ 	(66)

The off-boresight angle at the point (x, y, h) for the Station 1 antenna is:

θb1= cos-1VA1⋅V10= cos-1r12+ d1ρcosφ-α1+h0zr12D1A	(67)

Equations (24) and (63) are used to obtain the vector scalar product VA1.V10 in (67).

The horizontal hA1, and vertical vA1 polarization vectors associated with the unit vector VA1 , and hence the transmitting antenna (Station 1) are given by:

hA1= 1dB1-yx0= 1dB1-y0+ρsinφx0+ρcosφ0=-sinαA1cosαA10	(68)

vA1=1dB12(h0+z-h1)2rA1+(h0+z-h1)4rA12+dB12-dB1(h0+z-h1)rA1-(h0+z-h1)2rA1dB12=sinεA1cosαA1sinεA1sinαA1-cosεA1	(69)

dB1=x2+y2= d12+2ρd1cosφ-α1+ρ2	(70)

In getting (68)-(70), the unit vector in the incident direction i in (A-4) is replaced by the unit vector VA1 given by (63). Then the resultant is introduced into (A-3) and (A-5) respectively.

## 6.3	Station 2 characteristics

The position vector RA2 (RA2=rA2VA2, rA2 is the magnitude and VA2 is unit vector) extends from Station 2 to the integration element at the point A. The magnitude of this vector is the distance rA2 from Station 2 to the integration element at A(ρ,φ,z):

rA2= x-d2+ y2+h-h22	(71)

Equation (71) can be reduced through using (60) yielding:

rA2= r2'DA2

DA2= 1+ ρ2+z2+2ρd2'cosα2'-φ+2h0-h2z/r2'2

r2'= d2'2+h0-h22 ,     d2'=x0-d2+y02	(72)

RA2(ρ,φ,z)=x'y'z'=x0+ρcosφ-dy0+ρsinφh0+z-h2	(73)

The direction of the vector RA2is given by the unit vector VA2:

VA2=1rA2'x-dyh-h2= 1r2'DA2x0-d+ρcosφy0+ρsinφh0-h2+z=cosεA2cosαA2cosεA2sinαA2sinεA2	(74)

with:

α2=tan-1y0x0-d

εA2= sin-1h0-h2+zr2'DA2	(75)

αA2=tan-1y0+ρsinφx0-d+ρcosφ 	(76)

The off-boresight angle θb2 between the point A(x, y, h) for the Station 2 antenna the angle:

θb2=cos-1VA2⋅V20=cos-1cosεA2cosε2cosα2-αA2+sinε2sinεA2	(77)

In (77), ε2 and α2 are the elevation angle and the azimuth angle of boresight of Station 2 antenna, and they can be obtained from (25).

For the receiving antenna, the associated horizontal hA2 and vertical vA2 polarization vectors are:

hA2=1dB2-yx-d0=1dB2   -(y0+ρsinφ)x0-d+ρcosφ0=-sinαA2cosαA20	(78)

vA2=1rA2'dB2h-h2x-dh-h2y-dB22	(79)

vA2=1dB22(h0+z-h2)2rA2+(h0+z-h2)4rA22+dB22-dB2(h0+z-h2)rA1-(h0+z-h2)2rA2dB22=sinεA2cosαA2sinεA2sinαA2-cosεA2	(80)

dB2=x-d2+y2= d22+2ρd2cosφ-α2+ρ2	(81)

In getting (78)-(81), the unit vector in the scattering direction i in (A-4) is replaced by the unit vector VA1 given by (73). Then the resultant is introduced into (A-3) and (A-5) respectively.

## 6.4	Polarization transformation

At the integration volume, A(ρ, φ, z) the propagating signals undergoes through two polarization transformations. The first transformation is for the signal emitted by Station 1 antenna and illuminating the integration volume, and the second transformation is for the signal scattered by the integration volume toward Station 2 antenna. As shown in Annex A, to establish these two transformations the following parameters are required:

–	the scattering angle φs;

–	the angle αvi, and

–	the angle αvs.

The angles αvi, and αvs are the rotation angles of the incident and scattered vertical polarizations anticlockwise to the polarization vector 1i (Figure A-2).

The scattering angle can be written as:

φs=cos-1-VA2.VA1=cos-1xx-d+ y2+hh-h2r1Ar2A	(82)

The unit vectors VA1 and VA2 in (82) are substituted for from (63) and (74) respectively. Further reduction for (82) yields:

φs=cos-1r1A2-xd+hh2r1Ar2A	(83)

Then subsisting for x and h from (60) leads to the following scattering angle formulation:

φs=cos-1r1Ar2A- dx0+ρcosφ+h2h0+zr1Ar2A	(84)

The rotation angle αvi can be obtained from (A-12) which recalled in (85) for convenience:

αvi=tan-1v1A⋅V2Ah1A⋅V2A	(85)

Subsisting for the unit vectors v1A, h1A, and V2A from (70), (68), and (75) reduces (85) into:

αvi=tan-1hxx-d+hy2-dB12h-h2rA1-x-dy+yx	(86)

Then using (60) yields the required formulation for the rotation angle αvi:

αvi=tan-1h0+zx0+ρcosφd+ h2dB12r1ady0+ρsinφ	(87)

The rotation angle αvs can be written from (A-15) as:

αvs= cos-1-sinαvih1A⋅ v2A+ cosαviv1A⋅v2A	(88)

with:

h1A. v2A= 1dB1-y0+ρsinφx0+ρcosφ0⋅11+h0+z-h22-11dB2h0+z-h2dB2rh0+z-h2T=1dB11+h0+z-h22-y0+ρsinφ+x0+ρcosφdB2h0+z-h2	(89)

and

v1A⋅ v2A= 11+h0+z-h12-11dB1h0+z-h1dB1r1h0+z-h1 ⋅11+h0+z-h22-11dB2h0+z-h2dB2r2h0+z-h2T	(90)

with T stands for matrix transpose. A further reduction for (89) and (90) yields:

h1A⋅ v2A  =1dB11+h0+z-h22-y0+ρsinφ+x0+ρcosφdB2h0+z-h2	(91)

and

v1A⋅ v2A= 11+h0+z-h221+1dB12h0+z-h12+dB22r22h0+z-h22	(92)

In getting the vector scalar product in (91) and (92), equations (67), (68), and (73) are used.

## 6.5	Integration volume

The common volume between the two antenna beams is usually the integration volume. However, determining the common volume involves solution of several algebraic equations which is difficult to achieve. Accordingly, instead an alternate volume enclosing the common volume is used for performing the integration. The alternate volume is a cylindrical volume with a centre located at the point 0 (x0, y0, h0) where the rain cell axis intersects with Station 1 antenna main beam as shown in Figure 10. This volume has a symmetry with the azimuth angle φ. Due to this symmetry the integration over φ extends from 0 to 2π. On the other hand, the vertical dimensions of the alternate volume are determined by the points where main beams of either Station 1 antenna or Station 2 antenna, intersect with the axis of rain cell. The points associated with farther distances for the integration centre are selected.

FIGURE 10

Geometry for determining vertical dimensions of the integration volume

The geometry for obtaining the points of intersection of Station 1 antenna main beam with rain cell axis is given in Figure 10. From the triangle S10zup depicted in Figure 10, the distance between the point 0 and point where the upper portion of the antenna beam intersects with rain cell axis is:

z1max= x02+y02tan(ϵ1+0.5BW1)-z0	(93)

From the triangle S10zdn the distance between the point 0 and the lower portion of antenna beam intersects with rain cell axis is:

z1min= x02+y02tan(ϵ1-0.5BW1)-z0	(94)

Similar to (93)-(94), the corresponding distances for Station 2 antenna main beam be written as:

z2max= x0-d2+y02tan(ϵ2+0.5BW1)-z0	(95)

z2min= x0-d2+y02tan(ϵ2-0.5BW1)-z0	(96)

By selecting the distances of maximum values, the vertical dimensions of the integration volume, and hence the limits of integration over the vertical variable z can be written as:

zmax= max⁡(z1max,z2max  )	(97)

zmin=min⁡(z1min,z2min  ) 	(98)

Figure 11

Geometry for determining radius of the integration volume

The geometry for getting the radius ρmax, of the integration volume is given in Figure 11. From the triangle S1QS2, in Figure 12, the distance S1Q can be written as:

S1Q1=dsinε2+0.5BW2sinε1-0.5BW1+ε2+0.5BW2 	(99)

S2Q2=dsinε1-0.5BW2sinε2-0.5BW1+ε1+0.5BW2 	(100)

Subtracting the distance x0 from the horizontal projection of the distance S1Q gives the radius ρmax:

ρmax=S1Q1cosε1-0.5BW1-x0 	(101)

Incorporating (99) into (101) gives the explicit expression for the radius of the integration volume.

ρmax1=S1Q1cosε1-0.5BW1-x02+y02   	(102)

ρmax2=S2Q2cosε2-0.5BW1+d-x02+y02	(103)

# 7	References

[1]	Abramowitz, M., and I. A. Stegun ed., Handbook of Mathematical Functions with Formulas, Graphs and Mathematical Tables, Dover Pub. In. New York, 1965.

[2]	Ajayi, G. O. and Olsen, R. L. (1985), “Modelling of a tropical raindrop size distribution for microwave and millimeter wave applications”, Radio Sci., 20, pp 193-202.

[3]	Afullo, T. J. O., “Raindrop size distribution modeling for radio link design along the eastern coast of South Africa,” Progress in Electromagnetics Research B, Vol. 34, pp. 345-366, 2011.

[4]	Akuon, P.O., and T. J. O. Afullo, “Rain cell size mapping for microwave link design systems in South Africa,” Progress in Electromagnetic Research B, Vol. 35, pp. 263-285, 2011.

[5]	Arai,K., “Comparison between Rayleigh and Mie scattering assumptions for Z-R relation and rainfall rate estimation with TRMM/PR data,” International Journal of Advanced Research in Artificial Intelligence, Vol. 2, No. 8, 2013.

[6]	Åsen, W. and Gibbins C. J. (2002), “A comparison of rain attenuation and drop size distributions measured in Chilbolton and Singapore”, Radio Sci.Bohren, C. F.; and D. R. Huffman, Absorption and Scattering of Light by Small Particles, Chapter 4, John wiley & Sons, New York, 1983.

[7]	Goldhirsh, J.,” Two dimensional visualization of rain cell structure,” Radio Science, Vol. 35, No. 3, pp. 713-720, 2000.

[8]	Ishimaru, A., Wave Propagation and Scattering in Random Media, Vol. 1 Chapter 1, pp. 27-30, and Vol. II, Academic Press, New York, 1978.

[9]	Karam, M. A.; and A. K. Fung, “Electromagnetic energy absorbed within a Mie sphere,” Journal of Electromagnetic Waves and Applications, Vol. 7, No. 10, pp. 1379-1387, 1993.

[10]	Karam, M. A., and A. K. Fung, “Vector forward scattering theorem,” Radio Science, 17(4), pp. 752-756, 1982.

[11]	Karam, M. A., “Polarimetric optical theorem,” Journal of Optical Society of America A, vol. 15, no. 1, pp. 196-201, 1998.

[12] 	Luini, L., and C. Capsoni, “MultiEXCELL: A new rain field model for propagation applications,” IEEE Transaction on Antennas and Propagation, Vol. 59, No. 11, PP. 4286 – 4300, 2011.

[13]	Mulangu, C. T., S. J. Malinga, and T. J. Afullo, “Prediction of radar reflectivity along radio link, PIERS Proceedings, Taipi, March 25-28, 2013.

[14]	Owolawi, P. A., and T. Walingo, “Analysis of bi-static scattering due to hydrometeor on SHF and eHF links in a subtropical location,” Progress in Electromagnetic Research M, Vol. 42, pp. 95-107, 2015.

[15]	Tsang, L.; J. Pan, D. Liang, Z. Li, D. W. Cline, and Y. Tan, “Modeling active microwave remote sensing of snow using dense media radiative transfer (DMRT) theory with multiple – scattering effects,” IEEE Trans. Geoscience and Remote Sensing, Vol. 45, No. 4, pp. 990-1004, 2007.

[16]	Tsang, L., J. A. Kong, and K. H. Ding, Scattering of Electromagnetic Waves, Vol. I, Hoboken, NJ, Wiley, 2000.

[17]	Uijlenboet, R., “Raindrop size distributions and radar reflectivity-rain rate relationships for radar hydrology,” Hydrology and Earth System Science, Vol. 5, No. 4, pp. 615-627, 2001.

[18]	Ulbrich, C. W., “Natural variations in the analytical form of the raindrop size distribution,” Journal of Climate and Applied Meteorology, Vol. 22, pp. 1764-1775, 1983.

[19]	Veyrunes, O. (2000), “Influence des hydrométéores sur la propagation des ondes électromagnétiques dans la bande 30-100 GHz: Etudes théoretiques et statistiques”, Ph.D Thesis, University of Toulon and Var, June 2000.

[20]	Van de Hulst, H. C., Light Scattering by Small Particles, Dover Publication, Inc., New York, 1981.

[21]	Willis, P. T., “Functional fits to some observed drop size distributions and parameterization of rain,” Journal of the Atmospheric Science, Vol. 41, No. 9, pp. 1648-1661, 1984.

[22]	Yingbo, H., and T. P. Sarkar, “Matrix pencil method for estimating parameters of exponentially damped /undamped sinusoids in noise,” IEEE Trans. Acoustics, Speech, and Signal Processing, Vol. 38, No. 5, pp. 814-824, 1990.

Annex A

Raindrop scattered field representation

The far zone scattered field Es in arbitrary direction s from a single raindrop illuminated by a plane wave propagating in an arbitrary direction i is:

Es= Easas+Ebs bse-jkrr	(A-1)

In above, r is the distance between the raindrop center and the observation point. In addition, as and bs are two orthonormal unit vectors and each is orthogonal to the scattered direction s as shown in Figure A-1. Furthermore, k is the free space wavenumber (k=2π/λ, 𝛌 is the wavelength). One should keep in mind that there is a time factor of ejωt (ω=2π f,  f is frequency in Hz, and t is time in second) is assumed and suppressed.

Figure A-1

Polarization vectors of incident and scattered fields

bi

ai

i

s

as

bs

Eas and Ebs in (A-1), are the scattered field components which are related to the vertical Eai and horizontal Ebi components of the incident field through the scattering amplitude matrix elements fabs,i ‘s.

EasEbs= faa(s, i)fab(s, i)fba(s, i)fbb(s, i)EaiEbi	(A-2)

There are two common choices of the orthonormal unit systems (i, ai, bi) and s, as, bs that can be used to describe scattering by a raindrop:

–	horizontal and vertical polarization system, and

–	scattering plane system.

Horizontal – vertical polarization system

In this system, which is depicted in Figure A-2, the horizontal hi and vertical vi polarization vectors are defined with respect to the vertical on the Earth surface z. The horizontal polarization hi is orthogonal to the incident plane defined by the vertical on Earth surface (z axis) and the incident direction i.

hi= z × iz × i= -sinαicosαi0	(A-3)

In obtaining (A-3), the following formulation for the incident direction i is used:

i= cosεicosαicosεisinαisinεi	(A-4)

FIGURE A-2

The vertical - horizontal polarization system (i, hi, vi)

z

i

vi

εi

y

αi

hi

x

The vertical polarization vi is parallel to the incident plane and orthogonal to the horizontal polarization vector hi.

vi= hi×i = sinεicosαisinεisinαi-cosεi	(A-5)

Similar relations to (A-3) – (A-5) can be written in the scattering direction s through replacing the subscript i with the subscript s.

Other names for the horizontal polarization are TE polarization, perpendicular polarization, and s polarization. Other names for the vertical polarization are TM polarization, parallel polarization, and p polarization. Furthermore, this type of system coincides with spherical coordinate unit vectors of the antenna. Accordingly, it is used in this fascicle to describe the polarizations of station antennas. In this system the scattering amplitude matrix in (A-2) obeys this relation

EvsEhs= fvv(s, i)fvh(s, i)fhv(s, i)fhh(s, i)EviEhi	(A-6)

The scattering plane system

As shown in Figure A-3, in this system the polarization vectors 1i and 1s are taken to be similar and to be perpendicular to the scattering plane formed by the incident direction i and the scattered direction s.

1i= 1s= i ×si ×s = -sinαvihi+ cosαvivi	(A-7)

The angle αvi is the angle that rotates from the incident vertical polarization vector vi anticlockwise to the polarization vector 1i.

The polarization vectors 2iand 2s are taken to be parallel to the scattering plane, with:

2i= 1i × i1i × i =cosαvihi+sinαvivi	(A-8)

and

2s= 1s × s1s × s=cosφscosαvihi+sinαvivi-sinφsi	(A-9)

with φs is the scattering angle which is the angle between the incident and scattered directions:

cosφs=i ⋅s 	(A-10)

In obtaining (A-7) – (A-9), the unit vectors hi, vi, and i are considered as the coordinate system, and the scattering direction s is represented in that system as follows:

s=sinφscosαvihi+ sinαvivi+ cosφsi 	(A-11)

Based on (A-11), the angle αvi can be written as:

αvi=tan-1s⋅ vis ⋅hi 	(A-12)

FIGURE A-3

The scattering plane systems (i, 1i, 2i) and (s, 1s, 2s)

1i

2i

i

1s

2s

s

In this system the scattering amplitude matrix of (A-2) obeys this relation:

E1sE2s= f11(s, i)f12(s, i)f21(s, i)f22(s, i)E1iE2i	(A-13)

The advantage of this system is that the scattering amplitude takes a simple form for a particle of high symmetry such as raindrops. The disadvantage for this system is that the directions of the polarization vectors 1i and 2i depend on the scattered direction.

For a spherical raindrop, the scattering amplitude matrix elements are

f11s, i=jk S2φs,     f22s, i=jk S1φs

f12s, i=f21s, i=0	(A-14)

with S2φs and S1φs are Mie scattering amplitudes given in (34a) and (34b).

Based on (A-13) and (A-14) the scattering amplitude matrix for a spherical raindrop can reduces into

E1sE2s= jkS2φs00S1φsE1iE2i	(A-15)

Since the antenna polarizations are given in the vertical and horizontal polarization system, the scattering matrix elements given in (A-15) need to be transferred to the horizontal-vertical polarization system. This can be achieved using the tensor representation of the scattering amplitude matrix.

Fφs= jkS2φs1s1i + S1φs2s2i 	(A-16)

Then applying the scalar vector product from left by the polarization vector ps and from right by the polarization vector qi (p, q=v, h) gives the fpqs, i element of the scattering amplitude with the horizontal – vertical polarization system

Introducing (A-16) into (A-14) yields

fvvs, i=vs⋅Fφs⋅vi= jk S2φsvs⋅1s1i⋅vi+S1φsvs⋅2s2i.⋅vi	(A-17)

fvhs, i=vs⋅Fφs⋅hi= jkS2φsvs⋅1s1i⋅hi+S1φsvs⋅2s2i.⋅hi	(A-18)

fhvs, i=hs⋅Fφs.vi= jkS2φshs⋅1s1i⋅vi+S1φshs⋅2s2i⋅vi	(A-19)

fhhs, i=hs⋅Fφs.hi= jkS2φshs⋅ 1s1i⋅hi+S1φshs⋅2s2i⋅hi	(A-20)

To get explicit expressions for scalar vector products in (A-17) – (A-20) the following equations are constructed.

1s= -sinαvshs+ cosαvsvs	(A-21)

2s=  cosαvshs+sinαvsvs	(A-22)

with

αvs= cos-1-sinαvihi⋅ vs+cosαvivi⋅vs	(A-23)

Equations (A-21) and (A-22) are constructed by analogy to (A-7) and (A-8). Equivalently, the unit vectors hs, vs, and s are considered as a coordinate system, the unit polarization vectors 1s  and 2s are represented in that coordinate system.

Upon exploiting (A-7), (A-8), (A-21), and (A-22) in getting explicit expressions for the scalar vector products, (A-17) – (A-20) can be simplified as follows

fvvs, i= jk S2φscosαvscosαvi+S1φssinαvssinαvi	(A-24)

fvhs, i= jkS2φscosαvssinαvi-S1φssinαvscosαvi	(A-25)

fhvs, i= jkS2φssinαvscosαvi-S1φscosαvssinαvi	(A-26)

fhhs, i= jkS2φssinαvssinαvi+S1φscosαvscosαvi)	(A-27)

The scattering amplitudes in the forward direction s = i are required as shown in (31). In the forward direction the scattering angle φs has null value leading to equal values for Mie scattering amplitudes:

S10= S20= 12n=1∞2n+1 an+bn	(A-28)

Equation (A-28) is obtained by introducing (C-6) into (44a) and (44b). Introducing (A-28) into (A-24) – (A-27) with keeping in mind that in the forward direction αvs= αvi, yield the following formulation for the scattering amplitudes in the forward direction:

fvvi, i=fhhi, i= j2kn=1∞2n+1 an+bn	(A-29)

fvhi, i=fhvi, i=0  	(A-30)

Explicit formulation of Mie scattering coefficients, an’s and bn’s are given in (45).

Annex B

Ricatti-Bessel and Hankel functions in Mie scattering

Ricatti-Bessel ψnz and Hankel ξnz functions within Mie’s scattering coefficients (44) are related to the corresponding spherical Bessel jnz and spherical Hankel hn2z functions in (B-1).

ψnz= z jnz

ξnz= zhn2z=zjnz-jynz 	(B-1)

with ynz is Bessel function of second kind. The Ricatti-Bessel and Hankel functions and the corresponding spherical functions are governed by the following recurrence relation:

Fn+1z=2n+1zFnz- Fn-1(z)	(B-2)

with Fnz is either ψnz or ξnz. is the first order derivative Fn'z is governed by (B-3):

Fn'z=Fn-1z- nzFn(z)	(B-3)

Equations (B-4a) – (B-4e) give explicit expressions of ψnz for n=0,1,2,3,4:

ψ0z= sinz	(B-4a)

ψ1z= -cosz + sinZZ	(B-4b)

ψ2z= - 3coszz+ 3z2-1sinz	(B-4c)

ψ3z= 1- 15z2cosz- 6z-15z3sinz	(B-4d)

ψ4z= 10z- 105z3cosz+ 1- 45z2+ 105z4sinz	(B-4e)

Equations (B-5a) – (B-5e) give ξnz for n=0,1,2,3,4:

ξ0z=je-jz	(B-5a)

ξ1z= -e-jz z-1z	(B-5b)

ξ2z= -e-jz z2-3jz-3z2	(B-5c)

ξ3z= e-jz z3-6jz2-15z+15jz3	(B-5d)

ξ4z= je-jz z4-10jz3-45z2+105jz+1054	(B-5e)

The recurrence relation (B-2) can be used in building an algorithm for getting the spherical Bessel function of the second kind yn’s with:

y0z= -coszz

y1z= -coszz2- sinzz	(B-6)

The recurrence relation (B-2) cannot be used to get either the spherical jn's  or the Ricatti-Bessel functions ψn's of first kind. In building an algorithm for getting those functions, a backward recurrence relation for those functions can be used. In getting the backward recurrence relation (B-2) is divided by Fnz yielding:

1Rn+1(z)=2n+1z- Rn(z)	(B-7)

with

Rn(z)= Fn-1(z)Fn(z)	(B-8)

(B-7) can be rewritten as:

Rnz=2n+1z-1Rn+1z  	(B-9)

Equation (B-9) is the required backward relation with starting n value of:

nstart=1.1 z+11	(B-10)

The corresponding initial value Rnstart is given the following continued fraction:

Rnstartz= 2nstart+1z- 12(nstart+1)+1z- 12(nstart+2)+1z- 	(B-11)

The above continued fraction is terminated at a value of:

nfinal=z+7.5 z0.34+2	(B-12)

Annex C

Associated Legendre polynomial in Mie scattering

The associated Legendre polynomials and their derivatives are required for getting Mie scattering amplitude (44) are governed by the following recurrence relations:

πnμ=2n-1n-1 μ πn-1μ- nn-1πn-2μ	(C-1)

τnμ=nμπnμ- n+1 πn-1μ	(C-2)

With μ= cosθ' , and π0μ=0.

π1μ=1	(C-3a)

π2μ=3μ	(C-3b)

π3μ=325μ2-1	(C-3c)

π4μ=5μ2 7μ2-3	(C-3d)

and

τ1μ= μ	(C-4a)

τ2μ=6μ2-3	(C-4b)

τ3μ = 452 μ3- 332μ	(C-4c)

τ4μ= 52 28μ4-27μ2+3	(C-4d)

Furthermore, πnμ and τnμ are alternately even and odd function of μ:

πn-μ=-1n-1πnμ	(C-5a)

τn-μ=-1nπnμ	(C-5b)

Moreover, for θ'=0, μ=1, and:

πn1=τn1=12 nn+1	(C-6)

Annex D

Bi-static cross section regression coefficients

In getting the explicit expressions for η1 multi-variant linear regression shows that the dependence of η1on the scattering angle can be captured by (49) which is recalled here as (D-1)

η1= expu0+u1sin0.5φs2+u2sin0.5φs4+u3sin0.5φs6	(D-1)

The coefficients ui's (i=0,1,2,3) are related to rainfall rate as in (D-2)

ui= a0i+ a1iloge(R)+ a2iloge(R)2,               i=0, 1

ui= a0i+ a1iloge(R)+ a2iloge(R)2+a3iloge(R)3,      i=2,3	(D-2)

The dependence of any of the coefficients ani's of (D-2) on the frequency f can be written as in (D-3)

ani= m=07bn,mifm ,    i=0,1,2,3 &  n=0,1, 2,3	(D-3)

Moreover, the dependence of each of the coefficients bn,mi of (D-3) on temperature T is

bn,mi=c0 + c1T+ c2T2 	(D-4)

Values of the coefficients c0, c1and c2for each of the coefficients bn,mi's of (D-4) are provided in Tables D-1 through D-14 below. The coefficients are obtained through performing linear regression over simulated bi-static scattering cross data. The simulated data are generated from the local bistatic scattering cross section η1 of Equation (46) through changing the values of the following parameters:

–	Raindrop diameter D from 0.1 to 5 mm in step of 0.2 mm

–	Frequency f from 5 to 100 GHz in step of 0.5 GHz

–	Rainfall rate R from 1 to 150 mm/hr in step of 5 mm/hr

–	Temperature T from 0 to 35° C in step of 2° C, and

–	Scattering angle φs from 0 to 180 degrees in step of 1 degree.

The integration over the raindrop diameter in (46) is performed numerically through trapezoidal integration technique

Table D-1

Values of coefficients  c0, c1and c2 for b0,m0 of (D-3) in case of a00 of g0(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −23.033 | −0.019039 | −1.3511e-07 |
| 1 | 1.0988 | 0.0057909 | −1.8732e-05 |
| 2 | −0.053826 | −0.00051258 | 2.8893e-06 |
| 3 | 0.0017167 | 2.0326e-05 | −1.5242e-07 |
| 4 | −3.3231e-05 | −4.2625e-07 | 3.8089e-09 |
| 5 | 3.7396e-07 | 4.9297e-09 | −4.969e-11 |
| 6 | −2.2438e-09 | −2.9763e-11 | 3.2787e-13 |
| 7 | 5.5409e-12 | 7.3317e-14 | −8.6497e-16 |

Table D-2

Values of coefficients c0, c1and c2 for  b1,m0 of (D-3) in case of a10 of g0(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | 1.758 | 0.0078061 | −0.00019642 |
| 1 | −0.034774 | −0.0010409 | 4.8582e-05 |
| 2 | 0.0031934 | 1.2441e-05 | −3.9051e-06 |
| 3 | −0.00014758 | 1.6661e-06 | 1.5195e-07 |
| 4 | 3.3014e-06 | −7.0142e-08 | −3.2085e-09 |
| 5 | −3.8772e-08 | 1.1439e-09 | 3.7652e-11 |
| 6 | 2.3188e-10 | −8.5799e-12 | −2.3101e-13 |
| 7 | −5.5887e-13 | 2.4612e-14 | 5.779e-16 |

Table D-3

Values of coefficients c0, c1and c2 for  b2,m0 of (D-3) in case of a20 of g0(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.051224 | 0.00081531 | 1.1534e-05 |
| 1 | 0.0011587 | −0.00031961 | −2.3173e-06 |
| 2 | −8.8754e-05 | 3.5484e-05 | 1.4933e-07 |
| 3 | 9.6328e-07 | −1.6609e-06 | −4.7112e-09 |
| 4 | 5.927e-08 | 3.9523e-08 | 8.0972e-11 |
| 5 | −1.6618e-09 | −5.0408e-10 | −7.7464e-13 |
| 6 | 1.5626e-11 | 3.2862e-12 | 3.8749e-15 |
| 7 | −5.0972e-14 | −8.6057e-15 | −7.8859e-18 |

Table D-4

Values of coefficients c0, c1and c2 for  b0,m1 of (D-3) in case of a01 of g1(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | 0.28927 | 0.037271 | −0.00010078 |
| 1 | −0.11742 | −0.011059 | 6.6665e-05 |
| 2 | 0.010231 | 0.00093297 | −8.6068e-06 |
| 3 | −0.00041831 | −3.5477e-05 | 4.2065e-07 |
| 4 | 8.8529e-06 | 7.2358e-07 | −1.0192e-08 |
| 5 | −1.0313e-07 | −8.2014e-09 | 1.3111e-10 |
| 6 | 6.2591e-10 | 4.8736e-11 | −8.5865e-13 |
| 7 | −1.5469e-12 | −1.1849e-13 | 2.2546e-15 |

Table D-5

Values of coefficients c0, c1and c2 for  b1,m1 of (D-3) in case of a11 of of g1(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.2317 | −0.005093 | 0.00045796 |
| 1 | 0.020016 | −0.00076558 | −0.00011298 |
| 2 | 0.00060157 | 0.00020785 | 9.1237e-06 |
| 3 | −9.7303e-05 | −1.2384e-05 | −3.5392e-07 |
| 4 | 3.2711e-06 | 3.3064e-07 | 7.4324e-09 |
| 5 | −5.0187e-08 | −4.5084e-09 | −8.6694e-11 |
| 6 | 3.6714e-10 | 3.0694e-11 | 5.2868e-13 |
| 7 | −1.0386e-12 | −8.2853e-14 | −1.3148e-15 |

Table D-6

Values of coefficients c0, c1and c2 for b2,m1 of (D-3) in case of a21 of of g1(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.036841 | −0.0025519 | −2.6162e-05 |
| 1 | 0.012953 | 0.00084793 | 5.2688e-06 |
| 2 | −0.001305 | −8.5265e-05 | −3.3134e-07 |
| 3 | 5.9518e-05 | 3.7912e-06 | 9.6604e-09 |
| 4 | −1.454e-06 | −8.7204e-08 | −1.4711e-10 |
| 5 | 1.91e-08 | 1.085e-09 | 1.1752e-12 |
| 6 | −1.2719e-10 | −6.9419e-12 | −4.3973e-15 |
| 7 | 3.3748e-13 | 1.7914e-14 | 5.0804e-18 |

Table D-7

Values of coefficients c0, c1and c2 for b0,m2 of (D-3) in case of a02 of g2(D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.0022144 | −0.0014792 | 0.00030493 |
| 1 | −0.008123 | −0.00055742 | −8.8598e-05 |
| 2 | 0.0018507 | 0.00015755 | 8.3245e-06 |
| 3 | −8.484e-05 | −8.944e-06 | −3.572e-07 |
| 4 | 1.9127e-06 | 2.323e-07 | 7.9876e-09 |
| 5 | −2.2827e-08 | −3.1221e-09 | −9.6893e-11 |
| 6 | 1.4148e-10 | 2.1074e-11 | 6.0585e-13 |
| 7 | −3.5797e-13 | −5.6545e-14 | −1.5317e-15 |

Table D-8

Values of coefficientsc0, c1and c2 for b1,m2 of (D-3) in case of a12 of g2 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.048207 | −0.019603 | 3.8001e-06 |
| 1 | −0.00041118 | 0.0045669 | −1.0444e-05 |
| 2 | 0.0016887 | −0.00031651 | 1.7242e-06 |
| 3 | −0.00011195 | 1.0267e-05 | −1.0534e-07 |
| 4 | 3.0478e-06 | −1.776e-07 | 2.9975e-09 |
| 5 | −4.1397e-08 | 1.6755e-09 | −4.3294e-11 |
| 6 | 2.8014e-10 | −8.0718e-12 | 3.0902e-13 |
| 7 | −7.5246e-13 | 1.5353e-14 | −8.6683e-16 |

Table D-9

Values of coefficientsc0, c1and c2 for b2,m2 of (D-3) in case of a22 of g2 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.1208 | −0.0018073 | −7.7431e-05 |
| 1 | 0.039712 | 0.00099445 | 2.2037e-05 |
| 2 | −0.0039312 | −0.00012817 | −2.1388e-06 |
| 3 | 0.00017579 | 6.5734e-06 | 9.7426e-08 |
| 4 | −4.0495e-06 | −1.669e-07 | −2.3253e-09 |
| 5 | 5.0716e-08 | 2.2341e-09 | 3.0006e-11 |
| 6 | −3.2704e-10 | −1.512e-11 | −1.9832e-13 |
| 7 | 8.496e-13 | 4.0796e-14 | 5.2636e-16 |

Table D-10

Values of coefficientsc0, c1and c2 for b3,m2 of (D-3) in case of a32 of g2 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | 0.02176 | 0.0005804 | 9.4104e-06 |
| 1 | −0.0067089 | −0.00020457 | −2.5829e-06 |
| 2 | 0.0006556 | 2.156e-05 | 2.4395e-07 |
| 3 | −2.966e-05 | −9.9711e-07 | −1.0876e-08 |
| 4 | 6.9633e-07 | 2.3723e-08 | 2.5604e-10 |
| 5 | −8.7919e-09 | −3.0371e-10 | −3.2742e-12 |
| 6 | 5.6633e-11 | 1.9905e-12 | 2.15e-14 |
| 7 | −1.462e-13 | −5.2428e-15 | −5.6776e-17 |

Table D-11

Values of coefficients c0, c1and c2 for b0,m3 of D-3) in case of a03 of g3 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.046298 | −0.0057663 | −0.00018642 |
| 1 | 0.01272 | 0.0017156 | 4.8813e-05 |
| 2 | −0.0010278 | −0.0001572 | −4.0434e-06 |
| 3 | 3.4667e-05 | 6.2343e-06 | 1.5655e-07 |
| 4 | −6.1228e-07 | −1.3209e-07 | −3.2237e-09 |
| 5 | 5.8573e-09 | 1.5529e-09 | 3.65e-11 |
| 6 | −2.9595e-11 | −9.5349e-12 | −2.1505e-13 |
| 7 | 6.2019e-14 | 2.3843e-14 | 5.1597e-16 |

Table D-12

Values of coefficients c0, c1and c2 for b1,m3 of (D-3) in case of a13 of g3 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.15558 | −0.0042536 | −9.7633e-05 |
| 1 | 0.047606 | 0.0017754 | 3.2832e-05 |
| 2 | −0.0046542 | −0.00020881 | −3.4242e-06 |
| 3 | 0.00020312 | 1.0088e-05 | 1.6314e-07 |
| 4 | −4.6089e-06 | −2.4592e-07 | −4.0092e-09 |
| 5 | 5.6717e-08 | 3.2014e-09 | 5.2747e-11 |
| 6 | −3.5983e-10 | −2.1241e-11 | −3.5329e-13 |
| 7 | 9.2209e-13 | 5.6467e-14 | 9.4664e-16 |

Table D-13

Values of coefficients c0, c1and c2 for b2,m3 of (D-3) in case of a23 of g3 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | 0.11087 | 0.0061252 | 5.7319e-05 |
| 1 | −0.029622 | −0.0017355 | −1.5993e-05 |
| 2 | 0.0025942 | 0.00016036 | 1.5228e-06 |
| 3 | −0.00010795 | −6.8539e-06 | −6.7714e-08 |
| 4 | 2.3816e-06 | 1.5498e-07 | 1.5828e-09 |
| 5 | −2.8856e-08 | −1.9149e-09 | −2.0088e-11 |
| 6 | 1.8081e-10 | 1.2223e-11 | 1.3102e-13 |
| 7 | −4.5761e-13 | −3.1539e-14 | −3.4402e-16 |

Table D-14

Values of coefficients c0, c1and c2 for b3,m3 of (D-3) in case of a33 of g3 (D-2)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | −0.015838 | −0.00071563 | −6.1847e-06 |
| 1 | 0.0042926 | 0.00019142 | 1.6456e-06 |
| 2 | −0.00039171 | −1.6809e-05 | −1.5321e-07 |
| 3 | 1.6946e-05 | 6.9046e-07 | 6.7237e-09 |
| 4 | −3.8371e-07 | −1.5112e-08 | −1.5611e-10 |
| 5 | 4.6824e-09 | 1.8183e-10 | 1.9738e-12 |
| 6 | −2.9185e-11 | −1.1361e-12 | −1.2841e-14 |
| 7 | 7.3004e-14 | 2.8813e-15 | 3.3644e-17 |

Validation

Figures D-1 and D-2 below are calculated as a validation for the regression formulations of (D-1) – (D-4) and the associated regression coefficients reported in Tables D-1 through Table D-14.

Each figure provides a comparison between bi-static cross section η1 values based on the regression formulations of (D-1) through (D-4), and the corresponding values based on Mie scattering. In each figure the bi-static cross section values are calculated as a function of scattering angle at two different temperatures: 0° C, and 35° C. In Figure D-1, bi-static cross section values are calculated at 20 GHz, and in Figure D-2, the bi-static cross section values are calculated at 100 GHz.

Figure D-1

Comparison between two types of values of the local bi-static cross section (η1) at a frequency of 20 GHz

Figure D-2

Comparison between two types of values of the local bi-static cross section (η1) at a frequency of 100 GHz

As a conclusion to this annex, it is worth mentioning that as shown in Tables D-1 to D-14 the regression coefficients c0, c1, and c2 associated with the regression coefficients bn.mi of Equation (D-3) have very low values for higher orders of m (m>4). So introducing those coefficients into (D-4) and then introducing that resultants into (D-3) could leads to terms of very lower values. Those lower values terms could be overlooked in comparison to terms of higher values leading to error into the coefficients bn.mi's. To overcome such an issue the direct summations in (D-3) is turned into multiplications as follows

Rewrite Equation (D-3) as follows

ani= b0+fb1+fb2+fb3+fb4+b5+fb6+b7f	(D-5)

The coefficients bm (m=1,…,7) are the coefficients bn.mi's with dropping i and n for brevity. Then apply the software implementation on (D-5) instead of (D-3).

In a software code implementation of (D-5), the first summation performed by the code is the term b6+ b7f. Such a term has two terms that of equal values. So no term of those two terms could be overlooked in comparison to the other term.

Similar approach can be applied into the software implementations of (D-1), (D-2), and (D-4), as well as into the software implementation of Equations (E-2), (E-3), (E5), and (E-6) of Annex E.

Annex E

Specific attenuation regression coefficients

This Annex provides mathematical formulations for the regression coefficients of rain specific attenuation given in (52). This formulation is recalled as equation (E-1) for convenience.

γv=γh= κ Rα	(E-1)

Figures E-1 and E–2 depict comparisons between the coefficients α and 𝛋 of (E-1) and the corresponding coefficients for vertical and horizontal polarizations reported in Recommendation ITU-R P.838-3. The comparisons are performed as a function of frequency and at a temperature of 30 °C. One should note that the coefficients reported in Recommendation ITU-R P.838-3 have no temperature dependence.

The dependence of the regression coefficients α and 𝛋 of Equation (E-1) on frequency f (GHz) can be written as

α=  a0+a1x+a2 x2+a3x3+a4x4+a5x5+a6x6	(E-2)

and

κ=  expb0+b1x+b2x2+b3x3+b4x4+b5x5	(E-3)

with

x= loge(f)⁡	(E-4)

Each of the coefficients an (n=0,2,3,4,5,6) of equation (E-2) and the coefficients bn (n=0,1,2,3,4,5) of equation (E-3) depends on the temperature T (°C).

The dependence of each of the coefficients an (n=0,1,2,3,4,5,6) on temperature can be written as

an=c0+ c1T+ c2T2,     n=1,2,3,4,5,6	(E-5)

and the dependence of each of the coefficients bn (n=1,2,3,4,5) on temperature can be written as

bn=d,0+d1T+ d2T2,     n=1,2,3,4,5,6	(E-6)

The values of ci's and di’s (i=0,1,2) are given in Table E-1 and E-2 respectively. The coefficients are obtained through performing linear regression over simulated rain specific attenuation data. The simulated data are generated from the rain specific attenuation formulation of Equation (51) through changing the values of the following parameters:

–	Raindrop diameter D from 0.1 to 5 mm in step of 0.2 mm,

–	Frequency f from 5 to 100 GHz in step of 0.2 GHz,

–	Rainfall rate R from 1 to 150 mm/hr in step of 5 mm/hr, and

–	Temperature T from 0 to 35° C in step of 2° C.

The integration over the raindrop diameter in (51) is performed numerically through trapezoidal integration technique.

Table E-1

Values of the coefficients ci (i=1,2,3) of (E-5)

| m | c0 | c1 | c2 |
| --- | --- | --- | --- |
| 0 | 0.86481 | 0.0025984 | −3.2727e-05 |
| 1 | −0.32507 | −0.025593 | 0.00040852 |
| 2 | 0.70075 | 0.041632 | −0.00084479 |
| 3 | −0.4162 | −0.023144 | 0.00063446 |
| 4 | 0.11971 | 0.0054147 | −0.00022071 |
| 5 | −0.018495 | −0.00049312 | 3.6339e-05 |
| 6 | 0.0012143 | 8.1571e-06 | −2.2949e-06 |

Table 2

Values of the coefficients di (i=1,2,3) of (E-6)

| m | d0 | d1 | d2 |
| --- | --- | --- | --- |
| 0 | −9.2859 | −0.026677 | 7.4162e-05 |
| 1 | 1.5977 | −0.021172 | 0.001127 |
| 2 | 0.45627 | −0.0010862 | −0.0014558 |
| 3 | −0.15347 | 0.016763 | 0.00066036 |
| 4 | 0.040141 | −0.0062665 | −0.00012758 |
| 5 | −0.0049951 | 0.00064387 | 8.9007e-06 |

Figures E-1 and E-2 are calculated as a validation for the regression formulations of (E-1) – (E-6) and the associated regression coefficients reported in Tables E-1 and Table E-2. Each figure provides a comparison between rain specific attenuation values based on the regression formulations (E-1) – (E-6), and the corresponding values based on Mie scattering. In each figure the specific attenuation is calculated as a function of frequency at two different temperatures: 0° C , and 35° C. In Figure E-3, the specific attenuation is calculated at a rainfall rate of 5 mm/hr, and in Figure E-3 the specific attenuation is calculated at a rainfall rate of 100 mm/hr.

Figure E-1

Rain specific attenuation as a function of frequency (R = 5 mm/hr)

Figure E-2

Rain specific attenuation as a function of frequency (R = 100 mm/hr)

Annex F

Analytic solution for the scatter transfer integral

The analytic approach addressed in this fascicle is based on the Steepest Descent Method (SDM). To set the scattering transfer integral into a form suitable for applying the SDM, both the transmitting and receiving antennas are taken to have narrow beams. Accordingly, for the transmitting antenna, the linear form of the gain could be written as:

G1= Gmax1 exp-loge22θ/θ12+ 2φ/φ12	(F-1)

Gmax1 is the linear gain along the antenna boresight axis, θ and φ are the vertical and horizontal angles measured from the boresight axis of the transmitting antenna; and θ1 and φ1 are the half-power beamwidths of the antenna in the vertical and horizontal direction respectively. Similarly, the receiving antenna the gain could be written as:

G2= Gmax2 exp-loge22θ'/θ22+ 2φ'/φ22	(F-2)

Gmax2 is the linear gain along the antenna boresight axis, θ' and φ' are the vertical and horizontal angles measured from the boresight axis of the receiving antenna; and θ2 and φ2 are the half-power beamwidths of the antenna.

To simplify the mathematical formulations of the angles θ and φ, a local Cartesian coordinates (u, v, w) is created. The coordinate centre is positioned at intersection point x0,y0,h0 of the transmitter and receiver antenna boresights. The u axis is aligned along boresight of the transmitter antenna V10, and w axis is taken to be perpendicular on boresights of the transmitting and receiving antennas.

w= V10 × V20	(F-3)

and v axis is taken to be perpendicular on both u and w.

v= w × u	(F-4)

Within the coordinate (u, v, w), the angles θ and φ could be approximated as:

θ ≈ sinθ= vr1	(F-5a)

φ ≈ sinφ= wr1	(F-5b)

The angles θ' and φ' are expressed in terms of another local Cartesian coordinate (u',v',w') created through rotating the coordinate (u, v, w) around w axis toward the transmitter by an angle equal the scattering angle ϑs.

θ'≈ sinθ'= v'r2= v cosφs- u sinφsr2	(F-6a)

φ'≈ sinφ'= w'r2=wr2	(F-6b)

Introducing (f-5) into (F-1) ; and (F-6) into (F-2), then introducing the resultants into (58) reduce the scattering transfer integral into:

C= Fu,v,w exp-gu,v,w dudvdw	(F-7)

Fu,v,w= Gmax1Gmax2r12r22 Zpq exp-cKrain+Katm 	(F-8)

gu,v,w=4loge(2)vr1θ12+ wr1φ12+ wr2φ22+ vcosφs-usinφsr2θ22	(F-9)

Based on the SDM, the integral in (F-7) could be approximated as:

C ≈π32Δ Fusp,vsp,wsp exp-gusp,vsp,wsp	(F-10)

In the above, usp,vsp,wsp is the saddle point, and Δ is the determinant of the Hess matrix calculated at the saddle point.

Δ= ∂2∂xi∂xj3×3            ; xi=u,v,w;and xj=u,v,w	(F-11)

To get the saddle point, the first derivatives of gu,v,w are derived and then equated to zero.

∂gu,v,w∂u= 8loge(f)u sinφsr2θ22- vsinθscosθsr2θ22	(F-12a)

∂gu,v,w∂v= 8loge(f)v 1r1φ12+cosθsr2θ22- usinθscosθsr2θ22	(F-12b)

∂gu, v, w∂w= 8loge(f)w 1r1φ12+ 1r2φ22	(F-12c)

Equating each of the above derivatives to zero and solving the resultant equations imply that the saddle point occurs at the centre of two local coordinates (usp=vsp=wsp=0) which is the intersection point of the two antenna boresights (x0,y0,h0).

The second order derivatives of gu,v,w at the saddle point are:

∂2gu,v,w∂u2=8loge(2)sinφsr2θ22	(F-13a)

∂2gu,v,w∂v2=8loge(2)1r1θ12 + cosφsr2θ22	(F-13b)

∂2gu,v,w∂w2=8loge(2)1r1φ12 + 1r2φ22	(F-13c)

∂2gu,v,w∂w ∂u= ∂2gu,v,w∂w ∂v= 0	(F-13d)

∂2gu,v,w∂u ∂v= -  8loge(2)sinφs cosφsr2θ22 	(F-13e)

At the saddle point from (F-9), (F-11), (F-12), and (F-13), we can write:

gusp,vsp,wsp=0	(F-14a)

Δ= 8loge(2)r1φ12+ r2φ22r1φ1r2φ22 sinθsr1θ1r2θ22	(F-14b)

Introducing (F-14) into (F-10) yields the analytic approximation of the scatter transfer integral based on the SDM.

C≈1.206 Gmax1Gmax2θ1θ2φ1φ2Zpqsinφsr1φ12+ r2φ22  exp-Krain+Katm	(F-15)

______________

____________________