| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 3K/TEMP/64 Subject: Clutter loss measurement | Revision 1 to Document 3K/FAS/1-E |
|  | 3 July 2026 |
|  | English only |
| Working Party 3K |  |
| FASCICLE 3K/FAS/1-1 |  |
| Concerning Recommendation ITU-R P.2108 on clutter loss in the frequency range 30 MHz to 100 GHz Description of the method for clutter loss measurement |  |

Scope

This Fascicle provides guidance on measuring clutter loss for Earth-space, aeronautical and terrestrial paths to support further development of Recommendation ITU-R P.2108 and the associated Report, ITU-R P.2402.

# 1	Clutter loss

“Clutter loss” is defined, in Recommendation ITU-R P.2108, as the difference in the transmission loss or basic transmission loss with and without the presence of terminal clutter at either end of the path with all other path details being the same.

When measuring clutter loss, Lc (dB), it is obtained as the difference between the theoretically predicted free space basic transmission loss (Rec. ITU-R P.525) and the measured basic transmission loss.

# 2	Measurement scenario

The uncluttered terminal should be positioned at sufficient distance from the first obstacle and/or at sufficient height that the wave front is essentially plane and clear of local obstructions.

Measurements made with directional and omnidirectional antennas may be expected to give different results; antenna characteristics should, in any case, be carefully described.

The measurements need to include paths on street canyons of different widths, open squares, parking places etc. to have representative statistics of all environments.

# 3	Measurement equipment

If measurements are performed using continuous wave (CW) transmissions the measured data should be spatially averaged to eliminate multipath fading. An alternative to spatial averaging would be frequency averaging using a wide band signal.

Estimation of the path loss requires calibration of the system from back-to-back tests where the transmitter output is connected to the input of the receiver via attenuators and all the gain settings in the receiver’s RF chain and signal conditioning are tabulated.

If mobile measurements are made, care should be taken to avoid distorting spatial statistics by repeated sampling when the vehicle is stationary. Measurements will, ideally, be triggered by the vehicle odometer although post-processing may also be applied.

# 4	Antennas

The antenna at the Tc can be omnidirectional or scanned in azimuth.

In the case where a directional antenna is used at the Tu, the pointing of the antenna should ensure that the main beam passes over the roof of the buildings and the Tc is inside the main beam. The pointing of the antenna needs to be known to apply the antenna gain corrections at the Tu.

# 5	Analysis of clutter loss with respect to elevation angle

Elevation angle is an input parameter only for the Earth-space and aeronautical statistical clutter loss model of Section 3.3 of the Recommendation.

While elevation angle could be defined in a variety of ways (see Figure 2), the distinction between them will generally be irrelevant for the long paths to which the Section 3.3 model would be applied. It should also be noted that where elevation is defined with respect to local clutter height, the use of spatial averaging of multipath may make it impossible to determine an unambiguous geometry for each measurement point.

Measurement data submitted to the Study Group should not include elevation angle. The post-processing of such data can derive such angles using assumptions appropriate to the particular analysis (e.g. plane-Earth geometry, detailed terrain data, high-resolution building data).

Figure 1

Illustration of elevation angles

# 6	Parameters to be recorded

Each measurement set will ideally consist of a number of measurement samples in JSON format, following the schema embedded in this fascicle; if this is not possible a tabulated cumulative distribution function of loss could also be provided.

Researchers are asked to provide as much additional detail as possible; in particular, the geometry of the scenario. Photographs of the environment and measurement locations should be supplied.

When using directional antennas, antenna pointing information should be provided. Self-documenting information on antenna patterns should be provided as additional JSON metadata

The following parameters should be recorded when performing measurements of clutter loss, and submitted as a JSON data file. A schema for the submission of data is provided on the SG3 website.

TABLE 1

Measurement parameters

| Datum | JSON element | Type | Note |
| --- | --- | --- | --- |
| Metadata |  |  |  |
| Number of measurement points | datapoint_count | Integer |  |
| Dataset name | dataset_name | string | Free text |
| Location | location | string | Free text |
| Coordinate reference system | EPSG | Integer | 4 or 5 digit ID |
| Start/stop date/time | date.start_time date.stop_time | Structure (string) | ISO 8601 format |
| Noise floor | relative_noise_floor__dB | float | In terms of basic transmission loss |
| Transmitted waveform | waveform | string | CW, PRBS. Chirp, etc |
| Bandwidth | bandwidth__MHz | float | 0.0 if CW |
| Source | data_source | string | ‘measurements’, ‘raytracing’, ‘simulated’ |
| Notes | notes | String array | Free text |
| Measurement area | bounding_box.lat_min__deg bounding_box.lat_max__deg bounding_box.lon_min__deg bounding_box.lon_max__deg | Structure (float) | Decimal degrees, in WGS84 (EPSG 4326) |
| Terminal identification | uncluttered_terminal_is | string | ‘tx’ or ‘rx’ |
| Environment type | environment | string | Free text |
| Multipath averaging | averaging | string | ‘spatial’, ‘frequency’ |
| Uncluttered antenna data | uncluttered_antenna.type uncluttered_antenna.pol uncluttered_antenna.gain__dBi uncluttered_antenna.azimuth__deg uncluttered_antenna.elevation__deg | Structure (string, string, float, float, float) | type = ‘omnidirectional’ or ‘directional’ pol=’v’, ‘h’, ‘rhc’,’lhc’ |
| Cluttered antenna data | cluttered_antenna.type cluttered_antenna.pol cluttered_antenna.gain__dBi | Structure (string, float) | type = ‘omnidirectional’ or ‘directional’ If ‘directional’ assumed aligned with other terminal pol=’v’, ‘h’, ‘rhc’,’lhc’ |
| datapoints |  |  |  |
| Reference ID | id | int | sequential |
| RX latitude | rx_lat__deg | float | Decimal degrees WGS84 (EPSG 4326) |
| RX longitude | rx_lon__deg | float | Decimal degrees WGS84 (EPSG 4326) |
| TX latitude | tx_lat__deg | float | Decimal degrees WGS84 (EPSG 4326) |
| TX longitude | tx_lon__deg | float | Decimal degrees WGS84 (EPSG 4326) |
| Frequency | f__MHz | float |  |
| Tx antenna height above ground | h_tx__m | float |  |
| Rx antenna height above ground | h_rx__m | float |  |
| Cluttered antenna data | uncluttered_antenna.azimuth__deg uncluttered_antenna.elevation__deg | float float | Only required if antenna is directional and not aligned with uncluttered antenna |
| Basic transmission loss | L_btl__dB | float | Including all calibration |
| Elapsed time | time_offset__s | float | seconds |

_________________

____________________

Definitions of “transmission loss” and “basic transmission loss” are given in Recommendation ITU-R P.341.

JavaScript Object Notation (JSON)