|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  | COM 12 – C 47 – E |
| --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001-2004 |  |  | November 2002 |
|  |  |  |  | English only Original: English |
| Question(s): |  | 5/12 |  |  |
| STUDY GROUP 12 – CONTRIBUTION 47 |  |  |  |  |
| Source: |  | Rapporteur Q.5/12 |  |  |
| Title: |  | Rapporteur Report – Workshop on Headsets/Sending Part |  |  |

# 1	Introduction

During the last Study Group 12 meeting held in Geneva (27-31 May 2002), it was agreed to organise a workshop in Tenovis, Germany, in order to perform some measurements on the sending part of headsets, which has not been investigated so far.

This workshop was held from 19 to 20 September 2002 and the following participants were involved:

Martin Rung	GN Netcom

Allen Woo	Plantronics

Hans W. Gierlich	Head Acoustics

Gijs Dirks	Bruel & Kjaer

Luc Madec	Rapporteur of Q.5/12

# 2	Choice of Headsets

A preliminary investigation, driven by Martin Rung, and approved by the rest of the group, lead to a set of 10 headsets samples covering the range of the different headsets types available on the market today.

The list of the headsets is the following:

1)	JABRA EarWrap (from Rx study)

2)	PLX M130

3)	NOKIA (from Rx study)

4)	JABRA EarSet (Rx like JABRA EarBud from Rx study, but with microphone at the ear)

5)	PLX Encore NC (from Rx study)

6)	GNN Profile (from Rx study)

7)	GNN GN 2100 UNC

8)	PLX Mirage (from Rx study)

9)	DNS (from Rx study)

10)	HELLO DIRECT Executive

The headset types and respective microphone characteristics are summarised in the table below.

| Tx Type | Microphone Characteristics |  |  |
| --- | --- | --- | --- |
|  | Omni | Noise-cancelling | Directional |
| Short fixed boom | X |  | X 1 |
| Short adjustable boom | X |  | X 2 |
| Soap-on-a-rope (just hanging down) | X 3 |  |  |
| All-in-the-ear (like JABRA EarSet) |  |  | X 4 |
| Flexible long boom | X | X 5 |  |
| Stiff long boom (hinged) | X 6 | X 7 |  |
| Voice Tube (hinged) | X 8 |  |  |
| Soap-on-a-rope (collar clip) | X 9 |  | X 10 |

# 3	Positioning of the Headsets

A preliminary investigation and discussion lead to the following agreement:

	The positioning of the headset should reflect the way it is intended by the manufacturer to be used in a real situation.

	From the manufacturer instructions provided in the user guide, a positioning of the headset could be defined as being a Recommended Test Position.

	The precise location of the headset in space can then be described using a co-ordinate system.

The system used during this experimentation refers to the one used in Recommendation P.64, which defines “a set of Cartesian axes with origin at the centre of the lips”, and further defines the axes as follows:

–	x-axis: horizontal axis of the mouth, with positive direction into the mouth,

–	y-axis: horizontal, perpendicular to the x-axis, with positive direction towards the side of the mouth on which the handset is held,

–	z-axis: vertical, with positive direction upwards.

The use of the BMP position, as found in Recommendation P.58, was only applicable to one headset and was not considered to be a suitable method to be used for positioning during this experimentation.

# 4	Testing methodology during the experimentation

–	The experimentation started by positioning the headsets on the Head Acoustics HATS, following the positioning rules discussed above.

–	The headset was then placed and measured on the B&K HATS, using co-ordinates as close as possible of the ones found on the Head Acoustics HATS.

–	Each of the headsets was measured 2 times, after a complete repositioning.

–	The co-ordinates were measured with care in each case, using a  3D digitizer MicroScibe-3DX.

–	The excitation signal used was a pseudo-random noise: Level = – 4.7dBPa at MRP.

–	The frequency response curves were measured in 1/12th octave.

–	The electret microphone of each headset was directly connected to the same default supply with the following characteristics:

DC Voltage = 3V

Resistor = 2.2 kOhm

# 5	Discussion of the results

The complete results may be found in the Annex of this document.

The preliminary conclusions that could be drawn from the results are the following:

–	Globally the consistency of the results is considered as being rather good between the 2 HATS.

–	This is especially true for microphones located in the area where the behaviour of the 2 HATS, both complying with Recommendation P.58, is the same regarding the acoustic radiation impedance.

–	For microphones far from the mouth, such as soap on the roap models, some other effects appear above 4 kHz, such as the influence of the shoulders, which differ on the 2 HATS, and the directionality of both the microphone and the mouth.

–	On the same HATS, the positioning may be very critical for microphones close to the mouth.

–	A 3dB shift between the 2 measurements on the Head Acoustics HATS is observed for the Headset 1.

–	None of measurements conducted later in order to investigate this effect did not show this effect again. Since no satisfactory explanation having being found so far, it was decided to discount this measurement.

ANNEX

Measurement results

___________________