| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Car Communication" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG FITCAR-C-8 |
|  |  | English only Original: English |
|  |  | Ulm, Germany, 15 March 2007 |
| CONTRIBUTION |  |  |
| Source: | Volkswagen, Braunschweig Technical University |  |
| Title: | Digital Interface Concept for Factory-Fitted Speakerphones |  |

Digital Interface Concept for Factory-Fitted Speakerphones

Summary

In this Contribution we propose to require a digital interface to factory-fitted speakerphones. Aim is to ease testing and to improve reproducibility of the test or at least some specific test cases. In addition, at the end of the contribution we present a possibility to separate speech, residual noise, and the residual echo signal in the uplink just before speech encoding.

Useful Digital Interfaces

Digital interfaces to a hands-free system under test are mainly interesting in the uplink (sending) direction. Fig.1 gives an overview of the interfaces proposed in the following. The main idea is a better reproducibility of tests by using pre-recorded background noise and near-end talker (artificial) speech samples at the point right after the uplink input signal to the hands-free system under test is AD converted.

Fig. 1: Simultaneous read/write of digital signals in a test case with better reproducibility

We propose to require a digital simultaneous read/write 16 bit linear PCM access to the output path of the AD converter of the hands-free system in the uplink path. While the read signal d_l(n) can be stored externally, the write signal s_l(n)+n_l(n) shall be digitally added to the read signal d_l(n) in the handsfree system. Another simultaneous digital read access shall be realized in the uplink just after the handsfree system before speech encoding, yielding the enhanced uplink speech signal shat_l(n’).

Noise Recordings

This digital interface allows for convenient (separate) recording of signals via the microphone of the system under test. E.g., it allows for recording of artificially reproduced background noise n_l(n) by the 4-loudspeaker arrangement with subwoofer in the car cabin (according to [ETSI EG 202 396-1]). Furthermore, it comfortably will allow for recording of real noises during driving situations with the system under test. Moreover, microphone signals can then be recorded in virtually all test conditions giving information about recording level of the microphone and the AD converter.

Recorded noise signals n_l(n) shall be possible to be fed into the digital interface and be added to the current signal from the AD converter.

Near-End Speech Recordings and Offline Processing

The proposed digital interface can be used for recording (artificial) near-end  speech signals.

We propose that offline addition of recorded speech and noise and optional offline convolution with room impulse functions in case of close-talk mic recordings according to ITU processing standards (STL) shall be used to generate the speech plus noise portion of the uplink input to the system under test. This allows, e.g., for comfortably designing speech plus noise material for the Lombard effect related test proposed in the January 2007 FITCAR meeting and any noise reduction test.

Improved Test Cases

Two ways of handling this digital input signal are proposed: (a) The speech plus noise signal shall be added to the current signal from the AD converter inside the speakerphone system. (b) Handing over only the digital speech plus noise signal to the system under test. This is of interest for a reproducible test of the noise reduction.

The second read access of the digital interface (shat_l(n)) will allow to evaluate the noise reduction and echo cancelling properties by monitoring the near-end speaker signal plus residual noise plus residual echo.

The above described digital interfaces will allow for eased tests with better reproducibility. The test cases themselves can be carried out very similar as they are specified now already, except that part of the uplink input signals to the hands-free system stem from pre-recorded files. This will ease development and test at the equipment manufacturer side, but as well it will ease tests at the integrator side (car manufacturers).

For the physical access we propose to require the hardware integration of AES/EBU compliant interfaces, that can be activated and configured for reading/writing conveniently via car diagnosis means.

A digital interface opens the path to developing improved speech quality measures

With the already listed advantages of a digital interface as described above also totally new possibilities of quality measurement come up. We want to put the attention of the FITCAR focus group to the attached publication from DAGA, March 2007. In summary, first experiments show that from an auditive point of view it seems possible to separate speech, residual noise, and residual echo signal portions in the hands-free systems output signal in the uplink. Prerequisite is that pre-recorded speech and noise s_l(n)+n_l(n) is fed into the system under test, and an enhanced mixture signal shat_l(n’) is written to a file.

Braunschweig Technical University is inviting FITCAR participants to evaluate this scheme in-house. We will be able to provide audio examples at the March meeting in Ulm.

We are aware of the fact that this separation of signals will not cover the masking effect of residual echo and noise by the speech signal. However, an isolated judgement of the near-end speech signal portion of shat_l(n’) is definitely of high interest to the quality evaluation of hands-free systems.

Attachment: Experiments on Speech, Noise, and Echo Separationfor Quality Assessment of Hands-free Systems

_____________