| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Car Communication" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG FITCAR-C-16 |
|  |  | English only Original: English |
|  |  | Aachen, 27 November 2007 |
| CONTRIBUTION |  |  |
| Source: | Harman/Becker Automotive Systems and AKG Acoustics |  |
| Title: | Recommendations for Improvement of Microphone Measurements |  |

Chapter 6, Figure 2:

- Explicit definition of the allowable superimposed ripple voltage.

- Detailed description of the transmission range.

- Specification of a minimum load resistance is required

Chapter 9, Measurement Parameters and Requirements for Microphones and Microphone Arrangements

- The term “microphone arrangement” should be explained

Chapter 9.1, Microphone sensitivity

- Microphone parameters must be separated from any influence of integration

- It is recommended to determine first the sensitivity in the anechoic chamber and second the sensitivity of the microphone in the vehicle

Chapter 9.1.1, Microphone Sensitivity in anechoic conditions

- Minimum Sensitivity tolerance is required (+/-3dB)

- The definition of the overload point in this section is redundant to Chapter 9.2

- Test signal and level: It is recommended to use a third octave noise signal with a mid frequency of 1kHz in the anechoic chamber. The level of -10dBPa at MRP is to low, 0dBPa is recommended.

Chapter 9.1.2, Microphone Sensitivity in the car

- This chapter is to verify the microphone which fulfils the requirements in Chapter 9.1.1 integrated in the car.

- The mentioned requirement is no sensitivity parameter.

Chapter 9.2, Overload point

- The “overload point“ describes the “microphone linearity” (at medium sound levels and not at maximum sound levels)

- Due to the reduced sensitivity (300mV/Pa) the overload point should be increased from 15dB to 21dB

- This parameter could be substituted by an accurate definition of microphone distortion.

Chapter 9.3 Microphone Distortion

- The aim for the sensitivity reduction to 300mV/Pa was to increase the maximum sound pressure level. This sensitivity reduction should be considered in the levels for distortion measurements.

- The mentioned sound pressure levels for distortion measurements are very low.

- The acoustical measurements of a distortion value of 0.5% with an artificial head is difficult, because the artificial head itself has 0.5% THD at low sound pressure levels

- The parameter “maximum sound pressure level” is missing

Chapter 9.4 Microphone Frequency Response in the Car

- Tolerance mask: the upper limit for 4kHz should be increased to 4dB. This change considers requirements for voice recognition systems

Chapter 9.5 Idle Channel Noise

- This value should be reduced to -72dBV(P) for 300mV/Pa microphones.

Chapter 9.6 Ambient Noise Rejection (in the car)

- The D-Value is determined by the interaction of the microphone parameter “directivity” and the application parameter “distance”. In this context unambiguous recommendations for a  D-Value must consider the distance between speaker and microphone.

- For single microphones without signal processing we recommend a lookup-table with reachable D-Values measured with a non integrated reference microphone (omnidirectional) versus distance.

_____________