| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Car Communication" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG FITCAR-C-23 |
|  |  | English only Original: English |
|  |  | Göteborg, Sweden, 24 January 2008 |
| CONTRIBUTION |  |  |
| Source: | France Telecom |  |
| Title: | Comments on the draft specification “Hands-Free Communication in Motor Vehicles” (6th version) |  |

- Section 10.8 : distortion in sending –

The limit is given by reference to the distortion of the artificial mouth (which shall be lower than 1% for the test signal frequencies –see Recommendations P.51 and P.58)

It should be more relevant to define the distortion as an "absolute" than a "relative" value (eg, 30dB)

- Sections 10.2.1.1 and 10.2.2.1Delay

Clarification is needed on Ts and tsystem when considering the beginning and the end of the text.

The limit of 50ms seems to be too short if it is addressed to the sum of Ts and tsystem , when adding all the different delays due to the blocks defined in respectively figures 4 and 5.

- Section 10.10.1.2 : TCLw – Test-

It should be given the alternative to use an artificial voice signal (eg, P.50); some echo cancellers have not the same behaviour on pseudo random noise and on speech.

- Section 10.10.6.1 (Echo Performance with Time variant echo path)

It should be appropriate to give guidance on the material to be used, and in particular on its acoustical characteristics.

- Section 13 Guidance on subjective testing

In the document, subjective testing is restricted to Experts. However, the experience shows that non expert may be involved in these kinds of test.

The section could be updated in order to refer test sequences that can be done by experts and non experts (quality perceived), and those which require technical expertise and should be only done by experts (for diagnosis).

_____________